3GPP TSG-RAN WG1 #110-bis-e	R1-2210175

e-meeting, October 10th – 19th, 2022

Agenda Item:	9.5.1.2

Source:	Ericsson

Title:	On potential solutions for SL positioning

Document for:	Discussion, Decision

# Introduction

The study on expanded and improved NR positioning  SID [1] includes the following objective for sidelink positioning:

- Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2]

  - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1]

  - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1]

  - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1]

  - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required]

In this contribution, we provide our view on potential solutions for ranging and positioning.

# Discussion

## Terminology

Agreement

For the purpose of RAN1 discussion during this study item, at least the following terminology is used:

- Target UE: UE to be positioned (in this context, using SL, i.e. PC5 interface).

- Sidelink positioning: Positioning UE using reference signals transmitted over SL, i.e., PC5 interface, to obtain absolute position, relative position, or ranging information.

- Ranging: determination of the distance and/or the direction between a UE and another entity, e.g., anchor UE.

- Sidelink positioning reference signal (SL PRS): reference signal transmitted over SL for positioning purposes.

- SL PRS (pre-)configuration: (pre-)configured parameters of SL PRS such as time-frequency resources (other parameters are not precluded) including its bandwidth and periodicity.

- Continue discussion on additional terminology clarification(s) such as: Initiator UE, Responder UE, Sidelink Positioning group, reference UE, etc, including whether such terminology is needed within RAN1 discussion.

Agreement

For the purpose of RAN1 discussion during this study item, at least the following terminology is used:

- Anchor UE: UE supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., over the SL interface.

  - FFS: clarification of the knowledge of the location of the anchor UE

In RAN1#109-e, the following agreement was made:

Regarding the FFS, we note that UEs with unknown location can be supporting positioning of a target UE, at least for relative positioning. Hence, in our opinion, an Anchor UE according to the Agreement does not need to have known location.

An Anchor UE as defined in the agreement in RAN1#109-e does not need to have known location as such UEs can be supporting positioning of a target UE at least for relative positioning.

## SL PRS Resource Allocation

### Resource allocation schemes

On SL-PRS resource allocation, in RAN1#110, the following two schemes were agreed to be introduced for supporting SL positioning/ranging:

Agreement

Regarding SL-PRS resource allocation, both Scheme 1 and Scheme 2 should be introduced for supporting SL positioning/ranging:

- Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution)

  - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS.

- Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution)

  - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS

One remaining question is who decides which scheme to use for SL-PRS resource allocation when the SL UE is in coverage.  In our view, when the UE is in coverage, it should be up to network to configure which scheme should be used for SL-PRS resource allocation.  This is similar to the way mode2 operation controlled in SL communications where the network can control which scheme to use for in coverage UEs.  Hence, we make the following proposal:

Regarding SL-PRS resource allocation, when the SL UE is in coverage, gNB configuration of which scheme to use for SL-PRS resource allocation can be considered in NR Rel-18.

### Resource pools

Agreement

With regards to the SL Positioning resource allocation, one of the following alternatives should be introduced for supporting SL positioning/ranging:

- Alt. 1: only dedicated resource pool(s) can be (pre-)configured for SL-PRS

- Alt. 2: either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS

- Note: whether other signals/channels can be present in the dedicated resource pool can be further discussed

On SL positioning resource allocation, in RAN1#110, the following two alternatives were listed among which one of them will be downselected:

In our view, Alt 1 above is too restrictive as is does not allow the network operator to decide whether to share the resource pool between SL positioning and SL communications.  Given spectrum is a scarce resource, the operator should be equipped with the choice to decide how they want to use their spectrum.  We think Alt 2 is the only option that enables the operator to decide how they want to use their spectrum.  Hence, we make the following proposal:

Regarding SL positioning resource allocation, consider Alt 2 where either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS.

### SL PRS Resource design and configuration

#### SL PRS design

In RAN1#110, the following agreement was reached:

Agreement

For the sequence of the new reference signal for SL positioning/ranging, down select between Alt 1 and Alt 2:

- Alt. 1: pseudorandom-based. Use existing sequence of DL-PRS as a starting point.

- Alt. 2: ZC-based (SRS sequence as a starting point)

The SL PRS is transmitted from UE to UE in a context prone to interference. This is very similar to the issue that UL SRS addresses. It is well known that for sequence design, ZC sequences are more resilient to the interference caused by misaligned timing between UEs. There are limited amount of available sequences compared to Gold sequences. But considering that the positioning signals will not be many to be transmitted at the same time the amount of orthogonal sequences should not be considered a problem.

Additionally, transmitting a SL PRS with SRS-like properties will allow easy integration of the SL PRS with other UL SRSs, which could be multiplexed in the time frequency grid.  Finally, a UE capable of transmitting the SRS will in all likelihood be able to transmit the SL PRS.

Reuse the UL SRS for positioning design for SL PRS, with ZC sequences and the same comb configurations.

#### SL PRS resource configuration/activation/triggering

On SL-PRS configuration/activation/triggering, the following agreement was reached in RAN1#109-e:

Agreement

With regards to the configuration/activation/deactivation/triggering of SL-PRS, study the following options:

- Option 1: High-layer-only signaling involvement in the SL-PRS configuration

  - No Lower layer involvement, e.g., SL-MAC-CE or SCI or DCI, for the activation or the triggering of a SL-PRS.

  - Based on the study, this option may correspond to

    - A SL-PRS configuration that is a single-shot or multiple shots

    - A high-layer configuration that may be received from an LMF, a gNB, or a UE

- Option 2: High-layer and lower-layer signaling involvement in the SL-PRS configuration

  - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI

  - For example, high layer signaling can may be used for SL-PRS configuration and lower layer signaling can may be used for initiating SL positioning and/or configuration/triggering/activating/deactivating/indicating and potential resource indication/reservation transmission of SL-PRS.

- Option 3: Only lower-layer signaling involvement in the SL-PRS configuration

  - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI

- Note 1: Include aspects in the study related to flexibility, overhead, latency, and reliability as/if needed.

In RAN1#110bis-e, a follow up agreement was made:

Agreement

- With regards to the configuration/activation/deactivation/triggering of SL-PRS, Option 3 from the previous corresponding RAN1 #109 agreement will not be considered further.

- With regards to reservation of SL-PRS, it can be considered based on the Option 1 or Option 2 from the previous corresponding RAN1 #109 agreement.

Among the first two options, Option 1 is not flexible as SL-PRS can only be configured via higher layer signaling.  In our view, Option 1 is most suitable for periodic SL-PRS.  However, periodic SL-PRS may cause interference to other SL UEs, and increases the overhead of SL-PRS transmission.  Hence, we prefer Option 2 which allows either semi-persistent and/or aperiodic SL-PRS to be supported.  The overhead and interference to SL UEs can be reduced with the introduction of semi-persistent and/or aperiodic SL-PRS.  We thus make the following proposal:

With regards to the configuration/activation/deactivation/triggering of SL-PRS, consider Option 2 (High-layer and lower-layer signaling involvement in the SL-PRS configuration) in NR Rel-18.

## Hybrid positioning

One of the use cases where sidelink positioning measurements may be helpful is for instance in scenarios involving notable NLOS conditions, where positioning measurements performed on PC5 can be combined with uu based positioning measurements to improve positioning performance.  As shown in the evaluation agenda, PC5-only solutions for V2X may be limited by the geometry of the RSUs deployment,  but could leverage on the exisiting gNBs to improve the accuracy. Hence, solutions for positioning in this agenda also need to consider the possibility for combining positioning measurements over uu and PC5.  This can be ensured if SL measurements can also be reported to the network for the in-coverage scenario.

To enable the possibility for combining SL based positioning measurements and uu based positioning measurements to improve positioning performance, consider the possibility for SL UEs to report SL positioning measurements to the network when in coverage.

# Conclusions

Based on the discussion in the previous sections we propose the following:

Proposal 1	An Anchor UE as defined in the agreement in RAN1#109-e does not need to have known location as such UEs can be supporting positioning of a target UE at least for relative positioning.

Proposal 1	Regarding SL-PRS resource allocation, both Scheme 1 and Scheme 2 should be introduced for supporting SL positioning/ranging:

Proposal 2	Regarding SL-PRS resource allocation, when the SL UE is in coverage, gNB configuration of which scheme to use for SL-PRS resource allocation can be considered in NR Rel-18.

Proposal 3	Regarding SL positioning resource allocation, consider Alt 2 where either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS.

Proposal 4	Reuse the UL SRS for positioning design for SL PRS, with ZC sequences and the same comb configurations.

Proposal 5	With regards to the configuration/activation/deactivation/triggering of SL-PRS, consider Option 2 (High-layer and lower-layer signaling involvement in the SL-PRS configuration) in NR Rel-18.

Proposal 6	To enable the possibility for combining SL based positioning measurements and uu based positioning measurements to improve positioning performance, consider the possibility for SL UEs to report SL positioning measurements to the network when in coverage.

# References

RP-221814, Revised SID on Study on expanded and improved NR positioning, RAN#96, 2022

3GPP TS 22.261, "Service requirements for the 5G system".