3GPP TSG-RAN WG1 #110-bis-e	R1-2210179

e-meeting, October 10th – 19th, 2022

Agenda Item:	9.5.3

Source:	Ericsson

Title:	Positioning for RedCap UEs

Document for:	Discussion, Decision

# 1   Introduction

According to the Rel-18 study item description on study on expanded and improved new radio (NR) positioning [1], the objective on reduced capability (RedCap) positioning is as follows:

- Positioning support for RedCap UEs, considering the following: - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs [RAN1] - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with for RedCap UEs [RAN1, RAN2]

The target accuracy requirements and evaluation assumptions for RedCap positioning have been discussed in RAN1#109e and RAN1#110 meetings and the corresponding evaluation results are expected to be presented in the RAN1#110b meeting. The agreements for the target accuracy requirements of RedCap positioning are

Agreement - For the purpose of the Rel-18 study - The target accuracy requirements for RedCap UEs for commercial use cases are defined as follows: - Indoor and outdoor - Horizontal position accuracy (< 3 m) for 90% of UEs - Vertical position accuracy (< 3 m) for 90% of UEs - The target accuracy requirements for RedCap UEs for IIoT use cases are defined as follows: - Horizontal position accuracy (<1 m) for 90% of UEs - Vertical position accuracy (< 3 m) for 90% of UEs - Note: the requirements may not be met in all scenarios and use cases

RAN1#110 meeting also has discussed frequency hopping of DL PRS and UL SRS with the agreement as follows.

Agreement The potential benefits and performance gains of frequency hopping of the DL PRS and UL SRS can be investigated in release 18, which may take into account at least the following: - The impact of Doppler, phase offset, timing offset, power imbalance among hops - RedCap UE capability and complexity considerations - Impact of RF retuning during frequency hopping - Details of frequency hopping (including Tx hopping and/or Rx hopping, BWP switching) for the study are FFS

In this contribution, we share our views on RedCap positioning in terms of performance evaluations and potential enhancements.

# 2   Performance Evaluation

It has been agreed in RAN1#109e that all RAT-based positioning methods can be considered for evaluation of RedCap UE positioning performances. In this section, we provide our evaluation results for DL-TDOA, UL-TDOA and multi-RTT in UMi, UMa, IOO and InF-SH as shown in Fig. 1 – Fig. 4 with more information and data in Annex.

(a) FR1

Fig. 1 Horizontal positioning accuracy of 90% UEs in UMi (Case 1)

Fig. 1 provides the evaluation results for UMi in FR1. As shown in this figure, the target horizontal positioning accuracy of 3m@90% could be achieved via DL-TDOA, UL-TDOA or multi-RTT if the UE bandwidth is 100MHz in FR1, but could not if the UE bandwidth becomes 5MHz or 20MHz in FR1.

- In UMi, the target horizontal positioning accuracy could be achieved via DL-TDOA, UL-TDOA or multi-RTT in FR1 if UE bandwidth is 100MHz.

- In UMi, the target horizontal positioning accuracy could not be achieved via DL-TDOA, UL-TDOA or multi-RTT in FR1 if UE bandwidth is reduced to 20MHz or even 5MHz.

Fig. 2 shows the evaluation results for UMa in FR1 because FR1 is more suitable for the scenario of UMa due to its bigger coverage. According to the results, DL-TDOA has the potential to achieve the target horizontal positioning accuracy of 3m @ 67% if the UE bandwidth is 100MHz but has a big gap if the UE bandwidth is reduced to 20MHz or even 5MHz.

- In UMa, the horizontal positioning accuracy of 3m @ 67% could be achieved roughly via DL-TDOA in FR1 if UE bandwidth is 100MHz, but could not if UE bandwidth is reduced to 20MHz or even 5MHz.

(a) FR1

Fig. 2 Horizontal positioning accuracy of 67% UEs in UMa (Case 2)

(a) FR1

Fig. 3 Horizontal positioning accuracy of 90% convex UEs in IOO (Case 3)

Fig. 3 provides the evaluation results for IOO in FR1. According to this figure, in FR1, DL-TDOA, UL-TDOA and multi-RTT are able to achieve the target horizontal positioning accuracy if the UE bandwidth is 100MHz but has an obvious gap if the UE bandwidth is reduced to 20MHz or even 5MHz.

- In IOO with FR1, the target horizontal positioning accuracy could be achieved via DL-TDOA, UL-TDOA, or multi-RTT if UE bandwidth is 100MHz, but could not if UE bandwidth is reduced to 20MHz or even 5MHz.

Fig. 4 provides the evaluation results for InF-SH in FR1. According to this figure, in FR1, DL-TDOA, UL-TDOA and multi-RTT are able to achieve the target horizontal positioning accuracy of 1m @ 90% if the UE bandwidth is 100MHz but has a small gap if the UE bandwidth is reduced to 20MHz and a big gap if the UE bandwidth becomes 5MHz.

- In InF-SH with FR1, the target horizontal positioning accuracy could be achieved via DL-TDOA, UL-TDOA, or multi-RTT if UE bandwidth is 100MHz, and roughly achieved via multi-RTT is UE bandwidth is 20MHz.

(a) FR1

Fig. 4 Horizontal positioning accuracy of 90% convex UEs in InF-SH (Case 4)

Table 1 Summary of evaluation results for baseline performance

| Scenario | Solution |
| --- | --- |
|  |  | FR1 |  |  |
|  |  | 5MHz | 20MHz | 100MHz |
| UMi | DL-TDOA | N | N | Y |
|  | UL-TDOA | N | N | Y |
|  | Multi-RTT | N | N | Y |
| UMa | DL-TDOA | N | N | N |
|  | UL-TDOA | N | N | N |
|  | Multi-RTT | N | N | N |
| IOO | DL-TDOA | N | N | Y |
|  | UL-TDOA | N | N | Y |
|  | Multi-RTT | N | N | Y |
| InF-SH | DL-TDOA | N | N | Y |
|  | UL-TDOA | N | N | Y |
|  | Multi-RTT | N | N | Y |

Table 1 provides the summary of the evaluation results for baseline performances of DL-TDOA, UL-TDOA and multi-RTT in UMi, UMa, IOO and InF-SH. Based on this table, it could be observed that the enhancements to improve the horizontal positioning accuracy of RedCap UEs are necessary, especially when UE bandwidth is 5MHz or 20MHz in FR1.

To meet the requirements, enhancements to improve the horizontal positioning accuracy for RedCap UEs are necessary.

# 3   Enhancements

The horizontal positioning accuracies of RedCap UEs can be improved significantly via increasing UE bandwidth as observed in Section 2. One of the potential solutions for narrowband UE to get wideband measurement is frequency hopping over multiple narrowbands.

As shown in Fig. 5, with idea assumptions, the UEs with 5MHz bandwidth or 20MHz bandwidth can get similar horizontal positioning accuracy as the UEs with full bandwidth via PRS frequency hopping and/or SRS frequency hopping across full bandwidth.

- UEs with 5MHz bandwidth or 20MHz bandwidth can get similar horizontal positioning accuracy as UEs with full bandwidth via PRS frequency hopping and/or SRS frequency hopping across full bandwidth with idea assumptions.

(a) IOO, FR1

(b) InF-SH, FR1

Fig. 5 Horizontal positioning accuracy of 90% convex UEs in IOO (Case 3) and InF-SH (Case 4)

PRS frequency hopping and SRS frequency hopping are the potential solutions to improve the positioning accuracies of DL-TDOA, UL-TDOA and multi-RTT for RedCap UEs.

In DL-TDOA, if DL PRS signals hopping over multiple narrowbands are generated at the network side based on same DL system central frequency (e.g. intra-wideband), then the UE could post-compensate its phase offset by itself for PRS measurement across multiple narrowbands. Similarly, for UL-TDOA, the UE could pre-compensate its phase offset before transmitting UL SRS in different narrowbands if these narrowbands share same UL system central frequency at the network side.

Phase offset due to frequency hopping could be post-compensated for DL PRS and   UL SRS at the UE side if the involved carrier frequencies are related to same DL/UL system central frequencies at the network side.

To achieve similar measurement accuracy as full-bandwidth-based measurement given in Fig. 6-(a), RedCap UEs need to accumulate PRS measurements over multiple different narrowbands within full band at the different time slots as shown in Fig. 6-(b). Since narrowband UEs usually get same coverage as normal UEs by the solutions of repeated measurement and transmission in multiple slots, it could be considered that the complexity due to PRS frequency hopping and SRS frequency hopping is acceptable for RedCap UEs.

(a) PRS over full bandwidth     (b) PRS frequency hopping over multiple narrowbands

Fig. 6 PRS transmission

The complexity due to PRS frequency hopping and SRS frequency hopping is acceptable for RedCap UEs.

The SRS switching time for RedCap UE is ≤ 200us, 300us, 500us or 900us in RRC_CONNECTED mode when SRS switching time is performed between carriers, and 100us, 140us, 200us, 300us or 500us in RRC_INACTIVE mode when transmitting SRS outside initial UL BWP or with different SCS than the initial UL BWP. The impact of RF retuning during SRS frequency hopping on positioning performance may be marginal due to the short SRS switching time.

- The impact of RF retuning during SRS frequency hopping on positioning performance may be marginal due to the short SRS switching time.

# 4	Conclusion

In this contribution, we share our observations and proposals below for RedCap positioning.

Observation 1	In UMi, the target horizontal positioning accuracy could be achieved via DL-TDOA, UL-TDOA or multi-RTT in FR1 if UE bandwidth is 100MHz.

Observation 2	In UMi, the target horizontal positioning accuracy could not be achieved via DL-TDOA, UL-TDOA or multi-RTT in FR1 if UE bandwidth is reduced to 20MHz or even 5MHz.

Observation 3	In UMa, the horizontal positioning accuracy of 3m @ 67% could be achieved roughly via DL-TDOA in FR1 if UE bandwidth is 100MHz, but could not if UE bandwidth is reduced to 20MHz or even 5MHz.

Observation 4	In IOO with FR1, the target horizontal positioning accuracy could be achieved via DL-TDOA, UL-TDOA, or multi-RTT if UE bandwidth is 100MHz, but could not if UE bandwidth is reduced to 20MHz or even 5MHz.

Observation 5	In InF-SH with FR1, the target horizontal positioning accuracy could be achieved via DL-TDOA, UL-TDOA, or multi-RTT if UE bandwidth is 100MHz, and roughly achieved via multi-RTT is UE bandwidth is 20MHz.

Proposal 1	To meet the requirements, enhancements to improve the horizontal positioning accuracy for RedCap UEs are necessary.

Observation 6	UEs with 5MHz bandwidth or 20MHz bandwidth can get similar horizontal positioning accuracy as UEs with full bandwidth via PRS frequency hopping and/or SRS frequency hopping across full bandwidth with idea assumptions.

Proposal 2	PRS frequency hopping and SRS frequency hopping are the potential solutions to improve the positioning accuracies of DL-TDOA, UL-TDOA and multi-RTT for RedCap UEs.

Proposal 3	Phase offset due to frequency hopping could be post-compensated for DL PRS and pre-compensated for UL SRS at the UE side if the involved carrier frequencies are related to same DL/UL system central frequencies at the network side.

Proposal 4	The complexity due to PRS frequency hopping and SRS frequency hopping is acceptable for RedCap UEs.

Observation 7	The impact of RF retuning during SRS frequency hopping on positioning performance may be marginal due to the short SRS switching time.

# References

RP-213588, “SID on study on expanded and improved NR positioning”, RAN#94e, December 2021.

3GPP TR 37.875, “Study on support of reduced capability NR devices”, March 2021.

3GPP TR 38.857, “Study on NR positioning enhancements”, March 2021.

3GPP TR 38.855, “Study on NR positioning support”, March 2019.

3GPP TR 38.802, “Study on new radio access technology, physical layer aspects”, September 2017.

3GPP TS 22.104, “Service requirements for cyber-physical control applications in vertical domains”, Release 18, March 2021.

3GPP TS 22.261, “Service requirements for the 5G system”, Release 18, March 2021.

R1-2204954, “Consideration for RedCap positioning”, Ericsson, RAN1#109, May 2022.

R1-2207624, “Positioning for RedCap UEs”, Ericsson, RAN1#110, August 2022.

Annex B.6: Evaluation Results for Positioning for RedCap UEs

#### B.6.X		Results from source [Ericsson]

##### B.6.X.1	Description of evaluation scenarios

The scenarios to be presented in this section include

-	Baseline scenarios with UMi, UMa, IOO and InF-SH

-	IOO with the enhancements of PRS frequency hopping and SRS frequency hopping

Evaluation assumptions for system level analysis are provided in Table B.6.X.1-1 to Table B.6.X.1-1-7.

Table B.6.X.1-1: Positioning in UMi (Case 1), FR1 - evaluation scenarios and parameters

| Parameter | Case 101 (UMi, FR1) | Case 102 (UMi, FR1) | Case 103 (UMi, FR1) | Case 104 (UMi, FR1) | Case 105 (UMi, FR1) | Case 106 (UMi, FR1) | Case 107 (UMi, FR1) | Case 108 (UMi, FR1) | Case 109 (UMi, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario (baseline, otherwise state any modifications) | UMi |  |  |  |  |  |  |  |  |
| Carrier frequency | 3.5GHz |  |  |  |  |  |  |  |  |
| Subcarrier spacing | 30kHz |  |  |  |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 5MHz | 20MHz | 100MHz | 5MHz | 20MHz | 100MHz | 5MHz | 20MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS (Comb-6, 6 symbols) |  |  | PosSRS (Comb-8, 8 symbols) |  |  | DL-PRS (Comb-6, 6 symbols), PosSRS (Comb-8, 8 symbols) |  |  |
| Reference signal (type of sequence, number of ports, …) | Gold, single port |  |  | ZC, single port |  |  | Gold/ZC, single port |  |  |
| Number of sites | 19 |  |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 48 |  |  | 8 |  |  | 48 in DL and 8 in UL |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |  |
| Power-boosting level | 15.6dB |  |  | 18dB |  |  | 15.6dB in DL and 18dB in UL |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | high resolution no LOS/NLOS detection |  |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA PSO |  |  | UL-TDOA PSO |  |  | Multi-RTT PSO |  |  |
| Network synchronization assumptions | ideal |  |  |  |  |  |  |  |  |
| UE/gNB RX and TX timing error | ideal |  |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | - |  |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based |  |  |  |  |  |  |  |  |
| UE antenna configuration | (M, N, P, Mg, Ng) = (1,1,2,1,1), dH = 0.5λ |  |  |  |  |  |  |  |  |
| Number of UE branches | 1 |  |  |  |  |  |  |  |  |
| Description of enhancement solutions, if any | - |  |  |  |  |  |  |  |  |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1), (dH, dV) = (0.5, 0.8)λ |  |  |  |  |  |  |  |  |
| UE noise figure | 9dB |  |  |  |  |  |  |  |  |
| UE antenna height | 1.5m |  |  |  |  |  |  |  |  |
| gNB antenna height | 5m-20m, random |  |  |  |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |  |  |  |

Table B.6.X.1-2: Positioning in UMa (Case 2), FR1 - evaluation scenarios and parameters

| Parameter | Case 201 (UMa, FR1) | Case 202 (UMa, FR1) | Case 203 (UMa, FR1) | Case 204 (UMa, FR1) | Case 205 (UMa, FR1) | Case 206 (UMa, FR1) |
| --- | --- | --- | --- | --- | --- | --- |
| Scenario (baseline, otherwise state any modifications) | UMa |  |  |  |  |  |
| Carrier frequency | 3.5GHz |  |  |  |  |  |
| Subcarrier spacing | 30kHz |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 5MHz | 20MHz | 100MHz | 5MHz | 20MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS (Comb-6, 6 symbols) |  |  | PosSRS (Comb-8, 8 symbols) |  |  |
| Reference signal (type of sequence, number of ports, …) | Gold, single port |  |  | ZC, single port |  |  |
| Number of sites | 19 |  |  |  |  |  |
| Number of symbols used per occasion | 48 |  |  | 8 |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |
| Power-boosting level | 15.6dB |  |  | 18dB |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | high resolution no LOS/NLOS detection |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA PSO |  |  | UL-TDOA PSO |  |  |
| Network synchronization assumptions | ideal |  |  |  |  |  |
| UE/gNB RX and TX timing error | ideal |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | - |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based |  |  |  |  |  |
| UE antenna configuration | (M, N, P, Mg, Ng) = (1,1,2,1,1), dH = 0.5λ |  |  |  |  |  |
| Number of UE branches | 1 |  |  |  |  |  |
| Description of enhancement solutions, if any | - |  |  |  |  |  |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1), (dH, dV) = (0.5, 0.8)λ |  |  |  |  |  |
| UE noise figure | 9dB |  |  |  |  |  |
| UE antenna height | 1.5m |  |  |  |  |  |
| gNB antenna height | 20m-50m, random |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |

Table B.6.X.1-3: Positioning in IOO (Case 3), FR1 - evaluation scenarios and parameters

| Parameter | Case 301 (IOO, FR1) | Case 302 (IOO, FR1) | Case 303 (IOO, FR1) | Case 304 (IOO, FR1) | Case 305 (IOO, FR1) | Case 306 (IOO, FR1) | Case 307 (IOO, FR1) | Case 308 (IOO, FR1) | Case 309 (IOO, FR1) | Case 310 (IOO, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario (baseline, otherwise state any modifications) | IOO |  |  |  |  |  |  |  |  |  |
| Carrier frequency | 3.5GHz |  |  |  |  |  |  |  |  |  |
| Subcarrier spacing | 30kHz |  |  |  |  |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 5MHz | 20MHz | 5MHz | 20MHz | 100MHz | 5MHz | 20MHz | 5MHz | 20MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS (Comb-6, 6 symbols) |  |  |  |  | PosSRS (Comb-2, 2 symbols) |  |  |  |  |
| Reference signal (type of sequence, number of ports, …) | Gold, single port |  |  |  |  | ZC, single port |  |  |  |  |
| Number of sites | 12 |  |  |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 48 |  |  |  |  | 2 |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |  |  |
| Power-boosting level | - |  |  |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | high resolution no LOS/NLOS detection |  |  |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA PSO |  |  |  |  | UL-TDOA PSO |  |  |  |  |
| Network synchronization assumptions | ideal |  |  |  |  |  |  |  |  |  |
| UE/gNB RX and TX timing error | ideal |  |  |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | - |  |  |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based |  |  |  |  |  |  |  |  |  |
| UE antenna configuration | (M, N, P, Mg, Ng) = (1,1,2,1,1), dH = 0.5λ |  |  |  |  |  |  |  |  |  |
| Number of UE branches | 1 |  |  |  |  |  |  |  |  |  |
| Description of enhancement solutions, if any |  |  | frequency hopping over 100MHz |  |  |  |  | frequency hopping over 100MHz |  |  |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ |  |  |  |  |  |  |  |  |  |
| UE noise figure | 9dB |  |  |  |  |  |  |  |  |  |
| UE antenna height | 1.5m |  |  |  |  |  |  |  |  |  |
| gNB antenna height | 3m |  |  |  |  |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |  |  |  |  |
| Parameter | Case 311 (IOO, FR1) | Case 312 (IOO, FR1) | Case 313 (IOO, FR1) | Case 314 (IOO, FR1) | Case 315 (IOO, FR1) |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 5MHz | 20MHz | 5MHz | 20MHz | 100MHz |  |  |  |  |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS (Comb-6, 6 symbols), PosSRS (Comb-8, 8 symbols) |  |  |  |  |  |  |  |  |  |
| Reference signal (type of sequence, number of ports, …) | Gold/ZC, single port |  |  |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 48 in DL and 8 in UL |  |  |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Multi-RTT PSO |  |  |  |  |  |  |  |  |  |

Table B.6.X.1-4: Positioning in InF-SH (Case 4), FR1 - evaluation scenarios and parameters

| Parameter | Case 401 (InF-SH, FR1) | Case 402 (InF-SH, FR1) | Case 403 (InF-SH, FR1) | Case 404 (InF-SH, FR1) | Case 405 (InF-SH, FR1) | Case 406 (InF-SH, FR1) | Case 407 (InF-SH, FR1) | Case 408 (InF-SH, FR1) | Case 409 (InF-SH, FR1) | Case 410 (InF-SH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario (baseline, otherwise state any modifications) | InF-SH |  |  |  |  |  |  |  |  |  |
| Carrier frequency | 3.5GHz |  |  |  |  |  |  |  |  |  |
| Subcarrier spacing | 30kHz |  |  |  |  |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 5MHz | 20MHz | 5MHz | 20MHz | 100MHz | 5MHz | 20MHz | 5MHz | 20MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS (Comb-6, 6 symbols) |  |  |  |  | PosSRS (Comb-2, 2 symbols) |  |  |  |  |
| Reference signal (type of sequence, number of ports, …) | Gold, single port |  |  |  |  | ZC, single port |  |  |  |  |
| Number of sites | 18 |  |  |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 48 |  |  |  |  | 2 |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |  |  |
| Power-boosting level | - |  |  |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | high resolution no LOS/NLOS detection |  |  |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA PSO |  |  |  |  | UL-TDOA PSO |  |  |  |  |
| Network synchronization assumptions | Ideal |  |  |  |  |  |  |  |  |  |
| UE/gNB RX and TX timing error | Ideal |  |  |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | - |  |  |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based |  |  |  |  |  |  |  |  |  |
| UE antenna configuration | (M, N, P, Mg, Ng) = (1,1,2,1,1), dH = 0.5λ |  |  |  |  |  |  |  |  |  |
| Number of UE branches | 1 |  |  |  |  |  |  |  |  |  |
| Description of enhancement solutions, if any |  |  | frequency hopping over 100MHz |  |  |  |  | frequency hopping over 100MHz |  |  |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ |  |  |  |  |  |  |  |  |  |
| UE noise figure | 9dB |  |  |  |  |  |  |  |  |  |
| UE antenna height | 1.5m |  |  |  |  |  |  |  |  |  |
| gNB antenna height | 8m |  |  |  |  |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |  |  |  |  |
| Parameter | Case 411 (InF-SH, FR1) | Case 412 (InF-SH, FR1) | Case 413 (InF-SH, FR1) | Case 414 (InF-SH, FR1) | Case 415 (InF-SH, FR1) |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 5MHz | 20MHz | 5MHz | 20MHz | 100MHz |  |  |  |  |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS (Comb-6, 6 symbols), PosSRS (Comb-2, 2 symbols) |  |  |  |  |  |  |  |  |  |
| Reference signal (type of sequence, number of ports, …) | Gold/ZC, single port |  |  |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 48 in DL and 2 in UL |  |  |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Multi-RTT PSO |  |  |  |  |  |  |  |  |  |

##### B.6.X.2	Positioning accuracy evaluation results

Table B.6.X.2-1 provides summary of evaluation results for horizontal positioning error in UMi.

Table B.6.X.2-2 provides summary of evaluation results for horizontal positioning error in UMa.

Table B.6.X.2-3 provides summary of evaluation results for horizontal positioning error in IOO.

Table B.6.X.2-4 provides summary of evaluation results for horizontal positioning error in InF-SH.

Table B.6.X.2-1: Positioning in UMi (Case 1) - horizontal location error results

| Cases |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 101, UMi, FR1, DL-TDOA with 5MHz | All UEs | 8.9 | 12 | 15 | 19 |
| Case 102, UMi, FR1, DL-TDOA with 20MHz | All UEs | 3.9 | 5.1 | 6.5 | 8.3 |
| Case 103, UMi, FR1, DL-TDOA with 100MHz | All UEs | 1.0 | 1.4 | 1.9 | 2.7 |
| Case 104, UMi, FR1, UL-TDOA with 5MHz | All UEs | 10 | 14 | 17 | 22 |
| Case 105, UMi, FR1, UL-TDOA with 20MHz | All UEs | 4.2 | 5.5 | 6.9 | 8.5 |
| Case 106, UMi, FR1, UL-TDOA with 100MHz | All UEs | 1.1 | 1.6 | 2.2 | 3.0 |
| Case 107, UMi, FR1, Multi-RTT with 5MHz | All UEs | 8.5 | 11 | 14 | 17 |
| Case 108, UMi, FR1, Multi-RTT with 20MHz | All UEs | 3.1 | 3.9 | 4.8 | 5.8 |
| Case 109, UMi, FR1, Multi-RTT with 100MHz | All UEs | 0.9 | 1.1 | 1.4 | 1.9 |

Table B.6.X.2-2: Positioning in UMa (Case 2) - horizontal location error results

| Cases |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 201, UMa, FR1, DL-TDOA with 5MHz | All UEs | 18 | 25 | 34 | 45 |
| Case 202, UMa, FR1, DL-TDOA with 20MHz | All UEs | 6.7 | 9.7 | 13 | 20 |
| Case 205, UMa, FR1, DL-TDOA with 100MHz | All UEs | 1.8 | 3.6 | 8.4 | 15 |
| Case 206, UMa, FR1, UL-TDOA with 5MHz | All UEs | 34 | 48 | 79 | 178 |
| Case 207, UMa, FR1, UL-TDOA with 20MHz | All UEs | 12 | 20 | 52 | 180 |
| Case 208, UMa, FR1, UL-TDOA with 5MHz and FH | All UEs | 2.2 | 5.0 | 10 | 24 |
| Case 209, UMa, FR1, UL-TDOA with 20MHz and FH | All UEs | 3.2 | 9.3 | 20 | 86 |
| Case 210, UMa, FR1, UL-TDOA with 100MHz | All UEs | 6.1 | 17 | 62 | 185 |

Table B.6.X.2-3: Positioning in IOO (Case 3) - horizontal location error results

| Cases |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 301, IOO, FR1, DL-TDOA with 5MHz | (Optional) All UEs | 5.7 | 8.1 | 12 | 17 |
|  | Convex UEs | 4.0 | 5.4 | 7.1 | 10 |
| Case 302, IOO, FR1, DL-TDOA with 20MHz | (Optional) All UEs | 2.8 | 4.1 | 6.0 | 9.5 |
|  | Convex UEs | 1.9 | 2.6 | 3.3 | 4.5 |
| Case 303, IOO, FR1, DL-TDOA with 5MHz and FH | (Optional) All UEs | 0.5 | 0.8 | 1.3 | 2.2 |
|  | Convex UEs | 0.3 | 0.4 | 0.6 | 0.8 |
| Case 304, IOO, FR1, DL-TDOA with 20MHz and FH | (Optional) All UEs | 0.5 | 0.8 | 1.3 | 2.1 |
|  | Convex UEs | 0.3 | 0.4 | 0.6 | 0.8 |
| Case 305, IOO, FR1, DL-TDOA with 100MHz | (Optional) All UEs | 0.5 | 0.8 | 1.2 | 2.2 |
|  | Convex UEs | 0.3 | 0.4 | 0.6 | 0.8 |
| Case 306, IOO, FR1, UL-TDOA with 5MHz | (Optional) All UEs | 5.9 | 8.4 | 12 | 18 |
|  | Convex UEs | 4.2 | 5.8 | 7.5 | 11 |
| Case 307, IOO, FR1, UL-TDOA with 20MHz | (Optional) All UEs | 2.9 | 4.1 | 5.8 | 8.8 |
|  | Convex UEs | 2.0 | 2.5 | 3.3 | 4.6 |
| Case 308, IOO, FR1, UL-TDOA with 5MHz and FH | (Optional) All UEs | 0.5 | 0.8 | 1.3 | 2.1 |
|  | Convex UEs | 0.3 | 0.4 | 0.6 | 0.9 |
| Case 309, IOO, FR1, UL-TDOA with 20MHz and FH | (Optional) All UEs | 0.5 | 0.8 | 1.3 | 2.1 |
|  | Convex UEs | 0.3 | 0.4 | 0.6 | 0.9 |
| Case 310, IOO, FR1, UL-TDOA with 100MHz | (Optional) All UEs | 0.5 | 0.8 | 1.3 | 2.2 |
|  | Convex UEs | 0.3 | 0.4 | 0.6 | 0.9 |
| Case 311, IOO, FR1, Multi-RTT with 5MHz | (Optional) All UEs | 5.6 | 7.4 | 9.5 | 13 |
|  | Convex UEs | 5.6 | 7.9 | 9.9 | 14 |
| Case 312, IOO, FR1, Multi-RTT with 20MHz | (Optional) All UEs | 2.4 | 3.2 | 4.2 | 5.7 |
|  | Convex UEs | 2.1 | 3.0 | 4.1 | 5.6 |
| Case 313, IOO, FR1, Multi-RTT with 5MHz and FH | (Optional) All UEs | 0.3 | 0.5 | 0.7 | 1.2 |
|  | Convex UEs | 0.2 | 0.4 | 0.5 | 0.8 |
| Case 314, IOO, FR1, Multi-RTT with 20MHz and FH | (Optional) All UEs | 0.3 | 0.5 | 0.7 | 1.1 |
|  | Convex UEs | 0.2 | 0.4 | 0.5 | 0.8 |
| Case 315, IOO, FR1, Multi-RTT with 100MHz | (Optional) All UEs | 0.3 | 0.5 | 0.7 | 1.3 |
|  | Convex UEs | 0.2 | 0.4 | 0.5 | 0.8 |

Table B.6.X.2-4: Positioning in InF-SH (Case 4) - horizontal location error results

| Cases |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 401, InF-SH, FR1, DL-TDOA with 5MHz | (Optional) All UEs | 5.9 | 9.2 | 13 | 20 |
|  | Convex UEs | 4.8 | 6.7 | 8.8 | 12 |
| Case 402, InF-SH, FR1, DL-TDOA with 20MHz | (Optional) All UEs | 0.7 | 1.1 | 1.8 | 3.4 |
|  | Convex UEs | 0.5 | 0.8 | 1.1 | 1.7 |
| Case 403, InF-SH, FR1, DL-TDOA with 5MHz and FH | (Optional) All UEs | 0.04 | 0.06 | 0.09 | 0.2 |
|  | Convex UEs | 0.03 | 0.04 | 0.06 | 0.1 |
| Case 404, InF-SH, FR1, DL-TDOA with 20MHz and FH | (Optional) All UEs | 0.04 | 0.06 | 0.1 | 0.2 |
|  | Convex UEs | 0.03 | 0.04 | 0.06 | 0.1 |
| Case 405, InF-SH, FR1, DL-TDOA with 100MHz | (Optional) All UEs | 0.04 | 0.06 | 0.1 | 0.2 |
|  | Convex UEs | 0.03 | 0.04 | 0.06 | 0.1 |
| Case 406, InF-SH, FR1, UL-TDOA with 5MHz | (Optional) All UEs | 7.9 | 12 | 17 | 25 |
|  | Convex UEs | 6.3 | 8.7 | 11 | 15 |
| Case 407, InF-SH, FR1, UL-TDOA with 20MHz | (Optional) All UEs | 0.8 | 1.2 | 2.0 | 3.4 |
|  | Convex UEs | 0.6 | 0.8 | 1.2 | 2.0 |
| Case 408, InF-SH, FR1, UL-TDOA with 5MHz and FH | (Optional) All UEs | 0.04 | 0.06 | 0.1 | 0.2 |
|  | Convex UEs | 0.03 | 0.04 | 0.06 | 0.1 |
| Case 409, InF-SH, FR1, UL-TDOA with 20MHz and FH | (Optional) All UEs | 0.04 | 0.06 | 0.1 | 0.2 |
|  | Convex UEs | 0.03 | 0.04 | 0.06 | 0.1 |
| Case 410, InF-SH, FR1, UL-TDOA with 100MHz | (Optional) All UEs | 0.04 | 0.06 | 0.1 | 0.2 |
|  | Convex UEs | 0.03 | 0.04 | 0.06 | 0.1 |
| Case 411, InF-SH, FR1, Multi-RTT with 5MHz | (Optional) All UEs | 5.5 | 8.0 | 11 | 16 |
|  | Convex UEs | 4.8 | 6.8 | 9.6 | 13 |
| Case 412, InF-SH, FR1, Multi-RTT with 20MHz | (Optional) All UEs | 0.4 | 0.6 | 0.9 | 1.5 |
|  | Convex UEs | 0.4 | 0.5 | 0.8 | 1.1 |
| Case 413, InF-SH, FR1, Multi-RTT with 5MHz and FH | (Optional) All UEs | 0.02 | 0.03 | 0.05 | 0.07 |
|  | Convex UEs | 0.02 | 0.03 | 0.04 | 0.06 |
| Case 414, InF-SH, FR1, Multi-RTT with 20MHz and FH | (Optional) All UEs | 0.02 | 0.04 | 0.05 | 0.08 |
|  | Convex UEs | 0.02 | 0.03 | 0.04 | 0.07 |
| Case 415, InF-SH, FR1, Multi-RTT with 100MHz | (Optional) All UEs | 0.02 | 0.04 | 0.05 | 0.08 |
|  | Convex UEs | 0.02 | 0.03 | 0.04 | 0.06 |
|  | Convex UEs | 0.003 | 0.004 | 0.008 | 0.2 |