3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2210210

# e-Meeting, October 10-19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.202 | CR |  | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the channel combinations for MBS UE handling to TS38.202 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CBN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Table 6.2-2, for RRC_CONNECTED state and for PCell, the combination of (B and/or (D0 or (m1*D1+m2*D2+((m3*D3+m4*D4) or m5*(D5 or D6)) lacks explanation whether UE is required to support FDMed MCCH/MTCH/Multicast and SIB or FDMed MTCH/multicast with PBCH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add note 13 and note 14 in the table: - Note 13: For a UE supporting MBS multicast or broadcast reception in RRC_CONNECTED state, it is not required to support reception of FDMed MCCH/MTCH/multicast PDSCH and SIB PDSCH in PCell. - Note 14: For a UE supporting MBS multicast or broadcast reception in RRC_CONNECTED state, it is not required to support reception of FDMed MTCH/multicast PDSCH and PBCH in PCell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without the notes added, the combination of (B and/or (D0 or (m1*D1+m2*D2+((m3*D3+m4*D4) or m5*(D5 or D6)) may imply UE is required to support FDMed MCCH/MTCH/Multicast and SIB or FDMed MTCH/multicast with PBCH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.213 (R1-2210208), TS38.214 (R1-2210209) |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.2	Downlink

The tables 6.2-1, 6.2-2 describe the possible combinations of physical channels that can be received simultaneously in the downlink by one UE. Table 6.2-1 introduces notation for a "Reception Type" which represents a physical channel and any associated transport channel. Table 6.2-2 describes the combinations of these "Reception Types" which are supported by the UE depending on capabilities [8, TS 38.306], and enumerates how many of each can be received simultaneously. The UE shall be able to receive all TBs according to the indication on PDCCH. Any subset of the combinations specified in table 6.2-2 is also supported.

Table 6.2-1: Downlink "Reception Types"

| "Reception Type" | Physical Channel(s) | Monitored RNTI | Associated Transport Channel | Comment |
| --- | --- | --- | --- | --- |
| A | PBCH | N/A | BCH |  |
| B | PDCCH+PDSCH | SI-RNTI | DL-SCH | Note 1 |
| C0 | PDCCH | P-RNTI | N/A | Note 1, Note 2 |
| C1 | PDCCH+PDSCH | P-RNTI | PCH | Note 1 |
| D0 | PDCCH+PDSCH | RA-RNTI or Temporary C-RNTI or MsgB-RNTI | DL-SCH | Note 3 |
| D1 | PDCCH+PDSCH | C-RNTI, CS-RNTI, MCS-C-RNTI | DL-SCH |  |
| D2 | PDCCH | C-RNTI, CS-RNTI, MCS-C-RNTI | DL-SCH |  |
| D3 | PDCCH+PDSCH | G-RNTI, G-CS-RNTI | DL-SCH | Note 6 |
| D4 | PDCCH | G-CS-RNTI | N/A | Note 7 |
| D5 | PDCCH+PDSCH | MCCH-RNTI | DL-SCH | Note 8 |
| D6 | PDCCH+PDSCH | G-RNTI | DL-SCH | Note 9 |
| E | PDCCH | C-RNTI | N/A | Note 4 |
| F0 | PDCCH | Temporary C-RNTI | UL-SCH | Note 3 |
| F1 | PDCCH | C-RNTI, CS-RNTI, MCS-C-RNTI | UL-SCH |  |
| G | PDCCH | SFI-RNTI | N/A |  |
| H | PDCCH | INT-RNTI | N/A |  |
| J0 | PDCCH | TPC-PUSCH-RNTI | N/A |  |
| J1 | PDCCH | TPC-PUCCH-RNTI | N/A |  |
| J2 | PDCCH | TPC-SRS-RNTI | N/A |  |
| K | PDCCH | SP-CSI-RNTI | N/A |  |
| L0 | PDCCH | SL-RNTI | SL-SCH |  |
| L1 | PDCCH | SL-CS-RNTI | SL-SCH |  |
| M | PDCCH | SL Semi-Persistent Scheduling V-RNTI | SL-SCH | Note 5 |
| N | PDCCH | PS-RNTI | N/A |  |
| O | PDCCH | AI-RNTI | N/A |  |
| P | PDCCH | CI-RNTI | N/A |  |
| Q | PDCCH | PEI-RNTI | N/A | Note 1 |
| Note 1: These are received from PCell only. Note 2: In some cases UE is only required to monitor the short message within the DCI for P-RNTI. Note 3: These are received from PCell or PSCell. Note 4: This corresponds to PDCCH-ordered PRACH. Note 5: This corresponds to PDCCH scheduling LTE PC5. Note 6: This is for multicast in RRC connected state. Note 7: This corresponds to DL Semi-Persistent Scheduling release for multicast in RRC connected state. Note 8: This is for broadcast MCCH. Note 9: This is for broadcast MTCH. UE is not required to decode more than one PDSCH for MTCH simultaneously. |  |  |  |  |

Table 6.2-2: Downlink "Reception Type" combinations

| Supported Combinations |  |  |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCell | PSCell |  | SCell |  |  |  |
| 1. RRC_IDLE |  |  |  |  |  |  |
| 1.1 All UEs |  |  |  |  |  |  |
| A + (B and/or (C1 or Q) and/or D0) + F0 |  |  |  |  | Note 1 |  |
| 1.2 UEs supporting MBS broadcast reception |  |  |  |  |  |  |
| A+D5 |  |  |  |  |  |  |
| 2. RRC_INACTIVE |  |  |  |  |  |  |
| 2.1 All UEs |  |  |  |  |  |  |
| A + (B and/or (C1 or Q) and/or D0) + F0 |  |  |  |  | Note 1 |  |
| 2.2 UEs supporting MBS broadcast reception |  |  |  |  |  |  |
| A+D5 |  |  |  |  |  |  |
| 3. RRC_CONNECTED |  |  |  |  |  |  |
| (A + ((C0 + (B and/or (D0 or (m1*D1+m2*D2+((m3*D3+m4*D4) or m5*(D5 or D6))))) + E + F0 + n*F1 + G + H + J0 + J1 + J2 + K + O + L0 + L1 + M + N + P) or D5)) | (A + (D0 or (m1*D1+m2*D2)) + E + F0 + n*F1 + G + H + J0 + J1 + J2 + K + O + N + P) |  | m1*D1 + m2*D2 + ((m3*D3+m4*D4) or m5*(D5 or D6)) + E + n*F1 + G + H + J0 + J1 + J2 + K + O + L0 + L1 + M + P |  | Note 2, Note 3, Note 4, Note 5, Note 6, Note 7, Note 8, Note 9, Note 10, Note 11, Note 12, Note 13, Note 14 |  |
| Note 1: UE is not required to decode more than two PDSCH simultaneously, and decoding prioritization when more than two are received is up to UE implementation. Note 2: For PCell, UE is not required to decode SI-RNTI PDSCH simultaneously with C-RNTI PDSCH, unless in FR1. Note 3: Supported combinations are subject to UE capabilities for dual connectivity, carrier aggregation, receiving of group TPC commands, pre-emption indication and dynamic SFI monitoring. Note 4: The values of m2 ≥ 0 and n≥ 0 in the supported combinations are subject to the UE capability. Note 5: Support of monitoring PDCCH with SL-RNTI, SL-CS-RNTI, SL Semi-Persistent Scheduling V-RNTI are subject to UE capability. Note 6: The values of m1 ≥ 1 in the supported combinations are subject to the UE capability. Note 7: In Active time, a UE is not expected to monitor the DCI format for the PDCCH scrambled by PS-RNTI. Note 8: The PDCCH scrambled by PS-RNTI can only be configured on the PCell and PSCell. Note 9: For a UE supporting MBS multicast reception, the values of 1 ≥ m3 ≥ 0 and m4 ≥ 0 are subject to UE capability and applicable to RRC connected UEs. If m3 = 1, then m1 ≤ 1. Note 10: For a UE supporting MBS multicast or broadcast reception, the UE is not expected to be configured simultaneously with more than one component carrier for multicast or broadcast reception. Note 11: For a UE supporting MBS broadcast reception, the values of 1≥m5 ≥ 0 are subject to UE capability and applicable to RRC connected UEs. If m5=1, then m1≤1. Note 12: For a UE supporting MBS broadcast reception in RRC_CONNECTED state, it is required to support reception of FDMed MCCH PDSCH and PBCH in PCell. Note 13: For a UE supporting MBS multicast or broadcast reception in RRC_CONNECTED state, it is not required to support reception of FDMed MCCH/MTCH/multicast PDSCH and SIB PDSCH in PCell. Note 14: For a UE supporting MBS multicast or broadcast reception in RRC_CONNECTED state, it is not required to support reception of FDMed MTCH/multicast PDSCH and PBCH in PCell. |  |  |  |  |  |  |