3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2210213

# e-Meeting, October 10 – 19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.214 | CR |  | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to numerology and CP for positioning in RRC_INACTIVE state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The description of numerology and CP for positioning in RRC_INACTIVE state is incorrect as numerology includes the CP type/length. So the numerology and CP should not be used at the same time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change “numerology” to “subcarrier spacing” in RRC_INACTIVE state in clause 5.1.6.5 and clause 6.2.1.4 of TS 38.214. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of numerology and CP for positioning in RRC_INACTIVE state is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.5, 6.2.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: No inter-operability issue is identified. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.1.6.5	PRS reception procedure

========================= Unchanged parts =========================

The UE in RRC_INACTIVE mode is expected to prioritize the reception of any other DL signals and DL channels than the reception of DL PRS.

The UE in RRC_INACTIVE mode, subject to UE capability, is expected to process DL PRS outside or inside of the initial DL BWP. For DL PRS processing outside of the initial DL BWP, the UE may be configured with the same or different subcarrier spacing and CP for PRS resources than those of the initial DL BWP. For DL PRS processing inside of the initial DL BWP, the UE is configured with the same subcarrier spacing and CP for PRS resources as those of the initial DL BWP.

For a UE configured with preconfigured Measurement gap(s) for Positioning, when the UE receives an activation command, as described in clause 6.1.3.41 of [10, TS 38.321], for a preconfigured Measurement Gap for Positioning activation/deactivation, and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the command, the corresponding actions in [10, TS 38.321] and the UE assumptions shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

For a UE configured with PRS Processing Window(s), when the UE receives an activation/deactivation command, as described in clause 6.1.3.42 of [10, TS 38.321], for a PRS processing window activation, and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the command, the corresponding actions in [10, TS 38.321] and the UE assumptions shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. The UE is not expected to be indicated with more than 4 activated PRS processing windows by higher layer parameter DL-PPW-PreConfig across all active DL BWPs and is not expected to be indicated with the activated PRS processing windows DL-PPW-PreConfig that overlap in time.

6.2.1.4	UE sounding procedure for positioning purposes

========================= Unchanged parts =========================

Subject to UE capability, the UE may be configured with an SRS resource for positioning associated with the initial UL BWP, and the SRS resource is transmitted inside the initial UL BWP during RRC_INACTIVE mode with the same CP and subcarrier spacing as configured for the initial UL BWP. Subject to UE capability, the UE may be configured with an SRS resource for positioning outside the initial BWP including frequency location and bandwidth, subcarrier spacing, and CP length for transmission of the SRS in RRC_INACTIVE mode. If the transmission of SRS for positioning outside the initial BWP in RRC_INACTIVE mode with the switching time in unpaired spectrum, subject to UE capability, collides in time domain with other DL signals or channels or UL signals or channels, the SRS for positioning transmission is dropped in the symbol(s) where the collision occurs. If the transmission of SRS for positioning outside the initial BWP in RRC_INACTIVE mode with the switching time in paired spectrum or SUL band, subject to UE capability, collides in time domain with UL signals or channels on the same carrier, the SRS for positioning transmission is dropped in the symbol(s) where the collision occurs. The SRS resource for positioning outside the initial BWP in RRC_INACTIVE mode is configured in the same band and CC as the initial UL BWP.

If the UE in RRC_INACTIVE mode determines that the UE is not able to accurately measure the configured DL RS in SRS-SpatialRelationInfoPos for a SRS resource for positioning where the DL RS is semi-persistent or periodic, the UE stops transmission of the SRS resource for positioning.

The UE is not expected to simultaneously transmit SRS resources configured by the higher layer parameter SRS-PosResource on NUL and SUL band in RRC_INACTIVE mode.