3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2210217

e-Meeting, October 10 – 19, 2022

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion LS on Pemax,c of S-SSB transmission when multiple resource pool is configured in a carrier

Document for:	Discussion and Decision

- Introduction

A RAN4 LS was sent to RAN1 [1], where the following question was asked:

Question 1: RAN4 agreed not to define PEMAX,c for S-SSB configured Tx Power in TS38.101-1 as highlighted yellow below and would like to know if this can have any impact to RAN1/2, for example, Pcompensation defined in idle mode procedure in TS 38.304.

6.2E.4          Configured transmitted power for V2X

6.2E.4.1    General

The NR V2X UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f, c = MIN {PEMAX,c,  PPowerClass, V2X – MAX(MAX(MPRc , A-MPRc) + DTIB,c , P-MPRc), PRegulatory,c }

PCMAX_H,f, c = MIN {PEMAX,c, PPowerClass, V2X,  PRegulatory,c }

where

-  PCMAX,f,c is configured for PSSCH\PSCCH, S-SSB and PSFCH, respectively;

-  For the total transmitted power PCMAX,PSSCH/PSCCH, PEMAX,c is the value given by IE sl-maxTransPower, defined by TS 38.331.

-  For the total transmitted power PCMAX,S-SSB, the PCMAX_L,f,c and PCMAX_H,f,c are defined as follows:

PCMAX_L,f,c = MIN {PPowerClass, V2X – MAX(MAX(MPRc , A-MPRc) + DTIB,c , P-MPRc), PRegulatory,c}

PCMAX_H,f,c = MIN {PPowerClass, V2X,  PRegulatory,c}

RAN4 would like to know if there is an impact on RAN1 and RAN2 about the agreement RAN4 made on S-SSB.

RAN1 was asked to provide feedback on the above question. In this contribution, we discuss how to draft the reply LS to RAN4.

- Discussions

As per TS 38.213, the determination of power for S-SS/PSBCH blocks depends on PCMAX, which is defined in [8-1, TS 38.101-1]:

- 16.2.0 S-SS/PSBCH blocks A UE determines a power PS-SSB(i) for an S-SS/PSBCH block transmission occasion in slot i on active SL BWP b of carrier f as PS-SSB(i)=minPCMAX,PO,S-SSB+10log102μ∙MRBS-SSB+αS-SSB⋅PL [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PO,S-SSB is a value of dl-P0-PSBCH if provided; else, PS-SSB(i)=PCMAX - αS-SSB is a value of dl-Alpha-PSBCH, if provided; else, αS-SSB=1 - PL=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - MRBS-SSB=11 is a number of resource blocks for a S-SS/PSBCH block transmission with SCS configuration μ

From RAN1 perspective, PCMAX is subjected to what RAN4 defines, and therefore there is no RAN1 spec impact if RAN4 agrees not to use a (pre-)configured value to define the PEMAX,c for S-SSB TX power in TS 38.101-1.

Proposal 1:  Reply to RAN4 as follows:

- From RAN1 perspective, PCMAX is subject to what RAN4 defines, and therefore there is no RAN1 specification impact from RAN4 agreements.

- Conclusions

Following proposals were made:

Proposal 1:  Reply to RAN4 as follows:

- From RAN1 perspective, PCMAX is subject to what RAN4 defines, and therefore there is no RAN1 specification impact from RAN4 agreements.

# References

- R4-2214421, “LS on Pemax,c of S-SSB transmission when multiple resource pool is configured in a carrier”, RAN4, RAN4#104-e, August, 2022.