3GPP TSG-RAN WG1 Meeting #110bis-e	R1- 2210218

e-Meeting, October 10 – 19, 2022

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on the RAN4 LS on active TCI state list for UL TCI

Document for:	Discussion and Decision

# Introduction

In the last meeting, RAN4 has sent RAN1 an LS [1] on pathloss reference RS maintenance under the unified TCI state framework. In this paper, we provide our answers to the questions in [1].

# Discussion

RAN1 has agreed in RAN1#103e that as in Rel-16, a UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions.

Agreement On Rel.17 unified TCI framework: - Select at least one of the following alternatives by RAN1#104bis-e for path-loss measurement (PL-RS): - Alt1. PL-RS can be included in UL TCI state or (if applicable) joint TCI state. - FFS: Whether it is always included or not. If not included, PL-RS is the periodic DL-RS used as a source RS for determining spatial TX filter or the PL RS used for the UL RS in UL or (if applicable) joint TCI state. - Alt2. PL-RS can be associated with (but not included in) UL TCI state or (if applicable) joint TCI state - FFS: Exact association mechanism - FFS: Whether it is always associated or not. If not associated, PL-RS is the periodic DL-RS used as a source RS for determining spatial TX filter or the PL RS used for the UL RS in UL or (if applicable) joint TCI state - Alt3. The periodic DL-RS used as a source RS for determining spatial TX filter can be used as PL-RS. In case the periodic DL-RS used as a source RS for determining spatial TX filter is not used as PL-RS, reuse Rel.16 procedure with the same signaling structure (MAC CE+SRI field in UL-related DCI) to indicate PL-RS for UL transmission with minimum enhancement (e.g. pertaining to the use for PUCCH, or using default PL-RS) - PL-RS is not additionally configured in or associated to UL TCI state or (if applicable) joint TCI state - Alt4. UE calculates path-loss based on periodic DL RS configured as the source RS or a periodic QCL-Type-D/spatialRelationInfo source of the source RS in UL TCI state or (if applicable) joint TCI state - FFS: Whether UE can calculate path-loss based on DL periodic RS for path-loss calculation for UL RS in the UL TCI - FFS: Application time of PL-RS - NOTE: As in Rel-16, a UE does not expect to simultaneously maintain more than four path-loss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions

Unlike Rel-16, in Rel-17 unified TCI framework, there is no MAC-CE for PUSCH/SRS pathloss RS update and the pathloss for PUCCH is not configured in PUCCH power control configuration. In Rel-17, for unified TCI framework, the pathloss reference signal is associated to an UL (or joint) TCI state where pathlossReferenceRS-Id-r17 is included in the TCI state configuration. Similar to previous releases, up to 8 TCI states can be activated by MAC-CE for the UE. UE can be further indicated a joint TCI or UL TCI state per TRP from the set of activated TCI states.

Observation 1: In Rel-17, the pathloss for all UL channels/signals can be derived based on the pathloss RS associated to the activated/indicated TCI states.

Regarding the RS resource indexes qd, they are used in TS 38.213 for power control specifications and they are inherited from Rel-16. A Pathloss Reference RS Update MAC-CE is not defined for Rel-17. Instead, UE would determine the RS resource indexes qd from pathlossReferenceRS-Id-r17 included in the active and indicated TCI states.

Observation 2: UE would determine the RS resource indexes qd from pathlossReferenceRS-Id-r17 included in the active and indicated TCI states.

For a fast TCI state switching among activated TCI state, as defined in TS 38.133, UE is expected to maintain the pathloss RSs of all the pathloss reference RS in the active UL (or joint) TCI list.

Observation 3: UE is expected to maintain the pathloss RSs of all the pathloss reference RSs in the active UL (or joint) TCI states.

UE is not expected to simultaneously maintain more than four path-loss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions.

To avoid any unexpected behaviours of UE, network shall ensure the maximum number of PL-RS which UE is capable to maintain is no more than four. For example, network can configure the same pathloss reference RS for some of the active UL (or joint) TCI states to ensure that the total number of maintained pathloss reference RSs is no more than four for a UE per serving cell.

Observation 4: By implementation, network can configure the same pathloss reference RS for some of the active UL (or joint) TCI states to ensure that the total number of maintained pathloss reference RSs is no more than four for a UE per serving cell.

Proposal: Consider above observations and take them as a reference to draft the LS reply to RAN4.

# Conclusion

In this contribution, we observe and propose following:

Observation 1: In Rel-17, the pathloss for all UL channels/signals can be derived based on the pathloss RS associated to the activated/indicated TCI states.

Observation 2: UE would determine the RS resource indexes qd from pathlossReferenceRS-Id-r17 included in the active and indicated TCI states.

Observation 3: UE is expected to maintain the pathloss RSs of all the pathloss reference RSs in the active UL (or joint) TCI states.

Observation 4: By implementation, network can configure the same pathloss reference RS for some of the active UL (or joint) TCI states to ensure that the total number of maintained pathloss reference RSs is no more than four for a UE per serving cell.

Proposal: Consider above observations and take them as a reference to draft the LS reply to RAN4.

# References

[1] R4-2214972/R1-2208354 LS on active TCI state list for UL TCI, RAN WG4(Nokia), August 15 – August 26, 2022