3GPP TSG RAN WG1 #110bis-e	R1-2210257

e-Meeting, October 10th – 19th, 2022

Source: 	MediaTek Inc.

Title: 	Network Energy Savings Performance Evaluation

Agenda item:	9.7.1

Document for:	Discussion and Decision

# Introduction

The following agreements have been endorsed after RAN1#110:

Agreement in RAN1#110 For non-sleep mode, the relative power value in power model table for UL reception and/or DL transmission is provided based on reference configuration. Agreement in RAN1#110 For set 2 FR1 FDD TxRx reference configuration, confirm the WA as 32 in reference configuration. Agreement in RAN1#110 The total DL power level is 49 dBm for set 2 FR1 FDD reference configuration. Agreement in RAN1#110 For the purpose of evaluation, adopt the following as BS power consumption model. These entries for this table is per reference configuration set. - FFS: One or multiple values for relative power and transition time. Power state; Characteristic; Relative Power; Additional transition energy3; Total transition time. Deep sleep1; There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state.; P1=1; E1; T1. Light sleep; There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state. (P2>P1); P2; E2; T2. Micro sleep; There is neither DL transmission nor UL reception. Immediate transition is assumed for network energy saving study purpose from or to a non-sleep state.; P3; 0; 0. Active DL; There is only DL transmission.; P4; NA; NA. Active UL; There is only UL reception. FFS: Whether multiple P5 values are needed to address low power UL mode; P5; NA; NA. Note 1: Depending on implementations, there could be a state that the power is lower than deep sleep and requires larger total transition time, e.g. hibernating sleep or Quasi-off, which is not explicitly modeled in this study for evaluation purpose. Note 3: Unit in relative power times duration. FFS: Details on how transition energy is defined.. - For simultaneous DL and UL transmission for FDD, the power for UL reception is neglected in this study. - FFS: Optionally, a state machine where BS may transit between sleep modes without entering non-sleep mode can be considered. Companies are to report the involved sleep modes and the assumptions for inter-sleep mode transition time used in their evaluations. - FFS: Details on how to use the above table for low power uplink reception (e.g. for WUS). Working Assumption in RAN1#110 For reference configuration set 1, the values are provided as below. FFS set2 and set 3. Power state; Relative Power P; Total transition time T. Deep sleep; 1; 1; Cat 1: 50ms; Cat 2: 10s. Light sleep; Cat 1: 25; Cat 2: 2.1; Cat 1: 6 ms; Cat 2: 640 ms. Micro sleep; Cat1: 55; Cat 2: 5.5; 0; 0. Active DL; Cat 1: 280; Cat 2: 32; N.A.; N.A.. Active UL; Cat 1: 110; Cat 2: 6.5; N.A.; N.A.. Agreement in RAN1#110 For evaluation purpose, - a load (L) of a cell is a percentage of resources used for UE specific PDSCH / PUSCH - The following load scenarios are considered Load scenario; Characteristics. Idle/empty load; - Include cell-specific signals and channels, and - L = 0. low load; - Include cell-specific signals and channels, and - 0 < L ≤15. Light load; - Include cell-specific signals and channels, and - 0 < L ≤ [30]. Medium load; - Include cell-specific signals and channels, and - [30] < L ≤ [50]. For CA, the companies report whether the load is defined per CC or across all CCs.. Agreement in RAN1#110 - For FR1, urban micro can be optionally considered. - For FR2, urban micro is prioritized, with ISD=200 m is assumed. Agreement in RAN1#110 It is up to company report which traffic model is used among the agreed three traffic models in their evaluations. - Other models may be used as well. Parameter (e.g. packet size and arrival rate) adjustment can be optionally considered and reported. Agreement in RAN1#110 For set 3 FR2 reference configuration, the total DL power level and EIRP limit is set as 33 dBm and 63 dBm respectively. Note EIRP limit is also scaled with the number of TxRU. Agreement in RAN1#110 For evaluation purpose, network energy saving gain is computed based on the energy consumptions for a technique and the baseline over the same duration. Agreement in RAN1#110 For initial evaluations, there is always a non-sleep mode assumed between adjacent sleep modes. Agreement in RAN1#110 Companies to report the assumption details for the reception of a low-power UL channel/signal, if used, including power states, additional transition energy, and transition times, receiver details (e.g. architecture and receiver sensitivity), and other impact/change on the power consumption model. Agreement in RAN1#110 Update the RAN1 agreements with the following changes In the evaluation, - a load (L)%of a cell is a percentage of resources used for UE specific PDSCH/PUSCH. - The following load scenarios are considered. Load scenario; Characteristics. Idle/empty load; - Include cell-specific signals and channels, and - L = 0. low load; - Include cell-specific signals and channels, and - 0 < L ≤ 15. Light load; - Include cell-specific signals and channels, and - 150 < L ≤ 30. Medium load; - Include cell-specific signals and channels, and - 30 < L ≤ 50. For CA, the companies report whether the load is defined per CC or across all CCs.. Agreement in RAN1#110 - During the transition time period, relative power of sleep mode i is assumed to be consumed. Additional transition energy and total transition time also include energy and time for both ramping down and ramping up Agreement in RAN1#110 - The total transition time for set 2 and set 3 is the same as that for set 1. - Companies are encouraged to check the input and values provided in section 2.1.4.2 of R1-2208312 for further determination. Conclusion - Companies are encouraged to check discussion in section 2.2.2 of R1-2208312 for scaling discussion in the next meeting. Agreement in RAN1#110 - FFS whether to set exact requirements/QoS target for UPT and/or latency impact - Other KPIs can be optionally reported, conditioned with clear definition/descriptions provided. - Note for potential new channel/signals, e.g. WUS from UE, the assumption for detection reliability at BS side is reported (performance and complexity impact would subject to results and further discussion). Agreement in RAN1#110 It is up to company report the use of UE C-DRX. - the baseline configuration (for alignment/calibration) for C-DRX, if reported, can be as below; - Other inactivity timer values can be optionally reported Traffic type; FTP; IM; VoIP. Model; FTP model 3; FTP model 3; As defined in R1-070674. Assume max two packets bundled.. Packet size; 0.5 Mbytes; 0.1 Mbytes. Mean inter-arrival time; 200 ms; 2 sec. DRX Period; 160 ms; 320 ms; 40 ms. DRX Inactivity timer; 100 ms; 80 ms; 10 ms. On duration; FR1: 8 ms FR2: 4 ms; FR1: 10 ms FR2: 5 ms; FR1: 4 ms FR2: 2 ms. Agreement in RAN1#110 - For FR1, adopt the Reference SLS configurations in Annex-A in R1-2208312 as baseline SLS assumptions. - Other carrier frequencies can be optionally considered. - FFS For FR2 adopt the Reference SLS configuration used in Dense Urban Config.B in Table2 of RP-180524 for IMT-2020 with the following clarification/update as initial SLS assumption. - BS antenna configurations - 2 TxRU (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1) - (dH, dV) = (0.5λ, 0.8λ) (dg,H, dg,V) = (4.0λ, 3.6λ) - Traffic model & UE density - Follow previous agreements with adjusted UE density - Total transmit power per TRxP - Value scaled from that in set 3 reference configuration considering BW - Further adjustment/clarification can be discussed in the next meeting.

To accomplish the evaluation methodology, this contribution investigates the following remaining issues:

- Remaining Details on BS Power Consumption Values

- BS Power Consumption Scaling

- Remaining simulation assumption (SSB and SIB1 configurations)

# Remaining Details on BS Power Consumption Values

## Additional transition energy for sleep modes

According to FL summary [1], the additional transition energy Ei represents the energy that BS enters from non-sleep mode to a sleep mode i and BS leaves the same sleep mode to non-sleep mode. For evaluation purpose, it is calculated as

Ei≈12×∆×Ti,

where ∆ is the difference of the relative power between sleep mode i and micro sleep, and  Ti is the corresponding total transition time of sleep mode i, which is a two-way time, assuming no inter-sleep state transition.

For set 1, the additional energy (unit in relative power*(duration in ms)) is given by

| Power state | Additional transition energy Ei |  |
| --- | --- | --- |
|  | Category 1 | Category 2 |
| Deep sleep | 1350 | 22500 |
| Light sleep | 90 | 1088 |

With the proposed transition energy values, it should be reasonable to check that, at the sleep time threshold for a deeper sleep state, choosing the deeper sleep state can achieve lower power consumption with the additional transition energy values.

However, if we jointly check the proposed sleep values for Set 2 and Set 3, there is inconsistency for 
Category 2 BS with Set 2 values:

| Category 1 BS (Set 1; 30kHz SCS) | Category 2 BS (Set 1; 30kHz SCS) | Category 2 BS (Set 2; 15kHz SCS) |
| --- | --- | --- |
| Micro sleep for 6ms = 55*6*2 = 660 Light sleep for 6ms = 25*6*2 + 90 = 390 < 660 | Micro sleep for 640ms = 5.5*640*2 = 7040 Light sleep for 640ms = 2.1*640*2 + 1088 = 3776 < 7040 | Micro sleep for 640ms = 5*640 = 3200 Light sleep for 640ms = 2.6*640 + 1088 = 2752 < 3200 |
| Light sleep for 50ms = 25*50*2+90 = 2590 Deep sleep for 50ms = 1*50+1350 = 1450 < 2590 | Light sleep for 10s = 2.1*10000*2 + 1088 = 43088 Deep sleep for 10s = 1*10000*2+22500 = 42500 < 43088 | Light sleep for 10s = 2.6*10000 + 1088 = 27088 Deep sleep for 10s = 1*10000+22500 = 32500 > 37088 (1*10000+17000 = 27000 < 27088) |

To ensure consistency across Sets 1, 2 and 3, we suggest reducing the transmission energy Ei for deep sleep of Category 2 BS from 22500 to 17000, which ensures deep sleep, whenever selected according the sleep time threshold, achieves lower power consuming than light sleep. Accordingly, the following proposal is suggested:

Proposal 1: Endorse the following table of transition energy values for deep and light sleeps

| Power state | Additional transition energy Ei |  |
| --- | --- | --- |
|  | Category 1 | Category 2 |
| Deep sleep | 1350 | 22500 17000 |
| Light sleep | 90 | 1088 |

## Power consumption values for reference configuration Set 2 and Set 3

If proposal 1 can be agreed, there is no issue with the power value table suggested in FL summary [1] for Set 2 and Set 3. In this regard, we also have the following proposal:

Proposal 2: If the update in Proposal 1 is agreed, endorse the following table for Set 2 and Set 3:

| Power state | Relative Power P for Category 1 |  | Relative Power P for Category 2 |  |
| --- | --- | --- | --- | --- |
|  | Set 2 | Set 3 | Set 2 | Set 3 |
| Deep sleep | 1 | 1 | 1 | 1 |
| Light sleep | 23 | 20 | 2.6 | 1.8 |
| Micro sleep | 50 | 38 | 5 | 3 |
| Active DL | 240 | 152 | 40 | 8.4 |
| Active UL | 90 | 80 | 5.8 | 4.2 |

# BS Power Consumption Scaling

BS power consumption scaling is critical component of the evaluation methodology for characterizing network energy saving techniques. We will discuss the remaining issues of the scaling model from different aspects in each of the following subsections.

## DL scaling model

From the FL summary [1], there are two possible models:

- Revised Alt 1-update:

  - P= Pstatic+Pdynamic

  - Pdynamic=sa*Pdyn,ante+sf*spηsf,  sp*Pdyn,joint

  - TBD values of Pstatic, Pdyn,ante, Pdyn,joint and ηsf,  sp

- Revised Alt 3:

  - Psa,sf,sp=P3+sasfspηsf, spP4-P3

  - P3 and P4 are relative power values of micro sleep and active DL transmission, respectively

  - Companies report how to model ηsf, sp.

    - Note: modelling ηsf, sp as a single value for all different power and frequency resource allocation values is not realistic

- Where

  - sf is the resource usage ratio in frequency domain

  - sp is ratio of simulated DL power level per TxRU between the DL transmission and reference configuration

  - sa is percentage of active TxRUs with respect to the reference configuration

Comparing the two models, it looks common in the form P= Pstatic+Pdynamic. In Revise Alt 3, Pstatic=P3, i.e., the micro sleep power, which is also one proposed Pstatic value for Revised Alt 1-update. To merge the two models as much as possible, the following is there for suggested:

Proposal 3: For BS DL power consumption, P= Pstatic+Pdynamic with Pstatic=P3, i.e., the micro sleep power.

For Pdynamic , there exhibits common principle in the two candidate models that the power is linearly scaled w.r.t. sa. For the scaling related to sf and sp , there is also common understanding that the power consumption scaling is NOT linearly scaled w.r.t. sf and sp. Since Revise Alt 3 leaves this part of model for companies’ reports, this somehow poses difficulty for RAN1 to proceed further evaluation on NW ES techniques. In this regard, utilizing the proposal in Revised Alt 1-update as the baseline model to move forward would be a reasonable approach.

Proposal 4: For Pdynamic in Proposal 3,

- It is linearly scaled w.r.t. sa (percentage of active TxRUs with respect to the reference configuration)

- For the scaling w.r.t. sf and sp, adopt Pdyn,ante+sf*spη*Pdyn,joint as the baseline model, where

  - sf is the resource usage ratio in frequency domain

  - sp is ratio of simulated DL power level per TxRU between the DL transmission and reference configuration

  - η is a constant; FFS whether/how to use a non-linear function to represent η.

For the value of Pdyn,ante , we have the preference that the scaling can lead to 0.4× Active DL Power when sa= 1 while sf = 0, as suggested in Section 2.4 of [2]. For the value of η, it is a placeholder value and setting to a candidate value from companies’ inputs as working assumption should be acceptable. Finally, Pdyn,joint can be chosen so that P= Active DL power when sa=sf=sp=1. Accordingly, the following proposal is suggested:

Proposal 5: For the scaling w.r.t. sf and sp in Proposal 4,

- Pdyn,ante is chosen so that P=0.4× Active DL power when sa= 1 while sf = 0

  - Accordingly, the following values are utilized for different BS categories and Set 1/2/3

| Value of Pdyn,ante |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Category 1 BS |  |  | Category 2 BS |  |  |
| Set 1 | Set 2 | Set 3 | Set 1 | Set 2 | Set 3 |
| 57 | 46 | 22.8 | 7.3 | 11 | 0.36 |

- η=[0.5]

- Pdyn,joint is chosen so that P= Active DL power when sa=sf=sp=1

  - Accordingly, the following values are utilized for different BS categories and Set 1/2/3

| Value of Pdyn,joint |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Category 1 BS |  |  | Category 2 BS |  |  |
| Set 1 | Set 2 | Set 3 | Set 1 | Set 2 | Set 3 |
| 84 | 72 | 45.6 | 9.6 | 12 | 2.52 |

## UL Scaling model

For UL power consumption, it is related to BS receiver processing. Given this portion of power consumption is much smaller than DL transmission power consumption, a simple model is preferable for companies’ evaluations. In this regard, the proposal in Revised Alt 1-update is recommended:

Proposal 6: The BS power consumption for active UL is provided by

  - P=PstaticUL+sa*PdynamicUL

  - PstaticUL=P3, i.e., the micro sleep power

  - PdynamicUL is chosen so that P= Active UL power when sa=1

    - Accordingly, the following values are utilized for different BS categories and Set 1/2/3

| Value of PdynamicUL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Category 1 BS |  |  | Category 2 BS |  |  |
| Set 1 | Set 2 | Set 3 | Set 1 | Set 2 | Set 3 |
| 55 | 40 | 42 | 1 | 0.8 | 1.2 |

## Time domain scaling

For the time-domain scaling, there is need to extend existing formula for special slot with DL, gap, and UL. Assuming PStatic for DL and UL both equal to the micro sleep power, the following proposal is suggested:

Proposal 7: BS power consumption for a slot is modeled as:

P= PStatic + αDL*PdynamicDL + αUL*PdynamicUL

where αDL and αUL represents the ratios of the number of active DL and UL symbols to the number of total symbols in a slot, respectively.

## Scaling for PA-related transceiver processing enhancements

The purpose of power consumption scaling model is to characterize network energy saving techniques. With Proposals 3 – 7, it is ready for companies to evaluate NW ES techniques in frequency, spatial, power/PSD and time domains. However, it is not clear how to model the power consumption difference with PA-related transceiver processing enhancements. If RAN1 doesn’t have sufficient RF knowledge, resorting to RAN4 for providing suggested power consumption scaling for the PA-related enhancements can be a practical way forward to resolve the issue.

Proposal 8: If agreed, send a LS to request RAN4 for providing suggested power consumption scaling for PA-related transceiver processing enhancements.

# Remaining Details on Simulation Assumptions

According to FL summary [1], SIB1 setting remains to be specified. In recent RedCap study that evaluated SIB1 performance impact, the following SIB1 setting is considered as reference [3]. Given SIB1 TBS of 1256 bits, 48 RBs can be reasonably assumed for performance reason. Since SIB1 is also associated with SSB and transmitted along the SSB beams, a burst of SIB1 of 8 slots can be modelled, corresponding to the assumption of 8 SSBs in a SS burst. On the other, since transmission of a SIB1 burst is quite resource consuming (due to >2x RBs and 2x slots compared with a SS burst), it would be reasonable to assume longer periodicity, e.g., 80 ms, compared with that of a SS burst.

| Parameters | Values |
| --- | --- |
| PRBs/TBS/MCS | TBS is 1256 bits. SIB1 bandwidth is larger than 5MHz, e.g. 48PRBs. Companies to report the used number of PRBs, corresponding MCS, and whether interleaving mapping is assumed. |

By the above discussion, the following assumptions for SSB and SIB1 are suggested:

Proposal 9: Assume the following configuration for BS transmission of SSB and SIB1 at least for FR1 macro scenario:

- SSB: 20 RBs, 4 symbols, 2 SSB per slot, and 4 consecutive slots at periodicity of 20 ms

- SIB1: 48 RBs, 8 consecutive slots at periodicity of 80 ms, multiplexing pattern 1 with SSB

# Conclusion

In this contribution, we have the following proposals:

Proposal 1: Endorse the following table of transition energy values for deep and light sleeps

| Power state | Additional transition energy Ei |  |
| --- | --- | --- |
|  | Category 1 | Category 2 |
| Deep sleep | 1350 | 22500 17000 |
| Light sleep | 90 | 1088 |

Proposal 2: If the update in Proposal 1 is agreed, endorse the following table for Set 2 and Set 3:

| Power state | Relative Power P for Category 1 |  | Relative Power P for Category 2 |  |
| --- | --- | --- | --- | --- |
|  | Set 2 | Set 3 | Set 2 | Set 3 |
| Deep sleep | 1 | 1 | 1 | 1 |
| Light sleep | 23 | 20 | 2.6 | 1.8 |
| Micro sleep | 50 | 38 | 5 | 3 |
| Active DL | 240 | 152 | 40 | 8.4 |
| Active UL | 90 | 80 | 5.8 | 4.2 |

Proposal 3: For BS DL power consumption, P= Pstatic+Pdynamic with Pstatic=P3, i.e., the micro sleep power.

Proposal 4: For Pdynamic in Proposal 3,

- It is linearly scaled w.r.t. sa (percentage of active TxRUs with respect to the reference configuration)

- For the scaling w.r.t. sf and sp, adopt Pdyn,ante+sf*spη*Pdyn,joint as the baseline model, where

  - sf is the resource usage ratio in frequency domain

  - sp is ratio of simulated DL power level per TxRU between the DL transmission and reference configuration

  - η is a constant; FFS whether/how to use a non-linear function to represent η.

Proposal 5: For the scaling w.r.t. sf and sp in Proposal 4,

- Pdyn,ante is chosen so that P=0.4× Active DL power when sa= 1 while sf = 0

  - Accordingly, the following values are utilized for different BS categories and Set 1/2/3

| Value of Pdyn,ante |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Category 1 BS |  |  | Category 2 BS |  |  |
| Set 1 | Set 2 | Set 3 | Set 1 | Set 2 | Set 3 |
| 57 | 46 | 22.8 | 7.3 | 11 | 0.36 |

- η=[0.5]

- Pdyn,joint is chosen so that P= Active DL power when sa=sf=sp=1

  - Accordingly, the following values are utilized for different BS categories and Set 1/2/3

| Value of Pdyn,joint |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Category 1 BS |  |  | Category 2 BS |  |  |
| Set 1 | Set 2 | Set 3 | Set 1 | Set 2 | Set 3 |
| 84 | 72 | 45.6 | 9.6 | 12 | 2.52 |

Proposal 6: The BS power consumption for active UL is provided by

  - P=PstaticUL+sa*PdynamicUL

  - PstaticUL=P3, i.e., the micro sleep power

  - PdynamicUL is chosen so that P= Active UL power when sa=1

    - Accordingly, the following values are utilized for different BS categories and Set 1/2/3

| Value of PdynamicUL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Category 1 BS |  |  | Category 2 BS |  |  |
| Set 1 | Set 2 | Set 3 | Set 1 | Set 2 | Set 3 |
| 55 | 40 | 42 | 1 | 0.8 | 1.2 |

Proposal 7: BS power consumption for a slot is modeled as:

P= PStatic + αDL*PdynamicDL + αUL*PdynamicUL

where αDL and αUL represents the ratios of the number of active DL and UL symbols to the number of total symbols in a slot, respectively.

Proposal 8: If agreed, send a LS to request RAN4 for providing suggested power consumption scaling for PA-related transceiver processing enhancements.

Proposal 9: Assume the following configuration for BS transmission of SSB and SIB1 at least for FR1 macro scenario:

- SSB: 20 RBs, 4 symbols, 2 SSB per slot, and 4 consecutive slots at periodicity of 20 ms

- SIB1: 48 RBs, 8 consecutive slots at periodicity of 80 ms, multiplexing pattern 1 with SSB

# Reference

R1-2208312, “FL summary for Post-110-R18- NW_ES2”, Moderator (Huawei)

R1- 2206979, “NW energy savings performance evaluation”, RAN1#110, MediaTek Inc.

TR 38.865, “Study on further NR RedCap UE complexity reduction (Release 18)”