3GPP TSG RAN WG1 Meeting #110	bis-e																		R1-2210268

e-meeting, Oct 10th – 19th, 2022

Source:	Moderator (CATT)

Title:	FL Summary #2 for improved accuracy based on NR carrier phase measurement

Agenda item:	9.5.2.2

Document for:	Discussion and Decision

# Introduction

The Rel-18 SI “Study on expanded and improved NR positioning” was approved in RAN1#94e (RP-213588) [24], which includes the following objective:

- Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4]

  - Reference signals, physical layer measurements, and physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1]

  - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary

This contribution summarizes the following email discussion for the issues and proposals submitted to RAN1#110bis-e for above objective:

[110bis-e-R18-Pos-05] Email discussion on improved positioning accuracy based on NR carrier phase measurement by October 19 – Ren (CATT)

- Check points: October 14, October 19

The discussion includes the following aspects:

- Reference signals for NR carrier phase measurements

- Physical layer measurements for NR carrier phase positioning

  - DL/UL carrier phase measurements

  - Carrier phase measurements of multiple frequencies

  - Phase differences for DL-AOD

  - Phase-smoothed timing measurements

  - Other measurements

- NR carrier phase positioning techniques and physical layer procedures

  - Differential carrier phase positioning with PRU

  - Integer ambiguity

  - Multipath mitigation

  - Antenna phase center offsets

  - RTT carrier phase positioning

  - Additional issues

- Evaluation and observations

  - Evaluation assumptions

  - Evaluation results and observations

- Others

  - TRP time synchronization

  - Sidelink carrier phase positioning

Note: In the summary, the FL proposals marked with (H) are chosen as high-priority proposals for online/offline discussions. However, it is encouraged to provide inputs for all FL proposals. The FL may adjust the priority of the proposals based on the inputs of the interested companies during the meeting as necessary.

# (Closed)Reference Signals for Carrier Phase Measurements

Submitted Proposals:

- (vivo, R1-2208650[2]) Proposal 1:	To reuse the existing signal (SRS, or PRS) for carrier phase measurement.

- (China Telecom, R1-2208774[4]) Proposal 1: reuse the existed DL-PRS or UL SRS-Pos for phase based positioning.

- (China Telecom, R1-2208774[4]) Proposal 2: The possible modification on the pattern of reference signal especially the supplement of the short transmission period can be considered.

- (OPPO, R1-2208801[5]) Proposal 1: The NR phase measurement-based positioning shall be based on the existing DL PRS and SRS for positioning.

- (CATT, R1-2208983[6]) Proposal 7: The existing Rel-16 DL PRS and SRS for positioning can be used as the reference signal for supporting NR carrier phase positioning in Rel-18.

  - Note: Further optimization of the reference signal on top of existing R16 DL PRS and SRS for positioning may also be considered in Rel-18 or a future release.

- (Lenovo, R1-2209395[12]) Proposal 2: To improve the positioning performance of NR by applying the carrier phase positioning method, the PRS waveform should be continuous. This is enabled by performing raised cosine technique over the PRS waveform.

- (Lenovo, R1-2209395[12]) Proposal 3: To improve the positioning performance of NR by applying the carrier phase positioning method, contiguous mapping of PRS generating sequence in time domain needs to be studied.

- (Lenovo, R1-2209395[12]) Proposal 4: The range of values of the frequency offset  koffset  and the number of contiguous symbols LPRS for accurate carrier phase measurements need to be further investigated.

- (Lenovo, R1-2209395[12]) Proposal 5: RAN1 to further study the support of narrower bandwidth configurations for carrier phase-based positioning.

- (InterDigital, R1-2209489[13]) Proposal 5: Study new PRS designs for carrier phase measurements for UL and DL.

- (InterDigital, R1-2209489[13]) Proposal 6: A method to differentiate multiple PRSs, if they are transmitted at the same time-frequency resource, should be identified.

- (InterDigital, R1-2209489[13]) Proposal 13: Support both UE-based and UE-assisted NR carrier phase positioning.

- (Locaila, R1-2209543[15]) Proposal 6) For efficient carrier phase measurement and synchronization, re-designing of current NR PRS is necessary.

- (Locaila, R1-2209543[15]) Proposal 7) Study the proposed continuous PRS sequence with subcarrier allocation method to improve the phase measurement accuracy.

- (Samsung, R1-2209738[17]) Proposal 1: Study the use of Rel-16 DL and UL positioning reference signals with no or small Comb for the carrier phase method.

- (Samsung, R1-2209738[17]) Proposal 2: Study the use of Rel-16 DL and UL positioning reference signals using the same frequency offset (k') across OFDM symbols of a slot for the carrier phase measurement.

FL Comments:

One of the SI objectives is to study the Reference signals for the NR carrier phase measurements. Whether to introduce new reference signals for carrier phase measurements was intensively discussed in previous meetings.  The majority of companies support the reuse of the existing positioning signal (DL PRS or UL SRS) for carrier phase measurement (e.g., [25][26] [2, 4, 5, 6]). The obvious advantage of reusing Rel-16 DL and UL positioning reference signals is the minimum impact on the existing specification and the implementation. That is, it has no physical layer impact on the generation of the reference signals and may only have minimal impact on the reception of the reference signals. In addition, the carrier phase measurements can be obtained together with other existing positioning measurements from DL PRS and/or UL SRS for positioning. Based on the analysis and evaluation results provided by the majority of companies, it is clear that Rel-16 DL and UL positioning reference signals can be re-used as the reference signal for supporting NR carrier phase positioning.

On the other hand, when Rel-16 DL and UL positioning reference signals were designed, there is no consideration for the carrier phase positioning. Thus, some companies suggest modifications (e.g., [4][17]) or re-design of the reference signals (e.g., [12][13][15]), which could potentially provide better performance of carrier phase measurements.

As described in the following objective of the SID [24]:

- Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4]

  - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1]

  - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary

The focus of the study is on the reuse of existing PRS and SRS and the new reference signals are only considered if found necessary. Based on the investigation so far, it is clear that the existing PRS and SRS can be reused as the reference signals to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning. Thus, the FL suggests making a conclusion in this meeting. Then, we can decide whether to consider further enhancements to the reference signals in the next meeting.

## (H) (Round 1) Proposal 2-1.1

- The existing DL PRS and UL SRS for positioning can be re-used as the reference signals to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning.

- FFS: Whether to consider other enhancements of the existing DL PRS and UL SRS for better positioning performance

| Company | comments |
| --- | --- |
| vivo | Okay |
| Nokia/NSB | Support |
| CATT | Support |
| Fraunhofer | Support |
| Samsung | We can accept for progress. Regarding enhancements, we think that we should at least consider allocation of DL PRS or UL SRS to same sub-carriers in symbols of DL or UL slots. Therefore, we suggest: - The existing DL PRS and UL SRS for positioning can be re-used as the reference signals to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning. - FFS: allocation of DL PRS or UL SRS to same sub-carriers in symbols of a DL or UL slot respectively. - FFS: Whether to consider other enhancements of the existing DL PRS and UL SRS for better positioning performance |
| ZTE | Support. |
| MTK | Agree |
| Huawei, HiSilicon | We support to reuse the existing positioning reference signal and consider the dedicated enhancements for the carrier phase positioning. |
| CMCC | In the SID, it has already captured that focusing on reusing existing PRS/SRS. But basically, we are ok to have an explicit agreement on this. - Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4] - Reference signals, physical layer measurements, and physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1] - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary |
| Xiaomi | Support |
| Intel | Support. |
| Ericsson | Agree. Regarding FFS, we currently do not see any enhancements that could improve the performance. |
| Qualcomm | Support |
| Locaila | Support |
| Sharp | Support |
| Philips | Support |
| LGE | Support |
| IIT Kanpur, CEWiT | We support the proposal. |
| Apple | Okay |

Agreement The existing DL PRS and UL SRS for positioning can be re-used as the reference signals to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning. - FFS: Whether to consider enhancements of the existing DL PRS and UL SRS for better positioning performance

# Physical layer measurements for carrier phase positioning

Agreement (RAN1#109e) The study of the accuracy improvement based on NR carrier phase measurements in Rel-18 SI may include: - UE-based and UE-assisted carrier phase positioning, - UL carrier phase positioning and DL carrier phase positioning. - NR carrier phase positioning with the carrier phase measurements of one carrier frequency or multiple frequencies - Combination of NR carrier phase positioning with another standardized Rel. 17 positioning method, e.g., DL-TDOA, UL-TDOA, Multi-RTT, etc. - Note: The use of “carrier phase positioning” does not necessarily mean it is a standalone positioning method - FFS: whether SL carrier phase positioning is to be discussed in Rel-18 SI Agreement (RAN1#109e) - For the purposes of discussion, for NR downlink and/or uplink carrier phase positioning, the carrier phase (CP) at a RF frequency at a receiver is a phase that is a function of the signal propagation time from an Tx antenna reference point of a transmitter (e.g., a TRP or a UE) to a Rx antenna reference point of the receiver (e.g., a UE or a TRP). - The propagation time can be expressed in a fractional part of a cycle of the RF frequency and a number of integer cycles, but the CP may be independent of the number of integer cycles.

Submitted Proposals:

- (Huawei, R1-2208455[1]) Proposal 2: The double differential carrier phase and multi-frequency carrier phase should be considered for carrier phase positioning

- (Huawei, R1-2208455[1]) Proposal 3: For carrier phase jointly with DL-TDOA, phase difference between the measuring TRP and reference TRP can be reported by UE.

- (Huawei, R1-2208455[1]) Proposal 4: For carrier phase jointly with UL-TDOA, relative phase can be reported by TRP.

- (Huawei, R1-2208455[1]) Proposal 5: For UL carrier phase positioning method, the TRP can report carrier phase measurements on multiple frequencies or distance information computed from multi-frequency carrier phase measurements.

- (Huawei, R1-2208455[1]) Proposal 6: For DL LMF-based carrier phase positioning method, the UE can report carrier phase measurements on multiple frequencies or distance information computed from multi-frequency carrier phase measurements.

- (vivo, R1-2208650[2]) Proposal 2:	The carrier phase measurement is the phase of a fractional part of the propagation time.

- (vivo, R1-2208650[2]) Proposal 3:	NR carrier phase measurement can be the carrier phase difference between the Transmission Point (TP) and the reference TP.

- (Nokia, R1-2208736[3]) Proposal 6: RAN1 to study and identify necessary measurements to support CP positioning for both DL-based positioning and UL-based positioning.

- (Nokia, R1-2208736[3]) Proposal 8: RAN1 to study physical layer procedure to support virtual wavelength method to address integer ambiguity problem.

- (China Telecom, R1-2208774[4]) Proposal 4: support multiple carrier phase measurements, because it is benefit for integer ambiguity removal, and contribute to performance improvement of distance estimation.

- (China Telecom, R1-2208774[4]) Proposal 5: for the single carrier signal mechanism, support the round trip measurements of phase difference on both sides in order to eliminate the initial phase offset.

- (OPPO, R1-2208801[5]) Proposal 2: Study using carrier phase measurement of multiple carrier frequencies to resolve the issue of integer ambiguity.

- (OPPO, R1-2208801[5]) Proposal 3: The UE can measure and report carrier phase of multiple REs in DL PRS and the UE reports the carrier phase measurement and positioning measurement results of legacy positioning method.

- (OPPO, R1-2208801[5]) Proposal 4: Study measuring and reporting the relative phase of different Tx antenna ports of TRP for positioning.

- (OPPO, R1-2208801[5]) Proposal 5: Study the PRS enhancement for supporting measuring relative phase of Tx antenna ports of TRP

- (OPPO, R1-2208801[5]) Proposal 6: The TRP measures and reports carrier phase of multiple REs in SRS for positioning and the TRP also reports the carrier phase measurement and positioning measurement results of legacy positioning method.

- (CATT, R1-2208983[6]) Proposal 2: Support UE/TRP to report the carrier phase measurements corresponding to multiple carrier frequencies of different carriers, and/or corresponding multiple subcarrier frequencies of the same or different carriers for NR carrier phase positioning.

- (CATT, R1-2208983[6]) Proposal 8: The candidate measurements for carrier phase positioning may include:

  - DL/UL carrier phase measurements corresponding to the carrier frequency of a carrier, or corresponding to one or more subcarrier frequencies within the bandwidth of the carrier, between a Tx antenna of a UE/TRP and a Rx antenna of a TRP/UE

  - Single differential DL carrier phase measurement between different Tx antennas of a TRP or different TRPs, and a Rx antenna of a UE;

  - Single differential DL carrier phase measurement between different UE Rx antennas and a Tx antenna of the TRP;

  - Single differential UL carrier phase measurement between different UE Tx antennas and the same Rx antenna of the TRP.

- (Intel, R1-2209059[7]) Proposal 3: Study introduction of RSTD measurement based on carrier phase estimates of the DL PRS signal at the kth subcarrier frequency for the target and reference TRPs normalized by the carrier frequency (ωc + ωk).

  - FFS: If/how to introduce reporting of carrier phase measurements directly.

- (Intel, R1-2209059[7]) Proposal 7: For high accuracy positioning, study TX beamforming phase difference estimation for reference and target TRPs for mitigation of effect of PCO and ARP errors for DL RSTD based methods.

- (Intel, R1-2209059[7]) Proposal 9: Study RSTD measurement for the multi element RX antenna array, which requires RX beamforming phase difference estimation for the reference and target TRPs.

- (Intel, R1-2209059[7]) Proposal 10: Study introduction of RTOA measurement based on the carrier phase estimates of the UL SRS signal at the kth subcarrier frequency normalized by the carrier frequency (ωc + ωk).

  - FFS: If/how to introduce reporting of carrier phase measurements directly.

- (Intel, R1-2209059[7]) Proposal 12: Study RTOA measurement for the multi element TX antenna array, which requires TX beamforming phase difference estimation for the reference and target TRPs.

- (Intel, R1-2209059[7]) Proposal 13:	Study RTOA measurement for the multi element RX antenna array, which requires RX beamforming phase difference estimation for the reference and target TRPs.

- (NEC, R1- 2209152[8]) Proposal 1: How to quantitate the received carrier phase (also accompany with integer part) in the specification and report it from UE side should be further studied.

- (ZTE, R1-2209215[9]) Proposal 1: For multiple segments CP report within a carrier, center frequency or λ should be informed for each of each segment.

- (ZTE, R1-2209215[9]) Proposal 3: Phase or phase difference report should be supported at least for TDOA.

- (ZTE, R1-2209215[9]) Proposal 4: The same Phase Error Group(PEG) should be used for CP measurement.

- (ZTE, R1-2209215[9]) Proposal 8: The number of carrier phases to be reported could be determined based on the positioning accuracy and measurement overhead requirements.

- (Xiaomi, R1-2209293[10]) Proposal  2: Support positioning report containing the carrier phase and the corresponding carrier frequency for multiple carrier frequencies.

- (Xiaomi, R1-2209293[10]) Proposal  3: Study relative carrier phase measurement between RSs from different TRPs.

- (CMCC, R1-2209343[11]) Proposal 2: At least the following solutions to solve the integer ambiguity are considered feasible:

  - Fast search of integer ambiguity of the carrier phase obtained from more than one frequency;

  - Fast search of integer ambiguity of the carrier phase associated with the legacy positioning techniques.

- (InterDigital, R1-2209489[13]) Proposal 2: Study transmission and reception modes (e.g., 2 TX 1RX, 1Tx 2 RX) for phase-difference based positioning.

- (InterDigital, R1-2209489[13]) Proposal 3: For DL-based positioning, at least the following positioning methods should be supported: NR carrier phase positioning and joint carrier phase and DL-RSTD.

- (InterDigital, R1-2209489[13]) Proposal 7: Support the UE to select and report the reference for phase difference measurements.

- (InterDigital, R1-2209489[13]) Proposal 8: Support the UE to report the estimated number of cycles of a received carrier.

- (InterDigital, R1-2209489[13]) Proposal 9: Support the UE to perform PRS measurement in multiple frequencies in one measurement window.

- (InterDigital, R1-2209489[13]) Proposal 10: Support the UE to report the phase measurement in multiple frequencies in one measurement reporting.

- (IIT Kanpur, R1-2209546[16]) Proposal 1: Carrier phase difference measurement should be used as measurement quantity for carrier-phase based positioning.

- (Samsung, R1-2209738[17]) Proposal 6: Study using multiple carriers to determine a phase of a virtual carrier with a lower virtual frequency.

- (LGE, R1- 2209805[19]) Proposal 1: Consider both carrier phase measurement and subcarrier phase measurement based positioning methods.

- (LGE, R1- 2209805[19]) Proposal 2: Consider RSPD report for the carrier phase measurement.

## DL/UL carrier phase measurements

FL Comments

In RAN1#109e, it was agreed that the study of the accuracy improvement based on NR carrier phase measurements in Rel-18 SI may include: UE-based and UE-assisted, UL/DL carrier phase positioning, with the carrier phase measurements of one carrier frequency or multiple frequencies.

For the DL carrier phase measurements, it seems the majority of companies (e.g., [1][2][5]][6][7][8][10][13][16][19]) are supportive of reporting the difference of the carrier phases measured from different PRS signal(s), in a way similar to Rel-16 RSTD measurements. The approach allows the removing the impact of some UE-related errors (e.g., UE clock errors/initial phase) from the reported carrier phase measurements and may have less impact on the specification since it uses a similar way as Rel-16 RSTD reporting.

For UL carrier phase measurements, it is reasonable to simply follow a similar way as Rel-16 RTOA to report the UL carrier phase measurements obtained from the UL SRS signal from the target UE.

(Closed) Proposal 3.1-1

- For UE-assisted NR carrier phase positioning, at least consider the following alternatives

  - the difference between the carrier phase measured from the DL PRS signal(s) of the target TRP and the carrier phase measured from the DL PRS signal(s) of the reference TRP  from the UE to the LMF.

    - FFS: the definition of the DL carrier phase difference measurement

  - the carrier phase measured from the DL PRS signal(s) of a TRP

    - FFS: the definition of the DL carrier phase measurement

| Company | comments |
| --- | --- |
| InterDigital | Support |
| Nokia/NSB | In general okay but suggest to say “at least the difference between”. Also we may need to clarify that “measuring TRP” means target TRP not that the TRP is the one measuring |
| CATT | Support |
| Fraunhofer | Similar comment as Nokia on the measuring TRP |
| Samsung | We would also like to consider reporting the phase of a single TRP. Therefore, we suggest the following update: - For UE-assisted NR carrier phase positioning, study the following alternative: - Reporting from the UE to the LMF, the difference between the carrier phase measured from the DL PRS signal(s) of the measuring TRP and the carrier phase measured from the DL PRS signal(s) of the reference TRP can be reported from the UE to the LMF. - Reporting from the UE to the LMF, the carrier phase measured from the DL PRS signal(s) of a TRP. |
| ZTE | Support |
| MTK | 1, similar view as some other companies that “measuring TRP” “target TRP” 2, We assume this reporting should be when DL-TDOA/RSTD measurement is configured? 3, We assume the carrier phase here is the observed phase at DC when downconverting to baseband? Or UE could choose a frequency slightly away from DC, since it may have DC notch filter at RF circuit in order to mitigate the impact of DC offset to improve the dynamic range after ADC. Then several subcarriers around DC may be impacted |
| CMCC | Support |
| Xiaomi | Support in principle. And suggest to revise “measuring TRP” to “measured TRP”. |
| Intel | Before we can agree to this proposal, it needs to be clarified how is “carrier phase” defined. In particular, does this mean that carrier phase is difference is reported per subcarrier, or, else, what is the reference frequency assumption for the reported phase, e.g., center frequency of a carrier, etc.? In our view, a more efficient approach would be to report an equivalent RSTD type of metric where the RSTD metric is determined based on carrier phase measurements across different subcarriers. In any case, it needs to be clarified as to what is the reference frequency for the reported carrier phase and how a UE is expected to translate to this value from multiple observations at different subcarriers. |
| Ericsson | Agree with proposal except that “measuring TRP” should be changed to “target TRP” as mentioned by some companies. |
| Qualcomm | Agree with Nokia |
| Locaila | Support |
| Sharp | Support |
| Philips | We agree with others that 'target TRP' should be used instead of 'meaauring UE.' We also with Intel that at least a reference carrier should be defined for carrier phase measurement reporting. |
| LGE | We have similar view with Nokia. We prefer to add “at least” and think that “measuring TRP” shall be change to “target TRP” for more clear understanding. |
| IIT Kanpur, CEWiT | We support the proposal. |
| Apple | Okay with update from Nokia |

Agreement For UE-assisted NR carrier phase positioning, at least consider the following options - the difference between the carrier phase measured from the DL PRS signal(s) of the target TRP and the carrier phase measured from the DL PRS signal(s) of the reference TRP. - the carrier phase measured from the DL PRS signal(s) of a TRP

(H)(Round 1) Proposal 3.1-2

- For NR carrier phase positioning, the carrier phase measurements obtained by the TRP from the UL SRS for positioning purpose of a UE can be reported from the TRP to the LMF.

  - FFS: the definition of the UL carrier phase measurement

| Company | comments |
| --- | --- |
| Nokia/NSB | Support |
| CATT | Support |
| Fraunhofer | Okay with the intention. Can a TRP report more than a single carrier phase measurement from one instant (for example UL-AoA related measurements)? |
| Samsung | We are fine with the direction of the proposal. Suggest to say study … For NR carrier phase positioning, study reporting from the TRP to the LMF the carrier phase measurements obtained by the TRP from the UL SRS for positioning of a UE can be reported from the TRP to the LMF |
| ZTE | Support |
| Huawei, HiSilicon | There is no need to restrict the SRS to be SRS for positioning. We prefer to remove “for positioning” or change it to “for positioning purpose”. |
| CMCC | Support |
| Xiaomi | Does the carrier phase include the absolute phase or the relative phase to a reference phase? |
| Intel | As for the previous proposal (3.1-1), before we can agree to this proposal, it needs to be clarified how is “carrier phase” defined. In particular, does this mean that carrier phase is difference is reported per subcarrier, or, else, what is the reference frequency assumption for the reported phase, e.g., center frequency of a carrier, etc.? In our view, a more efficient approach would be to report an equivalent RSTD type of metric where the RSTD metric is determined based on carrier phase measurements across different subcarriers. In any case, it needs to be clarified as to what is the reference frequency for the reported carrier phase and how a UE is expected to translate to this value from multiple observations at different subcarriers. |
| Ericsson | Support |
| Qualcomm | Support |
| Locaila | We don’t agree on reusing existing SRS. We believe uplink reference symbol should also be enhanced at least on the configuration/pattern. In particular when considering the application of carrier phase to V2X, it should be completely redesigned. - For NR carrier phase positioning, the carrier phase measurements obtained by the TRP from the UL reference symbol SRS for positioning of a UE can be reported from the TRP to the LMF. |
| Sharp | Support |
| Philips | Support |
| LGE | Fine with the proposal, but prefer to add “at least” before “the carrier phase measurement”. |
| IIT Kanpur, CEWiT | We support the proposal. |
| Apple | Okay |

FL Comments:

With the consideration of comments in email discussion and the online discussion and agreement made Proposal 3.1-1, Proposal 3.1-2 is revised as follows for further discussion.

### (H)(Round 2) Proposal 3.1-2

- For UE-assisted NR carrier phase positioning, at least consider the carrier phase measured from the UL SRS for positioning purpose.

  - Note: The use of MIMO SRS for positioning purpose is transparent to UE.

| Company | comments |
| --- | --- |
| Nokia/NSB | Support |
| Samsung | Support |
| OPPO | Suggest to add one note because the proposal seems not to restrict to SRS resource for positioning. We shall make sure there is no impact on the SRS for MIMO - The measurement and reporting is transparent to UE. FL：I understamd OPPO’s concern, but we are not discussing the impact of specification in this proposal yet, and I assume it is also common understanding that the use of SRS for MIMO for positioning purpose should be ransparent to UE. If OPPO has strong concern, we may consider adding the followng note: - For UE-assisted NR carrier phase positioning, at least consider the reporting of the carrier phase measured from the UL SRS for positioning purpose. - Note: The use of MIMO SRS for positioning purpose is transparent to UE. |
| Intel | We prefer to align with the wording used for Proposal 3.1-1, i.e., “at least consider the reporting of the carrier phase measured from the UL SRS for positioning purpose.” FL: Okay. |
| vivo | Okay with intel modification |
| Locaila | We prefer to use the word reference symbol rather than SRS. We believe uplink reference symbol should also be enhanced at least on the configuration/pattern. In particular when considering the application of carrier phase to V2X, it should be completely redesigned. - For UE-assisted NR carrier phase positioning, at least consider the reporting of the carrier phase measured from the UL reference symbol SRS for positioning purpose. FL: “UL reference symbol” is currently undefined in 3GPP. For UL SRS for positioning, if there is some change of the configuration/pattern in R18, I would assume it will still be called “UL SRS for positioning”. |
| Huawei, HiSilicon | OK with Intel’s version. |
| Lenovo | Support |
| IIT Kanpur, CEWiT | We are okay with the porposal. |
| InterDigital | Support the modified proposal from Intel |
| Xiaomi | In proposal 3.1-1, both carrer phase difference and carrier phase are considered. So in proposal 3.1-2, the difference will be precluded? FL: For DL, UE receives the DL PRS from different TRPs, and reporting the measurements obtained from all TRPs. In this case, it is reasonable to reporting the carrer phase difference by selecting a reference TP. For UL, TRP reports the measurements obtained each UE separaetely. |
| LGE | Support Intel’s version |
| CATT | Support |

FL Comments:

It seems most companies are fine with the latest FL proposal in above table. We will use it for Round 3 discussion.

### (H)(Round 3) Proposal 3.1-2

- For UL UE-assisted NR carrier phase positioning, at least consider the carrier phase measured from the UL SRS for positioning purpose.

  - Note: The use of MIMO SRS for positioning purpose is transparent to UE.

| Company | comments |
| --- | --- |
| ZTE | We are wondering whether ‘UL’ should be added before ‘UE-assisted’, UE-assisted positioning is also considered in current DL positioning protocol. - For UL UE-assisted NR carrier phase positioning, at least consider the carrier phase measured from the UL SRS for positioning purpose. - Note: The use of MIMO SRS for positioning purpose is transparent to UE. |
| FL | To ZTE: Adding “UL” seems okay in my view. |
| Huawei, HiSilicon | In general for UL positioning, we do not say UE assisted, but rather network assisted. Anyway, we can live with the wording. |
| FL | To Huawei, Okay. Let us keep the wording. |
| Ericsson | we are ok with this. But this is only to scope what UL signals is to be used for NR CPP in UL and not a TR conclusion right? |
| FL | To Ericsson: Similar to the agreement made in DL side, my understanding is that the text will be captured in the Section 6 of the TR. Then, for the content in the conclusion session of the TR, we may decide in the next meeting. I assume the workding will be something like : |

## Carrier phase measurements of multiple frequencies

FL Comments

Many companies have investigated the benefits of using carrier phase measurements from multiple frequencies for NR carrier phase positioning (e.g., [1, 3, 4, 5, 6, 7, 9, 10, 11, 13, 17, 19]. Different from GNSS systems, where the carrier frequencies of the GNSS signals are all fixed, NR has a very rich spectrum, and the flexibility to support the carrier measurements from different frequencies of the same or different carriers, the same or different bands, and the same or different DL PRS frequency layers (Note: In FL’s understanding, in here the carrier phase measurements of different carriers are assumed to be obtained separately and independently. It does not require the support of carrier aggregation (CA)).

In the previous meeting, there was a discussion on the definition of the frequencies of carrier phase measurements when the multiple carrier phase measurements are obtained from different subcarriers. In FL’s understanding, the carrier frequency of a carrier phase measurement should always refer the frequency of a carrier wave (with respect to the DC) that is modulated to transmit signals. Thus, the frequency of the carrier phase measurement of a subcarrier should be the carrier frequency of the subcarrier.

(H)(Round 1) Proposal 3.2-1

- The carrier phase measurements of multiple frequencies can be considered for NR carrier phase positioning, where the multiple frequencies can be the frequencies of different carriers or the subcarriers within the same carrier.

  - Note: The frequency of a carrier phase measurement is considered to be the absolute frequency, i.e., relative to the direct current (DC).

- FFS: the physical layer procedures to enable the reporting of carrier phase measurements of multiple frequencies.

| Company | comments |
| --- | --- |
| vivo | We don’t understand the proposal since it is different from the FL comments, in the FL comment, the multiple carriers should be different frequencies. But in the proposal, it also can be a different subcarrier. In addition, we acknowledge the benefits of GNSS since the perfect syncopation and measurement at the same time. But, in Rel-16 or Rel-17, a MG or PPW only supports measuring at one frequency at the same time. We are not sure it is beneficial for NR carrier phase positioning. FL: Not sure if the confusion comes from the terms. Maybe we can use the formula defined from TS 38.211 for the explanation. The transmitted PRS signals can expressed as: sl(p,μ)t=sl(p,μ)ttstart,lμ≤t<tstart,lμ+Tsymb,lμ0otherwise sl(p,μ)t=k=0Ngrid,xsize,μNscRB-1ak,l(p,μ)ej2πk+k0μ-Ngrid,xsize,μNscRB2Δft-NCP,lμTc-tstart,lμ k0μ=Ngrid,xstart,μ+Ngrid,xsize,μ2NscRB-Ngrid,xstart,μ0+Ngrid,xsize,μ02NscRB2μ0-μ Where f0 is the carrier frequency, k is the subcarrier index, and Δf is subcarrier spacing. Let us assume Then, let us assume “the frequency of the carrier phase measurements of a carrier” refers to f0. Then, the carrier frequency of the subcarrier carrier is fk=f0+k+k0μ-Ngrid,xsize,μNscRB2Δf. A UE may provide the carrier phase corresponding to f0 from the received DL PRS signals, or measure the carrier phase corresponding of a subcarrier. The motivation for providing the carrier phase of subcarriers is that for a carrier with large BW, e.g., 100MHz, one may use them for improving carrier phase positioning performance, e.g., combining them to get the virtual carrier phase measurement to resolve the integer ambiguity. |
| InterDigital | Support |
| Nokia/NSB | We think it may be simpler to say: - The carrier phase measurements of multiple positioning frequency layers can be considered for NR carrier phase positioning. - Note: The frequency of a carrier phase measurement is considered to be the absolute frequency, i.e., relative to the direct current (DC). - FFS: the physical layer procedures to enable the reporting of carrier phase measurements of multiple positioning frequency layers. Subcarrier level reporting of phase is something totally different in our understanding and we suggest o discuss it separately. FL: If we use “positioning frequency layers”, then it may apply only for DL, since R16 only defines no PRS positioning frequency layers |
| CATT | Support |
| Fraunhofer | Agree that SC should be treated separately. Propose to refine the Nokia proposal a: - The carrier phase measurements of multiple coherent positioning frequency layers can be considered for NR carrier phase positioning. - Note: The frequency of a carrier phase measurement is considered to be the absolute frequency, i.e., relative to the direct current (DC). - FFS: the physical layer procedures and required reporting. FL: In my view, there is no need to consider "coherent”, since we are not discussing CA for carrier phase measurement here. |
| Samsung | We are fine with the direction of the proposal. But suggest the following updates - The carrier phase measurements of multiple frequencies can be considered studied for NR carrier phase positioning, where the multiple frequencies can be the frequencies of different carriers or the subcarriers within the same carrier. - Note: The frequency of a carrier phase measurement is considered to be the absolute frequency, i.e., relative to the direct current (DC). - FFS: the physical layer procedures to enable the reporting of carrier phase measurements of multiple frequencies. - FFS: carrier phase measurement reporting. FL: We have the agreement to “study” carrier phase measurements of multiple frequencies. |
| ZTE | We prefer implement carrier phase measurement for the subcarriers within the same carrier, instead of in different carriers. Because the calibration of phase discontinuity between carriers is required while implementing CP measurement in different carriers. So we suggest revise the proposal as follows: - The carrier phase measurements of multiple frequencies can be considered for NR carrier phase positioning, where the multiple frequencies can be the frequencies of different carriers or the subcarriers within the same carrier. - Note: The frequency of a carrier phase measurement is considered to be the absolute frequency, i.e., relative to the direct current (DC). FL: Similar to the comment to Fraunhofer. The proposal is not to consider the CA case. |
| MTK | 1, it seems to us that to say “positioning frequency layers” is not needed. The original wording of “multiple frequencies” is okay, since the intention is to conduct the linear combination on the observed phases of some frequencies 2, For the linear combination of phases, which is what GNSS said wide-lane, narrow-lane, may also produce some impact. The coefficients may enhance the noise even though the generalized wavelength (after combination) becomes longer. We would like to do more analysis and make conclusion at next meeting FL: Yes, Some combinations may better than others. I assume this can be supported by flexiable configuration to be worked out during WI. 3, if the multiple frequencies are the frequencies of different carriers, we have similar concern as vivo that these carriers may not be measured simultaneously. If the multiple frequencies are the subcarriers within a same carrier, then the simultaneous measurement is feasible FL: It does not mean all carriers have to be simultaneously measured in my view. |
| Huawei, HiSilicon | OK in general. The carrier phase measurements of multiple frequencies are helpful for the resolution of integer ambiguity. The note is confusing. On one hand it reads “absolute frequency”, and on the other, it reads “relative to”. We suggest to remove it. FL: If we assume everyone have the same understanding on what carrir frewurncy is, then there is no need to have the note. |
| CMCC | We think that the carrier phase measurements on frequencies of different carriers and on frequencies of different subcarriers can be separately considered, as the intention and effect may be different. Regarding the frequencies of different carriers, or of different PFLs as suggested by companies, we share similar views with vivo that as in R16/R17, UE can only measure one PFL at a time, the performance gain may be marginal and should be further studied. FL: The intention here is for resolving the integer ambiguity. It does not mean all carriers have to be simultaneously measured in my view. We are fine with the that part regarding frequencies of different subcarriers, which may be helpful for resolving the integer ambiguity. |
| Xiaomi | Support in principle and suggest the following update to align with the legacy PRS positioning frequency layer. - The carrier phase measurements of multiple frequencies can be considered for NR carrier phase positioning, where the multiple frequencies can be the frequencies of different carriers or positioning frequency layer or the subcarriers within the same carrier. - Note: The frequency of a carrier phase measurement is considered to be the absolute frequency, i.e., relative to the direct current (DC). - FFS: the physical layer procedures to enable the reporting of carrier phase measurements of multiple frequencies. |
| Intel | We share similar views as ZTE and support the version from ZTE, the feasibility of coherency between different frequency layers needs more study. Without this, we are not sure if reported phase measurements for different frequency layers can be meaningfully combined. |
| Ericsson | We don’t agree with the proposal: - Reporting the carrier phase of individual subcarriers means a potentially very large overhead. Reporting the carrier phase of individual subcarriers does not add any information compared to reporting the center carrier phase and the estimate time-of-arrival of the reference signal. The carrier phase of an individual subcarrier can be reconstructed from the phase of the center carrier and the time-delay of the path. (The relative phase of two different subcarriers is proportional to the time-delay of the signal.) FL: The issue here is that for supporting carrier phase positioning, the carrier phase measurement errors of the receiver (not accounting other errors that can be removed by )double differencial must be very small. TOA measurement noise is much larger than that. |
| Qualcomm | Agree with Nokia. Also agree with Ericsson that the value of individual subcarrier phase reporting is quite unclear |
| Locaila | Request for clarification: What’s the meaning of f0 in the FL’s answer to vivo ? What does "..different carriers..." mean? Does this mean that the UE must have multiple RF front-ends and samples at multiple different f0 frequencies at the same time? If so, single RF UE cannot perform any carrier phase operation. Is this view correct? PFL: Please check with TS 38.211 for the meaning of f0. Our proposed text is.. - The carrier phase measurements of multiple frequencies can be considered for NR carrier phase positioning, where the multiple frequencies can be the frequencies of different carriers or the subcarriers within the same carrier. |
| Sharp | We share the views from Nokia and CMCC that different carriers and the subcarriers within the same carrier should be discussed separately. |
| Philips | We agree with others that different carriers and the subcarriers within the same carrier should be discussed separately. |
| IIT Kanpur, CEWiT | We see the benefit of studying the carrier phase measurement over multi frequency considering the different carriers or the subcarriers of the same carrier frequency. We also agree that if multiple frequencies are different carriers, then measuring the carrier phase over multiple frequencies simultaneously may not be possible. We agree with intent of the proposal but going ahead we may need to separate the discussion on the carrier and the sub carriers. This can be clarified as a Note or FFS. |
| Apple | Given that we are “studying/considering” multiple frequencies/carriers of the same frequency, we are fine with the direction of the proposal. |

FL Comments

Multiple companies have suggested considering the carrier phase measurements of different carriers and the carrier phase measurements of subcarriers within the same carrier separately.

(H)(Round 2) Proposal 3.2-1a

- Reporting of the carrier phase measurements of multiple subcarriers within a carrier can be considered for NR carrier phase positioning.

| Company | comments |
| --- | --- |
| Nokia/NSB | We don’t support this proposal right now. Our understanding is that the phase of the subcarriers can’t just directly be used as a replacement of the phase of the first path and therefore it will not correspond to the distance in the way that the carrier phase does. FL: I assume we always look for the carreier phase of the first path, regardless whether it is the carrier phase corresponding to the carrier frequency of the carrier, or the carrier phase corresponding to a subcarrier within a carrier. In this moment, we haven’t decided yet the RF frequency for reporting carrier phase of a carrier yet. Let us assume RF carrier frequency is f0 as defined in TS 38.211, the carrier frequency of the subcarrier carrier k is fk=f0+k+k0μ-Ngrid,xsize,μNscRB2Δf. Using the carrer phases of different subcarriers, we could potentially define different measurements, e.g., reporting the carrer phases of different subcarriers directly, or selectively, or the difference of them, etc. |
| Samsung | Support with change. The reporting is based on the carrier phase measurements of multiple sub-carriers. - Reporting of based on the carrier phase measurements of multiple subcarriers within a carrier can be considered for NR carrier phase positioning. - This can apply to DL PRS or UL Positioning SRS FL: For the suggestion, I assume it means the measurement reported may not needs to be carrier phase of the subcarriers. For clarity, we may say: - Reporting of the measurements of based on the carrier phases of multiple subcarriers within a carrier can be considered for NR carrier phase positioning. The candidate measurements include, at least, the following: - the carrier phases of RF frequencies corresponding to multiple subcarriers - the difference of the carrier phases of the multiple subcarriers - the slope determined by the difference of the carrier phases of the multiple subcarriers |
| OPPO | The change by Samsung looks good to us |
| MTK | 1, We understand the intention of using the phases from multiple subcarriers to generalize a phase corresponding to using longer wavelength for transmission. Then the search effort for integer cycle ambiguity could be reduced 2, whether UE could combine with a given set of coefficients so that UE may only need to report a single value could be studied 3, it seems to us that the phases from multiple subcarriers corresponding to the first path could be derived by using a smaller size of IFFT to time domain for observation, or in frequency domain, the higher resolution receiver having matrix inversion is able to output the phase corresponding to the first path while other phases are suppressed. So when without CA, the linear phase combination may still work by utilizing subcarriers 4, we would like to study the impact of linear combination through coefficients, since it may also enhance the noise. Then we prefer to conclude this proposal in next meeting FL: I assume the listed in MTK’s comment covers many issues for further study. Maybe we can have a separate proposal for this. |
| Intel | We support the modification from Samsung. We have concerns on significant OH if CP msmts are reported at subcarrier level. Hence, “reporting based on the carrier phase measurements” is recommended. |
| Locaila | Support Samsung and Intel modification. |
| Huawei, HiSilicon | We want to clarify that reporting xx or reporting based on xx of multiple subcarrier should not be interpreted as the phase of CFR at this subcarrier. To make it clear, we suggest to write it as - Reporting of the carrier phase measurements of RF frequencies corresponding to multiple subcarriers within a carrier can be considered for NR carrier phase positioning. FL: the suggestions looks good to me. |
| Lenovo | We also share same views as Intel as we must have a compromise between overhead and number of reported carrier phase mesaurements of the multiple subcarriers. |
| FL | Based on the comments received, maybe we can modify the proposal as follows. - Reporting of the measurements based on the carrier phases of multiple subcarriers within a carrier can be considered for NR carrier phase positioning. The candidate measurements at least include the following: - the carrier phases of RF frequencies corresponding to multiple subcarriers - the difference of the carrier phases of the multiple subcarriers - the slope determined by the difference of the carrier phases of the multiple subcarriers |
| IIT Kanpur, CEWiT | We are fine with the proposal. We would like to know whether all these options could be considered for reporting, or we will down select one of them in future? FL: They are candidates for consideration. Thus, we can down-select them. To me, it is needed, since they basically reporting the same information is different way, but may have different advantages. |
| InterDigital | We support the proposal. We are also ok from the latest version from the FL. |
| Xiaomi | We support the latest proposal from FL |
| LGE | We are fine with the FL’s modified version. We have one question for more clear understanding. In the 2nd subbullet, we think that the difference of the carrier phase between subcarriers from different TRP is not intended. Is that correct understanding? FL: Good question. This can be further discussed in my view. In this moment, I haven’t seen the special benefit yet. |
| CATT | We support the latest proposal from FL. |

FL Comments:

It seems most companies are fine with the latest FL proposal in above table. We will use it for Round 3 discussion.

### (H)(Round 3) Proposal 3.2-1a

- Reporting of the measurements based on the carrier phases of multiple subcarriers within a carrier can be considered for NR carrier phase positioning. The candidate measurements at least include the following:

  - the carrier phases of RF frequencies corresponding to multiple subcarriers

  - the difference between the carrier phases of the multiple subcarriers

  - the slope determined of the carrier phases of the multiple subcarriers

| Company | comments |
| --- | --- |
| MTK | 1, we are not sure the 2nd and 3rd have any significant difference? It seems to us that when the carrier phase of some multiple subcarriers are measured, the difference of the carrier phases could be reported (similar to absolte RSRP and differential RSRP). But we here are talking about “measurement”, not “reporting”, then we are not sure 2nd and 3rd are different for what? |
| ZTE | The following bullets contains the same meaning I guess, the difference/slope of different subcarriers can be calculated using the measurement results of the first bullet. And for the third bullet, would it be better to revised as follows: - the slope determined by the difference of the carrier phases of the multiple subcarriers |
| FL | To MTK/ZTE: My understand is that for the 2nd sub-bullet, the intention is to allow the report the differences between the carrier phases, while the 3rd sub-bullet reports the slope derived from the differences between the carrier phases. To ZTE: The suggeste rewording is okay to me. |
| Nokia/NSB | We have some concerns on this proposal as we don’t think that everyone has the same understanding of what “measurements based on the carrier phases of multiple subcarriers” means. We would like to continue the discussion of this topic next week. |
| Samsung | We support this proposal. For third sub-bullet there is a small editorial change: - the slope determined of by the carrier phases of the multiple subcarriers |
| Huawei, HiSilicon | Do not support the last two bullets. Losing absolute frequency information from the measurement is no longer regarded as carrier phase |
| Qualcomm | Same view as Nokia. To elaborate some more: In general we are much more supportive of 3.2-1b than this proposal, and think we should defer this agreement until its value has been better established. In 3.2-1b, addition of multiple PFLs can only help in general (because it is providing additional measurements on top of the single PFL case). Whereas in this proposal, the idea appears to be to break up a single PFL into subbands or groups of “multiple subcarriers” and report measurements for each group. Are the measurements for each group going to be based only on the received subcarriers in that group, or are they based on all the received subcarriers? If using all the received subcarriers, then all we need is a single measurement corresponding to the phase of the 1st path as determined from all those subcarriers; so we don’t see the value of multiple measurements. If using only the received subcarriers within the group, then each measurement (of each of the groups) would be of inferior quality compared to the single measurement that used all the subcarriers, because of lower bandwidth, which limits the accuracy of resolving the first path. We could of course combine these multiple lower quality measurements to obtain a position estimate that would be more accurate than using any one of them individually. But it is highly unclear that this would be more accurate than just using all the subcarriers to compute a single carrier phase measurement. We understand that the motivation behind this proposal appears to be related to ideas of ‘lane combining’ (wide-laning and narrow-laning) in GNSS literature. But the right analogy for those GNSS ideas is 3.2-1b, not this proposal. Multiple PFLs in general cannot be combined coherently, and having them can only help (relative to a single PFL). But all the subcarriers in a PFL can be coherently processed, and so they should be – the value of reporting separate subbands is very unclear. FL: In my understanding, for the NR carrier phase positioning to successful, accurate measuring the carrier phase is not enough. It depends highly on how to determome the integer ambiguity correctly. Let us assume the carrier bandwidth is 100MHz. Using the all of the received signals in BW may get very accuracy carrier phase phi0 of the carrier frequency f0, say the phase error is very small, e.g., 0.1cm. But, the very small error of x0 alone is not good enough for the determinae integer ambiguity. For example, if “brute-force integer-ambiguity-resolution (IAR)” is used, the correctness of the integer ambiguity will be determined by TOA measurements. Once the integer ambiguity is not determined correctly, there will be large positioning error even the phase measurement error is very small . On the other hand, if you use 30MHz, 40MHz and 30MHz to get three carrier phases phi_1, phi_2, and phi_3,with different freqeucnies f0+df1, f0+df2, and f0+df3. The phase measurement could be larger, say 0.2cm. But, it is then possible to use the combination of the phi_1, phi_2, and phi_3 to create the new ‘virtual’ carrier phase with the wavelength much larger than phi_1, phi_2, and phi_3. With the proper selection of the freqeucnies df1, f0+df2, and f0+df3, the wavelength of the virtual carrier phase can be much larger, the successful rate for the integer ambiguity determination will be much higher. With the correct determination of the integer ambiguity, the positioning performance will be higher even if the accuracy of the phase measurement may be a little larger. The above comments are addressed mainly at the 1st sub-bullet. For the 2nd and 3rd sub-bullet, it has been pointed out earlier (e.g., by Huawei) that the differences or slope of the phase as a function of subcarrier-index is equivalent to a TOA, so it does not need a separate report, for example existing RSTD report serves the same function. |
| FL | Thanks for the comments. It seems we would need more time to work on this proposal in next week. |
| Ericsson | We have concerns on this proposal. We are still not conviced on the benefits of reporting carrier phases of individual subcarriers. As we stated previously, we think reporting center carrier phase and the estimate of the time-of-arrival of the reference signal is enough. We can discuss this proposal next week. |

(H)(Round 2) Proposal 3.2-1b

- Reporting of the carrier phase measurements of multiple DL positioning frequency layers can be considered for NR carrier phase positioning.

  - Note: It does not mean the DL PRS signals from multiple DL positioning frequency layers needs to be processed simultaneously nor coherently.

| Company | comments |
| --- | --- |
| Nokia/NSB | Support |
| Samsung | Support with change. The reporting is based on the carrier phase measurements of multiple positioning frequency layers. - Reporting of based on the carrier phase measurements of multiple DL positioning frequency layers can be considered for NR carrier phase positioning. FL: I assume there is no need to have the change, since the intention of the proposal is to report the carrier phase measurements from each of the DL positioning frequency layers |
| OPPO | Support |
| MTK | If different frequency layer is measured at different time instance, the UE may have been moving then the phase could be change. We don't understand the benefit |
| Intel | Thanks to FL for clarifications and updates. We are ok with multiple DL positioning freq layers, but prefer the modification from Samsung. |
| vivo | We are confused with the Samsung modification, For FL proposal, we wonder whether” reporting of” is needed. For Samsung modification, we are not sure what is reporting. In addition, the other impact from multiple frequency layer measurement also needs to be considered together if multiple DL positioning frequency layers measurement are considered So we propose the carrier phase measurements of multiple DL positioning frequency layers can be considered studied for NR carrier phase positioning, which may take into account at least the following - The performance gain of carrier phase measurements of multiple DL positioning frequency layers compare to carrier phase measurements of a DL positioning frequency layer - The impact of the timing gap between multiple DL positioning frequency layers measurements - The impact of independent initial phase error, independent frequency error, and/or independent ARP location errors of each DL positioning frequency layer on the carrier phase performance for multiple DL positioning frequency layers FL: In my view, there is no need to coniser the case that ARP error are independent for DL positioning frequency layer unless some companies want to simulate the case when each DL positioning frequency layer uses different TRP. |
| Huawei, HiSilicon | Why do we need this proposal? The existing UE positioning measurement is already per TRP per positioning frequency layer. This proposal with the Note does not provide any insight into what else carrier phase positioning can do. FL: I would agree with Huawei that UE positioning measurement is already per TRP per positioning frequency layer. I assume the concerns from some companies wants to know the benefits of UE reporting carrier phase measurements of multiple DL positioning frequency layers and under which conditions (e.g., does it need to report all measurements simultaneously). Although Rel-16 supports upto 4 DL positioning frequency layers, I think there was no evaluation of the benefits for UE to report positioning measurements from multiple DL positioning frequency layers. |
| Lenovo | We share vivo’s view as some factors need to be taken into account for the study of necessity and feasibility of reporting carrier phase measurements for different positioning frequency layers. |
| FL | ### (H)(Round 2) Proposal 3.2-1b - Further.study the benefits of using the carrier phase measurements of multiple DL positioning frequency layers for NR carrier phase positioning, which includes t - he impact of timing gap between the carrier phase measurements of multiple DL PFLs. - Note 1: The initial phase error and the frequency error for each PFLs can be modelled independently - Note 2: For each TRP, the PRS signals of all PFLs are transmitted from the same ARP. |
| IIT Kanpur, CEWiT | We support the updated proposal by FL. We also feel that it should be studied further. |
| Xiaomi | We support the latest proposal from FL |
| LGE | We are ok with the modified version from FL. |
| CATT | We support the latest proposal from FL |

FL Comments:

It seems most companies are fine with the latest FL proposal in above table. We will use it for Round 3 discussion.

### (H)(Round 3) Proposal 3.2-1b

Further study the benefits of using the carrier phase measurements of multiple DL positioning frequency layers for NR carrier phase positioning, which may include the impact of the time gap (if any) between the carrier phase measurements of multiple DL PFLs.

- Note 1: The initial phase error and the frequency error for each PFLs can be modelled independently

- Note 2: For the evaluation, the PRS signals of all PFLs of a TRP can be assumed to be transmitted from the same ARP or from different ARPs of the TRP.

- Note 3: The location error for ARPs can be modelled independently.

| Company | comments |
| --- | --- |
| vivo | Thank for the FL proposal, we are okay to remove ARP error, but the different timing errors (e,g, TEG) between multiple DL PFLs may need to be considered since the TEG ID may be different for different PFL Further study the benefits of using the carrier phase measurements of multiple DL positioning frequency layers for NR carrier phase positioning, which includes the impact of the time gap and timing error between the carrier phase measurements of multiple DL PFLs. - Note 1: The initial phase error and the frequency error for each PFLs can be modelled independently - Note 2: For each TRP, the PRS signals of all PFLs are transmitted from the same ARP. |
| MTK | 1, we are not sure whether timing error between PFLs should also be considered. It seems to us that it is related to TX chain structure |
| ZTE | We are wondering that the time gap mentioned here is used to identify the movement of the target UE? Or to combine with the Tx time of DL PRS to get the real propagation time? |
| FL | To vivo/MTK: For vivo’s comment, I share the similar view with MTK that it is unclear to me the motivation to add the “timing error” between PFLs. To ZTE: The time gap is added here to address the concern the UE may not measure the DL PFLs simultaneously, and thus there can be a time gap for the measurements obtained from different DL PFLs. |
| Nokia/NSB | We are overall supportive but have some small comments: 1. our first preference is to remove the entire end of the main bullet (e.g., stop the proposal after carrier phase positioning.) 2. if that is not acceptable then we should prefer to add (if any) to the time gap part. 3. We agree with above comments that timing error between PFLs does not need to be considered at this time. |
| FL | To Nokia: Okay. It seems reasonable to add “if any” to the time gap part, since some companies may want to see the impact of time gap in case there is the gap. |
| Vivo | We believe the performance will be improved if we simply increase the number of PFLs. But we think some errors between PFLs can be considered for evaluation for the boundaries of multi-carriers CPP. For GNSS, we understand the I timing error may not exist, but for NR devices, with the timing gap, and the RX/Tx chain structure, we are not sure all the timing is aligned for different PFLs measurements. In this case, timing offset due to timing gap or RX/Tx chain structure may need to be considered. In addition, we have some concerns about adding “if any”, based on current specifications, only one PFL can be measured in an MG or PPW, so we wonder which case can be seen as ‘no time gap’ |
| Qualcomm | We think Note2 might be unnecessarily restrictive; e.g. NR-TRP-LocationInfo-r16 is currently separate per PFL. |
| FL | To vivo: If I understand vivo’s concern correctly, it is about PFLs are transmitted with different Tx chains and received with different Rx chains, which could happen if PFLs are different bands or FR1/FR2. How about we add the following note to address vivo’s concern. - Note 3: The timing errors of the PFLs may not be the same for PFLs in different bands or frequency ranges. For the second comment, it depends on the assumption and UE capability. If a UE can only process one PFL in one PPW, then yest, there will be gap. On the other hand, for a UE support dual connectivity, it may process two PFLs simultaneously. I assume adding “if any” covers both cases. To Qualcomm: Yes. The spec does not limit PFLs in the same TRP-Location. But, for the effort of the evaluation, I am wondering if any company wants to simulate that case, since it makes the evaluation much more difficult. Maybe we can say - Note 2: For the evaluation purpose, the PRS signals of all PFLs of TRP are transmitted from the same ARP. |
| Ericsson | There are some comments on this proposal. So this may need some further discussion. We agree that timing errors between different PFLs may need to be considered as part of the study (e.g., if two PFLs are in different bands). We are not supportive of the revised Note 2. This note may ignore in evaluations ARP differences when transmitting PRS signals of different PFLs. So we prefer to model ARP errors as part of the study. |
| FL | To Ericsson: Okay. We can make the following change for supporting the evaluation of different ARPs. - Note 2: For the evaluation, the PRS signals of all PFLs of a TRP can be transmitted from the same ARP or from different ARPs of the TRP. - Note 3: The location error for ARPs can be modelled independently. |

## Carrier phase differences and DL-AOD

Submitted Proposals:

- (Intel, R1-2209059[7]) Proposal 8:	Study carrier phase-based azimuth and zenith angles of departure measurement for the multi element transmit antenna array for the DL-AOD positioning method.

- (Samsung, R1-2209738[17]) Proposal 10: Study and evaluate the performance of carrier-phase difference measurement for estimating the angle of arrival at the UE.

- (Samsung, R1-2209738[17]) Proposal 11: Study and evaluate the performance of carrier-phase difference measurement for estimating the angle of departure at the UE or the gNB.

- (Sharp, R1-2209785[18]) Proposal 1: Do not consider the phase-difference measurements for DL-AoD.

- (NTT DOCOMO, R1-2209909[20]) Proposal 3: Phase difference-based DL-AoD is out of the scope in Rel-18 NR.

- (Qualcomm, R1-2209992[21]) Proposal 2: Study supporting Phase-Difference AoD.

FL Comments

A TRP may support the transmission of the same DL signals from multiple Tx antennas (or antenna panels), and/or the reception of the UL signals from a UE with multiple Rx antennas (or antenna panels). In this case, the carrier-phase difference may be used for positioning purpose. For example, the phase difference of UL SRS from a UE obtained from different Rx antennas of a TRP is commonly used for UL-AOA measurements, although the carrier-phase difference is not reported to the LMF.

Currently, DL-AOD is estimated based on the RSRP measurements from different DL beams from a TRP. If NR carrier phase measurements are supported in Rel-18, the DL-AoD may be estimated and reported by a UE if a UE could have the capability to provide the carrier phase (or phaser-difference) of the DL PRS from different TRP Tx antennas.

The support of the DL-AOD with carrier-phase difference was discussed in previous meetings without a conclusion. One main concern is that DL-AoD is not explicitly included in the SI scope. In FL’s view, the use of the phase difference for supporting DL-AOD could be implicitly supported as long as the reporting of the carrier phase (or phaser-difference) from different TRP Tx/UE Rx antennas from UE to LMF are supported. In this case, it is up to LMF’s implementation on how to use them for positioning. The LMF may use the phase difference of different Tx antennas to first estimate the DL-AOD, and then use the DL-AOD for positioning, or the LMF may use the phase difference of different Tx antennas directly for positioning w/o first estimating the DL-AOD. In this case, there is no need to consider introducing new DL-AOD measurements.

(Round 1) Question 3.3-1

Interested companies are invited to provide their views on the following options:

- Option 1: No further discussion on phase-difference DL-AoD, because DL-AoD is not in the scope of the SI.

- Option 2: Study and evaluate the phase-difference DL-AoD in the SI.

- Option 3: No further discussion of phase-difference DL-AoD. Instead, supporting a UE to report the phase-difference measurements of different TRP Tx antennas/antenna panels/antenna ports, together with the information related to the TRP antennas. Then, how to use the phase-difference measurements for positioning, with or without explicitly estimating DL-AoD, is up to the LMF’s implementation.

| Company | comments |
| --- | --- |
| Nokia/NSB | No strong view but we are okay with Option 2. |
| CATT | Option 3 to avoid the issue with the SI scope. |
| Samsung | Support Option 2. |
| ZTE | We prefer Option 1. |
| Huawei, HiSilicon | We support Option 3 |
| Xiaomi | We prefer Option 3 |
| Ericsson | We prefer Option 1. Note the proposed phase-difference DL-AoD may potentially have large impacts and requirements in terms of phase coherency and assistance data etc. Since it is not explicitly mentioned in the SI, we prefer to avoid this discussion. |
| Qualcomm | Support Option 2. While DL-AoD is not explicitly listed in the SI scope, note that angle estimation is needed to compensate for the variation of PCO as a function of angle. A coarse angle estimation based on coarse position (from non-carrier-phase methods) may serve as a start, but this only gives GCS-based angle, whereas PCO compensation requires LCS-based angle. Thus, given the importance of DL-AoD to carrier-phase, and given the accuracy gains of phase-difference DL-AoD, we support Option 2. We agree that Option 3 will enable network to do phase-difference DL AoD, but it does not cover the case of UE-based DL-AoD. Regarding Ericsson’s comment about impacts on phase coherency and assistance data, we believe the impact will not be large on top of what is already required to support carrier phase. FL: It is unclear to me why UE-assisted DL-AoD can be supported, but UE-based DL-AoD cannot. I assume the same information that are provided by the gNB for UE-assisted DL-AoD is need for UE-based DL-AoD when the carrier phase measurement are used. |
| Locaila | Option 2. We have the same view as Qualcomm. |
| LGE | Prefer Option 1 |
| IIT Kanpur, CEWiT | We prefer Option 2 |
| Apple | Prefer Option 1 |
| FL | Based on the feedbacks, we have: Option 1: ZTE, Ericsson, LGE, Apple Option 2: Samsung, Qualcomm, Locaila, IIT Kanpur,CEWi Option 3: CATT, Huawei, HiSilicon, Xiaomi. The FL would suggest further discussion to see if we can converge to any of the options. |
| Lenovo | Support option 3 |
| FL | Based on the feedbacks, we have: Option 1: ZTE, Ericsson, LGE, Apple Option 2: Samsung, Qualcomm, Locaila, IIT Kanpur,CEWi Option 3: CATT, Huawei, HiSilicon, Xiaomi, Lenovo I am wondering if the companies supporting Option 2 is also willing to consider Option 3. In this way, we may be able to support the positioning performance as DL-AoD with carrier phase without explicity supporing DL-AoD. |

FL Comments

In FL’s understanding, the study of the benefits of using the carrier phases measured from multiple TRP Tx antenna panels from TRPs should be in the scope of the SID. Using the carrier phase measured from multiple TRP Tx antenna panels at least provide extra information that could be used to increase the accuracy and reliability of the NR carrier phase positioning. Therefore, given that we only have one meeting left for RAN1 to close the SI, the FL suggests raising the priority of this issue and add the following proposal for Round 4 discussion..

### (H)(Round 4) Proposal 3.3-1

- Study the benefits of using the carrier phases measured from the DL PRS transmitted from multiple TRP Tx antennas or antenna panels for NR carrier phase positioning and the potential impact on the specification.

| Company | comments |
| --- | --- |

## Phase-smoothed timing measurements

Submitted Proposals:

- (CATT, R1-2208983[6]) Proposal 4: Using carrier phase measurements to smooth existing timing measurements can be considered in Rel-18.

  - FFS: The impact on the specification.

- (Fraunhofer, R1-2209534[14]) Proposal 4: 	Support phase based smoothing for DL-TDoA and UL-TdoA methods in Rel-18.

  - Consider the signaling of the coherency transmission status to the UE (UE based positioning) or to the LMF to notify on the applicability of phase based smoothing.

- (NTT DOCOMO, R1-2209909[20]) Proposal 1: The carrier phase measurement can be used to adjust the measurement results of timing-based measurements such as RSTD and RTOA.

FL Comments

In GNSS positioning, the use of the carrier phase measurements for smoothing pseudorange measurements is considered to be an effective way to improve the positioning performance, due to the small measurement noise of carrier phase measurements. For NR positioning, using carrier phase measurements to smother other timing measurements may improve the NR positioning accuracy. The advantage of the approach is that it can be implemented without the need to resolve the integer ambiguity in carrier phase measurements. But, the method requires the receiver to be able to track the phase of the carrier at least for a short period of time. The impact on the specification may be minimum in FL’s view, e.g., the UE may need to inform LMF that it has the capability to support the feature, and the LMF may trigger the UE to use the feature only when it is necessary, considering that it may require more processing power in UE to support the feature.

(Round 1) Proposal 3.4-1

- Using carrier phase information for smoothing exiting timing measurements for DL-TDOA, UL-TDOA and Multi-RTT can be studied in Rel-18 for UE-assisted and UE-based positioning.

  - FFS: the impact on the specification.

| Company | comments |
| --- | --- |
| Nokia/NSB | Okay but we question the specification impact. FL: There can be some impact on the specification. UE may need to inform LMF that it has the capability to support it, and the LMF may enable it only when it is necessary. Here, we assume enabling it will have some negative on UE power consumption. |
| CATT | Support. Our understanding for specification impact is very small. |
| Fraunhofer | Support |
| Samsung | We are fine with direction of the proposal, but suggest to change “smoothing” to “refining” FL: In GNSS, the term of “carrier-smoothed” is used. Suggest keeping the same term for better understanding. |
| Huawei, HiSilicon | We do not support the proposal. We should first study the carrier phase measurements in Rel-18, and the phase-smoothed time measurements can be studied in further release. |
| Xiaomi | We are open to study it. |
| Intel | OK, and prefer the wording suggestion from Samsung. |
| Ericsson | Ok |
| IIT Kanpur, CEWiT | We support the proposal. |
| Apple | Okay |
| Lenovo | Support |

FL Comments

It seems most of feedbacks are fine with the proposal. Thus, the FL suggest raising its priority for further discussion.

(H)(Round 2) Proposal 3.4-1

- Using carrier phase information for smoothing existing timing measurements for DL-TDOA, UL-TDOA and Multi-RTT can be studied in Rel-18 for UE-assisted and UE-based positioning.

  - FFS: the impact on the specification.

| Company | comments |
| --- | --- |
| Samsung | Support |
| OPPO | We do not support the proposal. The scope of rel-18 is to study the carrier phase measurement, not to improve the exsiting measurement. At least that shall be low priority. FL: Yes. The SID says “for accuracy improvement based on NR carrier phase measurements”. The method of smoothing existing timing measurements is indeed based on the carrier phase measurement. I would not arguge with the priority. The proposal is only to study it. I assume the promponent should provide the evaluation results if they want to study it. |
| MTK | 1, there is typo “smoothing exiting timing measurements “ the existing 2, fine to study FL: corrected the typo. |
| Intel | OK to study. |
| Huawei, HiSilicon | Is it about adding the refined ToA with sub-wavelength (sub-sinuoid period level) derived from carrier phase change to the existing filtered ToA when UE is in motion? FL: The case mentioned by Huawei could be an important one when the UE is moving. |
| Lenovo | Support |
| IIT Kanpur, CEWiT | We are open to study it further. |
| Nokia/NSB | We consider this as low priority for the study. We first need to confirm that CPP is feasible for NR and if so recommend the essential steps to specify it. |
| InterDigital | Support |
| LGE | We are opent to study further but prefer to handle this issue as low priority. |
| CATT | Support |
| FL | Based on the feedbacks of 11 companies, one company (OPPO) does not support it, 8 companies are supportive, and 2 companies consider it is of low priority. I actually share the similar view as Nokia that this is something that can be easily down in WI phase due to the very small impact on the specification. Thus, my suggestion is to see if there is still any objection for the study during the email discussion. If no, we can have an agreement to study it. If we could not reach the consensus, I assume the interested companies may still bring the evaluation results to show the effectiveness of the method. |

FL Comments:

It seems most companies are fine with the Round 2 FL proposal. We will use it with minor change to include the impact on the specification for Round 3 discussion.

### (H)(Round 3) Proposal 3.4-1

- Using carrier phase information for smoothing existing timing measurements for DL-TDOA, UL-TDOA and Multi-RTT can be studied for UE-assisted and UE-based positioning, including the impact on the specification.

| Company | comments |
| --- | --- |
| ZTE | Low priority, this seems to enhance the existing methods, seems out scope. We prefer focusing on the carrier phase method itself in this release. |
| Nokia/NSB | Agree with ZTE. We don’t feel it is worth focusing on this issue and think “carrier phase information” is very vague. We can discuss more next week but we still feel that the focus needs to be on the feasibility and benefit of CPP in general. |
| Huawei, HiSilicon | Low priority. |
| Ericsson | there is possible benefit when carrier phase measurement is combined with other methods. If UE reports carrier phase measuremetns to LMF, the LMF can anyway combine these measurements with existing posisitinging measurements like DL-TDOA, etc. So, we support studying this use case. FL: Yes. This may be another way. This method can be used by the LMF if it can resolve the ineger ambiguity. But, this method may not have much impact on the specification. |

## (Closed)Other measurements

Submitted Proposals:

- (Xiaomi, R1-2209293[10]) Proposal 5: Study to report the Doppler parameters combined with the carrier phase value, or to report the adjusted carrier phase value based on Doppler parameters.

- (Lenovo, R1-2209395[12]) Proposal 7: Time-varying residual CFO has a negative impact on the transmission and reception phase of PRS(DL)/SRS(UL), which affects the accuracy of the carrier phase measurements, and additional residual CFO tracking, and compensation techniques could be studied for the carrier phase positioning case.

- (Fraunhofer, R1-2209534[14]) Proposal 6: 	In case of carrier aggregation using several bandwidth limited signals multipath effects causing phase ripple per BWP shall be taken into account for the evaluation of the performance of carrier phase measurements.

- (Samsung, R1-2209738[17]) Proposal 12: Study and evaluate the use of the UEs mobility to create a virtual antenna array.

- (Ericsson, R1-2210177[23]) Proposal 6	Study different ways for phase offset estimation and phase alignment, including joint position and phase offset estimation using multiple UEs.

FL Comments

In [10], it was proposed to study the reporting of the Doppler parameters. In [12], it is proposed to study the residual CFO tracking.  In [14], it was proposed the study the impact of multipath if carrier aggregation of using several BWPs, which is out-scape of the SI in FL’s view. In [12], it was proposed to study the impact of the UE mobility create a virtual antenna array. In [23], it was proposed to study different ways for phase offset estimation and phase alignment. In FL’s view, all these proposals could have some potential for improving the carrier phase positioning performance. However, due to the time limitation of the study item, we may have the difficulty to study of them.

### (Closed) Question 3.5-1

- Interested companies are encouraged to provide their views on following questions:

  - Q1: what is your view to “Study to report the Doppler parameters combined with the carrier phase value, or to report the adjusted carrier phase value based on Doppler parameter”?.

  - Q2: what is your view to study “additional residual CFO tracking, and compensation techniques for the carrier phase positioning case”?

  - Q4: what is your view to “Study and evaluate the use of the UEs mobility to create a virtual antenna array”?

  - Q5: what is your view to “Study different ways for phase offset estimation and phase alignment, including joint position and phase offset estimation using multiple UEs”?

| Company | comments |
| --- | --- |
| Samsung | Fine to study these techniques. Proponents to show benefit to justify. |
| ZTE | We prefer not to study about Doppler and CFO (Q1, Q2), because in our simulation, the positioning error caused by CFO is negligible. |
| Huawei, HiSilicon | Q1: We think reporting the Doppler parameters directly is not feasible since it may change rapidly because of the movement between UE and gNB. Q2: low priority Q4: We think in this release we do not need to study this topic. Q5: It appears to be an implementation issue. |
| Xiaomi | For Q1, if the Doppler spread is very large, the carrier frequency will be different at UE side and TRP side. Thus the wavelength will be not accurate. In this case, the carrier phase measured at UE side may be incorrect. If the carrier phase can be adjusted based on Doppler spread or reported together with Doppler spread, the carrier phase will be much more accurate. |
| Locaila | This is an important issue in particular for V2X application. However, considering the progress of current SI, we think RAN1 may not have proper time to discuss this issue in this release term. We would like to delay for next release for intensive discussion on the aspect of doppler. |
| FL | It seems we may need to have more companies’ input for these issues before we narrow down which of them to further study. |
| Lenovo | We prefer not to study Q4 in this release, otherwise it is good to further investigate Q1,Q2 and Q5. |
| CATT | For Q1, Q2: we the simiar view as ZTE that Doppler and CFO may not have much impact for indoor scenarios under discussion. For Q4: It may be studied under high-mobility case in the future release. For Q5: We need to focus more on the basic issues. More advanced features may be studied in the future release. |

# NR carrier phase positioning techniques and physical layer procedures

In this section, we focus on the NR carrier phase positioning techniques that may have potential impact on the physical layer procedures.

## Differential carrier phase positioning with PRU

Agreement (RAN1#109e) - In addition to the evaluation assumptions of NR Rel-16/17, the following error sources may also be considered during the evaluation: - Phase noise (FR2) - CFO/Doppler - Oscillator-drift - Transmitter/receiver antenna reference point location errors - Transmitter/receiver initial phase error - Phase center offset - Note: Other error sources are not precluded - Note: UE mobility can be considered in the evaluations - Note: one or more error sources can be evaluated jointly Note: companies should provide the error sources model with their evaluations Agreement (RAN1#109e) The use of PRUs to facilitate NR carrier phase positioning can be evaluated in the SI by RAN1.

Submitted Proposals:

- (Huawei, R1-2208455[1]) Proposal 2: The double differential carrier phase and multi-frequency carrier phase should be considered for carrier phase positioning.

- (Nokia, R1-2208736[3]) Proposal 11: RAN1 to study the CP positioning performance impact by introducing PRU for UE-assisted positioning, and identify the necessary physical layer procedure including aligning measurements by PRU and target UE.

- (Nokia, R1-2208736[3]) Proposal 12: RAN1 to study/identify the CP positioning performance impact by introducing PRU for UE-based positioning, and identify the necessary physical layer procedure.

- (CATT, R1-2208983[6]) Proposal 1: Double differential technique with PRU should be supported for NR carrier phase positioning.

- (Intel, R1-2209059[7]) Proposal 4: Study initial phase calibration of the TX generators for the reference and target TRPs by using the RSTD measurements obtained with the Positioning Reference Unit (PRU) with known coordinates.

- (Intel, R1-2209059[7]) Proposal 11: Study initial phase calibration of the RX generators for the reference and target TRPs by using the measurements obtained with the Positioning Reference Unit (PRU) with known coordinates.

- (Xiaomi, R1-2209293[10]) Proposal  4: Study PRU assisted carrier phase measurement for phase error mitigation.

- (CMCC, R1-2209343[11]) Proposal 1: For carrier phase positioning, the double differential carrier phase measurement should be considered to eliminate the inter TRP carrier phase synchronization errors.

  - The double differential carrier phase measurement can be obtained by the use of PRU.

- (Lenovo, R1-2209395[12]) Proposal 6: Support the use of Positioning Reference Units (PRUs) to help mitigate the following carrier phase errors:

  - To assist in providing an upper and lower bound on the integer ambiguity (IA) based on the selection of PRUs in the vicinity of the target-UE, which reduces IA search space.

  - Initial phase offset errors

- (InterDigital, R1-2209489[13]) Proposal 12: Support PRU for NR carrier phase-based positioning.

- (Locaila, R1-2209543[15]) Proposal 2) RAN1 should study the traffic burden related to oscillator accuracy when employing the proposed double difference method.

- (Locaila, R1-2209543[15]) Proposal 3) RAN1 should study the alternative method for carrier phase synchronization efficient than double difference method.

- (IIT Kanpur, R1-2209546[16]) Proposal 2: PRU should be used to remove the measurement error across TRPs for at least synchronization and clock errors in TRP for carrier phase-based measurements.

- (Samsung, R1-2209738[17]) Proposal 3: RAN1 to study how to provide PRU or PRU-like functionality under existing system.

- (LGE, R1- 2209805[19]) Proposal 3: Study whether and how to utilize PRU for performing double differencing at least for UE-assisted positioning method.

FL Comments

It is well known that in order to support cm-level carrier phase positioning, the time/frequency synchronization errors contained in the carrier phase measurement have to be eliminated. In GNSS carrier phase positioning, the double differential technique with the reference receiver(s) located at the known location(s) are commonly used to support GNSS carrier phase positioning. Similarly, for supporting NR carrier phase positioning, it is expected that there is a need to implement the double differential technique, which requires to have one or more PRUs located at known locations to either receive DL PRS signals or transmit UL SRS signals. In RAN1#109e, RAN1 agreed to evaluate the use of PRUs to facilitate NR carrier phase positioning in the SI. Based on the evaluation and analysis presented in this meeting, the majority of companies (e.g., [1][3][6][7][10][11][12][13][16][17][19]) consider it is necessary to support the use of PRU for supporting double differential carrier phase positioning. One company has raised the concern about the traffic burden when employing the double difference method by introducing a new architecture for inter-TRP time/frequency synchronization [15]. In FL’s view, although supporting perfect time/frequency synchronization between TRPs is highly desirable, the study of the new architecture for time/frequency synchronization is clearly not in the scope of this SI [24].

(H) Proposal 4.1-1

Capture the following TP into TR 38.859 (Section 6.3.1):

- The use of the positioning reference unit (PRU) to facilitate NR carrier phase positioning has been evaluated during the study item. Based on the evaluation results, the double differential technique with PRU is beneficial and should be considered for NR carrier phase positioning.

| Company | comments |
| --- | --- |
| Nokia/NSB | Okay in principle but should say that PRU is beneficial for NR carrier phase as many companies showed in their evaluation results. Many of the agreed error sources are clearly mitigated or removed from CP via PRU. |
| CATT | Support. Also fine with Nokia’s suggestion to add “PRU is beneficial for NR carrier phase positioning”. |
| Samsung | We should include methods to mitigate synchronization errors, so prefer to reword as: - The solutions of synchronization errors for NR carrier phase measurements have been investigated during the study item. Some of them are listed in the following: - The use of the positioning reference unit (PRU) to facilitate NR carrier phase positioning using the double differential technique, FFS the determination of PRU. - The use of round-trip-carrier phase measurement In addition, we have question on using the term “PRU”. The PRU was discussed in rel17 but not specified anything at least in RAN1, not sure if the PRU concept is clear. We somehow know what it is but not good enough for potential technical discussion. For example, if it’s just a device? Or a device can provide certain ‘information’ under certain condition? It should be clarified before we directly use the term PRU. Because for us, the “PRU” is more like a status of device rather than a forever identity. FL: 1) The definition of PRU is currently captured in TS 38.305. For the use of PRU for carrier phase positioning, I assume there will be analysis on the impact to the specification with in the SI or WI phase. 2) I would suggest not mixing RTT approaches, which will be discussed in Section 4.5. |
| ZTE | Support. Double differential technique with PRU can eliminate the side effect of some error sources on positioning performance. |
| MTK | Similar view as the above companies that using “is beneficial” seems better |
| CMCC | Support |
| Xiaomi | Support |
| Intel | Support in principle; and would like to second the suggestion from Nokia to change to “the double differential technique with PRU is beneficial for NR carrier phase positioning”. |
| Ericsson | We do not agree with the suggestion to conclude that “PRU is beneficial for NR carrier phase positioning”. Based on the results presented in the Tdocs referenced above, we don’t see the use of PRUs can effectively overcome some of the error sources or a combination of error sources. The following are some observations we found in the referenced Tdocs: - In [1], the following observations can be found: - Observation 1: The ARP error deteriorates the carrier phase positioning accuracy even the double differential method (with PRU) is used. - Observation 2: Double differential carrier phase positioning technique can eliminate some influence of PCO. Note that Results presented in Table 6 of [1] which take into account a combination of multiple error factors shows that target requirements are not met for most of the cases simulated even with PRUs. - In [17], the following observation can be found: - Observation 8: With larger phase error and ARP error, the performance is degraded severely. - In [3], the results presented are preliminary as mentioned in [3]. The impact of error sources and in particular the impact due to a combination of error sources does not seem to be taken into account in the results of [3]. - In [19], the results reported seem to be under ideal assumptions where error sources nor combination of error sources is not taken into account (see Appendix A which contains Sim assumptions for results presented in Section 3). - In [6], the following observation/proposal can be found: - Observation 13: The carrier phase positioning accuracy will degrade when ARP error increases. - Proposal 6: The impact of TRP ARP location error should be mitigated in order to improve the carrier phase positioning accuracy. - In [15], the following observation is made: - (Observation 2) Considering the accuracy of commercially available oscillators and the size of phase offset information necessary to broadcast, the proposed double difference method may cause heavy traffic burden to 5G NR system. - In [10, 11, 12, 13, 16], there are no evaluation results. These Tdocs mostly include study proposals. Based on these observations, it is premature to agree this proposal. Further study is needed to access performance of double differential technique with PRU in the presence of error sources, and particularly in the presence of a combination of error sources. FL: As commented by other companies, the use of PRU is a necessity for NR carrier phase positioning in my understanding. It does not mean the use of can remove all of the errors, but it is proven technique to eliminate, at least, the errors related the signals transmission in the transmitter and the errors related the signals transmission in the transmitter as well-known in carrier phase positioning field. |
| Qualcomm | Support, and agree with Nokia that PRU is beneficial for carrier phase. We don’t understand the Ericsson counter-argument, which seems to be summarizable as (a) PRU does not eliminate all the error sources, and (b) PRU could cause overhead. To (a), note that PRU does completely eliminate a very large source of error, namely the lack of phase-synchronization between TRPs. To (b), note that even in the worst case with one PRU report for every target UE report, it is still just doubling the positioning reporting, which could well be worth the positioning accuracy gains in certain cases, also in reality the overhead will be often far less than this worst case. |
| Locaila | We disagree on this proposal. We demonstrate with mathematical analysis and experiments in [15] that the double difference method using PRU can cause a serious overload on the 5G NR network. Therefore, the network overload issues should be considered together in this study. Due to oscillator instability, LMF may have to broadcast calibration information as frequently as the PRS cycle because the initial phase offset of gNB/TRP may vary each time the PRS is transmitted. In our view, the use of double difference solutions in 5G NR is a very expensive, inefficient, and practically impossible way to operate. We need to study more on the pros and cons of the proposed method with comparing the alternative solutions. FL: I would disagree with the statement that “the use of ouble difference solutions in 5G NR is a very expensive, inefficient, and practically impossible way to operate”. To the most, for DL carrier phase positioning, the traffic load for operating a PRU of the same as a UE when the PRU to transmit all of its measurements to the LMF. For UL carrier phase positioning, the traffic load for operating a PRU of the same as a UE for transmitting the UL SRS signals. |
| LGE | Support and fine with Nokia’s suggestion |
| IIT Kanpur, CEWiT | We support the proposal. |
| LGE | Support and fine with Nokia’s suggestion |

FL Comments

The proposal is revised based on Nokia’s suggestion, which seems to be supported by many companies.

(H) (Round 2) Proposal 4.1-1

Capture the following TP into TR 38.859 (Section 6.3.1):

- The use of the positioning reference unit (PRU) to facilitate NR carrier phase positioning has been evaluated during the study item. Based on the evaluation results, the double differential technique with PRU is beneficial for NR carrier phase positioning, , which include at least the following:

  - Eliminating the impact of the TRP timing synchronization error

  - Eliminating the impact of the initial phases of the transmitter and the receiver.

- The evaluation results for the effectiveness of these solutions are presented in Section 6.3.2.

| Company | comments |
| --- | --- |
| Samsung | Considering the importance of PRU to CPP, we think it should also study the potential spec impact of using PRU in CPP, as for the definition of PRU in TS 38.305, it’s quite unclear on how and when or whether a PRU could provide qualified measurement/report/UL transmission. - The use of the positioning reference unit (PRU) to facilitate NR carrier phase positioning has been evaluated during the study item. Based on the evaluation results, the double differential technique with PRU is beneficial and should be considered for NR carrier phase positioning. - FFS the potential spec impact of using PRU to facilitate NR carrier phase positioning FL: For the potential spec impact of using PRU to facilitate NR carrier phase positioning may be further discussed in the next meeting once we reached the consense of the benefit. |
| Intel | OK; also ok with sub-bullet suggested by Samsung. |
| Vivo | We prefer to list the sub-bullet(e.g detailed result) to illustrate that” the double differential technique with PRU is beneficial for NR carrier phase positioning” FL: For vivo’s comment, I think we may at least list the benefits that most of the companies shares, e.g., TRP synchronization error and the initial phase. - The use of the positioning reference unit (PRU) to facilitate NR carrier phase positioning has been evaluated during the study item. Based on the evaluation results, the double differential technique with PRU is beneficial for NR carrier phase positioning, which include at least the following: - Eliminating the impact of the TRP timing synchronization error - Eliminating the impact of the initial phases of the transmitter and the receiver FL: We could capture more in the Section for the evaluation results. |
| Locaila | No, we don’t support this proposal. In [15], we showed that gNB/TRP and LMF may need to receive offset information from PRUs and broadcast the information as frequently as they transmit PRS due to oscillator instability. Currently defined PRS transmission period is 4 msec ~ 10 seconds. There is no system information broadcast specification of 4 msec intervals. No matter what cycle the LMF broadcasts offset information, this will require a huge specification impact. In our simple calculations, at least 1 Kbps of constant broadcasting resources will be consumed per gNB. Therefore, UE-based positioning will be affected seriously. In a macro or highway environment, the number of surrounding gNB/TRPs that a single PRU can see will be at most 4~7. Suppose that there are 100,000 number of gNB/TRPs deployed. This will require at least 20,000 number of PRU deployment. LMFs must receive phase offset information from 20,000 PRUs at every 250 times per second. If the size of one offset message is about 1 kbit, LMFs must process 5 Gigabit of information every second and perform complex phase calculation. This is a huge network overload. Why is this not an expensive method, and not a burden on 5G networks? Solving this problem would be a nightmare for SA2. FL: Our understanding is that how to use the PRU is really up to the implementation. I would assume NR carrier phase positioning may first be used in indoor small environment, that is why we focus on InF-SH, and InF-DH environment. For highway environment, I am not sure if it is practical and economical to deploy many additional PRUs. I would assume RSU may be used as PRU. |
| Huawei, HiSilicon | We support the original proposal, but not SS, because it is not clear about the intention of adding to somehow a summary or conclusion text proposal to the TR. |
| Lenovo | In response to Samsung’s suggestions, we think an FFS should not be captured in TR 38.859 |
| IIT Kanpur, CEWiT | We support the proposal. And we are okay with the additional sub-bullet added by Samsung. |
| Nokia/NSB | Support the FL proposal. |
| InterDigital | We support adding the second sub-bullet from Samsung to study the potential spec impact of using PRU to support carrier phase positioning. |
| Locaila | We still don’t support FL’s proposal. As for the FL’s argument above, if the primary use of the PRU is for indoor environment, we don’t think PRU is not necessary at all. We think in most cases, all TRPs are wired in indoor environment. There are lots of time synchronization method available in wired network. In a simple way, external oscillators can be connected using RF cables, and the carrier phase of TRPs are perfectly synchronized. In addition, as for FL's idea of combining RSU+PRU for Highway environments, it is cool. Then RSUs should listen to the PRS signals sent by neighboring RSUs. Isn’t this somewhat similar to the master-slave synchronization structure proposed by Locaila in 6.1 below ? If so, would you support our proposal for synchronization structure? Thanks FL: As I commented multitimes previously and also by other companies, the study of gNB synchronization structure is clearly out of the scope of the SI. Thus, although I am personally very interested in the study of gNB time/frequency synchronization methods, including Locaila’s proposal, I would not, unfortunately, support the study of Locaila’s proposal in this SI. Thanks. |
| LGE | We support the FL’s proposal. |
| CATT | We support the FL’s proposal. |

FL Comments:

It seems most companies are fine with the Round 2 FL proposal with the modifications discussed in above table. We will use it for Round 3 discussion.

(H) (Round 3) Proposal 4.1-1

Capture the following TP into TR 38.859 as a conclusion (Section 6.3.1):

- The use of the positioning reference unit (PRU) to facilitate NR carrier phase positioning has been evaluated during the study item. Based on the evaluation results, the double differential technique with PRU is beneficial for NR carrier phase positioning, which includes at least the following:

  - Eliminating the impact of the TRP timing synchronization error

  - Eliminating the impact of the initial phases of the transmitter and the receiver.

- The evaluation results for the effectiveness of these solutions are presented in Section 6.3.2.

| Company | comments |
| --- | --- |
| Locaila | We have argued in previous discussions about the overhead of PRU in 5G NR system. We believe that PRU is not helpful for synchronizing carrier phases in the indoor as well as macro, highway environment. In particular in an indoor environment, TRPs or gNBs can be connected using an external oscillator and RF cable and synchronize carrier phase perfectcly. Any large factory or warehouse environment of hundreds of square meters can be fully synchronized using synchronization cables. Therefore, such system as a PRU and double difference method is not necessary at least in indoor environment. Nevertheless, writing these conclusions can give people the wrong message as if PRU is an essential element in all environments to implement carrier phase positioning. This may make widespread use of 5G carrier phase positioning difficult in market. Therefore, we propose to put following note to convey faithful information to the reader. ================================== Capture the following TP into TR 38.859 as a conclusion (Section 6.3.1): - The use of the positioning reference unit (PRU) to facilitate NR carrier phase positioning has been evaluated during the study item. Based on the evaluation results, the double differential technique with PRU is beneficial for NR carrier phase positioning, which includes at least the following: - Eliminating the impact of the TRP timing synchronization error - Eliminating the impact of the initial phases of the transmitter and the receiver. Note: other means for gNB/TRP synchronization such as using synchronization cable may also be effecitive in some environment (e.g. indoor). - The evolution results for the effectiveness of these solutions are presented in Section 6.3.2. |
| FL | To Locaila: It is unclear to me why “writing these conclusions can give people the wrong message as if PRU is an essential element in all environments to implement carrier phase positioning”. The conclusion here is about the evaluation of “double differential technique with PRU”. The study has not investigated the effectiveness of “other means for gNB/TRP synchronization such as using synchronization cable”. Thus, it is improper to add a note to say “other means for gNB/TRP synchronization such as using synchronization cable may also be effecitive in some environment (e.g. indoor)” . |
| Nokia/NSB | Agree with FL. We don’t understand the comments from Locaila. Actually in their paper they argue that a TRP/gNB can measure the PRS of another TRP/gNB to perform synchronization. This is in effective a similar mechanism to what the PRU does. How can Locaila claim that PRU is not helpful while promoting a very similar technical concept? |
| Huawei, HiSilicon | Share same understanding as FL and Nokia. We also noticed that PRU could be helpful cancelling the gNB ARP error via double differential methods. |
| Qualcomm | Similar view as FL/Nokia/Huawei. Also note that phase synchronization could be much more challenging than time synchronization, especially at higher carrier-frequencies. |
| Ericsson | We have concerns with this proposal. Firstly, why is ‘Eliminating the impact of the TRP timing synchronization error’ even mentioned in the proposal? In our view, study gNB synchronization error mititgation is not part of the study item. So, these issues that are out of scope should not be written part of the conclusions. FL: We have the following agreement to reuse the simulation assumptions of NR Rel-16/17. Agreement - Reuse the simulation assumptions of NR Rel-16/17 for carrier phase positioning - Note: Optional modification of the simulation assumptions defined in NR Rel-16/17 are allowed only if needed. In Network synchronization is included in the simulation assumptions of NR Rel-16/17 (e.g., TS 38.857). In addition, in the agreed template to collect the evaluation results, “(H)(Round 1) Proposal 17-1 in R1-2207690”, we also include the row on the Network synchronization assumption. It is important to consider the potential impact of the network synchronization errors in NR carrier phase positioning. Secondly, we don’t support writing conclusions already in this meeting. Conclusions should be written in the last meeting when results from every company is captured. What we could accept is capturing observations made by different companies and also list how many companies make similar observations just like in other AIs. FL: Okay. We could capture it as an observation in this meeting. This proposal needs further discussion next week. |
| Locaila | Let us explain our point in detail again into three indoor/highway/macro environment. 1) Indoor Environment (e.g InF, IOO, …) Figure 1 External clock signal sharing using RF cable The figure 1 above shows an example of three devices that can input an external clock signal source connected to the master clock oscillator. The upper master oscillator continuously distributes a clock synchronization signal of 10 MHz over the RF cable. The lower three devices output the carrier signal according to the clock signal supplied from the master. As a result, the carrier phase signals transmitted by the three devices are all synchronized as if they were transmitted by single device. Distributed gNB/TRPs in the indoor environment can also be connected by a synchronous cable that shares the clock signal as shown above. In this case, the carrier phase offset between TRPs can be eliminated as close to zero as ideal. Our experimental results show that even if the RF cable was extended several tens of meters, clock signals could still be distributed to remote devices with little loss, making it work like a single system. In other words, even in a large factory or warehouse with a size of several hundred square meters, TRPs can be connected using the synchronization cable. We believe such external clock distribution is a widespread technique to the field engineers. We don’t think it is necessary to investigate and evaluate the already widely practiced method for comparison with the double difference method. Thus PRUs are not necessary, at least in the factory, warehouse, office, and store environment. Simply sharing an external clock signal can effectively synchronize distributed TRPs and eliminate carrier phase errors, better than the proposed double difference method. 2) Highway Environment Figure 2 RSU+PRU highway deployment for carrier phase positioning The figure 2 above illustrates the structure of LMF and the RSUs which combine the function of PRU in a Highway environment. Our FL (CATT) also said RSU can incorporate with PRU, and we agree with that point. The gNBs of the master-slave synchronization structure proposed by Locaila in [15] can similarly be seen as a combined RSU and PRU device in some sense. I think Nokia's comment above is probably because of the structural similarity of RSU+PRU that we propose. However, the key difference between Locaila's proposal, and our main point of argument is not PRU itself, but about the involvement of LMF necessary for the double difference method. Note that LMF will be the place for collecting the phase offset information in UE-assisted-positioning. Similarily, we assume LMF will be the central place for gathering the offset information in following scenario. The figure below illustrates the procedure for exchanging messages with LMF. Since the highway environment is assumed, we suppose a UE-based-positioning in this scenario with a vehicle speed of 100 Km/h and a PRS transmission interval of 4 msec. Figure 3 Message timing diagram in LMF and double difference method RSU#1 and RSU#2 transmit PRS every 4 msec cycles. Since we assumed combined (RSU+PRU) device, they can calculate the carrier phase offset by receiving the PRS signal from the neighboring RSUs. All of these information should be delivered to the LMF. LMF must process phase offset information collected from the (RSU+PRU) devices within 4 msec, and broadcast the information again through RSUs. If there are hundreds of thousands of RSUs deployed, LMF should be able to process several gigabytes of information every 4 msec. We need supercomputing LMF !!!. Calibration information calculated by LMF should be broadcast again in 4 msec cycles via RSUs. Currently, no system broadcast specification has been defined at such frequent intervals. Of course, it will have a significant spec impact. Slow processing of the LMF will cause the vehicle to receive calibration information late and the position accuracy will be degraded. Slowing the PRS transmission cycle to reduce the LMF burden can also increase the positioning error of vehicles. After all, the PRU and double difference method, which requires LMF involvement, may not be appropriate in a Highway environment. Again, this doesn’t denying the combined use of RSU+PRU. Our point is that the double difference method will give burden to LMF and 5G NR system. On the other hand, the figure below depicts the timing procedure for applying the master-slave synchronization structure proposed by Locaila in [15]. Figure 4 Message timing diagram in master-slave fast synchronization [15] Similar to the above case, RSU#1 and RSU#2 transmit PRSs every 4 msec cycles. Since RSUs incorporate the PRU function, they receive the PRS signal from the neighboring RSUs to calculate the carrier phase offset. However in the Locaila [15] proposal, the calculated phase offset is used for rotating the carrier phase of the PRS signal to be transmitted in the next 4 msec cycle. This ensures that the carrier phase of the slave RSU always follows the carrier phase of the master RSU, and that the offset of the carrier phase received by the UE always remains zero. The figure below shows the result of synchronizing the carrier phase of master and slave gNB in the same way as above and observing the carrier phase difference for 5 minutes. Each point represents a phase difference calculated using 8 subcarriers of 2 gNBs. The UE is stationary during the observation period. Even though the master oscillator is drifting, the carrier phase difference of the two signals remains constant because the slave is faithfully following the master carrier phase. Figure 5 phase offset between master-slave gNBs (5min) As shown in the experimental results above, the master-slave structure is simpler than the double difference method, and it can synchronize the carrier phase quickly and effectively, without additing burden to LMF and 5G NR system. 3) Macro In a Macro environment, there should be at most 4~7 gNB/TRPs that a single PRU can see. This will require massive PRU deployment and management costs. If there are more PRU deployments, the traffic overhead concentrated on the LMF will also be very large, and the location accuracy degrade due to the processing delay of the LMF. We doubt the simulation results of the double difference method presented by many companies have considered these points. In conclusion, we argue that the double difference method using PRU is not a very effective means in the indoor/highway/macro environment. In particular it is not useful in the indoor environment. Therefore, we cannot agree with the proposed text above. We ask at least to include following note in the text in order to convey accurate information to the readers. Otherwise, we worry this text may give impression that the proposed double differential technique with PRU is the only method beneficial for NR carrier phase positioning, which is not true at least in indoor environment. ------------------ proposed text ----------------------------- Capture the following TP into TR 38.859 as a conclusion (Section 6.3.1): - The use of the positioning reference unit (PRU) to facilitate NR carrier phase positioning has been evaluated during the study item. Based on the evaluation results, the double differential technique with PRU is beneficial for NR carrier phase positioning, which includes at least the following: - Eliminating the impact of the TRP timing synchronization error - Eliminating the impact of the initial phases of the transmitter and the receiver. Note: other means for gNB/TRP synchronization such as using synchronization cable may also be effecitive in some environment (e.g. indoor). The evolution results for the effectiveness of these solutions are presented in Section 6.3.2 |
| FL | To Locaila: Thanks for further explain the proposed method for gNB/TRP synchronization. I would not make any judgement on proposed method, since the issue here is that the SI scope does not include study of the gNB/TRP synchronization structure such as using synchronization cable. If Locaila still has a different view, I would suggest Locaila to bring the issue to online session, or to the RAN1’ online session Chair. Then, I will follow the guidance the Chair. Thanks! |

FL Comments:

TR 38.859 (Section 6.3.1) is about the Potential Solutions for NR Carrier Phase Positioning. I am hoping every company can agree that double differential technique with PRU is studied as one the potential solutions and

PRU is beneficial for eliminating the impact of the initial phases of the transmitter and the receiver. Whether we can have the conclusion to recommend it for normative work can be further discussed in the next meeting.

### (H) (Round 4) Proposal 4.1-1

Capture the following TP into TR 38.859 (Section 6.3.1):

- The use of the positioning reference unit (PRU) to facilitate NR carrier phase positioning has been studied during the study item.

  - For DL NR carrier phase positioning, the PRU works as a UE to receive the DL PRS reference signals and provide the DL carrier phase measurements to the LMF, where the double differential measurements can be obtained by the difference of the DL carrier phase measurements from the target UE and those from the PRU for eliminating the measurement errors.

  - For UL NR carrier phase positioning, the PRU works as a UE to transmit the UL SRS signals for positioning purpose. The TRPs provides the UL carrier phase measurements obtained from the UL SRS signals of the target UE and of the PRU to the LMF, where the double differential measurements can be obtained by the difference of these UL carrier phase measurements for eliminating the measurement errors.

- Based on the evaluation results, the double differential technique with PRU is beneficial for NR carrier phase positioning, which includes at least the following:

  - Eliminating the impact of the initial phases of the transmitter and the receiver.

  - FFS: Eliminating the impact of the TRP timing synchronization error

- The evaluation results for the effectiveness of these solutions are presented in Section 6.3.2.

| Company | comments |
| --- | --- |

## Integer Ambiguity

Agreement (RAN1#109e) The impact of integer ambiguity on NR carrier phase positioning and potential solutions to resolve the integer ambiguity will be studied in the SI.

Submitted Proposals:

- (Nokia, R1-2208736[3]) Proposal 8: RAN1 to study physical layer procedure to support virtual wavelength method to address integer ambiguity problem.

- (China Telecom, R1-2208774[4]) Proposal 4: support multiple carrier phase measurements, because it is benefit for integer ambiguity removal, and contribute to performance improvement of distance estimation.

- (OPPO, R1-2208801[5]) Proposal 2: Study using carrier phase measurement of multiple carrier frequencies to resolve the issue of integer ambiguity.

- (Intel, R1-2209059[7]) Proposal 5: Study carrier phase integer ambiguity resolution using subcarrier measurements within the DL PRS signal bandwidth.

- (Intel, R1-2209059[7]) Proposal 6: For carrier phase integer ambiguity resolution, a multi-step process to identify coarse timing based on TOA estimate from processing multiple subcarriers and then using CPP for resolution of the fractional part of the carrier wavelength can be considered as a baseline approach.

- (ZTE, R1-2209215[9]) Proposal 2: The scope of integer N should be configured / reported for carrier phase based positioning.

- (ZTE, R1-2209215[9]) Proposal 5: The scope of integer N in OTDOA could be configured based on the measurement uncertainty.

- (ZTE, R1-2209215[9]) Proposal 6: The scope of integer N in AOA/AOD could be configured based on the angle measurement uncertainty.

- (ZTE, R1-2209215[9]) Proposal 7: The scope of integer N for different carrier could be selected based on the frequency feature.

- (Xiaomi, R1-2209293[10]) Proposal 1: Study multi-carrier based carrier phase positioning as the potential solution for integer ambiguity based on UE capability.

- (CMCC, R1-2209343[11]) Proposal 2: At least the following solutions to solve the integer ambiguity are considered feasible:

  - Fast search of integer ambiguity of the carrier phase obtained from more than one frequency;

  - Fast search of integer ambiguity of the carrier phase associated with the legacy positioning techniques.

- (IIT Kanpur, R1-2209546[16]) Proposal 3: For integer ambiguity detection, at least the following options should be studied:

  - Option 1: based on the TOA obtained from the phase-difference of multiple subcarriers within a carrier bandwidth

  - Option 2: based on one single or the combination of existing measurements (e.g., RSTD, RTOA, etc.)

  - Option 3: based on the slope of the carrier phase measurement with respect to frequency

  - Option 4: combination of above approaches

- (Samsung, R1-2209738[17]) Proposal 5: Study using legacy-based positioning measurements to estimate the number of integer cycles between the gNB and the UE.

- (Samsung, R1-2209738[17]) Proposal 6: Study using multiple carriers to determine a phase of a virtual carrier with a lower virtual frequency.

- (Samsung, R1-2209738[17]) Proposal 7: Study using the slope of the carrier phase measurement relative to frequency with multiple sub-carrier measurements to eliminate the integer ambiguity.

- (Sharp, R1-2209785[18]) Proposal 2: Subcarrier-level phase difference report can be discussed as the solution for the integer ambiguity issue.

- (LGE, R1- 2209805[19]) Proposal 5: Method for integer ambiguity resolution should be subject for further studies.

- (MediaTek, R1-2210099[22]) Proposal 3-1: UE may optionally report the uncertainty range of the integer cycle number for the measurement (for example, DL-RSTD measurement)

- (MediaTek, R1-2210099[22]) Proposal 3-2: RAN1 to further study the feasibility of the detection of uncertainty range of the integer cycle number and the reporting of the uncertainty range

FL Comments

In RAN1#109e, it was agreed that “the impact of integer ambiguity on NR carrier phase positioning and potential solutions to resolve the integer ambiguity will be studied in the SI.” Based on the contributions, the following different solutions have been studied:

- Based on the carrier phase measurements from multiple carrier frequencies (e.g., [1][3][4][5][7][10][11][16][17][18]). There are different ways to utilize carrier phase measurements from multiple carrier frequencies. One of them, which is commonly used in GNSS carrier phase positioning, is to use the carrier phase measurements from multiple carrier frequencies, the virtual carrier phase measurements can be created, which have much longer wavelengths, and thereby, much easier and more reliable solutions for integer ambiguity. There can be other approaches to utilize the carrier phase measurements from multiple carrier frequencies, e.g., by exploring the relationship between the integer ambiguities of the carrier phase measurements of different carrier frequencies. In general, how to use the carrier phase measurements from multiple carrier frequencies for reliable solution is mainly the implementation issue;

- With the combination of the legacy positioning measurements, e.g., RSTD, RTOA, to first determine the approximate UE’s position and search range of the integer ambiguities, and the search for the best integer ambiguities based on certain cost function (e.g., [11][16][17][22]). Similar approaches are also commonly used in GNSS carrier phase positioning;

- Using the phase-difference of the subcarriers to first determine the slop/TOA and the search range of the integer ambiguities (e.g., [16][17][18]).

In addition, it is proposed in [9][22] to report the search range of the integer ambiguity based on the UE/TRP measurement quality.

(H)(Round 1) Proposal 4.2-1

Capture the following TP into TR 38.859 (for Section 6.3.1):

- The solutions of integer ambiguity for NR carrier phase measurements have been investigated during the study item. Some of them are listed in the following:

  - The use of the virtual carrier phases, which are obtained based on the linear combination of the carrier phase measurements of multiple frequencies (or positioning frequency layers);

  - The use of the estimated TOA or slope based on the phase-difference of multiple subcarriers;

  - The use of the existing positioning measurements (e.g., RSTD, RTOA)

  - The use of the calculated UE position based on existing positioning methods

  - The combination of the above methods.

- The evaluation results for the effectiveness of these solutions are presented in Section 6.3.2.

| Company | comments |
| --- | --- |
| vivo | Based on my understanding, the above methods are used to get coarse cycles number, and we still need to do integer search within a certain range around the coarse cycles number. But there is no doubt that the better the value of the coarse cycle, the more helpful the search will be. FL: The comment is unclear to me. The proposal is to capture what we have studied during the study item. |
| Nokia/NSB | Okay in principle. Suggest to change the first sub-bullet to say positioning frequency layers FL: when we positioning frequency layers, we may limited to DL only, since the term “positioning frequency layers” is used only for DL in R16. |
| CATT | Support. |
| Samsung | Fine in principle |
| ZTE | We are wondering on the virtual carrier phases from multiple frequencies. For different PFL, there is a random phase offset between two PFL. Hence, the virtual carrier phases may be incorrect. FL: Virtual carrier phases can be formed based on the combination of the double differenced phase measurements. |
| Huawei, HiSilicon | Should “slope based on the phase-difference of multiple subcarriers” be interpreted as negative ToA? FL: I share the similar view. They can be the same information in different forms. |
| MTK | 1, for second subbullet “estimated TOA or slop” slope 2, to resolve the integer ambiguity, in our view, it can be done at “measurement level” and “position calculation level” At measurement level, this is item 2 and 3 At position calculation level, it should be through search by defining a cost function. The linear phase combination, as GNSS said wide-lane and narrow-lane, and as some companies said here “virtual carrier”, may also need to perform the search (brute force). So we suggest item 1 to revise the wording The search during position calculation. Further, the use of the virtual carrier phases, which are obtained based on the linear combination of the carrier phase measurements of multiple frequencies may have the potential to reduce the search 3, in our investigation, the integer ambiguity search/estimation at measurement level could assist the search at position calculation level, for example, by providing a smaller uncertainty range for LMF to search. I am wondering whether FL could also capture this? FL: Add a bullet “The use of the calculated UE position or UE position error ranges based on existing positioning methods |
| CMCC | We are generally fine with the TP. Similar views as companies that for the “multiple frequencies” of the 1st sub-bullet, it should be clearer, does it multiple PFLs, or multiple subcarriers, etc? FL: In my view, it can be PFLs (for DL case) or multiple subcarriers if the reporting of multiple subcarriers are supported. |
| Xiaomi | Fine with this proposal |
| Intel | OK in general, but as pointed out by ZTE, we think clarification of “multiple frequencies” would be necessary here. In our view, this includes at least use of measurements for different subcarriers within a carrier, but not yet sure of feasibility/usefulness of different frequency layers unless phase coherency can be assumed. Also, a small typo: “slop” should be “slope”. FL: It means both in my mind when the proposal was written. We can have further clarification based on the conclusion for the Proposal in Section 3.2. |
| Ericsson | From an information point of view, TOA (including a clock offset) is equivalent RSTD, RTOA. Hence we can merge bullets 2 and 3: - The use of the estimated TOA or slope of arg⁡(Hf) based on the phase-difference of multiple subcarriers. The use of the existing positioning measurements (e.g., RSTD, RTOA). Typo: TOA or slope based FL: There is a difference. TOA/slope are not defined in R16/R17. |
| Locaila | Ok |
| Philips | In general, we support the proposal but we share ZTE’s concern about virtual carrier phase. |
| LGE | We are generally fine with the proposal, but have similar view with other companies that “multiple frequencies” in the first sub-bullet should be modified for more clear meaning. This issue can be discussed after the discussion in proposal 3.2-1 is stabilized. |
| IIT Kanpur, CEWiT | We support the proposal. |
| Apple | Fine with proposal |

(H)(Round 2) Proposal 4.2-1

Capture the following TP into TR 38.859 (for Section 6.3.1):

- The methods of searching integer ambiguity for NR carrier phase measurements have been investigated during the study item. Some of them are listed in the following:

  - The use of the virtual carrier phases, which are obtained based on the linear combination of the carrier phase measurements of multiple subcarriers within a carrier, or multiple positioning frequency layers;

  - The use of the estimated slope based on the phase-difference of multiple subcarriers;

  - The use of the existing positioning measurements (e.g., RSTD, RTOA)

  - The use of the calculated UE position based on existing positioning methods

  - The combination of the above methods.

- The evaluation results for the effectiveness of these solutions are presented in Section 6.3.2.

| Company | comments |
| --- | --- |
| Samsung | OK |
| MTK | 1, suggest some wording change. “The use of the virtual carrier phases” “To reduce the search effort, the phases of a virtual carrier are used” the reason is, without linear combination of phases, LMF can still search. So we may justify the reason why to combine the phases, and the reason is to reduce the search effort 2, Just want to check that if there are some other methods to be evaluated in next meeting, we expect the new methods could also be captured into the TP FL: I assume a good approach should not only reduce the search range but also be reliable, e.g., identify the integer ambiguity correctly. So, using the word “resolving” as suggest by Intel may be better. We can only ‘at least’, so as can add new methods. |
| Intel | OK. Editorial suggestion: change “methods of searching integer ambiguity” to “methods of resolving integer ambiguity”. FL: good suggestion of the wording. |
| Vivo | We share the same understanding with MTK and think all the listed methods may be only used to reduce the search effort for high accuracy cpp, and then, anyway, “the use of cost function for searching integer” is needed, so it can be added as a method. FL: We can add “the use of cost function for searching integer” into the list. |
| Locaila | In the second sub-bullet, estimating the phase slope is one of the methods for integer number resolution, and there are other similar methods. We propose to avoid the specific word for generalization. - The methods of searching integer ambiguity for NR carrier phase measurements have been investigated during the study item. Some of them are listed in the following: - The use of the virtual carrier phases, which are obtained based on the linear combination of the carrier phase measurements of multiple subcarriers within a carrier, or multiple positioning frequency layers; - The use of the information estimated slope based on the phase-difference of multiple subcarriers; - The use of the existing positioning measurements (e.g., RSTD, RTOA) - The use of the calculated UE position based on existing positioning methods - The combination of the above methods. % a lengthy explanation is necessary for our reason for asking this modification. Slope calculation is an analytic method. There’s lots of papers showing that iterative method is more effective than the analytic method at least for the problem of integer number search. We also had similar experience and this should be shared and discussed in detail. Thanks. |
| Huawei, HiSilicon | We agree with the FL, we suggest to merge the following bullets. - The use of the estimated slope based on the phase-difference of multiple subcarriers; - The use of the existing positioning measurements (e.g., RSTD, RTOA) It may read - The use of the existing positioning measurements (e.g., RSTD, RTOA) including using the estimated slope based on the phase-difference of multiple subcarriers to obtain the high resolution ToA FL: The suggested change looks fine to me. |
| Lenovo | Agree with Intel’s suggestion |
| FL | ### (H)(Round 2) Proposal 4.2-1 Capture the following TP into TR 38.859 (for Section 6.3.1): - The methods of resolving integer ambiguity for NR carrier phase measurements have been investigated during the study item, which include at the following: - The use of the virtual carrier phases, which are obtained based on the linear combination of the carrier phase measurements of multiple subcarriers within a carrier, or multiple positioning frequency layers; - The use of the estimated slope based on the phase-difference of multiple subcarriers; - The use of the existing positioning measurements (e.g., RSTD, RTOA), which includes the use of the estimated slope based on the phase-difference of multiple subcarriers to obtain the high resolution ToA - The use of the calculated UE position based on existing positioning methods - The use of the cost functions - The combination of the above methods. - The evaluation results for the effectiveness of these solutions are presented in Section 6.3.2. |
| IIT Kanpur, CEWiT | We support the updated proposal. |
| Nokia/NSB | We support the updated FL proposal. |
| MTK | 1, , which include at the following , which include at least the following 2, it is good to add “the use of the cost function” proposed by vivo. Somehow “the use of the calculated UE position based on existing positioning methods” may also be related to usimg the cost function. For example, given a set of RSTD values (by existing DL-TDOA method) with adjustment by purified fractional cycle amount (through double difference), the least square algorithm (Chan’s algorithm) could calculate UE position. Based on this UE position, the integer cycle and fractional cycle amount for each link could be derived. The derived fractional cycle may compare with the purified fractional cycle to check the squared difference. Then for any RSTD value to be adjusted by using wavelength/light speed as the unit, a new fractional cycle could be derived and it is further compared with the purified fractional cycle to squared difference. The squared difference could be treated as the cost to determine how each RSTD value is adjusted by using wavelength/light speed at the unit |
| InterDigital | Support |
| Xiaomi | We support the latest proposal from FL |
| LGE | We support the updated proposal from FL. |
| CATT | We support the latest proposal from FL |

FL Comments:

It seems most companies are fine with the latest FL modifications discussed in above table. We will use it for Round 3 discussion.

(H)(Round 3) Proposal 4.2-1

Capture the following TP into TR 38.859 as a conclusion (for Section 6.3.1):

- The methods of resolving integer ambiguity for NR carrier phase measurements have been investigated during the study item, which includes, but not limited to, the following:

  - The use of the virtual carrier phases (a virtual carrier phase is defined as an linear combination of the carrier phase measurements of multiple subcarriers within a carrier, or multiple positioning frequency layers);

  - The use of the estimated slope based on the phase-difference of multiple subcarriers;

  - The use of the existing positioning measurements (e.g., RSTD, RTOA)

  - The use of the calculated UE position based on existing positioning methods

  - The use of the cost functions (e.g., Huawei[R1-220xxxx,], CATT[4])

  - The combination of two or more of the above methods.

- The evaluation results for the effectiveness of these solutions are presented in Section 6.3.2.

| Company | comments |
| --- | --- |
| MTK | Generally okay. But our previous comment saying that “The use of the calculated UE position based on existing positioning methods” could also be based on defining the cost function. So we are not so sure the difference between the two subbullets. |
| ZTE | We prefer revise the main bullet as - The methods of resolving integer ambiguity for NR carrier phase measurements have been investigated during the study item, which includes, but not limited to, the following: And for the sub-bullet - The use of the cost functions My concern is that how to define the cost function or evaluation parameters that is related to the cost function. Maybe can describe more details about the cost function or proposal reference for this sub-bullet. |
| FL | To MTK/ZTE: FL: I assume there will be some overlapping between the methods. The inputs of the cost function can be “calculated UE position”, “the measured carrier phase, and other information”. The definition of the cost function is normally implementation issue. |
| Samsung | We would like to keep the “estimated slope as separate sub-bullet” (which it it already is). In our understanding the existing positioning methods refer to Rel-16/17 methods which don’t include getting the estimated slope. We would also like to add “combination of two or more of the above methods” Therefore, we prefer the following: - The methods of resolving integer ambiguity for NR carrier phase measurements have been investigated during the study item, which include at the following: - The use of the virtual carrier phases, which are obtained based on the linear combination of the carrier phase measurements of multiple subcarriers within a carrier, or multiple positioning frequency layers; - The use of the estimated slope based on the phase-difference of multiple subcarriers; - The use of the existing positioning measurements (e.g., RSTD, RTOA), which includes the use of the estimated slope based on the phase-difference of multiple subcarriers to obtain the high resolution ToA - The use of the calculated UE position based on existing positioning methods - The use of the cost functions - The combination of two or more of the above methods. |
| FL | To Samsung: The suggestion to remove “which includes the use of the estimated slope based on the phase-difference of multiple subcarriers to obtain the high resolution ToA” is okay to me. Let us wait to see if other companies are fine with it. |
| Nokia/NSB | Support the update from SS. |
| Huawei, HiSilicon | We do not agree with argument from SS. In fact, HW’s evaluation results submitted in TR 38.855 using super-resolution is based on phase slope, and we think SS’s understanding that Rel-16 positioning methods do not include getting the estimated slope is not correct. One can easily approve that for an ideal single path dArgCFRfdf=dArgexp(-2πjτf))df=-2πτ The phase slope is equivalent to ToA. We even used least square on the unwrapped channel phase response to combat the noise. The reason that we use MUSIC in Rel-17 is because multi-path limits the accuracy of phase LS and the accuracy of phase LS cannot meet Rel-17 requirement. Anyway, there are multiple implementation methods to get high accuracy ToA, and phase slope, as one way of implementation, is nothing new, and should not be listed. We do not want to give external readers the impression that RAN1 NR positioning after two Releases still think that ToA and phase slope are different concepts.. |
| FL | To Huawei, In general, I would agree with Huawei from the phase slope, the ToA can be determined. |
| Ericsson | strictly speaking this is more of a statement of different solutions used rather than a conclusion. Also, we should list all the different techniques used and should avoid things like ‘but not limited to’ in the TR. If the list is incomplete, we can amend it next meeting. Some specific comments: -> Is the term ‘virtual carrier phases’ proper defined or well understood by everyone? If not, we should not mention it in the TR. -> the use of cost function is an implementation, but if we are listing it here we may need to provide some more information |
| Intel | The bullet “The use of cost functions” is rather vague and too abstract. Without further clarirty, almost any approach can be formulated to involve some kind of cost function. Also, agree with Ericsson that “not limited to” is something not appropriate for the TR, especially when it is about the techniques studied – we should be precise about what has been studied. |
| FL | To Ericsson/Intel: I share the similar view that the wording ‘but not limited to’ can be removed. The “virtual carrier phases” is actually described in the proposal. We may say: virtual carrier phase, which is defined as the linear combination of the carrier phase measurements of multiple subcarriers within a carrier, or multiple positioning frequency layers For cost function, maybe we can add a list of the sources that use the cost function. |

FL Comments:

The proposal attempts to summarize different methods of resolving integer ambiguity that are investigated in the SI with the references of the sources that use the methods.

### (H)(Round 4) Proposal 4.2-1

Capture the following TP into TR 38.859 (for Section 6.3.1):

- The methods of resolving integer ambiguity for NR carrier phase measurements have been investigated during the study item, which includes the following:

  - The use of the carrier phase measurements of multiple subcarriers within a carrier or multiple positioning frequency layers (Huawei [1], CATT[4])

  - The use of the estimated slope based on the phase-difference of multiple subcarriers (e.g., Samsung[17]);

  - The use of the existing positioning measurements, including RSTD, RTOA (Huawei [1], CATT[4], ZTE[9])

  - The use of the calculated UE position based on existing positioning methods (MediaTek[22])

  - The use of the cost functions (Huawei[1], CATT[4])

Note: Different cost functions are used by different companies (e.g., Huawei[1], CATT[4], MediaTek[22]))

  - The use of brute-force integer-ambiguity-resolution (IAR) (Qualcomm[21])

  - The combination of two or more of the above methods.

| Company | comments |
| --- | --- |
| FL | For the methods of resolving integer ambiguity, different companies have used different approaches. I am not actually not sure the methods used b ya companies unless it is indicated clearly in the contributions. Anyway, please let me know the methods you are using, so that I can add them into above proposal. |

(Round 1) Proposal 4.2-2

- Further study the impact of the solutions of integer ambiguity on the specification, which includes, but not limited to, the following:

  - Reporting of the TOA or slop obtained from the phase-difference of multiple subcarriers

  - Reporting of the search range of the integer ambiguity from UE/TRP

  - Reporting of the carrier phase measurements together with the legacy positioning measurements

| Company | comments |
| --- | --- |
| CATT | Okay to further study |
| Samsung | Fine in principle |
| ZTE | Besides the above reporting elements, LMF can also configure the integer ambiguity to UE while implementing CPP. So we suggest revise the proposal as follows: - Further study the impact of the solutions of integer ambiguity on the specification, which includes, but not limited to, the following: - Reporting of the TOA or slop obtained from the phase-difference of multiple subcarriers - Reporting of the search range of the integer ambiguity from UE/TRP - Reporting of the carrier phase measurements together with the legacy positioning measurements - LMF Configure the integer ambiguity between the TRP and target UE FL: It is unclear to me why and how the LMF configure the integer ambiguity between the TRP and target UE. Does it mean the LMF tell the search range of integer ambiguity for UE for UE-based positioning? If so, how does the LMF get the information? Keep in mind that in this case the LMF needs to know UE’s position pretty accuracy in order to provide such information. |
| Xiaomi | We want to clarify the motivation for reporting of the search range of the integer ambiguity. In my understanding, if it is UE-assisted positioning, UE report the carrier phase of multiple frequencies and LMF will perform the searching. And if it is UE based positioning, UE will perform the searching and calculate the position of UE, and it is also unnecessary to report the search range. FL: I assume the “Reporting of the search range of the integer ambiguity from UE/TRP” is for LMF-based. |
| Intel | OK. A small typo: “slop” should be “slope”. |
| Locaila | Ok |
| Philips | Support |
| LGE | Fine with further study |
| IIT Kanpur, CEWiT | We are okay with further study. |
| Apple | Fine with proposal |

FL Comments

It seems most of the feedbacks are supportive to (Round 1) Proposal 4.2-2. Thus, I change its Priority to High.

(H)(Round 2) Proposal 4.2-2

- Further study the impact of the solutions of integer ambiguity for UE-assisted carrier phase positioning on the specification, which includes, but not limited to, the following:

  - Reporting of the slope obtained from the phase-difference of multiple subcarriers from UE/TRP to LMF

  - Reporting of the search range of the integer ambiguity from UE/TRP from UE/TRP to LMF

  - Reporting of the carrier phase measurements together with the legacy positioning measurements from UE/TRP to LMF

| Company | comments |
| --- | --- |
| Samsung | OK |
| MTK | 1, for first subbullet, the slope could be derived by high resolution receiver, and the slope is the TOA. Then it should have been supported in previous release. 2, okay for second subbullet. The search effort could be quite large for LMF without any potential guidance |
| Intel | OK |
| Locaila | Again, please avoid using the specific word ‘slope’. - Further study the impact of the solutions of integer ambiguity for UE-assisted carrier phase positioning on the specification, which includes, but not limited to, the following: - Reporting of the information slope obtained from the phase-difference of multiple subcarriers from UE/TRP to LMF - Reporting of the search range of the integer ambiguity from UE/TRP from UE/TRP to LMF - Reporting of the carrier phase measurements together with the legacy positioning measurements from UE/TRP to LMF |
| Huawei, HiSilicon | The slope of phase-difference from multiple subcarrier is equivalent to ToA reporting. The unit of phase slope against frequency is sec. We do not need to study any spec impact for this. |
| Lenovo | OK |
| FL | To MTK/HW: If companies consider there is no need to study the impact of ToA/Slope. We can remove it. - Further study the impact of the solutions of integer ambiguity for UE-assisted carrier phase positioning on the specification, which includes, but not limited to, the following: - Reporting of the search range of the integer ambiguity from UE/TRP from UE/TRP to LMF - Reporting of the carrier phase measurements together with the legacy positioning measurements from UE/TRP to LMF |
| IIT Kanpur, CEWiT | We support the updated proposal. |
| Nokia/NSB | We are okay with the updated FL proposal. |
| InterDigital | Support the latest version of from the FL. |
| Locaila | As we explained in 4.2-1 above, slope calculation is only one of the methods. There’s other optimization methods effective for resolving integer number. Our proposal is not to use limiting word of specific implementation method. We propose following wording. - Further study the impact of the solutions of integer ambiguity for UE-assisted carrier phase positioning on the specification, which includes, but not limited to, the following: - Reporting of the information slope obtained from the phase-difference of multiple subcarriers from UE/TRP to LMF - Reporting of the search range of the integer ambiguity from UE/TRP from UE/TRP to LMF - Reporting of the carrier phase measurements together with the legacy positioning measurements from UE/TRP to LMF |
| Xiaomi | We support the latest proposal from FL |
| LGE | We prefer to have the 1st sub bullet in the original version. Intention of this proposal is to list potential candidate for reporting. The “slop obtained from the phase-difference of multiple subcarrier” can be design to have different reporting contents from exsiting positioning method. Moreover we can consider higher resolution for TOA(or TDOA) reporting to resolve integer ambiguity more precisely. Thus, we prefer to have seperate subbullet. |
| CATT | Support |
| CATT | Support |

FL Comments:

It seems most companies are fine with the latest FL modifications discussed in above table. But, there is at least one company (LGE) that prefer to keep the removed first bullet. Let us try to take Locaila’s suggestion for the change to see if it is fine to everyone.

(H)(Round 4) Proposal 4.2-2

- Further study the impact of the solutions of integer ambiguity for UE-assisted carrier phase positioning on the specification, which includes, but is not limited to, the following:

  - FFS: Reporting of the information (e.g., slope) obtained from the phase-difference of multiple subcarriers from UE/TRP to LMF

  - Reporting of the determined integer ambiguity and/or the search range of the integer ambiguity from UE/TRP from UE/TRP to LMF

  - Reporting of the carrier phase measurements together with the legacy positioning measurements from UE/TRP to LMF

  - LMF Configure the integer ambiguity range between the TRP and target UE

| Company | comments |
| --- | --- |
| ZTE | Besides the above reporting elements, LMF can also configure the integer ambiguity to UE while implementing CPP. So we suggest revise the proposal as follows: - Further study the impact of the solutions of integer ambiguity on the specification, which includes, but not limited to, the following: - Reporting of the TOA or slop obtained from the phase-difference of multiple subcarriers - Reporting of the search range of the integer ambiguity from UE/TRP - Reporting of the carrier phase measurements together with the legacy positioning measurements - LMF Configure the integer ambiguity between the TRP and target UE FL: It is unclear to me why and how the LMF configure the integer ambiguity between the TRP and target UE. Does it mean the LMF tell the search range of integer ambiguity for UE for UE-based positioning? If so, how does the LMF get the information? Keep in mind that in this case the LMF needs to know UE’s position pretty accuracy in order to provide such information. Reply to the FL’s reply: Does it mean the LMF tell the search range of integer ambiguity for UE for UE-based positioning?->Not exacty, not for UE-based, instead, I guess LMF configuration is applicable for UE-assisted positioning. For UE assisted positioning, UE can report the search range or measurement information together to the LMF, or UE can also report the measurement information only, and LMF can configure the integer ambiguity according to the received measurement information. That is to say, the integer ambiguity can be determined by UE or LMF. Can see whether other companies have similar concern. For UE assisted method, UE has no idea on its location. LMF can get the UE location from the existing positioning method or previous positioning results (for example, in last measurement, etc). Definitely, LMF can configure a integer range for UE to reduce UE searching burden. - LMF Configure the integer ambiguity range between the TRP and target UE |
| FL | To ZTE: Thanks for the explanation of the motivation to add “LMF Configure the integer ambiguity range between the TRP and target UE”. I assume it can be added for study. |
| Samsung | Prefer to keep slope in first sub-bullet - Reporting of the information obtained from the phase-difference of multiple subcarriers (e.g., slope) from UE/TRP to LMF |
| FL | To Samsung: It we want to add “(e.g., slope)”, it may be better to be placed after “Reporting of the information”. |
| Nokia/NSB | What information will be reported in the first sub-bullet? Just the slope? We also prefer to add a bullet for “reporting of the determined integer ambiguity from UE/TRP to LMF”. |
| FL | To Nokia: The information proposed by companies include the slope, TOA. I think other information may also be useful, e,g., the uncertainty information, which are related to the search range. I aasume “reporting of the determined integer ambiguity from UE/TRP to LMF” is closely related to “Reporting of the search range of the integer ambiguity from UE/TRP”. |
| Samsung | We are fine with the latest proposal. For the last sub-bullet, it is better to reword to be similar to the other sub-bullets LMF Configure Configuring the integer ambiguity range between the TRP and target UE by the LMF |
| Huawei, HiSilicon | Do not support if the first bullet is not removed. |
| Qualcomm | Agree with Huawei about 1st sub-bullet |
| FL | To Huawei/Qualcomm/Samsung and all: Will it be acceptable to add “FFS” to the first bullet? |
| Ericsson | ok to study further. |

### (H)(Round 4) Proposal 4.2-2

- Further study the impact of the solutions of integer ambiguity for UE-assisted carrier phase positioning on the specification, which includes, but is not limited to, the following:

  - Reporting of the determined integer ambiguity and/or the search range of the integer ambiguity from UE/TRP from UE/TRP to LMF

  - Reporting of the carrier phase measurements together with the legacy positioning measurements from UE/TRP to LMF

  - LMF Configure the integer ambiguity range between the TRP and target UE

  - FFS: Reporting of the information (e.g., slope) obtained from the phase-difference of multiple subcarriers from UE/TRP to LMF

| Company | comments |
| --- | --- |

## Multipath for Carrier Phase Positioning

Agreement (RAN1#109e) - The impact of multipath for the carrier phase positioning will be evaluated during the SI - The methods of mitigating the impact of multipath for the carrier phase positioning will be studied during the SI, if it is considered to be necessary after the evaluation. Agreement (RAN1#110) - Further evaluate the following multipath mitigation methods for the carrier phase positioning, which include, but are not limited to, the following: - The methods of estimating the carrier phase of the first path - Note: Both time-domain and frequency-domain methods can be considered - LOS/NLOS/ Multi-path indication for the carrier phase measurements for improving the accuracy of the position calculation - Rel-17 LOS/NLOS indicator can be used as the starting point - measurements of the first path and additional paths - E.g. carrier phase measurements, timing measurements - other channel information, such as RSRP/RSRPP, CIR/CFR, etc.

Submitted Proposals:

- (vivo, R1-2208650[2]) Proposal 4:	If carrier phase positioning is not a standalone positioning method, the carrier phase measurement can share existing LoS indication with existing positioning methods.

- (vivo, R1-2208650[2]) Proposal 5:	For multipath mitigation, the carrier phase measurement in the frequency domain with multipath mitigation（option D）can be adopted.

- (Nokia, R1-2208736[3]) Proposal 7: RAN1 to study methods for identifying independent taps and/or methods to aid a positioning receiver to determine and report measurements of independent channel taps that correspond to different wireless propagation paths.

- (CATT, R1-2208983[6]) Proposal 3: For the indication of LOS/NLOS for Rel-18 NR CPP, the LOS/NLOS indication in Rel-17 can be the starting point.

- CATT, R1-2208983[6]) Proposal 9: The candidate information in the measurement reporting for carrier phase positioning may include:

  - Measurement quality indications, which include one or a combination of LOS/NLOS indicator, Rician factor, SINR and variance of CPP measurement.

- (Intel, R1-2209059[7]) Proposal 1: Study LOS/NLOS links classification for the carrier phase measurements to improve the accuracy of the carrier phase positioning methods.

  - As a starting point, consider use of Rel-17 LOS/NLOS indicator reporting for NR carrier phase positioning methods.

- (Xiaomi, R1-2209293[10]) Proposal  6: Support the carrier phase for first path and additional path, and to report the likelihood of LoS path by LoS/NLoS indicator for each path carrier phase.

- (CMCC, R1-2209343[11]) Observation 2: By estimating the first path, the carrier phase measurement in time domain can help suppress the impact of multipath.

- (InterDigital, R1-2209489[13]) Proposal 11: Study the condition under which carrier phase should be measured and reported based on e.g.,

  - LOS/NLOS value

  - RSRP

- (Fraunhofer, R1-2209534[14]) Proposal 1: 	The carrier phase shall be determinable for different parts of the CIR , wherein each part may represent a path related to a multipath cluster reporting of the channel impulse response and shall be measured in the delay domain.

- (Fraunhofer, R1-2209534[14]) Proposal 2: 	Support reporting the phase measurements from the delay domain for the first and additional paths (N-path reporting for the phase).

- (Fraunhofer, R1-2209534[14]) Proposal 3:	Consider one or more of the following options for the delay domain phase measurement:

  - Option 1: The first path and N-path reporting is extended by a phase value for each reported path

  - Option 2: A magnitude and phase of the complex valued correlation value associated to the path is reported.

  - Option 3: The full complex valued CIR for a given length is reported.

  - Option 4: Relevant parts of the complex valued CIR are reported.

- (Samsung, R1-2209738[17]) Proposal 9: Study the condition for applying CPP method based on the NLOS and multi-path including first path detection.

- (Ericsson, R1-2210177[23]) Proposal 2	Any definition of carrier phase measurements should consider the aspect of multipath propagation.  i) One option is to assume that the measurement is for the first path. ii) Another option is to define carrier phase measurements for additional paths.

FL Comments

The performance of the carrier phase positioning is expected to be impacted by multiple factors as other positioning methods. In RAN1#109e, it was agreed to evaluate the impact of multipath for the carrier phase positioning, and study the methods of mitigating the impact of multipath for the carrier phase positioning.

Based on the contributions submitted in this meeting, many companies have studied the multipath mitigation for NR carrier phase positioning (e.g., [2][3][6][7][10][11][12][13][14][17][23]). Different approaches were also proposed for multipath mitigation, e.g., identify/separate the first path from the other paths (e.g., [2][3][10][11][14][17][23], using LOS/NLOS indication (e.g., [2][3][6][7][10][11][13][17]), as well as reporting additional channel information ([7][12][15]).

(H)(Round 1) Proposal 4.3-1

Capture the following TP into TR 38.859:

- The impact of multipath/NLOS on NR carrier phase positioning is evaluated during the study item. Based on the study, it is concluded that multipath/NLOS deteriorates the performance of carrier phase positioning and it is necessary to consider multipath mitigation for NR carrier phase positioning.

| Company | comments |
| --- | --- |
| InterDigital | Support |
| Nokia/NSB | We are not sure this is needed. Clearly multipath plays a role in all positioning methods and this is well known. What do we gain by adding this to the TR? |
| CATT | Support |
| Samsung | Suggest to add multi-path indication - The impact of multipath/NLOS on NR carrier phase positioning is evaluated during the study item. Based on the study, it is concluded that it is necessary to consider multipath mitigation or indication for NR carrier phase positioning. FL: I assume “indication” introduced in R17 is just one of the multipath mitigation methods. |
| ZTE | Support. We propose modify the description: The impact of multipath/NLOS on NR carrier phase positioning is evaluated during the study item and multipath/NLOS deteriorate the performance of carrier phase positioning. Based on the study, it is concluded that it is necessary to consider multipath mitigation for NR carrier phase positioning. |
| MTK | The sentence is okay for us as the observation. (similar case at sidelink POS for the increasing BW to improve accuracy for all cases) |
| CMCC | The intention of this TP is not quite clear, as of course multipath/NLOS will impact the performance. Basically, it would be better if we can capture something in TR like how many companies provide evaluations on mitigating multipath/NLOS on carrier phase positioning, and performance improvements with respect to solutions do not apply enhancements are observed. But similar as the situation as what we have agreed in yesterday’s online for SL positioning, if companies are fine with the TP, we can live with this general observation. |
| Xiaomi | Support and prefer Samsung’s version. |
| Intel | Support. |
| Ericsson | Ok. We tend to agree with CMCC comment, that it is better to capture the evaluation results and the observations related to the evaluation results in the TR instead of this statement. |
| Qualcomm | Same view as Nokia. Further, we think that just timing based methods report measurements based on earliest detected path, similarly, reporting phase measurement of the earliest detected path will be sufficient to address this issue. |
| Locaila | Ok |
| Philips | Support |
| LGE | Similar view with CMCC and Ericsson. It would be better to capture the evaluation results related to multipath impact. |
| IIT Kanpur, CEWiT | We support the proposal. |
| Apple | Fine with proposal |
| FL | For the suggestion of CMCC and Ericsson, we will consider adding the evaluation results in Section 6.3.2. ### (H)(Round 1) Proposal 4.3-1 Capture the following TP into TR 38.859 (for Section 6.3.1): - The impact of multipath/NLOS on NR carrier phase positioning is evaluated during the study item. Based on the study, it is concluded that multipath/NLOS deteriorates the performance of carrier phase positioning and it is necessary to consider multipath mitigation for NR carrier phase positioning. - The evaluation results for the impact of the multipath/NLOS on NR carrier phase positioning will be presented in Section 6.3.2. |
| CATT | Support |

### (H)(Round 3) Proposal 4.3-1

Capture the following TP into TR 38.859 as a conclusion (for Section 6.3.1):

- The impact of multipath/NLOS on NR carrier phase positioning is evaluated during the study item. Based on the study, it is concluded that multipath/NLOS deteriorates the performance of carrier phase positioning and it is necessary to consider multipath mitigation for NR carrier phase positioning.

- The evaluation results for the impact of the multipath/NLOS on NR carrier phase positioning will be presented in Section 6.3.2.

| Company | comments |
| --- | --- |
| ZTE | Support |
| Qualcomm | Generally supportive, but as has been pointed out by others in previous rounds as well, multipath mitigation in some form or the other is an integral part of almost any positioning scheme. So regarding the second sub-bullet, how will we organize a separate section devoted to multipath mitigation results? (in our view every result shown could qualify to be placed in that section) FL: In my understanding, Section 6.3.1 is more related to what issues have been studied. And Section 6.3.2 for more details of the evaluation results. |

(H)(Round 1) Proposal 4.3-2

Capture the following TP into TR 38.859:

- The methods of mitigating the impact of multipath for the carrier phase positioning are studied during the study item. Based on the study, the following candidate methods of multipath mitigation are suggested for further consideration:

  - The identify and separate the first path and other paths in time-domain/CIR

  - The reporting of the carrier phase of the first path, and optionally, the additional paths

  - The use of LOS/NLOS indication for the carrier phase measurements

    - Note: Rel-17 LOS/NLOS indicator reporting can be considered as a starting point, e.g., sharing of the indicator

  - The report of other channel information, such as RSRP/RSRPP/SINR, CIR/CFR, etc.

- FFS: The impact of the candidate methods on the specification.

| Company | comments |
| --- | --- |
| InterDigital | We propose to remove the revise the note as follows since how LOS/NLOS indicator can be used for phase measurement is not clear yet. Note: Rel-17 LOS/NLOS indicator reporting can be considered as a starting point, e.g., sharing of the indicator |
| Nokia/NSB | Support. |
| CATT | Support |
| Fraunhofer | Okay in principle; remove /SINR, CIR/CFR, etc. |
| Samsung | Fine in principle. We would like to add the multipath indication here as one of the mitigation methods: Identification and reporting of the multi-path status |
| ZTE | From our perspective, the second bullet concerning the report of the additional paths’ carrier phase shouldn’t be considered, because there’s no simulation result show that the carrier phases of additional paths can improve the CPP performance. So the proposal could be revised as follows: - The methods of mitigating the impact of multipath for the carrier phase positioning are studied during the study item. Based on the study, the following candidate methods of multipath mitigation are suggested for further consideration: - The identify and separate the first path and other paths in time-domain/CIR - The reporting of the carrier phase of the first path, and optionally, the additional paths - The use of LOS/NLOS indication for the carrier phase measurements - Note: Rel-17 LOS/NLOS indicator reporting can be considered as a starting point, e.g., sharing of the indicator - The report of other channel information, such as RSRP/RSRPP/SINR, CIR/CFR, etc. - FFS: The impact of the candidate methods on the specification. |
| MTK | For the first subbullet, The identify and separate the first path and other paths in time-domain/CIR, we don't need to talk about time-domain/CIR since it is related to implementation. We also evaluate using high resolution receiver (MOE detector in our paper) to estimate the phase corresponding to first path in frequency domain. So let it be determined by implementation. Then we don't think the first subbullet is needed |
| CMCC | Support |
| Xiaomi | Fine, and also suggest to remove “e.g., sharing of the indicator”, and “/SINR, CIR/CFR” |
| Intel | Support. |
| Ericsson | Ok. Remove ‘The’ in the first bullet: (i.e., ‘The identify and separate’ -> ‘Identify and separate’). |
| Locaila | Ok |
| Philips | Support |
| LGE | We have similar view with ZTE. We are not sure how much gain can be achieved with additional path reporting. |
| IIT Kanpur, CEWiT | These four bullets are fine but other methods should not be precluded, we suggest to delay this agreement for the next meeting. |
| Apple | OK |

FL Comments

It seems most of the feedbacks are supportive to (Round 1) Proposal 4.3-2 with small comments. The proposal is revised for further discussion.

(H)(Round 2) Proposal 4.3-2

Capture the following TP into TR 38.859:

- The methods of mitigating the impact of multipath for the carrier phase positioning are studied during the study item. Based on the study, the following candidate methods of multipath mitigation are suggested for further consideration:

  - The identify and separate the first path and other paths in time-domain/CIR

  - The reporting of the carrier phase of the first path, and optionally

  - The use of LOS/NLOS indication for the carrier phase measurements

    - Note: Rel-17 LOS/NLOS indicator can be considered as a starting point

  - The report of other channel information

  - Other methods are not precluded

- FFS: The impact of the candidate methods on the specification.

| Company | comments |
| --- | --- |
| Samsung | Suggest the following update: - The methods of mitigating the impact of multipath for the carrier phase positioning are studied during the study item. Based on the study, the following candidate methods of multipath mitigation are suggested for further consideration: - The reporting of the carrier phase of the first path - The use of LOS/NLOS indication for the carrier phase measurements - Note: Rel-17 LOS/NLOS indicator can be considered as a starting point - The report of other channel information - Identification and reporting of the multi-path condition - Other methods are not precluded - FFS: The impact of the candidate methods on the specification. FL: I assume “o Identification and reporting of the multi-path condition” is coverd already by the second-subbullet. |
| MTK | For the first subbullet, the “and optionally” at the end of the sentence should be deleted? FL: Okay |
| Intel | OK. |
| Huawei, HiSilicon | Agree with MTK. |
| Lenovo | agree with MTK, "and optionally" should be deleted." |
| IIT Kanpur, CEWiT | We support the proposal. |
| Nokia/NSB | We don’t support removing “the additional paths” FL: The let us put it back to, hopefully it is fine for others. |
| InterDigital | We support the proposal |
| Xiaomi | We support the proposal |
| LGE | We are fine with discussing “The impact of the candidate methods on the specification” during the meeting, but it seems like adding FFS point in the TR is not appropriate. FL: Good point. We can remove the “FFS”. Also this TP is not only for determining specification impact but also useful for understanding possible multipath mitigation method even if it can be implemented without spec work. So we prefer to have erased subbullet “The identify and separate the first path and other paths in time-domain/CIR”. FL: The let us put it back to, hopefully it is fine for others. |
| CATT | We support the proposal |

FL Comments:

The proposal is revised as followings for further discussion.

(H)(Round 3) Proposal 4.3-2

Capture the following TP into TR 38.859 as a conclusion (for Section 6.3.1):

- The methods of mitigating the impact of multipath for the carrier phase positioning are studied during the study item. Based on the study, the following candidate methods of multipath mitigation are suggested for further consideration :

  - Identify and separate the first path and other paths

  - Reporting of the carrier phase of the first path, and optionally, the additional paths

  - The use of LOS/NLOS indication for the carrier phase measurements

    - Note: Rel-17 LOS/NLOS indicator can be considered as a starting point

  - The report of other channel information, such as RSRP/RSRPP

  - Other methods are not precluded

| Company | comments |
| --- | --- |
| MTK | How to identify and separate the paths are due to implementation, We prefer to remove “in the time-domain/CIR “ in the first subbullet |
| ZTE | For the carrier phase of the additional paths, we still doubt about its availability and usage. Can see whether other company has the same concern. For the sub-bullet: - The report of other channel information We are wondering what kind of channel information should be clarified, this is a little bit broad. |
| FL | To MTK: The wording of “in the time-domain/CIR “ is during discussion. I think the suggestion to remove it okay since it is proposal is to conclusion. To ZTE: It has “such as RSRP/RSRPP/SINR, CIR/CFR, etc.” before. Let us include “RSRP/RSRPP” since they are existing measurements. |
| Ericsson | If we put this in the conclusion, what does ‘suggested for further consideration’ mean? The TR should be complete and we can capture which methods we used. This doesn’t sound like a conclusion but an observation. or do you mean some of this techniques are to be considered in the work item? if it is the later, then we need further discussion. We also need a good understanding of which aspects can be handled by implementation and which ones need standardization work. |
| FL | To Ericsson: The proposal lists the techniques that are studied in the SI, and recommended to be considered in the work item. Yes, we can continue to work on the potential impact of standardization work, and decide which of them to be recommended to the normative work in the next meeting. |

FL Comments:

The proposal is revised as follows for further discussion.

### (H)(Round 4) Proposal 4.3-2

- Further study the effectiveness of the following multipath mitigation methods for the carrier phase positioning and the potential on the standard work:

  - Identify and separate the first path and other paths.

  - Reporting of the carrier phase of the first path, and optionally, the additional paths.

  - The use of LOS/NLOS indication for the carrier phase measurements.

    - Note: Rel-17 LOS/NLOS indicator can be considered as a starting point.

  - The report of other channel information, such as RSRP/RSRPP.

| Company | comments |
| --- | --- |

## (Closed)Antenna Phase Centre Offsets

Agreement (RAN1#109e) - In addition to the evaluation assumptions of NR Rel-16/17, the following error sources may also be considered during the evaluation: - Phase noise (FR2) - CFO/Doppler - Oscillator-drift - Transmitter/receiver antenna reference point location errors - Transmitter/receiver initial phase error - Phase center offset - Note: Other error sources are not precluded - Note: UE mobility can be considered in the evaluations - Note: one or more error sources can be evaluated jointly - Note: companies should provide the error sources model with their evaluations Agreement (RAN1#110) In the evaluation of NR carrier phase positioning, the following the UE/TRP antenna phase center offset (PCO) model can be considered as the starting point: dPCO = a * dPhi + w where - a is the scale factor, a=[0, 1, 3] - FFS: other values - dPhi is the direction difference (in degrees): - Example 1, dPhi is the difference between the true and the calculated (or measured) directions between a transmitter (UE/TRP) and a receiver (TRP/UE). - Example 2: dPhi is the direction difference between one UE to two TRPs, or between one TRP to two UEs. - w is 0 or a random variable uniformly distributed within [-2, +2], or [-5, +5], or [-X, +X] degrees - FFS: value of X or left up to companies - Note: the above model is valid only when absolute value of dPhi < Y degrees - FFS: value of Y or left up to companies

Submitted Proposals:

- (Nokia, R1-2208736[3]) Proposal 9: RAN1 needs to study antenna phase center offset impact on the carrier phase positioning accuracy and identify potential solutions.

- (Nokia, R1-2208736[3]) Proposal 10: RAN1 should study the impacts and potential mitigation techniques of antenna phase center offset on PRU transmissions for UL CP.

- (Samsung, R1-2209738[17]) Proposal 8: Study methods to mitigate the effect of the phase centre offset, e.g., measuring carrier phase of 3 gNBs with rays that are sufficiently close in the spatial domain at the UE.

- (Ericsson, R1-2210177[23]) Proposal 8	Study variations in phase offset due to angle of departure and angle of arrival and methods to mitigate them.

FL Comments

In RAN1#109e, it was agreed to study the impact of antenna phase center offset on the carrier phase positioning accuracy. In RAN1#110, the UE/TRP antenna phase center offset (PCO) model was agreed to the evaluation. In this meeting, multiple companies propose further study the methods for mitigating the impact of the antenna phase center offset [3][17][23]. For mitigating the impact of the antenna phase center offset, there is a need for RAN1 to discuss whether the PCO mitigation will depend on the implementation, such as the calibration of the antenna phase center, etc., or there is a need to consider the support of PCO mitigation in the specification.

### (Closed) Question 4.4-1

- Interested companies are encouraged to provide their responses on the following questions:

  - Q1: Should RAN1 further discuss the methods for mitigating the impact of antenna phase center offset in this SI, or should it be left to the implementation?

  - Q2: If you consider RAN1 further discuss the methods for mitigating the impact of antenna phase center offset in this SI, then what is your view on the following options:

    - OP1: Study the impacts and potential mitigation techniques of antenna phase center offset on PRU transmissions for UL CP” in this SI

    - OP2: Study methods to mitigate the effect of the phase centre offset, e.g., measuring carrier phase of 3 gNBs with rays that are sufficiently close in the spatial domain at the UE

    - OP3: Study variations in phase offset due to angle of departure and angle of arrival and methods to mitigate them

| Company | comments |
| --- | --- |
| Nokia/NSB | Q1: yes we should further discuss. Q2: Support OP1 and OP3. |
| Samsung | Q1: Prefer to study in this SI. Not clear if this can be resolved by implementation alone. Q2: We are open to study various options. |
| Intel | Q1: Yes Q2: Support OP1 and OP3. |
| FL | Based on the feedbacks, it seems many companies are open to study. At this moment, I am not sure if we need any agreement on them sinve only 3 companies provide the feedback. Consider that we need to focus on other high-priority issue for the remaining time of this meeting, the FL would suggest close the discussion of this topic. |

## (Closed) RTT carrier phase positioning

Submitted Proposals:

- (Fraunhofer, R1-2209534[14]) Proposal 5: Support double difference method for phase based positioning enhancements utilizing PDoA measurements for RTT measurements without synchronization of the UE and the TRP.

- (Samsung, R1-2209738[17]) Proposal 4: Study the use of round-trip carrier phase measurement to eliminate UE and gNB clock biases. The round trip carrier phase measurement is the sum of the DL carrier phase measurement at the UE and the UL carrier phase measurement at the gNB.

- (LGE, R1-2209805[19]) Proposal 4: Consider nested RTT method for the carrier phase measurement based positioning

FL Comments

For the RTT positioning method, it is assumed that the impact of UE and gNB clock biases can be removed by the use of the UE/TRP Rx-Tx time differences to obtain the round-trip time. However, the frequency offset and the Doppler may still have an impact on the accuracy of estimated RTT. In [14], the double difference method for phase based positioning enhancements is proposed to address the issue.

In [17], it suggests using the “sum of the DL carrier phase measurement at the UE and the UL carrier phase measurement at the gNB” to obtain the round-trip carrier phase measurement to eliminate UE and gNB clock biases”, under the assumption that “The carrier phases at the reference time ϕu0 and ϕb0 can be set to 0. Similarly, the carrier phases at the transmission of the OFDM symbols, ϕu2 and ϕb1 can be set to 0.” In FL’s understanding, the initial phase of the transmitter and the receiver are unknown, and may not be simply set to 0.

In [19], nested RTT method is proposed to estimate propagation delay and reduce the impact of clock impairments. In FL’s understanding, the proposal method does not actually use the carrier phase measurements, but the gNB/UE Rx-Tx time difference measurements.

### (Closed) Question 4.5-1

- Interested companies are encouraged to provide their views on the following questions:

  - Q1: what is your view on “Support double difference method for phase based positioning enhancements utilizing PDOA measurements for RTT measurements without synchronization of the UE and the TRP”?.

  - Q2: what is your view on “Study the use of round-trip carrier phase measurement to eliminate UE and gNB clock biases. The round trip carrier phase measurement is the sum of the DL carrier phase measurement at the UE and the UL carrier phase measurement at the gNB”?

  - Q3: what is your view on “Consider nested RTT method for the carrier phase measurement based positioning”?

| Company | comments |
| --- | --- |
| Nokia/NSB | Q1/Q2: Our view is that RTT type phase measurements are only feasible in the case of the same LO being used for UL/DL. RAN1 may need to check if this is feasible for realistic assumptions. We are okay to study further. Q3: We don’t consider this high priority. |
| CATT | We are okay to further study these methods. |
| Samsung | At this stage, we are open to studying these methods. |
| ZTE | Multiple Tx / Rx phase offsets are involved in RTT. It is hard to be canceled by PRU. Hence, the performance gain is questionable. |
| Huawei, HiSilicon | Q2: Due to the unknown additional phase introduced in RF at Tx and Rx, the RTT phase measurement does not corresponds to the distance. |
| Xiaomi | We are open to study it. It is possible for the UE with capability to track the phase in a short period. |
| Intel | Same views as the FL, we are not sure of the feasibility and gains from these approaches to address CPP. |
| Qualcomm | For Q2, same view as Nokia and Huawei. For Q3, not high priority, we don’t see the relevance to carrier-phase. |
| Locaila | Open |
| Philips | We are open to study |
| LGE | We prefer to keep studying RTT carrier phase positioning. For Q3, double side RTT method is considered in SL positioning item and nested RTT structure is one possible way to support this feature. For the carrier phase measurement based multi-RTT, it is important to eliminated error sources for centimeter level accuracy, and double side or nested RTT structure would be useful to eliminate clock error compare to single side RTT method. We prefer to investigate further on this topic. |
| Apple | Q1/Q2: open to study. |
| FL | Based on the feedbacks, it seems many companies are open to study. At this moment, I am not sure if we need any agreement on them, assuming the interested companies may still bring their analysis to the next meeting. |

## Additional issues related to physical layer procedures

Submitted Proposals:

- (vivo, R1-2208650[2]) Proposal 6:	The carrier phase positioning cannot be seen as a standalone positioning.

- (OPPO, R1-2208801[5]) Proposal 3: The UE can measure and report carrier phase of multiple REs in DL PRS and the UE reports the carrier phase measurement and positioning measurement results of legacy positioning method.

- (OPPO, R1-2208801[5]) Proposal 6: The TRP measures and reports carrier phase of multiple REs in SRS for positioning and the TRP also reports the carrier phase measurement and positioning measurement results of legacy positioning method.

- (CATT, R1-2208983[6]) Proposal 10: The existing physical layer procedures for NR positioning can be re-used for supporting NR carrier phase positioning.

- (CMCC, R1-2209343[11]) Proposal 2: At least the following solutions to solve the integer ambiguity are considered feasible:

  - Fast search of integer ambiguity of the carrier phase obtained from more than one frequency;

  - Fast search of integer ambiguity of the carrier phase associated with the legacy positioning techniques.

- (Samsung, R1-2209738[17]) Proposal 5: Study using legacy-based positioning measurements to estimate the number of integer cycles between the gNB and the UE.

- (InterDigital, R1-2209489[13]) Proposal 4:  Study assistance information that allows the UE or network to mitigate unknown phase offset in phase measurements, e.g., phase error group.

- (NTT DOCOMO, R1-2209909[20]) Proposal 2: Assistance data which provides helpful information to operate NR carrier phase measurement can be studied.

FL Comments

In [2], it was proposed that carrier phase positioning cannot be seen as a standalone positioning method. In [5], it is proposed that the carrier phase measurements can be reported together with the positioning measurement results of legacy positioning method, and in [6], it is proposed existing physical layer procedures for NR positioning can be re-used for supporting NR carrier phase positioning. The FL shares a similar view as these proposals. In order to resolve the integer ambiguity, the NR carrier phase positioning needs to use the measurements of the legacy methods or the location provided by the legacy methods, as proposed in [11][17]. The carrier phase measurements can be reported together with existing positioning measurements. It is expected that most of the existing physical layer procedures for NR positioning are expected to be re-used with possibly some enhancements in the measurement configuration and measurement reporting.

In [13][20], it is suggested to study the assistance information for carrier phase positioning. In FL’s view, additional assistance information, which provides helpful information to operate NR carrier phase measurement, can be further studied if needed.

(H)(Round 1) Proposal 4.6-1

Capture the following TP into TR 38.859 (for Section 6.3.1):

- For the purpose of NR carrier phase positioning, there is a need to consider the reporting of the carrier phase measurements together with the positioning measurements of the existing positioning methods.

- Existing physical layer procedures of the legacy positioning methods can be reused to support the NR carrier phase positioning.

  - Note: Some enhancements of the measurement configuration and reporting are expected.

| Company | comments |
| --- | --- |
| vivo | We propose to revise the first bullet with soft wording, for example, - For the support of NR carrier phase positioning, there is a need to support the reporting of the carrier phase measurements together with the positioning measurements of the existing positioning methods. |
| Nokia/NSB | Don’t support. First this is not true for UE-based. Second we don’t think that it is always true that legacy methods need to be reported as well though it may happen most of the time. FL: We may add UE-assisted, and list it as an option that needs to be considered |
| CATT | We are fine with the mortification suggested by vivo. |
| Samsung | We would like to study the support of: - Standalone carrier phase positioning. For this measurement, no don’t have to combine CPP with other positioning techniques - CPP with other positioning techniques. FL: Maybe we can have both options for consideration |
| ZTE | If CPP were a kind of independent positioning method, the carrier phase measurements can be performed separately. If not, it should be done with legacy positioning method(s). Hence, the following modification is achieved: For dependent NR carrier phase positioning, there is a need to support the reporting of the carrier phase measurements together with the positioning measurements of the existing positioning methods. FL: maybe one way to address the concern |
| Xiaomi | We are fine with vivo’s version since standalone can’t be precluded in this stage. |
| Intel | OK with the suggested update from vivo, but perhaps we can remove “the support of” at the beginning, “For NR carrier phase positioning, …” should suffice. |
| Ericsson | We had long discussion about using the term ‘support’ in agreements in the SI phase (e.g., SL positioning techniques agenda in last meeting). So, we suggest not to use the word support now. We prefer the following revision: - For NR carrier phase positioning, there is a need to support consider the reporting of the carrier phase measurements together with the positioning measurements of the existing positioning methods. |
| Qualcomm | Agree with Nokia about UE-based, where there will not be any measurement reports. For the UE-assisted case, we generally agree that carrier phase measurements all by themselves may not be sufficient, because they may leave too much of unresolvable integer ambiguity. What is needed in addition is some way to make this ambiguity manageable, e.g., via a coarse position estimate. Reporting legacy positioning measurements is just one way to enable this (by enabling the legacy UE-assisted positioning), but need not be the only way |
| Locaila | We don’t support on this view. This proposal is only true to the case of so called ‘multiple, virtual carrier method’ which requires complex process for integer number search. FL: The proposal here is not about the ‘multiple, virtual carrier method’. If we use the subcarrier method, we may find the initial UE position without ambiguity. The length of subcarrier of 15KHz is 20Km and 30Hz SCS is 10Km. Why it wouldn’t be sufficient to resolve initial UE position using such long subcarrier ? We showed and proved from our field experiment that use of long subcarrier is sufficient to obtain initial position information without using any existing positioning method. However, we don’t see the opposite result or analysis proving the claim of “ .. carrier phase positioning cannot be seen as a standalone positioning method … “ . We argue the companies should prove this statement. |
| LGE | We do not support the proposal. As discussed so far, reporting of the TOA or slop obtained from the phase-difference of multiple subcarriers, which is not an existing positioning method, can be a solution for resolving integer ambiguity. FL This can be another alternative. |
| Apple | Fine with Ericsson’s update. |

(H)(Round 2) Proposal 4.6-1

Capture the following TP into TR 38.859 (for Section 6.3.1):

- For the purpose of NR carrier phase positioning, both of the following can be considered:

  - the reporting of the carrier phase measurements together with the existing positioning measurements

  - the reporting of the carrier phase measurements alone without the existing positioning measurements

| Company | comments |
| --- | --- |
| Samsung | Support |
| OPPO | Support |
| MTK | 1, we don't understand the use case to report carrier phase measurement alone. How integer part could get? FL: My understanding is that from carrier phases or carrier phase difference, one may obtain the ToA. |
| Intel | While we understand the motivation behind the second option, the way it is described can be misleading – e.g., it can include reporting of carrier phase measurements only (leading to the question from MTK). Accordingly, we suggest changing both sub-bullets to say “reporting of the carrier phase based measurements …”. FL: Okay this may address the concerns of other companies. |
| vivo | We share the same understanding with MTK, and wonder how to use the carrier phase measurement alone. |
| Locaila | To MTK and vivo : If we use the subcarrier method, we may find the initial UE position without ambiguity. The length of subcarrier of 15KHz is 20Km and 30Hz SCS is 10Km. We showed and proved from our field experiment that use of long subcarrier is sufficient to obtain initial position information without using any existing positioning method. Please see R1-2203634. |
| Huawei, HiSilicon | We also do not think carrier phase only is sufficient. For locaila’s comment, this phase difference across subcarriers is equivaleng to super resolution ToA, and it deos not justify standalone carrier positioning. |
| Lenovo | Agree with Intel’s and MTK’s views and support changing sub-bullets to say “reporting of the carrier phase based measurements” for better understanding of the proposal. |
| Locaila | Let us share our experiment result showing that the subcarrier method is sufficient to use standalone. The two figures below are the actual field test results phase graph between antennas placed in left and right ends of the photo. In this experiment, we measured the phase and coordinates of the UE using subcarriers of length 259m, 65m, 32m, and 16m, respectively. We moved 50m along the three lines shown in the picture of the soccer ground. The y-axis of the phase graph above is the radian scale phase difference between -pi ~ +pi range. The x-axis is the ordered number of the measurement points. The phase graph shows the phase difference value between the two antennas calculated at each 50 measurement points. The green line in the phase graph shows the phase difference measured using subcarriers of the longest 259 m wavelength. Because the subcarrier wavelength is sufficiently long, there was no phase wrapping (i.e sudden +pi to -pi change) occurred while moving the UE from end to end. This means that subcarrier method is enough to determine the initial position of the UE without integer ambiguity when using sufficiently large wavelength. The second sky blue line is the phase difference measured using a subcarrier with a wavelength of 65m. Because the wavelength is a little short, phase wrapping occurred once at the 100th measurement point. In other words, integer ambiguity occurs once. However, since we have the initial coarse position information measured at a wavelength of 259 m, we can solve how many times the phase wrapping occurred. The third red line is the phase difference measured using a subcarrier with a wavelength of 32m. This time, phase wrapping occurred at 60 points, 210 points, and 350 points on the x side. However, the exact integer number can be determined using the location information calculated in the previous process. By repeating this method,we can determine the integer number of carrier wave of 15 cm wavelength. In conclusion, we demonstrate with this experiment that the subcarrier method can sufficiently determine the initial location and integer number without the help of other legacy methods. |
| FL | Maybe we can have the following changes: ### (H)(Round 2) Proposal 4.6-1 Capture the following TP into TR 38.859 (for Section 6.3.1): - For the purpose of NR carrier phase positioning, both of the following can be further considered: - the reporting of the carrier phase measurements together with the existing positioning measurements - the reporting of the carrier phase-based measurements alone without reporting the existing positioning measurements |
| IIT Kanpur, CEWiT | We support the updated proposal. |
| Nokia/NSB | We are okay with the proposal but are these not the only two options? Meaning what do we gain by adding this to the TR? FL: I think the conclusion is useful, since it makes clear that both options will be considered in normative work instead of only one of them. |
| InterDigital | Support the latest FL’s poroosal |
| Xiaomi 2 | We support the latest proposal from FL |
| LGE | Support the updated FL’s proposal. |
| CATT | Support the updated FL’s proposal. |

(H)(Round 3) Proposal 4.6-1

Capture the following TP into TR 38.859 as a conclusion (for Section 6.3.1):

- For the purpose of NR carrier phase positioning:

  - the reporting of the carrier phase measurements together with the existing positioning measurements can be considered.

  - the reporting of the carrier phase-based measurements alone without reporting the existing positioning measurements will be further studied.

| Company | comments |
| --- | --- |
| vivo | We are not sure we can conclude directly that both can be considered in normative work， we propose to remove “ in normative work”. |
| MTK | For 2nd subbullet, we prefer for further study |
| ZTE | Would it be better to change ‘both’ to ‘one or more’, as follows: - For the purpose of NR carrier phase positioning, both on or more of the following can be considered in normative work: - the reporting of the carrier phase measurements together with the existing positioning measurements - the reporting of the carrier phase-based measurements alone without reporting the existing positioning measurements |
| FL | To vivo’s comment: We could remove “in normative work” for now and decide what be the recommended to normative work in the next meeting. To MTK’s comment: The proposal is for conclusion. If the 2nd subbullet is for further study, then we not need to have.a new agreement on it. Will MTK be okay to keep the second bullet as considered? To ZTE: The intention is both can be consisered, but limited to one of them only. |
| Samsung | We should also add “carrier phase-based measurements”. In first sub-bullet - For the purpose of NR carrier phase positioning, both of the following can be considered in normative work: - the reporting of the carrier phase-based measurements together with the existing positioning measurements - the reporting of the carrier phase-based measurements alone without reporting the existing positioning measurements |
| Huawei, HiSilicon | We cannot agree to the second subbullet. |
| FL | To Samsung: Okay. Change is made as suggested. To Huawei/MTK: - For the purpose of NR carrier phase positioning: - the reporting of the carrier phase measurements together with the existing positioning measurements can be considered. - the reporting of the carrier phase-based measurements alone without reporting the existing positioning measurements will be further studied. |
| Ericsson | This may be a bit early to conclude now. Let’s just discuss the conclusion in next meeting. |
| Intel | Same view as Ericsson – especially we should not have phrases like “will be further studied” or “can be considered” (for further study?) in the TR. |

FL Comments:

Since some companies prefer to wait for the next meeting to make the conclusion. Then my suggestion is to continue the study of the two appraoches.

### (H)(Round 4) Proposal 4.6-1

- Further study the following approaches for NR carrier phase positioning, and identify the potential impact on the standard.

  - the reporting of the carrier phase measurements together with the existing positioning measurements.

  - the reporting of the carrier phase-based measurements alone without reporting the existing positioning measurements.

| Company | comments |
| --- | --- |

(Closed) Proposal 4.6-2

- Additional assistance information, which provides helpful information to operate NR carrier phase measurement, can be further studied.

| Company | comments |
| --- | --- |
| Nokia/NSB | Okay but seems like a vague proposal. |
| CATT | Share the similar view as Nokia. It would be better to discuss specifically what the assistance information are. |
| ZTE | LMF can configure the integer ambiguity to UE, this can be added to the addition assistance information. Hence, the following modification is achieved: - Additional assistance information, which provides helpful information to operate NR carrier phase measurement, can be further studied. The additional assistance information may include: Integer ambiguity between the TRP and target UE. |
| Intel | Same view as Nokia and CATT. |
| Qualcomm | Same view as Nokia, CATT, Intel |
| LGE | Same view with other companies. |

FL Comments

Based on the feedbacks, tt seems it is too early to discuss additional assistance information. Thus, suggest closing the discussion of thius proposal in this meeting, so we can focus on other issues.

# Analysis and Evaluation Results

## Evaluation Assumptions

Agreement NR carrier phase positioning performance will be evaluated at least with the carrier phase measurements of a single measurement instance. Agreement - Reuse the simulation assumptions of NR Rel-16/17 for carrier phase positioning - Note: Optional modification of the simulation assumptions defined in NR Rel-16/17 are allowed only if needed. - The evaluation scenarios: - Baseline: InF-SH, InF-DH - Optional: IOO, Umi, Highway - Note 1: Other evaluation scenarios are not precluded. - Note 2: Existing Rel-17 DL/UL reference signals in Uu interface is to be used for the Highway scenario. - Frequency range: - Baseline: FR1 - Optional: FR2 Agreement - In addition to the evaluation assumptions of NR Rel-16/17, the following error sources may also be considered during the evaluation: - Phase noise (FR2) - CFO/Doppler - Oscillator-drift - Transmitter/receiver antenna reference point location errors - Transmitter/receiver initial phase error - Phase center offset - Note: Other error sources are not precluded - Note: UE mobility can be considered in the evaluations - Note: one or more error sources can be evaluated jointly - Note: companies should provide the error sources model with their evaluations Agreement (RAN1#109e) The use of PRUs to facilitate NR carrier phase positioning can be evaluated in the SI by RAN1. Agreement (RAN1#110) In the evaluation of NR carrier phase positioning, the following frequency errors can be considered, which are modeled independently for each UE and each TRP: - Initial Residual CFO (is the same for one measurement instances [or multiple phase measurement instances]): - Ideal: 0 (UE/TRP) - Practical: uniform distribution within - [-30, +30] Hz (FR1, UE), [-100, +100] Hz (FR1, UE), - [-120, +120] Hz (FR2, UE), [-400, +400] Hz (FR2, UE), - [-10, +10] Hz (for each TRP, FR1), - [-40, +40] Hz (for each TRP, FR2). - Oscillator-drift (is the same for one or multiple phase measurement instances for positioning fix): - Ideal: 0 (UE/TRP) - Practical: uniform distribution within [-0.1, 0.1] ppm (UE), [-0.02, +0.02] ppm (each TRP) within measurement duration - Note: The Doppler frequency can be determined based on the UE speed in the evaluation assumption. Agreement (RAN1#110) In the evaluation of NR carrier phase positioning, the offset between the initial phase of the transmitter and the initial phase of the receiver can be modeled as a random variable uniformly distributed within [0, X]. - Possible values of X: 2pi - Other values FFS Agreement (RAN1#110) In the evaluation of NR carrier phase positioning, the antenna reference point (ARP) location error of a TRP can be modeled as follows: - Ideal: no ARP error - Practical: a zero-mean, truncated Gaussian distribution with zero mean and standard deviation of T=[1, 5] cm truncated to 2T in each of (x, y, z) direction Agreement (RAN1#110) In the evaluation of NR carrier phase positioning, the following the UE/TRP antenna phase center offset (PCO) model can be considered as the starting point: dPCO = a * dPhi + w where - a is the scale factor, a=[0, 1, 3] - FFS: other values - dPhi is the direction difference (in degrees): - Example 1, dPhi is the difference between the true and the calculated (or measured) directions between a transmitter (UE/TRP) and a receiver (TRP/UE). - Example 2: dPhi is the direction difference between one UE to two TRPs, or between one TRP to two UEs. - w is 0 or a random variable uniformly distributed within [-2, +2], or [-5, +5], or [-X, +X] degrees - FFS: value of X or left up to companies - Note: the above model is valid only when absolute value of dPhi < Y degrees - FFS: value of Y or left up to companies Agreement (RAN1#110) For the evaluation of NR carrier phase positioning, UE position can be calculated by the use of the carrier phase measurements obtained at the M sequential time instances, where - Baseline: - M=1 - Optional : - M=4 - Other values of M - Companies should report their assumptions on UE mobility (e.g. speed)

Submitted Proposals:

- (Nokia, R1-2208736[3]) Proposal 1: No new values of X are added for the initial phase offset model between transmitter and receiver.

- (Nokia, R1-2208736[3]) Proposal 2: Adopt the PCO error model as the final error model on PCO for the SI.

- (Nokia, R1-2208736[3]) Proposal 3: For parameters X and Y leave it up to individual companies to provide the specific values used in their evaluations.

- (China Telecom, R1-2208774[4]) Proposal 3: apart from the initial phase offset and the CFO, the carrier phase measurement model for positioning should mainly consider the time offset between the transceiver.

- (CATT, R1-2208983[6]) Proposal 5: For evaluation of NR CPP in Rel-18 SI, initial random phases of TRPs/UEs should be considered to be TRP/UE-specific, e.g., it is the same for all carrier phase measurements of a carrier frequency for a specific TRP/UE.

- (CATT, R1-2208983[6]) Proposal 6: The impact of TRP ARP location error should be mitigated in order to improve the carrier phase positioning accuracy.

- (Intel, R1-2209059[7]) Proposal 2:	Clarify that the initial phase offsets can be modelled separately at transmitter and receiver nodes as independent random variables, each uniformly distributed within [0, 2pi].

- (Locaila, R1-2209543[15]) Proposal 1) Change the CFO metric in Table A.3-1 of current TR38.859 as below table.

Table A.3-1: Assumptions for evaluation of NR carrier phase positioning

| Assumptions | Value |  |
| --- | --- | --- |
| Scenarios | - Baseline: InF-SH, InF-DH - Optional: Indoor Open Office, Umi, Highway scenarios - Note 1: Other evaluation scenarios are not precluded - Note 2: Existing Rel-17 DL/UL reference signals for the Uu interface are to be used for the Highway scenario. |  |
| Frequency errors – Note 1 | Ideal | Practical |
| Initial residual CFO (is the same for one measurement instances [or multiple phase measurement instances]) | 0 (UE/TRP) | Uniform distribution within: • [-30, +30] Hz (FR1, UE), [-100, +100] Hz (FR1, UE), • [-120, +120] Hz (FR2, UE), [-400, +400] Hz (FR2, UE), • [-10, +10] Hz (for each TRP, FR1), • [-40, +40] Hz (for each TRP, FR2). • [-100, +100] Hz (FR1, UE, 1GHz) • [-400, +400] Hz (FR1, UE, 4GHz) • [-3K, +3K] Hz (FR2, UE, 30GHz) • [-7K, +7K] Hz (FR2, UE, 70GHz) • [-20, +20] Hz (FR1, TRP, 1GHz) • [-80, +80] Hz (FR1, TRP, 4GHz) • [-600, +600] Hz (FR2, TRP, 30GHz) • [-1.4K, +1.4K] Hz (FR2, TRP, 70GHz) |
| Oscillator-drift (is the same for one or multiple phase measurement instances for positioning fix) | 0 (UE/TRP) | Uniform distribution within: • [-0.1, 0.1] ppm (UE) • [-0.02, +0.02] ppm (each TRP) within measurement duration |

- (Locaila, R1-2209543[15]) Proposal 2) RAN1 should study the traffic burden related to oscillator accuracy when employing the proposed double difference method.

- Qualcomm, R1-2209992[21]) Proposal 1: Clarify the definition of measurement duration for oscillator drift modeling.

- (Ericsson, R1-2210177[23]) Proposal 5	Study if a 2D positioning assumption is realistic when the target accuracy level is in the order of centimetres/ decimetre.

- (Ericsson, R1-2210177[23]) Proposal 7	When reporting results for carrier phase-based positioning accuracy, if PRUs are used for double-differentiation then the PRU deployment should be reported.

FL Comments

It is proposed in [3][7] that the value of the initial phase offset is within [0, 2pi] with no other value of X. In [4] and [7], it is proposed the initial random phases of TRPs/UEs should be considered to be TRP/UE-specific. These proposals seem reasonable in FL’s view.

In [5], it is proposed to consider the time offset between the transceivers. Consider that the time offset is actually included in the template for the collection of the evaluation results, and thus, we may need to have a separate agreement on that.

In [15], it is proposed to change carrier frequency offset (CFO) range to match (oscillator-drift rate* the carrier frequency in 1s duration). In FL’s view, the change is not needed due to: 1) In the evaluation assumption, both CFO and Oscillator-drift are considered, which can be modelled independently and added together; and 2) the maximum CFO is up to the adjust rate of UE for frequency error; and c) the carrier phase measurements are normally estimated much shorter than 1s.

In [15], it is also proposed to study the traffic burden related to oscillator accuracy when employing the proposed double difference method. In FL’s view, the increase of the traffic loading for reporting the carrier phase measurements is not expected to be a significant issue in an NR network even if the PRreportsng the carrier phase measurements at the same rate as DL PRS periodicity. The measurements reported from a PRU can actually be used for supporting all UEs in the surrounding area of the PRU.

In [21], it is proposed to clarify the definition of measurement duration for oscillator drift modelling. In the agreement, it says “Oscillator-drift (is the same for one or multiple phase measurement instances for positioning fix”. To FL’s understanding, the measurement duration is the time duration for estimating the carrier measurements. For one measurement instance, the duration is the time duration for obtaining the measurement instance. If multiple measurement instances are used for a position fix, then the oscillator-drift keeps the same for all of the multiple phase measurement instances for the positioning fix.

In [23], it is proposed to study “if a 2D positioning assumption is realistic when the target accuracy level” for carrier phase positioning. In FL’s understanding, 3D measurements need to be used in the evaluation because of the height difference between the UE and gNB in the evaluation assumption. As for the evaluation result, the horizontal and vertical positioning accuracy may be presented separately, given that many applications may have different requirements for horizontal and vertical positioning accuracy.

In [23], it is also proposed to include the assumption of PRU deployment in the evaluation. In FL’s view, the PRU deployment is indeed helpful for understanding the evaluation results and thus it is suggested to add a row in the template table for collecting the information.

### (Closed) Question 5.1-1

Make the following modification to the previous agreement on the initial phase model:

- In the evaluation of NR carrier phase positioning, the offset between both the initial phase of the transmitter and the initial phase of the receiver can be modeled as a independent random variables uniformly distributed within [0, X2pi].

- Note: The initial phase of a transmitter applies to all carrier phase measurements of the same carrier frequency associated with the transmitter, and the initial phase of a receiver applies to all carrier phase measurements of the same carrier frequency associated with the receiver.

- Possible values of X: 2pi

  - Other values FFS

| Company | comments |
| --- | --- |
| Nokia/NSB | Support. |
| CATT | Support |
| Samsung | We don’t see the necessity of this proposal. |
| ZTE | Support. |
| Xiaomi | Support |
| Intel | Support. |
| Ericsson | Ok |
| Qualcomm | Support |
| Locaila | The proposal itself is ok, but… we show some disagreement on FL’s comment above. In 1) above, FL said, in the evaluation assumption, both CFO and Oscillator-drift are considered, which can be modelled independently and added together … -> nope. in the case of TRP, CFO and Oscillator-drift are tightly coupled as below equation. 1 CFO Hz=carrier_frequency×ppm ×10-6 The above equation means that due to oscillator instability, a constant % of the carrier phase is outside the true value. In general, RF carrier waves are produced by multiplying the oscillator wave by a constant, so the above relationship is established at the RF output stage. However, in the case of a moving UE, CFO may increase or decrease further under the influence of Doppler, but this is only for the case of the UE. It is important to model the initial phase offset between TRPs and evaluate the performance of the solution that you want to calibrate. We discussed the inefficiency of the double difference method due to the instability of the oscillator in [15]. If the CFO value of TRP is artificially and unrealistically lowered as above, this will make the evaluation results of the proposed double difference solution look better, which may lead to false conclusions. In 3) above, FL said the carrier phase measurements are normally estimated much shorter than 1s. … -> ppm/ppb means the % rate that occurs per second. This does not mean that the carrier phase should be measured evenly for 1 second. |
| LGE | Support |
| Apple | OK |
| FL | The following agreement was made during online discussion. Agreement Make the following modification to the previous agreement on the initial phase model with an additional note: - In the evaluation of NR carrier phase positioning, the offset between both the initial phase of the transmitter and the initial phase of the receiver can be modeled as a independent random variables uniformly distributed within [0, X2pi]. - Note: The initial phase of a transmitter applies to all subcarriers of the same carrier frequency associated with the transmitter, and the initial phase of a receiver applies to all subcarriers of the same carrier frequency associated with the receiver. |

FL Comments:

The note in above agreement covers the case for all subcarriers of the same carries. It does not cover the case for different carriers. For supporting the evaluation of the carrier phases from multiple carriers (positioning frequency layers), the FL suggest adding the following note to the above agreement.

Note: For the frequency errors, I assume there is no need to have new agreement. UE has only one oscillator and the frequency errors should be the same for different carriers. The existing agreement has defined per UE (also per TRP) for the frequency errors.

(H)(Round 2) Proposal 5.1-1a

●    Add the following note to the previous agreement on error modelling of the initial phase:

○    Note: The initial phases of a transmitter of different carriers can be assumed to be independent with each other. Similarly, the initial phases of a receiver can be assumed to be independent with each other.

| Company | comments |
| --- | --- |
| mtk | “. Similarly, the initial phases of a receiver can be assumed to be independent with each other. ” “a receiver” should be changed as “different receivers”? |
| Intel | OK, suggest updating the text as “of a transmitter for different carriers” and “of a receiver for different carriers”. |
| Huawei, HiSilicon | Intel’s version is fine. |
| FL | To: MTK/Intel/HW: Yes. The proposal can be changed as suggested for further discussion. ## (H)(Round 2) Proposal 5.1-1a ● Add the following note to the previous agreement on error modelling of the initial phase: ○ Note: The initial phases of a transmitter for different carriers can be assumed to be independent with each other. Similarly, the initial phases of a receiver for different carriers can be assumed to be independent with each other. |
| LGE | We are fine with the proposal |
| CATT | Support |

FL Comments:

The proposal is revised based on the comments as followings for further discussion.

### (H)(Round 3) Proposal 5.1-1a

●    Add the following note to the previous agreement on error modelling of the initial phase:

○    Note: The initial phases of a transmitter for different carriers can be assumed to be independent of each other. Similarly, the initial phases of a receiver for different carriers can be assumed to be independent of each other.

| Company | comments |
| --- | --- |
| Samsung | Small change, I think it is better to say “independent of” instead of “independent with” |
| FL | To Samsung: Okay |
| Qualcomm | Samsung proposed “independent of” (which we agree with), FL agreed, but current text says “independent with”. Possible double-correction when FL copied Samsung comments over from the earlier section to Section7 ? |
| FL | To Samsung/Qualcomm: Yes. My mistake. Corrected back to “independent of”. |

(H)(Round 1) Proposal 5.1-2

- Add a row of "PRU deployment" in Table B.4.X.1-1: NR carrier phase positioning enhancements – evaluation scenarios and parameters” in Draft TR 38.859.

| Company | comments |
| --- | --- |
| vivo | Okay |
| Nokia/NSB | Support |
| CATT | Support |
| Samsung | OK, but proponents need to report the details of PRU, i.e., what is the PRU (UE or TRP) (under what condition, e.g., perfect location information, perfect measurement, perfect reporting etc.); FL: Not sure the purpose to say “what is the PRU (UE or TRP)” and “perfect reporting” etc. For the location accuracy, I assume the companies may have the assumptions as TRP/ARP. |
| Xiaomi | Support |
| Intel | OK. Minor point, we can delete “Draft” for the TR reference. |
| Ericsson | Ok |
| Qualcomm | Support |
| Apple | OK |

FL Comments:

(H)(Round 1) Proposal 5.1-2 was discussed in online session, and there were comments to have more details on the “PRU deployment”.

(H)(Round 2) Proposal 5.1-2

- Add a row of "PRU assumptions" in Table B.4.X.1-1: NR carrier phase positioning enhancements – evaluation scenarios and parameters” in Draft TR 38.859.

  - Note: PRU deployment information may include the assumptions of the number of PRUs, the PRU locations and location errors etc.

| Company | comments |
| --- | --- |
| Nokia/NSB | Okay with the proposal but would also be fine with calling it “PRU assumptions”. FL: Okay, I would take the suggestion. |
| Samsung | OK |
| Huawei, HiSilicon | Then the Note should read - Note: PRU assumptions may include the assumptions of the number of PRUs, the PRU locations and location errors etc. FL: Yes.. The note can be changed as suggested. |
| CATT | Support |

### (H)(Round 3) Proposal 5.1-2

- Add a row of "PRU assumptions" in Table B.4.X.1-1: NR carrier phase positioning enhancements – evaluation scenarios and parameters” in Draft TR 38.859.

  - Note: PRU deployment assumptions may include the assumptions of the number of PRUs, the PRU locations and location errors etc.

| Company | comments |
| --- | --- |
| ZTE | PRU deployment information may include other features, such as PRU density, deployment scope, etc. |
| FL | To ZTE: I assume from the “PRU locations and the PRU locations” has, and in more details, the information related to the “PRU density, deployment scope, etc.”. |
| Nokia/NSB | We assume the note should be updated to say “PRU assumptions may include the number of PRUs, the PRU locations and location errors etc.” |
| FL | To Nokia: Yes, I made the correction as suggested. |

## Evaluation Results

Agreement (RAN1#110) Endorse the templates in section 17 under (H)(Round 1) Proposal 17-1 in R1-2207690 to collect carrier-phase based positioning simulation results, with the following notes: - The TR editor can adjust the sections/sub-sections arrangement - Adjust the titles of the tables to refer to NR carrier-phase based positioning - The detailed rows of the tables can be further discussed

Submitted Proposals:

- (Huawei, R1-2208455[1]) Proposal 1:  Rel-18 shall target the accuracy requirement of 1cm@50% for positioning using carrier phase measurement.

- (Nokia, R1-2208736[3]) Proposal 4: Include the above results in the TR.

- (Nokia, R1-2208736[3]) Proposal 5: RAN1 to conclude that NR CP provides significant accuracy gains over existing techniques.

- (Lenovo, R1-2209395[12]) Proposal 1: RAN1 to study and define the performance requirements of carrier phase positioning in terms of horizontal and vertical accuracy requirements based on the FR1/FR2 carrier wavelengths as well as distinguishing the accuracy according to different environments, e.g., indoor vs outdoor.

- (IIT Kanpur, R1-2209546[16]) Proposal 4: For the purpose of evaluation, the following target accuracy requirement is considered for NR carrier phase positioning:

  - Horizontal position accuracy (<10 cm for 80%) of UEs

- (Ericsson, R1-2210177[23]) Proposal 3	We propose that the TR captures the result of Observation 5, repeated:  Reporting the carrier phase of individual subcarriers does not add any information compared to reporting the center carrier phase and the estimate time-of-arrival of the reference signal.

- (Ericsson, R1-2210177[23]) Proposal 4	Capture the conclusion of Observation 8 in the technical report: For successful Multi-Hypothesis IAR, a redundancy in number of TRPs is needed, especially when the accuracy of each carrier phase measurements is reduced.

Many companies have provided the evaluation results based on the simulation assumptions agreed in the previous meetings, and also the observations from the analysis and/or the simulation.

For the simulation results, it was agreed in RAN1#110 to follow the practice of Rel-16/Rel-17 to use the templates in Section 17 under (H)(Round 1) Proposal 17-1 in R1-2207690 to collect the carrier phase positioning simulation results.

In addition, we may also need to provide a summary of the main findings of the study into TR 38.859, especially the issues that RAN1 has agreed to investigate. In the following, The FL tries to summarize the observations provided in the contributions [1-23], focusing on the error sources that were agreed to be evaluated and also the techniques of the carrier phase positioning that many companies are interested in. It is important to point out that the following summary does not include all of the observations from all of the companies, especially the issues that only commented by a single company. It does not mean an issue brought up only by a single company is not important. It is only because due to the limited meeting time, we may only be able to focus the discuss on the issues that more companies provide the observations/proposals. Thus, if a proposal/observation from a company attracts more interest from other companies, we will consider raising the priority of that proposal/observation. Also, it is FL’s understating that individual companies may include their observations together with their evaluation results in TR 38.859. In this way,  we will not miss any important findings obtained during the study.

Summary of the observations:

- About the overall carrier phase positioning performance:

  - Many companies have provided the evaluation results related to the NR carrier phase positioning (e.g., [1-3][5-7][9][14-15][17][19][21-23]). Many companies have observed significant improvement of the accuracy over Rel-17 is possible with NR carrier phase measurements, where centimeter-level positioning accuracy can be reached at least for some of the evaluation scenarios, e.g., LOS dominant environment, when the integer ambiguity is resolved correctly. It is also observed that the accuracy improvement can be quite different for different scenarios and the evaluation results from different companies show different levels of improvement. In FL’s understanding, this is expected because the performance of the carrier phase positioning techniques depends on many factors, as shown in the observations provided by the companies.

- About the estimation of the carrier phase:

  - Multiple companies have observed that different approaches can be used to obtain carrier phase measurements. However, there can be a significant difference on the accuracy of the carrier phase measurements due to the impact of multipath/NLOS (e.g., [2][6][11]).

- About the impact of initial phase error on carrier phase positioning:

  - It may significantly degrade the positioning performance if it is not eliminated (e.g., [2][4][7]). Many companies (e.g., [1][3][4][6][7][9][12]) have observed that the impact of initial phase error can be removed by the double differential technique.

- About the impact of CFO:

  - Some companies have observed that the impact of CFO on carrier phase positioning is negligible (e.g., [1][6][9][12]), and the evaluation results from some other companies show the CFO has a certain impact on carrier phase positioning (e.g., [2]).

- About double differential carrier phase positioning with PRU:

  - Many companies have observed that the double differential technique is an effective approach for eliminating the errors related to the transmitter and the receiver in the carrier phase measurements (e.g., [1][3][4][6][7][9][16][17][22]), which include the initial phase, oscillator-drifts, clock errors, etc. of the transmitter and the receiver.

- About ARP error:

  - Multiple companies have observed that ARP errors may deteriorate the carrier phase positioning accuracy (e.g., [1][2][6][9][17]) when the carrier phase measurements from a single frequency are used due to its impact on solution of the integer ambiguity.

- About the impact of PCO:

  - Multiple companies have observed that PCO will have the impact on the carrier phase positioning (e.g., [1][2][3][6]).

- About the integer ambiguity:

  - It is a common observation that properly resolution of the integer ambiguity is one of the main key issues for the success of carrier phase positioning (e.g., [1][2][3][4][5][7][8][17][19][20][21][22]).

- About the use of multi-frequency carrier phase measurements:

  - Many companies have observed that the use of multi-frequency carrier phase measurements is helpful for the resolution of integer ambiguity (e.g., [1][3][5][6][9][17][19][22]). In addition, multiple companies have observed the use of multi-frequency carrier phase measurements can also reduce impact of ARP position errors (e.g., [1][6][9])

- About the impact of multipath/NLOS:

  - It seems to be a common view that multipath/NLOS may impact significantly the performance of carrier phase positioning, and thus it is important to identify the first path properly to reduce the measurement error, and consider potential solutions to mitigate the impact of the multipath/NLOS.

### (Closed) Proposal 5.2-1

Capture the following TP into TR 38.859 as an evaluation observation (for Section 6.3.2):

- Significantly improvement of the accuracy over Rel-17 positioning methods is possible with NR carrier phase measurements., It is observed that centimeter-level positioning accuracy can be reached at least for some of the evaluation scenarios.

| Company | comments |
| --- | --- |
| vivo | Firstly, we prefer to add ‘without modeling error’ as a condition. In addition, maybe ‘some of the evaluation scenarios’ can be expressed more clearly(e.g, InF-SH) |
| Nokia/NSB | Support. We don’t agree with vivo’s suggestion as many of the modeled errors are clearly negligible with PRU. |
| CATT | Support. Share the similar view as Nokia. Our evaluation has included the errors, and also not limited to InF-SH scenarios. |
| ZTE | Support, we prefer clarify the specific evaluation scenarios and error models that can satisfy the centimeter-level position accuracy. |
| Huawei, HiSilicon | We need to describe how many UEs can achieve the centimeter-level positioning accuracy, such as ‘It is observed that centimeter-level positioning accuracy for 50% UEs can be reached at least for some of the evaluation scenarios’ We have another question to the FL: Do we need to set the requirement for carrier phase positioning in this release? |
| CMCC | Should we provide more details in this TP? A similar layout in SL positioning can be referred to, for example: - Evaluation results are provided by x sources out of y sources; - Significantly improvement of the accuracy over Rel-17 positioning methods is possible with NR carrier phase measurements; - From the evaluation, * A source observes that centimeter-level positioning accuracy can be reached |
| Ericsson | We don’t agree with this observation. Many companies evaluate the InF-SH scenario where already TDOA can provide centimetre level accuracy. Additionally, even with extremely optimistic assumptions on e.g. ARP error increases the positioning error significantly. The following are some observations we found in the referenced TDocs: - In [1], the following observations can be found: - Observation 3: The ARP error deteriorates the carrier phase positioning accuracy even the double differential method (with PRU) is used. - Observation 4: Double differential carrier phase positioning technique can eliminate some influence of PCO. Note that Results presented in Table 6 of [1] which take into account a combination of multiple error factors shows that target requirements are not met for most of the cases simulated even with PRUs. - In [17], the following observation can be found: - Observation 8: With larger phase error and ARP error, the performance is degraded severely. - In [3], the results presented are preliminary as mentioned in [3]. The impact of error sources and in particular the impact due to a combination of error sources does not seem to be taken into account in the results of [3]. - In [19], the results reported seem to be under ideal assumptions where error sources nor combination of error sources is not taken into account (see Appendix A which contains Sim assumptions for results presented in Section 3). - In [6], the following observation/proposal can be found: - Observation 13: The carrier phase positioning accuracy will degrade when ARP error increases. - Proposal 6: The impact of TRP ARP location error should be mitigated in order to improve the carrier phase positioning accuracy. - In [15], the following observation is made: - (Observation 2) Considering the accuracy of commercially available oscillators and the size of phase offset information necessary to broadcast, the proposed double difference method may cause heavy traffic burden to 5G NR system. - In [10, 11, 12, 13, 16], there are no evaluation results. These TDocs mostly include study proposals. Based on these observations, it is premature to agree this proposal. Further study is needed to access performance of NR carrier phase positioning in the presence of error sources, and particularly in the presence of a combination of error sources. |
| Qualcomm | Prefer to hold off on writing this. Our evaluations in FR1 with all errors included do not support this conclusion. Further, note that centimeter level accuracy is attained at certain percentiles even with Rel-16/17 approaches; so this alone is insufficient to justify ‘gains over Rel-16/17’. |
| Locaila | We generally agree, but it is premature conclusion at this stage. Further study is necessary. |
| LGE | Support |
| Apple | Update to “significantly” |
| OPPO | We also prefer to hold off to making this observation. Our evaluation results show that adding on carrier phase measurement on top of Rel-16/17 DL-TDOA does not provide better performance than Rel-16/17 DL-TDOA method due to the residual errors in integer ambiguity. |
| FL | Based on the comments received, we could first focus on the observation related to the error sources and the techniques, and wait for the next meeting to make the conclusion. Thus, suggest closing the discussion in this meeting. |

(H)(Round 1) Proposal 5.2-2

Capture the following TP into TR 38.859 as an evaluation observation (for Section 6.3.2):

- The double differential technique with PRU is an effective technique to eliminate the measurement errors related to the transmitter and the receiver in carrier phase measurements, which include the initial phase, the oscillator drifts, and the clock errors of the transmitter and the receiver.

| Company | comments |
| --- | --- |
| Nokia/NSB | Support. |
| CATT | Support. |
| Samsung | This could be an observation. - The double differential technique with PRU is an effective technique beneficial to eliminate the measurement errors related to the transmitter and the receiver in carrier phase measurements, which include the initial phase, the oscillator drifts, and the clock errors of the transmitter and the receiver. |
| ZTE | We propose modify the description as follows: - The double differential technique with PRU is an effective technique to eliminate the measurement errors related to the transmitter and the receiver in carrier phase measurements, which include the initial phase, the oscillator drifts, and the clock errors of the transmitter and the receiver. because in our simulation, the oscillator drifts cannot be eliminated by using double differential technique. |
| CMCC | Similar comments as Proposal 5.2-1. We are basically fine and recommend to have more details if possible. |
| Intel | Support. Perhaps instead of saying “oscillator drifts”, we can just say “frequency offsets” to combine both residual CFO and applicable oscillator drift. |
| Ericsson | Not agree. How dense must PRUs be deployed for the technique to be effective? Before we can agree to a such general statement we need first to know that. A PRU which is 100 m away from the UE (but still inside the building) may experience a very different radio environment and multipath conditions that makes is useless for the UE. |
| Qualcomm | Support. |
| Locaila | No, we don’t agree at all. |
| LGE | Support |

FL Comments:

Based on the comment, it would be better to have more details based on the evaluation observations in the TP based on the evaluation results from the companies.  The proposal is revised as follows:

(H)(Round 2) Proposal 5.2-2

Capture the following TP into TR 38.859 as an evaluation observation (for Section 6.3.2):

The impact of the carrier phase measurement errors related to the initial phase and the frequency errors (the CFO and the oscillator drifts) of the transmitter and the receiver are evaluated during the study item.

- The evaluation results from the companies (e.g., Huawei[1], vivo[2], CATT[6], ZTE[9]) should that these errors may significantly degrades the positioning performance. It is impossible to support centimeter-accuracy positioning if these errors are  not removed.

- The evaluation results from the companies (e.g., Huawei[1], CATT[6], ZTE[9]) also should that these errors can be removed effectively by the double differential technique with PRU.

- Thus, it can be concluded that the double differential technique with PRU is an effective technique to eliminate the carrier phase measurement errors that are related to both the transmitter and the receiver.

| Company | comments |
| --- | --- |
| Locaila | No, we don’t agree on the conclusion -> “Thus, … the double differential technique with PRU is an effective technique…” See our arguments we proposed above. There’s still negative aspects of PRU to study. It is not a proven technique except GNSS. Of course 5G NR is not GNSS. This is biased, misleading conclusion of RAN1 study. |
| LGE | 1. We are generally fine with the proposal but it seems like “should” should be changed to “shows”. 2. I am not sure all kinds of “the carrier phase measurement errors that are related to both the transmitter and the receiver” can be eliminated by the double differential technique. It seems like previous wording of the 1st round discussion is better. FL: As a compromise, we may include the last proposal in the observations from the evaluation results. |
| CATT | Support |
| Nokia/NSB | Okay |

FL Comments:

For simplicity, let us only capture the evaluation obaservation in this proposal.

(H)(Round 3) Proposal 5.2-2

Capture the following TP into TR 38.859 as an evaluation observation (for Section 6.3.2):

The impact of the carrier phase measurement errors related to the initial phase and the frequency errors (the CFO and the oscillator drifts) of the transmitter and the receiver are evaluated during the study item.

- The evaluation results from the companies (e.g., Huawei[1], vivo[2], CATT[6], ZTE[9]) show that these errors may significantly degrade the positioning performance. It is impossible to support centimeter-accuracy positioning if these errors are not removed.

- The evaluation results from the companies (e.g., Huawei[1], CATT[6], ZTE[9]) show that the initial phases of the transmitter and the receiver are removed effectively by the double differential technique with the use of PRU.

- The evaluation results from the companies (e.g., Huawei[1], CATT[6]) show that the frequency errors (the CFO and the oscillator drifts) of the transmitter and the receiver are removed effectively by the double differential technique with the use of PRU.

(Not captured in TR) Note: The number of sources and the references can be further updated in next meeting depending on companies’ updates of simulation results.

| Company | comments |
| --- | --- |
| vivo | For the first sub-bullet, ‘’should that’ may need to be changed to “show that” |
| ZTE | One thing we have to clarify is that the CFO wasn’t eliminated in our simulation, because the positioning error brought by frequency error is negligible as depicted in our evaluation result. Therefore, we suggest revise the proposal as follows: The impact of the carrier phase measurement errors related to the initial phase and the frequency errors (the CFO and the oscillator drifts) of the transmitter and the receiver are evaluated during the study item. - The evaluation results from the companies (e.g., Huawei[1], vivo[2], CATT[6], ZTE[9]) should show that these errors may significantly degrade the positioning performance. It is impossible to support centimeter-accuracy positioning if these errors are not removed. - The evaluation results from the companies (e.g., Huawei[1], CATT[6], ZTE[9]) show that the initial phase and the frequency errors (the CFO and the oscillator drifts) offsets of the transmitter and the receiver are removed effectively by the double differential technique with the use of PRU. - The evaluation results from the companies (e.g., Huawei[1], CATT[6]) show that the frequency errors (the CFO and the oscillator drifts) of the transmitter and the receiver are removed effectively by the double differential technique with the use of PRU. |
| FL | To vivo: Yes. It is corrected. To ZTE: The changes are fine to me. |
| Samsung | Don’t support At this time it is premature to make this conclusion. We should carefuly analyze the simulation results and underlying assumption (e.g, positioning accuracy, deployment density of PRUs, etc) before making such a conclusion. |
| FL | To Samsung: This is not a conclusion, but an observation based on companies’ evaluation. |
| Samsung | To make an observation all of the relavnt under laying assumptions have to be mentioned. For example, what are the assumptions on the location accuracy of the PRUs, density of PRUs etc. Let’s continue the discussion on this one. FL: We may need to first make the observation for each error source separately and then we can check the overall impact. Mixing every together may not help to understand the impact of each error source. By the way, PRU errors have any impact on the removing the initial phase, since the elimination of initial phase takes place during the operation of the double differencial, and the impact of PRU error takes place with the PRU location is used to calculate the UE location. |
| Ericsson | We agree with Samsung. We need to properly metion all the assumptions under which these results are observed. Also, can we be a bit quantitative? Let’s continue the discussion on this next week. FL: Okay. We could define first the conditions under which the impact of each error source is analyzed. |
| Intel | Agree with Samsung and Ericsson that further discussions are needed. We also need to better align our understanding of the impact from each error source instead of bundling phase and frequency errors together in the context of this proposal. FL: Okay. We can have separate proposals for initial phase and frequency errors. |

FL Comments:

Based on the discussion of  propoals in this section, it might be better for us to have a consistent way for the companies to provide their observations after the evaluation. Otherwise, it would be chanllenging for the FL to summarize the evaluation results for the  observations/comnclusions.

The TP for TR 38.859 to capture the evaluation results of NR carrier phase positioning may look like the following in my mind:

Impact of ARP errors on NR carrier phase positioning The impact of ARP errors on NR carrier phase positioning is evaluated by the sources ([X1], [X2],…) for ([InF-SH], [Inf-DH]) scenario with the following evaluation results: - The sources ([X1], [X2],…) evaluates that impact of ARP errors of [X] cm. - The evaluation results from the sources ([X1],,…) show the positioning accuracy of NR CPP is X m (50%) and Y m (80%) , while the positioning accuracy of DL- TDOA (or UL-TDOA) is X m (50%) and Y m (80%). - … - The sources ([X1], [X2],…) evaluates that impact of ARP errors of [X] cm with double differential techqniue described in [Ref.]. N1] sources shows NR CPP is [better/worse] than DL- TDOA (or UL-TDOA) at CDF (50%) and [N2] sources shows NR CPP is [better/worse] than DL- TDOA (or UL-TDOA) at CDF (80%): - The evaluation results from the sources ([X1],,…) show the positioning accuracy of NR CPP is X m (50%) and Y m (80%) , while the positioning accuracy of DL- TDOA (or UL-TDOA) is X m (50%) and Y m (80%). - … - The sources ([X1], [X2],…) evaluates that impact of ARP errors of [X] cm with double differential techqniue described in [Ref. and the carrier phases of multiple carrier frequencies described in [Ref.]: - The evaluation results from the sources ([X1], [X2],…) show the positioning accuracy of NR CPP is X m (50%) and Y m (80%) , while the positioning accuracy of DL- TDOA (or UL-TDOA) is X m (50%) and Y m (80%). - … - The sources ([X1], [X2],…) evaluates that impact of ARP errors of [X] cm with [other techqniue] described in [Ref.]: - The evaluation results from the sources ([X1], [X2],…) show the positioning accuracy of NR CPP is X m (50%) and Y m (80%) , while the positioning accuracy of DL- TDOA (or UL-TDOA) is X m (50%) and Y m (80%). - … From above evaluation results, we have the following observations for ARP errors, - Observation 1: Without the use of the double differential techqniues, … - Observation 2: Witht the use of the double differential techqniues, …. - … Based on above observations, we have the following conclusions for ARP errors: - Conclusion 1: Without the use of the double differential techqniues, … - Conclusion 2: Witht the use of the double differential techqniues, …. - … Impact of Initial Phases on NR carrier phase positioning … Impact of CFO errors on NR carrier phase positioning …

### (H)(Round 4) Proposal 5.2-2a

- To minimize the simulation effort and also for better comparison of the positioning performance from different companies, it is encouraged to focus on the use of the following evaluation assumptions.

  - baseline scenarios: InF-SH and InF-DH

  - FR1: BW 100 MHz

  - ARP error: T= [0, 1]cm

  - CFO: 0, [-30, +30] Hz (FR1, UE); [-10, +10] Hz (for each TRP, FR1),

  - PCO: a = [0, 3].

  - [TRP synchronization=0, 50ns]

- Note: Evalution results with other simulation assumptions agreed in the previous meeting are also welcome.

| Company | comments |
| --- | --- |

### (H)(Round 4) Proposal 5.2-2b

When providing the evaluation results, it is encouraged to provide the following information with their observations.

- The impact of each error sources (Initial Phase, CFO, Oscillator-drift, PCO, [TRP synchronization]:

  - Each of above error sources is evaluated separately, assuming there is no other errors.

- The CDF values: at least the horizontal accuracy of 50% and 80% UEs:

  - It would be better to have the results of both NR carrier phase positioning and the DL- TDOA/UL-TDOA, if DL- TDOA/UL-TDOA for comparison.

- Additional important assumptions and/or methods used in the evaluation: e.g., whether the double differential technique, and/or the carrier phases of multiple (sub)carriers:

  - If not mentioned, it is assumed “the double differential technique” and “the carrier phases of multiple (sub)carriers” are not used

| Company | comments |
| --- | --- |

### (H)(Round 4) Proposal 5.2-2c

Companies are encouraged to provide at least one evaluation result with all error sources (Initial Phase, CFO, Oscillator-drift, PCO, [TRP synchronization]) being modelled.

| Company | comments |
| --- | --- |

### (H)(Round 4) Proposal 5.2-2d

Capture the following TP into TR 38.859 as an evaluation observation (for Section 6.3.2):

The impact of the initial phases of the transmitter and the receiver on NR carrier phase positioning is evaluated during the study item.

- The evaluation results from the sources (e.g., Huawei[1], vivo[2], CATT[6], ZTE[9]) show that the initial phases of the transmitter and the receiver may significantly degrade the positioning performance. It is impossible to support centimeter-accuracy positioning if these errors are not removed.

- The evaluation results from the sources (e.g., Huawei[1], CATT[6], ZTE[9]) show that the initial phases of the transmitter and the receiver are removed effectively by the double differential technique with the use of the PRU.

(Not captured in TR) Note: The number of sources and the references can be further updated in next meeting depending on companies’ updates of simulation results.

| Company | comments |
| --- | --- |
| FL | The above proposal may not capture the evaluation results from all sources. Companies are welcome to provide inputs of their observations. |

### (H)(Round 4) Proposal 5.2-2e

Capture the following TP into TR 38.859 as an evaluation observation (for Section 6.3.2):

The impact of the CFOs of the transmitter and the receiver on NR carrier phase positioning are evaluated during the study item.

- The evaluation results from the sources (Huawei[1], ZTE[9]) shows the impact of CFOs on carrier phase positioning is negligible.

- The evaluation results from the source (vivo[2]) shows that with CFOs the performance of carrier phase positioning degrades, which may be worse than TDOA only positioning.

- The evaluation results from the source (CATT[4]) shows the impact of the CFOs on the positioning performance of carrier phase positioning is removed with the use of the double differential technique.

(Not captured in TR) Note: The number of sources and the references can be further updated in next meeting depending on companies’ updates of simulation results.

| Company | comments |
| --- | --- |
| FL | The above proposal may not capture the evaluation results from all sources. Companies are welcome to provide inputs of their observations. |

(H)(Round 1) Proposal 5.2-3

Capture the following TP into TR 38.859 as an evaluation observation (for Section 6.3.2):

- The correct resolution of the integer ambiguity is one of the key issues for accuracy improvement by the use of carrier phase positioning.

| Company | comments |
| --- | --- |
| Nokia/NSB | Okay |
| CATT | Support |
| ZTE | We are fine with this proposal. |
| MTK | We also find out that the carrier phase error (fractional cycle) would impact the search of integer ambiguity. It means the carrier phase error (through measurement and double difference method) may need to be within a threshold to ensure the correct search. I am wondering whether FL would capture our observation by saying one company has such observation? |
| CMCC | Similar comments as Proposal 5.2-1. We are basically fine and recommend to have more details if possible. |
| Intel | Agree. |
| Ericsson | Ok |
| Qualcomm | OK |
| Locaila | Ok |
| Philips | Agree |
| LGE | Support |
| Apple | OK |
| OPPO | Ok |

FL Comments:

Based on the comment, it would be better to have more details based on the evaluation observations in the TP based on the evaluation results from the companies.  The proposal is revised as follows:

(H)(Round 2) Proposal 5.2-3

Capture the following TP into TR 38.859 as an evaluation observation (for Section 6.3.2):

- The correct resolution of the integer ambiguity is one of the key issues for accuracy improvement by the use of carrier phase positioning. The evaluation results (e.g., Huawei[1],vivo[2], Nokia[3], OPPO[4], CATT[5], Intel[7], ZTE[9], LGE[19], Qualcomm[21], MediaTek[22]) show that centimeter-level positioning accuracy can be achieved if the integer ambiguity is resolved correctly. But, the accuracy of the arrier phase positioning could be significantly, or even worse than legacy methods, if the integer ambiguity are not resolved correctly.

| Company | comments |
| --- | --- |
| LGE | We are fine with the proposal. |
| CATT | Support |
| Nokia/NSB | Okay |

(H)(Round 3) Proposal 5.2-3

Capture the following TP into TR 38.859 as an evaluation observation (for Section 6.3.2):

- The correct resolution of the integer ambiguity is one of the key issues for accuracy improvement by the use of carrier phase positioning. The evaluation results (e.g., Huawei[1],vivo[2], Nokia[3], OPPO[4], CATT[5], Intel[7], ZTE[9], Samsung[17], LGE[19], Qualcomm[21], MediaTek[22]) show that centimeter-level positioning accuracy can be achieved if the integer ambiguity is resolved correctly. But, the accuracy of the carrier phase positioning could be degraded significantly, if the integer ambiguity is not resolved correctly.

(Not captured in TR) Note: The number of sources and the references can be further updated in next meeting depending on companies’ updates of simulation results.

| Company | comments |
| --- | --- |
| vivo | We think the following sentence may not be complete “But, the accuracy of the carrier phase positioning could be significant, or even worse than legacy methods” |
| ZTE | OK, in line with our simulation. |
| FL | To vivo’s comment: The sentence is indeed incomplete. It is changed to: But, the accuracy of the carrier phase positioning could be degraded significantly, or even worse than legacy methods, if the integer ambiguity is not resolved correctly. |
| Nokia/NSB | Sorry for the late comments but we think “worse than legacy methods” is a very vague statement. We prefer to remove it as it is already clear without it that CPP performance will be severely impacted if IA is not resolved. |
| FL | To Nokia: Okay. The text is modified as suggested. |
| Samsung | We have also shown that we can achieve cm-level accuracy by resolving the interger ambiguity. |
| vivo | We prefer to keep the deleted wording and maybe it can be changed to “worse than Rel-16/Rel-17 method ”, to illustrate, with the incorrect integer number, there may be no benefit compared to the existing method. |
| FL | To Samsung: I’ve added Samsung into the source list. To Nokia/vivo: Okay. Oneway to resolve the the issuesmay be we can list the sources that shows the worse than Rel-16/Rel-17 method. I will make the update soon to see if it can be accepted. |
| Ericsson | We prefer to be more quantitative with the observations. For instance, what gains/degrations do companies observed under which assumptions? “degraded significantly” is not very quantitative. So this proposal needs more work and we can discuss it next week. |

FL Comments:

From the discussion, it is a common observation on the accuracy when the integer ambiguity is resolved correctly. But, there are different views on the observation when the integer ambiguity is not resolved correctly, and some companies wants “quantitative” number for that. Consider there will be more evaluation results in the next meeting, may be we can work on the observation when the integer ambiguity is not resolved correctly in the next meeting.

### (H)(Round 4) Proposal 5.2-3

Capture the following TP into TR 38.859 as an observation (for Section 6.3.2):

- The correct resolution of the integer ambiguity is one of the key issues for accuracy improvement by the use of carrier phase positioning. The evaluation results (e.g., Huawei[1],vivo[2], Nokia[3], OPPO[4], CATT[5], Intel[7], ZTE[9], Samsung[17], LGE[19], Qualcomm[21], MediaTek[22]) show that centimeter-level positioning accuracy can be achieved when the integer ambiguity is resolved correctly.

| Company | comments |
| --- | --- |

### (Closed) Proposal 5.2-4

Capture the following TP into TR 38.859 as an evaluation observation (for Section 6.3.2):

- The use of multi-frequency carrier phase measurements is helpful for the correct resolution of the integer ambiguity and alleviates the impact of ARP location errors.

| Company | comments |
| --- | --- |
| vivo | For us, based on the FL explanation in3.2-1 that “multi-frequency” is a different frequency(DC) other than subcarrier. The proposal is too early since there are no related agreements for evaluation (e.g the error model (e.g phase error, timing draft) between different carriers)and without enough evaluation results. FL: Not sure if I made it clear. In my response to vivo’s comment in 3.2-1, I have used TS 38.211 as example to explain the frequency of a subcarrier in my mind. That is, if f0 is the carrier frequency, k is the subcarrier index, and Δf is subcarrier spacing. Then, the carrier frequency of the subcarrier carrier k is fk=f0+k+k0μ-Ngrid,xsize,μNscRB2Δf. Furthermore, even based on the current CATT evaluation, we don’t see an obvious improvement based on multiple frequencies, especially in a lower percentage. FL: Are you comparing the Case 1 and Case 2 in CATT’s paper? These cases are simulated w/o the error sources. They do not show the benefits of using multiple carriers. The benefits of using the phase measurement from multi-frequency are presented in Case 3 and Case 4, and the benefits are also captured in CATT’s observations. So, we propose to discuss the “multi-frequency” evaluation assumption first. FL: Yes. We already have the agreement for the initial phase of all subcarriers in a carrier. We will only need to add the case when the carrier phases are not from the same carrier (see (H)(Round 2) Proposal 5.1-1a). |
| Nokia/NSB | Similar as prior comments from us we should say positioning frequency layers |
| CATT | Support. |
| CMCC | Similar comments as Proposal 5.2-1. We are basically fine and recommend to have more details if possible. |
| Xiaomi | Support |
| Ericsson | Do not agree. We agree with comments form vivo. |
| Qualcomm | Clarify whether the proposal implies that increasing total bandwidth of all the PRS at different frequencies is helpful, or whether the proposal implies that dividing a given total bandwidth into multiple frequencies and making measurements separately on those is helpful. For alleviating ARP location errors, is it assumed that the ARPs for the different frequencies are all collocated? |
| Locaila | Same opinion as vivo. |
| Philips | We agree with vivo |
| FL | Based on the feedback, it might be better to wait for more evaluation results. Thus, the suggestion is to further discuss this issue in the next meeting. |

### (Closed) Proposal 5.2-5

Capture the following TP into TR 38.859 as evaluation observation (for Section 6.3.2):

- Different approaches can be used to obtain the carrier phase measurements, and different approaches may have different measurement accuracy.

| Company | comments |
| --- | --- |
| Nokia/NSB | Don’t support. What does this gain us? |
| CATT | Maybe we can include more specifically which approaches can provide better accuracy. |
| Qualcomm | Looks very vague |
| Locaila | Don’t support |
| LGE | Intention of this proposal is not clear for us. |
| FL | Given the time-left in this meeting, we may not have the time to discuss this issue. Since this is more related to implementino, the FL suggest losing the discussion for this proposal in this meeting. |

### (Closed) Proposal 5.2-6

Capture the following TP into TR 38.859 as an evaluation observation (for Section 6.3.2):

- The impact of carrier frequency offset, which is given in the evaluation assumption, on the accuracy of the carrier phase positioning is negligible.

| Company | comments |
| --- | --- |
| CATT | Okay. Maybe add the conditions under which the impact of the carrier frequency offset is negligible. |
| ZTE | We are fine with it. |
| Locaila | We don’t agree. |
| FL | Given the time-left in this meeting, the low interests on the proposal, the suggestion is close the discussion for this proposa in this meeting. |

# Others

## (Closed) TRP time synchronization

Submitted Proposals:

- ((Locaila, R1-2209543[15]) Proposal 4) We propose to study the proposed master-slave gNB synchronization structure and capture the solution in TR 38.859 as an alternative for double difference method.

- ((Locaila, R1-2209543[15]) Proposal 5) We propose to remove CFOs of TRPs defined in table A.3-1 in current TR38.859 on the case that proposed master-slave gNB synchronization structure is assumed.

FL Comments

TRP time synchronization is essential for timing-based positioning methods, including DL-TOA, UL-TDOA and carrier phase positioning. The proposed master-slave gNB synchronization structure could be a potential solution for precise TRP time synchronization. However, the investigation of TRP time synchronization is out of the scope of the SI in FL’s view. The issue was discussed in previous meeting, but only few companies were involved in the discussion. Thus, the suggestion from FL is no further discussion on this issue in this meeting.

(Closed) Proposal 6.1-1

- No further discussion of master-slave gNB synchronization structure in the SI.

| Company | comments |
| --- | --- |
| Nokia/NSB | Okay with the intention to not discuss. We should not use this type of terminology in 3GPP anymore. There was agreed at RAN level to update specifications to explicitly avoid this type of wording. |
| Samsung | This type of synchronization structure (main-secondary) is outside the scope of RAN1. So we agree with this proposal. |
| Locaila | We're just beginning to figure out what the problems are with introducing high-precision Carrier Phase Positioning into the 5G NR systems. It is not good to reject a particular solution for non-technical reasons from the beginning of the study phase. The master-slave synchronization method we propose is not designed simply by theoretical or simulation, but by actual use and validation of our field experiment. Using this method, we were able to stabilize the carrier phase offset of TRPs for a long time and successfully obtain high precision distance measurement results. This method does not require large PRU device deployments, nor does it cause a traffic overload on the network like the double difference method. Although there may be some structural impact on 5G gNB to apply this method, it may actually be a more cost-effective solution considering the cost of maintaining or managing the numerous PRUs required to apply the double difference method. We should continue to study the pros and cons of solutions in the framework of the big theme of TRP synchronization from a more open perspective |

FL Comments

The discussion is closed based on the feedbacks.

## (Closed) Sidelink carrier phase positioning

Agreement (RAN1#109e) The study of the accuracy improvement based on NR carrier phase measurements in Rel-18 SI may include: - UE-based and UE-assisted carrier phase positioning, - UL carrier phase positioning and DL carrier phase positioning. - NR carrier phase positioning with the carrier phase measurements of one carrier frequency or multiple frequencies - Combination of NR carrier phase positioning with another standardized Rel. 17 positioning method, e.g., DL-TDOA, UL-TDOA, Multi-RTT, etc. - Note: The use of “carrier phase positioning” does not necessarily mean it is a standalone positioning method - FFS: whether SL carrier phase positioning is to be discussed in Rel-18 SI

.Submitted Proposals:

- (Nokia, R1-2208736[3]) Proposal 13: Discuss SL carrier phase only under AI 9.5.1.2 during RAN1#110-bis.

- (InterDigital, R1-2209489[13]) Proposal 1: Postpone the study of SL carrier phase positioning to a future release beyond Rel. 18.

- (NTT DOCOMO, R1-2209909[20]) Proposal 4: It may be better to postpone the study of SL carrier phase positioning in future release.

- (Ericsson, R1-2210177[23]) Proposal 1	Carrier phase-based positioning using sidelink measurements shall not be considered in NR Rel. 18.

FL Comments

From the contributions submitted under this AI for carrier phase positioning, multiple companies [7][14][19][21] propose either not discussing SL carrier phase positioning in RAN1#110, or not considering SL carrier phase positioning in Rel-18. Thus, the FL suggests no further discussion of SL carrier phase positioning in RAN1#110 under this AI.

(Closed) Proposal 6.2-1

- No further discussion of SL carrier phase positioning in this meeting under this AI.

| Company | comments |
| --- | --- |
| InterDigital | Support |
| Nokia/NSB | Support |
| Xiaomi | Support |
| Intel | Support. |
| Locaila | Support |
| LGE | Support |
| Apple | Support |

FL Comments

The discussion is closed based on the feedbacks.

# Proposals for 1st Checkpoint (Oct. 14)

### (H)(Round 3) Proposal 3.1-2

- For UL UE-assisted NR carrier phase positioning, at least consider the carrier phase measured from the UL SRS for positioning purpose.

  - Note: The use of MIMO SRS for positioning purpose is transparent to UE.

| Company | comments |
| --- | --- |
| ZTE | We are wondering whether ‘UL’ should be added before ‘UE-assisted’, UE-assisted positioning is also considered in current DL positioning protocol. - For UL UE-assisted NR carrier phase positioning, at least consider the carrier phase measured from the UL SRS for positioning purpose. - Note: The use of MIMO SRS for positioning purpose is transparent to UE. |
| FL | To ZTE: Adding “UL” seems okay in my view. |
| Huawei, HiSilicon | In general for UL positioning, we do not say UE assisted, but rather network assisted. Anyway, we can live with the wording. |
| FL | To Huawei, Okay. Let us keep the wording. |
| Ericsson | we are ok with this. But this is only to scope what UL signals is to be used for NR CPP in UL and not a TR conclusion right? |
| FL | To Ericsson: Similar to the agreement made in DL side, my understanding is that the text will be captured in the Section 6 of the TR. Then, for the content in the conclusion session of the TR, we may decide in the next meeting. I assume the workding will be something like : |

### (H)(Round 3) Proposal 3.2-1a

- Reporting of the measurements based on the carrier phases of multiple subcarriers within a carrier can be considered for NR carrier phase positioning. The candidate measurements at least include the following:

  - the carrier phases of RF frequencies corresponding to multiple subcarriers

  - the difference between the carrier phases of the multiple subcarriers

  - the slope determined of the carrier phases of the multiple subcarriers

| Company | comments |
| --- | --- |
| MTK | 1, we are not sure the 2nd and 3rd have any significant difference? It seems to us that when the carrier phase of some multiple subcarriers are measured, the difference of the carrier phases could be reported (similar to absolte RSRP and differential RSRP). But we here are talking about “measurement”, not “reporting”, then we are not sure 2nd and 3rd are different for what? |
| ZTE | The following bullets contains the same meaning I guess, the difference/slope of different subcarriers can be calculated using the measurement results of the first bullet. And for the third bullet, would it be better to revised as follows: - the slope determined by the difference of the carrier phases of the multiple subcarriers |
| FL | To MTK/ZTE: My understand is that for the 2nd sub-bullet, the intention is to allow the report the differences between the carrier phases, while the 3rd sub-bullet reports the slope derived from the differences between the carrier phases. To ZTE: The suggeste rewording is okay to me. |
| Nokia/NSB | We have some concerns on this proposal as we don’t think that everyone has the same understanding of what “measurements based on the carrier phases of multiple subcarriers” means. We would like to continue the discussion of this topic next week. |
| Samsung | We support this proposal. For third sub-bullet there is a small editorial change: - the slope determined of by the carrier phases of the multiple subcarriers |
| Huawei, HiSilicon | Do not support the last two bullets. Losing absolute frequency information from the measurement is no longer regarded as carrier phase |
| Qualcomm | Same view as Nokia. To elaborate some more: In general we are much more supportive of 3.2-1b than this proposal, and think we should defer this agreement until its value has been better established. In 3.2-1b, addition of multiple PFLs can only help in general (because it is providing additional measurements on top of the single PFL case). Whereas in this proposal, the idea appears to be to break up a single PFL into subbands or groups of “multiple subcarriers” and report measurements for each group. Are the measurements for each group going to be based only on the received subcarriers in that group, or are they based on all the received subcarriers? If using all the received subcarriers, then all we need is a single measurement corresponding to the phase of the 1st path as determined from all those subcarriers; so we don’t see the value of multiple measurements. If using only the received subcarriers within the group, then each measurement (of each of the groups) would be of inferior quality compared to the single measurement that used all the subcarriers, because of lower bandwidth, which limits the accuracy of resolving the first path. We could of course combine these multiple lower quality measurements to obtain a position estimate that would be more accurate than using any one of them individually. But it is highly unclear that this would be more accurate than just using all the subcarriers to compute a single carrier phase measurement. We understand that the motivation behind this proposal appears to be related to ideas of ‘lane combining’ (wide-laning and narrow-laning) in GNSS literature. But the right analogy for those GNSS ideas is 3.2-1b, not this proposal. Multiple PFLs in general cannot be combined coherently, and having them can only help (relative to a single PFL). But all the subcarriers in a PFL can be coherently processed, and so they should be – the value of reporting separate subbands is very unclear. The above comments are addressed mainly at the 1st sub-bullet. For the 2nd and 3rd sub-bullet, it has been pointed out earlier (e.g., by Huawei) that the differences or slope of the phase as a function of subcarrier-index is equivalent to a TOA, so it does not need a separate report, for example existing RSTD report serves the same function. |
| FL | Thanks for the comments. It seems we would need more time to work on this proposal in next week. |
| Ericsson | We have concerns on this proposal. We are still not conviced on the benefits of reporting carrier phases of individual subcarriers. As we stated previously, we think reporting center carrier phase and the estimate of the time-of-arrival of the reference signal is enough. We can discuss this proposal next week. |

### (H)(Round 3) Proposal 3.2-1b

Further study the benefits of using the carrier phase measurements of multiple DL positioning frequency layers for NR carrier phase positioning, which may include the impact of the time gap (if any) between the carrier phase measurements of multiple DL PFLs.

- Note 1: The initial phase error and the frequency error for each PFLs can be modelled independently

- Note 2: For the evaluation, the PRS signals of all PFLs of a TRP can be assumed to be transmitted from the same ARP or from different ARPs of the TRP.

- Note 3: The location error for ARPs can be modelled independently.

| Company | comments |
| --- | --- |
| vivo | Thank for the FL proposal, we are okay to remove ARP error, but the different timing errors (e,g, TEG) between multiple DL PFLs may need to be considered since the TEG ID may be different for different PFL Further study the benefits of using the carrier phase measurements of multiple DL positioning frequency layers for NR carrier phase positioning, which includes the impact of the time gap and timing error between the carrier phase measurements of multiple DL PFLs. - Note 1: The initial phase error and the frequency error for each PFLs can be modelled independently - Note 2: For each TRP, the PRS signals of all PFLs are transmitted from the same ARP. |
| MTK | 1, we are not sure whether timing error between PFLs should also be considered. It seems to us that it is related to TX chain structure |
| ZTE | We are wondering that the time gap mentioned here is used to identify the movement of the target UE? Or to combine with the Tx time of DL PRS to get the real propagation time? |
| FL | To vivo/MTK: For vivo’s comment, I share the similar view with MTK that it is unclear to me the motivation to add the “timing error” between PFLs. To ZTE: The time gap is added here to address the concern the UE may not measure the DL PFLs simultaneously, and thus there can be a time gap for the measurements obtained from different DL PFLs. |
| Nokia/NSB | We are overall supportive but have some small comments: 1. our first preference is to remove the entire end of the main bullet (e.g., stop the proposal after carrier phase positioning.) 2. if that is not acceptable then we should prefer to add (if any) to the time gap part. 3. We agree with above comments that timing error between PFLs does not need to be considered at this time. |
| FL | To Nokia: Okay. It seems reasonable to add “if any” to the time gap part, since some companies may want to see the impact of time gap in case there is the gap. |
| Vivo | We believe the performance will be improved if we simply increase the number of PFLs. But we think some errors between PFLs can be considered for evaluation for the boundaries of multi-carriers CPP. For GNSS, we understand the I timing error may not exist, but for NR devices, with the timing gap, and the RX/Tx chain structure, we are not sure all the timing is aligned for different PFLs measurements. In this case, timing offset due to timing gap or RX/Tx chain structure may need to be considered. In addition, we have some concerns about adding “if any”, based on current specifications, only one PFL can be measured in an MG or PPW, so we wonder which case can be seen as ‘no time gap’ |
| Qualcomm | We think Note2 might be unnecessarily restrictive; e.g. NR-TRP-LocationInfo-r16 is currently separate per PFL. |
| FL | To vivo: If I understand vivo’s concern correctly, it is about PFLs are transmitted with different Tx chains and received with different Rx chains, which could happen if PFLs are different bands or FR1/FR2. How about we add the following note to address vivo’s concern. - Note 3: The timing errors of the PFLs may not be the same for PFLs in different bands or frequency ranges. For the second comment, it depends on the assumption and UE capability. If a UE can only process one PFL in one PPW, then yest, there will be gap. On the other hand, for a UE support dual connectivity, it may process two PFLs simultaneously. I assume adding “if any” covers both cases. To Qualcomm: Yes. The spec does not limit PFLs in the same TRP-Location. But, for the effort of the evaluation, I am wondering if any company wants to simulate that case, since it makes the evaluation much more difficult. Maybe we can say - Note 2: For the evaluation purpose, the PRS signals of all PFLs of TRP are transmitted from the same ARP. |
| Ericsson | There are some comments on this proposal. So this may need some further discussion. We agree that timing errors between different PFLs may need to be considered as part of the study (e.g., if two PFLs are in different bands). We are not supportive of the revised Note 2. This note may ignore in evaluations ARP differences when transmitting PRS signals of different PFLs. So we prefer to model ARP errors as part of the study. |
| FL | To Ericsson: Okay. We can make the following change for supporting the evaluation of different ARPs. - Note 2: For the evaluation, the PRS signals of all PFLs of a TRP can be transmitted from the same ARP or from different ARPs of the TRP. - Note 3: The location error for ARPs can be modelled independently. |

### (H)(Round 3) Proposal 3.4-1

- Using carrier phase information for smoothing existing timing measurements for DL-TDOA, UL-TDOA and Multi-RTT can be studied for UE-assisted and UE-based positioning, including the impact on the specification.

| Company | comments |
| --- | --- |
| ZTE | Low priority, this seems to enhance the existing methods, seems out scope. We prefer focusing on the carrier phase method itself in this release. |
| Nokia/NSB | Agree with ZTE. We don’t feel it is worth focusing on this issue and think “carrier phase information” is very vague. We can discuss more next week but we still feel that the focus needs to be on the feasibility and benefit of CPP in general. |
| Huawei, HiSilicon | Low priority. |
| Ericsson | there is possible benefit when carrier phase measurement is combined with other methods. If UE reports carrier phase measuremetns to LMF, the LMF can anyway combine these measurements with existing posisitinging measurements like DL-TDOA, etc. So, we support studying this use case. |
| Samsung | We agree with Ericsson |

### (H) (Round 3) Proposal 4.1-1

Capture the following TP into TR 38.859 as a conclusion (Section 6.3.1):

- The use of the positioning reference unit (PRU) to facilitate NR carrier phase positioning has been evaluated during the study item. Based on the evaluation results, the double differential technique with PRU is beneficial for NR carrier phase positioning, which includes at least the following:

  - Eliminating the impact of the TRP timing synchronization error

  - Eliminating the impact of the initial phases of the transmitter and the receiver.

- The evaluation results for the effectiveness of these solutions are presented in Section 6.3.2.

| Company | comments |
| --- | --- |
| Locaila | We have argued in previous discussions about the overhead of PRU in 5G NR system. We believe that PRU is not helpful for synchronizing carrier phases in the indoor as well as macro, highway environment. In particular in an indoor environment, TRPs or gNBs can be connected using an external oscillator and RF cable and synchronize carrier phase perfectcly. Any large factory or warehouse environment of hundreds of square meters can be fully synchronized using synchronization cables. Therefore, such system as a PRU and double difference method is not necessary at least in indoor environment. Nevertheless, writing these conclusions can give people the wrong message as if PRU is an essential element in all environments to implement carrier phase positioning. This may make widespread use of 5G carrier phase positioning difficult in market. Therefore, we propose to put following note to convey faithful information to the reader. ================================== Capture the following TP into TR 38.859 as a conclusion (Section 6.3.1): - The use of the positioning reference unit (PRU) to facilitate NR carrier phase positioning has been evaluated during the study item. Based on the evaluation results, the double differential technique with PRU is beneficial for NR carrier phase positioning, which includes at least the following: - Eliminating the impact of the TRP timing synchronization error - Eliminating the impact of the initial phases of the transmitter and the receiver. Note: other means for gNB/TRP synchronization such as using synchronization cable may also be effecitive in some environment (e.g. indoor). - The evolution results for the effectiveness of these solutions are presented in Section 6.3.2. |
| FL | To Locaila: It is unclear to me why “writing these conclusions can give people the wrong message as if PRU is an essential element in all environments to implement carrier phase positioning”. The conclusion here is about the evaluation of “double differential technique with PRU”. The study has not investigated the effectiveness of “other means for gNB/TRP synchronization such as using synchronization cable”. Thus, it is improper to add a note to say “other means for gNB/TRP synchronization such as using synchronization cable may also be effecitive in some environment (e.g. indoor)” . |
| Nokia/NSB | Agree with FL. We don’t understand the comments from Locaila. Actually in their paper they argue that a TRP/gNB can measure the PRS of another TRP/gNB to perform synchronization. This is in effective a similar mechanism to what the PRU does. How can Locaila claim that PRU is not helpful while promoting a very similar technical concept? |
| Huawei, HiSilicon | Share same understanding as FL and Nokia. We also noticed that PRU could be helpful cancelling the gNB ARP error via double differential methods. |
| Qualcomm | Similar view as FL/Nokia/Huawei. Also note that phase synchronization could be much more challenging than time synchronization, especially at higher carrier-frequencies. |
| Ericsson | We have concerns with this proposal. Firstly, why is ‘Eliminating the impact of the TRP timing synchronization error’ even mentioned in the proposal? In our view, study gNB synchronization error mititgation is not part of the study item. So, these issues that are out of scope should not be written part of the conclusions. Secondly, we don’t support writing conclusions already in this meeting. Conclusions should be written in the last meeting when results from every company is captured. What we could accept is capturing observations made by different companies and also list how many companies make similar observations just like in other AIs. This proposal needs further discussion next week. |
| Locaila | Let us explain our point in detail again into three indoor/highway/macro environment. 1) Indoor Environment (e.g InF, IOO, …) Figure 1 External clock signal sharing using RF cable The figure 1 above shows an example of three devices that can input an external clock signal source connected to the master clock oscillator. The upper master oscillator continuously distributes a clock synchronization signal of 10 MHz over the RF cable. The lower three devices output the carrier signal according to the clock signal supplied from the master. As a result, the carrier phase signals transmitted by the three devices are all synchronized as if they were transmitted by single device. Distributed gNB/TRPs in the indoor environment can also be connected by a synchronous cable that shares the clock signal as shown above. In this case, the carrier phase offset between TRPs can be eliminated as close to zero as ideal. Our experimental results show that even if the RF cable was extended several tens of meters, clock signals could still be distributed to remote devices with little loss, making it work like a single system. In other words, even in a large factory or warehouse with a size of several hundred square meters, TRPs can be connected using the synchronization cable. We believe such external clock distribution is a widespread technique to the field engineers. We don’t think it is necessary to investigate and evaluate the already widely practiced method for comparison with the double difference method. Thus PRUs are not necessary, at least in the factory, warehouse, office, and store environment. Simply sharing an external clock signal can effectively synchronize distributed TRPs and eliminate carrier phase errors, better than the proposed double difference method. 2) Highway Environment Figure 2 RSU+PRU highway deployment for carrier phase positioning The figure 2 above illustrates the structure of LMF and the RSUs which combine the function of PRU in a Highway environment. Our FL (CATT) also said RSU can incorporate with PRU, and we agree with that point. The gNBs of the master-slave synchronization structure proposed by Locaila in [15] can similarly be seen as a combined RSU and PRU device in some sense. I think Nokia's comment above is probably because of the structural similarity of RSU+PRU that we propose. However, the key difference between Locaila's proposal, and our main point of argument is not PRU itself, but about the involvement of LMF necessary for the double difference method. Note that LMF will be the place for collecting the phase offset information in UE-assisted-positioning. Similarily, we assume LMF will be the central place for gathering the offset information in following scenario. The figure below illustrates the procedure for exchanging messages with LMF. Since the highway environment is assumed, we suppose a UE-based-positioning in this scenario with a vehicle speed of 100 Km/h and a PRS transmission interval of 4 msec. Figure 3 Message timing diagram in LMF and double difference method RSU#1 and RSU#2 transmit PRS every 4 msec cycles. Since we assumed combined (RSU+PRU) device, they can calculate the carrier phase offset by receiving the PRS signal from the neighboring RSUs. All of these information should be delivered to the LMF. LMF must process phase offset information collected from the (RSU+PRU) devices within 4 msec, and broadcast the information again through RSUs. If there are hundreds of thousands of RSUs deployed, LMF should be able to process several gigabytes of information every 4 msec. We need supercomputing LMF !!!. Calibration information calculated by LMF should be broadcast again in 4 msec cycles via RSUs. Currently, no system broadcast specification has been defined at such frequent intervals. Of course, it will have a significant spec impact. Slow processing of the LMF will cause the vehicle to receive calibration information late and the position accuracy will be degraded. Slowing the PRS transmission cycle to reduce the LMF burden can also increase the positioning error of vehicles. After all, the PRU and double difference method, which requires LMF involvement, may not be appropriate in a Highway environment. Again, this doesn’t denying the combined use of RSU+PRU. Our point is that the double difference method will give burden to LMF and 5G NR system. On the other hand, the figure below depicts the timing procedure for applying the master-slave synchronization structure proposed by Locaila in [15]. Figure 4 Message timing diagram in master-slave fast synchronization [15] Similar to the above case, RSU#1 and RSU#2 transmit PRSs every 4 msec cycles. Since RSUs incorporate the PRU function, they receive the PRS signal from the neighboring RSUs to calculate the carrier phase offset. However in the Locaila [15] proposal, the calculated phase offset is used for rotating the carrier phase of the PRS signal to be transmitted in the next 4 msec cycle. This ensures that the carrier phase of the slave RSU always follows the carrier phase of the master RSU, and that the offset of the carrier phase received by the UE always remains zero. The figure below shows the result of synchronizing the carrier phase of master and slave gNB in the same way as above and observing the carrier phase difference for 5 minutes. Each point represents a phase difference calculated using 8 subcarriers of 2 gNBs. The UE is stationary during the observation period. Even though the master oscillator is drifting, the carrier phase difference of the two signals remains constant because the slave is faithfully following the master carrier phase. Figure 5 phase offset between master-slave gNBs (5min) As shown in the experimental results above, the master-slave structure is simpler than the double difference method, and it can synchronize the carrier phase quickly and effectively, without additing burden to LMF and 5G NR system. 3) Macro In a Macro environment, there should be at most 4~7 gNB/TRPs that a single PRU can see. This will require massive PRU deployment and management costs. If there are more PRU deployments, the traffic overhead concentrated on the LMF will also be very large, and the location accuracy degrade due to the processing delay of the LMF. We doubt the simulation results of the double difference method presented by many companies have considered these points. In conclusion, we argue that the double difference method using PRU is not a very effective means in the indoor/highway/macro environment. In particular it is not useful in the indoor environment. Therefore, we cannot agree with the proposed text above. We ask at least to include following note in the text in order to convey accurate information to the readers. Otherwise, we worry this text may give impression that the proposed double differential technique with PRU is the only method beneficial for NR carrier phase positioning, which is not true at least in indoor environment. ------------------ proposed text ----------------------------- Capture the following TP into TR 38.859 as a conclusion (Section 6.3.1): - The use of the positioning reference unit (PRU) to facilitate NR carrier phase positioning has been evaluated during the study item. Based on the evaluation results, the double differential technique with PRU is beneficial for NR carrier phase positioning, which includes at least the following: - Eliminating the impact of the TRP timing synchronization error - Eliminating the impact of the initial phases of the transmitter and the receiver. Note: other means for gNB/TRP synchronization such as using synchronization cable may also be effecitive in some environment (e.g. indoor). The evolution results for the effectiveness of these solutions are presented in Section 6.3.2 |

### (H)(Round 3) Proposal 4.2-1

Capture the following TP into TR 38.859 as a conclusion (for Section 6.3.1):

- The methods of resolving integer ambiguity for NR carrier phase measurements have been investigated during the study item, which includes, but not limited to, the following:

  - The use of the virtual carrier phases (a virtual carrier phase is defined as an linear combination of the carrier phase measurements of multiple subcarriers within a carrier, or multiple positioning frequency layers);

  - The use of the estimated slope based on the phase-difference of multiple subcarriers;

  - The use of the existing positioning measurements (e.g., RSTD, RTOA)

  - The use of the calculated UE position based on existing positioning methods

  - The use of the cost functions (e.g., Huawei[R1-220xxxx,], CATT[4])

  - The combination of two or more of the above methods.

- The evaluation results for the effectiveness of these solutions are presented in Section 6.3.2.

| Company | comments |
| --- | --- |
| MTK | Generally okay. But our previous comment saying that “The use of the calculated UE position based on existing positioning methods” could also be based on defining the cost function. So we are not so sure the difference between the two subbullets. |
| ZTE | We prefer revise the main bullet as - The methods of resolving integer ambiguity for NR carrier phase measurements have been investigated during the study item, which includes, but not limited to, the following: And for the sub-bullet - The use of the cost functions My concern is that how to define the cost function or evaluation parameters that is related to the cost function. Maybe can describe more details about the cost function or proposal reference for this sub-bullet. |
| FL | To MTK/ZTE: FL: I assume there will be some overlapping between the methods. The inputs of the cost function can be “calculated UE position”, “the measured carrier phase, and other information”. The definition of the cost function is normally implementation issue. |
| Samsung | We would like to keep the “estimated slope as separate sub-bullet” (which it it already is). In our understanding the existing positioning methods refer to Rel-16/17 methods which don’t include getting the estimated slope. We would also like to add “combination of two or more of the above methods” Therefore, we prefer the following: - The methods of resolving integer ambiguity for NR carrier phase measurements have been investigated during the study item, which include at the following: - The use of the virtual carrier phases, which are obtained based on the linear combination of the carrier phase measurements of multiple subcarriers within a carrier, or multiple positioning frequency layers; - The use of the estimated slope based on the phase-difference of multiple subcarriers; - The use of the existing positioning measurements (e.g., RSTD, RTOA), which includes the use of the estimated slope based on the phase-difference of multiple subcarriers to obtain the high resolution ToA - The use of the calculated UE position based on existing positioning methods - The use of the cost functions - The combination of two or more of the above methods. |
| FL | To Samsung: The suggestion to remove “which includes the use of the estimated slope based on the phase-difference of multiple subcarriers to obtain the high resolution ToA” is okay to me. Let us wait to see if other companies are fine with it. |
| Nokia/NSB | Support the update from SS. |
| Huawei, HiSilicon | We do not agree with argument from SS. In fact, HW’s evaluation results submitted in TR 38.855 using super-resolution is based on phase slope, and we think SS’s understanding that Rel-16 positioning methods do not include getting the estimated slope is not correct. One can easily approve that for an ideal single path dArgCFRfdf=dArgexp(-2πjτf))df=-2πτ The phase slope is equivalent to ToA. We even used least square on the unwrapped channel phase response to combat the noise. The reason that we use MUSIC in Rel-17 is because multi-path limits the accuracy of phase LS and the accuracy of phase LS cannot meet Rel-17 requirement. Anyway, there are multiple implementation methods to get high accuracy ToA, and phase slope, as one way of implementation, is nothing new, and should not be listed. We do not want to give external readers the impression that RAN1 NR positioning after two Releases still think that ToA and phase slope are different concepts.. |
| FL | To Huawei, In general, I would agree with Huawei from the phase slope, the ToA can be determined. |
| Ericsson | strictly speaking this is more of a statement of different solutions used rather than a conclusion. Also, we should list all the different techniques used and should avoid things like ‘but not limited to’ in the TR. If the list is incomplete, we can amend it next meeting. Some specific comments: -> Is the term ‘virtual carrier phases’ proper defined or well understood by everyone? If not, we should not mention it in the TR. -> the use of cost function is an implementation, but if we are listing it here we may need to provide some more information |
| Intel | The bullet “The use of cost functions” is rather vague and too abstract. Without further clarirty, almost any approach can be formulated to involve some kind of cost function. Also, agree with Ericsson that “not limited to” is something not appropriate for the TR, especially when it is about the techniques studied – we should be precise about what has been studied. |
| FL | To Ericsson/Intel: I share the similar view that the wording ‘but not limited to’ can be removed. The “virtual carrier phases” is actually described in the proposal. We may say: virtual carrier phase, which is defined as the linear combination of the carrier phase measurements of multiple subcarriers within a carrier, or multiple positioning frequency layers For cost function, maybe we can add a list of the sources that use the cost function. |
| Samsung | While, we agree that the derivative of the carrier phase with respect to frequency (i.e., slope) is a quantity with units of time, and that the TOA can be obtained by the slope of a carrier phase, what is being discussed here is different. The slope isn’t necessarily the TOA (but can be related to it). For example, the phase of the received signal relative to a reference phase at the UE can be expressed as: Δφue1=ϕu0+τ+δtb-δtu2πfc-ϕb1 mod 2π, δtb and δtu are the clock biases of the gNB and UE respectively, τ is the one-way propagation delay and ϕb1 and ϕu0 are the phases of the gNB and UE. Taking the derivative with respect to frequency (i.e., slope) is τ+δtb-δtu2π, which depends not just on τ, but also on the clock biases of the UE and gNB. This is the derivative of the carrier phase (not the carrier phase difference). This is not the same as the measurements of Rel-16, for example DL RSTD is the difference in time between two signals received from two TRPs, while this can be obtained by taking the slope of the carrier phase difference (after eliminating gNB clock biases), it is different from taking the slope of the carrier phase (what is being discussed here). This can then be combined with quantatities of other entities (e.g., gNB) to eliminate the clock biases. Therefore, we should keep “The use of the estimated slope based on the phase-difference of multiple subcarriers;” |

### (H)(Round 3) Proposal 4.2-2

- Further study the impact of the solutions of integer ambiguity for UE-assisted carrier phase positioning on the specification, which includes, but is not limited to, the following:

  - FFS: Reporting of the information (e.g., slope) obtained from the phase-difference of multiple subcarriers from UE/TRP to LMF

  - Reporting of the determined integer ambiguity and/or the search range of the integer ambiguity from UE/TRP from UE/TRP to LMF

  - Reporting of the carrier phase measurements together with the legacy positioning measurements from UE/TRP to LMF

  - LMF Configure the integer ambiguity range between the TRP and target UE

| Company | comments |
| --- | --- |
| ZTE | Besides the above reporting elements, LMF can also configure the integer ambiguity to UE while implementing CPP. So we suggest revise the proposal as follows: - Further study the impact of the solutions of integer ambiguity on the specification, which includes, but not limited to, the following: - Reporting of the TOA or slop obtained from the phase-difference of multiple subcarriers - Reporting of the search range of the integer ambiguity from UE/TRP - Reporting of the carrier phase measurements together with the legacy positioning measurements - LMF Configure the integer ambiguity between the TRP and target UE FL: It is unclear to me why and how the LMF configure the integer ambiguity between the TRP and target UE. Does it mean the LMF tell the search range of integer ambiguity for UE for UE-based positioning? If so, how does the LMF get the information? Keep in mind that in this case the LMF needs to know UE’s position pretty accuracy in order to provide such information. Reply to the FL’s reply: Does it mean the LMF tell the search range of integer ambiguity for UE for UE-based positioning?->Not exacty, not for UE-based, instead, I guess LMF configuration is applicable for UE-assisted positioning. For UE assisted positioning, UE can report the search range or measurement information together to the LMF, or UE can also report the measurement information only, and LMF can configure the integer ambiguity according to the received measurement information. That is to say, the integer ambiguity can be determined by UE or LMF. Can see whether other companies have similar concern. For UE assisted method, UE has no idea on its location. LMF can get the UE location from the existing positioning method or previous positioning results (for example, in last measurement, etc). Definitely, LMF can configure a integer range for UE to reduce UE searching burden. - LMF Configure the integer ambiguity range between the TRP and target UE |
| FL | To ZTE: Thanks for the explanation of the motivation to add “LMF Configure the integer ambiguity range between the TRP and target UE”. I assume it can be added for study. |
| Samsung | Prefer to keep slope in first sub-bullet - Reporting of the information obtained from the phase-difference of multiple subcarriers (e.g., slope) from UE/TRP to LMF |
| FL | To Samsung: It we want to add “(e.g., slope)”, it may be better to be placed after “Reporting of the information”. |
| Nokia/NSB | What information will be reported in the first sub-bullet? Just the slope? We also prefer to add a bullet for “reporting of the determined integer ambiguity from UE/TRP to LMF”. |
| FL | To Nokia: The information proposed by companies include the slope, TOA. I think other information may also be useful, e,g., the uncertainty information, which are related to the search range. I aasume “reporting of the determined integer ambiguity from UE/TRP to LMF” is closely related to “Reporting of the search range of the integer ambiguity from UE/TRP”. |
| Samsung | We are fine with the latest proposal. For the last sub-bullet, it is better to reword to be similar to the other sub-bullets LMF Configure Configuring the integer ambiguity range between the TRP and target UE by the LMF |
| Huawei, HiSilicon | Do not support if the first bullet is not removed. |
| Qualcomm | Agree with Huawei about 1st sub-bullet |
| FL | To Huawei/Qualcomm/Samsung and all: Will it be acceptable to add “FFS” to the first bullet? |
| Ericsson | ok to study further. |

### (H)(Round 3) Proposal 4.3-1

Capture the following TP into TR 38.859 as a conclusion (for Section 6.3.1):

- The impact of multipath/NLOS on NR carrier phase positioning is evaluated during the study item. Based on the study, it is concluded that multipath/NLOS deteriorates the performance of carrier phase positioning and it is necessary to consider multipath mitigation for NR carrier phase positioning.

- The evaluation results for the impact of the multipath/NLOS on NR carrier phase positioning will be presented in Section 6.3.2.

| Company | comments |
| --- | --- |
| ZTE | Support |
| Qualcomm | Generally supportive, but as has been pointed out by others in previous rounds as well, multipath mitigation in some form or the other is an integral part of almost any positioning scheme. So regarding the second sub-bullet, how will we organize a separate section devoted to multipath mitigation results? (in our view every result shown could qualify to be placed in that section) FL: In my understanding, Section 6.3.1 is more related to what issues have been studied. And Section 6.3.2 for more details of the evaluation results. |

### (H)(Round 3) Proposal 4.3-2

Capture the following TP into TR 38.859 as a conclusion (for Section 6.3.1):

- The methods of mitigating the impact of multipath for the carrier phase positioning are studied during the study item. Based on the study, the following candidate methods of multipath mitigation are suggested for further consideration :

  - Identify and separate the first path and other paths

  - Reporting of the carrier phase of the first path, and optionally, the additional paths

  - The use of LOS/NLOS indication for the carrier phase measurements

    - Note: Rel-17 LOS/NLOS indicator can be considered as a starting point

  - The report of other channel information, such as RSRP/RSRPP

  - Other methods are not precluded

| Company | comments |
| --- | --- |
| MTK | How to identify and separate the paths are due to implementation, We prefer to remove “in the time-domain/CIR “ in the first subbullet |
| ZTE | For the carrier phase of the additional paths, we still doubt about its availability and usage. Can see whether other company has the same concern. For the sub-bullet: - The report of other channel information We are wondering what kind of channel information should be clarified, this is a little bit broad. |
| FL | To MTK: The wording of “in the time-domain/CIR “ is during discussion. I think the suggestion to remove it okay since it is proposal is to conclusion. To ZTE: It has “such as RSRP/RSRPP/SINR, CIR/CFR, etc.” before. Let us include “RSRP/RSRPP” since they are existing measurements. |
| Ericsson | If we put this in the conclusion, what does ‘suggested for further consideration’ mean? The TR should be complete and we can capture which methods we used. This doesn’t sound like a conclusion but an observation. or do you mean some of this techniques are to be considered in the work item? if it is the later, then we need further discussion. We also need a good understanding of which aspects can be handled by implementation and which ones need standardization work. |
| FL | To Ericsson: The proposal lists the techniques that are studied in the SI, and recommended to be considered in the work item. Yes, we can continue to work on the potential impact of standardization work. |

### (H)(Round 3) Proposal 4.6-1

Capture the following TP into TR 38.859 as a conclusion (for Section 6.3.1):

- For the purpose of NR carrier phase positioning:

  - the reporting of the carrier phase measurements together with the existing positioning measurements can be considered.

  - the reporting of the carrier phase-based measurements alone without reporting the existing positioning measurements will be further studied.

| Company | comments |
| --- | --- |
| vivo | We are not sure we can conclude directly that both can be considered in normative work， we propose to remove “ in normative work”. |
| MTK | For 2nd subbullet, we prefer for further study |
| ZTE | Would it be better to change ‘both’ to ‘one or more’, as follows: - For the purpose of NR carrier phase positioning, both on or more of the following can be considered in normative work: - the reporting of the carrier phase measurements together with the existing positioning measurements - the reporting of the carrier phase-based measurements alone without reporting the existing positioning measurements |
| FL | To vivo’s comment: We could remove “in normative work” for now and decide what be the recommended to normative work in the next meeting. To MTK’s comment: The proposal is for conclusion. If the 2nd subbullet is for further study, then we not need to have.a new agreement on it. Will MTK be okay to keep the second bullet as considered? To ZTE: The intention is both can be consisered, but limited to one of them only. |
| Samsung | We should also add “carrier phase-based measurements”. In first sub-bullet - For the purpose of NR carrier phase positioning, both of the following can be considered in normative work: - the reporting of the carrier phase-based measurements together with the existing positioning measurements - the reporting of the carrier phase-based measurements alone without reporting the existing positioning measurements |
| Huawei, HiSilicon | We cannot agree to the second subbullet. |
| FL | To Samsung: Okay. Change is made as suggested. To Huawei/MTK: - For the purpose of NR carrier phase positioning: - the reporting of the carrier phase measurements together with the existing positioning measurements can be considered. - the reporting of the carrier phase-based measurements alone without reporting the existing positioning measurements will be further studied. |
| Ericsson | This may be a bit early to conclude now. Let’s just discuss the conclusion in next meeting. |
| Intel | Same view as Ericsson – especially we should not have phrases like “will be further studied” or “can be considered” (for further study?) in the TR. |

## (H)(Round 3) Proposal 5.1-1a

●    Add the following note to the previous agreement on error modelling of the initial phase:

○    Note: The initial phases of a transmitter for different carriers can be assumed to be independent of each other. Similarly, the initial phases of a receiver for different carriers can be assumed to be independent of each other.

| Company | comments |
| --- | --- |
| Samsung | Small change, I think it is better to say “independent of” instead of “independent with” |
| FL | To Samsung: Okay |
| Qualcomm | Samsung proposed “independent of” (which we agree with), FL agreed, but current text says “independent with”. Possible double-correction when FL copied Samsung comments over from the earlier section to Section7 ? |
| FL | To Samsung/Qualcomm: Yes. My mistake. Corrected back to “independent of”. |

## (H)(Round 3) Proposal 5.1-2

- Add a row of "PRU assumptions" in Table B.4.X.1-1: NR carrier phase positioning enhancements – evaluation scenarios and parameters” in Draft TR 38.859.

  - Note: PRU deployment assumptions may include the assumptions of the number of PRUs, the PRU locations and location errors etc.

| Company | comments |
| --- | --- |
| ZTE | PRU deployment information may include other features, such as PRU density, deployment scope, etc. |
| FL | To ZTE: I assume from the “PRU locations and the PRU locations” has, and in more details, the information related to the “PRU density, deployment scope, etc.”. |
| Nokia/NSB | We assume the note should be updated to say “PRU assumptions may include the number of PRUs, the PRU locations and location errors etc.” |
| FL | To Nokia: Yes, I made the correction as suggested. |

## (H)(Round 3) Proposal 5.2-2

Capture the following TP into TR 38.859 as an evaluation observation (for Section 6.3.2):

The impact of the carrier phase measurement errors related to the initial phase and the frequency errors (the CFO and the oscillator drifts) of the transmitter and the receiver are evaluated during the study item.

- The evaluation results from the companies (e.g., Huawei[1], vivo[2], CATT[6], ZTE[9]) show that these errors may significantly degrade the positioning performance. It is impossible to support centimeter-accuracy positioning if these errors are not removed.

- The evaluation results from the companies (e.g., Huawei[1], CATT[6], ZTE[9]) show that the initial phases of the transmitter and the receiver are removed effectively by the double differential technique with the use of PRU.

- The evaluation results from the companies (e.g., Huawei[1], CATT[6]) show that the frequency errors (the CFO and the oscillator drifts) of the transmitter and the receiver are removed effectively by the double differential technique with the use of PRU.

(Not captured in TR) Note: The number of sources and the references can be further updated in next meeting depending on companies’ updates of simulation results.

| Company | comments |
| --- | --- |
| vivo | For the first sub-bullet, ‘’should that’ may need to be changed to “show that” |
| ZTE | One thing we have to clarify is that the CFO wasn’t eliminated in our simulation, because the positioning error brought by frequency error is negligible as depicted in our evaluation result. Therefore, we suggest revise the proposal as follows: The impact of the carrier phase measurement errors related to the initial phase and the frequency errors (the CFO and the oscillator drifts) of the transmitter and the receiver are evaluated during the study item. - The evaluation results from the companies (e.g., Huawei[1], vivo[2], CATT[6], ZTE[9]) should show that these errors may significantly degrade the positioning performance. It is impossible to support centimeter-accuracy positioning if these errors are not removed. - The evaluation results from the companies (e.g., Huawei[1], CATT[6], ZTE[9]) show that the initial phase and the frequency errors (the CFO and the oscillator drifts) offsets of the transmitter and the receiver are removed effectively by the double differential technique with the use of PRU. - The evaluation results from the companies (e.g., Huawei[1], CATT[6]) show that the frequency errors (the CFO and the oscillator drifts) of the transmitter and the receiver are removed effectively by the double differential technique with the use of PRU. |
| FL | To vivo: Yes. It is corrected. To ZTE: The changes are fine to me. |
| Samsung | Don’t support At this time it is premature to make this conclusion. We should carefuly analyze the simulation results and underlying assumption (e.g, positioning accuracy, deployment density of PRUs, etc) before making such a conclusion. |
| FL | To Samsung: This is not a conclusion, but an observation based on companies’ evaluation. |
| Samsung | To make an observation all of the relavnt under laying assumptions have to be mentioned. For example, what are the assumptions on the location accuracy of the PRUs, density of PRUs etc. Let’s continue the discussion on this one. |
| Ericsson | We agree with Samsung. We need to properly metion all the assumptions under which these results are observed. Also, can we be a bit quantitative? Let’s continue the discussion on this next week. |
| Intel | Agree with Samsung and Ericsson that further discussions are needed. We also need to better align our understanding of the impact from each error source instead of bundling phase and frequency errors together in the context of this proposal. |

## (H)(Round 3) Proposal 5.2-3

Capture the following TP into TR 38.859 as an evaluation observation (for Section 6.3.2):

- The correct resolution of the integer ambiguity is one of the key issues for accuracy improvement by the use of carrier phase positioning. The evaluation results (e.g., Huawei[1],vivo[2], Nokia[3], OPPO[4], CATT[5], Intel[7], ZTE[9], Samsung[17], LGE[19], Qualcomm[21], MediaTek[22]) show that centimeter-level positioning accuracy can be achieved if the integer ambiguity is resolved correctly. But, the accuracy of the carrier phase positioning could be degraded significantly, if the integer ambiguity is not resolved correctly.

(Not captured in TR) Note: The number of sources and the references can be further updated in next meeting depending on companies’ updates of simulation results.

| Company | comments |
| --- | --- |
| vivo | We think the following sentence may not be complete “But, the accuracy of the carrier phase positioning could be significant, or even worse than legacy methods” |
| ZTE | OK, in line with our simulation. |
| FL | To vivo’s comment: The sentence is indeed incomplete. It is changed to: But, the accuracy of the carrier phase positioning could be degraded significantly, or even worse than legacy methods, if the integer ambiguity is not resolved correctly. |
| Nokia/NSB | Sorry for the late comments but we think “worse than legacy methods” is a very vague statement. We prefer to remove it as it is already clear without it that CPP performance will be severely impacted if IA is not resolved. |
| FL | To Nokia: Okay. The text is modified as suggested. |
| Samsung | We have also shown that we can achieve cm-level accuracy by resolving the interger ambiguity. |
| vivo | We prefer to keep the deleted wording and maybe it can be changed to “worse than Rel-16/Rel-17 method ”, to illustrate, with the incorrect integer number, there may be no benefit compared to the existing method. |
| FL | To Samsung: I’ve added Samsung into the source list. To Nokia/vivo: Okay. Oneway to resolve the the issuesmay be we can list the sources that shows the worse than Rel-16/Rel-17 method. I will make the update soon to see if it can be accepted. |
| Ericsson | We prefer to be more quantitative with the observations. For instance, what gains/degrations do companies observed under which assumptions? “degraded significantly” is not very quantitative. So this proposal needs more work and we can discuss it next week. |

## Proposals suggested for email endorsement for 1st Checkpoint (Oct. 14)

Based on the discussions, it seems the following round 3 proposals for 1st Checkpoint (Oct. 14)  (after removing the change marks) should have captured the comments received. So, I am planning to recommend them for email endorsement. However, if any company still have any concern, please include it in the following table for further discussion.

### (H)(Round 3) Proposal 3.1-2

         For UL UE-assisted NR carrier phase positioning, at least consider the carrier phase measured from the UL SRS for positioning purpose.

o    Note: The use of MIMO SRS for positioning purpose is transparent to UE.

| COMPANY | COMMENTS |
| --- | --- |

### (H)(Round 3) Proposal 3.2-1b

Further study the benefits of using the carrier phase measurements of multiple DL positioning frequency layers for NR carrier phase positioning, which may include the impact of the time gap (if any) between the carrier phase measurements of multiple DL PFLs.

         Note 1: The initial phase error and the frequency error for each PFLs can be modelled independently

         Note 2: For the evaluation, the PRS signals of all PFLs of a TRP can be assumed to be transmitted from the same ARP or from different ARPs of the TRP.

         Note 3: The location error for ARPs can be modelled independently.

| COMPANY | COMMENTS |
| --- | --- |

### (H)(Round 3) Proposal 4.3-1

Capture the following TP into TR 38.859 as a conclusion (for Section 6.3.1):

         The impact of multipath/NLOS on NR carrier phase positioning is evaluated during the study item. Based on the study, it is concluded that multipath/NLOS deteriorates the performance of carrier phase positioning and it is necessary to consider multipath mitigation for NR carrier phase positioning.

         The evaluation results for the impact of the multipath/NLOS on NR carrier phase positioning will be presented in Section 6.3.2.

| COMPANY | COMMENTS |
| --- | --- |

### (H)(Round 3) Proposal 5.1-1a

●    Add the following note to the previous agreement on error modelling of the initial phase:

○    Note: The initial phases of a transmitter for different carriers can be assumed to be independent of each other. Similarly, the initial phases of a receiver for different carriers can be assumed to be independent of each other.

| COMPANY | COMMENTS |
| --- | --- |

### (H)(Round 3) Proposal 5.1-2

         Add a row of "PRU assumptions" in Table B.4.X.1-1: NR carrier phase positioning enhancements – evaluation scenarios and parameters” in Draft TR 38.859.

o    Note: PRU deployment assumptions may include the assumptions of the number of PRUs, the PRU locations and location errors etc.

| COMPANY | COMMENTS |
| --- | --- |

# Proposals for 2nd GTW Session (Oct. 17)

### (H)(Round 4) Proposal 3.2-1b

Further study the benefits of using the carrier phase measurements of multiple DL positioning frequency layers for NR carrier phase positioning, which may include the impact of the time gap (if any) between the carrier phase measurements of multiple DL PFLs.

- Note 1: The initial phase error and the frequency error for each PFLs can be modelled independently

- Note 2: For the evaluation, the PRS signals of all PFLs of a TRP can be assumed to be transmitted from the same ARP or from different ARPs of the TRP.

- Note 3: The location error for ARPs can be modelled independently.

- Note 4: The timing errors of the PFLs may not be the same for PFLs in different bands or frequency ranges.

| COMPANY | COMMENTS |
| --- | --- |
| FL | vivo made the comment in email: ### First, based on the email discussion, the note has been suggested in the FL response but is not captured in the current proposal - Note 3: The timing errors of the PFLs may not be the same for PFLs in different bands or frequency ranges. ### For the second comment, we still have some concerns about the response about “if any” in the FL response, ie, “or a UE support dual connectivity, it may process two PFLs simultaneously. I assume adding “if any” covers both cases.” We would like to confirm that FL assumes “two simultaneously PFLs” is supported for future release for dual connectivity？ If is, we are not sure we needs to evaluate the feature in the stage. |
| FL | To vivo: In my view, for “if any” case, I assume any UE that have two Rx RF chains should be able to support receiving “two simultaneously PFLs”. I understand we have not introduced the capability in R17. But, since we are study it for R18. Thus, I’d assume it is meaningful to evaluate it. But, I don’t have a strong view one way or the other. Let us discuss it online to see other companies’ view. |

### (H)(Round 4) Proposal 5.2-2a

- To minimize the simulation effort and also for better comparison of the positioning performance from different companies, it is encouraged to focus on the use of the following evaluation assumptions .

  - baseline scenarios: InF-SH and InF-DH

  - FR1: BW 100 MHz

  - ARP error: T= [0, 1]cm

  - CFO: 0, [-30, +30] Hz (FR1, UE); [-10, +10] Hz (for each TRP, FR1),

  - PCO: a = [0, 3].

  - [TRP synchronization=0, 50ns]

- Note: Evalution results with other simulation assumptions agreed in the previous meeting are also welcome.

| Company | comments |
| --- | --- |

### (H)(Round 4) Proposal 5.2-2b

When providing the evaluation results, it is encouraged to provide the following information with their observations.

- The impact of each error sources (Initial Phase, CFO, Oscillator-drift, PCO, [TRP synchronization]:

  - Each of above error sources is evaluated separately, assuming there is no other errors.

- The CDF values: at least the horizontal accuracy of 50% and 80% UEs:

  - It would be better to have the results of both NR carrier phase positioning and the DL- TDOA/UL-TDOA, if DL- TDOA/UL-TDOA for comparison.

- Additional important assumptions and/or methods used in the evaluation: e.g., whether the double differential technique, and/or the carrier phases of multiple (sub)carriers:

  - If not mentioned, it is assumed “the double differential technique” and “the carrier phases of multiple (sub)carriers” are not used

| Company | comments |
| --- | --- |

### (H)(Round 4) Proposal 5.2-2c

- Companies are encouraged to provide at least one evaluation result with all error sources (Initial Phase, CFO, Oscillator-drift, PCO, [TRP synchronization]) being modelled.

| Company | comments |
| --- | --- |

### (H)(Round 4) Proposal 3.3-1

- Study the benefits of using the carrier phases measured from the DL PRS transmitted from multiple TRP Tx antennas or antenna panels for NR carrier phase positioning and the potential impact on the specification.

| Company | comments |
| --- | --- |

# References

- R1-2208455	Discussion on NR carrier phase positioning	Huawei, HiSilicon

- R1-2208650	Discussion on carrier phase measurement enhancements	vivo

- R1-2208736	Views on improved accuracy based on NR carrier phase measurement	Nokia, Nokia Shanghai Bell

- R1-2208774	Discussion on improved accuracy based on NR carrier phase measurement	China Telecom

- R1-2208801	Discussions on Carrier Phase Measurement for NR Positioning	OPPO

- R1-2208983	Discussion on improved accuracy based on NR carrier phase measurement	CATT

- R1-2209059	Carrier phase positioning in NR systems	Intel Corporation

- R1-2209152	Discussion on NR carrier phase positioning	NEC

- R1-2209215	Discussion on carrier phase measurement based positioning	ZTE

- R1-2209293	Improved accuracy based on NR carrier phase measurement		xiaomi

- R1-2209343	Discussion on carrier phase positioning	CMCC

- R1-2209395	On NR carrier phase measurements	Lenovo

- R1-2209489	Discussion on positioning based on NR carrier phase measurement	InterDigital, Inc.

- R1-2209534	NR carrier phase measurements for positioning 	Faunhofer IIS, Fraunhofer HHI

- R1-2209543	Discussion on double difference method and gNB synchronization	Locaila

- R1-2209546	Views on NR carrier phase measurement for positioning accuracy enhancement	IIT Kanpur, CEWiT

- R1-2209738	Discussion on NR Carrier Phase Measurement	Samsung

- R1-2209785	Views on improved accuracy based on NR carrier phase measurement	Sharp

- R1-2209805	Discussion on OFDM based carrier phase measurement in NR	LG Electronics

- R1-2209909	Discussion on improved accuracy based on NR carrier phase measurement	NTT DOCOMO, INC.

- R1-2209992	Phase Measurements in NR Positioning	Qualcomm Incorporated

- R1-2210099	On NR carrier phase measurement	MediaTek Inc.

- R1-2210177	Improved accuracy based on NR carrier phase measurement	Ericsson

- RP-213588 	Revised SID on Study on expanded and improved NR positioning, Intel (Email discussion moderator)

- R1-2205165 	FL Summary #2 Carrier Phase Measurements, Moderator (CATT)

- R1-2208206 	FL Summary #3 Carrier Phase Measurements, Moderator (CATT)

# Observations and proposals in the contributions [1-23]

- R1-2208455 Discussion on NR carrier phase positioning Huawei, HiSilicon Observation 1: the impact of CFO on carrier phase positioning is negligible. Observation 2: Double differential carrier phase can eliminate the random initial phase and oscillator-drift of the receiver and the transmitter. Observation 3: The ARP error deteriorates the carrier phase positioning accuracy even the double differential method (with PRU) is used. Observation 4: Double differential carrier phase positioning technique can eliminate some influence of PCO. - Observation 5: Multi-frequency carrier phase positioning method is robust to gNB ARP position errors and can achieve centimeter-level positioning accuracy. - Proposal 1: Rel-18 shall target the accuracy requirement of 1cm@50% for positioning using carrier phase measurement. - Proposal 2: The double differential carrier phase and multi-frequency carrier phase should be considered for carrier phase positioning. - Proposal 3: For carrier phase jointly with DL-TDOA, phase difference between the measuring TRP and reference TRP can be reported by UE. - Proposal 4: For carrier phase jointly with UL-TDOA, relative phase can be reported by TRP. - Proposal 5: For UL carrier phase positioning method, the TRP can report carrier phase measurements on multiple frequencies or distance information computed from multi-frequency carrier phase measurements. - Proposal 6: For DL LMF-based carrier phase positioning method, the UE can report carrier phase measurements on multiple frequencies or distance information computed from multi-frequency carrier phase measurements.

- R1-2208650 Discussion on carrier phase measurement enhancements vivo Observation 1 • For the carrier phase measurement in the frequency domain with option B, the performance accuracy is 0.43 radian @90% in InF- SH. • For the carrier phase measurement in the frequency domain with option B, the performance accuracy is 1.36 radian @90% in InF- DH. Observation 2: • For the carrier phase measurement in the time domain with option C, the performance accuracy is 1.47 radian @90% in InF- SH. • For the carrier phase measurement in the time domain with option C, the performance accuracy is 4.41 radian @90% in InF- DH. Observation 3: • For the carrier phase measurement in the frequency domain of multipath mitigation CIR with option D, the performance accuracy is 0.23 radian @90% in InF- SH. • For the carrier phase measurement in the frequency domain of multipath mitigation CIR with option D, the performance accuracy is 0.64 radian @90% in InF- DH. Observation 4: • For the carrier phase measurement, the performance of carrier phase measurement degraded significantly in the multiple path/NLoS scenario. Observation 5: • For the carrier phase measurement accuracy, the performance of carrier phase measurement in the frequency domain of multipath mitigation CIR with option D is the best compared to the carrier phase measurement in the frequency domain and time domain. Observation 6: • For the carrier phase positioning (case 101) with option D (carrier phase measurement in frequency domain of multipath mitigation CIR) and ideal integer cycles, the accuracy is 0.019m @90% and 0.0012@50% in InF- SH. o Nearly 4 times better performance compared to TDOA @90% o Nearly 30 times better performance compared to TDOA @50% • For the carrier phase positioning (case 102) with option D (carrier phase measurement in frequency domain of multipath mitigation CIR) and finer integer cycles estimation with cost function, the accuracy is 0.09@90% and 0.0014m @50% in InF- SH. o The performance is similar to TDOA's @90% o Nearly 30 times better performance compared to TDOA @50% Observation 7: • For the carrier phase positioning (case 201) with option D (carrier phase measurement in frequency domain of multipath mitigation CIR) and ideal integer cycles, the accuracy is 0.023m @90% and 0.003@50% in InF- DH. o Nearly 10 times better performance compared to TDOA @90% o Nearly 18 times better performance compared to TDOA @50% • For the carrier phase positioning (case 202) with option D (carrier phase measurement in frequency domain of multipath mitigation CIR) and finer integer cycles estimation with cost function, the accuracy is 0.82m @90% and 0.007@50% in InF- DH. o The performance is worse than TDOA @90% o Nearly 8 times better performance compared to TDOA @50% Observation 8: • For InF-SH scenario, without additional error, the positioning accuracy @50% with carrier phase positioning can be improved to within 1cm. • For InF-DH scenario, without additional error, the positioning accuracy @50% with carrier phase positioning can be improved to within 1cm. Observation 9: • With residual CFO, the positioning performance of carrier phase positioning degrades and may be worse than TDOA only positioning. o With UE residual CFO 30Hz and TRP residual CFO 10Hz, the accuracy drops to 0.56m @90% in InF- SH. o With UE residual CFO 100Hz and TRP residual CFO 10Hz, the accuracy drops to 0.66m @90% in InF- SH Observation 10: • With ARP error, the positioning performance of carrier phase positioning degrades significantly and was worse than TDOA only positioning. o With ARP error with T1=1cm, the accuracy is 0.28m @90% in InF- SH. o With ARP error with T1=5cm, the accuracy is 0.41m @90% in InF- SH Observation 11: • With PCO error, the positioning performance of carrier phase positioning degrades. But the performance is still better than TDOA only positioning @50% at least. Observation 12: • With initial phase error within [0, 2pi], the positioning performance of carrier phase positioning degrades significantly. And the performance will be worse than TDOA only positioning. - To reuse the existing signal (SRS, or PRS) for carrier phase measurement. - The carrier phase measurement is the phase of a fractional part of the propagation time. - NR carrier phase measurement can be the carrier phase difference between the Transmission Point (TP) and the reference TP - If carrier phase positioning is not a standalone positioning method, the carrier phase measurement can share existing LoS indication with existing positioning methods. - For multipath mitigation, the carrier phase measurement in the frequency domain with multipath mitigation（option D）can be adopted. - The carrier phase positioning cannot be seen as a standalone positioning.

- R1-2208736 Views on improved accuracy based on NR carrier phase measurement Nokia, Nokia Shanghai Bell Observation 1: In UL CP the PRU needs to transmit the SRS for positioning along with the target UE. Proposal 1: No new values of X are added for the initial phase offset model between transmitter and receiver. Propsoal 2: Adopt the PCO error model as the final error model on PCO for the SI. Proposal 3: For parameters X and Y leave it up to individual companies to provide the specific values used in their evaluations. Observation 2: The phase measurement error is less than 1 cm for 80 percent of UEs and the error is less than 4 cm error for 90 percent of the UEs in InF-SH model. Observation 3: Significantly improved accuracy over Rel-17 is possible with NR CP methods. Proposal 4: Include the above results in the TR. Proposal 5: RAN1 to conclude that NR CP provides significant accuracy gains over existing techniques. Proposal 6: RAN1 to study and identify necessary measurements to support CP positioning for both DL-based positioning and UL-based positioning. Proposal 7: RAN1 to study methods for identifying independent taps and/or methods to aid a positioning receiver to determine and report measurements of independent channel taps that correspond to different wireless propagation paths. Observation 4: The virtual wavelength method may provide the reduced search space of integer ambiguity variables Proposal 8: RAN1 to study physical layer procedure to support virtual wavelength method to address interger ambiguity problem. Observation 5: The antenna array phase center may not always aligned with the physical antenna reference point and it affects to phase and timing measurement accuracy. Proposal 9: RAN1 needs to study antenna phase center offset impact on the carrier phase positioning accuracy and identify potential solutions. Observation 6: PCO impacts also the SRS transmission of the PRU. Proposal 10: RAN1 should study the impacts and potential mitigation techniques of antenna phase center offset on PRU transmissions for UL CP. Proposal 11: RAN1 to study the CP positioning performance impact by introducing PRU for UE-assisted positioning, and identify the necessary physical layer procedure including aligning measurements by PRU and target UE. Proposal 12: RAN1 to study/identify the CP positioning performance impact by introducing PRU for UE-based positioning, and identify the necessary physical layer procedure. Proposal 13: Discuss SL carrier phase only under AI 9.5.1.2 during RAN1#110-bis. Observation 7: The ranging error that introduced due to the estimation error in phase is less than 1cm for 80% of realizations for TDL channels (D and E) with dominant LOS path. Observation 8: The first arrival path of TDL model follows the Ricean distribution, resulting in inherent phase interference introduce due to TDL channel model. An agreement on the treatment of Rayleigh path of the first arrival path is necessary for the LOS path phase study.

- R1-2208774 Discussion on improved accuracy based on NR carrier phase measurement China Telecom Observation 1: the initial phase offset between the transmitter and the receiver directly destroy the accuracy of distance estimation, which is bound to be eliminated. Observation 2: the initial phase offset for all the subcarriers of OFDM signals is identical, while is different for carriers in the single carrier signal scheme. Therefore, the elimination of the initial phase offset for OFDM is easier than single carrier signal. Proposal 1: reuse the existed DL-PRS or UL SRS-Pos for phase based positioning. Proposal 2: The possible modification on the pattern of reference signal especially the supplement of the short transmission period can be considered. Proposal 3: apart from the initial phase offset and the CFO, the carrier phase measurement model for positioning should mainly consider the time offset between the transceiver. Proposal 4: support multiple carrier phase measurements, because it is benefit for integer ambiguity removal, and contribute to performance improvement of distance estimation. Proposal 5: for the single carrier signal mechanism, support the round trip measurements of phase difference on both sides in order to eliminate the initial phase offset.

- R1-2208801 Discussions on Carrier Phase Measurement for NR Positioning OPPO Proposal 1: The NR phase measurement-based positioning shall be based on the existing DL PRS and SRS for positioning. Observation 1: Integer ambiguity in carrier phase measurement would impair the performance of NR positioning significantly. Observation 2: Combining carrier phase measurement with DL-TDOA method does not provide better performance than the DL-TDOA method. Observation 3: Using other positioning method to resolve the integer ambiguity could cause error of integer wavelengths. Proposal 2: Study using carrier phase measurement of multiple carrier frequencies to resolve the issue of integer ambiguity. Proposal 3: The UE can measure and report carrier phase of multiple REs in DL PRS and the UE reports the carrier phase measurement and positioning measurement results of legacy positioning method. Proposal 4: Study measuring and reporting the relative phase of different Tx antenna ports of TRP for positioning. Proposal 5: Study the PRS enhancement for supporting measuring relative phase of Tx antenna ports of TRP Proposal 6: The TRP measures and reports carrier phase of multiple REs in SRS for positioning and the TRP also reports the carrier phase measurement and positioning measurement results of legacy positioning method.

- R1-2208983 Discussion on improved accuracy based on NR carrier phase measurement CATT Observation 1: Double differential technique with PRU can effectively eliminate the impact of the TRP and UE clock offsets, the TRP and UE initial phase offsets, CFO and the oscillator-drift. Observation 2: Carrier phase measurements from multiple carrier frequencies are helpful for fast resolution of the integer ambiguity. Observation 3: The impact of multipath for NR CPP may be mitigated by UE/TRP implementation by the following approaches: - Mitigating the impact of multipath on the measurements by the improvement of hardware and software design of the antenna and receiver. - Mitigating the impact of multipath on the positioning solution by advanced data processing and positioning algorithms. Observation 4: NR carrier phase measurements can be used to get carrier phase-smoothed DL-TDOA, UL-TDOA or Multi-RTT measurements, which have higher accuracy than raw measurements. Observation 5: NR carrier phase measurement of a single measurement instance can be obtained by either the time domain approach or the frequency domain approach. Observation 6: The CRLB of the carrier phase measurement depends on the carrier frequency while the CRLB of the TOA measurement depends on the system bandwidth, which explains why NR CPP can achieve much higher positioning accuracy than TDOA positioning. Observation 7: For InF-SH scenario, the CPP positioning accuracy can reach 0.0121 m for 90% UEs with the carrier phase measurements from a single carrier frequency without error sources. Observation 8: For InF-SH scenario, the CPP positioning accuracy can reach 0.0069 m for 90% UEs with 3 carrier frequencies which is more precise than 0.1269 m of TDOA positioning without error sources. Observation 9: For InF-DH scenario, the carrier phase positioning accuracy can reach 0.034 m for 90% UEs with the carrier phase measurements from 3 carrier frequencies without error sources. Observation 10: It is feasible to reach the centimeter-level positioning accuracy by the use of carrier phase measurements from multiple carrier frequencies for both InF-SH and InF-DH scenarios. Observation 11: For InF-SH scenario, the carrier phase positioning accuracy can reach 0.0178 m for 90% UEs with 3 carrier frequencies which is more precise than 0.154 m of TDOA positioning with ARP error of 1cm. Observation 12: For InF-SH scenario, the carrier phase positioning accuracy can reach 0.069 m for 90% UEs with 3 carrier frequencies which is still more precise than 0.163 m of TDOA positioning with ARP error of 5 cm. Observation 13: The carrier phase positioning accuracy will degrade when ARP error increases. Observation 14: PCO error with example 2 degrades the performance of CPP slightly. The carrier phase positioning accuracy with PCO error can reach 1 cm level for 90% UEs. Observation 15: For InF-SH scenario, the horizontal positioning accuracy of NR CPP is is 0.1144 m for 90% UEs with single differential technique, and 0.012 m for 90% UEs with double differential technique. Observation 16: Double differential technique can fully eliminate the influence of frequency error, while single differential technique isaffected by frequency error. Observation 17: For InF-SH scenario, the horizontal positioning accuracy of NR CPP is 0.1512 m for 90% UEs with single differential technique, and 0.0121 m for 90% UEs with double differential technique. Observation 18: Double differential technique can fully eliminate the influence of initial phase offset, while single differential technique is affected by initial phase offset. Observation 19: Double differential technique can effectively eliminate the impact of the TRP and UE clock offsets, the TRP and UE initial phase offsets, CFO and the oscillator-drift. Observation 20: Multi-carrier NR carrier phase positioning is robust to TRP ARP error. Observation 21: The existing Rel-16 DL PRS and SRS for positioning can be used as the reference signals for supporting centi-meter level accuracy NR carrier phase positioning based on the evaluation results. Proposal 1: Double differential technique with PRU should be supported for NR carrier phase positioning. Proposal 2: Support UE/TRP to report the carrier phase measurements corresponding to multiple carrier frequencies of different carriers, and/or corresponding multiple subcarrier frequencies of the same or different carriers for NR carrier phase positioning. Proposal 3: For the indication of LOS/NLOS for Rel-18 NR CPP, the LOS/NLOS indication in Rel-17 can be the starting point. Proposal 4: Using carrier phase measurements to smooth existing timing measurements can be considered in Rel-18. FFS: The impact on the specification. Proposal 5: For evaluation of NR CPP in Rel-18 SI, initial random phases of TRPs/UEs should be considered to be TRP/UE-specific, e.g., it is the same for all carrier phase measurements of a carrier frequency for a specific TRP/UE. Proposal 6: The impact of TRP ARP location error should be mitigated in order to improve the carrier phase positioning accuracy. Proposal 7: The existing Rel-16 DL PRS and SRS for positioning can be used as the reference signal for supporting NR carrier phase positioning in Rel-18. - Note: Further optimization of the reference signal on top of existing R16 DL PRS and SRS for positioning may also be considered in Rel-18 or a future release. Proposal 8: The candidate measurements for carrier phase positioning may include: - DL/UL carrier phase measurements corresponding to the carrier frequency of a carrier, or corresponding to one or more subcarrier frequencies within the bandwidth of the carrier, between a Tx antenna of a UE/TRP and a Rx antenna of a TRP/UE - Single differential DL carrier phase measurement between different Tx antennas of a TRP or different TRPs, and a Rx antenna of a UE; - Single differential DL carrier phase measurement between different UE Rx antennas and a Tx antenna of the TRP; - Single differential UL carrier phase measurement between different UE Tx antennas and the same Rx antenna of the TRP. Proposal 9: The candidate information in the measurement reporting for carrier phase positioning may include: - Measurement quality indications, which include one or a combination of LOS/NLOS indicator, Rician factor, SINR and variance of CPP measurement. Proposal 10: The existing physical layer procedures for NR positioning can be re-used for supporting NR carrier phase positioning.

- R1-2209059 Carrier phase positioning in NR systems Intel Corporation - Study LOS/NLOS links classification for the carrier phase measurements to improve the accuracy of the carrier phase positioning methods. - As a starting point, consider use of Rel-17 LOS/NLOS indicator reporting for NR carrier phase positioning methods. - Clarify that the initial phase offsets can be modelled separately at transmitter and receiver nodes as independent random variables, each uniformly distributed within [0, 2pi]. - Study introduction of RSTD measurement based on carrier phase estimates of the DL PRS signal at the kth subcarrier frequency for the target and reference TRPs normalized by the carrier frequency (ωc + ωk). - FFS: If/how to introduce reporting of carrier phase measurements directly. - Study initial phase calibration of the TX generators for the reference and target TRPs by using the RSTD measurements obtained with the Positioning Reference Unit (PRU) with known coordinates. - Study carrier phase integer ambiguity resolution using subcarrier measurements within the DL PRS signal bandwidth. - For carrier phase integer ambiguity resolution, a multi-step process to identify coarse timing based on TOA estimate from processing multiple subcarriers and then using CPP for resolution of the fractional part of the carrier wavelength can be considered as a baseline approach. - For high accuracy positioning, study TX beamforming phase difference estimation for reference and target TRPs for mitigation of effect of PCO and ARP errors for DL RSTD based methods. - Study carrier phase-based azimuth and zenith angles of departure measurement for the multi element transmit antenna array for the DL-AOD positioning method. - Study RSTD measurement for the multi element RX antenna array, which requires RX beamforming phase difference estimation for the reference and target TRPs. - Study introduction of RTOA measurement based on the carrier phase estimates of the UL SRS signal at the kth subcarrier frequency normalized by the carrier frequency (ωc + ωk). - FFS: If/how to introduce reporting of carrier phase measurements directly. - Study initial phase calibration of the RX generators for the reference and target TRPs by using the measurements obtained with the Positioning Reference Unit (PRU) with known coordinates. - Study RTOA measurement for the multi element TX antenna array, which requires TX beamforming phase difference estimation for the reference and target TRPs. - Study RTOA measurement for the multi element RX antenna array, which requires RX beamforming phase difference estimation for the reference and target TRPs. - Usage of NLOS links for the carrier phase measurements may lead to positioning error due to an extra time delay associated with the propagation through the reflected path as well as the additional phase shift caused by the reflection from the obstacle. - Differential processing between a receiving node and multiple transmitters can be used to eliminate the unknown initial phase offset at receiver. - For NR CPP in the DL, RSTD measurement based on carrier phase estimates of the DL PRS signal at the kth subcarrier frequency for the target and reference TRPs normalized by the carrier frequency (ωc + ωk) can be introduced. - The accuracy of RSTD measurement based on carrier phase differences can be improved by averaging over N subcarriers comprising the DL PRS signal spectrum. - The initial phase difference between the TX generators of the reference and target TRPs needs to be calibrated to perform accurate RSTD measurements based on carrier phase differences. - The initial phase calibration of the TX generators for the reference and target TRPs can be performed by using RSTD measurements obtained with the Positioning Reference Unit (PRU) with known coordinates. - The initial phase calibration of the TX generators for the reference and target TRPs allows to improve the accuracy of the carrier phase based RSTD positioning. - The larger the carrier frequency, the more accurate estimate can be achieved for RSTD carrier phase-based measurement. - The advantage of accuracy for the high carrier frequency comes at the expense of the integer ambiguity issue for RSTD carrier phase-based measurement. - The carrier phase ambiguity issue can be resolved by using the subcarrier measurements within the DL PRS signal bandwidth. - The phase estimate and corresponding time measurement obtained using subcarrier measurements is not sensitive to the phase difference of the TX generators and calibration using PRU is not required in that case. - The accuracy of the phase estimate and corresponding time measurement is higher for the lager subcarrier spacing. - For the multi element transmit antenna array, the additional phase shift associated with the TX beamforming (and modelled as PCO) may have an impact on the carrier phase RSTD measurement and needs to be taken into consideration. - Additional phase shifts caused due to TX beamforming (PCO) and ARP errors have an additive effect – while ARP errors may be of static nature, phase shifts due to TX beamforming may be of more time-varying nature. - For high accuracy, RSTD measurements based on carrier phase estimates for the multi element TX antenna array requires TX beamforming phase difference estimation for the reference and target TRPs. - For the multi element RX antenna array, the additional phase shift associated with the RX beamforming may have an impact on the carrier phase RSTD measurement and needs to be taken into consideration. - The RSTD measurement for the multi element RX antenna array requires RX beamforming phase difference estimation for the reference and target TRPs. - For NR CPP in the UL, RTOA measurement based on carrier phase estimates of the UL SRS signal at the kth subcarrier frequency normalized by the carrier frequency (ωc + ωk) can be introduced. - The initial phase calibration of the RX generators for the reference and target TRPs can be performed by using measurements obtained with the Positioning Reference Unit (PRU) with known coordinates. - The initial phase calibration of the RX generators for the reference and target TRPs allows to have a common reference time for RTOA measurements across all TRPs. - For the multi element transmit antenna array, the additional phase shift associated with the TX beamforming may have an impact on the carrier phase RTOA measurement and needs to be taken into consideration. - The RTOA measurement for the multi element TX antenna array requires TX beamforming phase difference estimation for the reference and target TRPs. - For the multi element RX antenna array, the additional phase shift associated with the RX beamforming may have an impact on the carrier phase RTOA measurement and needs to be taken into consideration. - The RTOA measurement for the multi element RX antenna array requires RX beamforming phase difference estimation for the reference and target TRPs.

- R1-2209152 Discussion on NR carrier phase positioning NEC Observation 1: Integer cycle of phase with different carrier frequency represents different linear propagation distance. Proposal 1: How to quantitate the received carrier phase (also accompany with integer part) in the specification and report it from UE side should be further studied.

- R1-2209215 Discussion on carrier phase measurement based positioning ZTE Observation 1: In InF-SH, assuming receiver can get perfect phase between itself and the transmitter, very high positioning accuracy can be achieved, especially under a large searching range of integer values. Observation 2: PRU can alleviate the random initial phase error at gNB side from (0~2π) to about (0~0.02*2π) (i.e., 98% reduction). Observation 3: The carrier phase error from initial random phases at both TRP and UE sides can be mitigated by dual differential operation with the help of PRU. Observation 4: The carrier phase error within one site can be removed by single differential operation for the case of carrier phase method from multiple frequency sections in one carrier. Observation 5: An appropriate scope of integer should be given for integer search. Observation 6: The maximum ΔΦ is 0.0067*2π=0.0419 Rad for CFO=100Hz, BW=100MHz, SCS=30kHz, CombSize=2. Observation 7: A small frequency error has minor impact on CPP even under perfect Φ. Observation 8: The phase error ΔΦ=0.0067*2π=0.0419 Rad from CFO is much less than the phase estimation error@CDF=90% which is about 0.037*2π=0.2325 Rad. Observation 9: A small frequency error has negligible impact on CPP under practical Φ. Observation 10: More phase estimation error cause larger location error in the case without help of PRU or differential operation. Observation 11: The maximum Δd is 0.1414m (2D) or 0.1732m (3D) for ARP error=5 cm. Observation 12: The corresponding ΔΦ is Δdmax /λ=((0.1414/0.0857) mod 1.0)*2 π=0.6511*2 π=4.091 Rad (for 2D) for single Φ@3.5GHz, or 0.0472*2 π=0.2966 Rad (for 2D) for dual Φ@100MHz bandwidth. Observation 13: A small ARP error has large impact on CPP under perfect Φ, especially, for a short wave length. Observation 14: A long virtual wave length is helpful for alleviating the impact from ARP error. Observation 15: A small ARP error has significant impact on CPP under practical Φ, especially, for a short wave length. Observation 16: The UE can report the carrier phase with corresponding phase uncertainty, to evaluate the rationality of other positioning techniques. Proposal 1: For multiple segments CP report within a carrier, center frequency or λ should be informed for each of each segment. Proposal 2: The scope of integer N should be configured / reported for carrier phase based positioning. Proposal 3: Phase or phase difference report should be supported at least for TDOA. Proposal 4: The same Phase Error Group(PEG) should be used for CP measurement. Proposal 5: The scope of integer N in OTDOA could be configured based on the measurement uncertainty. Proposal 6: The scope of integer N in AOA/AOD could be configured based on the angle measurement uncertainty. Proposal 7: The scope of integer N for different carrier could be selected based on the frequency feature. Proposal 8: The number of carrier phases to be reported could be determined based on the positioning accuracy and measurement overhead requirements.

- R1-2209293 Improved accuracy based on NR carrier phase measurement xiaomi - Proposal 1: Study multi-carrier based carrier phase positioning as the potential solution for integer ambiguity based on UE capability. - Proposal 2: Support positioning report containing the carrier phase and the corresponding carrier frequency for multiple carrier frequencies. - Proposal 3: Study relative carrier phase measurement between RSs from different TRPs. - Proposal 4: Study PRU assisted carrier phase measurement for phase error mitigation. - Proposal 5: Study to report the Doppler parameters combined with the carrier phase value, or to report the adjusted carrier phase value based on Doppler parameters. - Proposal 6: Support the carrier phase for first path and additional path, and to report the likelihood of LoS path by LoS/NLoS indicator for each path carrier phase.

- R1-2209343 Discussion on carrier phase positioning CMCC Observation 1: The carrier phase measurement can be obtained in frequency domain or time domain of the base band signal. Observation 2: By estimating the first path, the carrier phase measurement in time domain can help suppress the impact of multipath. Proposal 1: For carrier phase positioning, the double differential carrier phase measurement should be considered to eliminate the inter TRP carrier phase synchronization errors. - The double differential carrier phase measurement can be obtained by the use of PRU. Proposal 2: At least the following solutions to solve the integer ambiguity are considered feasible: - Fast search of integer ambiguity of the carrier phase obtained from more than one frequency; - Fast search of integer ambiguity of the carrier phase associated with the legacy positioning techniques.

- R1-2209395 On NR carrier phase measurements Lenovo Observation 1: The accuracy requirements may be more stringent compared to past methods, based on the carrier wavelength, however the requirements may be different based on indoor and outdoor scenarios. Observation 2: The study of new RS designs for carrier phase may lead to larger specification impact. Observation 3: Re-using the current OFDM-based PRS/SRS design may provide some challenges due to the lack continuity in time to accurately estimate the carrier phase. The proposals are summarized as follows: Proposal 1: RAN1 to study and define the performance requirements of carrier phase positioning in terms of horizontal and vertical accuracy requirements based on the FR1/FR2 carrier wavelengths as well as distinguishing the accuracy according to different environments, e.g., indoor vs outdoor. Proposal 2: To improve the positioning performance of NR by applying the carrier phase positioning method, the PRS waveform should be continuous. This is enabled by performing raised cosine technique over the PRS waveform. Proposal 3: To improve the positioning performance of NR by applying the carrier phase positioning method, contiguous mapping of PRS generating sequence in time domain needs to be studied. Proposal 4: The range of values of the frequency offset koffset and the number of contiguous symbols LPRS for accurate carrier phase measurements need to be further investigated. Proposal 5: RAN1 to further study the support of narrower bandwidth configurations for carrier phase-based positioning. Proposal 6: Support the use of Positioning Reference Units (PRUs) to help mitigate the following carrier phase errors: - To assist in providing an upper and lower bound on the integer ambiguity (IA) based on the selection of PRUs in the vicinity of the target-UE, which reduces IA search space. - Initial phase offset errors Proposal 7: Time-varying residual CFO has a negative impact on the transmission and reception phase of PRS(DL)/SRS(UL), which affects the accuracy of the carrier phase measurements, and additional residual CFO tracking, and compensation techniques could be studied for the carrier phase positioning case.

- R1-2209489 Discussion on positioning based on NR carrier phase measurement InterDigital, Inc. Proposal 1: Postpone the study of SL carrier phase positioning to a future release beyond Rel. 18. Proposal 2: Study transmission and reception modes (e.g., 2 TX 1RX, 1Tx 2 RX) for phase-difference based positioning. Proposal 3: For DL-based positioning, at least the following positioning methods should be supported: NR carrier phase positioning and joint carrier phase and DL-RSTD. Proposal 4: Study assistance information that allows the UE or network to mitigate unknown phase offset in phase measurements, e.g., phase error group. Proposal 5: Study new PRS designs for carrier phase measurements for UL and DL. Proposal 6: A method to differentiate multiple PRSs, if they are transmitted at the same time-frequency resource, should be identified. Proposal 7: Support the UE to select and report the reference for phase difference measurements. Proposal 8: Support the UE to report the estimated number of cycles of a received carrier. Proposal 9: Support the UE to perform PRS measurement in multiple frequencies in one measurement window. Proposal 10: Support the UE to report the phase measurement in multiple frequencies in one measurement reporting. Proposal 11: Study the condition under which carrier phase should be measured and reported based on e.g., - LOS/NLOS value - RSRP Proposal 12: Support PRU for NR carrier phase-based positioning. Proposal 13: Support both UE-based and UE-assisted NR carrier phase positioning. Proposal 14: Study a mechanism to transfer measurements from the PRU to the UE in UE-based positioning.

- R1-2209534 NR carrier phase measurements for positioning Faunhofer IIS, Fraunhofer HHI Observation 1: Majority of the proposed methods mainly depend on the phase measurement corresponding to LOS path. Observation 2: Certain methods may have different constraints which is related to feasibility of the method. If the feasibility is clarified, the corresponding specification impact is expected to be straightforward. Observation 3: The phase change resulting from movement may be different for each multipath component. Observation 4: Compared to the TDoA (time difference of arrival) the PDoA (phase difference of arrival) shows a significant higher accuracy and a much lower sensitivity to the reduced bandwidth, even for scenarios considering multipath components with the opposite phase gradient as the LOS signal. Proposal 1: The carrier phase shall be determinable for different parts of the CIR , wherein each part may represent a path related to a multipath cluster reporting of the channel impulse response and shall be measured in the delay domain. Proposal 2: Support reporting the phase measurements from the delay domain for the first and additional paths (N-path reporting for the phase). Proposal 3: Consider one or more of the following options for the delay domain phase measurement: - Option 1: The first path and N-path reporting is extended by a phase value for each reported path - Option 2: A magnitude and phase of the complex valued correlation value associated to the path is reported. - Option 3: The full complex valued CIR for a given length is reported. - Option 4: Relevant parts of the complex valued CIR are reported. Proposal 4: Support phase based smoothing for DL-TDoA and UL-TDoA methods in Rel-18. Consider the signaling of the coherency transmission status to the UE (UE based positioning) or to the LMF to notify on the applicability of phase based smoothing. Proposal 5: Support double difference method for phase based positioning enhancements utilizing PDoA measurements for RTT measurements without synchronization of the UE and the TRP. Proposal 6: In case of carrier aggregation using several bandwidth limited signals multipath effects causing phase ripple per BWP shall be taken into account for the evaluation of the performance of carrier phase measurements.

- R1-2209543 Discussion on double difference method and gNB synchronization Locaila Proposal 1) Change the CFO metric in Table A.3-1 of current TR38.859 as below table. Table A.3-1: Assumptions for evaluation of NR carrier phase positioning Assumptions; Value. Scenarios; - Baseline: InF-SH, InF-DH - Optional: Indoor Open Office, Umi, Highway scenarios - Note 1: Other evaluation scenarios are not precluded - Note 2: Existing Rel-17 DL/UL reference signals for the Uu interface are to be used for the Highway scenario.. Frequency errors – Note 1; Ideal; Practical. Initial residual CFO (is the same for one measurement instances [or multiple phase measurement instances]); 0 (UE/TRP); Uniform distribution within: • [-30, +30] Hz (FR1, UE), [-100, +100] Hz (FR1, UE), • [-120, +120] Hz (FR2, UE), [-400, +400] Hz (FR2, UE), • [-10, +10] Hz (for each TRP, FR1), • [-40, +40] Hz (for each TRP, FR2). • [-100, +100] Hz (FR1, UE, 1GHz) • [-400, +400] Hz (FR1, UE, 4GHz) • [-3K, +3K] Hz (FR2, UE, 30GHz) • [-7K, +7K] Hz (FR2, UE, 70GHz) • [-20, +20] Hz (FR1, TRP, 1GHz) • [-80, +80] Hz (FR1, TRP, 4GHz) • [-600, +600] Hz (FR2, TRP, 30GHz) • [-1.4K, +1.4K] Hz (FR2, TRP, 70GHz). Oscillator-drift (is the same for one or multiple phase measurement instances for positioning fix); 0 (UE/TRP); Uniform distribution within: • [-0.1, 0.1] ppm (UE) • [-0.02, +0.02] ppm (each TRP) within measurement duration. Proposal 2) RAN1 should study the traffic burden related to oscillator accuracy when employing the proposed double difference method. Proposal 3) RAN1 should study the alternative method for carrier phase synchronization efficient than double difference method. Proposal 4) We propose to study the proposed master-slave gNB synchronization structure and capture the solution in TR 38.859 as an alternative for double difference method. Proposal 5) We propose to remove CFOs of TRPs defined in table A.3-1 in current TR38.859 on the case that proposed master-slave gNB synchronization structure is assumed. Proposal 6) For efficient carrier phase measurement and synchronization, re-designing of current NR PRS is necessary. Proposal 7) Study the proposed continuous PRS sequence with subcarrier allocation method to improve the phase measurement accuracy.

- R1-2209546 Views on NR carrier phase measurement for positioning accuracy enhancement IIT Kanpur, CEWiT Observation 1: PRU can act as an effective node for correcting the errors in the phase measurements for carrier phase-based measurements. Proposal 1: Carrier phase difference measurement should be used as measurement quantity for carrier-phase based positioning. Proposal 2: PRU should be used to remove the measurement error across TRPs for at least synchronization and clock errors in TRP for carrier phase-based measurements. Proposal 3: For integer ambiguity detection, at least the following options should be studied: - Option 1: based on the TOA obtained from the phase-difference of multiple subcarriers within a carrier bandwidth - Option 2: based on one single or the combination of existing measurements (e.g., RSTD, RTOA, etc.) - Option 3: based on the slope of the carrier phase measurement with respect to frequency - Option 4: combination of above approaches Proposal 4: For the purpose of evaluation, the following target accuracy requirement is considered for NR carrier phase positioning: - Horizontal position accuracy (<10 cm for 80%) of UEs

- R1-2209738 Discussion on NR Carrier Phase Measurement Samsung Proposal 1: Study the use of Rel-16 DL and UL positioning reference signals with no or small Comb for the carrier phase method. Proposal 2: Study the use of Rel-16 DL and UL positioning reference signals using the same frequency offset (k') across OFDM symbols of a slot for the carrier phase measurement. Observation 1: PRU can be considered for elimination of gNB and UE clock biases, but at the expense of additional network topology complexity. Proposal 3: RAN1 to study how to provide PRU or PRU-like functionality under existing system. Proposal 4: Study the use of round-trip carrier phase measurement to eliminate UE and gNB clock biases. The round trip carrier phase measurement is the sum of the DL carrier phase measurement at the UE and the UL carrier phase measurement at the gNB. Observation 2: If the coarse position estimate of the legacy-based positioning methods is accurate enough, the carrier phase measurement can be used to further improve the positioning accuracy. Proposal 5: Study using legacy-based positioning measurements to estimate the number of integer cycles between the gNB and the UE. Proposal 6: Study using multiple carriers to determine a phase of a virtual carrier with a lower virtual frequency. Observation 3: Using a combination of both long wavelength and short wavelength, we get a better carrier phase measurement result. Proposal 7: Study using the slope of the carrier phase measurement relative to frequency with multiple sub-carrier measurements to eliminate the integer ambiguity. Proposal 8: Study methods to mitigate the effect of the phase centre offset, e.g., measuring carrier phase of 3 gNBs with rays that are sufficiently close in the spatial domain at the UE. Observation 4: NLOS conditions and multi-path reflections impact the accuracy and usability of the carrier-phase method. Proposal 9: Study the condition for applying CPP method based on the NLOS and multi-path including first path detection. Proposal 10: Study and evaluate the performance of carrier-phase difference measurement for estimating the angle of arrival at the UE. Proposal 11: Study and evaluate the performance of carrier-phase difference measurement for estimating the angle of departure at the UE or the gNB. Proposal 12: Study and evaluate the use of the UEs mobility to create a virtual antenna array. Observation 5: Round-trip carrier phase derivative with respect to carrier frequency can provide centimetre level accuracy. Observation 6: The phase estimation in frequency domain is seriously degraded by multi path effect. Observation 7: Multi-path mitigation method could provide better phase estimation when LoS path probability is low. Observation 8: With larger phase error and ARP error, the performance is degraded severely.

- R1-2209785 Views on improved accuracy based on NR carrier phase measurement Sharp Proposal 1: - Do not consider the phase-difference measurements for DL-AoD. Proposal 2: - Subcarrier-level phase difference report can be discussed as the solution for the integer ambiguity issue. Observation 1: - Phase-smoothed timing measurements can be done by configuring both Rel-18 carrier phase measurement and Rel-17 method.

- R1-2209805 Discussion on OFDM based carrier phase measurement in NR LG Electronics Proposal 1: Consider both carrier phase measurement and subcarrier phase measurement based positioning methods. Proposal 2: Consider RSPD report for the carrier phase measurement. Proposal 3: Study whether and how to utilize PRU for performing double differencing at least for UE-assisted positioning method. Proposal 4: Consider nested RTT method for the carrier phase measurement based positioning Proposal 5: Method for integer ambiguity resolution should be subject for further studies. Observation 1: High positioning accuracy within a wavelength of the carrier frequency can be achieved by carrier phase measurement in NR OFDM system, but it has poor integer ambiguity condition. Observation 2: The subcarrier phase measurement is more robust against integer ambiguity than the carrier phase measurement. Observation 3: The subcarrier phase measurement does not require additional method for TX-RX phase calibration since it is based on differencing. Observation 4: Utilizing the subcarrier phase measurement alone may not enough to achieve centimetre level accuracy. Observation 5: Utilizing timing based positioning (e.g. ToA and TDoA) can help to resolve the integer ambiguity problem by reducing search space for integer value. Observation 6: Accuracy of the initial ToA or TDoA estimate has the direct impact on the CPM outcome. Observation 7: Utilizing multiple component carriers can help to resolve the integer ambiguity problem but cost additional UE complexity. Observation 8: For the CPM positioning and especially for the indoor scenarios, both horizontal and vertical positioning accuracy should be addressed. Observation 9: Higher ToA positioning accuracy can be achieved as larger BW size is used. Observation 10: For the baseband ToA positioning, the vertical (Z-coordinate) accuracy is much worse than horizontal, due to scenario geometry and singular same-height BS placement. Observation 11: For the 100 MHz BW, 90% of UEs can achieve centimeter level accuracy for both the horizontal and vertical positioning when no synchronization error is assumed. Observation 12: Higher CPM based positioning accuracy can be achieved as larger BW size is used.

- R1-2209909 Discussion on improved accuracy based on NR carrier phase measurement NTT DOCOMO, INC. Observation 1: - Regarding integer ambiguity resolution based on multiple carrier frequencies, which frequency granularity (e.g., multiple subcarriers or multiple positioning frequency layers) should be discussed. Observation 2: - The benefits of using TOA instead of existing measurement metric as RSTD/RTOA should be discussed firstly. Observation 3: - Integer ambiguity resolution based on the slope of the carrier phase measurement with respect to frequency needs to be discussed to clear the mechanism and concept. Proposal 1: - The carrier phase measurement can be used to adjust the measurement results of timing-based measurements such as RSTD and RTOA. Proposal 2: - Assistance data which provides helpful information to operate NR carrier phase measurement can be studied. Proposal 3: - Phase difference-based DL-AoD is out of the scope in Rel-18 NR. Proposal 4: - It may be better to postpone the study of SL carrier phase positioning in future release.

- R1-2209992 Phase Measurements in NR Positioning Qualcomm Incorporated Observation 1: Phase-Difference-based AoD is a positioning method that demonstrates performance gains in scenarios with small number of Tx beams at the transmitter side (e.g. FDD scenarios) - A performance of 1m at 80% in the InF-SH scenario, with 20 MHz, is achievable with Phase-Difference DL-AoD, whereas the legacy RSRPP-based DL-AoD, with 2 or 4 Tx beams achieve 5 and 2.2 m respectively Proposal 1: Clarify the definition of measurement duration for oscillator drift modeling Proposal 2: Study supporting Phase-Difference AoD.

- R1-2210099 On NR carrier phase measurement MediaTek Inc. Observation 2-1: The brute force search for the integer cycle number may fall into a solution of local minimum when the fractional cycle measurement error is larger than a threshold Observation 2-2: From the simulation, the fractional cycle error after any cancellation method may need to be kept within 0.025 wavelength to ensure the integer cycle search performance Proposal 3-1: UE may optionally report the uncertainty range of the integer cycle number for the measurement (for example, DL-RSTD measurement) Proposal 3-2: RAN1 to further study the feasibility of the detection of uncertainty range of the integer cycle number and the reporting of the uncertainty range

- R1-2210177 Improved accuracy based on NR carrier phase measurement Ericsson No table of figures entries found. - No table of figures entries found.