3GPP TSG RAN WG1 #110b-e	R1-22010271

e-Meeting, October 10th – 19th, 2022

Source:	CableLabs, Broadcom, Hewlett-Packard Enterprise, Cisco

Title:	SL-U Simulations Assumptions

Agenda Item:	9.4.1.1

Document for: 	Discussion and Decision

# Introduction

RANP #97, approved an updated Sidelink Evolution WID [1]. As part of this WID, the Objectives section #2 specifies

<<-ommited->>

- Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

<<-ommitted->>

    - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval.

<<-ommitted->>

We observe:

    - NR-U Evaluation Methodology was developed in Rel 15-16 [2], based on previous LAA-LTE Evaluation Methodology.

    - It should be noted that by the time [2] was released, there was no unlicensed access technology operating in the 6GHz spectrum. However, recently one chipset vendor announced [3] 1 Billion Wi-Fi 6E chipset have been shipped. More specifically, SL-U operation, intended for 6GHz, has to co-exist with other technologies (e.g. 802.11ax) using this spectrum.

    - Under these circumstances, any change to NR-U specifications may cause significant challenges to existent Wi-Fi 6 eco-system.

RAN #95e approved an updated SideLink Evolution WID [6], stating:

    - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

Observation 1: The EIRP densities are different between n46 and n102, also between indoor and outdoor environments.

This paper provides clarifications on the SL-U Evaluation Methodology, in the light of the above.

# Evaluation Assumptions.

The SL-U SID should make recommendations based on a commonly agreed set of simulations assumptions, differentiating between the 3 bands under consideration.

To properly estimate the SL-U coexistence performance in a share unlicensed spectrum environment, all simulations should use the same set of baseline assumptions. Using different sets of assumptions shall not be allowed since the results can’t be assessed.

It is understood that companies could also present other evaluation results based on some optional parameters.

We consider the proposed FCC, ISED (Figure 1) and EU 6 GHz (Figure 2) spectrum allocations, as shown in the Appendix as the spectral base for the propagation assumptions.

We propose the following Evaluation Assumptions based on [2], modified per actual FCC (USA), ISED (Canada) 6 GHz regulatory allowances. We split the evaluation assumptions into 3 categories: indoor office propagation, outdoor scenarios propagation and SL-U/Wi-Fi coexistence.

Table 1. Propagation Evaluation Parameters, sub 7GHz indoor

| Parameter | Value | Comments |
| --- | --- | --- |
| Carrier Frequency | 6 GHz (mandatory), 6405-6425 MHz, 5 GHz is optional | Channel 6405-6425MHz is the upper EU 6GHz channel and bordering US/ISED U-NII 6 |
| Carrier Channel Bandwidth | 20MHz baseline | 6405-6425MHz |
| Number of carriers | 1 |  |
| Number of beams | 1 |  |
| Number of users per operator | 5 UEs associated with each gNB per 20MHz |  |
| SCS | 30kHz |  |
| Channel Model | NR InH Mixed Office model |  |
| UE/STA EIRP density (band n96) | 5dBm/MHz | 18dBm (total across all TX antennas) |
| UE/STA EIRP density (band n46) (optional) | 11dBm/MHz | 250mW (U-NII 2B 5.35-5.47GHz); max 250mW EIRP |
| BS/AP Antenna gain | 0dBi |  |
| UE/STA Antenna gain | 0dBi |  |
| BS/AP Noise Figure | 5dB |  |
| UE/STA Receiver Noise Figure | 9dB |  |
| Minimum received power from serving cell for UE dropping | -95dBm |  |
| UE receiver | MMSE-IRC as the baseline receiver |  |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ | TR38.901, section #7.3 |
| UE/STA antenna Array configuration | Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ | TR38.901, section #7.3 |
| Traffic model | Table A.1.1, TR36.889 | Specific SL-U traffic needs could be discussed. |
| UE/STA to UE/STA link pathloss model | - Indoor Office (Table 7.2-2), - InH-Office propagation model | TR38.901 |
| gNB to gNB link pathloss model | - Indoor Office (Table 7.2-2), - InH-Office propagation model | TR38.901 |

Proposal 1: The sub 7GHz indoor propagation assumptions should be based on Table 1.

Table 2. Propagation Evaluation Parameters, sub 7GHz outdoor

| Parameter | Value | Comments |
| --- | --- | --- |
| Carrier Frequency | 6 GHz (mandatory), 6405-6425 MHz, 5 GHz is optional | Channel 6405-6425MHz is the upper EU 6GHz channel and bordering US/ISED U-NII 6 |
| Carrier Channel Bandwidth | 20MHz baseline | 6405-6425MHz |
| Number of carriers | 1 |  |
| Number of beams | 1 |  |
| Number of users per operator | 5 UEs associated with each gNB per 20MHz |  |
| SCS | 30kHz |  |
| Channel Model | NR UMi street canyon | TR38.901 |
| BS/AP Spectral Density (n96 band) - optional | 17dBm/MHz | - 6dB beam separation considered - Max EIRP=30dBm (20MHz) allowed |
| BS/AP Spectral Density (n46 band) | 23dBm/MHz |  |
| UE/STA Tx Power | 23dBm (total across all TX antennas) |  |
| BS/AP Antenna gain | 0 dBi |  |
| UE/STA Antenna gain | 0 dBi |  |
| BS/AP Noise Figure | 5dB |  |
| UE/STA Receiver Noise Figure | 9dB |  |
| Minimum received power from serving cell for UE dropping | -95dBm |  |
| UE receiver | MMSE-IRC as the baseline receiver |  |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ | TR38.901 |
| UE/STA antenna Array configuration | (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ | TR38.901 |
| Traffic model | Table A.1.1, TR36.889 | Specific SL-U traffic needs could be discussed. |
| UE/STA to UE/STA link pathloss model | UMi (street canyon pathloss model) | TR38.901 |
| gNB to gNB link pathloss model | UMi (street canyon pathloss model) | TR38.901 |

Proposal 2: The sub 7GHz outdoor propagation assumptions should be based on Table 2.

Table 3. SL-U/Wi-Fi Coexistence Parameters (sub 7GHz)

| Parameter | Value | Comments |
| --- | --- | --- |
| Common Assumptions |  |  |
| TDD DL/UL ratio | 50/50 |  |
| Primary LBT | Cat-4 LBT (exponential back-off) |  |
| MCOT duration | 6 ms |  |
| Max Modulation | QAM256 |  |
| CCA ED threshold | -72dBm | Channel BW=20MHz |
| CW{min, max} | DL{15,63} UL{15,1023} |  |
| Max number of air layers | 2 | MIMO rank |
| Slots/Subframe | 2 |  |
| NR-U/SL-U assumptions |  |  |
| DMRS | 1 symbol | Overhead |
| DRS | Enabled, 1ms |  |
| HARQ ACQ/CQI Feedback | 1 symbol per feedback |  |
| PDCCh | 1 symbol/slot |  |
| PDSCh Mapping | Type B |  |
| PUSCh Mapping | Type A |  |
| Scheduling | Proportional fair |  |
| gNB to UE COT sharing | Enabled |  |
| UE to UE COT sharing | N/A |  |
| Wi-Fi |  |  |
| RTS/CTS | Disabled | While not a realistic option, it simplifies the coding |
| MPDU size | 1500 bytes |  |
| Wi-Fi guard interval | Short |  |
| Frame Aggregation | A-MPDU |  |

Proposal 3: The sub 7GHz outdoor propagation assumptions should be based on Table 3.

# Conclusions

This paper proposes 3 detailed sets of assumptions (Indoor/Outdoor Propagation and SL-U/Wi-Fi Coexistence) to support the evaluation of SL-U performance in a shared spectrum environment in the presence of an incumbent access technology (802.11ax).

The use of proprietary sets of assumptions should be allowed as optional simulations assumptions. We observed and proposed the following:

Observation 1: The EIRP densities are different between n46 and n102 and also between indoor and outdoor environments.

Proposal 1: The sub 7GHz indoor propagation assumptions should be based on Table 1.

Proposal 2: The sub 7GHz outdoor propagation assumptions should be based on Table 2.

Proposal 3: The sub 7GHz outdoor propagation assumptions should be based on Table 3.

# References

[1] RP-221938, “NR SideLink Evolution”, RAN-P #97e, September 2022

[2] 3GPP TS38.889 (2018-12-19). “Study on NR-based access to unlicensed spectrum” (Release 16).

[3] https://www.broadcom.com/blog/broadcom-ships-one-billion-wi-fi-6-and-6e-chips

[4] Aruba blogs. “The 6 GHz Band: Say Goodbye to the Stone Age of Wi-Fi”,

[5] FCC, “Part 15 Subpart E U-NII 6 GHz. General Guidance Bands 5, 6, 7, 8”, May 2022

[6] RP-221938, “WID Revision: NR Sidelink evolution”, RAN Plenary #97e, September 2022

[7] FCC, “Title 47, Chapter 1, Subchapter A, Part 15 Subpart E U-NII 5.15-5.35, 5.47-5.895 and 5.925 GHz. General Guidance Bands 5, 6, 7, 8”, September 2021

# Appendix

Figure 1. Approved FCC 6GHz spectrum [4]. Canada's ISED follows the same spectrum assignation.

Table 4. Overview of U-NII Rules [5]

Figure 2. EU 6GHz spectrum allocation.