3GPP TSG RAN WG1 #110	bis-e		                                    		   R1-2210306

e-Meeting, October 10th – 19th, 2022

Source:	Moderator (OPPO)

Title:	Summary #2 on Rel-18 STxMP

Agenda Item:	9.1.4.1

Document for:	Discussion and Decision

# Introduction

The Rel-18 WID for MIMO Evolution for Downlink and Uplink includes the following objectives:

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable)

  - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission

    - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

This document summarizes the company proposals of AI 9.1.4.1, and mainly focus on following issues:

- Single-DCI based STxMP PUSCH

  - Tx schemes for single-DCI based STxMP PUSCH, including whether to support SFN

  - DMRS port indication for SDM scheme

  - SRS resource set configuration and TMPI indication for SDM scheme

  - Layer combinations for SDM scheme

  - 2 CWs for SDM scheme.

  - PTRS indication for SDM scheme

  - How to switch between SDM and other schemes.

- Multi-DCI based STxMP PUSCH

  - To support STxMP PUSCH in multi-DCI based system

  - Supported PUSCH type in STxMP PUSCH+PUSCH

  - Types of overlapping in time domain and frequency domain?

  - SRS resource set configuration and TMPI indication for STxMP PUSCH transmission in multi-DCI system

  - Configuration restriction for STxMP PUSCH+PUSCH

- STxMP PUCCH

  - STxMP PUCCH in single-DCI based system

  - STxMP PUCCH in multi-DCI based system

# Proposals for Friday GTW

Proposal 2-4: Regarding the TPMI/SRI indication for multi-DCI based STxMP PUSCH+PUSCH:

- Configure two SRS resource sets for CB or NCB.

  - FFS: how to associate coresetPoolIndex value implicitly or explicitly.

  - FFS the maximal number of SRS resources in each set for NCB

  - FFS the maximal number of SRS ports in each set for CB.

- FFS: Separate codebooks and separate maxRanks are configured for different SRS resource sets.

- For type 1 CG-PUSCH (if supported), FFS how to associate the PUSCH with one TRP

  - e.g., configure a coresetPoolIndex value in a type 1 CG-PUSCH

  - e.g., use a single CG to configure two type 1 CG PUSCHs for STxMP PUSCH+PUSCH

Proposal 1-7: For single-DCI based STxMP PUSCH:

- Support dynamic switching between SDM scheme and sTRP transmission

  - [FFS the indication of dynamic switching] vs [Use the DCI field ‘SRS resource set indicator’ to indicate the switching]

  - FFS the maxRank when switching to sTRP transmission,

- For the switch between SDM scheme and Rel-17 PUSCH TDM scheme, down-select one from the followings:

  - Alt1: Support dynamic switching between SDM scheme and Rel-17 PUSCH TDM scheme

    - FFS: how to support dynamic switching, e.g., using the indicated PUSCH repetition number

  - Alt2: Support RRC-based switching between SDM scheme and Rel-17 PUSCH TDM scheme

Proposal 1-1a Support SFN-based transmission scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18

Support(19): InterDigital, Spreadtrum, vivo, Ericsson, Lenovo, OPPO, Google, LG, CATT, Fujitsu, Intel, Xiaomi, MediaTek, Fraunhofer IIS/HHI, Samsung, NTT DOCOMO, Qualcomm, Nokia, NEC

Not support: Huawei,HISilicon

Proposal 3-1: Support to specify SFN scheme for single-DCI based STxMP PUCCH

- One PUCCH resource can be indicated with two TCI states and the PUCCH resource is transmitted by applying two TCI states at the same time.

Support(18): QC, Nokia/NSB, InterDigital, Lenovo, Google, NTT Docomo, ZTE, NEC, Fujitsu, LG, MediaTek, Spreadtrum, OPPO, Xiaomi, Intel, CATT, vivo, Ericsson.

Not Support: Huawei,Hisilicon.

# Single-DCI based STxMP PUSCH

## Issue #1: Tx scheme for single-DCI based STxMP PUSCH

### Summary

In RAN1#110 meeting, we reached the following:

Working assumption Support the following schemes for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18: - SDM scheme - In RAN1#110bis-e, RAN1 will only consider SFN based transmission scheme to support in addition to the above. Decision to support or not to be made in RAN1#110bis-e. - FFS: FDM-A scheme - FFS: FDM-B scheme - FFS: SFN-based transmission scheme For schemes other than SDM, final decision to support or not will be made in RAN1#110bis-e.

On pending issue is whether the SFN based transmission scheme shall be supported for single-DCI based STxMP PUSCH.

In the contributions, companies provided evaluation results on SFN based transmission scheme:

Huawei/HiSilicon provide SLS results of SFN in [1] - SLS results, throughput - SFN vs panel selection: ~10% RU; ~20%RU; ~50%RU. Total TRP; 23dBm; 26dBm; 23dBm; 26dBm; 23dBm; 26dBm. Mean; -7.47%; 1.63%; -7.10%; 1.40%; -15.40%; 2.00%. 5%-ile UE; -24.7%; 0.2%; -21.61%; 2.0%; -29.16%; 10.05%. 50%-ile UE; -5.84%; 2.20%; -6.75%; 0.6%; -14.76%; 0.72%. 95%-ile UE; -3.44%; 0.23%; -1.66%; 1.80%; -6.82%; 0.51%.

OPPO provided the following SLS results for SFN with FTP RU 80% in [8]:

LG provided both SLS and LLS results for SFN scheme in [10]: - SLS: throughput gain - LLS: BLER Figure 2. Average/edge UPT gain for RU=30% Figure 3. Average/edge UPT gain for RU=60% Figure 4. BLER performance for PUSCH

NTT DOCOMO provided the LLS results of SFN in [22]: - LLS: BLER performance SFN vs Rel-17 TDM scheme Figure1. BLER performance of STxMP PUSCH SFN scheme at low code rate Figure2. BLER performance of STxMP PUSCH SFN scheme at moderate code rate

FL Note, the evaluation results for SFN-bases scheme provided in tdocs can be summarized as:

- The SLS results of OPPO and LG showed significant throughput gain for Max Tx Power Option 1 (i.e., max TRP per UE) for various FTP loads.

- The SLS results of OPPO showed significant throughput gain for max Tx power option 2 (i.e., max TRP per panel) for high FTP load.

- The LLS results of LG showed that SFN can provide improve the BLER performance by ~3dB compared to baseline single-panel transmission.

- The LLS results of NTT DOCOMO showed that SFN scheme can provide similar or close BLER performance as TDM-based repetition.

- The SLS results of Huawei showed that SFN scheme can improve the system throughput with Max Tx power Option 2 but show throughput loss with Max Tx power Option 1.

Regarding whether to support SFN-based scheme for single-DCI based STxMP PUSCH, companies provided the following views in tdocs:

- Alt1: Support to specify SFN-based scheme for single-DCI based STxMP PUSCH

  - Support: InterDigital, Spreadtrum, vivo, Ericsson, Lenovo, OPPO, Google, LG, CATT, Fujitsu, Intel, Xiaomi, MediaTek, Fraunhofer IIS, Fraunhofer HHI, Samsung, NTT DOCOMO, Qualcomm, Nokia

- Alt2: Do not support SFN-based scheme for single-DCI based STxMP PUSCH

  - Support: Huawei/HiSilicon

The following companies also proposed to confirm the WA of supporting SDM scheme:

- InterDigital, Spreadtrum, vivo, Ericsson, Lenovo, Google, CATT, Xiaomi, MediaTek, Fraunhofer IIS, Fraunhofer HHI, Samsung, NTT DOCOMO, Qualcomm, Nokia

### Proposals for discussion

Per the decision at RAN1#110 meeting, we shall decide whether to support SFN-based scheme in RAN1#110bis-e meeting.  According views provided in the contributions, the majority of companies supports to specify SFN-based scheme and most of the SLS/LLS simulation results provided in contributions showed that the SFN can improve the system throughput for either Tx power option 1 or Tx power option 2, and also improve the BLER performance. Furthermore, as explained in contributions, the SFN-based scheme requires less specification effort and simple implementation but can improve the reliability, especially the cell-edge UEs.  Therefore, the moderator would like to propose to support the SFN-based scheme for single-DCI based STxMP PUSCH. Furthermore, many companies suggested to confirm the WA of supporting SDM. Some companies proposed to confirm the WA with clarifying that SDM with 1 CW. From the understanding of moderator, whether to support 2 CW is a separate pending issue. Thus, we do not need to include “with 1 CW” here.

Therefore, the following proposals are made: one to support SFN scheme and another one to confirm the WA. Companies are invited to provide views on whether the proposal can be supported, suggest modifications, or present additional aspects for the proposal.  The supporting shown in Tdocs are also listed under each proposal.

FL Proposal 1-1a Support SFN-based transmission scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18

Support: InterDigital, Spreadtrum, vivo, Ericsson, Lenovo, OPPO, Google, LG, CATT, Fujitsu, Intel, Xiaomi, MediaTek, Fraunhofer IIS, Fraunhofer HHI, Samsung, NTT DOCOMO, Qualcomm, Nokia

FL Proposal 1-1b Confirm the following working assumption:

Support the following schemes for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18:

- SDM scheme

Support: InterDigital, Spreadtrum, vivo, Ericsson, Lenovo, Google, CATT, Xiaomi, MediaTek, Fraunhofer IIS, Fraunhofer HHI, Samsung, NTT DOCOMO, Qualcomm, Nokia

Companies’ views on proposals 1-1a and 1-1b:

| Company | Support (Yes/No) | Comments |
| --- | --- | --- |
| QC | Yes | We support both proposals, but suggest discussing them together (i.e., in one proposal). We share the same understanding as the moderator that SDM scheme here means “1 CW” irrespective of whether we explicitly mention it or not, and supporting 2 CWs is a separate issue. |
| Nokia/NSB | Yes | Support both FL’s proposals 1-1a and 1-1b |
| InterDigital | Yes | Support both FL proposals. |
| Lenovo | Yes | Support both FL proposals and agree with FL that supporting 1 or 2 CWs is a separate issue. |
| Google | Yes |  |
| NTT Docomo | Yes | Support both proposals |
| ZTE | Yes with update | Regarding SFN-based transmission scheme, we are wondering whether it assumes two PUSCH repetitions use same precoders/layers/DMRS port/RV/etc except beam indication? If so, it should be explicitly indicated in proposal 1-1a. Consequently, beam indication related enhancement should be discussed in AI 9.1.1.1. FL Proposal 1-1a Support SFN-based transmission scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18 - Note: It is consensus in RAN1 that only per panel beam indication needs to be specified for this scheme. Regarding SDM scheme, basically, it must fulfill the request in WID that reach higher UPT. As we elaborate in section 2.5, the motivation of supporting SDM scheme without 2 CWs will be very meaningless and doubtful when considering performance gain and NW deployment complexity. On the other hand, UE implementation complexity of supporting 2 CWs SDM scheme can be accommodated, support both 1 CW and 2 CWs for SDM scheme can be the way-forward to address the above issues. FL Proposal 1-1b Confirm the following working assumption with the update below: Support the following schemes for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18: - SDM scheme with both 1 CW and 2 CWs |
| NEC | Yes |  |
| Mod2 |  | @ ZTE, - regarding the Proposal 1-1a: For SFN scheme, my understanding is that two PUSCH repetitions use same layers/DMRS ports/RVs, but the precoders could be different according to description in tdocs. Thus, your suggested sub-bullet is not needed. - Regarding the proposal 1-1b: whether to supporting 2 CW is a separate issue, as it is captured in Issue#5. So suggest not to that update. |
| Fujitsu | Yes | Support both proposals |
| LG | Yes | @ZTE: Regarding the Note suggested by ZTE, we don’t need such restriction and may further discuss the issue, if needed, after we agreed to support SFN. Also, we does not consider such restriction in evaluation and have no observation how much performance degradation is caused by such restriction as the same precoder and the same power control across the two panels. |
| Samsung | Yes (with update) | We support both proposals in general. For 1-1b, we can share same view as FL and QC. Supporting 2CWs is different issue. For 1-1a, our main preference is supporting SFN for sDCI based sTRP system as well as sDCI based mTRP system. Therefore, we suggest the following modification for 1-1a: FL Proposal 1-1a Support SFN-based transmission scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18 - Note: The SFN scheme for single-DCI based sTRP system in Rel-18 is not precluded - FFS: Definition of SFN based transmission with two panels and details (e.g. precoding, DMRS indication, …) |
| MediaTek | Yes | Support both FL proposals. |
| Spreadtrum | Yes | Support both FL’s proposals |
| OPPO | Yes | Support both FL’s proposals. |
| Fraunhofer IIS/HHI | Yes | Support both proposals |
| Xiaomi | Yes | Support both proposals |
| Mod 3 |  | @Samsung: the definition of SFN-based transmission scheme was defined very clearly in the agreement made in RAN1#109 meeting: Agreement For STxMP PUSCH in single-DCI based mTRP system, study and evaluate the following schemes for PUSCH: - … - SFN-based transmission scheme: all of the same layers/DMRS ports of one PUSCH are transmitted from two different UE panels simultaneously. - …. So FFS on definition of SFN-based is not needed. However, of course, the next step is to work on the detailed signaling design if it is agreed, which might be what is in your mind. And regarding your term “single-DCI based sTRP system”, it is kind of confusing. If you mean the Rel-15/16 sTRP transmission scheme, then it is already supported in the spec and it is not what being discussed here. If you mean when the gNB can configure this scheme to a UE with 2 panels? My understanding is the gNB can configure it according to the implementation as long as the UE reports its UE capability of multiple panels and supporting this scheme. In other word, The suggested Note seems not necessary |
| Huawei, HISilicon | No | FL Proposal 1-1a: We do not support Proposal 1-1a. We understand that supporting SFN for STxMP has a strong majority, however, very limited companies have actually investigated this scheme and, further, their corresponding results are mixed: - SLS for SFN is performed by only 3 companies (OPPO, LGE, HW) and while OPPO and LGE observed average/cell-edge throughput gain, HW observed average/cell-edge throughput loss compared to the baseline panel switching. - LLS for SFN is performed by only two companies (LGE, DCM) and while LGE shows a better BLER performance, DCM shows slightly worse performance compared to the baseline. We also have some questions regarding some of the SLS results. We noticed that some of the SLS results show very significant average throughput gain for SFN which is primarily motivated to improve the reliability specially for the cell-edge UEs. For instance, OPPO’s results show 19% and 9% average throughput gain for SFN compared to the panel selection for UE power assumption 2 and 1, respectively. However, in OPPO’s t-doc to RAN1 110 (R1-2206268), the average throughput gain for SDM compared to the panel selection for UE power assumption 2 and 1 was 23% and 14%, respectively. This means that the average throughput gain of SDM over SFN is only 3% and 4.5% for UE power assumption 2 and 1, respectively. To us, this is a bit of a surprising result since the SDM primary motivation is to increase the average throughput. In other words, while SFN which is supposedly a reliability-improving scheme offers 19% average throughput gain compared to the baseline, SDM which is supposedly a throughput-improving scheme offers only 3% average throughput gain to SFN. FL Proposal 1-1b: If all companies support changing the WA to an agreement, we can compromise for the sake of progress if the same structure as in WA is preserved. In other words, we can compromise to support the following: Proposal 1-1b (update 1): Support the following WA reached in RAN1 110: Working assumption For single-DCI based STxMP PUSCH SDM scheme, support the layer combinations of {1+1, 1+2, 2+1 and 2+2} for single CW case. - FFS on layer combinations of {1+3} and {3+1} considering the performance gain, system/UE complexity, specification efforts, etc. - FFS: the option of using layer combination of 0+n and n+0 for dynamic switch between single-panel and STxMP (n=1 or 2, 3 or 4). - This applies to SDM with 1 CW at least. Alternatively, if a more compact Agreement is preferred, we are also OK with the following: Proposal 1-1b (update 2): For single-DCI based STxMP PUSCH SDM scheme, support the layer combinations of {1+1, 1+2, 2+1 and 2+2} at least for 1 CW case. |
| Intel | Yes | Support both FL’s proposals. |
| ZTE2 |  | We still doubt the necessity of supporting 1 CW only for SDM STxMP PUSCH very much. Base on our further input in section 2.5 (proposal of 2 CWs SDM), we can at most compromise to make 2 CWs SDM as UE optional. FL Proposal 1-1b Confirm the following working assumption: Support the following schemes for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18: - SDM scheme with both 1 CW and 2 CWs. - 2 CWs SDM is UE optional. |
| MediaTek | Yes | We are also fine to confirm the original WA |
| CATT | Yes | Support both FL’s proposals 1-1a and 1-1b. |
| vivo | Support | Support FL’s proposal. Also OK with Samsung’s modification. |
| Samsung |  | @FL: Thank you for sharing your view. For definition of SFN, you are right. Sorry for confusion. Our intention is this scheme can make the gNB configure sTRP transmission scheme to the UE with 2 panels explicitly. The gNB can indicate the pair of SRS resources which each SRS resource is for each panel toward one TRP and one TPMI across two panels (e.g. if # of ports in one SRS resource is 2 and a pair of two SRS resources are indicated, the TPMI for the pair can be [4 x ν (layers)] matrix). The pair of SRS resources can be indicated based on SRS resource group which I explained in issue #3. In Rel-15/16, sTRP multi-panel transmission was only supported in spec-transparent manner. We think this explicit sTRP STx2P transmission can be beneficial for more flexible scheduling, e.g. gNB can schedule one of among mTRP STx2P, panel selection and sTRP STx2P explicitly. For added note and FFS, okay without them for the current stage. However, when we specify details for SRS resource set configuration, we suggest to consider two SRS resource sets with SRS resource group as one alternative for explicit sTRP STx2P. |
| NTT Docomo | Yes | We would like to clarify a bit on our simulation results of SFN. In the simulation, we compare BLER performance between SFN and Rel-17 TDM M-TRP repetition. In the simulation, we assume same precoding is used for two panels. SFN shows similar BLER performance as TDM M-TRP repetition. And we think SFN has some benefit compared TDM M-TRP repetition. It is simpler (same precoding/RV), has lower latency, needs less control signaling overhead (one TPMI is indicated). |
| Panasonic | Yes | Support proposals 1-1a and 1-1b. |
| OPPO2 | Yes | We would like to clarify our simulation of SFN scheme. SFN scheme improves the probability of successful transmission, especially for cell edge UEs. This significantly improves the throughput of cell edge UEs, which is shown in our results. |
| Ericsson | P1-1a: yes P1-1b: no | Proposal 1-1a: We think there is a benefit of SFN to facilitate single-layer transmission, but it is questionable if it’s in the WID scope: since it’s a new transmission scheme. (4-layer transmission already exists.) Proposal 1-1b: We do not support confirming the WA as long as 2CW are in scope. |
| Mod 4 |  | @Samsung: thanks for being flexible. @ZTE: as shown in the discussion, 2 CW is definitely another separate issue. @Ericsson: 2CW is separate and pending issue. Confirming the WA of SDM is does not mean the supporting of 2 CW. @HW: as companies explained, SFN improves the reliability, especially cell-edge UE, and then the throughput is improved. |

### Summary of Round 1

Regarding the FL proposal 1-1a, it has strong majority supporting and Huawei, HISilicon objects it. NTT DOCOMO and OPPO replied the HW’s comments on simulation result. From the understanding of moderator, the SLS results provided by companies does show the performance gain, especially to the cell-edge UE.

Considering all the comments collected in round 1 and also the proposals in contribution, moderator would suggest to put Proposal 1-1a in email endorsement.

Regarding the proposal 1-1b: ZTE suggested to confirm the WA with including 2 CW but HW and Ericsson suggested to confirm the WA with 1 CW. From the understanding of moderator, whether to support 2 CW is really a separate issue and this confirming the WA does not mean to say anything for 2 CW.  To make that clear, a note would be added in the proposal for further discussion.

### Proposals for Round 2 discussion

And the following is the updated proposal 1-1b

Updated Proposal 1-1b Confirm the following working assumption:

Support the following schemes for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18:

- SDM scheme

Note: whether to support 2 CW for SDM scheme is a separate issue.

Comments on Updated Proposal 1-1b:

| Company | Support (Yes/No) | Comments |
| --- | --- | --- |
| QC | Yes |  |
| LG | Yes |  |
| NTT Docomo | Yes |  |
| Fujitsu | Yes |  |
| MediaTek | Yes |  |
| CATT | Yes |  |
| ZTE |  | Thanks for FL further clarifying that 2 CWs for SDM scheme is separate issue of this WA. However, as companies mentioned that SDM scheme here without any statement/wording means 1 CW only, that cannot be acceptable to us according to our multiple rounds of elaborations in round 1. Again, we sincerely hope companies can understand infra vendor’s concern of practical MTRP deployment challenges as well as meaningful performance gain of 1 CW SDM when compared with 2 CW. In light of the above, we can compromise to: Updated Proposal 1-1b Confirm the following working assumption: Support the following schemes for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18: - SDM scheme with 1 CW and 2 CWs - 2 CWs SDM is UE optional. |
| OPPO | Yes |  |
| Xiaomi | Yes |  |
| InterDigital | Yes |  |
| Intel | Yes | For the note, does 2-CW SDM scheme include the SDM repetition scheme? |
| Huawei, HiSilicon | No | The WA in RAN1 110 is as follows: Working assumption For single-DCI based STxMP PUSCH SDM scheme, support the layer combinations of {1+1, 1+2, 2+1 and 2+2} for single CW case. - FFS on layer combinations of {1+3} and {3+1} considering the performance gain, system/UE complexity, specification efforts, etc. - FFS: the option of using layer combination of 0+n and n+0 for dynamic switch between single-panel and STxMP (n=1 or 2, 3 or 4). - This applies to SDM with 1 CW at least. If all companies prefer to confirm the WA, we wont object if the same wording as the WA is used. If FL insists to change the wording of WA (which, in our view, generally should be avoided unless there is a good reason which we fail to see in this case), we prefer to specifically mention that the support is only 1CW and 2CW is an FFS. We have difficulty understanding why it is avoided to mention that 1CW is supported and the 2CW is FFS. This is the common denominator that all companies can agree with and, to our understanding, the simplest and the clearest way to put it is as follows: Proposal: Support 1CW SDM scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18. - FFS: Support for 2CW SDM scheme. |
| Spreadtrum | Yes | Support the updated proposal and the codeword can be discussed in another issue. |
| Nokia/NSB | Yes |  |
| Samsung | Yes | Support FL’s proposal. And HW’s one can be fine. |
| Lenovo | Yes |  |
| Mod |  | @ZTE, again, 2 CW is separate issue and is already captured in Issue#5. @HW: actually the FL proposal does not change any WA wording. Working assumption Support the following schemes for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18: - SDM scheme - In RAN1#110bis-e, RAN1 will only consider SFN based transmission scheme to support in addition to the above. Decision to support or not to be made in RAN1#110bis-e. - FFS: FDM-A scheme - FFS: FDM-B scheme - FFS: SFN-based transmission scheme For schemes other than SDM, final decision to support or not will be made in RAN1#110bis-e.. The added note is to clarify one fact and to relax ZTE’s concern. |

## (Closed) Issue #2 : DMRS port indication for SDM scheme

### Summary

RAN1#110 agreed to study a few aspects of DMRS port indication for SDM scheme. In the contributions, few issues were discussed and summarized as follows.

The first issue is whether to use one or two DCI fields to indicate the DMRS ports for two panels of SDM scheme. The views in the contributions are:

- Alt1: For SDM scheme, reuse the ‘Antenna Ports’ field in DCI to indicate DMRS ports for two panels and the sum of ranks of two panels is used to determine the value of DMRS port indication table.

  - Supported by: Huawei/HiSilicon, InterDigital, ZTE, Spreadtrum, vivo, Ericsson, Lenovo, OPPO, CATT, Fujitsu,  Intel, Xiaomi, Sharp, Apple, Samsung, NTT DOCOMO, Qualcomm, Panasonic

- Alt2: Introduce a new DCI field for second antenna port indication

  - Supported: Nokia

The second issue is whether the indicated DMRS ports shall be in different or same CDM groups? The views in the Tdocs are:

- Alt1: the DMRS ports of different panels must be in two different CDM groups

  - Supported: Huawei/HiSilicon, Spreadtrum, Ericsson, OPPO, Intel, Sharp, MediaTek, Fraunhofer IIS, Fraunhofer HHI

- Alt2: the DMRS ports of different panels can be in same or different CDM groups, i.e., no restriction.

  - Supported: ZTE, Lenovo, LG, CATT, Xiaomi(1st preference), Samsung, NTT DOCOMO, Qualcomm

The third issue is how to determine the partition the indicated DMRS ports among PUSCH layers/panels. The contributions presented the following Alts:

- Alt1: Two different CDM groups are indicated. The 1st and 2nd CDM group are associated with layers indicated by 1st and 2nd TPMI/SRI field/TCI state

  - Supported: Intel, MediaTek, Fraunhofer IIS, Fraunhofer HHI

- Alt1-1: Two different CDM groups are indicated. Each CDM group is associated with layers indicated by one TPMI/SRI field. If the number of layers of two TPMI/SRI fields are different, CDM group is associated with the TPMI/SRI field with same number of layers. If number of layers of two TPMI/SRI fields are same, then 1st and 2nd TPMI/SRI field is associated with 1st and 2nd CDM group.

  - Supported by: Huawei/HiSilicon

- Alt2: DMRS ports partition between two panels is based on the number of layers indicated by two TPMI fields for codebook-based PUSCH or two SRI fields for non-codebook based PUSCH

  - Supported by: ZTE, Lenovo, LG, Xiaomi, Samsung, NTT DOCOMO

- Alt2-1: DMRS ports partition between two panels is based on the number of layers of each panel and also the value of ‘SRS resource value indicator’ field.

  - Supported by: Qualcomm

FL Note:  About the first issue, Alt1 has super majority. Thus moderator suggest to move forward with Alt1, i.e.. resuse the existing ‘Antenna Ports’ to indicate DMRS ports for both panels.

About the second issue, either Alt1 and Alt2 has good number of supporting company so far. The solution for the third issue would highly depend on the conclusion of the second issue. Since the following up design depends on the decision of the second issue, thus moderator would like to suggest to complete the down-selection in RAN1#110bis meeting.

### Proposal for discussion

Based on the companies’ views, the following proposal is made:

FL Proposal 1-2

- Reuse the DCI field ‘Antenna Ports’ in DCI format 0_1 and 0_2 to indicate DMRS ports for SDM scheme of single-DCI based STxMP PUSCH:

  - The total numbers of layers, L, indicated by two TPMI fields of CB PUSCH or two SRI fields of NCB PUSCH is used to determine the DMRS port indication table.

  - L1 of the indicated DMRS ports are associated with the L1 PUSCH layers which are indicated by the first TPMI field for CB PUSCH or the first SRI field for NCB PUSCH and the rest L- L1 of the indicated DMRS ports are associated with the L-L1 PUSCH layers which are indicated by the second TPMI field for CB PUSCH or the second SRI field for the NCB PUSCH.

    - FFS: how to partition the indicated DMRS ports among the PUSCH layers.

- Down-select one from the following two Alts for SDM scheme in RAN1#111:

  - Alt1: the DMRS ports associated with two TPMI/SRI fields must be in different CDM groups.

    - Supported: Huawei/HiSilicon, Spreadtrum, , OPPO, Intel, Sharp, MediaTek, Fraunhofer IIS, Fraunhofer HHI, Fujitsu, vivo,

  - Alt2: the DMRS ports associated with two TPMI/SRI fields can be in same or different CDM groups.

    - Supported: ZTE, Lenovo, LG, CATT, Xiaomi(1st preference), Samsung, NTT DOCOMO, Qualcomm, InterDigital, Apple, Ericsson

Companies’ views:

| Company Name | Support Proposal 1-2 (Yes/No) | Comments |
| --- | --- | --- |
| QC | Yes | Instead of “first” / “second” panel, we suggest to say “one” / “the other” panel to avoid confusion on whether this sub-bullet already implies the exact partitioning. Also, we prefer to say “SRS resource set” instead of “panel” assuming that proposal 1-3 will be agreed because “panel” is not defined so far, and may not be used in specification. - L1 of the indicated DMRS ports are associated with the L1 PUSCH layers of the first panel associated with one SRS resource set and the rest L- L1 of the indicated DMRS ports are associated with the L-L1 PUSCH layers of the second panels associated with the other SRS resource set. - FFS: how to partition the indicated DMRS ports among the PUSCH layers. Regarding Alt1 versus Alt2, we do not see any reason why the restriction in Alt1 is needed for UL. It should be up to gNB scheduling and as long as it is possible for gNB to schedule either within same or across different CDM groups, there is no need to define a restriction in the specification. Our understanding is that, this is really related to rank combination 1+2: - All existing DMRS ports for rank 3 can be used for 1+2 - If different CDM groups are needed, one option is to use the existing entry {0,1,2} but with SRS resource set indicator set “11” such that {0,1} are mapped to the second SRS resource set. Another option is to define a new DMRS port entry {0,2,3}, but we do not prefer this option because new DMRS tables would need to be introduced which is completely avoidable based on the existing signaling described above. |
| Nokia/NSB | No | First main bullet: Not support. Alt-2 provides very straightforward way to support antenna port indication with STxMP without need to specifying any extra DMRS antenna port partitioning between indicated layers and antenna panels. For Alt-2 there is always one-to-one mapping between the first antenna port and first TPMI/SRI as well as the second antenna port and second TPMI/SRI. Moreover, the number of potential antenna port combinations associated with one antenna indication is restricted leading to limited MU STxMP operation in uplink. By introducing the second antenna port indication, this problem can be avoided. |
| InterDigital | Yes | For the second bullet of FL Proposal 1-2, we don’t see a need for CDM group restriction, so our preference is on Alt2. |
| Lenovo | Yes | For the association between DMRS port(s) and PUSCH layer(s), we prefer the following updated wording to align with the 1st sub-bullet of the 1st main bullet: - L1 of the indicated DMRS ports are associated with the L1 PUSCH layers of the first panel corresponding to the first TPMI for CB PUSCH or the first SRI for the NCB PUSCH and the rest L- L1 of the indicated DMRS ports are associated with the L-L1 PUSCH layers of the second panels corresponding to the second TPMI for CB PUSCH or the second SRI for the NCB PUSCH. - FFS: how to partition the indicated DMRS ports among the PUSCH layers. |
| Google | Yes with modification | We think the second sub-bullet should be revised as follows: - L1 of the indicated DMRS ports are associated with the first indicated TPMI/SRI and the rest L- L1 of the indicated DMRS ports are associated with the second indicated TPMI/SRI. - FFS: how to partition the indicated DMRS ports. |
| NTT Docomo | Yes |  |
| ZTE | Yes with update | If two separated TPMI/SRI fields are supported in proposal 1-3, doesn’t it mean L1 and L2 (where L1+L2 = L) can be indicated by the first and second TPMI/fields directly? Regarding DMRS ports across two panels should be in the same or different CDM group, it is relative to Tx-chain/ panel architecture and the synchronization assumption of DMRS ports across two panels. If DMRS ports across two panels could be ideal synchronized, DMRS ports of two panels can be allocated in the same CDM group. Otherwise, DMRS ports of two panels should be in different CDM group. Hence Alt 2 should be supported. FL Proposal 1-2 - Reuse the DCI field ‘Antenna Ports’ in DCI format 0_1 and 0_2 to indicate DMRS ports for SDM scheme of single-DCI based STxMP PUSCH: - The total numbers of layers, L, indicated by two TPMI of CB PUSCH or two SRI fields of NCB PUSCH is used to determine the DMRS port indication table. - L1 of the indicated DMRS ports are associated with the L1 PUSCH layers which indicated by the first TPMI/SRI field and L2 of the indicated DMRS ports are associated with the L2 PUSCH layers which indicated by the second TPMI/SRI field. - FFS: how to partition the indicated DMRS ports among the PUSCH layers. - Down-select one from the following two Alts for SDM scheme in RAN1#110bis-e: - Alt1: the DMRS ports of two panels must be in different CDM groups. - Alt2: the DMRS ports of two panels can be in same or different CDM groups. |
| NEC | Yes |  |
| Mod2 |  | - Made wording change according to the comments by Qualcomm, Lenovo, google and ZTE - For Alt1 vs Alt 2: the list of supporting companies based on the proposal in Tdocs are also copied into the proposal. The supporting Alt 2 of interdigital is also added there. Please add your position on Alt 1 vs Alt 2 |
| Fujitsu | Yes | Alt1 is slightly preferred for ease of distinguish DMRS ports. |
| LG | Yes |  |
| Samsung | Yes | We prefer FL’s original proposal. We didn’t make details for TPMI/SRI fields and SRS resource set for STx2P. Therefore, more generally, we tried to specify relation between DMRS and panel first. Then, we can make relation between panel and SRS resource set (and TPMI/SRI) later. Among two alternatives for the CDM group, we don’t think the CDM group restriction is needed for UL transmission other than DL reception. We prefer Alt2 (the DMRS ports of two panels can be in same or different CDM groups). |
| MediaTek | Yes | For the second issue, we are fine with both alternatives. We think Alt 1 could provide better performance by reducing the interference between two TRPs but have less scheduling flexibility, and Alt 2 has better scheduling flexibility, but inter-TRP interference may be not handled well under some uncertainty on channel estimation. |
| Spreadtrum | Yes | Support FL Proposal 1-2. Support Alt 1 to ensure the orthogonality between DMRS ports in different CDM groups from two panels. If the DMRS ports of two panels are in same CDM group, the PUSCH decoding performance at TRP may deteriorate due to the different propagation delay and phase shift towards two TRPs. |
| OPPO | Yes | Support FL’s proposal. We prefer Alt 1 to reduce the interference between two panels. |
| Fraunhofer IIS/HHI | Yes | Support. Just a minor change. The language of the proposal goes from two TPMI/SRI fields to two panels. It can be maintained uniform throughout the proposal. For the two alternatives, the proposal can state “the DMRS ports associated with the two TPMI/SRI fields” instead of “the DMRS ports of two panels” |
| Xiaomi | Yes | For the second bullet of the proposal, we support Alt.2 as the scheduling flexibility can be maintained for the UL. |
| Apple | Yes | Support Alt2 |
| Mod3 |  | Made wording change in Alt1 and Alt2 according to the comments by Fraunhofer IIS/HHI The supporting companies list was updated by adding MediaTek, Apple to Alt2 according to the comments. |
| Huawei, HiSilicon | Yes with update. | First, we prefer the original form of L1 and L-L1 in the second sub-bullet which makes the first sub-bullet and second sub-bullet aligned. If companies prefer L1 and L2 in the second sub-bullet, it should be clarified L1+L2=L. Also, to be more accurate, we prefer to replace “first (second) TPMI/SRI field” with “first (second) TPMI field for CB PUSCH or the first (second) SRI field for the NCB PUSCH” as CB PUSCH also includes the SRI field. Finally, we prefer Alt1 due to the following reasons: - Alt 1 is more aligned with Rel-16 mTRP PDSCH wherein antenna ports correspond to two CDM groups each of which associated with a different TCI state (TRP). - Alt 1 can simplify the layer association with each panel. For instance, for both layer combinations 1+2 and 2+1, gNB can indicate DMRS entry {0,1,2} to UE as {0,1} and {2} are associated with two different CDM groups. Then, if the indicated layers in the first TPMI/SRI is 1 and the indicated layers in the second TPMI/SRI is 2, the DMRS entry {0,1,2} would correspond to 1+2 layer combination and if the indicated layers in the first TPMI/SRI is 2 and the indicated layers in the second TPMI/SRI is 1, the DMRS entry {0,1,2} would correspond to 2+1 layer combination - As some companies mentioned above, the delay between different TRPs and UE are different which results in different phase shifts at the TRPs. If, for instance, DMRS port {0} is transmitted to TRP 0 and DMRS port {1} is transmitted to TRP 1, then the orthogonality may be destroyed between them because of the phase shift. On the other hand, DMRS port {0,1} and DMRS port {2} belong to different CDM groups and are orthogonal by frequency division. In this case, the phase shift has no adverse effect on the orthogonality. FL Proposal 1-2 (updated) - Reuse the DCI field ‘Antenna Ports’ in DCI format 0_1 and 0_2 to indicate DMRS ports for SDM scheme of single-DCI based STxMP PUSCH: - The total numbers of layers, L, indicated by two TPMI of CB PUSCH or two SRI fields of NCB PUSCH is used to determine the DMRS port indication table. - L1 of the indicated DMRS ports are associated with the L1 PUSCH layers which indicated by the first TPMI/SRI field TPMI field for CB PUSCH or the first SRI field for the NCB PUSCH and L2 L-L1 of the indicated DMRS ports are associated with the L2 L-L1 PUSCH layers which indicated by the second TPMI/SRI field second TPMI field for CB PUSCH or the second SRI field for the NCB PUSCH. - FFS: how to partition the indicated DMRS ports among the PUSCH layers. - Down-select one from the following two Alts for SDM scheme in RAN1#110bis-e: - Alt1: the DMRS ports associated with two TPMI/SRI fields must be in different CDM groups. - Alt2: the DMRS ports associated with two TPMI/SRI fields can be in same or different CDM groups. |
| Intel | Yes | We support Alt1. |
| CATT | Yes | Alt2 is supported. It is not necessary for restricting CDM groups. The flexibility is deteriorated with such restriction. |
| Vivo | Yes | Support the first bullet. For the second bullet, we support Alt1. Cross-link interference of two panels is I. To ensure the performance of channel estimation, DMRS ports for two panels should be from two CDM groups. |
| Panasonic | Yes | For down-selection, further discussion is needed. But we can progress by agreeing to this proposal first. |
| Ericsson | Yes | Let’s try to agree on the first part first. We updated our preference on Alt1/Alt2. |
| Mod 4 |  | Thanks for the comments. The proposal is updated as follows: - Wording change according to HW’s comments. - Change the “110bis-e” to “111” it looks like we cannot done the down-selection in this meeting. |

## Issue #3: SRS resource configuration and TPMI/SRI indication for SDM scheme

### Summary

It was agreed to study the enhancement of SRS resource set configuration, SRI/TPMI indication for single-DCI based STxMP PUSCH in RAN1#109 meeting. For SDM scheme, contribution provides the following proposals.

Regarding the configuration of SRS resource sets for SDM scheme, companies (Huawei/HiSilicon, ZTE, Spreadtrum, vivo, Ericsson, OPPO, Google, LG, Intel, Xiaomi, Sharp, MediaTek, Apple, Samsung, NTT DOCOMO, Qualcomm, Panasonic) proposed to configure two SRS resource sets for either codebook-based PUSCH or non-codebook based PUSCH of SDM scheme.

Xiaomi also suggested that these two SRS resource sets could have different number of SRS resources and different maximum numbers of SRS ports.

Regarding the precoder and rank indication for SDM scheme, companies (ZTE, Spreadtrum, vivo, Ericsson, OPPO, Google, LG, CATT, Fujitsu, Intel, NEC, Xiaomi, Sharp, MediaTek, Apple, Samsung, NTT DOCOMO, Qualcomm) propose to use the two TPMI/SRI fields to separately indicate the precoder and/or ranks of each panel separately. In other word, the DCI indicates two TPMI/SRI fields for codebook based PUSCH and each TPMI field indicates the precoder(s) and rank for one panel separately. And The DCI indicates two SRI fields for non-codebook based PUSCH and each SRI field indicates SRI for one panel separately.

Vivo suggested to clarify that the rank combination indicated by two TPMI or SRI fields shall be one of the supported layer combinations. Companies also suggested to configure max rank per SRS resource set or TPMI field.

### Proposal for discussion

Based on the companies’ views, the following proposal is made:

FL Proposal 1-3: For SDM scheme of single-DCI based STxMP PUSCH:

- Configure two SRS resource sets for CB or NCB.

  - FFS: These two SRS resource sets can have different number of SRS resources for codebook-based or non-codebook based.

  - FFS: These two SRS resource sets can have different number of SRS ports for codebook-based.

- For codebook-based PUSCH, DCI indicates two TMPI fields and each TPMI field indicates the precoder and rank for each SRS resource set separately, which is interpreted as in Rel-15.

- For non-codebook based PUSCH and codebook-based PUSCH, DCI indicates two SRI fields and each SRI field indicates SRI for each SRS resource set separately.

- FFS: whether and how to associate each SRS resource set with a maxRank that might be different from the legacy maxRank, which is used to interpret the TPMI fields (for CB) or SRI fields (for NCB) in case of SDM scheme.

- FFS: how to indicate the precoder/rank/SRI for Type 1 CG-PUSCH

Companies’ views:

| Company Name | Support Proposal 1-3 (Yes/No) | Comments |
| --- | --- | --- |
| QC |  | We have the following comments: - Two SRIs are needed also for CB-based - maxRank is also needed to determine SRI fields interpretation for NCB-based. Also, whether one maxRank (common to both SRS resource sets but per SRS resource set, i.e., different than legacy maxRank) or two maxRank (separately configured for each of the two SRS resource sets) should be further discussed. We think one maxRank (other than legacy maxRank) may be enough - While indicated number of ports (CB) and resources (NCB) can be different, we are not sure if the configured number of SRS resources / configured max number of ports need to be different. More clarifications on the use case are needed. FL Proposal 1-3: For SDM scheme of single-DCI based STxMP PUSCH: - Configure two SRS resource sets for codebook based or non-codebook based. - FFS: These two SRS resource sets can have different number of SRS resources. - FFS: These two SRS resource sets can have different maximum number of SRS ports. - For codebook-based PUSCH, DCI indicates two TMPI fields and each TPMI field indicates the precoder and rank for each panel separately. - Each TMPI field is configured with a maxRank separately. - For non-codebook based PUSCH and codebook-based PUSCH, DCI indicates two SRI fields and each SRI field indicates SRI for each panel separately. - Each SRS resource set is associated with a configured maxRank=1 or 2 different than the legacy maxRank, which is used to interpret the TPMI fields (for CB) or SRI fields (for NCB) in case of SDM scheme - FFS: Whether one configured maxRank common to the two SRS resource sets is enough or not. - The numbers of layers indicated by the two TPMI or SRI fields shall be one of the supported layer combinations for SDM scheme |
| Nokia/NSB | Yes (with updates) | First main bullet: Support Second main bullet: We support that DCI indicates two TPMI fields and each TPMI field indicates the precoder and rank for each panel separately. However, it is difficult to justify the need for TPMI specific maxRank. For us, maxRank configured in PUSCH-Config can be used as the total maxRank such that the sum of layers associated with TPMI/SRIs is limited by maxRank. For S-DCI, all PUSCH transmissions and related layers are scheduled in the centralized manner. Third main bullet: Support Fourth main bullet: Support |
| InterDigital | Yes | A single maxRank seems to be sufficient to indicate the layers across TPMI/SRIs. |
| Lenovo |  | - For the first bullet on SRS resource set configuration: do not support both sub-bullets. - For the second bullet: separate SRI fields are also needed for CB PUSCH. - For the third and the fourth bullet: support. |
| Google | Yes with modification | We suggest the following revision: - Configure two SRS resource sets for codebook based or non-codebook based. - These two SRS resource sets can have different number of SRS resources. - These two SRS resource sets can have different number of SRS ports. - For codebook-based PUSCH, DCI indicates two TMPI fields and each TPMI field indicates the precoder and rank for each panel separately. - FFS: Each TMPI field is configured with a maxRank separately. - For non-codebook based PUSCH, DCI indicates two SRI fields and each SRI field indicates SRI for each panel separately. - The numbers of layers indicated by the two TPMI or SRI fields shall be aligned with the layer combinations indicated by the DMRS antenna ports for SDM scheme |
| NTT Docomo | Yes with updates | We share same view with QC that maxRank is also needed for NCB, and share same view with QC/Nokia that how to configure maxRank can be further discussed. |
| ZTE | Yes with update | Support proposal 1-3 in principle. Basically, two separated TPMI/SRI fields should be used for both CB PUSCH and NCB PUSCH. Regarding two SRS resource sets, independent parameter configuration (i.e., SRS resource #and SRS ports#) should be supported especially for dynamic switching between single-panel PUSCH and STxMP PUSCH. More precisely, single-panel mode via panel-1 and panel-2 can be performed with different parameter configuration. Furthermore, considering the supported maximum layer of single-panel mode can be larger than that of STxMP mode, (e.g., 2-layer PUSCH from panel-1 in STxMP SDM mode can switch to 4-layer PUSCH from panel-1 in single-panel mode), hence it is reasonable to support each TPMI field is configured with a maxRank separately for CB scheme and each SRS resource set configured with a NSRS separately for NCB scheme. Where, maxRank and NSRS should be the same to legacy. Otherwise, the two RRC parameters need to be reconfigured for single-panel mode, that deviates from the design of dynamic switching between single-panel mode and STxMP mode. In addition, it is better to use SRS resource set to replace panel/TRP. FL Proposal 1-3: For SDM scheme of single-DCI based STxMP PUSCH: - Configure two SRS resource sets for codebook based or non-codebook based. - These two SRS resource sets can have different number of SRS resources. - These two SRS resource sets can have different maximum number of SRS ports. - For codebook-based PUSCH, DCI indicates two TMPI fields and each TPMI field indicates the precoder and rank for each SRS resource set separately. - Each TMPI field is configured with a maxRank separately. - For non-codebook based PUSCH and codebook based PUSCH, DCI indicates two SRI fields and each SRI field indicates SRI for each SRS resource set separately. - The total numbers of layers indicated by the two TPMI or SRI fields jointly shall be one of the supported layer combinations for SDM scheme |
| NEC | Yes (may need updates) | We also have concerns about NCB, which is related to maximum number of layers for PUSCH supported Lmax (not only maxRank). Our preference is to manage it per panel, e.g., to have Lmax,1 and Lmax,2 respectively. |
| Mod2 |  | Thanks for the comments. The proposal is updated according to the comments. Re the comments by QC of associating each SRS resource set with a maxRank that is different from the legacy maxRank. Nokia commented that legacy maxRank can be used. So the proposal from QC is put in FFS sub-bullet. @ZTE and google: the purpose of last bullet is to clarify that the indicated layers by one DCI shall be one of the layer combinations that we agreed to support. It means that not only the total number of layers, but also the combination of {L1,L2}. So no update in last bullet. |
| Fujitsu | Yes |  |
| LG | Yes with updates | First bullet: In NCB case, all SRS resources should have a single port, so we suggest split the case for CB and NCB regarding the number of ports. - Configure two SRS resource sets. - FFS: for codebook based or non-codebook based, these two SRS resource sets can have different number of SRS resources. - FFS: for codebook based, these two SRS resource sets can have different number of SRS ports. |
| Samsung | Yes | Okay with updated proposal and LG’s update is clearer. However, to exploit all antenna ports across two panels for sTRP transmission, we think sTRP STx2P should be considered. Therefore, we suggest following modification FL Proposal 1-3: For SDM scheme of single-DCI based STxMP PUSCH: - Configure two SRS resource sets. - FFS: for codebook based or non-codebook based, these two SRS resource sets can have different number of SRS resources. - FFS: configure two SRS resource groups (like R17 NCJT CSI) in a SRS resource set to support single-DCI based sTRP STxMP PUSCH - FFS: for codebook based, these two SRS resource sets can have different number of SRS ports. |
| Mediatek | Yes with update | We agree to have separate max rank configurations for two SRS resource set, but one maxRank field is enough. Besides, to support dynamic switching between S-TRP and M-TRP operation, the up to 4 instead of up to 2 should be considered for the maxRank field. In the S-DCI based STxMP PSUCH to two TRPs, the max rank for second SRI/TPMI field could be specified as constant (ex. 2), where the constant value for the other SRS resource set may be decided according to the agreement of layer combination for SDM (Now, up to 2 layers for a TRP is majority). Moreover, both CG-PUSCH and DG-PUSCH should be considered in STxMP discussion, such that we suggest having the following updated proposal: FL Proposal 1-3: For SDM scheme of single-DCI based STxMP PUSCH: - Configure two SRS resource sets for codebook based or non-codebook based. - FFS: These two SRS resource sets can have different number of SRS resources. - FFS: These two SRS resource sets can have different number of SRS ports. - For codebook-based PUSCH, Two TMPI fields are indicated/configured and each TPMI field indicates/configures the precoder and rank for each SRS resource set separately. - For non-codebook based PUSCH and codebook-based PUSCH, Two SRI fields are indicated/configured and each SRI field indicates/configures SRI for each SRS resource set separately. - FFS: whether and how to associate each SRS resource set with a maxRank that might be different from the legacy maxRank, which is used to interpret the TPMI fields (for CB) or SRI fields (for NCB) in case of SDM scheme. - The numbers of layers indicated by the two TPMI or SRI fields shall be one of the supported layer combinations for SDM scheme |
| Spreadtrum | Yes | Support updated FL Proposal |
| OPPO | Yes | For third bullet, suggest to use ‘SRS resource set’ to replace ‘panel’. |
| Fraunhofer IIS/HHI | Yes |  |
| Xiaomi | Yes | Support the proposal, and LG’s revision of the first bullet is fine to us. |
| Apple | Yes | Support the updated FL’s proposal |
| Mod 3 |  | The proposal is updated as follows: - The first bullet and its sub-bullets are updated according to the comments of LG, Samsung, Xiaomi. - Update the 3rd bullet according the comments of MediaTek. @Samsung: your suggested FFS sub-bullet seems confusing. By “sTRP”, do you mean the Rel-15/16 sTRP transmission? If so, in it the transmission does not support explicit “two panels”. Just as specified for Rel-15/16 sTRP transmission, the UE is ndicated with one TMPI and one SRI and it is up to UE implementation to use one or more panels to transmit one PUSCH. |
| Intel | Yes | Support the updated FL’s proposal. We prefer to have separate maxRank configurations. |
| Huawei, HiSilicon | Yes with update. | OK in general although we prefer to simplify the FFS regarding max rank. We suggest the following. FL Proposal 1-3 (update): For SDM scheme of single-DCI based STxMP PUSCH: - Configure two SRS resource sets. - FFS: These two SRS resource sets can have different number of SRS resources for codebook-based or non-codebook based. - FFS: These two SRS resource sets can have different number of SRS ports for codebook-based. - For codebook-based PUSCH, DCI indicates two TMPI fields and each TPMI field indicates the precoder and rank for each SRS resource set separately. - For non-codebook based PUSCH and codebook-based PUSCH, DCI indicates two SRI fields and each SRI field indicates SRI for each SRS resource set separately. - FFS: whether and how to associate each SRS resource set with a maxRank that might be different from the legacy maxRank, which is used to interpret the TPMI fields (for CB) or SRI fields (for NCB) in case of SDM scheme. - The numbers of layers indicated by the two TPMI or SRI fields shall be one of the supported layer combinations for SDM scheme |
| ZTE2 |  | Note that each SRS resource set could be used to both STxMP PUSCH and TxSP PUSCH, that means RRC configured maxRank depends on the larger value between that of the legacy (up to 4) and Rel-18 (up to 2/3). We fail to see the necessity of including “… a maxRank that might be different from the legacy maxRank” at the current stage. It should be noticed that indicated layer number and configured maxRank could be used for SDM scheme as well as single panel scheme. FL Proposal 1-3: For SDM scheme of single-DCI based STxMP PUSCH: - Configure two SRS resource sets. - FFS: These two SRS resource sets can have different number of SRS resources for codebook-based or non-codebook based. - FFS: These two SRS resource sets can have different number of SRS ports for codebook-based. - For codebook-based PUSCH, DCI indicates two TMPI fields and each TPMI field indicates the precoder and rank for each SRS resource set separately. - For non-codebook based PUSCH and codebook-based PUSCH, DCI indicates two SRI fields and each SRI field indicates SRI for each SRS resource set separately. - FFS: whether and how to associate each SRS resource set with a maxRank, which is used to interpret the TPMI fields (for CB) or SRI fields (for NCB) in case of SDM scheme as well as when switch to single panel scheme. - The numbers of layers indicated by the two TPMI or SRI fields in case of SDM scheme shall be one of the supported layer combinations for SDM scheme |
| MediaTek |  | We have concern to remove CB/NCB from the main bullet and suggest to keep them. FL Proposal 1-3: For SDM scheme of single-DCI based STxMP PUSCH: - Configure two SRS resource sets for CB or NCB Since current proposal works for DG-PUSCH and maybe Type2 CG-PUSCH, we suggest to add one FFS for Type1 CG-PUSCH. - FFS: How to indicate the precoder/rank/SRI for Type1 CG-PUSCH |
| CATT | Yes | Support FL’s proposal. |
| Vivo | Yes | Support the updated proposal with a minor change. FL Proposal 1-3: For SDM scheme of single-DCI based STxMP PUSCH: - Configure two SRS resource sets for codebook based or non-codebook based. - FFS: These two SRS resource sets can have different number of SRS resources. - FFS: These two SRS resource sets can have different number of SRS ports. - For codebook-based PUSCH, DCI indicates two TMPI fields and each TPMI field indicates the precoder and rank for each SRS resource set separately. - For non-codebook based PUSCH and codebook-based PUSCH, DCI indicates two SRI fields and each SRI field indicates SRI for each SRS resource setpanel separately. - FFS: whether and how to associate each SRS resource set with a maxRank that might be different from the legacy maxRank, which is used to interpret the TPMI fields (for CB) or SRI fields (for NCB) in case of SDM scheme. - The numbers of layers indicated by the two TPMI or SRI fields shall be one of the supported layer combinations for SDM scheme |
| Samsung |  | @FL: As we explained in Issue#1, this is for explicit sTRP multi-panel transmission, not Rel-15/16 sTRP transmission. We think the gNB can support more flexible scheduling with this explicit sTRP STx2P. Therefore, we try to consider the added FFS (SRS resource group in the set) as an option for detail SRS resource set configuration. |
| Panasonic | Yes | Support the updated proposal |
| Ericsson |  | Basically OK, but too complicated. It is unclear what the rank of an SRS resource set is: propose to write - For codebook-based PUSCH, DCI indicates two TMPI fields and each TPMI field is interpreted as in Rel-15. Remove the final bullet. |
| Mod 4 |  | The proposal is updated as follows: - Per the comments of HW and ZTE, the bullet of FFS on maxRank is updated. - According to the comments of MediaTek, one FFS bullet is added and CB or NCB is added to the first bullet. - According to the comment of Ericsson, “which is interpreted as in Rel-15” is added to clarify. - @Samsung: your suggestion can be next step details, you could bring it up for next step detail discussion. |

### Summary of round 1

In Tuesday GTW session, we have the following version:

FL Proposal 1-3

For SDM scheme of single-DCI based STxMP PUSCH

- Configure two SRS resource sets for CB or NCB.

  - FFS: These two SRS resource sets can have different number of SRS resources for codebook-based or non-codebook based.

  - FFS: These two SRS resource sets can have different number of SRS ports for codebook-based.

- For codebook-based PUSCH, DCI indicates two TPMI fields

  - Each TPMI field is interpreted as in Rel-15

- For non-codebook based PUSCH and codebook-based PUSCH, DCI indicates two SRI fields and each SRI field indicates SRS resource(s) for each SRS resource set separately.

The moderator is going to move the discussion of this proposal to email.

## Issue #4 : Layer combination for SDM scheme

### Summary

It was agreed to support layer combinations of {1+1, 1+2, 2+1 and 2+2} for SDM scheme. And one FFS issue is whether to support {1+3}, {3+1}.  Another FFS is the option of {0+n, n+0}.

Agreement For single-DCI based STxMP PUSCH SDM scheme, support the layer combinations of {1+1, 1+2, 2+1 and 2+2} for single CW case. - FFS on layer combinations of {1+3} and {3+1} considering the performance gain, system/UE complexity, specification efforts, etc. - FFS: the option of using layer combination of 0+n and n+0 for dynamic switch between single-panel and STxMP (n=1 or 2, 3 or 4). - This applies to SDM with 1 CW at least.

Regarding the layer combination of 1+3 and 3+1, contributions provides the following views:

- Alt1: do not support 1+3 and 3+1

  - Support: Huawei/HiSilicon, Spreadtrum, vivo, Ericsson, OPPO, Google, CATT, CMCC (if 2CW is not supported), Qualcomm, Panasonic

- Alt2: support 1+3 and 3+1

  - Support: InterDigital, ZTE, Xiaomi, Nokia

Regarding the layer combinations of 0+n and n+0 for dynamic switch,

- Ericsson explained that applying two TMPI/SRI fields would support dynamic switch straightforwardly.

- Google suggested to defer the discussion after more details on transmission schemes for STxMP are concluded.

- Sharp suggested to support them for dynamic switch. Qualcomm proposed to not support additional layer combinations.

- Panasonic does not support to use 0+n and n+0 for dynamic combination.

- Nokia explained that the function of panel switch 0+n and n+0 can be supported by DCI field SRS resource set indicator and thus they are not needed.

FL note: it looks like that majority companies do not support 1+3 and 3+1. Considering this issue has been discussed couple of times, it is better to make a conclusion in this meeting. Re the issue of 0+n and n+0, its purpose is to support dynamic switch between SDM scheme and Rel-15/16 sTRP transmission. FL would suggest to focus on the discussion on whether/how to support dynamic switch.

### Proposal for discussion

Based on the companies’ views, the following proposal is made:

FL Proposal 1-4: Conclude that layer combination 1+3 and 3+1 are not supported for SDM scheme of single-DCI based STxMP PUSCH in Rel-18.

Companies’ views:

| Company Name | Support Proposal 1-4 (Yes/No) | Comments |
| --- | --- | --- |
| QC | Yes |  |
| Nokia/NSB | Yes | For the sake of progress, we are fine to support it. |
| InterDigital |  | This may depend on the outcome from Issue #5 on 2CW support. If 2 CW is not supported, then we are ok to support this proposal. |
| Lenovo | Yes | Support. |
| Google | Yes |  |
| NTT Docomo | Yes |  |
| ZTE | No | Given layer combination 2+2 has been agreed, we fail to see the logical that 1+3 and 3+1 should be precluded. For NCB scheme, if the MP-UE could support 2+2, it means 4 SRS resources across two panel can be used, so1+3/3+1 can be supported accordingly. Then why NW cannot schedule such layer combinations in this case? |
| Fujitsu | Yes |  |
| LG | No | Supporting 1+3 and 3+1 provides scheduling flexibility to optimize rank for each TRP’s UL channel. |
| Samsung | Yes | We prefer to prioritize layer combinations which were already agreed (1+1, 1+2, 2+1, 2+2). |
| MediaTek | Yes | This is a UE implementation issue, and we fail to see the need to additional layer combinations. |
| Spreadtrum | Yes |  |
| OPPO | Yes |  |
| Fraunhofer IIS/HHI | Yes |  |
| Xiaomi | No | More flexible layer combinations should be considered and supported, for RANK-4, only {2+2} is supported. And for the asymmetric panel implementation cases (various UE capability values have been supported), supporting of RANK-4 with {1+3, 3+1} would be a natural choice. These rank combinations are also applicable for 2CW to layer mapping case. |
| Apple | Yes | 1+3/3+1 is not well justified, besides excluding this combination simplifies other design aspects like maxRank |
| Intel | No | {1+3, 3+1} can provide more flexibility if the UE has certain capability. For example, a UE can support dynamic switching between sTRP and STxMP transmission. If the sTRP transmission is 4-layer, then the UE should have the capability of 3-layer transmission in STxMP, which means the {1+3, 3+1} layer combinations are supportable. |
| Huawei, HiSilicon | Yes | rank 3 transmission for a single TRP is not a typical scenario in FR2. Further, it was agreed in Rel-16 that at most {2+2} layer combination is supported for the downlink mTRP. Finally, if the layer combination {1+3} is supported, the number of Tx antenna ports for the first panel must be at least one, while, according to the current codebook design, the number of Tx antenna ports for the second panel must be 4 to support a 3-layer TPMI indication (see Table 6.3.1.5-6 in 38.211). Note that, according to the WID, “no new codebook is introduced for multi-panel simultaneous transmission”. This means that to support {1+3} or {3+1} layers, the total number of Tx antenna ports over two panels needs to be at least 5. However, the current specifications do not support more than 4 antenna port UE. Therefore, we have the following proposal. |
| CATT | Yes |  |
| vivo | Yes | We fail to see the necessity to support layers combination of {1+3} and {3+1}. Performance gain of layer combinations s of {1+3} and {3+1} compared with rank combination of {2+2} is unclear. Besides, considering per-panel power control, similar channel quality of two panels can be acquired. Scheduling of PUSCH with layer combinations of {1+3} or {3+1} seldom happens. |
| Panasonic | Yes | Support |
| Ericsson | Yes | The entire evaluation campaign assumed that each TRP could only receive with two layers, and that each UE panel was only capable of rank-2 transmission. Without that restriction, any benefit of STxMP would go away. |

### Summary of Round 1

From the round 1 discussion, we have the following supporting companies:

FL Proposal 1-4: Conclude that layer combination 1+3 and 3+1 are not supported for SDM scheme of single-DCI based STxMP PUSCH in Rel-18.

Support: QC, Nokia/NSB, InterDigital (if 2CW is not supported), Lenovo, Google, NTT DOCOMO, Fujitsu, Samsung, MediaTek, Spreadtrum, OPPO, Fraunhofer IIS/HHI, Apple, Huawei, HiSilicon, CATT, vivo, Panasonic, Ericsson

Not support: ZTE, LG, Xiaomi, Intel,

Intel/Xiaomi/LG explained that 1+3 and 3+1 can provide more scheduling flexibility and considering a UE can support switch between sTRP and STxMP transmission, a UE supports 3 layers for sTRP can naturally support combination of 1+3 and 3+1 in STxMP.

The companies objecting 1+3 and 3+1 explained that these combinations are not well justified and they prefer to simply the design.

Per the understanding of moderator, it seems more study is needed to make decision on these two layer combinations.

### Proposal for Round 2 discussion

Updated Proposal 1-4: Continue to study layer combinations {1+3} and {3+1} for SDM scheme considering the performance gain, system/UE complexity, specification efforts. Decision on support them or not to be made in RAN1#111.

Comments on Updated Proposal 1-4:

| Company | Support (Yes/No) | Comments |
| --- | --- | --- |
| QC | No | This is already FFS. We do not need a new agreement. This should be a “conclusion” given the situation. |
| LG | Yes |  |
| NTT Docomo |  | Similar view with QC that a new agreement may not be needed |
| Fujitsu | Yes |  |
| MediaTek | No | Study on this issue was agreed in RAN1#110, such that no more agreement is needed |
| CATT | No | Fail to see the necessity for a new agreement. |
| ZTE | Yes | Note that some companies commented 1+3 or 3+1 is relevant to the case of 2 CW SDM, we’d like to clarify that up to 2 layers per panel is set in our SLS with regards to 2 CW case. It proves whether to support 1+3/3+1 is decoupled to 2 CW SDM. |
| OPPO |  | We also think new agreement may not be needed. |
| Xiaomi | Yes |  |
| InterDigital | Yes |  |
| Intel | Yes |  |
| Huawei, HiSilicon | No | As we discussed in the first round, {1+3} and {3+1} layer combinations are out of the scope of this WID: if the layer combination {1+3} is supported, the number of Tx antenna ports for the first panel must be at least one, while, according to the current codebook design, the number of Tx antenna ports for the second panel must be 4 to support a 3-layer TPMI indication (see Table 6.3.1.5-6 in 38.211). Note that, according to the WID, “no new codebook is introduced for multi-panel simultaneous transmission”. This means that to support {1+3} or {3+1} layers, the total number of Tx antenna ports over two panels needs to be at least 5. However, the current specifications do not support more than 4 antenna port UE. Also, rank 3 transmission for a single TRP is not a typical scenario in FR2. Further, it was agreed in Rel-16 that at most {2+2} layer combination is supported for the downlink mTRP. Finally, |
| Spreadtrum |  | Agree with QC, the original FFS in RAN1#110 can be retained and used for subsequent discussion. |
| Nokia | Yes | It is too early discard this option. |
| Samsung | No |  |
| Lenovo | Yes |  |
| Mod |  | @All, indeed, we had FFS back in RAN1#110, but this new proposal is to say RAN1#111 is the deadline to make the decision. Please check if you are ok with this deadline. |

## Issue#5 2 CWs for SDM scheme

### Summary

Whether to support 2 CW for SDM scheme is pending issue.

The following 5 companies provided the evaluation results for 2CW:

- ZTE provided the following SLS results for 2 CWs vs 1 CW: Table 1 UPT performance of 1 CW v.s. 2 CWs in case of SDM scheme based STxMP PUSCH transmission RU; Transmission Scheme; Mean UE; 5%-ile UE; 50%-ile UE; 95%-ile UE. ~ 15%; Single panel with panel selection; 430.64; 187.05; 497.44; 524.71. 1 CW SDM STxMP; 474.71 10.2% (↑); 174.64 -6.6% (↓); 493.72 -0.7% (↓); 895.34 70.6% (↑). 2 CWs SDM STxMP; 494.11 14.7% (↑); 191.32 2.3% (↑); 512.59 3.0% (↑); 895.50 70.7% (↑). ~ 30%; Single panel with panel selection; 357.51; 111.00; 379.38; 524.71. 1 CW SDM STxMP; 387.08 8.3% (↑); 108.57 -2.2% (↓); 382.65 0.9% (↑); 790.22 50.6% (↑). 2 CWs SDM STxMP; 406.76 13.8% (↑); 119.03 7.2% (↑); 404.74 6.7% (↑); 808.84 54.2% (↑). ~ 50%; Single panel with panel selection; 262.11; 60.14; 234.78; 524.57. 1 CW SDM STxMP; 273.48 4.3% (↑); 57.96 -3.6% (↓); 237.31 1.1% (↑); 539.37 2.8% (↑). 2 CWs SDM STxMP; 290.84 11.0% (↑); 68.25 13.5% (↑); 253.33 7.9% (↑); 591.31 12.7% (↑). ~ 80%; Single panel with panel selection; 121.06; 20.09; 72.99; 416.95. 1 CW SDM STxMP; 118.43 -2.2% (↓); 19.79 -1.5% (↓); 68.25 -6.5% (↓); 408.14 -2.1% (↓). 2 CWs SDM STxMP; 128.21 5.9% (↑); 20.96 4.3% (↑); 75.25 3.1% (↑); 422.26 1.3% (↑). Figure 1 MCS level gap between two panels when 2 CWs SDM based STxMP PUSCH transmission

- Ericsson provided the SLS results of 2 CWs vs 1 CW and suggested that performance gain of 2 CWs is marginal: Figure 1: Mean and cell-edge throughput for indoor hotspot for STxMP one and two codewords. Figure 2: Mean and cell-edge throughput for dense urban for STxMP one and two codewords

- Lenovo provided the throughput comparison between 2 CWs vs 1 CW and observed 2 CW outperforms 1 CW in medium and high SNR region

- MediaTek provided the CDF of MCS level gap between two panels and suggested that the benefit of 2 CWs could be quite marginal CDF of MCS level gap between two codewords in SDM scheme under DU scenarios with different loading

- NTT DOCOMO compared the 2 CW vs 1 CW in terms of system throughput and observed that 2 CWs outperforms both 1 CW SDM and single panel TX in both low and high RU cases. Table I. Throughput performance of STxMP PUSCH SDM scheme RU; (Mbps); Single panel Tx; SDM scheme (1CW); SDM scheme (2CW). 20%; Avg. UPT; 251.02; 266.19 6.04% (↑); 272.07 8.39% (↑). 5% UPT; 81.84; 90.69 10.81% (↑); 95.33 16.48% (↑). 50%; Avg. UPT; 193.95; 198.42 2.31% (↑); 209.62 8.08% (↑). 5% UPT; 46.80; 43.24 -7.60% (↓); 58.97 26.01% (↑).

FL note: the evaluation results provided in tdoc can be summarized as:

- ZTE and NTT DOCOMO showed that SDM with 2 CW provided better throughput than both SDM with 1 CW and single panel transmission from low load to high load.

- Lenovo observed that SDM with 2 CW outperforms SDM with 1 CW in medium and high SINR region.

- Ericsson and MediaTek saw the gain of SDM with 2CW over SDM with 1 CW but explained that the gain might be marginal.

.

Regarding whether to support 2CWs in SDM scheme, the contributions provided the following views:

- Alt1: Support 2 CWs for SDM scheme

  - Support: InterDigital, ZTE, CATT, Xiaomi, NTT DOCOMO

- Alt2: Do not support 2 CWs for SDM scheme

  - Support:  Spreadtrum, vivo, Ericsson, Intel, MediaTek, Apple, Samsung, Panasonic, Nokia

The main arguments for not supporting 2 CW for SDM scheme provided in the contribution include: heavy specification impact due to changes in HARQ process and layer to codeword mapping, benefit of 2 CW scheduling may be hardly obtained, extra DCI overhead due to indicating MCS/RV/NDI for the second CW. The benefit of 2 CWs over 1 CW is marginal and could be even smaller with per panel power control, etc.

FL note: the views on 2 CWs for SDM quite diverge. The evaluation results provided in tdocs do show the gain of 2 CW but companies is concerned about the specification effort and complexity, and also possible marginal gain.

### Proposal for discussion

Regarding the issue of 2 CWs, please provide your views if you have not or update your views. The views shown in tdocs are also listed under each Alt.

FL Proposal 1-5: Regarding 2 CWs for SDM scheme, down-select one from the following:

- Alt1: Support 2 CWs for SDM scheme

  - Support: InterDigital, ZTE, CATT, Xiaomi, NTT DOCOMO, Lenovo, google

- Alt2: Do not support 2 CWs for SDM scheme

  - Support:  Spreadtrum, vivo, Ericsson, Intel, MediaTek, Apple, Samsung, Panasonic, Nokia, LG, Fraunhofer IIS/HHI, Apple

Companies’ views:

| Company | Support Alt1 or Alt2? | Comments |
| --- | --- | --- |
| QC | Alt2 | For DL, we do not have 2 CWs for SDM scheme. Why we should have this now for UL? The use case in UL is even more questionable since power control can balance the two links at least when UE is not power-limited. |
| Nokia/NSB | Alt2 | Support Alt2 |
| InterDigital | Alt1 | Support Alt1. |
| Lenovo | Alt1 | Support Alt1. |
| Google | Slightly prefer Alt1 |  |
| NTT Docomo | Alt1 |  |
| ZTE | [Change the table format to easily ventilate our views below] Support Alt1. Basically, as WID states, STxMP UL transmission must meet the higher UL throughput/ reliability when comparing baseline scheme (i.e., single-panel with panel selection or MTRP PUSCH TDM repetition). According to evaluation results of 1 CW SDM vs 2 CWs SDM by companies so far, it indeed proves that support of 2 CWs is the key point to reach meaningful performance gain for STxMP PUSCH SDM transmission. - ZTE shows that 1 CW SDM will always cause performance loss in low/medium/high load scenarios, and 2 CWs SDM could always get higher UPT. Wherein, TRP/panel-specific power control is enabled. In addition, ZTE also shows that most of MP-UEs need to be scheduled with different MCS for STxMP PUSCH SDM transmission when indoor scenario. - Ericsson assesses that performance gain of 2 CWs is marginal when comparing 1 CW. However, the SLS result shows that 2 CWs could obtain considerable UPT gain (~ 80% to 100%) for cell-edge Ues when medium load scenario. Besides, it should be noticed that performance of baseline scheme (single-panel with panel selection) should be compared herein. - Mediatek provides the result of MCS level gap between 2 CWs when DU scenario, where more than 50% MP-Ues are scheduled with different MCS. In particular, Mediatek’s simulation result provided in RAN1#110-e showed that 2 CWs SDM cannot provide sufficient UPT gain over 1 CW SDM. Note that the setting of MCS determination in 1 CW case is to jointly calculate the equalization of MCS across two TRPs, that is impractical due to the single MCS should be determined by the lower MCS among two panels to guarantee the demodulation performance of the worse panel. Otherwise, it is agnostic to NW to schedule STxMP PUSCH in this case. - Lenovo shows 2 CWs is highly required in medium and high SINR scenarios due to individual MCS could match each panel-TRP-link. - DOCOMO shows 1 CW SDM cannot outperform single-panel scheme in medium load scenario, but 2 CW could still guarantee UPT gain. Besides, it should be noticed that the traffic backhaul overload caused by 1 CW SDM scheme will be a Herculean challenge to NW deployment in reality. Due to the single date coding/ scrambling/ RV version/ modulating/ layer mapping of 1 CW SDM scheme, NW has to receive two PUSCHs at two TRPs completely at first and then process the combination of the receiving data jointly. It is worth to note that the processing of 1 CW SDM is the same to single-panel PUSCH transmission from NW’s perspective, where the only differentiation is the layers of PUSCH are transmitted with different beams. BTW, the processing of Rel-16 SDM MTRP PDSCH in UE side is also the same to that of STRP PDSCH. Keep the above in mind, when NW schedules several MP-Ues with 1 CW SDM scheme and the amount of receiving data in total within one occasion in each TRP side is considerably huge, it will lead to the traffic backhaul overload across TRPs. Meanwhile, it will also cause large traffic delay by wait to stop, which will be devastating to URLLC traffic, e.g., vehicle device enabled with multi-panel. For 2 CW SDM scheme, thanks to individual processing of each CW in each TRP, it can ease the pressure of NW deployment because traffic backhaul load is not required anymore. Taken the meaningful performance gain and NW deployment complexity into consideration, we do believe SDM STxMP PUSCH transmission must be enabled with 2 CWs. |  |
| Mod |  | Updated the list of supporting companies according to the latest comments. |
| LG | Alt 2 |  |
| Samsung | Alt2 | We want to share the similar view as QC. If two links between each panel and each TRP are not balanced even though power control, why should we support mTRP STx2P? When we consider the condition that we can get the maximum gain with mTRP, it could be the case that two channel links have similar or same quality. Otherwise, panel selection scheme or sTRP STx2P seems better than keeping mTRP STx2P. Therefore, 1CW with same MCS is enough to support SDM mTRP STx2P. For the backhaul, is this scheme for sDCI? For sDCI based scheme, we assume the ideal backhaul as we known. |
| MediaTek | Alt 2 | Support Alt 2. We fail to see the benefit of 2CW. First, we believe that MCS determination based on the layers across two TRPs is reasonable for 1CW case, because those layers belong to the same codeword. And joint/separate demodulation between TRPs is up to NW implementation in S-DCI-based operation. Second, The MCS gap distribution between two CWs in our contribution proves that the received power will be aligned by per-panel/TRP power control when the UE has sufficient UL power. And Ericsson also print out the observation that “With per-panel power control, the benefits of two codewords compared to one codeword would be even smaller.”. Third, we think the SDM scheme is designed for T-put enhancement instead of reliability enhancement. The cell-average throughput should be main KPI to justify the benefit of 2CW. Based on ZTE’s and DOCOMO’s results, 2CW provides less than 10% gain on cell-average UPT under any load, comparing to 1 CWs case. In our view, 2CW isn’t so attractive with large encoding complexity as cost |
| Spreadtrum | Alt2 | Support Alt2 |
| Fraunhofer IIS/HHI | Alt2 |  |
| Xiaomi | Slightly prefer Alt.1 |  |
| Apple | Alt2 | We share similar view as Samsung and Qualcomm |
| Mod 3 |  | Update the list of supporting companies according to comments. @MediaTek: I guessed you meant to say “Alt1” since in your comments, you said “We fail to see the benefit of 2CW” |
| Intel | Alt2 | Support Alt2 |
| Xiaomi | Slightly prefer Alt.1 | Wrong comment above sorry. 2 CWs can provide more scheduling flexibility which have more performance gain too. |
| Huawei, HiSilicon | Alt2 | We are not convinced 2 CW causes a meaningful performance gain that justifies the substantial specification effort. The discussion of 1 CW vs 2 CW is for the s-DCI scenario which assumes ideal backhaul. So, we don’t think the backhaul overload can be used to motivate 2 CWs. Second, though 2CW has benefits in flexibility on MCS selection, considerable performance gain only occurs when link quality of the two TRPs has a large gap, however, we believe this is not a typical use case for STxMP, wherein link quality of two TRPs should be comparable with each other. Moreover, based on performance evaluation provided by other companies in Rel16 and Rel18, this benefit requires accurate MCS determination. |
| ZTE2 | We’d like to share further views on top of companies’ inputs so far. QC, Samsung, MediaTek said that as long as per panel/TRP power control is enabled and assuming UE output power is unlimited, that’s sufficient to obtain UPT gain by 1 CW SDM. First, our evaluation results has already proved that enabling panel/TRP power control for 1 CW SDM cannot outperform TxSP with panel selection in any case (low-load to high-load scenarios). Second, it makes no sense to assume enough UE power is the pre-requisite of multi-panel devices. Note that two UE power options set in EVM and the diversity of MP-UE architecture in WID. Moreover, higher output UL power leads to severe inter-UE interference, which will negatively impact UPT instead. Third, due to UL output power is quite limited rather than that of DL, the design of Rel-16 MTRP PDSCH SDM cannot be the reason to preclude 2 CWs for SDM STxMP PUSCH transmission. Samsung’s and HW’s view on ideal back-haul assumption of SDCI MTRP is irrelevant to the traffic backhaul overload caused by 1 CW in MTRP deployment, it is indeed the processing of 1 CW where NW has to execute the combination of PUSCH receptions from two TRPs first anyways before demodulate/decoding the 1CW of PUSCH, that will case traffic backhaul overload among two TRPs especially in case of high traffic scenario, and which is the bottleneck of MTRP deployment in reality. Samsung said that “When we consider the condition that we can get the maximum gain with mTRP, it could be the case that two channel links have similar or same quality” and HW said that “... performance gain only occurs when link quality of the two TRPs has a large gap, however, we believe this is not a typical use case for STxMP, wherein link quality of two TRPs should be comparable with each other.”, it is indeed counter intuitive due to PUSCH transmitted from different panels toward to different TRPs in FR2. We proves that most of MP-Ues need to be scheduled with different MCS in above Figure 1. In addition, Samsung thought that “Otherwise, panel selection scheme or sTRP STx2P seems better than keeping mTRP STx2P”, it is the case of 1 CW SDM STxMP rather than 2 CWs. As stated in WID, Rel-18 STxMP aims to higher UPT, it can be reached via 2 CWs enabled to SDM STxMP. Regarding MediaTek said that “we think the SDM scheme is designed for T-put enhancement instead of reliability enhancement. The cell-average throughput should be main KPI to justify the benefit of 2CW”, it should also be the request of 1 CW SDM as the WID states that STxMP should reach the higher UPT when comparing baseline scheme (i.e. TxSP with panel selection). We proved that 1 CW SDM cannot outperform the legacy TxSP in many cases, nevertheless, 2 CWs SDM could guarantee the bottom line as required in WID. We sympathize with some companies’ concern about UE implementation complexity of 2 CWs SDM, and we sincerely hope that MTRP deployment challenges can be taken into consideration by companies. Therefor, we can compromise to support both 1 CW SDM and 2 CWs SDM, in which 2 CWs SDM can be UE optional. |  |
| CATT | Alt1 | Support Alt1 |
| MediaTek | Alt2 | Just to clarify. The max tx power is still limited in our simulations. It is not “unlimited”. |
| Vivo | Alt2 | Introducing 2 CW for two panels simultaneous transmission will increase UE complexity. 2 CW only serves higher throughput. It has no effect on reliability. While for SDM scheme with 1 CW, both throughput and reliability gain can be achieved. With panel-specific power control which can compensate for the channel variation, the performance gain of supporting two CWs with separate MCS versus one CW with one MCS is limited. For the deployment case in which channel quality of two panels varies a lot, 2 CW for two panels can be scheduled by two DCI separately in m-DCI based mTRP. |
| Samsung | Alt2 | @ZTE: Thank you for sharing your view. First, for backhaul overhead, we thought sDCI based scheme is based on ideal (or high capacity) backhaul. Anyway, if the backhaul has overhead issue in realistic, we can consider other combining option, e.g. LLR level combining after demodulation, instead of sharing received PUSCH without demodulation. As I know, spatial filtering and demodulation can be implemented via each TRP side (Yes, depending on implementation). This functionality for each TRP will be required for 2CW SDM case also (because each TRP will do spatial filtering and demodulation for each CW). If backhaul capacity is real problem, we can consider other realistic combining options (or mDCI based STx2P). |
| NTT Docomo | Alt.1. | First, we think even with per panel power control, there may still be case where power between two panels cannot be balanced e.g. one of the panels has reached maximum power. And we do not see why STxMP can only be used when power between two panels is very balanced. In contrast, with 2CW, STxMP can be beneficial when there is gap between power of two panels. Second, we share similar view with ZTE that 1CW has marginal or no gain compared to baseline (single panel) in some cases. And required specification effort is not that huge from our perspective. |
| Panasonic | Alt2 | We echo what Vivo mentioned. 2CW from two panels will increase UE complexity, so the merit of this scheme should take that into account. |
| ZTE3 |  | @Samsung: Thanks for sharing your views on MTRP deployment related issue. To my understanding, spatial filtering could be proceeded in each TRP due to 2 UL beams for SDM based PUSCH transmission, it is irrelevant to 1 CW or 2 CWs. Regarding demodulation processing in NW side, due to PUSCH modulated with one single MCS in 1 CW SDM scheme, combining before demodulation is always needed, which is same to the processing of STRP scenario. If we aim to support SDM STxMP, the issue (e.g. traffic backhaul overload caused in 1 CW SDM) address by switching to other scheme (e.g., MDCI STxMP or TxSP) cannot be the reason to preclude the UPT gain that could have been obtained by STxMP SDM with 2 CWs. |
| Ericsson | Alt2 | Our concerns with 2CW are: - Higher DCI overhead - Specification complexity: what fields would we duplicate? - Lack of alignment with other sDCI schemes |

### Summary of Round 1

From the round 1 inputs, we have the following supporting companies for each Alt:

- Alt1: Support 2 CWs for SDM scheme

  - Support (7): InterDigital, ZTE, CATT, Xiaomi, NTT DOCOMO, Lenovo, google

- Alt2: Do not support 2 CWs for SDM scheme

  - Support(14):  QC, Spreadtrum, vivo, Ericsson, Intel, MediaTek, Apple, Samsung, Panasonic, Nokia/NSB, LG, Fraunhofer IIS/HHI, Apple, Huawei/HiSilicon

According to the inputs in round 1, it looks unlikely to converge on this issue. The moderator would suggest do more study on this issue and come back to it in next meeting.

## Issue #6 PTRS indication for SDM scheme

### Summary

Companies discussed the issues of PTRS for SDM scheme of single-DCI based STxMP PUSCH transmission.

Regarding the number of PTRS ports for SDM scheme, companies provided the following view:

- Support 2 PTRS ports for SDM scheme of single-DCI based STxMP PUSCH

  - Supported: ZTE, Spreadtrum, vivo, Lenovo, OPPO, Intel, Qualcomm

Companies also proposed to enhance the PTRS-DMRS association for SDM scheme:

- ZTE proposed that to associate each PTRS port with one DMRS port of one panel/one SRS resource set: no indication is needed for {1+1}, Table 7.3.1.1.2-25 or 25A is used for {1+2} and {2+1}, Table 7.3.1.1.2-25 is used for {2+2}.

- Spreadtrum proposed to associate each PTRS port with DMRS port in different CDM groups.

- vivo suggested to enhance the PTRS-DMRS association so that PTRS port 0 and 1 are associated with one DMRS port in each CDM group, respectively

- Lenovo proposed to enhance the PTRS so that each PTRS port is used by one panel.

- OPPO suggested to associate PTRS port 0 and 1 with different panels.

- Intel proposed that one PTRS port is assigned for each panel

- Qualcomm proposed that 1st bit of PTRS-DMRS association field indicate association between PTRS port 0 with DMRS port of 1st SRS resource set and 2nd bit of PTRS-DMRS association field indicates association between PTRS port 1 with DMRS port of 2nd SRS resource set.

FL note: all the views support configuring 2 PTRS ports for SDM transmission. And all the views proposed to enhance the PTRS-DMRS association indication to associate PTRS port 0 and 1 with DMRS ports of two panels, even though the detailed methods might be different.

### Proposal for discussion

Based on the companies’ views, the following proposal is made:

FL Proposal 1-6: Support to configure up to 2 PTRS ports for SDM scheme of single-DCI based STxMP PUSCH transmission:

- For 2 PTRS ports, Ports 0 and 1 are associated with one DMRS port of two different panels respectively.

  - Study how to use the ‘PTRS-DMRS association’ field in DCI format 0_1 and 0_2 to indicate the PTRS-DMRS association for SDM scheme

Companies’ views:

| Company | Support the proposal? | Comments |
| --- | --- | --- |
| QC |  | PTRS ports is not mandatory for DL SDM. Hence, it should not be mandatory for UL. We suggest: FL Proposal 1-6: Support to configure up to 2 PTRS ports for SDM scheme of single-DCI based STxMP PUSCH transmission: - For 2 PTRS ports, Ports 0 and 1 are associated with one DMRS port of two different panels respectively. - Study how to use the ‘PTRS-DMRS association’ field in DCI format 0_1 and 0_2 to indicate the PTRS-DMRS association for SDM scheme |
| Nokia/NSB | Yes | Support FL’s proposal. |
| Lenovo | Yes | Support FL proposal. |
| Google | Yes with QC’s modification |  |
| NTT Docomo | Yes | Also fine with QC’s modification |
| ZTE | Yes with QC’s update |  |
| Mod2 |  | The proposal 1-6 is updated according QC’s comments. |
| LG | Yes | Support FL proposal. |
| Samsung | Yes | We are fine with updated proposal. |
| MediaTek | Yes |  |
| Spreadtrum | Yes | Support FL Proposal 1-6. |
| OPPO | Yes | Support the updated proposal. |
| Xiaomi | Yes with QC’s update |  |
| Apple | Yes | Support the updated FL’s proposal. |
| Intel | Yes | Support FL Proposal. |
| Huawei, HiSilicon | Yes | Support the updated FL’s proposal. |
| Vivo |  | a question for clarification, with this proposal does it mean that in the case of 4Tx on each panel and partial or non-coherent antennas two PTRS ports (0 and 1) cannot be configured for one panel? |
| Ericsson | No | The main bullet is as in Rel-17. The first subbullet is up to NW configuration. OK to study the DMRS to PTRS association. |
| Mod 4 |  | @vivo: for SDM, each of PTRS port would be associate with DMRS port of one panel, so that the first panel use one PTRS port and the other panel use the other PTRS port. @Ericsson: the main bullet is not 100% Rel-17 since Rel-17 does not have SDM. The main bullet says that SDM can use up to two ports. And those two PTRS ports are going to associate with DMRS ports of different panels. |

Issue#7 Switching between SDM and other schemes

### Summary of Round 1

The proposal 1-6 with updates according to the comments are supported by majority companies.

Regarding the comments of Ericsson, as explain in the above table, the main bullet is not as in Rel-17 since Rel-17 does not have SDM. The proposal intends to say that SDM can use up to 2 PTRS ports.

Based on the views collected in round 1, moderator would suggest to put Proposal 1-6 in email endorsement

## Issue#7 Switching between SDM and other schemes

### Summary

Companies provided views/proposals on the switching between SDM scheme and Rel15/16 sTRP transmission and switching between SDM scheme and Rel-17 TDM scheme.

For the switching between SDM scheme and Rel-15/16 sTRP transmission, the views in tdocs are:

- Support dynamic switching between SDM scheme and Rel-15/16 sTRP transmission

  - Supported: Huawei/HiSilicon, InterDigital, ZTE, Spreadtrum, vivo, Lenovo, LG, Intel, NEC, Xiaomi, Sharp, MediaTek, Fraunhofer IIS, Fraunhofer HHI, Samsung, Qualcomm, Panasonic, Nokia

Regarding how to support the dynamic switching between SDM scheme and Rel-15/16 sTRP transmission, the following proposals are provided in tdocs:

- Option 1: Use the ‘SRS resource set indicator’ DCI field

  - Support: Huawei/HiSilicon, Spreadtrum, vivo, Lenovo, LG, NEC, Xiaomi, MediaTek, Fraunhofer IIS, Fraunhofer HHI, Samsung, Qaulcomm, Nokia

- Option 2: use the DCI field for DMRS port indication to indicate the switching

  - Support: InterDigital

- Option 3: Use reserved/additional codepoint in the 1st or 2nd TPMI field to indicate the switching between SDM and Rel15/16 single-panel transmission.

  - Supported: ZTE

- Option 4: use the layer combination 0+n and n+0

  - Supported: Sharp

- Option 5: through the number of TCI states

  - Supported: Samsung

Regarding the Rel-15/16 sTRP transmission, companies also discussed the issues of maxRank when switching to Rel-15/16 sTRP transmission:

- vivo suggested when dynamically switching to Rel-15/16 sTRP transmission, maxRank can be 4.

- Qualcomm explained that there could exist two cases for the maxRank of sTRP iscusssion:

  - case 1: maxRank of sTRP is same to the maxRank of one SRS resource set configured for STxMP transmission,

  - case 2: The maxRank of sTRP transmission can be the sum of  maxRank of two SRS resource sets for STxMP PUSCH transmission.

FL note: All the companies support dynamic switch between SDM scheme and Rel-15/16 sTRP transmission. And majority companies support to use the ‘SRS resource set indicator’ to indicate the switching. Regarding the issue of maxRank when switching back to Rel-15/16 sTRP, FL understands that is a valid issue. The maxRank of sTRP transmission is not necessary to be same to the maxRank of one panel in STxMP transmission. So it is worthwhile to iscuss this issue.

For the switching between SDM scheme and Rel-17 TDM scheme, tdocs provided the following views:

- Alt1: Dynamic switching between SDM scheme and Rel-17 TDM scheme

  - vivo, LG, CATT, Fujitsu

- Alt2: RRC-based switching between SDM scheme and Rel-17 TDM scheme

  - Supported: Huawei/HiSilicon, ZTE, Spreadtrum

Regarding dynamic switching between SDM and Rel-17 TDM scheme, vivo and LG proposed to use the indicated repetition number when two TCI states are applied is used to switch between SDM and Rel-17 TDM scheme.

The views on how to switch between STxMP SDM and Rel-17 TDM diverge.

### Proposal for discussion

Based on the companies’ views, the following proposal is made:

FL Proposal 1-7: For single-DCI based STxMP PUSCH:

- Support dynamic switching between SDM scheme and Rel-15/16 sTRP transmission

  - Use the DCI field ‘SRS resource set indicator’ to indicate the switching

  - FFS the maxRank when switching to Rel-15/16 sTRP transmission,

    - e.g., the maxRank of Rel-15/16 transmission can be the maxRank of one SRS resource set for STxMP, the maxRank of Rel-15/16 transmision can be the sum of maxRanks of SRS resource sets for STxMP; the maxRank of Rel-15/16 transmission can be separately configured.

- Down-select one from the followings:

  - Alt1: Support dynamic switching between SDM scheme and Rel-17 PUSCH TDM scheme

    - FFS: how to support dynamic switching, e.g., using the indicated PUSCH repetition number

      - Support: vivo, LG, CATT, Fujitsu, Nokia, InterDigital

  - Alt2: Support RRC-based switching between SDM scheme and Rel-17 PUSCH TDM scheme

      - Support: Huawei/HiSilicon, ZTE, Spreadtrum, QC, OPPO, Fraunhofer IIS/HHI, Xiaomi, Apple

Companies’ views:

| Company Name | Support Proposal 1-7 (Yes/No) | Comments |
| --- | --- | --- |
| QC | Support | Between Alt1 and Alt2, we prefer Alt2 to simplify the design. The use case for dynamic switching between TDM and SDM is not very clear. |
| Nokia/NSB | Yes | First main bullet: support Second main bullet: Support Alt-1 |
| InterDigital |  | For first main bullet: SRS resource set indicator field switches between sTRP and mTRP. STxMP can be used in sTRP, or mTRP, so switching between single panel and STxMP should be separately indicated from the existing SRS resource set indicator fields. For second main bullet: Support Alt1. |
| Lenovo |  | Support FL proposal. |
| Google | No | This is based on unified TCI framework, the dynamic switching can be based on a new DCI field for TCI selection instead of SRS resource set indicator. |
| NTT Docomo | Yes |  |
| ZTE | Yes with update | It is not proper to rush into the details of indication way for dynamic switching in this stage. SRS set indicator introduce in Rel-17 MTRP PUSCH TDM repetition is used to indicate four modes: STRP mode with TRP1, STRP mode with TRP2, MTRP mode with {TRP 1, TRP2} order, and MTRP mode with {TRP 2, TRP1} order, where the indicated statues of MTRP order do not make any sense to STxMP mode. On the other hand, unnecessary DCI overhead increased by newly added field should be avoided. Note that two separated but complete TPMI/SRI fields are used for STxMP PUSCH, reversed codepoint or additional codepoint (with 1 bit increasing) can be used to indicate single-panel/TRP mode. Consequently, SRS set indicator field with 2 bits is not needed and its overhead can be saved. That also means it is unreasonable to reuse SRS resource set indicator to indicate dynamic switching between Rel-18 STxMP SDM mode and Rel-15/16 STRP mode. Regarding the part of maxRank, it is related to the ongoing discussion of issue#3, hence the details should be discussed later. FL Proposal 1-7: For single-DCI based STxMP PUSCH: - Support dynamic switching between SDM scheme and Rel-15/16 sTRP transmission - FFS the indication of dynamic switching. - FFS the maxRank when switching to Rel-15/16 sTRP transmission, - Down-select one from the followings: - Alt1: Support dynamic switching between SDM scheme and Rel-17 PUSCH TDM scheme - FFS: how to support dynamic switching, e.g., using the indicated PUSCH repetition number - Alt2: Support RRC-based switching between SDM scheme and Rel-17 PUSCH TDM scheme |
| NEC | Yes | Also fine to discuss details like maxRank later. |
| Mod |  | Made the following updates to proposal 1-7: - The supporting companies is added to each of Alt1 and Alt2: the positions presented in the Tdoc is added here and the supporting shown in inputted comments are also added. @ZTE: using “SRS resource set indicator” is supported by majority companies, from the perspective of moderator, it looks reasonable to go with this method. @google: whether to introduce new DCI field is still a pending issue in 9.1.1.1 from my understanding. |
| Fujitsu | Yes | Alt1 is preferred. |
| LG | Yes | Support Alt 1. In Rel16 PDSCH, dynamic switching between TDM and SDM is already supported for purpose of fast switching when both URLLC TB and eMBB TB are in DL traffic buffer. We can achieve the benefit in UL as well with Alt 1. |
| Samsung | Can support but later | We think this issue should be discussed after determining whether to support SFN scheme. We can consider switching between SDM and SFN as well as Rel-15/16 sTRP transmission. |
| MediaTek | Yes | Regarding maxRank configuration, we show our view in Issue#3. One maxRank configuration only is needed for all the PUSCH transmission regardless of PUSCH(s) to single TRP and PUSCH(s) to multiple TRP. |
| Spreadtrum | Yes | Support Alt2, since STxMP and TDM-based transmission schemes target different use cases, i.e. SDM scheme for throughput enhancement and TDM-based scheme for reliability. |
| OPPO | Yes | For second main bullet: support Alt 2. |
| Fraunhofer IIS/HHI | Yes | Support Alt2 |
| Xiaomi | Yes | Support Alt.2 for the simplicity of signaling design |
| Apple | Yes | Support Alt2 |
| Mod 3 |  | Updated the list of supporting companies according to latest comments. |
| Intel | Yes | Prefer Alt2. |
| Huawei, HiSilicon | Yes with update | First bullet: Support but we prefer to remove the example. The FFS for maxRank is sufficient. Second bullet: Support Alt 2. For uplink mTRP, one prerequisite for DCI-based dynamic switching between PUSCH repetition and STxMP PUSCH SDM is that the SRS resources for uplink channel acquisition can be shared by these two transmission modes so that the channel information for both schemes are available at the network side so the gNB can dynamically select either of them by DCI. However, the Rel-17 TDM based PUSCH repetition supports up to 4-layer transmission, which means the largest number of SRS antenna ports configured in the SRS resource set for codebook-based transmission can be 4. On the other hand, in STxMP PUSCH SDM scheme, since maximum of {2+2} layers are supported, UE is likely to be configured with 2-port SRS resources in the SRS resource set associated to each UE panel. In this case, the SRS configurations for the two transmission modes are not the same and, therefore, two different sets of SRS resource configuration (a total of 4 SRS resources sets) needs to be configured to UE to perform dynamic switching between PUSCH TDM repetition and STxMP PUSCH SDM. Considering the potential impact on both UE and NW complexity, we propose not to support this feature in Rel-18. Note that the size of TMPI/SRI fields in these two schemes would be different. To resolve the ambiguity in the DCI size, further specification effort is required which further justifies the more straightforward scheme in Alt2. We suggest: FL Proposal 1-7 (updates): For single-DCI based STxMP PUSCH: - Support dynamic switching between SDM scheme and Rel-15/16 sTRP transmission - Use the DCI field ‘SRS resource set indicator’ to indicate the switching - FFS the maxRank when switching to Rel-15/16 sTRP transmission, - e.g., the maxRank of Rel-15/16 transmission can be the maxRank of one SRS resource set for STxMP, the maxRank of Rel-15/16 transmision can be the sum of maxRanks of SRS resource sets for STxMP; the maxRank of Rel-15/16 transmission can be separately configured. - Down-select one from the followings: - Alt1: Support dynamic switching between SDM scheme and Rel-17 PUSCH TDM scheme |
| ZTE2 |  | @FL, SRS resource set indicator introduced in Rel-17 is used to indicate the association between TPMI/SRI field and SRS resource set. In issue#3, we haven’t reach any details of TPMI/SRI field and SRS resource set configuration, it makes no sense to conclude that SRS resource set indicator for Rel-17 MTRP PUSCH TDM repetition can be reused in Rel-18 STxMP SDM scheme. The indication of dynamic switching between STxMP and TxSP should wait the outcome of issue#3 anyways. Likewise, maxRank of STxMP and TxSP is the same case. FL Proposal 1-7: For single-DCI based STxMP PUSCH: - Support dynamic switching between SDM scheme and Rel-15/16 sTRP transmission - FFS the indication of dynamic switching. - FFS the maxRank when switching to Rel-15/16 sTRP transmission, - Down-select one from the followings: - Alt1: Support dynamic switching between SDM scheme and Rel-17 PUSCH TDM scheme - FFS: how to support dynamic switching, e.g., using the indicated PUSCH repetition number - Support: vivo, LG, CATT, Fujitsu, Nokia, InterDigital - Alt2: Support RRC-based switching between SDM scheme and Rel-17 PUSCH TDM scheme - Support: Huawei/HiSilicon, ZTE, Spreadtrum, QC, OPPO, Fraunhofer IIS/HHI, Xiaomi, Apple |
| MediaTek | Yes | Support Alt1 |
| CATT | Yes | Support using SRS resource set indicator. |
| Vivo | Yes | We are generally ok with the proposal, UE capability on maxrank for SDM and sTRP needs further discussion Regarding the second bullet, we are OK with both Alt1 and Alt2. |
| Panasonic | Yes | We are okay with alt1 (if no overhead, like for example if we agree to different CDM groups) and alt2. |
| Ericsson | No? | This is unclear: the first bullet refers to R15/16, and the second to R17. Is it the same? Or different? |

### Summary of round 1

The proposal 1-7 seem to be ok with majority company and some wording changes were suggested. The proposal will be updated with the wording change for round 2 discussion.

Re the latest comments by Ericsson: The proposal 1-7 is about switch between SDM vs Rel-15/16 sTRP and switch between SDM vs Rel-17 TDM. Wording will be refined to make it clearer.

Re the comment of Samsung: this issue is about the switch between SDM vs existing schemes. If SFN is agreed, we will need to discuss how to switch between SFN vs all other schemes. It seems there is no reason to not make decision for SDM now.

### Proposal for Round 2 Discussion

Here is the updated proposal 1-7:

Updated Proposal 1-7: For single-DCI based STxMP PUSCH:

- Support dynamic switching between SDM scheme and sTRP transmission

  - [FFS the indication of dynamic switching] vs [Use the DCI field ‘SRS resource set indicator’ to indicate the switching]

  - FFS the maxRank when switching to sTRP transmission,

    - e.g., the maxRank of Rel-15/16 transmission can be the maxRank of one SRS resource set for STxMP, the maxRank of Rel-15/16 transmision can be the sum of maxRanks of SRS resource sets for STxMP; the maxRank of Rel-15/16 transmission can be separately configured.

- For the switch between SDM scheme and Rel-17 PUSCH TDM scheme, down-select one from the followings:

  - Alt1: Support dynamic switching between SDM scheme and Rel-17 PUSCH TDM scheme

    - FFS: how to support dynamic switching, e.g., using the indicated PUSCH repetition number

  - Alt2: Support RRC-based switching between SDM scheme and Rel-17 PUSCH TDM scheme

Comments on Updated Proposal 1-7:

| Company | Support (Yes/No) | Comments |
| --- | --- | --- |
| QC |  | It seems super majority of companies are ok with using “SRS resource set” field to indicate the switching. If the concern is issue #3 (whether 2 SRS resource sets are supported), then we can decide on this proposal after Proposal 1-3. |
| LG |  | Same view with QC. |
| NTT Docomo | Yes | We are fine with this proposal. Meanwhile, we share similar view as QC that it would be good if we can agree on using “SRS resource set indicator” |
| Fujitsu | Yes | We are also fine if agreeing on using SRS resource set indicator. |
| MediaTek |  | It is very clear that using “SRS resource set indicator” is majority. However, some companies have concern on agreeing to use SRS resource set indicator before Proposal 1-3 is decided. We are fine to decide this proposal after Proposal 1-3 is finalized. |
| CATT |  | Same as the view in round1 discussion. We support to use SRS resource set indicator. |
| ZTE | Yes |  |
| OPPO |  | The proposal is fine for us. But it would be good if ‘SRS resource set indicator’ can be used for the indication of dynamic switching. |
| Xiaomi |  | Same view as QC. |
| InterDigital | Yes |  |
| Intel | Yes |  |
| Huawei, HiSilicon |  | Similar to other companies, we strongly prefer to use the DCI field ‘SRS resource set indicator’ to indicate the switching. If the consensus is not achievable on this, we are OK with the current form. |
| Spreadtrum | Yes | Support this updated proposal and prefer to reuse SRS resource set indicator field. |
| Nokia | Yes | Agree with QC comment. |
| Samsung |  | We think ‘Rel-15/16’ is unclear (field in DCI should be same as Rel-15/16? or other ambiguity). We suggest to modify the proposal as follow: Updated Proposal 1-7: For single-DCI based STxMP PUSCH: - Support dynamic switching between SDM scheme and Rel-15/16 sTRP transmission - FFS the indication of dynamic switching, Use the DCI field ‘SRS resource set indicator’ to indicate the switching - FFS the maxRank when switching to Rel-15/16 sTRP transmission, - e.g., the maxRank of Rel-15/16 transmission can be the maxRank of one SRS resource set for STxMP, the maxRank of Rel-15/16 transmision can be the sum of maxRanks of SRS resource sets for STxMP; the maxRank of Rel-15/16 transmission can be separately configured. - For the switch between SDM scheme and Rel-17 PUSCH TDM scheme, down-select one from the followings: - Alt1: Support dynamic switching between SDM scheme and Rel-17 PUSCH TDM scheme - FFS: how to support dynamic switching, e.g., using the indicated PUSCH repetition number - Alt2: Support RRC-based switching between SDM scheme and Rel-17 PUSCH TDM scheme We are also fine with reusing SRS resource set indicator for the switching between SDM and sTRP transmission. |
| Lenovo |  | We support to use SRS resource set indicator for the switching between SDM and sTRP scheme. |
| Mod |  | @Samsung, your suggestion is implemented in the proposal, just a simple wording change. @Qualcomm: regarding how to indicate dynamic switch, ZTE seems to have other design in mind instead of the majority view of using “SRS resource set indicator”. That is why it is FFS now. @ZTE: could you be flexible to be ok with “SRS resource set indicator” considering that all the other companies view of using ‘SRS resource set indicator’ and proposal 1-3 is stable in the email? |

# Multi-DCI based STxMP PUSCH

## (Closed) Issue #1: Support multi-DCI based STxMP PUSCH+PUSCH

### Summary

It was agreed to study the multi-DCI based STxMP PUSCH+PUSCH, where two PUSCHs associated with different TRPs can be transmitted simultaneously from different UE panels and the total number of these two overlapping PUSCHs is up to 4.

[109e] Agreement For multi-DCI based STxMP PUSCH+PUSCH transmission, study and evaluate the following aspects: - Two PUSCHs are associated with different TRPs and transmitted from different UE panels. The total number of layers of these two PUSCHs is up to 4. - Study STxMP of PUSCH+PUSCH transmission where it is some combination of DG-PUSCH, CG-PUSCH and msg3/msgA PUSCH. - The overlapping type(s) of fully/partially in time domain and fully/partially/non-overlapping in frequency domain are to be studied and justified for PUSCH+PUSCH. Note: The above study shall take into account the UE implementation and RF considerations. Note: Study the conditions required for STxMP PUSCH+PUSCH. Note: Other aspects are not precluded.

In the contributions, many companies support or are ok with multi-DCI based STxMP PUSCH+PUSCH:

- Huawei/HiSilicon, ZTE, Spreadtrum, vivo, Ericsson, Lenovo, OPPO, Google, LG, CATT, Intel, Xiaomi, CMCC, MediaTek, Fraunhofer IIS, Fraunhofer HHI, NTT DOCOMO, Qualcomm, Panasonic, Nokia.

And Nokia provided SLS simulation results:

Nokia - SLS simulation, Spectral efficiency and Resource utilization - Single panel UE vs UE with panel selection vs UE with STxMP Figure 6: System Level Simulation results comparing STxMP with single-panel UEs and two-panel Ues with panel selection: Mean UE SE (left) and Edge UE SE (right) versus traffic load for FTP model 1. Figure 7 : System Level Simulation results comparing STxMP with single-panel Ues and two-panel Ues with panel selection: Resource Utilization versus traffic load for FTP model 1 Observation 23: For the dense urban scenario, STxMP provides gains in mean UE throughput over panel selection only at low traffic loads. For edge UE throughput, STxMP provides a loss at all evaluated loads. Observation 24: At low traffic loads STxMP can provide benefits by enabling the use of system resources that would otherwise have been idle. However, at high traffic loads, the scheduling constraints imposed by STxMP on two TRPs often result in the sub-optimal allocation of the resources controlled by the two TRPs.

### Proposal for discussion

According to the views in contribution, the following companies showed supporting or are ok STxMP PUSCH+PUSCH in multi-DCI based system:

- Huawei/HiSilicon, ZTE, Spreadtrum, vivo, Ericsson, Lenovo, OPPO, Google, LG, CATT, Intel, Xiaomi, CMCC, MediaTek, Fraunhofer IIS, Fraunhofer HHI, NTT DOCOMO, Qualcomm, Panasonic, Nokia.

Therefore, the following proposal is made and it proposes to support the multi-DCI based STxMP PUSCH+PUSCH transmission.

FL Proposal 2-1 Support STxMP PUSCH+PUSCH transmission in multi-DCI based system in Rel-18.

- Two independent PUSCHs associated with different TRPs can be transmitted by a UE simultaneously in same active BWP.

- The total number of layers of these two PUSCHs is up to 4.

- FFS: whether the number of layers of each of these two PUSCHs is up to 2.

Companies’ views:

| Company | Support (Yes/No) | Comments |
| --- | --- | --- |
| QC | Yes | Similar to our earlier comment, “TRP” and “panel” are not defined. For multi-DCI, we should use coresetPoolIndex, which is already defined. However, it is ok to keep the current text if it is common understanding (that we are not going to define a new framework for multi-DCI) |
| Nokia/NSB | Yes | Support FL’s proposal. |
| InterDigital | Yes | Support FL’s proposal. |
| Lenovo | Yes | Support FL proposal. |
| Google | Yes with modification | For mDCI, we think we should restrict the number of layers per PUSCH to be 2 instead of the total number of layers. - Two independent PUSCHs associated with different TRPs can be transmitted from different UE panels simultaneously in same active BWP. - The number of layers of per PUSCH is up to 2. |
| NTT Docomo | Yes |  |
| ZTE | Yes | BTW, we don’t see the reason why per panel transmission layers should be specified. MDCI based STxMP scheme is irrelated with SDCI based STxMP SDM scheme. The original wording of second bullet is clear enough to be inline with WID. |
| NEC | Yes |  |
| Mod |  | @QC: some companies might have concern to use “coresetPoolIndex” directly for the current moment. That is why the general term “panel” is used here. @google: per the WID, the total number of layers of STxMP PUSCHs is up to 4. However, whether each number of layers in each PUSCH shall be limited to 2 is not decided yet. I think for your concern, we can add FFS on this. Thus, the proposal is updated per Google’s comment. |
| Fujitsu | Yes |  |
| LG | Yes |  |
| Samsung |  | Our preference is focusing sDCI STx2P first. However, if majority supports this, we are okay with this. |
| MediaTek | Yes with update | Agree with QC’s comment. We suggest have the following updated proposal: FL Proposal 2-1 Support STxMP PUSCH+PUSCH transmission in multi-DCI based system in Rel-18. - Two independent PUSCHs can be simultaneously transmitted by a UE in same active BWP. - The total number of layers of these two PUSCHs is up to 4. - FFS: whether the number of layers of each of these two PUSCHs is up to 2. |
| Spreadtrum | Yes | Support FL’s proposal. |
| OPPO | Yes | Support FL’s proposal. |
| Fraunhofer IIS/HHI | Yes |  |
| Xiaomi | Yes | Support FL’s proposal |
| Mod 3 |  | Update the Proposal according the comment by MediaTek and QC: @QC and MediaTek: the wording of “UE panel” was deleted @MediaTek: you suggested to delete “associated with different TRP”. About that, my understanding is we need this clarification because we cannot ask the UE to transmit two PUSCHs associated with the same CORESETPoolIndex value at that same time, right? |
| Intel | Yes | For the FFS, we think it may not be necessary to limit to 2 layers per panel. |
| Huawei, HiSilicon |  | Support FL’s proposal. |
| MediaTek | Yes | Regarding “associated with different TRP”, we are fine with it now. However, since “TRP” is not defined in spec, how to differentiate that two PUSCHs are associated with the same TRP or different TRPs need to be further discussed. - FFS: how to determine that two PUSCHs are associated with different TRPs? |
| CATT | Yes |  |
| vivo | Yes | We suggest to use SRS resource sets instead of UE panels which will not be reflected in spec. The association between SRS resource sets and TRPs is discussed in proposal 2-4. FL Proposal 2-1 Support STxMP PUSCH+PUSCH transmission in multi-DCI based system in Rel-18. - Two independent PUSCHs associated with different TRPs can be transmitted from different SRS resource setsUE panels simultaneously in same active BWP. - The total number of layers of these two PUSCHs is up to 4. - FFS: whether the number of layers of each of these two PUSCHs is up to 2. |
| Panasonic | Yes | Support |
| Ericsson | Yes | Why should the specification limit this to totally 4 layers? Also, it would seem strange to limit this number of layers per PUSCH to 2. |
| Mod 4 |  | @Ericsson, limiting to 4 layers seems to be from the WID. |

## Issue #2: Types of PUSCH supported in STxMP PUSCH

### Summary

It was agreed to study the combination of PUSCH types that can be supported in multi-DCI based STxMP PUSCH+PUSCH.

[109e] Agreement For multi-DCI based STxMP PUSCH+PUSCH transmission, study and evaluate the following aspects: - Two PUSCHs are associated with different TRPs and transmitted from different UE panels. The total number of layers of these two PUSCHs is up to 4. - Study STxMP of PUSCH+PUSCH transmission where it is some combination of DG-PUSCH, CG-PUSCH and msg3/msgA PUSCH. - The overlapping type(s) of fully/partially in time domain and fully/partially/non-overlapping in frequency domain are to be studied and justified for PUSCH+PUSCH.

In the contributions, the following views are provided for this issue:

- Spreadtrum: Support the combination of DG-PUSCH+DG-PUSCH, DG-PUSCH+CG-PUSCH and CG-PUSCH+CG-PUSCH

- vivo: Further study the conditions required for simultaneous transmission for the overlapped DG-PUSCHs/CG-PUSCHs/DG-PUSCH+CG-PUSCH

- Ericsson: Do not allow simultaneouos transmission of msg3/msgA and PUSCH using different panels and Further consider allowing STxMP of CG-PUSCH+DG-PUSCH transmission

- CATT: The following combinations of PUSCHs are supported for STxMP transmission: DG PUSCH and DG PUSCH, DG PUSCH and type 1/2 CG PUSCH and ype 1/2 CG PUSCH and type 1/2 CG PUSCH

- Intel: For multi-DCI based STxMP PUSCH + PUSCH transmission, DG + DG PUSCH, CG + CG PUSCH, and DG + CG PUSCH can be supported

- Xiaomi: Support the enhancement of DG-PUSCH and CG-PUSCH combinations for multi-DCI based the simultaneous transmission

- MediaTek: Support DG-PUSCH+DG-PUSCH, DG-PUSCH+CG-PUSCH, and CG-PUSCH+CG-PUSCH, do not support Msg3/MsgA.

- NTT DOCOMO: Support simultaneous transmission of overlapped DG PUSCH+DG PUSCH, CG PUSCH+PUSCH and DG PUSCH+CG PUSCH

- Qualcomm: support DG-PUSCH+DG-PUSCH, CG-PUSCH+CG-PUSCH, and DG-PUSCH+CG-PUSCH

- Nokia: Support both simultaneous DG-PUSCH transmission and CG-PUSCH transmission in Rel-18

FL: all the provided views support both DG-PUSCH and CG-PUSCH and no companies supports the msg3/msgA PUSCH in multi-DCI based STxMP PUSCH+PUSCH.

### Proposal for discussion

Based on the views in contribution, the following proposal is made:

FL Proposal 2-2 Multi-DCI based STxMP PUSCH+PUSCH transmission supports the following PUSCH combinations:

- DG-PUSCH + DG-PUSCH

- CG-PUSCH + CG-PUSCH

- CG-PUSCH + DG-PUSCH

Note: Here the CG-PUSCH could be a type 1 CG-PUSCH or type 2 CG-PUSCH.

Companies’ views:

| Company | Support (Yes/No) | Comments |
| --- | --- | --- |
| QC | Support |  |
| Nokia/NSB | Yes | Support FL’s proposal. |
| InterDigital | Yes | Support FL’s proposal. |
| Lenovo | Yes | Support FL proposal. |
| Google | Yes |  |
| NTT Docomo | Yes |  |
| ZTE |  | We can be fine if majority prefer. |
| MediaTek | Yes |  |
| Spreadtrum | Yes | Support FL’s proposal. |
| OPPO | Yes | Support FL’s proposal. |
| Xiaomi | Yes | Support FL’s proposal. |
| Intel | Yes | Support the FL Proposal. |
| Huawei, Hisilicon | Yes | Support FL’s proposal. |
| CATT | Yes | Support FL’s proposal |
| vivo | Yes |  |
| Samsung | Yes | Support FL’s proposal. |
| Ericsson | No | The scheduling decision must be based on a recent report, which states if STxMP is possible or not. A dynamic scheduling decision can then be made. If CG is included, we will need to handle prioritization and dropping. |

### Summary of round 1

QC, Nokia/NSB, InteDigital, Lenovo, Google, NTT Docomo, ZTE, MediaTek, Spreadtrum, OPPO, Xiaomi, Intel, Huawei, Hisilicon, CATT, vivo, Samsung support the proposal.

But Ericsson showed the concern on CG PUSCH. They explained that if CG included, we might need handle prioritization and dropping. For that, from the understanding of moderator, it is more like a system implementation issue and furthermore, HARQ retransmission can be used to pick up the failed PUSCH transmission, as in all the usual PUSCH transmission.

Given the inputs in round 1, Moderator would suggest to put Proposal 2-2 in email endorsement

## Issue #3: Overlapping Types in time domain and frequency domain

### Summary

It was agreed to study the overlapping type(s) in time domain and frequency domain for multi-DCI based STxMP PUSCH+PUSCH transmission:

[109e] Agreement For multi-DCI based STxMP PUSCH+PUSCH transmission, study and evaluate the following aspects: - Two PUSCHs are associated with different TRPs and transmitted from different UE panels. The total number of layers of these two PUSCHs is up to 4. - Study STxMP of PUSCH+PUSCH transmission where it is some combination of DG-PUSCH, CG-PUSCH and msg3/msgA PUSCH. - The overlapping type(s) of fully/partially in time domain and fully/partially/non-overlapping in frequency domain are to be studied and justified for PUSCH+PUSCH. Note: The above study shall take into account the UE implementation and RF considerations. Note: Study the conditions required for STxMP PUSCH+PUSCH. Note: Other aspects are not precluded.

In the contributions, the following views are provided for this issue:

- Huawei/HiSilicon: Two PUSCHs can be scheduled respectively by the two TRPs with fully/partially/non-overlapping resources in time/frequency domains in the same carrier.

- ZTE: fully or partially overlapping in time/frequency domain can be allowable

- Spreadtrum: Support the overlapping types of fully/partially overlapping in time domain and fully/partially/non-overlapping in frequency domain

- Vivo: for STxMP transmission, three scheduled PUSCH frequency-domain patterns can be considered with non-overlapped, partially-overlapped, and fully overlapped in frequency domain for two PUSCHs overlapping in the time domain

- Lenovo: scheduling two fully/partial/non-overlapped PUSCH transmissions

- OPPO: For multi-DCI based PUSCH transmissions, fully overlapping in time domain and fully/partial/non-overlapping in frequency domain are supported with high priority

- Google: For mDCI based STxMP on PUSCH, support to multiplex the PUSCHs triggered by two DCIs with partially and fully overlapped resources

- LG: In order to avoid symbol level PUSCH power control, consider scheduling restriction for time domain fully overlapped PUSCHs

- CATT: Fully/partially overlapping in time domain, Fully/partially/non-overlapping in frequency domain

- MediaTek: The two PUSCHs can be fully/partially overlapped in time domain and fully/partially/non overlapped in frequency domain

- Fraunhofer IIS, Fraunhofer HHI : Allow for fully/partially/non-overlapping transmissions in time and frequency domain for simultaneous PUSCH transmission from multiple panels

- Qualcomm: Support all overlapping types of fully/partially in time domain and fully/partially/non-overlapping in frequency domain (similar to PDSCH overlap in Rel-16 and subject to UE capability)

- Nokia: For multi-DCI, support all possible overlapping options, (i.e. fully/partial in time and fully/partial/non-overlapping in frequency), among separately scheduled simultaneous multi-panel transmissions

FL Note: it can be observed most of companies support fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain. However, Huawei/HiSilicon explained that partially overlapped PUSCH might cause symbol-level power adjustment, which may cause additional UE complexity. OPPO also mentioned that partially overlapping in time domain might increase the UE complexity due to power control. LG suggested to consider fully overlapping in time domain to avoid symbol level PUSCH power control. Per the understanding of FL, if the max Tx power is per UE (i.e., Max Tx power option 1), the symbol-level power adjustment might happen since the power is shared among two PUSCHs. But if the max Tx power is per panel (i.e, max Tx power option 2), the symbol-level power adjustment might not happen since the Tx power is applied to each PUSCH separately. To progress, FL suggest to go with all the overlapping types per the views of majority companies and FFS on how to handle the potential symbol-level adjustment issue.

### Proposal for discussion

Based on the views, the following proposal is made:

FL proposal 2-3: The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain.

- FFS how to handle the potential PUSCH power adjustment that could happen when two PUSCHs are partially overlapped in time domain.

- Note: No symbol-level power adjustment in the case of partially overlapping in time domain

Companies’ views:

| Company | Support (Yes/No) | Comments |
| --- | --- | --- |
| QC |  | FFS is not needed. LS is already sent to RAN4 regarding power sharing (it is unlikely that RAN4 replies that power sharing is needed with respect to PCMax). Hence, this can be discussed after we receive the reply from RAN4. |
| Nokia/NSB | Yes | Support FL’s proposal. |
| InterDigital | Yes | Support FL’s proposal. |
| Lenovo | Yes | Support FL proposal. |
| Google | Yes | OK to remove FFS |
| NTT Docomo | Yes |  |
| ZTE | Yes |  |
| LG | Yes with update | Regarding FFS, it does not have to be symbol-level, which make UE implementation complicated. We prefer make it more general as follows: FL proposal 2-3: The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain. - FFS how to handle the potential PUSCH power adjustment that could happen when two PUSCHs are partially overlapped in time domain. |
| Samsung | No (especially, partial in time domain) | We also have a concern on symbol-level power adjustment in partially overlapping case in time domain. Without clear operation for this case, we cannot support partially overlapping case in time domain. |
| MediaTek | No | We prefer to not have symbol-level power control that cause large complexity. We support partially overlapping in time-domain without introducing symbol-level power control. Agreed with QC, this issue highly depends on the reply from RAN4, such that we suggest to postpone this issue until getting RAN4’s feedback |
| Spreadtrum | Yes | Support deleting FFS. |
| OPPO | Yes | Support FL’s proposal. |
| Fraunhofer IIS/HHI | Yes with update | Ok with LG’s update |
| Xiaomi | Yes | Fine with LG’s revision |
| Mod 3 |  | Update the proposal as follows according to the latest comments: - Delete the “symbol-level” in the first sub-bullet per the comments by LG Samsung and MediaTek have concern on complexity caused by symbol-level power adjustment for partially overlapping in time domain. To resolve their concern, a note is added to clarify that symbol-level power adjustment is not included. |
| Intel | Yes | Support the FL proposal. |
| Huawei, HiSilicon |  | Support FL proposal |
| MediaTek | Yes | Ok to current FL proposal |
| CATT | Yes | Support FL’s proposal. |
| Vivo | Yes | We share similar views with QC. The FFS should be removed. Whether power sharing is required should be determined by the response LS. If power sharing should be defined, per PUSCH occasion power adjustment is better to avoid the phase consistency of channel estimation. FL proposal 2-3: The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain. - FFS how to handle the potential symbol-level PUSCH power adjustment that could happen when two PUSCHs are partially overlapped in time domain. |
| Samsung |  | @FL: Thank you for the consideration. We have one question for added note. Does it mean partially overlapping case is only supported for power option 2 (max power per panel)? |
| Ericsson | Yes | We could also be OK to wait for RAN4 feedback, and to agree on all the other overlap types. |

### Summary of Round 1

Samsung and MediaTek showed concern on potential symbol-level power adjustment in the case of partially overlapping in time domain. To relax their concern, sub-bullets were added.

### Proposal for Round 2 Discussion

So here is the latest proposal:

FL proposal 2-3: The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain.

- FFS how to handle the PUSCH power adjustment when two PUSCHs are partially overlapped in time domain.

- Note: No symbol-level power adjustment in the case of partially overlapping in time domain

@Samsung: the added notes do not mean partially overlapping case only support power option 2. I think it means no matter power option 1 or option 2, the power control shall make sure there is no symbol-level power adjustment for the case of partially overlapping in time domain.

@QC and vivo: let us keep the FFS to address the concern of companies on the case of partially overlapping in time domain.

Comments on Updated Proposal 2-3:

| Company | Support (Yes/No) | Comments |
| --- | --- | --- |
| Apple |  | It’s not clear to me what added note targets. Does it mean that for the case of partial overlapping in time, symbol level power adjustment is not expected? We still prefer to postpone this discussion after RAN4’s reply. |
| QC | Yes with revision | FL proposal 2-3: The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain. - FFS whether/how to handle the PUSCH power adjustment when two PUSCHs are partially overlapped in time domain (if RAN4 identifies the need for power sharing from PCMAX point of view) - Note: No symbol-level power adjustment in the case of partially overlapping in time domain |
| NTT Docomo | Yes | Also fine with QC’s revision |
| MediaTek | Yes | Okay for QC’s revision |
| CATT | Yes | Fine with current proposal. |
| ZTE | Yes | QC’s revision is also fine. |
| OPPO | Yes | Fine with QC’s revision |
| Xiaomi | Yes | Fine with QC’s revision |
| Intel | Yes | OK with QC’s revision. |
| Huawei, HiSilicon | Yes | OK with FL version. We cannot agree with QC’s version. The issue of power adjustment in the case of partial overlap in time is not necessarily related to power sharing. As long as there is a maximum total UE power for the UE class, the power adjustment in the case of partial overlap needs to be discussed. |
| QC |  | @Huawei: The issue is indeed about power sharing or PCMAX. Maximum total UE power for the UE class is RAN4 business, and only the configured max power (PCMAX) is of RAN1’s concern wrt power control. If this part is controversial, both FFS and Note can be deleted. W/o the red addition, we cannot accept the two sub-bullets. |
| Spreadtrum |  | Fine with the updated proposal from QC. |
| Nokia | Yes | Ok with QC addition. |
| Samsung | Yes | @FL: Thank you for response. We understood. We are fine with either FL’s or QC’s proposal. |
| Lenovo | Yes | Fine with QC’s revision |
| Mod |  | @Apple: regarding the note: yes, when PUSCH is overlapping partially in time domain, the UE still apply the same power on all symbols of each PUSCH. For example, 1st PUSCH is on symbols 1~6 and 2nd PUSCH is on symbols 5~8. They overlap on symbols 5 and 6. With the note, for 1st PUSCH, the UE apply the same power level on symbols 1~6 and for 2nd PUSCH, the UE apply the same power level on symbols 5~8. This note is to address the concerns of companies on high UE complexity caused by symbol-by-symbol power adjustment. @QC and HW: let us see if we can find a middle ground for the FFS bullet. We keep ‘whether’ here since any study can include “whether to ..” and delete the condition of (if RAN4…). This detailed condition would introduce more discussion, which I think shall be part of next level discussion. So how about this: - FFS whether/how to handle the PUSCH power adjustment when two PUSCHs are partially overlapped in time domain (if RAN4 identifies the need for power sharing from PCMAX point of view) |
| QC |  | We cannot accept removing the condition of (if RAN4…). LS has been already sent to RAN4. The FFS is relevant only if RAN4 identified the need for power sharing wrt PCMAX. Otherwise, there is no need for FFS or the note. The addition in red is technically accurate, procedurally the right thing to do, and is supported by super majority of companies. |

## Issue #4: SRS resource configuration and TMPI/SRI indication

### Summary

The issues of configuring SRS resource sets and precoder/rank indication for multi-DCI based STxMP PUSCH+PUSCH transmission are discussed in contributions. The views are summarized as:

- Qualcomm: For multi-DCI based PUSCH operation, support two SRS resource sets, where the first SRS resource set is associated with coresetPoolIndex value 0, and the second SRS resource set is associated with coresetPoolIndex value 1, The interpretation of the SRI/TPMI field of the DCI is based on the coresetPoolIndex value of the CORESET in which the DCI is received.

- Nokia: For precoder indication of multi-DCI fully/partially overlapping PUSCH transmission, no additional enhancement is required compared to legacy (Rel-15/16) TPMI indication

- Panasonic: Support separate codebook configuration for different Tx panels. For multi-DCI based PUSCH, support two SRS resource sets, where the first SRS resource set is associated with coresetPoolIndex value 0, and the second SRS resource set is associated with coresetPoolIndex value 1

- NTT DOCOMO: For DG PUSCH+DG PUSCH, association between PUSCH and TRP can be determined based on CORESETPoolIndex of scheduling DCI, For DG PUSCH+CG PUSCH or CG PUSCH+CG PUSCH, association between CG PUSCH and TRP needs to be further discussed, e.g., CORESETPoolIndex can be configured in CG configuration.

- MediaTek: The association between DG-PUSCH and a coresetPoolIndex value is determined according to the CORESET on which the scheduling DCI transmitted; The association between Type-2 CG-PUSCH and a coresetPoolIndex value is determined according to the CORESET on which the activation DCI transmitted; The association between Type-1 CG-PUSCH and a coresetPoolIndex value is determined according to the coresetPoolIndex value configured in the corresponding CG configuration

- Intel: For multi-DCI based STxMP CG + CG PUSCH transmission, a single CG configuration can be used to configure the CG PUSCH transmitted through the two panels

- LG: Support separate codebook configuration for different Tx panels

- Google: Support to configure 2 SRS resource sets for CB/NCB, where each SRS resource set is associated with each CORESETPoolIndex, The number of ports for SRS resource set for CB should be up to 2, The number of SRS resources for a SRS resource set for NCB should be up to 2

- Lenovo: The two PUSCHs sent from the two panels can have same or different ranks, and are signalled with independent TPMIs/SRIs

- Ericsson: each PUSCH is scheduled by its own PDCCH, and the UE would apply the precoder indication in the corresponding UL grant

FL note: the views can be summarized as the following aspects:

- two SRS resource sets are configured and one set is for each TRP.

- The precoder/rank/SRI for one PUSCH is indicated by the scheduling DCI with corresponding SRS resource set.

- DG-PUSCH and type 2 CG-PUSCH are associated with coresetPoolIndex through the scheduling/triggering DCI.

- There exist two different solutions for type 1 CG-PUSCH: (1) Solution 1: configuring a coresetPoolIndex in CG PUSCH (2) Solution 2L using a single CG configuration to configure CG PUSCHs transmitted from two panels.

- Panasonic and LG proposed to configure separate codebook for different panels and Google proposed to restrict the number of SRS resources or number of SRS ports in the SRS resource sets.

### Proposal for discussion

Based on the views, the following proposal is made:

FL proposal 2-4: Regarding the TPMI/SRI indication for multi-DCI based STxMP PUSCH+PUSCH:

- Configure two SRS resource sets for CB or NCB.

  - FFS: how to associate coresetPoolIndex value implicitly or explicitly.

  - FFS the maximal number of SRS resources in each set for NCB

  - FFS the maximal number of SRS ports in each set for CB.

- FFS: Separate codebooks and separate maxRanks are configured for different SRS resource sets.

- For type 1 CG-PUSCH, FFS how to associate the PUSCH with one TRP

  - e.g., configure a coresetPoolIndex value in a type 1 CG-PUSCH

  - e.g., use a single CG to configure two type 1 CG PUSCHs for STxMP PUSCH+PUSCH

Companies’ views:

| Company | Support (Yes/No) | Comments |
| --- | --- | --- |
| QC | Yes | For separate codebooks, we are not sure about the use case. Hence, we suggest to remove the second bullet, or make it FFS. |
| Nokia/NSB | Updates needed. | In general, the proposal has too many bullets to be covered with single proposal. First main bullet: We do not support it. We agree that the association is needed. However, there is no need specify any additional explicit association of CORESETPoolIndex with UL SRS resource set. In fact, for DG-PUSCH +DG-PUSCH, the UE can determine implicitly the association between CORESETPoolIndex and PUSCH as well as indicated SRS resource set based on CORESETPoolIndex of scheduling DCI. Second main bullet: We support configurations where different UL SRS resource sets are configured with different number of antenna ports and corresponding resources enabling two different precoding configurations according UE capability. Before making agreement on panel specific rank constraint, we should make agreement on the extension of capability value set reporting where also different antenna panels having same number of SRS antenna ports can be distinguished. This is issue is under discussion in AI 9.1.1.1. Third bullet: Support Fourth bullet: Support Fourth bullet: In principle, support |
| Lenovo | Yes | Agree with QC to remove the separate codebook configuration in the second bullet: - Separate codebooks and separate maxRanks are configured for different panels. |
| Google | Yes with modification | We do not see the need to configure different codebook and maxRank for different panels. Maybe the second bullet can be removed or we can add a FFS. |
| NTT Docomo | Yes |  |
| ZTE |  | As Nokia mentioned, for DG PUSCH and Type 2 CG PUSCH, the association between CORESETPoolIndex and SRS resource set/TPMI/SRI/etc can be derived from the scheduling DCI, we fail to see the necessity of this explicit association. For Type 1 CG PUSCH, the association in RRC-level is needed. FL proposal 2-4: Regarding the TPMI/SRI indication for multi-DCI based STxMP PUSCH+PUSCH: - Configure two SRS resource sets for CB or NCB and they are associated with different coresetPoolIndex values. - FFS the maximal number of SRS resources in each set for NCB - FFS the maximal number of SRS ports in each set for CB. - Separate codebooks and separate maxRanks are configured for different panels. - For type 1 CG-PUSCH, FFS how to associate the PUSCH with one TRP - e.g., configure a coresetPoolIndex value in a type 1 CG-PUSCH - e.g., use a single CG to configure two type 1 CG PUSCHs for STxMP PUSCH+PUSCH |
| Mod |  | Update the proposal 2-4 according to comments received so far: @Nokia and ZTE: “implicitly or explicitly ” was added in first bullet and one sub-bullet with FFS on how to associate was added to address your comments. @QC, Nokia, Lenovo and google: regarding the maxRank and codebook, the second bullet is put under FFS to address your comments. |
| LG |  | We fail to find any issue with the following two subbullets and suggest to keep them. It is about how association between CORESETPoolIndex and SRS resource set/TPMI/SRI can be derived from the scheduling DCI. Is there any company having different approach in mind? - For each DG-PUSCH, the TMPI/SRI indicated in the scheduling DCI corresponds to the SRS resource set associated with the same coresetPoolIndex value as the scheduling DCI. - For each type 2 CG-PUSCH, the TMPI/SRI indicated in the activation DCI corresponds to the SRS resource set associated with the same coresetPoolIndex value as the activation DCI. |
| Samsung |  | We think first, third and fourth bullets in the previous FL’s proposal are specifying clear relationship between TPMI/SRI fields in scheduling (activation) DCI and scheduled PUSCH transmission and associated SRS resource set. To avoid any ambiguity or any other operation (e.g. two SRS sets associated with same coresetPoolIndex etc), we think those bullets are meaningful. |
| MediaTek | Yes with update | We prefer to not mention “panel”. FL proposal 2-4: Regarding the TPMI/SRI indication for multi-DCI based STxMP PUSCH+PUSCH: - Configure two SRS resource sets for CB or NCB and they are implicitly or explicitly associated with different coresetPoolIndex values. - FFS: how to associate coresetPoolIndex value - FFS the maximal number of SRS resources in each set for NCB - FFS the maximal number of SRS ports in each set for CB. - FFS: Separate codebooks and separate maxRanks are configured for different SRS resource sets. - For type 1 CG-PUSCH, FFS how to associate the PUSCH with one TRP - e.g., configure a coresetPoolIndex value in a type 1 CG-PUSCH - e.g., use a single CG to configure two type 1 CG PUSCHs for STxMP PUSCH+PUSCH |
| Spreadtrum | Yes | If separate codebooks refer to different coherent type for UE panels, we think it is unnecessary to configure different codebooks for two panels. |
| OPPO | Yes | For second bullet, we are not sure about the use case for separate codebooks, but it is ok to put it under FFS. |
| Xiaomi | Yes | Support the updated version |
| Mod 3 |  | The proposal is updated by replacing “panels” with “SRS resource sets” according to the comments of MediaTek |
| Intel | Yes with updates | For the first bullet, we agree with Nokia and ZTE that associating coresetPoolIndex explicitly is not necessary. |
| Huawei, HiSilicon | Yes with update | This is a mDCI scheme and we don’t see how the same codebook or maxRanks could be configured for the two SRS resource sets. Therefore, we suggest to remove the FFS from the second main bullet. FL proposal 2-4: Regarding the TPMI/SRI indication for multi-DCI based STxMP PUSCH+PUSCH: - Configure two SRS resource sets for CB or NCB and they are implicitly or explicitly associated with different coresetPoolIndex values. - FFS: how to associate coresetPoolIndex value - FFS the maximal number of SRS resources in each set for NCB - FFS the maximal number of SRS ports in each set for CB. - FFS: Separate codebooks and separate maxRanks are configured for different SRS resource sets. - For type 1 CG-PUSCH, FFS how to associate the PUSCH with one TRP - e.g., configure a coresetPoolIndex value in a type 1 CG-PUSCH - e.g., use a single CG to configure two type 1 CG PUSCHs for STxMP PUSCH+PUSCH |
| MediaTek | Yes | Regarding whether the association between coresetPoolIndex value and SRS resource set is needed, even UE can determine the scheduled/activated PUSCH is associated with which coresetPoolIndex value according to the scheduling/activating DCI, it is ambiguous that the indicated SRI/TPMI should be apply to which SRS resource set if no association. The association be can simple, in our view, e.g., the 1st SRS set is always associated with coresetPoolIndex 0, the 2nd SRS set is always associated with coresetPoolIndex 1. |
| CATT | Yes in principle | Fine with the current proposal but the association between SRS and coresetPoolIndex should be specified. |
| Vivo | Yes | FFS on how to associate CORESETPoolIndex is enough, so we propose to delete the description on implicitly or explicitly associated with different CORESETPoolIndex values. The example, second sub bullet of the third bullet needs some clarification. FL proposal 2-4: Regarding the TPMI/SRI indication for multi-DCI based STxMP PUSCH+PUSCH: - Configure two SRS resource sets for CB or NCB and they are implicitly or explicitly associated with different coresetPoolIndex values. - FFS: how to associate coresetPoolIndex value - FFS the maximal number of SRS resources in each set for NCB - FFS the maximal number of SRS ports in each set for CB. - FFS: Separate codebooks and separate maxRanks are configured for different panels. - For type 1 CG-PUSCH, FFS how to associate the PUSCH with one TRP - e.g., configure a coresetPoolIndex value in a type 1 CG-PUSCH - e.g., use a single CG to configure two type 1 CG PUSCHs for STxMP PUSCH+PUSCH |
| Ericsson | No | Why is this needed? Already in Rel-16, we can configure two SRS resource sets, which is each scheduled by one PDCCH. That association is “implicit”, and works fine. |
| Mod 4 |  | @MediaTek and CATT: associate each SRS resource set with one CORESETPoolIndex value is one way. However, as you can see in the comments, some companies think it might not need such explicit association. So let us keep it open for now. @vivo: your suggestion is added in the proposal and “implicitly or explicitly” is added to make it more clear that either way is considered. @Ericsson: implicit method is still included here. “implicitly or explicitly” is added in the FFS sub-bullet to clarify either way is included. |

### Summary of Round 1

The proposal was revised a few times with wording change according to the inputs.

Ericsson commented that this proposal is not needed since it is already in Rel-16. But per the understanding of moderator, what proposed here is not in the spec yet. To support the PUSCH+PUSCH in multi-DCI system, such proposal is needed.

### Proposal for Round 2 Discussion

Here is the latest proposal from last round. Some change tracks are removed:

FL proposal 2-4: Regarding the TPMI/SRI indication for multi-DCI based STxMP PUSCH+PUSCH:

- Configure two SRS resource sets for CB or NCB.

  - FFS: how to associate coresetPoolIndex value implicitly or explicitly.

  - FFS the maximal number of SRS resources in each set for NCB

  - FFS the maximal number of SRS ports in each set for CB.

- FFS: Separate codebooks and separate maxRanks are configured for different SRS resource sets.

- For type 1 CG-PUSCH (if supported), FFS how to associate the PUSCH with one TRP

  - e.g., configure a coresetPoolIndex value in a type 1 CG-PUSCH

  - e.g., use a single CG to configure two type 1 CG PUSCHs for STxMP PUSCH+PUSCH

Comments on Updated Proposal 2-4, hope we can settle down the wording in this round

| Company | Support (Yes/No) | Comments |
| --- | --- | --- |
| QC | Support | @ Ericsson: We cannot configure two SRS resource sets in Rel-16. Two SRS resource sets was first introduced in Rel-17 for the purpose of single-DCI TDM scheme. |
| LG | Yes |  |
| NTT Docomo | Yes |  |
| MediaTek | Yes |  |
| CATT | Yes |  |
| ZTE | Yes | @Ericsson: In Rel-16, only one SRS resource set can be set to ‘codeBook’ or ‘nonCodebook’. |
| OPPO | Yes |  |
| Xiaomi | Yes |  |
| Intel | Yes |  |
| Huawei, HiSilicon | No | We fail to see how the same codebook or maxRank can be configured for the PUSCH+PUSCH in m-DCI scheme and prefer remove the FFS from - FFS: Separate codebooks and separate maxRanks are configured for different SRS resource sets. Also, CG-PUSCH is not supported yet and this needs to be reflected in the proposal. We suggest the following modification: FL proposal 2-4 (modified): Regarding the TPMI/SRI indication for multi-DCI based STxMP PUSCH+PUSCH: - Configure two SRS resource sets for CB or NCB. - FFS: how to associate coresetPoolIndex value implicitly or explicitly. - FFS the maximal number of SRS resources in each set for NCB - FFS the maximal number of SRS ports in each set for CB. - FFS: Separate codebooks and separate maxRanks are configured for different SRS resource sets. - For type 1 CG-PUSCH if supported, FFS how to associate the PUSCH with one TRP - e.g., configure a coresetPoolIndex value in a type 1 CG-PUSCH - e.g., use a single CG to configure two type 1 CG PUSCHs for STxMP PUSCH+PUSCH |
| Spreadtrum | Yes |  |
| Nokia | Yes | Ok with the latest FL version. We do not think group is having good understanding to list “separate maxRanks are configured for different SRS resource sets”. Having FFS is fine. |
| Samsung | Yes | We prefer to keep FFS for further study. |
| Lenovo | Yes |  |
| Mod |  | @HW and Nokia, given the comments in round 1 and round 2, I do agree that the group is not ready for have common understanding on “separate maxRanks and codebooks”. So let us keep it to FFS for now. @HW The “if supported” is added to Type 1 CG-PUSCH. With that, the proposal 2-4 seems to be stable if no more concerns are raised. |

## Issue #5: Configuration restriction for multi-DCI based STxMP PUSCH+PUSCH

### Summary

Companies proposed to study or specify some scheduling/configuration restriction on PUSCHs in multi-DCI based STxMP PUSCH+PUSCH:

- Huawei/HiSilicon: further study if UE is not expected to assume different DM-RS configuration with respect to the following parameters: the actual number of front-loaded DM-RS symbol(s), the actual number of additional DM-RS symbol(s), the actual DM-RS symbol location, DM-RS configuration type

- Spreadtrum: For the conditions required for STxMP PUSCH+PUSCH, relax the restriction on DMRS, e.g. DMRS ports from same or different CDM group for mTRP

- Google: For mDCI based STxMP on PUSCH, support to indicate up to 2 DMRS ports in the DCI

- CATT: The following scheduling restrictions can be considered by STxMP transmission: The maximum number of layers for each PUSCH is predefined in specification or is configured by higher layer signaling in advance; Two scheduled PUSCHs have the same DMRS configuration; The same active BWP and the same SCS are scheduled by two TRPs

- Xiaomi: the following rules are suggested to minimize the interference between two PUSCHs:  Same DMRS configurations for different PUSCHs, eg. DMRS type, the number of the FL DMRS, etc DMRS port(s) allocation of different PUSCHs belong to different CDM groups; When partially/fully overlapping occurs in time domain for STxMP, the PUSCH can use the “CDM groups without data” in “Antenna Port” field to perform the rate matching towards the DMRS port(s) of the other PUSCH on the overlapped symbols

FL note: to summarize, the proposals mainly focus on the following aspects:

- Whether same or different DMRS configurations can be applied to PUSCHs in STxMP PUSCH+PUSCH,

- For two overlapping PUSCHs, whether same or different CDM groups

- Whether/how to limit the max number of layers of each PUSCH in addition to total number of layers being <= 4.

- Whether and how PUSCH rate match to the DMRS ports of the other PUSCH.

From the understanding of FL, it seems worthwhile to study those aspects

### Proposal for discussion

According to the views, the following proposal is made:

FL proposal 2-5: For multi-DCI based STxMP PUSCH+PUSCH, study the following aspects:

- Whether different DMRS configurations can be applied to overlapping PUSCHs.

- Whether the DMRS ports of two overlapping PUSCHs shall be in different CDM groups.

- Whether/how to restrict the max number of layers per PUSCH in addition to the requirement of that the total number of layers of two overlapping PUSCHs is up to 4.

- Whether/how to perform PUSCH rate match towards to the DMRS ports of the other PUSCH on overlapped symbols in STxMP PUSCH+PUSCH transmission.

- Note: other aspects are not precluded

Companies’ views:

| Company | Support (Yes/No) | Comments |
| --- | --- | --- |
| QC |  | Other than max number of layers (third bullet), the rest seems to be related to gNB scheduling / channel estimation at the gNB. If clean channel estimation is needed, gNB can schedule according to these restrictions. If inter-beam interference is small and TRPs decode the two PUSCHs separately, then such restrictions may not be needed. Anyway, it is ok to study. |
| Nokia/NSB | Yes(with updates) | first bullet: Support to study second bullet: Support to study third bullet: Not support Fourth bullet: Support to study |
| Lenovo |  | It seems the 3rd bullet should be an implementation issue. Suggest to remove the 3rd bullet. |
| Google | No | We failed to see the necessity for all the study points. |
| NTT Docomo | Yes | Fine to further study |
| ZTE | Yes | Open to further study |
| LG | Yes | Open to further study |
| Samsung | Yes | Fine to study |
| MediaTek | Yes | Open to further study |
| Spreadtrum | Yes | Support to study |
| OPPO | Yes | Fine to further study |
| Xiaomi | Yes | Support to further study |
| Intel | Yes | Support. |
| Huawei, HiSilicon | Yes | OK to study |
| CATT | Yes | Support to further study |
| vivo | Yes |  |
| Ericsson |  | Anyone is free to study anything. We do not see the need for any of these enhancements (the third bullet is discussed under issue #1) |

### Summary of round 1

Google thought that it is not necessary to study those items. Ericsson commented that it is free to study anything but they do not see the need for the listed items. Lenovo and Nokia/NBS do not support 3rd bullet.

From the understanding of moderator, this is not a high priority issue for this meeting and it might be a better way for the company to bring up each individual issue along with the proposed solution, which might be helpful for the discussion. Given that, moderator suggested to cease the discussion on this issue. The company can study any issue they find and propose the issue and the solution to the issue in following meetings.

# STxMP PUCCH

It was agreed to evaluate the STxMP PUCCH transmission in single-DCI based system and in multi-DCI based system. In the contributions, companies provided evaluation results of STxMP PUCCH transmission in single-DCI based system and/or multi-DCI based system. Proposals for schemes of STxMP PUCCH in single-DCI based system and STxMP PUCCH in multi-DCI based system were also provided in tdocs

## Issue #1: single-DCI based STxMP PUCCH

### Summary

Companies provided evaluation results for STxMP PUCCH transmission in single-DCI based system and companies also provided views/proposal on whether to support STxMP PUCCH transmission in single-DCI based system and the transmission scheme for STxMP PUCCH.

Companies provided evaluation results of STxMP PUCCH in single-DCI based system:

Huawei/HiSilicon: - LLS and BLER performance - FDM based PUCCH: single PUCCH payload is Txed over two FDMed PUCCH resource: Figure 2. BLER performance of STxMP and TxSP Observation 7: The performance comparison of STxMP transmission of PUCCH+PUCCH over the baseline TxSP does not justify supporting this scheme.

NTT DOCOMO - LLS and BLER performance - SFN scheme Figure3. BLER performance of STxMP PUCCH SFN scheme for PUCCH format 1 Figure4. BLER performance of STxMP PUCCH SFN scheme for PUCCH format 3 Observation 3.1: - For both PUCCH format 1 and format 3, when X=6dB or X=20dB or when each TRP only receives signal from the best panel, SFN scheme is about 1dB worse than TDM repetition.

Regarding whether to support STxMP PUCCH in single-DCI based system:

- Huawei/HiSilicon proposed to not support it in single-DCI based system and MediaTek proposed that it shall be low priority for Rel-18.

- On the other hand, companies proposed to support some scheme for single-DCI based STxMP PUCCH transmission:

- SFN scheme, one PUCCH resource is indicated with two TCI states:

  - Supported: Spreadtrum, vivo, Ericsson, Lenovo, LG, CATT, Apple, Intel, NTT DOCOMO, Qualcomm, Nokia

- FDM scheme(s):

  - Supported: Lenovo, CATT, Apple

- The scheme of repeating one UCI in two PUCCH resources:

  - Supported: Google

- PUCCH CDM repetition scheme for PUCCH format 1/4:

  - Supported: Intel

- Scheme based on the STxMP PUSCH transmission scheme:

  - Supported: ZTE

FL note: Huawei/HiSilicon provided the BLER performance of FDM scheme of single-DCI based STxMP PUCCH and NTT DOCOMO provided the BLER performance of SFN scheme. And, SFN scheme of STxMP PUCCH is supported with many companies..

### Proposal for discussion

According to the views in tdocs, it looks like SFN scheme could be a good wayforward for single-DCI based STxMP PUCCH. It is supported by good number of companies. And in the tdocs, companies explained that only simple spec change, i.e., configuring two beams that are used at the same time, is needed to implement it. Therefore, the FL would like to make the following proposal:

FL proposal 3-1: Support to specify SFN scheme for single-DCI based STxMP PUCCH

- One PUCCH resource can be indicated with two TCI states and the PUCCH resource is transmitted by applying two TCI states at the same time.

Companies’ views:

| Company | Support Proposal 4-1 (Yes/No) | Comments |
| --- | --- | --- |
| QC | Yes |  |
| Nokia/NSB | Yes | Support FL’s proposal. |
| InterDigital | Yes | Support FL’s proposal. |
| Lenovo | Yes | Support FL proposal. |
| Google | Yes |  |
| NTT Docomo | Yes |  |
| ZTE | Yes |  |
| NEC | Yes |  |
| Fujitsu | Yes |  |
| LG | In principle, Yes | R-18 unified TCI framework does not have to be pre-requisite for STxMP PUCCH. FL proposal 3-1: Support to specify SFN scheme for single-DCI based STxMP PUCCH One PUCCH resource can be indicated with two TCI states or two spatial relations and the PUCCH resource is transmitted by applying two TCI states or two spatial relations at the same time |
| MediaTek | Yes |  |
| Spreadtrum | Yes | Support FL’s proposal |
| OPPO | Yes | Support FL’s proposal |
| Xiaomi | Yes | Support FL’s proposal |
| Mod 3 |  | @LG: my understanding is Rel18 is based on the Rel-17 unified TCI framework, thus there is no ‘spatial relations’ for PUCCH. |
| Intel | Yes | Support. |
| Huawei, Hisilicon | No | Do not support. Only two companies (NTT, HW) have performed LLS evaluations and neither of the companies have observed a BLER performance gain that justifies the support for SFN PUCCH. According to our simulation result, STxMP performance gain can be achieved only when no path loss gap exists between the two panels and, even in that case, it is still a marginal gain at a low SNR region. Note that 0dB path loss gap between the two panels is quite unrealistic in a practical scenario. |
| CATT | Yes |  |
| vivo | Yes |  |
| Ericsson | Yes |  |

### Summary of round 1

The proposal was supported by: QC, Nokia/NSB, InterDigital, Lenovo, Google, NTT Docomo, ZTE, NEC, Fujitsu, LG, MediaTek, Spreadtrum, OPPO, Xiaomi, Intel, CATT, vivo, Ericsson. but objected by: Huawei, Hisilicon.

NTT DOCOMO provided the BLER performance of SFN scheme and showed that SFN can provide BLER performance close to that of PUCCH TDM scheme.

Given the inputs in round 1, moderator would suggest to put Proposal 2-2 in email endorsement

## Issue #2: multi-DCI based STxMP PUCCH

### Summary

Companies provided evaluation results for STxMP PUCCH transmission in multi-DCI based system and companies also provided views/proposal on whether to support STxMP PUCCH transmission in multi-DCI based system. Proposals on detailed spec change related with UCI dropping/multiplexing for STxMP PUCCH+PUCCH are also provided in tdocs.

Evaluation results of STxMP PUCCH+PUCCH in multi-DCI based system are provided in tdocs:

Huawei/HiSilicon: - LLS and BLER performance - STxMP PUCCH+PUCCH: two independent PUCCHs are transmitted by two UE panels on two PUCCH resources overlapped in time and frequency domain. Figure 3. BLER performance of multi-DCI based STxMP and TxSP Observation 7: The performance comparison of STxMP transmission of PUCCH+PUCCH over the baseline TxSP does not justify supporting this scheme.

Regarding whether to support STxMP PUCCH+PUCCH in multi-DCI based system, views in the contributions are:

- Alt1: Do not support STxMP PUCCH in multi-DCI based system

  - Support: Huawei/HiSilicon,

- Alt2: Support the STxMP PUCCH+PUCCH in multi-DCI based mTRP system, two PUCCH resources with separate UCI payloads towards different TRP can be transmitted simultaneously from different UE panels.

  - Support: ZTE, Spreadtrum, vivo (frequency-domain non-overlapping can be 1st priority), Lenovo, CATT, Intel, MediaTek, NTT DOCOMO, Qualcomm, Nokia

Companies also provided views/proposals on UCI multiplexing/dropping if STxMP PUCCH+PUCCH is supported:

- ZTE: supporting transmitting UCI on different PUCCH instead of dropping or multiplexing

- vivo: consider TRP/panel-specific UCI multiplexing, and FFS on how to handle overlapped PUSCH/PUCCH for TRP/panel-specific UCI multiplexing.

- Lenovo: Study the UCI multiplexing rule in STxMP PUSCH and/or PUCCH transmission

- CATT: UCI multiplexing and PUSCH/PUCCH dropping for overlapped PUCCH/PUSCH transmission.

- Intel: Study new collision handling rules for the single PUSCH + STxMP PUCCH, single PUCCH + STxMP PUSCH, STxMP PUCCH + STxMP PUSCH

- MediaTek: study the UE procedure of reporting multiple CSI and priority rules if STxMP PUCCH is supported.

- NTT DOCOMO: PUCCHs associated with same panel/TRP shall follow the multiplexing/dropping rule of Rel-17 and PUCCHs associated with different panel/TRP can be STxMP.

- Qualcomm: FFS the UCI multiplexing such as per CORESETPoolIndex value UCI multiplexing.

MediaTek proposed to study how to determine the association between PUCCH resource and TRP and provided a few Alts.

### Proposal for discussion

For the work on multi-DCI based STxMP PUCCH+PUCCH, it is good to first align the understanding on what kinds of spec change/frame work is required to specify this PUCCH+PUCCH. Therefore, FL would like to first gather your answers to the following questions.

Question 3-2: Please provide your views on the following questions?

- Q1: do you support to specify STxMP PUCCH+PUCCH in multi-DCI based mTRP system, where two PUCCH resources with separate UCI payloads towards different TRPs can be transmitted simultaneously from different UE panels?

- Q2: If your answer to Q1 is yes, please explain spec changes/impacts/framework that you think is needed for supporting the multi-DCI based STxMP PUCCH+PUCCH in Rel-18.

FL note: Please input your answers/understanding to these two questions.  A clear description on your expected spec change for multi-DCI based PUCCH+PUCCH would be helpful to formulating next step proposal.

| Company | You answers |
| --- | --- |
| QC | Regarding the LLS in [1], we would like to point out that it actually shows gain based on our understanding at least for power assumption 2. This is because two UCIs with the same size are transmitted (effectively, UCI payload is twice in STxMP PUCCH with multi-DCI) while BLER curve is almost the same. Q1: Yes. Q2: The proper framework for this is in the case of separate feedback for multi-DCI, and the spec impact is limited to performing legacy UCI multiplexing per TRP. Please note that the issue of dropping due to PUCCH+PUSCH not being in scope is still there irrespective of whether we have this enhancement (PUCCH+PUCCH) or not. The main benefit is overhead reduction for PUCCH and allowing gNB to schedule / configure PUCCH w/o all the error cases that exist today as a result of Rel-16 restriction. In our contribution, we provided multiple examples and discussed why those are invalid gNB scheduling and the impact that it has on resource utilization (effectively, semi-static partition of a slot for UCI transmissions are needed in Rel-16). This is due to “TDM restriction” of Rel-16 as well as Rel-15 complicated iterative UCI multiplexing (PUCCH resource determination) and various issues if one of HARQ-Acks overlap with other UCIs even if the two HARQ-Acks do not overlap with each other. Performing UCI multiplexing per TRP and simultaneously transmitting the resulting PUCCHs will avoid all these issues. |
| Nokia/NSB | Q1: Support Q2: FFS different dropping rules |
| InterDigital | Q1: Support Q2: The multiplexing/collision/priority rules should be enhanced to support the overlapping PUCCH case when a UE is capable of STxMP. |
| Lenovo | Q1: Yes Q2: UCI multiplexing and UCI dropping rule should be enhanced. For example, the UE may need to perform per TRP UCI multiplexing with STxMP PUCCH capability. |
| Google | Q1: No. But we are open to the same UCI payload to improve reliability. |
| NTT Docomo | Q1: Yes Q2: UCI multiplexing/dropping procedure needs to be studied. We think the rule can be simple. For example, in case of PUCCH overlapping, overlapping among PUCCH transmissions associated with same panel/TRP can be resolved first with Rel-17 UCI multiplexing/dropping rule. Then, for overlapping between two PUCCH transmissions associated with different panel/TRP, the two PUCCHs can be transmitted simultaneously. |
| ZTE | Q1: Yes Q2: If UCI on different PUCCHs which are overlapped in time domain and also transmitted from different panels towards different TRPs, UCI could be loaded in these PUCCHs respectively. Otherwise, the legacy for UCI multiplexing and dropping rules should be followed. |
| LG | Q1: Yes Q2: same view with NTT and ZTE. |
| Samsung | Q1: No Q2: We want to handle this issue carefully. Other than PUSCH, reliability is most important performance for PUCCH. If max power per UE is assumed, we don’t think PUCCH reliability can always be maintained. Supporting sDCI based SFN PUCCH can be considerable to increase reliability, but this scheme can make each PUCCH transmission less reliable. And in Huawei’s LLS simulation result, BLER performance for PUCCH STx2P is degraded than panel selection scheme. We have a concern on PUCCH reliability with two different PUCCH+PUCCH STx2P. |
| MediaTek | Q1: Yes Q2: Study on UCI multiplexing/dropping rule and CSI collision rule in needed. |
| Spreadtrum | Q1: Yes Q2: FFS: UCI multiplexing/dropping procedure per TRP. |
| OPPO | Q1: Yes Q2: It is preferred to keep the UCI multiplexing/dropping rules simple. The overlapping PUCCHs associated with same panel/TRP can be handled with legacy UCI multiplexing/dropping rule, and the PUCCHs associated with different panels/TRPs can be transmitted simultaneously. |
| Xiaomi | Q1: Yes Q2:FFS the UCI multiplexing/dropping rules |
| Intel | Q1: Yes. Q2: Allowing different UCIs being transmitted simultaneously from two panels can achieve per TRP/panel UCI multiplexing/dropping, where legacy UCI multiplexing and collision handling rules can be reused as much as possible. |
| Huawei, HiSilicon | Q1: No. HW is the only company that performed LLS for m-DCI based PUCCH + PUCCH and did not observe any gain. |
| CATT | Q1: Yes Q2: UCI multiplexing and dropping rules should be supported and studied. Also, power control should also be studied. |
| vivo | Q1：Yes Q2：For STxMP transmission, two overlapped PUSCHs or two overlapped PUCCHs can be simultaneously transmitted towards two TRPs. How to multiplex the UCI on the overlapped PUSCHs or PUCCHs should be further discussed. One simple solution is to reuse legacy rules for each TRP. Since simultaneous transmission of a PUCCH and a PUSCH is not part of the WID, how to handle the overlapped PUCCH and PUSCH for TRP-specific UCI multiplexing should be further studied. |
| Ericsson | Q1: No Q2: we agree with Samsung. In addition, we are concerned about the spec impact regarding UCI collision/dropping/multiplexing. |

### Summary of round 1

Regarding the Q1: we have the following answers

- Yes: QC, Nokia/NSB, InterDigital, Lenovo, Google, NTT Docomo, ZTE, Spreadtrum, OPPO, Xiaomi, CATT, vivo,

- No: Samsung, Huawei, HiSilicon, Ericsson

Regarding the Q2, the expected spec change/framework can be summarized as:

- To support per TRP UCI multiplexing rule, and how to multiplex the UCI on PUCCHs of each TRP still follows the current specification.

- PUSCH+PUCCH is out of scope of this issue per the WID

Samsung and Huawei/HiSilicon raised the concern on reliability of PUCCH transmission when PUCCH+PUCCH are transmitted in mDCI system. That is a valid concern and more study on that seems needed. And Ericsson has concerns on the spec impact.

### Proposal for Round 2 Discussion

Proposal 3-2 on one Conclusion:

If STxMP PUCCH+PUCCH in multi-DCI based mTRP system is supported, the work scope includes:

- Specify per TRP UCI multiplexing rule. For PUCCHs of each TRP, how to multiplex the UCI still follows the current specification.

- The case of PUSCH+PUCCH is out of scope of this issue.

Comments on Updated Proposal 3-2

| Company | Support (Yes/No) | Comments |
| --- | --- | --- |
| QC | No | @FL: We already have an agreement for study, and there is no need for another agreement. Given the majority support, we suggest continuing the discussions to understand the concerns above better. We do not think evaluating the BLER performance can help here due to the reasons that I explain below. We would like to address the concerns from Samsung, Huawei, and Ericsson: @Samsung: Our understanding is that your point is about Tx power, not about increasing the reliability of PUCCH (same UCI) using multi-DCI framework. If that’s the correct understanding, then it is already clear (w/o the need for LLS) that power assumption 1 results in 3dB less max power per PUCCH and power assumption 2 does not result in any power reduction. Then even if we just focus on power assumption 1, the benefit of PUCCH+PUCCH remains when the deployment is not coverage-limited. From the PUSCH SLS evaluations, it is already observed that coverage is not an issue in InH. Hence, network can enable this feature in proper deployment and for proper UEs. As a comparison, in FR1, we have per-UE power constraint even across multiple CCs. However, we also do have the feature of “two PUCCH groups” to transmit two PUCCH simultaneously in different CCs. Even though your point is valid wrt max coverage also in this case, it is up to network when to use this feature, which is possible in legacy. @Huawei: Thank you for the LLS. Can you please confirm if the UCI size remains the same with in both TxSP (one UCI) as well as STxMP (two UCIs)? If so, comparing the BLER may not be a fair comparison since twice the payload is transmitted in STxMP. Furthermore, the goal of this enhancement is not to improve the BLER, but it is to reduce the PUCCH overhead and relax the Rel-16 error cases that result in very inefficient scheduling (please also see below for further explanation) @Ericsson: My understanding is that even w/o this enhancement, there is spec impact on UCI multiplexing. For example, assume that PUSCH 1 and PUSCH 2 are transmitted simultaneously, and a PUCCH overlaps with both. A rule is required to decide to multiplex UCI on which PUSCH(s). As companies clarified, the spec impact is limited to per-TRP UCI multiplexing. At the same time, the benefit is clear. The main goal of this enhancement is to relax the existing error cases in Rel-16 given that Rel-18 UE has the capability to transmit two PUCCHs at the same time. As we all know, Rel-16 puts a lot of constraints on PUCCH resource configuration/indication, which effectively results in semi-static split of resources among the two TRPs. Just a few examples are illustrated below (see also our contribution). Please note that Example 4 always results in error case, and examples 3 and 5 depends on the resultant PUCCH resources after Rel-15 UCI multiplexing as we do not have per-TRP UCI multiplexing, which illustrates the scheduling restriction in the absence of this enhancement: |
| NTT Docomo |  | Similar view as QC |
| CATT |  | We believe that STxMP PUCCH+PUCCH in multi-DCI should be supported. The scope for further studies should be stated clearly. |
| ZTE |  | MDCI based STxMP PUSCH+PUSCH transmission is indeed in the scope and is also supported by majority. Hence the second bullet should be agreeable. |
| Huawei, HiSilicon | No | HW is the only company that performed LLS for m-DCI based PUCCH + PUCCH and did not observe any gain. @Qualcomm: Thank you for the question. the UCI size remains the same in both TxSP as well as STx2P. But we disagree why it may be an unfair comparison: - To analyse the performance of STxMP PUCCH+PUCCH in multi-DCI, the BLER of two SDMed PUCCH should be compared with BLER of two TDMed PUCCH. We don’t see why the payload of each SDMed UCI should be half of the payload of each TDMed PUCCH to make it a fair comparison. If proponents of STxMP PUCCH+PUCCH argue that this scheme should be supported for a latency reduction, even if the latency is reduced, a price that needs to be paid is inter-PUCCH interference which causes an increase in BLER. - Multi-DCI SDM STx2P PUCCH+PUCCH in fact correspond to SDM transmission of two PUCCHs each of which associated with one of the two PDSCHs in a m-DCI SDM STx2P PDSCH+PDSCH. The question is why the payload of a UCI corresponding to a m-DCI STx2P PDSCH should be half of the payload of a UCI corresponding to a TxSP PDSCH? Finally, as for the latency reduction, we don’t recall seeing any tangible latency analysis of SDM STx2P PUCCH+PUCCH vs. TxSP PUCCH in proponents t-docs. Sending two PUCCHs at the same time, at best, nominally reduces a part of layer 1 latency which is just a fraction of the overall end to end latency (including higher layer latency and the delay between PDSCH reception to HARQ Tx). And even that comes at the price of a higher PUCCH BLER which actually may result in HARQ re-Tx and even increasing the overall latency. |
| QC |  | @Huawei: Thank you for the follow-up. We would like to clarify some misunderstanding as below: - Regarding “the BLER of two SDMed PUCCH should be compared with BLER of two TDMed PUCCH”, yes we agree. But then, do you use the same number of resources (same number of RB/symbols) between TDM and SDM? Otherwise, the BLER of TDM may be better but it is using twice the resources. Then, is it fair comparison between TDM and SDM by just looking at BLER? We do not think so. - Regarding “price that needs to be paid is inter-PUCCH interference which causes an increase in BLER”: This is not necessarily the case for multi-DCI. The two PUCCHs may be overlapping in time but non-overlapping in frequency, which still achieves the latency reduction benefit. - Regarding latency reduction, I think we can all agree that reducing “layer 1 latency” is the main focus in RAN1, which has been the motivation for many features so far, e.g., span-based PDCCH, sub-slot based PUCCH, PUSCH repetition Type B, mTRP FDM PDSCH Schemes A/B, etc. - In addition, as I commented above, latency reduction is not the only point of this enhancement. Relaxing strict Rel-16 scheduling restrictions is a more important benefit as the examples above illustrate. |
| Samsung |  | @QC: Thank you for further clarification. For power perspective, I see what your point is (if not coverage-limited). But for spec impact on UCI multiplexing, we are not sure that the spec impact is limited. We think many conditions for multiplexing should be considered, e.g., DCI for activation of SPS CSI reporting, handling PUCCH for periodic CSI reporting or overlapping condition for PUCCH with CSI and two PUCCHs with HARQ and so on. |
| Lenovo | Yes | Similar view with QC. |
| Mod |  | According to the comments received so far. The proposal is updated as follows: - The proposal of “Further performance ” is removed. - The whole proposal is changed to make an conclusion on the work scope if multi-DCI based PUCCH+PUCCH is supported, the purpose for this is to align the understanding among companies. Please share your views on that. |
| QC |  | @Samsung: Thank you for the response. In our understanding, the spec impact is not large. For DCI for activation of SPS CSI reporting, given that it is on PUSCH, it should not impact PUCCH+PUCCH enhancement. For other parts, we think per-TRP UCI multiplexing can handle all cases. @Mod: Given the wide support, and the fact that concerns have been addressed, we prefer to have an agreement instead of conclusion. |

# Other Issues

If there are other issues that should be discussed, please indicate them below.

Companies’ views:

| Company Name | Comments |
| --- | --- |
| Nokia/NSB | To improve further the configuration flexibility and DMRS channel estimation quality for simultaneous multi-panel PUSCH transmission in Rel-18, it would be beneficial to consider different ways to dynamically change between different DMRS configuration types for per PUSCH transmission. Current specification defines that DMRS type (i.e. type-1 or type-2) is semi-statically configured and valid for all PUSCH transmissions within a cell. Therefore, the current specification does not allow the possibility to adapt DMRS type according to TRP specific channel conditions for each PUSCH transmission. As a result of this, UL DMRS channel estimation quality can be degraded leading to degradation in PUSCH performance. Furthermore, DMRS resource overhead may turn out to be too high restricting efficient resource utilization. Based on this discussions, Rel-18 should support configuration of DMRS type per UL PUSCH transmission and study further different options how to support it. Proposal 3: Support configuration of DMRS type per simultaneous multi-panel PUSCH transmission. - FFS: details how to support it. Current specification defines that single precoding type (i.e. codebook or non-codebook) is semi-statically configured and valid for all PUSCH transmissions within a cell. Therefore, the specification does not allow possibility adapt precoding type according to TRP specific channel conditions for each PUSCH transmission. However, from a system perspective, this can be seen as a beneficial feature for providing further performance enhancements into simultaneous multi-panel PUSCH transmission because channel conditions associated with links between different TRPs and UE may vary significantly. Furthermore, additional performance gains can be expected when different precoding types could be jointly used for simultaneous multi-panel PUSCH transmission in M-TRP operation. Proposal 4: Support configuration of precoding type per simultaneous multi-panel PUSCH transmission. - FFS: details how to support it. |
| ZTE | Given that simultaneous uplink transmission in MTRP operation targets for different types of UE device (i.e., CPE,FWA, vehicle, industrial devices), it is beneficial to provide greater diversity for transmission schemes because of various MP-UE implementations. In addition to SDM scheme, FDM-B scheme can be supported. For FDM-B scheme, two PUSCH repetitions can be transmitted by two beams of different frequency domains but within the same time domain and occupy the same transmission layers. Compared with SDM scheme, the most advantage of this scheme may be no inter-layer transmission interference due to FDMed multiplexing. Besides, this scheme could reduce latency additionally when compared with Rel-17 TDM based MTRP PUSCH repetition scheme. Proposal: Support FDM-B scheme for single DCI based STxMP PUSCH transmission when considering the performance gain of transmission reliability, latency reduction, and the diversity of transmission scheme for MP-UE implementation. |

# Contributions for RAN1#110bis-e

- R1-2208444	Discussion on UL precoding indication for multi-panel transmission	Huawei, HiSilicon

- R1-2208498	Further Discussion for Uplink Multi-panel Transmission	InterDigital, Inc.

- R1-2208507	Enhancements on UL precoding indication for multi-panel transmission	ZTE

- R1-2208544	Discussion on UL precoding indication for multi-panel transmission	Spreadtrum Communications

- R1-2208631	Discussion on UL precoding indication for multi-panel transmission	vivo

- R1-2208678	UL precoding indication for multi-panel transmission	Ericsson

- R1-2208745	UL precoding indication for multi-panel transmission	Lenovo

- R1-2208797	Transmission scheme and UL precoding indicaton for multi-panel transmission	OPPO

- R1-2208875	On Simultaneous Multi-Panel Transmission	Google

- R1-2208896	UL precoding indication for multi-panel transmission	LG Electronics

- R1-2208950	UL precoding indication for multi-panel transmission	CATT

- R1-2209009	Discussion on UL precoding indication for multi-panel transmission	Fujitsu

- R1-2209044	UL precoding indication for multi-panel transmission	Intel Corporation

- R1-2209143	Discussion on UL precoding indication for multi-panel transmission	NEC

- R1-2209261	Enhancements on multi-panel uplink transmission	xiaomi

- R1-2209325	Discussion on UL precoding indication for multi-panel transmission	CMCC

- R1-2209384	Views on UL multi-panel transmission	Sharp

- R1-2209496	Simultaneous transmission across multiple UE panels	MediaTek Inc.

- R1-2209549	On UL precoding indication for simultaneous multi-panel transmission	Fraunhofer IIS, Fraunhofer HHI

- R1-2209573	Views on UL precoding indication for multi-panel simultaneous transmissions	Apple

- R1-2209719	Views on UL precoding indication for STxMP	Samsung

- R1-2209893	Discussion on multi-panel transmission	NTT DOCOMO, INC.

- R1-2209972	Simultaneous multi-panel transmission	Qualcomm Incorporated

- R1-2210026	UL precoding for multi-panel transmission	PANASONIC

- R1-2210066	Precoder Indication for Multi-Panel UL Transmission	Nokia, Nokia Shanghai Bell