3GPP TSG RAN WG1 #110bis-e		                                                      R1-2210316

e-Meeting, October 10th – 19th, 2022

Source:	Moderator (CATT)

Title:	Summary #3 of subband non-overlapping full duplex

Agenda Item:	9.3.2

Document for:	Discussion and Decision

- Introduction

This document summarizes the inputs under AI 9.3.2 on SBFD and the discussions under the following email thread in RAN1#110bis-e.

[110bis-e-R18-Duplex-02] Email discussion on subband non-overlapping full duplex by October 19 – Yanping (CATT)

- Check points: October 14, October 19

- General aspects of SBFD schemes

This section discusses the general aspects of SBFD schemes except self-interference, inter-gNB and inter-UE CLI handling schemes.

  - Summary of input contributions

The inputs from companies’ contributions on SBFD operation except self-interference, inter-gNB and inter-UE CLI handling schemes are summarized below as per moderator’s understanding. Moderator would like to apologize in advance if your views are not correctly captured or missed and encourage companies to correct/update the summary with revision marks if needed.

    - SBFD schemes across BWP(s) within a carrier

The following working assumption was made in RAN1#110.

Working Assumption For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline. - FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies - FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair

Huawei [6], CATT [12], Xiaomi [20], Samsung [25], DOCOMO [28] and Qualcomm [30] support to confirm the above working assumption.

Vivo [10] proposed to confirm the working assumption with the following update to clearly reflect the RB-set based SBFD scheme.

For SBFD operation within a TDD carrier, study RB-set based SBFD scheme where a RB set consists of one set of contiguous RBs within a single configured DL and UL BWP pair with aligned center frequencies as baseline. - FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies - FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair

Spreadtrum [9] proposed to only confirm the aligned center frequency part of the WA.

Sony [16] proposed to confirm the working assumption without the FFS.

Although moderator understands the intention from vivo to update the working assumption for clarity, it is not preferred from moderator’s perspective since similar proposal was discussed in the last meeting and the intention to reword the proposal in the last meeting was to avoid introducing new terminology. Moderator believes that companies have the same understanding even without further modification for clarification. Spreadtrum and Sony would like to not confirm part or all of the FFS bullets. However, given that they are anyway FFS, moderator would suggest confirming the FFS part as well. Therefore, it is proposed to confirm the working assumption without any update in Proposal 1-1.

SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies

SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is supported by Spreadtrum [9], CATT [12], Ericsson [18], DOCOMO [28], Qualcomm [30], Nokia [32] and WILUS [35].

It is proposed in Proposal 1-2 to agree to support SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline for SBFD operation within a TDD carrier.

SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies

One potential use case for SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies is for {D U} pattern shown below. LG [27] proposed to support SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies for this case. Huawei [6] analysed additional UE complexity and limitation of scheduling flexibility. To be more specific, UE has to do RF retuning in case of switching between UL and DL. In addition, this method may not work well for the frame structure including both X slot and U slot that UE may not make full use of the frequency resources in U slot.

Figure 2-1: single configured DL and UL BWP pair with unaligned center frequencies in [6]

CATT [12] provided a case illustrated below where there is no full DL symbol and a smaller DL BWP compared with UL BWP is configured. It is pointed out that shorter BWP switching delay needs to be considered and this case is configuration dependent and a larger DL BWP with aligned center frequency as UL BWP can be configured instead.

Figure 2-2: single configured DL and UL BWP pair with unaligned center frequencies in [12]

Intel [15], CMCC [21] and DOCOMO [28] do not see motivation to support single configured DL and UL BWP pair with unaligned center frequencies.

Samsung [25] think that unaligned center frequencies for the BWP pair would result in higher RF modem design cost for single BWP UE implementations, additional specification effort, additional switching delay requirements and RF configuration constraints and thus does not support.

Qualcomm [30] see useful scenario to support a single configured DL and UL BWP pair with unaligned center frequencies where network configures the UE with narrowband initial UL/DL BWP and then wideband first active BWP after RRC connection. If SBFD is indicated to only RRC connected UE, then during initial access, all UEs will be configured with center aligned narrowband UL/DL BWP as shown on left figure in the example below. This will restrict leveraging the UL-SB resources and hence don’t achieve UL coverage gain and latency reduction. On the other hand, if this restriction is lifted, two narrowband UL/DL BWP could be configured for the new UEs as shown in the right figure in the example below. This can be applicable for default BWP as well where UE could be scheduled only in one of the two DL subbands.

Figure 2-3: Narrowband UL/DL BWP with aligned and non-aligned center frequency [30]

Given the divergent views, more discussions are needed and no proposal is provided.

More than one configured DL and UL BWP pair with aligned/unaligned center frequencies

Example of multiple configured DL and UL BWP pairs with aligned center frequencies is illustrated below, where gNB configures BWP 1, 2 and 3 for UE and gNB may also configure BWP 4 with full bandwidth with the restriction that BWP 4 can be used only in 1st DL slot or last UL slot [15].

Figure 2-4: Multiple configured DL and UL BWP pairs with aligned center frequencies [15]

Given that there is either DL or UL within a BWP in a symbol, no new UE behaviour needs to be introduced to resolve UL/DL collision. In addition, separate configuration for different BWPs can be supported.

SBFD aware UE is required to support more than one BWP pair and there are potential issues due to frequent BWP switching and the interruption caused by BWP switching delay. Intel [15] discussed the impact on SPS PDSCH/Type-2 CG PUSCH, HARQ-ACK dropping and repetition due to BWP switching. To reduce BWP switching delay, multiple active BWPs or shorter BWP switching delay needs to be considered as discussed by many companies, which would increase the UE implementation complexity.

Ericsson [18] proposed to not support SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair considering the fundamental change to current specifications and negative impact on UE complexity.

Qulacomm [30] raised concerns on SBFD operation using more than one configured DL and UL BWP pair with aligned center frequency where each subband configured as BWP. Instead, it is proposed that there is one UL/DL BWP pair configured for TDD operation and one UL/DL BWP pair configured for SBFD operation and UE would need to switch from one BWP to the other based on some semi-static configuration of the BWP switching. UE knowledge of the semi-static BWP switch may result into optimized BWP switching delay. It is noted that the UL/DL BWP pair configured for SBFD operation is with unaligned center frequencies and DL BWP can be defined as non-contiguous RBs to cover both DL subbands.

Figure 2-5: Multiple configured DL and UL BWP pairs with unaligned center frequencies [30]

Intel [15] analyzed if both DL BWP and UL BWP in SBFD symbols shown above are within the same UE channel BW, the retune is not needed thus no interruption time between a SBFD symbol using DL BWP and next SBFD symbol using UL BWP; otherwise, similar retune time as existing BWP switch is needed.

Samsung [25] think that unaligned center frequencies for the BWP pair would result in higher RF modem design cost for single BWP UE implementations, additional specification effort, additional switching delay requirements and RF configuration constraints and thus does not support.

KDDI [36] observed that smaller BWP can suppress the inter-UE CLI (compared to the BWP including all RBs) by help of lower sampling rate and therefore proposed to study the potential benefit of SBFD scheme with more than one configured DL and UL BWP pair to utilize BWP adaptation based inter-UE CLI suppression.

Given the divergent views, more discussions are needed and no proposal is provided.

    - Subband location indication

      - Transparent vs. non-transparent SBFD operation

The following agreement was made in RAN1#110.

Agreement Study the following alternatives with Alt 4 prioritized, for SBFD operation at least for RRC_CONNECTED state. - SBFD operation Alt 1: - Time and frequency locations of subbands for SBFD operation are not known to UEs. - UE behaviors follow existing specifications without introducing new UE behaviors for SBFD operation at gNB side. - SBFD operation Alt 2: - Time and frequency locations of subbands for SBFD operation are not known to UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs - SBFD operation Alt 3: - Only time location of subbands for SBFD operation is known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time location of subbands for SBFD operation - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation. UE capability discussion is held in work item phase.

Among the four alternatives, ZTE [7] proposed to focus on Alt 4 and Alt 2 and postpone or deprioritize Alt 3. New H3C proposed to study Alt 1 and Alt 2 with lower priority and exclude Alt 3 for further study. Sony [16] proposed to further consider Alt 2 and do not further consider Alt 1 and Alt 3.

TCL [5], Ericsson [18], Xiaomi [20], Samsung [25], DOCOMO [28], Qualcomm [30] and ITRI [31] proposed to support Alt 4 only. Nokia [32] proposed that Rel-18 shall focus on Alt 4 as baseline.

Based on companies’ views, it is proposed to support SBFD operation Alt 4 for SBFD operation at least for RRC_CONNECTED state in Proposal 1-3.

      - Cell-common vs. UE-specific subband location indication

The following agreements were made in RAN1#110.

Agreement For indication of subband locations for SBFD operation, study semi-static configuration of subband time and frequency location as baseline. Agreement For semi-static configuration of subband location, consider same subband frequency resources across different SBFD symbols as baseline.

Qualcomm [30] observed that semi-static configuration of the UL/DL subbands is essential for the SBFD operation.

ZTE [7], CATT [12], CMCC [21] and MediaTek [26] think that the subband location is common to all SBFD aware UEs in a serving cell. CATT and CMCC think cell-common subband location indication should be supported. CATT, Panasonic [19] and LG [27] proposed to support subband location indication in SIB. MediaTek [26] proposed that SBFD layout in frequency and time is configured per carrier.

Ericsson [18] assumes that the subband location is indicated to SBFD aware UEs by extending TDD-DL-ULConfigDedicated, which UE-specific signaling.

Based on companies’ views, an initial proposal in Proposal 1-4 is provided to support semi-static configuration of subband time and frequency location and furthermore support semi-static configuration in SIB and FFS semi-static configuration in dedicated signalling.

      - Dynamic subband location indication

It was agreed to study semi-static configuration of subband time and frequency location as baseline. Companies discussed dynamic indication of time and frequency location of UL subband for efficient resource utilization and DL/UL traffic adaptation and the views are summarized below.

Dynamic indication of size and/or frequency location of UL subband

- Supported by: TCL, New H3C, Sony, Lenovo

- Not supported by: ZTE, vivo?, Ericsson, Intel, MediaTek, DOCOMO, Nokia

  - frequency reconfiguration of analog filter if analog filter is needed [7][18][21]

  - Radio resource can be partitioned in time domain instead [7][18]

  - Many UL transmissions have strong dependence on knowing the UL subband size and it is unrealistic to interrupt UE preparation of UL signals when a new subband configuration is dynamically signalled [18]

  - Complexity of CLI handling [15]

- FFS: Qualcomm

Dynamic indication of time location of UL subband

- Supported by: TCL, New H3C, ZTE, vivo, Intel, Sony, DOCOMO, Qualcomm, Nokia, WILUS

- Not supported by: MediaTek?

It is observed that many companies have concerns on dynamic indication of size and/or frequency location of UL subband and many companies support dynamic indication of time location of UL subband. Accordingly, an initial proposal in Proposal 1-5 is provided.

      - Subband time location indication

Based on companies’ inputs, there are generally two approaches for subband time location indication.

- Subband time locations are implicitly determined based on TDD UL-DL configuration.

- Subband time locations are explicitly indicated.

For the implicit approach, examples could be that semi-static DL/flexible symbols within a period determined by periodicity of the configured TDD pattern, e.g. DDDSU, are configured as SBFD symbols as discussed in [10], or symbols configured as ‘D’ in TDD-UL-DL-ConfigCommon are SBFD symbols as discussed in [18]. Qualcomm [30] proposed that not all DL slots are used for SBFD operation considering that legacy DL slot is important to protect DL reception of UEs that suffer from strong CLI especially when receiving common signalling and UEs that don’t support Rel-16 CLI framework.

For the explicit approach, depending on whether SBFD symbols are always contiguous in time, different methods can be considered for time location indication. For example, if SBFD symbols are contiguous in time, SBFD symbols can be indicated by a starting symbol and duration. Otherwise, gNB can indicate whether each slot/symbol within a duration is SBFD slot/symbol.

ZTE [7] discussed a method to explicitly configure a period, offset and length for a time domain window for SBFD operation as shown below.

Figure 2-6: Time domain window for full duplex [7]

Vivo [10] discussed a method that gNB can configure the periodicity and duration of the configured one or more subbands for potential UL transmission.

OPPO [11] proposed that UL subband can be explicitly configured in DL symbol and flexible symbol.

CATT [12] thinks the SBFD symbols within a periodicity are explicitly indicated and the periodicity can be explicitly configured or implicitly determined based on the periodicity of SSB or TDD UL-DL configuration.

Panasonic [19] proposed to consider introducing a new semi-static slot format where the legacy semi-static DL symbol/slot can be re-configured as UL or Flexible symbol/slot.

CMCC [21] assumes that gNB broadcasts legacy cell-specific TDD UL/DL configuration with no flexible slot in SIB1, e.g., DDDSU in the following figure. New cell-specific signaling is introduced to indicate SBFD slots/symbols which can override the fixed DL/UL slots/symbols in cell-specific TDD UL/DL configuration (Step 1). In addition, UE-specific RRC signalling/scheduling DCI/SFI can indicate the DL or UL link direction for SBFD slots/symbols (Step 2).

Figure 2-7: Indication of time location of subband [21]

MediaTek [26] provided examples for slot-level configurations for time-domain pattern in the following figure.

Figure 2-8: Relationship of TDD frame format and SBFD time-domain layout illustrated by rough slot-level configuration [26]

Based on companies’ inputs, an initial proposal in Proposal 1-6 is provided to support explicit indication of SBFD subband time locations.

      - Subband frequency location indication

In RAN#96 meeting, it was concluded that SBFD operation in UL symbol is second priority so it is not considered in the current discussion and can be discussed in future.

In RAN1#110 meeting, it was agreed as a working assumption that for SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline.

For subband frequency location indication, there are two options below.

- Option 1: the RB locations of each UL/DL subband are explicitly indicated

- Option 2: only the RB location of UL subband is explicitly indicated

Figure 2-9: Options for RB set and guard band configuration [18]

As shown in the above figure, for Option 1, a starting RB and number of RBs for each of the three UL/DL subbands are indicated. With this approach, guardband can be implicitly determined without additional signaling. Alternatively, the first and the last PRBs of each UL/DL subband can be indicated or PRBs of a UL/DL subband can be indicated based on a bitmap indication [14].

For Option 2, only the RB location of UL subband is explicitly indicated. Ericsson [18] discussed that the size of guardbands is also explicated indicated and the remaining RBs are implicitly determined as DL subband(s). Vivo [10] discussed that the frequency resources not covered by the configured UL subband within the TDD carrier can be regarded as belonging to a single subband, or each contiguous part of them can be regarded as a subband. The transmission direction for the other frequency resources can be determined based on where they locate, i.e. in semi-static DL symbols or semi-static flexible symbols, or simply regarded as DL. To be more specific, flexible subband(s) may exist in semi-static flexible symbol(s) and transmission direction of each flexible subband in semi-static flexible symbol(s) can be determined based on other existing dynamic signaling.

Based on companies’ inputs, an initial proposal in Proposal 1-7 is provided for SBFD subband frequency location indication.

Furthermore, the reference of the starting RB was discussed in [12] and it is proposed that frequency location of UL subband is with reference to a common RB, e.g. CRB 0 or RB indicated by offsetToCarrier, if supported in SIB.

Figure 2-10: UL subband indication with reference to a carrier [12]

MediaTek [26] proposed that frequency location of subbands is specified with respect to CRB#0.

The maximum number of subbands supported within a carrier from gNB’s perspective was discussed in RAN1#109-e without consensus. Many companies think that the number of subbands depends on inter-operation co-existence and proposed to support up to three subbands within a carrier [5][6][9][10][15][18][21][28][30]. Nokia [32] proposed to study U-D and D-U type of subband configurations as baseline and FFS support of D-U-D type of subband configuration (pending pros and cons analysis also based on feedback from RAN4).

An initial proposal in Proposal 1-8 is provided accordingly.

      - Link direction of UL/DL subband

The following agreement was made in RAN1#110.

Agreement For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, study the following options: - Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol - Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol - Option 3: The SBFD aware UE does not expect to be scheduled with DL reception within the UL subband and may be scheduled with UL transmission outside the UL subband in the SBFD symbol - Option 4: The SBFD aware UE may be scheduled with UL transmission outside the UL subband or DL reception within the UL subband in the SBFD symbol

Companies’ views on the four options are summarized below:

- Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol

  - Supported by: Huawei, ZTE, New H3C, Spreadtrum, CATT, Sony, Fujitsu, Samsung, Panasonic (if dynamic indication of subband is supported), Xiaomi, DOCOMO, Qualcomm, MediaTek (with amendments), Nokia, ITRI, WILUS

  - Not supported by:

- Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol

  - Supported by: ZTE, CMCC, Fujitsu, Samsung, Pansonic (if dynamic indication of subband is not supported), Xiaomi, Nokia, New H3C, CEWiT, NEC

  - Not supported by:

- Option 3: The SBFD aware UE does not expect to be scheduled with DL reception within the UL subband and may be scheduled with UL transmission outside the UL subband in the SBFD symbol

  - Supported by: Samsung (for SBFD operation in U/F slots), WILUS

  - Not supported by: Ericsson, CMCC, New H3C, Apple

- Option 4: The SBFD aware UE may be scheduled with UL transmission outside the UL subband or DL reception within the UL subband in the SBFD symbol

  - Supported by: MediaTek

  - Not supported by: Ericsson, CMCC, Apple

Based on companies’ inputs, the intention of Option 2/3/4 includes scheduling flexibility and CLI measurement in UL subband for Option 2/4.

Huawei [6], Panasonic [19], DOCOMO [28], Qualcomm [30], Nokia [32] and ASUSTek [33] think that the purpose of scheduling flexibility for Option 2/3/4 can be achived by dynamic update of time location of UL subband.

Sony’s observation is that Option 2, Option 3 and Option 4 where UL transmissions can occur outside the UL subband and/or DL transmission can occur within the UL subband, defeat the purpose of having the UE aware of SBFD locations. If flexibility is required, consider Alt 2 SBFD operation, where the UE does not need to be aware of SBFD locations and instead uses an “Overwrite” indicator to overwrite semi-static configured UL & DL symbols [16].

Vivo [10] thinks that for the SBFD aware UE, if DL reception is scheduled within the frequency boundary of the configured UL subband in semi-static DL symbol, it means that SBFD operation in the symbol is disabled and only DL reception(s) can be performed in the symbol. For SBFD symbol in semi-static flexible symbols, if DL reception is scheduled within the frequency boundary of the configured UL subband or if UL transmission is scheduled outside the frequency boundary of the configured UL subband, it means that SBFD operation in the symbol is disabled and the semi-static flexible symbol becomes a full DL symbol or a full UL symbol.

For Option 2, ZTE [7] would like to configure a subset of subband area and the DL transmission is only allowed in this specific area within the UL subband to mitigate the potential CLI. In addition, ZTE proposed to analyze gNB behavior as well. For example, to assist the gNB-gNB CLI measurement, the aggressor gNB may also transmit reference signal in the UL subband. Then the victim gNB can receive the reference in the UL subband and identify the potential aggressor gNBs.

CATT [12] , Intel [15] and ASUSTek [33] suggest discussing dynamic scheduled transmissions and configured transmission separately.

Samsung [25] proposed to separate discuss for SBFD in D/F slots and SBFD in U/F slots. For former case, Option 2 in addition to Option 1 is supported to avoid semi-static UL/DL resource partitioning. For the latter case, Option 3 is supported to allow UL transmission within configured DL subband. From moderator’s understanding, the agreement and the discussion do not consider SBFD operation in legacy UL symbols, which is deprioritized in RAN#96.

ZTE [7], CMCC [21] and Qualcomm [30] considered CLI measurement within UL subband for inter-UE inter-subband CLI handling. Qualcomm proposed that Option 1 doesn’t prevent UE to measure the CLI in the UL subband since CLI measurement is not DL reception from gNB and this shouldn’t be confused with DL scheduling in UL subband.

Serveral companies expressed concerns on allowing UL transmission outside UL subband (i.e. Option 3/4) [18][21][24]. The issue is similar as dynamic change of size and/or frequency location of UL subband.

MediaTek [26] can support Option 1 with some exceptions:

- Inter-subband UE-UE CLI measurement are not confined within the DL subband.

- UE assumes that the symbol is not partitionned if symbol is indicated to be ‘flexible’ by group common SFI signalling of UE TDD link direction.

- DL reception scheduled by DCI is allowed to overlap with subband in a symbol indicated as flexible by semi-static SBFD layout configuration.

Moderator agrees with the view that the discussion is related to whether dynamic subband location indication is supported or not since the intention for flexible scheduling can be achieved by dynamically changing subband locations.

Given that it is proposed to FFS dynamic indication of time location of SBFD subband in Proposal 1-5 and the concerns from options on supporting UL transmission outside UL subband, proposal in Proposal 1-9 is providedto exclude Option 3 and Option 4. Potential down-selection between Option 1 and Option 2 can be postponed after concluding whether dynamic indication of time location of SBFD subband is supported or not.

    - Resource allocation

The following agreement was made in RAN1#109-e.

Agreement Study the impact/potential enhancements of resource allocation in symbols with subbands that gNB would use for SBFD operation.

Companies’ inputs on impact/potential enhancements of resource allocation in SBFD symbols are summarized on a per physical channel/signal basis in the following sub-sections.

      - PDCCH

CORESET FDRA is based on a bitmap of RB groups consisting of 6 RBs. Lenovo [17] proposed to study enhanced CORESET frequency resource allocation considering that some groups of 6RBs may not be usable for CORESET as shown in the following figure.

Figure 2-11: Full duplex UL subband allocation and a set of 6RB groups for CORESET resource allocation [17]

Intel [15] proposed to study solutions for PDCCH in SBFD and non-SBFD symbol.

MediaTek [26] observed that configuring PDCCH monitoring over different CORESETs for SBFD and DL-only slots/symbols may require configuring separate search spaces per each slot in the periodic SBFD partition pattern, each with the same slot periodicity but different slot offsets, soon using up the maximum number of search spaces that can be configured and proposed to study enhancements to Search Space configuration which allow adaptation to SBFD partitioning.

Similarly, Nokia [32] proposed to study solutions enabling dynamic adaptation of PDCCH resources based on the occurrence of SBFD symbols and the corresponding DL-UL frequency domain partitioning.

      - PDSCH

With {DUD} subband frequency pattern, the available DL resources are partitioned into two DL subbands.

CATT [12] brought up that the existing configuration of rate matching resource cannot be used to resolve the issues since a UE is not expected to handle the case where PDSCH DMRS REs are overlapping with rate matching resources according to current specification.

RA type 0 can be used for allocating non-contiguous RBGs across DL subbands. Sony [16] observed that since RBG is the unit of allocation, FDRA Type 0 has a coarser frequency granularity compared to FDRA Type 1 and if the finer RBG granularity (i.e. RBG Configuration#1) is used, then FDRA Type 1 consumes more DCI bits compared to FDRA Type 0. In addition, FDRA Type 0 is not supported in Fallback DCI (Format 1_0).

For RA type 1 and non-interleaved VRB-to-PRB mapping, only contiguous PRBs can be allocated so that non-contiguous FDRA across DL subbands cannot be achieved. For RA type 1 and interleaved VRB-to-PRB mapping, scheduling restriction/difficulty are observed to avoid overlapping with UL subband.

There can be fractional RBGs at DL subband boundaries which cannot be allocated for PDSCH.

Figure 2-12: Partial RBG at boundary of the DL subband [30]

Samsung [25] discussed that for n78 and 100 MHz carrier BW and a UE DL BWP of 100 MHz, worst case, the nominal RBG size is always 16. DL scheduling to UEs in the 2 DL subbands of the SBFD slot then may suffer granularity issues. When the UL subband occupies center 51 RBs, 4*16=64 RBs of 273 RBs cannot be DL scheduled to the UE because an RBG would be in partial overlap with the UL subband. The effective loss may be reduced when considering that part of these partial overlap RBGs provide an implicit guard band configuration.

Enhancements including enhanced VRB-to-PRB mapping [17][21][32], rate matching around UL subbands [12][17] [25][30][32], puncturing the PRB/symbols within the UL subband [32], mirror image FDRA [16], interleaved mapping of odd and even VRBs over physical RBGs allocated by Type-0 FDRA [26], similar handling as fractional RBGs at BWP edges [26] and a 3rd more optimal set of RBG set sizes [25] were proposed.

Issues of FDRA of PDSCH due to different available resources in SBFD symbols and non-SBFD symbols are identified.

CATT [12] discussed the issue of PDSCH slot aggregation across SBFD slot and non-SBFD slot where the PDSCH resources in SBFD slot overlaps with UL subband as shown below.

Figure 2-13: Restriction for slot aggregation [12]

For the above issue, OPPO proposed three potential schemes, which applies to PUCCH/PUSCH repetition as well.

- Scheme 1: A PUCCH/PUSCH/PDSCH can be repeated within non-SBFD slots or within SBFD slots, but not between a SBFD slot and a non-SBFD slot. For this scheme, current transmission scheme can be reused directly.

- Scheme 2: A PUCCH/PUSCH/PDSCH can be repeated between SBFD slot and non-SBFD slot, and the same RB resources are used in both types of slots. In other words, the valid RB resource for the first PUCCH/PUSCH/PDSCH repetition is copied in subsequent slots.

- Scheme 3: A PUCCH/PUSCH/PDSCH can be repeated between SBFD slot and non-SBFD slot, and the different RB resources can be used in different types of slots. In other words, the valid RB resources for PUCCH/PUSCH/PDSCH repetition are determined separately in SBFD slots and non-SBFD slots.

CMCC has similar proposals for PDSCH slot aggregation with the following two options.

- Opt 1: PDSCH slot aggregation is only across SBFD slots or non-SBFD slots;

- Opt 2: PDSCH slot aggregation can be across SBFD slots and non-SBFD slots with some automatic adjustment on FDRA between SBFD slots and non-SBFD slots.

Figure 2-14: Potential PDSCH slot aggregation methods in SBFD [21]

DOCOMO [28] discussed similar issue in case of multiple PDSCHs scheduled by single DCI as shown below. One possible enhancement is to introduce an additional FDRA field in the DCI scheduling multiple PDSCHs/PUSCHs. Another possibility is to interpretate the FDRA field differently on SBFD symbol and non-SBFD. It is also possible that the PDSCH/PUSCH overlapping with UL/DL subabnd is simply dropped. The same applies to PUSCH.

Figure 2-15: Example of single DCI scheduling multiple PDSCHs [28]

Nokia discussed the similar issue for SPS PDSCH and proposed to study solutions to configure the UE with SBFD-specific SPS-PDSCH resources to be used during SBFD slots/symbols and it applies to CG PUSCH as well.

Besides, OPPO discussed TBS determination of PDSCH and proposed that the amount of RBs used to determine the TB size does not count the RBs overlapping with UL subband and the guard bands (if any) around the UL subband.

      - CSI-RS

In current specifications, CSI-RS is limited to only contiguous FDRA and the CSI-RS frequency occupancy must be configured in integer multiple of 4 RBs and has size of minimum DL BWP or 24 RBs. With {DUD} subband frequency pattern, different options can be considered for CSI-RS configuration as summarized below as per moderator’s understanding from companies’ inputs.

- Option 1: Two CSI-RS resources are configured

  - Option 1-1: Two CSI-RS resources link to two CSI reports [28][32]

  - Option 1-2: Two CSI-RS resources link to one CSI report [18][22][28][30][32]

- Option 2: Non-contiguous CSI-RS resource [12][17][18][28][30] [32]

- Option 3: One contiguous CSI-RS resource overlapping with UL subband [25]

Figure 2-16: CSI-RS resource/CSI report options

For Option 1, two CSI-RS resources each within a DL subband are configured. The two CSI-RS resources can link to two CSI reports (Option 1-1) or one CSI report (Option 1-2). DOCOMO discussed potential support of narrow band RB allocation for CSI-RS (e.g. smaller value than 24 for number of RBs).

Option 2 is to define a non-contiguous CSI-RS by excluding the frequency resources of the UL subband and guardband (if configured) [18][30] or to treat two contiguous CSI-RS resources as one effective CSI-RS resource, e.g. by defining a separate ID to refer to the effective CSI-RS resource which is associated with the two legacy IDs [18].

Option 3 allows CSI-RS resource to be overlapped with UL subband and guardband and CSI report configuration linked to the CSI-RS resource set can blank out the UL SB according to current specification.

In addition, Qualcomm [30] see potential impact on CSI processing latency and propose to study impact on UE processing latency.

CATT [12] and Samsung [25] discussed the issue that the subband boundary for CSI reporting may not be aligned with DL subband boundary. The CSI report for the CSI subband at the edge of DL subband(s) may be not accurate.

Samsung [25] discussed potential different CSI-RS Tx EPRE settings by the gNB in the two DL subbands of the SBFD slot.

      - PUCCH

Considering the different available resources in SBFD slot and non-SBFD slot for UL transmissions and potential different link quality in SBFD slot and non-SBFD slot, separate PUCCH configurations for SBFD and non-SBFD slots are discussed/proposed by ZTE [7], vivo [10], CMCC [21], ETRI [22], MediaTek [26], Qualcomm [30] and WILUS [35]. To be more specific, vivo proposed to consider separate antenna configurations, power control, planned frequency resources etc. and think the solution can be extended to other physical channels or signals, e.g. PDCCH, SPS PDSCH and CSI-RS in DL, and CG PUSCH, SRS, PRACH in UL. CMCC proposed to separately configure PUCCH frequency hopping offsets, PUCCH resource sets. In addition, separate configuration of PUSCH FH offsets and SRS resources for SBFD symbols and non-SBFD symbols are considered for PUSCH and SRS respectively. MediaTek proposed to support two frequency domain resource allocations for periodic and semi-persistent PUCCH transmission in SBFD slot and non-SBFD slot.

InterDigital [14] proposed to Study the feasibility and potential benefits of providing supplementary configurations to the UE to be used in case PUCCH/PUSCH frequency resources are mapped to outside of the SBFD UL boundaries.

For PUCCH repetition, similar as the issue discussed for PDSCH slot aggregation, MediaTek proposed to support two frequency domain resource allocations for PUCCH repetition based on slot type and UE can skip repetitions that overlaps with the DL subband in SBFD slots. In addition, frequency hopping can be enabled for PUCCH repetition. MediaTek proposed to support two frequency domain resource allocations for intra-slot and inter-slot frequency hopping for PUCCH transmission in SBFD and non-SBFD slot. In addition, a mechanism to disable frequency hopping in SBFD slots could help in allowing flexible frequency hopping in the UL-only slots. Ericsson proposed to study enhancements to frequency domain resource allocation and frequency hopping mechanisms for PUSCH and PUCCH configured with repetition in order to allow repetitions to occur in both SBFD and UL-only slots. Qualcomm think the issue can be resolved by adjusting the location of the hops that occur within the UL subband. Samsung [25] proposed that PUCCH repetitions can be configured to only use the SBFD slots/symbols and the same applies to multi-slot PUSCH transmissions. Sony [16] proposed that UE performs frequency hopping in SBFD slots if the bandwidth size of the UL subband is larger than a configured threshold and if there is resource available for the second hop, otherwise the UE disables frequency hopping in the SBFD slot. Sharp [29] think no frequency hopping enhancement is necessary for UE-specific PUCCH/PUSCH.

      - PUSCH

For the similar reason as for PUCCH, separate PUSCH configurations for SBFD and non-SBFD slots are discussed/proposed by ZTE [7], vivo [10], CMCC [21], Samsung [25], MediaTek [26], WILUS [35]. To be more specific, CMCC, Samsung and WILUS considered separate configuration for PUSCH frequency hopping. MediaTek considered separate FDRA for CG-PUSCH.

For PUSCH repetition, similar issue as for PDSCH exists. Samsung considered one possibility to restrict multi-slot PUSCH transmissions to only use the SBFD slots but not the normal UL slot in time-domain. MediaTek considered possible approach to support two FDRA for DG-PUSCH repetition, or to allow the repetitions to be performed on specific set of slots (e.g., UL-only slots) and skipped on other set of slots (e.g., SBFD slots). In addition, the UE can skip repetitions that overlap with the DL subband in SBFD slots.

For CG PUSCH, similar issue as for SPS PDSCH exists. MediaTek, Nokia proposed to support two FDRA for CG-PUSCH transmission based on slot type and support skipping of periodic transmissions (e.g., CG-PUSCH) in SBFD slots. Panasonic think it is preferable to decouple the configured UL transmission from the configuration/operation of SBFD symbol/slot. For example, the configured UL transmission can be associated with an applicable symbol type (e.g. SBFD symbol vs. normal UL symbol), such that UE will skip the UL transmission instance if it is not overlapping with the applicable symbol type.

For PUSCH frequency hopping, similar issue as for PUCCH exists and similar enhancements are considered.

In addition, Qualcomm proposed to further study the impact/potential of enhancement of available slot counting in SBFD.

      - SRS

Separate SRS configurations for SBFD and non-SBFD slots are discussed/proposed by ZTE [7], CATT [12], CMCC [21].

MediaTek [26] considered SRS resource colliding with DL subband in {UDU} pattern and observed that for periodic and semi-persistent SRS transmission, a single frequency domain resource allocation is configured for all UL slots. This may impact flexibility in resource allocation in SBFD since it has two types of slots with different bandwidths and for SRS transmission in SBFD, the resource allocation with and without frequency hopping may not be confined within the UL subband of a SBFD partitioned slot. Corresponding enhancements were proposed.

    - UE collision handling

For the Rel-18 duplex operation, SBFD operation will be supported at the gNB side, while UE only supports half duplex operation. Collision between DL signal/channel and UL signal/channel can occur for the baseline operation within a TDD carrier, i.e. SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies.

For discussion purpose, the following definitions are provided.

- Definitions:

  - Semi D: symbols configured as DL in tdd-UL-DL-ConfigurationCommon and/or tdd-UL-DL-ConfigurationDedicated

  - Semi U: symbols configured as UL in tdd-UL-DL-ConfigurationCommon and/or tdd-UL-DL-ConfigurationDedicated

  - UL: dynamic and semi-static PUCCH/PUSCH/SRS transmissions

  - DL: dynamic and semi-static PDCCH/PDSCH/CSI-RS receptions

Based on companies’ contributions, the following collision cases were discussed.

- Case 1: UL vs. DL

- Case 2: Semi D vs. UL

- Case 3: SSB vs. UL

- Case 4: Type 0 CSS vs. UL

- Case 5: Semi D vs. valid RO

- Case 6: valid RO vs. DL

- Case 7: Semi U vs. DL

- Case 8: Semi U vs. SSB

- Case 9: Semi U vs. CORESET 0

Among the cases, Case 7/8/9 target for SBFD operation in legacy UL symbols and should be de-prioritized according to RAN decision. Case 5 and 6 are related the initial access in UL subband discussed in 2.1.6. and can be discussed later.

Case 1: UL vs. DL

The collision of UL/DL transmission and transmission direction of subband is discussed in 2.1.2.6., e.g. UL transmission overlapping with DL subband, DL transmission overlapping with UL subband. Here we focus on the collision of one UL transmission and another DL reception overlapping in time from UE perspective.

CMCC [21], Qualcomm [30] discussed collision of higher layer configured UL transmission and DL reception and think it should be allowed that some higher layer configured UL channel/signals and DL channel/signals can be configured in the same SBFD symbols (e.g. as shown below), which is not allowed in the current specification. Some handling rules need to be defined.

Figure 2-17: Time-domain overlapping between higher layer configured UL transmission and DL reception [21]

Intel [15] discussed that if UE is configured/scheduled for UL or DL transmission in the symbol, a mechanism to determine DL reception or UL transmission in the symbol should be studied.

According, Proposal 1-13 is proposed to study Case 1.

Case 2: Semi D vs. UL

For legacy behavior, any UL signals or channels will be dropped in a symbol configured as semi D, which is clearly not appropriate for SBFD operation where UL subband can be configured in semi-static DL symbols. So at least in symbols configured as DL in TDD-UL-DL-ConfigCommon in which UL subband is configured, UL transmission in UL subband should be supported. But if the symbol is configured as DL in TDD-UL-DL-ConfigCommon and in tdd-UL-DL-ConfigurationDedicated, as shown in Figure   2 -7, whether UL transmission is allowed needs further discussion.

According, Proposal 1-15 is provided for Case 2.

Case 3: SSB vs. UL

For legacy behavior, any UL signals or channels will be dropped in an SSB symbol.

CMCC [21] and Qualcomm [30] discussed whether SBFD operation in SSB symbols is supported. CMCC think it is considered safer to not allow the SBFD symbols overlap with SSB symbols considering that UE-UE inter-subband CLI may impact the SSB measurement of IDLE/INACTIVE UEs and cannot be controlled by network since network does not know which UE will perform SSB measurement on these symbols. Qualcomm think it is important to allow SSBs to be configured in SBFD symbols.

If SBFD operation in SSB symbols is not supported, there should be no collision of UL transmissions and SSB. If SBFD operation in SSB symbols is supported, similar as semi D vs. UL, at least UL transmission in UL subband in SSB symbols in which UL subband is configured should be supported.

According, Proposal 1-16 is provided for Case 3.

Case 4: Type 0 CSS vs. UL

Different from SSB, UL transmission in symbols configured with Type 0 CSS is not prevented in the current specification. So it is not quite clear yet whether any enhancement is needed.

    - DL-UL switching

InterDigital [14] discussed that non-zero timing advance or switching time could result in dropping of respective slots, especially for back-to-back scheduling cases between DL and UL and proposed to consider a means for UE to detect if the allocated UL/DL timing alignment is enough.

Intel [15] discussed that for a SBFD aware UE, the gap for DL/UL switching is also needed between SBFD symbols, or between DL symbol and SBFD symbol. How to ensure such gap and the impact on back-to-back DL and UL signals/channels should be studied.

Xiaomi [20] proposed to further study how to configure or determine the guard period between DL region and UL subband

Figure 2-18: Examples of DL to UL switching point between DL and UL subband [20]

MediaTek [26] proposed per-subband, per-symbol indication of RB-symbol regions reserved for DL-UL switching as shown in the following figure.

Figure 2-19: Symbol-level SBFD layout configuration [26]

    - SBFD operation in RRC idle/inactive state

It was agreed to study SBFD operation at least for RRC_CONNECTED state.

Ericsson thinks that UL transmission during initial access should occur only within UL-only slots.

Fujitsu’s observation is that due to the CLI from PRACH transmission procedure on UL subband, the overall system performance will get decreased and thus proposed to wait for the discussion results of the PRACH coverage enhancement in the NR coverage enhancement agenda.

ZTE [7], CATT [12], Intel [15], Samsung [25], LG [27], Sharp [29], Qualcomm [30] and Nokia [32] support to study potential benenfits and enhancements for initial access in UL subband.

Based on companies’ inputs, an initial proposal in Proposal 1-17 is provided.

    - SBFD operation across carriers

It was agreed in RAN1#109-e to at least study SBFD operation within a TDD carrier.Several companies continued discussion of SBFD operation across carriers.

SBFD operation across carriers achieved by intra-band TDD CA with different TDD configurations are considered by Huawei [6], ZTE [7] and Intel [15]. Huawei proposed to study SBFD operation within a carrier and across carriers with equal priority. ZTE proposed to study half-duplex CA based scheme for sub-band full duplex and taking directional conflict handling mechanism in R16 half-duplex CA as a starting point. Intel pointed out retransmission restriction in the same carrier and think enhancement for cross-carrier transmission needs to be studied.

Figure 2-20: SBFD operation across carriers [6]

Qualcomm [30] discussed two options for SBFD operation across multiple component carriers as shown below. The first option is to utilize intra-band CA framework with different TDD DL/UL configurations across the component carriers. The other option is by fully reusing the same concept of SBFD within a component carrier across the component carriers. While it is observed that CA-based SBFD operation is interesting for e.g. FR 2-1, it is proposed that SBFD operation across multiple components is studied at later stage in Rel-18 after establishing the baseline study of SBFD operation within component carrier.

Figure 2-21: SBFD using multiple component carriers (intra-band CA)

  - [Closed] 1st round discussion

Proposal 1-1

Proposed Agreement:

Confirm the following working assumption in RAN1#110.

For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline.

- FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies

- FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair

|  | Company |
| --- | --- |
| Support | New H3C, TCL (VIVO version), ZTE, Sony, InterDigital, Samsung, Spreadtrum, MediaTek, Intel, Panasonic, Lenovo, ITRI, Ericsson (with modification), DOCOMO, Qualcomm, CMCC, CEWiT, Fujitsu, CATT, Sharp, vivo, Nokia, NSB, Xiaomi, WILUS, NEC |
| Not support |  |

| Company | Comments |
| --- | --- |
| TCL | In our view, the VIVO edited version of the working assumption clarifies that RB is the granularity of the DL and/or UL subbands. Thus we prefere the VIVO edited version of the working assumption. |
| Sony | Although we do not prefer the FSS for non-aligned BWPs, we can support this WA to move the SI forward. |
| Samsung | We can accept the FL proposal “as is”. We do have concerns about the UE modem impacts from unaligned center frequencies and do not believe this scenario is meaningful to study in Rel-18. It would be ok for us, to simply delete the FFS parts from the RAN1#110 WA, e.g., - FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies - FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair |
| Apple | The wording of this WA may lead different interpretations (also related to the next proposal). Is UL sub-band within the UL BWP? I assume yes. Are DL sub-bands within DL BWP, but not overlapping with UL BWP (and definitely not overlapping with UL subband)? Again, I assume yes, but these aspects need to be clarified |
| Spreadtrum | Although we do not see any benefit of unaligned center frequency for UL and DL BWP pair, we can live with them as FFS. |
| Ericsson | We share a similar concern about unaligned center frequencies as expressed in some of the comments above. Hence, we think to avoid large changes to the BWP framework in NR and impact to UE complexity that it is better to avoid further study of unaligned center frequencies. Our first preference would be to confirm the WA as edited by Samsung above. Second preference is to confirm the WA "as is," and then conclude on the unaligned center frequency issue in another proposal. |
| QC | Support to confirm working assumption from RAN1 #110. |
| CMCC | We also don’t see the motivation of first FFS. |
| Huawei, HiSilicon | Fine to confirm the working assumption. For the second FFS, which implies support of multiple active BWPs, we are a bit sceptical about the increased UE complexity and potential impact of BWP switching. |
| Sharp | We are OK to confirm the WA as is, but we prefer deleting the FFS sub-bullet as companies suggested. |
| Nokia, NSB | We agree with the working assumption that a baseline SBFD aware UE may only be capable of supporting single configured DL and UL BWP pair with aligned center frequencies. We are open to discuss the benefits of single configured DL and UL BWP pair with unaligned center frequencies, especially as discussed by Qualcomm. We are also open to discuss the benefits of supporting more than one configured/active DL and UL BWP pair (with or w/o aligned center frequencies), though in latter case, we think the advantages of such a scheme should be clearly quantified in order to justify the increased complexity both in the specifications and at the UE. |
| Xiaomi | We don’t support the FFS point but OK to confirm the working assumption first. |
| OPPO | We have some questions regarding to the proposal. Q1: Given “SBFD scheme within a single configured DL and UL BWP pair”, would the following figure (Figure 2-10 in this FL summary) be categorized as in main bullet or as in 2nd FFS bullet (assuming BWP0 belongs to one BWP pair and BWP1 belongs to another pair)? Q2: We suppose there is only one pair of active DL BWP and active UL BWP. But “active BWP” is not part of WA. Does it mean the SBFD scheme could occur in non-active DL BWP? Q3: Similar question as from Apple. Is UL subband operation considered to occur in UL BWP or in DL BWP? In addition, we agree with other companies that the “unaligned center frequency” should be removed from 1st and 2nd FFS bullets. |

Proposal 1-2

Proposed Agreement:

For SBFD operation within a TDD carrier, support SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony, Samsung, Spreadtrum, Intel, Panasonic, Ericsson (with wording change), DOCOMO, Qualcomm,CMCC, CEWiT, Fujitsu, Huawei, HiSilicon, CATT, Sharp, vivo, Nokia, NSB, ETRI, Xiaomi, WILUS |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | We understand the intention of this proposal and would be ok with the intention. However, the current wording may not clear about the following two aspects since “within” is used - Whether the UL subband has to confined within the UL BWP? From our perspective, it is reasonable to assume that UL subband has to be within UL BWP. - Whether the UL subband has to confined within the DL BWP, e.g., DL BWP = 10M, UL BWP=40M, UL subband=20M? From our perspective, it is not reasonable to assume that UL subband has to be within DL BWP. |
| InterDigital | We understand that the intention of this proposal is to at least support one solid SBFD scheme. Since the FFS points are still valid for further study, we suggest removing ‘single’ in the proposed sentence, not to be unnecessarily strong and restrictive and be the “baseline” at the moment: Revised Proposal: For SBFD operation within a TDD carrier, support SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline. |
| Apple | Please see comments to Proposal 1-1 |
| MediaTek | We don’t think RAN1 has done enough study to agree on supporting any scheme. |
| Intel | We support SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline, and we are open for further study of multiple BWP case |
| Lenovo | We agree to support SBFD scheme when a single pair of DL and UL BWPs with aligned center frequencies is configured. We suggest changing the wording as follows: “For SBFD operation within a TDD carrier, support SBFD scheme when a single pair of DL and UL BWPs with aligned center frequencies is configured”. |
| Ericsson | We support the intention of the proposal, but we have a general concern about this and several other proposals in this document. "Support" is only a word that should be used in agreements in a WI phase when agreeing to specify certain features. We are in an SI, hence "support" is not relevant. We think that rather than formulating proposals as "Support X …" better wording could be as follows (depending on the proposal): - It is agreed that X is the baseline … or - In further studies, it is agreed that X is the baseline … or - It is agreed that X is beneficial … Hence, suggest re-wording the proposal as follows: For SBFD operation within a TDD carrier, support it is agreed that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as is the baseline. |
| Nokia, NSB | See comments to Proposal 1-1. |
| ETRI | Fine with Ericsson’s version. |
| Xiaomi | One clarification question: is there any restriction on the UL subband configuration with this proposal? For example, UL subband can only be configured within the frequency range of UL BWP, or within the frequency range of DL BWP, or no restrictions within an activated carrier? |
| NEC | We agree with Ericsson that “support” is not required for the study item phase. And comparing Proposal-1-1 with this proposal, we do not see that we are advancing the discussion significantly. We would prefer to not discuss this proposal given that Proposal-1-1 is already being discussed. |
| OPPO | Same comments as for Proposal 1-1. |

Proposal 1-3

Proposed Agreement:

For SBFD operation at least for RRC_CONNECTED state, support SBFD operation Alt 4.

- SBFD operation Alt 4:

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony (conditional), InterDigital, Samsung, Spreadtrum, MediaTek, Intel, Panasonic, Lenovo, ITRI, Ericsson (with wording change), DOCOMO, Qualcomm,CMCC, CEWiT, Fujitsu, CATT, Sharp, vivo, Nokia, NSB, Xiaomi, WILUS, NEC |
| Not support |  |

| Company | Comments |
| --- | --- |
| Sony | We support Alt 4 only, if we support only Option 1 where the UE does not expect have DL transmission in UL subband, otherwise it defeats the whole purpose of making the UE aware of the subband locations. |
| Spreadtrum | We can add “at least ” support SBFD operation Alt 4, to leave some space for other alternatives. |
| Intel | We support Alt 4 for SBFD. Just to clarify, is it open for further study for other alternatives, or other alternatives are excluded for study? If it is still open, we tend to agree with Spreadtrum to add ‘at least’. |
| Ericsson | We support the intention of the proposal; however, we have the same general comment about the use of the word "support" as for Proposal 1-2. Hence suggest the following For SBFD operation at least for RRC_CONNECTED state, support it is agreed that SBFD operation Alt 4 is the baseline |
| Huawei, HiSilicon | We have similar question as Spreadtrum and Intel. Considering that Alt 1 cannot be precluded anyway, it is not clear this proposal really implies. |
| Nokia, NSB | We agree to de-prioritize Alt 1-3 for operation with SBFD aware UEs. |
| OPPO | Similar comment as from Ericsson. We understand this wording “support” does not give the same meaning as people generally use in RAN1 to describe inclusion of a feature in spec from UE behavior perspective, because we were told by FL in earlier meeting all Alt 1~4 are from gNB perspective. It is better to avoid using “support” here. We also think Alt-1 cannot be excluded anyway. |

Proposal 1-4

Proposed Agreement:

For indication of subband locations for SBFD operation, support semi-static configuration of subband time and frequency location as baseline.

- Semi-static configuration of subband time and frequency location in SIB is supported

- FFS semi-static configuration of subband time and frequency location in dedicated signaling

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony, Spreadtrum (main bullet), MediaTek, Intel, Panasonic, Lenovo, Ericsson (main bullet only) , Qualcomm, CMCC (with comment), CEWiT, Fujitsu (Main bullet), CATT, Sharp, Nokia, NSB, ETRI (main bullet, only), WILUS |
| Not support | InterDigital, Samsung (acceptable without sub-bullets or using modified wording), vivo, Xiaomi, NEC, OPPO (concern on SIB signaling in 1st subbullet) |

| Company | Comments |
| --- | --- |
| InterDigital | We have concerns on the first subbullet, where the baseline should be RRC configuration, not by SIB (and rather the option of SIB should be FFS if needed to be captured). And, we prefer having at least time-domain dymaic fallback functionality, otherwise SBFD operation is too restrictive in terms of resource utilization flexibility. So, we suggest revising the proposal: Revised Proposal: For indication of subband locations for SBFD operation, support semi-static configuration of subband time and frequency location as baseline. - Semi-static configuration of subband time and frequency location in SIBdedicated signaling by RRC is supported - [At least time-domain dynamic switching (fallback) to non-SBFD operation is supported] - FFS semi-static configuration of subband time and frequency location in SIBdedicated signaling |
| Samsung | We support the main bullet: “For indication of subband locations for SBFD operation, support semi-static configuration of subband time and frequency location as baseline.” We do not support the 2 sub-bullets. SBFD operation in RRC_CONNECTED is baseline. It is a Stage 3 design decision for RAN2 during the WI stage if the use of cell common and/or UE-dedicated signaling is best suited. Based on Rel-15 RRC principles, the use of both cell-common and UE-dedicated configurations may be later decided by RAN2. An example are the BWP common/dedicated, PDSCH or PUSCH common/dedicated configurations. Our preference is to either not to include any of the 2 sub-bullets at all or use the following modified wording to replace both sub-bullets. - FFS: signaling details to support semi-static configuration of subband time and frequency location using dedicated and/or common RRC signaling incl. SIB if supported |
| Spreadtrum | We support the main bullet. For the first and second sub-bullet, they can be both FFS. According to SIB or dedicated signalling indicates subband location, we are open to discuss. But the relationship beween the subband time location and current time domain singnalling should be discussed first. Similarly, the relationship of frequency location of subband and BWP/Carrier can be given before the signalling method. |
| Intel | We agree that cell-specific semi-static subband configuration should be the baseline, while we share similar view with Samsung that whether such cell-specific information is carried by SIB or UE specific RRC (but the information is same for all UEs) is WI aspect. |
| Ericsson | We support the intention of the main bullet We think the exact signaling details (SIB1, dedicated signaling, etc.) for SBFD aware UEs is better suited to WI phase, hence we think the two sub-bullets should be removed. At most the SI could identify several signaling options to be considered in a WI (to give some direction), but actual support of a particular option can't really be agreed in an SI. For the main bullet, we propose the following modification: For indication of subband locations for SBFD operation for SBFD aware UEs, support it is agreed that semi-static configuration of subband time and frequency location as is the baseline. |
| DOCOMO | Share same view as Samsung and Intel. We support cell specific configuration can be baseline, but the detailed signaling can be by SIB or UE specific RRC. |
| QC | We support broadcast of the subband configuration in SIB which is beneficial for initial access for SBFD-aware UE. |
| CMCC | From our understanding, the “subband time locations” in this proposal means a global configuration to indicate UE which slots/symbols can be used as SFBD operation, but whether the link diection in each SBFD slot/symbol is D or U can be semi-statically or dynamically indicated to UE using UE-specific RRC signalling/scheduling DCI/SFI... We suggest to add the following FFS: For indication of subband locations for SBFD operation, support semi-static configuration of subband time and frequency location as baseline. - Semi-static configuration of subband time and frequency location in SIB is supported - FFS semi-static configuration of subband time and frequency location in dedicated signaling FFS how to configure/indicate the link direction of SBFD symbols |
| Fujitsu | We also share similar opinions as other company’s views. We prefer that all sub bullets are removed or FFS. |
| Huawei, HiSilicon | We would like to have some clarification about the relation between proposal 1-4 and 1-5&1-6. Does proposal 1-4 include both implicit and explicit configuration of subband? |
| vivo | We agree that the subband location is common to all SBFD aware UEs in a serving cell, and we think all companies agree that SBFD operation applied for RRC-connected UEs. However, people have different views on whether to support SBFD operation in RRC idle/inactive state. The proposal should be modified as For indication of subband locations for SBFD operation, support semi-static configuration of subband time and frequency location as baseline. - Semi-static configuration of subband time and frequency location in dedicated signalin is supported - FFS Semi-static configuration of subband time and frequency location in SIB is supported - FFS semi-static configuration of subband time and frequency location in dedicated signaling |
| Nokia, NSB | We think that indication of the semi-static SBFD configuration in SIB is also useful to enable/optimize initial access procedure in SBFD slots. It should also be noted that support of semi-static configuration of TDD configuration using dedicated signalling is an optional feature in Re-15/Rel-17 NR specifications (FG 5-1a). |
| ETRI | We support the main bullet. |
| Xiaomi | We are supportive to a common UL subband configuration among UEs. However, we don’t support to indicate UL subband configuration via SIB. There are many issues needs to be discussed/resolved if SIB is used to configure subband, for example: - What is the impact on the BWP configuration if subband is configured via SIB? Does this means all the UE dedicated BWP pairs have to cantain the subband? - What if UE-dedicated subband is also supported? - It is typical that not all the SBFD aware UE needs a UL subband on DL symbols. Whether a UL subband is needed and what a UL subband configuration should be configured depends on the traffic load. |
| NEC | We agree to support semi-static configuration of subband time and frequency location as baseline, but we think that further discussion is required on whether configuration should be provided in SIB or dedicated signaling. From our perspective, whether this information should be provided in SIB or not is dependent on whether we agree to support idle mode procedures for SBFD. Hence we should wait for the message type discussion until idle mode procedures applicability is clear. |
| OPPO | We are fine with the main bullet only (with Ericsson’s wording change to replace “support”), and have concern on the 1st sub-bullet for using SIB signaling. Using SIB signaling likely means the SBFD symbol would be defined per TDD-Common signaling in SIB, instead of TDD-Dedicated in RRC, which further raised a question what should happen if UL subband is defined in a flexible symbol in TDD-common but later changed to UL symbol in TDD dedicated. Another concern for using SIB signaling is that the SBFD operation, if not transparent to UE, may require UE’s cooperation before UE is able to report any UE capability, which means the related UE behavior becomes mandatory. |

Proposal 1-5

Proposed Agreement:

For indication of subband locations for SBFD operation,

- Dynamic indication of frequency location and/or size of SBFD subband is not supported

- FFS dynamic indication of time location of SBFD subband

|  | Company |
| --- | --- |
| Support | New H3C with modification, TCL (with modification), ZTE, Spreadtrum, Intel, Panasonic, Ericsson (with modification), DOCOMO, CMCC, CEWiT, CATT, Sharp, Xiaomi, WILUS (with modification), NEC, OPPO |
| Not support | InterDigital (needs modification), Samsung (alternative proposed agreement), vivo, Nokia, NSB (modification) |

| Company | Comments |
| --- | --- |
| New H3C | During stufy phase, we shouldn’t limit to support dynamic indication of time location of SBFD subband and we should investigate on both time and frequency allocation of SBFD subband and it is helpful for gNB to more flexiblly scheme UL resource of SBFD. So our proposal is as below: For indication of subband locations for SBFD operation, - Dynamic indication of frequency location and/or size of SBFD subband is not supported - FFS dynamic indication of time and frequency location of SBFD subband |
| TCL | We support dynamic indication for both frequency location and time location of subbands. Dynamic indication of both frequency location and time location may provide gNB the flexibility to avoid all types of collision as mentioned in section 2.1.4. Besides, several companies support dynamic indication of frequency location of subbands, therefore we suggest to review this proposal for further study as mentioned by New H3C or review as given below. Proposed Agreement: For indication of subband locations for SBFD operation, - FFS Dynamic indication of frequency location and/or size of SBFD subband is not supported - FFS dynamic indication of time location of SBFD subband |
| ZTE | Consdiering strong support of dynamic indication of time location of SBFD subband, it would be better if we can agree to study this aspect. Thus, we propose to change “FFS” to “study” in the second bullet. |
| Sony | The purpose of dynamic SBFD location indication is provide flexibility to adapt to changes in DL & UL traffic ratio. Therefore, if dynamic indication of time location is considered, we might as well consider frequency location. The frequency locations can be fixed to a few possible configurations, e.g. semi-statically configured 2 or more frequency locations and dynamically indicate one of them. |
| InterDigital | As all the above companies support dynamic indication for study, the current list of Support/NotSupport seems misleading. We also support dynamic indication. Revision by New H3C seems better. Maybe this issue is better to be discussed with the previous Proposal 1-4 together. |
| Samsung | We do not see the usefulness of a separate dynamic indication to toggle on/off particular symbols in time-domain for SBFD. The SBFD UL subband can be (1) configured by RRC in terms of time-domain symbols and RBs in frequency-domain. Then (2) dynamic scheduling from the gNB can be used to assign symbols/RBs in the configured SBFD subband for actual data transmission/reception from/to UEs. Providing (3) dynamic indication mechanism to either enable/disable symbols configured as part of the RRC configuration can be studied, but then is better included as additional “- FFS: potential benefits and signaling impacts to support dynamic indication of frequency location and/or size or time location of SBFD subband.” under Proposal 1-4. |
| Intel | We are generally fine with the proposal that dynamic frequency location change is not supported while further study time location change. But we feel it would be better to discuss whether dynamic frequency location and/or size or time location of SBFD subband can be supported without touching which signaling is to be used, e.g, whether a separate explicit indication for UL/DL subband or implicit indication by scheduling DCI. |
| Panasonic | We think dynamic indication of time domain location of SBFD subband (or similar functionality) would be needed for scheduling flexibility. However, dynamic indication of frequency domain location would not be essential considering UE complexity (e.g., analogue filter change if supported). |
| Lenovo | We think that network should be able to configure multiple subband sizes within a given frequency location and dynamically indicate an applicable subband size to adapt to varying UL and DL traffics. |
| Ericsson | We support the intention of the main bullet and 1st sub-bullet. Regarding, the FFS, "dynamic indication of time location," is unclear to us, since in Proposal 1-4 it states that "semi-static indiation of time and frequency location is baseline. So does it mean that somehow a semi-statically configured sub-band time location is overridden by dynamic signaling, and if so, what is it overridden with? Or does it mean something else? We think this needs more discussion, so for now the FFS bullet should be removed. Further, we think that this issue is related to whether or not the Tx direction of a previously of a semi-statically configured subband can be overridden. Hence, we propose the following (which avoids the word "supported" as discussed multiple times in our above comments) For indication of subband locations for SBFD operation, - Dynamic indication of frequency location and/or size of SBFD subband is not supported further considered - FFS dynamic indication of time location of SBFD subband |
| DOCOMO | We support dynamic indication of time location of SBFD subband. |
| QC | We believe that dynamic indication of the time locations of the SBFD achieves all scheduling flexibility needed (e.g., convert SBFD symbol to TDD) and to adapt to traffic loads. On the other hand, dynamic adapataion of the frequency locations of the SBs may not be needed that frequent and can be further discussed. We propose the following edits: Support dynamic indication/update of the at least time locations of the SBFD subbands. - FFS: dynamic indication/update of the frequency locations of the DL and/or UL SBFD subbands. |
| CMCC | The dynamic change of subband locations/bandwidths will increase the complexicty of inter-subband CLI handling, which may not be pursued in the first release of SBFD study. |
| Huawei, HiSilicon | Given that there is no clear conclusion on what kinds of subbands will be defined, e.g. whether flexbile subband will be defined, and how there are configured, it is a bit difficult to discuss how to enable dynamic subband indication. |
| vivo | We understand companies’ concerns on dynamic change the UL subband boundary in frequency domain if gNB uses analog filter for SI. But there are other methods in antenna domain, spatial domain and frequeny domain without using analog filter to address the SI. For such solutions, it is feasible to adjust the UL subband boundary in frequency domain. For dynamic indication of time location of SBFD subband, there is no difficulty to support it. As compromise, we are fine with following For indication of subband locations for SBFD operation, - Dynamic indication of frequency location and/or size of SBFD subband is not supported - FFS Support dynamic indication of time location of SBFD subband |
| Nokia, NSB | We think that, based on support from many companies, the proposed agreement could be changed to: For indication of subband locations for SBFD operation, - Dynamic indication of frequency location and/or size of SBFD subband is not supported - FFS dynamic indication of time location of SBFD subband Dynamic indication of time location of SBFD subband is supported |
| ETRI | We support the following: FFS dynamic indication of time and/or frequency location of SBFD subband |
| Xiaomi | Considering the applicability of dynamic TDD, we don’t think dynamic indication of SBFD subband is needed. |
| WILUS | Companies provided potential benefits and motivations of dynamic indication of time location. Thus, at least dynamic indication of time location can be studied. |

Proposal 1-6

Proposed Agreement:

For indication of subband locations for SBFD operation, SBFD subband time locations are explicitly indicated within a period.

- FFS the periodicity is explicitly indicated or implicitly determined

- FFS whether the SBFD subband time locations are contiguous within a period

- FFS multiple periods are supported

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony, InterDigital, Samsung (modified wording), Spreadtrum (main bullet), Intel (main bullet), Panasonic (at least semi-static), Lenovo, ITRI, Ericsson (main bullet only), DOCOMO, Qualcomm, CMCC (with comment), CEWiT, Fujitsu, CATT, Sharp, vivo, Nokia, NSB (with reformulation of the FFS), ETRI (main bullet), Xiaomi, WILUS, NEC, OPPO |
| Not support |  |

| Company | Comments |
| --- | --- |
| New H3C | From our perspective, the SBFD subband time location should be contiguous, otherwise, there will be multiple GP. |
| Samsung | It makes sense to us that the SBFD subband configuration in time-domain will need to be signaled with respect to a “period”, e.g., compatible with the TDD UL-DL patterns 1 and 2 or some recurring symbol pattern, due to the presence of legacy UEs. We do not see the usefulness of the 3 sub-bullets. The 3 proposed FFS are Stage 3 signaling details to consider later during the WI stage. We should first decide if dynamic indication is supported at all and if SBFD SB configuration is supported on symbols carrying DL signals such as the SSB(s). We propose the following modified wording: “For indication of subband locations for SBFD operation and semi-static configuration of subband time and frequency location, SBFD subband time locations are explicitly indicated within a period.” |
| Spreadtrum | We support the main bullet, the details can be FFS, such as “FFS details”. Because we still suggest to discuss the relationship of this SBFD time domain signalling and tdd-UL-DL-ConfigurationCommon/dedicated. From our understanding, the period of tdd-UL-DL-ConfigurationCommon can be used derectly. So if there are two periods configured, those two periods also can be adopted for SBFD time domain signaling. |
| MediaTek | No strong objection, but we would like to first agree on the relation between the subbands signalling and the TDD signalling (e.g., is the subband configuration applicable to only “F” symbols, if so, why we need new “periodicity” for the subband configuration?) |
| Intel | In our understanding, based on analysis in section 2.1.2.4, the proposal 1-6 is for semi-static subband location. For dynamic subband location, it is still FFS according to proposal 1-5. Therefore, it is better to add ‘semi-static’ for the proposal. For 3 FFS points, we prefer to remove all 3 FFS points for now, because we think it is more like WI work as mentioned by Samsung. Having said that, we are fine to discuss whether SBFD subband time locations are contiguous, but NOT from signaling point of view, we think the impact on gNB complexity such as frequency reconfiguration of analog filter and usefulness can be discussed. |
| Panasonic | In our understanding, this proposal is intended for semi-static configuration as discussed in section 2.1.2.4. Then, we propose to modify the proposal as follows. For indicationsemi-static configuration of subband locations for SBFD operation, SBFD subband time locations are explicitly indicated within a period. - FFS the periodicity is explicitly indicated or implicitly determined - FFS whether the SBFD subband time locations are contiguous within a period - FFS multiple periods are supported |
| Ericsson | Similar view as Samsung. The FFSs are related to signaling details that are better suited to WI phase, thus we prefer to remove the sub-bullets. Also, is this proposal about dynamic indication or semi-static configuration? If it is about semi-static, we are fine with the intention of the main bullet. If it is about dynamic indication of time location, then we have the same comment as for Proposal 1-5 that dynamic indication of time location is unclear to us. In Proposal 1-4 it states that "semi-static indiation of time and frequency location is baseline. So does it mean that somehow a semi-statically configured sub-band time location is overridden by dynamic signaling, and if so, what is it overridden with? Or does it mean something else? We think this needs more discussion, so for now the main bullet should clearly state "semi-static." Further, we think that dynamic indication of time location is related to whether or not the Tx direction of a previously of a semi-statically configured subband can be overridden. This is a separate discussion. Hence we propose the following modification for the main bullet: For SBFD operation, it is agreed that semi-static configuration of the time locations of SBFD symbols within a period is the baseline |
| CMCC | Similar comment as proposal 1-4, suggest to add the following FFS: For indication of subband locations for SBFD operation, SBFD subband time locations are explicitly indicated within a period. - FFS the periodicity is explicitly indicated or implicitly determined - FFS whether the SBFD subband time locations are contiguous within a period - FFS multiple periods are supported - FFS how to configure/indicate the link direction of SBFD symbols |
| Huawei, HiSilicon | We are mostly fine with the main bullet and suggest to make the following update. The reason is that “explicit” seems to imply that the SBFD subband time locations are configured by indicating the symbol locations within the period which precludes other signaling possibilities. For indication of subband locations for SBFD operation, SBFD subband time locations are explicitly indicated within a period. |
| Nokia, NSB | We agree with Qualcomm and other companies that it should be possible to operate SBFD with DL-only slots/symbols. Therefore, we support explicit configuration of the semi-static SBFD subband time locations. We think though the FFS part is too detailed and could be rephrased as “Signalling details are FFS”. |
| OPPO | In our view, SBFD subband time locations should be aligned with TDD configuration, so the periodicity for SBFD subband time location keep the same as TDD configuration periodicity. Considering TDD configuration supports up to two periodicity values, the periodicity for SBFD subband time location can be up to two value. |

Proposal 1-7

Proposed Agreement:

For indication of subband locations for SBFD operation, at least support explicit indication of frequency location of UL subband.

- FFS whether frequency location of DL subband(s) are explicitly indicated or implicated determined.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony, InterDigital, Samsung (modified wording), Spreadtrum, MediaTek, Intel, Panasonic, Lenovo, ITRI , Ericsson (with modification), DOCOMO, Qualcomm, CMCC, CEWiT, CATT, Sharp, vivo, Xiaomi, WILUS, NEC (with modification) |
| Not support | Nokia, NSB, OPPO (concern on “DL subband” in subbullet) |

| Company | Comments |
| --- | --- |
| ZTE | From our perspective, we also need to study whether guard band should be indicated or not. The FFS can be updated as following. - FFS whether frequency location of DL subband(s) and/or guard band(s) are explicitly indicated or implicated determined. |
| Sony | We share similar view with ZTE, i.e. we should also consider whether guard subbands are explicitly indicated or not. |
| Samsung | We are in principle fine with the proposal, but prefer to clarify that the proposal is meant for the semi-static SBFD configuration case, Proposed modified wording for main bullet: “For indication of subband locations for SBFD operation and semi-static configuration of subband time and frequency location, at least support (…)” |
| MediaTek | We don’t agree with ZTE’s modification, all what the UE needs to know is the UL/DL subbands. |
| Intel | Similar comments as for proposal 1-6, in our understanding, the proposal 1-7 is for semi-static subband location. For dynamic subband location, it is still FFS according to proposal 1-5. Therefore, it is better to add ‘semi-static’ for the proposal. |
| Ericsson | Agree with Samsung that it should be clarified that this proposal is about semi-static configuration of the subban frequency location. Suggest minor wording change to avoid the word "support" as we have mentioned for several other proposals For semi-static configuration indication of subband frequency locations for SBFD operation, at least support it is agreed that explicit indication of the frequency location of at least the UL subband is the baseline. - FFS whether frequency location of DL subband(s) are explicitly indicated or implicated determined. |
| QC | In addition, the guardband indication should be studied. Fine with the wording of ZTE. |
| Huawei, HiSilicon | First, there is a typo in the subbullet, we assume “implicated” should be “implicitly”. In addition, we are wondering whether the proposal imply there is only UL subband and DL subband, is flexbile subband precluded? |
| Nokia, NSB | We think that the signaling should cover 2 or 3 (UL subband, guard band, DL subband) and they should be discussed and agreed together. |
| ETRI | We suggest the following: For semi-static configuration indication of subband frequency locations for SBFD operation, at least support it is agreed that explicit indication of the frequency location of at least the UL subband is required. FFS whether/how to explicitly indicate or implicitly determine frequency location of DL subband(s) and/or guard band(s) are explicitly indicated or implicated determined. |
| Xiaomi | It should be clarified that the mentioned mechanism is applied on DL symbols, in order to comply the guidance from RAN plenary. Proposed Agreement: For indication of subband locations for SBFD operation, at least support explicit indication of frequency location of UL subband. - FFS whether frequency location of DL subband(s) on DL symbols are explicitly indicated or implicated determined. |
| NEC | Same comment as in ZTE |
| OPPO | Two comments: - Agree with ZTE and other companies that the guard band should be taken into account. One UE may be configured with a guard band but no UL subband, where the guard band blocks all RBs belonging to a UL subband on gNB side from being used as DL. This may relate to Apple/OPPO’s question in Proposal 1-1: whether UL subband belongs to UL BWP or DL BWP? In either case, we believe the guard band runs in DL BWP. - It is not quite clear to us at this stage to necessarily introduce the concept of “DL subband”. The proposal is not made clear whether the “DL subband” refers to contiguous DL RBs in UL symbol, which is deprioritized by RAN plenary. If the intention is to refer to the “leftover RBs after taking UL-subband/guardband away”, we do not think “DL subband” is really needed given the current spec takes care of such case by “rate matching”. |

Proposal 1-8

Proposed Agreement:

The maximum number of subbands for SBFD operation in an SBFD symbol within a TDD carrier is three.

- Note: Support of three SBFD subbands may subject to RAN4’s study and conclusion

|  | Company |
| --- | --- |
| Support | New H3C, TCL, Sony, InterDigital, Samsung, Spreadtrum, MediaTek, Intel (also fine with ZTE’s version), Panasonic, Lenovo, ITRI, Ericsson (minor wording change), DOCOMO, Qualcomm, CMCC, CEWiT, Fujitsu, CATT, Sharp, vivo, ETRI, Xiaomi, WILUS, NEC |
| Not support | ZTE, Nokia, NSB, OPPO |

| Company | Comments |
| --- | --- |
| ZTE | Agree on this would open the door for some other SBFD cases that have never been discussed yet, e.g., UDU. Thus, to make it clear, we propose the following proposal. Proposal Alt.1: At least support the following SBFD subband pattern in an SBFD symbol within a TDD carrier. - {DL subband, UL subband, DL subband}; - {DL subband, UL subband}; - {UL subband, DL subband}; - Note: guard band may or may not be needed between DL subband and UL subband. Alternatively, we can also consider the following proposal. Proposal Alt.2: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one. |
| Samsung | We are ok in principle with the proposed wording, but it may be best to clarify the possible assumed subband configurations by listing them such as other companies have suggested. |
| Ericsson | Agree with the intention of the proposal. Suggest minor wording change like for all other proposals. For The maximum number of subbands for SBFD operation in an SBFD symbol within a TDD carrier it is agreed that is three is the baseline. - Note: Support of three SBFD subbands may subject to RAN4’s study and conclusion |
| CMCC | We also think the SBFD configuration patterns should also be discussed. |
| Huawei, HiSilicon | Similar comment to proposal 1-5, without the common understanding on what kinds of subbands will be defined, e.g. whether flexbile subband will be defined, and how there are configured, it is a bit difficult to discuss the maximum number of subbands that can be configured. |
| Nokia, NSB | We agree with ZTE that it should be ok as maximum number of UL subband for SBFD is one. |
| Xiaomi | We still prefer to clearly spell out the subband type, for example, the number of UL subbands for SBFD operation in a SBFD symbol within a TDD carrier is one. However, considering we have already discussed this proposal for several meeting, we can compromise to this proposal with the understanding that ‘three’ denotes the number of UL subband and DL subband. |
| OPPO | Agree with ZTE. The current proposal even allows {UL subband, UL subband, UL subband}. Prefer ZTE’s Proposal Alt.2. Further, it seems RAN1 should firstly further clarify SBFD symbol: For UL subband, could SBFD symbol be UL or Flexible? |

Proposal 1-9

Proposed Agreement:

SBFD aware UE does not transmit UL channel/signal outside the UL subband in SBFD symbols.

- FFS UE behaviour if a scheduled/configured UL transmission overlaps with DL subband(s) and guard band(s) if configured

|  | Company |
| --- | --- |
| Support | New H3C with clarification and modification, TCL, ZTE, Sony, InterDigital, Samsung, Spreadtrum, Intel, Panasonic, Lenovo, ITRI , Ericsson (main bullet, but proposal as a whole needs modification), DOCOMO, CMCC, CEWiT, Fujitsu, CATT, Sharp, Nokia, NSB (with modification) , Xiaomi, NEC |
| Not support | MediaTek, Qualcomm (support w/ updates), vivo, OPPO (unclear use of SBFD symbol) |

| Company | Comments |
| --- | --- |
| New H3C | We need clarify a little bit on what is resource attribute outside the UL subband in SBFD symbols. If resource attribute outside the UL subband in SBFD symbols is DL, above proposal is fine. If resource attribute outside the UL subband in SBFD symbols is flexible, we need FFS whether we need limit UE behaviour. So we suggest to add one condition to main bullet and one subbullet for flexible resource case. Our proposal is as blow: SBFD aware UE does not transmit UL channel/signal outside the UL subband in SBFD symbols if resource outside the UL subband in SBFD symbols is DL. - FFS UE behaviour if a scheduled/configured UL transmission overlaps with DL subband(s) and guard band(s) if configured - FFS UE behaviour if resource outside the UL subband in SBFD symbols is flexible. |
| TCL | We share similar views with New H3C |
| ZTE | Another aspect needs to be discussed is whether gNB can perform reception outside the UL subband in SBFD symbols. |
| Sony | New H3C raised a new aspect to be considered, i.e. can the SBFD symbol/slot contains Flexibe Subbands. That is do we support SBFD configurations like {FUF}, {FU} or {UF} or even {FUD} or {DUF}? |
| InterDigital | We also share similar views with New H3C. |
| MediaTek | We don’t see a need for such restriction. The gNB may ask the UE to send SRS in the DL subbands (if needed) for the purpose of channel measurement (DL-UL reciprocity) at the gNB and/or inter-UE CLI measurements (measured by other UEs in the DL subband). |
| Intel | In our understanding, the proposal does not exclude the possibility that the configured or scheduled (at least for repetition case) resource for UL channel/signal can overlap with DL subband and guard band if any. For such case, UE does not transmit UL channel/sginal ousdie the UL subband, which means, it is also open whether UE drops the whole transmission or perform partial transmission within UL subband, which is the intention of FFS point. Regarding the ‘Flexibe Subbands’ mentioned by other companies, we agree it is a valid point and we’re open for further study. Maybe we can revise the wording for the main bullet to forbid UL transmission in DL subbands and add FFS for flexible subbands as a sub-bullet. |
| Ericsson | We agree to the intention of the main bullet if it is meant to exclude Options 3 and 4 since those options implicitly require dynamic re-configuration of the analog filter prior to the ADC in the gNB receive chain. This is undesirable, and for some gNB Rx architectures infeasibile, from an implementation standpoint. Hence, it should be clarified that this is the intention, e.g., The baseline is that SBFD aware UE does not transmit UL channel/signal outside the UL subband in SBFD symbols. Note: this precludes Options 3 and 4 from the RAN1#110 agreement We're confused about the FFS since it says "scheduled UL" which refers to Options 3 and 4 which we thought the moderator was trying to preclude. Rather than formulating an entirely new proposal that may conflict with the RAN1#110 agreement, wouldn't it be better to try and address Options 1, 2, 3, and 4 from the RAN1#110 agreement first? Our first preference from that agreement is Option 1 only. |
| QC | In the last meeting, RAN1 listed 4 options for the of UE behaviour within the UL and DL subband. Option 1 was supported by all companies as baseline. For this proposal, we should also discuss the SBFD-aware UE behaviour for reception in DL subband. Agreement: For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, study the following options: - Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol. Suggest updates below. SBFD aware UE does not transmit UL channel/signal outside the UL subband in SBFD symbols or to receive DL reception within the UL subband in the SBFD symbol. - FFS UE behaviour if a smcheduled/configured UL transmission overlaps with DL subband(s) and guard band(s) if configured |
| CMCC | The issue proposed by New H3C deserves to be discussed. From our point of view, the condition of this proposal is restrict UE not transmiting outside UL subband in “SBFD symbols”, which the “SBFD symbols” means gNB is performing SFBD operation in these symbols, i.e., gNB receives in UL subband and transmists in DL subband simultaneously. In this sense, there is no such “flexbile subband”. |
| Huawei, HiSilicon | We have similar questions as New H3C. In our understanding, we can have a definition of transmission direction of subband first - The subband indicated as UL is defined as a set of consecutive RBs which is only used for UL transmission. - The subband indicated as DL is defined as a set of consecutive RBs which is only used for DL transmission. |
| vivo | We would like to clarify what is the meaing of “UL subband in SBFD symbols”: - Does the SBFD symbol include semi-static DL symbol and also semi-static flexible symbol? - Does the SBFD pattern in frequency domain only indicate the frequency location of UL subband (related to proposal 1-7)? For a semi-static flexible symbol configured with UL subband, from gNB perspective, it can use the semi-static flexible symbol as full UL symbol, as full DL symbol, or SBFD symbol with (DU) pattern in frequency domain as shown in Figure 1 below. If gNB use the semi-static flexible symbol as full UL symbol, then the UL channel/signal for a UE can be scheduled/configured outside the UL subband, from this scheduled UE perspective, the semi-static flexible symbol is no longer a SBFD symbol. Some companies said similar benefits can be achieved by supporting dynamic indication of the subband time domain locations for example, indicating the SBFD pattern is not applied in the semi-static flexible symbol, which means the SBFD operation becomes transparent for all UEs. Then the benfits of non-transparent way for non-scheduled UE is lost. Therefore, we suggest to first clarify what the SBFD symbol in the proposal means, and what is the preferred SBFD operation from NW side, and followed by defining UE behaviour and related signalling. Figure 1 Potential operation from gNB perspective for semi-staic F symbol w UL subband We notieced that following SBFD symbol was defined in RAN1#109-e meeting. Conclusion@RAN1#109-e meeting For discussion purpose only, SBFD symbols is defined as symbols with subbands that gNB would use for SBFD operation. However, above conclusion seems made from gNB perspective. To define UE behavior for SBFD operation, we think better to define SBFD symbol from UE perspective. Following can be the starting point: - SBFD symbols is defined from UE perspective as the symbols where the configured DL and UL sub-bands apply. |
| Nokia, NSB | We agree with the intention of the agreement though wording could be improved, e.g.: The SBFD aware UE does not expect to be scheduled or configured with DL reception with UL transmission outside the UL subband in SBFD symbols FFS UE behaviour if a scheduled/configured UL transmission overlaps with DL subband(s) and guard band(s) if configured |
| ETRI | We share similar views with New H3C |
| WILUS | We share the similar view with New H3C. If SBFD aware UE does not transmit UL channel/signal outside the UL subband in flexible symbols, SE will be reduced even compared to legacy UE. |
| OPPO | The use of “SBFD symbol” in the proposal is not clear. RAN1 has not even yet decided whether UL subband can be created on UL symbols. If yes, it is not reasonable to restrict uplink transmission outside the UL subband. If the SBFD symbol is a flexible symbol, it needs further discussion for the case where the flexible symbol is turned into UL by either TDD-Dedicated or dynamic SFI or uplink scheduling, which means the whole symbol is converted as UL. For now, it seems only safe to say: SBFD aware UE does not transmit UL channel/signal outside the UL subband in SBFD at least DL symbols. |

Proposal 1-10

Proposed Agreement:

Study impact and potential enhancements of non-contiguous FDRA across DL subbands for PDSCH.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, Sony, InterDigital, Samsung, MediaTek, Intel, Lenovo , Ericsson, DOCOMO, Qualcomm, CMCC, CEWiT, CATT, Sharp, Nokia, NSB, Xiaomi, WILUS (clarification), NEC |
| Not support |  |

| Company | Comments |
| --- | --- |
| New H3C | Maybe more explanations are needed below this. For example: Note: The non-contiguours FDRA for PDSCH means the PDSCH for one user is scheduled in two separated DL subbands. |
| ZTE | We are open to study this aspect. However, we think this issue should be considred as low prioty considering that network implementation can address the issue to some extent. Meanwhile, we should first address the key issues (e.g., subband configuration and subband configuration granularity) and then come back to this issue later on. |
| Spreadtrum | The proposal is not clear. The non-contiguous FDRA across DL subbands can be supported by resource allocation Type 0. It would be good to clarify which type or both types of PDSCH resource allocation can be studied. |
| CMCC | We think a general solution can be considered and pursued for DL resource allocation enhancement, including PDSCH, CSI-RS, unaligned bundaries. |
| Huawei, HiSilicon | Considering the limitation of rate matching resources number for PDSCH, it is better to study non-contiguous FDRA for RA type 1 in the case of {DUD}. |
| Sharp | As discussed in our contribution, the existing rate matching pattern cannot overlap with DMRS for PDSCH, which is intended not to change the channel estimation behaviour. Therefore, change to FDRA for PDSCH should be studied with high priority. Further, in our understanding, the fact that DCI format 1_0 only supports RA type-1 should be considered. |
| vivo | We would like to clarify the non-contiguous FDRA enhancements are for both RA Type 0 and RA Type 1? |
| Nokia, NSB | Ok to study, but we could start to separately study the impact for different types of FDRA, e.g. FDRA Type-0 and FDRA Type-1 with and without interleaving. |
| Xiaomi | Non-contiguous FDRA can be realized with RA type 0. The target RA type should be RA type 1? |
| WILUS | Please clarify whether the proposal include FDRA Type 1 or not. Current FDRA Type 0 already supports non-contiguous FDRA, while FDRA Type 1 does not support. |
| OPPO | It depends on whether there indeed exist two non-contiguous DL RB segments in SBFD operation (e.g., the max number in Proposal 1-8 is eventually two). In our view, two non-contiguous DL subband configuration leads more spec impacts. So we prefer to postpone this discussion till after decision on Proposal 1-8. |

Proposal 1-11

Proposed Agreement:

Study impact and potential enhancements of CSI-RS resource and CSI report configuration across DL subbands.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony, InterDigital, Samsung (modified wording), MediaTek, Intel, Lenovo, Ericsson (with clarification), DOCOMO, Qualcomm, CMCC, CEWiT, Huawei, HiSilicon, CATT, Sharp, vivo, Nokia, NSB, ETRI, Xiaomi, WILUS, NEC (with addition) |
| Not support |  |

| Company | Comments |
| --- | --- |
| Samsung | We are fine in principle with the proposed agreement but propose to remove “DL” (subbands) from the wording. |
| Spreadtrum | We support the intention. But the words may be changed a little, such as Study impact and potential enhancements of CSI-RS resource set configuration and CSI reporting. |
| Ericsson | We think it should be clarified that the discussion is around non-contiguous resource allocation. Hence suggest the following: Study impact and potential enhancements of CSI-RS frequency domain resource allocation and CSI report configuration considering non-contiguous across DL subbands. |
| QC | Regarding Samsung comment, the study of this enhancement is for CSI in SBFD symbols. If across DL subband is removed, then there is no enhancement needed. |
| CMCC | Same comment as proposal 1-11 |
| Nokia, NSB | We think that solution that blanks out the CSI-RS resource elements that are overlapping with the UL subband should be assumed as baseline. |
| ETRI | We suggest the following: Study impact and potential enhancements ofon CSI-RS resource and CSI report configuration across DL subbands for SBFD operation. |
| NEC | Would also like to study the part on impact on UE CSI measurement procedure Study impact and potential enhancements of CSI-RS resource and CSI report configuration and CSI measurements across DL subbands. |

Proposal 1-12

Proposed Agreement:

Study impact and potential enhancements due to unaligned boundaries of DL/UL subband and configuration granularity (e.g. RBG for PDSCH/PUSCH FDRA, CSI reporting subband).

|  | Company |
| --- | --- |
| Support | New H3C, TCL, Sony, InterDigital, MediaTek, Intel, Lenovo, CMCC, CEWiT, Huawei, HiSilicon, CATT, Sharp, Nokia, NSB, ETRI, Xiaomi, NEC |
| Not support | Samsung, Ericsson, vivo(clarification is needed) |

| Company | Comments |
| --- | --- |
| ZTE | Similar comment as in Proposal 1-10. We are open to study these aspects, but we think these should be treated as lower priority. |
| Samsung | @FL: Could you clarify the difference between proposals 1-10/1-11 versus 1-12? |
| Spreadtrum | For RBG for PDSCH/PUSCH FDRA, it is defined in Type-0 RA and Type-1 RA when scheduled by DCI 0_2/1_2. We ask for a clarification that whether both of RBG in the scope. Besides RBG, CSI reporting subband, PRG of PDSCH can be listed too. |
| Intel | In our understanding, unaligned DL/UL subband boundary with resource allocation granularity also exists for PDCCH (the resource allocation granularity is 6 PRBs). We suggest capturing PDCCH together with other channels as examples in the proposal for now, but of course, the impact may be different for different channels thus necessirty for enhancement for different channels can be different based on the outcome of study. |
| Ericsson | Support the general direction the proposal, but agree with Samsung that there seems to be overlap with Proposal 1-10 and 1-11, and there should be clarification. Perhaps it is better to formulate separate proposals for PDSCH, PUSCH, and CSI-RS. We also think that the case of an RBG (DL or UL) overlapping a guardband should be considered as part of the proposal. |
| DOCOMO | Agree with ZTE that this can be lower priority. |
| QC | Similar comments as other companies. This proposal is subset of proposal 1-10 and 1-11. In our views, 1-10 covers enhancements for Resource allocation Types including RBG. And Proposal 1-11 covers CSI reportin subband. |
| CMCC | Same comment as proposal 1-11 |
| vivo | Wee would like to clarify the relation between proposal 1-10, 1-11 and 1-12, whether they have some overlapping aspects? |
| Nokia, NSB | We are ok to study this though we are not sure that unaligned boundaries of DL/UL subband configuration granularity may be a major issue. First, the network may configure the UL-DL partitioning in frequency domain so that the boundaries of the UL/DL subbands are aligned with the boundaries of the RBGs. Second, RAN1 has not yet discussed what is the required SBFD configuration granularity in frequency domain. If no finer granularity than RBG is needed, then there may no be any impact due to unaligned boundaries of DL/UL subband and configuration granularity. |
| WILUS | If the intention of proposal 1-10 and 1-11 are to address the proposal 1-12, proposals seem to be overlapped. Otherwise, if the proposal 1-10 was made to purely enhanced FDRA (e.g, non-contiguous FDRA Type 1) it can be a separate proposal. |

Proposal 1-13

Proposed Agreement:

Study impact and potential enhancements for transmissions across SBFD symbols and non-SBFD symbols.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony, InterDigital, Samsung, MediaTek, Intel (minor revision), Panasonic, Lenovo, Ericsson (with clarification), DOCOMO, Qualcomm, CMCC, CEWiT, CATT, Sharp, Nokia, NSB (with clarification) , ETRI, Xiaomi, WILUS, NEC |
| Not support |  |

| Company |  | Comments |
| --- | --- | --- |
| New H3C |  | This can be discussed together with UL or DL transmission outside the UL subband or DL subband. |
| Spreadtrum |  | From our understanding, the proposal refers to the repetitions of one PDSCH/PUSCH/PUCCH, not two different channels which across SBFD/non-SBFD symbols. So we prefer to change it into: Study impact and potential enhancements for transmissions repetitions across SBFD symbols and non-SBFD symbols. |
| Intel |  | We’re not sure whether the proposal is just for PDSCH/PUSCH/PUCCH repetition or UL frequency hopping across different symbol types (because the wording in proposal is ‘across’) or the proposal also includes the case without repetition which is within one symbol type each time, e.g., separate PUCCH configuration for SBFD and non-SBFD symbol? We think both cases should be studied. To avoid confusion, we suggest to change ‘across’ to ‘in’ |
| Ericsson |  | Agree with direction of proposal, but clarification is needed. Agree with Spreadtrum's clarification, and further it would be useful to clarify further: Study impact and potential enhancements for PUSCH, PUCCH, and PDSCH transmissions repetitions across SBFD symbols and non-SBFD symbols. |
| QC |  | It should be clarified that transmission refers to UL transmission (e.g. SRS, PUCCH, PUSCH, PRACH) or both UL transmission and DL reception. We think the scope of proposal could be also cover study for potential enhancement of DL reception across SBFD and non-SBFD symbols (e.g. SPS-PDSCH, PDSCH repetiton, etc. ). |
| Huawei, HiSilicon |  | We have similar question as Intel. We suggest the following Study whether and how to perform multi-slot PUCCH/PUSCH/PDSCH transmissions w/ and w/o frequency hopping in or across SBFD symbos and non-SBFD symbols. |
| vivo | Are the transmissions above including following? - Repetitions across SBFD symbols and non-SBFD symbols? - SPS PDSCH and CG PUSCH? - Reference signals like CSI-RS, SRS etc? |  |
| Nokia, NSB |  | We re generally ok with the agreement, however it is not clear whether this only covers FDRA enhancements due to multi-slot transmissions across SBFD and non-SBFD slots (e.g. repetitions, frequency hopping) or it more generally apply to any resource allocation that may span over both SBFD and non-SBFD slots (e.g. CG-PUSCH, PDCCH, etc.). Therefore we propose to split the agreement/proposal into two: Proposed Agreement: Study impact and potential enhancements for multi-slot transmissions across SBFD symbols and non-SBFD symbols Proposed Agreement: Study impact and potential enhancements for semi-static resource configurations (PDCCH, SPS, CG-PUSCH, PUCCH, SRS)transmissions across SBFD symbols and non-SBFD symbols. |
| ETRI |  | We share similar view with Spreadtrum. |
| WILUS |  | Support. For the companies’ comments on ‘transmission’, we are fine to include not only UL transmission but also DL reception. However, it should not be confined to repetition case only. For example, since the 2nd hop location is determined based on the size of active BWP, it can across the UL subband boundary even for intra-slot FH. Additionally, in Rel-17 CovEnh WI, TB processing over multi-slot PUSCH was introduced to improve PUSCH coverage. In this case, transmission across SBFD symbols and non-SBFD symbols can be scheduled without repetitions. |
| NEC |  | It is not exactly clear that what transmission types are being considered in the proposal. From our understanding, we need to study both configured transmissions (e.g. CG, PUCCH) and can also correspond to multi-slot PDSCH/PUSCH transmissions. Perhaps we may need to emphasis this in the proposal: Study impact and potential enhancements for configured transmissions and multi-slot PUSCH/PDSCH transmissions across SBFD symbols and non-SBFD symbols |

Proposal 1-14

Proposed Agreement:

Study whether/how to handle collision of UL transmission and DL reception in a same SBFD symbol for SBFD aware UEs:

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony, InterDigital, Samsung, Spreadtrum, MediaTek, Intel, Lenovo , Ericsson (with modification), DOCOMO, Qualcomm, CMCC, CEWiT, Huawei, HiSilicon, CATT, Sharp, vivo, Nokia, NSB, ETRI, Xiaomi, WILUS, NEC |
| Not support |  |

| Company | Comments |
| --- | --- |
| Sony | Note that in addition to FL summary, we also noted collision between configured resources vs scheduled resources in different link directions, e.g. PDCCH Search Space colliding with Dynamic PUSCH/PUCCH or Configured PUSCH/PUCCH, SPS with Dynamic PUSCH/PUCCH, etc. |
| Intel | Maybe better to discuss different combiations of dynamic and configured scheduling separately, e.g., DG vs DG, CG vs CG, DG vs CG. |
| Ericsson | We are okay to study this, but think it is low priority. Detailed collision handling rules (e.g., as specified currently in 38.213 Section 11) are better suited for a WI. Maybe it would be more productive from a SI conclusion point of view to try to identify a list of potential collisions that may need to be handled in normative work. |
| QC | It is good to clarify that collion here refers to time-domain collison. |
| Nokia, NSB | We agree this should be studied. Related to the discussion section 2.1.4, we would like to point out that, based on the discussions in sections 2.1.2.2 and 2.1.2.4 and on the corresponding proposals/agreements Proposal 1-4 and Proposal 1-6, it could be that a new definition of ‘Semi SBFD’ symbol may be introduced to describe a symbol that is semi-statically configured as SBFD, and which is therefore different from a ‘semi D’ symbol. |

Proposal 1-15

Proposed Agreement:

UL transmissions within UL subband in symbols configured as DL in TDD-UL-DL-ConfigCommon are supported.

- FFS symbols configured as DL in TDD-UL-DL-ConfigCommon and in tdd-UL-DL-ConfigurationDedicated

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony (clarification on FFS), Samsung (modified wording), Spreadtrum, Panasonic, Lenovo (removing FFS part), Ericsson (main bullet), DOCOMO (clarification), Qualcomm, CMCC, CEWiT, Huawei, HiSilicon, CATT, Sharp, Nokia, NSB (w/o FFS) , Xiaomi, NEC |
| Not support | MediaTek, vivo, OPPO (proposal does not answer key question) |

| Company | Comments |
| --- | --- |
| New H3C | In the main sentence, the DL symbol in TDD-UL-DL-ConfigCommon has been mentioned. While in the sub-bullet, there is still DL symbol in TDD-UL-DL-ConfigCommon, what do you mean here? |
| ZTE | From our perspective, the flexible symbols can also be included in the FFS. |
| Sony | Same question as New H3C, on the FFS. I believe it is supposed to mean UL in TDD-UL-DL-ConfigCommon, i.e.: - FFS symbols configured as UL in TDD-UL-DL-ConfigCommon and DL & UL in tdd-UL-DL-ConfigurationDedicated |
| Samsung | We think that both DL and F symbols should be covered. We propose to update the wording to, UL transmissions within UL subband in symbols configured as DL or F in TDD-UL-DL-ConfigCommon are supported. FFS (…). |
| Intel | Is FFS point only for legacy UE or both legacy UE and SBFD aware UE? In our understanding, legacy UE can still be configured with UE-specific DL symbol even though gNB performs SBFD in the symbol. And also, what is TDD-UL-DL-ConfigCommon is not provided while tdd-UL-DL-ConfigurationDedicated is provided? Is the intention to not support SBFD operation in a UE-specific DL symbol for such case ? Besides, we also think SBFD operation in flexbile symbol can be supported. |
| Lenovo | Fine with the main bullet, but we don’t think FFS part is needed. |
| Ericsson | We agree to the main bullet (with the exception of the word "support" as we have commented in many other proposals). We still have concern on agreeing on 'F' symbols in TDD-DL-UL-ConfigCommon due to unclear support from legacy UEs. In our view, a baseline SBFD solution design should not rely on this, due to the need to coexist with legacy devices. We think the sub-bullet is pre-mature until there is resolution on the issue of how the time domain location of SBFD symbols is handled; hence we prefer to remove the sub-bullet. Suggest the following modification to avoid the word "support" as we have stated in other proposals: It is agreed that UL transmissions within UL subband in symbols configured as DL in TDD-UL-DL-ConfigCommon is the baseline are supported. |
| DOCOMO | Can we understand the intention of the FFS (i.e. difference between the main bullet and the FFS bullet) would be: symbols configured as flexible in TDD-UL-DL-ConfigCommon and configured as DL in tdd-UL-DL-ConfigurationDedicated? |
| QC | We support at least that SBFD operation in DL symbols should be supported whether symbols in indicated as DL by TDD-UL-DL-ConfigCommon or tdd-UL-DL-ConfigurationDedicated. In addition to supporting SBFD in the DL symobls, the ‘flexible’ symbols should be supported. An FFS should be added if some companies have concern on supporting SBFD in flexible symbols. |
| vivo | We think UL transmissions within UL subband in flexible symbols provided by TDD-UL-DL-ConfigCommon can also be supported. |
| Nokia, NSB | We think the FFS is not needed. Why would a gNB configure a symbol/slot as DL in tdd-UL-DL-ConfigurationDedicated when it is already configured as DL in TDD-UL-DL-ConfigCommon, if it intends to schedule UL in the corresponding symbol/slot? |
| WILUS | In our view, flexible symbols by TDD-UL-DL-ConfigCommon and/or TDD-UL-DL-ConfigDedicated also can be used for SBFD operation. Otherwise, resources in flexible symbols cannot be fully utilized for UL transmission (e.g., CG-PUSCH). |
| OPPO | The proposal does not seem to answer a key question: whether UL subband can be configured on UL symbol? The proposal only says the DL symbol portion of a UL subband can be used for UL transmission. In addition, we are a bit confused by subbullet, since “symbols configured as DL in TDD-UL-DL-ConfigCommon and in tdd-UL-DL-ConfigurationDedicated” is equivalent to “symbols configured as DL in TDD-UL-DL-ConfigCommon”, the same as in main bullet. |

Proposal 1-16

Proposed Agreement:

Study whether SBFD operation in SSB symbols is supported or not.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony, InterDigital, Samsung, Spreadtrum, MediaTek, Intel, Lenovo, ITRI , Ericsson, DOCOMO, Qualcomm, CMCC, CEWiT, Huawei, HiSilicon, CATT, Sharp, vivo, Nokia, NSB, ETRI, Xiaomi, WILUS, NEC, OPPO (clarification maybe needed) |
| Not support |  |

| Company | Comments |
| --- | --- |
| New H3C | We suggest that an FFS is added below this： - FFS：Study whether DL SBFD operation in PRACH symbols is supported or not. |
| DOCOMO | OK with the proposal, but we don’t support SBFD operation in SSB symbols, which would have impact on legacy UEs. |
| Nokia, NSB | We think the following aspects should be carefully considered: impact of UL subband on (i) legacy UE synchronization procedure and (ii) serving cell/neighbour cell measurements |
| OPPO | We are open to study this issue. And we slightly prefer to block SBFD operation in SSB symbols to avoid impact from self-interference and inter-UE CLI. BTW, it is better to clarify whether the proposal is from UE perspective or gNB perspective. |

Proposal 1-17

Proposed Agreement:

Study potential benefits and enhancements for initial access in UL subband.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony, InterDigital, Samsung, MediaTek, Intel, Lenovo, DOCOMO, Qualcomm, CMCC, CEWiT, CATT, Sharp, Nokia, NSB (with more focus on enhancements than on potential benefits), ETRI, Xiaomi, WILUS, Intel |
| Not support | Ericsson, NEC, OPPO |

| Company | Comments |
| --- | --- |
| Ericsson | It is unclear what "initial access in an UL subband means" We think the higher level issue is whether or not it is beneficial that SBFD capable UEs are aware of SBFD symbols in IDLE mode. That being said, we think the priority of this SI should be on CONNECTED mode, not IDLE mode. |
| Fujitsu | Although we are open to study on this topic, but it might be needed to consider at least two things as below - CLI between legacy UE and SBFD aware UE - Possible to be duplicated discussion with NR_Cov_Enh2 in terms of coverage enhancement |
| Sharp | We are supportive of the proposal. On the other hand, the key issue would be whether we support PRACH on UL subband in SBFD symbols. |
| vivo | SBFD operation should prioritize RRC connected UEs |
| Nokia, NSB | We think this has been discussed for several meeting and the benefits for introducing initial access on SBFD slots are quite clear (increased RACH capacity, reduced initial access delay, etc.). So, the agreement/proposal should focus on studying the enhancements needed to support initial access procedure on SBFD slots rather than on studying potential enhancements. |
| NEC | We think that there are not significant benefits which can be achieved by supporting SBFD for initial access. We should concentrate efforts of this release in stabalizing enhancements for connected mode operation. |
| OPPO | Similar comment as from Ericsson. This “initial access” here should be non-contention based in RRC_connected state, otherwise, either - SBFD operation from UE perspective needs to rely on UE basic capability; OR - Most of SBFD operation feature on UE side need to be mandatory without UE capability reporting. Nevertheless, only supporting initial access in RRC_Connected state does not seem to improve the coverage which is bottle-necked by initial access in RRC_IDLE. So it is more appropriate to seek coverage improvement on PRACH from other ways, e.g., using more conventional contiguous UL symbols/slots, given quite some UL transmissions are moved to UL subband in DL symbols. |

  - [Closed] 2nd round discussion

Proposal 1-1

Proposed Agreement:

Confirm the following working assumption in RAN1#110.

For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline.

- FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies

- FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair

|  | Company |
| --- | --- |
| Support | Ericsson, New H3C, Lenovo, ZTE, Xiaomi, TCL, Spreadtrum, ITRI, vivo，CMCC, NEC, Sony, WILUS, CATT, DOCOMO, KDDI, Samsung, CEWiT, Panasonic, Qualcomm, Nokia, NSB, Fujitsu, Sharp, Intel |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | In 1st round discussion, majority companies support to confirm the WA. Althoug some companies prefer to not study BWP pair with unaligned center frequency or SBFD scheme with multiple BWP pairs, it seems that majority companies can live with the FFS. Some companies commented that the proposal is not clear. From moderator’s point of view, the wording of the proposal was extensively discussed in the last meeting. I am not sure how to further improve the wording and encourage companies to provide suggestions if they see the need. |
| Ericsson | While its okay to onfirm the WA, is it really necessary to take two steps considering Proposal 1-2a? Why not simply try to agree on Proposal 1-2a first. This will effectively confirm the main bullet of Proposal 1-1. The FFSs are then still alive (for those companies interested in addressing them (not us)). |
| ZTE | We are ok with confirm the working assumption with the understanding that the following two questions will be clarified separately. - Whether the UL subband has to confined within the UL BWP? From our perspective, it is reasonable to assume that UL subband has to be within UL BWP. - Whether the UL subband has to confined within the DL BWP, e.g., DL BWP = 10M, UL BWP=40M, UL subband=20M? From our perspective, it is not reasonable to assume that UL subband has to be within DL BWP. |
| Xiaomi | We tend to agree with Ericsson that we can directly discuss propsal 1-2a. If proposal 1-2a is agreeable, we don’t need to discuss proposal 1-1. |
| Spreadtrum | We do not see any benefit of unaligned center frequency for UL and DL BWP pair, we can live with them as FFS. |
| vivo | We share with Ericsson’s views to directly go to proposal 1-2a. |
| NEC | We generally do not support using unaligned center frequencies for BWP pair but we can go ahead with the proposal for progress sake |
| Sony | We are one of the companies who do not like the unaligned BWP (the FFS) but can live with them for the moment. |
| KDDI | We prefer to keep the FFS part at this stage. Using smllaer BWP than entire bandwidth can reduce inter-UE CLI impact, thus contributing to the feasibility of SBFD. |
| Samsung | @ZTE: We share your understanding, e.g., reasonable to assume that the UL SB configured for the UE is confined within the UE UL BWP, but not reasonable to assume that that UL SB has to be confined within the UE DL BWP. That would be for us subject to discussion in the group and follow-up agreements. |
| QC | We agree with Moderator views on confirming the working assumption as is given it was discussed extensively over two meetings. |
| Nokia, NSB | Agree with Ericsson that it is not clear why we need two step approach with both Proposal 1-1 and Proposal 1-2a. We could have one proposal: “For SBFD operation within a TDD carrier, support it is agreed that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies asis the baseline. - FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies - FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair” |
| Ericsson2 | One further comment based on some of the above comments: our understanding is that UL and DL subbands will be configured per serving cell, and bandwidth parts come on top of that (as in Rel-15). Hence, to us, it does not make sense to talk about UL subband configured/confined within an UL BWP. Rather the discussion should be about whether or not an UL BWP is restricted to span only the UL subband. In our view, the UL BWP can span wider than the UL subband (e.g., whole carrier) so that no BWP switching is needed when going between SBFD symbols and UL-only symbols (see diagram below). In this diagram, even though the UL BWP spans the whole carrier, it will be needed to specify that the UE is not expected to transmit outside the UL subband in SBFD symbols. |
| InterDigital | Okay to confirm the WA as is. One comment on Ericsson’s view above: We don’t think the UL subband will be configured only associated with a CC level, but we think the baseline can be the UL subband can be configured with a BWP level (by UE-dedicated RRC), because this SBFD feature is a new feature and it is natural to configure this new feature along with a BWP-level configuration. |
| Intel | For ZTE’s question, in our understanding, 1st question: We share similar view with E///. If UL subband is configured by cell-specific signaling, it is possible that a UL subband partially overlaps with a UL BWP for a UE. For example, D:U:DL =40:20: 40MHz, it is possible that a UL BWP is only 10MHz so UL subband is not confined with the UL BWP. Would companies kindly explain why it is necessary to restrict UL subband within a UL BWP? Though UE or cell-specific configuration is not discussed now, it does not make sense to assume UE specific configuration (different UE has different UL subband size) as natural, and it does not make sense to assume it is BWP-based configuration even if it is UE specific configuration. 2nd question: UL subband is not limited within DL BWP. |
| Moderator | Based on the above feedback, majority companies are fine to confirm the WA as it is. Ericsson, Xiaomi, vivo and Nokia think that there is no need to confirm the WA and instead we can directly go to proposal 1-2a which confirms the main bullet of the WA while keeping the FFS open. It makes sense to me so that proposal 1-1 is no longer pursued in this meeting. Companies can continue discussing the FFS points in the future meetings. |

Proposal 1-2a

Proposed Agreement:

For SBFD operation within a TDD carrier, support it is agreed that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies asis the baseline.

|  | Company |
| --- | --- |
| Support | Ericsson, New H3C, Lenovo, ZTE, Xiaomi, TCL, Spreadtrum, ITRI, vivo，CMCC, NEC, Sony, WILUS, CATT, DOCOMO, Samsung, CEWiT, Panasonic, Qualcomm, Nokia, NSB, Fujitsu, Sharp, Intel , Huawei, HiSilicon |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | The proposal is related to previous proposal. The proposal is updated to avoid using “support”. Companies are encouraged to provide your suggestions on the wording change if you see the need. |
| ZTE | Similar comments as for proposal 1-1. We support this proposal as long as the two questions we raised in proposal 1-1 can be discussed separately. |
| Xiaomi | As we proposed in first round discussion, we would like to clarify the UL subband allocation with the proposed proposal: Is there any restriction on the UL subband configuration with this proposal? For example, UL subband can only be configured within the frequency range of UL BWP, or within the frequency range of DL BWP, or no restrictions within an activated carrier? |
| Spreadtrum | According to Xiaomi’s comments, we think UL subband should be within UL BWP with this proposal. |
| vivo | Our understanding of above proposal does not involve the UL subband configuration details, e.g. UL subband is configured within a serving cell or within a BWP. |
| CMCC | We also think the UL subband should be confined within UL BWP |
| Sony | On Xiaomi’s comments, this depends on whether we can have subbands configurations in symbols that were originally designated as UL since in the legacy system, the UE is supposed to be in the UL BWP during an UL symbol. Since we decided such case should be of lower priority, then for the moment we assume that the subbands are configured within DL BWP, i.e. symbols that are originally designated as DL. |
| Samsung | @ZTE, @Xiaomi: our thoughts on your question are shared in 1-1 (Round 2). |
| Nokia, NSB | See our comments to Propopsal 1-1. |
| OPPO | The key intention of the proposal seems to highlight both “single pair of BWP” and “aligned center frequencies”. For “single pair of BWP”, we have an earlier question in 1st round: whether the proposal makes the SBFD scheme in the following figure is the baseline (if UL subband is defined in relative to BWP so that the UL subband is associated with either BWP0 or BWP1 but not both) or not the baseline (if UL subband is defined in relative to carrier, e.g. relative to common RB #0 and it falls into more than one UL BWP, BWP0 and BWP1)? Meanwhile, we are a bit hesitated to define UL subband in relative to BWP, since that means the dynamic switch of BWP leads to dynamic switch of UL subband, even in the case where RAN1 cannot agree dynamic indication of UL subband. It seems to us that some concepts need to be clarified first before reaching “a single pair” baseline. |
| Moderator | Based on the above feedback, majority companies are fine with the proposal. Regarding the questions from ZTE and xiaomi, my understanding is that it is related to the detailed methods for subband location indication. For example, if it is configured per CC, then as commented by Ericsson and Intel for Proposal 1-1, the UL subband does not necessarily to be confined within UL subband. But if it is configured per BWP, it may make sense that UL subband is within UL BWP. Regardless, those details will be separately discussed later. For the question/comments from OPPO, I am not sure if I understand correctly. The figure is from CATT’s contribution, which illustrates an example of configuring UL subband with relative to carrier. As commented above, that is second-level detail and would be discussed later. With the above clarification, I assume Proposal 1-2a is stable and please indicate if you still have comments. |
| Huawei, HiSilion | On the relation between UL subband and UL BWP, the controversial point seems to be that companies have different views on how subband is defined or configured. Our view is that subband is defined per carrier. Therefore, UL subband and UL BWP can still be partially overlapped as also pointed out by the intel. And for othe configuraton of {DU} as illustrated below, whether the center frequency of the BWP is aligned or not may need to be discussed. So we prefer to confirm the WA as it is. |

Proposal 1-5b

Proposed Agreement:

For indication of subband locations for SBFD operation for SBFD aware UEs, it is agreed that semi-static configuration of subband time and frequency location is the baseline.

- Dynamic indication of frequency location and/or size of SBFD subband is not supported further considered

- FFS Study potential benefits and signaling impacts to support dynamic indication/update of time location of SBFD subband

|  | Company |
| --- | --- |
| Support | Ericsson (main bullet + 1st sub-bullet), New H3C with modification, Lenovo (only support the main bullet), ZTE Xiaomi(main bullet + 1st sub-bullet), TCL (with modification), Spreadtrum, vivo (modifications), NEC, WILUS, CATT, DOCOMO, Samsung (only main bullet), CEWiT, Panasonic, Qualcomm, Nokia, NSB, Sharp, Intel |
| Not support | Sony |

| Company | Comments |
| --- | --- |
| Moderator | Proposal 1-5b combines proposal 1-4a and 1-5a in section 5.1. I understand companies have different preferences on the support/study of dynamic subband location indication. Some companies prefer dynamic subband location indication in both time and frequency domains and some other companies do not see the need of dynamic subband location indication at all. Many companies have concerns on dynamic change of frequency location and/or size of SBFD subband due to the complexing in implementation, CLI handling and think the resource adaptation can be achieved in time domain, not necessarily need to be in frequency domain. Considering the different views, I think the current proposal is the best way forward at this point. |
| Ericsson | We still have concerns on the 2nd sub-bullet, and we prefer to remove it. We think Companies are still free to contribute on dynamic indication of time location in future meetings if they wish. My questions from the 1st round were not addressed. What does dynamic indication/update of time location of a subband actually mean? Does it simply mean that the Tx direction of an existing semi-statically configured subband is dyamically indicated/updated, or does it mean something else? Also, how are the dynamic and semi-statically configured time locations related? I think we need to have this discussion, since I believe that not all companies are on the same page on what dynamic indication of time location means. |
| New H3C | We still have concern on 1st subbullet and the reason why we can’t investigate on dynamic indication on frequency domain without any evaluation results isn’t clear to us . SO we suggest the modified proposal as below: Proposed Agreement: For indication of subband locations for SBFD operation for SBFD aware UEs, it is agreed that semi-static configuration of subband time and frequency location is the baseline. - Dynamic indication of frequency location and/or size of SBFD subband is not supported further considered - FFS Study potential benefits and signaling impacts to support dynamic indication/update of time/Frequency location of SBFD subband |
| Lenovo | Within a semi-statically configured frequency location, 2 subband sizes can be configured (i.e. one subband is within another subband), and MAC CE can indicate which subband size is an active subband size. This enables UL subband size adaptation for varying UL traffics, with reduced signaling (no RRC reconfiguration). And yet, interference coordination across neighboring gNBs is feasible in a time scale of 10~20ms. |
| ZTE | Similar view as morderator above, we think the current proposal is a good way forward. |
| Xiaomi | We share same views with Ericsson. We need to firstly make sure everyone on the same page regarding dynamic indication of time location of SBFD subband. Besides the issues raised by Ericsson, the relation between dynamic SBFD subband indication and SFI also needs to be clarified. All in all, the operations would be complicated while the benefits is questionable. |
| TCL | In our view, it is too early to preclude the dynamic indication of subbands frequency location, without any solid reasons. In addition we share similar view with Lenovo on “MAC CE can indicate which subband size is an active subband size”. Therefore we suggest to consider the New H3C revised version for this proposal, in order to investigate the benefits of dynamic indication of subbands frequency locations further. |
| Spreadtrum | We are fine with Ericsson’s proposal. |
| vivo | We think better to discuss what is the preferred operation from gNB perspective. To reply Ericsson and Xiaomi, from our understanding, the intention for the dynamic indication/update of time location of SBFD subband means following: - SBFD symbols are defined as the symbols where the configured DL and/or UL sub-bands apply. - A DL symbol that is semi-statically configured with UL subband can be dynamically determined by gNB to use as legacy DL symbol, or SBFD symbol. - A flexible symbol that is semi-statically configured with UL subband can be dynamically determined by gNB to use as legacy DL symbol, UL symbol or SBFD symbol. - A UL symbol that is semi-statically configured with DL subband can be dynamically determined by gNB as legacy UL symbol, or SBFD symbol as 2nd prioriy. To Xiaomi, we think it is better to first study the SBFD operation in semi-static DL symbol, semi-static flexible symbol and semi-static UL symbol for simplicity, then later on study the interaticon with dynamic SFI. To avoid confusion and as compromise to not consider dynamic change the subband size in frequency location, we suggest following modification: For indication of subband locations for SBFD operation for SBFD aware UEs, it is agreed that semi-static configuration of subband time and frequency location is the baseline. - Dynamic indication of frequency location and/or size of SBFD subband is not supported further considered - For a symbol configured with DL and/or UL subband, it is beneficial for gNB to dynamically determine whether to apply the DL and/or UL subband in the symbol. - FFS Study potential benefits and signaling impacts to support dynamic indication/update of time location of SBFD subband |
| CMCC | Reading companies’ comments, the meaning of dynamic indication/update of time location of SBFD subband at least including two interpretations: 1) change the link direction of certain UL subband to DL reception or cetain DL subband to UL transmission, i.e., UE has been confiued to transmit in UL subband in slot n, but gNB further change the UL subband into DL reeption 2) indicate the link direcetion of uncertain SBFD symbols, i.e., for slot n, UE can be dynamically change the link direcetion to UL or DL, if gNB indicates the link direcetion is UL, UE will transmit in the UL subband, ortherweise, gNB indicates the link direcetion is DL, UE will receive in the DL subband. From our point of view, these two interprataions can both be supported. Another comment is that the meaning of “subband locations for SBFD operation” in the main bullet, does it means both DL subband(s) and UL subband for the same SBFD symbol is configured to UE or only DL subband(s) or UL subband is configured to UE? |
| NEC | Although agreeing with Ericsson’s view that more details need to be provided for dynamic time indication, we still think that the proposal is okay as it just mentions to study the dynamic time indication for SBFD subband. The details for the same can be discussed once companies start to provide their solutions for this. However, we also would like to note that there has already been discussion whether DL transmission can be performed on UL subband, which can be used for introducing dynamic scheduling decisions from gNB perspective (e.g. configuring UL subband almost on all slots and then reutilising some of these slots for performing DL transmission dynamically based on traffic requirements). From our understanding we only need one solution for dynamic scheduling i.e. either dynamic DL scheduling on UL subband or dynamic UL subband indication, because both can produce similar effect. So, for the long term it would be preferable to understand how much benefit “dynamic UL subband indication” is providing as compared to “dynamic DL scheduling in configured UL subband”. |
| Sony | We have similar views with E///, New H3C, Lenovo and TCL, that we do not think we need to kill off dynamic frequency indication for the subbands. If we are happy to consider dynamic time location indication, why can’t we consider frequency? |
| WILUS | For the dynamic indication/update of time location, our understanding is that a semi-statically configured subband is dyamically indicated/updated to be adopted in certain time resources, e.g., indicate/update from {D-U-D} to {D} without dynamic indication of freq. location and/or size. Thus, freq. location and/or size of subbands are only semi-statically configured, while it can be indicated in different time locations. |
| CATT | From our perspective, we do not see the need of dynamic indication of subband time location. But we can live with the current proposal. |
| DOCOMO | We support the proposal. And we share same understanding as vivo on intention for the dynamic indication/update of time location of SBFD subband. |
| Samsung | We support the main bullet only. Regarding “dynamic indication/update” (1st and 2nd sub-bullet), it is not very clear what is meant. We should clarify. Our current understanding is below: The SBFD UL SB is configured by RRC for the UE. The UE knows the OFDM symbols and the RBs on these symbols of the SBFD UL SB. There is 1 such RRC configured SBFD UL SB for the UE. Dynamic, e.g., DCI-based, scheduling is the basic principle of system operation which we preserve. In any SBFD slot, the UE cannot be UL scheduled by DCI outside the RRC configured UL SB. In the nUL slot, the UE can be scheduled by DCI inside its UL BWP. If the UE can only be UL scheduled by DCI inside the RRC configured SBFD UL SB or if the UE can possibly be scheduled either DL or UL inside the UL SB is still open, e.g., Options 1 and 2 from RAN1#110. In our view this is in principle sufficient. We understand from some company proposals that there could be multiple (2, 4) RRC configured SBFD UL SBs per UE. Multiple RRC configured SBFD UL SBs per UE possibly differ in their time-domain symbol allocations and/or the configured RBs on an SBFD symbol. MAC CE based (de-)activation signaling then would select one of the multiple RRC configurations and the DCI-based dynamic scheduling then happens with respect to the activated SBFD UL SB of the UE. In addition, there are some company proposals to extend the SFI functionality. Even if the SBFD UL SB is configured by RRC for the UE, which SBFD symbols are then “schedulable” for the UE by means of DCI is signaled through extended SFI. In this sense, “dynamic indication/update of the SBFD subband” doesn’t appear meaningful to us. Even with SBFD, the actual OFDM symbol and RB allocations on SBFD symbols are always dynamically allocated to the UE using TDRA and FDRA in the UL or DL scheduling DCIs. New constraints exist with SBFD, e.g., UL scheduling only inside the configured UL SB on the SBFD symbol. SFI or MAC-CE based (de-)activation are additional signaling mechanisms ön top”. Would it make sense then to capture tthese in the following 2 FFS instead? - FFS: Multiple semi-static SBFD time/frequency configurations per UE and/or MAC-CE based (de-)activation of SBFD subband(s) - FFS: Applicability of and/or enhancements to Rel-15 SFI signaling for the case the SBFD subband is configured for the UE |
| QC | In our views, the forward steps of the proposals are in the subbuellts. As discussed online, it is common understanding that semi-static configuration is the baseline. We believe that at least dynamic SBFD time indication is beneficial to study. We don’t agree with companies comment to remove or FFS the subbullet. Then, how companies would study dynamic SBFD update if there is no study agreement. It is okay to update the proposal to clarify/define what is meant by dynamic SBFD update. In our views, it means that a semi-static configured SBFD symbol is dynamically updated to non-SBFD symbol. On the first subbullet, we think that further study is needed. So, FFS should be more appropriate. |
| Nokia, NSB | We think it is clear from previous discussions what dynamic indication/update of time location of a subband means. It means that a symbol/slot that is semi-statically configured as DL (or SBFD) can be dynamically indicated to be a SBFD (or DL) symbol/slot, respectively. In other words, whether the UL subband is present in a symbol/slot can be indicated dynamically to the UE. It can be studied whether it is enough to dynamically indicate this via the scheduling DCI (i.e. Option 2 discussed section 2.1.2.6), or some cell-specific signaling is more beneficial. However, based on large support in 1st round discussion, we think that ‘study dynamic indication/update of time location of a subband’ must be somehow captured in the agreement. |
| Ericsson 2 | Based on QC's and Nokia's comments (other companies may have different interpretation), it seems that "dynamic indication/update of time location of SBFD subband" means, for example, that a symbol is semi-statically configured as D-U-D, and then it can be dynamically indicated/updated to be D-D-D, i.e., DL-only (e.g., through some extension of SFI or by some sort of MAC-CE). To us, this is Option 2 in the RAN1#110 agreement. - Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol If the above is indeed the common understanding, we can be open to discuss D-U-D D-D-D (i.e., conversion to DL-only) as an FFS; however, the 2nd bullet of the proposal must be reworded to clarify the meaning of "dynamic indication/update of time location of SBFD subband." However, we are not open to discuss conversion of DL-only to SBFD (DL-only D-U-D), i.e., a symbol is semi-statically configured as DL-only, and then it is dynamically indicated/updated to be D-U-D. This means that the gNB would need to dynamically re-configure its analog filter in the Rx chain prior to the ADC. This is undesirable from an implementation perspective, and not feasible in some gNB Rx architectures, e.g., implementations based on direct RF sampling. In our view, this kind of conversion (DL-only D-U-D) should be done by RRC re-configuration rather than dynamically. |
| MediaTek | We don’t support the sub-bullets. The semi-static configuration of the subbands is not yet clear, so there is no baseline to compare with regarding if dynamic indication is needed or not. We suggest to postpone the discussion for dynamic indication after we have some details on the semi-static configuration. |
| InterDigital | We share similar views with New H3C, Lenovo, TCL, vivo, WILUS, DOCOMO, and Qualcomm. We support the revised proposal by New H3C. |
| OPPO | We certainly support the main proposal and the 1st sub-bullet. For the 2nd sub-bullet, we prefer to remove it totally (since it is a kind of FFS anyway) if its presence locks up the progress. But if companies cannot agree the whole proposal without keeping dynamic time indication in sight, we think a fair middle ground can be the following: - FFS Study potential benefits and signaling impacts (as 2nd priority) advantages and disadvantages to support dynamic indication/update of time location of SBFD subband Here “2nd priority” is to echo “baseline” in the main proposal as well as the “baseline” in Proposal 1-6a. |
| Intel | We’re generally fine with the proposal, but we'd like to delete 'update' for 2nd sub-bullet. In our understanding, to update time location, it also uses dynamic indication, so it is sufficient just to say dynamic indication without 'update'. We further suggest adding ‘implicit or explicit’ before dynamic indication to cover the case whether a separate explicit indication for time location of UL/DL subband or implicit indication by scheduling DCI as mentioned by CMCC. For indication of subband locations for SBFD operation for SBFD aware UEs, it is agreed that semi-static configuration of subband time and frequency location is the baseline. - Dynamic indication of frequency location and/or size of SBFD subband is not supported further considered - FFS Study potential benefits and signaling impacts to support explicit/implicit dynamic indication/update of time location of SBFD subband |
| Moderator | The views are still divergent. Samsung do not agree with the sub-bullets. MediaTek suggest discussing dynamic indication later. New H3C, Lenovo, TCL, Sony, QC, IDC would like to study dynamic frequency location indication. Ericsson, ZTE, Xiaomi, Spreadtrum, vivo, CMCC, NEC?, WILUS, CATT, DOCOMO, Nokia? , OPPO and Intel agree to not further consider dynamic frequency location indication. In addition, some clarifications on what “dynamic indication/update of time location of SBFD subband” were discussed. Ericsson commented that they are open to discuss D-U-D D-D-D (i.e., conversion to DL-only) but not open to discuss DL-only to SBFD (DL-only D-U-D). Intel proposed to add “explicit/implicit” to cover the possibility of implicit indication by scheduling DCI, which I assume is what Samsung has in mind. Based on the discussions, I still think the current proposal is the best way forward. Otherwise, we cannot achieve any agreements to study dynamic indication of subband location. I tried to update the proposal for the second sub-bullet below. Proposed Agreement: For indication of subband locations for SBFD operation for SBFD aware UEs, it is agreed that semi-static configuration of subband time and frequency location is the baseline. - Dynamic indication of frequency location and/or size of SBFD subband is not supported further considered - FFS Study potential benefits and signaling impacts to support explicit/implicit conversion from SBFD symbol to DL-only symbol dynamic indication/update of time location of SBFD subband |
| Huawei, HiSilicon | We think the latest proposal from the moderator is too restrictive. It seems to imply that SBFD operation can only be done on DL symbols by configuring UL subband on DL symbols. We think Intel’s version is more generic. Furthermore, this discussion is also related to whether the subband in the previous slots used for UL can be used for downlink transmission in the upcoming slots? |
| vivo | What is DL-only symbol and why the SBFD symbol can only be converted to DL-only symbol? We agree that for UL suband configured in the DL symbol, then the SBFD symbol can fallback to DL symbol; But for semi-static flexible symbol configured with UL subband, it can fallback as DL, UL and SBFD symbol. As also comment in the email, we suggest first clarifying that the SBFD operation can be configured in DL/flexible symbols provided by tdd-UL-DL-ConfigurationCommon. |
| OPPO | For the last bullet in the latest proposal from FL, we understand the intention of {DUD}->{DDD} is to allow the gNB to use resources in UL subband when the UL subband is not actually used for any UL transmission. Given now the “conversion” is one-way only, does it mean once the UL subband is borrowed for DL reception, the UL subband configuration is gone on the symbol? It looks like a dynamic event (i.e., a change-mind of gNB scheduling) can invalidate a semi-static configuration, which is not a good logic in our view. A better logic than the above is to let the symbol automatically returns to {DUD} once the event causing the conversion is over, to revive the semi-static configuration. On the other hand, this conversion-plus-auto-return mechanism looks an over-weighted mechanism for a purpose of just using UL subband instantaneously for DL reception. So our preference is to step back a bit from symbol conversion and to just mention the purpose of the study, e.g., - FFS Study potential benefits and signaling impacts to support using resources overlapping with UL subband for DL transmission/reception |
| CMCC | As the comment above, there are several interpretations of dynamic indication/update of time location of SBFD subband, at least including: 1) change SFBD symbol to normal DL symbol 2) dynamic inidate the link direction of SBFD symbol, i.e., whther UE transmits in UL subband or receives in DL subband on a SBFD symbol The moderator’s proposal only captures the first interpreatation, which is not fair to the second intepretation |

Proposal 1-6a

Proposed Agreement:

For semi-static configurationindication of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations are explicitly indicated within a period is the baseline.

- FFS the periodicity is explicitly indicated or implicitly determined

- FFS whether the SBFD subband time locations are contiguous within a period

- FFS multiple periods are supported

|  | Company |
| --- | --- |
| Support | Ericsson, New H3C, Lenovo, ZTE, Xiaomi, TCL, Spreadtrum, ITRI, vivo, CMCC, NEC, Sony, WILUS, CATT, DOCOMO, Samsung, CEWiT, Panasonic, Qualcomm, Nokia, NSB, Fujitsu, Sharp, OPPO, Intel, Huawei, HiSilicon |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | The proposal is updated by removing the FFS and add “semi-static” in the proposal according to the comments from companies. |
| Moderator | All companies agree with the proposal so the proposal is assumed to be stable for approval. |
| CMCC | We support this proposal in principle, but the current wording seems a little ambiguity, e.g., (1) the subband time locations only includes one subband type, i.e., UL subband that is UE can only perform UL in the configured UL subband time or (2) the subband time locations include both UL subband and DL subband, i.e., gNB configures a time window to UE which can perform SBFD operation in this time window, but whether the link direcetion of each symbol is UL or DL is separately configured or indicated. Considring we have already defined the SBFD symbols as the following, we suggest to replace subband time locations to SBFD symbols Conclusion For discussion purpose only, SBFD symbols is defined as symbols with subbands that gNB would use for SBFD operation. CMCC’s version: For semi-static configurationindication of SBFD symbols subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD symbols SBFD subband time locations are explicitly indicated within a period is the baseline. |

Proposal 1-8a

Proposed Agreement:

The maximum number of subbands for SBFD operation in an SBFD symbol within a TDD carrier is three for the study in RAN1.

- Note: Support of three SBFD subbands may subject to RAN4’s study and conclusion

- Note: Support of SBFD subband pattern {UDU} is 2nd priority as per RAN guidance.

|  | Company |
| --- | --- |
| Support | Ericsson, New H3C, Lenovo, Xiaomi, TCL, Spreadtrum, ITRI, NEC, Sony, WILUS, CATT, DOCOMO, Samsung, CEWiT, Panasonic, Qualcomm, Nokia, NSB, Fujitsu, Sharp, MediaTek, Intel |
| Not support | ZTE |

| Company | Comments |
| --- | --- |
| Moderator | The proposal was briefly discussed online. OPPO has concerns on DL subband. Actually, it is not clear to moderator what is wrong with DL subband. Moderator would like to encourage OPPO and other companies to share more views. |
| Ericsson | We share the moderators view; we do not understand what is the problem with referring to "DL subband" Just a question: Do we really expect RAN4 to tell RAN1 that 3 subbands is or is not feasible? |
| New H3C | We wonder why we can’t use this word”DL subband”. |
| ZTE | There are several issues mixing together in this proposal. The first issue is whether to define/have DL subband. From our perspective, it is ok to have DL subband if DL subband is within DL symbols/flexible symbols. For example, in case of DUD, there will be two DL subbands. The second issue is whether to support both DUD and DU/UD pattern. We noticed that some companies have concern on DUD. From our perspective, we are ok to have both DUD and DU/UD. The third issue is whether to support UDU pattern. RANP guidance clearly says this should be 2nd priority. Now the high-level design for DUD, DU/UD is not clear yet, we don’t think it is the right time to introduce UDU at this stage. In addition, UDU will introduce lots of new discussion points that require lots of energy. For example, we have multiple discussion points listed in the FL summary for non-contiguous DL for DUD. If UDU is introduced, RAN1 needs to discuss all these aspects for non-contihous UL for UDU. If companies still have different understandings on this FL proposal, we would suggest our previous proposal again. Proposal: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one. |
| TCL | The SBFD pattern in second note {UDU} may restrict the gNB implementation to use {UUD} or {DUU} pattern. In our view DL and UL subbands words may be more appropriate to use instead of specific subbands pattern. |
| vivo | RAN1 discussed two SBFD pattens in freq. domain in the semi-static DL symbol {D U}/{U D} and {D U D}, the two patterns have different impacts for resource allocation. Maybe we can first ask RAN4 which one is preferred from co-existence perspective. If RAN4 did not see the difference between the two patterns in the DL symbols, then {D U} or {U D}would be better choice; on the other hand, if RAN4 replies in some cases, {D U D} is better, then we can support both patterns. |
| CMCC | We think we should say which SFBD patterns can be supported, e.g., {DUD}, {DU}, {UD} not just saying the number of subbands. |
| NEC | We also think that round-1 proposal was more clear which used UL suband and DL subband notation. |
| Sony | Support the proposal. We were also puzzled why OPPO dislike the term “DL Subband” during the GTW session. We do not think it is an issue to use the term “DL subband”. I believe what is more important is where (i.e. what type of OFDM symbols) the subbands can be configured in. |
| Samsung | We are in principle fine with the agreement, e.g., not more than 3 subbandson an SBFD symbol. Ideally, we can make a bit more progress than that. We think ZTE and CMCC make a good point. For SBFD using the legacy D/F symbols, we can have {DUD}, {DU}, {UD}. We should also distinguish the cases of SBFD subband configuration from gNB versus UE perspective. From gNB-perspective {DUD} vs. {DU} entirely depends on deployment choice and the operator band segment. It will be either one or the other. From UE perspective, it is not a must to configure the entire gNB {…U…} as UE UL SB. If the SBFD UL SB is 20 MHz wide and center channel from the gNB perspective due to deployment needs, the UE1 can still be configured with a 10 MHz UL SB in the upper portion and the UE2 with a 10 MHz UL SB in the lower portion of the gNB total 20 MHz {…U…}. That would be done through UE-specific signaling. According to us, no need to introduce a {D U U D} case for that. |
| Nokia, NSB | We generally agree with the proposal. In our view, the only configurations that make sense are: D-U, U-D and D-U-D. To reply to Ericsson’s question, we would like to receive indication from RAN4 about the actual benefits (in terms of reduced UE-2-UE and gNB-2-gNB adjacent-channel CLI) by placing the UL subband in the middle of a TDD carrier (i.e. D-U-D configuration). As several resource allocation enhancements discussed in the SI are due to D-U-D SBFD configuration, we would like to understand if the added specification/processing complexity and limitation on resource utilization would result in a tangible gain in real deployments. |
| OPPO | The proposal seems to suggest (via 2nd note) that, "DL subband" may offer a side benefit to spend less RAN1 efforts for potential support of "DL subband in UL symbols (2nd priority by RANP)". However, this sharing use of terminology of concept between "implicit leftover DL RBs in DL symbol after taking UL subband and/or guardband away" and "explicit DL subband in UL symbols" might not be a good path in our view. For example, if subband is defined in relative to carrier (i.e., relative to Common RB index 0), could the DL subband in DL symbol (left over DL RBs) be defined relative to BWP or still relative to carrier? - From RAN1’s interest, the "leftover RB" seems to refer to “leftover within BWP” since the scheduling occurs in BWP, then the DL subband in UL symbols (relative to carrier) and DL subband in DL symbols (relative to BWP) would observe essential difference but under the same conceptual name, which is certainly not desirable. So if people still want to reserve the chance for DL subband in UL symbol, it seems safer to use a different name for DL leftover RBs in DL symbol. - If the "leftover RB" seems to refer to “leftover within carrier”, we agree this can be a right terminology in RAN4, but just wonder how this DL subband is used in RAN1, since most part of RAN1 spec procedures deal with BWP. Another reason for us to less like “DL subband” at this time is that RAN1 does not have a clear definition of “DL subband” for symbols holding an UL subband. The proposal 1-8a says a frequency map of {DUD} by providing DL subband, but there can be another equivalent map of {D…DUD…D} by also using DL subband – something like what the following figures show, where the left figure is what the proposal says, while the right one is equivalent from resource allocation perspective but does not match the proposal. Both figures allow explicit definition of DL-subband; and both can work with implicit definition as well with the difference that the implicit definition of DL subband is based on single UL subband in left figure and likely multiple UL subbands in the “period” in right figure. This is just one example on how the definition of DL-subband can impact the proposal. does RAN1 define the DL-subband as a parallel concept with UL subband or define the DL subband based on definition of UL suband? It is all vague so far. At last, the RAN #96 conclusion reads the following and does not say “{UDU} is 2nd priority”: “conclusion: UL symbol as 2nd priority is accepted, no intended suspension of continuation of work in WGs” The proposal seems to allow {UDU} on DL symbol as well but just as a 2nd priority. We do not support this implication. |
| Intel | Since the main butllet is ‘for the study in RAN1’, we think it is better to use ‘study’ rather than ‘support’ for both notes. |
| Moderator | It seems that OPPO has some potential complicated designs in mind. I assume even if the example provided by OPPO is supported, what companies have in mind is the design shown in the left figure. I am not sure whether any company is interested in the design in the right figure. Anyway, I am wondering whether we can try another way around, inline with the proposal from ZTE and vivo. Proposed Agreement: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one three for the study in RAN1. - The UL subband can be located at one side of the carrier. - The UL subband can be located at the middle part of the carrier - Note: Support of three SBFD subbands may subject to RAN4’s study and conclusion - Note: Support of SBFD subband pattern {UDU} is 2nd priority as per RAN guidance. |
| Huawei, HiSilicon | We are fine with the latest proposal from the moderator. |
| OPPO | To clarify, we do not have preference between the two figures in our earlier response. The figures are just examples showing different DL-subband concepts may impact the proposal itself. We support the latest FL proposal, just one minor thing: now “subject to RAN4 study and conclusion” should not apply to the main proposal of “up to one UL subband”, but to the location of UL subband (side of carrier vs. middle part). So maybe the wording can be: Proposed Agreement: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one three for the study in RAN1. - The UL subband can be located at one side of the carrier or the middle part of the carrier, subject to RAN4’s study and conclusion - Note: Support of SBFD subband pattern {UDU} is 2nd priority as per RAN guidance. |

Proposal 1-9

Proposed Agreement:

SBFD aware UE does not transmit UL channel/signal outside the UL subband in SBFD symbols.

- FFS UE behaviour if a scheduled/configured UL transmission overlaps with DL subband(s) and guard band(s) if configured

|  | Company |
| --- | --- |
| Support | Ericsson (main bullet), New H3C with modification, Lenovo (main bullet), ZTE, Xiaomi, TCL, Spreadtrum, ITRI, CMCC, NEC (with correction), Sony, CATT, DOCOMO, Samsung (main bullet only), CEWiT, Panasonic, Nokia, NSB, Fujitsu, Sharp, Intel |
| Not support | vivo, Qualcomm, MediaTek |

| Company | Comments |
| --- | --- |
| Moderator | In 1st round discussion, several companies mentioned flexible subband. But in the contributions, I see limited inputs so I am not sure what flexible subband companies have in mind. For example, whether the flexible subband refers to the remaining RBs other than the RBs for UL subband and guardband in semi-static flexible symbols, or a dedicated frequency resource indicated as flexible, which would be indicated as DL or UL later by other signalling? I hope companies can provide more details for my understandings. Thanks. |
| Ericsson | We support the main bullet. We do not support the sub-bullet. If the UE is allowed to transmit UL outside the UL subband, it means that that the gNB must adapt its analog filter in the Rx chain prior to the ADC. This is undesirable from an implementation perspective, and not feasible in some gNB Rx architectures, e.g., implementations based on direct RF sampling. Furthermore, we find the sub-bullet confusing – it seems to contradict the main bullet, and it seems to relate to Options 3 and 4 in the RAN1#110 agreement shown below. It seems many (most) companies prefer not to further consider those options. - Option 3: The SBFD aware UE does not expect to be scheduled with DL reception within the UL subband and may be scheduled with UL transmission outside the UL subband in the SBFD symbol - Option 4: The SBFD aware UE may be scheduled with UL transmission outside the UL subband or DL reception within the UL subband in the SBFD symbol |
| New H3C | We need clarify a little bit on what is resource attribute outside the UL subband in SBFD symbols. If resource attribute outside the UL subband in SBFD symbols is DL, above proposal is fine. If resource attribute outside the UL subband in SBFD symbols is flexible, we need FFS whether we need limit UE behaviour. So we suggest to add one condition to main bullet and one subbullet for flexible resource case. Our proposal is as blow: SBFD aware UE does not transmit UL channel/signal outside the UL subband in SBFD symbols if resource outside the UL subband in SBFD symbols is DL. - FFS UE behaviour if a scheduled/configured UL transmission overlaps with DL subband(s) and guard band(s) if configured - FFS UE behaviour if resource outside the UL subband in SBFD symbols is flexible. |
| Xiaomi | The FFS point seems to propose a case that an Uplink transmission partially locates within UL subband. If this is the correct understanding, we think it is a wrong scheduling/configuration and should not happen as main bullet already says UE only transmit uplink within UL subband. |
| Spreadtrum | We support the proposal. To address the concern of sub-bullet, it can be involved in collision handling part with separate discussion. So we are fine to remove it now. |
| vivo | Without common understanding on what is the SBFD symbol from UE perspective, and what is the preferred SBFD operation from gNB perspective, it is not constructive to continue discssing the UE behaviour and signalling design. We see different interpretations for the SBFD symbol from UE perspective: 1. SBFD symbol is the symbol where the DL and/or UL sub-bands is semi-statically configured. 2. SBFD symbol is the symbol where the semi-statically configured DL and/or UL sub-bands apply (not overriden). In addition, we see different assumptions on whether SBFD symbol includes only semi-static DL symbol or it also includes semi-static flexible symbol. From gNB perspective, we think it is beneficial to support following operation without introducing any implementation difficulty since the subband size in frequency domain will not be changed: - A DL symbol that is semi-statically configured with UL subband can be dynamically determined by gNB to use as legacy DL symbol, or SBFD symbol. - A flexible symbol that is semi-statically configured with UL subband can be dynamically determined by gNB to use as legacy DL symbol, UL symbol or SBFD symbol. - A UL symbol that is semi-statically configured with DL subband can be dynamically determined by gNB as legacy UL symbol, or SBFD symbol as 2nd prioriy. |
| NEC | We agree with the intention but understand that the proposal can have more clarity because it seems that proposal is currently indicating (based on subbullet) that UL transmission can be performed outside UL subband. We can clarify the proposal like below: SBFD aware UE does not transmit UL channel/signal outside the UL subband in SBFD symbols. - FFS how to address the conflict when a scheduled/configured UL transmission radio resources overlaps with DL subband(s) and guard band(s) if configured |
| Sony | The FFS is fine as it raises an issue that needs to be addressed especially for Type 0 (RBG) based allocation. |
| WILUS | In our view, the flexible subband refers to the remaining RBs other than the RBs for UL subband and guardband in semi-static flexible symbols. If SBFD aware UE does not transmit UL channel/signal outside the UL subband in flexible symbols, SE will be reduced even compared to legacy UE. |
| DOCOMO | In our understanding, SBFD symbol is the symbol where the semi-statically configured DL and/or UL sub-bands apply. Therefore, for a SBFD symbol, “outside UL subband” would be DL subband (and maybe also guard band). Therefore, UL transmission in DL subband (and maybe also guard band) is not supported. For the FFS bullet, we are fine to discuss it in collision handling part. |
| Samsung | We are fine with the main bullet, but propose a different wording: The SBFD aware UE does not expect to be configured or scheduled for UL transmissions outside the UL subband in SBFD symbols. This removes the need for the first part of the FFS. Need for explicitly configured guard bands is still under discussion and if agreed, we can agree on UE behavior then. |
| QC | The proposal is related to the four options of UL/DL transmission reception in SBFD symobls. We think that RAN1 should discuss and agree on option-1 which is the most meaningful baseline. A semi-static UL-SB is used for UL transmission and semi-static DL-SB is for DL reception. As simple as this! Any futher enhancement and/or dynamic updated should be discussed as next step. We should establish a baseline first. Suggest the following updates: SBFD aware UE does not transmit UL channel/signal outside the UL subband in SBFD symbols and doesn’t receive DL signal/channel outside the DL subband in the SBFD symbol. - FFS UE behaviour if a smcheduled/configured UL transmission overlaps with DL subband(s) and guard band(s) if configured In addition, we find it very buzzling that some companies object the study of dynamic SBFD time arguing that we should follow baseline semi-static configuration while they support dynamic update/override of the UL-subband direction by DCI. |
| Nokia, NSB | We agree with Ericsson that main bullet would be sufficient. We understand the FFS is trying to address what the UE should do if the gNB scheduled the UE for an UL transmission outside of the UL subband. But we think this should not be handled in specifications. It is enough to say the UE does not expect that, rest can be left to proper gNB a(and UE) implementation. Therefore, we would suggest using the terminology from RAN1#110 agreement, i.e.: “SBFD aware UE does not expect to be scheduled or configured with UL transmission outside the UL subband in SBFD symbols” |
| MediaTek | We don’t support the proposal because if we don’t allow the UE to transmit any UL signal in the DL subband, and we don’t allow the UE to receive any signal in the UL subband, then there will be issue with the inter-UE CLI measurements based on SRS. |
| OPPO | We agree that: SBFD aware UE does not transmit UL channel/signal outside the UL subband in a SBFD symbol, if the SBFD symbol is semi-statically configured for the UE as DL symbol in TDD-Common or TDD-Dedicated. The issue is that RAN1 study so far does not prevent a SBFD symbol from being a UL symbol. If UL subband occurs in UL symbol, why should the UL Tx be restricted within UL subband? If such restriction is allowed, the SBFD evaluation assumption in 9.3.1 may need revision. |
| Intel | In our understanding, the intention of the sub-bullet is to allow the possibility that the configured or scheduled (at least for repetition case) resource for UL channel/signal can overlap with DL subband and guard band if any, and study mechanism for UE to avoid transmit such UL channel/signal outside the DL subband, e.g., UE drops the whole transmission or perform partial transmission within UL subband. We think it is useful to allow such flexibility for configured channel/signal and repetition case. Regarding the ‘Flexibe Subbands’ mentioned by other companies, we agree it is a valid point and we’re open for further study, i.e., whether the resource outside UL subband in a flexible symbol is a DL subband or flexible subband. For FFS bullet, we can replace ‘DL subbands’ with ‘subbands other than UL’ to include potential ‘Flexibe Subbands’ case ? |
| Moderator | Regarding the FFS, it seems that there are some confusions. The intention of the FFS is as what Intel says, i.e. to allow the possibility that the configured or scheduled (at least for repetition case) resource for UL channel/signal can overlap with DL subband and guard band if any, and study mechanism for UE to avoid transmit such UL channel/signal outside the DL subband, e.g., UE drops the whole transmission or perform partial transmission within UL subband. To address the comments, the proposal is updated as follows. Proposed Agreement: SBFD aware UE does not transmit UL channel/signal outside the UL subband in SBFD symbols. - FFS UE dropping behaviour if a scheduled/configured UL transmission resources are outside the UL subband overlaps with DL subband(s) and guard band(s) if configured |
| Huawei, HiSilicon | Given that whether SBFD operation is possible on flexbile symbols has not been decided yet, we think it is a bit too early to make such agreement. We suggest to focus on the definition of UE behaviour within the DL/UL subband not the UE behaviour outside UL/DL subband - The UL subband is defined as a set of consecutive RBs which is configured for UL transmission by RRC signaling. - The DL subband is defined as a set of consecutive RBs which is configured for DL transmission by RRC signaling. |
| vivo | We do not support this proposal. As already suggest multiple times that clarification/agreement is needed on at least which symbol can be configured as SBFD symbol. For flexible symbol is SBFD symbol and if only the RB location of UL subband is explicitly indicated (Option 2 in FL summary Figure 2-9), why UE can not transmit the UL outside the UL subband? Figure 2-9: Options for RB set and guard band configuration [18] |
| OPPO | If we understand the companies’ comments correctly, there are at least three companies (including us) concerning the SBFD symbol type in this proposal. Could this SBFD symbol be UL symbol? could FL please clarify? |

Proposal 1-10a

Proposed Agreement:

Study impact and potential enhancements of non-contiguous FDRA for RA type 1 across DL subbands for PDSCH.

|  | Company |
| --- | --- |
| Support | Lenovo, Xiaomi, Spreadtrum (general), CMCC, NEC, Sony, CATT, CEWiT, Nokia, NSB, Sharp, Intel, Huawei, HiSilicon |
| Not support | Ericsson, Samsung |

| Company | Comments |
| --- | --- |
| Moderator | Companies asked clarification whether the proposal is for RA type 0 or RA type 1 and commented that non-contiguous FDRA for RA type 0 is already supported. Therefore, the proposal is updated to limit the discussion to RA type 1. |
| Ericsson | We can support the proposal if “RA type 1” is replaced by “Type 0”. We don’t understand why Type-1 should be enhanced to support non-contiguous. It is true that Type-0 already supports non-contiguous; however, the potential enhancement that is needed for Type 0 is to handle the case when an RBG partially overlaps a guard band or UL subband as discussed by several companies. If the motivation for enhancing Type1 is that Type1 is lower overhead than Type0, then the overhead savings disappears if Type1 is enhanced to support non-contiguous FDRA due to the need for indicating two SLIVs (one per DL subband) or one SLIV and a second start symbol (common length). If the intention that the RBG overlap issue is to be handled by Proposal 12, then we do not support Proposal 1-10a as written. |
| New H3C | Maybe more explanations are needed below this. For example: Note: The non-contiguours FDRA for PDSCH means the PDSCH for one user is scheduled in two separated DL subbands. |
| ZTE | We would be ok to study this but as we commented in the first round, this enhancement should has lower priority before finalizing the structure of subband configuration. |
| Spreadtrum | It needs clarification for interleaved or non-interleaved, or both. From our side, we can focuse on non-interleaved RA type 1 first. So : Study impact and potential enhancements of non-contiguous FDRA for non-interleaved RA type 1 across DL subbands for PDSCH. |
| Vivo | It is related to the decision on whether to support both (D U D) and (D U)/(U D) or only (D U)/(U D) is supported. We tend to agree with ZTE’s views. |
| CMCC | As we commented in the 1st round, the issues in 1-10/1-11/1-12 can be joint considerd to strive find a common solution. |
| Sony | Type 0 cannot be supported in fallback DCI, causes overlapping RBG and likely consume more DCI overheads. We see the benefit of considering some enhancements to Type 1 notably for {DUD} SBFD configuration. |
| WILUS | Non-contiguous FDRA can be supported by FDRA Type 0. Thus, further enhancement on FDRA type 1 is not necessary. |
| DOCOMO | We share similar view as ZTE and vivo. |
| Samsung | We are generally in support of FDRA enhancements but consider these as somewhat lower priority. PDSCH RA Type 0 is available for SBFD. RBG granularity issues exist, but are small, e.g., worst case is loss of 13 schedulable RBS with a 51 RB UL SB and 273 RB carrier BW using either RRC configurable set 1 or 2. If guard bands are needed, the loss is even smaller because un-schedulable RBs from the overlapping RBGs can absorbe the GBs. Improvements to PDSCH RA Type 1 have the potential to become very complex. |
| QC | Further clarification from supporting companies on what enhancement for RA Type 1 should be. In our views, a simple enhancement that UL-SB could be used implicity as RM pattern. This is applicable for either RA Type0 or Type1. |
| Nokia, NSB | Is the intention to cover enhancements for RA Type-1 in this proposal and enhancements for RA Type-0 (due to RBG partially overlapping with guard band and/or DL/UL subband) in Proposal 1-12? Then we support both proposals as we think it is important to study these enhancements separately. |
| Sharp | In our understanding, RA type-1 is priority. DCI format 1_0 doesn’t support RA type-0. RM pattern introduced in Rel-15 is not applicable, according to the following sentence in TS38.214 where the UE is not expected to handle the case where PDSCH DMRS REs are overlapping with REs for RM resource. A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH. Further, RM pattern is not dynamically applicable for DCI format 1_0. |
| MediaTek | We generally support FDRA enhancements. With SBFD, Type-0 have the issue of partial RBGs that can’t be allocated. Also, these is benefit in enhancing Type-1 to support non-contiguous allocation on different bands. Thus we support to include both types of FDRA in the proposal. |
| OPPO | We share the view that both Type-0 and Type-1 have the potential for enhancements. |
| Intel | We are fine with the proposal. We’d like to clarify, ‘Study impact and potential enhancements’ includes the study of necessity for the support of non-contiguous FDRA for RA type 1 across DL subbands for PDSCH, and same clarification for 10a ~ 13a. |
| Moderator | It seems that companies have different views on whether enhancements should be consider for RA type 0 only, or RA type 1 only or both. In addition, some companies think it is low priority issue. More discussions are needed. |
| Huawei, HiSilicon | We are open to study enhancement to both type 0 and type 1 RA. |

Proposal 1-11a

Proposed Agreement:

Study impact and potential enhancements of CSI-RS resource set frequency domain resource allocatin and CSI reporting configuration across DL subbands.

|  | Company |
| --- | --- |
| Support | Ericsson (with clarification), New H3C, Lenovo, ZTE, Xiaomi, TCL, Spreadtrum, vivo, CMCC, NEC, Sony, CATT, DOCOMO, Samsung, CEWiT, Qualcomm, Nokia, NSB, Sharp, MediaTek, Intel, |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | The proposal is updated with some editorial changes proposed by companies. |
| Ericsson | We think it should be clarified that the impact and potential enhancements are related to the fact that the DL subbands are non-contiguous. Hence we propose the following clarification: Study impact and potential enhancements of CSI-RS resource set frequency domain resource allocatin and CSI reporting configuration across non-contiguous DL subbands. |
| ZTE | Typo allocatin  allocation. |
| Xiaomi | OK with Ericsson’s version. |
| CMCC | Same comment as 1-10a |
| NEC | We again emphasis that CSI measurement aspects should be captured in the proposal |
| Samsung | Ok with Ericsson’s proposed wording. |
| CEWiT | Fine with Ericsson’s modification |
| Nokia, NSB | We generally agree with the proposal. Note that the issue of configuring CSI-RS resources in non-contiguous DL subbands is related to the issues targeted with Proposal 1-10a for FDRA Type-1, and potential enhancements could be common for these two proposals. |
| Moderator | The proposal is almost stable. The proposed update from Ericssion is also supported by multiple companies. It is for clarification and I assume other supporting companies would be fine with the update. @NEC, regarding your comments on CSI measurement aspects, I would encourage you to provde more details on what needs to be enhancements and I suggest to keept that as separate discussion. Hope it is fine with you. So the following updated proposal is assumed to be stable for approval. Please indicate if you still have comments. Proposed Agreement: Study impact and potential enhancements of CSI-RS resource set frequency domain resource allocatin and CSI reporting configuration across non-contiguous DL subbands. |
| Huawei, HiSilicon | Support the latest proposal from the moderator. |

Proposal 1-12

Proposed Agreement:

Study impact and potential enhancements due to unaligned boundaries of DL/UL subband and configuration granularity (e.g. RBG for PDSCH/PUSCH FDRA, CSI reporting subband).

|  | Company |
| --- | --- |
| Support | Ericsson (with clarification),New H3C, Lenovo, ZTE, Xiaomi, TCL, Spreadtrum (general), CMCC, NEC, Sony, WILUS, CATT, DOCOMO, Samsung, CEWiT, Qualcomm, Nokia, NSB, Sharp, MediaTek, Intel |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | Companies asked clarification of the relationship between proposal 1-12 and proposal 1-10/1-11. To my understanding, the issues are different although the solutions can be common. |
| Ericsson | Often the Chairman removes examples when proposals are discussed on-line. If the examples are removed, the proposal doesn't make sense anymore. Furthermore, there is an issue if an RBG or CSI reporting subband overlaps a guardband, and this should be captured in the proposal. Based on this, we think the proposal should be clarified to explicitly spell out what cases are covered: Study impact and potential enhancements due to unaligned misalignment of the boundaries of DL/UL subbands and/or guardbands with the configuration granularity of at least the following: - RBGs for PDSCH/PUSCH FDRA - CSI reporting subbands |
| Lenovo | 6RB groups for CORESET frequency domain allocation may not be aligned with subband boundaries. Thus, we suggest to include the following: “Study impact and potential enhancements due to unaligned boundaries of DL/UL subband and configuration granularity (e.g. RBG for PDSCH/PUSCH FDRA, RBG for CORESET FDRA, CSI reporting subband).” |
| ZTE | OK to study. |
| Spreadtrum | For RBG for PDSCH/PUSCH FDRA, it is defined in Type-0 RA and Type-1 RA when scheduled by DCI 0_2/1_2. We ask for a clarification that whether both of RBG in the scope. Besides RBG, CSI reporting subband, PRG of PDSCH can be listed too. |
| CMCC | Same comment as 1-10a |
| Samsung | We prefer to have one single proposal for all FDRA-related enhancements – for PDSCH/PUSCH and CSI resource set(s)/report(s). |
| QC | We are okay with FL original wording. Not sure if updated text from Ericsson on guardband is needed. The misalignment refers to UL-SB or DL-SB or CSI-subband misalignment with the RBG grid. |
| Nokia, NSB | We think that it may be premature to study enhancements due to unaligned boundaries of DL/UL subband and configuration granularity before RAN1 agrees on the required SBFD granularity in frequency domain. However, we are ok to study and discuss this. |
| Intel | We share similar view with Lenovo to also add CORESET. Regarding the relation between 1-10a/11a and 12, we agree we can strive for the unified solution. |
| Moderator | The proposal is supported by majority companies. Lenovo and Intel would like to consider PDCCH as well. Spreadtrum proposed to add PRG. Some companies want a single proposal for all FDRA-related enhancements. More discussions are needed. |
| Huawei, HiSilicon | Seems that all companies are fine to at least study the aspects listed by the FL. |

Proposal 1-13a

Proposed Agreement:

Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols.

|  | Company |
| --- | --- |
| Support | Ericsson, New H3C, Lenovo, ZTE, Xiaomi, TCL, Spreadtrum, CMCC, NEC, Sony, WILUS, CATT, DOCOMO, Samsung (ok with intent of proposal, but needs more specific wording), CEWiT, Panasonic, Qualcomm, Sharp, MediaTek, OPPO, Intel |
| Not support | Nokia, NSB |

| Company | Comments |
| --- | --- |
| Moderator | It seems that the intention of the proposal is not clear to companies and I apologize for that. The intention of the proposal is to cover both UL and DL transmissions. In addition, not only repetitions are considered, but single transmissions are also included. For example, it includes single slot PUCCH transmissions in ehaviour slot types, SRS transmission in ehaviour slot types, CG PUSCH/SPS PDSCH transmissions in ehaviour slot types, multi-PDSCH/PUSCH transmissions in ehaviour slot types scheduled by a single DCI, multi-slot PDSCH/PUSCH/PUCCH repetitions with or without FH etc. I hope with the above clarification, the intention is clear to the group and companies are welcome to provide additional comments. |
| Ericsson | I guess the intention is to make the proposal quite general and thus cover the case even for a single PUSCH or PDSCH or PUCCH within a slot where the slot can consist of SBFD symbols and non-SBFD symbols? If so, then I guess it is okay to remove “repetitions” as suggested by the Moderator since this would be implicitly included in the proposal |
| ZTE | We share similar view as moderator that all these scenarios can be studied, including multi-PDSCH/PUSCH transmissions in ehaviour slot types scheduled by a single DCI. |
| Samsung | Thank you, moderator, for providing background in the comment section. Would it be possible to update the proposal with the specific cases and enhancements, e.g., single-slot FH ehaviour, multi-slot …? |
| QC | Thanks for the moderator for the examples and clarifation. |
| Nokia, NSB | We still have the same comments as in the 1st round discussion. It is not clear if this agreement is only covering FDRA enhancements due to multi-slot transmissions across SBFD and non-SBFD slots (e.g. repetitions, frequency hopping) or is also covering enhancements to semi-static configurations (e.g. CG-PUSCH, PDCCH, etc.) that may overlap (in time domain) with either SBFD and non-SBFD slots/symbols. We think that latter case also needs to be covered as it received significant support from many companies in their Tdocs submitted to RAN1#110bis-e meeting. We propose to divide the proposal into two ones as: Proposed Agreement: Study impact and potential enhancements for multi-slot transmissions across SBFD symbols and non-SBFD symbols Proposed Agreement: Study impact and potential enhancements for semi-static resource configurations (PDCCH, SPS, CG-PUSCH, PUCCH, SRS)transmissions across SBFD symbols and non-SBFD symbols. |
| Intel | We’re fine with the proposal, as long as companies share same understanding with moderator, i.e., it includes multi-slot transmissions across SBFD and non-SBFD slots (e.g. repetitions, frequency hopping) and also enhancements to semi-static configurations (e.g. CG-PUSCH, PDCCH, etc.) that may overlap (in time domain) with either SBFD and non-SBFD slots/symbols. |
| Moderator | @ Nokia, please check my comments in the first row which clarifies the intention. I suggest keeping the proposal as it is with the same understanding of the intention of the proposal as I clarified. Please indicate if you still have comments. |
| Huawei, HiSilcon | With the clarification, we are fine to support the proposal. |
| vivo | This proposal seems too general. If majority is fine with current formulation, we are OK. |

Proposal 1-14a

Proposed Agreement:

Study whether/how to handle time domain collision of UL transmission and DL reception in a same SBFD symbol for SBFD aware UEs:

|  | Company |
| --- | --- |
| Support | Ericsson (only if modified),New H3C, Lenovo, ZTE, Xiaomi, TCL, Spreadtrum (with clarification) , ITRI, vivo, CMCC, NEC, Sony, WILUS, CATT, DOCOMO, Samsung, CEWiT, Qualcomm, Nokia, NSB, Sharp, MediaTek, OPPO, Intel |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | Minor updated based on QC’s comment to clarify that the collision is in time domain. |
| Ericsson | As we commented in the 1st round, we don’t think developing exact collision rules in the SI is appropriate, as this is better left to normative work. Hence, we think the proposal should be modified to identify collisions cases and these can be listed in the TR Study whether/how to handle Identify cases of time domain collision of UL transmission and DL reception in a same SBFD symbol for SBFD aware UEs: |
| Spreadtrum | As we commented for P1-9, time domain collison can be with wider scope, besides UL Tx and DL Rx, there can be configured UL transmission with DL subband, and configured DL reception with UL subband. |
| QC | Regarding Ericsson suggested edits, is the intentio only to limit the proposal on identifying the collision cases as a first step. Then, the second step is studying the identified cases and discuss a rule for resolving the collision. |
| Moderator | The proposal seems agreeable to majority companies. The comment from Ericsson is that the exact collision rules are to be discussed in normative work and we only need to identify the cases in SI. I assume that companies may have different views on whether to discuss the exact collision rules in SI. But on the other hand, I expect we would anyway first identify the cases even according to the current proposal. Therefore, for progress, I suggest we go with the update from Ericsson and we can further discuss whether to discuss the exact collision rules later. Please indicate if you are not fine with the way forward. Proposed Agreement: Study whether/how to handle Identify cases of time domain collision of UL transmission and DL reception in a same SBFD symbol for SBFD aware UEs: |
| Huawei, HiSilicon | Fine with the latest proposal from the moderator. |

Proposal 1-15a

Proposed Agreement:

It is agreed that UL transmissions within UL subband in symbols configured as DL in TDD-UL-DL-ConfigCommon are supportedallowed.

- FFS symbols configured as DL in TDD-UL-DL-ConfigCommon and in tdd-UL-DL-ConfigurationDedicated

|  | Company |
| --- | --- |
| Support | Ericsson, New H3C, Lenovo, ZTE, Xiaomi, TCL, Spreadtrum, vivo (with modifications), CMCC, NEC, CATT, DOCOMO, Samsung (slight re-wording proposed), CEWiT, Panasonic, Nokia, NSB, Sharp |
| Not support | Vivo, Sony, Intel |

| Company | Comments |
| --- | --- |
| Moderator | The original intention of the FFS is for the following case in Figure 2-7 as explained in section 2.1.4. The FFS bullet is removed based on comments from companies. |
| vivo | We suggest following modification It is agreed that UL transmissions within UL subband in DL and/or flexible symbols provided by configured as DL in TDD-UL-DL-ConfigCommon are supportedallowed |
| CMCC | The intention of Figure 2-7 is to propose UE-specifc RRC/DCI/SFI can semi-statically or dynamically change the link direcetion of SBFD slots/symbols, which the UE-spepcfic RRC is not restricted to tdd-UL-DL-ConfigurationDedicated. |
| Sony | We share similar view with vivo, i.e. we should, at this poit, also include Flexible symbols configured by TDD-UL-DL-ConfigCommon. |
| WILUS | In our view, flexible symbols by TDD-UL-DL-ConfigCommon and/or TDD-UL-DL-ConfigDedicated also can be used for SBFD operation. Otherwise, resources in flexible symbols cannot be fully utilized for UL transmission (e.g., CG-PUSCH). |
| CATT | In current specification, UL transmission is already allowed in semi-static flexible symbols configured in TDD-UL-DL-ConfigCommon so we do not think we need to add flexible in the proposal. |
| DOCOMO | Also OK with vivo’s version. |
| Samsung | We propose this wording: “It is agreed that UL transmissions within the UL subband can be configured or scheduled for SBFD-aware UEs in symbols configured as DL and/or F in TDD-UL-DL-ConfigCommon for legacy UEs.” |
| QC | Similar comments from first round. This limits UL transmission in UE dedicated UL symbols. In addition, it limits SBFD operation to only UL symbol and exluceds flexible symbols. It is good to clarify if this is the intention. The current proposal limits SBFD operation for legacy UE iff only SBFD allowed for DL symbols by common-TDD. Then, legacy UE will not be scheduled UL in these symbols. Updates from Samsung to include flexible symbols should be at least captured in the proposal. |
| Ericsson 2 | Contrary to some of the above comments, we still have concern on agreeing on SBFD in 'F' symbols indicated by TDD-DL-UL-ConfigCommon due to unclear support from legacy UEs. In our view, a baseline SBFD solution design should not rely on this, due to the need to coexist with legacy devices. To clarify whether or not legacy UEs operate as expected in 'F' symbols, we would be open to asking for feedback from RAN5 on potential conformance testing in this area. |
| OPPO | We agree the principle, but just believe this “UL transmissions .... are allowed” may still be potentially subject to the DL-UL collision rule as mentioned in Proposal 1-14a. |
| Intel | The proposal only says what is allowed, but not mention what is not allowed. We’d like to clarify, whether UL transmissions outside UL subband in symbols configured as DL in TDD-UL-DL-ConfigCommon is forbidden by the proposal, or it is open? Based on proposal 1-9, UL channel/signal outside the UL subband in semi-static DL cell should be forbidden, and we think the current proposal should be aligned with the proposal 1-9. For flexible symbol, we think UL transmissions within UL subband in cell-specific flexible symbol should also be alloweded while FFS whether UL transmissions outside of UL subband in cell-specific flexible symbol. |
| Moderator | The intention of the proposal was to remove the existing restriction that UE is not allowed to transmit UL in semi-static DL symbols. For flexible symbols, UL transmissions are already allowed and that is why flexible symbols are not included. But it is not the intention to preclude UL subband in flexible symbols. Maybe we can discuss in the next round where UL subband can be located, i.e. DL symbols only or DL and flexible symbols. |
| Huawei, HiSilicon | We support that UL subband can be configured on DL or F symbols. We are wondering whether DL subband can be configured on flexible symbols should be also be discussed. |
| vivo | If it will be discussed in the next round, it will impact on many important proposals which are conditioned on the assumption. We suggest to dicuss this issue in this round. |

Proposal 1-16

Proposed Agreement:

Study whether SBFD operation in SSB symbols is supported or not.

|  | Company |
| --- | --- |
| Support | Ericsson, New H3C, Lenovo, ZTE, Xiaomi, TCL, Spreadtrum, ITRI, vivo, CMCC, NEC, Sony, WILUS, CATT, DOCOMO, Samsung, CEWiT, Qualcomm, Nokia, NSB, Sharp, MediaTek, OPPO, Intel, Huawei, HiSilicon |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | It seems to me that the proposal is stable in the 1st round discussion. Please indicate if you are not fine with the proposal. |
| New H3C | We suggest that an FFS is added below this： FFS：Study whether DL SBFD operation in PRACH symbols is supported or not. |
| Moderator | This proposal is stable for approval. @New H3C, what you are proposing is not relevant to the current proposal. |

Proposal 1-17

Proposed Agreement:

Study potential benefits and enhancements for initial access in UL subband.

|  | Company |
| --- | --- |
| Support | Lenovo, ZTE, Xiaomi, TCL, Sony, WILUS, CATT, DOCOMO, Samsung, CEWiT, Qualcomm, Nokia, NSB, Sharp, MediaTek, Intel, New H3C, |
| Not support | Ericsson, Spreadtrum, NEC, Fujitsu, OPPO |

| Company | Comments |
| --- | --- |
| Moderator | The views are quite clear. Companies can provide additional inputs if any. |
| Ericsson | We do not support this proposal as we don't think that RACH capacity is a bottleneck, nor is initial access latency. We think the SI should focus on CONNECTED mode as 1st priority. This proposal is quite broad, and may impact multiple areas of IDLE mode as well as support for legacy UEs. |
| Spreadtrum | SBFD should focused on RRC connected mode. If it is brought in RRC-IDLE, a lot of new issues could be taken place, such as new set of SIB, new configuration of CORESET, PDCCH monitoring, PRACH to indicate SBFD UE capability, ect. Considering the legacy requirement of initial access and backward compatability, initial access in UL subband can be with low priority. |
| vivo | We suggest following mofication Study potential benefits and enhancements for initial access in UL subband as second priority. |
| Samsung | We support SBFD & RACH for Initial Access in principle, but also think this can be 2nd priority during the SID. Evaluating feasibility and benefits of SBFD operation in RRC_CONNECTED mode is of more immediate importance for the Rel-18 SID. |
| QC | We think it is very beneficial to study SBFD operation for initial access. The main benefit in our views is to improve UL coverage for PRACH and MSG3 via repetition. There are other benefits as discussed in our contribution to enable more opportunities for ROs and reduce initial access latency. Without a study proposal, then companies can’t evaluate the pros/cons of enabling SBFD for initial access. |
| Nokia, NSB | We think that the motivation to introduce SBFD is generally to improve UL capacity, UL coverage and latency. With increased UL capacity/coverage there may be need to also increase RACH/Msg3/Msg5 capacity/coverage. Moreover, supporting initial access in SBFD slots has clear advantage in terms of reducing initial access delay. We think that initial access in SBFD slots/symbols should not be precluded in the study item phase. The decision whether to support this in specifications can be postponed till approval of normative work (if any). |
| Fujitsu | We slightly prefer not to support the proposal. For the consideration that the gNB cannot expect the timing when the UE transmits the PRACH on UL subband, it will be more complicated to reduce/control CLI due to the PRACH transmission. |
| Sharp | We have similar understanding with companies. UL coverage is the one of main motivations of this study. At msg3 repetition should be considered in this SI. Further, PRACH should be also considered since PRACH repetition is to be specified in Rel-18. |
| OPPO | We do not support the proposal, because we think the SBFD operations from multiple UEs in a cell can leave more UL resources to PRACH, which already (though indirectly) enhances the PRACH coverage. We also have concern on making quite some SBFD operations as UE mandatory features if initial access prior to RRC_CONNECTED occurs in UL subband. |
| Intel | We think SBFD for initial access is useful, e.g., for Msg 3 PUSCH with repetition, if we don’t allow PUSCH in SBFD symbol, 4 repetitons for Msg 3 PUSCH takes 20ms, while 4 repetitions for Msg 3 PUSCH only takes 4 ms with SBFD. One question, if we can’t agree on study of SBFD in initial access, whether PRACH in RRC connected mode in UL subband is supported or not ? |
| Moderator | Based on the feedback, I do not expect that we can make any progress in this meeting so the proposal is not pursued in this meeting. |

  - [Closed] 3rd round discussion

Proposal 1-2a

Proposed Agreement:

For SBFD operation within a TDD carrier, it is agreed that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is the baseline.

| Company | Comments |
| --- | --- |
| Moderator | This proposal is stable in the 2nd round discussion. Please refer to section 2.3 for more information. Please indicate if you still have comments/questions. |
| Nokia, NSB | We are fine with it. |
| Ericsson | We support Proposal 1-2a |

Proposal 1-6a

Proposed Agreement:

For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline.

| Company | Comments |
| --- | --- |
| Moderator | This proposal is stable in the 2nd round discussion. CMCC had a comment and proposed to update the proposal as follows. Please refer to section 2.3 for more information. CMCC’s version: For semi-static configurationindication of SBFD symbols subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD symbols SBFD subband time locations are explicitly indicated within a period is the baseline. Please share your views whether any update is needed. |
| New H3C | We are fine with updated proposal. |
| WILUS | We are fine with updated proposal. |
| vivo | We are fine with both versions or just to say “Explicit configuration of SBFD symbols within a period is agreed as baseline”. |
| ITRI | We are fine with updated proposal. |
| DOCOMO | OK with original version or the updated version. |
| ZTE | OK with the FL proposal. |
| Spreadtrum | Support For CMCC’s version, it supports symbol level configuration. It can be further discussed. So we prefer the proposal from Moderator. |
| Xiaomi | We slightly prefer the original version. Regarding to the ambiguity commented by CMCC, we think SBFD operation is clear. We don’t need to mention what kind of subband or subbands are configured with a time domain configuration. What really matters is on which time units SBFD operation can be applied from our perspective. |
| Samsung | We support. |
| Sony | Fine with CMCC updated proposal. |
| Panasonic | We are OK with either the original version or updated version. |
| Intel | If all companies share same understanding that this proposal does not intend to introduce new symbol type for signaling, i.e., SBFD symbol in addition to DL/UL/Flexible symbol, we’re fine with the proposal. |
| CATT | We prefer the original version which seems to be more generic. For example, we think it is possible to configure the time location of UL subband only. In addition, for now, it is not clear to us whether the time locations of UL subband and DL subband are always the same, especially considering flexible symbols. |
| Nokia, NSB | We are fine with updated proposal. |
| Huawei, HiSilicon | We agree Intel that the SBFD symbols or SBFD subband time locations shoud not imply to introduce new symbols types. We prefer the original version since it is more generic. |
| Fujitsu | We are fine with FL proposal. |
| InterDigital | CMCC’s version is preferred, as in previous meeting we agreed to use ‘SBFD symbols’. Therefore, we suggest using “SBFD symbols” instead of “SBFD subband time locations”. |
| Lenovo | We support the proposal. |
| Sharp | We are OK with the proposal. |
| QC | Support the proposal. In our views, both wording ‘subband time locations’ and ‘SBFD symbols’ are equivanet. No strong preference. |
| Ericsson | We support Proposal 1-6a. We also support CMCC's version. Agree with the statement from Intel, hence maybe this could be clarified with a note? Note: This does not imply that a new symbol type is created in addition to the existing D/U/F |
| Moderator | Based on the comments, majority companies are fine with either version. Spreadtrum, Xiaomo, CATT, Huawei prefer the original version and think it is more generic. Intel and Ericsson are fine with the updated version with a note that it does not imply to introduce new symbol type. From moderator point of view, I think the original version is more generic and thus prefer the original one. |

Proposal 1-8b

Proposed Agreement:

The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one for the study in RAN1.

- The UL subband can be located at one side of the carrier.

- The UL subband can be located at the middle part of the carrier, subject to RAN4’s study and conclusion

|  | Company |
| --- | --- |
| Support | New H3C, WILUS, vivo, DOCOMO, ZTE, Spreadtrum, Xiaomi, Panasonic, Intel (2nd bullet clarification), CATT, NEC, Nokia, NSB (with update), Fujitsu, Lenovo, Sharp, Ericsson (with update) |
| Not support | Samsung (2nd bullet point), Sony (clarification needed), InterDigital (call ‘DL subband’ and total up to 3 SBs), Qualcomm |

| Company | Comments |
| --- | --- |
| Moderator | This proposal is reworded to avoid using DL subband. Please provide your comments/questions. I assume there may be potential comments on {UDU} pattern. As per the guidance from RAN plenary, it should be clear that we would not discuss SBFD operation in legacy UL symbols before we finalize SBFD operation in non-legacy UL symbols. So the understanding is that the current proposal does not take SBFD operation in legacy UL symbols/{UDU} pattern into account. In addition, a general proposal in Proposal 1-21 is provided for this aspect and companies are also welcome to provide comments there. |
| vivo | We are fine with the proposal. But we wondered whether we need to send LS to RAN4 so that they can feedback to us early? Because many enhancements related to the resource allocation are conditioned on the frequency-domain subband pattern. |
| ZTE | OK |
| Spreadtrum | We agree UDU do not need to consider in DL symbols. |
| Xiaomi | Same view as Spreadtrum. |
| Samsung | We would like to remove “subject to RAN4’s study and conclusion” from the 2nd sub-bullet. The UL SB can be located at either one side of the carrier or be located in the middle. We agree. In both cases, RAN4 will need to agree on applicable requirements (OOB, spurious domain,…) and we expect that possibly different sets of RF requirements can apply to these 2 cases. So it is not obvious why we would attach “suject to RAN4 study and conclusion” to one case but not the other. An altenrative is to have a single generic note applicable to the entire agreement. |
| Sony | Does this mean we stop considering {DUD} operation until RAN4 sends us an LS to provide feedback? As vivo said, there are a lot of enhancements that would benefit {DUD} configuration and would be good to study them. |
| MediaTek | We are not sure why it was necessary to “avoid using DL subband”. We are discussing subband full-duplex, which is clearly means there is UL reception on a subband and DL transmission on another subband. Regardless of if these subbands signalled to the UE or not, they still exist from gNB perspective. We are not one of the strong proponents of UDU configuration, but our understanding of the RAN1 guidance is different from what the moderator “it should be clear that we would not discuss SBFD operation in legacy UL symbols before we finalize SBFD operation in non-legacy UL symbols”. What “we finalize SBFD operation in non-legacy UL symbols” means? Are we going to officially finish part of the SI on SBFD operation in non-legacy UL symbols, then we start another part of the SI on SBFD operation in legacy UL symbols? In other understanding, RAN guidance implies that we work on both options, but we prioritize SBFD operation in non-legacy UL symbols. So, if there is a proposal that can be applicable for both options, then we should not artificially exclude operation of SBFD in legacy UL symbols. For us, the proposal implies {UDU} is not supported, because we say there is a single UL-subband and there is no exceptions mentioned. Thus, the proposal needs to be clarified accordingly. |
| Intel | For 2nd bullet, similar question as other companies, if DUD is subject to RAN4’s study and conclusion, whether it is more efficient that we wait RAN4’s conclusion and start to study for resource enhancement for DUD, e.g., proposal 1-10a and proposal 1-11 (agreed yesterday for CSI enhancement) if RAN4 considers DUD is needed, or continue study with lower priority now, or continue study with equal priority with other agreed study issues ? Do we need to send LS to RAN4 to get feedback timely ? |
| Nokia, NSB | Considering the 2nd subbullet is subject to RAN4 study and conclusion, we propose it to be updated as to support the 2nd subbullet after RAN4 study and conclusion, update as following: “The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one for the study in RAN1. - The UL subband can be located at one side of the carrier. - After RAN4’s study and conclusion, if supported by RAN4, the UL subband can be located at the middle part of the carrier, subject to RAN4’s study and conclusion ” |
| Huawei, HiSilicon | We are fine to support this and share similar view as Samsung that “subject to RAN4’s study and conclusion” can be removed. We also think MTK raise a valid point that low priority does not mean that we should delay the discussion forever. |
| InterDigital | Share similar views as MediaTek. Maybe it’s better and no problem to say ‘DL subband’ as well. Initial proposal by FL having three subbands sound more generic, as long as we say explicitly ‘UL subband’ and ‘DL subband’ together, at least for discussion purpose. For example, in the next Proposal 1-10a, we use ‘DL subband’ in the proposal, so it should be no problem. |
| QC | This agreement precludes the ‘UDU’ configuration and is against RAN guidance to consideras SBFD operation in UL symbols as 2nd priority. It doesn’t meant to preclude the discussion. We can live with the earlier version of the proposal in the previous round (Proposal 1-8a). |
| Ericsson | We don't understand why the "subject to RAN4's study and conclusion" only applies to the 2nd sub-bullet. It should apply to both. Moreover, we don't think RAN1 should wait for feedback; RAN1 can study both options, and if RAN4 finds later that one or the other is infeasible, they can tell us. Modified Proposal: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one for the study in RAN1. - The UL subband can be located at one side of the carrier. - The UL subband can be located at the middle part of the carrier, subject to RAN4’s study and conclusion Note: RAN1 considers the above two possibilities unless RAN4 concludes that one or the other is infeasible |
| Moderator | Based on the comments, moderator updated the proposal in Proposal 1-8c in section 5.3. |

Proposal 1-10a

Proposed Agreement:

Study impact and potential enhancements of non-contiguous FDRA for RA type 1 across DL subbands for PDSCH.

|  | Company |
| --- | --- |
| Support | New H3C, DOCOMO, ZTE, Xiaomi, Sony, Intel, CATT , NEC, Nokia, NSB, Huawei, HiSilicon, InterDigital (with revision), Lenovo, Sharp |
| Not support | Samsung, Ericsson |

| Company | Comments |
| --- | --- |
| Moderator | Based on 2nd round discussions, companies have different views on whether enhancements should be consider for RA type 0 only, or RA type 1 only or both. In addition, some companies think it is low priority issue. From my understanding, the issue of RA type 0 is mainly unaligned boundary between RBG and subband and it is discussed in a separate proposal so it seems ok to cover RA type 1 only in this proposal. Regarding why enhancements for RA type 1 is needed considering that RA type 0 can support non-contiguous FDRA already, as Sony commented, RA type 0 cannot be used in fallback DCI and is likely to have larger DCI overhead. Please provide your further comments. |
| New H3C | First of all, we need define the new term “DL suband” |
| WILUS | Thanks for the clarification. Now we understand the intention of the proposal. However, it may focus on ‘non-contiguous PDSCH scheduled via FDRA type 1’, not on the FDRA type 1 itself. Thus, we propose to modify the proposal as following: Proposed Agreement: Study impact and potential enhancements of non-contiguous PDSCH scheduled via FDRA type 1 across DL subbands. |
| Spreadtrum | We support the proposal. Also fine to separately discuss RA type 0. |
| Samsung | As we commented in Round 2, we are generally open to study RA enhancements. Specifically, PDSCH RA Type 1 enhancements, we see the risk that we repeat Rel-15 NR design and will end up working on a PDSCH RA Type 3. We think this is unwarranted. PDSCH RA Type 0 is available and working for the case of center SBFD SB, so from the feasibility perspective in the Rel-18 SID, the system is not broken when SBFD is introduced. |
| Sony | Fine with the proposal. Type 0 and Type 1 were introduced for legacy operations, where each has their own benefits and hence, it make sense that these benefits are also available for SBFD operations. That is, Rel-18 sholdn’t be a step back from Rel-17. |
| MediaTek | Support with change to include both FDRA types: Study impact and potential enhancements of non-contiguous FDRA for RA type 1 across DL subbands for PDSCH. |
| NEC | We agree with Sony that RA type 0 can not be used in all situations (e.g. due to its larger DCI overhead), we think that it is useful to also support RA type 1 for the study where potential enhancements need to be considered to ensure compatibility of interleaving procedure with SBFD operation. |
| InterDigital | Agree with WILUS’s revision. |
| Sharp | As discussed in our contribution, the existing rate matching pattern cannot overlap with DMRS for PDSCH, which is intended not to change the channel estimation behaviour. Therefore, change to FDRA for PDSCH should be studied with high priority. Further, in our understanding, the fact that DCI format 1_0 only supports RA type-1 should be considered. |
| QC | We support the updated proposal by MediaTek. |
| Ericsson | We do not support this proposal, and we very much agree with Samsung's comments that this could end up as RAN1 working on RA Type 3 where Type 3 is just another variant of Type 0. Why should we waste time doing that, especially when it is likely that the overhead would end up being similar (or even greater) than that for Type 0. The argument used by some companies is that we must enhance Type 1 because fallback DCI does not support Type 0. It is true that fallback DCI does not support Type 0, but just because RAN1 might enchance Type 1, it doesn't automatically mean that the enhanced Type 1 would be supported for fallback DCI, especially since it likely won't have lower overhead than Type 0. So what's the point? Fallback is fallback; and is used infrequently. It is not necessary to do special optimizations for fallback in order to schedule non-contiguos DL subbands. |

Proposal 1-12a

Proposed Agreement:

Study impact and potential enhancements due to unaligned boundaries of DL/UL subband and configuration granularity of at least the following:

- (e.g. RBG for PDSCH/PUSCH FDRA

- CSI reporting subband).

- [RBG for CORESET FDRA]

- [PRG of PDSCH]

|  | Company |
| --- | --- |
| Support | New H3C, WILUS, DOCOMO, Samsung (updated wording required), Sony, Intel, CATT, NEC, Nokia, NSB, InterDigital, Lenovo, Sharp (with updates), Qualcomm, Ericsson (with updates) |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | Based on 2nd round discussions, the proposal is supported by majority companies. Lenovo and Intel would like to consider PDCCH as well. Spreadtrum proposed to add PRG. Ericsson proposed to explicitly list the uses instead of as examples. The proposal is updated accordingly and companies are invited to share your views on whether to include the bullets in square brackets and additional comments/questions. |
| New H3C | Because we already have CSI related agreement, we wonder whether this proposal need mention CSI reporting again or not. |
| WILUS | Support with following modification: - [REG for CORESET] |
| ZTE | Support to add the following two new cases. - RBG for CORESET FDRA - PRG of PDSCH |
| Spreadtrum | The resource allocation of CORESET has huge impacts on PDCCH BD. Now UE support 3 CORESET per BWP and up to 12 per cell. If new resource configuration introduced for CORESET, actually it would lead to significant complexity increasing. Since the CORESET in SBFD symbols or non-SBFD symbols may be different RBG, it has different REG-CCE mapping and PDCCH candidates. Considering the big difficulty in PDCCH monitoring, we prefer to be careful when considering REG of CORESET. |
| Xiaomi | Spreadtrum raises a good point and we should be careful with CORESET. From our perspective, gNB can avoid collision between CORESET and UL subband via configuration. For PRG, I think it should be handled with RBG together. |
| Samsung | We propose the following main bullet and cases, “Study the impact and benefits of potential enhancements to resource allocation granularity in frequency-domain for SBFD operation, including at least the following… - RBG set size(s) for PDSCH RA Type 0 - CSI reporting subband bitmap” We do not think that PUSCH RA Type 0 needs to be included or further considered. It is not a UE mandatory feature and little if any implemented in practice. Also, CORESETs (>#0) can be allocated quite flexible as by Rel-15. It is not clear to us that a problem exists with respect to allocation granularity. We propose to exclude this case from further investigation as well. |
| MediaTek | We suggest changing “at least the following” to “such as”, because the intention was to provide examples. |
| Intel | We are fine with all sub-bullets. Regarding H3C’s question for CSI part, we think we can keep it here. It is different from the agreed proposal, which is mainly for DUD scenario, while unaligned boundaries issue is also for DU or UD case. For CORESET, the granularity is 6 PRBs which may not be sufficiently flexible to align the boundary, so we think we can study CORESET. |
| CATT | We also do not see a strong need for CORESET FDRA which is already very flexible. |
| Nokia, NSB | We support to study the 4 bullets in this early stage. |
| Huawei, HiSilicon | We are a bit hesitate to add RBG for CORESET given the potential impact to resource mapping and unclear potential benefit. |
| InterDigital | Support the proposal to study these aspects. |
| Lenovo | Regarding CORESET resource allocation, we think that solutions not requiring additional CORESET/search space configurations for SBFD symbols and yet providing scheduling flexiblity should be studied. |
| Sharp | We think RA type-1 is of importance. Please also add RB bundle for non-interleaved mapping, under the main bullet. |
| QC | We are generally okay with the direction of the study. On the CORESET RBG graunlairty, we share similar views as Huawei. First, the resolution of the CORESET is very fine (6RBs) as compared to the one of RA Type0 or CSI-SB. So, it is not motivated enough. Also, it will open the door to corner-case optimization which will consume RAN1 time without clear objective of the gain. |
| Ericsson | We are also do not support adding "REG for CORESET" for similar reasons as mentioned by several other companies. Furthermore, we think that dropping REGs or using partial REGs will have a negative impact on PDCCH performance. We think "PRG for PDSCH" can be bundled with RBG for PDSCH since this doesn't really affect resource allocation. We support keeping RBG for PUSCH for completeness. |

Proposal 1-13a

Proposed Agreement:

Study impact and potential enhancements for scheduled/configured UL transmissions and scheduled/configured DL receptions across SBFD symbols and non-SBFD symbols, including at least the following:

- Single slot PUCCH in SBFD symbols and non-SBFD symbols

- SRS in SBFD symbols and non-SBFD symbols

- CG PUSCH in SBFD symbols and non-SBFD symbols

- TBoMS across SBFD symbols and non-SBFD symbols

- Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols

- PDCCH in SBFD symbols and non-SBFD symbols

- SPS PDSCH in SBFD symbols and non-SBFD symbols

- PDSCH/PUSCH/PUCCH repetitions across SBFD symbols and non-SBFD symbols

- CSI-RS in SBFD symbols and non-SBFD symbols

|  | Company |
| --- | --- |
| Support | New H3C, vivo (with comments), DOCOMO, ZTE, Xiaomi, Sony, Panasonic, CATT, NEC, Nokia, NSB, InterDigital, Lenovo, Sharp, Qualcomm, Ericsson (with updates) |
| Not support | Samsung |

| Company | Comments |
| --- | --- |
| Moderator | Based on online discussions, companies would like to make the proposal more specific. The proposal is updated by listing the cases for consideration. Companies are invited to provide your comments/questions. |
| WILUS | We are fine with the proposal in principle. In addition to the single slot PUCCH case, we suggest including “Single slot PUSCH in SBFD symbols and non-SBFD symbols”. Since the 2nd hop location is determined based on the size of active BWP, it can across the UL subband boundary even for intra-slot FH. |
| vivo | For single slot PUCCH, we prefer to say single PUCCH considering sub-slot PUCCH; PDCCH repetitions should be also considered. |
| Spreadtrum | We try to rerange the cases by scheduled by DCI or configured by higher layer signalling as following, since one-shot transmission/reception is different form periodic. We may need to discuss them separately: - Channel or RS scheduled by DCI in SBFD symbols and non-SBFD symbols - PUSCH/PDSCH/PUCCH without repetition - PDSCH/PDSCH/PUCCH with repetition - TBoMS PUSCH - Multi-slot PUSCH/PDSCH - Multi-cell PUSCH/PDSCH - CSI-RS/SRS - Channel or RS configured by higher layer signalling w/o triggered by activation MAC-CE/DCI in SBFD symbols and non-SBFD symbols - SPS PDSCH/ Type1&Type 2 CG-PUSCH/PUCCH without repetition - SPS PDSCH/ Type1&Type 2 CG-PUSCH/PUCCH with repetition - Type 2 CG-PUSCH TBoMS - PDCCH in SBFD symbols and non-SBFD symbols |
| Xiaomi | We are fine to further study the list cases. To be more general, we think dynamic scheduling should also be included. Hence, we propose the following updates for single slot PDSCH and PUSCH: - CG PUSCH in SBFD symbols and non-SBFD symbols - SPS PDSCH in SBFD symbols and non-SBFD symbols |
| Samsung | The proposed agreement is broad in scope. The fact that we have now listed 9 sub-bullets doesn’t make it any better compared to what we discussed in the online session on Day 4. If this proposal was agreed, technically, we would be allowed to consider re-designing anything from CSI-RS sequences, CSI-RS RE mapping, DMRS density…, or even come up with completely new FH behavior on top of Rel-15 NR. This can’t be the intention and is not commensurate with the scope of the Rel-18 Duplexing SID. Company contributions showed that (1) different configurable time-domain behavior, SBFD slot vs. nUL slot, for multi-slot transmissions, (2) different configurations “in general” for single- and multi-slot scheduling and (3) specifically, different FH behavior for the SBFD vs. nUL slots may require solutions. We should be looking at SRS as well, and also need to think of both DG and CG/SPS type allocations. We propose the following wording, Study the impact and potential benefits of enhancements to PDSCH, PUSCH, PUCCH and SRS transmission when scheduled or configured on SBFD and non-SBFD symbols, respectively, for, - multi-slot and single-slot PDSCH and PUSCH transmissions using DG or CG/SPS, PUCCH transmission incl. PUCCH repetition, and SRS. Note: potential enhancements are limited in scope to provide the SBFD-aware UE with different transmission configurations applicable to a same DL/UL channel/signal when transmitted using SBFD and non-SBFD symbols, respectively, e.g., existing L1 DL/UL channel/signal design is re-used |
| Sony | Fine with FL’s proposal and thanks for the comprehensive list. |
| MediaTek | We suggest changing “at least the following” to “such as”, because the intention was to provide examples. |
| Intel | Thanks for the comprehensive list provided by FL. We support the proposal. For UL, we think both the case with or without hopping is included, right? Maybe we can add a note. Though different channel and signal is listed separately, RAN1 should strive for unified solution, if possible, e.g., separate parameters configured for channels/signals in SBFD and non-SBFD symbols. |
| NEC | We are fine with this proposal but wanted to clarify one point. Given that UL channels/signals FDRA procedures (like frequency hopping) could be impacted due to operation within UL subband. But we don’t seem to have separate discussion for the this. So, are these aspects also expected to be discussed in this issue? |
| Huawei, HiSilicon | Support with adding a new bullet: Inter-slot/intra-slot/inter-repetition/inter-group (for DMRS bundling) frequency hopping pattern enhacement for PUSCH/PUCCH across SBFD symbols and non-SBFD symbols. |
| InterDigital | Support the proposal to study these aspects. |
| QC | Thanks to the FL for the detailed subbulets. We are generally fine with the proposal direction. However, it could be good to put all the channels/signals in the body of the agreements and then focus on few points/bullets that covers Repetition and Periodic transmission or reception acoss SBFD and non-SBFD symbols. |
| Ericsson | We think it is useful to list some signals and channels, but we have a concern that this proposal may die on-line because one company will argue for one example, and another for a different one, and we will run out of time. Something along the lines of Samsung's proposal could be one way forward, although it is missing repetitions for the case of CG-PUSCH and SPS-PDSCH. Furthermore, it is not clear that strictly having separate RRC configurations for SBFD and non-SBFD symbols is the way to go. Maybe that's appropriate for some signals/channels, but maybe not all. Different behavior could be defined in RAN1 specs for how the UE should behave in an SBFD symbol. For example, frequency hopping. If we go with a list anyway, then the list proposed by Spreadtrum is more comprehensive, since the moderator proposal is missing the case of repetition for CG-PUSCH and SPS-PDSCH . We agree with the comments by some companies that frequency hopping needs to be mentioned, e.g., by adding the following bullet: At least FDRA and frequency hopping (if applicable) are considered |

Proposal 1-18

Proposed Agreement:

For SBFD aware UEs, in symbols configured as DL in TDD-UL-DL-ConfigCommon with UL subband configured by semi-static configuration,

- UL transmissions within UL subband are allowed

- UL transmissions outside UL subband are not allowed

- A set of contiguous RBs in DL BWP excluding RBs of UL subband and guardband(s) are defined as a DL subband for discussion purpose

  - FFS whether frequency location of DL subband and guardband(s) are explicitly indicated or implicitly determined

- DL receptions within DL subband(s) are allowed

- FFS whether DL receptions within UL subband are allowed or not

- FFS: interaction of transmissions and receptions with TDD-UL-DL-ConfigDedicated and dynamic SFI

|  | Company |
| --- | --- |
| Support | New H3C with modification, WILUS, vivo (with comment), DOCOMO, Xiaomi, Samsung (DL SB definition requires re-wording or deletion), Sony, Panasonic (with modification), Intel, CATT, NEC, Nokia, NSB (with update), Fujitsu, Lenovo, Qualcomm, Ericsson (with updates) |
| Not support | Sharp |

| Company | Comments |
| --- | --- |
| Moderator | Based on the comments in previous discussions, companies proposed to separately discuss SBFD operation in semi-static DL symbols and flexible symbols. This proposal is for cell-common DL symbols. Please check and provide your comments/questions. |
| New H3C | We need clarify the condition on UL transmissions outside UL subband are not allowed is that resource outside UL subband is configured to DL. We can further study on resource outside UL subband is configured to Flexible case. SO we suggest modifying this proposal as below： Proposed Agreement: For SBFD aware UEs, in symbols configured as DL in TDD-UL-DL-ConfigCommon with UL subband configured by semi-static configuration, - UL transmissions within UL subband are allowed - UL transmissions outside UL subband are not allowed when resource outside UL subband is indicated as DL - A set of contiguous RBs in DL BWP excluding RBs of UL subband and guardband(s) are defined as a DL subband for discussion purpose - FFS whether frequency location of DL subband and guardband(s) are explicitly indicated or implicitly determined - DL receptions within DL subband(s) are allowed - FFS whether DL receptions within UL subband are allowed or not - FFS: interaction of transmissions and receptions with TDD-UL-DL-ConfigDedicated and dynamic SFI - FFS UL transmissions outside UL subband are not allowed when resource outside UL subband is indicated as Flexible |
| vivo | We are gernally fine with the proposal. And we would like to add following bullets - Allow conversion from SBFD symbol to DL-only symbol - FFS explicit and/or implicit conversion |
| DOCOMO | We support the proposal in general, and we don’t support DL receptions within UL subband, which can be achieved by dynamic indication of SBFD time domain location, i.e. indicating the symbol as non-SBFD symbol. |
| Spreadtrum | We support the proposal. For the modification from New H3C, we think it is unnecessary, expecilly the flexible part. The flexible symbol besides the guard symbol in time domain cannot work well, due to CLI and other reasons. We really do not want to repeat the same story again. Actually the flexible part in frequency is guard band which is only one option can work well, which has been included. So no need to define flexible subband again. |
| Xiaomi | We are generally fine with the proposal. The main bullet only covers the DL symbols configured by TDD-UL-DL-ConfigCommon. As we know there are other type of DL symbol which is configured by TDD-UL-DL-ConfigDedicated and SFI. We think these two types of DL symbols deserves the same study in the future. For the last FFS, I am confused as the intention is to study the interaction with with TDD-UL-DL-ConfigDedicated and dynamic SFI. However, as mentioned in the mainbullet, the DL symbols semi-static which should not be changed either via UE-dedicated signaling or SFI. More clarification is needed. |
| Samsung | We are in principel fine with the proposal but see 2 issues. (1) “…configured as DL in TDD-UL-DL-ConfigCommon for legacy UEs” (2) The sub-bullet on the DL SB definition (for discussion purposes) needs some better wording according to us. We say “a set of contiguous RBs in DL BWP excluding RBs of UL subband and guardband(s) …”. When the UL SB is located center carrier, we will have 2 DL SBs, so 2 sets of contiguous RBs. This cse is not captured. Also, we can’t make the assumption yet that the SBFD DL and UL SBs will be configured with respect to the UE BWP like the proposal says. The SBFD SBs can be configured with respect to the CRB grid and then UE determines applicable RBs similar to the way the CSI-RS bandwidth is allocated and handled by the UE with respect to its own BWP in Rel-15. For purpose of this scheduling agreement, we don’t need a formal definition of the DL SB yet, and it can be discussed & agreed separately. We propose to delete the corresponding sub-bullet. |
| Panasonic | As some companies mentioned in the previous round discussion, we think DL/UL subbands should be defined within a CC instead of within a BWP. This definition would also be aligned with Proposal 1-8b (i.e., in Proposal 1-8b, the maximum number of UL subbands within a TDD carrier is discussd, rather than within a BWP). Thus, we propose the following modification. For SBFD aware UEs, in symbols configured as DL in TDD-UL-DL-ConfigCommon with UL subband configured by semi-static configuration, - UL transmissions within UL subband are allowed - UL transmissions outside UL subband are not allowed - A set of contiguous RBs in DL BWP a CC excluding RBs of UL subband and guardband(s) are defined as a DL subband for discussion purpose - FFS whether frequency location of DL subband and guardband(s) are explicitly indicated or implicitly determined - DL receptions within DL subband(s) are allowed - FFS whether DL receptions within UL subband are allowed or not - FFS: interaction of transmissions and receptions with TDD-UL-DL-ConfigDedicated and dynamic SFI |
| MediaTek | In general, we think SBFD symbols should be configured as “F” in TDD-UL-DL-ConfigCommon. This will enable better collision handing, and the SFI can be used to override the SBFD symbols (if needed). In addition, inter-UE CLI measurements within the UL subband should be allowed. |
| Intel | We’d like to clarify the last FFS. For last FFS, does it include that a cell-specific DL symbol with UL subband can be configured as UL symbol by UE-specific TDD/SFI, so R15 restriction of no collsion of opposite directions between cell-specific TDD/UE-specific TDD/SFI is relaxed? If so, it seems not the interaction between transmissions and receptions with TDD-UL-DL-ConfigDedicated and dynamic SFI, it is interlactin between TDD-UL-DL-ConfigCommon and TDD-UL-DL-ConfigDedicated/dynamic SFI. For the last FFS, does it also include the case whether UL subband is ‘disabled’ for this UE if a cell-specific DL symbol with UL subband is configured as DL symbol by UE-specific TDD/SFI, while R15 restriction of no collsion of opposite directions between cell-specific TDD/UE-specific TDD/SFI is still as Rel-15? Clarification would be appreciated, to better understand the FFS scope. Thanks. |
| Nokia, NSB | We think that the bullets are fine for semi-static confiugarion. While whether same principles can be used for case when SBFD is dynamically configured can be further study. Propose to update as following: “For SBFD aware UEs, in symbols configured as DL in TDD-UL-DL-ConfigCommon with UL subband configured by semi-static configuration, - UL transmissions within UL subband are allowed - UL transmissions outside UL subband are not allowed - A set of contiguous RBs excluding RBs of UL subband and guardband(s) in DL BWP are defined as a DL subband for discussion purpose - FFS whether frequency location of DL subband and guardband(s) are explicitly indicated or implicitly determined - DL receptions within DL subband(s) are allowed - FFS whether DL receptions within UL subband are allowed or not - FFS: interaction of transmissions and receptions with TDD-UL-DL-ConfigDedicated and dynamic SFI - FFS: whether the same principles can be applied for the case where UL subband is dynamically indicated. “ |
| Huawei, HiSlicon | On the third subbullet. We don't see the need to confining DL subband within DL BWP. What is the benfit of this restriction? On the last FFS, we have similar question as Intel on about the implications. |
| Fujitsu | We are generally fine with the proposal. One thing to be clarified is that, in the first FFS, do we need to indicate the DL subband? If gNB indicates the UL subband and guard band in symbols configured as DL in TDD-UL-DL-ConfigCommon, the remaining resources will become naturally DL subband, in our understanding. |
| InterDigital | Agree with Nokia’s revision. We don’t support Panasonic’s revision to change to a CC, and we support FL’s original wording of ‘in DL BWP’. We’re okay with further updates by New H3C and vivo. |
| Lenovo | Since an UL subband is defined with respect to a carrier, we think a DL subband should be defined such as a set of configuous RBs in a carrier excluding RBs of an UL subband and guardband(s): - A set of contiguous RBs in a carrier in DL BWP excluding RBs of UL subband and guardband(s) are defined as a DL subband for discussion purpose - FFS whether frequency location of DL subband and guardband(s) are explicitly indicated or implicitly determined |
| Sharp | As Intel mentioned, the last FFS should be removed. Dynamic SFI or dedicated TDD configuration are only used to change “Flexible” to “DL” or “UL”. The proposal is for symbols configured as “DL”. We agree with Panasonic that if subbands would be defined, those should be defined within the carrier. BWP should not be used since thier location and size can be changed dynamically. |
| QC | We are generally fine with the proposal as it defines the basics for UE operation in SBFD symbols. We think the two FFS should be deleted and be discussed as later stage. |
| Ericsson | We think the last FFS is very unclear, and the scope is undefined. We suggest to remove it. We agree with other companies' comments on subband being defined within a carrier, hence we think the change suggested by Lenovo is appropriate. Regarding the DL subband definition, we don't think the bullet precludes the case of 2 non-contiguous DL subbands as mentioned by Samsung. We think this case is still included (we suggest a minor edit to clarify this). In summary, we support the following: Proposed Agreement: For SBFD aware UEs, in symbols configured as DL in TDD-UL-DL-ConfigCommon with UL subband configured by semi-static configuration, - UL transmissions within UL subband are allowed - UL transmissions outside UL subband are not allowed - A set of contiguous RBs in a carrier DL BWP excluding RBs of UL subband and a guardband(s) are is defined as a DL subband for discussion purpose - FFS whether frequency location of DL subband(s) and guardband(s) are explicitly indicated or implicitly determined - DL receptions within DL subband(s) are allowed - FFS whether DL receptions within UL subband are allowed or not FFS: interaction of transmissions and receptions with TDD-UL-DL-ConfigDedicated and dynamic SFI |

Proposal 1-19

Proposed Agreement:

UL subband can be configured in symbols configured as flexible in TDD-UL-DL-ConfigCommon for SBFD aware UEs.

|  | Company |
| --- | --- |
| Support | New H3C, WILUS, vivo, ITRI, DOCOMO, ZTE, Spreadtrum, Xiaomi, Samsung (re-wording), Sony, Panasonic (with low priority), MediaTek, Intel, CATT, NEC, Nokia, NSB, Huawei, HiSilicon, Fujitsu, InterDigital, Lenovo, Sharp, Qualcomm |
| Not support | Ericsson |

| Company | Comments |
| --- | --- |
| Moderator | Based on previous discussion, a number of companies support UL subband in flexible symbols while at least one company does not. Let’s collect more views. |
| vivo | Felxible symbol exsits in current specification since Rel-15 and is the basis for Rel-16/18 dynamic TDD. It is naturally to apply SBFD operation to flexible symbols. |
| DOCOMO | Since flexible symbol may be indicated as UL in legacy rule, UL subband naturally can be configured in flexible symbols for non-SBFD operation case. Therefore, we think it may be better to clarify the intention is to support UL subband in the flexbile symbol for SBFD operation. Suggested modification: UL subband can be configured in symbols configured as flexible for SBFD operation in TDD-UL-DL-ConfigCommon for SBFD aware UEs. |
| Spreadtrum | There is a complete functions for flexible symbol in the spec, including link derection assumption and overriding rules. In order to have a full mechanism of SBFD, and co-operate well with 9.3.3 flexible duplex, we support to study UL subband in flexible symbols. |
| Samsung | Similar to 1-18, “… as flexible in TDD-UL-DL-ConfigCommon for legacy UEs …” |
| Sony | Share similar views with vivo. Flexible symbols are defined in legacy system and it would be natural to consider them in Rel-18 Duplex Evo. It will be really strange to remove features in Rel-18, that are available in Rel-17. Again, Rel-18 should NOT be a step back from Rel-17. |
| Panasonic | We are OK to study SBFD operation in flexible symbol. However, as we discussed, flexible symbol has UE implementation issue. Then, even if SBFD operation in flexible symbol is specified, it may not actually be used. Supporting SBFD operation on flexible symbol would need a lot of extra discussion on such as flexible subband. Thus, SBFD operation in flexible symbol would be better to be lower priority than SBFD operation in DL symbol. We suggest adding a note like the following. UL subband can be configured in symbols configured as flexible in TDD-UL-DL-ConfigCommon for SBFD aware UEs. - Note: The study of SBFD operation in flexible symbol has lower priority than that of SBFD operation in DL symbol. |
| Intel | We should not forbid SBFD operation in flexible symbol with ‘some uncertanty’ though we agree that it makes sense to not only rely on flexible symbol for SBFD operation. |
| NEC | We support this proposal as it allows configuration of UL subband in the flexible symbols which can then later be updated by TDD-UL-DL-ConfigDedicated. We may need to study the impact of updating such flexible symbols (configured with UL subband) to UL symbols by TDD-UL-DL-ConfigDedicated and how/whether to handle the scenario where such flexible symbols are not modified by TDD-UL-DL-ConfigDedicated. |
| QC | Support. In order to accommodate legacy UE leveraging UL-gain and latency reduction of SBFD operation, SBFD operation in legacy Flexible symbols should be considered as well. |
| Ericsson | We do not support Proposal 1-19. We still believe it is dangerous to design an SBFD system that relies on an assumption that legacy UEs will correctly interpret 'F' symbols in TDD-UL-DL-ConfigCommon for data scheduling where the transmission will be wholly determined by DCI. If companies insist that this is a safe assumption to make, then we can support this proposal but only after receiving feedback from RAN5 about conformance testing for support of 'F' symbols for data scheduling. We think it is far more productive for RAN1 to study a baseline operation mode that does not make unsafe assumptions, and thus focus first on SBFD operation in symbols configured as 'D' in TDD-UL-DL-ConfigCommon for legacy UEs. If RAN5 later provides positive feedback, then we can consider this proposal and work on supporting 'F' symbols as well. |

Proposal 1-20

Proposed Agreement:

For SBFD aware UEs, in symbols configured as flexible in TDD-UL-DL-ConfigCommon with UL subband configured by semi-static configuration,

- UL transmissions within UL subband are allowed

- FFS whether DL receptions within UL subband are allowed or not

- FFS whether UL transmissions outside UL subband are allowed or not

- FFS whether DL receptions outside UL subband are allowed or not

- FFS: interaction of transmissions and receptions with TDD-UL-DL-ConfigDedicated and dynamic SFI

|  | Company |
| --- | --- |
| Support | Support, WILUS, vivo (with comments), ITRI, ZTE, Xiaomi, Sony, Panasonic, CATT, NEC, Nokia, NSB (with update) , Huawei, HiSilicon, Fujitsu, Lenovo, Sharp, Qualcomm |
| Not support | Samsung, Ericsson |

| Company | Comments |
| --- | --- |
| Moderator | Similar as Proposal 1-18, this is for semi-static flexible symbols. Please check and provide your comments/questions. |
| WILUS | Regarding the 3rd and 4th subbullets, both UL transmission and DL reception outside UL subband can be equally studied (i.e., both allowed or neither not). Different behaviors between DL reception and UL transmission in flexible symbols are undesirable in TDD operation. |
| vivo | For flexible symbols, it is natraully to be used as full DL, full UL or SBFD symbols. We do not see the reason to remove the legacy usage of flexible symbols. Therefore, we suggest following modifications: For SBFD aware UEs, in symbols configured as flexible in TDD-UL-DL-ConfigCommon with UL subband configured by semi-static configuration, - UL transmissions within UL subband are allowed - Allow conversion from SBFD symbol to DL-only or UL-only or flexible-only symbol - FFS explicit and/or implicit conversion - FFS whether DL receptions within UL subband are allowed or not - FFS whether UL transmissions outside UL subband are allowed or not - FFS whether DL receptions outside UL subband are allowed or not - FFS: interaction of transmissions and receptions with TDD-UL-DL-ConfigDedicated and dynamic SFI |
| DOCOMO | Similar comment as Proposal 1-19, we think it’s better to clarify the intention of the proposal is for the case when the symbols configured as flexible in TDD-UL-DL-ConfigCommon with UL subband is indicated/configured as SBFD symbol. |
| Samsung | Could supporting companies provide some background why we should consider Options 3 and Options 4 with unlimited DL <-> UL re-scheduling inside/outside the configured SBFD SBs for the case of F but not DL? The fundamental limitation is still on the gNB side due to filtering. There is no difference if a symbol is configured as DL or F in SIB1, when the gNB receive path filtering doesn’t allow for the corresponding RF processing. |
| Sony | Share similar view with vivo. We should not degrade existing features in Rel-18. We think the mechanism of Flexible symbols should be maintained, i.e. it can be further converted to UL or DL. On vivo’s proposal, I think whether UL tx outside UL subband are allowed or not depends what has be indicated/configured on the flexible part of the symbol. If it isn’t converted by SFI, then we agree, UL Tx outside UL subband should be allowed. Hence a slight modification to vivo’s proposal: For SBFD aware UEs, in symbols configured as flexible in TDD-UL-DL-ConfigCommon with UL subband configured by semi-static configuration, - UL transmissions within UL subband are allowed - Allow conversion from SBFD symbol to DL-only or UL-only or flexible-only symbol - FFS explicit and/or implicit conversion - FFS whether DL receptions within UL subband are allowed or not - FFS whether UL transmissions outside UL subband may be allowed dependent on status of the flexible symbolor not - FFS whether DL receptions outside UL subband may be allowed depending on the status of the flexible symbol or not - FFS: interaction of transmissions and receptions with TDD-UL-DL-ConfigDedicated and dynamic SFI |
| MediaTek | Inter-UE CLI measurements within the UL subband should be allowed. Also, we support Vivo’s comment. |
| Intel | Maybe we can discuss this proposal after conclusion of proposal 1-19. One clarification question, the operation in flexible symbol looks like dynamic frequency and time location change of subband. Is it correct understanding ? |
| Nokia, NSB | We think that the bullets are fine for semi-static confiugarion. While whether same principles can be used for case when SBFD is dynamically configured can be further study. We propose to update as: “For SBFD aware UEs, in symbols configured as flexible in TDD-UL-DL-ConfigCommon with UL subband configured by semi-static configuration, - UL transmissions within UL subband are allowed - FFS whether DL receptions within UL subband are allowed or not - FFS whether UL transmissions outside UL subband are allowed or not - FFS whether DL receptions outside UL subband are allowed or not - FFS: interaction of transmissions and receptions with TDD-UL-DL-ConfigDedicated and dynamic SFI - FFS: whether the same principles can be applied for the case where UL subband is dynamically indicated. “ |
| QC | Support the proposal w/o the FFS subbulets. |
| Ericsson | We do not support Proposal 1-20 for the same reason that we do not support Proposal 1-19. We still believe it is dangerous to design an SBFD system that relies on an assumption that legacy UEs will correctly interpret 'F' symbols in TDD-UL-DL-ConfigCommon for data scheduling where the transmission will be wholly determined by DCI. If companies insist that this is a safe assumption to make, then we can support this proposal but only after receiving feedback from RAN5 about conformance testing for support of 'F' symbols for data scheduling. We think it is far more productive for RAN1 to study a baseline operation mode that does not make unsafe assumptions, and thus focus first on SBFD operation in symbols configured as 'D' in TDD-UL-DL-ConfigCommon for legacy UEs. If RAN5 later provides positive feedback, then we can consider this proposal and work on supporting 'F' symbols as well. Similar to Samsung, we also have strong concerns on operation that would require dynamic re-configuration of the analog filters in the gNB Rx receive chain. If SBFD with an UL subband is allowed to be configured in a symbols configured as 'F' by TDD-UL-DL-ConfigCommon, then the Tx direction for the RBs outside this UL subband are undetermined. If they can be dynamically indicated as either 'U' or 'D', then this will require filter changes. It is not clear from the proposal at all if this is the intention. It is too early to work on this proposal when we haven't even sorted out operation in symbols configured as 'D' by TDD-UL-DL-ConfigCommon. For all of these reasons, we cannot support Proposal 1-20. |

Proposal 1-21

Proposed Agreement:

SBFD operations in symbols configured as UL in TDD-UL-DL-ConfigCommon is not considered unless stated otherwise.

|  | Company |
| --- | --- |
| Support | WILUS, ZTE, Spreadtrum, Xiaomi, Samsung, Panasonic, CATT, NEC, Sharp, Ericsson |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | This is a new proposal based on some intial discussions over RAN1 reflector. Please check and provide your comments/questions. |
| New H3C | RAN guideline is so enough and it is n’t required to make further restrictions on SBFD operations in symbols configured as UL. |
| vivo | We think it is suffient to just follow RAN guidance that SBFD operation in UL symbol is second priority. |
| Samsung | Similar to 1-18, “… as UL in TDD-UL-DL-ConfigCommon for legacy UEs …” |
| Sony | We have similar views with New H3C and vivo. I believe everyone understood the guideline from RAN and hence no need to discuss this again. |
| Panasonic | We think this proposal is beneficial to clarify the scope of future discussion/agreements. |
| MediaTek | The proposal is not clear to us. |
| Huawei, HiSilicon | Agree with vivo and we don't see the need of this proposal. |
| QC | This proposal is not needed and against RAN guidance. It was concluded that this discussion is 2nd priority. It doesn’t mean that this discussion is precluded. Conclusion: UL symbol as second priority is accepted, no intended suspension of continuation of work in WGs |
| Ericsson | We think it is useful to clarify, and maybe this can be taken as a conclusion. There always seems to be the same question popping up for other proposals about whether or not the proposal applies to the SBFD operation in legacy 'U' symbols. So why not clarify this? In fact, we should make this conclusion first, so there is no further confusion. |

- gNB self-interference handling schemes

This section discusses gNB self-interference handling schemes for SBFD.

  - Summary of input contributions

Timing alignment

In legacy TDD system, a UE is provided a cell-specific TA offset NTA,offset. It can be provided in SIB1 and if not, UE assumes TA offset in Table 7.1.2-2 in 38.133. The cell-specific TA offset is used to reserve enough time for UL/DL switching, as shown in the following figure.

Figure 3-22: cell-specific TA offset in legacy TDD [6]

For SBFD operation at gNB side, several companies discussed inter-slot/inter-symbol interference due to non-aligned slot/symbol boundary which would impact self-interference cancellation in SBFD symbol at gNB side.

One possible solution proposed by companies is to configure NTA,offset=0. It is considered that there can be backward compatibility issue in [7][19]. Nokia [32] commented that though in principle NTA, offset can be configured by the network to any value among 0, 25600*Tc and 39936*Tc by parameter n-TimingAdvanceOffset, in practice, legacy UEs always assume the value specified in Table 7.1.2-2 in 38.133 (for the corresponding frequency range and co-existence scenario) independently of what the network indicates in system information. With NTA,offset=0, the gap for UL-to-DL switching may disappear and the gap realized based on scheduling avoidance can be larger [15][32].

An alternative solution is to have separate NTA,offset for SBFD slot and UL slot, i.e. NTA,offset=0 in SBFD symbols and NTA,offset>0 in non-SBFD symbols as discussed in [7][12][15][19][32]. Potential overlap between UL transmission in a SBFD symbol and next UL transmission in a non-SBFD symbol were identified in [12][15][19][32], as shown below.

Figure 3-23: Different NTA,offset values for SBFD symbols and non-SBFD symbols [12]

InterDigital [14] discussed that non-zero timing advance or switching time could result in inter-slot interference and proposed to consider a means for UE to detect if the allocated UL/DL timing alignment is accurate.

Based on companies’ inputs, an initial proposal in Proposal 2-1 is provided.

Feasibility study

CATT [12] provided preliminary analysis on RSIC (Ratio of gNB self-interference cancellation) requirement and the gap between the received power of self-interference and the existing BS in-band blocking requirement for FR1.

Ericsson [18] provided initial feasibility analysis of self-inteference mitigation solutions for FR1 SBFD operations, including self-inteference from direct TX to RX leakage, self-inteference from RX non-linearity spectrum regrowth, reciprocal mixing of phase noise, impact on ADC and additional filtering, example digital self-inteference cancellation solution for FR1 homodyne/heterodyne type base station receivers, summary of initial SBFD implementation analysis for a single-carrier single-sector BS, impact of multi-sector base station sites, alternative opportunistic TDD operation modes, further factors to study and modern gNB receiver architectures for multi-carrier and multi-band use cases.

Samsung [25] provided analysis of self-interference cancellation including spatial-domain and antenna isolation, frequency-domain and digital cancellation and additional desing aspects. In addition, FR1 and FR 2-1 testbed performance are provided.

Qualcomm [30] discussed self-interference mitigation techniques including spatial isolation, frequency isolation, Rx filtering, beam isolation and beamforming/nulling and digital self-interference mitigation with self-inteference link budget. In addition, Qualcomm OTA SBFD demonstration is provided.

Nokia [32] discussed feasibility of SBFD in particular gNB RF architecture change for SBFD. It is observed that At least 145 dB of RSI and double number of BS antenna elements (Opt 2) are needed to achieve an acceptable performance with some tradeoffs between coverage gain and average system spectral efficiency degradation especially at high load based on preliminary evaluation results.

Although in last meeting, companies were encouraged to provide feasibility analysis for SBFD in AI 9.3.2, the inputs are more or less the same as in previous meetings. Companies are encouraged to provide your views that whether/what feasibility aspects should be discussed in this AI.

  - [Closed] 1st round discussion

Proposal 2-1

Proposed Agreement:

Study impact/potential enhancements of unaligned slot/symbol boundary for DL and UL subbands in SBFD symbols.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, InterDigital, Samsung, Panasonic, DOCOMO, Qualcomm, CMCC, CATT, Sharp, Nokia, NSB, Xiaomi |
| Not support | Sony, Ericsson |

| Company | Comments |
| --- | --- |
| Sony | It isn’t clear what the enhancements would be since some of the companies that raised this issue suggested that it could be solved simply by configuring NTA,offset=0. |
| Intel | We are open for the study, but some clarifications is needed, - Similar to Sony’s comment, whether slot/symbol boundary for DL and UL subbands in SBFD symbol is unaligned or aligned depends on Nta_offset and Nta, and implementation based on current UL timing mechanism can provide aligned boundary. Thus, the study should first consider the possibility of both unaligned and aligned case. - Though this proposal is provided under gNB self-inteference handling section, the impact and potential enhancement is not only restricted to gNB self-inteference handling, e.g., the impact on DL/UL switching at gNB or at UE side is also to be considered. |
| Ericsson | Similar view as Sony We also think there can be an implementation impact both on the UE and gNB sides from switching back and forth between N_TA,offset = 0 and a non-zero value, e.g., required changes to FFT window placement. It is also not clear what problem is trying to be solved by doing such switching. We also agree with the observations from some companies that legacy UEs may not correctly interpret a configuration indicating N_TA, offset = 0. |
| Nokia, NSB | We think the impact is for RAN4 to study/discuss. Note that discussions are already going on in RAN WG4 on this issue. Based on input from RAN4, RAN1 should discuss potential enhancements. |
| Intel | We also agree with Nokia’s suggestion. |

Question 2-2

Whether/what feasibility aspects should be discussed in AI 9.3.2?

| Company | Comments |
| --- | --- |
| ZTE | The feasibility aspects may require some input from RAN4, e.g., UE and gNB requirement. In addition, it may also relate to simulation results in AI 9.3.1. We may wait for some futher input at this point. |
| Intel | It is highly relevant to RAN4 feedback and evaluation in AI 9.3.1. We are not so sure AI 9.3.2 is a good place for this discussion now. |
| QC | Yes, feasibility aspects should be discussed in AI 9.3.2. It is part of the study item objective for RAN1 and conclusion should be capture in the TR as agreed in RAN1 #109e. - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1).. Discussion points may include the following: - Self-interference mitigation techniques, - link-budget calculation for self-interference and cross-link interference. In addition, the feasibility may include the signalling aspects for Alt 1 (transparent SBFD operation) and coexistence with legacy operation. |
| CMCC | Agree with ZTE and Intel, which the feasibility is related to RAN4’s input and the evaluation works in AI 9.3.1. |
| Ericsson | The chairman guidance from last meeting was quite clear that companies are encouraged to provide feasibility analysis under AI 9.3.2 as per the current description of AI 9.3.2 ### 9.3.2 Subband non-overlapping full duplex Including study on possible solutions, feasibility, and impact to legacy operation assuming co-existence in co-channel and adjacent channels. It is also quite clear from the SID that feasibility analysis is a RAN1 objective: - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). Regarding which aspects that can be included in a feasibility analysis, we believe that a list includes at least those aspects summarized above by the moderator. Hence, our view is that feasibility analysis should be provided in 9.3.2. We prefer this over AI 9.3.1 since that AI is heavily loaded already. |
| vivo | We agree with ZTE’s views |
| Nokia, NSB | We think feasibility should be primarily discussed in RAN WG4 and in AI 9.3.1. |

  - [Closed] 2nd round discussion

Proposal 2-1a

Proposed Agreement:

Study impact/potential enhancements of potential unaligned slot/symbol boundary for DL and UL subbands in SBFD symbols.

|  | Company |
| --- | --- |
| Support | ZTE, Xiaomi, TCL, Spreadtrum, CMCC, CATT, Samsung, Nokia, NSB, Sharp |
| Not support | Ericsson |

| Company | Comments |
| --- | --- |
| Moderator | Based on 1st round discussion, proposal 2-1 was supported by majority companies. For the comment from Sony that the issue may be solved simply by configuring NTA,offset=0. I think that is part of the study and NTA,offset=0 is one candidate solution. To address Intel’s comment that the boundary may or may not be aligned, I added “potential” in the proposal. Nokia suggested waiting for RAN4’s input. Companies are encourage to provide additional inputs considering the comments. |
| Ericsson | We are okay to wait for input from RAN4 on this issue as suggested by Nokia |
| vivo | We agree with Nokia’s suggestion. |
| Sony | Share similar view with Nokia. |
| DOCOMO | Agree with Nokia’s suggestion. |
| Panasonic | We are OK to wait input from RAN4. |
| QC | We don’t think that this proposal is conditioned on RAN4 input. We don’t recall than RAN1 LS to RAN4 included any timing aspects. |
| Nokia, NSB | Unaligned symbol boundary between UL and DL in SBFD symbols has a significant impact on the level of self-interference experienced at the gNB in SBFD slots/symbols. Therefore, we think RAN1 can start discussing solutions while waiting for RAN4 input. |
| Moderator | A couple of companies propose to wait for RAN4’s inputs. However, based on my information, majority companies in RAN4 expect the issue to be discussed in RAN1. I encourage companies to check internally with you RAN4 colleauges and we can come back in the next round. |
| Huawei, HiSilicon | We are not sure whether there is a need to wait for RAN4 feedback. |

Question 2-2

Whether/what feasibility aspects should be discussed in AI 9.3.2?

| Company | Comments |
| --- | --- |
| ZTE | The feasibility aspects may require some input from RAN4, e.g., UE and gNB requirement. In addition, it may also relate to simulation results in AI 9.3.1. We may wait for some futher input at this point. |
| Intel | It is highly relevant to RAN4 feedback and evaluation in AI 9.3.1. We are not so sure AI 9.3.2 is a good place for this discussion now. |
| QC | Yes, feasibility aspects should be discussed in AI 9.3.2. It is part of the study item objective for RAN1 and conclusion should be capture in the TR as agreed in RAN1 #109e. - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1).. Discussion points may include the following: - Self-interference mitigation techniques, - link-budget calculation for self-interference and cross-link interference. In addition, - the feasibility may include the signalling aspects for Alt 1 (transparent SBFD operation) and coexistence with legacy operation. |
| CMCC | Agree with ZTE and Intel, which the feasibility is related to RAN4’s input and the evaluation works in AI 9.3.1. |
| Ericsson | The chairman guidance from last meeting was quite clear that companies are encouraged to provide feasibility analysis under AI 9.3.2 as per the current description of AI 9.3.2 ### 9.3.2 Subband non-overlapping full duplex Including study on possible solutions, feasibility, and impact to legacy operation assuming co-existence in co-channel and adjacent channels. It is also quite clear from the SID that feasibility analysis is a RAN1 objective: - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). Regarding which aspects that can be included in a feasibility analysis, we believe that a list includes at least those aspects summarized above by the moderator. Hence, our view is that feasibility analysis should be provided in 9.3.2. We prefer this over AI 9.3.1 since that AI is heavily loaded already. |
| vivo | We agree with ZTE’s views |
| Nokia, NSB | We think feasibility should be primarily discussed in RAN WG4 and in AI 9.3.1. |
| Moderator | Please continue provding your inputs. |
| Samsung | We share Qualcomm’s view. |

  - [Open] 3rd round discussion

Proposal 2-1a

Proposed Agreement:

Study impact/potential enhancements of potential unaligned slot/symbol boundary for DL and UL subbands in SBFD symbols.

|  | Company |
| --- | --- |
| Support | New H3C, ZTE, Samsung, Panasonic, CATT, Nokia, NSB, Huawei, HiSilicon, Sharp, Qualcomm |
| Not support | Ericsson |

| Company | Comments |
| --- | --- |
| Moderator | In 2nd round discussion, a couple of companies proposed to wait for RAN4’s inputs. However, based on my information, majority companies in RAN4 expect the issue to be discussed in RAN1. I encourage companies to check internally with you RAN4 colleauges and provide your comments. |
| Samsung | We see this as a RAN1 discussion. |
| Intel | It seems RAN4 is discussing this issue and expects to close this by 2nd round email discussion (e.g., let RAN1 handle the issue). Maybe we can wait for RAN4’s 2nd round conclusion in this meeting. |
| Nokia, NSB | Unaligned symbol boundary between UL and DL in SBFD symbols has a significant impact on the level of self-interference experienced at the gNB in SBFD slots/symbols. Therefore, we think RAN1 can start discussing solutions while waiting for RAN4 input. |
| Ericsson | This issue is being discussed in RAN4. We prefer to wait for RAN4's input on this topic. |

Question 2-2

Whether/what feasibility aspects should be discussed in AI 9.3.2?

| Company | Comments |
| --- | --- |
| ZTE | The feasibility aspects may require some input from RAN4, e.g., UE and gNB requirement. In addition, it may also relate to simulation results in AI 9.3.1. We may wait for some futher input at this point. |
| Intel | It is highly relevant to RAN4 feedback and evaluation in AI 9.3.1. We are not so sure AI 9.3.2 is a good place for this discussion now. |
| QC | Yes, feasibility aspects should be discussed in AI 9.3.2. It is part of the study item objective for RAN1 and conclusion should be capture in the TR as agreed in RAN1 #109e. - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1).. Discussion points may include the following: - Self-interference mitigation techniques, - link-budget calculation for self-interference and cross-link interference. In addition, - the feasibility may include the signalling aspects for Alt 1 (transparent SBFD operation) and coexistence with legacy operation. |
| CMCC | Agree with ZTE and Intel, which the feasibility is related to RAN4’s input and the evaluation works in AI 9.3.1. |
| Ericsson | The chairman guidance from last meeting was quite clear that companies are encouraged to provide feasibility analysis under AI 9.3.2 as per the current description of AI 9.3.2 ### 9.3.2 Subband non-overlapping full duplex Including study on possible solutions, feasibility, and impact to legacy operation assuming co-existence in co-channel and adjacent channels. It is also quite clear from the SID that feasibility analysis is a RAN1 objective: - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). Regarding which aspects that can be included in a feasibility analysis, we believe that a list includes at least those aspects summarized above by the moderator. Hence, our view is that feasibility analysis should be provided in 9.3.2. We prefer this over AI 9.3.1 since that AI is heavily loaded already. Furthermore, the performance results in 9.3.1 alone do not give an answer on feasibility, since the assumptions used for evaluation, e.g., on alpha_SI and 1 dB desense need to be discussed separately for different gNB power classes to see if feasible, e.g., WA, MR, LA. |
| vivo | We agree with ZTE’s views |
| Nokia, NSB | We think feasibility should be primarily discussed in RAN WG4 and in AI 9.3.1. |
| Samsung | We share Qualcomm’s view. |
| Moderator | Please continue provding your inputs. |

- Inter-gNB and inter-UE CLI handling schemes

This section discusses the potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD.

The guidance from Mr. Chair on discussions of CLI handling in AI 9.3.2 and AI 9.3.3 is as follows.

Guideline for future meetings - Note: AI 9.3.3 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for dynamic TDD and schemes that are common for both SBFD and dynamic/flexible TDD. - Note: AI 9.3.2 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD.

A couple of candidate solutions for CLI handling for gNB-to-gNB and UE-to-UE CLI handling were agreed in AI 9.3.3 in RAN1#109-e.

Agreement For study of potential enhancement to dynamic/flexible TDD and/or SBFD, followings are considered as candidates of potential enhancement method of gNB-to-gNB CLI handling, where further prioritization/down-scoping of candidate schemes for study can be done in the future meetings: - gNB-to-gNB CLI measurement and reporting - Coordinated scheduling - Spatial domain enhancements - Advanced receiver - UE and gNB transmission and reception timing - Power control based solution - Potential enhancements to Rel-16 RIM - Sensing based mechanism - Note: Whether or not a particular scheme requires OTA or backhaul information exchange should be identified - Note: Any other scheme(s) for inter-gNB CLI handling is/are not precluded. - Note: For potential enhancements to dynamic/flexible TDD and/or SBFD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion. - Note: Potential enhancements specific for SBFD will be discussed in 9.3.2 Agreement For study of potential enhancement to dynamic/flexible TDD and/or SBFD, followings are considered as candidates of potential enhancement method of UE-to-UE CLI handling, where further prioritization/down-scoping of candidate schemes for study can be done in the future meetings: - Potential enhancements to UE-to-UE CLI measurement/reporting - Coordinated scheduling - Spatial domain enhancements, - Advanced Receiver - UE and gNB transmission and reception timing - Power control based solution - Sensing based mechanism - Note: Whether or not a particular scheme requires OTA or backhaul information exchange should be identified - Note: Any other scheme(s) for UE-to-UE CLI handling is/are not precluded. - Note: For potential enhancements to dynamic/flexible TDD and/or SBFD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion. - Note: Potential enhancement specific for SBFD will be discussed in 9.3.2

  - Summary of input contributions

The inputs from companies’ contributions on inter-gNB and inter-UE CLI handling schemes are summarized below as per moderator’s understanding. Moderator would like to apologize in advance if your views are not correctly captured or missed and encourage companies to correct/update the summary with revision marks if needed.

    - UE-to-UE CLI handling

Potential enhancements to UE-to-UE CLI measurement/reporting

The following agreement was made in RAN1#110.

Agreement Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for SBFD, at least includes: - Measurement resource/reporting configuration - Measurement/reporting details (including UE processing delay) - Relevant information exchange (between gNBs) if needed - Usage of measurement at gNB Note: other enhancement(s) for gNB-to-gNB and UE-to-UE CLI handling specific for SBFD are not precluded.

ZTE [7], Spreadtrum [9], Intel [15], Sony [16], Panasonic [19], MediaTek [26], LG [27], DOCOMO [28], CMCC [21], Qualcomm [30] and Nokia [32] proposed to consider L1/L2 based CLI measurement/report. But at least ZTE, DOCOMO and Qualcomm think it is common for SBFD and dynamic/flexible TDD. Huawei [6]’s observation is that L3 based UE-UE CLI measurement and report seems to be sufficient and L1 based solutions may need to be justified in the study.

An initial proposal in Proposal 3-1 is provided and companies are also kindly requested to feedback the proper AI for discussion of L1/L2 based CLI measurement/report for UE-to-UE CLI handling.

CMCC [21] proposed to study the following two methods for inter-UE inter-subband CLI measurement.

- Method#1: victim UE measures RSSI/SINR within DL subband;

- Method#2: victim UE measures RSRP of aggressor UE’s CLI-SRS within UL subband.

Figure 4-24: Inter-UE inter-subband CLI measurement method#1 & method#2

MediaTek [26] considered the above two methods as well and proposed to study whether SRS-RSRP-based ranking of UE aggressor candidates is sufficient for the optimization of UL-DL inter-subband CLI and study the feasibility and cost of muting co-channel interferer for the assessment of inter-subband UE CLI using CLI-RSSI measurements. In addition, it was proposed to study the feasibility of using “reverse” UE-UE CLI measurement to protect legacy UEs.

Qualcomm [30] discussed that the energy of the inter-UE CLI (within the UL subband) may affect the dynamic range of the DL signal as the AGC state will be set based on the total energy within the receiver bandwidth. With assumptions that there is no change in UE RF requirements, the common assumption is no RF/LPF filter to reject the inter-UE CLI within the UL subband and there could large loss of dynamic range of the DL signal when there is large CLI. Therefore, it is beneficial for gNB to have UE CLI reports based on the CLI measurements within the UL subband as well.

Based on the inputs, an initial proposal in Proposal 3-2 is provided to study whether/how to perform CLI measurement in UL subband for UE-to-UE inter-subband CLI handling.

For CLI measurement/report in DL subbands, the impact/potential enhancements considering non-uniform CLI in DL subband are considered in [7][15][16][23][28][30].

InterDigital [14] proposed to study measurement resources and reporting configurations for subband-edge CLI measurement. For example, the UE can measure a delta parameter that is based on measuring and calculating the difference between a first CLI-RSSI measured from the resources in the edge of the configured RBs and a second CLI-RSSI measured from the resources located in the middle of the configured RBs.

CLI measurement and reporting based on a finer granularity are considered in [7][15][16][28][30]. In [16], it is proposed to divide the BWP or the victim subband into smaller frequency blocks, where CLI measurement and reporting are performed on each frequency block as shown below.

Figure 4-25: Measurement Subband Blocks [16]

In [30], it is proposed to support subband-based CLI reporting as shown below. In addition, to reduce subband CLI reporting overhead, it is proposed that UE report CLI measurements in specific subband(s) where CLI exceeds a configured CLI threshold.

Figure 4-26: CLI subband reporting [30]

Based on the inputs, an initial proposal in Proposal 3-3 is provided to study impact/potential enhancements for UE-to-UE CLI measurement/report considering non-uniform CLI in DL subbands.

The impact/potential enhancements considering non-contiguous measurement resource in frequency are considered in [15][17][28]. For example, in [15], gNB may configure single CLI measurement resource in two DL subbands for a UE as shown in the following figure.

Figure 4-27: Single CLI measurement resource in two DL subbands [15]

Based on the inputs, an initial proposal in Proposal 3-4 is provided to study impact/potential enhancements for CLI measurement/report considering considering non-contiguous measurement resource in frequency.

Spatial domain enhancements

Panasonic [19] proposed to consider spatial domain enhancement, e.g. UE can report one or several strongest interfered beam directions together with CLI-RSSI.

Intel [15] discussed beam-specific interference measurement/report and coordination considering that interference would vary between UEs with different Tx beam and Rx beam. For example, gNB may configure multiple CLI resources to enable CLI measurement with different Tx beams and Rx beams for a potential victim UE. The Tx beam information can be transparent or none-transparent to the victim UE, while the Rx beam information should be explicitly indicated, e.g., by providing reference signal for QCL assumption for Rx spatial filtering.

MediaTek [26] proposed that UE reports SRS-RSRP (or CLI-RSSI) per Rx antenna separately and SRS-RSRP measurement can be configured with QCL-TypeD (spatial relationship information).

Power control based solution

ZTE [7] proposed that UL subband resources can be divided into multiple areas and each area is mapped with a dedicated power control parameter set for compensating the inter-subband interference with different levels.

Panasonic [19] proposed to study UL transmission power limitation, e.g. UL transmission power is differentiated between SBFD symbol and normal UL symbol especially for higher layer configured UL transmission (i.e., the maximum UL transmission power is limited on SBFD symbols).

Sony [16] proposed to support non-uniform MCS and non-uniform power control in a PUSCH considering that inter subband CLI is non-uniform across a subband.

InterDigital proposed to study dynamic UL power control mechanism based on some dynamic factors such as the frequency gap, beam/spatial-domain parameter, or a priority indication on the UL, to mitigate the effects of the CLI dynamically.

UE and gNB transmission and reception timing

Nokia [32] observed that by knowing the time difference of arrival between DL (transmitted by the serving gNB) and UL RS (transmitted by the aggressor UE) at the victim UE, the gNB can estimate the intra-cell (aggressor)UE-to-(victim)UE propagation delay. This information can be used to assist the UE by e.g. providing the specific timing offset to be used when performing a CLI measurement. It is proposed to study schemes for measurement and reporting of time difference of arrival between DL RS and UL RS, and potential extensions to the CLI measurement framework to include assistance information consisting of e.g. specific timing offset (with respect to DL timing) to be used when performing a CLI measurement.

Sensing based mechanism

InterDigital [14] proposed to study an event-based CLI sensing behaviour at the victim UE side, where the event can at least include a case when the victim UE detects a PDSCH reception failure, which initiates a subband-wise CLI measurement/reporting for a subband switching to avoid the CLI.

Others

ZTE [7] proposed that different frequency densities can be configured for reference signals transmitted in different areas with different interference levels.

Nokia [32] think it would be beneficial if the UE could measure and report to the gNB its own in-band emissions. In principle the same measurement and reporting framework could be extended to out-of-band emissions. This functionality could be included within the subband CLI measurement framework by requesting UE to be able to measure the RSSI on a subband while transmitting on another subband.

InterDigital [14] proposed to consider mechanisms to apply measurement skipping on some SBFD slots/symbols and power adjustment in deriving a CSI, depending on a level of dynamic power management occurred in the SBFD scenario.

    - gNB-to-gNB CLI handling

For gNB-to-gNB CLI handling, exchange of intended subband configurations for SBFD operation across gNBs are considered to be beneficial by Intel [15], Panasonic [19], NEC [23], DOCOMO [28], Qualcomm [30] and Nokia [32].

An initial proposal in Proposal 3-5 is provided to study exchange of intended subband configurations for SBFD operation across gNBs for gNB-to-gNB CLI handling.

Huawei [6] proposed to study the following aspects related to advanced IRC receivers in SBFD.

- Feasibility and performance of muting the REs on the DL subband in UL DMRS symbols and the REs on the UL subband in DL DMRS to improve channel estimation and inter-cell interference estimation and suppression.

- Feasibility and performance of specific CLI measurement resources to improve gNB-gNB and UE-UE co-channel and adjacent-channel inter-subband CLI estimation and suppression.

Figure 4-28: Interference management for interference suppression based on advanced receiver [6]

In addition, Huawei [6] proposed to study the feasibility and performance of spatial domain coordination in SBFD to suppress blocking interference caused by DL signal transmitting from the aggressor in the direction of the victim, e.g., coordinated beamforming.

Spreadtrum [9] proposed to study dynamic CLI measurement/report among gNBs.

CMCC [21] proposed to study the following two methods for inter-gNB inter-subband CLI measurement. In addition, it is proposed to support inter-gNB coordination in time-domain, frequency-domain, spatial-domain, and power domain and backhaul signalling enhancement is needed to support inter-vendor cooperation.

- Method#1: victim gNB measures leakage power from aggressor gNB within UL subband;

- Method#2: victim gNB measures RSRP of aggressor gNB’s RS within DL subband.

Figure 4-29: Inter-gNB inter-subband CLI measurement method#1 & method#2 [21]

CATT [12] assumed the above method #1 and think that victim gNB should avoid transmitting in DL subband of CLI measurement symbols to get more accurate measurement results. In addition, given that only interference power is measured, there is no need to define dedicated CLI RS. In addition, it is proposed to study information exchange between gNBs of CLI measurement resources, planned transmission resources, beam related information and RSSI or SINR together with beam information.

  - [Closed] 1st round discussion

Proposal 3-1

Proposed Agreement:

Study L1/L2 based CLI measurement/report for UE-to-UE CLI handling [in AI 9.3.2 or AI 9.3.3].

|  |  | Company |
| --- | --- | --- |
| Support | AI 9.3.2 | TCL, ZTE, Sony, InterDigital, Spreadtrum (second, SBFD specific), Intel (1st), Lenovo, Ericsson (with modification; study in only one AI) , Nokia, NSB, NEC (only for discussion SBFD specific aspects) |
|  | AI 9.3.3 | New H3C, TCL, ZTE, Sony, InterDigital, Samsung, Spreadtrum (first), Intel (2nd ), Lenovo, DOCOMO, Qualcomm, CEWiT, Ericsson (with modification; study in only one AI), CATT, Sharp, Nokia, NSB, Xiaomi, NEC (for discussing common aspects), OPPO |
| Not support |  | vivo |

| Company | Comments |
| --- | --- |
| ZTE | We support this proposal. We don’t have a strong view on which agenda item to discuss this proposal as long as companies have a common understanding. |
| Sony | SBFD may have subband specific CLI measurements, where measurements occurs only in DL subband. |
| Samsung | No strong preference. |
| MediaTek | We suggest having all the inter-CLI handling schemes discussed in one AI. No strong preference on which AI. |
| Intel | We agree that L1/L2 enhancment can be commonly applied for both SBFD and dynamic TDD. Considering L1/L2 enhancment is more practical for intra-cell inter-UE case, which is SBFD specific, we slightly prefer to discuss it in 9.3.2. |
| Panasonic | We are OK with discussing this aspect in either AIs if both AIs agree to study L1/L2 based CLI measurement/report. |
| DOCOMO | L1/L2 based CLI measurement/report can be common enhancement for inter-subband and intra-subband CLI handling. According to RAN1#109 guidance, it should be discussed in AI 9.3.3. |
| CMCC | The main difference between intra-subband CLI and inter-subband CLI is how to deifne the measure methods as the FL proposal 3-2 and 3-3, the L1/L2 measuremnet and report procedure are common to intra-subband and inter-subband CLI. |
| Ericsson | We are open to consider this, but we think it is very important to consider a practical UE processing delay for CLI measurement/reporting. If the processing delay is long (as it is in Rel-15, e.g., for L1-RSRP, then there is likely no benefit from introduction of L1/L2 reporting compared to L3. Hence, we would like to see that practical UE processing delays are taken into account in such a study. Regarding where this study takes place, we do not think it is efficient that it would occur in two different agenda items. We can further discuss under which AI makes most sense, but certainly not both. Study L1/L2 based CLI measurement/report for UE-to-UE CLI handling accounting for practical UE processing delays [in AI 9.3.2 or AI 9.3.3]. |
| Huawei, HiSilicon | As discussed in our contribution, the UE-UE cross link interference is more dependent on the large fading, i.e., the path loss, but not fast fading. So the rapid changes of channel has less impacts on the UE-UE cross link interferences. Then it seems that the L3 CLI measurement and reporting is somewhat sufficient. Further UE-UE interference measurement and report on L1/L2 should be justified. In additional, there seems an overlap between 9.3.1 and 9.3.2. |
| vivo | We agree if study, can be handled in one AI. But we would like to know why current CSI measurement/report (L1) and CLI measurement/report (L3) are not sufficient? |
| Nokia, NSB | We think that the general framework of L1/L2 based CLI measurement/report could be studies in AI 9.3.3, but specific enhancements related to configuration of intra-cell L1/L2 CLI measurement/report could be studied in AI 9.3.2. |
| NEC | Given that L1/L2 based CLI measurement enhancements are also considered in 9.3.3, we do not want to duplicate the discussion since most of the aspects are expected to common both both SBFD and dynamic TDD. However, we do acknmowlegde that some of the aspects may be specific for SBFD operation. Hence, it would be preferable to discuss the SBFD specific aspects in 9.3.2. |
| Moderator | According to the guidance from Mr. Chair, L1/L2 based CLI measurement/report for UE-to-UE CLI handling is to be discussed in AI 9.3.3. So we will stop the discussion in AI 9.3.2. |

Proposal 3-2

Proposed Agreement:

Study whether/how to perform CLI measurement in UL subband for UE-to-UE inter-subband CLI handling.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony, InterDigital, Samsung, Spreadtrum, MediaTek, Panasonic, DOCOMO, Qualcomm, CMCC, CEWiT, CATT, Nokia, NSB, Xiaomi |
| Not support | Ericsson, NEC |

| Company | Comments |
| --- | --- |
| ZTE | We are supportive of this study. However, we think another aspect which needs to be discussed is how to identify the aggreor UE. We propose to add an FFS. FFS: How to identify aggressor UE. |
| Lenovo | For UE-to-UE inter-subband CLI, we think measurement on resources of a DL subband that are close to an UL subband is sufficient. |
| Ericsson | We don't understand the rationale – for UE-UE CLI, the aggressor transmits in the UL subband and the victim receives CLI in the DL subbands. Why should the UL subband be measured? |
| Huawei, HiSilicon | Same question as Ericsson. Again there seems some overlap between 9.3.1 and 9.3.2 where “measurement details” as agreed in the last meeting seems quite broad to cover this proposal. |
| vivo | We would like to clarify for UE-to-UE inter-subband CLI measurement, should not the UE measure in the DL subband where it suffers the CLI? |
| Nokia, NSB | The Rel-16 CLI framework is already quite flexible and allow configuring the UE to perform CLI measurements on any subband within the UE’s UL BWP. SO not clear from this proposal what additional needs to be studied. |
| NEC | We don’t see the need for UE to measure CLI in UL subband. UE should be measuring only DL-subband even for CLI which is generated due to UL subband transmission by another UE in vicinity. Because measuring DL-subband shall provide an overall measure of CLI which affects DL performance. |

Proposal 3-3

Proposed Agreement:

Study impact/potential enhancements for UE-to-UE CLI measurement/report considering non-uniform CLI in DL subbands.

- Note: Non-uniform CLI in DL subbands is subject to RAN4’s feedback

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony, InterDigital, Samsung, Spreadtrum, Intel (with low priority), DOCOMO, Qualcomm, CMCC, CEWiT, CATT, Nokia, NSB, Xiaomi, NEC |
| Not support | Ericsson, Huawei, HiSilicon |

| Company | Comments |
| --- | --- |
| ZTE | We are generally fine with this proposal with the following two comments. - The CLI is dependent on the frequency isolation. If the frequency isolation is large, it may be frequency flat. If the frequency isolation is not sufficient, it may be non-uniform. Even without RAN4’s feedback, we can still conduct some preliminary RAN1 study. Thus, we propose to delete the note. - gNB-to-gNB CLI or self-interference may also be non-uniform, for example, adjacent channel interferences may come from one side. We propose the following update. Proposed Agreement: Study impact/potential enhancements for UE-to-UE CLI measurement/report considering non-uniform CLI in DL subbands. Study impact/potential enhancements for gNB-to-gNB CLI measurement/report and/or gNB self-interference considering non-uniform CLI in UL subbands. - Note: Non-uniform CLI in DL subbands is subject to RAN4’s feedback |
| Sony | Share similar view with ZTE. We don’t need the note regarding RAN4’s feedback. |
| MediaTek | If the UE does the measurement in the UL subband, we are not sure how is it possible to report non-uniform CLI in DL subbands. Also, we don’t see an advantage in such approach. |
| Intel | According to RAN4’s feedback for LS, flat CLI or 2-stage CLI would be sufficient, which does not require non-uniform CLI measurement. If RAN1 wants to ignore RAN4’s feedback, it is better to first prove the benefit by RAN1 evaluation. Thus, we suggest to study it with low priority. |
| QC | The CLI leakage within the UE DL subbands is not uniform and has a shape (e.g. similar to IBE mask). Therefore, it is necessary to study how to measure the CLI in different subbands within the UE DL frequency resources. |
| Ericsson | In our view, this seems like over-optimization with limited potential for gain. |
| Nokia, NSB | As the Rel-16 CLI framework already supports configuration of CLI measurements in DL subband, we think that potential enhancements should focus on performing (non-uniform) CLI measurements in DL subbands while the UE is transmitting in UL subband, e.g. the UE is measuring and reporting its own emissions. This could provide useful information to the gNB scheduler. |

Proposal 3-4

Proposed Agreement:

Study impact/potential enhancements for CLI measurement/report considering considering non-contiguous measurement resource in frequency.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony, InterDigital, Intel (minor typo), Lenovo, DOCOMO, CMCC, CEWiT, Ericsson, CATT, Nokia, NSB, Xiaomi |
| Not support | Samsung, MediaTek |

| Company | Comments |
| --- | --- |
| ZTE | We think this proposal is related to the FL summary capture in section 2.1.3.3 (copied below). - Option 1: Two CSI-RS resources are configured - Option 1-1: Two CSI-RS resources link to two CSI reports [28][32] - Option 1-2: Two CSI-RS resources link to one CSI report [18][28][30][32] - Option 2: Non-contiguous CSI-RS resource [12][17][18][28][30] [32] - Option 3: One contiguous CSI-RS resource overlapping with UL subband [25] It would be better if we can put all these options under this proposal as starting point. |
| Samsung | We think this proposed agreement is already covered by proposal 1-11. No separate agreement is necessary. Maybe 1-11 and 3-14 can be combined in the proposal. |
| Intel | There are two ‘considering’ in the proposal. Please delete one 😊 |
| QC | In last meeting, we had an agreement to study CLI measurements and report. Then, want to clarify that the new study aspect in this proposal is study considering configuration of non-contigous CLI measurement resource within the DL subbands. Agreement: Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for SBFD, at least includes: - Measurement resource/reporting configuration - Measurement/reporting details (including UE processing delay). |
| Ericsson | We support the proposal Regarding the comments about overlapping with Proposal 1-11, we don't think so. Proposal 1-11 is about CSI-RS. This proposal is about CLI-RSSI measurement resources which are separately configured under MeasObjectCLI |
| Huawei, HiSilicon | We are fine to support this with the understanding that this covers both gNB-gNB and UE-UE CLI measurement. |
| Nokia, NSB | We think this should be studied as part of the L1/L2 CLI measurement & report framework (see Proposal 3-1). |
| NEC | Not really sure what we will be covering with this proposal, given that we are already discussing impact on CSI measurements/configuration due to non-contiguous allocations in Proposal-1-11 |

Proposal 3-5

Proposed Agreement:

Study exchange of intended subband configurations for SBFD operation across gNBs for gNB-to-gNB CLI handling.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, ZTE, Sony, InterDigital, Samsung, Intel, Panasonic, Lenovo, DOCOMO, Qualcomm, CMCC, CEWiT, CATT, vivo, Nokia, NSB, Xiaomi, NEC |
| Not support | Ericsson, Huawei, HiSilicon |

| Company | Comments |
| --- | --- |
| Sony | We proposed OTA SBFD/Slot Format configuration indication between gNBs for CLI management, assuming it is relevant also to Flexible/Dynamic TDD. We are fine to separate SBFD Format and Slot Format gNB-gNB information exchange in 9.3.2 and 9.3.3 respectively. |
| Ericsson | We don't agree to this as a standalone proposal. We should not study exchange of info just fo the sake of exchange of info. We should study a potential CLI mitigation approaches, and then identify as part of the approach whether or not it is needed to exchange relevant info. Please see all the agreements in 9.3.3. "Relevant information exchange" was included as a sub-bullet for a given CLI mitigation scenario. |
| Huawei, HiSilicon | We think the information exchange should be at least justified with detailed gNB-gNB CLI handling schemes. Without analysis and performance evaluations, it is hard to say one should support the information exchange. This also applies for other gNB-gNB CLI handling schemes. |

  - [Closed] 2nd round discussion

Proposal 3-2

Proposed Agreement:

Study whether/how to perform CLI measurement in UL subband for UE-to-UE inter-subband CLI handling.

|  | Company |
| --- | --- |
| Support | ZTE, CMCC, Sony, CATT, Samsung, Panasonic, Qualcomm |
| Not support | Ericsson, Lenovo, NEC, Nokia, NSB, Sharp |

| Company | Comments |
| --- | --- |
| Moderator | Based on 1st round discussion, proposal 3-2 was supported by majority companies. Ericsson and NEC do not think measurement in UL subband is needed. As summaried in section 4.1.1, at least four companies considered measurement in UL subband for UE-to-UE inter-subband CLI handling. The reason, to my understanding, includes identifying aggressor UE(s) and considering the affect on dynamic range of DL signal etc. Please check the relevant summary and contributions to see whether the proposal is acceptable. Note that the proposal is not to support CLI measurement in UL subband, but to study whether to measure CLI in UL subband. |
| Ericsson | If the intention of performing CLI measurements in the UL subband is to identify whether an aggressor is present, or to identify an aggressor, then why do we need this proposal? It seems like this functionality already exists froom Rel-16 either based on SRS-RSRP or CLI-RSSI. In both cases CLI measurement resources can be configured in a restricted bandwidth (e.g., UL subband). |
| Lenovo | Since UE would not receive anything in an UL subband, we don’t see the need to measure the interference in the UL subband. |
| ZTE | We are supportive of this study. However, we think another aspect which needs to be discussed is how to identify the aggreor UE. We propose to add an FFS. FFS: How to identify aggressor UE. |
| CMCC | For inter-UE inter-subband CLI measurement, we think the CLI measurement in UL subband is benefit, due to agfressor UE can only transmsit CLI-SRS within its UL subband,victim UE can measure the interference signal power in its UL subband directly based on the measurement result of CLI-SRS RSRP and report it to the network as well as the CLI-SRS index to help gNB identify the strongest aggressor UE. To answer Ericsson’s comments: this measurement is similar to Rel-16 CLI measuremnts, but the Rel-16 CLI restricts that UE can only measure CLI-SRS or RSSI within its DL BWP which is not suitable for SBFD. |
| NEC | Same comment as first round |
| Samsung | We share the view of CMCC. |
| QC | The main benefit is to report to the gNB that there is in-band blocking by some nearby aggressor UEs. The CLI measurements in the DL subband can’t provide such information. Echo CMCC comments. |
| Nokia, NSB | It is not clear what it is intended with “perform CLI measurement in UL subband”. With current Rel-16 CLI framework, the UE can already perform CLI-RSSI and CLI-SRS on any ‘subband’, at least within the UE’s BWP. We would like to understand what enhancements are in target here before agreeing any proposal. |
| Moderator | CMCC, Samsung and Qualcomm commented that Rel-16 CLI measurement only supported measurement in DL BWP but now we need to study CLI measurement in UL BWP. The intention of the measurement is to identify the aggressor UE and report in-band blocking provided by CMCC and QC. Companies are encouraged to take that into account and we can come back in the next round. |
| Huawei, HiSilicon | We are not sure whether there is a strong need to introduce measurement on UL subband. The victim UE can still perform measurement in the DL subband overlapping with the UL subband of the aggressor. |

Proposal 3-3

Proposed Agreement:

Study impact/potential enhancements for UE-to-UE CLI measurement/report considering non-uniform CLI in DL subbands.

- Note: Non-uniform CLI in DL subbands is subject to RAN4’s feedback

|  | Company |
| --- | --- |
| Support | ZTE, CMCC, NEC, Sony, CATT, Qualcomm, Nokia, NSB, Sharp, Intel (low priority) |
| Not support | Ericsson, Samsung |

| Company | Comments |
| --- | --- |
| Moderator | Based on 1st round discussion, proposal 3-3 was supported by majority companies. ZTE and Sony proposed to remove the note. Intel commented that the CLI is frequency flat based on RAN4’s feedback and the study should be with low priority. Ericsson thinks it is over-optimiation with limited potential for gain. ZTE proposed to have the same proposal for gNB-gNB CLI and/or self-inteference. Nokia seems suggesting something different from the proposal. Based on the feedback, it seems to me that the original proposal is the best we can do for now. Please continue providing your views based on the comments in 1st round discussion. |
| Ericsson | We still think this is an over-optimization, when some more basic and fundamental issues on UE-UE CLI have not been addressed, e.g., under what scenarios is UE-UE CLI actually observable – we have seen limited scenarios so far. Then, it has not been answered if straightforward CLI mitigation without such optimizations can do the job. We think the scope of study should be contained. |
| ZTE | We can first focus on the UE-to-UE CLI part. But from our perspective, similar issue happens for gNB-to-gNB CLI. |
| Sony | We do not think this is an over optimisation at all. The edge of a DL subband at the UE will likely experience higher CLI compared with other parts of the DL subband that are further away from the UL subband. If this is considered over optmisation than we should have not introduce concepts like “subband CQI” reporting in LTE and in NR, as these measurements recognises that the entire 100 MHz bandwidth or more would not have uniform interference across it. |
| Samsung | We do not think such a measurement even if it is introduced with meaningfully relevant accuracy requirements could be useful to SFBF operation. |
| QC | The inband leakage of the UE is frequency selective. RAN4 already defined IBE mask that defines the limiat of these leakage on per-RB level. It is very logical that UE measures and report the CLI in different chunks of frequency resources in the DL-subband. This concept is not new and already supported for CSI subband PMI/CQI reporting. |
| Nokia, NSB | We think that non-uniform CLI measurement/report is particularly useful if a UE can measure and report its own emissions in the DL subband, not only the power that is measured in DL subband due to transmission by other device(s). |
| Intel | We share similar view with E/// and Samsung. |
| Moderator | Ericsson, Samsung and Intel doubt the potential benefit of the proposal while Sony and QC believe it is logical/beneficial. It seems that more discussions are needed. |
| Huawei, HiSilicon | We also have some doubt on the usefulness but would fine to study it further. |

Proposal 3-4a

Proposed Agreement:

Study impact/potential enhancements for UE-to-UE [and gNB-to-gNB] CLI measurement/report considering considering non-contiguous measurement resource in frequency.

|  | Company |
| --- | --- |
| Support | Ericsson (only for UE-to-UE), Lenovo (without gNB-to-gNB), ZTE, CMCC, NEC, Sony, CATT, DOCOMO, Samsung, Qualcomm, Nokia, NSB (with update), Sharp, Intel (only for UE-UE CLI) |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | Based on 1st round discussion, proposal 3-4 was supported by majority companies. Regarding the comment on potential overlapping with the discussion/proposal for CSI-RS, moderator shares the same understanding as Ericsson that the proposal is for CLI-RSSI, not for CSI-RS. Huawei would like to clarify that the proposal covers both gNB-gNB and UE-UE CLI measurement. My intention was to cover UE-UE CLI only based on companies’ contribution. I would like to hear companies’ views from other companies. |
| Ericsson | We support the proposal only for UE-UE CLI measurement/reporting. For gNB-gNB CLI measurement, the measurement would be within the UL subband which has contiguous resources. |
| Lenovo | gNB-to-gNB CLI should be measured in an UL subband, which is contiguous for (D,U,D), (D,U), or (U,D) configurations. |
| ZTE | support to add gNB-gNB as well. |
| Samsung | See our comments under […duplexing-02] |
| Nokia, NSB | We think this should be studied as part of the L1/L2 CLI measurement & report framework. For applicability to gNB-to-gNB case, this seems to relate to the discussion on gNB-to-gNB CLI measurement and reporting which is going on in AI 9.3.3. It is probably ok to limit the study to UE-2-UE CLI in this AI. We agree with moderator that this proposal is for CLI-RSSI and not for CSI-RS. We think this should be added in the proposed agreement as below: “Study impact/potential enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency.” |
| Intel | We support the proposal only for UE-UE CLI measurement/reporting. We share same understanding with E///. |
| Moderator | It seems that companies can agree with UE-to-UE part. Nokia propose to add CLI-RSSI for clarification. The proposal is updated as follows and it is considered stable for approval. Please indicate if you still have comments. Proposed Agreement: Study impact/potential enhancements for UE-to-UE [and gNB-to-gNB] CLI-RSSI measurement/report considering considering non-contiguous measurement resource in frequency. |
| Huawei, HiSilicon | We are fine to first focus on UE-UE measurement first. |

Proposal 3-5

Proposed Agreement:

Study exchange of intended subband configurations for SBFD operation across gNBs for gNB-to-gNB CLI handling.

|  | Company |
| --- | --- |
| Support | Lenovo, ZTE, CMCC, NEC, Sony, CATT, DOCOMO, Samsung, Panasonic, Qualcomm, Nokia, NSB, Intel |
| Not support | Ericsson |

| Company | Comments |
| --- | --- |
| Moderator | Based on 1st round discussion, proposal 3-5 was supported by majority companies. Ericsson and Huawei do not agree with the proposal as a standalone proposal and should be justified with detailed gNB-gNB CLI handling schemes. To me, the proposal is to study exchange of SBFD configuration. As part of the study, companies should provide details on how to use the information and justify the potential gains. I keep the proposal unchanged and encourage companies to share more views on how to address the comments. |
| Ericsson | We still do not support this proposal for the same reasons as provided in the 1st round. We disagree with the moderators view on the motivation for the proposal. |
| Lenovo | Based on exchange of intended subband configurations, neighboring gNBs can coordinate UL subband configurations to minimize the impact of CLI. |
| Sony | Consider OTA exchange for SBFD and also Slot Formats. |
| Samsung | We support the idea of the proposal in principle, but anticipate the need for this signaling may later depend on practical deployment needs, e.g., when same SBFD config must be employed in the deployment (or vicinity), there is no need for such signaling. We do not consider exchange of short-term SBFD configuration changes, e.g., as by gNB scheduling, meaningful to assume during practical system operation. |
| Nokia, NSB | As exchange of intended TDD configuration has been specified for dynamic TDD in Rel-16, we think it makes sense to extend the signalling framework by enabling gNB to exchange information on the intended SBFD configuration. This is expected to enable coordination among gNBs to minimize the impact of the CLI. Anyway, we do not think RAN1 should spend a lot of time to discuss this in the study item phase. Some text could be included in the TR. Once the SBFD signalling framework is agreed during potential normative work on SBFD, RAN3 could then be tasked to extend the inter-gNB signalling framework introduced in Rel-16 for dynamic TDD. |
| Moderator | Ericsson is still not fine with the proposal. Samsung and Nokia are fine with the proposal but do not agree with short-term/OTA exchanges. More discussions are needed and proponent companies are encouraged to provide more inputs. |
| Huawei, HiSilicon | As commented in the first round, we still believe that the information exchange should be justified with detailed gNB-gNB CLI handling schemes. We don’t support this standalone proposal without mentioning how to make use of such this information. |

  - [Open] 3rd round discussion

Proposal 3-2

Proposed Agreement:

Study whether/how to perform CLI measurement in UL subband for UE-to-UE inter-subband CLI handling.

|  | Company |
| --- | --- |
| Support | New H3C, vivo (with comments), DOCOMO, ZTE, Samsung, Panasonic, CATT, InterDigital, Qualcomm |
| Not support | NEC, Nokia, NSB, Lenovo, Sharp, Ericsson |

| Company | Comments |
| --- | --- |
| Moderator | The main controversial part seems to be whether Rel-16 CLI measurement is sufficient to support CLI measurement in UL subband if needed. You can refer to section 4.3 for more information and please provide your further inputs based on the previous comments. |
| vivo | Allowing victim UE to do CLI measurement in the UL subband, we think it can be studied and discussed. Companies seems have different understanding on whether it is already supported in Rel-16. We found TS 38.214 clause 5.1.6.4 specifies following: 5.1.6.4 SRS reception procedure for CLI The SRS resources defined in Clause 6.4.1.4 of [4, TS 38.211] may be configured for SRS-RSRP measurement for CLI, as defined in Clause 5.1.19 of [7, TS 38.215]. The UE is not expected to measure SRS-RSRP with a subcarrier spacing other than the one configured for the active BWP confining the SRS resource. The UE is not expected to measure SRS-RSRP using the SRS-RSRP measurement resource which is not fully confined within the DL active BWP. Now assume the UL subband is entirely covered by a DL BWP, companies may have different interpretations on whether the CLI measurement in the UL subband in a DL BWP can still be viewed as conforming to above specification. |
| ZTE | Maybe we can update the proposal as following to address some concerns from companies. Proposal: Study the following for UE-to-UE inter-subband CLI handling. - whether/how to perform CLI measurement in UL subband for UE-to-UE inter-subband CLI handling. - whether/how to perform CLI measurement in DL subband for UE-to-UE inter-subband CLI handling by taking the Rel-16 as baseline. |
| Spreadtrum | We prefer the version from ZTE to list all options for next meeting. |
| Samsung | We think that ZTE’s suggestion do distinguish the 2 CLI handling cases makes much sense and support the proposal. |
| Sony | Not sure why UE needs to measure CLI in UL subband since it is the aggressor. However, we can study it. On ZTE’s proposal, I would have thought it is obvious that UE can measure CLI in DL subband but fine if we want to be explicit. |
| Panasonic | We share vivo’s view. If companies have different understanding on CLI measurement in UL subband, it would be beneficial to clarify that CLI measurement in UL subband is supported for SBFD operation. |
| Intel | It is unclear why we has to rely meausmrent in UL subband to identify blocking, why measumrent in DL subband can not be used to identify potential blocking, though the derived result may not be very accurate? It is unlcear why we need to extend Rel-16 CLI in DL BWP to UL BWP or out of DL BWP. In our view, the CLI resource in UL subband which is within bandwidth of DL BWP as shown in case 1 and case 2 is already supported by Rel-16. The only issue is whether we allow CLI measumrent on CLI resource in UL subband outside of active DL BWP as shown in case 2 and case 3. In our understanding, we don’t see clear motivation to support such enhancement. Case 1 Case 2 Case 3 |
| NEC | We understand that main discussion is - UE needs to measure UL subband in order to identify the aggressor UE - Can UE measure UL subband based on Rel-16 CLI measurement framework We acknowledge the above issues but from our understanding, to address above, it is up to network configuration to ensure that UE can measure UL subband for the purpose of CLI measurement. One possible mechanism to accomplish above is by providing UE specific TDD configurations/scheduling (for e.g. configure UL subband for CLI-RS transmitting UE in Symbol-N, while indicating Symbol-N as DL-only symbol for a CLI-RS measuring UE). We don’t think that any additional optimization is required on top of such mechanism. Hence, we don’t think that requirement for enhancement of this issue is high priority for now. |
| Nokia, NSB | It is still not clear what is the benefit and complexity of performing CLI measurement in UL subband. |
| Huawei, HiSilicon | As indicated in the second round, we are not sure whether there is a strong need to introduce measurement on UL subband. The victim UE can still perform measurement in the DL subband overlapping with the UL subband of the aggressor. |
| InterDigital | OK with ZTE’s revision |
| Lenovo | We think in-band blocking can be measured in RBs of a DL subband. |
| Sharp | In our understanding, the network can configure the same bandwidth for UL/DL BWP. Then, measurement in UL subband can be done under Rel-16 principle. We don’t think the definition of BWP should be changed from Rel-15. Our view is similar to Intel. For example, we are not sure why Case 3 in Intel’s comment happens? |
| QC | There is confudsion on the proposal. The discussion is related to UE SBFD operation within a single BWP pair as shown bu the example ‘case 1’ in the figure from Intel reply. In that scenario, UE knows that this UL-SB is used for UL transmission although it is confined within the DL BWP whereas the UL-SB mainly used for UL transmission. Then, proposal says that UE can still measure the CLI in these frequency resources although they are configured as UL-SB as some other aggressor UEs are performing UL transmissing in the UL-SB. |

Proposal 3-3

Proposed Agreement:

Study impact/potential enhancements for UE-to-UE CLI measurement/report considering non-uniform CLI in DL subbands.

- Note: Non-uniform CLI in DL subbands is subject to RAN4’s feedback

|  | Company |
| --- | --- |
| Support | New H3C, DOCOMO, ZTE, Spreadtrum, Sony, CATT, NEC, Nokia, NSB, InterDigital, Sharp, Qualcomm |
| Not support | Samsung, Lenovo, Ericsson |

| Company | Comments |
| --- | --- |
| Moderator | Ericsson, Samsung Intel and Huawei doubt the potential benefit of the proposal while Sony and QC believe it is logical/beneficial. You can refer to section 4.3 for more information and please provide your further inputs based on the previous comments. |
| ZTE | We share similar comment as Qualcomm provided in this 2 round of discussion. |
| Samsung | See our Round 2 comments. |
| Sony | As we commented earlier, even in LTE case with only 20 MHz bandwidth, we have Subband CQI measurement simply because the channel can be frequency selective and such finer granularity of CQI report is beneficial to the gNB scheduler. This will also benefit operations in100 MHz bandwidth or more in NR with DL/UL subbands due to leakage. Perhaps the question for those against simple measurement reports like subband CQI or CLI be: - Why would the channel property, all of a sudden, change from frequency selective to flat when we move from legacy to SBFD, e.g. if Slot n is DL and Slot n+1 is SBFD, would SBFD cause the channel to change to from frequency selective to flat fading? What type of subband filtering is expected at the UE or gNB that can perfectly prevent any leakage into another subband or have zero receiver selectivity from another subband? |
| Intel | Same view with E/// and Samsung. |
| NEC | We also have similar opinion as Sony and QC and think that this issue is relevant due to frequency-selective nature of CLI for SBFD operation. |
| Nokia, NSB | Same view as 2nd round. We think that non-uniform CLI measurement/report is particularly useful if a UE can measure and report its own emissions in the DL subband, not only the power that is measured in DL subband due to transmission by other device(s). |
| Huawei, HiSilicon | As indicated in the second round, we have some doubt on the benefit of non-unifrom UE-UE CLI measurement but we are open to study it. The subband CQI is useful since it can be directly used for scheduling and/or link adptaion while how to make use of the non-uniform UE-UE CLI is not that straightforward since one may need to consider how it is used such as coordinated scheduling. The benefit of coordinated scheduling depends on quite some factor such as traffic load as discussed in 9.3.3. |
| Lenovo | As long as a SBFD aware UE meet RAN4 RF requirements, it is not necessary that the UE report its own emission in a DL subband. UE’s emission level into the DL subband does not reflect an actual UE-to-UE CLI level. We think the proposal is an unnecessary over-optimization feature without clear benefit. |
| QC | We think a study is useful and needed. Depending on the outcome, we can conclude whether there are benefits or not. As shared in the earlier round, the inband leakage of the UE is frequency selective. RAN4 already defined IBE mask that defines the limiat of these leakage on per-RB level. It is very logical that UE measures and report the CLI in different chunks of frequency resources in the DL-subband. This concept is not new and already supported for CSI subband PMI/CQI reporting |

Proposal 3-5

Proposed Agreement:

Study exchange of intended subband configurations for SBFD operation across gNBs for gNB-to-gNB CLI handling.

|  | Company |
| --- | --- |
| Support | New H3C, DOCOMO, Samsung, Sony, Panasonic, Intel, NEC, Nokia, NSB, InterDigital, Lenovo, Qualcomm |
| Not support | Ericsson (still object to standalone proposal) |

| Company | Comments |
| --- | --- |
| Moderator | Ericsson and Huawei still think the proposal should not be a standalone proposal without mentioning how to make use of such this information. Samsung and Nokia are fine with the proposal but do not agree with short-term/OTA exchanges. You can refer to section 4.3 for more information and please provide your further inputs based on the previous comments. |
| vivo | We tend to agree the views with Ericsson and Huawei. |
| Sony | If semi-static SBFD is the only means to configure SBFD, then SBFD format can be exchanged via X-interface. If we introduce dynamic SBFD, then OTA signalling will be beneficial. |
| Intel | We think this is useful for the scenario with different subband configuration among cells. If only semi-static configuration is supported, relatively long-term exchange is sufficient, while we’re open for short-term exchange if dyamic configuration is supported (e.g., dynamic SBFD symbol location). Regarding E and Huawei’s comment, because most dynamic TDD/SBFD common CLI handling solutions are discussed in 9.3.2, while subband configuration information exchange is SBFD specific. So, do we expect to discuss in 9.3.2 and 9.3.3 ? |
| NEC | We are already discussing various issues for dynamic TDD in 9.3.3 which are based on assumption that coordinated scheduling is possible if gNBs are aware of contentious slots/symbols. The same issue is also applicable here. We agree that in semi-static case, nearby gNBs shall support common SBFD configuration but given that we are also discussing different aspects of dynamic SBFD scheduling (e.g. by DL transmission in UL subband or dynamic UL subband scheduling), there are expected to be frequent cases where we fall into similar scenarios as being discussed in 9.3.3 (i.e. co-subband gNB DL to gNB UL CLI). So, such information would be useful to identify set of conflicting symbols/slots and thereby take appropriate measures (e.g. coordinated scheduling) to alleviate the CLI. |
| Nokia, NSB | Same view as 2nd round. Especially, we do not think RAN1 should spend a lot of time to discuss this in the study item phase. Some text could be included in the TR. Once the SBFD signalling framework is agreed during potential normative work on SBFD, RAN3 could then be tasked to extend the inter-gNB signalling framework introduced in Rel-16 for dynamic TDD. |
| Huawei, HiSilicon | Our views have not changed. There is no point to agree on a information exchange solution without knowing how to make use of it. |
| Lenovo | One benefit of semi-static UL subband configuration is that it allows subband coordination among gNBs to manage CLI. Detailed signaling can be discussed in RAN3. |
| QC | We think it is useful to extend the info exhchange to include the intended DL/UL subband configuration for CLI handling especially in the scenario when gNB uses UL-SB for DL transmission by means of SBFD time locations update. |
| Ericsson | Share same view as Huawei. |

- Proposals for online sessions

  - Oct 11th (Tue, Week 1)

Proposal 1-3a

Proposed Agreement:

For SBFD operation at least for RRC_CONNECTED state, it is agreed that SBFD operation Alt 4 is the baseline.

- SBFD operation Alt 4:

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

For SBFD operation at least for RRC_CONNECTED state, support it is agreed that SBFD operation Alt 4 is the baseline. - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

Proposal 1-4a

Proposed Agreement:

For indication of subband locations for SBFD operation for SBFD aware UEs, it is agreed that semi-static configuration of subband time and frequency location is the baseline.

For indication of subband locations for SBFD operation for SBFD aware UEs, support it is agreed that semi-static configuration of subband time and frequency location as is the baseline. - Semi-static configuration of subband time and frequency location in SIB is supported - FFS semi-static configuration of subband time and frequency location in dedicated signaling

Proposal 1-5a

Proposed Agreement:

For indication of subband locations for SBFD operation for SBFD aware UEs,

- Dynamic indication of frequency location and/or size of SBFD subband is not further considered

- Study potential benefits and signaling impacts to support dynamic indication/update of time location of SBFD subband

For indication of subband locations for SBFD operation for SBFD aware UEs, - Dynamic indication of frequency location and/or size of SBFD subband is not supported further considered - FFS Study potential benefits and signaling impacts to support dynamic indication/update of time location of SBFD subband

Proposal 1-7a

Proposed Agreement:

For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required.

- FFS whether frequency location of DL subband(s) are explicitly indicated or implicitly determined.

For semi-static configuration indication of subband frequency locations for SBFD operation, at least support explicit indication of frequency location of UL subband is required. - FFS whether frequency location of DL subband(s) are explicitly indicated or implicatedimplicitly determined.

Proposal 1-8a

Proposed Agreement:

The maximum number of subbands for SBFD operation in an SBFD symbol within a TDD carrier is three for the study in RAN1.

- Note: Support of three SBFD subbands may subject to RAN4’s study and conclusion

- Note: Support of SBFD subband pattern {UDU} is 2nd priority as per RAN guidance.

The maximum number of subbands for SBFD operation in an SBFD symbol within a TDD carrier is three for the study in RAN1. - Note: Support of three SBFD subbands may subject to RAN4’s study and conclusion - Note: Support of SBFD subband pattern {UDU} is 2nd priority as per RAN guidance.

  - Oct 13th (Thu, Week 1)

Proposal 1-11b

Proposed Agreement:

Study impact and potential enhancements of CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subbands.

Proposal 1-13a

Proposed Agreement:

Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols.

Proposal 1-14b

Proposed Agreement:

Identify cases of time domain collision of UL transmission and DL reception in a same SBFD symbol for SBFD aware UEs:

Proposal 3-4b

Proposed Agreement:

Study impact/potential enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency.

Proposal 1-8b

Proposed Agreement:

The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one for the study in RAN1.

- The UL subband can be located at one side of the carrier.

- The UL subband can be located at the middle part of the carrier, subject to RAN4’s study and conclusion

Proposal 1-2a

Proposed Agreement:

For SBFD operation within a TDD carrier, it is agreed that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is the baseline.

Proposal 1-6a

Proposed Agreement:

For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline.

Proposal 1-16

Proposed Agreement:

Study whether SBFD operation in SSB symbols is supported or not.

  - Oct 17th (Mon, Week 2)

Proposal 1-2a

Proposed Agreement:

For SBFD operation within a TDD carrier, it is agreed that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is the baseline.

Proposal 1-8c

Proposed Agreement:

The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one for the study in RAN1.

- The UL subband can be located at one side of the carrier.

- The UL subband can be located at the middle part of the carrier, subject to RAN4’s study and conclusion

Note: RAN1 considers the above two possibilities unless RAN4 concludes that any one is infeasible.

Note: Two UL subbands for SBFD operation in an SBFD symbol within a TDD carrier due to SBFD operation in legacy UL symbols is subject to further RAN1 discussions which is 2nd priority as per RAN guidance.

Send an LS to RAN4 to inform the above agreement.

Proposal 1-18a

Proposed Agreement:

For SBFD aware UEs, in symbols configured as DL in TDD-UL-DL-ConfigCommon with UL subband configured by semi-static configuration,

- UL transmissions within UL subband are allowed

- UL transmissions outside UL subband are not allowed

- A set of contiguous RBs in DL BWPa carrier excluding RBs of UL subband and guardband(s) are defined as a DL subband for discussion purpose

  - FFS whether frequency location of DL subband(s) and guardband(s) are explicitly indicated or implicitly determined

- DL receptions within DL subband(s) are allowed

- FFS whether DL receptions within UL subband are allowed or not

- FFS whether/how the symbols can be converted to DL-only symbols

- FFS: interaction of transmissions and receptions with TDD-UL-DL-ConfigDedicated and dynamic SFI

Proposal 1-19

Proposed Agreement:

UL subband can be configured in symbols configured as flexible in TDD-UL-DL-ConfigCommon for SBFD aware UEs.

Proposal 1-13b

Proposed Agreement:

Study impact and potential enhancements for scheduled/configured UL transmissions and scheduled/configured DL receptions across SBFD symbols and non-SBFD symbols, including at least the following:

- Single slot Scheduled/configured PUCCH/PUSCH/PDSCH/PDCCH without repetition in SBFD symbols and non-SBFD symbols

- SRS/CSI-RS in SBFD symbols and non-SBFD symbols

- CG PUSCH in SBFD symbols and non-SBFD symbols

- TBoMS across SBFD symbols and non-SBFD symbols

- Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols

- PDCCH in SBFD symbols and non-SBFD symbols

- SPS PDSCH in SBFD symbols and non-SBFD symbols

- PDCCH/PDSCH/PUSCH/PUCCH repetitions across SBFD symbols and non-SBFD symbols

- CSI-RS in SBFD symbols and non-SBFD symbols

Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping, if applicable, is considered.

- Contact person

Please provide/update the information of the contact person in the following table to facilitate the discussions.

| Company | Name | Email address |
| --- | --- | --- |
| Sony | Shin Horng Wong | shinhorng.wong@sony.com |
| InterDigital | Jonghyun Park | jonghyun.park@interdigital.com |
| Sharp | Tomoki Yoshimura | yoshimurat@sharplabs.com |
| Qualcomm | Muhammad Abdelghaffar | mabdelgh@qti.qualcomm.com |
| New H3C | Lei Zhou | zhou.leih@h3c.com |
| New H3C | Lei Kong | Kong.lei@h3c.com |
| vivo | Lihui Wang | wanglihui@vivo.com |
| NEC | Pravjyot Singh Deogun | pravjyot.deogun@emea.nec.com |
| Xiaomi | Lei Wang | wanglei25@xiaomi.com |
| OPPO | Wenfeng Zhang | zhangwenfeng@oppo.com |
| Ericsson | Stephen Grant | stephen.grant@ericsson.com |
| Spreadtrum | Huan Zhou | Huan.Zhou@unisoc.com |
| CATT | Yanping Xing | xingyanping@catt.cn |
| Panasonic | Tomoya Nunome | nunome.tomoya@jp.panasonic.com |
| Intel | Yi Wang | yi5.wang@Intel.com |
| ITRI | Jen-Hsien Chen | itriA40175@itri.org.tw |
| Lenovo | Hyejung Jung | hyejung@motorola.com |
| ETRI | Hoondong Noh | hoondong.noh@etri.re.kr |
| ZTE | Xingguang WEI | wei.xingguang@zte.com.cn |
| Samsung | Marian Rudolf Kyungjun Choi | m.rudolf@partner.samsung.com kyungj.choi@samsung.com |
| CMCC | Tuo Yang Fei Wang | yangtuo@chinamobile.com wangfei@chinamobile.com |
| DOCOMO | Qiping Pi | piqp@docomolabs-beijing.com.cn |
| WILUS | David (Geunyoung) Seok | david.seok@wilusgroup.com |
| CEWiT | Priyanka Dey | priyanka@cewit.org.in |
| Nokia, NSB | Jingyuan Sun | Jingyuan.sun@nokia-sbell.com |
| Huawei, HiSilicon | Xinghua Song | songxinghua@huawei.com |
| MediaTek | Mohammed Al-Imari | Mohammed.Al-Imari@mediatek.com |
| LG Electronics | Hyunsoo Ko | hyunsoo.ko@lge.com |
| SK Telecom | Sanghoon Cho | seanc.cho@sk.com |
| KDDI | Masahito Umehara | ma-umehara@kddi.com |
| TCL | Shahid Jan | shahid.jan@tcl.com |
| Fujitsu | Taewoo LEE | lee.taewoo@fujitsu.com |

- Reference

- RP-220633		Revised SID: Study on evolution of NR duplex operation	CMCC

- R1-2205399		TR 38.858 skeleton for study on evolution of NR duplex operation	Rapporteur (CMCC)

- R1-2205520		Summary #2 of [109-e-R18-Duplex-03] Email discussion on subband non-overlapping full duplex	Moderator (CATT)

- R1-2208122		Summary #5 of subband non-overlapping full duplex		Moderator (CATT)

- R1-2208403		Discussion on subband non-overlapping full duplex	TCL Communication Ltd.

- R1-2208409		Discussion on potential enhancement on subband non-overlapping full duplex	Huawei, HiSilicon

- R1-2208484		Discussion of subband non-overlapping full duplex	ZTE

- R1-2208527		Discussion for subband non-overlapping full duplex	New H3C Technologies Co., Ltd.

- R1-2208552		Discussion on subband non-overlapping full duplex	Spreadtrum Communications

- R1-2208641		Discussion on subband non-overlapping full duplex	vivo

- R1-2208857		Discussion on subband non-overlapping full duplex	OPPO

- R1-2208974		Discussion on subband non-overlapping full duplex	CATT

- R1-2209021		Discussion on subband non-overlapping full duplex	Fujitsu

- R1-2209028		Discussion on subband non-overlapping full duplex	InterDigital, Inc.

- R1-2209052		Potential solutions for SBFD in NR systems	Intel Corporation

- R1-2209099		Considerations on Subband Full Duplex TDD Operations	Sony

- R1-2209126		Subband non-overlapping full duplex	Lenovo

- R1-2209175		Subband non-overlapping full duplex	Ericsson

- R1-2209240		Discussion on subband non-overlapping full duplex	Panasonic

- R1-2209284		Discussion on subband non-overlapping full duplex	xiaomi

- R1-2209336		Discussion on subband non-overlapping full duplex	CMCC

- R1-2209403		Discussion on subband non-overlapping full duplex enhancements	ETRI

- R1-2209421		Discussion on subband non-overlapping full duplex	NEC

- R1-2209583		Views on subband non-overlapping full duplex	Apple

- R1-2209729		SBFD feasibility and design considerations for NR duplex evolution	Samsung

- R1-2209770		Discussion on subband non-overlapping full duplex for NR	MediaTek Inc.

- R1-2209809		Study on Subband non-overlapping full duplex	LG Electronics

- R1-2209902		Discussion on subband non-overlapping full duplex	NTT DOCOMO, INC.

- R1-2209930		Discussion on subband non-overlapping full duplex	Sharp

- R1-2209983		Feasibility and techniques for Subband non-overlapping full duplex	Qualcomm Incorporated

- R1-2210030		Discussion on sub-band non-overlapping full duplex	ITRI

- R1-2210042		On subband non-overlapping full duplex for NR	Nokia, Nokia Shanghai Bell

- R1-2210093		Details of subband non-overlapping full duplex	ASUSTeK

- R1-2210108		Discussion on subband non-overlapping full duplex	CEWiT

- R1-2210138		Discussion on subband non-overlapping full duplex	WILUS Inc.

- R1-2210143		Inter-UE CLI Test Results for NR Duplex Evolution	KDDI Corporation

Appendix A: Observations/proposals from companies in RAN1#110bis-e

| R1-2208403 | Discussion on subband non-overlapping full duplex | TCL Communication Ltd. |
| --- | --- | --- |
| Observation 1: Re-using the existing configuration of conventional TDD for SBFD configuration may requires; - To include an explicit configuration of NRofUplinkRBs, and startingofUplinkRBs for UL sub-band in DL or flexible slots/symbols. - To include an explicit configuration of NRofDownlinkRBs, and startingofDownlinkRBs for DL sub-band in UL or flexible slots/symbols. Observation 2: Re-using the existing conventional TDD configuration for SBFD operation may encounter the following issues - Increas the higher layer singlaings - Increase the gNB complexity in handling of collisin btween SSB and UL sub-band tranmission Observation 3: Static configuration and dynamic indication and activation/de-activation of DL/UL sub-bands to the UE may - Reduce the higher layer signlaing - Increase the flexibility of gNB to handle the collision of SSB and UL sub-band trasnmission Observation 4: Allocation of frequency resources to a sub-band in the inner RBs or inner carrier of a TDD band may; - Reduces the interference from the adjacent gNB but it may increase the gNB self-interference - Requires at least three sub-bands within a TDD band - Requires two guard bands to separate the opposite direction sub-bands. Observation 5: Allocation of frequency resources to a sub-band in the outer RBs or outer carrier of a TDD band may; - Increase the interference from the adjacent gNB but reduces the gNB self-interference - Defines flexible numbers of sub-bands e.g. either two sub-bands or three sub-bands - Requires only one guard band. Observation 6: The UL sub-bands in DL or flexible slots/symbols in the inner carriers or inner RBs of a TDD band may create double fold gNB self-interference. Observation 7: The UL sub-bands in DL or flexible slots/symbols in the edge RBs may reduce the gNB self-interference. Observation 8: Less number of sub-bands within a TDD band may reduce the gNB self-interference. Proposal 1: Support Alt4 for subbands location indication to the SBFD aware UE. Proposal 2: study an alternative method of SBFD configuration, where RRC static configuration can be used to configure a pool of sub-bands to all the SBFD capable UEs in a cell and DCI or MAC CE can be used to indicate and activate/de-activate the DL/UL sub-bands to one or more UEs in a cell. Proposal 3: Consider a configurable bandwidth of DL and UL sub-bands. Proposal 4: Consider a minimum numbers of subbands as 2 and a maximum numbers of subbands as 3, within a TDD band. Proposal 5: It is up to the gNB implementation whether to select the inner carriers or outer carriers of a TDD band for an opposite direction sub-bands in DL, UL or flexible slots. Proposal 6: To avoid the intra sub-band interference in SBFD operation the neighbor’s gNB shall: - Keep the same bandwidth of UL to UL and DL to DL sub-bands among the neighbor gNBs - Assign alike sub-bands to UL and DL transmission among the neighbor gNBs - Allocate similar quantity of sub-bands to DL and UL transmission among the neighbor gNBs - Configure similar numbers of sub-bands among the neighbor gNBs |  |  |
| R1-2208409 | Discussion on potential enhancement on subband non-overlapping full duplex | Huawei, HiSilicon |
| Observation 1: SBFD operation across carriers does not require specific collision handling rules than the ones that also required by SBFD operation within a carrier. Observation 2: It seems that the L3 based UE-UE CLI measurement and report is sufficient and L1 based solutions may need to be justified in the study as well as other enhancement on UE-UE measurement and reporting. Proposal 1: SBFD operation within a carrier and across carriers can be studied with equal priority. Proposal 2: Confirm the Working Assumption in RAN1#110, i.e., for SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline. - FFS RB-set based subband configuration Proposal 3: The number of subbands in the same symbol for SBFD depends on inter-operator co-existence. - The transmission direction on a subband of an operator with SBFD should be same as that of another operator with legacy TDD, if the subband of the operator with SBFD is located neighboring to the operator with legacy TDD in frequency domain. - At most three subbands are sufficient in the same symbol, where two UL subbands and one DL subband, or one subband and two DL subbands. Proposal 4: Define the UL subband and DL subband as follows: - UL subband should be defined as a set of consecutive RBs which is only used for UL transmission; - DL subband should be defined as a set of consecutive RBs which is only used for DL transmission. Proposal 5: Study the flexible configuration and indication of DL and UL subband to facilitate flexible scheduling of frequency domain resources. Proposal 6: Study the following aspects related to advanced IRC receivers in SBFD. - Feasibility and performance of muting the REs on the DL subband in UL DMRS symbols and the REs on the UL subband in DL DMRS to improve channel estimation and inter-cell interference estimation and suppression. - Feasibility and performance of specific CLI measurement resources to improve gNB-gNB and UE-UE co-channel and adjacent-channel inter-subband CLI estimation and suppression. Proposal 7: Study the feasibility and performance of spatial domain coordination in SBFD to suppress blocking interference caused by DL signal transmitting from the aggressor in the direction of the victim, e.g., coordinated beamforming. Proposal 8: Study the feasibility and performance of applying filters at both transmitter and receiver sides in SBFD involving RAN4 on the following aspects. - Filter at transmitter to suppress the leakage interference. - Filter at receiver to suppress the blocking interference. - Guard band for filters. Proposal 9: For subband non-overlapping full duplex, the timing advance offset NTA,offset can be configured as 0 to avoid the inter-slot interference. Proposal 10: UE half-duplex on handling conflict UL/DL indicating signaling for the same OFDM symbol should be studied, e.g., - Any DL signals or channels are indicated to transmit on the DL subband in a symbol configured as semi-U. - Any UL signals or channels are indicated to transmit on the UL subband in a symbol configured as semi-D. - Any UL signals or channels are indicated to transmit on the UL subband in a symbol indicated to receive SSB on the DL subband. - SSB is indicated to receive on the DL subband in a symbol configured as semi-U. - Valid PRACH is indicated to transmit on UL subband in a symbol configured as semi-D. - CORESET 0 configured in the DL subband in a symbol configured as semi-U. |  |  |
| R1-2208484 | Discussion of subband non-overlapping full duplex | ZTE |
| Overview Proposal 1: Prioritize the subband full duplex simulation and use the simulation results to guide and focus the solution/scheme discussion. Proposal 2: The solution/scheme of duplex evolution takes the following challenges of conventional TDD operation into account. - Challenge : Ensuring UL throughput + UL coverage simultaneously. - Challenge : Ensuring UL throughput + DL&UL Latency simultaneously. - Challenge : Ensuring UL coverage + DL&UL Latency simultaneously. - Challenge : Ensuring DL throughput + DL&UL Latency simultaneously. - Challenge : Ensuring DL throughput + UL throughput simultaneously. - Challenge : Ensuring DL throughput + UL coverage simultaneously. Basic framework of SBFD configuration Observation 1: Alt.1 (transparent method with no specification impact for SBFD) has the following two options: - Option.1: Base station configures flexible symbols/slots to the UE via TDD-UL-DL-ConfigCommon and schedules DL/UL on these flexible symbols/slots according to existing rules. - Option.2: Based station configures UE-specific TDD slot formats for different UEs via TDD-UL-DL-ConfigDedicated and schedules DL/UL based on UE-specific TDD slot formats according to existing rules. Observation 2: Alt.2 (Transparent method without subband configuration for SBFD) has the following two options: - Option.1: Allocating some time/frequency resources in DL/flexible symbols for UL transmission Base station allocates some fixed time/frequency resources in the DL symbols (or DL symbols plus flexible symbols) via implementation. UE transmits UL transmission in the fixed time/frequency resources of the DL symbols according to gNB’s scheduling. - Option.2: New TDD slot format for SBFD overrides the legacy one Legacy UE follows the TDD slot format indicated by the legacy SIB or legacy UE-specific signalling. A new UE-specific TDD slot format is introduced for the SBFD UE, which can override the legacy TDD slot format indication/configuration. UE transmits UL transmission based on new UE-specific TDD slot format according to gNB’s scheduling/configuring. Observation 3: Compared with Alt.2, Alt.3 has higher spec impacts but without adding much value via indicating time location of subband only. Proposal 3: RAN1 studies the following solution for subband full duplex: configure one/multiple subbands in one BWP. - All the resource in the uplink subband are available for uplink transmission even if the uplink subband is overlapped with downlink or flexible symbols. Proposal 4: RAN1 studies the following solution for subband full duplex: support dual active BWP pairs, where each BWP pair includes one DL BWP and one UL BWP. - Different BWP pairs can be configured with different TDD slot configurations. Proposal 5: RAN1 studies half-duplex CA based scheme for sub-band full duplex and taking directional conflict handling mechanism in R16 half-duplex CA as a starting point. - Further study the necessity of enhancement to directional conflict handling mechanism if time allows in R18. Proposal 6: Regarding the 4 SBFD operation Alts reached in RAN1#110 meeting, focus the further study in Alt.4 and Alt.2. - Note: The study of Alt.3 can be postponed or deprioritized. Details of SBFD framework Proposal 8: Regarding the time/frequency-domain configuration of subband, - The location and bandwidth in frequency domain of subband can NOT be updated dynamically. - Further study the feasibility and benefits of dynamic update of subband time-domain location. Proposal 9: RAN1 studies the following solution for subband full duplex: - Time domain window can be defined for subband full duplex operation for better compatibility with less impact on legacy UE and procedures. Proposal 10: Study potential enhancements for collision handling between UL and DL for SBFD aware UE. Proposal 11: For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, prioritize the study of following options: - Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol - Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol FFS: Whether DL transmission is allowed in all the UL subband resources or only a certain area. FFS: gNB transmits DL reference signal in UL subband. Proposal 12: RAN1 further studies the resource allocation of relevant physical channels/signals in frequency domain and time domain for SBFD. - E.g., whether to have the same or separate UL configurations for PUCCH/PUSCH/SRS for UL subband and UL slots. Proposal 13: RAN1 further studies the potential enhancements for initial access in the UL subband. CLI management and cancellation Proposal 14: For SBFD, different options of TA offset determination for avoiding inter-slot interference should be studied, - Option 1: set NTA,offset=0 for all UL resource; - Option 2: define two values of NTA,offset for UL transmission in UL subband and UL slot, respectively. - NTA,offset=0 for UL subband - NTA,offset>0 for UL slot Observation 4: The uplink transmissions in the UL subband are subject to different interference levels due to the following aspects, - the frequency domain isolation between it and the DL subband. - time domain areas where DL transmissions are scheduled or configured in DL subband. Proposal 15: UL subband resources can be divided into multiple areas and each area is mapped with a dedicated power control parameter set for compensating the inter-subband interference with different levels. - The resources contained in each area can be indicated by DCI. Proposal 16: Further study subband CLI measurement and reporting for UE-to-UE CLI handling under SBFD, e.g., configuration and determination of the measurement subband size and measurement reporting overheads reduction, etc. Proposal 17: Different frequency densities can be configured for reference signals transmitted in different areas with different interference levels. |  |  |
| R1-2208527 | Discussion for subband non-overlapping full duplex | New H3C Technologies Co., Ltd. |
| Proposal 1: SBFD operation Alt.4 can be studied with higher priority, then Alt1 and Alt2 as lower priority. Alt3 is removed from the list. Proposal 2: SBFD operation Option 1 and Option 2 can be studied with high priority. Proposal 3: Remove SBFD operation Option 3 from the list, and keep studying SBFD operation option 4. Proposal 4: For DL, the similar options should be studied, and Option3 has higher priority than Option2, then, Option1. Proposal 5：Extend the functionality of flexible symbol for supporting SBFD operation, and the frame structure configuration and slot format indication mechanism in legacy TDD can be reused. Proposal 6：Support the configurations of a number of dedicated symbols as SBFD symbols, the dedicated SBFD symbols should be consecutive in a period of the frame structure. Proposal 7: Support dynamic SBFD resource allocation and coordination between cells for evolution of NR duplex operation by DCI. Proposal 8: RO for Type-1 random access procedure is supported to be configured in semi-static SBFD resource. Proposal 9: MsgA RO and MsgA PUSCH for Type-2 random access procedure is supported to be configured in semi-static SBFD resource. Proposal 10: The SSB overlapping with SBFD sub-band should carefully studied. The SSB can be configured inside the SBFD sub-band in the legacy UL symbols, and the SSB can be configured overlapping with SBFD sub-band in legacy Flexible or DL symbols. Proposal 11: The CORESET overlapping with SBFD sub-band should carefully studied. In In legacy UL symbol with SBFD configured, only CORESET0 can be configured inside the SBFD sub-band, but has lower priority than RO. Proposal 12: In legacy F symbol with SBFD configured, common CORESET has priority, then RO, then UE-specific CORESET. In In legacy DL symbol with SBFD configured, CORESET has priority. Proposal 13: UE is dynamically scheduled to transmit PUSCH w/o repetition in semi-static SBFD time-frequency resource in connection mode. Proposal 14: UE is dynamically scheduled to transmit PUSCH w/o repetition in dynamic allocated SBFD time-frequency resource in connection mode. Proposal 15: UE is scheduled to transmit SDT PUSCH in SBFD time-frequency resource in inactive mode. Proposal 16. UE is scheduled PDSCH in the DL SBFD resource. The confliction between PDSCH and UL channels and signals should be further studied. |  |  |
| R1-2208552 | Discussion on subband non-overlapping full duplex | Spreadtrum Communications |
| - For Type-0 and Type-1 resource allocation without interleave, with the schedule restriction of Option 1-1/1-2, gNB can decide available frequency resources for PDSCH or PUSCH, without specification impact. - Type-0 and Type-1 resource allocation without interleave, there can be some enhancements on resource allocation for Option 2 to improve the frequency efficiency. - For Type-1 resource allocation with interleave, it is very hard to do schedule either for Option 1-1/1-2, or Option 2. - In co-channel co-existence case of legacy UE/gNB and SBFD, there is no impact to the legacy gNB, but impact to legacy UE. CLI mitigation scheme for aggressor SBFD UEs should be studied. - Both for R18 duplex operation enhancement and R17 RedCap, to achieve good co-existence performance with legacy network/UE, the bandwidth part location needs to be carefully designed. - DL resource fragmentation caused by NR duplex operation may not be friendly to legacy UEs from UE power consumption perspective. - Regarding the informing method of time locations of subbands for SBFD operation, new signalling is a proper choice. - Regarding the informing method of frequency locations of subbands for SBFD operation, the starting PRB and length of continuous PRB number is enough. - For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, the SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol - For SBFD operation within a TDD carrier when configured in legacy downlink and flexible symbols, it can support one UL subband and up to two DL subbands within the carrier from gNB’s perspective. - SBFD operation across carriers can be supported if the general framework of SBFD within a TDD carrier can be applied. - Confirm this part in the WA: - For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline. - SBFD scheme with more than one configured DL and UL BWP pair with aligned center frequencies for a DL and UL BWP pair - L1-based CLI measurement/report for SBFD needs further study. - Subband-level information can be considered for gNB-to-gNB’s information sharing. - Dynamic CLI measurement/report among gNBs needs further study. - The introduction of SBFD operation should study co-existence performance with the legacy NR operation including RedCap deployment, which allow network to configure SBFD and RedCap BWP in a legacy NR carrier simultaneously, meanwhile minimizing both UL resource fragmentation and DL resource fragmentation. |  |  |
| R1-2208641 | Discussion on subband non-overlapping full duplex | vivo |
| Observation 1: SBFD operation Alt 1 requires the network to configure a large number of semi-static flexible symbols to accommodate SBFD resources. Observation 2: The legacy UEs may not be fully tested with a large number of semi-static flexible symbols in real commercial deployments, resulting in potential interoperability issue. Proposal 1: For legacy UEs, SBFD operation and resources should be transparent by using and following existing RAN1 specification. Proposal 2: Confirm the following working assumption with revised description for SBFD operation within a TDD carrier to avoid ambiguity. For SBFD operation within a TDD carrier, study RB-set based SBFD scheme where a RB set consists of one set of contiguous RBs within a single configured DL and UL BWP pair with aligned center frequencies as baseline. - FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies - FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair Proposal 3: A subband partition with DL subband(s) located at one side of the carrier and UL subband(s) located at the other side, can be considered for deployment scenarios with no or limited adjacent channel coexistence issue. Proposal 4: A subband partition with UL subband(s) located at the middle part of the carrier in a SBFD symbol (i.e., semi-static DL/flexible symbol configured by tdd-UL-DL-ConfigurationCommon and/or tdd-UL-DL-ConfigurationDedicated) can be considered for deployment scenarios where there is adjacent channel co-existence requirement. Proposal 5: Both semi-static DL symbols and semi-static flexible symbols are considered as candidates to be configured as SBFD symbols. Proposal 6: The gNB configures the location and bandwidth for one or more subbands, and the following two options can be considered: - Option 1: More than one subband can be configured and the transmission direction for each subband is also configured. - Option 2: Only the location of the UL subband is configured, without configuring the transmission direction outside the UL subband. Proposal 7: In case for frequency domain pattern of SBFD only the location of the UL subband is configured, for SBFD operation in semi-static flexible symbol(s), resources in flexible subband(s) can be used either as UL or DL. Proposal 8: Study the following two options for configuring the SBFD symbol(s) with a certain periodicity: - Option 1: The semi-static DL/flexible symbols within a period determined by periodicity of the configured TDD pattern can be configured as SBFD symbols. - Option 2: The periodicity and SBFD symbols within the periodicity can be explicitly configured. Proposal 9: RAN1 first discusses the desired SBFD operation, e.g., whether a symbol can be determined dynamically as a SBFD symbol or non-SBFD symbol before discussing the detailed signaling design and related UE behavior. Proposal 10: Study dynamic indication of whether or not to apply the SBFD frequency pattern in time domain. Proposal 11: Study potential enhancement to determine transmission direction for a symbol configured/scheduled with both UL transmission and DL reception. Proposal 12: Study potential enhancement for collision handling where a UL transmission occasion/DL reception occasion overlapping with at least one SBFD symbol. Proposal 13: Different transmission/reception parameters can be configured and applied to SBFD slot and UL/DL slot to better match conditions of different slot types for better performance. Proposal 14: Study potential enhancements for CSI measurement and reporting for SBFD. Proposal 15: No UE RF impact for CLI handling is expected to avoid additional UE complexity in Rel-18 SBFD operation. |  |  |
| R1-2208857 | Discussion on subband non-overlapping full duplex | OPPO |
| Observation 1: The legacy UE can run with half-duplex mode in SBFD symbols with both time-domain transparency and frequency-domain transparency to UL subband allocation at gNB. Observation 2: The SBFD subband allocation does not have to be the same over adjacent carriers, but the adjacent edge subbands are desired to have the same transmission direction. Proposal 1: Use a new terminology such as “duplexing subband” to replace “subband” in SBFD discussion. Proposal 2: Interaction between BWP operation and SBFD operation needs to be studied. Proposal 3: UL subband can be explicitly configured in DL symbol and flexible symbol. Proposal 4: The amount of RBs used to determine the TB size does not count the RBs overlapping with UL subband and the guard bands (if any) around the UL subband. Proposal 5: Considering that different slot types in multiple repetition slots, PUCCH/PUSCH/PDSCH repetition enhancement needs to be studied. Proposal 6: RAN1 study considers the subband-overlapping symbol as converted to “flexible symbol” or “uplink symbol” from UE perspective, and avoids defining UL operations directly on a DL symbol. |  |  |
| R1-2208974 | Discussion on subband non-overlapping full duplex | CATT |
| Observation 1: For semi-static transmission and repetitions other than the first repetition, gNB may not be able to avoid indication of UL transmission in DL subband or DL reception in UL subband. Observation 2: The following prerequisite are observed for SBFD scheme with more than one configured DL and UL BWP: - Multiple BWPs should be supported by SBFD aware UEs. - Shorter BWP switching delay or multiple active BWPs should be supported by SBFD aware UEs to reduce the interruption time. - Potential UE throughput limitation for SBFD capable UE in full DL/UL slot. Observation 3: There are many restrictions in terms of resource allocation if subband location is not known to UEs. Observation 4: PRACH in SBFD symbols is not feasible at least for Alt 1. Observation 5: SBFD operation in SSB symbols is not feasible at least for Alt 1. Observation 6: RBs for Msg 3 PUSCH and PUCCH during initial access may be located outside UL subband if time and frequency location of UL subband is not known to the UE. Observation 7: The RSIC requirement based on 1dB sensitivity degradation due to self-interference can be met based on the values provided in RAN4 reply LS. Observation 8: If different NTA,offset values are supported for a SBFD aware UE in SBFD symbols and non-SBFD symbols, a gap should be reserved in UL subband of flexible symbols to avoid the overlapping between different UL symbols. Observation 9: For gNB-gNB inter-subband CLI measurement of SBFD operation, there is no need to define dedicated CLI RS. Observation 10: For UE-UE inter-subband CLI measurement of SBFD operation, there is no need to define dedicated CLI RS. Proposal 1: For indication of subband locations for SBFD operation, study cell-common subband time and frequency location as baseline. Proposal 2: For SBFD operation within a single DL BWP and UL BWP pair, UL subband location indication in SIB can be supported. - Frequency location of UL subband, if supported in SIB, is with reference to a common RB - Time location of UL subband, if supported in SIB, is periodic and the periodicity can be explicitly configured or implicitly determined based on the periodicity of SSB or TDD UL-DL configuration Proposal 3: FFS whether time and frequency locations of DL subband for SBFD operation are explicitly indicated or implicitly derived based on time and frequency locations of UL subband and guard band information. Proposal 4: For SBFD operation Alt 4, study option 1 as baseline for SBFD aware UEs with the following update. - Option 1: The SBFD aware UE does not transmit UL outside the UL subband or receive DL within the UL subband in the SBFD symbol. Proposal 5: Confirm the following working assumption in RAN1#110. For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline. - FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies - FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair Proposal 6: Study assignment of fractional RBGs at subband boundaries for PDSCH with RA type 0. Proposal 7: Study non-contiguous FDRA across DL subbands for PDSCH with RA type 1. Proposal 8: Study assignment of fractional RBGs at subband boundaries for PUSCH with RA type 0. Proposal 9: Study frequency hopping for PUSCH due to different available frequency resources in SBFD symbols and non-SBFD symbols for PUSCH. Proposal 10: Study FDRA for DL and UL due to different available frequency resources in SBFD symbols and non-SBFD symbols for PDSCH/PUSCH/PUCCH with repetition. Proposal 11: For PUCCH, study separate/shared configurations for transmissions/receptions in SBFD symbols and non-SBFD symbols. Proposal 12: Study non-contiguous FDRA across DL subbands for CSI-RS. Proposal 13: Study CSI report for fractional CSI report subband at DL subband boundaries. Proposal 14: Study separate SRS resource configurations for SBFD symbols and non-SBFD symbols.. Proposal 15: Discuss whether PRACH in SBFD symbols is supported. Proposal 16: Discuss whether SBFD operation in SSB symbols is supported. Proposal 17: For collision between UL transmission and DL reception, study potential enhancements for determining the transmission direction. Proposal 18: Study potential enhancements for initial access in UL subband. Proposal 19: Study NTA,offset=0 for SBFD symbols and NTA,offset>0 for non-SBFD symbols for SBFD aware UEs. Proposal 20: For gNB-gNB inter-subband CLI measurement, study reserve resources in DL subband for CLI measurement in UL subband. Proposal 21: Further study the following information exchange between gNBs: - CLI measurement resources - planned transmission resources - beam related informationm - RSSI or SINR (together with beam information). Proposal 22: RSSI or SINR can be included in the SBFD specific CLI report. |  |  |
| R1-2209021 | Discussion on subband non-overlapping full duplex | Fujitsu |
| Observation 1: - It is observed that the interoperability issue may occur in the legacy UEs when the SBFD operation is performed on Flexible slots. Proposal 1: - It is better for RAN1 to have a common understanding the legacy UE behaviour on Flexible slots which is forward-compatible with the SBFD aware UE. Observation 2: - Due to the CLI from PRACH transmission procedure on UL subband, the overall system performance will get decreased. Proposal 2: - PRACH coverage enhancement is better to be discussed in further NR coverage enhancement agenda (NR_cov_enh2) Proposal 3: - The link direction of UL subband should be based on Option 1 and Option 2 from previous agreement. - Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol - Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol Proposal 4: - The clarification is needed for the SBFD aware UE operation with UL transmission outside the UL subband and DL transmission in the UL subband. Proposal 5: - The slot for UL transmission outside UL subband or DL reception within UL subband should be set as UL only or DL only, respectively. |  |  |
| R1-2209028 | Discussion on subband non-overlapping full duplex | InterDigital, Inc. |
| Observation 1. Mixed D/U regions informed to a UE per symbol/slot in a cell can be used for a subband-wise UL transmission or DL reception, which results in UL coverage/capacity enhancement while achieving parallel DL transmissions over non-overlapped RBs. Observation 2. Using SBFD schemes with more than one configured DL and UL BWP pair, where each BWP pair is associated with a SBFD configuration could result in potential benefits in scheduling flexibility, optimal resource allocations, lower latency, and so forth. Observation 3. DL throughput performance suffers considerably at high MCSs and approaches to almost zero when there is any degree of intra-subband CLI overlap with DL signal. Observation 4. Inter-carrier interference, resulting from time advance on inter-subband CLI may impact DL throughput performance significantly, especially at high MCS indices, when there is no adjacent inter-subband distance between the DL signal and the CLI, i.e., 0-RB gap. Observation 5. At high MCS indices, DL throughput performance recovers to 90% of the maximum throughput when the inter-subband distance between the DL signal and the CLI signal is at least 2-RB in this example scenario. Observation 6. In addition to the frequency gap between the DL signal and the CLI (UL) signal, the amount of frequency resources (PRBs) allocated to the DL signal relative to that for the CLI signal also impacts the normalized throughput performance significantly. Observation 7. Restricting DL subband transmissions on slots that correspond to UL slots in legacy TDD can improve uplink performance but negatively impacts downlink performance. Observation 8. The static/fixed subband partitioning, e.g., [DUD] = [40 20 40] RB split all the time, results in worse performance for SBFD compared with legacy TDD in downlink. Thus, flexible/versatile subband partitioning and its dynamic indication mechanisms should be considered to cope with varying traffic/channel conditions. Observation 9. Utilizing SBFD option 2 (total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD) improves SBFD performance. Observation 10. In the SBFD scenario, a UL transmission over a subband causes significant UE-to-UE CLI leakage on the adjacent DL subband depending on a frequency gap between the UL RBs and DL RBs of each subband. Observation 11. As a part of gNB implementation, the gNB may apply a downlink power backoff on some SBFD slots or symbols to deal with self-interference caused by the FD operation, which can impact to UE behaviours depending on the amount of power backoff. Observation 12. Inter-subband UE-to-UE CLI measurement in SBFD slots based on measuring over configured RB resources and averaging may result in down-estimation, as the subband-edge RBs experience higher CLI compared to RBs in the middle of the subband. Observation 13. Since a general CSI/beam reporting in NR is not based on dynamic CLI-related information, a victim UE may unpredictably experience DL performance degradation if a UE-to-UE CLI occurs especially when an aggressor UE is served by a different serving gNB/TRP. Observation 14. The issues in UL/DL timing alignment (between UL/DL SBs) in SBFD slots could result in inter-slot interference and dropping of respective slots, especially for back-to-back scheduling cases between DL and UL. Observation 15. In case the scheduled frequency resources for an UL transmission are located outside of the boundaries of the SBFD UL subbands, the UE may need further configurations to re-interpret the frequency resources to be mapped within the SBFD UL boundaries. Proposal 1. Study mechanisms on how to inform UE of mixed D/U regions per symbol/slot as an enhancement of SFI to achieve subband non-overlapping FD (SBFD). - Consider the cases where multiple BWPs/subbands are configured for SBFD within the same carrier. Proposal 2. Study the aspects of BWP switching for SBFD schemes with more than one configured DL and UL BWP pair, where each BWP pair is associated with a SBFD configuration. Proposal 3. Study performance of applying a frequency gap or guard RBs for a UL transmission in an SBFD framework for interference mitigation with regards to adjacent DL subbands. Proposal 4. Analysis on various downlink performance degradation aspects due to the SBFD operations compared with legacy TDD systems should also be an important part of the NR-Duplex study. Proposal 5. To overcome the degraded downlink performance due to the static/fixed subband partitioning, flexible/versatile subband partitioning and its dynamic indication mechanisms should be further discussed to cope with varying traffic/channel conditions. Proposal 6. Study dynamic UL power control mechanism based on some dynamic factors such as the frequency gap, beam/spatial-domain parameter, or a priority indication on the UL, to mitigate the effects of the CLI dynamically. Proposal 7. Consider mechanisms to apply measurement skipping on some SBFD slots/symbols and power adjustment in deriving a CSI, depending on a level of dynamic power management occurred in the SBFD scenario. Proposal 8. Study measurement resources and reporting configurations for subband-edge CLI measurement. Proposal 9. Study an event-based CLI sensing behaviour at the victim UE side, where the event can at least include a case when the victim UE detects a PDSCH reception failure, which initiates a subband-wise CLI measurement/reporting for a subband switching to avoid the CLI. Proposal 10. Study UL/DL timing alignment issues in subband non-overlapping full duplex systems. Proposal 11. Study the feasibility and potential benefits of providing supplementary configurations to the UE to be used in case PUCCH/PUSCH frequency resources are mapped (partially or totally) to outside of the SBFD UL boundaries. |  |  |
| R1-2209052 | Potential solutions for SBFD in NR systems | Intel Corporation |
| Observation 1: SBFD operation with DL subband in a legacy UL symbol impacts legacy gNB/UE, which leads to UL resource fragmentation, may affect Msg 4 PUCCH transmission and PRACH transmission, degrades received SINR of UL reception due to co-channel gNB-to-gNB CLI interference, and may decrease frequency diversity gain for typical configurations. Observation 2: Transparent SBFD operation (Alt 1) by reusing existing mechanism can enable SBFD operation in flexible symbols, but the benefit of SBFD would be marginal due to poor spectrum efficiency, complicated scheduler at gNB side, SFI monitoring burden at UE side, increased signalling overhead, limited use case depending on UE capability and number of flexible symbols, infeasibility of UE RF/digital subband filtering. Observation 3: Non-transparent SBFD operation relying on dynamic scheduling (Alt 2) can enable SBFD operation in flexible symbols and DL symbols, but the benefit of SBFD would still be limited due to poor spectrum efficiency at least for configured channels/signals, complicated scheduler at gNB side, SFI monitoring burden at UE side, increased signalling overhead, and infeasibility of UE RF/digital subband filtering. Observation 4: Non-transparent SBFD operation with time domain information for subband (Alt 3) can enable SBFD operation in flexible symbols and DL symbols for both dynamically scheduled and configures signals/channels, but the benefit of SBFD may be limited for cases involving frequency hopping, PUSCH/PUCCH repetitions, non-contiguous FDRA across DL subbands for PDSCH/CSI-RS, and UE RF/digital subband filtering. Observation 5: Non-transparent SBFD operation with both time and frequency domain information for subband (Alt 4) can enable SBFD operation in flexible symbols and DL symbols for both dynamically scheduled and configures signals/channels, providing best performance for SBFD comparing with other alternatives. Proposal 1: Deprioritize SBFD in a UL symbol in which at least one legacy UE transmits UL, or in a UL symbol which is semi-statically configured as UL for at least one legacy UE. - Note: Here, a “legacy UE” refers to a UE that does not support SBFD operation. Proposal 2: Support SBFD operation with UL subband in DL and flexible symbols configured by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated. - FFS whether support SBFD operation with UL subband in a cell-specific flexible symbol & UE-specific DL symbol for a SBFD-capable UE. Proposal 3: Support up to one UL subband and up to two DL subbands within a carrier from gNB’s perspective. Proposal 4: Further study dynamic subband location, with dynamic time location only as baseline. Proposal 5: Study methods for indication of guard bands between DL and UL subbands to a UE and related UE behaviour. Proposal 6: Support non-transparent SBFD operation with both time and frequency domain information for subband (Alt 4) as baseline. Further study details of the following schemes for Alt 4: - Scheme 1: Time & Frequency Set based SBFD with aligned centre frequency for a DL/UL BWP pair as baseline - Signalling design to indicate time and frequency resource - Mechanisms to resolve the collision between DL/UL signals and UL/DL subband - Scheme 2: BWP-based SBFD - Fast inter-BWP or intra-BWP (between DL/UL BWP with same BWP index) switching with or without aligned centre frequency of DL/UL BWP with same BWP index. - Enhancements to avoid interruption/dropping of signals due to BWP switching and handling configurations common across different UEs, e.g., PRACH or CSS. - Scheme 3: CA-based SBFD - Overlapped carriers for CA - Cross-carrier transmission, transmission over multiple carriers and enhancement to resolve collision between carriers. - Focus on scenarios wherein multiple component carriers may be configured within a significantly wide frequency band. Proposal 7: Study SBFD operation for RRC idle/inactive mode. - Study Msg 1 PRACH/PUSCH in UL subband with consideration of challenge for UE-to-UE CLI handling caused by uncertainty of UE transmitting Msg 1 PRACH/PUSCH. - Study PUSCH/PUCCH in UL subband scheduled by gNB, e.g., Msg 3 PUSCH and PUCCH for Msg 4 PDSCH. Proposal 8: Study potential enhancements to UE behaviour for collision handling between DL reception/UL transmission in symbols with at least one UL and one DL subband, considering different DL/UL channels and signals, configuration and scheduling timelines, requirements for different traffic/QoS, and coexistence with legacy UEs. Proposal 9: Study potential enhancements for resource allocation and L1 procedures to support efficient SBFD operation. - The resource allocation includes at least resource for PDSCH/PDCCH/PUSCH/PUCCH/CSI-RS/SRS, and PRACH if SBFD operation for initial access is supported. L1 procedure includes at least CSI measurement /report, power control, DMRS bundling UL timing and DL/UL switching. - For SBFD operation Alt 2/Alt 3/Alt 4, enhancements include separate parameter configuration for different set of time domain resources, and handling for back-to-back DL/UL signals/channels without sufficient switching time or overlapped UL transmissions. Enhancement for SBFD operation Alt 4 also includes resource allocation and L1 procedure impacted by non-continuous frequency resources, collision with DL/UL subband (including miss-alignment between DL/UL subband and FDRA resource allocation unit, and CSI reporting subband), different available frequency resources in SBFD and non-SBFD symbols. Proposal 10: Study enhancement for CLI measurement resource configuration/report mechanism to support single CLI measurement for non-contiguous PRBs in multiple DL subbands. Proposal 11: Study L1-based procedures and beam-based CLI measurement/report and coordination for inter-UE CLI handlings well as enhancements for information exchange between gNBs with DL/UL subband information to improve L3 based CLI handling. Proposal 12: Study potential enhancements for DL/UL subband information exchange for gNB-to-gNB CLI handling. |  |  |
| R1-2209099 | Considerations on Subband Full Duplex TDD Operations | Sony |
| Observation 1: Option 1, where the UE performs UL & DL transmissions only within UL subband and DL subband respectively, is the expected behaviour of a UE with SBFD locations awareness (Alt 4). Observation 2: Option 2, Option 3 and Option 4 where UL transmissions can occur outside the UL subband and/or DL transmission can occur within the UL subband, defeat the purpose of having the UE aware of SBFD locations. If flexibility is required, consider Alt 2 SBFD operation, where the UE does not need to be aware of SBFD locations and instead uses an “Overwrite” indicator to overwrite semi-static configured UL & DL symbols. Observation 3: For the {DUD}, subband configuration, FDRA Type 1 cannot schedule a PDSCH to occupy both DL subbands since it only allocates contiguous sets of RBs for a PDSCH. Observation 4: For the {DUD} subband configuration, FDRA Type 0 can be used to schedule a PDSCH to occupy RBs in both DL subbands. However, since RBG is the unit of allocation, FDRA Type 0 has a coarser frequency granularity compared to FDRA Type 1 and if the finer RBG granularity (i.e. RBG Configuration#1) is used, then FDRA Type 1 consumes more DCI bits compared to FDRA Type 0. Observation 5: FDRA Type 0 is not supported in Fallback DCI (Format 1_0). Observation 6: The Mirror Image FDRA is applicable regardless of whether the subband configuration is transparent or known to the UE. Observation 7: If the subband configuration is semi-statically signalled to the UE, a smaller FDRA bit size can be used in the DL Grant since the FDRA needs only to address the number of RBs in one of the DL subbands rather than the entire BWP. Observation 8: Frequency hopping in UL subband with small bandwidth may not be feasible if the PUSCH/PUCCH consumes most of the UL subband or may not provide any gain. Observation 9: Whether frequency hopping is enabled or disabled in UL subbands depends on the bandwidth size of the UL subbands. Observation 10: Inter subband CLI is non-uniform across a subband, where it is stronger for RBs in a subband that are closer to an adjacent subband compared to RBs that are further away from the adjacent subband. Observation 11: Since in SBFD, inter subband CLI is non-uniform across the victim subband, the CLI measurement reports should take this aspect into account. Observation 12: DL & UL messages may collide in SBFD slots and a mechanism is required to resolve such collisions. Proposal 1: SBFD operation where the SBFD locations are transparent, i.e. Alt 1, is not considered further in this SI. Proposal 2: Further consider Alt 2 of SBFD operation and introduce an “Overwrite” Indicator to the UE to dynamically indicate whether an UL transmission can be transmitted in OFDM symbols that are originally configured/indicated as DL, or a DL transmission can be received in OFDM symbols that are originally configured/indicated as UL. Proposal 3: Since Alt 3 has the disadvantage of not being able to use subband filtering and it does not have the flexibility to dynamically configuring SBFD locations, Alt 3 is not considered further in this SI. Proposal 4: For operations where the UE is aware of SBFD locations (Alt 4), the UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol. Proposal 5: Confirm the following Working Assumption with the following modifications: For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline. - FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies - FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair Proposal 6: Support dynamic configuration of SBFD locations. Proposal 7: Use reserved indices in the SFI, i.e., indices 56-254, to indicate SBFD formats. Proposal 8: Consider using a Mirror Image FDRA, where the DL Grant indicates a 1st set of RBs and a 2nd set of RBs is determined by reflecting the 1st set of RBs across the middle of the BWP. The scheduled PDSCH occupies the 1st set and the 2nd sets of RBs. The Mirror Image FDRA can be enabled or disabled in the DL Grant. Proposal 9: For a UE configured with frequency hopping for a PUSCH/PUCCH, the UE performs frequency hopping in SBFD slots if the bandwidth size of the UL subband is larger than a configured threshold and if there is resource available for the second hop, otherwise the UE disables frequency hopping in the SBFD slot. Proposal 10: Support non-uniform MCS in a PDSCH and PUSCH so that RBs of a PDSCH/PUSCH that are closer to an adjacent subband uses more robust MCS compared to RBs that are further away from an adjacent subband. Proposal 11: Support non-uniform power control in a PUSCH so that RBs closer to an adjacent subband are transmitted with lower power compared to RBs further away from the adjacent subband. Proposal 12: Support finer frequency granularity for CLI measurement and reporting, by dividing the BWP or the victim subband into smaller frequency blocks, where CLI measurement and reporting are performed on each frequency block. Proposal 13: CLI measurements are preformed and reported at the physical layer. Proposal 14: If DL & UL messages collide in an SBFD slot, the UE prioritises the dynamically scheduled transmission over the semi-statically configured resources (e.g. PDCCH Search Space, SPS, CG-PUSCH). |  |  |
| R1-2209126 | Subband non-overlapping full duplex | Lenovo |
| Observation 1: If frequency locations of subbands for SBFD operation are not known to UEs, it may be difficult even for SBFD aware UEs to properly receive downlink signals and channels scheduled on non-contiguous DL subbands in SBFD symbols. Proposal 1: Support SBFD operation Alt 4 (i.e. explicit subband configuration for SBFD operation): - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation. Proposal 2: PUSCH/PUCCH/CG-PUSCH/RACH/SRS configurations for operation in a full duplex UL subband can be indicated by an associated UL BWP ID. Separate CG PUSCH/PUCCH/RACH/SRS configuration(s) can be provided for the full duplex UL subband. Proposal 3: Study PUSCH repetition type A/B and TBoMS across SBFD and non-SBFD symbols/slots. Observation 2: Radio resources may not be efficiently used with a semi-static full duplex UL subband. Proposal 4: Study UL subband bandwidth adaptation for flexible SBFD operation. Proposal 5: For PDSCH reception with downlink type 0 resource allocation or with non-interleaved downlink type 1 resource allocation, a SBFD aware UE performs rate matching around an active UL subband. Proposal 6: For PDSCH reception with interleaved downlink type 1 resource allocation, a SBFD aware UE indexes VRBs and defines resource block bundles based on resource blocks not overlapping with an active UL subband. Proposal 7: Study enhanced CORESET frequency resource allocation for SBFD operation. Proposal 8: Study solutions to enable non-contiguous CSI-RS in frequency. Proposal 9: Study potential enhancements for UL/DL collision handling in time domain. Proposal 10: Support a non-contiguous CLI-RSSI measurement resource configuration in frequency for inter-subband CLI measurement. |  |  |
| R1-2209175 | Subband non-overlapping full duplex | Ericsson |
| Proposal 1 RAN1/4 to discuss whether or not practical solutions exist for improving inter-sector isolation in the SI to draw observations and conclusion on the feasibility and applicability of SBFD for the TR. Proposal 2 RAN1 to study alternative opportunistic TDD operation approaches than the SBFD approach for UL coverage, throughput and latency improvements. In particular, some of these alternatives do not require complicated hardware and do not suffer from self-interference or inter-sector interference of its own network. Proposal 3 RAN1/4 to discuss whether or not practical solutions exist for improving co-located inter-operator isolation in the SI to draw observations and conclusion on the feasibility and applicability of SBFD for the TR. Proposal 4 RAN1/4 to study solutions, performance and applicability for SBFD operations in multi-carrier and/or multi-band base stations. Proposal 5 Support Alt-4 for SBFD operation in RRC_CONNECTED state. Proposal 6 Do not support SBFD operation in legacy 'U' slots. Proposal 7 For SBFD operation for new (Rel-18) UEs, dedicated RRC signaling configures both the time domain pattern in terms of which slots/symbols are used for SBFD operation, and the frequency domain pattern in terms of RB sets with 'D' and 'U' direction (e.g., D-U-D) and guardbands between the RB sets. For example, the time/frequency pattern can be indicated via an enhancement of the existing TDD-UL-DL-ConfigDedicated IE. Proposal 8 Dynamic indication of the size and/or frequency location of the UL subband in SBFD symbols is not supported. Proposal 9 For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, Options 3 and 4 in the RAN1#110 agreement are not further studied. Proposal 10 For new (Rel-18) UEs supporting SBFD operation within a TDD carrier, support only a single active DL/UL BWP pair with aligned center frequencies. In both SBFD symbols and UL-only symbols, both the DL and UL BWPs can span the whole carrier. Proposal 11 UEs in IDLE mode are not aware of whether or not symbols/slots are used for SBFD operation. Proposal 12 For new (Rel-18) UEs, it is beneficial to support enhancements to CSI-RS to allow non-contiguous frequency domain resource allocation, i.e., in both 'D' RB sets. Proposal 13 For new (Rel-18) UEs in CONNECTED mode, study enhancements to frequency domain resource allocation and frequency hopping mechanisms for PUSCH and PUCCH configured with repetition in order to allow repetitions to occur in both SBFD and UL-only slots. |  |  |
| R1-2209240 | Discussion on subband non-overlapping full duplex | Panasonic |
| Observation 1: It is not sufficient to operate SBFD only over legacy semi-static Flexible symbol/slot. Method to utilize legacy semi-static DL symbol/slot should be studied. Proposal 1: If intra-carrier guard band is supported for SBFD, the guard band is outside of DL/UL subband. Proposal 2: For link direction of subband, if only semi-static configuration of subband symbols is supported, Option 2 (The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol) is supported. If dynamic indication of subband symbols is supported, Option 1 (The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol) is supported. The decision of two options can be based on the flexibility on the symbol allcoation Proposal 3: The (candidate) subband frequency and time locations should be sent over SIB in order to allow adjustment operator can check the subband configuration. Proposal 4: For SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies, the legacy semi-static configuration of RB set can be basically reused but the configuration should be sent over SIB. Proposal 5: Consider to introduce a new semi-static slot format for Rel-18 (and beyond) where the legacy semi-static DL symbol/slot can be re-configured as UL or Flexible symbol/slot. Proposal 6: Study potential enhancement on configured UL transmission to associate with either normal UL symbol or SBFD symbol. Proposal 7: For PUSCH with dynamic grant, the same FDRA is used between SBFD symbol and normal symbol. Proposal 8: Study how to configure timing advance offset for SBFD symbol and normal symbol Proposal 9: For CLI measurement and reporting, further discuss the following enhancements: - L1 report, instead of or on top of L3 report, to aid scheduling decision. The existing framework of CSI report can be reused for CLI report. - How to include spatial domain information to facilitate efficient UE pairing to avoid UE-UE interference Proposal 10: Study potential schemes for interference mitigation such as UL transmission power limitation, and exchange SBFD configurations. |  |  |
| R1-2209284 | Discussion on subband non-overlapping full duplex | xiaomi |
| Observation 1: The frequency resources available on the DL symbols for a SBFD UE are confined within the active UL BWP if the UL subband is transparent. Observation 2: It is difficult to mitigate intra-subband CLI if transparent UL subband is adopted. Observation 3: Transparent UL subband complicates multiplexing between UL and DL on the SBFD slot. Observation 4: If a RO exists in DL slots configured by tdd-UL-DL-ConfigurationCommon, it cannot be used for preamble transmission even if the RO locates in a transparent UL subband. Proposal 1: Subband related to duplex operation is transparent for legacy UE and the following legacy behaviour should be kept: - UE doesn’t expect to transmit on DL symbols. - UE doesn’t expect to receive on UL symbols. - UE can transmit or receive on flexible symbols depending on the indication from gNB - UE does not expect conflict between DL reception and UL transmission on the same flexible OFDM symbol. Proposal 2: Clarify whether to allow a SBFD UE to transmit preamble within the UL subband in a SBFD slot. Proposal 3: Non-transparent SBFD operation is sufficient and there is no need to further study transparent SBFD operation. Proposal 4: For Non-transparent SBFD operation, only Alt 4 is supported, i.e. both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. Proposal 5: For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, further study the following two options: - Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol - Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol Proposal 6: Confirm the following working assumption: Working Assumption For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline. - FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies - FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair. Proposal 7: SBFD operation is within an active BWP pair within a TDD carrier. Proposal 8: Half duplex CA based SBFD operation is not supported. Proposal 9: Further study how to configure or determine the guard period between DL region and UL subband. Proposal 10: Study whether and how to define a guard band between DL subband and UL subband. Proposal 11: If UL subband is configured via RRC signalling, the bit length of FDRA field in a DCI scheduling uplink on the UL subband is determined by the active UL BWP. Proposal 12: The following principles should be considered for handling UL/DL collision on the SBFD slots: - SBFD UE doesn’t expect collision between dynamic DL and dynamic UL - SBFD UE doesn’t expect collision between semi-static DL and semi-static UL - Dynamic DL/UL has higher priority when it conflicts with semi-static UL/DL - Broadcast DL has higher priority when it conflicts with UL on the UL subband Proposal 13: For subband non-overlapping full duplex, it cannot be applied to UL symbols. |
| R1-2209336 | Discussion on subband non-overlapping full duplex | CMCC |
| Proposal 1: For inter-gNB inter-subband CLI measurement, the following two methods can be further studied: - Method#1: victim gNB measures leakage power from aggressor gNB within UL subband; - Method#2: victim gNB measures RSRP of aggressor gNB’s RS within DL subband. Proposal 2: For inter-gNB inter-subband CLI handling, inter-gNB coordination in time-domain, frequency-domain, spatial-domain, and power domain should be supported. - Backhaul signalling enhancement is needed to support inter-vendor cooperation. Proposal 3: For inter-UE inter-subband CLI measurement, the following two methods can be further studied: - Method#1: victim UE measures RSSI/SINR within DL subband; - Method#2: victim UE measures RSRP of aggressor UE’s CLI-SRS within UL subband. Proposal 4: L1/L2 based inter-UE inter-subband CLI measurement should be supported, including periodic, semi-persistent, aperiodic and event-triggered. Proposal 5: Both {DUD} and {DU} SBFD subband configurations should be supported. Proposal 6: There is no strong motivation to support configured DL and UL BWP pair with unaligned center frequencies. Proposal 7: SBFD scheme with more than one configured DL and UL BWP pair can be further studied taking into account of the following aspects: - reducing BWP switching delay; - UE supporting multiple active BWPs simultaneously. Proposal 8: Two steps signalling can be considered for indication of time location of subband: - Step 1: Cell-specific signalling to indicate SBFD slots/symbols which can override the fixed DL/UL slots/symbols in cell-specific TDD UL/DL configuration. - Step 2: UE-specific RRC signalling/scheduling DCI/SFI to indicate the DL or UL link direction for SBFD slots/symbols. Proposal 9: A SBFD frequency region can be configured for SBFD aware UE including the starting PRB location and bandwidth: - UL: the SBFD frequency region is used as available UL transmission within UL BWP in SBFD slots/symbols. - DL: the SBFD frequency region is used as DL resource allocation enhancement information or a common rate matching pattern for all DL signals/channels within DL BWP in SBFD slots/symbols. Proposal 10: For SBFD operation Alt 4, support Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol. Proposal 11: Support non-continuous PRB-to-CRB mapping of DL BWP on SBFD slots. Proposal 12: For PDSCH slot aggregation, the following two options can be further studied: - Opt 1: PDSCH slot aggregation is only across SBFD slots or non-SBFD slots; - Opt 2: PDSCH slot aggregation can across SBFD slots and non-SBFD slots with some automatic adjustment on FDRA between SBFD slots and non-SBFD slots. Proposal 13: For SBFD symbols and non-SBFD symbols, the following parameters can be separately configured: - PUSCH/PUCCH frequency hopping offsets; - PUCCH resource sets; - SRS resources. Proposal 14: It is preferred to not allow the SBFD symbols overlap with the symbols that are indicated for reception of SS/PBCH blocks. Proposal 15: ROs for contention-free RACH can be configured in UL subband in SBFD symbols. ROs for contention-based RACH cannot be configured in UL subband in SBFD symbols. Proposal 16: Higher layer configured UL channel/signals and DL channel/signals can be configured in the same SBFD symbols, and some handling rules can be defined for such cases. |  |  |
| R1-2209403 | Discussion on subband non-overlapping full duplex enhancements | ETRI |
| Observation 1. DL/UL resource allocations for subband non-overlapping full duplex operation are applicable for Rel-15/-16 UEs by gNB implementations at the cost of uncertain level of guard band or less flexible DL/UL direction configuration/indication. Observation 2. DL-UL direction change of an SBFD aware UE in SBFD symbols is beneficial for the following use cases - Use case #1: Ease of planning on cell-specific signals / channels - Use case #2: Latency reduction for UL/DL beam indication, channel measurement, CSI feedback, etc. - Use case #3: Ease of co-existence with legacy UEs Observation 3. In case of UE with high capabilities, the channel coherence may be maintained longer than either of the allocated SBFD resource or TDD resource. Proposal 1. Multiple SBFD subbands with different D-U directions can be configured on a given time instance from a UE perspective. Proposal 2. RAN1 to study the following aspects of resource allocation for subband non-overlapping FD: - Support of UL/DL subband allocation based on explicit RB set and guard band configuration. - Detailed signaling methods for UL/DL subband allocation considering interaction with slot formation configuration. - SFI enhancement for dynamic SBFD symbol allocation for a given time instance. Proposal 3. Study semi-static and/or dynamic DL-UL direction change of an SBFD aware UE in SBFD symbols. Proposal 4. Consider the following options for SBFD aware UEs. - Option A (baseline): UE does not receive a DL transmission if the DL transmission overlaps, even partially, with a UL subband (and potentially corresponding guard band(s)), and UE does not transmit a UL transmission if the UL transmission overlaps, even partially, with a DL subband (and potentially guard band(s)). - Option B: If a DL transmission partially overlaps with a UL subband (and potentially corresponding guard band(s)), the UE can receive the non-overlapped DL transmission. - Option C: If a UL transmission partially overlaps with a DL subband (and potentially corresponding guard band(s)), the UE can transmit the non-overlapped UL transmission. Proposal 5. Study the potential specification impact to support power consistency and/or phase continuity of UL transmission(s) across SBFD operation boundary. Proposal 6. RAN1 to study CSI feedback enhancement for subband non-overlapping FD. |  |  |
| R1-2209421 | Discussion on subband non-overlapping full duplex | NEC |
| Observation-1: Full duplex operation can be currently achieved in NR by providing different tdd-UL-DL-ConfigurationDedicated or SFI to different UEs in a cell, however, higher DL/UL performance gains are expected if UEs are indicated SBFD time/frequency resources Observation-2: For semi-static physical channels following conflicts may occur during SBFD time/frequency resources: - UE may try to receive DL receptions (e.g. CSI-RS) within UL sub-bands resulting in incorrect channel estimation or reduced DL performance - UE may try to perform UL transmissions (e.g. SRS) within DL sub-bands resulting in increased interference to nearby UE receiving DL Observation-3: Following interference scenarios are possible for sub-band non-overlapping full duplex operation: - CLI between UEs of same cell i.e. UL transmission of one UE interfering with the DL reception of nearby UE - gNB experiencing interference in UL reception due to its DL transmission in adjacent sub-band - gNB experiencing interference in UL reception due to DL transmission from nearby gNB Proposal 1: - The SBFD aware UE can be dynamically scheduled with DL reception within the UL subband in the SBFD symbol Proposal 2: - The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband Proposal 3: - Support multiple BWP pairs configured to UE (with aligned center frequencies for a DL and UL BWP pair) where each BWP pair is associated with a different TDD/subband configuration Proposal 4: - If a single BWP is used to enable subband non-overlapping full duplex operation, study the impact for configured UL transmission or multi-slot PUSCH transmission scheduled by single DCI cross symbols with different duplex type, e.g., UL symbol only and SBFD symbol with UL sub-band. Proposal 5: - Support frequency guard band between UL and DL sub-bands for interference mitigation during SBFD - Study on how to apply frequency guard band adaptively by UE and gNB based on SBFD occurrences Proposal 6: - Study application of multiple antenna panels for SBFD operation at gNB, where different antenna panels are used for UL reception and DL transmission by gNB Proposal 7: - Study the impact of multiple antenna panels for SBFD on various UL/DL reception performance and channel estimation procedures. Proposal 8: - Study whether existing rate matching/puncturing/pre-emption procedures are sufficient to mitigate any UL/DL conflicts occurring at UE for semi-static physical channels during SBFD time/frequency resources Proposal 9: - Study enhancements for semi-static physical channel resource configuration and/or transmission/reception procedures for Rel-18 UE during SBFD time/frequency resources Proposal 10: - In Rel-18, for sub-band non-overlapping full duplex at the gNB side, study interference management for - inter-UE CLI in a cell - self-interference management for gNB - inter-gNB CLI Proposal 11: - In Rel-18, for sub-band non-overlapping full duplex at the gNB side, traffic characteristics served by gNBs should be taken into the interference management. Proposal 12: - Study exchange of SBFD configuration between gNBs for inter-gNB CLI mitigation Proposal 13: - Study the CSI report enhancement for SBFD operation and different type of CLI interference |  |  |
| R1-2209583 | Views on subband non-overlapping full duplex | Apple |
| Proposal 1: De-prioritize Option 3 and Option 4 for SBFD operation under Alt4. Proposal 2: Potential aggressor UE, UEA, is indicated to measure SRS (transmitted by potential victim UE) before PUSCH transmission. The indication can be through DCI scheduling the PUSCH for aggressor UE. - One (or more) aperiodic SRS resource sets are tagged with a RRC parameter indicating CTS purpose - Once SRS request bit-field activates such SRS resource set, (potential) UEA performs SRS-RSRP and/or CLI-RSSI over the activated SRS resource(s) Proposal 3: Study feasibility of a mechanism to indicate future release UE about cell duplex operation mode. Proposal 4: DL CLI indication, e.g., based on DL-PI, indicates which symbols were impacted by cross-link interference from aggressor UE(s). Proposal 5: Further study the feasibility, and impacts to legacy UE, for DL power adjustment |  |  |
| R1-2209729 | SBFD feasibility and design considerations for NR duplex evolution | Samsung |
| Observation 1: 80 dB in FR1 and 87 dB in FR2-1 antenna isolation using spatial separation and RF barrier can be achieved Observation 2: Stopgap performance of the RF barrier for FR1 100 MHz and FR2-1 100 MHz channel BW is feasible Observation 3: 45 dBc subband leakage ratio between the SBFD DL and UL subband when using non-overlapping frequency resources with digital pre-distortion can be achieved in FR1 Observation 4: 28 dBc subband leakage ratio between the SBFD DL and UL subband when using non-overlapping frequency resources can be achieved in FR2-1 Observation 5: Both in FR1 and FR2-1, SBFD can operate with only a few guard RBs between DL and UL subband when sufficient spatial isolation is guaranteed Observation 6: Digital SIC to remove Tx-to-Rx interference in the Rx path results in a noise rise of 0.9dB for SFBD in FR1 Observation 7: Digital SIC to remove Tx-to-Rx interference in the Rx path results in a noise rise of 0.7 dB with 1T1R and 1 dB with 2T2R panel configurations for SBFD in FR2-1 Observation 8: Additional Rx filtering in IF or BB can be applied for FR1 and FR2-1 receivers to increase robustness of the gNB Rx path with respect to ADC and LNA dynamic range without incurring undue insertion losses Observation 9: gNB power consumption aspects are considered in the SBFD feasibility analysis Observation 10: Backwards-compatibility for legacy UEs when SBFD is configured in the TDD cell can be achieved by using DDDSU in SIB1 tdd-UL-DL-ConfigCommon Observation 11: It cannot be assumed that SBFD using transparent mode when configuring DFFFU in SIB1 tdd-UL-DL-ConfigCommon is supported by all legacy UE implementations Observation 12: SBFD operation can be supported using a single NR carrier or TDD intra-band CA Proposal 1: RAN1 should evaluate the potential impacts from SBFD when a TDD serving cell using SBFD is configured as PCell, SCell or SpCell for a legacy UE Proposal 2: RAN1 to study potential benefits and specification impacts of explicit guard band configuration provided to the UE for the SBFD UL subband and DL subband(s) Proposal 3: Confirm the RAN1#110 working assumption that the SBFD scheme with a single configured DL and UL BWP pair with aligned center frequencies is studied as baseline Proposal 4: For SBFD operation within a TDD carrier, SBFD within a single DL and UL BWP pair with unaligned center frequencies is not supported Proposal 5: For SBFD operation within a TDD carrier, SBFD with more than one configured DL and UL BWP pair is supported only for the case of a DL and UL BWP pair with aligned center frequencies Proposal 6: For SBFD operation in RRC_CONNECTED mode, SBFD operation Alt. 4 is supported and SBFD operation Alt.2/Alt.3 are not supported Proposal 7: For SBFD operation Alt.4 and SBFD configured in D/F symbols/slots, support both Options 1 and 2 Proposal 8: For SBFD operation Alt.4 and if SBFD configured in U/F symbols/slots is supported, support only Option 3 Proposal 9: Multi-slot PUSCH transmissions and PUCCH repetitions can be configured to only use the SBFD slots/symbols Proposal 10: RAN1 to study the impacts of SBFD operation on PDSCH resource allocation types 0 and 1 and resource mapping in SBFD slots/symbols Proposal 11: RAN1 to study the impacts of SBFD operation on PUSCH resource allocation type 1 and frequency hopping Proposal 12: RAN1 to study the impacts of SBFD operation on CSI-RS resource set configuration and CSI reporting Proposal 13: RAN1 to study potential benefits and specification impacts when using SBFD slots/symbols for random access |  |  |
| R1-2209770 | Discussion on subband non-overlapping full duplex for NR | MediaTek Inc. |
| Observation 1: DL-UL switching points occur in separate symbols per subband. This motivates per-subband, per-symbol indication of RB-symbol regions reserved for DL-UL switching. Observation 2: In TDD frame format configuration by current specification, on symbols indicated as flexible by group common SFI signalling, transmission or reception is only carried out when scheduled by DCI. Observation 3: CORESET allocation readily supports non-contiguous frequency-domain allocation and monitoring up to three different CORESETs in a BWP. Observation 4: Configuring PDCCH monitoring over different CORESETs for SBFD and DL-only slots/symbols may require configuring separate search spaces per each slot in the periodic SBFD partition pattern, each with the same slot periodicity but different slot offsets, soon using up the maximum number of search spaces that can be configured. Observation 5: When using FDRA Type-0 for PDSCH scheduling, certain RBG(s) may be unavailable due to partial overlap with UL-subband (and/or any guard band) leading to resource fragmentation. Observation 6: VRB-interleaving is not supported by FDRA Type-0 and is mostly incompatible with SBFD by FDRA Type-1 allocation, since PRB segments likely overlap with RBs that are not available to downlink. This prohibits VRB-interleaving altogether. Observation 7: For CG-PUSCH transmission, a single FDRA is configured for all UL slots. This may impact flexibility in resource allocation in SBFD since it has two types of slots with different bandwidths. Observation 8: For DG-PUSCH with repetition, a single FDRA is defined for all slots within a repetition. This may impact flexibility in resource allocation in SBFD since it has two types of slots with different bandwidths. Observation 9: When frequency hopping is enabled for SBFD, the resource allocation with frequency hopping offset applied may not be confined within the UL subband of a SBFD partitioned slot. Observation 10: For periodic and semi-persistent PUCCH transmission, a single frequency domain starting position is configured for all UL slots. This may impact flexibility in resource allocation in SBFD since it has two types of slots with different bandwidths. Observation 11: For PUCCH repetition, a single frequency domain resource allocation is defined for all slots within a repetition. This may impact flexibility in resource allocation in SBFD since it has two types of slots with different bandwidths. Observation 12: When frequency hopping is enabled for PUCCH transmission in SBFD, the resource allocation for the first and second hops may not be confined within the UL subband of a SBFD partitioned slot. Observation 13: For periodic and semi-persistent SRS transmission, a single frequency domain resource allocation is configured for all UL slots. This may impact flexibility in resource allocation in SBFD since it has two types of slots with different bandwidths. Observation 14: For SRS transmission in SBFD, the resource allocation with and without frequency hopping may not be confined within the UL subband of a SBFD partitioned slot. Observation 15: For CG PUSCH transmission in SBFD systems PUSCH fragmentation will occur on UL-only slots since the frequency domain resource allocation on SBFD slots is at the centre of the UL BWP. Observation 16: The impact of PUSCH fragmentation on UL performance can be reduced by placing periodic PUSCH resources originating from a SBFD slot at the edge of an UL-only slot. Observation 17: Non-contiguous resource allocation using FDRA Type 0 is an optional feature for legacy UEs and cannot be relied upon in the case of PUSCH fragmentation in SBFD systems. Observation 17: Non-contiguous resource allocation using FDRA Type 0 is an optional feature for legacy UEs and cannot be relied upon in the case of PUSCH fragmentation in SBFD systems. Error: Reference source not found Error: Reference source not found - Only the aggregate SRS-RSRP value is reported dropping the values measured per Rx antenna. - SRS-resources transmitted over switched antennae will be reported on separately by measuring UE, causing inefficiency. Error: Reference source not found Proposal 16: On symbols indicated as flexible by SFI group common signalling, the SBFD partition status is assumed to be unknown by UE, irrespective of other configurations. Proposal 17: Support semi-static configuration of a subband within a symbol that indicates ‘flexible’ link direction for transmission or reception scheduled by DCI. Transmission or reception scheduled otherwise is deprioritized on such flexible subband symbols. Proposal 18: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol, with the following exceptions: - Inter-subband UE-UE CLI measurement are not confined within the DL subband. - UE assumes that the symbol is not partitionned if symbol is indicated to be ‘flexible’ by group common SFI signalling of UE TDD link direction. - DL reception scheduled by DCI is allowed to overlap with subband in a symbol indicated as flexible by semi-static SBFD layout configuration. Proposal 19: SBFD layout in frequency and time is configured per carrier. Proposal 20: Study enhancements to Search Space configuration which allow adaptation to SBFD partitioning. Proposal 21: Support allocation of fractional RBGs in Type-0 FDRA at DL subband boundaries. Proposal 22: Support interleaved mapping of odd and even VRBs over physical RBGs allocated using Type-0 FDRA. Proposal 8: Support two FDRA for CG-PUSCH transmission based on slot type. Proposal 9: Support skipping of periodic transmissions (e.g., CG-PUSCH) in SBFD slots. Proposal 10: Support two FDRA for DG-PUSCH repetition based on slot type. Proposal 11: Allow the support of skipping of PUSCH repetition that occurs in SBFD slots. Proposal 12: Modify the frequency hopping procedure for intra-slot and inter-slot frequency hopping to enable frequency hopping that is confined within the UL-SB in the SBFD slots. Proposal 13: Support PUSCH frequency hopping in the UL-only slots for inter-slot frequency hopping. Proposal 14: Support two frequency domain resource allocations for periodic and semi-persistent PUCCH transmission in SBFD based on slot type. Proposal 15: Support skipping of periodic and semi-persistent PUCCH transmissions in SBFD partitioned slots. Proposal 16: Support two frequency domain resource allocations for PUCCH repetition based on slot type. Proposal 17: Allow the support of skipping of PUCCH repetition that occurs in SBFD slots. Proposal 18: Support two frequency domain resource allocations for intra-slot and inter-slot frequency hopping for PUCCH transmission in SBFD based on slot type Proposal 19: Support PUCCH frequency hopping in the UL-only slots for inter-slot frequency hopping. Proposal 20: Support two frequency domain resource allocations for periodic and semi-persistent SRS transmission based on slot type. Proposal 21: Modify the frequency domain resource allocation procedure for SRS transmission to ensure that the frequency domain position is confined within the UL-SB in the SBFD slots Proposal 22: Support skipping of periodic and semi-persistent SRS transmissions in SBFD slots. Proposal 23: To avoid the problem of PUSCH fragmentation in SBFD systems, support the option of configuring two FDRA for CG PUSCH transmissions based on slot type. Proposal 24: Study whether SRS-RSRP-based ranking of UE aggressor candidates is sufficient for the optimization of UL-DL inter-subband CLI. Proposal 25: Study the feasibility and cost of muting co-channel interferer for the assessment of inter-subband UE CLI using CLI-RSSI measurements. Proposal 26: Study the need for Layer-1 UE-UE CLI measurement, and the details of its features. Proposal 27: RAN1 to study the feasibility of using “reverse” UE-UE CLI measurement to protect legacy UEs not supporting such measurements. Proposal 28: UE reports SRS-RSRP (or CLI-RSSI) per Rx antenna separately. Proposal 29: SRS-RSRP measurement resource can be configured to report an aggregate SRS-RSRP value measured over a set of SRS resources. Proposal 30: SRS-RSRP measurement can be configured with QCL-TypeD (spatial relationship information). |  |  |
| R1-2209809 | Study on Subband non-overlapping full duplex | LG Electronics |
| Proposal 1: SB-FD operation can be allowed in the time resource configured as DL and/or Flexible. Proposal 2: Indication of at least time resource for SB-FD operation can be provided as semi-static configuration of subband time and frequency location via system information. In addition, following options can be studied for the indication of SB-FD time location. - Option 1: Indicate each slot within a duration where SB-FD operation is allowed - Option 2: Indicate start time resource and number of contiguous time resource within a duration where SB-FD operation is allowed. Proposal 3: For supporting SB-FD operation Alt4, following aspects can be considered for study: - Additional TDD UL-DL configuration (i.e., introduce additional configuration for TDD-UL-DL-ConfigDedicated) - UE behavior according to frequency resource of DL/UL for half duplex time resource and SBFD time resource (e.g., UL frequency hopping, PUCCH resource allocation in frequency domain, SRS resource configuration, UL/DL FDRA, CSI-RS resource, DL Measurement, CSI measurement and reporting, etc.) Proposal 4: Study SB-FD operation for UE in IDLE/Inactive state. Proposal 5: Study whether TDD configuration mechanism should be updated for supporting SB-FD operation. If it is agreed that enhancement of TDD configuration for SB-FD operation is studied, following can be studied. - Which type of TDD configuration needs to be enhanced for supporting SB-FD operation Proposal 6: Study whether multiple type of TDD UL/DL pattern can be allowed for BWPs when SB-FD operation is adopted. Proposal 7: Study whether/how DL/UL collision rule is enhanced for efficient DL/UL operation at the SB-FD symbols Proposal 8: Support fallback operation of SB-FD (i.e., returning to the half duplex operation) Proposal 9: Study how to operate well both legacy HD TDD and SB-FD within time duration. Proposal 10: Study time boundary alignment between UL and DL within a slot for SB-FD. Following options can be studied. - Option 1: Symbol boundary alignment between DL signal/channel and UL signal/channel - Option 2: Slot boundary alignment without symbol boundary alignment between DL signal/channel and UL signal/channel Proposal 11: at least SBFD within a single configured DL and UL BWP pair with unaligned center frequency should be supported for {D U} frequency portioning case. Proposal 12: Study whether/how a BWP is consisted of discontinuous subbands. Proposal 13: Study whether/how different length of available BW of a BWP depending on time resource for HD or SB-FD is supported. Cross Link Interference Handling Proposal 14: For SB-FD specific UE-to-UE Cross Link Interference handling, followings can be studied. - Interference management schemes (e.g., long-term CLI measurement, scheduling, link adaption, power control, etc) for handling inter-subband emission and/or intra-subband CLI from aggressor gNB - L1/L2 based CLI measurement and reporting for intra-cell UE2UE CLI management Self-Interference Cancellation/Mitigation Proposal 15: For self-Interference cancellation/mitigation method for SB-FD, followings can be studied. - Simultaneous beam management for DL transmission and UL reception - Simultaneous power control for DL transmission and UL reception - gNB implementation for Self-Interference cancellation/mitigation (e.g., antenna separation, RF level SI cancellation, baseband level SI cancellation, subband filtering, etc.) Proposal 16: Study whether MIMO related configuration (e.g., antenna configuration, beam management, power control, CSI measurement/report, reference signal, etc.) can be differently applied according to duplex schemes (i.e., HD TDD and SB-FD) operated in gNB side. |  |  |
| R1-2209902 | Discussion on subband non-overlapping full duplex | NTT DOCOMO, INC. |
| Observation 1: SRS-RSRP measurement and reporting is not applicable for SBFD. Observation 2: Enhancements like beam specific CLI measurement/reporting and Layer 1 based CLI measurement/reporting are also applicable for SBFD. Proposal 1: - For SBFD operation with UL subband in semi-static DL/flexible symbol, study one UL subband in the midlle and two DL subbands in the edge. - If SBFD operation with DL subband in semi-static UL symbol is supported, the DL/UL subband frequency pattern needs further study. Proposal 2: - Study SBFD operation with UL subband in semi-static DL and flexible symbol in Rel-18. - SBFD operation with DL subband in semi-static UL symbol can be down-prioritized in Rel-18. Proposal 3: - Study Alt 4 (i.e. both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs) for SBFD operation. - Not support Alt 2 and Alt 3 for SBFD operation. Proposal 4: - Confirm the working assumption that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequency as baseline. - Study of SBFD scheme with more than one configured DL and UL BWP pair with aligned center frequencies for a DL and UL BWP pair should have lower priority. - Down-prioritize study of DL and UL BWP pair with unaligned center frequencies. Proposal 5: In order to indicate DL/UL time and frequency resource allocation (including number of PRBs for UL/DL subband) for subband non-overlapping full duplex operation, enhancements based on existing slot format indication, or new slot format indication can be considered. Proposal 6: - Study dynamic indication of DL/UL subband time domain allocation. - NOT support dynamic indication of DL/UL subband frequency domain allocation. Proposal 7: - Support only DL reception in DL subband (i.e. not support UL transmission in DL subband), only UL transmission in UL subband (i.e. not support DL reception in UL subband). - Not support UL transmission outside UL subband in SBFD symbol. Proposal 8: Not support SBFD operation with UL subband on SSB symbol or CORESET #0 symbol. Proposal 9: - Support PRACH and Msg 3 PUSCH transmission in UL subband in SBFD symbol. - Support UE monitoring RAR in DL subband in SBFD symbol, if UE doesn’t transmit PRACH in any valid RO on the symbol. Proposal 10: For single DCI scheduling multiple PDSCHs/PUSCHs, study possible enhancements when any of the multiple scheduled PDSCHs/PUSCHs overlapping with UL/DL subband. Proposal 11: - For collision handling of higher layer configured channels/signals with SBFD DL/UL subband, study possible enhancements for channels/signals with higher priority first, e.g. PDCCH as highest priority for study. - Study collision for PDSCH/PUSCH/PUCCH repetitions with UL or DL subband in SBFD symbol. Proposal 12: For Rel-18 subband non-overlapping full duplex, at least for higher layer configured channels/signals, three alternatives can be considered: - Alt 1: UE does not expect collision of higher layer configured DL reception or UL transmission with UL subband or DL subband in SBFD symbol. - Alt 2: Separate higher layer DL reception or UL transmission configuration for SBFD symbols and non-SBFD symbols, e.g. separate PDCCH monitoring configuration or PUCCH resource configuration. - Alt 3: No separate configuration, but special handling is needed if higher layer configured DL/UL channel/signal colliding with UL/DL subband in SBFD symbol, e.g. the DL/UL channel/signal is canceled or rate matched around DL/UL subband, or different interpretations are applied for frequency resource allocation for SBFD symbol and non-SBFD symbol. Proposal 13: Study SBFD impact on PUSCH repetition type B segmentation. Proposal 14: Study SBFD impact on SPS HARQ-ACK deferring. Proposal 15: Study possible enhancements on CSI-RS configuration and CSI reporting, including: - Study non-contiguous RB allocation for CSI-RS or narrow band RB allocation for CSI-RS (e.g. smaller value than 24 for number of RBs). - Study possible enhancements for CSI reporting overhead reduction for SBFD, e.g. to allow different frequency band configurations (i.e. different starting PRB and num-ber of RBs) for CSI-RS resources in the same CSI-RS resource set, or to associate different CSI-RS resources with different frequency band configurations and treat them as one “effec-tive CSI-RS”. - Study whether to separately report CSI for non-SBFD symbol/slot and for SBFD symbol/slot. Proposal 16: Study SBFD impact on UL cancellation DCI format 2_4. Proposal 17: Study CLI-RSSI measurement and reporting with multiple subbands for CLI-RSSI measurement resource. Proposal 18: Support inter-gNB coordination enhancement with DL/UL subband allocation exchanged between gNBs. |  |  |
| R1-2209930 | Discussion on subband non-overlapping full duplex | Sharp |
| Observation 1: For UEs aware of frequency domain subband location, it is possible to avoid resource allocation outside of DL subbands. Observation 2: Time domain subband location indication is beneficial for switching the gNB/UE behavior of the wideband non-continuous FDRA and the legacy FDRA. Observation 3: Mechanism to avoid allocating resource outside DL subbands is necessary for distributed resource allocation across DL subbands. In this contribution, we have the following proposals: Proposal 1: For SBFD aware UE with indicated time and frequency subband location, mechanism to avoid allocating resource outside of DL subbands should be supported. Proposal 2: Non-continuous FDRA across UL subbands should not be studied in Rel-18 SI. Proposal 3: No frequency hopping enhancement is necessary for UE-specific PUCCH/PUSCH. Proposal 4: Time and frequency domain subband location indication should be available for configured scheduling. Proposal 5: Impact on legacy UEs or UEs without SBFD capability should be carefully studied for SS/PBCH block placement in SBFD region. Proposal 6: Support for msg3 repetition in SBFD region should be further studied. Proposal 7: UCI multiplexing procedure can be further studied at least with the following options: - The gNB ensures that the new PUCCH resource is confined within the UL portion - Configure separate PUCCH resources for SBFD region and legacy UL region Proposal 8: Single subband PDSCH scheduling should be supported for SBFD. How to mitigate intra-cell UE-UE co-channel inter-subband CLI could be discussed at least with the following aspects: - Whether more stringent in-band emission requirements are required for Tx UE - Potential enhancements to reduce the CLI originated from the Rx UE - CLI measurement and reporting mechanism Proposal 9: Study further how to support enhancements on Rx UE for multi-subband PDSCH scheduling. At least the following possibilities can be considered further: 1) The UE only monitors subband#0 and subband#2 (like non-continuous CA) 2) The UE monitors wideband carrier including Subband#0, 1, and 2 (Legacy UE behaviour) Proposal 10: Study further how to support enhancements on Rx UE for cross-subband PDSCH scheduling. At least the following possibilities can be considered further: 1) The UE only monitors subband#0 and subband#2 (like non-continuous CA) 2) The UE monitors wideband carrier including Subband#0, 1, and 2 Proposal 11: New TDD configuration should be supported for SBFD. Proposal 12: Subband configuration should be supported. |  |  |
| R1-2209983 | Feasibility and techniques for Subband non-overlapping full duplex | Qualcomm Incorporated |
| Observation 1: SBFD operation across multiple component carriers can be achieved using two different design alternatives. - Alt1: intra-band CA using different TDD-DL-UL pattern across the CCs - Alt2: Reusing the same design concept of SBFD within component carrier across the CCs. Observation 2: SBFD operation across multiple CCs requires UE supports of CA as prerequisite while CA framework has some inherent UE complexity. Observation 3: Compared to single-CC SBFD, CA-based SBFD has some limitation where DL and UL BW is restricted to the component carrier bandwidth while the inter-channel guardband can’t be utilized. Observation 4: CA-based SBFD operation is interesting for higher band (e.g. FR2-1). Observation 5: SBFD operation at legacy UL slot is not precluded in Rel-18 study item. Observation 6: SBFD operation at legacy UL slot is beneficial in multiple deployment scenarios, e.g., greenfield deployment and UL heavy deployment (InH/InF) to reduce DL blockage and improve DL coverage. Observation 7 Legacy DL slot is important to protect DL reception of UEs that suffer from strong CLI especially when receiving common signalling and UEs that don’t support Rel-16 CLI framework. Observation 8 Transparent SBFD operation (Alt 1) using current 3GPP specification is possible. However, there are restrictions and limitations. - gNB to rely on dynamic scheduling within the SBFD slots and signals these slots as flexible on the cell-specific configuration. - Limited usage of configured UL signals/channels within the SBFD slots (e.g. SRS, CG, hopping, etc) - CSI-RS report overheads for the two DL subbands and limitation on PDSCH scheduling on both subbands Observation 9 Non-Transparent SBFD operation (Alt 4) resolves the limitation/restriction of transparent SBFD (Alt 1) and allow for the following benefits: - Improved UE selectivity, filtering and possible power savings due to reduced sampling rate. - SBFD-aware can transmit UL in RRC configured DL and vice versa - Enable some enhancement on resource allocation (e.g. CSI-RS) and subband scheduling. - Enable slots dependent configurations (e.g power control and timing) Observation 10: It is beneficial for the HD UE to be aware of gNB full duplex operation in specific slot format and the frequency resource’s locations of the DL and UL subbands. Observation 11: There is no extra benefits or gains for SBFD operations using alternative 2-3 over alternative 4. Observation 12 SBFD gNB may not need a guardband between UL and DL subband. However, from UE perspective, a guardband may be needed to reduce inter-UE CLI given there is no (or small) UE selectivity. Observation 13 Based on LLS, increasing the guardband between the scheduled DL and UL helps reducing the inter-UE CLI and recovering some TPUT loss. When inter-UE CLI is too large due to close UEs proximity, increasing the guardband is not helpful. Observation 14: Semi-static configuration of the UL/DL subbands is essential for the SBFD operation. Observation 15: Dynamic indication/update of the UL/DL subband can be useful in some scenarios (e.g. gNB fallback to HD mode due to strong interference and dynamic DL or US scheduling after slots updates). Observation 16: For SBFD scheme within a single configured UL/DL BWP pair: - At least for semi-static configuration of the subband locations, the frequency resources of the UL or DL subbands requires new RRC signalling for both the time and frequency location of the subbands which can be configured as cell-common or UE-dedicated per-BWP. - The new RRC signalling could be based on some enhancement/extension of current 3GPP mechanism for TDD-DL-UL pattern indication and SFI framework. - Non-aligned UL/DL BWP could be beneficial for some scenarios, e.g., narrowband UL/DL BPW for initial access or default BWP. Observation 17: For SBFD operation using more than one configured DL and UL BWP pair where each subband configured as BWP, it requires a lot of specification impacts and complicate UE behavior. - Requires two active BWPs at a time, each configured with its own DL/UL TDD pattern. - Requires cross-BWP scheduling and cross-BWP HARQ feedback. - Complicates BWP switching mechanism - Requires non-aligned BWP center frequency - Requires some restriction rules may be needed to have common parameters for both DL BWPs - RRC signalling overhead Observation 18: For SBFD operation using more than one configured DL and UL BWP pair where one BWP pair is configured for TDD operation and the pair is configured for SBFD operation, it requires less specification impact mainly to enhance the BWP framework (e.g. non-contiguous RB for BWP, non-aligned UL/DL center freq.) - One UL/DL BWP pair is active at a time - Semi-static configuration of BWP switching pattern to reduce BWP switching delay. - Enable UE to enhance BWP-based selectivity/filtering to reduce inter-UE CLI. Observation 19: Option #1 is sufficient baseline for SBFD operation of SBFD-aware UE and conforms with basic assumption/understandings of UL/DL subband operation. Observation 20: Options 2-4 increases the CLI, contradicts with basic understanding of UL/DL subband traffic direction and increases complexity and timeline for adaptive RF tunning and filtering. Observation 21: Dynamic SBFD symbol update (e.g., fallback to TDD mode or adapting subbands) provides the same functionality and flexibility of options 2, 3 and 4. Observation 22: UE indication of the UL/DL subband configurations at initial access is beneficial to enable: - Improve UL coverage for RACH messages by enabling repetition and/or frequency hopping - Enable additional RACH occasions which reduce the collision of the CBRA - Reduce the latency for random/initial access procedure. Observation 23: Transparent SBFD operation based on current 3GPP specification is possible. - SBFD symbols configured as flexible to enable dynamic UL/DL scheduling. - DL scheduling across both DL SBs using RA Type 0 with some limitation on granularity. - CORESET #0, SIB1 and Type-0 CSS can be configured in one the DL subband. Other CORESETs are very flexibly configured using bitmaps. - CSI-RS per each DL subband or wideband CSI-RS configuration in DL slot - Wideband SRS in UL symbols to enable DL CSI acquisition. Observation 24: For CSI-RS in SBFD symbols, gNB can configure: - Option1: Two contiguous CSI-RS resources per each subband and a single CSI report linked to the two resources. - Option 2: Non-contiguous CSI-RS across the two DL subbands. Observation 25: UE complexity increases to process the CSI-RS across the two DL subbands which increase CSI processing latency. Observation 26: The FDRA for RA Type 0 is flexible to enable DL scheduling across the two subbands. There could be some restriction on scheduling flexibility if subbands are not aligned with the RBG grid. Observation 27: A separate PUCCH resource set for SBFD operation is useful to enable PUCCH configuration at edge of the UL subband and to account for the different UL link quality and the different antenna/panels configuration at SBFD symbols than normal UL slot. Observation 28: Available slot counting considered time availability of all symbols based on tdd-UL-DL patterns. Observation 29: The available frequency resources for UL transmission are not the same across the legacy UL symbols and SBFD symbols. Observation 30: There is difference in link quality between SBFD slots and TDD slots due to residual self-interference , increased cross-link interference in SBFD slots and the different number of antennas between slots. Observation 31: It is challenging or restricting to configure semi-static signals and channels within SBFD and TDD symbols. Observation 32: R15/16 introduced resections on multiplexing DL/UL signals and channels at some slots. - UE does not expect to have both dedicated configured reception and transmission on Flexible symbol. - UE doesn’t transmit UL signal/channel at SSB symbol(s) and doesn’t receive DL signal/channel during valid RO (including gap) - UE doesn’t receive on RRC UL symbols and doesn’t transmit on RRC DL symbols. Observation 1: Tx-CLI will affect the dynamic range for the reception of the DL signal while the CLI -leakage (NL) will reduce the DL SINR. Both factors will have direct impact on DL reception quality. Observation 2: It is beneficial for the network to configure the UE with CLI resources at the UL and DL subband to measure the CLI and CLI leakage accordingly. Observation 35: The CLI measurements metrics depends on the CLI resource configuration, whether configured at UL or DL subband and which RS configured for the measurement (e.g., SRS, CSI-RS). Observation 36: Rel-16 CLI framework does not support subband CLI reporting, i.e., reporting CLI metric for one or more subbands in the measurement bandwidth. Observation 37: In SBFD, CLI leakage to adjacent subbands is not uniform over the measurement bandwidth and may require subband CLI reporting. Observation 38: The exchange of the UL/DL subband locations is beneficial for inter-gNB CLI avoidance in SBFD deployment, e.g. for deployment case where cells have different UL/DL subband configurations. Observation 39: Two separate panels with added EM spatial duplexer enables large spatial isolation which facilitate gNB full duplex without the need of complex RF circuitry of analogy interference cancelation or subband filters. Observation 40: More than 80 dB of spatial isolation could be achieved using two separate panels with spatial duplexer. Observation 41: For FR2, more than 80-90 dB of spatial isolation could be achieved using two separate panels at 28 GHz frequency. Observation 42: The frequency isolation could be approximated as flat, non-frequency selective profile and its value per-RB is ACLR+10×log10RBDL Observation 44: Evaluation results show more than 45 dB of frequency isolation for FR1 is achievable with 5 RBs guard band and max DL Tx Power which is aligned with the assumption of 45 dB ACLR. Observation 44: Subband filtering may improve gNB Rx selecting for self-interference, however, it is very challenging for massive MIMO deployment, add extra cost and complexity for supporting SBFD in multiple channels and adds insertion loss. Observation 45: With enough spatial isolation between the panels, there is no need for RF subband filtering. Observation 46: A baseband analog LPF may be used to reject the DL blocker and improve the ADC dynamic range. Observation 47: For FR2, it is feasible for implementation to achieve ACLR requirement without RF filtering. Observation 48: In massive deployment, the large number of digital and analog degrees of freedom can be utilized to provide spatial Tx/Rx beamform nulling for self-interference and clutter mitigation Observation 49: For FR2, the measured 28/39GHz path loss between Tx and Rx antennas including clutter reflections is typically approximately 80 dB or better for empty conference room environment. - Higher path loss is generally observed for larger angular separation between Tx and Rx beams. Observation 50: The residual self-interference including both direct leakage and clutter echo can be cancelled using non-linear digital cancellation algorithm. Observation 51: Digital NLIC can provide additional isolation and improvement to alleviate self-interference. Observation 52: Self-interference could be mitigated by means of spatial isolation, frequency isolation and digital IC which makes SBFD feasible with minimal impact on UL degradation Observation 53: For co-site deployment, gNB should have mitigation capability for the CLI of the co-sited sectors by means of improved spatial isolators and additional digital interference cancellation. - In addition, there could be some specification/requirement on the maximum radiation pattern towards the other co-sited sectors. Observation 54: A prototype of full duplex base station was demonstrated and validated feasibility of Sub-band full duplex gNB in wide-area deployments. Proposal 1: SBFD operation across multiple components is studied at later stage in Rel-18 after establishing the baseline study of SBFD operation within component carrier. Proposal 2: Not all DL slots are used for SBFD operation. Proposal 4: Support maximum of three subbands for SBFD operation within a TDD carrier. - SBFD pattern (DUD) and (DU) for legacy DL - SBFD pattern (UDU) and (UD) for legacy UL Proposal 4: Support the discussion of SBFD operation using only Alt 4 which achieves all gains of gNB SBFD operations and simplify UE behaviour (only one scheme can be specified). - Non-transparent SBFD operation using Alt-2 and Alt-3 are not considered in Rel-18. Proposal 5: RAN1 to further discuss UE-specific guardband configuration. Proposal 6: Support Semi-static configuration of the time and frequency location of UL/DL subbands as baseline. Proposal 7: Support dynamic indication/update of the at least time locations of the SBFD subbands. - FFS: dynamic indication/update of the frequency locations of the DL and/or UL SBFD subbands. Proposal 8: Confirm the working assumption and support SBFD scheme using single BWP as baseline for SBFD operation with potential enhancement of UL/DL BWP non-aligned center frequency. - Additionally, further discuss enhancement for SBFD operations using two or more BWP pairs with single active UL/DL BWP pair and semi-static switching of BWP pattern. Proposal 9: Support only Option 1 as the baseline for SBFD-aware UE scheduling. UE can be configured to measure CLI in the UL subband for AGC blocking and/or LNA compression Proposal 10: RAN1 to study potential benefits of initial access for SBFD-aware UE Proposal 11: Support broadcast of the UL/DL subband locations for the initial UL/DL BWP for SBFD-aware UE. Proposal 12: Support configurations of initial access messages on UL subbands of SBFD symbols for both RRC-idle and RRC connected UEs. Proposal 13: Study mechanism to facilitate SBFD aware UE to select SBFD capable cells. Proposal 14: gNB should handle legacy UE by utilizing Rel-16 CLI framework and proper scheduling. Proposal 15: For the coexistence study of legacy UE, No change in UE RF requirements. Proposal 16: UL-subband and guardband are considered as non-available resources for DL reception. PDSCH symbols are rate-matched around these resources and DMRS Proposal 17: RAN1 to further study non-contiguous CSI-RS configuration and impact to UE processing latency Proposal 18: Any further optimization for the FDRA field in scheduling DCI should be well motivated, e.g., partial RBG at DL subband(s) boundaries. Proposal 19: RAN1 to discuss separate PUCCH configuration for SBFD and UL slot. Proposal 20: RAN1 to discuss PUCCH transmission across with inter-slot frequency hopping across multiple SBFD and UL slots. Proposal 21: RAN1 to further study the impact/potential enhancement of frequency hopping with SBFD operation. Proposal 22: RAN1 to further study the impact/potential of enhancement of available slot counting in SBFD. Proposal 23: RAN1 to further study SBFD specific signals and channel configuration (e.g. CG and SPS). Proposal 24: RAN1 to further study separate operation parameters can be pre-configured for TDD and SBFD slots. Proposal 25: The restriction rules on the DL/UL channel/RS multiplexing can be relaxed in SBFD symbols for a SBFD-aware UE to improve resource utilization, reduce DL/UL switching delay and traffic latency. Proposal 26: Initial access RO can be configured in UL subband in SBFD symbol. - FFS: whether RO can be multiplexed with DL signal can be further discussed. - FFS: whether RO in SBFD symbols can be used at least for only SBFD-aware Proposal 27: The restriction rules on the DL/UL channel/RS multiplexing can be relaxed for both connected UEs and idle UEs. Proposal 28: SSB can be configured in DL subband in SBFD symbol. - FFS: whether SSB can be multiplexed with UL signal can be further discussed. Proposal 1: RAN1 to further study the SBFD-aware UE collision scenarios in SBFD symbols for the following cases, e.g. - Semi SFI D/U vs RRC U/D - Semi SFI D/U vs Dynamic U/D - RRC D/U vs RRC U/D - SBFD symbols with valid ROs - SBFD symbols with SSBs Proposal 30: RAN1 to further discuss CLI measurement resources configuration in the DL and/or UL subbands Proposal 31: For UL subband, RAN1 to further discuss CLI measurements reporting based on CLI-SRS-RSRP, and CLI-RSSI for UL-subband configured CLI resources and CLI-SINR and CLI-RSSI for CLI resources configured at the DL subband. Proposal 32: Support subband-based CLI reporting for accurate measurement of CLI leakage in SBFD. RAN1 to further discuss subband configurations within the CLI resource. Proposal 33: To reduce subband CLI reporting overhead, UE report CLI measurements in specific subband(s) where CLI exceeds a configured CLI threshold. - In addition, differential CLI reporting could be considered to reduce the overhead. Proposal 34: gNB to exchange information of the CLI resource configurations and/or CLI measurements. Proposal 35: Support exchange of the UL/DL subband locations between the cells. |  |  |
| R1-2210030 | Discussion on sub-band non-overlapping full duplex | ITRI |
| Proposal 1: Support SBFD operation Alt 4. Proposal 2: For SBFD operation 4, support option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol Proposal 3: How to handle conflict signalling within a SBFD symbol could be studied in Rel-18. |  |  |
| R1-2210042 | On subband non-overlapping full duplex for NR | Nokia, Nokia Shanghai Bell |
| Observation 1: SBFD cannot be operated without changes to the RF architecture and as such SBFD needs new physical implementations and cannot be software upgraded to existing and deployed base stations. Observation 2: For Opt 3 (same antenna area), the gains of SBFD are limited to the 5%-ile UL UE throughput and up to medium load only (up to 4x improvement @5%-ile). At medium and high load, SBFD reduces the average UE UL throughput by up to 60% even for high (close-to-ideal) RSI of 165 dB. Observation 3: For Opt 2 (same antenna gain), SBFD provide UL throughput gains across the 5%/50%/95%-iles for low and medium load conditions. For high load, the gains are limited to the 5%-ile only and average UE UL throughput reduction of 20%-60% (depending on RSI level) is observed. Observation 4: High UL throughput degradation of SBFD with both Opt 2 and Opt 3 SBFD antenna configuration is observed for high offered load of traffic even if high (close-to-ideal) RSI of 165 dB is assumed. In such load conditions, the main source of performance degradation comes from the inter-site gNB-gNB inter-subband interference rather than the gNB self-interference. Observation 5: Assuming similar ratio of DL resources for SBFD and TDD (XXXXX vs DDDSU), SBFD provides a degradation of DL throughput across all the percentiles of the Average DL UPT performance mainly as a consequence of the guardband (overhead) between DL and UL subbands. Degradation of up to 16% in average UPT is observed when same BS antenna area is assumed for both SBFD and TDD (Opt 3). Observation 6: SBFD significantly reduces the average UL packet delay experienced in the network, e.g. from 16.5 ms to <3.6 ms (~4x) reduction at low load and from 67.2 ms to <28 ms (2.4x) at medium load, while at high loads no benefits are observed for neither of 5%/50%/95%/mean packet delay statistics. Observation 7: SBFD significantly reduces the average and 95%-ile DL packet delay especially at low and medium loads of DL traffic. Observation 8: At least 145 dB of RSI and double number of BS antenna elements (Opt 2) are needed to achieve an acceptable performance with some tradeoffs between coverage gain and average system spectral efficiency degradation especially at high load. Observation 9: SBFD Alt 1 is not feasible due to backward compatibility issue with legacy devices. Observation 10: There is no clear advantage of Alt 2 as compared to Alt 3 besides an insignificant reduction in signalling overhead. Observation 11: Compared to Alt 4, Alt 3 may limit the scheduling flexibility in SBFD slots with negligible advantages in terms of reduced signaling overhead. Observation 12: The baseline BWP switch scheme is rather straightforward but has severe UE peak rate and signaling overhead issue. Observation 13: For the advanced BWP based scheme to make at all sense, there should be no additional gap due to BWP switch in the transition from SBFD slots to TDD slots, and vice versa. Observation 14: In terms of digital filter bandwidth adaptation at UE, there is substantially no difference between the RB-set based and the advanced BWP based approach. Observation 15: Unless new in-band emission requirements are specified, better UL transmission and DL reception filtering should not be considered as an advantage of any of the proposed schemes. Observation 16: Operation according to the baseline BWP based scheme is also feasible assuming RB-set based signaling. Observation 17: Only the advanced BWP based scheme requires UE supporting more than one configured DL and UL BWP pair and unaligned center frequencies for a DL and UL BWP pair. Observation 18: Enabling initial access in SBFD symbols can increase the RACH capacity and reduce initial access delay. Observation 19: Options available with current standard specifications to provide different CORESET configurations in different slots/symbols all present limitations in terms of either limited flexibility or excessive signaling overhead. Observation 20: Extension of existing rate matching techniques based on the knowledge of UL subband location is a straightforward solution (assuming SBFD operation Alt 4) that works for both Type 0 and Type 1 FDRA. Observation 21: Use of NTA, offset = 0 is only possible in SBFD slots and it increases the required guard time between the end of a DL transmission in a DL slot and the start of an UL reception in a SBFD slot. Observation 22: Use of NTA, offset = 0 in SBFD slots and NTA, offset > 0 in TDD UL slots further increases the required overhead needed to switch between legacy TDD DL/UL slots and SBFD slots Observation 23: By knowing the time difference of arrival between DL (transmitted by the serving gNB) and UL RS (transmitted by the aggressor UE) at the victim UE, the gNB can estimate the intra-cell (aggressor)UE-to-(victim)UE propagation delay. This information can be used to assist the UE by e.g. providing the specific timing offset to be used when performing a CLI measurement. Proposal 1: Sufficiently large gain under realistic assumptions should be observed from SBFD as compared to TDD to justify the complexity of introducing support for SBFD in the NR specifications. Proposal 2: The Rel-18 SI shall focus on SBFD operation Alt 4 as baseline. Proposal 3: Support RB-set based scheme as baseline. Proposal 4: Support initial access on SBFD symbols by introducing a new SBFD RACH configuration enabling at least Msg1/MsgA transmissions during SBFD symbols. Proposal 5: Study what is required for supporting initial access transmissions other than Msg1/MsgA in SBFD symbols. Proposal 6: For indication of subband locations for SBFD operation, also study dynamic configuration of subband location in time domain. Proposal 7: Study the required enhancements to the TDD frame format signaling framework (UL-DL TDD configuration, SFI, etc.) to enable SBFD operation. Proposal 8: The study on SBFD considers support of U-D and D-U type of subband configurations as baseline. Support of D-U-D type of subband configuration is FFS (pending pros and cons analysis also based on feedback from RAN4). Proposal 9: Study solutions enabling dynamic adaptation of PDCCH resources based on the occurrence of SBFD symbols and the corresponding DL-UL frequency domain partitioning. Proposal 10: Study the severity of PDSCH resource fragmentation problem (and potential solutions) for Type 0 and Type 1 FDRA. Proposal 11: Study and down-select solutions to solve the frequency resource fragmentation problem of CSI-RS in SBFD slots/symbols. Proposal 12: Study solutions to configure the UE with SBFD-specific CG-PUSCH or SPS-PDSCH resources to be used during SBFD slots/symbols. Proposal 13: Consider Option 1 (SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol) and Option 2 (SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol) as baseline for further discussions on UL/DL collision handling at the UE. Proposal 14: consider using NTA, offset = 0 in SBFD slots and NTA, offset > 0 in TDD UL to solve the FFT misalignment problem between UL Rx and DL Tx in SBFD slots, and study solutions to reduce the increased overhead when switching between TDD DL/UL slots and SBFD slots. Proposal 15: Send LS to RAN4 asking if and how much the UE emission requirements can be tightened for SBFD-aware UEs, what is the impact on the PA efficiency, and whether, for a given PA efficiency, the UE being able to match its UL/DL digital filter’s bandwidth to the UL/DL subband in SBFD symbols can help reducing the UE emissions. Proposal 16: Study L1/L2 based CLI, including potential enhancements to the RS configuration for the purpose of L1/L2 based CLI measurement/reporting. Proposal 17: Study schemes for measurement and reporting of time difference of arrival between DL RS and UL RS, and potential extensions to the CLI measurement framework to include assistance information consisting of e.g. specific timing offset (with respect to DL timing) to be used when performing a CLI measurement. Proposal 18: Study subband CLI measurements and reports, including subband CLI-RSSI measurements performed on a subband while the UE is transmitting on a different subband. Proposal 19: Study possible enhancements to the exchange of intended TDD configuration over Xn to include SBFD subband configuration. Proposal 20: The potential benefits of boosting the UE Tx power and/or reducing the gNB power in SBFD slots/symbols shall be further investigated as a potential method to boost the UL received SINR in slots/symbols affected by gNB self-interference. Proposal 21: RAN1 to study the potential benefits of SBFD aware UEs prioritizing SBFD cells and/or legacy UEs de-prioritizing SBFD cells when performing cell (re)selection. Detailed solutions are for RAN2 to discuss and specify if/when normative work for SBFD is agreed. |  |  |
| R1-2210093 | Details of subband non-overlapping full duplex | ASUSTeK |
| Observation 1: Different option of scheduling handling/constraint would be preferred subject to signaling details of subband SFI. Proposal 1: A joint design of scheduling expectation and signaling details of subband SFI is conducted by RAN1. Proposal 2: If subband SFI is designed in a semi-static manner, at least one of option 2, 3, or 4 should be considered. Observation 2: Collision handling rules are separately designed for DCI-based transmission/reception and configured-based transmission/reception. Proposal 3: when discussing the following four option, the “scheduling” based on DCI or “scheduling” based on configuration may need to be distinguished/separately considered as preferred options for the two types of scheduling could be different: - Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol - Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol - Option 3: The SBFD aware UE does not expect to be scheduled with DL reception within the UL subband and may be scheduled with UL transmission outside the UL subband in the SBFD symbol - Option 4: The SBFD aware UE may be scheduled with UL transmission outside the UL subband or DL reception within the UL subband in the SBFD symbol |  |  |
| R1-2210108 | Discussion on subband non-overlapping full duplex | CEWiT |
| Observation 1: Providing frequency location of subband and time instants of SBFD operation to UE will be helpful in scenarios of scheduled periodic signals. Proposal 1: Signaling of frequency location of sub-band and time instants of SBFD operation (SBFD operation alt 4) from gNB to SBFD aware UEs is supported. Observation 2: DL reception within the UL subband in the SBFD symbol will be useful for wideband signals like CSI-RS. Proposal 2: Support Option 2, “The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol”. Proposal 3: gNB configuring separate TDD configuration for the BWPs of the UE is supported. Observation 3: SBFD operation at gNB is limited to the slots/symbols configured as flexible by the TDD common configuration. Proposal 4: Provision to override common TDD configuration by dedicated TDD configuration/SFI in certain scenarios is supported. Proposal 5: TDD configurations starting with UL slots/symbols are supported. Observation 4: Different use cases of SBFD will have impacts on the different channels. Proposal 6: Study whether enhancements to data channel, control channel and RS are needed to support different use cases of SBFD like latency reduction, coverage enhancement etc. Observation 5: Using different panels at the gNB for DL and UL operation reduces interference but impacts reciprocity assumption between UL and DL channels. Proposal 7: Study the impacts of using different panels at the gNB for DL and UL operations on reciprocity assumption of channels. |  |  |
| R1-2210138 | Discussion on subband non-overlapping full duplex | WILUS Inc. |
| - Proposal 1: RAN1 to study dynamic activation/deactivation of semi-static SBFD configuration. - One of multiple SBFD configurations can be activated/deactivated at a given time. - Proposal 2: RAN1 prioritizes to study Option 1 or Option 3, i.e., the SBFD aware UE does not expect to be scheduled with DL reception within the UL subband. - The SBFD aware UE may be scheduled with UL transmission outside the UL subband in flexible symbols by tdd-UL-DL-ConfigurationCommon and/or tdd-UL-DL-ConfigurationDedicated. - Proposal 3: RAN1 prioritizes to study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies in terms of UE implementation and specification impact on BWP operation. - Enhancements on resource allocation can be studied for DL reception or UL transmission. - Proposal 4: RAN1 to study enhancement on frequency hopping for PUSCH and PUCCH in SBFD operation. - Different frequency hopping patterns can be determined across SBFD slot and normal slot in UL repetition case. - Proposal 5: RAN1 to study the UE behavior for collision cases among signals/channels with different directions. - Same or different UE behavior compared to the current UE behavior for collision handling. |  |  |
| R1-2210143 | Inter-UE CLI Test Results for NR Duplex Evolution | KDDI Corporation |
| Observation 1. For the victim UE in the high RSRP regime (RSRP = -95 dBm), - When the inter-UE distance < 1 m, the victim UE is suffered from the inter-UE co-channel CLI. - When the inter-UE distance < 1 m, DL SINR can be improved by 5~15 dB with “BWP on” operation. - However, if the power difference exceeds a certain value, “BWP on” operation gives no DL gain performance. Observation 2. For the victim UE in the high RSRP regime (RSRP = -95 dBm), - Even when the inter-UE distance < 0.1 m, the spectral leakage can be mitigated by using guard RB - In addition, when the BWP is used, the number of guard RBs can be reduced by more than 10 RB and spectral efficiency can be improved. For the victim UE in the low RSRP regime (RSRP = -110 dBm), - When the inter-UE distance < 1 m, the DL performance is still degraded even with the higher number of guard RBs due to limited dynamic range of ADC Observation 3. For the victim UE in the high RSRP regime (RSRP = -95 dBm), - Inter-UE adjacent-channel CLI is mitigated by using the guard RB and “BWP on” operation. For the victim UE in the low RSRP regime (RSRP = -110 dBm), - the DL performance is still degraded (same as the inter-UE co-channel CLI case) - Spectral leakage can be mitigated with the wider guard band (e.g., more than 40 MHz ) - However, limited dynamic range is inevitable when band-wise RF filtering is applied during UE processing Proposal 1. RAN1 to evaluate impacts of inter-UE adjacent-channel CLI and investigate the feasibility of mitigation methods for inter-UE adjacent channel CLI. Proposal 2. RAN1 to study the feasibility and potential benefit of SBFD scheme with more than one configured DL and UL pair with aligned/unaligned center frequencies for a DL and UL BWP pair. |  |  |

Appendix B: Previous agreements of SBFD

RAN1#109-e

Agreement

Study whether/how to inform the UE of the time and/or frequency location of subbands that gNB would use for SBFD operation.

Agreement

Study the impact/potential enhancements of resource allocation in symbols with subbands that gNB would use for SBFD operation.

Agreement

At least study SBFD operation within a TDD carrier

Conclusion

For discussion purpose only, SBFD symbols is defined as symbols with subbands that gNB would use for SBFD operation.

Conclusion

For discussion purpose, for SBFD operation within a TDD carrier, a SBFD subband consists of 1 RB or a set of consecutive RBs for the same transmission direction.

Agreement

The time and frequency location of subbands within a TDD carrier are not fixed in the specification.

- Subject to any RAN4 guidance on minimum or maximum subband and guardband size and subband location within TDD carrier.

- Note that whether the time and/or frequency location of subbands are informed to UE is separately discussed.

Guideline for future meetings

- Note: AI 9.3.3 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for dynamic TDD and schemes that are common for both SBFD and dynamic/flexible TDD.

- Note: AI 9.3.2 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD.

RAN1#110

Agreement

Study the following alternatives with Alt 4 prioritized, for SBFD operation at least for RRC_CONNECTED state.

- SBFD operation Alt 1:

  - Time and frequency locations of subbands for SBFD operation are not known to UEs.

  - UE behaviors follow existing specifications without introducing new UE behaviors for SBFD operation at gNB side.

- SBFD operation Alt 2:

  - Time and frequency locations of subbands for SBFD operation are not known to UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs

- SBFD operation Alt 3:

  - Only time location of subbands for SBFD operation is known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time location of subbands for SBFD operation

- SBFD operation Alt 4:

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

UE capability discussion is held in work item phase.

Agreement

For indication of subband locations for SBFD operation, study semi-static configuration of subband time and frequency location as baseline.

Agreement

For semi-static configuration of subband location, consider same subband frequency resources across different SBFD symbols as baseline.

Working Assumption

For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline.

- FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies

- FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair

Agreement

For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, study the following options:

- Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol

- Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol

- Option 3: The SBFD aware UE does not expect to be scheduled with DL reception within the UL subband and may be scheduled with UL transmission outside the UL subband in the SBFD symbol

- Option 4: The SBFD aware UE may be scheduled with UL transmission outside the UL subband or DL reception within the UL subband in the SBFD symbol

Agreement

Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for SBFD, at least includes:

- Measurement resource/reporting configuration

- Measurement/reporting details (including UE processing delay)

- Relevant information exchange (between gNBs) if needed

- Usage of measurement at gNB

Note: other enhancement(s) for gNB-to-gNB and UE-to-UE CLI handling specific for SBFD are not precluded.