3GPP TSG RAN WG1 Meeting #110bis-e					                     R1-2210334

e-Meeting, October 10th – 19th, 2022

Agenda item:	8.11

Source:		Moderator (LG Electronics)

Title:	Feature lead summary #1 for AI 8.11: Inter-UE coordination for Mode 2 enhancements

Document for:	Discussion and information

- Introduction

As the outcome of preparation phase discussion, the following maintenance issues were selected to be addressed at RAN1#110bis-e meeting.

[110bis-e-R17-Sidelink-04] Email discussion for maintenance on inter-UE coordination for mode 2 enhancements, for scheme 1 and for scheme 2, and on miscellaneous topics, for the following issues in R1-2210333 – Seungmin (LGE)

- Issues 2-15, 2-4, 2-10, 3-1, 3-3

- Editorial issues for providing to spec editors: issues 2-2, 2-7, 2-8, 2-9, 2-17, 2-11, 2-12, 2-13

- Check points: October 14, October 19

- High priority issues for discussion

  - IUC for mode 2 enhancement

    - [ACTIVE] Issue #2-15: Further clarification on the use of preferred resource set for resource reselection due to pre-emption/re-evaluation and/or the indication of non-preferred resource set to physical layer for resource reselection due to pre-emption/re-evaluation

      - Background

Two contributions [20][29] proposed to have further clarification on the use of preferred resource set for resource reselection due to pre-emption/re-evaluation and/or the indication of non-preferred resource set to physical layer for resource reselection due to pre-emption/re-evaluation.

[Sharp: R1-2209683] As summarized in [8], the potential issue which further clarifies the use of preferred resource set for resource re-selection due to pre-emption/re-evaluation (i.e. issue 19) was marked with high priority for future discussion. The related existing agreement in RAN1#106-e is copied as follows, Agreement - In scheme 1, at least following UE-B’s behavior in its resource (re-)selection is supported when it receives inter-UE coordination information from UE-A: - For preferred resource set, the following two options are supported: - Option A): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set in combination with its own sensing result - UE-B uses in its resource (re-)selection, resource(s) not belonging to the preferred resource set when condition(s) are met - FFS: Details of condition(s) - This option is supported when UE-B performs sensing/resource exclusion - FFS: Other details (if any) - Option B): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set - This option is supported at least when UE-B does not support sensing/resource exclusion - FFS: Whether the support is conditional or UE capability - FFS: Other details (if any) - FFS: Other option(s), and other details (if any) - For non-preferred resource set, - UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information - UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set - FFS: Details including - Whether/how UE-B can use in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set, definition of the overlap, and other details (if any) - When UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set - FFS: UE-B reselects in its resource (re-)selection, resource(s) to be used for its transmission when the resource(s) are fully/partially overlapping with the non-preferred resource set - FFS: Other option(s), and other details (if any). In our views, besides the issue of preferred resource set as mentioned in [9], when a non-preferred resource set is received from another UE, MAC layers shall indicate the non-preferred resource set to the physical layer for candidate resources exclusion procedure, i.e. Step 6b) as specified in clause 8.1.4C of TS38.214. For resource re-selection due to pre-emption/re-evaluation, the indication of the received non-preferred resource set to physical layer is missing in current TS38.321, as follows. TS38.321 #### 5.22.1.1 SL Grant reception and SCI transmission […] 3> if configured by RRC, sl-InterUE-CoordinationScheme1 enabling reception of preferred resource set and non-preferred resource set and when the UE has own sensing result as specified in clause 8.1.4 of TS 38.214 [7] and if a non-preferred resource set is received from a UE: 4> indicate the received non-preferred resource set to physical layer. […] #### 5.22.1.2a Re-evaluation and Pre-emption […] If the MAC entity has been configured with Sidelink resource allocation mode 2 to transmit using pool(s) of resources in a carrier as indicated in TS 38.331 [5] or TS 36.331 [21] based on sensing or random selection the MAC entity shall for each Sidelink process: 1> if a resource(s) of the selected sidelink grant which has not been identified by a prior SCI is indicated for re-evaluation by the physical layer as specified in clause 8.1.4 of TS 38.214 [7]; 2> remove the resource(s) from the selected sidelink grant associated to the Sidelink process; 2> randomly select the time and frequency resource from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] for either the removed resource or the dropped resource, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of either SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources, and that a resource can be indicated by the time resource assignment of an SCI for a retransmission according to clause 8.3.1.1 of TS 38.212 [9]; 2> replace the removed or dropped resource(s) by the selected resource(s) for the selected sidelink grant. 1> if any resource(s) of the selected sidelink grant which has been indicated by a prior SCI is indicated for pre-emption by the physical layer as specified in clause 8.1.4 of TS 38.214 [7]: 2> remove the resource(s) from the selected sidelink grant associated to the Sidelink process; 2> if one or multiple SL DRX is configured: 3> randomly select the time and frequency resource from the resources later than the resources for either the removed resource or the dropped resource indicated by a prior SCI, from the resource indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] which occur within the SL DRX active time as specified in clause 5.28.3 of the destination UE selected for indicating to the physical layer the SL DRX active time above, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of either SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources, and that a resource can be indicated by the time resource assignment of an SCI for a retransmission according to clause 8.3.1.1 of TS 38.212 [9]. 2> else: 3> randomly select the time and frequency resource from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] for either the removed resource or the dropped resource, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of either SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources, and that a resource can be indicated by the time resource assignment of an SCI for a retransmission according to clause 8.3.1.1 of TS 38.212 [9]. […]. Proposal 1: Send an LS to ask RAN2 to capture procedures related to preferred resource set and indication of non-preferred resource set to physical layer for resource re-selection due to pre-emption/re-evaluation. [DOCOMO: R1-2209881] 38.321 5.22.1.1 SL Grant reception and SCI transmission ... 3> if configured by RRC, sl-InterUE-CoordinationScheme1 enabling reception of preferred resource set and non-preferred resource set and when the UE does not have own sensing result as specified in clause 8.1.4 of TS 38.214 [7] and if a preferred resource set is received from a UE: 4> randomly select the time and frequency resources for one transmission opportunity from the resources belonging to the received preferred resource set for a MAC PDU to be transmitted to the UE providing the preferred resource set, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 3> if configured by RRC, sl-InterUE-CoordinationScheme1 enabling reception of preferred resource set and non-preferred resource set and when the UE has own sensing result as specified in clause 8.1.4 of TS 38.214 [7] and if a preferred resource set is received from a UE: 4> randomly select the time and frequency resources for one transmission opportunity within the intersection of the received preferred resource set and the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] for a MAC PDU to be transmitted to the UE providing the preferred resource set, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 4> if there are no resources within the intersection that can be selected as the time and frequency resources for the one transmission opportunity according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 5> randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 4> use the randomly selected resource to select a set of periodic resources spaced by the resource reservation interval for transmissions of PSCCH and PSSCH corresponding to the number of transmission opportunities of MAC PDUs determined in TS 38.214 [7]. … 5.22.1.2a Re-evaluation and Pre-emption ... 2> randomly select the time and frequency resource from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] for either the removed resource or the dropped resource, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of either SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources, and that a resource can be indicated by the time resource assignment of an SCI for a retransmission according to clause 8.3.1.1 of TS 38.212 [9]; … 2> if one or multiple SL DRX is configured: 3> randomly select the time and frequency resource from the resources later than the resources for either the removed resource or the dropped resource indicated by a prior SCI, from the resource indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] which occur within the SL DRX active time as specified in clause 5.28.3 of the destination UE selected for indicating to the physical layer the SL DRX active time above, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of either SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources, and that a resource can be indicated by the time resource assignment of an SCI for a retransmission according to clause 8.3.1.1 of TS 38.212 [9]. 2> else: 3> randomly select the time and frequency resource from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] for either the removed resource or the dropped resource, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of either SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources, and that a resource can be indicated by the time resource assignment of an SCI for a retransmission according to clause 8.3.1.1 of TS 38.212 [9]. … 5.22.1.2b Re-selection for using a received resource conflict indication ... 2> randomly select the time and frequency resource from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] excluding the conflict resource(s) for the removed resource, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of either SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources, and that a resource can be indicated by the time resource assignment of an SCI for a retransmission according to clause 8.3.1.1 of TS 38.212 [9];. In the current specification, non-preferred resources are excluded from S_A at PHY layer and preferred resources are preferentially selected in selection procedure from S_A at MAC layer. Once we see MAC spec carefully, it can be found that preferred resources are used in resource selection procedure but not in resource reselection procedure due to re-evaluation/pre-emption/conflict information. However, ‘resource (re)selection’ in RAN1 agreements include the resource reselection procedure due to re-evaluation/pre-emption/conflict information. MAC spec should be updated such that preferred resources, if available, are used when resource reselection is performed due to re-evaluation/pre-emption/conflict information. One note is that non-preferred resource is used also for this purpose in PHY layer. There would be no reason to skip using preferred resources. Proposal 2: - Send an LS to ask RAN2 to use preferred resources for resource reselection due to re-evaluation/pre-emption/conflict information, if available.

      - FL Proposals for Round 1

Draft proposal 2-15 (I):

- Send an LS to ask RAN2 to capture the followings in TS 38.321:

  - MAC layer uses the received preferred resource set for resource re-selection due to re-evaluation/pre-emption/resource conflict indication (if available)

  - MAC layer indicates the received non-preferred resource set to PHY layer for resource re-selection due to re-evaluation/pre-emption/resource conflict indication (if available)

    - [ACTIVE] Issue #2-4: Further clarification on how to determine the priority of PSFCH TX when the resource conflict is determined based on Condition 2-A-2

      - Background

Two contributions [4][18] proposed to have further clarification on how to determine the priority of PSFCH TX when the resource conflict is determined based on Condition 2-A-2.

[vivo: R1-2208611] Reason for change: 1. It has been agreed that the priority value of PSFCH TX for a resource conflict indication is the smallest priority value of the conflicting TBs. However, the rule is applied only for the case when the resource conflict is detected based on Condition 2-A-1. It has not discussed how to determine the PSFCH TX priority when the resource conflict is determined based on Condition 2-A-2, e.g., when UE-A does not expect to perform SL reception from UE-B due to its UL transmission. Since there is only a single SL TB transmission when determining the resource conflict based on condition 2-A-2, it is straightforward to assume that priority value of PSFCH TX for a resource conflict indication is the priority value associated with the TB transmission. Summary of change: Capture the following correction for Clause 16.2.4.2: 1. Clarification on the priority value of PSFCH for conflict indication when resource conflict is determined based on Condition 2-A-2, i.e., when there is only a single SCI format 1-A associated with the PSFCH. Consequence if not approved: It is not clear on how to determine the PSFCH TX priority when the resource conflict is determined based on Condition 2-A-2. ### 16.2.4.2 Simultaneous PSFCH transmission/reception For a PSFCH transmission or reception with HARQ-ACK information, a priority value for the PSFCH is equal to the priority value indicated by an SCI format 1-A associated with the PSFCH, or equal to the priority value determined by the corresponding SCI format 1-A for the conflicting resource if there is a single SCI format 1-A associated with the resource conflict determination. For PSFCH transmission with conflict information, a priority value for the PSFCH is equal to the smallest priority value determined by the corresponding SCI formats 1-A for the conflicting resources. ===omitted===. [Apple: R1-2209565] Reason for change: For condition 2-A-1 of inter-UE coordination scheme 2, where other UE’s reserved resource(s) identified by UE-A are fully/partially overlapping with resource(s) indicated by UE-B’s SCI in time-and-frequency, the priority value of PSFCH transmission with conflict information is equal to the smallest priority value of the conflicting TBs. For condition 2-A-2 of inter-UE coordination scheme 2, where UE-A, as an intended receiver of UE-B, does not expect to perform sidelink reception from UE-B due to half duplex operation on the resource(s) indicated by UE-B’s SCI, the priority value of PSFCH transmission with conflict information is undefined. Summary of change: For condition 2-A-2 of inter-UE coordination scheme 2, the priority value of PSFCH transmission with conflict information is equal to the priority value of UE-B’s SCI. Consequence if not approved: Specification is unclear for the priority value of PSFCH transmission with conflict information for condition 2-A-2. ==================== Start of Text Proposal ==================== #### 16.2.4.2 Simultaneous PSFCH transmission/reception For a PSFCH transmission or reception with HARQ-ACK information, a priority value for the PSFCH is equal to the priority value indicated by an SCI format 1-A associated with the PSFCH. For PSFCH transmission with conflict information, a priority value for the PSFCH is equal to the smallest priority value determined by the corresponding SCI formats 1-A for the conflicting resources, or is equal to the priority value determined by the corresponding SCI format 1-A associated with the resource(s) in slot(s) in which a UE does not expect to perform sidelink reception due to half duplex operation, if the UE is a destination UE of the TB associated with the SCI format 1-A. For PSFCH reception with conflict information, a priority value for the PSFCH is equal to the priority value determined by the corresponding SCI format 1-A for the conflicting resource. < Unchanged parts are omitted > ==================== End of Text Proposal ====================.

      - FL Proposals for Round 1

Draft proposal 2-4 (I):

- Reason for change:

- For Condition 2-A-2 of inter-UE coordination scheme 2, where UE-A, as an intended receiver of UE-B, does not expect to perform sidelink reception from UE-B due to half duplex operation on the resource(s) indicated by UE-B’s SCI, the priority value of PSFCH transmission with conflict information is undefined.

- Summary of change:

- For Condition 2-A-2 of inter-UE coordination scheme 2, the priority value of PSFCH transmission with conflict information is equal to the priority value of UE-B’s SCI.

- Consequences if not approved:

- Specification is unclear for the priority value of PSFCH transmission with conflict information for Condition 2-A-2.

--------------------------- Start of text proposal to TS 38.213 v17.3.0 --------------------------- #### 16.2.4.2 Simultaneous PSFCH transmission/reception For a PSFCH transmission or reception with HARQ-ACK information, a priority value for the PSFCH is equal to the priority value indicated by an SCI format 1-A associated with the PSFCH. For PSFCH transmission with conflict information, a priority value for the PSFCH is equal to the smallest priority value determined by the corresponding SCI formats 1-A for the conflicting resources, or is equal to the priority value determined by the corresponding SCI format 1-A associated with the resource(s) in slot(s) in which a UE does not expect to perform sidelink reception due to half duplex operation, if the UE is a destination UE of the TB associated with the SCI format 1-A. For PSFCH reception with conflict information, a priority value for the PSFCH is equal to the priority value determined by the corresponding SCI format 1-A for the conflicting resource. < Unchanged parts are omitted > --------------------------- End of Text proposal to TS 38.213 v17.3.0 ---------------------------

    - [ACTIVE] Issue #2-10: Further clarification on Condition 2-A-2 for Scheme 2

      - Background

Three contributions [9][19][20] proposed to have further clarification on Condition 2-A-2 for Scheme 2.

[ZTE: R1-2208718] Reason for change: For Condition 2-A-2, further clarification on conditions for UE to be UE-B is needed, since it is infeasible to transmit the collision indication if the PSFCH occasion for resource conflict information of the second UE is passed. Nor should the collision indication be transmitted if the indicationUEB flag in SCI Format 1-A from the second UE is set to 0, if indicationUEBScheme2 = 'enabled'. Based on this, these conditions should be added for Condition 2-A-2. Summary of change: Add the related description on conditions for UE to be UE-B for Condition 2-A-2 in TS38.213. Consequence if not approved: How to determine UE-B for Condition 2-A-2 of Scheme 2 is not clear. - 16.3.0 UE procedure for transmitting PSFCH with control information < Unchanged parts are omitted > A first UE determines a second UE for providing the conflict information to in a PSFCH as follows - if the first UE is an intended receiver of the second UE for a reserved resource of a PSSCH transmission in a slot, - does not expect to perform reception on the sidelink due to half-duplex operation in the slot, and - the PSFCH occasion for resource conflict information of the second UE is valid - the conflict information receiver flag in SCI Format 1-A from the second UE is set to 1, if indicationUEBScheme2 = 'enabled' - determines to transmit to the second UE the PSFCH with the conflict information. < Unchanged parts are omitted >. [Sharp: R1-2209682, R1-2209683] Reason for change: As explained in section 3.3 of R1-2209683, the RAN1 agreements on “conflict information receiver flag” in SCI format 1-A did not put any restriction on its applicability in Condition 2-A-1 or Condition 2-A-2. However, currently only the use of the flag in Condition 2-A-1 has been captured in TS 38.213. Summary of change: Addition of handling of conflict information receiver flag in Condition 2-A-2. Consequence if not approved: Incomplete support of the conflict information receiver flag. <Unchanged text omitted> ## 16.3 UE procedure for reporting and obtaining control information in PSFCH Control information provided by a PSFCH transmission includes HARQ-ACK information or conflict information. ### 16.3.0 UE procedure for transmitting PSFCH with control information <Unchanged text omitted> A first UE determines a second UE for providing the conflict information to in a PSFCH as follows - if the first UE is an intended receiver of the second UE for a reserved resource of a PSSCH transmission in a slot, - the conflict information receiver flag in SCI Format 1-A from the second UE is set to 1, if sl-IndicationUE-B = 'enabled' - does not expect to perform reception on the sidelink due to half-duplex operation in the slot, and - determines to transmit to the second UE the PSFCH with the conflict information. A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, typeAUEScheme2 is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - the conflict information receiver flag in SCI Format 1-A from the second UE and the third UE is set to 1, if sl-IndicationUE-B = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 <Unchanged text omitted>.

      - FL Proposals for Round 1

Draft proposal 2-10 (I):

- Reason for change:

- For Condition 2-A-2 of inter-UE coordination scheme 2, further clarification on conditions for UE to be UE-B is needed, since it is infeasible to transmit the conflict indication if the PSFCH occasion for resource conflict information of the second UE is passed. Nor should the conflict indication be transmitted if the conflict information receiver flag in SCI Format 1-A from the second UE is set to 0, if sl-IndicationUE-B = 'enabled'. Based on this, these conditions should be added for Condition 2-A-2.

- Summary of change:

- Add the related description on conditions for UE to be UE-B for Condition 2-A-2 in TS 38.213.

- Consequences if not approved:

- Incomplete support of the conflict information receiver flag.

--------------------------- Start of text proposal to TS 38.213 v17.3.0 --------------------------- ### 16.3.0 UE procedure for transmitting PSFCH with control information < Unchanged parts are omitted > A first UE determines a second UE for providing the conflict information to in a PSFCH as follows - if the first UE is an intended receiver of the second UE for a reserved resource of a PSSCH transmission in a slot, - does not expect to perform reception on the sidelink due to half-duplex operation in the slot, and - the PSFCH occasion for resource conflict information of the second UE is valid - the conflict information receiver flag in SCI Format 1-A from the second UE is set to 1, if sl-IndicationUE-B = 'enabled' - determines to transmit to the second UE the PSFCH with the conflict information. < Unchanged parts are omitted > --------------------------- End of Text proposal to TS 38.213 v17.3.0 ---------------------------

  - Others

    - [ACTIVE] Issue #3-1: Clarification on PDCCH repetition for SL DCI format 3-0 and/or 3-1

      - Background

Three contributions [39][40][41] proposed to have clarification on PDCCH repetition for SL DCI format 3-0 and/or 3-1.

[vivo: R1-2208614] In Rel-17, PDCCH repetition is introduced for PDCCH reliability enhancements in eMIMO. Two USS sets or two type3 CSS sets in the same BWP can be explicitly linked if they are configured with a same value for searchSpaceLinkingId. The two linked SS sets should have the same periodicity, same offset, same duration, same number of monitoring occasions, same AL, and the same number of candidates for each AL. The n-th monitoring occasion of one SS set in a slot is linked to n-th monitoring occasion of the other SS set in the same slot, and two linked PDCCH candidates in the two SS sets have the same candidate index. Moreover, the two PDCCH candidates of repetition should have the same number of CCEs and same coded bits, and correspond to the same DCI payload. An example of PDCCH repetition is as below, whereas the PDCCH candidate of SS0 is linked with the PDCCH candidate of SS1. The related RAN1 and RAN2 specifications are copied below. Figure 1. Example of PDCCH repetition TS 38.331 SearchSpaceLinkingId This parameter is used to link two search spaces of same type in the same BWP. If two search spaces have the same SearchSpaceLinkingId UE assumes these search spaces are linked to PDCCH repetition REF. When PDCCH repetition is monitored in two linked search space (SS) sets, the UE does not expect a third monitored SS set to be linked with any of the two linked SS sets. The two linked SS sets have the same SS set type (USS/CSS). The two linked SS sets have the same DCI formats to monitor. For intra-slot PDCCH repetition: The two SS sets should have the same periodicity and offset (monitoringSlotPeriodicityAndOffset), and the same duration. For linking monitoring occasions across the two SS sets that exist in the same slot: The two SS sets have the same number of monitoring occasions within a slot and n-th monitoring occasion of one SS set is linked to n-th monitoring occasion of the other SS set. The following SS sets cannot be linked with another SS set for PDCCH repetition: SS set 0, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, searchSpaceBroadcast, peiSearchSpace, and sdt-SearchSpace. SS set configured by recoverySearchSpaceId cannot be linked to another SS set for PDCCH repetition. When a scheduled serving cell is configured to be cross-carrier scheduled by a scheduling serving cell, two PDCCH candidates (with the same AL and candidate index associated with the scheduled serving cell) are linked only if the corresponding two SS sets in the scheduling serving cell are linked and two SS sets in the scheduled serving cell with the same SS set IDs are also linked.. TS 38.213 When PDCCH repetition is configured, a PDCCH reception includes two PDCCH candidates from corresponding two search space sets, - a PDCCH monitoring occasion is the union of the PDCCH monitoring occasions for the two PDCCH candidates - the start of the PDCCH reception is the start of the earlier PDCCH candidate - the end of the PDCCH reception is the end of the PDCCH candidate that ends later … For search space sets si and sj that include searchSpaceLinkingId with same value, a UE monitors, in monitoring occasions with same index according to each of search space sets si and sj in a slot, PDCCH candidates msi,nCI(L) and msj,nCI(L), with msi,nCI(L)=msj,nCI(L), for detection of a DCI format with same information. The UE expects ksi=ksj, osi=osj, Tsi=Tsj, Msi(L)=Msj(L), and a same number of non-overlapping PDCCH monitoring occasions per slot based on corresponding monitoringSymbolsWithinSlot, for search space sets si and sj.. There was no discussion in eMIMO on whether PDCCH repetition for SL DCI format is supported or not. On the one hand, there is no explicit agreement to support PDCCH repetition specifically for SL DCI. On the other hand, according to the existing specifications (i.e., field description of searchSpaceLinkingId in RAN2 specificaiton), this feature seems to already be applicable to any DCI format including SL DCI format. RAN1 should clarify whether PDCCH repetition is supported for SL DCI format. Observation 1. According to current specification, PDCCH repetition can be configured for any USS/type3 CSS with any DCI format including SL DCI format. Proposal 1. RAN1 should clarify if PDCCH repetition is supported for SS sets configured with SL DCI format 3-0 and/or 3-1. If PDCCH repetition is supported for SL DCI format, some clarifications are needed. In Uu, as there are two PDCCH candidates per PDCCH reception when PDCCH repetition is configured, a list of agreements have been achieved to specify which PDCCH candidate is used as the reference point to deriving the processing timeline, etc. Some related agreements and spec text are copied in annex. In summary, for Uu scheduling: - The PDCCH candidates that starts ealier in time is used for - Definition of counter DAI / total DAI and Type-2 HARQ-Ack codebook construction - As cDAI/tDAI is related to the accumulative number of monitoring occasions in which DCIs with the corresponding PUCCH are received, and the monitoring occasions are indexed in increasing order of the starting time of the occasions, it is natural to use PDCCH candidates starting earlier for cDAI/tDAI determination. - Determining the last DCI for PUCCH resource determination based on the PRI field of the last DCI - Similarly, as the PDCCH monitoring occasion is used to determine the last DCI format for PUCCH determination and the PDCCH monitoring occasions carrying DCIs associated with a same PUCCH slot are indexed in increasing order of the starting time of the occasions, the PDCCH candidates starting earlier is used for determining the PUCCH resource. - The PDCCH candidates that ends later in time is used for - For PUSCH preparation time (N2) - With regard to the PUSCH preparation time, as UE may drop the monitoring of some PDCCH candidates due to collision(e.g., collision with SSB/CRS), each PDCCH candidate should ensure sufficient time for PUSCH preparation and transmission, thus the later PDCCH is used to determine the ‘last symbol of the PDCCH carrying the DCI scheduling the PUSCH’. To support PDCCH repetition for SL DCI format, the same clarifications specified to Uu DCI should be copied to SL DCI as well (i.e., for PUCCH determination/SAI counting/PSSCH preparation time): - The PDCCH candidate that starts earlier is used for - SAI counting - Last DCI format 3-0 for PUCCH resource determination - The PDCCH candidate that ends later is used for - For PSSCH preparation time Draft CRs [1][2] based on the above changes are prepared for reference. On the other hand, if PDCCH repetition for SL DCI format is not supported, RAN1 should send a LS to RAN2 for updating TS 38.331, i.e., to exclude SL DCI format from PDCCH repetition, as follows: SearchSpaceLinkingId This parameter is used to link two search spaces of same type in the same BWP. If two search spaces have the same SearchSpaceLinkingId UE assumes these search spaces are linked to PDCCH repetition REF. When PDCCH repetition is monitored in two linked search space (SS) sets, the UE does not expect a third monitored SS set to be linked with any of the two linked SS sets. The two linked SS sets have the same SS set type (USS/CSS). The two linked SS sets have the same DCI formats to monitor. For intra-slot PDCCH repetition: The two SS sets should have the same periodicity and offset (monitoringSlotPeriodicityAndOffset), and the same duration. For linking monitoring occasions across the two SS sets that exist in the same slot: The two SS sets have the same number of monitoring occasions within a slot and n-th monitoring occasion of one SS set is linked to n-th monitoring occasion of the other SS set. The following SS sets cannot be linked with another SS set for PDCCH repetition: SS set 0, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, searchSpaceBroadcast, peiSearchSpace, and sdt-SearchSpace. SS set configured by recoverySearchSpaceId cannot be linked to another SS set for PDCCH repetition. When a scheduled serving cell is configured to be cross-carrier scheduled by a scheduling serving cell, two PDCCH candidates (with the same AL and candidate index associated with the scheduled serving cell) are linked only if the corresponding two SS sets in the scheduling serving cell are linked and two SS sets in the scheduled serving cell with the same SS set IDs are also linked. This parameter is not applicable to search space configured with dci-FormatsSL for monitoring for format 3-0, or for format 3-1, or for formats 3-0 and 3-1.. Proposal 2. If PDCCH repetition is supported for SS sets with SL DCI format, for a PDCCH reception including two PDCCH candidates, RAN1 should clarify which PDCCH candidate is used to determine the PUCCH for SL HARQ-ACK/SAI counting/PSSCH preparation time. Proposal 3. If PDCCH repetition is not supported for SS sets with SL DCI format, RAN1 should send a LS to RAN2 to remove the support of PDCCH repetition for SS sets configured with SL DCI format 3-0 and/or 3-1. [vivo: 2208615] Reason for change: When PDCCH repetition is configured for SL DCI format, it should be clarified that PDCCH reception includes two PDCCH candidates, PDCCH monitoring occasion is the union of the two PDCCH candidates, and PDCCH monitoring occasion is indexed based on their start time. The PDCCH candidate that starts earlier is used to determine the PUCCH resource for SL HARQ-ACK reporting and SAI counting. Summary of change: The same clarification on reporting UL HARQ-ACK for PDCCH repetition is applied for SL HARQ-ACK reporting: Clarify that when PDCCH repetition is configured for SL DCI format, PDCCH reception includes two PDCCH candidates, and PDCCH monitoring occasion is the union of the two PDCCH candidates, and PDCCH monitoring occasion is indexed based on their start time. The PDCCH candidate that starts earlier is used to determine the PUCCH resource for SL HARQ-ACK reporting and SAI counting. Consequence if not approved: PDCCH repetition is not supported for SL DCI format. ## 16.5 UE procedure for reporting HARQ-ACK on uplink In the remaining of this clause, when a PDCCH reception by a UE includes two PDCCH candidates from corresponding search space sets, as described in clause 10.1 - a PDCCH monitoring occasion is the union of the PDCCH monitoring occasions for the two PDCCH candidates - the start of the PDCCH reception is the start of the earlier PDCCH candidate - the end of the PDCCH reception is the end of the PDCCH candidate that ends later The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10 (except clause 10.4), 11.1, and 11.1.1. In the remaining of this clause, a last DCI format is the DCI format that a UE detects in a last PDCCH monitoring occasion from the PDCCH monitoring occasions for which the UE would provide SL HARQ-ACK information in a PUCCH in a same slot. A UE can be provided PUCCH resources or PUSCH resources [12, TS 38.331] to report HARQ-ACK information that the UE generates based on HARQ-ACK information that the UE obtains from PSFCH receptions, or from absence of PSFCH receptions. The UE reports HARQ-ACK information on the primary cell of the PUCCH group, as described in clause 9, of the cell where the UE monitors PDCCH for detection of DCI format 3_0. For SL configured grant Type 1 or Type 2 PSSCH transmissions by a UE within a time period provided by sl-PeriodCG, the UE generates one HARQ-ACK information bit in response to the PSFCH receptions to multiplex in a PUCCH transmission occasion that is after a last time resource, in a set of time resources. For PSSCH transmissions scheduled by a DCI format 3_0, a UE generates HARQ-ACK information in response to PSFCH receptions to multiplex in a PUCCH transmission occasion that is after a last time resource in a set of time resources provided by the DCI format 3_0. From a number of PSFCH reception occasions, the UE generates HARQ-ACK information to report in a PUCCH or PUSCH transmission. The UE can be indicated by a SCI format to perform one of the following and the UE constructs a HARQ-ACK codeword with HARQ-ACK information, when applicable - for one or more PSFCH reception occasions associated with SCI format 2-A with Cast type indicator field value of "10" - generate HARQ-ACK information with same value as a value of HARQ-ACK information the UE determines from the last PSFCH reception from the number of PSFCH reception occasions corresponding to PSSCH transmissions or, if the UE determines that a PSFCH is not received at the last PSFCH reception occasion and ACK is not received in any of previous PSFCH reception occasions, generate NACK - for one or more PSFCH reception occasions associated with SCI format 2-A with Cast type indicator field value of "01" - generate ACK if the UE determines ACK from at least one PSFCH reception occasion, from the number of PSFCH reception occasions corresponding to PSSCH transmissions, in PSFCH resources corresponding to every identity MID of the UEs that the UE expects to receive the PSSCH, as described in clause 16.3; otherwise, generate NACK - for one or more PSFCH reception occasions associated with SCI format 2-B or SCI format 2-A with Cast type indicator field value of "11" - generate ACK when the UE determines absence of PSFCH reception for the last PSFCH reception occasion from the number of PSFCH reception occasions corresponding to PSSCH transmissions; otherwise, generate NACK After a UE transmits PSSCHs and receives PSFCHs in corresponding PSFCH resource occasions, the priority value of HARQ-ACK information is same as the priority value of the PSSCH transmissions that is associated with the PSFCH reception occasions providing the HARQ-ACK information. The UE generates a NACK when, due to prioritization, as described in clause 16.2.4, the UE does not receive PSFCH in any PSFCH reception occasion associated with a PSSCH transmission in a resource provided by a DCI format 3_0 or, for a configured grant, in a resource provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the NACK is same as the priority value of the PSSCH transmission. The UE generates a NACK when, due to prioritization as described in clause 16.2.4, the UE does not transmit a PSSCH in any of the resources provided by a DCI format 3_0 or, for a configured grant, in any of the resources provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the NACK is same as the priority value of the PSSCH that was not transmitted due to prioritization. The UE generates an ACK if the UE does not transmit a PSCCH with a SCI format 1-A scheduling a PSSCH in any of the resources provided by a configured grant in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the ACK is same as the largest priority value among the possible priority values for the configured grant. The UE generates an ACK if the UE does not transmit a PSCCH with a SCI format 1-A scheduling a PSSCH in any of the resources provided by a DCI format 3_0 and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the ACK is same as the largest priority value among the possible priority values for the dynamic grant. For reporting HARQ-ACK information on uplink corresponding to one or multiple PSSCH transmissions with a corresponding SCI format with the field 'HARQ feedback enabled/disabled indicator' set to disabled, the UE generates HARQ-ACK information with the contents instructed by higher layer. The priority value of the HARQ-ACK information is same as the priority value of the PSSCH transmission. A UE does not expect to be provided PUCCH resources or PUSCH resources to report HARQ-ACK information that start earlier than Tprep= N+1∙2048+144∙κ∙2-μ∙Tc after the end of a last symbol of a last PSFCH reception occasion, from a number of PSFCH reception occasions that the UE generates HARQ-ACK information to report in a PUCCH or PUSCH transmission, where - κ and Tc are defined in [4, TS 38.211] - μ=min⁡(μSL,μUL), where μSL is the SCS configuration of the SL BWP and μUL is the SCS configuration of the active UL BWP on the primary cell - N is determined from μ according to Table 16.5-1 Table 16.5-1: Values of N μ; N. 0; 14. 1; 18. 2; 28. 3; 32. For DCI format 3_0, if present, the PSFCH-to-HARQ feedback timing indicator field values map to values for a set of number of slots provided by sl-PSFCH-ToPUCCH as defined in Table 16.5-2. Table 16.5-2: Mapping of PSFCH-to-HARQ feedback timing indicator field values to numbers of slots PSFCH-to-HARQ feedback timing indicator; Number of slots. 1 bit; 2 bits; 3 bits. '0'; '00'; '000'; 1st value provided by sl-PSFCH-ToPUCCH. '1'; '01'; '001'; 2nd value provided by sl-PSFCH-ToPUCCH. '10'; '010'; 3rd value provided by sl-PSFCH-ToPUCCH. '11'; '011'; 4th value provided by sl-PSFCH-ToPUCCH. '100'; 5th value provided by sl-PSFCH-ToPUCCH. '101'; 6th value provided by sl-PSFCH-ToPUCCH. '110'; 7th value provided by sl-PSFCH-ToPUCCH. '111'; 8th value provided by sl-PSFCH-ToPUCCH. With reference to slots for PUCCH transmissions and for a number of PSFCH reception occasions ending in slot n, the UE provides the generated HARQ-ACK information in a PUCCH transmission within slot n+k, subject to the overlapping conditions in clause 9.2.5, where k is a number of slots indicated by a PSFCH-to-HARQ feedback timing indicator field, if present, in a DCI format indicating a slot for PUCCH transmission to report the HARQ-ACK information, or k is provided by sl-PSFCH-ToPUCCH for a transmission scheduled by a DCI format or for a SL configured grant type 2, or by sl-PSFCH-ToPUCCH-CG-Type1 for a SL configured grant type 1. k=0 corresponds to a last slot for a PUCCH transmission that would overlap with the last PSFCH reception occasion assuming that the start of the sidelink frame is same as the start of the downlink frame [4, TS 38.211]. For a PSSCH transmission by a UE that is scheduled by a DCI format, or for a SL configured grant Type 2 PSSCH transmission activated by a DCI format, the DCI format indicates to the UE that a PUCCH resource is not provided when a value of the PUCCH resource indicator field is zero and a value of PSFCH-to-HARQ feedback timing indicator field, if present, is zero. For a SL configured grant Type 2 PSSCH transmission without a corresponding PDCCH, the DCI format activating the SL configured grant Type 2 indicates to the UE that a PUCCH resource is not provided when a value of the PUCCH resource indicator field is zero and a value of PSFCH-to-HARQ feedback timing indicator field, if present, is zero. For a SL configured grant Type 1 PSSCH transmission, a PUCCH resource can be provided by sl-N1PUCCH-AN and sl-PSFCH-ToPUCCH-CG-Type1. For transmission of HARQ-ACK information corresponding only to a SL configured grant Type 2 PSSCH transmission, including the PSSCH transmission(s) associated with the corresponding activation DCI format 3_0, a UE can be provided a PUCCH resource by sl-N1PUCCH-AN-Type2. If a PUCCH resource is not provided, the UE does not transmit a PUCCH with generated HARQ-ACK information from PSFCH reception occasions. For a PUCCH transmission with HARQ-ACK information, a UE determines a PUCCH resource after determining a set of PUCCH resources from up to four PUCCH resource sets provided by sl-PUCCH-Config, for OUCI HARQ-ACK information bits, as described in clause 9.2.1. The PUCCH resource determination is based on a PUCCH resource indicator field [5, TS 38.212] in a last DCI format 3_0, excluding DCI format 3_0 for the SL configured grant Type 2 activation, among the DCI formats 3_0 that have a value of a PSFCH-to-HARQ feedback timing indicator field indicating a same slot for the PUCCH transmission, that the UE detects and for which the UE transmits corresponding HARQ-ACK information in the PUCCH where, for PUCCH resource determination, detected DCI formats are indexed in an ascending order across PDCCH monitoring occasion indexes. The PUCCH resource indicator field values map to values of a set of PUCCH resource indexes, as described in clause 9.2.3. A UE transmits a PUCCH with HARQ-ACK information using PUCCH format 0 or PUCCH format 1 or PUCCH format 2 or PUCCH format 3 or PUCCH format 4 as described in clause 9.2.3. A UE does not expect to multiplex HARQ-ACK information for more than one SL configured grants in a same PUCCH. A priority value of a PUCCH transmission with one or more sidelink HARQ-ACK information bits is the smallest priority value for the one or more HARQ-ACK information bits. In the following, the CRC for DCI format 3_0 is scrambled with a SL-RNTI or a SL-CS-RNTI. #### 16.5.2.1 Type-2 HARQ-ACK codebook in physical uplink control channel A UE determines monitoring occasions for PDCCH with DCI format 3_0 for scheduling PSSCH transmissions with associated PSFCH reception occasions on an active DL BWP of a serving cell c, as described in clause 10.1, and for which the UE transmits HARQ-ACK information in a same PUCCH in slot n based on - PSFCH-to-HARQ feedback timing indicator field values, or a value provided by sl-PSFCH-ToPUCCH-CG-Type1, for PUCCH transmission with HARQ-ACK information in slot n in response to PSFCH receptions; - time gap field in DCI format 3_0 for scheduling PSSCH transmissions with associated PSFCH receptions; - time resource assignment in DCI format 3_0 for scheduling PSSCH transmissions with associated PSFCH receptions; - a configured sidelink resource pool bitmap; - a value of a period of PSFCH resources provided in sl-PSFCH-Period; - a value of a minimum time gap provided in sl-MinTimeGapPSFCH. The set of PDCCH monitoring occasions for DCI format 3_0 for scheduling PSSCH transmissions with associated PSFCH reception occasions is defined as the PDCCH monitoring occasions in the active DL BWP of the configured serving cell, indexed in ascending order of their start times The cardinality of the set of PDCCH monitoring occasions defines a total number M of PDCCH monitoring occasions. A UE is not expected to receive a DCI format 3_0 with CRC scrambled by SL-RNTI and a DCI format 3_0 with CRC scrambled by SL-CS-RNTI for scheduling retransmission corresponding to a SL configured grant Type 1 or a sidelink configured grant Type 2 simultaneously in a same monitoring occasion. A value of a counter sidelink assignment indicator (SAI) field in DCI format 3_0, excluding DCI format 3_0 for the SL configured grant Type 2 activation, denotes an accumulative number of PDCCH monitoring occasions where PSSCH transmissions with associated PSFCH receptions are scheduled, up to a current PDCCH monitoring occasion, in ascending order of PDCCH monitoring occasion index m, where 0≤m<M. Denote by VC-SAI,mSL the value of the counter SAI in DCI format 3_0 in PDCCH monitoring occasion m according to Table 16.5.2.1-1. If the UE transmits HARQ-ACK information in a PUCCH in slot n, the UE determines the o0ACK,o1ACK,…,oOACK-1ACK, for a total number of OACK HARQ-ACK information bits, according to the following pseudo-code: Set m=0 – PDCCH with DCI format 3_0 monitoring occasion index: lower index corresponds to earlier PDCCH with DCI format 3_0 monitoring occasion. [vivo: 2208616] Reason for change: When PDCCH repetition is configured for SL DCI, it should be clarified which PDCCH candidate between the two PDCCH candidates of the PDCCH reception is used for determining the PSSCH preparation time. Summary of change: Clarify that the PDCCH candidate that ends later is used for determining the PSSCH preparation time, by applying the same clarification for PUSCH. Consequence if not approved: Ambiguity on PSSCH preparation time remains. ## 8.6 UE PSSCH preparation procedure time For sidelink dynamic grant and for SL configured grant type 2 activation, if the first sidelink symbol in the sidelink allocation for a PSSCH for a transport block and the associated PSCCH, including the DM-RS and the duplicated symbol, as defined by the slot offset KSL of the scheduling DCI for dynamic grant or the activating DCI for SL configured grant type 2, is no earlier than at symbol L, where L is defined as the next sidelink symbol with its CP starting Tproc=(N2+d2,1)(2048+144)⋅κ2-μ⋅TC after the end of the reception of the last symbol of the PDCCH carrying the DCI scheduling the sidelink transmissions for dynamic grant or activating the SL configured grant type 2, then the UE shall transmit the PSSCH and the associated PSCCH. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining L, the PDCCH candidate that ends later in time is used. - N2 is based on µ of Table 8.6-1, where µ corresponds to the one of (µDL, µSL) resulting with the largest Tproc, where the µDL corresponds to the subcarrier spacing of the downlink with which the PDCCH carrying the DCI scheduling the PSSCH for dynamic grant or activating the SL configured grant type 2 was transmitted and µSL corresponds to the subcarrier spacing of the sidelink channel with which the PSSCH and the associated PSCCH are to be transmitted, and κ is defined in Clause 4.1 of [4, TS 38.211]. - d2,1 = 1. Otherwise the UE may ignore the scheduling DCI for dynamic grant or the activating DCI for SL configured grant type 2. The value of Tproc is used both in the case of normal and extended cyclic prefix. Table 8.6-1: PSSCH preparation time PSSCH preparation time N2 [symbols]. 0; 10. 1; 12. 2; 23. 3; 36. For sidelink resource allocation mode 1, the UE does not expect that the first sidelink symbol in the sidelink allocation for a PSSCH for retransmission of a transport block and the associated PSCCH, including the DM-RS and the duplicated symbol as defined by the "Time resource assignment" field of the corresponding DCI for dynamic grant or for SL configured grant type 2, or by sl-TimeResourceCG-Type1 for configured grant type 1 starts earlier than at symbol L where L is defined as the next sidelink symbol with its CP starting Tprep+δ after the end of the last symbol of the PSFCH occasion corresponding to the most recent transmission of PSSCH for the same transport block, where Tprep is defined in Clause 16.5 of [6, TS 38.213] and δ=5∙10-4 s. Otherwise the UE may skip the retransmission of the PSSCH and the transmission of the corresponding PSCCH..

      - FL Proposals for Round 1

[TBD]

    - [ACTIVE] Issue #3-3: Correction on SL timing

      - Background

One contribution [45] proposed to have a correction for SL timing based on CR R4-2204147.

[Nokia, R1-2210039] Reason for change: CR R4-2204147 was agreed by RAN4, after which the definitions of NTA,offset values in the RAN1 and RAN4 specifications are no longer consistent in the case of gNB based synchronization. If sidelink and uplink transmissions are in the same band, NTA,offset value is not equal to zero, but the value is defined in Table 7.1.2-2 of 38.133. Summary of change: NTA,offset values for sidelink are no longer specified in 38.211 but it is stated that values can be found in 38.133. Consequence if not approved: NTA,offset values for sidelink are not consistent in the RAN1 and RAN4 specifications. < Unchanged parts omitted > ## 8.5 Timing Transmission of a sidelink radio frame number i from the UE shall start (NTA,SL+NTA,offset)∙Tc seconds before the start of the corresponding timing reference frame at the UE. The UE is not required to receive sidelink or downlink transmissions earlier than the value of NTA,offset, which is given in [12, TS 38.133], after the end of a sidelink transmission. For sidelink transmissions: If the UE has a serving cell fulfilling the S criterion according to clause 8.2 of [13, TS 38.304] - The timing of reference radio frame i equals that of downlink radio frame i in the cell with the same uplink carrier frequency as the sidelink and - NTA,offset is given by clause 4.3.1 of [TS 38.211], Otherwise The timing of reference radio frame i and NTA,offset value are given by clause 12.2.2, 12.2.3, 12.2.4 or 12.2.5 of [12, TS 38.133] - NTA,offset=0 Figure 8.5-1: Sidelink timing relation The quantity NTA,SL equals to 0..

      - FL Proposals for Round 1

[TBD]

- Editorial issues for discussion

  - IUC for mode 2 enhancement

    - [ACTIVE] Issue #2-2: Deletion of the wording of "Note” in Section 8.1.4C of TS 38.214

      - Background

Two contributions [2][32] proposed to delete the wording of "Note” in Section 8.1.4C of TS 38.214.

      - FL Proposals for Round 1

[TBD]

    - [ACTIVE] Issue #2-7: Correction for missing functions of SCI format 2-C

      - Background

Three contributions [6][23][36] proposed to have a correction for missing functions of SCI format 2-C.

      - FL Proposals for Round 1

[TBD]

    - [ACTIVE] Issue #2-8: Correction for field naming alignment for SCI format 2-C in TS 38.214

      - Background

Two contributions [7][36] proposed to have a correction for field naming alignment for SCI format 2-C in TS 38.214.

      - FL Proposals for Round 1

[TBD]

    - [ACTIVE] Issue #2-9: Correction for field naming alignment for SCI format 1-A in TS 38.213

      - Background

Three contributions [8][19][22] proposed to have a correction for field naming alignment for SCI format 1-A in TS 38.213.

      - FL Proposals for Round 1

[TBD]

    - [ACTIVE] Issue #2-17: Further clarification on the case of no HARQ-ACK information in Section 16.2.4.2 of TS 38.213

      - Background

One contribution [22] proposed to have further clarification on the case of no HARQ-ACK information in Section 16.2.4.2 of TS 38.213

      - FL Proposals for Round 1

[TBD]

    - [ACTIVE] Issue #2-11: Correction on misalignment for RRC parameters in TS 38.214

      - Background

Five contributions [10][16][23][28][36] proposed to a correction on misalignment for RRC parameters in TS 38.214.

      - FL Proposals for Round 1

[TBD]

    - [ACTIVE] Issue #2-12: Correction on misalignment for RRC parameters in TS 38.213

      - Background

Five contributions [11][15][19][22][27] proposed to have a correction on misalignment for RRC parameters in TS 38.213.

      - FL Proposals for Round 1

[TBD]

    - [ACTIVE] Issue #2-13: Correction on misalignment for RRC parameters in TS 38.212

      - Background

Five contributions [12][21][26][35][38] proposed to have a correction on misalignment for RRC parameters in TS 38.212.

      - FL Proposals for Round 1

[TBD]

- List of RAN1 agreements on IUC for Mode 2 enhancements

## 1) RAN1#103-e meeting

- Conclusion:

  - The schemes of inter-UE coordination in Mode 2 are categorized as being based on the following types of “A set of resources” sent by UE-A to UE-B:

    - UE-A sends to UE-B the set of resources preferred for UE-B’s transmission

        - e.g., based on its sensing result

    - UE-A sends to UE-B the set of resources not preferred for UE-B’s transmission

        - e.g., based on its sensing result and/or expected/potential resource conflict

    - UE-A sends to UE-B the set of resource where the resource conflict is detected

    - FFS: details of resource conflict, e.g., including type of resource conflict

    - FFS: details of sensing operation at UE-A side

    - FFS: which type(s) of resource set information is(are) beneficial/feasible to which cast type(s)

    - Note: these different types may be used in combination with each other

  - From RAN1 perspective, further study on the feasibility/benefit of inter-UE coordination is required

  - Send an LS to RAN plenary

    - Final LS in R1-2009841

- Conclusion:

  - For the schemes of inter-UE coordination identified as feasible/beneficial, at least the following aspects are further discussed.

    - How/when UE-A determines the contents of ”A set of resources”, including consideration of UL scheduling

    - When UE-A sends ”A set of resources” to UE-B, including which UE(s) sends it

    - How UE-A and UE-B are determined

    - How UE-A sends ”A set of resources” to UE-B, including container used for carrying it, implicitly or explicitly or both

    - How/when/whether UE-B receives “A set of resources” and takes it into account in the resource selection for its own transmission

    - How/whether to define the relationship between support/signaling of inter-UE coordination and cast type

## 2) RAN1#104-e meeting

- Conclusion:

  - RAN1 concludes that the inter-UE coordination in Mode 2 is feasible, and is beneficial (e.g., reliability, etc.) compared to Rel-16 Mode 2 RA, and thus recommends specification of the feature.

    - The detailed observations can be found in the attachment of the LS

- Draft LS in R1-2102165, along with the attachment R1-2102166, is approved (with a typo fix)

  - Final LS in R1-2102168

## 3) RAN1#104bis-e meeting

- Agreement:

  - Support the following schemes of inter-UE coordination in Mode 2:

    - Inter-UE Coordination Scheme 1:

      - The coordination information sent from UE-A to UE-B is the set of resources preferred and/or non-preferred for UE-B’s transmission

        - FFS details including a possibility of down-selection between the preferred resource set and the non-preferred resource set, whether or not to include any additional information other than indicating time/frequency of the resources within the set in the coordination information

      - FFS condition(s) in which Scheme 1 is used

    - Inter-UE Coordination Scheme 2:

      - The coordination information sent from UE-A to UE-B is the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI

        - FFS details including a possibility of down-selection between the expected/potential conflict and the detected resource conflict

      - FFS condition(s) in which Scheme 2 is used

- Agreement:

  - Study further to determine the conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination:

    - Details include applicable scenario(s)/inter-UE coordination scheme(s)

    - E.g., only UE(s) among the intended receiver(s) of UE-B can be a UE-A, any UE can be a UE-A, high-layer configured, etc.

      - Including the possibility of being subject to certain conditions and/or capability

- Agreement:

  - When UE-B receives the inter-UE coordination information from UE-A, consider at least one of the following options (with details FFS including possibly down-selecting/merging one or more of the options below, applicable scenario(s)/condition(s) for each option, UE behavior) for UE-B’s to take it into account in the resource (re)-selection for its own transmission

    - For scheme 1:

      - Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information

      - Option 1-2: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based only on the received coordination information

      - Option 1-3: UE-B’s resource(s) to be re-selected based on the received coordination information

      - Option 1-4: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on the received coordination information

    - For scheme 2:

      - Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information

      - Option 2-2: UE-B can determine a necessity of retransmission based on the received coordination information

## 4) RAN1#106-e meeting

- Agreement:

  - For scheme 1, the following inter-UE coordination information signalling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B.

    - Set of resources preferred for UE-B’s transmission

    - Set of resources non-preferred for UE-B’s transmission

- Agreement:

  - For scheme 2, the following inter-UE coordination information signalling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B

    - Presence of expected/potential resource conflict on the resources indicated by UE-B’s SCI

      - FFS: UE behaviour when the presence of expected/potential resource conflict is detected by the transmitter

    - FFS: Whether to additionally support the presence of detected resource conflict on the resources indicated by UE-B’s SCI

- Agreement:

  - In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by an explicit request in Mode 2:

    - A UE that sends an explicit request for inter-UE coordination information can be UE-B

    - A UE that received an explicit request from UE-B and sends inter-UE coordination information to the UE-B can be UE-A

    - (Working assumption) At least a destination UE of a TB transmitted by UE-B can be UE A

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Additional details and conditions on UE-A and UE-B

  - (Working Assumption) In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by a condition other than explicit request reception in Mode 2:

    - A UE that satisfies the condition mentioned in the main bullet and sends inter-UE coordination information is UE-A

    - A UE that received inter-UE coordination information from UE-A and uses it for resource (re-)selection is UE-B

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Additional details and conditions on UE-A and UE-B

- Agreement:

  - In scheme 2, at least the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination transmission triggered by a detection of expected/potential resource conflict(s) in Mode 2:

    - A UE that transmitted PSCCH/PSSCH with SCI indicating reserved resource(s) to be used for its transmission, received inter-UE coordination information from UE-A indicating expected/potential resource conflict(s) for the reserved resource(s), and uses it to determine resource re-selection is UE-B

    - A UE that detects expected/potential resource conflict(s) on resource(s) indicated by UE-B’s SCI sends inter-UE coordination information to UE-B, subject to satisfy one of the following conditions, is UE-A

      - (Working assumption) At least a destination UE of one of the conflicting TBs, i.e., TBs to be transmitted in the expected/potential conflicting resource(s)

        - Whether a non-destination UE of a TB transmitted by UE-B can be UE-A is (pre-)configured

      - FFS: Additional details and condition(s) on UE-A and UE-B

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Definition of expected/potential resource conflict(s) and other details (if any)

- Agreement:

  - In scheme 2, the following UE-B’s behavior in its resource (re)selection is supported when it receives inter-UE coordination information from UE-A:

    - UE-B can determine resource(s) to be re-selected based on the received coordination information

      - UE-B can reselect resource(s) reserved for its transmission when expected/potential resource conflict on the resource(s) is indicated

        - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least following UE-B’s behavior in its resource (re-)selection is supported when it receives inter-UE coordination information from UE-A:

    - For preferred resource set, the following two options are supported:

      - Option A): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

        - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set in combination with its own sensing result

          - UE-B uses in its resource (re-)selection, resource(s) not belonging to the preferred resource set when condition(s) are met

            - FFS: Details of condition(s)

          - This option is supported when UE-B performs sensing/resource exclusion

          - FFS: Other details (if any)

      - Option B): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information

        - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set

          - This option is supported at least when UE-B does not support sensing/resource exclusion

            - FFS: Whether the support is conditional or UE capability

          - FFS: Other details (if any)

      - FFS: Other option(s), and other details (if any)

    - For non-preferred resource set,

      - UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

        - UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

          - FFS: Details including

            - Whether/how UE-B can use in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set, definition of the overlap, and other details (if any)

            - When UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

        - FFS: UE-B reselects in its resource (re-)selection, resource(s) to be used for its transmission when the resource(s) are fully/partially overlapping with the non-preferred resource set

      - FFS: Other option(s), and other details (if any)

- Agreement:

  - In scheme 2, at least the following is supported to determine inter-UE coordination information:

    - Among resource(s) indicated by UE-B’s SCI, UE-A considers that expected/potential resource conflict occurs on the resource(s) satisfying at least one of the following condition(s):

      - Condition 2-A-1:

        - Other UE’s reserved resource(s) identified by UE-A are fully/partially overlapping with resource(s) indicated by UE-B’s SCI in time-and-frequency

        - FFS: Other details (if any)

        - FFS: Whether/how to specify additional criteria and other details (if any) including signaling details of conflict indication

      - (Working Assumption) Condition 2-A-2:

        - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least the following is supported to determine inter-UE coordination information of preferred resource set:

    - UE-A considers any resource(s) satisfying all the following condition(s) as set of resource(s) preferred for UE-B’s transmission

      - Condition 1-A-1:

        - Resource(s) excluding those overlapping with reserved resource(s) of other UE identified by UE-A whose RSRP measurement is larger than a RSRP threshold

          - FFS: Other details (if any)

      - FFS: Condition 1-A-2:

        - Resource(s) excluding slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B

          - FFS: Other details (if any)

      - FFS: Condition 1-A-3:

        - Resource(s) satisfying UE-B’s traffic requirement (if available)

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least the following is supported to determine inter-UE coordination information of non-preferred resource set:

    - UE-A considers any resource(s) satisfying at least one of the following condition(s) as set of resource(s) non-preferred for UE-B’s transmission

      - Condition 1-B-1:

        - Reserved resource(s) of other UE identified by UE-A from other UEs’ SCI (including priority field) and RSRP measurement

          - FFS: Other details (if any)

      - FFS: Condition 1-B-2:

        - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

## 5) RAN1#106bis-e meeting

- Agreement:

  - For Scheme 2, PSFCH format 0 is used to convey the presence of expected/potential resource conflict on reserved resource(s) indicated by UE-B’s SCI

- Agreement:

  - For Condition 2-A-1 of Scheme 2, down-select one or more of following additional criteria to determine resource(s) where expected/potential resource conflict occurs

    - Option 1: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI:

      - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations

      - Strive to reuse Rel-16 specification wherever possible

    - Option 2: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is within a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

      - FFS: Whether the threshold depends on priority

    - Option 3: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) and the other UE is within a distance threshold of UE-B as determined by both UEs’ SCIs.

    - Option 4: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

      - FFS: Whether the threshold depends on priority

    - FFS: In case of collisions of resources for two UEs having TBs with UE A as destination UE, if needed

- Working Assumption

  - For Condition 1-B-1 of Scheme 1, the following two options are supported

    - Option 1: Reserved resource(s) of other UE(s) identified by UE-A whose RSRP measurement is larger than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s)

    - Option 2: Reserved resource(s) of other UE identified by UE-A whose RSRP measurement is smaller than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s) when UE-A is a destination of a TB transmitted by the UE(s)

- Working Assumption

  - For Scheme 1 with non-preferred resource set, support following condition:

    - Condition 1-B-2:

      - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

- Agreement:

  - For Condition 1-A-1 of Scheme 1, the set of resources preferred for UE-B’s transmission is a form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4

    - When the inter-UE coordination information transmission is triggered by UE-B’s explicit request, the candidate single-slot resource(s) are determined in the same way according to Rel-16 TS 38.214 Section 8.1.4 with at least following parameters provided by signaling from UE-B. FFS whether or not to apply RSRP threshold increase in Step 7) of Rel-16 TS 38.214 Section 8.1.4.

      - Priority value to be used for PSCCH/PSSCH transmission

        - It replaces prio_TX

      - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

        - It replaces L_subCH

      - Resource reservation interval

        - It replaces P_rsvp_TX

      - FFS: Starting/ending time location of resource selection window

    - FFS : In addition to Rel-16 procedure, use inter-UE coordination information from other UEs

      - If there is no consensus in RAN1#106bis-e, no further discussions for Rel-17

- Conclusion:

  - No consensus that UE-A uses inter-UE coordination information from other UEs when it determines the preferred resource set for Condition 1-A-1 of Scheme 1.

- Working Assumption

  - For Scheme 1 with preferred resource set, support following condition:

    - Condition 1-A-2:

      - Resource(s) excluding slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

      - This can be disabled by RRC (pre-)configuration

- Agreement:

  - For allocating PSFCH resources in Scheme 2, at least following can be (pre)configured separately from those for SL HARQ-ACK feedback.

    - Set of PRBs for PSFCH transmission/reception (sl-PSFCH-RB-Set)

- Agreement:

  - For Scheme 2,

    - Index of a PSFCH resource for inter-UE coordination information transmission is determined in the same way according to Rel-16 TS 38.213 Section 16.3 with at least following modification

      - P_ID is L1-Source ID indicated by UE-B’s SCI

      - M_ID is 0

    - FFS: How to set m_CS

    - FFS: How to set m_0

    - FFS: Whether M_ID can be (pre)configured

## 6) RAN1#107-e meeting

- Agreement:

  - A resource pool level (pre-)configuration uses either of the following options

    - Option 1: PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted

      - Reuse PSSCH-to-PSFCH timing as specified in TS 38.213 Section 16.3 to determine the PSFCH occasion for resource conflict indication

      - Time gap between the PSFCH and a slot where expected/potential resource conflict occurs is larger than or equal to T_3

    - Option 2: PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI

      - UE-A transmits the PSFCH in a latest slot that includes PSFCH resources for inter-UE coordination information and is at least T_3 slots of the resource pool before the PSSCH resource indicated by UE-B’s SCI in which expected/potential resource conflict occurs

      - FFS: How to account for processing timeline

    - Note that it is possible not to configure either option1 or option 2.

- Agreement:

  - For Condition 1-A-2 of Scheme 1, the set of resources preferred for UE-B’s transmission is a form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4

    - UE-A excludes candidate single-slot candidate(s) belonging to “slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation” after Step 6) of TS 38.214 Section 8.1.4

- Agreement:

  - When PSFCH TX/RX for Scheme 2 is overlapping with LTE SL TX/RX and/or UL in a UE, reuse prioritization rule as specified in TS 38.213 Section 16.2.4.1 and 16.2.4.3.1.

- Conclusion:

  - For Scheme 2, the values of the following parameters are the same as those for SL HARQ-ACK feedback in the same resource pool

    - Period of PSFCH resources (sl-PSFCH-Period)

    - Number of cyclic shift pairs used for a PSFCH transmission that can be multiplexed in a PRB (sl-NumMuxCS-Pair)

    - Number of PSFCH resources available for multiplexing information in a PSFCH transmission (sl-PSFCH-CandidateResourceType)

- Agreement:

  - For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives:

    - Alt 1 (Working Assumption): MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

        - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used.

          - FFS: UE capability details

          - 2nd SCI is UE RX optional

    - Alt 2: MAC CE is used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

    - FFS: Whether/How to use resource reservation information as coordination information

- Working Assumption:

  - A resource pool level (pre-)configuration can enable one of the following options:

    - Option 1:

      - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

        - For the case when UE-A is a destination UE of a TB transmitted by UE-B

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI:

            - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for UE-B and other UE respectively

        - For the case when UE-A is a destination UE of a TB transmitted by another UE

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a RSRP threshold according to the priorities included in the SCI:

            - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for other UE and UE-B respectively

    - Option 4:

      - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

        - For the case when UE-A is a destination UE of a TB transmitted by UE-B

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

        - For the case when UE-A is a destination UE of a TB transmitted by another UE

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of the resource(s).

      - Support of Option 4 is subject to UE capability

    - FFS: Whether/how RSRP threshold depends on priority, MCS, overlap

- Agreement:

  - For Scheme 1 with non-preferred resource set,

    - Physical layer at UE-B excludes in its resource (re-)selection, candidate single-slot resource(s) obtained after Step 6) of Rel-16 TS 38.214 Section 8.1.4 overlapping with the non-preferred resource set

- Agreement:

  - For Condition 1-A-1 of Scheme 1, when UE-A determines the set of resources preferred for UE-B’s transmission, apply RSRP threshold increase in the same way according to Rel-16 TS 38.214 Section 8.1.4.

    - FFS: Whether/how to introduce the maximum limit of RSRP threshold increase

- Agreement:

  - For Scheme 1, at least following parameters are provided by UE-B’s request:

    - Priority value to be used for PSCCH/PSSCH transmission

    - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

    - Resource reservation interval

- Agreement:

  - For Scheme 2, when PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI,

    - Time gap between the PSFCH and SCI(s) scheduling conflicting TBs is larger than or equal to X value.

      - FFS: Details of X

- Working Assumption:

  - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, for each pair of UEs scheduling the conflicting TBs, a UE with the higher priority value is UE-B.

    - FFS whether/how to set additional condition for UE-A to send PSFCH.

    - Conclude on whether/how to handle, or differently handle, the case when at least one of UEs scheduling conflicting TBs doesn’t support Scheme 2 at the subsequent meetings

- Agreement:

  - For inter-UE coordination information triggered by an explicit request in Scheme 1,

    - UE-A uses a TX resource pool used for UE-B’s request transmission to determine the set of resources and to transmit the set of resources to UE-B

- Agreement:

  - For inter-UE coordination information triggered by a condition rather than request reception in Scheme 1,

    - UE-A transmitting in a resource pool provides inter-UE coordination information associated with the same resource pool

## 7) RAN1#107bis-e meeting

- Agreement:

  - For Scheme 1, when the inter-UE coordination information transmission is triggered by UE-B’s explicit request,

    - Starting/Ending time locations of resource selection window is provided by UE-B’s explicit request

      - Starting/Ending time locations of resource selection window is a form of combination of DFN index and slot index

- Agreement:

  - When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI, time gap between the PSFCH and SCI(s) scheduling conflicting TBs is larger than or equal to X value

    - X = sl-MinTimeGapPSFCH

  - UE does not transmit the conflict indicator or receive the conflict indicator if the timeline is not satisfied

- Agreement:

  - For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives:

    - (Working assumption) Alt1: MAC CE and 2nd SCI are used as the container of an explicit request transmission from UE-B to UE-A

      - A single format SCI 2-C is used for inter-UE coordination information and request

        - 1 bit in format 2-C is used to indicate whether the SCI is used for request to coordination information or for conveying coordination information

      - SCI 2-C is UE RX optional

      - It is up to UE implementation to additionally use 2nd SCI (for UE-B).

    - Alt2: MAC CE is used as the container of an explicit request transmission from UE-B to UE-A

- Conclusion:

  - For Scheme 2, there is no consensus to support indication of the following

    - Condition type of a resource conflict

    - Time location of a resource conflict

- Agreement:

  - For Scheme 2,

    - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer.

    - When UE-B receives a conflict indicator for resource(s) indicated by its SCI,

      - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

        - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

      - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources.

    - FFS: Whether/How the conflict in periodic transmission is indicated by UE-A and handled by UE-B

- Agreement:

  - For PSFCH TX/RX or TX/TX prioritization in Scheme 2,

    - Priority value of PSFCH TX for a resource conflict indication is the smallest priority value of the conflicting TBs

    - Priority value of PSFCH RX for a resource conflict indication is priority value indicated by UE-B’s SCI

    - For PSFCH TX/RX or TX/TX prioritization between SL HARQ-ACK feedback(s) and resource conflict indication(s), PSFCH TX/RX for SL HARQ-ACK feedback is always prioritized over PSFCH TX/RX for a resource conflict indication

- Agreement:

  - For Scheme 1, unicast is supported for an explicit request transmission for inter-UE coordination information

    - Unicast is used for the inter-UE coordination information transmission triggered by the explicit request

- Working Assumption:

  - For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception

    - Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set

      - FFS: Under which conditions groupcast/broadcast can be supported

    - Unicast

      - FFS: Under which conditions unicast can be supported

- Agreement:

  - For determining preferred resource set in Scheme 1, the value of Cresel is determined by UE-A according to Rel-16 procedure.

    - This information is not conveyed to/from UE-B

    - When inter-UE coordination information is triggered by UE-B’s request, P_rsvp_TX used for determining SL_RESOURCE_RESELECTION_COUNTER according to Rel-16 procedure is provided by resource reservation interval indicated by UE-B’s request

- Agreement:

  - For the indication of resource set in Scheme 1, the value of Sl-MaxNumPerReserve is fixed to 3.

- Agreement:

  - The following working assumption is confirmed with modification in RED.

    - MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

        - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used.

          - FFS: UE capability details

          - 2nd SCI is UE RX optional

          - The field size of the indication of resource set in a SCI format 2-C is determined by [N=3]

- Agreement:

  - For inter-UE coordination information transmission in Scheme 1,

    - Inter-UE coordination information can be multiplexed with other data only if the source/destination ID pair is the same

      - Retransmission of the TB carrying inter-UE coordination information is supported

  - For explicit request transmission in Scheme 1,

    - Explicit request can be multiplexed with other data only if the source/destination ID pair is the same

      - Retransmission of the TB carrying request is supported

- Agreement:

  - For inter-UE coordination triggered by an explicit request in Scheme 1, whether or not to transmit the inter-UE coordination information upon the request reception is determined by UE-A’s implementation subject to the following procedures.

    - Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control

- Agreement:

  - For inter-UE coordination triggered by a condition rather than request reception in Scheme 1,

    - A resource pool level (pre-)configuration can enable one of the following alternatives:

      - Alt 1: it is up to UE-A’s implementation whether or not to trigger the inter-UE coordination information generation.

      - Alt 2: the inter-UE coordination information generation can be triggered only when UE-A has data to be transmitted together with the inter-UE coordination information to UE-B

    - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the inter-UE coordination information triggered by a condition.

- Agreement:

  - For inter-UE coordination triggered by UE-B’s explicit request in Scheme 1,

    - A resource pool level (pre-)configuration can enable one of the following alternatives:

      - Alt 1: it is up to UE-B’s implementation whether or not to trigger the request generation

      - Alt 2: the request generation can be triggered only when UE-B has data to be transmitted to UE-A

    - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the request transmission.

- Agreement:

  - For Scheme 1 with preferred resource set Option A,

    - MAC layer selects resources using S_A and the received preferred resource set

      - MAC layer firstly selects resources for transmissions within the intersection of S_A and the preferred resource set until it becomes impossible to select a resource within the intersection under the constraint defined in Rel-16.

        - It is up to the UE whether to use the preferred resource set from SCI format 2-C and/or MAC CE

      - After this, if the number of selected resources is smaller than the required number of transmissions for a TB, MAC layer selects resources for the remaining transmissions outside the intersection but inside S_A under the constraint defined in Rel-16.

- Agreement:

  - For Scheme 1 with preferred resource set Option B,

    - MAC layer selects resources belonging to the received preferred resource set under the constraint defined in Rel-16

      - It is up to the UE whether to use the preferred resource set from SCI format 2-C and/or MAC CE

- Agreement:

  - For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as indicated by UE-B’s explicit request.

    - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data

- Agreement:

  - For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of explicit request is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as that of a TB to be transmitted by UE-B.

    - For the case when the explicit request is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the explicit request and data

- Agreement:

  - For inter-UE coordination information triggered by a condition other than explicit request reception in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration.

    - FFS: Otherwise, the priority value is determined by UE-A’s implementation.

    - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data

- Agreement:

  - For sidelink transmission carrying inter-UE coordination information in Scheme 1,

    - UE-A performs its resource (re)selection according to the same procedure in TS 38.214 Section 8.1.4 to transmit the inter-UE coordination information to UE-B.

  - For sidelink transmission carrying request in Scheme 1,

    - UE-B performs its resource (re)selection according to the same procedure in TS 38.214 Section 8.1.4 to transmit the request for the inter-UE coordination information to UE-A if UE-B performs sensing/resource exclusion. Otherwise, at least UE-B can perform random selection

  - Note: RAN1 does not pursue specific enhancement of Rel-17 resource (re)selection for the transmission of inter-UE coordination information and its request.

- Working assumption:

  - First resource location of each TRIV is a slot offset with respect to a reference slot

    - Alt 2:

      - The slot offset is the number of logical slots from the reference slot

        - The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to [256]

          - FFS: The detailed value range including granularity

        - Slot offset for each TRIV to indicate the set of resources is separately indicated by inter-UE coordination information

    - For the reference slot,

      - The reference slot is the slot indicated by the inter-UE coordination information in a form of combination of DFN index and slot index

- Agreement:

  - For determining preferred resource set in Scheme 1, when inter-UE coordination information transmission is triggered by a condition other than explicit request reception,

    - Values of following parameters are (pre)configured for a resource pool. If there is no (pre)configuration, UE-A determines by its implementation the values of the following parameters

      - prio_TX

      - L_subCH

      - P_rsvp_TX

    - UE-A determines by its implementation values of following parameters

      - n+T_1, n+T_2

    - FFS: Whether/how to support (pre)configuration of n+T_1 and n+T_2

    - Note that it is up to RAN2 decision whether/how the values of these parameters are provided by PC5-RRC signaling from UE-B to UE-A and UE-A uses the received information to determine the preferred resource set

- Agreement:

  - For inter-UE coordination information is triggered by UE-B’s request,

    - A resource pool level (pre-)configuration can enable one of the following alternatives:

      - Alt 1:

        - Resource set type to be provided by inter-UE coordination information transmission is determined by UE-A’s implementation and its information is indicated by UE-A’s inter-UE coordination information

          - UE-A’s inter-UE coordination information indicates either preferred resource set or non-preferred resource set

      - Alt 2:

        - Resource set type to be provided by inter-UE coordination information transmission is indicated by UE-B’s request

          - UE-B’s request indicates either preferred resource set or non-preferred resource set

    - Note that it is up to RAN2 decision whether/how UE-B provides its support of sensing/resource exclusion to UE-A via PC5-RRC signaling and UE-A uses the received information to determine the type of resource set to be transmitted to UE-B

- Agreement:

  - For inter-UE coordination information is triggered by a condition other than explicit request reception,

    - Resource set type to be provided by inter-UE coordination information transmission is determined by UE-A’s implementation and its information is indicated by UE-A’s inter-UE coordination information

      - UE-A’s inter-UE coordination information indicates either preferred resource set or non-preferred resource set

- Working assumption:

  - For Scheme 2, (pre)configuration is supported to enable or disable that 1 LSB of reserved bits of a SCI format 1-A is used to indicate of whether UE scheduling a conflict TB can be UE-B or not.

    - FFS: UE-A's behavior for the case when at least one of UEs scheduling conflicting TBs is not capable of receiving the conflict indication

## 8) RAN1#108-e meeting

- Agreement:

  - For a slot offset that is (pre)configured to indicate the first resource location of each TRIV with respect to a reference slot,

    - Granularity of the slot offset is 1 logical slot

    - (Pre)configured maximum value of the slot offset is up to 8000

      - When both SCI format 2-C and MAC CE are used as the container of inter-UE coordination information, the maximum value of the slot offset is 255

      - When MAC CE only is used as the container of inter-UE coordination information, the maximum value of the slot offset is the (pre)configured maximum value

- Agreement:

  - A SCI format 2-C includes all the fields present in SCI format 2-A except cast type indicator

- Conclusion:

  - For cast type(s) of inter-UE coordination information with preferred resource set triggered by a condition other than explicit request reception, there is no consensus in RAN1 on the support of groupcast or broadcast for preferred resource set

- Agreement

  - For Scheme 2, m_CS for a resource conflict indication for the next reserved resource indicated by the corresponding UE-B’s SCI for either current TB transmission or next TB transmission is 0

- Agreement

  - For Scheme 2, when UE-B receives a conflict indicator for resource(s) indicated by its SCI, it up to UE-B’s implementation whether/how to set the reservation periodicity in the re-selected resource.

- Agreement

  - For Scheme 2,

    - m_0 for a resource conflict indication is derived in the same way as specified for HARQ-ACK information in TS 38.213 Section 16.3

    - A UE expects that different PRBs are (pre)configured between conflict indication and HARQ-ACK information

- Agreement:

  - For Scheme 1, when both SCI format 2-C and MAC CE are used as the container of an explicit request for inter-UE coordination information, the same bit field size for the request in a SCI format 2-C is applied to MAC CE

- Agreement:

  - For Scheme 1, when MAC CE only is used as the container of an explicit request for inter-UE coordination information, the same bit field size for the request in a SCI format 2-C is applied to MAC CE

- Conclusion:

  - For inter-UE coordination operation in Rel-17, RAN1 understands that only UE(s) in mode 2 can be UE-A

    - Note that RAN1 does not pursue specific enhancement of Rel-17 inter-UE coordination operation for handling the case where UE(s) in mode 1 can be UE-A

- Agreement

  - Confirm the following working assumption with modification in RED

- Working assumption made in RAN1#107bis-e: - First resource location of each TRIV is a slot offset with respect to a reference slot - Alt 2: - The slot offset is the number of logical slots from the reference slot - The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to [8000256] - FFS: The detailed value range including granularity - Slot offset for each TRIV except for first TRIV to indicate the set of resources is separately indicated by inter-UE coordination information - Slot offset for first TRIV is 0 - For the reference slot, - The reference slot is the slot indicated by the inter-UE coordination information in a form of combination of DFN index and slot index

- Agreement

  - MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

    - For the indication of resource set, the following is supported:

      - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

        - First resource location of each TRIV is separately indicated by the inter-UE coordination information

      - If N <= 2, MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If N > 2, only MAC CE is used.

        - FFS: UE capability details

        - 2nd SCI is UE RX optional

        - The field size of the indication of resource set in a SCI format 2-C is determined by N=2

- Agreement

  - For Scheme 1, each bit field size of a SCI format 2-C for inter-UE coordination information is given by following table:

    - Note that lowest subchannel index for the first resource location of each TRIV is separately indicated by inter-UE coordination information

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Resource combination(s) | 2*log2(N subchannel SLNsubchannel SL + 12Nsubchannel SL + 16)+9+Y Where N subchannel SL is provided by the higher layer parameter sl-NumSubchannel, Y=log2Nrsv_period with that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; Y=0 otherwise. |
| First resource location(s) | 8 |
| Reference slot location | 10+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 |
| Lowest subchannel indices for the first resource location of each TRIV | 2*log2NsubchannelSL where NsubchannelSL is provided by the higher layer parameter sl-NumSubchannel |
| (FFS) Actual number of resource combination | 1 Note: Support of this field is to be concluded by Feb 28. |

- Agreement

  - For Scheme 1, each bit field size of a SCI format 2-C for an explicit request for inter-UE coordination information is given by following table:

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Priority | 3 |
| Number of subchannels | log2(N subChannel SL) Where N subChannel SL is provided by the higher layer parameter sl-NumSubchannel |
| Resource reservation period | Y Where Y=log2Nrsv_periodwith that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResoure is configured; Y=0 otherwise. |
| Resource selection window location | 210+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 bit if determineResourceSetTypeScheme1 is set to ‘UE-B’s request’, otherwise, 0 bit |

  - This agreement does not imply that new field requested by RAN2 cannot be further added.

- Agreement

  - For Scheme 1, when MAC CE only is used as the container of inter-UE coordination information, each bit field size for inter-UE coordination information is given by following table from RAN1’s perspective, and RAN1 understands that the maximum value of N resource combinations to be conveyed in inter-UE coordination information is bounded so that the total payload size of inter-UE coordination information leads not to exceed the size of TB including the MAC CE

    - Details (e.g., whether/how to separately indicate the value of N in the inter-UE coordination information, how to put the following fields into MAC CE and the related field sizes in MAC CE) are up to RAN2

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Resource combination(s) | N*log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16)+9+Y Where N subChannel SL is provided by the higher layer parameter sl-NumSubchannel, Y=log2Nrsv_periodwith that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResoure is configured; Y=0 otherwise. |
| First resource location(s) | N-1*log2(X) Where X is provided by the (pre)configured maximum value of slot offset for the case when MAC CE only is used as a container of inter-UE coordination information |
| Reference slot location | 10+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 |
| Lowest subchannel indices for the first resource location of each TRIV | N*log2NsubchannelSL Where NsubchannelSL is provided by the higher layer parameter sl-NumSubchannel. |

- Conclusion:

  - There is no consensus in RAN1 on indicating actual number of resource combination in a SCI format 2-C for inter-UE coordination information.

    - Note: Different resource combinations can indicate the same set of resources for the case when only one resource combination is actually used

- Agreement

  - For Scheme 2,

    - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer.

    - When UE-B receives a conflict indicator for resource(s) indicated by its SCI,

      - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

        - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

      - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources.

- Agreement

  - Confirm the following working assumption with modification in RED. Note that the terminology of “indicationUEB flag” means the indication of whether UE scheduling a conflict TB can be UE-B or not.

    - Working Assumption:

      - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A,

        - for each pair of UEs scheduling the conflicting TBs whose PSFCH occasions for resource conflict indication are not yet passed and indicationUEB flag is set to 1 if the higher parameter of indicationUEBScheme2 is (pre)configured to ‘Enabled’, a UE with the higher priority value is UE-B. When the UEs in the pair have the same priority value, UE-A determines one of the UEs to be UE-B by its implementation.

          - UE-A considers the SCIs received earlier than or equal to sl-MinTimeGapPSFCH before the PSFCH occasion for conflict indication when determining UE-B.

- Agreement

  - A UE performs PSFCH TX/RX or TX/TX prioritization between SL HARQ-ACK feedback(s) and resource conflict indication(s) first, and then the UE performs prioritization between prioritized PSFCH TX(s) or RX(s) and LTE SL TX/RX or UL by reusing prioritization rule as specified in TS 38.213 Section 16.2.4.1 and 16.2.4.3.1.

- Conclusion:

  - RAN1 does not pursue specific enhancement of Rel-17 inter-UE coordination operation for handling the overlapping between UL with SL-HARQ-ACK information and PSFCH for a conflict indication, i.e., there is no case in Rel-17 where the overlapping between UL with SL-HARQ-ACK information and PSFCH for a conflict indication occur at a UE performing inter-UE coordination operation

- Conclusion:

  - There is no consensus in RAN1 to further introduce enhancement in Rel-17 on Mode 2 resource selection procedure to ensure the timeline (i.e., minimum time gap between PSFCH and a slot where a SCI is transmitted of sl-MinTimeGapPSFCH, minimum time gap between PSFCH and a slot where expected/potential resource conflict occurs on PSSCH resource indicated by a SCI of T_3) for a conflict indication.

- Agreement

  - For Scheme 1, when both SCI format 2-C and MAC CE are used as the container of inter-UE coordination information, the same inter-UE coordination information is indicated in the SCI format 2-C and the MAC CE

    - Details (e.g., how to put the fields of SCI format 2C for inter-UE coordination information into MAC CE and the related field sizes in MAC CE) are up to RAN2

- Conclusion:

  - When PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted,

    - if there is a PSFCH occasion satisfying “the minimum time gap (sl-MinTimeGapPSFCH) between the PSFCH occasion and a slot where the SCI is transmitted” but not satisfying “the minimum time gap (T_3) between the PSFCH occasion and a slot of the earliest reserved PSSCH resource indicated by the corresponding SCI after the PSFCH occasion”,

      - the PSFCH occasion cannot be used by UE-A for a conflict indication for reserved PSSCH resource other than the earliest reserved PSSCH resource indicated by the corresponding SCI after the PSFCH occasion

- Agreement

  - (Pre)configuration of parameters related to n+T_1 and n+T_2 for determining the set of preferred resources in inter-UE coordination information triggered by a condition other than explicit request reception is not supported.

    - Note that T_2 is no smaller than T_2,min and 0 <= T_1 <= Tproc,1 as specified in TS 38.214 section 8.1.4.

- Agreement

  - For inter-UE coordination information transmission, only when the cast type of inter-UE coordination information is unicast regardless of whether or not it is multiplexed with other data, a SCI format 2-C can be used in addition to MAC CE

- Agreement

  - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the same UE-A

    - It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

  - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A

    - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e

  - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A

    - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

- Agreement

  - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the different UE-As,

    - UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set.

  - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the different UE-As.

    - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e (except for the processing timeline)

  - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As,

    - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

- Agreement

  - Notations:

    - (n+T_1) – Start slot of resource selection window for determining the set of resources

      - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_1) is provided by UE-B’s request as per the existing agreement

      - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_1) is determined by UE-A’s implementation as per the existing agreement

    - (n+T_2) – End slot of resource selection window for determining the set of resources

      - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_2) is provided by UE-B’s request as per the existing agreement

      - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_2) is determined by UE-A’s implementation as per the existing agreement

    - (n’+T’_1) – Start slot of resource selection window used for sidelink transmission carrying inter-UE coordination information

    - (n’+T’_2) – End slot of resource selection window used for sidelink transmission carrying inter-UE coordination information

    - n' is the slot where UE procedure of determining TX resources of sidelink transmission carrying inter-UE coordination information is triggered

  - For inter-UE coordination information triggered by UE-B’s explicit request

    - Alt 1-1:

      - X1 ≤ (n’+T’_1)

      - (n’+T’_2) ≤ X2

  - For inter-UE coordination information triggered by a condition other than explicit request reception,

    - Alt 2-2:

      - (n’+T’_2) < X3

  - FFS: Values for X1, X2, X3

- Agreement

  - For sensing window for determining the set of resources in Scheme 1,

    - Notations:

      - For inter-UE coordination information triggered by UE-B’s explicit request, the values of (n+T_1) and (n+T_2) are provided by the request as per the existing agreement.

      - For inter-UE coordination information triggered by a condition other than explicit request reception, the values of (n+T_1) and (n+T_2) are determined by UE-A’s implementation as per the existing agreement.

      - T’’_1 is up to UE-A’s implementation under 0 <= T’’_1 <= Tproc,1

      - (n’+T’_1) – Start slot of resource selection window used for sidelink transmission carrying inter-UE coordination information

      - n' is the slot where UE procedure of determining TX resources of inter-UE coordination information is triggered

    - Alt 1:

      - No further change is supported. Note that the sensing window for determining the set of resources is already derived based on the location (n+T_1) and (n+T_2) used for determining the set of resources in TS38.214 section 8.1.4, i.e., sensing window is defined by the range of slots [(n+T_1) – T_0 – T’’_1, (n+T_1) – T_proc,0 – T’’_1].

- Agreement

  - For the case when it is not possible that the number of candidate single-slot resources after applying the received non-preferred resource set as per the existing agreement meets the requirement of X*M_total in step 7),

    - It is up to UE-B’s implementation whether to take the received non-preferred resource set in its resource selection after step 6) to meet this requirement

## 8) RAN1#109-e meeting

- Agreement:

  - TP#1 (for TS38.213 v17.1.0, clause 16.4) in section 8.1 of R1-2203717 is endorsed.

  - TP#2 (for TS38.213 v17.1.0, clause 16.2.3) in section 8.2 of R1-2203717 is endorsed.

- Agreement:

  - When UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A or different UE-As, when UE-B has own sensing results

    - No RAN1 specification change to TS 38.214 is deemed necessary on how it uses the non-preferred resource set in its resource (re)selection

    - It is up to UE-B implementation to use the preferred resource set in its resource (re)selection for transmissions to the UE A providing the preferred resource set

- Agreement:

  - Remove the sentence below in Section 8.1.5A of TS 38.214

[When a preferred resource set is indicated by an SCI format 2-C, if the transmission of the set was triggered by an explicit request, the value of the resource reservation interval P_(rsvp,m) is set to 0.]

- Agreement:

  - X1, X2, and X3 are determined by UE-A’s implementation under the constraints defined in the specification (e.g., SL-LatencyBoundIUC-Report-r17, requirement of T_2min)

    - UE-B can choose to not use any resource from the preferred/non-preferred resource set in its resource (re-)selection if that resource is earlier than (Tproc,0+Tproc,1+Tproc,2) after the resource of inter-UE coordination information transmission

      - For Tproc,2,

        - When only MAC CE is used for inter-UE coordination information transmission, it is equal to (Tproc,0+Tproc,1)

        - When MAC CE and SCI format 2-C are both used for inter-UE coordination information transmission, it is equal to Tproc,0

          - Note: this is assuming that SCI format 2-C is received

    - Whether or not to make the time gap from the resource of inter-UE coordination information transmission to preferred/non-preferred resource in the inter-UE coordination information larger than (Tproc,0+Tproc,1+Tproc,2) is up to UE-A implementation

- Agreement:

  - Adopt the following text proposal to TS 38.214 v17.1.0:

    - Reason for change:

      - RAN1 discussed how to alleviate an ambiguity of UE-B’s behavior in terms of handling the field of resource reservation period in the inter-UE coordination information considering that there is no special mechanism that makes UE-B distinguishable whether the inter-UE coordination information is triggered by an explicit request or triggered by a condition, and agreed that this field is always valid.

    - Summary of change:

      - Delete the sentence of “[When a preferred resource set is indicated by an SCI format 2-C, if the transmission of the set was triggered by an explicit request, the value of the resource reservation interval P_(rsvp,m) is set to 0.]” in Section 8.1.5A of TS 38.214.

    - Consequences if not approved:

      - The specification is not clear regarding UE-B’s behavior on handling the field of resource reservation period in the inter-UE coordination information.

----------------------- Start of text proposal to TS 38.214 v17.1.0 ----------- 8.1.5A UE procedure for determining slots and resource blocks indicated by a preferred or non-preferred resource set The set of slots and resource blocks indicated by a set of preferred or non-preferred resource(s) is determined as described below. The set of preferred or non-preferred resources r0,r1,r2,…, is indicated by a reference slot tref and M tuples (TRIVm,FRIVm,Prsvp,m), 1≤m≤M indicated by the 'resource combination(s)' field, where for each tuple TRIVm is indicated by the 9 MSBs, followed by FRIVm and Prsvp,m (if present). The reference slot tref is indicated by the 'reference slot location' field as a combination of DFN index and slot index [5, TS 38.212], with the 10 MSBs indicating the DFN index. TRIVm and FRIVm are interpreted according to clause 8.1.5, with the following modifications: - - the value of sl-MaxNumPerReserve is fixed to 3. - - "slot where SCI format 1-A was received" is replaced by slot indicated as the first resource location of a TRIVm. - - the first resource location of each TRIVm for m>1 is indicated by a slot offset tm in logical slots with respect to the reference slot tref; the first resource location of TRIV1 is at slot offset 0 with respect to the reference slot.- "the received SCI format 1-A, except the resource in the slot where SCI format 1-A was received" is replaced by "each tuple" - - the starting sub-channel nsubCH,0start of the first resource of each tuple is separately indicated. The starting sub-channel nsubCH,0start of the first resource of each tuple is indicated by the '[lowest subchannel index for the first resource location of each TRIV]' field. The resource reservation period Prsvp,m is encoded as in SCI format 1-A. If the set is indicated by an SCI format 2-C, the number of tuples is M=2. A UE forms the union of the subsets indicated by each tuple (TRIVm,FRIVm,Prsvp,m) to obtain the set r0,r1,r2,…. Prsvp,m ------------------------ End of Text proposal to TS 38.214 v17.1.0 ----------

## 8) RAN1#110 meeting

- Agreement

  - Reason for change:

    - The RAN1 agreement related to UE-B's behaviour of determining a resource used for its resource (re-)selection among the non-preferred resource set received from UE-A is not captured in the specification.

  - Summary of change:

    - The relevant UE-B's behaviour is described in Section 8.1.4C of TS 38.214.

  - Consequences if not approved:

    - The specification text is unclear as to which resource UE-B uses for its resource (re-)selection among the non-preferred resource set received from UE-A

--------------------------- Start of text proposal to TS 38.214 v17.2.0 --------------------------- 8.1.4C UE procedure for using a received non-preferred resource set A UE configured with the higher layer parameter interUECoordinationScheme1 uses a received non-preferred resource set as follows when performing resource (re-)selection: - the UE excludes in Step 6b) of clause 8.1.4 resource(s) overlapping with the non-preferred resource set. Note: If it is not possible to meet the requirement that the number of candidate single-slot resources remaining in the set SA be at least X⋅Mtotal after excluding resource(s) overlapping with the received non-preferred resource set, it is up to UE implementation whether or not to take into account the received non-preferred resource set to meet such requirement. Note: The UE is not required to use any resource from the non-preferred resource set in its resource (re-)selection if that resource is earlier than (Tproc,0+Tproc,1+Tproc,2) after the resource of inter-UE coordination information transmission, where Tproc,2 is equal to (Tproc,0+Tproc,1) when only MAC CE is used for inter-UE coordination information transmission, or Tproc,2 is equal to Tproc,0 when MAC CE and SCI format 2-C are both used for inter-UE coordination information transmission. Note: the case when Tproc,2 is equal to Tproc,0 is assuming that SCI format 2-C is received. --------------------------- End of Text proposal to TS 38.214 v17.2.0 ---------------------------

- Agreement

  - Reason for change

    - Regarding the cast type for SCI format 2-C, the following agreement was reached. However, it seems that there is no corresponding text in the specification.

Agreement For inter-UE coordination information transmission, only when the cast type of inter-UE coordination information is unicast regardless of whether or not it is multiplexed with other data, a SCI format 2-C can be used in addition to MAC CE

  - Summary of change

    - Clarify that SCI format 2-C can be used only for unicast.

  - Consequences if not approved

    - The cast type for which SCI format 2-C was agreed is not explicitly reflected in the specification.

--------------------------- Start of text proposal to TS 38.212 v17.2.0 --------------------------- 8.4.1.3 SCI format 2-C SCI format 2-C is used for the decoding of PSSCH, and providing inter-UE coordination information or requesting inter-UE coordination information. SCI format 2-C can be used only for unicast. The following information is transmitted by means of the SCI format 2-C: <Unchanged parts omitted> --------------------------- End of Text proposal to TS 38.212 v17.2.0 ---------------------------

- Reference

- R1-2208385	Discussion on correction for inter-UE coordination Scheme 2 determination of UE-B	FUTUREWEI

- R1-2208386	Discussion on resolving ambiguous text in 38.214	FUTUREWEI

- R1-2208609	Clarification on missing field descriptions of SCI 2-C	vivo

- R1-2208611	Clarification for inter-UE coordination scheme-2	vivo

- R1-2208612	Clarification on IUC related transmission based on latency bound	vivo

- R1-2208613	Corrections for missing functions of SCI 2-c	vivo

- R1-2208716	Field naming alignment for SCI format 2-C in TS38.214	ZTE, Sanechips

- R1-2208717	Clarification on Condition 2-A-1 for scheme 2	ZTE, Sanechips

- R1-2208718	Clarification on Condition 2-A-2 for scheme 2	ZTE, Sanechips

- R1-2208719	Corrections on misalignment for RRC parameters in TS 38.214	ZTE, Sanechips

- R1-2208720	Corrections on misalignment for RRC parameters in TS 38.213	ZTE, Sanechips

- R1-2208721	Corrections on misalignment for RRC parameters in TS 38.212	ZTE, Sanechips

- R1-2208920	Correction on resource exclusion behavior with non-preferred resource set	CATT, GOHIGH

- R1-2208921	Discussion on resource exclusion behavior with non-preferred resource set	CATT, GOHIGH

- R1-2209135	Draft CR on RRC parameter name and value misalignment in TS 38.213	NEC

- R1-2209136	Draft CR on RRC parameter name and value misalignment in TS 38.214	NEC

- R1-2209564	Correction on determining UE-B among UEs scheduling conflicting TBs	Apple

- R1-2209565	Correction on priority value of PSFCH transmission with conflict information for condition 2-A-2	Apple

- R1-2209682	Correction on handling of conflict information receiver flag	Sharp

- R1-2209683	Remaining issues on NR sidelink enhancement	Sharp

- R1-2209798	Draft CR on Inter-UE coordination in TS 38.212	ASUSTeK

- R1-2209799	Draft CR on Inter-UE coordination in TS 38.213	ASUSTeK

- R1-2209801	Draft CR on Inter-UE coordination in TS 38.214	ASUSTeK

- R1-2209830	Correction on description of valid PSFCH occasion for scheme 2 in TS 38.213	Huawei, HiSilicon

- R1-2209873	Draft CR on condition to be UE-A for SL IUC scheme 2	NTT DOCOMO, INC.

- R1-2209878	Editorial corrections for SL IUC (38.212)	NTT DOCOMO, INC.

- R1-2209879	Editorial corrections for SL IUC (38.213)	NTT DOCOMO, INC.

- R1-2209880	Editorial corrections for SL IUC (38.214)	NTT DOCOMO, INC.

- R1-2209881	Discussion on RAN2-related topics for SL maintenance	NTT DOCOMO, INC.

- R1-2209950	Draft CR on Non-preferred Resources	Qualcomm Incorporated

- R1-2209952	Discussion on Corrections to NR Sidelink	Qualcomm Incorporated

- R1-2210060	Draft CR on the Notes in section 8.1.4C of 38.214	Nokia, Nokia Shanghai Bell

- R1-2210124	[Draft] Clarification on valid PSFCH occassions for resource conflict information	Ericsson

- R1-2210128	[Draft] Correction to priority definition for Tx and Rx of PSFCH with conflict information	Ericsson

- R1-2210184	Corrections for SL Inter-UE coordination	Nokia, Nokia Shanghai Bell

- R1-2210185	Corrections for SL Inter-UE coordination	Nokia, Nokia Shanghai Bell

- R1-2210203	Correction on UE-A is the destination UE of a TB transmitted by UE-B in TS 38.214	Huawei, HiSilicon

- R1-2210204	Correction on RRC parameter names and values used for SCI format 1-A and 2-C in TS 38.212	Huawei, HiSilicon

- R1-2208614	Discussion on PDCCH repetition for sidelink	vivo

- R1-2208615	Clarification on PDCCH repetition for sidelink-38.213	vivo

- R1-2208616	Clarification on PDCCH repetition for sidelink-38.214	vivo

- R1-2209953	Draft CR on Power Control Parameters	Qualcomm Incorporated

- R1-2210130	[Draft] Modifications on SL open loop power control formulae	Ericsson

- R1-2209829	Correction on power control for PSCCH/PSSCH/PSFCH/S-SSB in TS 38.213	Huawei, HiSilicon

- R1-2210039	Correction on SL timing	Nokia, Nokia Shanghai Bell