3GPP TSG RAN WG1 #110bis-e			     R1-2210345

e-Meeting, October 10th – 19th, 2022

Source:	Moderator (NTT DOCOMO, INC.)

Title:	Summary #2 on 9.11.1 Coverage enhancement for NR NTN

Agenda Item:	9.11.1

Document for: 	Discussion and Decision

- Introduction

[110bis-e-R18-NTN-01] Email discussion on coverage enhancement for NR NTN by October 19 – Shohei (Docomo) - Check points: October 14, October 19

This document is made for discussion on coverage enhancement for NR NTN. Schedule for discussion is below in UTC time. FL requests companies to consider the schedule (but would be updated, especially gray-highlighted part).

- 1st email discussion: Monday 9:00 – Tuesday 23:59

- 1st GTW: Wednesday

- 2nd email discussion: Wednesday 16:00 – Thursday 23:59

- 1st check point: Friday 16:59

- Quiet period: Friday 23:59 – Sunday 23:59

- 3rd email discussion: Friday 9:30 – Monday 23:59

- 2nd GTW: Tuesday

- 2nd check point: Wednesday 16:59

This topic is mentioned in Rel-18 NR NTN WID as captured in Appendix-1. As discussed and concluded at the last RAN plenary meeting, we focus on coverage enhancement of PUCCH for Msg4 HARQ-ACK and discussion of DMRS-bundling for PUSCH. Although FL found that several companies propose other mechanisms in their contributions as summarized in section 5.4, they will not be handled since not aligned with the WID description.

In this meeting, FL’s plan is to agree at least the following aspects.

- For PUCCH of Msg4 HARQ-ACK

  - High-level concept

  - What should be discussed and (if possible) potential options

- For PUSCH DMRS-bundling

  - Necessity of RAN1 enhancement

  - (if necessary) high-level concept and (if possible) what should be discussed

In addition, ‘contact information’ in the last section is copied from the summary at the last meeting. Anyone can use/add/update/remove some of the list if necessary.

- Collections of agreements/conclusions in RAN1#110-bis

Agreement

For PUCCH for Msg4 HARQ-ACK,

- Support PUCCH repetition

  - Further discuss the specification impact for at least the following

    - Procedure and signaling (e.g., cell-specific configuration, request to gNB and dynamic indication from gNB, UE capability indication before Msg4, etc.)

    - Repetition factor

    - Repetition slot counting for FDD

  - Further study whether to enhance or support the following

    - Frequency hopping

    - DMRS bundling

Agreement

For PUCCH repetition for Msg4 HARQ-ACK,

- Discuss the following options of procedure to perform repetitions

  - Option 1: UE always performs repetition if configured in cell-specific manner

    - FFS: details of cell-specific configuration

    - FFS: behavior of UE being incapable of repetition

  - Option 2: UE requests repetition and is dynamically indicated to perform repetition

    - FFS: details of repetition request

    - FFS: details of configuration and dynamic repetition indication

  - Option 3: UE indicates repetition capability and is dynamically indicated to perform repetition

  - How UE indicates repetition capability before Msg4

Conclusion

For PUCCH repetition for Msg4 HARQ-ACK,

- The existing mechanism on repetition slot counting (as in section 9.2.6 of TS 38.213) can be applied.

  - FFS: whether specification update to apply the existing mechanism to PUCCH repetition for Msg4 HARQ-ACK is needed.

- Proposals for agreements/conclusions

Proposal 2-1_v3

For PUSCH DMRS bundling,

- Discuss further the need of enhancement in consideration of at least the following:

  - Phase difference due to timing drift and/or doppler shift.

    - e.g., whether/how long a UE can meet phase continuity requirement specified as Table 6.4.2.5-1 in 38.101-1 in consideration of frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-5 and timing error specified in Table 7.1C.2-1 of 38.133, whether RAN1 should introduce enhancement to meet the requirement or RAN4 should update the requirement or UE should pre-compensate phase difference by UE implementation, etc.

  - An event which causes power consistency and phase continuity not to be maintained.

    - e.g., whether the new event (e.g., corresponding to UL time/frequency pre-compensation, antenna switching) is necessary to determine actual TDW(s) from each nominal TDW or the existing specification can work without any specification change, etc.

If time is allowed,

Proposal 1-4_v10

For PUCCH transmission for Msg4 HARQ-ACK,

- Supported number of transmissions/repetitions are 1, 2, 4, 8.

  - Note: single-slot PUCCH transmission is performed as in the existing specification.

  - FFS: whether larger repetition factor number of transmissions is supported

  - FFS: whether/how single-slot PUCCH transmission is can be explicitly configured and/or indicated

- Discussion

Although companies are showing their kind evaluation results, FL noticed that different CNR values are used among companies. This would not be good to have same view on each observation. FL asks companies to use the following values for future evaluations. This table (other than UE antenna gain part) was made according to companies’ inputs at the last meeting and was used to make the agreed observations.

| Case | Satellite orbit | Satellite parameter set | Elevation angle [degree] | Frequency [GHz] | UE antenna gain [dBi] | TX: EIRP [dBm] | RX: G/T [dB/T] | No. of PRBs | Bandwidth [MHz] | Free space path loss [dB] | Atmospheric loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | Additional losses [dB] | CNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | LEO-1200 | 1 | 30 | 2.0 | -5.5 | 18.0 | 1.1 | 1 | 0.18 | 164.5 | 0.1 | 3.0 | 2.2 | 3.0 | 0.0 | -8.1 |
|  |  |  |  |  |  |  |  | 2 | 0.36 |  |  |  |  |  |  | -11.1 |
|  |  |  |  |  |  |  |  | 3 | 0.54 |  |  |  |  |  |  | -12.9 |
|  |  |  |  |  |  |  |  | 4 | 0.72 |  |  |  |  |  |  | -14.1 |
|  |  |  |  |  |  |  |  | 6 | 1.08 |  |  |  |  |  |  | -15.9 |

  - PUCCH enhancements for Msg4 HARQ-ACK

    - [Closed] Terminology / Focused situation

Before starting discussion on PUCCH for Msg4 HARQ-ACK, FL believe based on companies’ inputs that the target situation should be clarified. One company [17/Samsung] pointed out that ‘Msg4’ is not used in RAN1 specification and also it is possible to use common PUCCH resource after RRC connection establishment. In addition, [21/DCM] mentioned that Msg4 HARQ-ACK might be transmitted via dedicated PUCCH resource, e.g., in other than initial access.

FL’s understanding is that what we will enhance is coverage performance of PUCCH resource provided by PUCCH-ConfigCommon. By using terminology that is used in specification, we can say ‘enhancement for PUCCH to be used when dedicated PUCCH resource configuration is not provided’. It is assumed that the following proposal for conclusion can quickly be agreed.

      - 1st round

Proposal 1-1_v0 (for conclusion)

For coverage enhancement of PUCCH for Msg4 HARQ-ACK,

- RAN1 discuss enhancement for PUCCH to be used when dedicated PUCCH resource configuration is not provided

Q: Do you agree the above proposal for conclusion? If the answer is NO, please share the reason.

| Company | YES/NO | Comment (e.g. reason of NO) |
| --- | --- | --- |
| Apple | Yes |  |
| QC | Yes |  |
| OPPO | Yes |  |
| Samsung | Yes |  |
| NTT DOCOMO | Yes |  |
| Lenovo | Yes |  |
| CMCC | Yes |  |
| Huawei, HiSilicon |  | In our understanding, PUCCH for Msg4-ACK during initial access should be enhanced. If the UE has set-up the connection, gNB should be able to configure dedicated PUCCH resource, where the PUCCH’s coverage does not have problem. If the coverage has problem, gNB should configure the dedicated PUCCH resource. Therefore, we would prefer to add “during initial access” in the conclusion. |
| vivo | Yes | It is also fine to say ‘RAN1 discuss enhancement for PUCCH transmission on common PUCCH resource’. |
| Nokia, Nokia Shanghai Bell | No | The WID states that RAN1 should focus on specifying PUCCH enhancements for Msg4 HARQ-ACK. Introduction of PUCCH repetitions only for Msg4 HARQ-ACK in the specifications is possible by, for example, referring to the PUCCH resource indicated by a DCI 1_0 scrambled with TC-RNTI. We should preferably stick to the mandate given by the WID. |
| Xiaomi | Yes |  |
| Baicells | Yes |  |
| Thales | Yes |  |
| ZTE | Yes |  |
| Ericsson |  | The scope of the work should be PUCCH enhancements for Msg.4 HARQ-ACK on a common PUCCH resource. |
| LG | Yes |  |
| Intel | FFS | We also have the same understanding with Nokia: DCI 1_0 scrambled with TC-RNTI can be used to identify the targe case in specification. Common understanding is needed on the relation between DCI 1_0 scrambled with TC-RNTI and common/dedicated PUCCH resource. Before the agreement on terminology is made, we can use wording from the WID (PUCCH for Msg4 HARQ-ACK). |
| Panasonic | Yes |  |
| CATT | Yes |  |
| MediaTek |  | The wording proposed by moderator seem to expand the scope agreed in the WID. The clarification suggested by Ericsson is helpful. RAN1 may then prioritize PUCCH enhancements for Msg4 HARQ-ACK. |

      - 1st round summary

Support: Apple, QC, OPPO, Samsung, DCM, Lenovo, CMCC, vivo, Xiaomi, Baicells, Thales, ZTE, LGE, Panasonic, CATT (15)

Not support: HW, Nokia, Ericsson, Intel, MTK (5)

FL’s observation: Although more companies believe that the proposal is reasonable, several companies suggest following WID text strictly. The following is FL’s reply:

- To HW: As discussed in Rel-15, when dedicated PUCCH is provided is completely up to NW implementation. At least for PDSCH transmission right after Msg4 PDSCH TX/RX and initial access complete, it would be possible that UE does not have dedicated PUCCH configuration yet.

- To Nokia/Ericsson/Intel/MTK: at least there is no text of ‘PUCCH for Msg4 HARQ-ACK’ in RAN1 spec, so what is our target should be clarified. Exactly, it may be possible to use PUCCH resource indicated by DCI 1_0 scrambled with TC-RNTI, but at least there are some cases where C-RNTI for Msg4 is used as below. Strictly speaking, the terminology would also not be aligned with the WID text.

5.1.5 Contention Resolution … 1> if notification of a reception of a PDCCH transmission of the SpCell is received from lower layers: 2> if the C-RNTI MAC CE was included in Msg3: 3> if the Random Access procedure was initiated for SpCell beam failure recovery or for beam failure recovery of both BFD-RS sets of SpCell (as specified in clause 5.17) and the PDCCH transmission is addressed to the C-RNTI; or 3> if the Random Access procedure was initiated by a PDCCH order and the PDCCH transmission is addressed to the C-RNTI; or 3> if the Random Access procedure was initiated by the MAC sublayer itself or by the RRC sublayer and the PDCCH transmission is addressed to the C-RNTI and contains a UL grant for a new transmission: …

Therefore, FL prepares the following proposal. Further down-selection is necessary in this meeting if possible, or in future meeting.

Proposal 1-1_v1 (for conclusion)

For coverage enhancement of PUCCH for Msg4 HARQ-ACK,

- RAN1 discuss enhancement for PUCCH to be used for either of the following options

  - Option 1: when dedicated PUCCH resource configuration is not provided

  - Option 2: when PUCCH resource is indicated by a DCI format 1_0 with CRC scrambled by TC-RNTI

      - 2nd round

FL added another option to hear companies’ feeling and updated a bit correspondingly.

Proposal 1-1_v2 (for conclusion)

For coverage enhancement of PUCCH for Msg4 HARQ-ACK,

- Down-select from the following options

  - Option 1: RAN1 discuss PUCCH transmission when dedicated PUCCH resource configuration is not provided

  - Option 2: RAN1 discuss PUCCH transmission when PUCCH resource is indicated by a DCI format 1_0 with CRC scrambled by TC-RNTI

  - Option 3: No need to discuss terminology. For each proposal, ‘PUCCH for Msg4 HARQ-ACK’ should be used

Q: Which option should be taken? Please share the reason.

| Company | Option | Comment (e.g. reason) |
| --- | --- | --- |
| Apple | 3 | If the group cannot achieve consensus shortly, we prefer to keep the terminology as defined in WID. So far, the NR NTN agreements are based on the terminology of “PUCCH for Msg4 HARQ-ACK”. |
| Panasonic | 1 or 2 | For which case the repetition for PUCCH is applied needs to be clarified. But, before making agreement, we can discuss based on the terminology of “PUCCH for Msg4 HARQ-ACK”. |
| Samsung | 1 | For option 2, we don’t think that it can address all PUCCH coverage issue because at least dedicated RRC signaling message should be transmitted via fallback DCI without PUCCH repetition. Since we already identified there is PUCCH coverage issue if repetition is not enabled, we think that it should address all relevant cases. The issue we are addressing is applicable in general until a UE is provided dedicated PUCCH resources which is also an optional configuration (i.e. a NW is not required by the specifications to provide dedicated PUCCH resources to a UE). For option 3, in the end, we may need to discuss specification impact based on agreement. So, option 3 doesn’t address the issue. |
| OPPO | 1 | The observation on performance gap of PUCCH for Msg4 HARQ-ACK also applies to other PUCCH transmissions before dedicated PUCCH resource configuration, so the enhancement should not be limited to PUCCH for Msg4 HARQ-ACK. |
| LG | 1 or 3 | According to FL’s previous comments, there are some cases where C-RNTI for Msg4 is used. Therefore, Option 2 cannot be considered to include all PUCCH transmissions for Msg4 HARQ-ACK. |
| ZTE | 1 | If the group cannot be aligned, the option 3 which has same wording as WID can be considered now, which is also used in previous meetings. In future the terminology can be further discussed. |
| Lenovo | 1 | We think the main difference between PUCCH for msg4 HARQ-ACK and other PUCCH resource is whether it is common PUCCH resource configuration or dedicated PUCCH resource configuration. |
| NTT DOCOMO | 1 | We have the same view as OPPO. |
| Xiaomi | 1 or 3 | Both option 1 and option 3 are okay to us. |
| vivo | 1 or 3 | We support option 1 and can also support option 3 if supported by majority. |
| CMCC | 1 | We prefer Option 1 to discuss PUCCH transmission enhancement. If we can’t make a down-selection between Option 1 and Option 2 at this meeting, we can focus on technical issues based on the terminology of “PUCCH for Msg4 HARQ-ACK” |
| Huawei, HiSilicon | 2 with revision or 3 | For option 2, regarding comments from OPPO and LG, we think for option 2 the “with CRC scrambled by TC-RNTI” can be replaced by “during the initial access”. We can discuss based on option 3 and follow the scope of WID. |
| Baicells | 1 or 3 | Slightly prefer Option1. For option1, we have the same view as Oppo. For option3, we are also fine with it. |
| CATT | 1 | Same view with OPPO |
| Thales | 1 | We support option 1 |
| MediaTek | 3 | RAN1 can keep the terminology as defined in WID for “PUCCH for Msg4 HARQ-ACK” for now, and discuss technical issues |
| QC | 3 | Option 3 for now. Decision of Option 1 or 3 can be made after solution is identified. |
| Nokia, Nokia Shanghai Bell | Option 2 | Regarding FL’s comment on DCI scrambled with C-RNTI, it is our understanding that such DCI do not schedule a Msg4 PDSCH (with UE contention resolution identity), but the contention resolution is considered successful if the PDCCH is addressed to C-RNTI. In addition, when the PDCCH is addressed to the C-RNTI, the UE is expected to be in RRC_CONNECTED with a dedicated PUCCH configuration (if the network has decided to configure the UE for such). |
| Intel | 3 or 2 | We prefer Option 3 for now. Exact wording in specification can be agreed later. Also, we agree with comments from Nokia. |
| Ericsson | 3 | Keep “PUCCH for Msg4 HARQ-ACK” for now. We can discuss later if the scope should be according to option 1 or 2. |

      - 2nd round summary

Although a bit more companies support Option 1, still there are companies preferring Option 2 or 3. What behavior (e.g., when dedicated PUCCH is provided) should be assumed and what exact case is intended in this agenda are not aligned among companies. Therefore, FL will use ‘PUCCH for Msg4 HARQ-ACK’ for each proposal as before, and thus no further discussion is pursued at least in this meeting.

However, without alignment among companies, it will become so big issue when we have initial version of Rel-18 spec. FL recommends clarifying it in future, e.g., at the next RAN1 meeting or at the next RAN plenary to add a note for clarification.

FYI, in Rel-15 PUCCH AI, the following terminology was used.

Agreements: - For resource allocation for HARQ-ACK before RRC connection setup - 4-bit RMSI indicates an entry into a 16-row table and each row in the table configures a set of cell-specific PUCCH resources/parameters - PUCCH duration of at least {2, 14} - If frequency hopping is enabled for long PUCCH, the PRB(s) of the two hops are x PRBs away from each edge of the initial UL BWP. - FFS on x - FFS on whether to support frequency hopping for short PUCCH - FFS frequency hopping is always enabled or indicated in RMSI to enable/disable hopping - Check further offline on the UE feature related discussion Working assumption: - For HARQ-ACK resource allocation before a UE has a dedicated PUCCH configuration, - Frequeny hopping is always enabled for PUCCH transmission for FR1 and FR2.

    - [Closed] Enhancement mechanism - repetition

On enhancement mechanism of PUCCH to be used when dedicated PUCCH resource configuration is not provided, it can be seen that a lot of companies believe repetition should be introduced and that no other mechanism is proposed for this PUCCH (though there are other proposals commonly applied to UL channel, e.g., polarization perspective). As the companies mention in their contributions, FL thinks that repetition is straightforward way. For the details, further discussion would be necessary for at least the following five aspects, based on companies’ inputs and Rel-17 Msg3 repetition.

A: Procedure to perform repetitions (e.g., cell-specific configuration, request to gNB and dynamic indication from gNB, UE capability report, etc.)

[5/vivo] [7/OPPO] [8/CATT] [9/Intel] [11/xiaomi] [12/CMCC] [13/ETRI] [15/Apple] [17/Samsung] [18/Baicells] [19/Pana] [20/LGE] [21/DCM] [22/QC] [24/Nokia, NSB]

B: Repetition factor

[2/MTK] [3/HW, HiSi] [5/vivo] [6/ZTE] [7/OPPO] [8/CATT] [15/Apple] [16/Ericsson] [19/Pana] [21/DCM]

C: Frequency hopping

[5/vivo] [9/Intel (not support inter-slot FH)] [15/Apple]

D: Repetition slot counting

[5/vivo] [9/Intel]

E: DMRS bundling

[6/ZTE] [15/Apple] [18/Baicells] [19/Pana (not support)]

Therefore, the following proposal is made.

      - 1st round

Proposal 1-2_v0

For PUCCH to be used when dedicated PUCCH resource configuration is not provided,

- Support PUCCH repetition

  - Further discuss at least the following

    - Procedure to perform repetitions (e.g., cell-specific configuration, request to gNB and dynamic indication from gNB, UE capability report, etc.)

    - Repetition factor

    - Frequency hopping

    - Repetition slot counting

    - DMRS bundling

Q: Do you agree the above proposal? If the answer is NO, please share the reason.

| Company | YES/NO | Comment (e.g. reason of NO) |
| --- | --- | --- |
| Apple | Yes |  |
| QC | Yes | Suggest to rephrase the first bullet as – Procedure and signaling including indication of UE capability and PUCCH repetition |
| OPPO | Yes |  |
| Samsung | Partially | We prefer to focus on essential aspects as in the first two sub-bullets. Other aspects can be deprioritized. DMRS bundling will not be needed if sufficient repetition factor can be provided. |
| NTT DOCOMO | Yes |  |
| Lenovo | Yes | Also fine with QC’s version. |
| CMCC | Yes |  |
| Huawei, HiSilicon | Partially | Agree with Samsung. Also, DMRS bundling is an optional feature and we should not assume it for PUCCH used during RACH. |
| vivo | Yes | Since intra-slot frequency hopping of PUCCH on common PUCCH resource is already supported and considering inter-slot frequency hopping could be a UE capability, we suggest to change ‘frequency hopping’ to ‘inter-slot frequency hopping and the relevant UE capability’. |
| Nokia, Nokia Shanghai Bell | Yes | As stated in previous reply the main target should be PUCCH for HARQ-ACK for Msg4, as outlined in the WID. |
| Xiaomi | Yes | Also fine with QC’s update that adding the signaling in the first sub-bullet. |
| Baicells | Yes |  |
| Thales | Yes | Agree with the proposal. Qualcomm’s version is ok and can be slightly modified as follows: Procedure and signaling including indication of PUCCH repetition facture and indication of UE capability |
| ZTE | Yes |  |
| Ericsson | Yes | Agree to "support PUCCH repetition". The list of things to discuss further depends on potential agreements in section 4.1.3 to 4.1.7. |
| LG | Partially Yes | Agree with Samsung and Huawei. We prefer to prioritize the first two sub-bullets. |
| Intel | Partially | The following aspects should be included for the first sub-sub bullet (as part of examples) - Procedure to perform repetitions (e.g., cell-specific configuration, request to gNB and dynamic indication from gNB, UE capability report, PUCCH resource determination, Tx beam determination, available slot determination etc.) For the PUCCH resource determination and Tx beam determination, same PUCCH resource and the same Tx beam shall be considered for PUCCH repetitions. For available slot determination, definition of available slots shall be reused as defined for Msg3 PUSCH repetitions. DMRS bundling is optional UE feature and we do not think DMRS bundling can be supported for PUCCH repetition with HARQ-ACK feedback in response to Msg4 PDSCH. Similar discussion/conclusion was made for Msg3 PUSCH repetition. |
| Panasonic | Yes |  |
| CATT | Partially | We prefer to focus on essential aspects as in the first two bullets. Repetition times can be at least 2 or 4. |
| MediaTek | Yes | PUCCH repletion can be supported, QC and Thales revision are fine. The other bullets in the list can be further discussed. |

      - 1st round summary

It can be seen that all companies are fine with the main part. Controversial part is the sub-bullets. Summary for the sub-bullets is the following:

- OK to discuss all bullets: Apple, QC, OPPO, DCM, Lenovo, CMCC, vivo, Nokia, Xiaomi, Baicells, Thales, ZTE, Ericsson, Panasonic, MTK (15)

- Only the first two bullets: Samsung, HW, LGE, CATT (4)

- Other than DMRS bundling: Intel (1)

A lot of companies are OK with the proposal, but several companies suggest focusing the first two bullets. In addition, FL observed that a lot of companies think that at least any enhancement on the last two bullets is unnecessary as discussed in section below. On frequency hopping, the number of companies that are positive to support inter-slot FH is not so small; thus the bullet here can be maintained for further discussion. It is noted that ‘discussion’ does not mean ‘introduce enhancement’.

Regarding QC’s suggestion, FL applies the wording of ‘procedure and signaling’, but the remaining part is unchanged to avoid prioritization of discussion topics in the bullet.

To vivo, details can be discussed later, so FL suggest keeping as it is.

To Intel, FL does not understand what the relationship between TX beam and repetition is, whether there is any impact on PUCCH resource determination. On available slot determination, FL thinks the topic is the same as ‘repetition slot counting’.

Besides, the text for the target is updated as there is no consensus yet.

Proposal 1-2_v1

For PUCCH for Msg4 HARQ-ACK,

- Support PUCCH repetition

  - Further discuss at least the following

    - Procedure and signaling (e.g., cell-specific configuration, request to gNB and dynamic indication from gNB, UE capability report, etc.)

    - Repetition factor

    - Frequency hopping

    - Repetition slot counting

    - DMRS bundling

At Wednesday’s GTW, the following agreement was reached. This section can be closed. Thank you for constructive discussions.

Agreement

For PUCCH for Msg4 HARQ-ACK,

- Support PUCCH repetition

  - Further discuss the specification impact for at least the following

    - Procedure and signaling (e.g., cell-specific configuration, request to gNB and dynamic indication from gNB, UE capability indication before Msg4, etc.)

    - Repetition factor

    - Repetition slot counting for FDD

  - Further study whether to enhance or support the following

    - Frequency hopping

    - DMRS bundling

    - [Closed] Procedure to perform repetitions

From companies’ contributions, FL observed that there are multiple options of high-level procedure to perform repetition. One option is just cell-specific configuration as discussed by 7 companies [7/OPPO] [8/CATT] [11/xiaomi] [13/ETRI] [17/Samsung] [20/LGE] [21/DCM]. All UEs (but probably only capable UEs) shall perform repetitions since channel quality in a cell/beam is not so different among UEs. Another option is to follow Msg3 repetition mechanism, i.e., dynamic indication to perform repetition. This option is raised by at least 11 companies [5/vivo] [9/Intel] [12/CMCC] [13/ETRI] [15/Apple] [17/Samsung] [18/Baicells] [19/Pana] [20/LGE] [21/DCM] [22/QC]. UE decides whether repetition is necessary, UE requests repetition scheduling, and gNB indicates repetition. Furthermore, another option is raised by 2 companies [19/Pana] [21/DCM]; UE determines whether repetition is performed or not, regardless of gNB indication. However, this option is not proposed by any company (even the 2 companies do not prefer). Thus this option can be deprioritized. One note is that ‘how configured/indicated’ is the next step. Such a detailed part will be discussed later.

Besides, at least 3 companies [5/vivo] [22/QC] [24/Nokia, NSB] recommend discussing UE capability report of PUCCH repetition. However, FL noticed that whether such a capability report is necessary or not is dependent on the above high-level procedure. For example, if UE requests repetition to gNB, further capability report becomes unnecessary since the request implies the UE is capable of repetition. This discussion would also be the next step.

Based on the above, FL made the following proposal.

      - 1st round

Proposal 1-3_v0

For PUCCH repetition when dedicated PUCCH resource configuration is not provided,

- Discuss the following two options of procedure to perform repetitions

  - Option 1: UE always performs repetition if configured in cell-specific manner

    - FFS: details of cell-specific configuration

    - FFS: behavior of UE being incapable of repetition

  - Option 2: UE requests repetition and is dynamically indicated to perform repetition

    - FFS: details of repetition request

    - FFS: details of configuration and dynamic repetition indication

  - Whether/how UE reports repetition capability

Q: Do you agree the above proposal? If the answer is NO, please share the reason. If the answer is YES, which option is your preference?

| Company | YES/NO | Option | Comment (e.g. reason of NO) |
| --- | --- | --- | --- |
| Apple | Yes | 2 | Option 1 will waste PUCCH resources if all UEs in NTN coverage perform repetition, no matter the channel conditions. We think the number of PUCCH repetitions should be UE-dependent. For a UE in good channel condition, no PUCCH repetition is needed. For a UE in bad channel condition, a large number of PUCCH repetitions is used. Considering Msg3 PUSCH repetition is UE-specific, we could apply the similar schemes for PUCCH repetition when dedicated PUCCH resource configuration is not provided. |
| QC | Yes | 2 | Option 1 will waste a lot of resource. |
| OPPO | No |  | Option 1 is not our intention. We support a cell-specific indication of the number PUCCH repetition, but the UE performs repetition only if it requests to the gNB. LOS environment is prioritized in R18 NTN as per the WID, so the channel conditions among UEs in a NTN cell is not very different as pointed out in our contribution. In this case, dynamic indication of PUCCH repetition factor brings limited gains, but with lager spec impact. |
| Samsung |  | Further study Option 1 | Since no company observed that no repetition can meet the requirement, the repetition request by UE may not be a feasible/meaningful solution in NTN. Thus, Option 1 seems a good direction to further investigate. Options 1 and 2 are not mutually exclusive. When the UE is configured with repetitions, it would still be better for a gNB to indicate the number of repetitions in order to control the SNR level of the received signal. As for the UE capability reporting, it can be further discussed whether a UE that supports Msg3 repetitions also supports PUCCH repetitions. |
| NTT DOCOMO | Yes | 1 | We slightly prefer Option 1 for simplicity. |
| Lenovo | Yes | 2 | We think repetition will be UE-specific based on channel conditions. |
| CMCC | Yes |  | We are open to further discuss the two options. Considering the situations that all UEs in a NTN cell may suffer from a bad channel condition, cell-specific repetition configuration is reasonable. Option 2 can also be further discussed, and the same mechanism for Msg3 repetition in Rel-17 can be reused for PUCCH repetition when dedicated PUCCH resource configuration is not provided. |
| Huawei, HiSilicon | Yes |  | We are open for the two options for further discussion. |
| vivo | Yes |  | In our understanding, the UE request in Option 2 can be also regarded as a kind of UE capability report. Is UE request or UE capability report expected to be based on msg1 or msg3? If it is based on msg1, we agree that repetition mechanism of msg3 can be reused for PUCCH repetition for msg4 HARQ-ACK. If it is based on msg3, it would be different. We tend to prefer the report/request to be based on msg3 to avoid PRACH resource partitioning. |
| Nokia, Nokia Shanghai Bell | Yes | Option 2 | Option 1 is extremely inefficient from the point of view of network resources, because also UEs that do not need repetitions will do repetitions consuming network resources that could be used for other UEs. |
| Xiaomi | Yes |  | We are open to discuss both options. |
| Baicells | Yes |  | Both options can be considered. Slightly prefer option1 due to its low complexity. |
| Thales | Yes | 2 | Option 1 seems not an optimal design. We support dynamic indication to perform/trigger repetitions. This may depend on different factors e.g. elevation, uplink radio conditions.. Option 2 can be rephrased as follows: Option 2: UE may request repetition and it is dynamically indicated to perform repetition |
| ZTE | Yes | 1 | We think network controlled repetition is enough. In our simulation, only few repetitions are enough to mitigate performance gap. Moreover, the repetition number will not be varied frequently within a cell. Hence, cell-specific configuration from BS is enough. Moreover, we do not think UE request is needed. For Msg3 repetition, it is configured by RAR, which is group-specific configuration before contention resolution instead of UE specific configuration. Hence, in initial access, there is no need to design UE request mechanism for repetition. |
| Ericsson | Yes | Option 2 | Dynamic indication is preferred to allow adaptation of the number of repetitions to the actual channel quality that may differ among UEs and also vary with time e.g. for LEO with earth-fixed beams. |
| LG | Yes | 2 | We prefer to support option 2. Also, we agree with Apple’s comment. We can apply the similar schemes for PUCCH repetition as Msg3 PUSCH repetition. |
| Intel | Yes | Option 2 | We prefer option 2 since not all the UEs require the same repetition level for transmission. We also think and/or shall be considered for Option 2 as follows. - Option 2: UE requests repetition and/or is dynamically indicated to perform repetition |
| Panasonic | Yes | 2 | We think Option 2 is more efficient. The necessary number of repetitions are dependent on the propagation conditions for each UE. For example, UE located at nadir needs less repetitions than UE located at lower elevation angle. |
| CATT | Yes | Option 1 | Based on evaluation result, PUCCH repetition is needed, especially at the initial access. Hence, cell-specific indication of the number PUCCH repetition is convenient as one default assumption. We think Option1 should be required mandatorily. For Option2, it can be applied in the dedicated PUCCH resource after UE has reported its capability. |
| MediaTek | Yes |  | Both options can be discussed. |

      - 1st round summary

All companies other than OPPO are OK with this proposal and which option should be adopted seems divergent as below.

- Option 1: Samsung, DCM, (Baicells), ZTE, CATT (5)

- Option 2: Apple, QC, Lenovo, Nokia, Thales, Ericsson, LGE, Intel, Panasonic (9)

In FL’s understanding, OPPO’s proposal is a kind of Option 2.

Therefore, FL suggests agreeing the proposal as it is (except for the initial phrase), and then we can discuss further in future meeting.

Proposal 1-3_v1

For PUCCH repetition for Msg4 HARQ-ACK,

- Discuss the following two options of procedure to perform repetitions

  - Option 1: UE always performs repetition if configured in cell-specific manner

    - FFS: details of cell-specific configuration

    - FFS: behavior of UE being incapable of repetition

  - Option 2: UE requests repetition and is dynamically indicated to perform repetition

    - FFS: details of repetition request

    - FFS: details of configuration and dynamic repetition indication

  - Whether/how UE reports repetition capability

At Wednesday’s GTW, the following agreement was reached. This section can be closed. Thank you for constructive discussions.

Agreement

For PUCCH repetition for Msg4 HARQ-ACK,

- Discuss the following options of procedure to perform repetitions

  - Option 1: UE always performs repetition if configured in cell-specific manner

    - FFS: details of cell-specific configuration

    - FFS: behavior of UE being incapable of repetition

  - Option 2: UE requests repetition and is dynamically indicated to perform repetition

    - FFS: details of repetition request

    - FFS: details of configuration and dynamic repetition indication

  - Option 3: UE indicates repetition capability and is dynamically indicated to perform repetition

  - How UE indicates repetition capability before Msg4

At the next meeting, down-selection from the above options and listing options of further details will be discussed. FL would like to ask companies to study on top of this assumption.

    - [Email] Repetition factor

Which repetition factor is available needs to be discussed and there are inputs from companies on this issue. One company [2/MTK] recommends further discussion on repetition factor. Two companies [6/ZTE] [16/Ericsson] observed max 2 repetitions are sufficient. One note is that [16/Ericsson] assumes 1T2R in their simulation evaluation. Four companies [5/vivo] [7/OPPO] [8/CATT] [21/DCM] suggest 4 repetitions as maximum. One note is that [8/CATT] assumes 1T2R in their simulation evaluation and 4 symbols PUCCH, to make the proposal. Another note is that [7/OPPO] evaluated performance of 2 bits UCI. Three companies [15/Apple] [19/Pana] [21/DCM] propose 8 repetitions, while it seems that [19/Pana] [21/DCM] include some SNR margin. One company [3/HW, HiSi] recommends max 12 repetitions. The following are list of the rationale on why such a value is proposed.

[3/HW, HiSi]:

10 rep w/o FH: (Required SNR) – (CNR) = (-7.99) – (-8.04) = 0.05 dB

12 rep w/o FH: (Required SNR) – (CNR) = (-8.47) – (-8.04) = -0.43 dB

8 w/ FH: (Required SNR) – (CNR) = (-7.65) – (-8.04) = 0.39 dB

10 w/ FH: (Required SNR) – (CNR) = (-8.15) – (-8.04) = -0.11 dB

[5/vivo]:

2 rep: (Required SNR) – (CNR) = (-6.16) – (-7.61) = 1.45 dB

4 rep: (Required SNR) – (CNR) = (-9.11) – (-7.61) = -1.50 dB

[6/ZTE]:

no rep: (Required SNR) – (CNR) = (?) – (-8.1) = 4.5 dB

2 rep: (Required SNR) – (CNR) = (?) – (-8.1) = ? dB

[7/OPPO]:

no rep for 2 bits UCI: (Required SNR) – (CNR) = (-3.93) – (-8.11) = 4.18 dB

‘the repetition gain can be estimated by 10*log(number of repetition)’

[8/CATT]:

2 rep w/o FH w/ 1T2R w/ 4 symbols: (Required SNR) – (CNR) = (-9.0) – (-10.6) = 1.6 dB

4 rep w/o FH w/ 1T2R w/ 4 symbols: (Required SNR) – (CNR) = (-12.5) – (-10.6) = -1.9 dB

no rep w/o FH w/ 1T2R w/ 14 symbols: (Required SNR) – (CNR) = (-8.8) – (-10.6) = 1.8 dB

2 rep w/o FH w/ 1T2R w/ 14 symbols: (Required SNR) – (CNR) = (-12.7) – (-10.6) = -2.1 dB

[15/Apple]:

‘Considering that PUCCH format 1 (with up to 8 repetitions) can meet the performance requirement’

[16/Ericsson]:

no rep w/ 1T2R: (Required SNR) – (CNR) = (-6.7) – (-8.1) = 1.4 dB

2 rep w/ 1T2R: (Required SNR) – (CNR) = (-8.3) – (-8.1) = -0.3 dB

[19/Pana]:

2 rep: (Required SNR) – (CNR) = (-7.9) – (-8.1) = 0.2 dB

4 rep: (Required SNR) – (CNR) = (-11.0) – (-8.1) = -2.9 dB

8 rep: (Required SNR) – (CNR) = (-13.6) – (-8.1) = -5.5 dB

[21/DCM]:

8 rep: (Required SNR) – (CNR) = (-11.9) – (-8.2) = -3.7 dB

The yellow-highlighted part is unclear why they choose the parameters/values. In FL’s understanding, common PUCCH is always transmitted with FH; 14 symbols repetitions should be prioritized for coverage limitation scenario; -8.1 dB is the CNR value for 1PRB with -5.5 dBi antenna gain; It would be better to assume 1T1R based on some companies’ comment at the online session in the last meeting. In addition, OPPO’s comment of ‘the repetition gain can be estimated by 10*log(number of repetition)’ would be basically valid (while a bit different gain might be observed due to some reasons). Therefore, FL’s suggestion of max repetition factor is 4. However, this observation might be incorrect; FL would like to ask companies which value should be maximum.

      - 1st round

Proposal 1-4_v0

For PUCCH repetition when dedicated PUCCH resource configuration is not provided,

- Down-select from the following options of the maximum repetition factor

  - Option A: 2

  - Option B: 4

  - Option C: 8

  - Option D: 10

Q: Do you agree the above proposal? If the answer is NO, please share the reason. If the answer is YES, which option is your preference?

| Company | YES/NO | Option | Comment (e.g. reason of NO) |
| --- | --- | --- | --- |
| Apple | Yes | C | It was observed in RAN1 #110 meeting that a gap of 1.8 to 6 dB is for PUCCH for Msg4 HARQ-ACK. With the update of UE antenna gain assumption from 5 dB to 5.5 dB, the gap could be between 2.3 to 6.5 dB. To cover the upper bound of 6.5 dB, it seems that a maximum repetition of 8 is needed. |
| QC | Yes | C or D |  |
| OPPO | Yes | B | For the UCI bits, 1 UCI bit is used in our LLS simulation, but there is typo (2 bits UCI) in our simulation parameters. Sorry for the incovenience. In addition, PUCCH Format 1 with 14 symbols and FH should be taken as baseline for PUCCH repetition, and maximum repetition factor of 4 is enough to compensate the gap of 4.18dB. |
| Samsung | Yes | Option C | We prefer a larger value to ensure reliability in various scenarios. At least 8 - it is aligned with the maximum repetition value PUCCH supports in connected mode, but our preference is larger than 8. |
| NTT DOCOMO | Yes | C | In our simulations we observed that at least 4 repetitions would be needed. We think it would be straightforward to support up to 8 repetitions, similar to the existing PUCCH repetition. |
| Lenovo | Yes | C |  |
| CMCC | Yes | C | Considering the channel conditions may become worse in some employment scenarios, 8 repetitions is reasonable. Anyway gNB can configure/indicate smaller repetition factors based on the channel conditions. |
| Huawei, HiSilicon | Yes | C or D | Considering the range of observed gap, we prefer to keep the upper bound of the gap. Therefore, we prefer option D to keep some margin. However, we are fine to compromise to option C. |
| vivo | Yes | B or C | According to the simulation results from our contribution, the performance of PUCCH for Msg4 HARQ-ACK with 4 could already compensate the gap and meet the CNR calculated in link budget calculation. While the performance of PUCCH for Msg4 HARQ-ACK with 8 repetitions can match with the performance of normal PUCCH for ACK-missed. Thus, we support 4 and can also live with 8 as the maximum repetition number. |
| Nokia, Nokia Shanghai Bell | Yes | Option C | Align the maximum number of repetitions with PUCCH repetitions in RRC connected |
| Baicells | Yes | Option C |  |
| Thales | Yes | C or D | At least a maximum repetition factor of 8 should be supported. |
| ZTE | Yes | A | In our simulations, with 2 repetitions the required SNR is around the link budget. With other general UL enhancements, e.g., DMRS bundling, the performance gap can be compensated. |
| Ericsson | Yes |  | Assumptions (in particular the number of RX antennas in the satellite) should be aligned before determining max number of repetitions. |
| LG | Yes | C |  |
| Intel | Yes | Option D |  |
| Panasonic | Yes | B or C | Our simulations show that 4 repetitions can meet the requirement with single Rx. On the other hand, to support up to 8 repetitions would also be reasonable to align with the existing PUCCH repetitions. |
| CATT | Yes | A or B | Based on our simulation result, the SINR gap for PUCCH is only 1-2dB. If considering other non-ideal factors, the repetition times are not expected with more than 4. |
| MediaTek | Yes | C |  |

      - 1st round summary

All companies are fine with the current proposal and preferred options are summarized as below.

- Option A: ZTE, CATT

- Option B: OPPO, vivo, Panasonic, CATT

- Option C: Apple, QC, Samsung, DCM, Lenovo, CMCC, HW (compromise), vivo (live with), Nokia, Baicells, Thales, LGE, Panasonic, MTK

- Option D: QC, HW, Thales, Intel

It can be observed that most companies are OK with Option C. Although only a few companies prefer smaller number, it can be covered by Option C. One note from FL as summarized above is that at least CATT’s simulation assumption seems to be invalid. In addition, as pointed out by several companies, the same number as for dedicated PUCCH would be sufficient. Therefore, FL would like to try this proposal.

Proposal 1-4_v1

For PUCCH repetition for Msg4 HARQ-ACK,

- Option C is supported from the following options of the maximum repetition factor

  - Option A: 2

  - Option B: 4

  - Option C: 8

  - Option D: 10

      - 2nd round

FL updated a bit for simpler proposal, and for more acceptable.

Proposal 1-4_v2

For PUCCH repetition for Msg4 HARQ-ACK,

- Configurable and/or indicatable repetition factors are 2, 4, 8.

Q: Do you agree the above proposal? If the answer is NO, please share the reason.

| Company | YES/NO | Comment (e.g. reason of NO) |
| --- | --- | --- |
| Apple | Yes with comments | Just for completeness, 1 repetition needs to be added to the list. |
| Panasonic | Yes |  |
| Samsung |  | A value larger than 8 would be better. There is no reason to preclude a 4th value, e.g. 10 or 16. |
| OPPO | Yes | Although we prefer a smaller maximum repetition factor of 4, we can also live with the proposal, which alignes with dedicatded PUCCH configuration. However, a repetition factor larger than 8 is not expected. |
| LG | Yes | We are generally fine with this proposal. Additionally, repetition factor ‘1’ may be also considered in order to handle legacy UEs. |
| ZTE | No | We think a repetition number of 2 is enough when other general UL enhancements such as DMRS bundling are considered. Hence, only 1bit configuration is enough, which indicate repetition number 1 and 2. |
| Lenovo | Yes |  |
| NTT DOCOMO | Yes |  |
| vivo |  | We are fine with 2 and 4 repetitions. In addition, one repetition is also needed since gNB can still configure single repetition even if UE requests or indicates support of msg4 PUCCH repetition. If all company agree on 8 repetitions, we can also live with it. |
| CMCC | Yes with comments | Maybe we can add an FFS to allow us discuss whether 1 repetition can be added to the list. And we prefer not to introduce repetition factor larger than 8. |
| Huawei, HiSilicon |  | We prefer a larger value, e.g. 10, to have some margin. We can live with add a FFS: larger repetition value. |
| Baicells | Yes |  |
| CATT | Yes |  |
| Thales | Yes |  |
| MediaTek | Yes |  |
| QC | Yes | 1 repetition must be allowed. |
| Nokia, Nokia Shanghai Bell | Yes | Regarding Qualcomm’s comment, we find it obvious that some kind of legacy support would be obvious. No need to explicitly capture this. |
| Ericsson | Yes |  |

      - 2nd round summary

Proposal 1-4_v3

For PUCCH repetition for Msg4 HARQ-ACK,

- Configurable and/or indicatable repetition factors are 2, 4, 8.

  - FFS: whether larger repetition factor is supported

  - Note: PUCCH transmission without repetition is also applicable.

FL’s observation: Most companies are OK with 2/4/8; this should be agreeable. On larger value, only a few companies propose it. For progress, one FFS is added. They can propose it in future if it is really necessary. Regarding ‘1’, this is not ‘repetition’ and the existing parameters (nrofSlots, pucch-RepetitionNrofSlots-r17) have n2/n4/n8 as the candidates. It would be unnecessary to add ‘1’ for repetition factor. One note is added to clarify this. To ZTE, although FL understands that your simulation results show 2 is sufficient, other companies’ results show larger value is necessary. To push your proposal, FL thinks that you should check why other companies’ results are incorrect and RAN1 should decide repetition factor according to only your results.

Please follow email discussions for 1st check point.

    - [Closed] Frequency hopping

FL observed that 3 companies [5/vivo] [9/Intel] [15/Apple] refer to inter-slot FH when repetition is performed. In the existing specification, common PUCCH resource (w/o repetition) is transmitted with intra-slot FH always. In addition, in Rel-17 coverage enhancement, inter-slot FH was introduced for Msg3 repetition. At least discussion on inter-slot FH would be necessary. Companies’ views are, [15/Apple] prefers to introduce while [9/Intel] thinks this new mechanism is unnecessary. FL would like to hear other companies’ views.

      - 1st round

Proposal 1-5_v0

For PUCCH repetition when dedicated PUCCH resource configuration is not provided,

- Down-select from the following alternatives of the maximum repetition factor

  - Alt 1: Support inter-slot FH

    - FFS: details

  - Alt 2: Not support inter-slot FH

    - i.e., intra-slot FH is applied

Q: Do you agree the above proposal? If the answer is NO, please share the reason. If the answer is YES, which alternative is your preference?

| Company | YES/NO | Alt | Comment (e.g. reason of NO) |
| --- | --- | --- | --- |
| Apple | Yes | 1 | Like Msg3 repetition, the inter-slot FH can be introduced for PUCCH repetition when dedicated PUCCH resource configuration is not provided. Since the inter-slot FH benefits the case where PUCCH repetition is across multiple slots, we do not see a reason not introducing inter-slot FH for PUCCH repetition. |
| QC | Yes | 1 |  |
| OPPO | Yes |  | If inter-slot FH can bring additional performance gain compared with intra-slot FH, it should be supported. |
| Samsung | Yes | Alt. 2 | We think that sufficient repetition factor with intra-slot FH is enough to meet the requirement. |
| NTT DOCOMO | Yes | 1 | Inter-slot FH may be useful to improve channel estimation accuracy over intra-slot FH. |
| Lenovo | Yes | 1 |  |
| CMCC | Yes | 1 |  |
| Huawei, HiSilicon | Yes | Alt.2 | We didn’t observe attractive gain of inter-slot frequency hopping. |
| vivo | Yes | 1 |  |
| Nokia, Nokia Shanghai Bell | Yes | Alt1 |  |
| Baicells | Yes | Alt1 | Support inter-slot FH and further study details, so that DMRS-bundling can be well used. |
| Thales | Yes | 1 | We support inter-slot frequency hopping. |
| ZTE | Yes | 2 | For LOS scenario, the frequency selective is not significant, which indicates that the FH is not likely to provide much gain. Since intra-slot FH is already supported, there is no need to support inter-slot FH, whose gain is unclear. |
| Ericsson | Yes |  | Discuss/simulate further before down-selection. |
| LG |  |  | We can discuss this issue further after the repetition factor has been determined. By the way, the "the maximum repetition factor" in this proposal seems to be a typo. |
| Intel | Yes |  | We are open to consider inter-slot frequency hopping. |
| Panasonic | Yes | 1 | Inter-slot FH would have performance gain compared to intra-slot FH thanks to better channel estimation accuracy. |
| CATT | YES | 2 | Since the gain of inter-slot FH is limited, so we suppose inter-slot FH is deprioritized. |
| MediaTek |  |  | Discuss/simulate further before down-selection. |

      - 1st round summary

All companies are OK with this proposal and companies’ preference on Alt 1 vs Alt 2 is divergent. There are suggestions to perform further discussion/simulation. Therefore, FL suggests agreeing the proposal as it is in this meeting.

Proposal 1-5_v1

For PUCCH repetition for Msg4 HARQ-ACK,

- Down-select from the following alternatives of FH

  - Alt 1: Support inter-slot FH

    - FFS: details

  - Alt 2: Not support inter-slot FH

    - i.e., intra-slot FH is applied

Based on discussion so far, including discussion at the 1st GTW session, FL feels that further simulation evaluation is necessary to conclude whether inter-slot FH is necessary or not, and this would be already clear from the agreement. Therefore, additional discussion in this meeting does not help anymore, and then this section was closed.

    - [Closed] Repetition slot counting

At least 2 companies [5/vivo] [9/Intel] raised this issue in their contributions. When repetition is performed, some slot in the middle of repetitions may be unavailable. In this case, how to handle the unavailable slot is an issue. For example, for 8 repetitions starting at slot n, slot n+4 is not available for this repetition. Then whether the slot n is counted as a part of the 8 repetitions or not? There was this discussion for Msg3 repetition and enhancement to count repetition slots based on available slots was introduced. [9/Intel] suggests reusing this mechanism for this target channel.

Meanwhile, FL wonders if new agreement is really necessary. NTN focuses on FDD-band, where all slots would be available for the repetitions. Furthermore, FL thinks that the ‘available slots’ concept has already been supported for PUCCH repetition. Therefore, FL asks the following question.

      - 1st round

Q: Do you think enhancement of repetition slot counting is necessary? Please share rationale of the opinion.

| Company | YES/NO | Comment |
| --- | --- | --- |
| Apple | No | As mentioned by FL that NTN focuses on FDD band. All slots would be available for repetitions. |
| QC | No | We only consider FDD. |
| OPPO | No | Agree with FL that NTN only considers FDD-band. |
| Samsung | No | Agree with Moderator’s view. |
| NTT DOOCMO | No |  |
| Lenovo | No | For NTN with FDD band, there is no restriction to determine the available slots. |
| CMCC | No | Agree with FL that NTN focuses on FDD bands. |
| Huawei, HiSilicon | No |  |
| vivo | Yes | As FL commented, the ‘available slots’ concept has already been supported for PUCCH repetition. It can be easily introduced for PUCCH repetition when dedicated PUCCH resource configuration is not provided. Furthermore, TDD-band could be supported for the future release, especially above 10GHz. |
| Nokia, Nokia Shanghai Bell | No | Aligned with FL’s understanding |
| Xiaomi | No | Agree with Moderator’s view. |
| Baicells | No | Do not prefer introducing too much complexity. |
| ZTE | No | Agree that FDD is focused now. |
| Ericsson | No |  |
| LG | No | Agree with FL’s comment. |
| Intel | Yes | Agree with vivo. If we want to support complete and forward-compatible feature we need to extend available slot concept for it. |
| Panasonic | No | Agree with Moderator’s view. |
| CATT | No | Agree with Moderator’s view. NTN focuses on FDD-band, where all slots would be available for the repetitions. |
| MediaTek | No |  |

      - 1st round summary

Most companies believe that no discussion is necessary. On vivo’s comment, FL’s understanding is that the current spec text for repetition slot counting behavior is not including a text like ‘for repetition of dedicated PUCCH’. That is, if PUCCH repetition for Msg4 HARQ-ACK is introduced, the existing repetition slot counting is applied even without adding any new text.

Therefore, the proposal is not pursued anymore at least in this meeting.

      - 2nd round

Based on the agreement reached at the 1st GTW session, it is expected to have further discussion on what is specification impact on repetition slot counting to support PUCCH repetition for Msg4 HARQ-ACK. As observed above by FL and supported by a lot of companies, it can be assumed that the current specification works well, but some concern was raised. FL would like to ask what kind of specification impact is assumed.

Proposal 1-6_v0 (for conclusion)

For PUCCH repetition for Msg4 HARQ-ACK,

- The existing specification on repetition slot counting (as in section 9.2.6 of TS 38.213) can be applied without any update.

Q: Do you agree the above proposal for conclusion? If the answer is NO, please share what kind of enhancement is necessary.

| Company | YES/NO | Comment (e.g. what kind of enhancement is necessary) |
| --- | --- | --- |
| Apple | Yes | Available slot counting is only defined for PUSCH, it’s not applied to PUCCH. It’s not appropriate to change the available slot counting for PUCCH. So the current spec is enough. |
| Panasonic |  | We think the same principle as defined in 9.2.6 of 38.213 can be applied. But some update may be necessary, e.g. the name of RRC parameters. We can discuss such details in later phase. |
| Samsung | Yes |  |
| OPPO | Yes |  |
| LG | Yes |  |
| ZTE | Yes |  |
| Lenovo | Yes |  |
| NTT DOCOMO | Yes |  |
| Xiaomi | Yes |  |
| vivo |  | We agree with the proposal in principle. However, we are not sure whether any update is needed for msg4 PUCCH repetition in RRC idle/inactive state yet. Therefore, we propose to have the following update: Proposal 1-6_v0 (for conclusion) For PUCCH repetition for Msg4 HARQ-ACK, - The existing mechanism specification on repetition slot counting (as in section 9.2.6 of TS 38.213) for dedicated PUCCH repetition can be applied without any update. - FFS: whether any update is needed. |
| Huawei, HiSilicon | Yes |  |
| Baicells | Yes |  |
| CATT | Yes |  |
| Thales | Yes |  |
| MediaTek | Yes |  |
| QC | Yes |  |
| Nokia, Nokia Shanghai Bell | Yes |  |
| Intel |  | Support revision from vivo. |

      - 2nd round summary

Proposal 1-6_v1 (for conclusion)

For PUCCH repetition for Msg4 HARQ-ACK,

- The existing specification mechanism on repetition slot counting (as in section 9.2.6 of TS 38.213) can be applied without any update.

  - FFS: whether specification update is needed.

FL’s observation: It would be OK to leave room for spec change if really necessary. FL could not find text like only to dedicated PUCCH resource the section in 38.213 is applied; thus ‘for dedicated PUCCH repetition’ is removed from vivo/Intel’s suggestion.

Please follow email discussions for 1st check point.

      - Reflector-round summary

Via 3GPP RAN1 reflector, the following conclusion was reached. This section can be closed. Thank you for discussion.

Conclusion

For PUCCH repetition for Msg4 HARQ-ACK,

- The existing mechanism on repetition slot counting (as in section 9.2.6 of TS 38.213) can be applied.

  - FFS: whether specification update to apply the existing mechanism to PUCCH repetition for Msg4 HARQ-ACK is needed.

For future meeting, FL assumes that no discussion is triggered unless required specification update is found.

    - [Closed] DMRS bundling

Although three companies [6/ZTE] [15/Apple] [18/Baicells] believe that DMRS bundling is appliable to this discussed PUCCH as discussed and specified (if needed) for Msg3 PUSCH. Meanwhile, [19/Pana] evaluated PUCCH repetition performance with DMRS bundling, and the observation is that benefit of DMRS bundling is marginal. In addition, FL’s understanding is that DMRS bundling is not applicable to Msg3 repetition. It seems that why simple Msg3 repetition is insufficient is unclear. Therefore, FL asks the following question.

      - 1st round

Q: Do you think DMRS bundling should be introduced for PUCCH of Msg4 HARQ-ACK? Please share the reason.

| Company | YES/NO | Comment |
| --- | --- | --- |
| Apple | Yes | If PUCCH repetition is introduced, the DMRS bundling plus the inter-slot frequency hopping could enhance the performance of PUCCH reception. |
| QC | To be discussed later | DMRS bundling is a UE capability. Support of DMRS bundling for PUCCH for Msg4 HARQ-ACK may need additional signaling. |
| OPPO | Yes | DMRS bundling for PUCCH of Msg4 HARQ-ACK can be considered to further improve the coverage performance. |
| Samsung | No | We think that sufficient repetition factor is enough to meet the requirement, and it would be a simpler solution |
| NTT DOCOMO | No | We think that repetitions would be sufficient. |
| Lenovo | Yes | If DMRS bundling is introduced for PUSCH. Similar mechanism can be considered for PUCCH. |
| CMCC | Deprioritized | This issue is related to the discussion about repetition factor. The performance gain of DMRS bundling will be limited if the repetition factor is small. |
| Huawei, HiSilicon | No | DMRS bundling is an optional feature. We should not assume it during RACH procedure. |
| Vivo | No | We think introducing DMRS bundling for PUCCH for Msg4 HARQ-ACK is unnecessary. The support of DMRS bundling requires UE capability and the determination of TDW could complex the procedure of RACH. It was not supported for msg3 repetition either in Rel-17. |
| Nokia, Nokia Shanghai Bell | No | DMRS bundling feature is RRC configured so new enhancements should be introduced to support DMRS bundling for the PUCCH of the Msg4. We also have same understanding as FL that DMRS bundling is not applicable to Msg3 repetitions. |
| Xiaomi | No | We think repetition is sufficient. |
| Baicells | Yes | We have the same view with Apple. Performance of DMRS-bundling should be evaluated based on non-FH or inter-slot FH（based on Alt1 in Proposal 1-5_v0）. |
| Thales | No | Enhancement on PUCCH of Msg4 HARQ-ACK with only repetitions would be enough. |
| ZTE | Yes | With DMRS bundling, the required repetition number may be smaller, which saves PUCCH resources. |
| Ericsson | No | It is not motivated since the performance of PUCCH repetitions without DMRS bundling is sufficient. |
| LG | FFS | DMRS bundling of PUCCH for Msg4 HARQ-ACK can be discuss further after DMRS bundling of PUSCH has been determined. |
| Intel | No | As mentioned above, DMRS bundling is optional UE feature and we do not think DMRS bundling can be supported for PUCCH repetition with HARQ-ACK feedback in response to Msg4 PDSCH. Similar discussion/conclusion was made for Msg3 PUSCH repetition. |
| Panasonic | No | According to our simulation results, DMRS bundling of PUCCH format 1 for Msg4 has almost no gain. Our results shows that ACK miss-detection performance determines the required SNR rather than ACK to NACK error, and therefore, improvement of channel estimation due to DMRS bundling would not much help. |
| CATT | No | We think that sufficient repetition factor is enough to meet the requirement, and it would be a simpler solution. Moreover, gains on DMRS bundling for PUCCH are very limited. |
| MediaTek | Discuss later | DM RS bundling can first be discussed for PUSCH. |

      - 1st round summary

More companies believe that DMRS bundling is unnecessary for PUCCH repetition of Msg4 HARQ-ACK, and there are suggestions from several companies that discussion should be postponed to wait for progress of DMRS bundling for PUSCH.

Therefore, the proposal is not pursued anymore at least in this meeting.

Based on discussion so far, including discussion at the 1st GTW session, FL feels that further simulation evaluation is necessary to conclude whether DMRS bundling should be supported or not, and this would be already clear from the agreement. Therefore, additional discussion in this meeting does not help anymore, and then this section was closed.

FL assumes that discussion will be triggered after we had sufficient progress on DMRS bundling for PUSCH.a

    - [Closed] Others

There might be other discussion topics for this PUCCH repetition but might not. FL prepares a box to share companies’ views on further topics.

      - 1st round

Q: Please share it if you think there is another discussion topic relevant to PUCCH of Msg4 HARQ-ACK. If other companies tend to agree/disagree some suggestion from other company, please input your company name in either of the right two columns.

| Company | Which topic should be discussed? | Supporting company | Not supporting company |
| --- | --- | --- | --- |
| Baicells | One side effect of repetition should be considered: With repetition, Msg4 HARQ-ACK will cost more time.Therefore time domain resource may have heavy burden, and thus capacity is reduced. To counter this side effect on capacity, new method for frequency domain allocation should be taken. For example, different UE can use different PUCCH resources in frequency domain, so that the capacity can be increased, as illustrated in the figure below. |  |  |
| Intel | PUCCH resource determination and Tx beam determination. It should be clarified that the same PUCCH resource and the same Tx beam is applied for all PUCCH repetitions. |  |  |

      - 1st round summary

Baicells proposed to discuss capacity issue when the repetition is applied, but FL’s understanding is that the topic is out of scope since the issue is not coverage enhancement topic.

Intel proposed to discuss TX beam when the repetition is applied, but FL does not understand what the relationship between TX beam and repetition is. TX beam determination for PUCCH without repetition should be applied and thus further discussion on this issue is unnecessary. On PUCCH resource determination, if companies believe that enhancement on this point is necessary, they can propose it for ‘procedure and signaling’ part.

FL will prepare a box to share further view on these topics or any others.

      - 2nd round

Proposal 1-8_v0

For PUCCH repetition for Msg4 HARQ-ACK,

- Discuss capacity enhancement

- Discuss TX beam determination

Q: Do you agree to discuss the above topic?

| Company | YES/NO | Comment |
| --- | --- | --- |
| Apple | No | Out of WID scope |
| Panasonic | No | Capacity enhancement seems not specific to repetition of PUCCH for Msg4. Necessity of TX beam determination for PUCCH is not clear to us. |
| Samsung | No | It is clearly out of scope. |
| OPPO | No | Out of scope. |
| LG | No | Agree with FL’s comment. These topics seem to be out of the scope. |
| ZTE | No | Out of scope. |
| Lenovo | No | For the capacity related issue, PUCCH repetition for dedicated resource in legacy release have same issue. We don’t think any enhancement is necessary. For TX beam related issue, we think beam is not that critical in NTN as in NR. So we think this issue can be deprioritized. |
| NTT DOCOMO | No | As for capacity, that is out of the scope of the WID. Regarding Tx beam, we doubt that there could be beneficial enhancements for coverage in NTN. Existing mechanisms can be reused. |
| vivo | NO | This topic is for coverage enhancement, not for capacity enhancement. Therefore, we do not think first bullet is in scope of this work item. Regarding the Tx beam determination, we think the following text copied from 38.213 is enough, no matter whether the msg4 PUCCH is repeated or not. The UE transmits the PUCCH using the same spatial domain transmission filter as for a PUSCH transmission scheduled by a RAR UL grant as described in clause 8.3.. Therefore, discussion on 2nd bullet is not needed either. |
| Baicells | No | We are fine to compromise not to discuss capacity issue in this meeting. |
| CATT | No | Out of scope. |
| MediaTek | No | Out of scope |
| QC | No |  |
| Intel |  | The intention to discuss PUCCH resource determination and Tx beam determination is to make an agreement that the same PUCCH resource and the same beam is applied for all the PUCCH repetitions. We are not sure if it is clear based on the existing specification. |
| Ericsson | No | Out of scope |
| CMCC | No | 1st sub-bullet is out of scope. 2nd sub-bullet can depend on UE implementation. |
| Nokia, Nokia Shanghai Bell | No | Aligned with FL’s view on capacity enhancements. On Tx beam determination, we believe it is a UE implementation feature, and as long as the UE does not need to keep phase continuity (DMRS bundling), we do not see what should be discussed from a standard point of view. |

      - 2nd round summary

FL found that situation is clear: companies believe that there is no necessity to discuss the two proposals. Then further discussion is not pursued and this section was closed.

  - DMRS bundling for PUSCH taking into account NTN-specifics

    - [Open] Necessity of RAN1 enhancement

FL thanks companies for detailed evaluations/observations on whether DMRS bundling is applicable to PUSCH in NTN and whether RAN1 enhancement is necessary or not.

At least 4 companies [17/Samsung] [19/Pana] [21/DCM] [22/QC] argued that NTN timing error requirement 29Ts (=29x64xTc), which was defined by RAN4, corresponds to over 20 ms and thus time-domain error within 20 repetitions do not violate the requirement, while at least 3 companies [2/MTK] [3/HW, HiSi] [16/Ericsson] think that 29Ts corresponds to less than 20 ms. At least 5 companies [8/CATT] [17/Samsung] [19/Pana] [21/DCM] [22/QC] believe that there is no significant issue on frequency shift since the amount would be much smaller than 0.1 ppm frequency requirement.

In addition, 4 companies [6/ZTE] [11/xiaomi] [22/QC] [24/Nokia, NSB] pointed out that timing drift and/or doppler shift cause phase difference between repetitions even if UE maintains time/frequency pre-compensation within the repetitions. [11/xiaomi] observed 17 degrees rotation between two consecutive slots when 30 degrees elevation angle. [24/Nokia, NSB] observed 72 degrees variation between two consecutive slots, which is over 25 degrees requirement. On this issue, [22/QC] thinks that gNB can post-compensate this phase variation since the error is predictable.

Then, at least 7 companies [2/MTK] [3/HW, HiSi] [5/vivo] [6/ZTE] [8/CATT] [15/Apple] [16/Ericsson] propose to support some enhancement relative to DMRS bundling for NTN scenario while two companies [19/Pana] [21/DCM] do not think enhancement is necessary.

Based on the above, especially phase rotation issue, FL suggests the following proposal.

      - 1st round

Proposal 2-1_v0 (for conclusion)

For PUSCH DMRS bundling,

- RAN1 concluded that enhancements taking into account NTN-specifics (e.g., phase difference due to timing drift and/or doppler shift) are necessary.

  - FFS: whether/how to address issue of phase difference between two consecutive slots that is over the maximum allowable value

Q: Do you agree the above proposal for conclusion? If the answer is NO, please share the reason.

| Company | YES/NO | Comment (e.g. reason of NO) |
| --- | --- | --- |
| Apple | Yes |  |
| QC | Yes | Suggest change of FFS: FFS: how to address the phase variation and whether to have new requirements for phase difference within a bundling. |
| OPPO | Yes |  |
| Samsung | [No] | It is not clear to us whether enhancement is necessary or not. We would like to discuss more whether target scenario of LEO-1200 violates RAN4 requirement “Maximum allowable phase difference for DMRS bundling” Actually, we noticed that Nokia assumed 0.1ppm for calculating phase difference. However, as WID is clearly saying “The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS)”. From our evaluation results, at most 2Hz frequency difference is happened during 8 repetitions, then ΔΦ=2π×2×8 × 10-3=0.101 rad=6.48 degree which can satisfy the requirement. On the other hand, assuming at most 5 Hz frequency difference is happened during 20 repetitions, ΔΦ=2π×5×20 × 10-3=0.628 rad=36 degree which cannot satisfy the requirement although the assumption itself is not aligned with RAN4’s note (This requirement applies only for FDD bands, for supported DMRS bundling configurations of 16 slots). Thus, we would like to hear more views on this point. |
| NTT DOCOMO | Yes |  |
| Lenovo | Yes |  |
| CMCC | Yes |  |
| Huawei, HiSilicon | Yes | We think the enhancement is necessary. Regarding phase continuity issue, we have some analysis that antenna switching may also cause phase discontinuous. This should be also discussed. Regarding FFS, it is still not clear whether the phase difference would be larger than the maximum allowable value. Proposal 2-1_v0 (for conclusion) For PUSCH DMRS bundling, - RAN1 concluded that enhancements taking into account NTN-specifics (e.g., phase difference due to timing drift, antenna switching and/or doppler shift) are necessary. - FFS: whether/how to address issue of the phase difference between two consecutive slots that is can be over the maximum allowable value |
| vivo | Yes |  |
| Nokia, Nokia Shanghai Bell | Yes | It should be further discussed whether the requirements for the phase continuity should be applied at RAN1 level or at RAN4 level (from requirements side). Further, on the FFS point it is crucial that we address both phase difference between neighboring slots and maximum phase difference between slot ‘0’ and slot ‘p’. Pushing the responsibility of phase correction to the gNB side is not an acceptable solution, since the 0.1 PPM requirement is on the UE side. If UE/chipset vendors are OK with substantially tighter requirements on the PPM, we are open to discussions. |
| Xiaomi | Yes |  |
| Baicells | Yes |  |
| Thales | Yes | In our view the enhancement is necessary (especially for a the bundle size >2) |
| ZTE | Yes |  |
| Ericsson | Yes | Discuss further if phase variation can be pre-compensated by the UE. |
| LG | Yes |  |
| Intel | Yes |  |
| Panasonic |  | Further study/clarification would be needed as Samsung mentioned. |
| CATT | Yes |  |
| MediaTek | Yes | To our understanding, phase variation is predictable and can be pre-compensated by the UE |

      - 1st round summary

Although most companies are OK with the proposal, Samsung/Panasonic have concern. Samsung commented that in the reference scenario smaller error can be found, but FL thinks that Nokia’s point is not only frequency/phase error due to satellite movement but also frequency/phase error due to UE implementation. Basically, it is impossible for UE to use perfect frequency and 0.1 PPM error is allowed even for TN UE. Thus, even if frequency/phase difference within a DMRS bundling window is small enough, still 0.1 PPM error is allowed at UE side. For example, 0.08 PPM error at slot 0 to 0.1 PPM error at slot p. Of course, max allowable phase difference for DMRS bundling can/should be discussed, which is covered by FFS. However, anyhow FL observed from companies’ contributions/inputs that some mechanism e.g., for gNB to know/control when UE will/can perform update of UL pre-compensation is necessary.

Proposal 2-1_v1 (for conclusion)

For PUSCH DMRS bundling,

- RAN1 concluded that enhancements taking into account NTN-specifics (e.g., phase difference due to timing drift and/or doppler shift) are necessary.

  - FFS: whether/how to address issue of maximum allowable phase difference for DMRS bundling

      - 2nd round

Note that HW’s proposal is discussed in section 4.2.3.

Proposal 2-1_v1 (for conclusion)

For PUSCH DMRS bundling,

- RAN1 concluded that enhancements taking into account NTN-specifics (e.g., phase difference due to timing drift and/or doppler shift) are necessary.

  - FFS: whether/how to address issue of maximum allowable phase difference for DMRS bundling

Q: Do you agree the above proposal for conclusion? If the answer is NO, please share alternative proposal.

| Company | YES/NO | Comment (e.g. which proposal should be agreed instead) |
| --- | --- | --- |
| Apple | Yes |  |
| Panasonic |  | We don’t think this conclusion is necessary for now because what enhancement is necessary has not been identified (related to 4.2.2). Discussion on NTN-specifics (e.g., phase difference due to timing drift and/or doppler shift) is necessary, but RAN1 specification impact is not yet clear. If this conclusion is needed for further discussions, we would suggest to add FFS: RAN1 specification impact |
| Samsung | [No] | We don’t think that “why enhancement is necessary” has been studied in qualitatively and quantitatively. From moderator comments on “0.1 PPM error is allowed even for TN UE”, Yes, 0.1ppm is general requirement for both NTN and TN. Then, do you think that TN has similar issue (e.g., DMRS bundling would not work in TN)? Why this requirement is more important to NTN? For FFS, it is unclear to us why “whether to address issue ~” is considered while main bullet is saying “enhancement is necessary”. We need to discuss for having common understanding on what reasons NTN specific enhancement is necessary for DMRS bundling. Thus, we need to discuss first the framework something like as following as what we did for coverage performance evaluation. Coverage performance in NR NTN is evaluated according to the following steps. - Step 1: CNR is calculated as defined in 6.1.3.1 of TR38.821 - For polarization loss, - 3 dB polarization loss is assumed as baseline, and companies are encouraged to report the value and corresponding justification if other value is used - Step 2: Required SNR of target service is evaluated by LLS - Step 3: The CNR and the required SNR are compared. Since RAN4 defined the requirement for the maximum allowable phase difference for DMRS bundling, we should discuss whether/when the requirement can be met based reference scenario in the WID by comparing phase difference value (which is reported by companies) and the requirement (25, 30 degrees). Then, we can conclude whether enhancement is necessary or not with this evaluation. We think that it can naturally address FFS (whether to ~ ) in the proposal. |
| OPPO | Yes |  |
| LG | Yes |  |
| ZTE | Yes |  |
| Lenovo | Yes |  |
| NTT DOCOMO | Yes |  |
| Xiaomi | Yes |  |
| vivo |  | We support the main bullet of the proposal. However, for the FFS bullet, it is not clear to us what the maximum allowable phase difference means. Even in Rel-17 coverage DMRS bundling discussion, we just considered whether any rules or events should be defined to maintain power consistency and phase continuity. So, we propose the wording of FFS bullet as followed: - FFS: whether and how to define any NTN specific rules or events for supporting DMRS bundling of PUSCH in NTN. |
| CMCC | Yes |  |
| Huawei, HiSilicon |  | Depending on which set of NTN-specifics shall be considered, we can answer the question whether DMRS bundling needs to be supported. In our view, the case where transmission antenna switching is used with DMRS bundling should be discussed. We don’t think segmented transmission would introduce attractive coverage gain based on our evaluations. |
| Baicells |  | We are open for further discussion. We need clarification on whether and why the phase difference due to timing drift and/or doppler shift have impact on specifications. |
| CATT | Yes |  |
| Thales | Yes |  |
| MediaTek | Yes | Vivo proposal for wording for FFS is helpful. This issue for further study is not only phase difference, but more generally how to apply TA and Doppler shift correction without triggering an even with current TDW stopped and new TDW started. It is reasonable to discuss the need for enhancement as commented by Samsung. We provided some analysis in our TDoc. Assuming max repetitions of 20 for VoIP, for LEO it will be necessary to apply the TA several times within 20 ms to meet the RAN4 requirements for timing error. Applying the TA within 20 ms will trigger an event to our understanding. It would be beneficial to do DM RS channel bundling over up to 20 ms to improve channel estimation as low SNR. Antenna switching enhancement proposed by Huawei could be considered with DM RS bundling. To our understanding antenna switching will lead to an event for the phase continuity. |
| QC |  | We don’t have to make a conclusion now. Instead we can identify issues to be studied. Suggest the following: For PUSCH DMRS bundling, RAN1 to study the following by taking into account NTN-specifics (e.g., phase difference due to timing drift and/or doppler shift): - Solutions addressing phase difference - Rules and requirements for supporting DMRS bundling - Note: Support of antenna switching should be considered |
| Nokia, Nokia Shanghai Bell | Yes |  |
| Ericsson | Yes |  |

      - 2nd round summary

It seems that companies’ thinking is not aligned on why enhancement is necessary. As recommended by several companies, FL suggests studying further in consideration of issues raised by companies. At least in this meeting, the issues should be listed in an agreement, then companies can consider them for future meeting to conclude whether enhancement is necessary or not.

In FL’s understanding from companies’ inputs so far and contributions, at least there are two points for this issue. FL’s understanding on them is that they are independent issues basically, but it might be incorrect. Anyhow, further discussion is desirable.

- The first one is phase variation issue. Based on the current RAN4 spec, some restriction on actual TDW length might be necessary to meet the requirement, or RAN4 may update the requirement, or UE can pre-compensate the phase variation and no solution may be necessary.

- The other one is whether a new definition of ‘event’ to decide actual TDW(s) from nominal TDW is necessary or not. When UE performs time/frequency pre-compensation (not the above phase pre-compensation), phase continuity would not be maintained. However, the exact timing was not specified in Rel-17 and it is completely up to UE implementation. Then a new event to have the same understanding on the timing between UE and gNB may be necessary, may not. Companies’ views seem not aligned. Besides, antenna switching may also be the event as proposed by HW (and supported by several companies).

      - 3rd round

Proposal 2-1_v2

For PUSCH DMRS bundling,

- Discuss further the need of enhancement in consideration of at least the following:

  - Phase difference due to timing drift and/or doppler shift.

    - e.g., whether/how long a UE can meet phase continuity requirement specified as Table 6.4.2.5-1 in 38.101-1 in consideration of frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-15, whether RAN1 should introduce enhancement to meet the requirement or RAN4 should update the requirement or UE should pre-compensate phase difference by UE implementation, etc.

  - An event which causes power consistency and phase continuity not to be maintained.

    - e.g., whether the new event (e.g., corresponding to UL time/frequency pre-compensation, antenna switching) is necessary to determine actual TDW(s) from each nominal TDW or the existing specification can work without any specification change, etc.

Q: Do you agree the above proposal? If the answer is NO, please suggest alternative proposal that should be agreed.

| Company | YES/NO | Comment |
| --- | --- | --- |
| Apple | Yes with comments | We do not think antenna switching should be the new event to cause the power consistency and phase continuity. This antenna switching is not specific to NTN. According to WID, “study DMRS bundling for PUSCH taking into account NTN-specifics” does not apply to antenna switching. Hence, we think it should be removed from the example. |
| Lenovo |  | We agree with Apple that antenna switching should be deleted from the proposal. We think we should firstly discuss aspects related to UL time/frequency pre-compensation. |
| ZTE | Yes | For first bullet, the phase difference can be caused by timing drift and/or doppler drift. However, in the examples, only the 0.1ppm frequency error is listed, which is not the TA/Doppler drift generally considered in UL pre-compensation. We suggest to remove “in consideration of frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-1” to provide a general example. |
| FL |  | To ZTE: The same requirement can be found in 38.101-5, and there, ‘pre-compensation’ is also mentioned. Is the above proposal with update of red-color OK for you? |
| Samsung |  | In principle, we are fine with studying on “need of enhancement” for DMRS bundling. However, we have some concerns on “whether RAN4 should update the requirement” (this is up to RAN4), “UE should pre-compensate phase difference by UE implementation” (this is up to UE implementation). Thus we suggest to remove the “whether RAN1 should introduce enhancement to meet the requirement or RAN4 should update the requirement or UE should pre-compensate phase difference by UE implementation, etc.” We may ask RAN4 what is the maximum time interval before a phase compensation is needed, and based on RAN4 feedback we can further study whether there is a need for RAN1 to enhance the Rel-17 DM-RS bundling operation. Regarding “antenna switching”: we don’t think this is in scope of this WID. Thus, it should be removed. |
| NTT DOCOMO | Yes | We agree with Apple about antenna switching. |
| vivo | Yes | We are fine with the proposal to move forward. In our understanding, for 1st sub-bullet, phase difference can be studied in order to determine the time duration in which a UE can meet phase continuity requirement as specified in Table 6.4.2.5-1 of 38.101-1. The time duration determined can be regarded as a kind of the maximum duration that UE reports as agreed in Rel-17. No spec impact may be needed, although we are open to discuss this in detail next meeting. As for 2nd sub-bullet, we think some additional NTN specific events may need to be defined to support DMRS bundling in NTN. |
| Panasonic | Yes | Although the first bullet is “Phase difference due to timing drift and/or doppler shift”, the sub-bullet mention only about doppler shift aspect. We would suggest to change to “in consideration of frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-5 and timing error specified in Table 7.1C.2-1 of 38.133”. |
| Ericsson | Yes with comments | We agree with ZTE that also timing drift can cause phase difference and support their proposal to remove the "in consideration of frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-1". We agree with Apple that antenna switching should be removed from the example. |
| Baicells | Yes with comments | For the first bullet, as long as the phase continuity can be maintained and phase difference between DMRS symbols can be tracked without ambiguity, phase changing itself (due to timing drift and/or doppler shift) would not be a problem. Then we can focus more on the second bullet, i.e. events that may break the phase continuity or brings ambiguity of the phase difference. |
| Huawei, HiSilicon | Yes | As our simulation results showed, the antenna switching would be critical to overcome the large coverage gap under the NTN-channels. Therefore, we think the antenna switching should be considered as a NTN specific issue. Otherwise, the DMRS bundling benefit without antenna switching (-8.2dB with 12 DMRS bundle size@2%rBLER) would be worse than the antenna switching without DMRS bundling (-9.1dB @2%rBLER). Therefore, we think if we don’t consider the case where antenna switching and DMRS bundling are jointly used, there is no need to enhance DMRS bundling at all. |
| OPPO |  | We have some questions on the proposal: 1) the synchronization requirements include not only frequency error but also timing error. Thus, for the first bullet we should include timing error requirement specified in Table 7.1C.2-1 in TS38.133 as well. 2) a general concern from our side is that when RAN4 defined phase continuity requirement in 38.101-1, RAN4 did not consider NTN scenario and therefore the defined requirement margin is not supposed to cover the phase continuity caused by 'new event' such as NTN specific UL T/F pre-compensation. Thus, even companies evaluated/observed that the new event may still meet the RAN4 requirement as such without update, we however find that it is not fair -- as we have eaten up the margin which was not supposed to cover the new event. |
| Nokia, Nokia Shanghai Bell | Yes with comments | Agree with comment from Apple. The TX switching should not be considered here, as it is not to be seen as part of NTN-specifics, and should not be performed by a UE within an actual TDW nor create itself an actual TDW. Further, we would also like to raise the issue related to exactly where the phase difference is measured. For the gNB it is crucial that phases are aligned at the gNB rx antenna port such that any UE pre-compensation takes care of the impairments that happen over the service link. To clarify this aspect related to the first bullet we would suggest that we send a LS to RAN4 asking whether or not the +/- 0.1 PPM requirement is to be seen at the antenna connector at the gNB (implying that UE pre-compensation for service link impairments in time/frequency/phase domain is applied). |
| LG | Yes | We are fine with the proposal in general. We also agree with Apple about antenna switching. |

      - 3rd round summary

To Samsung, although FL understands Samsung has concern, which way should be adopted is up to further discussion. At least in current stage, there are arguments of up to RAN4 or up to UE, thus FL recommends keeping the text as it is. The text is just example, so it should be OK.

To Nokia, FL does not understand the necessity of LS to RAN4. UE pre-compensation is performed as specified in Rel-17. There should be no ambiguity.

Proposal 2-1_v3

For PUSCH DMRS bundling,

- Discuss further the need of enhancement in consideration of at least the following:

  - Phase difference due to timing drift and/or doppler shift.

    - e.g., whether/how long a UE can meet phase continuity requirement specified as Table 6.4.2.5-1 in 38.101-1 in consideration of frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-5 and timing error specified in Table 7.1C.2-1 of 38.133, whether RAN1 should introduce enhancement to meet the requirement or RAN4 should update the requirement or UE should pre-compensate phase difference by UE implementation, etc.

  - An event which causes power consistency and phase continuity not to be maintained.

    - e.g., whether the new event (e.g., corresponding to UL time/frequency pre-compensation, antenna switching) is necessary to determine actual TDW(s) from each nominal TDW or the existing specification can work without any specification change, etc.

    - [Closed] Enhancement mechanism

For the enhancement, it seems that 8 companies [5/vivo] [6/ZTE] [7/OPPO] [8/CATT] [15/Apple] [20/LGE] [12/CMCC] [23/Lenovo] propose segmented pre-compensation. Correspondingly, three companies [5/vivo] [12/CMCC] [23/Lenovo] propose to introduce a new ‘event’ which cause power consistency and phase continuity not to be maintained. In addition, details of this segmented pre-compensation are proposed, e.g., relationship between nominal/actual window and segment, candidate duration, configuration/indication details, etc. Meanwhile, such discussions are found only in a small number of contributions. It seems in this meeting contributions that companies focused on whether enhancement is necessary or not mainly; thus, FL assumes that further details will be discussed in future meeting. If progress is good and time is allowed, FL will prepare a new section to discuss details.

Besides, [17/Samsung] pointed out that an LS (R1-2108703(R4-2114991)) was received from RAN4, where an answer is ‘RAN4 has agreed that TA adjustment should be avoided across the PUSCH/PUCCH transmissions (i.e., from start of first transmission until the end of last transmission) for joint channel estimation’. If pre-compensation update is performed per actual TDW(s), this may not be aligned with the previous RAN4 decision.

Based on the above, FL prepares the following proposal.

      - 1st round

Proposal 2-2_v0

For PUSCH DMRS bundling,

- Support segmented UL time/frequency pre-compensation

  - Update of UL time/frequency pre-compensation is an event which causes power consistency and phase continuity not to be maintained

  - FFS: details (e.g., relationship between TDW and segment, etc.)

- Send an LS to RAN4 to inform this RAN1 discussion and to ask issue, if any

Q: Do you agree the above proposal? If the answer is NO, please share the reason.

| Company | YES/NO | Comment (e.g. reason of NO) |
| --- | --- | --- |
| Apple | Yes | We are not sure if the LS to RAN4 is necessary, since the proposal is to support segmented UL time/frequency pre-compensation where joint channel estimation is performed per segment. This seems not against the existing RAN4 agreement. |
| QC | No | Not sure about the intention of the proposal. DMRS bundling is supported as long as the associated requirements are satisfied. |
| OPPO | Yes | We are also not sure the necessity of the LS to RAN4 because it already mentions that the segmented UL time/frequency pre-compensation is an event in the 1st subbullet. |
| Samsung | [No] | This proposal is premature. We need to discuss and conclude on Proposal 2-1_v0 before discussing to support a solution. |
| NTT DOCOMO | Yes | Segmented UL time/frequency pre-compensation would be useful for gNB to control the timing of updating compensation values by UE. |
| Lenovo | Yes | Agree with Apple that there is no necessity for LS to RAN4. |
| CMCC | Yes | How frequently UE needs to adjust UL time/frequency pre-compensation should be discussed. |
| Huawei, HiSilicon | No | Antenna switching should be discussed to be jointly used with DMRS bundling, which shall impact the actual TDW of DMRS bundling. Regarding the LS cited by Samsung, after that, RAN1 has agreement as following in RAN1 #107-e: Agreement - UE should not perform UE autonomous TA adjustment during the actual time domain window. Therefore, it is allowed to do pre-compensation update per actual TDW(s) based on RAN1 agreement, and JCE is performed per actual TDW rather than the nominal TDW. See the following description in 38211-h30: If DM-RS bundling is applied to PUSCH and/or PUCCH repetitions and/or transport-block processing over multiple slots as described in clause 6.1.7 of [6, 38.214], the UE transmission shall be such that the channel over which a symbol on the antenna port used for uplink transmission is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed if the two symbols are transmitted within the same actual time-domain window. The specification defines the actual TDW based on the event. Therefore, we think we should follow this framework to discuss whether new event should be introduced that determines the actual TDW, rather than introducing segments and then discuss the relationship of segment and actual TDW. Antenna switching should be discussed to be jointly used with DMRS bundling, which shall impact the actual TDW. |
| vivo |  | Whether to support segment transmission seems to be a separate enhancement and is not coverage enhancement. We are fine to treat the update of UL time/frequency pre-compensation as an event which causes power consistency and phase continuity not to be maintained. We do not think an LS is needed. In our understanding, the answer, i.e. ‘RAN4 has agreed that TA adjustment should be avoided across the PUSCH/PUCCH transmissions (i.e., from start of first transmission until the end of last transmission) for joint channel estimation’, means that TA adjustment should be avoided per actual TDW, which corresponds to ‘from start of first transmission until the end of last transmission’. Note that ‘Uplink timing adjustment in response to a timing advance command’ is already defined as an event in TS 38.214 . And the joint channel estimation is actually performed within an actual TDW. Thus, if pre-compensation update is performed per actual TDW(s), this would not collide with the previous RAN4 decision. |
| Nokia, Nokia Shanghai Bell | No | Preferably we should be looking into existing solutions such as TDW. However, if RAN1 finds that existing solutions are not sufficient, we could look into segmentation. With respect to phase and power inconsistency RAN4 should be consulted in an early stage of this work. |
| Xiaomi | No | We think the proposal 2-1 should be firstly discussed. Based on the discussion results of Proposal 2-1, it is feasible to know the relationship between TDW and segment. |
| Baicells | Yes |  |
| Thales | No | The LS to RAN4 is not needed. Support segmented UL time/frequency pre-compensation needs further discussion in RAN1. We think phase discontinuity can be pre-compensated by UE implementation. |
| ZTE | Yes | DMRS bundling should be performed within same segment to avoid phase discontinuity. That is, the pre-compensated TA is thought fixed during DMRS bundling time window. This is not contradicted with ‘RAN4 has agreed that TA adjustment should be avoided across the PUSCH/PUCCH transmissions (i.e., from start of first transmission until the end of last transmission) for joint channel estimation’. Hence, the LS may not be needed. |
| Ericsson | Yes | The LS to RAN4 is not needed. |
| LG | Yes | We think the LS to RAN4 is not needed. |
| Panasonic | No | Agree with QC. |
| CATT | Yes | The LS to RAN4 is not needed. |
| MediaTek | No | It can be further discussed in the context of TDW. Our understanding is that update of UL time/frequency pre-compensation can be done while maintaining power consistency and phase continuity depending on UE capability. The LS to RAN4 is premature. |

      - 1st round summary

FL found that several companies supporting proposal 2-1 have doubt of necessity of proposal 2-2 and no proposal is shared from the companies. Therefore, FL thinks that firstly what kind of RAN1 enhancement is necessary should be asked. At least they can be listed in this meeting, and further discussion would be done in future meeting. The question will be triggered in 2nd round.

      - 2nd round

Proposal 2-2_v1

For PUSCH DMRS bundling, if RAN1 concludes enhancement is necessary,

- Discuss which option(s) should be supported

  - Option 1: Support segmented UL time/frequency pre-compensation

  - Option 2: Others

Q: Which option should be taken? For option 2, please share what kind of RAN1 enhancement should be introduced.

| Company | Option | Comment |
| --- | --- | --- |
| Apple | 1 |  |
| Panasonic |  | “segment” needs to be clarified, especially relation with TDW. What is the difference between segment and TDW? UE is just required to transmit PUSCH with keeping power and phase continuity as defined in RAN4 requirement within TDW. It is not clear if segment is needed on top of TDW. We think TDW as specified in Rel.17 should be the baseline. If TDW is not sufficient, we can discuss other approaches. |
| Samsung |  | We think that the source of divergence in the 1st round discussion is that we don’t know exact reason which aspect current specification cannot work for DMRS bundling in NTN network. Please note that we observed there are two camps: one camp think that UE SHOULD pre-compensation during actual TDW for DMRS bundling, while the other camp think that UE SHOULD NOT pre-compensation during actual TDW for DMRS bundling. We need to discuss first this point. |
| OPPO | 1 |  |
| LG | 1 | We think the UL segments introduced in Rel-17 IoT NTN can be a baseline. |
| ZTE | 1 |  |
| Lenovo | 1 |  |
| NTT DOCOMO | 1 |  |
| Xiaomi |  | We think TDW as specified in Rel.17 should be the baseline. |
| vivo |  | In our understanding, when to perform UL time/frequency pre-compensation could be UE implementation, which is not specified yet. Whether to support segment transmission based enhancement from Rel-17 IoT NTN seems to be a separate enhancement, which is not coverage enhancement. We do not think this kind wording of ‘segmented UL time/frequency pre-compensation’ is clear. Whether to adopt segmented UL time/frequency pre-compensation within a TDW needs further discussion. We propose to simply treat the update of UL time/frequency pre-compensation as an event which causes power consistency and phase continuity not to be maintained. We suggest to add a new option as followed: - Option 3: Support to define the update of UL time/frequency pre-compensation as an event violates power consistency and phase continuity |
| CMCC |  | Considering the impact of time and frequency pre-compensation, it should be regarded as a event which cause power consistency and phase continuity not to be maintained when the UE changes its time and/or frequency pre-compensation. |
| Huawei, HiSilicon | 2 | In R-17 CE for NR, it is agreed to do pre-compensation update per actual TDW(s). Thus, we should follow the R-17 EC for NR framework to discuss whether new event should be introduced that determines the actual TDW or nominal TDW, rather than introducing segments and then discuss the segmental UL time/frequency pre-compensation. Proposal 2-2_v1 For PUSCH DMRS bundling, if RAN1 concludes enhancement is necessary, Discussion new events (e.g. time/frequency pre-compensation, antenna switching etc.) which determine the actual TDW(s). |
| Baicells | 1 |  |
| CATT | 2 | Need to clarify the relationship between TDW and segment. |
| MediaTek | 2 | We share understanding with other companies on need for further discussion. FL proposal seem to pre-empt conclusion on enhancements as discussed in previous section. We share Samsung view that the main issue for discussion is whether UE SHOULD pre-compensation during actual TDW for DMRS bundling. This could be also discussed for new event as commented by Huawei. We think Huawei proposed text is helpful. |
| QC |  | The proposal is unclear. Segmented pre-compensation is supported as long as existing requirements are satisfied. We can focus on proposal 4.2-1. |
| Nokia, Nokia Shanghai Bell |  | We believe, first, the existing aspects of TDW feature introduced in Rel17 should be considered as the baseline. Other options can be considered after assessing the current TDW limitations, if any exist. |
| Ericsson | 1 or 2 | We are not sure what the difference is between segmented pre-compensation and a solution based on the TDW framework, if any. Further discussion is needed. |

      - 2nd round summary

Based on companies’ inputs, it would be clear that we should focus on section 4.2.1 first. At least it would be impossible to list enhancement options in this meeting; therefore, this section was closed.

    - [Closed] Others

There might be other discussion topics for this PUCCH repetition but might not. FL prepares a box to share companies’ views on further topics.

      - 1st round

Q: Please share it if you think there is another discussion topic relevant to DMRS bundling for PUSCH. If other companies tend to agree/disagree some suggestion from other company, please input your company name in either of the right two columns.

| Company | Which topic should be discussed? | Supporting company | Not supporting company |
| --- | --- | --- | --- |
| Huawei, HiSilicon | We have the following proposals, and antenna switching is very useful technique to improve the coverage of NTN. Therefore, we prefer to consider the case where DMRS bundling is jointly used with antenna switching. This is not a new topic but it should be discussed in section 4.2.2. Proposal 3: Introduce antenna switching as an event that triggers the “actual TDW determination” for DMRS bundling to minimize the coverage gaps of PUSCH VoIP by jointly utilization of antenna switching and DMRS bundling. |  |  |

      - 1st round summary

HW proposed to discuss antenna switching together with PUSCH DMRS bundling. FL is not sure this direction is really necessary and any other companies have interests.

FL will prepare a box to share further view on this topic or any others.

      - 2nd round

Proposal 2-3_v0

For PUSCH DMRS bundling, if RAN1 concludes enhancement is necessary,

- Consider antenna switching

Q: Do you agree to consider antenna switching?

| Company | YES/NO | Comment |
| --- | --- | --- |
| Apple | No | Antenna switching itself is not associated with PUSCH DMRS bundling. Also, we are not sure whether antenna switching will change the phase continuity, such as X polarized antenna. |
| Samsung | No |  |
| OPPO | No |  |
| LG | No | We are not sure the antenna switching is beneficial for PUSCH DMRS bundling. |
| ZTE | No |  |
| Lenovo | No | We think we should focus on the case without antenna switching firstly. |
| NTT DOCOMO | No |  |
| vivo |  | In our view, antenna switching is not NTN specific issue on DMRS bundling. Is this antenna switching event expected in TN as well? |
| Huawei, HiSilicon | Yes | @LG, if antenna switching is not used, the coverage gap of PUSCH can not be eliminated even if DMRS bundling is used. @Apple, are discussing uplink transmit antenna switching, where UE uses @vivo, TN may not need antenna switching to achieve the coverage. NTN coverage cannot fulfill the coverage if without antenna switching. Therefore, DMRS bundling should consider to enable the possibility to be used jointly with antenna switching. |
| Baicells |  | Antenna switching does associated with PUSCH DMRS bundling. If segmented DMRS bundling can solve this issue, we may left antenna switching to UE implementation. |
| CATT | NO |  |
| Thales | No |  |
| MediaTek | Yes | Ok to discuss |
| QC | Yes | Antenna switching is always supported. In our design of DMRS bundling, benefit and support of antenna switching should be considered. |
| Nokia, Nokia Shanghai Bell | No | In the Huawei contribution the results presented are considering 4 tx antennas, which is not in line with the current agreed simulation assumptions. Further, even when considering one UE antenna with two polarizations that may potentially support some kind of antenna switching the UE would most likely anyway be suffering from poor link budget (as this would most likely occur in NLOS situations). We do not see any added benefit of considering TX antenna switching. |
| Ericsson | No | Ok to discuss antenna switching but we see no need for this proposal. |

      - 2nd round summary

Although majority believe that there is no need to discuss antenna switching for DMRS bundling, several companies think it can be discussed further. Only one company showed simulation results on DMRS bundling and provided details on the issue; thus, FL recommends studying further on this mechanism in future. Dedicated proposal would be unnecessary in this stage, so this section was closed.

- Contribution summary

  - Link budget calculation

- 1 PRB

  - -7.6: [5/vivo]

  - -8.04: [3/HW, HiSi]

  - -8.1: [6/ZTE] [7/OPPO] [16/Ericsson] [19/Pana]

  - -8.2: [21/DCM]

- 2 PRBs

  - -10.6: [2/MTK]

  - -11.6: [4/Spreadtrum]

  - -11.1: [6/ZTE] [7/OPPO] [16/Ericsson]

- 6 PRBs

  - -15.9: [7/OPPO]

  - PUCCH enhancements for Msg4 HARQ-ACK

- Terminology

  - [17/Samsung] Observation 1: Support of PUCCH repetitions in Rel-18 NTN is about the case where a UE does not have dedicated PUCCH resources.

    - One aspect to note is about the terminology and the framework. “Msg4” is not defined in the specifications and the PDSCH providing contention resolution is not necessarily the first PDSCH after Msg3 PUSCH is correctly received. Also, the issue is applicable in general until a UE is provided dedicated PUCCH resources which is also an optional configuration (i.e. a NW is not required by the specifications to provide dedicated PUCCH resources to a UE).

  - [21/DCM] Proposal 1: Enable to apply repetition to PUCCH transmission when the UE does not have dedicated PUCCH resource configuration.

    - Observation 1: There is a case where UE performing PUCCH transmission for Msg4 HARQ-ACK has dedicated PUCCH resource configuration.

- Enhancement mechanism

  - Repetitions: [2/MTK] [3/HW, HiSi] [5/vivo] [6/ZTE] [7/OPPO] [8/CATT] [9/Intel] [11/xiaomi] [12/CMCC] [13/ETRI] [15/Apple] [16/Ericsson] [17/Samsung] [18/Baicells] [19/Pana] [20/LGE] [22/QC] [24/Nokia, NSB]

- Details of each potential enhancement

  - For repetitions

    - Repetition factor

      - X (FFS): [2/MTK]

      - 2: [6/ZTE] [16/Ericsson (1T2R)]

      - Max 4: [5/vivo] [7/OPPO] [8/CATT (1T2R)] [21/DCM]

        - [8/CATT]: PUCCH with 4 symbols for Msg.4 HARQ-ACK requires SNR about -12.5dB to achieve a BLER 10-2 and can meet requirements of link budget by 4 reptitions. PUCCH with 14 symbols for Msg.4 HARQ-ACK requires SNR about -12.7dB to achieve a BLER 10-2 and can meet requirements of link budget by 2 reptitions.

      - Max 8: [15/Apple] [19/Pana] [21/DCM]

      - Max 12: [3/HW, HiSi]

    - Necessity determination

      - Based on SSB RSRP measurement: [15/Apple]

      - UE measurement e.g. the geometry or location [19/Pana]

    - Indication

      - Discuss: [5/vivo]

      - UE-specific indication

        - By RAR UL grant: [12/CMCC] [22/QC]

          - [22/QC] CSI request bit

        - By DCI for Msg4 PDSCH: [9/Intel] [13/ETRI] [15/Apple] [17/Samsung] [18/Baicells] [19/Pana] [20/LGE] [21/DCM]

          - [15/Apple] [18/Baicells] [20/LGE]: With update of cell-specific configuration

          - [17/Samsung]: DAI field, PRI, HPN, MCS

          - [19/Pana]: PRI, PDSCH-to-HARQ, TPC

          - [17/Samsung] [18/Baicells]: new field

          - [21/DCM]: candidates are configured commonly or per PUCCH resource

        - By linkage with PRACH/Msg3 PUSCH: [9/Intel] [12/CMCC]

      - Cell-specific indication

        - [7/OPPO] [11/xiaomi] [17/Samsung] [21/DCM]

        - By SIB: [8/CATT] [13/ETRI] [20/LGE]

        - By pre-definition: [8/CATT]

      - Determined by UE: [19/Pana (not preference)] [21/DCM (not preference)]

    - UE request / repetition capability report

      - Discuss: [5/vivo] [24/Nokia, NSB]

      - By dedicated PRACH preamble or dedicated PRACH occasion: [12/CMCC] [15/Apple] [19/Pana]

      - Simultaneous trigger with Msg3 repetition

      - By using DMRS port 1 in Msg3 TX: [22/QC]

      - By using different CS for DMRS in Msg3 TX: [22/QC]

      - By using a reserved LCID codepoint in Msg3 TX: [22/QC]

    - FH

      - Discuss: [5/vivo]

      - Support inter-slot FH: [15/Apple]

      - Not support inter-slot FH: [9/Intel]

    - Repetition slot counting

      - Discuss: [5/vivo]

      - Based on available slot: [9/Intel]

    - Others

      - Support DMRS bundling for PUCCH of Msg4 HARQ-ACK: [6/ZTE] [15/Apple] [18/Baicells]

        - No: [19/Pana]

          - [19/Pana] Regarding the gain of DMRS bundling, almost no gain is seen for ACK miss detection performance. 2.6 dB and 2.3 dB gains are seen for NACK to ACK error performance of 1T1R and 1T2R, respectively. Because ACK miss detection performance is more critical, DMRS bundling would not help for better coverage of PUCCH for Msg4 HARQ-ACK.

      - Same PUCCH resource is used for all PUCCH repetitions [9/Intel]

      - Same Tx beam is applied for all the repetitions [9/Intel]

      - Sub-slot-based repetition: [18/Baicells]

      - dynamic PRB offset indication: [18/Baicells]

      - ‘pucch-ResourceCommon’ indicates 11 to 15 if repetition is configured: [20/LGE]

      - New table only with 14-symbol PUCCH: [20/LGE]

  - DMRS bundling for PUSCH taking into account NTN-specifics

- Required window size

  - [3/HW, HiSi] Observation 7: For PUSCH VoIP, setting configured TDW larger than 10 could not achieve additional DMRS bundling gain considering the time budget of 20ms for a single PUSCH VoIP packet.

  - [3/HW, HiSi] Observation 8: For PUSCH VoIP, DMRS bundling could provide a maximum 1.3 dB gain for the case when antenna switching is disabled.

  - [16/Ericsson (1T2R)] Observation 4: It is possible to meet the target CNR for PUSCH VoIP with segmented DMRS bundling when the segment length is limited to 7 slots.

  - [19/Pana] Observation 1: TDW length up to 10 slots is sufficient for PUSCH VoIP

- Necessity of RAN1 enhancement

  - Observations

    - [2/MTK] Proposal 2: Re-use DM RS bundling enhancement in Rel-17 NR coverage enhancement for NR NTN with maximum DM RS bundling window length to 8 ms, 16 ms, and 32 ms for LEO, MEO, and GEO respectively.

      - A maximum of 100 us/s delay drift is assumed based on TR 38.821. The UL transmission time is upper bounded by the Rel-17 NR NTN timing error requirement of 29.Ts ≈ 0.928 us, with 8 ms, 16 ms and 32 ms for LEO, MEO, and GEO respectively as highlighted in “green text” in Table1.

    - [2/MTK] Observation 2: For PUSCH and PUCCH with repetitions, the legacy NTN UE behaviour is to apply TA and Doppler shift correction on a slot basis.

      - UE pre-compensation during UL transmission segment depends on Phase discontinuity Δφ which occurs when UE pre-compensation of satellite delay is applied via sample skip at subframe boundary. The maximum phase discontinuity value can be determined as the product of the maximum satellite drift 0.1us/ ms (or 100 us/s) and sampling frequency Fs=Δf/2 as shown in Table 3. ∆∅=∆Tdrift×Fs×360 degrees

      - The phase discontinuity due to applying TA after one or several repetitions can be very significant for large UL bandwidth as shown in Table 2. The analysis would suggest that for large UL bandwidth and large TA drift, the phase discontinuity has to be pre-compensated by the UE before applying the TA in case of repetitions.

    - [3/HW, HiSi] Observation 6: To comply with the 29Ts RAN4 timing error requirement, the maximum configured TDW should be within 13 slots for the LEO-1200 satellite with an elevation angle of 30 degrees in NR NTN.

    - [6/ZTE] When 2 RB is used for transmission, the maximum phase discontinuity for edge carrier (locates at 180 kHz or 180 kHz) after 20ms transmission is 0.1*20*180*360 = 130degree. As a result, the DMRSs in different segments may have different phase properties and JCE performance will decrease due to bundling or DMRSs without good coherence.

    - [8/CATT] In terms of NTN specific scenarios, according to the parameters of NTN scenarios from TR38.821[3], as shown Table 11, maximum Doppler is 24ppm. When 2GHz is applied, the maximum Doppler shift is 48KHz. maximum Doppler shift variation is 554Hz/s, the duration of 100ms can cause maximum Doppler shift of 55.4Hz. For LEO-600, maximum timing shift variation is 50us/s, however, the normal CP is 4.68us at 15kHz SCS. If the duration of UL repetition transmission for one TB is over 40ms, maximum timing shift can more than 2us, which can cause performance reduction because of timing shift. According to the analysis above, frequency shift has almost no effect on synchronization and timing shift can cause performance reduction, so segment time-frequency pre-compensation should be applied at UL repetition transmission.

    - [9/Intel] Proposal 6: The following UE implementations can be further considered by RAN1:

      - Time/frequency shift is fixed for the duration corresponding to the DMRS bundling size

      - Time/frequency shift is constantly changed on every symbol without abrupt phase changes

    - [10/Sony] Proposal 1: RAN1 should study the feasibility and mechanism to ensure the UE can maintain the phase and amplitude continuity while performing TA and Doppler pre-compensation within each DMRS bundle.

    - [11/xiaomi] Proposal 2: RAN1 study the factors would impact the length of TDW in NTN considering UE’s pre-compensation on Doppler shift and open-loop TA.

      - In addition to the phase noise caused by electronic devices, the phase shift would also be impacted by the Doppler shift which is expected to much larger than the one in TN system. The phase shift can be calculated as follows:φ=2π*f_Doppler*t, where φ is the phase shift, f_Doppler is the Doppler shift and t is the time duration. In NTN, the Doppler shift is much larger than TN, and the value of Doppler shift varies with the elevation angle between UE and satellite. Assuming that the SCS=15KHz, the satellite altitude is 600 km and the carrier frequency is 2GHz, when the elevation angle is 90 degree, the phase difference between slot p-1 and slot p is around 34 degree if no frequency pre-compensation is applied. While if the elevation angle is 30 degree, the phase difference between slot p-1 and slot p is around 17 degree if no frequency pre-compensation is applied. In this sense, the length of the time window that UE can maintain phase continuity is varying with change of the elevation angle.

    - [16/Ericsson] Observation 3: With a timing drift of 84 ppm and an optimistic assumption on the efficiency of closed-loop TA control, the UE has to update its autonomous TA pre-compensation every 8 slots to avoid excessive timing offset. With a less optimistic assumption on the efficiency of closed-loop TA control, more frequent TA updates are needed.

      - The timing accuracy requirement for NTN UE is 29Ts (assuming 15 kHz SCS) (see clause 7.1c.2 of TS 38.133 [2]).

      - For 15 kHz SCS, the granularity of TAC is 8Ts (see clause 4.3 of TS 38.213 [3]), which means that the timing error could ideally be cancelled to within ±4 Ts. The tolerance of TA updates is 4Ts (see clause 7.3C.2.2 of TS 38.133 [2]). Therefore, (29-4-4)Ts = 0.684 µs can be assumed to be available to handle the timing drift. With a timing drift of 84 ppm, the timing error will exceed 0.684 µs in 0.684/84 s = 8.1 ms.

    - [17/Samsung] Observation 2: For PUSCH repetition of 4, the maximum difference of delay would be less than 0.1us and the maximum difference of Doppler shift would be less than 1Hz.

    - [17/Samsung] Observation 3: For PUSCH repetition of 20, the maximum difference of delay would be equal to 0.4us and the maximum difference of Doppler shift would be less than 5Hz.

    - [17/Samsung] Observation 4: Around 90 degrees, the impact of Doppler shift difference is much larger than the impact of delay difference. It is the opposite around 10 or 170 degrees.

    - [19/Pana] Observation 2: To keep the phase continuity during TDW length for PUSCH VoIP would be feasible.

      - Figure 4 shows time and frequency change rate for LEO 600 km. It is assumed that the satellite orbit is along with the equator and UE is located on latitude 0 degree. Curves with various gateway latitude (0-15 degrees) are shown in the figures. The maximum timing change rate is around 42 us/s and the maximum Doppler frequency change rate is 0.57ppm/s (i.e. 0.57 x 10-6 x 2GHz / s). It means that propagation delay is changed at most 42 ns per slot for SCS 15kHz and the Doppler frequency is changed at most 0.57 x 10-3 ppm per slot.

      - According to the timing error requirement shown in Table 1, timing error limit is 945ns (=29 x 64 x Tc) for SCS 15kHz. Therefore, the longest update period of TA to meet the timing error limit is 22.5 slots (=945ns/42ns). The longest update period of frequency to meet 0.1 ppm frequency requirement is 175 slots  (=0.1/(0.57x10-3).

    - [21/DCM] Observation 5: There seems to be no significant problem in not updating the timing/frequency pre-compensation values for 20 ms.

      - Proposal 5: UE should not update timing/frequency pre-compensation values with phase discontinuities within each actual TDW.

      - Observation 3: The receive timing deviation at gNB due to not updating the transmit timing pre-compensation value for 20 ms is less than 0.95 us.

      - Observation 4: The change in doppler shift over 20 ms is less than 1 Hz.

    - [22/QC] Observation 1: To meet the existing phase continuity requirement for DMRS bundling, the bundling window is less than 2 ms for 10 MHz carrier bandwidth if time and frequency precomepnsation remains unchanged during the window.

      - The above indicates that without additional time and frequency precompensation the UL transmit timing and frequency error can increases by 50 ns and 0.64 Hz per 1 ms.  Assuming perfect time and frequency precompensation at the beginning of a multi-slot UL transmission, a UE does not have to adjust the timing and frequency for a number of slots to satisfy the relevant performance requirements on transmit signal [3][4] without considering DMRS bundling.

      - With a timing drift rate 50 ns/ms, the phase change after x slots with 15 kHz SCS is =x*50e-9*N*15e3 (rad) where N is the tone index with DC tone indexed as 0. When N=300 and x=1, =0.225 or 12.9 degree. Apparently, when not compensated the timing drift within a multi-slot transmission due to satellite movement can have significant impact on DMRS bundling size.

    - [22/QC] Observation 2: DMRS bundling is feasible even when there are large phase variation due to predictable satellite movements.

      - Phase variation due to timing drfit are predictable and DMRS bundling is still possible. For UEs within a cell, the timing drift rates are typically very close and can be approximated as a constant for all UEs. Consequently, DMRS bundling is still possible. For instance, gNB can post-compnesate the phase variation by simply adjusting the FFT starting time or by subcarrier-wise phase rotation at gNB.

    - [23/Lenovo] Proposal 2: There is no impact of large propagation delay to joint channel estimation window.

    - [24/Nokia, NSB] Observation 1: In NR NTN, the 0.1 PPM frequency offset leads to phase difference of 72 degress and 36 degress for SCS 15 kHz and 30 kHz between any two consecutive slots, respectively, which is larger than the maximum allowable phase difference in DMRS bundling.

      - On the other hand, in NTN, the residual frequency offset of 0.1 PPM after UE-precompensation takes into account not only the effect of misalignment between the transmitter and receiver local ocillators but also the residual error from UE estimation and precompensation of doppler shift in the service link, which changes continuously due to both satellite and UE movements. This creates a challenge to the receiver frequency offset estimator, which cannot rely on an averaged estimate of the frequency offset based on multiple observations, especially important in cases of coverage shortage wherein the SNR is very low and the single shot estimates are less reliable. For this reason, in the following analysis we will consider a residual frequency offset of 0.1 PPM for an NTN scenario to understand its implications on DMRS bundling across PUSCH repetitions.

      - In NR NTN, currently, it is assumed that the residual frequency offset after pre-compensation is 0.1 PPM, deriving from the combined inaccuracies at the UE local oscillator and the UE autonomous time and frequency pre-compensation based on UE position and serving satellite ephemeris information. This maximum frequency offset from 0.1 PPM in FR1 and for a carrier frequency of 2 GHz is translated into 200 Hz. Now, taking 200 Hz, in 1 ms, such frequency offset leads to the phase difference of ΔΦ=2π×200×10^(-3)=1.2566 rad, or 72 degrees across DMRS symbols between neighboring slots. In other words, in every 5 ms the constellation points are rotated by 360 degrees. Taking the above discussion into account, when DMRS bundling is applied across “n” slots, one would expect the expansion of the maximum phase offset by n×72 degrees for SCS 15 kHz and n×36 for SCS 30 kHz. Clearly, this phase discontinuity is larger than the maximum allowable phase difference of 25 degrees for any two consecutive slots, i.e., slot “p-1” and slot “p” wherein DMRS bundling is applied.

  - Necessary?

    - Yes: [2/MTK] [3/HW, HiSi] [5/vivo] [6/ZTE] [8/CATT] [15/Apple] [16/Ericsson]

    - No: [19/Pana] [21/DCM]

    - Send an LS to RAN4: [17/Samsung] [22/QC]

      - [17/Samsung] with the following questions

        - Whether or not applied TA value can be changed over PUSCH repetitions for DMRS bundling in NTN?

          - If applied TA value cannot be changed, what is the maximum duration?

          - If applied TA value can be changed, how much difference can be tolerable and what is the maximum duration?

        - Whether or not applied Doppler shift can be changed over PUSCH repetition for DMRS bundling in NTN?

          - If applied Doppler shift value cannot be changed, what is the maximum duration?

          - If applied Doppler shift value can be changed, how much difference can be tolerable and what is the maximum duration?

      - [22/QC] Proposal 3: RAN1 to decide if DMRS bundling with post-compensation at gNB is supported. If supported, RAN1 asks RAN4 to consider associated requirement for DMRS bundling for segmented precompensation.

- Enhancement mechanism

  - DMRS bundling w/ maintaining power consistency and phase continuity: [2/MTK]

  - Introduce antenna switching as an event that triggers the “actual TDW determination”: [3/HW, HiSi]

    - [3/HW, HiSi] Observation 4: For eMBB UEs, at least 4 receiver antennas are usually equipped to support the commercialized NR band including N41, N77 and N78.

    - [3/HW, HiSi] Observation 5: About 2.3dB gain is observed by utilizing antenna switching for PUSCH of VoIP without DMRS bundling.

    - [3/HW, HiSi] Observation 10: The optimized DMRS bundling size with the best performance should be decided based on the trade-off between the antenna switching diversity gain and the joint channel estimation gain, considering the antenna switching cannot be applied within a DMRS bundling.

  - To update common TA/UE-specific TA is newly defined as an event which cause power consistency and phase continuity not to be maintained: [5/vivo] [12/CMCC] [23/Lenovo]

  - PUSCH segmented pre-compensation: [6/ZTE] [7/OPPO] [8/CATT] [15/Apple] [20/LGE]

    - [15/Apple] This mechanism is similar to uplink segmented transmission introduced in Rel-17 IoT NTN. In IoT NTN, a UE does not adjust its TA value and frequency offset value within an uplink segmented transmission duration.

  - Configure a frequency hopping interval: [18/Baicells]

- Details of potential enhancement

  - For segmented pre-compensation

    - Candidate value

      - 2/4/8/16 ms: [8/CATT]

    - Indication

      - SIB: [8/CATT] [15/Apple]

      - Dedicated RRC signaling: [8/CATT] [15/Apple]

    - Unit

      - Slot: [15/Apple]

    - TDW size vs segmented TX

      - TDW size (if configured) is upper bounded by segmented TX duration: [15/Apple] [20/LGE]

      - TDW size (if not configured) equals to min of {max supported duration, segmented TX duration, repetition duration, FH interval}: [15/Apple]

    - Gap between segments

      - No gap: [9/Intel]

        - At the last RAN1 meeting it was agreed to support UE capability on the gaps for segmented UL transmission for both eMTC and NB-IoT NTN. The gaps may be required in order to allow low-complexity implementation for adjustment of time/frequency shift for UL transmission at the UE. Considering that NR devices allow higher UE complexity comparing to eMTC/NB-IoT devices, there is no need to support additional gaps to adjust the UE pre-compensation values.

  - Others

- [4/Spreadtrum]: Proposal 1: Coverage enhancement is required for PRACH transmission in LEO-1200 scenarios.

  - Proposal 3: Beam-level repetition value configuration of PRACH can be considered.

  - Proposal 4: Repetitions enhancements for 2-Step RACH should be considered in R18 for smart phones in NTN.

- [5/vivo] Proposal 4: Do not support RAN protocol overhead reduction for NTN coverage enhancement in NR Rel-18.

- [5/vivo] Proposal 5: Circular polarization enhancement on Tx diversity could be studied for downlink coverage enhancements in NR NTN if needed.

- [6/ZTE] Proposal 3: Other enhancements including 2Tx, high power UE and polarization matching should also be supported to improve the performance for VoIP on PUSCH.

- [8/CATT] Study polarization diversity to enhance the UL coverage of NR NTN.

- [10/Sony] Proposal 2: RAN1 should enhance the polarization support for Rel-18 to improve the coverage of NTN.

- [10/Sony] Proposal 3: RAN1 can study how network can obtain the UE capabilities on supported polarization modes in Rel-18.

- [10/Sony] Proposal 4: RAN1 should study inter-user multiplexing over the polarization domain in Rel-18.

- [10/Sony] Proposal 5: RAN1 should study polarization indication per beam in Rel-18.

- [11/xiaomi] Proposal 3: The normative work in Rel-18 CE WI should take the coverage gap in NTN scenario into account.

- [13/ETRI] Proposal 2:  User equipment including power class 2 (HPUE) and power class 3 (handset) can transmit HARQ by increasing the average or instantaneous transmission power.

- [14/NEC] Proposal-1: At least two sets of RACH resources are required to be configured with different RACH formats or repetitions, to allow both VSAT UEs and commercial UEs gain access to NTN cell

- [14/NEC] Proposal-2:  Discuss how to configure multiple RACH resource sets per cell for initial random-access procedure

- [23/Lenovo] Proposal 1: Updated K-offset MAC CE is applied at the start of the first repetition of an uplink channel.

- [23/Lenovo] Proposal 4: Application of updated K-offset is only at the start of the joint channel estimation window.

- [23/Lenovo] Proposal 5: Study the impact of polarization loss on the coverage of initial access procedure

- [23/Lenovo] Proposal 6: Study the scenario where gNB adopts different polarization modes to serve UEs with different polarizations modes at different time instances.

- [23/Lenovo] Proposal 7: Study the association between polarization mode and RS.

- [23/Lenovo] Proposal 8: Study the polarization based scheme for uplink initial access channels.

- [23/Lenovo] Proposal 9: Study the impact of CSI prediction on system performance in NLOS channel conditions.

- [22/QC]: Proposal 4: RAN1 asks RAN4 to consider extending the required time window for coherent MIMO to accommodate the round trip delay in LEO.

- [22/QC]: Proposal 5: Support signaling of the polarization relationship between two transmit antenna ports: same polarization vs cross-polarized.

- References

- R1-2208388	Discussion on NR NTN coverage enhancement	THALES

- R1-2208395	Coverage enhancement for NR NTN	MediaTek Inc.

- R1-2208435	Discussion on coverage enhancement for NR NTN	Huawei, HiSilicon

- R1-2208567	Discussion on coverage enhancements for NTN	Spreadtrum Communications

- R1-2208662	Discussions on coverage enhancement for NR NTN	vivo

- R1-2208693	Discussion on coverage enhancement for NTN	ZTE

- R1-2208834	Discussion on coverage enhancement for NR NTN	OPPO

- R1-2208954	Discussion on UL coverage enhancement for NR NTN	CATT

- R1-2209071	On coverage enhancement for NR NTN	Intel Corporation

- R1-2209114	On coverage enhancement for NR NTN	Sony

- R1-2209264	Discussion on coverage enhancement for NR-NTN	xiaomi

- R1-2209356	Discussion on coverage enhancement for NR NTN	CMCC

- R1-2209411	Discussion on coverage enhancement for NR NTN	ETRI

- R1-2209422	Discussion on coverage enhancements aspects for NR NTN	NEC

- R1-2209599	On Coverage Enhancement for NR NTN	Apple

- R1-2209656	On coverage enhancements for NR NTN	Ericsson

- R1-2209750	On coverage enhancement for NR NTN	Samsung

- R1-2209768	Discussion on coverage enhancement for NR NTN	Baicells

- R1-2209796	Discussion on coverage enhancement for NR-NTN	Panasonic

- R1-2209802	Discussion on coverage enhancement for NR NTN	LG Electronics

- R1-2209921	Discussion on coverage enhancement for NR NTN	NTT DOCOMO, INC.

- R1-2210004	Coverage enhancements for NR NTN	Qualcomm Incorporated

- R1-2210023	Discussion on coverage enhancement for NR NTN	Lenovo

- R1-2210049	Considerations on coverage enhancements for NR over NTN	Nokia, Nokia Shanghai Bell

- Appendix-1 (Copy from WID RP-222654)

4.1.1 Coverage enhancement The Rel-18 NTN objectives are focused on the applicability of the solutions developed by general NR coverage enhancement to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement. Only NTN-specific characteristics are to be included in this coverage enhancement work, otherwise it should be part of another WI (e.g., UL enhancement of coverage). The following sentence will be revisited in RAN#99 as part of the DL enhancements discussion: “The evaluation should also take into account any related regulatory requirements, e.g., ITU limitation of power flux density.” No work on this topic will take place in RAN WGs before the discussion on DL enhancements in RAN#99. The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarisation loss (per antenna port). Note: It is understood that the enhancements defined for LEO can also apply to GEO and MEO scenarios as appropriate. No additional work is expected for MEO/GEO. The targeted services are VoIP using AMR 4.75 kbps and data transmission services with Low data rate of 3 kbps. The detailed objectives are for NTN: - To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g. repetition) [RAN1, RAN4] - To study DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) and, if necessary, specify enhancements to the Rel-17 procedures [RAN1]

- Appendix-2 (Outcomes of post meetings)

  - RAN1#109-e

Agreement

For NR NTN coverage enhancement, evaluate only handset terminals as UE type.

- i.e., VSAT is not considered.

Agreement

Coverage performance in NR NTN is evaluated according to the following steps.

- Step 1: CNR is calculated as defined in 6.1.3.1 of TR38.821

  - For polarization loss,

    - 3 dB polarization loss is assumed as baseline, and companies are encouraged to report the value and corresponding justification if other value is used

- Step 2: Required SNR of target service is evaluated by LLS

- Step 3: The CNR and the required SNR are compared

Agreement

Coverage performance in NR NTN is evaluated for GEO/LEO-1200/LEO-600 scenarios.

- Note: Service type for each scenario is discussed separately

- Note: Parameter set (Set-1/2) is discussed separately

- Note: MEO can be evaluated optionally

Agreement

For evaluation of coverage performance in NR NTN,

- It is assumed that carrier bandwidth is sufficiently large to transmit each channel.

- Companies are encouraged to report BWP bandwidth, when necessary (e.g. for frequency hopping).

- Note: each channel bandwidth is discussed separately.

Agreement

For VoIP, AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) with 20 ms data arriving interval is used in the evaluations.

- Each packet is transmitted within 20 ms, if packet combining is not used.

  - Companies are encouraged to evaluate at least packet transmission without combining

  - Companies are encouraged to report how to apply packet combining, if used.

  - Note: in packet combining, two packets can be combined into a single packet at TX side

    - Companies should report the impact on E2E latency

- VoIP is evaluated only in LEO scenario.

- Note 1: PRB/MCS/TBS determinations are discussed separately

- Note 2: companies should report if HARQ is used in the evaluations, and if evaluations depart from the assumption that each packet is transmitted within 20 ms

Agreement

Reuse Set-1/2 satellite parameters as in table 6.1.1.1-1/2 of TR38.821 for GEO/LEO-1200/LEO-600 and S-band, and as in table 6.1.1.1-1/2 of RP-220590 for MEO and S-band.

- In addition, evaluations assuming relevant ITU regulatory limitations on power flux density can be reported in the study phase.

  - Companies should report which value of EIRP density is used and corresponding justification.

Agreement

For link budget calculation, parameters in the following table is assumed.

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2 GHz for DL and UL (S-band) |
| Channel bandwidth | FFS |
| Satellite altitude | 600 km, 1200 km, 10000 km, 35786 km |
| Target elevation angle | [30 (LEO), 12.5 (GEO-Set 1) , 20° (GEO –Set 2), 30° (MEO)] |
| Atmospheric loss | Equation (6.6-8) in [2] |
| Shadowing margin | 3 dB |
| Scintillation loss | Section 6.6.6 in [2] Ionospheric loss: = 2.2 dB (note 1) Tropospheric loss: Table 6.6.6.2.1-1 of [2] |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Satellite antenna polarization | Circular polarization |
| Terminal type | [S band: (M, N, P) = (1,1,2)] |
| Free space path loss | Equation (6.6-2) in [2] |
| Terminal RF parameters | FFS |
| Satellite RF parameters | FFS |
| Polarization loss | As agreed separately |
| Outcome | CNR |
| - NOTE 1: Based on P3 curve for 1% of time from Figure 6.6.6.1.4-1 of [2] after frequency scaling. - dB - NOTE 2: [2] in this table is 3GPP TR 38.811 v15.2.0: "Study on New Radio (NR) to support non-terrestrial networks (Release 15)" |  |

Agreement

If corresponding channel (including SCS) is agreed as evaluation target channel, the following features introduced in Rel-17 Coverage enhancement WI can be applied in coverage evaluation of NR NTN.

- For VoIP, max 20 PUSCH repetitions if SCS = 15 kHz and packet combining/HARQ are not applied; otherwise, max 32 PUSCH repetitions with consideration of the impact on E2E latency

- For low-data rate service, max 32 PUSCH repetitions

- TBoMS

- Joint channel estimation (DMRS bundling)

  - Companies are encouraged to report how to apply

- Max 16 Msg.3 PUSCH repetitions

Agreement

For low-data rate service, the following target data rate is assumed.

- For DL, 3 kbps if satellite EIRP density lower than values in table 6.1.1.1-1/2 of TR38.821 for GEO/LEO-1200/LEO-600 and S-band, or values in table 6.1.1.1-1/2 of RP-220590 for MEO and S-band due to ITU regulatory limitations on power flux density is considered; otherwise, 1 Mbps

- For UL, 3 kbps and 100 kbps

  - FFS: which data rate applies for GEO/MEO/LEO

Agreement

For NR NTN coverage enhancement, the following channels/signals can be evaluated.

- PUSCH for VoIP

- PUSCH for low data rate service

- PUCCH format 1 with 2 bits

- PUCCH format 3 with 11 bits

- PRACH format 0

- PRACH format 2

- PRACH format B4

- PUSCH Msg.3

- PUCCH for Msg.4 HARQ-ACK

- SSB

- PDSCH for VoIP

- PDSCH for low data rate service

- PDSCH Msg.2

- PDSCH Msg.4

- PDCCH

- Broadcast PDCCH (PDCCH of Msg.2)

Agreement

Evaluate coverage performance for the following UE characteristics as in Table 6.1.1.1-3 of TR38.821 with update of polarization, Tx/Rx antenna gain, and antenna type and configuration.

| Characteristics | Handheld |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | 1 TX, 2TX (optional) / 2 RX with omni-directional antenna element Note: companies should provide their assumption on polarization |
| Polarisation | Linear |
| Rx Antenna gain | [X] dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | [X] dBi per element |

- X = -5 as working assumption

  - Send an LS to RAN4 to ask whether above antenna gain is valid and if invalid, appropriate value.

R1-2205622	[Draft] LS on UE antenna gain for NR NTN coverage enhancement	Moderator (NTT DOCOMO, INC.)

R1-2205623	LS on UE antenna gain for NR NTN coverage enhancement	RAN1, NTT DOCOMO, INC.

Final LS is endorsed in R1-2205623.

Agreement

For coverage performance evaluation, the following elevation angle is assumed.

- 30 deg for LEO, 12.5 deg for GEO-Set 1, 20 deg for GEO-Set 2, as in in Table 6.1.3.2-1 of TR38.821

  - Note: For GEO-Set 1, channel parameters for 10 deg is used in LLS.

- 30 deg for MEO

- Other elevation angles can be evaluated as optional

- Note: these values are elevation angles at the edge of the edge beam.

Agreement

For NR NTN coverage enhancement, evaluate the following cases.

| Case | Satellite orbit | Satellite parameter set | Elevation angle (deg) | Terminal | Frequency band | Service type |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | GEO | 1 | 12.5 | Handset | S-band | Low-data rate service |
| 2 | GEO | 2 | 20 | Handset | S-band | Low-data rate service |
| 3 (Optional) | LEO-1200 | 1 | 30 | Handset | S-band | VoIP |
| 4 | LEO-1200 | 2 | 30 | Handset | S-band | VoIP |
| 5 | LEO-1200 | 2 | 30 | Handset | S-band | Low-data rate service |
| 6 (Optional) | LEO-600 | 1 | 30 | Handset | S-band | VoIP |
| 7 | LEO-600 | 2 | 30 | Handset | S-band | VoIP |
| 8 (Optional) | LEO-600 | 2 | 30 | Handset | S-band | Low-data rate service |
| 9 (Optional, with higher priority than case 10) | MEO | 1 | 30 | Handset | S-band | Low-data rate service |
| 10 (Optional) | MEO | 2 | 30 | Handset | S-band | Low-data rate service |

Agreement

For coverage performance evaluation, the following are assumed for all channels/signals

- Channel model/Delay spread

  - Channel model as in Table 6.1.2-4 of TR38.821, assuming NTN-TDL-A (NLOS) and NTN-TDL-C (LOS)

- Evaluation scenario

  - Rural (LOS/NLOS)

  - Sub-urban (LOS/NLOS) (optional)

- Channel estimation: Realistic estimation

  - Companies are encouraged to report channel estimation method.

- SCS

  - 15 kHz only

- UE speed: 3 km/h

- Frequency drift: Not assumed

- Frequency offset: 0.1 ppm

Agreement

For coverage evaluation of PUSCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE transmit chains | 1, 2 (optional) |
| DMRS configuration | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| Waveform | DFT-s-OFDM, CP-OFDM (optional) |
| PUSCH duration | 14 OS |
| Repetitions | w/ type A repetition, optional for type B repetition. The actual number of repetitions is reported by companies. |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| PRBs/TBS/MCS for low data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | Any value of PRBs reported by companies will be considered in the discussion. QPSK, pi/2 BPSK (optional) |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PUCCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| PUCCH format | Format 1, 2bits UCI. Format 3, 11 bits UCI |
| Frequency hopping | w/ frequency hopping |
| BLER | - For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. - For PUCCH format 3: BLER for Ack/Nack, SR: 1% BLER for CSI: 1%, optional for 10%. |
| Number of UE transmit chains | 1 |
| DMRS configuration | Number of DMRS symbols for PUCCH Format 3: Reported by companies |
| Repetitions | w/ repetition. The maximum number of repetitions is 8. |
| PUCCH duration | 14 OS |
| Number of PRBs | 1 PRB |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PRACH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Format | Format 0, Format B4, Format 2 |
| SCS | Reported by companies. |
| Performance metric | 1% missed detection at 0.1% false alarm probability 10% missed detection: reported by companies if this value is used |
| Number of UE transmit chains | 1, 2 (optional) |
| Other parameters | Reported by companies. |

Agreement

For coverage evaluation of PUSCH Msg.3 in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| Number of UE transmit chains | 1, 2 (optional) |
| Number of DMRS symbol | w/o frequency hopping: 3, w/ frequency hopping: 2 for each hop |
| Waveform | DFT-s-OFDM |
| HARQ configuration | Whether/How is adopted is reported by companies. |
| PUSCH duration | 14 OS |
| Number of PRBs | 2 |
| TBS | 56 bits |
| Other parameters | Reported by companies. |

Agreement

For coverage evaluation of SSB in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 for 2GHz |
| Periodicity | 20ms |
| Performance metric | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index |
| Other parameters | Reported by companies. |

Agreement

For coverage evaluation of PDSCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Waveform | CP-OFDM |
| Number of UE receive chains | 2 for 2GHz |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| DMRS configuration | 3 DMRS symbols is used for PDSCH of Msg.2. For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| PRBs/TBS/MCS for low data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | Any value of PRBs reported by companies will be considered in the discussion. QPSK |
| PDSCH duration | 12 OS |
| Payload size for PDSCH of Msg.4 | 1040 bits |
| Other parameters | Reported by companies. |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PDCCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 for 2GHz |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | Reported by companies |
| BLER | 1% BLER optional for 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

  - RAN1#110

Conclusion

For Rel-18 coverage enhancement in NTN, NLOS environment is deprioritized.

Agreement

For NR-NTN coverage enhancement, RAN1 concludes that coverage enhancements specifically for GEO and MEO are de-prioritized in Rel-18.

- Potential enhancements for LEO can also apply to GEO and MEO

Agreement

For NR-NTN coverage enhancement in Rel-18, link budget of parameter set-1 for LEO-1200 operating at LOS is considered as the target to evaluate whether each channel/signal with the existing specification needs to be enhanced or not. The targeted performances are used to evaluate the following services:

- VoIP using AMR 4.75 kbps.

- Low data rate of 3 kbps.

- Potential enhancements for deployments with parameter set-1 can also apply for deployments for parameter set-2

Observation

For PUCCH format 1 with parameter set-1 for LEO-1200 operating at LOS,

- Five sources observed that the existing specification can meet the performance requirement

Conclusion

RAN1 concluded that enhancement is unnecessary for PUCCH format 1 with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation

For PUCCH format 3 with parameter set-1 for LEO-1200 operating at LOS,

- Six sources observed that the existing specification can meet the performance requirement

- One source observed that the existing specification cannot meet the performance requirement with at least 0.6 dB gap

Conclusion

RAN1 concluded that enhancement is unnecessary for PUCCH format 3 with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation

For PUCCH for Msg4 HARQ-ACK with parameter set-1 for LEO-1200 operating at LOS,

- One source observed that the existing specification can meet the performance requirement

- Three sources observed that the existing specification cannot meet the performance requirement with a gap of 1.8 to 6 dB.

Conclusion

RAN1 concluded that PUCCH for Msg4 HARQ-ACK should be enhanced to meet the coverage requirements for parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation

For PUSCH for low data rate of 3 kbps with parameter set-1 for LEO-1200 operating at LOS,

- Eight sources observed that the existing specification can meet the performance requirement

Conclusion

RAN1 concluded that enhancement is unnecessary for PUSCH for low data rate of 3 kbps with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation

For PRACH format 0 with parameter set-1 for LEO-1200 operating at LOS,

- One source observed that the existing specification can meet the performance requirement

- Eight sources observed that the existing specification cannot meet the performance requirement with a gap of 0.3 to 5.3 dB

For PRACH format 2 with parameter set-1 for LEO-1200 operating at LOS,

- Ten sources observed that the existing specification can meet the performance requirement

- Two sources observed that the existing specification cannot meet the performance requirement with a gap of 1.9 to 8.8 dB

For PRACH format B4 with parameter set-1 for LEO-1200 operating at LOS,

- Ten sources observed that the existing specification cannot meet the performance requirement with a gap of 1.2 to 11.9 dB

Note: for the observations above, some sources used 1 Rx antenna and some sources used 2 Rx antennas at the satellite.

Observation

For PUSCH for VoIP with parameter set-1 for LEO-1200 operating at LOS,

- Six sources observed that the existing specification can meet the performance requirement with a margin of 0 to 1.7 dB

  - One company simulated by using 20 repetitions without DMRS bundling

  - Four companies simulated by using 20 repetitions with DMRS bundling

  - One company simulated by using 32 repetitions with DMRS bundling

    - Note: this is the only result using frame combining by application layer

- Nine sources observed that the existing specification cannot meet the performance requirement with a gap of 0.3 to 8.6 dB

  - Eight companies simulated by using 20 repetitions without DMRS bundling

  - Seven companies simulated without frequency hopping

  - One company simulated by using 16 repetitions with DMRS bundling

Note: for the observations above, some sources used 1 Rx antenna and some sources used 2 Rx antennas at the satellite.

Observation

RAN1 concluded that enhancement for PUSCH for VoIP may be needed to meet the coverage requirements for parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain, when DMRS bundling is not applied.

Observation

For Msg3 PUSCH with parameter set-1 for LEO-1200 operating at LOS,

- Eight sources observed that the existing specification can meet the performance requirement

- One source observed that the existing specification cannot meet the performance requirement with a gap of 1.5 dB.

Conclusion

RAN1 concluded that enhancement is unnecessary for Msg3 PUSCH with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

- Appendix-3 (Contact information)

| Company | Name | Email |
| --- | --- | --- |
| FL (DCM) | Shohei Yoshioka | shohei.yoshioka@docomo-lab.com |
| Lenovo | Hongmei Liu | Liuhm6@lenovo.com |
| Apple | Chunxuan Ye | Chunxuan_ye@apple.com |
| Apple | Chunhai Yao | Chunhai_yao@apple.com |
| Xiaomi | Min Liu | Liumin10@xiaomi.com |
| Xiaomi | Yajun Zhu | zhuyajun@xiaomi.com |
| vivo | Zhipeng Lin | zhipeng.lin@vivo.com |
| vivo | Yong Wang | wy.wang.5g@vivo.com |
| Nokia | Frank Frederiksen | Frank.frederiksen@nokia.com |
| OPPO | Hao LIN | lin.hao@oppo.com |
| OPPO | Zuomin WU | wuzuomin@oppo.com |
| OPPO | Nande Zhao | zhaonande@oppo.com |
| Huawei, HiSilicon | Xiaolei TIE | tiexiaiolei@huawei.com |
| Huawei, HiSilicon | Ying Chen | chenying18@huawei.com |
| Huawei, HiSilicon | Xinghua Song | songxinghua@huawei.com |
| ZTE | Fangyu Cui | cui.fangyu@zte.com.cn |
| CATT | Deshan Miao | miaodeshan@catt.cn |
| Ericsson | Stefan Eriksson Löwenmark | stefan.g.eriksson@ericsson.com |
| Thales | Mohamed EL JAAFARI | mohamed.el-jaafari@thalesaleniaspace.com |
| Spreadtrum | Zhenzhu Lei | reven.lei@unisoc.com |
| MediaTek | Gilles Charbit | Gilles.charbit@mediatek.com |
| InterDigital | Moon-il Lee | Moonil.lee@interdigital.com |
| Sony | Samuel Atungsiri | Sam.Atungsiri@sony.com |
| Lockheed | Robert Olesen | robert.l.olesen@lmco.com |
| ETRI | Dukhyun You | dhyou@etri.re.kr |
| Panasonic | Akihiko Nishio | nishio.akihiko@jp.panasonic.com |
| Samsung | Sungjin Park | sj100.park@samsung.com |
| Samsung | Carmela Cozzo | carmela.c@samsung.com |
| Omnispace | Ron Olexa | rolexa@omnispace.com |
| NEC | Pravjyot Singh Deogun | pravjyot.deogun@emea.nec.com |
| Ligado | Clive Packer | clive@ligado.com |
| Hughes/EchoStar | Munira Jaffar | Munira.Jaffar@EchoStar.com; munirajaffar@hughes.com |
| Qualcomm | Xiao Feng Wang | wangxiao@qti.qualcomm.com |
| Qualcomm | LiangPing Ma | lpma@qti.qualcomm.com |
| Novamint | Thierry Bérisot | tberisot@novamint.com |
| GateHouse | Robert van der Pool | rvp@gatehouse.com |
| FGI | YenHua Li | danielli@fginnov.com |
| LG | Haewook Park | haewook.park@lge.com |
| LG | Seokmin Shin | seokmin.shin@lge.com |
| LG | Duckhyun Bae | duckhyun.bae@lge.com |
| NTT DOCOMO | Yoshinori Ojima | yoshinori.ojima@docomo-lab.com |
| Baicells | Xiang Yun | yunxiang@baicells.com |
| Baicells | Yong Ding | dingyong@baicells.com |