3GPP TSG-RAN WG1 #110bis-e	Tdoc R1-2210385

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.2.4.1

Source:	Moderator (Ericsson)

Title:	Summary #1 of Evaluation on AI/ML for positioning accuracy enhancement

Document for:	Discussion, Decision

# Introduction

This document summarizes the discussions during RAN1#110bis for the agenda item 9.2.4.1, Evaluation on AI/ML for positioning accuracy enhancement.

This discussion corresponds to the objectives related to the positioning use case described in RP-213599 (SID) below.

RP-213599 (SID): Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. … Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

# Evaluation Methodology

In this section, remaining issues of evaluation methodology are discussed for AI/ML based positioning.

## Input to ML model for AI/ML assisted positioning

One issue raised by Qualcomm and OPPO is about the input/output of the ML model when AI/ML assisted approach is simulated. Their proposals are copied below.

- Qualcomm (R1-2209980) For AI/ML-assisted positioning, the UE can train an AI/ML model in two approaches: - Single-TRP approach: Using as input the channel response from a single TRP, the AI/ML model outputs the TOA associated with that TRP. The same model is used for all TRPs. - Multi-TRP approach: The AI/ML model uses as input the channel responses from all 18 TRPs, and outputs 18 TOAs jointly. Proposal4: When reporting performance, companies should clearly report which of the two approaches (single or multi-TRP) was used for evaluation. Proposal5: For AI/ML-assisted positioning, the single-TRP approach is adopted for evaluation as a baseline.

- OPPO (R1-2208854) Proposal 2: For AI/ML assisted positioning, if the output of AI model is timing-based result (e.g., TOA), prioritize the scheme where the measurement results corresponding to all TRPs are used as the input of AI model - the scheme where the measurement results corresponding to single TRP are used as the input of AI model is deprioritized.

### 1st round discussion

OPPO suggested to prioritize multi-TRP setup:

- “… where the output of AI model are the estimated TOAs corresponding to the 18 TRPs.”

- “… where the measurement results corresponding to all TRPs are used as the input of AI model”

OPPO also suggested to deprioritize single-TRP setup.

Meanwhile QC proposes to adopt single-TRP approach as baseline for evaluation. Thus companies’ recommendations are not aligned.

While several companies evaluated the single-TRP setup for AI/ML assisted approach (e.g., [2][5][24]), no extensive evaluation has been performed to compare single-TRP and multi-TRP setup. In some submitted evaluations, it is not clear which setup was used in ML model construction.

It is proposed that the two constructions are defined to align understanding. When reporting performance, companies should clearly report which of the two constructions (single or multi-TRP) was used for evaluation, as suggested by Qualcomm (R1-2209980).

Proposal 2.1.1-1

For AI/ML-assisted positioning, companies report which construction is applied in their evaluation:

- Single-TRP construction: the input of the ML model is the channel measurement between the target UE and a single TRP, and the output of the ML model is for the same pair of UE and TRP.

- Multi-TRP construction: the input of the ML model contains N sets of channel measurements between the target UE and N (N>1) TRPs, and the output of the ML model contains N sets of values, one for each of the N TRPs.

|  | Company |
| --- | --- |
| Support | Nokia/NSB, NVIDIA, Apple, InterDigital, Fujitsu, LG, Qualcomm,CMCC, samsung,ZTE, HW/HiSi, CATT, CAICT, Lenovo, Ericsson |
| Not support |  |

| Company | Comments |
| --- | --- |
| Nokia/NSB | We are fine with this proposal. |
| LG | Fine with the proposal while MTRP-based measurement seems natural for positioning. |
| OPPO | Just for clarification, if RSTD is the output of the UE-sided AI model (which is corresponding to two TRPs), does it belong to above-mentioned single-TRP construction or multi-TRP construction? [Moderator] My understanding is, RSTD still belongs to single-TRP. When RSTD is the output, one TRP is considered the reference point. One value is generated at ML output. |
| Qualcomm | We are fine with the proposal. |
| HW/HiSi | Support |

## Generalization aspects

In RAN1#110, agreements were made on four generalization aspects. For this meeting, further discussion on generalization investigation have been submitted in the contributions.

- Qualcomm (R1-2209980) Proposal1: To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning: - Different realization of partial reflection and blocking changes - Training dataset from datasets {A0, A1,…, AN-1}, test dataset Bj from partially unseen reflections and blockings (i.e., partially different reflections and blockings than any in {A0, A1,…, AN-1} due to movement of reflecting objects, small environment variation over time, etc.). Here N>=1. FFS: modelling of {A0, A1,…, AN-1} and Bj based on TR 38.901. Proposal2: To model changes for investigating zone-specific generalizations at a given link between UE and TRP, one option is to consider a set CAj⊂C for generating channel coefficients for training dataset Aj, and consider a set CBj⊂C, CBj≠CAj, for generating channel coefficients of dataset Bi, where C is set of N=25 clusters between the UE and TRP. Then, the channel coefficient generation for training dataset Aj can follow these updated equations: Hu,sNLOSτ,t=n∈CAj, n≤22 i=13m∈RiHu,s,n,mNLOStδτ-τn,i-d3Dc-Δτ+n∈CAj, n≥3NHu,s,nNLOStδτ-τn-d3Dc-Δτ (training dataset Aj - NLOS) Hu,sLOSτ,t=1KR+1Hu,sNLOSτ,t+KRKR+1Hu,s,1LOStδτ-τ1-d3Dc. (training dataset Aj - LOS) and channel coefficient generation for testing dataset Bj can follow these updated equations: Hu,sNLOSτ,t=n∈CBj, n≤22 i=13m∈RiHu,s,n,mNLOStδτ-τn,i-d3Dc-Δτ+n∈CBj, n≥3NHu,s,nNLOStδτ-τn-d3Dc-Δτ (Testing dataset Bj-NLOS) Hu,sLOSτ,t=1KR+1Hu,sNLOSτ,t+KRKR+1Hu,s,1LOStδτ-τ1-d3Dc. (Testing dataset Bj-LOS)

- Faunhofer (R1-2209537) Proposal 2: The emulation of the impact of changes in the environment, the test data can be generated by weighted sum of two sets of channel data. Proposal 3: Split the channel data sets in three parts representing LOS part, part with high spatial correlation (e.g., single bounce reflections or specular reflections) and multipath components with lower spatial. Proposal 4: For better evaluation of the performance gain resulting from AI/ML based or assisted ToA estimator the simulation results shall be sorted according a level-of-difficulty metric.

- InterDigital (R1-2209484) Proposal 1: For the evaluation of AIML based positioning, adopt the following 3 categories of model generalization and evaluate the performance under each category - Category 1: Trained and inferred ideal input measurements - Category 2: Trained at ideal input measurements and inferred at non-ideal input measurements - Category 3: Trained at combination of ideal and non-ideal input measurements; inferred at non-ideal input measurements. Proposal 2: For the evaluation of Category 3 model generalization, agree on the split of the training data set between ideal(X%) and non-ideal(100-X) %) measurements. FFS: X.

- Samsung (R1-2209726) Proposal 1: RAN1 shall study the generalization ability for imperfect input/output data and how to model the imperfections. Proposal 3: RAN1 study the update/fine-tuning the model with limited number of data set or targeting generalization case.

- China Telecom (R1-2208772) Proposal 3: Different dataset size used for AI/ML model training should be studied to evaluate the model generalization capability.

- Huawei, HiSilicon (R1-2208433) Proposal 11 : For evaluation on the AI/ML-based positioning, the synchronization error of the transmission link between gNB and UE should also be considered. Proposal 12 : RAN1 should define some typical evaluation cases for evaluate the generalization in the aspect - UE timing error to better compare results of generalization studies among companies. In addition, further cases should also be captured to better explore and expand the generalization capabilities of the AI/ML model. One example is given in the template below.

- vivo (R1-2208638) Proposal 1: The positioning accuracy performance of AI/ML based positioning should be evaluated under different settings/scenarios (i.e. the training and testing dataset are from different settings) - different drop in the same scenario; - different clutter parameter settings; - different InF scenarios.

- Nokia (R1-2209371) Proposal-1: For evaluation of AI/ML based positioning, only InF-DH sub-scenario should be considered as part of the Rel-18 study item. Proposal-2: RAN1 should agree to limit the Rel-18 study on AI/ML for positioning accuracy enhancements use case to only intra-site variations, as previously agreed. Proposal-3: RAN1 to discuss and agree whether other approaches such as model update, finetuning or adapting the positioning method used, could be a better approach to handle possible variations within a particular scenario.

### 1st round discussion

In RAN1#110, the proposal below was considered “offline agreement”, but it couldn’t be confirmed as agreement during online discussion. The main concern was, different InF scenarios represent substantially different deployment environments. It was not a typical case that a ML model need to work well in different InF scenarios.

In the following, the same proposal is presented again to see if consensus is achievable.

Proposal 2.2.1-1

To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning:

(e) InF scenarios, e.g., training dataset from one InF scenario (e.g., InF-DH), test dataset from a different InF scenario (e.g., InF-HH)

|  | Company |
| --- | --- |
| Support | Vivo, Nokia/NSB, NVIDIA, Apple, Fujitsu, LG, OPPO, Samsung, CATT, CAICT, Lenovo |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | We can live with it if other companies would like to evaluate it. However, we don’t see a different observation can be made for this case as we already have different clutter settings for InF-DH. |
| HW/HiSi | Share the view of ZTE |
| Ericsson | We share the same view as ZTE. |

The other issue that was discussed in RAN1#110 is the modeling of minor changes in the environment. In this meeting, Qualcomm (R1-2209980) and Faunhofer (R1-2209537) provided detailed methods to modify the channel models in 38.901 to emulate minor changes in the environment (e.g., moving objects, small environment variations over time in a factory). In Qualcomm (R1-2209980), this is intended for the study of “intra-site (or zone-specific)” generalization.

Companies are invited to study carefully the methods described in Qualcomm (R1-2209980) and Faunhofer (R1-2209537), and provide input on whether/how to proceed on this generalization aspect.

Question 2.2.1-2

Should RAN1 investigate the model generalization capability for intra-site (or zone-specific) variations?

|  | Company |
| --- | --- |
| Yes | Qualcomm |
| No | Nokia/NSB, HW/HiSi, Ericsson |

| Company | Comments |
| --- | --- |
| Nokia/NSB | We think that the current agreements capture model generalization scenarios quite comprehensively, and the results highlight the issues that arise from applying an AI/ML model trained and tested using different scenarios/settings. We are not sure about the added value from including more scenarios as part of this evaluation. |
| Apple | If companies want to do so, they are free to bring results, but this study should not be mandatory. |
| Fujitsu | These concepts such as “inter-site”, “intra-site” or “zone” were proposed in 9.2.1 as guidelines, we prefer to clarify details of these concepts in general first. |
| HW/HiSi | Agree with Nokia |
| CATT | Share the same view with Nokia and Apple. |
| Ericsson | We share the same concern as Nokia. Additionally, in terms of how to modify the methodology and equations in 38.901, channel model experts need to be consulted to be sure if the proposed methods are valid. However, this task is out of scope for this study item. |

Question 2.2.1-3

If yes to Question 2.2.1-2: how to modify the channel models in 38.901 to emulate intra-site (or zone-specific) variations?

- Alt 1: according to Proposal 1 and 2 of R1-2209980

- Alt 2: according to Proposal 2 and 3 of R1-2209537

- Alt 3: Other (please describe)

|  | Company |
| --- | --- |
| Alt 1 | Qualcomm |
| Alt 2 |  |
| Alt 3 (Other, please describe) |  |

| Company | Comments |
| --- | --- |
| Fujitsu | Suggest not to do any limitation here, left this part for companies to report their own scheme freely. |
| Qualcomm | We support Alt 1 but are also fine to discuss other alternatives. |

## Channel estimation

In the contributions below, it is proposed that RAN1 should study the AI/ML performance with actual channel estimation, instead of ideal CIR for example.

- vivo (R1-2208638) Proposal 6: Further study the impact and potential solution of CIR estimation error on AI/ML based positioning performance.

- Google (R1-2208882) Proposal 1: The study of AI/ML based AI/ML based positioning should be based on actual channel estimation.

### 1st round discussion

In vivo (R1-2208638), evaluation results were first provided using error-free CIR ("The existing schemes are all evaluated under the assumption that ideal CIRs used for model training and inference can be obtained while ignoring the implementation imperfections."). Then detailed investigation was performed for a range of SINR values for the training dataset and test dataset (see results copied in section 5.5.3).

Moderator’s understanding is, it is not recommended to use SINR as a parameter for positioning study. SINR is a metric for a single link, whereas positioning requires input from numerous links (typically, at least 3 UE-TRP links), where each link can have different SINR depending on the pathloss between the UE and each TRP, the multipath channel condition, transmitter power, noise figure, etc. Thus it is sufficient to follow Google (R1-2208882) proposal that actual (i.e., noisy) channel estimation should be assumed in the evaluation.

It is noted that the agreed simulation assumption provides sufficient parameters to determine channel estimation performance for each link used for positioning, see sample parameter settings below.

TR 38.857, Table 6-1: Common scenario parameters applicable for all scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |

TR 38.857, Table 6.1-1: Parameters common to InF scenarios

| Total gNB TX power, dBm | 24dBm | 24dBm EIRP should not exceed 58 dBm |
| --- | --- | --- |

Thus, the following is proposed, which is modified from R1-2208882 (Google).

Proposal 2.3.1-1

For evaluation of AI/ML based positioning, the model performance is evaluated with actual (i.e., not ideal) channel estimation, using the evaluation assumption on gNB/UE TX power and noise figure.

|  | Company |
| --- | --- |
| Support | Ericsson |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | In principle, we support to study the impact of channel estimation error. We’d like to response and get clarification to moderator’s comment “it is not recommended to use SINR as a parameter for positioning study. SINR is a metric for a single link, whereas positioning requires input from numerous links (typically, at least 3 UE-TRP links), where each link can have different SINR depending on the pathloss between the UE and each TRP”. 1. Yes, there’re multiple links (i.e., between the UE and multiple TRPs) for positioning. However, there’s an SINR for each link/channel (i.e., the channel between the UE and a TRP). In our contribution R1-2208638 section 4.2.1, we studied the SINR distribution (practical lower and upper bound considering interference from all other links) and showed the range of practical SINR in Figure 12 for CIR as model input. 2. Since moderator stated “it is not recommended to use SINR as a parameter for positioning study”, can the moderator clarify what would be the metric for the non-ideal channel estimation/measurement accuracy (e.g., CIR measured for each UE/TRP pair) for model input? 3. Current proposal says “using the evaluation assumption on gNB/UE TX power and noise figure”, our understanding is that we need more discussion on other assumptions for the study of non-ideal channel estimation. For example, when one TRP transmit RS for CIR measurement, do all other TRPs transmit at the same time or not? If so, does RS from all TRPs overlapping or not? If no overlapping RS from all TRPs, what about the assumption regarding data transmission from other TRPs w.r.t. RS transmission from one TRP? |
| Apple | To clarify, is the expectation that we model a link between the UE and TRP and perform a channel estimation based on an RS signal ? |
| OPPO | More discussions are needed on how to stimulate the inference. For example, the neighboring cells may be transiting data which will lead to inference to the PRS reception. However, in the simulator for positioning, the data transmission is usually not simulated. For this case, how to model the inference? |
| Samsung | Ideally, this real channel estimation should be evaluated, but considered the workload, we feel it should left to companies who interested to present the result by themselves, just to state out the details on how it simulates. |
| ZTE | We think agreed tables are enough for study. This proposal is not needed. If we want to evaluate model performance under different SINR, which may belong to model generalization issue. Agree with Samsung, we should consider the workload. |
| HW/HiSi | We do not support the proposal, at least not at this stage. From reviewing the contributions, it is unclear to us how companies have modeled the channel estimation. In a first step, companies could report the channel estimation they have used. From our understanding, the impact of channel estimation on the positioning accuracy is much smaller than from e.g. the NW synchronization errors. |
| CATT | We think real channel estimation is helpful to evaluation the model performance. But how to model it and how to compare the simulation results between different companies due to different companies may have different real channel estimation capability should be further studied. |
| MediaTek | We found including TX power in can improve position accuracy, which is shown in section 3.2.1 of our Tdoc (R1-2209510) where non-normalized PDPs and normalized PDPs as model input are studied. Basically, we support to study the actual channel estimation based on gNB/UE TX power and noise figure which have been already agreed upon since RAN1#109e as parameters in the InF scenarios. And furthermore, we would like to ask for a clarification whether other potential imperfections such as timing and frequency synchronization error or RF implementation errors, etc., on channels estimation in UE or gNB should be included, even though these imperfections were agreed to study in RAN1#110 and several companies have already shown their results as part of the evaluation for generalization. If the answer is yes, each company can proposal how to add the impairments to channel estimation in the next meeting (RAN1#111). |
| CAICT | More evaluation results could be collected for further discussion and it depends on companies‘ will till next meeting. |
| Lenovo | Further study is required, but the evaluation of non-ideal CE can be considered optional as part of the evaluations. |
| Ericsson | The channel estimation error should be included as part of the evaluation, instead of error-free channel estimation. Exactly how to perform the channel estimation is up to each company to report. |

## Other

Other issues on evaluation assumptions and methodologies are also discussed by companies.

- Google (R1-2208882) Proposal 2: The study of AI/ML based AI/ML based positioning should take random phase offset between cells into account.

- MediaTek (R1-2209510) Proposal 4: For comparison of evaluation on AI/ML-based LOS/NLOS identification, it is suggested to align the sampling period in dataset.

- Fujitsu (R1-2209015) Proposal 2 It is suggested to have more studies on the imperfect or noisy labels for both the benefits and the potential disadvantages, companies are encouraged to provide more detailed and general evaluation results when using imperfect labels.

- Nokia (R1-2209371) Observation-4: The availability of good quality data with sufficient diversity of positioning ground truth labels with accurate information, for model training and testing/validation is one of the key challenges in AI/ML based positioning. Observation-5: It is important to note that training dataset size as an indication of user area density is valid only for uniform distribution of UEs within the simulation setting. Proposal-6: For evaluation of AI/ML based positioning, consider additional UE distribution options such as sparse or clustered deployment of UEs, while evaluating model performance. Proposal-7: For evaluation of AI/ML based positioning, RAN1 should assess the need for standardizing the procedures for triggering and/or controlling and/or monitoring the ML model adaptation and fine-tuning after model deployment.

### 1st round discussion

One issue pointed out by MediaTek (R1-2209510) is the sampling period (or equivalently, sampling rate) in dataset. Simulation results were provided in R1-2209510 to illustrate the impact of using sampling period of 1 ns vs 9.3 ns.

The issue of sampling period has not been discussed in previous meetings, and it is true that alignment between companies is necessary for fair comparison.

Moderator’s understanding is, the sampling period is a function of subcarrier spacing. Specifically, the sampling period can be calculated as follows:

Sampling rate = Nf×∆f

Sampling period = 1/(Nf×∆f)

Where FFT size Nf=4096 according to 38.211, and ∆f is subcarrier spacing.

Thus, for the agreed subcarrier spacing, the sampling periods are obtained as following for FR1 and FR2.

|  | FR1 | FR2 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 30kHz | 120kHz |
| Sampling period (ns) | 8.13 | 2.03 |

Thus the following is proposed.

Proposal 2.4.1-1

For evaluation of AI/ML based positioning, the sampling period is aligned to: 8.13 ns for FR1 and 2.03 ns for FR2.

|  | Company |
| --- | --- |
| Support | MediaTek, [Ericsson] |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | Question for clarification. Is the intention to align the size of model input among companies? If so, we can discuss this proposal after we reach agreement on proposal 4.1-1 (when we all use the same channel measurement (i.e., CIR) as model input for training/inference. |
| Samsung | We feel it’s just important for using the same settings for tratining data and inferernece data, we don’t need to limit to any specific type. |
| ZTE | Agree with vivo. |
| HW/HiSi | We do not see a need for this proposal. We share the moderator’s understanding how the sampling period is obtained from the agreed subcarrier spacing. Thus, the sampling rate should be already clear. |
| Ericsson | We are OK to have this proposal explicitly captured to avoid confusion. Alternatively, if all companies agree with how the sampling period is obtained from the agreed subcarrier spacing, then it is also OK not to have explicit agreement. |

# KPI

In the following, the remaining issues on KPI are discussed.

## Intermediate performance metric of assisted AI/ML positioning

In RAN1#110, the following agreement was made. In this meeting, several companies have suggestions on how to resolve the FFS bullet.

Agreement

For evaluation of AI/ML assisted positioning, an intermediate performance metric of model output is reported.

- FFS: Detailed definition of the intermediate performance metric of the model output

- Ericsson (R1-2208399) Proposal 3 For evaluation of AI/ML assisted positioning with LoS classification and ToA estimation as the intermediate model outputs, at least LoS classification accuracy and CDF percentile of ToA estimate errors (preferrably expressed in meters) should be reported as the intermediate performance metrics.

- LG (R1-2208903) Proposal #2: Determine intermediate performance metric with respect to sub use case. - At least for LOS/NLOS identification, the meaning of the value and the corresponding format is to be decided with high priority.

- CATT (R1-2208971) Proposal 2: For AI/ML-based positioning, in addition to AI-specific KPIs, the following KPIs are considered: - Eventual KPI: Positioning accuracy (e.g. 90% CDF percentiles of horizontal accuracy); - Intermediate KPI: The accuracy of intermediate measurement results (e.g. error of ToA/AoA/AoD) if estimating timing and/or angle of measurement based on AI/ML model is applied; - Intermediate KPI: The correct rate of LOS/NLOS identification if identifying LOS/NLOS identification based on AI/ML model is applied.

- Nokia (R1-2209371) Proposal-4: For evaluation of two-step or AI/ML assisted positioning, intermediate KPI(s) such as the accuracy of LOS/NLOS identification, accuracy of timing and/or angle of measurement, accuracy of the likelihood measurement, etc., should be reported together with the horizontal positioning accuracy.

- Faunhofer (R1-2209537) Proposal 5: For the evaluation of performance gain resulting from additional reporting for input data to the AI/ML model (network based ToA estimator, for example) the main performance metric shall be the ToA error statistic. Proposal 6: For the evaluation of performance gain resulting from additional reporting for output data to the AI/ML model system level simulations including positioning algorithms are required and the main performance indicator is the positioning accuracy and the probability of false positions.

### 1st round discussion

Based on the proposals in companies’ contributions, the following proposal seems to capture the majority view.

Proposal 3.1.1-1

For evaluation of AI/ML assisted positioning, the intermediate performance metrics include:

- LoS classification accuracy, if the model output includes LoS classification;

- ToA estimation accuracy (expressed in meters), if the model output includes ToA.

|  | Company |
| --- | --- |
| Support | Nokia/NSB, NVIDIA, InterDigital, Fujitsu, LG, Ericsson |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | Question for clarification. Is the reported accuracy for each link (corresponding to each TRP)? Or for each path since there may be multiple paths between the UE and a TRP? |
| Nokia/NSB | In our view, reporting the overall accuracy metrics could also be sufficient. |
| Apple | To clarify, ToA estimation accuracy (expressed in meters) = abs(ToA_estimate – ToA_actual) x speed_of_light |
| LG | In addition, for LOS/NLOS identification, the meaning of the value range (e.g. [0,1]) and the corresponding format of the value are to be decided with high priority where it can be reused as in Rel-17 or newly defined by taking AI/ML specific components into account. |
| OPPO | Suggest to add the following metric RSTD estimation accuracy, if the model output includes RSTD |
| Qualcomm | Please include accuracy of soft-information: - Timing soft-information accuracy (e.g., mean ToA error and variance of ToA), if the model output includes timing soft information. |
| CMCC | Mean TOA error can be used. |
| Samsung | Wording change: For evaluation of AI/ML assisted positioning, the following intermediate performance metrics are used include: - LoS classification accuracy, if the model output includes LoS classification; - ToA estimation accuracy (expressed in meters), if the model output includes ToA. |
| ZTE | Generally fine with this proposal. We think second bullet should be more general. - Timing/angle estimation accuracy (expressed in meters/degrees), if the model output includes timing/angle estimation. If we would like to give example of timing estimation, RSTD should also be included. |
| HW/HiSi | Support the proposal with the understanding that the intermediate accuracy is reported per link (not per path). |
| CATT | Fine with this proposal and also fine with ZTE’s update. |
| CAICT | We also fine to include RSTD. |
| Lenovo | Support |

## Other

Other views on KPI were also submitted by companies in their contributions, see below. Considering the common KPI agreement made under AI 9.2.1 in RAN1#110, the proposals below can be revisited if/when further agreements are made under AI 9.2.1 to include these metrics.

- Lenovo (R1-2209124) Proposal 2: The positioning AI/ML model evaluation methodology should support scenarios evaluating a model's robustness and adaptability, e.g., including how often an AI/ML evaluation model is updated based on a particular generalizability evaluation criterion. FFS any other relevant criteria. Proposal 3: In addition to FLOP counts, the evaluation should also consider the hardware and software platforms used to evaluate the positioning AI/ML algorithms, type of data being used as input, training type, e.g., offline vs online, complexity type, e.g., worst-case/average-case.

- Nokia (R1-2209371) Proposal-5: Optional KPIs such as position estimation latency, radio resource efficiency and higher layer signaling overhead should be reported together with the horizontal positioning accuracy.

# Input, output

Regarding input and output of the ML model, selected proposals are copied below.

- ZTE (R1-2208525) Proposal 1: For evaluations on AI for positioning, at least include following sub-use cases for direct AI/ML positioning and AI/ML assisted positioning: - Direct AI/ML positioning - Sub-use case 1-1: AI model input is path timings and RSRPPs from single port PRS - Sub-use case 1-2: AI model input is CIR (i.e., path timings, RSRPPs and path phases) from single port PRS - Sub-use case 1-3: AI model input is path timings and RSRPPs (or CIR) from multi-port PRS - AI/ML assisted positioning - Sub-use case 2-1: AI model output is DL PRS RSTD values - Sub-use case 2-2: AI model output is LOS/NLOS indicator Observation 3: With path phase information included in the AI model input, the positioning performance is improved obviously when compared to AI model input without path phase information. Proposal 3: Study and evaluate the performance of direct AI/ML positioning when AI model input includes path phase information. Observation 4: With measurement results from multi-port PRS included in the AI model input, better positioning performance can be observed when compared to AI model input only includes measurement results from single port PRS. Proposal 4: Study and evaluate the performance of direct AI/ML positioning when AI model input includes measurement results from multi-port PRS. Observation 5: The AI/ML based positioning method has excellent accuracy on the estimation of DL PRS RSTD values even in heavy NLOS conditions. Observation 6: AI/ML assisted positioning achieves better positioning performance than direct AI/ML positioning to some degree. Proposal 5: Study and evaluate the performance of AI/ML assisted positioning where the AI model output includes DL PRS RSTD values. Observation 7: The AI/ML based positioning method has a good accuracy rate of LOS/NLOS identification. Proposal 6: Study and evaluate the performance of AI/ML assisted positioning where the AI model output includes confidence levels of LOS/NLOS identification.

- vivo (R1-2208638) Proposal 2: Capture in the TR that time domain CIR as the model input for direct AI/ML positioning obtains the best performance compared to other model inputs. Proposal 3: Support time domain CIR as the model input at least for direct AI/ML positioning.

- Huawei, HiSilicon (R1-2208433) Proposal 5: For comparison of evaluation on AI/ML-based fingerprint positioning evaluation results, support the channel impulse responses (CIRs) as the model inputs.

- Google (R1-2208882) Proposal 5: For direct AI/ML positioning, consider to use CIR and L1-SINR from each cell as the input.

- Nokia (R1-2209371) Observation-7: The UE/TRP can report only the timing and RSRP values, and the signaling of CIRs from the UE to the network is not supported. Observation-8: If CIR is agreed as a baseline model input, that would imply that only UE-based direct or AI/ML assisted positioning methods are considered. However, in such a scenario, there might be challenges related to acquiring labeled training data from other UEs or from the network.

## 1st round discussion

Based on companies’ input, there is wide support to have CIR as model input. Thus the following is proposed.

Proposal 4.1-1

For evaluation of direct AI/ML positioning, support at least the time-domain channel impulse response (CIR) as the model input.

|  | Company |
| --- | --- |
| Support | vivo, NVIDIA, Apple, Fujitsu, LG, OPPO,CMCC, HW/HiSi, CATT, CAICT, Lenovo, Ericsson |
| Not support | Qualcomm |

| Company | Comments |
| --- | --- |
| Nokia/NSB | In our understanding, some companies have considered downlink CIR as the model input, whereas some have considered uplink CIR obtained from SRS estimation. Perhaps it is important to clarify which time-domain CIR value is considered here. We had also raised the question and would be interested to know other company views regarding how for e.g., downlink CIR information could be widely available across Ues and at the network-side, such that a dataset could be generated for testing and training. |
| Qualcomm | We do not understand the motivation of the proposal but generally prefer not to specify input of the AI/ML model. If the purpose to have this an observation of what has been used by companies for evaluation, then we are ok. |
| Samsung | We understand there is “at least” in the proposal, but still want to ask, why only time domain CIR is picked out. Because as we can see, the final TR will just capture the model input being used in the evaluation, the spec impact for a certain type of model input is under 9.2.4.2. Besides, this is not to try to exclude any model input to be evaluated. |
| ZTE | Current specification doesn’t support UE/gNB to report CIR (i.e., path timing and path RSRP without path phase). If this proposal is to discuss the baseline for evaluating direct AI/ML positioning, which should be based on the measurement that is already supported in spec. Then, we can further evaluate the performance gain over baseline method when using other model input. We have following suggestions. For evaluation of direct AI/ML positioning, support power delay profile (PDP, i.e., path timing and path RSRP) as a baseline for the model input. Companies are encouraged to evaluate other model input, - Option 1: time-domain channel impulse response - Option 2 : other model inputs are not precluded. |
| HW/HiSi | Support |

# Performed evaluation of direct AI/ML positioning

In this meeting, a large amount of evaluation work has been performed by companies for direct AI/ML positioning. These valuable results are very important to help RAN1 to make progress.

Selected results submitted by companies are copied below.

## Evaluation without generalization considerations

- Huawei, HiSilicon (R1-2208433) Table 3. Evaluation results for AI/ML model deployed on network-side, without model generalization Model structure; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. ResNet; CIR; UE coordinates; UE coordinates; {60%, 6m, 2m}; 25000; 5000; 34 K; 10M; 0.492. {40%, 2m, 2m}; 0.606.

- ZTE (R1-2208525) Figure. 4 CDFs of positioning errors for sub-use case 1-2 (Grid width is 0.5 m)

- vivo (R1-2208638) Table 2. Evaluation results of different model inputs for AI/ML model deployed on UE or Network side, without model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Train; Test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6,6,2}; 25k; 1k; 1.65M; 22.30M; 0.99. Power + delay + angle of the first path; Pos.; 0; {0.6,6,2}; 25k; 1k; 1.65M; 22.30M; 1.19. Power + delay of the first path; Pos.; 0; {0.6,6,2}; 25k; 1k; 1.65M; 22.30M; 1.31. Delay + angle of the first path; Pos.; 0; {0.6,6,2}; 25k; 1k; 1.65M; 22.30M; 1.43. Angle + power of the first path; Pos.; 0; {0.6,6,2}; 25k; 1k; 1.65M; 22.30M; 1.79. Observation 2: Different inputs of AI model will affect the positioning performance for AI/ML based positioning. Time domain channel CIR as the input of AI model obtains the best positioning accuracy. Proposal 2: Capture in the TR that time domain CIR as the model input for direct AI/ML positioning obtains the best performance compared to other model inputs. Proposal 3: Support time domain CIR as the model input at least for direct AI/ML positioning.

- Qualcomm (R1-2209980) Table 1 Evaluation results for AI/ML model deployed on UE-side, without model generalization, CNN Method; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. RFFP; CIR (2, 18,400); 2D location; 0%; {60%, 6m, 2m}; 15000; 2000; 1.5M params; 1.54G FLOPs; 2.77.

- China Telecom (R1-2208772) Table 3. Evaluation results for AI/ML model deployed on network-side, without model generalization Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; Test; Model complexity; Computational complexity; AI/ML. TOA; Predicted UE position; True UE position; [0.6,6m,2m]; 78400 samples; 1600 samples; 75.7k; 75.1k; 0.69. DL-TDOA; Predicted UE position; True UE position; [0.6,6m,2m]; 78400 samples; 1600 samples; 75.7k; 75.1k; 0.73. RSRP; Predicted UE position; True UE position; [0.6,6m,2m]; 78400 samples; 1600 samples; 75.7k; 75.1k; 0.52. RSRP+TOA; Predicted UE position; True UE position; [0.6,6m,2m]; 78400 samples; 1600 samples; 184.2k; 182.8k; 0.43. RSRP+DL-TDOA; Predicted UE position; True UE position; [0.6,6m,2m]; 78400 samples; 1600 samples; 184.2k; 182.8k; 0.38.

- CATT (R1-2208971) Table 1: Evaluation results for AI/ML model deployed on UE/LMF-side, without model generalization, resnet18 Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Type: CIR; Size: [18, 256, 2]; Type: UE’s position; Size: [1, 2]; UE’s position with 100% ground truth label; {60%, 6m, 2m}; Training: 19400; Validation: 1080;; 1080; 11.2M; 4.03G FLOPs; 0.98m. Table 2: Evaluation results for AI/ML model deployed on UE/LMF-side, without model generalization, resnet18 Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Type: CIR; Size: [18, 256, 2]; Type: UE’s position; Size: [1, 2]; UE’s position with 100% ground truth label; {60%, 6m, 2m}; Training: 19400; Validation: 1080;; 1080; 11.2M; 4.03G FLOPs; 0.84m.

- CAICT (R1-2209234) Table 2. Evaluation results for AI/ML model deployed on UE side, without model generalization, CNN Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; UE coordinate; 100% data with ground truth label; {60%, 6m, 2m}; 15000; 5000; 2.4M parameters; 4.8M FLOPs; <0.65m.

- xiaomi (R1-2209281) Table 1 Evaluation results for direct AI-based positioning with model deployed on UE or NW side, without model generalization, ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.6，6，2}; {0.6，6，2}; 70000; 10000; 21,277,442; 5.76GFlops; 0.4462. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.4，2，2}; {0.4，2，2}; 70000; 10000; 21,277,442; 5.76GFlops; 0.7566.

- CMCC (R1-2209332) Table II. Simulation results Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; UE location; UE coordinates; {60%, 6m, 2m}; 78400; 1600; 3.71 M; 7.41 M; 0.7 m. CIR+ RSRP; UE location; UE coordinates; {60%, 6m, 2m}; 78400; 1600; 3.71 M; 7.42 M; 0.35 m. TOA; UE location; UE coordinates; {60%, 6m, 2m}; 78400; 1600; 1.85 M; 3.7 M; 0.5 m. TOA+ RSRP; UE location; UE coordinates; {60%, 6m, 2m}; 78400; 1600; 1.85 M; 3.7 M; 0.34 m. TDOA; UE location; UE coordinates; {60%, 6m, 2m}; 78400; 1600; 1.85 M; 3.7 M; 0.41 m. TDOA+RSRP; UE location; UE coordinates; {60%, 6m, 2m}; 78400; 1600; 1.85 M; 3.7 M; 0.16 m.

- InterDigital (R1-2209484) Table 3 Summary of evaluation results Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. RSRP; UE position; percentage of training data set without label = 0%(default); {60%, 6m, 2m} for training and testing; 16000 samples; 4000 samples; 108 RSRP values per sample; 18.34 M FLOPs; 3.2796. RSRP; UE position; percentage of training data set without label = 0%(default); {60%, 6m, 2m} for training and {40%, 2m, 2m}for testing; 16000 samples; 4000 samples; 108 RSRP values per sample; 18.34 M FLOPs; 3.7447.

- Apple (R1-2209580) Table 3: Evaluation results for AI/ML model deployed on UE/network-side, without model generalization, with a CNN Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity (x106); Computational complexity (x106); AI/ML. CIR [18 x 256 x 2]; UE coordinates [1x2]; 0%; {60%, 6, 2}; 47500; 2500; 5.12; 2.43; 1.1m.

- Rakuten (R1-2209615) Table 1 Positioning accuracy of 90% UE Dataset; Positioning accuracy of 90% UE. Drop1; 0.67m. Drop80k; 5.55m.

- Samsung (R1-2209726) Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR with 18x256x2; 2D-coordinates; CIR (default); {60%，6m，2m}; 80000; 10000; 76K; 9.5M; 0.67.

## Evaluation of generalization aspects

### Generalization aspect: different drops

| - Ericsson (R1-2208399) Table 37 Evaluation results for AI/ML model deployed on network-side, with model generalization investigation where the model is trained in InF-DH {60%,6m, 2m}, and tested with the same drop or new drop compared to training. No network synchronization errors. The ML model contains 11 dense layers. Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training/ validation; test; Model complexity; Computation complexity. 18 RSRP values for a target UE; Horizontal position of the target UE; Ideal; {60%,6m, 2m}, same drop; 10,000 of 18 RSRP values; 4,000 of 18 RSRP values; around 3 million; Not available; 1.719. {60%,6m, 2m}, new drop; 19.86. Table 38 Evaluation results for AI/ML model deployed on network-side, with model generalization investigation where the model is trained in InF-DH {60%,6m,2m} without network synchronization error, and tested with new drop or network synchronization error. The ML model contains 11 dense layers. Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training/ validation; test; Model complexity; Computation complexity. 18 ToA values for a target UE; Horizontal position of the target UE; Ideal; {60%,6m, 2m}, same drop, T1=0 ns; 6,000 of 18 ToA values; 6,000 of 18 ToA values; around 3 million; Not available; 1.19. {60%,6m, 2m}, new drop, T1=0 ns; 19.38. {60%,6m, 2m}, same drop, T1=50ns; 12.4. |
| --- |
| - Huawei, HiSilicon (R1-2208433) Table 4. Evaluation results for AI/ML model generalization capability in the aspect: Different Drops Evaluated Scenarios; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). 1; Training; Drop 1; 25000; >10. Test; Drop 2; 5000. 2; Training; 5 Drops mixed; 25000 (5000/drop); 8.04. Test; Drop 2(outside of the trained Drops); 5000. 3; Training; 5 Drops mixed; 25000 (5000/drop); 1.28. Test; Drop 1 (inside the trained Drops); 5000. 4; Training; Drop 1 & 2 mixed; 25000 (12500/drop); 0.69. Test; Drop 1 (inside the trained Drops); 5000. |
| - vivo (R1-2208638) Table 7 Evaluation results of different drops for AI/ML model deployed on UE or Network side, with model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Train; Test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6,6,2}; 25k & Drop1; 1k & Drop1; 1.65M; 22.30M; 0.99. CIR; Pos.; 0; {0.6,6,2}; 25k & Drop1; 1k & Drop2; 1.65M; 22.30M; 6.00. CIR; Pos.; 0; {0.6,6,2}; 25k & Drop1; 1k & Drop3; 1.65M; 22.30M; 5.81. |
| - Qualcomm (R1-2209980) Figure 7 CDF of horizontal positioning errors of direct AI/ML positioning (solid plots: Baseline performance; dashed plots: Type 2 generalizations). |  |
| - Apple (R1-2209580) Table 5: Evaluation results for AI/ML model deployed on UE/network-side, with model generalization for different drops with a CNN Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity (x106); Computational complexity (x106); AI/ML. CIR [18 x 256 x 2]; UE coordinates [1x2]; 0%; {60%, 6, 2}; 47500; 2500; 5.12; 2.43; 3.1m. |
| - OPPO (R1-2208854) Table 4: Generalization performance: training and testing data sets are generated from different drops Accuracy achieved @90% (m); DL-TDOA; Direct: DL-TDOA+RSRP; Direct: Normalized CIR + RSRP; Indirect: Normalized CIR for all TRPs. w/o generalization; 1 drop for both training and testing; 8.20; 0.48; 0.33; 0.52. w/ generalization; 1 drop for training Another drop for testing; 9.92; 10.53; 10.11; 11.29. w/o generalization; 10 drops for both training and testing; 10.16; 0.46; 0.52; 1.03. w/ generalization; - drops for training Another 5 drops for testing; 10.2; 9.3; 6.55; 7.4. |

### Generalization aspect: different clutter parameters

- Ericsson (R1-2208399) Table 40 Evaluation results for AI/ML model deployed on network-side, with model generalization investigation where the model is trained on InF-DH {60%, 6m, 2m}, and tested with various InF-DH clutter parameters. All datasets use the same spatial correlation seed. No network synchronization error. The model is a CNN with 7 conv2D layers and 10 dense layers. Model; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training/ validation; Test; Model complexity; Computation complexity. CNN-1; Time domain CIR, 18x256x2 complex array; Horizontal position of the target UE; Ideal; {60%, 6m, 2m}; 80,000 / 10,000; 500; 6,463,776 complex parameters; 6,252,449,792 FLOPs; 0.9. {60%, 2m, 2m}; 4.5. {40%, 6m, 2m}; 1.0. {40%, 2m, 2m}; 7.6. Table 41 Evaluation results for AI/ML model deployed on network-side, with model generalization investigation where the model is trained on InF-DH {60%, 6m, 2m}, and tested with various InF-DH clutter parameters and different spatial correlation seeds. No network synchronization error. The model is a CNN with 7 conv2D layers and 10 dense layers. Model; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training/ validation; Test; Model complexity; Computation complexity. CNN-1; Time domain CIR, 18x256x2 complex array; Horizontal position of the target UE; Ideal; {60%, 6m, 2m} same drop; 80,000 / 10,000; 500; 6,463,776 complex parameters; 6,252,449,792 FLOPs; 0.9. {60%, 6m, 2m} new drop; >10. {60%, 2m, 2m} new drop; >10. {40%, 6m, 2m} new drop; >10. {40%, 2m, 2m} new drop; >10.

- Huawei, HiSilicon (R1-2208433) Table 5. Evaluation results for AI/ML model generalization capability in the aspect: Clutter parameters Evaluated Scenarios; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). 1; Training; Clutter paras: {40%, 2m, 2m}; 25000; >10. Test; Clutter paras: {60%, 6m, 2m}; 5000. 2; Training; Clutter paras: {60%, 6m, 2m} & {40%, 2m, 2m} mixed; 25000 (12500/ paras); 0.86. Test; Clutter paras: {60%, 6m, 2m}; 5000. 3; Training; Clutter paras: {60%, 6m, 2m} & {40%, 2m, 2m} mixed; 25000 (12500/ paras); 0.88. Test; Clutter paras: {40%, 2m, 2m}; 5000.

- vivo (R1-2208638) Table 8 Evaluation results of different cluster parameters for AI/ML model deployed on UE or Network side, with model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Train; Test; Train; Test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6, 6, 2}; {0.6, 6, 2}; 25k; 1k; 1.65M; 22.30M; 0.99. CIR; Pos.; 0; {0.6, 6, 2}; {0.4, 2, 2}; 25k; 1k; 1.65M; 22.30M; 8.67. CIR; Pos.; 0; {0.4, 2, 2}; {0.4, 2, 2}; 25k; 1k; 1.65M; 22.30M; 1.06. CIR; Pos.; 0; {0.4, 2, 2}; {0.6, 6, 2}; 25k; 1k; 1.65M; 22.30M; 4.77. CIR; Pos.; 0; {0.6, 6, 2} & {0.4, 2, 2}; {0.6, 6, 2}; 25k & 25k; 1k; 1.65M; 22.30M; 0.87. CIR; Pos.; 0; {0.6, 6, 2} & {0.4, 2, 2}; {0.4, 2, 2}; 25k & 25k; 1k; 1.65M; 22.30M; 0.94.

- Nokia (R1-2209371) Table 3. Evaluation results for AI/ML model deployed on UE-side, with model generalization, considering different clutter parameters. Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR from 18 BSs; 2D position (X, Y); True 2D position (X, Y); 40%; 10K+ (90%) including evaluation; 10K (10%); 1.9M; 19.2M flops; Typical generalization: 6.62517 (blue line Fig. 6) Custom generalization: 11.56156 (red line Fig. 6). CIR from 18 BSs; 2D position (X, Y); True 2D position (X, Y); 60%; 10K+ (90%) including evaluation; 10K (10%); 1.9M; 19.2M flops; Typical generalization: 8.70181 (blue line Fig. 7) Custom generalization: 14.36727 (red line Fig. 7).

- Apple (R1-2209580) Table 4: Evaluation results for AI/ML model deployed on UE/network-side, with model generalization for Clutter (train {60%,2m,2m}, test {40%, 2m, 2m}) with a CNN Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity (x106); Computational complexity (x106); AI/ML. CIR [18 x 256 x 2]; UE coordinates [1x2]; 0%; {60%, 6, 2}; 47500; 2500; 5.12; 2.43; 3.6m.

- xiaomi (R1-2209281) Table 3 Evaluation results for direct AI-based positioning with model deployed on UE or NW side, with model generalization (different clutter parameter), ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.6,6,2}; {0.4,2,2}; 70000; 10000; 21,277,442; 5.76GFlops; 7.0914. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.4,2,2}; {0.6,6,2}; 70000; 10000; 21,277,442; 5.76Gflops; 1.5328. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; Mix of {0.6,6,2}, {0.4,2,2}; {0.6,6,2}; 35000+35000; 10000; 21,277,442; 5.76Gflops; 0.5419. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; Mix of {0.6,6,2}, {0.4,2,2}; {0.4,2,2}; 35000+35000; 10000; 21,277,442; 5.76Gflops; 0.7684.

- CATT (R1-2208971) Table 3: Evaluation results for AI/ML model deployed on UE/LMF-side, with model generalization, resnet18 Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Type: CIR; Size: [18, 256, 2]; Type: UE’s position; Size: [1, 2]; UE’s position with 100% ground truth label; Training:{60%, 6m, 2m} Testing:{40%, 2m, 2m}; Training: 19400; Validation: 1080;; 1080; 11.2M; 4.03G FLOPs; 2.64m.

- OPPO (R1-2208854) Table 5: Generalization performance: training and testing data sets are with different clutter settings Accuracy achieved @90% (m); DL-TDOA; Direct: DL-TDOA+RSRP; Direct: Normalized CIR + RSRP; Indirect: Normalized CIR for all TRPs. Data setting C-1; 3.86; 8.05; 15.75; 16.09. Data setting C-2; 10.45; 10.8; 8.61; 8.88. Data setting C-3; 3.74; 7.78; 8.67; 8.49. Data setting C-4; 3.84; 4.86; 6.82; 6.66.

- InterDigtial (R1-2209484) Table 3 Summary of evaluation results Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. RSRP; UE position; percentage of training data set without label = 0%(default); {60%, 6m, 2m} for training and testing; 16000 samples; 4000 samples; 108 RSRP values per sample; 18.34 M FLOPs; 3.2796. RSRP; UE position; percentage of training data set without label = 0%(default); {60%, 6m, 2m} for training and {40%, 2m, 2m}for testing; 16000 samples; 4000 samples; 108 RSRP values per sample; 18.34 M FLOPs; 3.7447.

### Generalization aspect: network synchronization error

- Huawei, HiSilicon (R1-2208433) Table 6. Evaluation results for AI/ML model generalization capability in the aspect: Network synchronization error Evaluated Scenarios; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). 1; Training; Without network synchronization error; 25000; >10. Test; With network synchronization error @50ns; 5000. 2; Training; With network synchronization error @50ns; 25000; 3.02. Test; 5000. 3; Training; With network synchronization error @0&30&40&50ns; 25000 (6250/ paras); 2.51. Test; 5000 (1250/ paras). 4; Training; With network synchronization error @0&30&40&50ns; 25000 (6250/ paras); 4.28. Test; With network synchronization error @50ns; 5000 (1250/ paras).

- vivo (R1-2208638) Table 12 Evaluation results for AI/ML model deployed on UE or Network side, with model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6, 6, 2}; 25k; 1k & 0ns sync. error; 1.65M; 22.30M; 0.99. CIR; Pos.; 0; {0.6, 6, 2}; 25k; 1k & 2ns sync. error; 1.65M; 22.30M; 1.64. CIR; Pos.; 0; {0.6, 6, 2}; 25k; 1k & 10ns sync. error; 1.65M; 22.30M; 4.56. CIR; Pos.; 0; {0.6, 6, 2}; 25k; 1k & 50ns sync. error; 1.65M; 22.30M; 10.18. Table 13 Evaluation results for AI/ML model deployed on UE or Network side, with model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6, 6, 2}; 25k; 1k & 10ns sync. error; 1.65M; 22.30M; 4.56. CIR; Pos.; 0; {0.6, 6, 2}; 25k + 2k 10ns sync. error; 1k & 10ns sync. error; 1.65M; 22.30M; 1.16. CIR; Pos.; 0; {0.6, 6, 2}; 25k; 1k & 50ns sync. error; 1.65M; 22.30M; 10.18. CIR; Pos.; 0; {0.6, 6, 2}; 25k + 2k 50ns sync. error; 1k & 50ns sync. error; 1.65M; 22.30M; 1.52.

- Qualcomm (R1-2209980) Figure 5 CDF of horizontal positioning error for RFFP scheme under different TRPs’ synchronization assumptions (blue plot: TRPs are synchronized; magenta plot: TRPs have random synchronization error within [-10, 10] nanoseconds).

- xiaomi (R1-2209281) Table 5 Evaluation results for direct AI-based positioning with model deployed on UE or NW side, with model generalization (AI/ML model is trained by data set with ideal network synchronization and tested by data set with 100ns network synchronization error), ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.6，6，2}; {0.6，6，2}; 70000; 10000; 21,277,442; 5.76GFlops; 12.4486. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.4，2，2}; {0.4，2，2}; 70000; 10000; 21,277,442; 5.76GFlops; 14.5779. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; Mix of {0.6,6,2} {0.4,2,2}; {0.6,6,2}; 35000+35000; 10000; 21,277,442; 5.76GFlops; 13.5239. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; Mix of {0.6,6,2} {0.4,2,2}; {0.4,2,2}; 35000+35000; 10000; 21,277,442; 5.76GFlops; 13.2829. Table 6 Evaluation results for direct AI-based positioning with model deployed on UE or NW side, with model generalization (AI/ML model is trained by data set with 100ns network synchronization error and tested by data set with 100ns network synchronization error), ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.6,6,2}; {0.6,6,2}; 70000; 10000; 21,277,442; 5.76GFlops; 1.0666. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.4,2,2}; {0.4,2,2}; 70000; 10000; 21,277,442; 5.76GFlops; 1.7981. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; Mix of {0.6,6,2} {0.4,2,2}; {0.6,6,2}; 35000+35000; 10000; 21,277,442; 5.76GFlops; 1.3431. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; Mix of {0.6,6,2} {0.4,2,2}; {0.4,2,2}; 35000+35000; 10000; 21,277,442; 5.76GFlops; 1.9475.

- CATT (R1-2208971) Table 4: Evaluation results for AI/ML model deployed on UE/LMF-side, with model generalization [Training dataset with perfect network synchronization and testing dataset with network synchronization error], resnet18 Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Type: CIR; Size: [18, 256, 2]; Type: UE’s position; Size: [1, 2]; UE’s position with 100% ground truth label; {60%, 6m, 2m}; Training: 19400; Validation: 1080;; 1080; 11.2M; 4.03G FLOPs; 12.6m.

- OPPO (R1-2208854) Table 7: Generalization performance: Training w/o NW syn error, Test w/ NW syn error Accuracy achieved @90% (m); DL-TDOA; Direct: DL-TDOA+RSRP; Direct: Normalized CIR + RSRP; Indirect: Normalized CIR for all TRPs. w/ generalization; Data setting S-1 - drop w/o NW sync error for training, 1 drop w/ NW sync error for testing; 29.05; 5.29; 6.54; 27.85. w/o generalization; 1 drop w/o NW sync error for training and testing; 8.2; 0.48; 0.33; 0.52. w/ generalization; Data setting S-2 10 drops w/o NW sync error for training, 10 drops w/ NW sync error for testing; 36.91; 7.11; 11.77; 33.62. w/o generalization; 10 drops w/o NW sync error for training and testing; 10.16; 0.46; 0.52; 1.03.

### Generalization aspect: UE/gNB RX and TX timing error

- Huawei, HiSilicon (R1-2208433) Table 7. Evaluation results for AI/ML model generalization capability in the aspect: UE timing error Evaluated Scenarios; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). 1; Training; Without UE timing error; 25000; 3.12. Test; With UE timing error @10ns; 5000. 2; Training; With UE timing error @10ns; 25000; 0.61. Test; 5000. 3; Training; With UE timing error @0&10&20&30ns; 25000 (6250/ paras); 0.68. Test; 5000 (1250/ paras). 4; Training; With UE timing error @0&10&20&30ns; 25000 (6250/ paras); 0.89. Test; With UE timing error @30ns; 5000 (1250/ paras).

- Qualcomm (R1-2209980) Figure 4 CDF of horizontal positioning error for RFFP scheme under different UE clock drift conditions (green plot: RFFP performance in ideal settings when no clock drift present; blue plot: training accounts for UE clock drift and testing includes UE clock drift within [-150,150] nanoseconds).

- OPPO (R1-2208854) Table 8: Generalization performance: Training w/o UE timing error, Test w/ UE timing error Accuracy achieved @90% (m); DL-TDOA; Direct: DL-TDOA+RSRP; Direct: Normalized CIR + RSRP; Indirect: Normalized CIR for all TRPs. w/ generalization; Data setting T-1 1 drop w/o UE timing error for training, 1 drop w/ UE timing error for testing; 8.38; 0.48; 6.18; 26.79. w/o generalization; 1 drop w/o UE timing error for training and testing; 8.2; 0.48; 0.33; 0.52. w/ generalization; Data setting T-2 10 drops w/o UE timing error for training, 10 drops w/ UE timing error for testing; 10.28; 0.456; 12.393; 39.73. w/o generalization; 10 drops w/o UE timing for training and testing; 10.16; 0.46; 0.52; 1.03.

## Evaluation of model fine-tuning / re-training

- Ericsson (R1-2208399) Table 42 Evaluation results for AI/ML model deployed on network-side, with model fine-tuning. The CNN-1 model trained on InF-DH {60%, 6m, 2m} is updated to CNN-2 by further training with 1000 samples with {40%, 2m, 2m}. No network synchronization error. The model is a CNN with 7 conv2D layers and 10 dense layers. Model; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training/ validation; Test; Model complexity; Computation complexity. CNN-2; Time domain CIR, 18x256x2 complex array; Horizontal position of the target UE; Ideal; {60%, 6m, 2m}; 1000 for model update; 500; 6,463,776 complex parameters; 6,252,449,792 FLOPs; 3.5. {40%, 2m, 2m}; 4.3. Table 43 Evaluation results for AI/ML model deployed on network-side, with model retraining. The CNN-1 model trained in InF-DH {60%, 6m, 2m} is updated to CNN-3 by further training with a mixed {60%, 6m, 2m} + {40%, 2m, 2m} dataset with approximately 100,000 samples. No network synchronization error. The model is a CNN with 7 conv2D layers and 10 dense layers. Model; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training/ validation; Test; Model complexity; Computation complexity. CNN-3; Time domain CIR, 18x256x2 complex array; Horizontal position of the target UE; Ideal; {60%, 6m, 2m}; 100,000 for model update; 500; 6,463,776 complex parameters; 6,252,449,792 FLOPs; 1.0. {60%, 2m, 2m}; 1.6. {40%, 2m, 2m}; 2.0.

- Huawei, HiSilicon (R1-2208433) Table 8. Model update evaluation results for AI/ML model generalization capability Evaluated aspects; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). Before model update; After model update. Different Drops; Training; Drop 1; 25000; >10; 3.2. Fine-tuning; Drop 2; 1000. Test; Drop 2; 5000. Clutter parameters; Training; Clutter paras: InF-DH {60%, 6m, 2m}; 25000; >10; 3.1. Fine-tuning; Clutter paras: InF-DH {40%, 2m, 2m}; 1000. Test; Clutter paras: InF-DH {40%, 2m, 2m}; 5000. Network synchronization error; Training; Without network synchronization error; 25000; >10; 8.47. Fine-tuning; With network synchronization error @50ns; 5000. Test; With network synchronization error @50ns; 5000. UE timing error; Training; Without UE timing error; 25000; 7.24; 1.13. Fine-tuning; With UE timing error@20ns; 5000. Test; With UE timing error@20ns; 5000.

- Qualcomm (R1-2209980) Table 4 Evaluation results for AI/ML model deployed on UE-side, with model generalization (Type 2 - different drops), CNN Method; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training – Drop A; Test – Drop B; Model complexity; Computational complexity; AI/ML. RFFP; CIR (2, 18,400); 2D location; 0%; {60%, 6m, 2m}; 15000; 2000; 1.5M params; 1.54G FLOPs; 12.33. RFFP; CIR (2, 18,400); 2D location; 0%; {60%, 6m, 2m}; 15000 + 100 finetuning (Drop B); 2000; 1.5M params; 1.54G FLOPs; 10.47. RFFP; CIR (2, 18,400); 2D location; 0%; {60%, 6m, 2m}; 15000+ 240 finetuning (Drop B); 2000; 1.5M params; 1.54G FLOPs; 6.92. RFFP; CIR (2, 18,400); 2D location; 0%; {60%, 6m, 2m}; 15000+ 500 finetuning (Drop B); 2000; 1.5M params; 1.54G FLOPs; 6.07.

- NVIDIA (R1-2209629) Table 4: Summary of CDF percentiles of horizontal positioning accuracy with model finetuning. Training; Testing; Finetuning; 50%; 67%; 80%; 90%. Drop 1; Drop 1; N/A; 1.1 m; 1.5 m; 1.8 m; 2.3 m. Drop 1; Drop 2; No finetuning; 7.1 m; 9.3 m; 11.6 m; 14.5 m. Drop 1; Drop 2; Finetuning with 1k samples; 2.5 m; 3.3 m; 4.2 m; 5.3 m. Drop 1; Drop 2; Finetuning with 2k samples; 2.1 m; 2.7 m; 3.5 m; 4.3 m. Note: The original model was trained with 16k samples. Thus, 1k (resp. 2k) finetuning samples corresponds to 6.25% (resp. 12.5%) of the total 16k samples..

- CATT (R1-2208971) Table 5: Evaluation results for AI/ML model deployed on UE/LMF-side, with model generalization and fine-tuning, resnet18 Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Type: CIR; Size: [18, 256, 2]; Type: UE’s position; Size: [1, 2]; UE’s position with 100% ground truth label; Training:{60%, 6m, 2m} Fine-tuning and testing:{40%, 2m, 2m}; Training: 19400; Fine-tuning: 1080;; 1080; 11.2M; 4.03G FLOPs; 1.23m. Table 6: Evaluation results for AI/ML model deployed on UE/LMF-side, with model generalization and fine-tuning [Training dataset with perfect network synchronization and fine-tuning dataset with network synchronization error], resnet18 Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Type: CIR; Size: [18, 256, 2]; Type: UE’s position; Size: [1, 2]; UE’s position with 100% ground truth label; {60%, 6m, 2m}; Training: 19400; Fine-tuning: 1080;; 1080; 11.2M; 4.03G FLOPs; 2.23m.

## Evaluation of issues related to ground truth labels

- vivo (R1-2208638) Labeling error Table 14 Evaluation results for AI/ML model deployed on UE or Network side, with model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6, 6, 2}; 25k & std = 0; 1k; 1.65M; 22.30M; 0.99. CIR; Pos.; 0; {0.6, 6, 2}; 25k & std = 0.5; 1k; 1.65M; 22.30M; 1.51. CIR; Pos.; 0; {0.6, 6, 2}; 25k & std = 1; 1k; 1.65M; 22.30M; 2.17. CIR; Pos.; 0; {0.6, 6, 2}; 25k & std = 2; 1k; 1.65M; 22.30M; 3.55. Table 17 Evaluation results of semi-supervised learning for AI/ML model deployed on UE or Network side, without model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Train; test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 96%; {0.6, 6, 2}; 1k labeled & 25k unlabeled; 1k; 1.65M; 22.30M; 5.05. CIR; Pos.; 99%; {0.6, 6, 2}; 0.3k labeled & 25k unlabeled; 1k; 1.65M; 22.30M; 8.78. CIR; Pos.; 0; {0.6, 6, 2}; 1k; 1k; 1.65M; 22.30M; 12.06. CIR; Pos.; 0; {0.6, 6, 2}; 2k; 1k; 1.65M; 22.30M; 9.03. CIR; Pos.; 0; {0.6, 6, 2}; 2k; 1k; 1.65M; 22.30M; 5.53.

- Nokia (R1-2209371) Table 4. Evaluation results for AI/ML model deployed on UE-side, without model generalization, taking into consideration the impact of diverse data for model training and testing. Case; Model input; Model output; Label; Clutter param; Dataset size; Model Complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; Test; Model complexity; Computational complexity; AI/ML. 1; TOA from 18 TRPs; 2D position (x,y); True 2D position (x,y); 40%; ~600 (5%); ~ 11K (95%); 261K; 521K; Training accuracy = 1,69 Test accuracy = 136,7. 2; TOA from 18 TRPs; 2D position (x,y); True 2D position (x,y); 40%; ~ 6K (50%); ~ 6K (50%); 261K; 521K; Training accuracy = 1,19 Test accuracy = 2,59. 3; TOA from 18 TRPs; 2D position (x,y); True 2D position (x,y); 40%; ~ 6K (5% + 47% with Error); ~ 6K (47%); 261K; 521K; Training accuracy = 1,74 Test accuracy = 4,83. Observation-11: ML model trained using a sparse dataset (case 1) performs poorly in terms of horizontal positioning accuracy, especially in comparison to the scenario with much larger training and test dataset (case 2). Observation-12: Augmenting the sparse dataset with noisy labels could provide significant improvements in terms of the positioning accuracy of the AI/ML model. Proposal-12: RAN1 to further study ML model performance with sparse datasets, including possible solutions such as data augmentation.

## Other evaluation results

### Different deployment scenarios (e.g., InF-DH vs InF-HH)

- vivo (R1-2208638) Table 9 Evaluation results of different scenarios for AI/ML model deployed on UE or Network side, with model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6, 6, 2}; 25k & InF-DH; 1k & InF-DH; 1.65M; 22.30M; 0.99. CIR; Pos.; 0; {0.6, 6, 2}; 25k & InF-HH; 1k & InF-HH; 1.65M; 22.30M; 0.63. CIR; Pos.; 0; {0.6, 6, 2}; 25k & InF-SH; 1k & InF-SH; 1.65M; 22.30M; 0.87. CIR; Pos.; 0; {0.6, 6, 2}; 25k & InF-DH; 1k & InF-HH; 1.65M; 22.30M; >10. CIR; Pos.; 0; {0.6, 6, 2}; 25k & InF-DH; 1k & InF-SH; 1.65M; 22.30M; >10.

### Zone-specific changes

- Qualcomm (R1-2209980) Observation8: RFFP method can show improved robustness to slight environment changes such as time-varying blocking when trained on mixture of such changes. Table 7 Horizontal positioning error (meter) for RFFP with Type 3 generalizations (ML model trained on mixture of channel realizations) Train; Test; 50%tile; 67%tile; 80%tile; 90%tile. Mixed clusters; Odd clusters; 1.33; 1.59; 2.19; 2.80. Mixed clusters; Odd except clusters 1&3; 2.41; 3.07; 3.81; 4.87. Mixed clusters; Odd except clusters 1&5; 2.21; 2.97; 3.72; 4.64. Mixed clusters; Odd except clusters 5&7; 1.45; 1.84; 2.42; 3.15. Mixed clusters; Odd with clusters 2&4; 1.39; 1.63; 2.25; 2.89. Mixed clusters; Odd with clusters 6&8; 1.34; 1.60; 2.23; 2.87. Mixed clusters; Remove up to two random odd clusters and add up to two random even ones; 1.50; 1.822; 2.46; 3.17.

### Channel estimation

- vivo (R1-2208638) Table 10 Evaluation results for AI/ML model deployed on UE or Network side, without model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = -30dB; 1k & SINR = -30dB; 1.65M; 22.30M; 6.89. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = -25dB; 1k & SINR = -25dB; 1.65M; 22.30M; 4.31. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = -20dB; 1k & SINR = -20dB; 1.65M; 22.30M; 1.84. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = -10dB; 1k & SINR = -10dB; 1.65M; 22.30M; 1.83. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = 10dB; 1k & SINR = 10dB; 1.65M; 22.30M; 1.46. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = 30dB; 1k & SINR = 30dB; 1.65M; 22.30M; 1.34. Table 11 Evaluation results for AI/ML model deployed on UE or Network side, without model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = 30dB; 1k & SINR = 30dB; 1.65M; 22.30M; 1.34. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = 30dB; 1k & SINR = 10dB; 1.65M; 22.30M; 1.35. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = 30dB; 1k & SINR = -10dB; 1.65M; 22.30M; 2.65. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = 30dB; 1k & SINR = -20dB; 1.65M; 22.30M; 17.89. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = 30dB; 1k & SINR = -25dB; 1.65M; 22.30M; >10. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = 30dB; 1k & SINR = -30dB; 1.65M; 22.30M; >10.

### Positioning with multiple ports data

- vivo (R1-2208638) Table 18 Evaluation results of multiple ports for AI/ML model deployed on UE or Network side, without model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6, 6, 2}; 3k & 8 ports; 1k & 8 ports; 1.65M; 22.30M; 3.14. CIR; Pos.; 0; {0.6, 6, 2}; 3k & 1 port; 1k & 1 ports; 1.65M; 22.30M; 5.53.

### Reduced input dimension

- xiaomi (R1-2209281) Proposal 1: On the basis of satisfying the positioning accuracy requirement, study solution to reduce the model size, computation complexity and involved signalling overhead Table 7 Evaluation results for reduced input dimension for direct AI-based positioning and AI-based ToA predication, model deployed on UE or NW side, without model generalization, ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*24*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.6,6,2}; {0.6,6,2}; 70000; 10000; 21,277,442; 539.94MFlops; 0.8219. 18*24*2 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.6,6,2}; {0.6,6,2}; 70000; 10000; 21,285,650; 539.95MFlops; 0.8993.

## 1st round discussion

Based on the evaluation results submitted by companies, several observations were made by majority companies. No contradictory simulation results were submitted thus far. Accordingly, the following proposals are made, which are the result of merging several companies’ proposals.

Proposal 5.6-1 (overall)

Observation: Direct AI/ML positioning can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods in heavy NLOS scenarios.

|  | Company |
| --- | --- |
| Support | Nokia/NSB, NVIDIA, Apple, InterDigital, Fujitsu, LG, Qualcomm,CMCC, Samsung, CATT, CAICT, Lenovo, Ericsson |
| Not support |  |

| Company | Comments |
| --- | --- |
| Nokia/NSB | We are fine with these observations, however we are not sure about the benefits of agreeing such proposals. It would be beneficial to discuss which observations would progress the study. |
| LG | The purpose of the following proposals 5.6-1,2,3,4 is unclear since it is an overall observation on direct AI/ML positioning |
| OPPO | We agree the observation is valid for many cases according to the submitted contributions. However, it is premature to have some conclusion without any prerequisite (expecting the NLOS scenarios). As companies will submit more evaluation results in the future meetings, we prefer to defer this proposal to see more results of generalization performance. |
| Qualcomm | We are fine with the observation. Similar accuracy improvement observation can be made for mild NLOS and LOS scenarios as well. |
| Samsung | We just want to point out that, we have also shown the evaluation with the generalization aspect (different scenario, different clutter, different hall size), we hope FL can have our simulation results being captured here incase it will be used as TR input. [Moderator] Thanks for checking! Samsung results is captured in Section 5.1, see the last row in the big table. Besides, we want to say that different hall size will be seriously impact the results. Thus, somehow this aspect should be taken into account. |
| ZTE | If this is for TR conclusion, we think is too early to agree this. We should firstly agree how to align the evaluation assumptions. Then, we should consider how to capture the observations submitted from companies. TR conclusion is a final step. |
| HW/HiSi | We think this is a valuable observation and should be captured. It is summarizing part of the progress we have made so far. |
| CATT | Share the same view with HW. |
| CAICT | Agree HW’s point. |
| Lenovo | Support |

Proposal 5.6-2 (generalization over drops)

Observation: When other simulation assumptions are held constant, positioning accuracy of direct AI/ML positioning deteriorates significantly when the ML model is trained with dataset of one drop, and tested with dataset of a different drop (i.e., different set of random seeds for channel propagation).

|  | Company |
| --- | --- |
| Support | Apple, Fujitsu, OPPO, Qualcomm, Ericsson |
| Not support | [HW/HiS – needs also to be captured how performance can improved, there are many simulation results for that] |

| Company | Comments |
| --- | --- |
| Qualcomm | We are fine with the observation. |
| ZTE | OK with the observation. As commented in Proposal 5.6-1, the intention should be clarified. |
| HW/HiSi | This observation is in principle true, but it should be pointed out that enriching the training data set can improve the accuracy to some extent, even is the testing data set is not included in any of the drops used for training. This is e.g. shown in test scenario 2 of Table 4 in our paper (R1-2208433). Also, the performance can be increased with model fine-tuning. We think separate observation should also be made about improved performance through enriched training and/or fine-tuning. Currently these observations are missing, therefore, for the time being, we suggest to to add the following to the proposal: Updated Observation: When other simulation assumptions are held constant, positioning accuracy of direct AI/ML positioning deteriorates significantly when the ML model is trained with dataset of one drop, and tested with dataset of a different drop (i.e., different set of random seeds for channel propagation). - The performance is improved when - the training dataset is composed of multiple drops the accuracy can be improved, even if the drop used for testing is outside the drops used for training. - Model fine-tuning is performed, where the model is re-trained with a small dataset from the same drop as the tested datatset |
| CAICT | We prefer HW’s proposal. |
| Ericsson | Support. Also OK with HW’s update. |

Proposal 5.6-3 (generalization over clutter parameters)

Observation: Positioning accuracy of direct AI/ML positioning deteriorates when the ML model is trained with dataset of one clutter parameter setting (e.g., InF-DH {60%, 6m, 2m}), and tested with dataset of a different clutter parameter setting (e.g., InF-DH {40%, 2m, 2m}). As the clutter parameter setting of training dataset deviates further from that of training, worse positioning accuracy is observed.

|  | Company |
| --- | --- |
| Support | Apple, OPPO, Qualcomm, Samsung, CATT, CAICT |
| Not support | [HW/HiSi] |

| Company | Comments |
| --- | --- |
| InterDigital | The FL’s observation did not capture our results on generalization over clutter parameters. Please capture following results as a part of section 5.2.2 Generalization aspect: different clutter parameters (Row 2, Rable 3). Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. RSRP; UE position; percentage of training data set without label = 0%(default); {60%, 6m, 2m} for training and {40%, 2m, 2m}for testing; 16000 samples; 4000 samples; 108 RSRP values per sample; 18.34 M FLOPs; 3.7447. [Moderator] Thanks for pointing this out! Table 3 of InterDigital contribution is added to section 5.2.2 now, see last entry in the big table. |
| ZTE | OK with the observation. As commented in Proposal 5.6-1, the intention should be clarified. |
| HW/HiSi | Similar to the prvious observation, as soon as the clutter parameters used for testing are included in the dataset for training, the performance is increased. We think that is a valuable finding that should be captured, it is e.g. shown in Table 5 of our paper R1-2208433, where sub-meter accuracy can be reached as soon as the clutter parameters for the testing dataset are one part of the training data set. In practice that would mean that the model can be pre-trained with multiple clutter parameters and will then achieve a good performance during inference. Another option to improve teh performance is fine-tuning that also has been evaulated. If we do not make additional observations how performance cen be improved, we suggest to update the observation as following: Updated Observation: Positioning accuracy of direct AI/ML positioning deteriorates when the ML model is trained with dataset of one clutter parameter setting (e.g., InF-DH {60%, 6m, 2m}), and tested with dataset of a different clutter parameter setting (e.g., InF-DH {40%, 2m, 2m}). As the clutter parameter setting of training dataset deviates further from that of training, worse positioning accuracy is observed. - By using multiple clutter parameters to compose the training dataset, the accuracy is significantly improved if the clutter parameters used for the testing dataset are included in the training dataset. - The performance of the model can also be improved with fine-tuning, where the model is re-trained with a small dataset from the same drop as the tested datatset |
| CATT | Share the same view with HW which can clarify the fine-tuning is needed. But we also can live with current observation. |

Proposal 5.6-4 (generalization over network synchronization error)

Observation: Positioning accuracy of direct AI/ML positioning deteriorates when the ML model is trained with dataset of one network synchronization error setting (e.g., T1=0ns), and tested with dataset of a different network synchronization error setting (e.g., T1=50ns). As the network synchronization error setting of training dataset deviates further from that of training, worse positioning accuracy is observed.

|  | Company |
| --- | --- |
| Support | Apple, Fujitsu, OPPO, Ericsson |
| Not support | Qualcomm, HWHiSI, CATT |

| Company | Comments |
| --- | --- |
| Qualcomm | We prefer to defer agreeing on this observation until more evaluations are conducted. We agree that NW synchronization error can cause degradation to direct AI/ML positioning when this error is not considered during training. But we need to conduct more evaluations to understand whether training on a worse error setting, e.g., T1 = 50 nsec, and testing on a tighter synchronization, e.g., T1 = 20 nsec, can yield robust performance. |
| ZTE | Agree with Qualcomm, training dataset should also include synchronization error. |
| HW/HiSI | This observation seems too strong. It can give the impression that the Gnb synch error can even be larger than 50ns. But 50ns is the maximum error that is defined in the simulations assumptions. Also for this error it has been shown that the performance can be improved by enriching the training data set and/or fine-tuning. We think this valuable information should be added to the observation, if not an independent observation is made: We there suggest to update as follows: Updated Observation: Positioning accuracy of direct AI/ML positioning deteriorates when the ML model is trained with dataset of one network synchronization error setting (e.g., T1=0ns), and tested with dataset of a different network synchronization error setting (e.g., T1=50ns). As the network synchronization error setting of training dataset deviates further from that of training, worse positioning accuracy is observed. - The positioning accuracy can be improved by training the AI model with multiple network synchronization error assumptions and/or model fine-tuning, where the model is trained with a small size data set that experiences the same network synch-error as the testing dataset |
| CATT | Based on the simulation results provided by companies, we thin the generalization performance for network synchronization error deteriorates significantly from 0~1 to larger than 10m. But for Proposal 5.6-3 with different clutter parameters, the performance deteriorates from 0~1 to about 3m. Thus, we think we need differentiate the different level of deterioration. We also share the same view with HW, which can clarify the fine-tuning is needed. |
| CAICt | We prefer to have more simulation results and discussions on the effect of synchornization error before we make a concludion. |

# Performed Evaluation of AI/ML-assisted positioning

In this meeting, a large amount of evaluation work has been performed by companies for AI/ML-assisted positioning. These valuable results are crucial to help RAN1 to make progress.

Selected results submitted by companies are copied below.

## Evaluation without generalization considerations

- Ericsson (R1-2208399) Table 29 Accuracy of the LoS classification results for AI/ML assisted positioning Training dataset; Test dataset; Model I LoS classification accuracy; Model II LoS classification accuracy. {40%, 2m, 2m}; {40%, 2m, 2m} same drop; 0.957; 0.959. {40%, 2m, 2m} new drop; 0.959; 0.960. {50%, 2m, 2m} new drop; 0.960; 0.962. {60%, 2m, 2m} new drop; 0.962; 0.964. {60%, 6m, 2m} new drop; 0.952; 0.954. {40%, 2m, 2m} with no network synchronization errors; {40%, 2m, 2m} STD=0ns; 0.959. {40%, 2m, 2m} STD=2ns; 0.961. {40%, 2m, 2m} STD=6ns; 0.960. {40%, 2m, 2m} STD=20ns; 0.958. {40%, 2m, 2m} STD=50ns; 0.939. {40%, 2m, 2m} with STD = 25 ns network synchronization errors; {40%, 2m, 2m} STD=0ns; 0.948. {40%, 2m, 2m} STD=2ns; 0.950. {40%, 2m, 2m} STD=6ns; 0.954. {40%, 2m, 2m} STD=20ns; 0.956. {40%, 2m, 2m} STD=50ns; 0.955. Table 32 Evaluation results for AI/ML model deployed on network-side, without model generalization investigation. No network synchronization error. Two architectures of the ML model: Architecture I (6 layers: 3 Conv1D layers, 3 Dense layers) and Architecture II (9 layers: 6 Conv1D layers, 3 Dense layers) Model; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training/ validation; test; Model complexity; Computation complexity. I; Time domain CIR, 256x2 complex array; (1). LoS/NLoS classification (2). ToA estimate; Ideal; {40%,2m,2m}; 108,000 link CIR; 72,000 link CIR; 35,447 complex parameters; 404,655 FLOPs; 0.113. II; 36,512 complex parameters; 944,387 FLOPs; 0.063.

- Huawei, HiSilicon (R1-2208433) Table 9. Evaluation results for AI/ML model deployed on network-side, without model generalization Model structure; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CNN; Normalized PDP; LOS probability; Ideal LOS/NLOS identification (LOS probability=0% or 100%); {40%, 2m, 2m}; 18000; 9000; 582; 192K; 0.353.

- ZTE (R1-2208525) Figure.7 CDFs of positioning errors for sub-use case 2-1 (Grid width is 0.5 m)

- vivo (R1-2208638) Table 3. Evaluation results for AI/ML model deployed on UE or Network side, without model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6, 6, 2}; 25k; 1k; 1.65M; 22.30M; 0.99. CIR; TOA; 0; {0.6, 6, 2}; 25k; 1k; 44M; 1.45G; 0.60. CIR; Pos.; 0; {0.6, 6, 2}; 25k & Drop1; 1k & Drop2; 1.65M; 22.30M; 6.00. CIR; TOA; 0; {0.6, 6, 2}; 25k & Drop1; 1k & Drop2; 44M; 1.45G; 2.51. Table 4. CDF of estimation accuracy of intermediate feature TOA (meter) Scenario; Methods; 50%; 67%; 80%; 90%. InF-DH {0.6,6,2}; AI/ML based TOA est. (CIR-TOA); 0.43; 0.86; 1.69; 3.74. Observation 3: AI/ML based TOA estimation for positioning has great advantages in positioning performance, deployment flexibility, compatibility with existing positioning protocol framework, and generalization capability. Table 5. Evaluation results of LOS/NLOS identification accuracy for AI/ML model deployed on UE or Network side, without model generalization, full-connection network Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Accuracy of LOS/NLOS identification. Training; test; Model complexity; Computational complexity; AI/ML. CIR; LOS/NLOS; 0; {0.4, 2, 2}; 25k; 1k; 3.62M; 7.24M; >99%. R17 [9]; {0.4, 2, 2}; /; 93%. Table 6. Evaluation results for AI/ML model deployed on UE or Network side, without model generalization, full-connection network Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; LOS/NLOS; 0; {0.4, 2, 2}; 25k; 1k; 3.62M; 7.24M; 1.10. CIR; TOA; 0; {0.6, 6, 2}; 25k; 1k; 1.65M; 22.30M; 0.60. Observation 4: AI/ML based LOS/NLOS identification for positioning has the following advantages: - More accurate LOS/NLOS identification along with a confidence metric - Better compatibility with existing positioning protocol framework. - Great generalization capability. And disadvantages: - Positioning performance could suffer from severe degradation in heavy-NLOS scenarios. - Obtain LOS/NLOS labels is a challenging task for data collection. Proposal 4: Capture in the TR the benefits of AI/ML assisted positioning in terms of positioning accuracy and AI model generalization.

- Qualcomm (R1-2209980) - Table 8. Evaluation results for AI/ML model deployed on UE-side, without model generalization Method; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Hard-decision; CER (2, 64); Single value of ToA; 0%; {60%, 6m, 2m}; 18000 (1000 Ues * 18 TRPS); 18000 (1000 Ues * 18 TRPS); 22K params; 206K FLOPs; 25.0. Soft-information; CER (2, 64); Distribution of ToA; 0%; {60%, 6m, 2m}; 18000 (1000 Ues * 18 TRPS); 18000 (1000 Ues * 18 TRPS); 22K params; 206K FLOPs; 5.1. - Table 9. Evaluation results for AI/ML model deployed on UE-side, without model generalization Method; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Hard-decision; CER (2, 64); Single value of ToA; 0%; {40%, 4m, 2m}; 18000 (1000 Ues * 18 TRPS); 18000 (1000 Ues * 18 TRPS); 22K params; 206K FLOPs; 14.8. Soft-information; CER (2, 64); Distribution of ToA; 0%; {40%, 4m, 2m}; 18000 (1000 Ues * 18 TRPS); 18000 (1000 Ues * 18 TRPS); 22K params; 206K FLOPs; 0.5. Observation11: The soft-decision algorithm outperforms the hard-decision approach for AI-ML-assisted positioning. - The 90th percentile positioning error improves from 25.0 m to 5.1 m for the {60%, 6m, 2m} clutter setting - The 90th percentile positioning error improves from 14.8 m to 0.5 m for the {40%, 4m, 2m} setting.

- CATT (R1-2208971) Table 7: Evaluation results for AI/ML model deployed on UE/LMF-side, without model generalization, resnet18 Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Type: CIR; Size: [18, 256, 2]; Type: ToA; Size: [1, 18]; ToA with 100% ground truth label; {60%, 6m, 2m}; 20000; 1600; 12.7M; 4.42G FLOPs; 0.77m. Table 8: Evaluation results for AI/ML model deployed on UE/LMF-side, without model generalization, resnet18 Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Type: CIR; Size: [18, 256, 2]; Type: ToA; Size: [1, 2]; ToA with 100% ground truth label; {60%, 6m, 2m}; 20000; 1600; 12.7M; 4.42G FLOPs; 0.7m.

- xiaomi (R1-2209281) Table 2 Evaluation results for AI-based ToA predication with model deployed on UE or NW side, without model generalization, ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*256*2 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.6，6，2}; {0.6，6，2}; 70000; 10000; 21,285,650; 5.76Gflops; 0.6778. 18*256*2 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.4，2，2}; {0.4，2，2}; 70000; 10000; 21,285,650; 5.76Gflops; 0.8533.

- MediaTek (R1-2209510) Sampling period 1ns: Table 1. UE side model/CNN Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. non-normalized PDP(512*1); Los/Nlos probability; Ideal Los/Nlos; {40%, 2m, 2m}; 1800UE*18BS*4drop*0.9 =116640; 1800UE*18BS*4drop*0.1 =12960; 4166; 73.74k; 0.51. normalized PDP(512*1); Los/Nlos probability; Ideal Los/Nlos; {40%, 2m, 2m}; 1800UE*18BS*4drop*0.9 =116640; 1800UE*18BS*4drop*0.1 =12960; 4166; 73.74k; 5.9. Sampling period 9.3ns Table 2. UE side model/CNN Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. non-normalized PDP(256*1); Los/Nlos probability; Ideal Los/Nlos; {40%, 2m, 2m}; 1800UE*18BS*4drop*0.9 =116640; 1800UE*18BS*4drop*0.1 =12960; 2118; 36.876k; 17.2.

## Evaluation of generalization aspects

### Generalization aspect: different drops

- Ericsson (R1-2208399) Table 29 Accuracy of the LoS classification results for AI/ML assisted positioning Training dataset; Test dataset; Model I LoS classification accuracy; Model II LoS classification accuracy. {40%, 2m, 2m}; {40%, 2m, 2m} same drop; 0.957; 0.959. {40%, 2m, 2m} new drop; 0.959; 0.960. {50%, 2m, 2m} new drop; 0.960; 0.962. {60%, 2m, 2m} new drop; 0.962; 0.964. {60%, 6m, 2m} new drop; 0.952; 0.954. {40%, 2m, 2m} with no network synchronization errors; {40%, 2m, 2m} STD=0ns; 0.959. {40%, 2m, 2m} STD=2ns; 0.961. {40%, 2m, 2m} STD=6ns; 0.960. {40%, 2m, 2m} STD=20ns; 0.958. {40%, 2m, 2m} STD=50ns; 0.939. {40%, 2m, 2m} with STD = 25 ns network synchronization errors; {40%, 2m, 2m} STD=0ns; 0.948. {40%, 2m, 2m} STD=2ns; 0.950. {40%, 2m, 2m} STD=6ns; 0.954. {40%, 2m, 2m} STD=20ns; 0.956. {40%, 2m, 2m} STD=50ns; 0.955. Table 30 LoS link ToA estimation error results for AI/ML assisted positioning Training dataset; Test dataset; Model I 90%tile ToA error [m]; Model II 90%tile ToA error [m]. {40%, 2m, 2m}; {40%, 2m, 2m} same drop; 0.164; 0.091. {40%, 2m, 2m} new drop; 0.145; 0.082. {50%, 2m, 2m} new drop; 0.159; 0.090. {60%, 2m, 2m} new drop; 0.152; 0.087. {60%, 6m, 2m} new drop; 0.178; 0.097. {40%, 2m, 2m} with no network synchronization errors; {40%, 2m, 2m} STD=0ns; 0.082. {40%, 2m, 2m} STD=2ns; 0.089. {40%, 2m, 2m} STD=6ns; 0.093. {40%, 2m, 2m} STD=20ns; 0.099. {40%, 2m, 2m} STD=50ns; 0.161. {40%, 2m, 2m} with STD = 25 ns network synchronization errors; {40%, 2m, 2m} STD=0ns; 0.157. {40%, 2m, 2m} STD=2ns; 0.116. {40%, 2m, 2m} STD=6ns; 0.103. {40%, 2m, 2m} STD=20ns; 0.094. {40%, 2m, 2m} STD=50ns; 0.107.

- Qualcomm (R1-2209980) Observation13: The ML-assisted soft information reporting method using single-TRP approach generalizes well across inter-site changes with homogeneous clutter settings. Figure 18. CDF of horizontal positioning error for ML-based soft information reporting across drops

### Generalization aspect: different clutter parameters

- Ericsson (R1-2208399) Table 33. Evaluation results for AI/ML model deployed on network-side, with model generalization investigation where the model is trained in InF-DH {40%, 2m, 2m} and tested with various InF-DH clutter parameters and new drop. No network synchronization error. Two architectures of the ML model: Architecture I (6 layers: 3 Conv1D layers, 3 Dense layers) and Architecture II (9 layers: 6 Conv1D layers, 3 Dense layers) Model; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training/ validation; test; Model complexity; Computation complexity. I; Time domain CIR, 256x2 complex array; (1). LoS/NLoS classification (2). ToA estimate; Ideal; {40%,2m,2m}; 108,000 link CIR; 72,000 link CIR; 35,447 complex parameters; 404,655 FLOPs; 0.106. {50%,2m,2m}; 0.264. {60%, 2m, 2m}; 5.340. {60%, 6m, 2m}; 13.476. II; {40%,2m,2m}; 36,512 complex parameters; 944,387 FLOPs; 0.062. {50%,2m,2m}; 0.150. {60%, 2m, 2m}; 4.732. {60%, 6m, 2m}; 13.528.

- Huawei, HiSilicon (R1-2208433) Table 10. Evaluation results for AI/ML model generalization capability in the aspect: Clutter parameters Evaluated Scenarios; Dataset; Configurations; Dataset size; Identification rate. 1; Training; Clutter paras: InF-DH {40%, 2m, 2m}; 18000; 97.2%. Test; 9000. 2; Training; Clutter paras: InF-DH {40%, 2m, 2m}; 18000; 98.6%. Test; Clutter paras: InF-DH {60%, 6m, 2m}; 9000. 3; Training; Clutter paras: InF-DH {40%, 2m, 2m}; 18000; 97.7%. Test; Clutter paras: InF-DH {40%, 3m, 5m}; 9000.

- Qualcomm (R1-2209980) Observation14: Training on a mix of clutter settings achieves good accuracy in each setting without the overhead of model switching, while training a separate model for each setting provides better accuracy. - The 90th percentile error increases from 5.10 m to 7.34 m when testing on (60%, 6m, 2m) clutter, and - from 0.53 m to 0.91 m when testing on (40%, 2m, 2m) clutter Figure 19. CDF of horizontal positioning error for ML-based soft information reporting across clutter settings

- xiaomi (R1-2209281) Table 4 Evaluation results for AI-based ToA predication with model deployed on UE or NW side, with model generalization (different clutter parameter), ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*256*2 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.6,6,2}; {0.4,2,2}; 70000; 10000; 21,285,650; 5.76GFlops; 7.1173. 18*256*2 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.4,2,2}; {0.6,6,2}; 70000; 10000; 21,285,650; 5.76GFlops; 1.5413. 18*256*2 CIR; 18*1 UE TOA; 18*1 UE TOA; Mix of {0.6,6,2} {0.4,2,2}; {0.6,6,2}; 35000+35000; 10000; 21,285,650; 5.76GFlops; 0.6867. 18*256*2 CIR; 18*1 UE TOA; 18*1 UE TOA; Mix of {0.6,6,2}, {0.4,2,2}; {0.4,2,2}; 35000+35000; 10000; 21,285,650; 5.76GFlops; 0.7974.

- CATT (R1-2208971) Table 9: Evaluation results for AI/ML model deployed on UE/LMF-side, with model generalization, resnet18 Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Type: CIR; Size: [18, 256, 2]; Type: UE’s position; Size: [1, 2]; UE’s position with 100% ground truth label; Training:{60%, 6m, 2m} Testing:{40%, 2m, 2m}; 20000; 1600; 12.7M; 4.42G FLOPs; 3.11m.

### Generalization aspect: network synchronization error

- Ericsson (R1-2208399) Table 34. Evaluation results for AI/ML model deployed on network-side, with model generalization investigation where the model is trained in InF-DH {40%, 2m, 2m} with no network synchronization errors, and tested with InF-DH {40%, 2m, 2m} and a range of network synchronization errors. Architecture II (9 layers: 6 Conv1D layers, 3 Dense layers). Model; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training/ validation; test; Model complexity; Computation complexity. II; Time domain CIR, 256x2 complex array; (1). LoS/NLoS classification (2). ToA estimate; Ideal; {40%,2m,2m}, T1=0ns; 108,000 link CIR; 72,000 link CIR; 36,512 complex parameters; 944,387 FLOPs; 0.062. {40%,2m,2m}, T1=2ns; 0.997. {40%,2m,2m}, T1=6ns; 2.926. {40%,2m,2m}, T1=20ns; 9.200. {40%,2m,2m}, T1=50ns; 22.149. Table 35. Evaluation results for AI/ML model deployed on network-side, with model generalization investigation where the model is trained in InF-DH {40%, 2m, 2m} with network synchronization error STD T1=25ns, and tested with InF-DH {40%, 2m, 2m} and a range of network synchronization errors. Architecture II (9 layers: 6 Conv1D layers, 3 Dense layers). Model; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training/ validation; test; Model complexity; Computation complexity. II; Time domain CIR, 256x2 complex array; (1). LoS/NLoS classification (2). ToA estimate; Ideal; {40%,2m,2m}, T1=0 ns; 108,000 link CIR; 72,000 link CIR; 36,512 complex parameters; 944,387 FLOPs; 0.126. {40%,2m,2m}, T1=2 ns; 1.096. {40%,2m,2m}, T1=6 ns; 2.945. {40%,2m,2m}, T1=20 ns; 9.100. {40%,2m,2m}, T1=50 ns; 21.459.

- CATT (R1-2208971) Table 10: Evaluation results for AI/ML model deployed on UE/LMF-side, with model generalization [Training dataset with perfect network synchronization and testing dataset with network synchronization error], resnet18 Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Type: CIR; Size: [18, 256, 2]; Type: UE’s position; Size: [1, 2]; UE’s position with 100% ground truth label; {60%, 6m, 2m}; 20000; 1600; 12.7M; 4.42G FLOPs; 12.8m.

## Evaluation of model fine-tuning

- vivo (R1-2208638) Table 16 Evaluation results of fine-tuning for AI/ML model deployed on UE or Network side, with model generalization, CNN Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Train; Fine-tune; test; Model complexity; Computational complexity; AI/ML. CIR; TOA; 0; {0.6, 6, 2}; 25k & InF-DH; 0; 1k & InF-HH; 44M; 1.45G; >10. CIR; TOA; 0; {0.6, 6, 2}; 25k & InF-HH; 0; 1k & InF-DH; 44M; 1.45G; >10. CIR; TOA; 0; {0.6, 6, 2}; 25k & InF-DH; 1k& InF-HH; 1k & InF-HH; 44M; 1.45G; 0.28.

## Evaluation of issues related to ground truth labels

- Nokia (R1-2209371) Table 2: Model information and evaluation results for AI/ML model for on-demand labelling and random labelling. Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Required training data size to reach accuracy at 82%. Training; Test; Model complexity; Computational complexity; AI/ML. CIR from 1 BS; Classification 0/1; LOS (1) / NLOS (0); 40%; 10K including evaluation; 2K; 0.31M; 4.33M flops; 680 (red line in Fig. 2). Degree of classification confidence. CIR from 1 BS; Classification 0/1; LOS (1) / NLOS (0); 40%; 10K including evaluation; 2K; 0.24M; 2.38M flops; 2010 (blue line in Fig. 2).

## Other evaluation results

### Zone-specific changes

- Qualcomm (R1-2209980) Observation15: The ML-assisted soft information reporting using single-TRP approach generalizes well across zone-specific changes. Figure 20. CDF of horizontal positioning error for ML-based soft information reporting across zone-specific generalizations.

### Channel estimation

- vivo (R1-2208638) Table 10 Evaluation results for AI/ML model deployed on UE or Network side, without model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = -30dB; 1k & SINR = -30dB; 1.65M; 22.30M; 6.89. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = -25dB; 1k & SINR = -25dB; 1.65M; 22.30M; 4.31. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = -20dB; 1k & SINR = -20dB; 1.65M; 22.30M; 1.84. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = -10dB; 1k & SINR = -10dB; 1.65M; 22.30M; 1.83. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = 10dB; 1k & SINR = 10dB; 1.65M; 22.30M; 1.46. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = 30dB; 1k & SINR = 30dB; 1.65M; 22.30M; 1.34.

### Reduced input dimension

- xiaomi (R1-2209281) Proposal 1: On the basis of satisfying the positioning accuracy requirement, study solution to reduce the model size, computation complexity and involved signalling overhead Table 7 Evaluation results for reduced input dimension for direct AI-based positioning and AI-based ToA predication, model deployed on UE or NW side, without model generalization, ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*24*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.6,6,2}; {0.6,6,2}; 70000; 10000; 21,277,442; 539.94MFlops; 0.8219. 18*24*2 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.6,6,2}; {0.6,6,2}; 70000; 10000; 21,285,650; 539.95MFlops; 0.8993.

## 1st round discussion

Based on the evaluation results submitted by companies, several observations were made by majority companies. No contradictory simulation results were submitted thus far. Accordingly, the following proposals are made, which are the result of merging several companies’ proposals.

Proposal 6.6-1 (overall)

Observation: AI/ML assisted positioning can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods in low to medium NLOS scenarios (e.g., InF-DH {40%, 2m, 2m}).

|  | Company |
| --- | --- |
| Support | Qualcomm, Ericsson |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | In our contribution, we provided the results of AI/ML assisted positioning for heavy NLOS scenario (i.e., InF-DH {60%, 6m, 2m}) and showed significant gain as well. It's not clear to us why the observation only capturing gain in low to medium NLOS scenarios. |
| Nokia/NSB | We have similar concerns as vivo, and are not fully sure about the intent of the observation. If the intermediate feature that is the output of the AI/ML model is LOS/NLOS classification, we agree that such models are beneficial for low-to-medium NLOS scenarios. Perhaps it would be beneficial to clarify for which model outputs the observation is applicable. |
| LG | We have a similar view with Nokia. |
| OPPO | it is premature to have some conclusion without any prerequisite (expecting the NLOS scenarios). As companies will submit more evaluation results in the future meetings, we prefer to defer this proposal to see more results of generalization performance. |
| Qualcomm | We are ok with the observation |
| CMCC | We also see the performance gain in high NLOS scenario. |
| ZTE | Agree with vivo, even in heavy NLOS scenario, AI/ML assisted positioning can work well. |
| HW/HiSi | Agree with the observation. As a compromise, to address the previous comments, we can also remove the last part: Compromise updated Proposal 6.6-1 (overall) Observation: AI/ML assisted positioning can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods in low to medium NLOS scenarios (e.g., InF-DH {40%, 2m, 2m}). |
| CATT | In our contribution, we provide the results in high NLOS scenario (i.e., InF-DH {60%, 6m, 2m}) and showed significant gain as well. Agree with HW’s update. |
| Lenovo | The positive effect on heavy NLOS scenarios should also be considered as well as part of the general observation. |

Proposal 6.6-2 (generalization over drops)

Observation: ML model in AI/ML assisted positioning has robust generalization capability when the model is trained with dataset of one drop, and tested with dataset of a different drop (i.e., different set of random seeds for channel propagation).

|  | Company |
| --- | --- |
| Support |  |
| Not support | Qualcomm |

| Company | Comments |
| --- | --- |
| Qualcomm | We propose to add wording to reflect that this observation only applies to AI/ML assisted positioning that considers single TRP for model input. There is still no sufficient evaluations that show the robustness of AI/ML assisted positioning with multiple TRP input to different drops or clutter settings. |
| ZTE | Wait for more evaluation results in next meeting. This is related to Proposal 2.1.1-1. |
| HW/HISi | For LOS identification, this observation is true. But for ToA there are not enough results yet to observe this. We can agree now on the following: Updated Proposal 6.6-2 (generalization over drops) Observation: ML model in AI/ML assisted positioning has robust generalization capability at least for LOS identification when the model is trained with dataset of one drop, and tested with dataset of a different drop (i.e., different set of random seeds for channel propagation). FFS: ToA estimation |
| CATT | We prefer to wait for more evaluation results |

Proposal 6.6-3 (generalization over clutter parameters)

Observation: Even with high-quality input (e.g., LOS/NLOS identification) provided by ML model, positioning accuracy of the conventional positioning algorithm is limited by the inadequate LOS links in heavy NLOS scenario.

|  | Company |
| --- | --- |
| Support |  |
| Not support | Qualcomm, CATT |

| Company | Comments |
| --- | --- |
| Qualcomm | Observation is not related to generalization |
| HW/HiSi | It does not seem right to make an observation about the conventional algorithm in the context of generalization. Could it therefore please be clarified how the utilization of conventional algorithms relates to the generalization of the AI model over clutter parameters? In the proposed observation it is said that the output from the AI model has high quality. Then, for generalization, we can conclude that the AI performs well and do need to do any observation. |
| CATT | We wonder why we just focus on LOS/NLOS identification. From our simulation results, using TOA can still have some performance gain in heavy NLOS scenario. |

# Template for reporting evaluation results

Regarding the template for reporting evaluation results, the following suggestions were made.

- Huawei, HiSilicon (R1-2208433) Proposal 6: RAN1 should define a unified table structure to present results of generalization studies. It is proposed to have a different table for each dimension of generalization that is investigated, e.g. one for different drops, one for different clutter parameters, etc. One example is given in the template below. Table X. Evaluation results for AI/ML model generalization capability in the aspect: Different Drops Source; Evaluated Scenarios; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). Huawei, HiSilicon; 1; Training; Drop i; M. Test; Drop j; N. Proposal 7: RAN1 should define typical evaluation cases for model generalization in the aspect - Different Drops, to better compare results across companies. In addition, further cases should also be captured to better explore and expand the generalization capabilities of the AI/ML model. One example is given in the template below. Table X. Evaluation results for AI/ML model generalization capability in the aspect: Different Drops Source; Evaluated Scenarios; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). Huawei, HiSilicon; 1; Training; Drop i; M. Test; Drop j; N. 2; Training; k Drops mixed; M (M/k per drop). Test; Drop I (outside of the trained Drops); N. 3; Training; k Drops mixed; M (M/k per drop). Test; Drop j (inside the trained Drops); N. …; Training; …. Test; …. Proposal 8: RAN1 should define typical evaluation cases for model generalization in the aspect - Clutter parameters, to better compare results of generalization studies among companies. In addition, further cases should also be captured to better explore and expand the generalization capabilities of the AI/ML model. One example is given in the template below. Table X. Evaluation results for AI/ML model generalization capability in the aspect: Clutter parameters Source; Evaluated Scenarios; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). Huawei, HiSilicon; 1; Training; Clutter paras: a; M. Test; Clutter paras: b; N. 2; Training; Clutter paras: a&b mixed; M (M/2 per paras). Test; Clutter paras: a; N. 3; Training; Clutter paras: a&b mixed; M (M/2 per paras). Test; Clutter paras: b; N. …; Training; …. Test; …. Proposal 10: RAN1 should define typical evaluation cases for model generalization in the aspect - Network synchronization error, to better compare results of generalization studies among companies. In addition, further cases should also be captured to better explore and expand the generalization capabilities of the AI/ML model. One example is given in the template below. Table X. Evaluation results for AI/ML model generalization capability in the aspect: Network synchronization error Source; Evaluated Scenarios; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). Huawei, HiSilicon; 1; Training; Without network synchronization error; M. Test; With network synchronization error @A ns; N. 2; Training; With network synchronization error @A ns; M. Test; N. 3; Training; With network synchronization error @ B paras mixed; M (M/ B per paras). Test; N (N/ B per paras). …; Training; …. Test; …. Proposal 12: RAN1 should define some typical evaluation cases for evaluate the generalization in the aspect - UE timing error to better compare results of generalization studies among companies. In addition, further cases should also be captured to better explore and expand the generalization capabilities of the AI/ML model. One example is given in the template below. Table X. Evaluation results for AI/ML model generalization capability in the aspect: UE timing error Source; Evaluated Scenarios; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). Huawei, HiSilicon; 1; Training; Without UE timing error; M. Test; With UE timing error @A ns; N. 2; Training; With UE timing error @A ns; M. Test; N. 3; Training; With UE timing error @ B paras mixed; M (M/ B per paras). Test; N (N/ B per paras). …; Training; …. Test; …. Proposal 14: RAN1 should define typical cases to evaluate cases the performance gain brought by fine-tuning to better compare results among companies. One example is given in the template below. Table X. Model update evaluation results for AI/ML model generalization capability Source; Evaluated aspects; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). Before model update; After model update. Huawei, HiSilicon; Different Drops; Training; Drop i; M. Fine-tuning; Drop j; K. Test; Drop j; N. Clutter parameters; Training; Clutter paras: a; M. Fine-tuning; Clutter paras: b; K. Test; Clutter paras: b; N. UE timing error; Training; Without UE timing error; M. Fine-tuning; With UE timing error @A ns; K. Test; With UE timing error @A ns; N. …; Training; …. Fine-tuning. Test; ….

## 1st round discussion

The main point in Huawei/HiSi proposal is, separate entries are necessary to report the conditions used in obtaining the training/validation dataset, vs the condition used in obtaining the test dataset. Thus it is sufficient to expand the agreed template to as below. Companies can describe in details in settings used in training/validation and testing: drops, clutter parameters, network synchronization error, mixed data from two different drops, …

Note that this template has been used by companies in their contributions, e.g.,xiaomi (R1-2209281).

Based on the above, the template for reporting evaluation results is updated to the following.

Proposal 7.1-1

For both direct AI/ML positioning and AI/ML assisted positioning, the agreed table is updated to the following for reporting the evaluation results.

Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [short model description]

| Model input | Model output | Label | Settings (e.g., drops, Clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |

*Note: for AI/ML assisted positioning, if it is agreed that single-TRP vs multi-TRP should be reported, then this aspect can be added to table caption.

|  | Company |
| --- | --- |
| Support | Nokia/NSB, InterDigital, Fujitsu, LG, Qualcomm,CMCC, amsung, ZTE, CATT, CAICT |
| Not support |  |

| Company | Comments |
| --- | --- |
| Nokia/NSB | We are fine with this proposal. |
| OPPO | The template should not be mandated for the tdoc. In order to get some observation, companies may use different tables for easy comparison. One suggestion is that the moderator can prepare an excel file, and all companies inputs their values for the collection of evaluation results. |
| Qualcomm | We are ok with the proposal. |
| HW/HiSi | In principle fine with the proposal. But model structure could be reported in first column. One additional point that we hope we can spend time on during this meeting is to agree on typical cases for evaluating the generalization. This could then be compared across companies |

# Proposals for online discussion

## 1st round proposals for online discussion

Proposal 2.1.1-1

For AI/ML-assisted positioning, companies report which construction is applied in their evaluation:

- Single-TRP construction: the input of the ML model is the channel measurement between the target UE and a single TRP, and the output of the ML model is for the same pair of UE and TRP.

- Multi-TRP construction: the input of the ML model contains N sets of channel measurements between the target UE and N (N>1) TRPs, and the output of the ML model contains N sets of values, one for each of the N TRPs.

Proposal 2.2.1-1

To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning:

(e) InF scenarios, e.g., training dataset from one InF scenario (e.g., InF-DH), test dataset from a different InF scenario (e.g., InF-HH)

Proposal 3.1.1-1

For evaluation of AI/ML assisted positioning, the following intermediate performance metrics are used:

- LoS classification accuracy, if the model output includes LoS classification;

- Timing estimation accuracy (expressed in meters), if the model output includes timing estimation (e.g., ToA, RSTD).

- Angle estimation accuracy (in degrees), if the model output includes angle estimation (e.g., AoA, AoD).

Proposal 5.6-1

Observation: Direct AI/ML positioning can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods in heavy NLOS scenarios.

Proposal 5.6-2

Observation: When other simulation assumptions are held constant, positioning accuracy of direct AI/ML positioning deteriorates significantly when the ML model is trained with dataset of one drop, and tested with dataset of a different drop (i.e., different set of random seeds for channel propagation).

- The performance can be improved by better training dataset construction and/or model fine-tuning, for example:

  - The training dataset is composed of data from multiple drops;

  - Model fine-tuning is performed, where the model is re-trained with a small dataset from the same drop as the test datatset.

Proposal 7.1-1

For both direct AI/ML positioning and AI/ML assisted positioning, the template agreed in RAN1#110 is updated to the following for reporting the evaluation results.

Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [short model description]

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |

*Note: for AI/ML assisted positioning, if it is agreed that single-TRP vs multi-TRP should be reported, then this aspect can be added to table caption.

# Conclusion

TBD

# References

RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” 3GPP RAN#96, June 2022.

R1-2208399	Evaluation of AI/ML for Positioning Accuracy Enhancement	Ericsson

R1-2208433	Evaluation on AI/ML for positioning accuracy enhancement	Huawei, HiSilicon

R1-2208525	Evaluation on AI for positioning enhancement	ZTE

R1-2208638	Evaluation on AI/ML for positioning accuracy enhancement	vivo

R1-2208772	Evaluation on AI/ML for positioning accuracy enhancement	China Telecom

R1-2208854	Evaluation methodology and preliminary results on AI/ML for positioning accuracy enhancement	OPPO

R1-2208882	On Evaluation of AI/ML based Positioning	Google

R1-2208903	Evaluation on AI/ML for positioning accuracy enhancement	LG Electronics

R1-2208971	Evaluation on AI/ML for positioning enhancement	CATT

R1-2209015	Discussions on evaluation of AI positioning accuracy enhancement	Fujitsu

R1-2209124	Discussion on AI/ML Positioning Evaluations	Lenovo

R1-2209234	Some discussions on evaluation on AI-ML for positioning accuracy enhancement	CAICT

R1-2209281	Evaluation on AI/ML for positioning accuracy enhancement	xiaomi

R1-2209332	Discussion on evaluation on AI/ML for positioning accuracy enhancement	CMCC

R1-2209371	Evaluation of ML for positioning accuracy enhancement	Nokia, Nokia Shanghai Bell

R1-2209484	Evaluation on AI/ML for positioning accuracy enhancement	InterDigital, Inc.

R1-2209510	Evaluation on AI/ML for positioning accuracy enhancement	MediaTek Inc.

R1-2209537	Evaluation on AI/ML for positioning accuracy enhancement 	Faunhofer IIS, Fraunhofer HHI

R1-2209580	Evaluation on AI/ML for positioning accuracy enhancement	Apple

R1-2209615	Evaluation of AI/ML based positioning accuracy enhancement	Rakuten Symphony

R1-2209629	Evaluation of AI and ML for positioning enhancement	NVIDIA

R1-2209726	Evaluation on AI ML for Positioning	Samsung

R1-2209980	Evaluation on AI/ML for positioning accuracy enhancement	Qualcomm Incorporated

# Appendix A. Selected Agreements from RAN1#109e

In RAN1#109e meeting for the AI PHY SI, the following agreements were made for AI 9.2.4.1 (Evaluation on AI/ML for positioning accuracy enhancement).

Agreement The IIoT indoor factory (InF) scenario is a prioritized scenario for evaluation of AI/ML based positioning. Agreement For evaluation of AI/ML based positioning, at least the InF-DH sub-scenario is prioritized in the InF deployment scenario for FR1 and FR2. Agreement For InF-DH channel, the prioritized clutter parameters {density, height, size} are: - {60%, 6m, 2m}; - {40%, 2m, 2m}. - Note: an individual company may treat {40%, 2m, 2m} as optional in their evaluation considering their specific AI/ML design. Agreement For evaluation of AI/ML based positioning, reuse the common scenario parameters defined in Table 6-1 of TR 38.857. Agreement For evaluation of InF-DH scenario, the parameters are modified from TR 38.857 Table 6.1-1 as shown in the table below. - The parameters in the table are applicable to InF-DH at least. If another InF sub-scenario is prioritized in addition to InF-DH, some parameters in the table below may be updated. Agreement For AI/ML-based positioning evaluation, the baseline performance to compare against is that of existing Rel-16/Rel-17 positioning methods. - As a starting point, each participating company report the specific existing positioning method (e.g., DL-TDOA, Multi-RTT) used as comparison. Agreement For all scenarios and use cases, the main KPI is the CDF percentiles of horizonal accuracy. - Companies can optionally report vertical accuracy. Agreement The CDF percentiles to analyse are: {50%, 67%, 80%, 90%}. - 90% is the baseline. {50%, 67% 80%} are optional. Agreement Target positioning requirements for horizonal accuracy and vertical accuracy are not defined for AI/ML-based positioning evaluation. Agreement For evaluation of AI/ML based positioning, the KPI include the model complexity and computational complexity. - FFS: the details of model complexity and computational complexity Agreement Synthetic dataset generated according to the statistical channel models in TR38.901 is used for model training, validation, and testing. Agreement The dataset is generated by a system level simulator based on 3GPP simulation methodology. Agreement As a starting point, the training, validation and testing dataset are from the same large-scale and small-scale propagation parameters setting. Subsequent evaluation can study the performance when the training dataset and testing dataset are from different settings. Agreement For AI/ML-based positioning evaluation, RAN1 does not attempt to define any common AI/ML model as a baseline. Agreement The entry “UE horizontal drop procedure” in the simulation parameter table for InF is updated to the following. UE horizontal drop procedure; Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be selected from - (baseline) the whole hall area, and the CDF values for positioning accuracy is obtained from whole hall area. - (optional) the convex hull of the horizontal BS deployment, and the CDF values for positioning accuracy is obtained from the convex hull.. Agreement The entries “UE antenna height” and “gNB antenna height” in the simulation parameter table for InF is updated to the following. UE antenna height; Baseline: 1.5m (Optional): uniformly distributed within [0.5, X2]m, where X2 = 2m for scenario 1(InF-SH) and X2= for scenario 2 (InF-DH). …; …. gNB antenna height; Baseline: 8m (Optional): two fixed heights, either {4, 8} m, or {max(4,), 8}.. Agreement If spatial consistency is enabled for the evaluation, companies model at least one of: large scale parameters, small scale parameters and absolute time of arrival, where - the large scale parameters are according to Section 7.5 of TR 38.901 and correlation distance = for InF (Section 7.6.3.1 of TR 38.901) - the small scale parameters are according to Section 7.6.3.1 of TR 38.901 - the absolute time of arrival is according to Section 7.6.9 of TR 38.901 Agreement If spatial consistency is enabled for the evaluation of AI/ML based positioning, the baseline evaluation does not incorporate spatially consistent UT/BS mobility modelling (Section 7.6.3.2 of TR 38.901). - It is optional to implement spatially consistent UT/BS mobility modelling (Section 7.6.3.2 of TR 38.901). Agreement For evaluation of AI/ML based positioning, companies are encouraged to evaluate the model generalization. - FFS: the metrics for evaluating the model generalization (e.g., model performance based on agreed KPIs under different settings) Agreement Companies are encouraged to provide evaluation results for: - Direct AI/ML positioning - Companies are encouraged to describe at least the following implementation details for the evaluation - details of the channel observation used as the input of the AI/ML model inference (e.g., type and size of model input), model input acquisition and pre-processing - AI/ML assisted positioning - Companies are encouraged to describe at least the following implementation details for the evaluation - details of the channel observation used as the input of the AI/ML model inference (e.g., type and size of model input), model input acquisition and pre-processing - details of the output of the AI/ML model inference, how the AI/ML model output is used to obtain the UE’s location Agreement When reporting evaluation results with direct AI/ML positioning and/or AI/ML assisted positioning, proponent company is expected to describe if a one-sided model or a two-sided model is used. - If one-sided model (i.e., UE-side model or network-side model), the proponent company report which side the model inference is performed (e.g. UE, network), and any details specific to the side that performs the AI/ML model inference. - If two-sided model, the proponent company report which side (e.g., UE, network) performs the first part of interference, and which side (e.g., network, UE) performs the remaining part of the inference. Agreement For evaluation of AI/ML based positioning, the computational complexity can be reported via the metric of floating point operations (FLOPs). - Note: For AI/ML assisted methods, computational complexity for the AI/ML model is only one component of the overall complexity for estimating the UE’s location. - Note: Other metrics to measure the computational complexity are not precluded. Agreement For evaluation of AI/ML based positioning, details of the training dataset generation are to be reported by proponent company. The report may include (in addition to other selected settings, if applicable): - The size of training dataset, for example, the total number of UEs in the evaluation area for generating training dataset; - The distribution of UE location for generating the training dataset may be one of the following: - Option 1: grid distribution, i.e., one training data is collected at the center of one small square grid, where, for example, the width of the square grid can be 0.25/0.5/1.0 m. - Option 2: uniform distribution, i.e., the UE location is randomly and uniformly distributed in the evaluation area.

The following agreements were made for AI 9.2.4.2 (Other aspects on AI/ML for positioning accuracy enhancement).

Agreement Study further on sub use cases and potential specification impact of AI/ML for positioning accuracy enhancement considering various identified collaboration levels. - Companies are encouraged to identify positioning specific aspects on collaboration levels if any in agenda 9.2.4.2. - Note1: terminology, notation and common framework of Network-UE collaboration levels are to be discussed in agenda 9.2.1 and expected to be applicable to AI/ML for positioning accuracy enhancement. - Note2: not every collaboration level may be applicable to an AI/ML approach for a sub use case Agreement For further study, at least the following aspects of AI/ML for positioning accuracy enhancement are considered. - Direct AI/ML positioning: the output of AI/ML model inference is UE location - E.g., fingerprinting based on channel observation as the input of AI/ML model - FFS the details of channel observation as the input of AI/ML model, e.g. CIR, RSRP and/or other types of channel observation - FFS: applicable scenario(s) and AI/ML model generalization aspect(s) - AI/ML assisted positioning: the output of AI/ML model inference is new measurement and/or enhancement of existing measurement - E.g., LOS/NLOS identification, timing and/or angle of measurement, likelihood of measurement - FFS the details of input and output for corresponding AI/ML model(s) - FFS: applicable scenario(s) and AI/ML model generalization aspect(s) - Companies are encouraged to clarify all details/aspects of their proposed AI/ML approaches/sub use case(s) of AI/ML for positioning accuracy enhancement Agreement Companies are encouraged to study and provide inputs on potential specification impact at least for the following aspects of AI/ML approaches for sub use cases of AI/ML for positioning accuracy enhancement. - AI/ML model training - training data type/size - training data source determination (e.g., UE/PRU/TRP) - assistance signalling and procedure for training data collection - AI/ML model indication/configuration - assistance signalling and procedure (e.g., for model configuration, model activation/deactivation, model recovery/termination, model selection) - AI/ML model monitoring and update - assistance signalling and procedure (e.g., for model performance monitoring, model update/tuning) - AI/ML model inference input - report/feedback of model input for inference (e.g., UE feedback as input for network side model inference) - model input acquisition and pre-processing - type/definition of model input - AI/ML model inference output - report/feedback of model inference output - post-processing of model inference output - UE capability for AI/ML model(s) (e.g., for model training, model inference and model monitoring) - Other aspects are not precluded - Note: not all aspects may apply to an AI/ML approach in a sub use case - Note2: the definitions of common AI/ML model terminologies are to be discussed in agenda 9.2.1