3GPP TSG RAN WG1 #110bis-e		R1-2210445

e-Meeting, October 10th – 19th, 2022

Agenda item:	7.2

Source: 	Moderator (ZTE)

Title: 	Moderator Summary #1 of [110bis-e-NR-R16-06] Discussion on SL BWP

Document for:	Discussion and decision

Introduction

This contribution synthesizes companies' views towards the CR in R1-2209173.

Discussions

- Round 1 disucssion

First correction on SL BWP in TS 38.213

16	 UE procedures for sidelink

A UE is provided by SL-BWP-Config or SL-BWP-ConfigCommon a BWP for SL transmissions (SL BWP) with numerology and resource grid determined as described in [4, TS 38.211]. For a resource pool within the SL BWP, the UE is provided by sl-NumSubchannel a number of sub-channels where each sub-channel includes a number of contiguous RBs provided by sl-SubchannelSize. The first RB of the first sub-channel in the SL BWP is indicated by sl-StartRB-Subchannel. Available slots for a resource pool are provided by sl-TimeResource and occur with a periodicity of 10240 ms. For an available slot without S-SS/PSBCH blocks, SL transmissions can start from a first symbol indicated by sl-StartSymbol and be within a number of consecutive symbols indicated by sl-LengthSymbols. For an available slot with S-SS/PSBCH blocks, the first symbol and the number of consecutive symbols is predetermined.

<Unchanged parts are omitted>

Second correction on SL BWP in TS 38.213

16.5.1.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a SL BWP on a carrier, and an active UL BWP on the primary cell, as described in clause 12, a UE determines a set MA of occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on:

a)	a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-PSFCH-ToPUCCH for DCI format 3_0 or by sl-PSFCH-ToPUCCH-CG-Type1

b)	the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in SL-BWP-Config and BWP-Uplink for the SL BWP and the active UL BWP, respectively

c)	a configured sidelink resource pool bitmap

d)	a value of a period of PSFCH transmission occasion resources for a sidelink resource pool provided by a respective sl-PSFCH-Period

<Unchanged parts are omitted>

Q1 Please indicate your views on whether the above changes are needed and other suggestions if any.

| Company | View |
| --- | --- |
| ZTE,Sanechips | Ok to both changes. The second change (correction in S2.2) should be alignment CR while the first change (correction in S2.1), if agreed, should be company's CR. |
| Intel | We are Ok with the changes. |
| Ericsson | OK |
| Huawei, HiSilicon | OK, should the second change also mention SL-BWP-ConfigCommon? (Whereas BWP-Uplink already includes BWP-UplinkCommon). |
| OPPO | We are OK with the changes. |
| vivo | We are fine with the first change. Regarding the second change, it would be better to change to SL-BWP-Generic, because the SCS configuration is provided in SL-BWP-Generic, which in turn included in either SL-BWP-Config or SL-BWP-ConfigCommon. |
| Samsung | Ok to both changes. We think that the second one should be alignment CR (editorial change). Also, we think that the first one is an obvious missing and editorial change. |
| Nokia, Nokia Shanghai Bell | We are fine with the changes. |
| ZTE, Sanechips | The second change is for mode 1 only in HARQ ACK codebook generation, wherein the information in SL-BWP-ConfigCommon is not relevant. |
| Sharp | We are fine with both changes and share similar view with HW. |

- Outcome of Round 1 discussion

Correction on SL BWP in TS 38.213

<Unchanged parts are omitted>

16	 UE procedures for sidelink

A UE is provided by SL-BWP-Config or SL-BWP-ConfigCommon a BWP for SL transmissions (SL BWP) with numerology and resource grid determined as described in [4, TS 38.211]. For a resource pool within the SL BWP, the UE is provided by sl-NumSubchannel a number of sub-channels where each sub-channel includes a number of contiguous RBs provided by sl-SubchannelSize. The first RB of the first sub-channel in the SL BWP is indicated by sl-StartRB-Subchannel. Available slots for a resource pool are provided by sl-TimeResource and occur with a periodicity of 10240 ms. For an available slot without S-SS/PSBCH blocks, SL transmissions can start from a first symbol indicated by sl-StartSymbol and be within a number of consecutive symbols indicated by sl-LengthSymbols. For an available slot with S-SS/PSBCH blocks, the first symbol and the number of consecutive symbols is predetermined.

<Unchanged parts are omitted>

16.5.1.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a SL BWP on a carrier, and an active UL BWP on the primary cell, as described in clause 12, a UE determines a set MA of occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on:

a)	a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-PSFCH-ToPUCCH for DCI format 3_0 or by sl-PSFCH-ToPUCCH-CG-Type1

b)	the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in SL-BWP-Config or SL-BWP-ConfigCommon and BWP-Uplink for the SL BWP and the active UL BWP, respectively

c)	a configured sidelink resource pool bitmap

d)	a value of a period of PSFCH transmission occasion resources for a sidelink resource pool provided by a respective sl-PSFCH-Period

<Unchanged parts are omitted>

Conclusion

The following agreement is made concluding the discussion.

Agreement

Adopt the correction on SL BWP in R1-2210469 and R1-2210470. Final CRs:

R1-2210513 CR#0368 Cat-F 38.213 (Rel-16)

R1-2210514 CR#0369 Cat-A 38.213 (Rel-17)

References

[1]	R1-2209173, “Correction on SL BWP in TS 38.213”, ZTE,Sanechips, RAN1 #110b-e, October 2022