GPP TSG RAN WG1 #110bis-e	R1-2210463

e-Meeting, October 10th – 19th, 2022

Agenda item:	9.8.2

Source:	Moderator (Fujitsu)

Title:	Summary#2 on other aspects

Document for:	Discussion and Decision

- Introduction

This contribution summarizes the proposals in the contributions submitted under AI 9.8.2 about other aspects on NCR, based on the objectives of WID other than side control information and NCR behaviour in RP-222673.

The objectives of NR NCR WI follow the recommendations defined in TR 38.867 and will focus on scenarios and assumption listed below: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands based on the NCR model in TR38.867 - For only single hop stationary network-controlled repeaters - The NCR is transparent to the UE. - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously With these considerations, NR NCR supports the following features: Specify the signalling and behavior of the following side control information for controlling the NCR-Fwd [RAN1, RAN2] - Beamforming - UL-DL TDD operation - ON-OFF information Note: Power control aspect will be checked in RAN#98e. Specify control plane signalling and procedures [RAN2, RAN1] - The configuration of signalling for side control information indication - NOTE: Down-selection of solutions in section 7.2 of TR 38.867 is needed Specify the solution of network-controlled repeater management (i.e., the identification and authorization/validation of NCR) [RAN3, RAN2] - NOTE: Down-selection of solutions in section 8 of TR 38.867 is needed taking into account the feedback of other working groups (i.e., SA3 and SA5). From a security point of view, the feasibility of NCR validation procedure in solution 1 and the feasibility of solution 2 will be decided by SA3.The selected solution shall provide inter-vendor interoperability. Study the RRM functions to be supported and specify the RRM requirements of NCR-MT if necessary [RAN2, RAN4] Study and specify the RF and EMC requirements of NCR if necessary [RAN4] Note: The existing requirements defined in RAN4 can be reused if applicable. Note: The work in RAN4 for beam related is expected to start on FR2 first.

The model of NCR is as follows according to TR 38.867:

Agreement Capture the following model of network-controlled repeater in TR 38.867. - The NCR-MT is defined as a function entity to communicate with a gNB via Control link (C-link) to enable the information exchanges (e.g. side control information). The C-link is based on NR Uu interface. - Note: Side control information is at least for the control of NCR-Fwd - The NCR-Fwd is defined as a function entity to perform the amplify-and-forwarding of UL/DL RF signal between gNB and UE via backhaul link and access link. The behavior of the NCR-Fwd will be controlled according to the received side control information from gNB.

- Summary

  - The configuration for NCR-MT

In RAN1#109e meeting (May), the following agreement was achieved:

Agreement The NCR-MT can obtain the necessary configuration for receiving the L1/L2 signaling of the side control information. - Option 1: The necessary configuration is from RRC. - Option 2: The necessary configuration is from OAM or hard-coded. - Option 3: The necessary configuration is partially configured by RRC and partially configured by OAM or hard-coded.

In RAN2#119e and RAN3#117e, four candidate solutions on NCR management were agreed and captured in TR38.867. Though RAN2 and RAN3 do not explicitly conclude that RRC layer would be supported for NCR, it was observed in one contribution [Fujitsu] that all candidate solutions in TR 38.867 include RRC connection establishment procedure. Hence, one observation can be done:

Observation 2-1: Based on those four candidate solutions on NCR management given in TR38.867, it can be observed

- An NCR can establish RRC connection with the gNB at least via 4-step random access procedure.

- An NCR contains RRC layer and MAC layer.

Four companies would like to support Option 1 (the necessary configuration is from RRC) [Huawei, CMCC, NTT DOCOMO, Fujitsu]. Some companies directly assume that there is an RRC layer and some configurations for L1/L2 signaling of the side control information can be from the RRC layer [vivo, ZTE, CATT, xiaomi, Intel, Samsung, Qualcomm, Lenovo, Ericsson].

Based on above observation and summary, the FL would like to propose:

Proposal 2-1-1: At least the necessary configuration for receiving the PHY channels and L1/L2 signaling of the side control information from RRC is supported for NCR.

Considering that whether the solution involving OAM ws be supported or not is still unclear, the FL adds an ‘at least’ in the proposal. Before the conclusion on the candidate solution involving OAM is made in RAN2/RAN3, RAN1 can focus on the discussion about the necessary configuration for receiving the PHY channels and L1/L2 signaling of the side control information from RRC.

  - Initial access procedure of NCR-MT

In the SI, the NCR identification was one of scopes of RAN2/RAN3 and was concluded in TR 38.867. It is noticed that the NCR indication information can be sent in Msg5 or later than Msg5 according to those four candidate solutions on NCR management. Hence, from the point of view of RAN1, it can be observed:

Observation 2-2: Based on those four candidate solutions on NCR management given in TR38.867, it can be observed

- An NCR can access to the gNB as a normal UE, via Msg1 to Msg4.

- The gNB can identify an NCR after the initial access procedure successes (via Msg 5 or later).

Based on above observation and summary, legacy initial access procedure is enough for NCR to access to the gNB and establish an RRC connection with the gNB. Hence, the FL recommends companies to consider the following proposal:

Proposal 2-2-1: Legacy 4-step initial access procedure is reused for NCR.

In FL understanding, this proposal means:

- The legacy 4-step initial access procedure is reused. No enhancement for the initial access of NCR is needed.

  - No additional configuration about initial access is required for NCR.

  - No dedicated RACH resource is required for NCR.

- The NCR can access to the gNB like a legacy UE, via

  - Initial cell search and reading system information

  - Sending Msg1

  - Receiving Msg2

  - Sending Msg3, and

  - Receiving Msg4.

- Before the NCR indication defined in those four candidate solutions on NCR management is sent, the gNB may not be able to identify an NCR. During the initial access procedure, the NCR is treated as a normal UE:

  - One C-RNTI can be given to the NCR like what the gNB does to a normal UE.

  - The timing of NCR uplink can be adjusted according to the TA command in RAR (Msg2).

  - The power of NCR uplink can be adjusted according to the TPC command in RAR (Msg2).

  - PHY channels of NCR-MT

In RAN1#109e, the following agreements were achieved:

Agreement For an NCR-MT, the necessary configurations from RRC and/or OAM(or hard-coded) contain: - The configurations of PHY channels to carry the L1/L2 signaling: - The configurations for receiving PDCCH and PDSCH. - The configurations for transmitting PUCCH, if needed. - The configurations for transmitting PUSCH, if needed. - The configurations of L1/L2 signaling: - The configurations for DCI. - The configurations for UCI, if needed. - The configurations for MAC CE, if needed.

In above agreement, there is one point which needs to clarify. The L1/L2 signaling mentioned here is for the side control information rather than that for NCR-MT itself (e.g., the L1/L2 signaling for initial access and UL/DL transmission).

Some contributions considered issues relevant to this agreement. For example, regarding to the configuration on PDSCH, some companies pointed out that semi-static configurations, at least, for access link beam indication and ON/OFF, should be an RRC signaling and/or an MAC CE which are carried by PDSCH [Intel, Fujitsu]. Furthermore, the configuration on corresponding PDCCH for PDSCH scheduling is needed as well. Regarding to the configuration on UCI/PUCCH, some companies proposed that PUCCH is needed, at least, for carrying the ACK/NACK feedback of PDSCH [ZTE, Fujitsu, Intel, CMCC, Apple, Samsung]. It was also mentioned/proposed that the ACK/NACK feedback for PDCCH which dynamically indicated beam and/or ON OFF information may be necessary as well [Fujitsu, Intel, Apple].

Regarding to MAC CE and PUSCH mentioned in the above agreement, it is still unclear whether they will be utilized to carry side control information. It may depend on further discussion on side control information in 9.8.1.

However, regarding to the MAC CE and PUSCH for NCR-MT itself, it is obvious that both of them are necessary based on what we have done in Observation 2-1 and Observation 2-2. At least they are necessary for the legacy initial access procedure, i.e., PUSCH is needed for carrying Msg3 and Msg5 and MAC CE is needed for Msg2 and Msg3.

    - PUCCH (UCI) and PUSCH

Regarding to the UCI, PUCCH and PUSCH for NCR-MT, the FL would like to recommend companies to consider following proposals:

Proposal 2-3-1: At least ACK/NACK feedback of PDSCH is supported

- FFS: the details of ACK/NACK feedback of PDSCH

- FFS: the UCI of ACK/NACK feedback of PDCCH

- FFS: the UCI of SR

- FFS: the UCI of CSI

Proposal 2-3-2:

- PUCCH is supported for NCR.

  - FFS: the details of the supported PUCCH

- PUSCH is supported for NCR

  - FFS: the details of the supported PUSCH

    - PDCCH (DCI)

In some contributions, new DCI format for NCR is proposed [ZTE, Intel, Nokia, Apple, Samsung, Qualcomm, Ericsson]. Moreover, some contribution proposed that the new DCI format can at least be used for dynamic access link beam indication or/and the ON/OFF information. [Intel, Nokia, Qualcomm, Ericsson]

Some companies provide details of the DCI for access link beam indication or/and ON/OFF information. [ZTE, CATT, Intel, ETRI, Ericsson]

Except for the dynamic indication for beams in access link and/or ON/OFF, some companies also provide other potential indication methods which requires DCI. For example, RRC signaling is used to configure a list of beam patterns/ON OFF patterns, DCI is used to indicate one beam pattern from the configured pattern list [vivo]. The activation or deactivation signaling to beam pattern and/or ON-OFF pattern can be defined via DCI [vivo, Intel].

  - TA adjustment, power control and power saving for NCR-MT (C link)

    - TA adjustment for C link

In RAN1 #109e and RAN1 #110, the following conclusion and agreements about timing were achieved.

Conclusion: Legacy UE mechanism is sufficient to achieve DL/UL timing for NCR-MT. Agreement: For the signaling of the side control information of timing to align transmission / reception boundaries, new signaling is unnecessary.

To confirm the conclusion, the FL would like to propose:

Proposal 2-4-1: Legacy TA adjustment mechanism is reused for NCR-MT.

    - Power control for C uplink

Though the discussion on power control for NCR-Fwd was struggling, the power control of C uplink seems reasonable to be supported to make sure that the interference caused by uplink of NCR-MT is comparable to a normal UE. One company proposes that the repeater-MT inherits legacy UE power control [Ericsson]. The FL recommends companies to consider the following proposal:

Proposal 2-4-2: Legacy power control mechanism is reused for NCR-MT.

    - Power saving of C link

According to the discussion in the SI phase, one of the purposes to support NCR-Fwd ON/OFF is to save power of the NCR. For example, when no UE requires the service of the NCR, the gNB can indicate the NCR (NCR-Fwd) to ‘sleep’ to save the power consumption. In SI, some companies proposed that the NCR-MT can support DRX and UE power saving mode.

Although no explicit proposals were raised by companies in this meeting, the FL thinks that whether the legacy UE power saving mechanism is supported for NCR-MT or not is worth of considering. The FL would like to recommend companies to share your views on the support of legacy UE power saving mechanism for NCR-MT.

  - Beam-relevant functionalities for C link

In RAN1#109e and RAN1 #110, the following agreements were made:

Agreement (RAN1#109-e) As baseline, the same TCI states as C-link are assumed for beam at NCR-Fwd for backhaul link if the NCR-MT’s carrier(s) is within the set of carriers forwarded by the NCR-Fwd. - FFS: additional indication from gNB to determine the beam at NCR-Fwd for backhaul link or implicit determination of the beam at NCR-Fwd for backhaul link Note: the same assumption of the beam correspondence is applied for DL/UL of the backhaul link at NCR-Fwd as the DL/UL of the C-link at NCR-MT. Agreement (RAN1#110) In case that adaptive beams are adopted for C-link and backhaul link, the following mechanisms can be considered for the indication and determination of beams of backhaul link: - Option 1: The beam of backhaul link is indicated by a new signaling. - The new signaling is dynamic signaling and/or semi-static signaling (e.g., RRC signaling/ MAC CE) indicating a beam(s) from the set of beams of the C-link - This does not imply that the beam of backhaul link is always indicated by the new signaling - Option 2: The beam of backhaul link is determined by a pre-defined rule. - In slots/symbols with simultaneous DL receptions / UL transmissions in both C-link and backhaul link, the beam of backhaul link is the same as the beam of C-link. Otherwise, the beam of backhaul link follows one of the beams of the C link. - Other predefined rules are not precluded.

In the discussion on BH link beams, it is noticed that the beams indicated for C link are often treated as a reference for the beams of backhaul link [vivo, ZTE, CATT, xiaomi, Apple]. Moreover, some companies also discussed the CSI feedback in contributions [Samsung, Apple]. The conclusion on C link beams seems highly correlated with the discussion on the determination of BH link beams. However, the details of beam-relevant functionalities/mechanism of C link have not been discussed yet. The FL would like to suggest companies to consider and discuss the details of beam-relevant functionalities/mechanism of C link.

    - Beam indication mechanism

The FL recommends companies to consider following proposal on beam indication framework of C link.

Proposal 2-5-1: For NCR-MT which can support adaptive beams in C link,

- Rel-15 beam indication framework is reused.

- Rel-17 beam indication framework (i.e., the unified TCI) is reused as well, if the NCR supports.

    - CSI measurement and report

When NCR-MT utilizes adaptive beams in C link (and BH link), to support the beam indication, CSI measurement and report could be necessary. Hence, the FL recommends companies to consider following proposal.

Proposal 2-5-2: To support the legacy beam indication mechanisms for NCR-MT,

- The necessary legacy mechanism for receiving CSI-RS can be reused for NCR-MT.

- The necessary legacy mechanism for reporting CSI can be reused for NCR-MT.

- Note: this does not mean the legacy procedures for receiving CSI-RS and reporting CSI will be all supported. The details of the necessary mechanisms will be further discussed and decided.

    - SRS

Also, to support the beam indication for NCR-MT with adaptive beams in C link, the legacy sounding procedure may be necessary as well. The FL recommends companies to share your views on SRS and consider the following proposals:

Proposal 2-5-3: To support the legacy beam indication mechanisms for NCR-MT,

- The necessary mechanism of legacy UE sounding procedure can be reused for NCR-MT.

- Note: this does not mean the whole legacy UE sounding procedure will be supported. The details of the necessary mechanisms will be further discussed and decided.

    - Beam management

Besides, compared with fixed beams, the adaptive beams can provide better flexibility and robustness. Though the location of NCR is fixed, the signal propagation paths between the gNB and the NCR can be fluctuating and varying due to the movement of the objects in the environment. Some companies discussed and proposed to support the BFR mechanism for C-link/NCR-MT [Fujitsu, InterDigital, Samsung]. The FL recommends companies to share your views on BFR and consider the following proposals:

Proposal 2-5-4: For NCR-MT which can support adaptive beams in C link, Rel-15 BFR mechanism is reused.

  - Features and capabilities

Some companies discussed the “UE features” and capabilities/capability report in contributions. The relevant proposals are summarized here as a reference.

Regarding the features, one company pointed out that it is not essential to support of all mandatory features for an NCR-MT and further discussion on the support details of mandatory features can be discussed later [ZTE, Intel]. Another company proposed that L1 mandatory features for IAB-MT in Table 4.2.15.1-1 of TS38.306 can be considered as a baseline when the features of NCR-MT is discussed [LGE]. Also, this company mentioned that L1 features of NCR-MT can consider Rel-15/16/17. For example, it can be discussed whether TCI-related features including unified TCI introduced in Rel-17 FeMIMO need to be supported for beam indication of NCR-MT.

Summary on capability reporting proposed by companies is as follows [InterDigital, ZTE, Intel, NEC, Qualcomm, Lenovo, CAICT, xiaomi, Ericsson]:

- Access link

  - Information on beams

    - Number of supported beams [InterDigital, Intel, NEC, Qualcomm, CAICT]

    - Beam width/type [InterDigital, Intel, NEC, CAICT]

    - Spatial relation between beams [Intel]

    - Number of beams that can operate simultaneously [InterDigital]

    - Relation between logic indexes and beams [NEC]

  - Information on Antennas

    - Number of Antennas [NEC]

    - Beam array size and beam expansion (to construct wide beams from narrow beams) [Ericsson]

  - A time offset for beam indication application [ZTE]

  - Power information [Qualcomm]

- BH link

  - Capability of simultaneous UL transmission in C link and BH link [Intel, Qualcomm]

- Pass band information [Intel, Qualcomm, Ericsson]

- C link

  - Capability of dynamic TDD [Ericsson]

- Discussion round 1

  - The configuration for NCR-MT

Observation 2-1: Based on those four candidate solutions on NCR management given in TR38.867, it can be observed

- An NCR can establish RRC connection with the gNB at least via 4-step random access procedure.

- An NCR contains RRC layer and MAC layer.

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Ericsson | Support. |
| CMCC | Support |
| Intel | We agree with the observation. |
| NTT DOCOMO | We support. |
| LG | Support |
| Huawei, HiSilicon | Support |
| Vivo | We think the 2nd bullet can be captured as agreement. |
| ZTE | We agree with what is observed by FL. Considering that this is WI phase, and this observation is to make companies to have same understanding of TR which has no spec impact, it might be better to change it from observation to conclusion. |
| Fujitsu | Support |
| Apple | Suppoort |
| NEC | Support. |
| CEWiT | Support |
| Lenovo | Support |
| Xiaomi | Support |
| Sharp | Support |
| IIT-K | Support |
| Samsung | Support. Editorial suggestion to replace “NCR” with “NCR-MT”. |
| Qualcomm | Support. |
| Philips | Support |
| CATT | Ok to make this observation but this is not needed |
| InterDigital | Fine |
| KDDI | Support. |
| Sony | Support. |
| Panasonic | Support |

Proposal 2-1-1: At least the necessary configuration for receiving the PHY channels and L1/L2 signaling of the side control information from RRC is supported for NCR.

Considering that whether the solution involving OAM ws be supported or not is still unclear, the FL adds an ‘at least’ in the proposal. Before the conclusion on the candidate solution involving OAM is made in RAN2/RAN3, RAN1 can focus on the discussion about the necessary configuration for receiving the PHY channels and L1/L2 signaling of the side control information from RRC.

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Ericsson | Support in principle, however, if this is RRC or other signaling is not up to RAN1. |
| CMCC | Support |
| Intel | We support the proposal. |
| NTT DOCOMO | We support. |
| Nokia | We have a similar concern to Ericsson. In our view, this proposal is outside of the scope of RAN1. Any parameters identified to be configured by higher layers can be provided to RAN2/3 and details related to their implementation can be discussed within the scope of their work. |
| LG | Support the proposal, but it seems not RAN1 scope. |
| Huawei, HiSilicon | Support though this can be up to RAN2 to decide. |
| vivo | Support the proposal. By the way, L2 signaling of the SCI is implicitly agreed by this proposal, is it correct understanding? |
| ZTE | We understand the intention of the proposal. But the down-selection is also under the discussion of RAN2, it might be better to wait for RAN2’s decision. Meanwhile, RAN1 could focus on the PHY channel discussion assuming necessary configuration is from RRC. |
| Fujitsu | Support |
| Apple | Similar view as other companies that whether the configuration is via RRC or some other signaling is not up to RAN1 |
| NEC | Support in general. But whether the necessary configuration is from RRC or OAM would be discussed by RAN2. |
| CEWiT | Support |
| Lenovo | We support, also we think this is up to RAN2 to decide |
| Xiaomi | Support |
| Sharp | Support |
| IIT-K | Support |
| Samsung | Fine with the proposal, and agree with Ericsson that higher layer signalling can be left to RAN2. Editorial suggestion to replace “NCR” with “NCR-MT”. |
| Qualcomm | Support. |
| Philips | Support but agree with others that this is potentially out-of-scope for RAN1 |
| CATT | In order to make this agreement , RRC should be agreed , which seems not in ran1 scope. |
| InterDigital | We are fine with the proposal. |
| KDDI | Support. |
| Sony | Support. |
| Panasonic | Support |

  - Initial access procedure

Observation 2-2: Based on those four candidate solutions on NCR management given in TR38.867, it can be observed

- An NCR can access to the gNB as a normal UE, via Msg1 to Msg4.

- The gNB can identify an NCR after the initial access procedure successes (via Msg 5 or later).

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Ericsson | Support. |
| CMCC | Support in principle. For the first bullet, although we share exactly the same view, but the details such as 4 step RAHC or 2 step RACH depends on the UE capability of NCR-MT. The first part that NCR-MT can access to the gNB as a normal UE is more important. More details about how NCR-MT can access to the gNB could base on current spec and do not have spec impact for NCR. |
| Intel | We agree with the observation, though in which step to identify NCR (via Msg 5 or later) is RAN2’s work (RAN2 already agreed early identification via Msg1 or Msg3 for NCR is not needed in SI phase). |
| NTT DOCOMO | We support. |
| LG | Support. |
| Huawei, HiSilicon | Support |
| ZTE | Fine with the observation but suggest to change it to conclusion. |
| Fujitsu | Support |
| Apple | Support |
| NEC | Support. |
| CEWiT | Support |
| Lenovo | Fine with the proposal |
| Xiaomi | Support |
| Sharp | Support |
| IIT-K | Support |
| Samsung | Generally fine with the observation. Suggest to replace “NCR” with “NCR-MT, and replace “after the initial access procedure successes (via Msg 5 or later)” with “after successful completion of the random access procedure”. |
| Qualcomm | Support. |
| Philips | Support but agree with Intel that the decision for Msg 5 or later is for RAN2. |
| InterDigital | We are fine with the observation. |
| KDDI | Support. |
| Sony | Support with Samsung’s modification. |
| Panasonic | Support |

In FL understanding, this proposal means:

- The legacy 4-step initial access procedure is reused. No enhancement for the initial access of NCR is needed.

  - No additional configuration about initial access is required for NCR.

  - No dedicated RACH resource is required for NCR.

- The NCR can access to the gNB like a legacy UE, via

  - Initial cell search and reading system information

  - Sending Msg1

  - Receiving Msg2

  - Sending Msg3, and

  - Receiving Msg4.

- Before the NCR indication defined in those four candidate solutions on NCR management is sent, the gNB may not be able to identify an NCR. During the initial access procedure, the NCR is treated as a normal UE:

  - One C-RNTI can be given to the NCR like what the gNB does to a normal UE.

  - The timing of NCR uplink can be adjusted according to the TA command in RAR (Msg2).

  - The power of NCR uplink can be adjusted according to the TPC command in RAR (Msg2).

Proposal 2-2-1: Legacy 4-step initial access procedure is reused for NCR.

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Ericsson | Support. |
| CMCC | Fine with the proposal. |
| Intel | We support the proposal. |
| NTT DOCOMO | We support. |
| Nokia | Support the proposal. |
| LG | Support. |
| Huawei, HiSilicon | Support |
| Vivo | Yes We find many proposals discussing the MT functionality, are we going to determine the MT capability in this meeting? |
| ZTE | We are fine with the proposal. |
| Fujitsu | Support |
| Apple | Support |
| NEC | Support. |
| CEWiT | Support |
| Lenovo | Support |
| Xiaomi | Support |
| Sharp | Support |
| IIT-K | Support |
| Samsung | Support the proposal. Suggest to replace “initial access” with “random access”, and replace “NCR” with “NCR-MT”. |
| Qualcomm | Support. |
| Philips | Support |
| CATT | OK |
| InterDigital | We are fine with the proposal. |
| KDDI | Support. |
| Sony | Support. |
| Panasonic | Support |

  - PHY channels of NCR-MT

    - PUCCH (UCI) and PUSCH

Proposal 2-3-1: At least ACK/NACK feedback of PDSCH is supported

- FFS: the details of ACK/NACK feedback of PDSCH

- FFS: the UCI of ACK/NACK feedback of PDCCH

- FFS: the UCI of SR

- FFS: the UCI of CSI

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Ericsson | Partly support. The repeater should comply with standard UE behavior for standard UE signaling. For repeater-specific signaling, we support ACK/NACK for higher layer signaling (semi-static configuration) as already supported by legacy signaling but not for lower layer signaling (dynamic indication) since the low latency requirements of dynamic indications will render ACK/NACK (HARQ) useless. |
| CMCC | Support. We have the same understanding that for most signalling which are already defined for the normal UEs should be reused. For the 1st , 3rd and 4th sub-bullet, the FFS could be removed. The 2nd bullet could be discussed. |
| Intel | We are fine with the proposal. For 2nd FFS, we think it is also important to support HARQ-ACK feedback for PDCCH carrying side control information, otherwise, there is ambiguity of reception of dynamic side control information. We don’t think the latency for HARQ-ACK would be much larger than application latency itself without HARQ-ACK, and HARQ-ACK latency is much smaller than the number of slots which the side control information applies to (for dynamic indication, it also can indicate side control information for several slots). Furthermore, for semi-persistent side control information which can be activated by PDCCH, HARQ-ACK feedback is very helpful to avoid miss-alignment between gNB and NCR for a long time. |
| NTT DOCOMO | We support. |
| Nokia | Support the proposal. |
| LG | We are generally fine with the proposal, but don’t support the second FFS. For the second FFS, many DCIs carrying control information of UE/MT (e.g., DCI format 2_0/2_5) also have ambiguity issue, but do not require A/N feedback for the DCI. Also, similar to Ericsson, side control information by DCI needs to be applied immediately without A/N feedback. |
| Huawei, HiSilicon | Support except the second sub-bullet of FFS on PDCCH ACK/NACK feedback is not required. |
| Vivo | We are OK to discuss 2nd bullet, i.e., whether to support ACK/NACK feedback for PDCCH carrying SCI. |
| ZTE | We are generally fine with the proposal. But this proposal seems to focus on support of ACK/NACK feedback, which is in our view very necessary procedure of NCR. But since the exact signal or channel carrying side control information is still under discussion, we may decouple the ACK/NACK feedback procedure from the exact channel carrying side control information, which can be FFS part in sub-bullet. As for the last 2 sub-bullet on SR and CSI, we are fine to keep them. So we suggest the following: Proposal 2-3-1: At least ACK/NACK feedback of PDSCH in response to side control information is supported - FFS: the details of ACK/NACK feedback of PDSCH - FFS: the UCI of ACK/NACK feedback of PDCCH - FFS: the UCI of SR - FFS: the UCI of CSI |
| Fujitsu | Support. For HARQ-ACK feedback for PDCCH, we share the same view with Intel. |
| Apple | We support the proposal and would prefer to also agree on HARQ-ACK feedback for PDCCH |
| CEWiT | Support the proposal In our understanding HARQ feedback for PDCCH is essential in case of NCR to avoid performance degradation due to noise amplification and packet loss. Monitoring NCR operation based on UE feedback can lead to additional delay. |
| Lenovo | We are fine with the proposal. We are also fine with ACK/NACK for PDCCH (2nd bullet) if it doesn’t cause delay for applying the side control information. |
| Xiaomi | We support this proposal and we think HARQ-ACK feedback for PDCCH is necessary. |
| Sharp | Support |
| IIT-K | Support this proposal and think that HARQ-ACK/NACK feedback for PDCCH is necessary. |
| Samsung | Fine with the proposal, and prefer to remove the FFS for the last bullet (i.e., agree to support CSI for NCR-MT). We are OK to keep the FFS points for HARQ-ACK of PDCCH and for SR, which can be further discussed in the next meeting. Also, the proposal needs to mention NCR-MT. |
| Qualcomm | NCR-MT should follow legacy procedures. We are not sure what is there “for further study” regarding the 1st, 3rd and 4th bullets. We do not need to agree on every single legacy procedure to be supported by the NCR-MT (it should be assumed as the default), rather should only spend time on the new things that may need to be added (like 2nd bullet). |
| Philips | Support |
| CATT | No need to agree legacy procedures, as MT follows legacy UE behaviour. |
| InterDigital | We are fine with the proposal. |
| ETRI | We share similar view with QC. |
| KDDI | We share similar view with QC. There is no need to study further for legacy procedures. |
| Sony | In principle, support the proposal. We do not think there is much to discuss about FFS bullet 1, 3 and 4, as legacy can be adopted directly. For the second FFS bullet, we are not sure either it is needed. |
| Panasonic | Support |

Proposal 2-3-2:

- PUCCH is supported for NCR.

  - FFS: the details of the supported PUCCH

- PUSCH is supported for NCR

  - FFS: the details of the supported PUSCH

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Ericsson | Support. Both are required, e.g., for RA. |
| CMCC | Support. |
| Intel | The proposal is not very clear to us. If this proposal is only for normal UE behaviour for NCR-MT part (not for side control information specific design), proposal 2-2-1 already means PUCCH and PUSCH is supported, and it is unclear what kind of details to be discussed. But if it is for side control information specific design, we’d like to understand the motivation for PUSCH. We can understand that if NCR capability is supported for beam, we need PUSCH, but unfortunately, we have not achieved any consensus yet. In other words, necessity for PUSCH for side control information depends on progress on other aspects which are still pending. |
| NTT DOCOMO | We support, thought it’s up to the outcome of the discussion for side control signalling in 9.8.1. |
| Nokia | Support the proposal. Our understanding is that this proposal does not imply the need to enhance PUCCH and PUSCH for NCR side control information. |
| LG | Support. |
| Huawei, HiSilicon | Support |
| vivo | Yes |
| ZTE | We are generally fine with the proposal. But we don’t quite understand what is the details of PUCCH and PUSCH, if this refers to the configuration of PUCCH and PUSCH, we can directly confirm that legacy configuration can be reused. So we suggest the following: Proposal 2-3-2: - PUCCH and PUSCH with legacy configuration are supported for NCR-MT. |
| Fujitsu | Support |
| Apple | We are fine to support PUSCH considering RA procedure |
| NEC | Support. |
| CEWiT | Support |
| Lenovo | Support with the understanding that the legacy PUCCH/PUSCH configuration can be reused for NCR-MT. |
| Xiaomi | Support |
| Sharp | Supprot |
| IIT-K | Support |
| Samsung | Support. Suggest to clarify “NCR” means “NCR-MT”. |
| Qualcomm | Similar to our comment to Proposal 2-3-1: we do not prefer to spend time to “further study” baseline and straightforward aspects. Hence, we should remove “FFS” points, unless there are specific proposals to change some of the legacy designs. |
| Philips | Support. |
| CATT | No need to agree legacy procedures, as MT follows legacy UE behaviour. |
| ETRI | We think this one is fine as a conclusion. Same comment with Samsung that “NCR” means “NCR-MT”. |
| KDDI | We share similar view with QC. |
| Sony | Support. |
| Panasonic | Support |

    - PDCCH (DCI)

In some contributions, new DCI format for NCR is proposed [ZTE, Intel, Nokia, Apple, Samsung, Qualcomm, Ericsson]. Moreover, some contribution proposed that the new DCI format can at least be used for dynamic access link beam indication or/and the ON/OFF information. [Intel, Nokia, Qualcomm, Ericsson]

Some companies provide details of the DCI for access link beam indication or/and ON/OFF information. [ZTE, CATT, Intel, ETRI, Ericsson]

Except for the dynamic indication for beams in access link and/or ON/OFF, some companies also provide other potential indication methods which requires DCI. For example, RRC signaling is used to configure a list of beam patterns/ON OFF patterns, DCI is used to indicate one beam pattern from the configured pattern list [vivo]. The activation or deactivation signaling to beam pattern and/or ON-OFF pattern can be defined via DCI [vivo, Intel].

| Please share your views on DCI of NCR if any |  |
| --- | --- |
| Company | Comments |
| Ericsson | New beamforming DCI format is specified, following the DCI format 2_0/2_5 framework, providing an index to beam-to-time mappings provided by higher layers. There is no need for a separate ON/OFF DCI. |
| CMCC | New DCIs supporting side control information should be supported. |
| Intel | We suggest to first discuss new DCI format. For details of the DCI for beam and on/off, it can wait for some progress in 9.8.1. |
| NTT DOCOMO | New DCI may be necessary, but it’s up to the outcome of the discussion for side control signalling in 9.8.1. |
| Nokia | At least DCI format 0_0, 0_1, 1_0, and 1_1 should be required for dynamic PUSCH and PDSCH. Support for additional DCI format i.e., 2_X will likely depend on agreement related to NCR capability, e.g., slot format indication, or on further progress being made with regard to side control information. |
| LG | For dynamic indication of side control information including access link beam and ON-OFF information, a new DCI needs to be introduced. The new DCI may be NCR-MT specific DCI or NCR-MT group specific DCI. When an NCR-MT specific DCI is used, side control information delivered to an NCR-MT is contained to a DCI. From the DCI, side control information applied to the NCR-Fwd can be indicated. Alternatively, when NCR-MT group specific DCI is used, side control information for a group of NCR-MTs can be contained within a DCI. Within a DCI, side control information dedicated to different NCR-MTs may be transmitted through the same or different DCI fields. In this case, the NCR-MT can determine the position of the DCI field it receives within the DCI, and can obtain access link beam information applied to the NCR-Fwd through the DCI field. |
| Huawei, HiSilicon | We are open to discuss new DCI for side control information. For dynamic signalling based on DCI, a single DCI should be able to indicate multiple beams over multiple time domain resources such as to reduce the signalling overhead. |
| Vivo | We think a new DCI format should be defined to carry side control information. |
| ZTE | We support to introduce a new DCI format for NCR, but the exact content can be discussed in 9.8.1. |
| Fujitsu | We are open to discussing new DCI format. However, we think RAN1 should first clarify which legacy DCI formats will be supported by NCR and if any legacy DCI format can used for indicating some of or all information for controlling of NCR-Fwd. |
| Apple | We think that a new DCI format should be specified for NCR. Requirements for NCR-Fwd are different than typical UEs and the existing DCI format(s) may not be optimal for NCR. |
| CEWiT | Support new DCI format for indicating side control information to NCR. |
| Lenovo | @Moderator: Lenovo also proposed to support new NCR specific DCI format for side control information New DCI format is necessary to carry the side control information that excludes scheduling related information. Details on the exact design of the DCI can be discussed after agreeing on the details of the side control information. |
| Xiaomi | We support to discuss new DCI format for NCR. |
| Sharp | New DCI formats for NCR should be supported. |
| IIT-K | Fine with the proposal |
| Samsung | Support new DCI format for NCR-MT at least for dynamic beam indication and ON/OFF indication (that can share a unified signalling framework). |
| Qualcomm | We support defining a new DCI format, and discuss its details. |
| Philips | We are open to discuss new DCI format for NCR. |
| CATT | support defining a new DCI format, and discuss its details. |
| InterDigital | We are open to discuss the requirement and possible new DCI formats. However, we agree with the view by Fujitsu, i.e., RAN1 should first agree on which legacy DCI formats are supported by NCR. |
| ETRI | Support new DCI format for NCR. |
| KDDI | We support to define new DCI formats. The details are up to outcome of discussion in 9.8.1. |
| Panasonic | We support the new DCI formats. |

  - TA adjustment, power control and power saving for NCR-MT (C link)

    - TA adjustment for C link

To confirm the conclusion made in RAN1#109e, the FL would like to propose:

Proposal 2-4-1: Legacy TA adjustment mechanism is reused for NCR-MT.

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Ericsson | Support. |
| CMCC | Support |
| Intel | We support the proposal |
| NTT DOCOMO | We support. |
| Nokia | Support the proposal. |
| LG | Support. |
| Huawei, HiSilicon | Support |
| Vivo | Yes. |
| ZTE | We are fine with the proposal. |
| Fujitsu | Support |
| Apple | Support |
| NEC | Support. |
| CEWiT | Support |
| Lenovo | Support |
| Sharp | Support |
| IIT-K | Support |
| Samsung | Fine with the proposal. Suggest to add: “FFS: whether to include additional timing offset or N_Delta value for NCR-MT”. The gNB may need to provide a fixed slot/symbol timing adjustment to align the NCR-MT with the network-wide gNB TDD timing (similar to IAB). This is still based on legacy timing procedure, with the N_delta parameter helping if the internal delay of NCR is non-negligible. |
| Qualcomm | Support. |
| Philips | Support |
| CATT | Add ‘at least’ |
| InterDigital | We are fine with this proposal. |
| KDDI | Support. |
| Sony | Support. |

    - Power control for C uplink

Proposal 2-4-2: Legacy power control mechanism is reused for NCR-MT.

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Ericsson | Support. |
| CMCC | Support |
| Intel | We support the proposal |
| NTT DOCOMO | We support. |
| Nokia | Support the proposal. |
| LG | Support. |
| Huawei, HiSilicon | Support |
| Vivo | Yes |
| ZTE | We are fine with the proposal. |
| Fujitsu | Support |
| Apple | Support |
| NEC | Support. |
| CEWiT | Support |
| Lenovo | Support |
| Sharp | Support |
| IIT-K | Support |
| Samsung | Support |
| Qualcomm | Support. |
| Philips | Support |
| CATT | support |
| InterDigital | We are fine with this proposal. |
| KDDI | Support. |
| Sony | Support. |

  - Beam-relevant functionalities for C link

    - Beam indication mechanism

Proposal 2-5-1: For NCR-MT which can support adaptive beams in C link,

- Rel-15 beam indication framework is reused.

- Rel-17 beam indication framework (i.e., the unified TCI) is reused as well, if the NCR supports.

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Ericsson | Support. |
| CMCC | Fine with the proposal. Rel-17 based beam indication could be introduced depends on NCR capabilities. |
| Intel | We’re generally fine with the proposal. But we’d like to clarify, does this proposal mean whether NCR-MT can support adaptive beam depends on NCR capability? |
| NTT DOCOMO | We support that Rel-17 unified TCI framework is also supported by NCR-MT, since the indicated unified TCI for C-link can be used for backhaul link. |
| Nokia | Support the proposal. |
| LG | Support. |
| Huawei, HiSilicon | Support |
| Vivo | Does it mean Rel-17 unified TCI is optional and up to MT capability. Rel-17 beam indication framework (i.e., the unified TCI) is reused subject to MT capability |
| ZTE | We don’t agree with the second sub-bullet and we think Rel-15 beam indication framework is enough. First of all, NCR MT is a simplified unit compared to legacy UE which can be flexibly deployed to improve the coverage, so the complexity of NCR MT and the upgrading requirements to existing network should be minimized. If Rel-17 beam indication framework is adopted for NCR MT, it not only increases the complexity of NCR MT, but also requires to upgrade the existing network to also support Rel-17 beam framework. In addition, from our understanding, Rel-17 beam indication framework is introduced to unify the configuration of UL transmission and DL reception beam, although it also defines default TCI state, it’s not desirable for NCR MT to reuse Rel-17 framework just to utilize the default state, instead we can simply define a rule to follow last transmission which can achieve same target if needed. Therefore, we suggest to only reuse Rel-15 beam indication framework. |
| Fujitsu | Support. |
| Apple | Support |
| NEC | Support. |
| CEWiT | Support |
| Lenovo | Fine with the proposal |
| Xiaomi | Support |
| Sharp | Support |
| IIT-K | Support |
| Samsung | Fine with the proposal. Suggest to replace “is reused as well” with “can be reused as well” in the second bullet (i.e., decision up to gNB implementation/configuration). |
| Qualcomm | Support. |
| Philips | Support |
| CATT | support |
| InterDigital | We are fine with this proposal. |
| KDDI | Support. |
| Sony | Support. |
| Panasonic | Support. |

    - CSI measurement and report

Proposal 2-5-2: To support the legacy beam indication mechanisms for NCR-MT,

- The necessary legacy mechanism for receiving CSI-RS can be reused for NCR-MT.

- The necessary legacy mechanism for reporting CSI can be reused for NCR-MT.

- Note: this does not mean the legacy procedures for receiving CSI-RS and reporting CSI will be all supported. The details of the necessary mechanisms will be further discussed and decided.

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Ericsson | Support. |
| CMCC | Support |
| Intel | We support the proposal |
| NTT DOCOMO | We support. |
| Nokia | Support the proposal. |
| LG | Support. |
| Huawei, HiSilicon | Support |
| vivo | Yes |
| ZTE | Fine with the proposal. |
| Fujitsu | Support. |
| Apple | Support |
| NEC | Support. |
| CEWiT | Support |
| Lenovo | Support |
| Sharp | Support |
| IIT-K | Support |
| Samsung | Support |
| Qualcomm | Support. |
| Philips | Support |
| InterDigital | We are fine with this proposal. |
| KDDI | Support. |
| Sony | Support. |
| Panasonic | Support. |

    - SRS

Proposal 2-5-3: To support the legacy beam indication mechanisms for NCR-MT,

- The necessary mechanism of legacy UE sounding procedure can be reused for NCR-MT.

- Note: this does not mean the whole legacy UE sounding procedure will be supported. The details of the necessary mechanisms will be further discussed and decided.

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Ericsson | Support. |
| CMCC | Support |
| Intel | We support the proposal |
| NTT DOCOMO | We support. |
| Nokia | If WA regarding DL/UL beam correspondence for backhaul link is confirmed then UL beam sweeping may not be necessary. |
| LG | Support. |
| Huawei, HiSilicon | Support |
| Vivo | Yes |
| ZTE | Fine with the proposal. |
| Fujitsu | Support |
| Apple | Similar view as Nokia. This proposal can be discussed later once the WA on beam correspondence is agreed |
| NEC | Support. |
| CEWiT | Support |
| Lenovo | Support |
| Sharp | Support |
| IIT-K | Support |
| Samsung | Support the proposal. Suggest to clarify the first bullet by adding: “legacy UE sounding procedure including transmitting UL SRS can be…” |
| Qualcomm | Support. |
| Philips | Support |
| CATT | Support in principle |
| KDDI | Support. |
| Sony | Support. |
| Panasonic | Support |

    - Beam management

Proposal 2-5-4: For NCR-MT which can support adaptive beams in C link, Rel-15 BFR mechanism is reused.

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Ericsson | Support with the following modifications to align to the above proposals: For NCR-MT which can support adaptive beams in C link, Rel-15 BFR mechanism iscan be reused. |
| CMCC | Fine with E///’s updates. |
| Intel | Though we think BF is not likely to happen, we’re open for the discussion. |
| NTT DOCOMO | We are fine with Ericsson’s update. |
| Nokia | Fine with Ericsson’s proposed modification. |
| LG | Support. |
| Huawei, HiSilicon | Support |
| Vivo | Yes |
| ZTE | We think Rel-15 BFR mechanism is not needed for NCR-MT. Considering that NCR is a stationary network node, and the channel quality between gNB and NCR is expected to be relatively stable, if beam failure happens, it’s more likely the whole link fails due to the moving blockage, the recovery of beam only does not provide much benefit. Besides, even for legacy UE, BFR procedure is optional capability, we don’t think NCR-MT should also support BFR mechanism. |
| Fujitsu | Support. And we are fine with the updates from Ericsson. |
| Apple | We also think the BFR is not really essential for NCR-MT due to the relative static link/beam between gNB and NCR. |
| NEC | BFR mechanism may be simplified by considering the stability of NCR. |
| Lenovo | Support |
| Xiaomi | Support |
| Sharp | Support |
| IIT-K | Support |
| Samsung | Fine with the proposal and the update from Ericsson. |
| Qualcomm | Support. |
| CATT | Support. |
| InterDigital | We support the proposal with Ericsson’s update. |
| KDDI | Support. |
| Sony | Support. |
| Panasonic | Support. |

  - Summary on round 1

- Observation 2-1:

The observation 2-1 is updated as follows:

Observation Conclusion 2-1: Based on those According to the four candidate solutions on NCR management given in TR38.867, it can be observed

- An NCR-MT can establish RRC connection with the gNB at least via 4-step random access procedure.

- An NCR-MT contains RRC layer and MAC layer.

- Proposal 2-1-1:

22 companies provided their view on this proposal. Although almost all companies support the intention of this proposal, 12 companies pointed out that this proposal needs to be discussed by RAN2.

The discussion on this proposal is closed.

- Observation 2-2:

The observation 2-2 is updated as follows:

Observation Conclusion 2-2: Based on those According to the candidate solutions on NCR management given in TR38.867, it can be observed

- An NCR-MT can access to the gNB as a normal UE, via Msg1 to Msg4.

- The gNB can identify an NCR-MT after the successful completion of the initial access procedure successes (via Msg 5 or later).

Considering that 4-step random access is already captured in the conclusion 2-1, the FL removes ‘via Msg1 to Msg4’ to avoid redundancy.

- Proposal 2-2-1:

The proposal 2-2-1 is updated as follows

Proposal 2-2-1:

- Legacy 4-step initial access procedure is reused for NCR-MT.

- Legacy 4-step random access is reused for NCR-MT.

The comment from Samsung makes the FL realize that the concepts of the initial access and the random access are not the exact same. To make the proposal precise, the original proposal is split into two bullets. The intention of this proposal does not change.

- Proposal 2-3-1:

Original Proposal 2-3-1: At least ACK/NACK feedback of PDSCH is supported

- FFS: the details of ACK/NACK feedback of PDSCH

- FFS: the UCI of ACK/NACK feedback of PDCCH

- FFS: the UCI of SR

- FFS: the UCI of CSI

25 companies provided their view on this proposal.

- 25 companies support the main bullet.

- 5 companies do not support the 1st, 3rd and 4th FFS, because they think the NCR-MT should support legacy procedures [CMCC QC CATT ETRI KDDI].

- Regarding to the 2nd FFS:

  - 15 companies support to include the 2nd FFS in the proposal for further study. [CMCC, Intel, NTT DOCOMO, Nokia, Vivo, ZTE, Fujitsu, Apple, CEWiT, Lenovo, Xiaomi, Sharp, IIT-K, Philips, InterDigital]

    - Among them, 6 companies support the functionality described by the 2nd FFS bullet. [Intel Fujitsu, Apple, CEWiT, Xiaomi, IIT-K]

  - 7 companies do not support the 2nd FFS and would like to remove it from the proposal. [Ericsson, LG, Huawei, Hisilicon, Qualcomm, ETRI and KDDI]

- Regarding to the 4th FFS bullet, Samsung does not support it.

- Besides, ZTE comments that the ACK/NACK should not be discussed per PDSCH/PDCCH and recommend to directly discuss the ACK/NACK feedback of side control information.

The proposal 2-3-1 is updated as follows:

Proposal 2-3-1: At least ACK/NACK feedback of PDSCH is supported. The necessary legacy mechanism is reused.

- FFS: The details of the necessary legacy mechanism of ACK/NACK feedback of PDSCH.

- FFS: The UCI of ACK/NACK feedback of PDCCH

- FFS: Whether the UCI of SR is supported. Some legacy mechanism is reused if the UCI of SR is supported.

- FFS: Whether the UCI of CSI is supported. Some legacy mechanism is reused if the UCI of CSI is supported.

- Note: The PDSCH carries side control information and/or other information for NCR-MT.

The FL has the following considerations:

- The intention of this proposal is not to introduce new feature but to discuss which functionalities of legacy UEs will be supported. To clarify this intention, the main bullet and the 1st, 3rd, 4th sub-bullets of the proposal ware updated. Regarding to the 2nd sub-bullet, considering many companies propose to introduce new DCI for side control information indication, we cannot conclude now that the legacy mechanism is enough. Hence, the 2nd sub-bullet is left to be open.

- Besides, in FL’s understanding, the PDSCH can carry side control information and/or other information. To clarify this, one note is added.

- Proposal 2-3-2:

Similar to what we did to proposal 2-3-1, to clarify the intention is not to introduce new feature, proposal 2-3-2 is updated as follows:

Proposal 2-3-2:

- PUCCH is supported for NCR. The necessary legacy mechanism is reused.

  - FFS: The details of the necessary legacy mechanism of the supported PUCCH

- PUSCH is supported for NCR

  - FFS: The details of the necessary legacy mechanism of the supported PUSCH

- Proposal 2-4-1:

This proposal is supported by all companies which replied.

Samsung suggested to add “FFS: whether to include additional timing offset or N_Delta value for NCR-MT”. As described in the FL summary, the original intention of this proposal is to confirm the following conclusion and agreement. It was concluded that legacy UE timing mechanism is sufficient and no new signaling is necessary. In this case, the FL thinks that the current proposal is sufficient as well. The “additional timing offset or N_Delta value for NCR-MT” is out of the scope of this proposal.

Conclusion: Legacy UE mechanism is sufficient to achieve DL/UL timing for NCR-MT. Agreement: For the signaling of the side control information of timing to align transmission / reception boundaries, new signaling is unnecessary.

- Proposal 2-4-2:

This proposal is stable which is supported by all companies which replied.

- Proposal 2-5-1:

24 companies provided their views on this proposal. 23 companies support this proposal. 1 company [ZTE] does not support Rel-17 beam indication framework (i.e., unified TCI). In response to the comments of vivo and Samsung, the proposal is updated as follows:

Proposal 2-5-1: For NCR-MT which can support adaptive beams in C link,

- Rel-15 beam indication framework is reused.

- Rel-17 beam indication framework (i.e., the unified TCI) is reused as well, if the NCR supports. The gNB can configure the unified TCI for the NCR-MT, if the NCR-MT supports.

- Proposal 2-5-2:

This proposal is stable which is supported by all companies which replied.

- Proposal 2-5-3:

This proposal is supported by all companies which replied except for Nokia and Apple.

Nokia and Apple commented that this proposal should be discussed after the WA regarding DL/UL beam correspondence for backhaul link. However, there seems no WA on the correspondence of BH link. The only WA made in last RAN1 meeting is about the AC link rather than BH link.

Besides, Samsung suggested adding “legacy UE sounding procedure including transmitting UL SRS can be…”. More clarification may be needed. In FL’s understanding, the sounding procedure is mainly about how to configure SRS. If the NCR-MT cannot transmit SRS, the FL does not know how the legacy sounding procedure can be supported.

- Proposal 2-5-4:

22 companies support this proposal and 2 companies do not want to support BFR for C link.

The proposal 2-5-4 is updated as follows as suggested by Ericsson:

Proposal 2-5-4: For NCR-MT which can support adaptive beams in C link, Rel-15 BFR mechanism is can be reused.

    - Proposed proposals

Proposal 2-3-1: At least ACK/NACK feedback of PDSCH is supported. The necessary legacy mechanism is reused.

- FFS: The details of the necessary legacy mechanism of ACK/NACK feedback of PDSCH.

- FFS: The UCI of ACK/NACK feedback of PDCCH

- FFS: Whether the UCI of SR is supported. Some legacy mechanism is reused if the UCI of SR is supported.

- FFS: Whether the UCI of CSI is supported. Some legacy mechanism is reused if the UCI of CSI is supported.

- Note: The PDSCH carries side control information and/or other information for NCR-MT.

Observation Conclusion 2-1: Based on those According to the four candidate solutions on NCR management given in TR38.867, it can be observed

- An NCR-MT can establish RRC connection with the gNB at least via 4-step random access procedure.

- An NCR-MT contains RRC layer and MAC layer.

Observation Conclusion 2-2: Based on those According to the candidate solutions on NCR management given in TR38.867, it can be observed

- An NCR-MT can access to the gNB as a normal UE, via Msg1 to Msg4.

- The gNB can identify an NCR-MT after the successful completion of the initial access procedure successes (via Msg 5 or later).

Proposal 2-2-1:

- Legacy 4-step initial access procedure is reused for NCR-MT.

- Legacy 4-step random access is reused for NCR-MT.

Proposal 2-4-1: Legacy TA adjustment mechanism is reused for NCR-MT.

Proposal 2-4-2: Legacy power control mechanism is reused for NCR-MT.

Proposal 2-5-1: For NCR-MT which can support adaptive beams in C link,

- Rel-15 beam indication framework is reused.

- Rel-17 beam indication framework (i.e., the unified TCI) is reused as well, if the NCR supports. The gNB can configure the unified TCI for the NCR-MT, if the NCR-MT supports.

Proposal 2-5-2: To support the legacy beam indication mechanisms for NCR-MT,

- The necessary legacy mechanism for receiving CSI-RS can be reused for NCR-MT.

- The necessary legacy mechanism for reporting CSI can be reused for NCR-MT.

- Note: this does not mean the legacy procedures for receiving CSI-RS and reporting CSI will be all supported. The details of the necessary mechanisms will be further discussed and decided.

Proposal 2-5-3: To support the legacy beam indication mechanisms for NCR-MT,

- The necessary mechanism of legacy UE sounding procedure can be reused for NCR-MT.

- Note: this does not mean the whole legacy UE sounding procedure will be supported. The details of the necessary mechanisms will be further discussed and decided.

Proposal 2-5-4: For NCR-MT which can support adaptive beams in C link, Rel-15 BFR mechanism is can be reused.

Proposal 2-3-2:

- PUCCH is supported for NCR. The necessary legacy mechanism is reused.

  - FFS: The details of the necessary legacy mechanism of the supported PUCCH

- PUSCH is supported for NCR

  - FFS: The details of the necessary legacy mechanism of the supported PUSCH

- Discussion round 2

  - The configuration for NCR-MT [Close]

(none)

  - Initial access procedure [Close]

(none)

  - PHY channels of NCR-MT

    - PUCCH (UCI) and PUSCH

Proposal 2-3-1: ACK/NACK feedback for L1/L2 signaling of side control information is supported.

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Apple | Support |
| Mod | Since some comments received offline, the moderator would like to clarify a little. In WID, we have the following scope where RAN1 is listed as the involved working group. Specify control plane signalling and procedures [RAN2, RAN1] - The configuration of signalling for side control information indication - NOTE: Down-selection of solutions in section 7.2 of TR 38.867 is needed. Hence, what we discuss here is in RAN1 scope. Considering the reliability of the transmission of side control information is very important, ACK/NACK is necessary. If the side control information such as beam/ON OFF indication is missed or wrongly decoded, the gNB and the NCR may have different understanding on the NCR behavior at a certain moment. Companies are encouraged to share your views on this proposal. Analyses about potential impacts on implementation/standardization with or without A/N feedback for L1 and/or L2 signaling are highly appreciated. |
| ZTE | Generally support. Just to clarify, does the L1/L2 signaling include RRC signaling or not? In TR 38.867, it has been agreed that both semi-static and dynamic signaling are supported for beam information, and the semi-static indication specifically refers to RRC signaling in our view. Besides, we want to emphasize that ACK/NACK feedback should be supported for DCI carrying side control information. Actually, ACK/NACK feedback in response to PDCCH has already been supported in NR, it’s not something new, e.g. in Clause 9.1 in TS 38.213, it’s specified that “if the UE receives a PDCCH indicating a SPS PDSCH release, the UE generates one corresponding HARQ-ACK information bit”. |
| CEWiT | Support the proposal in general. We completely agree with the clarification provided by moderator regarding the relevance of feedback for side control information of NCR. We share similar view with ZTE regarding feedback for SCI signalled through RRC. We suggest a more generalized proposal Proposal 2-3-1: ACK/NACK feedback for L1/L2 signaling of side control information is supported. |
| Intel | We support HARQ-ACK feedback for both PDSCH and PDCCH, for NCR-MT function or for side control information. We fully agree with moderator’s analysis on the benefit of HARQ-ACK feedback for PDCCH carrying side control information. And also, as pointed out by ZTE and also what I tried to comment on Tothru yesterday, HARQ-ACK feedback for PDCCH is already supported by legacy UE, e.g., for SPS release, Rel-17 unified TCI. Regarding whether L1/L2 signaling includes RRC signaling, we share same view with ZTE that typically semi-static signaling is by RRC. |
| Panasonic | Generally, we are fine with the proposal. However, it is not obvious whether all L1/L2 signaling of side control information requires ACK/NACK feedback. Here we propose to add sub-bullet of "FFS whether all L1/L2 signaling of side control information support ACK/NACK feedback". |
| NTT DOCOMO | We are fine with the proposal and share the same understanding with moderator and ACK/NACK feedback can be supported by the current mechanism, although the NCR is stationary. |
| LG | Regarding L2 signaling for side control information, we think RRC signaling can be used for semi-static side control information instead of MAC-CE signaling. For L1 signaling for side control information, we’d like to discuss further on the benefit and specification impact of ACK/NACK feedback for DCI carrying side control information. It is desirable to reuse the legacy mechanism as much as possible. However, we are not sure that the legacy mechanism (e.g., ACK feedback for SPS PDSCH release) can be totally reused for DCI carrying side control information. In our view, a new DCI is necessary for side control information, instead of reusing the legacy DL grant (e.g., DCI format 1_0/1_1/1_2) such as PDCCH indicating SPS PDSCH release. In this case, it may have some new spec impacts (e.g., how to decide PUCCH slot timing, how to indicate PUCCH resource, how to mapping ACK bit in HARQ-ACK codebook, …). By the way, ACK/NACK feedback should be modified to ACK feedback in case of L1 signaling of side control information, since NACK feedback may not be available for DCI. In summary, we’d like to modify the proposal as follow. Proposal 2-3-1: ACK/NACK feedback at least for L1/L2 signaling of side control information via PDSCH (e.g., MAC-CE, RRC) is supported. - FFS: ACK feedback for PDCCH carrying side control information |
| Lenovo | We are fine with supporting ACK/NACK for PDCCH carrying control information if the majority want to support it, but we prefer LG’s updated proposal to keep it as FFS |
| IIT-K | Support in general. Agree with the moderator’s reasoning that for missed or wrongly decoded SCI, ACK/NACK is necessary. Also agree with ZTE, CEWiT and LG about feedback for SCI signaled through RRC. |
| vivo | Feedback for L2/L3 signaling is straightforward, it seems a conclusion. For the L1 SCI feedback, we support it. |
| Fujitsu | Support HARQ-ACK feedback for both side control information carried by PDSCH (RRC/MAC CE) and PDSCH, for the same reason as mentioned by many other companies. For LG’s comments that only ACK is needed for DCI, we agree with it and it should be the common understanding. Maybe “ACK/NACK” can be replaced by “HARQ-ACK”, which is the wording used in TS 38.213. In a summary, we suggest the following changes: Proposal 2-3-1: HARQ-ACK feedback for side control information carried by PDSCH (i.e. side control information in MAC CE, RRC messages) and PDCCH (i.e. side control information in DCI) is supported. |
| Sony | In general okay with the proposal, but prefer LG’s formulation with ACK for PDCCH as FFS. |
| Ericsson | Do not support the proposal. We support higher layer ACK/NACK and it is up to RAN2 to decide if that is used or not. This is carried by PDSCH and ACK/NACK can reuse existing HARQ functionality. L1 ACK/NACK, on the other hand, is expected to be applied to dynamic (and potentially ON/OFF) beam indication that has a limited duration. Additionally, for many network configurations, there may not even be an UL slot configured at a suitable time (not too early, not too late) of the indication duration. For these configurations, ACK/NACK would be of no use. The specification effort for a completely new L1 ACK/NACK should not be underestimated. This would be a new HARQ scheme for a very dynamic configuration that should fit PUCCH time resources and will have significant impact on the feasible TDD pattern. This is different from the two examples provided by Intel, SPS release and Rel-17 unified TCI. Additionally, we assume the backhaul link to be robust, some companies even propose not to include RLF. With proper network planning availability of the repeater should be a traditional wireless backhaul link with an availability of at least 4 nines. For such a link, the value of L1 ACK/NACK would be very limited. Companies supporting L1 PDCCH ACK/NACK should consider that it can take several retransmissions before a dynamic indication can be decoded using HARQ. During that time, the gNB would find itself in an ambiguous state, not knowing if the link is there or not. This risk should be avoided already at deployment. |
| Samsung | We support HARQ-ACK feedback for side information provided by higher layers (e.g., RRC) and/or L2 layer, which is standard procedure for data included in a PDSCH. Regarding HARQ-ACK feedback for side information provided by L1 (i.e., PDCCH including a DCI format for dynamic side control information for NCR), we think more discussion is needed. There is a trade-off between reliability and latency of L1 signaling. We prefer to decide on this aspect after RAN1 makes further progress on the design of time resources for dynamic beam / ON-OFF indication and corresponding application time. So, we suggest the following modification: Proposal 2-3-1: ACK/NACK feedback for PDSCH that includes L1/ higher layer or L2 signaling of NCR side control information is supported. - FFS: whether to support ACK/NACK feedback for PDCCH that includes L1 signaling of NCR side control information |
| KDDI | As some other companies mentioned, HARQ-ACK for the PDSCH conveying RRC signaling and/or MAC CE for side control information should be supported, which is normal operation for PDSCH conveying other data. Regarding HARQ-ACK for PDCCH conveying dynamic side control information, we think it needs to be discussed further because the problem such as latency indicated by some companies is understandable, although we slightly prefer to support it. |
| CMCC | We have to say we share the exactly the same idea with KDDI 😊. Yes the legacy behavior that the RRC and MAC CE information carried in the PDSCH should be supported. and the HARQ-ACK should be supported for those PDSCH. And for the HARQ-ACK of PDCCH carrying the dynamic side control signaling should be also supported. But the details should be FFS. The side control information or signaling without an acknowledgement in physical layer is not preferred. |
| Nokia | In our view this proposal may be too general. Side control information transmitted as PDCCH does not necessarily require ACK/NACK and we do not want to commit to it without clear justification. |
| InterDigital | We are fine with the proposal 2-3-1. |
| CATT | At this stage we can only agree HARQ-ACK for PDSCH . Regarding HARQ-ACK of PDCCH, we think more discussion is needed. |
| NEC | Support if the side control information here means only the explicit ones. |
| Huawei, HiSilicon | Don’t support the current proposal. ACK/NACK of PDSCH can follow legacy rules. However, ACK/NACK feedback of PDCCH is unnecessary since it brings extra and unpredictable latency, which may make the gNB scheduling invalid. |

Proposal 2-3-2: PUCCH and PUSCH are supported for NCR-MT.

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Apple | Fine to support |
| ZTE | Support |
| CEWiT | Support |
| Intel | OK. |
| Panasonic | Support |
| NTT DOCOMO | Support |
| LG | Support |
| Lenovo | Support |
| IIT-K | Support |
| Vivo | Yes |
| Fujitsu | Support |
| Sony | Support. |
| Ericsson | Support |
| Samsung | Support |
| KDDI | Support |
| CMCC | Support |
| Nokia | Support the proposal. |
| InterDigital | We are fine with the proposal 2-3-2. |
| CATT | support |
| NEC | Support. |
| Huawei, HiSilicon | Support |

    - PDCCH (DCI) [Close]

(none)

  - TA adjustment, power control and power saving for NCR-MT (C link)

    - TA adjustment for C link

Proposal 2-4-1: Legacy TA adjustment mechanism is supported for C link.

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Apple | Support |
| ZTE | Support |
| CEWiT | Support |
| Intel | Support |
| Panasonic | Support |
| NTT DOCOMO | Support |
| LG | Support |
| Lenovo | Support |
| IIT-K | Support |
| Vivo | Yes |
| Fujitsu | Support |
| Sony | Support |
| Ericsson | Support |
| Samsung | Fine with the proposal. As mentioned in the previous round, suggest to add: FFS: whether to include additional timing offset or N_Delta value for NCR-MT The gNB may need to provide a fixed slot/symbol timing adjustment to align the NCR-MT with the network-wide gNB TDD timing (similar to IAB). This is still based on legacy timing procedure, with the N_delta parameter helping if the internal delay of NCR is non-negligible. |
| KDDI | Support |
| CMCC | Support |
| Nokia | Support the proposal. |
| InterDigital | We are fine with the proposal 2-4-1. |
| CATT | Support |
| NEC | Support. |
| Huawei, HiSilicon | Support |

    - Power control for C uplink [Close]

(none)

  - Beam-relevant functionalities for C link

    - Beam indication mechanism [Close]

(none)

    - CSI measurement and report [Close]

(none)

    - SRS

Proposal 2-5-3: To support sounding procedure for NCR-MT in C link, the necessary mechanism of legacy UE sounding procedure is supported.

  - FFS: The details of the necessary mechanism of legacy UE sounding procedure

  - Note: this does not mean all legacy UE sounding procedure will be supported.

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Apple | If beam correspondence is assumed for UL/DL at NCR-MT, then we don’t think this proposal is needed. |
| Mod | The moderator agrees that the function of SRS for beam management may be unnecessary if the beam correspondence between UL/DL beams in C link is guaranteed. However, SRS is a basic feature of legacy UEs and can be used by the Gnb for some purposes, e.g., for UL timing tracking. There may be a potential risk if we directly conclude that SRS is unnecessary only because the beam correspondence between UL/DL beams in C link is guaranteed. Companies are encouraged to share your views on this proposal. Analyses about potential impacts on implementation/standardization with or without SRS are highly appreciated. |
| ZTE | Support. For usage of SRS, in addition to beam management, it can also be used for codebook or non-codebook based UL transmission, antenna switching. Usage ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching} |
| CEWiT | Support |
| Intel | Support. We agree with the assessment by Apple that beam management without SRS can work properly, if beam correspondence is assumed. But we also share similar view with ZTE that SRS may still be useful for other purposes. If we don’t support SRS, NCR may still work, e.g., using single port with single layer, but the question is, whether the performance degradation is neglectable if we don’t support SRS at all. |
| Panasonic | Support |
| NTT DOCOMO | Support |
| LG | We support the proposal, also we think Rel-15 sounding procedure is sufficient. We’d like to clarify whether NCR operation in FDD band is in the WI scope or not. If Rel-18 NCR can be operated in FDD band, beam correspondence between UL/DL cannot be assumed, so the function of SRS for beam management seems necessary for NCR-MT. We think this clarification is also required to the proposal to confirm the WA on the UL/DL beam correspondence of access link in 9.8.1. |
| Lenovo | Fine to support |
| IIT-K | Support |
| Vivo | Support |
| Fujitsu | Support. |
| Sony | Support. |
| Ericsson | Support |
| Samsung | Support Agree with other companies that SRS is used for various NCR-MT procedures, and is also beneficial for gNB measurement, configuration and scheduling. We understand any simplification/restrictions for SRS, if needed, can be further discussed based on the FFS point. |
| KDDI | Support |
| CMCC | Support. The legacy behavior of SRS could be a starting point, further narrowing down is needed. |
| Nokia | Support the proposal. |
| InterDigital | We are fine with the proposal 2-5-3 |
| CATT | Support |
| NEC | Support. |
| Huawei, HiSilicon | Support |

    - Beam management

Proposal 2-5-4: For NCR-MT which can support adaptive beams in C link, Rel-15 BFR mechanism is supported.

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Apple | As commented in the 1st round, we don’t think BFR mechanism is needed. However, if the majority want to support this legacy mechanis, we can accept |
| Mod | Like what the moderator comments on A/N feedback and SRS, BFR is an important feature of legacy UEs as well. The NCR-MT does or does not support BFR in C link may have impacts on BH link. The moderator would like to ask companies carefully consider this proposal. Analyses about potential impacts on implementation/standardization with or without BFR in C link are highly appreciated. |
| ZTE | Rel-15 BFR mechanism is not needed for NCR MT. As explained in last round, NCR is stationary and the link between Gnb and NCR is stable, the recovery of beam doesn’t provide benefit since the whole link would be failed if a moving blockage stays between Gnb and NCR, the only solution is to re-establish the radio link. Given that no benefit can be obtained by BFR mechanism, from complexity reduction perspective, we suggest to conclude that BFR is not needed for NCR MT. |
| Intel | We’re fine with BFR based on majority view, though we think BF would rarely happen considering NCR is stationary. BTW, we suggest to add BFD together with BFR. |
| Panasonic | Support |
| NTT DOCOMO | Support |
| LG | We don’t object to agree on the proposal, but we are not sure BFR mechanism is necessary for NCR-MT since it is stationary node. |
| Lenovo | We don’t see an issue of supporting the legacy BFR procedure or at least a simplified one based on similar reason for supporting dynamic beam indication, CSI-RS measurement, and ACK/NACK feedback for the C-link. I think these have been supported by majority companies assuming that the C-link is not fully stable. |
| Vivo | Support |
| Fujitsu | Support. Although NCR is stationary, it is possible that the active C-link beam is blocked sometimes while there is another C-link beam can be used. In such case, BFR is beneficial. And it can be an optional function for NCR-MT. And we agree with Intel that BFD could be added. |
| Sony | Support. |
| Ericsson | Support legacy behavior. |
| Samsung | We are OK to support Rel-15 BFR for the case of C-link with adaptive beam. For C-link with fixed beam, BFR does not apply. So, suggest to add the following note: Proposal 2-5-4: For NCR-MT which can support adaptive beams in C link, Rel-15 BFR mechanism is supported. - Note: BFR mechanism is not supported for NCR-MT which supports only fixed beam for C-link. |
| KDDI | Support. |
| CMCC | We are fine to support the legacy behavior of Rel-15 BFR for the C-link with adaptive beams. |
| Nokia | Support the proposal. |
| InterDigital | We are fine with proposal 2-5-4. |
| CATT | Support |
| NEC | Support. Further study on simplify the legacy procedure by considering stability may be needed to decrease the complexity. |
| Huawei, HiSilicon | Support |

  - Summary on round 2

In round 2, 15 companies input their views before the deadline.

- Proposal 2-3-1

9 companies support the ACK/NACK of both PDSCH and PDCCH. 5 companies support the ACK/NACK of PDSCH and propose to leave the ACK/NACK of PDCCH for further study. 1 company do not support the proposal but would like to support PDSCH with legacy HARQ functionality (please correct me if I misunderstood any). Based on the comments, the proposal 2-3-1 is updates as above which mainly based on the suggestion from LG and Fujitsu. Besides, in the discussion, higher layer ACK/NACK was mentioned by Ericsson. The FL shares the same view with Ericsson’s that higher layer ACK/NACK is a higher layer issue. Whether higher layer ACK/NACK is supported should be decided by RAN2 and is out of the intention of this proposal. Proposal 2-3-1 was approved by Chairman as follows.

Updated Proposal 2-3-1: HARQ-ACK feedback for PDSCH carrying the side control information from higher layer (e.g., MAC-CE, RRC) is supported. The legacy HARQ-ACK feedback mechanism is reused.

- FFS: Whether HARQ-ACK feedback for PDCCH carrying side control information is supported

- Note: This does not mean all legacy HARQ-ACK feedback mechanism will be supported.

- Proposal 2-3-1

Proposal 2-3-2 is supported by all companies which attended the discussion round 2. Proposal 2-3-2 was approved by Chairman as follows,

Proposal 2-3-2: PUCCH and PUSCH are supported for NCR-MT.

- Proposal 2-3-1

All companies support the proposal. Samsung would like to add an FFS into this proposal, “FFS: whether to include additional timing offset or N_Delta value for NCR-MT”. In FL’s understanding, the intention of this proposal is all about the legacy mechanism. We can discuss new feature in another proposal if necessary.

In the discussion after round 2, Samsung provided following comment to propose that an additional configurable timing offset value to align NCR-MT timing to network-wide TDD timing may be necessary. What Samsung proposed is not in line with the conclusion achieved in SI. The FL thinks the issue relevant to proposal 2-4-1 needs to be carefully discussed in next round. We may need to discuss whether the conclusion we made in RAN1#109e has to be revisited.

In the SI, we concluded that the legacy UE mechanism is sufficient to achieve DL/UL timing for NCR-MT. If the NCR-MT was to operate standalone, e.g., in absence of any NCR-Fwd operation, that would be sufficient. But we need to account for the presence of the RF delay in the NCR-Fwd when we multiplex the C-link. The NCR-Fwd inserts at least 1-2 usec between Rx from gNB on backhaul link and Tx to UE on access link, similarly for Rx from UE on access link and Tx to gNB on backhaul link. For large delay repeaters, several usec are incurred. The UE TA values will generally be larger than the NCR-MT TA due to larger distance between UEs and gNB when compared to distance between NCR and gNB. In addition, from the gNB perspective, the UE TA values will absorb 2 times the NCR-Fwd RF delay. If the one-way NCR-Fwd RF delay is 2 us, this is equivalent to an increase in UE-gNB distance of 2x300m. The gNB increases the UE TA by 2x 2 usec “as if the UE was 600 m away”. UE UL transmissions will start 4 us earlier to account for RF delay and at most 1 usec to account for actual UE-gNB distance. NCR-MT UL transmissions and the legacy NCR-MT TA however start at fractions of a usec earlier. So we have a delta of some 4-5 usec to cover between earlier start of UE UL transmissions where the NCR doesn’t know the TAs and a much later start of the NCR-MT UL transmissions with respect to the UL reference slot. The gNB should be able to advance the NCR-MT UL transmission such that it aligns better with the earlier UE UL. 4-5 us is too small to cover just using the (relative) TA command. The (absolute) TA command as used during initial access can more easily address such a range, but that would require an NCR-MT procedure change when compared to legacy UE behavior. Not doing anything would cost an additional guard symbol to use during scheduling by the gNB.

- Proposal 2-5-3

14 companies support this proposal. 1 company [Apple] does not support this proposal. The company thinks that the correspondence between UL/DL is sufficient.

In the discussion, some companies share more details about why SRS is necessary for NCR-MT:

- ZTE: For usage of SRS, in addition to beam management, it can also be used for codebook or non-codebook based UL transmission, antenna switching. - Usage ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching} - Intel: If we don’t support SRS, NCR may still work, e.g., using single port with single layer, but the question is, whether the performance degradation is neglectable if we don’t support SRS at all. - Samsung: Agree with other companies that SRS is used for various NCR-MT procedures, and is also beneficial for gNB measurement, configuration and scheduling.

The discussion on SRS is expected to be converged in next round.

Besides, LG raised a question on FDD band. It seems that the confusion of LG has been solved by the FL of 9.8.1 in another email thread.

- Proposal 2-5-4

11 companies would like to support this proposal. 2 companies provided their considerations on why BFR/BFD are necessary even though the NCR is assumed to be always stationary [Lenovo, Fujitsu]. They think even though the NCR device/equipment is stationary, the link between the NCR-MT and the gNB cannot be always stable. BFD/BFR can help to enhance the stability of C link. Two companies said they are not fully convinced why BFD/BFR is necessary for stationary NCRs, though they would like to support this proposal [Intel, LG].

2 companies do not support this proposal [Apple, ZTE]. They think stationary NCRs do not need BFD/BFR. Radio link re-establishment can be reused in case that the link is not stable [ZTE].

From the point of view of the FL, the discussion seems premature. More study/consideration is needed. The FL would like to encourage companies to carefully consider the necessity of BFD/BFR for NCR-MT and provide more details on the potential impact on C/BH link with/without BFD/BFR in company contributions for next RAN1 meeting.

- Discussion round 3

  - TA adjustment for C link

RAN1#109e: Conclusion: Legacy UE mechanism is sufficient to achieve DL/UL timing for NCR-MT.

In last round, one company would like to add an FFS to proposal 2-4-1 [Samsung]. From FL’s point of view, the ‘sufficient’ in the above conclusion is very clear that no design in addition to the legacy UE mechanism is needed to achieve DL/UL timing for NCR-MT. Hence, we may have to discuss whether the above conclusion should be revisited before we discuss whether an FFS bullet is needed for proposal 2-4-1.

The FL would like to encourage companies to provide your view on following issues:

- Do you think the above conclusion should be revisited?

- Please provide your answer to following questions if you think the revisit is necessary.

  - The problem: Why the legacy UE mechanism is not sufficient to achieve DL/UL timing for C-link/NCR-MT?

  - The solution: How can the DL/UL timing for C-link/NCR-MT be achieved if the legacy UE mechanism is not sufficient?

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Apple | We think that the conclusion from RAN1#109-e should not be revisited. |
| ZTE | - No. Legacy UE mechanism is sufficient. |
| LG | We think the conclusion in RAN1#109e is sufficient. According to the study result, the internal delay of NCR affects the timing of the access link and does not affect the timing of C-link and backhaul link. We don’t see the reason not to follow the TA adjustment mechanism of legacy UE for C-link. |
| vivo | Legacy TA mechanism is sufficient |
| Fujitsu | Answer to question 1: No, we should not revisit/reverse the conclusion. Legacy TA mechanism is sufficient. |
| Huawei, HiSilicon | We think the above conclusion is fine as it is. |
| Intel | We share same understanding with other companies that legacy UE TA mechanism is sufficient. |
| NEC | Legacy mechanism is sufficient. |
| Lenovo | We think the legacy TA mechanism is sufficient |
| Samsung | Unlike the Rel-16 IAB node which is a L2 relay and can set its own DL Tx timing in baseband ahead of its DL Rx timing by Rel-15 UE legacy TA/2 + configurable T_delta value, the NCR-Fwd cannot control the DL transmission timing of its RF amplification and forward path. Therefore, the UL transmit timing of the NCR-MT control link must be adjusted with respect to the NCR DL reference timing (common to NCR-MT DL Rx and NCR-Fwd Rx from backhaul) and the Rel-15 legacy UE TA procedure. The NCR should set its UL Tx timing as by Rel-15 UE legacy TA value + T_delta to avoid mis-aligned symbol boundaries and need for 1 additional guard symbol when scheduling the UEs being relayed by the NCR. Please check the following figure for an illustration of the issue. |
| Panasonic | No, there is no need for revisiting. |
| KDDI | We think we don’t need revisiting because the legacy TA mechanism is sufficient. |
| Sony | Do not revisit the above conclusion. Legacy mechanism is sufficient. |
| Ericsson | No need for revisiting. In response to Samsung: There are two timing problems: - DL/UL switching, and - Beam switching in either DL or UL. For DL/UL switching, the repeater’s TA will be larger than UEs that are being serviced by the gNB via the repeater since the propagation path for them are never shorter than that of the repeater. As a consequence, the repeater will receive a DL signal ahead of any serviced UE and transmit an UL signal delayed to the same UEs. Considering the marginal internal latency of the repeater-Fwd (on ns level, or single digit meters in distance), there will be a window in-between the repeater-Fwd has finished forwarding the DL transmission and the repeater-Fwd starts to forward the UL transmission, within which the repeater can change the repeater-Fwd’s direction. For beam switching, the repeater will switch beams in relation to its own DL Rx (or UL Tx timing). Considering the repeater will know it’s internal delay, this can also be accounted for to make the switching in the best possible instant in relation the any change in the access side beamforming and forwarding direction, provided the serviced UEs TA is correctly configured. |
| Mediatek | No, we don’t think the conclusion needs to be revisited. |
| Mod2 | @ Samsung Based on your figure, I draw a figure too. I’d like to show, given that the TA of the UE1 and UE 2 and UE 3 are all properly adjusted by the gNB via the legacy mechanism, - The UL signal of UE1 and UE2 align with each other when they are received by the antenna of AC link, shown as point B. - The amplified signal (the combination of UL signal of UE1 and UE2) and the UL signal of MT align with each other right before they are sent by the antenna of BH/C link, shown as point A. - The UL signal from the NCR (the combination of UL signal of UE1 and UE2 and NCR-MT) and the UL signal of UE3 align with each other when they are received by the Rx antenna of the gNB. Besides, UE 1 and UE 2 have dedicated reference signal which experience the overall latency from the UEs to the gNB. The gNB can correctly estimate the latency of UE1 an UE2 and indicate proper TA value to them to achieve the synchronization between UE1, UE2, UE3 and NCR-MT like the above figure shows. |
| Nokia | Conclusion does not need to be modified; legacy UE mechanism is sufficient. |
| InterDigital | We do not think the conclusion should be revised. The legacy UE mechanism is sufficient to achieve DL/UL timing for NCR-MT. |
| Sharp | Legacy mechanism is sufficient. |
| Qualcomm | No – no need to revisit the conclusion. |
| NTT DOCOMO | No, legacy mechanism is sufficient. |

The intention of Proposal 2-4-1 is about the mechanism to achieve DL/UL timing of C link. The FL would like to ask companies’ opinion which proposal you prefer, Proposal 2-4-1-v1 or Proposal 2-4-1-v1-SS.

Proposal 2-4-1-v2: The TA adjustment mechanism of legacy UEs is supported for NCR-MT in C link.

Proposal 2-4-1-v1-SS: The TA adjustment mechanism of legacy Ues is supported for C link.

- FFS: whether to include an additional configurable timing offset value to align NCR-MT timing to network-wide TDD timing

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Apple | We support Proposal 2-4-1-v1 |
| ZTE | We are generally fine with Proposal 2-4-1-v1. But considering that all other proposals state that sth is supported for NCR-MT in C-link, we suggest minor modification to align with other proposals. Proposal 2-4-1-v1: The TA adjustment mechanism of legacy Ues is supported for NCR-MT in C link. We’ve already had enough discussion in SI phase to discuss the timing issue, we don’t see the need to revert previous conclusion to have additional configurable TA value. As for the internal delay of NCR-Fwd, it can be resolved by configuring appropriate TA value incorporating the internal delay to UE using legacy TA adjustment. To be more specific, without repeater, UE can be provided by appropriate TA considering the RTT between UE and gNB, and with repeater, UE can be provided with appropriate TA which can also incorporate the internal delay of NCR-Fwd without awareness of NCR, therefore legacy UE mechanism is enough as we agreed in SI phase for NCR-MT. As for timing for NCR-Fwd, we have also agreed on the following in SI phase, it can be adjusted according the timing of NCR-MT, so there is also no issue for NCR-Fwd. - The DL receive timing of the NCR-Fwd is aligned with the DL receive timing of the NCR-MT. - The UL transmit timing of the NCR-Fwd is aligned with the UL transmit timing of the NCR-MT. - The DL transmit timing of the NCR-Fwd is delayed after the DL receive timing of the NCR-MT (or the NCR-Fwd) by an internal delay; - The UL receive timing of the NCR-Fwd is advanced before the UL transmit timing of the NCR-MT (or the NCR-Fwd) by an internal delay. |
| LG | We support Proposal 2-4-1-v1. Also fine with ZTE’s modification. |
| Mod | The proposal is updated as suggested by ZTE. |
| Vivo | Support Proposal 2-4-1-v2 |
| Fujitsu | We support Proposal 2-4-1-v1. The FFS in Proposal 2-4-1-v1-SS is for timing enhancement for NCR-MT. It is not aligned with the conclusion above that legacy UE mechanism is sufficient to achieve DL/UL timing for NCR-MT. In addition, RAN1 has had comprehensive and sufficient discussions on timing in SI and concluded that no enhancement for timing is needed for NCR (for both NCR-MT and NCR-Fwd). Consequently, timing issue is not captured in WID. The FFS is out of WID scope and should not be discussed. |
| Huawei, HiSilicon | Support Proposal 2-4-1-v2. |
| CMCC | Support Proposal 2-4-1-v2 |
| Intel | Support Proposal 2-4-1-v2 |
| NEC | We prefer proposal 2-4-1-v2. |
| Lenovo | We support proposal 2-4-1-v2 |
| Samsung | Support Proposal 2-4-1-v1-SS. We think there is a real issue for TA handling for NCR-MT which is worth additional consideration. We are open to discuss any solutions that companies may have, but we think an additional RRC configurable timing offset for the NCR on top the already RRC signalled (or default value) legacy Rel-15 UE N_TA,offset can be a simple and clean solution with small spec impact similar to the way Rel-16 IAB solved the issue. Please note, there is no new side control information for the NCR-Fwd. RRC configuration of the NCR is sufficient and anyway necessary when the NCR is initially configured by the gNB with TA parameters as by the legacy UE procedure. |
| Panasonic | We prefer Proposal 2-4-1-v1. |
| KDDI | We are fine with Proposal 2-4-1-v1. Also fine with ZTE’s modification. |
| Sony | We support proposal 2-4-1-v2. |
| Ericsson | Support Proposal 2-4-1-v2. |
| Nokia | Prefer Proposal 2-4-1-v2. |
| InterDigital | We support Proposal 2-4-1-v1. Also find with ZTE’s modification. |
| Sharp | Support Proposal 2-4-1-v2 |
| Qualcomm | Support Proposal 2-4-1-v2. |
| NTT DOCOMO | Support Proposal 2-4-1-v2. |

  - SRS for C link

In last round, some companies share details on potential impacts on C link with or without SRS for NCR-MT as follows:

- Apple: If beam correspondence is assumed for UL/DL at NCR-MT, then we don’t think this proposal is needed. - ZTE: For usage of SRS, in addition to beam management, it can also be used for codebook or non-codebook based UL transmission, antenna switching. - Usage ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching} - Intel: We agree with the assessment by Apple that beam management without SRS can work properly, if beam correspondence is assumed. But we also share similar view with ZTE that SRS may still be useful for other purposes. If we don’t support SRS, NCR may still work, e.g., using single port with single layer, but the question is, whether the performance degradation is neglectable if we don’t support SRS at all. - Samsung: Agree with other companies that SRS is used for various NCR-MT procedures, and is also beneficial for gNB measurement, configuration and scheduling.

The FL would like to encourage companies consider the above analyses and reconsider whether following proposal is agreeable.

Proposal 2-5-3: To support sounding procedure for NCR-MT in C link, the necessary mechanism of legacy UE sounding procedure is supported.

  - FFS: The details of the necessary mechanism of legacy UE sounding procedure

  - Note: This does not mean all legacy UE sounding procedure will be supported.

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Apple | Considering that we want to reuse most of the legacy UE procedures, the proposal could be acceptable. However, we would like clarification on the FFS that this doesn’t include any enhancements compared to legacy UE sounding procedure. In our view, FFS is mainly to determine whether and which legacy procedure is fully supported or only certain aspects |
| ZTE | Support. |
| LG | We support the proposal, also we think Rel-15 sounding procedure is sufficient. |
| Mod | @Apple: The intention of the FFS bullet is the same as you described. |
| vivo | Support |
| Fujitsu | We support the proposal. |
| Huawei, HiSilicon | Support |
| CMCC | Support |
| Intel | We support the proposal. |
| NEC | Support. |
| Lenovo | Support |
| Samsung | Support. UL beam management based on SRS can be considered for NCR-MT, including beam indication for PUSCH and PUCCH. In addition, beam correspondence for NCR-MT can be with or without UL beam sweeping, then SRS will be needed if NCR-MT supports beam correspondence *with* UL beam sweeping. Furthermore, SRS resource indicator (SRI) can be used for indication of PUSCH precoder (for PUSCH scheduled by DCI format 0_1/0_2). Therefore, support for SRS is clearly needed. We are open to discuss simplifications/restrictions on the usage of SRS, if applicable, to achieve lower complexity/cost NCR. |
| Panasonic | Support |
| KDDI | Support. |
| Sony | Support. |
| Ericsson | Support |
| Nokia | Support the proposal. |
| InterDigital | We support proposal 2-5-3. |
| Sharp | Support |
| CATT | Support |
| Qualcomm | Support. |
| NTT DOCOMO | Support. |

  - Others

Observation 2-1: Based on those four candidate solutions on NCR management given in TR38.867, it can be observed

- An NCR can establish RRC connection with the gNB at least via 4-step random access procedure.

- An NCR contains RRC layer and MAC layer.

Observation 2-2: Based on those four candidate solutions on NCR management given in TR38.867, it can be observed

- An NCR can access to the gNB as a normal UE, via Msg1 to Msg4.

- The gNB can identify an NCR after the initial access procedure successes (via Msg 5 or later).

In round 1, observation 2-1 and observation 2-2 have been supported by almost all companies. The FL would like to ask companies opinion, do you think whether the NCR-MT supports legacy initial access/random access procedure should be confirmed in RAN1 now. If your answer is ‘Yes’, please provide more details on why you think so.

| Companies are encouraged to share your views. |  |
| --- | --- |
| Company | Comments |
| Apple | Although we are fine with the two proposals, but not sure, if it is up to RAN1 to decide that NCR contains RRC and MAC layers. In principle, since we want to apply legacy procedures from UE, so the above observations are fine based on that assumption. Otherwise, it doesn’t make sense to reinvent the wheel related to initial access procedure. |
| ZTE | OK with either way. |
| LG | We are fine with both observations. |
| Mod | The intention of this comment box is not for seeking support of the observations. The FL would like to ask companies opinion about whether it should be confirmed in RAN1 now that the NCR-MT supports legacy initial access/random access procedure. |
| vivo | Observation 2-1 is better. it is beneficial to confirm the RRC layer to facilitate the further RAN1 discussion on the signalling design. |
| Fujitsu | OK with either way. |
| Huawei, HiSilicon | We think it is reasonable to assume that NCR-MT inherits the legacy UE capabilities, e.g., timing, power control. RAN1 can further discuss whether a certain functionality can be simplified given the specific conditions of NCR-MT. We are fine to have a formal confirmation for the initial access procedure, i.e. observation 2-2. |
| CMCC | We are fine to confirm that the NCR could access to the NCR as a normal UE and setup the RRC connection to the gNB which is similar to the IAB-MT. |
| Intel | We think we can confirm the legacy initial access/random access procedure is reused for NCR-MT, for RAN1’s part. But the detailed higher layer signaling itself during the initial access procedure should be RAN2/RAN3 to decide, which depends on options for verification. |
| Lenovo | We are fine to confirm reusing the legacy UE initial access for the NCR-MT, and the identification/NCR capability exchange is done after that. |
| Samsung | OK with the observation, although we think these are RAN2 domain, and may not require RAN1 conclusion. We are fine to conclude on RAN1 procedures, such as 4-step RACH, and so on, without reference to higher layer aspects. |
| Panasonic | We are fine with both observations. |
| KDDI | If there is no strong objection to confirm those observation now, we are fine with it because we believe it facilitate the further discussion. |
| Nokia | It does not seem necessary for RAN1 to capture the given Conclusions, as at least some portion of them relates to work beyond the scope of RAN1 and would not have bearing on RAN1 specs. It should be sufficient to state that NCR can rely on random access procedure as specified in Section 8 of TS 38.213. |
| Sharp | Fine with the observations. |
| CATT | RRC/MAC are within the ran2 scope , thus agreeing to the observation 1 is meaningless in RAN1. RAN1/2 are jointly responsible for random access procedure . Therefore we need to make observation 2 to a proposal, and if it is agreed, send LS to inform RAN2. We support observation 2 ( should be proposal then agreement) |
| Qualcomm | We support having such a formal conclusion /confirmation in RAN1. |
| NTT DOCOMO | We are fine with the both observations. |

  - Summary on round 3

- TA adjustment for C link

RAN1#109e:

Conclusion: Legacy UE mechanism is sufficient to achieve DL/UL timing for NCR-MT.

In last round, one company would like to add an FFS to proposal 2-4-1 [Samsung]. From FL’s point of view, the ‘sufficient’ in the above conclusion is very clear that no design in addition to the legacy UE mechanism is needed to achieve DL/UL timing for NCR-MT. Hence, we may have to discuss whether the above conclusion should be revisited before we discuss whether an FFS bullet is needed for proposal 2-4-1.

In round 3, 20 companies provided their views on whether the conclusion on UL/DL timing of NCR-MT should be revisited. 19 companies think the revisit is not necessary. One company [Samsung] thinks the conclusion may be not sufficient. An additional configurable timing offset are needed for NCR-MT. The reason why they propose so is carefully explained. However no second company thinks an additional new feature is required to achieve DL/UL timing for NCR-MT in C link.

The discussion on the issue raised by Samsung has already been carefully discussed. The FL would like to propose proposal 2-4-1 again. In this round, this version is supported by all companies except for Samsung.

Proposal 2-4-1-v2: The TA adjustment mechanism of legacy UEs is supported for NCR-MT in C link.

- SRS for C link

In this round, the discussion on SRS for C link is controversial. Proposal 2-5-3 is supported by all companies.

Proposal 2-5-3: To support the sounding procedure for NCR-MT in C link, the necessary mechanism of legacy UE sounding procedure is supported.

  - FFS: The details of the necessary mechanism of legacy UE sounding procedure.

  - Note: This does not mean all legacy UE sounding procedure will be supported.

- Others

Regarding to the confirmation of legacy initial access/random access procedure, most of companies are fine with it. According to the progress of the ongoing RAN2 NCR discussion, some solid conclusion relevant to RRC can be expected. We can wait for the progress of RAN2 and see what we can do in RAN1 in next meeting.

    - Proposed proposals

Proposal 2-4-1-v2: The TA adjustment mechanism of legacy UEs is supported for NCR-MT in C link.

Proposal 2-5-3: To support the sounding procedure for NCR-MT in C link, the necessary mechanism of legacy UE sounding procedure is supported.

- FFS: The details of the necessary mechanism of legacy UE sounding procedure.

- Note: This does not mean all legacy UE sounding procedure will be supported.

- Appendix: Company views

  - About configuration for NCR-MT

### R1-2208423	Configuration for side control information signalling of network-controlled repeaters	Huawei, HiSilicon

Proposal 1: The necessary configuration for receiving the PHY channels and L1/L2 signaling of the side control information is configured via RRC.

### R1-2209019	Discussion on control plane signaling and procedures relavant to RAN1	Fujitsu

Observation 1: NCR-MT will support RRC layer.

Proposal 1: RAN1 focuses on how the NCR-MT obtains configuration for receiving the L1/L2 signaling of side control information from RRC.

### R1-2209066	Discussions on L1/L2 signaling and configuration for side control information	Intel Corporation

Proposal 9: Support semi-static indication for side control information by MAC-CE or RRC signaling by a PDSCH. FFS details for each side control information by MAC-CE and/or RRC signaling.

### R1-2209351	Discussion on other aspects including control plane signalling and procedures relevant to RAN1	CMCC

Proposal 1: The protocol stacks of the MT part of NCR should contain RRC, MAC and physical layer to support the semi-static configurations and dynamic indications and feedbacks.

Proposal 3: If needed, the RRC configuration based mechanisms could be prioritized for further discussions.

### R1-2209745	On signaling and procedures for network-controlled repeaters	Samsung

Proposal 7: Support both semi-static and dynamic indication for NCR access beam, based on:

-	RRC/OAM only;

-	New MAC-CE with those in Rel-17 eIAB as starting point;

-	New DCI format, with or without HARQ-ACK information feedback.

### R1-2209916	Other aspects including control plane signalling and procedures relevant to RAN1	NTT DOCOMO, INC.

Proposal 1: The necessary configuration for receiving L1/L2 side control information is from RRC.

### R1-2210036	Discussion on signalling of side control information for network-controlled repeaters	Lenovo

Proposal 3: Consider ways to transmit legacy RRC and new RRC signaling to the repeater.

  - About initial access procedures

### R1-2208692	Discussion on other aspects for NCR	ZTE

Proposal 1: Initial access procedure without dedicated configured RACH resource is supported for NCR-MT.

### R1-2208990	Discussion on  control plane signalling and procedures for NR network-controlled repeaters	CATT

Proposal 13: The Identification information of the donor gNB and NCR is necessary and can be indicated by new signalling of L1/L2/L3.

### R1-2209351	Discussion on other aspects including control plane signalling and procedures relevant to RAN1	CMCC

Proposa1 2:It is proposed to discussed the RNTI issue of NCR considering the two options below.

•	Option 1: Define a Network-controlled repeater’s RNTI.

•	Option 2: reuse the C-RNTI for MT part of NCR.

### R1-2209745	On signaling and procedures for network-controlled repeaters	Samsung

Proposal 3: Support random access procedure for NCR-MT that can be based on the cell-specific system information, without the need for additional configuration signalling.

  - About PUSCH

### R1-2208692	Discussion on other aspects for NCR	ZTE

Proposal 3: NCR capability report can be carried by PUSCH as part of the C-link setup procedure.

Proposal 7: The legacy configuration of PUCCH and PUSCH can be reused for NCR-MT.

### R1-2209351	Discussion on other aspects including control plane signalling and procedures relevant to RAN1	CMCC

Proposal 4: PUCCH, PUSCH, UCI and MAC CE and the related configurations should be supported for the side control information.

### R1-2209745	On signaling and procedures for network-controlled repeaters	Samsung

Proposal 1: Support configuration of UL channels including PUSCH and PUCCH, as well as UL control information UCI and UL MAC CE, as necessary information for the NCR-MT.

### R1-2209594	On control plane signaling and procedures for NCR	Apple

Proposal 9: For NCR, UCI transmission via PUCCH between NCR and gNB should be considered

•	PUSCH is not necessary, if CSI feedback is not supported

  - About PUCCH

### R1-2208692	Discussion on other aspects for NCR	ZTE

Proposal 6: HARQ ACK feedback via PUCCH is supported for NCR-MT in response to side control information.

Proposal 7: The legacy configuration of PUCCH and PUSCH can be reused for NCR-MT.

### R1-2209019	Discussion on control plane signaling and procedures relavant to RAN1	Fujitsu

Observation 2: For PHY signal/channels, NCR-MT will at least support SSB, PDCCH, PDSCH, PRACH, PUSCH, PUCCH.

Proposal 2: NCR-MT supports PUCCH carrying HARQ-ACK for PDCCH/PDSCH with side control information.

### R1-2209066	Discussions on L1/L2 signaling and configuration for side control information	Intel Corporation

Proposal 10: Support HARQ-ACK feedback for PDSCH and PDCCH carrying side control information. Support single bit HARQ-ACK feedback in a PUCCH.

### R1-2209351	Discussion on other aspects including control plane signalling and procedures relevant to RAN1	CMCC

Proposal 4: PUCCH, PUSCH, UCI and MAC CE and the related configurations should be supported for the side control information.

### R1-2209594	On control plane signaling and procedures for NCR	Apple

Proposal 6: For NCR, at least HARQ-ACK feedback corresponding to downlink control information via PDCCH and/or PDSCH should be supported for UCI from NCR to gNB

Proposal 7: For NCR, supporting CSI feedback and SR for UCI should be a low priority

Proposal 9: For NCR, UCI transmission via PUCCH between NCR and gNB should be considered

•	PUSCH is not necessary, if CSI feedback is not supported

### R1-2209745	On signaling and procedures for network-controlled repeaters	Samsung

Proposal 1: Support configuration of UL channels including PUSCH and PUCCH, as well as UL control information UCI and UL MAC CE, as necessary information for the NCR-MT.

  - About PDCCH

### R1-2208692	Discussion on other aspects for NCR	ZTE

Proposal 8: New DCI format for NCR can be considered to support dynamic signaling for access beam indication.

Proposal 9: At least the beam indication with one beam index per indication should be supported.

Proposal 10: The beam pattern can be semi-statically configured/indicated by RRC signalling and dynamically indicated by beam pattern index via DCI.

Proposal 15: Explicit ON-OFF indication is supported with following signaling design:

	For the NCR-MT in RRC CONNECTED state

	Semi-static indication through RRC

	Dynamic indication through DCI

	For the NCR-MT in RRC IDLE/INACTIVE state if supported

	Semi-static indication through RRC

### R1-2208990	Discussion on control plane signalling and procedures for NR network-controlled repeaters	CATT

Proposal 1: The semi-static beam indication can be configured by RRC/MAC-CE and the dynamic beam indication can be transmitted in L1.

Proposal 2: The semi-static configuration by the RRC/MAC-CE can be at least symbol-level. The dynamic signalling for beam side information can indicate M segments within a indication duration, and each segment can have separate length and beam ID indication

### R1-2209066	Discussions on L1/L2 signaling and configuration for side control information	Intel Corporation

Proposal 1: Support new DCI format for side control information for NCR.

Proposal 2: Support single side control information type or multiple side control information types in a PDCCH per gNB configuration.

Proposal 3: Support mechanism to differentiate different side control information type by a PDCCH, if an NCR is configured with multiple side control information types and single side control information per PDCCH.

Proposal 4: Study whether same or smaller DCI size budget, maximum PDCCH monitoring candidates and maximum number of non-overlapped CCE as legacy UE is supported by NCR. FFS DCI size alignment.

Proposal 5: Study whether joint configuration and indication for time domain resources and beam indices or separate configuration and separate indication for time domain resources and beam indices by different bit field in a PDCCH.

Proposal 6: Study the reference time instant for starting symbol/slot for time domain resource indication, e.g., the reference time instant can be a configured SFN or SFN 0, or the slot with side control information PDCCH reception, or the slot with HARQ-ACK feedback of the side control information, or the earliest slot after the application time.

Proposal 7: Study whether on/off state is always assumed as ‘on’ for a set of time domain resources indicated by on/off side control information carried by PDCCH.

Proposal 8: Support on/off indication for multiple carrier groups within a PDCCH. Separate f time domain resources for different carrier groups can be indicated by the PDCCH.

Proposal 9: Support semi-static indication for side control information by MAC-CE or RRC signaling by a PDSCH. FFS details for each side control information by MAC-CE and/or RRC signaling.

### R1-2209087	"Discussion on	other aspects including control plane signalling and procedures relevant to RAN1"	Nokia, Nokia Shanghai Bell

Proposal 2.1:	Dynamic indication of beam configuration can be indicated jointly with dynamic indication of on-off control.

Proposal 2.2:	Dynamic indication of beam configuration and on-off control is an NCR specific signal.

Proposal 2.3:	Consider a DCI format similar to that of DCI format 2_0 for indication of access link beam index and on-off configuration.

### R1-2209409	Discussion on NCR signaling and procedures	ETRI

Proposal 1. RAN1 to support both dynamic and semi-static beam indication for NCR.

A time instance, which is not associated with specific beam(s), is considered as OFF duration.

Proposal 2. For beam and ON-OFF indication purposes, RAN1 to support SCI with the following fields:

Beam index indication field for ≥1 beam(s)

Slot index indication field for the allocated beams

Symbol index indication field (SLIV) for the allocated beams

Frequency resource indication field (e.g., applicable band or CC list) for the allocated beams

Panel and/or polarization indication field for the allocated beams

### R1-2209594	On control plane signaling and procedures for NCR	Apple

Proposal 1: For L1-based dynamic signaling of downlink side control information for NCR, DCI/PDCCH framework should be supported

Proposal 2: For carrying L1 dynamic signaling for downlink side control information for NCR, new DCI format should be considered

### R1-2209745	On signaling and procedures for network-controlled repeaters	Samsung

Proposal 7: Support both semi-static and dynamic indication for NCR access beam, based on:

-	RRC/OAM only;

-	New MAC-CE with those in Rel-17 eIAB as starting point;

-	New DCI format, with or without HARQ-ACK information feedback.

### R1-2209999	On other NCR aspects	Qualcomm Incorporated

Proposal 1 Support a unified signaling framework for indication of various side control information, including access and backhaul link beamforming information, explicit ON-OFF indication (if agreed), and DL/UL state indication (if agreed).

Proposal 3 Support both dynamic beam indication and semi-static beam indication for side control information.

-	Support L1 (DCI-based) signaling for dynamic indication of side control info.

-	Support L2/L3 (MAC-CE or RRC based) signaling for semi-static indication of side control info.

### R1-2210232	Signaling, procedures and other aspects in NCR	Ericsson

Proposal 6 Support multiple semi-static beam configurations for the access link where each configuration is configured with independent starting time, duration, periodicity, and priority.

FFS: resolution and range of above time parameters.

Proposal 7 A new DCI format is specified for access link beam indication which contains a set of beam pattern indications, one for each subband in a set of subbands.

FFS: Size of an indication and number of indications in the set.

Proposal 8 A DCI format for dynamic beam indication follows the DCI format 2_0/2_5 framework with each DCI index field providing an index to an RRC configured table of beam indication patterns over consecutive time intervals.

Proposal 9 Provide by higher layers a pre-configured time domain resource table with variable resource durations.

Proposal 10 Provide by higher layers a pre-configured beam pattern table, including the mapping between the beam pattern and the preconfigured time domain resources for a specific beam pattern ID.

  - About beam-relevant functionalities

### R1-2209745	On signaling and procedures for network-controlled repeaters	Samsung

Proposal 2: Support configuration of CSI-RS and SRS as necessary information for the NCR-MT.

### R1-2208685	L1/L2 signaling for side control information of network-controlled repeaters	InterDigital, Inc.

Proposal 3: Study efficient BFR procedures for NCR enabled networks, possibly considering the specific characteristics of these networks.

### R1-2209019	Discussion on control plane signaling and procedures relavant to RAN1	Fujitsu

Proposal 3: NCR-MT supports TA adjustment, BFD/BFR and RLM.

  - About feature and capabilities

### R1-2208684	Discussions on side control information for NR network-controlled repeaters	InterDigital, Inc.

Proposal 5. Consider below NCR capabilities to be reported for NCR-FWD access link:

-	Total number of supported beams and respective beamwidth (beam type)

-	Number of beams that can operate simultaneously

-	Beam associations/arrangement at NCR-FWD access link

### R1-2208691	Discussion on side control information for NCR	ZTE

Proposal 4: The Rel-15 mandatory features for UE may be reduced for NCR-MT.

Proposal 8: A time offset should be considered for the application of indicated beam based on NCR capability.

R1-2209066	Discussions on L1/L2 signaling and configuration for side control information	Intel Corporation
Proposal 12: Support NCR capability report at least for beam relevant information including number of beams, beam width and spatial relation information, passband information and whether capable of simultaneous UL transmission in C-link and backhaul-link.

### R1-2209154	Discussion on side control information to enable NR network-controlled repeaters	NEC

Proposal 7: It’s necessary to report a group capabilities related to access beam of NCR-Fwd to gNB, which includes beam width, maximum number of beam per beam width, number of antennas, and a mapping relationship between logic indexes and all the beams.

### R1-2209999	On other NCR aspects	Qualcomm Incorporated

Proposal 5 NCR-MT’s capability report follows the same framework as legacy UEs (i.e., via RRC).

Proposal 6 Support indication of NCR-Fwd’s specific information to the network, including

Beam-relate information (e.g., the number of supported TX/RX beams)

Power-related information (e.g., NCR-Fwd’s max power, max amplification gain, or power headroom)

Frequency-related information (e.g., supported passbands, and whether NCR supports selective forwarding across the passbands)

Multiplexing capability (i.e., whether it support simultaneous UL transmissions on the control-link by NCR-MT and backhaul link by NCR-Fwd)

FFS: signaling method (e.g., new L1/L2 signaling, or L3 signaling with a possibility of joint report with repeater’s other features/capabilities associated with NCR-MT)

### R1-2210036	Discussion on signalling of side control information for network-controlled repeaters	Lenovo

Proposal 1: For down-selection of the repeater management solutions, identify the NCR specific radio capability for both NCR-MT and NCR-Fwd, and the ways of reporting the capability to the network.

Proposal 2: Support reporting the RF characteristics of NCR-Fwd including beam characteristics of the access link to gNB during the access procedure of the NCR-MT

### R1-2209238	Discussion on side control information and NCR behavior	CAICT

Proposal 2: NCR capability should be defined regarding the NCR’s beam for access link, at least including number of supported beams and beam width.

### R1-2209297	Discussion on side control information and NCR behavior	xiaomi

Proposal 3: NCR beamforming capability report should be supported.

### R1-2210232	Signaling, procedures and other aspects in NCR	Ericsson

Proposal 2 Introduce capability signaling for support of subband operation in up to [8] subbands.

Proposal 3 Support subband based dynamic beam indication.

Proposal 4 Support beam arrangement reporting, including

a.	beam array size, and based on capability,

b.	beam expansion, to construct wide beams from narrow beams.

Proposal 13 The repeater-MT can indicate through capability signaling if it is supporting dynamic TDD UL/DL indication.