# 3GPP TSG RAN WG1 #110bis-e	                             	 R1-2210479

# e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.13.2

Source:	Moderator (Apple Inc.)

Title:	Summary #1 on [110bis-e-R18-LP_WUS-02] LP WUR architecture

Document for:	Discussion/Decision

# Introduction

In Rel-18, a study item was approved for low-power wake-up signal and receiver for NR (WID in RP-222644 [1]), and it includes the following objectives.

- Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4] - Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1] - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

This contribution summarizes the discussions on low-power wake-up receiver (LP WUR) architectures in RAN1#110bis-e.

[110bis-e-R18-LP_WUS-02] Email discussion on LP WUS receiver architecture by October 19 – Sigen (Apple)

- To be kicked off after first GTW session

- Check points: October 14, October 19

Section 2 provides a summary of the outcome. Section 3 documents the detailed discussions. Companies’ proposals from the contributions are captured in the Appendix.

# Outcome

# Proposals for GTW

## Proposals for 11/14 GTW

#### Proposal conclusion 1-1r2:

RAN1 does not intend to mandate the implementation of any specific type(s) of LP WUR architecture at the UE.

- Note: this does not prevent RAN4 from defining requirements for LP WUR in the normative phase.

#### Proposal 2-1r3:

For evaluation purpose, Further study at least the following three types of receiver RF architectures at least for LP-WUR based on OOK modulation:

- RF envelope detection architecture

- Heterodyne architecture with IF envelope detection

- Homodyne/zero-IF architecture with baseband envelope detection

- Note: the details of each type of receiver architecture are discussed separately.

#### Proposal 2-2r3: (RF envelope detection)

For OOK modulation, For evaluation purpose, study the RF envelope detection architecture can be studied based on at least the following diagram for LP-WUR based on OOK modulation.

- The RF signal is converted into baseband signal directly via an RF envelope detector.

- There is no Local Oscillator (LO) and no Phase-Locked Loop (PLL).

- 1-bit or multi-bit ADC is applied in baseband processing.

- RF LNA [and/or BB AMP] can be optionally applied [to improve sensitivity, with the cost of additional power consumption].

- High-Q matching network and/or RF BPF and/or BB LPF can be used to suppress The interference rejection for adjacent channel interference or interference from legacy NR signals on adjacent subcarriers can be potentially done using a high-Q matching network and RF BPF, which may require off-chip components and is band-specific.

#### Proposal 2-3r2: (Heterodyne)

For OOK modulation, For evaluation purpose, study the heterodyne architecture with IF envelope detection can be studied based on at least the following diagram for LP-WUR based on OOK modulation.

- The RF signal is down converted into IF signal via an RF mixer with a LO. After the amplification and filtering of IF signal, the signal is converted into baseband signal via an IF envelope detection.

  - There may be one or multiple IF stages depending on design.

- The choice of the LO is one of the major factors that determines the power consumption.

  - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, the corresponding frequency offset and increased phase noise should be taken into account in the design and evaluation.

  - One example of low power oscillators is the ring oscillator.

  - FLL (frequency locked loop) may replace PLL for non-coherent detection.

- 1-bit or multi-bit ADC is applied in baseband processing.

- [The IF should be low enough so that the IF components can be integrated on-chip, yet still high enough to avoid DC offset and flicker noise.]

- The interference rejection can be performed by a high-Q RF BPF and/or IF BPF and/or BB LPF.

- Multi-bit ADC can facilitate reducing interference in baseband processing.

- RF LNA and/or BB AMP can be optionally applied to further improve sensitivity, with the cost of additional power consumption.

- Image rejection filter is required.

- [Band or channel tuning can be performed by tuning the RF-LO frequency.]

#### Proposal 2-4r3: (zero-IF)

For OOK modulation, For evaluation purpose, study the homodyne/zero-IF architecture with baseband envelope detection can be studied based on at least the following diagram for LP-WUR based on OOK modulation.

- The RF signal is directly down converted into baseband signal via an RF mixer with a LO.

- The choice of the LO is one of the major factors that determines the power consumption.

  - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, such frequency offset and increased phase noise should be taken into account in the design and evaluation.

  - FLL (frequency locked loop) may replace PLL for non-coherent detection.

- 1-bit or multi-bit ADC is applied in baseband processing.

- The interference rejection can be performed by a high-Q RF BPF and/or BB BPF.

- No image rejection filter is required.

- RF LNA and/or BB AMP can be optionally applied [to further improve sensitivity, with the cost of additional power consumption].

#### Proposal 2-6r1: (FSK)

Further study the receiver architectures for FSK, with the two examples shown below:

- Example 1: parallel OOK receivers and a comparator circuit

- Example 2: using FM-to-AM detector

  - Alternative 1: a similar architecture as for OOK, except that the envelope detector is replaced by a FM-to-AM detector.

  - Alternative 2: the FM-to-AM detector can implemented in digital domain after ADC.

  - The FM-AM detector, using a frequency discriminator circuit, converts frequency shift into amplitude change, which can be implemented in either analog or digital domain. One example, as shown in the figure below, is a conventional quadrature FM discriminator. It multiplies received frequency modulated signal with a phase shifted version, followed by a low pass filter. The amplitude of the output signal is proportional to the frequency of the input signal.

- Note: Other architectures are not precluded.

#### Proposal 2-8r2:

For the analysis of a receiver architecture, companies are encouraged to provide at least the following (when applicable):

- Details of receiver components

  - Receiver architecture type

  - Presence of a RF LNA / IF AMP / BB AMP, and the corresponding gain, if any

  - Local oscillator

    - Type of oscillator and the corresponding frequency accuracy/drifting

  - Presence of PLL or FLL

  - ADC: sampling rate, bit-width

  - Assumed signal bandwidth and guard band

  - RF/IF/BB filter characteristics (e.g. type of filter, order), if any

  - Baseband processing (e.g., sequence correlation detection / decoding, other signal processing, if any)

  - Assumed frequency band(s)

- Performance metrics

  - Power consumption during active monitoring/reception and during off state (and breakdown if possible available)

  - Noise figure and/or receiver sensitivity

  - Interference rejection capability

  - Data rate

#### Proposal 1-2r3:

For the study of LP WUR architecture, at least the following should be considered:

- Power consumption during active monitoring/reception

- Noise figure

- Sensitivity/coverage

- Supported modulation/waveform

- Supported data rate

- Bandwidth and frequency location within a carrier

- Supported band(s)

- [Inter-cell interference]

- Interference rejection (including both adjacent-channel interference and interference from adjacent subcarriers occupied by legacy NR signals or other LP WUS)

- Mobility support

- Time/frequency impairments

- [Duty cycle handling of WUS and other signals (if any)]

- Support for sequence correlation detection/decoding

# LP WUR architectures

## General

### First round

Two companies (CATT, QC) explicitly stated that the exact receiver architecture should not be mandated or specified, and it should be left to UE implementation. This should be common understanding. The study of LP WUR architecture is to understand the feasibility and the trade-off between power consumption and performance.

#### Proposal 1-1:

3GPP shall not mandate the implementation of any specific type(s) of LP WUR architecture at the UE.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | From our understanding, the output of AI 9.13.2 is to provide necessary support for power consumption evaluation of AI 9.13.1 and LP-WUS design of AI 9.13.3, and related implementation should be captured in the TR. It does not mean the UE should be mandated to implement the architecture in the TR. However, some performance metrics, e.g., sensitivity, power consumption, should be guaranteed. |
| Spreadtrum | We are fine with proposal. |
| TCL | In our view, the common understanding of companies about the general architecture of LP-WUR will allow the companies to define those methods/procedures which is necessary to standardize, such as the LP-WUS monitoring procedure, the LP-WUR and MR synchronization procedure, and the power consumption of LP-WUR. |
| Nordic | Statement is true, but 3GPP will define requirements having some types of architecture in mind |
| Panasonic | We support this proposal. |
| vivo | We agree that the implementation of LP WUR architecture should not be mandated. In this study, we investigate the feasibility of the potential receiver architectures to figure out the proper architecture type satisfying the design targets in terms of power consumption, sensitivity, latency, and data rates as required by the use cases in the SI. The study outcome of proper architecture type can provide guidance on future implementation. |
| Intel | OK |
| Futurewei | We agree with the proposal, the purpose of the study should be the agreement on general architecture(s) with nominal values for power consumption, sensitivity, and supported modulation & data rates without any specific implementations for the evaluation. |
| Nokia, NSB | We agree with the proposal and like ZTE, we see a key output of this agenda item, are improved assumptions for the power evaluation model developed in AI 9.13.1 and also RAN4 evaluations, that take into consideration different architecture features/characteristics given NR main radio constraints. Also, in the assumed work item phase, RAN4 should determine requirements for the performance parameters, e.g. sensitivity and selectivity. Thus assumptions/values considered in SI phase should be realistic for different LP-WUR architectures so that LP-WUS design is feasible. |
| CATT | We support this proposal. We also agree that the minimum requirements of LW-WUR in term of power consumption, coverage, and minimum supporting data rate should be defined. |
| OPPO | We agree with this proposal. The design targets on power consumption, sensitivity and minimum data rate should be determined first. The LP WUR architecture is up to implementation. |
| InterDigital | Fine with the proposal. |
| MediaTek | We support this proposal. |
| Sharp | We are fine with the proposal. |
| QC | Support. The goal of UE architecture study is to investigate the feasibility of different architecture options and identify whether they can meet 3GPP LP-WUR design target. |
| Samsung | Agree with the proposal. The identified LP-WUR architecture is mainly for the evaluation purpose, i.e., to see that the reception performance of WUS and power consumption of the WUR can be guaranteed. So, the identified LP-WUR architecture should not be a mandatory UE architecture to be implemented. |
| Moderator | It seems that companies are generally fine with the proposal. To address some comments related to RAN4, I added a sub-bullet in the updated proposal below. #### Proposal 1-1r1: 3GPP shall not mandate the implementation of any specific type(s) of LP WUR architecture at the UE. - Note: this does not prevent RAN4 from defining requirements for LP WUR in the normative phase. In terms of expectation on the outcome of this AI, it is good to see comments from companies. I think companies’ views are aligned on the high level. I do not see a need to capture anything. |
| Everactive | Agree |
| Huawei, Hisilicon | In principle, we are fine with the proposal, on the understanding that in fact this is an SI so it automatically cannot ‘mandate’ anything. Nonetheless, for evaluation purposes we need to take into consideration the possible LP-WUR architectures and how they impact the selection of parameters in AI 9.13.1 and the selection of designs in AI 9.13.3. Or equivalently, for evaluation purposes we need to take into consideration the impact that the AI 9.13.1 chosen parameters and AI 9.13.3 chosen designs on the possible LP-WUR architectures. Hence. the TR is going to capture various Rx architectures and descriptions, etc… |
| vivo 2 | We support the proposal. |
| SONY | Agree with the proposal but as Nordic has also pointed out, 3GPP will define requirements which allow certain types of architecture. |
| Ericsson1 | OK in principle. Suggest updating “3GPP shall not mandate…” to “3GPP RAN1 shall not mandate…” |
| Moderator | Another attempt to address the comments: #### Proposal conclusion 1-1r2: RAN1 does not intend to 3GPP shall not mandate the implementation of any specific type(s) of LP WUR architecture at the UE. - Note: this does not prevent RAN4 from defining requirements for LP WUR in the normative phase. |
| ZTE, Sanechips | OK with this. |
| CMCC | OK with the Moderator2 proposal. The exact receiver architecture can be left for UE manufactures to implement according to the specified performance, and no need to be mandated in 3GPP as long as it can meet the specified performance requirements. |
| Nordic | OK |
| Futurewei 2 | We are fine with Proposal 1-1r2. |
| vivo | support |
| Nokia, NSB2 | OK. |
| Panasonic | Good with us. |
| Intel | OK |
| SONY | OK |
| MediaTek | Agree. The intention is to support evaluations and functionalities rather than mandating implementation. |
| Huawei, Hisilicon | This still does not have an evident purpose in a feasibility study item in a WG whose terms of reference would not allow such a mandate in any case. If it is like a pre-disclaimer that any later agreement cannot be interpreted as such a mandate, that is something we are OK with. |
| Everactive | OK |
| Ericsson2 | OK |
| QC | OK |
| Moderator | The plan is to take this to 10/14 GTW for agreement. @Huawei/HiSi, as I mentioned at the beginning, I believe all the companies are on the same page regarding the principle. This is just to set the stage properly, and avoid the topic popping up later on when we discuss the architectures. |

There have been various proposals on what factors should be taken into account in the LP WUR architecture discussion. The following proposal intends to capture the more critical factors but not necessarily every single factor.

#### Proposal 1-2:

For the study of LP WUR architecture, at least the following should be considered:

- Power consumption

- Noise figure and/or sensitivity and/or coverage

- Modulation

- Data rate

- Supported band(s)

- Interference rejection (including both adjacent-channel interference and interference from adjacent subcarriers occupied by legacy NR signals or other LP WUS)

- Mobility support

- Time/frequency synchronization

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | We are generally OK with the proposal. Besides, the following also should be captured: - Sequence correlation detection The number of supported sequences may have the impacts on the receiver memory size and receiver architecture due to performance compensation brought by CDM sequences. Additionally, besides modulation, waveform also needs to be considered since it may has an impact on filter design, ED design, and so on. Therefore, the following is suggested - Power consumption - Noise figure and/or sensitivity and/or coverage - Modulation/waveform - Data rate - Supported band(s) - Interference rejection (including both adjacent-channel interference and interference from adjacent subcarriers occupied by legacy NR signals or other LP WUS) - Mobility support - Time/frequency synchronization - Sequence correlation detection |
| Spreadtrum | We are fine with this proposal. |
| TCL | For this proposal we prefer the ZTE revised proposal. |
| Nordic | We would like to also consider-study also “LP-WUS position within carrier/band” and its impact on interference rejection. |
| Panasonic | We are okay and suggest to add inter-cell interference in the bullet of interference rejection, or in the second bullet, to reflect the sensitivity. |
| Vivo | - Interference rejection: it’s important to evaluate the interference rejection of the receiver architecture based on non-coherent detection where sophisticated digital processing is not available, and thus, interference rejection mainly relies on analog filters in RF/IF/BB, which should be considered in architecture design. And we have one question for clarification on adjacent-channel interference here, is it supposed to be discussed in RAN 1 or RAN 4? - Time/frequency synchronization Time/frequency error comes from frequency error of the clock and/or local oscillator. We prefer to use impairments, i.e., frequency error of clock/ local oscillator instead. - Power consumption in active state The power consumption of a particular receiver architecture should be the peak power when the receiver is actively receiving. Thus, we suggest the following modifications below: For the study of LP WUR architecture, at least the following should be considered: - Power consumption during active WUS monitoring - Noise figure and/or sensitivity and/or coverage - Modulation - Data rate - Supported band(s) - Interference rejection (including both adjacent-channel interference and interference from adjacent subcarriers occupied by legacy NR signals or other LP WUS) - Mobility support - Time/frequency synchronization impairments, i.e., frequency error of clock/ local oscillator |
| Intel | Bandwidth of LP-WUR should be included. Other consideration points include easily integrated with main radio, potentially sharing some component with main radio, e.g., antenna and matching network, etc. |
| Futurewei | We are OK with the proposal from Vivo with the additional suggestion of changing “Modulation” and “Data rate” to “Supported modulation” and “Supported data rate”. |
| Nokia, NSB | In general we support the proposal, with an additional bullet to cover the support of “Flexible frequency allocation/placement of the LP-WUS within a band.” We also with agree the “/waveform” edit suggested by ZTE. Unclear, if the ZTE suggested “sequence correlation detection” is also needed, given other bullets, covering sensitivity and interference with other LP-WUS. |
| CATT | We are OK with the proposal. |
| OPPO | We are fine with the proposal. |
| InterDigital | We are fine in principle with the following update. - We support to add “/waveform” as proposed by ZTE - Fine with the update from vivo, but prefer to change impairments time/frequency impairments |
| MediaTek | Some LP-WUR prototypes don’t support Time/frequency (T/F) synchronization to achieve ultra-low power consumption. The benefit to support T/F tracking is to enable duty cycle and improve receiver sensitivity and interference resilience. With this regard, we have the following text proposal. Time/frequency synchronization Duty cycle support (including Time/frequency tracking) |
| Lenovo | We are fine Please add Tx Waveform (eg., OFDM, DFT) Co-channel interference? Duty cycle of LP-WUS |
| Sharp | We are fine with the proposal. |
| QC | We suggest the following addition/modification - … - Noise figure - Sensitivity/coverage - Modulation/waveform - Time/freq synchronization and clock accuracy … |
| Samsung | We are fine with the proposal. |
| Moderator | Thanks for the good comments. The updated proposal below attempts to capture most of the comments above: #### Proposal 1-2r1: For the study of LP WUR architecture, at least the following should be considered: - Power consumption during active monitoring/reception - Noise figure and/or sensitivity and/or coverage - Sensitivity/coverage - [Impact of inter-cell interference may be considered.] - Modulation/waveform - Data rate - Bandwidth and frequency location within a carrier - Supported band(s) - Interference rejection (including both adjacent-channel interference and interference from adjacent subcarriers occupied by legacy NR signals or other LP WUS) - Mobility support - Time/frequency synchronization impairments - Duty cycle of WUS @ZTE, on “sequence correlation detection”, it seems to get down to the WUS design detail. Even though I agree this is something to be considered, but not sure if it is on the same high level as the other bullets. I would like to hear opinions from other companies on this issue. @Nordic, I think what you proposed can be included as part of “interference rejection”. This list is intended to be somewhat high level. Wonder if you may be fine with the current version. @Panasonic, I added inter-cell interference under sensitivity/coverage. I personally think this may not be necessary to be included here. In any case, the impact can be evaluated in link-level sims. Let us see what companies think. @vivo, thanks for raising a good question on whether ACI should be discussed in RAN1 or RAN4. Traditionally this was done in RAN4. One possible way is that when we send LS to RAN4, we request RAN4 to study ACI issue. Would like to hear more views. @Futurewei, I am not sure about the intention of adding “supported”. I don’t really see the difference. @Nokia/NSB, I added “Bandwidth and frequency location within a carrier”. Hopefully this addresses your concern. Now we have the sub-bullet for “time/frequency impairments”, and hopefully this can address the concern from a few companies. Note that this list only intends to remind companies about some important factors to consider, without going into details of each aspect. |
| Panasonic | Thanks for adding the inter-cell interference under sensitivity/coverage. We understand that conventionally we only evaluate by using noise limited condition in LLS. However, in this context of LP-WUS/WUR discussion, the OOK performance can be sensitive to symbol by symbol interference fluctuation, which can be caused by inter-cell interference beside the scope off interference rejection bullet. So we would like to see companies’ view on this aspect on whether/how to handle this. |
| Everactive | Agree with the initial list. Just to clarify, “Time/frequency synchronization” refers to synchronizing and aligning to the LP-WUS Propose adding: - Frequency accuracy of the RF LO - Frequency accuracy of the duty-cycling oscillator (leads to uncertainty in LP-WUR turn-on time) |
| Huawei, Hisilicon | We propose to clarify the proposal because it is not clear if all items in the suggested consideration list is related particularly to the Rx architecture rather than signal and/or procedure design. It is also not very clear what it means to “consider” these points. It appears the intention is to have them reported by proponents and (if agreed) capture them in the TR. We propose the following modification to the proposal: For the study of LP WUR architecture, at least the following features/capabilities should be reported per receiver architecture candidate (or class of candidates): - Power consumption during active monitoring/reception - Noise figure and/or sensitivity and/or coverage - Sensitivity/coverage - [Impact of inter-cell interference may be considered.] - Modulation/waveform - Data rate - Bandwidth and frequency location within a carrier - Supported band(s) - Interference rejection (including both adjacent-channel interference and interference from adjacent subcarriers occupied by legacy NR signals or other LP WUS) - Mobility support - Time/frequency synchronization impairments - Duty cycle of WUS - Sequence correlation detection / decoding - Support of any other signal processing However, it seems that Proposal 2-8 has a similar intention but is a better place to start. |
| vivo2 | As sensitivity/coverage is not only impacted by inter-cell interference but also other factors, we prefer to take inter-cell interference as a separate sub-bullet. Further, we suggest to delete ‘Duty cycle of WUS’ as it is more related to WUS design, rather than receiver architecture. #### Proposal 1-2r1: For the study of LP WUR architecture, at least the following should be considered: - Power consumption during active monitoring/reception - Noise figure and/or sensitivity and/or coverage - Sensitivity/coverage - [Impact of inter-cell interference may be considered.] - inter-cell interference - Modulation/waveform - Data rate - Bandwidth and frequency location within a carrier - Supported band(s) - Interference rejection (including both adjacent-channel interference and interference from adjacent subcarriers occupied by legacy NR signals or other LP WUS) - Mobility support - Time/frequency synchronization impairments - Duty cycle of WUS |
| SONY | We are OK with the updated proposal. |
| Ericsson1 | We are in principle OK with Proposal 1-2r1: suggested by vivo2. Can add ‘Supported’ before ‘Modulation/waveform’ and ‘Data rate’ for clarity. Agree with comments that impact of inter-cell interference should be considered. |
| Moderator | Another update to address companies’ comments: #### Proposal 1-2r2: For the study of LP WUR architecture, at least the following should be considered: - Power consumption during active monitoring/reception - Noise figure and/or sensitivity and/or coverage - Sensitivity/coverage - [Impact of inter-cell interference may be considered.] - Supported Mmodulation/waveform - Supported Ddata rate - Bandwidth and frequency location within a carrier - Supported band(s) - Inter-cell interference - Interference rejection (including both adjacent-channel interference and interference from adjacent subcarriers occupied by legacy NR signals or other LP WUS) - Mobility support - Time/frequency synchronization impairments - [Duty cycle handling of WUS] @Panasonic, on inter-cell interference, I think we can discuss the details under AI 9.13.1 evaluation methodology/assumptions. @Everactive, “time/frequency impairments” was suggested, and it is intended to be a generic bullet to make sure it is considered in the design. @Huawei/HiSi, I agree there seems to be some overlapping between this one and P2-8. My intention is that this proposal is to have a high-level list of important factors that should be considered in the architecture design (i.e. factors that can impact the architecture itself). P2-8 intends to provide a guidance on what kind of details should be provided for any specific architecture being proposed/investigated so that the most critical information is available for other companies to understand it. With this explanation, I wonder if it is ok not to include the last two bullets for now. We could consider adding them into P2-8. On “duty cycle of WUS”, my interpretation/guess was that the architecture design needs to be able to handle the duty cycle monitoring of WUS (e.g. the uncertainty in LP-WUR turn-on time as suggested by Everactive). I keep it in bracket for now to seek for more comments. |
| ZTE, Sanechips | We are generally fine. Besides, Sequence correlation detection has impacts on correlator/comparator, memory size, bitwidth of ADC, and the number of sequences for sequence correlation detection also have impacts on the detection performance and further impact on architecture selection. Therefore, we think sequence correlation detection should be added for consideration. |
| CMCC | OK with the proposal. We suggest to update some points with further notes as below: - Power consumption during active monitoring/reception - both the power consumption of the “core parts” and the “assisted parts” shall be taken into consideration, including VCO/Oscillator, IF Amp, Envelop Detector, BB Amplifier, Digital Circuit, Bias, PMIC, Bandgap voltage reference, GPIO, etc - Supported band(s) - the number of supported band(s) shall be taken into consideration - Time/frequency synchronization impairments - the transient period between “main radio turned on” and “main radio turned off/deep sleep” shall be taken into consideration in the evaluation methodology For “Supported band(s)”, to indicate the number of supported band(s) shall be taken into consideration. LP-WUR is band specific. If more bands are needed to be supported by main radio, more simultaneously active RF paths may be needed in low-power WUR, which will increase the die area/complexity/cost and decrease the benefits of power saving proportionally.To balance among the die area/benefits of power saving/complexity/cost, etc., the less bands supported, the more benefits can be obtained from the low-power WUR solution. |
| Nordic | OK |
| Fuuurewei 2 | We are fine with proposal 1-2r2, but we think that indicating “ power consumption during active monitoring/reception” and “Time/frequency impairments” already capture the impacts on “Duty-cycle handling of WUS” by the LP-WUR, so we are fine with removing the duty-cycle bullet. |

| vivo | We suggest to delete ‘duty cycle of handling of WUS’, the time uncertainty in LP-WUR is already covered by time/frequency impairments |
| --- | --- |
| Nokia, NSB2 | We are generally fine with the proposal. We think the “Duty cycle handling of WUS” point is a useful consideration to add for the reasons stated by the FL.Everactive. We support some “sequence correlation” related point (as suggested by ZTE), since, we do not see that the current points, cover the architecture modifications that may be required to support more aggressive LP-WUS intra-cell/carrier multiplexing/interference handling, eg multiple sequences/randomisation. |
| Panasonic | Thanks. We are okay with this version. The details can be up to the report from the companies when presenting the evaluations and detailed designs, in either 9.13.1 or this AI. |
| Intel | In addition to the listed aspect, channel coding scheme can be considered too. For example, Manchester coding, repetition, or a sequence-based transmission of one or more bits. Another point for consideration is potentially sharing some components with main radio, e.g., antenna and matching network, etc., which helps to reduced complexity of adding LP-WUR |
| Samsung | Agree with the proposal in general. One question for the first sub-bullet and the last sub-bullet. If LP-WUS is monitored periodically, it has two state, one is active WUS state and the other is inactive WUS state, why do we only consider the power consumption during active monitoring/ reception? |
| SONY | We would be supportive of adding “support for sequence correlation / decoding”. Presumably, some architectures are better at supporting sequence correlation / decoding than other architectures, just as some LP-WUR architectures are better at supporting some modulations / waveforms than others. |
| MediaTek | We are okay with the proposal. All sub-bullets have certain architectures impacts. We suggest considering duty cycle because it requires sufficient timing synchronization and parsing assistant information from LP-WUS payload. For mobility support, however, we wonder how to determine whether a LP WUR architecture has mobility support. |
| Huawei, Hisilicon | Thanks for the response on overlapping of P2-8. It may help if we can see what the purpose in terms of the TR is. If we are just writing down some basic facts of Rx architectures, as opposed to things relating to low-power architectures, it may be relevant to focus on 2-8 or merge them. If this proposal is kept then the following points need to be deleted: - Bandwidth and frequency location in a carrier - Supported bands - Inter cell interference - Mobility support - Duty cycle of WU Because they belong to 9.13.3. They are design of the signal and procedures, not a study design of an architecture, and it’s important we don’t start to interleave the agenda items as it leads to confusing discussion in later meetings. |
| Everactive | Thanks for addressing our comment. If there’s support from others, we think linearity of the receiver should also be considered and added to the list. |
| Ericsson2 | Support including “Sequence correlation detection / decoding”. Also ok to include “Duty cycle handling of WUS”. |
| QC | #### Thanks for the update. Generally speaking, we are OK with the proposal. However, we suggest the following change: #### Remove bracket around Duty cycle handling. #### Proposal 1-2r2: For the study of LP WUR architecture, at least the following should be considered: - Power consumption during active monitoring/reception - Noise figure and/or sensitivity and/or coverage - Sensitivity/coverage - [Impact of inter-cell interference may be considered.] - Supported Mmodulation/waveform - Supported Ddata rate - Bandwidth and frequency location within a carrier - Supported band(s) - Inter-cell interference - Interference rejection (including both adjacent-channel interference and interference from adjacent subcarriers occupied by legacy NR signals or other LP WUS) - Mobility support - Time/frequency synchronization impairments due to channel and clock accuracy - Duty cycle handling of low power signals (e.g., WUS, sync, others) |
| Moderator | #### Updated proposal, with diff marks on top of r2. #### Proposal 1-2r3: For the study of LP WUR architecture, at least the following should be considered: - Power consumption during active monitoring/reception - Noise figure - Sensitivity/coverage - Supported modulation/waveform - Supported data rate - Bandwidth and frequency location within a carrier - Supported band(s) - [Inter-cell interference] - Interference rejection (including both adjacent-channel interference and interference from adjacent subcarriers occupied by legacy NR signals or other LP WUS) - Mobility support - Time/frequency impairments - [Duty cycle handling of WUS and other signals (if any)] - Support for sequence correlation detection/decoding @CMCC, I do agree with your comments in general, but this proposal does not intend to include all the details. The “Supported band(s)” at least from my perspective is intended to also cover the number of supported band(s). On duty cycle handling of WUS, a few companies want to keep it and a few companies think it is not necessary. Given that it may not be obvious for some people, I would suggest we simply list it and save the debating time (as it does not hurt anything). @Intel, I think sharing components with main radio is a plus, but we may not want to add to the list here because it may be secondary. Also this list is not intended to be exhaustive anyway, with “at least” in the main bullet. @Samsung, to address your comment, maybe the simpler way is to remove “during active monitoring/reception”, which is also aligned with the spirit of keeping the bullet on high level. @MediaTek, on mobility support, my thinking is that the architecture (and the signal design) may need to consider certain signal strength measurement capability. @Huawei, I think the points you crossed out all have architecture impact, except for inter-cell interference, which is not too clear to me. E.g. bandwidth/frequency location impact the filter design, supported bands, including the number of supported bands, as explained by CMCC, impact the components that are needed, and the corresponding power consumption. For mobility support, please see my response to MediaTek. For duty cycle, it has been discussed by many companies already. For inter-cell interference, it is not exactly clear to me how it may impact the architecture. The only thing I can think of is the impact on the bit-width for ADC and baseband processing. I put it in bracket now to seek for more comments. This is not to suggest that inter-cell interference is not to be considered. It is just about how it impacts the architecture. @Everactive, the reason for adding linearity is not clear to me, but let us see what other companies think. |

Given that RAN4 is also tasked to study LP WUS/WUR, it is important that RAN1 sends an LS to RAN4 in time on the candidate receiver architectures so that RAN4 can perform feasibility study. Since RAN4 study starts in 2023Q1, it would be good if RAN1 can send the LS by November 2022, as proposed by vivo.

#### [CLOSED] Proposal 1-3:

RAN1 targets to send an LS to RAN4 on the candidate receiver architectures by the end of Nov meeting.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | We are not sure whether we have enough time to converge on the architectures. But at least, one candidate receiver architecture based on OOK should be selected and sent to RAN4. |
| Spreadtrum | We are fine with this proposal. |
| TCL | We are fine with this proposal |
| Nordic | We can target, but no need waste RAN1 time on agreeing above. |
| Panasonic | We support the proposal and also we think the request of confirmation of the power consumption values and sensitivity is needed, as RAN4 has better expertise on defining such parameter, although RAN1 needs these in early phase study. |
| vivo | Proper receiver architectures for WUR are supposed to be figured out based on both RAN1’s and RAN 4’s feasibility study, and thus, it’s necessary to send RAN1’s initial study outcome on the candidate receiver architectures with necessary information to RAN4 before their work starts. It is also preferred to add other related background information such as use cases/design targets/… together to RAN4 to their better understanding. |
| Intel | Agree |
| Futurewei | We share the same view as Nordic. |
| Nokia, NSB | We are fine with the proposal, though in addition to candidate receiver architectures, it would be useful to include “relevant RAN1 LP-WUS related assumptions” |
| CATT | We don’t need to sign a blank check at this time. We will determine at next meeting when we have sufficient conclusion to share with RAN4. |
| OPPO | We are fine with the proposal. |
| InterDigital | We agree with Nordic and Futurewei. |
| MediaTek | We are okay, but no agreement is needed. |
| Lenovo | We are fine with the proposal |
| QC | It should depend on the RAN1 progress. We don’t think this agreement is necessary. |
| Samsung | The LS to be sent to RAN4 will include not only the candidate receiver architecture but also other aspects in our view. Basically, the contents of the LS will be dependent on what the purpose of the LS is (e.g., what RAN1 will ask for to RAN4). However, it is not clear what RAN1 will ask for to RAN4 at this stage. So, we'd better reformulate the proposal rather than limit it to "candidate receiver architecture". Therefore, we think the proposal can be revised as follows: RAN1 targets to send an LS to RAN4 so that RAN4 can start their study on the LP-WUS/WUR by the end of Nov. meeting. |
| Moderator | Quite some companies do not see the need for making such an agreement, which has a valid point. So this proposal is dropped. But it is important for companies to keep this in mind, and we should send an LS to RAN4 by the end of Nov meeting so that RAN4 can start their study based on RAN1’s investigation. |
| Huawei, Hisilicon | We were fine with this proposal, but no need to spend time agreeing it, since it’s already in the workplan and has to arise naturally by agreement at the appropriate meeting. We are fine with the drop. |
| Nokia, NSB2 | Support FL assessment – fine to drop. |

## LP WUR Architectures

### First round

The following modulation schemes have been discussed by companies, which directly impact the LP WUR architectures:

- OOK (or ASK), including MC-OOK

- FSK, including MC-FSK

- OFDM

It is generally acknowledged that in the traditional NR receiver (or the main radio), the most power-hungry components include:

- LC oscillator with PLL (phase locked loop)

- LNA (low-noise amplifier) with very low noise figure in order to achieve good sensitivity

- ADC (analog-to-digital converter), with high sampling rate and large bit-width

- FFT and complex baseband processing

The architectures under discussion for LP WUR target for either removing one or more of these components or replacing them with less power-hungry components while sacrificing performance. OOK and FSK have the advantage of supporting non-coherent detection, which does not require phase tracking (which is power consuming). Note that most discussions did not go into the details of digital baseband processing yet because it highly depends on the WUS design. Moreover, the power consumption of the digital baseband processing is expected to relatively simple and should not dominate the overall power consumption.

For OOK modulation, many companies discussed the following three types of receiver RF architecture on the high level, even though there can be different names being used and some differences in details.

- RF envelope detection architecture

- Heterodyne architecture

- Zero-IF architecture

Given the strong interest in these architectures as the candidates for LP WUR, the intention of the next few proposals is to provide a high-level diagram/description for each type of the architecture, and the corresponding. Advantages/disadvantages. Please provide comments on whether they should be studied, and how the diagrams/descriptions should be improved.

#### Proposal 2-1: (OOK)

Further study OOK modulation, based on at least the following three types of receiver RF architectures:

- RF envelope detection architecture

- Heterodyne architecture

- Zero-IF architecture

- Note: down-selection may be performed in the future.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | Yes, we agree with this proposal. |
| Spreadtrum | We basically agree with this proposal, but also have the following comment: There are also many specific implementation schemes under these three architectures. Therefore, we need to give a baseline scheme for each category receiver. |
| TCL | We support this proposal. |
| Nordic | Support, but some more details need to be agreed for each architecture. |
| Panasonic | We agree. |
| Vivo | Agree with some modification as follows, Further study OOK modulation, based on at least the following three types of receiver RF architectures: - RF envelope detection architecture - Heterodyne envelope detection architecture - Zero-IF envelope detection architecture - Note: down-selection may be performed in the future. For the note, although RAN1/4 may study the pros and cons for each feasible receiver architecture, down-selection may not be necessary to conclude the study given the understanding that 3GPP does not mandate any particular receiver implementation. |
| Intel | Just for understanding, what is the purpose for the down-selection in the note? According to proposal 1-1, RAN1 may only make observations on different architectures. |
| Futurewei | We agree with the updated proposal from vivo. |
| Nokia, NSB | We support the proposal. |
| CATT | We are OK with the proposal. However, we don’t need to down select the proposals during study item. |
| OPPO | We think the proposal2-1 is an implementation issue. Maybe we can make agreement on proposal1-1 first. If it is decided that some types of receiver architectures should be defined for LP WUR, we can discuss the down selection of the three types of receiver architectures. |
| InterDigital | Fine with the proposal. |
| MediaTek | We support this proposal. |
| Lenovo | We agree |
| Sharp | We support the proposal. |
| QC | We are fine to study various architecture options for OOK waveform. But, we do not agree on making down selection. It is not clear why down selection is required. What we need eventually is the requirements in terms of sensitivity, FA, MD, data rate, etc. rather than receiver architecture. The choice of receiver architecture will be left as implementation choice. |
| Samsung | Agree with the proposal, in general. However, it would be good to clarify what is a purpose of this down-selection. In addition, we need to have clear terminology to define the different architectures. More specifically, we are not sure whether 'RF envelope detection architecture' and 'Zero-IF architecture' are appropriate terminologies. For example, zero-IF architecture can be replaced by homodyne receiver in order to keep the consistency with heterodyne architecture. Also, we are not sure what 'RF envelope detection architecture' means. |
| Moderator | It seems that the note on down-selection is causing a lot of confusion. Down-selection is not the accurate wording. My intention is to say that we may prioritize or de-prioritize certain architecture(s) in the future during the investigation. But such a note may not be necessary at this stage. If we see the need to prioritize or de-prioritize, we will make a new agreement anyway. So the note is now removed. Some companies commented that more details are needed. It is not clear to me whether the next 3 proposals can adequately address the concerns already, or more details are expected. No change is made at this moment. If you have any suggestions on more accurate terminology to use for the 3 types of architecture, please do so. I did not want to combine this proposal with the next 3 proposals together. So we need some names to refer to them. #### Proposal 2-1r1: (OOK) Further study OOK modulation, based on at least the following three types of receiver RF architectures: - RF envelope detection architecture - Heterodyne architecture with IF envelope detection - Homodyne/zero-IF architecture with baseband envelope detection |
| Everactive | Agree with the revised proposal |
| Huawei, Hisilicon | It’s necessary to retain the agenda item arrangement, whereby signal design belongs to 9.13.3, and thus this agenda item should not discuss nor agree to modulations such as OOK (even though we are fine with that modulation included). Instead, 9.13.2 should say: Further study OOK modulation, At least the following architectures are included in the study: - RF envelope detection architecture - Heterodyne architecture with IF envelope detection - Homodyne/zero-IF architecture with baseband envelope detection Note: Down-selection may be performed in the future. |
| vivo2 | We are ok with the proposal |
| SONY | We support studying these LP-WUR architectures. It seems odd that in the LP-WUR architecture agenda item, we have a proposal to study a modulation. Shouldn’t we be studying the three architectures when used with OOK modulation? Can’t some of these receiver architectures be used with other modulations / waveforms? Maybe the proposal could be updated as: Further study at least the following three types of receiver RF architectures when LP-WUS is based on OOK modulation: - RF envelope detection architecture - Heterodyne architecture with IF envelope detection - Homodyne/zero-IF architecture with baseband envelope detection |
| Ericsson1 | We see a “Further study…” proposal for OOK but not for other types (e.g., FSK, OFDM). Our understanding is at this initial stage all receiver types should be further studied and not just OOK based WUR. That said we are OK to discuss different types for receivers for OOK and suggest following updates. #### Proposal 2-1r2: (OOK) For LP-WUR based on OOK modulation, consider based on at least the following three types of receiver RF architectures: - RF envelope detection architecture - Heterodyne architecture with IF envelope detection - Homodyne/zero-IF architecture with baseband envelope detection |
| Moderator | #### Proposal 2-1r2: (OOK) Further study OOK modulation, based on at least the following three types of receiver RF architectures for LP-WUR based on OOK modulation: - RF envelope detection architecture - Heterodyne architecture with IF envelope detection - Homodyne/zero-IF architecture with baseband envelope detection @Huawei/HiSi, as the architecture depends on the modulation/waveform, it seems meaningless to talk about an architecture without mentioning the intended modulation/waveform. This does not intend to replace AI 9.13.3 on the WUS design. But given the inter-dependency in nature, I don’t see a way to avoid the discussion on modulation/waveform under this AI. The main bullet is modified based on the comments from Sony and Ericsson, to avoid the misleading info that we are trying to decide on modulation. Instead we are studying the architecture. Hopefully this also somewhat addresses Huawei/HiSi’s concern. I prefer to keep “Further study” in the proposal, because it gives clear information we will study them. As I commented earlier, we may also choose to prioritize/de-prioritize certain architectures later if the group agrees. For other types of signals such as FSK and OFDM, I do not have any proposal yet because it is not clear how strong the interest is from the group. If there is sufficient interest, I will make similar proposals. |
| ZTE, Sanechips | OK with it. |
| Nordic | OK |
| Futurewei 2 | We are OK with proposal 2-1r2. |
| vivo | support |
| Nokia, NSB2 | OK |
| Panasonic | Although we are also okay with current version, it can be more inclusive at this moment to say “…at least based on OOK modulation”. |
| Intel | OK with the proposal |
| Samsung | If most companies decide to have a high-level proposal at this stage to list the candidate LR architecture only based on OOK modulation, we can compromise and agree with the updated proposal. Consider that it may be difficult to have a more accurate terminology, we can add a Note to further clarify, which can be shown as: Note: The detail of each architecture can be studied further. |
| SONY | Can we just drop the “based on OOK modulation”? We have the three bulleted LP-WUR architectures. As part of the study outcome, can’t we just say “LP-WUR architecture A is great but only works with OOK” or “LP-WUR architecture B has a bit more power consumption than A but works with several different modulations”. In other words, isn’t part of the goal of the study to determine which modulations can be supported with which LP-WUR architectures? |
| MediaTek | Support. Good way to differentiate these three types of architectures. |
| Huawei, Hisilicon | This agenda item is dealing with wake-up receiver architectures and it should be kept that way. Additionally, in all cases OOK modulation or any other modulation/wave will be considered. Based on that we suggest #### Proposal 2-1r2: (OOK) Further study OOK modulation, based on at least the following three types of receiver RF architectures for LP-WUR based on OOK modulation: - RF envelope detection architecture - Heterodyne architecture with IF envelope detection - Homodyne/zero-IF architecture with baseband envelope detection Note: These three types of receivers are suitable for OOK modulation. |
| Everactive | We agree with the proposal on OOK architectures. Regarding the comment: “For other types of signals such as FSK and OFDM, I do not have any proposal yet because it is not clear how strong the interest is from the group” We have a strong interest in studying FSK LP-WUR architectures. |
| Ericsson2 | We are OK with studying the three types of receiver RF architectures mentioned in the proposal for LP-WUR based on OOK modulation. However, we still prefer formulation which reflects that at this initial stage all receiver types (e.g., OOK, FSK, OFDM) are further studied and not just OOK. |
| QC | OK |
| Moderator | Updated version with diff marks on top of r2: #### Proposal 2-1r3: (OOK) For evaluation purpose, Further study at least the following three types of receiver RF architectures at least for LP-WUR based on OOK modulation: - RF envelope detection architecture - Heterodyne architecture with IF envelope detection - Homodyne/zero-IF architecture with baseband envelope detection - Note: the details of each type of receiver architecture are discussed separately. It seems that some companies think this proposal implies that we do not want to study other modulation, which is not the intention here. As you can see already, we have the proposals later on that for FSK modulation and OFDM-based receiver also. Please note that all these proposals are driven by contributions and comments provided by companies, and I can only progress based on the info I receive. My difficulty is that I personally do not see how an architecture can be discussed without mentioning the modulation scheme. In fact, an architecture design starts from the assumption on the modulation scheme that we intend to handle. It may turn out that two modulation schemes can use similar architecture, but that should not affect this proposal. E.g. later in the proposal for FSK, we mention that it can use a similar architecture as OOK, by replacing envelope detection with FM-to-AM detector. I added another “at least” in the main bullet as suggested by Panasonic. Even though the sentence now reads a bit awkward with two “at least” in it, hopefully this can address the concern from some companies. I also removed “RF” from the main bullet, as the diagrams later on are not just for RF only. It is just that there is no detail provided for baseband processing. |

#### Proposal 2-2: (RF envelope detection)

For OOK modulation, the RF envelope detection architecture can be studied based on the following diagram.

- The RF signal is converted into baseband signal directly via an RF envelope detector.

- It can achieve very low power consumption due to the removal of power-hungry local oscillator (LO) and PLL.

- LNA can be optionally applied to improve sensitivity, with the cost of additional power consumption.

- Generally speaking, it has poor receiver sensitivity.

- Generally speaking, it has poor interference rejection.

  - The interference rejection for adjacent channel interference or interference from legacy NR signals on adjacent subcarriers can be potentially done using a high-Q matching network and RF BPF, which has high power consumption/cost/area and is band-specific.

- It suffers from flicker 1/f noise and DC offset.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | We are generally fine with the structure for RF envelope detection. However, it is not appropriate to say , it has poor receiver sensitivity or it has poor interference rejection. The conclusion should be based on the comparison with other architectures and the sensitivity range or other metrics details can be listed and compared together, which would help for candidate architecture convergence in future. |
| Spreadtrum | We are fine with this proposal. |
| TCL | We are fine with this proposal, but share similar views with ZTE about the sensitivity and poor interference rejection sub-bullets. |
| Nordic | Even though we “generally speaking” agree with last 3 bullets, rather than drawing conclusion already, we should agree on properties of the components, e.g. that Matching network and RF BPF are 100MHz band or tuned to 20MHz carrier only, or …. |
| Panasonic | We are generally fine but also think it is good to remove/relocate the bullets of receiver sensitivity and interference rejection, as they should be relatively compared to other receiver architectures, which we also agree on the relatively poor performance. |
| Vivo | It seems that we can describe the functionality of each block in the diagram first and get common understanding on this receiver architecture before commenting on its pros and cons. So we suggest the following: For OOK modulation, the RF envelope detection architecture can be studied based on the following diagram. - The RF signal is converted into baseband signal directly via an RF envelope detector. No Local oscillator (LO) and PLL. - It can achieve very low power consumption due to the removal of power-hungry local oscillator (LO) and PLL. - The interference rejection for adjacent channel interference or interference from legacy NR signals on adjacent subcarriers can be potentially done using a high-Q matching network and RF BPF, which has high power consumption/cost/area and is band-specific. - LNA can be optionally applied to improve sensitivity, with the cost of additional power consumption. - 1bit or multi-bit ADC is applied and multi-bit ADC can facilitate reducing interference in baseband processing. - Generally speaking, it has poor receiver sensitivity. - Generally speaking, it has poor interference rejection. - The interference rejection for adjacent channel interference or interference from legacy NR signals on adjacent subcarriers can be potentially done using a high-Q matching network and RF BPF, which has high power consumption/cost/area and is band-specific. - It suffers from flicker 1/f noise and DC offset. |
| Intel | We wonder high-Q matching network and RF BPF means off-chip component, e.g. high-Q inductor or RAW. If this is the common understanding, it can be captured in the proposal. |
| Futurewei | We agree with ZTE, Sanechips, TCL, and Nordic that we do not have to qualify the architecture’s sensitivity or interference rejection here and can rather focus on the properties of the components. We also suggest considering a general filtering block after RF ED. Further, we may consider an optional mixing stage after BB filtering to capture the 2-tone reception architecture as noted in our answer to Q 2-5. |
| Nokia, NSB | Generally, we are fine with the proposal, but we feel that this architecture potentially has another drawback that is worthy of a bullet: • Compared with other architectures, the lack of an LO, can lead to a poorer (even fixed) range of LP-WUS locations. For the slightly controversial bullets raised by ZTE, perhaps replacing “Generally speaking ..” with “Compared to the other architectures discussed here, it generally has ..” is suffice? |
| CATT | We are OK with the principle of the description. The description and functions need to further fine tuning. We kind of agree with other companies on the receiver sensitivity since it is a relative term. We prefer vivo’s modification. |
| OPPO | We are fine with the principle of the proposal. We share the views of other companies on the description modification. |
| MediaTek | Power hungry components, RF BPF, RF LNA and BB AMP can be optional, because RF BPF is to improve inference rejection, RF LNA is to improve sensitivity for the use of RF RD, and BB AMP and BB LPF are to improve sensitivity for the use of 1-bit ADC. #### Proposal 2-2-MTK: (RF envelope detection) - [omit unchanged] - RF LNA and BB AMP can be optionally applied to improve sensitivity, with the cost of additional power consumption. - RF BPF can be optionally applied to improve interference rejection, with the cast of power consumption/cost/area. - Generally speaking, it has poor receiver sensitivity, compared to Heterodyne and Zero-IF architectures. - Generally speaking, it has poor interference rejection, compared to Heterodyne and Zero-IF architectures. The interference rejection for adjacent channel interference or interference from legacy NR signals on adjacent subcarriers can be potentially done using a high-Q matching network and RF BPF, which has high power consumption/cost/area and is band-specific. |
| Lenovo | After the third bullet we can delete rest of the generally speak. As part of the conclusion of the study we can mention it |
| QC | We are generally fine with capturing pro/cons of different receiver architecture. But, we think it is too early to make conclusions. |
| Samsung | It should be discussed after proposal 2-1 is stable. |
| Moderator | Thanks for the good comments. Most companies think it is too early to conclude on pros/cons at this stage, so those statements are removed from the updated proposal. @MediaTek, on RF BPF, my understanding is that it is needed at least as a band filter, which filters the interference out of the band (which may consist of multiple carrriers). If we want to use it for ACI rejection, I wonder if the bullet on interference rejection can address the concern. @all, I kept the last bullet for now because it is a fact. Please comment further. @Samsung, ideally yes, it is better to discuss after P2-1 is stable. Given the feedback so far, it seems that companies are fine with it in general, maybe with some fine-tuning on wording, so it is good to discuss things in parallel given the limited time we have. #### Proposal 2-2r1: (RF envelope detection) For OOK modulation, the RF envelope detection architecture can be studied based on the following diagram. - The RF signal is converted into baseband signal directly via an RF envelope detector. - It can achieve very low power consumption due to the removal of power-hungry local oscillator (LO) and PLL. - There is no Local Oscillator (LO) and no Phase-Locked Loop (PLL). - RF LNA and BB AMP can be optionally applied to improve sensitivity, with the cost of additional power consumption. - Generally speaking, it has poor receiver sensitivity. - Generally speaking, it has poor interference rejection. - The interference rejection for adjacent channel interference or interference from legacy NR signals on adjacent subcarriers can be potentially done using a high-Q matching network and RF BPF, which may require off-chip components has high power consumption/cost/area and is band-specific. - It suffers from flicker 1/f noise and DC offset. |
| Everactive | Agree with the revised proposal I would add that the RF front-end is broadband, and band or channel tuning is performed with a tunable matching network, RF BPF, and/or LNA. Regarding interference rejection: In general, it is true that an ED-first architecture is more susceptible to interference. In the plot below of published receivers, all points below 10µW are ED-first. Many ED-first publications do not report their SIR(ACR). New methods for improving ACR in these architectures have been proposed in literature. 2-tone FSK and chirping are two examples. Off-chip high-Q matching and RF BPFs help with ACR, but are not easily tunable for multi-band, multi-carrier support. David D. Wentzloff, "Ultra-Low Power Radio Survey," [online]. www.eecs.umich.edu/wics/low_power_radio_survey.html |
| Huawei, Hisilicon | For proposal 2-2, 2-3, 2-4: It may be easier in this first meeting to remove all the text and keep only the diagram. The text can be (re-)added later based on evaluations and assessments. If the text is kept, then as in our other comments, this agenda item should not address signal design / modulation, rather should just identify the architectures. For OOK modulation, the RF envelope detection architecture can be studied based on the following diagram. - The RF signal is converted into baseband signal directly via an RF envelope detector. - It can achieve very low power consumption due to the removal of power-hungry local oscillator (LO) and PLL. - There is no Local Oscillator (LO) and no Phase-Locked Loop (PLL). - RF LNA and BB AMP can be optionally applied to improve sensitivity, with the cost of additional power consumption. - Generally speaking, it has poor receiver sensitivity. - Generally speaking, it has poor interference rejection. - The interference rejection for adjacent channel interference or interference from legacy NR signals on adjacent subcarriers can be potentially done using a high-Q matching network and RF BPF, which may require off-chip components has high power consumption/cost/area and is band-specific. - It suffers from flicker 1/f noise and DC offset. |
| vivo2 | - BB AMP: Due to non-linearity of envelop detection, signal, interference, and noise will be superposed after passing envelop detector, BB AMP will amplify them to the same level and thus, BB AMP may not help improving sensitivity. - Flicker 1/f noise and DC offset: only active envelop detector suffers flicker 1/f noise and DC offset, while passive envelop detector without relying on bias current can avoid this. So we suggest the following: #### Proposal 2-2r1: (RF envelope detection) For OOK modulation, the RF envelope detection architecture can be studied based on the following diagram. - The RF signal is converted into baseband signal directly via an RF envelope detector. - It can achieve very low power consumption due to the removal of power-hungry local oscillator (LO) and PLL. - There is no Local Oscillator (LO) and no Phase-Locked Loop (PLL). - RF LNA and BB AMP can be optionally applied to improve sensitivity, with the cost of additional power consumption. - Generally speaking, it has poor receiver sensitivity. - Generally speaking, it has poor interference rejection. - The interference rejection for adjacent channel interference or interference from legacy NR signals on adjacent subcarriers can be potentially done using a high-Q matching network and RF BPF, which may require off-chip components has high power consumption/cost/area and is band-specific. - It suffers from flicker 1/f noise and DC offset when active envelop detector is adopted. |
| SONY | The updated proposal from the moderator looks OK. We think the last bullet (on flicker noise and DC offset) should be removed. Whether the bullet describes a fact or not, the bullet says nothing about the architecture, but something about the performance. We can draw conclusions on performance later. - The figure doesn’t show clearly on some computer systems (e.g. mine). It looks like it is a zoomed JPEG. Can we include a figure that is better rendered? |
| Moderator | #### Proposal 2-2r2: (RF envelope detection) For OOK modulation, the RF envelope detection architecture can be studied based on the following diagram. - The RF signal is converted into baseband signal directly via an RF envelope detector. - It can achieve very low power consumption due to the removal of power-hungry local oscillator (LO) and PLL. - There is no Local Oscillator (LO) and no Phase-Locked Loop (PLL). - RF LNA [and BB AMP] can be optionally applied to improve sensitivity, with the cost of additional power consumption. - Generally speaking, it has poor receiver sensitivity. - Generally speaking, it has poor interference rejection. - The interference rejection for adjacent channel interference or interference from legacy NR signals on adjacent subcarriers can be potentially done using a high-Q matching network and RF BPF, which may require off-chip components has high power consumption/cost/area and is band-specific. - It suffers from flicker 1/f noise and DC offset. @Huawei/HiSi, for now I am still keeping “OOK modulation” in the main bullet. Please see my comments for P2-1. If later discussion suggests that the same architecture can also be used for other modulation schemes, we can have another agreement. I also wonder why you want to remove all the descriptions and just keep the diagram. The sub-bullets intend to be descriptive/informative at this stage, to complement the diagram. The controversial part involving any pros/cons assessment has been removed. Would appreciate if you can elaborate more on your concern, or more specifically, on which sub-bullets you have concern. Based on the comments, I will simply remove the last bullet for now and we can come back to this discussion later. @Everactive, thanks for the comments. @vivo, I am keeping “BB AMP” in the bracket now to seek for more input. |
| ZTE, Sanechips | Similar as other architechtures, the ADC also can be considered, which can be added as follows - 1bit or multi-bit ADC is applied in baseband processing. #### Additionally, we think we can add ‘at least’ to make it open at current stage, as follows: For OOK modulation, the RF envelope detection architecture can be studied at least based on the following diagram. |
| Futurewei 2 | We agree with (ZTE, Sanechips) suggestion to add “at least” to the main point and suggest the following text based on text in proposal 2-1r2. For OOK modulation, Study the RF envelope detection architecture can be studied based on at least the following diagram for LP-WUR based on OOK modulation. Otherwise, we suggest adding the following sub-bullet under the interference rejection bullet: FFS: interference rejection based on other BB processing techniques. |
| vivo | Support to add multi-bit ADC here. |
| Nokia, NSB2 | Agree with ZTE, if the text description is defining the key characteristics of the architecture, then the final stage detector should be mentioned. For the [BB Amp] option, we are ok to keep this but maybe prefixed by “and/or BB Amp” If we mention how interference rejection can be done (off-chip band-specific components), should we also mention how this architecture can be modified/developed to handle flexible placement of the LP-WUS? Also, is the terminology “band specific” understood to allow flexible positioning within a band or does it assume a fixed position with a band? |
| Panasonic | We are okay. |
| Intel | BB AMP may not help to suppress noise however it seems necessary to get a reasonable signal level for follow-up processing. We prefer to keep the bullet on ‘flicker 1/f noise and DC offset’. Otherwise, do we plan to have another agreement/conclusion to do interference analysis? |
| Samsung | It should be noticed that there is a trade-off between signal design/modulation and the architecture. If the intention of this proposal is to describe some details of the RF envelope detection architecture, rather than its pros and cons, the same logic should be followed, and the last bullet can be revised to illustrate the impact of module, which can be shown as following: - High-Q matching network and RF BPF can be used to suppress adjacent channel interference or interference from legacy NR signals on adjacent subcarriers, which may require off-chip components and is band-specific. In addition, ‘for evaluation purpose ‘should be added to make sure that a specific receiver architecture should not be enforced to implement, so the main bullet can be: For OOK modulation, the RF envelope detection architecture can be studied for evaluation purpose based on the following diagram. |
| SONY | Can we just removed the “for OOK modulation” at the start of the proposal. As Everactive point out, “New methods for improving ACR in these architectures have been proposed in literature. 2-tone FSK and chirping are two examples”. Hence, it seems like the RF envelope detection architecture is not tied to OOK modulation, but is more generic. |
| MediaTek | Okay. |
| Huawei, Hisilicon | #### We agree with SONY. #### This agenda item should not agree or imply to support particular modulation schemes that belongs to 9.13.3, so the opening sentence clause needs to be removed, hence it will begin with “The RF envelope detection ….”. The mention of OOK in P 2-1 is adequate to provide context to this proposal. Our concern on the text was in writing already now conclusion judgements before discussions. Other companies made a similar point. As a compromise, we can accept keeping the rest of the bullets together with the diagram, following the textual updates shown. With the deletion of - RF LNA [and BB AMP] can be optionally applied to improve sensitivity, with the cost of additional power consumption. |
| Everactive | #### The original schematic showed an optional 1-bit or multi-bit ADC. We agree with other companies that this should be included in the list. #### Otherwise, we are ok with the proposal. |
| QC | ADC ADC description could be added. In-band Interference rejection Suggest to add “ and BB LPF". RF BPF alone may not be good enough to filter out in-band NR signals in adjacent subcarriers. - The interference rejection for adjacent channel interference or interference from legacy NR signals on adjacent subcarriers can be potentially done using a high-Q matching network and RF BPF and BB LPF, which where PF BPF may require off-chip components has high power consumption/cost/area and is band-specific. |
| Moderator | Updated version with diff marks on top of r2: #### Proposal 2-2r3: (RF envelope detection) For OOK modulation, For evaluation purpose, study the RF envelope detection architecture can be studied based on at least the following diagram for LP-WUR based on OOK modulation. - The RF signal is converted into baseband signal directly via an RF envelope detector. - There is no Local Oscillator (LO) and no Phase-Locked Loop (PLL). - 1-bit or multi-bit ADC is applied in baseband processing. - RF LNA [and/or BB AMP] can be optionally applied [to improve sensitivity, with the cost of additional power consumption]. - High-Q matching network and/or RF BPF and/or BB LPF can be used to suppress The interference rejection for adjacent channel interference or interference from legacy NR signals on adjacent subcarriers can be potentially done using a high-Q matching network and RF BPF, which may require off-chip components and is band-specific. The diagram is updated below (changed the dashed line around RF LNA to solid line). I added the sentence for ADC. I did not add it originally because I thought it is obvious in the diagram already. But it is good to have some descriptive text. @Nokia/NSB, if you have anything to suggest for flexible placement of LP-WUS, please suggest and we can discuss. Otherwise, we have plenty of time to discuss in the future. We will continue to work on this, including the pros/cons analysis, once we have better understanding. Right now, it just captures the basic things that are agreeable to everyone at this stage. “Band specific” means that for each supported band, it needs to be different. It does not have much to do with the location of the signal within the band in my understanding. @Intel, for the bullet on “flicker noise and DC offset”, it is probably better that we can come back to this discussion later, as some companies think there are certain ways to handle it and this is part of pros/cons analysis. The main bullet is modified in a similar way as in P2-1. I think this particular diagram is for OOK modulation. Therefore, I think it makes sense to keep OOK in the main bullet. |

The diagram associated with Proposal 2-2r3 is updated as below:

#### Proposal 2-3: (Heterodyne)

For OOK modulation, the heterodyne architecture can be studied based on the following diagram.

- The RF signal is down converted into IF signal via an RF mixer with a LO. After the amplification and filtering of IF signal, the signal is converted into baseband signal via an IF envelope detection.

  - There may be multiple IF stages.

- The choice of the LO is one of the major factors that determines the power consumption.

  - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, the corresponding frequency offset should be taken into account in the design and evaluation.

  - One example of low power oscillators is the ring oscillator.

  - FLL (frequency locked loop) can replace PLL for non-coherent detection.

- Amplification and filtering at IF is more power efficient than at RF.

- The IF should be low enough so that the IF components can be integrated on-chip, yet still high enough to avoid DC offset and flicker noise.

- The interference rejection can be performed by a high-Q IF BPF.

- Generally speaking, it has good receiver sensitivity.

  - RF LNA (e.g. low-cost) can be optionally applied to further improve sensitivity, with the cost of additional power consumption.

- Image rejection filter is required.

- It has relatively high power consumption.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | Similar comment as in proposal 2-2 |
| Spreadtrum | We basically agree with this proposal, but we have one comment that in order to obtain better sensitivity, PLL may not be precluded at current. PLL can be further discussed to be compared with FLL. |
| Nordic | - Agree that for comparison we should study also the case with PLL - Image rejection filter is required, which must be a complex filter. - FLL compared to PLL is expected to have larger frequency error, resulting in uncertain position of LP-WUS in IF-band - FLL compared to PLL is expected to have increased phase noise which may results in aliasing of neighbour band to LP-WUS band - LO error will impact also timing accuracy and receiver will need to wake earlier or more often to synchronize. |
| Panasonic | We are generally supportive. Regarding “FLL (frequency locked loop) can replace PLL for non-coherent detection.”, we understand it is true in the context of OOK modulation, as confined in the main bullet. However, it is still in the initial stage of the study and other schemes are still not precluded, e.g. OFDM based MC-OOK and FSK. Thus we do not need to mandate the condition of using FLL. Given that, we propose to change to: o FLL (frequency locked loop) can possibly replace PLL depending on the detailed LP-WUS design. |
| Vivo | It seems that we can describe the functionality of each block in the diagram first and get common understanding on this receiver architecture before commenting on its pros and cons. So we suggest the following: For OOK modulation, the heterodyne architecture can be studied based on the following diagram. - The RF signal is down converted into IF signal via an RF mixer with a LO. After the amplification and filtering of IF signal, the signal is converted into baseband signal via an IF envelope detection. - There may be multiple IF stages. - The choice of the LO is one of the major factors that determines the power consumption. - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, the corresponding frequency offset should be taken into account in the design and evaluation. - One example of low power oscillators is the ring oscillator. - FLL (frequency locked loop) can replace PLL for non-coherent detection. - Amplification and filtering at IF is more power efficient than at RF. - The IF should be low enough so that the IF components can be integrated on-chip, yet still high enough to avoid DC offset and flicker noise. - The interference rejection can be performed by a high-Q IF BPF. - Generally speaking, it has good receiver sensitivity. - RF LNA (e.g. low-cost) can be optionally applied to further improve sensitivity, with the cost of additional power consumption. - Image rejection filter is required. - 1bit or multi-bit ADC is applied and multi-bit ADC can facilitate reducing interference in baseband processing. - It has relatively high power consumption. |
| Intel | We are supportive to the proposal |
| Futurewei | We are fine with the proposal, but we do not have to qualify the architecture’s sensitivity here. Further, we suggest indicating that the shown diagram can be used as a guideline for the evaluation but additional components (e.g., number of IF stages) and their impact on power consumption and other performance metrics should not be precluded from evaluation. |
| Nokia, NSB | Generally, agree with the proposal, though we have a few suggestions: (1) Similar comment as in 2-2 regarding the rephrasing of “generally speaking” (2) Should the generally better selectivity and support of flexible positioning of the LP-WUS(given the LO) be emphasised? |
| CATT | We are generally OK with the principle of proposal with fine tuning the word. Similarly, the receiver sensitivity should be described completely in terms of range of dBm not using relative term. |
| OPPO | We are fine with the principle of the proposal. We share the views of other companies on the description modification. For example, “relatively” is not clear for us to compare the three receiver architecture types. |
| MediaTek | Power hungry components, RF BPF and BB AMP can be optional, because of the same reasons given in Proposal 2-2. #### Proposal 2-3-MTK: (Heterodyne) - [omit unchanged] - Generally speaking, it has good receiver sensitivity, compared to RF envelope detection and Zero-IF architectures. - RF LNA (e.g. low-cost) can be optionally applied to further improve sensitivity, with the cost of additional power consumption. - RF BPF can be optionally applied to improve interference rejection, with the cast of power consumption/cost/area. - Image rejection filter is required. It has relatively high power consumption, compared to RF envelope detection and Zero-IF architectures. |
| Lenovo | We agree to the proposal but the last three bullet can be deleted |
| QC | We are generally fine with capturing pro/cons of different receiver architecture. But, we think it is too early to make conclusions. |
| Samsung | It should be discussed after proposal 2-1 is stable. |
| Moderator | I updated the proposal based on the comments. @MediaTek, regarding your comment on RF BPF, my understanding of this architecture is that RF BPF performs band filtering, while ACI rejection is performed by IF filter to save power. Not sure which interference you refer to in “interference rejection”. #### Proposal 2-3r1: (Heterodyne) For OOK modulation, the heterodyne architecture can be studied based on the following diagram. - The RF signal is down converted into IF signal via an RF mixer with a LO. After the amplification and filtering of IF signal, the signal is converted into baseband signal via an IF envelope detection. - There may be multiple IF stages. - The choice of the LO is one of the major factors that determines the power consumption. - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, the corresponding frequency offset should be taken into account in the design and evaluation. - One example of low power oscillators is the ring oscillator. - FLL (frequency locked loop) can may replace PLL for non-coherent detection. - Amplification and filtering at IF is more power efficient than at RF. - The IF should be low enough so that the IF components can be integrated on-chip, yet still high enough to avoid DC offset and flicker noise. - The interference rejection can be performed by a high-Q IF BPF. - Multi-bit ADC can facilitate reducing interference in baseband processing. - Generally speaking, it has good receiver sensitivity. - RF LNA (e.g. low-cost) and BB AMP can be optionally applied to further improve sensitivity, with the cost of additional power consumption. - Image rejection filter is required. - It has relatively high power consumption. |
| Everactive | Agree with the revised proposal Would add that band or channel tuning can be performed by tuning the RF-LO frequency. (this is in contrast to the ED-first architecture above) |
| Huawei, Hisilicon | Similar as proposal 2-2: Delete “For OOK modulation, the” from the start. But also consider just capturing only the diagram for this meeting. |
| vivo2 | Similar as proposal 2-2, we suggest to delete BB AMP here |
| SONY | Support the updated proposal. |
| Moderator | No changes are made on top of P2-3r1. @Huawei, HiSi, please refer to my comments on P2-2. @vivo2 and all, I haven’t made any update regarding “BB AMP”. But this is the same issue as in P2-2, and we can follow the same approach for both. Would appreciate the input from companies. |
| ZTE, Sanechips | The following statement is not appropriate for the architecture definition, and it is more like some aspects needs to be considered for each component design. So, we suggest to remove the following sentence and keep it open for future discussion. - The IF should be low enough so that the IF components can be integrated on-chip, yet still high enough to avoid DC offset and flicker noise. Additionally, to make the discussion open, we suggest modify the following sentence - Multi-bit ADC can facilitate reducing interference in baseband processing. as following: - 1bit or multi-bit ADC is applied in baseband processing. |
| Nordic | - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, the corresponding frequency offset and increased phase noise should be taken into account in the design and evaluation. |
| Futurewei 2 | We agree with suggestions from (ZTE, Sanechips) and Nordic on proposal 2-3r1, and similar to our comment on proposal 2-2, we suggest the following text for the main point: For OOK modulation, Study the heterodyne architecture can be studied based on at least the following diagram for LP-WUR based on OOK modulation. |
| vivo | ok |
| Nokia, NSB2 | Support the ZTE comments and we should strive for consistency between the architecture descriptions. Should “RF LNA (e.g. low-cost) and BB AMP” be “RF LNA (e.g. low-cost) and/or BB AMP”? |
| Panasonic | Thanks for the updates. It is good with us. |
| Intel | Since this is the IF based architecture, it would be helpful to include the following guideline - The IF should be low enough so that the IF components can be integrated on-chip, yet still high enough to avoid DC offset and flicker noise. |
| Samsung | Agree with the revised proposal in general. And similar comment as proposal 2-2. And ‘for evaluation purpose ‘should also be added in the main bullet, so the main bullet can be: For OOK modulation, the heterodyne architecture can be studied for evaluation purpose based on the following diagram. |
| SONY | We agree with Huawei on removing the “for OOK modulation” part, as per our comment on proposal 2-2. We quite like the bullets that ZTE don’t like (on choice of IF frequency and use of multi-bit ADC for interference issues) and would be happy to keep them in the proposal. |
| MediaTek | Everactive’s comment seems reasonable. However, we are not sure about any power/sensitivity performance impact. We suggest adding a new sub-bullet for the next meeting. FFS: band or channel tuning can be performed by tuning the RF-LO frequency |
| Huawei, Hisilicon | Same comments as P2-2. To remove “For OOK modulation,” and keep after |
| Everactive | There are many examples of ultra-low-power OOK heterodyne receivers that use uncertain-IF to relax the requirements on the baseband filtering and the LO (saving power). We propose adding one bullet to capture that as an architecture to study. For OOK modulation, the heterodyne architecture can be studied based on the following diagram. - The RF signal is down converted into IF signal via an RF mixer with a LO. After the amplification and filtering of IF signal, the signal is converted into baseband signal via an IF envelope detection. - There may be multiple IF stages. - The IF may be uncertain-IF Agree with MediaTek’s proposed bullet: band or channel tuning can be performed by tuning the RF-LO frequency |
| QC | Agree to add ADC bullet. We suggest the following modification. - There may be multiple IF stages. There could be one or more IF stages depending on design. In-band interference rejection could be done by IF BPF or BB LPF. We suggest to add these filters as well. - The interference rejection can be performed by a high-Q RF BPF, IF BPF, and BB LPF if any. |
| Moderator | Updated version with diff marks on top of r1: #### Proposal 2-3r2: (Heterodyne) For OOK modulation, For evaluation purpose, study the heterodyne architecture with IF envelope detection can be studied based on at least the following diagram for LP-WUR based on OOK modulation. - The RF signal is down converted into IF signal via an RF mixer with a LO. After the amplification and filtering of IF signal, the signal is converted into baseband signal via an IF envelope detection. - There may be one or multiple IF stages depending on design. - The choice of the LO is one of the major factors that determines the power consumption. - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, the corresponding frequency offset and increased phase noise should be taken into account in the design and evaluation. - One example of low power oscillators is the ring oscillator. - FLL (frequency locked loop) may replace PLL for non-coherent detection. - 1-bit or multi-bit ADC is applied in baseband processing. - [The IF should be low enough so that the IF components can be integrated on-chip, yet still high enough to avoid DC offset and flicker noise.] - The interference rejection can be performed by a high-Q RF BPF and/or IF BPF and/or BB LPF. - Multi-bit ADC can facilitate reducing interference in baseband processing. - RF LNA and/or BB AMP can be optionally applied to further improve sensitivity, with the cost of additional power consumption. - Image rejection filter is required. - [Band or channel tuning can be performed by tuning the RF-LO frequency.] The diagram is updated below (changed the dashed line around RF LNA to solid line). @ZTE, on the IF sentence, I keep it in bracket for now as quite a few companies prefer to keep it as a guideline. My suggestion is that you agree with the sentence technically, it should be ok to include it. Otherwise, we can discuss further. @MediaTek, I agree that Everactive has good comments here. But my view is that we do not need to add FFS bullet at this point (otherwise we may end up with many FFS points). All the aspects can be investigated, and this is just a very basic description. I added the sentence in bracket. If companies can agree to it, that would be great. If not, I suggest we simply remove it for now, and discuss it in the future. @Everactive, I think the “uncertain-IF” aspect is covered by the sub-bullets under LO already. Please check. |

The diagram associated with Proposal 2-3r2 is updated as below:

#### Proposal 2-4: (zero-IF)

For OOK modulation, the zero-IF architecture can be studied based on the following diagram.

- The RF signal is directly down converted into baseband signal via an RF mixer with a LO.

- The choice of the LO is one of the major factors that determines the power consumption.

  - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, such frequency offset should be taken into account in the design and evaluation.

  - FLL (frequency locked loop) can replace PLL for non-coherent detection.

- Signal processing at BB is easier and more power efficient than at RF or IF.

- The interference rejection can be performed by a high-Q BB BPF.

- No image rejection filter is required.

- Generally speaking, it has better receiver sensitivity than the RF envelope detection architecture but worse than heterodyne architecture.

  - RF LNA (e.g. low-cost) can be optionally applied to further improve sensitivity, with the cost of additional power consumption.

- It suffers from LO leakage/DC offset and flicker noise.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | Similar comment as in proposal 2-2 |
| Spreadtrum | We are fine with this proposal. |
| Nordic | - It suffers from LO leakage/DC offset and flicker noise, which can be partially tackled with using BPF at BB instead of LPF at BB. - FLL compared to PLL is expected to have larger frequency error, resulting in uncertain position of LP-WUS in base-band - FLL compared to PLL is expected to have increased phase noise which may results in aliasing of neighbour band to LP-WUS band - LO error will impact also timing accuracy and receiver will need to wake earlier or more often to synchronize. |
| Panasonic | Same comment as to proposal 2-3 |
| vivo | It seems that we can describe the functionality of each block in the diagram first and get common understanding on this receiver architecture before commenting on its pros and cons. So we suggest the following: For OOK modulation, the zero-IF architecture can be studied based on the following diagram. - The RF signal is directly down converted into baseband signal via an RF mixer with a LO. - RF LNA (e.g. low-cost) can be optionally applied to further improve sensitivity, with the cost of additional power consumption. - The choice of the LO is one of the major factors that determines the power consumption. - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, such frequency offset should be taken into account in the design and evaluation. - FLL (frequency locked loop) can replace PLL for non-coherent detection. - Signal processing at BB is easier and more power efficient than at RF or IF. - The interference rejection can be performed by a high-Q BB BPF. - No image rejection filter is required. - Generally speaking, it has better receiver sensitivity than the RF envelope detection architecture but worse than heterodyne architecture. - RF LNA (e.g. low-cost) can be optionally applied to further improve sensitivity, with the cost of additional power consumption. - 1bit or multi-bit ADC is applied and multi-bit ADC can facilitate reducing interference in baseband processing. - It suffers from LO leakage/DC offset and flicker noise. |
| Intel | We are supportive to the proposal |
| Futurewei | We are fine with the proposal, but we do not have to qualify the architecture’s sensitivity here. We also agree with Nordic that, based on the considered LP-WUS bandwidth, a BPF at BB may be considered instead of a LPF at BB. So, we would suggest considering a general filtering block after AMP. |
| Nokia, NSB | Similar comment as in proposal 2-2, regarding “generally speaking”. Also, should a bullet be added, to emphasis the support of flexible positioning of the LP-WUS within a band (as a consequence of the LO)? |
| CATT | We are generally OK with the principle of proposal with fine tuning the word. Similarly, the receiver sensitivity should be described completely in terms of range of dBm not using relative term. |
| OPPO | Similar comments with proposal 2-2 and 2-3. |
| MediaTek | BB BPF is NOT in the diagram. Does it mean RF BPF? #### Proposal 2-4-MTK: (zero-IF) - [omit unchanged] The interference rejection can be performed by a high-Q BB RF BPF optionally. |
| Lenovo | Same as above proposal |
| QC | We are generally fine with capturing pro/cons of different receiver architecture. But, we think it is too early to make conclusions. |
| Moderator | @Nordic, @Futurewei, I will update the diagram to replace BB LPF with a generic filter. The change in the last bullet is just to keep consistent language as in P2-2. #### @MediaTek, my understanding of this architecture is to avoid high-Q RF BPF. Would be good to hear other companies’ views. #### Proposal 2-4r1: (zero-IF) For OOK modulation, the zero-IF architecture can be studied based on the following diagram. - The RF signal is directly down converted into baseband signal via an RF mixer with a LO. - The choice of the LO is one of the major factors that determines the power consumption. - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, such frequency offset should be taken into account in the design and evaluation. - FLL (frequency locked loop) can may replace PLL for non-coherent detection. - Signal processing at BB is easier and more power efficient than at RF or IF. - The interference rejection can be performed by a high-Q BB BPF. - No image rejection filter is required. - Generally speaking, it has better receiver sensitivity than the RF envelope detection architecture but worse than heterodyne architecture. - RF LNA (e.g. low-cost) can be optionally applied to further improve sensitivity, with the cost of additional power consumption. - It suffers from LO leakage/DC offset and flicker noise and DC offset, which needs to be addressed. |
| Everactive | Agree with the revised proposal Would add that band or channel tuning can be performed by tuning the RF-LO frequency. (this is in contrast to the ED-first architecture above) |
| Huawei, Hisilicon | Similar as proposal 2-2: Delete “For OOK modulation, the” from the start. But also consider just capturing only the diagram for this meeting. |
| vivo 2 | Ok with the revised proposal |
| SONY | Updated proposal looks good. |
| Moderator | #### Proposal 2-4r2: (zero-IF) For OOK modulation, the zero-IF architecture can be studied based on the following diagram. - The RF signal is directly down converted into baseband signal via an RF mixer with a LO. - The choice of the LO is one of the major factors that determines the power consumption. - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, such frequency offset should be taken into account in the design and evaluation. - FLL (frequency locked loop) can may replace PLL for non-coherent detection. - Signal processing at BB is easier and more power efficient than at RF or IF. - The interference rejection can be performed by a high-Q BB BPF. - No image rejection filter is required. - Generally speaking, it has better receiver sensitivity than the RF envelope detection architecture but worse than heterodyne architecture. - RF LNA (e.g. low-cost) can be optionally applied to further improve sensitivity, with the cost of additional power consumption. - It suffers from LO leakage/DC offset and flicker noise and DC offset, which needs to be addressed. The last bullet is removed to be consistent with P2-2r2. @Huawei/HiSi, please see my comments for P2-2. |
| ZTE, Sanechips | Suggest to add ‘at least’ to make it open as follows For OOK modulation, the zero-IF architecture can be studied based on at least the following diagram. And suggest to consider 1bit or multi-bit ADC similar as other architectures. |
| Nordic | - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, the corresponding frequency offset and increased phase noise should be taken into account in the design and evaluation. |
| Futurewei 2 | We again agree with suggestions from (ZTE, Sanechips) and Nordic on proposal 2-4r2 for consistency, and further suggest the following text for the main point: For OOK modulation, Study the zero-IF architecture can be studied based on at least the following diagram for LP-WUR based on OOK modulation. |
| vivo | Support to add multi-bit ADC also here as it facilitates interference rejection in digital BB processing. |
| Nokia, NSB2 | For consistency, we agree with ZTE about the ADC bullet. Also, should the RF LNA bullet include “and/or BB Amp”? If we are mentioning the factors that determine LO choice, should tuneability (to support flexible LP-WUS placement) be considered? |
| Panasonic | We are okay. |
| Intel | We prefer to keep the last bullet for more information. otherwise, separate agreement/conclusion is fine too |
| Samsung | Agree with the revised proposal in general. And similar comment as proposal 2-2. And ‘for evaluation purpose ‘should also be added in the main bullet, so the main bullet can be: For OOK modulation, the zero-IF architecture can be studied for evaluation purpose based on the following diagram. |
| SONY | We agree with Huawei on removing the “for OOK modulation” part, as per our comment on proposal 2-2. |
| MediaTek | Everactive’s comment seems reasonable. However, we are not sure about any power/sensitivity performance impact. We suggest adding a new sub-bullet for the next meeting. FFS: band or channel tuning can be performed by tuning the RF-LO frequency |
| Huawei, Hisilicon | Additional to removing the mention of OOK (as per P2-2), for the LO description, similar as Heterodyne, we think the ring oscillator can also be one example of low power oscillators for zero-IF architecture. So we suggest to add this sentences: - One example of low power oscillators is the ring oscillator. We suggest the following deletion - RF LNA (e.g. low-cost) can be optionally applied to further improve sensitivity, with the cost of additional power consumption. |
| Everactive | Agree with MediaTek to add a bullet: band or channel tuning can be performed by tuning the RF-LO frequency |
| QC | Agree to add ADC bullet. |
| Moderator | Updated version with diff marks on top of r2: #### Proposal 2-4r3: (zero-IF) For OOK modulation, For evaluation purpose, study the homodyne/zero-IF architecture with baseband envelope detection can be studied based on at least the following diagram for LP-WUR based on OOK modulation. - The RF signal is directly down converted into baseband signal via an RF mixer with a LO. - The choice of the LO is one of the major factors that determines the power consumption. - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, such frequency offset and increased phase noise should be taken into account in the design and evaluation. - FLL (frequency locked loop) may replace PLL for non-coherent detection. - 1-bit or multi-bit ADC is applied in baseband processing. - The interference rejection can be performed by a high-Q RF BPF and/or BB BPF. - No image rejection filter is required. - RF LNA and/or BB AMP can be optionally applied [to further improve sensitivity, with the cost of additional power consumption]. The diagram is updated below. |

The diagram associated with Proposal 2-4r3 is also updated for the BB filter as commented by Nordic.

There are a few other receiver architectures discussed by companies but they do not seem to fit well into any of the three types discussed above, e.g.

- Futurewei: double-sampling (synchronized switching) architecture, sub-sampling architecture, 2-tone reception architecture

- InterDigital: a receiver with a received signal strength indicator (RSSI) circuit

- Nokia: reuse 5G main radio RF architecture

#### [CLOSED] Q 2-5: (additional architecture?)

Please comment on whether additional receiver architecture (other than the three types in Proposal 2-2) should be considered for OOK modulation. If yes, please provide details.

| Company | Comments |
| --- | --- |
| Futurewei | We would just like to note that: - The sub-sampling architecture is similar in concept to the heterodyne envelope detection architecture but utilizing a sub-sampling block (with lower LO frequency) instead of a mixer at the beginning of the architecture. - The 2-tone reception architecture is similar in concept to the RF ED but considers an additional mixing step at BB (with low LO frequency in conjunction with 2-tone signal design and which can be a passive mixing step) to improve the architecture’s sensitivity and interference rejection with minimal impact on power consumption. So, at this stage we would like to include at least the 2-tone reception architecture as part of the RF ED architecture. |
| Nokia, NSB | In our view, the Heterodyne receiver discussed in proposal 2-3, covers the main points we wanted to make about using a LP-WUR based on a conventional receiver design. |
| MediaTek | These architectures seem to be variations or enhancements of the three types in Proposal 2-2. In this case, they share the common parts with the three main types, and we can evaluate them by using a more general baseband model. |
| Samsung | It should be discussed after proposal 2-1 is stable. |
| Everactive | The proposed architectures have enough flexibility. See the plot below for OOK receivers published in literature. David D. Wentzloff, "Ultra-Low Power Radio Survey," [online]. www.eecs.umich.edu/wics/low_power_radio_survey.html |
| Moderator | The only proposal so far is from Futurewei to include the 2-tone reception architecture, with the diagram shown below: In general, I assume at this stage we are not trying to exclude any particular receiver architecture, if it shows the potential. The only thing I am wondering is whether we need to take the time to agree on each of these architecture diagrams, or we can leave it to each individual company to provide the details, especially if the interest comes from a single company or very few companies at this point. @Huawei/HiSi, so instead of having a dedicated proposal to include such an architecture diagram, would it be ok if we add a bullet in P2-1 stating that other architecture is not precluded (which was the original intention anyway)? Would also appreciate comments from other companies. |
| Futurewei 2 | The 2-tone reception scheme can already fall under the RF ED architecture in proposal 2-2, so we are fine with our suggestion on proposal 2-2 to not limit the architecture to just the shown diagram by using “at least” in the main point or adding “FFS: interference rejection based on other BB processing techniques”. |
| vivo | We agree that we are not trying to exclude any particular receiver architecture, however, it may not be possible to list all of the variants from the listed three architectures as summarized in proposal 2-1. We support to add a bullet in P2-1 stating that other architecture is not precluded |
| Nokia, NSB2 | Given the time we have, we feel that we can leave it to individual companies to promote alternative architectures. |
| SONY | If we want to include a 2-tone architecture, would it be possible to have a figure that shows where the 2 tones are? The figure above shows 8 multiphases. Also, it would be nice if we could try to make the different figures and terminology as consistent as possible (e.g. it isn’t clear why this figure has the “IF BPF” before the IF mixer while the heterodyne receiver has the “IF BPF” after the IF mixer). |
| Everactive | 2-tone OOK provides better selectivity than 1-tone OOK with an ED-first architecture, with minimal increase in power. We agree this is good to include in the ED-first section. |
| Moderator | As suggested by Futurewei, “at least” is added to Proposal 2-2 already. This question is closed for this meeting. |

FSK modulation has been mentioned by some companies (e.g. Huawei/HiSi, ZTE, Intel, Ericsson) as one of the schemes that can be considered, but discussion on the corresponding receiver architecture is quite limited. The following two receiver architectures were proposed by ZTE and Huawei.

Type-1: FSK can be implemented as parallel OOK receivers and a comparator circuit

Type-2: It can use a similar architecture as for OOK, except that the envelope detector is replaced by a FM-to-AM detector. ([Moderator’s comment: there is no detail provided for FM-AM detector.])

#### [CLOSED] Q 2-6: (FSK)

Please comment on whether FSK modulation should be further considered for LP WUR and why. If yes, please also provide the receiver architecture(s).

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | FSK can provide higher data rate. However, the impacts on power consumption and coverage should be further considered. Therefore, in the early stage, we do not need to preclude the FSK. As for the FM-AM converter in type-2 structure, we may need more details, e.g. the circuit structure. |
| Spreadtrum | Fine for discussion. |
| TCL | In our view, the both the OOK and FSK should be studied further at this stage. |
| Nordic | Higher data rate /modulation order can be achieved with OOK as well, e.g. DFT-S-OFDMA could increase data rate while keeping sub-carrier spacing the same as for surrounding NR signals. As seen from TYPE-1 complexity increased, since multiple BPFs is needed. Signal is narrower, and thus more vulnerable to frequency error of ring oscillator. From our point of, we prefer to focus study on OOK. |
| Panasonic | We are open to study FSK as the detailed performance comparison on power consumption and sensitivity are not done yet. |
| vivo | For type 1, it requires parallel hardware of amplitude detection to detect FSK, resulting in doubled power consumption. For type 2, the achieved power consumption and sensitivity highly depends on the design of FM-AM converter, more details from the proponents are highly appreciated. Furthermore, higher data rate can also be achieved by adopting M-ary ASK which can reuse the receiver architecture designed for OOK. Therefore, M-ary ASK is a more preferred solution providing higher data rate via the same receiver architecture as OOK. |
| Intel | FSK can provide better performance than OOK, with a relatively increased power consumption. We prefer to keep it on the table for study. At least the above Type-1 receiver is applicable for FSK, other receivers can be further proposed. |
| Futurewei | We are OK to include FSK modulation and corresponding LP-WUR architecture as part of the study. |
| Nokia, NSB | Fine for further discussion. |
| CATT | We are OK to further study of FSK along with OOK since it has the robustness and simplicity for the demodulation. |
| OPPO | We are open to study FSK modulation and the corresponding LP-WUR architecture. |
| MediaTek | We are open to discuss but prefer to focus OOK. Based on the diagram above, non-coherent FSK may lead to higher system overhead than OOK since two OOK waveforms may potentially need multiple guard bands to prevent interference. |
| Lenovo | We are fine to study FSK for performance comparison purpose |
| Sharp | We are open to study FSK modulation for LP-WUS |
| Samsung | It should be discussed after proposal 2-1 is stable. |
| Everactive | FSK should be considered for the LP-WUS for the following reasons. - Everything else equal, FSK provides a 3dB advantage in link-budget over OOK. - FSK receivers can achieve <1mW power with good sensitivity and SIR(ACR). See the plot below for FSK receivers published in literature. Regarding the proposal: The type-1 architecture proposed above for FSK should be considered. The EDs can be followed by optional ADCs, and proceeded by optional amplification. I am not familiar with the type-2 architecture and FM-AM detector. More detail should be included. David D. Wentzloff, "Ultra-Low Power Radio Survey," [online]. www.eecs.umich.edu/wics/low_power_radio_survey.html |
| Huawei, Hisilicon | This set of receive architectures can be captured as a fair report of what was considered in the SI. This agenda item does not have the remit to decide on supported/not supported modulation schemes. FSK can be demodulated in parallel OOK receivers or FM-AM detector. The FM-AM detector, using a frequency discriminator circuit, converts frequency shift into amplitude change, which can be implemented in either analog or digital domain [1]. One example, as shown in the figure below, is a conventional quadrature FM discriminator. It multiplies received frequency modulated signal with a phase shifted version, followed by a low pass filter. The amplitude of the output signal is proportional to the frequency of the input signal. [1] M. Silva Pereira, J. Caldinhas Vaz, C. Azeredo Leme, J. T. de Sousa and J. Costa Freire, "A 170 μA All-Digital GFSK Demodulator With Rejection of Low SNR Packets for Bluetooth-LE," in IEEE Microwave and Wireless Components Letters, vol. 26, no. 6, pp. 452-454, June 2016, doi: 10.1109/LMWC.2016.2562639. Note: Low power receiver that uses FM-AM reported in [1] has a total power consumption of around 200 μW. |
| vivo2 | We have one quick question to everactive for better understanding. When saying ‘Everything else equal, FSK provides a 3dB advantage in link-budget over OOK’, we are wondering whether it is assumed that the hardware of receiver, total power consumption, and the occupied time-frequency resources (including required guardband) are equal for FSK and OOK here, and FSK still provides 3dB gain in link-budget over OOK. |
| SONY | We are open to study LP-WUR for FSK. Our initial view is that an FSK receiver has higher power consumption and its other performance metrics such as noise figure of sensitivity are not better than an OOK receiver. These aspects can come out in the study. |
| Ericsson1 | As indicated in earlier comment, at this initial stage all receiver types should be further studied including LP-WUR for FSK. |
| Moderator | The comments show that many companies have interest to investigate the architectures for FSK to understand it better. So I put together Proposal 2-6 for further discussion below. I include the two types of architecture above as examples, as the input is rather limited. More discussion may be needed to refine the proposal. There is no need to comment further under this question. @Samsung, this is for FSK, while P2-1 is for OOK. They are not related. |

#### [CLOSED] Proposal 2-6: (FSK)

Further study the receiver architectures for FSK, with the two examples shown below:

- Example 1: parallel OOK receivers and a comparator circuit

- Example 2: use a similar architecture as for OOK, except that the envelope detector is replaced by a FM-to-AM detector.

  - The FM-AM detector, using a frequency discriminator circuit, converts frequency shift into amplitude change, which can be implemented in either analog or digital domain. One example, as shown in the figure below, is a conventional quadrature FM discriminator. It multiplies received frequency modulated signal with a phase shifted version, followed by a low pass filter. The amplitude of the output signal is proportional to the frequency of the input signal.

| Company | Comments |
| --- | --- |
| Moderator | It is encouraged that companies (especially those who support the further study of FSK) can provide more input on the receiver architecture for FSK, to help formulate the proposal better. |
| vivo | - For example 1, as it says parallel OOK receivers, besides the diagram shown above (corresponding to RF envelop detection in proposal 2-2), our question is whether the OOK receiver type of heterodyne and zero-IF defined in proposal 2-3 and 2-4 are also included. If they are also included, we prefer to make it clear. - For example 2, when it says ‘similar architecture as for OOK’, we have similar question as in example 1, whether all three architecture types defined in 2-1 can apply |
| Huawei, Hisilicon | For example 2, since the FM-AM detector can be either implemented in analog or digital domain, we change the proposal as follows: #### Proposal 2-6: (FSK) Further study the receiver architectures for FSK, with the two examples shown below: - Example 1: parallel OOK receivers and a comparator circuit - Example 2: using FM-to-AM detector - Alternative 1: a similar architecture as for OOK, except that the envelope detector is replaced by a FM-to-AM detector. - Alternative 2: the FM-to-AM detector can implemented in digital domain after ADC. - The FM-AM detector, using a frequency discriminator circuit, converts frequency shift into amplitude change, which can be implemented in either analog or digital domain. One example, as shown in the figure below, is a conventional quadrature FM discriminator. It multiplies received frequency modulated signal with a phase shifted version, followed by a low pass filter. The amplitude of the output signal is proportional to the frequency of the input signal. - Note: Other architectures are not precluded. |
| Everactive | The architecture will depend on the LP-WUS, in particular the bandwidth of the FSK signal and the frequency deviation. The narrower the signal bandwidth, the higher the quality factor (Q) of the BPFs. Higher Q BPFs leads to higher power. Example for wideband FSK (e.g. 5-20MHz): J. Im, H. -S. Kim and D. D. Wentzloff, "A 470µW −92.5dBm OOK/FSK Receiver for IEEE 802.11 WiFi LP-WUR," ESSCIRC 2018 - IEEE 44th European Solid State Circuits Conference (ESSCIRC), 2018, pp. 302-305, doi: 10.1109/ESSCIRC.2018.8494331. Example for narrowband FSK (e.g. <1MHz): O. Abdelatty et al., "A Low Power Bluetooth Low-Energy Transmitter with a 10.5nJ Startup-Energy Crystal Oscillator," ESSCIRC 2019 - IEEE 45th European Solid State Circuits Conference (ESSCIRC), 2019, pp. 377-380, doi: 10.1109/ESSCIRC.2019.8902902. |
| Ericsson2 | Ok to study |
| QC | We are OK with the proposal. However, based on the literature review, FSK receiver is higher complexity and can have more power consumption than OOK receiver. It is a matter of fact that the FSK receiver can be implemented using two parallel OOK receivers. |
| Moderator | Updated proposal in r1 below as suggested by Huawei. |

#### Proposal 2-6r1: (FSK)

Further study the receiver architectures for FSK, with the two examples shown below:

- Example 1: parallel OOK receivers and a comparator circuit

- Example 2: using FM-to-AM detector

  - Alternative 1: a similar architecture as for OOK, except that the envelope detector is replaced by a FM-to-AM detector.

  - Alternative 2: the FM-to-AM detector can implemented in digital domain after ADC.

  - The FM-AM detector, using a frequency discriminator circuit, converts frequency shift into amplitude change, which can be implemented in either analog or digital domain. One example, as shown in the figure below, is a conventional quadrature FM discriminator. It multiplies received frequency modulated signal with a phase shifted version, followed by a low pass filter. The amplitude of the output signal is proportional to the frequency of the input signal.

- Note: Other architectures are not precluded.

| Company | Comments |
| --- | --- |
| Moderator | @ZTE, @Huawei/HiSi, Please address the questions from vivo above. |

OFDM-based signal and receiver have been discussed by Ericsson, QC and Sony, which intends to maximally reuse the existing design of the main radio.

- Sony: “These receivers have a power consumption in the range of 1mW and sensitivity better than -100dBm.”

#### [CLOSED] Q 2-7: (OFDM-based signal/receiver)

Please comment on whether OFDM-based signal and receiver should be further considered for LP WUR and why.

| Company | Comments |
| --- | --- |
| ZTE, Sanchips | More clarification for the OFDM-based signal/receiver are needed, e.g., the generation structure and receiver structure. |
| Spreadtrum | Fine to discuss. |
| TCL | Further study may be required for this proposal |
| Nordic | OK, if proposal allows/includes also DFT-S-OFDMA |
| Panasonic | Even for OOK, our understanding is MC-OOK can still be OFDM-based signal depending on the more detailed design. Same applies to FSK. So we are open to that. |
| Vivo | From our observation, OFDM-based signal and receiver requires power-hungry local oscillator (LO) and PLL for coherent detection, which results in a very high power consumption, thus should not be considered further in this SI. Considering some sources saying that some existing receivers (e.g., Nb-IoT WUS) can operates in 2.1mW. We think targeting 1mW is not promising. |
| Intel | We prefer to not consider this direction since the SID says that ‘this study will not require existing signals to be used as WUS’ |
| Futurewei | We are open in general, but based on low-power receiver architectures literature, we are not sure if this type of receiver architecture can be considered for a targeted power consumption << 1mW. Also, we would like to suggest clarifying the proposal to differentiate between an OFDM receiver and an OOK/FSK LP-WUR for which the OOK/FSK signal is generated using an OFDM-based TX, i.e., OFDM-based signal design. |
| Nokia, NSB | Fine to discuss |
| CATT | We haver reservation on this proposal of OFDM-based signals since the power consumption and complexity of FFT is not the desired component of LP-WUS. |
| OPPO | We are fine with OFDM-based WUS generation. OFDM-based receiver needs further study on power consumption, form factor, complexity, and so on. |
| MediaTek | We are concerned that OFDM-based signal is unlikely to provide meaningful power saving gains comparing to Rel-16/17 power saving schemes. |
| Lenovo | Yes we need to study how to generate MC-OOK from OFDM/DFT-s-OFDM waveform |
| QC | As stated in our tdoc R1-2210012, CP-OFDM waveform for LP-WUS, similar to NB-IoT/eMTC WUS design in Rel-15 and Rel-16, could be just another option to consider. But we expect it would consume higher power than OOK based receiver. |
| Samsung | For further clarification, what’s the meaning of OFDM-based signal? I wonder whether the traditional NR signal is still OFDM-based signal after being converted to time domain and after OOK modulation. Actually, the LP-WUS should be transmitted based on the OFDM mode and modulated by OOK in time domain in our view. By the way, this question seems not necessary because it has nothing to do with any architectures mentioned in proposal 2-1. |
| Everactive | We can safely rule out OFDM receivers for this study based on power consumption reported in literature. Most OFDM receivers are well over 1mW active power. One OFDM receiver has been published below 1mW, which was designed for the MURS band in the US (150MHz). David D. Wentzloff, "Ultra-Low Power Radio Survey," [online]. www.eecs.umich.edu/wics/low_power_radio_survey.html |
| Huawei, Hisilicon | It’s not clear what “OFDM-based” signal refers to, but note that signal design belongs to agenda 9.13.3. In our opinion, the OOK/FSK can also be implemented based on OFDM, e.g. multi-carrier OOK/FSK. Generally, at this stage, we are fine to discuss OFDM-based signal/receiver. |
| SONY | We think that an OFDM-based receiver should be studied. There are various requirements on the LP-WUS, including support for mobility, support for operation in different frequency locations, resilience to co-channel interference etc. These are requirements for a cellular system. We think that these requirements are likely to be well met by an OFDM-based signal (we can include DFT-s-OFDM here). The eMTC / NB-IoT WUS is such an OFDM-based signal. Yes, the power consumption of this receiver architecture / signal type may be higher than the power consumption of other receiver types. If those other receiver types have a lower power consumption but are only suitable for PAN / LAN type deployments, then are they relevant to this study? This is a study item and as part of the study, we think that we should consider the benefits and drawbacks of various reasonable receiver architectures. We are also open to the study of OOK waveforms that are generated within an OFDM signal (as discussed with respect to MC-OOK etc). |
| Ericsson1 | All receiver types should be further studied at this initial stage including OFDM-based receiver. From our perspective, it is important to include at least following in the TR - Achievable power consumption with OFDM based architecture (may be higher than OOK/FSK but how much would be good to document in the same way as other options) - power consumption vs. system aspects (coverage/reliability, latency, overhead) trade-off for this option and other options. |
| Moderator | To answer to some of confusions, the OFDM-based signal/receiver here means that the signal is OFDM-based, and more important, the receiver is also OFDM-based, but the components may be chosen differently to reduce the power consumption. So MC-OOK/FSK (even though it can be generated using an OFDM transmitter) that is received using e.g. one of the architectures in Proposal 2-1/2-6 does not belong to this category. To avoid the confusion, I will use the term “OFDM-based receiver architecture”. There seems to be some divergence among companies in terms of whether we want to pursue this further. OK to study: Spreadtrum, Nordic, Futurewei (?), Nokia/NSB, OPPO (?), QC, Huawei/HiSi (?), SONY, Ericsson No further study: vivo, Intel, CATT, MediaTek, Everactive The opponents think the power consumption achievable by this architecture (e.g. a couple of mW or higher) is too high to be considered. The proponents think we may need such a receiver to achieve all the requirements target. Given that there are a slightly larger number of supporting companies than non-supporting companies, I included Proposal 2-6 below for further discussion. There is no need to comment further here. |

#### [CLOSED] Proposal 2-7: (OFDM-based)

Further study OFDM-based receiver architecture.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | Before studying OFDM-based receiver architecture, the following should be clarified: - The OFDM-based receiver architecture should be clarified clear similar as the zero-IF or other architectures. - The power consumption for the OFDM-based receiver architecture also needs to be clarified especially for the always-on LP-WUS. |
| Nordic | We miss-understood the original question, we do not want to study OFDM-based receiver with coherent detection |
| Futurewei 2 | We are open based on other companies support, but we are more inclined towards not supporting than supporting given the expected power consumption for this type of architecture, the fact that PEI (in RRC IDLE/INACTIVE) and WUS (in RRC Connected) were already addressing this type of architecture, and the fact that the SI does not mandate the use of existing NR signals as LP-WUS. |
| vivo | Thanks moderator for clarifying the confusing point and now it’s clear that MC-OOK/FSK (even though it can be generated using an OFDM transmitter) that is received using e.g. one of the architectures in Proposal 2-1/2-6 does not belong to this category. Further, OFDM-based receiver architecture here is kind of a very high level concept, rather than a receiver architecture type for LP-WUS. Detailed description on involved modules should be given to identify whether companies have common understanding on the OFDM-based receiver architecture. In our opinion, OFDM-based receiver uses high accuracy LO and PLL, as well as other sophisticated hardware modules which support coherent detection of OFDM-based signal, and thus, it’s very difficult to achieve power saving gain compared to R15/R16/R16 power saving scheme. We want to emphasize what stated in the objective of SI: ‘As opposed to the work on UE power savings in previous releases, this study will not require existing signals to be used as WUS’ and ‘Solutions should target substantial gains compared to the existing REL-15/16/17 UE power saving mechanisms’. Furthermore, if we understand it correctly, the main motivation of proposing OFDM receiver is to reuse the existing main radio hardware, then it’s not clear to us whether there exists anything new needs to be studied. To sum up, we suggest not to study of existing OFDM receiver in AI 9.13.2 |
| Panasonic | We are open but may agree on Nordic that this should be confined for non-coherent detection. |
| Intel | (the proposal should be 2-7, right?) If majority companies want to study on OFDM-based receiver, we are fine with it. However, it needs to clarify what is the target time/frequency level of synchronization for OFDM processing at receiver. The related impact to LO/PLL should be clarified too. |
| Samsung | It is not desirable to exclude any architecture at this early stage, but we prefer to deprioritize it considering the high power consumption. We can wait the evaluation results in 9.13.1, and then decide whether this architecture needs further study. |
| SONY | We support further studying OFDAM-based architecture. Agree with Ericsson that, “All receiver types should be further studied at this initial stage including OFDM-based receiver”. The study should identify the advantages and disadvantages of the various receiver architectures before arriving at conclusions. Presumably, this should be proposal 2-7. “2-6” is already used for FSK receivers. |
| MediaTek | No, we may retrigger sequence-based (SSS or TRS/CSI-RS) PEI discussion in Rel-16 again. |
| Huawei, Hisilicon | We are fine to discuss OFDM-based receiver architecture at this initial stage. Companies can further investigate the pros and cons and determine whether the OFDM-based receiver architecture can achieve the requirements for LP-WUR. |
| Everactive | Thank you for clarifying that this study is for an “OFDM-based receiver architecture.” We agree with Nordic, and do not want to study OFDM receivers. |
| Ericsson2 | Support to study. From our perspective, it is good to address the question of suitability (or unsuitability) for OFDM based receiver as part of the SI than simply not including even it for further study from the first meeting. Proposal can be reformulated as below Proposal: - Further study applicability of OFDM-based receiver architecture for LP-WUR/WUS considering at least following - Whether substantial power consumption reduction can be achieved compared to existing solutions |
| QC | As stated in our previous response, CP-OFDM waveform for LP-WUS, similar to NB-IoT/eMTC WUS design in Rel-15 and Rel-16, could be just another option to consider. But we see that it could consume higher power than OOK based receiver. |
| Moderator | Further study: SONY, Huawei/HiSi, Ericsson, QC No further study: Nordic, Futurewei (preferred), vivo, Panasonic (?), MediaTek, Everactive Need further clarification: ZTE/Sanechips, Intel Deprioritize: Samsung Now that the terminology has been clarified, there are more companies who do not see OFDM-based receiver attractive due to the high power consumption. A few companies also pointed out that before we agree to study it, the exact architecture to be investigated should be further clarified, e.g. how it is different from the existing NR main receiver, how to lower the power consumption given that high accuracy LO/PLL are required for coherent detection, etc. So we stop the discussion on this proposal for now. Instead, I am starting a new question inviting the supporting companies to provide more details on the architectures in mind. |

#### Q 2-7A: (OFDM-based signal/receiver)

For the companies who propose to study OFDM-based receiver architecture further, please provide details on the architecture(s), e.g., a diagram, how it is different from the NR receiver, how to reduce the power consumption compared to NR receiver.

| Company | Comments |
| --- | --- |

Assuming certain receiver RF architectures are agreed to be studied further, companies are encouraged to analyze and evaluate the feasibility/performance of the receiver architectures.

#### Proposal 2-8:

For the analysis of a receiver architecture, companies are encouraged to provide the following (when applicable):

- Receiver parameters

  - Presence of a RF LNA, and the corresponding gain, if any

  - Local oscillator

    - Type of oscillator and the corresponding frequency accuracy/drifting

  - Presence of PLL or FLL

  - ADC: sampling rate, bit-width

  - Assumed signal bandwidth and guard band

  - Filter characteristics (what type of filter, order)

  - Assumed frequency band(s)

- Performance metrics

  - Power consumption (and breakdown if available)

  - Noise figure and/or receiver sensitivity

  - Interference rejection capability

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | Generally agree with this proposal. In addition, AMP(BB or IF) should also be included. For each receiver architecture, the parameters may be different. Therefore, these parameters should be given for each receiver architecture. |
| Spreadtrum | We are fine with this proposal. |
| Nordic | OK |
| Panasonic | We are okay. |
| Vivo | We agree to provide the proposed receiver components and referred performance metrics when analysing receiver architectures for feasibility and suggest the following updates: For the analysis of a receiver architecture, companies are encouraged to provide the following (when applicable): - Receiver parameters components - Receiver RF architectures type, e.g., RF envelope detection architecture, Heterodyne architecture, Zero-IF architecture or any others. - Presence of a RF LNA, and the corresponding gain, if any - Presence of an IF amplifier, and the corresponding gain, if any - Local oscillator - Type of oscillator and the corresponding frequency accuracy/drifting - Presence of PLL or FLL - ADC: sampling rate, bit-width - Assumed signal bandwidth and guard band - Filter characteristics (what type of filter, order), including RF, IF, and BB - Assumed frequency band(s) - Performance metrics (according to one specific setup of the receiver components) - Power consumption (and breakdown if available) - Noise figure and/or receiver sensitivity - Interference rejection capability - Data rate |
| Intel | We are supportive to FL proposal |
| Futurewei | We are fine with vivo’s updated proposal. |
| Nokia, NSB | We are fine with Vivo’s updated proposal. |
| CATT | We are OK with the proposal or vivo’s update. |
| OPPO | We are fine with the proposal. |
| MediaTek | Performance metrics should include “data rate” |
| Lenovo | Assumed payload and achievable data rate should be compared for different Rx architecture |
| Sharp | We support the proposal. |
| QC | Companies can report their assumption on those parameters. |
| Samsung | It should be discussed after proposal 2-1 is stable. |
| Everactive | The default for power consumption is the active power consumption of the LP-WUR when receiving a LP-WUS. Additionally, should include the power when off, and a breakdown of power of the receiver blocks. |
| Huawei, Hisilicon | The relationship of this proposal, and proposal 1-2 needs clarifying. They seem to have the same basic intention. This proposal may be a better version to work further on. |
| SONY | “Data rate” should also be included as performance metrics. |
| Moderator | #### Proposal 2-8r1: For the analysis of a receiver architecture, companies are encouraged to provide the following (when applicable): - Details of receiver components parameters - Receiver architecture type - Presence of a RF LNA / IF AMP / BB AMP, and the corresponding gain, if any - Local oscillator - Type of oscillator and the corresponding frequency accuracy/drifting - Presence of PLL or FLL - ADC: sampling rate, bit-width - Assumed signal bandwidth and guard band - RF/IF/BB filter characteristics (e.g. what type of filter, order), if any - Assumed frequency band(s) - Performance metrics - Power consumption during active monitoring/reception and during off state (and breakdown if available) - Noise figure and/or receiver sensitivity - Interference rejection capability - Data rate |
| ZTE, Sanechips | #### Suggest to add ‘at least’ to make the door open as follows: For the analysis of a receiver architecture, companies are encouraged to provide at least the following (when applicable): |
| CMCC | We are fine with FL proposal in general, but still would like to have some further updates as below: For performance metrics, power consumption is preferred to be broken down to show the power consumption of all the necessary parts have been taken into consideration. #### Proposal 2-8r1: For the analysis of a receiver architecture, companies are encouraged to provide the following (when applicable): - Details of receiver components parameters - Receiver architecture type - Presence of a RF LNA / IF AMP / BB AMP, and the corresponding gain, if any - Local oscillator - Type of oscillator and the corresponding frequency accuracy/drifting - Presence of PLL or FLL - ADC: sampling rate, bit-width - Assumed signal bandwidth and guard band - RF/IF/BB filter characteristics (e.g. what type of filter, order), if any - Assumed frequency band(s) - Performance metrics - Power consumption during active monitoring/reception and during off state (and breakdown if available) - For each state (active monitoring/reception/off state), to indicate all the active power consuming parts , including RTC, GPIO, etc. - Noise figure and/or receiver sensitivity - Interference rejection capability - Data rate |
| Nordic | “and breakdown if available” is sufficient |
| Futurewei 2 | #### We agree with the updated proposal 2-8r1. |
| vivo | #### Ok with the proposal |
| Nokia, NSB2 | #### Support the proposal |
| Intel | #### We suggest to clarify in main bullet that this is for both OOK and FSK. #### The second bullet ‘performance metrics’ seems overlap with Proposal 1-2 |
| Samsung | #### We would like to further clarify the meaning of ‘gain’ in the second sub-bullet. Does it mean the absolute/relative power consumption or receiver sensitivity gain due to the presence of RF LNA/IF AMP/BB AMP? |
| SONY | #### OK with proposal. |
| MediaTek | #### Ok. |
| Huawei, Hisilicon | #### We suggest to add baseband processing in the details of receiver components, as this is likely to differ among architectures: - Baseband processing (sequence correlation detection / decoding, other signal processing, if any) For the analysis of a receiver architecture, companies are encouraged to provide the following (when applicable): - Details of receiver components parameters - Receiver architecture type - Presence of a RF LNA / IF AMP / BB AMP, and the corresponding gain, if any - Local oscillator - Type of oscillator and the corresponding frequency accuracy/drifting - Presence of PLL or FLL - ADC: sampling rate, bit-width - Assumed signal bandwidth and guard band - RF/IF/BB filter characteristics (e.g. what type of filter, order), if any - Assumed frequency band(s) - Baseband processing - Sequence correlation detection / decoding - Support of any other signal processing - Performance metrics - Power consumption during active monitoring/reception and during off state (and breakdown if available) - Noise figure and/or receiver sensitivity - Interference rejection capability - Data rate |
| Everactive | #### Ok with the proposal |
| QC | #### We are OK with the FL proposal. |
| Moderator | #### Updated version with diff marks on top of r1: #### Proposal 2-8r2: For the analysis of a receiver architecture, companies are encouraged to provide at least the following (when applicable): - Details of receiver components - Receiver architecture type - Presence of a RF LNA / IF AMP / BB AMP, and the corresponding gain, if any - Local oscillator - Type of oscillator and the corresponding frequency accuracy/drifting - Presence of PLL or FLL - ADC: sampling rate, bit-width - Assumed signal bandwidth and guard band - RF/IF/BB filter characteristics (e.g. type of filter, order), if any - Baseband processing (e.g., sequence correlation detection / decoding, other signal processing, if any) - Assumed frequency band(s) - Performance metrics - Power consumption during active monitoring/reception and during off state (and breakdown if possible available) - Noise figure and/or receiver sensitivity - Interference rejection capability - Data rate @Intel, this is a generic proposal and not necessarily tied to a particular architecture. It is probably better not to explicitly mention the modulation. The main bullet has “when applicable” in it. @Samsung, “gain” refers to the power gain of an amplifier, which improve the receiver sensitivity, not the power consumption. @Huawei/HiSi, it is not clear to me why you suggest removing interference rejection capability. |

## Other

In this section, the proposals that are (better to be) discussed in other agenda items (based on moderator’s assessment), or the proposals that are de-prioritized in this meeting are captured. Note that there are also some proposals not captured here, which are meant to be de-prioritized in this meeting.

There have been some proposals on the design targets on the power consumption, data rate and sensitivity/coverage. These are de-prioritized to avoid duplicated discussion because they are also being discussed under AI 9.13.3.

- Power consumption

  - Vivo: tens of uw ~ hundreds of uw

  - CATT: < [100] µW

  - Intel: 100uW – 1mW

  - MTK: less than 1mW, e.g., around 0.1mW to 0.5mW

  - QC: 1mW

- Sensitivity

  - Vivo: -80dBm ~ -100dBm

  - CATT: Receiver sensitivity/Maximum coupling loss of LP-WUS – [-80] dBm/ [126] dB

  - Intel: Strive to a sensitivity of LP-WUR that is not worse than the bottleneck channel of existing NR UE

  - Panasonic: should not require re-planning of the cell deployment

  - Samsung: The coverage of LP-WUS should be consistent with the legacy signal of the main receiver

  - Ericsson: strive to support similar coverage for LP-WUS as for Paging PDCCH

  - QC: the same coverage as other regular NR channels/signals is highly desirable

- Data rate

  - Vivo: tens of kbps ~ hundreds of kbps

  - CATT: Minimum achievable data rate – [160] bps

The following proposal may be better to be discussed as part of WUS signal design in AI 9.13.3.

| EURECOM | Proposal 1: Consider receiver architectures for hierarchical decoding of the WUS. |
| --- | --- |

The following proposal are de-prioritized in this meeting.

| TCL | Proposal 1: RAN1 to clarify the synchronization requirements of LP-WUR with the network, before receiving the WUS signal. [Moderator’s comment] This should be considered as part of the evaluation and design. |
| --- | --- |
| CATT | UE capability and transfer of UE capability - UE will report its supported category of low-power wakeup receiver in the UE capability. Network configuration - The network would configure the UE wakeup mechanism by low-power wakeup receiver if network supports the category of low-power wakeup receiver in the deployment. |

# Reference

- RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, RAN#97e, September 2021.

- R1-2208379	Low Power WUS Receiver Architectures Considerations and Modeling	FUTUREWEI

- R1-2208418	On architectures of LP-WUS receiver	Huawei, HiSilicon

- R1-2208481	Low Power WUS receiver architecture 	TCL Communication Ltd.

- R1-2208573	Discussion on low power WUS receiver architectures	Spreadtrum Communications

- R1-2208669	Discussion on low power wake-up receiver architecture	vivo

- R1-2208687	Discussion on LP-WUS receiver architectures	InterDigital, Inc.

- R1-2208699	Low Power WUS receiver architectures	Nokia, Nokia Shanghai Bell

- R1-2208844	Discussion on low power WUS receiver	OPPO

- R1-2208961	Design consideration of Low-Power WUS receiver	CATT

- R1-2209076	Discussion on LP-WUS receiver architecture	Intel Corporation

- R1-2209200	LP-WUS receiver architectures	ZTE, Sanechips

- R1-2209503	Low power WUS receiver architectures	MediaTek Inc.

- R1-2209606	On low power wake-up receiver architectures	Apple

- R1-2209622	Receiver architectures for low power WUS	Rakuten Symphony

- R1-2209634	Discussion on Low power WUS receiver architectures	Panasonic

- R1-2209666	Discussion on the receiver architecture for low power WUS	Lenovo

- R1-2209757	Receiver architecture for LP-WUS	Samsung

- R1-2209863	Low power WUS receiver architectures	Ericsson

- R1-2210011	Receiver architecture for LP-WUS	Qualcomm Incorporated

- R1-2210052	Discussion on Low power WUS receiver architectures	EURECOM

- R1-2210198	On LP-WUS architecture	Nordic Semiconductor ASA

- R1-2210223	On LP-WUS receiver architectures	Sony

# Appendix: Proposals from contributions

The detail of the baseband digital processing is not shown in the architecture. It depends on how WUS is designed. E.g. if we use a similar idea as LP WUR in 802.11ba, there is a preamble followed by the WUS. The WUR needs to perform correlation to detect the preamble first, and then detection/decoding of the WUS payload. This level of details can be discussed later after more clarity on WUS design.

## R1-2208379	Low Power WUS Receiver Architectures Considerations and Modeling	FUTUREWEI

Observation 1: In-band selectivity of the uncertain IF receiver architecture is limited by wide IF bandwidth required to deal with the LO uncertainty. Observation 2: Interference rejection and In-band selectivity of the synchronized switching receiver architecture is limited by RF FE band-pass filtering capability due to envelope detection at RF. Observation 3: In-band selectivity of the sub-sampling & uncertain IF receiver architecture is limited by wide IF bandwidth required to deal with the LO uncertainty, which can be improved by a period-based oscillator calibration circuit. Observation 4: Interference rejection and In-band selectivity of an RF envelope detection (double-sampling) receiver architecture can be improved using a 2-tone transmission scheme/signal design. Observation 5: Interference rejection and In-band selectivity of a mixer-first (uncertain-IF) receiver architecture can be improved using a combination of LC-DCO and distributed high-Q N-PPM filtering at the cost of increase in power consumption. Observation 6: Low-power receiver architectures of power consumption around 100μW such as dual uncertain-IF and 2-tone reception show reasonable sensitivity and good selectivity with effective bandwidths <4 MHz. Observation 7: Low-power receiver architectures of power consumption <60μW such as uncertain-IF, sub-sampling, and double-sampling may show reasonable sensitivity but at poor selectivity with effective bandwidths >10 MHz. Proposal 1: the study of the modeling of the wake-up receiver should - Consider the KPIs from system perspectives - Decide whether a single or a small number of models should be adopted - Reflect via a set of parameters the general design aspects of the receiver, including at least power consumption, sensitivity, and interference rejection / selectivity - Study the valid value range of the receiver parameter set and select a small number of sets (with different tradeoffs) for performance evaluation and signal design Proposal 2: A general LP-WUR model can consist of a filtering block for selectivity, an envelope detection block for non-linearity characterization, a single/multi-bit ADC block, and a DSP block for sequence correlation or message parsing. In general, examined low-power receiver architectures can be categorized as mixer-first architectures, such as the uncertain-IF, the sub-sampling, and the dual uncertain-IF architectures; and envelope detection first architectures, such as the double-sampling and the 2-tone reception architectures. Figure 2: An Example of Uncertain IF Receiver Architecture [5]. Figure 3: An Example of Synchronized Switching/Double-Sampling Receiver Architecture [6]-[7]. Figure 4: An Example of Sub-Sampling Receiver Architecture [8]. Figure 5: An Example of a 2-Tone Reception Envelope Detection Receiver Architecture [9]. Figure 6: A Representation of the Dual Uncertain-IF Receiver Architecture [10]. Figure 7: Wake-Up Receiver and Signal Design Evaluation Model.

## R1-2208418	On architectures of LP-WUS receiver	Huawei, HiSilicon

- Receive architectures with the following design aspects are studied for LP-WUR: - Use low quality LNA (the price is higher noise figure); - Use lower accuracy LO without PLL, for example, ring-type LO; - Avoid high-resolution and high sampling rate quantizer; - Avoid complicated baseband processing. - The LP-WUS needs to have a suitable design to permit simplified receiver architectures. - RAN1 to study: - Evaluating the power consumption of potential LP-WUR architectures to build the power model for calculating the power saving gain; - Determining the link and system level assumption for receiving LP-WUS based on the implementation of potential LP-WUR receivers, including: - Maximum time and frequency error; - Bit-width and sampling rate of ADC for digital baseband processing - Evaluating the coverage of LP-WUS based on the noise figure of potential LP-WUR receivers. - OOK, FSK can be potential LP-WUS candidate modulation schemes for low power wake-up signal modulation. - Study receivers (a) with a single down-conversion operation, and (b) without LO as potential LP-WUR architectures. Figure 2 OOK signal receiving scheme Figure 3 FSK signal received by FM-AM detector Figure 4 Low-IF implementation for wake-up receiver with single down-conversion operation Figure 5 Receiver implementation without LO for wake-up receiver

## R1-2208481	Low Power WUS receiver architecture 	TCL Communication Ltd.

Observation 1: The main radio of the LP-WUR still needs SSB burst or TRS to synchronize with the network before data, signalling reception. Observation 2: The non-coherent detection of low power wake up receiver may reduce the power consumption. Proposal 1: RAN1 to clarify the synchronization requirements of LP-WUR with the network, before receiving the WUS signal.

## R1-2208573	Discussion on low power WUS receiver architectures	Spreadtrum Communications

Observation 1: The power consumption and the sensitivity are tradeoff. Observation 2: The sensitivity may affect the resource coverage to meet the coverage requirement, and mobility. Observation 3: The improvement of the sensitivity is limited by the noise factor of the LNA. Proposal 1: Study the architectures in terms of the tradeoff between the power consumption and the sensitivity, especially for LNA and mixer. LP-WUR architectures Direct RF detection Frequency conversion Non-LNA TRF Mixer-first LNA-first Certain-IF UnCertain-IF Certain-IF UnCertain-IF Figure 1: Classification of the LP-WUR Architectures IMN SRF ED A D VBB BB Amp. Figure 2: The non-LNA DRF architecture The TRF is shown in the following figure. IMN SRF ED A D VBB BB Amp. LNA Figure 3: The TRF architecture IMN SRF ED A D VBB BB Amp. PLL vco Mix IF amp/filter Figure 4: The mixer-first FC architecture with (or without) a PLL IMN SRF ED A D VBB BB Amp. LNA PLL vco Mixer IF amp/filter Figure 5: The LNA-first FC architecture with (or without) a PLL

## R1-2208669	Discussion on low power wake-up receiver architecture	vivo

Observation 1 Design on low-power WUR architecture is a trade-off of power consumption, sensitivity and data rate. Observation 2 Due to demanding a band specific high-Q RF BPF, the receiver architecture with amplitude detection at RF is more suitable for devices supporting single band. Observation 3 The receiver architecture with amplitude detection at RF brings extreme low power consumption with limited sensitivity. Observation 4 For low-power receiver architecture with amplitude detection at IF, the power consumption can be reduced by replacing a high accuracy LO with a medium accuracy LO, and the frequency offset of the LO can be further studied. Observation 5 For low-power receiver architecture with amplitude detection at BB, low-power solution on flicker noise and DC offset issue should be studied. Observation 6 Analog RF filter is used for out-of-band interference rejection. Proposal 1 Adopt the following terminology for future discussion, - Main radio：the Tx/Rx module operating for legacy system - Low-power WUR: the Rx module operating for receiving/processing low-power WUS Proposal 2 The main radio and low-power WUR will exchange information between each other, such as - Low-power WUR can get initial configurations before work from the main radio - Low-power WUR can indicate ‘wake-up’ to the main radio - Low-power WUR can pass additional messages from LP-WUR to the main radio which are processed and parsed in the main radio but agnostics to the low-power WUR Proposal 3 Study non-coherent detection based low-power WUR receiver architectures supporting at least ASK (e.g., OOK) detection which satisfy design targets in terms of power consumption, sensitivity and data rate. Proposal 4 Study the feasibility of low-power receiver architecture with amplitude detection at RF for the single band devices targeting extreme low power consumption with limited sensitivity. Proposal 5 Study the feasibility of low-power WUR architecture with mixer-first followed by amplitude detection i.e., amplitude detection at IF and amplitude detection at BB, based on the design targets, e.g., - Power consumption : tens of uw ~ hundreds of uw - Sensitivity: -80dBm ~ -100dBm - Data rate: tens of kbps ~ hundreds of kbps Proposal 6 RAN1 sends the candidate receiver architectures with necessary information (description on the receiver architecture, power consumption, sensitivity, data rate, and etc.) to RAN4 by the end of Nov meeting. Proposal 7 Study the followings related to the analog IF/BB filter, i.e., filter bandwidth by considering the bandwidth of low-power WUS, ACI cancellation requirements, guardband allocation between low-power WUS and adjacent channel as well as power consumption budget. Proposal 8 Study multi-bit ADC, i.e., resolution and sampling rate by considering both power consumption budget and detection performance. Figure 3 Low-power receiver architecture with amplitude detection at RF Figure 4 Low-power receiver architecture with amplitude detection at IF Figure 5 Low-power receiver architecture with amplitude detection at BB Table 1 comparison of low power receiver architectures Modulation; Power consumption; RF sensitivity; Interference rejection. Type 1: amplitude detection at RF; ASK (OOK); uw[4]~tens of uw; -50dBm[4]~-70dBm (LNA); High-Q RF filter, multi-bit ADC. Type 2:amplitude detection at IF; ASK (OOK); tens of uw[5] ~ mw[6]; -70dBm[5]~-110dBm[6]; RF filter, high-Q IF filter, multi-bit ADC. Type 3:amplitude detection at BB; ASK (OOK); tens of uw [7]~ hundreds of uw[8]; -70dBm[7]~-90dBm[8]; RF filter, high-Q BB filter, multi-bit ADC.

## R1-2208687	Discussion on LP-WUS receiver architectures	InterDigital, Inc.

Proposal 1: Consider a receiver with a received signal strength indicator (RSSI) circuit as a candidate receiver for LP-WUS. Fig. 1 Architecture of the RSSI circuit Fig. 2 Architecture of the modified 10-stage Dickson based passive rectifier

## R1-2208699	Low Power WUS receiver architectures	Nokia, Nokia Shanghai Bell

Observation 1: The optimum LP-WUR architecture may vary depending on the use-case. Observation 2: For a LP-WUR architecture that reuses the main radio 5G RF section but with diversity disabled, the “ON” power consumption can be expected to be in the range of 0.1-0.25 of the 5G main radio. Observation 3: For a LP-WUR architecture that reuses the main radio 5G RF section, the power consumption of the LP-WUS detection cycle can be expected to be similar to the current main radio power consumption for a DRX paging cycle, with the main difference being due to the alternative baseband processing applied by the LP-WUR. Observation 4: To maximise power savings from the LP-WUR, a save/restore feature can be implemented, for fast and simple re-programming of LP-WUR (frequency synthesis, frontend setting, LNA and more). Proposal 1: The SI considers LP-WUS and LP-WUR designs that could reduce how often the main radio needs to be woken up for coverage and mobility related measurements, in order to reduce the overall power consumption of the device. Observation 5: For a LP-WUR architecture that reuses the main radio 5G RF section but with reduced gain and diversity disabled, the “ON” power consumption can be expected to be in the range of 0.05-0.1 of the 5G main radio. Observation 6: Power consumption can be traded off against sensitivity performance. Observation 7: For DRX operation, the LP-WUR choice of clock needs to consider the trade off between higher clock accuracy and corresponding clock source power consumption, with the support of longer duty cycles and the corresponding higher overall LP-WUR operation power consumption gain. Observation 8: If a reference clock with large frequency tolerances is assumed, additional guard band between the 5G and LP-WUS should be considered Figure 1: Example of a multi band low-power wake-up receiver architecture Figure 2: Multiband LP-WUR architecture with reduced gain

## R1-2208844	Discussion on low power WUS receiver	OPPO

Observation 1: LP WUR with RF envelope detector architecture has lowest power consumption, worst interference resiliency and worst sensitivity. Observation 2: LP WUR with mixer-first heterodyne architecture has highest power consumption, best interference resiliency and best sensitivity. Observation 3: LP WUR with mixer-first zero-IF architecture has moderate power consumption, interference resiliency and sensitivity. Observation 4: The target power consumption, coverage and interference resiliency dominate the evaluation of low-power wake-up receiver architectures. Fig.1 RF envelope detection architecture Fig.2 Mixer-first heterodyne architecture Fig.3 Mixer-first zero-IF architecture

## R1-2208961	Design consideration of Low-Power WUS receiver	CATT

- Proposal 1: The UE power consumption of preparation and detection of wakeup signal/channel should be significantly lower comparing to the wakeup indication by DCI formats 2_6 and 2_7 in the target study of low power wakeup receiver. - Proposal 2: The target power consumption and the receiver sensitivity/maximum coupling loss of the low-power wakeup at a given data rate should be defined as follows, - UE power consumption < [100] µW - Receiver sensitivity/Maximum coupling loss of LP-WUS – [-80] dBm/ [126] dB - Minimum achievable data rate – [160] bps - Proposal 3: The architecture and technologies of low-power wakeup receiver is UE implementation specific. The standard should focus on the discussion in the following aspects - Categorized low-power wakeup receiver - The low-power wakeup receiver implemented by the UE should be classified into multiple categories with each categories containing the achievable minimum receiver sensitivity and the maximum power consumption in the specification. - UE capability and transfer of UE capability - UE will report its supported category of low-power wakeup receiver in the UE capability. - Network configuration - The network would configure the UE wakeup mechanism by low-power wakeup receiver if network supports the category of low-power wakeup receiver in the deployment. Figure 2 An example of 4.5nW Receiver with -69dBm Sensitivity Figure 3 An example of 7-μW 2.4-GHz Wake-Up Receiver with -80 dBm Sensitivity and High Co-Channel Interferer Tolerance Figure 4 An example of a 2.4 GHz Receiver with dual-IF with -97 dBm Receiver Sensitivity

## R1-2209076	Discussion on LP-WUS receiver architecture	Intel Corporation

Observation 1: For a receiver using RF envelop detection - It can achieve lower complexity/power consumption than other receiver architectures. - It has worst sensitivity than other receiver architectures since it cumulates significant noise/interference including flicker noise. Observation 2: For a heterodyne receiver with IF envelop detection - It has better sensitivity and interference resilience - It consumes more power due to the LO/PLL - Image rejection is required - Low IF receiver architecture can be considered Observation 3: For a zero IF receiver - It simplifies the signal processing for low complexity/power consumption. - No issue of image interference - It has the issue on DC offset and flicker noise Proposal 1: - The target power consumption of LP-WUS is selected in range 100uW – 1mW - Strive to a sensitivity of LP-WUR that is not worse than the bottleneck channel of existing NR UE. Proposal 2 - Study receiver architecture which considers single or mulit-carrier -OOK/ FSK as modulation scheme for LP-WUR. Proposal 3 - The following receiver architecture should be considered in the study of LP-WUR - Receiver using RF envelop detection - Heterodyne receiver with IF envelop detection, specifically low IF receiver - Zero IF receiver Figure 2: Receiver using RF envelop detection Figure 3: Heterodyne receiver with IF envelop detection Figure 4: Zero IF receiver

## R1-2209200	LP-WUS receiver architectures	ZTE, Sanechips

Observation 1: For Zero-IF receiver, IF receiver and RF receiver for OOK-waveform based LP-WUS reception, - Zero-IF receiver has benefits of small size and is easy for monolithic integration; - IF receiver has the best receiver sensitivity; - RF receiver has the lowest cost. Observation 2: Compared with OOK based waveform, FSK based waveform can support higher data rate, however, more frequency bandwidth is also required. Proposal 1: The design on receiver architectures for LP-WUS reception could depend on the following factors, such as LP-WUS waveform, receiver complexity, power consumption and receiver sensitivity. - At least OOK based waveform for LP-WUS could be taken as the starting point and FSK also can be considered. - Zero-IF receiver Figure 1: Architecture of Zero-IF receiver Figure 2: Architecture of IF receiver Figure 3: Architecture of RF receiver Figure 5: Reception architecture of 2FSK

## R1-2209503	Low power WUS receiver architectures	MediaTek Inc.

Proposal 1 Mixer-first zero-IF architecture can be considered a starting point to determine target power consumption, interference resiliency, and receiver sensitivity. Proposal 2 The power consumption target can be less than 1mW, e.g., around 0.1mW to 0.5mW. Proposal 3 Low-power receivers usually use low-complexity devices, and the tradeoff between performance and power savings should be evaluated. Proposal 4 Evaluate the performance impact caused by low-power consumption devices, e.g., passive mixer, local oscillator, low pass filter, and N-bit ADC for the mixer-first zero-IF architecture. Proposal 5 Consider the performance and power consumption tradeoff parameters: noise figure, modulation, signal bandwidth, and repetition/coding gain. Figure 1: illustrations of LP-WUR architectures

## R1-2209606	On low power wake-up receiver architectures	Apple

Proposal 1: In-band signaling for LP WUS is prioritized in the study. Proposal 2: The supported band(s) and its impact on WUR architecture and the corresponding power consumption should be studied. Proposal 3: WUR should provide sufficient ACS capability to handle cellular environment. Proposal 4: Study OOK as one of the candidate modulation schemes for WUS. Proposal 5: For OOK, heterodyne architecture and zero-IF architecture are considered further, while RF envelope detector can be deprioritized. Proposal 6: RRM measurement enhancements should be considered in the receiver architecture discussion. - Type 1: RF envelope detector - Type 2: Heterodyne architecture (low-IF) - Type 3: zero-IF architecture Figure 1 Different types of receiver architectures for OOK

## R1-2209622	Receiver architectures for low power WUS	Rakuten Symphony

Proposal 1: Mixer-first zero-IF receiver architecture is assumed for the study. Direct envelope detection (ED). The drawbacks of the direct ED receiver are, due to lack of filtering at the RF, low channel selectivity and interference resiliency. In addition, lack of pre-ED gain limits the performance. It has the smallest power.. Mixer-first heterodyne. This receiver has better sensitivity and interference resilience. However, LO generation and IF operation require more power.. Mixer-first zero IF. In this receiver, a mixer translates the RF signal to a lower frequency, which is easier and more power efficient to filter and amplify. The power of LO generation tends to dominate the power consumption. This architecture can still achieve sub-mW power and high sensitivity..

## R1-2209634	Discussion on Low power WUS receiver architectures	Panasonic

Figure.1 Left: Brief illustration on interaction between LP-WUR and main radio. Right: Signal processing flow of some typical architecture candidates for LP-WUR.

## R1-2209666	Discussion on the receiver architecture for low power WUS	Lenovo

Different receiver architecture such as envelope detector based on direct envelope detector, zero-IF, low-IF and heterodyne can be selected for initial evaluation together with the candidate waveform such as MC-OOK and FSK. Proposal 1: Evaluate different receiver architecture for power consumption, data rate, sensitivity, interference rejection, coverage

## R1-2209757	Receiver architecture for LP-WUS	Samsung

Proposal 1: The coverage of LP-WUS should be consistent with the legacy signal of the main receiver. Proposal 2: The power consumption of the separate WUR should be reduced dramatically compared with main radio. Proposal 3: Study synchronization and interference issue in LP-WUS reception. Proposal4: Study the impact of the tradeoff between sensitivity, data rate and power consumption in the process of WUR designing. Proposal 5: Study various types of receivers (e.g., envelop detector, heterodyne detector) for LPWUS reception in RF end. Proposal 6: Considering the coverage and the decoding issue of LP-WUS, RF+BB architecture seems more suitable for receiving LP-WUS signals.

## R1-2209863	Low power WUS receiver architectures	Ericsson

Proposal 1 WUR architecture should support low power consumption and acceptable sensitivity. Proposal 2 WUR architecture should strive to support similar coverage for LP-WUS as for Paging PDCCH. Proposal 3 Following LP-WUR architecture options can be considered for further study i. OFDM based receiver capable of receiving OFDM based signals similar to existing NR signal(s) ii. receiver capable of decoding multi-carrier OOK/FSK based LP-WUS Proposal 4 At least the following aspects should be considered when studying LP-WUR architectures i. Power consumption (e.g., relative to main radio). ii. Noise Figure, other receiver impairments (e.g., clock accuracy/drift), and achievable detection sensitivity for different LP-WUS designs. iii. Suitability for operation in different NR bands. iv. Ability to handle adjacent NR transmissions with minimum/no guard-bands.

## R1-2210011	Receiver architecture for LP-WUS	Qualcomm Incorporated

Observation 1: Following aspect needs to be studied when assessing receiver architectures for LP-WUR. - Coverage/sensitivity - Power consumption - Complexity/Cost - RF requirement - Implementation Observation 2: The goal of UE architecture study is to investigate the feasibility of different architecture options and identify whether they can meet 3GPP LP-WUR design target. Proposal 1: 3GPP shall not mandate the implementation of certain receiver architecture. Proposal 2: 3GPP RAN1 determines the design target of LP-WUR for WAN application. Observation 3: LP-WUS having the same coverage as other regular NR channels/signals is highly desirable. Observation 4: 1mWms of energy consumption for LP-WUS monitoring every 2.56sec is equivalent to 0.4uW of additional average power consumption (0.4uW = 1mWms/2560ms). Observation 5: 1mW is a reasonable power consumption budget for R18 LP-WUR for WAN application. Figure 1 Zero IF architecture Figure 2 Tunned RF architecture Figure 3 Super regenerative receiver architecture Figure 4 Low IF architecture Figure 5 Uncertain IF architecture

## R1-2210052	Discussion on Low power WUS receiver architectures	EURECOM

Proposal 1: Consider receiver architectures for hierarchical decoding of the WUS.

## R1-2210198	On LP-WUS architecture	Nordic Semiconductor ASA

Observations-1: Designs including non-tunable HQ-RF filters are not suitable for cellular systems. Designs assuming those should not be further considered. Observation-2: LC-DCO/LC-VCO alone consumes more than what ring-oscillator-based LP-WUS design would consume in RF part. Proposal-1: Consider restricting LP-WUS location in the middle of NR carrier, this relaxing requirements on filtering and phase noise while possibly providing required sensitivity to achieve Required SNR at agreed MIL. Figure 1 Proposed architecture

## R1-2210223	On LP-WUS receiver architectures	Sony

Proposal 1: RAN1 studies LP-WUR architectures at a high level, including at least: - Rectifier-first LP-WUR - Mixer-first LP-WUR - OFDM-based receiver Proposal 2: For the studied LP-WUR architectures, RAN1 should consider at least the following attributes: - Power consumption - Sensitivity - Supportable signal formats, including supported modulations - Robustness to co-channel interference - Robustness to frequency offset - Mobility support - Operation at different carrier frequencies