3GPP TSG RAN WG1 #110bis-e						R1-2210500

e-Meeting, October 10th – 19th, 2022

Agenda item:	8.1

Source: 	Moderator (Huawei)

Title: 	Moderator Summary for Rel.17 NR FeMIMO maintenance: CSI Enhancement (Round 1)

Document for:	Discussion and Decision

- Introduction

The moderator summary of maintenance-related issues raised for Rel.17 NR_FeMIMO CSI enhancement in RAN1 110bis-e is given below, with following 5 tdocs. Company’s comments are requested before Wednesday UTC 15:00, i.e. the end of GTW session.

In general the target of email discussion is to endorse changes by this Thursday. I will provide final CR draft later and upload the CR by this Friday.

- Maintenance issues

R1-2208763	Draft CR on UE capability parameter for mTRP-CSI in TS 38.214	ZTE

R1-2208916	Clarification of LI reporting for Further Enhanced Type II Port Selection CSI feedback	CATT

R1-2209550	Clarification of LI bitwidth and mapping order of CSI fields for Further Enhanced Type II Port Selection CSI feedback	CATT

R1-2209846	Correction on codebook mode configuration for Rel-17 NCJT CSI measurement	Huawei, HiSilicon

R1-2209847	Discussion on the codebook mode configuration for NCJT CSI measurement Huawei, HiSilicon

- Issue #1: UE capability parameter update  (R1-2208763 38.214, ZTE)

| Company | Company inputs (if any) |
| --- | --- |
| Mod | Proposal: Agree with following TP update in alignment CR for 38.214 - If a CSI-ReportConfig is configured with codebookType set to 'typeI-SinglePanel' and the corresponding CSI-RS Resource Set for channel measurement is configured with two Resource Groups and N Resource Pairs, OCPU=X⋅N+M, where X is the number of CPUs occupied by a pair of CMRs subject to mTRP-CSI-numCPU-r17 and M is defined in clause 5.2.1.4.2. Please comment if anything is missing or any further changes/polishing are preferred. |
| Apple | We agree with the proposed CR |
| vivo | Agree |
| Samsung | Support |
| LG | Support |
| Lenovo | Support |
| CATT | Support |

- Issue #2: Clarification of LI reporting for FeTypeII (R1-2208916 38.214, R1-2209550 38.212, CATT)

| Company | Company inputs (if any) |
| --- | --- |
| Mod | Proposal: Agree with following TP updates for 38.214 A CSI Reporting Setting is said to have a wideband frequency-granularity if - reportQuantity is set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI', cqi-FormatIndicator is set to 'widebandCQI' and pmi-FormatIndicator is set to 'widebandPMI', or - reportQuantity is set to 'cri-RI-PMI-CQI', codebookType is set to 'typeII-PortSelection-r17' with M=1 and cqi-FormatIndicator is set to 'widebandCQI', or - reportQuantity is set to 'cri-RI-i1' or - reportQuantity is set to 'cri-RI-CQI' or 'cri-RI-i1-CQI' and cqi-FormatIndicator is set to 'widebandCQI', or - reportQuantity is set to 'cri-RSRP' or 'ssb-Index-RSRP' or 'cri-SINR', or 'ssb-Index-SINR' or 'cri-RSRP-CapabilityIndex' or 'ssb-Index-RSRP-CapabilityIndex' or 'cri-SINR-CapabilityIndex', or 'ssb-Index-SINR-CapabilityIndex' If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-LI-PMI-CQI', UE does not expect the CSI-ReportConfig to be configured with higher layer parameter codebookType set to 'typeII-r16' or 'typeII-PortSelection-r16' or 'typeII-PortSelection-r17'. Please comment if anything is missing or any further changes/polishing are preferred. |
| Apple | We agree with the proposed CR |
| vivo | Agree |
| Samsung | Support |
| LG | Support |
| Lenovo | Support |
| CATT | Support |