3GPP TSG RAN WG1 Meeting # 110bis-e	R1-2210640

# E-meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | - | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on priority of CG-UCI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (OPPO), ITRI, Huawei, HiSilicon, LG Electronics, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2022-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to RAN1#109e agreement, when cg-UCI-Multiplexing is enabled, the CG-UCI has the same priority as the PUSCH. Agreement When cg-UCI-Multiplexing is enabled, for PUSCH with CG UCI multiplexing with HARQ-ACK, if any, - CG-UCI has the same priority as the PUSCH. - Treat the CG-UCI of a certain priority as if a HARQ-ACK of the same priority. - Joint encode CG-UCI with HARQ-ACK of the same priority if it exists. - Then reuse the existing multiplexing rules. However, according to current specification, the rate matching procedure should consider the case of CG-UCI associated with priority index 1 when the PUSCH is associated with priority index 0 which is not consistent with the agreement and would introduce unnecessary UE behavior on the rate matching procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To remove the condition of rate matching for CG-UCI associated with priority index 1 when the PUSCH is associated with priority index 0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary UE behavior in the rate matching procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.4.1.6, 6.3.2.4.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR is based on RAN1’s common understanding, which has no impact on UE behavior. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

6.3.2.4.1.6	UCI with different priority indexes

< Unchanged parts are omitted >

If uci-MuxWithDiffPrio is configured, and HARQ-ACK bits associated with priority index 0, HARQ-ACK bits associated with priority index 1 and/or CG-UCI associated with priority index 1, and CSI part 1 if any are transmitted on a PUSCH:

-	Perform rate matching for HARQ-ACK with priority index 1 according to clause 6.3.2.4.1.1, by taking HARQ-ACK with priority index 1 as HARQ-ACK and replacing βoffsetPUSCH by βoffsetHARQ-ACK-HP, if HARQ-ACK bits associated with priority index 1 are transmitted without CG-UCI associated with priority index 1.

-	Perform rate matching for CG-UCI with priority index 1 according to clause 6.3.2.4.1.4, if CG-UCI associated with priority index 1 is transmitted without HARQ-ACK bits associated with priority index 1.

-	Perform rate matching for CG-UCI with priority index 1 and HARQ-ACK with priority index 1 according to clause 6.3.2.4.1.5, if both CG-UCI associated with priority index 1 and HARQ-ACK bits associated with priority index 1 are transmitted, by taking HARQ-ACK with priority index 1 as HARQ-ACK and replacing βoffsetPUSCH by βoffsetHARQ-ACK-HP.

-	If CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 1,

-	Perform rate matching for CSI part 1 according to clause 6.3.2.4.1.2, by taking HARQ-ACK with priority index 1 if any as HARQ-ACK, and taking CG-UCI associated with priority index 1 if any as CG-UCI.

-	Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.1.3, by taking HARQ-ACK with priority index 0 as CSI part 2 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and taking HARQ-ACK with priority index 1 if any as HARQ-ACK, and taking CG-UCI associated with priority index 1 if any as CG-UCI.

-	Otherwise,

-	Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.1.2, by taking HARQ-ACK with priority index 0 as CSI-part 1 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and taking HARQ-ACK with priority index 1 if any as HARQ-ACK, and taking CG-UCI associated with priority index 1 if any as CG-UCI.

-	Perform rate matching for CSI part 1 according to clause 6.3.2.4.1.3, by taking CSI part 1 as CSI part 2 and replacing βoffsetPUSCH by βoffsetCSI-part1, taking HARQ-ACK with priority index 0 as CSI-part 1 and taking HARQ-ACK with priority index 1 as HARQ-ACK, if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 0.

< Unchanged parts are omitted >

6.3.2.4.2.6	UCI with different priority indexes

< Unchanged parts are omitted >

If uci-MuxWithDiffPrio is configured, and HARQ-ACK bits associated with priority index 0, HARQ-ACK bits associated with priority index 1 and/or CG-UCI associated with priority index 1, and CSI part 1 if any are transmitted on a PUSCH:

-	Perform rate matching for HARQ-ACK with priority index 1 according to clause 6.3.2.4.2.1, by taking HARQ-ACK with priority index 1 as HARQ-ACK and replacing βoffsetPUSCH by βoffsetHARQ-ACK-HP, if HARQ-ACK bits associated with priority index 1 are transmitted without CG-UCI associated with priority index 1.

-	Perform rate matching for CG-UCI with priority index 1 according to clause 6.3.2.4.2.4, if CG-UCI associated with priority index 1 is transmitted without HARQ-ACK bits associated with priority index 1.

-	Perform rate matching for CG-UCI with priority index 1 and HARQ-ACK with priority index 1 according to clause 6.3.2.4.2.5, if both CG-UCI associated with priority index 1 and HARQ-ACK bits associated with priority index 1 are transmitted, by taking HARQ-ACK with priority index 1 as HARQ-ACK and replacing βoffsetPUSCH by βoffsetHARQ-ACK-HP.

-	If CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 1,

-	Perform rate matching for CSI part 1 according to clause 6.3.2.4.2.2, by taking HARQ-ACK with priority index 1 if any as HARQ-ACK, and taking CG-UCI associated with priority index 1 if any as CG-UCI.

-	Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.2.3, by taking HARQ-ACK with priority index 0 as CSI part 2 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and taking HARQ-ACK with priority index 1 if any as HARQ-ACK, and taking CG-UCI associated with priority index 1 if any as CG-UCI.

-	Otherwise,

-	Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.2.2, by taking HARQ-ACK with priority index 0 as CSI-part 1 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and taking HARQ-ACK with priority index 1 if any as HARQ-ACK, and taking CG-UCI associated with priority index 1 if any as CG-UCI.

-	Perform rate matching for CSI part 1 according to clause 6.3.2.4.2.3, by taking CSI part 1 as CSI part 2 and replacing βoffsetPUSCH by βoffsetCSI-part1, taking HARQ-ACK with priority index 0 as CSI-part 1 and taking HARQ-ACK with priority index 1 as HARQ-ACK, if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 0.

< Unchanged parts are omitted >