3GPP TSG-RAN WG1 #110bis-e	Tdoc R1-2210651

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.2.4.1

Source:	Moderator (Ericsson)

Title:	Summary #2 of Evaluation on AI/ML for positioning accuracy enhancement

Document for:	Discussion, Decision

# Introduction

This document summarizes the discussions during RAN1#110bis for the agenda item 9.2.4.1, Evaluation on AI/ML for positioning accuracy enhancement.

[110bis-e-R18-AI/ML-06] Email discussion on evaluation on AI/ML for positioning accuracy enhancement by October 19 – Yufei (Ericsson)

- Check points: October 14, October 19

This discussion corresponds to the objectives related to the positioning use case described in RP-213599 (SID) below.

RP-213599 (SID): Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. … Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

# Evaluation Methodology

In this section, remaining issues of evaluation methodology are discussed for AI/ML based positioning.

## Input to ML model for AI/ML assisted positioning

One issue raised by Qualcomm and OPPO is about the input/output of the ML model when AI/ML assisted approach is simulated. Their proposals are copied below.

- Qualcomm (R1-2209980) For AI/ML-assisted positioning, the UE can train an AI/ML model in two approaches: - Single-TRP approach: Using as input the channel response from a single TRP, the AI/ML model outputs the TOA associated with that TRP. The same model is used for all TRPs. - Multi-TRP approach: The AI/ML model uses as input the channel responses from all 18 TRPs, and outputs 18 TOAs jointly. Proposal4: When reporting performance, companies should clearly report which of the two approaches (single or multi-TRP) was used for evaluation. Proposal5: For AI/ML-assisted positioning, the single-TRP approach is adopted for evaluation as a baseline.

- OPPO (R1-2208854) Proposal 2: For AI/ML assisted positioning, if the output of AI model is timing-based result (e.g., TOA), prioritize the scheme where the measurement results corresponding to all TRPs are used as the input of AI model - the scheme where the measurement results corresponding to single TRP are used as the input of AI model is deprioritized.

### 1st round discussion

OPPO suggested to prioritize multi-TRP setup:

- “… where the output of AI model are the estimated TOAs corresponding to the 18 TRPs.”

- “… where the measurement results corresponding to all TRPs are used as the input of AI model”

OPPO also suggested to deprioritize single-TRP setup.

Meanwhile QC proposes to adopt single-TRP approach as baseline for evaluation. Thus companies’ recommendations are not aligned.

While several companies evaluated the single-TRP setup for AI/ML assisted approach (e.g., [2][5][24]), no extensive evaluation has been performed to compare single-TRP and multi-TRP setup. In some submitted evaluations, it is not clear which setup was used in ML model construction.

It is proposed that the two constructions are defined to align understanding. When reporting performance, companies should clearly report which of the two constructions (single or multi-TRP) was used for evaluation, as suggested by Qualcomm (R1-2209980).

Proposal 2.1.1-1

For AI/ML-assisted positioning, companies report which construction is applied in their evaluation:

- Single-TRP construction: the input of the ML model is the channel measurement between the target UE and a single TRP, and the output of the ML model is for the same pair of UE and TRP.

- Multi-TRP construction: the input of the ML model contains N sets of channel measurements between the target UE and N (N>1) TRPs, and the output of the ML model contains N sets of values, one for each of the N TRPs.

|  | Company |
| --- | --- |
| Support | Nokia/NSB, NVIDIA, Apple, InterDigital, Fujitsu, LG, Qualcomm,CMCC, samsung,ZTE, HW/HiSi, CATT, CAICT, Lenovo, Ericsson |
| Not support |  |

| Company | Comments |
| --- | --- |
| Nokia/NSB | We are fine with this proposal. |
| LG | Fine with the proposal while MTRP-based measurement seems natural for positioning. |
| OPPO | Just for clarification, if RSTD is the output of the UE-sided AI model (which is corresponding to two TRPs), does it belong to above-mentioned single-TRP construction or multi-TRP construction? [Moderator] My understanding is, RSTD still belongs to single-TRP. When RSTD is the output, one TRP is considered the reference point. One value is generated at ML output. |
| Qualcomm | We are fine with the proposal. |
| HW/HiSi | Support |

### 2nd round discussion

Proposal 2.1.1-1 was generally supported by companies.

One question was raised by OPPO about RSTD, which corresponds to measurements of two TRPs. Based on the RSTD IE descriptions from 37.355 (copied below), ‘single TRP’ in Proposal 2.1.1-1 can be understood as “this neighbour TRP”. Thus Proposal 2.1.1-1 is still valid.

37.355:

nr-RSTD This field specifies the relative timing difference between this neighbour TRP and the PRS reference TRP, as defined in TS 38.215 [36]. Mapping of the measured quantity is defined as in TS 38.133 [46].

Please check if you agree with above understanding. Accordingly, the proposal is updated by adding a note. Please check if the updated proposal below is acceptable.

Proposal 2.1.2-1

For AI/ML-assisted positioning, companies report which construction is applied in their evaluation:

- Single-TRP construction: the input of the ML model is the channel measurement between the target UE and a single TRP, and the output of the ML model is for the same pair of UE and TRP.

- Multi-TRP construction: the input of the ML model contains N sets of channel measurements between the target UE and N (N>1) TRPs, and the output of the ML model contains N sets of values, one for each of the N TRPs.

Note: For a measurement (e.g., RSTD) which is a relative value between a given TRP and a reference TRP, the TRP in “single-TRP” and “multi-TRP” refers to the given TRP only.

|  | Company |
| --- | --- |
| Support | ZTE, OPPO, NVIDIA, Nokia/NSB, Apple, Fujitsu, CAICT, Xiaomi,CMCC, Qualcomm |
| Not support | InterDigital (needs clarification about the note) |

| Company | Comments |
| --- | --- |
| vivo | To clarify, our understanding is that this is for model training construction only. There’s no limit to model inference after training. For example, when performing inference at a model trained with single-TRP construction, the input and output of that model can be for any pair of UE and TRP, not just the UE/TRP pair for training. If that’s the understanding, suggest rewording: “…, companies report which construction for model training is applied in their evaluation:“ |
| OPPO | Thanks for the clarification. It is clear to us now. |
| Samsung | Regarding vivo’s comment, we have different understanding, which the TRP as mentioned in bullet (a) should be the same TRP for training and inference. |
| Nokia/NSB | Our understanding is similar to Samsung. We are not sure if there could be different model input/output for training and inference. |
| Apple | Our understanding is similar to Samsung’s and Nokia/NSB’s for a non-generalization study. It can be discussed if for generalization, we adopt Vivo’s understanding. |
| Fujitsu | For the disussion above, we think the “neighbour TRP“ can be valid only for the moment when training or inference is performed (or the moment the data is collected), of course the “neighbor TRP“ will not be always constant when UE is moving or the environments are changed. Therefore we think both the moderator and VIVO’s expressions are correct, 1) At that MOMENT for training or inference, the input and output will correspond to the same pair of UE and TRP (e.g. UE and TRP1). 2) But the training and the inference may take place in different MOMENTS, then the input/output for training can be different from the input/output for inference (e.g. TRP1 for training and TRP2 for inference). So in our understanding, no conflicts here between the moderator and VIVO. |
| CATT | Regarding vivo and Samsung’s comment, our understanding is that the proposal only restricts the input and output of AI/ML model. The proposal doesn’t restrict which UE and TRP pair is used for training and inference. For example, for the Single-TRP construction, the proposal only restricts the same pair of UE and TRP should be used between output and input. However, whether the same or different pair of UE and TRP is used for the model training and inference is not related with this proposal. Just one minor comment, in the proposal, for the “ML model“, it’s better to align with previous agreement to change “ML model“ into “AI/ML model“. |
| InterDigital | From our perspective, RSTD requires two TRPs, reference and given TRP. However for a single TRP, according to the note, there’s an assumption that there is the reference TRP. So in terms of definition, that’s 2-TRP training. Our view of single-TRP training is that all measurements come from one TRP. RSRP based fingerprinting is one example of single-TRP training. RSTD based training will be applicable to multi-TRP. Therefore, the note should be applicable to multi-TRP only. |
| HW/HiSi | We have the same view as CATT, i.e. the proposal only restricts the input and output of the AI/ML model. Maybe the discussion has become unnecessarily complicated, since training got involved. If we would limit to inference at this stage, maybe it is acceptable to everyone? Maybe lile the update below? For AI/ML-assisted positioning, companies report which construction is applied for inference in their evaluation: - Single-TRP construction: the input of the ML model is the channel measurement between the target UE and a single TRP, and the output of the ML model is for the same pair of UE and TRP. - Multi-TRP construction: the input of the ML model contains N sets of channel measurements between the target UE and N (N>1) TRPs, and the output of the ML model contains N sets of values, one for each of the N TRPs. |
| LG | We have a similar understading with InterDigital regarding RSTD case where the measurement can be derived from the given TRP and reference TRP (i.e. 2 TRP cases). |

### 3rd round discussion

Regarding Proposal 2.1.2-1, further input was provided. Moderator’s understanding is below, based on companies’ input:

      - The proposal is only concerned with input and output of the AI/ML model. No need to mention training or inference. For example, for single-TRP, it only describes that the input and output are for the same pair of UE and TRP, which is true for training stage, and also true for inference stage. The proposal does not say anything about the pair used in training or the pair used in inference.

      - Regarding InterDigital and LG comment, it does not seem correct that RSTD is always multi-TRP just because there is a reference TRP. For RSTD, two different constructions are possible. Assuming there are N TRPs neighbor TRPs, then the proposal can be understood as follows for RSTD:

        - Single-TRP: the input of the model comes from a single TRP j together with the reference TRP, and the output of the model is RSTD(j) between the single TRP j and the reference TRP. If there are N TRPs, this model is applied to each TRP one at a time, j = 1, …, N.

        - Multi-TRP: the input of the model comes from all N TRPs together with the reference TRP, and the output of the model is the a list of N values [RSTD(1), RSTD(2), …, RSTD(N) ], where RSTD(j) is the time difference between the single TRP j and reference TRP.

For reference, the following Note is copied from 37.355 on reference TRP, i.e., an IE is used to signal the reference TRP.

NOTE 1:	The dl-PRS-ReferenceInfo defines the "RSTD reference" TRP. The nr-RSTD's and nr-RSTD-ResultDiff's in nr-DL-TDOA-MeasList are provided relative to the "RSTD reference" TRP.

With the above clarification, Proposal 2.1.2-1 is copied below for further checking.

Proposal 2.1.2-1

For AI/ML-assisted positioning, companies report which construction is applied in their evaluation:

- Single-TRP construction: the input of the ML model is the channel measurement between the target UE and a single TRP, and the output of the ML model is for the same pair of UE and TRP.

- Multi-TRP construction: the input of the ML model contains N sets of channel measurements between the target UE and N (N>1) TRPs, and the output of the ML model contains N sets of values, one for each of the N TRPs.

Note: For a measurement (e.g., RSTD) which is a relative value between a given TRP and a reference TRP, the TRP in “single-TRP” and “multi-TRP” refers to the given TRP only.

Please comment if you have STRONG concern on the proposal above and cannot accept it.

|  | Company |
| --- | --- |
| STRONG concern |  |

| Company | Comments |
| --- | --- |
| vivo | Need clarification. Let me give an example. 1. A model training with single-TRP construction. The same model was used across all 18 TRPs for inference. 2. Each TRP was trained with single-TRP construction. There’re 18 models for each TRP for inference. When companies report “single-TRP construction”, they may not mean the same setup. |
| CATT | Just one minor comment, in the proposal, for the “ML model“, it’s better to align with previous agreement to change “ML model“ into “AI/ML model“. For vivo’s comment, we think it can be further studied and simulated under the single-TRP construction. |

### 4th round discussion

An agreement has been made on single-TRP and multi-TRP construction. The note below asks companies to report whether the same model is used for all TRPs for not, when using single-TRP construction.

Note: For single-TRP construction, companies report whether they consider same model for all TRPs or N different models for TRPs

Moderator’s understanding is, the various methods of AI/ML model construction has different implications to model complexity and computation complexity. For single-TRP construction, for one AI/ML model, let the model complexity be PS, and let the computation complexity be CS. Correspondingly let the complexities be PM and CM if multi-TRP construction. Then the overall complexity is the following.

Table 1. Model complexity and computation complexity to support N TRPs for a target UE

|  | Model complexity to support N TRPs | Computation complexity to process N TRPs |
| --- | --- | --- |
| Single-TRP, same model for N TRPs | PS | N×CS |
| Single-TRP, N models for N TRPs | N×PS | N×CS |
| Multi-TRP (i.e., one model for N TRPs) | PM | CM |

Please share your view on the model complexity and computation complexity to support N TRPs for a target UE.

Question 2.1.4-1

For AI/ML assisted positioning, does Table 1 reflect the model complexity and computation complexity to support N TRPs? If not, please provide your suggestions.

|  | Company |
| --- | --- |
| Yes | Vivo, Fujitsu, LG, Nokia/NSB, Qualcomm |
| No |  |

| Company | Comments |
| --- | --- |
| Fujitsu | One concern is that it could be possible of using model A for several similar TRPs (such as sharing the similar imperfections e.g., sync errors) and model B for another several similar TRPs and so on, in this case, there will be M models for N TRPs with both M and N > 1 while M < N, then the complexity computation may be different. Anyway table 1 can be a decent starting point. [Moderator]This would be yet another variation. Since this is not in the agreed Note, and there is no company submitting evaluation results for this, let’s stay with the two agreed variants in the agreement for now. |
| LG | Similar understanding with Fujitsu. To address the issue, it would be expressed with big-o notation, e.g., O(N×PS) up to N-multiple complexity is needed. |
| ZTE | Generally, the model complexity of type 1(Single-TRP, same model for N TRPs) may be a bit larger than a model in type 2(Single-TRP, N models for N TRPs) as the former have more features to be learned. So the model complexity for type 1 and type 2 may not be proportional. So, it’s better that model model complexity for type 2 can be reported per model. |
| CATT | We have the same view with Fujitsu and ZTE. For Single-TRP, N models for N TRPs, suggest model complexity can be reported per model. |
| HW/HiSi | Can it please be clarified for the first row: Ps in the first row contains the parameters. Even if the model is the same at all gNBs, the parameters need to be duplicated since they need to be present in all gNB. We think that even for Ps, it should then be scaled with N, the complexity does not decrease when using the same model multiple time.. Or are we missing something here? Is the intention of this proposal to let companies report how the complexity has been obtained? [Moderator] Same model vs different model has implication to model training, model ID tracking, model transfer, model update, model monitoring, etc. If single model, only one needs to be handled in LCM stages. If different models, then N models need to be handled in LCM steps. |
| OPPO | Similar view as other companis that a same model for N TRPs may have larger model complexing and computation complexity. |
| CAICT | Share the same view as Fujitsu and other companies. |
| Nokia/NSB | We are fine with the added assumption that N models have different levels of model complexity – though this option seems significantly challenging from an implementation perspective. |

Question 2.1.4-2

For reporting AI/ML complexity (Model complexity, Computation complexity), should the complexity be those for N TRPs (i.e., not for a single TRP) when single-TRP construction is used? If not, please provide your suggestions.

|  | Company |
| --- | --- |
| Yes | [HW/HiSi], Nokia/NSB |
| No |  |

| Company | Comments |
| --- | --- |
| Fujitsu | Please refer to the comments above. |
| CATT | We think this Question 2.1.4-2 is included in Question 2.1.4-1. Question 2.1.4-1 focuses on the whole table is feasible or not, while Question 2.1.4-2 is only focus on the first two rows of the table is feasible or not. Is this correct understanding? |
| HW/HiSi | Agree if the model design is the same across different TRPs. |
| Nokia/NSB | Yes, total complexity for the use of AI/ML model should be reported. We think that the complexity is particularly relevant for UE-based inference, so the inference node (UE or network) is an important factor that needs to be clarified as well. |

### 5th, 6th round discussion

Based on the discussion in 4th round discussion, the proposal is formulated below to reflect companies’ views. Some thoughts in response to companies’ comments:

- The agreed note has two variants, which have evaluation results from companies. Thus it is recommended to address these two variants for now. In the future, if companies submit evaluations for other constructions (e.g., model A for a subset of TRPs and model B for another subset of TRPs), then RAN1 can further discuss.

  - “Note: For single-TRP construction, companies report whether they consider same model for all TRPs or N different models for TRPs”

- For single-TRP construction, same-TRP vs different-TRP have very different ramifications to LCM stages such as model training, model ID tracking, model transfer, model update, model monitoring, etc. Thus it is useful that companies report the overall complexities for positioning a target UE.

Proposal 2.1.5-1

For AI/ML assisted positioning, when single-TRP construction is used for the AI/ML model, companies report the AI/ML complexity (Model complexity, Computation complexity) for N TRPs, which are used to determine the position of a target UE.

Table. Model complexity and computation complexity to support N TRPs for a target UE

|  | Model complexity to support N TRPs | Computation complexity to process N TRPs |
| --- | --- | --- |
| Single-TRP, same model for N TRPs | PS Where PS is the model complexity for the same model. | N×CS Where CS is the computation complexity of the same model for one TRP. |
| Single-TRP, N models for N TRPs | i=1,…NPS,i Where PS,i is the model complexity for the i-th AI/ML model. | i=1,…NCS,i Where CS,i is the computation complexity for the i-th AI/ML model. |
| Multi-TRP (i.e., one model for N TRPs) | PM Where PM is the model complexity for the one model. | CM Where CM is the computation complexity for the one model. |

|  | Company |
| --- | --- |
| Support | Vivo, Fujitsu, ZTE, Qualcomm, NVIDIA, Ericsson, CATT |
| Not support | Hw/HiSi |

| Company | Comments |
| --- | --- |
| HWHiSI | Mabe we misunderstand the point here, it would be great if it coudl be clarified. So far we still have a concern on the single-TRP, same model. Even if the same parameters are used at each gNB, they still have to be duplicated since each gNB will get one copy of the model. Therefore, we think that also for this case the model complexity needs to be scaled with N to reflect the total complexity. @Yufei: thanks you for the clarification from the previous round “Same model vs different model has implication to model training, model ID tracking, model transfer, model update, model monitoring, etc. If single model, only one needs to be handled in LCM stages. If different models, then N models need to be handled in LCM steps.” Isn’t the LCM a separate discussion which is decoupled from the model complexity? |
| Moderator | To respond to @HW/HiSi comment: It does not seem reasonable to scale with N for total complexity for "single-TRP, same model“. It leaves the wrong impression that "single-TRP, same model“ have the same model complexity as "single-TRP, N different model“. The quick take away should be, it’s a lot easier, less complex, to train/store/monitor/update/... one model, as compared to N models. Also the table in the proposal is correct if considering UE-side model for a given UE. However, one can argue that duplicating for UE-side model is necessary for all 3 types of construction if considering the whole population of UEs, i.e., duplicating the model to distribute it to all NUE UEs. Overall, it does not seem to be a frutiful discussion to debate whether/how to account for duplication. Therefore, moderator would urge HW/HiSi to reconsider. |

## Generalization aspects

In RAN1#110, agreements were made on four generalization aspects. For this meeting, further discussion on generalization investigation have been submitted in the contributions.

- Qualcomm (R1-2209980) Proposal1: To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning: - Different realization of partial reflection and blocking changes - Training dataset from datasets {A0, A1,…, AN-1}, test dataset Bj from partially unseen reflections and blockings (i.e., partially different reflections and blockings than any in {A0, A1,…, AN-1} due to movement of reflecting objects, small environment variation over time, etc.). Here N>=1. FFS: modelling of {A0, A1,…, AN-1} and Bj based on TR 38.901. Proposal2: To model changes for investigating zone-specific generalizations at a given link between UE and TRP, one option is to consider a set CAj⊂C for generating channel coefficients for training dataset Aj, and consider a set CBj⊂C, CBj≠CAj, for generating channel coefficients of dataset Bi, where C is set of N=25 clusters between the UE and TRP. Then, the channel coefficient generation for training dataset Aj can follow these updated equations: Hu,sNLOSτ,t=n∈CAj, n≤22 i=13m∈RiHu,s,n,mNLOStδτ-τn,i-d3Dc-Δτ+n∈CAj, n≥3NHu,s,nNLOStδτ-τn-d3Dc-Δτ (training dataset Aj - NLOS) Hu,sLOSτ,t=1KR+1Hu,sNLOSτ,t+KRKR+1Hu,s,1LOStδτ-τ1-d3Dc. (training dataset Aj - LOS) and channel coefficient generation for testing dataset Bj can follow these updated equations: Hu,sNLOSτ,t=n∈CBj, n≤22 i=13m∈RiHu,s,n,mNLOStδτ-τn,i-d3Dc-Δτ+n∈CBj, n≥3NHu,s,nNLOStδτ-τn-d3Dc-Δτ (Testing dataset Bj-NLOS) Hu,sLOSτ,t=1KR+1Hu,sNLOSτ,t+KRKR+1Hu,s,1LOStδτ-τ1-d3Dc. (Testing dataset Bj-LOS)

- Faunhofer (R1-2209537) Proposal 2: The emulation of the impact of changes in the environment, the test data can be generated by weighted sum of two sets of channel data. Proposal 3: Split the channel data sets in three parts representing LOS part, part with high spatial correlation (e.g., single bounce reflections or specular reflections) and multipath components with lower spatial. Proposal 4: For better evaluation of the performance gain resulting from AI/ML based or assisted ToA estimator the simulation results shall be sorted according a level-of-difficulty metric.

- InterDigital (R1-2209484) Proposal 1: For the evaluation of AIML based positioning, adopt the following 3 categories of model generalization and evaluate the performance under each category - Category 1: Trained and inferred ideal input measurements - Category 2: Trained at ideal input measurements and inferred at non-ideal input measurements - Category 3: Trained at combination of ideal and non-ideal input measurements; inferred at non-ideal input measurements. Proposal 2: For the evaluation of Category 3 model generalization, agree on the split of the training data set between ideal(X%) and non-ideal(100-X) %) measurements. FFS: X.

- Samsung (R1-2209726) Proposal 1: RAN1 shall study the generalization ability for imperfect input/output data and how to model the imperfections. Proposal 3: RAN1 study the update/fine-tuning the model with limited number of data set or targeting generalization case.

- China Telecom (R1-2208772) Proposal 3: Different dataset size used for AI/ML model training should be studied to evaluate the model generalization capability.

- Huawei, HiSilicon (R1-2208433) Proposal 11 : For evaluation on the AI/ML-based positioning, the synchronization error of the transmission link between gNB and UE should also be considered. Proposal 12 : RAN1 should define some typical evaluation cases for evaluate the generalization in the aspect - UE timing error to better compare results of generalization studies among companies. In addition, further cases should also be captured to better explore and expand the generalization capabilities of the AI/ML model. One example is given in the template below.

- vivo (R1-2208638) Proposal 1: The positioning accuracy performance of AI/ML based positioning should be evaluated under different settings/scenarios (i.e. the training and testing dataset are from different settings) - different drop in the same scenario; - different clutter parameter settings; - different InF scenarios.

- Nokia (R1-2209371) Proposal-1: For evaluation of AI/ML based positioning, only InF-DH sub-scenario should be considered as part of the Rel-18 study item. Proposal-2: RAN1 should agree to limit the Rel-18 study on AI/ML for positioning accuracy enhancements use case to only intra-site variations, as previously agreed. Proposal-3: RAN1 to discuss and agree whether other approaches such as model update, finetuning or adapting the positioning method used, could be a better approach to handle possible variations within a particular scenario.

### 1st round discussion

In RAN1#110, the proposal below was considered “offline agreement”, but it couldn’t be confirmed as agreement during online discussion. The main concern was, different InF scenarios represent substantially different deployment environments. It was not a typical case that a ML model need to work well in different InF scenarios.

In the following, the same proposal is presented again to see if consensus is achievable.

Proposal 2.2.1-1

To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning:

(e) InF scenarios, e.g., training dataset from one InF scenario (e.g., InF-DH), test dataset from a different InF scenario (e.g., InF-HH)

|  | Company |
| --- | --- |
| Support | Vivo, Nokia/NSB, NVIDIA, Apple, Fujitsu, LG, OPPO, Samsung, CATT, CAICT, Lenovo |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | We can live with it if other companies would like to evaluate it. However, we don’t see a different observation can be made for this case as we already have different clutter settings for InF-DH. |
| HW/HiSi | Share the view of ZTE |
| Ericsson | We share the same view as ZTE. |

The other issue that was discussed in RAN1#110 is the modeling of minor changes in the environment. In this meeting, Qualcomm (R1-2209980) and Faunhofer (R1-2209537) provided detailed methods to modify the channel models in 38.901 to emulate minor changes in the environment (e.g., moving objects, small environment variations over time in a factory). In Qualcomm (R1-2209980), this is intended for the study of “intra-site (or zone-specific)” generalization.

Companies are invited to study carefully the methods described in Qualcomm (R1-2209980) and Faunhofer (R1-2209537), and provide input on whether/how to proceed on this generalization aspect.

Question 2.2.1-2

Should RAN1 investigate the model generalization capability for intra-site (or zone-specific) variations?

|  | Company |
| --- | --- |
| Yes | Qualcomm |
| No | Nokia/NSB, HW/HiSi, Ericsson |

| Company | Comments |
| --- | --- |
| Nokia/NSB | We think that the current agreements capture model generalization scenarios quite comprehensively, and the results highlight the issues that arise from applying an AI/ML model trained and tested using different scenarios/settings. We are not sure about the added value from including more scenarios as part of this evaluation. |
| Apple | If companies want to do so, they are free to bring results, but this study should not be mandatory. |
| Fujitsu | These concepts such as “inter-site”, “intra-site” or “zone” were proposed in 9.2.1 as guidelines, we prefer to clarify details of these concepts in general first. |
| HW/HiSi | Agree with Nokia |
| CATT | Share the same view with Nokia and Apple. |
| Ericsson | We share the same concern as Nokia. Additionally, in terms of how to modify the methodology and equations in 38.901, channel model experts need to be consulted to be sure if the proposed methods are valid. However, this task is out of scope for this study item. |

Question 2.2.1-3

If yes to Question 2.2.1-2: how to modify the channel models in 38.901 to emulate intra-site (or zone-specific) variations?

- Alt 1: according to Proposal 1 and 2 of R1-2209980

- Alt 2: according to Proposal 2 and 3 of R1-2209537

- Alt 3: Other (please describe)

|  | Company |
| --- | --- |
| Alt 1 | Qualcomm |
| Alt 2 |  |
| Alt 3 (Other, please describe) |  |

| Company | Comments |
| --- | --- |
| Fujitsu | Suggest not to do any limitation here, left this part for companies to report their own scheme freely. |
| Qualcomm | We support Alt 1 but are also fine to discuss other alternatives. |
| Apple | Agree with Fujitsu |

### 2nd round discussion

Proposal 2.2.1-1 was generally supported, and no objection was raised. Thus it will be brought to next online discussion.

For Question 2.2.1-2 and Question 2.2.1-3, there is no broad support to explore “intra-site (or zone-specific)” variation. It is pointed out that these can be considered general issue for AI 9.2.1 (the relevant agreement from AI9.2.1 is copied below for reference). Also individual companies can anyways bring results.

Agreement (AI 9.2.1) Use 3gpp channel models (TR 38.901) as the baseline for evaluations. Note: Companies may submit additional results based on other dataset than generated by 3GPP channel models

Given that this issue has been openly debated for two meetings, the following conclusion is presented for further checking.

Conclusion 2.2.2-1

For evaluation AI/ML based positioning, suspend the discussion on intra-site (or zone-specific) variations until concepts and channel model construction not in TR38.901 (e.g., “intra-site” or “zone”) are clarified under AI 9.2.1.

- Note: An individual company can still submit evaluation results for intra-site variation.

|  | Company |
| --- | --- |
| Support | [ZTE], OPPO, Nokia/NSB, Apple, Fujitsu, CAICT, InterDigital, Xiaomi,CMCC, HW/HiSi, LG |
| Not support |  |

| Company | Comment |
| --- | --- |
| Qualcomm | --Comment table added by Qualcomm— Just to clarify. By intra-site or zone-specific variation, we mean time-varying changes happening in a real-world deployment because of human mobility, movement of trucks or lift machines, partial changes in machinery locations, etc. It is quite important to verify that AI/ML positioning model would be robust to these changes. Our goal is to capture a conclusion in the TR on whether zone-specific models are robust to such mild to moderate time-varying changes. This reduces the need to frequently updating the model or considering retraining. It also proves the applicability of such models in real world scenarios. We would like to explore whether other companies view such a conclusion relevant. In addition, the 9.2.1 is generic and not supposed to touch on evaluation requirements that are specific to use cases. |
| ZTE | As cited by FL, 38.901 doesn’t include modeling method for intra-site evaluation, so it’s up to companies to optionally provide the result rather than suspend the discussion. |

HW/HiSi raised the issue that RAN1 should agree on a set of typical cases for verifying the generalization performance of an AI/ML model, i.e., similar to those agreed for CSI and beam management. Thus the following is proposed. Note that Configuration is omitted here, since all the generalization aspects belong to Scenario.

Proposal 2.2.2-2

For evaluation of AI/ML based positioning, at least the following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios:

- Case 1: The AI/ML model is trained/validated based on a training dataset from Scenario#A, and then the AI/ML model performs inference on a test dataset from the same Scenario#A;

- Case 2: The AI/ML model is trained/validated based on a training dataset from Scenario#A, and then the AI/ML model performs inference on a test dataset from a different scenario (Scenario#B);

- Case 3: The AI/ML model is trained/validated based on a mixed training dataset which contains data from Scenario#A and Scenario#B, and then the AI/ML model performs inference on a test dataset which is from a scenario in the mix (e.g., either Scenario#A or Scenario#B);

- Case 4: The AI/ML model is trained/validated based on a training dataset from Scenario#A, and then the AI/ML model is updated based on a fine-tuning dataset for a different scenario (Scenario#B). After that, the AI/ML model performs inference on a test dataset for Scenario#B;

Note: “Scenario” above refers to one or a combination of the generalization aspects: drops, clutter parameters, network synchronization error, UE/gNB RX and TX timing error (and other generalization aspects if agreed).

|  | Company |
| --- | --- |
| Support | InterDigital, Hw/HiSi |
| Not support |  |

| Company | Comments |
| --- | --- |
| OPPO | The proposal looks good. However, What’s the additional value of this proposal? We have agreed a list of comfigurations to evalaute the generalization. Do we will proposal more combinations of configuration/scenarios for the evaluation? We can live with it if majority companies support the duplicated agreement for each use cases. |
| Samsung | Dont see the point of this proposal. If FL wants to have some general description for generalization aspect evaluation for not just positoining use case but all three cases, we think it should be discussed in the framework agenda instead of here. As far as we know, it seems all three cases are somehow already moving in this direction. |
| NVIDIA | The value of this proposal is not clear, depsite similar versions were agreed for CSI and BM. In positioning, we’ve gone ahead and made more specific agreements for generalization study. That said, if all the other companies would like to have it, we can live with it as well. |
| Nokia/NSB | We agree with other companies and think that this proposal is not required. |
| Fujitsu | This proposal reminds us the long and useless discussion took place in 9.2.1 for a similar proposal, maybe because their topic is quite general so they have to spend a lot of time on the terminology and theoretical issues. As our 9.2.4 topic, we prefer to focus on the details of positioning use case itself, as the last meeting did. Therefore, we think this kind of proposal which is too general should be avoided in our 9.2.4 discussion. |
| CATT | We also think this proposal is not necessay. In CSI and beam management session, they first agree this general agreement and then discuss the detial list of configurations/senarios to evalaute the generalization. However, for positioning, we already have detial list of configurations/senarios to evalaute the generalization. |
| InterDigital | The FL’s proposal clarifies how generalization is evaluated. |
| Qualcomm | We do not see the need of this proposal at this stage. We have already considered several agreements (see below) that almost mirror this one. We are also discussing conclusions that relate to the cases listed above and it is not clear why we need to come back and define these cases. In addition, the current listing of cases is different from the one considered in CSI use case. We prefer Case2A listing as posted in that agreement. The note at the end of the proposal should also cover the robustness aspect related to time-varying changes. E.g., “Note: “Scenario” above refers to one or a combination of the generalization aspects: drops, clutter parameters, network synchronization error, UE/gNB RX and TX timing error, time-varying changes (e.g., different realization of partial reflection and blocking changes due to human and machinery movements) (and other generalization aspects if agreed)” Case1, Case 3 Agreement (RAN1-109e) As a starting point, the training, validation and testing dataset are from the same large-scale and small-scale propagation parameters setting. Subsequent evaluation can study the performance when the training dataset and testing dataset are from different settings. Case2 Agreement (RAN1-110) To investigate the model generalization capability, at least the following aspect(s) are considered for the evaluation for AI/ML based positioning: - Different drops - Training dataset from drops {A0, A1,…, AN-1}, test dataset from unseen drop(s) (i.e., different drop(s) than any in {A0, A1,…, AN-1}). Here N>=1. - Clutter parameters, e.g., training dataset from one clutter parameter (e.g., {40%, 2m, 2m}), test dataset from a different clutter parameter (e.g., {60%, 6m, 2m}); - Network synchronization error, e.g., training dataset without network synchronization error, test dataset with network synchronization error; - Other aspects are not excluded. Note: It’s up to participating companies to decide whether to evaluate one aspect at a time, or evaluate multiple aspects at the same time. Case4 Agreement (RAN1-110) For AI/ML-based positioning, for evaluation of the potential performance benefits of model finetuning, report at least the following: - training dataset setting (e.g., training dataset size necessary for performing model finetuning) horizontal positioning accuracy (in meters) before and after model finetuning. |
| HW/HiSi | We would suggest to specify some typical cases according to the agreement from last meeting to provide results that can be compared across companies. We think it would be helpful for the study if different companies can simulate the same cases according the agreement from last meeting. Therefore, we support the proposal. |
| ZTE | Generally fine with the proposal. According to agreement in 9.2.2.1, case 4 is not a separate case since it’s more like a model update/fine-tuning. We prefer to follow the original wordings. In addition, we prefer to add a FFS to encourage companies to evaluate generalization in different configurations. FFS: model generalization/scalability over different configurations(e.g., RS, TRP configuration) |

### 3rd round discussion

Regarding conclusion 2.2.2-1, there was broad support.

QC commented that the study of intra-site or zone-specific variation is valuable to real world scenarios. Interested companies can continue to submit evaluation results to share with RAN1. About “AI 9.2.1 is generic”, this is of course true. But it is reasonable that intra-site/zone-specific is considered generic and relevant to all sub-use cases, e.g., companies have submitted contributions to AI 9.2.1 on intra-site/zone/zone ID.

With the above understanding, Conclusion 2.2.2-1 is copied below for further checking.

Conclusion 2.2.2-1

For evaluation of AI/ML based positioning, suspend the discussion on intra-site (or zone-specific) variations until concepts and channel model construction not in TR38.901 (e.g., “intra-site” or “zone”) are clarified under AI 9.2.1.

- Note: An individual company can still submit evaluation results for intra-site variation.

Please comment if you have STRONG concern on the proposal above and cannot accept it.

|  | Company |
| --- | --- |
| STRONG concern |  |

Regarding Proposal 2.2.2-2, HW/HiSi recommended strongly to have this discussion. However, other companies pointed out that there is little value in duplicating existing agreements. Thus moderator recommends not to pursue this further.

### 6th round discussion

It was brought to moderator’s attention that when different InF scenarios are evaluated for generalization capability, the parameter setting of the InF scenarios should be compliant with TR38.901, see TS 38.901 Table 7.2-4 below.

TR 38.901 V17.0.0, Table 7.2-4: Evaluation parameters for InF

|  |  | InF |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters |  | InF-SL (sparse clutter, low BS) | InF-DL (dense clutter, low BS) | InF-SH (sparse clutter, high BS) | InF-DH (dense clutter, high BS) | InF-HH (high Tx, high Rx) |
| Layout | Room size | Rectangular: 20-160000 m2 |  |  |  |  |
|  | Ceiling height | 5-25 m | 5-15 m | 5-25 m | 5-15 m | 5-25 m |
|  | Effective clutter height hc | < Ceiling height, 0-10 m |  |  |  |  |
|  | External wall and ceiling type | Concrete or metal walls and ceiling with metal-coated windows |  |  |  |  |
| Clutter type |  | Big machineries composed of regular metallic surfaces. For example: several mixed production areas with open spaces and storage/commissioning areas | Small to medium metallic machinery and objects with irregular structure. For example: assembly and production lines surrounded by mixed small-sized machineries. | Big machineries composed of regular metallic surfaces. For example: several mixed production areas with open spaces and storage/commissioning areas | Small to medium metallic machinery and objects with irregular structure. For example: assembly and production lines surrounded by mixed small-sized machineries. | Any |
| Typical clutter size, dclutter |  | 10 m | 2 m | 10 m | 2 m | Any |
| Clutter density r (percentage of surface area occupied by clutter) |  | Low clutter density (<40%) | High clutter density (≥40%) | Low clutter density (<40%) | High clutter density (≥40%) | Any |
| BS antenna height |  | Clutter-embedded, i.e. the BS antenna height is below the average clutter height |  | Above clutter |  | Above clutter |
| UT location | LOS/NLOS | LOS and NLOS |  |  |  | 100% LOS |
|  | Height | Clutter-embedded |  |  |  | Above clutter |

Thus, for example:

- If InF-HH, the links are 100% LOS. Moderator’s understanding is, it is not so useful to evaluate for InF-HH to demonstrate the benefit of AI/ML based positioning.

- If InF-SH, then clutter size dclutter =10m, and Clutter density r<40%.

This means, the agreed clutter parameters {60%, 6m, 2m} and {40%, 2m, 2m} cannot be used together with InF-SH scenario. In general, companies need to ensure the selected simulation parameters are consistent with TR38.901.

It is also noted that, in Rel-17 TR 38.857, InF-SH is described in addition to InF-DH. As can be observed below, The clutter parameters for InF-SH is {20%, 2m, 10m}.

TR 38.857, Table 6.1-1: Parameters common to InF scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Channel model | InF-SH, InF-DH | InF-SH, InF-DH |
| Clutter parameters: {density , height ,size } | Low clutter density: {20%, 2m, 10m} High clutter density: - Baseline): {40%, 2m, 2m} for fixed UE antenna height and gNB antenna height - (Optional): {40%, 3m, 5m} - (Optional): {60%, 6m, 2m} |  |

Summarizing the above, the following is proposed.

Proposal 2.2.4-1

For AI/ML based positioning, if an InF scenario different from InF-DH is evaluated for the model generalization capability, the selected parameters (e.g., clutter parameters) are compliant with TR 38.901 Table 7.2-4 (Evaluation parameters for InF).

- If InF-SH scenario is evaluated, the clutter parameter is {20%, 2m, 10m} (i.e., according to TR 38.857 Table 6.1-1 (Parameters common to InF scenarios)).

|  | Company |
| --- | --- |
| Support | ZTE |
| Not support | vivo (OK with the main sentence, comment on the bullet) |

| Company | Comments |
| --- | --- |
| ZTE | OK with this clarification. But we should in mind, InF-DH should always be the baseline for evaluation. To be more specific, maybe we can simply agree that the InF scenario different from InF-DH is InF-SH scenario as you suggested in subullet. No other InF scenarios are required. |
| vivo | We agreed that Agreement To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning: (e) InF scenarios, e.g., training dataset from one InF scenario (e.g., InF-DH), test dataset from a different InF scenario (e.g., InF-HH) In our understanding, the evaluation in different InF scenario(s) is meant to see whether an AI/ML model trained in InF-DH (with either {60%, 6m, 2m} and/or {40%, 2m, 2m}) can work in some other scenario(s). On ZTE’s comment, what is the technical reason to NOT allowing evaluation on other scenarios, such as InF-SL, InF-DL, InF-HH? We disagree with above ZTE’s suggestion that only InF-SH is allowed as different scenario other than InF-DH. In fact, we think this is against the motivation for this agreement Agreement Study potential specification impact needed to enable the development of a set of specific models, e.g., scenario-/configuration-specific and site-specific models, as compared to unified models. We’re OK with the main sentence as that is aligned with the following (1st agreement made in this SI) Agreement - Use 3gpp channel models (TR 38.901) as the baseline for evaluations. - Note: Companies may submit additional results based on other dataset than generated by 3GPP channel models However, on the bullet “If InF-SH scenario is evaluated, the clutter parameter is {20%, 2m, 10m} (i.e., according to TR 38.857 Table 6.1-1 (Parameters common to InF scenarios)).”, we prefer to remove it for now as InF-SH is not the only scenario nor is the clutter parameter the only thing differ from the agreed simulation assumptions which was for InF-DH. For instance, what would the assumption on Layout (Hall size, BS locations) for other InF scenario(s)? |
| Moderator | Considering ZTE and vivo input, the sub-bullet can be updated to: Note: In TR 38.857 Table 6.1-1 (Parameters common to InF scenarios), InF-SH scenario uses the clutter parameter {20%, 2m, 10m} which is compliant with TR 38.901. |
| vivo | Thanks moderator for the update. About the added note, it still only mentions clutter parameter for InF-SH. Is the intention to not discuss other settings (e.g., Layout (Hall size, BS locations) for other InF scenario(s)) any further? |
| CATT | Thanks Moderator’s clarification and we are generally fine. But besides clutter parameter, we think Hall size for Inf-SH should also be discussed and aligned the simulation assumption. TR 38.857, Table 6.1-1: Parameters common to InF scenarios FR1 Specific Values; FR2 Specific Values. Channel model; InF-SH, InF-DH; InF-SH, InF-DH. Layout; Hall size; InF-SH: (baseline) 300x150 m (optional) 120x60 m InF-DH: (baseline) 120x60 m (optional) 300x150 m. BS locations; 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m. According to Table 6.1-1 in TR 38.875, the baseline of In-DH is 120x60 m, which is optional in InF-SH. For the generalization simulation, we prefer to use the same Hall size between InF-SH and InF-DH, i.e., if the baseline of InF-DH is used, the same Hall size (i.e., 120x60 m) is also used in InF-SH. Regarding the BS locations and other assumptions, which are the same between InF-SH and InF-DH, we don’t think it’s necessary to be discussed if the Note is focus on InF-SH scenario. |
| Moderator | I checked the other parameters (e.g., hall size, BS height) between TR38.857 and TR38.901. I don’t see any other problem. I only find clutter parameter to be confusing, i.e., TR38.857 does not mark out that {60%, 6m, 2m} and {40%, 2m, 2m} cannot be used together with InF-SH scenario. For the hall size, InF-SH has 120x60 m already, although it’s marked as Optional. I think companies would assume that for generalization study, the same hall size is used between InF-DH and InF-SH. To be safe, we can clarify in the note: Note: In TR 38.857 Table 6.1-1 (Parameters common to InF scenarios), InF-SH scenario uses the clutter parameter {20%, 2m, 10m} which is compliant with TR 38.901. For the investigation of generalization capability in terms of InF scenarios, other parameter settings (e.g., hall size) are assumed to be the same between InF-DH and InF-SH. In general, the intention to remind companies not to use settings that do not make sense in light of 38.901 ---- otherwise it raises questions whether such evaluation results are valid or not. I don’t think it’s necessary to re-discuss all evaluation assumptions for all the other InF scenarios. |

## (Closed) Channel estimation

In the contributions below, it is proposed that RAN1 should study the AI/ML performance with actual channel estimation, instead of ideal CIR for example.

- vivo (R1-2208638) Proposal 6: Further study the impact and potential solution of CIR estimation error on AI/ML based positioning performance.

- Google (R1-2208882) Proposal 1: The study of AI/ML based AI/ML based positioning should be based on actual channel estimation.

### 1st round discussion

In vivo (R1-2208638), evaluation results were first provided using error-free CIR (“The existing schemes are all evaluated under the assumption that ideal CIRs used for model training and inference can be obtained while ignoring the implementation imperfections.”). Then detailed investigation was performed for a range of SINR values for the training dataset and test dataset (see results copied in section 5.5.3).

Moderator’s understanding is, it is not recommended to use SINR as a parameter for positioning study. SINR is a metric for a single link, whereas positioning requires input from numerous links (typically, at least 3 UE-TRP links), where each link can have different SINR depending on the pathloss between the UE and each TRP, the multipath channel condition, transmitter power, noise figure, etc. Thus it is sufficient to follow Google (R1-2208882) proposal that actual (i.e., noisy) channel estimation should be assumed in the evaluation.

It is noted that the agreed simulation assumption provides sufficient parameters to determine channel estimation performance for each link used for positioning, see sample parameter settings below.

TR 38.857, Table 6-1: Common scenario parameters applicable for all scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |

TR 38.857, Table 6.1-1: Parameters common to InF scenarios

| Total gNB TX power, dBm | 24dBm | 24dBm EIRP should not exceed 58 dBm |
| --- | --- | --- |

Thus, the following is proposed, which is modified from R1-2208882 (Google).

Proposal 2.3.1-1

For evaluation of AI/ML based positioning, the model performance is evaluated with actual (i.e., not ideal) channel estimation, using the evaluation assumption on gNB/UE TX power and noise figure.

|  | Company |
| --- | --- |
| Support | Ericsson |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | In principle, we support to study the impact of channel estimation error. We’d like to response and get clarification to moderator’s comment “it is not recommended to use SINR as a parameter for positioning study. SINR is a metric for a single link, whereas positioning requires input from numerous links (typically, at least 3 UE-TRP links), where each link can have different SINR depending on the pathloss between the UE and each TRP”. - Yes, there’re multiple links (i.e., between the UE and multiple TRPs) for positioning. However, there’s an SINR for each link/channel (i.e., the channel between the UE and a TRP). In our contribution R1-2208638 section 4.2.1, we studied the SINR distribution (practical lower and upper bound considering interference from all other links) and showed the range of practical SINR in Figure 12 for CIR as model input. [Moderator] Agree that Figure 12 showed the range of practical SINR. But it is unclear what the SINR value refers to in evaluation results Table 10, 11, and Figure 13, when the UE observes several links at the same time in order to estimate position. - Since moderator stated “it is not recommended to use SINR as a parameter for positioning study”, can the moderator clarify what would be the metric for the non-ideal channel estimation/measurement accuracy (e.g., CIR measured for each UE/TRP pair) for model input? [Moderator] The channel estimation error comes from how each company simulate the receiver, given the simulation parameters such as TX power, noise figure, distance between the UE and a TRP, propagation channel, etc. - Current proposal says “using the evaluation assumption on gNB/UE TX power and noise figure”, our understanding is that we need more discussion on other assumptions for the study of non-ideal channel estimation. For example, when one TRP transmit RS for CIR measurement, do all other TRPs transmit at the same time or not? If so, does RS from all TRPs overlapping or not? If no overlapping RS from all TRPs, what about the assumption regarding data transmission from other TRPs w.r.t. RS transmission from one TRP? [Moderator] Agree that to be accurate, issues raised above should be considered. Moderator’s understanding is that it’s not the best use of time to discuss all the details. We can follow Rel-17 simulation assumption, which is already agreed. |
| Apple | To clarify, is the expectation that we model a link between the UE and TRP and perform a channel estimation based on an RS signal ? |
| OPPO | More discussions are needed on how to stimulate the inference. For example, the neighboring cells may be transiting data which will lead to inference to the PRS reception. However, in the simulator for positioning, the data transmission is usually not simulated. For this case, how to model the inference? |
| Samsung | Ideally, this real channel estimation should be evaluated, but considered the workload, we feel it should left to companies who interested to present the result by themselves, just to state out the details on how it simulates. |
| ZTE | We think agreed tables are enough for study. This proposal is not needed. If we want to evaluate model performance under different SINR, which may belong to model generalization issue. Agree with Samsung, we should consider the workload. |
| HW/HiSi | We do not support the proposal, at least not at this stage. From reviewing the contributions, it is unclear to us how companies have modeled the channel estimation. In a first step, companies could report the channel estimation they have used. From our understanding, the impact of channel estimation on the positioning accuracy is much smaller than from e.g. the NW synchronization errors. |
| CATT | We think real channel estimation is helpful to evaluation the model performance. But how to model it and how to compare the simulation results between different companies due to different companies may have different real channel estimation capability should be further studied. |
| MediaTek | We found including TX power in can improve position accuracy, which is shown in section 3.2.1 of our Tdoc (R1-2209510) where non-normalized PDPs and normalized PDPs as model input are studied. Basically, we support to study the actual channel estimation based on gNB/UE TX power and noise figure which have been already agreed upon since RAN1#109e as parameters in the InF scenarios. And furthermore, we would like to ask for a clarification whether other potential imperfections such as timing and frequency synchronization error or RF implementation errors, etc., on channels estimation in UE or gNB should be included, even though these imperfections were agreed to study in RAN1#110 and several companies have already shown their results as part of the evaluation for generalization. If the answer is yes, each company can proposal how to add the impairments to channel estimation in the next meeting (RAN1#111). |
| CAICT | More evaluation results could be collected for further discussion and it depends on companies‘ will till next meeting. |
| Lenovo | Further study is required, but the evaluation of non-ideal CE can be considered optional as part of the evaluations. |
| Ericsson | The channel estimation error should be included as part of the evaluation, instead of error-free channel estimation. Exactly how to perform the channel estimation is up to each company to report. |
| InterDigital | We have a similar view as Lenovo. |

### 2nd round discussion

Regarding Proposal 2.3.1-1, there are divergent views. Checking TR38.857 “Table 6-1: Common scenario parameters applicable for all scenarios”, the following row exists.

TR38.857, Table 6-1: Common scenario parameters applicable for all scenarios

| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |
| --- | --- |

Since it has been agreed to reuse the common scenario parameters defined in Table 6-1 of TR 38.857, each company can follow the above, and report their algorithm for channel estimation. Also, the above assumption means that the estimation is not ideal (“individual parameters estimation is applied”).

Thus, this issue can be closed by the conclusion below, following several companies’ recommendation.

Conclusion 2.3.2-1

For evaluation of AI/ML based positioning, channel estimation error is taken into account according to PHY/link level abstraction in TR38.857 Table 6-1, which is part of agreed simulation assumption.

|  | Company |
| --- | --- |
| Support | ZTE |
| Not support | InterDigital |

| Company | Comments |
| --- | --- |
| vivo | As we discussed in our contribution, there’s no ideal channel measurement when use an AI/ML model in real deployment. RAN1 spent a lot of efforts came up different cases to verify AI/ML model generalization performance. Evaluation results for generalization already showed that likely an AI/ML model will only work under some conditions. There’s active discussion to identify those conditions. We raise this issue because we strongly believe that channel measurement quality is also an important condition to study considering the input of model can only be based on actual channel measurement in real deployment. We find it very illogical to understand when companies say no to the study because they think the impact is small before conducting any evaluation. Having said that, we understand the concern on workload etc. and are okay to take the conclusion above and leave channel estimation error details to each company. |
| OPPO | Not support. We don’t think the “ individual parameters estimation is applied“ has that indication. We agree with other companies that it is up to each company. Companies are free to bring up with the evalaution results considering channel estimation error. Otherwise, if we follow the similar logic, the list will be very long. Do we have to make a agreement for each of the long list? For example, estimation error of doppler shift, quantization error, antenna Array Phase Center Offset Errors (both UE and gNB) ... |
| Samsung | It can be optional considered by company, no need to be mandatory. |
| Nokia/NSB | We are not sure if this proposal is required to be agreed, since currently companies are free to consider channel estimation error a spart of their evaluations. |
| Apple | We think that this should be optional |
| Fujitsu | To be short, we think this is not mandatory, excpet for the reasons mentioned by other companies above, we believe our evaluation should be focused on the validity and benefits for applying AI/ML model over the conventional counterparts. The channel estimation error will affect not only AI/ML but also conventional methods so it will be sufficient if the evaluation conditions can be aligned for comparison. The channel estimation error details can be optional for each companies. |
| CAICT | We also prefere channel estimation error as optional. |
| CATT | We don’t support this poposal. Interesting companies can provide the simulantion results considering channel estimation error. We don’t think it should be mandatory. |
| InterDigital | We have a similar view as OPPO. We should leave it up to companies to describe the assumption made on channel estimation error. |
| Xiaomi | We don’t support setting this evaluation as mandatory. Volunteer company can provide the results. |
| CMCC | It can be up to each company. |
| HW/HiSi | We also think that channel estimation error can be reported by companies. There is no need to make it mandatory. |
| LG | It should be optional which is up to interesting company(es) |
| ZTE2 | We can live with it. However, we feel it’s not necessary have a separate agreement for this as we have already agreed the tables. It’s up to companies to report its error modeling |

### 3rd round discussion

Regarding Conclusion 2.3.2-1, majority companies suggest that it should be up to each company to simulate channel estimation error. Thus, the following is proposed to close this discussion.

Proposal 2.3.3-1

For evaluation of AI/ML based positioning, it’s up to each company to take into account the channel estimation error in their evaluation. Companies describe the details of their simulation assumption, e.g., realistic or ideal channel estimation, error models, receiver algorithms.

|  | Company |
| --- | --- |
| Support | Fujitsu CMCC, CATT, CAICT |
| Not support |  |

### 4th round discussion

On the topic of channel estimation error, proposal 2.3.3-1 was discussed in the online session. While it is generally acceptable, no decision was made due to a concern on how to capture the evaluation results.

Moderator’s understanding is:

      - It’s up to each company how to model channel estimation error, i.e., RAN1 does not attempt to align this between companies.

      - For fair comparison, the participating company apply the same channel estimation algorithm to the conventional positioning method and the AI/ML based method.

      - Evaluation results submitted by companies can be captured in TR in the end, even if the channel estimation error is modelled differently between companies.

With the above understanding, the proposal is copied below for further checking.

Proposal 2.3.4-1

For evaluation of AI/ML based positioning, it’s up to each company to take into account the channel estimation error in their evaluation. Companies describe the details of their simulation assumption, e.g., realistic or ideal channel estimation, error models, receiver algorithms.

|  | Company |
| --- | --- |
| Support | vivo (with a note), Fujitsu, LG, Samsung, ZTE, CATT, OPPO, CAICT, Nokia/NSB, NVIDIA |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | Thanks moderator sharing her understanding to our comment during GTW. We propose to add a note to this proposed conclusion to reflect moderator’s understanding. Note: contributions based on optional evaluation assumptions are not precluded from being considered for discussion and decisions |
| Fujitsu | Fine with the proposal. |
| ZTE | Anyway, we have a lot of EVM settings to be optional. Whether/how to capture the observations can be further discussed. |
| CATT | Share the same view as ZTE. |
| HW/HiSi | Fine with the proposal. Regarding the suggested note from vivo, we are not really sure if this proposal is the right place for it. The note is very generically formulated and does not seem to relate to proposal. Also, we already have agreements for many parameters that companies can report other values. We would prefer to not having this note. |
| OPPO | Similar view as Huawei that we don’t support the vivo’s note. |
| Nokia/NSB | We would prefer to follow the same methodology for channel estimation as used in Rel-17 for positioning evaluation. We are fine with other companies using different assumptions and clarifyin the assumptions. [Moderator] Yes, it’s natural to follow the same methodology for channel estimation as used in Rel-17. |

### 5th round discussion

In the 4th round of discussion, vivo suggested a note, while several other companies do not see the need. As commented by several companies, there are many parameters/settings that companies can report. As long as companies share the common understanding that optional evaluation assumptions are not precluded from consideration, it’s fine not to have the note. Thus the proposal is repeated below for further checking.

Proposal 2.3.5-1

For evaluation of AI/ML based positioning, it’s up to each company to take into account the channel estimation error in their evaluation. Companies describe the details of their simulation assumption, e.g., realistic or ideal channel estimation, error models, receiver algorithms.

Please comment if you have STRONG concern on the proposal above and cannot accept it.

|  | Company |
| --- | --- |
| STRONG concern |  |

| Company | Comments |
| --- | --- |
| vivo | We’d like to echo the statement from moderator “As long as companies share the common understanding that optional evaluation assumptions are not precluded from consideration, it’s fine not to have the note.” With that understanding, we’re okay to take this proposed conclusion. |

## (Closed) Other

Other issues on evaluation assumptions and methodologies are also discussed by companies.

- Google (R1-2208882) Proposal 2: The study of AI/ML based AI/ML based positioning should take random phase offset between cells into account.

- MediaTek (R1-2209510) Proposal 4: For comparison of evaluation on AI/ML-based LOS/NLOS identification, it is suggested to align the sampling period in dataset.

- Fujitsu (R1-2209015) Proposal 2 It is suggested to have more studies on the imperfect or noisy labels for both the benefits and the potential disadvantages, companies are encouraged to provide more detailed and general evaluation results when using imperfect labels.

- Nokia (R1-2209371) Observation-4: The availability of good quality data with sufficient diversity of positioning ground truth labels with accurate information, for model training and testing/validation is one of the key challenges in AI/ML based positioning. Observation-5: It is important to note that training dataset size as an indication of user area density is valid only for uniform distribution of UEs within the simulation setting. Proposal-6: For evaluation of AI/ML based positioning, consider additional UE distribution options such as sparse or clustered deployment of UEs, while evaluating model performance. Proposal-7: For evaluation of AI/ML based positioning, RAN1 should assess the need for standardizing the procedures for triggering and/or controlling and/or monitoring the ML model adaptation and fine-tuning after model deployment.

### 1st round discussion

One issue pointed out by MediaTek (R1-2209510) is the sampling period (or equivalently, sampling rate) in dataset. Simulation results were provided in R1-2209510 to illustrate the impact of using sampling period of 1 ns vs 9.3 ns.

The issue of sampling period has not been discussed in previous meetings, and it is true that alignment between companies is necessary for fair comparison.

Moderator’s understanding is, the sampling period is a function of subcarrier spacing. Specifically, the sampling period can be calculated as follows:

Sampling rate = Nf×∆f

Sampling period = 1/(Nf×∆f)

Where FFT size Nf=4096 according to 38.211, and ∆f is subcarrier spacing.

Thus, for the agreed subcarrier spacing, the sampling periods are obtained as following for FR1 and FR2.

|  | FR1 | FR2 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 30kHz | 120kHz |
| Sampling period (ns) | 8.13 | 2.03 |

Thus the following is proposed.

Proposal 2.4.1-1

For evaluation of AI/ML based positioning, the sampling period is aligned to: 8.13 ns for FR1 and 2.03 ns for FR2.

|  | Company |
| --- | --- |
| Support | MediaTek, [Ericsson] |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | Question for clarification. Is the intention to align the size of model input among companies? If so, we can discuss this proposal after we reach agreement on proposal 4.1-1 (when we all use the same channel measurement (i.e., CIR) as model input for training/inference. |
| Samsung | We feel it’s just important for using the same settings for tratining data and inferernece data, we don’t need to limit to any specific type. |
| ZTE | Agree with vivo. |
| HW/HiSi | We do not see a need for this proposal. We share the moderator’s understanding how the sampling period is obtained from the agreed subcarrier spacing. Thus, the sampling rate should be already clear. |
| Ericsson | We are OK to have this proposal explicitly captured to avoid confusion. Alternatively, if all companies agree with how the sampling period is obtained from the agreed subcarrier spacing, then it is also OK not to have explicit agreement. |

### 2nd round discussion

For Proposal 2.4.1-1, there is no broad support to make explicit agreement about the sampling period. Could you please share your view on how to proceed? After checking companies’ views, we can further discuss if an explicit agreement is needed or not.

Question 2.4.2-1

Regarding the sampling period, which option to follow?

- Align sampling period between companies to facilitate performance comparison.

- No need to align sampling period between companies. Each company report the sampling period used in their evaluation.

|  | Company |
| --- | --- |
| Option 1 |  |
| Option 2 | Vivo, OPPO, Samsung, NVIDIA, Nokia/NSB, Apple, Fujitsu, CAICT, CATT, Xiaomi,CMCC, Qualcomm, Hw/HiSi, LG |

Question 2.4.2-2

If Option (1), which value to align to?

- Default sampling period = 8.13 ns for FR1 and 2.03 ns for FR2.

- Other values (please describe).

|  | Company |
| --- | --- |
| Option 1A |  |
| Option 1B |  |

| Company | Comments |
| --- | --- |
| vivo | Unless we can agree on a common channel measurement as model input, we feel no need to align sampling period. |
| ZTE | It’s better to report by companies about their sampling rate. If companies want to evaluate model generalization over different sampling rates, it’s always welcome. |
| Samsung | Same view as vivo and zte, and also our comment in previous round. |

### 3rd round discussion

According to 2nd round discussion, companies prefer to leave it up to each company to select the sampling period. Thus the following conclusion is suggested.

Proposal 2.4.3-1

For evaluation of AI/ML based positioning, the sampling period is selected by proponent companies. Each company report the sampling period used in their evaluation.

|  | Company |
| --- | --- |
| Support | CMCC, CAICT |
| Not support |  |

| Company | Comments |
| --- | --- |
| Fujitsu | If our understanding is correct, the sampling rate has been agreed in the subcarrier spacing parameter defined in the table of TR38.857. Is that necessary for companies to simulate different subcarrier spacing other than the agreed EVM from 38.857? |

# KPI

In the following, the remaining issues on KPI are discussed.

## (Closed) Intermediate performance metric of assisted AI/ML positioning

In RAN1#110, the following agreement was made. In this meeting, several companies have suggestions on how to resolve the FFS bullet.

Agreement

For evaluation of AI/ML assisted positioning, an intermediate performance metric of model output is reported.

- FFS: Detailed definition of the intermediate performance metric of the model output

- Ericsson (R1-2208399) Proposal 3 For evaluation of AI/ML assisted positioning with LoS classification and ToA estimation as the intermediate model outputs, at least LoS classification accuracy and CDF percentile of ToA estimate errors (preferrably expressed in meters) should be reported as the intermediate performance metrics.

- LG (R1-2208903) Proposal #2: Determine intermediate performance metric with respect to sub use case. - At least for LOS/NLOS identification, the meaning of the value and the corresponding format is to be decided with high priority.

- CATT (R1-2208971) Proposal 2: For AI/ML-based positioning, in addition to AI-specific KPIs, the following KPIs are considered: - Eventual KPI: Positioning accuracy (e.g. 90% CDF percentiles of horizontal accuracy); - Intermediate KPI: The accuracy of intermediate measurement results (e.g. error of ToA/AoA/AoD) if estimating timing and/or angle of measurement based on AI/ML model is applied; - Intermediate KPI: The correct rate of LOS/NLOS identification if identifying LOS/NLOS identification based on AI/ML model is applied.

- Nokia (R1-2209371) Proposal-4: For evaluation of two-step or AI/ML assisted positioning, intermediate KPI(s) such as the accuracy of LOS/NLOS identification, accuracy of timing and/or angle of measurement, accuracy of the likelihood measurement, etc., should be reported together with the horizontal positioning accuracy.

- Faunhofer (R1-2209537) Proposal 5: For the evaluation of performance gain resulting from additional reporting for input data to the AI/ML model (network based ToA estimator, for example) the main performance metric shall be the ToA error statistic. Proposal 6: For the evaluation of performance gain resulting from additional reporting for output data to the AI/ML model system level simulations including positioning algorithms are required and the main performance indicator is the positioning accuracy and the probability of false positions.

### 1st round discussion

Based on the proposals in companies’ contributions, the following proposal seems to capture the majority view.

Proposal 3.1.1-1

For evaluation of AI/ML assisted positioning, the intermediate performance metrics include:

- LoS classification accuracy, if the model output includes LoS classification;

- ToA estimation accuracy (expressed in meters), if the model output includes ToA.

|  | Company |
| --- | --- |
| Support | Nokia/NSB, NVIDIA, InterDigital, Fujitsu, LG, Ericsson |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | Question for clarification. Is the reported accuracy for each link (corresponding to each TRP)? Or for each path since there may be multiple paths between the UE and a TRP? |
| Nokia/NSB | In our view, reporting the overall accuracy metrics could also be sufficient. |
| Apple | To clarify, ToA estimation accuracy (expressed in meters) = abs(ToA_estimate – ToA_actual) x speed_of_light [Moderator] Yes, I think this is common understanding. |
| LG | In addition, for LOS/NLOS identification, the meaning of the value range (e.g. [0,1]) and the corresponding format of the value are to be decided with high priority where it can be reused as in Rel-17 or newly defined by taking AI/ML specific components into account. |
| OPPO | Suggest to add the following metric RSTD estimation accuracy, if the model output includes RSTD |
| Qualcomm | Please include accuracy of soft-information: - Timing soft-information accuracy (e.g., mean ToA error and variance of ToA), if the model output includes timing soft information. [Moderator] If ToA takes soft value (e.g., probability distribution of the TOA), how to report its accuracy? Please clarify. I imagine { mean ToA error, variance of ToA } is the soft output of ML model. |
| CMCC | Mean TOA error can be used. |
| Samsung | Wording change: For evaluation of AI/ML assisted positioning, the following intermediate performance metrics are used include: - LoS classification accuracy, if the model output includes LoS classification; - ToA estimation accuracy (expressed in meters), if the model output includes ToA. |
| ZTE | Generally fine with this proposal. We think second bullet should be more general. - Timing/angle estimation accuracy (expressed in meters/degrees), if the model output includes timing/angle estimation. If we would like to give example of timing estimation, RSTD should also be included. |
| HW/HiSi | Support the proposal with the understanding that the intermediate accuracy is reported per link (not per path). |
| CATT | Fine with this proposal and also fine with ZTE’s update. |
| CAICT | We also fine to include RSTD. |
| Lenovo | Support |

### 2nd round discussion

Based on the feedback from companies, Proposal 3.1.1-1 is updated. Note that “LOS/NLOS indicator” phrase is used to align with existing phrase in 37.355.

When the ML output is a soft value that represents a probability distribution (e.g., probability of LOS, {mean of ToA, variance of ToA} ), there is no clear description on intermediate performance metrics in companies’ contributions. Thus this part is left as FFS. Companies are invited to share views on how to define the intermediate performance metrics for such soft value outputs.

Proposal 3.1.2-1

For evaluation of AI/ML assisted positioning, the following intermediate performance metrics are used:

- LoS classification accuracy, if the model output includes LOS/NLOS indicator of hard values;

- Timing estimation accuracy (expressed in meters), if the model output includes timing estimation (e.g., ToA, RSTD).

- Angle estimation accuracy (in degrees), if the model output includes angle estimation (e.g., AoA, AoD).

- FFS. Whether/how to report intermediate performance metrics if the ML output is a soft value that represents a probability distribution (e.g., probability, mean and variance).

|  | Company |
| --- | --- |
| Support | ZTE, OPPO, NVIDIA, Nokia/NSB, CAICT, InterDigital |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | We haven’t get the answer from our previous clarification question. Is the reported accuracy for each link (corresponding to each TRP)? Or for each path since there may be multiple paths between the UE and a TRP? By definition in 37.355, for example, LOS/NLOS indicator is for a path “This field provides information on the likelihood of a Line-of-Sight propagation path from the source to the receiver” We can support this proposal if the accuracy report is per UE/TRP, not per path. |
| ZTE | As commented by vivo, we may need to FFS whether the intermediate result is per path or per TRP since current spec supports per path reporting. |
| OPPO | We support vivo’s proposal that the accuracy report is per UE/TRP, not per path. |
| Nokia/NSB | We are fine with the accuracy report that takes per UE/TRP link into consideration. |
| Apple | Please clarify the need for the following statement in the first bullet->„LOS/NLOS indicator of hard values”. Why not soft values ? Also from our interpretation, the LOS/NLOS indicator in 37.355 seems to be per UE/TRP and not per path. [Moderator] If soft value, then it indicates probability, and belong to the FFS bullet. ML can generate soft value, but it’s unclear how to define intermediate performance metric for it. See also reply to LG below. Note: from RAN1 107-e, the following conclusion was made: Conclusion Do not support LoS/NloS indicators for additional paths. |
| vivo | To Apple: To clarify further. We’re not referring to additonal path report, but the LOS/NLOS indicator report for different path from the same TRP. For example, there could be multiple PRS resources from the same TRP and UE may report RSTD for each of those PRS for the same TRP. Along with each RSTD, there coube LOS/NLOS indicator. That’s carried insiode IE NR-DL-TDOA-AdditionalMeasurementElement-r16. Similar thing for DL-AoD and Multi-RTT. Our understanding of AI/ML model trained to output LOS/NLOS indicator is currently for a TRP, not per PRS resource level yet. From RAN1#106bis-e Agreement: - For UL-TDOA, UL-AoA and Multi-RTT one LoS/NLoS indicator can be associated with each UL RTOA, UL SRS RSRP, UL-AoA and/or gNB Rx-Tx time difference measurement, respectively, and reported by gNB for each TRP that performed measurements for a given UE - For UL-TDOA, UL-AoA and Multi-RTT one LoS/NLoS indicator can be associated and reported by a TRP for a given UE - For DL-AoD and Multi-RTT one LoS/NLoS indicator can be associated with each DL PRS RSRP and/or UE Rx-Tx time difference measurement, respectively, and reported by UE for each TRP - For DL-AoD and Multi-RTT one LoS/NLoS indicator can be associated with each TRP in the measurement report from the UE - For DL-TDOA one LoS/NLoS indicator can be associated with each RSTD measurement performed with a target TRP and one LoS/NLoS indicator is associated with the RSTD measurement performed with a reference TRP - For DL-TDOA one LoS/NLoS indicator can be associated with each target TRP and one LoS/NLoS indicator can be associated with the reference TRP in the measurement report - FFS: Dependence of indication of a LOS/Nlos indicator on the presence of Rx beam index for DL-AoD - FFS: Whether the above bullets apply to additional path measurements. |
| CATT | We support vivo that the accuracy report is per UE/TRP, not per path, which should be described in the proposal. |
| Xiaomi | We also support vivo’s view |
| CMCC | We have one clarification question. Dose the Timing estimation accuracy/ Angle estimation accuracy means the average Timing estimation accuracy/ Angle estimation accuracy among all the testing samples? |
| Qualcomm | We prefer different wording of the proposal: For evaluation of AI/ML assisted positioning, the following intermediate performance metrics are used: - LoS classification accuracy, if the model output includes LOS/NLOS indicator of hard values; - Timing estimation accuracy (expressed in meters), if the model output includes timing estimation (e.g., ToA, RSTD). - Angle estimation accuracy (in degrees), if the model output includes angle estimation (e.g., AoA, AoD). - Companies provide info on how LoS classification accuracy and Timing/Angle estimation accuracy are evaluated, if the ML output is a soft value that represents a probability distribution (e.g., probability of LOS, probability of timing, probability of angle, mean and variance of Timing/Angle, etc.) |
| HW/HiSi | We echo vivo, that it has not been clarified yet, whether the accuracy for LOS is for each link or each path, per link is our understanding. For LOS identification, we wonder if it really is meaningful to report the LOS classification accuracy, since it does not reflect the final positioning accuracy. If one UE e.g. has 6 LOS links, then the position is determined by 3 links, and the classification accuracy of the remaining links is not important. For LOS identfication, we suggest that the LOS classification accuracy can be reported optionally in addition to the final position accuracy: Updated Proposal 3.1.2-1 For evaluation of AI/ML assisted positioning, the following intermediate performance metrics are used: - (optional) LoS classification accuracy, if the model output includes LOS/NLOS indicator of hard values; - Timing estimation accuracy (expressed in meters), if the model output includes timing estimation (e.g., ToA, RSTD). - Angle estimation accuracy (in degrees), if the model output includes angle estimation (e.g., AoA, AoD). - FFS. Whether/how to report intermediate performance metrics if the ML output is a soft value that represents a probability distribution (e.g., probability, mean and variance). [Moderator] It does not seem necessary to add optional to LOS/NLOS, since the bullet already has the condition “if the model output includes...“ |
| LG | To our understanding, there is no need to confine the LOS/NLOS indication with hard value. For LOS/NLOS identification, the value range for the indicator has been [0,1] in Rel-17 and it can indicate the accuracy in itself when the value close to 0 or 1, it can be regarded as NLOS and LOS, respectively and it is vague when the value is 0.5 (half and half for NLOS/LOS). In this sense, the original wording is preferred at least for LOS/NLOS sub-bullet. [Moderator] This explanation is about how to interpret the info carried by soft ML output. But the intermediate performance metrics should be about the quality of the probability, right? For example, assume the ML output reports a link to be 90% LOS. There can be cases like below: - The ML output is very good: this link is indeed very close to LOS. - Or, the ML output is very bad: this link is in fact NLOS (e.g., strong reflection), but ML recommends it as LOS with high confidence. The intermediate metric need to differentiate the cases above. |

### 3rd round discussion

Companies actively shared information on how to improve Proposal 3.1.2-1. Based on companies’ input, the updated proposal is presented below for discussion.

Proposal 3.1.3-1

For evaluation of AI/ML assisted positioning, the following intermediate performance metrics are used:

- LoS classification accuracy, if the model output includes LOS/NLOS indicator of hard values, where the LOS/NLOS indicator is generated for a pair of UE and TRP;

- Timing estimation accuracy (expressed in meters), if the model output includes timing estimation (e.g., ToA, RSTD).

- Angle estimation accuracy (in degrees), if the model output includes angle estimation (e.g., AoA, AoD).

- Companies provide info on how LoS classification accuracy and timing/angle estimation accuracy are estimated, if the ML output is a soft value that represents a probability distribution (e.g., probability of LOS, probability of timing, probability of angle, mean and variance of timing/angle, etc.)

|  | Company |
| --- | --- |
| Support | CAICT |
| Not support |  |

| Company | Comments |
| --- | --- |
| CMCC | To our understanding, the second and third bullet should be Average timing estimation accuracy/ Average angle estimation accuracy. For each data sample from the testing dataset, ToA estimation accuracy (expressed in meters) = abs(ToA_estimate – ToA_actual) x speed_of_light. The intermediate performance metrics should be the average estimation accuracy of all the testing data samples. |

## Other

Other views on KPI were also submitted by companies in their contributions, see below. Considering the common KPI agreement made under AI 9.2.1 in RAN1#110, the proposals below can be revisited if/when further agreements are made under AI 9.2.1 to include these metrics.

- Lenovo (R1-2209124) Proposal 2: The positioning AI/ML model evaluation methodology should support scenarios evaluating a model’s robustness and adaptability, e.g., including how often an AI/ML evaluation model is updated based on a particular generalizability evaluation criterion. FFS any other relevant criteria. Proposal 3: In addition to FLOP counts, the evaluation should also consider the hardware and software platforms used to evaluate the positioning AI/ML algorithms, type of data being used as input, training type, e.g., offline vs online, complexity type, e.g., worst-case/average-case.

- Nokia (R1-2209371) Proposal-5: Optional KPIs such as position estimation latency, radio resource efficiency and higher layer signaling overhead should be reported together with the horizontal positioning accuracy.

# Input, output

Regarding input and output of the ML model, selected proposals are copied below.

- ZTE (R1-2208525) Proposal 1: For evaluations on AI for positioning, at least include following sub-use cases for direct AI/ML positioning and AI/ML assisted positioning: - Direct AI/ML positioning - Sub-use case 1-1: AI model input is path timings and RSRPPs from single port PRS - Sub-use case 1-2: AI model input is CIR (i.e., path timings, RSRPPs and path phases) from single port PRS - Sub-use case 1-3: AI model input is path timings and RSRPPs (or CIR) from multi-port PRS - AI/ML assisted positioning - Sub-use case 2-1: AI model output is DL PRS RSTD values - Sub-use case 2-2: AI model output is LOS/NLOS indicator Observation 3: With path phase information included in the AI model input, the positioning performance is improved obviously when compared to AI model input without path phase information. Proposal 3: Study and evaluate the performance of direct AI/ML positioning when AI model input includes path phase information. Observation 4: With measurement results from multi-port PRS included in the AI model input, better positioning performance can be observed when compared to AI model input only includes measurement results from single port PRS. Proposal 4: Study and evaluate the performance of direct AI/ML positioning when AI model input includes measurement results from multi-port PRS. Observation 5: The AI/ML based positioning method has excellent accuracy on the estimation of DL PRS RSTD values even in heavy NLOS conditions. Observation 6: AI/ML assisted positioning achieves better positioning performance than direct AI/ML positioning to some degree. Proposal 5: Study and evaluate the performance of AI/ML assisted positioning where the AI model output includes DL PRS RSTD values. Observation 7: The AI/ML based positioning method has a good accuracy rate of LOS/NLOS identification. Proposal 6: Study and evaluate the performance of AI/ML assisted positioning where the AI model output includes confidence levels of LOS/NLOS identification.

- vivo (R1-2208638) Proposal 2: Capture in the TR that time domain CIR as the model input for direct AI/ML positioning obtains the best performance compared to other model inputs. Proposal 3: Support time domain CIR as the model input at least for direct AI/ML positioning.

- Huawei, HiSilicon (R1-2208433) Proposal 5: For comparison of evaluation on AI/ML-based fingerprint positioning evaluation results, support the channel impulse responses (CIRs) as the model inputs.

- Google (R1-2208882) Proposal 5: For direct AI/ML positioning, consider to use CIR and L1-SINR from each cell as the input.

- Nokia (R1-2209371) Observation-7: The UE/TRP can report only the timing and RSRP values, and the signaling of CIRs from the UE to the network is not supported. Observation-8: If CIR is agreed as a baseline model input, that would imply that only UE-based direct or AI/ML assisted positioning methods are considered. However, in such a scenario, there might be challenges related to acquiring labeled training data from other UEs or from the network.

## 1st round discussion

Based on companies’ input, there is wide support to have CIR as model input. Thus the following is proposed.

Proposal 4.1-1

For evaluation of direct AI/ML positioning, support at least the time-domain channel impulse response (CIR) as the model input.

|  | Company |
| --- | --- |
| Support | vivo, NVIDIA, Apple, Fujitsu, LG, OPPO,CMCC, HW/HiSi, CATT, CAICT, Lenovo, Ericsson |
| Not support | Qualcomm |

| Company | Comments |
| --- | --- |
| Nokia/NSB | In our understanding, some companies have considered downlink CIR as the model input, whereas some have considered uplink CIR obtained from SRS estimation. Perhaps it is important to clarify which time-domain CIR value is considered here. We had also raised the question and would be interested to know other company views regarding how for e.g., downlink CIR information could be widely available across Ues and at the network-side, such that a dataset could be generated for testing and training. |
| Qualcomm | We do not understand the motivation of the proposal but generally prefer not to specify input of the AI/ML model. If the purpose to have this an observation of what has been used by companies for evaluation, then we are ok. |
| Samsung | We understand there is “at least” in the proposal, but still want to ask, why only time domain CIR is picked out. Because as we can see, the final TR will just capture the model input being used in the evaluation, the spec impact for a certain type of model input is under 9.2.4.2. Besides, this is not to try to exclude any model input to be evaluated. |
| ZTE | Current specification doesn’t support UE/gNB to report CIR (i.e., path timing and path RSRP without path phase). If this proposal is to discuss the baseline for evaluating direct AI/ML positioning, which should be based on the measurement that is already supported in spec. Then, we can further evaluate the performance gain over baseline method when using other model input. We have following suggestions. For evaluation of direct AI/ML positioning, support power delay profile (PDP, i.e., path timing and path RSRP) as a baseline for the model input. Companies are encouraged to evaluate other model input, - Option 1: time-domain channel impulse response - Option 2 : other model inputs are not precluded. [Moderator] We can discuss the suggestion above later. Let’s start with the most commonly used model input type according to companies’ evaluations. |
| HW/HiSi | Support |

## 2nd round discussion

Based on companies’ input, it is generally acknowledged that CIR is a typical model input used in evaluation. QC can accept this to be reflected as an observation. Thus the following is proposed.

Proposal 4.2-1

Observation: For direct AI/ML positioning, time-domain channel impulse response (CIR) is a typical model input used in the evaluation.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | In the 1st round, previous proposal 4.1-1 actually align the mode input for evaluation. However, we’re not sure about the intention of this proposal or what progress we’re making here. We suggest to capture observations of performance gain of direct AI/ML positioning together with what has been used as model input rather than a single and separate bullet saying CIR is a typical input. |
| ZTE | We don’t understand what the “typical” means here. If we want to agree a baseline for comparison, it’s better to use measurement that is already supported in specification as we commented in last round. |
| OPPO | Not sure what the additional value of this observation is. The previous proposal makes more sense to align the inptus for evaluation. |
| Samsung | Just remove the “typical“. |
| NVIDIA | The meaning of “typical“ is unclear as an observation. |
| Nokia/NSB | We share similar concerns as other companies, especially the issue raised by ZTE. Does this proposal implicitly indicate that a new measurement would be supported in the specification related to CIR? If not, we are not sure as to how this observation would work in practice. |
| Apple | Agree with ZTE. On the CIR dimension, should there be a baseline dimension in time e.g. 256 or should each company report their preference ? |
| Fujitsu | Same concern here, what is the motivation for taking the CIR as a typical input? If a new measurement need to be included in the future specification, it should be discussed in 9.2.4.2. |
| CAICT | Prefere remove typical. |
| CATT | We prefer the original proposal, i.e., Proposal 4.1-1. The updated proposal has confusion on the meaning of “typical”. |
| InterDigital | The meaning of “typical” is not clear. For an observation, it is better to capture how many companies used a certain input (e.g., CIR, RSRP, etc), for direct AIML positioning. |
| Xiaomi | Just wondering how to apply this proposal (if it is agreed) in the furture discussion. Is the next step to discuss the potential specification impact of setting CIR as input ？ |
| CMCC | Prefer remove typical, and some CIR dimension can be reported by each company. |
| Qualcomm | We are ok with the observation, but it should be noted that the model input for evaluation is a company choice, and this is not meant to restrict input for evaluation. |
| HW/HiSi | Support |
| LG | The original version is preferred. |

Regarding AI/ML assisted positioning, LOS/NLOS identification is widely used as model input in companies’ evaluations. Also LOS/NLOS identification is an existing IE, thus reasonable to capture it, as proposed by some companies. Similar to existing LOS/NLOS indicator, the value can be reported as hard or soft value.

37.355:

LOS-NLOS-Indicator-r17 ::= SEQUENCE {

indicator-r17			CHOICE {

soft-r17				INTEGER (0..10),

hard-r17				BOOLEAN

},

…

}

LOS-NLOS-Indicator field descriptions

indicator This field provides information on the likelihood of a Line-of-Sight propagation path from the source to the receiver and has the following choices: - soft: This field specifies the likelihood of a LOS propagation path in the range between 0 and 1 with 0.1 steps resolution. Value ‘0’ indicates NLOS and values ‘1’ through ‘10’ provide an estimate of the propability for a LOS propagation path between source and receiver. Scale factor 0.1; range 0 to 1. - hard: This field specifies whether the propagation path between source and receiver is estimated to be LOS (true) or NLOS (false).

Proposal 4.2-2

For AI/ML assisted positioning, support at least the LOS/NLOS indicator as model input. The LOS/NLOS indicator can have hard value (Boolean) or soft value (likelihood of a LOS propagation path).

|  | Company |
| --- | --- |
| Support |  |
| Not support | vivo, CATT, InterDigital,HW/HiSi |

| Company | Comments |
| --- | --- |
| vivo | 1. First of all, do you mean LOS/NLOS indicator as model output? 2. LOS/NLOS indicator is not even mentioned in the 1st round of discussion. Now there’s a proposal in the 2nd round asking to „support LOS/NLOS indicator for AI/ML assisted psoitioning“. Moderaotor satetd „LOS/NLOS identification is widely used as model input in companies’ evaluations“. CIR was even widely used by companies for direct AI/ML positioing evalution, while proposal 4.1-1 proposing support CIR for evalution in the 1st round and proposal 4.2-1 proposing an observation where CIR is a typical model input used in evaluation in the 2nd round. 3. If the intention eh a capture observations based on evalution, then we request a fair treatment as CIR for model input and other evaluated model input/output. 4. We also have technical concern on LOS/NLOS indicator as model input for its‘ limitation. As moderator well summarized in proposal 6.6-3, there was an observation based on evaluation results: even with high-quality input (e.g., LOS/NLOS identification) provided by ML model, positioning accuracy eh av conventional positioning algorithm is limited by the inadequate LOS links in heavy NLOS scenario. |
| Nokia/NSB | eh ave a similar question as vivo, in terms of whether model input or output is meant in the proposal. |
| Apple | Agree with Vivo and Nokia |
| Fujitsu | If the moderator means the model input, then it may be useful for conventional methods but not AI for knowing the los/nlos indicator values. If the moderator means the model output, it has no problem to have this value as one of the potential output, but why we need to explicitly point out the LOS/NLOS indicator since no related discussion before. |
| CATT | Agree with vivo that LOS/NLOS indicator should be as model output. Moreover, based on companies simulation results, most companies also use TOA as model output for AI/ML assisted positioning. We think it should be further studied and compair companies simulation results. |
| InterDigital | We have a concern for this proposal. The LOS/NLOS indicator is derived using implementation and reliability of the indicator is not clear. Thus, the indicator should not be mandated to be used as an input for AIML based positioning. |
| Qualcomm | The proposal is not clear whether the model here is AI/ML or classical one. We do not see a need to specify AI/ML model output for AI/ML assisted positioning at this stage of the study. |
| HW/HiSi | We do not support if „model input“ is meant. There was no discussion about that earlier. Is this a typo? |
| ZTE | To our understanding, LOS/NLOS should be model output for AI/ML assisted positioning. |

## 3rd round discussion

Regarding Proposal 4.2-1, there are divergent views. The goal is to have useful agreements that help with RAN1 progress. In the following, the following is proposed, inspired by companies’ comments.

Proposal 4.3-1

For direct AI/ML positioning, if time-domain channel impulse response (CIR) is used as model input in the evaluation, companies report the CIR dimension NTRP * Nport * Nt, where NTRP is the number of TRPs, Nport is the number of receive antenna ports, Nt is the number of time domain samples.

- Baseline value for Nt: Nt = 256.

|  | Company |
| --- | --- |
| Support | CMCC, CATT, CAICT |
| Not support |  |

| Company | Comments |
| --- | --- |
| Fujitsu | Basically fine to us. For the value for Nt, it seems no need to give a designated value as baseline, we did one of our simulation as Nt = 256 but no specific technical background for this value. Anyway if other companies prefer to have 256 as the baseline we have no problem for it. |

For Proposal 4.2-2, there was a typo ☹, i.e., it should be “model output”. This is corrected below. Please check.

Proposal 4.3-2

For AI/ML assisted positioning, support at least the LOS/NLOS indicator as model output. The LOS/NLOS indicator can have hard value (Boolean) or soft value (likelihood of a LOS propagation path).

|  | Company |
| --- | --- |
| Support | CMCC |
| Not support | vivo, CATT |

| Company | Comments |
| --- | --- |
| Fujitsu | It will be better to define the predicted TOA as one of the model output together with the LOS/NLOS indicator. |
| vivo | As we stated in the 2nd round,we have technical concern on LOS/NLOS indicator as model output for its‘ limitation. As moderator well summarized in proposal 6.6-3, there was an observation based on evaluation results: even with high-quality input (e.g., LOS/NLOS identification) provided by ML model, positioning accuracy of conventional positioning algorithm is limited by the inadequate LOS links in heavy NLOS scenario. [Moderator] Not sure why this is a technical concern. If the radio environment does not give enough LOS links, the positioning performance is limited when applying the conventional positioning method. However, this does not mean there is no value to have LOS/NLOS indicator as output. The ML model can still help improving the positioning performance. For example, Ericsson R1-2208399 showed the following evaluation results. This demonstrates that the accurate LOS/NLOS indicator provided by ML is useful for improving positioning performance, except when there is almost no LOS available (e.g., LOS probability= 0.008 for clutter parameter {60%, 6m, 2m}). UE horizontal positioning accuracy at CDF=90% (meters) Clutter Parameter; Conventional positioning method (UTDOA); AI/ML-assited method ML output: LoS/NLoS classification, ToA. {40%, 2m, 2m}; 9.595; 0.106. {50%, 2m, 2m}; 16.775; 0.264. {60%, 2m, 2m}; 17.541; 5.340. {60%, 6m, 2m}; 15.849; 13.476. |
| CATT | As our previous comments, based on companies simulation results, most companies also use TOA as model output for AI/ML assisted positioning. Prefer to define both of TOA and LOS/NLOS indicator as model output. |

## 4th round discussion

For Proposal 4.3-1 (CIR), based on the limited response by companies, there was no objections. Thus it is copied below for further checking.

Proposal 4.4-1

For direct AI/ML positioning, if time-domain channel impulse response (CIR) is used as model input in the evaluation, companies report the CIR dimension NTRP * Nport * Nt, where NTRP is the number of TRPs, Nport is the number of receive antenna ports, Nt is the number of time domain samples.

- Baseline value for Nt: Nt = 256.

|  | Company |
| --- | --- |
| Support | Fujitsu, Samsung (except baseline), CATT, HW/HiSi (except baseline) , CAICT, Nokia/NSB |
| Not support |  |

| Company | Comments |
| --- | --- |
| Fujitsu | Same comments as the 3rd round. |
| Samsung | We just need company to report the value use, having a baseline for Nt is not meangingful. |
| ZTE | We should have PDP that is already supported in spec to see what is the performance gain if we use other channel information (e.g., CIR). In addition, for the Nt value, it’s technically belong to the path number. In current spec, only up to 8 additional paths are supported. So, we suggest to use 8 paths as a baseline. If we cannot go this way, we should allow companies to report its settings. For direct AI/ML positioning, if time-domain power delay profile(PDP)/channel impulse response (CIR) is used as model input in the evaluation, companies report the PDP/CIR dimension NTRP * Nport * Nt, where NTRP is the number of TRPs, Nport is the number of receive antenna ports, Nt is the number of time domain samples. [Moderator] Let’s start with CIR for now. Later we can check if companies support having a similar proposal for PDP. |
| HW/HiSi | We agree with the proposal in principle. For the baseline, even if many companies have used Nt=256, we agree with Samsung, that for now, we do not need a baseline and companies can just report the used value. |
| OPPO | “ Nport is the number of receive antenna ports“: The port should be removed since we haven’t defined “receive antenna ports“ [Moderator] If removing port, then it may be confused with antenna elements, right? |
| OPPO | We believe the same or simiar observation will be made in the further for the TR. However, we still think it is two early to have an “overall“ observation since companies are still planing to submit more evaluation resutls, especially for the generalization performance. As Fujitsu commented, this observation is based on some specific evalaution assumption. We suggest to focus on the observation for specific evalaution assumption/ scenarios, rather than rush into an “overall“ observation without any condition in this stage. |
| Nokia/NSB | We are fine with the proposal. However, it would be good to clarify the details related to what is the criterion to choose 256 samples, are these first 256 samples or some kind of filtering required for the samples. |
| Qualcomm | We are fine with the proposal, but there is no need to specify a baseline value for Nt. Would it also be relevant to explore number of TX ports (if applicable)? [Moderator] So far PRS and SRS are both single port. We can stay with this assumption for now. |

For Proposal 4.3-2 (LOS/NLOS), several companies recommended to include ToA also. Thus a bullet is added, i.e., ToA is supported if the corresponding conventional positioning method is based on timing estimation (i.e., not spatial estimation).

Vivo raised a question on technical merit of LOS/NLOS. In moderator’s understanding, evaluation results have demonstrated the technical merit, see reply to vivo comment in section 4.3.

Thus Proposal 4.3-2 is updated below.

Proposal 4.4-2

For AI/ML assisted positioning, support at least the LOS/NLOS indicator as AI/ML model output. The LOS/NLOS indicator can have hard value (Boolean) or soft value (likelihood of a LOS propagation path).

- If the corresponding conventional positioning method is based on timing estimation, support ToA estimation as the AI/ML model output.

|  | Company |
| --- | --- |
| Support | Nokia/NSB |
| Not support | vivo (need clarification) |

| Company | Comments |
| --- | --- |
| vivo | Thanks moderator’s reply to our comment in the 3rd round. Our point is that only LOS/NLOS indicator itself does not guarantee the performance as multiple evaluations showed and recognized by the moderator, for instance, in heavy-NLOS scenario. 1. It’s not clear to us what does ‘at least’ mean here. For example, is it implied that a model could be trained to output both LOS/NLOS indicator and another parameter? [Moderator] yes, that’s possible. Please see Ericsson R1-2208399 where one model can have both LOS/NLOS and ToA as output. 2. On the bullet added, need clarification as well on the wording “corresponding conventional positioning method”. [Moderator] Since it’s AI/ML assisted, there is a “corresponding conventional positioning method” that receives ML output. As I explained in my comment toward intermediate result accuracy in section 3, Rel-17 specified LOS/NLOS indicator is always associated and reported along with other measurements, DL-/UL-TDoA, DL-AoD and Multi-RTT, UL-AoA. There’s no independent report of LOS/NLOS indicator. Please refer to 37.355 for details. Let me give an example, NG-RAN node assisted positioning, gNB side model (trained to output LOS/NLOS indicator), AI/ML assisted positioning where the final positioning is done at LMF with conventional positioning method. Is it correct understanding that when gNB reports LOS/NLOS indicator (model output) associated with UTDOA, the model output should be both LOS/NLOS indicator and ToA? Or it means there’re 2 models at gNB, one output for LOS/NLOS indicator and one for ToA? [Moderator] It seems that both versions are possible for building the AI/ML model: - Output of one sub- ML model is LOS/NLOS indicator; ToA (or other measurements) is output of a separate sub- ML model. - ML model output have both LOS/NLOS indicator and ToA (or other measurements). |
| Fujitsu | We support adding TOA as the model output but we share the 2nd concern as VIVO commented above that some clarifications is needed here. In our 3rd round comment, we suggest to add the TOA as one of the possible output of the AI/ML assisted positioning due to the fact that a number of contributions (including ours) used TOA as the AIML model output for final positioning calculation, but typically the TOA output and LOS/NLOS indicator cannot be inferred by only one model. Therefore, when TOA is chosen as the output of the AIML assisted model, LOS/NLOS indicator cannot be the output at the same time, so the wording of this proposal may need some tiny modifications. [Moderator] Please see Ericsson R1-2208399 where one model can have both LOS/NLOS and ToA as output. |
| LG | Fine in general but the clarification seems needed. The proposal is not clear whether the model here is AI/ML only or with conventional scheme(s) is utilized. For LOS/NLOS identification, it would be estimated by AI/ML but the related other parameters, e.g. TOA is based on the conventional method and it can be the input of the AI/ML for estimating LOS/NLOS. |
| ZTE | Agree with CATT in last round, RSTD, TOA, LOS/NLOS indicator are commonly evaluated by companies, which should be treated equally. We propose to have two separate proposals: For AI/ML assisted positioning, support at least the LOS/NLOS indicator as AI/ML model output. The LOS/NLOS indicator can have hard value (Boolean) or soft value (likelihood of a LOS propagation path). For AI/ML assisted positioning, support RSTD/TOA as AI/ML model output. [Moderator] added “(or equivalently: DL RSTD, UL RTOA)”. |
| HW/HiSi | Support. |
| OPPO | The discussion is similar to the CIR as the input. We share simiar view as Fujitsu and ZTE that timing-basd output should be also included |
| Nokia/NSB | We are fine with the proposal. We agree with other companies that other intermediate measurements such as carrier phase (CP), AoA/AoD could be included as well. |
| Qualcomm | We do not see the need to specify the model output at this stage of the study. In addition, such a proposal should include other model outputs evaluated by companies. |

## 5th round discussion

For Proposal 4.4-1, there was concern to set a baseline value for Nt, even though Nt = 256 was a typical value used by companies’ in their evaluations. Thus the proposal is copied below without the sub-bullet.

Proposal 4.5-1

For direct AI/ML positioning, if time-domain channel impulse response (CIR) is used as model input in the evaluation, companies report the CIR dimension NTRP * Nport * Nt, where NTRP is the number of TRPs, Nport is the number of receive antenna ports, Nt is the number of time domain samples.

- Baseline value for Nt: Nt = 256.

Please comment if you have STRONG concern on the proposal above and cannot accept it.

|  | Company |
| --- | --- |
| STRONG concern | ZTE |

| Company | Comments |
| --- | --- |
| ZTE | We have a strong concern that we may over-estimate positioning performance if we only agree CIR as model input . For procedure wise, we should firstly identify the performance based on current specification(we already have evaluation results in our tdoc to use PDP as model input). Then, we can further explore whether some enhancements(e.g., CIR) can be made to further improve its performance. We cannot accept this proposal if PDP is not included. In last round, FL had a response to Qualcomm as follow: [Moderator] So far PRS and SRS are both single port. We can stay with this assumption for now. FL thinks we should stay with the assumption for sing port as it is in current spec. But CIR is not supported in current specification either, why those aspects are not treated equally? |
| Qualcomm | No strong concern but wanted to clarify that our understanding is that this proposal is not meant for specifying model input but to organize and clarify reporting of evaluation by companies. Some companies might consider multiple ports at TX side or both TX and RX sides. We propose the following updated wording: For direct AI/ML positioning, if time-domain channel impulse response (CIR) is used as model input in the evaluation, companies report the CIR dimension NTRP * Nport * Nt, where NTRP is the number of TRPs, Nport is the number of transmit/receive antenna ports pairs, Nt is the number of time domain samples. - Baseline value for Nt: Nt = 256. |

For Proposal 4.4-2, companies’ comment are taken into account to update the proposal. Regarding adding other types of ML model output, this can be left for future discussion.

Proposal 4.5-2

For AI/ML assisted positioning, support at least the LOS/NLOS indicator as AI/ML model output. The LOS/NLOS indicator can have hard value (Boolean) or soft value (likelihood of a LOS propagation path).

- If the corresponding conventional positioning method is based on timing estimation, support ToA (or equivalently: DL RSTD, UL RTOA) estimation as the AI/ML model output.

  - Here the conventional positioning method refers to the method that utilizes the output of AI/ML model inference to determine target UE location.

- FFS. Other types of AI/ML model output.

|  | Company |
| --- | --- |
| Support |  |
| Not support | Vivo, Qualcomm |

| Company | Comments |
| --- | --- |
| vivo | First of all, thanks moderator providing answers to our previous clarification questions. Our concern on the proposal is that currently ToA is worded as a bullet (i.e., not the same level as LOS/NLOS indicator). We interpreted that means LOS/NLOS indicator is supported as model output regardless. We acknowledge that an AI/ML model is trained to output both LOS/NLOS indicator and ToA in Ericsson’s Tdoc of R1-2208399. One question to ask is that, is there any comparison between an AI/ML model output both LOS/NLOS indicator and ToA and an AI/ML model trained to output ToA only? The reason is that in our evaluation, it is observed that AI/ML model trained to output ToA only achieves better performance than AI/ML model output LOS/NLOS indicator only. It is not clear to us why LOS/NLOS indicator a must if ToA is already reported by an AI/ML model. Before RAN1 evaluated and compared all those different model output options, we think it’s pre-mature to agree and decide now to “support LOS/NLOS indocator” |
| Fujitsu | We think the LOS/NLOS indicator and the ToA can be independent outputs, but in current proposal it seems that L/N indicator is a must and ToA can only be an optinal. |
| ZTE | Agree with vivo. All the timing and LOS/NLOS indicator should be treated equally rather than being a sub-bullet of the main bullet. |
| Qualcomm | We do not see a need to specify model output at this stage of the evaluation study. If there is a justified need to specify such an output, then the proposal should list all outputs considered by companies. |

## 6th round discussion

Based on the online discussion, the Proposal 4.5-1 (clean) is copied below for further checking.

Proposal 4.6-1

For direct AI/ML positioning, if time-domain channel impulse response (CIR) is used as model input in the evaluation, companies report the CIR dimension NTRP * Nport * Nt, where NTRP is the number of TRPs, Nport is the number of transmit/receive antenna port pairs, Nt is the number of time domain samples.

|  | Company |
| --- | --- |
| Support | ZTE |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | OK to have both PDP and CIR. |

As stated by ZTE, they do not object to CIR; they desire to discuss PDP as model input as well. Accordingly, the proposal on PDP is presented below for discussion. ZTE’s reason is, PDP is a type of existing measurement for positioning. At this moment, it is not clear to the moderator which existing measurement IE the proposed PDP is mapped to. @ZTE: please explain which existing IE the PDP refers to. To the best knowledge of the moderator, there is no existing IE that carries PDP of size 2x18x256 (example; as described in ZTE R1-2208525).

| Company | Comments |
| --- | --- |
| ZTE | - In our simulation, if we use PDP, the dimension of input data is 1x18x256 (please refer to Figure 1 in our tdoc). We acknowledge that current specification doesn’t support up 256 additional paths. This is why we think a baseline value for Nt is not necessary. In our contribution, we evaluate different path numbers via using PDP(e.g., Figure 2). As we stated in our contribution, if the path number is smaller than 256(section 2.1.1): For the cases when the number of paths (or delay tapes) fed into the AI model is smaller than 256, the remaining entries in the input data are set to zero. That is, the input data size keeps the same for all cases with different number of paths. - As for existing IEs, please see the following IEs from 37.355. It supports up to 8 additional path timings and path RSRP(i.e., DL PRS RSRPP), which definitely belongs to PDP. CIR is not supported in current specification as it requires phase information. NR-AdditionalPathList-r16 ::= SEQUENCE (SIZE(1..2)) OF NR-AdditionalPath-r16 NR-AdditionalPathListExt-r17 ::= SEQUENCE (SIZE(1..8)) OF NR-AdditionalPath-r16 NR-AdditionalPath-r16 ::= SEQUENCE { nr-RelativeTimeDifference-r16 CHOICE { k0-r16 INTEGER(0..16351), k1-r16 INTEGER(0..8176), k2-r16 INTEGER(0..4088), k3-r16 INTEGER(0..2044), k4-r16 INTEGER(0..1022), k5-r16 INTEGER(0..511), ... }, nr-PathQuality-r16 NR-TimingQuality-r16 OPTIONAL, ..., [[ nr-DL-PRS-RSRPP-r17 INTEGER (0..126) OPTIONAL ]] } -- ASN1STOP |

Proposal 4.6-2

For direct AI/ML positioning, if time-domain power delay profile (PDP) is used as model input in the evaluation, companies report the PDP dimension NTRP * Nport * Nt, where NTRP is the number of TRPs, Nport is the number of transmit/receive antenna port pairs, Nt is the number of time domain samples.

|  | Company |
| --- | --- |
| Support | ZTE |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | Our understanding on the intention of both proposal 4.6-1 and 4.6-2 is just asking comapnies to report the details of model input used in their evaluation. We don’t see a strong reason to have multiple proposals for the same purpose corresponding to each model input. Furthermore, reporting model input details should apply to AI/ML assisted positioning as well. Our suggested wording For the model input used in evalutions of AI/ML based positioning, companies report the details including the dimension NTRP * Nport * Nt, where NTRP is the number of TRPs, Nport is the number of transmit/receive antenna port pairs, Nt is the number of time domain samples. |
| Nokia/NSB | We are fine with vivo’s proposed update. It would make sense to merge both proposals. We also agree that the proposal might be applicable to AI/ML assisted positioning as well. |
| Moderator | Agree with vivo and Nokia/NSB that the same proposal is applicable to AI/ML assisted positioning. Regarding vivo’s suggested wording, it seems too general, i.e., other types of input may not have these dimensions. Thus the proposal is updated to Proposal 4.6-3 below. |
| vivo | On moderator’s comment, even if „other types of input may not have these dimensions“, the details of model input still need to be reported, no? |
| Qualcomm | It seems that different companies may end up having different assumptions on how to construct PDP. We suggest a slightly better wording and adding a note that asks companies to provide details on how they construct PDP: For the model input used in evalutions of AI/ML based positioning, if time-domain channel impulse response (CIR) or power delay profile (PDP) is used as model input in the evaluation, companies report the input dimension NTRP * Nport * Nt, where NTRP is the number of TRPs, Nport is the number of transmit/receive antenna port pairs, Nt is the number of time domain samples - Note: CIR and PDP may have different dimensions. - Note: Companies provide details on their assumption on how PDP is constructed and how (if applicable) it is mapped to Nt samples. |
| CATT | We also perfer to merge the two proposal and slightly prefer the QC’s suggestion. |
| Moderator | QC version looks good to me also. It’s copied below as Proposal 4.6-4 for all to further check. |

Proposal 4.6-3

For the model input used in evalutions of AI/ML based positioning, if time-domain channel impulse response (CIR) or power delay profile (PDP) is used as model input in the evaluation, companies report the CIR or PDP dimension NTRP * Nport * Nt, where NTRP is the number of TRPs, Nport is the number of transmit/receive antenna port pairs, Nt is the number of time domain samples.

- Note: CIR and PDP may have different dimensions.

Proposal 4.6-4

For the model input used in evalutions of AI/ML based positioning, if time-domain channel impulse response (CIR) or power delay profile (PDP) is used as model input in the evaluation, companies report the input dimension NTRP * Nport * Nt, where NTRP is the number of TRPs, Nport is the number of transmit/receive antenna port pairs, Nt is the number of time domain samples.

- Note: CIR and PDP may have different dimensions.

- Note: Companies provide details on their assumption on how PDP is constructed and how (if applicable) it is mapped to Nt samples.

# Performed evaluation of direct AI/ML positioning

In this meeting, a large amount of evaluation work has been performed by companies for direct AI/ML positioning. These valuable results are very important to help RAN1 to make progress.

Selected results submitted by companies are copied below.

## Evaluation without generalization considerations

- Huawei, HiSilicon (R1-2208433) Table 3. Evaluation results for AI/ML model deployed on network-side, without model generalization Model structure; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. ResNet; CIR; UE coordinates; UE coordinates; {60%, 6m, 2m}; 25000; 5000; 34 K; 10M; 0.492. {40%, 2m, 2m}; 0.606.

- ZTE (R1-2208525) Figure. 4 CDFs of positioning errors for sub-use case 1-2 (Grid width is 0.5 m)

- vivo (R1-2208638) Table 2. Evaluation results of different model inputs for AI/ML model deployed on UE or Network side, without model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Train; Test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6,6,2}; 25k; 1k; 1.65M; 22.30M; 0.99. Power + delay + angle of the first path; Pos.; 0; {0.6,6,2}; 25k; 1k; 1.65M; 22.30M; 1.19. Power + delay of the first path; Pos.; 0; {0.6,6,2}; 25k; 1k; 1.65M; 22.30M; 1.31. Delay + angle of the first path; Pos.; 0; {0.6,6,2}; 25k; 1k; 1.65M; 22.30M; 1.43. Angle + power of the first path; Pos.; 0; {0.6,6,2}; 25k; 1k; 1.65M; 22.30M; 1.79. Observation 2: Different inputs of AI model will affect the positioning performance for AI/ML based positioning. Time domain channel CIR as the input of AI model obtains the best positioning accuracy. Proposal 2: Capture in the TR that time domain CIR as the model input for direct AI/ML positioning obtains the best performance compared to other model inputs. Proposal 3: Support time domain CIR as the model input at least for direct AI/ML positioning.

- Qualcomm (R1-2209980) Table 2 Evaluation results for AI/ML model deployed on UE-side, without model generalization, CNN Method; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. RFFP; CIR (2, 18,400); 2D location; 0%; {60%, 6m, 2m}; 15000; 2000; 1.5M params; 1.54G FLOPs; 2.77.

- China Telecom (R1-2208772) Table 3. Evaluation results for AI/ML model deployed on network-side, without model generalization Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; Test; Model complexity; Computational complexity; AI/ML. TOA; Predicted UE position; True UE position; [0.6,6m,2m]; 78400 samples; 1600 samples; 75.7k; 75.1k; 0.69. DL-TDOA; Predicted UE position; True UE position; [0.6,6m,2m]; 78400 samples; 1600 samples; 75.7k; 75.1k; 0.73. RSRP; Predicted UE position; True UE position; [0.6,6m,2m]; 78400 samples; 1600 samples; 75.7k; 75.1k; 0.52. RSRP+TOA; Predicted UE position; True UE position; [0.6,6m,2m]; 78400 samples; 1600 samples; 184.2k; 182.8k; 0.43. RSRP+DL-TDOA; Predicted UE position; True UE position; [0.6,6m,2m]; 78400 samples; 1600 samples; 184.2k; 182.8k; 0.38.

- CATT (R1-2208971) Table 1: Evaluation results for AI/ML model deployed on UE/LMF-side, without model generalization, resnet18 Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Type: CIR; Size: [18, 256, 2]; Type: UE’s position; Size: [1, 2]; UE’s position with 100% ground truth label; {60%, 6m, 2m}; Training: 19400; Validation: 1080;; 1080; 11.2M; 4.03G FLOPs; 0.98m. Table 2: Evaluation results for AI/ML model deployed on UE/LMF-side, without model generalization, resnet18 Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Type: CIR; Size: [18, 256, 2]; Type: UE’s position; Size: [1, 2]; UE’s position with 100% ground truth label; {60%, 6m, 2m}; Training: 19400; Validation: 1080;; 1080; 11.2M; 4.03G FLOPs; 0.84m.

- CAICT (R1-2209234) Table 2. Evaluation results for AI/ML model deployed on UE side, without model generalization, CNN Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; UE coordinate; 100% data with ground truth label; {60%, 6m, 2m}; 15000; 5000; 2.4M parameters; 4.8M FLOPs; <0.65m.

- xiaomi (R1-2209281) Table 1 Evaluation results for direct AI-based positioning with model deployed on UE or NW side, without model generalization, ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.6，6，2}; {0.6，6，2}; 70000; 10000; 21,277,442; 5.76Gflops; 0.4462. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.4，2，2}; {0.4，2，2}; 70000; 10000; 21,277,442; 5.76Gflops; 0.7566.

- CMCC (R1-2209332) Table II. Simulation results Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; UE location; UE coordinates; {60%, 6m, 2m}; 78400; 1600; 3.71 M; 7.41 M; 0.7 m. CIR+ RSRP; UE location; UE coordinates; {60%, 6m, 2m}; 78400; 1600; 3.71 M; 7.42 M; 0.35 m. TOA; UE location; UE coordinates; {60%, 6m, 2m}; 78400; 1600; 1.85 M; 3.7 M; 0.5 m. TOA+ RSRP; UE location; UE coordinates; {60%, 6m, 2m}; 78400; 1600; 1.85 M; 3.7 M; 0.34 m. TDOA; UE location; UE coordinates; {60%, 6m, 2m}; 78400; 1600; 1.85 M; 3.7 M; 0.41 m. TDOA+RSRP; UE location; UE coordinates; {60%, 6m, 2m}; 78400; 1600; 1.85 M; 3.7 M; 0.16 m.

- InterDigital (R1-2209484) Table 3 Summary of evaluation results Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. RSRP; UE position; percentage of training data set without label = 0%(default); {60%, 6m, 2m} for training and testing; 16000 samples; 4000 samples; 108 RSRP values per sample; 18.34 M FLOPs; 3.2796. RSRP; UE position; percentage of training data set without label = 0%(default); {60%, 6m, 2m} for training and {40%, 2m, 2m}for testing; 16000 samples; 4000 samples; 108 RSRP values per sample; 18.34 M FLOPs; 3.7447.

- Apple (R1-2209580) Table 4: Evaluation results for AI/ML model deployed on UE/network-side, without model generalization, with a CNN Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity (x106); Computational complexity (x106); AI/ML. CIR [18 x 256 x 2]; UE coordinates [1x2]; 0%; {60%, 6, 2}; 47500; 2500; 5.12; 2.43; 1.1m.

- Rakuten (R1-2209615) Table 1 Positioning accuracy of 90% UE Dataset; Positioning accuracy of 90% UE. Drop1; 0.67m. Drop80k; 5.55m.

- Samsung (R1-2209726) Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR with 18x256x2; 2D-coordinates; CIR (default); {60%，6m，2m}; 80000; 10000; 76K; 9.5M; 0.67.

## Evaluation of generalization aspects

### Generalization aspect: different drops

| - Ericsson (R1-2208399) Table 37 Evaluation results for AI/ML model deployed on network-side, with model generalization investigation where the model is trained in InF-DH {60%,6m, 2m}, and tested with the same drop or new drop compared to training. No network synchronization errors. The ML model contains 11 dense layers. Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training/ validation; test; Model complexity; Computation complexity. 18 RSRP values for a target UE; Horizontal position of the target UE; Ideal; {60%,6m, 2m}, same drop; 10,000 of 18 RSRP values; 4,000 of 18 RSRP values; around 3 million; Not available; 1.719. {60%,6m, 2m}, new drop; 19.86. Table 38 Evaluation results for AI/ML model deployed on network-side, with model generalization investigation where the model is trained in InF-DH {60%,6m,2m} without network synchronization error, and tested with new drop or network synchronization error. The ML model contains 11 dense layers. Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training/ validation; test; Model complexity; Computation complexity. 18 ToA values for a target UE; Horizontal position of the target UE; Ideal; {60%,6m, 2m}, same drop, T1=0 ns; 6,000 of 18 ToA values; 6,000 of 18 ToA values; around 3 million; Not available; 1.19. {60%,6m, 2m}, new drop, T1=0 ns; 19.38. {60%,6m, 2m}, same drop, T1=50ns; 12.4. |
| --- |
| - Huawei, HiSilicon (R1-2208433) Table 4. Evaluation results for AI/ML model generalization capability in the aspect: Different Drops Evaluated Scenarios; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). 1; Training; Drop 1; 25000; >10. Test; Drop 2; 5000. 2; Training; 5 Drops mixed; 25000 (5000/drop); 8.04. Test; Drop 2(outside of the trained Drops); 5000. 3; Training; 5 Drops mixed; 25000 (5000/drop); 1.28. Test; Drop 1 (inside the trained Drops); 5000. 4; Training; Drop 1 & 2 mixed; 25000 (12500/drop); 0.69. Test; Drop 1 (inside the trained Drops); 5000. |
| - vivo (R1-2208638) Table 7 Evaluation results of different drops for AI/ML model deployed on UE or Network side, with model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Train; Test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6,6,2}; 25k & Drop1; 1k & Drop1; 1.65M; 22.30M; 0.99. CIR; Pos.; 0; {0.6,6,2}; 25k & Drop1; 1k & Drop2; 1.65M; 22.30M; 6.00. CIR; Pos.; 0; {0.6,6,2}; 25k & Drop1; 1k & Drop3; 1.65M; 22.30M; 5.81. |
| - Qualcomm (R1-2209980) Figure 7 CDF of horizontal positioning errors of direct AI/ML positioning (solid plots: Baseline performance; dashed plots: Type 2 generalizations). |  |
| - Apple (R1-2209580) Table 5: Evaluation results for AI/ML model deployed on UE/network-side, with model generalization for different drops with a CNN Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity (x106); Computational complexity (x106); AI/ML. CIR [18 x 256 x 2]; UE coordinates [1x2]; 0%; {60%, 6, 2}; 47500; 2500; 5.12; 2.43; 3.1m. |
| - OPPO (R1-2208854) Table 4: Generalization performance: training and testing data sets are generated from different drops Accuracy achieved @90% (m); DL-TDOA; Direct: DL-TDOA+RSRP; Direct: Normalized CIR + RSRP; Indirect: Normalized CIR for all TRPs. w/o generalization; 1 drop for both training and testing; 8.20; 0.48; 0.33; 0.52. w/ generalization; 1 drop for training Another drop for testing; 9.92; 10.53; 10.11; 11.29. w/o generalization; 10 drops for both training and testing; 10.16; 0.46; 0.52; 1.03. w/ generalization; - drops for training Another 5 drops for testing; 10.2; 9.3; 6.55; 7.4. |

### Generalization aspect: different clutter parameters

- Ericsson (R1-2208399) Table 40 Evaluation results for AI/ML model deployed on network-side, with model generalization investigation where the model is trained on InF-DH {60%, 6m, 2m}, and tested with various InF-DH clutter parameters. All datasets use the same spatial correlation seed. No network synchronization error. The model is a CNN with 7 conv2D layers and 10 dense layers. Model; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training/ validation; Test; Model complexity; Computation complexity. CNN-1; Time domain CIR, 18x256x2 complex array; Horizontal position of the target UE; Ideal; {60%, 6m, 2m}; 80,000 / 10,000; 500; 6,463,776 complex parameters; 6,252,449,792 FLOPs; 0.9. {60%, 2m, 2m}; 4.5. {40%, 6m, 2m}; 1.0. {40%, 2m, 2m}; 7.6. Table 41 Evaluation results for AI/ML model deployed on network-side, with model generalization investigation where the model is trained on InF-DH {60%, 6m, 2m}, and tested with various InF-DH clutter parameters and different spatial correlation seeds. No network synchronization error. The model is a CNN with 7 conv2D layers and 10 dense layers. Model; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training/ validation; Test; Model complexity; Computation complexity. CNN-1; Time domain CIR, 18x256x2 complex array; Horizontal position of the target UE; Ideal; {60%, 6m, 2m} same drop; 80,000 / 10,000; 500; 6,463,776 complex parameters; 6,252,449,792 FLOPs; 0.9. {60%, 6m, 2m} new drop; >10. {60%, 2m, 2m} new drop; >10. {40%, 6m, 2m} new drop; >10. {40%, 2m, 2m} new drop; >10.

- Huawei, HiSilicon (R1-2208433) Table 5. Evaluation results for AI/ML model generalization capability in the aspect: Clutter parameters Evaluated Scenarios; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). 1; Training; Clutter paras: {40%, 2m, 2m}; 25000; >10. Test; Clutter paras: {60%, 6m, 2m}; 5000. 2; Training; Clutter paras : {60%, 6m, 2m} & {40%, 2m, 2m} mixed; 25000 (12500/ paras); 0.86. Test; Clutter paras: {60%, 6m, 2m}; 5000. 3; Training; Clutter paras : {60%, 6m, 2m} & {40%, 2m, 2m} mixed; 25000 (12500/ paras); 0.88. Test; Clutter paras: {40%, 2m, 2m}; 5000.

- vivo (R1-2208638) Table 8 Evaluation results of different cluster parameters for AI/ML model deployed on UE or Network side, with model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Train; Test; Train; Test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6, 6, 2}; {0.6, 6, 2}; 25k; 1k; 1.65M; 22.30M; 0.99. CIR; Pos.; 0; {0.6, 6, 2}; {0.4, 2, 2}; 25k; 1k; 1.65M; 22.30M; 8.67. CIR; Pos.; 0; {0.4, 2, 2}; {0.4, 2, 2}; 25k; 1k; 1.65M; 22.30M; 1.06. CIR; Pos.; 0; {0.4, 2, 2}; {0.6, 6, 2}; 25k; 1k; 1.65M; 22.30M; 4.77. CIR; Pos.; 0; {0.6, 6, 2} & {0.4, 2, 2}; {0.6, 6, 2}; 25k & 25k; 1k; 1.65M; 22.30M; 0.87. CIR; Pos.; 0; {0.6, 6, 2} & {0.4, 2, 2}; {0.4, 2, 2}; 25k & 25k; 1k; 1.65M; 22.30M; 0.94.

- Nokia (R1-2209371) Table 3. Evaluation results for AI/ML model deployed on UE-side, with model generalization, considering different clutter parameters. Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR from 18 BSs; 2D position (X, Y); True 2D position (X, Y); 40%; 10K+ (90%) including evaluation; 10K (10%); 1.9M; 19.2M flops; Typical generalization: 6.62517 (blue line Fig. 6) Custom generalization: 11.56156 (red line Fig. 6). CIR from 18 BSs; 2D position (X, Y); True 2D position (X, Y); 60%; 10K+ (90%) including evaluation; 10K (10%); 1.9M; 19.2M flops; Typical generalization: 8.70181 (blue line Fig. 7) Custom generalization: 14.36727 (red line Fig. 7).

- Apple (R1-2209580) Table 6: Evaluation results for AI/ML model deployed on UE/network-side, with model generalization for Clutter (train {60%,2m,2m}, test {40%, 2m, 2m}) with a CNN Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity (x106); Computational complexity (x106); AI/ML. CIR [18 x 256 x 2]; UE coordinates [1x2]; 0%; {60%, 6, 2}; 47500; 2500; 5.12; 2.43; 3.6m.

- xiaomi (R1-2209281) Table 3 Evaluation results for direct AI-based positioning with model deployed on UE or NW side, with model generalization (different clutter parameter), ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.6,6,2}; {0.4,2,2}; 70000; 10000; 21,277,442; 5.76Gflops; 7.0914. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.4,2,2}; {0.6,6,2}; 70000; 10000; 21,277,442; 5.76Gflops; 1.5328. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; Mix of {0.6,6,2}, {0.4,2,2}; {0.6,6,2}; 35000+35000; 10000; 21,277,442; 5.76Gflops; 0.5419. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; Mix of {0.6,6,2}, {0.4,2,2}; {0.4,2,2}; 35000+35000; 10000; 21,277,442; 5.76Gflops; 0.7684.

- CATT (R1-2208971) Table 3: Evaluation results for AI/ML model deployed on UE/LMF-side, with model generalization, resnet18 Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Type: CIR; Size: [18, 256, 2]; Type: UE’s position; Size: [1, 2]; UE’s position with 100% ground truth label; Training:{60%, 6m, 2m} Testing:{40%, 2m, 2m}; Training: 19400; Validation: 1080;; 1080; 11.2M; 4.03G FLOPs; 2.64m.

- OPPO (R1-2208854) Table 5: Generalization performance: training and testing data sets are with different clutter settings Accuracy achieved @90% (m); DL-TDOA; Direct: DL-TDOA+RSRP; Direct: Normalized CIR + RSRP; Indirect: Normalized CIR for all TRPs. Data setting C-1; 3.86; 8.05; 15.75; 16.09. Data setting C-2; 10.45; 10.8; 8.61; 8.88. Data setting C-3; 3.74; 7.78; 8.67; 8.49. Data setting C-4; 3.84; 4.86; 6.82; 6.66.

- InterDigtial (R1-2209484) Table 7 Summary of evaluation results Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. RSRP; UE position; percentage of training data set without label = 0%(default); {60%, 6m, 2m} for training and testing; 16000 samples; 4000 samples; 108 RSRP values per sample; 18.34 M FLOPs; 3.2796. RSRP; UE position; percentage of training data set without label = 0%(default); {60%, 6m, 2m} for training and {40%, 2m, 2m}for testing; 16000 samples; 4000 samples; 108 RSRP values per sample; 18.34 M FLOPs; 3.7447.

### Generalization aspect: network synchronization error

- Huawei, HiSilicon (R1-2208433) Table 6. Evaluation results for AI/ML model generalization capability in the aspect: Network synchronization error Evaluated Scenarios; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). 1; Training; Without network synchronization error; 25000; >10. Test; With network synchronization error @50ns; 5000. 2; Training; With network synchronization error @50ns; 25000; 3.02. Test; 5000. 3; Training; With network synchronization error @0&30&40&50ns; 25000 (6250/ paras); 2.51. Test; 5000 (1250/ paras). 4; Training; With network synchronization error @0&30&40&50ns; 25000 (6250/ paras); 4.28. Test; With network synchronization error @50ns; 5000 (1250/ paras).

- vivo (R1-2208638) Table 12 Evaluation results for AI/ML model deployed on UE or Network side, with model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6, 6, 2}; 25k; 1k & 0ns sync. Error; 1.65M; 22.30M; 0.99. CIR; Pos.; 0; {0.6, 6, 2}; 25k; 1k & 2ns sync. Error; 1.65M; 22.30M; 1.64. CIR; Pos.; 0; {0.6, 6, 2}; 25k; 1k & 10ns sync. Error; 1.65M; 22.30M; 4.56. CIR; Pos.; 0; {0.6, 6, 2}; 25k; 1k & 50ns sync. Error; 1.65M; 22.30M; 10.18. Table 13 Evaluation results for AI/ML model deployed on UE or Network side, with model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6, 6, 2}; 25k; 1k & 10ns sync. Error; 1.65M; 22.30M; 4.56. CIR; Pos.; 0; {0.6, 6, 2}; 25k + 2k 10ns sync. Error; 1k & 10ns sync. Error; 1.65M; 22.30M; 1.16. CIR; Pos.; 0; {0.6, 6, 2}; 25k; 1k & 50ns sync. Error; 1.65M; 22.30M; 10.18. CIR; Pos.; 0; {0.6, 6, 2}; 25k + 2k 50ns sync. Error; 1k & 50ns sync. Error; 1.65M; 22.30M; 1.52.

- Qualcomm (R1-2209980) Figure 5 CDF of horizontal positioning error for RFFP scheme under different TRPs’ synchronization assumptions (blue plot: TRPs are synchronized; magenta plot: TRPs have random synchronization error within [-10, 10] nanoseconds).

- xiaomi (R1-2209281) Table 5 Evaluation results for direct AI-based positioning with model deployed on UE or NW side, with model generalization (AI/ML model is trained by data set with ideal network synchronization and tested by data set with 100ns network synchronization error), ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.6，6，2}; {0.6，6，2}; 70000; 10000; 21,277,442; 5.76Gflops; 12.4486. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.4，2，2}; {0.4，2，2}; 70000; 10000; 21,277,442; 5.76Gflops; 14.5779. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; Mix of {0.6,6,2} {0.4,2,2}; {0.6,6,2}; 35000+35000; 10000; 21,277,442; 5.76Gflops; 13.5239. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; Mix of {0.6,6,2} {0.4,2,2}; {0.4,2,2}; 35000+35000; 10000; 21,277,442; 5.76Gflops; 13.2829. Table 6 Evaluation results for direct AI-based positioning with model deployed on UE or NW side, with model generalization (AI/ML model is trained by data set with 100ns network synchronization error and tested by data set with 100ns network synchronization error), ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.6,6,2}; {0.6,6,2}; 70000; 10000; 21,277,442; 5.76Gflops; 1.0666. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.4,2,2}; {0.4,2,2}; 70000; 10000; 21,277,442; 5.76Gflops; 1.7981. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; Mix of {0.6,6,2} {0.4,2,2}; {0.6,6,2}; 35000+35000; 10000; 21,277,442; 5.76Gflops; 1.3431. 18*256*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; Mix of {0.6,6,2} {0.4,2,2}; {0.4,2,2}; 35000+35000; 10000; 21,277,442; 5.76Gflops; 1.9475.

- CATT (R1-2208971) Table 4: Evaluation results for AI/ML model deployed on UE/LMF-side, with model generalization [Training dataset with perfect network synchronization and testing dataset with network synchronization error], resnet18 Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Type: CIR; Size: [18, 256, 2]; Type: UE’s position; Size: [1, 2]; UE’s position with 100% ground truth label; {60%, 6m, 2m}; Training: 19400; Validation: 1080;; 1080; 11.2M; 4.03G FLOPs; 12.6m.

- OPPO (R1-2208854) Table 7: Generalization performance: Training w/o NW syn error, Test w/ NW syn error Accuracy achieved @90% (m); DL-TDOA; Direct: DL-TDOA+RSRP; Direct: Normalized CIR + RSRP; Indirect: Normalized CIR for all TRPs. w/ generalization; Data setting S-1 - drop w/o NW sync error for training, 1 drop w/ NW sync error for testing; 29.05; 5.29; 6.54; 27.85. w/o generalization; 1 drop w/o NW sync error for training and testing; 8.2; 0.48; 0.33; 0.52. w/ generalization; Data setting S-2 10 drops w/o NW sync error for training, 10 drops w/ NW sync error for testing; 36.91; 7.11; 11.77; 33.62. w/o generalization; 10 drops w/o NW sync error for training and testing; 10.16; 0.46; 0.52; 1.03.

### Generalization aspect: UE/gNB RX and TX timing error

- Huawei, HiSilicon (R1-2208433) Table 7. Evaluation results for AI/ML model generalization capability in the aspect: UE timing error Evaluated Scenarios; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). 1; Training; Without UE timing error; 25000; 3.12. Test; With UE timing error @10ns; 5000. 2; Training; With UE timing error @10ns; 25000; 0.61. Test; 5000. 3; Training; With UE timing error @0&10&20&30ns; 25000 (6250/ paras); 0.68. Test; 5000 (1250/ paras). 4; Training; With UE timing error @0&10&20&30ns; 25000 (6250/ paras); 0.89. Test; With UE timing error @30ns; 5000 (1250/ paras).

- Qualcomm (R1-2209980) Figure 4 CDF of horizontal positioning error for RFFP scheme under different UE clock drift conditions (green plot: RFFP performance in ideal settings when no clock drift present; blue plot: training accounts for UE clock drift and testing includes UE clock drift within [-150,150] nanoseconds).

- OPPO (R1-2208854) Table 8: Generalization performance: Training w/o UE timing error, Test w/ UE timing error Accuracy achieved @90% (m); DL-TDOA; Direct: DL-TDOA+RSRP; Direct: Normalized CIR + RSRP; Indirect: Normalized CIR for all TRPs. w/ generalization; Data setting T-1 - drop w/o UE timing error for training, 1 drop w/ UE timing error for testing; 8.38; 0.48; 6.18; 26.79. w/o generalization; 1 drop w/o UE timing error for training and testing; 8.2; 0.48; 0.33; 0.52. w/ generalization; Data setting T-2 10 drops w/o UE timing error for training, 10 drops w/ UE timing error for testing; 10.28; 0.456; 12.393; 39.73. w/o generalization; 10 drops w/o UE timing for training and testing; 10.16; 0.46; 0.52; 1.03.

## Evaluation of model fine-tuning / re-training

- Ericsson (R1-2208399) Table 42 Evaluation results for AI/ML model deployed on network-side, with model fine-tuning. The CNN-1 model trained on InF-DH {60%, 6m, 2m} is updated to CNN-2 by further training with 1000 samples with {40%, 2m, 2m}. No network synchronization error. The model is a CNN with 7 conv2D layers and 10 dense layers. Model; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training/ validation; Test; Model complexity; Computation complexity. CNN-2; Time domain CIR, 18x256x2 complex array; Horizontal position of the target UE; Ideal; {60%, 6m, 2m}; 1000 for model update; 500; 6,463,776 complex parameters; 6,252,449,792 FLOPs; 3.5. {40%, 2m, 2m}; 4.3. Table 43 Evaluation results for AI/ML model deployed on network-side, with model retraining. The CNN-1 model trained in InF-DH {60%, 6m, 2m} is updated to CNN-3 by further training with a mixed {60%, 6m, 2m} + {40%, 2m, 2m} dataset with approximately 100,000 samples. No network synchronization error. The model is a CNN with 7 conv2D layers and 10 dense layers. Model; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training/ validation; Test; Model complexity; Computation complexity. CNN-3; Time domain CIR, 18x256x2 complex array; Horizontal position of the target UE; Ideal; {60%, 6m, 2m}; 100,000 for model update; 500; 6,463,776 complex parameters; 6,252,449,792 FLOPs; 1.0. {60%, 2m, 2m}; 1.6. {40%, 2m, 2m}; 2.0.

- Huawei, HiSilicon (R1-2208433) Table 8. Model update evaluation results for AI/ML model generalization capability Evaluated aspects; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). Before model update; After model update. Different Drops; Training; Drop 1; 25000; >10; 3.2. Fine-tuning; Drop 2; 1000. Test; Drop 2; 5000. Clutter parameters; Training; Clutter paras: InF-DH {60%, 6m, 2m}; 25000; >10; 3.1. Fine-tuning; Clutter paras: InF-DH {40%, 2m, 2m}; 1000. Test; Clutter paras: InF-DH {40%, 2m, 2m}; 5000. Network synchronization error; Training; Without network synchronization error; 25000; >10; 8.47. Fine-tuning; With network synchronization error @50ns; 5000. Test; With network synchronization error @50ns; 5000. UE timing error; Training; Without UE timing error; 25000; 7.24; 1.13. Fine-tuning; With UE timing error@20ns; 5000. Test; With UE timing error@20ns; 5000.

- Qualcomm (R1-2209980) Table 8 Evaluation results for AI/ML model deployed on UE-side, with model generalization (Type 2 – different drops), CNN Method; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training – Drop A; Test – Drop B; Model complexity; Computational complexity; AI/ML. RFFP; CIR (2, 18,400); 2D location; 0%; {60%, 6m, 2m}; 15000; 2000; 1.5M params; 1.54G FLOPs; 12.33. RFFP; CIR (2, 18,400); 2D location; 0%; {60%, 6m, 2m}; 15000 + 100 finetuning (Drop B); 2000; 1.5M params; 1.54G FLOPs; 10.47. RFFP; CIR (2, 18,400); 2D location; 0%; {60%, 6m, 2m}; 15000+ 240 finetuning (Drop B); 2000; 1.5M params; 1.54G FLOPs; 6.92. RFFP; CIR (2, 18,400); 2D location; 0%; {60%, 6m, 2m}; 15000+ 500 finetuning (Drop B); 2000; 1.5M params; 1.54G FLOPs; 6.07.

- NVIDIA (R1-2209629) Table 4: Summary of CDF percentiles of horizontal positioning accuracy with model finetuning. Training; Testing; Finetuning; 50%; 67%; 80%; 90%. Drop 1; Drop 1; N/A; 1.1 m; 1.5 m; 1.8 m; 2.3 m. Drop 1; Drop 2; No finetuning; 7.1 m; 9.3 m; 11.6 m; 14.5 m. Drop 1; Drop 2; Finetuning with 1k samples; 2.5 m; 3.3 m; 4.2 m; 5.3 m. Drop 1; Drop 2; Finetuning with 2k samples; 2.1 m; 2.7 m; 3.5 m; 4.3 m. Note: The original model was trained with 16k samples. Thus, 1k (resp. 2k) finetuning samples corresponds to 6.25% (resp. 12.5%) of the total 16k samples..

- CATT (R1-2208971) Table 5: Evaluation results for AI/ML model deployed on UE/LMF-side, with model generalization and fine-tuning, resnet18 Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Type: CIR; Size: [18, 256, 2]; Type: UE’s position; Size: [1, 2]; UE’s position with 100% ground truth label; Training:{60%, 6m, 2m} Fine-tuning and testing:{40%, 2m, 2m}; Training: 19400; Fine-tuning: 1080;; 1080; 11.2M; 4.03G FLOPs; 1.23m. Table 6: Evaluation results for AI/ML model deployed on UE/LMF-side, with model generalization and fine-tuning [Training dataset with perfect network synchronization and fine-tuning dataset with network synchronization error], resnet18 Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Type: CIR; Size: [18, 256, 2]; Type: UE’s position; Size: [1, 2]; UE’s position with 100% ground truth label; {60%, 6m, 2m}; Training: 19400; Fine-tuning: 1080;; 1080; 11.2M; 4.03G FLOPs; 2.23m.

## Evaluation of issues related to ground truth labels

- vivo (R1-2208638) Labeling error Table 14 Evaluation results for AI/ML model deployed on UE or Network side, with model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6, 6, 2}; 25k & std = 0; 1k; 1.65M; 22.30M; 0.99. CIR; Pos.; 0; {0.6, 6, 2}; 25k & std = 0.5; 1k; 1.65M; 22.30M; 1.51. CIR; Pos.; 0; {0.6, 6, 2}; 25k & std = 1; 1k; 1.65M; 22.30M; 2.17. CIR; Pos.; 0; {0.6, 6, 2}; 25k & std = 2; 1k; 1.65M; 22.30M; 3.55. Table 17 Evaluation results of semi-supervised learning for AI/ML model deployed on UE or Network side, without model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Train; test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 96%; {0.6, 6, 2}; 1k labeled & 25k unlabeled; 1k; 1.65M; 22.30M; 5.05. CIR; Pos.; 99%; {0.6, 6, 2}; 0.3k labeled & 25k unlabeled; 1k; 1.65M; 22.30M; 8.78. CIR; Pos.; 0; {0.6, 6, 2}; 1k; 1k; 1.65M; 22.30M; 12.06. CIR; Pos.; 0; {0.6, 6, 2}; 2k; 1k; 1.65M; 22.30M; 9.03. CIR; Pos.; 0; {0.6, 6, 2}; 2k; 1k; 1.65M; 22.30M; 5.53.

- Nokia (R1-2209371) Table 4. Evaluation results for AI/ML model deployed on UE-side, without model generalization, taking into consideration the impact of diverse data for model training and testing. Case; Model input; Model output; Label; Clutter param; Dataset size; Model Complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; Test; Model complexity; Computational complexity; AI/ML. 1; TOA from 18 TRPs; 2D position (x,y); True 2D position (x,y); 40%; ~600 (5%); ~ 11K (95%); 261K; 521K; Training accuracy = 1,69 Test accuracy = 136,7. 2; TOA from 18 TRPs; 2D position (x,y); True 2D position (x,y); 40%; ~ 6K (50%); ~ 6K (50%); 261K; 521K; Training accuracy = 1,19 Test accuracy = 2,59. 3; TOA from 18 TRPs; 2D position (x,y); True 2D position (x,y); 40%; ~ 6K (5% + 47% with Error); ~ 6K (47%); 261K; 521K; Training accuracy = 1,74 Test accuracy = 4,83. Observation-11: ML model trained using a sparse dataset (case 1) performs poorly in terms of horizontal positioning accuracy, especially in comparison to the scenario with much larger training and test dataset (case 2). Observation-12: Augmenting the sparse dataset with noisy labels could provide significant improvements in terms of the positioning accuracy of the AI/ML model. Proposal-12: RAN1 to further study ML model performance with sparse datasets, including possible solutions such as data augmentation.

## Other evaluation results

### Different deployment scenarios (e.g., InF-DH vs InF-HH)

- vivo (R1-2208638) Table 9 Evaluation results of different scenarios for AI/ML model deployed on UE or Network side, with model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6, 6, 2}; 25k & InF-DH; 1k & InF-DH; 1.65M; 22.30M; 0.99. CIR; Pos.; 0; {0.6, 6, 2}; 25k & InF-HH; 1k & InF-HH; 1.65M; 22.30M; 0.63. CIR; Pos.; 0; {0.6, 6, 2}; 25k & InF-SH; 1k & InF-SH; 1.65M; 22.30M; 0.87. CIR; Pos.; 0; {0.6, 6, 2}; 25k & InF-DH; 1k & InF-HH; 1.65M; 22.30M; >10. CIR; Pos.; 0; {0.6, 6, 2}; 25k & InF-DH; 1k & InF-SH; 1.65M; 22.30M; >10.

### Zone-specific changes

- Qualcomm (R1-2209980) Observation8: RFFP method can show improved robustness to slight environment changes such as time-varying blocking when trained on mixture of such changes. Table 79 Horizontal positioning error (meter) for RFFP with Type 3 generalizations (ML model trained on mixture of channel realizations) Train; Test; 50%tile; 67%tile; 80%tile; 90%tile. Mixed clusters; Odd clusters; 1.33; 1.59; 2.19; 2.80. Mixed clusters; Odd except clusters 1&3; 2.41; 3.07; 3.81; 4.87. Mixed clusters; Odd except clusters 1&5; 2.21; 2.97; 3.72; 4.64. Mixed clusters; Odd except clusters 5&7; 1.45; 1.84; 2.42; 3.15. Mixed clusters; Odd with clusters 2&4; 1.39; 1.63; 2.25; 2.89. Mixed clusters; Odd with clusters 6&8; 1.34; 1.60; 2.23; 2.87. Mixed clusters; Remove up to two random odd clusters and add up to two random even ones; 1.50; 1.822; 2.46; 3.17.

### Channel estimation

- vivo (R1-2208638) Table 10 Evaluation results for AI/ML model deployed on UE or Network side, without model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = -30dB; 1k & SINR = -30dB; 1.65M; 22.30M; 6.89. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = -25dB; 1k & SINR = -25dB; 1.65M; 22.30M; 4.31. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = -20dB; 1k & SINR = -20dB; 1.65M; 22.30M; 1.84. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = -10dB; 1k & SINR = -10dB; 1.65M; 22.30M; 1.83. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = 10dB; 1k & SINR = 10dB; 1.65M; 22.30M; 1.46. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = 30dB; 1k & SINR = 30dB; 1.65M; 22.30M; 1.34. Table 11 Evaluation results for AI/ML model deployed on UE or Network side, without model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = 30dB; 1k & SINR = 30dB; 1.65M; 22.30M; 1.34. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = 30dB; 1k & SINR = 10dB; 1.65M; 22.30M; 1.35. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = 30dB; 1k & SINR = -10dB; 1.65M; 22.30M; 2.65. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = 30dB; 1k & SINR = -20dB; 1.65M; 22.30M; 17.89. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = 30dB; 1k & SINR = -25dB; 1.65M; 22.30M; >10. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = 30dB; 1k & SINR = -30dB; 1.65M; 22.30M; >10.

### Positioning with multiple ports data

- vivo (R1-2208638) Table 18 Evaluation results of multiple ports for AI/ML model deployed on UE or Network side, without model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6, 6, 2}; 3k & 8 ports; 1k & 8 ports; 1.65M; 22.30M; 3.14. CIR; Pos.; 0; {0.6, 6, 2}; 3k & 1 port; 1k & 1 ports; 1.65M; 22.30M; 5.53.

### Reduced input dimension

- xiaomi (R1-2209281) Proposal 1: On the basis of satisfying the positioning accuracy requirement, study solution to reduce the model size, computation complexity and involved ynchroni overhead Table 7 Evaluation results for reduced input dimension for direct AI-based positioning and AI-based ToA predication, model deployed on UE or NW side, without model generalization, ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*24*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.6,6,2}; {0.6,6,2}; 70000; 10000; 21,277,442; 539.94Mflops; 0.8219. 18*24*2 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.6,6,2}; {0.6,6,2}; 70000; 10000; 21,285,650; 539.95Mflops; 0.8993.

## 1st round discussion

Based on the evaluation results submitted by companies, several observations were made by majority companies. No contradictory simulation results were submitted thus far. Accordingly, the following proposals are made, which are the result of merging several companies’ proposals.

Proposal 5.6-1 (overall)

Observation: Direct AI/ML positioning can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods in heavy NLOS scenarios.

|  | Company |
| --- | --- |
| Support | Nokia/NSB, NVIDIA, Apple, InterDigital, Fujitsu, LG, Qualcomm,CMCC, Samsung, CATT, CAICT, Lenovo, Ericsson |
| Not support |  |

| Company | Comments |
| --- | --- |
| Nokia/NSB | We are fine with these observations, however we are not sure about the benefits of agreeing such proposals. It would be beneficial to discuss which observations would progress the study. |
| LG | The purpose of the following proposals 5.6-1,2,3,4 is unclear since it is an overall observation on direct AI/ML positioning |
| OPPO | We agree the observation is valid for many cases according to the submitted contributions. However, it is premature to have some conclusion without any prerequisite (expecting the NLOS scenarios). As companies will submit more evaluation results in the future meetings, we prefer to defer this proposal to see more results of generalization performance. |
| Qualcomm | We are fine with the observation. Similar accuracy improvement observation can be made for mild NLOS and LOS scenarios as well. |
| Samsung | We just want to point out that, we have also shown the evaluation with the generalization aspect (different scenario, different clutter, different hall size), we hope FL can have our simulation results being captured here incase it will be used as TR input. [Moderator] Thanks for checking! Samsung results is captured in Section 5.1, see the last row in the big table. Besides, we want to say that different hall size will be seriously impact the results. Thus, somehow this aspect should be taken into account. |
| ZTE | If this is for TR conclusion, we think is too early to agree this. We should firstly agree how to align the evaluation assumptions. Then, we should consider how to capture the observations submitted from companies. TR conclusion is a final step. |
| HW/HiSi | We think this is a valuable observation and should be captured. It is summarizing part of the progress we have made so far. |
| CATT | Share the same view with HW. |
| CAICT | Agree HW’s point. |
| Lenovo | Support |

Proposal 5.6-2 (generalization over drops)

Observation: When other simulation assumptions are held constant, positioning accuracy of direct AI/ML positioning deteriorates significantly when the ML model is trained with dataset of one drop, and tested with dataset of a different drop (i.e., different set of random seeds for channel propagation).

|  | Company |
| --- | --- |
| Support | Apple, Fujitsu, OPPO, Qualcomm, Ericsson |
| Not support | [HW/HiS – needs also to be captured how performance can improved, there are many simulation results for that] |

| Company | Comments |
| --- | --- |
| Qualcomm | We are fine with the observation. |
| ZTE | OK with the observation. As commented in Proposal 5.6-1, the intention should be clarified. |
| HW/HiSi | This observation is in principle true, but it should be pointed out that enriching the training data set can improve the accuracy to some extent, even is the testing data set is not included in any of the drops used for training. This is e.g. shown in test scenario 2 of Table 4 in our paper (R1-2208433). Also, the performance can be increased with model fine-tuning. We think separate observation should also be made about improved performance through enriched training and/or fine-tuning. Currently these observations are missing, therefore, for the time being, we suggest to to add the following to the proposal: Updated Observation: When other simulation assumptions are held constant, positioning accuracy of direct AI/ML positioning deteriorates significantly when the ML model is trained with dataset of one drop, and tested with dataset of a different drop (i.e., different set of random seeds for channel propagation). - The performance is improved when - the training dataset is composed of multiple drops the accuracy can be improved, even if the drop used for testing is outside the drops used for training. - Model fine-tuning is performed, where the model is re-trained with a small dataset from the same drop as the tested datatset |
| CAICT | We prefer HW’s proposal. |
| Ericsson | Support. Also OK with HW’s update. |

Proposal 5.6-3 (generalization over clutter parameters)

Observation: Positioning accuracy of direct AI/ML positioning deteriorates when the ML model is trained with dataset of one clutter parameter setting (e.g., InF-DH {60%, 6m, 2m}), and tested with dataset of a different clutter parameter setting (e.g., InF-DH {40%, 2m, 2m}). As the clutter parameter setting of training dataset deviates further from that of training, worse positioning accuracy is observed.

|  | Company |
| --- | --- |
| Support | Apple, OPPO, Qualcomm, Samsung, CATT, CAICT |
| Not support | [HW/HiSi] |

| Company | Comments |
| --- | --- |
| InterDigital | The FL’s observation did not capture our results on generalization over clutter parameters. Please capture following results as a part of section 5.2.2 Generalization aspect: different clutter parameters (Row 2, Rable 3). Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. RSRP; UE position; percentage of training data set without label = 0%(default); {60%, 6m, 2m} for training and {40%, 2m, 2m}for testing; 16000 samples; 4000 samples; 108 RSRP values per sample; 18.34 M FLOPs; 3.7447. [Moderator] Thanks for pointing this out! Table 3 of InterDigital contribution is added to section 5.2.2 now, see last entry in the big table. |
| ZTE | OK with the observation. As commented in Proposal 5.6-1, the intention should be clarified. |
| HW/HiSi | Similar to the prvious observation, as soon as the clutter parameters used for testing are included in the dataset for training, the performance is increased. We think that is a valuable finding that should be captured, it is e.g. shown in Table 5 of our paper R1-2208433, where sub-meter accuracy can be reached as soon as the clutter parameters for the testing dataset are one part of the training data set. In practice that would mean that the model can be pre-trained with multiple clutter parameters and will then achieve a good performance during inference. Another option to improve teh performance is fine-tuning that also has been ynchroni. If we do not make additional observations how performance cen be improved, we suggest to update the observation as following: Updated Observation: Positioning accuracy of direct AI/ML positioning deteriorates when the ML model is trained with dataset of one clutter parameter setting (e.g., InF-DH {60%, 6m, 2m}), and tested with dataset of a different clutter parameter setting (e.g., InF-DH {40%, 2m, 2m}). As the clutter parameter setting of training dataset deviates further from that of training, worse positioning accuracy is observed. - By using multiple clutter parameters to compose the training dataset, the accuracy is significantly improved if the clutter parameters used for the testing dataset are included in the training dataset. - The performance of the model can also be improved with fine-tuning, where the model is re-trained with a small dataset from the same drop as the tested datatset |
| CATT | Share the same view with HW which can clarify the fine-tuning is needed. But we also can live with current observation. |

Proposal 5.6-4 (generalization over network synchronization error)

Observation: Positioning accuracy of direct AI/ML positioning deteriorates when the ML model is trained with dataset of one network synchronization error setting (e.g., T1=0ns), and tested with dataset of a different network synchronization error setting (e.g., T1=50ns). As the network synchronization error setting of training dataset deviates further from that of training, worse positioning accuracy is observed.

|  | Company |
| --- | --- |
| Support | Apple, Fujitsu, OPPO, Ericsson |
| Not support | Qualcomm, HWHiSI, CATT |

| Company | Comments |
| --- | --- |
| Qualcomm | We prefer to defer agreeing on this observation until more evaluations are conducted. We agree that NW synchronization error can cause degradation to direct AI/ML positioning when this error is not considered during training. But we need to conduct more evaluations to understand whether training on a worse error setting, e.g., T1 = 50 nsec, and testing on a tighter synchronization, e.g., T1 = 20 nsec, can yield robust performance. |
| ZTE | Agree with Qualcomm, training dataset should also include synchronization error. |
| HW/HiSI | This observation seems too strong. It can give the impression that the Gnb synch error can even be larger than 50ns. But 50ns is the maximum error that is defined in the simulations assumptions. Also for this error it has been shown that the performance can be improved by enriching the training data set and/or fine-tuning. We think this valuable information should be added to the observation, if not an independent observation is made: We there suggest to update as follows: Updated Observation: Positioning accuracy of direct AI/ML positioning deteriorates when the ML model is trained with dataset of one network synchronization error setting (e.g., T1=0ns), and tested with dataset of a different network synchronization error setting (e.g., T1=50ns). As the network synchronization error setting of training dataset deviates further from that of training, worse positioning accuracy is observed. - The positioning accuracy can be improved by training the AI model with multiple network synchronization error assumptions and/or model fine-tuning, where the model is trained with a small size data set that experiences the same network synch-error as the testing dataset |
| CATT | Based on the simulation results provided by companies, we thin the generalization performance for network synchronization error deteriorates significantly from 0~1 to larger than 10m. But for Proposal 5.6-3 with different clutter parameters, the performance deteriorates from 0~1 to about 3m. Thus, we think we need differentiate the different level of deterioration. We also share the same view with HW, which can clarify the fine-tuning is needed. |
| CAICt | We prefer to have more simulation results and discussions on the effect of ynchronization error before we make a concludion. |

## 2nd round discussion

Based on companies’ input, Proposal 5.6-1 (overall) is generally acceptable. Thus it will be recommended for online discussion.

For Proposal 5.6-2 (generalization over drops), HW/HiSi suggested an updated observation. We can check the if longer version is acceptable. If not, we can revert back to the shorter version to make some progress.

Proposal 5.7-1

Observation: When other simulation assumptions are held constant, positioning accuracy of direct AI/ML positioning deteriorates significantly when the ML model is trained with dataset of one drop, and tested with dataset of a different drop (i.e., different set of random seeds for channel propagation).

- The positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning, for example:

  - The training dataset is composed of data from multiple drops;

  - Model fine-tuning is performed, where the model is re-trained with a small dataset from the same drop as the test dataset.

|  | Company |
| --- | --- |
| Support | ZTE, Fujitsu, HW/HiSi |
| Not support | OPPO, Apple |

| Company | Comments |
| --- | --- |
| ZTE | Generally fine with the proposal. |
| OPPO | In our view, more evaluation/disucssion are needed for the red part of model fine-tuning. We haven’t discussed much details of model fine-tuning. Generally speaking, it is pre-mature to make such kind of observations in hurry. It is just the 3rd meeting. They should be the output of the study item. |
| Apple | Agree with Oppo |
| Fujitsu | Generally fine, the moderator takes it as “observation“ which means it was observed based on the previous evaluations. Companies are free to provide further results. |
| CAICT | We are fine to defer the conclusion till more evaluation results related to fine-tuning are provided. |
| Xiaomi | More evaluation about the fine-tuning is needed before we get the conclusion |
| CMCC | We support the main bullet. The sub-bullet can be discussed in later meetings. |
| Qualcomm | The proposal covers options to enhance positioning performance with different drops. However, only two training aspects are considered to improve positioning accuracy (i.e., mixed training and fine-tuning). Another natural solution can be to consider drop-specific model training. We propose this additional wording: Observation: When other simulation assumptions are held constant, positioning accuracy of direct AI/ML positioning deteriorates significantly when the ML model is trained with dataset of one drop, and tested with dataset of a different drop (i.e., different set of random seeds for channel propagation). - The positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning, for example: - The training dataset is composed of data from multiple drops; - Model fine-tuning is performed, where the model is re-trained with a small dataset from the same drop as the test dataset. - This deterioration in positioning performance can be avoided by considering drop-specific model training that accounts for the testing drop settings. |
| HW/HiSi | We support the proposal. We think that evaluation results from multiple companies are indicating that this observation is valid. We ackowledge that more details should be studied and evaluated to additionally obtain more detailed results later. But the evaluation that companies have done so far are not pre-mature to capture this effort in an observation. We think this observation is a good starting point for further evaluation. We can also compromise to the following wording, if needed Updated Observation: When other simulation assumptions are held constant, positioning accuracy of direct AI/ML positioning deteriorates significantly when the ML model is trained with dataset of one drop, and tested with dataset of a different drop (i.e., different set of random seeds for channel propagation). - Companies have provided initial evaluation results that show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning, for example: - The training dataset is composed of data from multiple drops; - Model fine-tuning is performed, where the model is re-trained with a small dataset from the same drop as the test dataset. We think that the extra bullet added by QC says in principle the same as the last bullet of the updated proposal above. But we can also accept the QC modification. |
| LG | Same understanding with OPPO |
| vivo | We have questions on the wording suggestion from Qualcomm. The propsoed observation is about direct AI/ML positioning generalization performane evaluation with different drop, i.e., when the ML model is trained with dataset of one drop, and tested with dataset of a different drop. The bullet added by Qualcomm is actually says another seperate model trained for that testing drop. How is that categorized as generalization performance evaluation? We’re not sure about relevance here. |

For Proposal 5.6-3 (generalization over clutter parameters), HW/HiSi also suggested an updated observation. We can check the if longer version is acceptable. If not, we can revert to the shorter version to make some progress.

Proposal 5.7-2 (generalization over clutter parameters)

Observation: Positioning accuracy of direct AI/ML positioning deteriorates when the ML model is trained with dataset of one clutter parameter setting (e.g., InF-DH {60%, 6m, 2m}), and tested with dataset of a different clutter parameter setting (e.g., InF-DH {40%, 2m, 2m}). As the clutter parameter setting of training dataset deviates further from that of training, worse positioning accuracy is observed.

- By using multiple clutter parameters to compose the training dataset, the positioning accuracy is significantly improved if the clutter parameters used for the testing dataset are included in the training dataset.

- The performance of the model can also be improved with fine-tuning, where the model is re-trained with a small dataset from the same drop as the tested dataset.

|  | Company |
| --- | --- |
| Support | ZTE, Fujitsu, HW/HiSi |
| Not support | OPPO, Apple |

| Company | Comments |
| --- | --- |
| ZTE | Generally fine with the proposal. |
| OPPO | Similar comment as before |
| Samsung | Generally fine with the proposal. Another followup comment from 1st round in Proposal 5.6-1 (overall), we mentioned our evaluation results with the generalization aspect (different scenario, different clutter, different hall size) are not captured, we have not provide the table for these three results, just figures. The one in last row in 5.1 is just the general case. We could provide the table if it needs to be provided to TR input. |
| Apple | Similar comment as before |
| Fujitsu | Same comments as the previous one. |
| CAICT | We are fine to defer the conclusion till more evaluation results related to fine-tuning are provided. |
| CATT | Generally fine. But if most companies prefer the original one, we can also live with it. |
| Xiaomi | More evaluation about the fine-tuning is needed before we get the conclusion |
| Qualcomm | Similar to our comment above. Another natural solution can be to consider clutter-specific model training strategy. We propose this additional wording: Observation: Positioning accuracy of direct AI/ML positioning deteriorates when the ML model is trained with dataset of one clutter parameter setting (e.g., InF-DH {60%, 6m, 2m}), and tested with dataset of a different clutter parameter setting (e.g., InF-DH {40%, 2m, 2m}). As the clutter parameter setting of training dataset deviates further from that of training, worse positioning accuracy is observed. - By using multiple clutter parameters to compose the training dataset, the positioning accuracy is significantly improved if the clutter parameters used for the testing dataset are included in the training dataset. - The performance of the model can also be improved with fine-tuning, where the model is re-trained with a small dataset from the same drop as the tested dataset. - This deterioration in positioning performance can be avoided by considering clutter-specific model training that accounts for the expected testing clutter setting. |
| HW/HiSi | Support. Same comment as for 5.7-1. |
| vivo | Again, we have questions on the wording suggestion from Qualcomm. The propsoed observation is about direct AI/ML positioning generalization performane evaluation with different clutter settings.. The bullet added by Qualcomm is actually says another seperate model trained for that testing clutter setting. How is that categorized as generalization performance evaluation? We’re not sure about relevance here. |

Regarding Proposal 5.6-4 (generalization over network synchronization error), the view is divergent at the moment. This discussion is deferred as suggested by several companies.

## 3rd round discussion

For Proposal 5.7-1, the updated version Huawei/QC is more limited in scope. Thus the updated version below is provided for further checking.

Proposal 5.8-1 (generalization over drops)

Observation: When other simulation assumptions are held constant, positioning accuracy of direct AI/ML positioning deteriorates significantly when the ML model is trained with dataset of one drop, and tested with dataset of a different drop (i.e., different set of random seeds for channel propagation).

- Companies have provided initial evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning, for example:

  - The training dataset is composed of data from multiple drops;

  - Model fine-tuning is performed, where the model is re-trained with a small dataset from the same drop as the test dataset.

- This deterioration in positioning performance can be avoided by drop-specific model training which accounts for drop settings for test.

|  | Company |
| --- | --- |
| Support |  |
| Not support | vivo |

| Company | Comments |
| --- | --- |
| Fujitsu | Generally OK. |
| CMCC | OK with the proposal. Removing the red part to wait for more evaluation results about the fine-tuning is also fine to us. |
| vivo | We have questions on the 2nd bullet. The propsoed observation is about direct AI/ML positioning generalization performane evaluation with different drop, i.e., when the ML model is trained with dataset of one drop, and tested with dataset of a different drop. The 2nd bullet actually says another seperate model trained for that testing drop. How is that categorized as generalization performance evaluation? We’re not sure about relevance here. [Moderator] I assume you refer to the bullet on model re-training. It is not necessary to debate if model re-training is part of generalization evaluation or not. I remove such notation is Proposal marking. The important thing is that the observation reflects companies‘ evaluation results on different drops. |
| CAICT | Fine in general and ok with CMCC’s proposal. |

For Proposal 5.8-2, similar comments were made by companies. Thus the following version is presented, which follows the recommendation by Huawei/QC.

Proposal 5.8-2 (generalization over clutter parameters)

Observation: Positioning accuracy of direct AI/ML positioning deteriorates when the ML model is trained with dataset of one clutter parameter setting (e.g., InF-DH {60%, 6m, 2m}), and tested with dataset of a different clutter parameter setting (e.g., InF-DH {40%, 2m, 2m}). As the clutter parameter setting of training dataset deviates further from that of training, worse positioning accuracy is observed.

- Companies have provided initial evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning, for example:

  - The training dataset is composed of data from multiple clutter parameter settings, which include clutter parameter setting of the test dataset.

  - Model fine-tuning is performed, where the model is re-trained with a small dataset from the same clutter parameter setting as the test dataset.

- This deterioration in positioning performance can be avoided by clutter-specific model training which accounts for the expected clutter parameter for testing.

|  | Company |
| --- | --- |
| Support |  |
| Not support | vivo |

| Company | Comments |
| --- | --- |
| Fujitsu | Generally OK. |
| CMCC | OK with the proposal. Removing the red part to wait for more evaluation results about the fine-tuning is also fine to us. |
| vivo | Again, we have questions on the 2nd bullet. The propsoed observation is about direct AI/ML positioning generalization performane evaluation with different clutter settings.. The 2nd bullet actually says another seperate model trained for that testing clutter setting. How is that categorized as generalization performance evaluation? We’re not sure about relevance here. [Moderator] I assume you refer to the bullet on model re-training. It is not necessary to debate if model re-training is part of generalization evaluation or not. I remove such notation is Proposal marking. The important thing is that the observation reflects companies‘ evaluation results on different clutter settings. |
| CAICT | Fine in general and ok with CMCC’s proposal. |

## 4th round discussion

For the general observation of direct AI/ML positioning, this couldn’t be agreed. The concern was, there was no quantitative results to demonstrate what’s meant by ‘significantly improve’. Thus a quantitative description is added based on several companies’ evaluation results.

For the observations on drops and clutter parameters, there was a question why model re-training is counted as generalization evaluation. Moderator’s view is, It is not necessary to debate if model re-training is part of generalization evaluation or not. Model fine-tuning/re-training has been agreed to be studied. The observations are copied below for further checking.

Proposal 5.9-1

Observation: Direct AI/ML positioning can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods in heavy NLOS scenarios.

- Companies have submitted evaluation results to show that horizontal positioning error at CDF=90% can be reduced from >15m (conventional positioning method) to <1m (direct AI/ML positioning method) for clutter parameter {60%, 6m, 2m}.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | Question for clarification. Is this observation proposed to be captured into TR? [Moderator] The intention is, It’s useful to have some preliminary observation for sub use case discussion at RAN#98. SID: " - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 ". It can be discussed further how to capture results in TR. |
| Fujitsu | We wonder if the simulation condition should be added to this observation because the <1m results for AI/ML can only be achieved under rigid dataset selection (such as same drop, spatial consistency etc.) while the >15m results for the conventional methods can apply to different scenarios. Or shall we add some refining words such as “in the best case, <1m can be achieved by AIML ....”? |
| Apple | From review of the contributions, ≈ 1m is more accurate as opposed to < 1m |
| LG | Similar view with vivo |
| Samsung | We think the concern from oppo during online was that “it’s too early“. But we think it’s the basic obervation based on the results so far. The detailed observation on each cases/assumptoins will be seperatedly draw. |
| ZTE | We also feel it’s bit early to capture this in TR. We have two comments: Comment#1: Direct AI/ML positioning can also work well in low to medium LOS scenario Comment#2: For different model inputs, the positioning performance can be different. Shall we also capture what kinds of model input have been used to achieve good performance? |
| CATT | During online session, companies think this is too early. For the new added sub-bullet, we think the detail result should refer to some simulation results so that this observation can be proved. |
| HW/HiSi | Support. We think the observation captured the progress and evaluation results well. More detailed observations that will come later anyway. |
| OPPO | We believe the same or simiar observation will be made in the further for the TR. However, we still think it is two early to have an “overall“ observation since companies are still planing to submit more evaluation resutls, especially for the generalization performance. When we haven’t any detailed observations, how can we get an “overall“ observation without the conditions? As Fujitsu commented, this observation is based on some specific evalaution assumption. We suggest to focus on the observation for specific evalaution assumption/ scenarios, rather than rush into an “overall“ observation without any condition in this stage. |
| CAICT | Fine with the main bullet and more simulation results are prefered to finilize the wording in sub-bullet. |
| Nokia/NSB | We think that this observation is important to be at the end of the study. Now, we should prioritize other proposals related to agreeing on evaluation assumptions and related reporting of results. |

Proposal 5.9-2

Observation: When other simulation assumptions are held constant, positioning accuracy of direct AI/ML positioning deteriorates significantly when the ML model is trained with dataset of one drop, and tested with dataset of a different drop (i.e., different set of random seeds for channel propagation).

- Companies have provided initial evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning, for example:

  - The training dataset is composed of data from multiple drops;

  - Model fine-tuning is performed, where the model is re-trained with a small dataset from the same drop as the test dataset.

- This deterioration in positioning performance can be avoided by drop-specific model training which accounts for the drop settings for test.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | 1. Question for clarification. Is this observation proposed to be captured into TR? 2. I think our previous comment in the 2nd and 3rd round was misunderstood. Our point is that this proposed whole observation is based on evaluation results. The gain of testing dataset matched with trained model is already captured in observation in proposal 5.9-1. Either we remove the last bullet or if companies provided results and insisted on capturing this aspect again, that bullet should be the 3rd sub-bullet of the 1st bullet. - This deterioration in positioning performance can be avoided by drop-specific model training which accounts for the drop settings for test. |
| LG | Same as mentioned in the proposal 5.9.1 |
| ZTE | Similar comments in 5.9-1 should be clarified. |
| HW/HiSi | It looks fdine in principle. Questions for clarification on the last bullet: The drop-specific model training, - Does this mean off-line training in advance, e.g. when the model is trained with drop A and drop B and then inference is done e.g. with drop B?. - Does it is mean that the model is fine-tuned (which can be regarded as drop-specific training) with e.g. a small data set from the drop that is used for inference? |
| OPPO | We are ok with the main bullet since it is foucsing the specifci evalaution assumption. We are also ok with “ the positioning accuracy on the test dataset can be improved by better training dataset construction“ since better training data set will impove the performance seems universially true for AI model. For the other part, we prefer to wait for more evaluations. Meanwhile, one questions for clarification: 1.Do we plan to make a observation for intial evalaution resutls, and make an additional observation for further resutls? [Moderator] The intention is, It’s useful to have some preliminary observation for sub use case discussion at RAN#98. SID: " - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 ". It’ll be further discussed how to make additional observations, and how to capture observations in TR. |
| CAICT | Fine with the main bullet and more simulation results are prefered to finilize the wording in sub-bullet. |
| Nokia/NSB | We have a similar comment as with the previous observation. We request a clarification on the last bullet of the proposal:. “This deterioration in positioning performance can be avoided by drop-specific model training which accounts for the drop settings for test.” We wonder if this observation has any practical relevance, beyond simulator-based evaluations. |
| Qualcomm | We prefer the first bullet to include description on the amount of improvement expected from model finetuning and training on multiple drops. So far, most evaluations show some improvement but it still far from reaching the baseline performance. |

Proposal 5.9-3

Observation: Positioning accuracy of direct AI/ML positioning deteriorates when the ML model is trained with dataset of one clutter parameter setting (e.g., InF-DH {60%, 6m, 2m}), and tested with dataset of a different clutter parameter setting (e.g., InF-DH {40%, 2m, 2m}). As the clutter parameter setting of training dataset deviates further from that of training, worse positioning accuracy is observed.

- Companies have provided initial evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning, for example:

  - The training dataset is composed of data from multiple clutter parameter settings, which include clutter parameter setting of the test dataset.

  - Model fine-tuning is performed, where the model is re-trained with a small dataset from the same clutter parameter setting as the test dataset.

- This deterioration in positioning performance can be avoided by clutter-specific model training which accounts for the expected clutter parameter for testing.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | 1. Question for clarification. Is this observation proposed to be captured into TR? 2. I think our previous comment in the 2nd and 3rd round was misunderstood. Our point is that this proposed whole observation is based on evaluation results. The gain of testing dataset matched with trained model is already captured in observation in proposal 5.9-1. Either we remove the last bullet or if companies provided results and insisted on capturing this aspect again, that bullet should be the 3rd sub-bullet of the 1st bullet. - This deterioration in positioning performance can be avoided by clutter-specific model training which accounts for the expected clutter parameter for testing. |
| LG | Same as mentioned in the proposal 5.9.1 |
| ZTE | Similar comments in 5.9-1 should be clarified. |
| OPPO | Similar comment as before |
| CAICT | Same comments as before. |
| Nokia/NSB | Similar comments as before. We are fine in general with the assumption that fine-tuning is required for generalization, but without knowing the details of the dataset (e.g., quality) we wonder whether we can quantify the size of dataset for the fine-tuning. |
| Qualcomm | Same to previous proposal. We prefer the first bullet to include description on the amount of improvement expected from model finetuning and training on multiple clutter settings. So far, most evaluations show some improvement but it still far from reaching the baseline performance. |

## 5th round discussion

For Proposal 5.9-1, there are still comments on the detailed observation in the sub-bullet. If the sub-bullet is deleted, then there are questions on what’s meant by “significantly improve”. This proposal can wait for more evaluation results and more accurate observations.

For Proposal 5.9-2, the main bullet and the sub-bullet on better training dataset is generally acceptable. The fine-tuning aspect and drop-specific model can be further discussed. Thus the following is proposed.

Proposal 5.10-1

Observation: When other simulation assumptions are held constant, positioning accuracy of direct AI/ML positioning deteriorates significantly when the ML model is trained with dataset of one drop, and tested with dataset of a different drop (i.e., different set of random seeds for channel propagation).

- Companies have provided initial evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning, for example:

  - The training dataset is composed of data from multiple drops;

  - Model fine-tuning is performed, where the model is re-trained with a small dataset from the same drop as the test dataset.

- This deterioration in positioning performance can be avoided by drop-specific model training which accounts for the drop settings for test.

|  | Company |
| --- | --- |
| Support |  |
| Not support | Vivo |

| Company | Comments |
| --- | --- |
| vivo | We don’t see a strong need to agree this proposed observation right now given the comments from moderator ‘The intention is, It’s useful to have some preliminary observation for sub use case discussion at RAN#98. SID: „ - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 „. It’ll be further discussed how to make additional observations, and how to capture observations in TR.‘ We’re also not sure why the 2nd sub-bullet on fine-tuing is removed. Which aspect of fine-tuning is not refelcting the observation based on companies’ evaluation reuslts. If the argument is no detailed performance gain, that also applies to „better training dataset construction” when the bullet simply says “the positioning accuracy on the test dataset can be improved” We cannot accept this proposal. |
| Qualcomm | We suggest to postpone drawing observations and conclusions at this stage of the study until more evaluations are provided by other companies. In addition, it is better to capture this and other observations in a similar manner done in TR 38.857 of Rel17 (e.g., Section 8), where individual observations are captured and then a list of conclusions is provided. Regarding this observation, it also needs to capture full picture on how direct AI/ML positioning can be realized for multiple channel settings (i.e., drops). The modified observation captures only a single aspect on how to improve the deterioration in performance of direct AI/ML positioning for testing on different drop. In addition, the current wording does not describe how much improvement would the realized from this approach. When captured in TR, this will indicate that training dataset construction using multiple drops is the only solution. We need also to capture that drop-specific training, i.e., training multiple models for different drops, is another solution to avoid this deterioration in performance. In addition, it is not clear how the multiple drops in “The training dataset is composed of data from multiple drops” relate to the testing drop. Should multiple drops include the testing drop or shouldn’t? |

For Proposal 5.9-3, the main bullet and the sub-bullet on better training dataset is generally acceptable. The fine-tuning aspect and clutter-specific model can be further discussed. Thus the following is proposed.

Proposal 5.10-2

Observation: Positioning accuracy of direct AI/ML positioning deteriorates when the ML model is trained with dataset of one clutter parameter setting (e.g., InF-DH {60%, 6m, 2m}), and tested with dataset of a different clutter parameter setting (e.g., InF-DH {40%, 2m, 2m}). As the clutter parameter setting of training dataset deviates further from that of training, worse positioning accuracy is observed.

- Companies have provided initial evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning, for example:

  - The training dataset is composed of data from multiple clutter parameter settings, which include clutter parameter setting of the test dataset.

  - Model fine-tuning is performed, where the model is re-trained with a small dataset from the same clutter parameter setting as the test dataset.

- This deterioration in positioning performance can be avoided by clutter-specific model training which accounts for the expected clutter parameter for testing.

|  | Company |
| --- | --- |
| Support |  |
| Not support | vivo |

| Company | Comments |
| --- | --- |
| vivo | We don’t see a strong need to agree this proposed observation right now given the comments from moderator ‘The intention is, It’s useful to have some preliminary observation for sub use case discussion at RAN#98. SID: " - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 ". It’ll be further discussed how to make additional observations, and how to capture observations in TR.‘ We’re also not sure why the 2nd sub-bullet on fine-tuing is removed. Which aspect of fine-tuning is not refelcting the observation based on companies’ evaluation reuslts. If the argument is no detailed performance gain, that also applies to „better training dataset construction” when the bullet simply says “the positioning accuracy on the test dataset can be improved” We cannot accept this proposal. |
| Qualcomm | Similar to our comment on Proposal 5.10.1. We suggest to postpone drawing observations and conclusions at this stage of the study until more evaluations are provided by other companies. In addition, it is better to capture this and other observations in a similar manner done in TR 38.857 of Rel17 (e.g., Section 8), where individual observations are captured and then a list of conclusions is provided. For this observation, it also needs to capture full picture on how direct AI/ML positioning can be realized for multiple clutter settings. The modified observation captures only a single aspect on how to improve the deterioration in performance of direct AI/ML positioning for testing on different clutter settings. In addition, the current wording does not describe how much improvement would the realized from this approach. When captured in TR, this will indicate that training dataset construction using multiple clutter settings is the only solution. We need also to capture that clutter-specific training, i.e., training multiple models for different clutter settings, is another solution to avoid this deterioration in performance. |

# (Closed) Performed Evaluation of AI/ML-assisted positioning

In this meeting, a large amount of evaluation work has been performed by companies for AI/ML-assisted positioning. These valuable results are crucial to help RAN1 to make progress.

Selected results submitted by companies are copied below.

## Evaluation without generalization considerations

- Ericsson (R1-2208399) Table 29 Accuracy of the LoS classification results for AI/ML assisted positioning Training dataset; Test dataset; Model I LoS classification accuracy; Model II LoS classification accuracy. {40%, 2m, 2m}; {40%, 2m, 2m} same drop; 0.957; 0.959. {40%, 2m, 2m} new drop; 0.959; 0.960. {50%, 2m, 2m} new drop; 0.960; 0.962. {60%, 2m, 2m} new drop; 0.962; 0.964. {60%, 6m, 2m} new drop; 0.952; 0.954. {40%, 2m, 2m} with no network synchronization errors; {40%, 2m, 2m} STD=0ns; 0.959. {40%, 2m, 2m} STD=2ns; 0.961. {40%, 2m, 2m} STD=6ns; 0.960. {40%, 2m, 2m} STD=20ns; 0.958. {40%, 2m, 2m} STD=50ns; 0.939. {40%, 2m, 2m} with STD = 25 ns network synchronization errors; {40%, 2m, 2m} STD=0ns; 0.948. {40%, 2m, 2m} STD=2ns; 0.950. {40%, 2m, 2m} STD=6ns; 0.954. {40%, 2m, 2m} STD=20ns; 0.956. {40%, 2m, 2m} STD=50ns; 0.955. Table 32 Evaluation results for AI/ML model deployed on network-side, without model generalization investigation. No network synchronization error. Two architectures of the ML model: Architecture I (6 layers: 3 Conv1D layers, 3 Dense layers) and Architecture II (9 layers: 6 Conv1D layers, 3 Dense layers) Model; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training/ validation; test; Model complexity; Computation complexity. I; Time domain CIR, 256x2 complex array; (1). LoS/NLoS classification (2). ToA estimate; Ideal; {40%,2m,2m}; 108,000 link CIR; 72,000 link CIR; 35,447 complex parameters; 404,655 FLOPs; 0.113. II; 36,512 complex parameters; 944,387 FLOPs; 0.063.

- Huawei, HiSilicon (R1-2208433) Table 9. Evaluation results for AI/ML model deployed on network-side, without model generalization Model structure; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CNN; Normalized PDP; LOS probability; Ideal LOS/NLOS identification (LOS probability=0% or 100%); {40%, 2m, 2m}; 18000; 9000; 582; 192K; 0.353.

- ZTE (R1-2208525) Figure.7 CDFs of positioning errors for sub-use case 2-1 (Grid width is 0.5 m)

- vivo (R1-2208638) Table 3. Evaluation results for AI/ML model deployed on UE or Network side, without model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6, 6, 2}; 25k; 1k; 1.65M; 22.30M; 0.99. CIR; TOA; 0; {0.6, 6, 2}; 25k; 1k; 44M; 1.45G; 0.60. CIR; Pos.; 0; {0.6, 6, 2}; 25k & Drop1; 1k & Drop2; 1.65M; 22.30M; 6.00. CIR; TOA; 0; {0.6, 6, 2}; 25k & Drop1; 1k & Drop2; 44M; 1.45G; 2.51. Table 4. CDF of estimation accuracy of intermediate feature TOA (meter) Scenario; Methods; 50%; 67%; 80%; 90%. InF-DH {0.6,6,2}; AI/ML based TOA est. (CIR-TOA); 0.43; 0.86; 1.69; 3.74. Observation 3: AI/ML based TOA estimation for positioning has great advantages in positioning performance, deployment flexibility, compatibility with existing positioning protocol framework, and generalization capability. Table 5. Evaluation results of LOS/NLOS identification accuracy for AI/ML model deployed on UE or Network side, without model generalization, full-connection network Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Accuracy of LOS/NLOS identification. Training; test; Model complexity; Computational complexity; AI/ML. CIR; LOS/NLOS; 0; {0.4, 2, 2}; 25k; 1k; 3.62M; 7.24M; >99%. R17 [9]; {0.4, 2, 2}; /; 93%. Table 6. Evaluation results for AI/ML model deployed on UE or Network side, without model generalization, full-connection network Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; LOS/NLOS; 0; {0.4, 2, 2}; 25k; 1k; 3.62M; 7.24M; 1.10. CIR; TOA; 0; {0.6, 6, 2}; 25k; 1k; 1.65M; 22.30M; 0.60. Observation 4: AI/ML based LOS/NLOS identification for positioning has the following advantages: - More accurate LOS/NLOS identification along with a confidence metric - Better compatibility with existing positioning protocol framework. - Great generalization capability. And disadvantages: - Positioning performance could suffer from severe degradation in heavy-NLOS scenarios. - Obtain LOS/NLOS labels is a challenging task for data collection. Proposal 4: Capture in the TR the benefits of AI/ML assisted positioning in terms of positioning accuracy and AI model generalization.

- Qualcomm (R1-2209980) - Table 8. Evaluation results for AI/ML model deployed on UE-side, without model generalization Method; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Hard-decision; CER (2, 64); Single value of ToA; 0%; {60%, 6m, 2m}; 18000 (1000 Ues * 18 TRPS); 18000 (1000 Ues * 18 TRPS); 22K params; 206K FLOPs; 25.0. Soft-information; CER (2, 64); Distribution of ToA; 0%; {60%, 6m, 2m}; 18000 (1000 Ues * 18 TRPS); 18000 (1000 Ues * 18 TRPS); 22K params; 206K FLOPs; 5.1. - Table 9. Evaluation results for AI/ML model deployed on UE-side, without model generalization Method; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Hard-decision; CER (2, 64); Single value of ToA; 0%; {40%, 4m, 2m}; 18000 (1000 Ues * 18 TRPS); 18000 (1000 Ues * 18 TRPS); 22K params; 206K FLOPs; 14.8. Soft-information; CER (2, 64); Distribution of ToA; 0%; {40%, 4m, 2m}; 18000 (1000 Ues * 18 TRPS); 18000 (1000 Ues * 18 TRPS); 22K params; 206K FLOPs; 0.5. Observation11: The soft-decision algorithm outperforms the hard-decision approach for AI-ML-assisted positioning. - The 90th percentile positioning error improves from 25.0 m to 5.1 m for the {60%, 6m, 2m} clutter setting - The 90th percentile positioning error improves from 14.8 m to 0.5 m for the {40%, 4m, 2m} setting.

- CATT (R1-2208971) Table 7: Evaluation results for AI/ML model deployed on UE/LMF-side, without model generalization, resnet18 Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Type: CIR; Size: [18, 256, 2]; Type: ToA; Size: [1, 18]; ToA with 100% ground truth label; {60%, 6m, 2m}; 20000; 1600; 12.7M; 4.42G FLOPs; 0.77m. Table 8: Evaluation results for AI/ML model deployed on UE/LMF-side, without model generalization, resnet18 Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Type: CIR; Size: [18, 256, 2]; Type: ToA; Size: [1, 2]; ToA with 100% ground truth label; {60%, 6m, 2m}; 20000; 1600; 12.7M; 4.42G FLOPs; 0.7m.

- xiaomi (R1-2209281) Table 2 Evaluation results for AI-based ToA predication with model deployed on UE or NW side, without model generalization, ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*256*2 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.6，6，2}; {0.6，6，2}; 70000; 10000; 21,285,650; 5.76Gflops; 0.6778. 18*256*2 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.4，2，2}; {0.4，2，2}; 70000; 10000; 21,285,650; 5.76Gflops; 0.8533.

- MediaTek (R1-2209510) Sampling period 1ns: Table 1. UE side model/CNN Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. non-normalized PDP(512*1); Los/Nlos probability; Ideal Los/Nlos; {40%, 2m, 2m}; 1800UE*18BS*4drop*0.9 =116640; 1800UE*18BS*4drop*0.1 =12960; 4166; 73.74k; 0.51. normalized PDP(512*1); Los/Nlos probability; Ideal Los/Nlos; {40%, 2m, 2m}; 1800UE*18BS*4drop*0.9 =116640; 1800UE*18BS*4drop*0.1 =12960; 4166; 73.74k; 5.9. Sampling period 9.3ns Table 2. UE side model/CNN Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. non-normalized PDP(256*1); Los/Nlos probability; Ideal Los/Nlos; {40%, 2m, 2m}; 1800UE*18BS*4drop*0.9 =116640; 1800UE*18BS*4drop*0.1 =12960; 2118; 36.876k; 17.2.

## Evaluation of generalization aspects

### Generalization aspect: different drops

- Ericsson (R1-2208399) Table 10 Accuracy of the LoS classification results for AI/ML assisted positioning Training dataset; Test dataset; Model I LoS classification accuracy; Model II LoS classification accuracy. {40%, 2m, 2m}; {40%, 2m, 2m} same drop; 0.957; 0.959. {40%, 2m, 2m} new drop; 0.959; 0.960. {50%, 2m, 2m} new drop; 0.960; 0.962. {60%, 2m, 2m} new drop; 0.962; 0.964. {60%, 6m, 2m} new drop; 0.952; 0.954. {40%, 2m, 2m} with no network synchronization errors; {40%, 2m, 2m} STD=0ns; 0.959. {40%, 2m, 2m} STD=2ns; 0.961. {40%, 2m, 2m} STD=6ns; 0.960. {40%, 2m, 2m} STD=20ns; 0.958. {40%, 2m, 2m} STD=50ns; 0.939. {40%, 2m, 2m} with STD = 25 ns network synchronization errors; {40%, 2m, 2m} STD=0ns; 0.948. {40%, 2m, 2m} STD=2ns; 0.950. {40%, 2m, 2m} STD=6ns; 0.954. {40%, 2m, 2m} STD=20ns; 0.956. {40%, 2m, 2m} STD=50ns; 0.955. Table 11 LoS link ToA estimation error results for AI/ML assisted positioning Training dataset; Test dataset; Model I 90%tile ToA error [m]; Model II 90%tile ToA error [m]. {40%, 2m, 2m}; {40%, 2m, 2m} same drop; 0.164; 0.091. {40%, 2m, 2m} new drop; 0.145; 0.082. {50%, 2m, 2m} new drop; 0.159; 0.090. {60%, 2m, 2m} new drop; 0.152; 0.087. {60%, 6m, 2m} new drop; 0.178; 0.097. {40%, 2m, 2m} with no network synchronization errors; {40%, 2m, 2m} STD=0ns; 0.082. {40%, 2m, 2m} STD=2ns; 0.089. {40%, 2m, 2m} STD=6ns; 0.093. {40%, 2m, 2m} STD=20ns; 0.099. {40%, 2m, 2m} STD=50ns; 0.161. {40%, 2m, 2m} with STD = 25 ns network synchronization errors; {40%, 2m, 2m} STD=0ns; 0.157. {40%, 2m, 2m} STD=2ns; 0.116. {40%, 2m, 2m} STD=6ns; 0.103. {40%, 2m, 2m} STD=20ns; 0.094. {40%, 2m, 2m} STD=50ns; 0.107.

- Qualcomm (R1-2209980) Observation13: The ML-assisted soft information reporting method using single-TRP approach generalizes well across inter-site changes with homogeneous clutter settings. Figure 18. CDF of horizontal positioning error for ML-based soft information reporting across drops

### Generalization aspect: different clutter parameters

- Ericsson (R1-2208399) Table 33. Evaluation results for AI/ML model deployed on network-side, with model generalization investigation where the model is trained in InF-DH {40%, 2m, 2m} and tested with various InF-DH clutter parameters and new drop. No network synchronization error. Two architectures of the ML model: Architecture I (6 layers: 3 Conv1D layers, 3 Dense layers) and Architecture II (9 layers: 6 Conv1D layers, 3 Dense layers) Model; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training/ validation; test; Model complexity; Computation complexity. I; Time domain CIR, 256x2 complex array; (1). LoS/NLoS classification (2). ToA estimate; Ideal; {40%,2m,2m}; 108,000 link CIR; 72,000 link CIR; 35,447 complex parameters; 404,655 FLOPs; 0.106. {50%,2m,2m}; 0.264. {60%, 2m, 2m}; 5.340. {60%, 6m, 2m}; 13.476. II; {40%,2m,2m}; 36,512 complex parameters; 944,387 FLOPs; 0.062. {50%,2m,2m}; 0.150. {60%, 2m, 2m}; 4.732. {60%, 6m, 2m}; 13.528.

- Huawei, HiSilicon (R1-2208433) Table 10. Evaluation results for AI/ML model generalization capability in the aspect: Clutter parameters Evaluated Scenarios; Dataset; Configurations; Dataset size; Identification rate. 1; Training; Clutter paras: InF-DH {40%, 2m, 2m}; 18000; 97.2%. Test; 9000. 2; Training; Clutter paras: InF-DH {40%, 2m, 2m}; 18000; 98.6%. Test; Clutter paras: InF-DH {60%, 6m, 2m}; 9000. 3; Training; Clutter paras: InF-DH {40%, 2m, 2m}; 18000; 97.7%. Test; Clutter paras: InF-DH {40%, 3m, 5m}; 9000.

- Qualcomm (R1-2209980) Observation14: Training on a mix of clutter settings achieves good accuracy in each setting without the overhead of model switching, while training a separate model for each setting provides better accuracy. - The 90th percentile error increases from 5.10 m to 7.34 m when testing on (60%, 6m, 2m) clutter, and - from 0.53 m to 0.91 m when testing on (40%, 2m, 2m) clutter Figure 19. CDF of horizontal positioning error for ML-based soft information reporting across clutter settings

- xiaomi (R1-2209281) Table 4 Evaluation results for AI-based ToA predication with model deployed on UE or NW side, with model generalization (different clutter parameter), ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*256*2 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.6,6,2}; {0.4,2,2}; 70000; 10000; 21,285,650; 5.76GFlops; 7.1173. 18*256*2 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.4,2,2}; {0.6,6,2}; 70000; 10000; 21,285,650; 5.76GFlops; 1.5413. 18*256*2 CIR; 18*1 UE TOA; 18*1 UE TOA; Mix of {0.6,6,2} {0.4,2,2}; {0.6,6,2}; 35000+35000; 10000; 21,285,650; 5.76GFlops; 0.6867. 18*256*2 CIR; 18*1 UE TOA; 18*1 UE TOA; Mix of {0.6,6,2}, {0.4,2,2}; {0.4,2,2}; 35000+35000; 10000; 21,285,650; 5.76GFlops; 0.7974.

- CATT (R1-2208971) Table 9: Evaluation results for AI/ML model deployed on UE/LMF-side, with model generalization, resnet18 Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Type: CIR; Size: [18, 256, 2]; Type: UE’s position; Size: [1, 2]; UE’s position with 100% ground truth label; Training:{60%, 6m, 2m} Testing:{40%, 2m, 2m}; 20000; 1600; 12.7M; 4.42G FLOPs; 3.11m.

### Generalization aspect: network synchronization error

- Ericsson (R1-2208399) Table 34. Evaluation results for AI/ML model deployed on network-side, with model generalization investigation where the model is trained in InF-DH {40%, 2m, 2m} with no network synchronization errors, and tested with InF-DH {40%, 2m, 2m} and a range of network synchronization errors. Architecture II (9 layers: 6 Conv1D layers, 3 Dense layers). Model; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training/ validation; test; Model complexity; Computation complexity. II; Time domain CIR, 256x2 complex array; (1). LoS/NLoS classification (2). ToA estimate; Ideal; {40%,2m,2m}, T1=0ns; 108,000 link CIR; 72,000 link CIR; 36,512 complex parameters; 944,387 FLOPs; 0.062. {40%,2m,2m}, T1=2ns; 0.997. {40%,2m,2m}, T1=6ns; 2.926. {40%,2m,2m}, T1=20ns; 9.200. {40%,2m,2m}, T1=50ns; 22.149. Table 35. Evaluation results for AI/ML model deployed on network-side, with model generalization investigation where the model is trained in InF-DH {40%, 2m, 2m} with network synchronization error STD T1=25ns, and tested with InF-DH {40%, 2m, 2m} and a range of network synchronization errors. Architecture II (9 layers: 6 Conv1D layers, 3 Dense layers). Model; Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training/ validation; test; Model complexity; Computation complexity. II; Time domain CIR, 256x2 complex array; (1). LoS/NLoS classification (2). ToA estimate; Ideal; {40%,2m,2m}, T1=0 ns; 108,000 link CIR; 72,000 link CIR; 36,512 complex parameters; 944,387 FLOPs; 0.126. {40%,2m,2m}, T1=2 ns; 1.096. {40%,2m,2m}, T1=6 ns; 2.945. {40%,2m,2m}, T1=20 ns; 9.100. {40%,2m,2m}, T1=50 ns; 21.459.

- CATT (R1-2208971) Table 10: Evaluation results for AI/ML model deployed on UE/LMF-side, with model generalization [Training dataset with perfect network synchronization and testing dataset with network synchronization error], resnet18 Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. Type: CIR; Size: [18, 256, 2]; Type: UE’s position; Size: [1, 2]; UE’s position with 100% ground truth label; {60%, 6m, 2m}; 20000; 1600; 12.7M; 4.42G FLOPs; 12.8m.

## Evaluation of model fine-tuning

- vivo (R1-2208638) Table 16 Evaluation results of fine-tuning for AI/ML model deployed on UE or Network side, with model generalization, CNN Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Train; Fine-tune; test; Model complexity; Computational complexity; AI/ML. CIR; TOA; 0; {0.6, 6, 2}; 25k & InF-DH; 0; 1k & InF-HH; 44M; 1.45G; >10. CIR; TOA; 0; {0.6, 6, 2}; 25k & InF-HH; 0; 1k & InF-DH; 44M; 1.45G; >10. CIR; TOA; 0; {0.6, 6, 2}; 25k & InF-DH; 1k& InF-HH; 1k & InF-HH; 44M; 1.45G; 0.28.

## Evaluation of issues related to ground truth labels

- Nokia (R1-2209371) Table 2: Model information and evaluation results for AI/ML model for on-demand labelling and random labelling. Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Required training data size to reach accuracy at 82%. Training; Test; Model complexity; Computational complexity; AI/ML. CIR from 1 BS; Classification 0/1; LOS (1) / NLOS (0); 40%; 10K including evaluation; 2K; 0.31M; 4.33M flops; 680 (red line in Fig. 2). Degree of classification confidence. CIR from 1 BS; Classification 0/1; LOS (1) / NLOS (0); 40%; 10K including evaluation; 2K; 0.24M; 2.38M flops; 2010 (blue line in Fig. 2).

## Other evaluation results

### Zone-specific changes

- Qualcomm (R1-2209980) Observation15: The ML-assisted soft information reporting using single-TRP approach generalizes well across zone-specific changes. Figure 20. CDF of horizontal positioning error for ML-based soft information reporting across zone-specific generalizations.

### Channel estimation

- vivo (R1-2208638) Table 10 Evaluation results for AI/ML model deployed on UE or Network side, without model generalization, ViT Model input; Model output; Label; Clutter param; Dataset size & type; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = -30dB; 1k & SINR = -30dB; 1.65M; 22.30M; 6.89. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = -25dB; 1k & SINR = -25dB; 1.65M; 22.30M; 4.31. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = -20dB; 1k & SINR = -20dB; 1.65M; 22.30M; 1.84. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = -10dB; 1k & SINR = -10dB; 1.65M; 22.30M; 1.83. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = 10dB; 1k & SINR = 10dB; 1.65M; 22.30M; 1.46. CIR; Pos.; 0; {0.6, 6, 2}; 25k & SINR = 30dB; 1k & SINR = 30dB; 1.65M; 22.30M; 1.34.

### Reduced input dimension

- xiaomi (R1-2209281) Proposal 1: On the basis of satisfying the positioning accuracy requirement, study solution to reduce the model size, computation complexity and involved signalling overhead Table 7 Evaluation results for reduced input dimension for direct AI-based positioning and AI-based ToA predication, model deployed on UE or NW side, without model generalization, ResNet Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training; test; Model complexity; Computational complexity; AI/ML. 18*24*2 CIR; 2*1 UE coordinates; 2*1 UE coordinates; {0.6,6,2}; {0.6,6,2}; 70000; 10000; 21,277,442; 539.94MFlops; 0.8219. 18*24*2 CIR; 18*1 UE TOA; 18*1 UE TOA; {0.6,6,2}; {0.6,6,2}; 70000; 10000; 21,285,650; 539.95MFlops; 0.8993.

## 1st round discussion

Based on the evaluation results submitted by companies, several observations were made by majority companies. No contradictory simulation results were submitted thus far. Accordingly, the following proposals are made, which are the result of merging several companies’ proposals.

Proposal 6.6-1 (overall)

Observation: AI/ML assisted positioning can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods in low to medium NLOS scenarios (e.g., InF-DH {40%, 2m, 2m}).

|  | Company |
| --- | --- |
| Support | Qualcomm, Ericsson |
| Not support |  |

| Company | Comments |
| --- | --- |
| vivo | In our contribution, we provided the results of AI/ML assisted positioning for heavy NLOS scenario (i.e., InF-DH {60%, 6m, 2m}) and showed significant gain as well. It's not clear to us why the observation only capturing gain in low to medium NLOS scenarios. |
| Nokia/NSB | We have similar concerns as vivo, and are not fully sure about the intent of the observation. If the intermediate feature that is the output of the AI/ML model is LOS/NLOS classification, we agree that such models are beneficial for low-to-medium NLOS scenarios. Perhaps it would be beneficial to clarify for which model outputs the observation is applicable. |
| LG | We have a similar view with Nokia. |
| OPPO | it is premature to have some conclusion without any prerequisite (expecting the NLOS scenarios). As companies will submit more evaluation results in the future meetings, we prefer to defer this proposal to see more results of generalization performance. |
| Qualcomm | We are ok with the observation |
| CMCC | We also see the performance gain in high NLOS scenario. |
| ZTE | Agree with vivo, even in heavy NLOS scenario, AI/ML assisted positioning can work well. |
| HW/HiSi | Agree with the observation. As a compromise, to address the previous comments, we can also remove the last part: Compromise updated Proposal 6.6-1 (overall) Observation: AI/ML assisted positioning can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods in low to medium NLOS scenarios (e.g., InF-DH {40%, 2m, 2m}). |
| CATT | In our contribution, we provide the results in high NLOS scenario (i.e., InF-DH {60%, 6m, 2m}) and showed significant gain as well. Agree with HW’s update. |
| Lenovo | The positive effect on heavy NLOS scenarios should also be considered as well as part of the general observation. |

Proposal 6.6-2 (generalization over drops)

Observation: ML model in AI/ML assisted positioning has robust generalization capability when the model is trained with dataset of one drop, and tested with dataset of a different drop (i.e., different set of random seeds for channel propagation).

|  | Company |
| --- | --- |
| Support |  |
| Not support | Qualcomm |

| Company | Comments |
| --- | --- |
| Qualcomm | We propose to add wording to reflect that this observation only applies to AI/ML assisted positioning that considers single TRP for model input. There is still no sufficient evaluations that show the robustness of AI/ML assisted positioning with multiple TRP input to different drops or clutter settings. |
| ZTE | Wait for more evaluation results in next meeting. This is related to Proposal 2.1.1-1. |
| HW/HISi | For LOS identification, this observation is true. But for ToA there are not enough results yet to observe this. We can agree now on the following: Updated Proposal 6.6-2 (generalization over drops) Observation: ML model in AI/ML assisted positioning has robust generalization capability at least for LOS identification when the model is trained with dataset of one drop, and tested with dataset of a different drop (i.e., different set of random seeds for channel propagation). FFS: ToA estimation |
| CATT | We prefer to wait for more evaluation results |

Proposal 6.6-3 (generalization over clutter parameters)

Observation: Even with high-quality input (e.g., LOS/NLOS identification) provided by ML model, positioning accuracy of the conventional positioning algorithm is limited by the inadequate LOS links in heavy NLOS scenario.

|  | Company |
| --- | --- |
| Support |  |
| Not support | Qualcomm, CATT |

| Company | Comments |
| --- | --- |
| Qualcomm | Observation is not related to generalization |
| HW/HiSi | It does not seem right to make an observation about the conventional algorithm in the context of generalization. Could it therefore please be clarified how the utilization of conventional algorithms relates to the generalization of the AI model over clutter parameters? In the proposed observation it is said that the output from the AI model has high quality. Then, for generalization, we can conclude that the AI performs well and do need to do any observation. |
| CATT | We wonder why we just focus on LOS/NLOS identification. From our simulation results, using TOA can still have some performance gain in heavy NLOS scenario. |

## 2nd round discussion

Based on companies’ input, Proposal 6.6-1 (overall) need be updated to account for high NLOS scenario. Thus the updated version by HW/HiSi is used below.

Proposal 6.7-1 (overall)

Observation: AI/ML assisted positioning can significantly improve the positioning accuracy compared to existing RAT-dependent positioning.

|  | Company |
| --- | --- |
| Support | CATT, HW/HiSi |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | We feel that this can be discussed later until we have more results on model generalization for AI/ML assisted positioning. In addition, do we also need to say the accuracy of intermediate results is also observed. |
| OPPO | It is pre-mature to have a general observation since some acheme does not have good generalization performance for some cases. More evalautions are needed. |
| Nokia/NSB | We tend to agree with other companies that this proposal is premature at this stage of the evaluations. |
| Apple | Agree with other companies |
| CAICT | Similar view as OPPO. |
| CATT | We don’t think this observarion related with generalization. But if other companies think may need more simulation, we can also defer the discussion. |
| InterDigital | Similar view as ZTE |
| Qualcomm | The observation is too general and need to be more specific. It is too early to discuss this observation as there are many flavors of AI/ML assisted positioning. Observation of each flavor is better to be treated separately. In addition, the gain of AI/ML assisted positioning depends on clutter. For example, the gain of LOS identification cannot be seen in high clutter setting (e.g., {60%, 6m, 2m}). |
| HW/HiSi | Support Regarding the comments from ZTE and Oppo. The generalization performance should not be considered when making this observation. This aspect has not been considered for the corresponding observation on direct fingerprint positioning either (Proposal 5.6-1). |
| LG | Agree with other companies’ comment |

Regarding proposals 6.6-2 and 6.6-3, more discussion is needed.

# (Closed) Template for reporting evaluation results

Regarding the template for reporting evaluation results, the following suggestions were made.

- Huawei, HiSilicon (R1-2208433) Proposal 6: RAN1 should define a unified table structure to present results of generalization studies. It is proposed to have a different table for each dimension of generalization that is investigated, e.g. one for different drops, one for different clutter parameters, etc. One example is given in the template below. Table X. Evaluation results for AI/ML model generalization capability in the aspect: Different Drops Source; Evaluated Scenarios; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). Huawei, HiSilicon; 1; Training; Drop i; M. Test; Drop j; N. Proposal 7: RAN1 should define typical evaluation cases for model generalization in the aspect - Different Drops, to better compare results across companies. In addition, further cases should also be captured to better explore and expand the generalization capabilities of the AI/ML model. One example is given in the template below. Table X. Evaluation results for AI/ML model generalization capability in the aspect: Different Drops Source; Evaluated Scenarios; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). Huawei, HiSilicon; 1; Training; Drop i; M. Test; Drop j; N. 2; Training; k Drops mixed; M (M/k per drop). Test; Drop I (outside of the trained Drops); N. 3; Training; k Drops mixed; M (M/k per drop). Test; Drop j (inside the trained Drops); N. …; Training; …. Test; …. Proposal 8: RAN1 should define typical evaluation cases for model generalization in the aspect - Clutter parameters, to better compare results of generalization studies among companies. In addition, further cases should also be captured to better explore and expand the generalization capabilities of the AI/ML model. One example is given in the template below. Table X. Evaluation results for AI/ML model generalization capability in the aspect: Clutter parameters Source; Evaluated Scenarios; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). Huawei, HiSilicon; 1; Training; Clutter paras: a; M. Test; Clutter paras: b; N. 2; Training; Clutter paras: a&b mixed; M (M/2 per paras). Test; Clutter paras: a; N. 3; Training; Clutter paras: a&b mixed; M (M/2 per paras). Test; Clutter paras: b; N. …; Training; …. Test; …. Proposal 10: RAN1 should define typical evaluation cases for model generalization in the aspect - Network synchronization error, to better compare results of generalization studies among companies. In addition, further cases should also be captured to better explore and expand the generalization capabilities of the AI/ML model. One example is given in the template below. Table X. Evaluation results for AI/ML model generalization capability in the aspect: Network synchronization error Source; Evaluated Scenarios; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). Huawei, HiSilicon; 1; Training; Without network synchronization error; M. Test; With network synchronization error @A ns; N. 2; Training; With network synchronization error @A ns; M. Test; N. 3; Training; With network synchronization error @ B paras mixed; M (M/ B per paras). Test; N (N/ B per paras). …; Training; …. Test; …. Proposal 12: RAN1 should define some typical evaluation cases for evaluate the generalization in the aspect - UE timing error to better compare results of generalization studies among companies. In addition, further cases should also be captured to better explore and expand the generalization capabilities of the AI/ML model. One example is given in the template below. Table X. Evaluation results for AI/ML model generalization capability in the aspect: UE timing error Source; Evaluated Scenarios; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). Huawei, HiSilicon; 1; Training; Without UE timing error; M. Test; With UE timing error @A ns; N. 2; Training; With UE timing error @A ns; M. Test; N. 3; Training; With UE timing error @ B paras mixed; M (M/ B per paras). Test; N (N/ B per paras). …; Training; …. Test; …. Proposal 14: RAN1 should define typical cases to evaluate cases the performance gain brought by fine-tuning to better compare results among companies. One example is given in the template below. Table X. Model update evaluation results for AI/ML model generalization capability Source; Evaluated aspects; Dataset; Configurations; Dataset size; Horizontal positioning accuracy at CDF=90% (meters). Before model update; After model update. Huawei, HiSilicon; Different Drops; Training; Drop i; M. Fine-tuning; Drop j; K. Test; Drop j; N. Clutter parameters; Training; Clutter paras: a; M. Fine-tuning; Clutter paras: b; K. Test; Clutter paras: b; N. UE timing error; Training; Without UE timing error; M. Fine-tuning; With UE timing error @A ns; K. Test; With UE timing error @A ns; N. …; Training; …. Fine-tuning. Test; ….

## 1st round discussion

The main point in Huawei/HiSi proposal is, separate entries are necessary to report the conditions used in obtaining the training/validation dataset, vs the condition used in obtaining the test dataset. Thus it is sufficient to expand the agreed template to as below. Companies can describe in details in settings used in training/validation and testing: drops, clutter parameters, network synchronization error, mixed data from two different drops, …

Note that this template has been used by companies in their contributions, e.g.,xiaomi (R1-2209281).

Based on the above, the template for reporting evaluation results is updated to the following.

Proposal 7.1-1

For both direct AI/ML positioning and AI/ML assisted positioning, the agreed table is updated to the following for reporting the evaluation results.

Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [short model description]

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |

*Note: for AI/ML assisted positioning, if it is agreed that single-TRP vs multi-TRP should be reported, then this aspect can be added to table caption.

|  | Company |
| --- | --- |
| Support | Nokia/NSB, InterDigital, Fujitsu, LG, Qualcomm,CMCC, amsung, ZTE, CATT, CAICT |
| Not support |  |

| Company | Comments |
| --- | --- |
| Nokia/NSB | We are fine with this proposal. |
| OPPO | The template should not be mandated for the tdoc. In order to get some observation, companies may use different tables for easy comparison. One suggestion is that the moderator can prepare an excel file, and all companies inputs their values for the collection of evaluation results. |
| Qualcomm | We are ok with the proposal. |
| HW/HiSi | In principle fine with the proposal. But model structure could be reported in first column. One additional point that we hope we can spend time on during this meeting is to agree on typical cases for evaluating the generalization. This could then be compared across companies [Moderator] OK, we can try to agree on some typical cases. |

## 2nd round discussion

Based on the feedback, Proposal 7.1-1 is acceptable in general. One issue noticed from HW/HiSi contribution is, the template needs to be slightly different depending on if fine-tuning is evaluated or not. Thus the following are proposed.

Proposal 7.2-1

For both direct AI/ML positioning and AI/ML assisted positioning, if fine-tuning is not evaluated, the template agreed in RAN1#110 is updated to the following for reporting the evaluation results.

Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [short model description]

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |

Proposal 7.2-2

For both direct AI/ML positioning and AI/ML assisted positioning, if fine-tuning is evaluated, the template agreed in RAN1#110 is updated to the following for reporting the evaluation results.

Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [short model description]

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Fine-tune | Test | Train | Fine-tune | test | Model complexity | Computation complexity | AI/ML |

|  | Company |
| --- | --- |
| Support | ZTE, OPPO, Samsung, Nokia/NSB, Apple, Fujjitsu, CAICT, CATT, InterDigital,CMCC, HW/HiSi |
| Not support |  |

| Company | Comments |
| --- | --- |
| Qualcomm | There should also be an additional column to show KPI of intermediate info for AI/ML assisted positioning approach (e.g., 90%tile of Timing/Angle estimation error, LOS classification accuracy, KPI of soft information, etc.). For AI/ML assisted positioning, consider an addition column that indicates whether model input considers single or multiple TRP construction. [Moderator] Agree that these info should be reported by companies. But adding columns may make the table too big. Also the same table cannot be used for direct AI/ML approach and AI/ML assisted approach. Different Let’s add a note for these. |

## 3rd round discussion

The templates in the 2nd round discussion are generally acceptable.

As pointed out by QC, for AI/ML assisted positioning, companies are expected to report intermediate KPI and single-TRP/multi-TRP (if agreed).

Thus, it’s recommended that the templates in Proposal 7.2-1 and 7.2-2 can be kept, while the following can be separately proposed.

Proposal 7.3-1

For both direct AI/ML positioning and AI/ML assisted positioning, the description agreed in RAN1#110 is updated to the following for reporting the evaluation results.

To report the following in table caption:

- Which side the model is deployed

- Model generalization investigation, if applied

- Short model description: e.g., CNN

Further info for the columns:

- Model input: input type and size

- Model output: output type and size

- Label: meaning of ground truth label; percentage of training data set without label if data labeling issue is investigated (default = 0%)

- Clutter parameter: e.g., {60%, 6m, 2m}

- Dataset size, both the size of training/validation dataset and the size of test dataset

- AI/ML complexity: both model complexity in terms of “number of model parameters”, and computational complexity in terms of FLOPs

- Horizontal positioning accuracy: the accuracy (in meters) of the AI/ML based method

Note: To report other simulation assumptions, if any.

Note: For AI/ML assisted positioning, additionally report: intermediate performance metrics, [single-TRP or multi-TRP]

## 4th round discussion

The templates for reporting evaluation results have been agreed. On the remaining question of adding a note (suggested by QC), please check if this following is acceptable.

Proposal 7.4-1

For both direct AI/ML positioning and AI/ML assisted positioning, the description agreed in RAN1#110 is updated to the following for reporting the evaluation results.

To report the following in table caption:

- Which side the model is deployed

- Model generalization investigation, if applied

- Short model description: e.g., CNN

Further info for the columns:

- Model input: input type and size

- Model output: output type and size

- Label: meaning of ground truth label; percentage of training data set without label if data labeling issue is investigated (default = 0%)

- Clutter parameter: e.g., {60%, 6m, 2m}

- Dataset size, both the size of training/validation dataset and the size of test dataset

- AI/ML complexity: both model complexity in terms of “number of model parameters”, and computational complexity in terms of FLOPs

- Horizontal positioning accuracy: the accuracy (in meters) of the AI/ML based method

Note: To report other simulation assumptions, if any.

Note: For AI/ML assisted positioning, additionally report: intermediate performance metrics, single-TRP or multi-TRP construction.

|  | Company |
| --- | --- |
| Support | Fujitsu, Apple, LG, Samsung, ZTE, CATT, HW/HiSi, OPPO, Nokia/NSB, Qualcomm, NVIDIA |
| Not support |  |

| Company | Comments |
| --- | --- |

# Proposals for online discussion

## 1st round proposals for online discussion

Void. There was no time to have online discussion on 1st round proposals.

## 2nd and 3rd round proposals for online discussion

Proposal 2.2.1-1

To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning:

(e) InF scenarios, e.g., training dataset from one InF scenario (e.g., InF-DH), test dataset from a different InF scenario (e.g., InF-HH)

Proposal 5.6-1 (overall)

Observation: Direct AI/ML positioning can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods in heavy NLOS scenarios.

Proposal 7.2-1

For both direct AI/ML positioning and AI/ML assisted positioning, if fine-tuning is not evaluated, the template agreed in RAN1#110 is updated to the following for reporting the evaluation results.

Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [short model description]

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |

Proposal 7.2-2

For both direct AI/ML positioning and AI/ML assisted positioning, if fine-tuning is evaluated, the template agreed in RAN1#110 is updated to the following for reporting the evaluation results.

Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [short model description]

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Fine-tune | Test | Train | Fine-tune | test | Model complexity | Computation complexity | AI/ML |

Proposal 2.1.2-1

For AI/ML-assisted positioning, companies report which construction is applied in their evaluation:

- Single-TRP construction: the input of the ML model is the channel measurement between the target UE and a single TRP, and the output of the ML model is for the same pair of UE and TRP.

- Multi-TRP construction: the input of the ML model contains N sets of channel measurements between the target UE and N (N>1) TRPs, and the output of the ML model contains N sets of values, one for each of the N TRPs.

Note: For a measurement (e.g., RSTD) which is a relative value between a given TRP and a reference TRP, the TRP in “single-TRP” and “multi-TRP” refers to the given TRP only.

Conclusion 2.2.2-1

For evaluation of AI/ML based positioning, suspend the discussion on intra-site (or zone-specific) variations until concepts and channel model construction not in TR38.901 (e.g., “intra-site” or “zone”) are clarified under AI 9.2.1.

- Note: An individual company can still submit evaluation results for intra-site variation.

Proposal 2.3.3-1

For evaluation of AI/ML based positioning, it’s up to each company to take into account the channel estimation error in their evaluation. Companies describe the details of their simulation assumption, e.g., realistic or ideal channel estimation, error models, receiver algorithms.

Proposal 2.4.3-1

For evaluation of AI/ML based positioning, the sampling period is selected by proponent companies. Each company report the sampling period used in their evaluation.

Proposal 3.1.3-1

For evaluation of AI/ML assisted positioning, the following intermediate performance metrics are used:

- LoS classification accuracy, if the model output includes LOS/NLOS indicator of hard values, where the LOS/NLOS indicator is generated for a pair of UE and TRP;

- Timing estimation accuracy (expressed in meters), if the model output includes timing estimation (e.g., ToA, RSTD).

- Angle estimation accuracy (in degrees), if the model output includes angle estimation (e.g., AoA, AoD).

- Companies provide info on how LoS classification accuracy and timing/angle estimation accuracy are estimated, if the ML output is a soft value that represents a probability distribution (e.g., probability of LOS, probability of timing, probability of angle, mean and variance of timing/angle, etc.)

Proposal 5.8-1 (generalization over drops)

Observation: When other simulation assumptions are held constant, positioning accuracy of direct AI/ML positioning deteriorates significantly when the ML model is trained with dataset of one drop, and tested with dataset of a different drop (i.e., different set of random seeds for channel propagation).

- Companies have provided initial evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning, for example:

  - The training dataset is composed of data from multiple drops;

  - Model fine-tuning is performed, where the model is re-trained with a small dataset from the same drop as the test dataset.

- This deterioration in positioning performance can be avoided by drop-specific model training which accounts for the drop settings for test.

Proposal 5.8-2 (generalization over clutter parameters)

Observation: Positioning accuracy of direct AI/ML positioning deteriorates when the ML model is trained with dataset of one clutter parameter setting (e.g., InF-DH {60%, 6m, 2m}), and tested with dataset of a different clutter parameter setting (e.g., InF-DH {40%, 2m, 2m}). As the clutter parameter setting of training dataset deviates further from that of training, worse positioning accuracy is observed.

- Companies have provided initial evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning, for example:

  - The training dataset is composed of data from multiple clutter parameter settings, which include clutter parameter setting of the test dataset.

  - Model fine-tuning is performed, where the model is re-trained with a small dataset from the same clutter parameter setting as the test dataset.

- This deterioration in positioning performance can be avoided by clutter-specific model training which accounts for the expected clutter parameter for testing.

## 4th and 5th round proposals for online discussion

Proposal 2.3.5-1

For evaluation of AI/ML based positioning, it’s up to each company to take into account the channel estimation error in their evaluation. Companies describe the details of their simulation assumption, e.g., realistic or ideal channel estimation, error models, receiver algorithms.

Proposal 2.1.5-1

For AI/ML assisted positioning, when single-TRP construction is used for the AI/ML model, companies report the AI/ML complexity (Model complexity, Computation complexity) for N TRPs, which are used to determine the position of a target UE.

Table. Model complexity and computation complexity to support N TRPs for a target UE

|  | Model complexity to support N TRPs | Computation complexity to process N TRPs |
| --- | --- | --- |
| Single-TRP, same model for N TRPs | PS Where PS is the model complexity for the same model. | N×CS Where CS is the computation complexity of the same model for one TRP. |
| Single-TRP, N models for N TRPs | i=1,…NPS,i Where PS,i is the model complexity for the i-th AI/ML model. | i=1,…NCS,i Where CS,i is the computation complexity for the i-th AI/ML model. |
| Multi-TRP (i.e., one model for N TRPs) | PM Where PM is the model complexity for the one model. | CM Where CM is the computation complexity for the one model. |

Proposal 4.5-1

For direct AI/ML positioning, if time-domain channel impulse response (CIR) is used as model input in the evaluation, companies report the CIR dimension NTRP * Nport * Nt, where NTRP is the number of TRPs, Nport is the number of transmit/receive antenna port pairs, Nt is the number of time domain samples.

## 6th round proposals for online discussion

Proposal 2.1.5-1

For AI/ML assisted positioning, when single-TRP construction is used for the AI/ML model, companies report the AI/ML complexity (Model complexity, Computation complexity) for N TRPs, which are used to determine the position of a target UE.

Table. Model complexity and computation complexity to support N TRPs for a target UE

|  | Model complexity to support N TRPs | Computation complexity to process N TRPs |
| --- | --- | --- |
| Single-TRP, same model for N TRPs | PS When the model is at UE-side, where PS is the model complexity for the same model. FFS: if the model is at network-side | N×CS Where CS is the computation complexity of the same model for one TRP. |
| Single-TRP, N models for N TRPs | i=1,…NPS,i Where PS,i is the model complexity for the i-th AI/ML model. | i=1,…NCS,i Where CS,i is the computation complexity for the i-th AI/ML model. |
| Multi-TRP (i.e., one model for N TRPs) | PM Where PM is the model complexity for the one model. | CM Where CM is the computation complexity for the one model. |

Proposal 2.2.4-1

For AI/ML based positioning, if an InF scenario different from InF-DH is evaluated for the model generalization capability, the selected parameters (e.g., clutter parameters) are compliant with TR 38.901 Table 7.2-4 (Evaluation parameters for InF).

- Note: In TR 38.857 Table 6.1-1 (Parameters common to InF scenarios), InF-SH scenario uses the clutter parameter {20%, 2m, 10m} which is compliant with TR 38.901.

Proposal 4.6-4

For the model input used in evalutions of AI/ML based positioning, if time-domain channel impulse response (CIR) or power delay profile (PDP) is used as model input in the evaluation, companies report the input dimension NTRP * Nport * Nt, where NTRP is the number of TRPs, Nport is the number of transmit/receive antenna port pairs, Nt is the number of time domain samples.

- Note: CIR and PDP may have different dimensions.

- Note: Companies provide details on their assumption on how PDP is constructed and how (if applicable) it is mapped to Nt samples.

Proposal 7.4-1

For both direct AI/ML positioning and AI/ML assisted positioning, the description agreed in RAN1#110 is updated to the following for reporting the evaluation results.

To report the following in table caption:

- Which side the model is deployed

- Model generalization investigation, if applied

- Short model description: e.g., CNN

Further info for the columns:

- Model input: input type and size

- Model output: output type and size

- Label: meaning of ground truth label; percentage of training data set without label if data labeling issue is investigated (default = 0%)

- Clutter parameter: e.g., {60%, 6m, 2m}

- Dataset size, both the size of training/validation dataset and the size of test dataset

- AI/ML complexity: both model complexity in terms of “number of model parameters”, and computational complexity in terms of FLOPs

- Horizontal positioning accuracy: the accuracy (in meters) of the AI/ML based method

Note: To report other simulation assumptions, if any.

Note: For AI/ML assisted positioning, additionally report: intermediate performance metrics, single-TRP or multi-TRP construction.

# Agreements made at RAN1#110bis-e

For easy reference, the agreements made at this meeting for AI/ML positioning are copied below.

## AI 9.2.4.1 Agreements

Agreement

To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning:

(e) InF scenarios, e.g., training dataset from one InF scenario (e.g., InF-DH), test dataset from a different InF scenario (e.g., InF-HH)

Agreement

For both direct AI/ML positioning and AI/ML assisted positioning, if fine-tuning is not evaluated, the template agreed in RAN1#110 is updated to the following for reporting the evaluation results.

Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [short model description]

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |

Agreement

For both direct AI/ML positioning and AI/ML assisted positioning, if fine-tuning is evaluated, the template agreed in RAN1#110 is updated to the following for reporting the evaluation results.

Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [short model description]

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Fine-tune | Test | Train | Fine-tune | test | Model complexity | Computation complexity | AI/ML |

Agreement

For AI/ML-assisted positioning, companies report which construction is applied in their evaluation:

- Single-TRP construction: the input of the ML model is the channel measurement between the target UE and a single TRP, and the output of the ML model is for the same pair of UE and TRP.

- Multi-TRP construction: the input of the ML model contains N sets of channel measurements between the target UE and N (N>1) TRPs, and the output of the ML model contains N sets of values, one for each of the N TRPs.

Note: For a measurement (e.g., RSTD) which is a relative value between a given TRP and a reference TRP, the TRP in “single-TRP” and “multi-TRP” refers to the given TRP only.

Note: For single-TRP construction, companies report whether they consider same model for all TRPs or N different models for TRPs

Conclusion

For evaluation of AI/ML based positioning, suspend the discussion on intra-site (or zone-specific) variations until concepts and channel model construction not in TR38.901 (e.g., “intra-site” or “zone”) are clarified under AI 9.2.1.

- Note: An individual company can still submit evaluation results for intra-site variation.

Conclusion

For evaluation of AI/ML based positioning, the sampling period is selected by proponent companies. Each company report the sampling period used in their evaluation.

Agreement

For evaluation of AI/ML assisted positioning, the following intermediate performance metrics are used:

- LOS classification accuracy, if the model output includes LOS/NLOS indicator of hard values, where the LOS/NLOS indicator is generated for a link between UE and TRP;

- Timing estimation accuracy (expressed in meters), if the model output includes timing estimation (e.g., ToA, RSTD).

- Angle estimation accuracy (in degrees), if the model output includes angle estimation (e.g., AoA, AoD).

- Companies provide info on how LOS classification accuracy and timing/angle estimation accuracy are estimated, if the ML output is a soft value that represents a probability distribution (e.g., probability of LOS, probability of timing, probability of angle, mean and variance of timing/angle, etc.)

Conclusion

For evaluation of AI/ML based positioning, it’s up to each company to take into account the channel estimation error in their evaluation. Companies describe the details of their simulation assumption, e.g., realistic or ideal channel estimation, error models, receiver algorithms.

## AI 9.2.4.2 Agreements

Conclusion

- Defer the discussion of prioritization of online/offline training for AI/ML based positioning until more progress on online vs. offline training discussion in agenda 9.2.1.

Agreement

- Study and provide inputs on benefit(s) and potential specification impact at least for the following cases of AI/ML based positioning accuracy enhancement

- Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

- Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

- Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

- Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

Agreement

Regarding AI/ML model indication[/configuration], to study and provide inputs on potential specification impact at least for the following aspects on conditions/criteria of AI/ML model for AI/ML based positioning accuracy enhancement

- Validity conditions, e.g., applicable area/[zone/]scenario/environment and time interval, etc.

- Model capability, e.g., positioning accuracy quality and model inference latency

- Conditions and requirements, e.g., required assistance signalling and/or reference signals configurations, dataset information

- Note: other aspects are not precluded

Agreement

Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on potential specification impact for the following aspects

- Assistance signaling and procedure at least for UE-side model

- Report/feedback and procedure at least for Network-side model

- Note1: study is applicable to both of the following cases

  - Model inference and model monitoring at the same entity

  - Entity to perform the model monitoring is not the same entity for model inference

- Note2: other aspects are not precluded

# Conclusion

TBD

# References

RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” 3GPP RAN#96, June 2022.

R1-2208399	Evaluation of AI/ML for Positioning Accuracy Enhancement	Ericsson

R1-2208433	Evaluation on AI/ML for positioning accuracy enhancement	Huawei, HiSilicon

R1-2208525	Evaluation on AI for positioning enhancement	ZTE

R1-2208638	Evaluation on AI/ML for positioning accuracy enhancement	vivo

R1-2208772	Evaluation on AI/ML for positioning accuracy enhancement	China Telecom

R1-2208854	Evaluation methodology and preliminary results on AI/ML for positioning accuracy enhancement	OPPO

R1-2208882	On Evaluation of AI/ML based Positioning	Google

R1-2208903	Evaluation on AI/ML for positioning accuracy enhancement	LG Electronics

R1-2208971	Evaluation on AI/ML for positioning enhancement	CATT

R1-2209015	Discussions on evaluation of AI positioning accuracy enhancement	Fujitsu

R1-2209124	Discussion on AI/ML Positioning Evaluations	Lenovo

R1-2209234	Some discussions on evaluation on AI-ML for positioning accuracy enhancement	CAICT

R1-2209281	Evaluation on AI/ML for positioning accuracy enhancement	xiaomi

R1-2209332	Discussion on evaluation on AI/ML for positioning accuracy enhancement	CMCC

R1-2209371	Evaluation of ML for positioning accuracy enhancement	Nokia, Nokia Shanghai Bell

R1-2209484	Evaluation on AI/ML for positioning accuracy enhancement	InterDigital, Inc.

R1-2209510	Evaluation on AI/ML for positioning accuracy enhancement	MediaTek Inc.

R1-2209537	Evaluation on AI/ML for positioning accuracy enhancement 	Faunhofer IIS, Fraunhofer HHI

R1-2209580	Evaluation on AI/ML for positioning accuracy enhancement	Apple

R1-2209615	Evaluation of AI/ML based positioning accuracy enhancement	Rakuten Symphony

R1-2209629	Evaluation of AI and ML for positioning enhancement	NVIDIA

R1-2209726	Evaluation on AI ML for Positioning	Samsung

R1-2209980	Evaluation on AI/ML for positioning accuracy enhancement	Qualcomm Incorporated