3GPP TSG RAN WG1 Meeting #110bis-e	                         R1-2210662

e-Meeting, October 10th – 19th, 2022

Source:                Moderator (Lenovo)

Title:                     Feature lead summary #4 on multi-cell PUSCH/PDSCH scheduling with a single DCI

Agenda item:       9.9.1

Document for:     Discussion and Decision

# Introduction

This document summarizes the contributions submitted under the “9.9.1 Multi-cell PUSCH/PDSCH scheduling with a single DCI” agenda item of the Rel-18 work item on “Multi-Carrier Enhancements (MCE) for NR”.

[110bis-e-R18-MC_Enh-01] Email discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI by October 19 – Haipeng (Lenovo)

- Check points: October 14, October 19

The Rel-18 WI Multi-carrier enhancements was agreed during RAN#94-e meeting [1], where one of the objectives is targeted to specify a solution for multi-cell PUSCH/PDSCH scheduling with a single DCI. The detailed objectives in the WID are listed below:

1. Specify a solution for multi-cell PUSCH/PDSCH scheduling (one PDSCH/PUSCH per cell) with a single DCI [RAN1] - Identify the maximum number of cells that can be scheduled simultaneously - Consider both intra-band and inter-band CA operation - Consider both FR1 and FR2 - The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling

In this contribution, we summarize the related issues and proposals based on the contributions submitted in RAN1#110 under the agenda item 9.9.1 [1]-[26]. The whole feature lead summary is structured as follows:

From section 2 to 5, the main issues raised by company contributions are divided into 4 parts and each section covers one main issue. In each section, the background and related proposals submitted in this meeting are listed firstly in sub-section X.1, then summary on one or several sub-issues is provided in sub-section X.2 from moderator’s perspective. Based on the above summary, a set of proposals is recommended by moderator followed by one or multiple tables to collect company views for the initial proposals in the first round of e-mail discussion. If present, in each sub-section, the proposals will be updated round by round based on companies’ inputs. As e-mail discussion goes on, more sub-sections may be provided for further e-mail discussion and update.

In section 6, some proposals are selected for discussion in the online/offline sessions.

In Section 8, the agreements made in previous RAN1/RAN meetings are listed for reference.

Companies are highly encouraged to provide views as soon as possible. Moderator will try to update the proposals based on companies’ inputs at least on daily basis.

# Scenarios and basic framework

## Background and submitted proposals

Regarding this issue, companies’ views are summarized as below:

LG: - Proposal #1: Clarify whether the agreed same SCS restriction for co-scheduled cells is to be applied in cell level or BWP level. - Consider BWP level restriction that the cells with active BWP of same SCS can be co-scheduled by the multi-cell DCI. Nokia: - Proposal 2.1.2: The following is supported in Rel-18: - Case 2-1: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and the same spectrum type and frequency range (licensed or unlicensed, FR1 or FR2-1 or FR2-2) is used among all the co-scheduled cells including the scheduling cell. - This includes the support of a mix of FDD and TDD cells. - Case 2-2: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and the same spectrum type and frequency range (licensed or unlicensed, FR1 or FR2-1 or FR2-2) is used among all the co-scheduled cells which may be same or different carrier type to the scheduling cell. - This includes the support of a mix of FDD and TDD cells. - Using an unlicensed cell for scheduling a set of co-scheduled licensed cells is not supported - Proposal 2.1.3: Highest priority scenario should be intra-band CA operation for multi-cell scheduling using DCI formats 0_X / 1_X with lower priority for over inter-band CA operation.

## Moderator summary and proposals based on contributions

Based on conclusion from RAN#97, from moderator’s perspective, there is no open issue on supported scenarios and baseline operation.

Conclusion: - Followings are excluded from multi-cell PDSCH/PUSCH scheduling in Rel-18. - SCell schedules multiple cells including P(S)Cell - Different SCS among co-scheduled cells - Different carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2) among co-scheduled cells - Configuration of both multi-cell PDSCH/PUSCH scheduling and multi-TRP for a scheduled cell - Support for any sidelink scheduling

Regarding the issue raised by LG whether the agreed same SCS restriction for co-scheduled cells is applied in cell level or BWP level, moderator understands this is BWP level restriction as only active BWP per cell is scheduled for data transmission.

Regarding the issue raised by Nokia whether to add the restriction of same spectrum type and frequency range to Case 2-1 and Case 2-2, moderator understands RAN guidance has precluded the case where mixed licensed or unlicensed carriers are co-scheduled. Since there is no issue to support mixed TDD and FDD for co-scheduling and RAN conclusion keeps the possibility, moderator thinks no need to further preclude mixed TDD and FDD in RAN1.

Hence, no proposal is provided in this section for time being.

# DCI format design

Based on contributions submitted by companies, below issues are prioritized for discussion in this meeting. Within each sub-section, the summary from moderator’s perspective is listed and followed by draft proposals for further discussion round by round.

## Maximum number of cells scheduled by a single DCI

Regarding this issue, companies’ views are summarized as below:

Huawei, HiSilicon - Proposal 1: Confirm the work assumption that the maximum number of co-scheduled cells by a DCI format 1_X/0_X in Rel-18 is 4. - Proposal 2: The maximum number of configurable cells for co-scheduling should be same as the maximum number of co-scheduled cells by DCI format 0_X/1_X, i.e. 4. ZTE - Proposal 1: The maximum number of configurable cells for co-scheduling is equal to the maximum number of co-scheduled cells by a single MC-DCI. Spreadtrum Communications - Proposal 1: Confirm the working assumption, and maximum number of configurable cells for co-scheduling is 4. - The maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4. - The maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4. Xiaomi: - Proposal 2: Confirm the working assumption that the maximum number of co-scheduled cells which can be scheduled by a single DCI is 4. - Proposal 3: The number of the scheduled cells by a single DCI can be semi-statically configured with a value less than or equal to the maximum number of co-scheduled cells Lenovo: - Proposal 14: Confirm the working assumption that the maximum number of co-scheduled cells by a DCI format 0_X/1_X in Rel-18 is 4. - Proposal 15: For a UE, the maximum number of cells which can be co-scheduled by the DCI is configured by RRC signaling from a set of possible values of 2, 3 and 4. CMCC: - Proposal 3. The maximum number of configurable cells scheduled by a DCI format 0_X/1_X can be more than 4. Samsung: - Proposal 2: Confirm the WA from RAN1#110. The FFS can be left for UE capability discussions. Ericsson: - Proposal 1: Maximum number of configurable cells for co-scheduling by a DCI format 1_X/0_X is 4. NTT DOCOMO: - Proposal 2: Confirm following working assumption - The maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4. - The maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4. Nokia, Nokia Shanghai Bell - Proposal 3.1.1: RAN1 to confirm the RAN1#110 working assumption on the maximum number of co-scheduled cells, i.e. Working Assumption - The maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4. - The maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4.. - Proposal 3.1.2: To limit the DCI size, the maximum number of co-scheduled cells by a DCI format 0_X / 1_X should be based on RRC configuration (i.e. from the set of {2,3,4}). - Proposal 3.1.3: The maximum number of configurable cells for co-scheduling using DCI formats 0_X / 1_X is determined by the number of cells within a PUCCH group but not limited otherwise (i.e. >4 configurable cells are supported).

### Moderator summary and proposals based on contributions

Regarding the maximum number of schedulable cells by a single DCI, working assumption is made in RAN1#110 meeting which is also listed below.

Working Assumption - The maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4. - The maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4. - FFS: The maximum number of configurable cells for co-scheduling

Obviously, the payload size is rapidly increased with the number of schedulable cells. Considering the maximum 140 bits excluding CRC for DCI payload size due to the limitation on of Polar code interleaver, too many cells schedulable by a single DCI will lead to scheduling restriction. The determined maximum number of schedulable cells should consider both the limitation of 140bits for Polar coding and scheduling flexibility as well as the probability of scheduling a large number of carriers. So from this perspective, a reasonable absolute maximum of schedulable cells should be 4.

Regarding maximum number of schedulable carriers by a single DCI, 7 companies [Huawei, Spreadtrum, Xiaomi, Lenovo, Samsung, NTT DOCOMO, Nokia] propose to confirm the working assumption of maximum 4 co-scheduled cells by a single DCI format 0_X/1_X and no company propose max 8 cells scheduled by a single DCI. Hence, moderator suggests confirming the working assumption in the first round of discussions.

Regarding the FFS of the maximum number of configurable cells for co-scheduling, moderator suggests discussing this issue with the indication of co-scheduled cells and moves it to Section 4.

### (Closed)1st round of discussions

#### Proposal 2-1:

- Confirm below working assumption reached in RAN1#110 meeting.

Working Assumption

- The maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4.

- The maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| MTK | Support |
| Qualcomm | We are OK with the proposal. |
| Nokia/NSB | Support |
| Apple | Support |
| Langbo | Support |
| Huawei, HiSilicon | Support |
| Intel | We are fine with the proposal. However, based on the discussion in the last meeting, we also need to determine the maximum number of configured cells for multi-cell scheduling. |
| ZTE | Support to confirm the working assumption. |
| Moderator | @Intel: I plan to discuss the maximum number of configured cells for multi-cell scheduling in section 4.2. |
| CMCC | We are fine with the proposal. |
| New H3C | Support |
| Fujitsu | Support. |
| CATT | Support |
| vivo | Support |
| LGE | Support |
| Samsung | OK with the proposal |
| Spreadtrum | Support |
| Xiaomi | Support |
| NTT DOCOMO | Support. |
| Ericsson1 | OK to confirm the WA. On FL comment “Regarding the FFS of the maximum number of configurable cells for co-scheduling, moderator suggests discussing this issue with the indication of co-scheduled cells and moves it to Section 4” – the “maximum number of configurable cells for co-scheduling” impacts several important aspects including DCI format size (UE determines this based on RRC configuration or is it based on set of ‘co-scheduled’ cells) , how DCI field sizes are determined and BD complexity. So, it is good to clarify this in general instead of focussing just the ‘indication of co-scheduled cells’. |
| China Telecom | Support |
| ITRI | Support |
| Moderator2 | @Ericsson: Thanks for the good comments. Agree with you. We can discuss this issue together with DCI size and co-scheduled cell indicator. |
| Moderator3 | Since we have confirmed the WA, now this issue is concluded. Proposal 2-1: - Confirm below working assumption reached in RAN1#110 meeting. Working Assumption - The maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4. - The maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4. - FFS: The maximum number of configurable cells for co-scheduling |

## Scheduling possibilities

Huawei: - Proposal 3: Confirm the WA that multi-cell scheduling DCI and legacy DCI can be monitored simultaneously from the same scheduling cell and the following should also be supported: - Monitoring of the DCI format 0_X/1_X and the legacy DCI format(s) is supported for all scheduled cells within the set of co-scheduled cells. - At least fallback DCI, DCI 0_1/1_1 and DCI 0_2/1_2 should be supported to be monitored with DCI 0_X/1_X. - Proposal 4: Monitoring the DCI format 0_X/1_X from one scheduling cell and legacy DCI format(s) from the scheduled cell via self-scheduling should be supported. ZTE: - Proposal 11: Confirm the working assumption with the FFS deleted. - For a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X and legacy single cell scheduling DCI format(s) from a same scheduling cell. - The DCI format 0_X/1_X and the legacy DCI format(s) can be monitored simultaneously. Spreadtrum Communications - Proposal 8: The search space configured with DCI format 0_X/1_X cannot be configured with legacy DCI formats. Vivo - Proposal 1. For a scheduled cell, the case where different scheduling cells are configured for multi-cell scheduling and single-cell scheduling is not supported. China Telecom: - Proposal 2: For a cell within a set of configured cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring DCI format 0_X/1_X and legacy DCI format(s) from a same scheduling cell simultaneously: - At least when the cell is the same as scheduling cell - when the cell is not the same as scheduling cell, it depends on gNB configuration of legacy cross-carrier scheduling to support. - Proposal 3: For a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, not support monitoring the DCI format 0_X/1_X and legacy single cell scheduling DCI format(s) from different scheduling cells. CATT: - Proposal 1: Each cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X should support monitoring the DCI format 0_X/1_X and legacy DCI format simultaneously. - Proposal 2: The scheduling cell of legacy DCI format for each cell scheduled by DCI format 0_X/1_X should be the same scheduling cell for DCI format 0_X/1_X. - Proposal 4: If the UE has the capability of multi-cell scheduling, all legacy DCI format can be monitored with DCI format 0_X/1_X. Langbo： - Proposal 1: Confirm the working assumption that monitoring the DCI format 0_X/1_X and legacy single cell scheduling DCI format(s) from a same scheduling cell is supported for a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X. - Proposal 2: When DCI format 0_X/1_X and a legacy DCI format are configured to be monitored simultaneously, monitoring of the DCI format 0_X/1_X and the legacy DCI format is supported for each cell that can be co-scheduled. - Proposal 3: For a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, monitoring the DCI format 0_X/1_X from one scheduling cell and legacy DCI format(s) from another scheduling cell is not supported. Intel: - Proposal 2: For each cell within a set of configured cells which can be scheduled by a DCI format 0_X/1_X, - Support monitoring DCI format 0_X/1_X and legacy DCI format(s) simultaneously from a same scheduling cell. - Support monitoring DCI format 0_X/1_X and legacy DCI format(s) simultaneously from different scheduling cells. Lenovo: - Proposal 20: Confirm the working assumption that for a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring DCI format 0_X/1_X and legacy DCI format(s) from a same scheduling cell. - Proposal 21: Support monitoring DCI format 0_X/1_X and legacy DCI format(s) for all cells within a set of cells which can be co-scheduled by the DCI format 0_X/1_X. - Proposal 22: It is up to gNB configuration to support monitoring a subset or all legacy DCI format(s) with DCI format 0_X/1_X for a cell within a set of cells which can be co-scheduled by the DCI format 0_X/1_X. CMCC: - Proposal 1. For a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X and legacy single cell scheduling DCI format(s) from a same scheduling cell. - The DCI format 0_X/1_X and the legacy DCI format(s) can be monitored simultaneously for each cell within the co-scheduled cells. - All legacy DCI format(s) can be monitored with DCI 0_X/1_X. - Proposal 2. For single cell scheduling, support the switching between monitoring DCI format 0_X/1_X and legacy DCI format for a scheduled cell. FGI: - Proposal 2: For each scheduled cell, a UE monitors DCI format 0_X/1_X on at most one scheduling cell. - Proposal 3: Confirm the working assumption that ”For a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X and legacy single cell scheduling DCI format(s) from a same scheduling cell.” - Proposal 4: For a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X from one scheduling cell and legacy DCI format(s) from the scheduled cell via self-scheduling. Apple: - Proposal 1: For each scheduled cell, a UE is configured to monitor both DCI format 0_X/1_X and legacy DCI format(s) on at most one/same scheduling cell - It is not supported to monitor legacy DCI format(s) on an additional scheduling cell - Proposal 2: For multi-cell scheduling DCI format 0_X/1_X, do not support different DCI sizes corresponding to different number of scheduled cells Samsung: - Proposal 1: Do not further consider introduction of multiple scheduling cells for a scheduled cell. - Proposal 8: It is up to gNB whether to configure a UE to monitor PDCCHs for one or both SC-DCI formats and MC-DCI formats for any cell from a configured set of scheduled cells. - Proposal 9: A UE can be configured to monitor PDCCH for MC-DCI formats and SC-DCI formats, or to monitor PDCCH for downlink MC-DCI format (1_X) and uplink MC-DCI format (0_X), in different search space sets. - Proposal 10: When an MC-DCI format is used for scheduling a single cell, the UE interprets the MC-DCI format based on the same fields as for a SC-DCI format (e.g., DCI format 0_1/1_1). - Fields corresponding to fully disabled functionalities (e.g., CBG or multi-TRP operation) are reserved. Ericsson: - Proposal 2: When mc-DCI is configured for scheduling PUSCH/PDSCH on multiple cells, a mc-DCI can schedule PUSCH/PDSCH on all of the cells or a subset of those cells. - Proposal 3: When mc-DCI is configured for scheduling PUSCH/PDSCH on multiple cells, for each of those cells, UE can also be configured to monitor existing single cell DCI format(s) scheduling PUSCH/PDSCH (i.e. 1_1/1_2/0_1/0_2). NTT DOCOMO: - Proposal 7: Self-carrier scheduling by legacy DCI format(s) for each co-scheduled cell configured for multi-cell scheduling via DCI format 0_X/1_X should be supported. Qualcomm: - Proposal 5: Support dynamic indication of scheduling cell(s) - Enable configuration of more than one scheduling cells for a scheduled cell - Enable switch/fallback from multi-cell scheduling to legacy self-scheduling dynamically - Extend SSSG switching or BWP switching to enable this Nokia, Nokia Shanghai Bell - Proposal 3.2.1: For a scheduled cell, a UE monitors PDCCH for DCI format 0_X/1_X always on one scheduling cell. - Proposal 3.2.2: Confirm the following RAN1#110 working assumption on the support of simultaneous monitoring for DCI format 0_X/1X and legacy DCI format(s) from a same scheduling cell. Working Assumption For a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X and legacy single cell scheduling DCI format(s) from a same scheduling cell. - The DCI format 0_X/1_X and the legacy DCI format(s) can be monitored simultaneously. - FFS: whether monitoring of the DCI format 0_X/1_X and the legacy DCI format(s) is supported for one, a subset, or all cells within the set of cells. - FFS: number of different DCI sizes for 0_X/1_X and for legacy DCI formats - FFS: whether to support a subset or all legacy DCI format(s) to be monitored with DCI 0_X/1_X. - Proposal 3.2.4: For a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X and legacy single cell scheduling DCI format(s) from different scheduling cells for the case of legacy DCI format(s) self-scheduling. - The DCI format 0_X/1_X and the legacy DCI format(s) can be monitored simultaneously. - Proposal 3.3.1: Adopt RAN1#109-e Proposal 2-9, i.e. Single-stage DCI format is supported for multi-cell PDSCH or PUSCH scheduling. - Proposal 3.3.2: The maximum DCI size for DCI formats 0_X / 1_X is given by the maximum supported payload size of a single polar codeblock of 140 bits (excl. CRC) - Proposal 3.3.3: Support the monitoring for more than one multi-cell DCI 0_X / 1_X (at least on different scheduling cells) within a PUCCH group, where each of the DCI formats 0_X / 1_X can schedule a different (non-overlapping) subgroup of cells within a PUCCH group.

### Moderator summary and proposals based on contributions

- On at most one scheduling cell for monitoring DCI format 0_X/1_X

Regarding whether to support more than one scheduling cell for monitoring DCI format 0_X/1_X for each scheduled cell, RAN1#110 has extensively discussed this issue and achieved below version in FL summary#6.

#### Proposal 2-3rev1: - For each scheduled cell, a UE monitors PDCCH for DCI format 0_X/1_X always on one scheduling cell per configuration. - FFS: whether a UE monitors PDCCH for DCI format 0_X/1_X on one scheduling cell and the scheduling cell can be changed dynamically

Furthermore, this issue is discussed in RAN#97 with below conclusion/guidance:

Conclusion (RAN#97) Configuring more than one scheduling cell for DCI format 0_X/1_X for each scheduled cell is not supported for the multi-cell PUSCH/PDSCH scheduling in Rel-18.

Based on the RAN guidance, it has excluded the possibility of configuring more than one scheduling cell for DCI format 0_X/1_X for each scheduled cell and dynamic switching of scheduling cell. Moderator suggests concluding Proposal 2-3rev1 in previous meeting according to RAN guidance. Hence, Proposal 2-2 is provided.

- On simultaneous monitoring DCI format 0_X/1_X and legacy DCI formats from a same scheduling cell

Regarding PDCCH monitoring for multi-cell scheduling and single cell scheduling, RAN1#110 reaches below working assumption.

Working Assumption For a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X and legacy single cell scheduling DCI format(s) from a same scheduling cell. - The DCI format 0_X/1_X and the legacy DCI format(s) can be monitored simultaneously. - FFS: whether monitoring of the DCI format 0_X/1_X and the legacy DCI format(s) is supported for one, a subset, or all cells within the set of cells. - FFS: number of different DCI sizes for 0_X/1_X and for legacy DCI formats - FFS: whether to support a subset or all legacy DCI format(s) to be monitored with DCI 0_X/1_X

In RAN1#110, new DCI format introduced for multi-cell scheduling has been agreed. The multi-cell scheduling DCI can be used for scheduling a single cell or multiple cells so that it gives the full scheduling flexibility to gNB. Since the multi-cell scheduling is used in CA case for increasing data rate, for a scheduled cell within a set of cells which can be co-scheduled by DCI 0-X/1-X from a scheduling cell, in case of small data packet or retransmission, there is one possibility that gNB needs to only schedule a single cell, e.g., a single scheduled cell. Using legacy DCI for single-cell scheduling can greatly save CCE resources and obtain wide coverage, which is more efficient than using DCI format 0-X/1-X to do it although using DCI format 0-X/1-X can achieve single cell scheduling. On the other hand, UE may need to monitor fallback DCI from the scheduling cell. In that sense, simultaneously monitoring DCI 0-X/1-X and legacy DCI format from a same scheduling cell for a scheduled cell is needed.

Regarding the working assumption of supporting simultaneously monitoring DCI format 0_X/1_X and legacy single cell scheduling DCI format(s) from a same scheduling cell, 11 companies [Huawei, ZTE, China Telecom, CATT, Langbo, Intel, Lenovo, CMCC, FGI, Ericsson, Nokia] support confirming it in this meeting. So, Proposal 2-3 is provided.

In the working assumption, there are three FFS: Regarding the first FFS, the issue is whether monitoring of the DCI format 0_X/1_X and the legacy DCI format(s) is supported for one, a subset, or all cells within the set of cells. Considering each cell of the set of cells may be scheduled for retransmission via a single cell scheduling manner and HARQ retransmission is performed per cell, it makes sense to support monitoring both DCI format 0_X/1_X and the legacy DCI format(s) for all cells within the set of cells. Six companies [Huawei, CATT, Langbo, Lenovo, CMCC, Ericsson] express views to support monitoring both DCI format 0_X/1_X and the legacy DCI format(s) for all cells within the set of cells.

Regarding the 2nd FFS, the number of different DCI sizes for DCI format 0_X/1_X and for legacy DCI formats, this issue is discussed in BD/CCE counting and DCI size budget for multi-cell scheduling DCI.

Regarding the 3rd FFS, for the scheduled cell, whether to monitor a subset or all legacy DCI format(s) with DCI format 0_X/1_X should be configurable. E.g., it is feasible to configure monitoring both DCI format 0_0/1_0 and DCI format 0_X/1_X for a scheduled cell, i.e., using DCI format 0_X/1_X for multi-cell scheduling and DCI format 0_0/1_0 for fallback operation or single cell scheduling. Alternatively, it is also possible to configure monitoring both DCI format 0_1/1_1 and DCI format 0_X/1_X for a scheduled cell, i.e., using DCI format 0_X/1_X for multi-cell scheduling and DCI format 0_1/1_1 for single cell scheduling. So, it makes sense to support all legacy DCI formats dependent on gNB configuration. One company [Huawei] support at least DCI 0_1/1_1 and DCI 0_2/1_2 should be supported to be monitored with DCI 0_X/1_X. Four companies [CATT, Lenovo, CMCC, Ericsson] support monitoring all legacy single cell DCI formats with DCI format 0_X/1_X.

Hence, Proposal 2-4 is provided to address the two FFS issues in the working assumption.

- On simultaneous monitoring DCI format 0_X/1_X and legacy DCI formats from different scheduling cells

It is observed by some companies that if multi-cell DCI scheduling and single-cell DCI scheduling are restricted only from a same scheduling cell for each co-scheduled cell, it may lead to high DL control load on the scheduling cell even PDCCH scarcity. Therefore, for a UE, monitoring DCI format 0_X/1_X and legacy DCI format(s) from different scheduling cells for a scheduled cell within a set of configured cells which can be co-scheduled by a DCI format 0_X/1_X may be useful for PDCCH load balancing.

Whether to support monitoring DCI format 0_X/1_X and legacy DCI format(s) from different cells, companies’ views are summarized as below:

- Support monitoring DCI format 0_X/1_X and legacy DCI format(s) from different cells

  - Yes: Huawei, Intel, FGI, NTT DOCOMO, Nokia

  - No: China Telecom, CATT, Langbo, Apple, Samsung, LGE.

From moderator’s point of view, supporting monitoring DCI format 0_X/1_X and legacy DCI format(s) from different cells seems optimization and not essential to complete this multi-cell scheduling in Rel-18. Considering only 1 TU is arranged for Q4 2022 and possible standardization effort, moderator suggest deferring this issue.

### 1st round of discussions

#### Proposal 2-2:

- For a scheduled cell, a UE monitors PDCCH for DCI format 0_X/1_X always on one scheduling cell.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| MTK | Support |
| Qualcomm | No need to agree this. We think this has already been concluded in RAN#97. |
| Nokia, NSB | Support, but agree with QC. |
| Apple | Support but agree that this is already a conclusion in RAN#97-e |
| Langbo | Support, agree with QC. |
| Intel | Agree with QC. |
| ZTE | This aligns with the guidance agreed in the RAN1#97-e. We support this proposal and also agree with QC. |
| Moderator | Yes, this issue has been concluded in RAN#97. It is more helpful to understand this issue according to this proposal. If majority companies think this proposal is not necessary, we can just leave it here for information. |
| CMCC | Support |
| New H3C | Support |
| Fujitsu | Agree with QC. If we need to agree this, maybe other conclusions made in RAN#97 need to be agreed in this meeting as well. |
| CATT | Support |
| vivo | agree with QC. |
| LGE | Support |
| Samsung | No need for this proposal as mentioned by QC. Also, the wording “monitors” and “one” are open to interpretation. Prefer the wording from RAN#97-e. Conclusion (RAN#97) Configuring more than one scheduling cell for DCI format 0_X/1_X for each scheduled cell is not supported for the multi-cell PUSCH/PDSCH scheduling in Rel-18. |
| Spreadtrum | Support, also fine for information |
| Xiaomi | Agree with QC |
| NTT DOCOMO | We have a same understanding as QC. |
| Ericsson1 | We do not see need for agreement on this. The principle that there is one scheduling per scheduled cell is generally applicable (given the RAN#97 down-scoping) and not restricted to just DCI format 0_X/1_X. |
| China Telecom | Support, agree with QC |
| ITRI | Agree with QC |
| Moderator2 | OK, the proposal is deleted. |

#### Proposal 2-3:

- Confirm below working assumption reached in RAN1#110 meeting.

For a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X and legacy single cell scheduling DCI format(s) from a same scheduling cell.

- The DCI format 0_X/1_X and the legacy DCI format(s) can be monitored simultaneously.

  - FFS: whether monitoring of the DCI format 0_X/1_X and the legacy DCI format(s) is supported for one, a subset, or all cells within the set of cells.

- FFS: number of different DCI sizes for 0_X/1_X and for legacy DCI formats

- FFS: whether to support a subset or all legacy DCI format(s) to be monitored with DCI 0_X/1_X

#### Proposal 2-4:

- For a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X and all legacy single cell scheduling DCI format(s) from a same scheduling cell for all cells within the set of cells.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| MTK | We want to first clarify, for Proposal 2-3/2-4, is it correct that if PCell uses MC DCI to schedule {Pcell, Scell1, Scell2, Scell3}, then only Pcell would also monitor the legacy single cell scheduling DCI format(s)? (with the “…from a same scheduling cell” wording) If this is correct understanding, then we can support Proposal 2-3/2-4. At the same time, our preference for baseline is not to support MC-DCI and legacy cross-carrier scheduling at the same time |
| Qualcomm | On Proposal 2-3: We are not OK with confirming the working assumption. As we have discussed multiple times, this issue is highly correlated with PDCCH BD/CCE budget and DCI-size budget issues. Before confirming the working assumption, we should look at overall pictures. This is why we made it as a working assumption. On Proposal 2-4: We do not understand the difference from the 1st FFS of Proposal 2-3. Related to the 1st FFS and Proposal 2-4, following should be clarified: Whether/how to maintain BD/CCE budget per scheduled CC? Suppose a UE monitors DCI 0_X/1_X on CC 0 for CCs {1, 2, 3, 4}. Suppose a UE also monitors legacy DCI formats on CC 0 for each of CCs {1, 2, 3, 4}. Suppose the scheduling CC uses SCS 30kHz, in which case per-CC BD budget is 36 per slot. Take a look at PDCCH for the scheduled CC 1, the UE monitors the DCI 0_X/1_X and the legacy DCI formats. - Whether the total number of BDs for the DCI 0_X/1_X for CCs {1, 2, 3, 4} and the legacy DCI formats for CC 1 is capped by 36 or not? Our answer is yes, at least for basic MC-scheduling. Same for CCE budget. Although we are open to discuss advanced case, we would like to clarify the basic case first. We are not sure how Proposal 2-4 can work with per-scheduled-CC BD/CCE budget. Our view can be summarized as follows. - Association between nCI value and scheduled-CC-set should be first agreed. - For a basic MC-scheduling, BD/CCE budget per scheduled CC is kept unchanged. - There are following options on BD/CCE budget maintenance: - Opt.1: Count BD/CCE for DCI 0_X/1_X for each of the CCs in the scheduled-CC-set - The UE can be configured to monitor DCI 0_X/1_X for a scheduled-CC-set and legacy DCI formats for each CC in the scheduled-CC-set - Opt.2: The UE monitors DCI 0_X/1_X for a scheduled-CC-set and legacy DCI formats for one of the CCs in the scheduled-CC-set |
| Nokia, NSB | Support both, proposal 2-3 and proposal 2-4 |
| Apple | Support both proposals 2-3 and 2-4 |
| Langbo | Support proposal 2-3 For Proposal 2-4, what does it mean by “all legacy single cell scheduling DCI format(s)”? Two understandings from our perspective: 1. a legacy single cell DCI can be configured together with DCI format 0_X/1_X and the legacy DCI could be any existing single cell scheduling DCI; 2. multiple legacy DCI formats could be configured together with DCI format 0_X/1_X (not exceeding DCI size budget). And also, it is not clear for us which DCI format is considered as “legacy single cell scheduling DCI format(s)”, e.g, whether the DCI formats for multicast are considered as legacy single cell scheduling DCI formats. It would be clearer and more precise to list the supported legacy DCI formats. |
| Huawei, HiSilicon | Appreciate Moderator efforts and generally support the above proposals. However we have two comments related: - Although generally we support the current proposed intention in legacy case, we have different preference than MTK in terms of the case a Scell can also use MC-DCI and self-scheduling together. - We unfortunately have also different view on moderator consideration of: supporting monitoring DCI format 0_X/1_X and legacy DCI format(s) from different cells seems optimization and not essential to complete this multi-cell scheduling in Rel-18. From specification work perspective, we consider this is related to how BD/CCE is distributed, which anyway will need to be discussed. There could be no extra work for supporting this once we know that the total budget is kept regardless of where the scheduling DCIs being from, which is similar to the R17 Scell scheduling Pcell where DCIs can be monitored from different cells simultaneously. From use case perspective, we see need to have this supported functionally so that network can implement this feature with ensured flexibility. |
| ZTE | We support Proposal 2-3. For the Proposal 2-4, we slightly prefer the UE can monitor DCI format 0_X/1_X and all legacy single cell scheduling DCI formats from the different scheduling cells. This can balance the PDCCH load across the serving cells. If companies do want to introduce additional spec efforts on BD/CCE splitting, we think at least it can be supported for the cell which the BC/CCE of the DCI 0_X/1_X is not counted on since there is no such issue on these scheduled cells. For example, the DCI format 0_X/1_X transmitted on Cell 0 can schedule Cell 1-4. If the BD/CCE of DCI format 0_X/1_X is counted on the Cell 1. Then Cell 2, 3, and 4 can support self-cell scheduling by using legacy DCI formats. It is noted there are only legacy DCI formats counted in Cell 2, 3, and 4. |
| Moderator | @MTK: Yes, your understanding is correct. The main intention is DCI format 0_X/1_X and legacy DCI formats are always from a single scheduling cell. Regarding your second comment, I think simultaneous monitoring both DCI format 0_X/1_X and legacy DCI format can be supported as long as DCI size budget and BD/CCE budget are maintained for each cell as legacy. @Qualcomm: For your first comment, I am Ok to defer the WA after we conclude the DCI size budget and BD/CCE budget issues. For your second comment, my understanding on the first FFS is whether one, subset or all cells within the set of cells can be configured for monitoring both DCI format 0_X/1_X and legacy DCI formats from the same scheduling cell. E.g., Pcell uses DCI 0_X/1_X to co-schedule {Pcell, Scell 1, Scell 2, Scell 3}, it makes sense to support monitoring both DCI 0_X/1_X and legacy DCI formats from Pcell for each of {Pcell, Scell 1, Scell 2, Scell 3}. This is intention of Proposal 2-4. Regarding your other comments, I think it is relevant to BD/CCE and DCI size so that we can discuss the issue under Proposal 2-7 and 2-8. @Langbo: I think it is no need to limit a certain legacy DCI format in the proposal as long as the DCI size budget and BD/CCE budget can be maintained as legacy. Regarding the multicast DCI, both DCI format 4_0 and 4_1 are size aligned with DCI format 0_0/1_0, and DCI format 4_2 is configured for payload size. So seems no need to consider the size alignment issues for multicast DCI. @Huawei @ZTE: As you know, supporting monitoring DCI format 0_X/1_X and legacy DCI format(s) from different cells has significant impact on existing CA scheduling framework. RAN#97 has excluded two cases: (1) Pcell schedules multiple cells by DCI format 0_X/1_X when a sSCell is configured to schedule Pcell; (2) Scell schedules multiple cells including P(S)Cell. My understanding is there is only one scheduling cell for each scheduled cell in Rel-18 MC scheduling. So I tend to defer this issue. |
| CMCC | For Proposal 2-3, we support the main bullet. For Proposal 2-4, we are fine with the proposal, the configuration of DCI formats depends on the network, and there is no need to restrict monitoring only some specific DCI formats in the specifications. |
| New H3C | Support proposal 2-3 and proposal 2-4 |
| Fujitsu | We prefer not to simultaneously support MC-DCI and legacy CCS. As summarized by FL. RAN1 has already agreed that DCI format 0_X/1_X can support single-cell scheduling so that it gives the scheduling flexibility to gNB. DCI format 0_X/1_X capable of single-cell scheduling can achieve same scheduling target as legacy CCS. Meanwhile, the proposals are not quite clear to us. - RAN1 has agreed that DCI format 0_X/1_X can support single cell scheduling. When we say “a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X”, do we mean a cell that can be scheduled by a DCI format 0_X/1_X regardless of whether it can be scheduled only by single cells scheduling via DCI format 0_X/1_X? - For proposal 2-4, does it mean if support legacy DCI formats for one scheduled cell, the same legacy DCI format is also supported for all the other scheduled cells? |
| CATT | Support proposal 2-3 and proposal 2-4 |
| vivo | Support proposal 2-3. Proposal 2-4: the proposal implies that all the co-scheduled cell should support all legacy single cell scheduling DCI format(s), however, this is not possible. For example, Pcell uses mc-DCI to schedule {Pcell and Scell#1}, then for Scell#1, legacy fallback DCI 0-0/1-0 cannot be supported as there is only 1 scheduling cell(i.e. Pcell). Legacy fallback DCI 0-0/1-0 can be used for Pcell self-scheduling only in this case. We think the proposal can be revised as below, as it is up to NW configuration to decide which DCI format to use. #### Proposal 2-4: - For a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X and all legacy single cell scheduling DCI format(s) from a same scheduling cell for all cells within the set of cells it is up to Network to configure legacy DCI format(s) for single cell scheduling that is from the same scheduling cell as the scheduling cell of the DCI format 0_X/1_X |
| LGE | In our understanding, Proposal 2-4 is revised version of previous WA to resolve the 1st and 3rd FFS in Proposal 2-3. So we suggest to modify the wording in Proposal 2-3 and confirm the WA, rather than keeping both Proposal 2-3 and Proposal 2-4, as follows - Confirm below working assumption reached in RAN1#110 meeting. For a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X and all of the configured legacy single cell scheduling DCI format(s) from a same scheduling cell. - The DCI format 0_X/1_X and all of the legacy DCI format(s) configured by gNB can be monitored simultaneously. - FFS: whether monitoring of the DCI format 0_X/1_X and the legacy DCI format(s) is supported for one, a subset, or all cells within the set of cells. - FFS: number of different DCI sizes for 0_X/1_X and for legacy DCI formats - FFS: whether to support a subset or all legacy DCI format(s) to be monitored with DCI 0_X/1_X |
| Samsung | OK with Proposal 2-3 to confirm the WA, but due to limited WI time, it is preferable to directly agree to the modified WA per Proposal 2-4 so that the FFS points are resolved. A minor editorial update is suggested below for brevity/clarity. #### Proposal 2-4: Confirm WA from RAN1#110 as follows: For a all cells within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X and all legacy single cell scheduling DCI format(s) from a same scheduling cell for all cells within the set of cells. |
| Xiaomi | For proposal 2-3, we don’t see the need of the FFS parts. Our understanding of the proposal is to support only one scheduling cell for each scheduled cell which is in line with the legacy design. We don’t know why the first FFS is needed. For proposal 2-4, support. |
| NTT DOCOMO | Proposal 2-3: We support this proposal in general. We agree with HW/ZTE to support self-carrier scheduling by legacy DCI for each scheduled cell of DCI format 0_X/1_X at the same time since it is beneficial in terms of PDCCH load balancing. However, we are fine to discuss it later. Proposal 2-4: We support that monitoring of the DCI format 0_X/1_X and legacy single cell scheduling DCI format(s) from a same scheduling cellis supported for all cells within the set of cells. Regarding which legacy DCI format(s) can be monitored with DCI format 0_X/1_X, it should be discussed further to consider the possibility that subset of legacy DCI formats can be monitored simultaneously instead of all DCI formats. |
| Ericsson1 | OK with P2-3, For 2-4, we are OK with limiting to 4 non-fallback DCIs per scheduled cell (We do not see need for configuring all six DCI formats simultaneously 1_1/1_2/1_X and 0_1/0_2/0_X). |
| Huawei2 | Thanks for FL clarification. The below conclusion about one scheduling cell is only about the cell with DCI 1_X and 0_X, instead of total one. This has been clarified in RAN plenary discussion when it was made. We understand the progress concern. Let’s see whether this need extra spec work after BD/CCE discussion. Conclusion (RAN#97) Configuring more than one scheduling cell for DCI format 0_X/1_X for each scheduled cell is not supported for the multi-cell PUSCH/PDSCH scheduling in Rel-18. |
| China Telecom | On Proposal 2-3: Support. On Proposal 2-4: When it is supported, for MTK’s example, our understanding is not only Pcell, Scell1, Scell2, and Scell3 would also monitor the legacy single cell scheduling DCI format(s) transmitted on Pcell with cross carrier scheduling. If there are N cells within the set of configured cells to be co-scheduled by a DCI format 0_X/1_X, does Proposal 2-4 imply cross-carrier scheduling should be configured for legacy DCI from the scheduling cell to N cells when the N cells do not include the scheduling cell, or to N-1 cells when the N cells include the scheduling cell? Is it restriction on cross-carrier scheduling configuration for legacy DCI? Then for legacy DCI format(s) not having the CFI field, how to support the cross-carrier scheduling? |
| Qualcomm | This is response to moderator. Regarding the following: E.g., Pcell uses DCI 0_X/1_X to co-schedule {Pcell, Scell 1, Scell 2, Scell 3}, it makes sense to support monitoring both DCI 0_X/1_X and legacy DCI formats from Pcell for each of {Pcell, Scell 1, Scell 2, Scell 3}. This is intention of Proposal 2-4. Yes, the intention is obvious. However, the impact to the UE side is quite significant, depending on how the numbers of BDs, CCEs, and DCI-sizes per scheduled CC are capped. Therefore, we are suggesting to start from BD/CCE (and DCI-size). Regarding the following: As you know, supporting monitoring DCI format 0_X/1_X and legacy DCI format(s) from different cells has significant impact on existing CA scheduling framework. We do not think this has significant impact on existing CA scheduling framework. Rel-17 already supports this. If only one scheduling cell is determined at a time dynamically, the impact on UE implementation is lesser than supporting DCI format 0_X/1_X for a set of cells and legacy DCI formats for each of the set of cells, from the same scheduling cell, and we do not need to worry about the BD/CCE/DCI-size budget. |
| ITRI | Support proposal 2-3 and proposal 2-4 |
| Moderator2 | @Fujitsu: As long as the DCI budget and BD/CCE budget can be maintained as legacy, it seems not necessary to preclude the simultaneous monitoring both DCI formats. Regarding your second comment, I am not sure whether I fully understand your point. It is possible that DCI format 0_X/1_X can schedule a single cell from the set of cells as long as the indicator of co-scheduled cells can be designed to indicate a single cell. For Proposal 2-4, it does not imply same legacy DCI formats for each co-scheduled cell. It is up to gNB configuration. As mentioned by other companies, in case of Pcell scheduling Pcell and several Scells, DCI format 0-0/1-0 can be monitored on Pcell for Self-scheduling Pcell. However, DCI format 0-0/1-0 can’t be configured on Pcell for cross-carrier scheduling any Scell. So no need and not possible to restrict same legacy DCI formats for each co-scheduled cell. @vivo: Yes, you are right. In your example, fallback DCI 0-0/1-0 cannot be used for cross-carrier scheduling from Pcell to Scell. It is up to NW configuration to decide which DCI format to use. I try to update the proposal to address your concern. @LG @Samsung: Thanks for the good suggestions. I will consider it for update. @xiaomi: In the first FFS, DCI format 0_X/1_X and legacy DCI formats are only transmitted on a single scheduling cell. @NTT DOCOMO: The legacy DCI formats can be configured up to gNB. As mentioned above, DCI format 0-0/1-0 can’t be configured for a Scell for CCS. It is up to NW configuration to decide which DCI format to use @Ericsson: Yes, it may be not necessary to configure all non-fallback DCIs. It is up to NW configuration to decide which DCI format to use. Let me update the proposal focusing on 4 non-fallback DCIs. @China Telecom: On P2-4, in the example, I mean DCI is transmitted on Pcell for scheduling Pcell, Scells. For your question, for cross-carrier scheduling, only legacy non-fallback DCIs with CIF field can be used. @Qualcomm: Yes, I understand your concern. It is not necessary to stop this discussion until the conclusion of BD/CCE is made. Anyway, the discussion can be started in parallel at least for collecting other companies’ views. Regarding your second comment, yes, Rel-17 support sSCell scheduling Pcell. But there is only Pcell which can be configured with two scheduling cell, i.e., itself and sSCell. Since we have only 1 TU in Q4 before completing the MC scheduling, only essential issues can be considered now. @All: I made some update on the proposal below. Please continue to make inputs. #### (Merged)Proposal 2-3 and 2-4rev1: - Confirm below working assumption reached in RAN1#110 meeting. For all cells within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X and DCI format 0_1/1_1/0_2/1_2 from a same scheduling cell. - The DCI format 0_X/1_X and the DCI format 0_1/1_1/0_2/1_2 can be monitored simultaneously up to network configuration. |
| Nokia / NSB | Fine with the update. Just to check, the limitation to 0_1/1_1/0_2/1_2 applies to USS only, Pcell can still be scheduled with fallback DCI formats from CSS still – right? |
| Apple | Fine with the updated/merged moderator’s proposal |
| Qualcomm | We are not OK with the latest proposal. - Mandating monitoring DCI 0_X/1_X for a CC-set and legacy DCI formats for each of all the CC-set may require UE to support larger BD/CCE numbers for each scheduled CC. Unless we see how BD/CCE is counted and how we set BD/CCE budget, we cannot agree this way. - We raised a question what is the use-case of letting UE to monitor such many DCI formats for each of many scheduled CCs. If this is agreed, DCI size alignment will be quite complicated and moreover, we need various optional UE capabilities (we do not want to mandate this support for all the Ues supporting multi-cell scheduling). This is also related to DCI-size budget issue. Please see our further reply to Proposal 2-7 update. |
| Moderator3 | @Nokia: Yes, you are right. @Qualcomm: This proposal doesn’t mandate monitoring DCI 0_X/1_X for a CC-set and legacy DCI formats for each of all the CC-set. The intention of the proposal is to support monitoring DCI 0_X/1_X for a CC-set and legacy DCI formats for each of all the CC-set. It depends on gNB configuration. Yes, we can conclude BD/CCE counting firstly then confirm this WA to address your concern. Secondly, this proposal does not mandate monitoring the DCI format 0_X/1_X and all DCI format 0_1/1_1/0_2/1_2 from a same scheduling cell. The intention is to support monitoring the DCI format 0_X/1_X and all DCI format 0_1/1_1/0_2/1_2 from a same scheduling cell. I agree with you there is no need to let UE monitoring all legacy DCI formats for each of co-scheduled CCs. It is totally up to gNB configuration. I agree with you that the DCI size budget and BD/CCE budget should be maintained as legacy. To address your concern, can we add a note below the proposal: Note: This does not imply mandating UE to monitor DCI format 0_X/1_X and all of DCI format 0_1/1_1/0_2/1_2 for each cell of the set of cells. |
| Intel | We are fine with the updated proposal. |
| Qualcomm | Response to moderator: We do not see the difference between “mandate monitoring DCI 0_X/1_X for a CC-set and legacy DCI formats for each of all the CC-set” and “support monitoring DCI 0_X/1_X for a CC-set and legacy DCI formats for each of all the CC-set”, if it is “totally up to gNB configuration”. Our best compromise is to accept this as an advanced/optional functionality once we see further details of basics of BD/CCE budget/counting and DCI-size budget/counting. This is why we recommend to start with BD/CCE aspects to begin with. |
| LGE2 | In principle, we are fine with (Merged)Proposal 2-3 and 2-4rev1. For clarity, suggest some modification as follows #### (Merged)Proposal 2-3 and 2-4rev1: - Confirm below working assumption reached in RAN1#110 meeting. For any cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X and DCI format 0_1/1_1/0_2/1_2, if configured from a same scheduling cell. - The DCI format 0_X/1_X and the DCI format 0_1/1_1/0_2/1_2, if configured can be monitored simultaneously up to network configuration. |
| Vivo2 | - The DCI format 0_X/1_X and the DCI format 0_1/1_1/0_2/1_2 can be monitored simultaneously up to network configuration. Thanks for the update, but this bullet is not clear. ‘can monitor’ seems to depend on UE capability, which may not be up to NW configuration, maybe we can revise the wording as: - it is up to Network to configure DCI format 0_X/1_X and the DCI format 0_1/1_1/0_2/1_2 can be monitored for monitoring simultaneously for a cell |
| Moderator4 | @vivo: With the addition of “if configured” in (Merged)Proposal 2-3 and 2-4rev1, I think it implies up to NW configuration. |
| Samsung2 | Support the merged proposal in principle. Not sure if DCI formats 0_0/1_0 were omitted by mistake. If not, we would like to understand the reason, especially for the Pcell. |
| Langbo | As clarified by FL, not all of DCI format 0_1/1_1/0_2/1_2 need be configured by network. This hould be clarified in the proposal. We also think the network should have the flexibility to not configure any legacy DCI format. Our suggested revision as: - It is up to the network to configure whether DCI format 0_X/1_X and one of DCI format 0_1/1_1/0_2/1_2 can be monitored simultaneously and which combination of DCI format 0_1/1_1/0_2/1_2 is monitored simultaneously. |
| NTT DOCOMO2 | Thank FL for the reply and updates. We are fine that the legacy DCI format(s) which is(are) monitored with DCI format 0_X/1_X is up to NW configuration. In our understanding for the current proposal, all or subset of DCI format 0_1/1_1/0_2/1_2 can be configured to be monitored with 0_X/1_X but 0_0/1_0 cannot be configured for USS. On top of that, now it can be discussed further whether/how to maintain the current DCI size budget. We think that this discussion has a dependency on the discussion for DCI size alignment and BD/CCE counting, so we are also fine to agree on this proposal as WA for now. |
| Fujitsu | We are fine with the updates from LGE. |
| Qualcomm | We are not OK with the proposal due to the reason we already shared above. “Up to network configuration” implicitly mandate all Ues supporting MC-scheduling to support this. |
| Moderator5 | @LG: Your update is fine with me. @Samsung: One note is added to cover self-scheduling on Pcell via fallback DCI. @QC: Can we add a note like “UE is not mandated to support monitoring DCI 0_X/1_X for a set of cells and legacy DCI formats for each of the set of cells” to address your concern? #### (Merged)Proposal 2-3 and 2-4rev2: - Confirm below working assumption reached in RAN1#110 meeting. For any cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X and DCI format 0_1/1_1/0_2/1_2 if configured, from a same scheduling cell. - The DCI format 0_X/1_X and the DCI format 0_1/1_1/0_2/1_2 can be monitored simultaneously up to network configuration. - Note: DCI format 0_0/1_0 can be configured on Pcell for self-scheduling if Pcell is the scheduling cell. - Note: UE is not mandated to support monitoring DCI 0_X/1_X for a set of cells and legacy DCI formats for each of the set of cells |
| Apple | We are fine to support the updated/merged proposal |
| Qualcomm | Thanks, moderator. We can accept the following. #### (Merged)Proposal 2-3 and 2-4rev2 – QC update: - Confirm below working assumption reached in RAN1#110 meeting. For any cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, RAN1 specification supports monitoring the DCI format 0_X/1_X and DCI format 0_1/1_1 and/or 0_2/1_2, if supported by the UE and if configured, from a same scheduling cell. - The DCI format 0_X/1_X and the DCI format 0_1/1_1/0_2/1_2 can be monitored simultaneously. - Note: DCI format 0_0/1_0 can be configured on Pcell for self-scheduling if Pcell is the scheduling cell. - Note: Ues supporting multi-cell scheduling by a DCI format 0_X/1_X are not mandated to support monitoring DCI 0_X/1_X for a set of cells and legacy DCI formats for each of the set of cells - Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell. |
| Moderator6 | @Qualcomm: Thanks for the good suggestion. It is fine with me. Let’s check other companies’ views. @All: please check whether below update is fine with you. #### (Merged)Proposal 2-3 and 2-4 rev3: - Confirm below working assumption reached in RAN1#110 meeting. For any cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, RAN1 specification supports monitoring the DCI format 0_X/1_X and DCI format 0_1/1_1 and/or 0_2/1_2, if supported by the UE and if configured, from a same scheduling cell. - The DCI format 0_X/1_X and the DCI format 0_1/1_1/0_2/1_2 can be monitored simultaneously. - Note: DCI format 0_0/1_0 can be configured on PCell for self-scheduling if PCell is the scheduling cell. - Note: UEs supporting multi-cell scheduling by a DCI format 0_X/1_X are not mandated to support monitoring DCI 0_X/1_X for a set of cells and legacy DCI formats for each of the set of cells - Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell. |

### 2nd round of discussions

#### (Merged)Proposal 2-3 and 2-4 rev3:

- Confirm below working assumption reached in RAN1#110 meeting.

For any cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, RAN1 specification supports monitoring the DCI format 0_X/1_X and DCI format 0_1/1_1 and/or 0_2/1_2, if supported by the UE and if configured, from a same scheduling cell.

- The DCI format 0_X/1_X and the DCI format 0_1/1_1/0_2/1_2 can be monitored simultaneously.

- Note: DCI format 0_0/1_0 can be configured on PCell for self-scheduling if PCell is the scheduling cell.

- Note: UEs supporting multi-cell scheduling by a DCI format 0_X/1_X are not mandated to support monitoring DCI 0_X/1_X for a set of cells and legacy DCI formats for each of the set of cells

- Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | OK in principal |
| Qualcomm | We are OK with the proposal. |
| Fujitsu | We have one question. If a cell can be scheduled by a DCI format 0_X/1_X, but only by single-cell scheduling way (i.e. it cannot be simultaneously scheduled with other cells by the DCI format 0_X/1_X), does “a set of cells which can be co-scheduled by a DCI format 0_X/1_X” include the cell? If the intention is to include such a cell, we prefer to change “co-scheduled” to be “scheduled”, because “co-scheduled” here seems to imply the “set of cells” only includes cells which can be simultaneously scheduled with other cells. |
| Langbo | Fine with the proposal. |
| LGE2 | We have a concern about the 2nd Note. Note: UEs supporting multi-cell scheduling by a DCI format 0_X/1_X are not mandated to support monitoring DCI 0_X/1_X for a set of cells and legacy DCI formats for each of the set of cells Whether UE behaviour of monitoring those DCI formats is mandatory or optional should be in UE capability discussion, not here. So, this Note is unnecessary and should be removed from the proposal. Remaining parts of the proposal is fine. |
| Nokia/NSB | Similar question as LG on the UE capability here (if we have this, further clarifications would be needed): - would the UE at least need to support one cell for ‘SC-DCI’ scheduling, or can the UE say I don’t support it even for a single cell. - And assuming the UE gives capability of {1,2,3,4} – can the gNB then freely select the cells it would configure for SC-DCI scheduling? |
| NTT DOCOMO | Regarding the second note, our preference is that legacy DCI formats can be monitored for each of the set of cells. The motivation to support the scheduling by legacy DCI format with DCI format 0_X/1_X is that legacy DCI can provide better configuration flexibility of each DCI fields and also can schedule efficiently with smaller payload size compared to DCI format 0_X/1_X. Therefore, we prefer to support monitoring of legacy DCI format for each of the set of cells at least either cross-carrier scheduling or self-carrier scheduling. |
| Moderator | @Fujitsu: For each scheduled cell, there is at most one scheduling cell which can use DCI format 0_X/1_X for scheduling the cell. Maybe I misunderstood your point, please correct me. @Qualcomm: With the concern from LG and Nokia and DOCOMO, can we delete the 2nd note and discuss it in UE feature? We really need move on. |
| Xiaomi | We are confused on why the fallback DCI is not on the main bullet. The first note already allows this in our understanding. |
| Qualcomm | @ Moderator, LGE We are OK to discuss details in UE feature session. However, we would like to make sure that the UE is not mandated to support more than per-CC BD limit. Unless we take the above approach, we need the 2nd note. @ DOCOMO If companies want to mandate monitoring DCI 0_X/1_X for a set of cells and legacy DCIs for all the cells in the set, then BD/CCE should be counted on each of the set of cells. With this, more than per-CC BD limit is not required and we do not need to discuss UE capability aspect. We have asked multiple times – which does DOCOMO want? (Following figure illustrates the case where BD for DCI 0_X/1_X is counted on all the cells in the set, which is copied from Ericsson’s contribution) @ Nokia For your questions: - would the UE at least need to support one cell for ‘SC-DCI’ scheduling, or can the UE say I don’t support it even for a single cell. [QC] We are OK to say that the UE need to support at least one cell for “SC-DCI” scheduling. - And assuming the UE gives capability of {1,2,3,4} – can the gNB then freely select the cells it would configure for SC-DCI scheduling? [QC] Would be fine in the end, but we would like to check until further progress is made. For example, we are not sure whether CCs {1, 2, 3, 4} have the same or different characteristics. We are also not sure whether the UE monitors same or different set of legacy DCIs for different CCs amongst {1, 2, 3, 4}. |
| Apple | We are fine to support the updated proposal and are also fine with the note |
| ZTE | For the first NOTE, we guess it means that DCI format 0_0/1_0 and the other DCI formats (0_X/1_X, 0_1, 1_1, 0_2, 1_2) can be monitored simultaneously in the PCell. We are fine if the answer is yes. |
| MTK | We support the updated proposal and fine to include the note or not (discuss later). To our understanding, the note should be an additional optional UE capability (support DCI 0_X/1_X and legacy cross-carrier scheduling at the same time) |
| Huawei2 | Ok with the proposal. |
| Vivo3 | 1.With a lot of change that have been made this meeting, we don’t think it is correct to say ‘Confirm below working assumption reached in RAN1#110 meeting.’ in the main bullet. This is sth apparently different. 2.For the first note, we think Pcell is not the only case. Any cell that is configured as scheduling cell for mc-scheduling can be configured to use fallback DCI for self-scheduling, so we changed ‘Pcell’ to ‘the scheduling cell’ 3.For the second note, we are not sure about what ‘a set of cells’ refers to. Is it referring to ‘a set of cells which can be co-scheduled by a DCI format 0_X/1_X’ in the main bullet, which as clarified by FL before is the entire set of co-schedulable cells. Or is it referring to a different set (e.g, a cell combination)? I assume it should be the entire set - Confirm below working assumption reached in RAN1#110 meeting. For any cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, RAN1 specification supports monitoring the DCI format 0_X/1_X and DCI format 0_1/1_1 and/or 0_2/1_2, if supported by the UE and if configured, from a same scheduling cell. - The DCI format 0_X/1_X and the DCI format 0_1/1_1/0_2/1_2 can be monitored simultaneously. - Note: DCI format 0_0/1_0 can be configured on the scheduling cell for self-scheduling. - Note: UEs supporting multi-cell scheduling by a DCI format 0_X/1_X are not mandated to support monitoring DCI 0_X/1_X for the set of cells and legacy DCI formats for each of the set of cells - Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell. |
| CMCC | We are fine with the proposal. And also fine if DCI format 0_0/1_0 being included in the main bullet. |
| Intel | We are generally fine with the proposal. Some wording improvement - For the first note, it is not necessarily limited to PCell, but can be a SCell if the SCell is a scheduling cell. Therefore, we propose the following revision - Note: DCI format 0_0/1_0 can be configured on a cell PCell for self-scheduling if PCell the cell is the scheduling cell. - For the last note, since are not defined per scheduled but per group of scheduled cells with same numerology. The following update can be considered - Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell. |
| CATT | We are OK with the proposal. |
| Fujitsu | To FL, Thanks for the reply. However, our question is not about multiple scheduling cells for a scheduled cell. Rather, the question is similar to what we raised up by email for other proposals. We’d like to ask, whether cell 2/3 as below is “within a set of cells which can be co-scheduled by a DCI format 0_X/1_X”? Assume cell {0,1,2,3} are all the cells that can be scheduled by DCI format 0_X/1_X. And cell {0, 1}, cell 2, cell 3 can respectively be scheduled by a same DCI 0_X/1_X. In this example, cell 2/3 can only be scheduled alone (i.e. the cell would never be scheduled together with another cell by a same DCI 0_X/1_X). |
| Samsung3 | Our understanding is that the intention of this proposal is to clarify that the UE can be configured, for any/all cells, to (simultaneously) monitor PDCCH for DCI format 0_X/1_X and for single-cell scheduling DCI formats – this also includes DCI formats 0_0/1_0. If it does not, can the FL please explain why? Of course, it is up to gNB to decide which DCI formats are configured for which cells, but once configured, the UE shall follow the configuration. That is Rel-17 operation and there is no apparent reason why it should change due to multi-cell scheduling. Whether to allow UE capability for restricted PDCCH monitoring needs to be further justified and is a separate discussion. So either the second note should be removed or at least made as FFS, especially since the main bullet mentions “RAN1 specification supports…”. Therefore, we suggest the following revision: #### (Merged)Proposal 2-3 and 2-4 rev3: - Confirm below working assumption reached in RAN1#110 meeting. For any cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, RAN1 specification supports monitoring the DCI format 0_X/1_X and any of DCI formats 0_0/1_0, and/or 0_1/1_1 and/or 0_2/1_2, if supported by the UE and if configured, from a same scheduling cell. - The UE can be configured to monitor, for any cell within the set of cells, Tthe DCI format 0_X/1_X and any of the DCI formats 0_0/1_0/0_1/1_1/0_2/1_2 can be monitored simultaneously. - Note: DCI format 0_0/1_0 can be configured on PCell for self-scheduling if PCell is the scheduling cell. - Note FFS: UEs supporting multi-cell scheduling by a DCI format 0_X/1_X are not mandated to support monitoring DCI 0_X/1_X for a set of cells and legacy DCI formats for each of the set of cells - Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell. BD/CCE counting rules for DCI 0_X/1_X within the Rel-17 limits are separately discussed. |
| Moderator | With below agreement, we can conclude this discussion. (Merged)Proposal 2-3 and 2-4 rev4: Confirm below working assumption reached in RAN1#110 meeting with revision. Working Assumption - For any cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, RAN1 specification supports monitoring the DCI format 0_X/1_X and DCI format 0_0/1_0, 0_1/1_1, and/or 0_2/1_2 (if supported by the UE), if configured from a same scheduling cell. - The DCI format 0_X/1_X and the DCI format 0_0/1_0/0_1/1_1/0_2/1_2 can be monitored simultaneously. - Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell. |
| CATT | We support the FL’s updated proposal and ok with the note. |
| Apple | Support the merged proposal from FL |
| Moderator | @All: This issue has been concluded with above agreement. So no need to further input here. |

## Search space configuration

Huawei: - Proposal 10: Support Alt 2 or Alt 3 that search space of the DCI format 0_X/1_X can be configured on one cell or a subset of the set of cells and associated with the search space on the scheduling cell with the same search space ID. - Proposal 11: Support Alt 1 that the n_CI (or n_MI) in the search space equation is determined by a value configured for the set of cells. ZTE: - Proposal 7: MC-DCI used for scheduling a group of configured co-scheduled cells can be transmitted and detected in a USS of a scheduled cell if the scheduled cell is configured within the group of co-scheduled cells. Spreadtrum: - Proposal 3: Search space of the DCI format 0_X/1_X is configured on a subset of the set of cells and associated with the search space on the scheduling cell with the same search space ID. - Proposal 4: For monitoring PDCCH candidates for a set of cells which can be co-scheduled by a DCI format 0_X/1_X, the n_CI in the search space equation is determined by a value configured for one or more combinations of co-scheduled cells within the set of cells. - Proposal 5: The DCI size should be same for the CCE indexes of PDCCH candidates obtained by one CIF value. Vivo: - Proposal 2. For multi-cell scheduling, SS linkage between scheduling cell and scheduled cell is built in the same way as cross-carrier scheduling. - Proposal 3. Clarify whether it is allowed to transmit/receive multiple mc-DCIs scheduling different sets of co-scheduled cells in a same monitoring occasion. OPPO: - Proposal 8: PDCCH monitoring capability and DCI size could be counted based on the search space set configuration. Alt 3 and Alt 4 can be considered for the search space set configuration. - Proposal 10: the n_CI in the search space equation is determined by a value of CIF value of the cell or cell set which is configured with the search space sets for DCI format 0_X/1_X. CATT: - Proposal 6: For multi-cell scheduling, the nCI in the search space equation is determined by a value X configured for the set of cells co-scheduled by DCI format 0_X/1_X. - Proposal 7: For multi-cell scheduling, search space set(s) can be configured for one or more scheduled cells within a set of cells which can be co-scheduled by DCI format 0_X/1_X, and associated with the search space set on the scheduling cell with the same search space ID. Langbo: - Proposal 6: Search space of the DCI format 0_X/1_X is configured for each cell of the set of cells which can be co-scheduled and is associated with the search space for the scheduling cell with the same search space ID (i.e., Alt 1). - Proposal 7: A same number of candidates is configured for all the search spaces for the co-scheduled cells with the same search space ID. - Proposal 8: For a set of cells that can be potentially co-scheduled by a DCI format 0_X/1_X, the n_CI in the search space equation for each cell combination in the set of cells is determined by a same value configured for the set of cells. Fujitsu: - Proposal 4: For determining PDCCH candidates for multi-cell scheduling, consider the following 2 options. Option 1 is slightly preferred because it is similar to the method for legacy cross-carrier scheduling. - Option 1: the value of nCI and the number of PDCCH candidates per AL is per combination of scheduled cells - the value of nCI is equal to the value of the indicator in DCI format 0_X/1_X for indicating scheduled cells. - Option 2: the value of nCI and the number of PDCCH candidates per AL is per set of cells that can be scheduled by the DCI format 0_X/1_X - The set of cells includes all cells in the table defining combinations of scheduled cells. Intel: - Proposal 12: Search space set of the DCI format 0_X/1_X is configured on each cell of the set of cells that can be scheduled by the DCI format 0_X/1_X and associated with the search space set on the scheduling cell with the same search space ID - Only parameter nrofCandidates is included in the search space set configuration for a scheduled cell - The mapped CCE index is determined by nCI respectively configured for each cell the set of cells Xiaomi: - Proposal 5: The nCI in the search space equation is determined by a value configured for each combination of co-scheduled cells within the set of cells. - Proposal 6: The nrofCandidates configured on each of the co-scheduled cells associated with one should be identical. - Proposal 7: The CIF values (nCI) for single cell scheduling and multi-cell scheduling should be different to reduce the PDCCH blocking probability. Lenovo: - Proposal 23: It is up to gNB to configure search space for DCI format 0_X/1_X on one or multiple cells within a set of cells that can be co-scheduled by the DCI format 0_X/1_X. - Proposal 27: A single value is configured as n_CI for a set of cells which can be co-scheduled by DCI format 0_X/1_X . CMCC: - Proposal 5. The search space of DCI format 0_X/1_X can be configured on each cell of the set of co-scheduled cells and associated with the search space on scheduling cell with the same search space ID. - Proposal 6. The same nCI for calculating CCE indexes of PDCCH candidates can be configured for several combinations of co-scheduled cells within the set of configured cells. FGI: - Proposal 7: For search space configuration for a set of cells which can be co-scheduled by a DCI format 0_X/1_X, search space of the DCI format 0_X/1_X is configured on each cell of the set of cells and associated with the search space on the scheduling cell with the same search space ID. - Proposal 8: The n_CI in the search space equation is determined by a value configured for one or more combinations of co-scheduled cells within the set of cells. LG: - Proposal #8: Consider how to configure the search space for DCI format 0_X/1_X based on the following two options. - Option 1: Configure the search space for DCI format 0_X/1_X separately from the legacy non-fallback DCI format. - Option 2: Configure the search space for DCI format 0_X/1_X jointly with the legacy non-fallback DCI format. - Proposal #9: Clarify the meaning of “a set of cells” in Proposal 2-8rev2 on the SS configuration for multi-cell scheduling in the FL summary in RAN1#110. - Clarify whether it refers to the entire set of cells which can be scheduled by DCI format 0_X/1_X or a combination of co-scheduled cells by DCI format 0_X/1_X. - Proposal #10: Alt 1 is preferred with assumption that “a set of cells” in Proposal 2-8rev2 is the entire set of cells which can be scheduled by DCI format 0_X/1_X. - Proposal #11: Consider how to determine the number of PDCCH candidates for the SS on scheduling cell for multi-cell scheduling, based on following three options. - Option 1: The number of PDCCH candidates for the scheduling cell for DCI format 0_X/1_X is the same as the number of PDCCH candidates configured for one of the scheduled cells which are linked to the scheduling cell. - Option 2: The number of PDCCH candidates for the scheduling cell for DCI format 0_X/1_X is determined as the sum of those configured for all scheduled cells which are linked to the scheduling cell. - Option 3: The number of PDCCH candidates for the scheduling cell for DCI format 0_X/1_X is determined as the mean value of those configured for all scheduled cells which are linked to the scheduling cell. - Proposal #12: Consider Alt 1 or Alt 2 according to SS set configuration for the scheduling cell (i.e., whether it is based on the entire set of scheduled cells or each combination of co-scheduled cells). MTK: - Proposal 10: Use the CCE index formula in 38.213 10.1 to stagger the DCI with different set of scheduled cells, where nCI is replaced by the set index where one set contains a group of cells. One example is shown below: Scheduled cells combination for one multi-cell scheduling DCI; Which cell to count the corresponding DCI size and BD/CCE; nCI (set index). Cell 1, Cell 2, Cell 3, Cell 4; Cell 1; 0. Cell 2, Cell 3; Cell 2; 1. Cell 2, Cell 3, Cell 4; Cell 3; 2. Cell 1, Cell 3; Cell 3; 3. Apple: - Proposal 6: For search space configuration for a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support configuration of search space of the DCI format 0_X/1_X on the scheduling cell and linked with the set of cells configured by explicit RRC signaling - Proposal 7: For monitoring PDCCH candidates for a set of cells which can be co-scheduled by a DCI format 0_X/1_X, the n_CI in the search space equation is determined by a value configured for the set of cells if different DCI sizes are not supported for different number of co-scheduled cells Samsung： - Proposal 11: The UE determines PDCCH monitoring according to a search space set for a set of co-scheduled cells based on multi-cell extension of the Rel-17 search space linking procedure (Alt-1) or based on RRC configuration of the linking (Alt-4). - Proposal 12: For determining CCEs corresponding to PDCCH candidates for detection of an MC-DCI format: - The UE sets the nCI in the search space equation to be the cell-set-level CIF value, that is separately provided for each configured/schedulable set of cells. Ericsson: - Proposal 8: Linked search space mechanism is reused for mc-DCI search space configuration. - For each group of cells configured to be scheduled by a mc-DCI format, the linked search space on the “scheduled cell” (i.e. containing only BD/CCE info and linked search space ID) is configured on one of the cells in the group. - Proposal 9: For monitoring DCI 0_X/1_X PDCCH candidates for a set of cells, the n_CI to be used in the search space equation is explicitly configured for the set of cells. NTT DOCOMO: - Proposal 13: RAN1 should discuss search space configuration for a set of cells which can be co-scheduled by a DCI format 0_X/1_X based on the following alternatives; - Alt 1: Search space of the DCI format 0_X/1_X is configured on one cell - Alt 1-1: Search space of the DCI format 0_X/1_X is configured only on the scheduling cell and linked with the set of cells configured by explicit RRC signaling. - Alt 1-2: Search space of the DCI format 0_X/1_X is configured on one cell of the set of cells and associated with the search space on the scheduling cell with the same search space ID. - Alt 2: Search space of the DCI format 0_X/1_X is configured on more than one cell - Alt 2-1: Search space of the DCI format 0_X/1_X is configured on each cell of the set of cells and associated with the search space on the scheduling cell with the same search space ID. - Alt 2-2: Search space of the DCI format 0_X/1_X is configured on each combination of actually co-scheduled cells and associated with the search space on the scheduling cell with the same search space ID. - Proposal 14: For monitoring PDCCH candidates for a set of cells which can be co-scheduled by a DCI format 0_X/1_X, the n_CI in the search space equation is determined by a value configured for each combination of co-scheduled cells within the set of cells. Qualcomm: - Proposal 1: - Re-use CIF/nCI framework - Multiple cells can be mapped to a CIF/nCI value of a DCI format monitored on a scheduling cell - The DCI may schedule data on one, some, or all of the cells mapped to the CIF/nCI value Google: - Proposal 1: For search space configuration for a set of cells which can be co-scheduled by a DCI format 0_X/1_X, search space of the DCI format 0_X/1_X is configured on each cell of the set of cells and associated with the search space on the scheduling cell with the same search space ID. (Alt-1) - Proposal 2: The parameter n_CI in the search space equation is determined by a RRC configured value associated with the carrier indicator field in the DCI format. The association between the RRC configured value and the carrier indicator field can be determined in different discussion. Nokia: - Proposal 3.5.1: For search space configuration for a set of cells which can be co-scheduled by a DCI format 0_X/1_X, adopt Alt. 4, i.e. - Alt 4: Search space of the DCI format 0_X/1_X is configured only on the scheduling cell and linked with the set of cells configured by explicit RRC signaling. - Proposal 3.5.2: For monitoring PDCCH candidates for a set of cells which can be co-scheduled by a DCI format 0_X/1_X, adopt Alt. 1, i.e. - Alt 1: the n_CI in the search space equation is determined by a value configured for the set of cells.

### Moderator summary and proposals based on contributions

- On search space configuration for monitoring DCI format 0_X/1_X

According to the framework of search space configuration for legacy cross-carrier scheduling, the corresponding search space set is separately configured for each scheduled cell with the necessary information of search space ID, the number of PDCCH candidate and corresponding aggregation level for cross-carrier scheduling. Search space set linking is adopted for a UE to monitor PDCCH for a scheduled cell in a search space configured on the scheduling cell with same search space set ID. Based on search space linking, for cross-carrier scheduling, the UE monitors the PDCCH candidates on the scheduling cell according to the search space configuration on the corresponding scheduled cell.

For Rel-18 multi-cell scheduling, since the multi-cell scheduling DCI can schedule multiple cells, one issue needs to be discussed is whether the search space set configuration on the number of PDCCH candidates per aggregation level is configured for each co-scheduled cell, a subset of co-scheduled cells or a single scheduled cell of the co-scheduled cells.

For RAN1#110 meeting, this issue has been extensively discussed with below proposal captured in FL summary#6.

#### Proposal 2-8rev2: - For search space configuration for a set of cells which can be co-scheduled by a DCI format 0_X/1_X, below options are considered for further study: - Alt 1: Search space of the DCI format 0_X/1_X is configured on each cell of the set of cells and associated with the search space on the scheduling cell with the same search space ID. - Alt 2: Search space of the DCI format 0_X/1_X is configured on a subset of the set of cells and associated with the search space on the scheduling cell with the same search space ID. - Alt 3: Search space of the DCI format 0_X/1_X is configured on one cell of the set of cells and associated with the search space on the scheduling cell with the same search space ID. - Alt 4: Search space of the DCI format 0_X/1_X is configured only on the scheduling cell and linked with the set of cells configured by explicit RRC signaling. - Other alternatives are not precluded.

For RAN1#110bis meeting, a lot of companies express their views on this issue. The main options include configuring the search space for multi-cell scheduling on each of co-scheduled cells, a subset of co-scheduled cells, a single cell of the co-scheduled cells, or even only on scheduling cell. Companies’ views are summarized as below:

- Alt 1:

  - Supported by: Langbo, Intel, CMCC, FGI, LG, Samsung, NTT DOCOMO, Google

- Alt 2:

  - Supported by: Huawei, Spreadtrum, CATT, Lenovo

- Alt 3:

  - Supported by: Huawei, OPPO, CATT, Lenovo, Ericsson, NTT DOCOMO

- Alt 4:

  - Supported by: OPPO, Apple, Samsung, NTT DOCOMO, Nokia

Firstly, the legacy search space linkage should be followed when designing the search space configuration for multi-cell scheduling DCI. Secondly, it should be up to gNB to configure which cells within the set of cells that can be co-scheduled by DCI format 0_X/1_X are configured with search space for DCI format 0_X/1_X. It could be a single cell, a subset or all cells within the set of cells, dependent on gNB configuration so as to give full flexibility to gNB.

To avoid further discussion on the criteria of cell selection for search space configuration, moderator tries to merge Alt 1/2/3 and leave it to gNB configuration. The corresponding proposal is provided in Proposal 2-5.

- On n_CI value determination for monitoring DCI format 0_X/1_X

A second issue is about determining the CCEs for monitoring the multi-cell scheduling DCI. In legacy cross-carrier scheduling, for a scheduled cell, the CCEs for each configured aggregation level of PDCCH candidates are determined based on below equation, where nCI is the CIF value configured by CrossCarrierSchedulingConfig.

For Rel-18 multi-cell scheduling, one issue is how to determine the value of n_CI for co-scheduled cells. Several options are provided, e.g., the n_CI is determined by a value configured for the co-scheduled cells, the n_CI is determined for each combination of co-scheduled cells.

For RAN1#110 meeting, this issue has been extensively discussed with below proposal captured in FL summary#6.

#### Proposal 2-9: - For monitoring PDCCH candidates for a set of cells which can be co-scheduled by a DCI format 0_X/1_X, below alternatives are considered for further study: - Alt 1: the n_CI in the search space equation is determined by a value configured for the set of cells. - Alt 2: the n_CI in the search space equation is determined by a value configured for each combination of co-scheduled cells within the set of cells. - Alt 3: the n_CI in the search space equation is determined by a value configured for one or more combinations of co-scheduled cells within the set of cells. - Other alternatives are not precluded.

For RAN1#110bis meeting, a lot of companies express their views on this issue. Companies’ views are summarized as below:

- Alt 1:

  - Supported by Huawei, OPPO, CATT, Langbo, Fujitsu, Lenovo, LG, Apple, Samsung, Nokia

- Alt 2:

  - Supported by Fujitsu, xiaomi, MediaTek, Samsung, NTT DOCOMO

- Alt 3:

  - Supported by Spreadtrum, CMCC, FGI

Firstly, before decoding DCI format 0_X/1_X, UE can’t know which cell combination is scheduled. Correspondingly, for Alt 2, if the n_CI is determined by a value configured for each combination of co-scheduled cells within the set of cells, it will lead to different CCE indices of PDCCH candidates for monitoring DCI format 0_X/1_X. Similar problem happens for Alt 3 if multiple values are configured for multiple combinations of co-scheduled cells within the set of cells. Alt 1 is straightforward to configure a single value for the entire set of co-schedulable cells so that there is only one CCE index for monitoring DCI format 0_X/1_X. Upon detection of the DCI format 0_X/1_X, UE can know which cells are scheduled by checking the indicator of co-scheduled cells.

Hence, moderator intends to go with Alt 1 for the first round of discussions. The corresponding proposal is provided in Proposal 2-6.

### 1st round of discussions

#### Proposal 2-5:

- For a set of cells which is configured for multi-cell scheduling, search space of DCI format 0_X/1_X can be configured on one, a subset, or all cells of the set of cells and associated with the search space on the scheduling cell with the same search space ID:

- Note: It is up to gNB to determine which cell(s) of the set of cells the search space of the DCI format 0_X/1_X is configured on.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| MTK | We support Alt 2 but can live with Alt 1 (Proposal 2-5). |
| Qualcomm | We are not OK with the proposal. It is not clear to us what will be the total number of BDs/CCEs for DCI 0_X/1_X for a given set of scheduled CCs if it is configured to be monitored on various search space sets associated to multiple scheduled CCs. Suppose a UE monitors DCI 0_X/1_X on CC 0 for CCs {1, 2, 3, 4}. Suppose monitoring DCI 0_X/1_X on CC 0 is configured on search space sets associated to all of the CCs {1, 2, 3, 4}. Suppose the scheduling CC uses SCS 30kHz, in which case per-CC BD budget is 36 per slot. the UE monitors DCI 0_X/1_X on various SS sets on CC 0. - Whether the total number of BDs for the DCI 0_X/1_X for CCs {1, 2, 3, 4} across all the search space sets on CC 0 is capped by 36 or not? Our answer is yes, at least for basic MC-scheduling. Same for CCE budget and DCI-size budget. Although we are open to discuss advanced case, we would like to clarify the basic case first. We are not sure how Proposal 2-5 can work with per-scheduled-CC BD/CCE budget. Please see our response to Proposal 2-4 above as well. |
| Nokia, NSB | Clarification on the proposal appreciated: What is proposed here is a mix of Alt. 1, 2 & 3. So what should the UE expect here? If it can be only configured on one cell (Alt. 3) why would the gNB in addition configure any other cells (a subset or all cells) on top? Having exactly the same DCI (size, n_CI etc.) and the DCI content defining the scheduled cells, also the same number of PDCCH candidates would be needed there. So the proposed flexibility here is unclear to use. |
| Apple | We tend to agree with Nokia that if one cell is configured with the search space configuration for DCI format 0_X/1_X, then we don’t see the motivation to have additional cell(s) configured with the search space configuration for DCI format 0_X/1_X. From this point of view, only one cell configured with search space configuration for DCI format 0_X/1_X is reasonable option. Furthermore, that one cell can be the scheduling cell |
| Langbo | In our view, the simplest way is to fully reuse existing design, i.e., search space is configured per scheduled cell, and gNB guarantees that the numbers of candidates per AL in all configured search spaces that linked to a search space configured with DCI format 0_X/1_X are the same. In this way, no new signalling design in scheduling cell is needed and no ambiguity on the determination of maximum number of candidates in the search space formula. |
| Huawei, HiSilicon | Generally OK however, we agree with QC that this may be related to the BD budget. On the other hand, we also share the feeling that such flexibility of allowing all cases to network might be unclear, while it is clear that if at least one cell can be configured with the search space configurations, the SS function works and even RRC configurations may be reduced compared to configuring to each cell. And in this case, the BD/CCE distribution is also clear. So we still have a slight preference to at least support one cell case. We can further discuss other cases - subset, or each. |
| Intel | We are not OK with the proposal. Alt 1, i.e., reusing the legacy behaviour, provides the largest flexibility too. For example, gNB may configure a SearchSpaceID for single-cell scheduling on PCell, while Alt 1 can be still used to link this SearchSpaceID for DCI 0-X/1-X. |
| ZTE | The configuration is up to gNB implementation. This can bring sufficient flexibility for search space configuration. We support this proposal in principle. Based on the current search space linkage, DCI formats in the USS for scheduled cell will not be configured. We suggest the following modification. #### Proposal 2-5(update): - For a set of cells which is configured for multi-cell scheduling, search space of DCI format 0_X/1_X can be configured on one, a subset, or all cells of the set of cells and associated with the search space on the scheduling cell with the same search space ID and configured with DCI format 0_X/1_X : - Note: It is up to gNB to determine which cell(s) of the set of cells the search space of the DCI format 0_X/1_X is configured on. |
| Moderator | @Qualcomm: The BD/CCE issues can be discussed in next sub-section. @Nokia @Apple: The intention of this proposal is to leave to gNB to determine which cell(s) of the set of cells the search space of the DCI format 0_X/1_X is configured on as there is no clear majority view on Alt 1/2/3. So I try to merge three alternatives into one proposal. From UE’s point of view, it just follows gNB’s configuration for DCI monitoring. As mentioned by some companies, if SS is only configured on one scheduled cell, when the cell is deactivated or DRX off, then multi-cell scheduling can’t work. Configuring SS on more than one cell seems reasonable. @Langbo @Intel: Since one DCI format 0_X/1_X can schedule multiple cells, it is no need to configure same SS configurations on all the scheduled cells. A subset of cells is sufficient. The existing design is followed. @Huawei: As mentioned by some companies, if SS is only configured on one scheduled cell, when the cell is deactivated or DRX off, then multi-cell scheduling can’t work. Configuring SS on more than one cell seems reasonable. If majority companies think one cell is enough, we can go directly with it. @ZTE: I think no need to add it again as mentioned in the beginning. |
| CMCC | We prefer Alt 1, configuring the search space of DCI format 0_X/1_X on each co-scheduled cell can provide more scheduling flexibility. In our views, the scheduled cells are unknown to the UE before monitoring DCI, extra RRC signalling will be required for Alt 2 and Alt 3. |
| New H3C | We are fine with this proposal in general |
| Fujitsu | We are generally OK with the proposal. However, for the case when gNB configures search space on multiple cells, we suggest to clarifying in the proposal that the number of PDCCH candidates per AL configured in each cell is the same. Otherwise, we may need to further discuss which configured number should be used in case of different numbers, which is not expected. |
| CATT | We support proposal 2-5. Taking Qualcomm’s assumption as an example, we would like to share our view on how proposal 2-5 works with per-scheduled CC BD/CCE budget. Suppose a UE monitors DCI 0_X/1_X on CC 0 for CCs {1, 2, 3, 4}. Suppose monitoring DCI 0_X/1_X on CC 0 is configured on search space sets associated to all of the CCs {1, 2, 3, 4}. Suppose the scheduling CC uses SCS 30kHz, in which case per-CC BD budget is 36 per slot. The UE monitors DCI 0_X/1_X on various SS sets on CC 0. - The total number of BDs for the DCI 0_X/1_X for CCs {1, 2, 3, 4} across all the search space sets on CC 0 is capped by 36×4=144 - For each scheduled cell, the number of BDs for DCI format 0_X/1_X is capped by 36. Because the number of scheduled cell associated with scheduling cell is 4, the total number of BDs for the DCI format 0_X/1_X across a set of scheduled cells should be calculated based on 4 scheduled cells. Regarding search space (SS) of DCI format 0_X/1_X, it is up to gNB to determine how to configure search space configuration across a set of scheduled cells. If gNB configures SS of DCI format 0_X/1_X on only one scheduled cell (i.e. CC1), the associated BDs will be counted this scheduled cell CC1. If the gNB configures SS of DCI format 0_X/1_X on CC1 and CC2, the BDs configured on CC1 will be counted to CC1, the BDs configured on CC2 will be counted to CC2. In this way, it can provide gNB flexibly to configure/indicate the BDs of DCI format 0_X/1_X across a set of scheduled cells. |
| vivo | The wording ‘a set of cells which is configured for multi-cell scheduling’ needs clarification. Is it referring to the original wording ‘a set of cells which can be co-scheduled by a DCI format 0_X/1_X’ in Proposal 2-9 in last meeting or to a combination of cells scheduled by a mc-DCI? It has been clarified in the last meeting that the original wording refers to all the cells involved in mc-scheduling(or in other words, the entire cell scheduled cells involved in the RRC table discussed in Proposal 3-5). The original alt1 means that same SS(s) of DCI format 0_X/1_X will be configured for all co-scheduled cell combinations. If the wording still refers to the same meaning as ‘a set of cells which can be co-scheduled by a DCI format 0_X/1_X’, we suggest to align the wording with the previous agreement and proposal. Besides, the current proposal is a mix of Alt. 1, 2 and 3, if BD/CCE is anyway counted on a predefined scheduled cell, it does not matter whether linkage is built on one cell or multiple cells or each cell, we don’t see a clear benefit to allow such flexibility. |
| LGE | We can understand the intention of the FL proposal, but, with the proposed gNB’s configurations, it is not clear to us what the number of PDCCH candidates on the scheduling cell is. As FL commented above, the legacy search space linkage should be followed when designing the search space configuration for multi-cell scheduling DCI. In this case, the number of PDCCH candidates on the scheduling cell should be from the configured value(s) for the scheduled cell(s). So, the most important thing to discuss here is how to determine the number of PDCCH candidates on the scheduling cell. Suppose the gNB configures a SS of DCI 0_X/1_X for only one of co-scheduled cells, the corresponding number of PDCCH candidates on the scheduling cell can be determined by the SS configured for the scheduled cell. However, if the gNB configures SSs for each co-scheduled cell, how to determine the number of PDCCH candidates on scheduling cell can be an another issue. In this perspective, we don’t see the necessity of giving such full flexibility to gNB. Simply, a SS can be configured for one of co-scheduled cells and is linked with a SS on the corresponding scheduling cell. Alternatively, SS can be configured for each co-scheduled cell (with linkage) while the number of PDCCH candidates on the scheduling cell can be determined as the sum of values in the configured SS on each co-scheduled cell. One more thing to clarify is whether “a set of cells” in the proposal means the entire set of cells which can be scheduled by DCI 0_X/1_X. In our view, with interpretation of “a set of cells” in Proposal 2-5 as the entire set, SS configuration on each cell of all the set of entire cells can be well matched to Proposal 2-6. |
| Samsung | The purpose of search space linking is for the UE to identify search space sets on the scheduling cell for monitoring PDCCH candidates for a scheduled cell. Any benefit from merging Alt1/2/3 need to be determined and compared against a corresponding complexity. The direct solution of using explicit RRC configuration (Alt-4) is clean and avoids all unnecessary complications. |
| Spreadtrum | Actually, we also think the SS can be configured on one/subset/all scheduled cells, but not with same configuration/CIF values. If SS are configured on more than one cells, the CIF can be different to have different CCE indexes for PDCCH candidates. For a set of cells {CC1, CC2, CC3, CC4}, co-scheduled cell combinations include {CC1, CC2} and {CC3, CC4}. For the first combination, it uses the CCE indexes determined by CIF=0, while CIF=1 for the second. Since the CCE indexes and BDs are separate for those CIF related search space set, it can be configured on more than one cells. |
| NTT DOCOMO | We understand that it is up to gNB whether the search space of DCI format 0_X/1_X can be configured on one, a subset, or all cell(s) of the set of cells and we are fine to support this proposal. |
| Ericsson1 | Not OK. This seems unnecessarily complex. What extra functionality (e.g. more BDs/CCEs/scheduling) is achieved by having all alternatives in spec rather than selecting one alternative, e.g. Alt 1? - Alt 1: SS configured on one cell of the set of cells - Alt 2: SS configured on a subset of cells - Alt 3: SS configured on all cells of the set |
| Qualcomm | Following CATT’s statement is exactly what we have a concern. Suppose a UE monitors DCI 0_X/1_X on CC 0 for CCs {1, 2, 3, 4}. Suppose monitoring DCI 0_X/1_X on CC 0 is configured on search space sets associated to all of the CCs {1, 2, 3, 4}. Suppose the scheduling CC uses SCS 30kHz, in which case per-CC BD budget is 36 per slot. The UE monitors DCI 0_X/1_X on various SS sets on CC 0. - The total number of BDs for the DCI 0_X/1_X for CCs {1, 2, 3, 4} across all the search space sets on CC 0 is capped by 36×4=144 - For each scheduled cell, the number of BDs for DCI format 0_X/1_X is capped by 36. Because the number of scheduled cell associated with scheduling cell is 4, the total number of BDs for the DCI format 0_X/1_X across a set of scheduled cells should be calculated based on 4 scheduled cells. It is true that the UE anyway need to be able to monitor 144 BDs per slot in the above example. However, this does not mean that 144 BDs per slot have to be available for one scheduled CC. This requires same complexity increase as to support search space sharing. In Rel-15, more than 36 BDs are available for a scheduled CC only if the UE supports search space sharing (for DL and/or UL), which is based an optional UE capability. For example, if there are 36 BDs for each scheduled CC and if the UE supports search space sharing and if all conditions are met, the DCI for a scheduled CC can be transmitted on any of the 144 candidates. Same as this, we would like to emphasize that, at least a basic UE is allowed to process up to 36 BDs per slot for a grant for a scheduled CC in the above example. In principle, the feature should be beneficial for both NW and UE. The CATT’s proposal above is beneficial for NW but not for UE. From UE point of view, there is no benefit/incentive to support MC-scheduling in this case. |
| Apple2 | Thanks, moderator, for the clarification. Based on the clarification, if configuring search space on just one cell have certain issues in case of DRX for that cell, as pointed out, then we can consider Alt 1 as well, if Alt 4 is not preferred by majority. This doesn’t have the issues pointed out for single cell configured with search space for DCI format 0_X/1_X. However, combining the three alternatives is still not justifiable and adds to unnecessary complexity/optimization |
| Moderator2 | @CMCC: Yes, the scheduled cell is unknow before UE detects the DCI. Here, the proposal is just focused on which cell the search space of DCI format 0-X/1_X is configured on. It is different issue. @Fujitsu: Understand your concern. Please check the updated proposal in 2nd round where I intend to focus on a single cell case first. @vivo: Yes, your understanding is correct. ‘a set of cells which is configured for multi-cell scheduling’ implies the entire set of cells. As commented by one company in previous meeting, they think the original wording may be not quite clear since it may cover the case of a concrete scheduled cell combination because there is “co” used in the wording. So I changed it to “a set of cells which is configured for multi-cell scheduling”. With this clarification, is it OK to use the new wording? @All: based on the inputs, let’s focus on one cell case first. Please kindly check the update as below. #### Proposal 2-5rev: - For a set of cells which is configured for multi-cell scheduling, search space of DCI format 0_X/1_X can be configured on at least one cell of the set of cells and associated with the search space on the scheduling cell with the same search space ID: - Note: It is up to gNB to determine which cell of the set of cells the search space of the DCI format 0_X/1_X is configured on. |
| Nokia / NSB | Thanks for the update. But the wording with the proposal may not be fully clear now as ‘at least one cell’ still includes all 3 cases (i) one cell, (ii) subset and (iii) all cells. If the intention is for now to agree for (i) one cell – and continue the discussion on (ii) subset and (iii) all cells, then it may be more correct to say: #### Proposal 2-5rev Mod: - For a set of cells which is configured for multi-cell scheduling, search space of DCI format 0_X/1_X can be configured on at least one cell of the set of cells and associated with the search space on the scheduling cell with the same search space ID: - Note: It is up to gNB to determine which cell of the set of cells the search space of the DCI format 0_X/1_X is configured on. - FFS: It the search space of DCI format 0_X/1_X can be in addition configured on a subset or all cells of the set of cells. |
| Apple | In our view, we think companies have not good understanding on each of the alternatives now, and considering the limited time, we think that we should agree on the alternatives now and do not consider further alternatives anymore. On the proposal itself, we have two updates. First update is like what Nokia suggested i.e., to remove “at least” from the main bullet. Second update is related to which cell is the search space configured on. In our view, the most straightforward method is to use scheduling cell if it is one of the cell among the set of cells, otherwise, gNB can configure which cell. #### Proposal 2-5rev: - For a set of cells which is configured for multi-cell scheduling, search space of DCI format 0_X/1_X can be configured on at least one cell of the set of cells and associated with the search space on the scheduling cell with the same search space ID: - When scheduling cell is one of the cells of the set of cells, then scheduling cell is configured with the search space of DCI format 0_X/1_X - Otherwise, Note: It it is up to gNB to determine which cell of the set of cells the search space of the DCI format 0_X/1_X is configured on. |
| Qualcomm | Thanks, moderator, for accommodating our concern. The proposal is now much clear and better. We are OK with the moderator’s direction. For now, we prefer to leave the consensus as a working assumption. The reason why we prefer to keep it as the WA is that this may relates to the other discussions such as nCI-to-cell-set association, how to count/cap BD/CCE numbers, etc. If companies want to consider possibility of more than one cell, we are also fine to have the FFS proposed by Nokia. Regarding Apple’s addition “When scheduling cell is one of the cells of the set of cells, then scheduling cell is configured with the search space of DCI format 0_X/1_X”, we agree the proposal is straightforward but not sure if this configuration restriction avoids UE complexity increase. In summary, we are OK with Nokia’s proposal as a working assumption. |
| Moderator3 | @Nokia: Thanks for the good comments. I captured in below update. @Apple: Since you are OK with a single cell case, I suggest FFS which cell of the set of cells the search space of the DCI format 0_X/1_X is configured on so that we can further determine whether this cell is a scheduling cell if the scheduling cell is included in the set of scheduled cells or one of scheduled cells. @Qualcomm: OK to put WA for this proposal and add FFS from Nokia. #### Proposal 2-5rev2 (for working assumption): - For a set of cells which is configured for multi-cell scheduling, search space of DCI format 0_X/1_X can be configured on one cell of the set of cells and associated with the search space on the scheduling cell with the same search space ID: - FFS: which cell of the set of cells the search space of the DCI format 0_X/1_X is configured on. - FFS: whether the search space of DCI format 0_X/1_X can be additionally configured on other cells of the set of cells. |
| Intel | We are fine with the updated proposal for working assumption. We prefer only one cell of the set of cells for search space of DCI format 0_X and 1_X |
| CATT | Thanks for the update. If most of companies think it’s better to focus on ‘one cell’ case first, we are OK with the updated proposal in principle. We agree that it can up to gNB to determine which cell of the sets the search space of DCI format 0_X/1_X is configured on. This is the simplest method without requiring any additional signalling or spec impact. In additional, for the case if SS is only configured on one scheduled cell, when the cell is deactivated or DRX off, then multi-cell scheduling can’t work. We agree to add a FFS as FL’s version. #### Proposal 2-5rev (Modification): - For a set of cells which is configured for multi-cell scheduling, search space of DCI format 0_X/1_X can be configured on one cell of the set of cells and associated with the search space on the scheduling cell with the same search space ID: - Note: FFS: It is up to gNB to determine which cell of the set of cells the search space of the DCI format 0_X/1_X is configured on. - FFS: whether the search space of DCI format 0_X/1_X can be additionally configured on other cells of the set of cells. |
| LGE2 | We prefer the first revised version by FL as it is, i.e. Proposal 2-5rev, rather than other suggestions by companies. #### Proposal 2-5rev: - For a set of cells which is configured for multi-cell scheduling, search space of DCI format 0_X/1_X can be configured on at least one cell of the set of cells and associated with the search space on the scheduling cell with the same search space ID: - Note: It is up to gNB to determine which cell of the set of cells the search space of the DCI format 0_X/1_X is configured on. We think this can be a good compromised version considering Alt 1/2/3/4 in “Proposal 2-8rev2” which is the last FL proposal in RAN1#110. |
| Samsung2 | If Alt-4 is not preferred by majority, we prefer Alt-1 which is a direct extension of the Rel-17 search space linking framework. We also suggest to add an FFS point to further discuss the interaction with single-cell DCI formats. #### Proposal 2-5rev2 (for working assumption): - For a set of cells which is configured for multi-cell scheduling, search space of DCI format 0_X/1_X can be configured on one all cells of the set of cells and associated with the search space on the scheduling cell with the same search space ID: - FFS: which cell of the set of cells the search space of the DCI format 0_X/1_X is configured on. - FFS: whether the search space of DCI format 0_X/1_X can be additionally configured on other cells of the set of cells. - FFS: whether the search space of DCI format 0_X/1_X can be configured to also monitor legacy single-cell scheduling DCI formats. @Apple: if search space set for DCI format 0_X/1_X is configured only on the scheduling cell (and there is no other IE present to indicate a set of co-scheduled cells – as in Alt-4), then all search space sets configured on the scheduling cell for monitoring DCI format 0_X/1_X will be associated with all sets of co-scheduled cells that include the scheduling cell as one of the co-scheduled cell – which does not seem to be an intended behaviour. The purpose of search space linking is to enable the gNB to select which search space set is associated with which set of co-scheduled cells. |
| Langbo | We also prefer the original version or the first revised version by FL and think this is a good compromise till now. |
| ZTE | Do not support Proposal 2-5rev2 and support original version. Based on Qualcomm reply to CATT, although the DCI for a scheduled CC can be transmitted on any of the 144 candidates, the maximum BD is still 36 per scheduled cell and no complexity increased. Due to the search space sharing in current spec of SC-DCI, the most important condition is that the DCI size of a DCI format for scheduling different cells are same. Then the DCI scheduling different cells can be differentiated by CIF value. And the BD/CCE counting is not increased for this case. Now for the Rel-18 MC-DCI, the size of MC-DCI is a single value regardless of scheduling different sub-set of cells(e.g. cell#1&2, or cell#1&2&3) of a set of cells(e.g. cell#0&1&2&3). The MC-DCI can be sent in the USS configured on one or sub-set or all the set of cells. Furthermore, MC-DCI is used to schedule multiple scheduled cells, it is very restricted that search space of DCI format 0_X/1_X can be configured on one cell of the set of cells. |
| NTT DOCOMO2 | We are fine with the updated proposal by moderator as working assumption. For the cell of the set of cells the search space of the DCI format 0_X/1_X is configured on, we are fine to keep it as FFS. |
| Fujitsu | It is not clear to us whether Alt 4 is allowed by the proposal. If we will only focus on Alt 1-3, could we say, “For a set of cells which can be scheduled by DCI format 0_X/1_X”? Among Alt 1-3, we prefer Alt 1 and can accept Alt 3. |
| MTK | Similar view as QC, we are OK with Nokia’s proposal as a working assumption. Apple’s addition “When scheduling cell is one of the cells of the set of cells, then scheduling cell is configured with the search space of DCI format 0_X/1_X” sounds interesting and can be further discussed/checked (ex. listed in FFS). |
| CMCC | We are not fine with the updated proposal. Alt 1 is a straightforward way, and we prefer at least to consider the original proposal 2-5. |
| Moderator4 | @All: We don’t have sufficient TU and we need compromise. Original proposal can cover all the cases which I see a compromise. Let’s leave the detailed configuration to gNB. #### Proposal 2-5: - For a set of cells which is configured for multi-cell scheduling, search space of DCI format 0_X/1_X can be configured on one, a subset, or all cells of the set of cells and associated with the search space on the scheduling cell with the same search space ID: - Note: It is up to gNB to determine which cell(s) of the set of cells the search space of the DCI format 0_X/1_X is configured on. |
| Apple | We cannot accept the proposal as we already commented that combining three alternatives makes it much more complex and the benefit is not justified. From our perspective, Alt 4 is first preference, but for the sake of compromise, we can agree on Alt 3 or Alt 1 as well. But we cannot accept the combination of the three alternatives. Previous version of moderator’s proposal (Proposal 2-5rev2) as WA is also acceptable to us. |
| Qualcomm | Previous version of moderator’s proposal (Proposal 2-5rev2) as WA is good for us. |
| Moderator5 | @All: Thanks for the valuable inputs. Let’s keep “one or multiple cell” for further discussion. Please also check P2-7rev3 as both are relevant. Please note this is for WA. #### Proposal 2-5rev3 (for working assumption): - For a set of cells which is configured for multi-cell scheduling, search space of DCI format 0_X/1_X can be configured on one or multiple cells of the set of cells and associated with the search space on the scheduling cell with the same search space ID: - Note: It is up to gNB which cell(s) of the set of cells the search space of the DCI format 0_X/1_X is configured on. |

#### Proposal 2-6:

- For monitoring PDCCH candidates for a set of cells which is configured for multi-cell scheduling, the n_CI in the search space equation is determined by a value configured for the set of cells.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| MTK | We can live with Proposal 2-6. |
| Qualcomm | We support the proposal. |
| Nokia, NSB | Support |
| Apple | Support |
| Langbo | Support |
| Huawei, HiSilicon | Support |
| ZTE | Not support. In Proposal 2-5, the search space can be configured on one, a sub-set, or all the co-scheduled cells and associated with the search space configured in the scheduling cell. If n_CI is determined by a value configured for the set of cells, configuring search space on more than one scheduled cells is meaningless in case these search spaces have the same search space ID since the CCE index are obtained for these scheduled cells. The other issue is how to determine the BC/CCE budget. In case the configured n_CI is not equal to any n_CI for the co-scheduled cell for legacy scheduling, which cell is the BD/CCE budget counted in? In our understanding, the best way is to reuse the current mechanism. It is very simple. Since the search space of DCI format 0_X/1_X can be configured on one, a subset, or all cells of the set of cells and associated with the search space on the scheduling cell with the same search space ID, there is no additional issue for determine the CCE for the USS of a scheduled cell based on its CIF value, regardless of DCI formats. The updated proposal 2-6 is listed below. #### Proposal 2-6(update): For monitoring PDCCH candidates for a set of cells which is configured for multi-cell scheduling, the n_CI in the search space equation for each cell is determined by a CIF value configured for the set of each cell |
| Moderator | @ZTE: As mentioned in above summary, if the n_CI is determined by a value configured for each cell or cell combination within the set of cells, it will lead to different CCE indices of PDCCH candidates for monitoring DCI format 0_X/1_X. Alt 1 is straightforward to configure a single value for the entire set of co-schedulable cells so that there is only one CCE index for monitoring DCI format 0_X/1_X. Upon detection of the DCI format 0_X/1_X, UE can know which cells are scheduled by checking the indicator of co-scheduled cells. |
| CMCC | We support Alt 3. We don‘t see the issue that Alt 2/Alt 3 will cause different CCE indices of PDCCH candidates for monitoring DCI format 0_X/1_X. Compared with legacy one-to-one DCI scheduling, the total monitored PDCCH candidates in Alt 2 is the same for the scheduled cells. Furthermore, compared with Alt 2, Alt3 can reduce the monitored PDCCH candidates and CCE indices which are smaller than legacy one-to-one scheduling. In addition, Alt 3 have other benefits: 1) Make use of CCE resources more efficiently to reduce the PDCCH blocking probability compared with Alt 1. 2) Reduce the co-scheduled cell indicator filed bit length, as the working assumption in Proposal 2-1, the maximum co-scheduled cells number is 4, and total X combinations may use at most 4 bits information. If one n_CI value can be determined for several combinations of cells, the co-scheduled cell indicator filed bit length can be reduced. In a summary, Alt 3 is a balance between Alt 1 and Alt 2. |
| New H3C | Support |
| Fujitsu | Support |
| CATT | Support FL’s proposal. |
| Vivo | Similar comments as P2-5 If ‘a set of cells which is configured for multi-cell scheduling’ refers to entire set of co-schedulable cell, we can accept the proposal and prefer to reuse the previous wording. |
| LGE | Conditionally support Proposal 2-6. Need to clarify the meaning of “a set of cells” in Proposal 2-5 and Proposal 2-6 assuming both proposals are based on the same meaning of “a set of cell”. If “a set of cells” is the entire set of all cells which can be scheduled by DCI 0_X/1_X for both SS configurations (in Proposal 2-5) and n_CI determination (in Proposal 2-6), the above Proposal 2-6 is understood as Alt-1. On the other hand, if “a set of cells” means each combination of co-scheduled cells, for both SS configurations (in Proposal 2-5) and n_CI determination (in Proposal 2-6), the above Proposal 2-6 is understood as Alt-2. |
| Samsung | OK with the proposal. Based on the FL proposal from RAN1#110 (“combination of co-scheduled cells within the set of cells” in Alt2/3), some companies may want to consider sharing a same n_CI for different sets (or subsets/combinations) of cells. Although we do not support such sharing, it is an issue that may need to be resolved and we suggest to add the following FFS. FFS: whether a same n_CI value can be configured for different sets of cells that are configured for multi-cell scheduling. |
| Spreadtrum | We agree with CMCC, Alt 3 can provide more PDCCH locations and flexible than Alt 1.Different CCE indexes for PDCCH candidates do not have any issues if BD/CCE limits are satisfied, which is also works when cross carrier scheduling applies. Alt 3 can fallback to Alt 1, when there is all the cell combinations are associated with one CIF value. So gNB can configure to use Alt 1 or Alt 3. |
| Xiaomi | Our preference is to follow the legacy cross carrier scheduling framework. In the legacy design, the n_CI value is given by the CIF in the scheduling DCI. The proposal here seems to break the legacy design principle. Meanwhile, we don’t see an major issue to have different CCE index indices of PDCCH candidates for monitoring DCI format 0_X/1_X as along as the BD/CCE limits is maintained. |
| NTT DOCOMO | We can live with this proposal but tend to agree with ZTE. In the current specification for single cell cross-carrier scheduling, the CCE indices would be different depending on the scheduled cell. In that sense, it is reasonable to follow the mechanism that the n_CI can be determined based on the cell combination within the set of cells. It is unclear for us what is the concern if the CCE indices are different for each cell combinations within the set of cell. |
| Ericsson1 | OK. |
| Qualcomm | We think adding Samsung’s FFS can address the concern. |
| Moderator2 | @vivo: Yes. Please check the reason I replied to you under P2-5. @LGE: Here, “a set of cells which is configured for multi-cell scheduling” means the entire set not a concrete scheduled cell combination. @Samsung @Qualcomm: If a same n_CI value is configured for different sets of cells that are configured for multi-cell scheduling, with same CCE index I have concern on how UE determines the scheduled cells since differed set of cells may have different cell combinations. Let’s focus on the proposal firstly. @CMCC @Spreadtrum: I am not sure about the flexibility of providing more PDCCH locations. What’s the drawback of using a single PDCCH location? @xiaomi @NTT DOCOMO: In legacy, there is one-to-one scheduling. Now, we are talking about one-to-many. I don’t think there is drawback of using one CCE index for the set of cells. Although BD budget is maintained, I see the benefit of only one CCE location for UE complexity reduction. |
| Qualcomm | Thanks, moderator, for the response. We have interpreted Samsung’s FFS as opposite, i.e., it was considered as “FFS: if it is possible to configure multiple n_CI values for a set of cells and for one of the set of cells”. For example, if one n_CI value is configured to cells {1, 2, 3, 4}, the FFS is whether to allow another n_CI value to cell {1}, another n_CI value to cell {2}, and so on. We agree with moderator’s response if the moderator’s interpretation is correct. In short, we continue to support the original Proposal 2-6. |
| Moderator3 | @Qualcomm: If I understand your question correctly, it seems a n_CI value is configured for each combination of co-scheduled cells. Index; Scheduled cell combination; N_CI value. 0; Cell 1; X0. 1; Cell 2; X1. 2; Cell 1 + Cell 2; X2. 3; Cell 1 + Cell 2 + Cell 3 + Cell 4; X3. …. This configuration is similar to Alt 2. - Alt 2: the n_CI in the search space equation is determined by a value configured for each combination of co-scheduled cells within the set of cells. If scheduled cell combination can include single cell case, it is OK to use a single n_CI value for entire set of cells. Thanks for supporting original Proposal 2-6. |
| Intel | We are fine with the proposal. |
| LGE2 | Based on FL friendly response to our comments, we support Proposal 2-6. More details commented by companies (incl. FFS suggested by Samsung) can be further discussed. |
| Vivo2 | Based on FL friendly clarification on proposal2-8, there would be a single size for DL/UL mc-DCI for all cell combinations by RRC configuration or UE determination, thus there is no need to have multiple n_CIF values for different combinations, we support this proposal |
| OPPO | We are fine with the proposal. Meanwhile, a clarification is needed whether the search space of DCI format 0-X/1-X could also be used for monitoring legacy DCI format, which drives the n_CI for DCI format 0-X/1-X to be either same or different from the n_CI for legacy DCI format. This relates to the determination of the number of PDCCH candidate for DCI format 0-X/1-X and legacy DCI: either double of nrofCandidates or equal to nrofCandidates. |
| Samsung2 | Thanks to Moderator for the response. It seems that the terminology “set of cells which is configured for multi-cell scheduling” may cause a functionality issue. If a DCI format 0_X/1_X can indicate a first set {cell#1, cell#2} or a second set {cell#2, cell#3} or a third set {cell#1, cell#2, cell#3}, the UE can apply separate n_CI values for the first/second/third sets of cells. This is beneficial since different DCI formats for different sets of co-scheduled cells can correspond to different system parameters. In addition, separate n_CI values enable the UE to know the likely set of co-scheduled cells *before* decoding DCI format 0_X/1_X – this is a principle to help UE implementation for PDCCH monitoring since Rel-15, which will be lost if a same n_CI value is shared for the first/second/third set of cells. The n_CI values are same as the indicator values in DCI format 0_X/1_X for the first/second/third sets of cells, per FL Proposal 3-5. The fact that the UE can be configured a “super set” of cells with multi-cell scheduling configuration (e.g., the third cell {cell#1, cell#2, cell#3} or even a fourth set {cell#1, cell#2, cell#3, cell#4}) has little/no visibility in the specifications. The spec will be mainly/only concerned with sets of cells that can be actually indicated by the DCI format 0_X/1_X, as considered in Proposal 3-5. To further clarify our intention, we suggest to add the following two FFS points: - FFS: whether a same n_CI value can be configured for different sets of co-scheduled cells that can be indicated by a DCI format 0_X/1_X. - FFS: whether an indicator for co-scheduled cells in DCI format 0_X/1_X (which points to one row of a table defining combinations of co-scheduled cells) can be different from an n_CI value configured for the co-scheduled cells. @QC: your interpretation of our proposed FFS was correct. We have modified our FFS points to make it clearer. |
| ZTE | Still not supported. “As mentioned in above summary, if the n_CI is determined by a value configured for each cell or cell combination within the set of cells, it will lead to different CCE indices of PDCCH candidates for monitoring DCI format 0_X/1_X. ” We do not think this is an issue. Because in current spec, different CCE indices of the USS for different scheduled cells are already here and the resources are located on the scheduling cell. That is, - In case the search space of DCI format 0_X/1_X can be configured on one, a subset, or all cells of the set of cells, the CIF of one, a subset, or all cells of the set of cells are used to determine the CCE indices according to current spec. Then different CCE indices of PDCCH candidates for monitoring DCI format 0_X/1_X are derived due to the different USS of each scheduled cells. - In case the search space of DCI format 0_X/1_X can be configured on only one cell of the set of cells, the CIF of the cell of the set of cells are used to determine the CCE indices according to current spec. Then same CCE indices of PDCCH candidates for monitoring DCI format 0_X/1_X are derived due to only one CIF of the cell are used to determine the USS. Then, we do not think Alt 1 is the straightforward way. And since proposal 2-5 is still controversial, the following updated proposal 2-6 is given below. Proposal 2-6(update): For monitoring PDCCH candidates for a set of cells which is configured for multi-cell scheduling, the n_CI in the search space equation for a cell of the set of cells configured with the search space of DCI format 0_X/1_X and associated with the search space on the scheduling cell with the same search space ID is determined by a CIF value configured for the set of the cell |
| NTT DOCOMO2 | Thank FL for your reply and we can live with proposal 2-6. We have one clarification question for single cell cross-carrier scheduling case. Even if the scheduled cell is the same, the n_CI value for single cell cross-carrier scheduling can be different depending on the DCI format, i.e., legacy DCI format or DCI format 0_X/1_X, is it correct? |
| Fujitsu | According to the discussions above, some companies might interpret the proposal as Alt 2. If the proposal is support Alt 1, similar to Proposal 2-5, could we say, “for a set of cells which can be scheduled by DCI format 0_X/1_X” to avoid misleading? |
| MTK | We want to clarify is the original Proposal 2-6 equivalent to the following (in brown) mentioned example in Moderator 3? A n_CI value is configured for each combination of co-scheduled cells. Index; Scheduled cell combination; N_CI value. 0; Cell 1; X0. 1; Cell 2; X1. 2; Cell 1 + Cell 2; X2. 3; Cell 1 + Cell 2 + Cell 3 + Cell 4; X3. …. We think the answer is yes. Then can we add a note to Proposal 2-6 as below to make it more clear? - Note: A n_CI value is configured for each combination of co-scheduled cells. |
| CMCC | In our view, Alt 3 is a balance between Alt 1 and Alt 2 to map CCE index and PDCCH candidates, which can also include Alt 1 as the n_CI value can be determined by pre-definetion or network configuration. But if we further discuss the indication of co-scheduled cells, Alt 3 can provide additional benefits on reducing signalling overhead of the indicator in DCI. |
| Moderator4 | @MTK: the intention of the proposal is to configure a single value for all cells, e.g., {cell 1 to 4} instead of each combination. @Companies which don’t support the proposal: Could you please clarify the drawback of a single value for entire set of cells? @All: Since majority companies support this proposal, we tend to keep it and continue the discussion in 2nd round. |

### 2nd round of discussions

#### Proposal 2-5rev3(for working assumption):

- For a set of cells which is configured for multi-cell scheduling, search space of DCI format 0_X/1_X can be configured on one or more cells of the set of cells and associated with the search space on the scheduling cell with the same search space ID:

- Note: It is up to gNB which cell(s) of the set of cells the search space of the DCI format 0_X/1_X is configured on.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | OK in principal |
| Qualcomm | We disagree with ZTE on the following statement: “Based on Qualcomm reply to CATT, although the DCI for a scheduled CC can be transmitted on any of the 144 candidates, the maximum BD is still 36 per scheduled cell and no complexity increased. Due to the search space sharing in current spec of SC-DCI, the most important condition is that the DCI size of a DCI format for scheduling different cells are same. Then the DCI scheduling different cells can be differentiated by CIF value. And the BD/CCE counting is not increased for this case. Now for the Rel-18 MC-DCI, the size of MC-DCI is a single value regardless of scheduling different sub-set of cells(e.g. cell#1&2, or cell#1&2&3) of a set of cells(e.g. cell#0&1&2&3). The MC-DCI can be sent in the USS configured on one or sub-set or all the set of cells. Furthermore, MC-DCI is used to schedule multiple scheduled cells, it is very restricted that search space of DCI format 0_X/1_X can be configured on one cell of the set of cells.” We do not understand why a DCI for a scheduled CC can be transmitted on any of the 144 candidates does not increase the complexity. The UE cannot identify whether or not there is a scheduled data to process on the cell until all 144 BDs are completed, as opposed to the current spec without search space sharing, in which case the UE can identify whether or not there is a scheduled data to process on the cell once 36 BDs are completed. In the same example, channel estimation is terrible – UE needs to process 224 CCEs for channel estimation (for 144 BDs) to see if there is a scheduled data to process on a cell. Moreover, we do not quite understand why “MC-DCI is used to schedule multiple scheduled cells, it is very restricted that search space of DCI format 0_X/1_X can be configured on one cell of the set of cells”. Within the set of PDCCH candidates for the search space set(s) for DCI format 0_X/1_X, the NW transmits either legacy DCI format for single-cell scheduling, or DCI 0_X/1_X for multi-cell scheduling. We do not see any restriction or constraint for network. Considering the limited time until the close of this work item (and to respect moderator’seffort), we can try to compromise. Following should be fine for us. #### Proposal 2-5rev3(for working assumption) – QC update: - For a set of cells which is configured for multi-cell scheduling, search space of DCI format 0_X/1_X can be configured on one or more cells of the set of cells and associated with the search space on the scheduling cell with the same search space ID: - Note: It is up to UE capability and gNB configuration on how many cell(s) and/or which cell(s) of the set of cells the search space of the DCI format 0_X/1_X can be configured on. The minimum value of the UE capability for the number of cell(s) is 1. |
| Fujitsu | For clarification, does it mean all 4 alternatives are in the scope of the working assumption? If so, it seems to make things more complicated, because for each alternative, we need to further discuss potential issues and standard impacts. Considering the limited TU, we don’t think we should do so. As we observe Alt.1 and Alt.3 are the majority view, we prefer to make down-selection between them. |
| Langbo | Although we think Alt.1 is the most straightforward way with minimum specification impact, we can live with this compromised proposal. |
| LGE2 | Support Proposal 2-5rev3(for working assumption) by Moderator. In addition, we don’t think it is needed to discuss the UE capability here. |
| NTT DOCOMO | Support. |
| Moderator | @Qualcomm: With the concern from LG, can we not mention UE capability and discuss it in UE feature? @Fujitsu: To me, this is one possible compromise. Leave it to gNB configuration can avoid the discussion on down-selection between Alt 1 and Alt 3. |
| Xiaomi | We also don’t want to have the UE capability here. |
| Qualcomm | Not OK to postpone it. Combining all the proposals, a UE is required to support more than per-CC BD limit foreach scheduled cell, if no capability aspect is included in the proposal. Therefore, we are not OK to delete basic UE capability aspect from here. |
| Apple | We don’t support this version of the proposal due to the reasons commented earlier. We prefer version Proposal 2-5rev2. We prefer to agree either 1 or all cells that can be potentially scheduled rather than supporting the combination |
| ZTE | We support Proposal 2-5rev3. For the comments from Qualcomm, We do not understand why a DCI for a scheduled CC can be transmitted on any of the 144 candidates does not increase the complexity. The UE cannot identify whether or not there is a scheduled data to process on the cell until all 144 BDs are completed, as opposed to the current spec without search space sharing, in which case the UE can identify whether or not there is a scheduled data to process on the cell once 36 BDs are completed. In the same example, channel estimation is terrible – UE needs to process 224 CCEs for channel estimation (for 144 BDs) to see if there is a scheduled data to process on a cell. Below is our understanding. For legacy SC-DCI scheduling, assume there are four cell#0/1/2/3 scheduled by cell#0, the DCI for scheduling each cell will be sent in each USS on the scheduling cell and corresponding to each CIF. This will also result in the maximum 144BDs (36*4). For Rel-18 multi-cell scheduling, the motivation of WID is the reduction of control overhead. That is, the four SC-DCI can be merged in a single MC-DCI, but there is no need to reduce BD/CCE capability or scheduling flexibility. For the search space and corresponding BD/CCE of the MC-DCI, there is no need to also restrict on only one cell, which will lead to lower monitoring capability than legacy CA with SC-DCI. |
| Qualcomm | @ ZTE: Please see our response to Proposal 2-7rev3 on why it increase the complexity. Multi-cell scheduling WID targets improvements on both spectral efficiency and power efficiency. Increasing PDCCH processing burden never improves power efficiency (rather, negative impact on power usage). In any case, again, we do not see why 144 BDs are necessary just to send one grant. |
| MTK | We prefer QC’s updated version. We can understand the benefit of configure the same search space ID on multiple cells, but we think keep a basic workable function as stated by QC is more desired. |
| Huawei2 | For the moment, we can live with the proposal. |
| Vivo3 | same view Fujitsu, merging all alternatives would make the situation worse, as different alternatives leads to different design especially for the BD part. Although our first preference is alt1 ‘all cell’, we can accept alt3‘one cell' or alt4 for progress, but we prefer not to support a combination. |
| CMCC | Although we support Alt 1 as a straightforward way, we are fine with the proposal if majority support it. |
| Intel | Proposal 2-5 and 2-6 are related. Since Proposal 2-6 is to have single n_CI which gets good support, Proposal 2-5 should be to configure SS sets on one and only one cell. Then, the one n_CI (Proposal 2-6) can be used to derive the starting CCE indexes of the one SS set in Proposal 2-5 In summary, we prefer to delete the update ‘or more cells’ |
| CATT | We support the proposal. We share same view with ZTE that the search space of DCI format 0_X/1_X configured on more than one cells will not increase PDCCH monitoring capability of UE. In the current spec, if a UE is configured with N scheduled cells with PDCCH candidates monitored on a scheduling cell, the number of BD/CCE monitored on scheduling cell is capped by the total number of BD/CCE budget across N scheduled cells. For multi-cell scheduling, the UE will monitor PDCCH candidate of DCI format 0_X/1_X for a set of cells on scheduling cell. Thus, the number of PDCCH candidate monitored on scheduling cells should be up to the total number of BD across the set of cells, i.e.144. Note that the number of PDCCH candidate configured on each scheduled cell meet the BD limit of per cell. We think this is in line with current spec and UE capability. In addition, if the search space of DCI format 0_X/1_X is only allowed configuring on one cell, only the PDCCH monitoring capability of this cell can be used, but those of other cells can’t be fully used. This will cause the PDCCH monitoring capability of some of CCs within a set of CC to be wasted. It’s better to allow search space of DCI format 0_X/1_X configured on one or more cells of the set of cells, which can ensure the PDCCH monitoring of each cell in the set of CCs can be used by gNB configuration. |
| Samsung3 | We continue to think Alt-4 is a simple and clean solution, and avoids the various concerns and issues raised by companies for Alt1/2/3. But, for progress, we can be OK in principle with the FL proposal if that is majority view. However, we would like to clarify the following: - the “one or more cells” are different from the scheduling cell, for cases that the scheduling cell is included in the set of cells – otherwise search space linking will not be useful, as described in our comment to Apple in the previous round. - Since the proposal mentions “search space of DCI format 0_X/1_X”, we would like to clarify whether or not the same search space set can be used for monitoring legacy single-cell DCI formats as well (we think it should not). Also, agree with LGE/Xiaomi that UE capability can be discussed separately (OK to leave it as FFS, if strongly preferred so). So, we suggest to add the following two points: #### Proposal 2-5rev3(for working assumption): - For a each set of cells which is configured for multi-cell scheduling, a search space of DCI format 0_X/1_X can be configured on one or more cells of the set of cells and associated with the search space on the scheduling cell with the same search space ID: - Note: It is up to gNB which cell(s) of the set of cells the search space of the DCI format 0_X/1_X is configured on. - Note 2: The cell(s) of the set of cells on which the search space of the DCI format 0_X/1_X is configured does not include the scheduling cell, when the scheduling cell is included in the set of cells. - FFS: whether the search space of DCI format 0_X/1_X can be configured to also monitor legacy single-cell scheduling DCI formats. |
| Moderator | Three changes in rev4 compared with rev3: - “RAN1 spec supports” is added in order to soften the wording. - A second note is added to emphasize per-CC BD/CCE not increased. This note is what we have agreed in last week. - One FFS is added as proposed by Samsung. It needs FFS on the SS configuration of MC DCI and legacy DCI. #### Proposal 2-5 rev4(for working assumption): - For a set of cells which is configured for multi-cell scheduling, RAN1 specification supports configuring search space of DCI format 0_X/1_X on one or more cells of the set of cells and associated with the search space on the scheduling cell with the same search space ID: - Note: It is up to gNB which cell(s) of the set of cells the search space of the DCI format 0_X/1_X is configured on. - Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell. - FFS: whether the search space of DCI format 0_X/1_X can be configured to also monitor legacy single-cell scheduling DCI formats. |
| CATT | We are ok with the updated proposal. Regarding the last FFS, we don’t think the search space of DCI format 0_X/1_X can be used to configure legacy single-cell scheduling DCI formats. The SS of DCI format 0_X/1_X is used to determine PDCCH candidates for a set of cells rather than for only a cell. It’s better to configure DCI format 0_X/1_X and legacy single-cell scheduling DCI formats separately. |
| Apple | As commented earlier, we prefer to include search space configuration for DCI format 0_X/1_X either on one cell or all the cells in the set of cells, but not both options. As commented by Intel, considering single n_CI (with majority support), we prefer to keep only one cell. Also, we don’t support the last FFS However, as a compromise, we can consider proposal 2-5 rev4 with following update: #### Proposal 2-5 rev4(for working assumption): (Apple updates) - For a set of cells which is configured for multi-cell scheduling, RAN1 specification supports configuring search space of DCI format 0_X/1_X on one or more cells of the set of cells and associated with the search space on the scheduling cell with the same search space ID: - It is reported as a UE capability whether one or multiple cells in the set of cells can be configured with search space of DCI format 0_X/1_X - Note: It is up to gNB which cell(s) of the set of cells the search space of the DCI format 0_X/1_X is configured on. - Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell. - FFS: whether the search space of DCI format 0_X/1_X can be configured to also monitor legacy single-cell scheduling DCI formats. |
| Qualcomm | We prefer Apple’s version. |
| LGE | After observing the arguments from the companies related to complexity/implementation in the UE as well as the spec so far, to avoid further/complicated work considering limited TU, we think it would be reasonable to update as below to simplify PDCCH framework for multi-cell scheduling. #### Proposal 2-5 rev4(for working assumption): - For a set of cells which is configured for multi-cell scheduling, RAN1 specification supports configuring search space of DCI format 0_X/1_X on one or more cells of the set of cells and associated with the search space on the scheduling cell with the same search space ID: - Note: It is up to gNB which cell(s) of the set of cells the search space of the DCI format 0_X/1_X is configured on. - Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell. - FFS: whether the search space of DCI format 0_X/1_X can be configured to also monitor legacy single-cell scheduling DCI formats. |
| Nokia/NSB | We think there is some relation to Proposal 2-7 (as also the LGE proposed updates here & in Proposal 2-7) seems to imply. If we split the BD / CCEs based on the PDCCH candidates to the one or more serving cells (in Proposal 2-7), then we would end up also with ‘one or more’ here. If we end up counting this only for a single cell, then having also here a single cell would be sufficient. - Logically better to have Proposal 2-7 figured out first and then see if ‘one’ or ‘one of more’ would be needed here in the end. |
| Ericsson2 | As we mentioned earlier, the proposal to configure “on one or more cells of the set of cells” is unnecessarily complex. Our understanding is that configuring the linked search space on one cell does not increase any UE BD/CCE complexity, and to reflect this, the text in second Note is sufficient. The actual BD/CCE handling would be part of P2-7, and any BD/CCE implications should be applicable regardless of how the SS linking is configured for mc-DCI. |
| Fujitsu | We still think combining all alternatives would make things complicated and should be avoided due to the very limited TU. As for relation between Proposal 2-5 and 2-7, we commented earlier for Proposal 2-7, it is unnecessary to bundle search space configuration issue and DCI size/BD/CCE counting issue. Even if search space configuration is configured on one of the scheduled cells (from RRC signaling perspective), DCI size/BD/CCE can be counted per scheduled cell (on each scheduled cell) of DCI format 0_X/1_X. With that said, we can accept the updated Proposal 2-5 rev4 from LGE and updated Proposal 2-7rev6 from Ericsson. |
| Moderator2 | @All: Proposal 2-7 and Proposal 2-5 have been merged as below for further discussion. So far, the discussion on this proposal is also in e-mail. You are welcome to comment via e-mail. Merged P2-7 and P2-5 rev1: - For a set of cells which is configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - RAN1 specification supports following two options. - Option A: - DCI size of DCI format 0_X/1_X is counted on all the cells among the set of cells. - BD/CCE of the DCI format 0_X/1_X is counted on all the cells in the set of cells. - Search space of DCI format 0_X/1_X is configured on all the cells of the set of cells and associated with the search space of the scheduling cell with the same search space ID. - Option B: - DCI size of DCI format 0_X/1_X is counted on one cell among the set of cells. - FFS which cell DCI size of the DCI format 0_X/1_X is counted on. - BD/CCE of DCI format 0_X/1_X is counted on one cell among the set of cells. - FFS which cell BD/CCE of the DCI format 0_X/1_X is counted on. - Search space of DCI format 0_X/1_X is configured on one cell of the set of cells and associated with the search space of the scheduling cell with the same search space ID. - FFS which cell the SS of the DCI format 0_X/1_X is configured on. - It is subject to UE capability to support one or both of the options. - Note: BD/CCE number per scheduled cell is capped by Rel-17 per-cell limit. - Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell. |

#### Proposal 2-6:

- For monitoring PDCCH candidates for a set of cells which is configured for multi-cell scheduling, the n_CI in the search space equation is determined by a value configured for the set of cells.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | OK in principal |
| Qualcomm | We keep support the proposal. |
| Fujitsu | Fine with the spirit of the proposal. But we have one question similar to Proposal 2-3 and 2-4 rev3, does “the set of cells” include a cell that can be scheduled by a DCI format 0_X/1_X, but only by single-cell scheduling way (i.e. it cannot be simultaneously scheduled with other cells by the DCI format 0_X/1_X)? |
| Langbo | Support |
| LGE2 | Support |
| Nokia/NSB | Support |
| NTT DOCOMO | We are fine with the proposal. Just want to check our understanding on how to avoid the collision of n_CI/CCE indices between multi-cell scheduling by DCI format 0_X/1_X and cross-carrier scheduling by legacy DCI. It may be handled by gNB to configure n_CI value appropriately for each multi-cell scheduling and single cell cross-carrier scheduling, but not sure whether it is common understanding or not. |
| Moderator | @Fujitsu: For each scheduled cell, there is at most one scheduling cell which can use DCI format 0_X/1_X for scheduling the cell. Maybe I misunderstood your point, please correct me. @NTT DOCOMO: Regarding your question, I think gNB can configure different n_CI values for multi-cell scheduling DCI and legacy single cell scheduling DCI to avoid CCE index collision. |
| Xiaomi | Although we still prefer to reuse the legacy design framework as we don’t see an issue with that, e.g, the n_CI is given by the CIF value for a combination of scheduled cells. We can live with the proposal if it is majority’s view. |
| Apple | Support the proposal |
| ZTE | As we commented last time, the n_CI determination is same as legacy, i.e. using the n_CI configured for the legacy cross cell scheduling. For this proposal, first, it is not aligned with Proposal 2-5rev3. When more than one cells are configured with search space linked to the search space of the scheduling cell, the n_CI determined by a value configured for the set of cells is not aligned with the scheduled cells. In legacy scheduling, each cell is configured with n_CI. Therefore, this proposal also does not follow the legacy procedure. Second, the n_CI determined by a value configured for the set of the cells may limit the PDCCH candidates because the first CCE index for all the cells configured with search space for DCI format 0_X/1_X are always the same according to the equation. In other words, the PDCCH candidates for these cells overlaps totally. However, if the using the legacy n_CI, the PDCCH candidate for these cells have the different starting CCE index. This can achieve more PDCCH candidates locations and can provide more flexibility for the network to transit the DCI format 0_X/1_X. Third, a question is the value for the set of cells. If it is equal to the n_CI (for legacy scheduling) for one of the cells configured with search space for DCI format 0_X/1_X, then why not to directly use this n_CI. If it is not equal to the n_CI (for legacy scheduling) of any one of the cells configured with search space for DCI 0_X/1_X, how to determine the BD/CCE counting in this case considering only one size of DCI 0_1/1_X for these cells. It should be noted that, in the legacy procedure, which cell a PDCCH is counted in is related to the n_CI because the scheduled cells have different n_CI values and different locations of PDCCH candidate. But now, if we have only one n_CI for the set of cells, which cell the BD/CCE is counted in is not clear. This issue is not easily resolved as we can see. From our understanding, it can resolve the BD/CCE issue very easily if we use the legacy n_CI. Therefore, we propose the following proposal based on the Proposal 2-5rev3. Note it can cover both cases (one or more cells) in Proposal 2-5rev3. The current proposal is included in the second bullet. Proposal 2-6(updated) For monitoring PDCCH candidates for a set of cells which is configured for multi-cell scheduling, the n_CI in the search space equation for a cell of the set of cells configured with the search space of DCI format 0_X/1_X and associated with the search space on the scheduling cell with the same search space ID is determined by a CIF value configured for the set of the cell - In case the search space of DCI format 0_X/1_X can be configured on one, a subset, or all cells of the set of cells, the CIF of one, a subset, or all cells of the set of cells are used to determine the CCE indices according to current spec. Then different CCE indices of PDCCH candidates for monitoring DCI format 0_X/1_X are derived due to the different USS of each scheduled cells. - In case the search space of DCI format 0_X/1_X can be configured on only one cell of the set of cells, the CIF of the cell of the set of cells are used to determine the CCE indices according to current spec. Then same CCE indices of PDCCH candidates for monitoring DCI format 0_X/1_X are derived due to only one CIF of the cell are used to determine the USS. |
| MTK | Sorry we are still confused. According to moderator: - The intention of the proposal is to configure a single value for all cells, e.g., {cell 1 to 4} instead of each combination. Then it means the following to us: Index; Scheduled cell combination; N_CI value. 0; Cell 1; X0. 1; Cell 2; X0. 2; Cell 1 + Cell 2; X0. 3; Cell 1 + Cell 2 + Cell 3 + Cell 4; X0. …. Then basically there is only one N_CI value used. What is the point of using N_CI value here then? I think I missed something and need some hint. |
| Huawei2 | Support |
| Vivo3 | Support. |
| Intel | We are OK with the proposal |
| CATT | We support the proposal. |
| Moderator | @MTK: The table is just giving an example to you which shows a single value is for all combinations. Let me update the table as below: Scheduled cell combination Index; Scheduled cell combination. 0; Cell 1. 1; Cell 2. 2; Cell 1 + Cell 2. 3; Cell 1 + Cell 2 + Cell 3 + Cell 4. …. There is a single n_CI value configured for monitoring DCI 0_X/1_X for all or subset of {cell 1, cell 2, cell 3, cell 4}. |
| Samsung3 | We think the wording of the proposal “a set of cells which is configured for multi-cell scheduling” is not clear and open to interpretation. Here’s our response to the FL question from the first round of discussions: “@Companies which don’t support the proposal: Could you please clarify the drawback of a single value for entire set of cells?” - Support of separate n_CI values for different sets (or “combinations”, as referred to by some companies) for co-scheduled cells is needed for the same reasons since LTE Rel-10 and NR Rel-15. What is the reason to have a different approach for sets of co-scheduled cells in Rel-18 than for individual cells in Rel-17? - What is the impact on PDCCH blocking based on the proposal (if it is to mean all/union of sets of cells that can be co-scheduled) over using separate n_CI per set of co-scheduled cells? - The gNB may want to use, in a same slot, multiple DCI formats 0_X/1_X for corresponding multiple sets of cells (e.g., because different sets of cells require different channel/scheduling parameters), or even because the maximum number of scheduled cells for a scheduling cell is larger than what can be supported by a given DCI formats 0_X/1_X. If the multiple sets of cells correspond to a same n_CI, there may be a PDCCH blocking issue and randomization of the CCEs for PDCCH candidates using different n_CI values is needed. - If the UE determines a first search space set to be “linked” to a first set of cells, and a second search space set to be “linked” to a second set of cells, is same n_CI is assigned to both first and second sets of cells, in monitoring occasions that belongs to both the first and second search space sets, the UE determines same PDCCH candidates for both set of cells. Such design defeats the purpose of search space linking. - Since Rel-15, separate n_CI values have been used for different scheduled cells to enable the UE to know the likely scheduled cell before decoding the DCI. Same principle should be retained for Rel-18 – different sets of cells that are simultaneously scheduled are no different than different cells that are simultaneously scheduled in Rel-17. In short, the n_CI values are same as the values for the ‘indicator for co-scheduled cells’ in DCI format 0_X/1_X, per FL Proposal 3-5, for each set/combination of cells. We don’t see why these two parameters need to be separately configured. Therefore, we suggest to add the following FFS: #### Proposal 2-6: - For monitoring PDCCH candidates for a set of cells which is configured for multi-cell scheduling, the n_CI in the search space equation is determined by a value configured for the set of cells. - FFS: whether the set of cells is a set of co-scheduled cells configured for indication by a DCI format 0_X/1_X or is the union of all sets of co-scheduled cells |
| MTK | @Moderator: Thanks for further reply. For your mentioned - There is a single n_CI value configured for monitoring DCI 0_X/1_X for all or subset of {cell 1, cell 2, cell 3, cell 4}, - e.g., including {Cell 1}, {Cell 2}, {Cell 1 + Cell 2}, {Cell 1 + Cell 2 + Cell 3 + Cell 4}” Then, in the example above, it basically means all the DCI 0_X/1_X scheduling DCI would share the same nCI value, no matter which cells are scheduled. If this is correct understanding, then sorry we wrongly reported “support” in the first round, we actually support a per-cell-combination nCI. |
| Moderator | Supporting company (14): New H3C, Qualcomm, Langbo, LGE, Nokia, NTT DOCOMO, xiaomi, Apple, Huawei, vivo, Intel, CATT, Lenovo, Ericsson(confirmed in e-mail) ZTE/MTK/Samsung: support n_CI value per scheduled cell combination. Since majority companies support P2-6 and no drawback is identified so far, I intend to keep the proposal for further discussion. One thing is we can propose it for working assumption. Could ZTE/MTK/Samsung accept a WA as below? #### Proposal 2-6(for working assumption): - For monitoring PDCCH candidates for a set of cells which is configured for multi-cell scheduling, the n_CI in the search space equation is determined by a value configured for the set of cells. |
| CATT | We support the proposal. In legacy cross-carrier scheduling, the PDCCH candidates calculated by n_CI are used to monitor DCI format for a single cell, and the n_CI is configured based on per cell. For multi-cell scheduling, the PDCCH candidates calculated by n_CI are used to monitor DCI format for a set of cells, then the n_CI should be configured based on a set of cells configured for multi-cell scheduling. For the n_CI per each co-scheduled cell combination, if 16 co-scheduled cell combinations are configured with UE, the UE requires monitoring 16 times for a configured PDCCH candidate. This is obviously unreasonable. |
| Nokia/NSB | Support 2-6 (for agreement or working assumption) Agree with the comments by CATT. |
| Apple | Although we prefer to take it as an agreement, but would also be fine taking it as WA |
| Qualcomm | We are OK with the proposal. |
| LGE | We are OK with the proposal. |
| Ericsson2 | OK with the proposal for WA. |

## DCI size and BD/CCE budget

Huawei, HiSilicon - Proposal 12: Regarding to the DCI size and BD/CCE, Alt 1-3 (counted for one or more co-scheduled cells configured with PDCCH candidates for multi-cell scheduling) is preferred. ZTE - Proposal 8: BD/CCE counting for multi-cell scheduling DCI within a USS of a scheduled cell will be counted only in the scheduled cell. That is, if there is no same USS id associated with its scheduling cell on one scheduled cell within the group of configured co-scheduled cells, no BD/CCE counting for MC-DCI within the USS for the scheduled cell. - Proposal 9: For a scheduled cell within the group of configured co-scheduled cells configured with a USS comprising MC-DCI, the size of MC-DCI should be counted in the scheduled cell. That is, if there is no same USS id associated with its scheduling cell on one scheduled cell within the group of configured co-scheduled cells, the size of MC-DCI will not be counted in the scheduled cell. - Proposal 10: DCI size budget for a scheduling cell which is also a scheduled cell can be achieved by one of following. - The whole DCI budget can be used for DCI formats with CRC scrambled by C-RNTI - Maintain current DCI size budget and the MC-DCI can be only configured in the USS of one or more scheduled cells which is not a scheduling cell. Spreadtrum Communications - Proposal 6: For further study DCI size budget and BD/CCE budget for multi-cell scheduling DCI, existing DCI size budget is maintained per scheduled cell. Both DCI size and BD/CCE of DCI format 0_X/1_X are counted for one or more cells configured with PDCCH candidates for multi-cell scheduling among the cells that can be potentially scheduled by DCI 0_X/1_X. - Proposal 7: Only one size of DCI 0_X/1_X on one scheduling cell can be monitored. Vivo - Proposal 4. For BD/CCE counting for multi-cell scheduling DCI, alt2 and alt4 are supported. - Proposal 5. For DCI size budget handling for multi-cell scheduling DCI, alt 1-3/2-1/2-5 can be further considered. OPPO: - Proposal 9: Restriction for DCI format configuration can be considered to reduce the total number of different DCI size before DCI size alignment. CATT: - Proposal 3: The number of different DCI sizes for DCI format 0_X/1_X and for legacy DCI format should maintain ‘3+1’ DCI size budget per scheduled cell. - Proposal 8: For DCI size budget and BD/CCE budget for multi-cell scheduling DCI, below Alt 3 of Option 1 should be considered. - Alt 3: Both DCI size and BD/CCE of DCI format 0_X/1_X are counted for one or more cells configured with PDCCH candidates for multi-cell scheduling among the cells that can be potentially scheduled by DCI 0_X/1_X. Lanbo: - Proposal 4: For DCI format 0_X/1_X, PDCCH candidates and non-overlapped CCEs are counted on each co-scheduled cell, i.e., Alt 1 is supported. - Proposal 5: A set of co-scheduled cells are counted together as a single virtual cell for MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ calculation when the scheduling cell of the set of co-scheduled cells is configured with only DCI format 0_X/1_X. Intel： - Proposal 8 - Multiple configurations of the DCI fields can be configured for DCI format 0-X/1-X for flexibility, which may result in different DCI sizes - Proposal 9 - The size of DCI format 0_X/1_X is counted with scaling to each of the cells that can be scheduled by the DCI format 0_X/1_X. - Proposal 10: - If DCI format 0_X/1_X and legacy DCI format(s) simultaneously from a same scheduling cell - Rel-17 PDCCH monitoring limits MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ can be reused - Special handling on BD/CCE counting for MPDCCHtotal,slot,μ, CPDCCHtotal,slot,μ is not needed - The BD/CCE of a PDCCH of DCI format 0_X/1_X is counted with scaling to each of the cells that can be scheduled by the DCI format 0_X/1_X for MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ. - Proposal 11: - If DCI format 0_X/1_X and legacy DCI format(s) simultaneously from different scheduling cells, for a cell that can be scheduled by two scheduling cells with SCS μ1,μ2, - If μ1=μ2=μ for a cell, the UE determines MPDCCHtotal,TU,μ and CPDCCHtotal,TU,μ by counting one cell in the downlink cells with SCS μ. - If μ1≠μ2 for a cell, the UE can count the cell as α cell in the downlink cells with SCS μ1 and as β cell in the downlink cells with SCS μ2. Xiaomi: - Proposal 4: It is up to gNB’s configuration to maintain the DCI size budget. The dropping rule can be defined if needed to ensure the DCI budget of one or multiple co-scheduled cells is maintained. - Proposal 8: BD/CCE budget is counted only in one potentially co-scheduled cell. Lenovo: - Proposal 24: Existing “3+1” DCI size budget is maintained per scheduled cell. - Proposal 25: Both DCI size and BD/CCE of DCI format 0_X/1_X are counted only on one cell configured with search space for monitoring DCI format 0_X/1_X within a set of cells which can be co-scheduled by the DCI format 0_X/1_X . - Proposal 26: The payload size of DCI format 0_X/1_X is configured by RRC. CMCC: - Proposal 4. The DCI size of a DCI format 0_X/1_X should be fixed regardless the number of cells it schedules each time. - Proposal 8. Regarding DCI size budget for multi-cell scheduling DCI, support Alt 1-3: DCI size budget is maintained via DCI size alignment and DCI size budget of multi-cell scheduling DCI is counted only in one scheduled cell. - The scheduled cell can be configured by gNB or pre-defined in spec. - Proposal 9. The number of PDCCH candidates and non-overlapping CCEs corresponding to multi-cell scheduling DCI can be scaled down to each of co-scheduled cell according to the number of co-scheduled cells. FGI： - Proposal 5: DCI size budget of multi-cell scheduling DCI is counted only in one potentially co-scheduled cell. - Proposal 6: BD/CCE of a PDCCH candidate of multi-cell scheduling DCI is counted only in one potentially co-scheduled cell. NEC: - Proposal 2: DCI size budget is maintained via DCI size alignment. Align size of format 0_X/1_X after alignment of DCI format 0_1/1_1. LG: - Proposal #13: BD/CCE for DCI format 0_X/1_X is counted only in one scheduled cell. - Proposal #14: DCI size for DCI format 0_X/1_X is counted only in one scheduled cell. - Proposal #15: Existing DCI size budget should be maintained per scheduled cell, by applying DCI size alignment for DCI format 0_X/1_X to one or each scheduled cell. MTK: - Proposal 2: A UE applies Rel-17 PDCCH monitoring limits (i.e., same MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ as in Rel-17) in a slot and DCI size limit (i.e., “3+1” per scheduled cell) regardless of monitoring legacy DCI formats or DCI formats 0_X/1_X or both in the slot. - Proposal 3: Existing DCI size budget is maintained per scheduled cell regardless of UE is configured with DCI formats 0_X/1_X or not. - Proposal 4: BD/CCE is counted on the scheduled cell regardless of UE is configured with DCI formats 0_X/1_X or not. - Proposal 5: For a multi-cell scheduling DCI, use a pre-defined rule to determine which cell should the corresponding DCI size and BD/CCE counted on. For example: - The corresponding DCI size and BD/CCE is counted on the co-scheduled cell with the lowest ServCellIndex - The corresponding DCI size and BD/CCE is counted on the co-scheduled cell with the highest ServCellIndex - Proposal 6: Or, for a multi-cell scheduling DCI, use a RRC configured or pre-defined table to define which cell should the corresponding DCI size and BD/CCE counted on. For example: Scheduled cells combination for one multi-cell scheduling DCI; Which cell to count the corresponding DCI size and BD/CCE. Cell 1, Cell 2, Cell 3, Cell 4; Cell 1. Cell 2, Cell 3; Cell 2. Cell 2, Cell 3, Cell 4; Cell 3. Cell 1, Cell 3; Cell 3. - Proposal 7: For a multi-cell scheduling DCI, its search space is configured under the cell which the corresponding DCI size and BD/CCE is counted on. - Proposal 8: Or, search spaces for DCI 0_X/1_X are only configured on the scheduling cell, instead of using search space linking. Apple: - Proposal 5: For DCI size and BD/CCE budget with multi-cell scheduling, one of the two alternatives should be supported: - Alt 1: Both DCI size and BD/CCE of DCI format 0_X/1_X are counted on each of the cells that can be potentially scheduled by DCI 0_X/1_X. - No scaling to each co-scheduled cell - Alt 4: Both DCI size and BD/CCE of DCI format 0_X/1_X are counted on the scheduling cell. Samsung: - Proposal 13: The size of the MC-DCI format, for a given scheduling cell, is based on the maximum configured number and the corresponding parameters configurations of co-scheduled cells from the scheduling cell. - Proposal 14: For the “3+1” limit on UE budget for DCI sizes, adopt Alt 1-1a (per-slot variation of “3+1”) or Alt 1-3 (without modifying the Rel-17 procedure for matching DCI sizes - i.e. MC-DCIs need not be considered). - Proposal 15: A UE configured with multi-cell scheduling applies the Rel-17 PDCCH monitoring limits – there is no impact on UE hardware requirements for multi-cell scheduling over single-cell scheduling for a same maximum number of cells. - Proposal 16: For counting of PDCCH candidates and non-overlapping CCEs for multi-cell scheduling, adopt Alt-2 or Alt-3. - Proposal 17: RAN1 to separately discuss DCI size budget and PDCCH monitoring limits/counting based on agreements in RAN1#109-e, while striving for unified solutions, when possible. Ericsson: - Proposal 2: Size of mc-DCI (0_X/1_X) is explicitly configured by higher layers. - Proposal 3: Support independent configuration of mc-DCI for PUSCH and PDSCH. - Proposal 4: For a scheduled cell configured to be scheduled by mc-DCI, the total number of different DCI sizes with C-RNTI that the UE is configured to monitor is no more than 4 for the cell if it is an SCell and is no more than 5 if it is a PCell. - Proposal 5: BD/CCE counting for mc-DCI is based on the legacy Rel-15/16/17 BD/CCE counting mechanism with the following update - a BD/CCE attempt for a mc-DCI format is counted only once for comparison against aggregate MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ limits in numerology buckets. NTT DOCOMO: - Proposal 8: If the existing “3+1” DCI size budget is maintained, the subset of legacy DCI formats can be monitored with DCI format 0_X/1_X. - DCI size alignment procedure can be updated accordingly. - If the number of different size of DCI exceeds three, DCI size are aligned among legacy DCI formats but not between the legacy DCI format(s) and DCI format 0_X/1_X. - Proposal 9: The existing BD/CCE budget i.e., PDCCH monitoring limits of MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ, should be maintained. - Proposal 10: RAN1 should discuss on which cell(s) and how to count DCI size and BD/CCE based on the following alternatives; - Alt.1: DCI size and/or BD/CCE of DCI format 0_X/1_X are counted only in one cell - Alt.1-1: Scheduling cell of the multi-cell scheduling - Alt.1-2: One of the scheduled cells which can be scheduled by DCI format 0_X/1_X - Alt.2: DCI size and/or BD/CCE of DCI format 0_X/1_X are counted on each of the scheduled cells which can be scheduled by DCI format 0_X/1_X - Alt.2-1: No scaling to each scheduled cell - Alt.2-2 (for BD/CCE budget): Scaling to each scheduled cell based on the number of co-scheduled cells - Proposal 11: For DCI size of DCI format 0_X/1_X, it should be counted on the scheduling cell of the multi-cell scheduling. - Proposal 12: For BD/CCE budget of DCI format 0_X/1_X, consider the following two alternatives; - Alt.1-1: BD/CCE of DCI format 0_X/1_X are counted on scheduling cell of the multi-cell scheduling - Alt.2-2: BD/CCE of DCI format 0_X/1_X are counted on each of the scheduled cells which can be scheduled by DCI format 0_X/1_X with scaling to each scheduled cell based on the number of co-scheduled cells Qualcomm: - Proposal 2: - BD/CCE counting/limit are per CIF/nCI value - I.e., for a CIF/nCI value associated with multiple cells for multi-cell scheduling, BD/CCE limit is provided as one of {44, 36, 22, 20} for the given SCS configuration of the scheduling cell - Per-SCS BD limit according to the UE reported pdcch-BlindDetectionCA applies even when multiple cells are mapped to a CIF/nCI value - To derive per-SCS BD limit using MPDCCHtotal,slot,μ=Ncellscap∙MPDCCHmax,slot,μ∙NcellsDL,μj=03NcellsDL,j, NcellsDL,μ in the equation is replaced by the sum of (1) the number of configured DL cells for scheduling cells with SCS configuration m without CIF/nCI configuration and (2) the total number CIF/nCI values for scheduling cells with SCS configuration m with CIF/nCI configuration - Same for per-SCS CCE limit - Proposal 3: - DCI size counting/limit are per CIF/nCI value - In Rel-18, monitoring DCI format 0_X/1_X and DCI format 0_2/1_2 for the same CIF/nCI value is not supported - In Rel-18, DCI size alignment procedure specified in TS 38.212 7.3.1.0 for DCI format 0_2/1_2 is re-used for DCI format 0_X/1_X - Proposal 4: - For a given CIF/nCI value, a UE can be configured to monitor a DCI format 0_X/1_X for a set of scheduled cells and to monitor legacy DCI format(s) for up to one of the scheduled cells in the set - Additional CIF/nCI value(s) for monitoring legacy DCI format(s) for the scheduled cell(s) in the set can be supported subject to optional UE capability signalling ITRI: - Proposal 1: - DCI size budget can be maintained via either DCI size alignment or configured size. - DCI size budget of multi-cell scheduling DCI is counted only in one scheduled cell. - Proposal 2: - For multi-cell scheduling DCI, the BD/CCE are counted only in one scheduled cell (i.e., Alt. 2)). Nokia, Nokia Shanghai Bell - Proposal 3.3.4: The DCI size for DCI formats 0_X / 1_X is RRC configured. - Proposal 3.4.2: For the DCI size / BD / CCE counting, adopt either: - Alt 2: Both DCI size and BD/CCE of DCI format 0_X/1_X are counted only in a same cell among the cells that can be potentially scheduled by DCI 0_X/1_X - The applicable same cell for the counting is separately RRC configured for DCI formats 0_X and 1_X. - Alt. 3 (Nokia interpretation in red): Both DCI size and BD/CCE of DCI format 0_X/1_X are counted for one or more cells configured with PDCCH candidates for multi-cell scheduling among the cells that can be potentially scheduled by DCI 0_X/1_X - The share of BDs / CCEs is assigned to the one or more co-scheduled cells by RRC configuration separately for DCI formats 0_X and 1_X. - FFS: If the same share is applicable to the DCI size counting / budget or if for simplicity a single cell is separately RRC configured for the DCI size counting for DCI formats 0_X and 1_X

### Moderator summary and proposals based on contributions

Regarding DCI size budget and BD/CCE counting, the two issues are discussed separately in RAN1#109e and merged together in RAN1#110:

Agreement Further study DCI size budget including below options for multi-cell scheduling DCI: - Option 1: Existing DCI size budget is maintained per scheduled cell. - Alt 1-1: DCI size budget is maintained via DCI size alignment and DCI size budget of DCI format 0_X/1_X is counted for each of the co-scheduled cells. - Alt 1-2: DCI size budget is maintained via configured size for multi-cell scheduling DCI and DCI size budget of DCI format 0_X/1_X is counted for each of the co-scheduled cells. - Alt 1-3: DCI size budget is maintained via DCI size alignment and DCI size budget of multi-cell scheduling DCI is counted only in one scheduled cell. - Option 2: Existing DCI size budget is not necessarily maintained per scheduled cell. - Alt 2-1: DCI size budget of multi-cell scheduling DCI is counted only in one scheduled cell. - Alt 2-2: DCI size budget of multi-cell scheduling DCI is not counted per serving cell and not considered in the related serving cell specific DCI size alignment procedure, e.g., for K co-scheduled cells, gNB guarantee the total budget of 3*K DCI sizes is not exceeded. - Alt 2-3: voiding the “3+1” limit for multi-cell scheduling - Alt 2-4: the DCI size budget for DCI size alignment can be separately configured for each cell - Alt 2-5: DCI size budget of the scheduling cell can be increased to account for the DCI format for multi-cell scheduling. Accordingly, the DCI size budget of a scheduled cell can be reduced. - Other options/alternatives could be considered. Agreement Further study BD/CCE counting for multi-cell scheduling DCI based on below options: - Alt 1: counted on each co-scheduled cell - Alt 2: counted only in one scheduled cell - Alt 3: scaled down to each of co-scheduled cell according to the number of co-scheduled cells - Alt 4: counted as part of the scheduling cell instead of each scheduled cell - Alt 5: scaled down to each of scheduled cells excluding scheduling cell - Alt 6: counted on each co-scheduled cell excluding scheduling cell - Other alternatives could be considered.

#### (Merged)Proposal 2-6 and Proposal 2-7rev3: - In order to discuss BD/CCE budget in case a UE monitors DCI format 0_X/1_X or both legacy DCI formats and DCI formats 0_X/1_X in a slot on a scheduling cell, Rel-17 PDCCH monitoring limits (i.e.,MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ andCPDCCHtotal,slot,μ) in the case where there is only one scheduling cell per scheduled cell is used for further discussion. - For further study DCI size budget and BD/CCE budget for multi-cell scheduling DCI, below Option 1 is considered: - Option 1: Existing DCI size budget is maintained per scheduled cell. - Alt 1: Both DCI size and BD/CCE of DCI format 0_X/1_X are counted on each of the cells that can be potentially scheduled by DCI 0_X/1_X. - No scaling to each co-scheduled cell - Alt 2: Both DCI size and BD/CCE of DCI format 0_X/1_X are counted only in a same cell among the cells that can be potentially scheduled by DCI 0_X/1_X. - Alt 3: Both DCI size and BD/CCE of DCI format 0_X/1_X are counted for one or more cells configured with PDCCH candidates for multi-cell scheduling among the cells that can be potentially scheduled by DCI 0_X/1_X. - Alt 4: Both DCI size and BD/CCE of DCI format 0_X/1_X are counted on the scheduling cell. - FFS details on how to maintain the DCI size budget, e.g., via DCI size alignment or configured size for the DCI format 0_X/1_X. - Other alternatives are not precluded.

For RAN1#110bis meeting, regarding the DCI size budget for multi-cell scheduling, 9 companies [Huawei, ZTE, Spreadtrum, CATT, xiaomi, Lenovo, LG, MTK, ITRI] support keeping existing “3+1” DCI size budget per scheduled cell. From the perspective of UE implementation, increasing the existing DCI size budget will result in high complexity for UE to perform blind detection, which is not expected. In addition, if the existing “3+1” size budget per scheduled cell is not maintained, current DCI size alignment rules of legacy DCIs also needs to be changed because each cell may have more size budget. Since majority companies prefer maintaining existing DCI size budge per scheduled cell to avoid high implementation complexity at UE side, moderator suggest keeping this principle firstly then going into the detailed solutions to maintain the current DCI size budget.

For the merged proposal, 13 companies [Langbo, Apple, xiaomi, Lenovo, FGI, LG, MTK, ITRI, Nokia, Huawei, ZTE, Spreadtrum, CATT] expressed the preference on the detailed alternatives to determine DCI size alignment and BD/CCE counting. Their views are summarized below:

- Alt 1: Langbo, Apple

- Alt 2: xiaomi, Lenovo, FGI, LG, MTK, ITRI, Nokia

- Alt 3: Huawei, ZTE, Spreadtrum, CATT,

- Alt 4: Apple, NTT DOCOMO

Furthermore, 4 companies [Intel, CMCC, Samsung, NTT DOCOMO] propose BD/CCE of DCI format 0_X/1_X is scaled to each scheduled cell based on the number of co-scheduled cells.

Basically, for Alt 1, when DCI size and BD/CCE of DCI format 0_X/1_X are counted on each of the cells that can be potentially scheduled by the DCI 0_X/1_X, the size and BD/CCE of DCI format 0_X/1_X are counted multiple times which wastes PDCCH monitoring capability. For Alt 4, when DCI size and BD/CCE of DCI format 0_X/1_X is counted on the scheduling cell, it will lead to DCI size alignment issue in each slot or SS set dropping when the scheduling cell is PCell.

For Alt 2 and Alt 3, the DCI size of DCI format 0_X/1_X is counted on one or more cells that can be potentially scheduled by the DCI 0_X/1_X. The open issue is which cell(s) the DCI size and BD/CCE are counted. Some companies propose the size budget can be counted on the co-scheduled cell which has the minimum number of legacy DCI formats or has the minimum number of DCI sizes after alignment or SCell. Other companies propose configuring a cell for counting DCI size and BD/CCE.

Based on above, moderator suggests merging Alt 2 and Alt 3 as both DCI size and BD/CCE of DCI format 0_X/1_X are counted on one or more cells among the cells that can be potentially scheduled by DCI 0_X/1_X. How to determine the one or more cells can be FFS. Proposal 2-7 is provided for combining Alt 2 and Alt 3.

Regarding the DCI format design for multi-cell PDSCH/PUSCH scheduling, for a cell within a set of cells which can be scheduled by one DCI format 0_X/1_X, if UE is configured to simultaneously monitor both DCI format 0_X/1_X for scheduling one or multiple cells within the set of cells including the cell and DCI format 1_0/0_0/0_1/1_1 for scheduling single cell, then existing “3+1” DCI size budget can’t be maintained.

To avoid standardization effort on DCI size alignment to maintain “3+1” DCI size budget, 4 companies [Ericsson, ITRI, Nokia, Lenovo] propose to configure the payload size of DCI format 0_X/1_X by RRC signaling. How to align the DCI payload size can be left for gNB implementation. Proposal 2-8 is provided for discussion.

### 1st round of discussions

#### Proposal 2-7:

- For a set of cells which is configured for multi-cell scheduling,

- Existing DCI size budget is maintained on each cell of the set of cells.

- Both DCI size and BD/CCE of DCI format 0_X/1_X are counted on one or more cells among the set of cells.

  - FFS: Which cell(s) both DCI size and BD/CCE of DCI format 0_X/1_X are counted on

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| MTK | Support |
| Qualcomm | Regarding the 1st bullet “Existing DCI size budget is maintained on each cell of the set of cells”, it is not clear whether a size of DCI 0_X/1_X configured for CC {1, 2, 3, 4} is counted to each CC in the CC set {1, 2, 3, 4}. Regarding the 2nd bullet “Both DCI size and BD/CCE of DCI format 0_X/1_X are counted on one or more cells among the set of cells”, how to count is not a matter. The first question should be “whether/how to maintain BD/CCE budget per scheduled CC?”. If we agree to maintain BD/CCE budget per scheduled CC, it is not clear how “counting BD/CCE of DCI 0_X/1_X on one or some cells among the set of cells” works. This would increase BD/CCE budget per scheduled CC. |
| Nokia, NSB | Conditionally support – but in the proposal is still unclear if the DCI size and BD / CCE budget is counted as full (without scaling) on the one or more cells or if there is some scaling applied (similarly as pointed out be QC). We think that this needs to be clarified before being able to agree on it. |
| Apple | We support the 1st bullet. On 2nd bullet, it is unclear to us how the BD/CCE budget is counted in case of more than one cells. In addition, this option is not preferable as it will require further discussion on which cells and how to select for BD/CCE counting (as captured in FFS). In our view, it is reasonable option to count BD/CCE and DCI size on one cell and that cell can be scheduling cell. |
| Langbo | We are OK for the first sub-bullet. For the second sub-bullet, it is unclear to us how the BD/CCE is counted and whether the existing per scheduled cell and per scheduling cell group limitations are maintained or not. Clarifications are needed. |
| Huawei, HiSilicon | Support. We think this is at least a scenario that needs to support, without a scaling that is mentioned by Nokia, as this is similar to the case of legacy cross-carrier scheduling in terms of max BD/CCE. |
| Intel | The first sub-bullet is fine. It is not clear what is the real progress to have the second sub-bullet. We prefer to clarify and agree on some high level principle first, e.g., - The total budget of DCI size and BD/CCE should be not increased The DCI size and BD/CCE counting should be fair to each co-scheduled cell |
| ZTE | Based on the search space determination of the MC-DCI, it seems there is no additional issue of the FFS. The only additional issue is how to handle the DCI size budget/size alignment. The DCI size budget for scheduling cell is more challenging than a scheduled cell, because fallback DCI cannot be configured on the scheduled cell except the PCell scheduled by sSCell. As a result, existing DCI size budget is maintained per scheduled cell can be easily maintained at least for the scheduled cell which is not a scheduling cell. For a scheduling cell which is also a scheduled cell, it can be configured with DCI format 0_0/1_0, DCI format 0_1/1_1, DCI format 0_2/1_2 and DCI format 0_X/1_X, which is challenging to maintain up to 3 sizes of DCI formats with CRC scrambled by C-RNTI per serving cell. The updated proposal 2-7 is listed below. #### Proposal 2-7(update): - For a set of cells which is configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - Both DCI size and BD/CCE of DCI format 0_X/1_X are counted on one or more cells configured with same search space ID and DCI format 0_X/1_X on the scheduling cell among the set of cells. - FFS: how to handle DCI size alignment for a scheduling cell which is also a scheduled cell Which cell(s) both DCI size and BD/CCE of DCI format 0_X/1_X are counted on. |
| Moderator | @Qualcomm: The 1st bullet means, from each scheduled cell’s point of view, “3+1” size budget including all the configured DCI formats on it should be maintained. The 2nd bullet means both DCI size and BD/CCE of DCI format 0_X/1_X are counted on one or more cells. E.g., using DCI format 0_X/1_X from CC0 to schedule {CC1, CC2, CC3, CC4}, if CC1 has minimum legacy DCI formats, then the budget of DCI format 0_X/1_X is counted only on CC1. If the budget of DCI format 0_X/1_X can be counted on N cells, I think it is open to discuss whether/how to perform budget scaling per N cells or count budget on N cells without scaling. As mentioned in the first bullet, BD/CCE budget should be maintained as legacy. @Nokia: If the budget of DCI format 0_X/1_X can be counted on more than one cell, I think it is open to discuss whether/how to perform budget scaling. @Apple: If the budget of DCI format 0_X/1_X can be counted on more than one cell, I think it is open to discuss whether/how to perform budget scaling or count budget on N cells without scaling. If companies are OK with only one cell for counting budget, then I am happy to save the time to discuss the scaling operation. Additionally, when DCI size and BD/CCE of DCI format 0_X/1_X is counted only on the scheduling cell, it will lead to DCI size alignment issue in each slot or SS set dropping when the scheduling cell is PCell. @Langbo: E.g., using DCI format 0_X/1_X from CC0 to schedule {CC1, CC2, CC3, CC4}, if CC1 has minimum legacy DCI formats, then the budget of DCI format 0_X/1_X is counted only on CC1. If the budget of DCI format 0_X/1_X can be counted on N cells, I think it is open to discuss whether/how to perform budget scaling per N cells or count budget on N cells without scaling. As mentioned in the first bullet, BD/CCE budget is maintained. @Intel: Not sure about your point. The intention of 2nd bullet is which cell the size budget counting is on. The below bullet makes sense and has been covered in the 1st bullet. - The total budget of DCI size and BD/CCE should be not increased @ZTE: I agree with your point. But I don’t think your addition is really needed. Because the case that the scheduling cell is also one of co-scheduled cells has been covered by the main bullet. |
| CMCC | We are fine with the 1st bullet, it is reasonable to maintain the DCI size budget as the current specification for each cell. For the 2nd bullet, more clarifications are needed for DCI size counting and BD/CCE counting. We prefer to discuss DCI size and BD/CCE counting on one cell or scaled down to each co-scheduled cell separately. |
| New H3C | Support |
| Fujitsu | We support the first sub-bullet. The second sub-bullet is not clear to us. Hope these can be clarified. - Whether DCI size and BD/CCE are counted on same set of cells? If it may be unnecessary, separate discussions on the 2 issues are preferred. - Whether scaling (i.e. Alt 3 as below) is possible? - Alt 3: scaled down to each of co-scheduled cell according to the number of co-scheduled cells. Additionally, we share the same view with Intel that we can agree on the total budget first. |
| CATT | Support Proposal 2-7. Regarding the second bullet, the BD/CCE of DCI format 0_X/1_X is counted on one or more cells among the set of cells. It can up to gNB to configure BD/CCE of DCI format 0_X/1_X for one or more cells, and maintain BD/CCE budget per scheduled CC. For example, if gNB configures BD/CCE of DCI format 0_X/1_X on only one scheduled cell, the BDs will be counted this scheduled cell CC1. If the gNB configures BD/CCE of DCI format 0_X/1_X across CC1 and CC2, the BD/CCE configured on CC1 will be counted to CC1, the BD/CCE configured on CC2 will be counted to CC2. From the perspective of spec, there is no need to specify which cell(s) will be configured BD/CCE budget or how to count BD/CCE for each scheduled cell. The legacy BD/CCE budget and counting method can be reuse for DCI format 0_X/1_X. |
| vivo | For ‘a set of cells which is configured for multi-cell scheduling’, similar comments as P2-5 On 2nd bullet, same view as apple, BD/CCE is counted for a scheduled cell, the scheduled cell can be the scheduling cell or be determined by some other rule |
| LGE | Support the 1st bullet with understanding the existing DCI size budget is maintained on a cell of the set of cells regardless the cell is one of co-scheduled cells by DCI 0_X/1_X or not. For the 2nd bullet, DCI size and BD/CCE counting for more than one scheduled cells may cause the follow up issues such as how to select those cells and whether/how to scale/distribute the BD/CCE budget over those cells. To avoid these issues, one “scheduled” cell for counting DCI size and BD/CCE would be an easiest way. Note that one selected cell for DCI size counting or BD/CCE counting may or may not be the same, which can be further discussed. |
| Samsung | For the first bullet, our preference is to increase DCI size budget to avoid unnecessary padding of single-cell scheduling DCI formats, especially for the PCell, as that is against the whole purpose of introducing MC-DCI. However, for progress, we can be OK with the first bullet if that is the majority’s view. For the second bullet, we prefer to discuss DCI size counting and BD/CCE counting separately: - The size of MC-DCI format for any/all sets of co-scheduled cells is counted only on a single reference cell (FFS: reference cell). That avoids/minimizes padding. - The BD/CCE counting should be discussed together with the BD/CCE limits to provide the full picture. We prefer to keep Rel-17 BD/CCE limits and count BDs/CCEs for a PDCCH candidate in a search space set associated with a set of co-scheduled cell (per linking procedure of FL P2-5) by counting towards each cell from the set of co-scheduled cells according to scaling factors. So, we propose the following modification: #### Proposal 2-7: - For a set of cells which is are configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - Both DCI size and BD/CCE of DCI format 0_X/1_X are is counted on one or more reference cells among the set of cells. - FFS: the reference cell Which cell(s) both DCI size and BD/CCE of DCI format 0_X/1_X are counted on - Rel-17 BD/CCE limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ same as in Rel-17) are maintained for the cells. - BD/CCE of [a PDCCH according to a search space set associated with a set of cells for] DCI format 0_X/1_X is counted on all cells from the set of cells based on scaling factor. FFS the scaling factors. |
| Spreadtrum | One clarification is the BD/CCE counting is based on one CIF value of the search space. If only one CIF for all the cell combinations, they are counted on one cell. But if there are several CIF values associated with the search space on different cells, DCI size and BD/CCE of DCI format 0_X/1_X are counted on the cells with search space configuration. As the figure below, BD/CCE counting on CC2 with CIF=0, and on CC3 with CIF=1. |
| Xiaomi | We are fine with the first bullet. For the second bullet, the DCI size budget can be simply handled by the gNB, it seems no need to discuss how to count the DCI size among the co-scheduled cells. |
| NTT DOCOMO | We support the first sub-bullet. For the second sub-bullet, we think it is preferable to agree on the possible options which can be considered for DCI size and/or BD/CCE counting for better progress. For example, for BD/CCE count, it can be counted on 1) the scheduling cell or 2) on one scheduled cell if they are counted on one cell. Furthermore, the can be scaled to each co-scheduled cells if they are counted on more than one cell. However, we are also fine to support this proposal so far with the understanding that the BD/CCE of DCI format 0_X/1_X can be counted on each scheduled cell with scaling and it would be discussed further if it is counted on more than one cell. |
| Ericsson1 | Our preference is to increase the DCI size budget (no more than 4 for the cell if it is an SCell and is no more than 5 if it is a PCell). Configuration of DCI format 0_X/1_X for a scheduled cell should not preclude scheduling via regular non-fallback DCI format 0_1/1_1 for the same scheduled cell. With this principle, if existing DCI size budget is maintained, the regular non-fallback DCI format size must be increased (size matched) to that of DCI format 0_X/1_X. Increasing DCI size budget is better option than such potentially excessive size matching. For BD/CCE, clarification is needed on what counting on one or more cells among the set of cells means – is this to be reflected in the following parts from specification or something else? For each scheduled cell from the Ncells,0DL,μ downlink cells, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell more than minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates or more than minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ non-overlapped CCEs per slot. For cross-carrier scheduling, the number of PDCCH candidates for monitoring and the number of non-overlapped CCEs per span or per slot or per group of Xs slots are separately counted for each scheduled cell. We think duplicate counting of a BD/CCE attempt for a mc-DCI format should be avoided when counting BD/CCE across co-schedulable cells. |
| Qualcomm | Regarding DCI size budget, we are now OK with the moderator’s clarification. The 1st bullet means, from each scheduled cell’s point of view, “3+1” size budget including all the configured DCI formats on it should be maintained. Regarding BD/CCE counting/budget: If we take the ZTE’s approach above, we should make sure that the UE is allowed to support DCI 0_X/1_X monitoring on SS sets linked to one of the CCs that can be scheduled by the DCI 0_X/1_X. In addition, the UE should also be allowed to support legacy DCI format monitoring on SS sets linked to one of the CCs that can be scheduled by the DCI 0_X/1_X, which is the same CC as the one for the DCI 0_X/1_X. More than “one” can be considered for advanced (not-basic) UEs. Otherwise we cannot accept this way. If we take the Samsung’s approach above, we would like to make sure that a UE can indicate support of the scaling factor = 1. This might be simpler and might be acceptable to companies compared to ZTE’s approach. |
| Apple2 | Thanks, moderator for the clarification. We are also open to consider Alt 1, where the BD/CCE is counted on each of the co-scheduled cells, if Alt 4 is not preferred by majority of the companies. However, combining the three alternatives seems unjustifiable and adds unnecessary complexity. This should be avoided considering the limited time. |
| ITRI | Support the first bullet. Not support the second bullet. Alt. 2 is preferred. |
| Moderator2 | @Fujitsu: For your 1st question, the answer is yes to me. For your 2nd question, scaling is open for discussion. @Ericsson: Seems all companies except Ericsson are OK with first bullet. Increasing DCI size budget can’t be acceptable to UE vendors. @All: As many comments on the case of more than one cell configured SS of DCI 0_X/1_X, let me try to reformulate the proposal as below: #### Proposal 2-7rev1: - For a set of cells which is configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - Both DCI size and BD/CCE of DCI format 0_X/1_X are counted at least on one cell among the set of cells. - FFS: Which cell(s) both DCI size and BD/CCE of DCI format 0_X/1_X are counted on - FFS: different cells for DCI size counting and BD/CCE counting |
| Nokia/NSB | Same comment as to the search space change: if the intention is for now to agree / focus on the single cell case, the ‘at least on one cell’ again includes all options – including (i) one cell, (ii) subset of cells and (iii) all cells. |
| Apple | Similar comments and updates as for the search space configuration is valid here as well, i.e., if scheduling cell is one of the cells of the set of cells, then both DCI size and BD/CCE of DCI format 0_X/1_X are counted on the scheduling cell. |
| Qualcomm | Thanks, moderator, to accommodate our concern. Focusing on BD/CCE count, we still have a concern on this way. Suppose the following case. - A UE monitors DCI 0_X/1_X for CCs {1, 2, 3, 4} on CC-0. The BD/CCE is configured to be counted on CC 1, for example. - The UE monitors legacy DCI formats for each of CCs {1, 2, 3, 4}. Let’s focus on CC-2 as one scheduled CC. - The UE monitors legacy DCI formats for CC-2, whose BDs/CCEs are counted on CC-2 (as legacy cross-carrier scheduling). If the UE does not report pdcch-BlindDetectionCA and if SCS of CC-0 is 30kHz, then max # of BDs for the legacy DCI formats for CC-2 is 36. - The DCI 0_X/1_X can schedule CC-2 (even it is possible to schedule CC-2 only), but its BDs/CCEs are counted on CC-1. Max # of BDs for the DCI 0_X/1_X can also be 36. - In total, the UE has to monitor and process, 36 + 36 = 72 BDs, to get a grant for CC-2. Because of this, we think either of the following is necessary: Opt.1: a UE is allowed not to monitor legacy DCI formats for all the CCs in the CC-set In the same example above, BD/CCE of the legacy DCI formats for CC-1 is counted on the same pool as DCI 0_X/1_X, i.e., on CC-1. If the UE does not monitor legacy DCI formats for other CCs {2, 3, 4}, max # of BDs for a scheduled CC is limited to 36. Opt.2: a BD/CCE of DCI 0_X/1_X is counted on all the CCs in the CC-set. In the same example above, if BD/CCE of DCI 0_X/1_X is counted on each of the CC {1, 2, 3, 4}, max # of BDs for a scheduled CC is limited to 36. We are open to either of the options, but would like to make sure that BD/CCE number per scheduled CC can be capped by the per-CC limit, at least for basic multi-cell scheduling. |
| Moderator 3 | @Nokia: Thanks for the good comments. Below update is made. @Apple: We can FFS Which cell(s) both DCI size and BD/CCE of DCI format 0_X/1_X are counted on on so that we can further determine whether this cell is a scheduling cell if the scheduling cell is included in the set of scheduled cells or one of scheduled cells. @Qualcomm: In your example, when DCI format 0_X/1_X is used for multi-cell scheduling CC2 or single-cell scheduling CC2, the BD/CCE budget is counted on CC1, which is max 36. When DCI format 0_1/1_1 is used for single-cell scheduling CC2, the BD/CCE budget is counted on CC2, which is max 36. As long as BD/CCE budget is maintained for each of CC1 and CC2, seems UE complexity is not increased. Maybe I don’t fully understand your question. Please correct me. Further update is shown in below: #### Proposal 2-7rev2: - For a set of cells which is configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - Both DCI size and BD/CCE of DCI format 0_X/1_X are counted on one cell among the set of cells. - FFS: Which cell(s) both DCI size and BD/CCE of DCI format 0_X/1_X are counted on - FFS: different cells for DCI size counting and BD/CCE counting - FFS: whether DCI size and BD/CCE of DCI format 0_X/1_X can be additionally counted on other cells of the set of cells. |
| Intel | We do not support the updated proposal. The most important issue is to ensure fairness among the co-scheduled cells in terms of DCI size counting and BD/CCE counting. - We do not think counting on a single cell among the set of configured cells would be desirable, since it impacts the scheduling of the selected one cell. - We prefer to count the DCI size and BD/CCE to all cells with scaling applied. The scaling is to avoid overcounting, to exploit full UE capability on PDCCH detection for scheduling flexibility. Note: MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μare considering all cells by its definition, i.e., they are defined as the budget for all cells with same numerology of the scheduling cell. |
| CATT | Thanks for the update. We are ok with the FL’s direction to discuss BD/CCE. We think both BD/CCE and DCI size should be counted on the same scheduled cell. For R15 cross-scheduling, the BD/CCE and DCI size are counted on the scheduled cell which is configured with PDCCH candidates of search space. For example, the UE is configured search space of DCI-1 on scheduled cell CC1, and is configured to monitor this DCI-1 on scheduling cell CC0. In this case, the BD/CCE and DCI size of DCI-1 will be counted to scheduled cell CC1 based on the configured PDCCH candidates of search space. One straightforward approach to reuse same principle for DCI format 0_X/1_X, that is, both DCI size and BD/CCE of DCI format 0_X/1_X are counted to the scheduled cell(s) configured with PDCCH candidates of DCI format 0_X/1_X. We prefer to list some options below first FFS for down-selections. In additional, it’s not clear the motivation of counting DCI size and BD/CCE of DCI format 0_X/1_X on other cells of the set of cells. We can’t get point to add last FFS. #### Proposal 2-7rev2: - For a set of cells which is configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - Both DCI size and BD/CCE of DCI format 0_X/1_X are counted on one cell among the set of cells. - FFS: Which cell(s) both DCI size and BD/CCE of DCI format 0_X/1_X are counted on - Scheduled cell configured with PDCCH candidates of DCI format 0_X/1_X - Scheduling cell - others - FFS: different cells for DCI size counting and BD/CCE counting - FFS: whether DCI size and BD/CCE of DCI format 0_X/1_X can be additionally counted on other cells of the set of cells. |
| Qualcomm | Response to moderator: “In your example, when DCI format 0_X/1_X is used for multi-cell scheduling CC2 or single-cell scheduling CC2, the BD/CCE budget is counted on CC1, which is max 36. When DCI format 0_1/1_1 is used for single-cell scheduling CC2, the BD/CCE budget is counted on CC2, which is max 36.” Yes, above is correct. Then, the UE has to process 36 BDs counted on CC-1 and 36 BDs counted on CC-2 in a slot to see if there is a grant for data on CC-2. This requires UE to process doubled number of BDs/CCEs for DCIs for a scheduled CC. Let us rephrase Opt.1 in our earlier reply below. Again, we are open with either option. Opt.1: a UE monitoring DCI 0_X/1_X for a CC-set is allowed to support monitoring legacy DCI formats only for one CC in the CC-set and DCI 0_X/1_X whose BD/CCE/DCI-size are counted on the same CC in the CC-set In the same example above, BD/CCE of the legacy DCI formats for CC-1 is counted on the same pool as DCI 0_X/1_X, i.e., on CC-1. If the UE does not monitor legacy DCI formats for other CCs {2, 3, 4}, max # of BDs for a scheduled CC is limited to 36. Opt.2: a BD/CCE of DCI 0_X/1_X is counted on all the CCs in the CC-set. In the same example above, if BD/CCE of DCI 0_X/1_X is counted on each of the CC {1, 2, 3, 4}, max # of BDs for a scheduled CC is limited to 36. |
| LGE2 | Support the updated proposal, i.e. Proposal 2-7rev2. |
| OPPO | We are OK with the updated proposal 2-7rev2, except the second FFS. We fail to see the reason why the DCI size and BD/CCE need to be counted on different cells. If the DCI size is counted on cell-1 and BD/CCE is counted on cell-2, how to use the BD/CCE capability on cell-2 to monitoring the DCI format 0_X/1_X with the DCI size counted on cell-1? |
| Samsung2 | We continue to think that the two issues can be better/faster resolved if treated separately, per agreements from RAN1#109-e. - The size of DCI format 0_X/1_X can be assumed same for all sets of co-scheduled cells, and such size can be counted on a single reference cell. - The BD/CCE for a DCI format 0_X/1_X is based on which search space the PDCCH is monitored, which in turn is based on which set of co-scheduled cells is associated with the search space set. Therefore, the UE can “distribute” the BD/CCE count among the set of co-scheduled cells using the scaling factor. @ QC: Regarding a comment about scaling factor = 1, we think such assumption will unnecessarily and quickly “saturate” the BD/CCE budget as it implies each PDCCH candidate for DCI format 0_X/1_X will be counted separately for each of the co-scheduled cells, e.g. as 4 BDs across 4 co-scheduled cells. This would defeat the purpose of multi-cell scheduling for reducing PDCCH candidates and increasing PDCCH monitoring capability. As commented above, we agree to keep the Rel-17 limits per-scheduled-cell and per-SCS-scheduling-cells regardless of the configured DCI formats, but one PDCCH candidate for DCI format 0_X/1_X needs to be counted as one BD across the set of co-scheduled cells. In such case, scaling factor = 1 is not needed. |
| Langbo | We share similar views as QC. Even though the BD/CCE budgets do not exceed the sum of per scheduled cell limitations, it does not mean the monitoring complexity of one scheduled cell is not increased, as the budgets cannot be dynamically shared among all scheduled cell. We still think BD/CCE of DCI 0_X/1_X is counted on all the CCs in the CC-set is the most favourable way in terms of UE complexity. We think scaling the BD/CCE number to all cells is also fine. |
| NTT DOCOMO2 | For the second sub-bullet, we share the similar view as Intel. We are fine to count DCI size and BD/CCE budget on all the cells of set of the cells. Especially for BD/CCE counting, we prefer to support scaling to each cell within the set of cells which can avoid duplicate counting and improve the scheduling flexibility. |
| MTK | We are generally fine with the direction of Proposal 2-7rev2. At the same time, we also think QC’s concern is valid. Maybe we can add one note to Proposal 2-7rev2 as below (same sentence mentioned by QC): - Note: BD/CCE number per scheduled CC is capped by the per-CC limit, at least for basic multi-cell scheduling. |
| CMCC | We share the similar views with Samsung that the DCI size and BD/CCE counting can be discussed separately. |
| Qualcomm | Regarding Samsung’s comment on scaling factor, we have different understanding. WIDstates that multi-cell scheduling is for high spectral efficiency (by DCI overhead reduction) and for high power efficiency (by simplifying PDCCH monitoring). If a UE is required to implement very advanced PDCCH monitoring capability just because of MC-scheduling, there is no point for the UE to support MC-scheduling to begin with. So we cannot accept to mandate it. |
| Apple | We are also fine to count DCI size and BD/CCE budget on all the cells of set of the cells, as also supported by Intel and DOCOMO. We can further consider scaling to avoid duplicate counting |
| Moderator4 | @All: Based on your inputs, I try to find some middle ground for compromise. We need move on. #### Proposal 2-7rev3: - For a set of cells which is configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - Both DCI size and BD/CCE of DCI format 0_X/1_X are counted on all cells configured with PDCCH candidates for multi-cell scheduling among the set of cells. - It is up to network which cell(s) the PDCCH candidates of the DCI format 0_X/1_X are configured. - Both DCI size budget and BD/CCE budget of the DCI format 0_X/1_X are equivalently scaled to all the cells configured with PDCCH candidates of the DCI format 0_X/1_X. - Note: BD/CCE number per scheduled cell is capped by Rel-17 per-cell limit. |

s

#### Proposal 2-8:

- The payload size of DCI format 0_X/1_X is configured by RRC.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| MTK | We are not sure why we need RRC to configure payload size of DCI format 0_X/1_X. Willing to hear views from more companies. |
| Qualcomm | 1st bullet of Proposal 2-7 should be clarified first. |
| Nokia, NSB | Support. And we think we need a separate configuration for 0_X from 1_X. |
| Apple | It will be good to hear the motivation on the need for RRC configuration for the payload size. In addition, is this payload size configuration independent of the actual number co-scheduled cells by DCI format 0_X/1_X? |
| Langbo | Same questions as Apple. |
| Huawei, HiSilicon | General support and we think even if not agreed, this would be the case - the DCI field type discussion already implies such. |
| Intel | We prefer to reuse or extend the existing DCI size alignment rule |
| ZTE | In current spec, the payload size of the non-fallback DCI format is configured by the network since many fields are configured by RRC. We think it is reasonable to follow this principle. We are generally fine with this the principle of this proposal. A question is does this mean the payload size of DCI format 0_X/1_X is configured by RRC directly? For example, RRC configures how many bits the DCI format 0_X/1_X has. We think the legacy way of configuring each field is sufficient. |
| Moderator | @MTK: The intention is to avoid effort on size alignment discussion and different bandwidths for different scheduled cell combinations. @Qualcomm: Please check my reply under Proposal 2-7. @Nokia: Yes, separate configuration can be considered if agreed. @Apple @langbo: The intention is to avoid effort on size alignment discussion and different bandwidths for different scheduled cell combinations. Yes, if the payload size is configured, it is independent to actual scheduled cells. @ZTE: This proposal implies the DCI size of 0_X/1_X is directly configured. This method is also used for DCI format 4-2 and other DCI format 2_X series. If following DCI format 0_1/1_1 way, then we have to discuss how to determine the FDRA field/BWP field for each co-scheduled cells as long as per cell configuration is different. To save DCI size alignment discussion, it is simple to configure the size of MC DCI directly. |
| New H3C | Support |
| Fujitsu | Same view as Apple. |
| CATT | Support. In our understanding, if the payload size of DCI format 0_X/1_X is configured by RRC, DCI format 0_X/1_X doesn’t require to participate in the DCI size alignment procedure in the section 7.3.1.0 of TS 38.212. It is up to gNB to maintain ‘3+1’ budget for each scheduled cell. |
| Vivo | Need clarification on whether the size is per entire set of scheduled cell or per scheduled cell or per scheduled combination configured? |
| LGE | We don’t see the reason to have RRC configuration for determining DCI payload size. The payload size of DCI 0_X/1_X can be reasonably determined as a maximum size among the size required for each combination of co-scheduled cells and the corresponding DCI size is fixed regardless of which combination of cells are scheduled by DCI 0_X/1_X. This is straightforward and is similar to the method of determining the payload size of multi-TTI scheduling DCI in FR2-2 for Rel-17. |
| Samsung | We suggest to list different options for DCI size alignment and include RRC configured size and the existing DCI size alignment procedure of TS 38.212 – both are supported in Rel-17. Also, one basic issue to resolve is whether a payload size of DCI format 0_X/1_X can depend on a set of co-scheduled cells (associated with the search space set). We suggest to clarify as follows. - The payload size of DCI format 0_X/1_X is same for all sets of co-scheduled cells. - FFS: The payload size of DCI format 0_X can be different from the payload size of DCI format 1_X. |
| Spreadtrum | We understand the intention. However, we are afraid it is hard to find a existing DCI size with C-RNTI for DCI 0_X/1_X, since they are assumed with bigger size than DCI 0_0/1/2 and DCI 1_0/1/2. |
| Xiaomi | One clarification question, is it correct understanding that different payload size of the MC-DCI will be configured with different number of co-scheduled cells (configured rather than actual scheduled). |
| NTT DOCOMO | We can support this proposal but we think it is preferable to conclude which legacy DCI formats are monitored with DCI format 0_X/1_X and/or whether the current DCI size budget is maintained first. Then this proposal can be discussed with how to maintain the current DCI size budget considering the simultaneous monitoring with legacy DCI format(s). |
| Ericsson1 | Support. |
| Qualcomm | We are still not sure how this proposal can resolve the issue of DCI size budget 3+1 for each scheduled CC (seems similar question as Apple). If DCI 0_0/1_0, 0_1/1_1, 0_2/1_2, and 0_X/1_X are monitored for a given scheduled CC, how the proposal can ensure 3+1 size budget for the scheduled CC? We think we can simplify the scenario in Rel-18. For example, monitoring all non-fallback DCI formats, 0_1/1_1, 0_2/1_2, and 0_X/1_X can be excluded in Rel-18. |
| Apple2 | Thanks for clarification, but in our view, we still don’t see the motivation to introduce RRC configuration for the size. Also, as some other companies commented, we should also discuss if same DCI size for DCI format 0_X/1_X is applicable for any combination of co-scheduled cells. In our view, same size should be considered . |
| ITRI | Support |
| Moderator2 | @vivo: The size is determined based on entire set of cells configured for multi-cell scheduling. @LGE: @Speadtrum: @QC: The intention is to avoid effort on size alignment discussion and different bandwidths for different scheduled cell combinations. If DCI size is counted on one scheduled cell with minimum legacy DCI formats, then it is easier to follow RRC configured size without need to check all the possible DCI size then find the max one. |
| Qualcomm | If a UE monitors DCI 0_0/1_0, 0_1/1_1, 0_2/1_2 for a scheduled CC and processes DCI size alignment procedure in 38.212, the DCI size budget for unicast DCI, which is 3, is fully consumed. It is still not clear how the proposal enables keeping DCI size budget = 3 for a scheduled CC. Moderator indicated “if DCI size is counted on one scheduled cell with minimum legacyDCI formats”. This implies that the assumption here is that the DCI 0_X/1_X is counted on a cell where the UE does not monitor all of the legacy DCI formats? |
| Moderator3 | @Qualcomm: Using RRC configuring DCI size is also applied to DCI format 4-2. The gNB can maintain size budget. E.g., there are below DCI formats which may need to be monitored by a UE: (1) DCI format 1-0/0-0 (2) DCI format 0-1 for scheduling a single cell (3) DCI format 0-X for scheduling one or multiple cells (4) DCI format 1-1 for scheduling a single cell (5) DCI format 1-X for scheduling one or multiple cells To keep “3+1” DCI size budget, one way is to align the payload size of (2) and (4), (3) and (5) then the total number of different payload sizes with C-RNTI is 3, i.e., DCI format 0-0/1-0, DCI format 0-1/1-1, and DCI format 0-3/1-3. The padding bits may be appended to (2) and (3) until both (2) and (4) have same size, (3) and (5) have same size considering DL DCI usually requires more bits than UL DCI. Based on RRC configured size, UE doesn’t need to determine the payload size according to the maximum scheduled bandwidths on active BWPs of all cells of entire set of cells. @All: Let’s check companies’ views - Alt 1: The payload size of DCI format 0_X/1_X is configured by RRC. - Alt 2: The payload size of DCI format 0_X/1_X is determined by UE. Please add your company name in below tables. Support Alt 1; CATT, Nokia/NSB, New H3C. Support Alt 2; Intel, Qualcomm, LGE, Langbo, NTT DOCOMO, Applle. |
| LGE2 | Alt 2 is preferred as done for multi-TTI scheduling DCI in Rel-17 |
| OPPO | Alt-2 is preferred. The example in Moderator3 seems to make the DCI size budget available to legacy DCIs to be 2+1. It is not clear to us whether this budget reduction would impact the logic in existing specification. We think the Alt-2 could be defined more precisely: - Alt 2: The payload size of DCI format 0_X/1_X is determined derived by UE based on RRC configurations. |
| Samsung2 | Unless DCI size budget is satisfied by certain restrictions in the specifications (e.g., a reference cell for DCI size counting that includes < 3 single-cell DCI formats, as follow-up of FL Proposal 2-7), the DCI size alignment procedure in 38.212 for DCI formats with C-RNTI needs to be modified to capture the impact of DCI formats 0_X/1_X, such as additional zero padding for single-cell DCI formats, regardless of whether DCI formats 0_X/1_X have configured size or not. Also, it is not clear to us how an RRC configured size for DCI format 0_X/1_X can avoid the need to discuss the size of FDRA field or other DCI fields. We are open to discuss configured size for DCI format 0_X/1_X, but we think either such DCI size restrictions or general modifications to 38.212 in the presence of DCI formats 0_X/1_X are more essential issues and need to be discussed first. Furthermore, we basically agree with the FL comment above “The size is determined based on entire set of cells configured for multi-cell scheduling” and think this requires a formal RAN1 agreement, e.g., as we suggested earlier: - The payload size of DCI format 0_X/1_X is same for all sets of co-scheduled cells. - FFS: The payload size of DCI format 0_X can be different from the payload size of DCI format 1_X. |
| Langbo | Alt 2 is preferred. |
| New H3C | We prefer Alt.1 |
| NTT DOCOMO2 | We don’t see the strong need for RRC configuration of DCI format 0_X/1_X size. How to maintain the current DCI size budget, e.g., whether the size of format 0_X/1_X and legacy DCI format(s) need to be aligned, should be discussed separately. |
| MTK | With the explanation from moderator, we are fine with both Alt 1 and Alt 2. |
| Apple | We prefer Alt 2 |
| Moderator4 | @All: Based on the input, I propose Alt 2 for further discussion with Samsung’s addition. #### Proposal 2-8rev1: - The payload size of DCI format 0_X/1_X is determined by UE and same for a set of cells which can be scheduled by the DCI format 0_X/1_X. |

### 2nd round of discussions

#### Proposal 2-7rev3:

- For a set of cells which is configured for multi-cell scheduling,

- Existing DCI size budget is maintained on each cell of the set of cells.

- Both DCI size and BD/CCE of DCI format 0_X/1_X are counted on all cells configured with PDCCH candidates for multi-cell scheduling among the set of cells.

  - It is up to network which cell(s) the PDCCH candidates of the DCI format 0_X/1_X are configured on.

  - Both DCI size budget and BD/CCE budget of the DCI format 0_X/1_X are scaled to all the cells configured with PDCCH candidates of the DCI format 0_X/1_X.

- Note: BD/CCE number per scheduled cell is capped by Rel-17 per-cell limit.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | OK in principal |
| Qualcomm | We can try similar approach as for Proposal 2-5rev3, as a compromise. Then we suppose the 2nd sub-bullet under the 2nd sub-bullet is no longer necessary. #### Proposal 2-7rev3 – QC update: - For a set of cells which is configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - Both DCI size and BD/CCE of DCI format 0_X/1_X are counted on all the cell(s) configured with PDCCH candidates for the DCI format 0_X/1_X among the set of cells. - It is up to UE capability and network configuration on how many cell(s) and/or which cell(s) the PDCCH candidates of the DCI format 0_X/1_X can be configured on. The minimum value of the UE capability for the number of cell(s) is 1. - Note: BD/CCE number per scheduled cell is capped by Rel-17 per-cell limit. |
| Fujitsu | Fine with the first bullet. Do not support the second bullet. Fistly, it is not clear what does “cells configured with PDCCH candidates for multi-cell scheduling among the set of cells” mean? Is it same as “cells the search space of the DCI format 0_X/1_X is configured on” as in Proposal 2-5rev3? Secondly, even assume it is, we think it is unnecessary to bundle the DCI size/BD/CCE counting issue with search space configuration issue. What is the benefit to do so? We prefer to decouple the two issues. And for the counting, we prefer to count DCI size and BD/CCE on all the cells of set of the cells and support scaling to avoid duplicate counting. |
| Langbo | Same questions as Fujitsu regarding the bundling of the DCI size/BD/CCE counting issue with search space configuration. Is the intention to have a compromised proposal? We think the cells on which the BD/CCEs are counted have big impacts on UE complexity. We can accept QC’s version as compromise. |
| LGE2 | Fine with the first bullet. Not support the second bullet. Firstly, it is also not clear to us the necessity of the DCI size and BD/CCE counting only on cells configured with PDCCH candidates. Next, it is not clear how DCI size budget is scaled to those cells. Does it mean that DCI size on each cell could be counted to the value less than 1? Rather than that, the DCI size counting on one cell would be a better and simple method. With that, for a simplified/unified UE implementation, BD/CCE counting can also be on one cell. In this perspective, we suggest the following updates. #### Proposal 2-7rev3 - LG update: - For a set of cells which is configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - DCI size and BD/CCE of DCI format 0_X/1_X are counted on one of the cells [configured with PDCCH candidates for multi-cell scheduling] among the set of cells. - [It is up to network which cell(s) the PDCCH candidates of the DCI format 0_X/1_X are configured on.] - Note: BD/CCE number per scheduled cell is capped by Rel-17 per-cell limit. In addition, we don’t think it is needed to discuss the UE capability here. |
| Nokia/NSB | Comments on the last bullet and the note: If we refer to the ‘Rel-17 per cell’ capability, actually the capability per cell has not been changed in Rel-17. What has been changed in Rel-17 is the association of CCEs / BDs to the different cells. This could be maybe made a bit more clear. Which the scaling seems to work well for BDs / CCEs (larger integer number) compared to BDs (does not really help too much there to our understanding) |
| NTT DOCOMO | We cannot get the intention of DCI size budget scaling. Therefore, we suggest to modify the second sub-bullet as follows; - Both DCI size budget and BD/CCE budget of the DCI format 0_X/1_X are is scaled to all the cells configured with PDCCH candidates of the DCI format 0_X/1_X. In addition, we also would like to check the intention of “all cells configured with PDCCH candidates for multi-cell scheduling among the set of cells” in this proposal. It means the cells which are included in the RRC-configured table, i.e., the RRC table in Proposal 3-5rev1, but not actually co-scheduled cell(s), is it correct understanding? |
| Moderator | @Qualcomm: With the concern from LG, can we not mention UE capability and discuss it in UE feature? @Fujitsu @Langbo: For your first question, yes. For your second question, I think we don’t need to configure SS of DCI format 0_X/1_X on each cell of the set of cells or repeat same SS configuration on all cells because we are designing one-to-many scheduling which is different to legacy one-to-one scheduling. Secondly, it gives flexibility to gNB to configure SS on one or multiple cells. For a cell without SS configuration, does it make sense to count DCI size or BD/CCE on it? Anyway, it is up to gNB configuration and gNB can determine which cells or how many cells the SS is configured on. As you prefer scaling operation, this scaling operation is already covered in the proposal. @LG: From section 10.1 of TS 38.213, the UE counts a number of DCI size per scheduled cell based on configured PDCCH candidates, which is described as below: TS 38.213 v17.2.0 Section 10.1 A UE expects to monitor PDCCH candidates for up to 4 sizes of DCI formats that include up to 3 sizes of DCI formats with CRC scrambled by C-RNTI per serving cell. The UE counts a number of sizes for DCI formats per serving cell based on a number of configured PDCCH candidates in respective search space sets for the corresponding active DL BWP. So it makes sense to count DCI size budget and BD/CCE budget of DCI 0_X/1_X on cells configured with SS of DCI 0_X/1_X. Secondly, it gives flexibility to gNB to configure SS of DCI 0_X/1_X on one or multiple cells. Regarding DCI size, I can update the proposal as DOCOMO suggests. @NTT DOCOMO: Regarding “all cells configured with PDCCH candidates for multi-cell scheduling among the set of cells”, it means all cells configured with SS for DCI format 0_X/1_X which is similar to Proposal 2-5rev3(for working assumption). @Nokia: Do you have suggestion on the note? @All: based on suggestions form LG and DOCOMO, the proposal is updated below: #### Proposal 2-7rev5: - For a set of cells which is configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - Both DCI size and BD/CCE of DCI format 0_X/1_X are counted on all cells configured with PDCCH candidates for multi-cell scheduling among the set of cells. - It is up to network which cell(s) the PDCCH candidates of the DCI format 0_X/1_X are configured on. - BD/CCE budget of the DCI format 0_X/1_X is scaled to all the cells configured with PDCCH candidates of the DCI format 0_X/1_X. - Note: BD/CCE number per scheduled cell is capped by Rel-17 per-cell limit. |
| Xiaomi | We don’t think the second subbullet is needed. At least for the DCI size budget, it can be up to the gNB’s control to meet the requirement. |
| Qualcomm | We are not OK with the update due to the reason we have repeated over multiple meetings. We are OK with either: (1) more than per-CC BD limit per scheduled cell as an optional UE capability, or (2) count BD/CCE to each of all the cells associated to the DCI 0_X/1_X, so that # of BDs/CCEs do not exceed per-CC limit for any of the scheduled cell. And our compromised proposal takes (1). We are fine with (2) if there is an alternative proposal to address the concern based on (2). |
| Apple | We would still prefer to have DCI size and BD/CCE of DCI format 0_X/1_X counted on all the cells that can be potentially co-scheduled rather than some set of cells that can be configured with PDCCH candidates. But if the majority is okay with the latest version, we can agree for the sake of progress |
| Qualcomm | We can add a bit more explanation on why we see a technical difficulty on the number of BDs or CCEs. Suppose a UE monitors DCI 1_X for CCs {1, 2, 3, 4} on CC-0 with SCS 30kHz. Per-CC BD limit is 36. For legacy CA, the UE is able to process 144 BDs in total on CC-0 for all the scheduled CCs. The question is for the DCI 1_X, whether to define BD cap as 144 increases the complexity. In the legacy CA, UE can first check the RRC configurations for each scheduled CC and can determine for which scheduled CC the UE start processing PDCCH (ordering or prioritization of PDCCH processes). Suppose a UE has 36 PDCCH decoders. Then, the UE can prioritize PDCCH process for the most time critical CC (based on e.g., TDRA table, max potential TB size, etc). The UE does not need to process all 144 BDs in parallel – UE can first process 36 BDs for the most time critical scheduled CC and then another 36 BDs for the second time critical scheduled CC, and so on. In theory, the UE can implement smaller number of PDCCH decoders than 144, e.g., 36 decoders with successive decodes per 36 BDs. Of course, the UE has to decode all 144 candidates in a slot; however, it is possible for the UE to make order of how to process the candidates. Now with DCI 1_X, it can schedule any of CCs {1, 2, 3, 4}. If all the 144 BDs can be the PDCCH candidates for the DCI 1_X, there is no way for the UE to determine which 36 BDs to process first, since all the candidates are equal. This is our concern on counting BD/CCE for DCI 0_X/1_X to any of the scheduled cells. Next is monitoring DCI 1_X for CCs {1, 2, 3, 4} and legacy DCI for each CC from CCs {1, 2, 3, 4}. Suppose DCI 1_X is counted on CC-1 and the corresponding # of BDs is 36. There is another 36 BDs counted on CC-2, which is for legacy DCI format for scheduling CC-2. Then, there are 36 + 36 = 72 candidates that can carry a DCI format scheduling data on CC-2. The above example UE (having 36 decoders) cannot complete process of PDCCH that schedules data on CC-2 by 36 BDs. Similar to the previous paragraph, the ordering/prioritization is no longer working. Therefore, we prefer to limit BD/CCE numbers per scheduled CC up to the existing per-CC limit, at least for basic UE. This is the basis of PDCCH monitoring for CA from Rel-15. We are open to consider advanced case as an optional if there is a real need, although we do not see an actual scenario where 144 BDs are necessary to carry one DCI format. In summary, we still prefer to our update version in the 2nd round. |
| ZTE | First, we think it would be better to determine this issue after search space related issues determined or discuss these two issues together, because the most important thing is BD/CCE budget. It may affect the flexibility on the network scheduling. It a proposal for BC/CCE budget and CCE budget should be made, we prefer to remove the second sub-bullet under the second sub-bullet. First, DCI size budget scaling to all the cells is not clear to us. In addition, scaling the BD/CCE budget is not good. It may reduce the flexibility because the BD/CCE budget on each cell is reduced compared with the legacy. We think we should follow the legacy procedure as possible due to the limited TU. |
| MTK | We kind of emphasize with QC’s concern. Maybe we can add the basic UE capability suggested to QC to Proposal 2-7rev5. A multi-cell scheduling is a whole new mechanism compared to legacy single cell scheduling, so we think keeping a basic version is good. |
| Huawei2 | We are OK with the first bullet. Regarding the second sub-bullet under 2nd bullet, it is not clear how it is scaled down to each scheduled cells and mostly, such approach does not provide network sufficient flexibility to offload the PDCCH to a cell with better PDCCH coverage/capacity. |
| CMCC | In our views, different ways to count DCI size and BD/CCE number can also work. Regarding DCI size counting for multi-cell scheduling DCI, it is reasonable that DCI size is counted only in one scheduled cell. While for BD/CCE counting, considering that other PDCCH candidates monitored to schedule the selected cell will be limited if BD/CCE only counted on one scheduled cell, it is better that BD/CCE numbers being scaled down to each of co-scheduled cells to provide more scheduling flexibility. |
| Intel | We support the proposal. |
| Vivo4 | We are not clear how to scale BD/DCI size to each cell. as discussed in P2-8, there is only 1 mc-DCI size for the entire set. Assuming the set is {CC1 2 3 4}, while {CC2 CC3} have PDCCH candidate, how to scale the single DCI size to both C22 and CC3? E.g. Counted as 0.5 size for each cell? |
| Samsung3 | We can be OK with the first bullet, but we still see several issues with the second bullet: - There seems to be a majority for counting the DCI size of DCI format 0_X/1_X for a single reference cell (FFS, the reference cell, e.g., based on gNB configuration). We think this can be agreed separately from the BD/CCE issue. - For BD/CCE, the current text implies that such BD/CCE for DCI format 0_X/1_X may be counted only one for a subset of cells on which the search space set is configured (per Proposal 2-5rev3). In our understanding, the purpose of search space linking in Proposal 2-4rev3 is to link a search space set with a set of cells. Once the linkage is identified, the corresponding PDCCH is associated with the set of cells, not only a subset of it. Otherwise, PDCCH resources for different cells will not be efficiently used. In the extreme case that BD/CCE is counted for only one cell from the set of cells, the BD/CCE budget of that cell is quickly “saturated”, while the BD/CCE budget of other co-scheduled cells are wasted. For progress, we can consider RRC-configured scaling factors among the set of cells. So, we suggest the following two sub-proposals (A and B): #### Proposal 2-7rev5A: - For a set of cells which is are configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - DCI size of DCI format 0_X/1_X is counted on one cell among the set of cells - It is up to network configuration which cell the DCI size is counted on. #### Proposal 2-7rev5B: - For a set of cells which is configured for multi-cell scheduling associated with a search space set for DCI format 0_X/1_X (per search space linking procedure), - Both DCI size and BD/CCE of DCI format 0_X/1_X are is counted on all cells configured with for a PDCCH candidates for multi-cell scheduling according to the search space set is counted on all cells among the set of cells. - BD/CCE budget of the DCI format 0_X/1_X is scaled to all the cells among the set of cells configured with PDCCH candidates of the DCI format 0_X/1_X. - At least support scaling factors adding up to 1 among the set of cells. - FFS: whether to support scaling factors adding up to a number >1 among the set of cells - FFS: equal scaling factor for all cells or scaling factor up to gNB configuration - Note: BD/CCE number per scheduled cell is capped by Rel-17 per-cell limit. @Qualcomm: There is no relevance between high spectral efficiency/high power efficiency and how PDCCH candidates are counted. We have also repeatedly stated that we do not support increasing the UE PDCCH monitoring capability to support MC-DCI. In your example, the UE eventually decodes all 144 BDs in a same slot, so power efficiency does not seem to be changed, although different UE implementations may process the PDCCHs with different “speeds”. The only issue is what is the most effective way to distribute the existing PDCCH monitoring capability. |
| Fujitsu | We do not support the second bullet. It is not clear what is “the all cells configured with PDCCH candidates for multi-cell scheduling” referring to. Is it the cells configured with search space as in Proposal 2-5rev3? If yes, we should conclude Proposal 2-5rev3 first (still very controversial) and use the wording aligned with Proposal 2-5rev3. More importantly, what’s the necessity/benefits to bundle the two issues? Even if search space is configured on a subset of scheduled cells, it should be fine to count DCI size/BD/CCE on only one scheduled cell or each of the scheduled cells (with scaling to avoid double counting). |
| Qualcomm | We disagree with Samsung that different UE implementations may process the PDCCHs with different “speeds” do not impact on power efficiency. Ultimately what you are saying is that there is no power impact for supporting FG3-5b (compared to FG3-1). Or, essentially it means that per-CC limit is not necessary (i.e., means total limit is sufficient). To resolve the chaotic situation, we can try another compromise: Proposal: - A UE is allowed to indicate support of the legacy PDCCH BD/CCE/DCI-size budget for a given scheduled cell for any DCI formats. - FFS: details Once the above is agreed, we can accept following main bullets of the proposals. Proposal 2-4: - For a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X and all legacy single cell scheduling DCI format(s) from a same scheduling cell for all cells within the set of cells. Proposal 2-7rev5: - Existing DCI size budget is maintained on each cell of the set of cells. - Both DCI size and BD/CCE of DCI format 0_X/1_X are counted on all cells configured with PDCCH candidates for multi-cell scheduling among the set of cells. |
| Vivo5 | - Both DCI size budget and BD/CCE budget of the DCI format 0_X/1_X are scaled to all the cells configured with PDCCH candidates of the DCI format 0_X/1_X. One more comment, what’s the ‘DCI size budget of the DCI format 0_X/1_X’ for? are we expecting a separate limit for the number of mc-DCI size in addition to ‘3+1’? The original wording ‘DCI size ….of DCI format 0_X/1_X is counted on’ is more accurate. Otherwise it is not clear why should a ‘budget’ be scaled for multiple cells. With regard to Qualcomm's comments on BD difficulty, we think that this is just one possible implementation. Currently, the spec does not impose any restrictions on the ordering of BD or PDCCH processing for different cells, UE only needs to ensure that all 144 BDs are decoded prior to the corresponding scheduled PDSCH transmission. So, we don’t think advanced capability is always needed when counting BD/CCE of on one cell. But we are ok to keep the last note. Besides, counting BD/CCE on one cell is just reusing a similar way as handling DCI format 2-X, i.e, counts DCI format 2-X as part of the Pcell DCI size budget. Thus, we still think counting BD/CCE for one cell is more beneficial. #### Proposal 2-7rev5: - For a set of cells which is configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - Both DCI size and BD/CCE of DCI format 0_X/1_X are counted on all one cells configured with PDCCH candidates for multi-cell scheduling among the set of cells. - It is up to network which cell(s) the PDCCH candidates of the DCI format 0_X/1_X are configured on. - Both DCI size budget and BD/CCE budget of the DCI format 0_X/1_X are scaled to all the cells configured with PDCCH candidates of the DCI format 0_X/1_X. - Note: BD/CCE number per scheduled cell is capped by Rel-17 per-cell limit. |
| CATT | Support the proposal in principle. But, we have concerns on the ‘BD/CCE budget of the DCI format 0_X/1_X is scaled to all the cells configured with PDCCH candidates of the DCI format 0_X/1_X.’ . In legacy BD/CCE counting mechanism, the UE counts the number of BD/CCE per cell based the number of configured PDCCH candidates. In our view, it’s better to reuse the same BD/CCE counting mechanism for multi-cell scheduling, which is the BD/CCE is counted to the cells configured with PDCCH candidate of DCI format 0_X/1_X based on the number of configured PDCCH candidates on the cell. On one hand, this method can enable gNB to allocate the BD/CCE of DCI format 0_X/1_X to the cells based available BD/CCE of each cells, to avoid counting a fixed number of BD/CCE to one cell. On other hand, multi-cell scheduling is extended based on the legacy CA framework, this method is consistent with the current spec without any spec impact. Besides, we don’t see the benefit of scaling the BD/CCE budget to the all cells configured with PDCCH candidate. This means that all cells needs to ensure sufficient BD/CCE for DCI format 0_X/1_X. Assume the BD/CCE of DCI format 0_X/1_X is 30 and counted to the cell1 and cell 2 with scaling factor 0.5, the BD/CCE counted on the cell 1 and cell 2 are 15 separately. But expect the BD/CCE of legacy DCI format, the remaining BD/CCE on cell 1 is 25 and the remaining BD/CCE is 10. In this case, if the scaling factor is configured, although the sum of BD/CCE on cell 1 and cell 2 is sufficient to the BD/CCE of DCI format 0_X/1_X, the BD/CCE of DCI format 0_X/1_X can’t be counted one cell 1 and cell 2. Here, what we want to say is that the scaling factor will cause unnecessary limit to BD/CCE counting rule. We prefer to update the proposal as follows, #### Proposal 2-7rev5: - For a set of cells which is configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - Both DCI size and BD/CCE of DCI format 0_X/1_X are counted on all cells configured with PDCCH candidates for multi-cell scheduling among the set of cells. - It is up to network which cell(s) the PDCCH candidates of the DCI format 0_X/1_X are configured on. - BD/CCE budget of the DCI format 0_X/1_X is counted based on the scaled to all the cells number of configured with PDCCH candidates of the DCI format 0_X/1_X on each cells. - Note: BD/CCE number per scheduled cell is capped by Rel-17 per-cell limit. |
| Moderator6 | @All: based on current inputs, below proposal is provided for further discussion: #### Proposal 2-7rev6: - For a set of cells which is configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - DCI size of DCI format 0_X/1_X is counted on one cell among the set of cells configured with PDCCH candidates for multi-cell scheduling. - It is up to network which cell DCI size of the DCI format 0_X/1_X is counted on. - BD/CCE budget of the DCI format 0_X/1_X is counted on all the cells configured with PDCCH candidates of the DCI format 0_X/1_X. - FFS: counting details - Note: BD/CCE number per scheduled cell is capped by Rel-17 per-cell limit. |
| CATT | We are ok with the first, third and last sub-bullet. To limit the complexity of PDCCH monitoring per cell, the number of PDCCH candidates per cell is subject to BD/CCE budget and the number of DCI size is subject to ‘3+1’ DCI size budgets. The BD/CCE budget per cell is used to limit the maximum number of attempts for PDCCH monitoring, and the DCI size budget is used to limit maximum number of DCI sizes assumed by decoder. If the BD/CCE of DCI format 0_X/1_X is counted on one or more cells, then corresponding DCI size should be counted on those cells as well. This is because BD/CCE counting and DCI size counting cannot be decoupled from the perspective of PDCCH monitoring capability. Thus, we suggest to modify the second bullet as follows: - DCI size of DCI format 0_X/1_X is counted on one or more cells among the set of cells configured with PDCCH candidates for multi-cell scheduling. |
| Nokia/NSB | We are a bit worried about the change from ‘scaling’ to ‘counting’ for the BD/CCE: If this still leaves the option open to count all the BDs / CCEs of 0_X/1_X for all the configured cells without any specific ‘scaling’ – then we really don’t see a need to support the configuration of the PDCCH candidates to more than one cell. |
| Apple | Although we prefer that both DCI size and BD/CCE counting have similar mechanism, but, as a compromise, we are fine to support the proposal 2-7rev6. We tend to agree with Nokia to possibly change the wording from counting to “FFS: scaling/counting details” |
| Qualcomm | On BD/CCE: We are not sure whether companies have common understanding on the note of Proposal 2-7rev6, “Note: BD/CCE number per scheduled cell is capped by Rel-17 per-cell limit”. Our understanding is that existing per-cell BD/CCE limit applies for any DCI formats (including DCI 0_X/1_X and legacy DCIs) for a given scheduled cell. It seems some companies have different understanding. We would like to make sure that at least one type of UEs is allowed to support max number of BDs for any DCI formats for a scheduled cell = Rel-17 per-cell limit, min⁡(MPDCCHmax,slot,μ, MPDCCHtotal,slot,μ). Same for CCE limit. To address this, there are following options: - Opt.1: Count BD/CCE of a DCI 0_X/1_X on all the cells associated with the DCI 0_X/1_X (as Apple proposed earlier). - Opt.2: Count BD/CCE of a DCI 0_X/1_X on all the cells configured with PDCCH candidates of the DCI 0_X/1_X, and the number of BDs/CCEs for any DCI formats for a scheduled cell does not exceed Rel-17 per-cell limit - For PCell, PDCCH overbooking applies - For SCell, network must guarantee this - Opt.3: Count BD/CCE of a DCI 0_X/1_X on all the cells configured with PDCCH candidates of the DCI 0_X/1_X, where the numbers of cell(s) that can be configured with PDCCH candidates of the DCI 0_X/1_X and that can be configured with PDCCH candidates of the legacy DCI formats are up to the UE capability (minimum value of these UE capabilities is 1, in which case the Rel-17 per-cell limit can be maintained for any DCI formats for a given scheduled cell). Proposal 2-7rev6 excludes Opt.1. In addition, if our understanding on the note is correct,Opt.3 is also excluded. We would like to know if the intention is to go with Opt.2 above. DCI size: We prefer to leave this after some more progress on BD/CCE and Proposal 2-8rev1. |
| LGE | After observing the arguments from the companies related to complexity/implementation in the UE as well as the spec so far, to avoid further/complicated work considering limited TU, we think it would be reasonable to update as below to simplify PDCCH framework for multi-cell scheduling. #### Proposal 2-7rev6: - For a set of cells which is configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - DCI size of DCI format 0_X/1_X is counted on one cell among the set of cells configured with PDCCH candidates for multi-cell scheduling. - It is up to network which cell DCI size of the DCI format 0_X/1_X is counted on. - BD/CCE budget of the DCI format 0_X/1_X is counted on one cell among the set of cells all the cells configured with PDCCH candidates of the DCI format 0_X/1_X. - FFS: counting details - Note: BD/CCE number per scheduled cell is capped by Rel-17 per-cell limit. |
| Qualcomm | LGE’s proposal set over Proposal 2-5 rev4(for working assumption) and Proposal 2-7rev6 with the FL Proposal 2-6(for working assumption) makes the design being consistent overall, and would be a good starting point to complete the discussion: For a DCI 0_X/1_X associated with a cell set, (1) single n_CI value is configured for CCE determination, (2) SS set(s) linked to a single cell is configured for monitoring, and (3) BD/CCE and DCI size are counted on the single cell. The single cell for SS set(s) monitoring and BD/CCE/DCI size counting for DCI 0_X/1_X is the one associated to the n_CI value configured for the cell set. We can be supportive with this direction but the following needs to be resolved. Consider following: - A DCI 0_X/1_X for CCs {1, 2, 3, 4} is monitored on CC-0 - The n_CI value for the DCI 0_X/1_X for CCs {1, 2, 3, 4} is set to 1 - BDs/CCEs/DCI-sizes for the DCI 0_X/1_X are counted on CC-1 - [Due to the agreed WA based on (Merged)Proposal 2-3 and 2-4 rev4] The UE may monitor legacy DCI formats for all of CCs {1, 2, 3, 4} on CC-0, as well as DCI 0_X/1_X for CCs {1, 2, 3, 4} - The n_CI values for the legacy DCI formats for CCs {1, 2, 3, 4} are set to 1, 2, 3, 4, respectively For simplicity, we can focus on BD number here. The BD number for DCI 0_X/1_X is counted on CC-1 and the BD number for legacy DCI formats for CC-2 is counted on CC-2. - If the total number of BDs for any DCI formats for CC-2 is capped by Rel-17 per-CC limit (as per note in the last bullet), the sum of BDs for DCI 0_X/1_X for CCs {1, 2, 3, 4} and BDs for legacy DCI formats for CC-2 is capped by a single value. Then there is no point to count BDs for DCI 0_X/1_X for CCs {1, 2, 3, 4} on CC-1. This eventually is equivalent to the case where BDs of DCI 0_X/1_X are counted on all the CCs {1, 2, 3, 4}. - If the total number of BDs for DCI formats for CC-2 is NOT capped by Rel-17 per-CC limit (as per some other companies’ understanding), then the UE is not able to get a grant (or is not able to identify there is no grant) within the number of BDs = Rel-17 per-CC limit, which is not acceptable to us. We are OK with the above 1, but then we need to revise the proposal such that BD counting is on all the cells in the set of cells associated with DCI 0_X/1_X. For the above 2, we can accept if having UE capability is agreed together. For the UE capability, there are two options. - Opt.1: UE capability indicates whether the BDs for DCI 0_X/1_X are counted on all the cells in the cell set, or counted on one of the cells in the set. - Opt.2: UE capability indicates whether the UE can monitor legacy DCI format for one of the cells in the cell set, or for more than one cells in the cell set. Considering that there are a lot of companies stating that legacy DCI formats for all the cells in the cell set are necessary (though we do not agree with this), we can go with the above 2 with Opt.1. LGE’s proposal can be updated as follows. We are also open to Opt.1 above. #### Proposal 2-7rev6: - For a set of cells which is configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - RAN1 support following two options. - Opt.a: - DCI size of DCI format 0_X/1_X is counted on all the cells among the set of cells. - BD/CCE of the DCI format 0_X/1_X is counted on all the cells in the set of cells - Opt.b: - DCI size of DCI format 0_X/1_X is counted on one cell among the set of cells configured with PDCCH candidates for multi-cell scheduling. - It is up to network which cell DCI size of the DCI format 0_X/1_X is counted on. - BD/CCE budget of the DCI format 0_X/1_X is counted on one cell among the set of cells all the cells configured with PDCCH candidates of the DCI format 0_X/1_X. - FFS: counting details - Note: BD/CCE number per scheduled cell is capped by Rel-17 per-cell limit. |
| Ericsson2 | For P2-7rev6 - 2nd bullet : In our understanding, the first bullet implies that the DCI size budget per scheduled cell is maintained, which means the “3+1” rule is maintained per-scheduled cells. Given that, clarification is needed on intention of 2nd bullet, particularly what the meaning of “DCI size of DCI format 0_X/1_X counting”. Why isn’t counting per-scheduled cell with 3+1 budget enough? The subbullet “it is up to network” seems to be adding unnecessary complexity and should be removed. - 3rd bullet : Similar question as for 2nd bullet. Clarification is needed on the meaning of “BD/CCE budget of DCI format 0-X/1-X is counted”. As we mentioned in our tdoc, duplicate counting of BD/CCE candidate for mc-DCI should be avoided, and this should be reflected in the update. We suggest adding the latter as a separate note. - For note added by QC, we suggest reusing the same note as from the RAN1 agreement made this week, or at least clarify what the difference would be. - Also suggest to remove “configured with PDCCH candidates for multi-cell scheduling” as it would already be reflected by main bullet, i.e. it is for “• For a set of cells which is configured for multi-cell scheduling,”. This can also separate out the RRC configuration aspect of whether linked search space in on only one cell of the set or more than one cell of the set. Suggested updates are as below. #### Proposal 2-7rev6: - For a set of cells which is configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - DCI size of DCI format 0_X/1_X is counted on each cell among the set of cells. - BD/CCE budget of the DCI format 0_X/1_X is counted on all the cellsof the set of cells. - FFS: counting details Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., and ) for PDCCH candidates for each scheduled cell. Note : Duplicate counting of a BD/CCE attempt for 0_X/1_X is avoided when counting BD/CCE across co-scheduled cells for comparison against the aggregate limits MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ. |
| Fujitsu | Please refer to our detailed comments in Proposal 2-5. Here in short, we believe the counting can be independent to RRC configuration of search space. There is no convinced benefit/necessity to bundle the two things together. We can accept the updated Proposal 2-7rev6 from Ericsson. |
| Moderator2 | @All: Proposal 2-7 and Proposal 2-5 have been merged as below for further discussion. So far, the discussion on this proposal is also in e-mail. You are welcome to comment via e-mail. Merged P2-7 and P2-5 rev1: - For a set of cells which is configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - RAN1 specification supports following two options. - Option A: - DCI size of DCI format 0_X/1_X is counted on all the cells among the set of cells. - BD/CCE of the DCI format 0_X/1_X is counted on all the cells in the set of cells. - Search space of DCI format 0_X/1_X is configured on all the cells of the set of cells and associated with the search space of the scheduling cell with the same search space ID. - Option B: - DCI size of DCI format 0_X/1_X is counted on one cell among the set of cells. - FFS which cell DCI size of the DCI format 0_X/1_X is counted on. - BD/CCE of DCI format 0_X/1_X is counted on one cell among the set of cells. - FFS which cell BD/CCE of the DCI format 0_X/1_X is counted on. - Search space of DCI format 0_X/1_X is configured on one cell of the set of cells and associated with the search space of the scheduling cell with the same search space ID. - FFS which cell the SS of the DCI format 0_X/1_X is configured on. - It is subject to UE capability to support one or both of the options. - Note: BD/CCE number per scheduled cell is capped by Rel-17 per-cell limit. - Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell. |

#### Proposal 2-8rev1:

- For a set of cells which can be scheduled by DCI format 0_X/1_X, the payload size of DCI format 0_X/1_X is derived by UE based on RRC configuration and same for the set of cells.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| New H3C | OK in principal |
| Qualcomm | We are OK with the proposal. |
| Langbo | Support |
| LGE2 | Generally fine. But, for clarity, we suggest the following updates. - For a set of cells which can be scheduled by DCI format 0_X/1_X, the payload size of DCI format 0_X/1_X is derived by UE based on RRC configuration of co-scheduled cell combination and same for the set of cells. - The payload size 0_X/1_X is the same for all the co-scheduled cell combinations |
| Nokia/NSB | Support. And we would support the addition / clarification by LGE2 above. |
| NTT DOCOMO | Support. |
| Xiaomi | Clarification with the “same for the set of cells” is needed. |
| Apple | Fine to support |
| ZTE | We are fine with proposal. |
| MTK | Support. As for LGE2’s supplement, does it mean the payload size would be same when one cell is scheduled or four cells are scheduled? Wouldn’t that introduce heavy zero padding? |
| Huawei2 | Support |
| Vivo3 | Support the spirit of the proposal, if I understand correct, further clarification is needed #### Proposal 2-8rev1: - For a set of cells which can be scheduled by DCI format 0_X/1_X, the payload size of DCI format 0_X/1_X is derived by UE based on RRC configuration, and the payload size is the same for all the co-scheduled cell combinations within the set of cells. |
| CMCC | We are fine with the proposal. |
| Intel | We support the FL proposal. |
| Moderator | @LG @Nokia @vivo: Your wording is clear and OK to me. Considering DCI 0_X may have different size than DCI 1_X, let me split the two DCI formats. @xiaomi: It means there is a single size for the set of cells. @MTK: Yes, your understanding is correct. @All: Please check below update: #### Proposal 2-8rev2: - For a set of cells which can be scheduled by DCI format 0_X/1_X, the payload size of DCI format 0_X/1_X is derived by UE based on RRC configuration of co-scheduled cell combinations within the set of cells. - The payload size 0_X is the same for all the co-scheduled cell combinations. - The payload size 1_X is the same for all the co-scheduled cell combinations. |
| Samsung3 | Support the revised proposal in principle, although the wording can be slightly improved |
| Nokia/NSB | Support the updated version by moderator. |
| Apple | Support the latest version by moderator |
| CATT | Support the latest version by moderator |
| Qualcomm | Is it correct understanding below? - The payload size of a DCI format 0_X is determined based on RRC configuration of the set of cells that can be scheduled by the DCI format 0_X (with possible DCI size alignment procedure, if any and if supported). Once the size is determined, the size does not change regardless of which cell(s) the DCI format 0_X actually schedules data amongst the set of cells. - The payload size of a DCI format 1_X is determined based on RRC configuration of the set of cells that can be scheduled by the DCI format 1_X (with possible DCI size alignment procedure, if any and if supported). Once the size is determined, the size does not change regardless of which cell(s) the DCI format 1_X actually schedules data amongst the set of cells. If so, we are OK with the Proposal 2-8rev2. |
| LGE | Support the latest version by moderator |
| Moderator | @QC: Yes, your understanding is correct. That’s the intention of this proposal. |
| Ericsson2 | We can be fine with the principle of the subbullets, but we still do have concern the DCI size alignment procedure which is still a bit unclear at this time, and considering there can be up to six DCI formats (1-1/0-1/1-2/0-2/1-X/0-X). We would be OK with agreeing to the subbullets, but would like to leave the size derivation part FFS for now, including anadditional option of explicit configuration. Below are suggested updates. #### Proposal 2-8rev2: - For a set of cells which can be scheduled by DCI format 0_X/1_X, - The payload size 0_X is the same for all the co-scheduled cell combinations within the set of cells. - The payload size 1_X is the same for all the co-scheduled cell combinations within the set of cells. - FFS: the payload size of DCI format 0_X/1_X is derived by UE based on RRC configuration of co-scheduled cell combinations within the set of cells or is explicitly configured by RRC signaling. |
| Nokia/NSB | We support the proposed Ericsson changes, which make it clear what is agreed and what is still open for discussion. |
| Moderator2 | @Ericsson: Given the current situation, I think the feasible way is to derive DCI size of DCI format 0_X/1_X as what we have done for DCI format 0_1/1_1. With your proposed update, seems there is no progress. |
| Ericsson3 | Regarding FL comment, we think our proposed text makes the payload size across different combinations clear, which is essentially the two subbullets. How to achieve the size of mc-DCI should be discussed further, particularly considering the DCI size alignment aspects are pending. |

## Other related issues

MediaTek - Proposal 1: RAN1 to consider both 1-segment and 2-segment DCI design (as shown in Figure 1) and compare their pros and cons to support R18 multi-cell PUSCH/PDSCH scheduling with a single DCI. - Proposal 12: For the 2-segment aggregated DCI, the 1st and 2nd segment DCI are decoded separately on the same scheduling cell. The 1st and 2nd segment DCI are then linked together to form one multi-cell scheduling DCI. The link procedure of 1st and 2nd segment DCI can be based on some designated DCI bit values of the 1st or 2nd segment DCI - The 2nd segment is identified by one additional DCI bit for both 1st/2nd segment DCI (Ex. 0 1st segment, 1 2nd segment) - The 1st and 2nd segments are linked if they are on the same scheduling cell and same slot/symbol - Among cells with bitmap value 1 in the 1st segment, scheduling information of the cell with lowest ServCellIndex is put in the 1st DCI segment. - Add one additional IE SegmentId under SearchSpace to specify this search space is for 1st segment or 2nd segment - The linked 1st segment and 2nd segment DCI should be “both DL scheduling DCIs” or “both UL scheduling DCIs” - Use the CCE index formula in 38.213 10.1 to stagger the 1st DCI segment and 2nd DCI segment, where for the 2nd DCI segment, nCI is replaced by the corresponding RRC configuration, for example, under SearchSpace, or a predefined number; for the 1st DCI segment, nCI is set to zero. - For 2-segment DCI, the short segment shares the same DCI length as fallback DCI (0_0 and 1_0, with potential zero padding), and the long segment shares a longer DCI length (with potential zero padding) - Long DL segment size and long UL segment size can still be different

# DCI field design

Based on contributions submitted by companies, below issues are prioritized for discussion in this meeting. Within each sub-section, the summary from moderator’s perspective is listed and followed by draft proposals for further discussion round by round.

## DCI field types

Huawei, HiSilicon - Proposal 5: Fields in multi-cell scheduling DCI 1-X are advised to classify as follows: - Type-1A: Enhanced Type 3 codebook indicator, HARQ-ACK retransmission indicator, Priority Indicator, Minimum applicable scheduling offset indicator, Scell dormancy indication, PDCCH monitoring adaptation indication, PUCCH Cell indicator. - Type-2: Modulation and coding scheme, HARQ process number. - Type-3: Time domain resource assignment, Frequency domain resource assignment, VRB-to-PRB mapping, Rate matching indicator, Bandwidth part indicator, Antenna port(s), SRS request, ZP CSI-RS trigger, PRB bundling size indicator, DMRS sequence initialization, Transmission configuration indication, UL/SUL indicator. - Proposal 6: Fields in multi-cell scheduling DCI 0-X are advised to classify as follows: - Type-1A: 1st downlink assignment index, 2nd downlink assignment index, Beta_offset indicator, UL-SCH Indicator, Priority indicator, Invalid symbol pattern indicator, Minimum applicable scheduling offset indicator, Scell dormancy indication, PDCCH monitoring adaptation indication, PUCCH Cell indicator. - Type-2: Modulation and coding scheme, HARQ process number. - Type-3: UL/SUL indicator, Bandwidth part indicator, Time domain resource assignment, Frequency domain resource assignment, Frequency hopping flag, TPC command for scheduled PUSCH, Second TPC command for scheduled PUSCH, SRS resource indicator (SRI), Precoding information and number of layers, Second Precoding information, Antenna port, SRS request, SRS offset indicator, CSI request, PTRS-DMRS association, DMRS sequence initialization, Open-loop power control parameter set indication. - Proposal 7: Fields are not recommended to be included in multi-cell scheduling DCI: - PDSCH group index, New feedback indicator, Number of requested PDSCH group(s), CBG transmission information, CBG flushing out information, Sidelink assignment index. ZTE - Proposal 2: Some fields should not be included in the MC-DCI. - Proposal 3: Except for the fields that must be separately indicated (e.g., NDI, RV), 2 sub-groups can be designed for a configurable field, where each sub-group corresponds to one separate indication of this field. - Proposal 4: The fields are categorized as below. - Type-1 field: - Type-1A:HPN - Type-1B:Indicator of co-scheduled cells - Type-1C:CSI request - Type-2 field: RV with 1 bit - Type-3 field: PRB bundling size indicator, Rate matching indicator, ZP CSI-RS trigger, Antenna port(s), SRS request, DMRS sequence initialization, TPC for scheduled PUSCHs, Modulation and coding scheme, Bandwidth part indicator, Time domain resource assignment, Frequency domain resource assignment, VRB-to-PRB mapping, SRI - Proposal 5: The frequency resource indication for the co-scheduled cells with different BWP sizes in the case of shared FDRA field should be specified. Spreadtrum Communications - Proposal 9: According to sub-group Type-3 - One cell in the configuration cells for co-scheduling always belongs to a same sub-group, for all the fields that is configured as sub-group type. - Type-1A/1C can be used for each sub-field of sub-group Type-3, while Type-B cannot - Size of bits in sub-group Type-3 depend on the maximum bit length of all cells in the cell group - Proposal 10: Only support same priority of PUSCHs on multi-cells scheduled by one DCI0_X, and same priority of HARQ-ACK of PDSCHs on multi-cells scheduled by one DCI 1_X in Rel-18. - Proposal 11: Priority indicator can be Type-1A, or not exist if only allows low priority scheduling. - Proposal 12: The fields in DCI format 1_X are with the below type classification: - Type-1 fields include below: - Carrier indicator co-scheduled cells (Type-1B) - Time domain resource assignment (Type-1B) - Priority indicator (Type-1A or NA) - Enhanced Type 3 codebook indicator (Type-1A) if eType-3 HARQ-ACK is supported - PDCCH monitoring adaptation indication (Type-1C) - Type-3 fields include below: - Frequency resource related - Frequency domain resource assignment - VRB-to-PRB mapping - Rate matching indicator - MIMO related: - PRB bundling size indicator - ZP CSI-RS trigger - Antenna port(s) - TCI - SRS request - CSI request - DMRS sequence initialization - SRS offset indicator - Modulation and coding scheme - HARQ process number - Fields not needed or supported for multi-carrier scheduling include below: - Bandwidth part indicator - Second TPC command for scheduled PUCCH - Transport block 2 - Modulation and coding scheme - NDI - RV - CBG - CBG transmission information - CBG flushing information - HARQ-ACK retransmission indicator - Minimum applicable scheduling offset indicator - Scell dormancy indication - Proposal 13: The fields in DCI format 0_X are with the below type classification: - Type-1 fields include below: - Carrier indicator o/Indicator/bitmap of co-scheduled cells (Type-1B) - Time domain resource assignment (Type-1B) - Priority indicator (Type-1A or NA) - PDCCH monitoring adaptation indication (Type-1C) - Type-2 fields include below: - TPC command for scheduled PUSCH (Type2) - Type-3 fields include below: - Frequency resource related - Frequency domain resource assignment - Frequency hopping flag - MIMO related: - SRS resource indicator - Precoding information and number of layers - Antenna port(s) - SRS request - SRS offset indicator - CSI request - PTRS-DMRS association - DMRS sequence initialization - Modulation and coding scheme - HARQ process number - beta offset indicator - Fields not needed or supported for multi-carrier scheduling include below: - DFI flag - Bandwidth part indicator - SRS resource set indicator - Second SRS resource indicator - Second Precoding information - Second PTRS-DMRS association - UL-SCH indicator - Transport block 2 - Modulation and coding scheme - NDI - RV - CBG - CBG transmission information - Invalid symbol pattern indicator - Minimum applicable scheduling offset indicator - Scell dormancy indication - Open-loop power control parameter set indication Vivo - Proposal 6. DCI format 0_X/1_X includes - Type-1A: - Priority index at least for DL scheduling - Type-1C - CSI request - Beta-offset - Type-2 fields include: - MCS - Type-3 field: - FDRA - TDRA - VRB/PRB - PRB bundling - Rate matching - SRS - DMRS initialization - Proposal 7. Scheduling granularity of FDRA can be scaled or determined considering the BW of all the scheduled cells to reduce DCI size. - Proposal 8. The size of a field applying to one or more scheduled cells in a DCI format 0_X/1_X is determined based on the configuration of the corresponding co-scheduled cell(s), and/or the configuration of the scheduling cell. If multiple sizes for DCI format 0_X/1_X are obtained after determining each field size, DCI format 0_X/1_X with a smaller size should be zero-padded to align to the largest DCI format 0_X/1_X size among all the DCI format 0_X/1_X. China Telecom: - Proposal 1: Support co-scheduled UL cell configured with SUL for multi-cell PUSCH scheduling with a single DCI. - Proposal 4: The following are considered for the design of frequency domain resource assignment field: - Same resource allocation type for each of the cells to be co-scheduled. - Larger RBG size based on the total bandwidth of all the BWPs of the cells to be co-scheduled, and one RBG mapped to a single cell for resource allocation type 0. - Joint indication of the respective RIV for each of the cells to be co-scheduled, and configured resource allocation granularity for resource allocation type 1. - Proposal 5: For bandwidth part indicator field, when it is configured to indicate different BWPs for the cells to be co-scheduled, for only one cell the initial BWP is supported to be indicated by the field. - Proposal 6: In DCI format 0-X/1-X, the bits containing separate scheduling information of a type 2 or 3 field are mapped to cells or sub-groups of cells that can be co-scheduled according to the ServCellIndex increasing order, and the bits of the same field containing scheduling information for the different cells or sub-groups of cells are consecutive. OPPO: - Proposal 2: Common indication can be used for TDRA field. Legacy TDRA table or new TDRA table could be considered. - Proposal 3: The same frequency domain resource allocation type from {Type-0 only, Type-1 only, dynamic switch} shall be configured for all cells potentially scheduled by one DCI, and the same type-0 or type-1 FDRA type should be used for all cells actually scheduled by one DCI. - Proposal 4: FDRA field could be a shared DCI field (type-1 as agreed in RAN1 #109/#110). The bit size of this single FDRA field is determined by the maximum size of FDRA indication required across all the potentially scheduled cells. - Proposal 5: MCS indication shall be separated for each scheduled PDSCH/PUSCH. - Proposal 6: HARQ process number field could be a common indication for scheduled cells. - Proposal 7: The DCI field for power control can be separated per scheduled cell. CATT: - Proposal 9: For DCI format 0_X/1_X, the fields of indication of co-scheduled cells, TDRA, HARQ process ID can be designed as Type-1A field. - Proposal 10: For DCI format 0_X/1_X, the fields of MCS can be indicated by a reference MCS and (N-1) offset MCS via joint indication to indicate MCS for the co-scheduled cells, where N is the maximum number of co-scheduled cells. - Proposal 11: The bitwidth of each NDI for a co-scheduled cell and each RV for a co-scheduled cell is 1 bit and 2 bits for each cell, respectively. - Proposal 12: The following fields are suggested to be designed as type-3 field: - The fields in DCI format 0_X: FDRA, frequency hopping flag, SRS indicator, precoding and layers, antenna port and PTRS-DMRS can be designed as Type-3 field. - The fields in DCI format 1_X: FDRA, VRB-to-PRB, PRB bundling size, rate matching indicator, antenna port and TCI state can be designed as Type-3 field. Fujitsu: - Proposal 1: At least the following DCI fields can be included in DCI format 1_X. - Type-1 - Carrier indicator (indicator of scheduled cells) - Type-2 - TB1: Modulation and coding scheme - HARQ process number - Type-3 - Bandwidth part indicator - Frequency domain resource assignment - Time domain resource assignment - VRB-to-PRB mapping - PRB bundling size indicator - Rate matching indicator - ZP CSI-RS trigger - Antenna port(s) - Transmission configuration indication - SRS request - DMRS sequence initialization - Proposal 2: At least the following DCI fields can be included in DCI format 0_X. - Type-1 - Carrier indicator (indicator of scheduled cells) - CSI request - beta_offset indicator - UL-SCH indicator - Type-2 - Modulation and coding scheme - HARQ process number - Type-3 - Bandwidth part indicator - Frequency domain resource assignment - Time domain resource assignment - Frequency hopping flag - TPC command for scheduled PUSCH - SRS resource indicator - Precoding information and number of layers - Antenna port(s) - SRS request - PTRS-DMRS association - DMRS sequence initialization Intel: - Proposal 3 - Support Type-3 DCI field for multi-cell scheduling: - Type-3 field: Common or separate to each of the co-scheduled cells, or separate to each sub-group, dependent on explicit configuration. - Proposal 4 - Type-1 DCI field for multi-cell scheduling at least includes - Time domain resource assignment - BWP and cell index - HARQ process number - DMRS sequence initialization - beta offset indicator - CSI request - Rate matching indicator - ZP CSI-RS trigger - VRB-to-PRB mapping - SRS request - Proposal 5 - For multi-cell scheduling, carrier indication field is included in the DCI to determine a set of carriers from a configured table. - Proposal 6 - For multi-cell scheduling, a row of the TDRA table can configure separate resource allocation in time for all the configured cells. - Proposal 7 - Type-3 DCI field for multi-cell scheduling at least includes - Modulation and coding scheme - Frequency domain resource assignment CAICT: - Proposal 1: Further enhancements are considered to overcome the scheduling restriction if CRC bit filed is designed as a type-1 field. - Proposal 2: CRC bit filed in DCI format 0_X/1_X is considered as Type-1B field. Multiple predefined RNTIs are used to discriminate different scheduling modes of the scheduled PDSCH/PUSCH by DCI format 0_X/1_X. Xiaomi: - Proposal 9: The “Indicator of the co-scheduled cells” should be a type-1 DCI field and could be a type-1A or type-1B DCI field subject to the indication mechanism. - Proposal 10: The “Frequency domain resource assignment” and “HARQ process number” should be type-2 DCI fields - Proposal 11: The “CBGTI”, “CBGFI” and “Bandwidth part indicator” shouldn’t be included in a MC-DCI. - Proposal 12: All other fields can belong to the type-3 DCI field. Lenovo - Proposal 3: BWP indicator in DCI format 0_X/1_X is separate for each of co-scheduled cells. - Proposal 4: Frequency domain resource allocations for co-scheduled cells are joint indicated by FDRA field in DCI format 0_X/1_X by pointing to one FDRA combination from a RRC configured list. - Proposal 5: Time domain resource allocations for co-scheduled cells are joint indicated by TDRA field in DCI format 0_X/1_X by pointing to one TDRA combination from a RRC configured list. - Proposal 6: VRB-to-PRB mapping, PRB bundling size indicator, and rate matching indicator in DCI format 0_X/1_X are shared or separate to co-scheduled cells dependent on RRC configuration. - Proposal 7: Separate MCS fields are included in DCI format 0_X/1_X. - Proposal 8: Separate HARQ process numbers are included in DCI format 0_X/1_X. - Proposal 9: ZP CSI-RS trigger, Antenna port(s), TCI, SRS request, and DMRS sequence initialization in DCI format 0_X/1_X are shared or separate to co-scheduled cells dependent on RRC configuration. - Proposal 10: Single bit priority indicator is included in DCI format 0_X/1_X indicating same priority for co-scheduled cells. - Proposal 11: Single ChannelAccess-Cpext is included in DCI format 0_X/1_X indicating same channel access information for co-scheduled cells. - Proposal 12: PDCCH monitoring adaptation indication, PUCCH Cell indicator, Minimum applicable scheduling offset indicator, and Scell dormancy indication are excluded from DCI format 0_X/1_X. - Proposal 13: CBGTI/CBGFI, PDSCH group index, new feedback indicator, and number of requested PDSCH groups are excluded from DCI format 0_X/1_X. CMCC: - Proposal 10. FDRA field can be configured as Type-1 field to reduce DCI size, and the following designing can be considered. - The FDRA field size can be determined by the minimum BWP size among BWPs of all co-scheduled cells. - For resource allocation type 0, the common indication for bitmap and RBG size is used for all co-scheduled cells, and UE interprets the indication respectively to each scheduled cell with scaling factor K. - For resource allocation type 1, the common indication of RIV is used for all co-scheduled cells, and the UE can interpret the indication respectively with the scaling factor K. - Proposal 11. The fields in DCI format 1_X can be classified as the following: - Type-1 fields include below: - Type-1A fields: - Identifier for DCI formats - Downlink assignment index - TPC for scheduled PUCCH - PUCCH resource indicator - Indication of scheduled cells - PDSCH-to-HARQ_feedback time - DMRS sequence initialization - Frequency domain resource assignment - Type-2 fields include below: - Modulation and coding scheme - New data indicator - Redundancy version - Type-3 fields include below: - Bandwidth part indicator - Time domain resource assignment - VRB-to-PRB mapping - PRB bundling size indicator - Rate matching indicator - ZP CSI-RS trigger - HARQ process number - Antenna port(s) - Transmission configuration indication - SRS request FGI: - Proposal 1: Adopt the type classification of DCI fields for DCI format 1_X in Table 1 and DCI format 0_X in Table 2. LG: - Proposal #4: Clarify and update the classification of DCI field types and details of each field type as the followings. - Type-1C: A single field indicates an information to only one of co-scheduled cells, and a (predefined or preconfigured) default value is applied to other co-scheduled cells. - Type-2 (& 3): The size of separate field per cell (or sub-group) is reduced compared to the case of single-cell scheduling to avoid increase of for multi-cell DCI payload size. - Type-3: The cell sub-grouping is done per field (or field group) for scheduling (DCI) efficiency/ flexibility, based on all the configured cells (not per co-scheduled cell combination). - Type-4 (new): This field is omitted in multi-cell scheduling case (in this case, a default value is applied to the scheduled cells), but it is present in single-cell scheduling case. - Proposal #5: Classify each DCI field (in current DL/UL DCI format) as in Tables 1 and 2, based on the Type-1/2/3/4 field with clarifications/updates in Proposal #4. - Need to consider and address the FFS points listed in “Note” column for some DCI fields in the Tables 1 and 2. - Consider how to do the ordering of multiple fields corresponding to different cells or sub-groups in the multi-cell DCI. - Proposal #6: Consider to limit the maximum number of co-scheduled TBs (e.g. to 4) for multi-cell PDSCH scheduling to avoid DCI size increase due to separate MCS/RV/NDI indication per TB. - Proposal #7: Consider to define the maximum number of configured cells for multi-cell scheduling as UE capability considering trade-off between performance and complexity. Apple: - Proposal 9: MCS field belong to Type-2, with separate indication for each co-scheduled cell Samsung: - Proposal 4: Support the following fields in MC-DCI as Type-1 fields: - Type-1A fields, such as: frequency hopping (FH), TCI state; - Also, Type-3 field that are configured as cell-common (Type-1A), such as: TDRA, FDRA, antenna port (Aps), SRI, TMPI. - When TDRA is determined as Type-1A field, the MC-DCI indicates a same row index from corresponding legacy single-cell TDRA tables or new multi-cell TDRA tables. - When FDRA is determined as Type-1A field, and the co-scheduled cells have active DL/UL BWPs with different size (i.e., different number of RBs), the UE determines the FDRA for each cell from corresponding LSBs of the FDRA field. - Type-1B fields, such as: Rate matching indicator, aperiodic ZP CSI-RS, SRS request, TCI state; - Also, Type-3 field that are configured as cell-specific and determined to be Type-1B, such as: TDRA. - When TDRA is determined as a Type-1B field, the MC-DCI indicates a row index from a joint multi-cell TDRA table. - Type-1C fields, such as: CSI request, UL-SCH, and beta offset. - Note 1: Depending on the interpretation, “cell-set-level CIF” can be considered a Type-1A or Type-1B field, without any impact on the UE behavior. - Note 2: When TCI state field is present in MC-DCI format, the corresponding value provides new indicated DL/UL/joint TCI state for future transmission/receptions (per Rel-17 unified TCI framework). - Proposal 5: Adopt HPN and MCS as additional Type-2 fields in an MC-DCI format. - Also Type-3 fields that are configured as cell-specific and determined to be Type-2, such as, FDRA, SRI, TPMI; - Support configurable size for RV, HPN, as well as Type-3 fields that are determined as Type-2, such as SRI and TPMI; - Support differential indication for MCS. - Proposal 6: Bits of Type-2 fields for non-scheduled cells, instead of being reserved, are used to improve scheduling/throughput by improving accuracy of values for Type-2 fields (e.g., FDRA) for scheduled cells. - Proposal 7: Support the following parameters that are applicable to multi-cell scheduling without values in the MC-DCI: - BWP-ID: per RRC configuration or follow the most recent active BWPs for the corresponding cells as indicated by SC-DCI formats (when indication in MC-DCI is not configured by RRC); - UL/SUL indicator: per spec (e.g., only NUL) or per RRC configuration, or follow the most recent uplink carrier for the corresponding cell as indicated by an SC-DCI format; - TCI state: follow the indicated DL/ joint TCI state (per Rel-17 unified TCI framework) for the corresponding cell as indicated by an SC-DCI format (when indication in MC-DCI is not configured by RRC); - DMRS sequence initialization: per RRC configuration; - VRB-to-PRB bundling field, or PRB bundling size: per RRC configuration (when indication in MC-DCI is not configured by RRC); Ericsson: - Proposal 11: For mc-DCI, at least the following fields should be applicable for each subgroup of cells scheduled by mc-DCI - TDRA - VRB-to-PRB mapping - PRB bundling size indicator - Rate matching indicator - ZP CSI-RS trigger - Antenna port(s) - Transmission configuration indication - DM-RS sequence initialization - BWP - Proposal 12: For each cell, support separate configuration of RBG size(s) used for PUSCH/PDSCH scheduling using mc-DCI. - Proposal 13: For frequency domain resource allocation (FDRA) using mc-DCI, support joint coding of individual RIVs of each cell to reduce overhead for FDRA type 1. - Proposal 14: For mc-DCI, RV field size is explicitly configurable per cell (0/1/2 bits). - Proposal 15: For mc-DCI, support joint coding of MCS indication for co-scheduled cells in a subgroup - MCS index for a first cell is five bits (Index 1), and for each of remaining cells in the subgroup, differential MCS index is indicated relative to the Index 1. - For each of the remaining cells, up to 3 bits for differential MCS index, and the differential MCS offsets are configured by higher layers. NTT DOCOMO: - Proposal 3: The following DCI fields are not supported in the DCI format 0_X/1_X; - UL/SUL indicator in DCI format 0_X - CBGTI and CBGFI in DCI format 0_X/1_X - Multi-TRP related fields - Second TPC command for scheduled PUSCH in DCI format 0_X - Second SRS resource indicator in DCI format 0_X - Second Precoding information in DCI format 0_X - Second PTRS-DMRS association in DCI format 0_X - Second TPC command for scheduled PUCCH in DCI format 1_X - Proposal 4: The following DCI fields in DCI format 0_X/1_X are Type-1A field; - Scell dormancy indication - Priority indicator - PDCCH monitoring adaptation indication - Indicator of co-scheduled cells (CIF) - Proposal 5: HARQ process number in DCI format 0_X/1_X is Type-2 field. - Proposal 6: The DCI fields in DCI format 0_X/1_X are Type-3 unless it is agreed as Type-1 or 2 field. Qualcomm: - Proposal 6: - TDRA, BWP indicator, VRB-to-PRB, PRB bundling size, RM-indicator, ZP-CSI-RS indicator, TCI-state, DMRS sequence initialization, FH flag, beta-offset indicator, SRS request, priority indicator: joint indication: Type-1B - Similar to TDRA for Rel-16/Rel-17 multi-slot PDSCH/PUSCH scheduling - MCS: Type-3 - Configured ‘mcs-Table’ is the same for all the cells in the group - ‘cqi-Table’ for the cells in the group is expected to be the same for appropriate link adaptation - Consider up to 2 groups per DCI - Antenna port(s), Precoder information and number of layer(s), SRS resource indicator, TPC for PUSCH: Type-3 - RRC parameters are configured so that the same look-up table is referred for the cells - CSI reporting type/config for these cells are expected to be the same for appropriate link adaptation - Allow 1 or 2 groups per DCI - FDRA: Type-1B or Type-2 - Opt.1: Type-1B, where: - Continuous RB indexing over the RBs of the multiple co-scheduled cells is introduced - Type-0 and Type-1 for the continuous RB indexing over multiple cells is supported - Opt.2: Type-2, where: - For Type-0, larger RBG size is supported - For Type-1, RBG-based RIV is used - HARQ process indicator: Type-1A or Type-2 - HPID field is a common field for all the co-scheduled cells - Consider to introduce a “HPID offset indication” for each cell - Scell dormancy indicator, PDCCH monitoring adaptation, CSI request, UL-SCH indicator, PDCCH monitoring adaptation: Type-1A - ChannelAcces-Cpext, minimum K0/K2 offset: Type-1A - UL/SUL indicator: Not included Nokia: - Proposal 4.2: RAN1 to discuss if the RRC parameters for DCI format 0_1/1_1 scheduling or the Rel-16 RRC parameters for DCI format 0_2/1_2 are reused for DCI formats 0_X/1_X operation, or if alternatively new separate configurations for DCI formats 0_X/1_X are introduced. - Proposal 4.3: Adopt the following DCI field types for DCI format 1_X assuming also monitoring for single cell DCI is supported: DCI FIELDS OF FORMAT 1_X; FIELD TYPE; COMMENTS. Identifier for DCI formats; Type 1A; Agreed at RAN1#110. Co-scheduled cell indicator; Type 1B; The assumption is this field would Points to Code point in a RRC configured table (see proposal 4.1). Bandwidth part indicator; Type 1A; This field could also be omitted if it is assumed that the multi cell scheduling is always scheduling the active BWP of the co-scheduled cells.. Frequency domain resource assignment; Type 3; Type 1 resource allocation with larger granularity (as supported for DCI format 1_2) can be considered.. Time domain resource assignment; Type 3. VRB-to-PRB mapping; Type 3. PRB bundling size indicator; Type 3. Rate matching indicator; Type 3. ZP CSI-RS trigger; Type 2; Separate field since a UE is not expected to receive more than one DCI with non-zero CSI request field per slot per cell. A UE is not expected to receive DCI with non-zero CSI request field within a cell group in a slot overlapping with any slot receiving DCI with non-zero CSI request field in the same cell group.. TB1: Modulation and coding scheme; Type 3; Common could be useful e.g. for intra-band operation, whereas for inter-band operation clearly separate DCI field would be needed.. TB1: New data indicator; Type 2; Agreed at RAN1#110. TB1: Redundancy version; Type 2; Agreed at RAN1#110. TB2: Modulation and coding scheme; Type 2. TB2: New data indicator; Type 2; Agreed at RAN1#110. TB2: Redundancy version; Type 2; Agreed at RAN1#110. HARQ process number; Type 2. Downlink assignment index; Type 1A; Agreed at RAN1#110. TPC command for scheduled PUCCH; Type 1A; Agreed at RAN1#110. PUCCH resource indicator; Type 1A; Agreed at RAN1#110. PDSCH-to-HARQ timing indicator; Type 1A; Agreed at RAN1#110. One shot HARQ ACK request; Type 1A; Agreed at RAN1#110. Enhanced Type 3 codebook indicator; Type 1A; Only a single k1 value can be indicated à only a single enh. Type 3 HARQ-ACK CB can be triggered. HARQ-ACK retransmission indicator; Type 1A; Only a single k1 value can be indicated à only a HARQ-ACK CB can be triggered for re-transmission. Antenna port(s); Type 3. Transmission configuration indication; Type 3. SRS request; Type 3. DMRS sequence initialization; Type 2. Priority indicator; Type 1A; Same as k1, PRI, TPC for PUCCH, …. PDCCH monitoring adaptation indication; Type 1C; Refers to the scheduling cell only. PUCCH Cell indicator; Type 1A; Same as k1, PRI, TPC for PUCCH, …. - Proposal 4.4: Adopt the following DCI field types for DCI format 0_X assuming also monitoring for single cell DCI is supported: DCI FIELDS OF FORMAT 0_X; FIELD TYPE; COMMENTS. Identifier for DCI formats; Type 1A; Agreed at RAN1#110. Co-scheduled cell indicator; Type 1B; The assumption is this field would Points to Code point in a RRC configured table (see proposal 4.1). Frequency domain resource assignment; Type 3; Type 1 resource allocation with larger granularity (as supported for DCI format 0_2) can be considered.. Time domain resource assignment; Type 3. Frequency hopping flag; Type 3. Modulation and coding scheme; Type 3; Common could be useful e.g. for intra-band operation, whereas for inter-band operation clearly separate DCI field would be needed.. New data indicator; Type 2; Agreed at RAN1#110. Redundancy version; Type 2; Agreed at RAN1#110. HARQ process number; Type 2. Downlink assignment index; Type 1A; Agreed at RAN1#110 Note: Assumption here would be, that only a single HARQ-ACK codebook (and single PHY priority) is to be multiplexed on the set of scheduled PUSCHs.. TPC command for scheduled PUSCH; Type 1C; Interpretation is that this TPC command is applicable to the UL of the scheduling cell only. If the scheduling cell cannot be scheduled by the multi-cell DCI, the TPC field is not present.. SRS resource indicator; Type 1C; Interpretation is that the SRI is applicable to the UL of the scheduling cell only. Precoding information and number of layers; Type 3; Common (e.g. for intra-band UL CA) or cell specific depending on the scenario.. Antenna ports; Type 3; Common (e.g. for intra-band UL CA) or cell specific depending on the scenario.. SRS request; Type 1C; Interpretation is that the SRS request is applicable to PUSCH of the first scheduled cell only. SRS offset indicator; Type 1C; Interpretation is that the CSI request is applicable to PUSCH of the first scheduled cell. CSI request; Type 1C; Interpretation is that the CSI request is applicable to PUSCH of the first scheduled cell (i.e. the first in the table row) We don’t see a need to trigger more than one CSI request in a triggering DCI.. PTRS-DMRS association; Type 3. beta_offset indicator; Type 1A; Same as the DAI. DMRS sequence initialization; Type 1A. Open-loop power control parameter set indication; Type 1A; To be aligned with PHY priority indicator definition (same PHY priority, same OL TPC parameter sets).. Priority indicator; Type 1A; Same as DAI (only single DAI meaning single priority only). Invalid symbol pattern indicator; Type 1A. PDCCH monitoring adaptation indication; Type 1C; Applies to the scheduling cell only..

### Moderator summary and proposals based on contributions

Regarding DCI field type classification, RAN1#110 reached below agreement:

Agreement For discussing field design of DCI format 0_X/1_X which schedules more than one cell, reformulate the types of DCI fields as below: - Type-1 field: - Type-1A field: A single field indicating common information to all the co-scheduled cells - Type-1B field: A single field indicating separate information to each of co-scheduled cells via joint indication - Type-1C field: A single field indicating an information to only one of co-scheduled cells - Type-2 field: Separate field for each of the co-scheduled cells - Type-3 field: Common or separate to each of the co-scheduled cells, or separate to each sub-group, dependent on explicit configuration. - Note: One sub-group comprises a subset of co-scheduled cells where a single field is commonly applied to the co-scheduled cell(s) belonging to a same sub-group. - Note: Handling of any parameters applicable to multi-cell scheduling where corresponding fields are not included in DCI format 0_X/1_X (if any) will be separately discussed. Agreement - For DCI format 1_X/0_X which can schedule more than one cell, - Type-1 fields at least include below: - Type-1A: - Identifier for DCI formats - Downlink assignment index - TPC for scheduled PUCCH - PUCCH resource indicator - PDSCH-to-HARQ timing indicator - One-shot HARQ-ACK request - Type-2 fields at least include below: - New data indicator per TB - Redundancy version per TB - FFS: Other fields to be included in DCI format 1_X/0_X and which type of the fields belongs to. - FFS: size for each field

In RAN1#110bis meeting, regarding the DCI field types, companies’ views are summarized in below Table 1 and Table 2.

Table 1: DCI field types for DCI format 1_X (“√” means the field has been agreed in previous meeting)

| DCI FIELDS | Type 1A | Type 1B | Type 1C | Type 2 | Type 3 | Omitted |
| --- | --- | --- | --- | --- | --- | --- |
| Identifier for DCI formats | √ |  |  |  |  |  |
| Indicator of co-scheduled cells | CATT, Fujitsu, Intel, xiaomi, CMCC, FGI, LG, Samsung, DCM, | ZTE, SPRD, xiaomi, Lenovo, Samsung, Nokia |  |  |  |  |
| Bandwidth part indicator | Intel, Nokia | QC, | LG, | Lenovo, | HW, ZTE, Fujitsu, CMCC, Ericsson, | SPRD, xiaomi, FGI, LG, Samsung, |
| Frequency domain resource assignment | OPPO, CMCC, LG, | Lenovo, |  | xiaomi, FGI, | HW, ZTE, SPRD, vivo, CATT, Fujitsu, Intel, LG, Samsung, Nokia |  |
| Time domain resource assignment | OPPO, CATT, Intel, | SPRD, OPPO, Lenovo, FGI, LG, QC, |  |  | HW, ZTE, vivo, Fujitsu, CMCC, LG, Samsung, Ericsson, Nokia |  |
| VRB-to-PRB mapping | Intel, FGI, | QC, | LG, |  | HW, ZTE, SPRD, vivo, CATT, Fujitsu, Lenovo, CMCC, Ericsson, Nokia | Samsung, |
| PRB bundling size indicator |  | QC, | LG, |  | HW, ZTE, SPRD, vivo, CATT, Fujitsu, Lenovo, CMCC, FGI, Ericsson, Nokia | Samsung, |
| Rate matching indicator | Intel, | Samsung, QC, | LG, |  | HW, ZTE, SPRD, vivo, CATT, Fujitsu, Lenovo, CMCC, FGI, Ericsson, Nokia |  |
| ZP CSI-RS trigger | Intel, | Samsung, QC, | LG, | Nokia | HW, ZTE, SPRD, Fujitsu, Lenovo, CMCC, FGI, Ericsson, |  |
| TB1: Modulation and coding scheme | LG, | CATT, |  | HW, vivo, OPPO, Fujitsu, xiaomi, Lenovo, CMCC, Apple, Samsung, QC, | ZTE, SPRD, FGI, Intel, LG, Nokia |  |
| TB1: New data indicator | √ |  |  |  |  |  |
| TB1: Redundancy version | √ |  |  |  |  |  |
| TB2: Modulation and coding scheme |  |  |  | Samsung, | FGI, | SPRD, |
| TB2: New data indicator | √ |  |  |  |  | SPRD, |
| TB2: Redundancy version | √ |  |  |  |  | SPRD, |
| HARQ process number | ZTE, OPPO, CATT, Intel, LG, QC, |  |  | HW, Fujitsu, Samsung, DCM, QC, Nokia | SPRD, CMCC, FGI, LG, |  |
| Downlink assignment index | √ |  |  |  |  |  |
| TPC command for scheduled PUCCH | √ |  |  |  |  |  |
| Second TPC command for scheduled PUCCH |  |  |  |  |  | SPRD, DCM, |
| PUCCH resource indicator | √ |  |  |  |  |  |
| PDSCH-to-HARQ timing indicator | √ |  |  |  |  |  |
| One shot HARQ ACK request | √ |  |  |  |  |  |
| Enhanced Type 3 codebook indicator | HW, SPRD, FGI, Nokia |  |  |  |  | Samsung, |
| HARQ-ACK retransmission indicator | HW, Nokia |  |  |  |  | SPRD, FGI, Samsung, OPPO |
| Antenna port(s) |  |  |  | LG, | HW, ZTE, SPRD, CATT, Fujitsu, Lenovo, CMCC, FGI, LG, Samsung, Ericsson, QC, Nokia |  |
| Transmission configuration indication | Samsung, | LG, Samsung, QC, |  | LG, | HW, SPRD, CATT, Fujitsu, Lenovo, CMCC, FGI, Ericsson, Nokia | Samsung, |
| SRS request | Intel, | Samsung, | FGI, LG, |  | HW, ZTE, SPRD, vivo, Fujitsu, Lenovo, CMCC, Nokia |  |
| DMRS sequence initialization | Intel, CMCC, | QC, | LG, | Nokia | HW, ZTE, SPRD, vivo, Fujitsu, Lenovo, FGI, Ericsson, | LG, Samsung, |
| Priority indicator | HW, SPRD, vivo, Lenovo, FGI, LG, DCM, Nokia |  |  |  |  | SPRD, |
| PDCCH monitoring adaptation indication | HW, DCM, QC, |  | SPRD, FGI, Nokia |  |  | Lenovo, Samsung, |
| PUCCH Cell indicator | HW, FGI, Nokia |  |  |  |  | Lenovo, |
| Minimum applicable scheduling offset indicator | HW, |  | LG, |  |  | SPRD, Lenovo, FGI, LG, Samsung, |
| Scell dormancy indication | HW, DCM, QC, |  |  |  |  | SPRD, Lenovo, FGI, LG, Samsung, |
| PDSCH group index |  |  |  |  |  | HW, Lenovo, Samsung, |
| New feedback indicator |  |  |  |  |  | HW, Lenovo, Samsung, |
| Number of requested PDSCH group(s) |  |  |  |  |  | HW, Lenovo, Samsung, |
| CBG transmission information |  |  |  |  |  | HW, SPRD, xiaomi, Lenovo, FGI, Samsung, DCM, |
| CBG flushing out information |  |  |  |  |  | HW, SPRD, xiaomi, Lenovo, FGI, Samsung, DCM, |
| ChannelAccess-Cpext | Lenovo, LG, Samsung, QC, |  |  |  |  | FGI, |

Table 2: DCI field types for DCI format 0_X(“√” means the field has been agreed in previous meeting)

| DCI FIELDS OF FORMAT 0_X | Type 1A | Type 1B | Type 1C | Type 2 | Type 3 | Omitted |
| --- | --- | --- | --- | --- | --- | --- |
| Identifier for DCI formats | √ |  |  |  |  |  |
| Indicator of co-scheduled cells | CATT, xiaomi, Fujitsu, FGI, LG, Samsung, DCM, | ZTE, SPRD, xiaomi, Lenovo, Samsung, Nokia |  |  |  |  |
| DFI flag |  |  |  |  |  | SPRD, FGI, |
| BWP indicator |  |  | LG, | Lenovo, | HW, ZTE, Fujitsu, | SPRD, xiaomi, FGI, LG, Samsung, |
| Frequency domain resource assignment | OPPO, LG, | Lenovo, QC, |  | xiaomi, FGI, QC, | HW, ZTE, SPRD, vivo, CATT, Fujitsu, Intel, LG, Samsung, Nokia |  |
| Time domain resource assignment | OPPO, CATT, | SPRD, OPPO, FGI, LG, |  | Lenovo, | HW, ZTE, vivo, Fujitsu, LG, Samsung, Nokia |  |
| Frequency hopping flag | Samsung, | QC, | LG, |  | HW, SPRD, CATT, Fujitsu, Lenovo, FGI, Nokia | LG, |
| Modulation and coding scheme | LG, | CATT, |  | HW, vivo, OPPO, Fujitsu, xiaomi, Apple, Lenovo, Samsung, QC, | SPRD, Intel, FGI, LG, Nokia |  |
| New data indicator | √ |  |  |  |  |  |
| Redundancy version | √ |  |  |  |  |  |
| HARQ process number | ZTE, OPPO, CATT, LG, QC, |  |  | HW, Fujitsu, Lenovo, Samsung, DCM, QC, Nokia | SPRD, FGI, LG, |  |
| Downlink assignment index | √ |  |  |  |  |  |
| TPC command for scheduled PUSCH |  |  | LG, | SPRD, OPPO, Nokia | HW, ZTE, Fujitsu, FGI, Samsung, QC, |  |
| Second TPC command for scheduled PUSCH |  |  |  |  | HW, | DCM, |
| Precoding information and number of layers |  |  |  | LG, | HW, SPRD, CATT, Fujitsu, FGI, LG, Samsung, QC, Nokia |  |
| Second Precoding information |  |  |  |  | HW, | SPRD, Samsung, DCM, |
| Antenna ports |  |  |  | LG, | HW, SPRD, CATT, Fujitsu, FGI, LG, Samsung, QC, Nokia |  |
| SRS request |  | Samsung, QC, | LG, Nokia |  | HW, ZTE, SPRD, vivo, CATT, Fujitsu, Lenovo, FGI, |  |
| SRS resource indicator |  | LG, | Nokia | LG, | HW, ZTE, SPRD, Fujitsu, FGI, Samsung, QC, |  |
| SRS offset indicator |  |  | Nokia |  | HW, SPRD, | FGI, |
| SRS resource set indicator |  |  |  |  |  | SPRD, Samsung, |
| Second SRS resource indicator |  |  |  |  |  | SPRD, Samsung, DCM, |
| CSI request | Intel, Samsung, QC, |  | ZTE, vivo, Fujitsu, FGI, LG, Samsung, Nokia |  | HW, SPRD, |  |
| PTRS-DMRS association |  |  | LG, | LG, | HW, SPRD, CATT, Fujitsu, FGI, Nokia | Samsung, |
| Second PTRS-DMRS association |  |  |  |  |  | SPRD, Samsung, DCM, |
| beta_offset indicator | HW, Intel, Samsung, Nokia | QC, | vivo, Fujitsu, FGI, LG, Samsung, |  | SPRD, FGI, |  |
| DMRS sequence initialization | Nokia |  | LG, |  | HW, SPRD, vivo, Fujitsu, FGI, | LG, Samsung, |
| Open-loop power control parameter set indication | Nokia |  | LG, |  | HW, FGI, | SPRD, |
| Priority indicator | HW, SPRD, Lenovo, FGI, LG, DCM, Nokia | QC, |  |  |  | SPRD, |
| Invalid symbol pattern indicator | HW, FGI, Samsung, Nokia |  | LG, |  |  | SPRD, |
| PDCCH monitoring adaptation indication | HW, DCM, QC, |  | SPRD, FGI, Nokia |  |  | Lenovo, Samsung, |
| UL-SCH indicator | HW, Samsung, QC, |  | Samsung, |  |  | SPRD, |
| Minimum applicable scheduling offset indicator | HW, |  | LG, |  |  | SPRD, Lenovo, FGI, LG, Samsung, |
| Scell dormancy indication | HW, DCM, |  |  |  |  | SPRD, Lenovo, FGI, LG, Samsung, |
| UL/SUL indicator |  |  | Fujitsu, FGI, |  | HW, | FGI, LG, Samsung, DCM, QC, |
| CBGTI |  |  |  |  |  | SPRD, Lenovo, FGI, DCM, |
| ChannelAccess-Cpext | Lenovo, LG, Samsung, QC, |  |  |  | LG, | FGI, |
| Sidelink assignment index |  |  |  |  |  | HW, Samsung, |

Firstly, based on guidance from RAN#97 and agreement reached in RAN1#110 meeting as shown in below, enhanced Type-2 HARQ-ACK codebook, CBG-based transmission, multi-TRP and sidelink relevant fields should be excluded from DCI format 0_X/1_X.

Agreement UE does not expect to be configured both CBG-based PDSCH/PUSCH transmission and the multi-cell PDSCH/PUSCH scheduling on the same or different cells within a same PUCCH group. Conclusion: - Deprioritize any optimization for unlicensed spectrum operation for designing the multi-cell PUSCH/PDSCH scheduling in Rel-18. - Enhanced Type-2 HARQ-ACK codebook is not supported for the multi-cell PUSCH/PDSCH scheduling in Rel-18. Conclusion: - Followings are excluded from multi-cell PDSCH/PUSCH scheduling in Rel-18. - Scell schedules multiple cells including P(S)Cell - Different SCS among co-scheduled cells - Different carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2) among co-scheduled cells - Configuration of both multi-cell PDSCH/PUSCH scheduling and multi-TRP for a scheduled cell - Support for any sidelink scheduling

Hence, Proposal 3-1 is provided to exclude such fields.

Secondly, considering DCI format 0_X/1_X is aiming at multi-cell scheduling for improving data rate and simultaneous monitoring legacy DCI formats and DCI format 0_X/1_X is supported, the DCI fields for other purposes can be realized via legacy DCI formats and should not be considered as much as possible to simplify DCI format 0_X/1_X design and reduce DCI format 0_X/1_X payload size. In that sense, below fields can be excluded from DCI format 0_X/1_X.

- DFI flag

- PDCCH monitoring adaptation indication

- HARQ-ACK retransmission indicator

- PUCCH Cell indicator

- Minimum applicable scheduling offset indicator

- Scell dormancy indication

- UL/SUL indicator

Hence, Proposal 3-2 is provided to exclude such fields.

Regarding Indicator of co-scheduled cells, it is to be discussed in next sub-section.

Thirdly, based on above summary, majority companies consider below fields are Type-1 fields:

- Indicator of co-scheduled cells

- Priority indicator

- beta offset indicator

- CSI request

- ChannelAccess-Cpext

- UL-SCH indicator

- Invalid symbol pattern indicator

Majority companies consider below fields are Type-2 fields:

- MCS

- HARQ process number

Majority companies consider below fields are Type-3 fields:

- Bandwidth part indicator

- Frequency domain resource assignment

- Time domain resource assignment

- VRB-to-PRB mapping

- PRB bundling size indicator

- Rate matching indicator

- ZP CSI-RS trigger

- Antenna port(s)

- Transmission configuration indication

- SRS request

- DMRS sequence initialization

- Frequency hopping flag

- TPC command for scheduled PUSCH

- Precoding information and number of layers

- SRS resource indicator

- SRS offset indicator

- PTRS-DMRS association

- Open-loop power control parameter set indication

Since different companies have different preference, moderator suggests reaching consensus on Type-1 and Type-2 fields firstly and leave remaining fields as Type-3. So, Proposal 3-3 is provided for 1st round of discussions.

Regarding the field of time domain resource assignment, 9 companies [Spreadtrum, OPPO, CATT, Intel, Lenovo, LG, FGI, Samsung, Qualcomm] propose using legacy Rel-17 multi-slot scheduling method, i.e., time domain resource allocations for co-scheduled cells are joint indicated by TDRA field in DCI format 0_X/1_X by pointing to one row of a RRC configured TDRA table, similar to regard TDRA as Type-1B field. Nine companies [Huawei, ZTE, vivo, Fujitsu, CMCC, LG, Samsung, Ericsson, Nokia] propose Type-3 field for TDRA without details shown when TDRA is configured as a common field to co-scheduled cells.

Since TDRA field is relevant to Type-1 HARQ-ACK codebook design, moderator suggests following Rel-17 multi-PDSCH scheduling to design TDRA for Rel-18 multi-cell scheduling. Proposal 3-4 is provided for 1st round of discussion.

### 1st round of discussions

#### Proposal 3-1:

- At least below fields are excluded from DCI format 1_X/0_X:

- CBGTI

- CBGFI

- PDSCH group index

- New feedback indicator

- Number of requested PDSCH group(s)

- Sidelink assignment index

- Second TPC command for scheduled PUSCH

- Second SRS resource indicator

- Second Precoding information

- Second PTRS-DMRS association

- Second TPC command for scheduled PUCCH

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| MTK | Support |
| Qualcomm | We are OK with the proposal. |
| Nokia, NSB | Support |
| Apple | Support |
| Langbo | Support |
| Huawei, HiSilicon | Support |
| Intel | We support the proposal. |
| ZTE | We support this proposal since the corresponding features cannot be configured together with multi-cell scheduling. |
| CMCC | Support |
| New H3C | Support |
| Fujitsu | Support |
| CATT | Support |
| vivo | We support the proposal. |
| LGE | Support |
| Samsung | OK with the proposal |
| Spreadtrum | Support |
| Xiaomi | Support |
| NTT DOCOMO | Support |
| ITRI | Support |
| OPPO | Support |
| Moderator | Agreed. So need further inputs now. |

#### Proposal 3-2:

- At least below fields are excluded from DCI format 1_X/0_X:

- DFI flag

- PDCCH monitoring adaptation indication

- HARQ-ACK retransmission indicator

- PUCCH Cell indicator

- Minimum applicable scheduling offset indicator

- Scell dormancy indication

- UL/SUL indicator

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| MTK | Support |
| Qualcomm | We prefer to discuss these fields a bit more carefully. First of all, we have to conclude whether the UE monitors legacy DCI formats for all the CCs in the scheduled-CC-set by a DCI format 0_X/1_X. In addition, some of the fields do not require specific work to support (e.g., PUCCH cell indicator, PDCCH monitoring adaptation, etc). Not sure why these have to be excluded. |
| Nokia, NSB | Partially Support. - We think the dynamic PUCCH cell switching should still be supported (so the PUCCH cell indicator should be there / supported) – as also pointed out by QC. - If we don’t support HARQ-ACK re-transmission triggering with the DCI (where the MCS field is used for some indication in addition), would we then not support Type 3 HARQ-ACK codebook triggering as well (or at least not the enh. Type 3 HARQ-ACK codebook triggering – as the same issues with the MCS field usage will be there as for the HARQ-ACK re-transmission triggering). |
| Apple | Fine to support the proposal |
| Langbo | Support |
| Huawei, HiSilicon | We understand Moderator consideration about legacy DCI use. However, if we go with such direction, it means the design of multi-carrier DCI encourages the use of legacy DCI simultaneously. From our point of view, have such function is more about insurance that when something cannot be realized by the multi-carrier DCI, we still have legacy DCI can be scheduled, so the network performance will not be restricted. In this sense, we should actually try to include as much as possible of the current DCI fields unless that: 1) it requires additional spec work so has to be used by legacy DCI; 2) it significantly increase the DCI size that could penalize the multi-carrier DCI performance. Based on the above, we think the current proposal require a bit more considerations. Actually, as we commented in the last meeting, if we just agree on Proposal 3-1 and leave others to network or as one of Type 1 or Type 3, the discussion may be simpler. We can make working assumption as such and see whether the inclusion of other fields will cause significant additional specification work, if so, we could try to eliminate those at that time when needed. |
| Intel | We are fine with the proposal. We do not think optimization is needed given the limited TU. |
| ZTE | We support this proposal. |
| Moderator | @Qualcomm: The intention is to simplify the multi-cell DCI design including only the necessary fields. @Nokia: OK to defer the discussion on PUCCH cell switching. Since we have agreed One-shot HARQ-ACK request indicator in DCI format 1_X, Type 3 CB can be supported as well. For eType-3 CB, if supported, explicit indicator needs to be included in DCI 1_X. HARQ-ACK retransmission indicator if supported, MCS is reused, which impacts on data scheduling. So I propose not include it. @Huawei: Yes, we can leave some non-Type1/2 fields as Type 3 for gNB implementation. My thinking is if we can exclude some fields, then it is easier for gNB not to consider the fields during the configuration. |
| CMCC | We think that UL/SUL indicator should not be precluded, since SUL can be supported for multi-cell PDSCH/PUSCH scheduling. |
| New H3C | Support |
| Fujitsu | Support |
| CATT | Support |
| vivo | We support the proposal. |
| LGE | Support only for the case when the DCI 1_X/0_X schedules multiple cells. Otherwise, in case when the DCI 1_X/0_X schedules only one cell, there is no reason to exclude the above fields from the DCI 1_X/0_X. |
| Samsung | OK with the proposal. It would be preferable to focus the discussion on the purposes of the WID instead of trying to include non-core functionalities, at least at this time. For example, the PUCCH cell switching mentioned by other companies was introduced for latency reduction in URLLC and has very limited applicability (and, even then, there is no PUSCH latency reduction) – that is not needed for eMBB. We’re generally OK with introducing non-core functionalities to MC-DCI but would be good to focus on the main design and not be distracted by them at this stage |
| Spreadtrum | Support |
| NTT DOCOMO | We share the same view with HW that the field(s) can be included in the DCI format 0_X/1_X as long as it does not require optimization to support and significantly impact to DCI size. Thus, if there is no strong concern to support, we are fine to include these field depending on the gNB implementation. |
| Ericsson1 | At least Scell dormancy, PDCCH monitoring adaptation indication should be supported in DCI 1_X/0_X. |
| Qualcomm | We tend to agree with Huawei. We still fail to understand why excluding many of the above fields can simplify the design. It encourages use of legacy DCI formats more, which increases the UE complexity. In principle, we think if monitoring legacy DCI formats for all of the scheduled CCs as in legacy, there is no benefit/incentive for a UE to support multi-cell scheduling. It just increases the complexity much. A solution should offer benefit for both NW and UE. |
| Moderator2 | #### @All: The proposal is updated as below: #### Proposal 3-2rev1: - At least below fields are excluded from DCI format 1_X/0_X: - DFI flag - HARQ-ACK retransmission indicator - Minimum applicable scheduling offset indicator |
| Nokia / NSB | We still don’t get, why the DCI format 1_X is able to triggering the Type-3 HARQ-ACK CB (support the triggering based on RAN1#110) but don’t support the HARQ-ACK re-transmission triggering. As both are there basically for the same purpose (i.e. retrieve lost HARQ information) it is unclear to us why to treat them differently here (as both are 1 bit triggers in the first place). As |
| Qualcomm | Agree with Nokia. |
| LGE | One clarification to FL’s formulation. Are the above fields excluded from DCI 1_X/0_X even in case when the DCI schedules only one cell? If yes, why those are to be excluded even though the inclusion of those fields doesn’t increase DCI payload size? As done for the multi-TTI scheduling in Rel-17, it is straightforward that all the fields are included as in legacy if the DCI schedules only one PDSCH/PUSCH. |
| Moderator3 | @Nokia @Qualcomm: When HARQ-ACK re-transmission is triggered, the MCS field is used for retransmission slot indication. Since DCI 0-X/1-X is mainly used for data scheduling, it is better to no t reuse MCS or FDRA field for other purpose. @LG: Thanks for the good comments. Now I make update on the proposal: #### Proposal 3-2rev2: - At least below fields are excluded from DCI format 1_X/0_X which schedules more than one cell: - DFI flag - HARQ-ACK retransmission indicator - Minimum applicable scheduling offset indicator |
| Intel | We can live with the proposal. |
| Nokia/NSB | Thanks to moderator for the reply on the MCS field usage for the HARQ re-tx. Please note that for enh. Type 3 CB there is the MCS field re-use issues as well. So do we then only support R16 Type 3 triggering (but no R17 enh. Type 3) triggering then with 1_X (based on the argumentation, FDRA or MCS field should not be used otherwise)? |
| Moderator4 | @Nokia: I think this proposal doesn’t exclude eType-3 if Enhanced Type 3 codebook indicator which having 0, 1, 2, or 3 bit is included in DCI 1_X. |
| OPPO | We are a bit confused by the updated proposal 3-2rev2, which says the existence of certain fields depends on the actual number of scheduled cells. But in order to know the actual number of scheduled cells, UE needs to know the DCI size which in turn depends on whether the corresponding fields do exist. It seems the determination logic runs in circle. |
| NTT DOCOMO2 | We can support the updated proposal. |
| MTK | The argument from Huawei seems valid to us, and the suggestion from DOCOMO to leave the presence of these additional function field to gNB implementation seems like a good idea. We are fine with Proposal 3-2rev2. If there are other controversial fields, we can also just remove them and leave them to gNB implementation. |
| Qualcomm | Agree with OPPO. |
| Moderator5 | @OPPO @QC: Before checking the indicator of co-scheduled cells, UE has to determine the payload size of DCI 0-X/1-X firstly based on the maximum size for the set of cells as discussed in DCI size determination part. Upon detection of the DCI, then UE check indicator of co-scheduled cells to get the info of number of scheduled cells. This is similar to what we have done in Rel-16/17 multi-PUSCH scheduling: CBGTI is present if the DCI schedules a single PUSCH and not present if DCI schedules more than one PUSCH. UE can know the number of scheduled PUSCHs after checking TDRA field. |
| Moderator6 | Please continue the discussion in 2nd round. |

#### Proposal 3-3:

- For DCI format 1_X/0_X,

- Type-1 fields at least include below:

  - Type-1A:

    - Priority indicator

    - Invalid symbol pattern indicator

    - ChannelAccess-Cpext

    - UL-SCH indicator

  - Type-1B fields at least include below:

    - Indicator of co-scheduled cells

  - Type-1C fields at least include below:

    - beta offset indicator

    - CSI request

- Type-2 fields at least include below:

  - MCS

  - HARQ process number

- FFS: Other fields

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| MTK | Support |
| Qualcomm | We need to discuss and clarify how RRC parameters are provided for DCI 0_X/1_X based scheduling. Take “Priority indicator” as an example of Type-1A. Is it proposed here to introduce new RRC parameters “priorityIndicatorDCI-1-X” and “priorityIndicatorDCI-0-X” such that the priority of PDSCHs or PUSCHs scheduled by a DCI format 0_X or 1_X follow the indication? Or, the priority indicator field in DCI 0_X/1_X follows the existing RRC parameters (e.g., priorityIndicatorDCI-1-0 and priorityIndicatorDCI1-1) configured to each CC in the scheduled-CC-set? Same clarification is necessary for Invalid symbol pattern indicator, beta-offset indicator, CSI request and MCS. |
| Nokia, NSB | Fine with the intention – but a question for clarification on UL-SCH indicator: we are fine to only have a single field present (i.e. Type 1), but with Type 1A it is a bit unclear to us why the gNB would scheduled multiple PUSCHs and then indicate for all of them UL-SCH to be not present? And as QC pointed out, we will still need to clarify e.g. for the priority indicator which RRC configuration is used / applied here. |
| Apple | In principle, fine with the proposal, but agree with QC that clarification on RRC configuration for DCI format 0_X/1_X should be discussed. This could be discussed separately |
| Langbo | We are generally fine with the proposal, but as pointed out by other companies that further clarifications are needed. |
| Intel | We have similar concern on the RRC configuration and whether UL-SCH indicator is for Type 1A. For MCS, we prefer to consider this for Type 3. Different sub-group may have different channel conditions, so it would be more desirable to signal MCS for the cells in the same sub-group. For HPN, we think same HPN would be sufficient for multi-cell scheduling to save payload size, but we are fine with this if majority supports. |
| ZTE | We also think CSI request is classified as Type-1Csince only A-CSI can be transmitted only on one cell at a time according to the current spec. The UL-SCH indicator should also be Type-1C since it is used for CSI request. ChannelAccess-Cpext should not be included since unlicensed band should be de-prioritized. The HPN field should be shared by the co-scheduled cells. Each cell has a HARQ entity. All the co-scheduled cells can share the same HPN. The MCS should be classified as Type-3. The network can configure whether it shared or separate. In some cases, the shared MCS can be applied. For example, in the factory industry, the interferences are the same between the cells. For invalid symbol pattern indicator, we think it should be Type-3 since invalid symbol pattern is configured per cell. The network can configure whether it is shared or separate according to the invalid symbol pattern configuration for each cell. |
| Moderator | @Qualcomm: The intention is to just include a single bit priority indicator in MC DCI. As for detailed RRC configuration, I don’t have preference now but think new parameters are OK. @Nokia: Ok to list UL-SCH as Type-1C. @Intel @ZTE: Ok to list UL-SCH as Type-1C. HARQ is managed per cell. It is too restrictive if only a same HPN is used for co-scheduled cells. Same MCS may lead performance degradation. It is better to set MCS as Type 2 as what 2 TB case does. |
| CMCC | We are fine with the Type-2 fields listed above, while Type-1 fields may need more clarification, which can be further discussed. |
| Fujitsu | For RRC configuration, we are open to reusing the legacy RRC parameter or introducing new RRC parameter. It can be separately discussed. For UL-SCH indicator, we think it should be Type-1C. And it should apply to the same cell as CSI request, because it is used to indicate whether the PUSCH with CSI would carry UL-SCH data as well or not. |
| CATT | We are ok with the proposal 3-3, expect for the field of MCS. Consider the channel conditions of scheduled cells for intra-band CA are practically same, the ‘differential/offset indication’ can be used for MCS indication. The MCS of one cell is used as a reference MCS, and the MCS of other cells are indicated by the differential/offset to reference MCS. The bitwidth of the field of MCS indication can be reduced compared to regarding as type-2 fields. |
| Vivo | We have similar concern on whether invalid symbol pattern indicator is for Type 1A. why all PUSCHs should consider the invalid symbol pattern if invalid symbol pattern is configured per cell？ It has been agreed that any optimization for unlicensed spectrum operation should be deprioritized in RAN#97. If ChannelAccess-Cpext is for type1A, does it mean that the default field type without optimization in mc-DCI is type-1A? if yes, it seems that all the other unlicensed spectrum operation related field should also be type-1A. We are ok with the following part of the proposal: - Type-1B fields at least include below: - Indicator of co-scheduled cells - Type-1C fields at least include below: - beta offset indicator - CSI request - Type-2 fields at least include below: - MCS - HARQ process number |
| LGE | Not support We suggest following classifications, considering DCI overhead reduction. Invalid symbol pattern indicator: Type-1C UL-SCH indicator: Type-1C MCS: Type-3 (or 1A) HARQ ID: Type-1A (or 3) |
| Samsung | Mostly OK with the proposal and would like to suggest the following: - UL-SCH should be Type-1C – it does not make sense to schedule PUSCHs on multiple cells without UL-SCH (similar to CSI request). - Would like clarification on the FL intention to list “Indicator of co-scheduled cells” as Type-1B (e.g., why not Type-1A?). We agree that only one value is needed for this field (and support to use an RRC configured table to do so, as in FL P3-5). - For additional progress, suggest to consider Frequency Hopping (FH) for Type-1A, and additional fields for Type-1B or Type-1C such as: Rate Matching pattern indication, ZP CSI-RS trigger, SRS request. |
| Spreadtrum | We have comments for those fields: - Invalid symbol pattern indicator: it is used for PUSCH repetition type B and only applied when the cell is configured with invalidSymbolPatternIndicatorDCI0_1/2, so it is not common for all cells. For simplify, it can be exclude from DCI 0_X and apply the invalid symbol pattern - ChannelAccess-Cpext can be excluded. - UL-SCH indicator: we think it can be excluded, because it is unnecessary to have a PUSCH without UL-SCH but only for A-CSI feedback through DCI 0_X. The legacy DCI format can be used. - beta offset indicator: as we commented before, the beta offset indicator do not only for the PUSCH with HARQ-ACK, but also for the PUSCH with any type of UCI, such CSI. From this side, all the cells configured with dynamic beta offset should indicated which value to use, otherwise, it would be problem when CSI is multiplexed on the PUSCH. |
| NTT DOCOMO | We are fine with the proposal in general. We think MCS which has a large payload should be type-3 but fine to support it as type-2 if majority of companies supports. For the clarification on RRC configuration for DCI format 0_X/1_X, we are fine to discuss it separately. |
| Ericsson1 | Not OK with Type 2 field for MCS. MCS is five bits per TB, which for 2 TBs x 4 carriers implies 40 bits. We propose to use joint coding of MCS fields across scheduled cells in a subgroup with five bits for one cell, and up to 3 bits indicating differential MCS for each of the other cells in the subgroup. |
| Moderator2 | @CATT: Definition of Type 2 field is a field separate for each schedule cell. Even if MCS is designed with differential method, still separate fields are applicable for co-scheduled cells. In that sense, it is Type-2 field. @vivo: Channel access field is Type-1A means same LBT type/CAPC/CPE for scheduled cells. There is only one channel access field, not sure about your comments “all the other unlicensed ..”. @LG: It is too restrictive for same HPN for all co-scheduled cells. Same MCS for co-scheduled cells may lead to performance degradation. @Samsung: Indicator of co-scheduled cells indicates whether a cell is scheduled is or not, which is separate information for each co-scheduled cell. In that sense, seems the field is type 1B. @Spreadtrum: Unlicensed spectrum is not excluded in RAN#97. So only including the channel access field should be OK. But no optimization for it. @Ericsson: Same reply as to CATT. @All: the proposal is updated as below: #### Proposal 3-3rev1: - For DCI format 1_X/0_X, - Type-1 fields at least include below: - Type-1A: - Priority indicator - ChannelAccess-Cpext - Type-1B fields at least include below: - Indicator of co-scheduled cells - Type-1C fields at least include below: - beta offset indicator - CSI request - UL-SCH indicator - Invalid symbol pattern indicator - Type-2 fields at least include below: - MCS - HARQ process number FFS: Other fields |
| Qualcomm | We agree with Intel/ZTE that MCS should belong to Type-3. Having up to 4 MCS fields in a DCI format 0_X/1_X is too much. |
| LGE | We are fine with Type-1 part in the above update. But not OK with Type-2 part with same reason as Intel/ZTE/QC since always separating such large (4 or 5 bits) size field in DCI is too much overhead compared to R17 multi-TTI scheduling DCI. Therefore, it is reasonable to include MCS and HARQ ID in Type-3 field. |
| Moderator3 | @LG @QC: @Intel: Even we set MCS as Type-3, it also covers the case of separate MCS field for each co-scheduled cell. There is still overhead issue. Unless MCS is common to all the co-scheduled cells, the overhead is an issue. |
| Intel | We share similar view as LGE, and we still have concerns on the MCS and HPN. As commented by other companies, MCS has 5 bits, which would increase a large DCI payload if it is individually signalled for each scheduled cells. We still prefer MCS to be included as Type 3. For HPN, we think same HPN would work without much issue. If companies have concern on the flexibility, we are fine to consider this as Type 3. |
| LGE2 | @FL: If MCS and HPN is always separate field in DCI, the number of co-scheduled cells and/or configuration of 2-TB PDSCH is also limited always. On the other hand, if MCS and HPN can be common field per cell subgroup due to their similarity, such limitation would be relaxed, then more efficiency/gain of multi-cell scheduling could be achieved. |
| NTT DOCOMO2 | If the field is Type-3, the size of the field can be reduced flexibly depending on the CA scenario/deployment policy, and hence we still think MCS should be Type-3. |
| Fujitsu | We are OK with Proposal 3-3rev1 |
| Moderator6 | Please continue the discussion in 2nd round where a lot of fields are listed for Type 3. |

#### Proposal 3-4:

- For a set of cells co-scheduled by a DCI format 0_X/1_X, time domain resource allocations for the set of cells are jointly indicated by TDRA field in the DCI format 0_X/1_X by pointing to one row of a RRC configured TDRA table.

- Separate {SLIV, mapping type, scheduling offset K0 (or K2)} are configured in the row for each of co-scheduled PDSCHs/PUSCHs.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| MTK | Support the main bullet. For the sub-bullet, not sure why we need separate indication. |
| Qualcomm | We are OK with the proposal. However, same clarification is necessary as for Proposal 3-3 – whether the RRC configured TDRA table for PDSCH/PUSCH of each CC in the scheduled-CC-set is based on the existing parameters (e.g., pusch-TimeDomainAllocationList and pdsch-TimeDomainAllocationList) or to define new parameters for this purpose. We consider we can re-use the existing parameters, since DCI 0_X/1_X must be a combined DCI of DCI 0_1/1_1. We are open to further discuss taking into account the balance between UE complexity and scheduling flexibility for each field. |
| Apple | Support the proposal |
| Langbo | Fine with the proposal. |
| Intel | We are fine with the proposal. |
| ZTE | We are fine with this proposal in principle. This can reduce the overhead for TDRA field. We think how to configure the RRC table should be clarified. For example, the RRC table is configured in which cell or configured for each cell? |
| Moderator | @Qualcomm @ZTE: Regarding the RRC configuration, existing parameter is configured per cell, I think it is easier to configure TDRA table for the set of co-scheduled cells. |
| Fujitsu | For clarification, is this proposal is only for multi-cell scheduling by DCI format 0_X/1_X (i.e. not for single-cell scheduling by DCI format 0_X/1_X) ? If yes, we are fine with the proposal. And for single-cell scheduling by DCI format 0_X/1_X, another TDRA table (e.g. configured by legacy RRC parameter for TDRA table configuration) can be used. |
| CATT | Support the proposal |
| LGE | Our view is to classify TDRA as Type-3 field, thus we suggest following update. For a subgroup set of cells co-scheduled by a DCI format 0_X/1_X, time domain resource allocations for the cells within the subgroup set of cells are jointly indicated by TDRA field in the DCI format 0_X/1_X by pointing to one row of a RRC configured TDRA table. |
| Samsung | OK with the proposal. Suggest to clarify “indicated by a single TDRA field”. |
| NTT DOCOMO | We support the joint indication of TDRA itself but would like to clarify whether this TDRA field indicate the actually co-scheduled cells by DCI format 0_X/1_X or other field, e.g., CIF, indicate the actually co-scheduled cells. |
| Ericsson1 | Not OK. We would be OK if the TDRA table are configured per scheduled cell like in legacy RRC configuration. - For a set of cells co-scheduled by a DCI format 0_X/1_X, time domain resource allocations for the set of cells are jointly indicated by TDRA field in the DCI format 0_X/1_X by pointing to one row of a RRC configured TDRA table per scheduled cell. - For each of the co-scheduled PDSCHs/PUSCHs, the same row index is used. - Separate {SLIV, mapping type, scheduling offset K0 (or K2)} are configured in the row for each of co-scheduled PDSCHs/PUSCHs. |
| ITRI | Support the proposal |
| Moderator2 | @Fujitsu: I think you raised a good question. For this proposal, as mentioned in main bullet, the intention is only covering multi-cell case. @LG: If you agree with using TDRA field pointing to one row of a RRC configured TDRA table, it seems no need to differentiate the TDRA field indication resources for a subgroup or entire set of cells. It is up to configuration. For cells within one subgroup, gNB can configure same SLIV for the subgroup. @Samsung: Ok to capture that. @NTT DOCOMO: I think the actually co-scheduled cells are indicated by the indicator of co-scheduled cells. Using TDRA table for indicating that purpose will lead to too many rows in the table. @Ericsson: I don’t understand how it works if a single TDRA field is used to indicate same row index for all the per-cell configured TDRA tables. It seems lack of flexibility. @All: the proposal now is updated as below: #### Proposal 3-4: - For a set of cells co-scheduled by a DCI format 0_X/1_X, time domain resource allocations for the set of cells are jointly indicated by a single TDRA field in the DCI format 0_X/1_X by pointing to one row of a RRC configured TDRA table. - Separate {SLIV, mapping type, scheduling offset K0 (or K2)} are configured in the row for each of co-scheduled PDSCHs/PUSCHs. |
| Nokia / NSB | Not our preferred way, but willing to compromise here. Just to check: how about size of the single TDRA field – is it limited to 4bits (as without multi-PDSCH/PUSCH), limited to max 6bits (as for multi-PDSCH/PUSCH) or how many rows are we going to support at maximum here? |
| Apple | Support the updated proposal |
| Qualcomm | OK with the updated proposal. |
| LGE | Although we prefer Type-3 field for this TDRA to provide flexibility on TDRA table configuration, we can live with the above FL’s update for the progress. BTW, as commented by Nokia, size of the common TDRA field needs to be considered, and it is reasonable to increase the size compared to legacy due to joint indication. |
| Moderator3 | @Nokia @LG: I think max 6 bits as multi-PDSCH scheduling should be sufficient for TDRA table design. |
| Intel | We are fine with the updated proposal. |
| OPPO | The proposal may need two clarifications in our view. - The TDRA is configured per BWP in current spec. So which of the following is the intended table construction? - Opt-1: table_row_index -> {TDRA for cell-x & BWP-i, TDRA for cell-y & BWP-j, …..}. With this, there is no need to use BWP indicator field. - Opt-2: table_row_index -> {TDRA for cell-x & BWP-i, TDRA for cell-x & BWP-j, TDRA for cell-y & BWP-k, TDRA for cell-y & BWP-h…..}. With this, BWP indicator field is still needed. - Opt-3: table_row_index -> <TDRA for cell-x, TDRA for cell-y, …..>. The selected TDRA applies to whatever BWP determined by BWP indicator field. - Assume a UE is configured with 5 cells that can be co-scheduled. Each 0_X/1_X can schedule 4 cells. The proposal is not made clear between the following two cases: - Case-1: there is only one table serving all co-scheduled cell combinations. - Case-2: there can be up to i=145i=30 tables. UE needs to check CIF field to determine which table is used. The proposal seems to suggest Case-2, by saying “For a set of cells co-scheduled by a DCI format 0_X/1_X”, but we want to check whether this is indeed the intention. |
| NTT DOCOMO2 | We are fine with the updated proposal. |
| Fujitsu | Thanks for FL clarification. Given that, we are fine with the proposal. |
| MTK | OK with the updated proposal. |
| Moderator3 | @OPPO: For your 1st question, I think Opt 3 is preferred as there are BWP indicator and co-scheduled cell indicator. We don’t need to configure too much in the table. For your 2nd question, Case 1 is the intention, one table is defined including several co-scheduled cell combinations. For time being, I think we can FFS the details of the table design. |
| Moderator4 | Let’s continue the discussion in 2nd round based on online discussion outcome. |

### 2nd round of discussions

#### Proposal 3-2rev2:

- At least below fields are excluded from DCI format 1_X/0_X:

- DFI flag

- HARQ-ACK retransmission indicator

- Minimum applicable scheduling offset indicator

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| LGE | We still don’t see the reason to exclude the above fields from DCI 1_X/0_X even in case when the DCI 1_X/0_X schedules only one cell, since the payload size of DCI 1_X/0_X is determined by the maximum size among combinations of co-scheduled cells, as FL already explained in the 1st round as below. Before checking the indicator of co-scheduled cells, UE has to determine the payload size of DCI 0-X/1-X firstly based on the maximum size for the set of cells as discussed in DCI size determination part. Upon detection of the DCI, then UE check indicator of co-scheduled cells to get the info of number of scheduled cells. This is similar to what we have done in Rel-16/17 multi-PUSCH scheduling: CBGTI is present if the DCI schedules a single PUSCH and not present if DCI schedules more than one PUSCH. UE can know the number of scheduled PUSCHs after checking TDRA field.. For this reason, we support the 1st round proposal from the FL below. #### Proposal 3-2rev2: - At least below fields are excluded from DCI format 1_X/0_X which schedules more than one cell: - DFI flag - HARQ-ACK retransmission indicator - Minimum applicable scheduling offset indicator |
| New H3C | OK with LGE’s modificaiton |
| Qualcomm | Maybe we do not need to spend time on this topic, since if we do not reach consensus on the fields that requires particular solution for multi-cell scheduling, the fields will anyway be unsupported. For now, excluding DFI flag, regardless of whether the DCI 0_X schedules one cell or multiple cells, would be acceptable to all. Assuming that, we think we can try following. #### Proposal 3-2rev2 – QC update: - At least below fields are excluded from DCI format 1_X/0_X: - DFI flag |
| Langbo | Support, but also share similar views as QC that the fields without consensus will be automatically excluded from DCI format 1_X/0_X. |
| Nokia/NSB | Agree with Qualcomm. |
| NTT DOCOMO | We don’t have strong view to preclude these fields but can accept this proposal. |
| Apple | We are fine with QC’s updated proposal |
| ZTE | We can support this proposal. |
| MTK | Agree with Qualcomm. Also can live with moderator proposal. |
| Huawei2 | We are fine with moderato proposal. On the other hand, we view that for other fields that are not explicitly precluded, they are by default supported in the DCI 1_X/0_X and could be configurable, e.g. Type 3. We understand some company may consider by default they can be Type 1 and we are open to take that approach, as long as there is no significant extra spec work. |
| Intel | We are fine with the proposal. |
| CATT | We support the proposal. |
| Samsung3 | OK with the proposal, and fine with the change by LGE. |

#### Proposal 3-3rev2:

- For DCI format 1_X/0_X,

- Type-3 fields at least include below:

  - MCS

  - HARQ process number

  - Bandwidth part indicator

  - Frequency domain resource assignment

  - VRB-to-PRB mapping

  - PRB bundling size indicator

  - Rate matching indicator

  - ZP CSI-RS trigger

  - Antenna port(s)

  - Transmission configuration indication

  - SRS request

  - DMRS sequence initialization

  - Frequency hopping flag

  - TPC command for scheduled PUSCH

  - Precoding information and number of layers

  - SRS resource indicator

  - SRS offset indicator

  - PTRS-DMRS association

  - Open-loop power control parameter set indication

  - FFS: Other fields

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| LGE | Not support the proposal We think 7 fields {MCS, HARQ, FDRA, AP, TCI, TPMI, SRI} can be Type-3, but other fields should not be Type-3. Rather, Type-1C is sufficient for those fields. Therefore, the following update is suggested. #### Proposal 3-3rev2: - For DCI format 1_X/0_X, - Type-3 fields at least include below: - MCS - HARQ process number - Frequency domain resource assignment - Antenna port(s) - Transmission configuration indication - Precoding information and number of layers - SRS resource indicator - FFS: Other fields |
| New H3C | OK in principal and we only need propose necessary parameters like LGE’s modification. |
| Qualcomm | It would be better to clarify that number of sub-groups has not been determined. So, type-3 here could be type-2 for some cases/fields. Is this correct? |
| Langbo | The fields listed by LGE are acceptable for us. |
| NTT DOCOMO | We are fine with the proposal in general except HPN. We still think that the common management of HPN across co-scheduled cell is not practical use and prefer to support as Type-2 field. In our understanding, considering the limit of DCI size, i.e., 140 bits without CRC, almost all parameters of Type-3 fields are configured as Type1 field and several field can be configured to indicate separate value for each/subset of co-scheduled cells in practice. We support this framework that gNB can flexibly configure these field to indicate common/separate value for each/subset of the cells. |
| Xiaomi | We don’t think it is good practice to directly discuss type 3 DCI field. Once we identify the DCI field that belongs to type1 and type2, others can go to type 3. |
| Apple | Fine to support |
| ZTE | We are fine with the intention to list the controversial but necessary fields as Type3. However, we think this agreement can be made after we realize it is actually controversial. In addition, we think the FDRA field should be discussed on how to indicate the frequency resource for the co-scheduled cells when it is configured as shared |
| MTK | Fine to support |
| Huawei2 | We do not think we need to discuss Type 3 field at all. We need to spend more time on identify type 1/2 filed that is more critical for DCI size boundary determination, since they are either separate or common. Once we know those, we probably already understand how much the DCI size can be maximumly and minimally, with the understanding that all other fields can configurable. The Type 1 and Type 2 are also critical as boundary of the performance. For example, common field will imply most network restriction while separate field is most flexible. In that sense, we also support MCS and HARQ process number to be separately indicated. |
| CMCC | We think that some fields listed above need to be further discussed and could be identified as Type-1 or Type -2 fields. |
| Nokia / NSB | We like the list of LGE a bit more than the full list. As LG, at least for some of them we don’t fully get the point of having the option to have them not as ‘Type 1’. Flexibility is clearly good but we may not overdo it. So we agree with Xiaomi on this respect. |
| Intel | We do not support the proposal. Many fields in the list can be simply considered as Type 1, e.g., VRB-to-PRB mapping, PRB bundling size indicator, Rate matching indicator, ZP CSI-RS trigger, Antenna port(s), Transmission configuration indication, etc. In our view, only MCS, FDRA, HPN needs to be included in Type 3. |
| Samsung3 | We suggest the following: - {FDRA, antenna port (Aps), SRI, TMPI} as Type-3 fields; - {Rate matching indicator, aperiodic ZP CSI-RS, SRS request, TCI state, frequency hopping} as Type-1; - {MCS, HPN} should be Type-2 field; - {DMRS sequence initialization, VRB-to-PRB bundling field, PRB bundling size} need not be present and can follow RRC configuration; - BWP-ID either not present and following the active BWP, or as Type-1A. In particular for MCS, we think the MCS should be separate per scheduled cell (i.e., Type-2 field). If there is no possibility for link adaptation as in Rel-17 CA, there will simply be no gains from MC-DCI overhead reduction as few tens or even hundreds of DCI bits will be more than offset by system throughput losses due to the inability to do proper link adaptation. If there is concern about overhead, differential MCS should be used as the channel conditions among cells in intra-band CA will not be significantly different (e.g. same path-loss, different fast fading, different interference). We should also consider having same MCS for 2 TBs of a same PDSCH. In addition, when a Type-3 field is configured by the gNB to be cell-specific, more discussion is needed to decide whether they are indicated by a single value (as in Type-1B) or with multiple values (as in Type-2). Also, when FDRA is configured as cell-common, it should be clarified how to determine the FDRA value for cells with different size of UL/DL BWPs. |
| Moderator | To me, I think MCS/HPN should be type 2 fields for scheduling flexibility. However, if RAN1 can’t reach consensus on Type-2 MCS/HPN, I would suggest leaving them as Type 3. So far, I don’t see any other fields except MCS/HPN which should be Type-2 fields. As for overhead MCS/HPN, we can further discuss how to reduce it instead of taking it as Type-3. @All: please check below proposal: #### Proposal 3-3rev3: - For DCI format 1_X/0_X, - Type-2 fields at least include below: - MCS - HARQ process number - FFS: overhead reduction for MCS and HPN |
| CATT | We still think the overhead of MCS field is too large, especially in case of 2 TB scheduling. The differential MCS method can be further considered to reduce the overhead of MCS filed. For the convenience of discussion, it’s better to add the method of ‘differential MCS’ as FFS. #### Proposal 3-3rev3: - For DCI format 1_X/0_X, - Type-2 fields at least include below: - MCS - HARQ process number - FFS: overhead reduction for MCS and HPN - FFS: differential MCS |
| Nokia/NSB | Seems we are going a bit in circles on that one here. Fine with the HPN field – but not ottherwise: On MCS: we think this should be Type 3, leaving the gNB the option to save MCS overhead especially for intra-band CA. On the FFS: we think there is not really time in this WI to discuss any overhead reduction techniques here anymore. Looking at the time left to complete the WI, the only option seems to be to take the current signalling mechanisms we have as they are. |
| Apple | Fine to support with the proposal 3-3rev3 |
| Qualcomm | We think MCS does not need to be Type-2. Link-adaptation in general is carried out using MCS, Antenna port(s), TCI (for DL), TPMI/SRI (for UL). Not clear how much beneficial if MCS only is Type-2 among those fields. If majority companies prefer Type-2 for MCS, we would not object it. |
| LGE | Not support the update Proposal 3-3rev3. We basically think that MC DCI should be designed to have possibility of reducing DCI overhead for indication of MCS and HPN (and FDRA), to be comparable with Rel-17 multi-TTI scheduling DCI. With Type-3, since gNB could select/configure proper field type among Type-2 and Type-1 (per cell or subgroup) according to situation, Type-3 is preferred for gNB flexibility between scheduling flexibility and DCI overhead. Therefore, we suggest the following update. #### Proposal 3-3rev3: - For DCI format 1_X/0_X, - Type-32 fields at least include below: - MCS - HARQ process number - FFS: overhead reduction for MCS and HPN |

#### Proposal 3-4rev1:

- For a set of cells co-scheduled by a DCI format 0_X/1_X, time domain resource allocations for the set of cells are jointly indicated by a single TDRA field in the DCI format 0_X/1_X.

- Separate {SLIV, mapping type, scheduling offset K0 (or K2)} is indicated for each of co-scheduled PDSCHs/PUSCHs.

- FFS details of the TDRA table design

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| LGE | Support |
| New H3C | OK in principal |
| Qualcomm | We are OK with the proposal. |
| Fujitsu | OK with the proposal. |
| Langbo | Support |
| NTT DOCOMO | We support this proposal in general but it should be clarified whether this TDRA field indicates the actually co-scheduled cells by DCI format 0_X/1_X or the actually co-scheduled cells are indicated by another field, e.g., CIF, as discussed on GTW session. In our view, actually co-scheduled cells are indicated by the co-scheduled cell indicator in DCI format 0_X/1_X and this TDRA table does not need to be optimized for actually co-scheduled cells, i.e., this TDRA would be configured for the set of cells configured for multi-cell scheduling. We are also fine to capture this point as FFS. |
| Moderator | @NTT DOCOMO: I have same understanding with you. Since we have co-scheduled cell indicator, I don’t think we need TDRA field to further indicate same purpose. If TDRA table is used for indicating same purpose, I am afraid the TDRA table will be quite complicated and include more rows which requires more bits. |
| Xiaomi | Support |
| Apple | Although we prefer the previous version (before the updates in GTW), but if acceptable by majority, we can accept this |
| ZTE | Support |
| MTK | Fine with the proposal |
| Huawei2 | Support |
| Intel | @Moderator and NTT DOCOMO, our understanding is there could be two options: - Option 1: each row includes N {SLIV, mapping type, scheduling offset K0 (or K2)}, where N is number of actually scheduled cells, where N is determined based on the CIF indication - Option 2: each row includes N {SLIV, mapping type, scheduling offset K0 (or K2)}, where N is number of configured cells, and UE needs to determine the TDRA based on the CIF indication. Our understanding is that the above options are still open. Please correct if our understanding is not accurate. |
| CATT | We support the proposal. |
| Moderator2 | @Intel: Yes, both options are open. That is the intention of “FFS details of the TDRA table design”. If companies would like to discuss the details of table design, we can further share views based on Intel’s options in next round. |
| Samsung3 | OK with the proposal. We suggest to discuss other configuration aspects that impact Type-1 CB, e.g., same or different K0, relationship between TDRA and K1 set for single-cell scheduling and multi-cell scheduling. Without these key elements, it will be difficult to conclude the Type-1 CB specification. |
| Moderator | @All: with below agreement, this discussion can be concluded now. Proposal 3-4rev2: (So far, no concern or objection is received) - For a set of cells co-scheduled by a DCI format 0_X/1_X, time domain resource allocations for the set of cells are jointly indicated by a single TDRA field in the DCI format 0_X/1_X. - Separate {SLIV, mapping type, scheduling offset K0 (or K2)} is indicated for each of co-scheduled PDSCHs/PUSCHs. - FFS details of the TDRA table design For detailed TDRA table design, you are welcome to share your views on below two options: - Option 1: each row includes N {SLIV, mapping type, scheduling offset K0 (or K2)}, where N is number of actually scheduled cells, where N is determined based on the CIF indication - Option 2: each row includes N {SLIV, mapping type, scheduling offset K0 (or K2)}, where N is number of configured cells, and UE needs to determine the TDRA based on the CIF indication. |
| CATT | We slightly prefer option 1. The TDRA table consists of the TDRA table configured for each cell which can be applied to both of legacy single-cell scheduling and multi-cell scheduling. |
| Nokia/NSB | Support the intention of Option 2: For Option 1, if we have a separate indicator (in one way or the other) in the DCI indicating which cells are actually scheduled, Option 1 would mean that we need to duplicate the options of different scheduled cells in the TDRA as well. E.g. considering cells 1&2 and 1, 2 & 3 are scheduled, for Option 1 two rows with the same entries would be needed even though the same TDRA would be applicable for the first 2 cells here. |
| Apple | We prefer option 1. Since the row of the TDRA will be indicated, so it makes sense that the indicate row includes actually scheduled cells. |
| Qualcomm | It seems our understanding is Option 3. - Option 3: A DCI format 0_X/1_X for a cell set associated with a CIF value has a single TDRA field of N bis. Network configures up to 2N {SLIV, mapping type, scheduling offset K0 (or K2)} for each cell in the cell set. If the UE detects the DCI format 0_X/1_X associated with the CIF value, the UE reads the TDRA field having N bits and identifies {SLIV, mapping type, scheduling offset K0 (or K2)} for each cell in the cell set. For a cell in the cell set, if the number of configured {SLIV, mapping type, scheduling offset K0 (or K2)} is smaller than 2N, there could be cases where the codepoint of the TDRA field indicates null – in this case, UE considers that the cell is not scheduled (the scheduling information is not valid). |
| Ericsson2 | In our view, the principle of individual TDRA table configured per scheduled cell from Rel-15/16/17 could be retained even for mc-DCI. Then, each the joint index in the mc-DCI can simply point to an entry of the respective TDRA table for each of the indicated scheduled cells in mc-DCI. This could also avoid duplicate TDRA tables for legacy DCI and mc-DCI (we can discuss that separately if needed). Do the options in FL proposal assume separate (duplicate) TDRA configurations for legacy and mc-DCI? Also, we prefer to use the term “Indicator of co-scheduled cells” from the agreement this meeting. The options mention CIF which currently only has a legacy definition. |

## Indication of scheduled cells

Huawei: - Proposal 8: Indication of co-scheduled cells can be determined through combination of RRC configuration, MAC-CE activation and DCI indication. - Proposal 9: Size of multi-cell scheduling DCI can be determined by the number of co-scheduled cells indicated by MAC CE. ZTE - Proposal 6: A codepoint of CIF field indicates a row of a table comprising the co-scheduled cells. - The table is configured by RRC. - The PDSCH and PUSCH can be configured with different table. Spreadtrum: - Proposal 2: Support Proposal 3-3rev2 in RAN1#110 with update - For multi-cell scheduling, the co-scheduled cells are indicated by an indicator in DCI format 0_X/1_X which points to one row of a table defining combinations of co-scheduled cells. - The table is configured by RRC signalling. - Separate tables can be configured for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling Vivo： - Proposal 9. For the indication of the co-scheduled cell, option1 is supported. - Proposal 10. The maximum number of configurable cells for co-scheduling is 8. China Telcom: - Proposal 7: For the indication of actual co-scheduled cells, a dedicated indicator field pointing to one row of a table indicates maximum number of co-scheduled cells. And within the indicated maximum number of co-scheduled cells, the field containing separate information for each cell indicates whether the cell is actually scheduled or not. - A code point of FDRA, or a code point of MCS and RV is considered for the indication of not scheduling cell(s). OPPO: - Proposal 1: RAN1 determines carrier indication method from bitmap-based indication and cell-combination table based indication. Bitmap-based indication should be selected whenever allowed by the DCI payload budget. CATT: - Proposal 5：The actual co-scheduled cells can be indicated by an indicator in DCI format 0_X/1_X, and each value corresponds to a combination of co-scheduled cells that is configured by RRC signalling. Langbo: - Proposal 9: Option 1, i.e., an indicator in the DCI points to one row of a table defining combinations of scheduled cells is supported for multi-cell scheduling. - Proposal 10: Separate tables are configured respectively for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling. Fujitsu: - Proposal 3: For multi-cell scheduling, the co-scheduled cells are indicated by an indicator in DCI format 0_X/1_X which points to one row of a table defining combinations of scheduled cells. - The indicator is CIF. - The table is configured by RRC signaling. CAICT: - Proposal 3: A table defining combinations of the length of bit fields for each cell scheduling is preconfigured. One flag in the DCI indicates one row of the table which implicitly indicates the scheduled cells. Xiaomi: - Proposal 13: The associations between a CIF value and a combination of co-scheduled cells can be configured following legacy configuration framework to support the MC-scheduling with a single DCI. Lenovo: - Proposal 1: The maximum number of configurable cells for co-scheduling is 16 in Rel-18. - Proposal 2: The co-scheduled cells are indicated in a DCI field pointing to one co-scheduled cell combination from a RRC configured table. CMCC: - Proposal 7. Considering the indication of co-scheduled cells, possible combinations of scheduled cells can be pre-configured by RRC signaling, and the co-scheduled cells are jointly indicated by nCI and indicator in DCI formats 0_X/1_X. FGI: - Proposal 9: Option 3 (first preference) or Option 1 (second preference) is adopted for indicating co-scheduled cells. NEC: - Proposal 1: RRC configures a static configuration of cell combination which includes list of cells used for DCI format 0_X/1_X, and DCI could dynamically indicate whether the cell in the cell combinations is used in this DCI. LG: - Proposal #2: Support Option 1 where the indicator pointing to one of (scheduled) cell combinations is signalled via CIF field in the multi-cell DCI. - Support common CIF table between multi-cell PDSCH scheduling and multi-cell PUSCH scheduling by following current cell-level CIF without DL/UL differentiation. - Proposal #3: Consider the case where an invalid cell (e.g. in deactivated state or with dormant BWP or with UL/DL collision or with invalid FDRA/TDRA) is included within co-scheduled cells. - Drop the PDSCH/PUSCH scheduled for the invalid cell and the corresponding HARQ-ACK feedback is omitted or mapped as NACK. MTK: - Proposal 9: The green part in Figure 2 consist of a bitmap indicating cell(s) scheduled by the 1st and/or 2nd DCI segment: - Bitmap size = number of configured scheduled cells for this scheduling cell (Ex. 4 bc1bc2bc3bc4) - Each bit corresponds to one of the configured scheduled cells, with MSB to LSB of the bitmap corresponding to the first to last configured scheduled cells in ascending order of ServCellIndex - Number of 1’s in the bitmap <= maximum number of cells that can be scheduled simultaneously (Ex. 3) - Only an “1” in the bit map means there is no 2nd segment (Only for 2-segment DCI design) Apple: - Proposal 3: The maximum number of co-scheduled cells by both DCI format 0_X and format 1_X in Rel-18 is 4 - Maximum number of configurable cells for co-scheduling is 4 - Maximum number of configurable/co-scheduled cells exactly supported by UE is subject to UE capability - Proposal 8: The co-scheduled cells are indicated in the DCI using a bitmap corresponding to a set of configured cells that can be scheduled by the DCI 0_X/1_X Samsung: - Proposal 3: RRC configures ‘cell-set-level’ CIF values that correspond to configured sets of co-scheduled cells (Option 1). Ericsson: - Proposal 15: A carrier selection field (CSF) is introduced for indication of co-scheduled cells by DCI format 0_X/1_X, wherein the field is a bitmap corresponding to the set of configured cells that can be scheduled by the DCI format. NTT DOCOMO: - Proposal 1: For multi-cell scheduling, the co-scheduled cells are indicated by an indicator in DCI format 0_X/1_X which points to one row of a table defining combinations of co-scheduled cells. - The table is configured by RRC signaling with up to 8 entries. - Separate tables can be configured for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling. - Reusing CIF field in the DCI as the indicator ITRI: - Proposal 3: - For multi-cell scheduling, the co-scheduled cells are indicated by the CIF field. The mapping between co-scheduled cells and CIF is configured by RRC signaling. Google: - Proposal 3: For multi-cell scheduling, the co-scheduled cells are indicated by an indicator in DCI format 0_X/1_X which points to one row of a table defining combinations of co-scheduled cells, where the table is configured by RRC signalling. Nokia, Nokia Shanghai Bell - Proposal 4.1: Adopt the latest moderator Proposal 3-3rev2 from RAN1#110-e, with the following proposed changes in red: #### Proposal 3-3rev2: - For multi-cell scheduling, the co-scheduled cells are indicated by an indicator in DCI format 0_X/1_X which points to one row of a table defining combinations of co-scheduled cells. - The table is configured by RRC signaling. - FFS: Separate tables can be configured for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling. - FFS: reusing CIF field in the DCI as the indicator - Introduce a new ‘co-scheduled cell indicator’ with a bitwidth of élog2(table size)ù.

### Moderator summary and proposals based on contributions

Regarding indicator of co-scheduled cells, RAN1#109e meeting reaches below agreement:

Agreement For multi-cell scheduling, the co-scheduled cells are indicated by DCI format 0_X/1_X. At least the following options are considered: - Option 1: An indicator in the DCI points to one row of a table defining combinations of scheduled cells. - The table is configured by RRC signaling. - FFS: Separate tables can be configured for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling. - Option 2: An indicator in the DCI is a bitmap corresponding to a set of configured cells that can be scheduled by the DCI 0_X/1_X - FFS: Separate sets of configured cells for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling. - Option 3: using existing field (e.g., CIF, FDRA) to indicate whether one or more cells are scheduled or not - Other options are not precluded. - Note: It does not preclude other DCI information fields (e.g., BWP) to be jointly indicated by the indicator of the co-scheduled cells.

Regarding the detailed design of indicator of co-scheduled cells, 22 companies have expressed their views and preference which are listed below:

Option 1:

- Supported by 19 companies [ZTE, Spreadtrum, vivo, China Telecom, CATT, Langbo, Fujitsu, CAICT, xiaomi, Lenovo, CMCC, FGI, NEC, LG, Samsung, NTT DOCOMO, ITRI, Google, Nokia]

Option 2:

- Supported by 4 companies [Oppo, MediaTek, Apple, Ericsson]

Option 3:

- Supported by 1 company [FGI]

Other options:

- Huawei

Based on above, vast majority companies prefer Option 1: predefining a table with each row defining a combination of scheduled cells and using DCI to indicate one row of the table. So the DCI overhead can be reduced and the scheduling flexibility is guaranteed. Regarding Option 2, as mentioned by majority companies, the bit overhead for indicating the co-scheduled cells is a main concern, especially when the UE is configured with a large number of cells. Option 3 may need more clarification on detailed design. Since usually more DL serving cells are aggregated in DL CA compared to UL CA, separate configuration of tables for DCI format 0_X and 1_X are needed.

Hence, moderator suggests going with Option 1 to make progress on this issue.

This is one FFS issue on maximum number of configurable cells for co-scheduling, one company [Lenovo] propose max 16 cells configured for multi-cell scheduling considering the maximum number of aggregated cells per cell group is 16. One company [vivo] propose maximum number of configurable cells for co-scheduling is 8. One company [Apple] propose maximum number of configurable cells for co-scheduling is 4. One company [Nokia] propose maximum number of configurable cells for co-scheduling using DCI formats 0_X / 1_X is determined by the number of cells within a PUCCH group but not limited otherwise (i.e., >4 configurable cells are supported). Option 1 can support multiple scheduled cell combinations among a large number of cells within a same PUCCH cell group.

From moderator’s point of view, it is not necessary to restrict maximum 4 cells configured for co-scheduling.

### 1st round of discussions

#### Proposal 3-5:

- For multi-cell scheduling, the co-scheduled cells are indicated by an indicator in DCI format 0_X/1_X which points to one row of a table defining combinations of co-scheduled cells.

- The table is configured by RRC signaling.

- Separate tables can be configured for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| MTK | We can live with the proposal. |
| Qualcomm | We are OK with the main bullet and 1st sub-bullet. We are still not sure what the 2nd sub-bullet means. We need FFS on the 2nd sub-bullet unless the other details of PDCCH monitoring are clear. |
| Nokia, NSB | Support |
| Apple | Support the proposal |
| Langbo | Support |
| Huawei, HiSilicon | General Ok |
| Intel | We are fine with the proposal. |
| ZTE | We support this proposal. This can ensure the scheduling flexibility with the minimum indication overhead. The configuration for the downlink and uplink can be different since the UE may have different capabilities for downlink and uplink or different data rate requirements. Regarding the number of configurable cells for co-scheduling, we think it should not exceed 4 because we don’t see the need to support a larger number. On the contrary, supporting a number greater than 4 may bring more issues. For example, it may increase the DCI size because the network needs to consider all the co-scheduled cells. If the network wants to support more than 4 cells for multi-cell scheduling, these cells can be divided into more than one groups. Each group has at most 4 co-scheduled cells. This can improve the flexibility on the network configuration and can reduce the DCI size for format 0_X/1_X compared than configuring these cells in one group. |
| Moderator | @Qualcomm: Since UL CA and DL CA may be different on the configured number of cells, separate tables can be configured for DL and UL. |
| CMCC | We prefer to discuss the detailed indication method after the discussion of Alt 1/Alt 2/Alt3 in Proposal 2-6. |
| New H3C | Support |
| Fujitsu | We are generally fine with the proposal. But we are not sure whether this proposal is also applied to single-cell scheduling by DCI format 0_X/1_X. Hopefully it can be clarified. |
| CATT | Support the proposal |
| vivo | Support |
| LGE | We are fine with the proposal except for the last sub-bullet. We think common CIF table for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling is reasonable extension of current cell-level CIF table without DL/UL differentiation. Moreover, it could avoid potential PDCCH related complexity in terms of SS set composition and BD operation. |
| Samsung | Support |
| Spreadtrum | Support The UL cell number is always less than DL cell. DL cell combinations cannot directly get the right UL cells, sometimes it may result in single UL cell scheduling. Second, separate UL tables can configure suitable UL co-scheduling cells, reduce unnecessary UL cell combinations, and set proper multi-cell scheduling DCI fields types and bits number. |
| Xiaomi | We can live with this proposal, but wants to clarify what is the “an indicator” here means. In our understanding, we can follow the legacy design, that means using the CIF as the indicator. |
| NTT DOCOMO | Support this proposal. |
| Ericsson1 | Not OK. Since the maximum number of co-scheduled cells is 4, the number of bits needed in a DCI 0_X/1_X is a maximum of four bits. In many other typical cases, the number of cells will be 2 or 3. Given this, there is no need to create unnecessary complexity by configuring an explicit table via RRC. |
| China Telecom | We prefer to discuss the proposal after we get common understanding on the maximum number of configurable cells for co-scheduling. |
| Apple2 | We are also fine with Ericsson’s proposal to consider bitmap corresponding to 4 co-scheduled cells. |
| Moderator2 | @Ericsson @China Telecom: To me, it is not necessary to restrict maximum 4 cells configured for co-scheduling. The maximum number of cells for co-scheduling can be max number of cells within a PUCCH group. @All: this proposal is updated below. #### Proposal 3-5rev1: - For multi-cell scheduling, the co-scheduled cells are indicated by an indicator in DCI format 0_X/1_X which points to one row of a table defining combinations of co-scheduled cells. - The table is configured by RRC signaling. |
| Nokia / NSB | Could we at least keep the separately configured tables as FFS for now? To have at least only a sub-set of cells schedulable there for PUSCH compared to PDSCH due to usual UL CA UE limitations? There seems to be some confusion within companies of the CIF field: Based on proposal 2.6 – single n_CI value configured, the CIF field would need to indicate the n_CI to differentiate from single-cell x-scheduling DCIs as QC pointed out in their contribution. But this CIF could then not be used at the same time to indicate the (sub)set of scheduled cells! So this cannot be mapped in this respect to the x-scheduling framework in this respect to our understanding. |
| Apple | We are fine with the updated proposal |
| Qualcomm | OK with the proposal. Thanks for the update. |
| LGE | We are also fine with the above update. |
| Moderator3 | @Nokia: I think this possibility of separate TDRA table for DL and UL is still there as we don’t exclude it. We can also discuss separate table when preparing RRC parameters. Regarding CIF field, my understanding is a single n_CI value is configured for the set of cells. When DCI format 0_X/1_X can be used for single cell scheduling, one way is to configure single cell cases in the rows of the table. |
| Intel | We are fine with the proposal. |
| OPPO | We share the view with China Telecom: To discuss/determine the max number of cells for potential co-scheduling. If this max number can be relative large, it may deserve to consider a upper-bound on number of rows in this CFI table. |
| Langbo | Although we prefer to at least keep the DL/UL separate tables as FFS, we can live with this updated proposal. |
| NTT DOCOMO2 | We are fine with the updated proposal. |
| Fujitsu | According to the discussions, we understand that this proposal also covers the case of single cell scheduling by DCI format 0_X/1_X. However, it is not quite clear by the current text. It may help if we implement the following updates:s For multi-cell scheduling, the one or more co-scheduled cells are indicated by an indicator in DCI format 0_X/1_X which points to one row of a table defining combinations of co-scheduled cells. |
| MTK | We kind of agree with Ericsson1 that RRC configuration may be a waste if the number of configurable cells for co-scheduling is less than or equal to 4 as mentioned by ZTE. We think a bitmap mechanism is good enough. Having said that, we can live with current proposal. |
| CMCC | We think supporting more than 4 cells can be configured for multi-cell scheduling and n_CI value in Proposal 2-6 need to be discussed first. |
| Moderator4 | @Fujitsu: In the beginning of this bullet, it says, “for multi-cell scheduling”, so no need to add “one or more” here. |
| Moderator4 | Let’s continue the discussion in 2nd round with addition of max number of configured cells for co-scheduling. |

### 2nd round of discussions

#### Proposal 3-5rev1:

- For a set of cells which is configured for multi-cell scheduling, maximum number of cells within the set of cells can be larger than 4.

- For multi-cell scheduling, the co-scheduled cells are indicated by an indicator in DCI format 0_X/1_X which points to one row of a table defining combinations of co-scheduled cells.

- The table is configured by RRC signaling.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| LGE | Support the 2nd bullet, but not OK with the 1st bullet since it may affect UE complexity and implementation in terms of PDCCH BD/SS/DCI size aspects. Therefore, it is better to be UE capability, and we suggest the following update. #### Proposal 3-5rev1: - For a set of cells which is configured for multi-cell scheduling, maximum number of cells within the set of cells is considered as UE capability can be larger than 4. - For multi-cell scheduling, the co-scheduled cells are indicated by an indicator in DCI format 0_X/1_X which points to one row of a table defining combinations of co-scheduled cells. - The table is configured by RRC signaling. |
| New H3C | OK in principal |
| Qualcomm | We tend to agree with LGE that, because of the current direction on PDCCH monitoring in the previous sections, indeed the number of cells within the set of cells impact on the UE complexity. We suggest to postpone the discussion on the 1st bullet until we see further details of PDCCH monitoring aspects (including BD/CCE/DCI-size issues). We support the 2nd bullet. |
| Fujitsu | To FL, Thanks for your clarification. However, if we do not add “one or more”, we have a question. Is the method also applied to the case of single cell scheduling by DCI format 0_X/1_X? Or, do we intend to separately discuss how to indicate scheduled cell for single cell scheduling by DCI format 0_X/1_X? |
| Langbo | Fine with the proposal. |
| Nokia/NSB | Support the proposal. |
| NTT DOCOMO | We support the first bullet. We also support the second sub-bullet. In our understanding, this table is configured separately from that for cross-carrier scheduling by legacy DCI. |
| Moderator | @LG @QC: Since the first bullet is relevant to co-scheduled cell indicator, three companies who prefer bitmap would like to have some conclusion on the first bullet. @Fujitsu: I think your question is more relevant to the detailed table design. Since the DCI 0_X/1_X also support single-cell scheduling, the table should include single cell cases; otherwise, I don’t know to how to support it. |
| Xiaomi | We are fine with the second bullet. For the first bullet, we have the same feeling that it may be associate to the BD/CCE issues and thus can be postponed. |
| Apple | We can support the 2nd bullet, but 1st bullet is not acceptable to use, due to similar reasons mentioned by LGE. However, if we want to conclude on specific value, then we would be fine to support up to 4, but not larger than 4. We have serious concern on UE complexity related to PDCCH monitoring if maximum number of cells within the cells can be larger than 4. |
| ZTE | We support the second bullet. For the first bullet, as commented above, we don’t see the benefit of supporting more than 4 configurable cells sine dividing more than 4 configurable cells into multiple group with each group supporting multi-cells scheduling is a better choice. It can bring more flexibility and is beneficial for DCI size and scheduling. We suggest removing the first bullet. |
| MTK | Similar view as Apple/LGE. Also agree with ZTE. |
| Huawei2 | Support 2nd bullet. |
| Nokia/NSB | Question to companies not supporting the first bullet: How do you then operate the case of up to 8 cells within a PUCCH cell group? Do you need to configured a second MC-DCI configuration (separately e.g. for cells 1…4 ad 5…8). With being able to configure more of them, a single MC-DCI configuration would be sufficient (… with no issues in terms of DAI interpretation etc.). So if we don’t go for the first bullet, what is your solution to the case of having more than 4 cells in the PUCCH group? |
| Intel | We can be fine to support > 4 configured cells for multi-cell scheduling. However, we suggest to put a limit, e.g., 8. This would have impact on the signalling structure. We are fine with the second bullet. |
| CATT | We support the proposal. |
| Moderator | Since no one object the 2nd bullet, we can delete the first bullet and discuss it when preparing RRC parameters. #### Proposal 3-5rev2: - For a set of cells which is configured for multi-cell scheduling, maximum number of cells within the set of cells can be larger than 4. - For multi-cell scheduling, the co-scheduled cells are indicated by an indicator in DCI format 0_X/1_X which points to one row of a table defining combinations of co-scheduled cells. - The table is configured by RRC signaling. |
| Samsung3 | Support the previous version and fine with the updated proposal. We think the maximum number of cells with configuration for DCI format 0_X/1_X can be up to 8 cells for a given scheduling cell, same as in Rel-17 CA framework. |
| Nokia/NSB | Maybe we could have separate agreements on first or second bullet. But we think we need a decision also on the first bullet ASAP, as this will affect on other related discussions here as well. |
| Moderator4 | @All: Seems we have to agree something about maximum number of cells within the set of cells before we conclude the indication of scheduled cells. Let me update the proposal as below: #### Proposal 3-5rev3: - For a set of cells which is configured for multi-cell scheduling, maximum number of cells within the set of cells can be up to 8. - For multi-cell scheduling, the co-scheduled cells are indicated by an indicator in DCI format 0_X/1_X which points to one row of a table defining combinations of co-scheduled cells. - The table is configured by RRC signaling. |
| CATT | Share same view with Samsung, we support the updated proposal. |
| Nokia/NSB | Support rev3. |
| Apple | Sorry, but we cannot support the proposal with value 8. This is highly undesirable from UE complexity point of view with multi-cell scheduling. If we want to agree on maximum configurable cells, we support 4. Otherwise, as a compromise, we can support that “the maximum number of cells within the set of cells is reported as a UE capability” |
| Qualcomm | We do not think the 1st bullet is reasonable as mentioned by Apple. We support the 2nd bullet as we have discussed over email, assuming that this does not mean we agree something about RRC signalling. |
| LGE | Regarding the 1st bullet, does it mean that the spec would support up to 8 as the maximum, and actual maximum supported by UE would be reported as capability? If the above understanding is correct, we are fine with the updated Proposal 3-5rev3. |
| Moderator | @LGE: for 1st bullet, yes, it means spec can support up to 8 as the maximum, and actual maximum supported by UE would be reported as capability. @Apple @Qualcomm @Nokia: Please check whether below update is OK with you. #### Proposal 3-5rev4: - For a set of cells which is configured for multi-cell scheduling, RAN1 specification supports up to 8 cells within the set of cells . - The maximum number of cells within the set of cells is reported as a UE capability. - For multi-cell scheduling, the co-scheduled cells are indicated by an indicator in DCI format 0_X/1_X which points to one row of a table defining combinations of co-scheduled cells. - The table is configured by RRC signaling. |
| Ericsson2 | We still have concerns on Moderator proposal 3-5rev4. Copying the FL email comments below and our followup responses. Regarding Proposal 3-5rev2, for your first question, my understanding is RRC table can also reduce bit overhead even in your example which seems an extreme case. Assuming only CC1 and CC2 are configured in the set of cells for multi-cell scheduling, RRC table can be configured with only two rows, one row only including CC1, another row including both CC1 and CC2. Then only one bit in the DCI 0_X/1_X is enough. As for CC2, it can be solely scheduled by legacy DCI. [Ericsson] Thanks for explanation. This is exactly our point. RRC table leads to extra blocking for CC2 scheduling, while even without the RRC table, each cell can be scheduled with mc-DCI or legacy DCI. Regarding the maximum number of cells with configuration for DCI format 0_X/1_X, I think it can be up to 8 cells for a given scheduling cell within one PUCCH group. I don’t think this max number will lead to DCI format design and DCI size/BD/CCE complexity. Because maximum number of co-scheduled cells is 4 in specification and can be configurable to UE, the DCI format design is based on the value of 4 instead of the maximum number of cells configured for co-scheduling. It is similar to maximum 2 MCS/NDI/RV fields included in the DCI format, which is not based on the max 16 cells which can be configured with 2 codewords per cell. As for DCI size/BD/CCE complexity, it depends on the detailed solution. E.g., if we can agree the DCI size/BD/CCE budget is counted on one or two cells of the entire set of cells, then it has no extra complexity due to max 8 cells configured in the entire set of cells. [Ericsson] Thanks for explanation. In our view, the maximum should be 4 cells per set. It will increase UE complexity since the DCI size has to be determined based on the “configuration of 8 cells” even though at max four cells can be co-scheduled at a time. If a cell1 can be co-scheduled as (cell 1,2,3,4) and also as (1,5,6,7) – this means the DCI size/BD/CCE for scheduling cell 1 has to consider the RRC configuration of all seven cells instead of just three cells – this is unnecessarily complex. For more than four cells in a PUCCH group, two mc-DCIs can be used. |
| Nokia/NSB | @Ericsson Having more than ‘one’ MC-DCI (with different set of cells schedulable) operational brings the following additional problems / issues compared to a ‘single’ MC-DCI: - As the DCI size is independently ‘derived’ (if the payload size is not directly RRC configured) leads to the case of having potentially two additional DCI sizes that need to be handled. If DCI size alignment is performed to maintain the DCI size budget, this will increase the needed DCI size alignment even more. - If the DCI sizes of the two MC-DCIs is the same, how to distinguish the two MC-DCIs here? If the intention is to use different n_CI values, this again will lead to increase required number of BD / CCEs that need to be handled. If the same n_CI is to be applied, we may need again a differentiation bit between the 1st and 2nd MC-DCI configured DCIs? Clearly, if the DCI size is ‘derived based on RRC’ configuration (as seems to be the current trend, we would have preferred direct RRC configuration of the RRC payload) needing to consider more cells will increase the complexity there. But we think this additional complexity will definitely outweigh the drawbacks of needing to have two independent configurations (and handling) of more than one MC-DCI for PDSCH and more than one MC-DCI for PUSCH within a cell group. |
| Moderator2 | @Ericsson: I agree with Nokia that there may be DCI size alignment issue and DCI identification issue if max 4 cells are included per set. |
| Ericsson3 | Thanks for further comments. Can moderator clarify the following since the discussion on multiple sets seems to be linked between multiple proposals? Aren’t multiple sets of cells allowed with below proposal also? Also, can proponents of value 8 clarify how DCI size distinguishment, size alignment, BD/CCE handling, etc becomes simpler if 8 is used instead of 4? #### Proposal 2-6rev1: - For monitoring PDCCH candidates for a set of cells which is configured for multi-cell scheduling, the n_CI in the search space equation is determined by a value configured for the set of cells. - The UE can be configured one or multiple sets of cells which are configured for multi-cell scheduling. - When multiple sets of cells are configured for multi-cell scheduling, separate n_CI values are configured for different sets of cells. - When multiple sets of cells are configured for multi-cell scheduling, a cell in one set of cells can’t be included in another set of cells. In our understanding, assuming eight CCs configured for the UE,for full carrier scheduling, two mc-DCIs are anyway needed within a slot to be able to simultaneously schedule all eight carriers even if 8 is chosen instead of 4. Then with 8, it can become easily complicated if the carrier grouping indication in the DCIs keeps varying assuming at every scheduling instance, there is flexibility to group the eight carriers into up to two DCIs. At this point in the WI phase, it is preferrable to use the simple option of using 4 also for maximum number of configurable cells. |

## Other related issues

ITRI: - Proposal 4: - A DCI format 0_X can activate/release a type 2 CG PUSCH transmission on a co-scheduled cell. - A DCI format 1_X can activate/release a SPS PDSCH reception on a co-scheduled cell.

# HARQ enhancements

Based on contributions submitted by companies, below issues are prioritized for discussion in this meeting. Within each sub-section, the summary from moderator’s perspective is listed and followed by draft proposals for further discussion round by round.

## Background and submitted proposals

Regarding this issue, companies’ views are summarized as below:

Huawei, HiSilicon - Proposal 13: Reference PDSCH can be the last PDSCH of co-scheduled PDSCHs by multi-cell scheduling DCI. - Proposal 14: For Type-2 HARQ-ACK codebook, some details can be clarified as follows: - C-DAI/T-DAI is counted per DCI. - A DCI scheduling more than one cell is associated with the second sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one. - If at least one cell of the set of cells which can be co-scheduled by a DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell can be the number of TBs of maximum cells which can be co-scheduled. - Proposal 15: Multi-cell PDSCH/PUSCH scheduling and multi-PDSCH/PUSCH scheduling transmission on the same or different cell within a same PUCCH group are not expected to be configured simultaneously. Unless time allows, related design of HARQ-ACK codebook can be further discussed. ZTE - Proposal 12: The first PDSCH scheduled by DCI 1_X and the corresponding k1 offset should be used to determine the PUCCH slot. - Proposal 13: No additional is needed for supporting the Type-1 codebook for multi-cell scheduling. - Proposal 14: MC-DCI should be associated with the first sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one. - Proposal 15: The number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell is equal to 2*N when at least one cell of the set of cells which can be co-scheduled by a DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling. Spreadtrum: - Proposal 14: It can use the cell index as the reference, i.e. the PDSCH of the lowest cell index - Proposal 15: If Type-1B is used for Time domain resource allocation field of DCI 1_X, SLIV pruning and K1 set extension should be applied for Type-1 HARQ-ACK codebook - Proposal 16: Clarify whether or not eType3 HARQ-ACK codebook are in the scope. Vivo - Proposal 11. For multi-PDSCH scheduled by DCI format 1-X, the reference PDSCH to determine the PUCCH transmission for HARQ-ACK report is the PDSCH with the latest starting time among the co-scheduled PDSCHs. - Proposal 12. UE does not expect to be configured with both multi-PDSCH/PUSCH scheduling and multi-cell PDSCH/PUSCH scheduling for the same or different scheduled cells within a same PUCCH cell group. - Proposal 13. For type-1 HARQ-ACK codebook, K1 extension should be supported, additionally, the joint coded SLIVs of other co-scheduled cells need to be considered, and the R15 SLIV pruning procedure can be reused. - Proposal 14. The HARQ-ACK for multi-cell PDSCH/PUSCH scheduled by DCI format 1-X should be in the second sub-codebook even when the actual number of PDSCH reception is one. - Proposal 15. For type 2 HARQ-ACK codebook, if at least one cell of the set of cells which can be co-scheduled by a DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling, the number of HARQ-ACK information bits for each DCI format 1_X that can schedule more than one cell should be the maximum of the number of HARQ-ACKs corresponding to the sets of co-scheduled cells. - Proposal 16. For type 2 HARQ-ACK codebook, the reference PDSCH to determine the DAI counter is the PDSCH with smallest serving cell index among the set of co-scheduled cells. - Proposal 17. The type 3 HARQ-ACK codebook can be supported for multi-cell PDSCH scheduling. OPPO: - Proposal 11: The reference PDSCH used for HARQ-ACK timing determination is the last configured PDSCH, e.g. the PDSCH with the last ending symbol among the co-scheduled PDSCHs. - Proposal 12: DCI scheduling more than one cell is associated with the first sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one. - Proposal 13: The following alternatives can be considered to determine the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell: - Alt 2: The number of HARQ-ACK bits for each DCI format 1_X is the sum of largest N values in {Ci}, Ci is the maximum number of codewords for cell i in the set of cells ； - Alt 3: The number of HARQ-ACK bits for each DCI format 1_X is max{Mj}, where Mj is the sum of the maximum number of codewords configured for each cell in the cell combination j. - Proposal 14: The smallest/largest serving cell index among the co-scheduled cells is used as reference for C-DAI counting. - Proposal 15: For Type-1 HARQ-ACK codebook construction, the following alternatives can be considered: - Alt 1: the candidate PDSCH slots are still determined by the configured K1 set while restrict that the slot offset(s) between the co-scheduled PDSCH(s) and the HARQ-ACK is always within the configured K1 set. - Alt 2: the K1 set is extended based on the slot offset between the reference PDSCH and other co-scheduled PDSCHs and the candidate PDSCH slots are determined by the per-cell extended K1 set. - Proposal 16: DCI format 1_X should not be used to trigger HARQ-ACK retransmission. - Proposal 17: To trigger a Type-3 HARQ-ACK codebook, One-shot HARQ-ACK request field in DCI format 1_X is set to “1”. - Furthermore, if no FDRA field is valid in the same DCI, no PDSCH is scheduled. - Proposal 18: To trigger an eType-3 HARQ-ACK codebook, One-shot HARQ-ACK request field in DCI format 1_X is set to “1”, - If Enhanced Type 3 codebook indicator field is configured as one Type-1A field, it indicates a set index of configured CCs/HARQ processes; - If Enhanced Type 3 codebook indicator field is not configured, - If every MCS field in DCI 1_X is used to carry valid MCS information for a TB, the first set of configured CCs/HARQ processes is triggered; otherwise, the first MCS field not carrying valid MCS information for a TB is used to indicate a set index of configured CCs/HARQ processes. CATT: - Proposal 13: The reference PDSCH of K1 should be defined as the last PDSCH among actual co-scheduled cells by a DCI format 1_X. - Proposal 14: For Type-2 CB of multi-cell scheduling, a reference serving cell index can be defined to count DAI or determine last DCI format, where the reference serving cell index can be the smallest serving cell index among the actual co-scheduled cells. - Proposal 15: A DCI scheduling more than one cell is associated with the second sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one, and the UE reports NACK for the PDSCH(s) collision with semi-static UL symbol(s). - Proposal 16: The number of HARQ-ACK bits can be equal to N+M, where N is the maximum number of cells and the M is the number of cells configured with maximum 2 codewords per PDSCH without spatial bundling within the cells which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE. - Proposal 17: When Type-1 HARQ-ACK is configured for multi-cell scheduling, the K1 set extension procedure should be performed for each cell based on the K1 set and TDRA table configured for multi-cell scheduling. - Proposal 18: When a cell can be scheduled by both DCI format 1_X and legacy DCI format, the actual K1 set for Type-1 CB generation is provided by the union of K1 set configured for legacy single-cell scheduling and the extended K1* set for multi-cell scheduling. Lanbo： - Proposal 11: Rel-15/16 Type-1 HARQ-ACK codebook construction is reused if separate TDRA fields are used in DCI format 1_X. - Proposal 12: The PDSCH that ends last is used as the reference PDSCH for corresponding HARQ-ACK slot determination. Intel: - Proposal 1 - Configuration of both multi-cell PDSCH scheduling and multi-PDSCH scheduling for the same or different cell within a PUCCH group is supported. - Configuration of both multi-cell PUSCH scheduling and multi-PUSCH transmission in the same or different cell is supported. - Proposal 13 - For PUCCH slot/sub-slot determination, the reference PDSCH is the last PDSCH, i.e., with the last ending symbol in co-scheduled cells. - Proposal 14 - ForType-1 HARQ-ACK codebook for multi-cell scheduling, - For candidate DL slots, with restriction of same SCS for carriers, the candidate DL slots can be determined by minor modification of Rel-17 multi-PDSCH scheduling extended K1 based on K1 for reference PDSCH, and slot offset between reference PDSCH and PDSCHs in different CCs, or simplify based on configured K1 values as in Rel-15 with the restriction that the effective K1 for each CC is always a subset of the configured K1. - For candidate PDSCH within a DL slot, SLIV pruning is based on SLIVs for the corresponding CC, i.e., only adding the single SLIV for the corresponding CC rather than all SLIVs of the row into a set for SLIV pruning for that CC. - Proposal 15 - Type-2 HARQ-ACK codebook considers at least two sub-codebooks for single-cell PDSCH and multi-cell PDSCH scheduling. - PDCCH scheduling single PDSCH is in 1st sub-codebook and PDCCH scheduling PDSCHs in multiple cells regardless of number of cells with valid PDSCHs is in 2nd sub-codebook. - For 2nd sub-codebook, if at least one cell of the set of cells which can be co-scheduled by a DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling, the number of HARQ-ACK bits per DCI format 1_X is determined by the maximum sum of number of TBs for co-scheduled PDSCHs by the DCI format 1_X. - DAI is separately counted within each sub-codebook. - For 2nd sub-codebook, DAI ordering is based on the serving cell index of a reference PDSCH per PDCCH, which is determined by the PDSCH with smallest serving cell index. - For simultaneous configuration of multi-cell PDSCH scheduling and multi-PDSCH transmission with different DCIs on the same or different cell(s) within a same PUCCH group, both multi-cell PDSCH and multi-PDSCH scheduling belong to 2nd sub-codebook. CAICT: - Proposal 4: The reference PDSCH is the last one among these PDSCHs. - Proposal 5: When the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one, the corresponding HARQ-ACK is included in the first sub-codebook. Xiaomi: - Proposal 14: The reference PDSCH is the last co-scheduled PDSCH in time domain. - Proposal 15: Further check if it is a valid case that the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one if more than one cell is co-scheduled by the MC-DCI - Proposal 16: If at least one cell of the set of cells which can be co-scheduled by a DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell is equal to the actually number of co-scheduled TBs. Lenovo - Proposal 16: Simultaneous configuration of multi-cell scheduling and multi-PDSCH scheduling within a same PUCCH group is not supported in Rel-18. - Proposal 17: For Type-2 HARQ-ACK codebook, whether a DCI scheduling more than one cell is associated with the first sub-codebook or the second sub-codebook is based on the number of co-scheduled cells indicated in the DCI. - Proposal 18: For Type-2 HARQ-ACK codebook, if at least one cell of the set of cells which can be co-scheduled by a DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling, the number of HARQ-ACK information bits per DCI format 1_X is equal to the maximum number of co-schedulable TBs among all the possible scheduling cell combinations. - Proposal 19: The last PDSCH among PDSCHs co-scheduled by one DCI format 1_X is used as the reference PDSCH for determining the PUCCH slot. FGI: - Proposal 10: The reference PDSCH is the PDSCH received in the lastet DL slot (nD), and if there are more than one PDSCHs received in the latest DL slot (nD), the PDSCH received in the cell with the lowest serving cell index is determined as the reference PDSCH. - Proposal 11: The reference PDSCH is used for DAI counting. - Proposal 12: For Type-2 HARQ-ACK codebook, if at least one cell of the set of cells which can be co-scheduled by a DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell is equal to 2*N. NEC: - Proposal 3: For HARQ-ACK codebook Type-1 and for joint TDRA indication among cells, a reference cell among multiple cells is used to generate type-1 codebook. Each PDSCH on non-reference cell has the same HARQ-ACK bit position with the corresponding PDSCH indicated by TDRA on the reference cell. LG: - Proposal #16: Consider the last (ending) PDSCH among co-scheduled cells (with actual PDSCH reception) as the reference PDSCH used to determine HARQ-ACK feedback timing. - Proposal #17: Support simultaneous configuration of multi-cell PDSCH/PUSCH scheduling and Rel-17 multi-slot PDSCH/PUSCH scheduling within a same PUCCH group. - Proposal #18: Consider how to construct Type-1 HARQ-ACK codebook in case with multi-cell PDSCH scheduling, in terms of following two aspects. - SLIV pruning procedure for the cell schedulable by the multi-cell DCI (e.g. extension of SLIV set as for Rel-17 multi-PDSCH scheduling) - Determination of K1 set for the cell schedulable by the multi-cell DCI (e.g. extension of K1 set as for Rel-17 multi-PDSCH scheduling) - Proposal #19: Consider the following aspects for Type-2 HARQ-ACK codebook in case with multi-cell PDSCH scheduling, to resolve FFS points. - A DCI scheduling multiple cells is associated with the first sub-codebook when only one of the PDSCHs scheduled by the DCI is actually received by UE. - The number of HARQ-ACK bits per multi-cell scheduling DCI in case when at least one of the scheduled cells is configured with maximum 2 TBs per PDSCH, is equal to M where M is the maximum number of TBs which can be co-scheduled (over multiple cells) by a DCI format 1_X in the PUCCH group for the UE. - Proposal #20: Determine the counter-DAI value and the last DCI based on a reference PDSCH among the co-scheduled cells. - The reference PDSCH needs to be determined based on the scheduled cell index and/or the PDSCH start timing. - Proposal #21: Consider other aspects related to the multi-cell PDSCH/PUSCH scheduling, including the followings. - How to indicate TB disabling for PDSCH - How to handle the out-of-order HARQ issue MTK: - Proposal 11: Reference PDSCH can be the last PDSCH of co-scheduled PDSCHs by multi-cell scheduling DCI. Apple: - Proposal 4: Multi-cell scheduling DCI shall not introduce out-of-order PDSCH/PUSCH scheduling or out-of-order HARQ-ACK for any scheduled cell Samsung: - Proposal 18: For determination of the PUCCH resource/slot with HARQ-ACK corresponding to multiple PDSCHs on multiple serving cells scheduled by an MC-DCI format, the reference PDSCH is the PDSCH corresponding to the cell with the largest cell index. - The cell index of the reference PDSCH is used to determine the order of DCI formats for PUCCH resource determination. - Proposal 19: The set of K1 values for MC-DCI format 1_X is: - Option 1: separately provided from the sets of K1 values for SC-DCI formats, or - Option 2: same/subset of K1 values configured for SC-DCI formats. - FFS: whether the set of K1 values for MC-DCI format 1_X can include values that are not configured for any SC-DCI format. - Proposal 20: For the Type-1 HARQ-ACK codebook, down-select one of: - Option 1: the UE expects to receive co-scheduled PDSCHs in a same slot (i.e., same K0 value); - Option 2: the UE can receive co-scheduled PDSCHs in different slots (i.e., different K0 value). - Proposal 21: The TDRA table for multi-cell scheduling: - Alt-0: re-use legacy single-cell TDRA tables that are restricted to be same for all co-scheduled cells; - Alt-1: re-use single-cell TDRA tables that can be different for different co-scheduled cells; - Alt-2: a single TDRA table for multi-cell scheduling is determined based on the intersection of single-cell TDRA tables; - Alt-3: a single, cell-common, TDRA table for multi-cell scheduling is configured by RRC; - Alt-4: a single joint multi-cell TDRA table is configured by RRC and provides separate TDRA information for each cell from a set of co-scheduled cells. - FFS: whether the TDRA table for multi-cell scheduling can include rows that are not configured in any single-cell TDRA table. - Proposal 22: For Type-1 HARQ-ACK codebook generation: - When K1 values and TDRA rows for multi-cell scheduling are also provided for single-cell scheduling on a cell, and all co-scheduled PDSCHs have a same K0 value, Type-1 CB is same as in Rel-17; - Otherwise, candidate PDSCH receptions are generated based on: - Only single-cell scheduling TDRA tables for K1 values used only for SC-DCI formats; - Only multi-cell scheduling TDRA tables for K1 values used only for MC-DCI formats; - Union of single-cell and multi-cell scheduling TDRA tables for K1 values common to SC-DCI and MC-DCI formats. - Proposal 23: For the two Type-2 HARQ-ACK sub-CBs in presence of multi-cell scheduling, clarify the following: - HARQ-ACK information corresponding to DCI formats that do not schedule a PDSCH is included in the first sub-CB; - HARQ-ACK information corresponding to an MC-DCI formats 1_X that schedules multiple PDSCHs is included in the first sub-CB when the UE receives only one PDSCH, from the multiple PDSCHs, due to collision with UL symbols. - Proposal 24: When a UE is configured with multi-cell scheduling and Type-2 HARQ-ACK codebook: - if spatial bundling is not configured, the number Nmax of HARQ-ACK bits in the 2nd Type-2 sub-CB for each MC-DCI is a maximum number of associated TBs among different sets of co-scheduled cells. - Proposal 25: For the 2nd Type-2 HARQ-ACK sub-codebook corresponding to multi-cell scheduling, the UE appends a number of “NACK” values, to the HARQ-ACK information bits for co-scheduled PDSCHs, until Nmax bits are generated. - Proposal 26: For a Type-2 HARQ-ACK codebook for multi-cell scheduling, and for an MC-DCI format 1_X that schedules multiple PDSCHs on a set of co-scheduled cells: - the parameter “serving cell” in the definition of counter DAI in the MC-DCI format 1_X is defined based on a largest cell index from co-scheduled cells. - Proposal 27: If a PUCCH overlaps with a PUSCH scheduled by an SC-DCI format and a PUSCH scheduled by an MC-DCI format 0_X, the UE multiplexes the UCI in a PUSCH scheduled by an SC-DCI format. Ericsson: - Proposal 16: For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_X, the reference PDSCH among the set of co-scheduled PDSCHs is the PDSCH with the latest ending symbol. - Proposal 17: Type-1 HARQ-ACK codebook is supported when a DCI 1_X schedules PDSCHs across cells. - Proposal 18: For Type-1 HARQ-ACK codebook generation corresponding to a set of co-scheduled PDSCHs scheduled by a DCI 1_X across cells, the timing occasion corresponding to Kx (e.g., Kx=maxK1 ) in MA,c is used for a co-scheduled PDSCH on cell c that ends earlier than maxK1 UL slots from the corresponding PUCCH slot. In case of presence of other HARQ-ACK information corresponding to occasion Kx, bundling of HARQ-ACK information is performed. - Proposal 19: The value of the DAI field in a DCI format 0_X is applicable for HARQ-ACK multiplexing in any of the PUSCHs when that PUSCH for HARQ-ACK multiplexing is determined following the existing procedures. - Proposal 20: It is not expected that the HARQ-ACK bit(s) carried by a PUCCH to include HARQ-ACK information bit(s)for PDSCH(s) scheduled by a multi-cell DCI together with HARQ-ACK bit(s) for PDSCH(s) scheduled by a multi-slot DCI. - Proposal 21: UL/SUL scheduling combined with multi-cell PUSCH scheduling and presence of UL/SUL indicator in DCI format 0_X should reuse the existing procedures. NTT DOCOMO: - Proposal 15: For PDSCH-to-HARQ feedback timing indicator, the reference PDSCH should be the PDSCH which ends at last in time domain. - Proposal 16: Even when the number of cells with actual PDSCH reception is one due to collision with semi-static TDD DL/UL configuration, a DCI scheduling more than one cell is associated with the second sub-codebook. - Proposal 17: If at least one cell of the set of cells which can be co-scheduled by a DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell should be the multiple of the number of codewords and the maximum number of cells which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE. - Proposal 18: Support simultaneous configuration of multi-slot PDCH scheduling and multi-cell PDSCH scheduling within the same PUCCH group. - For Type-2 HARQ-ACK codebook construction, - 2 sub-codebooks are generated - First sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) which can schedule a single PDSCH and a second sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) which can schedule more than one PDSCH - DAI is counted per sub-codebook - The number of HARQ-ACK information bits for 2nd sub-codebook is determined by the number of PDSCHs scheduled by a single DCI, number of codewords and spatial bundling. - The maximum number of PDSCHs scheduled by a multi-slot PDSCH scheduling DCI and multi-cell PDSCH scheduling DCI would be determined as the larger number between the maximum number of PDSCHs in a row of TDRA table for multi-slot PDSCH scheduling and the maximum number of cells which can be co-scheduled by a DCI format 1_X. Qualcomm: - Proposal 7: - For HARQ-ACK feedback on PUCCH for PDSCH reception(s) scheduled by a DCI format 1_X, the reference PDSCH for PUCCH slot/sub-slot determination is the latest PDSCH amongst the scheduled PDSCH(s) by the DCI format 1_X - A DCI scheduling more than one cell is associated with the second sub-codebook when the number of cells with actual PDSCH reception is one due to collision with uplink symbols configured by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated - If at least one cell of the set of cells which can be co-scheduled by a DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell is equal to 2*N, where N is the maximum number of cells which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE Google: - Proposal 4: Transmissions scheduled by the DCI format 1_X associate with the same HARQ sub-codebook, regardless the number of scheduled cells. - Proposal 5: Multi-PDSCH scheduling is not supported with multi-cell scheduling DCI, TDRA field in the MC-DCI indicates a row in RRC configured table, where the row include only one value. Nokia: - Proposal 5.1.1: The Priority indicator in DCI format 1_X defines the PHY priority of the HARQ-ACK information of all the co-scheduled PDSCHs / cells. - Proposal 5.1.2: dl-DataToUL-ACK is used for operation of DCI format 1_X. - Proposal 5.1.3: The reference PDSCH is the PDSCH of the first cell in the table row of the indicated co-scheduled cells. - Proposal 5.1.4: The PDSCH of the first cell in the table row of the indicated co-scheduled cells and/or its associated cell is used for the last DCI format determination (for PRI) and DAI counting. - Proposal 5.2: Confirm the RAN1#109-e working assumption with the following modifications: Working Assumption - Type-1, Rel-15 Type-2, Rel-16 Type-3 and Rel-17 Type-3All HARQ-ACK codebooks types (Type-1/2/3) are applicable when multi-cellcarrier PDSCH scheduling is configured.. - Proposal 5.3.1: The Type 1 HARQ-ACK codebook construction is not enhanced / changed for the purpose of multi-cell PDSCH scheduling: - FFS: required additional restrictions - Proposal 5.3.2: If the UE is configured with Type 1 HARQ-ACK codebook, the UE is not expecting HARQ-ACK information of a PDSCH scheduled through multi-cell scheduled using DCI format 1_X that cannot be mapped to the Type 1 HARQ-ACK CB of a PUCCH. - Proposal 5.4.1: The HARQ-ACK of a DCI scheduling PDSCH on more than one cell is always associated with the second Type-2 HARQ-ACK sub-codebook (even if the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one). - Proposal 5.4.2: For Type 2 HARQ-ACK codebook construction, if at least one DL cell that can be scheduled by DCI format 1_X is configured with 2-TB PDSCH scheduling without spatial bundling, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell is determined based on 2 times the maximum number of cells co-scheduled by a DCI format 1_X in the PUCCH-group for the UE. - Proposal 5.4.3: To restrict the required changes to / complexity of the Type 2 HARQ-ACK codebook due to multi-slot PDSCH scheduling, adopt either: - Alt. 1: Configuration of both multi-cell PDSCH scheduling and multi-slot PDSCH scheduling for the same or different cell within a PUCCH group is not supported. - or at least Alt. 2: DCI format 1_X does not support multi-slot PDSCH scheduling. It is not expected that the HARQ-ACK bit(s) carried by a PUCCH to include HARQ-ACK information bit(s) for PDSCH(s) scheduled by a multi-cell DCI together with HARQ-ACK bit(s) for PDSCH(s) scheduled by a multi-slot DCI. - Proposal 5.5: Support (Rel-16) Type-3 and (Rel-17) Enhanced Type-3 HARQ-ACK triggering using DCI format 1_X. - The triggering in the DCI is separately RRC configured (from DCI formats 0_1 / 0_2).

## Moderator summary and proposals based on contributions

- On reference PDSCH for HARQ-ACK feedback timing determination

Regarding HARQ-ACK feedback timing determination, as agreed in RAN1#110, the PUCCH slot is determined based on the reference PDSCH and the indicated K1 value. There is one FFS details of reference PDSCH.

Agreement - When UE detects a DCI format 1_X scheduling a set of PDSCHs, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot , where is a number of slots and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format and is the last UL slot overlapping with the DL slot for the reference PDSCH reception for slot-based PUCCH or an UL slot overlapping with the end of the reference PDSCH reception in DL slot for sub-slot based PUCCH. - FFS details of reference PDSCH

Regarding reference PDSCH for HARQ-ACK feedback timing determination, 19 companies express their preferences on the reference PDSCH and the summary is listed below.

- Last PDSCH (supported by 14 companies):

  - Huawei, OPPO, CATT, Langbo, Intel, CAICT, xiaomi, Lenovo, FGI, LG, MTK, Ericsson, NTT DOCOMO, Qualcomm

- 1st PDSCH (supported by 2 companies):

  - ZTE, Nokia

- The lowest cell index (supported by 1 company):

  - Spreadtrum,

- The largest cell index (supported by 1 company):

  - Samsung,

- The latest staring PDSCH (supported by 1 company):

  - vivo,

Since using the last PDSCH as the reference PDSCH to determine HARQ-ACK feedback timing can give UE more processing time, moderator suggests going with last PDSCH among co-scheduled cells as the reference. Proposal 4-1 is provided for the first round of discussion.

- On Type-1 HARQ-ACK codebook

For Type-1 HARQ-ACK codebook, as mentioned by several companies, the determination of the Type-1 codebook is related to the design of TDRA indication in the multi-cell PDSCH scheduling DCI. If a Type-1B TDRA field is included in the multi-cell scheduling DCI where a TDRA table is defined for the co-scheduled cells with each row indicating multiple SLIVs for the multiple scheduled cells, then SLIV pruning and K1 set extension need to be considered.

Proposal 3-4 is provided to collect companies’ views on TDRA field design. For time being, moderator propose deferring this issue until the Type-1B TDRA indication with a TDRA table defined with each row indicating multiple SLIVs for the multiple scheduled cells is agreed or conclusion on TDRA field design is made.

- On Type-2 HARQ-ACK codebook

For Type 2 HARQ-ACK codebook, below agreement is made in previous RAN1 meeting:

Agreement - For Type-2 HARQ-ACK codebook, two sub-codebooks are generated with a first sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling a single cell and a second sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling more than one cell. - Separate DAI counting for DCI(s) with each scheduling a single cell and DCI(s) with each scheduling more than one cell. - FFS whether a DCI scheduling more than one cell is associated with the first sub-codebook or the second sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one. - Type-2 HARQ-ACK codebook is generated by concatenating the first sub-codebook and the second sub-codebook. - If at least one cell of the set of cells which can be co-scheduled by a DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling, - FFS: the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell; - Otherwise, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell is equal to N, where N is the maximum number of cells which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE. - HARQ-ACK information bits for co-scheduled PDSCHs by a DCI format 1_X is ordered based on serving cell indices associated with co-scheduled PDSCHs. - HARQ-ACK bundling across co-scheduled cells is not supported for multi-cell scheduling.

Regarding the first FFS in above agreement, whether a DCI scheduling more than one cell is associated with the first sub-codebook or the second sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one, companies’ views in RAN1#110bis meeting are summarized as below:

- A DCI scheduling more than one cell is associated with the first sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one.

  - Supported by ZTE, OPPO, CAICT, LG, Samsung,

- A DCI scheduling more than one cell is associated with the second sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one.

  - Supported by Huawei, vivo, CATT, Intel, Lenovo, NTT DOCOMO, Qualcomm, Nokia

From moderator’s point of view, this issue is not essential, and it is better to follow the legacy operation in Rel-17 above 52.6GHz. Furthermore, for simplicity, the type of sub-codebook should be based on the number of cells indicated by the DCI regardless of the actual number of PDSCH reception so that UE complexity is reduced.

Hence, moderator suggests associating the DCI with the second sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one. Proposal 4-2 is provided for discussion.

Regarding the second FFS in above agreement, if at least one cell of the set of cells which can be co-scheduled by a DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling, how to determine the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell, companies’ views in RAN1#110bis meeting are summarized as below:

- If at least one cell of the set of cells which can be co-scheduled by a DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling,

  - the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell is equal to the maximum number of TBs which can be co-scheduled.

    - Supported by Huawei, vivo, CATT, Intel, Lenovo, LG, Samsung,

  - the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell is equal to 2*N, where N is the maximum number of cells which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE.

    - Supported by ZTE, FGI, NTT DOCOMO, Qualcomm, Nokia

Determining the number of HARQ-ACK information bits per DCI format 1_X according to the maximum number of co-schedulable TBs among all the possible scheduling cell combinations can avoid any unnecessary overhead in HARQ-ACK sub-codebook generation and lead to least signaling overhead. At most 3 bits can be saved per DCI format 1_X when one DCI format 1_X can schedule maximum 4 cells and only one cell is configured with maximum 2 codewords without spatial bundling.

However, as pointed by some companies, if the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell is equal to the maximum number of TBs which can be co-scheduled, UE has to check all the scheduled cell combinations in same PUCCH group leading to the largest number of HARQ-ACK bits and uses the related HARQ-ACK bit number accordingly.

Hence, for simplicity, moderator suggests determining the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell is equal to 2*N, where N is the maximum number of cells which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE. Proposal 4-3 is provided for discussion.

For Type-2 HARQ-ACK codebook, there is a third open issue on DAI counting and last DCI format determination. In legacy single cell scheduling operation, DAI is associated with the scheduled cell. For multi-cell scheduling, one DCI schedules multiple serving cells. One of co-scheduled cells should be selected for determining DAI order.

On the other hand, for PUCCH resource determination in legacy single cell scheduling operation, the DCI formats are first indexed in an ascending order across serving cell indexes for a same PDCCH monitoring occasion and are then indexed in an ascending order across PDCCH monitoring occasion indexes. For multi-cell scheduling, similar to DAI counting, one of co-scheduled cells should be selected to order the detected DCI format 1_X.

For RAN1#110bis meeting, companies’ views are summarized as below:

- The reference PDSCH to determine DAI counter/last DCI format is the PDSCH with smallest serving cell index among the set of co-scheduled cells.

  - Supported by vivo, OPPO, CATT, Intel,

- The reference PDSCH to determine DAI counter/last DCI format is the PDSCH with largest serving cell index among the set of co-scheduled cells.

  - Supported by OPPO, Samsung,

- The PDSCH of the first cell in the table row of the indicated co-scheduled cells and/or its associated cell is used for the last DCI format determination (for PRI) and DAI counting.

  - Supported by Nokia,

- The reference PDSCH for HARQ-ACK feedback timing determination is used for DAI counting.

  - Supported by FGI

From moderator’s point of view, the simplest way is to define the reference PDSCH for determining DAI counting/last DCI format as the PDSCH with smallest serving cell index among the co-scheduled cells.

Hence, Proposal 4-4 is provided for discussion.

- On simultaneous configuration of multi-cell PDSCH scheduling and multi-PDSCH scheduling in a PUCCH group

This issue was extensively discussed in previous RAN1 meetings and RAN#97 meeting. However, no consensus has been reached so far. For RAN1#110bis meeting, companies’ views are summarized as below:

- On simultaneous configuration of multi-cell PDSCH scheduling and multi-PDSCH scheduling in same PUCCH group

  - Not support: Huawei, vivo, Ericsson, Lenovo, Google, Qualcomm, Nokia

  - Support: Intel, LG, NTT DOCOMO

For RAN1#110bis meeting, 7 companies [Huawei, vivo, Ericsson, Lenovo, Google, Qualcomm, Nokia] don’t support simultaneous configuration of multi-cell PDSCH scheduling and multi-PDSCH scheduling within a same PUCCH group. 3 companies [Intel, LG, NTT DOCOMO] support simultaneous configuration of multi-cell PDSCH scheduling and multi-PDSCH transmission within a same PUCCH group and propose maximum 2 HARQ-ACK sub-codebooks where HARQ-ACK information bits for multi-cell scheduling and HARQ-ACK information bits for multi-PDSCH scheduling are merged into one sub-codebook according to the larger number between the maximum number of PDSCHs in a row of TDRA table for multi-PDSCH scheduling and the maximum number of cells which can be co-scheduled by a DCI format 1_X. Since a DCI format 1_X can schedule maximum 4 PDSCHs while a DCI format 1_1 for multi-PDSCH scheduling can schedule maximum 8 PDSCHs, too many padding bits may be needed for multi-cell scheduling.

Based on above, to simplify HARQ-ACK codebook design and avoid the case where the number of sub-codebooks exceeds 2 or too many padding bits in the second sub-codebook, moderator suggests Rel-18 don’t support simultaneous configuration of multi-cell PDSCH scheduling and multi-PDSCH scheduling within a same PUCCH group.

Hence, Proposal 4-5 is provided for discussion.

- On support of HARQ-ACK codebook types

In RAN1#110 meeting, the supported HARQ-ACK codebook types have been extensively discussed and below proposal is captured in FL summary#6.

#### Proposal 4-4: - Updating below working assumption. Working Assumption - HARQ-ACK codebook types (Type-1, Rel-15 Type-2, Rel-16 Type-3, Rel-17 Type-3) are applicable when multi-cell PDSCH scheduling is configured. - Rel-16 Type-2 HARQ-ACK codebook is not applicable when multi-cell PDSCH scheduling is configured.

According to the conclusion from RAN#97 meeting, enhanced Type-2 HARQ-ACK codebook is not supported for multi-cell scheduling in Rel-18.

Conclusion: - Deprioritize any optimization for unlicensed spectrum operation for designing the multi-cell PUSCH/PDSCH scheduling in Rel-18. - Enhanced Type-2 HARQ-ACK codebook is not supported for the multi-cell PUSCH/PDSCH scheduling in Rel-18. - Type-1 HARQ-ACK codebook is supported only for the case where co-scheduled cells by a DCI format 1_X have same SCS/carrier type/duplex mode in Rel-18. - Additional restriction(s) can be discussed in RAN1 - Configuring more than one scheduling cell for DCI format 0_X/1_X for each scheduled cell is not supported for the multi-cell PUSCH/PDSCH scheduling in Rel-18.

Since Rel-16 Type-3 HARQ-ACK codebook and Rel-17 Type-3 HARQ-ACK codebook can be applied directly for multi-cell scheduling, moderator suggests confirming below working assumption and Proposal 4-6 is provided for discussion.

## 1st round of discussions

#### Proposal 4-1:

- For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_X, the reference PDSCH is the PDSCH ending last among the set of co-scheduled PDSCHs.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| MTK | Support |
| Qualcomm | We support the proposal. |
| Nokia, NSB | We can live with this. But maybe we would need to be more precise what ‘last’ means here, i.e. the PDSCH ending last… |
| Apple | Support the proposal with the understanding that the last PDSCH means “PDSCH ending last” |
| Langbo | Support the proposal if “the last PDSCH” means “the PDSCH ending last”. |
| Huawei, HiSilicon | Support |
| Intel | We support the proposal, with the clarification that it is the PDSCH with last ending symbol. |
| ZTE | The last PDSCH should clarified when the last PDSCH indicated by the DCI is actually not transmitted due to slot format collision. |
| Moderator | OK to revise it as “the PDSCH ending last”. |
| CMCC | Support |
| New H3C | Support |
| CATT | Share same understanding with Apple. |
| vivo | Ok with Nokia’s clarification. |
| LGE | OK |
| Samsung | Ordering of PDSCHs based on cell index is aligned with Rel-17 behavior and enables a unified solution for PUCCH slot determination, DAI definition, and last DCI definition. So, the PDSCH with the largest cell index should be used. For progress, OK to go with the majority if a benefit/reason for not having a unified treatment for the above parameters is explained. |
| Spreadtrum | OK with the proposal for progress. |
| Xiaomi | Support the Nokia’s update. |
| NTT DOCOMO | Support this proposal and also fine to clarify that the last PDSCH means “the PDSCH ending last”. |
| Ericsson | OK with the latest revision to clarify “PDSCH ending last” Samsung: Reference PDSCH is important for timing of PUCCH. How, for CB construction, based on indicated DAI in DCI, we associate DAI to co-scheduled PDSCHs, is a next problem. In solving that, I agree with your view that we should reuse the existing ordering. |
| ITRI | Support |
| Moderator2 | The proposal is updated as below: #### Proposal 4-1rev1: - For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_X, the reference PDSCH is the PDSCH ending last among the set of co-scheduled PDSCHs. |
| Apple | Support the updated proposal |
| Qualcomm | OK with the updated proposal. |
| Intel | We are fine with the updated proposal. |
| OPPO | OK with the principle. The proposal may need to tell a time unit to measure “ending last”, and hopefully there would never be two PDSCH with the same “ending” in that time unit. |
| New H3C | OK with the updated proposal. |
| NTT DOCOMO2 | We are fine with the updated proposal. |
| MTK | Support the updated proposal. |
| Moderator3 | @OPPO: Even there are two PDSCHs ending at same symbol, it does not impact on HARQ-ACK feedback timing determination. @Samsung: For HARQ-ACK timing determination, using the last PDSCH as the reference PDSCH to determine HARQ-ACK feedback timing can give UE more processing time. If using the last PDSCH for associating DAI, there will be problem if two PDSCHs ending at same symbol, then we have to associate the DAI with cell index. In that sense, the unified reference PDSCH can’t be achieved. |
| Moderator4 | Based on inputs, let’s take Proposal 4-1rev1 for discussion in 2nd round. |

#### Proposal 4-2:

- For Type-2 HARQ-ACK codebook, a DCI format 1_X scheduling more than one cell is associated with the second sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| MTK | Fine with the proposal |
| Qualcomm | We are not OK with the proposal. Not sure why we have to take a different approach from Rel-17 multi-slot PDSCH scheduling. |
| Nokia, NSB | Support |
| Apple | Fine with the proposal |
| Langbo | Support |
| Huawei, HiSilicon | Support |
| Intel | We support the proposal, i.e., same rule as defined in FR2-2 should be considered. |
| ZTE | It should be associated with the first sub-codebook to reduce the codebook size. The network and the UE both know the semi-static slot formats. There is no ambiguity on the codebook size between the UE and the network. We should ensure the minimum codebook size to reduce the resource utilization. |
| Moderator | @Qualcomm: This takes same approach as Rel-17. Please check it. |
| New H3C | Support |
| CATT | Support |
| vivo | Support |
| LGE | Not support We think that associating with the 1st sub-codebook is reasonable to reduce the UCI overhead/resource required for HARQ-ACK feedback. In other words, associating with the 2nd sub-codebook would cause unnecessary HARQ-ACK overhead/resource which is not desirable for the Type-2 CB case. |
| Samsung | Agree with QC that Rel-17 used the 1st sub-CB, and same solution can be re-used. |
| Xiaomi | We want to clarify if this issue exits. Our understanding on this issue is that the PDSCH transmissions on some of the co-scheduled cells need to be hold due to the collision with the semi-static TDD configuration. However, the UE does not expect tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated for the reference cell to indicate a symbol as uplink and to detect a DCI format scheduling a reception on the symbol on another cell as indicated in 38.213. Thus it is doubtful to us if this is valid issue for MC scheduling. |
| NTT DOCOMO | Support this proposal. |
| Ericsson | Not support because the proposal is a clarification of the agreement and is not needed. In the agreement, the 2nd CB corresponding to PDSCHs scheduled by DCI 1_X. Hence, the identifier is the scheduling DCI. Now, how many PDSCHs, on how many cells, for any reasons, that is scheduled by DCI 1_X does not matter. As long as PDSCHs are scheduled by DCI 1_X, would belong to second CB. |
| ITRI | Support |
| Moderator | @Companies which prefer 1st sub-codebook: Please kindly check below agreement I cited it from R17 B52.6GHz. I think we don’t need to repeat same issue in R18 MC because we really don’t have sufficient time as R17 B52.6GHz. To me, the most straightforward thing is to reuse R17 spec to save time. Agreement: For a PDSCH that is scheduled by multi-PDSCH scheduling DCI and is skipped due to collision with semi-static UL symbol(s), - For Type-1 HARQ-ACK codebook generation, the PDSCH is not considered and the HARQ-ACK bit corresponding to the PDSCH is not reported by UE. - Note: Rel-16 procedure can be reused to handle this case. - For Type-2 HARQ-ACK codebook generation, UE reports NACK for the PDSCH. - FFS on HARQ-ACK bit ordering - Note: Codebook generation in case time domain bundling is enabled can be separately discussed if time domain bundling is supported. |
| Qualcomm | We are OK with the Proposal (sorry mislead and made a wrong comment above – the proposal is consistent with Rel-17 multi-PDSCH scheduling). |
| Moderator4 | Based on R17 agreement, I hope all companies can accept this proposal. Let’s take Proposal 4-2 for discussion in 2nd round. |

#### Proposal 4-3:

- For Type-2 HARQ-ACK codebook, if at least one cell of a set of cells which can be co-scheduled by DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell of the set of cells is equal to 2*N, where N is the maximum number of cells which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| MTK | Fine with the proposal |
| Qualcomm | We support the proposal. |
| Nokia, NSB | Support |
| Apple | Support the proposal |
| Langbo | Support |
| Huawei, HiSilicon | We can live with it. |
| Intel | We do not support the proposal. It should be the maximum sum of number of TBs for co-scheduled PDSCHs by the DCI format 1_X, in order to reduce HARQ-ACK payload size. Above all, the payload size is quite important for HARQ-ACK codebook. We don’t think there is any difficulty to count the sum of number of configured TBs since it is purely based on semi-statically configured signaling. |
| ZTE | We support this proposal. It follows the legacy mechanism for Type-2 codebook. |
| Moderator | @Intel: Determining the number of HARQ-ACK information bits per DCI format 1_X according to the maximum number of co-schedulable TBs among all the possible scheduling cell combinations can save at most 3 bits per DCI format 1_X when one DCI format 1_X can schedule maximum 4 cells and only one cell is configured with maximum 2 codewords without spatial bundling. But, UE has to check all the scheduled cell combinations in same PUCCH group leading to the largest number of HARQ-ACK bits and uses the related HARQ-ACK bit number accordingly. Hence, it is simpler to determine the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell is equal to 2*N, where N is the maximum number of cells which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE. |
| New H3C | Support |
| CATT | In order to save the HARQ-ACK bits of Type-2 Codebook, the number of HARQ-ACK bits can be equal to the maximum number of TBs which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE. |
| vivo | We share the same view with intel. The proposal will lead to a lot of redundant HARQ-ACK bits, which is detrimental to the reliability of the PUCCH. |
| LGE | Not support We fundamentally disagree with the proposal since we don’t understand why Type-2 CB need to have such unnecessary UCI overhead, even though the Type-2 CB has originally been designed to reduce HARQ-ACK overhead (by using DCI overhead for C/T-DAI signalling) so far compared to Type-1 CB. We don’t see the checking on the combinations of co-scheduled cells by multi-cell DCI would be complexity since it is already being done in Rel-17 for multi-TTI scheduling, i.e., checking all the TDRA rows configured in all the cells configured with the multi-TTI scheduling within PUCCH group, to find the maximum number of SLIVs for determining the HARQ-ACK bits for 2nd sub-CB. |
| Samsung | Prefer to use maximum number of TBs which can be co-scheduled. There is no reason to unnecessarily increase HARQ-ACK payload, reduce coverage, and there is no complexity for using the maximum number of TBs that can be co-scheduled. Also, we are not sure of the intention for the comment from FL in the discussion above: “UE has to check all the scheduled cell combinations in same PUCCH group leading to the largest number of HARQ-ACK bits and uses the related HARQ-ACK bit number accordingly”. That is a most trivial matter and is done semi-statically. |
| Xiaomi | Our preference is the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell is equal to the actually number of co-scheduled TBs. The spirit of the type 2 codebook is to determine the number of HARQ-ACK bits based on the actual scheduled TBs. We think the principle should be persisted here |
| NTT DOCOMO | Support this proposal. |
| Ericsson | Not support. We share same view as other companies that the size of CB, can be the maximum number of IBs. But it doesn’t mean that is it always the maximum number of TBs (if the size is fixed, what is the point of using Type-2). |
| Moderator2 | For a UE, when there are multiple sets of cells configured for multi-cell scheduling, e.g., set 1 including {CC1, CC2}, set 2 including {CC3, CC4, CC5, CC6} and set 3 including {CC7, CC8, CC9, CC10, CC11, CC12}, cells in each set are configured independently with 1 or 2 TBs per cell with or without spatial bundling, then it is complicated for UE to determine a maximum co-schedulable TBs for all sets. Why not just set 2*N although this is sub-optimal? As mentioned above, max 3 bits are saved per DCI format 1-X. |
| OPPO | We do see there is some better solution than 2*N. HARQ-ACK codebook size highly impacts PUCCH reliability, so we prefer to go with Alt 1: the number of HARQ-ACK information bits for each DCI format 1_X is equal to the maximum number of TBs which can be co-scheduled. On the other hand, if the concern is UE has to check all the possible cell combinations, we can simplify the above method as Alt 3: Alt 3: The number of HARQ-ACK bits for each DCI format 1_X is the sum of largest N values in {Ci}, Ci is the maximum number of codewords for cell i in the set of cells. For example, assuming the maximum number of codewords for cell 1/2/3 is 1 while cell 4 is configured with maximum 2 codewords. The cell combination that can be co-scheduled by DCI format 1_X are {cell 1+cell 2+cell 3}, {cell 3+cell 4}, then if Alt 2 (2*N) is adopted, the number of HARQ-ACK bits for each DCI format 1_X is 2*3=6 bits, while if Alt 1 is used, the number of HARQ-ACK bits is max{1+1+1, 1+2}=3; if the new Alt 3 is applied, the number of HARQ-ACK bits is 2+1+1=4, and UE does not need to check all the possible cell combinations. |
| LGE | @FL: We really don’t see complexity issue here since similar (or even worse) situations were already with R17 multi-TTI scheduling. For a UE, when there are multiple cells configured with multi-TTI scheduling, the maximum number of SLIVs configured in TDRA table would be different across cells, and 1/2-TB PDSCH configuration would also be different across cells, and moreover, the configuration of time-domain bundling and its bundling size would also be different across cells. Even in this case, we have specified to determine HARQ-ACK size per multi-TTI DCI as the max number of HARQ-ACK bits considering all combinations across all cells. It is quite fundamental since Type-2 CB has been designed to minimize HARQ-ACK overhead so far (by sacrificing DCI overhead for DAI signalling). |
| Moderator3 | @LG: Thanks for the clarification. Ok, let me update the proposal to check whether companies can accept it? #### Proposal 4-3rev1: - For Type-2 HARQ-ACK codebook, if at least one cell of a set of cells which can be co-scheduled by DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell of the set of cells is equal to M, where M is the maximum number of TBs which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE. |
| Moderator4 | Based on inputs, let’s take Proposal 4-3rev1 for discussion in 2nd round. |

#### Proposal 4-4:

- For Type-2 HARQ-ACK codebook, for a set of cells which is co-scheduled by a DCI format 1_X, the reference PDSCH to determine DAI counting/last DCI format for the DCI format 1_X is the PDSCH with smallest serving cell index among the set of co-scheduled cells.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| MTK | We prefer to also use the last PDSCH as reference PDSCH. Open to consider this proposal if clear majority thinks so. |
| Qualcomm | OK with the proposal. |
| Nokia, NSB | We could be OK with this |
| Apple | Fine to support the proposal |
| Langbo | We can live with this proposal |
| Huawei, HiSilicon | Ok |
| Intel | We are fine to use smallest serving cell index for DAI counting. For last PDCCH, we think it can be discussed with low priority. The current spec still works without any enhancement, considering gNB typically sets the same PRI for all PDCCHs in the same PDCCH MO (that’s why we didn’t introduce any enhancement for last DCI format for multiple PDCCHs for the same serving cell in the same PDCCH MO in Rel-16 MR-DC). If companies really want to introduce a rule for last PDCCH, we prefer to use the last PDSCH. Besides, in our understanding, if the intention to introduce enhancement for last PDCCH is for PRI determination, it seems PRI determination is not only for type-2 codebook but also for type-1 codebook? |
| ZTE | We support this proposal. |
| New H3C | Support |
| CATT | Support |
| vivo | Support the part for determining the DAI counting. As intel pointed out, if the reference PDSCH of the last DCI is to determine PUCCH, then the proposal should be applicable to other CB type, e.g., type-1. We suggest to have a separate proposal for the PUCCH resource determination. #### Proposal 4-4-1: - For Type-2 HARQ-ACK codebook, for a set of cells which is co-scheduled by a DCI format 1_X, the reference PDSCH to determine DAI counting/last DCI format for the DCI format 1_X is the PDSCH with smallest serving cell index among the set of co-scheduled cells. #### Proposal 4-4-2: - For Type-2 HARQ-ACK codebook, for a set of cells which is co-scheduled by a DCI format 1_X, the reference PDSCH to determine DAI counting/last DCI format for PUCCH determination of the DCI format 1_X is the PDSCH with smallest serving cell index among the set of co-scheduled cells. |
| LGE | Support in principle, except for one clarification. We may need to consider the case where the smallest scheduled cell index is the same for more than one DCI 1_X. |
| Samsung | Agree in principle that ordering based on cell index should be used. We suggest to use largest cell index both for this proposal and for FL P4-1 – please see comment to FL P4-1. |
| NTT DOCOMO | We support this proposal. Especially when there are more than one PDSCHs which ends at the same timing, how to determine the reference PDSCH for DAI counting/last DCI format determination needs to be clarified. |
| Ericsson | Not support (but agree with intention. Please see below): Agree with vivo that we have to separate DAI and last DCI. For DAI: - We agree with vivo’s modification. For last DCI: - We should use “the PDSCH ending last”. In fact, the same reference PDSCH that is used for timing of PUCCH. Otherwise, it is unnecessary complicated. |
| Moderator2 | Thanks vivo/Ericsson for the good comments. Now the proposal is updated as below: #### Proposal 4-4rev1: - For Type-2 HARQ-ACK codebook, for a set of cells which is co-scheduled by a DCI format 1_X, the reference PDSCH to determine DAI counting is the PDSCH with smallest serving cell index among the set of co-scheduled cells. - For a set of cells which is co-scheduled by a DCI format 1_X, the reference PDSCH to determine last DCI format for PUCCH determination of the DCI format 1_X is the PDSCH ending last among the set of co-scheduled cells. |
| New H3C | OK with the updated proposal. |
| MTK | OK with the updated proposal. |
| LGE | On the updated proposal 4-4rev1 in above, we are fine with the 1st bullet. But, we are a bit confused with the 2nd bullet, due to following question point. How the last DCI is determined in case where multi-cell DCI(s) and single-cell DCI(s) are exist together? |
| Moderator3 | @LG: Based on P4-4rev1, assuming PDSCH 1 on CC1 is the reference PDSCH for determining last DCI of DCI 1_X, if a single cell DCI schedules PDSCH 2 on CC2, and CC1<CC2, then 1_X is the last DCI compared with the single cell DCI. The legacy ordering is reused here. |
| Qualcomm | We support the first bullet. This seems already stable. Regarding the second bullet, the current spec is clear to begin with, unless a DCI schedules PDSCHs on multiple cells. In case a DCI schedules PDSCHs on multiple cells, the following “serving cell index” for ordering becomes unclear, since this serving cell index is the cell index where PDSCH is scheduled in the current spec. TS38.213: For a PUCCH transmission with HARQ-ACK information, a UE determines a PUCCH resource on the cell of the PUCCH transmission, as described in clause 9.A, after determining a set of PUCCH resources for OUCI HARQ-ACK information bits, as described in clause 9.2.1. The PUCCH resource determination is based on a PUCCH resource indicator field [5, TS 38.212], if present, in a last DCI format, excluding the SPS activation DCI, among the DCI formats that have a value of a PDSCH-to-HARQ_feedback timing indicator field, if present, or a value of dl-DataToUL-ACK, or dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-DCI-1-2-r17, or dl-DataToUL-ACK-MulticastDciFormat4_1, indicating a same slot for the PUCCH transmission, that the UE detects and for which the UE transmits corresponding HARQ-ACK information in the PUCCH. For PUCCH resource determination, detected DCI formats are first indexed in an ascending order across serving cells indexes for a same PDCCH monitoring occasion and are then indexed in an ascending order across PDCCH monitoring occasion indexes. For indexing DCI formats within a serving cell for a same PDCCH monitoring occasion, if the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs on an active DL BWP of a serving cell, and with ackNackFeedbackMode = joint for the active UL BWP, detected DCI formats from PDCCH receptions in the first CORESETs are indexed prior to detected DCI formats from PDCCH receptions in the second CORESETs. In case a DCI schedules PDSCHs on multiple cells, we need to pick up one of the cells to match with the above text. We are open to either smallest cell index or largest cell index amongst the scheduled cells, and hence we are fine with the moderator’s original proposal. We are fine to split into two bullets but do not quite understand why PDSCH ending timing is coming up as an issue. It was not in the above rules for determining the last DCI. |
| Moderator4 | @All: Based on inputs and e-mail discussion, the proposal is updated as below. #### Proposal 4-4rev2: - For Type-2 HARQ-ACK codebook, for a set of cells which is co-scheduled by a DCI format 1_X, the reference PDSCH to determine DAI counting is the PDSCH with smallest serving cell index among the set of co-scheduled cells. - For a set of cells which is co-scheduled by a DCI format 1_X, the PDSCH with the smallest serving cell index among the set of co-scheduled cells is used to determine last DCI format for PUCCH determination among DCI formats within a same PDCCH MO. Please continue the discussion in 2nd round. |

#### Proposal 4-5:

- UE does not expect to be configured both multi-PDSCH scheduling and multi-cell PDSCH scheduling on the same or different cells within a same PUCCH group.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| MTK | Support |
| Qualcomm | We prefer to keep this open. |
| Nokia, NSB | Support |
| Apple | Support the proposal |
| Langbo | Support |
| Huawei, HiSilicon | Support |
| Intel | We do not support the proposal. Given that multi-slot PDCCH monitoring and multi-PDSCH scheduling were defined as basic features for FR2-2, where one PDCCH can be used to schedule more than one PDSCHs in a cell, if only single-slot PDSCH is configured, most of the slots are not used for PDSCH transmission, which would degrade the spectrum efficiency substantially |
| ZTE | We think they can be configured simultaneously as long as the gNB can ensure the HARQ-ACK information bits for multi-cell scheduling and the HARQ-ACK information bits for multi-PDSCH scheduling are not multiplexed in the same PUCCH or PUSCH. |
| Moderator | @Intel @ZTE: Actually, I agree with you on the use case. But we don’t have enough TU. As you know, only 1TU is left for whole Q4. We have to perform down-scoping. |
| New H3C | Support |
| CATT | Support |
| vivo | Support. |
| LGE | We still think simultaneous configuration of the above two features needs to be supported to avoid scheduling restriction in FR1+FR2 CA case since the multi-TTI scheduling is basic feature in FR2-2 operation. Regarding the concern on Type-2 HARQ-ACK codebook, simply the DCIs scheduling multiple PDSCHs (regardless whether the DCI schedules multiple cells or multiple slots) are merged into a same sub-codebook. But, considering the progress, we can live with the proposal if we are the only company objecting such way of configuration restriction. |
| Samsung | Support |
| Xiaomi | Support |
| NTT DOCOMO | We share the similar view with QC, Intel and ZTE to keep it open. In our view, the only thing needs to be discussed to support multi-slot PDSCH simultaneously is how to determine the number of HARQ-ACK bits for type-2 codebook and it does not complex the discussion. Therefore, to efficient scheduling of wide frequency band, we prefer to support multi-slot PDSCH scheduling and multi-cell scheduling simultaneously. |
| Ericsson | We can be OK to support simultaneous configuration as LG suggest. We can disallow then simultaneous transmission of HARQ-ACK feedback of both in the same CB. Please see our corresponding proposal in our contribution. |
| Moderator2 | @LGE @QC @ZTE If HARQ-ACK information bits for multi-PDSCH scheduling and multi-cell scheduling are merged into a same sub-codebook, I am afraid too many padding bits have to be inserted since a DCI format 1_X can schedule maximum 4 PDSCHs while a DCI format 1_1 for multi-PDSCH scheduling can schedule maximum 8 PDSCHs. If it works, why simultaneous CBG configuration and multi-PDSCH scheduling can’t be supported? Seems max number of CBGs per TB is 8 equals to max number of PDSCHs scheduled by one DCI. Based on above, to simplify HARQ-ACK codebook design and avoid the case where the number of sub-codebooks exceeds 2 or too many padding bits in the second sub-codebook, moderator suggests Rel-18 don’t support simultaneous configuration of multi-cell PDSCH scheduling and multi-PDSCH scheduling within a same PUCCH group. |
| OPPO | Support. |
| New H3C | Support |
| Moderator3 | @LGE @QC @ZTE Hope you are OK with the proposal. If you have concern please provide it in 2nd round. |

#### Proposal 4-6:

- Confirm below working assumption:

Working Assumption

  - HARQ-ACK codebook types (Type-1, Rel-15 Type-2, Rel-16 Type-3, Rel-17 Type-3) are applicable when multi-cell PDSCH scheduling is configured.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| MTK | Generally fine with the proposal |
| Qualcomm | We think no need to hurry to confirm this. |
| Nokia, NSB | Support |
| Apple | Support the proposal |
| Langbo | Support |
| Huawei, HiSilicon | Support but also Ok to wait. WA can be automatically confirmed in the end if no issue is found. |
| Intel | We are fine with the proposal. |
| ZTE | Support. |
| CMCC | We are fine with the proposal. |
| New H3C | Support |
| CATT | Support |
| vivo | Support. |
| LGE | Support |
| Samsung | Support |
| Xiaomi | Support |
| NTT DOCOMO | Support |
| Ericsson | Support |
| ITRI | Support |
| OPPO | Support. |
| Moderator | Since no objection, I list it in Section 7 for e-mail approval. |

## 2nd round of discussions

#### Proposal 4-1rev1:

- For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_X, the reference PDSCH is the PDSCH ending last among the set of co-scheduled PDSCHs.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| LGE | OK |
| Qualcomm | We support this |
| Langbo | Support |
| Nokia/NSB | Support |
| NTT DOCOMO | Support. |
| Xiaomi | Clarification on the “ending last” need to be clarified. Is it the one indicated or actually transmitted considering the unlicensed spectrum operation. Suggest to update as: - For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_X, the reference PDSCH is the PDSCH ending last as indicated in the DCI format 1_X among the set of co-scheduled PDSCHs. |
| Apple | Support |
| ZTE | Support |
| MTK | Support |
| Huawei2 | Support |
| Intel | We are fine with the proposal. |
| CATT | We support the proposal. |
| Samsung3 | Thanks to Moderator and Ericsson for the comments in the previous round. We think PUCCH processing timeline should not be an issue, since K1 value is configured/indicated by the gNB, so suitable K1 value can be provided that gives the UE sufficient time. Therefore, using the largest cell index for the reference PDSCH will not cause any issues. However, for progress, we can be OK with the FL proposal, considering that ordering based on cell index can be still used for Proposal 4-3. Also, we would like to add a Note to clarify the case when the reference PDSCH collides with semi-static UL symbol and therefore is not received. #### Proposal 4-1rev1: - For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_X, the reference PDSCH is the PDSCH ending last among the set of co-scheduled PDSCHs. - Note: a ‘PDSCH ending last’ that collides with semi-static UL symbol can be the reference PDSCH |
| Nokia/NSB | We support the note by Samusung if such clarification is needed. Clearly Sec. 11.1 is handled only later on – and should not affect the HARQ timing. |
| Moderator | @Samsung @Nokia: As suggested by xiaomi, with “as indicated in the DCI format 1_X”, it can address the issue mentioned by Samsung. #### Proposal 4-1rev2: - For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_X, the reference PDSCH is the PDSCH ending last as indicated in the DCI format 1_X among the set of co-scheduled PDSCHs. Proposal 4-1rev2 is also discussed in e-mail. You are welcome to raise your concern via e-mail. |
| New H3C | Support updated proposal |

#### Proposal 4-2:

- For Type-2 HARQ-ACK codebook, a DCI format 1_X scheduling more than one cell is associated with the second sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| LGE | Although we still think associating with the 1st sub-CB is preferred/reasonable to reduce (unnecessary) HARQ-ACK overhead, we can live with the proposal if we are the only company objecting to it. |
| Qualcomm | We support this |
| Langbo | Support |
| Nokia/NSB | Support |
| NTT DOCOMO | Support. |
| Xiaomi | Support |
| Apple | Support |
| ITRI | Support |
| ZTE | We think obviously the first sub-codebook is a better choice, which can reduce HARQ-ACK overhead. It is noted that, in proposal 4-3rev1, the solution with low HARQ-ACK overhead is suggested. But we can live with this proposal for progress. |
| MTK | Support |
| Huawei2 | Support |
| Intel | We are fine with the proposal. |
| CATT | We support the proposal. |
| Samsung3 | Thanks Moderator for sharing the RAN1 agreement. We still think using the 1st sub-CB saves UCI overhead, but for progress, we are OK with the proposal. @ Ericsson: would like to clarify that per agreement in previous meeting, selection of the 1st or 2nd sub-CB is based on the number of scheduled PDSCHs. So, if a DCI format 0_X/1_X schedules only one PDSCH, the UE generates only one HARQ-ACK bit and is included in the 1st sub-CB. |
| Moderator | Thank you all for the support. Below please kindly check the addition which intends to address Speadtrum’s concern. Proposal 4-2rev1: - For Type-2 HARQ-ACK codebook, a DCI format 1_X scheduling more than one cell is associated with the second sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one. - If a UE is scheduled by a DCI format 1_X to receive PDSCH over multiple cells, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicates that, for a cell from the multiple cells, at least one symbol from a set of symbols where the UE is scheduled PDSCH reception in one the cell is an uplink symbol, the UE does not receive the PDSCH in the cell. - If a UE is scheduled by a DCI format 0_X to transmit PUSCH over multiple cells, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicates that, for a cell from the multiple cells, at least one symbol from a set of symbols where the UE is scheduled PUSCH transmission in one the cell is a downlink symbol, the UE does not transmit the PUSCH in the cell. Proposal 4-2rev1 is also discussed in e-mail. You are welcome to raise your concern via e-mail. |
| New H3C | Support updated proposal |

#### Proposal 4-3rev1:

- For Type-2 HARQ-ACK codebook, if at least one cell of a set of cells which can be co-scheduled by DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell of the set of cells is equal to M, where M is the maximum number of TBs which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| LGE | Support |
| Qualcomm | Although we still prefer 2*N, we can accept the proposal. |
| Langbo | Support |
| Nokia/NSB | Do not support: We still prefer 2*N to follow the operation for SC-DCI scheduling since Rel-15. We think this to be some unnecessary complication here (and may limit flexibility, e.g. in case one supports more than one ‘MC-DCI configuration’ within a PUCCH group. It seems most companies were fine with 2*N – so unclear why to adopt this one. |
| NTT DOCOMO | We understand how the number of HARQ-ACK information bits can be further optimized based on this proposal, thus we support this proposal. |
| Moderator | @Nokia: I don’t think this issue is essential. All that we need to do is to define a simple rule. Max TB number can save bits without extra complexity. Let’s try this method for time being. |
| Xiaomi | Support |
| LGE2 | @Nokia: We don’t think 2*N is to follow the logic of SC-DCI scheduling since if at least one cell is configured with 2-TB in the SC-DCI case, the HARQ-ACK size per DCI is determined as 2 which is the maximum number of TBs that can be scheduled by one DCI. Applying the same logic to this MC-DCI case, if at least one of the cells configured for co-scheduling is configured with 2-TB, the HARQ-ACK size per DCI is determined as the maximum number of TBs that can be scheduled by one MC-DCI, not as 2*N which is just N * the maximum number of TBs per cell and would exceed possible maximum number of scheduled TBs. The above is fundamental/basic principle of Type-2 CB designed to reduce HARQ-ACK overhead compared to Type-1 CB, and this principle is not new and has already been applied so far, for example, for CBG-based PDSCH and R17 multi-TTI scheduling. Besides, we don’t see complexity issue with the above since similar (or even more complicated) situations were already with R17 multi-TTI scheduling. Please see our relevant comments provided in the 1st round. |
| Apple | Our preference is 2*N, but if majority prefers the latest verion, we can accept this for sake of progress |
| ITRI | Support |
| ZTE | We support this proposal because it can reduce the HARQ-ACK overhead. |
| MTK | Our preference is also 2*N, but can accept this for sake of progress |
| Huawei2 | Good to see this and support. |
| Nokia/NSB | @LGE & others – explanation why we think it is not good to have ‘N’ bits here. If we agree to the first bullet of Proposal 3-5rev1 as well (i.e. can configure more than 4 cells, but only up to 4 are scheduled) then I will be able to survive with a single MC-DCI configuration. But if we limit that to 4 cells only, you actually may need 2 ‘configured MC-DCIs’ to be able to schedule the up to 8 cells with the MC-DCI operation. If then, the ‘number of bits’ for the multi-cell DCI is not aligned for DAI operation of MC-DCI here between the 2 MC-DCIs (scheduling e.g. cells #1…4 and cells#5…8), we will run into problems. So it may be OK (based on our understanding) if we agree also the 1st bullet of Proposal 3-5rev1 – but otherwise, this will create quite some issues here! |
| Intel | We are fine with the proposal. |
| CATT | We support the proposal. |
| LGE | @Nokia: We think even in case with the first bullet of Proposal 3-5rev1, the gNB could configure two MC-DCIs in a same PUCCH group according to situation or some reason. Anyway, in case with two MC-DCIs, still the same principle (i.e., the maximum number of TBs that can be scheduled by one MC-DCI) can be applied for the PUCCH group. For example, if MC-DCI #1 can schedule cell1/2/3/4 with max 1/2/2/1 TBs and MC-DCI #2 can schedule cell 5/6/7/8 with max 1/2/1/1 TBs, then “the maximum number of TBs that can be scheduled by one MC-DCI” would be 6 (= max{6, 5}). Note that similar cases were already with Rel-17 multi-TTI scheduling. When there are multiple cells configured with multi-TTI scheduling for a UE, within a same PUCCH group, the maximum number of SLIVs configured in TDRA table would be different across cells, and 1/2-TB PDSCH configuration would also be different across cells, and moreover, the configuration of time-domain bundling and its bundling size would also be different across cells. Even in this case, we have specified to determine HARQ-ACK size per multi-TTI DCI as the max number of HARQ-ACK bits considering all combinations within a same PUCCH group. |
| Samsung3 | We support the revised proposal. |
| Nokia/NSB | @LG: I agree this can be done, but then we have dependencies here based on more configuration. But there is then some ‘cross-multi-configuration’ aspect that needs to be taken into account. Having 2*N would simplify that based on our understanding. |
| LGE | @Nokia: Checking multi-configurations across multiple cells is already required to determine HARQ-ACK bits for multi-TTI scheduling in Rel-17, such as TDRA table (number of SLIVs) configuration, 1/2-TB PDSCH configuration, spatial bundling configuration, time-bundling on/off configuration, and time-domain bundling unit configuration. Rather, less number of configurations can be checked in case of multi-cell scheduling, for example, co-scheduled cell configuration, 1/2-TB PDSCH configuration, and spatial bundling configuration (without checking time-domain bundling, TDRA table). If you meant cross-CC configuration such as co-scheduled cell configuration, UE basically needs to consider/check it already in order to read MC-DCI fields, decode PDCCH SS, count BD/CCE across multiple cells, and so on. (not only for determining HARQ-ACK bits) |
| New H3C | Support updated proposal |
| Moderator4 | So far, the discussion on this proposal is also in e-mail. You are welcome to comment via e-mail. |

#### Proposal 4-4rev2:

- For Type-2 HARQ-ACK codebook, for a set of cells which is co-scheduled by a DCI format 1_X, the reference PDSCH to determine DAI counting is the PDSCH with smallest serving cell index among the set of co-scheduled cells.

- For a set of cells which is co-scheduled by a DCI format 1_X, the PDSCH with the smallest serving cell index among the set of co-scheduled cells is used to determine last DCI format for PUCCH determination among DCI formats within a same PDCCH MO.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| LGE | Support |
| Qualcomm | We support this |
| Langbo | Support |
| Nokia/NSB | Support |
| NTT DOCOMO | Support. |
| Apple | Support |
| ITRI | Support |
| ZTE | We support this proposal. |
| MTK | Support |
| Huawei2 | Support |
| Intel | We are fine with the first bullet. For 2nd bullet, we suggest FFS for now, because currently we don’t see any solution on the table that can uniquely identify one DCI as last DCI, e.g., as mentioned by LGE in the email reflector, there can be multiple PDCCHs scheduling PDSCHs with same smallest or largest serving cell index, or there can be multiple PDCCH scheduling PDSCHs with same ending symbol mentioned by CATT. Then, we need additional rule to find one DCI from these DCIs as a unique last DCI and apparently the scheme for last DCI is different from timing reference or DAI ordering. There is no unified solution for these 3 issues. In addition, we’d like to also point out that, the system anyway can work without any special handling for last DCI, because gNB can indicate same PRI for all PDCCHs in the same MO, which is aligned with the assumption for R16 MC-DC. Actually, in R16 RAN1 did not agree to use PDSCH starting symbol for last DCI determination, RAN1 only agreed to use PDSCH starting symbol for DAI ordering, because companies agreed that gNB can indicate same PRI for multiple PDCCHs in same MO scheduling PDSCHs in the same serving cell. Therefore, we suggest deprioritizing the discussion for last DCI for now. We’d better spend time on more critical issue considering very limited time for this WI. |
| CATT | We support the proposal. |
| Moderator | @Intel: For the second bullet, I understand your concern on unified solution. For HARQ-ACK timing, we have to use the PDSCH ending last in the co-scheduled PDSCHs for determining the HARQ-ACK feedback timing so that UE can have sufficient processing time. Secondly, since there may be more than one PDSCH with same ending position on more than one cell, we have to select the reference PDSCH for DAI ordering and last DCI format determination based on serving cell index, right? Hence, we can’t find a unified solution to determine HARQ-ACK feedback timing and DAI ordering or last DCI determination. Thirdly, as clarified in previous meeting, TS38.213 may not capture a terminology as “reference PDSCH”. Everything can be clearly captured without “reference PDSCH”. So unified reference PDSCH seems not quite necessary. With this clarification, can you live with the second bullet of Proposal 4-4rev2 for sake of progress? |
| Samsung3 | Support Regarding Intel’s comment we have different understanding. For the case where multiple PDCCHs scheduling PDSCHs with same smallest or largest serving cell index, similar issue exists in Rel-16 for single cell scheduling, there is no difference here. We should focus on the new issue brought up by multi-cell scheduling. Besides this case, using the largest/smallest cell index can be the unified solution which is our original intention for Proposal 4-1. Regarding gNB implementation solution, it brings unnecessary scheduling restriction on gNB and can result in additional complexity for gNB. We think this is a new issue because of multi-cell scheduling and should be addressed. |
| Moderator | Proposal 4-4rev2 is discussed via e-mail. You are welcome to raise you concern via e-mail. |
| New H3C | Support updated proposal |

#### Proposal 4-5:

- UE does not expect to be configured both multi-PDSCH scheduling and multi-cell PDSCH scheduling on the same or different cells within a same PUCCH group.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| LGE | We can live with the proposal for the progress. |
| Qualcomm | We can live with the proposal for the progress. |
| Langbo | Support |
| Nokia/NSB | Support |
| NTT DOCOMO | We would like to repeat our comment in the previous round. In our view, multi-slot PDSCH scheduling and multi-cell PDSCH scheduling can be supported at the same time without any complex discussion. We can maintain the 2-sub-codebook framework for Type-2 HARQ-ACK codebook, i.e., first sub-codebook represents HARQ-ACK for single PDSCH scheduling and second sub-codebook represents HARQ-ACK for the scheduling of more than one PDSCH. Therefore, to efficient scheduling of wide frequency band, we prefer to support multi-slot PDSCH scheduling and multi-cell scheduling simultaneously. |
| Moderator2 | @NTT DOCOMO: I think everyone knows the benefit and complexity. We really don’t need to repeat same discussion as in Rel-17 B52.6. We have so limited TU. |
| Apple | We are fine to support |
| ITRI | Support |
| ZTE | We think it should not be excluded as commented before. Given the current situation and the limited TU, we can live with this proposal for the progress. |
| MTK | Support |
| Huawei2 | Support |
| Intel | Share similar view as NTT DOCOMO. As mentioned previously, we are not convinced why these two cannot be configured simultaneously. We do not think there is much spec impact even for HARQ-ACK CB generation. |
| CATT | We support the proposal. |
| Moderator3 | @NTT DOCOMO @Intel: I understand your concern and agree with the use case of supporting both multi-cell scheduling and multi-PDSCH scheduling is quite useful. However, considering the limited TU we have and only two companies prefer the feature, can you live with it for sake of progress? |
| Samsung3 | Support |
| New H3C | Support updated proposal |
| Moderator4 | So far, the discussion on this proposal is also in e-mail. You are welcome to comment via e-mail. |

# FL observations

#### Proposal 4-1rev2:

- For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_X, the reference PDSCH is the PDSCH ending last as indicated in the DCI format 1_X among the set of co-scheduled PDSCHs.

#### Proposal 4-4rev3:

- For Type-2 HARQ-ACK codebook, for a set of cells which is co-scheduled by a DCI format 1_X, the reference PDSCH to determine DAI counting is the PDSCH with smallest serving cell index among the set of co-scheduled cells.

- For a set of cells which is co-scheduled by a DCI format 1_X, the PDSCH with the smallest serving cell index among the set of co-scheduled cells is used to determine last DCI format for PUCCH determination among DCI formats within a same PDCCH MO.

  - FFS: which PDSCH is used to determine the last DCI format in case when both DCI format 1_X and other DCI format 1_0/1_1/2_1/1_X are received in a same PDCCH monitoring occasion on a same scheduling cell for scheduling PDSCHs on same scheduled cell

#### Proposal 3-5rev7:

·         For a set of cells which is configured for multi-cell scheduling, RAN1 specification supports up to 4 cells within the set of cells.

o    FFS: The maximum number of cells within the set of cells is reported as a UE capability.

o    A DCI format 0_X/1_X can schedule PUSCH(s)/PDSCH(s) on a combination of co-scheduled cells among the set of cells.

·         For multi-cell scheduling, the co-scheduled cells are indicated by an indicator in DCI format 0_X/1_X which points to one row of a table defining combinations of co-scheduled cells.

- The table is configured by RRC signaling.

#### Proposal 2-6rev1:

- For monitoring PDCCH candidates for a set of cells which is configured for multi-cell scheduling, the n_CI in the search space equation is determined by a value configured for the set of cells.

- The UE can be configured one or multiple sets of cells which are configured for multi-cell scheduling.

- When multiple sets of cells are configured for multi-cell scheduling, separate n_CI values are configured for different sets of cells.

- When multiple sets of cells are configured for multi-cell scheduling, a cell in one set of cells can’t be included in another set of cells.

#### Proposal 2-8rev2:

- For a set of cells which can be scheduled by DCI format 0_X/1_X, the payload size of DCI format 0_X/1_X is derived by UE based on RRC configuration of co-scheduled cell combinations within the set of cells.

  - The payload size 0_X is the same for all the co-scheduled cell combinations.

  - The payload size 1_X is the same for all the co-scheduled cell combinations.

# References

- R1-2208426	Discussion on multi-cell scheduling with a single DCI	Huawei, HiSilicon

- R1-2208486	Discussion on Multi-cell PUSCH/PDSCH scheduling with a single DCI	ZTE

- R1-2208564	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	Spreadtrum Communications

- R1-2208658	Discussion on multi-cell scheduling	vivo

- R1-2208779	Discussion on multi-cell scheduling with a single DCI	China Telecom

- R1-2208860	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	OPPO

- R1-2208991	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	CATT

- R1-2208997	Discussions on multi-cell PUSCH/PDSCH scheduling with a single DCI	Langbo

- R1-2209020	Consideration on multi-cell PUSCH/PDSCH scheduling with a single DCI	Fujitsu

- R1-2209067	Discussions on multi-cell scheduling with a single DCI	Intel Corporation

- R1-2209239	Discussions on multi-cell PUSCH/PDSCH scheduling with a single DCI	CAICT

- R1-2209299	Discussion on the design of multi-cell scheduling with a single DCI	xiaomi

- R1-2209305	On multi-cell scheduling via a single DCI	Lenovo

- R1-2209352	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	CMCC

- R1-2209416	Discussion on Multicarrier scheduling with a single DCI	FGI

- R1-2209426	Discussion on Multi-cell PXSCH scheduling with a single DCI	NEC

- R1-2209454	Discussion on Multi-cell PUSCH/PDSCH scheduling with a single DCI	LG Electronics

- R1-2209516	On multi-cell PUSCH/PDSCH scheduling with a single DCI	MediaTek Inc.

- R1-2209595	On multi-cell PUSCH/PDSCH scheduling with a single DCI	Apple

- R1-2209746	On multi-cell PUSCH/PDSCH scheduling with a single DCI	Samsung

- R1-2209860	Multi-cell PUSCH/PDSCH scheduling with a single DCI	Ericsson

- R1-2209917	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	NTT DOCOMO, INC.

- R1-2210000	Multi-cell PUSCH/PDSCH scheduling with a single DCI	Qualcomm Incorporated

- R1-2210032	Discussion on multi-cell scheduling with a single DCI	ITRI

- R1-2210034	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	Google Inc.

- R1-2210151	On multi-cell PUSCH/PDSCH scheduling with a single DCI	Nokia, Nokia Shanghai Bell

# List of agreements:

## Agreements made in RAN1#109-e

Agreement

Agree the following terminologies ONLY for convenience of discussion:

- DCI format 0_X is used for scheduling multiple PUSCHs on multiple cells with one PUSCH per cell

- DCI format 1_X is used for scheduling multiple PDSCHs on multiple cells with one PDSCH per cell.

The above does not imply introducing new DCI format(s) at this point.

Agreement

- Different TBs are scheduled on different cells by DCI format 0_X.

- Different TBs are scheduled on different cells by DCI format 1_X.

Agreement

Fallback DCI (i.e., DCI formats 0_0 and 1_0) does not support multi-cell scheduling.

Agreement

The DCI for multi-cell scheduling is monitored only in USS set.

Agreement

- PDSCH cannot be scheduled by DCI format 0_X.

- PUSCH cannot be scheduled by DCI format 1_X.

Agreement

- All the co-scheduled cells by a DCI format 1_X and the scheduling cell are included in the same PUCCH group.

- FFS: All the co-scheduled cells by a DCI format 0_X and the scheduling cell are included in the same [cell or PUCCH group].

Agreement

- DCI format 0-X/1-X on a scheduling cell can be used to schedule PUSCHs/PDSCHs on multiple cells including the scheduling cell.

- DCI format 0-X/1-X on a scheduling cell can be used to schedule PUSCHs/PDSCHs on multiple cells not including the scheduling cell.

Agreement

- For a UE, the maximum number of cells scheduled by a DCI format 0_X can be same or different to the maximum number of cells scheduled by a DCI format 1_X.

Working Assumption

- All HARQ-ACK codebook types (Type-1/2/3) are applicable when multi-carrier PDSCH scheduling is configured.

Agreement

- One value for the maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is selected from {3, 4, 8}.

- For a UE, the maximum number of co-scheduled cells by a DCI format 0_X can be smaller than or equal to the maximum number supported in Rel-18.

Agreement

- One value for the maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is selected from {3, 4, 8}.

- For a UE, the maximum number of co-scheduled cells by a DCI format 1_X can be smaller than or equal to the maximum number supported in Rel-18.

Agreement

- (Working assumption) DCI format 0_X/1_X is a new DCI format for multi-cell scheduling

- DCI format 0_X can be used for single cell PUSCH scheduling.

- DCI format 1_X can be used for single cell PDSCH scheduling.

- FFS: UE monitors one of or both multi-cell scheduling DCI and legacy single cell scheduling DCI for a scheduled cell.

Agreement

- DCI format 0-X/1-X can be transmitted on PCell.

- DCI format 0-X/1-X can be transmitted on a SCell at least when the DCI format 0-X/1-X does not schedule PUSCH/PDSCH on PCell.

- FFS whether a DCI format 0-X/1-X can be transmitted on an SCell if the DCI format 0-X/1-X schedules PUSCH/PDSCH on PCell.

Agreement

Further study DCI size budget including below options for multi-cell scheduling DCI:

- Option 1: Existing DCI size budget is maintained per scheduled cell.

  - Alt 1-1: DCI size budget is maintained via DCI size alignment and DCI size budget of DCI format 0_X/1_X is counted for each of the co-scheduled cells.

  - Alt 1-2: DCI size budget is maintained via configured size for multi-cell scheduling DCI and DCI size budget of DCI format 0_X/1_X is counted for each of the co-scheduled cells.

  - Alt 1-3: DCI size budget is maintained via DCI size alignment and DCI size budget of multi-cell scheduling DCI is counted only in one scheduled cell.

- Option 2: Existing DCI size budget is not necessarily maintained per scheduled cell.

  - Alt 2-1: DCI size budget of multi-cell scheduling DCI is counted only in one scheduled cell.

  - Alt 2-2: DCI size budget of multi-cell scheduling DCI is not counted per serving cell and not considered in the related serving cell specific DCI size alignment procedure, e.g., for K co-scheduled cells, gNB guarantee the total budget of 3*K DCI sizes is not exceeded.

  - Alt 2-3: voiding the “3+1” limit for multi-cell scheduling

  - Alt 2-4: the DCI size budget for DCI size alignment can be separately configured for each cell

  - Alt 2-5: DCI size budget of the scheduling cell can be increased to account for the DCI format for multi-cell scheduling. Accordingly, the DCI size budget of a scheduled cell can be reduced.

- Other options/alternatives could be considered.

Agreement

Further study BD/CCE counting for multi-cell scheduling DCI based on below options:

- Alt 1: counted on each co-scheduled cell

- Alt 2: counted only in one scheduled cell

- Alt 3: scaled down to each of co-scheduled cell according to the number of co-scheduled cells

- Alt 4: counted as part of the scheduling cell instead of each scheduled cell

- Alt 5: scaled down to each of scheduled cells excluding scheduling cell

- Alt 6: counted on each co-scheduled cell excluding scheduling cell

- Other alternatives could be considered.

Agreement

For multi-cell scheduling, the co-scheduled cells are indicated by DCI format 0_X/1_X. At least the following options are considered:

- Option 1: An indicator in the DCI points to one row of a table defining combinations of scheduled cells.

  - The table is configured by RRC signaling.

  - FFS: Separate tables can be configured for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling.

- Option 2: An indicator in the DCI is a bitmap corresponding to a set of configured cells that can be scheduled by the DCI 0_X/1_X

  - FFS: Separate sets of configured cells for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling.

- Option 3: using existing field (e.g., CIF, FDRA) to indicate whether one or more cells are scheduled or not

- Other options are not precluded.

- Note: It does not preclude other DCI information fields (e.g., BWP) to be jointly indicated by the indicator of the co-scheduled cells.

Agreement

For design of multi-cell scheduling DCI, companies are encouraged to consider following types of DCI fields:

- Type-1 field: A single field indicating common information to all the co-scheduled cells or separate information to each of co-scheduled cells via joint indication or an information to only one of co-scheduled cells

- Type-2 field: Separate field for each of the co-scheduled cells, or each sub-group comprising one or more co-scheduled cells where a single field is commonly applied to the co-scheduled cells belonging to a same sub-group

- Type-3 field: Common or separate to each of the co-scheduled cells or to each sub-group.

  - FFS: whether it is dependent on explicit configuration or implicit condition (e.g., intra or inter band CA, FR1 or FR2).

- Other types are not precluded.

## Agreements made in RAN1#110

Agreement

All the co-scheduled cells by a DCI format 0_X and the scheduling cell are included in the same PUCCH group.

Agreement

Confirm below working assumption reached in RAN1#109e meeting.

- (Working assumption) DCI format 0_X/1_X is a new DCI format for multi-cell scheduling

Working Assumption

For a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X and legacy single cell scheduling DCI format(s) from a same scheduling cell.

- The DCI format 0_X/1_X and the legacy DCI format(s) can be monitored simultaneously.

  - FFS: whether monitoring of the DCI format 0_X/1_X and the legacy DCI format(s) is supported for one, a subset, or all cells within the set of cells.

- FFS: number of different DCI sizes for 0_X/1_X and for legacy DCI formats

- FFS: whether to support a subset or all legacy DCI format(s) to be monitored with DCI 0_X/1_X

Working Assumption

- The maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4.

- The maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4.

- FFS: The maximum number of configurable cells for co-scheduling

Agreement

For discussing field design of DCI format 0_X/1_X which schedules more than one cell, reformulate the types of DCI fields as below:

- Type-1 field:

  - Type-1A field: A single field indicating common information to all the co-scheduled cells

  - Type-1B field: A single field indicating separate information to each of co-scheduled cells via joint indication

  - Type-1C field: A single field indicating an information to only one of co-scheduled cells

- Type-2 field: Separate field for each of the co-scheduled cells

- Type-3 field: Common or separate to each of the co-scheduled cells, or separate to each sub-group, dependent on explicit configuration.

  - Note: One sub-group comprises a subset of co-scheduled cells where a single field is commonly applied to the co-scheduled cell(s) belonging to a same sub-group.

- Note: Handling of any parameters applicable to multi-cell scheduling where corresponding fields are not included in DCI format 0_X/1_X (if any) will be separately discussed.

Agreement

- For DCI format 1_X/0_X which can schedule more than one cell,

- Type-1 fields at least include below:

  - Type-1A:

    - Identifier for DCI formats

    - Downlink assignment index

    - TPC for scheduled PUCCH

    - PUCCH resource indicator

    - PDSCH-to-HARQ timing indicator

    - One-shot HARQ-ACK request

- Type-2 fields at least include below:

  - New data indicator per TB

  - Redundancy version per TB

- FFS: Other fields to be included in DCI format 1_X/0_X and which type of the fields belongs to.

- FFS: size for each field

Agreement

- When UE detects a DCI format 1_X scheduling a set of PDSCHs, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot , where  is a number of slots and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format and  is the last UL slot overlapping with the DL slot  for the reference PDSCH reception for slot-based PUCCH or an UL slot overlapping with the end of the reference PDSCH reception in DL slot  for sub-slot based PUCCH.

- FFS details of reference PDSCH

Agreement

- For Type-2 HARQ-ACK codebook, two sub-codebooks are generated with a first sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling a single cell and a second sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling more than one cell.

- Separate DAI counting for DCI(s) with each scheduling a single cell and DCI(s) with each scheduling more than one cell.

- FFS whether a DCI scheduling more than one cell is associated with the first sub-codebook or the second sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one.

- Type-2 HARQ-ACK codebook is generated by concatenating the first sub-codebook and the second sub-codebook.

- If at least one cell of the set of cells which can be co-scheduled by a DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling,

  - FFS: the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell;

- Otherwise, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell is equal to N, where N is the maximum number of cells which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE.

- HARQ-ACK information bits for co-scheduled PDSCHs by a DCI format 1_X is ordered based on serving cell indices associated with co-scheduled PDSCHs.

- HARQ-ACK bundling across co-scheduled cells is not supported for multi-cell scheduling.

Agreement

UE does not expect to be configured both CBG-based PDSCH/PUSCH transmission and the multi-cell PDSCH/PUSCH scheduling on the same or different cells within a same PUCCH group.

Agreement

- At least cases 1-1 and 1-2 on SCS are supported:

- Case 1-1: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and same SCS is used among all the co-scheduled cells including the scheduling cell.

- Case 1-2: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and same SCS is used among all the co-scheduled cells which may be same or different to the SCS of the scheduling cell.

- Case 1-3: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and different SCS is used among the co-scheduled cells including the scheduling cell.

- Case 1-4: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and different SCS is used among the co-scheduled cells.

- FFS: Whether Case 1-3 or 1-4 is additionally supported.

## Agreements made in RAN#97

Conclusion:

- Deprioritize any optimization for unlicensed spectrum operation for designing the multi-cell PUSCH/PDSCH scheduling in Rel-18.

- Enhanced Type-2 HARQ-ACK codebook is not supported for the multi-cell PUSCH/PDSCH scheduling in Rel-18.

- Type-1 HARQ-ACK codebook is supported only for the case where co-scheduled cells by a DCI format 1_X have same SCS/carrier type/duplex mode in Rel-18.

- Additional restriction(s) can be discussed in RAN1

- Configuring more than one scheduling cell for DCI format 0_X/1_X for each scheduled cell is not supported for the multi-cell PUSCH/PDSCH scheduling in Rel-18.

Conclusion:

- Followings are excluded from multi-cell PDSCH/PUSCH scheduling in Rel-18.

- SCell schedules multiple cells including P(S)Cell

- Different SCS among co-scheduled cells

- Different carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2) among co-scheduled cells

- Configuration of both multi-cell PDSCH/PUSCH scheduling and multi-TRP for a scheduled cell

- Support for any sidelink scheduling

Conclusion:

- Following is excluded from multi-cell PDSCH/PUSCH scheduling in Rel-18.

- PCell schedules multiple cells by DCI format 0_X/1_X when a sSCell is configured to schedule PCell

## Agreements made in RAN1#110bis

Agreement

Confirm the following working assumption reached in RAN1#110 meeting.

Working Assumption

- The maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4.

- The maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4.

- FFS: The maximum number of configurable cells for co-scheduling

Agreement

At least the following fields are excluded from DCI format 1_X/0_X:

- CBGTI

- CBGFI

- PDSCH group index

- New feedback indicator

- Number of requested PDSCH group(s)

- Sidelink assignment index

- Second TPC command for scheduled PUSCH

- Second SRS resource indicator

- Second Precoding information

- Second PTRS-DMRS association

- Second TPC command for scheduled PUCCH

Agreement

For DCI format 1_X/0_X, Type-1 fields at least include the following:

- Priority indicator

- Indicator of co-scheduled cells

- beta offset indicator

- CSI request

- UL-SCH indicator

- FFS: ChannelAccess-CPext

Agreement

Confirm below working assumption reached in RAN1#110 meeting with revision.

Working Assumption

- For any cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, RAN1 specification supports monitoring the DCI format 0_X/1_X and DCI format 0_0/1_0, 0_1/1_1, and/or 0_2/1_2 (if supported by the UE), if configured from a same scheduling cell.

- The DCI format 0_X/1_X and the DCI format 0_0/1_0/0_1/1_1/0_2/1_2 can be monitored simultaneously.

- Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell.

Agreement

For a set of cells co-scheduled by a DCI format 0_X/1_X, time domain resource allocations for the set of cells are jointly indicated by a single TDRA field in the DCI format 0_X/1_X.

- Separate {SLIV, mapping type, scheduling offset K0 (or K2)} is indicated for each of co-scheduled PDSCHs/PUSCHs.

- FFS details of the TDRA table design

Agreement

Confirm below working assumption:

Working Assumption

HARQ-ACK codebook types (Type-1, Rel-15 Type-2, Rel-16 Type-3, Rel-17 Type-3) are applicable when multi-cell PDSCH scheduling is configured.

Working Assumption

For a set of cells which is configured for multi-cell scheduling,

- Existing DCI size budget is maintained on each cell of the set of cells.

- DCI size of DCI format 0_X/1_X is counted on one cell among the set of cells.

  - FFS which cell DCI size of the DCI format 0_X/1_X is counted on.

- BD/CCE of DCI format 0_X/1_X is counted on one cell among the set of cells.

  - FFS which cell BD/CCE of the DCI format 0_X/1_X is counted on.

- Search space of DCI format 0_X/1_X is configured on one cell of the set of cells and associated with the search space of the scheduling cell with the same search space ID.

  - FFS which cell the SS of the DCI format 0_X/1_X is configured on.

- FFS: How to address Rel-17 BD/CCE limit for any given cell (operating the feature under Rel-17 BD/CCE limit)

- Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell.

Agreement

- UE does not expect to be configured both multi-PDSCH scheduling and multi-cell PDSCH scheduling on the same or different cells within a same PUCCH group.

Agreement

- For Type-2 HARQ-ACK codebook, if at least one cell of a set of cells which can be co-scheduled by DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell of the set of cells is equal to M, where M is the maximum number of TBs which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE.

Agreement

- For Type-2 HARQ-ACK codebook, a DCI format 1_X scheduling more than one cell is associated with the second sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one.

- If a UE is scheduled by a DCI format 1_X to receive PDSCH over multiple cells, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicates that, for a cell from the multiple cells, at least one symbol from a set of symbols where the UE is scheduled PDSCH reception in the cell is an uplink symbol, the UE does not receive the PDSCH in the cell.

- If a UE is scheduled by a DCI format 0_X to transmit PUSCH over multiple cells, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicates that, for a cell from the multiple cells, at least one symbol from a set of symbols where the UE is scheduled PUSCH transmission in the cell is a downlink symbol, the UE does not transmit the PUSCH in the cell.