3GPP TSG-RAN WG1 Meeting #110b-e	R1-2210789

# e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0371 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 editorial corrections for TS 38.214 (mirrored to Rel-17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core, NR_L1enh_URLLC-Core, NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2022-10-24 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 6.1.2.1, 6.1.2.3.1 and 6.1.2.3.3, during DAPS handover, a UE transmits only on the target cell, and cancels the transmission to source cell in the following handover cases that are specified in Clause 15 in TS 38.213. For DAPS handover that is not intra-frequency, if - the UE indicates support of interFreqUL-TransCancellationDAPS-r16, and - UE does not indicate a capability for power sharing between source and target MCG in DAPS handover or the UE is not provided with uplinkPowerSharingDAPS-Mode, and - UE transmissions on the target cell and the source cell are in overlapping time resources, the UE transmits only on the target cell, and cancels the transmission to source cell. For intra-frequency DAPS handover, if - UE transmissions on the target cell and the source cell are in overlapping time resources, the UE transmits only on the target cell and cancels the transmission on the source cell.. However, the cancellation due to DAPS handover is missed for determination of time domain resources for PUSCH repetitions in TS 38214 In clause 5.2.1.4.2, as summarized in R1-2208707, misalignment of the parameter names of COT duration and CSI-RS validation between TS 38.214 and TS 38.331. 38.331-h20 CO-DurationsPerCell-r16 ::= SEQUENCE { servingCellId-r16 ServCellIndex, positionInDCI-r16 INTEGER(0..maxSFI-DCI-PayloadSize-1), subcarrierSpacing-r16 SubcarrierSpacing, co-DurationList-r16 SEQUENCE (SIZE(1..64)) OF CO-Duration-r16 } csi-RS-ValidationWithDCI-r16 ENUMERATED {enabled} |
| Summary of change: |  | In clauses 6.1.2.1, 6.1.2.3.1 and 6.1.2.3.3, added a missing dropping rule for PUSCH repetitions. In clause 5.2.1.4.2, as summarized in R1-2208707, alignment of the parameter names between TS 38.214 and TS 38.331, that is: - Changed “csi-RS-ValidationWith-DCI” to “csi-RS-ValidationWithDCI” Changed “CO-DurationPerCell-r16” to “CO-DurationsPerCell-r16” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.4.2, 6.1.2.1, 6.1.2.3.1, 6.1.2.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

##### 5.2.1.4.2	Report Quantity Configurations

A UE may be configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to either 'none', 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', 'cri-RSRP', 'cri-SINR', 'ssb-Index-RSRP', 'ssb-Index-SINR', 'cri-RI-LI-PMI-CQI', 'cri-RSRP-Capability[Set]Index', 'ssb-Index-RSRP-Capability[Set]Index', 'cri-SINR-Capability[Set]Index' or 'ssb-Index-SINR-Capability[Set]Index'.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'none', then the UE shall not report any quantity for the CSI-ReportConfig.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI', the UE shall report a preferred precoder matrix for the entire reporting band, or a preferred precoder matrix per subband, according to Clause 5.2.2.2.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-i1',

-	the UE expects, for that CSI-ReportConfig, to be configured with higher layer parameter codebookType set to 'typeI-SinglePanel' and pmi-FormatIndicator set to 'widebandPMI' and,

-	the UE shall report a PMI consisting of a single wideband indication ( in Clause 5.2.2.2.1) for the entire CSI reporting band.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-i1-CQI',

-	the UE expects, for that CSI-ReportConfig, to be configured with higher layer parameter codebookType set to 'typeI-SinglePanel' and pmi-FormatIndicator set to 'widebandPMI' and,

-	the UE shall report a PMI consisting of a single wideband indication ( in Clause 5.2.2.2.1) for the entire CSI reporting band. The CQI is calculated conditioned on the reported assuming PDSCH transmission with  precoders (corresponding to the same but different  in Clause 5.2.2.2.1), where the UE assumes that one precoder is randomly selected from the set of  precoders for each PRG on PDSCH, where the PRG size for CQI calculation is configured by the higher layer parameter pdsch-BundleSizeForCSI.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-CQI',

-	if the UE is configured with higher layer parameter non-PMI-PortIndication contained in a CSI-ReportConfig, r ports are indicated in the order of layer ordering for rank r and each CSI-RS resource in the CSI resource setting is linked to the CSI-ReportConfig based on the order of the associated NZP-CSI-RS-ResourceId in the linked CSI resource setting for channel measurement given by higher layer parameter resourcesForChannelMeasurement. The configured higher layer parameter non-PMI-PortIndication contains a sequence  of port indices, where  are the CSI-RS port indices associated with rank ν and  where is the number of ports in the CSI-RS resource. The UE shall only report RI corresponding to the configured fields of PortIndexFor8Ranks.

-	if the UE is not configured with higher layer parameter non-PMI-PortIndication, the UE assumes, for each CSI-RS resource in the CSI resource setting linked to the CSI-ReportConfig, that the CSI-RS port indices  are associated with ranks  where  is the number of ports in the CSI-RS resource.

-	When calculating the CQI for a rank, the UE shall use the ports indicated for that rank for the selected CSI-RS resource. The precoder for the indicated ports shall be assumed to be the identity matrix scaled by .

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP-Capability[Set]Index' or 'ssb-Index-RSRP-Capability[Set]Index',

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'disabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single report nrofReportedRS (higher layer configured) different CRI or SSBRI for each report setting.

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'enabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single reporting instance two different CRI or SSBRI for each report setting, where CSI-RS and/or SSB resources can be received simultaneously by the UE either with a single spatial domain receive filter, or with multiple simultaneous spatial domain receive filters.

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting-r17, the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single reporting instance nrofReportedRSgroup, if configured, group(s) of two CRIs or SSBRIs selecting one CSI-RS or SSB from each of the two CSI Resource Sets for the report setting, where CSI-RS and/or SSB resources of each group can be received simultaneously by the UE.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-SINR', 'ssb-Index-SINR', 'cri-SINR-Capability[Set]Index' or 'ssb-Index-SINR-Capability[Set]Index',

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'disabled', the UE shall report in a single report nrofReportedRS (higher layer configured) different CRI or SSBRI for each report setting.

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'enabled', the UE shall report in a single reporting instance two different CRI or SSBRI for each report setting, where CSI-RS and/or SSB resources can be received simultaneously by the UE.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRP', 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', 'cri-RI-LI-PMI-CQI', 'cri-SINR', or 'cri-SINR-Capability[Set]Index ', and Ks>1 resources are configured in the corresponding resource set for channel measurement, then the UE shall derive the CSI parameters other than CRI conditioned on the reported CRI, where CRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet for channel measurement, and (k+1)-th entry of associated csi-IM-Resource in the corresponding csi-IM-ResourceSet (if configured) or (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet (if configured for CSI-ReportConfig with reportQuantity set to 'cri-SINR' or 'cri-SINR-Capability[Set]Index ') for interference measurement. If Ks=2 CSI-RS resources are configured, each resource shall contain at most 16 CSI-RS ports. If 2<Ks≤8 CSI-RS resources are configured, each resource shall contain at most 8 CSI-RS ports.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is configured with Ks≥2 resources, two Resource Groups with K1≥1 resources in Group 1, K2≥1 resources in Group 2, K1+K2=Ks, and N Resource Pairs:

-	each resource can contain, subject to UE capability, at most 32 CSI-RS ports. For two Resource Groups with Ki  resources (i=1,2), if maxK1,K2=1, the resource in NZP-CSI-RS-ResourceSet shall contain at most 32 CSI-RS ports; if  maxK1,K2=2, each resource in NZP-CSI-RS-ResourceSet shall contain at most 16 CSI-RS ports; if  2<maxK1,K2<8, each resource in NZP-CSI-RS-ResourceSet shall contain at most 8 CSI-RS ports.

-	each of the N Resource Pairs is associated to a CRI value.

-	The CSI-ReportConfig may be configured with higher layer parameter sharedCMR. M1 and M2 are the numbers of resources associated to a CRI value, other than the N CRIs defined above, in Group 1 and Group 2, respectively, with M=M1+M2, such that the total number of CRI values configured for the CSI-ReportConfig is M+N.

-	If the higher layer parameter csi-ReportMode is set to 'Mode1' and the higher layer parameter numberOfSingleTRP-CSI-Mode1 is set to X∈{0}, M1=M2=0; otherwise,

-	if the higher layer parameter csi-ReportMode is set to 'Mode1' and the higher layer parameter numberOfSingleTRP-CSI-Mode1 is set to X∈{1,2}, or if csi-ReportMode is set to 'Mode2',

-	if sharedCMR is configured: M1=K1 and M2=K2; otherwise

-	if sharedCMR is not configured, only the resources in Group 1 and Group 2 that are not referred to in any Resource Pair are associated to M CRI values other than the N CRIs defined above.

-	If interference measurement is performed on CSI-IM, M+N resources are configured in the corresponding csi-IM-ResourceSet. The M resources for channel measurement defined above are resource-wise associated with the first M CSI-IM resources by the ordering of the CSI-RS resources and CSI-IM resources in the corresponding Resource Set. The N Resource Pairs for channel measurement are associated to the last N CSI-IM resources by the ordering of the CSI-RS Resource Pairs and CSI-IM resources in the CSI-IM Resource Set. The UE may assume that the two CSI-RS resources for channel measurement in a Resource Pair and the associated CSI-IM resource for interference measurement are resource-wise QCLed with respect to 'typeD'.

-	The UE is not expected to be configured with NZP CSI-RS for interference measurement other than the NZP CSI-RS resources for channel measurement configured in the N Resource Pairs.

-	The UE expects, for that CSI-ReportConfig, to be configured with higher layer parameter codebookType set to 'typeI-SinglePanel', and

-	The UE shall derive the CSI parameters other than CRI(s) conditioned on the reported CRI(s), as follows:

-	If the higher layer parameter csi-ReportMode is set to 'Mode1' and the higher layer parameter numberOfSingleTRP-CSI-Mode1 is set to X∈{0,1,2}, X+1 CRI(s) are reported:

-	one CRI k1 k1≥0 corresponds to the configured (k1+1)-th entry of the associated N Resource Pairs in the corresponding CSI-RS Resource Set for channel measurement, and (M+k1+1)-th entry of the corresponding CSI-IM Resource Set, if configured. The UE shall report two RIs, two PMIs, two LIs (if configured), associated to the resource in Group 1 and the resource in Group 2, respectively, of the (k1+1)-th Resource Pair, and one CQI; and

-	if X=1, one CRI k2 (k2≥0) corresponds to the configured (k2+1)-th entry of the associated M resources in the corresponding CSI-RS Resource Set for channel measurement, and (k2+1)-th entry of the corresponding CSI-IM Resource Set, if configured. The UE shall report one RI, one PMI, one LI (if configured) and one or two CQIs conditioned on CRI k2; or

-	if X=2, one CRI k2 (k2≥0) corresponds to the configured (k2+1)-th entry of the associated M1 resources in Group 1 of the corresponding CSI-RS Resource Set for channel measurement, and (k2+1)-th entry of the associated resources in the corresponding CSI-IM Resource Set, if configured, and one CRI k3 (k3≥0) corresponds to the configured (k3+1)-th entry of the associated M2 resources in Group 2 of the corresponding CSI-RS Resource Set for channel measurement, and (M1+k3+1)-th entry of the corresponding CSI-IM Resource Set, if configured. The UE shall report one RI, one PMI, one LI (if configured) and one or two CQIs conditioned on CRI k2 and one RI, one PMI, one LI (if configured) and one or two CQIs conditioned on CRI k3.

-	If the higher layer parameter csi-ReportMode is set to 'Mode2', one CRI k1 (k1≥0) is reported, which corresponds to the (k1+1)-th entry of the M+N resources or Resource Pairs in the corresponding CSI-RS Resource Set for channel measurement, and (k1+1)-th entry of the associated resources in the corresponding CSI-IM Resource Set, if configured. The first M codepoints of the CRI correspond to resources associated to Group 1 and Group 2. The last N codepoints of the CRI correspond to the N configured Resource Pairs. The UE shall report one RI, one PMI, one LI, if configured, and one or two CQIs conditioned on CRI k1 if k1<M; or two RIs, two PMIs, two LIs, if configured, associated to the resource in Group 1 and the resource in Group 2, respectively, of the (k1-M+1)-th Resource Pair, and one CQI, otherwise.

-	For a reported CRI corresponding to an entry of the N Resource Pairs configured in the corresponding CSI-RS Resource Set for channel measurement:

-	the UE shall not report a total number of layers larger than four.

-	the two RIs are reported with a joint RI index corresponding to one of the four rank combinations: 1,1, 1,2, 2,1, {2,2}.

-	The CodebookConfig in CSI-ReportConfig can be configured with two RI restriction parameters. One parameter applies to a reported RI when conditioned on a CRI corresponding to an entry of the M CSI-RS resources defined above. Another parameter applies to a reported joint RI index when conditioned on a CRI corresponding to an entry of the N Resource Pairs and indicates one or more of the four rank combinations that are allowed to correspond to the reported PMIs and RIs.

-	The CodebookConfig in CSI-ReportConfig can be configured with two Codebook Subset Restrictions. The first restriction applies to a reported PMI associated to a CSI-RS resource in Group 1. The second restriction applies to a reported PMI associated to a CSI-RS resource in Group 2.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'ssb-Index-RSRP' or 'ssb-Index-RSRP-Capability[Set]Index', the UE shall report SSBRI, where SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'ssb-Index-SINR' or 'ssb-Index-SINR-Capability[Set]Index', the UE shall derive L1-SINR conditioned on the reported SSBRI, where SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList  in the corresponding CSI-SSB-ResourceSet for channel measurement, and (k+1)-th entry of associated csi-IM-Resource in the corresponding csi-IM-ResourceSet (if configured) or (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet (if configured) for interference measurement.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', ' cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI' or 'cri-RI-LI-PMI-CQI', then the UE is not expected to be configured with more than 8 CSI-RS resources in a CSI-RS resource set contained within a resource setting that is linked to the CSI-ReportConfig.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-LI-PMI-CQI', UE does not expect the CSI-ReportConfig to be configured with higher layer parameter codebookType set to 'typeII-r16' or 'typeII-PortSelection-r16'.

If the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'cri-SINR', 'none', 'cri-RSRP-Capability[Set]Index' or 'cri-SINR-Capability[Set]Index and the CSI-ReportConfig is linked to a resource setting configured with the higher layer parameter resourceType set to 'aperiodic', then the UE is not expected to be configured with more than 16 CSI-RS resources in a CSI-RS resource set contained within the resource setting.

The LI indicates which column of the precoder matrix of the reported PMI corresponds to the strongest layer of the codeword corresponding to the largest reported wideband CQI. If two wideband CQIs are reported and have equal value, the LI corresponds to strongest layer of the first codeword. If the UE is configured with a CSI-ReportConfig with reportQuantity set to 'cri-RI-LI-PMI-CQI' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is configured with two Resource Groups and N Resource Pairs, and the UE reports a CRI associated to a Resource Pair, and a rank combination {ν1,ν2}, the first LI indicates which column of the precoder matrix of the first reported PMI corresponds to the strongest of the first ν1 layers of the codeword and the second LI indicates which column of the precoder matrix of the second reported PMI corresponds to the strongest of the last ν2 layers of the codeword.

For operation with shared spectrum channel access, if the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI' or 'cri-RI-LI-PMI-CQI', the UE shall derive:

-	the CSI parameters without averaging two or more instances of any periodic or semi-persistent nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet for channel measurement or for interference measurement located in different DL transmissions,

-	the instances of the nzp-CSI-RS-Resources are not in the same channel occupancy duration indicated by DCI format 2_0, if the UE is provided at least one of SlotFormatIndicator or co-DurationList; or

-	the instances of the nzp-CSI-RS-Resources occur within a set of consecutive symbols which are not all occupied by PDSCH(s) and/or aperiodic CSI-RS(s) indicated by DCI formats, if any, and the corresponding PDCCH(s), if the UE is neither provided with CO-DurationsPerCell nor SlotFormatIndicator, but is provided with csi-RS-ValidationWithDCI

-	the interference measurements for computing CSI value based on periodic/semi-persistent CSI-IM measured only in OFDM symbol(s) that fulfill the same conditions under which the UE is expected to receive periodic/semi-persistent CSI-RS as described in Clause 11.1 and Clause 11.1.1 of [6, TS 38.213].

If the UE is configured with the higher layer parameter SSB-MTC-AdditionalPCI, the UE is allowed to report in a single reporting instance up to four SSBRIs for each report setting, where SSB resources are associated with PCI indices referring to the PCI of the serving cell and PCI(s) different from the PCI of the serving cell within the set of PCIs configured.

<omitted text>

#### 6.1.2.1	Resource allocation in time domain

<omitted text>

If a UE would transmit a PUSCH of PUSCH repetition Type A when AvailableSlotCounting is enabled and K>1 or a TB processing over multiple slots over N∙K slots, and the UE does not transmit the PUSCH of a TB processing over multiple slots or the PUSCH repetition Type A in a slot from the N∙K slots, according to clause 9, Clause 11.1, Clause 11.2A and Clause 15 of [6, TS 38.213], the UE counts the slots in the number of N∙K slots.

For PUSCH repetition Type A, in case K>1,

-	If the PUSCH is scheduled by DCI format 0_1 or 0_2

-	if AvailableSlotCounting is enabled, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

-	Otherwise, the same symbol allocation is applied across the N∙K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot.

-	Else if the PUSCH is scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

For TB processing over multiple slots:

-	For unpaired spectrum, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

-	For paired spectrum or supplementary uplink band, the same symbol allocation is applied across the  N∙K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K consecutive slots applying the same symbol allocation in each slot.

-	For the case of reduced capability half-duplex UE, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

For a PUSCH transmission scheduled by DCI format 0_1, or 0_2, or 0_0 with CRC scrambled by TC-RNTI, the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … N∙K -1, is determined according to table 6.1.2.1-2.

For a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … N∙K -1, is determined according to the first row of Table 6.1.2.1-2.

Table 6.1.2.1-2: Redundancy version for PUSCH transmission

| rvid indicated by the DCI scheduling the PUSCH | rvid to be applied to nth transmission occasion (repetition Type A) or TB processing over multiple slots) or nth actual repetition (repetition Type B) |  |  |  |
| --- | --- | --- | --- | --- |
|  | ((n-(n mod N))/N) mod 4 = 0 | ((n-(n mod N))/N) mod 4 = 1 | ((n-(n mod N))/N) mod 4 = 2 | ((n-(n mod N))/N) mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

When transmitting MsgA PUSCH on a non-initial UL BWP, if the UE is configured with startSymbolAndLengthMsgA-PO, the UE shall determine the S and L from startSymbolAndLengthMsgA-PO.

When transmitting MsgA PUSCH, if the UE is not configured with startSymbolAndLengthMsgA-PO, and if the TDRA list PUSCH-TimeDomainResourceAllocationList is provided in PUSCH-ConfigCommon, the UE shall use msgA-PUSCH-TimeDomainAllocation to indicate which values are used in the list. If PUSCH-TimeDomainResourceAllocationList is not provided in PUSCH-ConfigCommon, the UE shall use parameters S and L from table 6.1.2.1.1-2 or table 6.1.2.1.1-3 where msgA-PUSCH-TimeDomainAllocation indicates which values are used in the list. The time offset for PUSCH transmission is described in [6, TS 38.213].

For PUSCH repetition Type A and TB processing over multiple slots, a PUSCH transmission in a slot of a multi-slot PUSCH transmission is omitted according to the conditions in Clause 9, Clause 11.1, Clause 11.2A, Clause 15 and Clause 17.2 of [6, TS 38.213].

For PUSCH repetition Type B, except for PUSCH transmitting CSI report(s) with no transport block, the number of nominal repetitions is given by numberOfRepetitions. For the n-th nominal repetition, n = 0, …, numberOfRepetitions - 1,

-	The slot where the nominal repetition starts is given by , and the starting symbol relative to the start of the slot is given by .

-	The slot where the nominal repetition ends is given by , and the ending symbol relative to the start of the slot is given by .

Here is the slot where the PUSCH transmission starts, and is the number of symbols per slot as defined in Clause 4.3.2 of [4, TS38.211].

For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows:

-	A symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated is considered as an invalid symbol for PUSCH repetition Type B transmission.

-	For operation in unpaired spectrum, symbols indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon for reception of SS/PBCH blocks are considered as invalid symbols for PUSCH repetition Type B transmission.

-	For a reduced capability half-duplex UE in paired spectrum and for PUSCH repetition Type B transmission, symbols indicated by ssb-PositionsInBurst in SIB1 or by ssb-PositionsInBurst in ServingCellConfigCommon, or by ssb-PositionsInBurst in SSB-MTC-AdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH blocks configured for L1 beam measurement/reporting for reception of SS/PBCH blocks are considered as invalid symbols for PUSCH repetition Type B transmission.

-	For operation in unpaired spectrum, symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set are considered as invalid symbol(s) for PUSCH repetition Type B transmission.

-	For operation in unpaired spectrum, if numberOfInvalidSymbolsForDL-UL-Switching is configured, numberOfInvalidSymbolsForDL-UL-Switching symbol(s) after the last symbol that is indicated as downlink in each consecutive set of all symbols that are indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated are considered as invalid symbol(s) for PUSCH repetition Type B transmission. The symbol(s) given by numberOfInvalidSymbolsForDL-UL-Switching are defined using the reference SCS configuration referenceSubcarrierSpacing provided in tdd-UL-DL-ConfigurationCommon.

-	For operation with shared spectrum channel access with semi-static channel occupancy, symbols in an idle duration associated with a periodic channel occupancy as described in Clause 4.3.1.1 of [16, 37.213], or in an idle duration in a period associated with an initiated channel occupancy as described in Clause 4.3.2. of [16, TS 37.213] are considered as invalid symbol(s) for PUSCH repetition Type B transmission.

-	The UE may be configured with the higher layer parameter invalidSymbolPattern, which provides a symbol level bitmap spanning one or two slots (higher layer parameter symbols given by invalidSymbolPattern). A bit value equal to 1 in the symbol level bitmap symbols indicates that the corresponding symbol is an invalid symbol for PUSCH repetition Type B transmission. The UE may be additionally configured with a time-domain pattern (higher layer parameter periodicityAndPattern given by invalidSymbolPattern), where each bit of periodicityAndPattern corresponds to a unit equal to a duration of the symbol level bitmap symbols, and a bit value equal to 1 indicates that the symbol level bitmap symbols is present in the unit. The periodicityAndPattern can be {1, 2, 4, 5, 8, 10, 20 or 40} units long, but maximum of 40 msec. The first symbol of periodicityAndPattern every 40 msec/P periods is a first symbol in frame 𝑛𝑓 mod 4 = 0, where P is the duration of periodicityAndPattern-r16 in units of msec. When periodicityAndPattern is not configured, for a symbol level bitmap spanning two slots, the bits of the first and second slots correspond respectively to even and odd slots of a radio frame, and for a symbol level bitmap spanning one slot, the bits of the slot correspond to every slot of a radio frame. If invalidSymbolPattern is configured, when the UE applies the invalid symbol pattern is determined as follows:

-	if the PUSCH is scheduled by DCI format 0_1, or corresponds to a Type 2 configured grant activated by DCI format 0_1, and if invalidSymbolPatternIndicatorDCI-0-1  is configured,

-	if invalid symbol pattern indicator field is set 1, the UE applies the invalid symbol pattern;

-	otherwise, the UE does not apply the invalid symbol pattern;

-	if the PUSCH is scheduled by DCI format 0_2, or corresponds to a Type 2 configured grant activated by DCI format 0_2, and if invalidSymbolPatternIndicatorDCI-0-2  is configured,

-	if invalid symbol pattern indicator field is set 1, the UE applies the invalid symbol pattern;

-	otherwise, the UE does not apply the invalid symbol pattern;

-	otherwise, the UE applies the invalid symbol pattern.

-	If the UE

-	is configured with multiple serving cells within a cell group and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the multiple serving cells, and

-	indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and

-	is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells,

-	a symbol indicated to the UE for reception of SS/PBCH blocks in a first cell of the multiple serving cells by ssb-PositionsInBurst in SIB1 or by ssb-PositionsInBurst in ServingCellConfigCommon, or by ssb-PositionsInBurst in SSB-MTC-AdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH blocks configured for L1 beam measurement/reporting is considered as an invalid symbol for PUSCH repetition Type B transmission in

-	any of the multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and

-	any one of the cells corresponding to the same band as the first cell, irrespective of any capability indicated by simultaneousRxTxInterBandCA

and

-	a symbol is considered as an invalid symbol in another cell among the set of serving  cell(s) provided with directionalCollisionHandling-r16 for PUSCH repetition Type B transmission with Type 1 or Type 2 configured grant except for the first Type 2 PUSCH transmission (including all repetitions) after activation if the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated on the reference cell as defined in Clause 11.1 of [6, TS 38.213], or the UE is configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on the reference cell in the symbol.

For PUSCH repetition Type B, after determining the invalid symbol(s) for PUSCH repetition type B transmission for each of the K nominal repetitions, the remaining symbols are considered as potentially valid symbols for PUSCH repetition Type B transmission. If the number of potentially valid symbols for PUSCH repetition type B transmission is greater than zero for a nominal repetition, the nominal repetition consists of one or more actual repetitions, where each actual repetition consists of a consecutive set of all potentially valid symbols that can be used for PUSCH repetition Type B transmission within a slot. An actual repetition with a single symbol is omitted except for the case of L=1. An actual repetition is omitted according to the conditions in Clause 9, Clause 11.1, Clause 11.2A, Clause 15 and Clause 17.2 of [6, TS 38.213]. The UE shall repeat the TB across actual repetitions. The redundancy version to be applied on the nth actual repetition (with the counting including the actual repetitions that are omitted) is determined according to table 6.1.2.1-2, where N=1.

For PUSCH repetition Type B, when a UE receives a DCI that schedules aperiodic CSI report(s) or activates semi-persistent CSI report(s) on PUSCH with no transport block by a 'CSI request' field on a DCI, the number of nominal repetitions is always assumed to be 1, regardless of the value of numberOfRepetitions. When the UE is scheduled to transmit a PUSCH repetition Type B with no transport block and with aperiodic or semi-persistent CSI report(s) by a 'CSI request' field on a DCI, the first nominal repetition is expected to be the same as the first actual repetition. For PUSCH repetition Type B carrying semi-persistent CSI report(s) without a corresponding PDCCH after being activated on PUSCH by a 'CSI request' field on a DCI, if the first nominal repetition is not the same as the first actual repetition, the first nominal repetition is omitted; otherwise, the first nominal repetition is omitted according to the conditions in Clause 9, Clause 11.1, Clause 11.2A, Clause 15 and Clause 17.2 of [6, TS 38.213].

For PUSCH repetition Type B, when a UE is scheduled to transmit a transport block and aperiodic CSI report(s) on PUSCH by a 'CSI request' field on a DCI, the CSI report(s) is multiplexed only on the first actual repetition. The UE does not expect that the first actual repetition has a single symbol duration.

<omitted text>

##### 6.1.2.3.1	Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant

The procedures described in this clause apply to PUSCH transmissions of PUSCH repetition Type A with a Type 1 or Type 2 configured grant.

The higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions. If cg-RetransmissionTimer is provided, the redundancy version for uplink transmission with a configured grant is determined by the UE. If the parameter repK-RV is not provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, the redundancy version for uplink transmissions with a configured grant shall be set to 0. If the parameter repK-RV is provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, for the nth transmission occasion among K repetitions, n=1, 2, …, K, it is associated with (mod((n-mod(n, N))/N,4)+1)th value in the configured RV sequence, where N=1. If a configured grant configuration is configured with startingFromRV0 set to 'off', the initial transmission of a transport block may only start at the first transmission occasion of the K repetitions. Otherwise, the initial transmission of a transport block may start at

-	the first transmission occasion of the K repetitions if the configured RV sequence is {0,2,3,1},

-	any of the transmission occasions of the K repetitions that are associated with RV=0 if the configured RV sequence is {0,3,0,3},

-	any of the transmission occasions of the K repetitions if the configured RV sequence is {0,0,0,0}, except the last transmission occasion when K≥8.

When the transmission occasions are associated with the first and second SRS resource sets, if the parameter repK-RV is provided in the configuredGrantConfig, for the nth transmission occasion among all transmission occasions that are associated with the SRS resource set of the first transmission occasion, it is associated with (mod(n-1,4)+1)th value in the configured RV sequence, and for the nth transmission occasion among all transmission occasions that are not associated with the SRS resource set of the first transmission occasion, it is associated with (mod(n-1,4)+1)th value in the adjusted RV sequence and the adjustment is based on additional shifting operation on the configured RV sequence, where the shifting operation is defined as rvi+ rvsmod 4 where rvi is the ith RV value (i=1, 2, 3, 4) in the configured RV sequence and rvs is configured by the higher layer parameter sequenceOffsetforRV in configuredGrantConfig. When the transmission occasions are associated with the first and second SRS resource sets, if a configured grant configuration is configured with startingFromRV0 set to 'off', the initial transmission of a transport block may only start at the first transmission occasion of the K repetitions. Otherwise, the initial transmission of a transport block may start at

-	the first transmission occasion associated with RV = 0 corresponding to the first or second SRS resource set of the K repetitions if the configured RV sequence is {0,2,3,1},

-	any of the transmission occasions of the K repetitions that are associated with RV=0 if the configured RV sequence is {0,3,0,3},

-	any of the transmission occasions of the K repetitions if the configured RV sequence is {0,0,0,0}, except the last transmission occasion when K≥8.

After the initial transmission of a transport block, later transmission occasions among the K repetitions associated with any RV value and associated to any of the first or second SRS resource set can be used for transmitting the transport block.

For any RV sequence, the repetitions shall be terminated after transmitting K repetitions, or at the last transmission occasion among the K repetitions within the period P, or from the starting symbol of the repetition that overlaps with a PUSCH with the same HARQ process scheduled by DCI format 0_0, 0_1 or 0_2, whichever is reached first. In addition, the UE shall terminate the repetition of a transport block in a PUSCH transmission if the UE receives a DCI format 0_1 with DFI flag provided and set to '1', and if in this DCI the UE detects ACK for the HARQ process corresponding to that transport block.

The UE is not expected to be configured with the time duration for the transmission of K repetitions larger than the time duration derived by the periodicity P. If the UE determines that, for a transmission occasion, the number of symbols available for the PUSCH transmission in a slot is smaller than transmission duration L, the UE does not transmit the PUSCH in the transmission occasion.

For both Type 1 and Type 2 PUSCH transmissions with a configured grant, when K > 1,

-	For unpaired spectrum:

-	If AvailableSlotCounting is enabled, the UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission applying the same symbol allocation in each slot.

-	A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

-	Otherwise, the UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration.

-	For paired spectrum and SUL band:

-	The UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration.

-	If AvailableSlotCounting is enabled, and in case of reduced capability half-duplex UE, the UE shall repeat the TB across the N∙K slots applying the same symbol allocation in each slot. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

A Type 1 or Type 2 PUSCH transmission with a configured grant in a slot is omitted according to the conditions in Clause 9, Clause 11.1, Clause 11.2A, Clause 15 and Clause 17.2 of [6, TS 38.213].

<omitted text>

##### 6.1.2.3.3	Transport Block repetition for uplink transmissions of TB processing over multiple slots with a configured grant

The procedures described in this clause apply to PUSCH transmissions of TB processing over multiple slots with a Type 2 configured grant.

The higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions. If the parameter repK-RV is not provided in the configuredGrantConfig, the redundancy version for uplink transmissions with a configured grant shall be set to 0. If the parameter repK-RV is provided in the configuredGrantConfig, the nth transmission occasion among N∙K transmissions occasions, n=0,1, …, N∙K -1, is associated with (mod((n-mod(n, N))/N,4)+1)th value in the configured RV sequence. When K=1, or when K>1 and the configured grant configuration is configured with startingFromRV0 set to 'off', the initial transmission of the transport block may only start at the first transmission ocasion of the N∙K transmission occasions. Otherwise, the initial transmission of the transport block may start at

-	The first transmission occasion of the N∙K transmission occasions if the configured RV sequence is {0,2,3,1}.

-	Any transmission occasion n associated with RV=0, and for which n mod N =0, if the configured RV sequence is {0,3,0,3} or {0,0,0,0}.

The UE is not expected to be configured with the time duration for N∙K transmissions larger than the time duration derived by the periodicity P. If the UE determines that, for a transmission occasion, the number of symbols available in a slot for the PUSCH transmission of TB processing over multiple slots is smaller than transmission duration L, the UE does not transmit the PUSCH in the transmission occasion.

For unpaired spectrum:

-	The UE determines N∙K slots for a PUSCH transmission of TB processing over multiple slots with a Type 2 configured grant activated by DCI format 0_1 or 0_2, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst, and the TDRA information field value in the DCI format 0_1 or 0_2.

-	A slot is not counted in the number of N∙K slots for a PUSCH transmission of TB processing over multiple slots with a Type 2 configured grant activated by DCI format 0_1 or 0_2 if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

For paired spectrum and SUL band:

-	The UE determines N∙K consecutive slots for a PUSCH transmission of TB processing over multiple slots with a Type 2 configured grant activated by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2.

-	For the case of a reduced capability half-duplex UE, the UE determines N∙K slots for a PUSCH transmission of TB processing over multiple slots with a Type 2 configured grant activated by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

For Type 2 PUSCH transmission with a configured grant of TB processing over multiple slots, the UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission applying the same symbol allocation in each slot. A Type 2 PUSCH transmission with a configured grant of TB processing over multiple slots is omitted in a slot according to the conditions in Clause 9, Clause 11.1, Clause 11.2A and Clause 15 of [6, TS 38.213].

<omitted text>