3GPP TSG-RAN WG1 Meeting #110b-e	R1-2210790

# e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0372 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 editorial corrections for TS 38.214 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO, NR_SL_enh-Core, NR_ext_to_71GHz-Core, NR_pos_enh-Core, NR_UE_pow_sav_enh-Core, NR_MBS-Core |  |  |  | Date: |  | 2022-10-24 |
| Category: | F |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NR_FeMIMO In clause 5.1.5, as discussed in R1-2210520, RRC parameters for multi-CC lists are named as simultaneousTCI-UpdateList1-r17, simultaneousTCI-UpdateList2-r17, simultaneousTCI-UpdateList3-r17, and simultaneousTCI-UpdateList4-r17. However, in current version of TS 38.331, they are actually named as simultaneousU-TCI-UpdateList1-r17, simultaneousU-TCI-UpdateList2-r17, simultaneousU-TCI-UpdateList3-r17, and simultaneousU-TCI-UpdateList4-r17. In clause 5.1.5, as discussed in R1-2210521, it is mentioned that - The UE is not expected to be configured with TCI-State in a CC in a band if the UE is configured dl-OrJoint-TCIStateList in any CC in the same band; and - The UE can assume that when the UE is configured with TCI-State in any CC in the CC list configured by simultaneousTCI-UpdateList1-r16, simultaneousTCI-UpdateList2-r16, simultaneousSpatial-UpdatedList1-r16, or simultaneousSpatial-UpdatedList2-r16, the UE is not configured with dl-OrJoint-TCIStateList in any CC within the same band in the CC list. Above is meant to specify that Rel-17 unified TCI state and the legacy TCI state cannot be simultanuously configured for a UE in the same band. However, according to the latest 38.331, both tci-StatesToAddModList and dl-OrJoint-TCIStateList configure TCI-State which means that TCI-State is used for both Rel-17 unified TCI state and the legacy TCI state. Therefore, the current text in Clause 5.1.5 of 38.214 should be corrected to clarify that dl-OrJoint-TCIStateList and tci-StatesToAddModList cannot be configured for a UE in the same band. In clause 5.1.5, as discussed in R1-2210517: RRC parameter names TCIState, UL-TCIState and DLorJointTCIState are not aligned with TS 38.331. In clause 5.1.5, as discussed in R1-2210518, in R17 unified TCI framework, if a SRS resource within an SRS resource set which is not configured with followUnifiedTCIstateSRS, it can be updated with a TCIstate or UL-TCIState by a MAC CE which is 6.1.3.59 or 6.1.3.60 specified in TS38.321. However, in TS38.214 V17.3.0, it refers 6.1.3.47 in TS38.321 for beam indication for SRS resource while the MAC CE in 6.1.3.47 is used to activate common beams for DL and UL channels but not for SRS resource. In clause 5.1.5, as discussed in R1-2210519, in RAN1 #106b-e meeting, it was agreed that the largest number of configured TCI states for joint DL/UL TCI state is 128 per BWP per CC for Rel-17 unified TCI framework, the “per BWP” should be captured in the specification. Agreement On Rel.17 unified TCI framework, for Rel-17 unified TCI, the largest number of configured TCI states is given as follows (following Rel-15/16 principles): - When a UE is configured with joint DL/UL TCI: the largest number of configured TCI states for joint DL/UL TCI state update is 128 per BWP per CC. In clause 5.2.1.6, as discussed in R1-2208763, according to latest TS 38.331/TS38.306, UE capability RRC parameter for indicating the number of CPUs for NCJT hypothesis is not captured in TS 38.214. In clause 5.2.1.4.2, as discussed in R1-2209307, the RRC parameters of the number of reported groups per CSI-report are not aligned between 38.331 and 38.214. In TS 38.331, in the IE CSI-ReportConfig, the number of reported resource groups per CSI-report is configured as follows. groupBasedBeamReporting-v1710 SEQUENCE { nrofReportedGroups-r17 ENUMERATED {n1, n2, n3, n4} } The name of the number of reported resource groups in TS 38.214 is not aligned with that in TS 38.331. In clauses 5.1 and 5.1.5, as discussed in R1-2210076: Reason 1: The acronym SFN is not defined in 38.214. In some clause in 38.214 the “SFN offset” is mentioned and SFN referrs to subframe number. In clause 5.1.5 “dynamic switching between SFN PDSCH and non-SFN PDSCH” is unclear and shall be replaced by UE capability indication sfn-SchemeA-DynamicSwitching-r17 or sfn-SchemeB-DynamicSwitching-r17 defined in 38.306. Quote from 38.306: sfn-SchemeA-DynamicSwitching-r17 Indicates whether the UE supports dynamic switching between single-TRP and PDSCH SFN scheme A by TCI state field in DCI formats 1_1 and 1_2. The UE supporting this feature shall indicate sfn-SchemeA-r17 or sfn-SchemeA-PDSCH-only-r17. Reason 2: UE capability “[dynamicSFN]” is not defined, the name is a place holder to be updated when the corresponding capability parameter(s) is available in 38.306, which is sfn-SchemeA-DynamicSwitching-r17 or sfn-SchemeB-DynamicSwitching-r17 Reason 3: To indicate UE capability “DCI scheduling without TCI field”, UE capability parameter “sfn-DefaultDL-BeamSetup-r17” from 38.306 is added to refer to the corresponding UE feature. Quote from 38.306: sfn-DefaultDL-BeamSetup-r17 Indicates whether the UE supports the following features: - For FR2 only, PDSCH reception using default beam for enhanced SFN scheme when PDSCH is scheduled with offset less than threshold. - For FR1 and FR2, PDSCH reception using default beam for enhanced SFN scheme when TCI field is not present in DCI when PDSCH is scheduled with offset equal or larger than the threshold, if applicable. - For FR2 only, aperiodic CSI-RS reception using default beam for enhanced SFN scheme when scheduling offset is less than threshold. The UE indicating support of this feature shall also indicate sfn-schemeA-r17 or sfn-schemeB-r17. In clauses 6.1 and 6.2.5, as discussed in R1-2210077, the UE capability indication for supporting default PUSCH or default SRS beam, when SFN PDCCH is configured, is described using temporary parameter name “[DefaultBeamPL-ForPUSCH-SfnPdcch]” and “[DefaultBeamPL-ForSRS-SfnPdcch]”. These temporary parameters shall be replaced with their corresponding capability parameter “sfn-DefaultUL-BeamSetup-r17” defined in 38.306. Quote from 38.306: sfn-DefaultUL-BeamSetup-r17 Indicates whether the UE supports the following features: - Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured. - Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured. - Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured. The UE indicating support of this feature shall also indicate sfn-schemeA-r17 or sfn-schemeB-r17 or sfn-SchemeA-PDCCH-only-r17. In clause 6.2.1, as discussed in R1-2210551, there is the following agreement in RAN1#106b Agreement Bit width of SOI depends on the maximum number of “t” values configured for any of the aperiodic SRS resource sets - The SOI field is 0 bit if the maximum number of ‘t’ values is one - If at least one resource set has “t” configured - For the resource sets with “t” value configured, each of them is configured with K values of “t”, where 1<=K<=4 - t=0 applies for the resource set(s) without “t” configured in RRC - If none of the resource sets is configured with “t” values, follow Rel-15 approach to determine slot offset. But in current specification, t=0 applies for each SRS resource set when there is at least one SRS resource set without ‘t’ configured in RRC. It doesn’t match above agreement and may lead a useless configuration for ‘t’ for another SRS resource set. In addition, the wording ‘otherwise’ in the section about the case where none of the resource sets is configured with “t” values is controversial, because that there are two conditions before the ‘otherwise’. Then, from spec perspective, it is ambiguous about what is the exact condition corresponding to the ‘otherwise’, i.e., the case where at least one of the resource sets is configured with “t” values and/or the case where the UE is not configured with ca-SlotOffset for at least one of the triggered cell and triggering cell. NR_SL_enh-Core In clause 8.1.4, as discussed in R1-2210287, several editorial inconsistencies have been identified. In clauses 8.1.3, 8.2.1, 8.3, 8.5.1.2, 8.5.2.2, 8.5.2.3, as discussed in R1-2210335, SCI format 2-C is missing in clauses of RV, PSSCH reception and SL CSI. In clauses 8.1 and 8.1.5A, as discussed in R1-2210335, the names used to identify the same field in SCI format 2-C in TS 38.214 are different from those used in TS 38.212. The brackets around the field names in TS 38.214 shall be removed. In clause 8.1.4C, as discussed in R1-2210336, normative text was inserted with the word ‘Note’. NR_ext_to_71GHz-Core In clause 5.1.6.2, as discussed in R1-2209440, RRC parameter dmrs-FD-OCC-DisabledForRank1-PDSCH to disable FD-OCC is described in 38.331, but this has not exactly implemented in 38.214 specification. In clauses 5.1 and 6.1, as discussed in R1-2209694, based on TS38.331, RAN2 introduces new RRC parameters nrofHARQ-ProcessesForPDSCH-v1700 and nrofHARQ-ProcessesForPUSCH-r17 to indicate 32 HARQ process numbers for PDSCH reception and PUSCH, respectively. In clause 5.1.5, as discussed in R1-2208828, according to TS 38.331 and TS 38.306, when a UE reports a capability using higher-layer parameter beamCorrespondenceWithoutUL-BeamSweeping, this higher-layer parameter should be set to ‘supported’. But in current spec TS 38.214, this parameter is set to ‘1’ to indicate this UE capability, which is not aligned with the value provided in TS 38.331. NR_pos_enh-Core In clause 5.1.6.5, as discussed in R1-2209458, in the current TS 38.214, some higher layer parameters are still in bracket. In clause 5.1.6.5, as discussed in R1-2208602, for determining the DL PRS priority, different descriptions are used for DL signals and channels in 38.214, including ‘DL signals and channels’, ‘DL signals/channels’, ‘DL signal/channels’. NR_UE_pow_sav_enh-Core In clause 5.1.6.1.1, as discussed in R1-2208478 and R1-2209857, some parameter names for Rel-17 UE power saving are not aligned with the parameter names used in TS 38.331. NR_MBS-Core In clauses 5.1.2.1, 5.1.2.1.1, 5.1.2.2, 5.1.2.2.1, 5.1.2.3, 5.1.3.1, 5.1.3.2, 5.1.4.1, 5.1.4.2, 5.1.6.2, 5.1.6.3, as discussed in R1-2209318, some RRC parameters are not aligned between 38.331 and 38.214. |
| Summary of change: |  | NR_FeMIMO In clause 5.1.5, as discussed in R1-2210520, replaced simultaneousTCI-UpdateList1-r17, simultaneousTCI-UpdateList2-r17, simultaneousTCI-UpdateList3-r17, and simultaneousTCI-UpdateList4-r17 with simultaneousU-TCI-UpdateList1-r17, simultaneousU-TCI-UpdateList2-r17, simultaneousU-TCI-UpdateList3-r17, and simultaneousU-TCI-UpdateList4-r17 respectively. In clause 5.1.5, as discussed in R1-2210521,TCI-State is changed to tci-StatesToAddModList in two occasions to clarify the actual intention of the specification that Rel-17 unified TCI state and the legacy TCI state cannot be simulatanuously configured in the same band. In clause 5.1.5, as discussed in R1-2210517: changed “TCIState” to “TCI-State”. changed “UL-TCIState” to “TCI-UL-State”. changed “DLorJointTCIState” to “TCI-State”. changed “if UE is configured with TCI-State configurations with DLorJointTCIState” to “if UE is configured with TCI-State in dl-OrJoint-TCIStateList”. In clause 5.1.5, as discussed in R1-2210518, change the reference of MAC CE for beam indication of SRS resource from 6.1.3.47 to 6.1.3.59 or 6.1.3.60 on unified TCI framework. In clause 5.1.5, as discussed in R1-2210519, complement the “per BWP” description in the largest number of configured TCI states for Rel-17 unified TCI. In clause 5.2.1.6, as discussed in R1-2208763, replaced [UE capability] by mTRP-CSI-numCPU-r17 In clause 5.2.1.4.2, as discussed in R1-2209307, aligned the following RRC parameter names in 38.214 with the RRC specification in 38.331: - nrofReportedRSgroup nrofReportedGroups In clauses 5.1 and 5.1.5, as discussed in R1-2210076: Change 1: Replaced undefined terminology and acronym “SFN PDSCH and non-SFN PDSCH” with correct UE capability parameters from 38.306. Change 2: Replaced undefined UE capability “[dynamicSFN]” with correct UE capability parameters from 38.306. Change 3: Added UE capability parameter sfn-DefaultDL-BeamSetup-r17 before “DCI scheduling without TCI field” to clarify the related UE capability. In clauses 6.1 and 6.2.5, as discussed in R1-2210077: Change 1: Replaced undefined UE capability “[DefaultBeamPL-ForPUSCH-SfnPdcch]” with related UE capability parameter from 38.306. Change 2: Replaced undefined UE capability “[DefaultBeamPL-ForSRS-SfnPdcch]” with correct UE capability parameter from 38.306. In clause 6.2.1, as discussed in R1-2210551, clarified the description for available slot offset and the condition for AP-SRS timeline (i.e., in the case that the UE is not configured with ca-SlotOffset for both the triggered and triggering cell). NR_SL_enh-Core In clause 8.1.4, as discussed in R1-2210287, several editorial inconsistencies have been fixed. In clauses 8.1.3, 8.2.1, 8.3, 8.5.1.2, 8.5.2.2, 8.5.2.3, as discussed in R1-2210335, SCI format 2-C is added in corresponding clauses of RV, PSSCH reception and SL CSI. In clauses 8.1 and 8.1.5A, as discussed in R1-2210335, for SCI format 2-C, correct field names to align with TS 38.212, removed brackets around field names. In clause 8.1.4C, as discussed in R1-2210336, removed the word ‘Note’ in three places. NR_ext_to_71GHz-Core In clause 5.1.6.2, as discussed in R1-2209440, [dmrs-FD-OCC-disableForRank1PDSCH] in 38.214 is changed to dmrs-FD-OCC-DisabledForRank1-PDSCH. In clauses 5.1 and 6.1, as discussed in R1-2209694, added the RRC parameters nrofHARQ-ProcessesForPDSCH-v1700 and nrofHARQ-ProcessesForPUSCH-r17 to determine 32 HARQ process numbers. In clause 5.1.5, as discussed in R1-2208828, aligned the value setting of beamCorrespondenceWithoutUL-BeamSweeping in TS 38.214 with TS 38.331 and TS 38.306. NR_pos_enh-Core In clause 5.1.6.5, as discussed in R1-2209458, confirm the parameter names in the bracket. In clause 5.1.6.5, as discussed in R1-2208602, multiple descriptions are used for DL signals/channels. In this CR, the description is aligned in 38.214 to ‘DL signals and channels’. NR_UE_pow_sav_enh-Core In clause 5.1.6.1.1, as discussed in R1-2208478 and R1-2209857, updates on the parameter names to align with TS 38.331： - Updated the parameter name of “numberOfresources” to “nrofResources”; - Updated “scramblingID” to “ScramblingID”; Update “TRS-ResourceSetConfig” to “trs-ResourceSetConfig”. NR_MBS-Core In clauses 5.1.2.1, 5.1.2.1.1, 5.1.2.2, 5.1.2.2.1, 5.1.2.3, 5.1.3.1, 5.1.3.2, 5.1.4.1, 5.1.4.2, 5.1.6.2, 5.1.6.3, as discussed in R1-2209318, aligned the following RRC parameter names in 38.214 with the RRC specification in 38.331: - pdsch-AggregationFactorMulticast à pdsch-AggregationFactor - pdsch-Config-Multicast à MBS-RNTI-SpecificConfig in the description of multicast dynamic scheduling PDSCH repetition number configuration - PDSCH-ConfigMulticast, PDSCH-Config-Multicast, pdsch-Config-Multicast à pdsch-ConfigMulticast in other sections - pdsch-ConfigPTM à PDSCH-ConfigPTM - PDSCH-Config-MTCH à pdsch-ConfigMTCH - PDSCH-Config-MCCH à pdsch-ConfigMCCH - SPS-Config-Multicast à SPS-Config - CFR-Config-Multicast à CFR-ConfigMulticast - xOverhead-Multicast à xOverheadMulticast Changed the second “PDSCH-Config-MCCH” to “pdsch-ConfigMTCH” in sentence abouth LTE-CRS ratematching pattern configuration for broadcast reception in section 5.4.1.2 |  |  |  |  |  |

| Consequences if not approved: | NR_FeMIMO In clause 5.1.5, as discussed in R1-2210520: misalignment between TS 38.214 and TS 38.331. In clause 5.1.5, as discussed in R1-2210521, the current specification is incorrect as it states that UE is not expected to be configured with TCI-State and dl-OrJoint-TCIStateList in the same band. However, dl-OrJoint-TCIStateList is a container that includes a set of TCI-State. In clause 5.1.5, as discussed in R1-2210517, incorrectly RRC parameters cause confusion on Rel-17 TCI state. In clause 5.1.5, as discussed in R1-2210518, beam indication of SRS resource based on unified TCI framework is not correct. In clause 5.1.5, as discussed in R1-2210519, the maximum number of TCI states per BWP per CC in the specification is unclear for the network to configure. In clause 5.2.1.6, as discussed in R1-2208763, the specification on the number of CPUs for NCJT hypotheses is incomplete. In clause 5.2.1.4.2, as discussed in R1-2209307, incorrect RRC parameter names. In clauses 5.1 and 5.1.5, as discussed in R1-2210076 the description in 38.214 is unclear and inconsistent with 38.306. In clauses 6.1 and 6.2.5, as discussed in R1-2210077, the UE capability name in 38.214 is inconsistent with 38.306. In clause 6.2.1, as discussed in R1-2210551, it will lead to misalignment for AP-SRS timeline between gNB and UE sides. NR_SL_enh-Core In clauses 8.1.3, 8.2.1, 8.3, 8.5.1.2, 8.5.2.2, 8.5.2.3, as discussed in R1-2210335, SCI format 2-C cannot support functions of RV, PSSCH reception and SL CSI. In clauses 8.1 and 8.1.5A, as discussed in R1-2210335, SCI format 2-C field names remain inconsistent with TS 38.212. In clause 8.1.4C, as discussed in R1-2210336, some implementations may mistakenly ignore the specification. In clause 5.2.1.4.2, as summarized in R1-2208707, implementation error may occur due to the non-existence of the parameter. NR_ext_to_71GHz-Core In clause 5.1.6.2, RRC parameter misalignment between 38.214 and 38.331 specifications. In clauses 5.1 and 6.1, 32 HARQ processes cannot be used. In clause 5.1.5, misalignment between different specifications. NR_pos_enh-Core In clause 5.1.6.5, the specification 38.214 is not complete. In clause 5.1.6.5, multiple descriptions are used for DL signals and channels. NR_UE_pow_sav_enh-Core In clause 5.1.6.1.1, some parameter names for Rel-17 UE power saving are not aligned with the name used in TS 38.331. NR_MBS-Core In clauses 5.1.2.1, 5.1.2.1.1, 5.1.2.2, 5.1.2.2.1, 5.1.2.3, 5.1.3.1, 5.1.3.2, 5.1.4.1, 5.1.4.2, 5.1.6.2, 5.1.6.3, incorrect RRC parameter names for Rel-17 MBS in 38.214 |  |  |  |
| --- | --- | --- | --- | --- |
| Clauses affected: | 5.1, 5.1.2.1, 5.1.2.1.1, 5.1.2.2, 5.1.2.2.1, 5.1.2.3, 5.1.3.1, 5.1.3.2, 5.1.4.1, 5.1.4.2, 5.1.5, 5.1.6.1.1, 5.1.6.2, 5.1.6.3, 5.1.6.5, 5.2.1.4.2, 5.2.1.6, 6.1, 6.2.1, 8.1, 8.1.3, 8.1.4, 8.1.4.C, 8.2.1, 8.3, 8.5.1.2, 8.5.2.2, 8.5.2.3 |  |  |  |
|  | Y | N |  |  |
| Other specs |  | X | Other core specifications | TS/TR ... CR ... |
| affected: |  | X | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  | X | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |
| This CR's revision history: |  |  |  |  |

<omitted text>

## 5.1	UE procedure for receiving the physical downlink shared channel

For downlink, a maximum of 16 HARQ processes per cell are supported by the UE, or subject to UE capability, a maximum of 32 HARQ processes per cell as defined in [13, TS 38.306]. The number of processes the UE may assume will at most be used for the downlink is configured to the UE for each cell separately by higher layer parameter nrofHARQ-ProcessesForPDSCH or nrofHARQ-ProcessesForPDSCH-v1700, and when no configuration is provided the UE may assume a default number of 8 processes.

A UE shall upon detection of a PDCCH with a configured DCI format 1_0, 1_1, 4_0, 4_1, 4_2 or 1_2 decode the corresponding PDSCHs as indicated by that DCI. When the UE is scheduled with multiple PDSCHs by a DCI, HARQ process ID indicated by this DCI applies to the first PDSCH not overlapping with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, HARQ process ID is then incremented by 1 for each subsequent PDSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPDSCH applied if nrofHARQ-ProcessesForPDSCH is provided, or with modulo operation of nrofHARQ-ProcessesForPDSCH-v1700 applied if or nrofHARQ-ProcessesForPDSCH-v1700 is provided,or with modulo operation of 8 applied, otherwise. HARQ process ID is not incremented for PDSCH(s) not received if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to receive a PDSCH that overlaps in time with another PDSCH. When HARQ feedback for the HARQ process ID is not disabled, or for the HARQ process associated with the first SPS PDSCH when HARQ-feedbackEnablingforSPSactive is provided and enabled, the UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process, where the timing is given by Clause 9.2.3 of [6, TS 38.213]. For HARQ-ACK subject to HARQ-ACK deferral described in Clause 9.2.5.4 of [6 TS 38.213], the expected transmission of HARQ-ACK corresponds to the expected transmission HARQ-ACK in a first slot. When HARQ feedback for the HARQ process ID is disabled, the UE is not expected to receive another PDCCH carrying a DCI scheduling a PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process or to receive another PDSCH without corresponding PDCCH for the given HARQ process that starts until Tproc,1 after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet and PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex, in a given scheduled cell, the UE is not expected to receive a first PDSCH and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH, where the two resources are in different slots for the associated HARQ-ACK transmissions, each slot is composed of symbols [4] or a number of symbols indicated by subslotLengthForPUCCH if provided, and the HARQ-ACK for the two PDSCHs are associated with the HARQ-ACK codebook of the same priority. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet and PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex, in a given scheduled cell, the UE is not expected to receive a first PDSCH, and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH if the HARQ-ACK for the two PDSCHs are associated with HARQ-ACK codebooks of different priorities. For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH ending in symbol i on a scheduling cell, the UE is not expected to be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH that ends later than symbol i of a scheduling cell,. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], the PDCCH ending in symbol i is determined based on the PDCCH candidate that ends later in time. In a given scheduled cell, for any PDSCH corresponding to SI-RNTI, the UE is not expected to decode a re-transmission of an earlier PDSCH with a starting symbol less than N symbols after the last symbol of that PDSCH, where the value of N depends on the PDSCH subcarrier spacing configuration , with N=13 for =0, N=13 for =1, N=20 for =2, N=24 for =3, N=96 for m=5, and N=192 for m=6.

When receiving PDSCH scheduled with SI-RNTI, P-RNTI, G-RNTI for broadcast or MCCH-RNTI, the UE may assume that the DM-RS port of PDSCH is quasi co-located with the associated SS/PBCH block with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable.

When receiving PDSCH scheduled with RA-RNTI, or MSGB-RNTI, the UE may assume that the DM-RS port of PDSCH is quasi co-located with the SS/PBCH block or the CSI-RS resource the UE used for RACH association as applicable, and transmission with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable. When receiving a PDSCH scheduled with RA-RNTI in response to a random access procedure triggered by a PDCCH order which triggers contention-free random access procedure for the SpCell [10, TS 38.321], the UE may assume that the DM-RS port of the received PDCCH order and the DM-RS ports of the corresponding PDSCH scheduled with RA-RNTI are quasi co-located with the same SS/PBCH block or CSI-RS with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable.

When receiving PDSCH in response to a PUSCH transmission scheduled by a RAR UL grant or corresponding PUSCH retransmission, or when receiving PDSCH in response to a PUSCH for Type-2 random access procedure, or a PUSCH scheduled by a fallbackRAR UL grant or corresponding PUSCH retransmission, the UE may assume that the DM-RS port of PDSCH is quasi co-located with the SS/PBCH block the UE selected for RACH association and transmission with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable.

If the UE is not configured for PUSCH/PUCCH transmission for at least one serving cell configured with slot formats comprised of DL and UL symbols, and if the UE is not capable of simultaneous reception and transmission on serving cell c1 and serving cell c2, the UE is not expected to receive PDSCH on serving cell c1 if the PDSCH overlaps in time with SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) on serving cell c2 not configured for PUSCH/PUCCH transmission.

The UE is not expected to decode a PDSCH in a serving cell scheduled by a PDCCH with C-RNTI, CS-RNTI or MCS-C-RNTI and one or multiple PDSCH(s) required to be received according to this Clause in the same serving cell without a corresponding PDCCH transmission if the PDSCHs partially or fully overlap in time except if the PDCCH scheduling the PDSCH ends at least 14∙2max⁡(0,μ-3) symbols before the earliest starting symbol of the PDSCH(s) without the corresponding PDCCH transmission, where m and the symbol duration are based on the smallest numerology between the scheduling PDCCH and the PDSCH, in which case the UE shall decode the PDSCH scheduled by the PDCCH. When the PDCCH reception incudes two PDCCH candidates from two respectvie search space sets, as described in clause 10 of [6, TS 38.213], for the purpose of determining the PDCCH with C-RNTI, CS-RNTI or MCS-C-RNTI scheduling the PDSCH ends at least 14∙2max⁡(0,μ-3) symbols before the earliest starting symbol of the PDSCH(s) without the corresponding PDCCH transmission, the PDCCH candidate that ends later in time is used.

The UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, G-RNTI, MCCH-RNTI, G-GS-RNTI or CS-RNTI if another PDSCH in the same cell scheduled with RA-RNTI or MSGB-RNTI partially or fully overlap in time.

The UE in RRC_IDLE and RRC_INACTIVE modes shall be able to decode two PDSCHs each scheduled with SI-RNTI, P-RNTI, RA-RNTI or TC-RNTI, with the two PDSCHs partially or fully overlapping in time in non-overlapping PRBs.

The UE:

-	is expected to decode PDSCH scheduled with MCCH-RNTI and PBCH in Pcell that partially or fully overlaps in time in non-overlapping PRBs in Pcell.

-	is not expected to decode PDSCH scheduled with broadcast G-RNTI and PBCH in Pcell that partially or fully overlaps in time in non-overlapping PRBs in Pcell.

-	is not expected to decode PDSCH scheduled with multicast G-RNTI and PBCH in Pcell that partially or fully overlaps in time in non-overlapping PRBs in Pcell.

On a frequency range 1 cell, the UE shall be able to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI and, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI that partially or fully overlap in time in non-overlapping PRBs, unless the PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI requires Capability 2 processing time according to clause 5.3 in which case the UE may skip decoding of the scheduled PDSCH with C-RNTI, MCS-C-RNTI, or CS-RNTI.

On a frequency range 2 cell, the UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI if in the same cell, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI partially or fully overlap in time.

The UE is expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI during a process of autonomous SI acquisition.

The maximum number of PDSCHs scheduled per slot per component carrier with C-RNTI/CS-RNTI and G-RNTI/G-CS-RNTI/MCCH-RNTI that the UE shall be able to decode is the same as the indicated UE capability for the number of unicast PDSCHs per slot per component carrier. If the UE is capable of receiving FDMed unicast and multicast PDSCH per slot per carrier, the UE shall be able to decode a PDSCH scheduled with C-RNTI/CS-RNTI and a PDSCH scheduled with G-RNTI for multicast/G-CS-RNTI that partially or fully overlap in time in non-overlapping PRBs. If the UE is capable of receiving FDMed unicast and broadcast PDSCH per slot per carrier, the UE shall be able to decode a PDSCH scheduled with C-RNTI/CS-RNTI and a PDSCH scheduled with G-RNTI for broadcast/MCCH-RNTI that partially or fully overlap in time in non-overlapping PRBs.

If the UE is configured by higher layers to decode a PDCCH with its CRC scrambled by a CS-RNTI or G-CS-RNTI, the UE shall receive PDSCH transmissions without corresponding PDCCH transmissions using the higher-layer-provided PDSCH configuration for those PDSCHs.

The UE it is not expected to support reception of FDMed MCCH PDSCH and MTCH PDSCH in PCell or SCell, or FDMed multiple MTCH PDSCHs in PCell or SCell, or FDMed MCCH/MTCH/multicast PDSCH and SIB PDSCH in Pcell, or FDMed multicast PDSCHs in Pcell or Scell, or FDMed multicast PDSCH and MCCH/MTCH for broadcast in Pcell or Scell, or FDMed MCCH/MTCH/multicast PDSCH and paging PDSCH, or FDMed MCCH/MTCH/multicast PDSCH and SIB1 PDSCH that partially or fully overlap in time in non-overlapping PRBs.

If a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, the UE may expect to receive multiple PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain. The UE may expect the reception of full/partially-overlapped PDSCHs in time, only when PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex. For a ControlResourceSet without coresetPoolIndex, the UE may assume that the ControlResourceSet is assigned with coresetPoolIndex as 0. When the UE is configured with SSB-MTC-AdditionalPCI, ControlResourceSets corresponding to different coresetPoolIndex values may be associated with different physical cell IDs via activated TCI states of the ControlResourceSets, where ControlResourceSets corresponding to one coresetPoolIndex is associated with the serving cell physical cell ID and ControlResourceSets corresponding to another coresetPoolIndex can be associated with another physical cell ID. When the UE is scheduled with full/partially/non-overlapped PDSCHs in time and frequency domain, the full scheduling information for receiving a PDSCH is indicated and carried only by the corresponding PDCCH, the UE is expected to be scheduled with the same active BWP and the same SCS. When the UE is scheduled with full/partially-overlapped PDSCHs in time and frequency domain, the UE can be scheduled with at most two codewords simultaneously. When PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex, the following operations are allowed:

-	For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH associated with a value of coresetPoolIndex ending in symbol i, the UE can be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH associated with a different value of coresetPoolIndex that ends later than symbol i.

-	In a given scheduled cell, the UE can receive a first PDSCH in slot i, with the corresponding HARQ-ACK assigned to be transmitted in slot j, and a second PDSCH associated with a value of coresetPoolIndex different from that of the first PDSCH starting later than the first PDSCH with its corresponding HARQ-ACK assigned to be transmitted in a slot before slot j.

If PDCCHs that schedule corresponding PDSCHs are associated to the same or different ControlResourceSets having the same value of coresetPoolIndex, the UE procedure for receiving the PDSCH upon detection of a PDCCH follows Clause 5.1.

A UE does not expect to be configured with repetitionScheme if the UE is configured with higher layer parameter repetitionNumber for the same PDSCH.

When a UE is configured by higher layer parameter repetitionScheme set to one of 'fdmSchemeA', 'fdmSchemeB', 'tdmSchemeA', if the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within one CDM group in the DCI field 'Antenna Port(s)'.

-	When two TCI states are indicated in a DCI and the UE is set to 'fdmSchemeA', the UE shall receive a single PDSCH transmission occasion of the TB with each TCI state associated to a non-overlapping frequency domain resource allocation as described in Clause 5.1.2.3.

-	When two TCI states are indicated in a DCI and the UE is set to 'fdmSchemeB', the UE shall receive two PDSCH transmission occasions of the same TB with each TCI state associated to a PDSCH transmission occasion which has non-overlapping frequency domain resource allocation with respect to the other PDSCH transmission occasion as described in Clause 5.1.2.3.

-	When two TCI states are indicated in a DCI and the UE is set to 'tdmSchemeA', the UE shall receive two PDSCH transmission occasions of the same TB with each TCI state associated to a PDSCH transmission occasion which has non-overlapping time domain resource allocation with respect to the other PDSCH transmission occasion and both PDSCH transmission occasions shall be received within a given slot as described in Clause 5.1.2.1.

When a UE is configured by the higher layer parameter repetitionNumber in PDSCH-TimeDomainResourceAllocation, the UE may expect to be indicated with one or two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' together with the DCI field 'Time domain resource assignment' indicating an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation and DM-RS port(s) within one CDM group in the DCI field 'Antenna Port(s)'.

-	When two TCI states are indicated in a DCI with 'Transmission Configuration Indication' field, the UE may expect to receive multiple slot level PDSCH transmission occasions of the same TB with two TCI states used across multiple PDSCH transmission occasions in the repetitionNumber consecutive slots as defined in Clause 5.1.2.1.

-	When one TCI state is indicated in a DCI with 'Transmission Configuration Indication' field, the UE may expect to receive multiple slot level PDSCH transmission occasions of the same TB with one TCI state used across multiple PDSCH transmission occasions in the repetitionNumber consecutive slots as defined in Clause 5.1.2.1.

When a UE is not indicated with a DCI that DCI field 'Time domain resource assignment' indicating an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation, and it is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within two CDM groups in the DCI field 'Antenna Port(s)' and it is not configured with higher layer parameter sfnSchemePdsch, the UE may expect to receive a single PDSCH where the association between the DM-RS ports and the TCI states are as defined in Clause 5.1.6.2.

When a UE is not indicated with a DCI that DCI field 'Time domain resource assignment' indicating an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation, and it is indicated with one TCI states in a codepoint of the DCI field 'Transmission Configuration Indication', the UE procedure for receiving the PDSCH upon detection of a PDCCH follows Clause 5.1.

When a UE is configured with higher layer parameter sfnSchemePdsch set to either 'sfnSchemeA' or 'sfnSchemeB' for a DL BWP and

-	if the UE reports its capability of sfn-SchemeA-DynamicSwitching-r17 or sfn-SchemeB-DynamicSwitching-r17, the UE is indicated with one or two TCI state(s) in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2, or

-	otherwise, the UE is not expected to be indicated with one TCI state per any of TCI codepoint by MAC CE, and the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2, and

the UE procedure for receiving the PDSCH upon detection of a PDCCH follows clause 5.1 and the QCL assumption for the PDSCH as defined in clause 5.1.5.

When a UE is configured with both sfnSchemePdsch and sfnSchemePdcch, the UE shall expect that sfnSchemePdsch and sfnSchemePdcch are set to the same scheme, either 'sfnSchemeA' or 'sfnSchemeB'.

If a UE is configured with sfnSchemePdcch set to 'sfnSchemeA' for a DL BWP and activated with two TCI states by MAC CE, and the UE does not report its capability of [nonSfnPdsch-sfnPdcch], the UE is expected to be configured with sfnSchemePdsch set to 'sfnSchemeA' and indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication', if the PDSCH is scheduled by DCI format 1_1/1_2.

If a UE is configured with sfnSchemePdcch set to 'sfnSchemeB' for a DL BWP and activated with two TCI states by MAC CE, the UE is expected to be configured with sfnSchemePdsch set to 'sfnSchemeB' and indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication', if the PDSCH is scheduled by DCI format 1_1/1_2.

When a UE is configured with sfnSchemePdsch and/or sfnSchemePdcch, the UE shall expect that the sfnSchemePdsch and/or sfnSchemePdcch configuration are the same in all DL BWP within a CC other than initial BWP, and the UE shall expect that the sfnSchemePdsch and/or sfnSchemePdcch configuration are the same in all CCs in a same frequency band if the UE is configured with CA.

If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, after resolving overlapping with symbols in the slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated, a UE receives one or more PDSCHs without corresponding PDCCH transmissions in the slot as specified below.

‒	Step 0: set j=0, where j is the number of selected PDSCH(s) for decoding. Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot

‒	Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q, set j=j+1. Designate the received PDSCH as survivor PDSCH.

‒	Step 2: The survivor PDSCH in step 1 and any other PDSCH(s) overlapping (even partially) with the survivor PDSCH in step 1 are excluded from Q.

‒	Step 3: Repeat step 1 and 2 until Q is empty or j is equal to the number of unicast/multicast PDSCHs in a slot supported by the UE

<omitted text>

#### 5.1.2.1	Resource allocation in time domain

When the UE is scheduled to receive PDSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocation table. The determination of the used resource allocation table is defined in Clause 5.1.2.1.1. The indexed row defines the slot offset K0, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PDSCH mapping type to be assumed in the PDSCH reception.

Given the parameter values of the indexed row:

-	The slot allocated for the PDSCH is Ks, where , if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, and Ks = , otherwise, and where n is the slot with the scheduling DCI, and K0 is based on the numerology of PDSCH, and  and are the subcarrier spacing configurations for PDSCH and PDCCH, respectively, and

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset, for the cell receiving the PDCCH respectively, Nslot, offset, PDSCHCA and μoffset,PDSCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDSCH, as defined in clause 4.5 of [4, TS 38.211].

-	The reference point S0 for starting symbol S is defined as:

-	if configured with referenceOfSLIVDCI-1-2, and when receiving PDSCH scheduled by DCI format 1_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI with K0=0, and PDSCH mapping Type B, the starting symbol S is relative to the starting symbol S0 of the PDCCH monitoring occasion where DCI format 1_2 is detected; when the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], the PDCCH candidate that starts later in time is used for the purpose of determining the starting symbol S0;

-	otherwise, the starting symbol S is relative to the start of the slot using S0=0.

-	The number of consecutive symbols L counting from the starting symbol S allocated for the PDSCH are determined from the start and length indicator SLIV:

if  then

else

where, and

-	the PDSCH mapping type is set to Type A or Type B as defined in Clause 7.4.1.1.2 of [4, TS 38.211].

The UE shall consider the S and L combinations defined in table 5.1.2.1-1 satisfying  for normal cyclic prefix and  for extended cyclic prefix as valid PDSCH allocations:

Table 5.1.2.1-1: Valid S and L combinations

| PDSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A | {0,1,2,3} (Note 1) | {3,…,14} | {3,…,14} | {0,1,2,3} (Note 1) | {3,…,12} | {3,…,12} |
| Type B | {0,…,12} | {2,…,13} | {2,…,14} | {0,…,10} | {2,4,6} | {2,…,12} |
| Note 1: S = 3 is applicable only if dmrs-TypeA-Position = 3 |  |  |  |  |  |  |

When configured with SCS m = 5 or m = 6, the UE does not expect to be scheduled with more than one unicast PDSCH in a slot, by a single DCI scheduling multiple PDSCHs or by multiple DCIs, where multiple DCIs are not associated with CORESETs having different coresetpoolIndex.

When receiving PDSCH scheduled by DCI format 1_1 or 1_2 in PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, if the UE is configured with pdsch-AggregationFactor in pdsch-config, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots. When receiving PDSCH scheduled by DCI format 1_1 or 1_2 in PDCCH with CRC scrambled by CS-RNTI with NDI=0, or PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format 1_1 or 1_2, the same symbol allocation is applied across the pdsch-AggregationFactor, in sps-Config if configured, or across the pdsch-AggregationFactor in pdsch-config otherwise, consecutive slots. The UE may expect that the TB is repeated within each symbol allocation among each of the pdsch-AggregationFactor consecutive slots and the PDSCH is limited to a single transmission layer. For PDSCH scheduled by DCI format 1_1 or 1_2 in PDCCH with CRC scrambled by CS-RNTI with NDI=0, or PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format 1_1 or 1_2, the UE is not expected to be configured with the time duration for the reception of pdsch-AggregationFactor repetitions, in sps-Config if configured, or across the pdsch-AggregationFactor in pdsch-config otherwise, larger than the time duration derived by the periodicity P obtained from the corresponding sps-Config. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, …pdsch-AggregationFactor -1, is determined according to table 5.1.2.1-2 and "rvid indicated by the DCI scheduling the PDSCH" in table 5.1.2.1-2 is assumed to be 0 for PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format 1_1 or 1_2.

When receiving PDSCH scheduled by DCI format 4_1, or 4_2 in PDCCH with CRC scrambled by G-RNTI, if the UE is configured with pdsch-AggregationFactor in the MBS-RNTI-SpecificConfig associated with the corresponding G-RNTI, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots. When receiving PDSCH scheduled by DCI format 4_1 or 4_2 for multicast reception in PDCCH with CRC scrambled by G-CS-RNTI, or PDSCH without corresponding PDCCH transmission using associated SPS-Config and activated by the DCI format 4_1 or 4_2 in PDCCH with CRC scrambled by G-CS-RNTI, the same symbol allocation is applied across the pdsch-AggregationFactor, in associated SPS-Config if configured, or 1 otherwise, consecutive slots. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, …pdsch-AggregationFactor -1, is determined according to table 5.1.2.1-2 and "rvid indicated by the DCI scheduling the PDSCH" in table 5.1.2.1-2 is assumed to be 0 for PDSCH scheduled without corresponding PDCCH transmission using SPS-Config and activated by DCI format 4_1 or 4_2. When receiving PDSCH scheduled by DCI format 4_0 in PDCCH with CRC scrambled by G-RNTI for MTCH, if the UE is configured with pdsch-AggregationFactor in the PDSCH-ConfigPTM, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots, and the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, …pdsch-AggregationFactor -1, is determined according to table 5.1.2.1-2.

When receiving PDSCH scheduled by DCI in PDCCH with CRC scrambled by G-CS-RNTI for multicast reception or G-RNTI, if the DCI field 'Time domain resource assignment' indicates an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation in the pdsch-ConfigMulticast or PDSCH-Config-Broadcast, the same SLIV is applied for all PDSCH transmission occasions across the repetitionNumber consecutive slots. When receiving PDSCH scheduled without corresponding PDCCH transmission using associated SPS-Config and activated by DCI in PDCCH with CRC scrambled by G-CS-RNTI for multicast reception, if the DCI field 'Time domain resource assignment' of the activating DCI indicates an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation in the pdsch-ConfigMulticast, the same SLIV is applied for all PDSCH transmission occasions across the repetitionNumber consecutive slots.

If a UE is configured with higher layer parameter repetitionNumber or if the UE is configured by repetitionScheme set to one of ' fdmSchemeA', ' fdmSchemeB' and 'tdmSchemeA', the UE does not expect to be configured with pdsch-AggregationFactor for the same PDSCH.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PDSCH, the UE does not expect to be configured with higher layer parameter repetitionNumber in pdsch-TimeDomainAllocationListForMultiPDSCH-r17.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, the UE does not apply pdsch-AggregationFactor in PDSCH-config, if configured, to DCI format 1_1 on the DL BWP of the serving cell.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, when any two DL DCIs end in the same symbol and at least one of the DCIs schedules multiple PDSCHs, the UE does not expect that the scheduled PDSCH(s) by the two DCIs  have overlapping spans, where the span associated with a DCI is defined from the beginning of the first scheduled PDSCH or up to the end of the last scheduled PDSCH.

Table 5.1.2.1-2: Applied redundancy version when pdsch-AggregationFactor is present

| rvid indicated by the DCI scheduling the PDSCH | rvid to be applied to nth transmission occasion |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

A PDSCH reception in a slot of a multi-slot PDSCH reception is omitted according to the conditions in clause 11.1 and clause 17.2 of [6, TS38.213].

The UE is not expected to receive a PDSCH with mapping type A in a slot, if the PDCCH scheduling the PDSCH was received in the same slot and was not contained within the first three symbols of the slot. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], if the two PDCCH candidates scheduling the PDSCH with mapping Type A were received in the same slot as the PDSCH, both PDCCH candidates are expected to be contained within the first three symbols of the slot.

The UE is not expected to receive a PDSCH with mapping type B in a slot, if the first symbol of the PDCCH scheduling the PDSCH was received in a later symbol than the first symbol indicated in the PDSCH time domain resource allocation. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], the UE is not expected to receive a PDSCH with mapping type B in a slot, if the first symbol of the PDCCH candidate that starts later in time scheduling the PDSCH was received in a later symbol than the first symbol indicated in the PDSCH time domain resource allocation.

When the UE is configured with minimumSchedulingOffsetK0 in an active DL BWP it applies a minimum scheduling offset restriction indicated by the 'Minimum applicable scheduling offset indicator' field in DCI format 1_1 or DCI format 0_1 if the same field is available. When the UE is configured with minimumSchedulingOffsetK0 in an active DL BWP and it has not received 'Minimum applicable scheduling offset indicator' field in DCI format 0_1 or 1_1, the UE shall apply a minimum scheduling offset restriction indicated based on 'Minimum applicable scheduling offset indicator' value '0'. When the minimum scheduling offset restriction is applied the UE is not expected to be scheduled with a DCI in slot n to receive a PDSCH scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI with K0 smaller than  K0min⋅2μ'2μ, where K0min and μ are the applied minimum scheduling offset restriction and the numerology of the active DL BWP of the scheduled cell when receiving the DCI in slot n, respectively, and μ' is the numerology of the new active DL BWP in case of active DL BWP change in the scheduled cell and is equal to μ, otherwise. The minimum scheduling offset restriction is not applied when PDSCH transmission is scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI in common search space associated with CORESET0 and default PDSCH time domain resource allocation is used, in the search space set provided by recoverySearchSpaceId when monitoring PDCCH as described in [6, TS 38.213] or when PDSCH transmission is scheduled with SI-RNTI, MSGB-RNTI or RA-RNTI. The application delay of the change of the minimum scheduling offset restriction is determined in Clause 5.3.1.

The UE is not expected to be configured with referenceOfSLIVDCI-1-2 for serving cells configured for cross-carrier scheduling with a scheduling cell of a different downlink SCS configuration.

When a UE is configured by the higher layer parameter repetitionScheme set to 'tdmSchemeA' and indicated DM-RS port(s) within one CDM group in the DCI field 'Antenna Port(s)', the number of PDSCH transmission occasions is derived by the number of TCI states indicated by the DCI field 'Transmission Configuration Indication' of the scheduling DCI.

-	If two TCI states are indicated by the DCI field 'Transmission Configuration Indication', the UE is expected to receive two PDSCH transmission occasions, where the first TCI state is applied to the first PDSCH transmission occasion and resource allocation in time domain for the first PDSCH transmission occasion follows Clause 5.1.2.1. The second TCI state is applied to the second PDSCH transmission occasion, and the second PDSCH transmission occasion shall have the same number of symbols as the first PDSCH transmission occasion. If the UE is configured by the higher layers with a value K in StartingSymbolOffsetK, it shall determine that the first symbol of the second PDSCH transmission occasion starts after K symbols from the last symbol of the first PDSCH transmission occasion. If the value K is not configured via the higher layer parameter StartingSymbolOffsetK, K  = 0 shall be assumed by the UE. The UE is not expected to receive more than two PDSCH transmission layers for each PDSCH transmission occasion. For two PDSCH transmission occasions, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n=0, 1 applied respectively to the first and second TCI state. The UE expects the PDSCH mapping type indicated by DCI field 'Time domain resource assignment' to be mapping type B, and the indicated PDSCH mapping type is applied to both PDSCH transmission occasions.

-	Otherwise, the UE is expected to receive a single PDSCH transmission occasion, and the resource allocation in the time domain follows Clause 5.1.2.1.

When a UE configured by the higher layer parameter PDSCH-config that indicates at least one entry contains repetitionNumber in PDSCH-TimeDomainResourceAllocation,

-	If two TCI states are indicated by the DCI field 'Transmission Configuration Indication' together with the DCI field 'Time domain resource assignment' indicating an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation and DM-RS port(s) within one CDM group in the DCI field 'Antenna Port(s)', the same SLIV is applied for all PDSCH transmission occasions across the repetitionNumber consecutive slots, the first TCI state is applied to the first PDSCH transmission occasion and resource allocation in time domain for the first PDSCH transmission occasion follows Clause 5.1.2.1.

When the value indicated by repetitionNumber in PDSCH-TimeDomainResourceAllocation equals to two, the second TCI state is applied to the second PDSCH transmission occasion. When the value indicated by repetitionNumber in PDSCH-TimeDomainResourceAllocation is larger than two, the UE may be further configured to enable cyclicMapping or sequenticalMapping in tciMapping.

-	When cyclicMapping is enabled, the first and second TCI states are applied to the first and second PDSCH transmission occasions, respectively, and the same TCI mapping pattern continues to the remaining PDSCH transmission occasions.

-	When sequenticalMapping is enabled, first TCI state is applied to the first and second PDSCH transmission occasions, and the second TCI state is applied to the third and fourth PDSCH transmission occasions, and the same TCI mapping pattern continues to the remaining PDSCH transmission occasions.

The UE may expect that each PDSCH transmission occasion is limited to two transmission layers. For all PDSCH transmission occasions associated with the first TCI state, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n is counted only considering PDSCH transmission occasions associated with the first TCI state. The redundancy version for PDSCH transmission occasions associated with the second TCI state is derived according to Table 5.1.2.1-3, where additional shifting operation for each redundancy version rvs is configured by higher layer parameter sequenceOffsetforRV and n is counted only considering PDSCH transmission occasions associated with the second TCI state.

Table 5.1.2.1-3: Applied redundancy version for the second TCI state when sequenceOffsetforRV is present

| rvid indicated by the DCI scheduling the PDSCH | rvid to be applied to nth transmission occasion with second TCI state |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | (0+ rvs) mod 4 | (2+ rvs) mod 4 | (3+ rvs) mod 4 | (1+ rvs) mod 4 |
| 2 | (2+ rvs) mod 4 | (3+ rvs) mod 4 | (1+ rvs) mod 4 | (0+ rvs) mod 4 |
| 3 | (3+ rvs) mod 4 | (1+ rvs) mod 4 | (0+ rvs) mod 4 | (2+ rvs) mod 4 |
| 1 | (1+ rvs) mod 4 | (0+ rvs) mod 4 | (2+ rvs) mod 4 | (3+ rvs) mod 4 |

-	If one TCI state is indicated by the DCI field 'Transmission Configuration Indication' together with the DCI field 'Time domain resource assignment' indicating an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation and DM-RS port(s) within one CDM group in the DCI field 'Antenna Port(s)', the same SLIV is applied for all PDSCH transmission occasions across the repetitionNumber consecutive slots, the first PDSCH transmission occasion follows Clause 5.1.2.1, the same TCI state is applied to all PDSCH transmission occasions. The UE may expect that each PDSCH transmission occasion is limited to two transmission layers. For all PDSCH transmission occasions, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n is counted considering PDSCH transmission occasions.

-	Otherwise, the UE is expected to receive a single PDSCH transmission occasion, and the resource allocation in the time domain follows Clause 5.1.2.1.

For [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] in pdsch-Config each PDSCH has a separate SLIV, mapping type and K0. The number of scheduled PDSCHs is signalled by the number of indicated SLIVs in the row of the pdsch-TimeDomainAllocationListForMultiPDSCH-r17 signalled in DCI format 1_1.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, and the UE is indicated re-transmission of PDSCH corresponding to a DL SPS by DCI format 1_1, the UE does not expect that the number of indicated SLIVs in the row of the pdsch-TimeDomainAllocationListForMultiPDSCH-r17 by the DCI is more than one.

##### 5.1.2.1.1	Determination of the resource allocation table to be used for PDSCH

Table 5.1.2.1.1-1 and Table 5.1.2.1.1-1A define which PDSCH time domain resource allocation configuration to apply. Either a default PDSCH time domain allocation A, B or C according to tables 5.1.2.1.1-2, 5.1.2.1.1-3, 5.1.2.1.1-4 and 5.1.2.1.1-5 is applied, or the higher layer configured pdsch-TimeDomainAllocationList or [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] or pdsch-TimeDomainAllocationListDCI-1-2 is applied. For operation with shared spectrum channel access in frequency range 1, as described in [16, TS 37.213], UE reinterprets S and L in row 9 of Table 5.1.2.1.1-2 as S=6 and L=7.

Table 5.1.2.1.1-1: Applicable PDSCH time domain resource allocation for DCI formats 1_0, 1_1, 4_0, 4_1 and 4_2

| RNTI | PDCCH search space | SS/PBCH block and CORESET multiplexing pattern | PDSCH-ConfigCommon includes pdsch-TimeDomainAllocationList | PDSCH-Config includes pdsch-TimeDomainAllocationList | pdsch-ConfigMCCH / pdsch-ConfigMTCH includes pdsch-TimeDomainAllocationList Or pdsch-ConfigMulticast includes pdsch-TimeDomainAllocationList | PDSCH-Config includes pdsch-TimeDomainAllocationListForMultiPDSCH-r17 | PDSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SI-RNTI | Type0 common | 1 | - | - | - | - | Default A for normal CP |
|  |  | 2 | - | - | - | - | Default B |
|  |  | 3 | - | - | - | - | Default C |
| SI-RNTI | Type0A common | 1 | No | - | - | - | Default A |
|  |  | 2 | No | - | - | - | Default B |
|  |  | 3 | No | - | - | - | Default C |
|  |  | 1,2,3 | Yes | - | - | - | Pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| RA-RNTI, MSGB-RNTI, TC-RNTI | Type1 common | 1,2,3 | No | - | - | - | Default A |
|  |  | 1,2,3 | Yes | - | - | - | Pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| P-RNTI | Type2 common | 1 | No | - | - | - | Default A |
|  |  | 2 | No | - | - | - | Default B |
|  |  | 3 | No | - | - | - | Default C |
|  |  | 1,2,3 | Yes | - | - | - | Pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| MCCH-RNTI | Type 0/0B common for broadcast | 1 | No | - | No | - | Default A |
|  |  | 2 | No | - | No | - | Default B |
|  |  | 3 | No | - | No | - | Default C |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigMCCH |
| G-RNTI for broadcast | Type 0/0B common for broadcast | 1 | No | - | No | - | Default A |
|  |  | 2 | No | - | No | - | Default B |
|  |  | 3 | No | - | No | - | Default C |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigMTCH, if configured, otherwise TimeDomainAllocationList provided in pdsch-ConfigMCCH |
| C-RNTI, MCS-C-RNTI, CS-RNTI | Any common search space associated with CORESET 0 | 1, 2, 3 | No | - | - | - | Default A |
|  |  | 1, 2, 3 | Yes | - | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| C-RNTI, MCS-C-RNTI, CS-RNTI | Any common search space not associated with CORESET 0 UE specific search space | 1,2,3 | No | No | - | - | Default A |
|  |  | 1,2,3 | Yes | No | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | Yes | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-Config |
|  |  | 1,2,3 | No/Yes | - | - | Yes | pdsch-TimeDomainAllocationListForMultiPDSCH-r17 provided in PDSCH-Config (Note 2) |
| G-RNTI, G-CS-RNTI (for multicast) | Type-X common search space for multiast | 1,2,3 | No | - | No | - | Default A |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon (Note 1) |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigMulticast (Note 1) |
| Note 1: For a UE that supports multicast, the same TDRA table applies to all G-RNTIs and G-CS-RNTIs (configured for multicast) if configured on a given serving cell. Note 2: If pdsch-TimeDomainAllocationListForMultiPDSCH-r17 is provided, it is applicable to DCI format 1_1 only. |  |  |  |  |  |  |  |

<omitted text>

#### 5.1.2.2	Resource allocation in frequency domain

Two downlink resource allocation schemes, type 0 and type 1, are supported. The UE shall assume that when the scheduling grant is received with DCI format 1_0, 4_0 or 4_1 then downlink resource allocation type 1 is used.

If the scheduling DCI is configured to indicate the downlink resource allocation type as part of the 'Frequency domain resource assignment' field by setting a higher layer parameter resourceAllocation in PDSCH-Config to 'dynamicSwitch', for DCI format 1_1 or setting a higher layer parameter resourceAllocationDCI-1-2 in PDSCH-Config to 'dynamicSwitch' for DCI format 1_2 or setting a higher layer parameter resourceAllocation in pdsch-ConfigMulticast  to 'dynamicSwitch' for DCI format 4_2, the UE shall use downlink resource allocation type 0 or type 1 as defined by this DCI field. Otherwise the UE shall use the downlink frequency resource allocation type as defined by the higher layer parameter resourceAllocation in PDSCH-Config for DCI format 1_1 or by the higher layer parameter resourceAllocationDCI-1-2 for DCI format 1_2 or by the higher layer parameter resourceAllocation in pdsch-ConfigMulticast for DCI format 4_2.

If a bandwidth part indicator field is not configured in the scheduling DCI or the UE does not support active BWP change via DCI, the RB indexing for downlink type 0 and type 1 resource allocation is determined within the UE's active bandwidth part. If a bandwidth part indicator field is configured in the scheduling DCI and the UE supports active BWP change via DCI, the RB indexing for downlink type 0 and type 1 resource allocation is determined within the UE's bandwidth part indicated by bandwidth part indicator field value in the DCI. The UE shall upon detection of PDCCH intended for the UE determine first the downlink bandwidth part and then the resource allocation within the bandwidth part.

For a PDSCH scheduled with a DCI format 1_0 in any type of PDCCH common search space, regardless of which bandwidth part is the active bandwidth part, RB numbering starts from the lowest RB of the CORESET in which the DCI was received; otherwise RB numbering starts from the lowest RB in the determined downlink bandwidth part. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the downlink RB set of a PDSCH when scheduled by DCI format 1_0, the CORESET with lower ID among two CORESETs associated with two PDCCH candidates is used.

##### 5.1.2.2.1	Downlink resource allocation type 0

In downlink resource allocation of type 0, the resource block assignment information includes a bitmap indicating the Resource Block Groups (RBGs) that are allocated to the scheduled UE where a RBG is a set of consecutive virtual resource blocks defined by higher layer parameter rbg-Size configured by PDSCH-Config and the size of the bandwidth part as defined in Table 5.1.2.2.1-1.

Table 5.1.2.2.1-1: Nominal RBG size P

| Bandwidth Part Size | Configuration 1 | Configuration 2 |
| --- | --- | --- |
| 1 – 36 | 2 | 4 |
| 37 – 72 | 4 | 8 |
| 73 – 144 | 8 | 16 |
| 145 – 275 | 16 | 16 |

The total number of RBGs (NRBG) for a downlink bandwidth part i of size NBWP,isize PRBs is given by NRBG=NBWP,isize+NBWP,istartmodP/P, where

-	the size of the first RBG is RBG0size=P-NBWP,istartmodP,

-	the size of last RBG is RBGlastsize=NBWP,istart+NBWP,isizemodP if NBWP,istart+NBWP,isizemodP>0 and P otherwise,

-	the size of all other RBGs is P.

In downlink resource allocation of type 0 scheduled using a DCI with CRC scrambled by G-RNTI or G-CS-RNTI, the resource block assignment information bitmap is calculated based on the description above with the following changes: the parameter NBWP,istart is the starting PRB of the CFR, NBWP,isize is the size of the common frequency resource (CFR) and the value of the higher layer parameter rbg-Size is configured by pdsch-ConfigMulticast.

The bitmap is of size bits with one bitmap bit per RBG such that each RBG is addressable. The RBGs shall be indexed in the order of increasing frequency and starting at the lowest frequency of the bandwidth part. The order of RBG bitmap is such that RBG 0 to RBG are mapped from MSB to LSB. The RBG is allocated to the UE if the corresponding bit value in the bitmap is 1, the RBG is not allocated to the UE otherwise.

<omitted text>

#### 5.1.2.3	Physical resource block (PRB) bundling

The PRB bundling procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of prb-BundlingTypeDCI-1-2 instead of prb-BundlingType as well as vrb-ToPRB-InterleaverDCI-1-2 instead of vrb-ToPRB-Interleaver. The PRB bundling procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_2, by applying the parameters of prb-BundlingType given by pdsch-ConfigMulticast as well as vrb-ToPRB-Interleaver given by pdsch-ConfigMulticast.

A UE may assume that precoding granularity is  consecutive resource blocks in the frequency domain.  can be equal to one of the values among {2, 4, wideband}.

If  is determined as "wideband", the UE is not expected to be scheduled with non-contiguous PRBs and the UE may assume that the same precoding is applied to the allocated resource associated with a same TCI state or a same QCL assumption.

If  is determined as one of the values among {2, 4}, Precoding Resource Block Group (PRGs) partitions the bandwidth part i with  consecutive PRBs. Actual number of consecutive PRBs in each PRG could be one or more.

The first PRG size is given by  and the last PRG size given by  if , and the last PRG size is if . For PDSCH scheduled by PDCCH with DCI scrambled using G-RNTI or G-CS-RNTI, NBWP,istart is the starting PRB of the CFR and NBWP,isize is the CFR.

The UE may assume the same precoding is applied for any downlink contiguous allocation of PRBs in a PRG.

For PDSCH carrying SIB1 scheduled by PDCCH with CRC scrambled by SI-RNTI, a PRG is partitioned from the lowest numbered resource block of CORESET 0 if the corresponding PDCCH is associated with CORESET 0 and Type0-PDCCH common search space and is addressed to SI-RNTI; otherwise, a PRG is partitioned from common resource block 0.

If a UE is scheduled a PDSCH with DCI format 1_0 or 4_0, the UE shall assume that  is equal to 2 PRBs.

When receiving PDSCH scheduled by PDCCH with DCI format 1_1 with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI,  for bandwidth part is equal to 2 PRBs unless configured by the higher layer parameter prb-BundlingType given by PDSCH-Config.

When receiving PDSCH scheduled by PDCCH with DCI format 1_1 with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI, if the higher layer parameter prb-BundlingType is set to 'dynamicBundling', the higher layer parameters bundleSizeSet1 and bundleSizeSet2 configure two sets of  values, the first set can take one or two  values among {2, 4, wideband}, and the second set can take one  value among {2, 4, wideband}.

If the PRB 'bundling size indicator' signalled in DCI format 1_1 as defined in Clause 7.3.1.2.2 of [5, TS 38.212]

-	is set to '0', the UE shall use the  value from the second set of  values when receiving PDSCH scheduled by the same DCI.

-	is set to '1' and one value is configured for the first set of  values, the UE shall use this  value when receiving PDSCH scheduled by the same DCI

-	is set to '1' and two values are configured for the first set of  values as 'n2-wideband' (corresponding to two  values 2 and wideband) or 'n4-wideband' (corresponding to two  values 4 and wideband), the UE shall use the value when receiving PDSCH scheduled by the same DCI as follows:

-	If the scheduled PRBs are contiguous and the size of the scheduled PRBs is larger than ,  is the same as the scheduled bandwidth, otherwise  is set to the remaining configured value of 2 or 4, respectively.

When receiving PDSCH scheduled by PDCCH with DCI format 1_1 with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI, if the higher layer parameter prb-BundlingType is set to 'staticBundling', the  value is configured with the single value indicated by the higher layer parameter bundleSize.

When a UE is configured with nominal RBG size P=2 for bandwidth part i according to Clause 5.1.2.2.1, or when a UE is configured with interleaving unit of 2 for VRB to PRB mapping provided by the higher layer parameter vrb-ToPRB-Interleaver given by PDSCH-Config for bandwidth part i, the UE is not expected to be configured with = 4.

For a UE configured by the higher layer parameter repetitionScheme set to 'fdmSchemeA' or 'fdmSchemeB', and when the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within one CDM group in the DCI field 'Antenna Port(s)',

-	If  is determined as "wideband", the first nPRB2 PRBs are assigned to the first TCI state and the remaining nPRB2 PRBs are assigned to the second TCI state, where nPRB is the total number of allocated PRBs for the UE.

-	If  is determined as one of the values among {2, 4}, even PRGs within the allocated frequency domain resources are assigned to the first TCI state and odd PRGs within the allocated frequency domain resources are assigned to the second TCI state, wherein the PRGs are numbered continuously in increasing order with the first PRG index equal to 0.

-	The UE is not expected to receive more than two PDSCH transmission layers for each PDSCH transmission occasion.

For a UE configured by the higher layer parameter repetitionScheme set to 'fdmSchemeB', and when the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within one CDM group in the DCI field 'Antenna Port(s)', each PDSCH transmission occasion shall follow the Clause 7.3.1 of [4, TS 38.211] with the mapping to resource elements determined by the assigned PRBs for corresponding TCI state of the PDSCH transmission occasion, and the UE shall only expect at most two code blocks per PDSCH transmission occasion when a single transmission layer is scheduled and a single code block per PDSCH transmission occasion when two transmission layers are scheduled. For two PDSCH transmission occasions, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n=0, 1 are applied to the first and second TCI state, respectively.

<omitted text>

#### 5.1.3.1	Modulation order and target code rate determination

For the PDSCH scheduled by a PDCCH with DCI format 1_0, format 1_1, format 1_2, format 4_0, format 4_1 or format 4_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, SI-RNTI, RA-RNTI, MSGB-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI or P-RNTI, or for the PDSCH scheduled without corresponding PDCCH transmissions using the higher-layer-provided PDSCH configuration SPS-Config,

if the higher layer parameter mcs-Table-r17 given by PDSCH-Config is set to 'qam1024', and the PDSCH is scheduled by a PDCCH with DCI format 1_1 with CRC scrambled by C-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-4 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif mcs-TableDCI-1-2-r17 given by PDSCH-Config is set to 'qam1024', and the PDSCH is scheduled by a PDCCH with DCI format 1_2 with CRC scrambled by C-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-4 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the higher layer parameter mcs-TableDCI-1-2 given by PDSCH-Config is set to 'qam256', and the PDSCH is scheduled by a PDCCH with DCI format 1_2 with CRC scrambled by C-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with MCS-C-RNTI, the higher layer parameter mcs-TableDCI-1-2 given by PDSCH-Config is set to 'qam64LowSE', and the PDSCH is scheduled by a PDCCH with DCI format 1_2 scrambled by C-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the higher layer parameter mcs-Table given by PDSCH-Config is set to 'qam256', and the PDSCH is scheduled by a PDCCH with DCI format 1_1 with CRC scrambled by C-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the higher layer parameter mcs-Table given by pdsch-ConfigMulticast is set to 'qam256', and the PDSCH is scheduled by a PDCCH with DCI format 4_1 or 4_2 with CRC scrambled by G-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the higher layer parameter mcs-Table given by pdsch-ConfigMCCH and pdsch-ConfigMTCH is set to 'qam256', and the PDSCH is scheduled by a PDCCH with DCI format 4_0 with CRC scrambled by MCCH-RNTI or G-RNTI for MTCH

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the higher layer parameter mcs-Table given by pdsch-ConfigMulticast is set to 'qam64LowSE', and the PDSCH is scheduled by a PDCCH with DCI format 4_1 or 4_2 with CRC scrambled by G-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with MCS-C-RNTI, the higher layer parameter mcs-Table given by PDSCH-Config is set to 'qam64LowSE', and the PDSCH is scheduled by a PDCCH with a DCI format other than DCI format 1_2 in a UE-specific search space with CRC scrambled by C-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is configured with MCS-C-RNTI, and the PDSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with the higher layer parameter mcs-Table given by SPS-config, and the higher layer parameter mcs-Table-r17 given by PDSCH-Config is set to 'qam1024',

-	if the PDSCH is scheduled by a PDCCH with DCI format 1_1 with CRC scrambled by CS-RNTI or

-	if the PDSCH with SPS activated by DCI format 1_1 is scheduled without corresponding PDCCH transmission using SPS-Config,

-	the UE shall use IMCS and Table 5.1.3.1-4 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with the higher layer parameter mcs-Table given by SPS-config, and the higher layer parameter mcs-TableDCI-1-2-r17 given by PDSCH-Config is set to 'qam1024',

-	if the PDSCH is scheduled by a PDCCH with DCI format 1_2 with CRC scrambled by CS-RNTI or

-	if the PDSCH with SPS activated by DCI format 1_2 is scheduled without corresponding PDCCH transmission using SPS-Config,

-	the UE shall use IMCS and Table 5.1.3.1-4 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with the higher layer parameter mcs-Table given by SPS-config, and the higher layer parameter mcs-TableDCI-1-2 given by PDSCH-Config is set to 'qam256',

-	if the PDSCH is scheduled by a PDCCH with DCI format 1_2 with CRC scrambled by CS-RNTI or

-	if the PDSCH with SPS activated by DCI format 1_2 is scheduled without corresponding PDCCH transmission using SPS-Config,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with the higher layer parameter mcs-Table given by SPS-Config, and the higher layer parameter mcs-Table given by PDSCH-Config is set to 'qam256',

-	if the PDSCH is scheduled by a PDCCH with DCI format 1_1 with CRC scrambled by CS-RNTI or

-	if the PDSCH with SPS activated by DCI format 1_1 is scheduled without corresponding PDCCH transmission using SPS-Config,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is configured with the higher layer parameter mcs-Table given by SPS-Config set to 'qam64LowSE'

-	if the PDSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or

-	if the PDSCH is scheduled without corresponding PDCCH transmission using SPS-Config,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is configured with the higher layer parameter mcs-Table given by SPS-Config or mcs-Table of pdsch-ConfigMulticast in the same CFR-ConfigMulticast set to 'qam64LowSE'

-	if the GC-PDSCH is scheduled by a GC-PDCCH with CRC scrambled by G-CS-RNTI or

-	if the GC-PDSCH is scheduled without corresponding GC-PDCCH transmission using SPS-Config,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

else

-	the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

end

#### 5.1.3.2	Transport block size determination

In case the higher layer parameter maxNrofCodeWordsScheduledByDCI in PDSCH-config indicates that two codeword transmission is enabled, then one of the two transport blocks is disabled by DCI format 1_1 if IMCS = 26 and if rvid = 1 for the corresponding transport block. In case the higher layer parameter maxNrofCodeWordsScheduledByDCI in pdsch-ConfigMulticast indicates that two codeword transmission is enabled, then one of the two transport blocks is disabled by DCI format 4_2 if IMCS = 26 and if rvid = 1 for the corresponding transport block. When the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], either the first or the second transport block of all scheduled PDSCHs is disabled by the DCI format 1_1 if IMCS = 26 and if rvid = 2 for the corresponding transport block of all scheduled PDSCHs. If both transport blocks are enabled, transport block 1 and 2 are mapped to codeword 0 and 1 respectively. If only one transport block is enabled, then the enabled transport block is always mapped to the first codeword.

For the PDSCH assigned by a PDCCH with DCI format 1_0, 1_1, 4_0, 4_1, 4_2 or 1_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI or SI-RNTI, if Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤27, else if Table 5.1.3.1-4 is used and 0≤IMCS≤26 or a table other than Table 5.1.3.1-2 and Table 5.1.3.1-4 is used and 0 ≤ IMCS ≤28, the UE shall, except if the transport block is disabled in DCI format 1_1, first determine the TBS as specified below:

1)	The UE shall first determine the number of REs (NRE) NRE)within the slot.

-	A UE first determines the number of REs allocated for PDSCH within a PRB () by , where is the number of subcarriers in a physical resource block, Nsymbslot is the number of symbols of the PDSCH allocation within the slot, NDMRSPRB is the number of REs for DM-RS per PRB in the scheduled duration including the overhead of the DM-RS CDM groups without data, as indicated by DCI format 1_1 or format 1_2 or as described for format 1_0 in Clause 5.1.6.2, and  NohPRBis the overhead configured by higher layer parameter xOverhead in PDSCH-ServingCellConfig. If the xOverhead in PDSCH-ServingCellconfig is not configured (a value from 6, 12, or 18), the  is set to 0. If the PDSCH is scheduled by PDCCH with a CRC scrambled by SI-RNTI, RA-RNTI, MSGB-RNTI or P-RNTI, NohPRB is assumed to be 0. If the PDSCH is scheduled by PDCCH with a CRC scrambled by G-RNTI or G-CS-RNTI or PDSCH without PDCCH is activated by PDCCH with a CRC scrambled by G-CS-RNTI, NohPRB NohPRBis the overhead configured by higher layer parameter xOverhead-Multicast in pdsch-ConfigMulticast. If the xOverhead-Multicast in pdsch-ConfigMulticast is not configured, the NohPRB is set to 0.

-	A UE determines the total number of REs allocated for PDSCH ()NRE) by NRE= NRE'* nPRB, where nPRB is the total number of allocated PRBs for the UE.

2)	Unquantized intermediate variable (Ninfo) TBStemp) is obtained by TBStemp= NRE*R*Qm*ʋ.

If

Use step 3 as the next step of the TBS determination

else

Use step 4 as the next step of the TBS determination

end if

3)	When , TBS is determined as follows

-	quantized intermediate number of information bits , where .

-	use Table 5.1.3.2-1 find the closest TBS that is not less than .

Table 5.1.3.2-1: TBS for

| Index | TBS | Index | TBS | Index | TBS | Index | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 24 | 31 | 336 | 61 | 1288 | 91 | 3624 |
| 2 | 32 | 32 | 352 | 62 | 1320 | 92 | 3752 |
| 3 | 40 | 33 | 368 | 63 | 1352 | 93 | 3824 |
| 4 | 48 | 34 | 384 | 64 | 1416 |  |  |
| 5 | 56 | 35 | 408 | 65 | 1480 |  |  |
| 6 | 64 | 36 | 432 | 66 | 1544 |  |  |
| 7 | 72 | 37 | 456 | 67 | 1608 |  |  |
| 8 | 80 | 38 | 480 | 68 | 1672 |  |  |
| 9 | 88 | 39 | 504 | 69 | 1736 |  |  |
| 10 | 96 | 40 | 528 | 70 | 1800 |  |  |
| 11 | 104 | 41 | 552 | 71 | 1864 |  |  |
| 12 | 112 | 42 | 576 | 72 | 1928 |  |  |
| 13 | 120 | 43 | 608 | 73 | 2024 |  |  |
| 14 | 128 | 44 | 640 | 74 | 2088 |  |  |
| 15 | 136 | 45 | 672 | 75 | 2152 |  |  |
| 16 | 144 | 46 | 704 | 76 | 2216 |  |  |
| 17 | 152 | 47 | 736 | 77 | 2280 |  |  |
| 18 | 160 | 48 | 768 | 78 | 2408 |  |  |
| 19 | 168 | 49 | 808 | 79 | 2472 |  |  |
| 20 | 176 | 50 | 848 | 80 | 2536 |  |  |
| 21 | 184 | 51 | 888 | 81 | 2600 |  |  |
| 22 | 192 | 52 | 928 | 82 | 2664 |  |  |
| 23 | 208 | 53 | 984 | 83 | 2728 |  |  |
| 24 | 224 | 54 | 1032 | 84 | 2792 |  |  |
| 25 | 240 | 55 | 1064 | 85 | 2856 |  |  |
| 26 | 256 | 56 | 1128 | 86 | 2976 |  |  |
| 27 | 272 | 57 | 1160 | 87 | 3104 |  |  |
| 28 | 288 | 58 | 1192 | 88 | 3240 |  |  |
| 29 | 304 | 59 | 1224 | 89 | 3368 |  |  |
| 30 | 320 | 60 | 1256 | 90 | 3496 |  |  |

4)	When , TBS is determined as follows.

-	quantized intermediate number of information bits , where and ties in the round function are broken towards the next largest integer.

-	if

, where

else

if

, where

else

end if

end if

else if Table 5.1.3.1-2 is used and ,

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using 0 ≤ IMCS ≤27. If there is no PDCCH for the same transport block using 0 ≤ IMCS ≤27, and if the initial PDSCH for the same transport block is semi-persistently scheduled, the TBS shall be determined from the most recent semi-persistent scheduling assignment PDCCH.

else if Table 5.1.3.1-4 is used and 27≤IMCS≤31,

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using 0≤IMCS≤260 ≤ IMCS ≤27. If there is no PDCCH for the same transport block using 0≤IMCS≤260 ≤ IMCS ≤27, and if the initial PDSCH for the same transport block is semi-persistently scheduled, the TBS shall be determined from the most recent semi-persistent scheduling assignment PDCCH.

else

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using 0 ≤ IMCS ≤28. If there is no PDCCH for the same transport block using 0 ≤ IMCS ≤28, and if the initial PDSCH for the same transport block is semi-persistently scheduled, the TBS shall be determined from the most recent semi-persistent scheduling assignment PDCCH.

The UE is not expected to receive a PDSCH assigned by a PDCCH with CRC scrambled by SI-RNTI with a TBS exceeding 2976 bits.

For a UE configured with the higher layer parameter repetitionScheme set to 'fdmSchemeB' and indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within one CDM group in the DCI field 'Antenna Port(s)', the TBS determination follows the steps 1-4 with the following modification in step 1: a UE determines the total number of REs allocated for PDSCH ()NRE) by NRE= NRE'* nPRB, where nPRB is the total number of allocated PRBs corresponding to the first TCI state, and the determined TBS of PDSCH transmission occasion associated with the first TCI state is also applied to the PDSCH transmission occasion associated with the second TCI state. For a UE configured with the higher layer parameter repetitionScheme set to 'tdmSchemeA' and indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within one CDM group in the DCI field 'Antenna Port(s)', the TBS determination follows the steps 1-4 with the following modification in step 1: a UE determines the number of REs allocated for PDSCH within a PRB () by , where  is the number of symbols of the PDSCH allocation within the slot corresponding to the first TCI state, and the determined TBS of PDSCH transmission occasion associated with the first TCI state is also applied to the PDSCH transmission occasion associated with the second TCI state.

For the PDSCH assigned by a PDCCH with DCI format 1_0 with CRC scrambled by P-RNTI, or RA-RNTI, MsgB-RNTI, TBS determination follows the steps 1-4 with the following modification in step 2: a scaling  is applied in the calculation of Ninfo, where the scaling factor is determined based on the TB scaling field in the DCI as in Table 5.1.3.2-2.

Table 5.1.3.2-2: Scaling factor of Ninfo for P-RNTI, RA-RNTI and MSGB-RNTI

| TB scaling field | Scaling factor S |
| --- | --- |
| 00 | 1 |
| 01 | 0.5 |
| 10 | 0.25 |
| 11 |  |

The NDI and HARQ process ID, as signalled on PDCCH, and the TBS, as determined above, shall be reported to higher layers.

<omitted text>

#### 5.1.4.1	PDSCH resource mapping with RB symbol level granularity

The procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying only the parameters of rateMatchPatternGroup1DCI-1-2, rateMatchPatternGroup2DCI-1-2 instead of rateMatchPatternGroup1 and rateMatchPatternGroup2. The procedures for PDSCH scheduled by PDCCH with DCI format 1_0 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_0, by applying only the parameters of rateMatchPatternToAddModList configured in pdsch-ConfigMCCH or pdsch-ConfigMTCH.

The procedures for PDSCH scheduled by PDCCH with DCI format 1_0 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_1, and the procedures for PDSCH scheduled by DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_2 by applying only the parameters of rateMatchPatternToAddModList, rateMatchPatternGroup1 and rateMatchPatternGroup2 configured in pdsch-ConfigMulticast.

A UE may be configured with any of the following higher layer parameters indicating REs declared as not available for PDSCH:

-	rateMatchPatternToAddModList given by PDSCH-Config, by pdsch-ConfigMulticast, by ServingCellConfig or by ServingCellConfigCommon, or by pdsch-ConfigMCCH or pdsch-ConfigMTCH and configuring up to 4 RateMatchPattern(s) per BWP and up to 4 RateMatchPattern(s) per serving-cell. The RateMatchPatterns configured for MBS multicast are counted into the ones that are configured per BWP. The RateMatchPattern(s) configured for MBS broadcast is counted into the ones that are configured per serving-cell. A RateMatchPattern may contain:

-	within a BWP, when provided by PDSCH-Config or pdsch-ConfigMulticast or within a serving cell when provided by ServingCellConfig or ServingCellConfigCommon, or by pdsch-ConfigMCCH or pdsch-ConfigMTCH, a pair of reserved resources with numerology provided by higher layer parameter subcarrierSpacing given by RateMatchPattern when configured per serving cell or by numerology of associated BWP when configured per BWP. The pair of reserved resources are respectively indicated by an RB level bitmap (higher layer parameter resourceBlocks given by RateMatchPattern) with 1RB granularity and a symbol level bitmap spanning one or two slots (higher layer parameters symbolsInResourceBlock given by RateMatchPattern) for which the reserved RBs apply. A bit value equal to 1 in the RB and symbol level bitmaps indicates that the corresponding resource is not available for PDSCH. For each pair of RB and symbol level bitmaps, a UE may be configured with a time-domain pattern (higher layer parameter periodicityAndPattern given by RateMatchPattern), where each bit of periodicityAndPattern corresponds to a unit equal to a duration of the symbol level bitmap, and a bit value equal to 1 indicates that the pair is present in the unit. The periodicityAndPattern can be {1, 2, 4, 5, 8, 10, 20 or 40} units long, but maximum of 40 msec. The first symbol of periodicityAndPattern every 40 msec/P periods is a first symbol in frame nf mod 4 = 0, where P is the duration of periodicityAndPattern in units of msec. When periodicityAndPattern is not configured for a pair, for a symbol level bitmap spanning two slots, the bits of the first and second slots correspond respectively to even and odd slots of a radio frame, and for a symbol level bitmap spanning one slot, the bits of the slot correspond to every slot of a radio frame. The pair can be included in one or two groups of resource sets (higher layer parameters rateMatchPatternGroup1 and rateMatchPatternGroup2). The rateMatchPatternToAddModList given by ServingCellConfig or ServingCellConfigCommon configuration in numerology µ applies only to PDSCH of the same numerology µ.

-	within a BWP, a frequency domain resource of a CORESET configured by ControlResourceSet with controlResourceSetId or ControlResourceSetZero and time domain resource determined by the higher layer parameters monitoringSlotPeriodicityAndOffset, duration and monitoringSymbolsWithinSlot of all search-space-sets configured by SearchSpace and time domain resource of search-space-set zero configured by searchSpaceZero associated with the CORESET as well as CORESET duration configured by ControlResourceSet with controlResourceSetId or ControlResourceSetZero. This resource not available for PDSCH can be included in one or two groups of resource sets (higher layer parameters rateMatchPatternGroup1 and rateMatchPatternGroup2).

A configured group rateMatchPatternGroup1 or rateMatchPatternGroup2 contains a list of indices of RateMatchPattern(s) forming a union of resource-sets not available for a PDSCH dynamically if a corresponding bit of the 'Rate matching indicator' field of the DCI format 1_1 scheduling the PDSCH is equal to 1. The REs corresponding to the union of resource-sets configured by RateMatchPattern(s) that are not included in either of the two groups are not available for a PDSCH scheduled by a DCI format 1_0, a PDSCH scheduled by a DCI format 1_1, and PDSCHs with SPS. When receiving a PDSCH scheduled by a DCI format 1_0 or PDSCHs with SPS activated by a DCI format 1_0, the REs corresponding to configured resources in rateMatchPatternGroup1 or rateMatchPatternGroup2 are not available for the scheduled PDSCH or the activated PDSCHs with SPS. When receiving PDSCHs with SPS activated by a DCI format 1_1, the REs corresponding to configured resources in rateMatchPatternGroup1 or rateMatchPatternGroup2 are not available for the PDSCHs with SPS if a corresponding bit of the Rate matching indicator field of the DCI format 1_1 activating the PDSCHs with SPS is equal to 1.

For a bitmap pair included in one or two groups of resource sets, the dynamic indication of availability for PDSCH applies to a set of slot(s) where the rateMatchPatternToAddModList is present among the slots of scheduled PDSCH.

If a UE monitors PDCCH candidates of aggregation levels 8 and 16 with the same starting CCE index in non-interleaved CORESET spanning one OFDM symbol:

-	and if a detected PDCCH scheduling the PDSCH has aggregation level 8, the resources corresponding to the aggregation level 16 PDCCH candidate are not available for the PDSCH;

-	when at least one of the PDCCH candidates of aggregation levels 8 and 16 linked as indicated by higher layer parameter searchSpaceLinking, the PDCCH candidates of aggregation level 16 and any other PDCCH candidate(s) linked with any of the PDCCH candidates of aggregation level 8 and 16 are not available for the PDSCH reception at the UE, if a detected PDCCH scheduling the PDSCH is associated with the PDCCH candidates of aggregation level 8 or 16.

If a PDSCH scheduled by a PDCCH would overlap with resources in the CORESET containing the PDCCH, the resources corresponding to a union of the detected PDCCH that scheduled the PDSCH and associated PDCCH DM-RS are not available for the PDSCH. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], the resources corresponding to a union of the two PDCCH candidates scheduling the PDSCH and the associated PDCCH DM-RS are not available for the PDSCH. When precoderGranularity configured in a CORESET where the PDCCH was detected is set to 'allContiguousRBs', the associated PDCCH DM-RS are DM-RS in all REGs of the CORESET. Otherwise, the associated DM-RS are the DM-RS in REGs of the PDCCH.

#### 5.1.4.2	PDSCH resource mapping with RE level granularity

The procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 instead of aperiodic-ZP-CSI-RS-ResourceSetsToAddModList.

The procedures for PDSCH scheduled by PDCCH with DCI format 1_0 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_1 and the procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_2, by applying the parameters of aperiodicZP-CSI-RS-ResourceSetsToAddModList in pdsch-ConfigMulticast instead of aperiodic-ZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config.

A UE may be configured with any of the following higher layer parameters:

-	REs indicated by the 'RateMatchPatternLTE-CRS' in lte-CRS-ToMatchAround in ServingCellConfig or ServingCellConfigCommon configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH.

-	REs indicated by 'RateMatchPatternLTE-CRS' in lte-CRS-PatternList1-r16 in ServingCellConfig configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH.

-	For the UE for broadcast reception, REs indicated by 'RateMatchPatternLTE-CRS' in pdsch-ConfigMCCH or pdsch-ConfigMTCH configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH.

-	Each RateMatchPatternLTE-CRS configuration contains v-Shift consisting of LTE-CRS-vshift(s), nrofCRS-Ports consisting of LTE-CRS antenna ports 1, 2 or 4 ports, carrierFreqDL representing the offset in units of 15 kHz subcarriers from (reference) point A to the LTE carrier centre subcarrier location, carrierBandwidthDL representing the LTE carrier bandwidth, and may also configure mbsfn-SubframeConfigList representing MBSFN subframe configuration. A UE determines the CRS position within the slot according to Clause 6.10.1.2 in [15, TS 36.211], where slot corresponds to LTE subframe.

-	If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH:

-	if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList1-r16 if the PDSCH is associated with coresetPoolIndex set to '0', or the CRS pattern(s) in lte-CRS-PatternList2-r16 if PDSCH is associated with coresetPoolIndex set to '1';

-	otherwise, REs indicated by lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16, in ServingCellConfig.

-	Within a BWP, the UE can be configured with one or more ZP CSI-RS resource set configuration(s) for aperiodic, semi-persistent and periodic time-domain behaviours (higher layer parameters aperiodic-ZP-CSI-RS-ResourceSetsToAddModList,  sp-ZP-CSI-RS-ResourceSetsToAddModList and p-ZP-CSI-RS-ResourceSet respectively comprised in PDSCH-Config), with each ZP CSI-RS resource set consisting of at most 16 ZP CSI-RS resources (higher layer parameter ZP-CSI-RS-Resource) in numerology of the BWP. The REs indicated by p-ZP-CSI-RS-ResourceSet are declared as not available for PDSCH. The REs indicated by sp-ZP-CSI-RS-ResourceSetsToAddModList and aperiodic-ZP-CSI-RS-ResourceSetsToAddModList are declared as not available for PDSCH when their triggering and activation are applied, respectively. The following parameters are configured via higher layer signaling for each ZP CSI-RS resource configuration:

-	zp-CSI-RS-ResourceId in ZP-CSI-RS-Resource determines ZP CSI-RS resource configuration identity.

-	nrofPorts in CSI-RS-ResourceMapping defines the number of CSI-RS ports, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211].

-	cdm-Type in CSI-RS-ResourceMapping defines CDM values and pattern, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211].

-	resourceMapping in ZP-CSI-RS-Resource defines the OFDM symbol and subcarrier occupancy of the ZP CSI-RS resource within a slot that are given in Clause 7.4.1.5 of [4, TS 38.211].

-	periodicityAndOffset in ZP-CSI-RS-Resource defines the ZP-CSI-RS periodicity and slot offset for periodic/semi-persistent ZP CSI-RS.

-	For the UE in RRC_CONNECTED mode for multicast reception, p-ZP-CSI-RS-ResourceSet can be configured in pdsch-ConfigMulticast for GC-PDSCH rate matching, subject to UE capability. The REs indicated by p-ZP-CSI-RS-ResourceSet are declared as not available for GC-PDSCH. The REs indicated by p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config for unicast do not apply for GC-PDSCH and the REs indicated by p-ZP-CSI-RS-ResourceSet configured in pdsch-ConfigMulticast for multicast do not apply for unicast PDSCH. The total number of periodic ZP-CSI-RS-Resources that a UE can be configured with is the same as for unicast in Rel-16. If p-ZP-CSI-RS-ResourceSet is configured in both PDSCH-Config and pdsch-ConfigMulticast, it is subject to UE capability whether the p-ZP-CSI-RS-ResourceSet configured in pdsch-ConfigMulticast can be different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config.

-	For the UE in RRC_CONNECTED mode for multicast reception, sp-ZP-CSI-RS-ResourceSet can be configured in pdsch-ConfigMulticast for GC-PDSCH rate matching, subject to UE capability. The REs indicated by sp-ZP-CSI-RS-ResourceSet are declared as not available for GC-PDSCH when their triggering and activation delivered by unicast PDSCH are applied. The REs indicated by sp-ZP-CSI-RS-ResourceSet configured in PDSCH-Config for unicast do not apply for GC-PDSCH and the REs indicated by sp-ZP-CSI-RS-ResourceSet configured in pdsch-ConfigMulticast for multicast do not apply for unicast PDSCH. The total number of semi-persistent ZP-CSI-RS-Resources that a UE can be configured with is the same as for unicast.

The UE may be configured with a DCI field for triggering the aperiodic ZP CSI-RS. A list of ZP-CSI-RS-ResourceSet(s), provided by higher layer parameter aperiodic-ZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config, is configured for aperiodic triggering. The maximum number of aperiodic ZP-CSI-RS-ResourceSet(s) configured per BWP is 3. The bit-length of DCI field ZP CSI-RS trigger depends on the number of aperiodic ZP-CSI-RS-ResourceSet(s)configured (up to 2 bits). Each non-zero codepoint of 'ZP CSI-RS' trigger in DCI format 1_1 triggers one aperiodic 'ZP-CSI-RS-ResourceSet' in the list aperiodic-ZP-CSI-RS-ResourceSetsToAddModList by indicating the aperiodic ZP CSI-RS resource set ID. The DCI codepoint '01' triggers the resource set with 'ZP-CSI-RS-ResourceSetId' set to '1', the DCI codepoint '10' triggers the resource set with 'ZP-CSI-RS-ResourceSetId' set to '2', and the DCI codepoint '11' triggers the resource set with 'ZP-CSI-RS-ResourceSetId' set to '3'. Codepoint '00' is reserved for not triggering aperiodic ZP CSI-RS. When receiving PDSCH scheduled by DCI format 1_0 or PDSCHs with SPS activated by DCI format 1_0, the REs corresponding to configured resources in aperiodic-ZP-CSI-RS-ResourceSetsToAddModList or in aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 are available for PDSCH.

When the UE is configured with multi-slot and single-slot PDSCH scheduling, the triggered aperiodic ZP CSI-RS is applied to all the slot(s) of the PDSCH scheduled or the PDSCHs with SPS activated by the PDCCH containing the trigger.

For a UE configured with a list of semi-persistent ZP-CSI-RS-ResourceSet(s) provided by higher layer parameter sp-ZP-CSI-RS-ResourceSetsToAddModList:

-	when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, as described in clause 6.1.3.19 of [10, TS 38.321], for ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on the PDSCH RE mapping corresponding to the activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ2μKmac∙kmac where  is the SCS configuration for the PUCCH and μKmac is the subcarrier spacing configuration for kmac with a value of 0 for frequency range 1, and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided.

-	when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, as described in clause 6.1.3.19 of [10, TS 38.321], for activated ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on cessation of the PDSCH RE mapping corresponding to the de-activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ2μKmac∙kmac where  is the SCS configuration for the PUCCH and μKmac is the subcarrier spacing configuration for kmac with a value of 0 for frequency range 1, and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided.

### 5.1.5	Antenna ports quasi co-location

The UE can be configured with a list of up to M TCI-State configurations within the higher layer parameter PDSCH-Config to decode PDSCH according to a detected PDCCH with DCI intended for the UE and the given serving cell, where M depends on the UE capability maxNumberConfiguredTCIstatesPerCC. Each TCI-State contains parameters for configuring a quasi co-location relationship between one or two downlink reference signals and the DM-RS ports of the PDSCH, the DM-RS port of PDCCH or the CSI-RS port(s) of a CSI-RS resource. The quasi co-location relationship is configured by the higher layer parameter qcl-Type1 for the first DL RS, and qcl-Type2 for the second DL RS (if configured). For the case of two DL RSs, the QCL types shall not be the same, regardless of whether the references are to the same DL RS or different DL RSs. The quasi co-location types corresponding to each DL RS are given by the higher layer parameter qcl-Type in QCL-Info and may take one of the following values:

-	'typeA': {Doppler shift, Doppler spread, average delay, delay spread}

-	'typeB': {Doppler shift, Doppler spread}

-	'typeC': {Doppler shift, average delay}

-	'typeD': {Spatial Rx parameter}

The UE can be configured with a list of up to 128 TCI-State configurations, within the higher layer parameter dl-OrJoint-TCIStateList in PDSCH-Config for providing a reference signal for the quasi co-location for DM-RS of PDSCH and DM-RS of PDCCH in a BWP/CC, for CSI-RS, and to provide a reference, if applicable, for determining UL TX spatial filter for dynamic-grant and configured-grant based PUSCH and PUCCH resource in a BWP/CC, and SRS.

If the TCI-State or UL-TCI-State configurations are absent in a BWP of the CC, the UE can apply the TCI-State or UL-TCI-State configurations from a reference BWP of a reference CC. The UE is not expected to be configured with tci-StatesToAddModList, SpatialRelationInfo or PUCCH-SpatialRelationInfo, except SpatialRelationInfoPos in a CC in a band, if the UE is configured with dl-OrJoint-TCIStateList or UL-TCIState in any CC in the same band. The UE can assume that when the UE is configured with tci-StatesToAddModList in any CC in the CC list configured by simultaneousTCI-UpdateList1-r16, simultaneousTCI-UpdateList2-r16, simultaneousSpatial-UpdatedList1-r16, or simultaneousSpatial-UpdatedList2-r16, the UE is not configured with dl-OrJoint-TCIStateList or UL-TCI-State in any CC within the same band in the CC list.

The UE receives an activation command, as described in clause 6.1.3.14 of [10, TS 38.321] or 6.1.3.47 of [10, TS 38.321], used to map up to 8 TCI states and/or pairs of TCI states, with one TCI state for DL channels/signals and/or one TCI state for UL channels/signals to the codepoints of the DCI field 'Transmission Configuration Indication' for one or for a set of CCs/DL BWPs, and if applicable, for one or for a set of CCs/UL BWPs. When a set of TCI state IDs are activated for a set of CCs/DL BWPs and if applicable, for a set of CCs/UL BWPs, where the applicable list of CCs is determined by the indicated CC in the activation command, the same set of TCI state IDs are applied for all DL and/or UL BWPs in the indicated CCs. If the activation command maps TCI-State and/or UL-TCI-State to only one TCI codepoint, the UE shall apply the indicated TCI-State and/or UL-TCI-State to one or to a set of CCs /DL BWPs, and if applicable, to one or to a set of CCs /UL BWPs once the indicated mapping for the one single TCI codepoint is applied as described in [11, TS 38.133].

When the bwp-id or cell for QCL-TypeA/D source RS in a QCL-Info of the TCI state is not configured, the UE assumes that QCL-TypeA/D source RS is configured in the CC/DL BWP where TCI state applies.

When tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, a UE configured with dl-OrJoint-TCIStateList with activated TCI-State or UL-TCI-State receives DCI format 1_1/1_2 providing indicated TCI-State and/or UL-TCI-State for a CC or all CCs in the same CC list configured by simultaneousU-TCI-UpdateList1-r17, simultaneousU-TCI-UpdateList2-r17, simultaneousU-TCI-UpdateList3-r17, simultaneousU-TCI-UpdateList4-r17. The DCI format 1_1/1_2 can be with or without, if applicable, DL assignment. If the DCI format 1_1/1_2/ is without DL assignment, the UE can assume the following:

-	CS-RNTI is used to scramble the CRC for the DCI

-	The values of the following DCI fields are set as follows:

-	RV = all '1's

-	MCS = all '1's

-	NDI = 0

-	Set to all '0's for FDRA Type 0, or all '1's for FDRA Type 1, or all '0's for dynamicSwitch (same as in Table 10.2-4 of [6, TS 38.213]).

After a UE receives an initial higher layer configuration of dl-OrJoint-TCIStateList with more than one TCI-State and before application of an indicated TCI state from the configured TCI states:

-	The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block the UE identified during the initial access procedure

After a UE receives an initial higher layer configuration of dl-OrJoint-TCIStateList with more than one TCI-State or more than one UL-TCI-State and before application of an indicated TCI state from the configured TCI states:

-	The UE assumes that the UL TX spatial filter, if applicable, for dynamic-grant and configured-grant based PUSCH and PUCCH, and for SRS applying the indicated TCI state, is the same as that for a PUSCH transmission scheduled by a RAR UL grant during the initial access procedure

After a UE receives a higher layer configuration of dl-OrJoint-TCIStateList with more than one TCIState as part of a Reconfiguration with sync procedure as described in [12, TS 38.331] and before applying an indicated TCI state from the configured TCI states:

-	The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH, and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block or the CSI-RS resource the UE identified during the random access procedure initiated by the Reconfiguration with sync procedure as described in [12, TS 38.331].

After a UE receives a higher layer configuration of dl-OrJoint-TCIStateList with more than one TCI-State or more than one TCI-UL-State as part of a Reconfiguration with sync procedure as described in [12, TS 38.331] and before applying an indicated TCI state from the configured TCI states:

-	The UE assumes that the UL TX spatial filter, if applicable, for dynamic-grant and configured-grant based PUSCH and PUCCH, and for SRS applying the indicated TCI state, is the same as that for a PUSCH transmission scheduled by a RAR UL grant during random access procedure initiated by the Reconfiguration with sync procedure as described in [12, TS 38.331].

If a UE receives a higher layer configuration of dl-OrJoint-TCIStateList with a single TCI-State, that can be used as an indicated TCI state, the UE obtains the QCL assumptions from the configured TCI state for DM-RS of PDSCH and DM-RS of PDCCH, and the CSI -RS applying the indicated TCI state.

If a UE receives a higher layer configuration of dl-OrJoint-TCIStateList with a single TCI-State or a single TCI-UL-State, that can be used as an indicated TCI state, the UE determines an UL TX spatial filter, if applicable, from the configured TCI state for dynamic-grant and configured-grant based PUSCH and PUCCH, and SRS applying the indicated TCI state.

When a UE configured with dl-OrJoint-TCIStateList would transmit a PUCCH with HARQ-ACK information or a PUSCH with HARQ-ACK information corresponding to the DCI carrying the TCI State indication and without DL assignment, or corresponding to the PDSCH scheduled by the DCI carrying the TCI State indication, and if the indicated TCI State is different from the previously indicated one, the indicated TCI-State and/or TCI-UL-State should be applied starting from the first slot that is at least  beamAppTime symbols after the last symbol of the PUCCH or the PUSCH. The first slot and the  beamAppTime symbols are both determined on the active BWP with the smallest SCS among the active BWP(s) of the carrier(s) applying the beam indication.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, and the UE is receiving a DCI carrying the TCI-State indication and without DL assignment, the UE does not expect that the number of indicated SLIVs in the row of the pdsch-TimeDomainAllocationListForMultiPDSCH-r17 by the DCI is more than one.

If the UE is configured with SSB-MTC-AddtionalPCI and with PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, the UE receives an activation command for CORESET associated with each coresetPoolIndex, as described in clause 6.1.3.14 of [10, TS 38.321], used to map up to 8 TCI states to the codepoints of the DCI field 'Transmission Configuration Indication' in one CC/DL BWP. When a set of TCI state IDs are activated for a coresetPoolIndex, the activated TCI states corresponding to one coresetPoolIndex is associated with the serving cell physical cell ID and activated TCI states corresponding to another coresetPoolIndex can be associated with another physical cell ID.

When a UE supports two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' the UE may receive an activation command, as described in clause 6.1.3.24 of [10, TS 38.321], the activation command is used to map up to 8 combinations of one or two TCI states to the codepoints of the DCI field 'Transmission Configuration Indication'. The UE is not expected to receive more than 8 TCI states in the activation command.

When the DCI field 'Transmission Configuration Indication' is present in DCI format 1_2 and when the number of codepoints S in the DCI field 'Transmission Configuration Indication' of DCI format 1_2 is smaller than the number of TCI codepoints that are activated by the activation command, as described in clause 6.1.3.14 and 6.1.3.24 of [10, TS38.321], only the first S activated codepoints are applied for DCI format 1_2.

When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ2μKmac∙kmac where  is the SCS configuration for the PUCCH and μKmac is the subcarrier spacing configuration for kmac with a value of 0 for frequency range 1, and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided. If tci-PresentInDCI is set to 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET scheduling the PDSCH, and the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than timeDurationForQCL if applicable, after a UE receives an initial higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the SS/PBCH block determined in the initial access procedure with respect to qcl-Type set to 'typeA', and when applicable, also with respect to qcl-Type set to 'typeD'.

If a UE is configured with the higher layer parameter tci-PresentInDCI that is set as 'enabled' for the CORESET scheduling a PDSCH, the UE assumes that the TCI field is present in the DCI format 1_1 of the PDCCH transmitted on the CORESET. If a UE is configured with the higher layer parameter tci-PresentDCI-1-2 for the CORESET scheduling the PDSCH, the UE assumes that the TCI field with a DCI field size indicated by tci-PresentDCI-1-2 is present in the DCI format 1_2 of the PDCCH transmitted on the CORESET. If a UE is configured with the higher layer parameter tci-PresentInDCI that is set as 'enabled' for the CORESET scheduling the multicast PDSCH, the UE assumes that the TCI field is present in the DCI format 4_2 of the PDCCH transmitted on the CORESET. If the PDSCH is scheduled by a DCI format not having the TCI field present, and the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, where the threshold is based on reported UE capability [13, TS 38.306], for determining PDSCH antenna port quasi co-location, the UE assumes that the TCI state or the QCL assumption for the PDSCH is identical to the TCI state or QCL assumption whichever is applied for the CORESET used for the PDCCH transmission within the active BWP of the serving cell.

When a UE is configured with both sfnSchemePdcch and sfnSchemePdsch scheduled by DCI format 1_0 or by DCI format 1_1/1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable:

-	if the UE supports sfn-DefaultDL-BeamSetup-r17 for DCI scheduling without TCI field, the UE assumes that the TCI state(s) or the QCL assumption(s) for the PDSCH is identical to the TCI state(s) or QCL assumption(s) whichever is applied for the CORESET used for the reception of the DL DCI within the active BWP of the serving cell regardless of the number of active TCI states of the CORESET. If the UE does not support sfn-SchemeA-DynamicSwitching-r17 or sfn-SchemeB-DynamicSwitching-r17, the UE should be activated with the CORESET with two TCI states.

-	else if the UE does not support sfn-DefaultDL-BeamSetup-r17 for DCI scheduling without TCI field, the UE shall expect TCI field present when scheduled by DCI format 1_1/1_2.

When a UE is configured with sfnSchemePdsch and sfnSchemePdcch is not configured, when scheduled by DCI format 1_1/1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, the UE shall expect TCI field present.

For PDSCH scheduled by DCI format 1_0, 1_1, 1_2, when a UE is configured with sfnSchemePdcch set to 'sfnSchemeA' and sfnSchemePdsch is not configured, and there is no TCI codepoint with two TCI states in the activation command, and if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL if applicable and the CORESET which schedules the PDSCH is indicated with two TCI states, the UE assumes that the TCI state or the QCL assumption for the PDSCH is identical to the first TCI state or QCL assumption which is applied for the CORESET used for the PDCCH transmission within the active BWP of the serving cell.

If a PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. For a single slot PDSCH, the indicated TCI state(s) should be based on the activated TCI states in the slot with the scheduled PDSCH. For a multi-slot PDSCH or the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state(s) should be based on the activated TCI states in the first slot with the scheduled PDSCH(s), and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH(s). When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL.

Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the UE is not provided dl-OrJoint-TCIStateList-r17, and if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD',

-	the UE may assume that the DM-RS ports of PDSCH(s) of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers).

-	If a UE is configured with enableDefaultTCI-StatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in different ControlResourceSets,

-	the UE may assume that the DM-RS ports of PDSCH associated with a value of coresetPoolIndex of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of coresetPoolIndex as the PDCCH scheduling that PDSCH, in the latest slot in which one or more CORESETs associated with the same value of coresetPoolIndex as the PDCCH scheduling that PDSCH within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol and they are associated with same value of coresetPoolIndex, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers).

-	If a UE is configured with enableTwoDefaultTCI-States, and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH or PDSCH transmission occasions of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. When the UE is configured by higher layer parameter repetitionScheme set to 'tdmSchemeA' or is configured with higher layer parameter repetitionNumber, and the offset between the reception of the DL DCI and the first PDSCH transmission occasion is less than the threshold timeDurationForQCL, the mapping of the TCI states to PDSCH transmission occasions is determined according to clause 5.1.2.1 by replacing the indicated TCI states with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states based on the activated TCI states in the slot with the first PDSCH transmission occasion. In this case, if the 'QCL-TypeD' in both of the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers)

-	If a UE is not configured with sfnSchemePdsch, and the UE is configured with sfnSchemePdcch set to 'sfnSchemeA' and there is no TCI codepoint witih two TCI states in the activation command and the CORESET with the lowest ID in the latest slot is indicated with two TCI states, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the first TCI state of two TCI states indicated for the CORESET. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET with single active TCI state. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers).

-	In all cases above, if none of configured TCI states for the serving cell of scheduled PDSCH is configured with qcl-Type set to 'typeD', the UE shall obtain the other QCL assumptions from the indicated TCI state(s) for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH.

Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the UE is provided dl-OrJoint-TCIStateList-r17, and if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD', regardless of configuration of followUnifiedTCIstate,

-	if the indicated TCI state is associated with the PCI of the serving cell, the indicated TCI state is applied to PDSCH reception.

-	if the indicated TCI state is associated with a PCI different from the serving cell, the UE may assume that the DM-RS ports of PDSCH(s) of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In the CA case, if the 'QCL-TypeD' of the PDSCH DM-RSs from respective CCs in a band are different in a slot, the QCL-TypeD assumption of the PDSCH DM-RS in the CC with lowest CC ID in the band is applied to all the PDSCH DM-RSs in the CCs in the band. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers).

If the PDCCH carrying the scheduling DCI is received on one component carrier, and a PDSCH scheduled by that DCI is on another component carrier:

-	The timeDurationForQCL is determined based on the subcarrier spacing of the scheduled PDSCH. If µPDCCH < µPDSCH an additional timing delay d2μPDSCH2μPDCCH is added to the timeDurationForQCL, where d is defined in 5.2.1.5.1a-1, otherwise d is zero;

-	When the UE is configured with enableDefaultBeamForCCS, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, or if the DL DCI does not have the TCI field present, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell.

A UE that has indicated a capability beamCorrespondenceWithoutUL-BeamSweeping set to 'supported', as described in [13, TS 38.306], can determine a spatial domain filter to be used while performing the applicable channel access procedures described in [16, TS 37.213] prior to a UL transmission on the channel as follows:

-	if UE is indicated with an SRI corresponding to the UL transmission, the UE may use a spatial domain filter that is same as the spatial domain transmission filter associated with the indicated SRI,

-	if UE is configured with TCI-State in dl-OrJoint-TCIStateList or TCI-UL-State, the UE may use a spatial domain transmit filter that is same as the spatial domain receive filter the UE may use to receive the DL reference signal associated with the indicated TCI state.

When the PDCCH reception includes two PDCCH from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the time offset between the reception of the DL DCI and the corresponding PDSCH, the PDCCH candidate that ends later in time is used. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the configuration of tci-PresentInDCI or tci-PresentDCI-1-2, the UE expects the same configuration in the first and second CORESETs associated with the two PDCCH candidates; and if the PDSCH is scheduled by a DCI format not having the TCI field present and if the scheduling offset is equal to or larger than timeDurationForQCL, if applicable, PDSCH QCL assumption is based on the CORESET with lower ID among the first and second CORESETs associated with the two PDCCH candidates.

For a periodic CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s):

-	'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block where SS/PBCH block may have a PCI different from the PCI of the serving cell. The UE can assume center frequency, SCS, SFN offset are the same for SS/PBCH block from the serving cell and SS/PBCH block having a PCI different from the serving cell, or

-	'typeC' with an SS/PBCH block and, when applicable,'typeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, where SS/PBCH block may have a PCI different from the PCI of the serving cell. The UE can assume center frequency, SCS, SFN offset are the same for SS/PBCH block from the serving cell and SS/PBCH block having a PCI different from the serving cell.

For periodic/semi-persistent CSI-RS, if the UE is configured with dl-OrJoint-TCIStateList, the UE can assume that the indicated TCI-State is not applied.

For an aperiodic CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE shall expect that a TCI-State indicates qcl-Type set to 'typeA' with a periodic CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, qcl-Type set to 'typeD' with the same periodic CSI-RS resource.

For a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without the higher layer parameter repetition, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s):

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, or

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with an SS/PBCH block, where SS/PBCH block may have a PCI different from the PCI of the serving cell. The UE can assume center frequency, SCS, SFN offset are the same for SS/PBCH block from the serving cell and SS/PBCH block having a PCI different from the serving cell, or

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

-	'typeB' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info when 'typeD' is not applicable.

For a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s):

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, or

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

-	'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block, the reference RS may additionally be an SS/PBCH block having a PCI different from the PCI of the serving cell. The UE can assume center frequency, SCS, SFN offset are the same for SS/PBCH block from the serving cell and SS/PBCH block having a PCI different from the serving cell.

For the DM-RS of PDCCH, if the UE is not configured with dl-OrJoint-TCIStateList, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s):

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, or

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without higher layer parameter repetition and, when applicable, 'typeD' with the same CSI-RS resource.

When a UE is configured with sfnSchemePdcch set to 'sfnSchemeA', and CORESET is activated with two TCI states, the UE shall assume that the DM-RS port(s)of the PDCCH in the CORESET is quasi co-located with the DL-RSs of the two TCI states. When a UE is configured with sfnSchemePdcch set to 'sfnSchemeB', and a CORESET is activated with two TCI states, the UE shall assume that the DM-RS port(s)of the PDCCH is quasi co-located with the DL-RSs of the two TCI states except for quasi co-location parameters {Doppler shift, Doppler spread} of the second indicated TCI state.

For the DM-RS of PDSCH, if the UE is not configured with dl-OrJoint-TCIStateList, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s):

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, or

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition,or

-	typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without higher layer parameter repetition and, when applicable, 'typeD' with the same CSI-RS resource.

For the DM-RS of PDCCH, if the UE is configured with dl-OrJoint-TCIStateList, the UE shall expect that an indicated DLorJointTCIState indicates one of the following quasi co-location type(s):

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, or

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.

For the DM-RS of PDSCH, if the UE is configured with dl-OrJoint-TCIStateList, the UE shall expect that an indicated TCI-State indicates one of the following quasi co-location type(s):

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, or

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.

When a UE is configured with sfnSchemePdsch set to 'sfnSchemeA', and the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' in a DCI scheduling a PDSCH, the UE shall assume that the DM-RS port(s)of the PDSCH is quasi co-located with the DL-RSs of the two TCI states. When a UE is configured with sfnSchemePdsch set to 'sfnSchemeB', and the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' in a DCI scheduling a PDSCH, the UE shall assume that the DM-RS port(s)of the PDSCH is quasi co-located with the DL-RSs of the two TCI states except for quasi co-location parameters {Doppler shift, Doppler spread} of the second indicated TCI state.

<omitted text>

##### 5.1.6.1.1	CSI-RS for tracking

A UE in RRC connected mode is expected to receive the higher layer UE specific configuration of a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info.

For a NZP-CSI-RS-ResourceSet configured with the higher layer parameter trs-Info, the UE shall assume the antenna port with the same port index of the configured NZP CSI-RS resources in the NZP-CSI-RS-ResourceSet is the same.

-	For frequency range 1, the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. If no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated, then the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot.

-	For frequency range 2 the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of two periodic CSI-RS resources in one slot or with a NZP-CSI-RS-ResourceSet of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot.

A UE configured with NZP-CSI-RS-ResourceSet(s) configured with higher layer parameter trs-Info may have the CSI-RS resources configured as:

-	Periodic, with the CSI-RS resources in the NZP-CSI-RS-ResourceSet configured with same periodicity, bandwidth and subcarrier location.

-	Periodic CSI-RS resource in one set and aperiodic CSI-RS resources in a second set, with the aperiodic CSI-RS and periodic CSI-RS resource having the same bandwidth (with same RB location) and the aperiodic CSI-RS being configured with qcl-Type set to 'typeA' and 'typeD', where applicable, with the periodic CSI-RS resources. For frequency range 2, the UE does not expect that the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is smaller than beamSwitchTiming + d ∙2μCSIRS/2μPDCCH in CSI-RS symbols, where beamSwitchTiming is UE reported value defined in [13, TS 38.306], the reported value is one of the values of {14, 28, 48}∙2max(0, μCSIRS-3), and the beam switching timing delay d is defined in Table 5.2.1.5.1a-1 if  µPDCCH < µCSIRS , else d is zero. The UE shall expect that the periodic CSI-RS resource set and aperiodic CSI-RS resource set are configured with the same number of CSI-RS resources and with the same number of CSI-RS resources in a slot. For the aperiodic CSI-RS resource set if triggered, and if the associated periodic CSI-RS resource set is configured with four periodic CSI-RS resources with two consecutive slots with two periodic CSI-RS resources in each slot, the higher layer parameter aperiodicTriggeringOffset indicates the triggering offset for the first slot for the first two CSI-RS resources in the set.

A UE does not expect to be configured with a CSI-ReportConfig that is linked to a CSI-ResourceConfig containing an NZP-CSI-RS-ResourceSet configured with trs-Info and with the CSI-ReportConfig configured with the higher layer parameter timeRestrictionForChannelMeasurements set to 'configured'.

A UE does not expect to be configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to other than 'none' for aperiodic NZP CSI-RS resource set configured with trs-Info.

A UE does not expect to be configured with a CSI-ReportConfig for periodic NZP CSI-RS resource set configured with trs-Info.

A UE does not expect to be configured with a NZP-CSI-RS-ResourceSet configured both with trs-Info and repetition.

Each CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter NZP-CSI-RS-Resource with the following restrictions:

-	the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), as defined by higher layer parameter CSI-RS-resourceMapping, is given by one of

-	, , or for frequency range 1 and frequency range 2,

-	, , , , ,  or  for frequency range 2.

-	a single port CSI-RS resource with density  given by Table 7.4.1.5.3-1 from [4, TS 38.211] and higher layer parameter density configured by CSI-RS-ResourceMapping.

-	if carrier Ngridsize,μ=52, NBWP,isize=52, μ=0 and the carrier is configured in paired spectrum, the bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is X resource blocks, where X ≥ 28 resources if the UE indicates trs-AddBW-Set1 for the trs-AdditionalBandwidth capability for CSI-RS for tracking or addBW-Set1 for the aperiodicCSI-RS-AdditionalBandwidth capability for aperiodic CSI-RS for fast SCell activation and X ≥ 32 if the UE indicates trs-AddBW-Set2 for the AdditionalBandwidth capability for CSI-RS for tracking or addBW-Set2 for the aperiodicCSI-RS-AdditionalBandwidth capability for aperiodic CSI-RS for fast SCell activation; in these cases, if the UE is configured with CSI-RS comprising X<52 resource blocks, the UE does not expect that the total number of PRBs allocated for DL transmissions but not overlapped with the PRBs carrying CSI-RS for tracking is more than 4, where all CSI-RS resource configurations shall span the same set of resource blocks; otherwise, the bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is the minimum of 52 and NBWP,isize resource blocks, or is equal to NBWP,isize resource blocks. For operation with shared spectrum channel access, freqBand configured by CSI-RS-ResourceMapping, is the minimum of 48 and NBWP,isize resource blocks, or is equal to NBWP,isize resource blocks.

-	the UE is not expected to be configured with the periodicity of  slots if the bandwidth of CSI-RS resource is larger than 52 resource blocks.

-	the periodicity and slot offset for periodic NZP CSI-RS resources, as given by the higher layer parameter periodicityAndOffset configured by NZP-CSI-RS-Resource, is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211].

-	same powerControlOffset and powerControlOffsetSS given by NZP-CSI-RS-Resource value across all resources.

A UE in RRC_IDLE or RRC_INACTIVE can receive a higher layer configuration of TRS occasions via a trs-ResourceSetConfig.

-	For frequency range 1, the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a TRS-ResourceSet consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. If no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon, then the UE may be configured with one or more TRS resource set(s), where a TRS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot.

-	For frequency range 2 the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a TRS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot or by a TRS-ResourceSet of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot.

Each NZP CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter TRS-ResourceSet with the following restrictions for a UE in RRC_IDLE or RRC_INACTIVE:

-	the number of periodic NZP CSI-RS resources configured by a TRS-ResourceSet is given by nrofResources

-	the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), is one of

-	, , or for frequency range 1 and frequency range 2,

-	, , , , ,  or  for frequency range 2.

-	where the first symbol location in a slot is indicated by firstOFDMSymbolInTimeDomain in the TRS-ResourceSet and the second symbol location in a slot is firstOFDMSymbolInTimeDomain + 4

-	a single port CSI-RS resource with density  given by Table 7.4.1.5.3-1 from [4, TS 38.211].

-	the bandwidth and the frequency location of the NZP CSI-RS resource, is given by the higher layer parameter nrofRBs, startingRB and frequencyDomainAllocation in a TRS-ResourceSet and applies to all resources in a TRS-ResourceSet. Bandwidth, nrofRBs, and the initial CRB index, startingRB, of the NZP CSI-RS resource configured by TRS-ResourceSet are not restricted by initial DL BWP.

-	UE is not required to receive TRS occasions outside the initial DL BWP.

-	the periodicity for periodic NZP CSI-RS resources, is given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet, is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211], applies to all resources in a TRS-ResourceSet. The slot offset given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet provides the location of the first slot containing the periodic NZP CSI-RS resources configured by by a TRS-ResourceSet.

-	the UE does not expect the TRS-ResourceSet to be configured with the periodicity of  slots if the bandwidth of NZP CSI-RS resource is larger than 52 resource blocks.

-	the UE may assume the sub-carrier spacing of the NZP CSI-RS resources configured by TRS-ResourceSet to be same as the sub-carrier spacing of the initial DL BWP.

-	powerControlOffsetSS given by a TRS-ResourceSet applies to all resources in a TRS-ResourceSet.

-	the QCL information for periodic NZP CSI-RS resources, is given by the higher layer parameter ssb-Index configured by a TRS-ResourceSet, is a SS/PBCH block, applies to all resources in a TRS-ResourceSet.

-	One or more scrambling IDs according to scramblingID-Info where if a single scramblingIDforCommon is configured, it applies to all NZP-CSI-RS resources in the resource set, otherwise, each NZP-CSI-RS resource is provided with a scrambling IDs according to scramblingIDperResourceListWith2 or scramblingIDperResourceListWith4.

-	the UE may assume the following quasi co-location type(s):

-	'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block.

For each TRS-ResourceSet the index of the associated bit in TRS availability indication field [5, TS 38.212], is given by the higher layer parameter indBitID.

<omitted text>

#### 5.1.6.2	DM-RS reception procedure

The DM-RS reception procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 instead of dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB.

The DM-RS reception procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_2, by applying the parameters of dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB in pdsch-ConfigMulticast instead of dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB in PDSCH-Config.

When receiving PDSCH scheduled by DCI format 1_0, 4_0, or 4_1, or receiving PDSCH before dedicated higher layer configuration of any of the parameters dmrs-AdditionalPosition, maxLength and dmrs-Type, the UE shall assume that the PDSCH is not present in any symbol carrying DM-RS except for PDSCH with allocation duration of 2 symbols with PDSCH mapping type B (described in clause 7.4.1.1.2 of [4, TS 38.211]), and a single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 1000 is transmitted, and that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE and in addition

-	For PDSCH with mapping type A and type B, the UE shall assume dmrs-AdditionalPosition='pos2' and up to two additional single-symbol DM-RS present in a slot according to the PDSCH duration indicated in the DCI as defined in Clause 7.4.1.1 of [4, TS 38.211], and

-	For PDSCH with allocation duration of 2 symbols with mapping type B, the UE shall assume that the PDSCH is present in the symbol carrying DM-RS.

When receiving PDSCH scheduled by DCI format 1_1 by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI or DCI format 4_2 by PDCCH with CRC scrambled by G-RNTI or G-CS-RNTI,

-	the UE may be configured with the higher layer parameter dmrs-Type, and the configured DM-RS configuration type is used for receiving PDSCH in as defined in Clause 7.4.1.1 of [4, TS 38.211].

-	the UE may be configured with the maximum number of front-loaded DM-RS symbols for PDSCH by higher layer parameter maxLength given by DMRS-DownlinkConfig..

-	if maxLength is set to 'len1', single-symbol DM-RS can be scheduled for the UE by DCI, and the UE can be configured with a number of additional DM-RS for PDSCH by higher layer parameter dmrs-AdditionalPosition, which can be set to 'pos0', 'pos1', 'pos2' or 'pos3'.

-	if maxLength is set to 'len2', both single-symbol DM-RS and double symbol DM-RS can be scheduled for the UE by DCI, and the UE can be configured with a number of additional DM-RS for PDSCH by higher layer parameter dmrs-AdditionalPosition, which can be set to 'pos0' or 'pos1'.

-	and the UE shall assume to receive additional DM-RS as specified in Table 7.4.1.1.2-3 and Table 7.4.1.1.2-4 as described in Clause 7.4.1.1.2 of [4, TS 38.211].

For the UE-specific reference signals generation as defined in Clause 7.4.1.1 of [4, TS 38.211], a UE can be configured by higher layers with one or two scrambling identity(s), nIDDMRS,i i = 0,1 which are the same for both PDSCH mapping Type A and Type B.

A UE may be scheduled with a number of DM-RS ports by the antenna port index in DCI format 1_1 as described in Clause 7.3.1.2 of [5, TS 38.212].

For DM-RS configuration type 1,

-	if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 30} in Table 7.3.1.2.2-1 and Table 7.3.1.2.2-2 of Clause 7.3.1.2 of [5, TS 38.212], or

-	if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 12} in Table 7.3.1.2.2-1A and {2, 9, 10, 11, 30 or 31} in Table 7.3.1.2.2-2A of Clause 7.3.1.2 of [5, TS 38.212], or

-	if a UE is scheduled with two codewords,

the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE.

For DM-RS configuration type 2,

-	if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 10 or 23} in Table 7.3.1.2.2-3 and Table 7.3.1.2.2-4 of Clause 7.3.1.2 of [5, TS38.212], or

-	if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 10, 23 or 24} in Table 7.3.1.2.2-3A and {2, 10, 23 or 58} in Table 7.3.1.2.2-4A of Clause 7.3.1.2 of [5, TS 38.212], or

-	if a UE is scheduled with two codewords,

the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE.

If a UE receiving PDSCH scheduled by DCI format 1_2 is configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2  or dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 or a UE receiving PDSCH scheduled by DCI format 1_0 or DCI format 1_1 is configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB, the UE may assume that the following configurations are not occurring simultaneously for the received PDSCH:

-	any DM-RS ports among 1004-1007 or 1006-1011 for DM-RS configurations type 1 and type 2, respectively are scheduled for the UE and the other UE(s) sharing the DM-RS REs on the same CDM group(s), and

-	PT-RS is transmitted to the UE.

The UE is not expected to simultaneously be configured with the maximum number of front-loaded DM-RS symbols for PDSCH by higher layer parameter maxLength being set equal to 'len2' and more than one additional DM-RS symbol as given by the higher layer parameter dmrs-AdditionalPosition.

The UE is not expected to assume co-scheduled UE(s) with different DM-RS configuration with respect to the actual number of front-loaded DM-RS symbol(s), the actual number of additional DM-RS, the DM-RS symbol location, and DM-RS configuration type as described in Clause 7.4.1.1 of [4, TS 38.211].

The UE does not expect the precoding of the potential co-scheduled UE(s) in other DM-RS ports of the same CDM group to be different in the PRG-level grid configured to this UE with PRG =2 or 4.

The UE does not expect the resource allocation of the potential co-scheduled UE(s) in other DM-RS ports of the same CDM group to be misaligned in the PRG-level grid to this UE with PRG=2 or 4.

When receiving PDSCH scheduled by DCI format 1_1, the UE shall assume that the CDM groups indicated in the configured index from Tables 7.3.1.2.2-1, 7.3.1.2.2-1A, 7.3.1.2.2-2, 7.3.1.2.2-2A, 7.3.1.2.2-3, 7.3.1.2.2-3A, 7.3.1.2.2-4, 7.3.1.2.2-4A of [5, TS. 38.212] contain potential co-scheduled downlink DM-RS and are not used for data transmission, where "1", "2" and "3" for the number of DM-RS CDM group(s) in Tables 7.3.1.2.2-1, 7.3.1.2.2-1A, 7.3.1.2.2-2, 7.3.1.2.2-3, 7.3.1.2.2-3A, 7.3.1.2.2-4, 7.3.1.2.2-4A of [5, TS. 38.212] correspond to CDM group 0, {0,1}, {0,1,2}, respectively.

When receiving PDSCH scheduled by DCI format 1_0, 4_0, or 4_1, the UE shall assume the number of DM-RS CDM groups without data is 1 which corresponds to CDM group 0 for the case of PDSCH with allocation duration of 2 symbols, and the UE shall assume that the number of DM-RS CDM groups without data is 2 which corresponds to CDM group {0,1} for all other cases.

The UE is not expected to receive PDSCH scheduling DCI which indicates CDM group(s) with potential DM-RS ports which overlap with any configured CSI-RS resource(s) for that UE.

If the UE receives the DM-RS for PDSCH and an SS/PBCH block associated with the same PCI in the same OFDM symbol(s), then the UE may assume that the DM-RS and SS/PBCH block are quasi co-located with 'typeD', if 'typeD' is applicable. Furthermore, the UE shall not expect to receive DM-RS in resource elements that overlap with those of the SS/PBCH block associated with the same PCI as the DM-RS, and the UE can expect that the same or different subcarrier spacing is configured for the DM-RS and SS/PBCH block in a CC except for the case of 240 kHz where only different subcarrier spacing is supported. A DM-RS for PDSCH is said to be associated with an additional PCI if the indicated TCI state for the PDSCH is associated with the additional PCI, otherwise a DM-RS for PDSCH is associated with serving cell PCI.

If at least one TCI codepoint indicates two TCI states and the UE receives the DM-RS for PDSCH and an SS/PBCH block in the same OFDM symbol(s), then the UE may assume that at least one DM-RS port for the PDSCH and SS/PBCH block are quasi co-located with 'QCL-TypeD', if 'QCL-TypeD' is applicable.

If the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in different ControlResourceSets, and the UE receives the DM-RS for PDSCH(s) and an SS/PBCH block in the same OFDM symbol(s), then the UE may assume that at least one DM-RS port for the PDSCH(s) and SS/PBCH block are quasi co-located with 'QCL-TypeD', if 'QCL-TypeD' is applicable.

If a UE is configured by the higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, the UE may be scheduled with fully or partially overlapping PDSCHs in the time and frequency domain by multiple PDCCHs with the following restrictions,

-	the UE is not expected to assume different DM-RS configuration with respect to the actual number of front-loaded DM-RS symbol(s), the actual number of additional DM-RS symbol(s), the actual DM-RS symbol location, and DM-RS configuration type.

-	the UE is not expected to assume DM-RS ports in a CDM group indicated by two TCI states.

When a UE is not indicated with a DCI that DCI field 'Time domain resource assignment' indicating an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation, the UE is not configured with sfnSchemePdsch and it is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within two CDM groups in the DCI field 'Antenna Port(s)',

-	the first TCI state corresponds to the CDM group of the first antenna port indicated by the antenna port indication table, and the second TCI state corresponds to the other CDM group.

If a UE is configured with higher layer parameter dmrs-FD-OCC-DisabledForRank1-PDSCH and the UE is scheduled with PDSCH with single DM-RS port, the UE may assume that set of orthogonal DM-RS antenna ports from the same CDM group using different set of wf(k') codes are not associated with the transmission of PDSCH to another UE.

#### 5.1.6.3	PT-RS reception procedure

The procedures on PT-RS reception described in this clause apply to a UE receiving PDSCH scheduled by DCI format 1_2 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 or dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 and to a UE receiving PDSCH scheduled by DCI format 1_0 or DCI format 1_1 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB. The procedures on PT-RS reception described in this clause apply to a UE receiving PDSCH scheduled by DCI format 4_1 or 4_2 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB in pdsch-ConfigMulticast.

A UE shall report the preferred MCS and bandwidth thresholds based on the UE capability at a given carrier frequency, for each subcarrier spacing applicable to data channel at this carrier frequency, assuming the MCS table with the maximum Modulation Order as it reported to support.

<omitted text>

#### 5.1.6.5	PRS reception procedure

The UE can be configured with one or more DL PRS resource set configuration(s) as indicated by the higher layer parameters NR-DL-PRS-ResourceSet and NR-DL-PRS-Resource as defined by Clause 6.4.3 [17, TS 37.355]. Each DL PRS resource set consists of K≥1 DL PRS resource(s) where each has an associated spatial transmission filter. The UE can be configured with one or more DL PRS positioning frequency layer configuration(s) as indicated by the higher layer parameter NR-DL-PRS-PositioningFrequencyLayer. A DL PRS positioning frequency layer is defined as a collection of DL PRS resource sets which have common parameters configured by NR-DL-PRS-PositioningFrequencyLayer.

The UE assumes that the following parameters for each DL PRS resource(s) are configured via higher layer parameters NR-DL-PRS-PositioningFrequencyLayer, NR-DL-PRS-ResourceSet and NR-DL-PRS-Resource.

A DL PRS positioning frequency layer is configured by NR-DL-PRS-PositioningFrequencyLayer, consists of one or more DL PRS resource sets and it is defined by:

-	dl-PRS-SubcarrierSpacing defines the subcarrier spacing for the DL PRS resource. All DL PRS resources and DL PRS resource sets in the same DL PRS positioning frequency layer have the same value of dl-PRS-SubcarrierSpacing. The supported values of dl-PRS-SubcarrierSpacing are given in Table 4.2-1 of [4, TS38.211], excluding the value of 240kHz.

-	dl-PRS-CyclicPrefix defines the cyclic prefix for the DL PRS resource. All DL PRS Resources and DL PRS Resource sets in the same DL PRS positioning frequency layer have the same value of dl-PRS-CyclicPrefix. The supported values of dl-PRS-CyclicPrefix are given in Table 4.2-1 of [4, TS38.211].

-	dl-PRS-PointA defines the absolute frequency of the reference resource block. Its lowest subcarrier is also known as Point A. All DL PRS resources belonging to the same DL PRS resource set have common Point A and all DL PRS resources sets belonging to the same DL PRS positioning frequency layer have a common Point A.

The UE expects that it will be configured with dl-PRS-ID each of which is defined such that it is associated with multiple DL PRS resource sets. The UE expects that one of these dl-PRS-ID along with a nr-DL-PRS-ResourceSetID and a nr-DL-PRS-ResourceID-r16 can be used to uniquely identify a DL PRS resource.

The UE may be configured by the network with nr-PhysCellID, nr-CellGlobalID, and nr-ARFCN [17, TS 37.355] associated with a dl-PRS-ID.

-	If nr-PhysCellID or nr-CellGlobalID is provided, and if nr-PhysCellID, nr-CellGlobalID and nr-ARFCN associated with the dl-PRS-ID, if provided, are the same as the corresponding information of a serving cell, the UE may assume that the DL PRS is transmitted from the serving cell;

-	Otherwise, the UE may assume that the DL PRS is not transmitted from a serving cell.

If the UE assumes that the DL PRS is transmitted from a serving cell, and if the serving cell is the same as the serving cell defined by the SS/PBCH block, the UE may assume that the DL PRS and the SS/PBCH block are transmitted from the same serving cell.

If the UE assumes that the DL PRS is not transmitted from a serving cell, and if nr-PhysCellID is provided, and is the same as physical cell ID of the SS/PBCH block from a non-serving cell of the same band as the DL PRS, the UE may assume that the DL PRS and the SS/PBCH block are transmitted from the same non-serving cell.

A DL PRS resource set is configured by NR-DL-PRS-ResourceSet, consists of one or more DL PRS resources and it is defined by:

-	nr-DL-PRS-ResourceSetID defines the identity of the DL PRS resource set configuration.

-	dl-PRS-Periodicity-and-ResourceSetSlotOffset defines the DL PRS resource periodicity and takes values TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing=15, 30, 60 and 120 kHz respectively and the slot offset for DL PRS resource set with respect to SFN0 slot 0. All the DL PRS resources within one DL PRS resource set are configured with the same DL PRS resource periodicity. The UE does not expect that the product of DL PRS resource periodicity TperPRS, the higher layer parameter dl-prs-MutingBitRepetitionFactor and the size of the bitmap of dl-PRS-MutingOption1 exceeds 2μ×10240, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing=15, 30, 60 and 120 kHz respectively.

-	dl-PRS-ResourceRepetitionFactor defines how many times each DL-PRS resource is repeated for a single instance of the DL-PRS resource set and takes values TrepPRS∈1,2,4,6,8,16,32. All the DL PRS resources within one resource set have the same resource repetition factor.

-	dl-PRS-ResourceTimeGap defines the offset in number of slots between two repeated instances of a DL PRS resource with the same nr-DL-PRS-ResourceID within a single instance of the DL PRS resource set. The UE only expects to be configured with dl-PRS-ResourceTimeGap if dl-PRS-ResourceRepetitionFactor is configured with value greater than 1. The time duration spanned by one instance of a nr-DL-PRS-ResourceSet is not expected to exceed the configured value of DL PRS periodicity. All the DL PRS resources within one resource set have the same value of dl-PRS-ResourceTimeGap.

-	dl-PRS-MutingOption1 and dl-PRS-MutingOption2 define the time locations where the DL PRS resource is expected to not be transmitted for a DL PRS resource set. If dl-PRS-MutingOption1 is configured, each bit in the bitmap of dl-PRS-MutingOption1 corresponds to a configurable number provided by higher layer parameter dl-prs-MutingBitRepetitionFactor of consecutive instances of a DL PRS resource set where all the DL PRS resources within the set are muted for the instance that is indicated to be muted. The length of the bitmap can be {2, 4, 6, 8, 16, 32} bits. If dl-PRS-MutingOption2 is configured each bit in the bitmap of dl-PRS-MutingOption2 corresponds to a single repetition index for each of the DL PRS resources within each instance of a nr-DL-PRS-ResourceSet and the length of the bitmap is equal to the values of dl-PRS-ResourceRepetitionFactor. Both dl-PRS-MutingOption1 and dl-PRS-MutingOption2 may be configured at the same time in which case the logical AND operation is applied to the bit maps as described in Clause 7.4.1.7.4 of [4, TS 38.211].

-	NR-DL-PRS-SFN0-Offset defines the time offset of the SFN0 slot 0 for the DL PRS resource set with respect to SFN0 slot 0 of reference provided by nr-DL-PRS-ReferenceInfo.

-	dl-PRS-ResourceList determines the DL PRS resources that are contained within one DL PRS resource set.

-	dl-PRS-CombSizeN defines the comb size of a DL PRS resource where the allowable values are given in Clause 7.4.1.7.3 of [TS38.211]. All DL PRS resource sets belonging to the same DL PRS positioning frequency layer have the same value of dl-PRS-CombSizeN.

-	dl-PRS-ResourceBandwidth defines the number of resource blocks configured for DL PRS transmission. The parameter has a granularity of 4 PRBs with a minimum of 24 PRBs and a maximum of 272 PRBs. All DL PRS resources sets within a DL PRS positioning frequency layer have the same value of dl-PRS-ResourceBandwidth.

-	dl-PRS-StartPRB defines the starting PRB index of the DL PRS resource with respect to reference Point A, where reference Point A is given by the higher-layer parameter dl-PRS-PointA. The starting PRB index has a granularity of one PRB with a minimum value of 0 and a maximum value of 2176 PRBs. All DL PRS resource sets belonging to the same DL PRS positioning frequency layer have the same value of dl-PRS-StartPRB.

-	dl-PRS-NumSymbols defines the number of symbols of the DL PRS resource within a slot where the allowable values are given in Clause 7.4.1.7.3 of [4, TS38.211].

A DL PRS resource is defined by:

-	nr-DL-PRS-ResourceID determines the DL PRS resource configuration identity. All DL PRS resource IDs are locally defined within a DL PRS resource set.

-	dl-PRS-SequenceID is used to initialize cinit value used in pseudo random generator as described in Clause 7.4.1.7.2 of [4, TS 38.211] for generation of DL PRS sequence for a given DL PRS resource.

-	dl-PRS-CombSizeN-AndReOffset defines the starting RE offset of the first symbol within a DL PRS resource in frequency. The relative RE offsets of the remaining symbols within a DL PRS resource are defined based on the initial offset and the rule described in Clause 7.4.1.7.3 of [4, TS 38.211].

-	dl-PRS-ResourceSlotOffset determines the starting slot of the DL PRS resource with respect to corresponding DL PRS resource set slot offset.

-	dl-PRS-ResourceSymbolOffset determines the starting symbol of a slot configured with the DL PRS resource.

-	dl-PRS-QCL-Info defines any quasi co-location information of the DL PRS resource with other reference signals. The DL PRS may be configured with QCL 'typeD' with a DL PRS associated with the same dl-PRS-ID, or with rs-Type set to 'typeC', 'typeD', or 'typeC-plus-typeD' with a SS/PBCH Block from a serving or non-serving cell.

-	dl-PRS-ResourcePrioritySubset defines a subset of DL-PRS resources for the DL PRS resource for the purpose of prioritization of DL-AoD reporting.

The UE assumes constant EPRE is used for all REs of a given DL PRS resource.

The UE may be indicated by the network that DL PRS resource(s) can be used as the reference for the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements in a higher layer parameter nr-DL-PRS-ReferenceInfo. The reference indicated by the network to the UE can also be used by the UE to determine how to apply higher layer parameters nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty. The UE expects the reference to be indicated whenever it is expected to receive the DL PRS. This reference provided by nr-DL-PRS-ReferenceInfo may include a dl-PRS-ID, a DL PRS resource set ID, and optionally a single DL PRS resource ID or a list of DL PRS resource IDs [17, TS 37.355]. The UE may use different DL PRS resources or a different DL PRS resource set to determine the reference for the RSTD measurement as long as the condition that the DL PRS resources used belong to a single DL PRS resource set is met. If the UE chooses to use a different reference than indicated by the network, then it is expected to report the dl-PRS-ID, the DL PRS resource ID(s) or the DL PRS resource set ID used to determine the reference.

The UE may be configured to report quality metrics NR-TimingQuality corresponding to the DL RSTD and UE Rx-Tx time difference measurements which include the following fields:

-	timingQualityValue which provides the best estimate of the uncertainty of the measurement

-	timingQualityResolution which specifies the resolution levels used in the timingQualityValue field.

The UE expects to be configured with higher layer parameter nr-DL-PRS-ExpectedRSTD, which defines the time difference with respect to the received DL subframe timing the UE is expected to receive DL PRS, and nr-DL-PRS-ExpectedRSTD-Uncertainty, which defines a search window around the nr-DL-PRS-ExpectedRSTD.

For DL UE positioning measurement reporting in higher layer parameters NR-DL-TDOA-SignalMeasurementInformation or NR-Multi-RTT-SignalMeasurementInformation the UE can be configured to report the DL PRS resource ID(s) or the DL PRS resource set ID(s) associated with the DL PRS resource(s) or the DL PRS resource set(s) which are used in determining the UE measurements DL RSTD, or UE Rx-Tx time difference, respectively.

For the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements the UE reports an associated higher layer parameter nr-TimeStamp. The nr-TimeStamp can include the dl-PRS-ID, the SFN and the slot number for a subcarrier spacing. These values correspond to the reference which is provided by nr-DL-PRS-ReferenceInfo.

The UE is expected to measure the DL PRS resource outside the active DL BWP or with a numerology different from the numerology of the active DL BWP if the measurement is made during a configured measurement gap. When the UE is expected to measure the DL PRS resource, the UE may request a measurement gap via higher layer parameter NR-PRS-MeasurementInfoList [12, TS 38.331] or as specified in clause 6.1.3.40 of [10, TS 38.321]. The UE may be preconfigured with one or more measurement gaps each associated with an measPosPreConfigGapId. When the UE requests activation or deactivation of a measurement gap as specified in clause 6.1.3.40 of [10, TS 38.321] it can request one of the preconfigured measurement gaps by referring to the measPosPreConfigGapId. The UE may have one of the preconfigured measurement gap(s) activated or deactivated as specified in clause 6.1.3.41 of [10, TS 38.321].

The UE assumes that the DL PRS from the serving cell is not mapped to any symbol that contains SS/PBCH block from the serving cell. If the time frequency location of the SS/PBCH block transmissions from non-serving cells are provided to the UE then the UE also assumes that the DL PRS from a non-serving cell is not mapped to any symbol that contains the SS/PBCH block of the same non-serving cell.

The UE may be configured to measure and report, subject to UE capability, up to 4 DL RSTD measurements per pair of dl-PRS-ID with each measurement between a different pair of DL PRS resources or DL PRS resource sets within the DL PRS configured for those dl-PRS-ID. If the UE is not configured to report with multiMeasInSameReport-r17, the up to 4 measurements being performed on the same pair of dl-PRS-ID and all DL RSTD measurements in the same report use a single reference timing. If the UE is configured to report with multiMeasInSameReport-r17, the up to 4 measurements being performed on the same pair of dl-PRS-ID and all DL RSTD measurements in the same measurement instance of the same report use a single reference timing.

The UE may be configured to measure and report, subject to UE capability, up to 24 DL PRS-RSRP measurements on DL PRS resources associated with the same dl-PRS-ID. When the UE reports DL PRS-RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements associated with it within the DL PRS resource set. When the UE reports DL PRS-RSRP measurements for a DL PRS resource, the reported multiple DL PRS-RSRP measurements associated with the same or different higher layer parameter nr-DL-PRS-RxBeamIndex may have the same or different timestamps. The UE may be configured to measure and optionally report, subject to UE capability, up to 24 DL PRS RSRPP for the first detected path on DL PRS resources associated with the same dl-PRS-ID.

The UE may be configured to optionally report a differential DL RSRPP for a PRS resource with reference to nr-DL-PRS-FirstPathRSRP-Result and/or a differential DL PRS RSRP with reference to nr-DL-PRS-RSRP-Result via higher layer parameter NR-DL-AoD-AdditionalMeasurementElement.

For each PRS resource, the UE may be configured, subject to UE capability, with dl-PRS-ResourcePrioritySubset that is associated with this PRS resource, where the subset of PRS resources associated with the PRS resource can be in the same or different PRS resource set than the PRS resource. The UE may include UE measurements for the subset of PRS resources in NR-DL-AoD-AdditionalMeasurementElement if the UE measurements of the associated PRS resource are reported, where the UE measurement can be DL PRS-RSRP and/or DL PRS-RSRPP. The UE may report DL PRS-RSRP and/or DL PRS-RSRPP measurements only for the subset of PRS resources. Subject to UE capability, the UE may be configured with boresight direction via higher layer parameter DL-PRS-BeamInfoElement for each PRS resource.

The UE may be provided with beam/antenna information via higher layer parameter NR-TRP-BeamAntennaInfo.

The UE may request to be provided with either expected DL-AoD/ZoD and uncertainty range(s) of expected DL-AoD/ZoD, or expected DL-AoA/ZoA and uncertainty range(s) of the expected DL-AoA/ZoA. The UE may be provided with expected DL-AoD/ZoD and uncertainty range(s) of the expected DL-AoD/ZoD. The UE may be provided with expected DL-AoA/ZoA and uncertainty range(s) of the expected DL-AoA/ZoA. The uncertainty range(s) of the expected DL-AoD/DL-AoA may be configured within [0, 60]. The uncertainty range(s) of expected DL-ZoD/DL-ZoA may be configured within [0, 30].

The UE may be configured to measure and report, subject to UE capability, up to 4 UE Rx-Tx time difference measurements corresponding to a single configured SRS resource or resource set for positioning. Each measurement corresponds to a single received DL PRS resource or resource set which can be in different DL PRS positioning frequency layers.

The UE may be configured to measure and report via higher layer parameter additionalPaths or additionalPathsExt, subject to UE capability, the timing and the quality metrics of up to 8 additional detected paths, that are associated with each RSTD or UE Rx – Tx time difference. The timing of each additional path is reported relative to the path timing used for determining nr-RSTD or nr-UE-RxTxTimeDiff. For UE positioning measurement reporting in higher layer parameters NR-DL-TDOA-SignalMeasurementInformation or NR-Multi-RTT-SignalMeasurementInformation, the UE may be configured to measure and report, subject to UE capability, the DL PRS-RSRPP of the first path and the up to 8 additional paths that are associated with each RSTD or UE Rx – Tx time difference.

The UE may be requested, subject to UE capability, to measure and report one or more of the DL RSTD, DL PRS-RSRP, DL PRS-RSRPP, or UE Rx-Tx time difference measurements with either Nsample or 4 samples, where Nsample = 1 or 2 is as defined in [11, TS 38.133], via higher layer parameter requestedDL-PRS-ProcessingSamples [17, TS 37.355] which applies for all DL PRS positioning frequency layers.

The UE may be requested, subject to UE capability, to report LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-IndicatorRequest. The UE can report LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator associated with each DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements. The UE can report LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator associated with each dl-PRS-ID in a measurement report. For the LoS/NLoS indicator(s) associated with DL RSTD, the UE may report one indicator associated with the dl-PRS-ID indicated by higher layer parameter dl-PRS-ReferenceInfo and one indicator associated with the dl-PRS-ID of the DL RSTD measurement. A UE may be provided with LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator, and it may be associated with each DL PRS resource of each configured dl-PRS-ID or may be associated with each configured dl-PRS-ID. The values of the higher layer parameter LOS-NLOS-Indicator may be soft values (0, 0.1, …, 0.9, 1) or hard values (0, 1) with the values corresponding to the likelihood of LoS, with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS.

If the UE is configured with DL-PRS-QCL-Info and the QCL relation is between two DL PRS resources, then the UE assumes those DL PRS resources are associated with the same dl-PRS-ID. If DL-PRS-QCL-Info is configured to the UE with QCL set to 'type-D' with a source DL PRS resource then the nr-DL-PRS-ResourceSetId and the nr-DL-PRS-ResourceId of the source DL PRS resource are expected to be indicated to the UE.

The UE is expected to measure the DL PRS outside the measurement gap, subject to UE capability, if the DL PRS is inside the active DL BWP and has the same numerology as the active DL BWP and is within the DL PRS processing window indicated by higher layer parameter DL-PPW-PreConfig. The UE is not expected to measure the DL PRS outside the measurement gap if the expected received timing difference between the DL PRS from the non-serving cell and that from the serving cell, determined by the higher layer parameters nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty, is larger than maximum Rx timing difference provided by UE capability. For receiving the DL PRS outside the measurement gap and within the DL PRS processing window, the UE determines the DL PRS priority as indicated by higher layer parameter priority subject to UE capability or as implied by UE capability:

-	with value 'st1' where the DL PRS is higher priority than all the DL signals and channels except SSB, or

-	with value 'st2' where the DL PRS is lower priority than PDCCH and the PDSCH scheduled by DCI formats 1_1 or 1_2 with the priority indicator field in the corresponding DCI format set to 1, and is higher priority than other DL signals and channels except SSB, or

-	with value 'st3' where the DL PRS is lower priority than all the DL signals and channels except SSB.

Inside one DL-PPW-PreConfig the UE is only expected to measure a single DL PRS positioning frequency layer.

When the UE is expected to measure the DL PRS outside the measurement gap in a configured PRS processing window with type1A and if the DL PRS is determined to be higher priority than the DL signals and channels inside the PRS processing window, those DL signals and channels are not expected to be measured by the UE. When the UE is expected to measure the DL PRS outside the measurement gap in a configured PRS processing window with type1B and if the DL PRS is determined to be higher priority than the DL signals and channels inside the PRS processing window, those DL signals and channels in the same band as the DL PRS are not expected to be measured by the UE. When the UE is expected to measure the DL PRS outside the measurement gap in a configured PRS processing window with type2 if the DL PRS is determined to be higher priority than the DL signals and channels inside the PRS processing window, those DL signals and channels from the impacted serving cells are not expected to be measured by the UE on the overlapped symbols with the DL PRS, where impacted serving cells refer to the serving cell on which the DL-PPW-PreConfig is configured for a frequency range 1 band, and all the serving cells in the same band as the DL PRS for a frequency range 2 band.

When the UE has an activated PRS processing window with type1A or type1B and the UE determines the presence of other DL signals and channels, except SSB, of higher priority than the DL PRS in the PRS processing window no later than N2 symbols, defined in clause 6.4 for the subcarrier spacing μ of the DL PRS, before the first symbol of the PRS processing window, the UE is expected to receive the other DL signals and channels and drop all PRS within the PRS processing window. When the UE has an activated PRS processing window with type2 and the UE determines the presence of other DL signals and channels, except SSB, of higher priority than the DL PRS on a symbol configured with the DL PRS no later than N2 symbols, defined in clause 6.4 for the subcarrier spacing μ of the DL PRS, before the DL PRS symbol, the UE is expected to receive the other DL signals and channels and drop the DL PRS symbol.

When the UE has an activated PRS processing window with type1A or type1B and the UE determines the presence of other DL signals and channels, except SSB, of higher priority than the DL PRS in the PRS processing window later than N2 symbols, defined in clause 6.4 for the subcarrier spacing μ of the DL PRS, before the first symbol of the PRS processing window, the UE is not required to receive the other DL signals and channels and may receive the DL PRS and consider the DL PRS as higher priority in the PRS processing window. When the UE has an activated PRS processing window with type2 and the UE determines the presence of other DL signals and channels, except SSB, of higher priority than the DL PRS on a symbol configured with the DL PRS later than N2 symbols, defined in clause 6.4 for the subcarrier spacing μ of the DL PRS, before the DL PRS symbols, the UE is not required to receive the other DL signals and channels and may receive the DL PRS symbol and consider the DL PRS as higher priority in that symbol.

Within a positioning frequency layer, the DL PRS resources are sorted in the decreasing order of priority for measurement to be performed by the UE, with the reference indicated by nr-DL-PRS-ReferenceInfo being the highest priority for measurement, and the following priority is assumed:

-	Up to 64 NR-SelectedDL-PRS-IndexPerTRP of the DL PRS positioning frequency layer are sorted according to priority if nr-SelectedDL-PRS-IndexListPerFreq is provided, or up to 64 NR-DL-PRS-AssistanceDataPerTRP of the frequency layer are sorted according to priority otherwise;

-	Up to 2 DL-SelectedPRS-ResourceSetIndex per dl-PRS-ID of the DL PRS positioning frequency layer are sorted according to priority if dl-SelectedPRS-ResourceSetIndexList is provided, or up to 2 NR-DL-PRS-ResourceSet per dl-PRS-ID of the DL PRS positioning frequency layer are sorted according to priority otherwise.

The UE DL PRS processing capability is defined in [TS 37.355]. For the purpose of DL PRS processing capability, the duration K msec of DL PRS symbols within P msec window, is calculated by

-	Type 1 duration calculation with UE symbol level buffering capability

K=s∈SKs
Ks=Tsend-Tsstart

-	Type 2 duration calculation with UE slot level buffering capability

K=12μS

-	S is the set of slots based on the numerology of the DL PRS of a serving cell within the P msec window in the positioning frequency layer that contains potential DL PRS resources considering the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS Resource Sets.

-	For Type 1, Tsstart, Tsend is the smallest interval in msec within slot s corresponding to an integer number of OFDM symbols based on the numerology of the DL PRS of a serving cell that covers the union of the potential PRS symbols and determines the PRS symbol occupancy within slot s, where the interval Tsstart, Tsend considers the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS resource sets (target and reference).

-	For Type 2, μ is the numerology of the DL PRS, and S is the cardinality of the set S.

The UE may be configured to report one or more measurement instances, each with its own timestamp, on DL RSTD, DL PRS-RSRP, and/or UE Rx-Tx time difference measurements, in a single measurement report.

Timing Error Group(s) (TEG(s)) at UE side are defined:

-	UE Rx TEG is associated with one or more DL measurements, which have the Rx timing error difference within a certain margin.

-	UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, which have the 'Rx timing errors+Tx timing errors' difference within a certain margin.

The UE may be configured to report, subject to UE capability, via high layer parameter nr-UE-RxTEG-Request, the association information of DL RSTD measurement(s) with UE Rx TEG(s) via higher layer parameter nr-UE-Rx-TEG-ID when the UE reports the DL RSTD measurement(s). The UE may report up to 4 RSTD measurements associated with different DL PRS resources per UE Rx TEG per dl-PRS-ID.

The UE may report a UE Rx TEG ID via higher layer parameter nr-UE-Rx-TEG-ID for a RSTD reference time dl-PRS-ReferenceInfo and a UE Rx TEG ID for each DL RSTD measurement, where the DL RSTD can be DL RSTD measurement in NR-DL-TDOA-MeasElement and/or NR-DL-TDOA-AdditionalMeasurementElement.

The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs subject to UE capability, RSTD measurements on a PRS resource associated with a dl-PRS-ID using up to 8 different UE Rx TEGs with the same dl-PRS-ReferenceInfo. The higher layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs applies to all DL PRS positioning frequency layers.

The UE may be provided with association information of DL PRS resource(s) with Tx TEGs via higher layer parameter dl-prs-trp-Tx-TEG-ID for a dl-PRS-ID.

The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request, subject to UE capability, the association information of UE Rx-Tx time difference measurement(s) with UE RxTx TEG(s) via higher layer parameter nr-UE-RxTx-TEG-ID. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different DL PRS resources per UE RxTx TEG per dl-PRS-ID.

The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request, subject to UE capability, the association information of UE Rx-Tx time difference measurement(s) with the UE Rx TEG(s) and UE Tx TEG(s) via the higher layer parameters of nr-UE-Rx-TEG-ID, and nr-UE-Tx-TEG-Index. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different DL PRS resources per UE Rx TEG per dl-PRS-ID.

The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs subject to UE capability, UE Rx-Tx time difference measurements on a PRS resource associated with a dl-PRS-ID using up to 8 different UE Rx TEGs. The high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs applies to all DL PRS positioning frequency layers.

The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTxTEGs subject to UE capability, UE Rx-Tx time difference measurements with the same UE Tx TEG using up to 8 different UE RxTx TEGs. The high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTxTEGs applies to all DL PRS positioning frequency layers.

The UE in RRC_INACTIVE mode is expected to prioritize the reception of any other DL signals and DL channels than the reception of DL PRS.

The UE in RRC_INACTIVE mode, subject to UE capability, is expected to process DL PRS outside or inside of the initial DL BWP. For DL PRS processing outside of the initial DL BWP, the UE may be configured with the same or different numerology and CP for PRS resources than those of the initial DL BWP. For DL PRS processing inside of the initial DL BWP, the UE is configured with the same numerology and CP for PRS resources as those of the initial DL BWP.

For a UE configured with preconfigured Measurement gap(s) for Positioning, when the UE receives an activation command, as described in clause 6.1.3.41 of [10, TS 38.321], for a preconfigured Measurement Gap for Positioning activation/deactivation, and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the command, the corresponding actions in [10, TS 38.321] and the UE assumptions shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

For a UE configured with PRS Processing Window(s), when the UE receives an activation/deactivation command, as described in clause 6.1.3.42 of [10, TS 38.321], for a PRS processing window activation, and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the command, the corresponding actions in [10, TS 38.321] and the UE assumptions shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. The UE is not expected to be indicated with more than 4 activated PRS processing windows by higher layer parameter DL-PPW-PreConfig across all active DL BWPs and is not expected to be indicated with the activated PRS processing windows DL-PPW-PreConfig that overlap in time.

<omitted text>

##### 5.2.1.4.2	Report Quantity Configurations

A UE may be configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to either 'none', 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', 'cri-RSRP', 'cri-SINR', 'ssb-Index-RSRP', 'ssb-Index-SINR', 'cri-RI-LI-PMI-CQI', 'cri-RSRP-Capability[Set]Index', 'ssb-Index-RSRP-Capability[Set]Index', 'cri-SINR-Capability[Set]Index' or 'ssb-Index-SINR-Capability[Set]Index'.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'none', then the UE shall not report any quantity for the CSI-ReportConfig.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI', the UE shall report a preferred precoder matrix for the entire reporting band, or a preferred precoder matrix per subband, according to Clause 5.2.2.2.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-i1',

-	the UE expects, for that CSI-ReportConfig, to be configured with higher layer parameter codebookType set to 'typeI-SinglePanel' and pmi-FormatIndicator set to 'widebandPMI' and,

-	the UE shall report a PMI consisting of a single wideband indication ( in Clause 5.2.2.2.1) for the entire CSI reporting band.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-i1-CQI',

-	the UE expects, for that CSI-ReportConfig, to be configured with higher layer parameter codebookType set to 'typeI-SinglePanel' and pmi-FormatIndicator set to 'widebandPMI' and,

-	the UE shall report a PMI consisting of a single wideband indication ( in Clause 5.2.2.2.1) for the entire CSI reporting band. The CQI is calculated conditioned on the reported assuming PDSCH transmission with  precoders (corresponding to the same but different  in Clause 5.2.2.2.1), where the UE assumes that one precoder is randomly selected from the set of  precoders for each PRG on PDSCH, where the PRG size for CQI calculation is configured by the higher layer parameter pdsch-BundleSizeForCSI.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-CQI',

-	if the UE is configured with higher layer parameter non-PMI-PortIndication contained in a CSI-ReportConfig, r ports are indicated in the order of layer ordering for rank r and each CSI-RS resource in the CSI resource setting is linked to the CSI-ReportConfig based on the order of the associated NZP-CSI-RS-ResourceId in the linked CSI resource setting for channel measurement given by higher layer parameter resourcesForChannelMeasurement. The configured higher layer parameter non-PMI-PortIndication contains a sequence  of port indices, where  are the CSI-RS port indices associated with rank ν and  where is the number of ports in the CSI-RS resource. The UE shall only report RI corresponding to the configured fields of PortIndexFor8Ranks.

-	if the UE is not configured with higher layer parameter non-PMI-PortIndication, the UE assumes, for each CSI-RS resource in the CSI resource setting linked to the CSI-ReportConfig, that the CSI-RS port indices  are associated with ranks  where  is the number of ports in the CSI-RS resource.

-	When calculating the CQI for a rank, the UE shall use the ports indicated for that rank for the selected CSI-RS resource. The precoder for the indicated ports shall be assumed to be the identity matrix scaled by .

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP-Capability[Set]Index' or 'ssb-Index-RSRP-Capability[Set]Index',

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'disabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single report nrofReportedRS (higher layer configured) different CRI or SSBRI for each report setting.

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'enabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single reporting instance two different CRI or SSBRI for each report setting, where CSI-RS and/or SSB resources can be received simultaneously by the UE either with a single spatial domain receive filter, or with multiple simultaneous spatial domain receive filters.

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting-r17, the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single reporting instance nrofReportedGroups, if configured, group(s) of two CRIs or SSBRIs selecting one CSI-RS or SSB from each of the two CSI Resource Sets for the report setting, where CSI-RS and/or SSB resources of each group can be received simultaneously by the UE.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-SINR', 'ssb-Index-SINR', 'cri-SINR-Capability[Set]Index' or 'ssb-Index-SINR-Capability[Set]Index',

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'disabled', the UE shall report in a single report nrofReportedRS (higher layer configured) different CRI or SSBRI for each report setting.

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'enabled', the UE shall report in a single reporting instance two different CRI or SSBRI for each report setting, where CSI-RS and/or SSB resources can be received simultaneously by the UE.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRP', 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', 'cri-RI-LI-PMI-CQI', 'cri-SINR', or 'cri-SINR-Capability[Set]Index ', and Ks>1 resources are configured in the corresponding resource set for channel measurement, then the UE shall derive the CSI parameters other than CRI conditioned on the reported CRI, where CRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet for channel measurement, and (k+1)-th entry of associated csi-IM-Resource in the corresponding csi-IM-ResourceSet (if configured) or (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet (if configured for CSI-ReportConfig with reportQuantity set to 'cri-SINR' or 'cri-SINR-Capability[Set]Index ') for interference measurement. If Ks=2 CSI-RS resources are configured, each resource shall contain at most 16 CSI-RS ports. If 2<Ks≤8 CSI-RS resources are configured, each resource shall contain at most 8 CSI-RS ports.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is configured with Ks≥2 resources, two Resource Groups with K1≥1 resources in Group 1, K2≥1 resources in Group 2, K1+K2=Ks, and N Resource Pairs:

-	each resource can contain, subject to UE capability, at most 32 CSI-RS ports. For two Resource Groups with Ki  resources (i=1,2), if maxK1,K2=1, the resource in NZP-CSI-RS-ResourceSet shall contain at most 32 CSI-RS ports; if  maxK1,K2=2, each resource in NZP-CSI-RS-ResourceSet shall contain at most 16 CSI-RS ports; if  2<maxK1,K2<8, each resource in NZP-CSI-RS-ResourceSet shall contain at most 8 CSI-RS ports.

-	each of the N Resource Pairs is associated to a CRI value.

-	The CSI-ReportConfig may be configured with higher layer parameter sharedCMR. M1 and M2 are the numbers of resources associated to a CRI value, other than the N CRIs defined above, in Group 1 and Group 2, respectively, with M=M1+M2, such that the total number of CRI values configured for the CSI-ReportConfig is M+N.

-	If the higher layer parameter csi-ReportMode is set to 'Mode1' and the higher layer parameter numberOfSingleTRP-CSI-Mode1 is set to X∈{0}, M1=M2=0; otherwise,

-	if the higher layer parameter csi-ReportMode is set to 'Mode1' and the higher layer parameter numberOfSingleTRP-CSI-Mode1 is set to X∈{1,2}, or if csi-ReportMode is set to 'Mode2',

-	if sharedCMR is configured: M1=K1 and M2=K2; otherwise

-	if sharedCMR is not configured, only the resources in Group 1 and Group 2 that are not referred to in any Resource Pair are associated to M CRI values other than the N CRIs defined above.

-	If interference measurement is performed on CSI-IM, M+N resources are configured in the corresponding csi-IM-ResourceSet. The M resources for channel measurement defined above are resource-wise associated with the first M CSI-IM resources by the ordering of the CSI-RS resources and CSI-IM resources in the corresponding Resource Set. The N Resource Pairs for channel measurement are associated to the last N CSI-IM resources by the ordering of the CSI-RS Resource Pairs and CSI-IM resources in the CSI-IM Resource Set. The UE may assume that the two CSI-RS resources for channel measurement in a Resource Pair and the associated CSI-IM resource for interference measurement are resource-wise QCLed with respect to 'typeD'.

-	The UE is not expected to be configured with NZP CSI-RS for interference measurement other than the NZP CSI-RS resources for channel measurement configured in the N Resource Pairs.

-	The UE expects, for that CSI-ReportConfig, to be configured with higher layer parameter codebookType set to 'typeI-SinglePanel', and

-	The UE shall derive the CSI parameters other than CRI(s) conditioned on the reported CRI(s), as follows:

-	If the higher layer parameter csi-ReportMode is set to 'Mode1' and the higher layer parameter numberOfSingleTRP-CSI-Mode1 is set to X∈{0,1,2}, X+1 CRI(s) are reported:

-	one CRI k1 k1≥0 corresponds to the configured (k1+1)-th entry of the associated N Resource Pairs in the corresponding CSI-RS Resource Set for channel measurement, and (M+k1+1)-th entry of the corresponding CSI-IM Resource Set, if configured. The UE shall report two RIs, two PMIs, two LIs (if configured), associated to the resource in Group 1 and the resource in Group 2, respectively, of the (k1+1)-th Resource Pair, and one CQI; and

-	if X=1, one CRI k2 (k2≥0) corresponds to the configured (k2+1)-th entry of the associated M resources in the corresponding CSI-RS Resource Set for channel measurement, and (k2+1)-th entry of the corresponding CSI-IM Resource Set, if configured. The UE shall report one RI, one PMI, one LI (if configured) and one or two CQIs conditioned on CRI k2; or

-	if X=2, one CRI k2 (k2≥0) corresponds to the configured (k2+1)-th entry of the associated M1 resources in Group 1 of the corresponding CSI-RS Resource Set for channel measurement, and (k2+1)-th entry of the associated resources in the corresponding CSI-IM Resource Set, if configured, and one CRI k3 (k3≥0) corresponds to the configured (k3+1)-th entry of the associated M2 resources in Group 2 of the corresponding CSI-RS Resource Set for channel measurement, and (M1+k3+1)-th entry of the corresponding CSI-IM Resource Set, if configured. The UE shall report one RI, one PMI, one LI (if configured) and one or two CQIs conditioned on CRI k2 and one RI, one PMI, one LI (if configured) and one or two CQIs conditioned on CRI k3.

-	If the higher layer parameter csi-ReportMode is set to 'Mode2', one CRI k1 (k1≥0) is reported, which corresponds to the (k1+1)-th entry of the M+N resources or Resource Pairs in the corresponding CSI-RS Resource Set for channel measurement, and (k1+1)-th entry of the associated resources in the corresponding CSI-IM Resource Set, if configured. The first M codepoints of the CRI correspond to resources associated to Group 1 and Group 2. The last N codepoints of the CRI correspond to the N configured Resource Pairs. The UE shall report one RI, one PMI, one LI, if configured, and one or two CQIs conditioned on CRI k1 if k1<M; or two RIs, two PMIs, two LIs, if configured, associated to the resource in Group 1 and the resource in Group 2, respectively, of the (k1-M+1)-th Resource Pair, and one CQI, otherwise.

-	For a reported CRI corresponding to an entry of the N Resource Pairs configured in the corresponding CSI-RS Resource Set for channel measurement:

-	the UE shall not report a total number of layers larger than four.

-	the two RIs are reported with a joint RI index corresponding to one of the four rank combinations: 1,1, 1,2, 2,1, {2,2}.

-	The CodebookConfig in CSI-ReportConfig can be configured with two RI restriction parameters. One parameter applies to a reported RI when conditioned on a CRI corresponding to an entry of the M CSI-RS resources defined above. Another parameter applies to a reported joint RI index when conditioned on a CRI corresponding to an entry of the N Resource Pairs and indicates one or more of the four rank combinations that are allowed to correspond to the reported PMIs and RIs.

-	The CodebookConfig in CSI-ReportConfig can be configured with two Codebook Subset Restrictions. The first restriction applies to a reported PMI associated to a CSI-RS resource in Group 1. The second restriction applies to a reported PMI associated to a CSI-RS resource in Group 2.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'ssb-Index-RSRP' or 'ssb-Index-RSRP-Capability[Set]Index', the UE shall report SSBRI, where SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'ssb-Index-SINR' or 'ssb-Index-SINR-Capability[Set]Index', the UE shall derive L1-SINR conditioned on the reported SSBRI, where SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList  in the corresponding CSI-SSB-ResourceSet for channel measurement, and (k+1)-th entry of associated csi-IM-Resource in the corresponding csi-IM-ResourceSet (if configured) or (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet (if configured) for interference measurement.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', ' cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI' or 'cri-RI-LI-PMI-CQI', then the UE is not expected to be configured with more than 8 CSI-RS resources in a CSI-RS resource set contained within a resource setting that is linked to the CSI-ReportConfig.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-LI-PMI-CQI', UE does not expect the CSI-ReportConfig to be configured with higher layer parameter codebookType set to 'typeII-r16' or 'typeII-PortSelection-r16'.

If the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'cri-SINR', 'none', 'cri-RSRP-Capability[Set]Index' or 'cri-SINR-Capability[Set]Index and the CSI-ReportConfig is linked to a resource setting configured with the higher layer parameter resourceType set to 'aperiodic', then the UE is not expected to be configured with more than 16 CSI-RS resources in a CSI-RS resource set contained within the resource setting.

The LI indicates which column of the precoder matrix of the reported PMI corresponds to the strongest layer of the codeword corresponding to the largest reported wideband CQI. If two wideband CQIs are reported and have equal value, the LI corresponds to strongest layer of the first codeword. If the UE is configured with a CSI-ReportConfig with reportQuantity set to 'cri-RI-LI-PMI-CQI' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is configured with two Resource Groups and N Resource Pairs, and the UE reports a CRI associated to a Resource Pair, and a rank combination {ν1,ν2}, the first LI indicates which column of the precoder matrix of the first reported PMI corresponds to the strongest of the first ν1 layers of the codeword and the second LI indicates which column of the precoder matrix of the second reported PMI corresponds to the strongest of the last ν2 layers of the codeword.

For operation with shared spectrum channel access, if the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI' or 'cri-RI-LI-PMI-CQI', the UE shall derive:

-	the CSI parameters without averaging two or more instances of any periodic or semi-persistent nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet for channel measurement or for interference measurement located in different DL transmissions,

-	the instances of the nzp-CSI-RS-Resources are not in the same channel occupancy duration indicated by DCI format 2_0, if the UE is provided at least one of SlotFormatIndicator or co-DurationList; or

-	the instances of the nzp-CSI-RS-Resources occur within a set of consecutive symbols which are not all occupied by PDSCH(s) and/or aperiodic CSI-RS(s) indicated by DCI formats, if any, and the corresponding PDCCH(s), if the UE is neither provided with CO-DurationPerCell nor SlotFormatIndicator, but is provided with csi-RS-ValidationWith-DCI

-	the interference measurements for computing CSI value based on periodic/semi-persistent CSI-IM measured only in OFDM symbol(s) that fulfill the same conditions under which the UE is expected to receive periodic/semi-persistent CSI-RS as described in Clause 11.1 and Clause 11.1.1 of [6, TS 38.213].

If the UE is configured with the higher layer parameter SSB-MTC-AdditionalPCI, the UE is allowed to report in a single reporting instance up to four SSBRIs for each report setting, where SSB resources are associated with PCI indices referring to the PCI of the serving cell and PCI(s) different from the PCI of the serving cell within the set of PCIs configured.

<omitted text>

#### 5.2.1.6	CSI processing criteria

The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC in a component carrier, and simultaneousCSI-ReportsAllCC across all component carriers. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU(n), the UE is not required to update the N-M requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤M≤N is the largest value such that n=0M-1OCPU(n)≤NCPU-L  holds.

A UE is not expected to be configured with an aperiodic CSI trigger state containing more than NCPU Reporting Settings. Processing of a CSI report occupies a number of CPUs for a number of symbols as follows:

-	OCPU=0  for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info configured

-	OCPU=1  for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR', 'ssb-Index-SINR', 'cri-RSRP-Capability[Set]Index', 'ssb-Index-RSRP-Capability[Set]Index', 'cri-SINR-Capability[Set]Index', 'ssb-Index-SINR-Capability[Set]Index ' or 'none' (and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured)

-	for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', or 'cri-RI-LI-PMI-CQI',

-	if max{ µPDCCH, µCSI-RS, µUL} ≤ 3, and if a CSI report is aperiodically triggered without transmitting a PUSCH with either transport block or HARQ-ACK or both when L = 0 CPUs are occupied, where the CSI corresponds to a single CSI with wideband frequency-granularity and to at most 4 CSI-RS ports in a single resource without CRI report and where codebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', OCPU=NCPUOCPU=NCPU,

-	If a CSI-ReportConfig is configured with codebookType set to 'typeI-SinglePanel' and the corresponding CSI-RS Resource Set for channel measurement is configured with two Resource Groups and N Resource Pairs, OCPU=X⋅N+M, where X is the number of CPUs occupied by a pair of CMRs subject to mTRP-CSI-numCPU-r17 and M is defined in clause 5.2.1.4.2.

-	otherwise, OCPU=KsOCPU=KS, where Ks Ks is the number of CSI-RS resources in the CSI-RS resource set for channel measurement.

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', the CPU(s) are occupied for a number of OFDM symbols as follows:

-	A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the report.

-	An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the CPU occupation duration, the PDCCH candidate that ends later in time is used.

-	An initial semi-persistent CSI report on PUSCH after the PDCCH trigger occupies CPU(s) from the first symbol after the PDCCH until the last symbol of the scheduled PUSCH carrying the report. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the CPU occupation duration, the PDCCH candidate that ends later in time is used.

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured, the CPU(s) are occupied for a number of OFDM symbols as follows:

-	A semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation, until Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion.

-	An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol between Z3 symbols after the first symbol after the PDCCH triggering the CSI report and Z3' symbols after the last symbol of the latest one of each CSI-RS/SSB resource for channel measurement for L1-RSRP computation.

where (Z3,Z3') are defined in the table 5.4-2.

In any slot, the UE is not expected to have more active CSI-RS ports or active CSI-RS resources in active BWPs than reported as capability. NZP CSI-RS resource is active in a duration of time defined as follows. For aperiodic CSI-RS, starting from the end of the PDCCH containing the request and ending at the end of the scheduled PUSCH containing the report associated with this aperiodic CSI-RS. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinking, for the purpose of determining the NZP CSI-RS resource active duration, the PDCCH candidate that ends later in time among the two linked PDCCH candidates is used. For semi-persistent CSI-RS, starting from the end of when the activation command is applied, and ending at the end of when the deactivation command is applied. For periodic CSI-RS, starting when the periodic CSI-RS is configured by higher layer signalling, and ending when the periodic CSI-RS configuration is released. If a CSI-RS resource is referred N times by one or more CSI Reporting Settings, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted N times. For a CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, if a CSI-RS resource is referred X times by one of the M CSI-RS resources, where M is defined in clause 5.2.1.4.2, and/or one or two Resource Pairs, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted X times.

<omitted text>

## 6.1	UE procedure for transmitting the physical uplink shared channel

PUSCH transmission(s) can be dynamically scheduled by an UL grant in a DCI, or the transmission can correspond to a configured grant Type 1 or Type 2. The configured grant Type 1 PUSCH transmission is semi-statically configured to operate upon the reception of higher layer parameter of configuredGrantConfig including rrc-ConfiguredUplinkGrant without the detection of an UL grant in a DCI. The configured grant Type 2 PUSCH transmission is semi-persistently scheduled by an UL grant in a valid activation DCI according to clause 10.2 of [6, TS 38.213] after the reception of higher layer parameter configuredGrantConfig not including rrc-ConfiguredUplinkGrant. If configuredGrantConfigToAddModList is configured, more than one configured grant configuration of configured grant Type 1 and/or configured grant Type 2 may be active at the same time on an active BWP of a serving cell.

The UE can be configured with a list of up to 64 TCI-UL-State configurations within the higher layer parameter BWP-UplinkDedicated. Each TCI-UL-State configuration contains a parameter for configuring one reference signal, if applicable, for determining UL TX spatial filter for dynamic-grant and configured-grant based PUSCH and PUCCH resource in a CC, and SRS.

For the PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant activated by DCI format 0_0 or 0_1, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubset, maxRank, scaling of UCI-OnPUSCH, which are provided by pusch-Config. For the PUSCH transmission corresponding to a Type 2 configured grant activated by DCI format 0_2, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubsetDCI-0-2, maxRankDCI-0-2, scaling of UCI-OnPUSCH, resourceAllocationType1GranularityDCI-0-2 provided by pusch-Config. If the UE is provided with transformPrecoder in configuredGrantConfig, the UE applies the higher layer parameter tp-pi2BPSK, if provided in pusch-Config, according to the procedure described in clause 6.1.4 for the PUSCH transmission corresponding to a configured grant. When the UE is configured dl-OrJoint-TCIStateList or TCI-UL-State, the UE shall perform PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant or a dynamic grant according to the spatial relation, if applicable, with a reference to the RS for determining UL Tx spatial filter. The RS is determined based on an RS configured with qcl-Type set to 'typeD' of the indicated TCI-State or an RS in the indicated TCI-UL-State. The reference RS in the indicated TCI-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. The reference RS in the indicated TCI-UL-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, an SRS resource in an SRS resource set with the higher layer parameter usage set to 'beamManagement', or SS/PBCH block associated with the same or different PCI from the PCI of the serving cell.

For the PUSCH retransmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, the parameters in pusch-Config are applied for the PUSCH transmission except for p0-NominalWithoutGrant, p0-PUSCH-Alpha, powerControlLoopToUse, pathlossReferenceIndex described in clause 7.1 of [6, TS 38.213], mcs-Table, mcs-TableTransformPrecoder described in clause 6.1.4.1 and transformPrecoder described in clause 6.1.3.

For a UE configured with two uplinks in a serving cell, PUSCH retransmission for a TB on the serving cell is not expected to be on a different uplink than the uplink used for the PUSCH initial transmission of that TB.

A UE shall upon detection of a PDCCH with a configured DCI format 0_0, 0_1 or 0_2 transmit the corresponding PUSCH as indicated by that DCI unless the UE does not generate a transport block as described in [10, TS38.321]. Upon detection of a DCI format 0_1 or 0_2  with 'UL-SCH indicator' set to '0' and with a non-zero 'CSI request' where the associated reportQuantity in CSI-ReportConfig set to 'none' for all CSI report(s) triggered by 'CSI request' in this DCI format 0_1 or 0_2, the UE ignores all fields in this DCI except the 'CSI request' and the UE shall not transmit the corresponding PUSCH as indicated by this DCI format 0_1 or 0_2. When the UE is scheduled with multiple PUSCHs by a DCI, HARQ process ID indicated by this DCI applies to the first PUSCH not overlapping with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, HARQ process ID is then incremented by 1 for each subsequent PUSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPUSCH applied if nrofHARQ-ProcessesForPUSCH is provided, or with modulo operation of nrofHARQ-ProcessesForPUSCH-r17 applied if nrofHARQ-ProcessesForPUSCH-r17 is provided, or with modulo operation of 16 applied, otherwise. HARQ process ID is not incremented for PUSCH(s) not transmitted if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to transmit a PUSCH that overlaps in time with another PUSCH. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet for the active BWP of a serving cell and PDCCHs that schedule two non-overlapping in time domain PUSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex, for any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i on a scheduling cell,, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i of the scheduling cell. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the PDCCH ending in symbol i, the PDCCH candidate that ends later in time is used. The UE is not expected to be scheduled to transmit another PUSCH by a DCI format 0_0 with CRC scrambled by TC-RNTI, for a given HARQ process with the DCI received before the end of the expected transmission of the last PUSCH for that HARQ process if the latter is scheduled by a DCI format 0_0 with CRC scrambled by TC-RNTI or by an UL grant in RA Response. The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0, 0_1 or 0_2 scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI for a given HARQ process with the DCI received before the end of the expected transmission of the last PUSCH for that HARQ process if the latter is scheduled by a DCI with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI.

If a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet for the active BWP of a serving cell and PDCCHs that schedule two non-overlapping in time domain PUSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex, for any two HARQ process IDs  in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH associated with a value of coresetPoolIndex ending in symbol i, the UE can be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH associated with a different value of coresetPoolIndex that ends later than symbol i.

A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a PUSCH on a given serving cell overlapping in time with a transmission occasion, where the UE is allowed to transmit a PUSCH with configured grant according to [10, TS38.321], starting in a symbol j on the same serving cell if the end of symbol i is not at least N2 symbols before the beginning of symbol j, if the UE is not provided prioritizationBetweenLP-DG-PUSCHandHP-CG-PUSCH or prioritizationBetweenHP-DG-PUSCHandLP-CG-PUSCH, or the UE is provided prioritizationBetweenLP-DG-PUSCHandHP-CG-PUSCH or prioritizationBetweenHP-DG-PUSCHandLP-CG-PUSCH and the two PUSCHs have the same priority index as described in Clause 9 of [6, TS 38.213]. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH.

If a UE receives an ACK for a given HARQ process in CG-DFI in a PDCCH ending in symbol i to terminate a transport block repetition in a PUSCH transmission with a configured grant on a given serving cell with the same HARQ process after symbol i, the UE is expected to terminate the repetition of the transport block in a PUSCH transmission starting from a symbol j if the gap between the end of PDCCH of symbol i and the start of the PUSCH transmission in symbol j is equal to or more than N2 symbols. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH and the subcarrier spacing of the PDCCH indicating CG-DFI. A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a PUSCH on a given serving cell for a given HARQ process, if there is a transmission occasion where the UE is allowed to transmit a PUSCH with configured grant according to [10, TS38.321] with the same HARQ process on the same serving cell starting in a symbol j after symbol i, and if the gap between the end of PDCCH and the beginning of symbol j is less than N2 symbols. The value N2 in symbols is determined according to the UE processing capability defined in clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH.

For PUSCH scheduled by DCI format 0_0 on a cell, the UE shall transmit PUSCH according to the spatial relation, if applicable, corresponding to the dedicated PUCCH resource with the lowest ID within the active UL BWP of the cell, as described in Clause 9.2.1 of [6, TS 38.213]. If the dedicated PUCCH resource with the lowest ID within the active UL BWP of the cell corresponds to two spatial relations, the UE shall transmit the PUSCH according to the spatial relation with the lower ID.

For PUSCH scheduled by DCI format 0_0 on a cell and if the higher layer parameter enableDefaultBeamPL-ForPUSCH0-0 is set 'enabled', the UE is not configured with PUCCH resources on the active UL BWP and the UE is in RRC connected mode, the UE shall transmit PUSCH according to the spatial relation, if applicable, with a reference to the RS configured with qcl-Type set to 'typeD' corresponding to the QCL assumption of the CORESET with the lowest ID on the active DL BWP of the cell. If the CORESET is indicated with two TCI states, sfnSchemePdcch is configured and the UE supports sfn-DefaultUL-BeamSetup-r17, the UE shall use the first TCI state as the QCL assumption.

For PUSCH scheduled by DCI format 0_0 on a cell and if the higher layer parameter enableDefaultBeamPL-ForPUSCH0-0 is set 'enabled', the UE is configured with PUCCH resources on the active UL BWP where all the PUCCH resource(s) are not configured with any spatial relation and the UE is in RRC connected mode, the UE shall transmit PUSCH according to the spatial relation, if applicable, with a reference to the RS configured with qcl-Type set to 'typeD' corresponding to the QCL assumption of the CORESET with the lowest ID on the active DL BWP of the cell in case CORESET(s) are configured on the cell. If the CORESET is indicated with two TCI states, sfnSchemePdcch is configured and the UE supports sfn-DefaultUL-BeamSetup-r17, the UE shall use the first TCI state as the QCL assumption.

For uplink, 16 HARQ processes per cell are supported by the UE, or subject to UE capability, a maximum of 32 HARQ processes per cell as defined in [13, TS 38.306]. The number of processes the UE may assume will at most be used for the uplink is configured to the UE for each cell separately by higher layer parameter nrofHARQ-ProcessesForPUSCH, or nrofHARQ-ProcessesForPUSCH-r17, and when no configuration is provided the UE may assume a default number of 16 processes.

<omitted text>

### 6.2.1	UE sounding procedure

The UE may be configured with one or more Sounding Reference Signal (SRS) resource sets as configured by the higher layer parameter SRS-ResourceSet or SRS-PosResourceSet. For each SRS resource set configured by SRS-ResourceSet, a UE may be configured with SRS resources (higher layer parameter SRS-Resource), where the maximum value of K is indicated by UE capability [13, 38.306]. When SRS resource set is configured with the higher layer parameter SRS-PosResourceSet, a UE may be configured with K ≥1 SRS resources (higher layer parameter SRS-PosResource), where the maximum value of K is 16. The SRS resource set applicability is configured by the higher layer parameter usage in SRS-ResourceSet. When the higher layer parameter usage is set to 'beamManagement', only one SRS resource in each of multiple SRS resource sets may be transmitted at a given time instant, but the SRS resources in different SRS resource sets with the same time domain behaviour in the same BWP may be transmitted simultaneously.

For the SRS resource set(s) configured in srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage set to 'antennaSwitching' or 'beamManagement', the UE expects the same SRS resource set(s) with the same usage being configured in srs-ResourceSetToAddModList.

When the UE is configured dl-OrJoint-TCIStateList or TCI-UL-State, the UE can assume that SRS resource(s) in any SRS resource set, except SRS resource set for positioning and an SRS resource set configured with followUnifiedTCIstateSRS, can be configured with TCI-State or TCI-UL-State or updated as described in clause 6.1.3.59 or 6.1.3.60 of [10, TS 38.321]. The reference RS in the TCI-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. The reference RS in the TCI-UL-State(s) can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, an SRS resource with the higher layer parameter usage set to 'beamManagement', or SS/PBCH block associated with the same or different PCI from the PCI of the serving cell.

If an SRS resource set, except an SRS resource set for positioning, is configured with followUnifiedTCIstateSRS, the UE shall transmit the target SRS resource(s) within the SRS resource set according to the spatial relation, if applicable, with a reference to the RS used for determining UL TX spatial filter. The RS is determined based on an RS configured with qcl-Type set to 'typeD' in QCL-Info of the indicated TCI-State or an RS in the indicated TCI-UL-State. The reference RS in the indicated TCI-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. The reference RS in the indicated TCI-UL-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, an SRS resource with the higher layer parameter usage set to 'beamManagement', or SS/PBCH block associated with the same or different PCI from the PCI of the serving cell.

For aperiodic SRS at least one state of the DCI field is used to select at least one out of the configured SRS resource set(s).

The following SRS parameters are semi-statically configurable by higher layer parameter SRS-Resource or SRS-PosResource.

-	srs-ResourceId or SRS-PosResourceId determines SRS resource configuration identity.

-	Number of SRS ports, as defined by the higher layer parameter nrofSRS-Ports and described in clause 6.4.1.4 of [4, TS 38.211]. If not configured, nrofSRS-Ports is 1.

-	Time domain behaviour of SRS resource configuration as indicated by the higher layer parameter resourceType, which may be periodic, semi-persistent, aperiodic SRS transmission as defined in clause 6.4.1.4 of [4, TS 38.211].

-	Slot level periodicity and slot level offset as defined by the higher layer parameters periodicityAndOffset-p or periodicityAndOffset-sp for an SRS resource of type periodic or semi-persistent. The UE is not expected to be configured with SRS resources in the same SRS resource set SRS-ResourceSet or SRS-PosResourceSet with different slot level periodicities. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a slot level offset is defined by the higher layer parameter slotOffset. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a list of up to four different available slot offset values from the reference slot n + k to the slot where the aperiodic SRS resource set is transmitted where n is the slot with triggering DCI and k is slotOffset, can be configured by the higher layer parameter availableSlotOffsetList. The parameter availableSlotOffsetList can be configured up to 4 different values. For an SRS-PosResourceSet configured with higher layer parameter resourceType set to 'aperiodic', the slot level offset is defined by the higher layer parameter slotOffset for each SRS resource.

-	Number of OFDM symbols in the SRS resource, starting OFDM symbol of the SRS resource within a slot including repetition factor R as defined by the higher layer parameter resourceMapping and described in clause 6.4.1.4 of [4, TS 38.211]. If R is not configured, then R is equal to the number of OFDM symbols in the SRS resource.

-	SRS bandwidth and , as defined by the higher layer parameter freqHopping and described in clause 6.4.1.4 of [4, TS 38.211]. If not configured, then= 0.

-	Frequency hopping bandwidth , as defined by the higher layer parameter freqHopping and described in clause 6.4.1.4 of [4, TS 38.211]. If not configured, then = 0.

-	Defining partial frequency sounding factor and start RB index for partial frequency sounding as defined by the higher layer parameters FreqScalingFactor PF and StartRBIndex kF, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then PF = 1 and kF,= 0.

-	Defining start RB index hopping for partial frequency sounding in different SRS frequency hopping periods for aperiodic/periodic/semi-persistent SRS based on the hopping pattern khop as described in clause 6.4.1.4.3 in [4, TS 38.211. If not configured, then start RB hopping is not enabled and khop is fixed to be 0 for all SRS symbols.

-	Defining frequency domain position and configurable shift, as defined by the higher layer parameters freqDomainPosition and freqDomainShift, respectively, and described in clause 6.4.1.4 of [4, TS 38.211]. If freqDomainPosition is not configured, freqDomainPosition is zero.

-	Cyclic shift, as defined by the higher layer parameter cyclicShift-n2, cyclicShift-n4, or cyclicShift-n8 for transmission comb value 2, 4 or 8, and described in clause 6.4.1.4 of [4, TS 38.211].

-	Transmission comb value, as defined by the higher layer parameter transmissionComb described in clause 6.4.1.4 of [4, TS 38.211].

-	Transmission comb offset, as defined by the higher layer parameter combOffset-n2, combOffset-n4, and combOffset-n8 for transmission comb value 2, 4, or 8, and described in clause 6.4.1.4 of [4, TS 38.211].

-	SRS sequence ID, as defined by the higher layer parameter sequenceId in clause 6.4.1.4 of [4, TS 38.211].

-	The configuration of the spatial relation between a reference RS and the target SRS, where the higher layer parameter spatialRelationInfo or spatialRelationInfoPos, if configured, contains the ID of the reference RS. The reference RS may be an SS/PBCH block, CSI-RS configured on serving cell indicated by higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise, or an SRS configured on uplink BWP indicated by the higher layer parameter uplinkBWP, and serving cell indicated by the higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise. When the target SRS is configured by the higher layer parameter SRS-PosResourceSet, the reference RS may also be a DL PRS configured on a serving cell or a non-serving cell indicated by the higher layer parameter dl-PRS, or an SS/PBCH block of a non-serving cell indicated by the higher layer parameter ssb-Ncell. If the UE is configured with dl-OrJoint-TCIStateList or TCI-UL-State, the reference RS may additionally be an SS/PBCH block associated with a PCI different from the PCI of the serving cell.

The UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying  adjacent OFDM symbols within the last 6 symbols of the slot, or at any symbol location within the slot if resourceMapping-r16 is provided subject to UE capability, where all antenna ports of the SRS resources are mapped to each symbol of the resource. When the SRS is configured with the higher layer parameter SRS-PosResourceSet the higher layer parameter resourceMapping-r16 in SRS-PosResource indicates an SRS resource occupying NS∈1,2,4,8,12 adjacent symbols anywhere within the slot. When the SRS is configured with the higher layer parameter SRS-ResourceSet, the higher layer parameter resourceMapping-r17 in SRS-Resource indicates an SRS resource occupying NS∈1,2,4,8,10,12,14 adjacent symbols anywhere within the slot.

If a PUSCH with a priority index 0 and SRS configured by SRS-Resource are transmitted in the same slot on a serving cell, the UE may only be configured to transmit SRS after the transmission of the PUSCH and the corresponding DM-RS.

If a PUSCH transmission with a priority index 1 or a PUCCH transmission with a priority index 1 would overlap in time with an SRS transmission on a serving cell, the UE does not transmit the SRS in the overlapping symbol(s).

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource is set to 'periodic':

-	if the UE is configured with the higher layer parameter spatialRelationInfo, spatialRelationInfo-PDC or spatialRelationInfoPos containing the ID of a reference 'ssb-Index', 'ssb-IndexServing', or 'ssb-IndexNcell', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo, spatialRelationInfo-PDC or spatialRelationInfoPos contains the ID of a reference 'csi-RS-Index' or 'csi-RS-IndexServing', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, if the higher layer parameter spatialRelationInfo, spatialRelationInfo-PDC or spatialRelationInfoPos containing the ID of a reference 'srs' or 'srs-spatialRelation', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS. When the SRS is configured by the higher layer parameter SRS-PosResource and if the higher layer parameter spatialRelationInfoPos contains the ID of a reference 'dl-PRS', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference DL PRS. When the SRS is configured by the higher layer parameter SRS-Resource and if the higher layer parameter spatialRelationInfo-PDC contains the ID of a reference 'dl-PRS-PDC', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference DL PRS for RTT-based propagation delay compensation according to clause 9.

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource is set to 'semi-persistent':

-	when a UE receives an activation command, as described in clause 6.1.3.17 or 6.1.3.36 of [10, TS 38.321], for an SRS resource, and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the corresponding actions in [10, TS 38.321] and the UE assumptions on SRS transmission corresponding to the configured SRS resource set shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. The activation command also contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the activated SRS resource set. When the SRS is configured with the higher layer parameter SRS-ResourceSet, each ID in the list refers to a reference SS/PBCH block, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the activation command if present, same serving cell as the SRS resource set otherwise, or SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the activation command if present, same serving cell and bandwidth part as the SRS resource set otherwise. When the SRS is configured with the higher layer parameter SRS-PosResourceSet, each ID in the list of reference signal IDs may refer to a reference SS/PBCH block on a serving or non-serving cell indicated by PCI field in the activation command, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the activation command if present, same serving cell as the SRS resource set otherwise, SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the activation command if present, same serving cell and bandwidth part as the SRS resource set otherwise, or DL PRS resource of a serving or non-serving cell associated with a dl-PRS-ID indicated by DL-PRS ID field in the activation command.

-	if an SRS resource in the activated resource set is configured with the higher layer parameter spatialRelationInfo or spatialRelationInfoPos, the UE shall assume that the ID of the reference signal in the activation command overrides the one configured in spatialRelationInfo or spatialRelationInfoPos.

-	when a UE receives a deactivation command [10, TS 38.321] for an activated SRS resource set, and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, the corresponding actions in [10, TS 38.321] and UE assumption on cessation of SRS transmission corresponding to the deactivated SRS resource set shall apply starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

-	if the UE is configured with the higher layer parameter spatialRelationInfo or spatialRelationInfoPos containing the ID of a reference 'ssb-Index', 'ssb-IndexServing', or 'ssb-IndexNcell' the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo or spatialRelationInfoPos contains the ID of a reference 'csi-RS-Index' or 'csi-RS-IndexServing', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, if the higher layer parameter spatialRelationInfo or spatialRelationInfoPos contains the ID of a reference 'srs' or 'srs-SpatialRelation', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS or of the reference semi-persistent SRS. When the SRS is configured by the higher layer parameter SRS-PosResourceSet and if the higher layer parameter spatialRelationInfoPos contains the ID of a reference 'dl-PRS', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference DL PRS.

If the UE has an active semi-persistent SRS resource configuration and has not received a deactivation command, the semi-persistent SRS configuration is considered to be active in the UL BWP which is active, otherwise it is considered suspended.

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource is set to 'aperiodic':

-	the UE receives a configuration of SRS resource sets,

-	the UE receives a downlink DCI, a group common DCI, or an uplink DCI based command where a codepoint of the DCI may trigger one or more SRS resource set(s). For SRS in a resource set with usage set to 'codebook' or 'antennaSwitching', the minimal time interval between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2  symbols and an additional time duration  Tswitch. Otherwise, the minimal time interval between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2 +14 symbols and an additional time duration Tswitch. The minimal time interval unit of OFDM symbol is counted based on the minimum subcarrier spacing given by min(µPDCCH, µUL) where µUL is given by min(µUL,carrier1, µUL,carrier2, µSRS) when the UE is configured with the higher layer parameter uplinkTxSwitchingOption set to 'dualUL' for uplink carrier aggregation, and by µSRS otherwise. µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively.

-	Tswitch, µUL,carrier1 and µUL,carrier2 are defined in clause 6.4.

-	When UE reporting [Triggering SRS only in DCI 0_1/0_2], the UE can be indicated with DCI 0_1 and 0_2 to trigger aperiodic SRS without data and without CSI as described in clause 7.3.1.1 of TS38.212. Otherwise, except for DCI format 0_1/0_2 with CRC scrambled by SP-CSI-RNTI, a UE is not expected to receive a DCI format 0_1/0_2 with UL-SCH indicator of "0" and CSI request of all zero(s) as described in clause 7.3.1.1 of [5, TS 38.212].

-	If the UE receives the DCI triggering aperiodic SRS in slot n and at least one resource set is configured with parameter availableSlotOffset across all configured BWPs in a component carrier except when SRS is configured with the higher layer parameter SRS-PosResource,

-	If ca-SlotOffset is configured, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in the (t + 1)-th available slot counting from slot ,

-	otherwise the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in the (t + 1)-th available slot counting from slot n⋅2μSRS2μPDCCH+k, where

-	k is configured via higher layer parameter slotOffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command, respectively;

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot, offset, SRSCA and μoffset,SRS are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

-	An available slot is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. From the first symbol carrying the SRS request DCI to the last symbol of the triggered SRS resource set, UE does not expect to receive SFI indication, UL cancellation indication or dynamic scheduling of DL channel/signal(s) on flexible symbol(s) that may change the determination of available slot.

-	t is configured via higher layer parameter availableSlotOffsetList with up to four different values of AvailableSlotOffset for each triggered SRS resources set and it is based on the subcarrier spacing of the triggered SRS transmission. When one or more SRS resource sets across all configured BWPs in a component carrier are configured, and at least one resource set is configured with availableSlotOffsetList parameter of more than one values, the indicated value of t is indicated by SOI field in DCI scheduling PUSCH/PDSCH and DCI 0_1/0_2 without data and without CSI request described in [5, TS 38.212]. The UE shall apply indicated value t specifically for those sets with configured availableSlotOffsetList parameter. When one or more SRS resource sets across all configured BWPs in a component carrier are configured and at least one resource set is configured with availableSlotOffsetList parameter, and the availableSlotOffsetList parameter for each SRS resource set has only one value, the UE shall apply the configured value specifically for those sets with configured availableSlotOffsetList parameter. For SRS resource set configured with availableSlotOffsetList parameter, each of resource set is configured with K values of AvailableSlotOffset. For SRS resource set configured without availableSlotOffsetList parameter, t = 0 is applied for the resource set.

-	If the UE receives the DCI triggering aperiodic SRS in slot n and none of the resource sets is configured with parameter availableSlotOffsetList across all configured BWPs in a component carrier except when SRS is configured with the higher layer parameter SRS-PosResourceKs=n⋅2μSRS2μPDCCH+k+Koffset⋅2μSRS2μKoffsetKoffset

- if the UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in slot ,

-	otherwise, the UE transmits aperiodic SRS in each of the triggered resource set(s) in slot Ks=n⋅2μSRS2μPDCCH+k+Koffset⋅2μSRS2μKoffset, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6 TS 38.213], and where

-	k is configured via higher layer parameter slotOffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

-	μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1.

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the  Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the  and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

-	If the UE receives the DCI triggering aperiodic SRS in slot n and when SRS is configured with the higher layer parameter SRS-PosResource, the UE transmits every aperiodic SRS resource in each of the triggered SRS resource set(s) in slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, Ks=n⋅2μSRS2μPDCCH+k+Koffset⋅2μSRS2μKoffset, otherwise, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6 TS 38.213], and where

-	k is configured via higher layer parameter slotOffset for each aperiodic SRS resource in each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

-	μKoffset is the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1.

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the  Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the  and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

-	if the UE is configured with the higher layer parameter spatialRelationInfo or spatialRelationInfoPos containing the ID of a reference 'ssb-Index', 'ssb-IndexServing' or 'ssb-IndexNcell', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo or spatialRelationInfoPos contains the ID of a reference 'csi-RS-Index' or 'csi-RS-IndexServing', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, or of the latest reference aperiodic CSI-RS. If the higher layer parameter spatialRelationInfo or spatialRelationInfoPos contains the ID of a reference 'srs' or 'srs-SpatialRelation', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS or of the reference semi-persistent SRS or of the reference aperiodic SRS. When the SRS is configured by the higher layer parameter SRS-PosResourceSet and if the higher layer parameter spatialRelationInfoPos contains the ID of a reference 'dl-PRS', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference DL PRS.

-	when a UE receives an spatial relation update command, as described in clause 6.1.3.26 of [10, TS 38.321], for an SRS resource configured with the higher layer parameter SRS-Resource, and when the HARQ-ACK corresponding to the PDSCH carrying the update command is transmitted in slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions on updating spatial relation for the SRS resource shall be applied for SRS transmission starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. The update command contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the updated SRS resource set. Each ID in the list refers to a reference SS/PBCH block, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the update command if present, same serving cell as the SRS resource set otherwise, or SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the update command if present, same serving cell and bandwidth part as the SRS resource set otherwise. When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', the UE shall not expect to be configured with different spatial relations for SRS resources in the same SRS resource set.

The UE is not expected to be configured with different time domain behavior for SRS resources in the same SRS resource set. The UE is also not expected to be configured with different time domain behavior between SRS resource and associated SRS resources set.

For operation in the same carrier, the UE is not expected to be configured on overlapping symbols with a SRS resource configured by the higher layer parameter SRS-PosResource and a SRS resource configured by the higher layer parameter SRS-Resource with resourceType of both SRS resources as 'periodic'.

For operation in the same carrier, the UE is not expected to be activated or triggered to transmit SRS on overlapping symbols with a SRS resource configured by the higher layer parameter SRS-PosResource and a SRS resource configured by the higher layer parameter SRS-Resource with resourceType of both SRS resources as 'semi-persistent' or 'aperiodic'.

For operations in the same carrier, the UE is not expected to be configured on overlapping symbols with more than one SRS resources configured by the higher layer parameter SRS-PosResource with resourceType of the SRS resources as 'periodic'.

For operations in the same carrier, the UE is not expected to be activated or triggered to transmit SRS on overlapping symbols with more than one SRS resources configured by the higher layer parameter SRS-PosResource with resourceType of the SRS resources as 'semi-persistent' or 'aperiodic'.

For intra-band and inter-band CA operations, a UE can simultaneously transmit more than one SRS resource configured by SRS-PosResource on different CCs, subject to UE's capability

For intra-band and inter-band CA operations, a UE can simultaneously transmit more than one SRS resource configured by SRS-PosResource and SRS-Resource on different CCs, subject to UE's capability.

The SRS request field [5, TS38.212] in DCI format 0_1, 1_1, 0_2 (if SRS request field is present), 1_2 (if SRS request field is present) indicates the triggered SRS resource set given in Table 7.3.1.1.2-24 of [5, TS 38.212]. The 2-bit SRS request field in DCI format 2_3 indicates the triggered SRS resource set given in clause 7.3 of [5, TS 38.212] and defined by the entries of the higher layer parameter srs-ResourceSetToAddModList if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB', or indicates the SRS transmission on a set of serving cells configured by higher layers if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA'.

For PUCCH and SRS on the same carrier, a UE shall not transmit SRS when semi-persistent or periodic SRS is configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s), or only L1-SINR report(s). A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK, link recovery request (as defined in clause 9.2.4 of [6, 38.213]) and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s).

In case of intra-band contiguous carrier aggregation, or in inter-band or intra-band non-contiguous CA band combination if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by UE, the UE is not expected to be indicated with a SRS transmission from a carrier and to be configured or scheduled with a PUSCH/UL DM-RS/UL PT-RS/PUCCH transmission from a different carrier in the same symbol.

In case of intra-band contiguous carrier aggregation, or in inter-band CA band combination if simultaneous SRS and PRACH transmissions are not supported by UE, or in case of intra-band non-contiguous CA band combination if the UE is not configured with higher layer parameter intraBandNC-PRACH-simulTx-r17, the UE shall not transmit simultaneously SRS resource(s) from a carrier and PRACH from a different carrier.

In case of intra-band contiguous carrier aggregation, or in inter-band CA band combination if simultaneous SRS and MsgA transmissions are not supported by UE, the UE shall not transmit simultaneously SRS resource(s) from a carrier and MsgA from a different carrier.

In case a SRS resource with resourceType set as 'aperiodic' is triggered on the OFDM symbol(s) configured with periodic/semi-persistent SRS transmission, the UE shall transmit the aperiodic SRS resource and only the periodic/semi-persistent SRS symbol(s) overlapping within the symbol(s) are dropped, while the periodic/semi-persistent SRS symbol(s) that are not overlapped with the aperiodic SRS resource are transmitted. In case a SRS resource with resourceType set as 'semi-persistent' is triggered on the OFDM symbol(s) configured with periodic SRS transmission, the UE shall transmit the semi-persistent SRS resource and only the periodic SRS symbol(s) overlapping within the symbol(s) are dropped, while the periodic SRS symbol(s) that are not overlapped with the semi-persistent SRS resource are transmitted.

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', and a guard period of Y symbols is configured according to Clause 6.2.1.2, the UE shall use the same priority rules as defined above during the guard period as if SRS was configured.

When a spatialRelationInfo is activated/updated for a semi-persistent or aperiodic SRS resource configured by the higher layer parameter SRS-Resource by a MAC CE for a set of CCs/BWPs, where the applicable list of CCs provided by higher layer parameter simultaneousSpatial-UpdatedList1 or simultaneousSpatial-UpdatedList2 is determined by the indicated CC in the MAC CE, the spatialRelationInfo is applied for the semi-persistent or aperiodic SRS resource(s) with the same SRS resource ID for all the BWPs in the determined CCs.

When the higher layer parameter enableDefaultBeamPL-ForSRS is set 'enabled', and if the higher layer parameter spatialRelationInfo for the SRS resource, except for the SRS resource with the higher layer parameter usage in SRS-ResourceSet set to 'beamManagement' or for the SRS resource with the higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook' with configuration of associatedCSI-RS or for the SRS resource configured by the higher layer parameter SRS-PosResourceSet, is not configured in FR2 and if the UE is not configured with higher layer parameter(s) pathlossReferenceRS, and if the UE is not configured with different values of coresetPoolIndex in ControlResourceSets, and is not provided at least one TCI codepoint mapped with two TCI states, the UE shall transmit the target SRS resource in an active UL BWP of a CC,

-	according to the spatial relation, if applicable, with a reference to the RS configured with qcl-Type set to 'typeD' corresponding to the QCL assumption of the CORESET with the lowest controlResourceSetId in the active DL BWP in the CC. If the CORESET is activated with two TCI states, sfnSchemePdcch is configured and the UE supports sfn-DefaultUL-BeamSetup-r17, UE shall use the first TCI state as the QCL assumption.

-	according to the spatial relation, if applicable, with a reference to the RS configured with qcl-Type set to 'typeD' in the activated TCI state with the lowest ID applicable to PDSCH in the active DL BWP of the CC if the UE is not configured with any CORESET in the active DL BWP of the CC

<omitted text>

## 8.1	UE procedure for transmitting the physical sidelink shared channel

Each PSSCH transmission is associated with an PSCCH transmission.

That PSCCH transmission carries the 1st stage of the SCI associated with the PSSCH transmission; the 2nd stage of the associated SCI is carried within the resource of the PSSCH.

If the UE transmits SCI format 1-A on PSCCH according to a PSCCH resource configuration in slot n and PSCCH resource m, then for the associated PSSCH transmission in the same slot

-	one transport block is transmitted with up to two layers;

-	The number of layers (ʋ) is determined according to the 'Number of DMRS port' field in the SCI;

-	The set of consecutive symbols within the slot for transmission of the PSSCH is determined according to clause 8.1.2.1;

-	The set of contiguous resource blocks for transmission of the PSSCH is determined according to clause 8.1.2.2;

Transform precoding is not supported for PSSCH transmission.

Only wideband precoding is supported for PSSCH transmission.

The DM-RS antenna ports  in Clause 8.4.1.1.2 of [4, TS38.211] are determined according to the ordering of DM-RS port(s) given by Tables 8.3.1.1-3 in Clause 8.3.1.1 of [5, TS 38.212].

The UE shall set the contents of the SCI format 2-A as follows:

-	the UE shall set value of the 'HARQ process number' field as indicated by higher layers.

-	the UE shall set value of the 'NDI' field as indicated by higher layers.

-	the UE shall set value of the 'Redundancy version' field as indicated by higher layers.

-	the UE shall set value of the 'Source ID' field as indicated by higher layers.

-	the UE shall set value of the 'Destination ID' field as indicated by higher layers.

-	the UE shall set value of the 'HARQ feedback enabled/disabled indicator' field as indicated by higher layers.

-	the UE shall set value of the 'Cast type indicator' field as indicated by higher layers.

-	the UE shall set value of the 'CSI request' field as indicated by higher layers.

The UE shall set the contents of the SCI formats 2-B as follows:

-	the UE shall set value of the 'HARQ process number' field as indicated by higher layers.

-	the UE shall set value of the 'NDI' field as indicated by higher layers.

-	the UE shall set value of the 'Redundancy version' field as indicated by higher layers.

-	the UE shall set value of the 'Source ID' field as indicated by higher layers.

-	the UE shall set value of the 'Destination ID' field as indicated by higher layers.

-	the UE shall set value of the 'HARQ feedback enabled/disabled indicator' field as indicated by higher layers.

-	the UE shall set value of the 'Zone ID' field as indicated by higher layers.

-	the UE shall set the 'Communication range requirement' field as indicated by higher layers.

The UE shall set the contents of the SCI format 2-C as follows:

-	the UE shall set value of the 'HARQ process number' field as indicated by higher layers.

-	the UE shall set value of the 'NDI' field as indicated by higher layers.

-	the UE shall set value of the 'Redundancy version' field as indicated by higher layers.

-	the UE shall set value of the 'Source ID' field as indicated by higher layers.

-	the UE shall set value of the 'Destination ID' field as indicated by higher layers.

-	the UE shall set value of the 'HARQ feedback enabled/disabled indicator' field as indicated by higher layers.

-	the UE shall set value of the 'CSI request' field as indicated by higher layers.

-	the UE shall set value of 'Providing/Requesting indicator' field as indicated by higher layers.

-	if 'Providing/Requesting indicator' indicates SCI format 2-C is used to convey an explicit request for inter-UE coordination information:

-	the UE shall set value of the 'Priority' field as indicated by higher layers.

-	the UE shall set value of the 'Number of subchannels' field as indicated by higher layers.

-	the UE shall set value of the 'Resource reservation period' field as indicated by higher layers.

-	the UE shall set value of the 'Resource selection window location' field as indicated by higher layers.

-	the UE shall set value of the 'Resource set type' field as indicated by higher layers if higher layer parameter determineResourceSetTypeScheme1 is configured to 'UE-B's request'; otherwise this field is omitted.

-	if 'Providing/Requesting indicator' indicates SCI format 2-C is used to convey inter-UE coordination information:

-	the UE shall set value of the 'Resource set type' field as indicated by higher layers.

-	the UE shall set value of the 'Resource combination(s)' field (clause 8.1.5A) as indicated by higher layers.

-	the UE shall set value of the 'Lowest subchannel indices' as indicated by higher layers

-	the UE shall set value of the 'First resource location' as indicated by higher layers

-	the UE shall set value of the 'Reference slot location' as indicated by higher layers

<omitted text>

### 8.1.3	Modulation order, target code rate, redundancy version and transport block size determination

The redundancy version is given by the "Redundancy version" field in SCI format 2-A, 2-B or 2-C.

<omitted text>

### 8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this PSSCH/PSCCH transmission:

-	the resource pool from which the resources are to be reported;

-	L1 priority, prioTX;

-	the remaining packet delay budget;

-	the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot, LsubCH;

-	optionally, the resource reservation interval, Prsvp_TX, in units of msec.

-	if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission as part of re-evaluation or pre-emption procedure, the higher layer provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and a set of resources (r0',r1',r2',…) which may be subject to pre-emption.

-	it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) and (r0',r1',r2',…) , and T3 is equal to Tproc,1SL, where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP.

-	Optionally, the indication of resource selection mechanism(s), as sl-AllowedResourceSelectionConfig, which may comprise of full sensing only, partial sensing only, random resource selection only, or any combination(s) thereof.

The following higher layer parameters affect this procedure:

-	sl-SelectionWindowList: internal parameter T2min is set to the corresponding value from higher layer parameter sl-SelectionWindowList for the given value of prioTX.

-	sl-Thres-RSRP-List: this higher layer parameter provides an RSRP threshold for each combination pi, pj, where pi is the value of the priority field in a received SCI format 1-A and pj is the priority of the transmission of the UE selecting resources; for a given invocation of this procedure, pj = prioTX.

-	sl-RS-ForSensing selects if the UE uses the PSSCH-RSRP or PSCCH-RSRP measurement, as defined in clause 8.4.2.1.

-	sl-ResourceReservePeriodList

-	sl-SensingWindow: internal parameter T0 is defined as the number of slots corresponding to sl-SensingWindow msec

-	sl-TxPercentageList: internal parameter X for a given prioTX is defined as sl-TxPercentageList (prioTX) converted from percentage to ratio

-	sl-PreemptionEnable: if sl-PreemptionEnable is provided, and if it is not equal to 'enabled', internal parameter priopre is set to the higher layer provided parameter sl-PreemptionEnable.

-	Optionally, minimum number of Y slots as Ymin (sl-MinNumCandidateSlotsPeriodic), which indicates the minimum number of Y slots that are included in the candidate resources corresponding to periodic-based partial sensing operation.

-	Optionally, minimum number of Y' slots as Y'min   (sl-MinNumCandidateSlotsAperiodic), which indicates the minimum number of Y' slots that are included in the candidate resources corresponding to contiguous partial sensing operation.

-	Optionally, sensing occasion as sl-PBPS-OccasionReservePeriodList, which indicates the subset of periodicity values from sl-ResourceReservePeriodList used to determine periodic sensing occasions in periodic-based partial sensing. If not configured, all periodicity values from sl-ResourceReservePeriodList are used to determine periodic sensing occasions in periodic-based partial sensing.

-	Optionally, additional sensing occasions as sl-Additional-PBPS-Occasion, which indicates that UE additionally monitors periodic sensing occasions that correspond to a set of values. The possible values of the set at least includes the most recent sensing occasion before the first slot of the candidate slots for a given reservation periodicity and the last periodic sensing occasion prior to the most recent one for the given reservation periodicity. If not (pre-)configured, the UE monitors the most recent sensing occasion before the first slot of the candidate slots for the given periodicity used to determine periodic sensing occasions in periodic-based partial sensing.

-	Optionally, indication of the size in logical slots of contiguous partial sensing window for periodic transmissions as defined by the parameter sl-CPS-WindowPeriodic.

Optionally, indication of the size in logical slots of contiguous partial sensing window for aperiodic transmissions as defined by the parameter sl-CPS-WindowAperiodic.

-	Optionally, indication of whether UE is required to perform SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time as sl-PartialSensingInactiveTime.

The resource reservation interval, Prsvp_TX, if provided, is converted from units of msec to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7.

When the resource pool is (pre-)configured with sl-AllowedResourceSelectionConfig including full sensing, and full sensing is configured in the UE by higher layers, the UE performs full sensing.

When periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool, the resource pool is (pre-)configured with sl-AllowedResourceSelectionConfig including partial sensing, and partial sensing is configured by higher layer, the UE performs periodic-based partial sensing, unless other conditions state otherwise in the specification.

When a UE is triggered by higher layer to report resources for resource (re-)selection in a mode 2 Tx pool, the resource pool is (pre-)configured with sl-AllowedResourceSelectionConfig including partial sensing, and partial sensing is configured by higher layer, the UE performs contiguous partial sensing, unless stated otherwise in the specification.

Notation:

t'0SL, t'1SL, t'2SL,… denotes the set of slots which belongs to the sidelink resource pool and is defined in Clause 8.

The following steps are used:

1)	A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 for UE performing periodic-based partial sensing correspond to one candidate single-slot resource, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] for UE performing contiguous partial sensing if Prsvp_TX=0, correspond to one candidate single-slot resource, where

-	selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL  , where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP;

-	if  T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to  T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots).

-	Y is selected by UE where Y≥Ymin.

-	Y' is selected by UE where Y'≥Ymin'. When the UE performs contiguous partial sensing and if Prsvp_TX=0, the UE selects a set of Y' candidate slots with corresponding PBPS and/or CPS results (if available). If the number of candidate single-slot resources Y' is smaller than Ymin', it is up to UE implementation to include other candidate slots.

The total number of candidate single-slot resources is denoted by Mtotal.

2)	The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL), when the UE performs full sensing, where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which belongs to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

When the UE performs periodic-based partial sensing, the UE shall monitor slots at ty-k×Preserve''SL, where t'ySL is a slot of the selected candidate slots and Preserve' is Preserve converted to units of logical slot according to clause 8.1.7. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

The value of Preserve corresponds to sl-PBPS-OccasionReservePeriodList if (pre-)configured, otherwise, the values correspond to all periodicity from sl-ResourceReservePeriodList.

The UE monitors k sensing occasions determined by sl-Additional-PBPS-Occasion, as previously described, and not earlier than n –T0. For a given periodicity Preserve, the values of k correspond to the most recent sensing occasion earlier than t'y0SL-(Tproc,0SL+Tproc,1SL ) if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if sl-Additional-PBPS-Occasion is (pre-)configured. t'y0SL is the first slot of the selected Y candidate slots of PBPS.

When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot t'y0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. If Prsvp_TX≠0  the value of M is (pre-)configured with the sl-CPS-WindowPeriodic. If sl-CPS-WindowPeriodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

When the UE performs contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) disabled and if Prsvp_TX=0, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at least M consecutive logical slots before t'y0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than t'y0SL. The value of M is (pre-)configured with the sl-CPS-WindowAperiodic. If sl-CPS-WindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

Whether the UE is required to performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by higher layer parameter sl-PartialSensingInactiveTime. When it is enabled, if UE performs periodic-based partial sensing on the slots in SL DRX inactive time for a given periodicity corresponding to Preserve, UE monitors only the default periodic sensing occasions (most recent sensing occasion) from the slots; if UE performs contiguous partial sensing on the slots in SL DRX inactive time, UE monitors a minimum of M slots from the slots.

3)	The internal parameter Th(pi,pj)  is set to the corresponding value of RSRP threshold indicated by the i-th field in sl-Thres-RSRP-List, where i=pi+pj-1*8.

4)	The set SA is initialized to the set of all the candidate single-slot resources.

5)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

-	the UE has not monitored slot t'mSL in Step 2.

-	for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList and a hypothetical SCI format 1-A received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met.

5a)	If the number of candidate single-slot resources Rx,y remaining in the set SA is smaller than X⋅Mtotal, the set SA is initialized to the set of all the candidate single-slot resources as in step 4.

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

a)	the UE receives an SCI format 1-A in slot t'mSL, and 'Resource reservation period' field, if present, and 'Priority' field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213];

b)	the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX,prioTX;

c)	the SCI format received in slot t'mSL or the same SCI format which, if and only if the 'Resource reservation period' field is present in the received SCI format 1-A, is assumed to be received in slot(s) t'm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where if the UE is configured with full sensing by its higher layer, t'n'SL = n if slot n belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise slot t'n'SL is the first slot after slot n belonging to the set t'0SL,t'1SL,...,t'T'max-1SL; If UE is configured with partial sensing by its higher layer, t'n'SL=t'yiSL-Tproc,1SL if slot t'yiSL-Tproc,1SL belongs to the set t'0SL,t'1SL,...,t'T'max-1SL(t0'SL, t1'SL,⋯, tTmax'-1'SL), otherwise, slot t'n'SL is the first slot after slot t'yiSL-Tproc,1SL belonging to the set t'0SL,t'1SL,...,t'T'max-1SL(t0'SL, t1'SL,⋯, tTmax'-1'SL). Otherwise Q=1. If the UE is configured with full sensing by its higher layer, Tscal is set to selection window size T2 converted to units of msec. If UE is configured with partial sensing by its higher layer, Tscal=t'yLSL-(t'yiSL-Tproc,1SL) shall be converted to milliseconds, where slot t'yLSL is the last slot of the Y or Y' candidate slots. The slot t'yiSL is the first slot of the selected/remaining set of Y or Y' candidate slots.

6a)	This step is executed only if the procedure in clause 8.1.4A is triggered.

6b)	This step is executed only if the procedure in clause 8.1.4C is triggered.

7)	If the number of candidate single-slot resources remaining in the set SA is smaller than X⋅Mtotal, then Th(pi,pj) is increased by 3 dB for each priority value Th(pi,pj) and the procedure continues with step 4.

7a)	If sidelink DRX active time of RX UE is provided by the higher layer and there is no candidate single-slot resource remained within the sidelink DRX active time in the set SA, the UE based on its implementation additionally selects and includes at least one candidate single-slot resources within the sidelink DRX active time in the set SA.

The UE shall report set SA to higher layers.

If a resource ri from the set (r0,r1,r2,…) is not a member of SA, then the UE shall report re-evaluation of the resource ri to higher layers.

If a resource ri' from the set (r0',r1',r2',…) meets the conditions below then the UE shall report pre-emption of the resource ri' to higher layers.

-	ri' is not a member of SA, and

-	ri' meets the conditions for exclusion in step 6, with ThprioRX,prioTX set to the final threshold after executing steps 1)-7), i.e. including all necessary increments for reaching X⋅Mtotal, and

-	the associated priority prioRX, satisfies one of the following conditions:

-	sl-PreemptionEnable is provided and is equal to 'enabled' and prioTX>prioRX

-	sl-PreemptionEnable is provided and is not equal to 'enabled', and prioRX<priopre and prioTX>prioRX

Table 8.1.4-1: Tproc,0SL depending on sub-carrier spacing

| μSL | Tproc,0SL [slots] |
| --- | --- |
| 0 | 1 |
| 1 | 1 |
| 2 | 2 |
| 3 | 4 |

Table 8.1.4-2: Tproc,1SL depending on sub-carrier spacing

| μSL | Tproc,1SL [slots] |
| --- | --- |
| 0 | 3 |
| 1 | 5 |
| 2 | 9 |
| 3 | 17 |

When the UE performs periodic-based partial sensing and contiguous partial sensing, and when the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX≠0,

-	During the qth reservation period (q=0,1,2,…, Cresel-1), candidate resource set (SA) is initialized to the remaining Y candidate slots starting from slot t'yiSL and ending at the last slot of the Y candidate slots, where the slot indices of the remaining Y candidate slots are equal to t'y+q×Prsvp_TX'SL, where t'ySL is a slot index of Y candidate slots used in the initial resource (re)selection.

-	t'yiSL is the first candidate slot after slot n+T3.

-	The UE performs PBPS for the remaining Y candidate slots according to ty'-k×Preserve''SL, wheret'y'SL is a slot belonging to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection, where the values of k correspond to the most recent sensing occasion earlier than t'yiSL-(Tproc,0SL+Tproc,1SL ) if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if sl-Additional-PBPS-Occasion is (pre-)configured.

-	The UE performs CPS starting from M logical slots earlier than t'yiSL to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL.

-	By default, M is 31 unless (pre-)configured with another value by sl-CPS-WindowPeriodic.

When the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX≠0,

-	Candidate resource set (SA) is initialized to the remaining Y' candidate slots starting from slot t'yiSL and ending at the last slot of the Y' candidate slots, where t'yiSL is the first candidate slot after slot n+T3.

-	It is up to UE implementation that UE may perform PBPS for periodic sensing occasions after the resource (re)selection when higher layer parameter sl-MultiReserveResource is enabled

-	UE performs CPS starting from at least M consecutive logical slots earlier than t'yiSL to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL

-	For minimum size M of the contiguous partial sensing window [n+TA, n+TB], by default, M is 31 unless (pre-)configured with another value, by sl-CPS-WindowAperiodic.

When the minimum M slots for CPS cannot be guaranteed, UE senses in all available slots starting from the resource (re)selection trigger slot of the same TB to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL. The UE re-evaluation and pre-emption checking is based on all available sensing results after n –T0.

<omitted text>

### 8.1.4C	UE procedure for using a received non-preferred resource set

A UE configured with the higher layer parameter interUECoordinationScheme1 uses a received non-preferred resource set as follows when performing resource (re-)selection:

-	the UE excludes in Step 6b) of clause 8.1.4 resource(s) overlapping with the non-preferred resource set.

If it is not possible to meet the requirement that the number of candidate single-slot resources remaining in the set SA be at least X⋅Mtotal after excluding resource(s) overlapping with the received non-preferred resource set, it is up to UE implementation whether or not to take into account the received non-preferred resource set to meet such requirement.

The UE is not required to use any resource from the non-preferred resource set in its resource (re-)selection if that resource is earlier than (Tproc,0SL+ Tproc,1SL+ Tproc,2SL) after the resource of inter-UE coordination information transmission, where Tproc,2SL is equal to (Tproc,0SL+ Tproc,1SL) when only MAC CE is used for inter-UE coordination information transmission, or Tproc,2SL is equal to Tproc,0SL when MAC CE and SCI format 2-C are both used for inter-UE coordination information transmission.

The case when Tproc,2SL is equal to Tproc,0SL is assuming that SCI format 2-C is received.

<omitted text>

### 8.1.5A	UE procedure for determining slots and resource blocks indicated by a preferred or non-preferred resource set

The set of slots and resource blocks indicated by a set of preferred or non-preferred resource(s) is determined as described below.

The set of preferred or non-preferred resources r0,r1,r2,…, is indicated by a reference slot tref and M tuples (TRIVm,FRIVm,Prsvp,m), 1≤m≤M indicated by the 'resource combination' field, where for each tuple TRIVm is indicated by the 9 MSBs, followed by FRIVm and Prsvp,m (if present).

The reference slot tref is indicated by the 'Reference slot location' field as a combination of DFN index and slot index [5, TS 38.212], with the 10 MSBs indicating the DFN index. TRIVm and FRIVm are interpreted according to clause 8.1.5, with the following modifications:

-	the value of sl-MaxNumPerReserve is fixed to 3.

-	"slot where SCI format 1-A was received" is replaced by slot indicated as the first resource location of a TRIVm.

-	the first resource location of each TRIVm for m>1 is indicated by a slot offset tm in logical slots with respect to the reference slot tref; the first resource location of TRIV1 is at slot offset 0 with respect to the reference slot.

-	"the received SCI format 1-A, except the resource in the slot where SCI format 1-A was received" is replaced by "each tuple"

-	the starting sub-channel nsubCH,0start of the first resource of each tuple is separately indicated.

The starting sub-channel nsubCH,0start of the first resource of each tuple is indicated by the 'Lowest subChannel indices' field. The resource reservation period Prsvp,m  is encoded as in SCI format 1-A.

If the set is indicated by an SCI format 2-C, the number of tuples is M=2.

A UE forms the union of the subsets indicated by each tuple (TRIVm,FRIVm,Prsvp,m) to obtain the set r0,r1,r2,….

<omitted text>

### 8.2.1	CSI-RS transmission procedure

A UE transmits sidelink CSI-RS within a unicast PSSCH transmission if the following conditions hold:

-	CSI reporting is enabled by higher layer parameter sl-CSI-Acquisition; and

-	the 'CSI request' field in the corresponding SCI format 2-A or 2-C is set to 1.

The following parameters for CSI-RS transmission are configured for each CSI-RS configuration:

-	sl-CSI-RS-FirstSymbol indicates the first OFDM symbol in a PRB used for SL CSI-RS

-	sl-CSI-RS-FreqAllocation indicates the number of antenna ports and the frequency domain allocation for SL CSI-RS.

When the UE is configured with Qp={1,2} CSI-RS port(s) in sidelink and the number of scheduled layers is nlayerPSSCH,

-	The CSI-RS scaling factor βCSIRS specified in clause 8.4.1.5.3 of [4, TS 38.211] is given by βCSIRS=βDMRSPSSCH∙nlayerPSSCHQp  where βDMRSPSSCH is the scaling factor for the corresponding PSSCH specified in clause 8.3.1.5 of [4, TS 38.211].

<omitted text>

## 8.3	UE procedure for receiving the physical sidelink shared channel

For sidelink resource allocation mode 1, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, 2-B and 2-C, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate.

For sidelink resource allocation mode 2, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, 2-B and 2-C, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate.

A UE is required to decode neither the corresponding SCI formats 2-A, 2-B and 2-C nor the PSSCH associated with an SCI format 1-A if the SCI format 1-A indicates an MCS table that the UE does not support.

<omitted text>

#### 8.5.1.2	Triggering of sidelink CSI reports

The CSI-triggering UE is not allowed to trigger another aperiodic CSI report for the same UE before the last slot of the expected reception or completion of the ongoing aperiodic CSI report associated with the SCI format 2-A or 2-C with the 'CSI request' field set to 1, where the last slot of the expected reception of the ongoing aperiodic CSI report is given by [10, TS38.321].

An aperiodic CSI report is triggered by an SCI format 2-A or 2-C with the 'CSI request' field set to 1.

A UE is not expected to transmit a sidelink CSI-RS and a sidelink PT-RS which overlap.

<omitted text>

#### 8.5.2.2	Reference signal (CSI-RS)

The UE can be configured with one CSI-RS pattern as indicated by the higher layer parameters sl-CSI-RS-FreqAllocation, sl-CSI-RS-FirstSymbol in SL-CSI-RS-Config.

Parameters for which the UE shall assume non-zero transmission power for CSI-RS are configured according to clause 8.2.1.

A UE is not expected to be configured such that a CSI-RS and the corresponding PSCCH can be mapped to the same resource element. A UE is not expected to receive sidelink CSI-RS and PSSCH DM-RS, nor CSI-RS and 2nd-stage SCI, on the same symbol.

Sidelink CSI-RS shall be transmitted according to [4, TS 38.211] in the resource blocks used for the PSSCH associated with the SCI format 2-A or 2-C triggering a report.

#### 8.5.2.3	CSI reference resource definition

The CSI reference resource in sidelink is defined as follows:

-	In the frequency domain, the CSI reference resource is defined by the group of sidelink physical resource blocks containing the sidelink CSI-RS to which the derived CSI relates.

-	In the time domain, the CSI reference resource for a CSI reporting in sidelink slot n is defined by a single sidelink slot nCSI_ref where nCSI_ref is the same sidelink slot as the corresponding CSI request.

If configured to report CQI index and RI index, in the CSI reference resource, the UE shall assume the following for the purpose of deriving the CQI index and RI index:

-	The reference resource uses the CP length and subcarrier spacing configured for the SL BWP.

-	Redundancy Version 0.

-	PSCCH occupies 2 OFDM symbols.

-	The number of PSSCH and DM-RS symbols is equal to sl-LengthSymbols‒2.

-	Assume no REs allocated for sidelink CSI-RS.

-	Assume no REs allocated SCI format 2-A, SCI format 2-B or SCI format 2-C.

-	Assume the same number of DM-RS symbols as the smallest one configured by the higher layer parameter sl-PSSCH-DMRS-TimePatternList.

-	Assume no REs allocated for sidelink PT-RS.

-	Assume sidelink CSI-RS RE power is the same as PSSCH RE power.

-	The PSSCH transmission scheme where the UE may assume that PSSCH transmission would be performed with up to 2 transmission layers as defined in Clause 8.3.1.4 of [4, TS 38.211]. For CQI calculation, the UE should assume that PSSCH signals on antenna ports in the set [1000,…, 1000+ν-1] for ν layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…, 3000+P-1], as given by

y3000(i)⋯y3000+P-1(i)=W(i)x0(i)⋯xν-1(i)

where  is a vector of PSSCH symbols from the layer mapping defined in Clause 8.3.1.4 of [4, TS 38.211], P∈1,2 is the number of CSI-RS ports. If only one CSI-RS port is configured, W(i) is 1. Otherwise, W(i) is the identity matrix.

<omitted text>