3GPP TSG-RAN WG1 Meeting #111	R1-2210896

Toulouse, France, November 14 – 18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | xxxx | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Q formula in step 6 of sensing and resource exclusion procedure in TS 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For periodic-based partial sensing (PBPS), a UE, based on (pre-)configuration, needs to additionally monitor the last periodic sensing occasion prior to the most recent one for a given reservation periodicity. However, this behavior is not captured correctly in step 6 of sensing and resource exclusion procedure in section 8.1.4 of TS 38.214. The determination on Q value is specified in step 6, where there are different conditions – on whether satisfying Prsvp_RX< Tscal (condition 1) and n'-m≤Prsvp_RX' (condition 2), i.e. whether a received SCI can indicate multiple periodic reservations within a UE’s selection window. Under either condition, current spec only requires a UE to check most recent sensing occasion before the reference slot, and if no SCI is successfully decoded at most recent sensing occasion, the UE will assume no periodic transmission (based on Prsvp_RX) will be collided in its selection window. Thus specification of Q should be updated to take into account monitored slot m, which is the most second recent monitoring slot for PrsvpRX before reference slot n', if PrsvpRX belongs to (pre-)configured Preserve, for both Prsvp_RX< Tscal (case 1) and Prsvp_RX≥ Tscal (case 2), as shown in the figure below. Figure 1: Q takes the monitored result on the most 2nd recent slot into account |  |  |  |  |  |  |  |  |
| Summary of change: |  | In step 6 of sensing and resource exclusion procedure as specified in section 8.1.4 of TS 38.214, the determination of Q is updated to allow a UE to exclude resources based on not only the most recent sensing occasion, but also the last sensing occasion prior to the most recent one, when high layer parameter is (pre-)configured to monitor additionally sensing occasion for PBPS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification does not fully capture existing RAN1 agreement, and Rel-17 partial sensing feature is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged parts are omitted >

### 8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

< Unchanged parts are omitted >

The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

a)	the UE receives an SCI format 1-A in slot t'mSL, and 'Resource reservation period' field, if present, and 'Priority' field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213];

b)	the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX,prioTX;

c)	the SCI format received in slot t'mSL or the same SCI format which, if and only if the 'Resource reservation period' field is present in the received SCI format 1-A, is assumed to be received in slot(s) t'm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7,

- if the UE is configured with partial sensing by its higher layer and higher layer parameter additionalPeriodicSensingOccasion is configured, if  Prsvp_RX'<n'-m≤2·Prsvp_RX', Prsvp_RX=Preserve and the sensing occasion tm'SL belongs to ty-k×Preserve''SL for a given periodicity Preserve, Q=TscalPrsvp_RX+1

- otherwise,  Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX'; Q=1 otherwise;

where if the UE is configured with full sensing by its higher layer, t'n'SL = n if slot n belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise slot t'n'SL is the first slot after slot n belonging to the set t'0SL,t'1SL,...,t'T'max-1SL; If UE is configured with partial sensing by its higher layer, t'n'SL=t'yiSL-Tproc,1SL if slot t'yiSL-Tproc,1SL belongs to the set (t0'SL, t1'SL,⋯, tTmax'-1'SL), otherwise, slot t'n'SL is the first slot after slot t'yiSL-Tproc,1SL belonging to the set (t0'SL, t1'SL,⋯, tTmax'-1'SL).Q=1 If the UE is configured with full sensing by its higher layer, Tscal is set to selection window size T2 converted to units of msec. If UE is configured with partial sensing by its higher layer, Tscal=t'yLSL-(t'yiSL-Tproc,1SL) shall be converted to milliseconds, where slot t'yLSL is the last slot of the Y or Y' candidate slots. The slot t'yiSL is the first slot of the selected/remaining set of Y or Y' candidate slots.

< Unchanged parts are omitted >