3GPP TSG RAN WG1 #111		R1-2211017

Toulouse, France, November 14th – 18th, 2022

Source:	vivo, Guangdong Genius

Title:                     Discussion on UE further complexity reduction

Agenda Item:	9.6.1

Document for:	Discussion and Decision

- Introduction

In RAN1 #110bis-e meeting, following agreements were made for UE complexity reduction [1]:

Initial BWP Agreement: For a cell supporting both Rel-17 and Rel-18 RedCap UEs, - The Rel-18 RedCap UEs can share the same separate initial DL/UL BWP as the Rel-17 RedCap UEs. - FFS: whether to support an additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs Number of PRBs Agreement: Replace the agreement on the maximum number of PRBs supported by UE with the following: For UE BB bandwidth reduction, for PUSCH, down-select between the following options for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable: - Option 1: 28 PRBs for 15 kHz SCS and 14 PRBs for 30 kHz SCS - Option 2: 27 PRBs for 15 kHz SCS and 13 PRBs for 30 kHz SCS - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - Option 4: 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS For UE BB bandwidth reduction, for PDSCH (at least for unicast), down-select between the following options for the maximum number of PRBs that the UE can process per slot: - Option 1: 28 PRBs for 15 kHz SCS and 14 PRBs for 30 kHz SCS - Option 2: 27 PRBs for 15 kHz SCS and 13 PRBs for 30 kHz SCS - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - Option 4: 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS Same option will be selected for both PDSCH (at least for unicast) and PUSCH. SIB1 bandwidth Agreement: Replace the agreement on SIB1(PDSCH) for UE BB bandwidth reduction with the following: For UE BB bandwidth reduction, for SIB1 (PDSCH), - Allow the scheduling of SIB1 to be larger than 5 MHz (as in legacy operation) - FFS: UE post-FFT buffering “assumption” OSI bandwidth Agreement: Replace the agreement on broadcast OSI (PDSCH) for UE BB bandwidth reduction with the following: For UE BB bandwidth reduction, for broadcast OSI (PDSCH), - Allow the scheduling of broadcast OSI (PDSCH) to be larger than 5 MHz (as in legacy operation) Paging bandwidth Agreement: For UE BB bandwidth reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, down-select between the following options: - Option 1: Restrict the scheduling of paging channel to be within 5 MHz - Option 2: Allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation) - FFS: whether 5MHz is assumed to be physically contiguous RAR bandwidth Agreement: For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, down-select between the following options: - Option 1: Restrict the scheduling of RAR PDSCH to be within 5 MHz - Option 2: Allow the scheduling of RAR PDSCH to be larger than 5 MHz (as in legacy operation) - FFS: whether 5MHz is assumed to be physically contiguous PUSCH bandwidth Agreement: For UE BB bandwidth reduction, a UE is not expected to receive an UL grant in a DCI with a PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Agreement: - For UE BB bandwidth reduction, a UE is not expected to be configured with a CG grant with a PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. - For UE BB bandwidth reduction, it is FFS whether a UE can be expected to receive an UL grant in a RAR with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Agreement: - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X - If UE peak data rate reduction is supported as a standalone feature, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ Y. - FFS: the value of Y - Note: Whether this option is supported will be decided in RAN plenary.

In this contribution, we provide our views on UE BB bandwidth reduction, early identification for Rel-18 RedCap UE.

- UE BB bandwidth reduction

About UE bandwidth reduction to 5MHz, there were extensive discussions in RAN1#110bis-e meeting including selection between BW3 and PR3 and the assumptions of UE’s post FFT buffering.

Firstly, we would like to emphasize that for the following objective of UE BB bandwidth reduction [2], the WID mentioned 5MHz UE BB bandwidth, it does not clarify the resource allocation can only span a bandwidth of maximum 5 MHz or it can span a bandwidth of maximum 20 MHz, it is also not clear how to handle the broadcast PDSCH transmissions/receptions. It is more constructive to first discuss and decide these details in RAN1 and put aside the objective itself intends for BW3 or PR3.

- UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth.

  - Bandwidth for UE’s Post-FFT data buffering

During the discussion in the last meeting, companies have different assumptions for UE’s post-FFT data buffering. It is true that the specification will and should not enforce any specific UE implementation on post-FFT buffering. However, the assumptions for post-FFT buffering will impact on the UE behavior, network scheduling and performance, which may have specification impacts. In the following, we provide our views.

- Assumption 1: The bandwidth for Post-FFT data buffering is 5MHz.

For assumption 1, UE should know where the 5MHz BB bandwidth is before PDCCH decoding, i.e, the starting/ending PRB. In order to achieve this, the location for the 5MHz BB bandwidth can be always fixed within a carrier, or can be hopped within the carrier based on some predefined hopping patterns. This assumption achieves the most cost reductions for post-FFT buffering. The additional cost reduction for 5MHz post-FFT buffering is up to 1% for most configurations compared to 20MHz post-FFT buffering according to [3]. From NW perspective, assumption 1 has significant scheduling/configuration restrictions especially for the shared broadcast PDSCHs like SIB1/OSI and paging PDSCH. From UE side, when SIB1/OSI is scheduled to be larger than 5 MHz based on the agreements, if UE needs to soft combine multiple SIB/OSI transmissions, it increases UE implementation complexity and power consumption; Otherwise, large performance degradation can be expected if puncturing is performed. Furthermore, for paging PDSCH that is not periodic message, if 5 MHz post-FFT buffer is assumed, the UE may miss paging in case the NW multiplexes large number of paging records in one paging message and the resource scheduled for paging is larger than 5MHz.  From specification perspective, defining the fixed locations or hopping patterns for 5MHz UE BB bandwidth for UE in RRC-idle/connected mode will involve large specification efforts.

Observation 1: If the assumption for UE’s post-FFT data buffering is 5MHz,

- It can achieve up to 1% additional cost reduction compared to 20 MHz Post-FFT data buffering.

- Specification impacts include at least defining the location(s) for 5MHz within the carrier.

- Significant scheduling/configuration restrictions at NW side.

- For SIB1/OSI scheduled to be larger than 5MHz, if soft combining is required, it increases UE implementation complexity and power consumption; otherwise, significant coverage loss is expected by puncturing.

- For paging PDSCH scheduled to be larger than 5MHz, significant coverage loss is expected by puncturing.

- Assumption 2: The bandwidth for Post-FFT data buffering is between 5MHz and 20MHz.

For assumption 2, the bandwidth for the post-FFT data buffering would be variant but can be smaller than 20MHz, which depends on the time needed for PDCCH decoding. For example, during the SI, some companies use cross-slot scheduling to reduce the bandwidth for Post-FFT data buffering down to 5MHz; while some companies assumed UE needs to buffer 20MHz bandwidth for X+Y=5 symbols and 5MHz for (14-X-Y) =9 symbols per slot, where X=2 symbols for PDCCH transmission and Y=3 symbols for PDCCH decoding time [4].  For cross-slot scheduling, it is supported in Rel-16 for unicast data scheduling, it is not supported for broadcast PDSCH in current specification, the K0 in the default PDSCH time domain allocation A always equals to 0. The potential cost reduction benefit is dependent on the UE PDCCH processing time, which is not specified currently. For the cross-slot scheduling case, the PDCCH processing time may not need to be specified as it is expected that one slot could be sufficient for PDCH processing at least for low SCS cases (e.g. 15KHz, 30KHz), however, the PDCCH processing time for same slot scheduling case would have to be specified otherwise the achievable post-FFT buffer reduction is not guaranteed. For shared SIB1/OSI with resources larger than 5MHz, same as assumption 1, UE can soft combine multiple SIB/OSI transmissions to ensure the coverage. However, for paging PDSCH with resources that can be scheduled up to 20MHz, large performance degradation can be expected if the UE only processes 5MHz even if the post-FFT buffering can be larger than 5MHz, but smaller than 20MHz.

Observation 2: If the assumption for UE’s post-FFT data buffering is between 5MHz and 20MHz,

- It may achieve smaller than 1% additional cost reduction compared to 20 MHz Post-FFT data buffering.

- It may have specification impacts to reduce UE’s post-FFT data buffering below 20MHz, e.g. PDCCH processing time.

- For SIB1/OSI scheduled to be larger than 5MHz, if soft combining is required, it increases UE implementation complexity and power consumption; otherwise, significant coverage loss is expected by only processing 5MHz.

- For paging PDSCH scheduled to be larger than 5MHz, significant coverage loss is expected by only processing 5MHz.

- Assumption 3: The bandwidth for Post-FFT data buffering is 20MHz.

For assumption 3, the UE’s post-FFT data buffering is 20MHz in all OFDM symbols within one slot. For broadcast PDSCH especially SIB1/OSI/paging, the UE can buffer the resources of maximum 20MHz. Since there is no timeline requirement for processing SIB1/OSI/paging, it is natural for UE with 5MHz BB bandwidth to process the SIB1/OSI/paging in later slots to ensure good coverage performance

Observation 3: If the assumption for UE’s post-FFT data buffering is 20MHz,

- No specification impacts.

- For SIB1/OSI/paging scheduled to be larger than 5MHz, to ensure good coverage performance, UE can receive and buffer up to 20 MHz and process for a bit slower in the following slots without increasing UE implementation complexity.

Based on above analysis, 20MHz post-FFT buffering is the best solution. Therefore, we support following conclusion that was made in the last meeting but not yet agreed.

Conclusion: For UE BB bandwidth reduction, UE cannot expect that network will assume that the UE post-FFT buffer size can be smaller than 20MHz in one slot/symbol.

Alternatively, following conclusion can be considered if companies prefer to leave to UE implementation on potential post-FFT buffering reduction.

Conclusion:

- For UE BB bandwidth reduction, RAN1 assumes that the UE post-FFT buffer size is 20MHz in the future work.

- RAN1 will not specify any scheduling restrictions/optimizations to support reducing the UE's post-FFT buffer size below 20MHz.

- Whether UE's post-FFT buffer size can be reduced below 20MHz is left to UE implementation.

In the following, we discuss UE behavior for receiving/processing SIB1/OSI, Paging, MSG2/MSG4, MSG3 and unicast PDSCH/PUSCH assuming that there is no specification impacts to support further reduction of UE’s post-FFT buffering.

  - 5MHz BB bandwidth for broadcast PDSCH

- SIB1/OSI

In case the resources scheduled for SIB1 and OSI is larger than 5MHz, during the discussions in the last meeting, to achieve the good coverage, there are two popular options for receiving/processing SIB1/OSI.

Option 1: UE buffers 20MHz until the PDCCH is decoded, then the UE processes multiple same/different 5MHz (across different SIB1/OSI repetition versions) and does soft combining based on implementation.

Option 2: UE buffers 20MHz for all 14 symbols in a slot and UE processes in the following slots.

For option 1, further discussion is needed on whether to define the location of 5MHz that UE should receive or left to UE implementation to decide which 5MHz to receive. As already analyzed in section 2.1, soft combining increases UE implementation complexity, power consumption and access delay. In addition, NW needs to ensure the same TBS for multiple SIB1/OSI transmissions to support such soft combining at the UE side.

For option 2, since no HARQ-ACK feedback is required for SIB1/OSI, no specification impact is expected to support processing the SIB1/OSI in the later slots. Furthermore, it relaxes UE implementation complexity compared to the soft combining. Therefore, following proposal is made.

Proposal 1: When the resource for SIB1/OSI is scheduled larger than 5MHz, it is preferred that the eRedCap UE is capable of buffering 20MHz so that all the PRBs scheduled for SIB1/OSI can be received in a slot and UE processing can take longer time in the following slots.

- Paging PDSCH

For paging PDSCH, it is possible to have dedicated paging message for Rel-18 RedCap UE so that the scheduled resources can be within 5MHz. However, such restriction increases the paging overhead and is not necessary since no timeline requirement is defined for the event triggered by paging PDSCH. Therefore, Option 2 that allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation) is more preferred. Given paging PDSCH is not periodic signaling, option 2 that UE buffers 20MHz for all 14 symbols in a slot and UE processes 5MHz per slot for the following slots is preferred for handling the paging PDSCH.

Proposal 2: For UE BB bandwidth reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (shared with legacy UEs).

Proposal 3: When the resource for paging channel is scheduled larger than 5MHz, it is preferred that the eRedCap UE is capable of buffering 20MHz so that all the PRBs scheduled for paging can be received in a slot and UE processing 5MHz can take longer time in the following slots.

- MSG2 (RAR) and MSG4

For MSG2, similar as paging that it is not periodic signaling. But different from SIB1/OSI/paging, its reception/processing has impact on the MSG3 transmission timing, the related timeline requirement specified in TS 38.213 [5] as following:

With reference to slots for a PUSCH transmission scheduled by a RAR UL grant, if a UE receives a PDSCH with a RAR message ending in slot 𝑛 for a corresponding PRACH transmission from the UE, the UE transmits the PUSCH in slot 𝑛+𝑘2+Δ+2𝜇∙𝐾cell,offset, where 𝑘2 and Δ are provided in [6, TS 38.214] and 𝐾cell,offset is provided by CellSpecific_Koffset; otherwise, if not provided, 𝐾cell,offset=0.

On one hand, if the resource allocated for RAR PDSCH is allowed to be larger than 5 MHz and UE processes 5MHz per slot for several slots, the timeline requirement for transmitting MSG3 may not be met. On the other hand, if the resource allocated for RAR PDSCH is restricted to be within 5 MHz, then the early indication by MSG1 to identify RedCap UE is needed which requiring the NW to always configure/enable the early indication in MSG1. During the last meeting discussion, it seems companies assume the typical TBS for MSG2 is small hence it will not require the resource spanning more than 5MHz, or the RAR performance can be guaranteed by gNB implementation via the low code rate and power boosting etc. To relax NW scheduling/configuration restrictions, avoid further splitting the PRACH resources and also relax UE processing complexity, following compromised solution can be considered.

Proposal 4: For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs,

- If early indication by MSG1 is available, restrict the scheduling of RAR PDSCH to be received by eRedCap UEs to be within 5 MHz.

- NW should ensure that the RAR intended for  non-RedCap/Rel-17 RedCap/Rel-18 eRedCap UEs are confined within 5MHz to avoid impacts for non-RedCap UEs.

- If early indication by MSG1 is not available, allow the scheduling of RAR PDSCH to be larger than 5 MHz (as in legacy operation).

- In case Rel-18 eRedCap UE cannot meet the timeline requirement for MSG3 transmission, UE drops the MSG3 transmission.

For MSG4, there is also a minimum timeline requirement defined for HARQ-ACK feedback, as shown in following [5].

A minimum time between the last symbol of the PDSCH reception and the first symbol of the corresponding PUCCH transmission with the HARQ-ACK information is equal to 𝑁𝑇,1+0.5 msec. 𝑁𝑇,1 is a time duration of 𝑁1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured. For 𝜇=0, the UE assumes 𝑁1,0=14 [6, TS 38.214].

Therefore, similar solution as for MSG2 can be considered, except that the early identification can be based on MSG3 which is mandatorily supported by Rel-17 RedCap UEs. However, different from MSG2, MSG4 supports re-transmissions, no specific handling can also be one option in case the resource scheduled for MSG4 is larger than 5 MHz.

Proposal 5: For UE BB bandwidth reduction, for MSG4 to Rel-18 RedCap UEs, down-select from following options:

- Option 1: Restrict the scheduling of MSG4 to be within 5 MHz.

- Note that for such restriction only apply to all RedCap UEs (in case the cell supporting both Rel-17 and Rel-18 eRedCap UEs) , if early identification is available in MSG3.

- Option 2: Allow the scheduling of MSG4 to be larger than 5 MHz (as in legacy operation). In case Rel-18 RedCap UE cannot meet the timeline requirement for corresponding PUCCH transmission with the HARQ-ACK information, UE drops the PUCCH.

  - 5MHz BB bandwidth for MSG3

In the last meeting, it was agreed that for UE BB bandwidth reduction, a UE is not expected to receive an UL grant in a DCI or configured with a CG grant with a PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Firstly, we woud like to clarify whether the agreement is for unicast PUSCH transmission or it also inlcudes MSG3 Retransmission that is scheduled by UL grant in a DCI scrambled by TC-RNTI.

Proposal 6: It should be clarified whether the PUSCH in the following agreements include MSG3 retransmission scheduled by UL grant in a DCI scrambled by TC-RNTI

- For UE BB bandwidth reduction, a UE is not expected to receive an UL grant in a DCI with a PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable.

For MSG3, the TBS in most cases are either 56 bits or 72 bits and the required resource would not span more than 5 MHz. Therefore, it is not big restriction at the NW side to ensure the Msg3 PUSCH resource allocation within 5MHz. It should be noted that even with this restriction, it does not mean the separate early identification by MSG1 is required. R18 RedCap UE can always share the early identification with Rel-17 RedCap UEs.

Proposal 7: For UE BB bandwidth reduction, for MSG3 to Rel-18 RedCap UEs:

- If early indication by MSG1 is available, the eRedCap UE is not expected to receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable.

- For a cell supporting both Rel-17 and Rel-18 eRedCap UEs, NW should ensure that the MSG3   to be received by Rel-18 eRedCap UEs are confined within 5MHz.

- If early indication by MSG1 is not available, the UE can receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable, and UE drops the MSG3 transmission for such case.

  - 5MHz BB bandwidth for unicast PDSCH

For R18 RedCap UEs in RRC_connected mode, NW already knows UE BB bandwidth capability that UE can only process 5MHz per slot. In case the resource allocation for unicast PDSCH is more than 5MHz, similar processing as for SIB1/OSI can be used. However, it will impact on the HARQ-ACK feedback timeline, and/or some scheduling restrictions like some scheduling gap between two DCIs scheduling the unicast PDSCH is required. Therefore, for unicast PDSCH, NW should ensure that the scheduled number of PRBs do not exceed 5MHz e.g. 25PRBs@15KHz SCS and 11 PRBs@30KHZ SCS. In addition, given UE’s post FFT-buffering assumed by RAN1 is 20MHz, it is beneficial and more flexible to support the non-contiguous PRB allocation for unicast PDSCH cross 20MHz.

Proposal 8: For UE BB bandwidth reduction, for unicast PDSCH, support non-contiguous PRB allocation for unicast PDSCH cross 20MHz BWP with the restriction that the maximum number of PRBs in the resource allocation corresponds to 5 MHz.

  - Separate initial DL/UL BWP

For a cell supporting both Rel-17 and Rel-18 RedCap UEs, in case the separate initial DL/UL BWP is configured for Rel-17 RedCap, Rel-18 RedCap UE should also use the same separate initial DL/UL BWP even if the bandwidth of the initial DL/UL BWP configured for legacy UEs is not larger than 20MHz. In case the separate initial DL/UL BWP is not configured for Rel-17 RedCap, it is also not necessary to support a separate initial DL/UL BWP specific to Rel-18 RedCap UEs.

For a cell supporting only Rel-18 RedCap UEs and legacy UEs, Rel-18 eRedCap can reuse the separate initial DL/UL BWP operation framework specified for Rel-17 RedCap with the exception that the bandwidth for the separate initial DL/UL BWP should not exceed 5MHz.

Proposal 9: For a cell supporting both Rel-17 and Rel-18 eRedCap UEs,

- The Rel-18 RedCap UEs can share the same separate initial DL/UL BWP as the Rel-17 RedCap UEs.

- An additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs is not supported.

Proposal 10: For a cell supporting only Rel-18 RedCap UEs and non-RedCap UEs, one separate initial DL/UL BWP with the maximum bandwidth of 5MHz for Rel-18 RedCap UEs may be configured by reusing the separate initial DL/UL BWP operation framework specified for Rel-17 RedCap.

- UE peak data rate reduction

Based on the agreement, RAN1 supports UE peak data rate reduction at least as an add-on to UE BB bandwidth reduction, the remaining issue is to determine the relaxed constraint value. Based on [2], the supported peak data rate for Rel-18 RedCap targets to 10Mbps. For 5MHz UE BB bandwidth, assuming the maximum number of PRBs is 25 for 15KHz SCS and 11 for 30KHz SCS with 64QAM and 1Rx/1layer,

- If the constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ 3.4, the DL/UL peak data rate can be achieved for 15KHz SCS is 11.4 Mbps/12.2Mbps, for 30KHz SCS is 10 Mbps/10.7Mbps.

- If the constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ 3.2, the DL/UL peak data rate can be achieved for 15KHz SCS is 10.7 Mbps/11.4Mbps, for 30KHz SCS is 9.4 Mbps/10 Mbps.

- If the constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ 3, the DL/UL peak data rate can be achieved for 15KHz SCS is 10Mbps/10.7Mbps, for 30KHz SCS is 8.8 Mbps/9.4Mbps.

Proposal 11: For UE peak data rate reduction added on to UE BB bandwidth reduction, down-select from the values of {3.4, 3.2, 3} to further relax the constraint vLayers·Qm·f ≥ 4.

If UE peak data rate reduction is supported as a standalone feature which should be decided in Dec. RAN plenary, the constraint vLayers·Qm·f ≥ 4 should be relaxed to vLayers·Qm·f ≥ 1 for 10Mbps target peak data rate.

Proposal 12: If UE peak data rate reduction is supported as a standalone feature, the constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ 1.

- Early identification for Rel-18 RedCap

For Rel-18 RedCap UE early identification, we think the same early identification as Rel-17 can be reused. About whether to support the separate early identification for Rel-18 eRedCap, further discussion is needed to have common understanding for the usage and benefits to distinguish Rel-17 RedCap UE and Rel-18 RedCap UE. From our understanding, allowing the distinguish between Rel-17 and Rel-18 RedCap is mainly motivated by transmission bandwidth restriction for MSG2/MSG4 and MSG3, e.g. NW can ensure that the scheduled number of PRBs for MSG2/MSG4 and MSG3 for Rel-18 eRedCap UE do not exceed 5MHz. Based on our discussions in section 2, for a cell support both Rel-17 and Rel-18 RedCap UEs, no strong justification is found to support the separate early indication for Rel-18 RedCap UEs. On the other hand, the introduced separate early identification especially by MSG1 makes the gNB scheduling more complex and PRACH resources be more fragmented.  Therefore, we propose following:

Proposal 13:

- For a cell supporting both Rel-17 and Rel-18 RedCap UEs, separate early identification specific for Rel-18 eRedCap is not supported, shared early identification is used for Rel-18 RedCap UEs.

- For a cell supporting only Rel-18 RedCap UEs and non-RedCap UEs, early identification can be configured for Rel-18 RedCap UEs by reusing the same early identification mechanism specified for Rel-17 RedCap.

- Conclusion

This contribution discusses complexity reduction features and early identification aspect for Rel-18 enhanced RedCap devices. The observations and proposals are summarized as following:

Observations:

Observation 1: If the assumption for UE’s post-FFT data buffering is 5MHz,

- It can achieve up to 1% additional cost reduction compared to 20 MHz Post-FFT data buffering.

- Specification impacts include at least defining the location(s) for 5MHz within the carrier.

- Significant scheduling/configuration restrictions at NW side.

- For SIB1/OSI scheduled to be larger than 5MHz, if soft combining is required, it increases UE implementation complexity and power consumption; otherwise, significant coverage loss is expected by puncturing.

- For paging PDSCH scheduled to be larger than 5MHz, significant coverage loss is expected by puncturing.

Observation 2: If the assumption for UE’s post-FFT data buffering is between 5MHz and 20MHz,

- It may achieve smaller than 1% additional cost reduction compared to 20 MHz Post-FFT data buffering.

- It may have specification impacts to reduce UE’s post-FFT data buffering below 20MHz, e.g. PDCCH processing time.

- For SIB1/OSI scheduled to be larger than 5MHz, if soft combining is required, it increases UE implementation complexity and power consumption; otherwise, significant coverage loss is expected by only processing 5MHz.

- For paging PDSCH scheduled to be larger than 5MHz, significant coverage loss is expected by only processing 5MHz.

Observation 3: If the assumption for UE’s post-FFT data buffering is 20MHz,

- No specification impacts.

- For SIB1/OSI/paging scheduled to be larger than 5MHz, to ensure good coverage performance, UE can receive and buffer up to 20 MHz and process for a bit slower in the following slots without increasing UE implementation complexity.

Conclusion and Proposals:

Conclusion: For UE BB bandwidth reduction, UE cannot expect that network will assume that the UE post-FFT buffer size can be smaller than 20MHz in one slot/symbol.

Conclusion:

- For UE BB bandwidth reduction, RAN1 assumes that the UE post-FFT buffer size is 20MHz in the future work.

- RAN1 will not specify any scheduling restrictions/optimizations to support reducing the UE's post-FFT buffer size below 20MHz.

- Whether UE's post-FFT buffer size can be reduced below 20MHz is left to UE implementation.

Proposal 1: When the resource for SIB1/OSI is scheduled larger than 5MHz, it is preferred that the eRedCap UE is capable of buffering 20MHz so that all the PRBs scheduled for SIB1/OSI can be received in a slot and UE processing can take longer time in the following slots.

Proposal 2: For UE BB bandwidth reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (shared with legacy UEs).

Proposal 3: When the resource for paging channel is scheduled larger than 5MHz, it is preferred that the eRedCap UE is capable of buffering 20MHz so that all the PRBs scheduled for paging can be received in a slot and UE processing 5MHz can take longer time in the following slots.

Proposal 4: For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs,

- If early indication by MSG1 is available, restrict the scheduling of RAR PDSCH to be received by eRedCap UEs to be within 5 MHz.

- NW should ensure that the RAR intended for  non-RedCap/Rel-17 RedCap/Rel-18 eRedCap UEs are confined within 5MHz to avoid impacts for non-RedCap UEs.

- If early indication by MSG1 is not available, allow the scheduling of RAR PDSCH to be larger than 5 MHz (as in legacy operation).

- In case Rel-18 eRedCap UE cannot meet the timeline requirement for MSG3 transmission, UE drops the MSG3 transmission.

Proposal 5: For UE BB bandwidth reduction, for MSG4 to Rel-18 RedCap UEs, down-select from following options:

- Option 1: Restrict the scheduling of MSG4 to be within 5 MHz.

- Note that for such restriction only apply to all RedCap UEs (in case the cell supporting both Rel-17 and Rel-18 eRedCap UEs) , if early identification is available in MSG3.

- Option 2: Allow the scheduling of MSG4 to be larger than 5 MHz (as in legacy operation). In case Rel-18 RedCap UE cannot meet the timeline requirement for corresponding PUCCH transmission with the HARQ-ACK information, UE drops the PUCCH.

Proposal 6: It should be clarified whether the PUSCH in the following agreements include MSG3 retransmission scheduled by UL grant in a DCI scrambled by TC-RNTI

- For UE BB bandwidth reduction, a UE is not expected to receive an UL grant in a DCI with a PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable.

Proposal 7: For UE BB bandwidth reduction, for MSG3 to Rel-18 RedCap UEs:

- If early indication by MSG1 is available, the eRedCap UE is not expected to receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable.

- For a cell supporting both Rel-17 and Rel-18 eRedCap UEs, NW should ensure that the MSG3   to be received by Rel-18 eRedCap UEs are confined within 5MHz.

- If early indication by MSG1 is not available, the UE can receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable, and UE drops the MSG3 transmission for such case.

Proposal 8: For UE BB bandwidth reduction, for unicast PDSCH, support non-contiguous PRB allocation for unicast PDSCH cross 20MHz BWP with the restriction that the maximum number of PRBs in the resource allocation corresponds to 5 MHz.

Proposal 9: For a cell supporting both Rel-17 and Rel-18 eRedCap UEs,

- The Rel-18 RedCap UEs can share the same separate initial DL/UL BWP as the Rel-17 RedCap UEs.

- An additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs is not supported.

Proposal 10: For a cell supporting only Rel-18 RedCap UEs and non-RedCap UEs, one separate initial DL/UL BWP with the maximum bandwidth of 5MHz for Rel-18 RedCap UEs may be configured by reusing the separate initial DL/UL BWP operation framework specified for Rel-17 RedCap.

Proposal 11: For UE peak data rate reduction added on to UE BB bandwidth reduction, down-select from the values of {3.4, 3.2, 3} to further relax the constraint vLayers·Qm·f ≥ 4.

Proposal 12: If UE peak data rate reduction is supported as a standalone feature, the constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ 1.

Proposal 13:

- For a cell supporting both Rel-17 and Rel-18 RedCap UEs, separate early identification specific for Rel-18 eRedCap is not supported, shared early identification is used for Rel-18 RedCap UEs.

- For a cell supporting only Rel-18 RedCap UEs and non-RedCap UEs, early identification can be configured for Rel-18 RedCap UEs by reusing the same early identification mechanism specified for Rel-17 RedCap.

6 	References

- RAN1#110bis-e meeting, chairman’s notes

- 3GPP RP-222675, New WID on enhanced support of reduced capability NR devices, Ericsson

- 3GPP TR 38.865 V1.0.0, Study on further NR RedCap UE complexity reduction, Release 18, 2022-09

- R1-2205739, Potential solutions for further RedCap UE complexity reduction, Ericsson

- 3GPP TS 38.213, Physical layer procedures for control, (Release 17), V17.3.0 (2022-09)