3GPP TSG RAN WG1 #111	R1-2211057

# Toulouse, France, November 14th – 18th, 2022

Source:	ZTE

Title:	Evaluation on AI for CSI feedback enhancement

Agenda Item:	9.2.2.1

Document for:	Discussion and Decision

- Introduction

According to discussion in 3GPP RAN1#110b-e meeting [1], some progress has been made on evaluation methodology for CSI feedback enhancement. In this contribution, we provide our views on further details for evaluation methodology and share some initial evaluation results. In our companion contribution [2], representative sub-use case selection and potential specification impacts are discussed accordingly.

- Generic issues on evaluation methodology

  - Baseline for performance

- Questions in RAN1#110bis-e Question: For the evaluation of CSI enhancements, do you think there is need to introduce an additional throughput baseline based on ideal CSI (e.g., ideal eigenvector), which is taken as an upper bound? E.g., the baseline of ideal CSI is used for - Option 1: For calibration purpose - Option 2: To be taken as the eventual KPI for AI/ML-based performance comparison

In RAN1#110bis-e, the above issue was raised for introducing an upper bound for throughput baseline [3]. It is necessary to evaluate the system performance, e.g. throughput, between AI approaches and the ideal eigenvector feedback (i.e., the upper bound of performance). Since the legacy eType II approach shows average performance and differs from companies which is difficult to calibrate among companies, and the upper bound may not have much difference, it could make cross checking on eventual KPIs (e.g. UPT, throughput) much easier. Furthermore, it is helpful to compare the AI performance with the ideal CSI feedback to see how much room is left. Therefore, we propose ideal CSI feedback should be adopted as an additional throughput baseline at least for calibration purpose.

Proposal 1: Ideal CSI feedback should be adopted as an additional throughput baseline at least for calibration purpose.

As we know, in MU-MIMO, precoding can be refined considering MU-interference of scheduled UEs using approaches such as SLNR and zero-forcing. In such case, MCS estimation for refined precoding is quite challenging at gNB side. As discussed in MIMO agenda (section 3.7) [4], providing sufficient channel information (e.g. wideband Rxx including receiver side information, full rank report including eigenvectors and eigenvalues, etc) can significantly increase system performance. Due to sufficient channel information of UEs at gNB side, accurate precoding adjustments for MU scheduling could be possible to avoid interference across UEs. To avoid possible overlapping enhancements, we propose to consider adopting additional baseline for comparison.

Proposal 2: Enhancements on CSI feedback using non-AI/ML approaches (e.g. Wideband (WB) covariance matrix, full rank information) can be considered as an additional baseline for performance comparison.

- Evaluation methodology on CSI compression in spatial and frequency domain

  - Model input and model output

- Agreements in RAN1#109-e Agreement: For the evaluation of the AI/ML based CSI compression sub-use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information. - At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side. - Agreements in RAN1#110bis-e Agreement: For the evaluation of the AI/ML based CSI compression sub use cases, at least the following types of AI/ML model input (for CSI generation part)/output (for CSI reconstruction part) are considered for evaluations - Raw channel matrix, e.g., channel matrix with the dimensions of Tx, Rx, and frequency unit - Companies to report the raw channel is in frequency domain or delay domain - Precoding matrix - Companies to report the precoding matrix is a group of eigenvector(s) or an eType II-like reporting (i.e., eigenvectors with angular-delay domain representation) - Other input/output types are not precluded - Companies to report the combination of input (for CSI generation part) and output (for CSI reconstruction part), - Note: the input and output may be of different types

In RAN1#109-e meeting [5], AI/ML based CSI compression sub-use case was agreed. For ease of comparison, the data type & data size of the training data input to AI models are encouraged to be discussed and aligned if possible. In our contribution proposed for RAN1#109-e [6], we provided the model input and the model output details for rank=1. In this contribution, more details of rank>1 will be elaborated.

For rank>1, in our initial evaluation, we propose two cases for AI/ML model to compress and recover the sub-band eigenvectors for CSI compression sub-use case.

- Case 1: Layer common-A single model is applied to all layers and all ranks

For Case 1, we mix the sub-band eigenvectors of multiple layers from multiple UEs as training samples so the total number of training samples is, where  denotes the layer number and  denotes the number of UEs. For model testing/inference, the input of AI model is a group of sub-band eigenvectors obtained from the same layer, which can be a three-dimensional tensor with a size of , where 2 denotes real and imaginary parts of each complex channel coefficient,  denotes the number of transmit antenna ports, and  denotes the number of sub-bands. Then, the output of AI model is recovered eigenvectors. Essentially, the AI input is still one layer of data and the model structure is the same as rank=1. The detailed AI/ML framework is shown in Figure 1(a).

Figure 1(a) The input and output of AI/ML models for Case 1

- Case 2: Rank specific- Multiple models are trained and each model is applied to a specific rank

For Case 2, we generate the sub-band eigenvectors of multiple layers from multiple UEs as training samples so the total number of training sample is , where  denotes the number of UE samples. For model testing/inference, the input of AI model is a group of sub-band eigenvectors obtained from  layers, which can be a three-dimensional tensor with a size of , where 2 denotes real and imaginary parts of each complex channel coefficient,  denotes the layer number,  denotes the number of transmit antenna ports, and  denotes the number of sub-bands. Then, the output of AI model is recovered eigenvectors. Different from Case 1, the AI input is multiple layers of data and the model structure needs to be changed from rank=1. The detailed AI/ML framework is shown in Figure 1(b).

Figure 1(b) The input and output of AI/ML models for Case 2

Proposal 3: For rank>1, two cases on model input/output can be considered for intermediate KPIs and eventual performance evaluation as a starting point.

- Case 1: Layer common-A single model is applied to all layers and all ranks

- Case 2: Rank specific- Multiple models are trained and each model is applied to a specific rank

  - CSI training dataset and inference dataset separation

- Questions in RAN1#110 Question: For the evaluation of the AI/ML based CSI compression sub use cases, do you think there is a need to align the method to avoid correlation of samples between the training dataset and inference dataset?

In RAN1#110 meeting [7], the issue of CSI training dataset and inference dataset separation was raised for discussion, since inappropriate dataset separation for AI training/inference may cause an unconvincing conclusion. For example, if the testing data derives from the training dataset, the testing data and training data are so similar that the trained model may show better performance than the real model performance. For dataset construction for training and inference, there are at least two methods can be applied. With the first method, UE channel data may be generated for one TTI from multiple drops, which brings abundant channel diversity. With the second method, UE channel data may be generated for multiple TTIs from a few drops, which may introduce strong time correlation across different TTIs of the same drop. Two different dataset constructions may show different performance gains, where AI model will show real performance gains with the first method while inaccurate performance gains may be obtained using the second method. From our perspective, the first method can be adopted for dataset separation, i.e. dataset should be generated in diverse drops rather than in diverse TTIs.

Proposal 4: For CSI dataset construction, dataset should be generated in diverse drops rather than in divers TTIs at least for calibration purpose (e.g., only one TTI can be used for generating training dataset in a single drop).

  - CSI Payload size

- Proposal in RAN1#110bis-e Proposal 3.9.2: For the CSI payload size calculation for AI/ML-based CSI compression as well as the legacy TypeII codebook, the following two options are considered for further down-selection - Option 1: Payload size is calculated based on the maximum rank. - Option 2: Payload size is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE. FFS the following understandings - Option 2a: The above-mentioned “CSI payload” is calculated as each CSI reported payload with a given rank - Option 2b: The above-mentioned “CSI payload” is calculated as max allowed bits at the given rank

In RAN1#110bis-e meeting [3], the issue of CSI payload size still has no consensus after discussion. Option 1 worked well in legacy CSI evaluation for maximum payload size based on maximum rank with a given configured parameter combination, which is easy for companies to confirm the feedback overhead. For Option 2, it is preferable to study Option 2b, since Option 2a is too difficult to implement and max allowed bits at the given rank in Option 2b can be considered as a method to calculate realistic resource for CSI payload in SLS evaluation. According to our SLS results in sub-section 4.1-4.3, there is small difference for performance comparison between the payload size of Option 1 and Option 2b due to the similar scheduling distribution of Rank 1/2 and Rank 3/4. Therefore, we propose to adopt Option 1 as baseline evaluation metric and Option 2b is up to companies to report.

Proposal 5: For CSI payload size calculation, Option 1 is adopted as baseline evaluation metric and Option 2b is up to companies to report.

- Preliminary Evaluation Results for CSI compression

We perform a preliminary simulation on spatial-frequency domain CSI compression sub-use case to evaluate the performance of AI models and the Rel-16 eTypeII is also simulated for baseline comparison. In SLS, we further compare the throughput between AI-based feedback and ideal CSI feedback to see how much performance margin that AI-based approach is to an upper bound, where ideal eigenvector feedback is used as ideal CSI feedback. For Rel-16 eTypeII, 8 parameter combinations are used, and wherein ParComb 7 and ParComb 8 are directly extended for Rank=3 and Rank=4. In order to fairly compare the SLS performance, AI-based approach has the same overhead as that of Rel-16 eTypeII, i.e., corresponding to the 8 parameter combinations. For SLS evaluation metric, two types of overhead calculation are adopted. The first type of overhead calculation is used as the maximum overhead based on the maximum rank, and the second type of overhead is calculated as the weighted average of max allowed bits per rank based on the ratio of reported rank by UE. For simplicity, the first type of overhead is marked as Maximum overhead, and the second type is Weighted Avg. overhead. The detailed simulation assumptions are shown in Table 8-1. In addition, detailed AI model training parameters are listed in Table 8-2. The system-level channel data is generated from 3000 simulation drops and 210 UEs per simulation drop, which results in 630000 samples in total. Then, the datasets are randomly divided into three parts which are training, validation, and testing datasets with 600000, 10000, and 20000 samples respectively. In our evaluation, ideal sub-band eigenvectors are used as the input of AI models for training, validation and testing. The number of parameters ranges from about 9M-11M, where the CSI generation part is about 4M-5M and the CSI reconstruction part is 5M-6M. In addition, the FLOPs of CSI generation part and CSI reconstruction part are approximately 25M-26M and 27M-29M, respectively.

  - Rank up to 1

In this section, we initially evaluate the intermediate KPIs, e.g. SGCS, between our AI approach and the Rel-16 eType II. The results are shown in Figure 2(a). According to the simulation results, the AI based CSI recovery shows 7%-8% SGCS gains over the Rel-16 eType II. In order to explore the eventual UPT performance of our AI model, the SLS is performed for the case of up to 4 layers MU scheduling under FTP3 traffic model with 70% RU and the simulation results are shown in Figure 2(b) for average UPT gain and Figure 2(c) for 5% tail UPT gain. Only less than 1% average UPT performance gain can be obtained since Rel-16 eTypeII can already achieve good performance in single layer scheduling, while 2% ~ 20% performance gain for 5% tail UPT can be obtained.

Figure 2(a) Subband-level SGCS simulation results

Figure 2(b) Average UPT gain with RU=70%

Figure 2(c) 5% tail UPT gain with RU=70%

Observation 1:  For rank 1, AI-based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS. With the same feedback overhead, AI-based CSI recovery can obtain 7%-8% SGCS gains.

Observation 2: For rank 1, AI-based CSI reconstruction achieves very little average UPT gain, where less than 1% average UPT gain over Rel-16 eTypeII with the same feedback overhead is obtained, and the margin of AI-based CSI recovery to ideal CSI feedback is a little more than 1% as AI-based CSI reconstruction and Rel-16 eTypeII can already work well in single layer scheduling.

Observation 3: For rank 1, AI-based CSI reconstruction achieves 2% ~ 20% performance gain for 5% tail UPT, and the margin of AI-based CSI recovery to ideal CSI feedback is 7%.

  - Rank up to 2

In this section, the intermediate and eventual simulation results for Rel-16 eTypeII and AI based CSI recovery are provided. Figure 3 illustrates the SGCS comparison between the AI model and Rel-16 Type II codebook for each layer under the rank=2 case. As can be seen, the 1st layer outperforms the 2nd layer in terms of SGCS, where the reason may be that the 1st layer contains less features compared to the 2nd layer. Moreover, AI outperforms Rel-16 Type II codebook in terms of SGCS for each layer, indicating higher accuracy of CSI recovery by AI. In addition, Case 1(layer common) generally shows better SGCS performance than Case 2(rank specific).

Figure 3 SGCS simulation results for each layer

Observation 4: For rank =2, AI based CSI reconstruction with Case 1 method (single-layer model input and single-layer model output) shows performance gains in SGCS per layer over the Rel-16 eType II.

Observation 5: Case 1(layer common) can achieve better performance than Case 2 (rank specific).

To evaluate the eventual performance of AI/ML approaches, we further evaluate average UPT versus feedback overhead and 5% tail UPT versus feedback overhead under up to 8 layers MU scheduling and FTP 3 traffic model with 50% and 70% resource utilization (RU). Regarding the SGCS performance between the two cases of AI models, Case1 AI model is adopted for the subsequent eventual throughput simulation. The simulation results with RU=50% are shown in Figure 4(a) ~ Figure 4(d), wherein Figure 4(a) for average UPT gain versus Maximum overhead, Figure 4(b) for 5% tail UPT gain versus Maximum overhead, Figure 4(c) for average UPT gain versus Weighted Avg. overhead, and Figure 4(d) for 5% tail UPT gain versus Weighted Avg. overhead.

Figure 4(a) Average UPT gain                       Figure 4(b) 5% tail UPT gain

versusMaximum overhead with RU=50%              versus Maximum overhead with RU=50%

Figure 4(c) Average UPT gain                       Figure 4(d) 5% tail UPT gain

versusWeighted Avg. overhead with RU=50%              versus Weighted Avg. overhead with RU=50%

The simulation results with RU=70% are shown in Figure 4(e) ~ Figure 4(h), where Figure 4(e) is for average UPT gain versus Maximum overhead, Figure 4(f) is for 5% tail UPT gain versus Maximum overhead, Figure 4(g) is for average UPT gain versus Weighted Avg. overhead, and Figure 4(h) is for 5% tail UPT gain versus Weighted Avg. overhead.

Figure 4(e) Average UPT gain                       Figure 4(f) 5% tail UPT gain

versusMaximum overhead with RU=70%              versus Maximum overhead with RU=70%

Figure 4(g) Average UPT gain                       Figure 4(h) 5% tail UPT gain

versusWeighted Avg. overhead with RU=70%              versus Weighted Avg. overhead with RU=70%

Observation 6: For up to rank 2, with the same feedback overhead, AI based CSI reconstruction has about 4%-6% average UPT gain over the Rel-16 eType II under the case of 50% RU and 5%-8.5% average UPT gain can be obtained by AI based CSI recovery under the case of 70% RU.

Observation 7: For up to rank 2, with the same feedback overhead, AI based CSI reconstruction has about 10%-37% performance gain for 5% tail UPT over the Rel-16 eType II under the case of 50% RU and 10%-25% performance gain for 5% tail UPT can be obtained by AI based CSI recovery under the case of 70% RU.

Observation 8: With regard to up to rank 2, for average UPT, the margin of AI-based CSI recovery to ideal CSI feedback is 6% under the case of 50% RU, and 11% under the case of 70% RU.

Observation 9: With regard to up to rank 2, for 5% tail UPT, the margin of AI-based CSI recovery to ideal CSI feedback is 30% under the case of 50% RU, and 42% under the case of 70% RU.

Observation 10: The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for maximum rank up to 2.

Observation 11: AI/ML approaches show better average throughput performance with heavy traffic load for up to rank 2.

Observation 12: For up to rank 2, Maximum overhead shows similar performance comparison to Weighted Avg. Overhead.

  - Rank up to 4

In this section, the intermediate and eventual simulation results for Rel-16 eTypeII and AI based CSI recovery are provided. Figure 5 shows the comparison of SGCS between the AI model and Rel-16 Type II codebook for each layer under the case of rank=4. As can be seen, there are larger performance gains in the 3rd layer and the 4th layer than the 1st layer and the 2nd layer in terms of SGCS, where the reason may be that some feedback bits of layer 1/2 are sacrificed and distributed to layer 3/4 compared with eType II. However, AI still outperforms Rel-16 Type II codebook in terms of SGCS for each layer, indicating higher accuracy of CSI recovery by AI.

Figure 5 SGCS for each layer for rank=4

Observation 13: AI-based CSI reconstruction shows performance gain in SGCS per layer over the Rel-16 eType II for rank 3/4.

Observation 14: AI-based CSI reconstruction shows larger performance gains in layer 3/4 than layer 1/2 in terms of SGCS with the assumption of the same feedback overhead of each layer.

To evaluate the eventual performance of AI/ML approaches, we further evaluate average UPT versus feedback overhead and 5% tail UPT versus feedback overhead under up to 12 layers MU scheduling and FTP 3 traffic model with 50% and 70% resource utilization (RU). The simulation results with RU=50% are shown in Figure 6(a) ~ Figure 6(d), wherein Figure 6(a) for average UPT gain versus Maximum overhead, Figure 6(b) for 5% tail UPT gain versus Maximum overhead, Figure 6(c) for average UPT gain versus Weighted Avg. overhead, and Figure 6(d) for 5% tail UPT gain versus Weighted Avg. overhead.

Figure 6(a) Average UPT gain                       Figure 6(b) 5% tail UPT gain

versusMaximum overhead with RU=50%              versus Maximum overhead with RU=50%

Figure 6(c) Average UPT gain                       Figure 6(d) 5% tail UPT gain

versusWeighted Avg. overhead with RU=50%              versus Weighted Avg. overhead with RU=50%

The simulation results with RU=70% are shown in Figure 6(e) ~ Figure 6(h), wherein Figure 6(e) for average UPT gain versus Maximum overhead, Figure 6(f) for 5% tail UPT gain versus Maximum overhead, Figure 6(g) for average UPT gain versus Weighted Avg. overhead, and Figure 6(h) for 5% tail UPT gain versus Weighted Avg. overhead.

Figure 6(e) Average UPT gain                       Figure 6(f) 5% tail UPT gain

versus Maximum overhead with RU=70%              versus Maximum overhead with RU=70%

Figure 6(g) Average UPT gain                       Figure 6(h) 5% tail UPT gain

versusWeighted Avg. overhead with RU=70%              versus Weighted Avg. overhead with RU=70%

Observation 15: For up to rank 4, with the same feedback overhead, AI-based CSI reconstruction has about 4%-13% average UPT gain over the Rel-16 eType II under the case of 50% RU and 6%-16% average UPT gain can be obtained by AI based CSI reconstruction under the case of 70% RU.

Observation 16: For up to rank 4, with the same feedback overhead, AI-based CSI reconstruction has about 7%-13% performance gain for 5% tail UPT over the Rel-16 eType II under the case of 50% RU and 13%-22% performance gain for 5% tail UPT can be obtained by AI-based CSI recovery under the case of 70% RU.

Observation 17: With regard to up to rank 4, for average UPT, the margin of AI-based CSI recovery to ideal CSI feedback is 28% under the case of 50% RU, and 32% under the case of 70% RU.

Observation 18: With regard to up to rank 4, for 5% tail UPT, the margin of AI-based CSI recovery to ideal CSI feedback is 30% under the case of 50% RU, and 55% under the case of 70% RU.

Observation 19: The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for up to rank 4.

Observation 20: AI/ML approaches show better throughput performance with heavy traffic load for up to rank 4.

To evaluate the eventual performance of AI/ML approaches, we further evaluate average sector throughput versus feedback overhead and 5% tail UPT versus feedback overhead under up to 12 layers MU scheduling and full buffer traffic model. The simulation results are shown in Figure 7(a) ~ Figure 7(d), where Figure 7(a) for average sector throughput versus Maximum overhead, Figure 7(b) for 5% tail UPT gain versus Maximum overhead, Figure 7(c) for average sector throughput gain versus Weighted Avg. overhead, and Figure 7(d) for 5% tail UPT gain versus Weighted Avg. overhead.

Figure 7(a) Average sector throughput gain                   Figure 7(b) 5% tail UPT gain

versus Maximum overhead                              versus Maximum overhead

Figure 7(c) Average sector throughput gain               Figure 7(d) 5% tail UPT gain

versus Weighted Avg. overhead                    versus Weighted Avg. overhead

Observation 21: For up to rank 4, with the same feedback overhead, AI-based CSI reconstruction has about 9%-14% average sector throughput gains over the Rel-16 eType II under full buffer traffic.

Observation 22: For up to rank 4, with the same feedback overhead, AI-based CSI reconstruction has about 4%-7% performance gain for 5% tail UPT over the Rel-16 eType II under full buffer traffic.

Observation 23: With regard to up to rank 4, for average sector throughput, the margin of AI-based CSI recovery to ideal CSI feedback is 22% under full buffer traffic.

Observation 24: With regard to up to rank 4, for 5% tail UPT, the margin of AI-based CSI recovery to ideal CSI feedback is 48% under full buffer traffic.

Observation 25: With regard to up to rank 4, Maximum overhead shows similar performance comparison to Weighted Avg. overhead.

Proposal 6: The case of rank>1 should be prioritized in later discussion.

  - Model generalization

In this section, we provide some results for the evaluation of model generalization capability.

Various deployment scenarios:

We simulate AI generalization for various deployment scenarios, where the following cases are considered.

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

- Case 4: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B

Table 5-1 provides generalization performance in term of SGCS for scenarios of UMa, UMi, InH and mixed scenarios. The training dataset consists of 600000 samples and the testing dataset consists of 20000 samples for each generalization case. The mixed training dataset has two types, where the first type is Mixed type 1 with UMa:UMi=1:1, and the second type is Mixed type 2 with UMa:UMi:InH=1:1:1. For comparison, Table 4-1 also shows performance gains of baseline scheme (i.e., case 1). As shown in the Table 4-1, the generalization results of case 2 show large negative effect, where more than 0.02 performance loss exists in most testing scenarios. However, the generalization results of case 3 show marginal performance loss over baseline scheme, indicating that the mixed dataset is beneficial to model generalization.

Table 4-1 SGCS for various deployment scenarios

| Model training | Model testing Overhead: total 644bits with each layer for 322bits SGCS: Layer1/Layer2 SGCS gains: (Layer1/Layer2) |  |  |
| --- | --- | --- | --- |
|  | UMa | UMi | InH |
| UMa | 0.9218/0.8661 (0/0) | 0.9137/0.8560 (-0.0177/-0.0252) | 0.9339/0.9087 (-0.0165/-0.0222) |
| UMi | 0.9226/0.8636 (0.0008/-0.0025) | 0.9314/0.8812 (0/0) | 0.9367/0.9084 (-0.0137/-0.0225) |
| InH | 0.8706/0.7972 (-0.0512/-0.0689) | 0.8510/0.7685 (-0.0804/-0.1127) | 0.9504/0.9309 (0/0) |
| Mixed type 1: UMa:UMI=1:1 | 0.9226/0.8668 (0.0008/0.0007) | 0.9228/0.8696 (-0.0086/-0.0116) | 0.9403/0.9155 (-0.0101/-0.0154) |
| Mixed type 2: UMa:UMI:InH=1:1:1 | 0.9211/0.8652 (0.0007/-0.0009) | 0.9217/0.8691 (-0.0097/-0.0121) | 0.9609/0.9420 (0.0105/0.0111) |

Observation 26: Case 3(AI model trained in mixed deployment scenarios) shows good generalization performance for various deployment scenarios.

Observation 27: The AI model trained in the scenario of UMa/UMi shows good generalization performance to InH dataset, while the AI model trained in the scenario of InH shows generalization degradation to UMa/UMi dataset.

According to the above observation, we simulate the Case 4 that the AI/ML model trained based on training dataset from the scenario of InH is updated based on a fine-tuning dataset from the scenario of UMa/UMi and then the updated AI/ML model is tested on the dataset from the scenario of UMa/UMi. Table 4-2 provides corresponding generalization performance in term of SGCS. The training dataset consists of 600000 samples, the testing dataset consists of 20000 samples and the fine-tuning dataset comprises 10000 and 50000 samples. As shown in the Table 4-2, the generalization results of Case 4 show 1%-5% performance gain over Case 2, indicating that the fine-tuning dataset is beneficial to model generalization.

Table 4-2 SGCS for various deployment scenarios with fine-tuning

| Model training | Model testing Overhead: total 644bits with each layer for 322bits SGCS: Layer1/Layer2 SGCS gains: (Layer1/Layer2) |  |
| --- | --- | --- |
|  | UMa | UMi |
| UMa | 0.9218/0.8661 (0/0) | N/A |
| UMi | N/A | 0.9314/0.8812 (0/0) |
| InH | 0.8706/0.7972 (-0.0512/-0.0689) | 0.8510/0.7685 (-0.0804/-0.1127) |
| InH with fine-tuning dataset from UMi/UMa under 10K and 50K samples | 10K: 0.8781/0.8054 (-0.0437/-0.0607) 50K: 0.8827/0.8121 (-0.0391/-0.054) | 10K: 0.8621/0.7815 (-0.0693/-0.0997) 50K: 0.8783/0.8075 (-0.0531/-0.0737) |

Observation 28: The AI model trained in the scenario of InH and fine-tuned with the dataset from a scenario of UMa/UMi shows 1%-5% SGCS performance gain over the AI model trained only in the scenario of InH under the testing dataset from the scenario of UMa/UMi.

Observation 29: With the increased samples of fine-tuning dataset, the AI model shows better generalization performance.

Various configurations on bandwidth:

We simulate AI generalization for different datasets with bandwidths of 10MHz (52RBs) and 20MHz (104RBs). According to agreements made in previous meetings, 4RBs per sub-band for 10MHz and 8RBs per sub-band for 20MHz are adopted, hence the AI input sizes keep the same for both datasets, which are. Besides, two generalization cases are designed as follows and the results are shown in Figure 8. Compared with the cases of training and testing following the same bandwidth configuration, there is no obvious performance loss for both Case A and Case B.

- Case A: 10M dataset for training/validation and 20M dataset for testing

- Case B: 20M dataset for training/validation and 10M dataset for testing

Figure 8 AI generalization for different bandwidth configurations

Observation 30: AI/ML approaches can achieve good generalization performance for the case that the training/validation dataset and testing dataset are generated with different bandwidth configurations.

Various configurations on antenna port number:

We also simulate AI generalization for various antenna port numbers, where three cases of generalization are considered. Table 4-3 provides generalization performance in term of SGCS for port numbers of 16, 32 and their mixed version. The training dataset consists of 600000 samples and the testing dataset consists of 20000 samples for each generalization operation. For comparison, Table 4-3 also shows performance gains of baseline scheme (i.e., case 1). In addition, the generalizations of case 2 have large performance degradation, where generalization performance reduces more than 0.05. However, the case 3 generally shows better generalization performance than case 2, where the SGCS performance loss is less than 0.02.

Table 4-3 SGCS for various antenna port number

| Model training | Model testing: Overhead:total 644bits with each layer for 322bits SGCS: Layer1/Layer2 SGCS gains: (Layer1/Layer2) |  |
| --- | --- | --- |
|  | 16 Ports (Zero padding to 32 ports) | 32 Ports |
| 16 Ports (Zero padding to 32 ports) | 0.9470/0.9047 (0/0) | TBD |
| 32 Ports | 0.9151/0.8525 (-0.0319/-0.0522) | 0.9218/0.8661 (0/0) |
| Mixed: 16 Ports:32 Ports = 1:1 | 0.9553/0.9168 (0.0083/0.0121) | 0.9136/0.8534 (-0.0082/-0.0127) |

Observation 31: Case 3(AI model trained in mixed configurations of antenna port numbers) shows good generalization performance for various antenna port numbers.

Observation 32: AI model trained in the configuration of 32 antenna ports can maintain performance for 16 antenna ports.

Proposal 7: Case 3(AI model trained with mixed dataset) can be utilized to study AI/ML generalization capability.

- Preliminary Evaluation Results for CSI Prediction

As agreed in previous meetings, the evaluation assumptions in Rel-18 MIMO CSI prediction have been reused. In addition, most companies think there should be two baselines for time domain prediction. Especially for non-AI/ML based CSI prediction approach, it may be difficult to align the assumptions as companies may use different algorithms (e.g., Wiener filter, Kalman filter, etc.). However, it’s important to avoid over-estimating AI based time domain prediction. Though some companies may argue that non-AI/ML based CSI prediction approach should consider the enhancements made in Rel-18 MIMO by compressing multiple PMIs in Doppler domain, the codebook design in Rel-18 is not yet complete. As suggested by moderator in RAN1#110bis-e, we can start with some study on time domain CSI prediction without Doppler-domain compression.

Proposal 8: If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, the nearest historical CSI as well as non-AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML based CSI prediction is reported by companies.

- Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO which does not necessarily need to report multiple PMIs with Doppler-domain compression

We perform a preliminary simulation on the sub-use case of CSI prediction to evaluate the performance of AI models, the nearest historical CSI and a non-AI approach (i.e., Wiener filtering) are adopted for performance comparison. For AI/ML model, detailed training parameters are listed in Table 8-3. The channel data is divided into three parts which are training, validation, and testing datasets with 80000, 2000, and 2000 samples respectively. In our evaluation, historical channel measurements are used as the input of AI models for training, validation and testing. During the training process, since the input size of historical samples of raw channel matrices is so large that it may result in high AI model complexity. In order to reduce model complexity, we propose a Rx-common type of model input for AI model training, i.e., a single model is applied to all Tx-Rx chains. The parameter number of our AI/ML model is about 14.3K. In addition, the FLOPs of prediction in a future occasion are 3.9M.

Proposal 9: For CSI prediction, a Rx-common type of model input/output can be considered for intermediate KPIs and eventual performance evaluation as a starting point.

- Rx common-A single model is applied to all Tx-Rx chains

  - Performance comparison with non-AI approach

In this section, we initially evaluate the intermediate KPIs, e.g. NMSE and SGCS, between our AI approach and the non-AI baselines. We simulate both 4 historical CSIs and 10 historical CSIs as the inputs of AI model to predict 3 CSIs in future occasions, respectively. The results are shown in Table 5-1 and Table 5-2. According to the simulation results, both AI-based CSI prediction and Wiener filtering-based CSI prediction can completely outperform the nearest historical CSI. However, Wiener filtering-based non-AI CSI prediction shows competitive performance compared with the AI-based approach. This may be due to the simple channel model used for time prediction so that channel variation is only caused by Doppler shift but the large-scale and small-scale parameters are almost static in the measurement window and prediction window. Thus, non-AI/ML based CSI prediction approach can already work well in such assumptions. We may need to justify the scenarios that AI/ML based CSI prediction shows obvious advantages over non-AI/ML based CSI prediction, e.g., reduced decorrelation distance, high UE speed, low LOS probability etc.

Table 5-1 CSI prediction based on 4 historical CSIs

| # Predicted time | # P=1 (+5ms) |  |  | # P=2 (+10ms) |  |  | # P=3 (+15ms) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Type | # AI | # Wiener filtering | # Nearest historical CSI | # AI | # Wiener filtering | # Nearest historical CSI | # AI | # Wiener filtering | # Nearest historical CSI |
| # NMSE # (dB) | # -12.37 | # -11.37 | # 0.733 | # -6.24 | # -4.45 | # 1.93 | # -5.44 | # -2.36 | # 3.74 |
| # SGCS | # 0.952 | # 0.944 | # 0.780 | # 0.800 | # 0.787 | # 0.727 | # 0.762 | # 0.716 | # 0.709 |

Table 5-2 CSI prediction based on 10 historical CSIs

| # Predicted time | # P=1 (+5ms) |  |  | # P=2 (+10ms) |  |  | # P=3 (+15ms) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Type | # AI | # Wiener filtering | # Nearest historical CSI | # AI | # Wiener filtering | # Nearest historical CSI | # AI | # Wiener filtering | # Nearest historical CSI |
| # NMSE # (dB) | # -18.94 | # -20.24 | # 0.713 | # -9.50 | # -9.92 | # 1.97 | # -6.42 | # -5.63 | # 3.75 |
| # SGCS | # 0.989 | # 0.992 | # 0.782 | # 0.906 | # 0.924 | # 0.731 | # 0.806 | # 0.829 | # 0.711 |

Observation 33: Both AI-based CSI prediction and Wiener filtering-based CSI prediction can completely outperform the nearest historical CSI. However, Wiener filtering-based non-AI CSI prediction shows even better performance than the AI-based approach when enough historical CSIs are applied.

Observation 34: The prediction accuracy of AI-based approach and Wiener filtering-based approach drops seriously when the predicted time becomes longer due to the channel aging. However, AI-based CSI prediction can maintain the performance for a longer time than Wiener filtering-based CSI prediction.

Proposal 10: Further evaluate various scenarios to justify that AI/ML based CSI prediction shows obvious advantages over non-AI/ML based CSI prediction, e.g., reduced length of measurement window, reduced decorrelation distance, high UE speed, low LOS probability, etc.

  - Diverse historical CSI samples for AI CSI prediction

In order to evaluate the performance influenced by the length of observation window, we set diverse samples of historical CSIs as AI model inputs. The simulation results are shown in Table 5-3. According to the simulation results, the prediction accuracy of AI-based approach and Wiener filtering-based approach improves with the increasing number of historical CSIs as model input while AI-based CSI prediction shows less or even no performance gain over Wiener filtering-based algorithm with the increasing number of input historical CSIs.

Table 5-3 CSI prediction based on diverse samples of historical CSIs

| # Historical samples | # T=3 |  | # T=4 |  | # T=6 |  | # T=8 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Type | # AI | # Wiener filtering | # AI | # Wiener filtering | # AI | # Wiener filtering | # AI | # Wiener filtering | # Nearest historical CSI |
| # NMSE(dB) | # -10.23 | # -5.44 | # -12.37 | # -11.37 | # -15.45 | # -17.11 | # -17.27 | # -20.69 | # 0.7134 |
| # SGCS | # 0.926 | # 0.820 | # 0.952 | # 0.944 | # 0.979 | # 0.988 | # 0.985 | # 0.996 | # 0.780 |

Observation 35: The prediction accuracy of both AI-based approach and Wiener filtering-based approach improve with the increasing number of historical CSIs as input and presents certain positive correlation with the number of input historical CSIs.

Observation 36: AI-based CSI prediction shows less or even no performance gain over Wiener filtering-based algorithm with the increasing number of input historical CSIs.

Proposal 11: Various lengths of observation window and prediction window should be evaluated to have a fair comparison between AI-based CI prediction and non-AI based CSI prediction.

  - Diverse input CSI types for AI CSI prediction

In sub-section 5.1 and 5.2, the input of AI-based model and Wiener filtering-based algorithm is the historical samples of channel matrices and the output is the predicted channel matrices. In order to evaluate the performance influenced by the diverse input types, we adopt the samples of historical sub-band eigenvectors processed from the raw channel matrices as input and the corresponding predicted eigenvectors as output. For AI/ML model, detailed training parameters are listed in Table 8-4. The parameter number of our AI/ML model is about 16.03K. In addition, the FLOPs of prediction in a future occasion are 1.21M. The simulation results are shown in Table 5-4. According to the simulation results, the prediction accuracy of AI-based approach outperforms non-AI approaches when eigenvectors are adopted as model input. However, the performance drops dramatically compared with the input of raw channel, since the spatial/time/frequency consistency features may be impacted during pre-processing on raw channels to get corresponding eigenvectors.

Table 5-4 CSI prediction based on 10 historical sub-band eigenvectors

| # Predicted time | # P=1 (+5ms) |  |  | # P=2 (+10ms) |  |  | # P=3 (+15ms) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Type | # AI | # Wiener filtering | # Nearest historical CSI | # AI | # Wiener filtering | # Nearest historical CSI | # AI | # Wiener filtering | # Nearest historical CSI |
| # SGCS | # 0.8444 | # 0.8041 | # 0.7993 | # 0.7676 | # 0.7340 | # 0.7494 | # 0.7460 | # 0.6994 | # 0.7310 |
| # NMSE # (dB) | # -5.088 | # -3.679 | # -3.521 | # -3.833 | # -2.375 | # -2.773 | # -3.536 | # -1.812 | # -2.581 |

Observation 37: The prediction accuracy of AI-based approach outperforms non-AI approaches when sub-band eigenvectors are adopted as model input.

Observation 38: The performance of eigenvector prediction for AI-based approach drops dramatically compared with the input of raw channel, and AI-based approach shows marginal performance gain over non-AI algorithms.

Proposal 12: Further study the input and output types for the sub-use case of AI/ML based CSI prediction.

- Conclusion

In this contribution, we discuss the evaluations on AI/ML for CSI feedback enhancement, and provide preliminary simulation results. We have the following observations and proposals.

Proposal 1: Ideal CSI feedback should be adopted as an additional throughput baseline at least for calibration purpose.

Proposal 2: Enhancements on CSI feedback using non-AI/ML approaches (e.g. Wideband (WB) covariance matrix, full rank information) can be considered as an additional baseline for performance comparison.

Proposal 3: For rank>1, two cases on model input/output can be considered for intermediate KPIs and eventual performance evaluation as a starting point.

- Case 1: Layer common-A single model is applied to all layers and all ranks

- Case 2: Rank specific- Multiple models are trained and each model is applied to a specific rank

Proposal 4: For CSI dataset construction, dataset should be generated in diverse drops rather than in divers TTIs at least for calibration purpose (e.g., only one TTI can be used for generating training dataset in a single drop).

Proposal 5: For CSI payload size calculation, Option 1 is adopted as baseline evaluation metric and Option 2b is up to companies to report.

Proposal 6: The case of rank>1 should be prioritized in later discussion.

Proposal 7: Case 3(AI model trained with mixed dataset) can be utilized to study AI/ML generalization capability.

Proposal 8: If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, the nearest historical CSI as well as non-AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML based CSI prediction is reported by companies.

- Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO which does not necessarily need to report multiple PMIs with Doppler-domain compression

Proposal 9: For CSI prediction, a Rx-common type of model input/output can be considered for intermediate KPIs and eventual performance evaluation as a starting point.

- Rx common-A single model is applied to all Tx-Rx chains

Proposal 10: Further evaluate various scenarios to justify that AI/ML based CSI prediction shows obvious advantages over non-AI/ML based CSI prediction, e.g., reduced length of measurement window, reduced decorrelation distance, high UE speed, low LOS probability, etc.

Proposal 11: Various lengths of observation window and prediction window should be evaluated to have a fair comparison between AI-based CI prediction and non-AI based CSI prediction.

Proposal 12: Further study the input and output types for the sub-use case of AI/ML based CSI prediction.

Observation 1: For rank 1, AI based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS. With the same feedback overhead, AI based CSI recovery can obtain 7%-8% SGCS gains.

Observation 2: For rank 1, AI-based CSI reconstruction achieves very little average UPT gain, where less than 1% average UPT gain over Rel-16 eTypeII with the same feedback overhead is obtained, and the margin of AI-based CSI recovery to ideal CSI feedback is a little more than 1% as AI-based CSI reconstruction and Rel-16 eTypeII can already work well in single layer scheduling.

Observation 3: For rank 1, AI based CSI reconstruction achieves 2% ~ 20% performance gain for 5% tail UPT, and the margin of AI to Ideal is 7%.

Observation 4: For rank 2, AI based CSI reconstruction with Case 1 method (single-layer model input and single-layer model output) shows performance gains in SGCS per layer over the Rel-16 eType II.

Observation 5: Case 1(layer common) can achieve better performance than Case 2 (rank specific).

Observation 6: For up to rank 2, with the same feedback overhead, AI based CSI reconstruction has about 4%-6% average UPT gain over the Rel-16 eType II under the case of 50% RU and 5%-8.5% average UPT gain can be obtained by AI based CSI recovery under the case of 70% RU.

Observation 7: For up to rank 2, with the same feedback overhead, AI based CSI reconstruction has about 10%-37% performance gain for 5% tail UPT over the Rel-16 eType II under the case of 50% RU and 10%-25% performance gain for 5% tail UPT can be obtained by AI based CSI recovery under the case of 70% RU.

Observation 8: With up to rank 2, for average UPT, the margin of AI to Ideal is 6% under the case of 50% RU, and 11% under the case of 70% RU.

Observation 9: With up to rank 2, for 5% tail UPT, the margin of AI to Ideal is 30% under the case of 50% RU, and 42% under the case of 70% RU.

Observation 10: The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for maximum rank up to 2.

Observation 11: AI/ML approaches show better average throughput performance with heavy traffic load for up to rank 2.

Observation 12: For up to rank 2, Maximum overhead shows similar performance comparison to Weighted Avg. Overhead.

Observation 13: AI based CSI reconstruction shows performance gain in SGCS per layer over the Rel-16 eType II for rank 3/4.

Observation 14: AI based CSI reconstruction shows larger performance gains in layer 3/4 than layer 1/2 in terms of SGCS with the assumption of the same feedback overhead of each layer.

Observation 15: For up to rank 4, with the same feedback overhead, AI based CSI reconstruction has about 4%-13% average UPT gain over the Rel-16 eType II under the case of 50% RU and 6%-16% average UPT gain can be obtained by AI based CSI reconstruction under the case of 70% RU.

Observation 16: For up to rank 4, with the same feedback overhead, AI based CSI reconstruction has about 7%-13% performance gain for 5% tail UPT over the Rel-16 eType II under the case of 50% RU and 13%-22% performance gain for 5% tail UPT can be obtained by AI based CSI recovery under the case of 70% RU.

Observation 17: With up to rank 4, for average UPT, the margin of AI to Ideal is 28% under the case of 50% RU, and 32% under the case of 50% RU.

Observation 18: With up to rank 4, for 5% tail UPT, the margin of AI to Ideal is 30% under the case of 50% RU, and 55% under the case of 70% RU.

Observation 19: The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for up to rank 4.

Observation 20: AI/ML approaches show better throughput performance with heavy traffic load for up to rank 4.

Observation 21: For up to rank 4, with the same feedback overhead, AI based CSI reconstruction has about 9%-14% average sector throughput gains over the Rel-16 eType II under full buffer traffic.

Observation 22: For up to rank 4, with the same feedback overhead, AI based CSI reconstruction has about 4%-7% performance gain for 5% tail UPT over the Rel-16 eType II under full buffer traffic.

Observation 23: With up to rank 4, for average sector throughput, the margin of AI to Ideal is 22% under full buffer traffic.

Observation 24: With up to rank 4, for 5% tail UPT, the margin of AI to Ideal is 48% under full buffer traffic.

Observation 25: With regard to up to rank 4, Maximum overhead shows similar performance comparison to Weighted Avg. overhead.

Observation 26: Case 3(AI model trained in mixed deployment scenarios) shows good generalization performance for various deployment scenarios.

Observation 27: The AI model trained in the scenario of UMa/UMi shows good generalization performance to InH dataset, while the AI model trained in the scenario of InH shows generalization degradation to UMa/UMi dataset.

Observation 28: The AI model trained in the scenario of InH and fine-tuned with the dataset from a scenario of UMa/UMi shows 1%-5% SGCS performance gain over the AI model trained only in the scenario of InH under the testing dataset from the scenario of UMa/UMi.

Observation 29: With the increased samples of fine-tuning dataset, the AI model shows better generalization performance.

Observation 30: AI/ML approaches can achieve good generalization performance for the case that the training/validation dataset and testing dataset are generated with different bandwidth configurations.

Observation 31: Case 3(AI model trained in mixed configurations of antenna port numbers) shows good generalization performance for various antenna port numbers.

Observation 32: AI model trained in the configuration of 32 antenna ports can maintain performance for 16 antenna ports.

Observation 33: Both AI-based CSI prediction and Wiener filtering-based CSI prediction can completely outperform the nearest historical CSI. However, Wiener filtering-based non-AI CSI prediction shows even better performance than the AI-based approach when enough historical CSIs are applied.

Observation 34: The prediction accuracy of AI-based approach and Wiener filtering-based approach drops seriously when the predicted time becomes longer due to the channel aging. However, AI-based CSI prediction can maintain the performance for a longer time than Wiener filtering-based CSI prediction.

Observation 35: The prediction accuracy of both AI-based approach and Wiener filtering-based approach improve with the increasing number of historical CSIs as input and presents certain positive correlation with the number of input historical CSIs.

Observation 36: AI-based CSI prediction shows less or even no performance gain over Wiener filtering-based algorithm with the increasing number of input historical CSIs.

Observation 37: The prediction accuracy of AI-based approach outperforms non-AI approaches when sub-band eigenvectors are adopted as model input.

Observation 38: The performance of eigenvector prediction for AI-based approach drops dramatically compared with the input of raw channel, and AI-based approach shows marginal performance gain over non-AI algorithms.

- References

- Chairman’s notes, RAN1 #110bis-e.

- R1-2201158, Discussion on other aspects for AI CSI feedback enhancement, ZTE Corporation.

- R1-2210753 Summary#7 of [110bis-e-R18-AIML-02]_CSI_Eva

- R1-2210937, CSI enhancement for high/medium UE velocities and CJT, ZTE

- Chairman’s notes, RAN1 #109-e

- R1-2203248, Evaluation assumptions on AIML for CSI feedback, ZTE Corporation

- R1-2207840 Summary#3_9.2.2.1_CSI_Eva_v002, RAN1 #110

- Appendix

Table 8-1 SLS assumptions for AI/ML based CSI feedback

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, 2GHz |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 41 dBm for 10MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz |
| Simulation bandwidth |  | 10MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation. |
| MIMO layers |  | Maximum MU 4 layers for max rank=1 Maximum MU 8 layers for max rank=2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme CSI feedback periodicity (full CSI feedback) : 5 ms, Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | 2 OFDM symbols for PDCCH，type 2 for DMRS(24 REs/PRB/slot) CSI-RS overhead(32 REs/PRB/5 slot) |
| Traffic model |  | FTP 3 |
| Traffic load (Resource utilization) |  | RU 50% and 70% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead |
| Baseline for performance evaluation |  | Rel-16 TypeII Codebook |

Table 8-2  Training parameters of AI/ML model for CSI compression

| Parameter | Value |
| --- | --- |
| Backbone | Transformer |
| Parameter type | Real value |
| Input CSI type | Eigenvectors of the ideal channel matrix estimated by UE |
| Output CSI type | Recovered eigenvectors by AI/ML model in gNB |
| Data-processing | Normalization |
| Quantization | Vector quantization |
| Training dataset | 600000 |
| Validation dataset | 10000 |
| Testing dataset | 20000 |
| Batch size | 400 |
| Optimizer | Adam |
| Loss function | MSE |

Table 8-3  Training parameters of AI/ML model for CSI prediction based on raw channels

| Parameter | Value |
| --- | --- |
| Backbone | ResNet |
| Parameter type | Real value |
| Input CSI type | Historical channel matrices measured by UE |
| Output CSI type | Predicted channel matrix by AI/ML model in UE |
| Model input size | () |
| Model output size | () |
| Training dataset | 80000 |
| Validation dataset | 2000 |
| Testing dataset | 2000 |
| Batch size | 200 |
| Optimizer | Adam |
| Loss function | MSE |

Table 8-4  Training parameters of AI/ML model for CSI prediction based on eigenvectors

| Parameter | Value |
| --- | --- |
| Backbone | ResNet |
| Parameter type | Real value |
| Input CSI type | Sub-band eigenvectors |
| Output CSI type | Predicted eigenvectors by AI/ML model in UE |
| Model input size | () |
| Model output size | () |
| Training dataset | 80000 |
| Validation dataset | 2000 |
| Testing dataset | 2000 |
| Batch size | 200 |
| Optimizer | Adam |
| Loss function | MSE |