3GPP TSG RAN WG1 #111 meeting                                                            			  R1-2211066

Toulouse France, Nov 14th – Nov 18th, 2022

Agenda Item:	9.3.3

Source:	TCL

Title:	Potential Enhancement on dynamic/flexible TDD

Document for:	Discussion and Decision

# Introduction

In 3GPP TSG RAN1 # 110bis-e meeting, regarding the potential enhancement on dynamic/flexible TDD the following agreements were achieved [1].

Agreement For gNB-to-gNB co-channel CLI measurement, the potential benefit of uplink resources muting can be studied further. Note: Proponents of uplink resource muting are encouraged to provide evaluation result for comparison of performance between two cases when uplink resource muting based gNB-gNB CLI handling schemes including both UE transparent and non-UE transparent schemes is applied or not. Agreement For gNB-to-gNB co-channel CLI measurement, consider as baseline reusing existing DL channel(s)/signal(s)/measurement_resource(s) - For example, SSB, NZP/ZP-CSI-RS, DMRS for PDCCH/PDSCH, CSI-IM, RSSI measurement resource, etc. - FFS: Which type of DL channel(s)/signal(s) can be used for gNB-to-gNB co-channel CLI measurement - FFS: How resources are used/configured Agreement For UE-to-UE co-channel CLI measurement, consider as baseline reusing existing channel(s)/signal(s)/measurement_resource(s) - For example, SRS resources defined in Rel-16 for SRS-RSRP measurement, CLI-RSSI resources defined in Rel-16 for CLI-RSSI measurement - FFS potential enhancements Agreement For UE-to-UE co-channel CLI handling, study L1/L2 based UE-to-UE CLI measurement and reporting - Note: Accounting for UE processing/reporting delay – companies to share their assumptions - Note: Proponents are encouraged to provide the mechanism of L1/L2 based CLI measurement and reporting, and to provide the benefits of L1/L2 based CLI measurement and reporting compared with existing L3 CLI/CSI measurement and report with evaluation result - Note: Accounting for information exchange delay between gNBs (if applicable) Agreement For details of spatial domain coordination method for gNB-to-gNB co-channel CLI handling, at least followings can be studied. - Recommended/restricted Beams between gNBs - Beam nulling between gNBs - Beam pairing between gNBs - Other schemes are not precluded.

In this contribution, we provide our views on the details of coordinated scheduling for time/frequency resources which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD.

# Coordinated Scheduling for time/frequency resources between gNBs

Coordinated scheduling for time/frequency resources were discussed in 3GPP RAN1#110bis-e meeting, however the details of coordinated scheduling, such as the relevant information exchange and the scheduling adaptation solutions except muting resources were not agreed for further study. In this section we provide our views on the details of coordinated scheduling for time/frequency resources between gNBs.

## Relevant information exchange

The basic requirement of coordinated scheduling for time/frequency resources between gNBs is the relevant information exchange between the gNBs. Based on the relevant information exchange the gNBs can perform coordinated scheduling i.e. muting UL resources among gNBs or blanking DL resources at the aggressor gNB. In addition, the relevant information exchange may different from one scenario to another scenario according to the implementation of dynamic TDD and SBFD operations. For instance, if both the aggressor and victim gNBs perform dynamic TDD operation at the same time slots, they may need to exchange the dynamic TDD slots format only. On the other hand, if aggressor gNB performs dynamic TDD operation and the victim gNB performs SBFD operation at the same time slots, they may need to exchange the dynamic TDD slots format as well as the DL and UL subbands information (i.e. the starting and numbers of RBs for DL and UL subband).

Observations 1: For coordinated scheduling of time frequency resources between the gNBs, it may necessary to consider the relevant information exchange between the aggressor and victim gNBs based on the implementation of dynamic TDD and SBFD operation.

Observation 2: An aggressor gNB performing dynamic TDD operation may exchange slot format with its victim gNBs, and an aggressor gNB performing SBFD operation may exchange SBFD slot format as well as the starting and numbers of RBs assigned for each DL and UL sub-bands.

Proposal 1: For coordinated scheduling of time frequency resources between the gNB, consider the exchange of the following relevant information between the gNBs.

- TDD slot format

- SBFD slot format

- RBs assigned to the DL and UL subband

## Scheduling Adaptation

Based on the information exchange among the aggressor and victim gNBs, the gNBs can perform scheduling adaptation to reduce or avoid the gNB to gNB and UE to UE CLI. For instance, an aggressor gNB which performs dynamic TDD operation can mute those RBs which corresponds to the RBs of the opposite direction sub-band at a victim gNB, to avoid or reduce the co-channel gNB to gNB and UE to UE CLI as shown in Figure 1. Furthermore, the gNB which performs SBFD operation can assign more sub-bands resources to the same transmission direction of the gNB which performs dynamic TDD operation, in order to minimize the number of muting RBs at the aggressor gNB.

Figure 1 Scheduling adaptation of dynamic TDD (DL) at gNB1 and SBFD operation at gNB2

In addition, the coordinated scheduling for time frequency resources shall not be restricted to muting or blanking the DL and UL resources only. Several other coordinated scheduling based solutions can be adopted to reduce the effect of CLI. For instance, the time window based solutions can be used by the neighbor gNBs  to reduce the effect of CLI as shown in Figure 2, where the aggressor and victim gNBs can assign a time domain windows of X slots/symbols to the dynamic TDD operation and SBFD operation respectively.

Figure 2 Scheduling adaptation of same direction dynamic TDD (DL) and SBFD operation at gNB1 and gNB2

Observation 5: In coordinated scheduling for time frequency resources between gNBs, muting the DL RBs or blanking the UL RBs can reduce the effect of gNB to gNB co-channel CLI.

Observation 6: In simultaneous implementation of dynamic TDD and SBFD operation at a gNB and its neighbor gNB the following scheduling adaptation techniques can reduce or avoid the gNB to gNB co-channel CLI.

- Each gNB can assign a time window to the dynamic TDD operation and a time window to the SBFD operation.

- Allocating the same numbers of slots or symbols in the time windows assigned to the dynamic TDD or SBFD operation across the neighbor gNBs.

Proposal 2: For coordinated scheduling of time frequency resources between gNBs for gNB to gNB co-channel CLI handling, consider the study of at least the following.

- RB based Resource muting or blanking to support CLI mitigation in dynamic TDD and SBFD operation.

- Time domain window based solution to handle CLI in both dynamic TDD and SBFD operation.

# Spatial Domain coordination

In spatial domain, the neighbor gNBs can exchange the relevant information of the beams which is assigned to the DL transmission and UL transmission as shown in Figure 3. In this way, the neighbor gNBs can adjust their beams according to each other which may help to reduce the gNB to gNB and UE to UE CLI.

Figure 3 Spatial Domain Enhancement for dynamic TDD and SBFD operation

Observation 7: Inter gNB coordination among the neighbor gNBs can identify suitable beams to reduce the impact of inter gNB CLI.

Proposal 3: Consider the information exchange of Tx and Rx beams, based on the beam IDs assigned to the DL and UL transmission among the neighbor base stations.

# Conclusion

In this contribution we discussed, coordinated scheduling for time/frequency resources, and spatial domain coordination, which are common for dynamic/flexible TDD and SBDF operation, and made the following observations and proposals.

Observations 1: For coordinated scheduling of time frequency resources between the gNBs, it may necessary to consider the relevant information exchange between the aggressor and victim gNBs based on the implementation of dynamic TDD and SBFD operation.

Observation 2: An aggressor gNB performing dynamic TDD operation may exchange slot format with its victim gNBs, and an aggressor gNB performing SBFD operation may exchange SBFD slot format as well as the starting and numbers of RBs assigned for each DL and UL sub-bands.

Observation 5: In coordinated scheduling for time frequency resources between gNBs, muting the DL RBs or blanking the UL RBs can reduce the effect of gNB to gNB co-channel CLI.

Observation 6: In simultaneous implementation of dynamic TDD and SBFD operation at a gNB and its neighbor gNB the following scheduling adaptation techniques can reduce or avoid the gNB to gNB co-channel CLI.

- Each gNB can assign a time window to the dynamic TDD operation and a time window to the SBFD operation.

- Allocating the same numbers of slots or symbols in the time windows assigned to the dynamic TDD or SBFD operation across the neighbor gNBs.

Observation 7: Inter gNB coordination among the neighbor gNBs can identify suitable beams to reduce the impact of inter gNB CLI.

Proposal 1: For coordinated scheduling of time frequency resources between the gNB, consider the exchange of the following relevant information between the gNBs.

- TDD slot format

- SBFD slot format

- RBs assigned to the DL and UL subband

Proposal 2: For coordinated scheduling of time frequency resources between gNBs for gNB to gNB co-channel CLI handling, consider the study of at least the following.

- RB based Resource muting or blanking to support CLI mitigation in dynamic TDD and SBFD operation.

- Time domain window based solution to handle CLI in both dynamic TDD and SBFD operation.

Proposal 3: Consider the information exchange of Tx and Rx beams, based on the beam IDs assigned to the DL and UL transmission among the neighbor base stations.

# References

3GPP TSG RAN WG1 #110bis-e meeting (Oct 10th – Oct 19th 2022)