3GPP TSG RAN WG1 Meeting #111	R1-2211097

Toulouse, France, November 14th – 18th, 2022

Agenda item:		9.7.1

Source:	Nokia, Nokia Shanghai Bell

Title:		NW Energy Savings Performance Evaluation

Document for:		Discussion and Decision

# Introduction

Based on the email and online GTW discussions in RAN1#110bis-e meeting, the agreements and working assumption were made as shown in Appendix B.

In this contribution, we will provide our simulation results of some energy saving techniques in different domains based on the template table provided by the FL from RAN1#110bis-e meeting. And in our companion contribution, we discuss the agenda item on network energy saving techniques [1].

# System-level evaluation results

The system-level simulation parameters are presented in Table 11 of Appendix A as per the latest agreements. The evaluation results are provided for the urban macro scenario, which is to be prioritized.

In the following, we provide the detailed system-level simulation results of the evaluated energy saving techniques in different domains.

For all the results, we evaluated the BS performance in terms of ES gain (as compared to a baseline defined for each technique) and the UPT with respect to Set 1 reference configuration and using CAT2 power values.

## Techniques in time domain

### Technique #A-1: Adapting transmission/reception of common channels/signals

We simulated the Technique #A-1 defined in [1] by relaxing the periodicity of SIB1/SSB transmission and random-access occasion (RO) monitoring from 20 ms (as baseline) to 1280 ms, which enables the BS to enter sleep states, when it is possible.

In our simulations, we consider a single cell scenario. We assume that UEs can be initially either in RRC_IDLE or in RRC_CONNECTED state. For RRC_IDLE UEs, they need to acquire SSBs and SIB1, and perform RA procedure with RRC state switching to RRC_CONNECTED state before any data activity. All CP and UP procedures related to RRC state switching are based on the assumptions used in [2] and [3], respectively. When UEs are moved to RRC_CONNECTED state, they apply the Connected- Discontinuous Reception (DRX) settings corresponding to FTP traffic considered in the evaluation of Technique #A-1.

For the calculation of DL-UPT, the delay relative to the DRX cycle is omitted, where the scheduling delay is only considered to emphasize the impact of the load and the RRC switching delay for UEs starting at RRC_IDLE state.

On the evaluation results provided in Figure-1, with unloaded scenario (i.e., 0% load), up to 84% ES gain is achieved as compared to the baseline, which is explained by longer BS sleeping opportunities. With SIB1/SSB/RO periodicity longer than 640ms, the gains are significantly increased because the BS may have larger chance to enter the light sleep state for better energy saving.

For loaded scenarios, the ES gains are lower as compared to the unloaded scenario due to the DL data activity which prevents the BS from entering a deeper sleep state than micro sleep state. However, the simulation results show that the ES gain is still significant, which is around 51% for all load levels with a periodicity of 1280 ms for SSB and SIB1 transmission and RO monitoring.

The DL-UPT is decreased for UEs initially in RRC_IDLE state due to longer periodicity of SSB/SIB1/RO impacting the initial access delay required before data reception. For RRC connected UEs, they experience a DL-UPT of 200 Mbps, 142 Mbps and 78 Mbps, at low, light and medium load, respectively.

| NW energy saving scheme | ES gain for each configuration | UPT | Evaluation methodology/baseline assumption | Note |
| --- | --- | --- | --- | --- |
| Relaxed periodicity of SIB1/SSB TX and RO monitoring | Load; SSB/SIB1/RO periodicity [ms]; ES gain. 0%; Below 640; Up to 54%. 1280; 84%. >0%; Below 640; Up to 48%. 1280; 51%. | - UEs initially in RRC_IDLE state SSB/SIB1/RO periodicity (ms); Load; Average DL-UPT [Mbps]. 20; 15%; 129. 30%; 100. 50%; 65. 160; 15%; 83. 30%; 70. 50%; 55. - UEs initially in RRC_CONNECTED state Load; Average DL-UPT (Mbps) (Irrespective of the SIB1/SSB/RO periodicity). 15%; 200. 30%; 142. 50%; 80. | Results are provided as per Set 1-Cat 2 power values. Baseline: Single cell with 20 ms periodicity of SSB/SIB1 Tx and RO monitoring. Traffic: DL FTP traffic is assumed at different load levels. UEs: initially assumed to be in RRC_IDLE state or in RRC_CONNECTED state. | Slot-level modeling is assumed. |

Figure 1: ES gain and impact on DL-UPT (for UEs assumed initially in RRC_IDLE state) from relaxing periodicity of SIB1/SSB TX and RO monitoring.

Observation-1: Adapting the transmission periodicity of SSB/SIB1 and/or uplink random access opportunities brings significant ES gains of 84% for 0% load and up to 51% for loaded scenarios at the expense of DL-UPT only for UEs initially in RRC_IDLE state (whereas the network can ensure that there is no impact on UPT for RRC Connected UEs).

### Technique #A-3: Wake up of gNB triggered by UE wake up signal (WUS)

For the evaluation of Technique #A-3 defined in [1], we consider that the BS is capable of transmitting Discovery Reference Signal (DRS) i.e. PSS+SSS, with a certain periodicity to announce its presence. It helps UEs to acquire synchronization before sending a wake-up signal (WUS) request in case of UL data arriving, which triggers the SSB/SIB1 transmission by the BS.

For DRS, we assumed a simplified SSB occupying only 2-symbols, and consisting of PSS and SSS, whereas for WUS, the legacy PRACH signal is assumed.

In our evaluation on a single cell, the SSB/SIB1 are transmitted on demand, only when WUS is received from a UE requesting full SSB/SIB1 transmission before its UL data transmission (as per IM traffic model defined as per RAN1 agreements).

The results are shown in Figure 2 and in the table below. By reducing the time spent in active DL state for transmitting SSB/SIB1, the unloaded BS can achieve up to 84% ES gain with a DRS/WUS period of 1280ms. This gain is slightly decreased when the UL traffic load is increased, where 67% ES gain is obtained for 15% UL traffic load with the same DRS/WUS period. This is explained by the reduced time in sleep state (e.g. light sleep state in the case where DRS/WUS period is higher than 640 ms) as compared to the unloaded scenario, in which the BS can enter and remain in light sleep until the next DRS/WUS period.

With longer DRS/WUS periodicity from 20 to 160ms, the UL-UPT is decreased from 13 Mbps to 6 Mbps. This comes with a longer random-access delay perceived by RRC_IDLE UEs that intend to wake up a dormant cell for acquiring full SSB/SIB1, and for UL data transmission.

| NW energy saving scheme | ES gain for each configuration | UPT | Evaluation methodology/baseline assumption | Note |
| --- | --- | --- | --- | --- |
| DRS + On-demand SSB/SIB1 (triggered by WUS) | Load; DRS/WUS periodicity [ms]; ES gain. 0%; 20; 50%. Below 640; Up to 55%. 1280; 84%. 15%; 20; 46%. Below 640; Up to 53%. 1280; 67%. | Load; DRS/WUS periodicity [ms]; Average UL-UPT [Mbps]. 15%; 20; 13. 160; 6. | Results are provided as per Set 1-Cat 2 power values. Baseline: Single cell with 20 ms periodicity of SSB/SIB1 Tx and RO monitoring. Traffic: UL IM traffic is assumed at 15% load level. UEs: are assumed to be initially in RRC_IDLE state. | Slot-level modeling is assumed. |

Figure 2: ES gain and impact on UL-UPT from enabling DRS/WUS-triggering SSB/SIB1 transmission.

Observation-2: Enabling on-demand SSB/SIB1 transmission triggered by the UE’s WUS (instead of legacy SSB/SIB1 transmission and RO monitoring) enables the BS to achieve significant ES gain by reducing the time spent in Active DL. However, on the other hands, it comes with a longer random-access delay perceived by RRC_IDLE UEs that intend to wake up a dormant cell for acquiring the SSB/SIB1 before the UL data transmission.

### Technique #A-6 Adaptation of SSB/SIB1

For the adaptation of SSB/SIB1 transmission, the target scenario is the intra-band/collocated cells with non-CA case, consisting of a coverage cell (i.e., Macro cell, with no ES schemes applied) and a capacity cell (i.e. small/pico cell) in which we apply the following ES schemes:

- SSB-less cell: the capacity cell provides SIB1 and monitors for RO with a periodicity ranging from 20 ms (baseline) to 1280 ms. We assume that the UE acquires synchronization based on the coverage cell.

- SIB1-less cell: the capacity cell provides SSB and monitors for RO with a periodicity ranging from 20 ms (baseline) to 1280 ms, its SIB1 is provided by the coverage cell, as a supplementary SIB1, occupying additional resources either in frequency or time domain.

- SSB&SIB1-less cell: the capacity cell only monitors for RO; SIB1 is provided by the coverage cell, and we assume that the UE acquires synchronization based on the coverage cell.

In our simulations, we assume that UEs are camping in the capacity cell, initially in RRC_IDLE, or they can be in RRC_CONNECTED state in the capacity cell. We considered DL FTP traffic generating load at different levels at the capacity cell. However, the coverage cell is maintained with a fixed load of 50%.

| NW energy saving scheme | ES gain for each configuration | UPT | Evaluation methodology/baseline assumption | Note |
| --- | --- | --- | --- | --- |
| SSB-less | Load; RO period [ms]; ES gain. 15%; 20; 26%. 160; 48%. 1280 ms; 53%. | - UEs initially in RRC_IDLE Load; RO period [ms]; Average DL-UPT [Mbps]. 15%; 20; 135. 160; 85. - UEs initially in RRC_CONNECTED 200Mbps is achieved at 15% load regardless of the RO monitoring period. | Results are provided as per Set 1-Cat 2 power values. Baseline consisting of: - Coverage cell with 20 ms periodicity of SSB/SIB1 Tx and RO monitoring, loaded at 50%. - Capacity cell with 20 ms periodicity of SSB/SIB1 Tx and RO monitoring, loaded at 15%. Traffic: DL FTP traffic is assumed. UEs: are camping in the capacity cell and initially assumed to be in RRC_IDLE state or in RRC_CONNECTED state. | Slot-level modeling is assumed. |
| SIB1-less | Load; SSB/RO period [ms]; ES gain. 15%; 20; 14%. 160; 51%. 1280 ms; 54%. | - UEs initially in RRC_IDLE state Load; RO period [ms]; Average DL-UPT [Mbps]. 15%; 20; 132. 160; 84. - UEs initially in RRC_CONNECTED 200Mbps is experienced at 15% load regardless of the SSB/RO monitoring period. |  |  |
| SSB&SIB1-less | RO period [ms]; Load; ES gain. 20; 0%; 53%. 15%; 51%. 30%; 49%. 50%; 44%. 160; 0%; 55%. 15%; 53%. 30%; 51%. 50%; 47%. | RO period [ms]; Load; Average DL-UPT [Mbps]. UEs initially in RRC_IDLE; UEs initially in RRC_CONNECTED. 20; 15%; 135; 202. 30%; 105; 142. 50%; 74; 90. 160; 15%; 85; 202. 30%; 72; 142. 50%; 56; 90. |  |  |

The evaluation results of SSB-less scheme with different periodicity of RO are provided in Figure 3. The ES gain is increased with relaxed RO monitoring occasions and achieves 53%. For the DL-UPT experienced by RRC_IDLE UEs, the observed decrease is justified by longer delays for initial access before DL data transmission.

Figure 3: ES gain and impact on DL-UPT from enabling SSB-less scheme at the capacity cell at low load.

By enabling SIB1-less scheme at the capacity cell, the coverage cell is responsible for providing supplementary SIB1 of capacity cell, where the supplementary SIB1 of capacity cell can be provided either via additional time resources of coverage cell or via additional frequency resources of coverage cell.

The SIB1-less scheme performance is provided in Figure 4. Up to 54% ES gain is observed at the capacity cell (with 15% load). It is at the expense of an increase in the energy consumption at the coverage cell (showing a negative ES gain) if time resources are used to deliver the supplementary SIB1 (relative to the capacity cell), hence additional time spent in Active DL state instead of entering micro sleep state. However, it is expected that the impact is negligeable when the supplementary SIB1 is provided in the frequency domain with additional PRBs usage as the BS would be already in Active DL state, transmitting together with the SIB1 of coverage cell in the same time occasion.

For the DL-UPT experienced by UEs initially in RRC_IDLE state, we see a decrease proportional to the longer time delay introduced by SSB acquisition and RO opportunity.

Figure 4: ES gain and impact on DL-UPT from enabling SIB1-less scheme at the capacity cell at low load

In Figure 5, we evaluate the performance of SSB&SIB1-less scheme. The ES gains and DL-UPT experienced by UEs assumed initially in RRC_IDLE state is shown at different load levels at the capacity cell and with respect to the RO periodicity.

For all load levels, the SSB&SIB1-less scheme provides the higher ES gain as compared to standalone SSB-less and SIB1-less scheme due to a reduced time in Active DL state for SSB/SIB transmission. This is significantly seen at 0% load, where the achieved ES gain is 53% with RO=20ms as shown in i) of Figure-5. And when introducing low traffic with different load in the capacity cell, the ES gain decreases but remains significant achieving around 50% with SSB&SIB1-less scheme. As shown in ii) of Figure-5, when relaxing the RO monitoring occasions from 20ms to 160ms, we observe increased ES gains for all SIB1/SSB adaptation schemes justified by more time spent in micro sleep state for the BS instead of Active UL state.

For the impact on the DL-UPT experienced by UEs initially in RRC_IDLE state, we observe almost the same performance for the three schemes at a given RO periodicity and a given load level. The DL-UPT decreases proportionally to the load level and the RO monitoring delay. However, for UEs in RRC_CONNECTED state, the DL-UPT is only affected by the load levels observed at the capacity cell.

- RO periodicity=20 ms UEs initially assumed in RRC_IDLE state

- RO periodicity=160 ms, UEs initially assumed in RRC_IDLE state

Figure 5: ES gain and DL-UPT for the SSB&SIB1-less schemes

Observation-3: Enabling SSB and/or SIB1 less operation in capacity cell brings ~50% ES gains from reducing the opportunities for the BS to perform DL transmission. The ES gains are increased by further relaxing the RO periodicity at the cost of a decreased DL-UPT experienced by UEs initially in RRC_IDLE/INACTIVE state.

## Techniques in Spatial domain

### Technique #C-1: Dynamic adaptation of spatial element

For the evaluation of Technique #C-1 as defined in [1], we consider a reduced number of Tx from 64 (as per the reference configuration) to 32. The relative power levels are derived using the agreed scaling method (assuming A=0,4 and a single value of η=1).

| NW energy saving scheme | ES gain for each configuration | UPT | Evaluation methodology/baseline assumption | Note |
| --- | --- | --- | --- | --- |
| Reduced number of active Tx | Load; ES gain. 15%; 26,5%. 30%; 27,9%. 50%; 28,2%. | Load; Average DL-UPT [Mbps]. 64 TRx; 32 TRx. 15%; 202,53; 163,26. 30%; 141,59; 117,64. 50%; 90,39; 75,47. | Results are provided as per Set 1-Cat 2 power values. Baseline: Single cell operation as per SET 1. Traffic: DL FTP traffic is assumed at different load levels. UEs: UEs are assumed to be in RRC_CONNECTED state. | Slot-level modeling is assumed. Spatial domain adaptation delay=3 ms. Micro sleep state is assumed during the spatial domain adaptation. - A=0,4 - A single value of η=1. |

Figure 6: ES gain and impact on DL-UPT from reducing the number of spatial element.

The evaluation results of dynamic adaptation of BS’s spatial elements are captured in Figure 6. At all load levels, we observe at around 27% ES gain from reducing the number of transmitters from 64 (as per the baseline for set 1) to 32 TRx. This is due to lower relative power consumed by the BS when being in Active DL state. However, it comes with a cost of around 16% reduction of the average DL-UPT, at different load levels.

Observation-4: the adaptation of spatial elements can bring significant ES gain at the expense of a decrease in the average DL-UPT.

## Techniques in Power domain

### Technique #D-1: Adaptation of transmission power of signals and channels

For the evaluation of Technique #D-1 as defined in [1], we consider different transmission power of signals and channels ranging from 49 dBm (as per Set 2 reference configuration) to 46 dBm, 43 dBm and 40 dBm.

The evaluation results are provided in Figure 7, a significant ES gain is achieved from reducing the total DL transmit power by 3 dBm and further by 6 dBm. The ES gain is increased proportionally to the load levels (i.e., longer time occupied by the BS in Active DL state and hence higher opportunities to apply reduced DL transmit power). At 40 dBm (corresponding to 9 dBm reduction), the ES gain are slightly smaller for the low and light load levels due to degraded DL-UPT, and hence longer time needed by the BS to serve UEs.

| NW energy saving scheme | ES gain for each configuration | UPT | Evaluation methodology/baseline assumption | Note |
| --- | --- | --- | --- | --- |
| Reduced DL transmit power | Load; ES gain vs. 49dBm. 46 dBm; 43 dBm; 40 dBm. 15%; 8%; 10,9%; 9,4%. 30%; 13,6%; 18,2%; 17,3%. 50%; 16,8%; 23,6%; 25%. | Load; Average DL-UPT [Mbps]. 49 dBm; 46 dBm; 43 dBm; 40 dBm. 15%; 154; 144; 134; 119. 30%; 110; 104; 97; 88. 50%; 79; 76; 70; 63. | Results are provided as per Set 2-Cat 2 power values. Baseline: Single cell operation with maximum transmit power of 49 dBm. Traffic: DL FTP traffic is assumed at different load levels. UEs: UEs are assumed in RRC_CONNECTED state. | Slot-level modelling is assumed. - A=0,4 - A single value of η=1. |

Figure 7: ES gain and impact on DL-UPT from reducing the total DL transmit power.

Observation-5: The adaptation of DL transmit power can bring significant ES gain at the expense of a degradation in the average DL-UPT caused by longer transmission times, which can be mitigated by scheduling more PRBs to a UE, whenever possible.

# Conclusions

In this contribution, we provided our simulation results of some energy saving techniques in different domains, and we have the following observations and proposals:

Observation-1: Adapting the transmission periodicity of SSB/SIB1 and/or uplink random access opportunities brings significant ES gains of 84% for 0% load and up to 51% for loaded scenarios at the expense of DL-UPT only for UEs initially in RRC_IDLE state (whereas the network can ensure that there is no impact on UPT for RRC Connected UEs).

Observation-2: Enabling on-demand SSB/SIB1 transmission triggered by the UE’s WUS (instead of legacy SSB/SIB1 transmission and RO monitoring) enables the BS to achieve significant ES gain by reducing the time spent in Active DL. However, on the other hands, it comes with a longer random-access delay perceived by RRC_IDLE UEs that intend to wake up a dormant cell for acquiring the SSB/SIB1 before the UL data transmission.

Observation-3: Enabling SSB and/or SIB1 less operation in capacity cell brings ~50% ES gains from reducing the opportunities for the BS to perform DL transmission. The ES gains are increased by further relaxing the RO periodicity at the cost of a decreased DL-UPT experienced by UEs initially in RRC_IDLE/INACTIVE state.

Observation-4: the adaptation of spatial elements can bring significant ES gain at the expense of a decrease in the average DL-UPT.

Observation-5: The adaptation of DL transmit power can bring significant ES gain at the expense of a degradation in the average DL-UPT caused by longer transmission times, which can be mitigated by scheduling more PRBs to a UE, whenever possible.

# References

- R1-2211098, “Network Energy Saving Techniques”, Nokia, Nokia Shanghai Bell, RAN1#111, Nov. 14th-18th, 2022.

- TR 37.910 V16.1.0, Study on self evaluation towards IMT-2020 submission (Release 17)

- Nokia, IMT-2020 self-evaluation: UP latency analysis for FDD and dynamic TDD with UE processing capability 1 (eMBB), R1-1808448, August 2018.

# Appendix A – System-level simulation assumptions

Table: System-level simulation parameters and assumptions

| Parameters | Set 1 FR1 | Set 2 FR1 |
| --- | --- | --- |
| General parameters |  |  |
| BS type | Macro | Macro |
| Network layout and inter-site distance | 21 cells Wraparound (ISD=500) | 21 cells Wraparound (ISD=500) |
| Channel model | 3D-Uma as in TR 38.901 | 3D-Uma as in TR 38.901 |
| Link direction | Downlink | Downlink |
| Frequency range | 4GHz or 2.6 Ghz | 2100 MHz |
| Duplex | TDD | FDD |
| Frame structure | DDDSU (S: 10D:2G:2U) |  |
| Subcarrier spacing | 30 kHz | 15 kHz |
| Simulation bandwidth | 100 MHz | 20 MHz (Equal split of 10 MHz for UL and DL) |
| Number of carriers | 1 CC | 1 CC |
| Slot size | 14 OFDM symbols | 14 OFDM symbols |
| BS parameters |  |  |
| BS antenna configuration | 64 Tx (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2, 1, 1; 4, 8), (dH, dV) = (0.5λ, 0.8λ) | 32 Tx (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2, 1, 1; 2, 8), (dH, dV)=(0.5, 0.8)λ |
| Total Tx power | 55dBm | 49 dBm |
| BS height [7] | 25m | 25m |
| BS noise figure | FR1: 5 dB | FR1: 5 dB |
| BS antenna element gain | 8 dBi | 8 dBi |
| UE parameters |  |  |
| UE antenna configuration | 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5λ, N/Aλ) | 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5λ, N/Aλ) |
| UE max transmit power [7] | 23 dBm | 23 dBm |
| UE BWP | 100 Mhz | 10 Mhz |
| UE height | 1.5m | 1.5m |
| UE noise figure | FR1: 9 dB | FR1: 9 dB |
| UE antenna element gain | 0 dBi | 0 dBi |
| UE receiver | MMSE-IRC | MMSE-IRC |
| UE deployment | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h |
| Traffic model and C-DRx configuration | Follow previous RAN1 agreements | Follow previous RAN1 agreements |
| UE density/NW Load | Follow previous RAN1 agreements | Follow previous RAN1 agreements |
| Transmission parameters |  |  |
| Maximum supported Modulation and coding scheme | Up to 256QAM | Up to 256QAM |
| Transmission scheme | SU-MIMO | SU-MIMO |
| SU dimension | For 4Rx: Up to 4 layers | For 4Rx: Up to 4 layers |
| Guard band ratio on simulation bandwidth | TDD: 2.08% (272 RB for 30kHz SCS and 100 MHz bandwidth) | FDD: 6.4% (104RB for 15kHz SCS and 20 MHz BW) |
| Channel estimation | Non-Ideal | Non-Ideal |
| DL CSI measurement | Precoded CSI-RS | Precoded CSI-RS |
| DL codebook | Type I codebook | Type I codebook |
| CSI feedback | Subband CQI, 2 RBs per CQI, 2ms period | Subband CQI, 2 RBs per CQI, 2ms period |
| HARQ scheme | Ideal | Ideal |
| Max HARQ retransmission | 3 | 3 |
| Target BLER | 10% of first transmission | 10% of first transmission |
| Power control parameters | Open loop, P0=-80dBm, alpha=0.8 | Open loop, P0=-80dBm, alpha=0.8 |
| Scheduling algorithm | PF | PF |
| Cell selection algorithm | RSRP Slow Fading | RSRP Slow Fading |
| CSI acquisition | Periodic, CQI on 2 ms period | Periodic, CQI on 2 ms period |
| Common Reference Signal (RS) configuration |  |  |
| SSB periodicity | 20 ms | 20 ms |
| SS blocks per SSB burst | Up to 8 | Up to 4 |
| SSB time resource | 4 symbols for each SSB |  |
| SSB frequency resource | 20 RBs | 20 RBs |
| SIB1 transmission repetition periodicity | 20 ms, multiplexing pattern 1 with SSB | 20 ms, multiplexing pattern 1 with SSB |
| SIB1 time resource | 1 slot | 1 slot |
| SIB1 frequency resource | 24 RBs for 20 ms periodicity | 24 RBs for 20 ms periodicity |

# Appendix B - Agreements from RAN1#110bis-e

Agreement: Confirm the previous Working Assumption with the following update - For RAN1 evaluation purpose, for reference configuration set 1/2/3, the values are provided as below. - The transition time is confirmed without update. - FFS: The time unit to be used when calculating the energy consumption Power state; Relative Power P for Category 1; Relative Power P for Category 2. Set 1; Set 2; Set 3; Set 1; Set 2; Set 3. Deep sleep; 1; 1; 1; 1; 1; 1. Light sleep; 25; 23 25; 20 25; 2.1; 2.6 2.1; 1.8 2.1. Micro sleep; 55; 50; 38; 5.5; 5; 3. Active DL; 280; 240 200; 152; 32; 40 26; 8.4 17.6. Active UL; 110; 90; 80; 6.5; 5.8; 4.2. Agreement: For set 1/2/3, the additional energy (unit in relative power*(duration in ms)) is Power state; Additional transition energy Ei. Category 1; Category 2. Deep sleep; 1350 1000; 22500 17000. Light sleep; 90; 1088. Agreement: Capture in TR that, - The BS power model defined in this study is a simplified model for the purposes of evaluations, considering single-RAT NR BSs only. This does not mean a BS cannot benefit from the identified techniques when serving multi-RAT. Transition among power states, transition time, are implementation specific, and different BS types may support a different number of power states with different characteristics, i.e., power consumption values and required transition time. Agreement: - All calculation of energy consumption should use the same time unit (companies to indicate which time unit they used) Agreement - The BS power consumption for active DL transmission is provided by PDL=PstaticDL+PdynamicDL - PstaticDL: a static part of power for BS in active, which is not scaled based on reference configurations. - Baseline: PstaticDL=P3 - Optional: PstaticDL=1.5*P3 - PdynamicDL: a dynamic part of power for BS in active, which is scaled based on reference configuration. - Baseline: PdynamicDL=sa*Pdyn,ante+sf*spηsf, sp*Pdyn,joint, where sa, sf, sp is the fraction of active TRxRUs, the ratio of RF bandwidth and maximum system BW and the ratio of PSD per TxRU between the DL transmission and reference configuration, respectively - &Pdyn,ante= A*(P4-PstaticDL)&Pdyn,joint/ηsf=1, sp=1= (1-A) *(P4-PstaticDL) - Pdyn,joint/ηsf, sp is the power part related to PA. - For simplicity - A = baseline: 0.4; optional: [0.1, 0.7] - For η - If one value of ηsf, sp is used for evaluation, ηsf, sp=1 for any sf, sp - If two values of ηsf, sp are used for evaluation, ηsf, sp = 0.76 if sf* sp<0.5; otherwise, ηsf, sp=1. - Companies to report the assumption used in evaluation. - The BS power consumption for active UL transmission is provided by PUL=PstaticUL+PdynamicUL - PstaticUL: a static part of power for BS in active, which is not scaled based on reference configurations. - PdynamicUL=sa*Pdyn,anteUL: a dynamic part of power for BS in active, which is scaled based on reference configuration and sa is the percentage of active TRxRUs - Baseline - PstaticUL=P3 - Pdyn,anteUL=P5-P3 when no scaling is applied (i.e. scaling factor is 1) - For multi-carrier: the total power consumption of BS is calculated as is the sum of the power consumption of each CC; - for intra-band multi-carrier with contiguous CCs, the power consumption of each additional CC is scaled by [0.7]. - For multi-TRP, the total power consumption of BS is assumed as is the sum of the power consumption of each TRP - Company to report whether Pstatic is shared among TRPs (if shared, Pstatic is accounted once) - Company to additionally report the assumption for antenna adaptation delay, e.g. immediate, with a transition time of [1-3] ms, etc. - In time domain, - The power consumption in a slot is the sum of the power consumption associated with symbols in the slot. The symbol may correspond to uplink symbol, downlink symbol, or symbol without uplink and downlink. - Company to report how the summation is performed along with evaluation results. - Other values for the above scaling formula, and other scaling approaches can be optionally reported, including - PDL=P40.4+0.6*sfspηsf, sp0.4+0.6*sa. At least ηsf, sp= 1 is supported. Additional one or two more values are FFS. - PUL = P5 (0.8+ 0.2 sf) * (0.4+ 0.6*sa). - Sf is the ratio of RF BW to the maximum system BW Agreement: - For FR1 SLS assumptions, add parameters in the below table as additional SLS parameters. Set 1 FR1; Set 2 FR1. 1; Channel model; 3D-Uma as in TR 38.901; 3D-Uma as in TR 38.901. 2; percentage of high loss and low loss building type; 100% low loss; 100% low loss. 3; Guard band ratio on simulation bandwidth; TDD: 2.08% (272 RB for 30kHz SCS and 100 MHz bandwidth); FDD: 6.4% (104RB for 15kHz SCS and 20 MHz BW). 4; HARQ scheme; Ideal; Ideal. 5; Max HARQ retransmission; 3; 3. 6; Target BLER; 10% of first transmission; 10% of first transmission. 7; Power control parameters; Open loop, P0=-80dBm, alpha=0.8; Open loop, P0=-80dBm, alpha=0.8. 10; SS blocks per SSB burst; Up to 8; Up to 4. 11; SSB time resource; 4 symbols for each SSB; 4 symbols for each SSB. 12; SSB frequency resource; 20 RBs; 20 RBs. - For (Set 3) FR2 SLS assumptions, use Table below as baseline assumptions BS type; Micro; UE BWP; 100 Mhz. Network layout and inter-site distance; 21 cells Wraparound (ISD=200m, as agreed); UE height; 1.5m. Channel model; UMi; UE noise figure; 13 dB. Link direction; Downlink; UE antenna element gain; 5 dBi. Frequency range; 30GHz; UE receiver; MMSE-IRC. Duplex; TDD; UE deployment; 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h. Frame structure; DDDSU (S: 10D:2G:2U); Traffic model and C-DRx configuration; follow previous RAN1 agreement. Subcarrier spacing; 120 kHz; UE density/NW Load; Follow previous RAN1 agreements. Simulation bandwidth; 100 MHz; Maximum supported Modulation and coding scheme; Up to 256QAM. Number of carriers; 1 CC; Guard band ratio on simulation bandwidth; 7.8% (64 RB for 120kHz SCS and 100 MHz bandwidth). Slot size; 14 OFDM symbols; Channel estimation; Ideal. BS antenna configuration; 2 TxRU: Baseline: [(M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;1,1); (dH, dV) = (0.5λ, 0.5λ) (dg,H, dg,V) = (2.5λ, 2.5λ) Optional: (M, N, P, Mg, Ng)=(8:16:2:2:2)]; HARQ scheme; Ideal. Total Tx power; 33 dBm, EIRP limited to 63 dBm (as agreed in ref. conf. set 3); Max HARQ retransmission; 3. BS height; 10m; Target BLER; 10% of first transmission. BS noise figure; 7 dB; Power control parameters; Open loop, Alpha=1, P0=-106 dBm. BS antenna element gain; 8 dBi; Scheduling algorithm; PF. UE antenna configuration; 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5λ, N/Aλ); Cell selection algorithm; RSRP Slow Fading. UE max transmit power; 23 dBm; SS blocks per SSB burst; Up to 64. - Other parameters can be optionally reported. - Company can optionally report the actual total DL transmit power allocation for the baseline and the proposed technique, if different from the agreed reference configuration. - For TDD frame structure of e.g. DDDSU, the S slot is assumed as S = 10 DL symbols : 2 Guard symbols :2 UL symbols. - Additionally, for FR1, include the following SLS assumptions as an optional scenario: - BS antenna configuration: 4T - BS Total Tx power: derived based on the scaling methodology - SS blocks per SSB burst: reduced to 1 - Other assumptions are same as those corresponding to Set 2 reference configuration. - Additional transition energy is calculated taken into account the discussion and agreements for additional transition energy for Set 1/2/3 - Company to report the details