3GPP TSG RAN WG1 #111                        	                                             R1- 2211209

Toulouse, France, November 14th – 18th, 2022

Source:           CATT

Title:                Remaining Issues of Evaluation Methodology and Power Model for Network Energy Saving

Agenda Item:  9.7.1

Document for: Discussion and Decision

Introduction

In RAN1#110b-e [1], the network energy savings performance evaluation, including evaluation methodology, base station energy consumption model, KPIs and evaluation results were discussed. Some agreements were made as follows:

Agreement Confirm the previous Working Assumption with the following update - For RAN1 evaluation purpose, for reference configuration set 1/2/3, the values are provided as below. - The transition time is confirmed without update. - FFS: The time unit to be used when calculating the energy consumption Power state; Relative Power P for Category 1; Relative Power P for Category 2. Set 1; Set 2; Set 3; Set 1; Set 2; Set 3. Deep sleep; 1; 1; 1; 1; 1; 1. Light sleep; 25; 23 25; 20 25; 2.1; 2.6 2.1; 1.8 2.1. Micro sleep; 55; 50; 38; 5.5; 5; 3. Active DL; 280; 240 200; 152; 32; 40 26; 8.4 17.6. Active UL; 110; 90; 80; 6.5; 5.8; 4.2. Agreement For set 1/2/3, the additional energy (unit in relative power*(duration in ms)) is Power state; Additional transition energy Ei. Category 1; Category 2. Deep sleep; 1350 1000; 22500 17000. Light sleep; 90; 1088. Agreement Capture in TR that, - The BS power model defined in this study is a simplified model for the purposes of evaluations, considering single-RAT NR BSs only. This does not mean a BS cannot benefit from the identified techniques when serving multi-RAT. Transition among power states, transition time, are implementation specific, and different BS types may support a different number of power states with different characteristics, i.e., power consumption values and required transition time. Agreement - All calculation of energy consumption should use the same time unit (companies to indicate which time unit they used) Agreement - The BS power consumption for active DL transmission is provided by PDL=PstaticDL+PdynamicDL - PstaticDL: a static part of power for BS in active, which is not scaled based on reference configurations. - Baseline: PstaticDL=P3 - Optional: PstaticDL=1.5*P3 - PdynamicDL: a dynamic part of power for BS in active, which is scaled based on reference configuration. - Baseline: PdynamicDL=sa*Pdyn,ante+sf*spηsf, sp*Pdyn,joint, where sa, sf, sp is the fraction of active TRxRUs, the ratio of RF bandwidth and maximum system BW and the ratio of PSD per TxRU between the DL transmission and reference configuration, respectively - &Pdyn,ante= A*(P4-PstaticDL)&Pdyn,joint/ηsf=1, sp=1= (1-A) *(P4-PstaticDL) - Pdyn,joint/ηsf, sp is the power part related to PA. - For simplicity - A = baseline: 0.4; optional: [0.1, 0.7] - For η - If one value of ηsf, sp is used for evaluation, ηsf, sp=1 for any sf, sp - If two values of ηsf, sp are used for evaluation, ηsf, sp = 0.76 if sf* sp<0.5; otherwise, ηsf, sp=1. - Companies to report the assumption used in evaluation. - The BS power consumption for active UL transmission is provided by PUL=PstaticUL+PdynamicUL - PstaticUL: a static part of power for BS in active, which is not scaled based on reference configurations. - PdynamicUL=sa*Pdyn,anteUL: a dynamic part of power for BS in active, which is scaled based on reference configuration and sa is the percentage of active TRxRUs - Baseline - PstaticUL=P3 - Pdyn,anteUL=P5-P3 when no scaling is applied (i.e. scaling factor is 1) - For multi-carrier: the total power consumption of BS is calculated as is the sum of the power consumption of each CC; - for intra-band multi-carrier with contiguous CCs, the power consumption of each additional CC is scaled by [0.7]. - For multi-TRP, the total power consumption of BS is assumed as is the sum of the power consumption of each TRP - Company to report whether Pstatic is shared among TRPs (if shared, Pstatic is accounted once) - Company to additionally report the assumption for antenna adaptation delay, e.g. immediate, with a transition time of [1-3] ms, etc. - In time domain, - The power consumption in a slot is the sum of the power consumption associated with symbols in the slot. The symbol may correspond to uplink symbol, downlink symbol, or symbol without uplink and downlink. - Company to report how the summation is performed along with evaluation results. - Other values for the above scaling formula, and other scaling approaches can be optionally reported, including - PDL=P40.4+0.6*sfspηsf, sp0.4+0.6*sa. At least ηsf, sp= 1 is supported. Additional one or two more values are FFS. - PUL = P5 (0.8+ 0.2 sf) * (0.4+ 0.6*sa). - Sf is the ratio of RF BW to the maximum system BW Agreement - For FR1 SLS assumptions, add parameters in the below table as additional SLS parameters. Set 1 FR1; Set 2 FR1. 1; Channel model; 3D-Uma as in TR 38.901; 3D-Uma as in TR 38.901. 2; percentage of high loss and low loss building type; 100% low loss; 100% low loss. 3; Guard band ratio on simulation bandwidth; TDD: 2.08% (272 RB for 30kHz SCS and 100 MHz bandwidth); FDD: 6.4% (104RB for 15kHz SCS and 20 MHz BW). 4; HARQ scheme; Ideal; Ideal. 5; Max HARQ retransmission; 3; 3. 6; Target BLER; 10% of first transmission; 10% of first transmission. 7; Power control parameters; Open loop, P0=-80dBm, alpha=0.8; Open loop, P0=-80dBm, alpha=0.8. 10; SS blocks per SSB burst; Up to 8; Up to 4. 11; SSB time resource; 4 symbols for each SSB; 4 symbols for each SSB. 12; SSB frequency resource; 20 RBs; 20 RBs. - For (Set 3) FR2 SLS assumptions, use Table below as baseline assumptions BS type; Micro; UE BWP; 100MHz. Network layout and inter-site distance; 21 cells Wraparound (ISD=200m, as agreed); UE height; 1.5m. Channel model; UMi; UE noise figure; 13dB. Link direction; Downlink; UE antenna element gain; 5 dBi. Frequency range; 30GHz; UE receiver; MMSE-IRC. Duplex; TDD; UE deployment; 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h. Frame structure; DDDSU (S: 10D:2G:2U); Traffic model and C-DRx configuration; follow previous RAN1 agreement. Subcarrier spacing; 120 kHz; UE density/NW Load; Follow previous RAN1 agreements. Simulation bandwidth; 100 MHz; Maximum supported Modulation and coding scheme; Up to 256QAM. Number of carriers; 1 CC; Guard band ratio on simulation bandwidth; 7.8% (64 RB for 120kHz SCS and 100 MHz bandwidth). Slot size; 14 OFDM symbols; Channel estimation; Ideal. BS antenna configuration; 2 TxRU: Baseline: [(M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;1,1); (dH, dV) = (0.5λ, 0.5λ) (dg,H, dg,V) = (2.5λ, 2.5λ) Optional: (M, N, P, Mg, Ng)=(8:16:2:2:2)]; HARQ scheme; Ideal. Total Tx power; 33 dBm, EIRP limited to 63 dBm (as agreed in ref. conf. set 3); Max HARQ retransmission; 3. BS height; 10m; Target BLER; 10% of first transmission. BS noise figure; 7 dB; Power control parameters; Open loop, Alpha=1, P0=-106 dBm. BS antenna element gain; 8 dBi; Scheduling algorithm; PF. UE antenna configuration; 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5λ, N/Aλ); Cell selection algorithm; RSRP slow fading. UE max transmit power; 23dBm; SS blocks per SSB burst; Up to 64. - Other parameters can be optionally reported. - Company can optionally report the actual total DL transmit power allocation for the baseline and the proposed technique, if different from the agreed reference configuration. - For TDD frame structure of e.g. DDDSU, the S slot is assumed as S = 10 DL symbols : 2 Guard symbols :2 UL symbols. - Additionally, for FR1, include the following SLS assumptions as an optional scenario: - BS antenna configuration: 4T - BS Total Tx power: derived based on the scaling methodology - SS blocks per SSB burst: reduced to 1 - Other assumptions are same as those corresponding to Set 2 reference configuration. - Additional transition energy is calculated taken into account the discussion and agreements for additional transition energy for Set 1/2/3 - Company to report the details

This contribution discusses the remaining evaluation methodology and power model of NR network energy savings, which include evaluation methodology, and KPIs. At last, the evaluation results are provided.

Discussion

In RAN1#110b-e, most of power model and evaluation assumption/methodology for network energy saving were agreed. It needs to further clarify some issues based on the agreements, which includes:

- Baseline of system configuration for normal network operation for the evaluation of network energy consumption

- Key performance index of network energy saving

This section discuss the above remaining issues.

Remaining issues of Evaluation assumptions

Baseline of system configuration for normal network operation for the evaluation of network energy consumption

In RAN1#110b-e, it is agreed for the evaluation that the network energy saving gain is computed based on the energy consumptions for a technique and the baseline over the same duration.

The baseline assumption of system configuration for normal network operation should be specified in order to obtain the energy consumption of normal network operation and to identify the potential network energy saving technique. The normal network operation includes periodic SSB transmission, periodic RACH resource for initial access and random access procedures, the periodic system broadcast information, and resource allocation and transmission of DL/UL control channels. Furthermore, some network operations, such as MIMO configuration, and carrier configuration for CA/DC should also be specified in order to have the clear scope of network energy consumption for the normal NR network operation.

The baseline network energy consumption should be based on normal network operation to provide services to UE without excessive latency in network access as follows,

- Periodic SSB transmission at each cell

  - Including periodic RACH resource for initial access and random access procedures

- Periodic system broadcast information at a cell

  - Including paging transmission

- Resource allocation and transmission of DL/UL control channels

  - CORESET is located at each slot for UE PDCCH monitoring

  - UL control channel resource is allocated for each slot

    - Periodic SR resource allocation

- Baseline for scenario specific system configurations

  - MIMO

    - Periodic CSI-RS transmission

    - Periodic CSI feedback

  - CA/DC

    - PDCCH and CSI-RS configuration in SCell

We could classify the network energy consumption into two categories, which are network sleeping state and network active state as illustrated in Figure 1and Figure 2.

- Network sleeping state: there is no DL/UL transmission of data and control channels. SSB could be transmitted to enable UE detection of the cell for access. CORESET#0 is configured for RMSI and Paging transmission.

Figure 1: The network operation of network sleep state

- Network active state:  both DL and UL control and data are in active transmission.

Figure 2: The network operation of network active mode

Proposal 1: The baseline configuration and normal network operation should be defined in order to obtain the energy consumption of normal network operation and to identify the potential network energy saving technique.

Key performance index of network energy saving

The key performance index (KPI) is the evaluation reference of the network energy saving technique and the associated impact on the network/UE operation. Considering the impact to legacy UE, the baseline assumption is the condition of normal network operation with network energy saving technique supporting both Rel-18 and beyond UEs and legacy UEs. The network energy saving gain should be based on energy consumption of network energy saving techniques over that of the baseline network operation. Since the evaluation of network energy saving is proposed to be based on system level simulation, the related system performance outputs, such as system throughput and latency, and UE performance, such as UE power consumption, could be captured together.

- Network energy consumption: the network energy consumption adopted relative model and averaged in slot.

- Energy saving gains: the gain of network energy saving scheme to baseline operation.

- System performance and user performance

  - Throughput/SE

  - User perceived throughput/Latency

  - User power consumption

  - System throughput and/or resource utilization/overhead (if applicable) should be reported as the result of the evaluation, in addition to power saving gain.

- If a new signal/channel is introduced, the performance metrics include

  - Performance

  - Complexity

  - Overhead for reception of the signal/channel

- If the new signal is used for detection,  the performance metrics include

  - False alarm rate

  - Miss-detection rate

Proposal 2: The KPI and relative system performance should be captured along with network energy saving gain in the evaluation of network energy consumption.

NES evaluation results

In this section, the evaluation results of network energy saving scheme in time/frequency/spatial/power domain are provided as examples. The details of the simulation results and analysis are shown in our companion contribution [2]. The simulation assumption and network energy models are based on the agreement in [1].

Time domain energy saving results

Technique #A-1: Adaptation of common signals and channels

The network energy saving gain could be achieved by configuring SSB with longer periodicity with SSB beam sweeping. The details of the simulation parameters and analysis are shown in our companion contribution [2]. The evaluation cases are shown as follows:

- Baseline: normal SSB transmission with 20 ms periodicity.

- Case1: Zero system load, SSB and SIB1 with periodicity varied 40ms, 80ms, 160ms.

- Case2: Non-zero system load, SSB and SIB1 with periodicity varied 40ms, 80ms, 160ms, CSI-RS/TRS with periodicity 10ms and FTP3 traffic with different number of users in a cell to characterize the system load.

The network energy saving performance of increasing SSB transmission periodicity scheme is shown in Table 1.

Table 1: Energy saving gain of increasing common control channel periodicity with different system loads

|  | SSB/SIB transmission periodicity | System loads |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Zero | 9%(low) | 15%(light) | 30%(Medium) |
| Energy saving gain | 20ms(baseline) | - | - | - | - |
|  | 40ms | 10.2% | 3.0% | 1.9% | 1.3% |
|  | 80ms | 72.7% | 18.8% | 5.2% | 2.2% |
|  | 160ms | 84.8% | 19.6% | 5.6% | 2.6% |
| Note: a) Set 1 of category 1 power model is applied. b) L=4, i.e., 4 SSB within one SSB burst for FR1. |  |  |  |  |  |

From the results in Table 1, it could be observed that:

- For zero system load with the increasing of common control channel periodicity, it could obtain network energy saving gain from 10.2% to 84.8% with the  common control channel periodicity increasing from 20 ms to 40ms, 80ms, and 160 ms, respectively. Major network energy saving gain could be achieved within the common control channel periodicity of 160ms.

- For non-zero system loads with increase of common control channel periodicity, the network energy saving gain is observed from 1.3% to 19.6% with different common control channel periodicity. Limited energy saving gain is observed for medium system load and higher obvious energy saving gain for low system load.

Observation 1: The network energy saving technique of adaptation of common control channel periodicity (Technique#A-1) in the range of 40, 80, and 160 ms provides the energy saving gain of 10.2% to 84.8%, for zero system load, 3.0%-19.6% for low system load, 1.9%-5.6% for light system load, and 1.3-2.6% for medium system load, which the system load is derived from different number of users in FTP3 traffic model.

Technique #A-4: Adaptation of DTX/DRX

gNB could reduce the energy consumption with the DTX transmission in low system load state by configuring  periodicity of DTX cycle same as that  of C-DRX, which has same configuration for all UEs. The DTX starting point of each cycle is offset before that of the UE DRX to allow UE preparation of coherent detection at DRX ON. The DTX ON duration is longer than that of the UE DRX to allow UE preparation and extend the processing time before and after DRX ON. The details of gNB DTX/DRX schemes are shown in our companion contribution [2].

- Baseline: The gNB is always on to provide service to the UE.

- gNB DTX/DRX scheme: The gNB DTX aligns with the UE DRX cycles, in which same on-durations of UE DRX are configured for all UEs.

The evaluation results are provided in Table 2.

Table 2: Energy saving gain (ESG) of the gNB DTX transmission under different system loads

|  | System load = 9% | System load = 15% | System load = 30% |
| --- | --- | --- | --- |
| Average ESG of DTX | 71.3% | 62.6% | 47.8% |

From the evaluation results in Table 2, it could be observed that,

- gNB DTX network energy saving scheme with fully aligned DRX-ON configuration for all UEs can achieve 47.8%~71.3% energy saving gain under the different system loads. The energy saving gain is due to the gNB getting in DTX OFF that gNB could enter deep sleep.

Observation 2: The energy saving techniques of adaptation of DTX/DRX (Technique #A-4) can obtain 47.8% -71.3% energy saving gain for low, light and median system loads.

Technique #A-6: Adaptation of SSB/SIB1

The system level evaluation of semi-static/dynamic cell ON/OFF scheme with detail assumptions and analysis of cell ON/OFF network energy scheme in our companion document [2].

- Baseline: Cell is always ON.

- Case1: Cell OFF when cell has no any served UE.

- Case2: Cell OFF when cell has no any served UE. And the Cell with ON adapts CELL DTX when there is no data transmission.

The evaluation results are provided in Table 3.

Table 3: Energy saving gain (ESG) of semi-static/dynamic cell ON/OFF

|  | Percentage of Cell ON | Network Energy Saving gain of Cell ON/OFF scheme |
| --- | --- | --- |
| Cell without DTX | 65.5% | 22.0% |
| Cell with DTX | 65.5% | 43.6% |

From the evaluation results in Table 3, it could be observed that:

- The simulation assumption is based on the periodic transmission of 20ms SSB/SIB and 10ms CSI-RS/TRS.   The dynamic cell ON/OFF network energy saving scheme by neighboring serving cell could obtain 22.0% network energy saving gain. With additional gNB DTX scheme during cell ON, 43.6% network energy saving gain can be achieved for semi-static/dynamic cell ON/OFF scheme triggered by neighbour cell.

Observation 3: The network energy saving technique of adaptation of SSB (Technique#A-6) with semi static cell ON/OFF provides the energy saving gain of 22.0%-43.6% with and without the additional cell DTX, respectively, during cell ON, which the low system load assumes 1 UE per cell in FTP3 traffic model.

Frequency domain energy saving results

Technique #B-1: Multi-carrier energy savings enhancements

The system level evaluation is performed for SCells activation/deactivation enhancement and the details are shown in our companion contribution [2]. The evaluation results are provided in Table 4.

- Baseline: Legacy SCells activation/deactivation with normal SSB and Rel-17 temporary TRS transmission.

- SCells activation/deactivation enhancement: SCells RF turning off operating with triggering of SCell activation time in advanced for acquisition of additional RS for AGC/channel tracking.

Table 4: The energy saving gain of SCell activation/deactivation enhancement

| The evaluation scheme | System load = 8% | System load = 16% | System load = 32% |
| --- | --- | --- | --- |
| Average ESG of SCell activation/deactivation enhancement | 30.3% | 29.1% | 20.3% |

From the Table 4, it could be observed that:

- The gNB could obtain 20.3% ~ 30.3% network energy saving gain with different system loads.

- There is no UPT/latency performance loss for SCells activation/deactivation enhancement scheme in the evaluation because triggering of SCell activation based on arrival time of high layer packets is  advance in time before scheduled PDSCH transmission on the SCell to allow UE synchronization and measurements before UE is scheduled for reception/transmission at SCells.

Observation 4: The network energy saving technique of SCell activation/deactivation enhancement technique (Technique #B-1) in frequency domain provides the network energy saving gain of 20.3%~30.3% under low, light and medium system loads, which the system load is derived from different number of users in  FTP3 traffic model.

Spatial domain energy saving results

Technique #C-1: Dynamic adaptation of spatial elements

Most of network energy consumed at gNB is in AAU. Network could obtain energy saving gain by dynamic adaptation the number of TxRU for AAU. When the system load is light with UE having good radio  channel condition, gNB could adjust the number of TxRUs from 64TxRUs to 32TxRUs to reduce gNB energy consumption. The details are shown in our companion contribution [2]. The evaluated baseline and dynamic TxRU adaptation scheme are listed as below.

- Baseline: gNB with 64 TxRUs

- Dynamic TxRU adaptation scheme: gNB dynamic TxRU adaptation from 64 TxRUs to 32 TxRUs

The evaluation results of the gNB with TxRU dynamic on/off under the different system loads are shown in Table 5.

Table 5: The energy saving gain (ESG) of the gNB with TxRU dynamic adaptation under different system loads

| System load | Average ESG | Average UPT loss | Average latency increase |
| --- | --- | --- | --- |
| 16% | 12.30% | 1.60% | 2.50% |
| 32% | 19.60% | 1.80% | 2.60% |
| 46% | 25.40% | 3.20% | 5.30% |

From the evaluation results in Table 5, it could be observed that:

- gNB could obtain 12.3% ~ 25.4% energy saving gain with  1.6%~3.2% UPT loss and 2.5%~5.3% latency increase  by TxRU dynamic on/off.

Observation 5: The network energy saving technique of Adaptation of Dynamic antenna adaptation scheme (Technique#C-1) provides 12.3%-25.4% network energy saving gain with 1.6%-3.2% UPT loss and 2.5%~ 5.3% latency loss, under the different system loads, which the system load is derived from different number of users in  FTP3 traffic model.

Technique #C-2: TRP muting/adaptation in multi-TRP operation

The system level evaluation is performed for dynamic TRP adaptation scheme with the details shown in our companion contribution [2]. The evaluation results are provided in Table 6.

- Baseline: Two TRPs are always ON.

- Dynamic ON/OFF of multi-TRP: If one TRP is turn off, 160ms CSI-RS transmission is applied for this TRP.

Table 6: The energy saving gain of the TRP muting/adaptation in m-TRP operation under the different system loads

| The evaluation scheme | System load = 8% | System load = 16% | System load = 32% |
| --- | --- | --- | --- |
| Average ESG of TRP muting/adaptation in m-TRP operation | 28.4% | 28.7% | 19.7% |
| UE ESG of TRP muting/adaption in m-TRP operation | 12.7% | 12.7% | 12.4% |

From the Table 6, it could be observed that:

- For low/medium system loads, dynamic multi-TRP adaptation scheme could obtain 19.7%~28.7% network energy saving gain without UPT/latency loss.

- Considering the impact on UE power consumption, TRP muting/adaption in m-TRP operation could obtain 12.4%~12.7% UE power saving gain due to PDCCH monitoring reduction from the muting TRP.

Observation 6: The network energy saving technique of TRP muting/adaptation in multi-TRP operation (Technique#C-2) provides 19.7%-28.7% network energy saving gain without UPT/latency loss for low, light and medium system loads, which the system load is derived from different number of users in FTP3 traffic model. The TRP muting adaptation network energy saving also benefits the UE saving in achieving the power saving gain around 12.5% by reducing the PDCCH monitoring of muting TRP.

Power domain energy saving results

Technique #D-1: Adaptation of transmission power of signals and channels

The system level evaluation is performed for power adaptation and the details are shown in our companion contribution [2]. The evaluation result of network energy saving for dynamic power adaptation is shown in Table 7.

- Baseline: Cell without any NES technique.

- Power adaptation: Cell could adapt its transmission power.

Table 7: The energy saving gain of adaption of transmission power of PDSCH

| The evaluation scheme | System load = 9% | System load = 18% | System load = 23% |
| --- | --- | --- | --- |
| Average ESG of power adaptation | 3.9% | 7.7% | 9.2% |

From the Table 7, it could be observed that:

- The gNB could obtain 3.9% ~ 9.2% network energy saving gain with different system loads by dynamic adaptation of transmission power of PDSCH.

Observation 7: The network energy saving technique of Adaptation of transmission power of signals and channels (Technique #D-1) provides 3.9%-9.2% network energy saving gain for different system loads, which the system load is derived from different number of users in FTP3 traffic model.

Conclusion

In this contribution, it discussed the evaluation methodology and provided the results. We have the following observations and proposals,

Proposal 1: The baseline configuration and normal network operation should be defined in order to obtain the energy consumption of normal network operation and to identify the potential network energy saving technique.

Proposal 2: The KPI and relative system performance should be captured along with network energy saving gain in the evaluation of network energy consumption.

Observation 1: The network energy saving technique of adaptation of common control channel periodicity (Technique#A-1) in the range of 40, 80, and 160 ms provides the energy saving gain of 10.2% to 84.8%, for zero system load, 3.0%-19.6% for low system load, 1.9%-5.6% for light system load, and 1.3-2.6% for medium system load, which the system load is derived from different number of users in FTP3 traffic model.

Observation 2: The energy saving techniques of adaptation of DTX/DRX (Technique #A-4) can obtain 47.8% -71.3% energy saving gain for low, light and median system loads.

Observation 3: The network energy saving technique of adaptation of SSB (Technique#A-6) with semi static cell ON/OFF provides the energy saving gain of 22.0%-43.6% with and without the additional cell DTX, respectively, during cell ON, which the low system load assumes 1 UE per cell in FTP3 traffic model.

Observation 4: The network energy saving technique of SCell activation/deactivation enhancement technique (Technique #B-1) in frequency domain provides the network energy saving gain of 20.3%~30.3% under low, light and medium system loads, which the system load is derived from different number of users in  FTP3 traffic model.

Observation 5: The network energy saving technique of Adaptation of Dynamic antenna adaptation scheme (Technique#C-1) provides 12.3%-25.4% network energy saving gain with 1.6%-3.2% UPT loss and 2.5%~ 5.3% latency loss, under the different system loads, which the system load is derived from different number of users in  FTP3 traffic model.

Observation 6: The network energy saving technique of TRP muting/adaptation in multi-TRP operation (Technique#C-2) provides 19.7%-28.7% network energy saving gain without UPT/latency loss for low, light and medium system loads, which the system load is derived from different number of users in FTP3 traffic model. The TRP muting adaptation network energy saving also benefits the UE saving in achieving the power saving gain around 12.5 by reducing the PDCCH monitoring of muting TRP.

Observation 7: The network energy saving technique of Adaptation of transmission power of signals and channels (Technique #D-1) provides 3.9%-9.2% network energy saving gain for different system loads, which the system load is derived from different number of users in FTP3 traffic model.

References

- RAN1 chairman notes, 3GPP TSG RAN WG1 #110b-e, October 10th – 19th, 2022.

- R1-2211210, Network Energy Saving techniques in time, frequency, and spatial domain, CATT, RAN1#111, November 14th – 18th, 2022.