3GPP TSG RAN WG1 #111			R1-2212015

Toulouse, France, November 14th – 18th, 2022

3GPP TSG-RAN WG2 Meeting #119bis 	R2-2211028

Electronic October 2022

Title:	Draft reply LS on further further questions on feMIMO RRC parameters

Response to:	R2-2211028

Release:	Rel-17

Work Item:	NR_feMIMO-Core

Source:	RAN1

To:	RAN2

CC:	-

Contact Person:

#### Name:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: 	-

# Overall description

RAN2 asked RAN1 for some questions on Rel-17 FeMIMO RRC parameters as follows. Therefore, RAN1 would like to provide corresponding answers.

BWP and cell ID in unified TCI state for SRS

RAN2 discussed the need for BWP ID and cell ID for unified TCI state for SRS.

In SRS-config, SRS-Resource is configured with the unified TCI states:

srs-TCIState-r17                        CHOICE {

srs-UL-TCIState-r17                     TCI-UL-State-Id-r17,

srs-DLorJoint-TCIState-r17              TCI-StateId

}                                                                                                      OPTIONAL    -- Need R

]]

RAN2 is wondering whether there should be serving cell and BWP ID explicitly associated with the present UL TCI state or DL/Joint TCI state of SRS.

For the case that the UL TCI state is present, RAN2 understands there are two alternatives associating the BWP and serving cell with the UL TCI state:

- Add the separate fields BWP ID and Cell ID in srs-UL-TCIState-r17

- Clarify in the field description of srs-TCIState-r17 that the present UL-TCI-State is associated with the BWP and serving cell where the SRS-Config is configured.

For the case that the Joint TCI state is present, RAN2 understands there are also two alternatives associating the BWP and serving cell with the Joint TCI state:

- Add the separate fields BWP ID and Cell ID in srs-DlorJoint-TCIState-r17

- Clarify in the field description of srs-TCIState-r17 that the Joint TCI state is associated with the serving cell where the SRS-Config is configured and the current active DL BWP of this serving cell.

Question 1

According to above RAN2 understandings, RAN2 would like to ask RAN1 to explain what the association of BWP and cell ID for srs-TCIState is.

Answer 1

RAN1 discussed how the BWP ID and the Cell ID are determined for the UL TCI state and the Joint TCI state for SRS. The UL TCI state is provided by ul-TCI-StateList-r17 provided by BWP-UplinkDedicated of the BWP and serving cell where the SRS-Config is configured. The Joint TCI state is provided by dl-OrJoint-TCIStateList-r17 provided by PDSCH-Config of the serving cell where the SRS-Config is configured and the current active DL BWP of this serving cell.

codebookmode for codebookConfig

RAN2 also discussed the need of codebookMode for CodebookConfig-r17.

Question 2

RAN2 would like to ask RAN1

- Is there a need to add the parameter codebookMode which selects mode1 or mode2 in the IE CodebookConfig-r17?

Answer 2a)

Yes, RAN1 would like to confirm that the parameter codebookMode shall be added in the IE CodebookConfig-r17.

- If a) is needed, which types of codebook the parameter is needed, for example:

  - typeI-SinglePanel-Group1-r17

  - typeI-SinglePanel-Group2-r17

  - typeII-PortSelection-r17

Answer 2b)

Among above, codebookMode is needed for case of a (typeI-SinglePanel-Group1-r17) and b (typeI-SinglePanel-Group2-r17).

- Additionally, if a) is needed, for each type of codebook that needs a parameter codebookMode, please confirm if a separate parameter is needed or if they share the same configuration of codebookMode.

Answer 2c)

RAN1 already discussed this issue and concluded that one common parameter, codebookMode, is enough for case of a (typeI-SinglePanel-Group1-r17) and b (typeI-SinglePanel-Group2-r17).

For configuring pathloss reference signal for unified TCI state

RAN2 would like to inform RAN1 of how pathloss reference RS is configured for unified TCI state operation. In each BWP-UplinkDedicated UE may be configured with up to 64 pathloss reference RS by field pathlossReferenceRSToAddModList. Each joint or UL TCI state may be linked to one pathloss reference RS from this list. There is further MAC CE and DCI operation to indicate UE which pathloss reference RSs to follow.

Question 3

RAN2 assumes RAN1 informs RAN2 in case the above described pathloss reference signal configuration for unified TCI state is not what RAN1 has intended.

Answer 3

The pathloss reference signal (PLRS) is provided by the indicated TCI state by pathReferenceRS-Id-r17. Other than indication of the unified TCI state by MAC CE or DCI signalling, there is no additional signalling for the PLRS.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account in their further work

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #112 	27 February – 3 March, 2023		Greece

TSG-RAN WG1 Meeting #112b-e 	17 – 26, April, 2023				Online