3GPP TSG-RAN WG1 Meeting #111	R1-2212043

Toulouse, France, November 14th – 18th, 2022

Agenda Item:	9.3.2

Source:	Samsung

Title:	SBFD feasibility and design considerations for NR duplex evolution

Document for:	Discussion

Introduction

The SID for NR duplex evolution aims to identify solutions and evaluate their feasibility to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation in unpaired spectrum [1].

In the Rel-18 study item, it is assumed that the duplex enhancements are deployed at the gNB side and that the TDD UE operates in half-duplex. Subband full-duplex (SBFD) operation in a time-domain resource is limited to FDM, e.g., using non-overlapping subband allocations for simultaneous DL transmissions to UEs and for UL receptions from UEs respectively. Note that in the remainder of this contribution we will simply refer to SBFD for subband full-duplex operation using non-overlapping subband allocations, e.g., as assumed by the Rel-18 SID.

In Section 2 we discuss backwards compatibility and benefits of using gNB-side SBFD when legacy UEs are present in the NR TDD cell, e.g., SBFD transparent mode. In Sections 3 and 4, we discuss practical SBFD design requirements, SBFD configuration and indication and SBFD scheduling aspects when introducing simultaneous transmission and reception capability at the gNB for the NR TDD bands. In Section 5, we consider several UE-side enhancements to provide support to gNB-side SBFD operation in later NR releases.

Section 6 shows and discusses findings based on Samsung FR1 and FR2-1 testbed results to demonstrate that SBFD operation is feasible and viable for commercial NR network implementations. Section 6 also discusses aspects related to gNB power consumption during SBFD operation.

SBFD and legacy UEs

When SBFD operation is supported by the gNB, backwards compatibility for legacy UEs must be assured. UE backwards compatibility must be considered from the perspective of both functional and operational backwards compatibility.

Functional backwards compatibility implies that legacy Rel-15 to Rel-17 UEs can still connect to and establish service in the NR TDD cell. Transmission and reception of data and RRC/NAS signaling should still be possible without undue performance impact. In addition, legacy UEs can potentially benefit from the availability of SBFD in an NR TDD cell, e.g., when DFFFU is configured in the cell. Operational backwards compatibility means that legacy UEs should still be able to use existing deployed NR network features such as NR CA, EN-DC, or NR-DC when SBFD operation is introduced in a serving cell.

Legacy UEs in SA mode when operating on a TDD cell with SBFD at the gNB can access the TDD cell as their serving cell. These UEs must be able to acquire the TDD cell with SBFD during initial cell-selection, complete initial network access in the TDD cell with SBFD and later perform cell re-selection in RRC_IDLE or RRC_INACTIVE when camped on the TDD cell.

Using n78, channel BW = 100 MHz, SCS = 30kHz and DXXSU as example in Figure 1 where gNB-side SBFD operation is enabled in the 2nd and 3rd slot, the gNB can configure SIB1 for the legacy UEs with tdd-UL-DL-ConfigCommon as {dl-UL-TransmissionPeriodicity, nrofDownlinkSlots, nrofDownlinkSymbols, nrofUplinkSlots, nrofUplinkSymbols} = {P = 2.5ms, 3 DL slots, 12 DL sym, 0 UL sym, 1 UL slot}, or DDDSU. Legacy NR UEs then consider only the last 2 symbols in slot #3 as flexible symbols. This disables the possibility to schedule any UL transmissions for legacy UEs using the SBFD UL subband when SBFD operation is supported in slots #1 and #2.

When SIB1 tdd-UL-DL-ConfigCommon for the legacy UEs is configured with {dl-UL-TransmissionPeriodicity, nrofDownlinkSlots, nrofDownlinkSymbols, nrofUplinkSlots, nrofUplinkSymbols} = {P = 2.5ms, 1 DL slot, 0 DL sym, 0 UL sym, 1 UL slot}, or DFFFU, the gNB can in principle schedule UL transmissions to legacy UEs using the SBFD UL subband in flexible symbols of slots #1 and #2 as shown in Figure 8. The UL BWPs of the legacy UEs must be configured inside the BS channel BW such that they contain the SBFD UL subband.

Note that we assume here that no UE-dedicated, e.g., no UE-specific RRC UL-DL frame configuration is provided to the UEs by RRC. Similarly, SFI, e.g., DCI format 2_0 is not assumed as configured. This corresponds to typical NR TDD operation where FG 5-1 support is mandatory for the UE, but FG 5-1a (UE-specific RRC UL-DL frame configuration) and FG 3-6 (SFI) are optional UE features (and not usually implemented).

Figure 1: SBFD operation for legacy UEs without specification impact, e.g., transparent mode

Unfortunately, as was already observed by several companies in RAN1#109-e, different Rel-15 UE modem chipsets implement inconsistent behavior when following gNB scheduling assignments for the semi-static symbols/slots configured as F. It is therefore not a safe assumption to make that SBFD using transparent mode can always be used for scheduling legacy UEs.

When DFFFU is configured and if the legacy UE scheduling behavior for the F slots is implemented as required by core specifications, the gNB would schedule UL transmissions to legacy UEs using the SBFD UL subband of slots #1 and #2, e.g., using PUSCH resource allocation type 1. Limitations would exist with respect to CSI-RS resource configurations when configured in these slots. UL scheduling by the gNB cancels UE reception for such RRC configured DL signals. Similar considerations apply to RRC configured UL signals/channels. Reception of a scheduling DCI by the UE will cancel the configured UL transmission and the UE transmits or receives the scheduled signal/channel. The Rel-15 UE can be configured with multiple open loop power-control parameter sets when scheduled for UL transmissions in SBFD slots and the normal UL slot(s).

- Observation 1: Backwards-compatibility for legacy UEs when SBFD is configured in the TDD cell can be achieved by using DDDSU in SIB1 tdd-UL-DL-ConfigCommon

- Observation 2: It cannot be assumed that SBFD using transparent mode when configuring DFFFU in SIB1 tdd-UL-DL-ConfigCommon and scheduling/configuring DL/UL transmissions for legacy UEs in the SBFD UL subband results in consistent UE behavior

When SBFD is supported in a TDD serving cell of an NR band, legacy NR UEs can use intra-band CA, inter-band CA, FR1-FR2 CA, EN-DC or NR-DC. The TDD cell with SBFD must be able to operate as PCell, SCell or SpCell for the legacy UEs.

Using the example of FR1-FR1 inter-band CA in Figure 2, it can be expected that the mid-band TDD cell with SBFD will often be configured as SCell. Some relevant deployment cases are n3 + n78 or n41+ n78. The n78 (3.5 GHz) TDD cell with SBFD can use up to 100 MHz channel BW. The n3 (1.7 GHz) FDD carrier or the n41 (2.5 GHz) TDD carrier provide the coverage layer and are configured as PCell for the legacy UEs. Note that simultaneous Tx/Rx capability is mandated for the UE for n3 + n78, but not for n41 + n78. In this case, the PCell is in a different band and the TDD SCell with SBFD will then carry DL data. The TDD SCell with SBFD will then mostly be used for DL data, so the SCell UL does not benefit from the availability of SBFD.

Using the example of FR1-FR2 CA, it is possible that the mid-band TDD cell with SBFD is configured as PCell. One relevant case is n78 + n257. For example, the n257 (28 GHz) TDD cell is then configured with 200 MHz and 120 kHz. Note that simultaneous Tx/Rx capability is mandatory for the UE for this combination (like for any FR1-FR2 combination). The FR1 TDD PCell with SBFD carries DL & UL control (L1, RRC, NAS) for both CCs. The FR1 TDD PCell with SBFD may carry both DL and UL data, so improved UL performance in the PCell with SBFD is beneficial.

Figure 2: SBFD on SCell for FR1 inter-band CA (n41 + n78)

Examples for EN-DC or NR-DC are inter-band EN-DC within FR1, e.g., DC_1A_n78A or FR1-FR2 inter-band DC_n78A-n257A using 2 bands. Note that UL data is then present in both cases on the n78 TDD cell with SBFD, so any benefits or penalties incurred from introduction of SBFD apply.

Deployment of SBFD in a network region of an NR band by the operator should not prevent the operator from continuing to serve the legacy UEs in that band to avoid the need for segregation of the legacy and later release UEs through frequency layers in the deployment. Therefore, it is necessary to consider the use and impact of the SBFD UL subband when the TDD cell with SBFD is configured as PCell, SCell or SpCell using CA or DC. From the point-of-view of functional backwards-compatibility for the legacy UEs, it is also necessary to continue to support the case that the TDD cell with SBFD in band is configured as PCell, SCell or SpCell. In terms of operational backwards-compatibility, it is important to continue to support the case that the later release UEs providing improved support for SBFD at the gNB can still use CA or DC “as is” without changes to the operator’s deployment grid.

- Proposal 1: RAN1 should evaluate the potential impacts from SBFD when a TDD serving cell using SBFD is configured as PCell, SCell or SpCell for a legacy UE

SBFD design principles

When SBFD is supported by the gNB in the NR TDD cell, 2 different approaches for the network to configure the UE are possible. The simplest approach is that the gNB configures the SBFD UL subband and the DL subband(s) for the UE inside the UE channel BW, e.g., using single-carrier operation with SBFD in the serving cell as shown in Figure 3. An additional possibility is to configure the SBFD UL subband for the UE by means of a SUL configuration. Another approach is the use of intra-band CA for SBFD as shown in Figure 4. The latter approach is of particular interest when SBFD is deployed in FR2-1.

Figure 3: SBFD using single carrier in FR1

Figure 4: SBFD using intra-band contiguous CA in FR2-1

Using intra-band TDD carrier aggregation for SBFD in FR2-1, either 3 or 4 contiguous CCs can be configured for the UE in the aggregated BS channel BW. With 3 CC contiguous intra-band TDD CA, the SBFD UL subband can be placed in the center of the aggregated transmission BW configuration. Using 3-CC intra-band TDD CA in FR2-1, the SBFD UL subband is configured with an UL-heavy UL-DL frame configuration in one of the CCs and the DL subband(s) are configured for the 2 remaining CCs. When intra-band TDD CA is configured for the UE, it is in principle possible to configure the DL BWPs corresponding to the DL subbands independently from the size of the SBFD UL subband, use features such as R16 SCell Dormancy for reception in the DL subbands, or configure the CCs with unaligned frame boundaries and the slot offset. Note that the use of FR1 intra-band CA for purpose of enabling SBFD operation in the NR band is significantly more challenging than using a single NR carrier for SBFD operation in FR1.

When using intra-band TDD CA for SBFD in FR2-1, more flexibility to introduce SBFD operation exists when compared to FR1. The support of NR FR2-1 CA with Independent Beam Management (IBM) and Common Beam Management (CBM) with simultaneous DL reception on different CCs from the co-located and non-collocated TRPs has been introduced in Rel-16 and Rel-17. IBM implies that a UE is capable of DL simultaneous reception on different UE panels/chains using separate beams on different CCs and requires improved UE BB and RF capabilities, e.g., multiple BB chains and support of multiple antenna panels. Rel-18 FR2 enhancements work has started and will introduce requirements for FR2-1 UEs capable of multi-beam/multiple Rx chain simultaneous DL reception on a single CC to achieve improved RF, RRM and UE demodulation performance.

In our view, while the focus of the Rel-18 SID Duplexing enhancements is on SBFD operation using a single NR carrier, feasibility aspects and potential enhancements for SBFD operation using intra-band contiguous CA in FR2-1 should also be investigated at a later stage of the Rel-18 SID. One area for such potential enhancements in FR2-1 is the Rel-16 directional collision handling.

- Proposal 2: Further study potential enhancements to CA-based SBFD operation in FR2-1

Using a single NR RF carrier, SBFD implements full-duplex transmission at the gNB in the BS channel bandwidth. In the SBFD slot, the gNB can schedule DL and UL resources to UEs using non-overlapping frequency-domain resources. The TDD UEs operate in half-duplex.

When a UE is scheduled for DL reception in a slot, no simultaneous UL transmission from the same UE can occur using the same time-domain resources. The gNB must properly account for Rx-Tx and Tx-Rx switching times of the UE when scheduling DL and UL for the UE in different slots. UL transmission from a UE in an SBFD slot occur in the configured SBFD UL subband. DL transmissions to UEs in the SBFD slot occur in either one or both of the SBFD DL subband(s) at higher and lower range of the BS channel BW using ‘DUD’ as shown in Figure 5.

Figure 5: SBFD configuration and scheduling using ‘DUD’

In frequency-domain, it can be expected that the SBFD UL subband is often placed at the center of the BS channel BW, e.g., ‘DUD’, which provides most frequency separation and isolation towards the adjacent channels in band segments owned by other TDD operator(s). In cases where an operator holds a band segment at the edge of the NR band, placing the SBFD UL subband at the edge of the BS channel BW, e.g., ‘DU’ or ‘UD’, is an alternative. For the cases of ‘DU’ or ‘UD’, there will then be only 1 DL subband configured by the gNB at the upper or lower edge of the BS channel bandwidth.

The size of the SBFD UL subband cannot be assumed fixed. At least for FR1 evaluation purposes, it may be assumed that the operator owns a contiguous 100 MHz band segment in n78 and that the SBFD UL subband is 20-25 MHz wide. However, many operators own n78 band segments of smaller size, e.g., 40 or 60 MHz. Some operators may even hold only 2 disjoint n78 band segments. This makes it necessary to also account for the need to configure an SBFD UL subband of smaller size than 20 MHz.

In time-domain, not all symbols and slots can be used for SBFD operation. Particular consideration must be given to DL symbols carrying the SSB(s), configured for CORESET#0, scheduling the PDSCH carrying SIB1 and system information messages > 1 in general, and DL symbols configured for the TRS.

With respect to SBFD operation in SSB(s), the following RAN1#110bis-e agreement was taken.

Agreement

Study whether SBFD operation in SSB symbols is supported or not.

Agreement

Study whether SBFD operation in SSB symbols is supported or not.

The SSB(s) determine the DL cell coverage using the common DL beams in the deployment grid. Legacy Rel-15 UEs are allowed to use EPRE assumptions for the DL power relationships when measuring SSS and DMRS on PBCH symbols during cell (re-)selection. If SBFD operation in SSB symbols is allowed, depending on the antenna configuration selected to support Duplexing in the serving cell, the SSB power allocation per RE is reduced, e.g., by 3 dB when only half of the DL TRX are available for DL transmissions in the DL subband containing the SSB(s). DL EPRE can in principle be compensated for by the gNB, but limits exist with respect to the Rel-15 UE-side EPRE assumptions when implementing SS-RSRP or SS-RSRQ measurements. Any change to the DL link budget dimensioning determining DL coverage of the cell assumed by the operator for the deployment grid should be avoided.

In our view, it should therefore not be expected that the DL symbols configured and indicated for SSB(s) can also be configured for UL transmissions using the SBFD UL subband due to legacy UE backwards compatibility considerations.

More flexibility to configure SBFD operation exists for DL symbols which are configured for the CORESET#0 or used for scheduling PDSCH carrying SIB1 and SI>1. The CORESET#0 bandwidth, e.g., the initial DL BWP, can be set to 24, 48 or 96 RBs and more than 1 time-domain symbol can be configured for the CORESET#0 allocation. DL link adaptation for PDSCH carrying system information can be adjusted by FDRA, TDRA and MCS settings to account for a potentially reduced number of available DL TRX and different DL beam settings if needed.

For TRS, the possible symbol locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots) are restricted for FR1 and FR2-1. Not being able to configure the SBFD UL subband on the symbols configured by trs-Info also means that the 3-symbol gap between a pair of CSI-RS resources and for most symbol pair configuration options, also the gap between the two pairs of CSI-RS resources cannot be scheduled using SBFD. For FR1 n78, SCS = 30 kHz, Xp = 40 msec, at least 18 symbols then become not-schedulable for SBFD transmissions using the SBFD UL SB, which corresponds to a loss of 6% of available time-domain resources. Note that the bandwidth of a CSI-RS resource configured as TRS is at least 52 RBs.

- Proposal 3: SBFD operation in SSB symbols is not supported.

- Proposal 4: Further study whether SBFD operation in DL symbols configured for CORESET#0 or scheduled for PDSCH carrying SI is supported or not.

- Proposal 5: Further study whether SBFD operation in DL symbols configured for TRS is supported or not.

SBFD configuration and scheduling

It was agreed in RAN1#109-e that the time and frequency location of subbands within a TDD carrier are not fixed in the specification.

In RAN1#110bis-e, it was agreed that SBFD operation Alt.4 is baseline. It was also agreed that for semi-static configuration of the subband frequency locations for SBFD operation, at least the explicit indication of frequency location of the UL subband is required. It was left FFS if frequency location(s) of other subbands types are explicitly indicated or implicitly determined, e.g., for the DL subband(s) or guard band(s). It was also agreed that for semi-static configuration of the subband time locations for SBFD operation, explicit configuration of SBFD subband time locations within a period is the baseline. Furthermore, two proposed FL agreements were discussed with respect to configuration and UE scheduling in slots/symbols semi-statically configured as DL and F, respectively, e.g., FL proposals 1-18 and 1-20.

In this section, provided our views on the need to explicitly configure the DL subband(s), the need and signaling details for guard band configurations, DL scheduling in the configured SBFD UL subband and the need (or not) for potentially different behavior when using SBFD operation in legacy DL vs. F symbols/slots.

Guard bands and DL subband configuration

Guard band(s) between the SBFD DL subband(s) and the UL subband are required by the gNB to avoid inter-subband CLI and to control DL-UL self-interference levels for many expected gNB-side SIC implementations. In Section 6 we show that time-domain digital SIC for example reduces the number of guard RBs. SBFD operation assuming the possibility for the gNB to use a number of guard band RBs is therefore very meaningful, but the size of the guard band must remain configurable because it is a function of the gNB SIC implementation.

Fixed size guard bands, e.g., as a function of the RF band combination for a given SBFD UL subband size can potentially reduce cell throughput in the SBFD deployment since the resources in the guard band are then always reserved and cannot be scheduled. Also, the guard band would then be present even if neighbor cells do not schedule the SBFD UL subband for UL transmission in the slot, e.g., un-schedulable RBs would exist even if the guard band size could be reduced due to less CLI.

In principle, it is possible to consider that the gNB creates the guard band(s) implicitly through DL/UL scheduling (Figure 6). The gNB schedules or configures the DL or UL signals/channels and creates guard band(s) according to its needs by not assigning the desired number of guard band RBs next to the configured UL subband. UEs simply follow the gNB DL/UL scheduling decision or RRC configured grants and do not know about the existence of the guard band(s). Note that for legacy Rel-15 UEs operating in a TDD cell with SBFD, this kind of implicit guard band configuration must always be used by the gNB. When no explicit guard band configuration is provided by the gNB to UEs, this gives increased flexibility to the gNB implementation. For example, the gNB can then schedule UL or DL signals/channels from/to UEs in the SBFD symbol when much spatial isolation is possible using only small or no guard band. In cases where little spatial isolation between DL and UL UE pairings scheduled in the SBFD slot is available, larger guard bands be used by the gNB. Different time-domain symbols can use a different number of guard band RBs. No specification impact exists.

When the guard band(s) are explicitly configured for the UE (Figure 6), then the UE implementation for transmitting or receiving in SBFD slots can potentially benefit from additional UE Rx and Tx-side BB filtering on the PDSCH and PUSCH allocation BW in the DL and UL subband(s), respectively. While it should not be assumed that SBFD-aware UE implementations must use more stringent Tx/Rx BB filtering than what is currently necessary to meet the existing Rel-15 NR UE RF requirements to improve their in-channel selectivity and in-band blocking performance, it is at least desirable to enable such UE functionality as future capability.

Figure 6: Guard band configuration using explicit configuration vs. gNB scheduling-based

In SBFD slots with a configured UL subband, the UE operational bandwidth can be scaled down to the size of the UL subband when compared to the normal UL slot where the UE operational bandwidth is given by the size of the UL BWP. UE modem functional blocks such as D/A converter and BB can use a lower voltage and/or a lower clock rate due to less data sample for processing. This is beneficial for the UE Tx PA regime and reduces UE power consumption. The existence of the explicitly configured guard band RBs may also be exploited by the UE implementation when implementing BB filtering of the Tx-side waveform in the PUSCH allocation BW when treating the guard band RBs as excess bandwidth.

We note however that UE Tx-side BB filtering to make the waveform “gentler” for purpose of optimizing the UE PA is subject to many constraints, e.g., the available UL transmit power and PAPR must be controlled with respect to EVM, ACLR, IBE and SEM. It can therefore not be assumed that potential gains from UE knowledge of the explicitly configured guard band(s) are uniform or are always available.

Explicit configuration of the guard band(s) must be considered in conjunction with explicitly configured or implicitly determined DL subband(s). It has already been agreed that the frequency-domain occupancy of the SBFD UL subband is explicitly configured.

When the frequency-locations of the SBFD DL subband(s) are also explicitly configured by RRC, an implicitly determined guard band can be created by not configuring a number of RBs for either the DL or UL subband(s). In this case, the UE still knows the size of the guard-band separating the explicitly configured DL and UL subbands and potential enhancements to the UE-side Tx and Rx filter implementation in BB relying on the knowledge of the number of excess RBs provided by the guard band are still possible.

If the DL subband(s) are not explicitly configured, then the UE must use the knowledge of the DL and UL NR carrier BW parameters and the UL subband size to determine the remaining RBs in a symbol with SBFD operation as DL subband(s). This approach is possible at least in principle, but not sufficiently flexible especially when guard band(s) are considered.

In summary, we think that the UE knowledge of the number of guard band RBs on the SBFD symbol is beneficial but prefer that the frequency location of the DL subband(s) is explicitly configured for the UE, which allows the UE to determine the size of the guard band implicitly as unused RBs not allocated to either UL or DL subband(s).

- Proposal 6: At least for RRC_CONNECTED mode, the frequency location(s) of the DL subband(s) are explicitly indicated.

- Proposal 7: At least for RRC_CONNECTED mode, the frequency location(s) of the guard band(s) are implicitly determined by the UE as RBs not configured as either UL or DL subband(s).

DL and UL scheduling

For SBFD operation, one key Rel-15 NR feature to preserve is the support for dynamic TDD operation where the gNB scheduler can dynamically determine the transmission direction. NR uses 3 different signaling mechanisms to provide the information to the UE whether a transmission resource is used for UL or DL. The basic principle is that the UE monitors the configured PDCCH and then transmits or receives according to the received DCI. The UL-DL frame configuration can be signaled to the UE as cell-specific pattern, e.g., SIB1 tdd-UL-DL-ConfigCommon, and a UE-specific pattern can additionally be configured by tdd-UL-DL-ConfigDedicated. The third possibility is to configure SFI, e.g., DCI format 2_0.

Generally, it is very desirable to avoid that the introduction of the SBFD feature results in hard UL-DL resource partitioning in the TDD cell. When the SBFD subband configuration is signaled by the gNB to the UE, such configuration determines where an UL or DL transmission may occur. gNB scheduling then actually assigns the corresponding radio resources to actual UL or DL transmissions as needed.

Scheduling aspects for SBFD operation were discussed during RAN1#110 and RAN1#110bis-e. One design question whether to allow for a 3rd subband, e.g., flexible subband type in addition to the UL and DL subband types. Related is the question whether a configured UL subband can also be DL scheduled by the gNB for the same or at least different UEs. Another design question is whether SFBD operation configured on legacy DL or F symbols should possibly allow for different scheduling approaches, e.g., a flexible subband type may exist in a legacy F symbol but not a legacy DL symbol.

In our view, SFBD operation when the UL subband is configured on legacy D or legacy F symbols should be the same. In particular, the gNB configured frequency location of the SBFD UL subband inside the gNB channel BW is first chosen according to fixed deployment needs, e.g., depending on the operator band segment. The frequency-domain location and size of the SBFD UL subband then depends on the capabilities of the gNB-side SBFD implementation. For many WA or MR base station class implementations, it can be expected that gNB-side SI cancellation capabilities will often restrict the possibility of scheduling UL transmissions outside the configured SBFD UL subband due to RF and BB constraints. These will therefore preclude scheduling UL transmissions for SFBD-aware or the legacy UEs on SBFD symbols outside the configured UL subband in any SBFD time-domain resources. While legacy UEs cannot be made to UL transmit using legacy DL symbols with SBFD operation, the underlying gNB implementation for SIB1-configured and indicated DL and F symbols is still the same. In the case of some LA base station class implementations or for FR2-1, less constraints exist. For the LA BS classes where less hardware constraints imposed by the SIC implementation exist, the SBFD UL subband can then be RRC configured sufficiently large, e.g., up to the size of the NR channel BW. The SBFD-aware UE can then be either DL or UL scheduled over up to the full NR channel BW in the SBFD slot. Still, even then when it is possible to configure almost the entire NR carrier BW as UL subband, the same gNB implementation capabilities will apply to legacy DL and legacy F symbols, so there is no need to distinguish these symbol types with respect to SFBD operation.

Knowing that it is not possible for the gNB for UL schedule outside the configured UL subband for many gNB implementations, the remaining design question is then to consider if the configured UL subband can possibly be used by the gNB for DL scheduling.

In our view, for SBFD using both legacy D/F symbol types, the gNB scheduler should still be able to schedule the SBFD UL subband for DL transmissions to UEs as needed. To avoid semi-static resource partitioning, the SBFD symbol can be used as DL-only scheduling symbol. This principle is shown in Figure 7 where an UL transmission using the configured SBFD UL subband is scheduled for the UE in slot #1 but is then used for DL scheduling to the same UE in slot #2. When the gNB configures an SBFD symbol as ‘DUD’, the symbol may be scheduled as ‘DDD’ or all-DL. It might be reasoned then that the configured UL subband is in fact inherently a “flexible” subband. Therefore, there is no need for a separate ‘flexible’ subband type in addition to the UL and UL subband type(s). Note however that flexibility is more limited for the DL subband(s). Only DL scheduling can occur in the DL subband(s). UL scheduling using the configured DL subband(s) is not possible.

Figure 7: SBFD operation Alt.4 with dynamic DL/UL scheduling of the configured UL subband

Especially when UE-side Tx/Rx BB filtering depending on the frequency locations of the configured DL or UL SBFD subband(s) inside the UE DL or UL BWP is used, the need for filter re-configuration and switching time must be accounted for. The ability of the gNB scheduler to exploit the possibility of DL scheduling the UL subband will depend on inter-cell intra-subband DL-UL CLI levels. Overall system-level performance will determine if exploiting the ability of the gNB to dynamically DL schedule a configured UL subband is meaningful. This will depend on the network load, offered DL and UL traffic, and other factors. For many low-load and small RU usage scenarios, it can be expected that CLI levels during SFBD operation are manageable.

One design option is to allow DL scheduling of the configured UL subband for any UE, e.g., including SBFD-aware UEs configured with the SBFD UL subband. Another design option is to limit the possibility of DL scheduling in the configured SBFD UL subband to other UEs, e.g., the UE configured with the SBFD UL SB in the slot cannot be DL scheduled inside the UL subband, but other UEs can be DL scheduled in the UL subband. We recommend investigating these aspects further in the Rel-18 SID.

- Proposal 8: For the SBFD-aware UE semi-statically configured with the UL subband in legacy D and F symbols,
-	UL transmissions within the configured UL subband are allowed in the symbol
-	UL transmissions outside the configured UL subband are not allowed in the symbol
-	DL receptions inside the configured UL subband are allowed in the symbol

Signaling aspects

Even though most Stage 3 design aspects should be left to the later WID, some consideration must be given to the gNB signaling to indicate the SBFD subband(s) to the SBFD-aware UE.

Providing the SBFD UL subband configuration to the UE in terms of the allowed SBFD time-domain symbols/slots where the SBFD UL subband can be configured independently from legacy SIB1 tdd-UL-DL-ConfigCommon is necessary. The advantage is that transmission behavior of SBFD-aware UEs in the TDD supporting SBFD operation can be configured independently from the transmission behavior for legacy UEs. For example, it is then possible to schedule the UL transmissions in the SBFD UL subband even in the symbols/slots configured as DL for the legacy UEs or to only schedule UL transmissions in the SBFD UL subband in the SBFD slots, but not the normal UL slot. It is then also possible to define separate UE behavior not currently possible in Rel-17 with respect to CSI reception and SRS transmissions for purpose of inter-subband CLI measurements and associated reporting in the TDD cell with SBFD operation.

At least for SBFD operation when the UE is in RRC_CONNECTED mode, there is need to explicitly configure the SBFD UL and DL subbands. gNB-side SBFD operation is best configured with respect to the NR carrier, e.g., not with respect to a UE BWP. The frequency location of the SBFD UL subband has a start RB and allocation bandwidth. The SBFD subband configuration is signaled to the UE with respect to the CRB grid similar to the Rel-15 CSI-RS resource configuration. In the cases of both Rel-15 CSI-RS resource configuration and the SBFD subband configuration, the UE considers as applicable the configured RBs falling within its configured DL or UL BWP.

In time-domain, UL transmissions in the SBFD UL subband may not be possible on all symbols in a slot, e.g., when PDCCH monitoring at the beginning of the slot is configured on the first symbol of the slot. Certain DL symbols such as SSB may be excluded additionally from configuring SBFD such as discussed in Section 2. Configuration of the SBFD time-locations for the SBFD subband(s) with symbol level granularity at least for RRC_CONNECTED mode is desirable in terms of forward-compatibility and design flexibility. It should be avoided to introduce a number of explicit exception rules with respect to which symbols cannot support SBFD operation. Support for SIB1-based indication of SBFD subbands however is necessary in the strict sense only when SBFD operation is supported during initial access, e.g., when the UE is in RRC_IDLE/INACTIVE mode. SBFD configuration and signaling must therefore be considered separately for SBFD operation in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode.

PDSCH/PUSCH allocations using less than a minimum of 4 symbols allocated by TDRA are not supported by Rel-15 NR. Rel-15 NR short PUCCH transmissions using 1 or 2 symbols can be used if SBFD is configured for 1 symbol or 2 contiguous symbols. Therefore, when SBFD operation is configured for symbols in a slot, it may be expected that in most cases, most of the symbols of the slot are then configured for SBFD operation.

- Proposal 9: At least for RRC_CONNECTED mode, the SBFD subbands are configured with respect to the CRB grid.

- Proposal 10: Frequency locations and time locations of SBFD subbands can be configured independently for a UE in RRC_CONNECTED mode and a UE in RRC_IDLE/INACTIVE modes (if supported).

UE enhancements for improved SBFD performance

During RAN1#110bis-e, two proposed FL agreements were discussed with respect to potential enhancements to resource allocation enhancements and for scheduled/configured UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, e.g., FL proposals 1-12 and 1-13. In this section, we provide our views on the potential for several UE-side SFBD enhancements and their criticality with for existing NR functionality other than CLI management.

Note that we additionally discuss CLI enhancements and provide further details on gNB-to-gNB and UE-to-UE CLI mitigation and handling common to SBFD and d/f-TDD in our companion contribution [2].

In summary, we consider gNB coordinated scheduling in time/frequency including gNB-to-gNB spatial domain coordination, power-domain enhancements such as gNB DL transmit power coordination and UE UL Tx power restrictions as promising candidate methods for CLI mitigation in TDD networks supporting SBFD operation. Some of these features require new Xn/F1AP signaling support. Network-side coordinated transmission of NZP/ZP CSI-RS resource(s) set(s) for purpose of CLI signal power and interference measurements and corresponding signal power measurement and interference measurement resources can be of benefit to advanced gNB scheduler implementations. Enhancements to UE transmission and reception timing when using SBFD in the TDD cell should be studied. In addition, we also consider enhancements to UE CLI reporting such as subband based and spatial domain-based reporting and fast L1 CLI reports as beneficial to improve the UE-side support for SBFD operation in the network [2].

PDSCH/PUSCH resource allocation

Resource allocation in time-domain

An example with per-slot scheduling during SBFD operation in the TDD cell is shown in Figure 7 for SBFD operation Alt.4 with dynamic DL/UL scheduling of the UL subband for the case of an n78 (3.5 GHz) carrier with 100 MHz channel BW.

CORESET#1 is configured in the DL subband of the SBFD slot for the SBFD-aware UE. Search Space #1 can be configured once per slot. For AL8 (48RBs) @30kHz and using a 1 symbol CORESET, some 17-18 MHz are required. CORESET#1 can be configured inside the DL subband. Benefits of such a scheduling approach include good DL and UL scheduling latency. For smaller channel BW such as n78 using 40 MHz or 60 MHz, the CORESET#1 can still be configured across one of the DL subband and the UL subband. Drawbacks include that not all time-domain symbols are available when scheduling UL transmissions using the SBFD UL subband in the SFBD slots. A loss of 1 dB is incurred for UL transmissions when not all 14 symbols in the slot can be scheduled for UL transmissions.

For UL and DL scheduling in a TDD cell with SBFD, both single slot and multi-slot scheduling of PUSCH and PDSCH must be considered. One important use case for TDD with SBFD is improved UL coverage in the existing deployment grid. SBFD enables to significantly increase the UL slot utilization ratio by making more time-domain resources available to UEs in bad coverage which transmit at maximum configured UE output power. In DDDSU, the UE can only transmit UL in 20% of the time-domain resources at maximum power, whereas with SBFD and DXXXU it can potentially transmit in up to 80% of the time-domain resources. Rel-15 PUSCH aggregation and Rel-16/Rel-17 PUSCH repetition Type A/B features are therefore important when scheduling the UE across multiple slots.

In the TDD cell supporting SBFD operation, UEs requiring best UL coverage use multi-slot PUSCH scheduling, e.g., Rel-15 PUSCH aggregation or Rel-16/Rel-17 PUSCH Type A repetition (Figure 8).

CORESET#1 must then be configured in the SBFD DL subband of the SBFD slot such that all time-domain symbols per slot can be used for the PUSCH repetition in a slot. The UL link budget is improved because up to 14 symbols in a slot where the UL subband is configured can be used for PUSCH transmission. Potentially, TDD can now also harvest the additional Rel-17 JCE gains because 4-slot DMRS bundling across contiguous slots without DL/UL switching is enabled by SBFD.

Figure 8: SBFD operation with multi-slot PUSCH transmission for best UL coverage

Several antenna and panel design options exist to support gNB-side full-duplex operation. Design options may be expected to change over time as technology matures. For example, the antenna panel to provide spatial separation of Tx and Rx paths and for gNB side SIC can be based on a fully split design where ports are either dedicated to DL transmission or to UL reception. Another possibility is shared design where a certain number of ports can be used for both DL transmission and UL reception with SBFD, but a number of ports is dedicated to either DL transmission or to UL reception, respectively. Therefore, it cannot be expected that the number of TRXs and the available Tx or Rx aperture area used for Tx and/or Rx in normal DL or normal UL slots is necessarily the same when compared to the DL transmission or UL reception in SBFD slots. The link gain and resulting link budget when transmitting an UL signal or channel in the SBFD UL subband of a slot is different when compared to transmitting the same UL signal or channel in the normal UL slot. Similar considerations apply to transmission of DL signals/channels in the DL subbands of SBFD slots.

PUSCH transmissions of a Rel-15 PUSCH aggregation and Rel-16/Rel-17 PUSCH repetition Type A when transmitted in the SBFD UL subband of SBFD slots and in the normal UL slot will therefore be subject to mis-matched link adaptation settings. It is desirable to restrict the PUSCH transmissions of a PUSCH repetition with respect to the time-domain resources in a TDD cell with SBFD. It should be possible to transmit such PUSCH repetitions using only the UL subband of the SBFD slots, but not in the normal UL slot. Note that even when PUSCH repetitions are limited to the use of the SBFD UL subband in DXXXU, the resulting UL slot utilization ratio is still higher than currently possible with DDDSU.

Another important design consideration for SBFD operation in the UL is that the separation of SBFD symbols/slots and the UL slot(s) with respect to the resource allocation in time-domain when introducing gNB-side SBFD operation also allows to increase the achievable UL spectral efficiency and UL cell throughput in the normal UL slot(s). UL transmissions from low-SINR UEs can be offloaded to the SBFD slots. UEs in good geometry can then be scheduled with high MCS settings using large contiguous RB allocations in the normal UL slot(s). Given that most existing UE implementations only support UL contiguous resource allocation, e.g., PUSCH resource allocation type 1, the offloading of the low-SINR UEs to the SBFD UL subband in the SBFD slots prevents resource fragmentation in the normal UL slot.

- Proposal 12: Multi-slot PUSCH transmissions and PUCCH repetitions can be configured to only use the SBFD slots/symbols

Resource allocation in frequency-domain

PDSCH resource allocation types 0 and 1 for DL scheduling and PUSCH resource allocation types 0 and 1 for UL scheduling must be considered when introducing SBFD operation in the TDD network. PDSCH RA Type 0 and Type 1 only for PDSCH without interleaving and PUSCH RA Type 1 for PUSCH without interleaving are UE mandatory features (FG 5-1). PUSCH RA Type 0 and dynamic switching between PDSCH Type 0 and 1 or between PUSCH Type 0 and 1 are separate optional UE features with capability signaling (FG 5-2 to 5-4). Almost contiguous UL CP-OFDM is an optional UE feature (RAN4 FG 2-7).

We assume that a guard band between DL subband(s) and the UL subband in SBFD slots is at least implicitly provided by gNB scheduling. The gNB must also ensure that the PUSCH is allocated or that PUCCH resource are configured for the UE inside the SBFD UL subband when scheduling the UE in the SBFD UL subband of an SBFD slot. Note that this section discusses frequency-domain RA for the case that SBFD operation is configured in legacy DL or F slots.

Using PDSCH resource allocation type 0, the resource block assignment information includes a bitmap indicating the RBGs that are indicated to the UE. The RBG size depends on the UE BWP. One of two possible sizes can be signaled by RRC. For n78 and 100 MHz carrier BW and a UE DL BWP of 100 MHz, worst case, the nominal RBG size is always 16. DL scheduling to UEs in the 2 DL subbands of the SBFD slot then may suffer granularity issues. When the UL subband occupies center 51 RBs, 4*16=64 RBs of 273 RBs cannot be DL scheduled to the UE because an RBG would be in partial overlap with the UL subband. The effective loss may be reduced when considering that part of these partial overlap RBGs provide an implicit guard band configuration.

Using PDSCH resource allocation type 1, the resource block assignment information indicates a set of contiguously allocated inter-leaved or non-interleaved VRBs to the UE. Clearly, DL scheduling across the UL subband in the SBFD slot is then not possible. PDSCH RA Type 1 can still be used without limitation for smaller PDSCH allocations contained within one of the 2 SBFD DL subbands.

An additional possibility is to mask RBs and symbols of the SBFD UL subband in an SBFD slot using a Rel-15 or Rel-16 cell- or UE-specific RateMatchPattern configured for the UE. Support of FG 5-26 (semi-static rate-matching resource set configuration for DL) is mandatory for legacy UEs, but dynamic signaling support of the bitmap 1/2/3 as by FG 5-27 is only optional. Note that when the gNB uses the configured UL subband in the SBFD slot to DL schedule a UEs, the RRC configured RateMatchPattern cannot be disabled by the scheduling DCI format 1_1. The gNB can then only schedule PDSCH using (non-interleaved) PDSCH RA type 1 either in the lower or upper DL subband of the SBFD slot, or PDSCH across both DL subbands cannot be mapped across the UL subband at all (Figure 9).

In principle, it is possible to consider 3 types of PDSCH resource allocation enhancements for SBFD operation. Option 1 is no change compared to Rel-15 behavior. Option 2 is to introduce a 3rd more optimal set of RBG set sizes for use with PDSCH RA Type 1. Option 3 is to define and introduce a new RA Type. In our view, Option 3 is not meaningful, given the limited impact from granularity observed with partially overlapping RBGs for PDSCH RA Type 0 and the significant associated evaluation and specification efforts. Option 2 may be seen as a low-cost approach, e.g., RRC signaling based solution. Similar considerations apply with respect to the use of the existing RateMatchPatterns.

Figure 9: SBFD operation and use of PDSCH resource allocation type 1 with RMP

For PUSCH resource allocation type 1, the resource block assignment information indicates a set of contiguously allocated inter-leaved or non-interleaved VRBs to the UE. Clearly, UL scheduling across the UL subband in the SBFD slot is possible within the existing set of constraints inherited from Rel-15.

In the case of multi-slot PUSCH using the UL subband of SBFD slots and the normal UL slot, frequency-hopping if configured can result in limitations when SBFD operation is used in the TDD cell. It can be expected that the SBFD UL subband is often placed in the center of the NR carrier. During multi-slot PUSCH transmissions across the SBFD and normal UL slots, the same FDRA and FH behavior must apply in these slots as by existing Rel-15 specifications, e.g., FH in SBFD slots must be configured by the gNB such that hops are confined to the UL subband, and the same configured hop size then also applies to the normal UL slot. The available contiguous UL scheduling bandwidth in the normal UL slot is then fragmented. One option to avoid such an issue is to restrict multi-slot PUSCH transmissions to only use the SBFD slots but not the normal UL slot in time-domain. Another option is to introduce a new or separate additional FH configuration for the normal UL slot for use with PUSCH RA Type 1. Note that similar issues resulting in fragmentation of the UL scheduling BW in the normal UL slot can also exist when single-slot PUSCH scheduling is used due to constraints arising from the UE DL/UL BWP pair configuration with respect to the channel occupancy of the SBFD UL subband.

- Proposal 13: RAN1 to study enhancements to SBFD operation for PDSCH resource allocation type 0

- Proposal 14: RAN1 to study enhancements to SBFD operation for PUSCH resource allocation type 1

CSI-RS resource set configuration and CSI reporting

When SBFD operation is introduced in a TDD cell, the potential impact and limitations of CSI-RS configuration and reports must be considered. We first discuss configuration of CSI-RS resource set(s) and then discuss CSI reporting.

An example for possible CSI-RS resource set configurations is shown in Figure 10. When SBFD operation is supported in the TDD cell, a CSI resource set in the SBFD slot can be configured with a frequency occupation located entirely outside the SBFD UL subband or the CSI resource set contains the UL subband of an SBFD slot, e.g., is configured across the DL and UL subbands. Partial overlap scenarios, e.g., the CSI resource set is configured with a frequency occupation comprising parts of the UL subband of a full-duplex slot is also possible. In Rel-15, the CSI-RS frequency occupancy must be configured in integer multiple of 4 RBs and has size of minimum DL BWP or 24 RBs. Either every or every other RB is occupied by CSI-RS REs inside the configured CSI bandwidth. Special rules apply to the case of the CSI-RS resource set with density = 3 (TRS), where the minimum occupied BW must be 52 RBs. The CSI bandwidth configuration applies independent of type, e.g., applies to both CSI-RS designated by RRC as NZP or ZP. Note that CSI-IM occupy every RB inside the configured CSI bandwidth. The CSI bandwidth is configured independently for different (multiple) CSI resource sets. The UE implementation must support a minimum number of multiple configured CSI-RS resource configurations per frame.

Figure 10: SBFD operation and configuration of CSI-RS resource sets

When the CSI bandwidth of the configured CSI-RS resource set is confined within the DL subband of the SBFD slot, no CSI reporting is possible for the UE for outside the CSI bandwidth. Configuration of multiple CSI resource sets becomes necessary, e.g., 2, to cover both DL subbands of the SBFD slot. Another consideration is then that the gNB scheduler cannot obtain CSI reports from the UE for purpose of DL scheduling in the UL subband. There is no immediate impact on aperiodic, semi-persistent and periodic CSI-RS resources or CSI reports. If the gNB scheduler requires CSI reporting inside the configured UL subband of the SBFD slot from the UE, aperiodic triggered CSI reporting and associated CSI-RS resource set must be used. Dependencies exist with respect to reference slot for the CSI reports.

When the CSI bandwidth of the configured CSI-RS resource set contains the DL and UL subbands of the SBFD slot, either full or partially, CSI reporting can be impacted. Note that Rel-15 NR already allows to toggle on or off individual (CSI) subbands through the CSI reporting configuration (38.214). The reportFreqConfiguration contained in the CSI-ReportConfig indicates the frequency granularity of the CSI report. A CSI Reporting Setting configuration defines a CSI reporting band as a subset of subbands of the bandwidth part, where the reportFreqConfiguration indicates the csi-ReportingBand as a contiguous or non-contiguous subset of subbands in the bandwidth part for which CSI shall be reported. When wideband CQI reporting is configured, a wideband CQI is reported for each codeword for the entire CSI reporting band. When subband CQI reporting is configured, one CQI for each codeword is reported for each subband in the CSI reporting band. In consequence, even when the CSI-RS resource set is configured across the DL and UL subbands of the SBFD slot, the CSI report configuration linked to the CSI-RS resource set can blank out the UL SB. Granularity issues exist similar to the case of PDSCH RA Type 0. The CSI reporting subband size can be configured from two possible values depending on the UE DL BWP size. For example, 16 or 32 PRBs subband size can be configured for a UE DL BWP of 273 RBs.

Another consideration when introducing SBFD operation in the TDD network is the need to apply different CSI-RS Tx EPRE settings by the gNB in the two DL subbands of the SBFD slot. One reason is that even when digital BF is used, a different number of TRX may be available to the gNB for DL beamforming operation in the upper DL subband of the SBFD slot and the lower DL subband of the SBFD slot. Tx power imbalances exist across the DL subbands. Another reason is the direct Tx-Rx leakage interference cancellation by the gNB in presence of live DL and UL scheduling. Equal RB occupancy in the first and second DL parts of the full-duplex slot is not always possible due to varying UE traffic loads. Similarly, an equal or balanced RB occupancy in the UL subband is not always possible. The Tx EPRE of the DL signal(s) or channel(s) may be adjusted accordingly by the gNB accounting for DL and/or UL scheduling in the SBFD slot, but Rel-15 NR forces to set the same EPRE setting for both DL subbands in the SBFD slot for the configured CSI-RS resource set.

In summary, the possibility to configure multiple CSI resource sets and multiple CSI reports and trigger conditions as by Rel-15 NR provides much flexibility to configure per-SBFD subband CSI-RS resources and CSI reports. However, limitations exist with respect to the configuration of multiple CSI-RS resource sets with a frequency occupation limited to either the first or the second DL subband and when configured across the DL and UL subbands. UE complexity can be increased, and CSI reporting restrictions can result due to the need for the UE to support more concurrently active CSI measurement and reporting configurations during SBFD operation.

- Proposal 15: RAN1 to study enhancements to resource allocation with SBFD operation for CSI-RS resource set configuration and CSI reporting

Random Access signals/channels

When the SBFD UL subband is available to schedule UL transmissions from the UE, consideration must be given to which types of UL signals or channels can be configured or scheduled using the SBFD UL subband. In section 5 and 6.1 we discussed PUSCH and PUCCH scheduling aspects.

Using the SBFD UL subband for PRACH transmissions (Figure 11) has the immediate benefit that fragmentation of the contiguous UL scheduling BW in the normal UL slot can be avoided. UL throughput and spectral efficiency is increased in the TDD cell with SBFD, because good SINR UEs allowing for high instantaneous UL data rates immediately benefit from the availability of larger available contiguous UL scheduling BW. UEs experiencing good SINR or requiring high UL data rates are best scheduled in the normal UL slot(s) where CLI is controlled network wide.

It must be considered that RACH occasions in the UL slot are rarely configured at the carrier edges in practical NR TDD deployments for various reasons. RACH in the UL slot segments part of the schedulable UL bandwidth. Most UEs support only the mandatory PUSCH resource allocation type 1, e.g., contiguous PUSCH allocations. When placing PRACH ROs in SBFD slots, PRACH creates CLI only when actually used by a UE selecting a random-access resource. A PRACH RO which is configured, but not used by any UE in the SBFD slot does not create CLI towards other UEs in the slot. Note that when PRACH ROs are configured in the normal UL slot, they can seldom be re-used for UL data scheduling and shared data/random access reception by the gNB, e.g., when configured, such resource are not available for UL data.

Figure 11: SBFD operation using the SBFD UL subband or normal UL slot(s)

Allowing for configuration of PRACH in the SBFD UL subband and use of the SBFD UL subband for RRC and NAS signaling exchanges during the 5G Initial Attach procedure also reduces UE initial acquisition time in NR TDD. It must be considered that about 8-10 DL and UL messages must be exchanged by network and UE during initial attach. These signaling exchanges are mostly “atomic” and “in sequence”, e.g., successful completion of a NAS procedure such as Authentication Request/Response is required before a subsequent NAS procedure such as NAS Identity Request can be initiated. NR TDD 30 kHz when compared to NR FDD 15 kHz in the NR low band is penalized by the average UL frame alignment delay incurred for each of the UL RRC and NAS messages due to limited number of UL slots. This typically results in 2x longer initial acquisition duration when comparing NR TDD to FDD.

While we see several potential benefits in allowing for the possibility to allow for PRACH transmission using the SBFD UL subband of SBFD slots, SBFD operation at least initially should attempt to evaluate feasibility for full-duplex operation in RRC_CONNECTED mode. RACH procedures are of relevance to RRC_CONNECTED mode in a more limited context, e.g., for network-controlled cell-level mobility handling procedures using contention-free random-access procedures where access information to the target cell is provided through RRC Handover Command message.

- Proposal 16: RAN1 to study potential benefits and specification impacts when using SBFD slots/symbols for random access

SBFD feasibility and implementation aspects

When SBFD is implemented at the gNB, the received UL signal at the gNB is subject to co-channel cross-link interference (CLI) from the gNB side transmitter. Methods to cancel the CLI include passive methods which rely on the antenna isolation between Tx and Rx antennas, active methods which utilize RF or digital signal processing, hybrid methods using a combination of these, and filtering.

Achieving a sufficient level of SIC is the most critical part when implementing SBFD at the gNB. Without adequate SIC capability, the interference from the transmitted DL signal would corrupt the received UL signal (Figure 12(a)). To solve this problem, various SIC schemes can be used. Using the example of Figure 12(b), SIC capability can be provided through the antenna or panel design (A), can be applied in RF domain to the RF signal (B) or in digital signal domain (C), or a combination of these.

Figure 12: gNB transceiver architecture with self-interference cancellation capability

For example, antenna SIC can be used to minimize the leakage power from the Tx ports to the Rx ports of the panel, and digital SIC is then used to handle any residual interference after antenna SIC. DL out-band signal power flowing into the UL Rx path can be effectively suppressed below the noise floor level to guarantee the UL receiver performance. Also, by combining digital pre-distortion (DPD) at the Tx path and digital SIC at the Rx path, the out-band interference from the DL signal to the UL signal can be effectively mitigated by the gNB such that the need for a guard band between the UL and DL signals is minimized. In FR2, the use of separate antenna panels can provide additional spatial isolation.

To check and demonstrate the feasibility and viability of SBFD, Samsung has developed and tested two different testbeds, one operating at FR1 3.5 GHz and one for FR2-1 26 GHz. These validate the feasibility of SBFD operation when implemented at the gNB-side.

Self-interference Cancellation

Spatial-domain and antenna isolation

To simultaneously transmit and receive in the TDD gNB radio unit using SBFD, sufficient Tx/Rx isolation is required. NR TDD radio units use duplexers and multiplexers for the antenna panels which are shared for Tx-mode and Rx-mode in TDM. Such existing RF components alone do not provide sufficient isolation when introducing SBFD in the TDD gNB radio unit.

One solution to increase the Tx/Rx isolation during simultaneous transmission and reception by the gNB is to physically separate the Rx panel and the Tx panel, e.g., separation in antenna domain. Tx/Rx isolation can be increased first simply by increasing the spatial distance. Tx/Rx isolation performance can be further increased when an additional RF barrier structure is used. Using the RF barrier between the Tx and Rx panels affects the required spatial distance separating the Tx and Rx panels. A well-designed RF barrier can minimize the need for large spatial separation and mostly preserve the existing antenna form factor and enclosed volume when compared to legacy TDD. To design an efficient RF barrier, various electromagnetic resonator structures can be incorporated into the antenna design, e.g., wall(s), gap(s), or a combination of them. These result in surface wave nulling and can further block the undesired leakage signals from the Tx panel to the Rx panel.

Figure 13 shows measurement results with respect to the distance between upper and lower antenna panels in our FR1 3.5 GHz SBFD testbed.

While it can be expected that spatial isolation numbers vary depending on the form and particular layout configuration of antenna elements in the upper and lower panels, we have shown that >80 dB antenna isolation is possible between the Tx and Rx panels in FR1.

Figure 13: FR1 testbed and SIC performance when varying distance between upper and lower panel

We observe similar and even better antenna isolation performance with the FR2-1 26 GHz testbed where panel separation can be exploited.

Figure 14 shows the FR2-1 testbed using 2 Tx panels and 2 Rx panels. Unlike the FR1 3.5 GHz testbed where SBFD performance is verified for a single NR carrier setup, the FR2-1 gNB-side testbed uses intra-band contiguous CA with 4 (or 3 CCs). Total aggregated BW is 400 (or 300) MHz with 100 MHz per CC. 3 (or 2) CCs are used for the DL and 1 CC for the UL.

Like described in the case of the FR1 3.5 GHz testbed, the Tx panel and the Rx panel in the FR2-1 26 GHz testbed are separated by a separation distance. Additional Tx/Rx isolation performance is then enabled by using an RF barrier, e.g., an additional EM resonant between the panels. In the case of FR2-1 26 GHz, since each panel can perform more directive beamforming in analog domain than possible in FR1 using mMIMO panels, the FR2-1 antenna isolation performance is better than what is achievable in FR1. An average of 87 dB antenna isolation can be observed based on the measurement results obtained from our FR2-1 testbed.

Figure 14: FR2-1 testbed and SIC performance when varying the operating frequency

An important design consideration for increased spatial isolation provided by the RF barrier is whether such stopband performance is stable over a wide enough frequency range. EM isolators and resonant structures are designed around a specific center frequency, e.g., 3.5 GHz. Therefore, design of the resonant structure must account properly for the channel BW and NR operating band under consideration to provide a sufficiently large stopband between Tx and Rx panel. Another consideration is that undesired Tx/Rx interference is created by multiple EM sources, e.g., antenna elements in the Tx panel. Therefore, diffusion of the corresponding surface waves is more challenging when isolating the Tx and Rx panel. Despite these challenges, our FR1 3.5 GHz and FR2-1 26 GHz testbeds have achieved isolation performance that show almost uniform antenna and panel isolation performance with respect to frequency for the 100 MHz channel BW of the NR carrier in 3.5 GHz and 100 MHz CC BW in 26 GHz. Figure 14 shows measurement results from the FR2-1 testbed with respect to achievable antenna isolation as a function of the operating frequency.

Observation 3: 80 dB in FR1 and 87 dB in FR2-1 antenna isolation using spatial separation and RF barrier can be achieved

Observation 4: Stopgap performance of the RF barrier for FR1 100 MHz and FR2-1 100 MHz channel BW is feasible

Frequency-domain and digital cancellation

On top of the spatial isolation to prevent undesired Tx-Rx interference from the Tx panel to the Rx panel during simultaneous transmission and reception by the gNB radio unit, additional Tx/Rx isolation can be achieved in frequency-domain.

The Tx signal and the Rx signal are respectively allocated to non-overlapping frequency-domain resources on the same time-domain symbol during simultaneous transmission and reception, e.g., SBFD. At least the waveform roll-off therefore reduces the magnitude of the Tx-Rx interference to which the Rx signal is subjected. Additionally, BB filtering can be applied to further increase the achievable isolation.

The use of frequency-domain isolation between the Tx and Rx signal allocations is primarily an approach that serves the purpose of reducing the amount of self-interference which must be further cancelled by a digital cancellation stage. Note that TDD gNB radio unit design must also account for ADC and LNA in the receiver path, e.g., to prevent Rx saturation or blocking by the spectral leakage created from the undesired Tx signal.

In the case of gNB-side SBFD operation, the SBFD UL subband can be considered as out-of-channel with respect to the 1 or 2 SBFD DL subband(s). Undesired spectral leakage from the DL Tx signal in the gNB into the Rx path are reduced similar to the case of out-of-channel leakage, e.g., comparable to the gNB Tx-side Adjacent Channel Leakage Ratio (ACLR) for coexistence between two operators on adjacent channels in the same NR band. Note that ACLR is determined by the non-linear characteristics of the PA and corresponding RF requirements are set by RAN4, e.g., 45 dBc for the gNB Tx.

While it can be considered to assume that the achievable Tx-to-Rx interference from the SBFD DL subband to the UL subband can only guarantee performance according to the less stringent in-channel RF requirements, our FR1 3.5 GHz testbed implementation shows that the use of digital pre-distortion (DPD) techniques to improve upon the non-linearity characteristics of the PA can achieve 45 dBc isolation between the SBFD DL and UL subbands.

Figure 15 shows the achievable isolation in frequency domain for FR1 SFBD when Tx-to-Rx leakage is also compensated for by DPD based on the FR1 3.5 GHz testbed.

Observation 5: 45 dBc subband leakage ratio between the SBFD DL and UL subband when using non-overlapping frequency resources with digital pre-distortion can be achieved in FR1

Figure 15: FR1 testbed and PSD for SBFD DL and UL SBs after antenna isolation and digital pre-distortion

In the case of FR2-1, frequency-domain isolation for SBFD is of particular importance. Non-linear characteristics of mmWave PAs are worse than those of FR1 mid-band PAs. 3GPP ACLR requirements are more relaxed in FR2-1 when compared to FR1. This is due to beamforming providing isolation in FR2-1, implying that the probability of a blocker coming from the same direction is much lower than in FR1. Another consideration is that in FR1, the difference between the out-of-channel requirements like the ACLR and in-channel requirements like EVM is large. The PA linearity requirement is therefore dominated by out-of-channel requirements, e.g., ACLR. In FR2-1, these are at comparable levels. Spectral regrowth due to IM3 is dominant for in-channel requirements and as such, PA linearity requirements are rather driven by EVM and possibly in-band emissions. Another design challenge for DPD in FR2-1 is that PA characteristics must be carried through a feedback link from the output of the PA. In the case of mmWave, it is more difficult than in FR1 to create such a feedback link due to signal attenuation. Therefore, it is significantly more challenging to exploit DPD in FR2-1 such as done for FR1.

Despite these design challenges for gNB-side SBFD operation, our FR2-1 26 GHz testbed measurement results in Figure 16 show that 28 dBc leakage ratio between DL and UL subband (or component carriers) are still possible, e.g., similar to ACLR as existing out-of-channel requirement for FR2-1.

Observation 6: 28 dBc subband leakage ratio between the SBFD DL and UL subband when using non-overlapping frequency resources can be achieved in FR2-1

Figure 16: FR2-1 testbed and PSD for SBFD DL and UL subbands after antenna isolation and filtering

Additional design aspects

Frequency-domain separation to achieve a 45 dBc leakage ratio in FR1 benefits from the presence of a few RBs guard-band, e.g., 5 RBs, between the SBFD DL and UL subband. Since DPD is affected not only by the non-linear PA characteristics but also by noise caused from memory effects and PA temperature, absence of guard RBs is not meaningful to assume. In addition, presence of a few guard RBs between the DL and UL subband helps to balance the interference power per subcarrier in the UL subband which helps digital SIC performance.

Time-domain digital SIC can be used to further reduce the number of guard RBs. For example, if there is sufficient spatial isolation such that no ADC or LNA saturation in the Rx path occurs, the residual amount of interference power per subcarrier does not impact the digital SIC performance with time-domain SIC. Instead of using the frequency-domain digital SIC after FFT, time-domain filter taps which adapt according to the self-interference channel estimation can be applied to the Tx signal and remove the estimated leakage signal from the time-domain received signal. Using time-domain SIC, the guard-band size does not impact the digital SIC performance. The guard band can be smaller or the gNB radio unit can even operate without any guard band between the SBFD DL and UL subband.

Figures 17 (FR1) and 18 (FR2-1) show the achievable performance using time-domain SIC in our FR1 3.5 GHz and FR2-1 26 GHz testbeds respectively. Note that no guard band between the SBFD DL subband and UL subband was used.

Observation 7: Both in FR1 and FR2-1, SBFD can operate with only a few guard RBs between DL and UL subband when sufficient spatial isolation is guaranteed

Another consideration is that the desired received signal is mixed with the undesired DL leakage signal in the Rx path of the gNB radio, e.g., after ADC. The unwanted DL leakage signal must be removed by receiver processing using digital SIC. It is necessary to estimate the interference channel between the Tx panel and the Rx panel. Digital SIC performance is helped when synchronization to accurately remove the Tx signal from the Rx signal can be obtained. In principle, two methods exist to estimate the interference channel. One approach is to store information on a Tx signal that has passed through the PA with a feedback link and then estimate the interference channel over-the-air to remove the interference from the Rx signal. Another approach is to use only over-the-air estimation. Without a feedback link, the whole combined channel can still be estimated through the Rx panel. We used the first approach in the FR1 3.5 GHz testbed and the second approach in the FR2-1 26 GHz testbed.

As we observed, both approaches can effectively estimate the interference channels and eliminate undesired Tx-to-Rx interference. Based on the measurement results in the testbeds, multiple interference channels can be removed simultaneously. No more than 0.9 dB noise rise (INR) was measured in our FR1 testbed. In the case of FR2-1, noise rise performance of 0.7 dB for 1T1R and 1 dB for 2T2R configurations were measured as shown in Figures 17 and 18.

Observation 8: Digital SIC to remove Tx-to-Rx interference in the Rx path results in a noise rise of 0.9dB for SFBD in FR1

Observation 9: Digital SIC to remove Tx-to-Rx interference in the Rx path results in a noise rise of 0.7 dB with 1T1R and 1 dB with 2T2R panel configurations for SBFD in FR2-1

Figure 17: FR1 testbed and Rx signal after digital SIC for INR < 0.9 dB

Figure 18: FR2-1 testbed and Rx signal after digital SIC for 1T1R (left) and 2T2R (right)

To prevent ADC saturation in the Rx path of the gNB radio unit supporting SBFD, Rx filtering can be used to suppress the leakage from the Tx side interfering signal. Additional Rx filters can provide protection to avoid potential dynamic range and saturation issues for ADC or LNA when demodulating the UL subband in the Rx path of the gNB. Note that for RF filters with sharp roll-off’s, the order of the filter must increase, and so must then the size of the filter. Additional insertion losses are incurred which negatively affect the link budget. Additionally, analog filters such as IF and BB filters can be employed. For example, when the receiver is designed to use zero IF architecture, the receiver can use the lowpass filter to further remove the leakage signal after applying the mixer. By combining multiple LNAs, filter loss can be compensated more easily.

Observation 10: Additional Rx filtering can be applied for FR1 and FR2-1 receivers to increase robustness of the gNB Rx path with respect to ADC and LNA dynamic range without incurring undue insertion losses

FR1 and FR2-1 testbed performance

We show end-to-end performance results for SBFD based on our FR1 and FR2-1 testbeds in Table 1 and Table 2. Note that FR2-1 results were obtained using an outdoor test environment.

In the case of SBFD in FR1 n78, performance is compared for 1-slot PUSCH transmission without DL interference for the legacy TDD case and 5-slots repeated PUSCH transmissions with DL interference for the first 4 slots (Table 1). For PUSCH transmission, MCS index 4 is selected and PUSCH transmitted over a 20 MHz UL subband placed in the center of the 100 MHz channel BW. When all repeated PUSCH transmissions are combined by the gNB receiver, an SNR gain of 6.1 dB is observed. Note that the FR1 testbed used the fully implemented SIC capability.

In the case of SBFD in FR2-1, we tested throughput performance over 4x100 MHz carriers. 1 CC among 4 CCs is used for the SBFD UL transmissions from the UE. PUSCH uses MCS index 24. For the legacy TDD case, only one FR2-1 CC is used for the PUSCH transmission without DL self-interference. Despite DL leakage somewhat degrading the UL receiver performance, we observe that SBFD using SIC allows for 4.13x throughput gain compared to legacy TDD in our FR2-1 testbed.

Table 1: FR1 testbed subband configuration for PUSCH and SNR performance

| Uplink throughput | TDD | SBFD |
| --- | --- | --- |
| SBFD Subband setting | DDDDU (100 MHz) | DDDD-- (40 MHz) UUUUU (20 MHz) DDDD-- (40 MHz) |
| SNR gain using repetitions | 0 dB | 6.1 dB |

Table 2: FR2-1 testbed CC configuration for PUSCH and throughput performance

| Uplink throughput | TDD | SBFD |
| --- | --- | --- |
| SBFD carrier setting | DDDD-- (4th CC, 100MHz) DDDDU (3rd CC, 100MHz) DDDD-- (2nd CC, 100MHz) DDDD-- (1st CC, 100MHz) | DDDD-- (4th CC) UUUUU (3rd CC) DDDD-- (2nd CC) DDDD-- (1st CC) |
| Throughput gain with new transmission | 75 Mbps (1x) | 310 M bps (4.13x) |

gNB power consumption

When evaluating the feasibility and performance of SBFD schemes for Local Area (LA), Medium Range (MR) and Wide Area (WA) base station classes in the Rel-18 SID NR duplex evolution, gNB power consumption must be considered as part of the feasibility analysis.

The SBFD implementation approach selected by a network vendor may affect the observed gNB power consumption profile during sustained system operation in a variety of ways, e.g., RFFE components, gNB scheduling, and site deployment aspects.

The direct Tx-Rx self-leakage link budgets for LA, MR and WA BS classes should be considered first when analyzing the need for higher linearity Tx RFFE or more RF components in the gNB implementation. In the case of the LA BS class, FR1 antenna isolation of 75 – 80 dB or more avoids Rx desensitization. There is little need for additional SI mitigation measures. The MR BS class somewhat more benefits from increased Tx linearity but relying on Tx-side analog/digital DPD or Rx-side digital SIC are meaningful design alternatives while preserving existing gNB implementations. In both cases, we do not expect significantly impacted gNB power consumption incurred by the RFFE components. WA BS classes are more challenging not only because of the larger Tx-Rx self-leakage budget, but also due to the very high Tx power regime of the PAs. In the Rx path, analog filtering may be required.

We would like to point out however, that even for the WA BS class, expected hardware and power consumption impacts when implementing SBFD are highly dependent on the vendor baseline. For example, the complexity of the analog/digital DPD component does not necessarily scale linearly with the number of Tx and Rx ports, respectively, depending on the selected SBFD implementation. Pre-distortion to increase the linearity in the self-interference channel(s) can be implemented with respect to the Rx ports only to reduce complexity. Power consumption then scales accordingly. Similar considerations apply to the eventual need of higher linearity or high-power filtering components. If the existing baseline implementation uses a PA supporting ACLR = -45 dBc for the 100 MHz channel bandwidth to meet out-of-band requirements, and the vendor design already maintains the -45 dBc leakage power ratio even for the in-band case, then no additional components and no corresponding power consumption increase for PA and pre-distortion components is incurred.

When the base station implementation supports SBFD operation, the impact of gNB scheduling must be considered. SBFD in a slot uses a smaller DL transmission BW, e.g., 2x40 or 80 MHz than the TDD channel BW, e.g., 100 MHz in a DL-only slot. The Tx PA efficiency is increased with SBFD when compared to TDD due to less DC bias which in turn increases the linearity of Tx PA. The need to run pre-distortion and digital SIC components in the base station also depends on the DL-UL co-scheduling allocations. gNB power consumption is increased in slots with simultaneous DL and UL transmissions. SBFD DL slots scheduled for UL-only allocations may see decreased gNB power consumption.

Site deployment aspects such as the selected antenna and panel architecture when SBFD is deployed affect the gNB power consumption primarily through the number of TRXs. However, the use of antenna option 1 where half of the antenna panel is used for Tx when compared to the TDD baseline would result in less power consumption with SBFD. The use of antenna option 2 and 3 would result in a higher power consumption, but this increase can be considered equivalent to the deployment of a second antenna panel in the TDD baseline case.

In summary, gNB power consumption should be considered as part of the SBFD feasibility analysis required by the Rel-18 SID. When SBFD is implemented, the gNB power consumption profile can be affected by several contributing factors, e.g., gNB RFFE components, gNB scheduling, and site deployment aspects. Based on our results presented in this section for the FR1 and FR2-1 testbeds, we consider that gNB power consumption and complexity when implementing SBFD is very manageable. We also note that some of these such as the potential need for more high-linearity RFFE components highly depend on the existing vendor baseline and their selected SBFD implementation approach.

Observation 11: gNB power consumption aspects are considered in the SBFD feasibility analysis

Conclusion

In this contribution, we made the following observations and proposals:

- Observation 1: Backwards-compatibility for legacy UEs when SBFD is configured in the TDD cell can be achieved by using DDDSU in SIB1 tdd-UL-DL-ConfigCommon

- Observation 2: It cannot be assumed that SBFD using transparent mode when configuring DFFFU in SIB1 tdd-UL-DL-ConfigCommon and scheduling/configuring DL/UL transmissions for legacy UEs in the SBFD UL subband results in consistent UE behavior

Observation 3: 80 dB in FR1 and 87 dB in FR2-1 antenna isolation using spatial separation and RF barrier can be achieved

Observation 4: Stopgap performance of the RF barrier for FR1 100 MHz and FR2-1 100 MHz channel BW is feasible

Observation 5: 45 dBc subband leakage ratio between the SBFD DL and UL subband when using non-overlapping frequency resources with digital pre-distortion can be achieved in FR1

Observation 6: 28 dBc subband leakage ratio between the SBFD DL and UL subband when using non-overlapping frequency resources can be achieved in FR2-1

Observation 7: Both in FR1 and FR2-1, SBFD can operate with only a few guard RBs between DL and UL subband when sufficient spatial isolation is guaranteed

Observation 8: Digital SIC to remove Tx-to-Rx interference in the Rx path results in a noise rise of 0.9dB for SFBD in FR1

Observation 9: Digital SIC to remove Tx-to-Rx interference in the Rx path results in a noise rise of 0.7 dB with 1T1R and 1 dB with 2T2R panel configurations for SBFD in FR2-1

Observation 10: Additional Rx filtering can be applied for FR1 and FR2-1 receivers to increase robustness of the gNB Rx path with respect to ADC and LNA dynamic range without incurring undue insertion losses

Observation 11: gNB power consumption aspects are considered in the SBFD feasibility analysis

- Proposal 1: RAN1 should evaluate the potential impacts from SBFD when a TDD serving cell using SBFD is configured as PCell, SCell or SpCell for a legacy UE

- Proposal 2: Further study potential enhancements to CA-based SBFD operation in FR2-1

- Proposal 3: SBFD operation in SSB symbols is not supported.

- Proposal 4: Further study whether SBFD operation in DL symbols configured for CORESET#0 or scheduled for PDSCH carrying SI is supported or not.

- Proposal 5: Further study whether SBFD operation in DL symbols configured for TRS is supported or not.

- Proposal 6: At least for RRC_CONNECTED mode, the frequency location(s) of the DL subband(s) are explicitly indicated.

- Proposal 7: At least for RRC_CONNECTED mode, the frequency location(s) of the guard band(s) are implicitly determined by the UE as RBs not configured as either UL or DL subband(s).

- Proposal 8: For the SBFD-aware UE semi-statically configured with the UL subband in legacy D and F symbols,
-	UL transmissions within the configured UL subband are allowed in the symbol
-	UL transmissions outside the configured UL subband are not allowed in the symbol
-	DL receptions inside the configured UL subband are allowed in the symbol

- Proposal 9: At least for RRC_CONNECTED mode, the SBFD subbands are configured with respect to the CRB grid.

- Proposal 10: Frequency locations and time locations of SBFD subbands can be configured independently for a UE in RRC_CONNECTED mode and a UE in RRC_IDLE/INACTIVE modes (if supported).

- Proposal 12: Multi-slot PUSCH transmissions and PUCCH repetitions can be configured to only use the SBFD slots/symbols

- Proposal 13: RAN1 to study enhancements to SBFD operation for PDSCH resource allocation type 0

- Proposal 14: RAN1 to study enhancements to SBFD operation for PUSCH resource allocation type 1

- Proposal 15: RAN1 to study enhancements to resource allocation with SBFD operation for CSI-RS resource set configuration and CSI reporting

- Proposal 16: RAN1 to study potential benefits and specification impacts when using SBFD slots/symbols for random access

- References

- RP-222110 “Revised SID: Study on evolution of NR duplex operation”; RAN#97

- R1-2212044 “Dynamic and flexible TDD for NR duplex evolution”; Samsung