| 3GPP TSG-RAN WG1 Meeting #110bis-e Toulouse, France, November 14th – November 18th, 2022 |  | R1-2212147 |
| --- | --- | --- |

Agenda item:	9.14.3

Source: 	Qualcomm Incorporated

Title: 	Dynamic Switching between CP-OFDM and DFT-S-OFDM

Document for:	Discussion/Decision

# Introduction

This document discusses aspects related to dynamic waveform switching (DWS) between CP-OFDM and DFT-S-OFDM. In the previous RAN1 meeting, the following agreements were made on this topic:

In the following sections we discuss our views on the use cases and applicability of dynamic waveform switching, the signalling mechanism to enable DWS, and potential PHR enhancements that may be useful to facilitate DWS.

# Applicability of Dynamic Waveform Switching

In the previous meeting we agreed to specify DWS for DG-PUSCH scheduled by DCI Format 0_1 or 0_2. It was further discussed whether DWS should be enabled for DG-PUSCH scheduled by DCI Format 0_0 or not, but no agreements were made regarding this. DCI Format 0_0 serves as the fallback DCI and any dynamic behaviour associated with this format is certainly not desirable. Further, its best to avoid any change to this format’s size to accommodate DWS and this rules out the use of an explicit bit to indicate the selected waveform. Implicit switching of the waveform based on other fields in the DCI can be extended to DCI format 0_0 if applicable. We suggest that we wait for some additional progress on how we intend to support DWS for DCI Format 0_1 and 0_2 and then revisit extending this support for Format 0_0.

Proposal 1: Applicability of DWS to DG-PUSCH scheduled by DCI format 0_0 can be considered after the switching mechanism for DCI Formats 0_1 and 0_2 is agreed.

On the applicability of DWS for CG-PUSCH, we note the following points:

- A UE may be configured with up to 12 configured grants (limited by maxNrOfConfiguredGrantConfig-r16) per BWP. Both Type 1 and Type 2 configurations are included, and each configuration is given a unique index given by ConfiguredGrantConfigIndex-r16.Activation or deactivation of a Type 2 configured grant references this index.

- For each of the configuredGrantConfigs provided to a UE, the gNB is allowed to independently determine whether transform precoding is enabled or not. Thus, there can be a subset of configuredGrantConfigs where transformPrecoding is enabled, and another subset where it is not enabled.

- For Type 1 CG-PUSCH, the UE is provided information on number of RBs, precoder, and number of layers via RRC. Some of these parameters may be fundamentally incompatible with a DFT-S-OFDM waveform.

Given that a UE can be provided with up to 12 different grants, with the flexibility to independently configure the waveform for each of the grants, we do not see a strong need to extend DWS to configured grants.  Further Type 1 CG-PUSCH poses additional challenges as switching waveforms also entails changing other transmission parameters provided via RRC. For these reasons, we make the following proposal:

Proposal 2: DWS is not applicable to Type 1 CG-PUSCH.

The presence of up to 12 configured grants diminishes any strong motivation to extend DWS to Type 2 CG-PUSCH.  The gNB has full flexibility to activate/release configured grants based on the transmission requirements and this provides an opportunity to switch waveforms as well. Having said this, if the signaling mechanism introduced to support DWS for DG-PUSCH scheduled by DCI Format 0_1 or 0_2 can be extended in a straightforward manner to CG-PUSCH as well, then we are open to extending support for Type 2 CG-PUSCH as well. We make the following proposal in this regard:

Proposal 3: If dynamic waveform switching is to be supported for Type 2 CG-PUSCH, reuse the waveform switching framework developed for DG-PUSCH triggered by DCI Format 0_1 or 0_2.

Dynamic waveform switching for Msg3:

In the current specification, Msg3 waveform is determined by msg3-transformPrecoder. Some companies propose to also enable DWS for Msg3. Setting aside questions on the necessity/motivation, we are concerned that enabling this feature would require some indication of UE capability to support this feature. Early indication of a UE’s capability when it is not yet in connected state entails significant amount of work and it is probably best to avoid this in the absence of a strong motivation.

Proposal 4: DWS is not supported for Msg3 transmission and retransmission.

# Switching Mechanism

In the last meeting two options were listed for further discussion. The first option was to use the scheduling DCI to indicate the waveform type to use and the second option uses a non-scheduling DCI to indicate the waveform type to use.

Persistent vs. Non-persistent switching

This discussion can also be viewed as a choice between non-persistent waveform switching and persistent waveform switching. In the first case, the waveform type indicated via DCI is only applicable to that particular grant, while in the second case, the signalling is “sticky”, i.e., once a signalling is received to use a particular waveform that waveform type is used until another signalling is received that asks the UE to switch the waveform type.

A DCI-based approach to realize persistent waveform switching could potentially be error-prone as the UE may miss the DCI. UE and gNB will no longer be in sync in such a scenario, and it may take a while to recover from this.

For this reason, if we go with a DCI-based switching mechanism, a non-persistent switching behaviour is preferred. Although a practical use case to switch waveforms back and forth is not envisioned, the above guidance is driven by the overall robustness of the operation. Within Alt 1, an implicit determination of the waveform seems a natural approach to take as an explicit bit does not seem to be necessary.

Proposal 5: For DG-PUSCH scheduled by DCI Format 0_1 or 0_2, enable dynamic waveform signalling by reusing an existing field in the scheduling DCI (Alt 1-B). The waveform type only applies to the PUSCH transmission scheduled by the dynamic grant.

DCI Size determination

Assuming the scheduling DCI is used to indicate the waveform type to be used by a UE, the DCI contents are to be sized so that the UE can be indicated either the CP-OFDM or the DFT-S-OFDM waveform. To accommodate parameters tailored to either waveform, we make the following proposal to determine the DCI size:

Proposal 6: When dynamic waveform signalling is enabled, for bitfields whose bit width currently depends on the configured waveform type via the transformPrecoder parameter in RRC, the bit width is set to be the larger of the two values that results when transformPrecoder is enabled or disabled. The interpretation of the bitfield is dependent on the signalled waveform.

# PHR Enhancements for Aiding Dynamic Waveform Switching

In the last meeting, it was agreed to study potential enhancements to power headroom reporting to aid dynamic waveform switching. The goal of this objective is to aid the gNB to decide when to trigger a waveform switch based on how much additional power a UE can deliver using DFT-S-OFDM compared to CP-OFDM. There are two potential directions for enhancements that one could consider. The first is focused on trigger conditions that initiate the UE to report power headroom and the second is on the reporting structure itself.

Currently a UE triggers a PHR based on periodic timer or prohibit timer expiring, pathloss changing by a certain value, etc. To this list, it might be worthwhile to consider adding trigger conditions that get activated anytime a UE may be getting close to its maximum power, i.e., power headroom level falls below a threshold. These trigger conditions can be viewed by the gNB as a subtle indication from the UE to consider switching the waveform.

The second line of enhancements that might be worth considering is to make changes to the reporting structure and modify it appropriately so that the report carries the desired information for both waveforms. Currently a UE reports power headroom and maximum power (Pcmax) associated with a waveform. A possible option is to modify the current reporting framework to let the UE report two sets of values for power headroom and Pcmax assuming the same underlying transmission parameters.

Proposal 7: Enhancements to power headroom reporting to assist that scheduler in dynamic waveform switching focuses on the following two aspects:

- New trigger conditions to report power headroom, for e.g., UE triggers a PHR whenever power headroom falls below a certain threshold.

- Enhanced reporting structure, for e.g., UE reports power headroom and Pcmax for each of the waveforms

# Conclusion

We make the following observations and proposals for dynamic switching between CP-OFDM and DFT-S-OFDM.

Proposal 1: Applicability of DWS to DG-PUSCH scheduled by DCI format 0_0 can be considered after the switching mechanism for DCI Formats 0_1 and 0_2 is agreed.

Proposal 2: DWS is not applicable to Type 1 CG-PUSCH.

Proposal 3: If dynamic waveform switching is to be supported for Type 2 CG-PUSCH, reuse the waveform switching framework developed for DG-PUSCH triggered by DCI Format 0_1 or 0_2.

Proposal 4: DWS is not supported for Msg3 transmission and retransmission.

Proposal 5: For DG-PUSCH scheduled by DCI Format 0_1 or 0_2, enable dynamic waveform signalling by reusing an existing field in the scheduling DCI (Alt 1-B). The waveform type only applies to the PUSCH transmission scheduled by the dynamic grant.

Proposal 6: When dynamic waveform signalling is enabled, for bitfields whose bit width currently depends on the configured waveform type via the transformPrecoder parameter in RRC, the bit width is set to be the larger of the two values that results when transformPrecoder is enabled or disabled. The interpretation of the bitfield is dependent on the signalled waveform.

Proposal 7: Enhancements to power headroom reporting to assist that scheduler in dynamic waveform switching focuses on the following two aspects:

- New trigger conditions to report power headroom, for e.g., UE triggers a PHR whenever power headroom falls below a certain threshold.

- Enhanced reporting structure, for e.g., UE reports power headroom and Pcmax for each of the waveforms

# Reference

- RP-221858, Revised WID on Further NR Coverage Enhancements, 3GPP TSG RAN Meeting #96, RAN1 103-e, Budapest, Hungary, June 6-9, 2022