3GPP TSG RAN WG1 Meeting #111	R1-2212905

Toulouse, France, November 14th – 18th, 2022

Source:	Moderator (Samsung)

Title:	Feature lead summary #5 evaluation of AI/ML for beam management

Agenda Item:  9.2.3.1

Document for:	 Discussion and Decision

- Introduction

In RAN#94-e, Rel-18 new study item on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” was approved. The following use cases were identified as the initial set:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

The performance of AI/ML based algorithms for the use cases includes the following aspects:

- Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set:

  - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations.

    - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed.

    - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study.

    - Need for common assumptions in dataset construction for training, validation and test for the selected use cases.

    - Consider adequate model training strategy, collaboration levels and associated implications

    - Consider agreed-upon base AI model(s) for calibration

    - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes

  - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases.

    - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline

    - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

In this contribution summarized the discussions and proposal on evaluation methodology (EVM) and KPIs from contributions submitted to AI 9.2.3.1 for beam management (BM).

In the coming RAN 1 meeting, the issues tagged as FL5 can be the focus of the discussion.

FL1: Question 0

- Please consider entering contact info below for the points of contact for this email discussion.

| Company | Point of contact | Email address |
| --- | --- | --- |
| Ericsson | Henrik Ryden | Henrik.a.ryden@ericsson.com |
| vivo | Hao Wu | hao.wu@vivo.com |
| Samsung | Jackson Wang Yeon-Geun Lim | h0809.wang@samsung.com yg.lim@samsung.com |
| CATT | Da Wang | wangda@catt.cn |
| Huwei, HiSilicpn | Thorsten Schier | Thorsten.schier@huawei.com |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| Qualcomm | Hamed Pezeshki | hamedp@qti.qualcomm.com |
| OPPO | Jianfei (Jeffrey) Cao | caojianfei@oppo.com |
| Lenovo | Srinivas Kothapalli | vkothapalli@lenovo.com |
| MediaTek | Gyubum Kyung Yu-Jen Ku | gyubum.kyung@mediatek.com yu-jen.ku@mediatek.com |
| Xiaomi | Mingju LI | limingju@xiaomi.com |
| ZTE | Wenfeng Liu | liu.wenfeng@zte.com.cn |
| CMCC | Jun Zuo | zuojun@chinamobile.com |
| NTT DOCOMO | Haruhi Echigo Liu Liu | haruhi.echigo.fw@nttdocomo.com liul@docomolabs-beijing.com.cn |
| LG Electronics | Haewook Park | haewook.park@lge.com |
| CAICT | Xiaofeng Liu | Liuxiaofeng1@caict.ac.cn |
| Lenovo | Srinivas Kothapalli | vkothapalli@lenovo.com |

General evaluation assumptions

The following proposals were proposed by companies:

| Company | Proposals |
| --- | --- |
| Google [6] | Proposal 1: Add BS height = 10m as a second option as evaluation assumption to be aligned with evaluation assumption in other agenda items and to create more beams for indoor UEs in vertical domain. Observation 1: CIR based spatial domain beam prediction outperforms the L1-RSRP based beam prediction. Observation 2: UE orientation could not help to increase the accuracy for L1-RSRP based beam prediction. Observation 3: The ML input with measurement error could cause significant performance degradation. |
| Ericsson [9] | Proposal 1 Evaluate more percentiles for the L1-RSRP error (e.g. 95th,99th percentile) Proposal 2 For beam prediction evaluations consider providing the results with measurement accuracy noise modelled as additive gaussian noise with 95% of the density function within the measurement accuracy range, and/or uniformly distributed noise Proposal 3 Study the impact of measurement imperfections on model performance for the considered beam prediction use cases. Proposal 4 Consider the following to mitigate the L1-RSRP measurement inaccuracy impact in ML based beam prediction a. RAN4 to explore possibility to tighten requirements on L1-RSRP measurement accuracy b. Define different UE capability based on their capability in fulfilling a measurement accuracy requirement. |
| Interdigital[10] | Proposal 14: For traffic model, support the following evaluation assumptions: - For beam information related KPIs, no traffic model is needed to be defined as UE is only measuring reference signals not decoding actual PDSCHs. - For system performance related KPIs, FTP traffic should be used to reflect practical traffics for the evaluation. - For FTP traffic model, FTP model 3 is preferred as generating a new UE for each packet (FTP model 1) is not appropriate for evaluating benefits from AI/ML based beam prediction. Proposal 15: For UE distribution, support the following evaluation assumptions: - For FTP traffic model, 10 UEs per cell/sector with 50% and 70% RUs is preferred. - 80% outdoor UEs and 20% indoor UEs for spatial domain beam prediction as defined in TR 38.901 (Option 1) Observation 11: AI aided beam selection achieves more than 95% selection accuracy when error margin is larger than 0.5 dB by consuming 50%/33% of the measurement overhead for the exhaustive measurement. Observation 12: AIML-based RSRP estimation always outperforms the baseline especially when less RSRP measurements are available as it achieves a higher selection accuracy by 35% when error margin is 0.5 dB. |
| Intel [12] | Proposal 5: For system performance KPIs, if supported, only full-buffer traffic models should be used. Proposal 8: For initial evaluations fix UE orientation towards the direction of motion. Proposal 9: Spatially consistent large-scale parameter generation should be used for mobility evaluations. Additionally, only spatial consistency model B in [4] can be used for mobility evaluation. Proposal 10: The UE trajectory should be sampled at least at the minimum decorrelation distance of the large-scale parameters corresponding to the scenario of evaluation. |
| Qualcomm [23] | Proposal 3: For BM-Case2, consider the scenario in which the UE orientation changes as a function of UE trajectory. - FFS: details of this function Observation 1: At least for BM-Case2, AI/ML-based methods will provide an advantage in high-stress scenarios where frequent UE orientation changes lead to rapid changes in the best beams. Observation 2: For BM-Case2 with high UE rotation speeds, the AI/ML-based method (LSTM) strongly outperforms the sample-and-hold baseline, especially in the UE Rx beam prediction and Tx-Rx beam pair prediction use cases. - The rapid rotation leads to significant changes in best-beam RSRPs between measured cycles; the LSTM is able to predict for these changes, while the sample-and-hold scheme breaks down. |
| MTK [24] | Proposal 7: Study and evaluate the performance of AI/ML beam prediction using the dataset generated by the ray-tracing simulations. |

From feature lead point of view, there is no urgency to discuss the above proposals in this meeting.

FL5: (FFS) Question 1.1

FL5: As pointed out by some companies, companies are encouraged to study the impact with L1-RSRP error, (measurement/quantization error) in next meeting.

KPIs on AI/ML in beam management

  - Beam prediction accuracy related KPIs

2.1.1 (Medium)Clarification on beam predication accuracy

Agreement - The options to evaluate beam prediction accuracy (%): - Top-1 (%): the percentage of “the Top-1 genie-aided beam is Top-1 predicted beam” - Top-K/1 (%): the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams” - Top-1/K (%) (Optional): the percentage of “the Top-1 predicted beam is one of the Top-K genie-aided beams” - Where K >1 and values can be reported by companies. Agreement - For DL Tx beam prediction, the definition of Top-1 genie-aided Tx beam considers the following options - Option A, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams - Option B, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) - FFS on specific Rx beam(s) - Note: specific Rx beams are subset of all Rx beams Agreement - For DL Tx-Rx beam pair prediction, the definition of Top-1 genie-aided Tx-Rx beam pair considers the following options: - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams - Option B: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx over all Tx beams with specific Rx beam(s) - FFS on specific Rx beam(s) - Note: specific Rx beams are subset of all Rx beams

Some observations/proposals were made in the contributions on beam prediction accuracy related KPIs:

| Company | Proposals/observations |
| --- | --- |
| Huawei/HiSi [2] | Proposal 4: For DL Tx beam (pair) prediction, the definition of Top-1 genie-aided Tx beam (pairs), Option B, the specific Rx beam means that it is one fixed Rx beam out of the available Rx beams during one round of inference. In our view, the “specific Rx beam” means that one fixed Rx beam is used for DL beam (pair) prediction during one round of inference. The obtained DL beam (pair) represents then the best Tx beam for this given Rx beam (pair). This definition, is more suitable to be used in our view when the AI/ML model infers the Top-K Tx beams rather than for inferring the best DL Tx-Rx beam pairs. |
| Vivo [3] | We prefer to prioritize Option A for the definition of Top-1 genie aided Tx beam or Tx-Rx beam pair. Option B can be additionally considered as an optional intermediate KPI with specific Rx beam assumption not being the best Rx beam. |
| ZTE [4] | Observation 1: If Top-K beams are predicted by the adopted AI model, a refined beam sweeping over these Top-K predicted beams may be needed to obtain an optimal beam, which is up to NW. Proposal 2: In the beam pair prediction, it’s better to take measurement results of all Rx beams as the model input instead of performing any sampling at the Rx beam space. |
| Fujitsu [5] | Proposal 3: For DL Tx-Rx beam pair prediction, the Top-1 genie-aided Tx-Rx beam pair is defined as the Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams. |
| CATT [7] | Proposal 1: For DL Tx-Rx beam pair prediction, Option A is selected for the definition of Top-1 genie-aided Tx beam, i.e., the Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams. Proposal 2: For DL Tx beam prediction, Option B is selected for the definition of Top-1 genie-aided Tx beam, i.e., the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s). - The specific Rx beam is one DL Rx beam selected from all Rx beams, which specific Rx beam can be reported by the company. |
| Spreadtrum [8] | Proposal 2: The definition of Top-1 genie-aided beam: - For DL Tx-Rx beam pair prediction, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s); - For DL Tx-Rx beam pair prediction, the Top-1 genie-aided Tx-Rx beam pair is the Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams. |
| Intel [12] | Proposal 6: For DL Tx and DL Tx-Rx beam pair prediction, the definition of top 1 genie aided beams considers the best UE beam on the best panel as the specific Rx beam |
| OPPO [13] | Proposal 3: For DL Tx beam prediction, adopt Option B in which Top-1 genie-aided Tx beam is selected over all Tx beams with specific Rx beam(s). Proposal 4: For DL Tx-Rx beam prediction, adopt Option A in which Top-1 genie-aided Tx-Rx beam pair is selected over all Tx and Rx beams. |
| Lenovo [17] | Proposal 11 Definition of Top-1 genie-aided Tx beam for DL Tx beam prediction should depend on the measurement model. Select “Option A” only if the measurement model allows Tx beam measurements across all Rx beams. If such is not the case and the RSRP of Tx beams can be measured for only specific Rx beam(s), then “Option B” should be adopted. Proposal 12 For DL Tx-Rx beam pair prediction, select “Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams” as the definition for Top-1 genie-aided Tx-Rx beam pair. |

Summary of the views from companies

| Case | Option A | Option B |
| --- | --- | --- |
| DL Tx beam prediction | Lenovo (if allows measurements from all Rx beams), vivo, OPPO (training??), LG, CAICT (upper bound), xiaomi, Google, Mediatek, Huawei/HiSi | Huawei/HiSi, CATT, Spreadtrum, OPPO (inference??), Lenovo (measurement from specific Rx beams) , vivo (Optional) , LG(optional), CAICT (practical), InterDigital, , Fujitsu, ZTE, Xiaomi |
| Tx-Rx beam pair prediction | Vivo, ZTE, Fujitus, CATT, Intel, OPPO, Lenovo, LG, CAICT(upper bound),xiaomi, Google, Mediatek, Fujitsu, Huawei/HiSi | Huawei/HiSi, vivo(optional), Intel, LG (optional), CAICT (practical),xiaomi, InterDigital, |

FL2: (closed) Question 2.1.1

After feature lead has the following question:

- If we go for Option B for either prediction case, isn’t this can only represent sub-optimal result? Don’t we also want to know how far AI/ML can go?

- If we go for Option B, e.g., for DL Tx beam prediction, if the “Top-1 genie-aided Tx beam” is with specific Rx beam(s), does this imply that during data collection phase, AI/ML only collects the data for specific Rx beam(s) (during training phase to generate cost function), and we only expect AI to provide the results based on the specific Rx beams(s)? Otherwise, if AI collects data for all Tx/Rx beams, why not use option A? Further, if only collected measurements from specific Rx beam(s), why only declares UE only has “specific Rx beam(s)”

Please share your views on the options and questions above:

| Company | Comments |
| --- | --- |
| OPPO | For DL Tx beam prediction, during training phase, we think Option A makes much more sense in picking up the best Tx beams among all Tx an Rx beams. For this part, we agree with the assessment from FL. But when comes to inference phase, the DL Tx beam prediction should go with Option B with specific Rx beam(s). Otherwise, we cannot find clear boundary between Tx beam prediction and Tx-Rx beam pair prediction. FL2: This is for evaluation. I cannot get the point for training phase. training methodology is up to companies. |
| Apple | It may not be feasible to assume all the panels are active simultaneously, hence requiring exhaustively examining all the Rx beams may not be practical. The current agreement already allows Option A, we are not sure why Option A should be the only option. |
| LG | Although our preference is Option A, we are also ok with option B as optional. |
| CAICT | Option A is the upper bound for DL Tx beam(pairs) prediction. Option B consider the practical limitation. |
| vivo | We think at least Opt A should be the baseline for evaluating prediction accuracy, as the final purpose of beam management is to get the best Tx-Rx beam pair. However, Opt B can only serve a quite specific use case of finding a sub-optimal beam. Hence Opt B can be optionally used if the goal of the EVM is just to get a sub-optimal beam with a specific Rx beam. Further, this is KPI discussion, which is used to compare different schemes. If we use different KPIs for DL Tx beam prediction and Tx-Rx beam pair prediction, what KPI should we use in the evaluation to compare these two schemes? |
| Xiaomi | From our point of view, separate model will be used for DL Tx beam prediction and DL Tx-Rx beam pair prediction. For DL Tx-Rx beam pair prediction, we prefer Option A. While for DL Tx beam prediction, both Option A and Option B can be studied with separate model. It means that with Option B, only data for specific Rx beam(s) can be collected. And for Option A, data for all Tx/Rx beams should be collected. |
| InterDigital | We support option B for both DL Tx beam and DL Tx beam pair. |
| Google | Option B is not practical in real implementation. We think option A is better. UE can perform Rx beam tracking based on different SSB symbols. Although the actual UE does not always search all Rx beams, it still performs certain Rx beam sweeping for SSB reception. |
| CATT | We think the main different between Tx beam prediction and beam pair prediction is Tx beam prediction is based on the specific Rx beam, i.e., Option B, while beam pair prediction is based on all Tx and Rx beams. Otherwise, there is no different between the Top-1 genie-aided Tx beam for Tx beam prediction and the Tx beam of Top-1 genie-aided beam pair for beam pair prediction. |
| MediaTek | We prefer Option A. The used Rx beam for both training and inference is up to UE’s implementation. Take gNB side model as an example, gNB will not know the “specific Rx beam” that UE is using. Therefore, it is better to define Top-1 genie-added beam based on all Tx and Rx beams. |
| Fujitsu | For DL Tx-Rx beam pair prediction, we prefer option A. For DL Tx beam prediction, it may correspond to P1/P2 stage of hierarchical beam sweeping. The specific Rx beam can be considered an Omni wide beam on UE. Option B is our preference for DL Tx beam prediction We think different prediction cases can consider different option. |
| HW/HiSi | I think here was a misunderstanding what we meant in the citied proposal. We do not prefer Option B over Option A. What we meant with the proposal is how to resolve the FFS “specific beam” for Option B. We think that for the DL Tx beam prediction, no down-selection between Option A and Option B should be done. Which Option to take depends on the implementation, i.e. if all RX beams are considered for inference or only one specific (and then the the best RX beam is found in P3). For beam pair prediction it should be Option A in our view. |
| Spreadtrum | For DL Tx beam prediction, it is unfair if option A is used because the Top-1 predicted beam may never be the Top-1 genie-aided beam, and the evaluation of the accuracy of the whole system may have a great impact |
| ZTE | For DL Tx beam prediction, we support option B. The AI model can be used to predict the optimal Tx beam under a fixed Rx beam, which is consistence with the P2 beam refinement procedure. This can be taken as the optimal result under certain Rx beam instead of sub-optimal result. For DL Tx-Rx beam pair prediction, both options can be further studied. Besides, the specific Rx beam(s) in both agreements need to be clarified and aligned across companies. |
| FL2 | Preference from companies is listed in the table before. Some offline discussion is needed. In my view, we don’t need to care what is practical. The KPI is just to evaluate how much gap from the best. For example, MSE for channel estimation, no one can get idea channel in real word, but MSE is still used to measure the performance of an algorithm. I am quite surprised that companies commented on Opt A is not practical. |
| Xiaomi | Just to correct our views in the table. |
| Samsung | We share the same view of FL2. We think that the purpose of to define this KPI is what is the optimal performance gain from AI/ML-based prediction. Thus, we prefer Option A as baseline. |
| Intel | The goal of this study item should be evaluating the gain/loss of AI/ML both with respect to a baseline as well with upper bounds which can be considered ideal or genie-aided schemes. This can show how good the AI/ML model performance can be with respect to ideal as well as realistic assumptions. Therefore, in our view both Option A and B have value as KPTs. On FL’s question, we think during the performance evaluation phase, we need to collect data on all beams and then use Option A (ideal) and Option B (more realistic) to benchmark model performance. |

FL5: (FFS) Question 2.1.1

We hadn’t discussed this issue in this meeting. However, I think it would be good to clarify a little bit. From my point of view, this KPI is just used in evaluation. To better understand the gap to idea/ genie-aided beam/RSRP. However, from companies’ input, I feel like there may have some mis-understanding with loss function or something else, since some companies consider what can be obtained in practical. For current L1-RSRP based/accuracy based KPIs, I don’t think we can obtain genie-aided Top K beam or idea L1-RSRP either. In my understanding, the reference should be the best of best, i.e., Top 1 beam among all Tx-Rx pairs.

In next meeting, companies are encouraged to share the views on this issue.

2.1.2 (closed)Predicted L1-RSRP

Some observations/proposals were made in the contributions on beam prediction accuracy related KPIs:

| Company | Proposals/observations |
| --- | --- |
| Huawei/HiSi [2] | Observation 2: The 2 potential AI/ML-model outputs are the probability of the best beam IDs and the predicted L1-RSRP values. • The already agreed KPI for the L1-RSRP difference between the ideal L1-RSRP of the predicted beam and the ideal L1-RSRP of the genie-aided best beam is applicable when the AI/ML-model is inferring the probability/best beam ID. • The not yet agreed KPI of the L1-RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 genie-aided beam is applicable when the AI/ML-model is inferring the predicted L1-RSRP values. |
| Fujitsu [5] | Proposal 2: To evaluate the performance of predicted L1-RSRP, further study the following options: - Opt 1: The L1-RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 predicted beam - Opt 2: The L1-RSRP difference between the predicted L1-RSRP of Top-1 genie-aided beam and the ideal L1-RSRP of Top-1 genie-aided beam |
| Xiaomi [11] | Proposal 2: To evaluate the performance of predicted L1-RSRP, other than existing KPI of L1-RSRP difference (ideal RSRP Diff to genie-aided beam), further consider additional KPI as: - (Diff to genie-aided beam): The L1-RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 genie-aided beam when the Top-1 predicted beam is same or different from the Top-1 genie-aided beam. Observation 2: the average L1-RSRP difference between the ideal L1-RSRP of Top-1 genie-aided beam and the predicted L1-RSRP of the Top 1 predicted beam of AI based beam prediction in spatial domain with fixed set B is less than 0.5 dB, even considering both the case of Top 1 predicted beam is same as Top 1 genie-aided beam and the case of Top 1 predicted beam is different from Top 1 genie-aided beam. |
| OPPO [13] | Proposal 5: Support another definition of L1-RSRP difference • The difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam |
| China Telecom [14] | Observation 1: Modelling the spatial beam prediction task as a classification model provides better performance with less training overhead. |
| Samsung [22] | Proposal # 4: No need to define new KPI for predicted L1-RSRP unless the benefit with reported L1-RSRP is verified. |
| MTK [24] | Proposal 1: To evaluate the performance of predicted L1-RSRP, other than existing KPI of L1-RSRP difference (ideal RSRP Diff to genie-aided beam), further consider additional KPI as: • L1-RSRP difference (Diff to genie-aided beam): The L1-RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 genie-aided beam • Other options are not precluded and can be reported by companies. |

Summary of the views:

- No need to define additional KPI:

  - Supported by: Samsung, [China Telecom]?, Futurewei, LG, DoCoMo, Interdigital, Google, CATT, Huawei/HiSi

- Support to define additional KPI:

  - Opt 1: The L1-RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 predicted beam

    - Supported by: Fujitsu, MTK, OPPO, vivo, Spreadtrum, Ericsson

  - Opt 2: The L1-RSRP difference between the predicted L1-RSRP of Top-1 genie-aided beam and the ideal L1-RSRP of Top-1 genie-aided beam

    - Supported by: Fujitsu

  - Opt 3: (Diff to genie-aided beam): The L1-RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 genie-aided beam when the Top-1 predicted beam is same or different from the Top-1 genie-aided beam.

    - Supported by: Xiaomi

More view needs to be collected, pleases consider the following questions from feature lead:

FL1: (closed) Question 2.1.2a

- Whether this is predicted L1-RSRP difference is needed/beneficial for system performance point of view?

- If this is needed, for gNB side AI model or for UE side AI model, or both?

- In FL’s view, this KPI is not applicable for classification model. Do we need to compare the performance between different type of AI models?

- Which option do you support?

Please share your views on the options and questions above:

| Company | Comments |
| --- | --- |
| OPPO | Maybe it’s due to a copy-and-paste error, we re-capture our preference under Opt. 1, rather than Opt.2 in summary above. To Q1: assume the AI/ML model at UE side, UE may report the predicted L1-RSRP of predicted Top-K beam for NW to be aware of the link quality of predicted beam(s). Therefore, it seems reasonable to evaluate the L1-RSRP gap between the predicted beam and genie-aided beam as in Opt.1. To Q2: for evaluation purpose, this L1-RSRP difference can be applicable for both UE-side and model. To Q3: we also understand this KPI is applicable for linear regression model and surely, we cannot compare two types of AI models (i.e. one for beam prediction and one for L1-RSRP prediction of predicted beam). But it seems okay to compare the predicted L1-RSRP of predicted beams with that of genie-aided beams. One more question is that if predicted L1-RSRP is to be reported by UE (for the case of UE-side model), should we evaluate this L1-RSRP difference? Or we only adopt the L1-RSRP difference already agreed? |
| Futurewei | Q1: there is no need to add “predicted L1-RSRP difference” for beam prediction purpose. The goal is to predict the Top-1/K beam(s) or beam pair(s) correctly to achieve close to optimal system performance. Prediction accuracy of L1-RSRP cannot indicate the performance of Top-1/K prediction accuracy. Q2: If L1-RSRP is part of the prediction outcome for either NW-side or UE-side model, companies may define proper mechanism(s) to verify the accuracy and how to verify the agreed-upon intermediate KPI(s), e.g., Top-1/K prediction accuracy. Q4: We support “No need to define additional KPI”. |
| LG | No need additional KPI such as “predicted L1-RSRP difference” |
| NTT DOCOMO | We support ‘No need to define additional KPI’. Option 1~3 oblige the AI/ML model to output the L1-RSRP with similar range/distribution as that of the actual L1-RSRPs. |
| vivo | Q1: Yes Q2: Both Q3: It seems not needed to compare different models. Q4: We support Opt 1. It can provide extra information of RSRP difference compared with the one we have agreed, which is the difference between the ideal RSRP of the best predict beam and the RSRP of the best gene-aided beam. Opt 2 is more similar with the one we have agreed. |
| Xiaomi | Q1: Yes, the predicted L1-RSRP is beneficial for system performance. Since in legacy beam report, both RS ID and the corresponding L1-RSRP is reported. That means the L1-RSRP is essential to be reported for gNB to know the link quality of each beam and know the gap between different beams. So predicted L1-RSRP should be supported as model output. In order to evaluate the accuracy of the predicted L1-RSRP, it is better to define a L1-RSRP difference between predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 genie-aided beam. Q2: both Q3: From our point of view, this KPI is only needed for AI model with predicted L1-RSRP as model output. |
| InterDigital | Q1. Not needed. Q3. No need to define different type of AI models as the type of AI model is a part of implementation. Q4. We support no additional KPI. |
| Google | Q1: No. |
| CATT | Q1: Not needed. |
| MediaTek | Q1: Yes Q2: Both Q3: We agree that this KPI is not applicable for classification model. If we want to evaluate a classification model, we can use the RSRP difference KPI that has already been agreed. If we want to evaluate a regression model, we can use this new KPI. Q4: To measure the system performance, we need to use Opt2. |
| Fujitsu | Q1: Yes Q2: only UE-side model Q3: if classification model is evaluated, the beam prediction accuracy can be used as KPI. |
| HW/HiSi | We don’t think it is really needed. But if some companies want to study it as output, that is fine for us. Not sure how to use it as a KPI. |
| Spreadtrum | Q1: Yes Q2: Both Q3: No need to compare different AI models. Q4: We support Opt 1. The agreed KPI can only provide the difference between 2 ideal RSRP which can not reflect the accuracy of prediction RSRP. In terms of prediction, RSRP is the main output of the AI model Therefore, it is indispensable to evaluate the RSRP accuracy of the output. For option2, the comparison is neither on the same beam nor under the same condition (predicted/ideal), so the KPI is of little significance. |
| ZTE | Q1: Yes. The predicted RSRP represents the corresponding link quality and can guide the final beam selection at gNB over the predicted top-K beams. Besides, the reporting of RSRP is already supported in current spec. Q2: This KPI is benefit for both side AI model. Q3: This KPI is only applicable for regression model and whether to use a regression model or a classification model is up to implementation. |
| Ericsson | Q1: Yes Q2: Both Q3: Share the view that it is not needed to compare different types of AI/ML model. We should be open to understand the potential also for regression models. Q4: Use option 1 as a starting point. |

FL2: (closed) Question 2.1.2b

Proposal 2.1.2a (as conclusion)

- The predicted L1-RSRP difference is L1-RSRP difference the between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 predicted beam

| Company | Comments |
| --- | --- |
| FL2 | As a compromise, we can define what is “predicted L1-RSRP difference” first so that companies can report it if beneficial. But we don’t define this as KPI for now. |
| Xiaomi | Support in principle and suggest the following update Proposal 2.1.2a (as conclusion) - The predicted L1-RSRP difference is L1-RSRP difference the between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 predicted beam - Companies are encouraged to report it if predicted L1-RSRP is an AI/ML model output. |
| Samsung | We are okay with Proposal 2.1.2.a as conclusion. |
| FL3 | Proposal 2.1.2b (as conclusion) - The predicted L1-RSRP difference is L1-RSRP difference the between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 predicted beam - Companies are encouraged to report it if predicted L1-RSRP is an AI/ML model output. |
| NTT DOCOMO | Support. It could be useful for model monitoring to check the L1-RSRP prediction accuracy for the regression AI/ML models. But no need to define it as KPI in EVM because it does not affect the system performance. |
| Futurewei | As concluded in today’s online session, this conclusion is not adopted. If any company’s model predicts L1-RSRP may determine how to evaluate the prediction accuracy and there is no need to define/specify it. When reporting performance for BM, we should still use the agreed-upon intermediate KPIs. |

2.1.3 Others

Some observations/proposals were made in the contributions on beam prediction accuracy related KPIs:

| Company | Proposals/observations |
| --- | --- |
| Huawei/HiSI [2] | Proposal 15: For temporal beam prediction evaluation, results for Top-K, K>1 should be presented in addition to Top-1 results. - The Top-1 predicted beam can be derived as the eventual result after the second round sweeping based on the AI/ML inferred Top-K beams. |
| INTERDIGITAL [10] | Observation 2: Beam information related KPI is not a measure to evaluate actual performance benefits from AI/ML based beam prediction, however, beam information related KPI can be used as a temporal measure to have an insight as beam information related KPI requires less computational overhead for evaluation. Observation 3: Specification enhancements should be justified based on actual system performance benefits such as throughputs not based on beam related KPIs such as L1-RSRP difference. Proposal 7: Support beam information related KPIs as optional for temporal measures. - Support average L1-RSRP difference of Top-1 predicted beam in addition to the agreed options for beam prediction accuracy (%). |
| OPPO [13] | Proposal 2: For beam prediction accuracy, adopt the metric of Top-K/1 (%). |
| China Telecom [14] | Proposal 1: For calibration purpose or performance comparison, the open-source dataset with aligned simulation assumption is encouraged to be provided by companies. |
| APPLE [18] | Proposal 1: The KPI for AI based beam prediction could be the beam prediction accuracy and the L1-RSRP distribution for the AI predicted beam. The KPI with RSRP can be used for making decision/drawing conclusion in the whole Rel-18 study item. |
| CMCC [15] | Proposal 2: The definition of beam prediction accuracy (%) with 1 dB margin for Top-K beams is: The percentage of “the ideal highest L1-RSRP of the predicted Top-K beams is within 1 dB of the L1-RSRP of the Top-1 genie-aided beam” |

2.2 System performance related KPIs

2.2.1 (low priority) User throughput

Several companies mentioned that the system performance shall be also evaluated:

| Company | Proposals/observations |
| --- | --- |
| InterDigital [10] | Proposal 3: Support system performance related KPIs as mandatory KPIs. - Support Avg. and 5% UE tput for system performance KPIs. |
| OPPO [13] | Observation 9: When beam prediction accuracy is high (at least 80%) and L1-RSRP difference is small (within 1 dB), the system-level performance, i.e. spectrum efficiency or throughput, may only provide non-essential insight, therefore focusing on L1-RSRP for beam prediction would be good enough. |
| Qualcomm [23] | Proposal 5: At least for BM-Case1, consider spectral efficiency CDF for SLS evaluations as a KPI. |
| MTK [24] | Proposal 6: To evaluate the system level throughput performance of AI/ML beam management, both average user throughput and cell edge user throughput need to be reported. |

2.2.3 (low priority) UCI report

In RAN 1 #110, the following agreement on UCI report was agreed.

Agreement - To evaluate the performance of AI/ML in beam management at least for NW side beam prediction, UCI report overhead can be further studied as one of KPI options. - FFS: number of UCI reports and UCI payload size

The following was discussed in contributions:

| Company | Proposals/observations |
| --- | --- |
| Vivo[3] | UCI reporting overhead reduction, including the number of UCI report and UCI payload size, should be considered as basic KPI. RRC singling overhead can be considered as optional KPI if huge amount of data, such as training data, assistant information, and AI model data, is exchanged via RAN air interference. |
| INTERDIGITAL[10] | Observation 7: UCI report overhead is not a metric for evaluation, but assumption for evaluation. |
| Lenovo[17] | Proposal 8 Reporting overhead should be considered as a KPI. Consider the number of UCI reports and the size of each UCI report (in bits) as a measure of the reporting overhead. |
| DoCoMo[21] | Proposal 2: Consider the number of transmissions for UCI and UCI payload size as KPI: |
| MTK[24] | Proposal 5: To define the UCI report overhead, first discuss the number of UCI reports and how the report is quantized. |

2.2.2 RS overhead/RS overhead reduction

2.2.2.1 For BM Case1

There were several proposals/discussions related to RS overhead[reduction] for BM-Case1:

| Company | Proposals/observations |
| --- | --- |
| Huawei/HiSI[2] | Observation 5: For the evaluation of the overhead for BM-Case1, for reporting RS overhead reduction - Option A1: For a given scheme, it does not take the overhead of second round beam measurements into account and it relates the overhead reduction to the size of Set A of the given scheme. A fair performance/overhead comparison between schemes with different sizes of Set A is not possible. - Option A2: For a given scheme, it can take all beams that are needed for measurement (Set B and Top-K) into account. The overhead reduction is related to the size needed for measurements of a baseline scheme. A fair performance/overhead comparison between schemes with different and same sizes of Set A is possible. - Option A3: For a given scheme, it can take all beams that are needed for measurement (Set B and Top-K) into account. But it relates the overhead reduction to the size of Set A of the given scheme. A fair performance/overhead comparison between schemes with different sizes of Set A is not possible. Observation 6: For the evaluation of the overhead for BM-Case1, for reporting RS overhead (Option B) - Option 1: Only counts the beams that are needed as input to the AI/ML model. Which is misleading, since with different settings of Top-K the performance and total overhead is significantly impacted - Option 2: Gives the information about the full overhead that is needed to obtain the best beam. Proposal 10: For the evaluation of the overhead reduction for BM-Case1, - Support Option A2: RS OH reduction%=1-NM - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme - Companies report the assumption on beam sweeping - If one of Options A1 or A3 is additionally supported, then for reporting also the RS overhead according to Option B2 (Alt.1) should be reported |
| Vivo [3] | Both two overhead metrics, i.e. RS overhead reduction and RS overhead, can be supported for different evaluation purpose in BM-Case1 and BM-Case2. Support RS OH reduction%=1-N+PM and RS OH = N + P for the evaluation of the overhead for BM-Case1. |
| ZTE [4] | Proposal 1: RS overhead reduction should be considered as a basic KPI for evaluation and can be defined as 1-N/M, where N is the number of beams (or beam pairs) for measurement and M is the number of beams (or beam pairs) for prediction. |
| CATT [6] | Proposal 4: For the KPI of RS overhead for BM-Case1, Option 1 is preferred for RS overhead reduction: - Option 1: RS OH reduction%=1-NM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement - where M is the total number of beams (pairs) to be predicted. Proposal 5: For the evaluation of the overhead for BM-Case1, RS overhead reduction is enough. |
| Spreadtrum [8] | Proposal 3: For RS overhead or RS overhead reduction, option 1 should be considered as KPI for spatial domain beam prediction. |
| InterDigital [10] | Observation 4: The motivation of Option 3 (RS OH reduction%=1-N+PM) for RS overhead reduction is not clear there’s no need to support additional beam sweeping only for AI/ML based beam prediction. Observation 5: Although RS overhead reduction can be defined by using number of beams by assuming AI/ML based beam management and baseline use same RE overheads perbeam, number of beams does not reflect actual RS overheads. Observation 6: RS overhead is not a metric for evaluation, but assumption for evaluation. Proposal 4: Approve the working assumption on the evaluation of the RS overhead without Option 3 for Option A and Option B. Proposal 6: Define RS overhead by using actual RE overheads not using number of beams. |
| Xiaomi [11] | Proposal 1: For the evaluation of the overhead for BM-Case1, support Option 1 in Option A. |
| OPPO [13] | Proposal 6: For RS overhead reduction [%] of BM-Case1, adopt 1-N/M (Option 1) if there is no following up beam sweeping procedure. Proposal 7: For RS overhead reduction [%] of BM-Case1, adopt 1-(N+P)/M (Option 3) if there is following up beam sweeping procedure • Adopt P (Alt.1) is the number of Top-K selected beams (pairs) for beam sweeping (if applicable) |
| Lenovo [17] | For RS overhead reduction, “exhaustive search”, where we search across all the M beams (beam-pairs) for the best beam (beam-pair), should be the baseline. With exhaustive search as the baseline, adopt the following definition for the RS overhead reduction KPI: "RS overhead reduction"=1-1/M ( 1/τ ∑_(i=1)^τ▒N_i ), where N_i is the number of beam measurements in i^th time slot, τ is the total number of time slots required for beam prediction and M is the total number of beams (beam-pairs) |
| LGE [19] | Proposal 4. For KPI/metric related to overhead, only option A is supported. Among the sub-options, option A-1 is preferred. |
| DoCoMo [21] | Proposal 1: Option 3 in Option A and Option 2 in Option B should be selected as the overhead estimation methods. - The selection of alternatives in Option 3 or Option 2 depends on the targeted scenarios where the BM-Case1 is used for. |
| Samsung [22] | Proposal # 5: Replace the working assumption with the following: - For the evaluation of the overhead for BM-Case1, RS overhead reduction is considered as one KPI for evaluation, defined by the following options: - Option 1: RS OH reduction%=1-NM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement - where M is the total number of beams (pairs) to be predicted - Option 2: RS OH reduction%=1-NM - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme - Companies report the assumption on beam sweeping |
| MTK [24] | Proposal 2: For the evaluation of the RS overhead for BM-Case1: - RS overhead reduction, FFS for potential down selection: - Option 1: RS OH reduction%=1-NM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement - where M is the number of beams (pairs) to be predicted - Option 2: RS OH reduction%=1-NM - where N is the total number of measurements required (with reference signal (SSB and/or CSI-RS)) for all the beam sweeping procedures (P1 and/or P2/P3) - where M is the number of measurements (with reference signal (SSB and/or CSI-RS)) required for Option 1 of the baseline scheme - Companies report the assumption on beam sweeping procedures |
| Nokia/NSB [25] | Observation 20: In BM-Case1 DL Tx- Rx beam pair prediction, training model with fixed beam number in random Set B outperforms the training model with varied beam number in random Set B. Observation 21: In BM-Case1 DL Tx-Rx beam pair prediction, training model with random Set B has a nonnegligible performance drop compared training model with fixed Set B. And large K for top-K beam search is necessary for maintaining the performance. Proposal 9: RAN1 further investigates the performance comparison and overhead reduction between DL Tx beam prediction and DL Tx-Rx beam pair prediction. |

Summary of the proposals:

|  |  | Supported by |
| --- | --- | --- |
| Option A: RS overhead reduction | Option 1 | ZTE, CATT, Spreadtrum, InterDigital, xiaomi, OPPO, LGE, Samsung, MTK (9) |
|  | Option 1’ - RS overhead reduction=1-1M 1τi=1τNi, - where Ni is the number of beam measurements in ith time slot, τ is the total number of time slots required for beam prediction and M is the total number of beams (beam-pairs) | Lenovo |
|  | Option 2 | Huawei/HiSi, InterDigital, Samsung, MTK(5) |
|  | Option 3 | Vivo, OPPO(Alt1), DoCoMo(3) |
| Option B RS overhead | Option 1 | Spreadtrum, InterDigital (2) |
|  | Option 2 | Vivo, InterDigital, DoCoMo (3) |

Majority companies (9) support option A-1. In feature lead’s view, if the output is Top1 Tx-Rx beam pairs, option A-1 is enough. If this is for DL Tx beam prediction and output is Top 1 DL Tx beam, since different companies may have different assumption on Rx beam sweeping, option A-1 does not include Rx beam sweeping.

For option 2 (5 companies) and option 3 (3 companies), in feature lead’s view, option A-2 can cover option A-3. We can further clarify that N is the total number of beams(pairs) requires for AI/ML, “including the beams(pairs) for beam sweeping.”. If we go for option A-3, another round of discussion is needed to down select between alternatives.

If option 2 is agreed, because M already provides the beams required by baseline schemes, it can already provide enough information on the RS overhead. Therefore, in FL’s view, there is no need to report RS overhead.

FL1:(closed) Question 2.2.2.1

Proposal 2.2.2.1a

- For the evaluation of the overhead for BM-Case1, adoption the following metrics:

  - RS overhead reduction,

    - Option 1: RS OH reduction%=1-NM

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement

      - where M is the total number of beams (pairs) to be predicted

    - Option 2: RS OH reduction%=1-NM

      - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams(pairs) for beam sweeping

      - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme, including the beams(pairs) for beam sweeping

      - Companies report the assumption on beam sweeping

  - RS overhead is not precluded and can be reported by companies

Please share your views on proposal 2.2.2.1a:

| Company | Comments |
| --- | --- |
| Futurewei | We support Option 1. |
| LG | We support Option 1. |
| NTT DOCOMO | We support Option 2 to include the possible additional measurements by beam sweeping after the prediction. |
| CAICT | Support FL’s proposal and Option 2 is acceptable. |
| vivo | Fine with the proposal |
| Xiaomi | We think for Option 1 and Option 2, it will be much better to update as below: - Option 1: RS OH reduction%=1-NM - Where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML input - …… - Option 2: RS OH reduction%=1-NM - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML input, including and the beams(pairs) for beam sweeping based on the AI/ML model output after the AI/ML model inference - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme, including the beams(pairs) for beam sweeping based on the AI/ML model output after the AI/ML model inference - Companies report the assumption on beam sweeping FL: for your comment for option 1 as well as the first comment for option 2, I think there is another case, as mentioned im 4.3, that UE may measure more beams while only reports a subset, e.g., 4 beams. the measurement of the reported 4 beams are used as AI inputs. Therefore, current wording may be more inclusive. |
| InterDigital | We support both Option 1 and Option 2 and one of the options can be used and reported by each company. In addition, as we clarified, RS overhead should be reported as evaluation assumption not as a metric for the evaluation. Having said that, we propose to delete the following bullet. RS overhead is not precluded and can be reported by companies |
| Google | Support option 1 |
| CATT | We support Option 1. |
| MediaTek | We can keep both Option1 and 2. |
| Fujitsu | Support option 2. |
| HW/HiSi | We support option 2. To make it crystal how the N and are M understood, we suggest a slight modification of the description: - Option 2: RS OH reduction%=1-NM - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams(pairs) for beam sweeping of the inferred top-K beams - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme, including the beams(pairs) for beam sweeping of the inferred top-K beams - Companies report the assumption on beam sweeping - RS overhead is not precluded and can be reported by companies We think if Option 1 is used, then RS overhead should be reported as well. The reason is to be able to compare the overhead between schemes with different size of Set A. For Opton 1 that would otherwise not be possible in our view. Even if our preference is Option 2, we are fine to keep both Option 1 and Option 2 and let companies report. |
| FL | Proposal 2.2.2.1b - For the evaluation of the overhead for BM-Case1, adoption the following metrics: - RS overhead reduction, - Option 1: RS OH reduction%=1-NM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement - where M is the total number of beams (pairs) to be predicted - Option 2: RS OH reduction%=1-NM - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) for beam sweeping after the prediction if applicable - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme, including the beams (pairs) for beam sweeping after the prediction if applicable - Companies report the assumption on beam sweeping - RS overhead is not precluded and can be reported by companies |
| Spreadtrum | Support option 1 |
| ZTE | Both options can be reported by companies. |
| FL | Agreement - For the evaluation of the overhead for BM-Case1, adoption the following metrics: - RS overhead reduction, - Option 1: RS OH reduction%=1-NM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML - where M is the total number of beams (pairs) to be predicted - Option 2: RS OH reduction%=1-NM - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme, including the beams (pairs) required for additional measurements before/after the prediction if applicable - Companies report the assumption on additional measurements |

2.2.2.2 For BM-Case2

There were several proposals/discussions related to RS overhead[reduction] for BM-Case2:

| Company | Proposals/observations |
| --- | --- |
| Huawei/HiSI [2] | Proposal 12: For the evaluation of the overhead for temporal domain AM//ML-based BM, the observation and prediction window are jointly considered, and two metrics should be reported - The RS overhead, consisting of the beams being swept in Set B during observation and the Top-K beams for P-2 beam sweeping during prediction (if applicable) - RS OH=t1=1T1N+t2=1T2K for K>1 and RS OH=t1=1T1N for K = 1 - The RS overhead reduction compared to an exhaustive beam sweep over Set A during observation and the Top-K beams for P-2 beam sweeping during prediction (if applicable) - RS OH RD %=1-t1=1T1N+t2=1T2K t=1T1+T2M for K > 1 and RS OH RD %=1-t1=1T1N t=1T1+T2M for K = 1 - Where: M is beams in Set A of the baseline, N is beams in Set B and K is the number of beams as inference output |
| Vivo [3] | Support RS OH reduction%=1-t1=1T1N+t2=1T2P t=1T1+T2M and RS OH=t1=1T1N+t2=1T2Pfor the evaluation of the overhead for BM-Case2. |
| Xiaomi [11] | Proposal 5: Study the following options on RS overhead reduction for temporal beam prediction with set B is same as Set A: - Option 1: RS OH%=1-NN+M - For the case of the same periodicity of history measurement instance and future time instance, where N is the number of history measurement instance and M is the number of predicted future time instance. - Option 2: RS OH%=1-1L - For the case of the periodicity of history measurement instance is L times of that of future time instance. |
| OPPO [13] | Proposal 8: For BM-Case2, study how to capture the overhead reduction, considering the T1 duration (measurement on Set B) and T2 duration (prediction among Set A). |
| DoCoMo [21] | Proposal 3: Consider the number of RS overhead (reduction) independently for Pattern 1 and Pattern 2 with the calculation methods proposed in Table 3. Proposal 4: Adopt Pattern 1 as well as Pattern 2 as the baseline assumption for BM-Case 2. |
| Samsung [22] | Proposal # 6: For the evaluation of the overhead for BM-Case2, RS overhead reduction is considered as one KPI for evaluation, defined by the following options: - Option 1: RS OH reduction %=1-t1=1T1Nt1 t=1T1+T2Mt - where Nt1 is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement (in Set B) in each time instance t1=1…T1. - where Mt is the total number of beams (pairs) to be predicted (in Set A) in each time instance t=1…T1+T2 - Option 2: RS OH reduction%=1-t1=1T1Nt1 t=1T1+T2Mt - where Nt1 is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each time instance t1=1…T1. - Where Mt is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme in each time instance t=1…T1+T2 (if applicable) - Companies report the assumption on beam sweeping |
| MTK [24] | Proposal 3: For the evaluation of the overhead for BM-Case2, further study the following two metrics: - RS overhead reduction, FFS for potential down selection: - Option 1: RS OH reduction %=1-NM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement - where M is the number of beams (pairs) to be predicted - Option 2: RS OH reduction %=1-t1=1TN t=1TM - where N is the total number of measurements required (with reference signal (SSB and/or CSI-RS)) for all the beam sweeping procedures (P1 and/or P2/P3) in each slot of T - where M is the number of measurements (with reference signal (SSB and/or CSI-RS)) required for Option 1 of the baseline scheme in each slot of T - Companies report the assumptions on beam sweeping procedures and T (e.g. T includes observation and/or prediction windows) - Other options can be reported by companies Proposal 4: For QCL relation overhead, first study and list the scenarios when such QCL relation overhead exists, then discuss how to define the KPI. |

FL2:(closed) Question 2.2.2.2a

Proposal 2.2.2.2a

- For the evaluation of the overhead for BM-Case2, adoption the following metrics:

  - RS overhead reduction,

    - Option 1: RS OH reduction %=1-t1=1T1N t=1T1+T2M

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement (in Set B) in each slot of T1 for AI/ML

      - where M is the total number of beams (pairs) to be predicted (in Set A) in each slot of both T1 and T2

    - Option 2: RS OH reduction%=1-NM

      - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - Companies report the assumption on additional measurements

Please share your views on proposal 2.2.2.2a:

| Company | Comments |
| --- | --- |
| FL2 | In FL’s understanding, Option 2 for BM-Case1 can also apply to BM-Case2. |
| Xiaomi | As commented before, these two options can only be reused for scenario#1 (In the figure below) for BM-Case 2. It means that we can consider N history measurement instance and M future instance as an unit. And we suggest the following update for these two options in order to differentiate Option 1 and Option 2. Actually, in Option 1, N is the number of beam (pairs) required for measurement for input. Maybe N is larger than the number of beam (pairs) in set B and we just try to clarify that N does not include beam (pairs) for additional measurements. but Option 2, it is for AL/ML scheme, to differentiate with baseline scheme. Proposal 2.2.2.2a - For the evaluation of the overhead for BM-Case2, adoption the following metrics: - RS overhead reduction, - Option 1: RS OH reduction %=1-t1=1T1N t=1T1+T2M - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement (in Set B) in each slot of T1 for AI/ML input - where M is the total number of beams (pairs) to be predicted (in Set A) in each slot of both T1 and T2 - Option 2: RS OH reduction%=1-NM - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML scheme, including the beams (pairs) required for additional measurements before/after the prediction if applicable - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme, including the beams (pairs) required for additional measurements before/after the prediction if applicable - Companies report the assumption on additional measurements But for scenario#2 (in the figure below), these two options can’t be reused. It means that beam for short periodicity will be predicted based on the measurements with long periodicity. In this scenario, if the periodicity of future instance is 1/L of history measurement instance, the RS overhead is 1-1L for set A= set B. while for other cases (set A is not same as set B), the RS overhead is 1-NL*M, where M the total number of beams (pairs) to be predicted (in Set A) and N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement (in Set B) in each slot for AI/ML input. |
| Samsung | In Option1, some notations are missed, so we suggest to modify it as follows: - Option 1: RS OH reduction %=1-t1=1T1N t=1T1+T2Mt1=1T1Nt1 t=1T1+T2Mt - where N Nt1 is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement (in Set B) in each slot of T1 for AI/ML time instance t1=1…T1 - where M Mt is the total number of beams (pairs) to be predicted (in Set A) in each slot of both T1 and T2 instance t=1…T1+T2 For Option 2, we are fine with FL’s proposal. |
| FL3 | check Proposal 2.2.2.2b - For the evaluation of the overhead for BM-Case2, adoption the following metrics: - RS overhead reduction, - Option 1: RS OH reduction %=1-t1=1T1Nt1 t=1T1+T2Mt - where Nt1 is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement (in Set B) in each time instance t1=1…T1 for AI/M - where Mt is the total number of beams (pairs) to be predicted (in Set A) in each instance t=1…T1+T2 - Option 2: RS OH reduction%=1-NM - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme, including the beams (pairs) required for additional measurements before/after the prediction if applicable - Companies report the assumption on additional measurements - Option 3: RS OH%=1-NLM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each time instance - where M is the total number of beams (pairs) to be predicted for each time instance - where L is ratio of periodicity of time instance for measurements to of periodicity of time instance for prediction |
| NTT DOCOMO | Since the prediction is not applied for the baseline, there is no need to mention additional measurements before/after the prediction for the baseline. Hence, suggest to update the Option 2 as follows. - Option 2: RS OH reduction%=1-NM - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme, including the beams (pairs) required for additional measurements before/after the prediction if applicable - Companies report the assumption on additional measurements FL3: first of all, we already agree the same wording for BM-Case1. Secondly, whether additional measurements are needed or not depend on which baseline to choose. Thirdly, there is a wording “if applicable”. I don’t see strong motivation to change it. |

FL3:(closed) Question 2.2.2.2b

Proposal 2.2.2.2b

- For the evaluation of the overhead for BM-Case2, adoption the following metrics:

  - RS overhead reduction,

    - Option 1: RS OH reduction %=1-t1=1T1Nt1 t=1T1+T2Mt

      - where Nt1 is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement (in Set B) in each time instance t1=1…T1 for AI/M

      - where Mt is the total number of beams (pairs) to be predicted (in Set A) in each instance t=1…T1+T2

    - Option 2: RS OH reduction%=1-NM

      - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - Companies report the assumption on additional measurements

    - Option 3: RS OH%=1-NLM

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each time instance

      - where M is the total number of beams (pairs) to be predicted for each time instance

      - where L is ratio of periodicity of time instance for measurements to of periodicity of time instance for prediction

Please share your views on proposal 2.2.2.2b:

| Company | Comments |
| --- | --- |
| FL3 | Please check the updates on Option 3 proposed by Xiaomi for BM Case 2. |
| Xiaomi2 | Support and try to provide more explanation for option 3. Option 3 is used for the scenario in the following figure. Which means the beam for short periodicity will be predicted by the measurement results with long periodicity. So when considering the RS overhead reduction, we only need to take one long periodicity as an unit. It doesn’t mean the model input only contains measurement results of only one long short periodicity. It is because that the window (containing history measurement instance and future time instance) in the following figure can slide in time. it means the measurement result for the last long periodicity can also be considered as model input for the next round inference, which is different from the other scenario that the window can’t slide (since the predicated beam information can’t be considered as model input). For example, if long periodicity is 400ms, short periodicity is 100ms (i.e., L=4), set B contains 8 beams, set A contains 32 beams, so the RS for AI/ML scheme will be only 8 beams (i.e., set B for one long periodicity, N ) and the RS for baseline scheme will be 32*4 (set A for 4 short periodicity, LM). i.e., the RS overhead reduction= 1-NLM |
| NTT DOCOMO | We support to include Option 3 for evaluating the RS overhead in BM-Case 2. In addition, since both Set B = Set A and Set B < Set A are considered in the evaluation, we think some additional measurements after the prediction would also be required. Therefore, we would like to modify the description of Option 3 as follows: - Option 3: RS OH%=1-NLM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each time instance before prediction and the beams (pairs) required for additional measurements after the prediction if applicable - where M is the total number of beams (pairs) to be predicted for each time instance - where L is ratio of periodicity of time instance for measurements to of periodicity of time instance for prediction |
| MediaTek | We are OK with Option3, only small typo: (1) “RS OH” should be “RS OH reduction” and (2) “where L is ratio of periodicity of time instance for measurements to of periodicity of time instance for prediction”. We are also wondering if Option3 can be used for the case when measurement after prediction is required. For Option2, we think it is better to specify that both N and M are the total measurement required for all the time instances in T1 and T2, especially now we have Option3 which doesn’t count all the measurement in T1. Thus, we suggest the following, - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in T1+T2, including the beams (pairs) required for additional measurements before/after the prediction if applicable - where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme in T1+T2, including the beams (pairs) required for additional measurements before/after the prediction if applicable |
| Samsung | Regarding Option 3, M should include the total number of beams in T1 to compare baseline scheme. Also, L should be larger than 1 (in our understanding L should be the number of predictions but not sure the intention of Xioami). We think Xioami may provide the better formula of Option 3. Regarding Option 1 and Option 2, when the historical measurements in T1 are reused to predict multiple/consequent T2 of future time instance, the overhead reduction may be miscalculated based on current formula. We suggest to add some clarifications. - Option 1: RS OH reduction %=1-t1=1T1Nt1 t=1T1+T2Mt - where Nt1 is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement (in Set B) in each time instance t1=1…T1 for AI/ML - where Mt is the total number of beams (pairs) to be predicted (in Set A) in each instance t=1…T1+T2 - where T2 is determined by considering multiple/consequent predictions if applicable - Option 2: RS OH reduction%=1-NM - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme, including the beams (pairs) required for additional measurements before/after the prediction if applicable - Companies report the assumption on additional measurements - Companies report the assumption on T1 and T2 patterns |
| ZTE | We are fine with option 3. Besides, there is a typo in option 1 that set B in the bracket should be deleted. As has been clarified before, set B is used as model input, which does not need to be the same as the beam set for measurement. Thus, we suggest a same wording for BM-Case1. - RS overhead reduction, - Option 1: RS OH reduction %=1-t1=1T1Nt1 t=1T1+T2Mt - where Nt1 is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement (in Set B) in each time instance t1=1…T1 for AI/ML - where Mt is the total number of beams (pairs) to be predicted (in Set A) in each instance t=1…T1+T2 |
| LG | We are fine with option 3 and editorial changes from Samsung and ZTE. |
| Ericsson | According to offline discussion. Remove option 1. Propose the following change to option 2. - Option 2: RS OH reduction%=1-NM - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme, including the beams (pairs) required for additional measurements before/after the prediction if applicable - Companies report the assumption on additional measurements - Companies to report the assumption on baseline scheme |

FL4:(closed) Question 2.2.2.2c

Proposal 2.2.2.2c (as working assumption)

- For the evaluation of the overhead for BM-Case2, adoption the following metrics:

  - RS overhead reduction,

    - Option 2: RS OH reduction%=1-NM

      - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme, including the beams (pairs) required for additional measurements if applicable

      - Companies report the assumption on additional measurements

    - Option 3: RS OH reduction%=1-NLM

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each time instance

      - where M is the total number of beams (pairs) to be predicted for each time instance

      - where L is ratio of periodicity of time instance for measurements to periodicity of time instance for prediction

      - Companies report the assumption on T1 and T2 patterns

    - Other options are not precluded and can be reported by companies.

| Companies | comments |
| --- | --- |
| HW/HiSi | Option 3 does not seem to cover the case that more than 1 beam is inferred for each predicted time instance (i.e. Top-K is inferred with K >1). However, for this case we have performed simulation results and would like to describe its overhead reduction. For measurement in the observation phase, at each of the T1 instances, N beams are measured. And the model output gives K beams for each of the T2 prediction instances. At each prediction instance the K beams are measured to identify the best Tx beam. This would add to the total overhead. If M still is assumed as in option 3, the overhead reduction could be described as below, Option 3’: RS OH reduction%=1-N+K-1LM If K=1, Option 3’=Option 3 and if K > 1, the overhead reduction would become less, which is reasonable, since additional beams have to be measured during each prediction instance. It would be great to hear more views on the above proposal, it could accommodate both the Option 3 when the AI model deliver the top-1 beam and also the situation when it infers top-K beams. |
| FL | Agreement - For the evaluation of the overhead for BM-Case2, adoption the following metrics: - RS overhead reduction, - Option 2: RS OH reduction%=1-NM - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme - Companies report the assumption on additional measurements - FFS: Option 3: RS OH reduction%=1-NLM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each time instance - where M is the total number of beams (pairs) to be predicted for each time instance - where L is ratio of periodicity of time instance for measurements to periodicity of time instance for prediction - Companies report the assumption on T1 and T2 patterns - Other options are not precluded and can be reported by companies. |

FL5:(FFS) Question 2.2.2.2c

In FL’s view, as explained by Xiaomi, the following case is valid. And if we consider a sliding window, counting ratio of periodicity might be enough, at least when AI/ML tries to predict Top 1 beam. However, I also understand that companies who treat Top K (K>1) as their use case may think for opt 3, additional beam sweeping is also needed. Please share your view on this.

Moreover, based on current opt 1 for BM-Case1 and opt 3 for BM-Case2, if best Rx beam is assumed to obtain the measurement, the beam sweeping to obtain the best Rx beam is not counted.  In my view, Opt 2 can be used for both sub-use cases. However, if there is good equation to represent it. We can have some further discussion. Otherwise, we can let companies to choose a proper option to report for RS overhead.

2.2.3 (Medium) Other KPIs

Latency reduction

- Lenovo [17]

  - Latency Reduction = 1- Total transmission time of N beamsTotal transmission time of M beams

  - Proposal 4	Consider Beam Prediction Accuracy, Overhead Reduction and Latency Reduction as the key KPIs in evaluating an AI/ML model for beam management and consider adopting the definitions proposed above.

FL5:(FFS) Question 2.2.3

We haven’t opened the discussion for latency reduction yet. If we can have some clarification or discussion in section 4.7. I think we can also have some definition of KPI on latency reduction. This can be treated after we agreed on 4.7.

2.4 (Low priority) Baseline performance

FL5: we can have some further discussion during observations discussion phase.

Some companies provided some analysis on baseline performance for benchmark.

| Company | Proposal/observation |
| --- | --- |
| Huawei/HiSi [2] | Proposal 2: For evaluation of DL Tx sweeping, the following two baselines can be considered: - Lower performance bound: A lower performance bound obtained by non-AI/ML-based legacy sparse beam sweeping with the same overhead as the AI/ML-based approach - Upper performance bound: Exhaustive beam sweeping over all available beams Proposal 3: For evaluation of DL Tx-Rx beam pair sweeping, the following two baselines can be considered: - Baseline 1: Non-AI/ML solution with legacy Tx beam sweeping plus legacy Rx beam sweeping, to justify the performance gain over non-AI/ML. - Baseline 2: AI/ML-based DL Tx beam prediction plus the legacy Rx beam sweeping, to justify the performance gain over DL Tx beam prediction. |
| INTERDIGITAL [10] | Observation 1: Legacy beam management with Rel-17 without AI/ML algorithms is not an appropriate baseline as implementation-based AI/ML operation is available for UE and gNB implementations. Proposal 1: ‘No collaboration framework: AI/ML algorithms purely implementation based and not requiring air-interface changes’ could be an appropriate baseline to accurately evaluate the benefits of AI/ML with specification enhancements. |
| Intel [12] | Proposal 4 For baseline performance evaluation, Option 2 should correspond to hierarchical beam search where, based on sub-use case being evaluated, set B may be a subset of set A or set B can contain both wide and correlated narrow beams. |
| OPPO [13] | Proposal 9: For spatial domain beam prediction, select the best beam within Set A via exhaustive beam sweeping (Option 1) as baseline. Proposal 10: For temporal domain beam prediction, select the best beam for T2 within Set A via exhaustive beam sweeping (Option 1a) as baseline. |
| Lenovo [17] | Proposal 10 Adopt “Option 1: Select the best beam within Set A of beams based on the measurement of all RS resources or all possible beams of beam Set A (exhaustive beam sweeping)" as the baseline for spatial beam prediction. |

AI/ML model Generalization

Generalization is one of the important aspects to verify the performance of AI/ML model.

Working Assumption The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point: - Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Note: Companies to report the ratio for dataset mixing - Note: number of the multiple scenarios/configurations can be larger than two - FFS the detailed set of scenarios/configurations - FFS other cases for generalization verification, e.g., - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

3.1 (High)EVM for generalization

The follow discussions/proposals were summarized:

| Company | Proposals/observations |
| --- | --- |
| Futurewei [1] | Proposal 1: Adopt Case 2A as a separate evaluation option for model generalization as follows: For the evaluation of the potential performance benefits of model fine-tuning of beam management use case, which is optionally considered by companies, the following case is taken: - The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B - Company to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance Note: the above has been agreed during the discussion of CSI feedback enhancement use case. Proposal 2: For model generalization evaluation and result collection, reuse the attributes included in Table X as starting point (Table X was agreed as working assumption from RAN1#110bis-e [2]) while companies should report the attributes for training scenario/configuration and inference scenario/configuration separately. Proposal 3: For model generalization evaluation and result collection, further include the following attributes in the evaluation result report template in addition to those specified in Table X from RAN1#110bis-e [2]: - Deployment scenarios in training and inference (e.g., UMa, UMi, or others) - Outdoor/indoor UE distributions in training and inference (e.g., 80% indoor, 20% outdoor, or others) - UE mobilities in training and inference (e.g., 3 km/h indoor, 30 km/h outdoor, or others) - UE configurations in training and inference (e.g., antenna setup and port layouts, or others) - gNB configurations in training and inference (e.g., antenna setup and port layouts, or others) - Traffic model assumed in training and inference (e.g., full buffer, FTP mode 1, or others) Note: companies only need to populate the attributes that are relevant to their evaluations (not all the attributes) |
| Huawei/HiSi [2] | Proposal 5: For BM Case-1 and BM Case 2, to verify the generalization performance of an AI/ML model over various scenarios/configurations, the following configuration is considered: - Various Set B of beam (pairs) |
| Vivo [3] | Confirm the working assumption for reporting the evaluation results with minor modification, i.e. Table X. Evaluation results for [BM-Case1 or BM-Case2] without model generalization with generalization-Case1 for [DL Tx beam prediction or Tx-Rx beam pair prediction or Rx beam prediction] Support to report generalization-Case2 and generalization-Case3 related assumptions, at least including model input data for training and inference with different configurations and/or scenarios. Proposal 10: Support to confirm generalization related working assumption for further evaluation study in AI based beam prediction scheme. Proposal 11: To study and evaluate generalization, various UE configurations and gNB settings, such as different number of Tx beams and Rx beams, and different gNB/UE antenna configurations, should be prioritized. Proposal 20: Further study generalization performance for different antenna configurations and different beam shapes in BM-Case1. Proposal 22: Further study beam pair prediction scheme with expected information as AI input for improving generalization performance in BM-Case2. Proposal 24: Further study generalization performance for different antenna configurations and different beam shapes in BM-Case2. |
| ZTE [4] | Proposal 3: Different beam constructions in Set B (number/pattern of beams (pairs) in Set B, etc) should be considered for the evaluation of model generalization capability. |
| CATT [7] | Proposal 8: For the study of generalization, we should focus on the influence of different patterns or different Set B for generalization. |
| InterDigital [10] | Proposal 2: For verification of generalization performance, confirm Case 1 and Case 2 and further study Case 2A and Case 3. |
| Xiaomi[11] | Proposal 7: To investigate the model generalization capability, the following aspect(s) can be considered with high priority for the evaluation for AI/ML in beam management: - Different UE parameters: UE speed, number of Rx beam - Different Scenarios, UMa, UMi including UE distribution, etc |
| OPPO [13] | Proposal 1: Study the techniques of pre-processing at model input and post-processing at model output to enable the generalization capability of AI/ML model. |
| Lenovo [16] | Proposal 2 Include “Various Set A of beams (beam-pairs)” in the configurations for verifying the generalizability of AI/ML models for beam management. Proposal 3 Defer the discussion on including “Various Set B of beams(pairs)” in the list of configurations to be considered for evaluating generalization till the selection of Set B is finalized. Proposal 4 While discussing/deciding about evaluating the generalizability of AI/ML models, RAN1 should not decide/mandate how to train the AI/ML models. Proposal 5 Generalizability of a proposed AI/ML model for beam management is evaluated by computing the agreed KPIs, inclusive of the gains achieved and the costs incurred, by the model for each of the different network conditions/scenarios/parameter values. Proposal 6 Discuss how to decide on the generalization ability of an AI/ML model based on the KPIs, inclusive of the gains achieved, and the costs incurred, that are evaluated for each of the different network conditions/scenarios/parameter values. Further, consider the threshold-based methods for further study. |
| APPLE [18] | Proposal 2: For AI model generalization, generalization performance regarding analog beam design including Set A design, antenna configurations including M/N, and antenna spacing and deployment scenario should be considered. Observation 1: If explicit Tx beam shape information for different datasets is not available to model trainer, it may be difficult to design AI model to generalize well over different scenarios/configurations. However, acquiring explicit Tx beam shape information at the UE side may be difficult due to concerns on disclosing proprietary information. Observation 2: it may be difficult to support fine-tuning of a deployed AI model on the UE side. |
| LGE [19] | Proposal 1. For model generalization, confirm working assumption. Also, case 2A is optionally considered by companies. Proposal 2. For model generalization of BM case 1 and 2, additionally consider various Set B of beam(pairs). |
| Samsung [22] | Proposal # 7: For UE side AI/ML model, the following can be considered to verify the generalization performance. - Different gNB setting: number of Tx beam, Tx beam widths, Tx beam pattern, number or pattern in Set B (when applicable), - Different Scenario, like UMa, UMi including UE distribution, UE speed, UE trajectories Proposal # 8: For gNB side AI/ML model, the following can be considered as a starting point to verify the generalization performance. - Different UE parameters: e.g., UE antenna config, number of Rx beam (when applicable), - Scenario: e.g. UMa, UMi including UE distribution e.g., outdoor: in door, UE trajectories, UE speed |
| Qualcomm [23] | Proposal 1: Study the impact of change in gNB codebook with and without codebook-related assistance information on the generalization performance of UE-side AI/ML models. To evaluate the generalization performance, consider the following cases: - Case 1: The AI/ML model is trained based on training dataset using Codebook #A at gNB side, and then the AI/ML model performs inference/test on a dataset generated using the same Codebook #A at gNB side - Case 2: The AI/ML model is trained based on training dataset using Codebook #A at gNB side, and then the AI/ML model performs inference/test on a different dataset than the one generated using Codebook #A at gNB side, e.g., a dataset using Codebook #B at gNB side - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple gNB codebooks including gNB Codebook #A and a different gNB codebook than Codebook #A, e.g., gNB Codebook #B, and then the AI/ML model performs inference/test on a dataset from a single gNB codebook from the multiple gNB codebooks, e.g., Codebook #A, Codebook #B. - Evaluate the above cases with and without codebook-related assistance information and compare the performance. - Note: To see the pure impact of gNB codebook on generalization performance, other aspects of Scenario#A/ConfigurationA and ScenarioB/ConfigurationB can be kept the same as a starting point, i.e., only difference between the two scenarios/configurations is gNB codebook. |
| Nokia/NSB [25] | Observation 16: Several configurations/scenarios can be considered for assessing the ML model generalization capabilities. In this study, we considered the gNB antenna array dimensions, but other configurations/scenarios are not precluded. Supporting multiple configurations may further affect ML performance. Proposal 6: RAN1 further investigates the ML model generalization capabilities of BM Case-1 with other scenarios that can include: • Channel propagation models, e.g. UMa/UMi. • LOS/NLOS Scenarios with different Outdoor/Indoor UEs distribution. Proposal 7: RAN1 further investigates the ML model generalization capabilities of BM Case-1 with gNB antenna array dimensions, e.g., 4x8 and 8x16, and UE antenna array dimensions/Number of panels. Proposal 13: Support RAN1 to further study scenarios/ trajectory models for verify the ML model generalization for BM-Case2. Proposal 14: Support RAN1 to further study different approaches to improve the ML model generalization for BM-Case2 for different UE speeds. |

FL1:(closed) Question 3.1.1a

Proposal 3.1.1a: Confirm the working assumption for AI/ML in BM with following updates:

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

      - Note: Companies to report the ratio for dataset mixing

      - Note: number of the multiple scenarios/configurations can be larger than two

- FFS the detailed set of scenarios/configurations

- The following case for generalization verification, can be optionally considered by companies:

  - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/MsL model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

    - Company to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance

Please share your views on proposal 3.1.1a:

| Company | Comments |
| --- | --- |
| OPPO | Fine with the confirmation of WA with slight changes on Case 2A. |
| Apple | For fine-tuning, actually there are a number of issues to deal with. As discussed in our paper, for Case 2A, the feasibility should be investigated first. |
| Futurewei | We are ok with the proposal. Companies may decide whether fine tuning is applicable for their proposed approach(es). |
| LG | We are ok with the proposal. |
| NTT DOCOMO | Fine with the proposal. |
| CAICT | Support. |
| Vivo | Fine with the proposal. |
| Xiaomi | Support to confirm the working assumption and fine with the updated case 2A. |
| InterDigital | As we mentioned in our contribution, we prefer to confirm only with Case 1/Case 2 and Case 2A/Case3 can be optional. |
| Google | OK |
| CATT | Fine with the proposal. |
| MediaTek | We are OK with the updated changes in Case 2A. |
| Qualcomm | OK |
| Lenovo | While considering generalizability, we need not bother how the AI/ML model has been developed, particularly, how it has been trained. While developing the AI/ML model (especially, when it’s being trained), the developer could have adopted any strategy to make the model generalizable across multiple scenarios/configurations. While discussing and making agreements on how generalizability should be evaluated, we do not see a need to mention how the AI/ML model should be trained. Companies may report how the proposed AI/ML model was trained, for the purpose of re-producibility of proponent conclusions during the evaluations. For evaluating the generalizability of an AI/ML model, its performance should be evaluated across all the agreed scenarios/configurations (A, B, C, etc.) without considering whether the AI/ML model is trained for scenario/configuration A or B or C or any mixture/combination of scenarios/configurations. |
| Fujitsu | Fine with the proposal. |
| HW/HiSi | Ok |
| Spreadtrum | Fine with the proposal. |
| ZTE | Fine with the proposal. |

For the set of scenarios/configuration, the following agreements were agreed:

Agreement - For BM Case-1 and BM Case 2, to verify the generalization performance of an AI/ML model over various scenarios/configurations, the set of scenarios/configurations are considered focusing on one or more of the following aspects as a starting point: - Scenarios - Various deployment scenarios - Various outdoor/indoor UE distributions - Various UE mobility - Configurations - Various UE parameters - Various gNB settings - [Various Set B of beam(pairs)] - Other aspects of scenarios/configurations are not precluded - The selected scenarios/configurations for generalization verification may consider the AI model inference node (e.g., @UE or @gNB) and use case (e.g., BM-Case1, or BM-Case2) - Companies to report the selected scenarios/configurations for generalization verification - Note: other approaches for achieving good generalization performance for AI/ML-based schemes are not precluded.

The following proposals were proposed:

Futurewei

- Deployment scenarios in training and inference (e.g., UMa, UMi, or others)

- Outdoor/indoor UE distributions in training and inference (e.g., 80% indoor, 20% outdoor, or others)

- UE mobilities in training and inference (e.g., 3 km/h indoor, 30 km/h outdoor, or others)

- UE configurations in training and inference (e.g., antenna setup and port layouts, or others)

- gNB configurations in training and inference (e.g., antenna setup and port layouts, or others)

- Traffic model assumed in training and inference (e.g., full buffer, FTP mode 1, or others)

Huawei/HiSi:  Various Set B of beam (pairs)

Vivo:

- various UE configurations and gNB settings, such as different number of Tx beams and Rx beams, and different gNB/UE antenna configurations, should be prioritized.

- different antenna configurations and different beam shapes

ZTE: Different beam constructions in Set B (number/pattern of beams (pairs) in Set B, etc)

CATT:  influence of different patterns or different Set B for generalization.

Lenovo: Include “Various Set A of beams (beam-pairs)”

Apple: Set A design, antenna configurations including M/N, and antenna spacing and deployment scenario should be considered

Xiaomi:

- Different UE parameters: UE speed, number of Rx beam

- Different Scenarios, UMa, UMi including UE distribution, etc

LGE: additionally consider various Set B of beam(pairs).

Samsung:

- For UE side AI/ML model, the following can be considered to verify the generalization performance.

  - Different gNB setting: number of Tx beam, Tx beam widths, Tx beam pattern, number or pattern in Set B (when applicable),

  - Different Scenario, like UMa, UMi including UE distribution, UE speed, UE trajectories

- For gNB side AI/ML model, the following can be considered as a starting point to verify the generalization performance.

  - Different UE parameters: e.g., UE antenna config, number of Rx beam (when applicable),

  - Scenario: e.g. UMa, UMi including UE distribution e.g., outdoor: in door, UE trajectories, UE speed

Qualcomm: codebook

Nokia/NSB:

- BM Case 1:

- Channel propagation models, e.g. UMa/UMi. , LOS/NLOS Scenarios with different Outdoor/Indoor UEs distribution.

- gNB antenna array dimensions, e.g., 4x8 and 8x16, and UE antenna array dimensions/Number of panels

- BM Case 2:

- study scenarios/ trajectory models for verify the ML model generalization for BM-Case2.

- approaches to improve the ML model generalization for BM-Case2 for different UE speeds.

FL1:(closed) Question 3.1.2a

Proposal 3.1.2a

For BM Case-1 and BM Case 2, to verify the generalization performance of an AI/ML model over various scenarios/configurations, additionally considering

- Various Set B of beam(pairs)

- DL Tx beam codebook

Please share your views on proposal 3.1.2a:

| Company | Comments |
| --- | --- |
| OPPO | On various Set B of beam(pairs), is there any restriction on using various Set B, e.g. one fixed Set B for training and another fixed Set B for inference? If that’s not the case, we intend to understand it is up to each company’s implementation and to be reported by each company. Is this understanding correct? Perhaps one or more examples can be given to guide the generalization evaluation. One minor comment on DL Tx beam codebook (the 2nd bullet) is that can we understand it as included in the agreed bullet “various gNB settings” in last meeting? |
| Apple | It seems our proposal on considering “Set A design” is missing. So we suggest to add “various set A design” to the list. |
| Futurewei | We think the 2 additional options need clarification like both fixed and variable Set B options(?) |
| LG | We are fine with first subbullet, but the meaning of DL Tx beam codebook is ambiguous. |
| NTT DOCOMO | Support the proposal. |
| CAICT | We support the first sub-bullet. |
| vivo | We are okay to consider various Set B of beam(pairs). For DL Tx beam codebook, is it about what set of beams to be used in gNB side, like the number of beams, and the specific beam pattern including beam width and granularity? If so, isn’t it included in gNB settings? |
| Xiaomi | As for various set B of beam(pairs), we think that evaluation on generalization performance is only needed for fixed set B. since for various set B, the sample for training and testing are associated with different set B, no additional evaluation is needed. So it is better to update “ various set B of beam(pairs)” to “different fixed set B of beam(pairs)” While for DL Tx beam codebook, we share same view as OPPO. |
| InterDigital | We are fine with the proposal. |
| Google | OK |
| CATT | Support the first sub-bullet. The DL Tx beam codebook is unclear for us. Need more clarification. |
| MediaTek | For the 1st bullet about various Set B, we would like know how to distinguish the study of this bullet with the study of Option2 of the following agreement? To us, Option2 already cover the generalization study for various Set B. Agreement - Study the following options on the selection of Set B of beams (pairs) - Option 1: Set B is fixed across training and inference - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), FFS: - Opt A: Set B is changed following a set of pre-configured patterns - Opt B: Set B is randomly changed among pre-configured patterns - Opt C: Set B is randomly changed among Set A beams (pairs) - The number of beams(pairs) in Set B can be fixed or variable - Note: BM-Case1 and BM-Case2 may be considered for different option. - Other options are not precluded.. |
| Qualcomm | Support |
| Lenovo | - Defer the discussion on “Various Set B of beam (pairs)” till we decide on Set B (whether fixed set B or variable set B). The discussion would be more meaningful once we get the clarity on Set B. - In various scenarios/configurations for generalization, we should include Set A, the total beams available. In case of Tx beam prediction, Set A would be Tx beam codebook and thus, yes, it should be included. |
| Fujitsu | We support the first sub-bullet. |
| Hw/HiSi | We are fine with the proposal in principle. We definitely support the first sub-bullet. The second sub-bullet could also be ok. As some companies commented, that might need clarification. I think that companies for now could report/describe their DL Tx codebook. Maybe the following update could then be acceptable? Updated Proposal 3.1.2a For BM Case-1 and BM Case 2, to verify the generalization performance of an AI/ML model over various scenarios/configurations, additionally considering - Various Set B of beam(pairs) - DL Tx beam codebook - Note: Companies report their assumption in the DL Tx codebook |
| Spreadtrum | OK the first sub-bullet. |
| ZTE | Support the first sub-bullet. |
| Ericsson | Share the view from OPPO that the listed items could be part of the gNB settings as agreed in previous meeting. |
| Xiaomi | According to the offline discussion of proposal 4.3.3a, we suggest to align the wording of Proposal 3.1.2b with Proposal 4.3.3a. It is unnecessary to consider the generalization of random set B without any preconfigured patterns. Proposal 3.1.2b For BM Case-1 and BM Case 2, to verify the generalization performance of an AI/ML model over various scenarios/configurations, additionally considering - Various Set B of beam(pairs) different patterns of set B(s) for fixed Set B (option 1) and different pre-configured patterns of Set B(s) (Option 2A and 2B) - DL Tx beam codebook |

FL2:(closed) Question 3.1.2b

Proposal 3.1.2b

For BM Case-1 and BM Case 2, to verify the generalization performance of an AI/ML model over various scenarios/configurations, additionally considering

- Various Set B of beam(pairs)

- DL Tx beam codebook

Please share your views on proposal 3.1.2b:

| Company | Comments |
| --- | --- |
| FL2 | With clarification that DL Tx beam codebook is part of “ various gNB settings” |
| Lenovo | Selecting Set B as fixed set or a variable set is still under discussion in proposal 4.3.4a. We should first come to an agreement on Proposal 4.3.4a before discussing this proposal. - If Set B is agreed to be a variable set allowing its size and elements to be varied across time and during training and inference, then including Set B does not make any difference to the evaluation of generalizability. - If Set B is chosen to be a fixed set, then verifying the generalizability of AI/ML models across various Set B does not make sense and could become problematic. Consider the following example: Let an AI/ML model is based on reinforcement learning (RL) approach. Then it would select what beams to measure dynamically while making the beam prediction because the next beam to be selected by an RL method depends on its previous actions and the corresponding rewards it observes in the past. Such a model would not work if we fix beforehand which beams should it measure to make the beam prediction -- in other words, such a model would not work if we fix set B. |
| Samsung | We support the proposal 3.1.2b. Several cases about various Set B are not included in gNB settings. For example, Set B under Opt C and Opt D in proposal 4.3.4a may not be gNB settings but this may depend on UE’s Set B selection. |
| Apple | Various set A can be considered for the same reason for various set B FL3: Can Various set A also be covered in various gNB settings? |
| Intel | It is not clear to us how a model generalized for various Set A. Can proponents clarify further with examples? |
| FL3 | Agreement For BM Case-1 and BM Case 2, to verify the generalization performance of an AI/ML model over various scenarios/configurations, additionally considering - Various Set B of beam(pairs) |

FL3:(closed) Question 3.1.3a

Proposal 3.1.3a:

- For generalization performance verification, consider the following scenarios/configurations:

  - Scenarios

    - Various deployment scenarios,

      - e.g., UMa, UMi and others,

      - e.g., 200m ISD or 500m ISD and others

    - Various outdoor/indoor UE distributions, e.g., 100%/0%, 20%/80%, and others

    - Various UE mobility,

      - e.g., 3km/h, 30km/h, 60km/h and others

    - Carrier frequency

  - Configurations

    - Various UE parameters, e.g., number of UE Rx beams (including number of panels and UE antenna array dimensions)

    - Various gNB settings, e.g., DL Tx codebook (including various Set A of beam(pairs) and gNB antenna array dimensions)

    - Various Set B of beam (pairs)

    - T1 for measurement /T2 for prediction for BM-Case2

Please share your views on proposal 3.1.3a:

| Company | Comments |
| --- | --- |
| FL3 | The examples are based on observations and proposals from contributions. |
| vivo | For various gNB settings, we suggest to make the following change. Various gNB settings, e.g., DL Tx codebook (including various Set A of beam(pairs) and gNB antenna array dimensions) Various gNB settings, e.g., DL Tx codebook (including the number of beams (pairs) in Set A, various Set A of beam(pairs) and gNB antenna array dimensions) |
| Futurewei | We are ok with the proposal in general. We suggest adding another bullet to indicate “other scenarios/configurations are not precluded”. |
| Xiaomi | Support |
| CAICT | Support |
| MediaTek | We are OK with this proposal. |
| Samsung | Regarding ‘T2 for prediction for BM-Case2’, we don’t think this is generalization problem but this is just model’s task. For ‘T1 for measurement’, this may be included in gNB setting since different UE-side models may be trained from different T1 assumptions. But, we are not sure why a model needs to be generalized for different T1. Regarding various gNB settings, we are fine with vivo’s modification in principle. Also, a beam pair is not only for gNB setting, but the combination of UE parameters. Therefore, we suggest to modify the proposal as follows: Updated Proposal 3.1.3a: - For generalization performance verification, consider the following scenarios/configurations: - Scenarios - Various deployment scenarios, - e.g., UMa, UMi and others, - e.g., 200m ISD or 500m ISD and others - Various outdoor/indoor UE distributions, e.g., 100%/0%, 20%/80%, and others - Various UE mobility, - e.g., 3km/h, 30km/h, 60km/h and others - Carrier frequency - Configurations - Various UE parameters, e.g., DL Rx beam codebook (including the number of UE Rx beams, (including the number of panels, UE beam directions, and UE antenna array dimensions) - Various gNB settings, e.g., DL Tx beam codebook (including the number of gNB Tx beams (pairs) in Set A, various gNB beam directions of beams in Set A of beam(pairs) and gNB antenna array dimensions), T1 for measurement - Various Set B of beam (pairs) - T1 for measurement /T2 for prediction for BM-Case2 |
| ZTE | We suggest to remove the last sub-bullet since it has not been discussed in the generalization part. Besides, some examples can also be provided for various Set B of beam (pairs). Thus, we suggest the following revision. - Configurations - Various UE parameters, e.g., number of UE Rx beams (including number of panels and UE antenna array dimensions) - Various gNB settings, e.g., DL Tx codebook (including various Set A of beam(pairs) and gNB antenna array dimensions) - Various Set B of beam (pairs), e.g., number of beams, different beam grids/combs - T1 for measurement /T2 for prediction for BM-Case2 |
| LG | We are fine with ZTE’s modification. One more clarification can be DL Tx codebook in 2nd bullet. Instead of DL Tx codebook, we prefer to use DL Tx beams. |
| Spreadtrum | Support |
| FL4 | Agreement - For generalization performance verification, consider the following - Scenarios - Various deployment scenarios, - e.g., UMa, UMi and others, - e.g., 200m ISD or 500m ISD and others - e.g., same deployment, different cells with different configuration/assumption - e.g., gNB height and UE height - FFS: e.g., Carrier frequencies - Various outdoor/indoor UE distributions, e.g., 100%/0%, 20%/80%, and others - Various UE mobility, - e.g., 3km/h, 30km/h, 60km/h and others - Configurations (parameters and settings) - Various UE parameters, e.g., number of UE Rx beams (including number of panels and UE antenna array dimensions) - Various gNB settings, e.g., DL Tx beam codebook (including various Set A of beam(pairs) and gNB antenna array dimensions) - Various Set B of beam (pairs) - T1 for measurement /T2 for prediction for BM-Case2 - Other scenarios/configurations(parameters and settings) are not precluded and can be reported by companies. |

FL5:(FFS) Question 3.1.4

Companies are encouraged to provide some analysis on the reasonable scenarios/configurations for each use cases (e.g., BM-Case1 vs BM-Case 2) and/or AI/ML inference location (e.g., UE/gNB side model) if applicable.

3.2 (Medium)Evaluation results for generalization

Companies also provided evaluation results for generalization:

| Company | Proposal/observations |
| --- | --- |
| Vivo[3] | Observation 17: Performance loss can be observed if there is difference in beam shape patterns for training and validation in BM-Case2. Observation 18: For the case using local beam ID as model input, beam loss and accuracy degenerate significantly compared to the performance of AI model training and inference with beam pointing angle. |
| ZTE[4] | Observation 4: The case of AI generalization that model training and testing are performed in mixed datasets with different beam constructions can achieve a better performance than that of the case of <T8 R1>, but is outperformed by the case of <T32 R1>. Observation 9: If the AI model is trained with samples of single UE speed or mixed UE speeds, a little performance loss is observed as the UE speed in testing samples increases from 30km/h to 90km/h. Observation 10: For the generalization evaluation, the AI based method can also achieve a better performance than that of the non-AI method in scenarios with mixed UE speeds. Observation 11: A little performance loss is observed as the AI model is trained with a single UE speed and tested with mixed UE speeds. |
| Fujitsu [5] | Observation 7: The performance of AI/ML model has slight degradation with the model trained with dataset of Uma but tested in Umi. Observation 8: The model trained by hybrid dataset of Uma and Umi will improve the generalization performance of AI/ML model. |
| Google [6] | Observation 4: Beam pattern mismatch could cause significant performance degradation. |
| CATT [7] | Observation 2: When AI/ML model is trained and inferenced with different Set B, e.g., the Set B of training is 32 and inference is 16, the Top-1 accuracy is degraded. Observation 3: When AI/ML model is trained and inferenced with different scenario configurations, e.g., the Set B of training is Uma and inference is Umi, the Top-1 accuracy can be comparable. Observation 4: When AI/ML model is trained and inferenced with different UE distributions, e.g., the Set B of training of UE distributions is 0% and inference is 50%, the Top-1 accuracy can be comparable. Observation 5: When AI/ML model is trained and inferenced with different carrier frequency, e.g., the Set B of training is 20G and inference is 30G, the Top-1 accuracy can be comparable. Observation 6: When AI/ML model is trained and inferenced with different ISD, e.g., the Set B of training of ISD is 200 and inference is 500, the Top-1 accuracy can be comparable. |
| Ericsson [9] | - With identical antenna configuration, initial evaluations indicates that a model trained in one cell is found to be generalized well while the performance heavily depends on the sector is selected for the inference. - Generalization results indicate the importance of having model monitoring procedures that detects issues when a model trained in one cell is used in another. |
| Xiaomi [11] | Observation 3: AI model trained by hybrid data of Uma and Umi for beam prediction in spatial domain can provide good generalization capability for Uma or Umi. While AI model trained by data of only Uma or only Umi provide a little worse generalization capability for different scenario, i.e., Uma model for Umi inference, or Umi model for Uma inference. Observation 4: AI model trained by hybrid data of different UE distribution for beam prediction in spatial domain can provide good generalization capability. While AI model trained by data of only UE distribution Option A provides a little worse generalization capability for UE distribution Option B. Observation 5: AI model for beam prediction in spatial domain can provide good generalization capability among different number of UE Rx beam, e.g., AI model with more Rx beam number can be applied for beam prediction of less Rx beam number. Observation 9: AI model for beam prediction in time domain trained by data of 30km/h or only 60 km/h or hybrid can provide good generalization capability to UE speed with both 60km/h and 30km/h. |
| Intel [12] | Observation 4: The effect of UE speed and measurement interval to the number of unique beams is not significant. Observation 5: The preliminary data statistics indicate that relatively few beam changes occur over the length of UE trajectories, therefore high prediction accuracy of any model should be further investigated since the accuracy may be biased by input data with relatively few beam switches and high KPIs may be overall misleading since the model may not successfully predict beam changes despite apparently good performance on paper. |
| OPPO [13] | - Thanks to generalization capability of well-trained AI/ML model, changing scenario from UMa to UMi may not necessarily deteriorate the beam prediction performance. - Changing beam pair configuration on Set B and Set A from training phase to inference phase would slightly lower the beam prediction performance. - When more predicted beam pairs are provided by AI/ML model, e.g. Top-K = 4, the beam selection accuracy can be up to 95% and avg. L1-RSRP difference can be lower than 1dB. - For AI/ML model trained with UE speed 30km/h and tested with UE speed higher than 60km/h, the generalization performance on beam prediction accuracy and avg. L1-RSRP difference significantly drops. |
| Apple [18] | Observation 3: - For generalization performance Case -1, trained AI models perform well, provide better beam prediction accuracy than the conventional approach. - For generalization performance Case -2, trained AI models can perform much worse than that for GP Case-1, they may lead to even worse beam prediction accuracy than the conventional approach. - For generalization performance Case-3, trained AI models can perform much worse than that for GP Case-1, even though are in general better than that for GP case-2. the AI performance with set B beam at 8 beams with GP Case-3 is roughly the same as the AI performance with set B at 4 beams with GP Case-1. |
| DoCoMo [21] | Observation 1: The performance of AI/ML based beam prediction BM Case-1 is acceptable even when different Rx beams are assumed for training and inference. Observation 2: AI/ML based beam prediction BM Case-1 could barely provide the gain if different Tx beams are assumed for training and inference. Observation 3: AI/ML based beam prediction BM Case-1 could provide the fairly good performance when the model is trained with mixed pre-configured patterns. |
| Samsung [22] | Observation # 19: For DL TX beam prediction and beam pair prediction, training a model with a mixture of dataset drawn from a range of UE speeds allows the model to perform well over a range of UE speeds. |
| Qualcomm [23] | Observation 5: Homogeneous inter-site scenarios (e.g., same deployment, different cells) have generally better generalization performance compared to heterogeneous inter-site scenarios (e.g., cells from different deployments). Observation 6: For heterogeneous inter-site scenarios (e.g., cells from different deployments), incorporating datasets from different deployments in the training may improve the heterogenous inter-site generalization without significantly affecting the performance in the seen cells. |
| Nokia/NSB [25] | Observation 13: The ML model trained with only outdoor UEs may NOT generalize well for indoor UEs, whereas the ML model trained with mixed indoor/outdoor UEs may generalize well for both indoor and outdoor UEs. Observation 14: The ML model trained with only indoor UEs may be applied also for outdoor UEs without compromising the performance with respect to the ML model trained with mixed indoor/outdoor UEs. Observation 15: model's generalization capabilities should be assessed considering the different combinations of configurations as ML model performances can be affected significantly. Observation 16: Several configurations/scenarios can be considered for assessing the ML model generalization capabilities. In this study, we considered the gNB antenna array dimensions, but other configurations/scenarios are not precluded. Supporting multiple configurations may further affect ML performance. Observation 18: In BM-Case1 DL TX beam prediction, compared to training model with fixed Set B, training model with random Set B can provide similar performance when ¦Set B¦/¦Set A¦ is large (i.e. 32/64) but the performance will become inferior when ¦Set B¦/¦Set A¦ is small. Observation 21: In BM-Case1 DL Tx-Rx beam pair prediction, training model with random Set B has a nonnegligible performance drop compared training model with fixed Set B. And large K for top-K beam search is necessary for maintaining the performance. Observation 25: For BM-Case2, the ML model using as input only RSRPs has performance that decreases when Set B is a subset of Set A and if no advanced algorithm is applied for beam selection in Set B. Observation 26: For BM-Case2, the ML model using as input only RSRPs has performance that decreases when increasing the length of the prediction window. Proposal 13: Support RAN1 to further study scenarios/ trajectory models for verify the ML model generalization for BM-Case2. Proposal 14: Support RAN1 to further study different approaches to improve the ML model generalization for BM-Case2 for different UE speeds. |

The following table summarized the observations from companies

| Scenarios/configurations | Observations |
| --- | --- |
| deployment scenarios (e.g., UMa, UMi) | - Slight degradation with Case 2 (different) and improve the generalization performance with Case 3 (mixed) (Fujitsu) - Top-1 accuracy can be comparable with Case 2(different) (CATT) - For instance, for 100% Outdoor UE distribution, the performance degrades if the model is trained based on the deployment scenario (i.e., UMa) and the inference is done in a different deployment scenario (i.e., UMi). (Case 2) (Ericsson) - However, for 80%/20% In/outdoor UE distribution, the performance does not always depend on the deployment scenario (UMa or UMi) if different deployment scenarios are considered for training and inference. Therefore, it is not clear the impact of deployment scenario (UMa or UMi) for 80%/20% In/outdoor UE distribution, which needs further investigations. (Ericsson) - AI model trained by hybrid data of Uma and Umi for beam prediction in spatial domain can provide good generalization capability for Uma or Umi. (Case 3) (xiaomi) - While AI model trained by data of only Uma or only Umi provide a little worse generalization capability for different scenario, i.e., Uma model for Umi inference, or Umi model for Uma inference. (Case 2) (Xiaomi) - may not necessarily deteriorate the beam prediction performance. (Case 2)(OPPO) - Homogeneous inter-site scenarios (e.g., same deployment, different cells) have generally better generalization performance compared to heterogeneous inter-site scenarios (e.g., cells from different deployments). (Qualcomm) - For heterogeneous inter-site scenarios (e.g., cells from different deployments), incorporating datasets from different deployments in the training may improve the heterogenous inter-site generalization without significantly affecting the performance in the seen cells (Qualcomm) |
| UE distribution | - Top-1 accuracy can be comparable with Case 2(different) (CATT) - challenging to use a model trained based on 100% UE distribution to perform the inference of the scenario with 80%/20% In/outdoor UE distribution. (Case 2?) (Ericsson) - However, the performance is much better if using a model trained based on 80%/20% In/outdoor UE distribution to perform the inference of the scenario with 100% UE distribution. Therefore, the model training should ensure a mix of various UE deployments. (Case 3?) (Ericsson) - Case 3 can provide good generalization capability. (xiaomi) - Case 2 provides a little worse generalization capability. (Case 2) (xiaomi) - outdoor UEs may NOT generalize well for indoor UEs (Case 2) (Nokia/NSB) - the ML model trained with mixed indoor/outdoor UEs may generalize well for both indoor and outdoor UEs. (Case 3)(Nokia/NSB) |
| Different ISD | - Top-1 accuracy can be comparable with Case 2(different) (CATT) - the performance degrades when the model is trained only using the data generated by one deployment scenario (i.e., ISD=200) and perform the inference in another deployment scenario (i.e., ISD=500). Therefore, the model training would benefit from a mix of various deployment scenarios in terms of generalizing to different ISD. (Ericsson) |
| UE speeds | - Case 2 and Case 3, a little performance loss (ZTE) - Case 2 and Case 2, good generalization capability (xiaomi) - not significant (Intel) - the generalization performance on beam prediction accuracy and avg. L1-RSRP difference significantly drops. (Case 2)(OPPO) - allows the model to perform well over a range of UE speeds (Case 3)(Samsung) - BM-Case2, when the ML model trained at a specific UE speed is applied to a different UE speed (Case #2), the performance degrades significantly, especially if the UE speeds are very different. (Nokia/NSB) |
| Variable set B | - Top-1 accuracy is degraded case 2(different number) (CATT) - would slightly lower the beam prediction performance. (OPPO) - Different Tx beam in Set B, AI/ML based beam prediction BM Case-1 could barely provide the gain (Case 2)(DCM) - mixed pre-configured patterns AI/ML based beam prediction BM Case-1 could provide the fairly good performance (Case 3) (DCM) - training model with random Set B can provide similar performance when ¦Set B¦/¦Set A¦ is large (i.e. 32/64) but the performance will become inferior when ¦Set B¦/¦Set A¦ is small. (Case 3)(Nokia/NSB) - training model with random Set B has a nonnegligible performance drop compared training model with fixed Set B. And large K for top-K beam search is necessary for maintaining the performance. (Case 3)(Nokia/NSB) |
| Carrier frequency | Top-1 accuracy can be comparable with Case 2(different) (CATT) |
| Different number of UE Rx beam | - (Case 2?) AI model for beam prediction in spatial domain can provide good generalization capability among different number of UE Rx beam, e.g., AI model with more Rx beam number can be applied for beam prediction of less Rx beam number. (xiaomi) - The performance of AI/ML based beam prediction BM Case-1 is acceptable (DCM) |
| Measurement window | not significant (Intel) |
| Prediction window | as input only RSRPs has performance that decreases when increasing the length of the prediction window. (Nokia) |
| Variable number of predicted beam pairs | When more predicted beam pairs are provided by AI/ML model, e.g. Top-K = 4, the beam selection accuracy can be up to 95% and avg. L1-RSRP difference can be lower than 1dB. (OPPO) |
| Beam shape patterns/ gNB antenna array | - Performance loss (Vivo) - cause significant performance degradation (Google) - FL1: what is the mismatched beam pattern? is the input of AI L1-RSRP or CIR? - The ML model does not generalize well for the larger antenna array size and the beam prediction accuracy falls below 50%. (Case 2) (Nokia/NSB) - Performances are lower than the case when the ML model is trained and tested on the same configuration of the gNB antenna array. Nevertheless, some performance degradations are expected compared to the case when the ML model is trained with data from only one configuration of the gNB antenna array. (Case3)(Nokia/NSB) . |

FL5:(FFS) Question 3.2

As listed in the above table. I will summarize observations reported form companies for each scenarios/settings/parameter.

Please feel free to extend the table agreed as working assumption to present the results for generalization.

AI/ML related assumptions

  - (Low priority) Inputs of AI/ML models

In RAN 1 #110, the following alternatives were agreed.

Agreement For the sub use case BM-Case1 and BM-Case2, further study the following alternatives for the predicted beams: - Alt.1: DL Tx beam prediction - Alt.2: DL Rx beam prediction - Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam) - Note1: DL Rx beam prediction may or may not have spec impact

In this section, further discussion on inputs for each alternative. The following observations/proposals were provided in contributions:

| Company | Proposal/observation |
| --- | --- |
| Huawei/HiSi [2] | Observation 1: For the AI/ML-based beam prediction mechanism, Alt.1 (DL Tx beam prediction) may also achieve best Tx-Rx beam combination by DL Tx beam prediction and legacy Rx beam sweeping. Proposal 1: For the evaluation of AI/ML-based beam prediction mechanism, - Alt.1 (DL Tx beam prediction) should be considered as the starting point. - Both Case A (best Rx beam) and Case B (same specific Rx beam) can be adopted and reported by companies - Alt.3 (Tx-Rx beam pair prediction) can be also evaluated to justify potential additional performance gain over Alt.1. - Alt.2 (DL Rx beam prediction) can be considered with lower priority. |
| Vivo [3] | Proposal 23: Further study performance comparison between enhanced beam pair prediction and DL Tx beam prediction with various Rx beam assumptions, such as worst Rx beam, second best Rx beam, random Rx beam per sample, etc., in BM-Case2. |
| Spreadtrum [8] | Proposal 4: Set B to be a subset of set A for spatial domain beam prediction can be used as baseline, - If AI/ML inference is at NW side, beams in Set B can be determined by NW implementation. - If AI/ML inference is at UE side, beams in Set B can be determined with a fix pattern. Proposal 5: For the AI/ML model input of BM-Case 1, - Alt 1 and Alt 4 should be studied with priority. - Whether to choose Alt 1 or Alt 4 needs further discussion according to the beam pattern selection. - If Set B is fixed, Alt 1 will be selected; - If Set B is variable, Alt 4 will be selected. - For the corresponding DL Tx and/or Rx beam ID in Alt 4, it should be input in AI model explicitly. - Alt 2 should be clarified which assistance information can be used as AI model input. Proposal 6: For temporal beam prediction, evaluate and further study “Set A and Set B are the same” as the baseline. Proposal 7: For the AI/ML model input of BM-Case 2, - Alt 1 and Alt 3 should be studied with priority. - Whether to choose Alt 1 or Alt 3 needs further discussion according to the beam pattern selection. - If Set B is fixed, Alt 1 will be selected; - If Set B is variable, Alt 3 will be selected. - For the corresponding DL Tx and/or Rx beam ID in Alt 3, it should be input in AI model explicitly. Alt 2 should be clarified which assistance information can be used as AI model input. |
| Samsung [22] | Observation # 21: With L1-RSRPs of fixed Rx beam(s) as AI inputs can provide better performance than L1-RSRP of random Rx beam(s) for DL Tx-Rx beam pair prediction for BM-Case 1. Observation # 22: For beam pair prediction for BM-Case 1, AI with inputs as L1-RSRPs of fixed Tx beams and implicit beam ID information can provide better performance than non-AI based approach. Observation # 22: For beam pair prediction for BM-Case 1, AI with inputs as L1-RSRPs of fixed Tx beams and implicit beam ID information can provide better performance than non-AI based approach. |
| Qualcomm [23] | Proposal 2: For BM-Case1 and BM-Case2, consider the following factors for UE-side and gNB-side AI/ML models: • Feasibility and availability of inputs to the AI/ML model at each side • If feasible, study the benefits, impact on system operation, and trade-offs for making a given set of inputs available at each side |
| MTK [24] | Proposal 14: For AI/ML-based spatial domain beam prediction evaluation, adopt the RSRP of beams in Set B as the AI/ML model inputs. Additional information to the input of AI/ML model is not excluded. Proposal 15: Adopt one of the following as the output of AI/ML model: (i) beam index of highest RSRP Set A of beams. (ii) RSRPs of all the Set A of beams. |
| Nokia/NSB [25] | Observation 10: With the same RS resources, the DL Tx beam prediction has better beam performance regarding the model intermedia KPI. Observation 11: The Top-1 beam pair prediction performance is much degraded when ¦Set A¦/¦Set B¦ is low, but the Top-K/1 (i.e. K=4) performance metric are improved to a tolerable level. Therefore, the second level beam pair search (best K beam pair) is most likely nonnegligible for the beam management scheme for beam pair prediction. Observation 12: The beam pair prediction performance is significantly improved in Top-K/1 (i.e. K=4) metric. Proposal 4: The top-K beam pair search is needed for the beam management scheme of beam pair prediction. Proposal 5: RAN1 further investigates the comparison between independent Tx beam and joint Tx-Rx beam pair prediction. |

  - (Low priority) Number of beams Tx and/or Rx beams for Set A and Set B

In last meeting the following agreement was made for the number of beams:

Agreement - For the evaluation of both BM-Case1 and BM-Case2, 32 or 64 downlink Tx beams (maximum number of available beams) at NW side. - Other values, e.g., 256, etc, are not precluded and can be reported by companies. - For the evaluation of both BM-Case1 and BM-Case2, 4 or 8 downlink Rx beams (maximum number of available beams) per UE panel at UE side. - Other values, e.g., 16, etc, are not precluded and can be reported by companies.

The following observations/proposals were provided in contributions:

| Company | Proposal/observation |
| --- | --- |
| Huawei/HiSi [2] | Observation 3: Using 256 beams in Set A constructed from a dense codebook increases the angular resolution compared to a 64-DFT codebook, while the same sparse sweeping procedure for inference and gNB configurations can be applied. Proposal 6: To assess the RSRP gains achievable with 256 Tx beams compared to Exhaustive 64 beam sweeping, companies are encouraged to evaluate a dense codebook with overlapping beams for the construction of Set A. |
| Spreadtrum [8] | Proposal 1: For both BM-Case1 and BM-Case2, unifying the number of beams contained in set A and set B should be considered. |
| InterDigital [10] | Proposal 13: Number of beams in Set B should be decided and reported by each company. |
| Nokia/NSB [25] | Observation 1: For BM-Case1, a large number of beams in Set B (e.g., 32) may not improve the prediction accuracy and the system throughput. Therefore, ML-based beam selection should consider a Set B with a maximum of 16 beams when Set A has 64 beams, hence Set B should have a max of ¼ of Set A beams. Observation 2: The design of Set A/B together with the ML model design should provide comparable or better sector throughput and cell-edge UE throughput compared to the non-ML baseline. |

  - (High)Selection of Set B of beams(pairs)

RAN 1 #110bis agreed three options of the selection of Set B of beams(pairs).

Agreement - Study the following options on the selection of Set B of beams (pairs) - Option 1: Set B is fixed across training and inference - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), FFS: - Opt A: Set B is changed following a set of pre-configured patterns - Opt B: Set B is randomly changed among pre-configured patterns - Opt C: Set B is randomly changed among Set A beams (pairs) - The number of beams(pairs) in Set B can be fixed or variable - Note: BM-Case1 and BM-Case2 may be considered for different option. - Other options are not precluded.

The following proposals/observations were about the section of Set B:

| Company | Proposal/observation |
| --- | --- |
| Futurewei [1] | Observation 1: In spatial-domain beam prediction using fixed Set B beam pattern in AI/ML model training phase, performance degrades significantly when Set B beam pattern changes in model inference phase. Observation 2: In spatial-domain beam prediction using fixed Set B beam pattern in AI/ML model training phase, performance degrades significantly when at least one of the beam pairs in Set B is missing during model inference phase. Observation 3: In spatial-domain beam prediction using variable Set B beam patterns in AI/ML model training phase, performance degrades significantly when Set B beam patterns changes in model inference phase. Observation 4: In spatial-domain beam prediction using variable Set B beam patterns in AI/ML model training phase, when Set B pattern changes in model inference phase, less performance degradation is observed when only a subset of the beam patterns used as input in the inference phase are different from the beam patterns used in the training phase compared to when all the beam patterns used as input in inference phase are different than the training phase. |
| Huawei/HiSi [2] | Observation 4: For the selection of Set B, under Option 2, it is more realistic for the gNB to choose among a set of pre-configured Set B (Option A/B), rather than varying over totally random beams in Set B (Option 2C). - Among the pre-configured Sets, Option A can be regarded as special of Option B. Proposal 7: For BM-Case-1 and Case-2, for the selection of Set B, consider Option 1 (Set B is fixed across training and inference) as a starting point. - For Option 2 (Set B is variable), Option B can be evaluated optionally further. |
| Vivo [3] | Proposal 12: Unless an excellent generalization performance can be proved in option 1, i.e. a fixed subset in Set B for training and same fixed subset in Set B for validation, fixed set B selection scheme should be deprioritized. Proposal 15: Support Opt C in option 2, i.e. Set B is randomly changed among pre-configured patterns, with both Tx and Rx beam information as AI input. |
| ZTE [4] | Observation 7: Compared with the case that Set B is fixed across training and inference, a slight performance gain is observed if Set B is changed following a set of pre-configured patterns. |
| Fujitsu [5] | Observation 1: For option 2-A and option 2-B of variable Set B, the performance of AI/ML model may have no difference if the model is trained with dataset constructed by a set of pre-configured patterns. Proposal 1: For the option 2-A and option 2-B of variable Set B, it’s suggested to remove the option 2-A. Observation 3: For variable size of Set B, the model trained by hybrid dataset will improve the performance of AI/ML model. Observation 4: For variable size of Set B, the performance of the model trained by hybrid dataset is almost the same as the model trained by separated dataset for fixed Set B. Observation 5: The performance of AI/ML model is sensitive to the patterns of beams in Set B with the same size. Observation 6: For the same size but variable patterns of Set B, the model trained by hybrid dataset with variable patterns will improve the performance a lot for the AI/ML model. |
| CATT [7] | Observation 1: For Beam pair prediction, using pre-configured patterns for training and inference, the Top-1 accuracy can be greater than 93%. Proposal 6: For BM-Case1, random pattern of Opt B, i.e., Set B is randomly changed among pre-configured patterns, can be used to improve beam pair prediction accuracy. Proposal 7: For Tx beam prediction, random pattern of Opt B, i.e., Set B is randomly changed among pre-configured patterns, can be used to improve beam pair prediction accuracy. |
| Ericsson [9] | We consider a scheme with gNB-side inference where the UE measures a fixed set of beams, but only reports beams with RSRP exceeding a certain threshold relative to the strongest beam, i.e., only beams with an RSRP at most X dB below the RSPR of the strongest measured beam are reported. Observation 5 By allowing variable number of reported beams via UE pre-processing of measurements, the reporting overhead can be substantially reduced with little performance degradation. |
| Xiaomi [11] | - Scheme 2: L1-RSRP of variable set B and corresponding beam pair IDs - Scheme 3: L1-RSRP of fixed set B |
| Intel [12] | Proposal 2: The variability of Set B can only be due to updating the L1 measurements corresponding to beams or beam-pairs in Set B at different intervals. The cardinality of the set should not change across training and inference. |
| OPPO [13] | Proposal 1: Apply fixed Set B across training and inference phases for BM-Case1 and BM-Case2. |
| CMCC [15] | Proposal 1: On selection of Set B of beams (pairs), study the following options when Set B is variable -  Set B is randomly changed among pre-configured patterns -  Set B is randomly changed among Set A beams (pairs) |
| Lenovo [17] | Selection of Set B: - Option 1 is a special case of Option 2 and selecting option 2 does not preclude having a fixed set B (i.e., option 1). Further, option 1 constrains the type of learning techniques that can be used while developing AI/ML models for beam management. - Select option 2 for Set B. Under option 2, allow the set B to have variable number of beams at each instant of time during training and/or inference and allow the beams in set B to change across time during training and/or inference. - An alternative is to keep both option 1 and option 2 open without further down selection and the option selected for Set B should be indicated in AI/ML model evaluations. |
| LGE [19] | Proposal 3. Option 1 can be considered as a baseline for selection of Set B of beams. |
| CAICT [20] | Observation 1: AI-based solution could achieve good performance for BM-Case 1 beam pair prediction with same training and validation set configurations. Observation 2: Fixed Set B (Option 1) achieves better performance than randomly chosen pre-configured pattern schemes for BM-Case 1 beam pair prediction. Proposal 1: Fixed Set B (Option 1) could be used as baseline for BM-Case 1 further beam pair comparation. |
| DoCoMo [21] | Observation 4: With mixed Set B configuration in training and inference, the AI/ML model could still provide good performance compared to legacy method. |
| Samsung [22] | Observation # 1: Opt A (Fixed Set B) is more suitable for BM-Case2 considering multiple reports at different time instance. Opt A may be also beneficial for BM-Case1. Observation # 2: Opt B (Change with pre-configured pattern) may be reasonable if different Set B is needed for different UEs. Observation # 3: Opt C (randomly changed among Set A) may be reasonable for AI/ML at UE side since different gNB may have different Set B. Observation # 4: Opt D (randomly changed among a pre-configured subset of Set A) require less L1-RSRP report overhead and may be more reasonable for AI/ML at gNB side. Proposal # 1: Support the following options on the selection of Set B of beam(pairs) for evaluation for AI/ML at gNB side - Option 1: Set B is fixed across training and inference - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/ measurement during training and/or inference) - Opt A: Set B is changed following a set of pre-configured patterns - Opt B (optional): Set B is randomly changed among pre-configured patterns - Opt D (optional): Set B is randomly changed among a pre-configured subset of Set A of beams (pairs) - The number of beams(pairs) in Set B can be fixed or variable Proposal # 2: Support the following options on the selection of Set B of beam(pairs) for evaluation for AI/ML at UE side - Option 1: Set B is fixed across training and inference - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/ measurement during training and/or inference) - Opt A: Set B is changed following a set of pre-configured patterns - Opt B (optional): Set B is randomly changed among pre-configured patterns - Opt C (optional): Set B is randomly changed among Set A beams (pairs) Observation # 12: With large number of beams in Set B, e.g., ¼ beams of Set A, fixed Set B or pre-known different patterns in each time step has similar performance. Observation # 13: When the number of beams in Set B is smaller, e.g., 1/8 beams of Set A, pre-known different patterns in each timestep can provide better performance for the KPIs allows some error. However, for the accuracy of Top 1 beam, fixed pattern has better performance. Observation # 14: With measurements of more beams of Set B in one timestep, the performance is better performance than with the measurements of beams of Set B collected from different timesteps.Observation # 28: Even if UE reports only half of Set B’, we can observe gain in top K/1 prediction accuracy compared to non-AI. As the number of reporting beams increases, there is a benefit, but it does not increase significantly. Proposal # 3: For the case of fixed Set B’, adopt the maximum 4 number of reported beams out of Set B’ as a baseline for evaluation. |
| Qualcomm [23] | Observation 3: For BM-Case1 and the sub use case of “Set B is a subset of Set A”, and for a fixed Set B option, the beam prediction performance largely depends on how Set B is selected. For instance, if set A includes beams in both azimuth and elevation, selecting Set B beams only in azimuth dimension may adversely impact prediction performance. |
| MTK [24] | Proposal 16: For AI/ML-based spatial domain beam prediction evaluation, study the subset selection (number and combination) if Set B is variable (Option2 on the selection of Set B of beams in the RAN1 #110 agreement). Proposal 17: Study and evaluate a more comprehensive Set B design, including joint designing the number of beams in Set B and their beam shape for spatial beam prediction. Observation 9: With a greater number of beams in Set B, both models achieve higher Top-K/1 accuracy. However, greater number of beams in Set B requires more beam RSRP measurements. Observation 10: The selection of beams in Set B will affect the prediction accuracy of the AI/ML-based spatial beam prediction. Observation 11: The spatial beam prediction by using multi-arm beam design in Set B performs better than using subset beam design in Set B. Observation 12: The spatial beam prediction by using wide beam design in Set B does not outperforms the performance by using subset beam design in Set B. |
| Nokia/NSB [25] | Observation 8: For Set B is different to Set A with Set B is wide beam, the KPI for the wide beam codebook design should be both prediction accuracy and throughput performance. Proposal 2: For BM-Case1, RAN1 may further study the case of Set A/B are DL Tx and Set B/Set A are different. • Set B is a wide beam codebook and Set A is a refined beam codebook • Advance Set B designs are needed to provide sufficient refined beam prediction performance. Observation 9: Selecting the Rx beam by following the configured QCL-D info may or may not be the optimum choice for the beam pair prediction. Proposal 3: Advanced Rx beam selection procedure other than following the configured QCL-D info should be considered for beam pair prediction. Observation 17: In BM-Case1 DL TX beam prediction, training model with fixed beam number in random Set B outperforms the training model with varied beam number in random Set B. Observation 18: In BM-Case1 DL TX beam prediction, compared to training model with fixed Set B, training model with random Set B can provide similar performance when ¦Set B¦/¦Set A¦ is large (i.e. 32/64) but the performance will become inferior when ¦Set B¦/¦Set A¦ is small. Observation 19: In BM-Case1 DL TX beam prediction, the top-K beam search is needed for the model trained with random Set B. Proposal 8: RAN1 further investigates the tradeoff between NW side model trained with fixed Set B and UE side model trained with random Set B. |

Summary of the views from companies.

| Options |  | Supporting companies |
| --- | --- | --- |
| Option 1 |  | Huawei/HiSi, Intel, OPPO, LGE (baseline), CAICT (baseline) |
| Option 2 | Opt A | InterDigital |
|  | Opt B | Huawei/HiSi, CATT (BM-Case1), Fujitus?, CMCC |
|  | Opt C | Vivo, CMCC |

As mentioned by several companies, different options may be used for different cases. Therefore, feature lead would like to collect views from companies on which cases the option of Set B is meaningful, assuming no need to further verify generalized performance. For example, Option 1 with fixed Set B, it may only apply for gNB side model, assuming gNB can configure a fixed Set B.

First of all, the following two proposed observations for option 1 and option 2C:

FL1:(closed) Question 4.3.1a

Proposed Observation 4.3.1a

- Fixed Set B across training and inference is more suitable for AI/ML training and inference at NW with pre-configured Set B to UEs.

Proposed Observation 4.3.2a

- Random Set B among Set A beams(pairs) is more suitable for AI/ML training and inference at UE side.

Please provide your view on the proposed observation 4.3.1a and observation 4.3.2a

| Companies | comments |
| --- | --- |
| OPPO | Speaking of suitableness, we think it would be better to mention from which aspect fixed Set B or random Set B is suitable. For example, in terms of beam prediction accuracy (%), we tend to observe that fixed Set B outperforms random Set B no matter where the AI/ML model is. For Observation 4.3.1a, we are fine. But for Observation 4.3.2a, could we suggest to clarify in which sense random Set B is suitable, such as flexibility? |
| Apple | - Any possible conclusion should be tied to “cases” (Case1, Case2, Case3, etc) in Section 3.1. So suggesting the proposals are changed for specifically Cases (Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.) |
| CAICT | - We have similar view as OPPO and fixed Set B could be used for NW or UE side as baseline for further performance comparison. |
| vivo | 4.3.1a: Even for NW side AI model, we don’t think it is practical to always use a fixed beam across training and inference. Performance can be degraded a lot if a specific beam in the fixed beam set B suffers loss in measurement performance, e.g., when this beam is blocked. 4.3.1b: We are okay with this observation. Both Opt B and Opt C are to be considered for random Set B. |
| Xiaomi | First we are not clear about the motivation of such observations. Second, for observation 4.3.1a, even for NW side model, it is possible that fixed set B is not suitable for different UEs with different Rx beam parameters, e.g. different number of Rx beam. And for observation 4.3.2a, if model training is performed at UE side, it can be UE specific. Thus it is suitable to realize fixed set B. |
| Google | No matter to use fixed or random set B beam, the beam measurement and quantization error could be a problem. |
| CATT | We think the fixed pattern is also suitable for UE-sided model and random pattern is also suitable for NW-sided model. There is no big different between UE-sided model and NW-sided model in evaluation. Thus, we don’t think these two observations are constructive. |
| MediaTek | We don’t support these observations. We think both selections of fix/random Set B are suitable to either gNB or UE side models. For example, for observation 4.3.1a, gNB can configure different Set B of beams based on link quality of each beam. Also, for observation 4.3.2a, since we haven’t agreed on whether UE will use just one model for different cells at this point, UE still can have the option to use different models for different cells, in that case fix Set B is also feasible for each model trained and inferenced at UE side. |
| Qualcomm | We do not see the basis for this qualitative conclusion that random and fixed Set B is better for NW-side versus UE-side training and inference. It is hard to conclude one option is “more suitable” than the other, and we would rather have the proper justification and analysis for a more targeted comparison. |
| Lenovo | - Need more clarity on suitability. What are the criteria for saying a certain set B is suitable than the other options? We think the main criteria for Set B should be such that it should be feasible to implement it in practice and it is equally important that the selected set B should not be restrictive. During the study phase, we should explore the usefulness of all possible AI/ML methods in beam management and make our agreements (including the agreement on Set B) to enable us to do so. - We support Option C under Option 2. As explained in R1-2211775, Option 2 is more generic and includes Option 1. Selecting Option 1 means we are eliminating certain types of AI/ML techniques such as reinforcement learning, sequential learning, active hypothesis learning etc. form the study. We should be open to consider and evaluate all types of learning techniques during the study phase to know the full potential of AI/ML methods. |
| Fujitsu | Share the view of Qualcomm |
| HW/HiSi | Maybe some clarification is needed, what suitable means in this context (e.g. better performance, less envisioned spec impact, less overhead)? Performance-wise, the fixed Set B is better than the a variable set B. But the generalization performance of a variable set B is better, e.g. when the model is trained with some pre-configured sets and inference is done with one of the pre-configured sets. It should be noted, though, that the performance of pre-configured set B decreases when the number of Sets increased. Thus, a good trade-off between performance and generalization capability needs to be found. |
| Spreadtrum | We think it should be discussed based on which side the AI/ML inference is conducted. If AI/ML inference is at NW side, the NW can select K beams out of Set A that have the best inference performance for the specific scenario. Thus, the random beam pattern can be used. Besides, if AI/ML inference is at UE side, a fixed beam pattern allows UE to implement a simple AI model. Also a fixed beam pattern is beneficial to achieve better inference performance. |
| Ericsson | This conclusion should be based on evaluation results, and we anyway expect such conclusion could be scenario/configuration dependent. |
| FL | This is closed. |

Moreover, considering the analysis and observations from the following two companies.

| Ericsson [9] | We consider a scheme with gNB-side inference where the UE measures a fixed set of beams, but only reports beams with RSRP exceeding a certain threshold relative to the strongest beam, i.e., only beams with an RSRP at most X dB below the RSPR of the strongest measured beam are reported. Observation 5 By allowing variable number of reported beams via UE pre-processing of measurements, the reporting overhead can be substantially reduced with little performance degradation. |
| --- | --- |
| Samsung [22] | Observation # 1: Opt A (Fixed Set B) is more suitable for BM-Case2 considering multiple reports at different time instance. Opt A may be also beneficial for BM-Case1. Observation # 2: Opt B (Change with pre-configured pattern) may be reasonable if different Set B is needed for different UEs. Observation # 3: Opt C (randomly changed among Set A) may be reasonable for AI/ML at UE side since different gNB may have different Set B. Observation # 4: Opt D (randomly changed among a pre-configured subset of Set A) require less L1-RSRP report overhead and may be more reasonable for AI/ML at gNB side. Observation # 28: Even if UE reports only half of Set B’, we can observe gain in top K/1 prediction accuracy compared to non-AI. As the number of reporting beams increases, there is a benefit, but it does not increase significantly. Proposal # 3: For the case of fixed Set B’, adopt the maximum 4 number of reported beams out of Set B’ as a baseline for evaluation. |

FL1:(closed) Question 4.3.2

Please provide your views on the following questions:

A: what is the suitable scenario for Option 2A?

B: what is the suitable scenario for Option 2B?

C: Any other scenario than proposed observation 4.3.2a for Option 2C?

D: Do you support to define Option 2B as proposed by Samsung? Or it can be treated as a special case of Option 2B?

E: Any other scenario than proposed observation 4.3.1a for Option 1(fixed Set B)?

| Companies | comments |
| --- | --- |
| FL1 | If FL’s view, Opt D proposed by Samsung can be treated as a special case of Option 2B, considering the pre-configured patterns including all the potential report from UE in Set B’ (a fixed subset of set A). But this is different from Opt C which needs to simulate for all potential beams in Set A. Companies are invited to share views of the above questions. |
| OPPO | For Option 2A, it can be suitable for BM-Case2, where the location a UE moves to can be assigned with different pre-defined beam patterns. For other randomly changed beam pattern for Set B, we failed to see strong motivation, but predictable performance loss when compared with fixed Set B. The technical reason could be due to the nature of AI/ML model for classification which is trained to output the Top-K with the same input. |
| vivo | Our understanding of Opt 2B is gNB can freely select a beam set B in a pre-determined sets B for measurement in either training and inference. This can be used in both NW-side model and UE-side model to make sure the better beams can be selected by gNB for set B. |
| Xiaomi | For option 2A, it may be suitable for BM-case 2, different set B can be used for different history measurement instance. For Option 2B, for DL Tx-Rx beam pairs prediction, some set B such as a set B with only one Rx beam can’t provide good beam prediction accuracy for predicting DL Tx-Rx beam pairs with multiple Rx beams and it is better to preclude such set B. So set B including Tx-Rx beam pairs with different Rx beams can be used. And considering different number of Rx beams, different set B with different number of Rx beams can provide good generalization performance. |
| InterDigital | For us, the difference between Option 2A and 2B is not very clear. For example, if the random change is based on an equation with seed (e.g., UE ID), we believe that this is also based on a predefined pattern. |
| Google | Probably the scenario for option 2A/2B/2C could be the coverage for some set B beams is not good to provide accurate beam measurement results for inference. |
| MediaTek | A,B: For Option 2A and Option2B, we think both options can be a suitable candidate for BM-Case2. For example, if the future trajectory of a UE is known (e.g. highway case), a fixed pre-configured pattern of variable Set B (Option2A) can be used. On the other hand, if the trajectory of a UE is unknown and beam blockage frequently happens for fix Set B, Option 2B can be used. D: In our understanding, the motivation of OptD proposed by Samsung is to cover the issue when not all the beams that UE measured is reported, which is valid because now the definition of Set B is the input of the AI/ML model, not the set of beams that UE measures. We don’t think it is a special case of Option2B since the set of beams whose measurement are reported are not pre-configured and might depend on the measurement result (e.g., for a beam with very low RSRP, such beam measurement might not be used as AI/ML input). |
| HW/HiSi | In our understanding, option 2A is a special case of Option 2B, it could be seen as one realization out of the randomly selected set B. A possible use case could be to increase the generalization performance. As mentioned in the previous reply, the performance of pre-configured set B decreases when the number of Sets increased. Thus, a good trade-off between performance and generalization capability needs to be found. |
| NTT DOCOMO | Support Observation 4.3.1a. As of Observation 4.3.2a, we think it is better to specify which Opt in Opt2 is more suitable for UE side model. We think the Set B of UE side model can be the same for each gNB. Hence, the variable pre-configured Set B is sufficient and Opt2C is unnecessary even for UE side model. |
| CATT | The same view as Question 4.3.1. Regarding the evaluation results, it cannot differentiate with NW-sided and UE-sided, even for multiple entities. Thus, we don’t think those observations are necessary. |
| Lenovo | - Please refer to our response to question 4.3.1 - Option 2C is applicable for inference at gNB side as well. |

FL2:(closed) Question 4.3.2b

Proposal 4.3.4a

- Study the following options on the selection of Set B of beams (pairs)

  - Option 1: Set B is fixed across training and inference

  - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), FFS:

    - Opt A: Set B is changed following a set of pre-configured patterns

    - Opt B: Set B is randomly changed among pre-configured patterns

    - Opt C: Set B is randomly changed among Set A beams (pairs)

    - Opt D: Set B is randomly changed among a pre-configured subset of Set A of beams (pairs)

    - The number of beams(pairs) in Set B can be fixed or variable

    - Note: BM-Case1 and BM-Case2 may be considered for different option.

  - Other options are not precluded.

| Companies | comments |
| --- | --- |
| FL2 | The intention is to extend the options for variable Set B. please share the comment. Or any updated wording for Opt A/B/C |
| Lenovo | We should keep open the option of choosing the Set B for the companies so that companies can chose between option 1 and option 2. Under option 2, when the size and the elements of set B are variable, there is no need to further specify the options. The companies should report what Set B they have used during training and inference. Hence, we propose to modify the proposal as follows: Study the following options on the selection of Set B of beams (pairs) - Option 1: Set B is fixed across training and inference - Option 2: Set B is variable (e.g., different beams (pairs) in each time instance/report/measurement during training and/or inference) Companies to report the details of Set B that they have employed for during training and inference. |
| Xiaomi | First, we think opt D is a subcase of Opt C. Second, Currently Opt C and Opt D can only be used for set B is a subset of set A. if set B is different from Set A, Opt E should be added. #### FL2:(H) Question 4.3.2b Proposal 4.3.4a - Study the following options on the selection of Set B of beams (pairs) - Option 1: Set B is fixed across training and inference - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), FFS: - Opt A: Set B is changed following a set of pre-configured patterns - Opt B: Set B is randomly changed among pre-configured patterns - Opt C: Set B is randomly changed among Set A beams (pairs) - Opt D: Set B is randomly changed among a pre-configured subset of Set A of beams (pairs) - Opt E: Set B is randomly changed among all possible Set B patterns - The number of beams(pairs) in Set B can be fixed or variable - Note: BM-Case1 and BM-Case2 may be considered for different option. - Other options are not precluded. |
| Samsung | We support the Proposal 4.3.4a. |
| FL3 | For NW side AI, there is no configuration, but the pattens are known to NW. @xiaomi, I think options can also be used when Set B is different from Set A Proposal 4.3.4b - Study the following options on the selection of Set B of beams (pairs) - Option 1: Set B is fixed across training and inference - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), FFS: - Opt A: Set B is changed following a set of pre-configured/pre-known patterns - Opt B: Set B is randomly changed among pre-configured/ pre-known patterns - Opt C: Set B is randomly changed among Set A beams (pairs) - Opt D: Set B is randomly changed among a pre-configured/ pre-known subset of Set A of beams (pairs) - The number of beams(pairs) in Set B can be fixed or variable - Note: BM-Case1 and BM-Case2 may be considered for different option. - Other options are not precluded. |
| NTT DOCOMO | We have a suggestion to further differentiate Opt C and Opt D on Proposal 4.3.4b. Proposal 4.3.4b - Study the following options on the selection of Set B of beams (pairs) - Option 1: Set B is fixed across training and inference - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), FFS: - Opt A: Set B is changed following a set of pre-configured/pre-known patterns - Opt B: Set B is randomly changed among pre-configured/ pre-known patterns - Opt C: Set B is randomly changed among all the Set A beams (pairs) - Opt D: Set B is randomly changed among a pre-configured/ pre-known subset of Set A of beams (pairs) - The number of beams(pairs) in Set B can be fixed or variable - Note: BM-Case1 and BM-Case2 may be considered for different option. - Other options are not precluded. |

FL3:(closed) Question 4.3.2c

Proposal 4.3.4c

- Study the following options on the selection of Set B of beams (pairs)

  - Option 1: Set B is fixed across training and inference

  - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), FFS:

    - Opt A: Set B is changed following a set of pre-configured/pre-known patterns

    - Opt B: Set B is randomly changed among pre-configured/ pre-known patterns

    - Opt C: Set B is randomly changed among Set A beams (pairs)

    - Opt D: Set B is randomly changed among a set of beams (pairs) for measurement

    - The number of beams(pairs) in Set B can be fixed or variable

    - Note: BM-Case1 and BM-Case2 may be considered for different option.

  - Other options are not precluded.

| Companies | comments |
| --- | --- |
| FL3 | @DCM, I don’t see big motivation to change Opt C. The change of the wording of Opt D is to include for both cases of Set B is a subset of Set A or different from Set A. For a clarification, Opt A, B, D can be used for both set B is subset of Set A, and set B is different from set A. |
| Futurewei | In general, we are ok with the proposal but have a clarification question. As Opt B is that Set B is randomly changed among pre-configured/pre-known patterns, can we interpret Opt A is that Set B is changed in a controlled manner among a set of pre-configured/pre-known patterns? |
| CAICT | We are fine to add Opt. D as FL proposed. |
| MediaTek | We are OK with this proposal. Thank FL for updating wording in OptD. |
| Samsung | Support the proposal 4.3.4c. |
| ZTE | For option D, we think it’s not accurate to say that set B is ‘randomly’ changed among a set of beams (pairs) for measurement since set B should generally include the beams with larger measured RSRP. Besides, as with Option A/B/C, there may be multiple sets of beams for measurement. Thus, we suggest the following revision. - Opt D: Set B is randomly changed among a set(s) of beams (pairs) for measurement |
| LG | We are generally ok with the proposal by FL. |
| Spreadtrum | We are OK with this proposal. |
| Ericsson | Share the view with ZTE to delete the term “randomly” for option D. It should also be possible with deterministic change of beams. It is easiest to remove randomly than mentioning all possible alternatives. Likewise, we also think randomly can be removed for option B and C. |

FL5:(FFS) Question 4.3.2e

Please find some further clarification/examples of each option. Companies are encouraged to have further clarifications on the options (including whether it is reasonable/valid or not), and provide some evaluations for each valid option.

Proposal 4.3.4e

- Study the following options on the selection of Set B of beams (pairs)

  - Option 1: Set B is fixed across training and inference

FL: for fixed Set B, training and inference is fixed.

  - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/ report/measurement during training and/or inference), FFS:

    - Opt A: Set B is changed following a set of pre-configured/pre-known candidates of Set B patterns in pre-configured/pre-known order

FL// In my view, this may be more suitable for BM-Case2, or BM-Case1 with LSTM to processing different reports. In this option, the Set B changed with pre-known order from in each time instance/report/

measurement. In an example, the inputs of AI/ML have 2 RSRP measurements of each report, and 2 report:  Set B1 = {b1, b2} and Set B2={b3, b4}. Set A = {b1, b2, b3, b4, b5….}. From AI inputs point of view, it it requires two Set B. the order of Set B1 and Set B2 is pre-know. From another angle, Set B =Set B1+Set B2.

In this case, Set B can be different of a subset of Set A.

    - Opt B: Set B is randomly changed among pre-configured/ pre-known candidates of Set B patterns

FL// in this case, there are several pre-known set Bs, e.g., Set B1 = {b1, b2} and Set B2={b3, b4}. Different Set B may configure to different UEs. therefore, from AI input point of view, in each training/inference occasion, the measurement may from Set B1 or Set B2. Therefore, I was thinking of it is “randomly” for this option.

Different from Opt A, the AI inputs is only one candidates of Set B. and the number of candidates of Set B is limited.

In this case, Set B can be different of a subset of Set A.

    - Opt C: Set B is randomly changed among candidates of Set B which is subsets of Set A beams (pairs)

FL// in this case the Set B is from Set A, but it is not pre-known. The number of candidates of Set B can be quite many(depends on the number of beams in Set A and Set B). compared with Opt B, this may require a lot of data for training in order to make AI learns from all the potential pattens.

In this case, Set B is a subset of Set A.

    - Opt D: Set B is randomly changed among candidates of Set B which is subsets of Set C (a set of beams (pairs) for measurement)

FL// for Opt D, there may have a Set C of beam/pairs for measurement, and Set B is a subset of Set C. Set B is not fixed (if Set B is fixed, it is option 1). An example for Opt D is, UE is configured to measurement 8 beams (Set C) but only report Top 4 of beams and measurements, or some other thresold. In order to limited the possibility of candidates which may be beneficial for performance of AI/ML, AI can learn all combinations of Set C. Comparing with Opt C, it has less candidates of Set B. Opt D can be treated as a special case of Opt B (by listing all the combinations of Set C).

In this case, Set B can be different of a subset of Set A.

    - The number of beams(pairs) in Set B can be fixed or variable

    - Note: BM-Case1 and BM-Case2 may be considered for different option.

  - Other options are not precluded.

| Companies | comments |
| --- | --- |
| FL5 | Companies are encouraged to discuss (wording), analysis (feasible, scenarios) and performance on the options above. |

FL1:(closed) Question 4.3.3

Moreover, considering the analysis and observations from the following two companies on different selection of Set B.

| Qualcomm [23] | Observation 3: For BM-Case1 and the sub use case of “Set B is a subset of Set A”, and for a fixed Set B option, the beam prediction performance largely depends on how Set B is selected. For instance, if set A includes beams in both azimuth and elevation, selecting Set B beams only in azimuth dimension may adversely impact prediction performance. |
| --- | --- |
| MTK [24] | Proposal 17: Study and evaluate a more comprehensive Set B design, including joint designing the number of beams in Set B and their beam shape for spatial beam prediction. Observation 10: The selection of beams in Set B will affect the prediction accuracy of the AI/ML-based spatial beam prediction. Observation 11: The spatial beam prediction by using multi-arm beam design in Set B performs better than using subset beam design in Set B. Observation 12: The spatial beam prediction by using wide beam design in Set B does not outperforms the performance by using subset beam design in Set B. |

Based on the above observation/proposals:

Proposal 4.3.3a

- Companies report the pattern of Set B.

- Further study the performance with different patterns of set B for fixed Set B.

Please share your view on proposal 4.3.3a:

| Companies | comments |
| --- | --- |
| FL1 | From FL point of views, it will be good to provide some observation on the selection of fixed Set B pattern with best effort. |
| OPPO | We agree with the assessment from QC that the beam selection of Set B highly impact the performance of beam prediction. Thus, we are fine with the FL proposal to report the pattern of Set B during training phase and inference phase. |
| Futurewei | We are ok with the proposal. |
| LG | Fine with the proposal. |
| vivo | We are also with this proposal. |
| Xiaomi | We are fine with the proposal to report the pattern of set B by each company. |
| InterDigital | Fine with the proposal. |
| Google | OK |
| MediaTek | We support this proposal. |
| Qualcomm | Support |
| Lenovo | Support. We propose to modify the proposal as follows: - Companies report the pattern of Set B. - Further study the performance with different patterns of set B for fixed Set B. |
| Fujitsu | Fine with the proposal. |
| HW/HiSi | We do not object the proposal, but how would the Set B then reported? Is the intention to have it in the table, or is it reported separately? |
| NTT DOCOMO | Support |
| CAICT | We are fine to have some further study on the pattern of Set B. |
| FL3 | Agreement - Companies report the pattern of Set B. - Further study the performance with different patterns of set B(s) for fixed Set B (Option 1) and different pre-configured/pre-known patterns of Set B(s) (Option 2A and 2B). |

  - (Medium)Assumption of Rx beams for DL Tx beam prediction

| Company | Proposal/observation |
| --- | --- |
| Vivo [3] | Proposal 19: Study different Rx beam assumptions in DL Tx beam prediction scheme, such as best Rx beam, 2nd best Rx beam, random Rx beam, and worst Rx beam, which may bring various performance evaluation results, at least in BM-Case1. Proposal 23: Further study performance comparison between enhanced beam pair prediction and DL Tx beam prediction with various Rx beam assumptions, such as worst Rx beam, second best Rx beam, random Rx beam per sample, etc., in BM-Case2. - Observation 11: Significant performance deterioration can be observed in two-step beam prediction with non-best Rx beam, even for the 2nd best Rx beam. - Observation 12: The performance of two-step beam prediction with the best Rx beam provides considerable improvement, as decreased prediction difficulty from predicting 256 beam pairs to 32 beam pairs by acquiring precise best Rx beam of each sample. - Observation 13: Large performance deterioration can be observed if the Rx beam assumptions of training and inference are different for DL Tx beam prediction scheme. |
| ZTE [4] | Observation 5: The AI based Tx beam prediction with the optimal Rx beam is slightly better than that of the AI based method with a fixed (e.g, first) Rx beam. Observation 8: For AI/ML based temporal beam prediction, Tx beam prediction obtains a better performance than that of the Tx-Rx beam pair prediction under the optimal Rx beam or a fixed Rx beam. |
| DoCoMo [21] | Observation 1: The performance of AI/ML based beam prediction BM Case-1 is acceptable even when different Rx beams are assumed for training and inference. |
| Samsung [24] | Observation # 9: Using the L1-RSRP of the “best” Rx beam with exhaustive beam sweep as inputs can provide the best performance for the accuracy of Top-1/N beam prediction than fixed or randomly selected one or two Rx beams with fixed or random Tx beams for BM-Case 1. Observation # 10: With L1-RSRPs of fixed Rx beam(s) as AI inputs can provide better performance than L1-RSRP of random Rx beam(s) for DL Tx beam prediction for BM-Case 1. Observation # 20: Using the L1-RSRP of the “best” Rx beam with exhaustive beam sweep as inputs can provide the best performance for the accuracy of Top-1/N beam prediction than fixed one Rx beam or randomly selected one or two Rx beams with fixed or random Tx beams for DL Tx-Rx beam pair prediction for BM-Case 1. Observation # 21: With L1-RSRPs of fixed Rx beam(s) as AI inputs can provide better performance than L1-RSRP of random Rx beam(s) for DL Tx-Rx beam pair prediction for BM-Case 1. |
| Nokia/NSB [24] | Observation 9: Selecting the Rx beam by following the configured QCL-D info may or may not be the optimum choice for the beam pair prediction. Proposal 3: Advanced Rx beam selection procedure other than following the configured QCL-D info should be considered for beam pair prediction. |

FL1:(closed) Question 4.4a

Proposal 4.4a

At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam:

- Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping

- Option 2: Measurements of fixed Rx beam(s)

- Option 3: Measurements of random Rx beam(s)

- Other options are not precluded and can be reported by companies.

Please provide your view on the proposal 4.4a.

| Companies | comments |
| --- | --- |
| FL1 | In feature lead’s view, there is no need to list the option with worse Rx beam, random Rx beam should be sufficient to verify the performance. In practical, it doesn’t make sense to find the worse Rx beam and use the report to predict for the best beam(s). |
| NTT DOCOMO | Ok with the proposal. |
| vivo | Ok with the proposal. |
| Xiaomi | We are not clear about the use case of Option 3. |
| InterDigital | Fine with the proposal. |
| Google | OK, but we think the measurement accuracy for option 2/3 could be a big problem. |
| CATT | Fine with the proposal |
| MediaTek | We prefer Option1 but we are OK with this proposal. |
| Qualcomm | OK |
| Lenovo | Prefer option 1. |
| Fujitsu | it’s not clear for the reason of option 3 |
| HW/HiSi | Option 1 and 2 seem reasonable. For Option 3, is the intention that different Tx beams are measured with different Rx beams? |
| FL | Proposal 4.4b At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam: - Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping - Option 2: Measurements of fixed Rx beam(s) - FFS: Option 3: Measurements of random Rx beam(s) - Other options are not precluded and can be reported by companies. |
| Lenovo | Prefer option 1. |
| ZTE | Fine with the proposal. |
| Ericsson | Ok with the proposal |

FL2:(closed) Question 4.4b

Proposal 4.4b

At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for inputs for AI/ML model

- Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping

- Option 2a: Measurements of specific Rx beam(s) per inference occasion

- Option 2b: Measurements of specific Rx beam(s) for all inference

- Option 3: Measurements of random Rx beam(s) per inference occasion

- Other options are not precluded and can be reported by companies.

| Companies | comments |
| --- | --- |
| FL2 | Based on some offline discussion, the inputs for training may not be needed. Companies may have different format of inputs. For example, for Option 1/2a, maybe no need to provide Rx beam information to AI model. Alternatively, for Option 2a/2b/3, Rx beam index may be implicitly provided to the AI model. The intention of this proposal it for evaluation purpose that companies can report their assumption on Rx beams. And hopefully, some observations can be made based on the evaluation results, for example, whether there is performance difference when there is no Rx beam information, etc |
| HW/HiSi | In our understanding, as commented in the off-line, how to select the specific beam is important for the evaluation. When utilizing prior information, the UE could already have a good understanding about which RX beam would be best to use. Another option, would be that to select one of the available RX beams in a pre-defined manner (e.g. round-robin). We think that this could be further studied or the assumptions on the fixed beam could be reported. To reflect this we suggest to update the proposal as follows. Updated Proposal 4.4b At least for evaluation on the performance o f DL Tx beam prediction, consider the following options for Rx beam for inputs for AI/ML model - Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping - Option 2: Measurements of specific Rx beam(s - Option 2a: Measurements of specific Rx beam(s) per inference occasion - Option 2b: Measurements of specific Rx beam(s) for all inference - FFS how to select the best beam (e.g. round-robin, based prior information, etc) - Option 3: Measurements of random Rx beam(s) per inference occasion - Other options are not precluded and can be reported by companies. |
| Samsung | We support proposal 4.4b. |
| Apple | We support HW’s modification. |
| Intel | Based on offline discussion, it is still not clear for us how this proposal applies to training dataset for option 2/3. The proposal mentions inference occasion for Option 2/3 but not for Option 1. Therefore, from our perspective, we would want to make this proposal more generic such that it can apply to both training and inference. We suggest the following wording update based on the version from HW. Further Updated Proposal 4.4b At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing inputs for AI/ML model for training and inference - Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample - Option 2: Measurements of specific Rx beam(s) - Option 2a: Measurements of specific Rx beam(s) per inference occasion model input sample - Option 2b: Measurements of specific Rx beam(s) for all inference model input samples - FFS how to select the best beam (e.g. round-robin, based prior information, etc) - Option 3: Measurements of random Rx beam(s) per inference occasion model input sample Other options are not precluded and can be reported by companies. |
| FL3 | Agreement At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable - Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample - Option 2: Measurements of specific Rx beam(s) - Option 2a: Measurements of specific Rx beam(s) per model input sample - Option 2b: Measurements of specific Rx beam(s) for all model input sample - FFS how to select the specific Rx beam(s) - Option 3: Measurements of random Rx beam(s) per model input sample - Other options are not precluded and can be reported by companies. |

FL5:(FFS) Question 4.4

FL5: companies are encouraged to compare/analyze the performance on the Rx assumption, including beam sweeping assumption if needed (e.g., with best Rx beam)

  - (Medium) Assumption of time domain information for BM-Case 2

FL5: in next meeting, companies are encouraged to compare the performance with different inputs (number/periodicity of instances for measurement) and outputs (if applicable)

| Company | Proposal/observation |
| --- | --- |
| Huawei/HiSi [2] | Proposal 13: If the AI/ML model outputs predict multiple time instances, if intermediate KPIs are calculated, then they are calculated for each prediction instance. Proposal 14: For AI/ML-based temporal domain beam prediction, regarding the relationship between Set A and Set B: - The size of Set B smaller than Set A should be considered as baseline. - Both can be considered in evaluations: Set B is a subset of Set A; Set B contains wide beams with full direction which are different from Set A with narrow beams. - Set B equal to Set A can be optionally used for performance comparison in evaluations. |
| Vivo [3] | Proposal 17: Further study expected information method in BM-Case2. |
| Xiaomi [11] | Proposal 4: At least for BM-Case 2, consider the following assumptions for evaluation - Periodicity of time instance(s) for prediction: - 20ms, 40ms, 80ms, 160ms, - Note: the periodicity of time instances for prediction can be same as that for measurement/report, or the periodicity of time instance for each measurement/report can be multiple times of that for prediction. - Other values can be reported by companies. - Number of time instances for prediction: - 1, 2, 4 - Other values can be reported by companies. Proposal 6: Adopt the evaluation methodologies listed below for temporal beam prediction: - Set A and set B are the same set. - The periodicity of future time instance can be 80ms/160ms - The periodicity of future time instance can be same or 1/N of history measurement instance - AI model: - Input: - L1-RSRP of set B in 4 history measurement instances - Output - Top K beams of set A in 1/2/4 future instances - KPI: - Consider Beam prediction accuracy and RS overhead reduction related KPIs with high priority. |
| Intel [12] | Observation 2: Ping-pong effect is observed with the best beam index selection among close time-domain samples, but the measured RSRP of best beam pairs during ping-pong effect can be very small. Observation 3: Ping-pong effect for best beam indices can be reduced by thresholding (hysteresis implementation) and/or smoothening the RSRP for best beam selection. Proposal 11: RAN1 should further discuss input sample length and the number of beam changes or beam dwelling time for BM-Case 2 to ensure model performance is not misleading |
| MTK [24] | Proposal 8: Evaluate the impact of different observation and prediction window sizes to the performance of AI/ML temporal beam prediction. Proposal 10: When the prediction window size is fixed, evaluate and study the optimal observation window size in terms of prediction accuracy and RS overhead. Proposal 11: Study more scenarios where additional information may improve the temporal beam prediction performance. Observation 2: By fixing the observation window size, the accuracy performance becomes better when prediction window size is lower. Observation 3: By fixing the prediction window size, the accuracy performance increases when the observation window size increases. However, the performance will saturate. |
| Nokia/NSB [25] | Observation 23: In BM-Case2, compared to baseline option 2, the ML method shows beam prediction performance improvement in different future time instants even when the testing prediction window is longer than the training prediction window, i.e. [F+1, …, F’]. Observation 24: In BM-Case2, the ML method only using beam RSRP input will be more useful for beam prediction for a relatively short future period. Proposal 11: For BM-Case2, RAN1 further evaluates different values for K, F, and their ratio K/F with different UE speeds. Proposal 13: Support RAN1 to further study scenarios/ trajectory models for verify the ML model generalization for BM-Case2. |

  - (Medium) Assistance information

FL5: in next meeting, companies are encouraged to report the performance with assistance information, as well as the feasibility of the assistance information

Assistance information were discussed and some observations are summarized:

| Company | Proposal/observation |
| --- | --- |
| Huawei/HiSi[2] | Proposal 8: For the evaluation of assistance information, - Information that in Rel-17 is regarded as proprietary and/or privacy shall not be considered as a candidate for assistance information - The benefits of other potential assistance information should be evaluated in 9.2.3.1 firstly, before a study on potential spec impact is conducted in 9.2.3.2 |
| Vivo[3] | Observation 7: Compared with Set 5, assistance information brings considerable gain in random subset selection scheme, especially for Tx/Rx beam angle as assistant information. Proposal 13: Assistance information, such as Tx/Rx beam ID or angle in connection with input RSRPs, should be used as AI input with random subset selection for both BM-Case1 and BM-Case2. Proposal 14: Suggest to use both Tx and Rx beam information as assistance information for further performance improvement in random subset selection. Proposal 18: Further study multiple expected beam information simultaneously used in AI input. Proposal 21: Further study assistance information, such as beam shape pattern, 3dB beam width, etc., as model input to address performance deterioration for generalization of different beam shapes in BM-Case1. Proposal 25: Further study assistance information, such as beam shape pattern, 3dB beam width, etc., as model input to address performance deterioration for generalization of different beam shapes in BM-Case2. Proposal 26: Suggest to use beam pointing angle or other physical IDs reflecting beam pointing angle information as assistance information for AI model input. |
| Ericsson[9] | Observation 6 UE location as assistance information can substantially improve prediction performance for outdoor UEs. |
| APPLE[18] | Proposal 3: considering generalization performance can be poor for AI models trained without Tx beam shape information, consider NW-trained cell-specific AI models for AI enabled beam management |
| Qualcomm[23] | Proposal4: For BM-Case1 and BM-Case2, study the performance of AI/ML models with and without incorporating assistance information and compare the performance. - Study the existing trade-offs including overhead required for singling of assistance information and corresponding performance benefits. - The agreed KPIs related to beam prediction accuracy and RS overhead reduction can be used for performance evaluation. - Examples of such assistance information: information about gNB beam shape, beam boresight directions, 3dB beamwidth, etc. Observation 4: For spatial domain beam prediction (BM-Case1), for at least the case in which Set B is a subset of Set A, assistance information from gNB about gNB beam boresight directions and information about gNB antenna array structure is beneficial in boosting spectral efficiency across UEs. |
| MTK[24] | Observation 6: Temporal beam prediction by adding additional UE angle information directly to the input of the model did not show significant gains compared to predicting without UE angle information. Observation 17: The spatial prediction accuracy does not improve much by using UE angles directly as the additional input, at least for the ratio of Set B and Set A sizes is between 1/6 to 1. Observation 18: The spatial prediction accuracy does not improve much by using UE angles directly as the additional input, under various selections of Set B. |
| Nokia/NSB [25] | Observation 3: For BM-Case1, Set B RSRP may not be sufficient for beam prediction input in certain cases. Observation 4: For BM-Case1, the ML model using as input only RSRP measurements has performances that reduce significantly by changing the number of RSRP measurements from 8 to 4, i.e. further down sampling Set A, from a ratio of ¼ to a ratio of 1/8. Observation 5: For BM-Case1, when the ML model uses the UE angle as the assistance information, it has a better performance than all the other variants. Observation 6: For BM-Case1, the ML model using as input RSRP measurements and UE Position has performances that outweigh the performance of the ML model using only RSRP. Observation 7: For BM-Case1, using assistance information like Beam Angle and Beam ID related to the measured beams may not significantly improve the performance of the ML model using as input only RSRP with a fixed pattern. Proposal 1: For BM-Case1, RAN1 further study the use of assistance information at the ML model input. The following assistance information can be prioritized: • the beam angle and/or the beam boresight direction for the measured DL Tx beams from NW to UE. • the UE position information. • the UE’s angle relative to a panel array of the gNB Proposal 12: For BM-Case2, RAN1 further verifies whether there is any use of using assistance information at the input of the ML model for different UE speeds and trajectory. The UE position information can be prioritized. Observation 24: In BM-Case2, the ML method only using beam RSRP input will be more useful for beam prediction for a relatively short future period. Observation 25: For BM-Case2, the ML model using as input only RSRPs has performance that decreases when Set B is a subset of Set A and if no advanced algorithm is applied for beam selection in Set B. Observation 26: For BM-Case2, the ML model using as input only RSRPs has performance that decreases when increasing the length of the prediction window. Observation 27: For the UE speed of 30 Km/h and prediction windows of 40 and 80 ms, the ML model using as input RSRP and assistance info (UE position) does not provide significant gains to the ML model using as input only RSRP. Proposal 12: For BM-Case2, RAN1 further verifies whether there is any use of using assistance information at the input of the ML model for different UE speeds and trajectory. The UE position information can be prioritized. |

Will be discussed in 9.2.3.2.

  - (High) Assumptions for beam management procedures

Huawei/HiSi[2]

Figure 2 below provides a flow chart to illustrate how the AI/ML-based BM is operated. In the outlined approach, the Network-side AI/ML model is considered where the AI/ML model is assumed to be trained and performs inference at the gNB side. Supervised learning is considered in this evaluation, where the UE feeds back the L1-RSRP for each Tx beam as the ground-truth information for training input. For the AI/ML training phase, the gNB performs beam sweeping over sparse (narrow or wide) beams in Set B, and the UE feeds back the L1-RSRPs of the sparse beams and the optimal beam ID over the full beam set (i.e., Set A) to the gNB, which then will be used to train the AI/ML model. When the trained AI/ML model is used for inference, the gNB will sweep the sparse beams for the 1st round sweeping (e.g., 16 beams), and the UE will report the corresponding L1-RSRPs for all the measured sparse beams to the gNB for inferring the Top-K beams. CSI-RS beam sweeping based on the inferred Top-K beams will then be carried out in the 2nd round sweeping of P-2 as in the legacy system, and the optimal Tx beam from the Top-K beams will then be fed back from the UE. The Tx beam sweeping/prediction procedure is performed for each Rx beam, so the globally best Tx beam is reported after sweeping the Rx beams.

Figure 2. Flow chart for AI/ML-based spatial domain beam management

Vivo [3]

One important issue for AI based BM study is whether we need to support P1 BM procedure (beam pair prediction) or P2 and P3 beam management procedures as in the current NR specification. In P1, AI predicts RSRPs of all beam pairs based on measurement with both Tx beams and Rx beams. For P2 or P3, AI only predicts Tx beam RSRPs or Rx beam RSRPs based on measurement with only Tx beams or Rx beams. For example, in P2, a given number of Tx beams configured along with beam management resources are received by UE with the best Rx beam acquired from previous P3 processing. Then, the best Tx beam and its beam quality can be obtained in gNB by relative beam report with maximum beam index and its L1-RSRP. Consequently, during P2 processing, there is no need to predict any Rx beam information from gNB’ perspective.

Thus, in addition to enhanced beam pair prediction scheme, i.e. beam pair prediction with expected beam information, 2-step beam prediction scheme will be studied to imitate P2 + P3 beam management process in below two sub use cases, i.e.  spatial domain beam prediction and temporal domain beam prediction. Besides, we will focus on spatial domain beam prediction with P2 processing AI model firstly with exhaustive P3 beam searching to simplify the modeling process and performance evaluation.

Ericsson [9]

The following three options for beam prediction have been considered in evaluations:

Option 1) AI/ML model predicts Top-1 beam

Option 2) AI/ML model predicts Top-K beams

For example, if Set B is SSB beams, we have the following steps:

P1. Measure the SSB beams

P1*. Predict a set of K CSI-RS beams

P2. Measure the K CSI-RS beams

FL5:(FFS) Question 4.7a

Proposal 4.7a

For both BM-Case1 and BM-Case2, further study the following options to apply AI/ML in beam management procedure,

- Option 1: Use AI/ML model to predict Top-1 DL Tx beam in Set A

  - For UE side AI/ML, the Top-1 DL Tx beam can be recommended by UE for DL data transmission

  - For gNB AI/ML, the Top-1 DL Tx beam can be used for DL data transmission

  - P3 may be needed for Rx beam sweeping for the predicted Top-1 DL Tx beam

- Option 2: Use AI/ML model to predict Top-K Tx beams in Set A

  - For UE side AI/ML, the Top-K Tx beams can be recommended by UE for P2

  - For gNB AI/ML, the Top-K DL Tx beam can be used for P2

  - P2 is used to select Top-1 DL Tx beam for DL data transmission

  - P3 may be needed for Rx beam sweeping for the predicted Top-1 DL Tx beam

- Option 3: Use AI/ML model to predict Top-1 Tx-Rx beam pair in Set A

  - For UE side AI/ML,

    - the Top-1 Tx-Rx beam pair can be used for DL data reception

    - the Tx beam of the Top-1 can be recommended by UE for DL data transmission

    - P3 procedure is not needed

  - For gNB side AI/ML,

    - the Tx beam of the Top-1 Tx-Rx beam pair can be used for DL data transmission

    - [the Rx beam of Top-1 Tx beams can be configured/recommended to UE]

    - [P3 procedure is not needed if the Rx beam of Top-1 Tx beam is configured/recommended to UE]

- Option 4: Use AI/ML model to predict Top-K Tx-Rx beam pairs in Set A

  - For UE side AI/ML,

    - the Top-K Tx-Rx beam pairs can be used for DL data reception

    - the Top-K Tx beams can be recommended by UE for P2

      - P2 is used to select Top-1 DL Tx beam for DL data transmission

    - P3 procedure is not needed

  - For gNB side AI/ML,

    - the Tx beams of the Top-K Tx-Rx beam pairs can be used for P2

      - P2 is used to select Top-1 DL Tx beam for DL data transmission

    - [the corresponding Rx beam of each Tx beam can be configured/recommended to UE]

    - [P3 procedure is not needed if Rx beams are configured/recommended to UE]

- Other options are not precluded.

| Company | Proposal/observation |
| --- | --- |
| FL5 | Proposal 4.7a summarized when/how to apply AI into beam management procedure. I tried to separate the different options from UE side vs gNB side AI/ML, DL Tx beam prediction vs Tx-Rx beam pair prediction, as well as when AI/ML tries to predict Top 1(best) beam or Top K beams. In next meeting, we can have some discussion on this issue. I think this will be very helpful for us to better understand how/how much AI/ML can help in certain KPI. Companies are encouraged to provide some analysis on this issue, and mapping the results/evaluations to certain cases. |

  - (on hold) Others

Some other input/output related discussion:

| Company | Proposal/observation |
| --- | --- |
| Huawei/HiSi[2] | Proposal 9: The evaluation for beam prediction should focus on a one-sided AI/ML model. |
| InterDigital [10] | Proposal 8: As most of implementations are using SSB for beam management to reduce RS overheads, utilization of SSB can be assumed as default RS resources. CSI-RS can be used as optional if preferred. Proposal 9: Support ‘Set B is a subset of Set A’ when Set A and Set B are utilized in a same frequency range for both temporal/spatial domain prediction. Proposal 10: Support ‘Set A and Set B are different’ when Set A and Set B are utilized in different frequency ranges for both temporal/spatial domain prediction. Proposal 11: AI/ML based beam management based on association between different frequency ranges should supported for both between FR1 and FR2-1 and between FR2-1 and FR2-2. Proposal 12: For conventional scheme to obtain performance KPIs, current specification for beam management (i.e., up to 4 CRIs with L1-RSRP/SINR or SRS based prediction) should be considered. |
| APPLE [18] | Proposal 4: for BM Case 1, further study issues for Combination 1/2/3: - [Combination 1] Model training on the NW-side, inference on the NW-side - [Combination 2] Model training on the NW-side, inference on the UE-side - [Combination 3] Model training on the UE-side, inference on the UE-side, - address the question on the feasibility of acquiring Tx beam shape information to construct an AI model which would generalize well, - address the question on the feasibility of constructing and using cell-specific AI models from the UE side. Proposal 5: for BM Case-2, further study issues for Combination 1/2/3: - [Combination 1] Model training on the NW-side, inference on the NW-side - [Combination 2] Model training on the NW-side, inference on the UE-side - [Combination 3] Model training on the UE-side, inference on the UE-side, - address the question on the feasibility of acquiring Tx beam shape information to construct an AI model which would generalize well, - address the question on the feasibility of constructing and using cell-specific AI models from the UE side. |

Will be discussed in 9.2.3.2.

Evaluation results for AI/ML in beam management

  - (on hold) Evaluation results for BM-Case 1

FL5: In this section I tried to categorize the observations for difference purpose. Together with performance analysis on Rx beam, Set B, generalization, other cases can also be studied, e.g., performance of using Tx beam or Tx-Rx pair as AI output to predict Tx beam, considering the options in proposal 4.7a.

    - General observations

| Company | Observation |
| --- | --- |
| Huawei/HiSi [2] | Observation 7: For spatial domain beam prediction, AI/ML-based schemes under the 64-DFT codebook outperform the legacy approach in most of the cases in terms of beam selection accuracy, e.g.,: • AI/ML-based Top-5 prediction reaches almost the upper performance bound with a prediction accuracy of 94.95% but with an overhead reduction of 67.17%. On the other hand, for the same overhead reduction, the established legacy Baseline approach can only achieve a prediction accuracy of 55.3% • With AI/ML-based Top-3 prediction, the overhead compared to the legacy Baseline approach can be further reduced by another 8%, while the prediction still is much higher (89.2% as opposed to 55.3%) Observation 8: For spatial domain beam prediction, AI/ML-based schemes under the 64-DFT codebook outperform the legacy approach in most of the cases in terms in terms of average L1-RSRP difference, e.g.,: • For AI/ML-based Top-5 prediction, the L1-RSRP difference compared to genie-aided beam prediction in Exhaustive 64 is as low as 0.03 dB, with an overhead reduction of 67.17%. On the other hand, for the same overhead reduction, the established legacy Baseline approach can only achieve an average L1-RSRP difference of 1.02dB • With AI/ML-based Top-3 prediction, the overhead compared to the legacy Baseline approach can be further reduced by another 8%, while the average L1-RSRP difference is still is much smaller (0.08dB as opposed to 1.02dB) Observation 11: The AI/ML-based beam prediction based on the Set A with 256 beams provides a considerable gain over the legacy upper bound Exhaustive 64 in achievable L1-RSRP for a small fraction of the overhead associated with an Exhaustive 64 sweep. |
| Ericsson [9] | Observation 1 In outdoor scenarios, AI/ML can reduce beam spatial-domain beam prediction overhead substantially while maintaining good accuracy for 4x8 (32 beams in Set A). Observation 2 In scenarios with primarily indoor UEs, spatial-domain beam predication is more challenging. Observation 3 With the adopted beam pattern, the conventional scheme could have very good performance which significantly outperforms the baseline schemes and have similar performance as AI/ML schemes. |
| InterDigital [10] | Observation 11: AI aided beam selection achieves more than 95% selection accuracy when error margin is larger than 0.5 dB by consuming 50%/33% of the measurement overhead for the exhaustive measurement. Observation 12: AIML-based RSRP estimation always outperforms the baseline especially when less RSRP measurements are available as it achieves a higher selection accuracy by 35% when error margin is 0.5 dB. |
| Xiaomi [11] | Observation 1: AI based beam prediction in spatial domain can provide good performance. And the performance can be further improved by inputting corresponding beam pair ID in addition to measured L1-RSRP or by inputting L1-RSRP of same beam pair IDs. |
| OPPO [13] | - Spatial domain beam prediction can yield beam prediction accuracy (at least 80%) while overhead/latency reduction rate is 75%. - The system level metric, i.e. spectrum efficiency or throughput, is not sensitive to the L1-RSRP difference introduced by spatial domain beam prediction. - : CDF of the L1-RSRP gap for the Top-1 predicted beam - For 80% of the incorrect spatial domain beam prediction cases, the L1-RSRP difference can be kept within 2dB. |
| NVIDIA [16] | Observation 2: AI/ML-based algorithms for beam prediction in spatial domain can achieve performance comparable to that of exhaustive beam search, while the reference signal overhead, measurement effort, reporting overhead, and latency can be much reduced. |
| Samsung [22] | Observation # 7: For spatial domain prediction, AI can provide better performance in terms of beam prediction accuracy than non-AI based scheme with the measurements of a given subset of beams to select a best beam among a full set of beams. Observation # 8: With the help of AI, SSB/RS overhead for measurements, UE measurement efforts, reporting overheads can be reduced to achieve a target performance for beam selection. |

    - Performance of DL Tx beam prediction

| Company | Observation |
| --- | --- |
| CMCC [15] | Observation 2: Compared with baseline option 1, BM-Case 1 with DL Tx beam prediction has minor loss of prediction accuracy for Top-2 and Top-3 beam prediction but has large beam sweeping overhead reduction. |

    - Performance of Tx-Rx beam pair prediction

| Company | Observation |
| --- | --- |
| Ericsson [9] | - Joint TX/RX prediction can give quite good performance while significantly reducing RS overhead compared to measurements of all RX beams for each TX beam in Set B. |
| CMCC [15] | Observation 1: Compared with baseline option 1, BM-Case 1 with Tx-Rx beam pair prediction has minor loss of prediction accuracy for Top-3 beam pair but has large beam sweeping overhead reduction. |

    - Comparison between DL Tx beam and Tx-Rx beam pair

| Company | Observation |
| --- | --- |
| Samsung [22] | Observation # 23: For DL Tx beam prediction in BM-Case 1, L1-RSRPs with implicit Tx beam index as AI inputs and best Tx beam as AI outputs and can provide a better performance than with L1-RSRPs with implicit Tx beam index and Rx beam index as AI inputs and best Tx-Rx beam pair as AI outputs. |
| MTK [24] | Observation 20: For spatial beam prediction, the prediction performance of the best beam pair by using DNN is significantly worse than predicting the best Tx beam. |
| CEWiT [26] | Observation 1: Compared to the spatial domain Tx-Rx beam pair prediction, spatial domain Tx beam prediction achieves a better accuracy with lower mean RSRP difference. |

    - Fixed Set B vs variable Set B

| Company | Observation |
| --- | --- |
| Futurewei [1] | Observation 1: In spatial-domain beam prediction using fixed Set B beam pattern in AI/ML model training phase, performance degrades significantly when Set B beam pattern changes in model inference phase. Observation 2: In spatial-domain beam prediction using fixed Set B beam pattern in AI/ML model training phase, performance degrades significantly when at least one of the beam pairs in Set B is missing during model inference phase. Observation 3: In spatial-domain beam prediction using variable Set B beam patterns in AI/ML model training phase, performance degrades significantly when Set B beam patterns changes in model inference phase. Observation 4: In spatial-domain beam prediction using variable Set B beam patterns in AI/ML model training phase, when Set B pattern changes in model inference phase, less performance degradation is observed when only a subset of the beam patterns used as input in the inference phase are different from the beam patterns used in the training phase compared to when all the beam patterns used as input in inference phase are different than the training phase. |
| Huawei/HiSi[2] | Observation 10: Variable pattern selected from pre-configured patterns (Option 2B) can achieve close performance as the fixed pattern approach but with better generalization on different patterns. |
| Vivo [3] | Observation 1: Fixed subset selection scheme with different fixed patterns brings tremendous performance difference. Observation 2: Better performance gain can be obtained for one fixed subset selected by well-designed rule or enumerated with predefined searching criterion. Observation 3: The performance with different training and validation fixed subsets is quite poor and not acceptable, i.e., fixed set B selection scheme suffers serious generalization issue. Observation 4: Fixed beam subset in Set B can have good performance in ideal scenarios but it lacks flexibility. Issues like blockage and inter-cell interference can bring negative impact on the performance of fixed subset. Observation 5: Random subset selection scheme, which allows multiple random subsets in training, can improve generalization performance as well as beam management related performance if compared to mismatched subset with always using one subset in training. Observation 6: Set 5 with random beam subset still suffers tremendous performance deterioration due to huge number of combinations of selecting a target number of beams from total beam pairs. Observation 8: Restricting the selection of random subset from pre-searched best X beam subsets can improve the performance of BM Case 1 prediction. Such semi-random selection with Tx/Rx beam angle information as input barely suffers performance loss compared with the best beam subset. Observation 9: Semi-random beam subset scheme has potential to approach the performance upper bound, i.e. the best fixed subset, if the performance of each subset in top-N best subsets has similar performance of top-1 best subset. |
| ZTE [4] | Observation 2: The spatial domain beam prediction with a fixed beam pattern achieves a sufficiently high performance with only 25% beam overhead being used. Observation 3: With the same sampling rate on the whole beam space, the Tx beam prediction obtains a better performance than that of the Tx-Rx beam pair prediction. |
| CMCC [15] | Observation 2: Fixed Set B (Option 1) achieves better performance than randomly chosen pre-configured pattern schemes for BM-Case 1 beam pair prediction. |
| CAICT [20] | Observation 1: AI-based solution could achieve good performance for BM-Case 1 beam pair prediction with same training and validation set configurations. |
| Samsung [22] | Observation # 11: With L1-RSRP of fixed Tx beams in Set B of beams as AI inputs can provide better performance than with random Tx beam in Set B of beams for DL Tx beam prediction for BM-Case 1. Observation # 12: With large number of beams in Set B, e.g., ¼ beams of Set A, fixed Set B or pre-known different patterns in each time step has similar performance. Observation # 13: When the number of beams in Set B is smaller, e.g., 1/8 beams of Set A, pre-known different patterns in each timestep can provide better performance for the KPIs allows some error. However, for the accuracy of Top 1 beam, fixed pattern has better performance. Observation # 14: With measurements of more beams of Set B in one timestep, the performance is better performance than with the measurements of beams of Set B collected from different timesteps. |
| Qualcomm [23] | Observation 3: For BM-Case1 and the sub use case of “Set B is a subset of Set A”, and for a fixed Set B option, the beam prediction performance largely depends on how Set B is selected. - For instance, if set A includes beams in both azimuth and elevation, selecting Set B beams only in azimuth dimension may adversely impact prediction performance. |

    - Set B is different from Set A

| Company | Proposal/observations |
| --- | --- |
| Huawei/HiSi [2] | Observation 9: It can be observed that better prediction accuracy is achieved when Set B is a subset of Set A compared to the case where Set B is a wide beam set, especially when K=1; with the increase of K, the gap between two options becomes narrower. Observation 11: The AI/ML-based beam prediction based on the Set A with 256 beams provides a considerable gain over the legacy upper bound Exhaustive 64 in achievable L1-RSRP for a small fraction of the overhead associated with an Exhaustive 64 sweep. |
| Samsung [22] | Observation # 17: For spatial domain prediction, AI can help gNB to predict the best narrow beam set that including the best narrow beam for UE to measure with high probability. |
| Nokia/NSB [25] | Observation 8: For Set B is different to Set A with Set B is wide beam, the KPI for the wide beam codebook design should be both prediction accuracy and throughput performance. Proposal 2: For BM-Case1, RAN1 may further study the case of Set A/B are DL Tx and Set B/Set A are different. • Set B is a wide beam codebook and Set A is a refined beam codebook • Advance Set B designs are needed to provide sufficient refined beam prediction performance. |

  - (on hold) Evaluation results for BM-Case2

| Company | Observation |
| --- | --- |
| Fujitsu [5] | Observation 2: For fixed set B constructed with predefined even-sampling rate (4 or 8) from beams of Set A, - Comparing with the exhaustive beam sweeping, the RS overhead is reduced 75%@4 sampling rate and 87.5% @8 sampling rate for Top-1 predicted beam. - The lower sampling rate achieves the better performance. - The beam prediction accuracy of Top-2 is larger than 80% for both cases (88%@4 sampling rate and 81%@8 sampling rate). - The average L1-RSRP difference is 1.6dB@4 sampling rate and 3.1dB@8 sampling rate respectively. - The 70% probability is achieved with less than 2dB@4 sampling rate and 3.5dB@8 sampling rate for the estimated L1-RSRP difference of Top-1 predicted beam. - For the complexity of AI/ML model, - The AI/ML model has total 427 parameters with single-float data type. The computational complexity of AI/ML model is about 426K float point of operations. |
| Huawei/HiSi [2] | Observation 12: For temporal beam prediction, AI/ML based methods are more robust than legacy approaches to variations of the UE speed. - When the time instance is 0.08s in the observation and prediction window, for UE speed 30km/h, the AI/ML Top-8 approach is 42% better than for the legacy baseline but for a UE speed of 90 km/h, the AI/ML Top-8 prediction accuracy is 47% better than for the legacy baseline - When the time interval is 0.16s in the observation and prediction window, for UE speed 30km/h, the AI/ML Top-8 approach is 48% better than for the legacy baseline but for UE speed 90 km/h, the AI/ML Top-8 prediction accuracy is 77% better than for the legacy baseline. Observation 13: For temporal beam prediction, lower spatial consistency has more impact on the prediction accuracy achieved by the legacy approach than on accuracy achieved by the AI/ML-based methods. This can be seen from the results when different time instances are evaluated. - For UE at 30km/h, the accuracy of AI/ML Top-8 degrades 3.35% but the baseline degrades 4.8% when stretching the two prediction instances from 0.08s to 0.16s - For UE at 90km/h, the accuracy of AI/Ml Top-8 degrades 0.93% but the baseline degrades 9.56% when stretching the two prediction instances from 0.08s to 0.16s |
| Vivo[3] | Observation xx: For BM-Case2, compared with non-AI scheme, beam pair prediction scheme improves beam prediction accuracy and reduces average L1-RSRP difference significantly. Observation 16: For BM-Case2, compared with non-AI 2-step scheme, AI based 2-step scheme improves beam prediction accuracy and reduces average L1-RSRP difference significantly. |
| Ericsson [9] | Observation 9 The observed prediction performance improvement over baseline when number of beams in set B is <=16 is mainly due to the spatial domain prediction ability Observation 10 With set A equal to set B and having 30/60 km/h straight line moving UEs with/without rotation, AI/ML temporal TX beam prediction at T2=40ms, 160mss, 240ms shows limited gain over baseline method due to the slow variations in the propagation. |
| InterDigital [10] | Observation 13: AIML-based classification has a higher accuracy in predicting the best beam in the future prediction window frames. Observation 14: AIML-based regression has a lower RMSE than linear regression and a lower rate of decay with predicting RSRP values in further future frames. |
| Xiaomi [11] | Observation 6: both AI based beam prediction scheme 1 and scheme 2 in time domain can provide good performance. - Scheme 1 assumes same periodicity for history measurement instance and future time instance. - Scheme 2 assumes that periodicity for history measurement instance is N times of future time instance. It can reduce more RS overhead than scheme 1. Observation 7: Set B < set A causes much more performance degradation compared to set B=set A for temporal beam prediction. Observation 8: The performance may degrade when larger N (history measurement instances) is assumed. |
| OPPO [13] | Observation 5: Temporal domain beam prediction can provide beam prediction accuracy (at least 77%) while overhead/latency reduction can be up to 50% (for the case of K = 4 and F = 4). Observation 6: Beam predication accuracy slightly decreases from 87.1% to 77.1% (the case of Top-1) when F increases from 1 to 4, but strongly increases from 77.1% to 98.8% (the case of F = 4) when predicted beam number increases from Top-1 to Top-4. Observation 7: For 80% of the incorrect temporal domain beam prediction cases, the L1-RSRP difference is lower than 3.5dB. Observation 8: Spatial and temporal domain beam prediction can provide beam prediction accuracy (at least 74.4%) while overhead/latency reduction can be up to 87.5% (for the case of K = 4, F = 4 and Set B = 32 beam pairs, Set A = 128 beam pairs). Observation 9: Spatial and temporal domain beam prediction can provide beam prediction accuracy (at least 64.5%) while overhead/latency reduction can be up to 87.5% (for the case of K = 8, F = 8 and Set B = 32 beam pairs, Set A = 128 beam pairs). Observation 10: For spatial and temporal domain beam prediction, the longer beam prediction period (e.g. F = 8 prediction instances), the deeper performance loss can be observed given the same measurement period (e.g. K = 8 measurement instances). |
| NVIDIA [16] | Observation 3: AI/ML-based algorithms for beam prediction in time domain can simply use a history of the best beam index to perform the prediction. Observation 4: AI/ML-based algorithms for beam prediction in time domain can help lower reference signal overhead and reduce UE’s measurement requirement. |
| Samsung [22] | Observation # 26: In the case of non-AI, there is almost no performance degradation due to the increase in target predict time. Since the coverage of beams in Set B is wide, Top-1 prediction accuracy of the selected beam in Set B slightly decreases as the target predict time increases. Observation # 27: In the case of AI, the performance is superior to non-AI, but it can be observed that it decreases as the target predict time increases. Due to the narrow coverage of beams in Set A, it would be hard for AI to learn the Top-1 beam after longer time later based on the latest measurement. |
| Nokia/NSB [26] | Observation 23: In BM-Case2, compared to baseline option 2, the ML method shows beam prediction performance improvement in different future time instants even when the testing prediction window is longer than the training prediction window, i.e. [F+1, …, F’]. Observation 24: In BM-Case2, the ML method only using beam RSRP input will be more useful for beam prediction for a relatively short future period. Observation 25: For BM-Case2, the ML model using as input only RSRPs has performance that decreases when Set B is a subset of Set A and if no advanced algorithm is applied for beam selection in Set B. Observation 26: For BM-Case2, the ML model using as input only RSRPs has performance that decreases when increasing the length of the prediction window. Observation 27: For the UE speed of 30 Km/h and prediction windows of 40 and 80 ms, the ML model using as input RSRP and assistance info (UE position) does not provide significant gains to the ML model using as input only RSRP. |

Reference

[1] R1-2210843, Continued discussion on evaluation of AI ML for beam management, FUTUREWEI

[2] R1-2210887, Evaluation on AI/ML for beam management,	Huawei, HiSilicon

[3] R1-2211000, Evaluation on AI/ML for beam management,	vivo

[4] R1-2211059, Evaluation on AI for beam management, ZTE

[5] R1-2211075 Evaluation on AI/ML for beam management,	Fujitsu

[6] R1-2211126, On Evaluation of AI/ML based Beam Management, Google

[7] R1-2211191 Evaluation on AI/ML for beam management,	CATT

[8] R1-2211229, Evaluation on AI for beam management, Spreadtrum Communications

[9] R1-2211288, Evaluation on AI/ML for beam management, Ericsson

[10] R1-221315, Discussion for evaluation on AI/ML for beam management,	InterDigital, Inc.

[11] R1-2211357 Evaluation on AI/ML for beam management,	xiaomi

[12] R1-2211395 Evaluations for AI/ML beam management,	Intel Corporation

[13] R1-2208852, Evaluation methodology and preliminary results on AI/ML for beam management, OPPO

[14] R1-2211527, Evaluation on AI/ML for beam management,	China Telecom

[15] R1-2209330 Discussion on evaluation on AI/ML for beam management,	CMCC

[16] R1-2209627 Evaluation of AI and ML for beam management,	NVIDIA

[17] R1-2211775 Evaluation on AI/ML for beam management,	Lenovo

[18] R1-2211807 Evaluation on AI/ML for beam management,	Apple

[19] R1-2211869, Evaluation on AI/ML for beam management, LG Electronics

[20] R1-2211913 Some discussions on evaluation on AI-ML for Beam management	, CAICT

[21] R1-2211979 Discussion on evaluation on AI/ML for beam management,	NTT DOCOMO, INC.

[22] R1-2212038 Evaluation on AI ML for Beam management, Samsung

[23] R1-2212110 Evaluation on AI/ML for beam management,	Qualcomm Incorporated

[24] R1-2212228 Evaluation on AI/ML for beam management,	MediaTek Inc.

[25] R1-2212329 Evaluation of ML for beam management,	Nokia, Nokia Shanghai Bell

[26] R1-2212423 Evaluation on AI/ML for beam management,	CEWiT

Appendix: Agreements

  - Agreements in RAN 1 #109e

R1-2205269	Feature lead summary #1 evaluation of AI/ML for beam management	Moderator (Samsung)

From May 17th GTW session

Agreement

- For dataset construction and performance evaluation (if applicable) for the AI/ML in beam management, system level simulation approach is adopted as baseline

  - Link level simulation is optionally adopted

Agreement

- At least for temporal beam prediction, companies report the one of spatial consistency procedures:

  - Procedure A in TR38.901

  - Procedure B in TR38.901

Agreement

- At least for temporal beam prediction, Dense Urban (macro-layer only, TR 38.913) is the basic scenario for dataset generation and performance evaluation.

  - Other scenarios are not precluded.

- For spatial-domain beam prediction, Dense Urban (macro-layer only, TR 38.913) is the basic scenario for dataset generation and performance evaluation.

  - Other scenarios are not precluded.

Agreement

- At least for spatial-domain beam prediction in initial phase of the evaluation, UE trajectory model is not necessarily to be defined.

Agreement

- At least for temporal beam prediction in initial phase of the evaluation, UE trajectory model is defined. FFS on the details.

Agreement

- UE rotation speed is reported by companies.

  - Note: UE rotation speed = 0, i.e., no UE rotation, is not precluded.

Agreement

- For AI/ML in beam management evaluation, RAN1 does not attempt to define any common AI/ML model as a baseline.

Conclusion

Further study AI/ML model generalization in beam management evaluating the inference performance of beam prediction under multiple different scenarios/configurations.

- FFS on different scenarios/configurations

- Companies report the training approach, at least including the dataset assumption for training

Agreement

- For evaluation of AI/ML in BM, the KPI may include the model complexity and computational complexity.

  - FFS: the details of model complexity and computational complexity

Agreement

- For spatial-domain beam prediction, further study the following options as baseline performance

  - Option 1: Select the best beam within Set A of beams based on the measurement of all RS resources or all possible beams of beam Set A (exhaustive beam sweeping)

    - FFS CSI-RS/SSB as the RS resources

  - Option 2: Select the best beam within Set A of beams based on the measurement of RS resources from Set B of beams

    - FFS: Set B is a subset of Set A and/or Set A consists of narrow beams and Set B consists of wide beams

    - FFS: how conventional scheme to obtain performance KPIs

    - FFS: how to determine the subset of RS resources is reported by companies

  - Other options are not precluded.

Decision: As per email decision posted on May 22nd,

Agreement

- For dataset generation and performance evaluation for AI/ML in beam management, take the parameters (if applicable) in Table 1.2-1b for Dense Urban scenario for SLS

Table 1.2-1b Assumptions for Dense Urban scenario for AI/ML in beam management

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz - SCS: 120 kHz |
| Deployment | 200m ISD, - 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) Other deployment assumption is not precluded |
| Channel mode | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE Speed | - For spatial domain beam prediction, 3km/h - For time domain beam prediction: 30km/h (baseline), 60km/h (optional) - Other values are not precluded |
| UE distribution | - FFS UEs per sector/cell for evaluation. More UEs per sector/cell for data generation is not precluded. - For spatial domain beam prediction: FFS: - Option 1: 80% indoor ,20% outdoor as in TR 38.901 - Option 2: 100% outdoor - For time domain prediction: 100% outdoor |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration | - [One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline] - [Four panels: (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2), (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ as optional] - Other assumptions are not precluded. Companies to explain TXRU weights mapping. Companies to explain beam selection. Companies to explain number of BS beams |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | [Panel structure: (M,N,P) = (1,4,2)] - 2 panels (left, right) with (Mg, Ng) = (1, 2) as baseline - Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Companies to explain number of UE beams |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| Beam correspondence | Companies to explain beam correspondence assumptions (in accordance to the two types agreed in RAN4) |
| Link adaptation | Based on CSI-RS |
| Traffic Model | FFS: - Option 1: Full buffer - Option 2: FTP model Other options are not precluded |
| Inter-panel calibration for UE | Ideal, non-ideal following 38.802 (optional) – Explain any errors |
| Control and RS overhead | Companies report details of the assumptions |
| Control channel decoding | Ideal or Non-ideal (Companies explain how it is modelled) |
| UE receiver type | MMSE-IRC as the baseline, other advanced receiver is not precluded |
| BF scheme | Companies explain what scheme is used |
| Transmission scheme | Multi-antenna port transmission schemes Note: Companies explain details of the using transmission scheme. |
| Other simulation assumptions | Companies to explain serving TRP selection Companies to explain scheduling algorithm |
| Other potential impairments | Not modelled (assumed ideal). If impairments are included, companies will report the details of the assumed impairments |
| BS Tx Power | [40 dBm] |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200m |
| BS Antenna height | 25m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |

Agreement

- For temporal beam prediction, the following options can be considered as a starting point for UE trajectory model for further study. Companies report further changes or modifications based on the following options for UE trajectory model. Other options are not precluded.

  - Option #2: Linear trajectory model with random direction change.

    - UE moving trajectory: UE will move straightly along the selected direction to the end of an time interval, where the length of the time interval is provided by using an exponential distribution with average interval length, e.g., 5s, with granularity of 100 ms.

      - UE moving direction change: At the end of the time interval, UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°].

      - UE move straightly within the time interval with the fixed speed.

    - FFS on UE orientation

  - Option #3: Linear trajectory model with random and smooth direction change.

    - UE moving trajectory: UE will change the moving direction by multiple steps within an time internal, where the length of the time interval is provided by using an exponential distribution with average interval length, e.g., 5s, with granularity of 100 ms.

      - UE moving direction change: At the end of the time interval, UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°].

      - The time interval is further broken into N sub-intervals, e.g. 100ms per sub-interval, and at the end of each sub-interval, UE change the direction by the angle of A_diff/N.

      - UE move straightly within the time sub-interval with the fixed speed.

    - FFS on UE orientation

  - Option #4: Random direction straight-line trajectories.

    - Initial UE location, moving direction and speed: UE is randomly dropped in a cell, and an initial moving direction is randomly selected, with a fixed speed.

      - The initial UE location should be randomly drop within the following blue area

where d1 is the minimum distance that UE should be away from the BS.

        - Each sector is a cell and that the cell association is geometry based.

        - During the simulation, inter-cell handover or switching should be disabled.

For training data generation

    - For each UE moving trajectory: the total length of the UE trajectory can be set as T second if it is in time, of set as D meter if it is in distance.

      - The value of T (or D) can be further discussed

      - The trajectory sampling interval granularity depends on UE speed and it can be further discussed.

    - UE can move straightly along the entire trajectory, or

    - UE can move straightly during the time interval, where the time interval is provided by using an exponential distribution with average interval length ΔT

      - UE may change the moving direction at the end of the time interval. UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°]

    - If the UE trajectory hit the cell boundary (the red line), the trajectory should be terminated.

      - If the trajectory length (in time) is less than the length of observation window + prediction window, the trajectory should be discarded.

      - At the current stage, the length of observation window + prediction window is not fixed and the companies can report their values.

    - FFS on UE orientation

- Generalization issue is FFS

Agreement

- For temporal beam prediction, further study the following options as baseline performance

  - Option 1a: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources or all possible beams from Set A of beams at the time instants within T2

  - Option 2: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources from Set B of beams at the time instants within T1

    - Companies explain the detail on how to select the best beam for T2 from Set A based on the measurements in T1

  - Where T2 is the time duration for the best beam selection, and T1 is a time duration to obtain the measurements of all the RS resource from Set B of beams.

    - T1 and T2 are aligned with those for AI/ML based methods

  - Whether Set A and Set B are the same or different depend on the sub-use case

  - Other options are not precluded.

Agreement

- For dataset generation and performance evaluation for AI/ML in beam management, take the following assumption for LLS as optional methodology

| Parameter | Value |
| --- | --- |
| Frequency | 30GHz. |
| Subcarrier spacing | 120kHz |
| Data allocation | [8 RBs] as baseline, companies can report larger number of RBs First 2 OFDM symbols for PDCCH, and following 12 OFDM symbols for data channel |
| PDCCH decoding | Ideal or Non-ideal (Companies explain how is oppler) |
| Channel model | FFS: LOS channel: CDL-D extension, DS = 100ns NLOS channel: CDL-A/B/C extension, DS = 100ns Companies explains details of extension methodology considering spatial consistency Other channel models are not precluded. |
| BS antenna configurations | - One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline - Other assumptions are not precluded. Companies to explain TXRU weights mapping. Companies to explain beam selection. Companies to explain number of BS beams |
| BS antenna element radiation pattern | Same as SLS |
| BS antenna height and antenna array downtile angle | 25m, 110° |
| UE antenna configurations | Panel structure: (M, N, P) = (1, 4, 2), - 2 panels (left, right) with (Mg, Ng) = (1, 2) as baseline - 1 panel as optional - Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Companies to explain number of UE beams |
| UE antenna element radiation pattern | Same as SLS |
| UE moving speed | Same as SLS |
| Raw data collection format | Depends on sub-use case and companies’ choice. |

Decision: As per email decision posted on May 25th,

Agreement

- For UE trajectory model, UE orientation can be independent from UE moving trajectory model. FFS on the details.

  - Other UE orientation model is not precluded.

Agreement

- Companies are encouraged to report the following aspects of AI/ML model in RAN 1 #110. FFS on whether some of aspects need be defined or reported.

  - Description of AI/ML model, e.g, NN architecture type

  - Model inputs/outputs (per sub-use case)

  - Training methodology, e.g.

    - Loss function/optimization function

    - Training/ validity /testing dataset:

      - Dataset size, number of training/ validity /test samples

      - Model validity area: e.g., whether model is trained for single sector or multiple sectors

      - Details on Model monitoring and model update, if applicable

  - Others related aspects are not precluded

Agreement

- To evaluate the performance of AI/ML in beam management, further study the following KPI options:

  - Beam prediction accuracy related KPIs, may include the following options:

    - Average L1-RSRP difference of Top-1 predicted beam

    - Beam prediction accuracy (%) for Top-1 and/or Top-K beams, FFS the definition:

      - Option 1: The beam prediction accuracy (%) is the percentage of “the Top-1 predicted beam is one of the Top-K genie-aided beams”

      - Option 2: The beam prediction accuracy (%) is the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”

    - CDF of L1-RSRP difference for Top-1 predicted beam

    - Beam prediction accuracy (%) with 1dB margin for Top-1 beam

      - The beam prediction accuracy (%) with 1dB margin is the percentage of the Top-1 predicted beam “whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the Top-1 genie-aided beam”

    - the definition of L1-RSRP difference of Top-1 predicted beam:

      - the difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam

    - Other beam prediction accuracy related KPIs are not precluded and can be reported by companies.

  - System performance related KPIs, may include the following options:

    - UE throughput: CDF of UE throughput, avg. and 5%ile UE throughput

    - RS overhead reduction at least for spatial-domain beam prediction at least for top-1 beam:

      - 1-N/M,

        - where N is the number of beams (with reference signal (SSB and/or CSI-RS)) required for measurement

        - where (FFS) M is the total number of beams

        - Note: Non-AI/ML approach based on the measurement of these M beams may be used as a baseline

      - FFS on whether to define a proper value for M for evaluation.

    - Other System performance related KPIs are not precluded and can be reported by companies.

  - Other KPIs are not precluded and can be reported by companies, for example:

    - Reporting overhead reduction: (FFS) The number of UCI report and UCI payload size, for temporal /spatial prediction

    - Latency reduction:

      - (FFS) (1 – [Total transmission time of N beams] / [Total transmission time of M beams])

        - where N is the number of beams (with reference signal (SSB and/or CSI-RS)) in the input beam set required for measurement

        - where M is the total number of beams

    - Power consumption reduction: FFS on details

Final summary in R1-2205641.

  - Agreement in RAN 1 #110

Agreement

The Following updated based on the agreements in RAN 1 #109-e is adopted

| Parameters | Values |
| --- | --- |
| UE distribution | - FFS 10 UEs per sector/cell for system performance related KPI (if supported) [e.g,, throughput] for full buffer traffic (if supported) evaluation (model inference). - X UEs per sector/cell for system performance related KPI for FTP traffic (if supported) evaluation (model inference). - Other values are not precluded - Number of UEs per/sector per cell during data collection (training/testing) is reported by companies if relevant - More UEs per sector/cell for data generation is not precluded. |
| UE Antenna Configuration | - Antenna setup and port layouts at UE: [1,2,1,4,2,1,1], 2 panels (left, right) - [Panel structure: (M,N,P) = (1,4,2)] - panels (left, right) with (Mg, Ng) = (1, 2) as baseline - Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Companies to explain number of UE beams |

Agreement

The Following updated based on the agreements in RAN 1 #109-e is adopted

| Parameters | Values |
| --- | --- |
| UE Speed | - For spatial domain beam prediction, 3km/h - For time domain beam prediction: 3km/h(optional), 30km/h (baseline), 60km/h (optional), 90km/h (optional), 120km/h (optional) - Other values are not precluded |
| UE distribution | - For spatial domain beam prediction: - Option 1: 80% indoor ,20% outdoor as in TR 38.901 - Option 2: 100% outdoor - For time domain prediction: 100% outdoor |

Agreement

- If UE orientation is modeled, it can be independently modeled from UE moving trajectory model.

  - This is not precluded that UE orientation coupled with UE moving trajectory model.

Agreement

- Study the following options on the selection of Set B of beams (pairs)

  - Option 1: Set B is fixed across training and inference

    - FFS on the beams of Set B

  - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each report/measurement during training and/or inference)

    - FFS on fixed or variable number of beams (pairs)

    - FFS on the details

  - Other options are not precluded.

  - FFS on the number of beams (pairs) in Set B

  - Note: This does not preclude the alternative that Set B is different from Set A.

Agreement

- To evaluate the performance of AI/ML in beam management at least for NW side beam prediction, UCI report overhead can be further studied as one of KPI options.

  - FFS: number of UCI reports and UCI payload size

  - Agreements in RAN 1 #110bis

Working Assumption

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

      - Note: Companies to report the ratio for dataset mixing

      - Note: number of the multiple scenarios/configurations can be larger than two

- FFS the detailed set of scenarios/configurations

- FFS other cases for generalization verification, e.g.,

      - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

Conclusion

- For system performance related KPI (if supported) evaluation (model inference), companies report either of the following traffic model:

  - Option 1: Full buffer

  - Option 2: FTP model with detail assumptions (e.g., FTP model 1, FTP model 3)

Agreement

- BS antenna configuration:

  - antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ

  - Other assumptions are not precluded

- BS Tx power for evaluation:

  - 40dBm (baseline)

  - Other values (e.g. 34 dBm) are not precluded and can be reported by companies

- UE antenna configuration (Clarification of agreement in RAN 1 #110):

  - antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right)

  - Other assumptions are not precluded

Agreement

- For the evaluation of both BM-Case1 and BM-Case2, 32 or 64 downlink Tx beams (maximum number of available beams) at NW side.

  - Other values, e.g., 256, etc, are not precluded and can be reported by companies.

- For the evaluation of both BM-Case1 and BM-Case2, 4 or 8 downlink Rx beams (maximum number of available beams) per UE panel at UE side.

  - Other values, e.g., 16, etc, are not precluded and can be reported by companies.

Agreement

- The options to evaluate beam prediction accuracy (%):

  - Top-1 (%): the percentage of “the Top-1 genie-aided beam is Top-1 predicted beam”

  - Top-K/1 (%): the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”

  - Top-1/K (%) (Optional): the percentage of “the Top-1 predicted beam is one of the Top-K genie-aided beams”

  - Where K >1 and values can be reported by companies.

Agreement

- For DL Tx beam prediction, the definition of Top-1 genie-aided Tx beam considers the following options

  - Option A, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams

  - Option B, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s)

    - FFS on specific Rx beam(s)

    - Note: specific Rx beams are subset of all Rx beams

Agreement

- For DL Tx-Rx beam pair prediction, the definition of Top-1 genie-aided Tx-Rx beam pair considers the following options:

  - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams

  - Option B: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx over all Tx beams with specific Rx beam(s)

    - FFS on specific Rx beam(s)

    - Note: specific Rx beams are subset of all Rx beams

Agreement

- For BM Case-1 and BM Case 2, to verify the generalization performance of an AI/ML model over various scenarios/configurations, the set of scenarios/configurations are considered focusing on one or more of the following aspects as a starting point:

  - Scenarios

    - Various deployment scenarios

    - Various outdoor/indoor UE distributions

    - Various UE mobility

  - Configurations

    - Various UE parameters

    - Various gNB settings

    - [Various Set B of beam(pairs)]

  - Other aspects of scenarios/configurations are not precluded

  - The selected scenarios/configurations for generalization verification may consider the AI model inference node (e.g., @UE or @gNB) and use case (e.g., BM-Case1, or BM-Case2)

- Companies to report the selected scenarios/configurations for generalization verification

- Note: other approaches for achieving good generalization performance for AI/ML-based schemes are not precluded.

Working Assumption

For both BM-Case1 and BM-Case 2, the following table is adopted as working assumption for reporting the evaluation results.

Table X. Evaluation results for [BM-Case1 or BM-Case2] without model generalization for [DL Tx beam prediction or Tx-Rx beam pair prediction or Rx beam prediction]

|  |  |  | Company A | …… |
| --- | --- | --- | --- | --- |
| Assumptions |  | Number of [beams/beam pairs] in Set A |  |  |
|  |  | Number of [beams/beam pairs] in Set B |  |  |
|  |  | Baseline scheme |  |  |
| AI/ML model input/output |  | Model input |  |  |
|  |  | Model output |  |  |
| Data Size |  | Training |  |  |
|  |  | Testing |  |  |
| AI/ML model |  | [Short model description] |  |  |
|  |  | Model complexity |  |  |
|  |  | Computational complexity |  |  |
| Evaluation results [With AI/ML / baseline] | [Beam prediction accuracy (%)] | [KPI A] |  |  |
|  |  | [KPI B] … |  |  |
|  | [L1-RSRP Diff] | [Average L1-RSRP diff] … |  |  |
|  | [System performance] | [RS overhead Reduction (%)/ RS overhead] |  |  |
|  |  | [UCI report] |  |  |
|  |  | [UPT] … |  |  |

To report the following in table caption:

      - Which side the model is deployed

Further info for the columns:

- Assumptions

  - Number of beams/beam pairs in Set A

  - Number of beams/beam pairs in Set B

  - Baseline scheme, e.g., Option 1 (exhaustive beam sweeping), Option 2(based on measurements of Set B), or baseline described by companies

  - Other assumptions can be added later based on agreements

- Model input: input type(s)

- Model output: output type(s), e.g., the best DL Tx and/or Rx beam ID, and/or L1-RSRPs of N beams(pairs)

- Dataset size, both the size of training/validation dataset and the size of test dataset

- Short model description: e.g., CNN, LSTM

- Model complexity, in terms of “number of model parameters” and/or size (e.g. Mbyte)”, and

- Computational complexity in terms of FLOPs

- Evaluation results: agreed KPIs, with AI/ML / with baseline scheme (if applicable)

Note: To report other simulation assumptions, if any.

Agreement

- Study the following options on the selection of Set B of beams (pairs)

  - Option 1: Set B is fixed across training and inference

  - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), FFS:

    - Opt A: Set B is changed following a set of pre-configured patterns

    - Opt B: Set B is randomly changed among pre-configured patterns

    - Opt C: Set B is randomly changed among Set A beams (pairs)

    - The number of beams(pairs) in Set B can be fixed or variable

    - Note: BM-Case1 and BM-Case2 may be considered for different option.

  - Other options are not precluded.

Working assumption

- For the evaluation of the overhead for BM-Case1, further study the following two metrics for potential down selection:

  - Option A: RS overhead reduction, FFS for potential down selection:

    - Option 1: RS OH reduction%=1-NM

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement

      - where M is the total number of beams (pairs) to be predicted

    - Option 2: RS OH reduction%=1-NM

      - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML

      - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme

      - Companies report the assumption on beam sweeping

    - Option 3: RS OH reduction%=1-N+PM

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement

      - where M is the total number of beams (pairs) to be predicted

      - FFS the following alternatives consider different targets (e.g., beam or beam pair) for prediction:

        - Alt1: P is the number of Top-K selected beams (pairs) for beam sweeping (if applicable)

        - Alt2: P is the number of Top-K selected beams (pairs) not in Set B for beam sweeping (if applicable)

        - Alt3: P is the number of beams used for beam sweeping to get the best Rx beam (if applicable)

        - Companies report the assumption on beam sweeping

    - Other options can be reported by companies

  - Option B: RS overhead, FFS for potential down selection:

    - Option 1: RS OH = N,

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement

    - Option 2: RS OH = N + P

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement

      - FFS the following alternatives consider different targets (e.g., beam or beam pair) for prediction:

        - Alt1: P is the number of Top-K selected beams (pairs) for beam sweeping (if applicable)

        - Alt2: P is the number of Top-K selected beams (pairs) not in Set B for beam sweeping (if applicable)

        - Alt3: P is the number of beams used for beam sweeping to get the best Rx beam (if applicable)

        - Companies report the assumption on beam sweeping

    - Other options can be reported by companies

Agreement

- At least for BM-Case 2, consider the following assumptions for evaluation

  - Periodicity of time instance for each measurement/report in T1:

    - 20ms, 40ms, 80ms, [100ms], 160ms, [960ms]

    - Other values can be reported by companies.

  - Number of time instances for measurement/report in T1 can be reported by companies.

  - Time instance(s) for prediction can be reported by companies.

  - Agreements in RAN 1 #111

    - Agreements on 15th Nov

Agreement

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

      - Note: Companies to report the ratio for dataset mixing

      - Note: number of the multiple scenarios/configurations can be larger than two

- FFS the detailed set of scenarios/configurations

- The following case for generalization verification, can be optionally considered by companies:

  - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

    - Company to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance

    - FFS: Investigate of the feasibility the fine-tuning on the UE/Network side

Agreement

- For the evaluation of the overhead for BM-Case1, adoption the following metrics:

  - RS overhead reduction,

    - Option 1: RS OH reduction%=1-NM

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML

      - where M is the total number of beams (pairs) to be predicted

    - Option 2: RS OH reduction%=1-NM

      - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - Companies report the assumption on additional measurements

    - Agreements on 16th Nov

Agreement

- Companies report the pattern of Set B.

- Further study the performance with different patterns of set B(s) for fixed Set B (Option 1) and different pre-configured/pre-known patterns of Set B(s) (Option 2A and 2B).

Agreement

For BM Case-1 and BM Case 2, to verify the generalization performance of an AI/ML model over various scenarios/configurations, additionally considering

- Various Set B of beam(pairs)

Agreement

At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable

- Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample

- Option 2: Measurements of specific Rx beam(s)

  - Option 2a: Measurements of specific Rx beam(s) per model input sample

  - Option 2b: Measurements of specific Rx beam(s) for all model input sample

  - FFS how to select the specific Rx beam(s)

- Option 3: Measurements of random Rx beam(s) per model input sample

- Other options are not precluded and can be reported by companies.

    - Agreements on 17th Nov

Agreement

- For the evaluation of the overhead for BM-Case2, adoption the following metrics:

  - RS overhead reduction,

    - Option 2: RS OH reduction%=1-NM

      - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme

      - Companies report the assumption on additional measurements

    - FFS: Option 3: RS OH reduction%=1-NLM

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each time instance

      - where M is the total number of beams (pairs) to be predicted for each time instance

      - where L is ratio of periodicity of time instance for measurements to periodicity of time instance for prediction

    - Companies report the assumption on T1 and T2 patterns

    - Other options are not precluded and can be reported by companies.