| 3GPP TR 38.835 V0.3.1 (2022-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Study on XR enhancements for NR (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Abbreviations	7

4	Introduction to Extended Reality	8

4.1	Extended Reality Types	8

4.2	Human Perception and Tracking	8

4.3	Capture, Encoding and Delivery	9

4.3.1	Video	9

4.3.2	Audio	10

4.4	XR Engines and Rendering	10

4.5	Characteristics and Requirements	10

4.5.1	General	10

4.5.2	Video	11

4.5.3	Audio	11

4.5.4	Pose Information	11

5	XR Enhancements for NR	11

5.1	 XR Awareness	11

5.1.1	General	11

5.1.2	Layer 2 Structure	12

5.2	 Power Saving Techniques	13

5.2.1	Physical Layer Enhancements	13

5.2.2	Layer 2 Enhancements	13

5.3	 Capacity Improvements Techniques	13

5.3.1	Physical Layer Enhancements	13

5.3.2	Layer 2 Enhancements	13

6	Conclusions	14

Annex A: Evaluation Methodology	15

Annex B: Evaluation Studies	16

Annex C (informative): RAN2 Agreements	17

C.1	 RAN2#119-e	17

C.2	 RAN2#119bis-e	17

Annex Z (informative): Change history	19

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is intended to capture the output of the study item on XR Enhancements for NR [10], which aims at investigating power saving and capacity enhancements techniques tailored for XR services, as well as means to provide XR-awareness in RAN.

This study follows a series of earlier studies conducted in 3GPP by SA1 [2], SA4 [5] [6] [7] and RAN1 [8]. It is  complemented by work in SA2 [12], SA4 [11] and SA6 [4].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.842: "Study on Network Controlled Interactive Service (NCIS) in the 5G System (5GS)".

[3]	3GPP TR 23.748: "Study on enhancement of support for Edge Computing in 5G Core network(5GC)".

[4]	3GPP TR 23.758: "Study on application architecture for enabling Edge Applications".

[5]	3GPP TR 26.918: "Virtual Reality (VR) media services over 3GPP".

[6]	3GPP TR 26.926: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems".

[7]	3GPP TR 26.928: "Extended Reality (XR) in 5G".

[8]	3GPP TR 38.838: "Study on XR (Extended Reality) evaluations for NR".

[9]	3GPP TR 23.700-60: "Study on architecture enhancement for XR and media services".

[10]	RP-220285: "Study on XR Enhancements for NR".

[11]	SP-210043: "Feasibility Study on Typical Traffic Characteristics for XR Services and other Media".

[12]	SP-220705: "Study on XR (Extended Reality) and media services s".

[13]	S4-220505: "LS Reply on QoS support with PDU Set granularity".

[14]	S4aV220921: "Reply LS on further details on XR traffic".

[15]	3GPP TS 38.300: "NR and NG-RAN Overall description; Stage-2".

Editor's Note: hyperlinks, responsible groups and corresponding releases are used for convenience, they can be removed once the TR is presented for approval.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Field of view: the angle of visible field expressed in degrees measured from the focal point.

PDU Set: A PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. a frame or video slice for XRM Services, as used in TR 26.926 [6]). In some implementations all PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information. In other implementations, the application layer can still recover parts or all of the information unit, when some PDUs are missing.

Multi-modal Data: Multi-modal Data is defined to describe the input data from different kinds of devices/sensors or the output data to different kinds of destinations (e.g. one or more UEs) required for the same task or application. Multi-modal Data consists of more than one Single-modal Data, and there is strong dependency among each Single-modal Data. Single-modal Data can be seen as one type of data.

Data Burst: Data produced by the application in a short period of time, comprising PDUs from one or more PDU Sets.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 38.300 [15] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and in TS 38.300 [15].

3DoF	Three Degrees of Freedom

6DoF	Six Degrees of freedom

AR	Augmented Reality

DASH	Dynamic Adaptive Streaming over HTTP

FEC	Forward Error Coding

FoV	Field of view

FPS	Frames Per Second

GBR	Guaranteed Bit Rate

GFBR	Guaranteed Flow Bit Rate

HEVC	High-Efficiency Video Coding

HMD	Head-Mounted Display

HUD	Heads-Up Display

PDB	Packet Delay Budget

PDU	Protocol Data Unit

PER	Packet Error Rate

PSDB	PDU-Set Delay Budget

PSER	PDU-Set Error Rate

QCI	QoS Class Identifier

QFI	QoS Flow ID

QoE	Quality of Experience

QoS	Quality of Service

VR	Virtual Reality

XR	Extended reality

# 4	Introduction to Extended Reality

## 4.1	Extended Reality Types

Extended Reality (XR) refers to all real-and-virtual combined environments and human-machine interactions generated by computer technology and wearables. XR is an umbrella term for different types of realities (see TR 26.918 [5] and TR 26.928 [7]):

-	Virtual reality (VR) is a rendered version of a delivered visual and audio scene. The rendering is designed to mimic the visual and audio sensory stimuli of the real world as naturally as possible to an observer or user as they move within the limits defined by the application. Virtual reality usually, but not necessarily, requires a user to wear a head mounted display (HMD), to completely replace the user's field of view with a simulated visual component, and to wear headphones, to provide the user with the accompanying audio. Some form of head and motion tracking of the user in VR is usually also necessary to allow the simulated visual and audio components to be updated in order to ensure that, from the user's perspective, items and sound sources remain consistent with the user's movements.

-	Augmented reality (AR) is when a user is provided with additional information or artificially generated items or content overlaid upon their current environment. Such additional information or content will usually be visual and/or audible and their observation of their current environment may be direct, with no intermediate sensing, processing and rendering, or indirect, where their perception of their environment is relayed via sensors and may be enhanced or processed.

-	Mixed reality (MR) is an advanced form of AR where some virtual elements are inserted into the physical scene with the intent to provide the illusion that these elements are part of the real scene.

Other terms used in the context of XR are Immersion as the sense of being surrounded by the virtual environment as well as Presence providing the feeling of being physically and spatially located in the virtual environment. The sense of presence provides significant minimum performance requirements for different technologies such as tracking, latency, persistency, resolution and optics.

This document uses the acronym XR throughout to refer to equipment, applications and functions used for VR, AR and MR. Examples include, but are not limited to HMDs for VR, optical see-through glasses and camera see-through HMDs for AR and MR and mobile devices with positional tracking and camera. They all offer some degree of spatial tracking and the spatial tracking results in an interaction to view some form of virtual content.

## 4.2	Human Perception and Tracking

For providing XR experiences that make the user feel immersed and present, several relevant quality of experience factors have been collected (see TR 26.926 [7]). Presence is the feeling of being physically and spatially located in an environment. Presence is divided into 2 types: Cognitive Presence and Perceptive Presence. Cognitive Presence is the presence of one's mind. It can be achieved by watching a compelling film or reading an engaging book. Cognitive Presence is important for an immersive experience of any kind. Perceptive Presence is the presence of one's senses. To accomplish perceptive presence, one's senses, sights, sound, touch and smell, have to be tricked. To create perceptive presence, the XR Device has to fool the user's senses, most notably the audio-visual system. XR Devices achieve this through positional tracking based on the movement. The goal of the system is to maintain your sense of presence and avoid breaking it. Perceptive Presence is the objective to be achieved by XR applications.

The Human field of view (FoV) is defined as the area of vision at a given moment (with a fixed head). It is the angle of visible field expressed in degrees measured from the focal point. The monocular FoV is the angle of the visible field of one eye whereas the binocular FoV is the combination of the two eyes fields (see TR 26.918 [5]). The binocular horizontal FoV is around 200-220°, while the vertical one around 135°. The central vision, which is about 60°, is also called the comfort zone where sensibility to details is the most important. Although less sensitive to definition, the peripheral vision is more receptive to movements.

In XR, actions and interactions involve movements and gestures. Thereby, the Degrees of Freedom (DoF) describe the number of independent parameters used to define movement in the 3D space (see TR 26.928 [7]):

-	3DoF: three rotational and un-limited movements around the X, Y and Z axes (respectively pitch, yaw and roll). A typical use case is a user sitting in a chair looking at 3D 360 VR content on an HMD.

Figure 4.2-1: 3DoF

-	6DoF: 3DoF with full translational movements along X, Y and Z axes. Beyond the 3DoF experience, it adds (i) moving up and down (elevating/heaving); (ii) moving left and right (strafing/swaying); and (iii) moving forward and backward (walking/surging). A typical use case is a user freely walking through 3D 360 VR content (physically or via dedicated user input means) displayed on an HMD.

Figure 4.2-2: 6DoF

An XR View describes a single view into an XR scene for a given time. Each view corresponds to a display or portion of a display used by an XR device to present the portion of the scene to the user.

An XR Viewport describes a viewport, or a rectangular region, of a graphics surface. The XR viewport corresponds to the projection of the XR View onto a target display. An XR viewport is predominantly defined by the width and height of the rectangular dimensions of the viewport.

An XR Pose describes a position and orientation in space relative to an XR Space. An essential element of XR is the spatial tracking of the viewer pose.

## 4.3	Capture, Encoding and Delivery

### 4.3.1	Video

XR content may be represented in different formats, e.g. panoramas or spheres depending on the capabilities of the capture systems. Since modern video coding standards are not designed to handle spherical content. projection is used for conversion of a spherical (or 360°) video into a two-dimensional rectangular video before the encoding stage. After projection, the obtained two-dimensional rectangular image can be partitioned into regions (e.g. front, right, left, back, top, bottom) that can be rearranged to generate "packed" frames to increase coding efficiency or viewport dependent stream arrangement.

There are mainly three approaches that can be considered for 360 video delivery (see TR 26.918 [5]):

-	Single stream approach: the single stream approach consists in providing the full 360 video and showing the interesting part only. Solutions that lie within this group have the drawback that either they may not be scalable or they may impose a big challenge in terms of required network resources (high bitrate of high resolution video) and required processing at the client side (decode a very high resolution video).

-	Multi-stream approach: the multi-stream approach consists of encoding several streams, each of them emphasizing a given viewport and making them available for the receiver, so that the receiver decides which stream is delivered at each time instance.

-	Tiled stream approach: the tiled stream approach consists in emphasizing the current user viewport through transmitting non-viewport samples with decreased resolution. The tiles can be provided as one common bitstream (using motion-constrained HEVC tiles) or as separate video streams.

In modern video codecs (see S4-220505 [13]):

-	Complex prediction structures are used that take into account application constraints, encoding complexity, latency and dynamic decisions in the encoding. This may result in irregularities, for example based on sequence properties. In particular for low-latency delivery with error resiliency, different flavours of encoding operations are in use, and the concept of I, P and B pictures is not generally applicable.

-	All PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information in some implementations; while in some others, receivers may use the data up to the first lost fragmentation unit to recover at least parts of the video data and apply error concealment afterward.

-	Furthermore, in motion-compensated predicted video decoding, some frames refer to other frames based on the video encoding configuration but also based on dynamic operational decisions. As a consequence, a PDU Set may “depend” on previously received PDU Sets. However, such dependencies do not necessarily result in discarding dependent information units.

### 4.3.2	Audio

For Audio, we can distinguish channel-based and object-based representations (see TR 26.918 [5]):

-	Channel-based representation using multiple microphones to capture sounds from different directions and post-processing techniques are well known in the industry, as they have been the standard for decades.

-	Object-based representations represent a complex auditory scene as a collection of single audio elements, each comprising an audio waveform and a set of associated parameters or metadata. The metadata embody the artistic intent by specifying the transformation of each of the audio elements to playback by the final reproduction system. Sound objects generally use monophonic audio tracks that have been recorded or synthesized through a process of sound design. These sound elements can be further manipulated, so as to be positioned in a horizontal plane around the listener, or in full three-dimensional space using positional metadata.

## 4.4	XR Engines and Rendering

XR engines provide a middleware that abstracts hardware and software functionalities for developers of XR applications (see TR 26.928 [7]). Typical components include a rendering engine for graphics, an audio engine for sound, and a physics engine for emulating the laws of physics. In the remainder of this Technical Report, the term XR engine is used to provide any type of typical XR functionalities as mentioned above.

The processing of an XR engine is not exclusively carried out in the device GPU. In power and resource constrained devices, it can be assisted or split across the network through edge computing (see TR 22.842 [2]): the UE sends the sensor data in uplink direction to the cloud side in a real time manner and when the cloud side receives the sensor data, it performs rendering computing and produces the multimedia data and then sends back to the user devices for display. This is where NR can play an essential role.

## 4.5	Characteristics and Requirements

### 4.5.1	General

In general, it is difficult to identify characteristics common for different XR applications since they heavily depend on the application choices, such as the application itself, the codec in use, the data formats and the encoding operation (see S4-220505 [13]). In particular, low-latency XR and cloud gaming video services such as Split-Rendering or Cloud Gaming typically would not use the traditional coding structure with a fixed Group of-Picture (GOP). In addition, the field of low-latency video delivery is undergoing heavy innovation and new coding methods may be established frequently. Thus, the traffic characteristics and requirements derived from the work done in SA4 (TR 26.926 [6] and TR 26.928 [7]) and listed below, can only be used as a baseline when specific examples for XR traffic characteristics are needed - bearing in mind that they are not universally applicable for all XR applications.

### 4.5.2	Video

The frame rate for XR video varies from 30 frames per second up to 90 or even 120 frames per second, with a typical minimum of 60 for VR (see TR 26.918 [5] and TR 26.926 [6]).

Editor's Note: exact frame rates to be clarified.

According to TR 26.918 [5], the latency of action of the angular or rotational vestibulo-ocular reflex is known to be of the order of 10 ms or in a range from 7-15 milliseconds and it seems reasonable that this should represent a performance goal for XR systems.  This results in a motion-to-photon latency of less than 20 milliseconds, with 10ms being given as a goal.

Regarding the bit rates, between 10 and 200Mbps can be expected for XR depending on frame rate, resolution and codec efficiency (see TR 26.926 [6] and TR 26.928 [7]).

### 4.5.3	Audio

According to TR 26.918 [5], due to the relatively slower speed of sound compared to that of light, it is natural that users are more accustomed to, and therefore tolerant of, sound being relatively delayed with respect to the video component than sound being relatively in advance of the video component. Recent studies have led to recommendations of an accuracy of between 15 ms (audio delayed) and 5 ms (audio advanced) for the synchronization, with recommended absolute limits of 60 ms (audio delayed) and 40 ms (audio advanced) for broadcast video.

### 4.5.4	Pose Information

To maintain a reliable registration of the virtual world with the real world, as well as to ensure accurate tracking of the XR Viewer pose, XR applications require highly accurate, low-latency tracking of the device at about 1kHz sampling frequency. The size of a XR Viewer Pose associated to time, typically results in packets of size in the range of 30-100 bytes, such that the generated data is around several hundred kbit/s if delivered over the network (see TR 26.928 [7]).

Pose information has to be delivered with ultra-high reliability, therefore, similar performance as URLLC is expected i.e. packet loss rate should be lower than 10E-4 for uplink sensor data – see TR 22.842 [2].

Editor's Note: LS sent to SA4 to clarify the requirements of pose information.

# 5	XR Enhancements for NR

## 5.1		XR Awareness

### 5.1.1	General

In both uplink and downlink, XR-Awareness contributes to optimizations of gNB radio resource scheduling and relies at least on the notions of PDU set and Data Burst (see TR 23.700-60 [9]): a PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. a frame or video slice), while a Data Burst is a set of data PDUs generated and sent by the application in a short period of time.

NOTE:	A Data Burst can be composed of multiple PDUs belonging to one or multiple PDU Sets.

In order to handle PDUs efficiently in both UL and DL, the following information would be useful:

Semi-static information provided by the CN:

-	The PDU-Set Delay Budget (PSDB);

-	The PDU-Set Error Rate (PSER);

-	Traffic parameters (e.g. periodicity);

-	Jitter information (e.g. range).

Dynamic information:

-	The PDUs belonging to a PDU set (this includes the means to determine at least the PDU set boundaries);

-	The PDUs belonging to a Data Burst.

When a certain number of PDUs of a PDU set are known to be required by the application layer to use the corresponding unit of information (for instance due to the absence or limitations of error concealment techniques, see TR 26.926 [6]), as soon as the number of PDUs known to be lost exceeds this number, the remaining PDUs of that PDU set are no longer needed by the application and may be subject to discard operation (see subclause 5.3.2).

NOTE 1:	This depends on the application and it cannot always be assumed that the remaining PDUs and/or dependent PDU sets are not useful and can safely be discarded.

NOTE 2:	In case of Forward Error Coding (FEC), active discarding of PDUs when assuming that a large enough number of packets have already been transmitted for FEC to recover without the remaining PDUs is not recommended as it might trigger an increase of FEC packets (see S4aV220921 [14]).

### 5.1.2	Layer 2 Structure

Depending on how the mapping of PDU sets onto QoS flows is done in the NAS and how QoS flows are mapped onto DRBs in the AS, we can distinguish the following alternatives (as depicted on Figure 5.1.2-1 below):

-	111: one-to-one mapping between types of PDU sets and QoS flows in the NAS and one-to-one mapping between QoS flows and DRBs in the AS. From a Layer 2 structure viewpoint, this alternative is already possible and requires as many DRBs as types of PDU sets. Providing different QoS for the types of PDU sets sent in different DRBs is already possible.

-	NN1: one-to-one mapping between types of PDU sets and QoS flows in the NAS and possible multiplexing of QoS flows in one DRB in the AS. From a Layer 2 structure viewpoint, this alternative is already possible but gives each QoS flows multiplexed in a DRB the same QoS. Providing different QoS for the types of PDU sets (i.e. QoS flows) multiplexed in a single DRB is currently not possible.

-	N11: possible multiplexing of types of PDU sets in one QoS flow in the NAS and one-to-one mapping between QoS flows and DRBs in the AS. From a Layer 2 structure viewpoint, this alternative is already possible but gives each QoS flow/DRB one QoS. Providing different QoS for the types of PDU sets multiplexed in a single QoS flow/DRB is currently not possible.

-	N1N: possible multiplexing of types of PDU sets in one QoS flow in the NAS and demultiplexing of types of PDU sets from one QoS flow on multiple DRBs in the AS. From a Layer 2 structure viewpoint, demultiplexing of types of PDU sets from one QoS flow onto multiple DRBs is currently not possible.

Editor's Note: the mapping of PDU sets on QoS flows is up to SA2 and it is FFS how DRB(s) is/are mapped to LCH(s) for each of the alternatives.

Figure 5.1.2-1: Mapping Alternatives

In addition, the notion of PDU set does not impact the granularity of:

-	SDAP SDU handling: SDAP still maps every incoming SDU to a single PDU for a single PDCP entity;

-	Retransmissions: HARQ still relies on MAC PDUs and ARQ on RLC PDUs.

## 5.2		Power Saving Techniques

### 5.2.1	Physical Layer Enhancements

### 5.2.2	Layer 2 Enhancements

Some XR frame rates (30fps, 60fps, 90fps, 120fps) corresponds to periodicity which is not an integer (33.33ms, 16.66ms, 11.11ms and 8.33ms respectively).

## 5.3		Capacity Improvements Techniques

### 5.3.1	Physical Layer Enhancements

Considering large and variable size of UL video frame (see TR 38.838 [8]) with CG scheduling, the following enhancements have been recommended related to CG:

-     Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration;

-     Dynamic indication of unused CG PUSCH occasion(s) based on UCI (e.g., CG-UCI or a new UCI) by the UE.

The corresponding capacity performance evaluation results are available in Annex B.1.6.

The evaluation results for other proposed and studied capacity enhancement schemes are available in Annex B.

### 5.3.2	Layer 2 Enhancements

In order to enhance the scheduling of uplink resources for XR, the following improvements are envisioned:

-	New BS table(s) to reduce the quantisation errors in BSR reporting (e.g. for high bit rates);

-	Delay knowledge of buffered data, consisting of at least remaining time, and distinguishing how much data is buffered for which delay.

Editor's Note: FFS if dynamic reporting of the uplink delay (e.g. via BSR) is needed, or whether PSDB is sufficient.

Furthermore, when, for a PDU set, the number of PDUs known to either be lost or associated to discarded SDUs, exceeds a threshold (see subclause 5.1.1), all remaining PDUs of that PDU set could be discarded at the transmitter to free up radio resources.

NOTE: this means that the granularity of the discard operation at PDCP in the transmitter should be the PDU set.

Editor's Note: the discard operation could also be captured in 5.2.2.

# 6	Conclusions

Editor's Note: this clause will capture the conclusions of the SI.

Annex A:
Evaluation Methodology

Editor's Note: this Annex will capture the deployment scenarios, traffic models and KPI.

Annex B:
Evaluation Studies

Editor's Note: this Annex will capture the evaluation studies.

## B.1	Capacity performance evaluation results

### B.1.1	Dynamic scheduling

#### B.1.1.1	Multi-PDSCH scheduling by a single DCI

This clause captures the capacity performance evaluation results for dynamic scheduling when multi-PDSCH is scheduled by a single DCI. Due to large XR video frame as per 38.838, resources in a single slot may be not enough to convey all the data of a frame, resulting in resource allocation spanning more than one slot. Thereby, in this clause, multi-PDSCH scheduling by a single DCI is evaluated to transmit XR video frame over multiple slots.

The performance of single-PDSCH scheduling, where X symbol(s) are always reserved for PDCCH transmission in each slot (scheme 1.1 in Tables B.1.1.1-1,2,3) has been compared against various schemes for multi-PDSCH scheduling. Particularly, the following schemes, where multi-PDSCH is scheduled by a single DCI, have been evaluated:

Scheme 1.2: Multi-PDSCH scheduling, where unoccupied CORESET in a slot can be re-used for PDSCH transmission.

Scheme 1.3: Multi-PDSCH scheduling, where X symbol(s) are always reserved for PDCCH transmission in each slot.

Scheme 1.4: Multi-PDSCH scheduling with FDRA enhancement - more than one FDRA indication is contained in the scheduling DCI, each of which is applied to one or more scheduled PDSCH.

Scheme 1.5: Multi-PDSCH scheduling enhancement with early HARQ-ACK feedback - multiple PUCCHs may be considered where HARQ-ACK for the earlier PDSCH(s) can be reported earlier than the later PDSCH(s) scheduled by the same DCI, to reduce latency of HARQ-ACK feedback.

Scheme 1.6: Multi-PDSCH scheduling where single PDCCH schedules up to 4 PDSCHs and full flexibility in terms of resource (RB allocation) as well as MCS for each of the 4 PDSCHs.

Scheme 1.7: Multi-PDSCH scheduling, where X symbols per slot are used for PDCCH if there is at least one UE that needs to be scheduled with first TB out of multiple TBs and 0 symbols per slot are used for PDCCH if no UE with the first TB out of multiple TBs needs to be scheduled.

The performance results of the above schemes are reported in Table B.1.1.1-1, Table B.1.1.1-2, Table B.1.1.1-3 in terms of the ratio of satisfied users.

Table B.1.1.1-1: FR1, DL, InH, VR/AR and CG at 60 FPS

| Source | Tdoc Source | Scheme | TDD format | SU/MU-MIMO | Data rate (Mbps) | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source [vivo] | R1-2208661 | 1.1* | DDDSU | SU-MIMO | 30 | 10 | 9.8 | 9 | 96.61% | Note 1,3 |
|  |  |  |  |  | 45 |  | 6.08 | 6 | 91.67% |  |
| Source [vivo] | R1-2208661 | 1.1** | DDDSU | SU-MIMO | 30 | 10 | 8.16 | 8 | 91.07% | Note 1,3 |
|  |  |  |  |  | 45 |  | 4.66 | 4 | 97.22% |  |
| Source [vivo] | R1-2208661 | 1.1*** | DDDSU | SU-MIMO | 30 | 10 | 10.25 | 10 | 93.06% | Note 1,3 |
| Source [vivo] | R1-2208661 | 1.2* | DDDSU | SU-MIMO | 30 | 10 | 10.37 | 10 | 95% | Note 1, 2,3 |
|  |  |  |  |  | 45 |  | 6.27 | 6 | 94.91% |  |
| Source [vivo] | R1-2208661 | 1.2** | DDDSU | SU-MIMO | 30 | 10 | 9.58 | 9 | 95.14% | Note 1, 2,3 |
|  |  |  |  |  | 45 |  | 5.57 | 5 | 96.67% |  |
| Source [vivo] | R1-2208661 | 1.2*** | DDDSU | SU-MIMO | 30 | 10 | 10.1 | 10 | 91.28% | Note 1, 2,3 |
| Source [vivo] | R1-2208661 | 1.3* | DDDSU | SU-MIMO | 30 | 10 | 8.81 | 8 | 93.89% | Note 1,3 |
| Source [vivo] | R1-2208661 | 1.3** | DDDSU | SU-MIMO | 30 | 10 | 7.25 | 7 | 92.02% | Note 1,3 |
| Source [vivo] | R1-2208661 | 1.3*** | DDDSU | SU-MIMO | 30 | 10 | 9.68 | 9 | 98.37% | Note 1,3 |
| Source [vivo] | R1-2208661 | 1.4* | DDDSU | SU-MIMO | 30 | 10 | 11.09 | 11 | 90.8% | Note 1 |
| Source [vivo] | R1-2208661 | 1.5* | DDDSU | SU-MIMO | 30 | 10 | 10.59 | 10 | 95.72% | Note 1 |
| Source [InterDigital] | R1-2209658 | 1.1** | DDDSU | SU-MIMO | 30 | 10 | 5.3 | 5 | 94% | Note 1 |
|  |  |  |  |  | 45 |  | 3.1 | 3 | 91.5% |  |
| Source [InterDigital] | R1-2209658 | 1.1** | DDDSU | SU-MIMO | 30 | 15 | 7.2 | 7 | 94.5% | Note 1 |
| Source [InterDigital] | R1-2209658 | 1.6**** | DDDSU | SU-MIMO | 30 | 10 | 7.8 | 7 | 97% | Note 1,2 |
|  |  |  |  |  | 45 |  | 4.1 | 4 | 91% |  |
| Source [InterDigital] | R1-2209658 | 1.6**** | DDDSU | SU-MIMO | 30 | 15 | 10 | 10 | 100% | Note 1,2 |
| Source [ZTE] | R1-2209198 | 1.1** | DDDSU | SU-MIMO | 30 | 10 | 9.1 | 9 | 91% | Note 1 |
|  |  |  |  |  | 60 |  | 3.4 | 3 | 97% |  |
| Source [ZTE] | R1-2209198 | 1.7** | DDDSU | SU-MIMO | 30 | 10 | 7.9 | 7 | 96% | Note 1,2,4 |
|  |  |  |  |  | 60 |  | 3.7 | 3 | 99% |  |
| Note 1: BS antenna parameters: 32TxRUs, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1,4,4) Note 2: No symbol for PDCCH is reserved in the slot where no scheduling DCI is transmitted Note 3: Results does not consider any other PDCCH that may occupy the CORESET(s) than scheduling DCI, e.g. broadcast PDCCH Note 4: Results consider 2 symbols are used for PDCCH, if at least one UE needs to be scheduled with the first TB out of multiple TBs * Number of PDCCH symbols per slot = 1 ** Number of PDCCH symbols per slot = 2 *** Number of PDCCH symbols per slot = 6RE **** Number of PDCCH symbols 4 PDSCHs/PUSCHs = 4 |  |  |  |  |  |  |  |  |  |  |

Table B.1.1.1-2: FR1, DL, DU, VR/AR and CG at 60 FPS

| Source | Tdoc Source | Scheme | TDD format | SU/MU-MIMO | Data rate (Mbps) | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source [InterDigital] | R1-2209658 | 1.1** | DDDSU | SU-MIMO | 30 | 10 | 5.5 | 5 | 95% | Note 1 |
|  |  |  |  |  | 45 |  | 3.7 | 3 | 100% |  |
| Source [InterDigital] | R1-2209658 | 1.1** | DDDSU | SU-MIMO | 30 | 15 | 7.5 | 7 | 95% | Note 1 |
| Source [InterDigital] | R1-2209658 | 1.6**** | DDDSU | SU-MIMO | 30 | 10 | 8.4 | 8 | 94.5% | Note 1 |
|  |  |  |  |  | 45 |  | 5.1 | 5 | 91.5% |  |
| Source [InterDigital] | R1-2209658 | 1.6**** | DDDSU | SU-MIMO | 30 | 15 | 9.1 | 9 | 91% | Note 1 |
| Note 1: BS antenna parameters: 32TxRUs, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) * Number of PDCCH symbols per slot = 1 ** Number of PDCCH symbols per slot = 2 *** Number of PDCCH symbols per slot = 6RE **** Number of PDCCH symbols per slot = 4 |  |  |  |  |  |  |  |  |  |  |

Table B.1.1.1-3: FR1, DL, UMa, VR/AR at 60 FPS

| Source | Tdoc Source | Scheme | TDD format | SU/MU-MIMO | Data rate (Mbps) | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source [ZTE] | R1-2209198 | 1** | DDDSU | SU-MIMO | 30 | 10 | 8.8 | 8 | 97% | Note 1 |
| Source [ZTE] | R1-2209198 | 7** | DDDSU | SU-MIMO | 30 | 10 | 7.6 | 7 | 96% | Note 1,2,4 |
| Note 1: BS antenna parameters: 64TxRUs, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1:4,8) Note 2: No symbol for PDCCH is reserved in the slot where no scheduling DCI is transmitted Note 3: Results did not consider any other PDCCH that may occupy the CORESET(s) than scheduling DCI, e.g. broadcast PDCCH Note 4: Results consider 2 symbols are used for PDCCH, if the slot is not the first slot of multiple TB scheduling for one UE but is the first slot of multiple TBs scheduling or the slot of single TB scheduling for another UE * Number of PDCCH symbols per slot = 1 ** Number of PDCCH symbols per slot = 2 *** Number of PDCCH symbols per slot = 6RE **** Number of PDCCH symbols per slot = 4 |  |  |  |  |  |  |  |  |  |  |

Based on the evaluation results in Table B.1.1.1-1 and Table B.1.1.1-3, the following observations regarding multi-PDSCH scheduling by a single DCI as compared to single PDSCH scheduling can be made.

- For FR1, InH, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source [vivo] that the capacity is decreased from 9.8 UEs per cell with single-PDSCH scheduling to 8.81 UEs per cell with multi-PDSCH scheduling, where 1 symbol is always reserved for PDCCH transmission in each slot (-10%). Similar trend is observed when number of PDCCH symbols per slot is equal to 6RE or 2 symbols.

- For FR1, InH, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source [vivo] that the capacity is increased from 9.8 UEs per cell with single-PDSCH scheduling to 10.37 UEs per cell with multi-PDSCH scheduling, where unoccupied CORESET in a slot can be re-used for PDSCH transmission (1 symbol for PDCCH transmission) (6%). For VR/AR single-stream traffic model, 45Mbps, 10ms PDB, the results show similar trend.

- For FR1, InH, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source [vivo] that the capacity is decreased from 10.25 UEs per cell with single-PDSCH scheduling to 10.1 UEs per cell with multi-PDSCH scheduling, where unoccupied CORESET in a slot can be re-used for PDSCH transmission (6RE for PDCCH transmission) (-1.46%).

- For FR1, InH, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source [ZTE] that the capacity is decreased from 9.1 UEs per cell with single-PDSCH scheduling to 7.9 UEs per cell with multi-PDSCH scheduling, where 2 symbols per slot are used for PDCCH if there is at least one UE that needs to be scheduled with first TB out of multiple TBs (-13%). For FR1, UMa scenario, the results show similar trend.

- For FR1, InH, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 60Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source [ZTE] that the capacity is increased from 3.4 UEs per cell with single-PDSCH scheduling to 3.7 UEs per cell with multi-PDSCH scheduling, where 2 symbols per slot are used for PDCCH if there is at least one UE that needs to be scheduled with first TB out of multiple TBs (8%).

Based on the evaluation results in Table B.1.1.1-1 and Table B.1.1.1-2, the following observations regarding enhanced multi-PDSCH scheduling by a single DCI can be made.

- For FR1, InH, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source [vivo] that the capacity is increased from 10.37 UEs per cell with multi-PDSCH scheduling to 10.59 UEs per cell with multi-PDSCH scheduling enhancement of early HARQ-ACK feedback (unoccupied CORESET in a slot can be re-used for PDSCH transmission, 1 symbol for PDCCH transmission) (2.12%).

- For FR1, InH, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source [vivo] that the capacity is increased from 10.37 UEs per cell with multi-PDSCH scheduling to 11.09 UEs per cell with multi-PDSCH scheduling FDRA enhancement (unoccupied CORESET in a slot can be re-used for PDSCH transmission, 1 symbol for PDCCH transmission) (7%).

- For FR1, InH, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source [InterDigital] that the capacity is increased from 5.3 UEs per cell with single-PDSCH scheduling to 7.2 UEs per cell with multi-PDSCH scheduling, where single PDCCH schedules up to 4 PDSCHs/PUSCHs and full flexibility in terms of resource (RB allocation) as well as MCS for each of the 4 PDSCHs/PUSCHs (36%). For VR/AR single-stream traffic model, 45Mbps, 10ms PDB and for CG single-stream traffic model, 15ms PDB, the results show similar trend.

- For FR1, DU, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source [InterDigital] that the capacity is increased from 5.5 UEs per cell with single-PDSCH scheduling to 8.4 UEs per cell with multi-PDSCH scheduling, where single PDCCH schedules up to 4 PDSCHs/PUSCHs and full flexibility in terms of resource (RB allocation) as well as MCS for each of the 4 PDSCHs/PUSCHs (53%). For VR/AR single-stream traffic model, 45Mbps, 10ms PDB and for CG single-stream traffic model, 15ms PDB, the results show similar trend.

### B.1.2	Cooperative MIMO via DL interference probing based on SRS enhancement

This clause captures the capacity performance evaluation results for cooperative MIMO in TDD with downlink interference probing via SRS and precoding via bi-directional training (BiT). In TDD system, channel UL-DL reciprocity is utilized to obtain DL channel state information for improved performance of single-cell (non-cooperative) MU-MIMO. However, inter-cell interference is still the bottleneck that limits further capacity improvement.

The performance of MU-MIMO with zero forcing precoding (scheme 2.1 in Tables B.1.2-1,2) has been compared against Cooperative MIMO with DL interference probing (scheme 2.2 in Tables B.1.2-1,2).

Scheme 2.2: Cooperative MIMO with DL interference probing via SRS and precoding via bi-directional training (BiT), is the scheme where SRS triggering and transmission are enhanced to directly reflect DL interference spatial information (utilizing UL-DL reciprocity). Each gNB performs its own (MU) scheduling and instructs the scheduled UEs to sound on the scheduled PRBs. Then by TDD reciprocity, a gNB seeing strong UL interference from a certain spatial direction on the SRS resources (for example, via estimating the UL spatial covariance matrix of interference signals) implies that in DL transmission the gNB will cause strong interference to UE(s) in that direction. The gNB can then adjust the precoding for DL interference avoidance during the PDSCH transmissions. Each gNB measures the corresponding UL SRS resources and adjusts its DL precoding accordingly to achieve multi-cell DL interference coordination/avoidance in a distributed way, and hence the so-called ‘Bi-directional Training’ (BiT).

The performance results are reported in B.1.2-1 and B.1.2-2 in terms of the ratio of satisfied users.

Table B.1.2-1: FR1, DL, DU, VR/AR and CG at 60 FPS

| Source | Tdoc Source | Scheme | TDD format | SU/MU-MIMO | Data rate (Mbps) | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source [Futurewei] | R1-2208377 | 2.1 | DDDSU | MU-MIMO | 45 | 10 | 12.2 | 12 | 90% | Note 1 |
|  |  |  |  |  |  | 15 | 17.4 | 17 | 94% |  |
| Source [Futurewei] | R1-2208377 | 2.2 | DDDSU | MU-MIMO | 45 | 10 | 16.9 | 16 | 91% | Note 1 |
|  |  |  |  |  |  | 15 | 21.7 | 21 | 92% |  |
| Source [Futurewei] | R1-2208377 | 2.1 | DDDSU | MU-MIMO | 30 | 10 | 21.7 | 21 | 91% | Note 1 |
|  |  |  |  |  |  | 15 | 27.1 | 27 | 91% |  |
| Source [Futurewei] | R1-2208377 | 2.2 | DDDSU | MU-MIMO | 30 | 10 | 25.8 | 25 | 91% | Note 1 |
|  |  |  |  |  |  | 15 | 30.1 | 30 | 91% |  |
| Source [Futurewei] | R1-2208377 | 2.1 | DDDUU | MU-MIMO | 45 | 10 | 8 | 8 | 90% | Note 1 |
|  |  |  |  |  |  | 15 | 12.7 | 12 | 96% |  |
| Source [Futurewei] | R1-2208377 | 2.2 | DDDUU | MU-MIMO | 45 | 10 | 13.1 | 13 | 90% | Note 1 |
|  |  |  |  |  |  | 15 | 16.9 | 16 | 95% |  |
| Source [Futurewei] | R1-2208377 | 2.1 | DDDUU | MU-MIMO | 30 | 10 | 13.7 | 13 | 92% | Note 1 |
|  |  |  |  |  |  | 15 | 21.5 | 21 | 93% |  |
| Source [Futurewei] | R1-2208377 | 2.2 | DDDUU | MU-MIMO | 30 | 10 | 19.9 | 19 | 93% | Note 1 |
|  |  |  |  |  |  | 15 | 25.6 | 25 | 93% |  |
| Note 1: BS antenna parameters: 64TxRUs, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1:4,8) |  |  |  |  |  |  |  |  |  |  |

Table B.1.2-2: FR1, DL, UMa, VR/AR and CG at 60 FPS

| Source | Tdoc Source | Scheme | TDD format | SU/MU-MIMO | Data rate (Mbps) | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source [Futurewei] | R1-2208377 | 2.1 | DDDSU | MU-MIMO | 45 | 10 | 7 | 7 | 90% | Note 1 |
|  |  |  |  |  |  | 15 | 9.7 | 9 | 93% |  |
| Source [Futurewei] | R1-2208377 | 2.2 | DDDSU | MU-MIMO | 45 | 10 | 9 | 9 | 90% | Note 1 |
|  |  |  |  |  |  | 15 | 11.6 | 11 | 92% |  |
| Source [Futurewei] | R1-2208377 | 2.1 | DDDSU | MU-MIMO | 30 | 10 | 11.4 | 11 | 94% | Note 1 |
|  |  |  |  |  |  | 15 | 14.2 | 14 | 91% |  |
| Source [Futurewei] | R1-2208377 | 2.2 | DDDSU | MU-MIMO | 30 | 10 | 13.2 | 13 | 92% | Note 1 |
|  |  |  |  |  |  | 15 | 16 | 16 | 90% |  |
| Source [Futurewei] | R1-2208377 | 2.1 | DDDUU | MU-MIMO | 45 | 10 | 5.2 | 5 | 90% | Note 1 |
|  |  |  |  |  |  | 15 | 6.8 | 6 | 94% |  |
| Source [Futurewei] | R1-2208377 | 2.2 | DDDUU | MU-MIMO | 45 | 10 | 6.6 | 6 | 93% | Note 1 |
|  |  |  |  |  |  | 15 | 9.4 | 9 | 91% |  |
| Source [Futurewei] | R1-2208377 | 2.1 | DDDUU | MU-MIMO | 30 | 10 | 8.8 | 8 | 90% | Note 1 |
|  |  |  |  |  |  | 15 | 11.4 | 11 | 94% |  |
| Source [Futurewei] | R1-2208377 | 2.2 | DDDUU | MU-MIMO | 30 | 10 | 10.4 | 10 | 92% | Note 1 |
|  |  |  |  |  |  | 15 | 13.3 | 13 | 93% |  |
| Note 1: BS antenna parameters: 64TxRUs, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1:4,8) |  |  |  |  |  |  |  |  |  |  |

Based on the evaluation results in Table B.1.2-1 and Table B.1.2-2, the following observations can be made.

- For FR1, DU, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with MU-MIMO, 64TxRU and DDDSU, it is observed from Source [Futurewei] that the capacity is increased from 21.7 UEs per cell with MU-MIMO and zero forcing precoding to 25.8 UEs per cell with Cooperative MIMO via SRS enhancements (19%). For 15ms PDB, the capacity gain is 11%. For UMa scenario, the results show similar trend.

- For FR1, DU, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with MU-MIMO, 64TxRU and DDDSU, it is observed from Source [Futurewei] that the capacity is increased from 12.2 UEs per cell with MU-MIMO and zero forcing precoding to 16.9 UEs per cell with Cooperative MIMO via SRS enhancements (39%). For 15ms PDB, the capacity gain is 24%. For UMa scenario, the results show similar trend.

- For FR1, UMa, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with MU-MIMO, 64TxRU and DDDSU, it is observed from Source [Futurewei] that the capacity is increased from 7 UEs per cell with MU-MIMO and zero forcing precoding to 9 UEs per cell with Cooperative MIMO via SRS enhancements (28%). For 15ms PDB, the capacity gain is 19%.

- For FR1, DU, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with MU-MIMO, 64TxRU and DDDUU, it is observed from Source [Futurewei] that the capacity is increased from 13.7 UEs per cell with MU-MIMO and zero forcing precoding to 19.9 UEs per cell with Cooperative MIMO via SRS enhancements (45%). For 15ms PDB, the capacity gain is 19%.

- For FR1, UMa, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with MU-MIMO, 64TxRU and DDDUU, it is observed from Source [Futurewei] that the capacity is increased from 8.8 UEs per cell with MU-MIMO and zero forcing precoding to 10.4 UEs per cell with Cooperative MIMO via SRS enhancements (18%). For 15ms PDB, the capacity gain is 16%.

- For FR1, DU, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with MU-MIMO, 64TxRU and DDDUU, it is observed from Source [Futurewei] that the capacity is increased from 8 UEs per cell with MU-MIMO and zero forcing precoding to 13.1 UEs per cell with Cooperative MIMO via SRS enhancements (64%). For 15ms PDB, the capacity gain is 33%.

- For FR1, UMa, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with MU-MIMO, 64TxRU and DDDUU, it is observed from Source [Futurewei] that the capacity is increased from 5.2 UEs per cell with MU-MIMO and zero forcing precoding to 6.6 UEs per cell with Cooperative MIMO via SRS enhancements (27%). For 15ms PDB, the capacity gain is 38%.

### B.1.3	Enhanced CQI for CBG-based transmissions

This clause captures the capacity performance evaluation results of enhanced CQI (eCQI) for code block group (CBG)-based transmissions. The use of CBG-based transmissions allows avoiding a re-transmission of the full transport block in case the receiver fails to correctly decode the full TB. A large size of one video frame as per TR 38.838, results in a large TB(s) size to convey a video frame.

The performance of the current legacy link adaptation with TB-based transmission legacy CQI reporting (scheme 3.1 in Table B.1.3-1) has been compared against CBG-based transmission with eCQI (scheme 3.2 in Table B.1.3-1). The legacy CQI corresponds to the highest supported MCS while not exceeding a 10% BLER target for the TBs.

Scheme 3.2: eCQI  is the scheme where UE estimates the highest supported MCS (expressed via a CQI index), assuming that downlink transmissions occupy a set of downlink physical resource blocks termed the CSI reference resource with M code block groups, while the error probability of at most N failed code block groups does not exceed P. Parameters M, N, and P may be configured by the network, or fixed to values that are attractive for XR services. The configuration of the UE to use eCQI may be conducted with RRC signaling (note that configuration of current CQI schemes for a UE to use is also via RRC). The reporting of the eCQI can be in the form of an eCQI index (from current CQI tables [TS38.214 - Tables in 5.2.2.1] or enhanced CQI tables) that will guide the gNB to choose the supported modulation scheme, effective code rate, and overall efficiency that for its PDSCH transmissions. For the CBG-based transmission with eCQI, results are shown for parameter settings M=8 assuming either N=2 or N=4 and P= 50%.

The performance results are reported in Table B.1.3-1 in terms of the ratio of satisfied users.

Table B.1.3-1: FR1, DL, InH, VR/AR and CG at 60 FPS

| Source | Tdoc Source | Scheme | TDD format | SU/MU-MIMO | Data rate (Mbps) | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source [Nokia] | R1-2209536 | 3.2* | DDDSU | SU-MIMO | 30 | 10 | 6.35 | 6 | 99% | Note 1,2 |
|  |  |  |  |  |  | 15 | 7.31 | 7 | 99% |  |
| Source [Nokia] | R1-2209536 | 3.2* | DDDSU | SU-MIMO | 45 | 10 | 4.15 | 4 | 95% | Note 1,2 |
|  |  |  |  |  |  | 15 | 5.12 | 5 | 95% |  |
| Source [Nokia] | R1-2209536 | 3.2** | DDDSU | SU-MIMO | 30 | 10 | 6.22 | 6 | 97% | Note 1,2 |
|  |  |  |  |  |  | 15 | 7.31 | 7 | 99% |  |
| Source [Nokia] | R1-2209536 | 3.2** | DDDSU | SU-MIMO | 45 | 10 | 4.10 | 4 | 93% | Note 1,2 |
|  |  |  |  |  |  | 15 | 4.83 | 4 | 100% |  |
| Source [Nokia] | R1-2209536 | 3.1 | DDDSU | SU-MIMO | 30 | 10 | 5.45 | 5 | 95% | Note 1 |
|  |  |  |  |  |  | 15 | 6.31 | 6 | 98% |  |
| Source [Nokia] | R1-2209536 | 3.1 | DDDSU | SU-MIMO | 45 | 10 | 3.35 | 3 | 99% | Note 1 |
|  |  |  |  |  |  | 15 | 4.18 | 4 | 94% |  |
| Note 1: BS antenna parameters: 32TxRUs, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1,4,4) Note 2: non-iid CBG errors in a TB are assumed * At most N failed code block groups, N= 4 ** At most N failed code block groups, N= 2 |  |  |  |  |  |  |  |  |  |  |

Based on the evaluation results in Table B.1.3-1, the following observations can be made.

- For FR1, InH, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source [Nokia] that the capacity is increased from 5.45 UEs per cell with legacy CQI to 6.35 UEs per cell with eCQI, where the error probability of at most N=4 failed code block groups out of 8 CBGs does not exceed P = 50% (17%). For N=2, the results show similar trend.

- For FR1, InH, DL, with 100MHz bandwidth for CG single-stream traffic model, 30Mbps, 15ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source [Nokia] that the capacity is increased from 6.31 UEs per cell with legacy CQI to 7.31 users per cell with eCQI, where the error probability of at most N=4 failed code block groups out of 8 CBGs does not exceed P = 50% (16%). For N=2, the results show similar trend.

- For FR1, InH, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source [Nokia] that the capacity is increased from 3.35 UEs per cell with legacy CQI to 4.15 UEs per cell with eCQI, where the error probability of at most N=4 failed code block groups out of 8 CBGs does not exceed P = 50% (24%). For N=2, the results show similar trend.

- For FR1, InH, DL, with 100MHz bandwidth for CG single-stream traffic model, 45Mbps, 15ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source [Nokia] that the capacity is increased from 4.18 UEs per cell with legacy CQ to 5.12 UEs per cell with eCQI, where the error probability of at most N=4 failed code block groups out of 8 CBGs does not exceed P = 50% (22%). For N=2, the results show similar trend.

### B.1.4	Enhanced CQI based on DMRS

This clause captures the capacity performance evaluation results of DMRS-based enhanced CQI reporting where the initial transmission is based on legacy CQI reporting, while the retransmission is DMRS based CQI reporting.

The performance of the current TB-based transmission with legacy CQI reporting (scheme 4.1 in Tables B.1.4) has been compared against CBG-based transmission (scheme 4.2 in Tables B.1.4) as well as TB-based re-transmission and DMRS-based CQI (scheme 4.3 in Tables B.1.4).

Scheme 4.1: TB-based re-transmission (baseline scheme). In this scheme, TB-based retransmission is applied with legacy CSI reporting. Different outer-loop link adaptation configurations are used.

Scheme 4.2: CBG-based re-transmission. In this scheme, CBG-based retransmission is applied with legacy CSI reporting. Different outer-loop link adaptation configurations are used. The number of CBGs per TB is equal to 8.

Scheme 4.3: TB-based re-transmission with DMRS-based CQI. In this scheme, TB-based transmission/retransmission is applied where for the initial TB transmission legacy CQI reporting is used, while for the retransmission DMRS-based CQI reporting is used.

The performance results are reported in Table B.1.4-1 and Table B.1.4-2 in terms of the ratio of satisfied users.

Table B.1.4-1: FR1, DL, InH, VR/AR and CG at 60 FPS

| Source | Tdoc Source | Scheme | TDD format | SU/MU-MIMO | Data rate (Mbps) | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source [Ericsson] | R1-2208402 | 4.1* | DDDSU | SU-MIMO | 30 | 10 | 8.03 | 8 | 90% | Note 1,2 |
|  |  |  |  |  |  | 15 | 10.22 | 10 | 92% |  |
| Source [Ericsson] | R1-2208402 | 4.2* | DDDSU | SU-MIMO | 30 | 10 | 8.08 | 8 | 91% | Note 1,2,3 |
|  |  |  |  |  |  | 15 | 10.34 | 10 | 93% |  |
| Source [Ericsson] | R1-2208402 | 4.3* | DDDSU | SU-MIMO | 30 | 10 | 8.52 | 8 | 93% | Note 1,2 |
|  |  |  |  |  |  | 15 | 11.37 | 11 | 94% |  |
| Source [Ericsson] | R1-2208402 | 4.1** | DDDSU | SU-MIMO | 30 | 10 | 7.21 | 7 | 92% | Note 1,2 |
|  |  |  |  |  |  | 15 | 8.29 | 8 | 93% |  |
| Source [Ericsson] | R1-2208402 | 4.2** | DDDSU | SU-MIMO | 30 | 10 | 7.25 | 7 | 93% | Note 1,2,3 |
|  |  |  |  |  |  | 15 | 8.33 | 8 | 93% |  |
| Source [Ericsson] | R1-2208402 | 4.3** | DDDSU | SU-MIMO | 30 | 10 | 7.49 | 7 | 94% | Note 1,2 |
|  |  |  |  |  |  | 15 | 8.66 | 8 | 94% |  |
| Source [Ericsson] | R1-2208402 | 4.1*** | DDDSU | SU-MIMO | 30 | 10 | 7.89 | 7 | 96% | Note 1,2 |
|  |  |  |  |  |  | 15 | 9.48 | 9 | 94% |  |
| Source [Ericsson] | R1-2208402 | 4.2*** | DDDSU | SU-MIMO | 30 | 10 | 8.01 | 8 | 90% | Note 1,2,3 |
|  |  |  |  |  |  | 15 | 9.59 | 9 | 95% |  |
| Source [Ericsson] | R1-2208402 | 4.3*** | DDDSU | SU-MIMO | 30 | 10 | 8.53 | 8 | 94% | Note 1,2 |
|  |  |  |  |  |  | 15 | 10.31 | 10 | 93% |  |
| Note 1: BS antenna parameters: 32TxRUs, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1,4,4) Note 2: Interleaved VRB-to-PRB mapping is applied Note 3: iid CBG errors in a TB are assumed * without outer loop, BLER = 10% ** with outer loop, BLER = 10% *** with outer loop, BLER = 22% |  |  |  |  |  |  |  |  |  |  |

Table B.1.4-2: FR1, DL, UMa, VR/AR and CG at 60 FPS

| Source | Tdoc Source | Scheme | TDD format | SU/MU-MIMO | Data rate (Mbps) | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source [Ericsson] | R1-2208402 | 4.1* | DDDSU | SU-MIMO | 30 | 10 | 5.59 | 5 | 94% | Note 1,2 |
|  |  |  |  |  |  | 15 | 9.0 | 9 | 90% |  |
| Source [Ericsson] | R1-2208402 | 4.2* | DDDSU | SU-MIMO | 30 | 10 | 5.80 | 5 | 95% | Note 1,2,3 |
|  |  |  |  |  |  | 15 | 9.15 | 9 | 91% |  |
| Source [Ericsson] | R1-2208402 | 4.3* | DDDSU | SU-MIMO | 30 | 10 | 6.79 | 6 | 94% | Note 1,2 |
|  |  |  |  |  |  | 15 | 10.28 | 10 | 93% |  |
| Source [Ericsson] | R1-2208402 | 4.1** | DDDSU | SU-MIMO | 30 | 10 | 5.88 | 5 | 96% | Note 1,2 |
|  |  |  |  |  |  | 15 | 7.26 | 7 | 92% |  |
| Source [Ericsson] | R1-2208402 | 4.2** | DDDSU | SU-MIMO | 30 | 10 | 5.9 | 5 | 97% | Note 1,2,3 |
|  |  |  |  |  |  | 15 | 7.29 | 7 | 93% |  |
| Source [Ericsson] | R1-2208402 | 4.3** | DDDSU | SU-MIMO | 30 | 10 | 6.13 | 6 | 91% | Note 1,2 |
|  |  |  |  |  |  | 15 | 7.40 | 7 | 93% |  |
| Source [Ericsson] | R1-2208402 | 4.1*** | DDDSU | SU-MIMO | 30 | 10 | 6.40 | 6 | 93% | Note 1,2 |
|  |  |  |  |  |  | 15 | 8.24 | 8 | 92% |  |
| Source [Ericsson] | R1-2208402 | 4.2*** | DDDSU | SU-MIMO | 30 | 10 | 6.46 | 6 | 93% | Note 1,2,3 |
|  |  |  |  |  |  | 15 | 8.32 | 8 | 93% |  |
| Source [Ericsson] | R1-2208402 | 4.3*** | DDDSU | SU-MIMO | 30 | 10 | 7 | 7 | 90.0% | Note 1,2 |
|  |  |  |  |  |  | 15 | 8.84 | 8 | 96% |  |
| Note 1: BS antenna parameters: 64TxRUs, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1:4,8) Note 2: Interleaved VRB-to-PRB mapping is applied Note 3: iid CBG errors in a TB are assumed * without outer loop, BLER = 10% ** with outer loop, BLER = 10% *** with outer loop, BLER = 22% |  |  |  |  |  |  |  |  |  |  |

Based on the evaluation results in Table B.1.4-1 and Table B.1.4-2, the following observations can be made.

- For FR1, InH, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source [Ericsson] that the capacity is increased by 6%, 4% and 8% when DMRS-based CQI is used as compared to legacy CQI for outer-loop link adaptation configurations of ‘disabled’, BLER=10% and BLER=22%, respectively.

- For FR1, InH, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 15ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source [Ericsson] that the capacity is increased by 11%, 4% and 9% when DMRS-based CQI is used as compared to legacy CQI for outer-loop link adaptation configurations of ‘disabled’, BLER=10% and BLER=22%, respectively.

- For FR1, UMa, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [Ericsson] that the capacity is increased by 21%, 4% and 9% when DMRS-based CQI is used as compared to legacy CQI for outer-loop link adaptation configurations of ‘disabled’, BLER=10% and BLER=22%, respectively.

- For FR1, UMa, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 15ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [Ericsson] that the capacity is increased by 14%, 2% and 7% when DMRS-based CQI is used as compared to legacy CQI for outer-loop link adaptation configurations of ‘disabled’, BLER=10% and BLER=22%, respectively.

### B.1.5	Soft HARQ-ACK enhancements

This clause captures the capacity performance evaluation results of soft HARQ-ACK enhancements. HARQ allows to recover transport blocks that are not successfully decoded by means of retransmissions. It can be leveraged to use MCS close to what the radio channel can support while maintaining high reliability. The gNB chooses the MCS for a transmission based on its knowledge of the channel conditions, but it cannot always precisely predict how they evolve.

The performance of Baseline HARQ-ACK (scheme 5.1 in Tables B.1.5-1,2,3) has been compared against different schemes with soft HARQ-ACK. Particularly, the following schemes, with soft HARQ-ACK, have been evaluated:

Scheme 5.2: Soft HARQ-ACK indicating delta MCS: UE provides enhanced HARQ-ACK feedback beyond the single bit ACK/NACK status in the form of a Delta MCS based on PDSCH decoding. The soft HARQ-ACK feedback uses SINR measurements done on each TB and link curves obtained from link level simulations. Then the information the UE feeds back to the gNB is an estimation of how far the experienced SINR is from the SINR that would allow a reliable decoding of the TB.

Scheme 5.3: Soft HARQ-ACK indicating number of redundant transmissions: 2-bit soft-HARQ feedback is reported by the UE and used by the gNB scheduler. The 2-bit soft-HARQ feedback represents 4 states: ACK, and NACK with n=1, 2, or 3, where “n” indicates how many redundant transmissions are requested by the UE.

The performance results are reported in Table B.1.5-1, Table B.1.5-2, Table B.1.5-3 in terms of the ratio of satisfied users.

Table B.1.5-1: FR1, DL, InH, VR/AR at 60 FPS

| Source | Tdoc Source | Scheme | TDD format | SU/MU-MIMO | Data rate (Mbps) | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source [Qualcomm] | R1-2210003 | 5.2* | DDDSU | MU-MIMO | 60 | 10 | 2.93 | 2 | 97.7% | Note 1 |
|  |  |  |  |  | 45 |  | 5.4 | 5 | 92.3% |  |
|  |  |  |  |  | 30 |  | 8.96 | 8 | 95% |  |
| Source [Qualcomm] | R1-2210003 | 5.1* | DDDSU | MU-MIMO | 60 | 10 | 0 | 0 | N.A. | Note 1 |
|  |  |  |  |  | 45 |  | 4.5 | 4 | 93.1% |  |
|  |  |  |  |  | 30 |  | 8.5 | 8 | 92.9% |  |
| Source [Qualcomm] | R1-2210003 | 5.2** | DDDSU | MU-MIMO | 60 | 10 | 2.1 | 2 | 91.25% | Note 1 |
|  |  |  |  |  | 45 |  | 4.1 | 4 | 91%% |  |
|  |  |  |  |  | 30 |  | 7.8 | 7 | 93.5% |  |
| Source [Qualcomm] | R1-2210003 | 5.1** | DDDSU | MU-MIMO | 60 | 10 | 0 | 0 | N.A. | Note 1 |
|  |  |  |  |  | 45 |  | 0 | 0 | N.A. |  |
|  |  |  |  |  | 30 |  | 0 | 0 | N.A. |  |
| Source [Qualcomm] | R1-2210003 | 5.2*** | DDDSU | MU-MIMO | 60 | 10 | 1.17 | 1 | 91.25% | Note 1 |
| Source [Qualcomm] | R1-2210003 | 5.1*** | DDDSU | MU-MIMO | 60 | 10 | 0 | 0 | N.A. | Note 1 |
| Source [ZTE] | R1-2203607 | 5.2*** | DDDSU | MU-MIMO | 60 | 10 | 3.3 | 3 | 93% | Note 1 |
| Source [ZTE] | R1-2203607 | 5.1*** | DDDSU | MU-MIMO | 60 | 10 | 0 | 0 | N.A. | Note 1 |
| Note 1: BS antenna parameters: 32TxRUs, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1:4,4) * gNB processing delay k3 (from HARQ feedback to retransmission) = 4 slots ** gNB processing delay k3 (from HARQ feedback to retransmission) = 6 slots *** gNB processing delay k3 (from HARQ feedback to retransmission) = 8 slots |  |  |  |  |  |  |  |  |  |  |

Table B.1.5-2: FR1, DL, DU, VR/AR at 60 FPS

| Source | Tdoc Source | Scheme | TDD format | SU/MU-MIMO | Data rate (Mbps) | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source [Qualcomm] | R1-2210003 | 5.2* | DDDSU | MU-MIMO | 60 | 10 | 4.6 | 4 | 94.5% | Note 1 |
|  |  |  |  |  | 45 |  | 7.5 | 7 | 93% |  |
|  |  |  |  |  | 30 |  | 11.9 | 11 | 94.1% |  |
| Source [Qualcomm] | R1-2210003 | 5.1* | DDDSU | MU-MIMO | 60 | 10 | 0 | 0 | N.A. | Note 1 |
|  |  |  |  |  | 45 |  | 6.6 | 6 | 92.2% |  |
|  |  |  |  |  | 30 |  | 11.3 | 11 | 91.3% |  |
| Source [Qualcomm] | R1-2210003 | 5.2** | DDDSU | MU-MIMO | 60 | 10 | 2.8 | 2 | 92.9% | Note 1 |
|  |  |  |  |  | 45 |  | 5.5 | 5 | 92.4% |  |
|  |  |  |  |  | 30 |  | 10.4 | 10 | 92% |  |
| Source [Qualcomm] | R1-2210003 | 5.1** | DDDSU | MU-MIMO | 60 | 10 | 0 | 0 | N.A. | Note 1 |
|  |  |  |  |  | 45 |  | 0 | 0 | N.A. |  |
|  |  |  |  |  | 30 |  | 0 | 0 | N.A. |  |
| Source [Qualcomm] | R1-2210003 | 5.2*** | DDDSU | MU-MIMO | 60 | 10 | 2 | 2 | 90.1% | Note 1 |
| Source [Qualcomm] | R1-2210003 | 5.1*** | DDDSU | MU-MIMO | 60 | 10 | 0 | 0 | N.A. | Note 1 |
| Source [FW] | R1-2208377 | 5.1**** | DDDUU | MU-MIMO | 45 | 10 | 8.4 | 8 | 91% | Note 1 |
| Source [FW] | R1-2208377 | 5.3**** | DDDUU | MU-MIMO | 45 | 10 | 7.9 | 7 | 90% | Note 1 |
| Note 1: BS antenna parameters: 64TxRUs, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1:4,8) * gNB processing delay k3 (from HARQ feedback to retransmission) = 4 slots ** gNB processing delay k3 (from HARQ feedback to retransmission) = 6 slots *** gNB processing delay k3 (from HARQ feedback to retransmission) = 8 slots **** UE processing delay and gNB processing delay = 10 slots |  |  |  |  |  |  |  |  |  |  |

Table B.1.5-3: FR1, DL, UMa, VR/AR at 60 FPS

| Source | Tdoc Source | Scheme | TDD format | SU/MU-MIMO | Data rate (Mbps) | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source [FW] | R1-2208377 | 5.1**** | DDDUU | MU-MIMO | 45 | 10 | 6.2 | 6 | 90.3% | Note 1 |
| Source [FW] | R1-2208377 | 5.3**** | DDDUU | MU-MIMO | 45 | 10 | 6.1 | 6 | 91.3% | Note 1 |
| Note 1: BS antenna parameters: 64TxRUs, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1:4,8) **** UE processing delay and gNB processing delay = 10 slots |  |  |  |  |  |  |  |  |  |  |

Based on the evaluation results in Table B.1.5-1, Table B.1.5-2, Table B.1.5-3 the following observations can be made.

- For FR1, DU, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [Qualcomm] that the capacity is increased from 11.3 UEs per cell with baseline HARQ-ACK to 11.9 UEs per cell with soft HARQ-ACK indicating delta MCS based on PDSCH decoding and k3=4 slots (5%). For InH scenario, the results show similar trend.

- For FR1, DU, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [Qualcomm] that the capacity is increased from 6.6 UEs per cell with baseline HARQ-ACK to 7.5 UEs per cell with soft HARQ-ACK indicating delta MCS based on PDSCH decoding and k3=4 slots (14%). For InH scenario, the results show similar trend.

- For FR1, DU, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 60Mbps, 10ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [Qualcomm] that the capacity is increased from 0 UEs per cell with baseline HARQ-ACK to 4.6 UEs per cell with soft HARQ-ACK indicating delta MCS based on PDSCH decoding and k3=4 slots. For InH scenario, the results show similar trend.

- For FR1, DU, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [Qualcomm] that the capacity is increased from 0 UEs per cell with baseline HARQ-ACK to 10.4 UEs per cell with soft HARQ-ACK indicating delta MCS based on PDSCH decoding and k3=6 slots. For InH scenario, the results show similar trend.

- For FR1, DU, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [Qualcomm] that the capacity is increased from 0 UEs per cell with baseline HARQ-ACK to 5.5 UEs per cell with soft HARQ-ACK indicating delta MCS based on PDSCH decoding and k3=6 slots. For InH scenario, the results show similar trend.

- For FR1, DU, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 60Mbps, 10ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [Qualcomm] that the capacity is increased from 0 UEs per cell with baseline HARQ-ACK to 2.8 UEs per cell with soft HARQ-ACK indicating delta MCS based on PDSCH decoding and k3=6 slots. For InH scenario, the results show similar trend.

- For FR1, DU, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 60Mbps, 10ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [Qualcomm] that the capacity is increased from 0 UEs per cell with baseline HARQ-ACK to 2 UEs per cell with soft HARQ-ACK indicating delta MCS based on PDSCH decoding and k3=8 slots. For InH scenario, the results show similar trend.

- For FR1, InH, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 60Mbps, 10ms PDB, 60 FPS, with MU-MIMO, it is observed from Source [ZTE] that the capacity is increased from 0 UEs per cell with baseline HARQ-ACK to 3.3 UEs per cell with soft HARQ-ACK indicating delta MCS based on PDSCH decoding and k3=8 slots.

- For FR1, DU, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [Futurewei] that the capacity is decreased from 8.4 UEs per cell with baseline HARQ-ACK to 7.9 UEs per cell with soft HARQ-ACK indicating how many redundant transmissions are requested by the UE (-6%).

- For FR1, UMa, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [Futurewei] that the capacity is decreased from 6.2 UEs per cell with baseline HARQ-ACK to 6.1 UEs per cell with soft HARQ-ACK indicating how many redundant transmissions are requested by the UE (-2%).

### B.1.6	Configured grant scheduling

This clause captures the capacity performance evaluation results for configured grant scheduling. A UE can transmit UL data using CG resources after configuration (of a CG Config Type 1) or activation (of a CG Config Type 2), without the need of receiving UL grant from the gNB.

The performance of dynamic grant (DG) scheduling has been compared against configured grant (CG) scheduling. Particularly, the following schemes have been evaluated:

Scheme 1.1: DG scheduling with SR followed by UL grant with BSR and data. It is assumed that an SR is triggered upon arrival of a new XR packet in the UE buffer. An UL resource is then granted to the UE to transmit BSR and a number of UL data. A periodicity of SR and a size of initial UL grant may vary and are indicated separately in Table B.1.6-1, Table B.1.6-2, Table B.1.6-3.

Scheme 1.2: DG scheduling with SR followed by UL grant with BSR only. A periodicity of SR may vary and is indicated separately in Table B.1.6-1, Table B.1.6-2, Table B.1.6-3.

Scheme 2: Pre-scheduling dynamic grant: The scheduling is based on dynamic grants where it is assumed that the network is provided with XR traffic periodicity. An initial grant to the UE when its traffic is expected is transmitted (implementation-based learning) without using SR. A size of initial UL grant may vary and is indicated separately in Table B.1.6-1, Table B.1.6-2, Table B.1.6-3.

Scheme 3: Dynamic scheduling with genie BSR (DG with genie BSR): The scheduling is based on dynamic grants where it is assumed BSR is available with zero delay at the scheduler when a new XR packet arrives in the UE buffer, to be used for indicating UL grants to the UE. Hence, in this case, no SR or BSR delay is assumed.

Scheme 4: Single CG configuration, where single PUSCH occasion per CG period is pre-configured with certain periodicity without relying on SR. In this scheme no scheduling delay is assumed. CG periodicity may vary and is indicated separately in Table B.1.6-1, Table B.1.6-2, Table B.1.6-3.

Scheme 5: Multiple CG configurations, where multiple PUSCH occasions per CG period are pre-configured with certain periodicity without relying on SR. In this scheme no scheduling delay is assumed. CG periodicity may vary and is indicated separately in Table B.1.6-1, Table B.1.6-2, Table B.1.6-3.

Scheme 5.1: Multiple CG configurations, where multiple PUSCH per CG occasion are pre-configured with certain periodicity without relying on SR. In this scheme scheduling delay for scheduling more packets with DG is assumed as 2.5 ms. CG periodicity may vary and is indicated separately in Table B.1.6-1, Table B.1.6-2, Table B.1.6-3.

Scheme 6.1: Hybrid CG+DG scheduling – the scheduling is based on a combined use of configured and dynamic grants. SR resources are not used. Instead, CG resources are configured with a certain size in every UL slot in order to transmit BSR and data when new data arrives. Whenever a XR packet arrives in a buffer, the UE uses the nearest possible CG occasion for BSR transmission and possibly some amount of data. The network can thus use the BSR to provide dynamic grants for the following data transmission. CG periodicity and size of CG grant may vary and are indicated separately in Table B.1.6-1, Table B.1.6-2, Table B.1.6-3.

Scheme 6.2: Hybrid CG+DG scheduling – the scheduling is based on a combined use of configured and dynamic grants. SR resources are not used. Instead, CG resources are configured with a certain size in every UL slot in order to transmit BSR only when a new XR packet arrives. Whenever a new XR packet arrives in the UE buffer, the UE uses the nearest possible CG PUSCH occasion for BSR transmission. The network can thus use the BSR to provide dynamic grants for the corresponding data scheduling. CG periodicity and size of CG grant may vary and are indicated separately in Table B.1.6-1, Table B.1.6-2, Table B.1.6-3.

Scheme 6.3: Hybrid CG+DG scheduling – the scheduling is based on a combined use of configured and dynamic grants. SR resources are not used. Instead, CG resources are configured with a certain size in every UL slot in order to transmit BSR and data when a new XR packet arrives. Whenever a new XR packet arrives in the UE’s buffer, the UE uses the nearest possible CG PUSCH occasion for BSR transmission and possibly some amount of data. The network can thus use the BSR to provide dynamic grants for the following data transmission. The PDCCH monitoring window for the dynamic grant is pre-configured and associated with the CG PUSCH occasion in each CG period. The enhanced BSR that the BSR report will be triggered at the first used CG PUSCH occasion in each CG period to indicate to the gNB the remaining packet size is assumed. If the BSR is reported by the UE with the status “not empty”, the UE would monitor PDCCH in the subsequent slots for dynamic grant after XR packet transmission with the CG resource, in which the dynamic grant is used to schedule the remaining data of XR packet. If the BSR is reported by the UE with the status “empty”, the UE would not monitor PDCCH in the subsequent slots of the PDCCH monitoring window for XR packet transmission. Furthermore, if the XR packets are completely transmitted during the monitoring window, the UE can be indicated to go to sleep. If the XR packets arrive during the monitoring window, the UE can be indicated to skip the PDCCH monitoring until the packets arrive. CG periodicity and size of CG grant may vary and are indicated separately in Table B.1.6-1, Table B.1.6-2, Table B.1.6-3.

Scheme 7: Enhanced CG with flexible resource allocation, where at least one PUSCH occasion per CG period is pre-configured with certain periodicity without relying on SR. In this scheme, it is assumed the UE can indicate to gNB unused CG PUSCH resources via CG-UCI. CG periodicity may vary and is indicated separately in Table B.1.6-1, Table B.1.6-2, Table B.1.6-3.

Scheme 8: Enhanced CG with dynamic adaptation of CG parameters and indication of unused/used CG PUSCH occasion(s). In this scheme, the UE adjusts CG parameters dynamically: MCS, number of PRBs, number of layers within the CG resource. The UE indicates these scheduling parameters to the gNB via UCI transmitted separately in the same CG resource, and whether the subsequent CG PUSCH occasions will be used/unused until next XR packet arrival. It is assumed that at least one PUSCH occasion per CG period is pre-configured with certain periodicity without relying on SR. CG periodicity may vary and is indicated separately in Table B.1.6-1, Table B.1.6-2, Table B.1.6-3.

Scheme 9: Enhanced CG with indication of unused/used CG PUSCH occasion(s), where the UE only indicates if the subsequent CG PUSCH occasions will be used/unused until next XR packet arrival. It is assumed that at least one PUSCH occasion per CG period is pre-configured with certain periodicity without relying on SR. CG periodicity may vary and is indicated separately in Table B.1.6-1, Table B.1.6-2, Table B.1.6-3.

Scheme 10: Enhanced CG with MAC CE based dynamic resource adjustment indication. In this scheme, BSR delay or resource adjustment indication delay based on MAC CE is 2.5ms. It is assumed that at least one PUSCH occasion per CG period is pre-configured with certain periodicity without relying on SR. CG carries BSR reports and video data transmission. CG periodicity may vary and is indicated separately in Table B.1.6-1, Table B.1.6-2, Table B.1.6-3.

Scheme 11: Enhanced CG with UCI based dynamic resource adjustment indication. In this scheme, resource adjustment indication delay based on UCI is 0.5ms. It is assumed that at least one PUSCH occasion per CG period is pre-configured with certain periodicity without relying on SR. CG carries BSR reports and video data transmission. CG periodicity may vary and is indicated separately in Table B.1.6-1, Table B.1.6-2, Table B.1.6-3.

Scheme 12: Enhanced CG with UCI based dynamic resource adjustment indication. In this scheme, resource adjustment indication delay based on UCI is 0.5ms. It is assumed that at least one PUSCH occasion per CG period is pre-configured with certain periodicity without relying on SR. CG carries dynamic resource adjustment indication only. CG periodicity may vary and is indicated separately in Table B.1.6-1, Table B.1.6-2, Table B.1.6-3.

Table B.1.6-1: FR1, UL, InH, AR at 60 FPS

| Source | Tdoc Source | Scheme | TDD format | SU/MU-MIMO | Data rate (Mbps) | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source [CATT] | R1-2211175 | 1.1 | DDDUU | SU-MIMO | 10 | 30 | 5.4 | 5 | 90% | Note 1,2,12 |
| Source [CATT] | R1-2211175 | 4* | DDDUU | SU-MIMO | 10 | 30 | <1 | 0 | N.A. | Note 1 |
| Source [CATT] | R1-2211175 | 6.3* | DDDUU | SU-MIMO | 10 | 30 | 7.3 | 7 | 91% | Note 1,7.2,14 |
| Source [CATT] | R1-2211175 | 2* | DDDUU | SU-MIMO | 10 | 30 | 7.2 | 7 | 90% | Note 1,10,13 |
| Source [InterDigital] | R1-2211843 | 1.1 | DDDSU | SU-MIMO | 10 | 30 | 7.1 | 7 | 91% | Note 1,2,3-2,9 |
|  |  |  |  |  |  | 10 | 2 | 2 | 100% |  |
| Source [InterDigital] | R1-2211843 | 4** | DDDSU | SU-MIMO | 10 | 30 | 6 | 6 | 100% | Note 1 |
|  |  |  |  |  |  | 10 | 4.7 | 4 | 98% |  |
| Source [InterDigital] | R1-2211843 | 7** | DDDSU | SU-MIMO | 10 | 30 | 7.5 | 7 | 100% | Note 1 |
|  |  |  |  |  |  | 10 | 5.2 | 5 | 99% |  |
| Source [vivo] | R1-2212595 | 5*** | DDDSU | SU-MIMO | 10 | 30 | 11.3 | 11 | 92.42% | Note 1,5 |
|  |  |  |  |  |  | 10 | 6.22 | 6 | 93.26% |  |
| Source [vivo] | R1-2212595 | 10*** | DDDSU | SU-MIMO | 10 | 30 | 11.85 | 11 | 93.46% | Note 1,5 |
|  |  |  |  |  |  | 10 | 6.42 | 6 | 94.44% |  |
| Source [vivo] | R1-2212595 | 11*** | DDDSU | SU-MIMO | 10 | 30 | 12.87 | 12 | 95.18% | Note 1,5 |
|  |  |  |  |  |  | 10 | 7.81 | 7 | 96.23% |  |
| Source [vivo] | R1-2212595 | 6.2*** | DDDSU | SU-MIMO | 10 | 30 | 13.96 | 13 | 94.08% | Note 1,6,11 |
|  |  |  |  |  |  | 10 | 6.12 | 6 | 92.13% |  |
| Source [vivo] | R1-2212595 | 12*** | DDDSU | SU-MIMO | 10 | 10 | 8.79 | 8 | 93.52% | Note 1,6 |
| Source [vivo] | R1-2212595 | 5*** | DDDSU | SU-MIMO | 10 | 10 | 7.53 | 7 | 94.41% | Note 1,7 |
| Source [vivo] | R1-2212595 | 11*** | DDDSU | SU-MIMO | 10 | 10 | 8.83 | 8 | 93.92% | Note 1,7 |
| Source [vivo] | R1-2212595 | 1.1*** | DDDSU | SU-MIMO | 10 | 30 | 13.94 | 13 | 93.96% | Note 1,3,3.1,8 |
|  |  |  |  |  |  | 10 | 2.33 | 2 | 95.69% |  |
| Source [vivo] | R1-2212595 | 1.1*** | DDDSU | SU-MIMO | 10 | 30 | 13.82 | 13 | 93.5% | Note 1,4,4.1,8 |
|  |  |  |  |  |  | 10 | 0 | 0 | N.A. |  |
| Source [vivo] | R1-2212595 | 5*** | DDDUU | SU-MIMO | 10 | 30 | 13.41 | 13 | 91.67% | Note 1,5 |
|  |  |  |  |  |  | 10 | 10.13 | 10 | 94.18% |  |
| Source [vivo] | R1-2212595 | 10*** | DDDUU | SU-MIMO | 10 | 30 | 13.85 | 13 | 92.72% | Note 1,5 |
|  |  |  |  |  |  | 10 | 10.76 | 10 | 94.61% |  |
| Source [vivo] | R1-2212595 | 11*** | DDDUU | SU-MIMO | 10 | 30 | 14.94 | 14 | 92.45% | Note 1,5 |
|  |  |  |  |  |  | 10 | 14.54 | 14 | 93.07% |  |
| Source [vivo] | R1-2212595 | 6.2*** | DDDUU | SU-MIMO | 10 | 30 | 16.73 | 16 | 92.26% | Note 1,6,11 |
|  |  |  |  |  |  | 10 | 6.57 | 6 | 95.15% |  |
| Source [vivo] | R1-2212595 | 1.1*** | DDDUU | SU-MIMO | 10 | 30 | 15.56 | 15 | 93.42% | Note 1,3,3.1,8 |
|  |  |  |  |  |  | 10 | 3.51 | 3 | 93.24% |  |
| Source [vivo] | R1-2212595 | 1.1*** | DDDUU | SU-MIMO | 10 | 30 | 15.2 | 15 | 92.8% | Note 1,4,4.18 |
|  |  |  |  |  |  | 10 | 0 | 0 | N.A. |  |
| Source [vivo] | R1-2212595 | 12*** | DDDUU | SU-MIMO | 10 | 10 | 10.32 | 10 | 94.17 | Note 1,6 |
| Source [vivo] | R1-2212595 | 5*** | DDDUU | SU-MIMO | 10 | 10 | 13.63 | 13 | 92.46 | Note 1,7 |
| Source [vivo] | R1-2212595 | 11*** | DDDUU | SU-MIMO | 10 | 10 | 15.15 | 15 | 92.21 | Note 1,7 |
| Note 1: BS antenna parameters: 32TxRUs, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1:4,4) Note 2: SR periodicity = 5 ms Note 3: SR delay = 3 ms Note 3.1: SR delay contains at least: delay for aligning to the nearest SR transmission occasion when an XR packet arrives, gNB processing delay for the reported SR. Note 3.2: Scheduling delay = 5ms Note 4: SR delay = 5 ms Note 4.1: SR delay contains at least: delay for aligning to the nearest SR transmission occasion when an XR packet arrives, gNB processing delay for the reported SR. Note 5: Size of configured grant = 166.7 kbit, and the number of PUSCH occasions depends on some factors such as channel quality of the UE, and available resource, etc. Note 6: Size of configured grant = 80 bit Note 7: Size of configured grant = 83.4 kbit, and the number of PUSCH occasions depends on some factors such as channel quality of the UE, and available resource, etc. Note 7.1: Size of configured grant = 100 kbit Note 7-2: Size of configured grant = 12096 bytes (128 PRBs with MCS 24) Note 8: Size of initial UL grant = 83.4 kbit Note 9: Size of initial UL grant = 400 kbits Note 10: Size of initial UL grant = 128 PRBs with dynamic MCS selection Note 11: BSR delay = 2.5 ms Note 12: Scheduling delay is 5 ms-15 ms Note 13: Scheduling delay is 0 ms-15 ms Note 14: Scheduling delay is 2 ms-15 ms * CG periodicity = 16 ms ** CG periodicity = 10 ms *** CG periodicity = (17,17,16) ms to align each CG occasion with a corresponding XR packet |  |  |  |  |  |  |  |  |  |  |

Table B.1.6-2: FR1, UL, DU, AR at 60 FPS

| Source | Tdoc Source | Scheme | TDD format | SU/MU-MIMO | Data rate (Mbps) | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source [Ericsson] | R1-2210923 | 1.1 | DDDSU | SU-MIMO | 10 | 30 | 6.42 | 6 | 92% | Note 1,2,10,18 |
|  |  |  |  |  |  | 15 | 0.68 | 0 | N.A. |  |
| Source [Ericsson] | R1-2210923 | 1.1 | DDDSU | SU-MIMO | 10 | 30 | 6.8 | 6 | 95% | Note 1,2,11,14,18 |
|  |  |  |  |  |  | 15 | 2.06 | 2 | 90% |  |
| Source [Ericsson] | R1-2210923 | 2 | DDDSU | SU-MIMO | 10 | 30 | 6.86 | 6 | 95% | Note 1,2,11,18 |
|  |  |  |  |  |  | 15 | 4.54 | 4 | 93% |  |
| Source [Ericsson] | R1-2210923 | 4* | DDDSU | SU-MIMO | 10 | 30 | 6.35 | 6 | 93% | Note 1,2,8,15 |
|  |  | 4** |  |  |  | 15 | 2.75 | 2 | 92% |  |
| Source [Ericsson] | R1-2210923 | 6.1* | DDDSU | SU-MIMO | 10 | 30 | 6.97 | 6 | 95% | Note 1,2,8,14,18 |
|  |  | 6.1** |  |  |  | 15 | 4.97 | 4 | 94% |  |
| Source [Ericsson] | R1-2210923 | 3 | DDDSU | SU-MIMO | 10 | 30 | 7.1 | 7 | 91% | Note 1,2 |
|  |  |  |  |  |  | 15 | 5.02 | 5 | 90% |  |
| Source [Huawei] | R1-2212650 | 1.1 | DDDSU | MU-MIMO | 10 | 10 | 0 | 0 | N.A. | Note 1,3,12,14 |
| Source [Huawei] | R1-2212650 | 1.1 | DDDSU | MU-MIMO | 10 | 10 | 0 | 0 | N.A. | Note 1,3,13,14 |
| Source [Huawei] | R1-2212650 | 2 | DDDSU | MU-MIMO | 10 | 10 | 1.7 | 1 | 94.29% | Note 1,12,13,15 |
| Source [Huawei] | R1-2212650 | 4*** | DDDSU | MU-MIMO | 10 | 10 | 1 | 1 | 90.95% | Note 1,9-1 |
| Source [Huawei] | R1-2212650 | 1.1 | DDDSU | MU-MIMO | 10 | 15 | 0 | 0 | N.A. | Note 1,3,12,14,18 |
| Source [Huawei] | R1-2212650 | 1.1 | DDDSU | MU-MIMO | 10 | 15 | 0 | 0 | N.A. | Note 1,3,13,14,18 |
| Source [Huawei] | R1-2212650 | 2 | DDDSU | MU-MIMO | 10 | 15 | >3 | 3 | 96% | Note 1,13,15 |
| Source [Huawei] | R1-2212650 | 6.1*** | DDDSU | MU-MIMO | 10 | 15 | 0 | 1 | N.A. | Note 1,9-1,18 |
| Source [Huawei] | R1-2212650 | 5.1*** | DDDSU | MU-MIMO | 10 | 15 | 1.4 | 1 | 97.14% | Note 1,7-2,9-1,18 |
| Source [Huawei] | R1-2212650 | 9*** | DDDSU | MU-MIMO | 10 | 15 | 1.7 | 1 | 98.57% | Note 1,9-1,17,18 |
| Source [Sony] | R1-2211625 | 1.1 | DDDSU | SU-MIMO | 10 | 10 | 0 | 0 | N.A. | Note 1,3,11,19 |
|  |  |  |  |  |  | 15 | 3 | 3 | 90% |  |
|  |  |  |  |  |  | 30 | 6.35 | 6 | 91% |  |
| Source [Sony] | R1-2211625 | 4** | DDDSU | SU-MIMO | 10 | 10 | 0 | 0 | N.A. | Note 1 |
|  |  | 4** |  |  |  | 15 | 2.5 | 2 | 92% |  |
|  |  | 4* |  |  |  | 30 | 5.6 | 5 | 94.5% |  |
| Source [Sony] | R1-2211625 | 8** | DDDSU | SU-MIMO | 10 | 10 | 4 | 4 | 90% | Note 1,8,17 |
|  |  | 8** |  |  |  | 15 | 4.4 | 4 | 92.5% | Note 1,8,17 |
|  |  | 8* |  |  |  | 30 | 6.4 | 6 | 96.5% | Note 1,9,17 |
| Source [Sony] | R1-2211625 | 9** | DDDSU | SU-MIMO | 10 | 10 | 1.6 | 1 | 96% | Note 1,8,17 |
|  |  | 9** |  |  |  | 15 | 3.25 | 3 | 92.5% | Note 1,8,17 |
|  |  | 9* |  |  |  | 30 | 6 | 6 | 90% | Note 1,9,17 |
| Source [ZTE] | R1-2211906 | 5**** | DDDSU | SU-MIMO | 20 | 30 | <1 | 0 | N.A. | Note 1,4 |
|  |  |  |  |  |  | 15 | <1 | 0 | N.A. |  |
|  |  |  |  |  |  | 10 | <1 | 0 | N.A. |  |
| Source [ZTE] | R1-2211906 | 1.2 | DDDSU | SU-MIMO | 20 | 30 | 3.5 | 3 | 94% | Note 1,3,19 |
|  |  |  |  |  |  | 15 | <1 | 0 | N.A. |  |
|  |  |  |  |  |  | 10 | <1 | 0 | N.A. |  |
| Source [ZTE] | R1-2211906 | 1.1 | DDDSU | SU-MIMO | 20 | 30 | 3.6 | 3 | 95% | Note 1,3,13-1,18 |
|  |  |  |  |  |  | 15 | 2.1 | 2 | 91% |  |
|  |  |  |  |  |  | 10 | <1 | 0 | N.A. |  |
| Source [ZTE] | R1-2211906 | 9**** | DDDSU | SU-MIMO | 20 | 30 | 3.9 | 3 | 96% | Note 1,5 |
|  |  |  |  |  |  | 15 | 3.5 | 3 | 93% |  |
|  |  |  |  |  |  | 10 | 2.6 | 2 | 95% |  |
| Source [ZTE] | R1-2211906 | 6.1**** | DDDSU | SU-MIMO | 20 | 30 | 3.7 | 3 | 95% | Note 1, 7-1, 9,18 |
|  |  |  |  |  |  | 15 | 2.1 | 2 | 91% |  |
|  |  |  |  |  |  | 10 | <1 | 0 | N.A. |  |
| Source [ZTE] | R1-2211906 | 1.1 | DDDSU | SU-MIMO | 20 | 30 | 3.4 | 3 | 93% | Note 1,3,16,18 |
|  |  |  |  |  |  | 15 | <1 | 0 | N.A. |  |
| Source [ZTE] | R1-2211906 | 6.1**** | DDDSU | SU-MIMO | 20 | 30 | 3.5 | 3 | 93% | Note 1,7-1,16,18 |
|  |  |  |  |  |  | 15 | <1 | 0 | N.A. |  |
| Source [ZTE] | R1-2211906 | 5**** | DDDSU | SU-MIMO | 10 | 15 | <1 | 0 | N.A. | Note 1,6 |
|  |  |  |  |  |  | 10 | <1 | 0 | N.A. |  |
| Source [ZTE] | R1-2211906 | 1.2 | DDDSU | SU-MIMO | 10 | 15 | 4 | 4 | 90% | Note 1,3,19 |
|  |  |  |  |  |  | 10 | <1 | 0 | N.A. |  |
| Source [ZTE] | R1-2211906 | 1.1 | DDDSU | SU-MIMO | 10 | 15 | 7 | 7 | 90% | Note 1,3,13-1,18 |
|  |  |  |  |  |  | 10 | <1 | 0 | N.A. |  |
| Source [ZTE] | R1-2211906 | 9**** | DDDSU | SU-MIMO | 10 | 15 | 8.2 | 8 | 91% | Note 1,7 |
|  |  |  |  |  |  | 10 | 5.4 | 5 | 91% |  |
| Source [ZTE] | R1-2211906 | 6.1**** | DDDSU | SU-MIMO | 10 | 15 | 7.1 | 7 | 91% | Note 1,7-1, 9,18 |
|  |  |  |  |  |  | 10 | <1 | 0 | N.A. |  |
| Note 1: 64TxRUs, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1:4,8) Note 2: SR periodicity = 5ms Note 3: SR periodicity = 2.5 ms Note 4: 10 CG configurations with single CG PUSCH in a period in each CG configuration Note 5: 10 CG PUSCH in a period Note 6: 8 CG configurations with single CG PUSCH in a period in each CG configuration Note 7: 8 CG PUSCH in a period Note 7-1: Single CG PUSCH in a period Note 7-2: at least one PUSCH per CG occasion is pre-configured. Note 8: Size of configured grant = 60 kbit Note 9: Size of configured grant = 100 kbit Note 9-1: Size of configured grant is different for different UEs and it is up to configured resources and MCS Note 10: Size of initial UL grant = 288 bits Note 11: Size of initial UL grant = 117 kbit Note 12: Size of initial UL grant = 400 bits Note 13: Size of initial UL grant = 83.3 kbit Note 13-1: Size of initial UL grant is ideal at scheduler Note 14: No knowledge of XR traffic periodicity is assumed Note 15: It is assumed that the XR periodicity, frame size range and arrivals related information is known by gNB Note 16: BSR error probability is 10% Note 17: The indication of unused occasions was conveyed via new separate UCI Note 18: Scheduling delay is 2.5 ms Note 19: Scheduling delay is 5 ms * CG periodicity = 5 ms ** CG periodicity = 2.5 ms *** CG periodicity = (17,17,16) ms, the periodicities of packet arrival and CG PUSCH have been aligned ****CG periodicity = 16.5 ms, the periodicities of packet arrival and the first CG PUSCH in each period have been aligned |  |  |  |  |  |  |  |  |  |  |

Table B.1.6-3: FR1, UL, InH, AR pose/control at 250 FPS

| Source | Tdoc Source | Scheme | TDD format | SU/MU-MIMO | Data rate (Mbps) | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source [vivo] | R1-2212595 | 1.1 | DDDSU | SU-MIMO | 0.2 | 10 | >30 |  |  | Note 1,2,4 |
| Source [vivo] | R1-2212595 | 1.1 | DDDSU | SU-MIMO | 0.2 | 10 | >30 |  |  | Note 1,3,4 |
| Source [vivo] | R1-2212595 | 4* | DDDSU | SU-MIMO | 0.2 | 10 | >30 |  |  | Note 1 |
| Note 1: BS antenna parameters: 32TxRUs, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1:4,4) Note 2: SR delay = 3 ms Note 3: SR delay = 5 ms Note 4: Size of initial UL grant = 100 bytes * CG periodicity = 5 ms |  |  |  |  |  |  |  |  |  |  |

Based on the evaluation results in Table B.1.6-1, Table B.1.6-2 the following observations can be made.

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [Ericsson] that the capacity is increased from 6.42 UEs per cell with DG scheduling with SR followed by small initial UL grant (288 bits) with BSR and data (scheme 1.1) to 6.86 UEs per cell with pre-scheduling dynamic grant (scheme 2) (7%). The capacity gain for 15ms PDB is 567%.

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [Ericsson] that the capacity is increased from 6.8 UEs per cell with DG scheduling with SR followed by large initial UL grant (117 kbit) with BSR and data (scheme 1.1) to 6.86 UEs per cell with pre-scheduling dynamic grant (scheme 2) (1%). The capacity gain for 15ms PDB is 120%.

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [Ericsson] that the capacity is decreased from 6.42 UEs per cell with DG scheduling with SR followed by small initial UL grant (288 bits) with BSR and data (scheme 1.1) to 6.35 UEs per cell with single CG configuration (scheme 4) (-1%).

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [Ericsson] that the capacity is decreased from 6.8 UEs per cell with DG scheduling with SR followed by large initial UL grant (117 kbit) with BSR and data (scheme 1.1) to 6.35 UEs per cell with single CG configuration (scheme 4) (-7%).

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 15ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [Ericsson] that the capacity is increased from 0.68 UEs per cell with DG scheduling with SR followed by small initial UL grant (288 bits) with BSR and data (scheme 1.1) to 2.75 UEs per cell with single CG configuration (scheme 4) (304%).

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 15ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [Ericsson] that the capacity is increased from 2.06 UEs per cell with DG scheduling with SR followed by large initial UL grant (117 kbit) with BSR and data (scheme 1.1) to 2.75 UEs per cell with single CG configuration (scheme 4) (33%).

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [Ericsson] that the capacity is increased from 6.35 UEs per cell with single CG configuration (scheme 4) to 6.86 UEs per cell with pre-scheduling dynamic grant (scheme 2) (8%). The capacity gain for 15ms PDB is 65%.

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [Ericsson] that the capacity is increased from 6.35 UEs per cell with single CG configuration (scheme 4) to 6.97 UEs per cell with hybrid scheduling CG+DG, where CG resources are configured with a certain size in every UL slot in order to transmit BSR and data when new data arrives (scheme 6.1) (10%). The capacity gain for 15ms PDB is 81%.

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [Ericsson] that the capacity is increased from 6.86 UEs per cell with pre-scheduling dynamic grant (scheme 2) to 6.97 UEs per cell with hybrid scheduling CG+DG, where CG resources are configured with a certain size in every UL slot in order to transmit BSR and data when new data arrives (scheme 6.1) (2%). The capacity gain for 15ms PDB is 9%.

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [Ericsson] that the capacity is increased from 6.97 UEs per cell with hybrid scheduling CG+DG, where CG resources are configured with a certain size in every UL slot in order to transmit BSR and data when new data arrives (scheme 6.1) to 7.1 UEs per cell with dynamic scheduling with genie BSR (scheme 3) (2%). The capacity gain for 15ms PDB is 1%.

- For FR1, InH, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [InterDigital] that the capacity is decreased from 7.1 UEs per cell with DG scheduling with SR followed by initial UL grant with BSR and data (scheme 1.1) to 6 UEs per cell with single CG configuration (scheme 4) (-15%).

- For FR1, InH, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [InterDigital] that the capacity is increased from 2 UEs per cell with DG scheduling with SR followed by initial UL grant with BSR and data (scheme 1.1) to 4.7 UEs per cell with single CG configuration (scheme 4) (135%).

- For FR1, InH, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [InterDigital] that the capacity is increased from 7.1 UEs per cell with DG scheduling with SR followed by initial UL grant with BSR and data (scheme 1.1) to 7.5 UEs per cell with enhanced CG with flexible resource allocation (scheme 7) (6%).

- For FR1, InH, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [InterDigital] that the capacity is increased from 2 UEs per cell with DG scheduling with SR followed by initial UL grant with BSR and data (scheme 1.1) to 5.2 UEs per cell with enhanced CG with flexible resource allocation (scheme 7) (160%).

- For FR1, InH, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [CATT] that the capacity is decreased from 5.4 UEs per cell with DG scheduling with SR followed by initial UL grant with BSR and data (scheme 1.1) to 0 UEs per cell with single CG configuration (scheme 4).

- For FR1, InH, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [CATT] that the capacity is increased from 5.4 UEs per cell with DG scheduling with SR followed by initial UL grant with BSR and data (scheme 1.1) to 7.3 UEs per cell with hybrid CG+DG scheduling (scheme 6.3) (35%).

- For FR1, InH, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [CATT] that the capacity is increased from 5.4 UEs per cell with DG scheduling with SR followed by initial UL grant with BSR and data (scheme 1.1) to 7.2 UEs per cell with pre-scheduling dynamic grant (scheme 2) (33%).

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [Sony] that the capacity is decreased from 6.35 UEs per cell with DG scheduling with SR followed by initial UL grant with BSR and data (scheme 1.1) to 5.6 UEs per cell with single CG configuration (scheme 4) (-11%). The capacity drop for 15ms PDB is 17%.

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [Sony] that the capacity is increased from 6.35 UEs per cell with DG scheduling with SR followed by initial UL grant with BSR and data (scheme 1.1) to 6.4 UEs per cell with enhanced CG with dynamic adaptation of CG parameters and indication of unused/used CG PUSCH occasion(s) (scheme 8) (1%). The capacity gain for 15ms PDB is 44%.

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [Sony] that the capacity is decreased from 6.35 UEs per cell with DG scheduling with SR followed by initial UL grant with BSR and data (scheme 1.1) to 6 UEs per cell with enhanced CG with indication of unused/used CG PUSCH occasion(s) (scheme 9) (-6%).

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 15ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [Sony] that the capacity is increased from 3 UEs per cell with DG scheduling with SR followed by initial UL grant with BSR and data (scheme 1.1) to 3.25 UEs per cell with enhanced CG with indication of unused/used CG PUSCH occasion(s) (scheme 9) (8%).

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 20Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [ZTE] that the capacity is increased from 3.5 UEs per cell with DG scheduling with SR followed by UL grant with BSR only (scheme 1.2) to 3.6 UEs per cell with DG scheduling with SR followed by initial UL grant with BSR and data (scheme 1.1)  (3%). The trend is similar for PDB=10 ms and 15 ms as well as for 10Mbps scenario.

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 20Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [ZTE] that the capacity is decreased from 3.6 UEs per cell with DG scheduling with SR followed by UL grant with BSR and data (scheme 1.1) to 0 UEs per cell with multiple CG configurations (scheme 5). The trend is similar for PDB=10 ms and 15 ms as well as for 10Mbps scenario.

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 20Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [ZTE] that the capacity is increased from 3.6 UEs per cell with DG scheduling with SR followed by UL grant with BSR and data (scheme 1.1) to 3.9 UEs per cell with enhanced CG with indication of unused/used CG PUSCH occasion(s) (scheme 9) (8%).

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 20Mbps, 15ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [ZTE] that the capacity is increased from 2.1 UEs per cell (scheme 1.1) to 3.5 UEs per cell with enhanced CG with indication of unused/used CG PUSCH occasion(s) (scheme 9) (66.7%).  The trend is similar if compare (scheme 9) with (scheme 6.1) for PDB=15 ms and for 20Mbps scenario.

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 20Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [ZTE] that the capacity is increased from 3.7 UEs per cell with hybrid CG+DG scheduling (scheme 6.1) to 3.9 UEs per cell with enhanced CG with indication of unused/used CG PUSCH occasion(s) (scheme 9) (5%).

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 15ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [ZTE] that the capacity is increased from 4 UEs per cell (scheme 1.2) to 8.2 UEs per cell with enhanced CG with indication of unused/used CG PUSCH occasion(s) (scheme 9) (95.2%).

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 20Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [ZTE] that the capacity is increased from 3.5 UEs per cell (scheme 6.1) assuming 10% BSR error probability to 3.9 UEs per cell with enhanced CG with indication of unused/used CG PUSCH occasion(s) (scheme 9) (11.4%).

- For FR1, InH, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [vivo] that the capacity is increased from 2.33 UEs per cell with DG scheduling with SR followed by initial UL grant with BSR and data (scheme 1.1) to 6.22 UEs per cell with multiple CG configurations (scheme 5) (167%).

- For FR1, InH, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [vivo] that the capacity is decreased from 2.33 UEs per cell with DG scheduling with SR followed by initial UL grant with BSR and data (scheme 1.1) to 6.12 UEs per cell with hybrid CG+DG scheduling (scheme 6.2) (163%).

- For FR1, InH, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [vivo] that the capacity is decreased from 6.12 UEs per cell with hybrid CG+DG scheduling (scheme 6.2) to 6.42 UEs per cell with enhanced CG with MAC CE based dynamic resource adjustment indication (scheme 10) (5%).

- For FR1, InH, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [vivo] that the capacity is decreased from 6.12 UEs per cell with hybrid CG+DG scheduling (scheme 6.2) to 7.81 UEs per cell with enhanced CG with UCI based dynamic resource adjustment indication (scheme 11) (28%).

- For FR1, InH, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [vivo] that the capacity is decreased from 6.12 UEs per cell with hybrid CG+DG scheduling (scheme 6.2) to 8.79 UEs per cell with enhanced CG with UCI based dynamic resource adjustment indication (scheme 12) (44%).

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 10ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [Huawei] that the capacity is increased from 0 UEs per cell with DG scheduling with SR followed by small initial UL grant (400 bits) with BSR and data (scheme 1.1) to 1 UEs per cell with pre-scheduling dynamic grant (scheme 2).

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 10ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [Huawei] that the capacity is increased from 0 UEs per cell with DG scheduling with SR followed by large initial UL grant (83.3 kbit) with BSR and data (scheme 1.1) to 1 UEs per cell with pre-scheduling dynamic grant (scheme 2).

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 30ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [Huawei] that the capacity is increased from 1 UEs per cell with cell with pre-scheduling dynamic grant (scheme 2) to 1.7 UEs per cell with single CG configuration (scheme 4) (70%).

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 15ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [Huawei] that the capacity is increased from 0 UEs per cell with DG scheduling with SR followed by small initial UL grant (400 bits) with BSR and data (scheme 1.1) to 3 UEs per cell with pre-scheduling dynamic grant (scheme 2).

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 15ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [Huawei] that the capacity is increased from 0 UEs per cell with DG scheduling with SR followed by large initial UL grant (83.3 kbit) with BSR and data (scheme 1.1) to 3 UEs per cell with pre-scheduling dynamic grant (scheme 2).

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 15ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [Huawei] that the capacity is decreased from 3 UEs per cell with cell with pre-scheduling dynamic grant (scheme 2) to 1.4 UEs per cell with multiple CG configuration (scheme 5.1) (-53%).

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 15ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [Huawei] that the capacity is decreased from 3 UEs per cell with cell with pre-scheduling dynamic grant (scheme 2) to 0 UEs per cell with Hybrid CG+DG scheduling (scheme 6.1).

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 15ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [Huawei] that the capacity is decreased from 3 UEs per cell with cell with pre-scheduling dynamic grant (scheme 2) to 1.7 UEs per cell with Enhanced CG with indication of unused/used CG PUSCH occasion(s) (scheme 9) (-43%).

### B.1.7	Scheduling restrictions due to RRM measurements

This clause captures the capacity performance evaluation results with and without intra- and inter frequency RRM measurement restrictions. Scheduling restrictions apply for the UEs during time-intervals where it is performing intra-frequency RRM measurements at FR2, or gap assisted inter-frequency RRM measurements. Such scheduling restrictions does not allow to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS. When XR frame arrival collides with RRM measurements, the XR frame will be delayed for the duration of SMTC window or measurement gap (MG).

The performance with SMTC/MG scheduling restrictions (scheme 2 and 4 in Table B.1.7-1 and Table B.1.7-2) has been compared against no SMTC/MG scheduling restrictions as well as with adaptation of MG period. Particularly, the following schemes have been evaluated:

Scheme 1: Without SMTC scheduling restrictions. In this scheme, no scheduling restrictions for FR2 intra-frequency measurements were assumed.

Scheme 2: With SMTC scheduling restrictions. In this scheme, scheduling restrictions every 20 ms time period for an SMTC window of 5 ms for FR2 intra-frequency measurements were assumed. In case the SMTC windows with scheduling restrictions collide with time periods where the gNB would need to schedule the XR transmission, XR frame transmission will be delayed.

Scheme 3: No scheduling restrictions during measurement gap. In this scheme, no scheduling restrictions during measurement gap for inter-frequency measurements were assumed.

Scheme 4: With scheduling restrictions during measurement gap. In this scheme, scheduling restrictions for the duration of measurement gap length of 6 ms are applied. Measurement gaps (MG) are configured to allow UE to do inter-frequency neighbour cell measurement and the corresponding RF tuning for RRM purposes (e.g. mobility, load balancing, CA set-up). In case the MG with scheduling restrictions collide with time periods where the gNB would need to schedule the XR transmission, XR frame transmission will be delayed.

Scheme 5.1: Adaptation of MG period. The MG period is adapted to be 2 times when PCell RSRP is 2dB better than the best neighbor cell and 4 times when PCell RSRP is 4dB better than the best neighbour cell. The UE speed is 30km/h.

Scheme 5.2 Adaptation of MG period. The MG period is adapted to be 4 times when PCell RSRP is 2dB better than the best neighbor cell and 8 times when PCell RSRP is 4dB better than the best neighbour cell. The UE speed is 30km/h.

Table B.1.7-1: FR2, DL, InH, VR/AR at 60 FPS

| Source | Tdoc Source | Scheme | TDD format | SU/MU-MIMO | Data rate (Mbps) | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source [Nokia] | R1-2211552 | 1 | DDDSU | SU-MIMO | 30 | 10 | 7 | 7 | 90% | Note 1 |
| Source [Nokia] | R1-2211552 | 2 | DDDSU | SU-MIMO | 30 | 10 | 3.1 | 3 | 92% | Note 1 |
| Source [Nokia] | R1-2211552 | 1 | DDDSU | SU-MIMO | 30 | 15 | 9 | 9 | 91% | Note 1 |
| Source [Nokia] | R1-2211552 | 2 | DDDSU | SU-MIMO | 30 | 15 | 6.25 | 6 | 93% | Note 1 |
| Note 1: BS antenna parameters: 2TxRUs, (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1) |  |  |  |  |  |  |  |  |  |  |

Table B.1.7-2: FR1, DL, DU, VR/AR at 60 FPS

| Source | Tdoc Source | Scheme | TDD format | SU/MU-MIMO | Data rate (Mbps) | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source [MediaTek] | R1-2212254 | 3 | DDDSU | SU-MIMO | 30 | 10 | 10.6 | 10 | 90.67% | Note 1 |
| Source [MediaTek] | R1-2212254 | 4* | DDDSU | SU-MIMO | 30 | 10 | 0 | 0 | N.A. | Note 1 |
| Source [MediaTek] | R1-2212254 | 4** | DDDSU | SU-MIMO | 30 | 10 | 2 | 2 | 88.1% | Note 1 |
| Source [MediaTek] | R1-2212254 | 5.1* | DDDSU | SU-MIMO | 30 | 10 | 0.5 | 0 | N.A. | Note 1 |
| Source [MediaTek] | R1-2212254 | 5.1** | DDDSU | SU-MIMO | 30 | 10 | 7.2 | 7 | 90.48% | Note 1 |
| Source [MediaTek] | R1-2212254 | 5.2* | DDDSU | SU-MIMO | 30 | 10 | 0.8 | 8 | N.A. | Note 1 |
| Source [MediaTek] | R1-2212254 | 5.2** | DDDSU | SU-MIMO | 30 | 10 | 8.7 | 8 | 92.62% | Note 1 |
| Source [MediaTek] | R1-2212254 | 4*** | DDDSU | SU-MIMO | 30 | 10 | 0 | 0 | N.A. | Note 1 |
| Source [MediaTek] | R1-2212254 | 4**** | DDDSU | SU-MIMO | 30 | 10 | 7.2 | 7 | 90.48% | Note 1 |
| Source [MediaTek] | R1-2212254 | 5.1*** | DDDSU | SU-MIMO | 30 | 10 | 0.9 | 0 | N.A. | Note 1 |
| Source [MediaTek] | R1-2212254 | 5.1**** | DDDSU | SU-MIMO | 30 | 10 | 8.7 | 8 | 92.62% | Note 1 |
| Source [MediaTek] | R1-2212254 | 5.2*** | DDDSU | SU-MIMO | 30 | 10 | 5.5 | 5 | 90.32% | Note 1 |
| Source [MediaTek] | R1-2212254 | 5.2**** | DDDSU | SU-MIMO | 30 | 10 | 9.2 | 9 | 90.66% | Note 1 |
| Note 1: BS antenna parameters: 64TxRUs, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1:4,8) * MG configuration 40 ms, all UE ** MG configuration 40 ms, cell-edge 20% UE *** MG configuration 80 ms, all UE **** MG configuration 80 ms, cell-edge 20% UE |  |  |  |  |  |  |  |  |  |  |

Based on the evaluation results in Table B.1.7-1 and Table B.1.7-2 the following observations can be made.

- For FR2, InH, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 2TxRU, it is observed from Source [Nokia] that the capacity is increased from 3.1 UEs per cell with SMTC scheduling restrictions for FR2 intra-frequency measurements (5 ms SMTC window every 20 ms) to 7 UEs per cell without SMTC scheduling restrictions for FR2 intra-frequency measurements (126%). For 15ms PDB the capacity gain is 44%.

- For FR1, DU, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [MediaTek] that the capacity is increased from 0 UEs per cell with MG scheduling restrictions for inter-frequency measurements for all UEs (6 ms MG length every 40 ms) to 10.6 UEs per cell without MG scheduling restrictions for inter-frequency measurements. Similar trend is observed for MG configuration 80 ms.

- For FR1, DU, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [MediaTek] that the capacity is increased from 2 UEs per cell with MG scheduling restrictions for inter-frequency measurements for all 20% UEs (6 ms MG length every 40 ms) to 10.6 UEs per cell without MG scheduling restrictions for inter-frequency measurements (400%). Similar trend is observed for MG configuration 80 ms.

- For FR1, DU, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [MediaTek] that the capacity is increased from 0 UEs per cell with MG scheduling restrictions for inter-frequency measurements for all UEs (6 ms MG length every 40 ms) to 0.5 UEs per cell with adaptation of MG period for inter-frequency measurements, where MG period is adapted to be 2 times when PCell RSRP is 2dB better than the best neighbor cell and 4 times when PCell RSRP is 4dB better than the best neighbour cell. Similar trend is observed for MG configuration 80 ms.

- For FR1, DU, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [MediaTek] that the capacity is increased from 2 UEs per cell with MG scheduling restrictions for inter-frequency measurements for all 20% UEs (6 ms MG length every 40 ms) to 7.2 UEs per cell with adaptation of MG period restrictions for inter-frequency measurements, where MG period is adapted to be 2 times when PCell RSRP is 2dB better than the best neighbor cell and 4 times when PCell RSRP is 4dB better than the best neighbour cell (250%). Similar trend is observed for MG configuration 80 ms.

- For FR1, DU, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [MediaTek] that the capacity is increased from 0 UEs per cell with MG scheduling restrictions for inter-frequency measurements for all UEs (6 ms MG length every 40 ms) to 0.8 UEs per cell with adaptation of MG period for inter-frequency measurements, where MG period is adapted to be 2 times when PCell RSRP is 2dB better than the best neighbor cell and 4 times when PCell RSRP is 4dB better than the best neighbour cell. Similar trend is observed for MG configuration 80 ms.

- For FR1, DU, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source [MediaTek] that the capacity is increased from 2 UEs per cell with MG scheduling restrictions for inter-frequency measurements for all 20% UEs (6 ms MG length every 40 ms) to 8.7 UEs per cell with adaptation of MG period restrictions for inter-frequency measurements, where MG period is adapted to be 2 times when PCell RSRP is 2dB better than the best neighbor cell and 4 times when PCell RSRP is 4dB better than the best neighbour cell (335%). Similar trend is observed for MG configuration 80 ms.

### B.1.8	Buffer status report

This clause captures the capacity performance evaluation results of enhancements related to buffer status report (BSR).

The performance of legacy BSR (scheme 1 in Table B.1.8-1 and Table B.1.8-2) has been compared against enhanced BSR schemes. Particularly, the following schemes have been evaluated:

Scheme 1: Legacy dynamic scheduling with legacy BSR.

Scheme 2: BSR with precise buffer size, where new buffer status (BS) table(s) with finer granularity is designed.

Scheme 3: XR-specific triggering mechanism of BSR report. In this scheme, the pre-scheduling dynamic grant based on the XR awareness is considered and XR specific PDCCH monitoring is aligned with the periodicity of XR packet generation. The BSR is also enhanced to have XR-specific triggering mechanism of BSR report to minimize the scheduling delay. The XR-specific BSR report triggering mechanism is that the BSR report will be triggerred at the 1st PUSCH of each XR packet at each cycle to inform gNB the remaining packet size to be transmitted after the 1st PUSCH transmission of the XR traffic. When gNB receive the BSR, gNB would schedule UE the UL PUSCH transmission until the completion of XR packet delivery in this cycle.

Table B.1.8-1: FR1, UL, InH, AR at 60 FPS

| Source | Tdoc Source | Scheme | TDD format | SU/MU-MIMO | Data rate (Mbps) | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source [CATT] | R1-2211175 | 1 | DDDUU | SU-MIMO | 10 | 30 | 5.4 | 5 | 90 | Note 1,2 |
| Source [CATT] | R1-2211175 | 3 | DDDUU | SU-MIMO | 10 | 30 | 7.2 | 7 | 90 | Note 1,2 |
| Note 1: BS antenna parameters: 32TxRUs, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1:4,4) Note 2: SR/BSR periodicity = 5 ms |  |  |  |  |  |  |  |  |  |  |

Table B.1.8-2: FR1, UL, DU, AR at 60 FPS

| Source | Tdoc Source | Scheme | TDD format | SU/MU-MIMO | Data rate (Mbps) | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source [ZTE] | R1-2211906 | 1* | DDDSU | MU-MIMO | 10 | 30 | 9.5 | 9 | 95 | Note 1,2 |
|  |  |  |  |  | 20 |  | 3.4 | 3 | 91 |  |
| Source [ZTE] | R1-2211906 | 2 | DDDSU | MU-MIMO | 10 | 30 | 10.9 | 10 | 94 | Note 1,2 |
|  |  |  |  |  | 20 |  | 5.1 | 5 | 90 |  |
| Note 1: BS antenna parameters: 64TxRUs, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1:4,8) Note 2: SR periodicity = 2.5 ms * The length of the field for the Short BSR format and the Short Truncated BSR format is 5 bits |  |  |  |  |  |  |  |  |  |  |

Based on the evaluation results in Table B.1.8-1 and Table B.1.8-2 the following observations can be made.

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 30ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [ZTE] that the capacity is increased from 9.5 UEs per cell with Legacy BSR to 10.9 UEs per cell with BSR with precise buffer size (15%).

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 20Mbps, 30ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [ZTE] that the capacity is increased from 3.4 UEs per cell with Legacy BSR to 5.1 UEs per cell with BSR with precise buffer size (50%).

- For FR1, InH, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 30ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source [CATT] that the capacity is increased from 5.4 UEs per cell with dynamic grant scheduling to 7.2 UEs per cell with XR-specific triggering mechanism of BSR report (33%).

### B.1.9	UL delay-aware scheduling

This clause captures the capacity performance evaluation results of enhancements related to UL delay aware scheduling.

The performance proportional fair scheduling (scheme 1 in Table B.1.9-1) has been compared against delay-aware scheduling. Particularly, the following schemes have been evaluated:

Scheme 1: Proportional fair (PF) scheduling, where gNB uses PF scheduling, i.e., the scheduling priority of each user is calculated as the ratio of the instantaneous data rate over the historical data rate. The gNB does not consider delay during scheduling.

Scheme 2: Delay-aware scheduling, where gNB is not aware of the exact UL data arrival time and can only apply SR or BSR reception time for delay-aware scheduling.

Scheme 3: Delay-aware scheduling, where gNB is aware of the exact data arrival time and uses the exact data arrival time for delay-aware scheduling.

Table B.1.9-1: FR1, UL, DU, AR at 60 FPS

| Source | Tdoc Source | Scheme | TDD format | SU/MU-MIMO | Data rate (Mbps) | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source [Huawei] | R1-2210907 | 1 | DDDSU | MU-MIMO | 10 | 15 | 3 | 3 | 90 | Note 1, 2 |
| Source [Huawei] | R1-2210907 | 2 | DDDSU | MU-MIMO | 10 | 15 | 3.2 | 3 | 90.5 | Note 1, 2 |
| Source [Huawei] | R1-2210907 | 3 | DDDSU | MU-MIMO | 10 | 15 | 3.8 | 3 | 91.5 | Note 1, 2 |
| Note 1: BS antenna parameters: 64TxRUs, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1:4,8) Note 2: SR/BSR periodicity = 5 ms |  |  |  |  |  |  |  |  |  |  |

Based on the evaluation results in Table B.1.9-1 the following observations can be made.

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 15ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [Huawei] that the capacity is increased from 3 UEs per cell with proportional fair scheduling to 3.2 UEs per cell with delay-aware scheduling, where gNB is not aware of the exact data arrival time (7%).

- For FR1, DU, UL, with 100MHz bandwidth for AR single-stream traffic model, 10Mbps, 15ms PDB, 60 FPS, with MU-MIMO and 64TxRU, it is observed from Source [Huawei] that the capacity is increased from 3.2 UEs per cell with delay-aware scheduling, where gNB is not aware of the exact data arrival time to 3.8 UEs per cell with delay-aware scheduling, where gNB is aware of the exact data arrival time (19%).

### B.1.10	XR-specific playoutDelayForMediaStartup for gNB scheduling awareness

This clause captures the capacity performance evaluation results of enhancements related to gNB awareness about playoutDelayForMediaStartup and appLayerBufferLevel, where the former is the waiting time that the user experiences for media start-up and the latter is buffer level, which indicates the playout duration for which media data of all active media components is available starting from the current playout time, where.

The performance dynamic grant scheduling (scheme 1 in Table B.1.10-1) has been compared against the scheme with playoutDelayForMediaStartup and appLayerBufferLevel awareness. Particularly, the following schemes have been evaluated:

Scheme 1: Dynamic grant scheduling.

Scheme 2: XR-specific playoutDelayForMediaStartup scheme. In this scheme, the playoutDelayForMediaStartup and appLayerBufferLevel is shared by the UE with the gNB. The scheme assumes, the feedback of XR-specific playoutDelayForMediaStartup and appLayerBufferLevel from UE would give the gNB scheduler additional delay budget in scheduling the XR data transmission to achieve additional link adaptation gain. For example, when the reported appLayerBufferLevel is 3 frames, gNB scheduler extends PDB for scheduling the XR packet on PDSCH for additional 50 ms. gNB prioritizes the UE scheduling based on the extended delay budged from the reported playout delay size and buffer level, in which a group UEs in the same priority queue are with the same length of playout delay.

Table B.1.10-1: FR1, DL, InH, VR/AR at 60 FPS

| Source | Tdoc Source | Scheme | TDD format | SU/MU-MIMO | Data rate (Mbps) | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source [CATT] | R1-2211175 | 1 | DDDSU | MU-MIMO | 30 | 10 | 10.9 | 10 | 90.97% | Note 1,2 |
| Source [CATT] | R1-2211175 | 1 | DDDSU | SU-MIMO | 30 | 10 | 3.7 | 3 | 92.5% | Note 1,2 |
| Source [CATT] | R1-2211175 | 1 | DDDSU | MU-MIMO | 30 | 10 | 11.5 | 11 | 95.83% | Note 1,3 |
| Source [CATT] | R1-2211175 | 1 | DDDSU | SU-MIMO | 30 | 10 | 5.8 | 5 | 96.7% | Note 1,3 |
| Source [CATT] | R1-2211175 | 2* | DDDSU | MU-MIMO | 30 | 10 | 16 | 16 | 95% | Note 1 |
| Source [CATT] | R1-2211175 | 2** | DDDSU | MU-MIMO | 30 | 10 | 20 | 20 | 92% | Note 1 |
| Source [CATT] | R1-2211175 | 2*** | DDDSU | MU-MIMO | 30 | 10 | 20 | 20 | 91% | Note 1 |
| Source [CATT] | R1-2211175 | 2** | DDDSU | SU-MIMO | 30 | 10 | 7.3 | 7 | 91.3% | Note 1 |
| Source [CATT] | R1-2211175 | 2*** | DDDSU | SU-MIMO | 30 | 10 | 11.8 | 11 | 98.3% | Note 1 |
| Source [CATT] | R1-2211175 | 2**** | DDDSU | SU-MIMO | 30 | 10 | 11.3 | 11 | 94.4% | Note 1 |
| Note 1: BS antenna parameters: 32TxRUs, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1:4,4) Note 2: C-DRX configuration (16,12,4) Note 3: UE always on * 2 frames playout delay ** 3 frames playout delay *** 4 frames playout delay **** mixed playout delay {3, 4}) |  |  |  |  |  |  |  |  |  |  |

Based on the evaluation results in Table B.1.10-1 the following observations can be made.

- For FR1, DL, InH, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with MU-MIMO and 32TxRU, C-DRX configuration (16,12,4), it is observed from Source [CATT] that the capacity is increased from 10.9 UEs per cell with grant scheduling to 16/20/20 UEs per cell with XR-specific playoutDelayForMediaStartup scheme with 2/3/4 frames playout delay, respectively (47%/83%/83%).

- For FR1, DL, InH, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with MU-MIMO and 32TxRU, UE always on, it is observed from Source [CATT] that the capacity is increased from 11.5 UEs per cell with grant scheduling to 16/20/20 UEs per cell with XR-specific playoutDelayForMediaStartup scheme with 2/3/4 frames playout delay, respectively (39%/74%/74%).

- For FR1, DL, InH, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 32TxRU, C-DRX configuration (16,12,4), it is observed from Source [CATT] that the capacity is increased from 3.7 UEs per cell with grant scheduling to 7.3/11.8/11.3 UEs per cell with XR-specific playoutDelayForMediaStartup scheme with 3/4 and mixed {3,4} frames playout delay, respectively (97%/219%/205%).

- For FR1, DL, InH, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 32TxRU, UE always on, it is observed from Source [CATT] that the capacity is increased from 5.8 UEs per cell with grant scheduling to 7.3/11.8/11.3 UEs per cell with XR-specific playoutDelayForMediaStartup scheme with 3/4 and mixed {3,4} frames playout delay, respectively (26%/103%/95%).

Annex C (informative):
RAN2 Agreements

# C.1		RAN2#119-e

Agreements from RAN2#119-e meeting:

-	RAN2 does not intend to ask RAN1 to change their simulation assumptions;

-	RAN2 should take SA2/SA4 work into account.

-	RAN2 assumes that PDU Set based parameters and PDU Set related information may be used for better support of XR services. RAN2 can consider both UL and DL directions.

-	RAN2 will study PDU Set based parameters and PDU Set related information handling in Network and UE.

-	RAN2 to adopt the current SA2 definition of PDU Set as an application media unit as working assumption, subjected to further guidance from SA2 and SA4.

-	XR awareness discussion in RAN2 should consider PDU set characteristics and how to use the information available on those (for UL and/or DL). Can also consider how to handle data bursts.

-	RAN2 can study e.g. periodicity, arrival time, jitter and frame-size variations for XR awareness to enable power savings and capacity enhancements. Can study also how often such parameters change (i.e. how dynamic they are).

-	RAN2 can consider how PDU sets can be mapped to DRBs (FFS if SA2 discussion on PDU set mapping to QoS (sub-)flows impacts this).

-	RAN2 to focus on the following issues for power saving, as well necessary parameters XR-awareness to support such enhancements, i.e.:

-	DRX enhancements to address the issues of DRX cycle mismatch and jitter;

-	Identify necessary parameters from CN for XR-awareness for power saving.

-	Enhancements to Rel-17 PDCCH adaptation can be discussed based on RAN1 feedback, if they have any RAN2 impact.

-	RAN2-specific aspects can be studied based on contributions (e.g. multiple XR traffic flows with different periodicities, SFN wrap-around, RAN2-specific CDRX aspects, …).

-	As starting point, RAN2 can further discuss the solutions in TR 38.838 that can impact on L2 operation (e.g., BSR, LCP, assistance information for scheduling, packet discarding, prioritization) for XR-specific capacity improvement. RAN2-specific solutions are not precluded (even if RAN1 hasn’t discussed them before).

-	Enhancement to SPS/CG should be justified for XR scheduling and should be evaluated against dynamic grant (DG) scheduling which should be considered as baseline. Should justify why enhancements are needed.

-	RAN2 considers SPS enhancements may not be needed in Rel-18 XR since PDCCH capacity is not assumed to be a problem for XR. FFS if SPS has some power consumption benefits.

# C.2		RAN2#119bis-e

Agreements from RAN2#119bis-e meeting:

-	From RAN2 viewpoint, the following information would be useful for PDU set handling in UL and DL:

-	Semi-static information (from CN to RAN): At least PSER and PSDB;

-	Dynamic information: At least identifying which PDU belongs to which data burst/PDU set is also needed, including means to determine at least PDU set boundaries.

-	Capture the models 1a/b, 2a/b (from R2-2209777) in TR and indicate what is possible in current specifications and how. FFS how LCH options work in each case

-	SDAP maps each data packet in a PDU set to a single PDCP SDU, as in legacy (i.e. each PDU is only mapped to a single SDU).

-	HARQ and RLC re-/transmissions for XR traffic are done as in legacy (i.e. they are not based on XR PDU sets).

-	For UE transmitter, the PDCP discard should be performed per PDU set basis.

-	For UE transmitter, the PDCP discard is managed per SDU for PDU set, the PDCP entity discards all PDCP SDUs associated with the PDU set.

-	At least RRC pre-configuration and switching of configurations of DRX could be considered for enhancements of XR power saving. Other solutions are not precluded and can be further discussed.

-	Introduce new BS table(s) to reduce the quantisation errors (e.g. for high bit rates). FFS how new BSR tables are created and how they impact BSR formats (can be discussed in WI phase).

-	Delay information consists of at least “remaining time”.

-	RAN2 considers a delay information is useful for XR. FFS if dynamic reporting from UE to network (e.g. via BSR) is needed, or whether PSDB is sufficient. If we have delay information, it needs to distinguish how much data is buffered for which delay value. Stage-3 details (e.g. what’s contained, how the triggering is done) can be discussed in the WI phase.

-	If we have delay information reporting, RAN2 aims to define how the UE determines the “remaining time” in the delay information.

-	Current CG configurations can be reused for UL XR traffic. FFS if enhancements are needed (RAN1 is already discussing something). RAN2 can discuss this in the next meeting.

-	RAN2 can discuss potential enhancement to provide some assistant information on UL XR traffic for CG configurations at the gNB. FFS whether TSCAI can already provide all necessary information.

-	RAN2 discuss whether additional traffic or QoS related information on downlink traffic beyond what has been agreed by SA2 needs to be provided to RAN for UE power savings.

-	RAN2 study what traffic and QoS related information on uplink traffic (e.g. counterpart of what has been agreed by SA2) should be provided to RAN for UE power savings and how the information may be provided to RAN.

-	Capture in TR that traffic parameters and Jitter are semi-static info.

-	Can capture also SA2 agreements related to how they impact RAN2.

Annex Z (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-04 | RAN1#109 | R1-2204673 |  |  |  | Initial Skeleton | 0.0.1 |
| 2022-08 | RAN2#119 | R2-2207373 |  |  |  | Initial Skeleton | 0.0.1 |
| 2022-08 | RAN2#119 | R2-2207374 |  |  |  | Updated Structure | 0.0.2 |
| 2022-08 | RAN2#119 | R2-2208748 |  |  |  | First Endorsed Baseline | 0.1.0 |
| 2022-08 | RAN2#119 | R2-2208749 |  |  |  | Table of Content updated | 0.1.1 |
| 2022-09 | RAN2#119 | R2-2209220 |  |  |  | Overview and first RAN2 agreements included | 0.2.0 |
| 2022-10 | RAN2#119bis | R2-2210814 |  |  |  | Relevant definitions from 23.700-60 included Useful pieces of information from SA4 LS added (S4-220505 and S4aV220921) RAN2 agreements on PDU set handling, discard, L2 structure captured, BS tables and delay reporting added. | 0.3.0 |
| 2022-11 | RAN2#120 | R2-2212908 |  |  |  | Minor editoral corrections | 0.3.1 |