3GPP TSG RAN WG1 #112                                                             R1-2300447

Athens, Greece, February 27th – March 3rd, 2023

Source:	vivo

Title:	Other aspects on AI/ML for beam management

Agenda Item:	9.2.3.2

Document for:	Discussion and Decision

Introduction

In this contribution, we focus on the above two possible sub-use cases of beam management case1 and case2, i.e., spatial domain beam prediction and time domain beam prediction, for studying potential specification impacts.

- Major issues on AI beam sub use cases

Beam prediction solutions for BM-Case1 and BM-Case2

According to performance comparisons for different beam prediction solutions in our evaluation contribution [2], observations can be summarized as below:

- Original beam pair prediction

  - Beam pair prediction with fixed pattern can have good performance in ideal scenarios but it lacks flexibility;

  - Beam pair prediction with random pattern including L1-RSRP and Tx/Rx beam ID or angle as assistance information into the AI model is helpful to reduce performance loss;

  - Beam pair prediction with semi-random selection with Tx/Rx beam angle information as input barely suffers performance loss compared with the best fixed pattern;

  - Output size of original beam pair prediction is associated with the total number of Tx beams and the total number of Rx beams, which limits AI model deployed in difference gNB/UE capabilities with a distinct number of Tx/Rx beams.

- Enhanced beam pair prediction

  - Beam pair prediction with expected Tx and/or Rx beam information can enable the utilization of a trained AI model to different numbers of Tx or Rx beams with marginal performance loss;

  - Generalization performance on various number of Tx and/or Rx beams can be improved by using expected Tx beam information and/or expected Rx beam information as AI input for different AI deployment scenarios.

- DL Tx beam prediction

  - DL Tx beam prediction scheme can achieve the best performance with dynamically applicable to different number of Rx beams only when P2 procedure assuming the best Rx beam.

  - As the prediction targets of this solution is only associated with Tx beams, the generalization performance as well as beam prediction accuracy is improved with conditional Rx beam assumption.

Among all these solutions, one particular issue which requires more early discussion is beam pair prediction (including original beam pair prediction and enhanced beam pair prediction) v.s. DL Tx beam prediction (with DL Rx beam assumption). This decision impacts directions of further evaluation and standard impact discussion. Specifically, the model input of enhanced beam pair prediction scheme at least includes measured L1-RSPR with pre-configured pattern selection, corresponding Tx/Rx beam angle information, and expected Tx and/or Rx beam angle information, while measured L1-RSPR with pre-configured pattern selection and corresponding Tx/Rx beam angle information shall be used in DL Tx beam prediction with Rx beam assumption. We, thus propose,

Study the two AI-based beam prediction solutions for both BM-Case1 and BM-Case2, i.e. enhanced beam pair prediction scheme and DL Tx beam prediction scheme, and considering specification impacts with generalization aspects, such as Set B construction, supported number of Tx/Rx beams, various number of antenna configurations, etc.

Issues on training/inference location

Another issue that can impact beam prediction solution is AI/ML training and inference location. It was agreed that AI/ML training is at either gNB side or UE side, while AI/ML inference can be at gNB side or UE side as well. Thus, there are 4 combinations of AI/ML training and inference location to be discussed:

- Alt.1. AI/ML model training and inference at NW side

- Alt.2. AI/ML model training and inference at UE side

- Alt.3. AI/ML model training at NW side, AI/ML model inference at UE side

- Alt.4. AI/ML model training at UE side, AI/ML model inference at gNB side

Alt.1 and Alt.2 is a one-side AI solution where this AI model is trained and inferenced at same side, while Alt.3 and Alt.4 needs model transfer in principle. Besides, this discussion is related with collaboration level discussion in 9.2.1. Alt.1 or Alt.2 may need the support of level-y, and Alt.3 or Alt.4 may need the support of level-z.

For Alt.4, if AI models in multiple accessed UEs should be transferred to gNB, the required memory storage in NW side seems unaccepted. We believe Alt.4 is not a suitable solution for further potential specification study.

For Alt.1 combined with above two prediction solutions, i.e. NW side AI/ML beam prediction with enhanced beam pair prediction and DL Tx beam prediction, it seems that all needed L1-RSRPs with related assistance information from UE side, such as Rx beam pointing angle, Rx beam width, expected Rx beam information etc., should be signaled to gNB that will increase beam report overhead dramatically.

In contrast, the overhead of beam report is generally smaller in UE-side AI/ML with enhanced beam pair prediction compared to network-side solution. As for the AI/ML model for beam prediction, only Top-k predicted L1-RSRP and beam indication information should be reported. However, extra specification impacts are needed, such as Tx sweeping beam pattern indication and Tx sweeping beam information indication, for improving beam prediction performance. As considering generalization performance in cell switching with different number of gNB Tx beams, and the output size of UE-side AI/ML with DL Tx beam prediction is fixed related to training stage, this UE side AI model with DL Tx beam prediction solution seems not applicable any more if accessed cell is switched, unless special consideration is given on the input/output design such as using expected information as elaborated in section 2.1. Thus, for Alt.2, although report overhead can be reduced, extra signaling from gNB should be indicated and beam prediction solution is limited with considering generalization/scalability aspect.

For Alt.3, the AI model can be a cell specific solution that it needs to be trained at Network side and then delivered to its accessed UE. Generalization for one model is a good study point, but it cannot address all the issues caused by NW channel environment change or configuration change. Hence Alt 3 is needed to make sure the overall performance is satisfactory. Although Alt.3 faces the AI/ML model representation format issue and model delivery overhead issue, the UL report overhead can be reduced as the model delivery is to be carried in DL, in comparison with Alt.1. Generally, UL resource is more limited than DL, esp. considering model delivery is likely to be higher layer DL signaling. Further, Alt 3 does not require training model in UE side, which is an energy saving solution to improve user experience.

As we analyzed above, assistance information, such as antenna configuration, Tx beam angle, etc., should be signaled to UE for Alt.2. However, such NW-side information can be sensitive in some cases. UE and NW may have mismatch on interpreting these parameters. If mismatch NW-side beam information is used, significant performance deterioration can be observed for AI based beam prediction scheme in Alt.2 which can be found in below tables.

Table 1: performance comparison between matched NW beam information and mismatch NW beam information for AI based beam pair prediction scheme in BM-Case1

| Simulation assumption | Ave. RSRP diff. [dB] | Accuracy for Top-1 [%] | Accuracy for Top-1 with 1dB margin [%] | Accuracy for Top-4/1 [%] |
| --- | --- | --- | --- | --- |
| Matched NW beam information | 1.91 | 56.68 | 68.10 | 86.03 |
| Mismatched NW beam information | 20.67 | 2.74 | 12.08 | 9.29 |

*Note: Results and more simulation assumptions can be found in our evaluation contribution [2] of section 4.2.1.1.

Table 2: performance comparison between matched NW beam information and mismatch NW beam information for AI based beam pair prediction scheme in BM-Case2

| Simulation assumption | Ave. RSRP diff. [dB] | Accuracy for Top-1 [%] | Accuracy for Top-4/1[%] |
| --- | --- | --- | --- |
| Matched NW beam information | 0.80 | 74.01 | 92.96 |
| Mismatched NW beam information | 12.58 | 9.17 | 45.17 |

*Note: Results and more simulation assumptions can be found in our evaluation contribution [2] of section 4.3.3.

Large performance loss can be found if using mismatch NW-side beam information in AI input from above two tables in both BM-Case1 and BM-Case2. By contrast, if an AI model inferenced at UE side is trained by network, a cell specific AI solution can be achieved with generalization consideration and infra vendors may not need to signal such information. Even though some NW-side beam information shall be signaled to UE for AI model performance improvement in Alt.3, NW can always use correct beam information for training as model is trained by NW. Thus, we strongly recommend Alt.3 should be studied with considering aspects on model generalization and NW information disclosure issues.

To sum up, we have following observations based on our analysis,

- Report overhead may increase dramatically but with less specification impacts for Alt. 1 with enhanced beam pair prediction solution and DL Tx beam prediction solution.

- Report overhead can be reduced to top-k L1-RSRP and its related Rx beam information, but assistance information including NW-side information, such as antenna configuration, Tx beam angle, etc., should be signaled to UE for Alt.2.

- Due to UE side model training, if mismatch NW-side beam information is signaled to UE, significant performance deterioration can be observed for AI based beam prediction scheme in Alt.2.

- Report overhead and UE energy/complexity is limited for Alt.3, but model transfer is needed.

- For Alt.3, a cell specific AI solution can be achieved with generalization consideration and infra vendor may not need to disclose NW-side information such as antenna configuration, Tx beam angle, etc.

It can be found that, except Alt.4, other alternatives may have its pros and cons. Thus, at current stage, we recommend other companies who want to deprioritize Alt.3 can further study its pros and cons compared to Alt.2.

For the sub use case BM-Case1 and BM-Case2, support to study Alt.3 for AI/ML model training and inference:

- Alt.3. AI/ML model training at NW side, AI/ML model inference at UE side

Set B selection scheme

In our evaluation contribution [2], it has been proved that only one pre-defined beam subset with fixed pattern included in AI input may show good performance in theory, but it lacks flexibility as in practical implementation, since a particular beam or beam pair may suffer performance loss due to unexpected channel variation like blockage, and may cause large interference.

Further, random pattern selection scheme, which allows multiple random patterns in training, can improve generalization performance as well as beam management related performance if compared to mismatched pattern with always using one pattern in training. However, Set B with random beam patterns selected from the full set still suffers tremendous performance deterioration due to huge number of combinations of selecting a target number of beams from total beam pairs.

Small performance loss can be observed from Set B with pre-configured patterns with assistance information in comparison with fixed pattern scheme, and it provides significant performance improvement compared to totally random pattern scheme. We, thus, propose,

Support to study specification impact on Set B with pre-configured beam patterns which can provide comparable gain to fixed scheme but with higher flexibility and better generalization performance. How to select pre-configured patterns can be FFS.

Necessity and feasibility of assistance information

We demonstrate in our evaluation contribution that AI input including L1-RSPR measured from fixed pattern with different assistance information, such as Tx beam ID/angle or Tx/Rx beam ID/angle, provides similar performance compared to fixed pattern without any assistance information.

However, Set B with different number of pre-configured best patterns achieves considerable performance improvement compared to the same Set B without any assistance information, and it can be observed that this pre-configured beam pattern scheme has potential to approach the performance of the best fixed pattern. Thus, we propose,

At least support Tx/Rx beam angle/ID information as assistance information for performance improvement for both BM-Case1 and BM-Case2. Other assistance information can be FFS.

- Further, we have shown that beam prediction performance degenerates significantly if using local beam ID as model input.  Companies may have concern that using angles as assistant information may disclose implementation details from vendors. Hence, we think assistant information needs to defined carefully to balance proprietary information protection and AI/ML performance. Take beam angle as an example. We think the input beam angle doesn’t need to be the exact beam angle used in gNB’s or UE’s beamforming implementation. Instead, it can be a mapped beam angle information where the mapping between the real angle and input information can be known only by the gNB or UE to perform beamforming.

- Even assistance information, such as Tx and Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beam-width, etc., is widely used for AI input in performance evaluation across companies, there are companies negative to disclose any proprietary/privacy information based on previous discussion. However, for AI model trained at UE side, using the above information is essential to ensure beam prediction performance for the AI model inference. Besides, there is no clear definition of proprietary/privacy information. As we discussed above for beam angle, the delivered information from NW can be a virtual parameter mapped from the real beam shape information (Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beam-width, etc.), where the mapping between the real beam shape and the virtual beam shape can be known only by NW. Then NW does not need to disclose any privacy information if it does not want to. How to ensure the beam prediction performance while trying to protect such proprietary/privacy information requires further study. Thus, we propose,

For the determination/selection of assistance information,

- The performance, model generalization and potential specification impacts should be considered.

- Study how to protect sensitive proprietary/privacy information and disclose beam specific related assistance information.

Proprietary protection mechanism shall be studied in order not to disclose proprietary information for a given assistance information. An example of such proprietary protection mechanism is shown in the following figure.

Figure 1: proprietary processing for model training and model inference at UE-side

In Figure 1, a proprietary processing module belongs to NW can be applied to map the real beam angles to proprietary processed beam angles or IDs. Take UE side model as an example. Beam resources with proprietary processed Tx beam angle information are transmitted to the UE, and then measurement RSRPs + corresponding proprietary processed Tx beam angles + corresponding Rx beam angles without proprietary processing can be used as AI input for a UE-side AI/ML model.

Therefore, the delivered information from NW can be a virtual Tx beam information mapped from the real Tx beam information, where the mapping between the real Tx beam information and the virtual Tx beam information can be known only by NW. Then NW does not need to disclose any privacy information if it does not want to. Such mapped information can still be useful for generalization performance at UE side. The key point is a same mapping function shall be maintained across training and validation.

In our evaluation contribution [2], slight performance deterioration can be observed from proprietary protection with mathematical function processing compared to beam prediction using beam angle directly for BM-Case1 and BM-Case2, if a same processing function is maintained for training and validation. We, thus, propose,

Support proprietary protection mechanism for proprietary/privacy information disclosing issue. Detailed proprietary protection mechanism can be FFS.

Suggest to use proprietary processed assistance information as model input to address performance deterioration and sensitive proprietary information disclosure issues in both BM-Case1 and BM-Case2, where a same mapping function is maintained for training and inference.

Input of BM-Case1 and BM-Case2

In RAN1#109-e meeting, following conclusions were made for AI/ML input,

Conclusion

Regarding the sub use case BM-Case1, further study the following alternatives for AI/ML input:

- Alt.1: Only L1-RSRP measurement based on Set B

- Alt.2: L1-RSRP measurement based on Set B and assistance information

  - FFS: Assistance information. The following were mentioned by companions in the discussion:  Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.), expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction), UE position information, UE direction information, Tx beam usage information, UE orientation information, etc.

    - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side.

- Alt.3: CIR based on Set B

- Alt.4: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID

- Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives

- Note2: All the inputs are “nominal” and only for discussion purpose.

Conclusion

Regarding the sub use case BM-Case2, further study the following alternatives of measurement results for AI/ML input (for each past measurement instance):

- Alt.1: Only L1-RSRP measurement based on Set B

- Alt 2: L1-RSRP measurement based on Set B and assistance information

  - FFS: Assistance information. The following were mentioned by companies in the discussion:, Tx and/or Rx beam angle, position information, UE direction information, positioning-related measurement (such as Multi-RTT), expected Tx and/or Rx beam/occasion for the prediction (e.g., expected Tx and/or Rx beam angle for the prediction, expected occasions of the prediction), Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight directions (azimuth and elevation), 3dB beamwidth, etc.) , increase ratio of L1-RSRP for best N beams, UE orientation information

    - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side.

- Alt.3: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID

- Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives

- Note2: All the inputs are “nominal” and only for discussion purpose.

- We demonstrate AI input including L1-RSPR measured from pre-configured beam patterns, corresponding Tx beam information, corresponding Rx beam information, and expected Tx and/or expected Rx beam information, can provide comparable performance but with higher flexibility and better generalization performance. We, thus, propose,

Regarding to BM-Case1 and BM-Case 2, at least prioritize following AI input information for further study on specification impact:

- L1-RSPR measurement based on Set B

- Corresponding DL Tx beam pointing angle/ID

- Corresponding DL Rx beam pointing angle/ID

- Expected Tx and/or expected Rx beam angle/ID

- Further discuss other information, such as Tx and/or Rx beam shape information, 3dB beam-width, etc.

Output of BM-Case1 and BM-Case2

In RAN1#110 meeting, the following agreement was made for AI/ML output,

Agreement

Regarding the sub use case BM-Case1 and BM-Case2, study the following alternatives for AI/ML output:

- Alt.1: Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams

  - E.g., N predicted beams can be the top-N predicted beams

- Alt.2: Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and other information

  - FFS: other information (e.g., probability for the beam to be the best beam, the associated confidence, beam application time/dwelling time, Predicted Beam failure)

  - E.g., N predicted beams can be the top-N predicted beams

- Alt.3: Tx and/or Rx Beam angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams

  - E.g., N predicted beams can be the top-N predicted beams

  - FFS: details of Beam angle(s)

- FFS: how to select the N DL Tx and/or Rx beams (e.g., L1-RSRP higher than a threshold, a sum probability of being the best beams higher than a threshold, RSRP corresponding to the expected Tx and/or Rx beam direction(s))

- Note1: It is up to companies to provide other alternative(s)

- Note2: Beam ID is only used for discussion purpose

- Note3: All the outputs are “nominal” and only for discussion purpose

- Note4: Values of N is up to each company.

- Note5: All of the outputs in the above alternatives may vary based on whether the AI/ML model inference is at UE side or gNB side.

- Note 6: The Top-N beam IDs might have been derived via post-processing of the ML-model output

The main difference between Alt.1 and Alt.3 is the result type of AI/ML model output, where beam ID or beam angle is used respectively. As we described above, for the case using local beam ID as model input, beam prediction performance degenerate significantly. If global beam ID is used, we believe beam ID and beam angle has the similar essential meaning which maintains a same understanding on beam ID and angle mapping across different AI model owners and users. Thus, from our perspective, it is indeed the same meaning for Alt.1 and Alt.3 with global beam ID used.

For Alt.2, beam ID and other information, such as probability for the beam to be the best beam, the associated confidence, beam application time/dwelling time, predicted beam failure, can be obtained from AI output instead of L1-RSRP information. As there are no top-k predicted L1-RSRPs can be directly acquired by gNB in this Alt, regardless of AI model inference location, additional beam sweeping resources for predicted beam RSRP confirmation are needed with beam management latency increased. However, how to use the other information and its benefit are not clear.

Another issue for output is how to produce the N output beams/beam pairs. As we elaborate above for model input and in section 4.2.2 of our EVM contribution [2], to make a trained AI model scalable to different number of Tx/Rx beams, it is beneficial to produce the N output beams based on expected beam information input to the AI model.

Support to prioritize following AI output for further study on specification impact:

- Tx and/or Rx Beam ID(s)/angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams.

        - The N predicted Tx/Rx beams can be produced according to the expected beam information input to the AI model

        - FFS: study global beam ID or local beam ID

        - FFS: study global beam information, e.g. global beam ID or beam angle, with minimum exposures of implementation details

Suggest to deprioritize Alt.2, i.e. Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and other information, for further study specification impact.

- Discussion on specification impact

In this section, we will analyze specification impact on aspects including model deployment with various collaboration levels, life cycle management, and training/inference procedure for beam management sub use cases.

  - Discussion on collaboration levels

The beam management AI/ML Model can be a one-sided model, which is either a UE-side model or a Network-side model. There are now two schemes for both case1 and case2 of beam management. One scheme is beam pair prediction, and the other is two-step (DL Tx beam) prediction. The model inference of former requires assistant information from the other side. For example, if model inference is conducted on UE-side, and beam angle information of BS-side helps improve prediction accuracy significantly. Hence, the recommended collaboration level of beam pair prediction scheme is at least level-y as presented in our companion contribution [3]. For two-step scheme, evaluation result shows that compared to using second best Rx beam, using best Rx beam achieves much higher Tx beam prediction accuracy. For instance, when model inference of Tx beam prediction is performed at the network, and assistant configuration at the UE, i.e. configuration of best Rx beam, helps enhance performance. Therefore, the recommended collaboration level of two-step scheme is also level-y. If model is trained at one side and model inference is performed at the other side, then level-z is required.

For case 1 and case 2 of beam management, both collaboration level-y, and collaboration level-z can be considered.

For life cycle management of beam management, three subcases are considered according the supportable model update levels.

- The simplest level is that model has been offline trained and both model parameter and structure cannot be changed. In this case, life cycle management procedure is denoted as subcase 1, which includes model activation, data collection for model inference, model inference, data collection for model monitoring, model monitoring and model deactivation.

- The second level is to support updating model parameter or structure w/o model transfer. Take beam pair prediction scheme as an example, if a UE moves from one cell to another, and assume that beam shape of target cell is different from source cell, and generalization of model is not good enough. It is better to perform model switching, besides subcase 1.

  - If all available models cannot provide satisfied performance, then new models are generated based on model training. Based on training is performed on the 3rd part server or model inference side, there are two subcategories.

    - If model training is performed at the third-party server, life cycle management includes training data collection, model switching and subcase 1.

    - If model is trained on the inference side, life cycle management consists of data collection for model training, model training, model validation, model testing and subcase 1.

- The third level is to support updating model parameter and/or structure by model transfer. This level corresponds to level z in [3].

  - Specifically, to perform model updating, model training is conducted on the other side in the system different from model inference side. Then the life cycle management includes data collection for model training, model training, model validation, model testing, model transfer and subcase 1.

  - It is worthy to note that in this level, the relevant part of model training can also be offline work without much specification impact, when multiple models are trained before deployment and stored in the other side of model inference end.  In this subcase, life cycle management includes model switching, model transfer and sub-case 1.

Take the following supportable model update choices as one aspect for defining model update levels of beam management.

- Choice 0: No model update during lifecycle management

- Choice 1: Updating model parameters w/o model transfer

- Choice 2: Updating model parameters with model transfer

- Study the lifecycle management signaling and procedures for each of the collaboration levels and model updating choices.

There are two LCM methods, model ID-based LCM and functionality-based LCM. The main difference is that functionality-based LCM does not have NW explicitly indicated model switch, whereas model switch is performed by UE autonomously. For functionality-based LCM in beam prediction use cases, if a UE moves from one cell to another, assuming that configurations like beam shape of target cell are different from source cell, and all available models cannot provide satisfied performance, new models need to be generated based on model training. Based on training is performed on the 3rd party server or model inference side, there are two subcategories.

  - If model training is performed at the third-party server, life cycle management includes training data collection and subcase 1.

  - If model is trained on the inference side, life cycle management consists of data collection for model training, model training, model validation, model testing and subcase 1.

However, model training at the 3rd party server suffers long latency, and model training on the inference side, e.g. UE, may be restricted by UE computation capability. A better approach is to train model at network side, with low latency and sufficient computation capability. However, this approach cannot be supported by functionality-based LCM, as it requires model transfer or delivery, and model ID based LCM is a better tool to realize this approach.

On the other hand, for model ID-based LCM, model switch can be performed to select specific model and flexibly matched to the new scenario, as NW has full knowledge of the scenario parameters. However, for functionality-based LCM, functionality update is similar to legacy software update, which do not always make functionality match well with a totally new scenario never been trained before.

Both model ID-based LCM and functionality-based LCM should be studied for beam management.

  - Potential specification impact under LCM procedure

In this section, we will analyze specification impacts on these 3 targets, i.e. model training, model inference, and model monitoring, with 3 beam prediction solutions which are enhanced beam pair prediction, DL Tx beam prediction and DL Rx beam prediction. Specifically, specification impact for DL Rx beam prediction for P3 also requires analysis if it is proven beneficial in further evaluation in Rel-18. Besides, as mentioned in section 2.2 regarding with training and inference location, at least 3 alternatives shall be studied, i.e. Alt.1: AI/ML model training and inference at NW side, Alt.2: AI/ML model training and inference at UE side, Alt.3: AI/ML model training at NW side and AI/ML model inference at UE side. We will first analyze beam prediction solution for different model deployment scenarios,

Beam prediction solution analysis for Alt.1:

As both model training and inference shall be at NW side for Alt.1, the output of AI based beam prediction solution should be related to the number of Tx beams only. Expected Rx beam information can be used as AI input in option 1 for enhanced beam pair prediction solution to improve generalization performance, and it may not be practical to implement DL Rx beam prediction in NW side, i.e. option 3. Thus, only option 1 with expected Rx beam information scheme and option 2 shall be discussed with Alt.1.

Beam prediction solution analysis for Alt.2:

AI model in Alt.2 shall be trained and inferenced at UE side. Considering generalization purpose, expected Tx beam information can be signaled to UE as AI model input for enhanced beam pair prediction solution and DL Tx beam prediction solution to predict all beam pairs or Tx beams by performing multiple prediction running cycles. Similar to current P3 stage, option 3 with DL Rx beam prediction can be realized in UE side for Rx beam prediction through implementation but with a few specification impacts.

Beam prediction solution analysis for Alt.3:

For Alt.3, as an AI model should be trained in network side and transferred to UE side for inference, in order to avoid too frequent model transfer and thus reduce signaling overhead, this AI model should at least adapt various number of Rx beams for different accessed UEs. Thus, enhanced beam pair prediction with expected Rx beam and DL Tx beam prediction is used which is same as Alt.1 training stage.

Therefore, we have following combinations (options) under each alternative for further study,

- Alt.1 (AI/ML model training and inference at NW side)

  - Option 1: Enhanced beam pair prediction with expected Rx beam(s)

  - Option 2: DL Tx beam prediction

  - Option 3: DL Rx beam prediction

- Alt.2 (AI/ML model training and inference at UE side)

  - Option 1: Enhanced beam pair prediction with expected Tx beam(s)

  - Option 2: DL Tx beam prediction with expected Tx beam(s)

  - Option 3: DL Rx beam prediction

- Alt.3 (AI/ML model training at NW side and AI/ML model inference at UE side)

  - Option 1: Enhanced beam pair prediction with expected Rx beam(s)

  - Option 2: DL Tx beam prediction

  - Option 3: DL Rx beam prediction

    - Data collection

In RAN1#110 meeting, the following agreement was made for studying data collection,

Agreement

For the data collection for AI/ML model training (if supported), study the following aspects as a starting point for potential necessary specification impact:

- Signaling/configuration/measurement/report for data collection, e.g., signaling aspects related to assistance information (if supported), Reference signals

- Content/type of the collected data

- Other aspect(s) is not precluded

      - Model training and inference at NW side

Figure 2: Data collection for enhanced beam pair prediction with Alt.1

For enhanced beam pair prediction in data collection procedure, where the AI model is deployed in NW side, the following steps are needed:

- Step1: A specific beam pair resource configuration should be signaled to UE

- Step2: UE receives beam pair resources for L1-RSRP measurement

- Step3: UE reports measurement results with/without relative Rx beam information to gNB, where the Rx beam information includes measured Rx beam information and/or expected Rx beam information

Based on the above steps, the following potential specification impacts can be identified:

- Enhanced resource configuration

  - Specific beam pair resource configuration, e.g. Rx beam pattern indication.

- Enhanced beam report

  - All measured L1-RSRP and/or relative Rx beam information (including measured Rx beam information and/or expected Rx beam information) should be reported to gNB

  - Considering report overhead reduction, such as reducing unnecessary L1-RSRPs in a report where the omitted L1-RSRPs may be smaller than a pre-defined threshold. Enhanced L1-RSRP quantization method (e.g., enhance the resolution of RSRP quantization) also needs to be considered as L1-RSRP quantization error may have large impact on training performance. Further, multi-resolution L1-RSRP quantization can be considered to balance the report overhead and L1-RSRP accuracy.

Figure 3: Data collection for DL Tx beam prediction with Alt.1

For DL Tx beam prediction in data collection procedure, where the AI model is deployed in NW side, the following steps are needed:

- Step1: A P3 + P2 resource configuration should be signaled to UE for improving AI prediction performance

- Step2: UE measures P3 beam sweeping resources to obtain the best Rx beam

- Step3: UE measures P2 beam sweeping resources based on the best Rx beam searched from step2

- Step4: UE only reports P2 measured results with/without relative best Rx beam information to gNB

Based on the above steps, the following potential specification impacts can be identified:

- Enhanced resource configuration

  - P3 + P2 resource configuration that Rx beam assumption of P2 resource measurement is the best Rx beam searched from P3 procedure.

- Enhanced beam report

  - Only P2 measured L1-RSRP and/or relative best Rx beam information should be reported to gNB

  - Considering report overhead reduction, such as reducing unnecessary L1-RSRP reporting where the omitted L1-RSRPs may be smaller than a pre-defined threshold. Enhanced L1-RSRP quantization method (e.g., enhance the resolution of RSRP quantization) also needs to be considered as L1-RSRP quantization error may have large impact on training performance. Further, multi-resolution L1-RSRP quantization can be considered to balance the report overhead and L1-RSRP accuracy.

Further, for beam (or RS indicator) report considering overhead reduction, UE can report all measurement results without RS indicator, or UE shall report valid RSRP results larger than noise level or a pre-defined RSRP level for improving data quality in data collection procedure. Therefore, UE can report valid RSRP results with the corresponding RS indicators if the number of invalid RSRP results is smaller than half of the number of beams or beam pairs in Set A. On the other hand, if the number of invalid RSRP results is larger than half of the number of beams or beam pairs in Set A, it saves more beam report overhead if UE just reports valid RSRP results and RS indicators corresponding to the beams or beam pairs in Set A with invalid RSRP results.

Thus, we have following proposals for Alt.1 in data collection procedure,

Regarding the data collection for AI/ML model training at NW side, study potential specification impact on resource configuration:

- Specific beam pair resource configuration

- Enhanced P3+P2 resource configuration that Rx beam assumption of P2 resource measurement is the best Rx beam searched from P3 procedure

Regarding the data collection for AI/ML model training at NW side, study potential specification impact on assistance information:

- Rx beam information as assistance information from UE to NW, including measured Rx beam information, expected Rx beam information, and best Rx beam information.

- Proprietary processed assistance information

Regarding the data collection for AI/ML model training at NW side, study potential specification impact on measurement report:

- UE measures the beams of Set A and reports M1 L1-RSRPs optionally with M2 RS indicators, where M1 and M2 can be larger than 4.

        - If M1 is equal to the number of beams or beam pairs in Set A (noted as X), corresponding RS indicators may be not needed.

        - If M1 is smaller than X/2, corresponding M2 RS indicators are needed

        - If M1 is smaller than X, but larger than X/2, RS indicators are needed for indicating M2 beams or beam pairs in Set A not included in the measurement report.

Regarding the data collection for AI/ML model training at NW side, study potential specification impact on report overhead reduction:

- Reducing unnecessary L1-RSRP report where the omitted L1-RSRPs may be lower than a pre-defined threshold

Regarding the data collection for AI/ML model training at NW side, study potential specification impact on quantization enhancement for RSRP quality improvement:

- High-precision L1-RSRP quantization

- Multi-resolution L1-RSRP quantization, e.g. high-resolution quantization for a group of best RSRPs and low-resolution quantization for others

      - Model training and inference at UE side

Figure 4: Data collection for enhanced beam pair prediction with Alt.2

For enhanced beam pair prediction in data collection procedure, where the AI model is deployed in UE side, the following steps are needed:

- Step1: UE may send a request for data collection purpose with P1 resources requirement

- Step2: A specific beam pair resource configuration with Tx beam indication (including measured Tx beam information and/or expected Tx beam information) should be signaled to UE

- Step3: UE receives beam pair resources for L1-RSRP measurement

Based on the above steps, the following potential specification impacts can be identified:

- Enhanced request signaling

  - UE may request beam pair sweeping resources for data collection purpose

  - UE may request the minimum number of beams in advance or with resource request

  - UE may request Tx beam information (for measured Tx beam and/or expected Tx beam as model input) that should be signaled from gNB

- Enhanced resource configuration

  - Specific beam pair resource configuration

  - Tx beam information (such as ID or angle for measured Tx beam and/or expected Tx beam as model input) may be signaled in resource configuration

Figure 5: Data collection for enhanced DL Tx beam prediction with Alt.2

For enhanced DL Tx beam prediction in data collection procedure, where the AI model is deployed in UE side, the following steps are needed:

- Step1: UE may send a request for data collection purpose with P3+P2 resources requirement

- Step2: A P3+P2 resource configuration with Tx beam information (such as ID or angle for model input, including measured Tx beam information and/or expected Tx beam information) should be signaled to UE

- Step3: UE measures P3 beam sweeping resources to obtain the best Rx beam

- Step4: UE measures P2 beam sweeping resources based on the best Rx beam searched from step3

Based on the above steps, the following potential specification impacts can be identified:

- Enhanced request signaling

  - UE may request P3+P2 beam sweeping resources for data collection purpose

  - UE may request the minimum number of beams in advance or with P2+P3 resource request

  - UE may request Tx beam information (such as ID or angle for model input, including measured Tx beam information and/or expected Tx beam information) that should be signaled from gNB

- Enhanced resource configuration

  - P3 + P2 resource configuration that Rx beam assumption of P2 resource measurement is the best Rx beam searched from P3 procedure.

  - Tx beam information (such as ID or angle for model input, including measured Tx beam information and/or expected Tx beam information) may be signaled in resource configuration

Figure 6: Data collection for DL Rx beam prediction with Alt.2

For DL Rx beam prediction in data collection procedure, where the AI model is deployed in UE side, the following steps are needed:

- Step1: UE may send a request for data collection purpose with P3 resources requirement

- Step2: A P3 resource configuration with Tx beam indication should be signaled to UE

- Step3: UE measures P3 beam sweeping resources

Based on the above steps, the following potential specification impacts can be identified:

- Enhanced request signaling

  - UE may request P3 beam sweeping resources for data collection purpose

  - UE may request the minimum number of repetitions for P3 procedure in advance or with resource request

  - UE may request Tx beam information that should be signaled from gNB

We, thus, propose,

Regarding the data collection for AI/ML model training at UE side, study potential specification impact on resource configuration:

- Specific beam pair resource configuration

- Enhanced P3+P2 resource configuration that Rx beam assumption of P2 resource measurement is the best Rx beam searched from P3 procedure

Regarding the data collection for AI/ML model training at UE side, study potential specification impact on assistance information:

- Tx beam information as assistance information from NW to UE

- Proprietary processed assistance information

Regarding the data collection for AI/ML model training at UE side, study the potential specification impact on request signaling:

- Resource request signaling for data collection from UE to NW

  - Beam pair resources request

  - P3 and/or P2 beam sweeping resources request

- Minimum resource number request for data collection from UE to NW

  - Minimum number of beams requested for model training w or w/o resource request signaling

  - Minimum number of repetitions requested for model training w or w/o resource request signaling

      - Model training at NW side and model inference at UE side

As an AI model should be trained in network side, and enhanced beam pair prediction with expected Rx beam and DL Tx beam prediction is used, which is same as Alt.1 training stage, Alt.3 has similar data collection procedure as well as potential specification impacts. Therefore, we propose,

For Alt.3. which is AI/ML model training at NW side and inference at UE side, it has similar data collection procedure and potential specification impacts as Alt.1, i.e. both model training and model inference at NW side, for enhanced beam pair prediction and DL Tx beam prediction scheme.

    - Model inference

In RAN1#110 meeting, the following agreement was made for studying model inference,

Agreement

In order to facilitate the AI/ML model inference, study the following aspects as a starting point:

- Enhanced or new configurations/UE reporting/UE measurement, e.g., Enhanced or new beam measurement and/or beam reporting

- Enhanced or new signaling for measurement configuration/triggering

- Signaling of assistance information (if applicable)

- Other aspect(s) is not precluded

In RAN1#110bis-e meeting, following agreements were made for further study,

Agreement

For BM-Case1 with a UE-side AI/ML model, study the potential specification impact of L1 signaling to report the following information of AI/ML model inference to NW

- The beam(s) that is based on the output of AI/ML model inference

- FFS: Predicted L1-RSRP corresponding to the beam(s)

- FFS: other information

Agreement

For BM-Case2 with a UE-side AI/ML model, study the potential specification impact of L1 signaling to report the following information of AI/ML model inference to NW

- The beam(s) of N future time instance(s) that is based on the output of AI/ML model inference

  - FFS: value of N

- FFS: Predicted L1-RSRP corresponding to the beam(s)

- Information about the timestamp corresponding the reported beam(s)

  - FFS: explicit or implicit

- FFS: other information

      - Model training and inference at NW side

Figure 7: Model inference procedure for enhanced beam pair prediction with Alt.1

For enhanced beam pair prediction in model inference procedure, where the AI model is deployed in NW side, the following steps are needed:

- Step1: UE may send expected Rx beam information, such as supported DL Rx beam angles, supported DL Rx beam scope, etc.

- Step2: Beam pair resource configuration from gNB with pre-configured patterns

- Step3: UE receives beam pair resources for L1-RSRP measurement

- Step4: UE report measured results with/without relative Rx beam information to gNB

Based on the above steps, the following potential specification impacts can be identified:

- New Signaling

  - Expected DL Rx beam information report, such as supported DL Rx beam angle or scope at UE side.

- Enhanced resource configuration

  - Specific beam pair resource configuration, e.g. Rx beam pattern indication

  - Resource configuration with multiple pre-configured patterns

- Enhanced beam report

  - All measured L1-RSRP and/or relative Rx beam information should be reported to gNB

  - Pattern-based beam report, e.g. report RSRPs for one of pre-configured patterns

  - Considering report overhead reduction, such as reducing unnecessary L1-RSRP in a report where the omitted L1-RSRPs may be lower than a pre-defined threshold. Further overhead reduction needs to be studied for BM Case 2 where higher beam report overhead is required to report beam information for multiple occasions. Enhanced L1-RSRP quantization method (e.g., enhance the resolution of RSRP quantization) also needs to be considered as L1-RSRP quantization error may have large impact on training performance. Further, multi-resolution L1-RSRP quantization can be considered to balance the report overhead and L1-RSRP accuracy.

- Enhanced TCI indication

  - Both Rel-15/16 TCI and Rel-17 unified TCI frameworks need to be considered.

Figure 8: Model inference procedure for DL Tx beam prediction with Alt.1

For DL Tx beam prediction in model inference procedure, where the AI model is deployed in NW side, the following steps are needed:

- Step1: A P3 + P2 resource configuration should be signaled to UE for improving AI prediction performance

- Step2: UE measures P3 beam sweeping resources to obtain the best Rx beam

- Step3: UE measures P2 beam sweeping resources based on the best Rx beam searched from step2

- Step4: UE only reports P2 measured results with/without relative best Rx beam information to gNB

Based on the above steps, the following potential specification impacts can be identified:

- Enhanced resource configuration

  - P3 + P2 resource configuration that Rx beam assumption of P2 resource measurement is the best Rx beam searched from P3 procedure.

  - Resource configuration with multiple pre-configured patterns

- Enhanced beam report

  - Only P2 measured L1-RSRP and/or relative best Rx beam information should be reported to gNB

  - Pattern-based beam report, e.g. report RSRPs for one of pre-configured patterns

  - Considering report overhead reduction, such as enhanced L1-RSRP quantization method, reducing unnecessary L1-RSRP reporting where the omitted L1-RSRPs may be lower than a pre-defined threshold. Further overhead reduction needs to be studied for BM Case 2 where higher beam report overhead is required to report beam information for multiple occasions.

- Enhanced TCI indication

  - Both Rel-15/16 TCI and Rel-17 unified TCI frameworks need to be considered.

Similar to data collection stage for report overhead reduction issue, UE can report all measurement results without RS indicator, or UE shall report valid RSRP results larger than noise level or a pre-defined RSRP level for improving data quality in data collection procedure. Thus, we have following proposals for Alt.1 in model inference procedure.

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study potential specification impact on resource configuration for AI/ML model inference:

- Specific beam pair resource configuration

- Enhanced P3+P2 resource configuration that Rx beam assumption of P2 resource measurement is the best Rx beam searched from P3 procedure

- Resource configuration with multiple pre-configured patterns

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study potential specification impact on assistance information for AI/ML model inference:

- Rx beam information as assistance information from UE to NW, including measured Rx beam information, expected Rx beam information, and best Rx beam information.

- Proprietary processed assistance information

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study potential specification impact on measurement report for AI/ML model inference:

- UE measures the beams of Set B and reports M1 L1-RSRPs optionally with M2 RS indicators

        - If M1 is equal to the number of beams or beam pairs in Set B (noted as X), corresponding RS indicators may be not needed.

        - If M1 is smaller than X/2, corresponding M2 RS indicators are needed

        - If M1 is smaller than X, but larger than X/2, RS indicators are needed for indicating M2 beams or beam pairs in Set B not included in the measurement report.

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study potential specification impact on report overhead reduction for AI/ML model inference:

- Reducing unnecessary L1-RSRP report where the omitted L1-RSRPs may be lower than a pre-defined threshold

- Pattern-based beam report if beam resource configuration with multiple pre-configured patterns is supported

- Study how to further reduce report overhead of time domain beam prediction for measurement results of multiple occasions.

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study potential specification impact on quantization enhancement for RSRP quality improvement for AI/ML model inference:

- High-precision L1-RSRP quantization

- Multi-resolution L1-RSRP quantization, e.g. high-resolution quantization for a group of best RSRPs and low-resolution quantization for others.

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study potential specification impact on TCI indication for AI/ML model inference:

- Enhanced TCI indication based on both Rel-15/16 and Rel-17 unified TCI frameworks

      - Model training and inference at UE side

Figure 9: Model inference procedure for enhanced beam pair prediction with Alt.2

For enhanced beam pair prediction in model inference procedure, where the AI model is deployed in UE side, the following steps are needed:

- Step1: Expected Tx beam information should be signaled to UE in advance for AI model cycling

- Step2: gNB should be configured a specific beam pair resource configuration with Tx beam indication

- Step3: UE receives beam pair resources for L1-RSRP measurement

- Step4: UE report predicted L1-RSRP and relative Tx/Rx beam information

Based on the above steps, the following potential specification impacts can be identified:

- New Signaling

  - Expected DL Tx beam information indication, such as DL Tx beam angles or angle scopes.

  - Renew beam pattern request from UE if bad L1-RSRPs are measured from previous beam pattern

- Enhanced resource configuration

  - Specific beam pair resource configuration

  - Tx beam information (such as ID or angle) may be signaled with resource configuration

- Enhanced beam report

  - Predicted L1-RSRP and relative Tx/Rx beam information should be reported to gNB. Beam report enhancements including overhead reduction mentioned in previous subsections need to studied here as well.

  - A fallback beam report including only measured results, rather than predicted results, as well as non-predicted L1-RSRP indication if all measured results may not be suitable for AI based beam prediction.

Figure 10: Model inference procedure for enhanced DL Tx beam prediction with Alt.2

For enhanced DL Tx beam prediction in model inference procedure, where the AI model is deployed in UE side, the following steps are needed:

- Step1: Expected Tx beam information should be signaled to UE in advance for AI model cycling

- Step2: gNB should be configured a P3+P2 resource configuration with Tx beam indication

- Step3: UE measures P3 beam sweeping resources to obtain the best Rx beam

- Step4: UE measures P2 beam sweeping resources based on the best Rx beam searched from step3

- Step5: UE report predicted L1-RSRP and relative Tx beam information

Based on the above steps, the following potential specification impacts can be identified:

- New Signaling

  - Expected DL Tx beam information indication, such as DL Tx beam angles or angle scopes.

  - Renew beam pattern request if bad L1-RSRPs are measured from previous beam pattern

- Enhanced resource configuration

  - P3 + P2 resource configuration that Rx beam assumption of P2 resource measurement is the best Rx beam searched from P3 procedure.

  - Tx beam information (such as ID or angle) may be signaled with resource configuration

- Enhanced beam report

  - Predicted L1-RSRP and relative Tx beam information should be reported to gNB. Beam report enhancements including overhead reduction mentioned in previous subsections need to studied here as well.

  - A fallback beam report including only measured results, rather than predicted results, as well as non-predicted L1-RSRP indication if all measured results may not suitable use for AI based beam prediction

Figure 11: Model inference procedure for DL Rx beam prediction with Alt.2

For DL Rx beam prediction in model inference procedure, where the AI model is deployed in UE side, the following steps are needed:

- Step1: UE may send a request or capability report for model inference purpose with P3 resources requirement

- Step2: A P3 resource configuration with Tx beam indication should be signaled to UE

- Step3: UE measures P3 beam sweeping resources

Based on the above steps, the following potential specification impacts can be identified:

- Enhanced resource configuration

  - UE may report minimum number of repetitions for model inference before P3 resource configuration

  - Tx beam information may be signaled to UE

Therefore, we have following proposals for Alt.2 in model inference procedure,

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact on resource configuration for AI/ML model inference:

- Specific beam pair resource configuration

- Enhanced P3+P2 resource configuration that Rx beam assumption of P2 resource measurement is the best Rx beam searched from P3 procedure

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact on beam resource request for AI/ML model inference:

- Renew beam pattern request w or w/o beam pattern suggestion from UE to NW

- Minimum resource number request from UE to NW

  - Minimum number of requested beams

  - Minimum number of requested repetitions

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact on assistance information for AI/ML model inference:

- Tx beam information as assistance information from NW to UE

- Proprietary processed assistance information

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact on beam report for AI/ML model inference:

- Predicted L1-RSRP report and study how to report predicted beam indicator

- Fallback beam report to indicate invalid measured results for AI/ML based beam prediction

- Study how to further reduce report overhead of time domain beam prediction for predicted results of multiple occasions.

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact on TCI indication for AI/ML model inference:

- Enhanced TCI indication based on both Rel-15/16 and Rel-17 unified TCI frameworks

      - Model training at NW side and model inference at UE side

As same inference location for Alt.1 and Alt.3, we will focus on specification impacts on model transfer related procedure.

Figure 12: Model inference procedure for enhanced beam pair prediction and DL Tx beam prediction with Alt.3

gNB may transfer the trained model which can adapt different capabilities of accessed UEs, and then the UE may activate the model based on the model configuration autonomously or with explicit activation indication from RAN node after UE model confirmation signaling. Thus, the UE performs L1-RSRP measurement and following model inference procedure.

In model inference procedure, Alt.3, i.e. model training at NW side and model inference at UE side, with enhanced beam pair prediction and DL Tx beam prediction scheme has similar specification impacts as an AI model trained and inferenced at UE side.

Study signaling aspects enhancement related to the procedure of model transfer, model registration and model activation, for the case with AI/ML model training at NW side and AI/ML model inference at UE side.

    - Model monitoring

In RAN1#110bis-e meeting, following agreements were made for studying model monitoring,

Agreement

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection:

- Atl1. UE-side Model monitoring

  - UE monitors the performance metric(s)

  - UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation

- Atl2. NW-side Model monitoring

  - NW monitors the performance metric(s)

  - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

- Alt3. Hybrid model monitoring

  - UE monitors the performance metric(s)

  - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

Agreement

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study the NW-side model monitoring:

- NW monitors the performance metric(s) and makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

Agreement

Regarding NW-side model monitoring for a network-side AI/ML model of BM-Case1 and BM-Case2, study the potential specification impacts from the following aspects

- Beam measurement and report for model monitoring

- Note: This may or may not have specification impact.

In RAN1#111 meeting, following agreement was made for further study,

Agreement

Regarding NW-side model monitoring for a network-side AI/ML model of BM-Case1 and BM-Case2, study the necessity and the potential specification impacts from the following aspects:

- UE reporting of beam measurement(s) based on a set of beams indicated by gNB

- Signaling, e.g., RRC-based, L1-based

- Note: Performance and UE complexity, power consumption should be considered

      - Monitoring procedure with a network-side AI/ML model

Compared with the current agreement mentioned above, a more detailed categorization with two possible monitoring procedures with a NW-side AI/ML model of BM-Case1 and BM-Case2 can be identified as below,

- Atl1. NW-side model monitoring

  - Step 1: UE performs resource measurement and reports corresponding measurement results including set B results and set A label data

  - Step 2: NW performs beam prediction and predicted result comparison with label data to obtain performance metric(s)

  - Step 3: NW makes decision(s) of model selection/activation/deactivation/switching/fallback operation

- Atl2. Hybrid model monitoring

  - Step 1: UE performs resource measurement and reports partial measurement results (set B) used for beam prediction

  - Step 2: NW performs beam prediction based on set B results and indicates inference result (e.g., top-N predicted results) to UE

  - Step 3: UE performs predicted result comparison with label data to obtain performance metric(s) and reports monitoring result(s) to gNB

  - Step 4: NW makes decision(s) of model selection/activation/deactivation/switching/fallback operation

For Alt1., UE may need to report all measurement results to gNB, including beams in Set A and Set B, which brings significant report overhead. On the other hand, for Alt2., UE may only report partial measurement results used for beam prediction. Then, UE shall obtain performance metric(s) and feedback monitoring results to gNB after receiving top-N predicted results from gNB.

Based on the above, for BM-Case1 and BM-Case2 with a network-side AI/ML model, we propose,

For BM-Case1 and BM-Case2 with a NW-side AI/ML model, study the following alternatives for model monitoring:

- Atl1. NW-side model monitoring

  - UE performs resource measurement and reports corresponding measurement results including set B results and set A label data

  - NW performs beam prediction and predicted results comparison with label data to obtain performance metric(s)

  - NW makes decision(s) of model selection/activation/deactivation/switching/ fallback operation

- Atl2. Hybrid model monitoring

  - UE performs resource measurement and reports set B results used for NW-side beam prediction

  - NW performs beam prediction based on set B results and indicates inference result (e.g., top-N predicted results) to UE

  - UE performs predicted result comparison with label data to obtain performance metric(s) and reports monitoring result(s) to gNB

  - NW makes decision(s) of model selection/activation/deactivation/switching/ fallback operation

To improve readability, NW-side model monitoring procedure (Alt 1) is used as an example in the following analysis with a network side AI/ML model for further specification impact study. Note that the measurement resource configuration part is common for Alt 1 and Alt 2. The different part is what to report as monitoring report, and further, Alt 2 requires extra signaling from NW indicating inference result.

Figure 13: Model monitoring procedure for enhanced beam pair prediction with NW-side AI/ML model

For enhanced beam pair prediction in model monitoring procedure, where the AI model is operated in NW side, the following steps are needed:

- Step1: A specific beam pair resource configuration should be signaled to UE

- Step2: UE receives beam pair resources for L1-RSRP measurement

- Step3: UE report measured results with/without relative Rx beam information to gNB as labels

- Step4: gNB can monitor model performance

Based on the above steps, the following potential specification impacts can be identified:

- Enhanced resource configuration

  - Specific beam pair resource configuration, e.g. Rx beam pattern indication.

- Enhanced beam report

  - All measured L1-RSRP and/or relative Rx beam information should be reported to gNB as labels

  - Considering report overhead reduction, such as reducing unnecessary L1-RSRP in a report where the omitted L1-RSRP may be lower than a pre-defined threshold. Enhanced L1-RSRP quantization method (e.g., enhance the resolution of RSRP quantization) also needs to be considered as L1-RSRP quantization error may have large impact on training performance. Further, multi-resolution L1-RSRP quantization can be considered to balance the report overhead and L1-RSRP accuracy.

Figure 14: Model monitoring procedure for DL Tx beam prediction with NW-side AI/ML model

For DL Tx beam prediction in model monitoring procedure, where the AI model is operated in NW side, the following steps are needed:

- Step1: A P3 + P2 resource configuration should be signaled to UE for improving AI prediction performance

- Step2: UE measures P3 beam sweeping resources to obtain the best Rx beam

- Step3: UE measures P2 beam sweeping resources based on the best Rx beam searched from step2

- Step4: UE only reports P2 measured results with/without relative best Rx beam information to gNB as labels

- Step5: gNB can monitor model performance

Based on the above steps, the following potential specification impacts can be identified:

- Enhanced resource configuration

  - P3 + P2 resource configuration that Rx beam assumption of P2 resource measurement is the best Rx beam searched from P3 procedure.

- Enhanced beam report

  - Only P2 measured L1-RSRP and/or relative best Rx beam information should be reported to gNB as labels

  - Considering report overhead reduction, such as reducing unnecessary L1-RSRP reporting where the omitted L1-RSRP may be lower than a pre-defined threshold. Enhanced L1-RSRP quantization method (e.g., enhance the resolution of RSRP quantization) also needs to be considered as L1-RSRP quantization error may have large impact on training performance. Further, multi-resolution L1-RSRP quantization can be considered to balance the report overhead and L1-RSRP accuracy.

Thus, we have following proposals in model monitoring procedure,

For BM-Case1 and BM-Case2 with a NW-side AI/ML model, study the potential specification impact on resource configuration for model monitoring:

- Specific beam pair resource configuration

- P3+P2 resource configuration that Rx beam assumption of P2 resource measurement is the best Rx beam searched from P3 procedure

For BM-Case1 and BM-Case2 with a NW-side AI/ML model, study the potential specification impact on assistance information for model monitoring:

- Rx beam information as assistance information from UE to NW, including measured Rx beam information, expected Rx beam information, and best Rx beam information.

- Proprietary processed assistance information

For BM-Case1 and BM-Case2 with a NW-side AI/ML model, study the potential specification impact on report overhead reduction for model monitoring:

- Reducing unnecessary L1-RSRP report where the omitted L1-RSRPs may be lower than a pre-defined threshold

For BM-Case1 and BM-Case2 with a NW-side AI/ML model, study potential specification impact on quantization enhancement for model monitoring:

- High-precision L1-RSRP quantization

- Multi-resolution L1-RSRP quantization, e.g. high-resolution quantization for a group of best RSRPs and low-resolution quantization for others.

      - Monitoring procedure with a UE-side AI/ML model

Regarding model monitoring for a UE-side AI/ML model of BM-Case1 and BM-Case2, 3 monitoring procedures were discussed,

- Alt1. UE-side model monitoring, and UE monitors the performance metric(s)

- Alt2. NW-side model monitoring, and NW monitors the performance metric(s)

- Alt3. Hybrid model monitoring, and UE monitors the performance metric(s)

From our perspective, the word “monitor(s)” is unclear as it may include label data measurement and acquisition, set B measurement and beam prediction, and predicted result comparison with label data. Thus, for BM-Case1 and BM-Case2 with a UE side AI/ML model, we propose,

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection:

- Atl1. UE-side model monitoring

  - UE performs label data measurement, set B measurement and beam prediction and predicted result comparison with label data to obtain performance metric(s)

  - UE makes decision(s) of model selection/activation/deactivation/switching/

- fallback operation

- Atl2. NW-side model monitoring

  - UE performs label data measurement and set B measurement and beam prediction

  - NW performs predicted result comparison with label data to obtain performance metric(s)

  - NW makes decision(s) of model selection/activation/deactivation/switching/ fallback operation

- Atl3. Hybrid model monitoring

  - UE performs label data measurement, set B measurement and beam prediction and predicted result comparison with label data to obtain performance metric(s)

  - NW makes decision(s) of model selection/activation/deactivation/switching/ fallback operation

Among the above three alternatives, the common part is measurement resource configuration, and the major difference is monitoring report.

- No monitoring results report is required for Alt 1. It can be used for functionality-based LCM and model ID based LCM. For Model ID based LCM, UE may need to send a model switch recommendation to NW.

- UE reports label data as the monitoring report to NW in Alt 2. It can be used for functionality-based LCM and model ID based LCM.

- UE reports performance metrics as monitoring report to NW in Alt 3. It can be used for functionality-based LCM and model ID based LCM.

A more detailed analysis on the monitoring procedure is given below.

Figure 15: Model monitoring procedure for enhanced beam pair prediction with UE-side AI/ML model

For enhanced beam pair prediction in model monitoring procedure, where the AI model is operated in UE side, the following steps are needed:

- Step1: UE may send monitoring request information with P1 resources requirement including the number of requested labels, such request can also be triggered by certain events

- Step2: A specific beam pair resource configuration with Tx beam indication should be signaled to UE

- Step3: UE receives beam pair resources for L1-RSRP measurement

- Step4 (for Alt 2 and Alt 3): UE may report monitoring results to gNB

- Step5 (for Alt 2 and Alt 3): NW makes decision(s) of model selection/activation/ deactivation/switching/fallback operation

Based on the above steps, the following potential specification impacts can be identified:

- Enhanced request signaling

  - UE may request beam pair sweeping resources for model monitoring purpose including the number of requested labels, and potentially some associated triggering events to be defined

  - UE may request the minimum number of beams in advance or with resource request

  - UE may request Tx beam information that should be signaled from gNB

- Enhanced resource configuration

  - Specific beam pair resource configuration

  - Tx beam information (such as ID or angle) may be signaled with resource configuration

- New signaling

  - Monitoring result report, e.g., label data report or performance metric report

  - Monitoring indication from network for further model management decisions

Figure 16: Model monitoring procedure for DL Tx beam prediction with UE-side AI/ML model

For enhanced DL Tx beam prediction in model monitoring procedure, where the AI model is operated in UE side, the following steps are needed:

- Step1: UE may send a request for model monitoring purpose with P3+P2 resources requirement including the number of requested labels, such request can also be triggered by certain events

- Step2: A P3+P2 resource configuration with Tx beam indication should be signaled to UE

- Step3: UE measures P3 beam sweeping resources to obtain the best Rx beam

- Step4: UE measures P2 beam sweeping resources based on the best Rx beam searched from step3

- Step5 (for Alt 2 and Alt 3): UE may report monitoring results to gNB

- Step6 (for Alt 2 and Alt 3): NW makes decision(s) of model selection/activation/ deactivation/switching/fallback operation

Based on the above steps, the following potential specification impacts can be identified:

- Enhanced request signaling

  - UE may request P3+P2 beam sweeping resources for model monitoring purpose including the number of requested labels, and potentially some associated triggering events to be defined

  - UE may request the minimum number of beams in advance or with P2+P3 resource request

  - UE may request Tx beam information that should be signaled from gNB

- Enhanced resource configuration

  - P3 + P2 resource configuration that Rx beam assumption of P2 resource measurement is the best Rx beam searched from P3 procedure.

  - Tx beam information (such as ID or angle) may be signaled with resource configuration

- New signaling

  - Monitoring result report, e.g., label data report or performance metric report

  - Monitoring indication from network for further model decision

Thus, we have following propose for model monitoring,

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the potential specification impact on resource configuration for model monitoring:

- Specific beam pair resource configuration

- P3+P2 resource configuration that Rx beam assumption of P2 resource measurement is the best Rx beam searched from P3 procedure

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the potential specification impact on assistance information for model monitoring:

- Tx beam information as assistance information from NW to UE

- Proprietary processed assistance information

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the potential specification impact on request signaling for model monitoring:

- Resource request signaling for data collection from UE to NW

  - Beam pair resources request for model monitoring purpose including the number of requested labels, and potentially some associated triggering events to be defined

  - P3+P2 beam sweeping resources request for model monitoring purpose including the number of requested labels, and potentially some associated triggering events to be defined

- Minimum resource number request for data collection from UE to NW

  - Minimum number of requested beams for model monitoring w or w/o resource request signaling

  - Minimum number of requested repetitions for model monitoring w or w/o resource request signaling

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the potential specification impact on monitoring report for model monitoring:

- Monitoring result report from UE to NW, including label data report or performance metric report

- Conclusions

In this contribution, we discuss some issues on AI/ML for beam management and have the following observations:

- Report overhead may increase dramatically but with less specification impacts for Alt. 1 with enhanced beam pair prediction solution and DL Tx beam prediction solution.

- Report overhead can be reduced to top-k L1-RSRP and its related Rx beam information, but assistance information including NW-side information, such as antenna configuration, Tx beam angle, etc., should be signaled to UE for Alt.2.

- Due to UE side model training, if mismatch NW-side beam information is signaled to UE, significant performance deterioration can be observed for AI based beam prediction scheme in Alt.2.

- Report overhead and UE energy/complexity is limited for Alt.3, but model transfer is needed.

- For Alt.3, a cell specific AI solution can be achieved with generalization consideration and infra vendor may not need to disclose NW-side information such as antenna configuration, Tx beam angle, etc.

and proposals:

- Study the two AI-based beam prediction solutions for both BM-Case1 and BM-Case2, i.e. enhanced beam pair prediction scheme and DL Tx beam prediction scheme, and considering specification impacts with generalization aspects, such as Set B construction, supported number of Tx/Rx beams, various number of antenna configurations, etc.

For the sub use case BM-Case1 and BM-Case2, support to study Alt.3 for AI/ML model training and inference:

- Alt.3. AI/ML model training at NW side, AI/ML model inference at UE side

Support to study specification impact on Set B with pre-configured beam patterns which can provide comparable gain to fixed scheme but with higher flexibility and better generalization performance. How to select pre-configured patterns can be FFS.

At least support Tx/Rx beam angle/ID information as assistance information for performance improvement for both BM-Case1 and BM-Case2. Other assistance information can be FFS.

For the determination/selection of assistance information,

- The performance, model generalization and potential specification impacts should be considered.

- Study how to protect sensitive proprietary/privacy information and disclose beam specific related assistance information.

Support proprietary protection mechanism for proprietary/privacy information disclosing issue. Detailed proprietary protection mechanism can be FFS.

Suggest to use proprietary processed assistance information as model input to address performance deterioration and sensitive proprietary information disclosure issues in both BM-Case1 and BM-Case2, where a same mapping function is maintained for training and inference.

Regarding to BM-Case1 and BM-Case 2, at least prioritize following AI input information for further study on specification impact:

- L1-RSPR measurement based on Set B

- Corresponding DL Tx beam pointing angle/ID

- Corresponding DL Rx beam pointing angle/ID

- Expected Tx and/or expected Rx beam angle/ID

- Further discuss other information, such as Tx and/or Rx beam shape information, 3dB beam-width, etc.

Support to prioritize following AI output for further study on specification impact:

- Tx and/or Rx Beam ID(s)/angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams.

        - The N predicted Tx/Rx beams can be produced according to the expected beam information input to the AI model

        - FFS: study global beam ID or local beam ID

        - FFS: study global beam information, e.g. global beam ID or beam angle, with minimum exposures of implementation details

Suggest to deprioritize Alt.2, i.e. Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and other information, for further study specification impact.

For case 1 and case 2 of beam management, both collaboration level-y, and collaboration level-z can be considered.

Take the following supportable model update choices as one aspect for defining model update levels of beam management.

- Choice 0: No model update during lifecycle management

- Choice 1: Updating model parameters w/o model transfer

- Choice 2: Updating model parameters with model transfer

- Study the lifecycle management signaling and procedures for each of the collaboration levels and model updating choices.

Both model ID-based LCM and functionality-based LCM should be studied for beam management.

Regarding the data collection for AI/ML model training at NW side, study potential specification impact on resource configuration:

- Specific beam pair resource configuration

- Enhanced P3+P2 resource configuration that Rx beam assumption of P2 resource measurement is the best Rx beam searched from P3 procedure

Regarding the data collection for AI/ML model training at NW side, study potential specification impact on assistance information:

- Rx beam information as assistance information from UE to NW, including measured Rx beam information, expected Rx beam information, and best Rx beam information.

- Proprietary processed assistance information

Regarding the data collection for AI/ML model training at NW side, study potential specification impact on measurement report:

- UE measures the beams of Set A and reports M1 L1-RSRPs optionally with M2 RS indicators, where M1 and M2 can be larger than 4.

        - If M1 is equal to the number of beams or beam pairs in Set A (noted as X), corresponding RS indicators may be not needed.

        - If M1 is smaller than X/2, corresponding M2 RS indicators are needed

        - If M1 is smaller than X, but larger than X/2, RS indicators are needed for indicating M2 beams or beam pairs in Set A not included in the measurement report.

Regarding the data collection for AI/ML model training at NW side, study potential specification impact on report overhead reduction:

- Reducing unnecessary L1-RSRP report where the omitted L1-RSRPs may be lower than a pre-defined threshold

Regarding the data collection for AI/ML model training at NW side, study potential specification impact on quantization enhancement for RSRP quality improvement:

- High-precision L1-RSRP quantization

- Multi-resolution L1-RSRP quantization, e.g. high-resolution quantization for a group of best RSRPs and low-resolution quantization for others

Regarding the data collection for AI/ML model training at UE side, study potential specification impact on resource configuration:

- Specific beam pair resource configuration

- Enhanced P3+P2 resource configuration that Rx beam assumption of P2 resource measurement is the best Rx beam searched from P3 procedure

Regarding the data collection for AI/ML model training at UE side, study potential specification impact on assistance information:

- Tx beam information as assistance information from NW to UE

- Proprietary processed assistance information

Regarding the data collection for AI/ML model training at UE side, study the potential specification impact on request signaling:

- Resource request signaling for data collection from UE to NW

  - Beam pair resources request

  - P3 and/or P2 beam sweeping resources request

- Minimum resource number request for data collection from UE to NW

  - Minimum number of beams requested for model training w or w/o resource request signaling

  - Minimum number of repetitions requested for model training w or w/o resource request signaling

For Alt.3. which is AI/ML model training at NW side and inference at UE side, it has similar data collection procedure and potential specification impacts as Alt.1, i.e. both model training and model inference at NW side, for enhanced beam pair prediction and DL Tx beam prediction scheme.

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study potential specification impact on resource configuration for AI/ML model inference:

- Specific beam pair resource configuration

- Enhanced P3+P2 resource configuration that Rx beam assumption of P2 resource measurement is the best Rx beam searched from P3 procedure

- Resource configuration with multiple pre-configured patterns

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study potential specification impact on assistance information for AI/ML model inference:

- Rx beam information as assistance information from UE to NW, including measured Rx beam information, expected Rx beam information, and best Rx beam information.

- Proprietary processed assistance information

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study potential specification impact on measurement report for AI/ML model inference:

- UE measures the beams of Set B and reports M1 L1-RSRPs optionally with M2 RS indicators

        - If M1 is equal to the number of beams or beam pairs in Set B (noted as X), corresponding RS indicators may be not needed.

        - If M1 is smaller than X/2, corresponding M2 RS indicators are needed

        - If M1 is smaller than X, but larger than X/2, RS indicators are needed for indicating M2 beams or beam pairs in Set B not included in the measurement report.

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study potential specification impact on report overhead reduction for AI/ML model inference:

- Reducing unnecessary L1-RSRP report where the omitted L1-RSRPs may be lower than a pre-defined threshold

- Pattern-based beam report if beam resource configuration with multiple pre-configured patterns is supported

- Study how to further reduce report overhead of time domain beam prediction for measurement results of multiple occasions.

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study potential specification impact on quantization enhancement for RSRP quality improvement for AI/ML model inference:

- High-precision L1-RSRP quantization

- Multi-resolution L1-RSRP quantization, e.g. high-resolution quantization for a group of best RSRPs and low-resolution quantization for others.

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study potential specification impact on TCI indication for AI/ML model inference:

- Enhanced TCI indication based on both Rel-15/16 and Rel-17 unified TCI frameworks

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact on resource configuration for AI/ML model inference:

- Specific beam pair resource configuration

- Enhanced P3+P2 resource configuration that Rx beam assumption of P2 resource measurement is the best Rx beam searched from P3 procedure

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact on beam resource request for AI/ML model inference:

- Renew beam pattern request w or w/o beam pattern suggestion from UE to NW

- Minimum resource number request from UE to NW

  - Minimum number of requested beams

  - Minimum number of requested repetitions

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact on assistance information for AI/ML model inference:

- Tx beam information as assistance information from NW to UE

- Proprietary processed assistance information

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact on beam report for AI/ML model inference:

- Predicted L1-RSRP report and study how to report predicted beam indicator

- Fallback beam report to indicate invalid measured results for AI/ML based beam prediction

- Study how to further reduce report overhead of time domain beam prediction for predicted results of multiple occasions.

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact on TCI indication for AI/ML model inference:

- Enhanced TCI indication based on both Rel-15/16 and Rel-17 unified TCI frameworks

In model inference procedure, Alt.3, i.e. model training at NW side and model inference at UE side, with enhanced beam pair prediction and DL Tx beam prediction scheme has similar specification impacts as an AI model trained and inferenced at UE side.

Study signaling aspects enhancement related to the procedure of model transfer, model registration and model activation, for the case with AI/ML model training at NW side and AI/ML model inference at UE side.

For BM-Case1 and BM-Case2 with a NW-side AI/ML model, study the following alternatives for model monitoring:

- Atl1. NW-side model monitoring

  - UE performs resource measurement and reports corresponding measurement results including set B results and set A label data

  - NW performs beam prediction and predicted results comparison with label data to obtain performance metric(s)

  - NW makes decision(s) of model selection/activation/deactivation/switching/ fallback operation

- Atl2. Hybrid model monitoring

  - UE performs resource measurement and reports set B results used for NW-side beam prediction

  - NW performs beam prediction based on set B results and indicates inference result (e.g., top-N predicted results) to UE

  - UE performs predicted result comparison with label data to obtain performance metric(s) and reports monitoring result(s) to gNB

  - NW makes decision(s) of model selection/activation/deactivation/switching/ fallback operation

For BM-Case1 and BM-Case2 with a NW-side AI/ML model, study the potential specification impact on resource configuration for model monitoring:

- Specific beam pair resource configuration

- P3+P2 resource configuration that Rx beam assumption of P2 resource measurement is the best Rx beam searched from P3 procedure

For BM-Case1 and BM-Case2 with a NW-side AI/ML model, study the potential specification impact on assistance information for model monitoring:

- Rx beam information as assistance information from UE to NW, including measured Rx beam information, expected Rx beam information, and best Rx beam information.

- Proprietary processed assistance information

For BM-Case1 and BM-Case2 with a NW-side AI/ML model, study the potential specification impact on report overhead reduction for model monitoring:

- Reducing unnecessary L1-RSRP report where the omitted L1-RSRPs may be lower than a pre-defined threshold

For BM-Case1 and BM-Case2 with a NW-side AI/ML model, study potential specification impact on quantization enhancement for model monitoring:

- High-precision L1-RSRP quantization

- Multi-resolution L1-RSRP quantization, e.g. high-resolution quantization for a group of best RSRPs and low-resolution quantization for others.

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection:

- Atl1. UE-side model monitoring

  - UE performs label data measurement, set B measurement and beam prediction and predicted result comparison with label data to obtain performance metric(s)

  - UE makes decision(s) of model selection/activation/deactivation/switching/

- fallback operation

- Atl2. NW-side model monitoring

  - UE performs label data measurement and set B measurement and beam prediction

  - NW performs predicted result comparison with label data to obtain performance metric(s)

  - NW makes decision(s) of model selection/activation/deactivation/switching/ fallback operation

- Atl3. Hybrid model monitoring

  - UE performs label data measurement, set B measurement and beam prediction and predicted result comparison with label data to obtain performance metric(s)

  - NW makes decision(s) of model selection/activation/deactivation/switching/ fallback operation

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the potential specification impact on resource configuration for model monitoring:

- Specific beam pair resource configuration

- P3+P2 resource configuration that Rx beam assumption of P2 resource measurement is the best Rx beam searched from P3 procedure

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the potential specification impact on assistance information for model monitoring:

- Tx beam information as assistance information from NW to UE

- Proprietary processed assistance information

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the potential specification impact on request signaling for model monitoring:

- Resource request signaling for data collection from UE to NW

  - Beam pair resources request for model monitoring purpose including the number of requested labels, and potentially some associated triggering events to be defined

  - P3+P2 beam sweeping resources request for model monitoring purpose including the number of requested labels, and potentially some associated triggering events to be defined

- Minimum resource number request for data collection from UE to NW

  - Minimum number of requested beams for model monitoring w or w/o resource request signaling

  - Minimum number of requested repetitions for model monitoring w or w/o resource request signaling

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the potential specification impact on monitoring report for model monitoring:

- Monitoring result report from UE to NW, including label data report or performance metric report

References

Chairman’s notes for 3GPP TSG RAN WG #94-e Meeting.

R1-2300447, Evaluation on AI/ML for beam management, RAN WG1 #111

R1-2210997, Discussions on AI/ML framework, RAN WG1 #111

Chairman’s notes for 3GPP TSG RAN WG1 #110 Meeting

Chairman’s notes for 3GPP TSG RAN WG1 #110bis e-Meeting