3GPP TSG RAN WG1 #112	                               R1-2300837

Athens, Greece, February 27th – March 3rd, 2023

Source:	NEC

Title:	Discussion on resource allocation for SL positioning reference signal

Agenda Item:	9.5.1.3

Document for:	Discussion and Decision

- Introduction

Based on the discussion of the potential solutions for SL positioning during RAN1#111 and RAN1#110bis-e meetings,, some agreements were achieved as follows [1][2]:

Agreement With regards to the SL Positioning resource allocation, support - Alt. 2: either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS. - Note: this does not imply that the design is the same for both types of resources pools - Note: shared resources pool(s) should be supported with backward compatibility Agreement From RAN1 perspective, at least the following 2 operation scenarios are recommended for normative work: - Operation Scenario 1: PC5-only-based positioning. - Operation Scenario 2: Combination of Uu- and PC5-based positioning. Agreement For Scheme 2, with regards to Resource allocation mechanism for SL-PRS, pick one or both of the following options: - Option 1: A sensing based resource allocation should be introduced - Option 2: A random resource selection should be introduced - In either option 1 or 2, the legacy designs for UE autonomous resource allocation should be used as a starting point. Study if/what enhancements may be needed. Agreement A dedicated SL-PRS resource pool is (pre-)configured in the only SL BWP of a carrier. Agreement Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE receives a SL-PRS resource allocation signaling from the network. Consider one or more of the following options: - Opt. 1: through higher layers from the LMF - Opt. 2: through Dynamic grant, or through configured grant type 1/type 2 from gNB - Up to further discussion which one or more of these shall be applicable Agreement With regards to SL signaling of the reservation/indication of SL-PRS resource(s) for dedicated resource pool and shared resource pool (if supported) for positioning: - Option A.1: SCI can be used for reserving/indicating one or more SL-PRS resource(s) - Note: This does NOT mean that only SCI is being used. There can still be higher layer signaling for the purpose of indicating a part of SL-PRS configuration. - FFS: Whether SCI is single stage SCI or two stage SCI - FFS: SL-MAC-CE or other higher-layer signaling reservation/indication Agreement At least for a dedicated resource pool for positioning, - With regards to the bandwidth of SL-PRS transmission, downselect from the following alternatives: - Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool - Note: Companies are encouraged to provide their analysis and views on the above alternatives - FFS: Bandwidth of SL-PRS transmission for shared resource pool (if supported) Agreement Study further the granularity of time-domain resource allocation for SL-PRS transmission. Agreement With regards to the SL-PRS time domain behavior, at least study the following behaviors from Tx UE perspective: - Periodic SL-PRS - SL-PRS is transmitted periodically with a transmission periodicity - FFS: any additional details, including whether or not higher layers can start/stop transmission. - Semi-persistent SL-PRS - SL-PRS is transmitted periodically with a transmission periodicity after activation and until deactivation - FFS: any additional details - Aperiodic SL-PRS - SL-PRS is transmitted at least once after [triggering/request] - Note: the brackets in the above means that companies are encouraged to study further whether “triggering” and/or “request” should be used and provide their definitions. - FFS: any additional details - FFS: Applicability of the above time behaviors for scheme 1 & scheme 2 - FFS: Rx UE behavior is separately discussed. - FFS: What mechanism(s) are used for activation/deactivation/triggering is part of the study

Considering maximum leveraging existing design in SL and positioning [3] [4], this contribution shares some views on resource allocation for SL positioning reference signal.

- Resource allocation for SL-PRS

  - Shared resource pool for positioning

When a UE is out of network coverage, it can only communicate with other UEs via PC5 interface in SL. While PSCCH/PSSCH cannot be transmitted in all slots and all RBs for NR system. Resource pools are defined for the respective channels. For OOC scenarios, UE can apply autonomous resource allocation, which is determined through a sensing procedure conducted autonomously by the transmitting UE. In such a case, the UE selects an appropriate amount of resources randomly and the resources selected are not periodic in general. Since the allocated resources can be random and thus non-contiguous in frequency domain. Hence, the resource allocation of SL-PRS should match these allocated resources for SL positioning.

Proposal 1: At least for OOC scenarios, non-contiguous frequency resource could be considered for the SL positioning.

Regarding two alternatives for SL-PRS resource allocation, i.e. dedicated resource pool and shared resource pool, the application of each one has an impact on existing SL specification. Since the positioning accuracy depends on SL-PRS bandwidth and received SNR at least for TOA based positioning method, the wideband frequency resource should be a common requirement for SL-PRS transmission on either shared resource pool or dedicated resource pool. Normally, the bandwidth of shared resource pool is not necessarily sufficient to meet the wideband requirement. Then the legacy resource pools sharing the same time resource configuration (by sl-TimeResource) in a BWP could be jointly used to provide sufficient contiguous or even non-contiguous frequency resource.

Proposal 2: Considering the combination of shared resource pools to provide wideband frequency resource for SL positioning.

  - Dedicated resource pool for positioning

Alternatively, the dedicated resource pool with wider bandwidth than legacy resource pool can be introduced only for positioning. Based on the (pre)configuration and/or the triggering mechanism for SL-PRS transmission, periodic or aperiodic time-frequency resource could be allocated through either scheme 1 or scheme 2 method. At least when the control information transmission for SL-PRS resource allocation is not supported in dedicated resource pool, there should be an association between the dedicated resource pool carrying SL-PRS and the shared resource pool carrying corresponding control information. To efficiently utilize the dedicated resource, the dedicated resource pool may be shared by multiple UEs performing the same or different positioning procedure, which operate in the same or different shared resource pools related to separate positioning procedure.

Proposal 3: At least when a dedicated resource pool is used only for SL-PRS transmission without other channel, the association between the dedicated resource pool and one or multiple shared resource pools should be considered.

  - Channels in dedicated resource pool

Different from the legacy resource pool in SL, a dedicated resource pool has a unique function only for SL positioning. If only SL-PRS transmission is supported in dedicated resource pool, then based on the association between the dedicated resource pool and one or more shared resource pools, the control information related to SL-PRS transmission has to multiplex the PSCCH in a shared resource pool with the legacy resource reservation/indication for common traffic data of PSSCH in the shared resource pool. Since the resource of two-stage PSCCH in shared resource pool is limited and the control information related to SL-PRS is only available depending on the ongoing positioning procedure, hence there is not a sufficient and necessary condition to transmit SL-PRS resource related control information in shared resource pool.

On the contrary, the legacy resource reservation/indication method could be reused in the dedicated resource pool, where a PSCCH only carries the control information associated with intending SL-PRS transmission(s). Especially, if more than one shared resource pools associate with the same dedicated resource pool, the PSCCH in dedicated resource pool seems to be the only path for conveying the SCI associated with SL-PRS transmission. Further, to reduce the complexity of reliable SL-PRS reception, the form of two-stage SCI in legacy PSCCH could be reused with necessary modification of new information, new field or new interpretation inserted.

Proposal 4: Regarding a dedicated resource pool for SL positioning, only SL-PRS and PSCCH which carries SCI associated with SL-PRS transmission(s) is expected to be transmitted, namely regarding which channels can be included in a dedicated resource pool in addition to SL-PRS, the Opt.2 should be supported.

  - Bandwidth and resource allocation granularity of SL-PRS

Further, considering different SL-PRS bandwidth corresponding to various positioning accuracy requirements, at least the frequency range of dedicated resource pool could accommodate one or more SL-PRS transmissions for different positioning procedures in a FDM way. While the simultaneous positioning procedures may be performed by the same or different positioning groups, and occupy overlapped or partly overlapped resources in time domain. Namely, if a dedicated resource pool is associated with one or more shared resource pools, the time-frequency resource in the dedicated pool may be allocated in a way of FDM or TDM for SL-PRS transmissions in different parallel or sequential positioning procedures. Accordingly, the bandwidth of SL-PRS can be smaller than that of the dedicated resource pool.

Proposal 5: The bandwidth of SL-PRS can be same or smaller than that of the dedicated resource pool for positioning. Namely regarding the bandwidth of SL-PRS transmission, Alt. 1 should be selected.

Proposal 6: The multiplexed SL-PRS transmissions within a slot should be supported at least for dedicated resource pool.

  - Physical SL positioning procedure

Though periodic, semi-persistent and aperiodic SL-PRS transmission are supported in SL positioning, the resource allocation for SL-PRS transmission is quite different from that for PRS in NR, namely the gNB based resource scheduling is not always feasible to SL-PRS resource allocation. At least for OOC scenario, resource allocation has to rely on the inter-UE sensing based autonomous mode. So, no matter for dedicated resource pool or shared resource pool, the SL-PRS resource allocation may encounter uncertain impacts, such as the hidden nodes in sensing procedure, as well as the half duplex issue for receiving UE. Hence, a feedback from the receiving UE could be considered to improve the reliable SL-PRS reception with low latency. Instead of measurement report of SL-PRS depending on the specific resource allocation, a simple feedback for the quick SL-PRS retransmission could reused the existing feedback channel in shared resource pool without allocating additional resource.

Proposal 7: The feedback of SL-PRS reception from receiving UE should be considered to request the quick SL-PRS retransmission.

In Rel-16/17 positioning, especially in DL positioning, all positioning methods including OTDOA, RTT, AOA/D, etc. assume that the references points with already known location, i.e., gNBs are fixed. In V2X, when anchor vehicle or road unit with known location is used as reference point, it is quite likely these nodes are moving and the movement of reference points might affect the positioning accuracy. According to [5], the UE velocity up to 250 km/h needs to be supported for outdoor and tunnel areas so that the relative velocity can be up to 500 km/h between two moving vehicles. This brings some new challenges and one of them is that the location of the reference points is no longer fixed as shown in Fig. 1.

Fig. 1 Relative movement of reference points

In Fig. 1, two vehicles are moving toward each other and if location of vehicle A is assumed to be known, it can be used as a reference point in addition to the gNB along the road for the positioning of vehicle B. However, unlike the fixed gNB/RSU along the road, vehicle A is moving. For example, assuming the relative velocity is 500 km/h and maximum allowed positioning latency is 100 ms, the distance between the original location of the moving reference points, i.e. vehicle A at the beginning of the positioning procedure and the location at the end of the procedure can be more than 13 meters, which might cause some issues, especially for use cases/scenarios that require sub-meter accuracy.

Proposal 8: The movement of reference points should be taken into account for the study of SL positioning methods including OTDOA, RTT, AOA/D, etc., especially for DL positioning.

  - SL positioning resource allocation mode

At least for OOC scenarios in SL positioning, mode 2 scheme is the only solution for SL-PRS resource allocation without network’s participation. Aiming to improve the reliability and efficiency of resource selection and coordinate the transmission of different nodes in such case, the inter-UE coordination (IUC) based approach can realize SL-PRS resource related information exchanging between target UE and anchor node(s). Both Scheme 1 and Scheme 2 IUC procedure can be considered as a starting point for selecting more reliable SL-PRS resource as well as avoiding the inter-UE block or interference.

Further, the specific features in positioning procedure, such as the bi-directional transmission of SL-PRS/measurement report and the parallel transactions with low latency in typical positioning methods, should be taken into account to meet the specific requirement on the SL-PRS resource allocation. Hence, at least the enhancement on the requesting/providing form and content may be necessary for the application of IUC in SL positioning.

Proposal 9: Both Scheme 1 IUC procedure and Scheme 2 IUC procedure can be considered as the starting point for SL-PRS transmission on at least dedicated resource pool for positioning. Namely, regarding the options of Inter-UE coordination for SL-PRS, Option 3 should be supported.

Proposal 10: The IUC procedure could be applied based on necessary enhancement on the positioning related indication.

The positioning measurement reports can be transmitted along with other data or separately. Therefore, both dedicated and shared resource pools may be supported for measurement reports if the data other than SL-PRS can be transmitted on dedicated resource pool. However, if shared resource pool is configured for measurement reports, higher priority should be given to such reports to guarantee the timely reception of the reports. Further, considering the limited size of measurement report and the low latency requirement of report in OOC scenarios, it may be carried as lower layer or higher layer signaling.

Proposal 11: Both dedicated and shared resource pools may be configured for measurement reports. If shared resource pool is configured, it should be prioritized.

- Conclusion

This contribution shares our views on resource allocation for SL positioning reference signal, and the following proposals are made:

Proposal 1: At least for OOC scenarios, non-contiguous frequency resource could be considered for the SL positioning.

Proposal 2: Considering the combination of shared resource pools to provide wideband frequency resource for SL positioning.

Proposal 3: At least when a dedicated resource pool is used only for SL-PRS transmission without other channel, the association between the dedicated resource pool and one or multiple shared resource pools should be considered.

Proposal 4: Regarding a dedicated resource pool for SL positioning, only SL-PRS and PSCCH which carries SCI associated with SL-PRS transmission(s) is expected to be transmitted, namely regarding which channels can be included in a dedicated resource pool in addition to SL-PRS, the Opt.2 should be supported.

Proposal 5: The bandwidth of SL-PRS can be same or smaller than that of the dedicated resource pool for positioning. Namely regarding the bandwidth of SL-PRS transmission, Alt. 1 should be selected.

Proposal 6: The multiplexed SL-PRS transmissions within a slot should be supported at least for dedicated resource pool.

Proposal 7: The feedback of SL-PRS reception from receiving UE should be considered to request the quick SL-PRS retransmission.

Proposal 8: The movement of reference points should be taken into account for the study of SL positioning methods including OTDOA, RTT, AOA/D, etc., especially for DL positioning.

Proposal 9: Both Scheme 1 IUC procedure and Scheme 2 IUC procedure can be considered as the starting point for SL-PRS transmission on at least dedicated resource pool for positioning. Namely, regarding the options of Inter-UE coordination for SL-PRS, Option 3 should be supported.

Proposal 10: The IUC procedure could be applied based on necessary enhancement on the positioning related indication.

Proposal 11: Both dedicated and shared resource pools may be configured for measurement reports. If shared resource pool is configured, it should be prioritized.

References

- Chairman’s notes for 3GPP TSG RAN1#110bis-e meeting

- Chairman’s notes for 3GPP TSG RAN1#111 meeting

- RP-223549, “Revised SID on Study on expanded and improved NR positioning”

- 3GPP TR 37.985 V17.1.1, “Overall description of Radio Access Network (RAN) aspects for Vehicle-to-everything (V2X) based on LTE and NR,” Release 17, 2022-03.

- 3GPP TR 38.845 V17.0.0, “Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases,” Release 17, 2021-09.