3GPP TSG RAN WG1 #112			R1-2301007

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.5.1.2.

Source: 	CMCC

Title: 	Discussion on measurements and reporting for SL positioning

Document for:	Discussion and Decision

Introduction

In RAN#98-e meeting, a new WID on expanded and improved NR positioning was approved [1], in which the application of NR positioning is further expanded to enable sidelink positioning and ranging, improved accuracy, integrity and power efficiency, and positioning for RedCap UEs. To support sidelink positioning and ranging, the following objectives were identified:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution, we provide our initial views on measurements and reporting to support RTT-type solutions using SL, SL-AoA, and SL-TDOA.

Discussion

- 2.1 SL measurements

SL RSTD measurement

During RAN1#111 meeting, it was agreed that for SL-TDOA, DL-TDOA-like operation and UL-TDOA-like operation should be introduced. To support DL-TDOA-like methods, SL PRS-based RSTD measurement was agreed to be introduced.

In NR R16/17 positioning, UE DL RSTD measurement was defined to support DL-TDOA positioning method, of which the definition in TS 38.215 is provided as follows:

| Definition | DL reference signal time difference (DL RSTD) is the DL relative timing difference between the Transmission Point (TP) [18] j and the reference TP i, defined as TSubframeRxj – TSubframeRxi, Where: TSubframeRxj is the time when the UE receives the start of one subframe from TP j. TSubframeRxi is the time when the UE receives the corresponding start of one subframe from TP i that is closest in time to the subframe received from TP j. Multiple DL PRS resources can be used to determine the start of one subframe from a TP. For frequency range 1, the reference point for the DL RSTD shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSTD shall be the antenna of the UE. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE |

Understanding of SL RSTD measurement

In the last RAN1 meeting, joint Uu- and PC5-based positioning was recommended for normative work and this objective was included in the WID. To consider such scenario, we would like to first clarify whether SL RSTD considering the time difference of a DL PRS and a SL PRS is included or not.

Observation 1: Whether SL RSTD considering the time difference of a DL PRS and a SL PRS for joint Uu- and PC5-based positioning should be clarified.

In our views, if SL RSTD considering the time difference of a DL PRS and a SL PRS is included in the scope, further enhancements may be introduced for legacy LPP signalling. For example, as shown in below, considering the in-coverage or partial coverage scenarios where joint Uu- and PC5-based positioning is used, if a UE determines the anchor UE and/or the SL PRS resource as the reference for SL RSTD measurement, the corresponding measurement reporting should be enhanced. To limit the workload of the normative work, our preference is to de-prioritize this case.

Figure 1: Illustration of SL RSTD measurement with time difference of a DL PRS and a SL PRS.

Proposal 1: SL RSTD measurement considering time difference of a DL PRS and a SL PRS for joint Uu- and PC5-based positioning is de-prioritized.

SFN/DFN initialization time

In NR positioning, RTD information was defined to provide time synchronization information between a reference TRP and a list of neighbour TRP. In sidelink positioning, as a UE can select GNSS, a cell, or a reference UE as its synchronization source and to obtain the DL timing and TDD pattern, therefore, different anchor UEs to support SL TDOA positioning may have different DFN or SFN initialization time, and a SFN/DFN time synchronization information between the anchor UEs should be defined.

Proposal 2: Define time synchronization information between the anchor UEs to assist SL TDOA positioning.

Reference anchor UE/reference SL PRS

In NR positioning, a UE can be indicated by the network the reference information for DL RSTD measurements. The reference information may include TRP ID, DL PRS resource set ID, or one or more DL PRs resource ID. Meanwhile, the UE may use difference DL PRS resource or resource set to determine the DL RSTD measurement. In NR positioning, as the network is able to obtain some priori information of the UE location or UE measurements, it can determine a “best” TRP/DL PRS resource set/DL PRS resource as the reference to perform measurement.

In sidelink positioning, however, as it is applicable in out-of-coverage scenarios, whether and how the reference anchor UE/SL PRS resources is (pre-)configured should be discussed, and this issue is also related to who and how the SL PRS configuration is provided to the target UE. To our understanding, the most straightforward solution is to let UE determine the reference anchor UE and/or SL PRS resource and carry the corresponding information in measurement reporting.

Proposal 3: For sidelink positioning, whether and how the reference anchor UE/SL PRS resources is (pre-)configured should be discussed.

Proposal 4: The UE can determine the reference anchor UE and/or SL PRS resource by itself and carry the corresponding information in the measurement reporting.

RSTD search window

In NR positioning, RSTD search window per neighbouring TRP is provided to target UE in the assistance data based on expectedRSTD and expectedRSTD-Uncertainty. The RSTD search window is predicted by LMF based on a priori UE location by, e.g., applying E-CID positioning method. By providing the RSTD search window, it assists the target UE to locate and monitor a PRS from neighbouring TRPs.

In sidelink positioning, as the communication range between two UEs via sidelink is limited, e.g., several hundred meters. In our views, it seems not necessary to define RSTD search window for a target UE to receive SL PRS from anchor UEs. On the other hand, in NR positioning, LMF is the entity and has priori information to provide the RSTD search window. In sidelink positioning, however, it is not clear who and how to provide the RSTD search window.

Observation 2: As the communication range between two UEs via sidelink is limited, it seems not necessary to define RSTD search window for a target UE to receive SL PRS from anchor UEs.

Observation 3: In sidelink positioning, it is not clear who and how to provide the RSTD search window.

Proposal 5: RSTD search window is not required to be configured/provided to the UE.

SL RTOA measurement

To support UL-TDOA-like operation, SL PRS-based RTOA measurement was also agreed to be defined during the normative work.

The definition of gNB UL RTOA measurement to support UL-TDOA positioning was as follows:

| Definition | The UL Relative Time of Arrival (TUL-RTOA) is the beginning of subframe i containing SRS received in Reception Point (RP) [18] j, relative to the RTOA Reference Time [16]. The UL RTOA reference time is defined as T0+tSRS, where - T0 is the nominal beginning time of SFN 0 provided by SFN Initialization Time [15, TS 38.455] - tSRS=10nf+nsf×10-3, where nf and nsf are the system frame number and the subframe number of the SRS, respectively. Multiple SRS resources can be used to determine the beginning of one subframe containing SRS received at a RP. The reference point for TUL-RTOA shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. |
| --- | --- |

In sidelink, the synchronization source of a UE to obtain DL timing includes GNSS, a cell (gNB/eNB), or a reference UE. The UE selection of the synchronization source is based on the synchronization type (GNSS-based or gNB/eNB-based synchronization), the priority level of synchronization source and S-SSB RSRP. In such cases, the anchor UE may be synchronized to GNSS, and T0 to define the SL RTOA reference time should be the nominal beginning time of DFN 0.

Proposal 6: The SL RTOA is the beginning of subframe i containing SL PRS received in anchor UE j, relative to the RTOA reference time.

- The SL RTOA reference time is defined as T0+tSL PRS, where

  - T0 is the nominal beginning time of SFN 0 or DFN 0 provided by SFN or DFN Initialization time.

  - tSL PRS=10nf+nsf×10-3, where nf and nsf are the system frame number or direct frame number and the slot number of the SL PRS, respectively.

SL Rx-Tx time difference

To support RTT-type solutions using SL, SL Rx-Tx time difference measurement was also agreed to be defined during the normative work.

In NR positioning, the definition of UE Rx-Tx time difference measurement to support multi-RTT positioning was as follows:

| Definition | The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink subframe #i from a Transmission Point (TP) [18], defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the TP. Multiple DL PRS or CSI-RS for tracking resources, as instructed by higher layers, can be used to determine the start of one subframe of the first arrival path of the TP. For frequency range 1, the reference point for TUE-RX measurement shall be the Rx antenna connector of the UE and the reference point for TUE-TX measurement shall be the Tx antenna connector of the UE. For frequency range 2, the reference point for TUE-RX measurement shall be the Rx antenna of the UE and the reference point for TUE-TX measurement shall be the Tx antenna of the UE. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE |

In sidelink, there is only sidelink slots and UE follows DL timing. Hence, the definition of SL Rx-Tx time difference should be accordingly updated, where TUE-Rx is the UE received timing of sidelink slot #i from a UE, defined by the first detected path in time, and TUE-Tx is the UE transmit timing of sidelink slot #j that is closest in time to the slot #i received from a UE.

Proposal 7: The SL Rx-Tx time difference is defined as TUE-Rx - TUE-Tx, where

- TUE-Rx is the UE received timing of sidelink slot #i from a UE, defined by the first detected path in time.

- TUE-Tx is the UE transmit timing of sidelink slot #i from the nominal transmit timing

SL RSRP/RSRPP measurement

In NR R16 positioning, DL PRS-RSRP was defined to support DL-AoD. In addition, DL PRS-RSRPP was further introduced to improve the positioning accuracy, the definition is captured as follows respectively:

DL PRS-RSRP

| Definition | DL PRS reference signal received power (DL PRS-RSRP), is defined as the linear average over the power contributions (in [W]) of the resource elements that carry DL PRS reference signals configured for RSRP measurements within the considered measurement frequency bandwidth. For frequency range 1, the reference point for the DL PRS-RSRP shall be the antenna connector of the UE. For frequency range 2, DL PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported DL PRS-RSRP value shall not be lower than the corresponding DL PRS-RSRP of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE |

DL PRS-RSRPP

| Definition | DL PRS reference signal received path power (DL PRS-RSRPP), is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry DL PRS signal configured for the measurement, where DL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time. For frequency range 1, the reference point for the DL PRS-RSRPP shall be the antenna connector of the UE. For frequency range 2, DL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE |

The same definition based on SL PRS should be used. Therefore, we propose that:

Proposal 8: SL RSRP is defined as the linear average over the power contributions (in [W]) of the resource elements that carry SL PRS configured for RSRP measurements within the considered measurement frequency bandwidth.

Proposal 9: SL RSRPP is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry SL PRS configured for the measurement, where SL RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time.

SL AoA/ZoA measurement

In NR R16 positioning, UL AoA/ZoA was defined to support UL AoA positioning. The definition is as follows:

| Definition | UL Angle of Arrival (UL AoA) is defined as the estimated azimuth angle and vertical angle of a UE with respect to a reference direction, wherein the reference direction is defined: - In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction - In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relatize to z-axis of LCS and positive to x-y plane direction. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901 [15]. The UL AoA is determined at the gNB antenna for an UL channel corresponding to this UE. |
| --- | --- |

In our views, as the definition of UL AoA/ZoA, the reference direction in both GCS and LCS can be considered for SL AoA/ZoA.

For absolute positioning, no matter the location is calculated at the target UE or the server UE, as the reference direction in LCS is dependent on UE location and antenna orientation, GCS is required to align the measurements performed by difference UEs locally. The measurements can be reported in GCS, or in LCS along with the LCS-GCS transformation information.

For relative positioning/ranging, there are two cases. One is that the target UE is the device to perform the SL-AoA/ZoA measurements; the other is that the anchor UE is the device to perform the SL-AoA/ZoA measurements and the measurements are then reported back to the target UE. In the latter case, if the anchor UE is not capable to support GCS, this may lead to misunderstanding of the measurements. In our views, if an anchor UE does not support GCS, it can inform the target UE in capability transfer procedure, and the anchor UE will be the one sending the SL PRS and let the target UE to be the one perform SL-AoA/ZoA measurements.

Proposal 10: For the SL AoA/ZoA measurement, the reference direction in both GCS and LCS should be supported.

- 2.2 SL measurement reporting

In NR positioning, UE-based and UE-/RAN-assisted positioning were specified, where for UE-based positioning, the target UE itself is the entity to calculate the location, and for UE-/RAN-assisted positioning, UE and/or gNB should report measurement to the LMF, which is the entity to calculate the location. In addition, in NR, UE-based positioning is only applicable to DL only positioning method, i.e., DL TDOA and DL-AoD.

In sidelink positioning, we think that both UE-based and UE-assisted positioning should be supported, in which for UE-based positioning, the target UE is the entity to calculate the absolute/relative location/ranging information, and for UE-assisted positioning, LMF (in case of joint Uu- and PC5-based positioning in in-coverage or partial coverage scenarios) or server UE (in case of PC5-only positioning) is the entity to calculate the absolute/relative location. For UE-based positioning, we think that whether it is applicable to UL-like SL absolute positioning method, i.e., UL-like SL TDOA and SL AoA, can be further discussed. If it is the case, measurement reporting from anchor UEs to the target UE should be considered.

Proposal 11: In sidelink positioning, whether UE-based positioning is applicable to UL-like SL absolute positioning method, i.e., UL-like SL TDOA and SL AoA, and if supported, the corresponding measurement reporting from anchor UEs to the target UE should be further discussed.

For UE-assisted positioning, and considering DL-like SL absolute positioning, i.e., DL-like SL TDOA, the SL RSTD measurement and/or SL RSRP/RSRPP measurement should be reported from the target UE to LMF or location server, associated with SL PRS index, anchor unique UE ID, and time stamp.

Proposal 12: For DL-like UE-assisted SL absolute positioning, the SL RSTD measurement and/or SL RSRP/RSRPP measurement should be reported to the LMF or location server, associated with SL PRS index, anchor unique UE ID, and time stamp.

For relative positioning/ranging, the SL Rx-Tx time difference measurement, SL AoA/ZoA measurement and/or SL RSRP/RSRPP measurement should be reported to from the anchor UE to the target UE, associated with SL PRS index, UE ID (i.e., source ID and/or destination ID) and time stamp.

Proposal 13: For relative positioning/ranging, the SL Rx-Tx time difference measurement, SL AoA/ZoA measurement and/or SL RSRP/RSRPP measurement should be reported to from the anchor UE to the target UE, associated with SL PRS index, UE ID (i.e., source ID and/or destination ID) and time stamp.

Conclusions

In this contribution, we provide our views on measurements and reporting for SL positioning, and the following observations and proposals are made:

Observation 1: Whether SL RSTD considering the time difference of a DL PRS and a SL PRS for joint Uu- and PC5-based positioning should be clarified.

Observation 2: As the communication range between two UEs via sidelink is limited, it seems not necessary to define RSTD search window for a target UE to receive SL PRS from anchor UEs.

Observation 3: In sidelink positioning, it is not clear who and how to provide the RSTD search window.

Proposal 1: SL RSTD measurement considering time difference of a DL PRS and a SL PRS for joint Uu- and PC5-based positioning is de-prioritized.

Proposal 2: Define time synchronization information between the anchor UEs to assist SL TDOA positioning.

Proposal 3: For sidelink positioning, whether and how the reference anchor UE/SL PRS resources is (pre-)configured should be discussed.

Proposal 4: The UE can determine the reference anchor UE and/or SL PRS resource by itself and carry the corresponding information in the measurement reporting.

Proposal 5: RSTD search window is not required to be configured/provided to the UE.

Proposal 6: The SL RTOA is the beginning of subframe i containing SL PRS received in anchor UE j, relative to the RTOA reference time.

- The SL RTOA reference time is defined as T0+tSL PRS, where

  - T0 is the nominal beginning time of SFN 0 or DFN 0 provided by SFN or DFN Initialization time.

  - tSL PRS=10nf+nsf×10-3, where nf and nsf are the system frame number or direct frame number and the slot number of the SL PRS, respectively.

Proposal 7: The SL Rx-Tx time difference is defined as TUE-Rx - TUE-Tx, where

- TUE-Rx is the UE received timing of sidelink slot #i from a UE, defined by the first detected path in time.

- TUE-Tx is the UE transmit timing of sidelink slot #i from the nominal transmit timing

Proposal 8: SL RSRP is defined as the linear average over the power contributions (in [W]) of the resource elements that carry SL PRS configured for RSRP measurements within the considered measurement frequency bandwidth.

Proposal 9: SL RSRPP is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry SL PRS configured for the measurement, where SL RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time.

Proposal 10: For the SL AoA/ZoA measurement, the reference direction in both GCS and LCS should be supported.

Proposal 11: In sidelink positioning, whether UE-based positioning is applicable to UL-like SL absolute positioning method, i.e., UL-like SL TDOA and SL AoA, and if supported, the corresponding measurement reporting from anchor UEs to the target UE should be further discussed.

Proposal 12: For DL-like UE-assisted SL absolute positioning, the SL RSTD measurement and/or SL RSRP/RSRPP measurement should be reported to the LMF or location server, associated with SL PRS index, anchor unique UE ID, and time stamp.

Proposal 13: For relative positioning/ranging, the SL Rx-Tx time difference measurement, SL AoA/ZoA measurement and/or SL RSRP/RSRPP measurement should be reported to from the anchor UE to the target UE, associated with SL PRS index, UE ID (i.e., source ID and/or destination ID) and time stamp.

References

- RP-223549, New WID on Expanded and Improved NR Positioning, Intel Corporation, CATT, Ericsson, 3GPP TSG RAN Meeting 98-e.