3GPP TSG RAN WG1 #112	R1- 2301070

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.8.1

Source: 	LG Electronics

Title: 	Discussion on side control information and NCR behaviour

Document for:	Discussion and decision

Introduction

In RAN#97-e meeting, a work item on NR network-controlled repeaters is approved and the corresponding description is provided in [1]. According to the WID, objectives of the work item are as follows in RAN1 perspective.

Specify the signalling and behavior of the following side control information for controlling the NCR-Fwd [RAN1, RAN2] - Beamforming - UL-DL TDD operation - ON-OFF information Note: Power control aspect will be checked in RAN#98e. Specify control plane signalling and procedures [RAN2, RAN1] - The configuration of signalling for side control information indication - NOTE: Down-selection of solutions in section 7.2 of TR 38.867 is needed

The assumptions and scenarios for NCR is described in WID as follows:

The objectives of NR NCR WI follow the recommendations defined in TR 38.867 and will focus on scenarios and assumption listed below: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands based on the NCR model in TR38.867 - For only single hop stationary network-controlled repeaters - The NCR is transparent to the UE. - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously

In this contribution, we discuss the remaining topics regarding signalling and behaviour of NCR based on above work item objectives.

Access link beam indication

For access link beam indication, following agreements were made in the last meeting [2]. Based on the agreements, indication of access link beam index and corresponding time resource by side control information are discussed in this section.

Agreement The following is supported to deliver the information to characterize the supported physical beam of NCR-Fwd for access link: Option-2: The information is informed to gNB and NCR via OAM - Note-1: In this option, how to characterize the beam information is based on implementation (e.g., declaration from NCR vendor). - Note-2: In this option, the beam(s) used by NCR-Fwd for access link is configured for gNB and NCR by OAM based on implementation. - The beam index in SCI corresponds to the configured beam(s) sequentially. Agreement For each periodic beam indication for access link, one RRC signalling is used including the information defined by the following: - A list of X (1≤X≤Xmax) forwarding resource, each is defined as {Beam index, time resource} - FFS: The value of Xmax, other details Each time resource is defined by {Starting slot defined as the slot offset in one period, starting symbol defined by symbol offset within the slot, duration defined by the number of symbols} with dedicated field. - The periodicity is configured as part of the RRC signaling for periodic beam indication - The same periodicity is assumed for all time resource(s) in one periodic beam indication. - The reference SCS is configured as part of the RRC signaling for periodic beam indication - The same reference SCS is assumed for all time resource(s) in one periodic beam indication. Agreement For each aperiodic beam indication for access link, one DCI is used with the information defined by Option-1: - Lmax fields are used to indicate the beam information and each field refers to one beam index ; - Note: The bitwidth of this field is determined by the number of beams used for access link. - Tmax fields to indicate the time resource; - Note: A list of time resource is pre-defined by RRC signalling. The bitwidth of this field for time resource indication is determined by the length of list. - FFS: The value of Tmax - Down-select between Tmax=1 or Tmax=Lmax. - FFS: How to define the association between time indication and beam indication Each time resource is defined by {Starting slot defined as the slot offset, starting symbol defined by symbol offset within the slot, duration defined by the number of symbols} with dedicated field.

Signaling for access link beam indication

For signalling of access link beam indication, periodic, semi-persistent and aperiodic indication can be supported. The common remaining issue among periodic, semi-persistent and aperiodic indication could be time resource configuration. A set of time resources can be indicated by aperiodic indication of access link beam should be configured to NCR-MT in advance by RRC according to previous agreement, it can be assumed that same principle is applied for defining time resource. With those reasons, time resource for access link beam indication is discussed in section 2.1.1, and other topics specific for configuration methods are discussed in following sections separately.

Time resource for access link beam indication

In discussing the time resource for the NCR's access link beam indication, what need to be kept in mind is that the NCR is indicated to perform forwarding operation for the time duration when the signal/channel between the NCR and the UE and/or the gNB is actually transmitted and received. Therefore, the configuration of time resources to which beam indications are applied should be considered based on indications of time resources to which existing signals/channels are allocated. In NR, a slot is basic unit of configuration for transmission or reception of signal/channel. Without loss of generality, for time resources for access link beam indication, time resources configured over multiple slots should also be considered.

In case of repetition or TBoMS configuration, the time resource for transmission is generally indicated by slot offset, SLIV (start and length indicator value), number of slots (or repetition number). Except for the PUSCH repetition type B, SLIV is commonly applied for the multiple slots determined by repetition. That is, symbol resources in which actual signal/channel transmission is performed according to SLIV can be non-contiguous when signals/channels are repeatedly transmitted in consecutive slots.

So far, the wording had been agreed is {starting slot defined as the slot offset in one period, starting symbol defined by symbol offset within the slot, duration defined by the number of symbols}. There was a discussion that a method of indicating a symbol interval beyond a slot boundary is required to reflect time resources when a repetition of signal/channel is configured in multiple slots. However, since conventional repetition can be performed with non-contiguous symbol resources, actual signals/channels do not properly reflect such transmitted resources except the only case that SLIV indicates the full slot. That implies this method is likely to indicate a time resource where a signal/channel between the UE and the NCR is not transmitted, which concludes causing unnecessary interference to the network due to the fact that noise is amplified and transmitted by NCR. The only possible solution to resolve it, i.e., to suppress interference to the network, is to indicate NCR to be OFF during those time duration after beam indication, which is waste of signalling.

Figure 1. Example of access link beam configuration for NCR

The detailed example of time resource configuration is given; consider the case of NCR serving user A and user B simultaneously. The NCR should apply “beam index a” for PDSCH repetition for user A from the 4-th symbol to the 7-th symbol for three consecutive slots, and NCR should apply “beam index b” for PDSCH repetition for user B from the 8-th symbol to the 11-th symbol in the same slots. The example is presented in Figure 1. With the time duration defined by symbol duration beyond the slot boundary, six different time resources are required to indicate “beam index a” and “beam index b” which is not efficient at all. In this background, when a time resource in the form of an indication common SLIV in the different slots is supported, the beam configuration in example can be much simpler by configuring “beam index a” and “beam index b” to the two time resources, respectively.

Proposal 1: For time resource of access link beam indication, multiple slots indication based on common SLIV should be supported.

For the next, the case where actual repetition is configured should be investigated specifically. As an example, PDSCH repetition is assumed. For the repetition number of PDSCH/SPS-PDSCH, same symbol allocation is applied across the K consecutive slots. It should be noted that “consecutive” slots. Due to the limited number of configurable repetition number and limitation of “consecutive” slots, the actual PDSCH transmission could not be performed for all of configured consecutive slots. For example, consider the case PDSCH repetition is configured in 8 slots from slot #0 to slot #7.In the middle of the corresponding slot, the slot format of slot #1, slot #2 and slot #6 is indicated to be flexible and uplink by TDD configuration. Then, PDSCH is not transmitted in the corresponding slots. It is shown in Figure 2.

Figure 2. Example of PDSCH configuration and following access link beam for NCR

In short, the actual transmission/reception of signal configured by repetition can be non-consecutive slots even when the repetition is configured for consecutive slots. In this case, at least the same beam should be configured in the time resource when the actual PDSCH is transmitted, that is, the same beam should be configured in a multiple non-contiguous slots.

Proposal 2: It is necessary to consider that the time resource is composed of non-continuous slots.

Against this background, two alternatives can be considered for the indication of common SLIV repeated in multiple slots. As a first alternative, a slot offset, symbol offset, duration, and repetition number are included in a single time resource based on the existing NR repetition configuration scheme (i.e., number of repetitions and SLIV). An example of such an indication is shown in Figure 3.

Figure 3. Time resource defined by {slot offset, symbol offset, duration, repetition number}

It is the most similar configuration method with conventional TDRA configuration. However, non-consecutive multiple slots cannot be indicated directly. Therefore considering signal/channel to be relayed is located in non-consecutive slots as shown in Figure 2, multiple time resources is required even for relaying single repetition. In example of Figure 2, three time resources ({slot #0}, {slot #3, slot #4, slot #5}, {slot #7}) are required which implies increment of signalling overhead due to configuration limitation on time resources.

To overcome such limitation on configuration, slot format configuration information can be additionally used for determining time resources to indicate non-consecutive multiple slots as a time resource.

There could be a way to determine that only symbols with the same link direction are valid time resources for beam indication. For instance, it could be considered to exclude a certain slot from the slot group for the time resource if a slot contains symbols that link direction is changed (such as flexible symbols or UL symbols in the example in Figure 2). This method is consistent with the agreed NCR behaviour that NCR operates “OFF” in flexible resource by common TDD configuration. Another advantage of this method is that, if the slot format is updated and the actual signal for the time resource being transmitted changes, an update of the time resource for access link beam indication may not be necessary or may only be required for some of the resources.

As a second alternative, there is a method of including multiple slot offsets and a single symbol offset and duration in the single time resource. In this case, it would be appropriate to apply the symbol offset and duration as common parameters within the multiple slots indicated in single time resource. An example of this is shown in Figure 4. With this method, information for multiple slots that are subject to the same SLIV can be indicated at once, enabling a single time resource to be composed of discontinuous slots.

Figure 4. Time resource defined by {slot offset list, symbol offset, duration}

Proposal 3: The time resource of access link beam indication is defined by:

- Alt. 1) slot offset, SLIV (symbol offset and symbol duration), number of slot(s)

- Alt. 2) slot offset list, SLIV (symbol offset and symbol duration)

Periodic configuration

The conflict between/within periodic configuration needs to be discussed for the periodic configuration. In this context, "conflict between/within periodic configuration" refers to the case where different forwarding resources indicate different beams for overlapping time resources included in multiple periodic access link beam indications configured in NCR. This means that if different forwarding resources indicate different beams for the same time resource, it can result in a conflict within a periodic configuration. However, since periodic configuration is signalled via RRC, our understanding of this is that NCR assumes that different time resources in the same or different periodic configurations do not overlap with each other.

Proposal 4: NCR-MT does not expect conflict between/within periodic configuration(s) of access link beam indication.

Semi-persistent configuration

Similar to other semi-persistent configurations (e.g., SPS-PDSCH, CG-PUSCH), it would be appropriate to reuse the periodic configuration method for semi-persistent configuration, unless there are specific reasons not to. That is, like in periodic configurations, each semi-persistent configuration should support independent settings of periodicity and reference SCS for each configuration, based on the same logical extension where the same periodicity and reference SCS are agreed upon within a configuration. In addition, there may be insufficient time to discuss the structure of DCI for activation/deactivation of semi-persistent configuration, and it would be desirable to indicate it through MAC-CE, as in activation/deactivation of periodic configuration.

In addition, there is a need to discuss whether to perform activation or deactivation of single or multiple configurations with a single indication. First, it is widely understood that semi-persistent configuration should indicate activation or deactivation of multiple {beam index, time resource} forwarding resources with a single indication. The difference between activation/deactivation of a single configuration via a single indication and activation/deactivation of multiple configurations via a single indication is the RRC signalling for candidate configurations. That is, activation/deactivation of a single configuration via a single indication involves preconfiguring candidate configurations that include forwarding resource(s) via RRC and setting a single configuration for activation/deactivation. With full flexibility assumed, this method would require setting up a number of candidate configurations equivalent to the number of possible combinations of forwarding resources via RRC in advance.

On the other hand, the activation/deactivation of multiple configurations via a single indication involves pre-configuration of candidate configurations that include forwarding resource(s) by RRC configuration and then activating/deactivating multiple configurations from among them. Similarly, assuming full flexibility, the number of candidate configurations that needs to be configured in advance using RRC depends on the number of configurations that can be indicated at once. Assuming that there is no limitation on the number of candidate configurations, but if there is no limit to the number, only the candidate configurations that include one forwarding resource per configuration are needed, and the number of candidate configurations required would be equal to the number of required forwarding resources.

Figure 5. Example of RRC candidate configuration for semi-persistent configuration

Imagine a case there are three forwarding resources supported by the NCR, which are forwarding resource a, forwarding resource b, and forwarding resource c. To support full flexibility, the activation/deactivation of multiple configurations via a single indication can be used by setting up three of candidate configurations containing a single forwarding resource by RRC in advance and activating/deactivating using MAC-CE for one or multiple configurations. On the other hand, to support full flexibility using the activation/deactivation of single configuration via single indication, all possible combinations of forwarding resources a, b, and c should be set up by RRC in advance, and a single configuration needs to be activated/deactivated using MAC-CE. This issue is illustrated in Figure 5. The problem with setting up these possible combinations in advance is that when the reference SCS and periodicity vary by forwarding resource, the possible combinations should be set up according to the longest periodicity and the largest reference SCS. In other words, configurable reference SCS and periodicity becomes meaningless, and it also increases signalling overhead. Therefore, activation/deactivation of multiple configurations via a single indication should be supported.

Proposal 5: Multiple configuration is activated/deactivated via single indication for semi-persistent indication.

If it is assumed that multiple configuration is activated/deactivated via single indication, example shown in Figure 5 is quite dramatic to be generalized since it is dedicated example for flexibility comparison between methods. Since NCR serves multiple of UEs, it is very unlikely that every single forwarding resources have different reference SCS and periodicity and each configuration consist of single forwarding resources. In general, it would be more realistic that single configuration contains multiple forwarding resources and among the forwarding resources, reference SCS and periodicity can be shared.

Proposal 6: Single configuration can contain multiple forwarding resources and reference SCS and periodicity can be shared within some of forwarding resources.

Aperiodic configuration

The aperiodic configuration is indicated by DCI, which indicates the access link beam indication, so a discussion is needed on the beam information field and time resource field of DCI. In addition, since the DCI payload size cannot vary instantaneously, a method is needed to indicate that only some of a certain number of beam indications indicated by DCI are valid beam configurations, and this also needs to be discussed.

Beam information field of DCI

The bit width of the beam information field depends on the number of beam indication fields, Lmax, and the number of beams that can be indicated in a single beam indication field, M. NCR needs to be aware of information on M and Lmax to determine the DCI payload size. It goes without saying that Lmax and M should be values that take into account the number of supported beams by NCR, NAC.

It needs to be investigated case by case to determine value. For the first, assuming that M is a fixed value and less than the number of supported access link beams, NAC. Then, NCR needs to pre-configure the information for the M beams that can be indicated in the DCI out of the NAC beams. Since NAC cannot be a single value among different NCRs, this pre-configuration is required. Conversely, if M is greater than NAC, it would result in waste of beam indication field bits. Depending on NCR capabilities or the frequency range, the number of access link beams supported by the NCR may differ. Additionally, due to measurement overhead and signalling overhead of UE, only a limited number of beams among those supported by the NCR may need to be used. Given these cases, the number of beams that can be indicated in a single beam indication field should be a configurable value, indicated by the gNB to NCR as M (M ≤ NAC) before the gNB issues the aperiodic configuration via DCI. This is shown in Figure 6 as an example. This method might restrict some of beams that can be dynamically indicated, but the gNB can dynamically reconfigure the beams as necessary.

Figure 6. Example of configuration for access link beam index for side control information and associated physical beams

For the next, consider the case where Lmax is a fixed value. In this case, if the gNB wants to indicate a number of beam indications that is less than Lmax using aperiodic indication to NCR, it would result in waste of DCI payload. On the other hand, if the gNB wants to indicate a number of beam indications that is greater than Lmax, it would cost multiple indication of DCI. To prevent waste of payload, waiting for the gNB to always need Lmax beam indications would make timely beam indication impossible. The appropriate value for Lmax would depend on the payload of signals forwarded by NCR, the number of UEs supported by NCR, and the number of beams. Therefore, it would be more appropriate to make Lmax configurable rather than fixed value.

Proposal 7: The beam information field size and number of beam information field of DCI for aperiodic access link beam configuration is pre-configured by gNB to NCR in advance.

Time resource field of DCI

For aperiodic indication of access link beam, there are two options for the time resource. The first option is to indicate the time resource using the same number of fields as the beam indication (Tmax=Lmax), and the second option is to indicate the time resource using a single field (Tmax=1). The difference between the two options lies in how finely the time resource is divided, as the aperiodic indication time resource is predefined by RRC. An example is shown in Figure 7.

For the first option where Tmax=Lmax, if m bits are needed to represent each time resource, then the bitwidth of time resource field will be size of m × Lmax. To support full flexibility, let the number of candidate time resources indicated by RRC should be 2m. For comparison, in the second option where Tmax=1, if n bits are required to represent the time resources, then the time resource field will be n bits, and the number of candidate time resources indicated by RRC is 2n. For both options to support the same indication flexibility, a relationship of 2m!=2n is required, and even in cases where the number of beams indicated by single indication is not fixed, n must satisfy  n=2m. This leads to a significant overhead for candidate time resources, and therefore Tmax=Lmax is more appropriate.

Figure 7. Example of DCI bit field and RRC candidate time resource configuration

Proposal 8: For time resource of aperiodic configuration, Tmax = Lmax is supported.

Valid beam configuration indication

Last but not least, it needs to be considered when only L (<Lmax) number of beams are indicated by single aperiodic indication. When there are Lmax beam information fields and time resource fields in a DCI, not all Lmax fields may be used for actual beam indication. When only L < Lmax beam indication fields are used for actual beam indication, the NCR-MT should be able to determine that the remaining Lmax - L fields are unused fields. For this purpose, it is necessary to make the time resource field can indicate ‘no time resource’. If it is indicated by a time resource field, the beam information field corresponding to the time resource field can be determined as an unused field. To support it, if the time resource field indicates a specific value (e.g., 0), it can be interpreted as "no time resource," and the beam indication through the related beam information field may not be applied.

Proposal 9: In the case where only L (< Lmax) beams are indicated in the aperiodic beam indication, it is necessary to determine which of the Lmax beam information fields are not used for beam indication.

- If the time resource field indicates a specific value, the associated beam information field can be considered as not used for beam indication.

Conflict of access link beam indication

In the last meeting, the conflict issue of access link beam indication between two different indications over same resource was discussed.

Conflict between dynamic/semi-persistent/periodic indications for access link

It is natural for dynamic indication to have higher priority than periodic indication. When considering cases of conflicts between different signals/channels in the transmission, such as in the following examples, where the transmission of a previously scheduled signal/channel is overridden by the transmission of a later scheduled signal/channel, it is important to give priority to the dynamic indication:

- eMBB data transmission being overridden by URLLC data transmission

- SRS transmission being overridden by PUSCH/PUCCH transmission in case of collision between transmission resources

Given the behavior of existing UEs, a new beam indication should override a previous beam indication for the same time resource aligned to UE behavior. At least semi-static/semi-persistent configurations should be overridden by dynamic indication. Some companies argue that periodic indication should have priority to protect some of important signals/channels (e.g., SSB, CSI-RS, CORESET, etc.). However, we can expect the network to perform semi-persistent or dynamic indication to avoid such conflicts.

Proposal 10: In case of conflict between semi-static/semi-persistent configuration and dynamic indication of access link beam indication in a symbol, dynamic indication overrides semi-static/semi-persistent configuration.

NCR behavior of conflict between access link beam indications

For the above explained conflict case between dynamic indication and periodic/semi-persistent indication, i.e., the case where different forwarding resources indicate different beams for overlapping time resources, detailed discussion is required. It can be divided into two parts; for the first, different beams are indicated to identical time resources by different forwarding resources. That is, time resource of each conflicted forwarding resources has same slot offset, symbol offset and duration with same reference SCS and periodicity, if configured. In such case, NCR can determine only single beam configuration is valid according to priority rule between beam configurations.

The latter case to be discussed is more complicated; different beams are indicated to different but partially overlapped time resources by different forwarding resources. For example, when the first beam configuration indicates {beam index k, time resource α}, {beam index l, time resource β} and the second beam configuration indicates {beam index x, time resource π}, only part of time resources, time resource π and each part of time resource α and time resource β, respectively, can overlap. It is illustrated in Figure 8.

Figure 8. Example of partially overlapped time resource between beam configurations

Although it is quite a complicated situation, the behavior of NCR should be defined clearly on those partially overlapped time resources and remained time duration due to the overlap. If not, NCR applies a certain beam according to implementation or even can be turned off, which means that UEs served by NCR cannot expect stable beam from NCR, concludes with performance degradation or increment of signaling overhead to such UEs. In that sense, it is investigated in detail by dividing cases. If a conflict occurred between the high priority beam configuration and the low priority beam configuration, and the two time resources partially overlapped. Among the time intervals indicated by the time resources of the low priority beam configuration, for the intervals overlapping with the time resources of the high priority beam configuration, it is natural that NCR applies beam indicated by high priority beam configuration. On the other hand, among the time intervals indicated by the time resources of the low priority beam configuration, for the time intervals excluding the intervals overlapping with the time resources of the high priority beam configuration, the behaviour of NCR should be defined.

To this end, the interpretation for time resource of beam configuration should be carefully investigated. It can be interpreted in two ways; since the NCR does not aware of which signal/channel to be relayed, the time resource is only for indication of certain amount of the time duration. Based on such interpretation, NCR applies beam indicated by lower priority beam configuration for the certain time duration. It is straightforward method but it cannot be applied to such cases when reference SCS between time resources is different.

And for the second, since NCR relays actual transmitted signal/channel from gNB or UEs, the concept of time resource is identical to any time resource configuration of signal/channel. Based on such interpretation, same rule can be applied; NCR applies beam indicated by lower priority beam configuration for the certain time duration. On the other hand, it can be considered that NCR does not apply any beam indicated by beam configuration, i.e., NCR can be expected to be OFF. It is aligned to some of rules for the conventional UE, i.e., transmit of nominal repetition in PUSCH repetition type B is dropped when the nominal repetition consist of only 1 symbol, or transmit of PUSCH is dropped when collision between PUSCH repetition and PUCCH repetition occurs. It might seems to be over complicated but it can be a simple solution to cover corner cases such as less than a symbol is left due to different reference SCS between time resources.

Proposal 11: If a conflict occurred between the high priority beam configuration and the low priority beam configuration, and the two time resources partially overlapped, the behavior of NCR should be considered for;

- Among the time intervals indicated by the time resources of the low priority beam configuration, for the intervals overlapping with the time resources of the high priority beam configuration,

  - NCR applies beam indicated by high priority beam configuration

- Among the time intervals indicated by the time resources of the low priority beam configuration, for the time intervals excluding the intervals overlapping with the time resources of the high priority beam configuration,

  - Alt. 1) NCR applies beam indicated by lower priority beam configuration.

  - Alt. 2) NCR is expected to be OFF.

Backhaul link beam indication

In this section, details for backhaul link beam indication are discussed. Regarding backhaul link beam indication, following is agreed in previous meeting.

Agreement The semi-static beam indication for backhaul link is supported as: - If the beam indication framework in Rel-15 is used for NCR-MT - The DL beam is indicated by MAC CE to select one of TCI state ID from the RRC-configured list of beams for C-link - The UL beam is indicated by SRI on C-link via MAC CE. - If the beam indication framework in Rel-17 is used for NCR-MT - The DL and UL beam are indicated by MAC CE to select one of TCI state ID from the RRC-configured list of beams for C-link Agreement The following pre-defined rules are applied to determine the beam for backhaul link: - In the time domain resource with simultaneous downlink reception or uplink transmission in C-link and backhaul link, the beam of backhaul link is the same as the beam of C-link regardless whether there is beam indicated by the dedicated signal for backhaul link. - In the time domain resource without simultaneous downlink reception or uplink transmission in C-link and backhaul link, if the NCR does not support capability with the new signalling for backhaul beam indication or if no beam is indicated for backhaul link by the dedicated signal, - When Rel-15/16 beam indication framework is used for C-link, - The beam determined by QCL assumption for CORESET with the lowest ID and spatial relationship for PUCCH with lowest PUCCH resource ID in the C-link is applied for the DL and UL of backhaul link, respectively. - When Rel-17 beam indication framework (i.e., unified TCI framework) is used for C-link, the indicated unified TCI for C-link DL and UL is applied for the DL and UL of backhaul link, respectively. Otherwise, the beam indicated by the dedicated signalling is applied for backhaul link.

Simultaneous operation in C-link and backhaul link

According to agreements have been made, backhaul link beam of NCR-Fwd follows that of NCR-MT when simultaneous operation. If not, backhaul link beam of NCR-Fwd is indicated via MAC-CE. Besides, it has not been deeply discussed interpretation on time duration of simultaneous operation based on definition of it, further discussion is required. First, there is no ambiguity for NCR-Fwd ON/OFF state because it is indicated directly or through beam configuration. On the other hand, a common understanding of the time period in which NCR-MT operates is required since it can be different. In that perspective, it is separately discussed for UL and DL.

Firstly, UL of NCR-MT is discussed. There can be two ways to determine the time resources in which the NCR-MT operates in the UL. One way is to consider only for the time resource that actual uplink signal/channel is transmitted by NCR-MT as the time resources in which NCR-MT operates. In this case, NCR-MT's scheduled UL Tx is known to the gNB, so there is no misunderstanding between the gNB and NCR-MT. However, when NCR-MT performs CBRA (contention based random access), the gNB can only know whether NCR-MT actually performed UL Tx when the gNB successfully receives the transmission. The gNB cannot know how many UL Tx were actually performed by NCR-MT. Additionally, dedicated RO (RACH Occasion) for NCR, such as eIAB, has not been agreed upon, so NCR's RO will be shared with UE's RO. The beam of NCR-Fwd in the time resources can change depending on whether the beam follows the indication or is configured by MAC-CE, which greatly affects UE's CBRA. To eliminate such ambiguity, it would be appropriate to define the time resources in which NCR-MT is expected to perform uplink transmission as the UL operate time resources of NCR-MT.

Next, DL of NCR-MT is considered. Like UL, there is no ambiguity in scheduled DL Rx, but if NCR-MT distinguishes between measurement or monitoring and reception, it may define the DL operation of the C-link or define the DL operation segment depending on the success of the DL reception. In this case, the gNB and NCR cannot have a common understanding of the backhaul link beam setting. For example, NCR-MT may measure SSBs transmitted by the gNB, even if it does not actually receive them. If the beam used to relay the SSB measured by NCR-MT is different from the beam used to relay the received SSB, NCR-MT's effective channel may change from the perspective of UE served by NCR. Therefore, for the same reason, the time resources in which NCR-MT receives the signal for measurement/monitoring over the C-link should be defined as the DL operate time resources.

Proposal 12. For the determination of simultaneous operation of C-link and Fwd-link,

- The C-link is considered to be operate in the time resources in which NCR-MT is expected to perform UL Tx (at least including RO)

- The C-link is considered to be operate in the time resources in which NCR-MT is expected to perform DL Rx and measurement/monitoring (at least including SSB)

Further details on backhaul link beam indication

It is agreed that DL beam of backhaul link beam is indicated via TCI state ID and there was a discussion that whether the TCI state ID is BWP-specific parameter or not. Although it is not simple to investigate, it is our understanding that TCI state ID is BWP-specific parameter. In section 5.1.5 of TS 38.214, following is described; “When a set of TCI state IDs are activated for a set of CCs/DL BWPs and if applicable, for a set of CCs/UL BWPs, where the applicable list of CCs is determined by the indicated CC in the activation command, the same set of TCI state IDs are applied for all DL and/or UL BWPs in the indicated CCs.”

Also, maximum number of TCI state ID is 128 according to TS 38.331. In addition, among the “collection of updated higher layer parameter list” discussed in RAN1#108e to organize the RRC parameters in the Rel-17 maintenance phase, some of the TCI state related parameters of MIMO are extracted in Table 1.

Table 1. Part of RRC parameter discussed in Rel-17 MIMO maintenance

| WI code | Sub-feature group | RAN1 specification | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_feMIMO-Core | MultiBeam | 38.214 Clause 5.1.5 | maxNrofTCI-States_r17 | New | Number of TCI states: for joint DL/UL TCI and (in case of separate DL/UL TCI) DL TCI | 128 | Per UE per cell per BWP Multiplicity and type constraint definitions (not RRC par, not configurable) | UE-specific |

From the table, it can be known that maximum number of TCI state is 128 per BWP, which implies TCI state ID is BWP specific parameter. In this context, it can be inferred that the TCI state ID is a parameter that is configured per UE, per cell, and per BWP. Therefore, when indicating the backhaul link beam based on the TCI state ID, the BWP ID must also be determined. There may be various ways to determine this BWP ID, but as this is related to the setting of the backhaul link beam, reliability is the most important factor. In other words, the gNB and NCR should always have a common understanding of the BWP ID for indication of the TCI state ID, and it would be appropriate to use the default BWP as a reference. This should apply to both the Rel-15/16 TCI state ID, where the DL beam is indicated, and the Rel-17 unified TCI state ID, where both DL and UL beams are indicated.

Proposal 13. For backhaul link beam indication based on Rel-15/16 TCI state ID and Rel-17 beam indication framework, default BWP is used for TCI state ID indication.

In the context of backhaul link beam indication, there are two types of NCRs to be considered: those that cannot receive the backhaul link beam indication (NCRs that do not support adaptive beam for backhaul link), and those that can receive the backhaul link beam indication (NCRs that support adaptive beam for backhaul link). Each cases are discussed separately.

First, for NCRs that cannot receive the backhaul link beam indication, the pre-defined rules for determination of the backhaul link beam will be applied based on whether the NCR is performing simultaneous operation or not, so there is no remained ambiguity in setting the backhaul link beam for such NCRs as long as there is a clear common understanding of the time duration for simultaneous operation between gNB and NCR.

Second, for NCRs that can receive the backhaul link beam indication, the backhaul link beam to be applied by the NCR depends on two factors; whether it is performing simultaneous operation and whether it has received the backhaul link beam indication. Again, as long as there is a clear understanding of the time duration for simultaneous operation, the appropriate backhaul link beam can be determined for each case. However, if these cases occur sequentially, there may be ambiguity in the NCR's backhaul link beam decision.

For example, if an NCR that can receive the backhaul link beam indication is performing non-simultaneous operation and receives the backhaul link beam indication, it will apply the specified beam for the backhaul link. Then, if the NCR starts performing simultaneous operation, the C-link beam will be applied for the backhaul link. However, if the NCR reverts to non-simultaneous operation, it is unclear whether the NCR should apply the previously indicated beam for previous non-simultaneous operation or apply beam determined by pre-defined rule for non-simultaneous operation. This is illustrated in Figure 9.

Figure 9. Example of backhaul link beam determination of NCR

In other words, if C-link and Fwd-link perform non-simultaneous operation, simultaneous operation, and non-simultaneous operation sequentially, there can be two possible cases for NCR's determination of the backhaul link beam during the second non-simultaneous operation period. The NCR can consider that the DL/UL beam is indicated during the time interval, case 1 in Figure 9, or not indicated, case 2 in Figure 9. As time resources or valid time duration for indicating the backhaul link beam were not discussed, NCR's behavior according to the agreement is understood to be case 1. However, this method makes it impossible for NCR to determine the backhaul link beam according to the pre-defined rule once it has been indicated. Additionally, indication of the backhaul link beam is based on the TCI state ID and SRI, which may become outdated if the gNB does not update them according to channel changes. Therefore, instead of it, it is preferable for NCR-MT to consider that the beam indicated to apply for the backhaul link during the non-simultaneous operation period is valid until NCR's backhaul link beam changes according to the pre-defined rule. In this case, there is no remained concern that the beam will become outdated after simultaneous operation and also saves continuous signaling overhead to prevent outdated beam for backhaul link.

Proposal 14. Semi-static beam indication for backhaul link is valid until backhaul link beam is changed due to pre-defined rule.

ON-OFF indication

Before getting start of discussion regarding ON-OFF indication, the default behavior of NCR should be defined clearly. That is, following agreement made in RAN1#110 which was study item phase needs to be captured in specification and considered to be baseline.

Agreement The NCR-Fwd is always expected to be “OFF” unless otherwise explicitly or implicitly indicated by gNB. - Note-1: This applies to the case regardless of the RRC state of NCR-MT. - Note-2: Indication (e.g., received when NCR-MT in RRC-connected) or DRX state of NCR-MT to control the ON-OFF behaviour of NCR-Fwd when the NCR-MT is in RRC-idle/inactive is not precluded. The above is not meant to imply any signalling design for NCR-Fwd ON-OFF.

Proposal 15. Capture following agreement made in SI phase:

- The NCR-Fwd is always expected to be “OFF” unless otherwise explicitly or implicitly indicated by gNB.

Other than that, remaining issue for ON-OFF indication would be whether and how to indicate OFF state to NCR and NCR behaviour on ON-OFF operation when NCR operates FDD. As FL commented in previous meeting, FDD operation of NCR is under the scope to be deal with. There would be not much difference between TDD and FDD in terms of beam indication framework, however ON/OFF indication of TDD and FDD can be different since the frequency of DL and UL is different, ON/OFF indication can be separately indicated for DL and UL. Based on those, ON state indication, OFF state indication is discussed in section 4.1, 4.2 respectively.

ON state indication

In terms of ON state indication of NCR-Fwd, three alternatives were made in the last meeting [2].

Agreement For FR2, the “ON” state of NCR-Fwd is indicated: - Alt-2: Implicit indication via the beam indication (i.e., if there is beam indication, the NCR is assumed to be ON over the indicated time domain resource associated with corresponding beam(s)) Agreement For FR1, the “ON” state of NCR-Fwd is indicated: Alt-2: Indication via the beam indication (i.e., if there is beam indication, the NCR is assumed to be ON over the indicated time domain resource associated with corresponding beam(s)) - When there is only one beam, the sole purpose of the beam indication is for indicating “ON” state of NCR-Fwd

The beam indication in agreement for “ON” indication, i.e., “if there is beam indication, the NCR is assumed to be ON over the indicated time domain resource associated with corresponding beam(s)”, should be remained to be access link beam indication for prevent further spec impact since time resource for backhaul link beam indication has not been discussed. It needs to be clarified.

In terms of NCR operating FDD band, separate “ON” indication on DL and UL is desirable in terms of interference minimization and power saving. However according to the discussion so far, it has been agreed to assume beam correspondence without distinguishing between DL and UL beams in the indication of access link beams, so it will be difficult to indicate ON for each link direction. Therefore, for paired spectrum case, both DL and UL are assumed to be ON over the indicated time domain resource associated with corresponding beam(s).

Proposal 16: In case of FDD, both DL and UL of NCR-Fwd are assumed to be ON over the time domain resource indicated by access link beam indication.

OFF state indication

OFF state indication for NCR operating TDD

First of all, let us consider the NCR operating in TDD bands. As mentioned earlier, the NCR-Fwd is always expected to be OFF unless otherwise explicitly or implicitly indicated by the gNB [3]. Therefore, indicating the OFF state for resources where the ON/OFF state is not explicitly indicated is redundant. From this perspective, the configuration of periodic/semi-persistent OFF states for NCR operating in TDD bands is considered further optimization.

On the other hand, aperiodic OFF state indication is necessary. This is not only for interference suppression to other networks but also for canceling the pre-configured access link beam indication and indicating the OFF state considering it is agreed that determining the ON state is based on beam indication. There are two main methods to indicate explicit OFF indication for aperiodic OFF state indication. The first is to define new signaling for dynamic OFF indication, and the second is to reuse the existing signaling structure, such as access link beam indication signaling.

When considering a dedicated signaling structure for dynamic OFF indication, a time resource is required to indicate duration operating OFF. However, since the NCR-MT already has various time resources for access link indication, reusing them would be appropriate to reduce signaling overhead and allow for flexible OFF indication. Therefore, it would be appropriate to base it on the beam indication framework. One possible way is to reserve a specific code point or beam index in the beam information field for OFF state indication, as shown in Table 2. In this example, when the number of candidate beams for access link beam indication is N, the beam information field of the dynamic access link beam indication DCI, indicating the OFF state based on the beam indication framework, has a bit width of log2(N+1).

Table 2. Example of Beam/OFF indication field for TDD NCR

| value | Description |
| --- | --- |
| 0 | OFF |
| 1 | Physical access link beam index n0 |
| 2 | Physical access link beam index n1 |
| … | … |
| N | Physical access link beam index N-1 |

In this case, each beam/OFF indication can correspond to one time resource indication. To do this, for example, a pair of beam/OFF indication fields and time resource indication fields could exist, so beam/OFF indication 1 could be applied to the time resource indicated by time resource indication 1, and beam/OFF indication 2 could be applied to the time resource indicated by time resource indication 2, as shown in Figure 10.

Figure 10. DCI fields for OFF state indication combined with Beam indication

Proposal 17: OFF state indication is combined to beam indication, i.e., a certain value of beam information field is dedicated for OFF state indication.

OFF state indication for NCR operating FDD

TDD literally means that DL and UL are not performed at the same time, so DL beam and UL beam are distinguished in the time domain. That is, the DL beam and UL beam are distinguished based on the configured time, and there is no need to specify them separately. In contrast, in FDD, DL and UL are performed at different frequencies at the same time. Against this background, it can also be considered to indicate separate OFF for DL and UL in FDD for power saving similar to the UE performing HD operation in the FDD band for UE power saving (Rel-17 RedCap HD-FDD UE), or for the interference minimization.

On the other hand, even for FDD-based NCR, in the case of paired spectrum, both DL and UL are assumed to be ON over the indicated time domain resource associated with corresponding beam(s). Therefore, it would be appropriate to indicate the ON state as it is, but to perform the OFF state indication for DL and UL separately. In other words, for NCR operating in FDD, if the aperiodic indication by beam indication field is indicated as three specific values, they represent DL OFF state, UL OFF state, and DL+UL OFF state, respectively.

Table 3. Example of Beam/OFF indication field for FDD NCR

| value | Description |
| --- | --- |
| 0 | DL OFF |
| 1 | UL OFF |
| 2 | DL & UL OFF |
| 3 | Beam index 0 |
| 4 | Beam index 1 |
| … | … |
| N-2 | Beam index N-1 |

As in TDD, if the number of candidate beams for access link beam indication is N, the beam information field of the dynamic access link beam indication DCI indicating the OFF state of FDD NCR is of bit-width log2(N+3). The example of DCI field is represented in Table 3.

Proposal 18: For NCR operating FDD, DL OFF state, UL OFF state and DL & UL OFF state are combined to beam indication, i.e., three number of certain values of beam information field are dedicated for DL OFF state, UL OFF state and DL & UL OFF state.

Others

Forwarding frequency resource applying side control information

Regarding the frequency resource in which the NCR is operated, it seems necessary to clarify whether the NCR-Fwd can perform forwarding only for a single forwarding band or whether it can perform a forwarding operation for multiple forwarding bands.

If NCR-Fwd can perform a forwarding operation for multiple forwarding bands, independent side control information (e.g., beam information, ON-OFF information) for each forwarding band may be necessary. In this case, it is necessary to discuss how NCR-MT receives side control information for each forwarding band from serving cell(s). In order for the NCR-MT to receive forwarding band specific side control information, the NCR-MT can receive side control information for a forwarding band in a serving cell which is located inband with the forwarding band. Otherwise, the NCR-MT may receive side control information for multiple forwarding bands from a serving cell.

This method of indicating side control information for multiple forwarding bands can be extended as follows; when NCR-Fwd supports one forwarding band, side control information is differently applied for each frequency resource within the forwarding band (e.g., per carrier or carrier group). To this end, a field indicating the index value of the forwarding frequency resource should be included in the DCI that delivers side control information.

Proposal 19: Forwarding frequency resource (e.g., carrier index, or carrier-group index) to which side control information is applied can be indicated with side control information.

Capability reporting

According to the TR 38.867, the following assumption is considered as baseline for the timing of NCR.

- The DL receive timing of the NCR-Fwd is aligned with the DL receive timing of the NCR-MT.

- The UL transmit timing of the NCR-Fwd is aligned with the UL transmit timing of the NCR-MT.

- The DL transmit timing of the NCR-Fwd is delayed after the DL receive timing of the NCR-MT (or the NCR-Fwd) by an internal delay;

- The UL receive timing of the NCR-Fwd is advanced before the UL transmit timing of the NCR-MT (or the NCR-Fwd) by an internal delay.

During the study item phase, there was a discussion about whether it is necessary to report the value of internal delay applied by NCR-Fwd to the network. If the amount of internal delay exceeds the CP length, the internal delay information is required for the gNB to configure appropriate guard intervals for DL-UL switching and timing adjustment. Therefore, it is necessary that the value of internal delay is reported to the network as NCR capability.

Proposal 20: The value of internal delay of NCR-Fwd can be reported to the network as NCR capability.

Summary

In this contribution, we have discussed on potential methods and enhancements for access link beam indication, backhaul link beam indication and ON-OFF indication. From the discussion, we obtained following proposals:

Proposal 1: For time resource of access link beam indication, multiple slots indication based on common SLIV should be supported.

Proposal 2: It is necessary to consider that the time resource is composed of non-continuous slots.

Proposal 3: The time resource of access link beam indication is defined by:

- Alt. 1) slot offset, SLIV (symbol offset and symbol duration), number of slot(s)

- Alt. 2) slot offset list, SLIV (symbol offset and symbol duration)

Proposal 4: NCR-MT does not expect conflict between/within periodic configuration(s) of access link beam indication.

Proposal 5: Multiple configuration is activated/deactivated via single indication for semi-persistent indication.

Proposal 6: Single configuration can contain multiple forwarding resources and reference SCS and periodicity can be shared within some of forwarding resources.

Proposal 7: The beam information field size and number of beam information field of DCI for aperiodic access link beam configuration is pre-configured by gNB to NCR in advance.

Proposal 8: For time resource of aperiodic configuration, Tmax = Lmax is supported.

Proposal 9: In the case where only L (< Lmax) beams are indicated in the aperiodic beam indication, it is necessary to determine which of the Lmax beam information fields are not used for beam indication.

- If the time resource field indicates a specific value, the associated beam information field can be considered as not used for beam indication.

Proposal 10: In case of conflict between semi-static/semi-persistent configuration and dynamic indication of access link beam indication in a symbol, dynamic indication overrides semi-static/semi-persistent configuration.

Proposal 11: If a conflict occurred between the high priority beam configuration and the low priority beam configuration, and the two time resources partially overlapped, the behavior of NCR should be considered for;

- Among the time intervals indicated by the time resources of the low priority beam configuration, for the intervals overlapping with the time resources of the high priority beam configuration,

  - NCR applies beam indicated by high priority beam configuration

- Among the time intervals indicated by the time resources of the low priority beam configuration, for the time intervals excluding the intervals overlapping with the time resources of the high priority beam configuration,

  - Alt. 1) NCR applies beam indicated by lower priority beam configuration.

  - Alt. 2) NCR is expected to be OFF.

Proposal 12. For the determination of simultaneous operation of C-link and Fwd-link,

- The C-link is considered to be operate in the time resources in which NCR-MT is expected to perform UL Tx (at least including RO)

- The C-link is considered to be operate in the time resources in which NCR-MT is expected to perform DL Rx and measurement/monitoring (at least including SSB)

Proposal 13. For backhaul link beam indication based on Rel-15/16 TCI state ID and Rel-17 beam indication framework, default BWP is used for TCI state ID indication.

Proposal 14. Semi-static beam indication for backhaul link is valid until backhaul link beam is changed due to pre-defined rule.

Proposal 15. Capture following agreement made in SI phase:

- The NCR-Fwd is always expected to be “OFF” unless otherwise explicitly or implicitly indicated by gNB.

Proposal 16: In case of FDD, both DL and UL of NCR-Fwd are assumed to be ON over the time domain resource indicated by access link beam indication.

Proposal 17: OFF state indication is combined to beam indication, i.e., a certain value of beam information field is dedicated for OFF state indication.

Proposal 18: For NCR operating FDD, DL OFF state, UL OFF state and DL & UL OFF state are combined to beam indication, i.e., three number of certain values of beam information field are dedicated for DL OFF state, UL OFF state and DL & UL OFF state.

Proposal 19: Forwarding frequency resource (e.g., carrier index, or carrier-group index) to which side control information is applied can be indicated with side control information.

Proposal 20: The value of internal delay of NCR-Fwd can be reported to the network as NCR capability.

Reference

- ZTE Corporation, “New WID on NR network-controlled repeaters,” RP-222673, e-Meeting, September 12 – 16, 2022

- Chair’s note in RAN1#111 meeting

- 3GPP TR 38.867 V 18.0.0