3GPP TSG RAN WG1 #112	    			            R1-2301287

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.12.1

Source: 	Samsung

Title: 	On L1 enhancements for inter-cell beam management

Document for:	Discussion and Decision

# Introduction

In earlier NR releases, inter-cell mobility (handover) is triggered by L3 measurements and involves RRC signaling for Reconfiguration with Synchronization, which involves a complete L1 and L2 reset. This leads to longer latency, and larger overhead. To streamline the handover process L1/L2-based mobility is being introduced in the Rel-18 [1], with the following objective:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction:

- Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3]

- Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1]

- L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2]

  - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet

- Timing Advance management [RAN1, RAN2]

- CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3]

Note 2: FR2 specific enhancements are not precluded, if any.

Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios:

    - Standalone, CA and NR-DC case with serving cell change within one CG

    - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected)

    - Both intra-frequency and inter-frequency

    - Both FR1 and FR2

    - Source and target cells may be synchronized or non-synchronized

In this contribution we present Samsung’s views on L1 enhancements to inter-cell beam management to support L1/L2 mobility.

# Discussion

## Enhancements to L1 measurements and reporting

In Rel-17, for inter-cell beam management (ICBM), it has been agreed to support inter-cell intra-frequency SSB L1-RSRP measurement. RAN4 defined inter-cell intra-frequency SSB L1-RSRP measurement as [2]:

  - For SSB L1-RSRP measurement, follow the definition of L3 measurement:

    - A measurement is defined as a SSB based intra-frequency L1 measurement provided the center frequency and SCS of the SSB of the neighbor cell is the same as SSB of the serving cell indicated in ServingCellConfigCommon

- Note: RAN4 will revisit the definition based on RAN1/2 conclusion.

RAN1 can follow the same definition as RAN4. In addition to supporting L1 intra-frequency measurement, RAN1 has agreed in RAN1#111 to also support L1 inter-frequency measurement.

Agreement

For Rel-18 LTM, L1 inter-frequency measurement is supported from RAN1 point of view.

For SSB-based measurements, if the centre frequency or the sub-carrier spacing of the SSB of the non-serving cell is different from that of the serving cell, this is considered L1 inter-frequency measurement scenario.

Observation 1: For Rel-18 LTM, for SSB-based L1-RSRP measurements, the measurement is defined as inter-cell intra-frequency L1 measurement when the centre frequency and the sub-carrier spacing of the SSB of the non-serving cell is the same as those of the SSB of the serving cell, otherwise the measurement is considered L1 inter-frequency measurement.

For CSI-RS-based measurements, if supported, RAN4 has not agreed on the definition of scenarios corresponding to intra-frequency measurement and inter-frequency measurement. Those scenarios should be left to RAN4 to decide.

Observation 2: For Rel-18 LTM, if CSI-RS-based measurements are supported, it is up to RAN4 to decide the corresponding intra-frequency and inter-frequency scenarios.

## Measurement Reference Signal

In Rel-17, only SSB resources were used as measurement RS for L1 inter-cell measurements. In RAN1#111, it was agreed to support SSB resources as measurement for L1/L2 mobility in Rel-18 for both L1 intra-frequency measurement and L1 inter-frequency measurement, and to further study the support of CSI-RS.

Agreement

- For candidate cell measurement for Rel-18 LTM,

  - SSB based L1-RSRP is supported for intra-frequency measurement

  - SSB based L1-RSRP is supported for inter-frequency measurement from RAN1 point of view

  - FFS: L1-SINR, CSI-RS based L1-RSRP

In Rel-18, we can consider CSI-RS as an additional reference signal for the beam measurement report. Rel-15, defined a CSI-RS resource for mobility that is used for handover purposes, such CSI-RS resource can be considered for handover purposes.

Proposal 1: For enhancements to L1 measurement and reporting for L1/L2 mobility, legacy RS types supporting mobility management (CSI-RS for Mobility) should be considered as source RS for beam measurement and reporting.

Additionally, for ICBM as defined in Rel-17, CSI-RS resources are configured that are associated through a QCL relation with an SSB associated with the PCI of a cell that is different from the PCI of the serving cell. We can further study whether these CSI-RS resources can also be used for beam measurement reporting. These CSI-RS resources can include CSI-RS for tracking, or CSI-RS for beam management.

Proposal 2: For enhancements to L1 measurement and reporting for L1/L2 mobility, study the support of TRS and CSI-RS for beam management associated through a QCL relation to an SSB associated with a PCI of a cell that is different from the PCI of the serving cell as a source RS for beam measurement and reporting.

Another open point from RAN1#110b-e is about the use of uplink measurements for L1/L2 mobility. UL measurements are performed by the gNB and hence can be left to network implementation.

Observation 3: Uplink measurements for L1/L2 mobility are performed by gNB and hence be left for network implementation.

## Measurement Metric

In RAN1#110b-e and RAN1#111, the measurement metric was discussed. It was agreed (as described in the aforementioned agreement) to support L1 RSRP for L1 intra-frequency measurement and L1 inter-frequency measurement, and to further study the support of L1-SINR measurement. RSRP measurement provides a reasonable metric for making a handover decision. A cell can be selected as a target cell for handover if it has a better RSRP than the RSPR of the current serving for example by a certain threshold. RSRP measurements don’t take into account the interference in cell. In some cases, the target cell might have a sufficiently better RSRP than the serving cell, but has significantly higher interference, which would lead to a lower signal quality if handover were to take place to the target cell. In those cases, it might seem that using an SINR-type measurement is a better metric than the L1-RSRP metric or using an SINR-type measurement in addition to the L1-RSRP. However, the L1-SINR, which is the ratio between L1-signal power and the interference of a slot can be quite a noisy measurement that can vary significantly from one measurement instance to another. There could be several ways to overcome variability of the L1-SINR:

- A first alternative is taking an average (e.g., exponential average) of the L1-SINR across several slots. The longer the averaging window the more stable the L1-SINR. However, this comes at an expense of a longer latency.

- A second alternative is calculating a longer term average (e.g., exponential average) of the L1 interference over a window with a certain size. The long term average interface is then used to divide L1-RSRP to get an L1-SINR measurement that can be used for handover purposes, i.e., L1-SINR = (L1-RSRP/Long term average interference). The L1-RSRP is measurement in one measurement instance (i.e., without averaging), hence the latency of calculating this metric can be short.

Proposal 3: Further study the necessity, reliability and latency of L1-SINR measurement metric for L1/L2 mobility.

## Measurement Framework and Configuration

Rel-17 defined the beam reporting framework for ICBM, where a beam report includes K SSBRI-RSRP pairs, the RSRP of the first pair is a 7-bit absolute value, and the remaining K-1 pairs have a 4-bit differential RSRP. K can be up to 4. The SSB-RI can correspond to an SSB of a non-serving cell based on the corresponding additionPCIIndex associated with the SSB.

Agreement (RAN1#107-e)

On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP, Rel-15 L1-RSRP reporting format is reused for all SSBRI-RSRP pairs in one L1-RSRP reporting instance, i.e. for K>1, (K-1) 4-bit differential L1-RSRP(s) calculated relative to the reference (absolute) 7-bit L1-RSRP

The same framework can be used for L1 intra-frequency measurement as well as L1 inter-frequency. The L1 measurement report can be designed to include both L1 intra-frequency measurements and L1 inter-frequency measurement, or two separate L1 measurement reports can be provided, one for L1 intra-frequency measurements and the other for L1 inter-frequency measurements. A potential advantage of having separate reports is that the timing of inter frequency measurement and intra-frequency measurement can be different based on the time of the measurement gap and SMTC. Such a decision can be made based on further input from RAN4.

Proposal 4: Study the following alternatives for the L1 measurement report taking into account the timing of respective measurements:

- Alternative 1: A common measurement report for L1 intra-frequency measurement and L1 inter-frequency measurement.

- Alternative 2: Separate measurement reports for L1 intra-frequency measurement and L1 inter-frequency measurement.

In RAN1#111, the configuration framework for L1 measurements was discussed, but not agreement was reached, several options were considered based on the latest feature lead summary:

##### [FL proposal 1-7-v3]

- For Rel-18 LTM, further study the following structure for L1 measurement configurations.

  - Option-1: Based on CSI measurement configuration specified in Rel-17 ICBM

    - CSI-MeasConfig for serving cell and candidate cell(s), which requires inter-DU coordination

    - For inter-frequency, at least the frequency information, SMTC or measurement gap (MG) with candidate cell are additionally introduced

  - Option-2: Introduce an independent a measurement configuration for candidate cell(s) [and serving cell] from serving cell configurations, which, and the measurement configuration is decoupled with serving cell configuration.Introduce an independent measurement configuration for all candidate cell(s) and serving cell(s) (FL note: Suggested by DOCOMO and their intention is to enable reporting for candidate cells with that for serving cell ) from serving cell configurations, which is decoupled with serving cell configuration.

    - L1 measurement resource set can be configured outside candidate cell configurations (i.e. ServingCellConfig or CellGroupConfig)

    - Different CSI-MeasCofig is configured for candidate cell

  - Option-3: ReUse measurement configuration for each candidate cell

    - L1 measurement resource set can be configured inside candidate cell configurations (i.e. ServingCellConfig or CellGroupConfig)

  - Option-4: Do not include RS information or cell information in measurement configurations

    - For intra-frequency, neither SSB/RS indices nor PCI is configured.

    - For inter-frequency, neither SSB/RS indices nor PCI is configured, but frequency information is configured

Rel-18 LTM can be considered as an extension of the Rel-17 inter-cell beam management (ICBM). In Rel-17, additional PCIs and corresponding SSBs are configured by IE SSB-MTC-AdditionalPCI-r17.

SSB-MTC-AdditionalPCI-r17 ::= SEQUENCE {
         additionalPCIIndex-r17 	AdditionalPCIIndex-r17,
         additionalPCI-r17 			PhysCellId,
         periodicity-r17 				ENUMERATED { ms5, ms10, ms20, ms40, ms80,                  ms160, spare2, spare1 },
         ssb-PositionsInBurst-r17 CHOICE {
                shortBitmap 			     BIT STRING (SIZE (4)),
                mediumBitmap 		     BIT STRING (SIZE (8)),
                longBitmap 			     BIT STRING (SIZE (64))
         },
         ss-PBCH-BlockPower-r17 INTEGER (-60..50)

}

This IE can be extended to include the PCIs/SSBs/ARFCN of inter-frequency cells. In Rel-17, the maximum number of Additional PCIs is 7 (maxNrofAdditionalPCI-r17 ::=), this can be increased to 15 or 31 in Rel-18 LTM to account for cells on other frequencies.

Proposal 5: For Rel-18 LTM measurement configuration, reuse the Rel-17 ICBM measurement configuration framework, with the following changes:

- Include the ARFCN of the cell in the SSB-MTC-AdditionalPCI information element.

- Increase the maximum number of additional PCIs (FFS maximum value).

## Measurement Report Container

Another aspect of beam measurement reporting is the container of the beam measurement report. In Rel-17, the container the L1 inter-cell measurement report is UCI. The Rel-17 beam measurement report contains K pairs of SSBRI-RSRP or CRI-RSRP. The first pair includes a 7-bit absolute RSRP, and remaining K-1 pairs includes a 4-bit differential RSRP. Where K can be up to 4. The size of report can be up to: (6 (field for SSBRI or CRI) + 7 (field for first L1-metric)+2(Capability Index)) + (4(value of K) – 1) * (6 (field for SSBRI or CRI) + 4 (field for remaining L1-metric) +2(Capability Index)) = 51 bits. If the size of the report were to increase for L1/L2 mobility for example having K=6 and 7 bits for the SSBRI/CRI field, the size of the report would be less than 100 bits, which can be carried by a UCI container on PUCCH or PUSCH.

In RAN1#111, it was agreed to at least support UCI reporting on PUSCH based on semi-persistent and aperiodic based reporting. UCI reporting on PUCCH was left as an open point.

Agreement

- For gNB scheduled L1 measurement report for Rel-18 LTM, report as UCI is supported

  - Semi-persistent report on PUSCH, and aperiodic report on PUSCH are supported

    - FFS: periodic and semi-persistent PUCCH

  - In a single report instance, report for serving cell and candidate cell(s) for intra-frequency and/or inter-frequency can be included.

We see a benefit in supporting UCI reporting on PUCCH at least for the periodic case as this can provide lower latency without having the network trigger the report.

Proposal 6: For Rel-18 LTM, the gNB scheduled L1 measurement report can be transmitted using at least periodic PUCCH.

A further enhancement to the Rel-17 inter-cell measurement report is to have the number of actual reported groups be determined by the UE. For this case, the two-part UCI structure (specified in Rel-15) can be used to send the beam measurement report. Here, part 1 is of a fixed payload size, which, e.g., includes the beam measurement(s) for the serving cell, and used to indicate the size of the payload in part 2, e.g., by indicating the number of remaining pairs of SSBRI-RSRP or CRI-RSRP reported in part 2; part 2 includes the remaining pairs of SSBRI-RSRP or CRI-RSRP. As part 1 only contains the minimum/basic information needed for the inter-cell operation – e.g., the serving cell beam and/or the best beam, low-rate code can be applied for part 1, which in turn, can improve its detection probability. This is in contrast to including all beam measurements in one report (or a single-part UCI); for this case, if the single-part UCI is dropped, the inter-cell beam management performance would be significantly degraded especially when both serving and non-serving cells are mixed in one report and the number of reporting cells/beams are relatively large. Furthermore, with the two-part UCI structure, the UE can autonomously decide the report size for various channel conditions.

Proposal 7: Support reporting a variable number of SSBRI (or CRI)/RSRP/Capability index groups, wherein a UE reports in a single reporting instance a two-part beam report using the two-part UCI structure.

- Part 1 includes the beam measurement(s) for the serving cell and indicates the size of part 2.

- Part 2 includes the remaining beam measurement(s).

## UE Triggered Measurement Report

In RAN1#111, the following agreement on UE-event triggered reporting was made.

Agreement

- For L1 measurement report for Rel-18 L1/L2 mobility, if UE event triggered report for L1 measurement is supported based on further study

    - At least the following aspects may be considered

      - How to define UE event and exact definition of events,

      - Report container

      - Resource allocation/assignment for UE event triggered report

      - Necessity of indication to gNB when the condition UE event is met, and how

      - Necessity to define the condition to start/stop the reporting,

      - Contents of the report/reporting format, PCI, RS ID, measurement result etc.

      - The interaction with filtered L1 measurement results (if supported)

      - Support of simultaneous configuration of both UE event triggered and any of periodic/semi-persistence/aperiodic reporting, and solutions when both of them are configured.

      - Report destination, whether the report is sent to serving cell only or can be sent to one or more candidate cell(s).

      - Benefit when L3 measurement is involved

Event triggered reporting is beneficial to reduce overhead associated with measurement reporting and reduce latency, the report is only sent when an event is triggered at the UE. When the UE detects a change in channel conditions and finds a “better” it can send a measurement report to network. There are several ways for the UE to provide the measurement report to network.

- The UE can directly report the measurement. For example, UE can trigger a RACH procedure and include the measurement report in the payload of RACH Msg3 or MsgA PUSCH.

- The UE can send a request for uplink resources to transmit measurement report.

Proposal 8: For L1 measurement report for Rel-18 L1/L2 mobility, support UE/event triggered report for L1 measurement results. Further study how the UE provides an event triggered measurement report to the network.

## Beam indication

For inter-cell beam management, the TCI state can be associated with a cell having a PCI different from the PCI of the serving cell. The same framework can be used for L1/L2 mobility as starting point. In RAN1#111, it was agreed to at least use Rel-17 unified TCI state framework for Rel-18 LTM beam indication. However, there was an open point on whether to support Rel-18 LTM when at least one of the cells doesn’t support the Rel-17 unified TCI state framework.

Agreement

- The beam indication of candidate cell(s) for Rel-18 LTM should be designed based on the following:

  - Beam indication for Rel-18 LTM is designed based on Rel-17 unified TCI framework, if both serving cell and candidate cell support Rel-17 unified TCI framework

  - FFS: whether/how to design mechanism for Beam indication for Rel-18 LTM when at least one from serving cell and candidate cell supports only Rel-15 TCI framework.

  - Note: How and whether to indicate the new serving cell(s) and timing for beam indication are separately discussed

Unlike Rel-15 TCI/spatial relation framework, Rel-17 unified TCI state framework already supports beam indication for non-serving cells. Introducing such capability to the Rel-15 TCI/spatial relation framework would be a major design undertaking.

Observation 4: Rel-15 TCI/spatial relation framework doesn’t support beam indication for non-serving cells.

Furthermore, we don’t see a strong benefit in designing two solutions (based on the Rel-15 TCI/spatial relation framework and Rel-17 unified TCI state framework) for the same feature. This increases the design effort. A pre-requisite for supporting Rel-18 LTM is the support of Rel-17 unified TCI framework.

Proposal 9: A pre-requisite for supporting Rel-18 LTM is the support of Rel-17 unified TCI state framework.

To reduce any interruption caused by cell switch, the indication of a TCI state associated with a cell (e.g., target serving cell) having a PCI different from the PCI of the serving cell can trigger:

- AP CSI-RS resource on the target serving cell followed by a AP CSI measurement report. The AP CSI measurement report provides more refined beam measurements for the target serving cell.

- AP-SRS resource to the target serving cell. The AP-SRS resource to the target serving cell allows the network to have more refined beam measurements for the target serving cell

Proposal 10: For beam indication for L1/L2 mobility, an indicated TCI state on the target serving cell can trigger

- AP CSI-RS resource on the target serving cell followed by a AP CSI measurement report.

- AP-SRS resource to the target serving cell.

## Dynamic cell switch

In RAN1#111, the following agreement was made:

Agreement

- For beam indication timing for Rel-18 LTM,

    - Support Scenario 2: Beam indication together with cell switch command,

      - For Rel-17 unified TCI framework,

        - Beam indication indicates TCI state for each target serving cell

  - FFS: Scenario 1: Beam indication before cell switch command

  - FFS: Scenario 3: Beam indication after cell switch command

- FFS: Activation of TCI state(s) of target serving and/or candidate cell(s).

There are three scenarios for indicating the TCI state of the target cell relative to the time of the cell switch command:

- (Scenario 1 this is still FFS) The beam is indicated first to target cell; this is then followed by the cell switch command. There are two sub-scenarios for scenario:

  - (Scenario 1.1) The first sub-scenario is to have the UE start communicating using the indicated TCI state with the target cell before the cell switch command. This would be the case of ICBM. A cell switch command is then sent to complete the handover procedure. The advantage of using ICBM beam management for L1/L2 mobility is to reduce latency, signalling overhead and interruption time.

  - (Scenario 1.2) The second sub-scenario is to have the UE not use the indicated TCI state (associated with the target serving cell) right away, but rather wait for the cell switch command. When the cell switch command is received the UE uses the indicated TCI state (associated with the target serving cell) and starts communicating with the target serving cell. This scenario can be applicable for example, in case of inter-frequency handover when the UE can’t communicate with the target cell before the cell switch command is received.

- (Scenario 2 this has been agreed but there are remaining design details) The beam is indicated to the target cell with the cell switch command. There are two sub-scenarios for scenario

  - (Scenario 2.1) The first sub-scenario is to have TCI state of the target serving cell be included in the cell switch command.

  - (Scenario 2.2) The second sub-scenario is to have the indicated TCI state of the target serving cell act as the cell switch command. This can be achieved by having the beam indication, when the indicated TCI state is associated with a cell having a PCI different from the PCI of the serving cell (e.g., target serving cell), act as the trigger for cell switch.

Observation 5: Having TCI state indication act as the trigger for cell switch can reduce the latency, signalling overhead and interruption time of handover.

To achieve the benefits of TCI state indication-triggered cell switch, the UE should have the target cell configuration before the indication of a TCI state associated with a cell having a PCI different from the PCI of the serving cell (i.e., the target cell). This allows a full cell switch that is triggered by or associated with the TCI state indication. The same also applies for the dynamic cell switch command. It should be noted that, RAN2 is already considering preparing the target cell configuration to dynamic switching with a full configuration.

RAN2 to consider preparation of target cell configurations capable of dynamic switching without need for full configuration.

- (Scenario 3) The cell switch command is indicated first to the target serving cell, this is followed by a beam indication for the target serving cell. We think that this scenario can have a longer latency.

As aforementioned, there are several ways to achieve a dynamic cell switch based on beam indication that RAN1 can further consider:

- Network-triggered dynamic cell switch. This can be based on indication of a TCI state associated with a cell (the target serving cell) having a PCI different from the PCI of the serving cell. The cell switch can happen at the application time of the TCI state (whether the TCI state is included in the cell switch command (i.e., scenario 2.1), or the TCI state indication itself triggers cell switch (scenario 2.2)), this would have fastest cell switch time as illustrated in Figure 1. One potential issue with doing the cell switch at the time of TCI state application is that it could lead to ping-pong effect if subsequent TCI states are associated with the source serving cell. To avoid, this issue a separate signal can be used to do the cell switch (Scenario 1.1). This is illustrate in Figure 2, where the UE is first indicated a TCI state associated with cell having a PCI different from the PCI of the serving cell, this is subsequently followed by a signal (that can be based on additional criterial) to perform the cell switch. A variant of Figure 2 is not to apply the TCI state of the target serving cell right away, but rather wait for the cell switch command as illustrated in Figure 3.

Figure 1: Cell switch at the application time of the TCI state

Figure 2: Cell switch after application TCI state using a separate signal.

Figure 3: Applying previously indicated TCI state at time of dynamic cell switch.

Proposal 11: Rel-18 LTM, in addition to the beam being indicated to the target cell with the cell switch command, consider support of the following,

- The beam is indicated first to target cell; this is then followed by the cell switch command.

- UE-initiated dynamic cell switch. The UE has knowledge of the signal strength from the surrounding TRPs (base stations), by measuring the RSRP of the reference signals transmitted from these TRPs. Hence, it would be in the best position to make the fastest decision on handover. The UE can indicate a handover request in its measurement report. The cell-switch can be triggered once the measurement report is received by the network (subject to a cell-switch latency), this is illustrated in Figure 4. Alternatively, the network, can transmit additional signaling to confirm the cell switch.

- Figure 4: UE initiated cell switch.

Proposal 12: For enhancements L1/L2 mobility, study UE-initiated dynamic cell switch.

When dynamic cell switching is provided via TCI indication, the indication should provide QCL information for both of DL reception and UL transmission. If only one of separated DL TCI state or separated UL TCI state is indicated, the UE may understand that cell switching happens only for DL or UL while the other link is still accessed via the source serving cell. Unless such connectivity is agreed as a supported DC scenario, RAN1 should not support indicating only one separate TCI state via non-serving cell TCI indication.

Proposal 13: Non-serving cell TCI indication should include joint TCI or both of separate DL/UL TCI states except for DC scenario.

# Conclusions

The following observations and proposals have been made regarding L1 enhancements to support L1/L2 mobility:

Observation 1: For Rel-18 LTM, for SSB-based L1-RSRP measurements, the measurement is defined as inter-cell intra-frequency L1 measurement when the centre frequency and the sub-carrier spacing of the SSB of the non-serving cell is the same as those of the SSB of the serving cell, otherwise the measurement is considered L1 inter-frequency measurement.

Observation 2: For Rel-18 LTM, if CSI-RS-based measurements are supported, it is up to RAN4 to decide the corresponding intra-frequency and inter-frequency scenarios.

Proposal 1: For enhancements to L1 measurement and reporting for L1/L2 mobility, legacy RS types supporting mobility management (CSI-RS for Mobility) should be considered as source RS for beam measurement and reporting.

Proposal 2: For enhancements to L1 measurement and reporting for L1/L2 mobility, study the support of TRS and CSI-RS for beam management associated through a QCL relation to an SSB associated with a PCI of a cell that is different from the PCI of the serving cell as a source RS for beam measurement and reporting.

Observation 3: Uplink measurements for L1/L2 mobility are performed by gNB and hence be left for network implementation.

Proposal 3: Further study the necessity, reliability and latency of L1-SINR measurement metric for L1/L2 mobility.

Proposal 4: Study the following alternatives for the L1 measurement report taking into account the timing of respective measurements:

- Alternative 1: A common measurement report for L1 intra-frequency measurement and L1 inter-frequency measurement.

- Alternative 2: Separate measurement reports for L1 intra-frequency measurement and L1 inter-frequency measurement.

Proposal 5: For Rel-18 LTM measurement configuration, reuse the Rel-17 ICBM measurement configuration framework, with the following changes:

- Include the ARFCN of the cell in the SSB-MTC-AdditionalPCI information element.

- Increase the maximum number of additional PCIs (FFS maximum value).

Proposal 6: For Rel-18 LTM, the gNB scheduled L1 measurement report can be transmitted using at least periodic PUCCH.

Proposal 7: Support reporting a variable number of SSBRI (or CRI)/RSRP/Capability index groups, wherein a UE reports in a single reporting instance a two-part beam report using the two-part UCI structure.

- Part 1 includes the beam measurement(s) for the serving cell and indicates the size of part 2.

- Part 2 includes the remaining beam measurement(s).

Proposal 8: For L1 measurement report for Rel-18 L1/L2 mobility, support UE/event triggered report for L1 measurement results. Further study how the UE provides an event triggered measurement report to the network.

Observation 4: Rel-15 TCI/spatial relation framework doesn’t support beam indication for non-serving cells.

Proposal 9: A pre-requisite for supporting Rel-18 LTM is the support of Rel-17 unified TCI state framework.

Proposal 10: For beam indication for L1/L2 mobility, an indicated TCI state on the target serving cell can trigger

- AP CSI-RS resource on the target serving cell followed by a AP CSI measurement report.

- AP-SRS resource to the target serving cell.

Observation 5: Having TCI state indication act as the trigger for cell switch can reduce the latency, signalling overhead and interruption time of handover.

Proposal 11: Rel-18 LTM, in addition to the beam being indicated to the target cell with the cell switch command, consider support of the following,

- The beam is indicated first to target cell; this is then followed by the cell switch command.

Proposal 12: For enhancements L1/L2 mobility, study UE-initiated dynamic cell switch.

Proposal 13: Non-serving cell TCI indication should include joint TCI or both of separate DL/UL TCI states except for DC scenario.

# References

- RP-222332 “Revised WID on Further NR mobility enhancements”, 3GPP RAN#97e, e-Meeting, September, 2022.

- R4-2220733 (R1-2300031), “Reply LS on L1 intra- and inter- frequency measurement and configurations for L1/L2-based inter-cell mobility”, 3GPP RAN4, Toulouse, France, November, 2022.