3GPP TSG RAN WG1 #112 	R1-2301397

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.1.2

Source: 	Qualcomm Incorporated

Title: 	CSI enhancements for medium UE velocities and Coherent-JT

Document for:	Discussion/Decision

# Introduction

In the Rel-18 MIMO WID [1], the following objectives were scoped for CSI:

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - … - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

In this contribution, we continue to discuss aspects related to the above three features: Type-II-Doppler, Type-II-CJT, and TRS-based TDCP reporting.

# Type-II-Doppler codebook refinement

Remaining issues for Type-II-Doppler include: CQI definition in time-domain, NZC bitmap – these two have been discussed with considerable attention over the last few meetings. Besides, the requirement of CMR (CSI-RS burst) in time-domain has not been touched much, which will also be discussed in this section.

## CQI and reference resource

RAN1#111 agreements [2]:

Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), assuming 1 CQI in one sub-band and one CSI reporting instance, down-select (by RAN1#112) one from the following alternatives: - Alt1. The CQI is associated with the entire duration of the CSI reporting window and all the N4 W2 matrices - Alt2A. The CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Alt2B. The CQI is associated with the first/earliest d slots of the CSI reporting window and the first/earliest one of the N4 W2 matrices Note: The N4 W2 matrices represent the combining coefficients before DD compression at the UE, or after DD de-compression at the gNB Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, decide by RAN1#112 whether including X>1 CQIs in one sub-band and one CSI reporting instance are supported - If supported, also decide the value(s) of X and the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots)

The issue with time-domain CQI “prediction,” is different from PMI regarding baseline:

- For PMI, baseline Rel-16 eType-II holds constant until the next report;

- For CQI, baseline already has OLLA as a well-known legacy time-domain tracking tool for MCS/CQI.

Therefore, even for Rel-18 Type-II-Doppler CSI with the already-agreed CSI window (WCSI), legacy single-slot CQI can still work: Link adaptation (OLLA) can amend/predict MCS/CQI variation over time e.g. at some slots within WCSI other than one defined as reference resource, whose CQI is reported.

Observation 1: For Type-II-Doppler, CQI defined with non-full WCSI does not ruin the completeness of its framework.

In another aspect, from performance perspective, CQI prediction itself is not reliable due to:

- Interference cannot be predicted, which makes the more future CQI, the less meaningful it is;

- Unlike PMI prediction (assuming based on channel extrapolation), where the power of a certain extrapolated H is not important – since precoder only needs relativity b/w ports – however, CQI based on the power of precoded channel H⋅W certainly rely on H power

  - At least for MMSE-based channel extrapolation, H degrades with time-correlation, which means it will fade over time – this is not true for the power of a real future channel with a reported precoder H⋅W

Some evaluation results can be found in Figure 1, where no gain is observed with future CQI reported (X>1 with Alt1), over one single first-slot CQI (Alt2A).

Figure 1. Type-II-Doppler CQI: Single-slot v.s. X>1, over Rel-16 baseline

Some evaluation assumptions and parameters are listed below (generally also followed by other evaluations throughout this Section 2, unless otherwise specified; More assumptions can be found in Section 2.5):

- CSI window parameters (slots):

  - WCSI=16: N4=8, d=2;

  - δ=0

- CSI-RS: K=4, m=2

- Number of selected TD bases: Q=3

- Legacy parameters:

  - L=4,

  - p_1,2=0.5 (M_1,2=7 for N_3=13); p_3,4=0.25 (M_3,4=4 for N_3=13)

- Scheduling assumption:

  - Rel-16 baseline feedback periodicity: 5 slots

  - Type-II-Doppler: 11 slots (slightly less overhead than Rel-16 with less frequent feedback)

- OLLA assumed for all schemes (Rel-16 baseline, Alt2A, and Alt1);

- MMSE-based channel extrapolation

Observation 2: For Type-II-Doppler, more future CQI report (with Alt1) does not bring UPT gain (over Alt2A), under MMSE-based channel extrapolation.

Last but not least, UE complexity is proportional to number of slots with CQI definition, regardless of X=1 (thus averaging is needed) or X>1.

Observation 3: For Type-II-Doppler, CQI defined with entire WCSI (Alt1) has the highest UE complexity regardless of X=1 or X>1.

Therefore, based on performance and UE complexity, we propose

Proposal 1: For Type-II-Doppler, support the first slot of WCSI as reference resource for CQI report (Alt2A).

### Timeline anchor

Since it already has both PMI and CQI defined in a new time location than legacy (for any CQI Alts of the agreement), it is natural for Rel-18 TD CSI to have its new reference resource definition.

Legacy CSI reference resource actually has two roles: (1) For RAN4 validation test for PMI/CQI report; (2) To take into account CSI timeline – as a “timeline anchor,” e.g.

Section 5.2.2.5, TS 38.214 After the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement no later than CSI reference resource and drops the report otherwise.

For Rel-18 TD CSI, it is straight-forward to decouple timeline anchor and reference resource.

Proposal 2: For Type-II-Doppler, decouple the definition of Rel-18 CSI reference resource and “timeline anchor,” where the “timeline anchor” can reuse Rel-15 CSI reference resource definition.

Figure 2. Rel-15 CSI reference resource slot can be used as timeline anchor for Rel-18 TD CSI

## NZC selection bitmap

RAN1#111 discussion proposal [4]:

Latest status from RAN1#111: For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, down-select one from the following alternatives: - Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e. the size of the bitmap for each selected DD basis vector is 2LMv - Alt2. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design and further compressed using source-coding (e.g Huffman code) - Alt3A: A single 2-dimensional bitmap of size MQ to report the selected S pairs of FD basis vector and DD basis vector and a single 2-dimensional bitmap of size 2LS for indicating the location of the NZCs, where each row corresponds to a selected SD basis vector and each column corresponds to one of the selected S pairs of FD basis vector and DD basis vector. - Alt3B: A single 2-dimensional bitmap of size 2LQ to report the selected S pairs of SD components and DD basis vector and a single 2-dimensional bitmap of size MS for indicating the location of the NZCs, where each row corresponds to a selected FD basis vector and each column corresponds to one of the selected S pairs of SD component and DD basis vector. - Alt3C: A single 2-dimensional bitmap of size 2LM to report the selected S pairs of SD component and FD basis vector and a single 2-dimensional bitmap of size QS for indicating the location of the NZCs, where each row corresponds to a selected DD basis vector and each column corresponds to one of the selected S pairs of SD component and FD basis vector. - Alt4. A bitmap that includes bits associated with the set of {(f1, s1, t1)} with d(f1, s1, t1)<=d, where d is the threshold that can be configured by gNB, df1, s1,t1= minf1-f0, Mv-f1-f0+ mins1-s0, L – s1-s0+ mint1-t0, Q – t1-t0. s0, f0 and t0 denotes a reference SD basis index and a reference FD basis index and a reference DD basis index associated with SCI, respectively.

According to the above agreement, Alt 1 is a fully-free 3-dimensional {beam, delay, Doppler} bitmap with 2LMQ-bit size, while Alt3A, Alt3B and Alt3C can be expected to save bitmap overhead with 2-stage bitmaps, by exploiting channel sparsity in {beam, delay, Doppler} space.

Take Alt3C as an example, if the non-zero locations at legacy SD&FD are selected in a first stage by 2LM-bit bitmap, which has a maximum possible value of S; Then at a second stage, a QS-bit bitmap can be used to indicate the NZCs for each of the Q TD bases. By doing this, a total bitmap size is 2LM+QS, and can achieve an intermediate bitmap size between fully-free Alt1 and more-restricted Alt2. An intuitive illustration can be found in Figure 3.

As long as βSD&FD=S2LM<Q-1Q, it would result in bitmap size reduction: 2LM+QS < 2LMQ. Similar derivation of overhead reduction can be obtained for Alt2A or Alt2B.

Some evaluation results provided in Figure 4 show that the 2-stage bitmap has similar performance as the fully-free 3D bitmap, where Alt2C is slightly better than Alt2A, with larger overhead – the corresponding overhead calculation can be found in Table 1.

Figure 3. Alt3C 2-stage bitmap in 3-dimensional {beam, delay, Doppler} space

Figure 4. Type-II-Doppler NZC bitmap: 3D (Alt1) v.s. 2-stage (Alt3C and Alt3A)

Table 1. CSI overhead calculation

Observation 4: For Type-II-Doppler, 2-stage (2LM+QS)-bit bitmap (Alt3C) achieves same average throughput as 2LMQ-bit 3D bitmap, while overall feedback overhead can be reduced about 5.8% to 9.9% (reduced from 973 bits, to 917 or 877 bits).

Proposal 3: For Type-II-Doppler, support 2-stage NZC selection bitmap:

- 1st preference (Alt3C): A first 2LM-bit bitmap, and a second QS-bit bitmap;

- 2nd preference (Alt3A): A first QM-bit bitmap, and a second 2LS-bit bitmap.

## CMR requirement

RAN1#110bis-e agreements [3]:

Agreement On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, support the following CSI-RS resource types/structures for CMR: - Time-domain behaviour for NZP CSI-RS resource: periodic (P), semi-persistent (SP), aperiodic (AP) - FFS: Whether to introduce constraints on allowed configuration - Down select from the following: - Alt1. Support K>1 NZP CSI-RS resources, received via a single triggering instance, for aperiodic (AP) -CSI-RS-based channel measurement in a same CSI-RS resource set where the separation between 2 consecutive AP-CSI-RS resources is m slot(s): - Alt2. Support one NZP CSI-RS resource in a CSI-RS resource set, where K>1 occasions are received via a single triggering instance, for aperiodic (AP)-CSI-RS-based channel measurement where the separation between 2 consecutive AP-CSI-RS resources is m slot(s). - For any of the alternatives: - No CRI is reported - FFS: Details, e.g., supported value(s) of K, m, other use cases for the AP-CSI-RS resources (e.g., for training filter coefficients, prediction or performance monitoring) - Support only one NZP CSI-RS resource for P or SP-CSI-RS-based channel measurement Agreement On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, support the following CSI-RS resource types/structures for CMR, support the following: - (Alt1) Support K>1 NZP CSI-RS resources, received via a single triggering instance, for aperiodic (AP) CSI-RS-based channel measurement in a same CSI-RS resource set where the separation between 2 consecutive AP-CSI-RS resources is m slot(s)

For an aperiodic Type-II-Doppler CSI report configured with P/SP CSI-RS, UE needs to be prepared for an aperiodic report potentially triggered any time. This may require UE buffer a series of recently received CSI-RS occasions, and can dramatically increase the requirement of UE memory and cause wasted power consumption, especially when operating at a large bandwidth.

Another probably more severe issue if the CSI-RS occasions are not received in a “causal” way is, the receiving may lack phase continuity, which will make the channel-based extrapolation not even workable. Until current RAN1 discussion, channel-based extrapolation is the general assumption, thus it is reasonable to assume phase-continuity needed to receive a burst of CSI-RS occasions for a report.

Therefore, a sufficient number of CSI-RS occasions after PDCCH triggering should be satisfied for the report.

Proposal 4: For Type-II-Doppler, specify restriction on P/SP CSI-RS: A threshold/minimum number of CSI-RS occasions should be satisfied between the end of the triggering PDCCH and Z’ symbols prior to the report PUSCH.

## Long latency issue with PUSCH scheduling

To accommodate a burst CSI-RS occasions, and to accommodate CSI timeline, PDCCH-to-PUSCH distance should be long enough. For instance, 4 CSI-RS occasions and 4-slot periodicity would take at least 13 slots; in plus with CSI timeline 5 slots (take Z2’=69 symbols for 30kHz SCS) – 18 slots in total as shown in Figure 5.

Figure 5. Long distance of PDCCH-to-PUSCH

The long PDCCH-to-PUSCH distance can cause two issues:

- Issue 1: Longer latency for UL-SCH conveyed on the report PUSCH;

- Issue 2: Potentially reduced UL throughput due to the in-order HARQ rule: No scheduling is feasible in between.

Figure 6. In-order HARQ rule

One possible solution can be, split the CSI measurement and reporting triggering by 2-stage PDCCH:

- PDCCH1: Triggers CSI measurement/computation, or additionally with AP CSI-RS burst (if applicable, i.e. if the triggered AP CSI state is associated with AP CSI-RS burst);

- PDCCH2: Schedules the report PUSCH (e.g. PUSCH2 in Figure 7);

- The linkage to associate PDCCH1 and PDCCH2 can be, a same AP CSI state codepoint is indicated.

For PDCCH1, it does not necessarily need to be a UL grant. But given that UL grant DCI format 0_1 o 0_2 already has such triggering field, reusing UL grant DCI for PDCCH1 may need less standard efforts.

An example illustrated in Figure 7 has some timeline from legacy definition (Z, Z’, and N2) simply extended to this case.

Figure 7. 2-stage PDCCH triggered TD CSI

Proposal 5: For Type-II-Doppler, study 2-stage PDCCH triggering mechanisms to prevent the latency and throughput reduction of UL-SCH due to the PUSCH conveying aperiodic Type-II-Doppler CSI: A 1st PDCCH to trigger CSI measurement/computation (and AP CSI-RS, if applicable), and, a 2nd PDCCH to trigger report.

## Evaluation assumption

Based on EVM assumptions [5], customized with:

| Scenario | - UE speed: {10, 30, 60} kmph - Mobility model: No spatial consistency assumed |
| --- | --- |
| Carrier frequency | 2GHz |
| ISD | 500m |
| gNB antenna setup | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| UE antenna setup | 4Rx: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ, or 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |

# Type-II-CJT codebook refinement

Remaining issues for Type-II-Doppler include: Selected number of SD bases (Ln), FD basis selection for mode-1 CB, UCI packing/omission, CBSR etc. Generally speaking, the above issues are either already discussed a lot, or straight-forward to consider as an extension of Rel-16 sTRP eType-II. However, we identify that a TRP-power issue has not been discussed much, which will be analysed in Section 3.3.

## SD basis selection

RAN1#111 agreements [2]:

Agreement On the Type-II codebook refinement for CJT mTRP, regarding the SD basis selection, for a configured value of NTRP, a set of NL combinations of values for {L1, ..., LNTRP} is gNB-configured via higher-layer (RRC) signaling - When NL>1, the selected combination of values for {L1, ..., LNTRP} is reported in CSI part 1 using an indicator, selected from the NL configured combinations - NL =1 is one of the supported candidate values - FFS: Other supported value(s) of NL, and its respective UE capability - FFS: The supported combinations of values for {L1, ..., LNTRP} - Following the legacy design, the SD basis selection for the n-th (n=1,...,N) selected CSI-RS resource is indicated in CSI part 2 using a combinatorial indicator selected from a set of PCSI-RS/2Ln codepoints where, for Rel-16-based refinement PCSI-RS = 2*N1N2. - The supported candidate values for each of the Ln parameters include the legacy candidate values, i.e. {2,4,6} for Rel-16-based refinement, and - for Rel-17-based refinement, the gNB configures a set of N_L combinations for {alpha1, ..., alphaNTRP} Ln=αnPCSI-RS/2 where αn=12,34, 1 - Following the legacy design, for all the selected N CSI-RS resources, the SD basis oversampling group for each CSI-RS resource is indicated in CSI part 2 using an indicator selected from a set of O1O2 codepoints. FFS: Whether the set of NL combinations of values for {L1, ..., LNTRP} can be implicitly derived

General framework of SD basis selection has been agreed as in the above agreement, and the main remaining issue is how to configure and report the number of selected SD bases (Ln) for each TRP.

For layer-common SD basis selection, it is a common sense to select before SVD, which is generally according to the power of each digital beam. According to pre-RAN1#112 offline discussion [4], there are two general options for the configuration of ParamCombo:

- ParamCombo option 1: Ltot,pv,β, where Ltot=n=1NLn (or similarly with a Lmax≥n=1NLn);

- ParamCombo option 2: pv,β, with a linkage to another parameter: {Ln}-combo table.

In our view, the above option 1 is more like Rel-16, and needs less standard efforts – the explicit value of {Ln}-combo can leave to implementation, and standard only needs to define e.g. 8 combos of Ltot,pv,β.

With ParamCombo option 2, however, we may need to do a full search of every possible value combo {Ln}, under every possible value combo of pv,β, to find reasonable linkages between value combo of pv,β and {Ln}.

Observation 5: For Type-II-CJT, a total value Ltot=n=1NLn may require less standard efforts than {Ln}-combo.

Proposal 6: For Type-II-CJT, for the configuration of selected number of SD bases,

- 1st preference: Via ParamCombo Ltot,pv,β;

- 2nd preference: From a set of standard-defined {Ln}-combos.

It is noted that, even with ParamCombo option 2 (where there may not be an explicit Ltot configured), a fixed total number of selected SD bases is beneficial for a relatively fixed UCI payload size, for gNB to appropriately allocate UL resources for the CSI report. Therefore, for one certain configuration of NL>1 value combinations of {L1, ..., LNTRP}, a fixed value of n=1NTRPLn should be ensured in order not to variate UCI payload size too much.

Proposal 7: For Type-II-CJT, for the configuration of selected number of SD bases with NL>1 value combinations of {L1, ..., LNTRP}, a same value of n=1NTRPLn should be guaranteed.

## FD basis selection

RAN1#111 agreements [2]:

Agreement On the Type-II codebook refinement for CJT mTRP, for mode-1, study and down select (no later than RAN1#112) only one from the following schemes: - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources - Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset) - Alt3. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf,n where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf,n is independently selected across N CSI-RS resources For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis. Note: Per previous agreements, the number of selected FS basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources

The UPT performance of Alt2 and Alt3 do not differ, and thus it is Alt1 vs. Alt2/3. If we take mode-2 FD-joint CB as baseline, Alt1 can be literally seen as a “compromise” between mode-1 CB Alt2/3 and mode-2 CB (may be called mode-1.5 codebook).

However, from the evaluation results in Figure 8, Alt1 shows performance loss over both mode-2 and mode-1 Alt2/3 at smaller number of selected FD bases (e.g. M=2, via pv=1/8). Evaluation assumptions are:

- Under co-site 3-TRP (i.e. outdoor2 OptA in the agreed EVM [5]) deployment, each with 8 ports (more evaluation assumption in Section 3.8)

- Layer-specific FD index offset

- Algorithm: Find the per-layer strongest TRP, and other TRPs follow the FD indices “pattern” of the strongest TRP, to find each FD offset

Figure 8. Type-II-CJT FD selection: Independent (Alt2/3) vs. offset (Alt1), over baseline mode-2 CB

Observation 6: For Type-II-CJT, mode-2 and mode-1 CB Alt2/3 have similar performance, while mode-1 CB Alt1 can have performance loss at smaller number of selected FD bases (M).

Besides, from algorithm perspective, we failed to see a physical meaning why doing Alt1 – we failed to exploit TRP-relative delay with the FD index offset. FD basis selection has been agreed to be layer-specific (following Rel-16), and it is a common sense to select FD after SVD, which can cause FD offset totally different from TRP-relative delay.

As for overhead, it is apparent that mode-2 FD-joint CB is the best, as shown in Table 2. Therefore, we propose

Proposal 8: For Type-II-CJT, for FD basis selection,

- 1st preference: No need to support mode-1 FD-independent CB;

- 2nd preference: Mode-1 with TRP-independent FD selection (Alt2)

Table 2. Overhead of FD basis selection Alts: Mode-2 vs. mode-1 Alt2/3 vs. mode-1 Alt1

Then for Alt2 (if supported), we propose simply extending existing mechanisms regarding N3≤19 and N3>19 respectively as in the following proposal:

Proposal 9: For Type-II-CJT, for FD basis selection under mode-1 FD-independent codebook, largely reuse Rel-16 sTRP mechanism for independent FD basis selection of TRPs (Alt2):

- For N3≤19, SCI-TRP’s FD selection by log2N3-1Mv-1×rank bits, while N-1 remaining TRPs’ selections by N-1×log2N3Mv×rank bits independently;

- For N3>19,

  - For the 1st stage selection (window), SCI-TRP’s FD window by log22Mv×rank bits, while N-1 remaining TRPs’ windows by N-1×log2N3×rank bits independently (the N-1 remaining TRPs’ sized-2Mv windows may not comprise SCI-related FD basis);

  - For the 2nd stage selection, SCI-TRP’s by log22Mv-1Mv-1×rank bits, while N-1 remaining TRPs’ by N-1×log22MvMv×rank bits independently

As for Alt3, our understanding regarding the interpretation of its associated “FD index offset” is, the offset between the strongest FD index (not necessarily 0 for the other N-1 TRPs than the SCI-TRP), to SCI-TRP’s strongest index 0. – Without these offsets reported, Alt2 FD selection can still work fine, just no information regarding other N-1 TRPs’ strongest FD.

Whether Alt3 is needed over Alt2, may depend on the UCI packing order of quantized NZCs:

- For legacy, SCI-TRP would have permutated FD order of 0,N3-1,1,N3-2,2,…  (index 0 is the strongest FD);

- For other certain TRP#n, if analogous to legacy, would have an FD order of [offsetn, offsetn-1, offsetn+1, offsetn-2, offsetn+2, …] (index offsetn is the strongest FD; Besides, cyclic shift operation here omitted for simplicity)

Observation 7: For Type-II-CJT, for FD selection Alt3 with FD index offset reported, it can support a similar FD permutation analogous to legacy Rel-16 for each TRP.

The question is, whether FD permutation is still needed for the UCI packing order. For Rel-16 priority function Pril,i,f=2⋅L⋅υ⋅πf+υ⋅i+l, FD is the most significant dimension to determine NZC priorities. However, for mTRP case, it should be a natural question: Whether still FD, or the newly-introduced TRP-dimension, is the most significant dimension for group 1 and 2 split of NZCs. If TRP-dimension is the main determining factor (i.e. “outermost” indexing for priority function), FD permutation would be much less useful.

Observation 8: For Type-II-CJT, FD permutation is not meaningful for CJT, if TRP-dimension is the most significant dimension for group 1 and 2 split of NZCs.

## UCI packing order of quantized NZCs

UCI packing is related to UCI omission. In Rel-16 mechanism, the quantized W2 coefficients reported in CSI part 2 are near-equally divided into two groups: Group 1 and 2, where group 2 has lower priority regarding UCI omission. In Rel-16, (permuted) FD is the outermost indexing for UCI packing, and therefore, FD is the most significant dimension regarding coefficient grouping.

For CJT mTRP, we additional has TRP-dimension. There can be at least the following two benefits for TRP-dimension as the outermost indexing:

- No need to consider the potentially complicated TRP-specific FD permutation for mode-1 CB (as analysed in the last Section 3.2 and concluded with Observation 8);

- Beneficial for network energy saving, e.g. save the energy of omitted TRP(s).

Figure 9. Type-II-CJT coefficient packing order: TRP-dimension as the outermost indexing

Proposal 10: For Type-II-CJT, UCI packing of quantized coefficient use TRP-dimension as the outermost indexing.

## TRP power and PDSCH-to-CSIRS Pc ratio

RAN1#110bis-e agreements [3]:

Agreement On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group and Strongest Coefficient Indicator (SCI) design, for each layer: - One (common) SCI applies across all N CSI-RS resources - Further down-select one from the following alternatives by RAN1#110bis-e: - Alt1. One group comprises one polarization across all N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2) - FFS: Amplitude quantization table considering transmission power difference between multiple TRPs - For each of the amplitude groups (other than the group associated with the SCI), the reference amplitude is reported - Alt3. One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N) - For each of the (2N–1) amplitude groups (other than the group associated with the SCI), the reference amplitude is reported FFS: The need for “strongest” TRP/TRP-group indicator in addition to the SCI

It has been agreed W2 normalization is joint across all TRPs, i.e. one single SCI for all the TRPs. Given that the associated W2 coefficients of each TRP are independently reported, the transmission power of different TRPs are not necessarily the same, and naturally has a requirement for network to allocate relative DL power amongst TRPs.

Figure 10. TRP power illustration: Different TRPs are not necessarily with the same power.

Power allocation amongst TRPs may be against the typical implementation of existing networks, where the DL may generally have a fixed transmission power. However, to enable CJT, variable DL power has to be assumed, based on the till-now agreements of codebook structure (either mode-1 or mode-2 CB) with single-SCI-W2.

Proposal 11: For Type-II-CJT, it should be a natural assumption for network implementation to allow variable transmission power for a CJT-PDSCH transmitted from a certain TRP.

One issue exists for PDSCH-to-CSIRS power offset definition (Pc ratio), due to

- It has been agreed one TRP corresponds to one CSI-RS resource since some early meeting of Rel-18;

- In existing standard, Pc ratio is per-resource configured

Therefore, if we follow current standard, it means the relative power of PMI (precoder) from any 2 TRPs are pre-determined by RRC config.

However, TRP-relative power of CJT-PDSCH is determined by the calculated precoder: PW2TRP#1PW2TRP#2, where PW2 is defined as squared summation of coefficients’ amplitude. The value of PW2TRP#1PW2TRP#2 should be determined by measurement, which is not necessarily same as config.

Observation 9: Per-TRP Pc ratio definition can’t be guaranteed by the calculated PMI according to CSI measurement.

Proposal 12: For Type-II-CJT, PDSCH-to-CSIRS Pc ratio is defined as the ratio between

- Summation of PDSCH power across all TRPs, and

- Summation of CSI-RS power across all TRPs

Pc=10log10PPDSCHtotalPCSI-RStotal=10log10n=1NPPDSCHTRP#nn=1NPCSI-RSTRP#n

Therefore, for CJT, Pc ratio is per CSI-RS resource set configured.

## CMR requirement

For Rel-17 NCJT with 2 TRPs, a pair of NZP CSI-RSs is required to be within at most 2 consecutive slots, without DL/UL switching in between.

For CJT, shorter timing restriction for CMR should be ensured, since phase coherence naturally is more sensitive than NCJT regarding timing. Besides, Rx phase coherence is generally only required within one slot.

Proposal 13: For Type-II-CJT, support CMR timing restriction stricter than NCJT: All NZP CSI-RS resources in the resource set are received within one same slot, and no DL/UL switch in between.

## Larger delay-spread issue

The larger delay-spread of distributed TRPs can cause more severe frequency-selectivity, possibly within a precoder subband, if the distance b/w TRPs (ISD) is large.

According to EVM [5], however, the 200m or 500m ISD for the two layouts copied from the EVM as in Figure 11 may not be large enough to exceed the max unambiguous delay of a subband, as illustrated in Table 3 for comparison.

- The site radius is about 100m or 250m site radius;

- The TRP-relative delay from certain reflection path can even be further smaller than the site radius

Figure 11. EVM layouts [5]

Table 3. Max unambiguous delay of 2- or 4-RB subband @15kHz and 30kHz SCS

Besides, due to time limitation of Rel-18, maybe this issue can be delayed to further study in a later release.

Proposal 14: Defer the study of TRP-relative delay compensation to later than Rel-18.

## Evaluation assumption

Based on EVM assumptions [5], customized with:

| Scenario | Outdoor2 – OptionA - NTRP=3 intra-site cluster - 21-sector |
| --- | --- |
| Carrier frequency | 2GHz |
| ISD | 200m |
| Channel generation model | - Per-TRP delay - Ideal synchronization |
| gNB antenna setup | 8 ports: (4,4,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ |
| UE antenna setup | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |

# TDCP reporting measured via TRS

RAN1#111 agreements [2]:

Agreement For the Rel-18 TRS-based TDCP reporting, down select only one of the following alternatives by RAN1#112: - AltA.1 (Doppler spread) as described in R1-2210523 - AltA.2 (Doppler shift): A UE is configured to report the Doppler shifts corresponding to the M strongest peaks of the wideband Doppler spectrum, for each of the N≥1 configured TRS resources - A TDCP report can be configured with N periodic TRS resources (e.g., N=2 with one TRS resource per TRP) - Parameter M is RRC configured with candidate values TBD, e.g. M=1,2,3,… - Wideband Doppler spectrum is calculated from the wideband time correlation function, given, as an example, by At,τ=ct,τct,0 , where ct,τ=n=0N-1hnt+τ∙hn*t and hn is the channel for subcarrier n. - AltB (TD correlation profile) as described in R1-2210523 Down-selection is to done based on, at least, the (single-)user throughput (LLS) performance comparison among the alternatives assuming: - Three special cases of an agreed use case (companies can select only one or more): aiding gNB to determine switching between Type-I and Rel-16 eType-II codebooks, or to determine SRS periodicity in the UL-SRS reciprocity-based precoding scheme; or aiding the gNB implementation in CSI prediction for TDD - In their simulations on switching between Type-I and Rel-16 eType-II codebooks, companies should state how to calculate the metric for the determination and how to set the threshold, and what the UE reports. - In their simulations on UL-SRS reciprocity-based precoding scheme, companies should state how to set the SRS periodicity based on the reported metrics, and what the UE reports; and the results should be displayed in terms of user throughput vs SRS overhead - In their simulations on CSI prediction for TDD, the results should be the correlation between real channel and predicted channel, and what the UE reports; aided by the reported metric. - Other scenarios of the agreed use cases can optionally be simulated - Based on the agreed EVM for sTRP and mTRP Note: Different alternatives may or may not apply to different use cases FFS: The need for a measure of confidence level in the TDCP report, and/or UE behaviour when the quality of TDCP measurement is not sufficiently high FFS: TDCP parameter(s) signalled with respect to each alternative

For the current two alternatives, time-correlation (AltB) would mean lower complexity at UE side than Doppler spread (AltA), since time-correlation is anyway an intermediate step to calculate Doppler spread. Besides, from UE implementation perspective, time-correlation has larger chance to reuse (part of) existing tracking algorithms, since time-correlation has almost no ambiguous understanding for its definition.

In contrast, Doppler spread depends very on proprietary implementation. – If the standard happens to define a Doppler spread same as what used for UE implementation, UE is happy to reuse the calculation; Otherwise, UE has to do double-work, i.e. Doppler shift/spread calculation for its own non-report-relevant processing, as well as extra efforts on calculating another version of Doppler spread required by Rel-18 TDCP report.

For time-correlation, the following equation (known as “geometric average”) can be robust to random phase jump over time (with the absolute operation), as well as robust to AGC between time l-τ and l (with the normalization).

Aτ=kH*l-τ,kHl,kkH*l-τ,kHl-τ,kkH*l,kHl,k=kH*l-τ,kHl,kkHl-τ,k2kHl,k2, where

l-τ and l are time-domain index denoting two TRS symbols with lag τ, and k is frequency-domain index denoting all subcarriers of TRS.

Proposal 15: For TRS-based TDCP report, support time-correlation report (AltB), and allow UE implementation to use time correlation calculation by Aτ=kH*l-τ,kHl,kkHl-τ,k2kHl,k2.

Then regarding time-correlation profiles with longer lag than 2 slots for lower-speed scenario (so-called “inter-burst” time-correlation), it is not related to any legacy UE receiving behavior for TRS. To support more lag values, some time pattern of TRS (periodicity, or linkage between P/AP TRS) anyway has to be defined. Two main concerns are:

- RS overhead. For example, to support an inter-burst lag like 5 or 10 msec, a 4 burst of 4 symbols of single-port CSI-RS need to be sent where 3 of them may not be necessary for the targeted lag;

- It may be difficult to avoid using AP-TRS (since P-TRS with 5 or 10 msec is too dense), but AP-TRS seems to have never been actually implemented according to our understanding – but even if with AP-TRS, the redundant overhead with 4 symbols is still not resolved.

Therefore, time pattern fit for the longer lag for time correlation can simply be defining single-port CSI-RS occasions with the targeted lags.

Proposal 16: For TRS-based TDCP report, for time-correlation with longer lag, study single-port CSI-RS with new time spacing/pattern.

# Conclusion

In this contribution, we discuss issues related to CSI enhancement for three features: Type-II-CJT mTRP, Type-II-Doppler, and TDCP reporting measured via TRS. Based on the observations:

Observation 1: For Type-II-Doppler, CQI defined with non-full WCSI does not ruin the completeness of its framework.

Observation 2: For Type-II-Doppler, more future CQI report (with Alt1) does not bring UPT gain (over Alt2A), under MMSE-based channel extrapolation.

Observation 3: For Type-II-Doppler, CQI defined with entire WCSI (Alt1) has the highest UE complexity regardless of X=1 or X>1.

Observation 4: For Type-II-Doppler, 2-stage (2LM+QS)-bit bitmap (Alt3C) achieves same average throughput as 2LMQ-bit 3D bitmap, while overall feedback overhead can be reduced about 5.8% to 9.9% (reduced from 973 bits, to 917 or 877 bits).

Observation 5: For Type-II-CJT, a total value Ltot=n=1NLn may require less standard efforts than {Ln}-combo.

Observation 6: For Type-II-CJT, mode-2 and mode-1 CB Alt2/3 have similar performance, while mode-1 CB Alt1 can have performance loss at smaller number of selected FD bases (M).

Observation 7: For Type-II-CJT, for FD selection Alt3 with FD index offset reported, it can support a similar FD permutation analogous to legacy Rel-16 for each TRP.

Observation 8: For Type-II-CJT, FD permutation is not meaningful for CJT, if TRP-dimension is the most significant dimension for group 1 and 2 split of NZCs.

Observation 9: Per-TRP Pc ratio definition can’t be guaranteed by the calculated PMI according to CSI measurement.

We propose:

Proposal 1: For Type-II-Doppler, support the first slot of WCSI as reference resource for CQI report (Alt2A).

Proposal 2: For Type-II-Doppler, decouple the definition of Rel-18 CSI reference resource and “timeline anchor,” where the “timeline anchor” can reuse Rel-15 CSI reference resource definition.

Proposal 3: For Type-II-Doppler, support 2-stage NZC selection bitmap:

- 1st preference (Alt3C): A first 2LM-bit bitmap, and a second QS-bit bitmap;

- 2nd preference (Alt3A): A first QM-bit bitmap, and a second 2LS-bit bitmap.

Proposal 4: For Type-II-Doppler, specify restriction on P/SP CSI-RS: A threshold/minimum number of CSI-RS occasions should be satisfied between the end of the triggering PDCCH and Z’ symbols prior to the report PUSCH.

Proposal 5: For Type-II-Doppler, study 2-stage PDCCH triggering mechanisms to prevent the latency and throughput reduction of UL-SCH due to the PUSCH conveying aperiodic Type-II-Doppler CSI: A 1st PDCCH to trigger CSI measurement/computation (and AP CSI-RS, if applicable), and, a 2nd PDCCH to trigger report.

Proposal 6: For Type-II-CJT, for the configuration of selected number of SD bases,

- 1st preference: Via ParamCombo Ltot,pv,β;

- 2nd preference: From a set of standard-defined {Ln}-combos.

Proposal 7: For Type-II-CJT, for the configuration of selected number of SD bases with NL>1 value combinations of {L1, ..., LNTRP}, a same value of n=1NTRPLn should be guaranteed.

Proposal 8: For Type-II-CJT, for FD basis selection,

- 1st preference: No need to support mode-1 FD-independent CB;

- 2nd preference: Mode-1 with TRP-independent FD selection (Alt2)

Proposal 9: For Type-II-CJT, for FD basis selection under mode-1 FD-independent codebook, largely reuse Rel-16 sTRP mechanism for independent FD basis selection of TRPs (Alt2):

- For N3≤19, SCI-TRP’s FD selection by log2N3-1Mv-1×rank bits, while N-1 remaining TRPs’ selections by N-1×log2N3Mv×rank bits independently;

- For N3>19,

  - For the 1st stage selection (window), SCI-TRP’s FD window by log22Mv×rank bits, while N-1 remaining TRPs’ windows by N-1×log2N3×rank bits independently (the N-1 remaining TRPs’ sized-2Mv windows may not comprise SCI-related FD basis);

  - For the 2nd stage selection, SCI-TRP’s by log22Mv-1Mv-1×rank bits, while N-1 remaining TRPs’ by N-1×log22MvMv×rank bits independently

Proposal 10: For Type-II-CJT, UCI packing of quantized coefficient use TRP-dimension as the outermost indexing.

Proposal 11: For Type-II-CJT, it should be a natural assumption for network implementation to allow variable transmission power for a CJT-PDSCH transmitted from a certain TRP.

Proposal 12: For Type-II-CJT, PDSCH-to-CSIRS Pc ratio is defined as the ratio between

- Summation of PDSCH power across all TRPs, and

- Summation of CSI-RS power across all TRPs

Pc=10log10PPDSCHtotalPCSI-RStotal=10log10n=1NPPDSCHTRP#nn=1NPCSI-RSTRP#n

Therefore, for CJT, Pc ratio is per CSI-RS resource set configured.

Proposal 13: For Type-II-CJT, support CMR timing restriction stricter than NCJT: All NZP CSI-RS resources in the resource set are received within one same slot, and no DL/UL switch in between.

Proposal 14: Defer the study of TRP-relative delay compensation to later than Rel-18.

Proposal 15: For TRS-based TDCP report, support time-correlation report (AltB), and allow UE implementation to use time correlation calculation by Aτ=kH*l-τ,kHl,kkHl-τ,k2kHl,k2.

Proposal 16: For TRS-based TDCP report, for time-correlation with longer lag, study single-port CSI-RS with new time spacing/pattern.

# References

- RP-213598, MIMO Evolution for Downlink and Uplink, Dec. 6th – 17th, 2021

- Chairman’s Notes RAN1#111, Nov. 14th – 18th, 2022

- Chairman’s Notes RAN1#110bis-e, Oct. 10th – 19th, 2022

- R1-2301248, Summary of OFFLINE discussion on Rel-18 MIMO CSI, Samsung, RAN1#112, Feb. 27th – Mar. 3rd, 2023

- R1-2205289, Rel-18 MIMO CSI EVM, May 9th – 20th, 2022