3GPP TSG RAN WG1 #112	R1-2301548

Athens, Greece, February 27th – March 3rd, 2023

Source:	Sharp

Title:	Views on Coverage enhancement for NR NTN

Agenda Item:	9.11.1

Document for:	Discussion and Decision

Introduction

In RAN1#111, the following conclusions and working assumptions were made for the coverage enhancement for NR NTN [1]:

Conclusion For the study of NTN-specific PUSCH DMRS bundling, RAN1’s understanding is that Phase variation due to constant frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-1 does not have impact on the phase continuity requirement for two adjacent slots specified as Table 6.4.2.5-1 in 38.101-1, according to annex F.9 and F.4 of 38.101-1. Conclusion RAN1 concluded that PUSCH DMRS bundling with sufficient TDW size should be applicable in NTN to meet the performance requirement for VoIP - FFS: How to determine TDW size, including UE capability. - Note: The above does not mean the performance requirements will be satisfied with DMRS bundling Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - One or more repetition factors may be configured via SIB - If only one repetition factor is configured via SIB and if the value is one of {[1], 2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor - FFS: whether UE requests repetition or indicates repetition capability - If multiple factors from {1, 2, 4, 8} are configured via SIB, PUCCH repetition for Msg4 HARQ-ACK may be dynamically determined and indicated by gNB - FFS: whether UE requests repetition or indicates repetition capability - FFS: whether repetition factor is indicated by UE - FFS: UE behavior when repetition factor is not configured via SIB - FFS: whether one or more UE capabilities are needed for the above is for further discussion

In this contribution, we share our views on coverage enhancement for NR NTN.

Discussions

Msg4 HARQ-ACK

In RAN1#111, the following working assumption was made for Msg4 HARQ-ACK repetition.

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - One or more repetition factors may be configured via SIB - If only one repetition factor is configured via SIB and if the value is one of {[1], 2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor - FFS: whether UE requests repetition or indicates repetition capability - If multiple factors from {1, 2, 4, 8} are configured via SIB, PUCCH repetition for Msg4 HARQ-ACK may be dynamically determined and indicated by gNB - FFS: whether UE requests repetition or indicates repetition capability - FFS: whether repetition factor is indicated by UE - FFS: UE behavior when repetition factor is not configured via SIB - FFS: whether one or more UE capabilities are needed for the above is for further discussion

Signaling for request or capability report

The possible ways to provide information from the UE to the gNB prior to Msg4 are very limited. Only Msg1 and Msg3 can be considered. For Msg 1, the information is provided by PRACH resource/sequence, which causes PRACH resource segmentation. As for Msg3, the payload is limited, and use of reserved values of LCID is the only way to carry additional information. In either way, the amount of additional information should be minimized, i.e., at most 1 bit. Moreover, the UE capable of Msg4 HARQ-ACK repetition does not always need repetitions. Therefore, the information the UE provides to the gNB should be whether to request Msg4 HARQ-ACK repetition. It should be noted that UEs not capable of Msg4 HARQ-ACK repetition cannot request Msg4 HARQ-ACK repetition. This is just like Rel-17 Msg3 repetition request.

For the signaling options, considering PRACH segmentation due to Msg 1 based scheme, signaling via LCID should be the most preferable. Only when RAN2 does not take this option, RAN1 may consider Msg 1 based scheme.

Proposal 1:

- UE informs gNB of whether to request Msg4 HARQ-ACK repetition via LCID value in MAC sub-header

  - Ask RAN2 if they take this option.

Determination of whether to request

In Rel-17 Msg3 repetition, whether the UE requests repetitions or not is determined based on DL RSRP level. For Msg4 HARQ-ACK repetition request, in the last meeting, the majority of companies supported the use of the similar metric with Rel-17 Msg3 repetition. However, no agreement was made, because it was not concluded whether UE requests the repetition for Msg 4 HARQ-ACK or not.

During the discussion in the last meeting, some companies commented that the repetition request can be up to UE implementation. However, in the NTN operation with Rel-18 Msg4 HARQ-ACK repetition, it is very likely that Rel-17 Msg3 repetition would also be applied, because Msg 3 PUSCH has normally less coverage than PUCCH for Msg4 HARQ-ACK. This means that RSRP based determination before the initial access is anyway required in such an NTN operation. Relying on UE implementation for only Msg4 HARQ-ACK does not simplify the procedure. Therefore, it is suggested that repetition request for Msg4 HARQ-ACK should be based on DL RSRP level.

Proposal 2:

- Repetition request of PUCCH for Msg4 HARQ-ACK is transmitted when the following condition is met:

  - The RSRP of the downlink pathloss reference is less than a threshold.

Dynamic indication of repetition factor

In Rel 17 Msg3 repetition, MCS field in RAR UL grant is used to indicate repetition factor. The reason is that the channel to which the MCS field value in RAR UL grant applies and the channel that the repetition is needed are the same. In this situation, higher MCSs are not necessary for the UE which requested Msg3 repetition, because the Msg 3 repetition request means that the expected Msg3 reception power at the gNB side would not be sufficient.

Here, we are discussing the enhancement on HARQ-ACK for Msg4. Unlike Msg3 repetition, the channel to which the MCS field value applies and the channel that the repetition is needed are different. The MCS field indicates the MCS for Msg4 PDSCH but not PUCCH for Msg4 HARQ-ACK. Even though the quality of PUCCH for Msg4 HARQ-ACK is predicted to be insufficient, that does not necessarily mean only lower MCSs are feasible for Msg4 PDSCH. Rather, that may limit the network scheduling on Msg4 PDSCH.

Observation 1:

- Even though the quality of PUCCH for Msg4 HARQ-ACK is predicted to be insufficient, that does not necessarily mean only lower MCSs are feasible for Msg4 PDSCH.

Looking at the information fields in DCI format 1_0 with TC-RNTI, 2-bit DAI field is reserved. The use of such a reserved field would be more preferable, because it has no impact on Msg4 PDSCH scheduling.

Proposal 3:

- 2-bit DAI field in the DCI format 1_0 with TC-RNTI is used to indicate a repetition factor which applies to the PUCCH carrying Msg4 HARQ-ACK.

DMRS bundling for PUSCH

One of the major discussions for DMRS bundling for PUSCH in NR NTN operation is which direction is taken between the following:

- Direction 1: Timing of UE pre-compensation update is determined first, and then actual TDW is decided based on the timing

- Direction 2: Actual TDW is determined first, and then UE pre-compensation update is allowed only at each boundary of two adjacent actual TDWs

In Rel-17 NR NTN, the timing of UE pre-compensation update is left to UE implementation. If this principle is violated, Rel-18 NR NTN would require NTN UEs to implement different UE pre-compensation processing from the legacy processing. Such a situation should be avoided.

Proposal 4:

- In Rel-18 NR NTN, the timing of UE pre-compensation update is left to UE implementation.

In Rel-17 DMRS bundling for CovEnh, the first nominal TDW starts at the first PUSCH repetition. In other words, the UE cannot be aware of boundaries of nominal TDW before the PUSCH is scheduled. If the timing of UE pre-compensation update is affected by the PUSCH scheduling, it would cause the new restriction on the UE pre-compensation update timing. This is not desirable. Therefore, it is suggested that actual TDW should be determined based on when UE performs pre-compensation update causing phase discontinuity and power inconsistency.

Proposal 5:

- Actual TDW should be determined based on when UE performs pre-compensation update causing phase discontinuity and power inconsistency.

Conclusion

In this contribution, we have the following observation and proposals:

Proposal 1:

- UE informs gNB of whether to request Msg4 HARQ-ACK repetition via LCID value in MAC sub-header

  - Ask RAN2 if they take this option.

Proposal 2:

- Repetition request of PUCCH for Msg4 HARQ-ACK is transmitted when the following condition is met:

  - The RSRP of the downlink pathloss reference is less than a threshold.

Observation 1:

- Even though the quality of PUCCH for Msg4 HARQ-ACK is predicted to be insufficient, that does not necessarily mean only lower MCSs are feasible for Msg4 PDSCH.

Proposal 3:

- 2-bit DAI field in the DCI format 1_0 with TC-RNTI is used to indicate a repetition factor which applies to the PUCCH carrying Msg4 HARQ-ACK.

Proposal 4:

- In Rel-18 NR NTN, the timing of UE pre-compensation update is left to UE implementation.

Proposal 5:

- Actual TDW should be determined based on when UE performs pre-compensation update causing phase discontinuity and power inconsistency.

References

“RAN1 Chairman’s Notes,” RAN1 #111, November 2022.