3GPP TSG RAN WG1 #112           	                                   R1- 2301596

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item: 7.1

Source: MediaTek Inc.

Title: On PUSCH power control adjustment state and TPC command value of Msg3 retransmission

Document for: Discussion and decision

- Introduction

In this contribution, we discuss the PUSCH power control adjustment state fb,f,ci,l and TPC command value δb,f,c of Msg3 retransmission. In 38.213 V17.4.0 [1] 7.1.1, there is

- related 38.213 spec on PUSCH power control adjustment state and TPC command value following a RACH flow (as shown in Section 2),

and also

- related 38.213 spec on PUSCH power control adjustment state and TPC command value following a normal flow (as shown in Section 3).

In current spec, for PUSCH power control adjustment state fb,f,ci,l and TPC command value δb,f,c of Msg3 retransmission, it is not clear UE should follow the RACH flow or the normal flow. The intention of this document is to fix/clarify this issue in 38.213 (as discussed in Section 4).

- Related 38.213 spec on PUSCH power control adjustment state and TPC command value following a RACH flow

In 38.213 V17.4.0 [1], there is related spec on PUSCH power control adjustment state and TPC command value following a RACH flow, which is for the case UE receives a random access response message in response to a PRACH transmission or a MsgA transmission:

7.1.1    UE behaviour

…

If the UE transmits a PUSCH …

-	If the UE receives a random access response message in response to a PRACH transmission or a MsgA transmission on active UL BWP b of carrier f of serving cell c as described in clause 8

-	fb,f,c0,l=∆Prampup,b,f,c+δmsg2,b,f,c, where l=0 and

-	δmsg2,b,f,c is a TPC command value indicated in a random access response grant of the random access response message corresponding to a PRACH transmission according to Type-1 random access procedure, or in a random access response grant of the random access response message corresponding to a MsgA transmission according to Type-2 random access procedure with RAR message(s) for fallbackRAR, on active UL BWP b of carrier f of serving cell c, and …

8.2    Random access response - Type-1 random access procedure

…

Table 8.2-1: Random Access Response Grant Content field size

| RAR grant field | Number of bits |
| --- | --- |
| Frequency hopping flag | 1 |
| PUSCH frequency resource allocation | 14, for operation without shared spectrum channel access 12, for operation with shared spectrum channel access |
| PUSCH time resource allocation | 4 |
| MCS | 4 |
| TPC command for PUSCH | 3 |
| CSI request | 1 |
| ChannelAccess-CPext | 0, for operation without shared spectrum channel access 2, for operation with shared spectrum channel access |

Table 8.2-2: TPC Command δmsg2,b,f,c for PUSCH

| TPC Command | Value (in dB) |
| --- | --- |
| 0 | -6 |
| 1 | -4 |
| 2 | -2 |
| 3 | 0 |
| 4 | 2 |
| 5 | 4 |
| 6 | 6 |
| 7 | 8 |

Observation 1: In 38.213 V17.4.0 [1], there is related spec on PUSCH power control adjustment state and TPC command value following a RACH flow as shown above, which is for the case UE receives a random access response message in response to a PRACH transmission or a MsgA transmission. For the TPC command δmsg2,b,f,c of PUSCH power control adjustment state fb,f,c0,l, it occupies 3 bits and covers a range of -6dB to 8dB as shown in Table 8.2-1 and Table 8.2-2 of 38.213 [1].

- Related 38.213 spec on PUSCH power control adjustment state and TPC command value following a normal flow

In 38.213 V17.4.0 [1], there is related spec on PUSCH power control adjustment state and TPC command value following a normal flow, which is for the PUSCH scheduled by DCI 0_0/0_1/0_2:

7.1.1    UE behaviour

…

For the PUSCH power control adjustment state fb,f,c(i,l) for active UL BWP b of carrier f of serving cell c in PUSCH transmission occasion i

…

-	fb,f,ci,l=fb,f,ci-i0,l+m=0CDi-1δPUSCH,b,f,c(m,l) is the PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion i if the UE is not provided tpc-Accumulation, where

-	The δPUSCH,b,f,c values are given in Table 7.1.1-1

…

-	fb,f,ci,l=δPUSCH,b,f,ci,l is the PUSCH power control adjustment state for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion  if the UE is provided tpc-Accumulation, where

-	δPUSCH,b,f,c absolute values are given in Table 7.1.1-1

…

Table 7.1.1-1: Mapping of TPC Command Field in a DCI format scheduling a PUSCH transmission, or in DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, or in DCI format 2_3, to absolute and accumulated δPUSCH,b,f,c values or δSRS,b,f,c values

| TPC Command Field | Accumulated δPUSCH,b,f,c or δSRS,b,f,c [dB] | Absolute δPUSCH,b,f,c or δSRS,b,f,c [dB] |
| --- | --- | --- |
| 0 | -1 | -4 |
| 1 | 0 | -1 |
| 2 | 1 | 1 |
| 3 | 3 | 4 |

Observation 2: In 38.213 V17.4.0 [1], there is related spec on PUSCH power control adjustment state and TPC command value following a normal flow as shown above, which is for the PUSCH scheduled by DCI 0_0/0_1/0_2. For the TPC command δPUSCH,b,f,c of PUSCH power control adjustment state fb,f,ci,l, it occupies 2 bits and covers a range of -1dB to 3dB as shown in Table 7.1.1-1 of 38.213 [1].

- Discussion on PUSCH power control adjustment state and TPC command value of Msg3 retransmission

From the 38.213 [1] 7.1.1 spec shown in Section 2, for Msg3 first transmission in the case UE receives a random access response message in response to a PRACH transmission or a MsgA transmission, the TPC command δmsg2,b,f,c follows Table 8.2-2 (3 bits) in 38.213 [1]. However, for Msg3 retransmission, it would be scheduled by DCI format 0_0 which has only 2 bits for the TPC command [2]. In this case, it is not reasonable to follow Table 8.2-2 (3 bits) in 38.213 [1] as a 2-bits TPC command can only cover -6dB to 0dB.

Observation 3: From the 38.213 [1] 7.1.1 spec shown in Section 2, for Msg3 first transmission in the case UE receives a random access response message in response to a PRACH transmission or a MsgA transmission, the TPC command δmsg2,b,f,c follows Table 8.2-2 (3 bits) in 38.213 [1]. However, for Msg3 retransmission, it would be scheduled by DCI format 0_0 which has only 2 bits for the TPC command [2]. In this case, it is not reasonable to follow Table 8.2-2 (3 bits) in 38.213 [1] as a 2-bits TPC command can only cover -6dB to 0dB.

Hence, we have the following proposal:

Proposal 1: RAN1 to agree on the following for Rel-17 and later releases:

  - For Msg3 retransmission, the TPC command follows Table 7.1.1-1 (2 bits) in 38.213.

Besides, for Msg 3 retransmission, the power should be able to be incremented to enable a more reliable retransmission, so the PUSCH power control adjustment state fb,f,ci,l should follow the close-loop formula similar to normal PUSCH.

Proposal 2: RAN1 to agree on the following for Rel-17 and later releases, to allow Msg 3 retransmission power to be incremented to enable a more reliable retransmission:

  - For Msg3 retransmission, the PUSCH power control adjustment state fb,f,ci,l follows the close-loop formula as

fb,f,ci,l=fb,f,c0,l+m=1iδPUSCH,b,f,c(m,l), where l=0

Proposal 3: RAN1 to develop a corresponding CR for Proposal 2 for Rel-17 and later releases.

- Conclusion

In this contribution, we discuss the PUSCH power control adjustment state and TPC command value of Msg3 retransmission and have the following observations and proposals:

Observation 1: In 38.213 V17.4.0 [1], there is related spec on PUSCH power control adjustment state and TPC command value following a RACH flow as shown in Section 2, which is for the case UE receives a random access response message in response to a PRACH transmission or a MsgA transmission. For the TPC command δmsg2,b,f,c of PUSCH power control adjustment state fb,f,c0,l, it occupies 3 bits and covers a range of -6dB to 8dB as shown in Table 8.2-1 and Table 8.2-2 of 38.213 [1].

Observation 2: In 38.213 V17.4.0 [1], there is related spec on PUSCH power control adjustment state and TPC command value following a normal flow as shown in Section 3, which is for the PUSCH scheduled by DCI 0_0/0_1/0_2. For the TPC command δPUSCH,b,f,c of PUSCH power control adjustment state fb,f,ci,l, it occupies 2 bits and covers a range of -1dB to 3dB as shown in Table 7.1.1-1 of 38.213 [1].

Observation 3: From the 38.213 [1] 7.1.1 spec shown in Section 2, for Msg3 first transmission in the case UE receives a random access response message in response to a PRACH transmission or a MsgA transmission, the TPC command δmsg2,b,f,c follows Table 8.2-2 (3 bits) in 38.213 [1]. However, for Msg3 retransmission, it would be scheduled by DCI format 0_0 which has only 2 bits for the TPC command [2]. In this case, it is not reasonable to follow Table 8.2-2 (3 bits) in 38.213 [1] as a 2-bits TPC command can only cover -6dB to 0dB.

Proposal 1: RAN1 to agree on the following for Rel-17 and later releases:

  - For Msg3 retransmission, the TPC command follows Table 7.1.1-1 (2 bits) in 38.213.

Proposal 2: RAN1 to agree on the following for Rel-17 and later releases, to allow Msg 3 retransmission power to be incremented to enable a more reliable retransmission:

  - For Msg3 retransmission, the PUSCH power control adjustment state fb,f,ci,l follows the close-loop formula as

fb,f,ci,l=fb,f,c0,l+m=1iδPUSCH,b,f,c(m,l), where l=0

Proposal 3: RAN1 to develop a corresponding CR for Proposal 2 for Rel-17 and later releases.

- Reference

3GPP TS 38.213 V17.4.0 5G; NR; Physical layer procedures for control

3GPP TS 38.212 V17.4.0 5G; NR; Multiplexing and channel coding