3GPP TSG-RAN WG1 #112	R1- 2301668

Athens, Greece, February 27th – March 3rd , 2023

Agenda Item:	8.12

Source:	Ericsson

Title:	draft reply LS on SPS configuration for unicast and multicast

Document for:	Discussion

# Introduction

In the RAN2 LS “draft reply LS on SPS configuration for unicast and multicast” [1], RAN2 askes two questions related to SPS configuration for unicast and multicast:

LS from R1-2300015 RAN2 discussed unicast and multicast SPS configuration and made following agreements for Rel-17: Check with RAN1 whether NW can configure SPS in one BWP using sps-Config and sps-ConfigMulticastToAddModList-r17 simultaneously. Check with RAN1 whether other indexes than 0 can be used if a single DL SPS configuration for multicast is configured.. In the description for sps-Config (IE for single unicast SPS) in the current RRC spec, there’s a restriction that NW cannot configure SPS in one BWP using sps-Config and sps-ConfigToAddModList (IE for one or multiple unicast SPS) simultaneously: sps-Config UE specific SPS (Semi-Persistent Scheduling) configuration for one BWP. Except for reconfiguration with sync, the NW does not reconfigure sps-Config when there is an active configured downlink assignment (see TS 38.321 [3]). However, the NW may release the sps-Config at any time. Network can only configure SPS in one BWP using either this field or sps-ConfigToAddModList.. Q1: RAN2 would like to ask RAN1’s view on whether similar restriction is required when configuring SPS for both unicast and multicast in one BWP, i.e., network cannot use sps-Config to configure unicast SPS and simultaneously use sps-ConfigMulticastToAddModList-r17 to configure multicast in one BWP. Q2: Another question RAN2 would like to ask RAN1 is that when a single multicast SPS is configured for a group of UEs and there is no SPS configuration for unicast for at least one UE in the group, whether sps-ConfigIndex other than 0 can be configured by network for the single multicast SPS.

# discussion

## View on Q1

The restriction for using SPS-config or SPS-ConfigToAddModList is due to the fact that SPS-config is meant for UEs supporting only a single SPS configuration, while SPS-ConfigToAddModList-r16 is for UEs supporting multiple configurations.  The restriction should therefore apply also for multicast SPS.

Reqarding Q1, reply to RAN2 that the network cannot use sps-Config to configure SPS and simultaneously use sps-ConfigMulticastToAddModList-r17 to configure multicast in one BWP for a given UE.

## View on Q2

If a single multicast SPS is configured for a group of UEs, it may be configured in different ways:

- Using SPS-config, if all UEs do not support multiple SPS configurations

- Using a single entry in sps-ConfigMulticastToAddModList-r17

Thus the answer to Q2 depends on the UE capabilities. If at least 1 UE only supports a single configuration of SPS, then all UEs must use configuration 0. In other cases another index can be used.

Reqarding Q2, reply to RAN2 that when a single multicast SPS is configured for a group of UEs and there is no SPS configuration for unicast for at least one UE the network may configure a multicast with index different from 0, if all UEs in the group support multiple SPS configurations for multicast. Otherwise, the configuration index must be 0.

# References

R1-2300015 LS on SPS configuration for unicast and multicast, RAN2

# draft reply LS

3GPP TSG RAN WG1 #112			R1-23NNNN

Athens, Greece, February 27th – March 3rd, 2023

# Title:	[DRAFT] reply LS on SPS configuration for unicast and multicast

# Response to:	R1-2300015

# Release:	Release 17

# Work Item:	NR_MBS-Core

Source:	RAN1

To:	RAN2

Cc:	-

Contact Person:

#### Name:	Florent Munier

#### E-mail Address:	florent.munier@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN1 would like to thank RAN2 for the questions in the LS in R1-2300015. The questions are copied below for convenience:

RAN2 discussed unicast and multicast SPS configuration and made following agreements for Rel-17: Check with RAN1 whether NW can configure SPS in one BWP using sps-Config and sps-ConfigMulticastToAddModList-r17 simultaneously. Check with RAN1 whether other indexes than 0 can be used if a single DL SPS configuration for multicast is configured.. In the description for sps-Config (IE for single unicast SPS) in the current RRC spec, there’s a restriction that NW cannot configure SPS in one BWP using sps-Config and sps-ConfigToAddModList (IE for one or multiple unicast SPS) simultaneously: sps-Config UE specific SPS (Semi-Persistent Scheduling) configuration for one BWP. Except for reconfiguration with sync, the NW does not reconfigure sps-Config when there is an active configured downlink assignment (see TS 38.321 [3]). However, the NW may release the sps-Config at any time. Network can only configure SPS in one BWP using either this field or sps-ConfigToAddModList.. Q1: RAN2 would like to ask RAN1’s view on whether similar restriction is required when configuring SPS for both unicast and multicast in one BWP, i.e., network cannot use sps-Config to configure unicast SPS and simultaneously use sps-ConfigMulticastToAddModList-r17 to configure multicast in one BWP. Q2: Another question RAN2 would like to ask RAN1 is that when a single multicast SPS is configured for a group of UEs and there is no SPS configuration for unicast for at least one UE in the group, whether sps-ConfigIndex other than 0 can be configured by network for the single multicast SPS.

Based on the discussion in RAN1#112, RAN1 reached the following conclusions:

For Q1: it is RAN1’s view that the network cannot use sps-Config to configure SPS and simultaneously use sps-ConfigMulticastToAddModList-r17 to configure multicast in one BWP for a given UE.

For Q2: It is RAN1’s view that when a single multicast SPS is configured for a group of UEs and there is no SPS configuration for unicast for at least one UE the network may configure a multicast with index different from 0, if all UEs in the group support multiple SPS configurations for multicast. Otherwise, the configuration index must be 0.

2. Actions:

To RAN2:

ACTION: 	RAN1 respectfully asks RAN2 to take the above answers into account.

3. Date of Next RAN1 Meetings:

RAN1#112b-e	2023‑04‑17 - 2023‑04‑26, online

RAN1#113	2023‑05‑22 - 2023‑05‑26, Incheon, Korea