3GPP TSG-RAN WG1 Meeting #112	R1-2301767

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.8.2

Source:	Ericsson

Title:	Other aspects including control signalling and procedures in NCR

Document for:	Discussion, Decision

# Introduction

In this contribution, we discuss our view on required NCR-MT capabilities considering the NCR deployment, and the behavior of NCR-Fwd when BFR/RLF happen in the control link. In addition to this contribution, we discuss signalling aspects for the required control information in our accompanying contribution [1].

# Discussion

## NCR-MT’s beam management capability

For Rel-18 NCR, the following agreements regarding legacy CSI measurement/report mechanisms and sounding procedure was made in the RAN1#110bis-e meetings [2].

Agreement (RAN1#110bis-e) To support CSI measurement/reporting mechanisms for NCR-MT in C-link - The necessary legacy mechanism for receiving CSI-RS is reused for NCR-MT. - The necessary legacy mechanism for reporting CSI is reused for NCR-MT. - FFS: The details of the necessary mechanisms will be further discussed and decided. - Note: this does not mean all the legacy procedures for receiving CSI-RS and reporting CSI will be supported. Agreement (RAN1#110bis-e) To support the sounding procedure for NCR-MT in C link, the necessary mechanism of legacy UE sounding procedure is supported. - FFS: The details of the necessary mechanism of legacy UE sounding procedure. - Note: This does not mean all legacy UE sounding procedure will be supported.

The FFSs in both the above agreements can be discussed together with the RAN1 capabilities which are needed for the basic operation of operator deployed NCR, i.e., those features without which the system cannot operate. Specifically, RAN1 should not modify existing UE functionalities that have been carefully developed and specified. As such, we think, a repeater node, similar to an IAB-node, is a stationary node deployed according to the network planning done by the operator. The planning can be expected to result in a stable control/backhaul link between the controlling gNB and the NCR. Hence, many UE features related to mobility and measurement are not needed for NCR-MTs. As a baseline, the NCR-MT can have the same mandatory features as an IAB-MT agreed in the RAN1#101-e meeting [3], where specifically basic CSI feedback (2-32) and basic SRS (2-52) [4] below are included, cf. Table 1.

Agreement (RAN1#101-e) - Wide-area IAB-MTs support the following Rel. 15 layer-1 mandatory UE features (as defined in TR38.822) - Without capability - 0-1, 0-3, 0-4, 1-1 (only 1 preamble for component 1, component 2, component 3 except paging), 2-1, 2-5, 2-6, 2-12, 2-16, 2-16a, 2-32 (only components 1-4 and 7), 2-50 (only components 1,2), 2-52 (only components 1, 2), 3-1 (only components 1,2,3,4,5), 4-1, 5-1 (only components 1/2/3/4/5/6/7/9/10/12), 6-1, 7-1, 8-3 - With capability signaling which shall be set to '1' - 1-3, 2-22, 4-10 - The rest of Rel-15 layer-1 UE features other than the ones as listed above are optional for wide-area IAB-MTs. - Note: Mandatory MT capabilities are independent from DU capabilities and do not imply a corresponding mandatory DU capability. - The UE feature list for local-area IAB-MTs is FFS

Table 1: Mandatory features for IAB-MT.

| ### Features | Index | Feature group | Mandated component | Without capability | With capability set to 1 |
| --- | --- | --- | --- | --- | --- |
| 0. Waveform, modulation, subcarrier spacings, and CP | 0-1 | CP-OFDM waveform for DL and UL |  | Yes |  |
|  | 0-3 | DL modulation scheme |  | Yes |  |
|  | 0-4 | UL modulation scheme |  | Yes |  |
| 1. Initial access and mobility | 1-1 | Basic initial access channels and procedures | 1 preamble for component 1, component 2, component 3 except paging | Yes |  |
|  | 1-3 | SS block based RLM |  |  | Yes |
| 2. MIMO | 2-1 | Basic PDSCH reception |  | Yes |  |
|  | 2-5 | Basic downlink DMRS for scheduling type A |  | Yes |  |
|  | 2-6 | Basic downlink DMRS for scheduling type B |  | Yes |  |
|  | 2-12 | Basic PUSCH transmission |  | Yes |  |
|  | 2-16 | Basic uplink DMRS (uplink) for scheduling type A |  | Yes |  |
|  | 2-16a | Basic uplink DMRS for scheduling type B |  | Yes |  |
|  | 2-22 | Aperiodic beam report |  |  | Yes |
|  | 2-32 | Basic CSI feedback | component 1-4 and 7 | Yes |  |
|  | 2-50 | Basic TRS | components 1,2 | Yes |  |
|  | 2-52 | Basic SRS | components 1,2 | Yes |  |
| 3. DL control channel and procedure | 3-1 | Basic DL control channel | components 1,2,3,4,5 | Yes |  |
| 4. UL control channel and procedure | 4-1 | Basic UL control channel |  | Yes |  |
|  | 4-10 | Dynamic HARQ-ACK codebook |  |  | Yes |
| 5. Scheduling/HARQ operation | 5-1 | Basic scheduling/HARQ operation | components 1,2,3,4,5,6,7,9,10,12 | Yes |  |
| 6. CA/DC, BWP, SUL | 6-1 | Basic BWP operation with restriction |  | Yes |  |
| 7. Channel coding | 7-1 | Channel coding |  | Yes |  |
| 8. UL TPC | 8-3 | Basic power control operation |  | Yes |  |

The deployment situation for an IAB-node and NCR is similar, hence the required features for IAB-MT and NCR-MT are similar.

Use the identified and agreed IAB-MT capabilities as a starting point for repeater-MT capabilities.

## Beam Link Failure

The following agreement was made for NCR-MT regarding Rel-15 legacy BFD/BFR/RLM in the RAN1#111 meeting [5].

Agreement (RAN1#111) As optional functionalities for the NCR-MT, at least Rel-15 legacy BFD/BFR/RLM mechanisms are supported - FFS: The behavior of NCR-Fwd when BFR/RLF happen in C link.

Since in the RAN2#120 meeting [6], RAN2 has already agreed to the following behavior for NCR-Fwd on NCR-MT RLF, hence, we do not see a need to further discuss RLF in RAN1.

Agreement (RAN2#120) On NCR-MT RLF: - After RLF is declared by NCR-MT, NCR-MT performs cell selection and trigger RRC re-establishment; - If NCR-MT enters RRC_IDLE due to no suitable cell is find, NCR-Fwd is OFF; - During RRC re-establishment procedure, NCR-Fwd is OFF.

There are RAN2 agreements on the behaviour for NCR-Fwd on NCR-MT RLF.

As agreed in the RAN1#109-e meeting [7], the control link and the backhaul link are expected to experience the same large-scale channel properties, the BFR of both control and backhaul links will be based on the measurement done by NCR-MT. The NCR-MT can measure the backhaul link beam by measuring on the NCR-MT’s TCI states which include the backhaul beam (the backhaul DL beam is selected from the RRC-configured list of beams for C-link). Since the gNB can be fully aware of the beam link failure of the control and/or backhaul link, the gNB can avoid the configuration of side control information during the beam link failure, or legacy BFR mechanism (i.e., multi-TRP BFR) can also be applied to the backhaul link. In any case, during the beam link failure of the control link or backhaul link, NCR-Fwd should be OFF.

Agreement (RAN1#109-e) Capture the following assumption of network-controlled repeater in TR 38.867. - As baseline, same large-scale properties of the channel, i.e., channel properties in Type-A and Type-D (if applicable), are expected to be experienced by C-link and backhaul link (at least when the NCR-MT and NCR-Fwd operating in same carrier).

As an optional functionality of the NCR-MT, legacy BFR mechanism (i.e., multi-TRP BFR) can also be applied to the backhaul link, if the NCR supports adaptive beams in control and backhaul links.

During the beam link failure of the control or backhaul link, NCR-Fwd is OFF.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The deployment situation for an IAB-node and NCR is similar, hence the required features for IAB-MT and NCR-MT are similar.

Observation 2	There are RAN2 agreements on the behaviour for NCR-Fwd on NCR-MT RLF.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Use the identified and agreed IAB-MT capabilities as a starting point for repeater-MT capabilities.

Proposal 2	As an optional functionality of the NCR-MT, legacy BFR mechanism (i.e., multi-TRP BFR) can also be applied to the backhaul link, if the NCR supports adaptive beams in control and backhaul links.

Proposal 3	During the beam link failure of the control or backhaul link, NCR-Fwd is OFF.

# References

R1-2301766, “Side-control information and NCR behavior,” Ericsson, RAN1 #112, February 2023.

“Chairman’s Notes”, 3GPP TSG RAN1 #110-bis, October 2022

“RAN1 Chairman’s Notes”, 3GPP TSG RAN1 #101-e, June 2020.

TR 38.822, “User Equipment (UE) feature list”, 3GPP, V16.4.0, December 2022.

“Chairman’s Notes”, 3GPP TSG RAN1 #111, November 2022.

R2-2213009, “Report from NCR Repeater breakout session”, Session Chair (Apple), 3GPP TSG-RAN WG2 Meeting #120, Toulouse, France, November 2022.

“Chairman’s Notes”, 3GPP TSG RAN1 #109-e, May 2022.