3GPP TSG RAN WG1 #112bis-e	        		   			R1-230xxxx

# e-Meeting, April 17th – 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Network Controlled Repeaters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NCR |  |  |  |  |  | Date: |  |  | 2023-04-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of network controlled repeaters (NCR) in NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce support for NCR in NR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for NCR in NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 20 (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS 38.321, TS 38.331 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Unchanged parts are omitted ***

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in [1, TR 21.905].

## *** Unchanged parts are omitted ***

MBS	Multicast broadcast services

MCG	Master cell group

MCS	Modulation and coding scheme

NCR	Network controlled repeater

NCR-FWD	NCR forwarding

NCR-MT	NCR mobile termination

NDI	New Data Indicator

NE-DC	E-UTRA NR dual connectivity with MCG using NR and SCG using E-UTRA

NR-DC	NR NR dual connectivity

PBCH	Physical broadcast channel

## *** Unchanged parts are omitted ***

# 20	Network controlled repeater

An NCR includes an NCR-MT entity and an NCR-FWD entity. The NCR-MT communicates with a serving cell via a control link based on the Uu interface. The NCR-FWD provides a communication link between a serving cell and UEs via  the backhaul link for signalling between the serving cell and the NCR-FWD, and the access link for signalling between the NCR-FWD and the UEs.

Throughout this specification, unless otherwise noted, statements using the term "UE" in Clauses 4 through 13 are equally applicable to the NCR-MT.

A procedure for the NCR-MT to perform cell search, system information acquisition, random access procedure, UCI reporting, or PDCCH monitoring is same as a corresponding one for a UE. A procedure for the NCR-MT to perform PDSCH reception, CSI-RS measurements and CSI determination, PUSCH transmission, or SRS transmission is same as a corresponding one for a UE as described in [6, TS 38.214].

The NCR-FWD transmits or receives only after the NCR-MT receives an indication for one or more beams [20, TS 38.106] for the NCR-FWD to use for transmissions or receptions over corresponding one or more time resources on the control link. In the following, a beam is associated with QCL parameters for transmission by the NCR-FWD or the NCR-MT, or with QCL parameters for reception by the NCR-FWD or the NCR-MT.

When the NCR-MT performs a link recovery procedure as described in Clause 6, the NCR-FWD does not transmit or receive until the link recovery procedure is complete [11 TS 38.321].

The reception timing on the backhaul link and on the control link is same.

The NCR can be provided, through the NCR-MT, tdd-UL-DL-ConfigurationCommon and, additionally, tdd-UL-DL-ConfigurationDedicated. The NCR-FWD receives on the backhaul link or transmits on the access link only in symbols indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. The NCR-FWD receives on the access link or transmits on the backhaul link only in symbols indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

When the NCR simultaneously receives via both the control link and the backhaul link in a set of symbols, a TCI state for receptions on the backhaul link is same as a TCI state for receptions on the control link in the set of symbols. When the NCR simultaneously transmits via both the control link and the backhaul link in a set of symbols, a spatial filter for transmissions on the backhaul link is same as a spatial filter for transmissions on the control link in the set of symbols.

When the NCR does not simultaneously receive on the control link and the backhaul link

-	if the NCR does not support determination of a TCI state for receptions on the backhaul link based on an indication of a TCI state by the serving cell, or if the NCR does not receive an indication of a TCI state, for receptions on the backhaul link [11, TS 38.321]

-	if the NCR does not receive an indication of a unified TCI state for receptions by the NCR-MT, receptions on the backhaul link use same QCL parameters as the ones for PDCCH receptions in a CORESET with the lowest controlResourceSetId

-	else, receptions on the backhaul link use the QCL parameters provided by an indicated unified TCI state for receptions by the NCR-MT

-	else receptions on the backhaul link use QCL parameters provided by a TCI state in a MAC CE [11, TS 38.321].

When the NCR does not simultaneously transmit on the control link and the backhaul link

-	if the NCR does not support determination of a spatial filter for transmissions on the backhaul link based on an indication of a unified TCI state or SRI by the serving cell, or if the NCR-MT does not receive an indication of a unified TCI state or SRI for determining a spatial filter, for transmissions on the backhaul link

-	if the NCR does not receive an indication of a unified TCI state for transmissions by the NCR-MT, transmissions on the backhaul link use a same spatial filter as the one associated with the PUCCH resource with the smallest pucch-ResourceId in PUCCH-ResourceSet

-	else, transmissions on the backhaul link use a spatial filter corresponding to an indicated unified TCI state for transmissions by the NCR-MT.

-	else transmissions on the backhaul link use a spatial filter corresponding to a TCI state or SRI provided by a MAC CE [11, TS 38.321].

The NCR-FWD uses a same beam for transmissions and receptions on the access link during respective time resources associated with the beam.

The NCR can be provided by ncr-PeriodicFwdResourceSetToAddModList a list of sets of resources for transmissions or receptions on the access link. A set of resources, from the list of sets of resources, is provided by NCR-PeriodicFwdResourceSet and occurs with a periodicity provided by ncr-periodicity. A resource from the set of resources is provided by NCR-PeriodicFwdResource and includes a pair of a time resource provided by ncr-PeriodicTimeResource and a beam [20, TS 38.106] with an index provided by ncr-beamIndex. The time resource starts at a slot that is offset by slotOffsetPeriodic slots from the start of the period for the set of resources and at a symbol that is offset by symbolOffset from the start of the slot, and has a duration provided by durationInSymbols for a SCS provided by ncr-referenceSCS.

The NCR can be provided by ncr-SemiPersistentFwdResourceSetToAddModList a list of sets of resources for transmissions or receptions on the access link and a MAC CE command can indicate a set of resources for the NCR to use or to stop using based on a corresponding identity provided by ncr-SemiPersistentFwdResourceSetId [11, TS 38.321]. The NCR uses or stops using the set of resources starting from the first slot that is after slot k+3Nslotsubframe,μ where k is the slot where the NCR-MT would transmit a PUCCH with HARQ-ACK information associated with the PDSCH providing the MAC CE command and μ is the SCS configuration for the PUCCH transmission. The set of resources is provided by NCR-SemiPersistentFwdResourceSet and occurs with a periodicity provided by ncr-periodicity. A resource from the set of resources is provided by NCR-SemiPersistentFwdResource and includes a pair of a time resource provided by ncr-SemiPersistentTimeResource and a beam with an index provided by ncr-beamIndex, where beamIndex can be updated by the MAC CE command. The time resource starts at a slot that is offset by slotOffsetSemiPersistent slots from the start of the period for the set of resources and at a symbol that is offset by symbolOffset from the start of the slot, and has a duration provided by durationInSymbols for a SCS provided by ncr-referenceSCS.

The NCR-MT can be configured to monitor PDCCH according to USS sets for detection of a DCI format 5_0 with CRC scrambled by an NCR-RNTI. A time resource and a corresponding beam index for transmissions or receptions on the access link are indicated by corresponding fields in DCI format 5_0 [4, TS 38.212]. When the NCR detects more than one DCI formats 5_0 that indicate beam indexes for time resources overlapping in a set of symbols, the NCR uses for the set of symbols a beam index that is indicated by a DCI format 5_0 that the NCR-MT detects in a most recent PDCCH monitoring occasion. The time resource starts at a slot that is offset by slotOffsetAperiodic slots from a reference slot and at a symbol that is offset by symbolOffset from the start of the slot, and has a duration provided by durationInSymbols for a SCS provided by ncr-referenceSCS. The reference slot is a slot that is after a slot of a PDCCH reception that provides the DCI format 5_0 by a number of slots indicated by XXX.

If

-	a first time resource provided by NCR-SemiPersistentFwdResourceSet is indicated by a MAC CE command and is associated with a first beam index, and

-	a second time resource is provided by NCR-PeriodicFwdResourceSet and is associated with a second beam index, and

-    the first time resource overlaps with the second time resource in a set of symbols, and

the NCR applies the first beam index for transmissions or receptions on the access link in the set of symbols.

If

-	a first time resource is provided by NCR-PeriodicFwdResourceSet or NCR-SemiPersistentFwdResourceSet and is associated with a first beam index, and

-	a second time resource is indicated by DCI format 5_0 and is associated with a second beam index provided by the DCI format 5_0, and

-    the first time resource overlaps with the second time resource in a set of symbols,

the NCR applies, for transmissions or receptions on the access link in the set of symbols,

-	the first beam index if NCR-PeriodicFwdResourceSet or NCR-SemiPersistentFwdResourceSet includes priorityFlag, and

-	the second beam index if NCR-PeriodicFwdResourceSet or NCR-SemiPersistentFwdResourceSet does not include priorityFlag.

The NCR does not expect overlapping time resources provided by either NCR-PeriodicFwdResourceSet or NCR-SemiPersistentFwdResourceSet to be associated with different beam indexes.