3GPP TSG-RAN WG1 Meeting #113		R1-2304454

Incheon, Korea, May 22nd – May 26th, 2023

Title:	Draft LS reply on report of switching periods in Rel-18 UL Tx switching

Response to:	R1-2304333 (R2-2304567)

Release:	Rel-18

Work Item:	NR_MC_enh-Core

Source:	vivo [RAN1]

To:	RAN2

Cc:	RAN4

Contact person:	Siqi Liu

liusiqi@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

RAN1 thanks RAN2 for the LS informing about the work on report of switching periods in Rel-18 UL Tx switching. Regarding the following questions, RAN1 would provide the following answers.

Question 1. (To RAN1 and RAN4) RAN2 respectfully asks RAN1 and RAN4 to take above agreement on RAN2 intention into account and asks for feedback if there is any issue. RAN2 could not conclude whether the UE needs to explicitly report if it supports 2Tx-2Tx switching for every band pair used for Rel-18 UL Tx switching. RAN2 is not sure which is the correct understanding: - The UE always supports 2Tx-2Tx switching on a pair of bands if the UE supports 2 layers/ports UL MIMO on the two bands - The UE may not support 2Tx-2Tx switching on a pair of bands even if the UE supports 2 layers/ports UL MIMO on the two bands (i.e., per-band-pair UE capability to report whether to support 2Tx-2Tx switching is needed, e.g. based on the presence/absence of 2Tx-2Tx switching period). Question 2. (To RAN4) RAN2 respectfully asks RAN4 to take below RAN2 assumptions into account and asks for feedback if there is any issue: - For the band pair supporting 2Tx-2Tx switching, the UE always support 1Tx-2Tx switching. - The UE reports whether it supports 2Tx-2Tx switching via per-band-pair UE capability. RAN2 has discussed how the gNB knows which of the reported switching periods (for 1Tx-2Tx switching or for 2Tx-2Tx) should be applied for every switching but could not conclude. Question 3. (To RAN4) RAN2 respectfully asks RAN4 which of the options below matches RAN4 understanding on the selection of applied switching periods when both switching periods of 2Tx-2Tx switching and 1Tx-2Tx switching can be reported for the same band pair. Option 1: Based on implicit rules, e.g. 2Tx-2Tx switching period is only applicable when performing UL switching between two bands (e.g. 2P+0P<=>0P+2P) and 1Tx-2Tx period is applied for the other switching cases (e.g. UL Tx switching that involves 3 or 4 bands, such as band A + band B<=>band C, band A+ band B <=>band C + band D). FFS on the switching case of 2P+0P<=>1P+1P. Option 2: Based on explicit RRC configuration, i.e., gNB configures which period is applied. FFS on the granularity of the configuration. Question 4. (To RAN1) RAN2 respectfully asks RAN1 to take above discussion on RAN2 and question to RAN4 into account and asks for feedback if there is any issue.

Answer:

- For Question 1, in RAN1’s understanding, the UE may not support 2Tx-2Tx switching on a pair of bands even if the UE supports 2 layers/ports UL MIMO on the two bands.

- For Question 4, in RAN1’s understanding, the gNB can configure which period to apply when both switching periods of 2Tx-2Tx switching and 1Tx-2Tx switching can be reported for the same band pair. If 2Tx-2Tx switching period is configured(e.g., by uplinkTxSwitching-2T-Mode), the switching period is applied for all switching cases regardless of the number of involved bands in the Tx switching.

2	Actions

To RAN2

RAN1 respectfully asks RAN2 to take the above information into account in related work.

3	Dates of next TSG-RAN WG1 meetings

TSG RAN WG1 Meeting #114			August 21th – 25th, 2023						Toulouse

TSG RAN WG1 Meeting #114bis			October 9th – 13th, 2023							Xiamen