3GPP TSG RAN WG1 #113	R1-2304507

Incheon, Korea, May 22nd – May 26th, 2023

Source:	vivo

Title:	Discussion on Rel-18 MIMO UE features

Agenda Item:	9.16.1

Document for:	Discussion and Decision

Introduction

In this contribution we provide initial views on UE features for sub-items in Rel-18 MIMO enhancement AI.

Discussion

Following tentative UE features for different sub-items are identified according to agreements up to RAN1#112b-e.

Unified TCI extension for MTRP

| xx | Feature | Detail |
| --- | --- | --- |
|  | Unified TCI with joint DL/UL TCI update for S-DCI based MTRP | Basic feature: 1. One MAC-CE activated full set of {1st joint TCI state, 2nd joint TCI state} for S-DCI based MTRP 2. TCI state indication for update and activation a) MAC CE based TCI state indication for full set of active {1st joint TCI state, 2nd joint TCI state} Agreement On unified TCI framework extension, consider all the intra and inter-cell MTRP schemes specified in Rel-16 and Rel-17 Consider, if STxMP is supported, Rel-18 MTRP scheme(s) with STxMP Agreement On unified TCI framework extension for S-DCI based MTRP operation, support the followings: - For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {first joint TCI state, second joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) |
|  | Unified TCI with joint DL/UL TCI update for S-DCI based MTRP with more than one MAC-CE activated joint TCI state | 1. More than one TCI codepoint with full set or subset of {1st joint TCI state, 2nd joint TCI state} for S-DCI based MTRP a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) Agreement On unified TCI framework extension for S-DCI based MTRP operation, support the followings: - For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {first joint TCI state, second joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) Agreement On unified TCI framework extension for S-DCI based MTRP, in one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL/UL TCI state(s) for one or both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list |
|  | Application of TCI state for AP CSI-RS | 1. {‘per CSI-RS resource set’, ‘per CSI-RS resource’} Agreement On unified TCI framework extension for S-DCI based MTRP, an RRC configuration can be provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTrigger State for each CSI-RS resource set or for each CSI-RS resource in each aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the CSI-RS resource if the aperiodic CSI-RS resource set for CSI/BM is configured to follow unified TCI state - Above applies at least if the offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in the aperiodic CSI-RS resource set is equal to or larger than a threshold (if the threshold is needed) - FFS: If the UE is configured for CSI-RS resource set, for an aperiodic CSI-RS resource set configured with two Resource Groups for NCJT CSI and configured to follow unified TCI state, if above RRC configuration is not provided to the aperiodic CSI-RS resource set, the UE shall apply the first indicated joint/DL TCI state to the CSI-RS resource(s) in Group 1 and the second indicated joint/DL TCI state to the CSI-RS resource(s) in Group 2. - ‘per CSI-RS resource set’ or ‘per CSI-RS resource’ is up to UE capability |
|  | Joint TCI state application to PDSCH-CJT | 1. Supported number of indicated joint TCI states for PDSCH-CJT, {1-basic, 2-optional} Agreement On unified TCI framework extension, up to 2 joint TCI states can be indicated by MAC-CE/DCI and applied to CJT-based PDSCH reception (PDSCH-CJT) in a BWP/CC configured with joint DL/UL TCI mode - Support of 1 or 2 indicated joint TCI states for PDSCH-CJT is up to UE capability - FFS: QCL type(s)/assumption(s) of the indicated joint TCI state(s) applied to PDSCH-CJT - Note: On how to inform UE to apply which indicated joint TCI state(s) to target channel(s)/signal(s) in the BWP/CC, it is discussed individually in AI 9.1.1.1 |
|  | Unified TCI with joint DL/UL TCI update for M-DCI based MTRP | Basic feature: 1. One MAC-CE activated joint TCI state per CORESETPoolIndex for M-DCI based MTRP 2. TCI state indication for update and activation per CORESETPoolIndex Agreement On unified TCI framework extension, consider all the intra and inter-cell MTRP schemes specified in Rel-16 and Rel-17 Consider, if STxMP is supported, Rel-18 MTRP scheme(s) with STxMP |
|  | Unified TCI with joint DL/UL TCI update for M-DCI based MTRP with more than one MAC-CE activated joint TCI state | 1. More than one TCI codepoint with a joint TCI state per CORESETPoolIndex for M-DCI based MTRP a) MAC-CE+DCI-based TCI state indication per CORESETPoolIndex (use of DCI formats 1_1/1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication per CORESETPoolIndex (use of DCI formats 1_1/1_2 without DL assignment) Agreement On unified TCI framework extension for M-DCI based MTRP: - The existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) associated with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value - FFS: The UE shall apply the indicated joint/DL/UL TCI state(s) specific to a coresetPoolIndex value to channel(s)/signal(s) that have explicit or implicit association with the same coresetPoolIndex value - A coresetPoolIndex value field is included in TCI state activation command (MAC-CE) to indicate that the mapping between the activated TCI state(s) and the TCI codepoint(s) is specific to which coresetPoolIndex value Agreement On unified TCI framework extension for M-DCI based MTRP: - For a serving cell configured with joint DL/UL TCI mode, one joint TCI state can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) |
|  | Unified TCI with separate DL/UL TCI update for S-DCI based MTRP | Basic feature: 1. One MAC-CE activated full set of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state} for S-DCI based MTRP 2. TCI state indication for update and activation a) MAC CE based TCI state indication for full set of active {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state} Agreement On unified TCI framework extension, consider all the intra and inter-cell MTRP schemes specified in Rel-16 and Rel-17 Consider, if STxMP is supported, Rel-18 MTRP scheme(s) with STxMP Agreement On unified TCI framework extension for S-DCI based MTRP operation, support the followings: - For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) |
|  | Unified TCI with separate DL/UL TCI update for S-DCI based MTRP with more than one MAC-CE activated separate TCI state | 1. More than one TCI codepoint with full set or subset of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state} for S-DCI based MTRP a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) Agreement On unified TCI framework extension for S-DCI based MTRP operation, support the followings: - For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) Agreement On unified TCI framework extension for S-DCI based MTRP, in one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL/UL TCI state(s) for one or both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list |
|  | Unified TCI with separate DL/UL TCI update for M-DCI based MTRP | Basic feature: 1. One MAC-CE activated full set of {DL TCI state, UL TCI state} per CORESETPoolIndex for M-DCI based MTRP 2. TCI state indication for update and activation per CORESETPoolIndex Agreement On unified TCI framework extension, consider all the intra and inter-cell MTRP schemes specified in Rel-16 and Rel-17 Consider, if STxMP is supported, Rel-18 MTRP scheme(s) with STxMP |
|  | Unified TCI with separate DL/UL TCI update for M-DCI based MTRP with more than one MAC-CE activated separate TCI state | 1. More than one TCI codepoint with full set or subset of {DL TCI state, UL TCI state} per CORESETPoolIndex for M-DCI based MTRP a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) Agreement On unified TCI framework extension for M-DCI based MTRP: - The existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) associated with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value - FFS: The UE shall apply the indicated joint/DL/UL TCI state(s) specific to a coresetPoolIndex value to channel(s)/signal(s) that have explicit or implicit association with the same coresetPoolIndex value - A coresetPoolIndex value field is included in TCI state activation command (MAC-CE) to indicate that the mapping between the activated TCI state(s) and the TCI codepoint(s) is specific to which coresetPoolIndex value Agreement On unified TCI framework extension for M-DCI based MTRP: - For a serving cell configured with separate DL/UL TCI mode, a DL TCI state, an UL TCI state, or a pair of DL and UL TCI states can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) |

Support of 2TA

| xx | Feature | Detail |
| --- | --- | --- |
|  | 2TAs for UL M-DCI based MTRP operation, intra-cell | Agreement Enhancement on two TAs for UL multi-DCI for multi-TRP operation is supported in Rel-18. Note 1: whether (1) the network signals two TACs or (2) the network signals one TAC and the UE deriving the second TA can be further studied. |
|  | 2TAs for UL M-DCI based MTRP operation, inter-cell | Agreement Support two TA enhancement for both intra-cell and inter-cell multi-DCI multi-TRP scenarios in Rel-18. |
|  | CFRA triggered by PDCCH order for intra-cell M-DCI based MTRP | Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support CFRA triggered by PDCCH order for both intra-cell and inter-cell cases. |
|  | CFRA triggered by PDCCH order for inter-cell M-DCI based MTRP |  |
|  | PRACH configuration associated with additional PCI | Agreement For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, support PRACH configuration associated with additional configured PCIs different from the PCI of the serving cell. FFS: details |
|  | PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP | Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP at least for inter-cell Multi-DCI. |

CSI feedback enhancement

CSI enhancement for high/medium UE velocities

We provide our suggestions on basic and optional features for CSI enhancement for high/medium velocities based on the features agreed till RAN1#112bis-e. All the optional feature requires extra UE capability report. The ones highlighted in green are already captured in existed agreements.

| xx | Feature | Detail |
| --- | --- | --- |
|  | Basic: Basic feature of CSI enhancement for high/medium UE velocities. | N4=1 R=1 Rank<=2 Basic parameter combinations d=CSI-RS periodicity for P CSI-RS or d=m for AP CSI-RS l = slot (n+δ) where δ ≥ 0 X=1 and the CQI is associated with only one slot |
|  | Optional: support of N4>1 | Based on previous RAN1 study and evaluation results, most of the gain for CSI enhancement for high/medium UE velocities comes from CSI prediction. Further time domain compression (N4>1) provides limited gain compared with prediction, while compression causes extra UE complexity. Hence we suggest to make the support of N4>1 optional. |
|  | Optional: Support of R=2 | Following legacy principle. |
|  | Optional: Support of rank>2 | Following legacy principle. |
|  | Optional: Support of L=6 | Following legacy principle |
|  | Optional: Support of d=1 for AP CSI-RS if m>1 | If m>1, support of d=1 requires UE to perform extra interpolation to get the channel for each DD unit. Hence we support to make it optional due to its high complexity. |
|  | Optional: support of l = (n – nCSI,ref ) for CSI reference slot | Agreement On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, when UE-side prediction is assumed, support UE “predicting” channel/CSI after slot l where the location of slot l is configured (from multiple candidate values) by gNB via higher-layer signalling - Candidates of slot l location include the legacy CSI reference resource location (n – nCSI,ref ) and slot (n+δ) where δ ≥ 0 - FFS: Possible value(s) of δ and possible value(s) of WCSI Note: Per legacy behavior, the legacy CSI reference resource, i.e., (n – nCSI,ref ), is reused for locating the last CSI-RS occasion used for a CSI report For a UE that supports UE-side prediction, the support of l = (n – nCSI,ref ) is UE optional. |
|  | Optional: Support of X=1 and the CQI is associated with two slots. | Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), as well as the number of CQIs (=X) in one sub-band and one CSI reporting instance, support only the following: - Basic feature: X=1 and the CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Optional features: - X=1 and the CQI is associated with: - the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - the last slot of the CSI reporting window (slot l+WCSI–1) and the N4-thW2 matrix - X=2 and - The 1st CQI is associated with the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - The 2nd CQI is associated with the middle slot of the CSI reporting window (slot l+WCSI/2) and the (N4 /2)-thW2 matrix - FFS: Whether/how to include CQI overhead reduction for X=2 |
|  | Optional: Support of X=2 for CQI. | Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), as well as the number of CQIs (=X) in one sub-band and one CSI reporting instance, support only the following: - Basic feature: X=1 and the CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Optional features: - X=1 and the CQI is associated with: - the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - the last slot of the CSI reporting window (slot l+WCSI–1) and the N4-thW2 matrix - X=2 and - The 1st CQI is associated with the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - The 2nd CQI is associated with the middle slot of the CSI reporting window (slot l+WCSI/2) and the (N4 /2)-thW2 matrix - FFS: Whether/how to include CQI overhead reduction for X=2 |

CSI enhancement for coherent JT

We provide our suggestions on basic and optional features for CSI enhancement for CJT based on the features agreed till RAN1#112bis-e. All the optional feature requires extra UE capability report. The ones highlighted in green are already captured in existed agreements.

| xx | Feature | Detail |
| --- | --- | --- |
|  | Basic: Basic feature of CSI enhancement for CJT | Mode 2 R=1 Rank<=2 Basic parameter combinations 2NN1N2 <= 32 N_L = 1 Configuring N = N_TRP As a component, UE reports the maximum value for Ltot it supports. |
|  | Optional: support of Mode 1 | Based on RAN1 evaluation, the gain of mode 1 compared with mode 2 is marginal, and mode 1 causes extra calculation of FD offset. Hence we suggest to make the support of mode 1 optional. Mode 1 with φn∈0,1,2,…,N3-1 |
|  | Optional: Support of R=2 | Following legacy principle. |
|  | Optional: Support of rank>2 | Following legacy principle. |
|  | Optional: Support of L=6 for N_TRP =1 | Following legacy principle |
|  | Optional: Support of N_L>1 | As N_L >1 requires UE to calculate multiple SD hypothesis and thus multiple SVDs after SD projection, we suggest to make it optional due to its high complexity. |
|  | Optional: support of dynamic selection of N<N_TRP | Agreement On the Type-II codebook refinement for CJT mTRP, the selection of N CSI-RS resources is performed by UE and reported as a part of CSI report where N∈{1,..., NTRP} - N is the number of cooperating CSI-RS resources, while NTRP is the maximum number of cooperating CSI-RS resources configured by gNB via higher-layer signaling - The selection of N out of NTRP CSI-RS resources is reported via NTRP-bit bitmap in CSI part 1 - Note: The value of N is inferred from the selection - A restricted configuration (gNB-configured via higher-layer signaling) where N=NTRP is supported - NTRP-bit bitmap is not reported when the restriction is configured - FFS: Whether other RRC-configured TRP selection restriction including configuring the value of N is supported - This feature is UE optional Note: This agreement does not impact the decision on Ln being configured by gNB or selected by UE Note: per WID and previous agreement, the candidate values for NTRP of are 1, 2, 3, and 4. Note: only one transmission hypothesis is reported. UE is not mandated to calculate CSI for multiple transmission hypotheses. |
|  | Optional: The support of 2NN1N2 >32 | Agreement On the Type-II codebook refinement for CJT mTRP: - The maximum value of 2NN1N2 is 128 - The support of 2NN1N2 >32 is UE optional for UEs supporting Rel-18 CJT CSI enhancement - Note: Following the legacy specification on the maximum number of NZP CSI-RS ports per CSI-RS resource, the maximum value of 2N1N2 is 32. |
|  | Optional: The support of pv= {1/2, 1/2, 1/2, 1/2} | Agreement On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, support at least the following combinations of {pv,} from where the value of {pv,} is gNB-configured via higher-layer (RRC) signaling: - FFS by RAN1#112: whether other combinations can be supported FFS (by RAN1#112bis-e): Whether/how the supported combinations of {M} for Rel-17-based refinement are derived from the supported combinations of {pv ,} for Rel-16-based refinement pv for layers 1-4; ; Condition(s). {1/8, 1/8, 1/16, 1/16}; ¼; --. ½; --. {1/4, 1/4, 1/8, 1/8}; ¼ (*); --. ½ (*); --. {1/4, 1/4, 1/4, 1/4}; ¾ (*); --. {1/2, 1/2, 1/2, 1/2}; ½; - Only applicable when NTRP≤3 and NL=1 - Optional. |
|  | Optional: Support of fractional phi_n for mode 1. | Agreement On the Type-II codebook refinement for CJT mTRP, for mode-1, support the use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources, i.e. (example formulation) Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where: - Wf is commonly selected across N CSI-RS resources - φn is the layer-common FD basis selection offset for CSI-RS resource n relative to a layer-common reference CSI-RS resource n with φn=0 - Therefore, (N – 1) FD basis selection offset values φnn≠n are reported - Basic feature: φn∈0,1,2,…,N3-1 - Optional feature: φn∈0,14,12…,N3-14 - FFS: UCI design details, details on n |

TDCP

We provide our suggestions on basic and optional features for CSI enhancement for TDCP based on the features agreed till RAN1#112bis-e. All the optional feature requires extra UE capability report. The ones highlighted in green are already captured in existed agreements.

| xx | Feature | Detail |
| --- | --- | --- |
|  | Basic: Basic feature of CSI enhancement for TDCP | Y=1 with delay≤ Dbasic symbols, only wideband quantized normalized amplitude is reported |
|  | Optional: Support of delay>Dbasic | Agreement For aiding gNB determination of codebook switching and SRS periodicity with the Rel-18 TRS -based TDCP reporting, support reporting quantized wideband normalized amplitude/phase of the time-domain correlation profile with Y≥1 delay(s) as follows: - Basic feature: Y=1 with delay≤ Dbasic symbols, only wideband quantized normalized amplitude is reported - FFS: Candidate values for delay - Optional feature: Y=1 with delay>Dbasic symbols and Y≥1, wideband quantized normalized amplitude and phase for each delay are reported - For Y>1, the phase can be configured to be absent for all the Y delays - TBD: Whether the value of Y is configurable or following the delays from the configured TRS resource - TBD: Candidate value(s) for Y>1 - FFS: Value of Dbasic |
|  | Optional: Support of Y>=1 with phases | Agreement For aiding gNB determination of codebook switching and SRS periodicity with the Rel-18 TRS -based TDCP reporting, support reporting quantized wideband normalized amplitude/phase of the time-domain correlation profile with Y≥1 delay(s) as follows: - Basic feature: Y=1 with delay≤ Dbasic symbols, only wideband quantized normalized amplitude is reported - FFS: Candidate values for delay - Optional feature: Y=1 with delay>Dbasic symbols and Y≥1, wideband quantized normalized amplitude and phase for each delay are reported - For Y>1, the phase can be configured to be absent for all the Y delays - TBD: Whether the value of Y is configurable or following the delays from the configured TRS resource - TBD: Candidate value(s) for Y>1 - FFS: Value of Dbasic |

DMRS enhancement

| xx | Feature | Detail |
| --- | --- | --- |
|  | R18 DMRS eType1 for PDSCH/PUSCH | Agreement For enhanced FD-OCC length for DMRS of PDSCH/PUSCH for Rel.18 eType 1 DMRS, support - Opt.1-2: Length 4 FD-OCC is applied to 4 REs of DMRS within a PRB or across consecutive PRBs within an CDM group |
|  | R18 DMRS eType2 for PDSCH/PUSCH | Agreement For enhanced FD-OCC length for DMRS of PDSCH/PUSCH, support the following FD-OCC length: - For Rel.18 DMRS type 1, down select from the following in RAN1#110bis-e: - Opt.1-1: Length 6 FD-OCC is applied to 6 REs of DMRS within a PRB within an CDM group. - Opt.1-2: Length 4 FD-OCC is applied to 4 REs of DMRS within a PRB or across consecutive PRBs within an CDM group - For Rel.18 DMRS type 2: - Length 4 FD-OCC is applied to 4 REs of DMRS within a PRB within an CDM group |
|  | PDSCH scheduling without restriction for DMRS eType1 | Agreement For FD-OCC length 4 in Rel.18 eType 1 DMRS for PDSCH, support the following: - Introduce UE capability to report whether UE can be scheduled PDSCH without the scheduling restriction for FD-OCC length 4 in Rel.18 eType 1 DMRS. - If this capability is not supported by the UE, UE expects that gNB shall apply the scheduling restriction for PDSCH for FD-OCC length 4 in Rel.18 eType 1 DMRS. - The scheduling restriction above means satisfying all of the following at least for other than M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme. - 1) The number of consecutively scheduled PRBs for PDSCH is even. - 2) The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even. - 3) FFS: Restriction on scheduling of different UEs in case of MU-MIMO. - FFS: Scheduling restriction for M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme. - Note1: Up to UE how to implement DMRS channel estimation. - Note2: No further RAN1 specification enhancement is introduced to handle the orphan REs (e.g. if the total number of REs of DMRS in a CDM group is not multiples of 4, how to handle the remainder of REs) for UE that is scheduled PDSCH without the scheduling restriction. - Note 3: Other scheduling restrictions, if identified in future meetings, are not precluded. |

SRS enhancement

| xx | Feature | Detail |
| --- | --- | --- |
|  | Cyclic shift hopping for SRS with usage ‘antennaSwitching’ for CJT scenario | Agreement For SRS interference randomization, support: - Opt. 3: Both cyclic shift hopping and comb offset hopping. - At least the two features can be separately configured - FFS: Combined cyclic shift hopping and comb offset hopping for a UE - FFS: Separate or combined with SRS sequence group hopping / sequence hopping - FFS: Associated UE capability |
|  | Comb offset hopping for SRS with usage ‘antennaSwitching’ for CJT scenario | Agreement For SRS interference randomization, support: - Opt. 3: Both cyclic shift hopping and comb offset hopping. - At least the two features can be separately configured - FFS: Combined cyclic shift hopping and comb offset hopping for a UE - FFS: Separate or combined with SRS sequence group hopping / sequence hopping FFS: Associated UE capability |
|  | Time-domain hopping granularity for comb offset hopping | Agreement For a SRS resource configured with comb offset hopping, if the repetition factor R > 1, within a slot, the time-domain hopping behavior depends on the OFDM symbol index l' of each symbol or the first symbol across the R repetitions based on RRC configuration, and FFS configuration details. - UE can indicate whether it supports one or both the options. Details to be discussed in UE feature. |
|  | Comb offset hopping with group / sequence hopping | Agreement Whether SRS comb offset hopping can be combined with one of group / sequence hopping on a SRS resource depends on UE feature/capability design. - FFS: Whether SRS cyclic shift hopping can be combined with one of group / sequence hopping on a SRS resource depends on UE feature/capability design. - FFS: UE feature/capability design details. |
|  | The maximum number of simultaneous SRS resources for non-codebook | Agreement For SRS resource set(s) with usage ‘nonCodebook’ support 8 1-port SRS resources in one or multiple OFDM symbols. - Note: The maximum number of simultaneous SRS resources is determined via UE-capability signalling. |
|  | Time domain mapping of 8-port SRS resource with usage ‘codebook’ | Agreement For one single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH, when the SRS resource is configured with n ports (n <= 8) and m OFDM symbols (m >= 1), at least support the n ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof). - n can be 8 - m takes the legacy values, i.e., 1,2,4,8,10,12,14. Agreement For single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH or ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with 8 ports and m OFDM symbols (m > 1), support the case of 8 ports mapped onto the m OFDM symbols - Option 1: Different SRS ports are mapped onto different OFDM symbols (i.e., TDM) - FFS: m can be legacy values, i.e., 2,4,[8,10,12,14]. |
|  | Time domain mapping of 8-port SRS resource with usage ‘antennaSwitching’ | Agreement For an 8-port SRS resource in a SRS resource set ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with m OFDM symbols (m >= 1), at least support the 8 ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof). - m takes the legacy values, i.e., 1,2,4,8,10,12,14. Agreement For single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH or ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with 8 ports and m OFDM symbols (m > 1), support the case of 8 ports mapped onto the m OFDM symbols - Option 1: Different SRS ports are mapped onto different OFDM symbols (i.e., TDM) - FFS: m can be legacy values, i.e., 2,4,[8,10,12,14]. |

Support of STXMP

| xx | Feature | Detail |
| --- | --- | --- |
|  | S-DCI based SDM PUSCH | Agreement For single-DCI based STxMP PUSCH SDM scheme, support the layer combinations of {1+1, 1+2, 2+1 and 2+2} |
|  | Dynamic switching SDM - STRP | Agreement Support dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission |
|  | S-DCI based SFN PUSCH | Agreement Support SFN-based transmission scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18 |
|  | M-DCI based SDM PUSCH | Agreement Support STxMP PUSCH+PUSCH transmission in multi-DCI based system in Rel-18. - Two independent PUSCHs associated with different TRPs can be transmitted by a UE simultaneously in same active BWP. - The total number of layers of these two PUSCHs is up to 4. Agreement Multi-DCI based STxMP PUSCH+PUSCH transmission at least supports the following PUSCH combinations: - DG-PUSCH + DG-PUSCH - CG-PUSCH + DG-PUSCH |
|  | S-DCI based SFN PUCCH | Agreement Support the SFN scheme for single-DCI based STxMP PUCCH transmission |

Support of UL 8TX transmission

| xx | Feature | Detail |
| --- | --- | --- |
| x-x | 8Tx PUSCH 1, full coherent {N1, N2} = {2, 2}, {4, 1} 2, partial coherent, Ng=2 3, partial coherent, Ng=4 4, non-coherent | Agreement 8TX PUSCH is supported in Rel-18 Agreement For 8TX PUSCH, at least support - Ng=1, 2, 4 Note: The above does not restrict the Ng for the non-coherent case |
|  | Codebook-based PUSCH MIMO transmission, max layers | Agreement Support up to X layers for codebook and non-codebook UL transmission for 8TX UE where X=4, 8 is determined based on separate UE capability - For uplink transmission with rank<=4, single CW is supported - For uplink transmission with rank>4, whether single or dual CW is used will be decided in RAN1 meeting #110b-e |
|  | Non-codebook based 8Tx PUSCH, max layers | Agreement Support up to X layers for codebook and non-codebook UL transmission for 8TX UE where X=4, 8 is determined based on separate UE capability - For uplink transmission with rank<=4, single CW is supported - For uplink transmission with rank>4, whether single or dual CW is used will be decided in RAN1 meeting #110b-e |
|  | Full power modes | Agreement Framework for full power PUSCH transmission by an 8TX UE - To support full power transmission with Mode0, Rel-16 Mode0 (fullPower ) is re-used. - FFS if any change is required in the specifications. - Working Assumption To support full power transmission with Mode1, Rel-16 Mode1 (fullPowerMode1) is re-used. - FFS if more than one of the 8TX full coherent precoders is used per rank. - Working Assumption To support full power transmission with Mode2, Rel-16 Mode2 (fullPowerMode2) is re-used. - FFS definition of precoder groups (G0, G1, …) - FFS enhancements for SRS configuration |

Conclusions

In this contribution, initial UE features for Rel-18 MIMO sub-items are provided in section 2 based on agreements up to RAN1#112b-e.