3GPP TSG RAN WG1 #113			R1-2304556

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.3.2

Source: 	Spreadtrum Communications

Title: 		Discussion on subband non-overlapping full duplex

Document for:	Discussion and decision

Introduction

In RAN#94e, SI on evolution of NR duplex operation was approved [1]. As described in the SID, study on subband non-overlapping full duplex will be conducted in Rel-18, which is a key scheme of duplex enhancement in Rel-18 to provide uplink latency reduction, coverage enhancement and spectrum efficiency improvement.

In this contribution, we discuss some technical issues related to subband non-overlapping full duplex (SBFD) based on the agreements and conclusions achieved in RAN1#111 [2]. Since DL subband in UL symbols is as 2nd priority, only UL/DL subbands in DL symbols or flexible symbols is targeted in this contribution.

Discussion

Subband non-overlapping duplex schemes

The UL/DL resource partition is the fundamental to realize the subband non-overlapping full duplex at the gNB side, such as time/frequency domain location etc.

Subband time location and indication

For SBFD operation Alt 4, UE knows subband time domain location. During the RAN1#110b meeting, the explicit configuration of SBFD subband was agreed as the baseline. Furthermore, the time domain location is periodic.

Agreement (RAN1#110b) For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline.

Regarding the informing method of time locations, new signalling is a proper choice. Because the current three level of time domain signalling have its own position and working mechanism in determination UL or DL directions. It is hard to reinterpret into SBFD time domain information. Considering backward compatible, new additional signalling for SBFD time domain is better.

For SBFD subband time locations within a period, the period of tdd-UL-DL-ConfigurationCommon can be used directly. If there are two periods configured by tdd-UL-DL-ConfigurationCommon, those two periods also can be adopted for SBFD time domain signalling.

Furthermore, the relationship/override rules with the current three time domain signalling should be decided. Since we had agreed that semi-static configuration of subband time location as baseline, this new semi-static signalling can indicate the position of SBFD operation with UL subband can be in DL and flexible symbols configured by tdd-UL-DL-ConfigurationCommon and/or tdd-UL-DL-ConfigurationDedicated if configured. One issue is the order among those three signalling. It is common understanding that tdd-UL-DL-ConfigurationCommon is the first configuration for slot format. So SBFD time location can be before or after tdd-UL-DL-ConfigurationDedicated if any. The following figure gives two examples for those two cases. Figure 1A is cell-common SBFD time location. And Figure 1B is a type of UE-specific subband location indication.

| A:SBFD time location indicated before UE-dedicated signalling, by cell-common signalling | B: SBFD time location indicated by UE-dedicated signalling |
| --- | --- |

Figure 1: Cell-common vs. UE-specific subband time location indication

Table 1 gives some comparison for cell-common and UE-specific subband location indications. It is clear that both types of signalling have its advantages. Both of them can be study further, especially overall considerations together with collision handling is necessary.

Table 1: Cell-common vs. UE-specific subband location indication

|  | Cell-common SBFD time location indication | UE-specific subband location indication |
| --- | --- | --- |
| Configuration signalling | by SIB | By UE-specific signalling |
| Restrictions | SBFD time location indication can be within D and F configured by tdd-UL-DL-ConfigurationCommon | SBFD time location indication can be within D and F configured by tdd-UL-DL-ConfigurationDedicated |
| Override by the subsequent signalling | tdd-UL-DL-ConfigurationDedicated may further configured the F symbol into UL. SBFD symbols need modifications. | SBFD symbols can be based on tdd-UL-DL-ConfigurationDedicated, no modification. |
| Granularity of time location | DL and Flexible region are continuous, it is possible to have continuous SBFD time location. | Some Flexible symbols can be configured into UL, it may lead to discontinuous SBFD time location. |

- The slot configuration period of tdd-UL-DL-ConfigurationCommon can be used as the period of SBFD time domain location, including slot configuration period in pattern1 and pattern2 (if any).

- Both of Cell-common and UE-specific subband time location indication can be study further.

Granularity of SBFD time domain

Before further giving the detailed time domain signalling, it needs to identify the granularity of SBFD time domain, continuous slots, discontinuous slots, or symbols. At least slot level time location can be supported. According to continuous or discontinuous slots in a period, they can both in the scope. Since there may be some pure DL symbols requirement to guarantee the DL throughput.

For the configuration granularity of SBFD time domain, there is no clear benefit to give symbol level SBFD. Symbol level SBFD operation can be with lower priority, especially considering the DL-UL switching and UL-DL switching. In addition, symbol level SBFD would be more signalling overhead.

The following study point and observation were agreed during the previous meetings. We think it can be divided into two parts, the first is the configuration granularity of SBFD time domain, the second is final/actual SBFD symbols according to the rules, at least legacy TDD DL/UL configuration. Other aspects such as SSB, scheduling and dynamic SFI may also impact the actual SBFD symbol region, but they should be separately discussed due to the large complexity.

Agreement (RAN1#112) Study whether or not a slot can consist of both SBFD and non-SBFD symbols including - Benefits - Use cases - Scheduling flexibility - Implementation complexity - Compatibility with legacy TDD DL/UL configuration Conclusion(RAN1#112b-e) The following RAN1 observation is made: One motivation for allowing that a slot can consist of both SBFD and non-SBFD symbols is for compatibility with symbol-level TDD UL/DL configuration. Frequent switching between SBFD and non-SBFD symbols may increase the implementation complexity and interruptions of transmissions/receptions during transition. - Further study whether limitation(s) on the maximum number of switching points between SBFD and non-SBFD symbols within a slot, a TDD UL/DL pattern period, and/or semi-static SBFD configuration period (if different from TDD UL/DL pattern period) are needed - Further study scenarios a guard period between SBFD and non-SBFD symbols is required/not required and the length of the guard period if required Note: Whether or not a physical channel/signal occasion is mapped to both SBFD and non-SBFD symbols within a slot is a separate discussion.

For the final/actual SBFD symbols in a slot, it is another story due to more impact factors.

- From the observation it is quite clear that due to TDD UL/DL configuration, a slot can consist of both SBFD and non-SBFD symbols. First of all, DL symbol and flexible symbol configured by tdd-UL-DL-ConfigurationCommon can be used as SBFD operation, but UL symbol cannot. When the slot is self-contained structure, DL, flexible and UL symbols, it can be applied for SBFD operation only within the DL and Flexible symbols, but not UL symbols, if gNB and UE can support SBFD and TDD switching. However, the frequent switching between SBFD and non-SBFD symbols should be avoid. One reasonable restriction is the maximum number of switching points between SBFD and non-SBFD symbols within a slot is up to 1, since the TDD UL/DL configuration can only lead to one switching point in a slot by tdd-UL-DL-ConfigurationDedicated, such as DL-F-UL, or F-UL, or DL-F patterns. Among these three patterns, only DL-F-UL and F-UL would cause a switching point between SBFD and non-SBFD symbols, because UL symbol cannot be SBFD symbol in Rel-18.

- For other aspects, such as semi-static/dynamic scheduling, dynamic SFI, they can be discussed in collision handling part.

- At least slot level can be study for SBFD time location, including continuous and discontinuous slots in the period.

- Symbol level configuration granularity of SBFD time domain location is with lower priority.

- Maximum number of switching points between SBFD and non-SBFD symbols within a slot is up to 1 due to TDD UL/DL configuration.

  - Since the TDD UL/DL configuration can only lead to one switching point in a slot by tdd-UL-DL-ConfigurationDedicated.

Subband frequency location indication

Regarding the informing method of frequency locations of subbands for SBFD operation, semi-static configuration of subband frequency location of UL/DL subband is with relative to CRB grid, thus we think the CRB0 and length of continuous CRB number for UL/DL subband is enough.

Agreement (RAN1#111) For the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid.

- Regarding the informing method of frequency locations of subbands for SBFD operation, it can be configured relative to CRB0 and length of continuous CRB number.

It was agree that explicit or implict indication of frequency location of DL subband(s).

Agreement(RAN1#112b-e) At least for semi-static SBFD, the following two options are viable solutions for frequency location configuration of DL subband(s) and guardband(s) if any. - Option 1: Frequency locations of DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s). - Option 2: The number of RBs for guardband(s), if any, is explicitly configured. DL subband(s) are implicitly derived as RBs which are not within UL subband or guardband(s).

We prefer explicit indication of frequency location of UL subband and DL subbands, and guard bands locations are not included in any of DL and UL subbands.

- First, it is simple to avoid any arguments of whether guard bands are exist and whether those can be used for Tx and Rx.

- Second, the location and size of UL subband and DL subband(s) are purely frequency locations can be applied for UL or DL. It is clearly for both of discussions and definitions.

- For the guardband(s), they can be unawareness. UE knows exactly the resources for UL and DL, all the collisions can be resolved by these information, such as whether UL Tx is within/outside UL subband, or DL Rx is within/outside DL subband. Guardbands do not have special handling.

For the frequency location of DL subband(s) is explicitly indicated or implicitly determined, we support cell-common explicit indication of DL subbands.

A DL subband is a set of contiguous RBs. So at most there would be two DL subbands. Every DL subbands are resources excludes guard band directly by configuration.

- For frequency location configuration of DL subband(s) and guardband(s) if any,

  - Option 1 can be simple to avoid any arguments of whether guard bands are exist and whether those can be used for Tx and Rx.

  - The location and size of UL subband and DL subband(s) are purely frequency locations can be applied for UL or DL. It is clearly for both of discussions and definitions.

  - For the guardband(s), they can be unawareness.

Thus, the following proposal for UL/DL subband and guard bands.

- For frequency location configuration of DL subband(s) and guardband(s) if any use Option 1, frequency locations of DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

- The cell-common explicit indication of frequency location of UL/DL subband(s) are the baseline for study.

SBFD aware UE behaviors

DL/flexible symbol by TDD-UL-DL-ConfigCommon

According to the following agreement, the options of UL transmission and DL reception in UL and DL subband in DL symbol and Flexible symbol were agreed.

Agreement (RAN1#112) For dynamic SBFD, - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options: - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following options: - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed and UL transmissions outside semi-statically configured UL subband are not allowed - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed - UL transmissions outside the semi-statically configured UL subbands are not allowed - Option 3: DL receptions outside semi-statically configured DL subband(s) are allowed - UL transmissions outside the semi-statically configured UL subbands are allowed Dynamic SBFD should be compared with dynamic TDD and/or semi-static SBFD in terms of performance, implementation complexity, switching latency. For each option, additional conditions may apply to determine whether the option is applicable.

Although option 2 for DL and option 2/3 for flexible may provide extra scheduling flexibility, it acquires more significant efforts to define the application conditions when RBs outside DL subbands can be allowed for DL and RBs outside UL subband can be allowed for UL, necessary evolutions to compare the performance gain is required, etc.

For RBs outside DL subbands can be allowed for DL and RBs outside UL subband can be allowed for UL, it seems allow the DCI schedule UL/DL to change the whole symbol into UL-only or DL-only symbol, or singling to disable SBFD operation. However, there can be several problems to allow these overriding.

- If the conversion to DL-only symbol can be caused by dynamic scheduling of PDSCH/CSI-RS, the target UE is aware of this override command and disable SBFD operation in the corresponds regions, however the other UEs do not known this conversion, they still try to transmit UL which is configured by higher layer signalling in the UL subband. This unwanted UL will be serious interference for the UE which receive the DL. Similarly, the RBs outside UL subband used as UL would have the same interference issue for gNB. Clearly, the dynamic scheduling based DL-only or UL-only conversion cannot work for SBFD operation in flexible symbols.

- Second, such the CG-PUSCH and PDSCH in slot 2, UE can drop PDSCH reception due to it is collide with UL regions. However, if PDSCH scheduled by a DCI can be valid DL reception, whether CG-PUSCH should be dropped, even if it is within a legal UL region. There will be more ambiguous collisions need to handle, and we find it is hard to give a unified solution, since sometimes it gets the DL or UL direction based on the configuration, but sometime it allows override. It gives too much flexibility for the link directions, and leads to some unwanted and unnecessary complex collision handling. D subbands in flexible symbol only means it can be used for DL reception, but only necessarily have when SBFD operation is enabled in this symbol.

So we do not support Option 2 for DL, and Option 2/3 for flexible.

Figure 2: Example for DL receptions within UL subband

- DCI schedule UL/DL to support DL in RBs outside DL subbands and UL in RBs outside UL subband cannot work.

- Support Option 1 for SBFD operation in a symbol configured as DL/flexible in TDD-UL-DL-ConfigCommon.

SSB symbols

The following agreement in RAN1#112b-e meeting encouraged to consider SBFD operation in SSB symbols, we support Option 2.

Agreement(RAN1#112b-e) Study the following options for SBFD operation in SSB symbols. - Option 1: UL subband cannot be configured in an SSB symbol - FFS handling of misaligned periodicities between SSB and semi-static SBFD subband time location configuration - Option 2: An UL subband can be configured in an SSB symbol - FFS whether/when and/or under which conditions an SBFD-aware UE transmits in the UL subband or may receive SSB in the symbol.

First, Option 1 has trouble to avoid the semi-static SBFD subband time location configuration overlapping with SSB periodicities. Considering they are separately configured, SSB periodicity typically is larger than SBFD period configuration, it can be beneficial to allow SBFD symbol can be configured in an SSB symbol.

Second, according to Option 2, UE behaviours in the case of UL subband is overlapping with SSB symbol should be further clarified. We think SBFD-aware UE cannot transmit in the SSB symbol but can only receive within the entire DL BWP in the SSB symbol.

- Because gNB do not aware of the SSB reception of the RRC Connected UE, it cannot guarantee the UL can be transmitted from the UE side. So UL transmission has a high possibility cannot be sent by UE, gNB is more likely to avoid UL scheduling in SSB symbols.

- Next, SSB has the highest priority in legacy, dynamic scheduling or semi-static or period scheduling, even with SBFD configuration overlapping with SSB symbols, according to current UE behaviour, UL transmissions are always dropped.

- For SBFD operation in SSB symbols, an UL subband can be configured in an SSB symbol.

- When UL subband is overlapping with SSB symbol, SBFD-aware UE cannot transmit in the SSB symbol but can only receive within the entire DL BWP in the SSB symbol.

Transmission/Reception enhancements

Impacts of non-contiguous DL subbands

Impacts of non-contiguous DL subbands for CSI-RS and PDSCH, it would be good to have unified solution to resolve this issue.

CSI-RS in frequency domain

The agreements of CSI-RS resource for non-contiguous DL subbands in previous RAN1 meetings provided Option 1, Option 2-1 and 2-2 three options, and gave some observations.

Agreement: (RAN1#112) Study the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs considering the following options: - Option 1: Two contiguous CSI-RS resources that are linked - Option 2: One CSI-RS resource - Option 2-1: Non-contiguous CSI-RS resource allocation - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s) Conclusion(RAN1#112b-e) For the options agreed to study in RAN1#112 for frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, the following observations are agreed. - For all the options, there is no impact on CSI-RS sequence generation. - Option 1 requires additional signalling to link two CSI-RS resources in two DL subbands. - Option 2-1 requires new RRC structure to configure non-contiguous RBs for one CSI-RS resource, which may require additional signalling overhead. - Option 2-2 can reuse the existing signalling design for CSI-RS resource configuration. Option 2-2 can be used to resolve the potential unaligned boundaries between CSI-RS resource configuration and SBFD subbands - Further discussion is required on the UE complexity due to: - UE capability of maximum number of configured CSI-RS resources - Processing non-contiguous CSI-RS

All the options can solve the problem due to non-contiguous DL subbands, especially Option 2-2 is more suitable. Because it can work for the transmissions across SBFD and non-SBFD symbols, a same set of CSI-RS resources can be used for SBFD symbols and non-SBFD symbols. It does not impact the maximum number of configured CSI-RS resources. And regarding to processing non-contiguous CSI-RS, we think complexity increasing is affordable for UE, since the CSI sub-band granularity in frequency domain requires finer sub-band granularity measurement already, which can be contiguous and non-contiguous. Thus from this point of view, Option 2-2 is preferred.

- For the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband(s) is supported.

Resource allocation granularity

For RBG for PDSCH/PUSCH FDRA, CSI-RS granularity, CSI reporting subband and PRG of PDSCH, they are quite similar. A unified solution is preferred.

RBG and PRG

According to the issue of unaligned boundaries of DL/UL subband and configuration granularity in the frequency, the agreement below were agreed during the last meeting.

Agreement(RAN1#112b-e) For SBFD-aware UEs, Option 1 with update is agreed for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands for better resource utilization. For an RBG that overlaps the subband boundary, - Option 1 (with update): - The Part of the DL RBG inside the DL subband can be used - The Part of the UL RBG inside the UL subband can be used

Agreement (RAN1#112) For SBFD-aware UEs, study at least the following issues for PDSCH: - PRG(s) with size of 2 and 4 that overlaps with subband boundary - Wideband precoder in case of non-contiguous DL subbands Agreement(RAN1#112b-e) If PRG is determined as wideband, study the following two options: - Option 1: non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated - FFS: Precoding assumption within and across the two DL subbands - Option 2: non-contiguous frequency resources across two DL subbands cannot be allocated The study should include the impact on UE complexity

RBG always larger than or equal size to PRG size applied for UE if both are configured. Considering the case that PRG has same size as RBG, when part of the DL RBG inside the DL subband is used for PDSCH, it also needs to define UE behaviours for those part of DL PRG inside the DL subband. It is not complete if only partial RBG is allowed but partial PRG is not allowed.  Take RBG and PRG size are both 4 for PDSCH FDRA for example, as shown in Figure 7, RBG 3 and 6 have partial part of DL RBG inside the DL subband, PRB 26/25/24 can be used for PDSCH. But regarding the UE assumption for physical resource block bundling, if partial PRG is not allowed, it would impact the agreed partial RBG applied for PDSCH. So we suggest to use the part of the DL PRG inside the DL subband, too.

Figure 4: SBFD impacts on RBG and PRG

Wideband PRG has more strict requirements, including contiguous PRBs and the size of scheduled PRBs is larger than half of the BWP size.  For larger than half of the BWP size, it is very hard to be satisfied if only contiguous PRBs are allowed for a PDSCH. So Option 2 is basically excluded from the typical deployment D-U-D case. Option 1 is supported. With regarding to precoding assumption within and across the two DL subbands, it is clear that same precoding is assumed within each of two DL subband. However, according to precoding assumption across two DL subbands, we still prefer to keep the legacy UE behaviour, that same precoding is applied to the allocated resources, no matter they are across DL subbands or not.

- For a PRG that overlaps with subband boundary, partial DL PRG inside the DL subband can be used.

- If PRG is determined as wideband, non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated (Option 1).

  - Same precoding is applied to all the allocated resources across two DL subbands.

Transmissions across SBFD and non-SBFD symbols in different slots

The following agreement provides good further discussion directions for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols.

Agreement (RAN1#112) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) - Study the following options for SBFD-aware UEs: - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols - UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following: - PDSCH/PUSCH/PUCCH repetitions - SPS PDSCH/CG PUSCH - TBoMS - Multi-PUSCH/PDSCH scheduled by a single DCI - Periodic/semi-persistent SRS/CSI-RS/PUCCH - PDCCH Agreement: (RAN1#112) Study at least the followings for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots: - Whether/how to have separate resources - Whether/how to have separate FH parameters - Whether/how to have separate UL power control parameters - Whether/how to have separate beam/spatial relation Conclusion(RAN1#112b-e) For the two options agreed in RAN1#112 for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), the following observations are agreed. - Option 1 can be achieved by gNB configuration or scheduling to ensure that all transmission/reception occasions are confined to either SBFD symbols or non-SBFD symbols. Alternatively, Option 1 can be achieved by additional indication or rules to determine the transmission/reception occasions are valid within one symbol type and are invalid within the other symbol type. - The frequency resources, power control and beam/spatial relation for all the transmission/reception occasions can be the same for Option 1 but may be different for Option 2. If different, it may require additional specification efforts. - Option 1 may or may not increase the transmission/reception latency if the transmission/reception in the other symbol type is postponed and may degrade the performance if the transmission/reception in the other symbol type is dropped. Option 2 may or may not reduce the transmission/reception latency and improve coverage.

For scheduled/configured UL transmissions and scheduled/configured DL receptions across SBFD symbols and non-SBFD symbols, the table below gives some analysis.

Table 2: Summary of UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols

| Cases |  | Across SBFD symbols and non-SBFD symbols in different slots | If yes, Option 1 or Option 2? |
| --- | --- | --- | --- |
| PDCCH |  | Yes | Option 2 No extra change, handled by proper configuration of CORESET and monitoring occasion to never overlap with UL subband in SBFD symbols |
| Scheduled | PDSCH/PUSCH/PUCCH with repetition | Yes | Option 2 The main objective for this AI is to improve the UL coverage and latency |
|  | TBoMS PUSCH | Yes | Option 2 |
|  | Multi-slot PUSCH/PDSCH | Yes | Option 2 |
| Configured | SPS PDSCH/ Type1&Type 2 CG-PUSCH/PUCCH without repetition | Yes | Option 2 |
|  | SPS PDSCH/ Type1&Type 2 CG-PUSCH/PUCCH with repetition | Yes | Option 2 |
|  | Type 2 CG-PUSCH TBoMS | Yes | Option 2 |
| Scheduled/configured CSI-RS/SRS |  | Yes | Option 2 |

Resource allocation in frequency domain including frequency hopping

For all the other scheduled or configured UL transmission and DL reception except PDCCH and scheduled PUSCH/PDSCH/PUCCH without repetition, they share the same situation that different available resources are in SBFD symbols and non-SBFD symbols. The following agreement was agreed.

Agreement(RAN1#112b-e) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), if the transmissions/receptions can be in SBFD symbols and non-SBFD symbols with different available resources, study at least the following frequency resource allocation options for PDSCH, CSI-RS, PUSCH, PUCCH, SRS for SBFD-aware UE: - Option 1: Separate FDRA determination for SBFD slots and non-SBFD slots. - Option 1-1: Separate FDRA configurations/indications for SBFD slots and non-SBFD slots - Option 1-2: Separate frequency resources determined for SBFD slots and non-SBFD slots based on single FDRA configuration/indication - Option 1-3: single FDRA configuration/indication and RB offset(s) - Option 2: Perform rate matching or puncturing on the RBs outside DL/UL subbands for DL/UL channels/signals. - Option 3: A DL/UL channel/signal overlapping with RBs outside DL/UL subbands in a SBFD slot is dropped or postponed. Note: Different options can be studied for different signals/channels.

Regarding to Option 3, there it is not a good solution, considering the objective to improve UL coverage, and it cannot harm the DL coverage. Due to SBFD can provide more continuous/discontinuous UL time resources, it reduce the gain if a DL/UL channel/signal overlapping with RBs outside DL/UL subbands in a SBFD slot, and may decrease the DL performance.

For Option 2, it is a method to use SBFD and non-SBFD slots, but it cannot be applied for PUSCH, PUCCH, CSI-RS and SRS. Since reference signalling do not support any type of rate matching and puncturing. In addition, PUCCH also has big trouble to apply rate matching or puncturing, due to significant impacts to the UCI mapping mechanism in legacy. The only possible one is rate matching on the RBs outside DL subbands for PDSCH, we are open to further study it. But puncturing on PDSCH is not clear, we do not understand how to do puncturing on PDSCH.

We support Option 1, especially Option 1-1 and 1-2, they can fully satisfy the target objectives of SBFD item, and provide enough scheduling flexibility for both of DL and UL, can be easy applied to configured and scheduled channels or signals. However, Option 1-3 does not suitable for PDSCH/CSI-RS, since RB offset has some restriction on the frequency location, not very well for the non-contiguous DL subband.

- For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), among separate FDRA determination methods:

  - Option 3 would reduce the SBFD performance gain if a DL/UL channel/signal overlapping with RBs outside DL/UL subbands in a SBFD slot.

  - Option 2 cannot be applied for PUSCH, PUCCH, CSI-RS and SRS. Since reference signalling do not support any type of rate matching and puncturing. In addition, PUCCH/PUSCH also has big trouble to apply rate matching or puncturing, due to significant impacts to the UCI mapping mechanism in legacy. The only possible one is rate matching on the RBs outside DL subbands for PDSCH.

- For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), if the transmissions/receptions can be in SBFD symbols and non-SBFD symbols with different available resources, further consider the following two options of separate FDRA determination for SBFD slots and non-SBFD slots for PDSCH, CSI-RS, PUSCH, PUCCH, SRS for SBFD-aware UE

  - Option 1-1: Separate FDRA configurations/indications for SBFD slots and non-SBFD slots

  - Option 1-2: Separate frequency resources determined for SBFD slots and non-SBFD slots based on single FDRA configuration/indication

Resource allocation in time domain

Different from frequency domain, we are not sure why time domain resource need to improve, maybe because SBFD region and non-SBFD regions contain different symbols. However, the time granularity of SBFD has not been decided yet, if SBFD and non-SBFD are both slot based, there would be no problem. It is unnecessary to provide solution for the potential case. So the resource allocation in time domain can be postponed until there are sufficient driven.

- The resource allocation in time domain for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols can be postponed until there are sufficient driving.

Power domain

Similarly as resource allocation in time domain, power domain also need to postpone the discussion. Because the power domain enhancement is from the benefits of the CLI handling. However, due to it is still in process without sufficient convince, power domain enhancement can start later.

- The power domain for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols can be postponed until there are sufficient driving from the CLI.

Spatial domain

Same as power domain, spatial domain improvement comes from the CLI handling, such as the preferred or non-preferred beam. We can hold it until more information is provided, e.g. the difference requirements of spatial domain in SBFD and non-SBFD regions.

- The spatial domain for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols can be postponed until there are sufficient driving from the CLI.

PDCCH

When PDCCH monitoring occasions crossing SBFD and non-SBFD symbols in different slots, it can be guaranteed by PDCCH monitoring occasion and CORESET configuration to never overlap with UL subband in SBFD symbols. Since if the frequency domain location of a CORESET in a SBFD symbol can also work for a non-SBFD symbol, CORESET location does not need to consider how to take full advantage of DL frequency resources, since PDSCH can use the other leftover parts. The location of CORESET only has to ensure the PDCCH capacity, and this part can be satisfied by enough RB number in CORESET.  SBFD is to improve UL delay and coverage, DL resource for is not bottleneck, so a clean CORESET and search space configuration has no problem. In addition, CORESET bitmap configuration has already sufficient flexibility to handle UL subband in frequency domain.

Agreement(RAN1#112b-e) For the case that: - The monitoring periodicity of a search space is such that different monitoring occasions in different slots occur in SBFD and non-SBFD symbols, respectively, and, - The associated CORESET overlaps the boundary of a DL subband in SBFD symbols Consider whether/how the above could be supported considering both existing tools in specifications on CORESET and search space configuration as well as at least the following options for potential enhancement for SBFD-aware UE: - Option 1: Separate valid resources for the CORESET in SBFD symbols and in non-SBFD symbols. - Option 2: Rate matching or puncturing on the REG(s) of a PDCCH outside DL subband(s). - Option 3: UE does not monitor a PDCCH candidate if it is mapped to one or more REs that overlap with REs outside DL subband(s). - Option 4: Drop search space(s) when the associated CORESET overlaps with RBs outside DL subband(s) - Option 5: Separate search spaces associated with a CORESET in SBFD and non-SBFD symbols Note: Whether these enhancements are applicable to only USS or also CSS

Option 1 would increase the CORESET number per BWP, now it is up to 3 in Rel-15, and 5 for m-TRP. When separate valid resources for the CORESET in SBFD symbols and in non-SBFD symbols is applied, it will increase the complexity of UE implementation, since it has larger CORESET requirements, which may limit UE capability of maximum number of CORESET. Even if the CORESET index is not changed, it is not same between SBFD and non-SBFD symbol. Thus the number of CORESET is fundamentally increased.

Option 2: Rate matching or puncturing on the REG(s) is an unpractical solution.  RM or puncturing never ever supported for PDCCH/REG, due to a huge change to REG-CCE mapping. Frankly speaking, Option 2 is a castle in the air.

For Option 5, we really has trouble to understanding it. If a CORESET in “Separate search spaces associated with a CORESET in SBFD and non-SBFD symbols” means same CORESET, it does not help to solve the non-contiguous resources or misaligned granularity in SBFD symbol. If a CORESET means any CORESET, not always same CORESET, it is same as Option 1.

So, in summary, PDCCH monitoring and CCE-REG mapping is the most complex part for UE implementation, existing tools in specifications on CORESET and search space configuration already have sufficient flexibility for PDCCH, no enhancement is required.

- When PDCCH monitoring occasions crossing SBFD and non-SBFD symbols in different slots,

  - Option 1 would increase the CORESET number per BWP, which will limit UE capability of maximum number of CORESET.

  - Option 2: Rate matching or puncturing on the REG(s) is an unpractical solution, due to a huge change to REG-CCE mapping

- When PDCCH monitoring occasions crossing SBFD and non-SBFD symbols in different slots, it can be guaranteed by PDCCH monitoring occasion and CORESET configuration to never overlap with UL subband in SBFD symbols.

Transmissions across SBFD and non-SBFD symbols in one slot

For the point of “If the PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD and non-SBFD in the same slot if configured”, as shown in the following figure, PDSCH in slot 1 and PUSCH in slot 3 both mapped to SBFD and non-SBFD regions. We do not think it is a reasonable case.

Agreement(RAN1#112b-e) Study whether the transmission/reception occasion of a physical channel/signal can be mapped to SBFD and non-SBFD symbols within a slot for a UE, and whether a UE can transmit/receive in the occasion mapped to SBFD symbols and non-SBFD symbols including: - Use-case(s) including the locations and number of switching points of the SBFD and non-SBFD symbols in the slot. - Potential benefits if any - Phase continuity - Potential interruption of transmissions/receptions during transition - Required guard time if any - Potential impact on performance - Impact on link adaptation, channel estimation, and other procedures - UL transmission timing if any - Implementation complexity - Applicability for SBFD aware UE and non-SBFD aware UEs - NOTE: There are more than one scenario where a transmission overlaps SBFD and non-SBFD symbols and some may or may not face the aspects listed above - NOTE: This study doesn’t mean RAN1 agreement on a slot consisting of SBFD and non-SBFD symbols.

- First, timing advance for CLI handling in SBFD operation and flexible duplex is an important issue, which were discussed by a lot of companies in their contribution, and proposed to use different TA in SBFD and non-SBFD region. This solution would lead to two TA for one PUSCH in slot 3. It is too early to make decision before we have a clear picture for TA enhancement.

- Second, DL-UL switching is another issue would impact the allowance, e.g. the orange box in the slot 1 shows a possible DL-UL switching pattern for DL symbol change to UL subband. Frankly speaking, the DL-UL switching pattern cause trouble for PDSCH covering SBFD and non-SBFD in a slot.

- Third, some enhancements were agreed for SBFD regions, such as different frequency resource, time domain resource, power control, and spatial domain. It is hard to decide which frequency/time/power/spatial domain parameters or indications should be used for these specific channel.

In a summary, the PUCCH/PUSCH/PDSCH/PDCCH mapped to SBFD and non-SBFD in the same slot is a corner case, which have a lots of potential problems may impact the main progress of our SBFD discussion. We propose not to discuss this issue in the SI stage again.

Figure 5 Examples for PUSCH/PDSCH mapped to SBFD and non-SBFD in the same slot

- PUCCH/PUSCH/PDSCH/PDCCH cannot be mapped to SBFD and non-SBFD regions in the same slot.

Reports across SBFD and non-SBFD symbols

For reports across SBFD and non-SBFD symbols, the following agreements were achieved during last meeting.

Agreement(RAN1#112b-e) - For semi-static SBFD, for a CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS resources within DL subband(s) are valid for SBFD-aware UE. - For semi-static SBFD, for a CSI reporting subband which overlaps with SBFD subband boundaries, CSI report is derived based on CSI-RS resources excluding CSI-RS resources outside DL subband(s). Agreement(RAN1#112b-e) For SBFD-aware UEs, study the following options for CSI report associated with periodic/semi-persistent CSI-RS in case the periodicity is such that CSI-RS instances occur in both SBFD and non-SBFD symbols: - Option 1: two CSI-ReportConfigs, where one is associated with SBFD symbols and the other is associated with non-SBFD symbols - Option 1-1: One CSI-ReportConfig is associated with a CSI-RS restricted to SBFD symbols only and the second CSI-ReportConfig is associated with a second CSI-RS restricted to non-SBFD symbols only; - Option 1-2: Both CSI-ReportConfigs are associated with the same CSI-RS. The CSI report associated with one CSI-ReportConfig is derived based on CSI-RS instances in SBFD symbols only. The CSI report associated with the second CSI-ReportConfig is derived based on CSI-RS instances in non-SBFD symbols only. - Option 2: one CSI-ReportConfig associated with both SBFD symbols and non-SBFD symbols - Option 2-1: One CSI-ReportConfig is associated with two CSI-RSs which are restricted to SBFD symbols and non-SBFD symbols respectively. Separate CSI measurements are derived based on the first and second CSI-RSs respectively. - Option 2-2: One CSI-ReportConfig is associated with one CSI-RS. The CSI report is derived based on CSI-RS which can be in SBFD symbols or non-SBFD symbols in different time instances. - FFS impact on UE CSI processing and reporting timeline Note: Whether the CSI-RS resource can be used for SBFD and non-SBFD symbols may depend on, e.g., gNB implementation of same/different antenna configuration in both symbols. Option 1-1 can be supported according to existing specification by gNB configuration of appropriate periodicities to ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. But it may restrict the gNB configuration flexibility and enhancements can be considered by additional indication or rules to determine the CSI-RS is valid within one symbol type and is invalid in the other symbol type. Option 2-2 can be supported according to existing specification to configure measurement restriction so that UE would not average CSI measurements across SBFD and non-SBFD symbols.

In the current specification, each CSI reporting is associated with a DL BWP and contains the parameter(s) for one CSI reporting band, the following table summarizes main configuration parameters in the a CSI-ReportConfig, and try to give the potential issues for CSI reporting across SBFD and non-SBFD symbols.

Table 3: Main configuration parameters in the a CSI-ReportConfig

|  | Parameters in CSI reporting | Reasons |
| --- | --- | --- |
| Potential different impacts/configurations from SBFD and non-SBFD symbols | Resources for channel measurement and interference measurement | CMR and IMR across DL subbband in SBFD symbols, but it can be solved by one or two CSI-RS resources that are linked |
| Same across SBFD and non-SBFD symbols | frequency granularity for CQI and PMI (Wideband or subband in reportFreqConfiguration, and subbandSize) | Prefer to be same across SBFD and non-SBFD symbols. No clearly benefit to have different frequency granularity for CQI and PMI |
|  | time-domain behavior | Same One from 'aperiodic', 'semiPersistentOnPUCCH', 'semiPersistentOnPUSCH', or 'periodic' |
|  | Codebook configuration including codebook subset restriction | Same |
|  | Measurement restriction configurations, for Channel Measurements, and Interference Measurements | Same Configured or not configured |
|  | CSI-related quantities, including cqi-Table and cqi-BitsPerSubband | Same quantities, with same CQI table and CQI bits for subband |

Clearly, only resources for channel measurement and interference measurement have reasons to have different configurations across SBFD and non-SBFD symbols, it can be solved by one or two CSI-RS resources that are linked in the section 2.2.1. The other parameters do not have strong reasons or benefits to be different. So we think Option 2: same CSI reporting for SBFD symbols and non-SBFD symbols is enough for CSI reporting. Especially, Option 2-2 is preferred: One CSI-ReportConfig is associated with one CSI-RS. The CSI report is derived based on CSI-RS which can be in SBFD symbols or non-SBFD symbols in different time instances.

- For SBFD-aware UEs, for CSI report associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols), support Option 2: same CSI reporting for SBFD symbols and non-SBFD symbols

  - Option 2-2 is preferred: One CSI-ReportConfig is associated with one CSI-RS. The CSI report is derived based on CSI-RS which can be in SBFD symbols or non-SBFD symbols in different time instances.

UE collision handling

There was a proposal in the last meeting without close attention due to limited time.

Proposal 1-12 At least the following cases of time domain conflict of UL transmission and DL reception in the same SBFD symbol are identified. - Dynamic DL reception vs. semi-statically configured UL transmission - Dynamic UL reception vs. semi-statically configured DL transmission - Semi-statically configured DL reception vs. semi-statically configured UL transmission Note: PRACH transmissions and SSB receptions are not included.

According to collision between UL transmissions and DL receptions in the same SBFD symbol, the following cases can be discussed, where if the UL transmission is transmitted or DL reception is received using resources indicated by a DCI, then it belongs to “dynamic UL/DL transmission”; if the UL transmission is transmitted or DL reception using semi-statically configured resources or other repetitions/transmissions besides the first one of the dynamic UL transmission or DL reception, then it belongs to “semi-static UL transmission”.

- Collision between dynamic UL transmissions and dynamic DL receptions

- Collision between dynamic DL receptions and semi-statically UL transmissions

- Collision between dynamic UL transmissions and semi-statically DL receptions

- Collision between semi-statically UL transmissions and semi-statically DL receptions

For collision between dynamic UL transmissions and dynamic DL receptions, it can treated as an error case, as same as current specification stated. It is natural and easy for gNB to have non-overlapping dynamic UL transmissions and dynamic DL receptions.

For the 2nd, 3rd, and 4th collisions, the allowed transmission direction depends on the SBFD time/frequency location indication discussion, which also relative to UE assumption of link direction before collision handling. There are some alternatives can be considered:

Alt 1: UE can use either DL or UL subband in a SBFD symbol, depends on DL reception or UL transmission. Such as tdd-UL-DL-ConfigurationDedicated still provide whole symbol direction, as same as current specification. UE can use either DL or UL subband, depends on the DL reception or UL transmission as shown in the following figure B. Dynamic DL receptions and semi-statically UL transmissions, dynamic UL transmissions and semi-statically DL receptions and semi-statically UL transmissions and semi-statically DL receptions are all possible. This alternative is more complex when handling the collision, but with flexibilities comparing with Alt 2.

Alt 2: UE can only use one subband direction of a symbol through configuration signalling. Such as tdd-UL-DL-ConfigurationDedicated can provide further UE’s subband direction. It selects DL subband or UL subband of SBFD symbols. As shown in the following figure A, tdd-UL-DL-ConfigurationDedicated for one UE1 indicates UL, so only UL subband can be used, while slot 1&2 are DL, only DL subbands can applied. For UE2, slot 1 and 2 are UL, UL transmission in UL subband is valid, and DL in DL subbands are not allowed. In this alternative, semi-statically UL/DL is cancelled if it is against the transmission direction of subband, according to the current collision handling. And dynamic DL/UL should be aligned with transmission direction of subband. Obviously, this alternative can reuse the current collision handling as much as possible, it is easier for all the collision decision, but with less flexibility due to only one of DL or UL subband has been predefined, the opposite direction cannot be used.

| A | B |
| --- | --- |

Figure 7: UE assumption of link direction before collision handling

- For collision between dynamic UL transmissions and dynamic DL receptions, it can treated as an error case

- Before to study collision between UL transmissions and DL receptions, it needs to align the understanding of UE assumption of link direction:

  - Alt 1: UE can use either DL or UL subband in a SBFD symbol, depends on DL reception or UL transmission.

  - Alt 2: UE can only use one subband direction of a symbol through configuration signalling.

Regarding collision between transmissions/receptions with transmission direction of subband, it has some connection with SBFD aware UE behaviours especially in flexible symbol, and frequency domain resource allocation for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols. Before further step, we can process the two issues first.

CLI handling (gNB/UE)

During Rel-15, the CLI interference mitigation schemes for duplexing flexibility were initially discussed. In Rel-16, UE-to-UE CLI measurement on SRS-RSRP and CLI-RSSI was introduced [4], and for gNB-to-gNB CLI slot configurations sharing among gNBs were provided via backhaul signaling in CLI handling and RIM for NR.

UE-to-UE CLI handling

For co-channel operation, UE-to-UE intra-cell co-channel inter-subband CLI and UE-to-UE inter-cell co-channel inter-subband CLI are specific to SBFD operation, as shown in Figure 8. Their definitions are:

The interference is caused by UL transmission of the aggressor UE in a first set of contiguous RBs in UL subband in a carrier to DL reception of the victim UE in a second set of contiguous RBs in DL subband in the same cell in the same carrier, where the two contiguous RB sets are non-overlapping in frequency.

The interference is caused by UL transmission of the aggressor UE in a first set of contiguous RBs in UL subband in a carrier to DL reception of the victim UE in a second set of contiguous RBs in DL subband in the neighboring cell in the same carrier, where the two contiguous RB sets are non-overlapping in frequency.

Figure 8: Interference in co-existence case with SBFD

Inter-UE inter-subband CLI measurement

According to the location of CLI measurement, there were three methods provided during the previous meeting.

Agreement(RAN1#112) For inter-UE inter-subband CLI measurement, study at least the following methods: - Method#1: victim UE measures RSSI within DL subband - FFS: Whether SINR can be measured - Method#2: victim UE measures RSRP of aggressor UE within UL subband - Method#3: victim UE measures RSSI within UL subband - Note: the restriction in Rel-16 that CLI is only measured within DL BWP does not forbid UE to measure CLI in UL subband when UL subband is confined within DL BWP. Agreement(RAN1#112b-e) For inter-UE inter-subband CLI measurement, study Method#2 and Method#3 considering: - Necessity/benefit compared with measurement within DL subband - Whether/how to estimate CLI from RSRP/RSSI measurements within UL subband / guardband - Whether UE is required to measure RSRP/RSSI within UL subband and receive DL in DL subband(s) simultaneously - Whether existing CLI measurement and report framework can be reused to support RSRP/RSSI measurements within UL subband - If not, identify the potential impact

DL subband in Method#1 can be measured for UE-to-UE inter-subband CLI handling. Because victim UE’s reception in DL subband are impacted. It is the most straight forward method for inter-UE inter-subband CLI measurement.

However, Method#2/3 has strong relationship with SBFD aware UE behaviors in DL and Flexible symbols. If Option 1 is applied, Method#2/3 is excluded by default.

- Victim UE measures CLI within DL subband for UE-to-UE inter-subband CLI handling.

gNB-to-gNB CLI mitigation

gNB-to-gNB inter-cell co-channel inter-subband CLI is caused by DL transmission of the aggressor gNB in a first set of contiguous RBs in a carrier to UL reception of the victim gNB in a second set of contiguous RBs in the neighboring cell in the same carrier, where the two contiguous RB sets are non-overlapping in frequency.

Conclusion Time misalignment at gNB between UL receptions and DL transmissions due to configuration of non-zero NTA,offset at UE can lead to increased interference assuming no gNB transmit chain side impairments and no filtering of DL subband(s) in the gNB Rx chain. - FFS the case with gNB transmit chain impairments and/or filtering of DL subband(s) in the gNB Rx chain - FFS whether/how to mitigate the interference increase, including impact to legacy UEs

Similarly, victim gNB measures CLI within UL subband for gNB-to-gNB inter-subband CLI handling.

- Victim gNB measures CLI within UL subband for gNB-to-gNB inter-subband CLI handling.

Others

SBFD operation

Obviously, gNB is aware of all DL/UL directions on DL and UL subbands, one question is whether a UE should know the deployment of SBFD or not, and what type of SBFD information is necessary. From the discussions of last RAN1 meeting, the following SBFD operation Alt 4 was agreed as the baseline.

Agreement For SBFD operation at least for RRC_CONNECTED state, it is agreed that SBFD operation Alt 4 is the baseline. - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

According to this issue, we provide our views.

For SBFD operation Alt 1, it is the fundamental for legacy UE to work well in SBFD supporting network. It does not need extra work.  It is a completely transparent SBFD scheme from the UE’s point of view. All the SBFD operation can be only within any Flexible slots/symbols configured by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated (if any).

For SBFD operation Alt 2: UE does not aware of any SBFD information on time and frequency domain. But new UE behaviours can be introduced for SBFD aware UE, such as allowing additional collision handling cases, new resources allocation etc.

- First, UE only do what it is told to do. Such as if configured with or scheduled the DL reception, UE only do the DL channel reception on the DL BWP. It does care about whether there is some UL transmission from another UE meanwhile within the frequency domain resource of its DL BWP. Rel-18 UE still does not know any additional information about SBFD.

- For example, unlike legacy UE, Rel-18 SBFD UE supports to override a DL symbol to flexible/UL symbol by a type of signaling. Such as tdd-UL-DL-ConfigurationDedicated singling can override the DL symbol configured by tdd-UL-DL-ConfigurationCommon into UL symbol, UE sets its link directions according to the later tdd-UL-DL-ConfigurationDedicated singling. While another UE does not override according to its own tdd-UL-DL-ConfigurationDedicated singling. Through this type of UE-specific singling, different UE can have different link directions, to achieve the SBFD operations from gNB’s perspective. The SBFD operations can be within any Flexible and DL slots/symbols configured by tdd-UL-DL-ConfigurationCommon. After the overriding, the current DL and UL collision handling is done based on these directions. No further new collision handling is needed. Figure example, SBFD is used at gNB side with DUD subband configuration in slot 1-3 in Figure 1. DL slot 3 are override into UL for UE1, while DL slot 1/2 are override into UL for UE2. gNB can simultaneously schedule downlink UE in downlink subbands and uplink UE in uplink subband with SBFD operations on flexible slots 1/2/3.

- One benefit of Alt 2 is it can be used in the current network deployment more smoothly, because flexible slots/symbols are little used now. So DL slots/symbols can be override by tdd-UL-DL-ConfigurationDedicated signaling provides more SBFD operation chances.

- Another advantage of this scheme is original three signalling types can be used as a starting point, without extra time domain signaling for UL subband location for UE. There is no difference between legacy UL symbol and UL subband from the UE behaviours’ point of view.

Figure 9: Example for Alt 2

For SBFD operation Alt 3: UE assumes the link directions of slot/symbol based on more overriding rule on the current mechanisms, maybe know some time domain SBFD information e.g. the slots that override by new rules.

- This option requires the gNB to take care of the scheduling cautiously, such as there would be no obscure DL/UL judgement, i.e. UE does not expect to do semi-static DL reception and semi-static UL transmission simultaneously, or dynamic DL reception and dynamic UL transmission in at least one overlapping symbol. Fortunately, those two ambiguous cases are not allowed in the current specification, it can also apply this limitation to SBFD. For the remaining collisions, there would be no problem based on the priority between DL and UL direction. So SBFD can be realized transparently to UE via scheduling

- For example, SBFD is used at gNB side with DUD subband configuration in slot 1-3 in Figure 1 Option 1-1. UE1 receives downlink signal in slots 0/1/2 and transmit uplink signal in slot 3/4 according to slot configuration DFFFU and DL scheduling. UE2 transmits uplink signal in slot 1, 2 and 4 and receives downlink signal in slot 0 and 3 according to slot configuration DFFFU and DL scheduling in DL slot 0 and flexible slot 3, and UL scheduling in flexible slot 1, 2 and UL slot 4. In this case, gNB can simultaneously scheduling downlink UE in downlink subbands and uplink UE in uplink subband with SBFD operations on flexible slots 1/2/3.

Figure 10: Example for Alt 3

- In addition to Alt 4, Alt 2 can be further studied for SBFD operation.

  - UE does not aware of any SBFD information on time and frequency domain.

Co-existence with legacy UE/ gNB

Co-channel co-existence case was specified as deployment case 3 in sub item 9.3.1 in last meeting. Evaluation is conducted on the assumption that among the cells belonging to the operator, some of them use legacy TDD operation while the others use SBFD operation with the same SBFD subband configuration. 1-layer and 2-layer deployment cases are in the scope and the CLI analysis are shown in Figure 9.

Figure 11: co-channel co-existence scenarios for 1-layer and 2-layer

Considering UL subband in legacy DL slots for SBFD, the UL reception SBFD gNB suffers inter/intra subband CLI from DL transmission of legacy gNB while legacy UE suffers inter/intra subband CLI from UEs transmitting UL signal in SBFD cells. Because the performance of legacy UEs could be seriously degraded by intra subband CLI caused by SBFD UEs, further study on intra subband CLI mitigation on SBFD UEs is needed.

- In co-channel co-existence case of legacy UE/gNB and SBFD, there is no impact to the legacy gNB, but impact to legacy UE. CLI mitigation scheme for aggressor SBFD UEs should be studied.

For SBFD operation, one key issue is to handle gNB-to-gNB adjacent-channel CLI and/or co-channel-CLI of the same or different operators. Two SBFD patterns in Figure 10can be potential candidates for SBFD operation, which embed a small UL band into DL slots or allocate a small UL band at the edge carrier of DL slots. The candidate subband patterns are beneficial to reduce both inter-subband CLI & inter-operator CLI at gNB side, and inter-subband CLI & inter-operator CLI at UE side, as the rest DL band part of DL slots can act as guard band for those new UL band in the original DL slots.

Figure 12: The candidate subband patterns for SBFD operation

According to the approved Rel-18 NR duplex SID, adjacent-channel co-existence with the legacy operation should study the feasibility of and impact on RF requirements. As RedCap operation has been specified in Rel-17 and R18 RedCap is on the road, from our perspective, good coexistence performance should be studied not only between SBFD capable UE and legacy non-RedCap UE, but also between SBFD capable UE and R17 RedCap UE.

During R17 RedCap discussion, one of the most important coexistence issues is to solve PUSCH resource fragmentation, which will cause UL peak data rate reduction for non-RedCap UEs not supporting non-contiguous UL resource allocation. Finally, network is possible to configure RedCap UL and DL BWP at the edge of channel bandwidth to avoid PUSCH resource fragmentation, while can still maintain TDD DL-UL center frequency alignment.

- Both for R18 duplex operation enhancement and R17 RedCap, to achieve good co-existence performance with legacy network/UE, the bandwidth part location needs to be carefully designed.

As well known, UE power consumption is tightly depended on DL BWP bandwidth based on the power consumption model in 38.840 [3]. Although non-RedCap UEs support non-contiguous DL resource allocation, DL resource fragmentation introduced by NR duplex operation may not be friend to non-RedCap UEs from UE power consumption perspective. Figure 11 below is an example of DL resource fragmentation caused by SBFD operation.

Figure 13: An example of DL resource fragmentation caused by SBFD operation

- DL resource fragmentation caused by NR duplex operation may not be friendly to legacy UEs from UE power consumption perspective.

The main RedCap market is focused on use cases with relatively low cost/complexity, low energy consumption, and low data rate requirements. Supporting SBFD operation will not only increased the complexity for RedCap UE, but also will complicate the whole system design. Therefore, SBFD operation is not recommended for RedCap UE.

To avoid or minimize the DL resource fragmentation problem introduced by NR duplex operation, one possible solution is to configure SBFD band close to the DL BWP of R17 RedCap which is configured at the carrier edge. One of most benefit of this solution is that UL SBFD can use RedCap DL BWP as guard band to mitigate the inter-operator CLI, while can maximally avoid DL resource fragmentation for legacy non-RedCap UEs. Another potential solution is to configure SBFD band at the opposite side of edge carrier while R17 RedCap DL BWP is configured at the other side of carrier edge. These two example solutions are depended on operators’ network deployments, which are illustrated in the following Figure 12.

Figure 14: Example deployments for SBFD and R17 RedCap

- The introduction of SBFD operation should study co-existence performance with the legacy NR operation including RedCap deployment, which allow network to configure SBFD and RedCap BWP in a legacy NR carrier simultaneously, meanwhile minimizing both UL resource fragmentation and DL resource fragmentation.

Conclusion

Based on the analyses and discussions on SBFD, following observations and proposals are given:

- For frequency location configuration of DL subband(s) and guardband(s) if any,

  - Option 1 can be simple to avoid any arguments of whether guard bands are exist and whether those can be used for Tx and Rx.

  - The location and size of UL subband and DL subband(s) are purely frequency locations can be applied for UL or DL. It is clearly for both of discussions and definitions.

  - For the guardband(s), they can be unawareness.

- DCI schedule UL/DL to support DL in RBs outside DL subbands and UL in RBs outside UL subband cannot work.

- For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), among separate FDRA determination methods:

  - Option 3 would reduce the SBFD performance gain if a DL/UL channel/signal overlapping with RBs outside DL/UL subbands in a SBFD slot.

  - Option 2 cannot be applied for PUSCH, PUCCH, CSI-RS and SRS. Since reference signalling do not support any type of rate matching and puncturing. In addition, PUCCH/PUSCH also has big trouble to apply rate matching or puncturing, due to significant impacts to the UCI mapping mechanism in legacy. The only possible one is rate matching on the RBs outside DL subbands for PDSCH.

- The resource allocation in time domain for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols can be postponed until there are sufficient driving.

- The power domain for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols can be postponed until there are sufficient driving from the CLI.

- The spatial domain for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols can be postponed until there are sufficient driving from the CLI.

- When PDCCH monitoring occasions crossing SBFD and non-SBFD symbols in different slots,

  - Option 1 would increase the CORESET number per BWP, which will limit UE capability of maximum number of CORESET.

  - Option 2: Rate matching or puncturing on the REG(s) is an unpractical solution, due to a huge change to REG-CCE mapping

- In co-channel co-existence case of legacy UE/gNB and SBFD, there is no impact to the legacy gNB, but impact to legacy UE. CLI mitigation scheme for aggressor SBFD UEs should be studied.

- Both for R18 duplex operation enhancement and R17 RedCap, to achieve good co-existence performance with legacy network/UE, the bandwidth part location needs to be carefully designed.

- DL resource fragmentation caused by NR duplex operation may not be friendly to legacy UEs from UE power consumption perspective.

- The slot configuration period of tdd-UL-DL-ConfigurationCommon can be used as the period of SBFD time domain location, including slot configuration period in pattern1 and pattern2 (if any).

- Both of Cell-common and UE-specific subband time location indication can be study further.

- At least slot level can be study for SBFD time location, including continuous and discontinuous slots in the period.

- Symbol level configuration granularity of SBFD time domain location is with lower priority.

- Maximum number of switching points between SBFD and non-SBFD symbols within a slot is up to 1 due to TDD UL/DL configuration.

  - Since the TDD UL/DL configuration can only lead to one switching point in a slot by tdd-UL-DL-ConfigurationDedicated.

- Regarding the informing method of frequency locations of subbands for SBFD operation, it can be configured relative to CRB0 and length of continuous CRB number.

- For frequency location configuration of DL subband(s) and guardband(s) if any use Option 1, frequency locations of DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

- The cell-common explicit indication of frequency location of UL/DL subband(s) are the baseline for study.

- Support Option 1 for SBFD operation in a symbol configured as DL/flexible in TDD-UL-DL-ConfigCommon.

- For SBFD operation in SSB symbols, an UL subband can be configured in an SSB symbol.

- When UL subband is overlapping with SSB symbol, SBFD-aware UE cannot transmit in the SSB symbol but can only receive within the entire DL BWP in the SSB symbol.

- For the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband(s) is supported.

- For a PRG that overlaps with subband boundary, partial DL PRG inside the DL subband can be used.

- If PRG is determined as wideband, non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated (Option 1).

  - Same precoding is applied to all the allocated resources across two DL subbands.

- For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), if the transmissions/receptions can be in SBFD symbols and non-SBFD symbols with different available resources, further consider the following two options of separate FDRA determination for SBFD slots and non-SBFD slots for PDSCH, CSI-RS, PUSCH, PUCCH, SRS for SBFD-aware UE

  - Option 1-1: Separate FDRA configurations/indications for SBFD slots and non-SBFD slots

  - Option 1-2: Separate frequency resources determined for SBFD slots and non-SBFD slots based on single FDRA configuration/indication

- When PDCCH monitoring occasions crossing SBFD and non-SBFD symbols in different slots, it can be guaranteed by PDCCH monitoring occasion and CORESET configuration to never overlap with UL subband in SBFD symbols.

- PUCCH/PUSCH/PDSCH/PDCCH cannot be mapped to SBFD and non-SBFD regions in the same slot.

- For SBFD-aware UEs, for CSI report associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols), support Option 2: same CSI reporting for SBFD symbols and non-SBFD symbols

  - Option 2-2 is preferred: One CSI-ReportConfig is associated with one CSI-RS. The CSI report is derived based on CSI-RS which can be in SBFD symbols or non-SBFD symbols in different time instances.

- For collision between dynamic UL transmissions and dynamic DL receptions, it can treated as an error case

- Before to study collision between UL transmissions and DL receptions, it needs to align the understanding of UE assumption of link direction:

  - Alt 1: UE can use either DL or UL subband in a SBFD symbol, depends on DL reception or UL transmission.

  - Alt 2: UE can only use one subband direction of a symbol through configuration signalling.

- Victim UE measures CLI within DL subband for UE-to-UE inter-subband CLI handling.

- Victim gNB measures CLI within UL subband for gNB-to-gNB inter-subband CLI handling.

- In addition to Alt 4, Alt 2 can be further studied for SBFD operation.

  - UE does not aware of any SBFD information on time and frequency domain.

- The introduction of SBFD operation should study co-existence performance with the legacy NR operation including RedCap deployment, which allow network to configure SBFD and RedCap BWP in a legacy NR carrier simultaneously, meanwhile minimizing both UL resource fragmentation and DL resource fragmentation.

References

- RP-213591, Study on Evolution of NR Duplex Operation, CMCC, RAN#94-e, December 6-17, 2021

- Draft_Minutes_report_RAN1#112b-e

- R1-2304031	Summary #4 of subband non-overlapping full duplex	Moderator (CATT)