3GPP TSG RAN WG1 Meeting #113			    	            	R1-2304803

Incheon, Korea, May 22nd – 26th, 2023

Source:	Intel Corporation

Title:	Evaluations on LP WUS

Agenda item:	9.11.1

Document for:	Discussion and Decision

# Introduction

In last RAN1 meetings, the assumptions for the evaluation of power consumption/latency and link level simulation were agreed. Therefore, in this contribution, we provide the preliminary evaluation results.

# Power consumption / latency

In idle/inactive state, both basic IDRX operation and enhanced power saving schemes in Rel-16/17 can be considered to verify the potential gain of LP-WUS based operation. Specifically, there are four combinations for the reference cases: 1) IDRX without PEI; 2) IDRX with PEI; 1) eDRX without PEI; 2) eDRX with PEI. LP-WUS can be introduced to the existing IDRX operations in following three options. The relaxation on RRM by main radio or the LP-WUS to support RRM or related RRM should be studied.

- Option 1: LP-WUS serves as one bit indication to start IDRX procedure without PEI

- Option 2: LP-WUS serves as one bit indication to start IDRX procedure with PEI

- Option 3: LP-WUS carries paging subgroup information which can be same as PEI PDCCH.

Two operation modes for LP-WUS detection, always ON or with a duty cycle, were discussed in [3] which can be integrated with the above 3 options. We provide preliminary results on power consumption for LP-WUS based operation compared to the 4 reference cases and the 6 cases by combing the above 3 options for LP-WUS and the two operation modes. It assumes IDRX cycle of 1.28s and eDRX cycle 61.44s. The paging arrival rate is set to 0.1% or 0.01% per UE per paging cycle. It is assumed there are 10 UEs in the paging group. It assumes the number of paging subgroup is 8 for options based on PEI and subgroup indication by LP-WUS. The FAR for LP-WUS detection is assumed to 1% or 0.1%. 5 SSBs are assumed for sync/resync for LP-WUS waking-up. If PEI PDCCH is configured, it is assumed the PEI PDCCH is between the 3rd and 4th SSB. Both serving cell RRM and intra/inter-frequency RRM are done when RRM is performed in an occasion by MR. For duty-cycle based LP-WUS operation, it assumes 1ms in every 100ms is available for LP-WUS detection. A duty cycle operation always provides low power consumption than always on operation. In certain scenarios, always on operation may result in even larger power consumption than reference cases while duty cycle operation can still provide power saving gain.

# Power consumption w/o consideration of RRM

The power saving gain of LP-WUS highly depends on the assumption for RRM measurement by main radio by MR. In this section, we perform evaluations for power consumption/latency based on LP-WUS without consideration of the RRM by MR. The results are applicable if LP-WUS can support RRM measurement with sufficient accuracy. Otherwise, the results can be seen as an upper bound for the benefit that can be achieved by LP-WUS. The power consumption value is normalized to the value for IDRX with PEI.

Table 1 provides the power consumption for different schemes assuming (Ramp-up and down transition energy, ramp-up time) of (15000, 400ms).

Table 1: Power consumption w/o RRM by MR (15000, 400ms)

|  | R_E = 1%, FAR 1% |  |  |  |  | R_E = 1%, FAR 0.1% |  |  |  |  | R_E = 0.1%, FAR 0.1% |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LR on power | 0.01 | 0.1 | 0.5 | 1 | 4 | 0.01 | 0.1 | 0.5 | 1 | 4 | 0.01 | 0.1 | 0.5 | 1 | 4 |
| IDRX, no PEI | 1.42 |  |  |  |  | 1.42 |  |  |  |  | 1.43 |  |  |  |  |
| IDRX, with PEI | 1 |  |  |  |  | 1 |  |  |  |  | 1 |  |  |  |  |
| eDRX, no PEI | 0.24 |  |  |  |  | 0.24 |  |  |  |  | 0.24 |  |  |  |  |
| eDRX, with PEI | 0.21 |  |  |  |  | 0.21 |  |  |  |  | 0.2 |  |  |  |  |
| LP-WUS always on, Option 1 | 0.73 | 0.78 | 0.96 | 1.2 | 2.58 | 0.68 | 0.72 | 0.91 | 1.14 | 2.53 | 0.15 | 0.18 | 0.38 | 0.6 | 2 |
| LP-WUS always on, Option 2 | 0.68 | 0.72 | 0.91 | 1.14 | 2.52 | 0.63 | 0.67 | 0.86 | 1.09 | 2.48 | 0.13 | 0.17 | 0.36 | 0.6 | 1.99 |
| LP-WUS always on, Option 3 | 0.11 | 0.14 | 0.33 | 0.56 | 1.95 | 0.1 | 0.14 | 0.32 | 0.55 | 1.94 | 0.03 | 0.07 | 0.26 | 0.49 | 1.88 |
| LP-WUS duty cycle, Option 1 | 0.74 | 0.74 | 0.74 | 0.74 | 0.75 | 0.68 | 0.68 | 0.68 | 0.69 | 0.7 | 0.14 | 0.14 | 0.15 | 0.15 | 0.16 |
| LP-WUS duty cycle, Option 2 | 0.68 | 0.68 | 0.68 | 0.68 | 0.69 | 0.63 | 0.63 | 0.63 | 0.63 | 0.65 | 0.13 | 0.13 | 0.14 | 0.13 | 0.15 |
| LP-WUS duty cycle, Option 3 | 0.1 | 0.1 | 0.1 | 0.11 | 0.12 | 0.09 | 0.09 | 0.1 | 0.1 | 0.11 | 0.03 | 0.03 | 0.03 | 0.03 | 0.05 |

Based on the results in Table 1, significant benefit on power saving in LP-WUS based operation are observed except when the paging arrival rate or the relative on power is high. The issue can be simply solved by duty-cycle based LP-WUS detection. Reduced power consumption is observed with reduced FAR or reduced paging arrival rate R_E. The duty-cycle based operation can save much more power than always on operation. For the 3 options for LP-WUS operations, Option 3 (LP-WUS to indicate paging sub-group) provides the lowest power consumption. Since paging subgroup is indicated by LP-WUS, it avoids unnecessary waking up the main radio if there is no paging message to the UE.

The power consumptions for (40000, 800ms) is provided in Table 2. Comparing with Table 1, large increase of power consumption is observed for high paging arrival rate R_E = 1% which results in frequent application of transmission energy of 40000 units. The power increase for lower R_E = 0. 1% is marginal.

Table 2: Power consumption w/o RRM by MR (40000, 800ms)

|  | R_E = 1%, FAR 1% |  |  |  |  | R_E = 1%, FAR 0.1% |  |  |  |  | R_E = 0.1%, FAR 0.1% |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LR on power | 0.01 | 0.1 | 0.5 | 1 | 4 | 0.01 | 0.1 | 0.5 | 1 | 4 | 0.01 | 0.1 | 0.5 | 1 | 4 |
| IDRX, no PEI | 1.42 |  |  |  |  | 1.42 |  |  |  |  | 1.43 |  |  |  |  |
| IDRX, with PEI | 1 |  |  |  |  | 1 |  |  |  |  | 1 |  |  |  |  |
| eDRX, no PEI | 0.43 |  |  |  |  | 0.43 |  |  |  |  | 0.43 |  |  |  |  |
| eDRX, with PEI | 0.4 |  |  |  |  | 0.4 |  |  |  |  | 0.39 |  |  |  |  |
| LP-WUS always on, Option 1 | 1.71 | 1.76 | 1.93 | 2.17 | 3.56 | 1.59 | 1.63 | 1.81 | 2.04 | 3.43 | 0.18 | 0.22 | 0.41 | 0.64 | 2.04 |
| LP-WUS always on, Option 2 | 1.66 | 1.69 | 1.88 | 2.12 | 3.5 | 1.53 | 1.58 | 1.76 | 1.99 | 3.38 | 0.18 | 0.22 | 0.4 | 0.64 | 2.03 |
| LP-WUS always on, Option 3 | 0.23 | 0.27 | 0.46 | 0.69 | 2.06 | 0.22 | 0.25 | 0.44 | 0.66 | 2.05 | 0.03 | 0.08 | 0.26 | 0.49 | 1.89 |
| LP-WUS duty cycle, Option 1 | 1.71 | 1.71 | 1.71 | 1.72 | 1.73 | 1.58 | 1.58 | 1.59 | 1.59 | 1.6 | 0.18 | 0.18 | 0.18 | 0.18 | 0.2 |
| LP-WUS duty cycle, Option 2 | 1.66 | 1.66 | 1.66 | 1.66 | 1.66 | 1.53 | 1.53 | 1.53 | 1.53 | 1.55 | 0.17 | 0.18 | 0.18 | 0.18 | 0.2 |
| LP-WUS duty cycle, Option 3 | 0.22 | 0.21 | 0.23 | 0.23 | 0.24 | 0.21 | 0.21 | 0.22 | 0.22 | 0.23 | 0.03 | 0.03 | 0.03 | 0.04 | 0.05 |

Observation 1: For idle/inactive mode, without consideration on RRM by main radio

- For power consumption of LP-WUS operation compared with IDRX and eDRX

  - Significant benefit on power saving in LP-WUS operation are observed for both IDRX and eDRX, except when the paging arrival rate or the relative on power is high.

  - Large power saving gain is still observed even with extremely high total transition energy 40000 units if paging arrival rate and FAR are reduced or LP-WUS (Option 3) is applied.

  - Reduced power consumption is observed with reduced FAR or reduced paging arrival rate R_E.

  - The duty-cycle based operation can save much more power than always on operation.

  - The lowest power consumption is observed when paging subgroup is indicated by LP-WUS (Option 3)

# Power consumption with RRM

For a UE in idle/inactive mode, it is important for the UE to do RRM measurement for potential cell reselection. The RRM measurement by main radio in every paging cycle in the existing NR may consume large power. Therefore, relaxed RRM measurement by main radio was proposed in early RAN1 meetings. The RRM based on LP-WUS is further proposed which may compensate the performance on RRM measurement and cell reselection. However, whether LP-WUS based RRM is practical needs further study. In this section, we perform evaluations for power consumption/latency based on LP-WUS considering relaxed RRM measurement by main radio. The power consumption value is normalized to the value for IDRX with PEI.

Table 3 provides the power consumption for different schemes assuming (Ramp-up and down transition energy, ramp-up time) of (15000, 400ms). It is assumed that the RRM by MR is done in every 20 paging cycles.

Table 3: Power consumption w/ RRM by MR (15000, 400ms)

|  | R_E = 1%, FAR 1% |  |  |  |  | R_E = 1%, FAR 0.1% |  |  |  |  | R_E = 0.1%, FAR 0.1% |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LR on power | 0.01 | 0.1 | 0.5 | 1 | 4 | 0.01 | 0.1 | 0.5 | 1 | 4 | 0.01 | 0.1 | 0.5 | 1 | 4 |
| IDRX, no PEI | 1.42 |  |  |  |  | 1.42 |  |  |  |  | 1.43 |  |  |  |  |
| IDRX, with PEI | 1 |  |  |  |  | 1 |  |  |  |  | 1 |  |  |  |  |
| eDRX, no PEI | 0.24 |  |  |  |  | 0.24 |  |  |  |  | 0.24 |  |  |  |  |
| eDRX, with PEI | 0.21 |  |  |  |  | 0.21 |  |  |  |  | 0.2 |  |  |  |  |
| LP-WUS always on, Option 1 | 0.81 | 0.86 | 1.04 | 1.27 | 2.66 | 0.77 | 0.81 | 0.99 | 1.22 | 2.61 | 0.34 | 0.39 | 0.57 | 0.81 | 2.2 |
| LP-WUS always on, Option 2 | 0.75 | 0.8 | 0.98 | 1.21 | 2.6 | 0.71 | 0.76 | 0.94 | 1.17 | 2.56 | 0.34 | 0.38 | 0.57 | 0.8 | 2.2 |
| LP-WUS always on, Option 3 | 0.36 | 0.4 | 0.58 | 0.81 | 2.2 | 0.35 | 0.39 | 0.58 | 0.81 | 2.2 | 0.31 | 0.35 | 0.54 | 0.77 | 2.17 |
| LP-WUS duty cycle, Option 1 | 0.81 | 0.81 | 0.81 | 0.81 | 0.83 | 0.76 | 0.77 | 0.77 | 0.77 | 0.78 | 0.34 | 0.34 | 0.34 | 0.35 | 0.36 |
| LP-WUS duty cycle, Option 2 | 0.75 | 0.75 | 0.76 | 0.75 | 0.77 | 0.71 | 0.71 | 0.72 | 0.72 | 0.73 | 0.34 | 0.34 | 0.34 | 0.34 | 0.36 |
| LP-WUS duty cycle, Option 3 | 0.35 | 0.35 | 0.35 | 0.36 | 0.37 | 0.35 | 0.35 | 0.35 | 0.35 | 0.37 | 0.31 | 0.31 | 0.31 | 0.31 | 0.33 |

Based on the results in Table 3, large power saving gain is observed except when LP-WUS is always on with on power of 2 or 4 units. The gain is reduced comparing with Table 1 due to the power consumption for RRM by main radio in every 20 paging cycles. Similarly, reduced power consumption is observed with reduced FAR or reduced paging arrival rate R_E. The duty-cycle based operation still save more power, however the benefit over always on decreases comparing with Table 1. For the 3 options for LP-WUS operations, Option 3 (LP-WUS to indicate paging sub-group) provides the lowest power consumption which becomes larger for higher paging arrival rate.

For the results of (40000, 800ms) in Table 4, increased power consumption is observed compared to Table 3. Always on LP-WUS operation almost always cause losses except Option 3 or lower relative on power, paging arrival rate R_E and FAR. Duty-cycle operation of LP-WUS still provides gains for LP-WUS Option 3 or low paging arrival rate and FAR.

Table 4: Power consumption w/o RRM by MR (40000, 800ms)

|  | R_E = 1%, FAR 1% |  |  |  |  | R_E = 1%, FAR 0.1% |  |  |  |  | R_E = 0.1%, FAR 0.1% |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LR on power | 0.01 | 0.1 | 0.5 | 1 | 4 | 0.01 | 0.1 | 0.5 | 1 | 4 | 0.01 | 0.1 | 0.5 | 1 | 4 |
| IDRX, no PEI | 1.42 |  |  |  |  | 1.42 |  |  |  |  | 1.43 |  |  |  |  |
| IDRX, with PEI | 1 |  |  |  |  | 1 |  |  |  |  | 1 |  |  |  |  |
| eDRX, no PEI | 0.43 |  |  |  |  | 0.43 |  |  |  |  | 0.43 |  |  |  |  |
| eDRX, with PEI | 0.4 |  |  |  |  | 0.4 |  |  |  |  | 0.39 |  |  |  |  |
| LP-WUS always on, Option 1 | 1.9 | 1.95 | 2.13 | 2.37 | 3.75 | 1.81 | 1.85 | 2.03 | 2.26 | 3.65 | 0.85 | 0.89 | 1.08 | 1.31 | 2.71 |
| LP-WUS always on, Option 2 | 1.85 | 1.88 | 2.08 | 2.31 | 3.69 | 1.75 | 1.8 | 1.98 | 2.21 | 3.6 | 0.84 | 0.88 | 1.07 | 1.3 | 2.7 |
| LP-WUS always on, Option 3 | 0.87 | 0.91 | 1.1 | 1.33 | 2.71 | 0.86 | 0.9 | 1.08 | 1.32 | 2.7 | 0.77 | 0.81 | 1 | 1.23 | 2.63 |
| LP-WUS duty cycle, Option 1 | 1.9 | 1.91 | 1.9 | 1.91 | 1.92 | 1.8 | 1.8 | 1.8 | 1.81 | 1.82 | 0.84 | 0.85 | 0.85 | 0.85 | 0.86 |
| LP-WUS duty cycle, Option 2 | 1.85 | 1.85 | 1.85 | 1.85 | 1.86 | 1.75 | 1.75 | 1.75 | 1.75 | 1.77 | 0.84 | 0.84 | 0.84 | 0.84 | 0.86 |
| LP-WUS duty cycle, Option 3 | 0.86 | 0.86 | 0.87 | 0.87 | 0.88 | 0.86 | 0.86 | 0.86 | 0.86 | 0.88 | 0.77 | 0.77 | 0.77 | 0.77 | 0.78 |

Observation 2: For the power consumption of LP-WUS operation in idle/inactive mode

- For (15000, 400ms) and RRM by MR in every 20 paging cycles

  - large power saving gain is observed except when LP-WUS is always on with on power of 2 or 4 units. However, the gain is reduced compared to the case without RRM by MR.

  - Reduced power consumption is observed with reduced FAR or reduced paging arrival rate R_E.

  - The duty-cycle based operation still save more power, however the benefit over always on decreases.

  - The lowest power consumption is observed when paging subgroup is indicated by LP-WUS

- For (40000, 800ms)

  - Always on LP-WUS operation almost always cause losses except Option 3 or lower relative on power, paging arrival rate R_E and FAR

  - Duty-cycle operation of LP-WUS still provides gains for LP-WUS Option 3 or low paging arrival rate and FAR

  - Reduced power consumption is observed with reduced FAR or reduced paging arrival rate R_E.

# Latency

The latency performance for LP-WUS operation is not impacted by the RRM operation, which is provided in Figure 1. As expected, LP-WUS enables a latency slightly higher than IDRX mainly due to the 400ms or 800ms ramp-up time and sync/resync time. Note: significant power saving gain for LP-WUS operation compared to IDRX was observed as captured in Table 1. For the comparation with eDRX, LP-WUS achieves significant gain in term of latency.

Figure 1: Latency of LP-WUS operation in idle/inactive mode

Observation 3: For idle/inactive mode,

- For latency of LP-WUS operation compared with IDFT and eDRX

  - LP-WUS enables a latency slightly higher than IDRX but significantly lower than eDRX.

# Link level simulation

As discussed in [3], the LP-WUS may contain two parts: Preamble part is for AGC and carries a sequence for synchronization, while message part may be used to carry multi-bit information for wake-up. Both parts of LP-WUS needs to be detected for correct wake-up indication. The link performance of the two parts may not be same. The final performance of LP-WUS detection needs to consider the two parts jointly. In the simulation, the message part is only processed when the preamble part is successfully identified. Based on the agreement in last RAN1 meetings, we provide detailed simulation assumptions in Table 5.

Table 5: Simulation assumptions

| Attributes | Assumptions |
| --- | --- |
| Carrier frequency | 2.6GHz |
| Channel structure | Preamble + message |
| Coding scheme | Manchester coding 1/2,1/4, 1/8 |
| Waveform | OOK-4 |
| Payload size | 4 information bits (no CRC) |
| WUS waveform | OOK-1, OOK-4 |
| SCS | OOK-1: 160/20/240kHz OOK-4: 15/30kHz |
| gNB Channel BW | 20MHz |
| WUS Bandwidth | 12/10/8/4 PRBs with SCS 30kHz 24 PRB for SCS 15kHz 3/6/12 PRBs for SCS 60/120/240kHz |
| Guard band | 1 PRB on each side of LP-WUS BW |
| Adjacent subcarrier interference | Signal to interference power ratio 0/3/6 dB |
| Filter | 5th order Butterworth LP filter |
| Sampling Rate | Down-sampling factor 8, i.e., 3.84MHz |
| Frequency error/drifts | 0/20/100/200 ppm |
| ADC | 1/2/4, ideal |
| Channel Model | TDL-C 300ns |
| Number of Rx for LP-WUS | 1 Rx |
| UE speed | 3 km/h |

# Simulation results

According to the agreed simulation assumptions, link level simulations were performed to check multiple design aspects for LP-WUS. The FAR is set to 1% in the results. SCS 30kHz is assumed in the evaluation except for Figure 8. The results in this section is focusing on OOK-4, while the comparison between OOK-4 and OOK-1 is provided in our companion contribution [3].

- The proper BW of LP-WUS

- The number of MC-OOK symbols that can be multiplexed in an OFDM symbol of main radio

- The impact of Manchester coding with spreading factor 2 or 4

- The ADC bit-width

- The impact of adjacent subcarrier interference

- The impact of time/frequency error

- The impact of SCS

The link performance for LP-WUS with different BW is provided in Figure 2. A larger BW always has better link performance due to the increase of transmission power and the larger frequency diversity benefit. The frequency diversity gain may not increase for even higher BW, which causes large overhead in frequency too. In summary, a BW of 12PRB achieves a good balance of link performance and resource efficiency.

Figure 2: BLER for LP-WUS with different BW

Thanks to the filter at the LP-WUR, it doesn’t have a large impact to the LP-WUS detection performance with the agreed adjacent subcarrier interference. Even for the case signal to interference ratio of -6dB, the performance degradation is marginal as shown in Figure 3. In summary, OOK based LP-WUS is insensitive to adjacent subcarrier interference.

Figure 3: BLER for LP-WUS with different adjacent subcarrier interference

The number of OOK symbols in an OFDM symbol, i.e., M, or equivalently the OOK symbol duration is another key factor for LP-WUS design. With a fixed M, the different spreading factor by Manchester coding results in different total transmission duration hence the total energy for an information bit. The results are provided in Figure 4. From the results, with equal total duration of an information bit, a larger OOK symbol duration (low M) is better to overcome the multiple delay of the channel which cause ISI. On the other hand, a larger duration for an information bit (low M or high SF) may lose some power gain due to non-coherent detection (about 1~2 dB performance gain is observed when OOK symbol duration is doubled, i.e., 3dB power increase). The choice of proper M and SF depends on the target data rate and coverage requirement.

Figure 4: BLER for LP-WUS with different number of OOK symbols per OFDM symbol

Observation 4: Based on the link level simulations

- A BW of 12PRB for LP-WUS achieves a good balance of link performance and resource efficiency

- On the impact of the number of OOK symbols per OFDM symbol and the Manchester spreading factor

  - with equal total duration of an information bit, a larger OOK symbol duration (smaller M) is better

  - a larger duration for an information bit (smaller M or larger SF) may lose some power gain due to non-coherent detection.

- ADC of 3 bits or more achieves ideal performance.

- The performance of LP-WUS is not sensitive to frequency error.

- The timing error has significant impact on OOK based LP-WUS transmission if the time error is not corrected.

- the LP-WUS performance for SCS 15 or 30kHz are almost equal assuming the same total duration per information bit.

# Preliminary MIL for link budget

As shown in Figure 4, the required SNR for 1% BLER for LP-WUS detection ranges from -3.5 ~ 6dB. This range of required SNR can be used to derive some preliminary insight on the link budget MIL for LP-WUS. Note: the above required SNR is determined assuming payload of 4 bits together with certain assumption on the number of OOK symbols per OFDM symbol and the spreading factor of Manchester coding. The required SNR hence will change when the simulation assumptions are changed.

Table 3: Link budget MIL

| System configuration | Common PDCCH |  | PUSCH (1Mbps) | LP-WUS |
| --- | --- | --- | --- | --- |
| Carrier frequency (GHz) | 2.6 |  |  |  |
| Pathloss model | TDL-C, 300ns |  |  |  |
| UE speed (km/h) | 3 |  |  |  |
| Number of transmit chains | 4 | 4 | 1 | 4 |
| Number of receive antennas | 2 | 1 | 4 | 1 |
| PSD | 33 | 33 |  | 33 |
| Occupied BW (PRB) | 48 | 48 | 10 | 12 |
| Transmit power for occupied BW | 45.38 | 45.38 | 23 | 39.35 |
| Data channel EIRP | 62.86 | 62.86 | 22 | 56.84 |
| Receiver noise figure (dB) | 7.00 | 7.00 | 5.00 | 15.00 |
| Required SNR (dB) | -8.3 | -4.50 | -5.25 | -3.5 – 6 |
| Receiver sensitivity | -97.12 | -93.32 | -99.61 | -93.95 |
| Link budget (MIL) | 162.78 | 158.98 | 140.35 | 149.98 - 140.48 |

From the above calculation, MIL for LP-WUS can be better than PUSCH. However, there is a big gap between the MIL for LP-WUS and common PDCCH with one or two Rx. Note: the MIL of unicast PUSCH largely depends on the assumed data rate for the UL traffic in coverage or RedCap study.

Observation 5:

- A preliminary calculation shows that the MIL for LP-WUS can be better than PUSCH but is much worse than common PDCCH.

# Conclusions

In this contribution, we presented our views on methodology and performance metric for the evaluations for LP-WUS/WUR. We made the following observations

Observation 1: For idle/inactive mode, without consideration on RRM by main radio

- For power consumption of LP-WUS operation compared with IDRX and eDRX

  - Significant benefit on power saving in LP-WUS operation are observed for both IDRX and eDRX, except when the paging arrival rate or the relative on power is high.

  - Large power saving gain is still observed even with extremely high total transition energy 40000 units if paging arrival rate and FAR are reduced or LP-WUS (Option 3) is applied.

  - Reduced power consumption is observed with reduced FAR or reduced paging arrival rate R_E.

  - The duty-cycle based operation can save much more power than always on operation.

  - The lowest power consumption is observed when paging subgroup is indicated by LP-WUS (Option 3)

Observation 2: For the power consumption of LP-WUS operation in idle/inactive mode

- For (15000, 400ms) and RRM by MR in every 20 paging cycles

  - large power saving gain is observed except when LP-WUS is always on with on power of 2 or 4 units. However, the gain is reduced compared to the case without RRM by MR.

  - Reduced power consumption is observed with reduced FAR or reduced paging arrival rate R_E.

  - The duty-cycle based operation still save more power, however the benefit over always on decreases.

  - The lowest power consumption is observed when paging subgroup is indicated by LP-WUS

- For (40000, 800ms)

  - Always on LP-WUS operation almost always cause losses except Option 3 or lower relative on power, paging arrival rate R_E and FAR

  - Duty-cycle operation of LP-WUS still provides gains for LP-WUS Option 3 or low paging arrival rate and FAR

  - Reduced power consumption is observed with reduced FAR or reduced paging arrival rate R_E.

Observation 3: For idle/inactive mode,

- For latency of LP-WUS operation compared with IDFT and eDRX

  - LP-WUS enables a latency slightly higher than IDRX but significantly lower than eDRX.

Observation 4: Based on the link level simulations

- A BW of 12PRB for LP-WUS achieves a good balance of link performance and resource efficiency

- On the impact of the number of OOK symbols per OFDM symbol and the Manchester spreading factor

  - with equal total duration of an information bit, a larger OOK symbol duration (smaller M) is better

  - a larger duration for an information bit (smaller M or larger SF) may lose some power gain due to non-coherent detection.

- ADC of 3 bits or more achieves ideal performance.

- The performance of LP-WUS is not sensitive to frequency error.

- The timing error has significant impact on OOK based LP-WUS transmission if the time error is not corrected.

- the LP-WUS performance for SCS 15 or 30kHz are almost equal assuming the same total duration per information bit.

Observation 5:

- A preliminary calculation shows that the MIL for LP-WUS can be better than PUSCH but is much worse than common PDCCH.

# References

[1] RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, vivo

[2] R1-2302816, Discussion on LP-WUS receiver architecture, Intel Corporation

[3] R1-2302817, Discussions on L1 signal design and procedure for LP-WUS, Intel Corporation