3GPP TSG RAN WG1 #113	 	                                                                                           R1-2304894

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.2.2.2

Source:	            		Xiaomi

Title:	Further discussion on specification impact for CSI feedback based on AI/ML

Document for:	Discussion and Decision

Introduction

In this contribution, we further discuss the potential specification impact on AI/ML based CSI compression feedback with two-sided model.

Discussion on potential specification impact for CSI compression feedback with two-sided AI/ML model

AI/ML model training collaboration type

In the RAN1#112-bis e-meeting, the pros and cons of different offline training collaboration types for CSI compression feedback based on two-sided AI/ML model were preliminarily discussed. According to the discussion, and the pros and cons are summarized by feature leader in Table 1.

Table 1. The pros/cons of different offline training collaboration types

| Training types Characteristics | Type 1 |  | Type 2 | Type 3 |  |
| --- | --- | --- | --- | --- | --- |
|  | NW-sided | UE-sided |  | NW first | UE first |
| Whether model can be kept proprietary | No | No | Yes | Yes (Note 3) | Yes (Note 3) |
| Whether require privacy-sensitive dataset sharing | No (Note 1) | No | No (Note 1) | No (Note 1) | No (Note 1) |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | Yes. With assisted information signaling. Less flexible than Type 1-NW side. | Difficult | Semi-flexible. | Semi-flexible. With assisted information signaling |
| Whether gNB/device specific optimization is allowed | Restricted | Restricted | Yes | Yes | Yes |
| Model update flexibility after deployment | Flexible (note 4) | Conditional, flexible with assisted information (note 4) | Not flexible (note 4) | Semi-flexible | Conditional semi-flexible, with assisted information (note 4) |
| Feasibility of allowing UE side and NW side to develop/update models separately | Limited (Note 2) | Limited (Note 2) | Infeasible | Feasible | Feasible |
| Whether gNB can maintain/store a single/unified model for a CSI report configuration | Yes No | No | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Whether UE device can maintain/store a single/unified model for a CSI report configuration | No | Yes | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Extendibility: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use | Limited (Note 2) | Limited (Note 2) | Limited | Support | Support |
| Whether training data distribution can match the inference device | Conditional, with assisted information from UE for device specific model. | Yes | Restricted | Conditional, with assisted information from UE | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | Yes for device specific model. No for device-agnostic model. | Limited Yes | Compatible | Compatible | Compatible |
| Model performance based on evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |

Note 1: Assume high accuracy PMI is not privacy sensitive data. FFS: other information such as channel matrix and assisted information.

Note 2: For example, after deploying model 1 on the UE side, a new UE model can be obtained by using model 1 as the teacher model and using knowledge distillation method. Model 1 can also refer to a nominal model while the real deployed model can be developed based on the nominal model.

Note 3: Assume information on model structure is not required to be disclosed in training collaboration type 3.

Note 4: Flexibility after deployment is evaluated by the amount of offline cross-vendor co-engineering effort. Flexible indicates minimum additional co-engineering between vendors, semi-flexible indicates additional co-engineering effort between vendors.

Note 5: Yellow highlighted rows are for further discussion.

According to whether the training entity has known the device information of the opposite entity, such as hardware or software implementation information, type 1 can be further categorized into device-agnostic and device-specific. For device-specific, the training entity has known the device information through offline method. The two different sub-types of Type 1 have different characteristics. In order to make common understanding for training type 1, it is necessary to firstly discuss the sub-types. However, considering the AI/ML model is trained in an offline way, the training entity can know the device information in proprietary method, i.e., the device information is not transferred through air interface. It does not require additional specification effort. For device-agnostic type, the trained model may be incompatible with device when training the two-sided model does not know the device information, or the device may obtain the sub-optimal model caused by device hardware or software implementation.  Thus, device-specific for type 1 can be considered to train two-sided AI/ML model.

Proposal 1: Device-specific for type 1 can be considered to train two-sided AI/ML model.

In Table 1, it has already contained 5 training types. In addition, some companies suggest the training type 4, i.e., gradient exchange sequential training is introduced. Thus, there are total 6 training types. Considering there are two left meeting in Rel-18 SI, we suggest at most two types are down-selected for study. For type 4, it has been discussed in AI 9.2.2.1. We can defer to the discussion of type 4 until it has preliminary conclusion in AI 9.2.2.1.

If AI/ML model is jointly trained by UE, i.e., joint training of the two-sided model at UE side, NW-side will store different AI/ML models for different UE vendors. Even for the same UE vendor, it may train different two-sided models if a generalized or scalability two-sided model cannot be trained for different scenarios and configurations. This implies that NW-sided will store and manage a lot of NW-sided part models, which will be a challenge task for network. Similarly, For UE-first separate training, it also has the same problem. Therefore, we should focus on discussing the pros/cons of joint training of the two-sided model at NW side for type 1 and NW-first separate training for type 3. According to above discussion, we just need to discuss the pros and cons of device-specific training type for type 1.

Observation 1: NW may store and manage a lot of NW-sided part models for joint training of the two-sided model at UE side for type 1 and UE-first separate training for type 3.

Proposal 2: The pros and cons of the joint training of two-sided model at NW side for type 1 with device-specific and NW-first separate training for type 3 should be firstly studied and discussed.

In Table 1, the pros and cons of joint training of the two-sided model at NW side for type 1 and NW-first separate training for type 3 were provided based on discussion in the last meeting. In principle, we are fine with the table. But, we have different views for some characteristics of the two training types, and discuss them respectively as follows.

- Whether model can be kept proprietary

For joint training of the two-sided model at NW side with device-specific, NW will transfer or deliver the UE-side part model to UE after training a two-sided model. Hence, the model proprietary cannot be kept for type 1. For type 3. Model transfer or deliver is not needed, which makes the proprietary of model be kept for type 3.

- gNB/device specific optimization – i.e., whether hardware-specific optimization of the model is possible, e.g. compilation for the specific hardware

For type 1, since two-sided model is jointly trained with device-specific. gNB can know the UE’s device information in an offline method. This implies that both gNB and UE can train device specific optimization model. For type 3, CSI generation part and CSI reconstruction model are separately trained at gNB and UE side. Thus, the trained partial model is device specific optimization for gNB and UE.

- Model update flexibility after deployment

For type 1, it refers to model transfer or deliver. This implies that the deployed model can be flexibly updated for different configurations or scenarios. However, UE needs to re-train UE-sided part model if the model is updated for type 3. The training dataset should be transferred to UE by gNB, which refers to the collaboration between gNB and UE in terms of engineering.

- Feasibility of allowing UE side and NW side to develop/update models separately

It is obviously that type 3 can allow UE and NW to develop and update models separately. However, it is not feasible that UE and NW to implement such operation for type 1. NW-sided part model and UE-sided part model need to jointly trained and they are paired. Once one part model is updated. The other part model should be updated accordingly. Otherwise, they will not be paired.

- Whether gNB or UE can maintain/store a single/unified model for a CSI report configuration

For type 1, since gNB trains the two-sided model by using joint training at network side and NW-first separated training according to training dataset from different UEs in a scenario, gNB can train and maintain or store a unified model for different UEs. For different cells, the channel condition is different for one UE. Assume that the UE in cell 1 use model 1. Once the UE moves to the cell 2, it will adopt model 2 which is transferred from cell 2 due to channel variation. We can see that UE is hard to maintain a single/unified model for type 1 for a CSI report configuration.

- Extendibility: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use

For such extendibility, in fact, it requires UE-side model or NW-side model to train or update their model separately. According to discussion on the feasibility of allowing UE side and NW side to develop/update models separately, type 3 has such extendibility. However, Type 1 does not have such ability.

- Whether training data distribution can be matched to the device that will use the model for inference

For type 1, gNB has known the hardware or software implementation information. The input format of model during model training is same to that of model inference. It insures the training data distribution can match the inference device.  However, for type 3, gNB does not know the input format of UE-side model during model training, the training data distribution may not match the inference device. In order to address the issue, UE can indicate the input format to gNB.

According to above discussion, the pros and cons of joint training of two-sided model at NW sided with device specific and NW-first separate training are given in Table 2. In the table, the red part denotes our updated views for these characteristics in addition to agreement on the pros and cons of other characteristics.

Table 2: The pros and cons of joint training of two-sided model at NW sided and NW-first separate training

| Training types Characteristics | Type 1 (Joint training of the two-sided model at NW side with device-specific) | Type 3 (NW-first separate training) |
| --- | --- | --- |
| Whether model can be kept proprietary | No | Yes (Note 3) |
| Whether require privacy-sensitive dataset sharing | No (Note 1) | No (Note 1) |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | Semi-flexible. |
| Whether gNB/device specific optimization is allowed | Yes | Yes |
| Model update flexibility after deployment | Flexible (note 4) | Semi-flexible (note 4) |
| Feasibility of allowing UE side and NW side to develop/update models separately | Limited (Note 2) | Feasible |
| Whether gNB can maintain/store a single/unified model for a CSI report configuration | Yes | Pending evaluation in 9.2.2.1 |
| Whether UE device can maintain/store a single/unified model for a CSI report configuration | No | Pending evaluation in 9.2.2.1 |
| Extendibility: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use | No | Support |
| Whether training data distribution can match the inference device | Yes | Conditional, with assisted information from UE |
| Software/hardware compatibility (Whether device capability can be considered for model development) | Yes | Compatible |
| Model performance based on evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |

Data collection

In [1]-[2], the following agreements on data collection were achieved.

Agreement In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact of UE side data collection enhancement including at least - Enhancement of CSI-RS configuration to enable higher accuracy measurement. - Assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc. - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side. - Signaling for triggering the data collection In CSI compression using two-sided model use case, further discuss the necessity, feasibility, and potential specification impact for NW side data collection including at least: - Enhancement of SRS and/or CSI-RS measurement and/or CSI reporting to enable higher accuracy measurement. - Contents of the ground-truth CSI including: - Data sample type, e.g., precoding matrix, channel matrix etc. - Data sample format: scaler quantization and/or codebook-based quantization (e.g., e-type II like). - Assistance information (e.g., time stamps, and/or cell ID, Assistance information for Network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator) - Latency requirement for data collection - Signaling for triggering the data collection Agreement In CSI compression using two-sided model use case, further study the necessity and potential specification impact of the following aspects related to the ground truth CSI format for NW side data collection for model training: - Scalar quantization for ground-truth CSI - FFS: any processing applied to the ground-truth CSI before scalar quantization, based on evaluation results in 9.2.2.1 - Codebook-based quantization for ground-truth CSI - FFS: Parameter set enhancement of existing eType II codebook, based on evaluation results in 9.2.2.1 - Number of layers for which the ground truth data is collected. And whether UE or NW determine the number of layers for ground-truth CSI data collection.

According to this agreement, data collection can be implemented at UE and gNB side. The necessary, feasibility and potential specification impact for them are respectively discussed from reference signal enhancement, signalling for triggering the data collection, assistance information for data collection, etc.

UE side data collection

Due to limited the memory storage and power of UE, data should be collected for training model at UE server side. Therefore, we mainly consider data collect at UE server side to discuss the potential specification impact.

- Reference signal enhancement

It is enough flexibly according to the time-frequency domain pattern design of CSI-RS resource in current specification. The enhancement of pattern design of CSI-RS resource is not necessary. SINR has impact on obtaining higher accuracy channel measurement. In order to obtain higher accuracy channel measurement, the simplest way is to increase CSI-RS transmission power, which can be achieved by changing the value of powerControlOffset or powerControlOffset in current CSI-RS resource configuration. Therefore, it is not necessary to enhance CSI-RS resource configuration for UE side data collection.

- Assistance information for data collection

Assume that a generated AI/ML model can be trained for different scenarios and configurations. It is not necessary to introduce ID for categorizing the data. Otherwise, the data ID can be introduced to indicate the characteristic of the collected data, such that UE server is able to train AI/ML model by using the suitable data. But the data ID can be indicated to UE server through proprietary way, which does not have impact on specification.

- Signalling for triggering the data collection

In order to collect data at UE server side, UE server will indicate UE to start or stop data collecting. The interaction signalling between them is propriety, which does not have impact on specification. But UE still need to send signalling to network so that network configures CSI-RS resource for estimating downlink channel. Once UE server has obtained enough training data, it will inform UE to stop collecting data. Accordingly, UE will send trigger signalling to network for stopping CSI-RS transmission.  If UE or network wants to update AI/ML model, they can also send trigger signalling to UE server or UE respectively for collecting new dataset. Such procedure is illustrated in Fig.1. Note that the trigger signalling between UE and network should be specified. The detail signalling design between them can be discussed during work item in next release.

Fig.1. The illustration of signalling interaction procedure among UE, network and UE server.

NW side data collection

- Reference signal enhancement

According to above discussion, higher accuracy measurement can be achieved by using the current CSI-RS resource configuration. It’s not necessary to enhance CSI-RS resource configuration. In current specification, CSI-RS resource is UE-specific reference signal. This means that CSI-RS resource is configured for each user through a specific configuration signalling. For network side data collection, this will consume a lot of signalling overhead. Note that there is no such issue for UE side data collection. In addition, the configured CSI-RS resource per UE may be different. In order to collect training data, network expects the dimension of collected data be same as far as possible. I.e., the number of CSI-RS resource port for all users should be same. In order to achieve this object, the simple way is that CSI-RS resource are configured per cell. It can not only reduce signalling overhead, but also make the dimension of collected data be same.

- Assistance information for data collection

In our view, configuration ID or Cell ID as an assistance information is not necessary. Since CSI-RS resource is configured as UE-specific, gNB can obtain channel information for each UE which corresponds to different configurations. Therefore, gNB is able to distinguish and categorize the collect data according to the CSI-RS resource configure for each UE. The data is collected at NW side. NW side knows which users are served by which cells. In addition, NW can control which users should report channel information for data collection. The users which do not belong to the serving cell will not be indicated for reporting data. Hence, the data ID is also not necessary to be included for network side data collection.

- Signalling for triggering the data collection

The procedure of data collection refers to starting and stop of data collection. Since the data collection is implemented at NW side, NW should decide when to initiate and stop data collection. For example, when a gNB detects that there are many accessed users, gNB will indicate the users to report the data for training model. Once gNB collects enough dataset, it will indicate all users to stop reporting data. Different from UE side data collection, network will initiate the trigger signalling to collect dataset. As shown in Fig.1., the signalling procedure to collect dataset for model training should be studied or specified if needed.

- Data collection for model performance monitoring

According to above agreement, scalar quantization or codebook-based quantization for NW side data collection for model training is adopted as the ground truth CSI format. Similarly, the ground truth CSI format for NW side data collection for model training should be specified as well if dataset is delivered through air interface. In our view, the ground truth CSI format can still be scalar quantization or codebook-based quantization for model performance monitoring. As for adopting which format, both performance and overhead should be considered if dataset is transferred through air interface.

As given above discussion on the triggering signalling of data collection for model training, it also needs to study the signalling procedure to collect dataset for model performance monitoring. E.g., if gNB wants to monitor the current in use AI/ML model performance, gNB will trigger a signalling to collect dataset. Whether the trigger signalling is physic signalling or high layer signalling can be further studied.

Observation 2: For UE side and network side collection, it is not necessary to enhance CSI-RS resource since higher accuracy channel measurement can be obtained by current CSI-RS resource configuration.

Observation 3: For UE side and network side data collection, the data ID is not necessary to report as an assistance information considering that the data ID can be obtained in a proprietary way or network configuration.

Proposal 3: For network side data collection, cell-specific CSI-RS resource configuration can be supported to reduce configuration signalling overhead.

Proposal 4: For network side data collection for model performance monitoring, the potential specification impact on the ground truth CSI format, e.g., either scalar quantization or codebook-based quantization, can be studied.

CSI configuration and reporting

The following agreement on CSI reporting were achieved in [1][2]. According to the agreements, the feasibility and methods of CSI reporting priority, CSI omission, codebook subset restriction (CBSR) and CSI processing unit (CPU) should be studied for CSI reporting. Before discussing CSI omission, we will firstly discuss UCI design since CSI omission is based on UCI design.

Agreement In CSI compression using two-sided model use case, further study the following aspects for CSI configuration and report: - NW configuration to determine CSI payload size, e.g., possible CSI payload size, possible rank restriction and/or other related configuration. - How UE determines/reports the actual CSI payload size and/or other CSI related information within constraints configured by the network. Agreement In CSI compression using two-sided model use case, further study the feasibility and methods to support the legacy CSI reporting principles including at least: - The priority rule regarding CSI collision handling and CSI omission - Codebook subset restriction - CSI processing Unit Agreement The study of AI/ML based CSI compression should be based on the legacy CSI feedback signaling framework. Further study potential specification enhancement on - CSI-RS configurations (No discussion on CSI-RS pattern design enhancements) - CSI reporting configurations - CSI report UCI mapping/priority/omission - CSI processing procedures. - Other aspects are not precluded.

CSI configuration and report

For legacy CSI compression feedback based on codebook, the maximum CSI payload size is determined by network configuration. However, the actual CSI payload size is determined according to the selected RI by UE. The actual CSI payload size should be always no more than the maximum CSI payload size. Otherwise, CSI omission will occur and CSI reporting is omitted according to the CSI omission rule in the current specification. There is no much difference on CSI compression feedback between codebook based and AI/ML mode based. They just adopt two different algorithms to calculate or report CSI. Therefore, the similar CSI configuration and CSI reporting can be adopted. The maximum CSI payload size for AI/ML based CSI compression feedback is determined by network. Assume the rank value is fixed and the maximum CSI payload size is configured by network. For one CSI compression reporting based on codebook, the actual CSI payload size can be calculated according to the configurated codebook parameter. While the actual CSI payload size can be determined by the applied CSI generation part model for CSI compression feedback based on two-side AI/ML model, where CSI generation part model can be selected by UE if more than one partial models are configured. How to determine the selected CSI generation part model is paired with the CSI reconstruction part model at network side depending on the two-side AI/ML model identification, which will be discussed in subsection 2.5, i.e., AI/ML functionality identification and/or model identification

Proposal 5: For CSI compression using two-sided model, the following potential specification impact for CSI configuration and report should be studied.

- The maximum CSI payload size is configured by network.

- The actual CSI payload size is determined by the CSI generation part model configured or selected by UE if one or more CSI generation part models are configured.

CSI reporting priority rule

When multiple CSI reports are sent to gNB, it will incur CSI collision, which results some CSI reports dropped. In current specification, CSI reporting is dropped through the predefined priority rule, and the rule is defined through the expression PriiCSIy,k,c,s=2∙Ncells∙Ms∙y+Ncells∙Ms∙k+Ms∙c+s. Each CSI report is associated with a priority value PriiCSIy,k,c,s. The smaller value PriiCSIy,k,c,s, the higher priority CSI reports.

AI/ML-based CSI compress feedback or beam management just instead of legacy CSI feedback or beam reporting through AI model inference. From this perspective, it is straightforward to legacy priority rule is reused for determining the priority of CSI reporting. However, once legacy CSI reporting and AI/ML-based CSI reporting are transmitted to gNB through different CSI reports. CSI collision may occur. For such case, how to drop them needs to be addressed. In our view, the following two methods can be considered to determine the priority of AI/ML-based CSI feedback.

- Alt1: A priority value PriiCSIy,k,c,s with new parameter value

- Alt2:  Introducing new parameter PriiCSIy',y,k,c,s

For Alt1, it is straightforward to determine the priority of AI/ML-based CSI feedback. For example, k=0 and k=1 respectively denotes L1-RSRP or L1-SINR reporting and other than L1-RSRP or L1-SINR reporting. In order to denote the priority of AI/ML-based CSI compression feedback, k can set to 0, 1 and 2, where k=0 still denotes L1-RSRP or L1-SINR reporting, let k=1 denote the AI/ML-based CSI compression reporting, and k=2 denote other than L1-RSRP or L1-SINR reporting and AI/ML-based CSI compression reporting.

For Alt2, a new parameter y' is introduced to denotes AI/ML-based CSI compression reporting. E.g., PriiCSIy,k',k,c,s= α∙Ncells∙Ms∙My∙y+β∙Ncells∙Ms∙My'∙y'+γ∙Ncells∙Ms∙k+Ms∙c+s. My' denotes the number of AI/ML-based CSI reporting, y'=0 denotes the AI/ML-based beam reporting, y'=1 denotes AI/ML based CSI reporting other than AI/ML-based beam reporting. Both alternatives can be adopted to determine the priority of AI/ML-based CSI reporting.

Proposal 6: The legacy priority rule can be reused to define the priority the AI/ML based CSI reporting, and a priority value PriiCSIy,k,c,sPriiCSIy,k,c,s PriiCSIy,k,c,swith new parameter value or introducing new parameter  PriiCSIy',y,k,c,s is used to indicate the priority of CSI reporting.

UCI design

In current specification, CSI reporting can be transmitted through PUSCH or PUCCH. For Type II or eType II codebook, CSI reporting includes two parts, i.e., Part 1 and Part 2 to report the contents of CSI through PUSCH. The PMI is included in Part 2. For AI/ML based CSI feedback with two-sided model, the compression information can be regarded as one kind of PMI. Therefore, the compression information should be included in Part 2. RI and CQI are still included in Part 1 if RI, CQI and compression information are reported together. According to discussion on CSI measurement in subsection 2.3, RI and CQI can be independently reported in second stage. For such as case, CSI compression feedback can be reported through one part or two parts.

Proposal 7: CSI reporting with two parts, i.e., Part 1 and Part 2 or only one part for AI/ML based CSI feedback with two-sided model can be supported.

CSI omission

When there is no enough uplink resource for transmitting all contents in a CSI reporting, CSI omission will occur. I.e., partial contents in the CSI reporting will omitted. However, if some important CSI parts are dropped, it will result significant performance loss. In order to address this issue, CSI Part 2 for eType II codebook are divided into three groups. According to discussion on UCI design in subsection 2.4.2, CSI reporting based on AI/ML model can include two parts or only one part. In our view, the CSI Part 2 for two parts and only one part can also be divided into N>1 groups to implement CSI omission. Different from eType II codebook, AI/ML-based CSI compression feedback may not include non-zero coefficients and SD basis, FD basis, indication information of non-zero coefficients, and so on. There are mainly compressed quantization information for AI/ML based CSI compression.  The question is how to divide the compressed quantization information into N groups.

According to discussion in AI 9.2.2.1, for the two-sided AI/ML model, it includes layer-common model, layer-specific model, rank-common model and rank-specific. For different models, the length or contents are different as well. Before discussing CSI omission, which types of model adopted should be firstly determined.

Proposal 8: The compressed quantization information is divided into N>1 groups for CSI omission, where the values N and how to divide into N groups needs to further study.

Codebook subset restriction (CBSR)

In traditional Type II codebook-based CSI feedback, the codebook subset restriction would be configured to UE for eliminating the inference. While for AI based CSI feedback, how to eliminate the interference is one issue to be solved. In previous meeting, two options were agreed as input-CSI-NW for further study, option 1a is the precoding matrix in spatial-frequency domain, and option1b is the precoding matrix represented using angular-delay domain projection. In our understanding, option 1b is codebook-based solution and can apply the codebook subset restriction naturally. For example, the UE can transfer the precoding matrix in spatial-frequency domain to precoding matrix in angular-delay domain by using a restricted DFT codebook, and the restricted parameters can be configured by gNB.

Observation 4: The CBSR can be implement when input-CSI-NW the precoding matrix represented using angular-delay domain projection.

Performance monitoring

In [2]-[3], the following agreements on performance monitoring was identified. According to the agreements, model performance monitoring could be implemented at NW-side and UE-side. For UE-side model performance monitoring, it includes without CSI reconstruct model and with CSI reconstruct model at UE side. The feasibility, specification impact, overhead, complexity, and latency about them are respectively discussed.

Agreement In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact for intermediate KPIs based monitoring including at least: - NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side. - UE-side monitoring based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW or obtained from the network side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - UE-side monitoring based on the output of the CSI reconstruction model at the UE-side - Note: CSI reconstruction model at the UE-side can be the same or different comparing to the actual CSI reconstruction model used at the NW-side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - FFS: Other solutions, e.g., UE-side uses a model that directly outputs intermediate KPI. Network-side monitoring based on target CSI measured via SRS from the UE. Note: Monitoring approaches not based on intermediate KPI are not precluded Note: the study of intermediate KPIs based monitoring should take into account the monitoring reliability (accuracy), overhead, complexity, and latency. Agreement In CSI compression using two-sided model use case, for NW-side monitoring, further study the necessity, feasibility and potential specification impact to enable performance monitoring using an existing CSI feedback scheme as the reference. - The association between AI/ML scheme and existing CSI feedback scheme for monitoring - Note: The metric for monitoring and comparison includes intermediate KPI and eventual KPI. - Other aspects are not precluded. Agreement In CSI compression using two-sided model use case, for UE-side monitoring, further study potential specification impact on triggering and means for reporting the monitoring metrics, including periodic/semi-persistent and aperiodic reporting, and other reporting initiated from UE.

- The feasibility and reliability

In theory, it is feasible that the three methods are used to monitor model performance. But the reliability needs to evaluated through simulation in AI 9.2.2.1. For NW-side model performance monitoring, gNB can collect different target CSIs from different UEs. Therefore, gNB can obtain reliable target CSI to help gNB monitoring model performance. Even though one target CSI from a UE is not accurate, gNB still monitors the model accurately. While output CSI is obtained from one UE for UE-side performance monitoring, the reliability needs to further study and evaluate. The reason is that the performance degradation may not be incurred by AI/ML model. It is possible the channel condition of the user is changed, which also results performance loss. For such case, the model at gNB side does not need to update or switch.

- Overhead

If the target CSI or output CSI is transmitted in a proprietary way, the overhead does not need to considered. Otherwise, how to reduce the overhead needs to study. Since target CSI or output CSI can be precoding matrix or channel matrix, it will need a lot of quantization indication overhead, especially when the number of subbands is configured with larger values. Therefore, the transmission overhead of target CSI and output CSI is usually larger. In order to reduce the overhead, higher resolution codebook quantization, e.g., Rel-16/17 Type II codebook can be considered if there are obviously performance loss.

- Complexity

According to the calculation SGCS and NMMSE, they only refer to operation of addition and multiplication. Hence, the computation complexity can be affordable for UE side and network side.  If the intermediate KPI, such as SGCS or NMMSE, is adopted to monitor model performance, the computation complexity for performance monitoring can be ignored.

- Specification impact

For NW-side mode performance monitoring, it was agreed to study potential specification impact by using existing CSI feedback scheme as a reference according to above agreement. Model performance monitoring can be performed at UE side as well. Accordingly, the potential specification impact by using existing CSI feedback scheme as a reference should be studied as well. The motivation of using existing CSI feedback scheme as a reference is to avoid the wrong judgement. For example, the inferred performance by AI/ML is degraded due to significant channel variation for one user. While other users still kept the acceptable performance. It can be observed that the performance loss is not incurred by the AI/ML model in use. Similarly, for UE side performace monitoring, existing CSI feedback can be as a reference as well to judge whether to update AI/ML model or fall-back. The metric for UE side performance monitoring can also include intermediate KPI and eventual KPI. If the metric is intermediate KPI, network needs to send the output CSI of CSI reconstruction part model to UE. UE respectively calculates the intermediate KPI by using the output CSI and existing CSI feedback scheme. UE will obtain the monitoring results through comparing the two intermediate KPIs. If the metric is eventual KPI, UE can calculate the eventual KPIs through the received beamforming PDSCH or CSI-RS from network. The beam of PDSCH or CSI-RS is obtained by network via the existing CSI feedback scheme or the output CSI of CSI restructure part model. Note that the beaming type should be indicated to UE. During the calculation of intermediate KPI or eventual KPI, the channel should be same for AI/ML model inference or existing CSI feedback scheme. i.e., the CSI measurement and calculation for AI/ML model inference should be associated with that for existing CSI feedback scheme.

Based on above discussion, we have the following observations and proposal.

Observation 5:  The feasibility and reliability need to evaluate for the three methods of model performance monitoring.

Observation 6: The transmission overhead of target CSI or output CSI is usually larger for model performance.

Observation 7: For NW-side model performance monitoring, network calculates the intermediate KPI, which save the calculation complexity of UE-side.

Observation 8: The method, container and time behaviour of target CSI or output CSI transmission need to be specified.

Proposal 9: For UE-side monitoring using an existing CSI feedback scheme as the reference, the following potential specification should be studied:

- The association between AI/ML scheme and existing CSI feedback scheme for monitoring.

- The precoding type indication/configuration of the beamformed PDSCH or CSI-RS.

- The metric for monitoring and comparison includes intermediate KPI and eventual KPI

AI/ML functionality identification and/or AI/ML model identification for AI/ML based CSI compression feedback

According to discussion on AI/ML framework in AI 9.2.1, for UE-side model and UE-part of two-sided model, they can be identified through AI/ML functionality identification and/or AI/ML model identification. The related agreement on model identification and lifecycle management (LCM) are given as follows. In this subsection, we will discuss how to implement AI/ML functionality identification and/or AI/ML model identification for AI/ML based CSI compression feedback.

Agreement For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. FFS: Relationship between functionality identification and model identification FFS: Performance monitoring and RAN4 impact FFS: detailed understanding on model Agreement - AI/ML-enabled Feature refers to a Feature where AI/ML may be used. Agreement - For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature. Agreement - For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models: - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level - FFS: Other aspects that may constitute Functionality - FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda. - For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models: - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda. - FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s). - FFS: relationship between functionality-based LCM and model-ID-based LCM - Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion. Conclusion From RAN1 perspective, it is clarified that an AI/ML model identified by a model ID may be logical, and how it maps to physical AI/ML model(s) may be up to implementation. - When distinction is necessary for discussion purposes, companies may use the term a logical AI/ML model to refer to a model that is identified and assigned a model ID, and physical AI/ML model(s) to refer to an actual implementation of such a model. Agreement - Study necessity, mechanisms, after functionality identification, for UE to report updates on applicable functionality(es) among [configured/identified] functionality(es), where the applicable functionalities may be a subset of all [configured/identified] functionalities. - Study necessity, mechanisms, after model identification, for UE to report updates on applicable UE part/UE-side model(s), where the applicable models may be a subset of all identified models.

Based on above agreements, we think AI/ML-based CSI compression feedback is one of AI/ML-enabled feature. For such feature, it includes multiple models or pairing models at UE side. In our view, these models can be identified through functionality, model ID or both functionality and ID.

For AI/ML functionality identification, the definition of AI/ML functionality should be firstly given, such that UE and gNB can achieve common understanding what is the functionality of each model. For simplicity, the AI/ML functionality for CSI compression can be defined by its application condition, supported configuration parameter or CSI payload size outputted by CSI generation part model. Based on such definition, each CSI generation part model and/or CSI reconstruction part model at UE side can be defined, and is reported to gNB through UE capability reporting. For example, there are two AI/ML functionality. According to functionality definition, AI/ML functionality 1 supports maximum payload size is 250 bits and to inference in Uma, UMi or indoor with speed less than 10 Km/h.  AI/ML functionality 2 supports fixed payload size is 100 bits and to inference in UMa with speed higher than 10 Km/h. UE will report the two AI/ML functionalities to gNB through UE capability reporting.

For AI/ML model identification, it is directly method to define an AI/ML model. Each model has its model ID. According to discussion in RAN1, they assume model ID is unique “globally”. UE just sent the model ID to gNB. gNB will know UE support which model IDs. In order to identify what is the model, it requests to character the model with detail information. Such as, we still assume that there are models, and model ID 1 supports maximum payload size 120 bits and application into UMa, UMi and indoor with speed no more than 3 km/h. Accordingly, model ID 2 supports maximum payload size 120 bits and application into UMa with speed more than 30 km/h.

In addition to AI/ML functionality identification and AI/ML model identification, AI/ML model can be identified by both AI/ML functionality and AI/ML model identification. It can be illustrated in Fig.2. For the AI/ML- enabled CSI compression feature, the feature includes two functionality which applied to different scenarios for CSI compression feedback. For each functionality, it includes two mode. Different model ID corresponds to different maximum payload size. Although Model ID1 or Model ID3 can be adopted to infer the payload size with less than 60 bits, the inference performance of Model ID1 and Model ID3 is less than that of Model ID2 and Model ID4, respectively. Therefore, they should be divided into different AI/ML models.

Fig.2. Illustration of model identification based on AI/ML functionality identification and AI/ML model identification

Proposal 10:  UE side model or UE part model for CSI compression feedback can be identified through AI/ML functionality, AI/ML model, or both functionality and model. How to define AI/ML functionality and/or AI/ML model for CSI compression feedback should be firstly studied and discussed.

Conclusions

In this contribution, potential specification impact for AI/ML based CSI compression feedback with two-sided model is discussed, and the observations and proposals are given as follows:

Observations

Observation 1: NW may store and manage a lot of NW-sided part models for joint training of the two-sided model at UE side for type 1 and UE-first separate training for type 3.

Observation 2: For UE side and network side collection, it is not necessary to enhance CSI-RS resource since higher accuracy channel measurement can be obtained by current CSI-RS resource configuration.

Observation 3: For UE side and network side data collection, the data ID is not necessary to report as an assistance information considering that the data ID can be obtained in a proprietary way or network configuration.

Observation 4: The CBSR can be implement when input-CSI-NW the precoding matrix represented using angular-delay domain projection.

Observation 5:  The feasibility and reliability need to evaluate for the three methods of model performance monitoring.

Observation 6: The transmission overhead of target CSI or output CSI is usually larger for model performance.

Observation 7: For NW-side model performance monitoring, network calculates the intermediate KPI, which save the calculation complexity of UE-side.

Observation 8: The method, container and time behaviour of target CSI or output CSI transmission need to be specified.

Proposals

Proposal 1: Device-specific for type 1 can be considered to train two-sided AI/ML model.

Proposal 2: The pros and cons of the joint training of two-sided model at NW side for type 1 with device-specific and NW-first separate training for type 3 should be firstly studied and discussed.

Proposal 3: For network side data collection, cell-specific CSI-RS resource configuration can be supported to reduce configuration signalling overhead.

Proposal 4: For network side data collection for model performance monitoring, the potential specification impact on the ground truth CSI format, e.g., either scalar quantization or codebook-based quantization, can be studied.

Proposal 5: For CSI compression using two-sided model, the following potential specification impact for CSI configuration and report should be studied.

- The maximum CSI payload size is configured by network.

- The actual CSI payload size is determined by the CSI generation part model configured or selected by UE if one or more CSI generation part models are configured.

Proposal 6: The legacy priority rule can be reused to define the priority the AI/ML based CSI reporting, and a priority value PriiCSIy,k,c,sPriiCSIy,k,c,s PriiCSIy,k,c,swith new parameter value or introducing new parameter  PriiCSIy',y,k,c,s is used to indicate the priority of CSI reporting.

Proposal 7: CSI reporting with two parts, i.e., Part 1 and Part 2 or only one part for AI/ML based CSI feedback with two-sided model can be supported.

Proposal 8: The compressed quantization information is divided into N>1 groups for CSI omission, where the values N and how to divide into N groups needs to further study.

Proposal 9: For UE-side monitoring using an existing CSI feedback scheme as the reference, the following potential specification should be studied:

- The association between AI/ML scheme and existing CSI feedback scheme for monitoring.

- The precoding type indication/configuration of the beamformed PDSCH or CSI-RS.

- The metric for monitoring and comparison includes intermediate KPI and eventual KPI

Proposal 10:  UE side model or UE part model for CSI compression feedback can be identified through AI/ML functionality, AI/ML model, or both functionality and model. How to define AI/ML functionality and/or AI/ML model for CSI compression feedback should be firstly studied and discussed.

References

- 3GPP RAN1#112 meeting, Chairman’s notes.

- 3GPP RAN1#112bis e-meeting, Chairman’s notes.

- 3GPP RAN1#110 bis e-meeting, Chairman’s notes.